Changes:

05/23/01 - Removed reti as syscall and replaced it with system insn.

04/06/01 - Replaced BP with DP.  Mentioned reti #7 stack overflow trap.

02/26/01 - revise to match beta release.  Renamed FSR as IP.

10/18/00 - revise for new software stack and calling sequence changes.

09/05/00 - update stack, calling-sequence and register details to

reflect reality.

Attached are some notes on the runtime environment for the ip2k

compiler.  I'm trying to balance the utilization of the very limited

resources present on-chip against the features that are known or

suspected to result in compiler problems.

1/ One-of-a-kind resources like the W, MULH and IP.  Every

GCC port I've worked with has had at least TWO of limited

resources. This is very much like the accumulator-based

machines which folk-wisdom says are not good candidates for

GCC ports.

CONCLUSIONS: MULH and W are invisible to the register

allocator and are implicitly modified (and marked as used by

the compiler)

IP and DP will be managed by the compiler to support general

memory accesses without introducing problems with managing the

register contents.

2/ Now that we have a real stack with push/pop and a

displacement addressing mode, we will take advantage of it.

================

Notes on the runtime architecture for the ip2k.

Given the limited support for memory and stack locations and the

limited depth of the function-call stack, we have to make a large

number of trade-offs.

1.0 Basics.

1.1 Stack

The stack is accessed via the SP (for frames smaller than 128 bytes).

All other accesses are 'indirect' using the DP or IP.

OBSERVATION: Any instruction pattern with more than ONE memory

reference may cause reload to fail - in fact, since we may end up

with reload having to use a stack temp that can't be directly

accessed, we could easily run out of registers for loading

1.2 Memory

Data pointers are 16 bits, allowing access to all of the register file

directly.

Instruction pointers are 16 bits and are not compatible with data

pointers.

1.3 Memory access to local and argument data

All QI and HI moves are directly coded.  Larger modes are split into

HI size chunks.  All HI mode operations must support IP addresses

which can be difficult because we must adjust IP on-the-fly.

1.4 Special Registers

$FF is initialized at startup to contain a zero and is available

for various code-generation purposes that require a zeroed register

to be available.

$fd..$fe - frame pointer chain. (preserved)

$80..$87 - 8 bytes of function return register.

$88..$8b - 4 bytes of additional scratch

No other registers are considered as special.

2.0 Function Calls

2.1 Parameter Passing

Parameters are passed on the call stack.  No alignment restrictions

or implicit widening is necessary.  [e.g. a 'char' parameter is passed

as a single 8-bit quantity.

2.2 Return values

Function results are returned in $80..$87.  Note that CHAR values

are widened to 'int' and returned in $80..$81.  But a function

declared as

extern char f();

will only examine $81 for the return value.

Structures larger than 8 bytes are returned by a caller provided

pointer to a temporary.  That pointer is passed as a hidden first

parameter.

2.3 Caller Responsibility

After pushing all the arguments to be passed calls are normally done with a

PAGE  #function

CALL  function

instruction sequence.  The linker is responsible for 'relaxation' to

remove unnecessary PAGE instructions.

Indirect calls are handled via runtime support somewhat like a

trampoline.

On resumption, the caller must pop all parameters off the stack.

2.2.2 Callee Responsibility

For non-leaf functions, the caller must save the RETURN PC on the

software stack and restore it to the CALLH/CALLL registers before

executing the RET instruction.

The caller must also preserve IP and DP and create a FP chain so

that the debugger can examine stack frames.

On function return, the callee loads $80..$87 with the result,

restores any preserved registers (including the saved callh/l),

restores the saved PC into the callh/l stack-top and returns to the

caller with

ret

The FP chain is required to allow the debugger to trace the call

stack and to support examining variables that are stack resident.

2.4 Simulator system calls

The ip2k-elf simulator uses the "system" instruction as a system-call

trap.  The call stack contains an argument list (syscall number and other

required arguments).  See devo/sim/ip2k/ip2k.c for more details.

A special syscall number (0xff) is reserved for use by the compiler to

denote stack overflow.  The syscall is automatically generated in a

function prolog when the -fstack-check option is used and requires no

subsequent arguments.  The syscall gets triggered when the SP has decremented

below the end of the static data area.  Note that any HEAP data corruption

is only detected when a malloc call overflows into the dynamic heap.

Simulator syscalls are valuable in running various compiler validation tests.

3.0 Startup

===========

3.1 Static Initialized Data

The simulator will arrange to load static data into the register file

before starting execution.

The compiler assumes that all static data is in the register file.

The compiler does not support access to data stored in SRAM or FLASH

memory.  It is the programmer's responsibility to code such

references in assembly language.