How to go about and create your own text mode

---------------------------------------------

First of all: if you are not familiar with configuring the X-server, you'd

better start off there, as it is MUCH more stable, MUCH less buggy, and MUCH

better documented. You will also learn how to use several tools (vgaset,

SuperProbe, ConfigXF86, ...) which can aid you to design your own custom

X-Windows mode. The same tools can then be used here.

For people less familiar with monitor timing stuff, and unwilling or unable

to consult the XFree86 documentation, read the "general notes on monitor

timings" section below. It's a small (and incomplete) introduction on how

monitors work, and what you need to send it to get some sort of a picture

on it.

You will probably need the X-server (e.g. to do 'X -probeonly') to determine

your available pixel clocks, and maybe even to determine what chipset you

have.

The TextConfig file

===================

The "REPORT" file in this distribution should contain an up-to-date list of

all supported chipsets, and on which specific boards it has worked at least

once ;-)

General guidelines:

* The TextConfig file MUST be in your /etc directory. If you don't like

that, change the appropriate line in the Makefile and recompile. It

should be straightforward.

* remarks can be inserted after a '#' (hash) sign. Everything after the

hash sign will be disregarded.

* You may (or may not) enclose certain strings in double quotes (") if you

like, but they are NOT used in any way. They are only of "cosmetic

importance".

* empty lines are allowed.

* Read the comments in the TextConfig file as well. They explain some

modes as they come along.

CHIPSET

-------

First of all, you have to define which chipset is on your VGA card: e.g.:

ChipSet "ET4000"

Other possible chipsets are:

- "S3"		ANY S3-based card, including those from Diamond,

Number 9 and SPEA/Video7.

- "CLGD542x"	Cirrus Logic chipsets. This COULD also work with the

CL-GD543X chipsets ("Alpine" chips)

- "ET4000"

- "TVGA8900"

- "TVGA9000"

- "PVGA1"

- "WDC90C0X"

- "WDC90C1X"

- "WDC90C2X"

- "WDC90C3X"

- "ATI"		All ATI cards BEFORE the MACH32

- "ATIMACH32"	MACH32, MACH64

- "VIDEO7" 	Headland Technologies based Video 7 boards only.

Older V7 boards use C&T or Cirrus chips.

Newer V7/SPEA cards use S3.

OPTION

------

Special option flags can be inserted in the TextConfig file. They change the

default behaviour for each card programmed into SVGATextMode.

The syntax is simply:

Option <my_option>

There are some examples in the TextConfig file.

Currently the following options are allowed (for the specified chip sets):

ET4000: "hibit_high" or Option "hibit_low"

This flag inverts the meaning of the 4th clock selection bit of an

ET4000 card. See the XFree86 documentation for more information. The

file README.Tseng from the XFree86 package, included in this archive,

should be able to help you define the correct "hibit" flag.

WDC90CXX: "swap_hibit"

Some WDC cards need this flag. It inverts the meaning of the third clock

selection bit (bit #2). It has the same meaning as in the XFREE config

file (XF86Config).

ET4000: "ET4000_AltClockSel"

This selects an alternative clock selection method for some very rare

ET4000 based cards that don't work with the normal clock selection

method. These cards can only use their 8 lowest clocks under X-Windows

(the X-server does NOT support the alternate clock selection method).

This option should allow them to use all 16 clocks (i.e. those reported

by the DOS utility "dmode.exe" which is delivered with most ET4000 boards.

NOTE that the XFREE code does NOT report the correct clocks!)

Read doc/README.ET4000.AltClockSel !

ET4000: "Legend"

ET4000 based Sigma Legend boards need this option for correct clock

selection.

CLGD542X: "XFAST_DRAM" , "FAST_DRAM" , "MED_DRAM" , "SLOW_DRAM"

Most Cirrus boards can take higher pixel clocks when you increase the

default DRAM speed. But some CIrrus cards crash your machine when DRAM

speed is to high. Use this option to tune the speed to your card. The

higher you can set the DRAM speed, the better. "XFAST_DRAM" is the

fastest, and the most dangerous. It works on at least one CLGD-5428

card, and it crashes on at least another 5422 card... Use caution.

Faster pixel clocks usually need faster DRAM speed.

A special option, allowed for ALL chips is "LOADFONT". When enabled, this

will tell SVGATextMode to load a new font from the specified FontPath (or

the default one) using the specified font loader (or the default one). See

the font loading section.

If you are configuring SVGATextMode for the first time, it is best to leave

all options at their default (i.e. no special "option" lines). If something

doesn't work OK, THEN is the time to start trying option flags.

If you know your card needs the same option in XWindows, you could insert it

from the first time on. Most options behave in the same way as they do in

the XConfig (XF86Config) file.

CLOCKS

------

Then, you need to define what clocks you have. Again, you can get those from

your X-server by typing "X -probeonly" (without the quotes).

Note: Cirrus Logic cards DO NOT NEED A "Clocks" LINE (if you select the

"CLGD542x" chipset, any "clocks" line will be disregared).

Clocks 25.3 28.175 ....

You can have multiple clocks lines in the config file. ALL of them will be

used as if you typed all clocks on one line.

The order of the clocks MUST be the same as on the card: the first one will

be programmed as clock number 0, the second as number 1, etc. Again, if you

are familiar with configuring The X-server, this should be all too obvious

to you.

WARNING: define only the clocks you have, since this program justs assumes

they exist. Even if your card has only 8 clocks, and you define 16 of

them, it will still try to program one out of 16 clocks. Without warning.

This should change in the future...

And also: the program assumes the clock VALUES are correct. All timing

parameters that you are given at the end of the program are based on those

numbers! So if you say that clock number 3 is 40 MHz while its REAL value is

75 Mhz, the Horizontal and vertical frequencies given at the end will be

incorrect by the same ratio!

ClockProg:

----------

[text taken and adapted from XFREE docs]

ClockProg "command" [textclock]

This  optional entry runs command to set the clock on the graphics board

instead of using the internal code. The command string must consist of the

full pathname (and no flags). When using this option, a Clocks entry is

required to specify which clock values are to be made available to the

server (up to 128 clocks may be specified).  The optional textclock value is

used to tell the server that command must be run to restore the textmode

clock at server exit (or when VT switching).  textclock must match one of

the values in the Clocks entry.  This parameter is required when the clock

used for text mode is a programmable clock.

Note for SVGATextMode: the [textclock] argument is not used in SVGATextMode,

since we're talking about text mode. Let's hope the XFREE server WILL use

this, so the SVGATextMode mode will be restored properly.

NOTE for XFREE users: You MUST define the textclock in your XF86Config file

when you are using a clock program, because there is NO way the XFREE server

can find out what clock you were using when you started X. Most clock chips

don't allow reading back what was programmed in them, so the X-server is

unable to restore the textmode clock when switching back to text mode,

UNLESS you tell it what clock was used!

Two arguments are passed to the command. The first is the clock frequency in

MHz as a floating point number and the second is the index of the clock in

the Clocks entry:

<your external clockprogram> <frequency in Mhz> <clock index of that frequency>

The command should return an exit status of 0 when successful, and something

in the range 1-254 otherwise.

NOTE: as mentionned in the text above, you need to specify a clocks line,

although the clock chip can make any clock within a certain range, and not

just the ones in the clocks line. This requirement is necessary for external

clock programs that program an "old-style" clock chip with just a fixed set

of (mostly 16) clocks, like most "cheaper" VGA cards use. In that case, the

program will probably ignore the frequency parameter passed to it, but will

use the index number to select the appropriate clock.

This is also a usefull tool for those having a VGA card that is (yet)

unsupported by SVGATextMode: Just write your own clock setting program that

complies with this "standard", test it, and send it to me for inclusion in

the next release ;-)

This should now be the same syntax as the external clock calling procedure used

by the XFREE3.x package.

The clock program path must be a FULLY SPECIFIED PATH to a clock setting

program that is SETUID ROOT! If it is not correctly SETUID ROOT, you will

get an error:

./SVGATextMode: ERROR: 'system' returned error code 35584

No environment variables will be used. If your clock selection program uses

another interface, use a script that calls that one.

One external clock setting program is included in the "contrib/v7mercury"

directory, for the Video 7 Mercury cards (S3-based, with an ICD2061 clock

chip). It might have become obsolete since SVGATextMode supported more clock

chips internally.

ClockChip:

----------

Special clockchips, for S3 boards that do not use the "default" clock chips,

can be selected with this option. Just add

ClockChip <clockchip_string>

to your TextConfig file. The meaning and syntax of this option is the same

as for the XFREE 3.x servers. The following clockchip strings are defined:

"icd2061a"

some work needs to be done: there are two different ways of using this

chip (at least: Diamond and SPEA V7)

"ics2595"

"s3_sdac"

"s3gendac"

Also works for ICS5300 (which is a GenDAC).

"sc11412"

"ti3025"

TI ViewPoint 3020/3025 RAMDAC with builtin clock synthesizer.

Terminals

---------

When NO 'Terminals' line is defined, SVGATextMode will try to resize all

ACTIVE VT's automatically. The only limitation is that it can only detect

the first 16. If you have more than 16 VT's (63 possible!), or if the

automatic resizing does not satisfy you, define a 'terminals' line. Only the

terminals defined in it will be resized. The rest will be left "untouched".

The 'Terminals' line tells SVGATextMode which terminal devices will be

affected by a possible screen resize, and need to be resizd. It will resize

all mentionned terminals after switching to another mode.

Terminals tty6 tty5 tty4 tty3 tty2 tty1

This is equivalent to typing 'stty rows <y> cols <x>' at every of those

terminals. With the added advantage that any applications running in those

terminals will be sent an "SIGWINCH" signal by the kernel. Many full-screen

terminal applications will redraw their screen into the new sizes upon

receipt of that signal.

But some programs will NOT know about the new screen size, and will not work

correctly anymore after a screen resize. They need a more drastic approach:

ResetProg

---------

The "reset program" is called when SVGATextMode has finished its job

succesfully. The syntax is:

ResetProg <path>

<path> must be a fully specified path.

The reset program could be used to "reset" any application that is affected

when the screen size changes due to using SVGATextMode. It could for example

be a script that kills selection and restarts it, or sends some other

application an appropriate signal to let it know the screen has changed.

The reset program will be called with TWO arguments: the H and V size of the

new text mode.

Example: SVGATextMode 100x37 will call:

/home/root/some_script 100 37

Font Loading

------------

SVGATextMode can also automatically load a new font when the text mode has

changed. This is enabled by adding

Option "LoadFont"

to the TextConfig file.

In order to load a new font, SVGATextMode needs to know a few things.

First of all: the font loading program. This is an external program, that

will be called with the specified font as an argument. It can specified as:

FontProg "/usr/bin/setfont"

Where "/usr/bin/setfont" is of course the full pathname of the font program.

If this line is NOT present, but you enabled font loading with the

"LoadFont" option, SVGATextMode will use the default path

"/usr/bin/setfont". In this case, the line specified is a no-op, since it

replaces the default font path with the same one...

And secondly, you can also tell the font program where the fonts are

located:

FontPath "/usr/lib/kbd/consolefonts"

If not specified, the font path defaults to "/usr/lib/kbd/consolefonts".

Last but not least: you must let SVGATextMode know which font you want used

for which character cell size. This is accomplished by entering a font

table, as shown below:

FontSelect 8x8 9x8 Cyr_a8x8   # use same font for 8-wide and 9-wide text

...

FontSelect 8x16 Cyr_a8x16

For each possible character cell size you intend to use, a font file must be

specified. You do not have to add a font file for ALL possible combinations

of fonts from 1 to 32 pixels high and 8 to 9 pixels wide. But if you select

a text mode with a font size that has no entry in the font table, and font

loading is enabled, SVGATextMode WILL put you in the new text mode, but

won't change the font (since it doesn't know what font to load). It will

issue a warning that font loading was enabled, but no font specified.

CURSOR

------

Some people complained about the "silly underscore cursor" that SVGATextMode

sets the VGA card to. Well, now you can change your cursor to your heart's

desire...

To avoid needing a cursor definition on each and every text mode line, there

is just ONE cursor definition in the TextConfig file, which is then used for

ALL text modes. This seemed a little more "clean".

The syntax is simply:

cursor <start>-<end>

Where <start> is the starting line of the cursor, and <end> is of course the

end. The size of the cursor depends on the size of the selected font. If you

selected a 16-line high font, then setting the cursor to show from line 7 to

line 8, it'll me smack in the middle of the line. defining the same numbers

for an 8-line font would put the cursor on the bottom of the text line (=

and underscore cursor).

That's why the cursor definition as shown above will ALWAYS be relative to a

32-line font (the largest possible font size). If the REAL font size is

smaller, it will be scaled to fit that font size. So defining an underscore

cursor for 32-line fonts will also get you an underline cursor for any other

font size.

example:

cursor 28-31

results in an underscore cursor for ALL text modes, while

cursor 0-31

will get you a block cursor.

The first case, when used in a 16-line font, is scaled with a factor 16/32,

and thus will in reality be a cursor from line 14-15.

HorizSync/VertRefresh

---------------------

The HorizSync and VertRefresh lines allow you to protect your monitor from

getting a mode from the VGA card it cannot handle. This is usefull for both

avoiding the problem of accidently getting into a non-syncing mode, and also

avoids the (very small) possibility of monitor damage due to extremely

out-of-range sync inputs.

The syntax is the same as the one used in the XFREE package:

HorizSync <from>[-<to>] [, ...]

VertRefresh <from>[-<to>] [, ...]

where <from> and <to> are floating point numbers in kHz (for HorizSync) or

Hz (for VertRefresh). When a single number (= only <from>) is defined, a

margin of +/- 0.3 (kHz or Hz) will be allowed: 'HorizSync 56' will allow a

horizontal sync range of 55.7 to 56.3 kHz.

A more complex line could look like this:

HorizSync 30-32,48,56      # 30 to 32 kHz, 48 and 56 kHz allowed

Not defining the HorizSync/VertRefresh variables implies the default values:

HorizSync 30-32          # 30 to 32 kHz

VertRefresh 50-80        # 50 to 80 Hz

Change these to use the full capabilities of your monitor.

Changing them BEYOND what your monitor can handle, COULD result in damage

to the monitor! See your monitor's user's manual for details.

TEXT MODE LINES

---------------

The text mode lines are the real thing: they set up the video card for the mode

you request. A TextConfig file can contain LOTS of configuration lines. It

would be best that you leave the "default" config lines in the example

TextConfig file as they are.

Add your own configuration lines at the end. If you patch a line, first copy

it to the end of the file, and change it there. You don't have to rename it,

as SVGATextMode will take the LAST line with the same label it finds. So if

your custom line, with the same name as an "original" one is at the end, the

custom line will be used.

That way you will always have the original lines as a reference, in case you

screw your own line up. You can then always copy it again.

Now let's analyse a new text mode:

"100x37"  50   800 848 880 1032    600 600 606 630   -hsync -vsync font 8x16

<---1-->  <2>  <------3------->    <-------4----->   <-----5-----> <---6--->

(1) This is the identification string for this text mode. If you the type

"SVGATextMode 100x37" the program will try to program this one.

(2) The pixel clock frequency. Since all VGA cards can also use every

clock divided by two, a clock of 50 MHz can also be called upon as 25

MHz. The program will automagically make the VGA card divide it by

two.

(3) Horizontal timing parameters (active video size, start H-sync, stop

H-sync, end video-line). See the XFree documentation for a thorough

understanding of these, or try the section om monitor timins below.

See also the TextConfig file for some explanation on the effect of

selecting an 9-pixel font on these timings!  Your textmode will have

one eighth the number of characters per line as in the active video

size (the first number of the horizontal timing parameters) REGARDLESS

of the font size specified.  So in this case : 800/8 = 100 characters

per line.

(4) Vertical Timings. Equivalent to the horizontal ones. The number of

textlines (rows) in your textmode will depend on the font size

selected: number_of_textlines = number_of_active_lines / font_height.

Here we have 600 active lines, and a font of 16 pixels high (see

below), so there will be 600/16 = 37 textlines.

(5) Hsync and Vsync polarity (positive (+) or negative (-)). Most modern

monitors don't care about sync polarities, BUT most USE them.

Simple fixed-frequency or dual frequency monitors (so non-multisync,

e.g.  only 32 and 48 kHz, instead of the entire range from 32 to 48),

and some non-digital-control ones use the polarities to change their

vertical screen size.

Cheap monitors have the habit of not having a constant vertical image

size, independent of the vertical refresh frequency. So your monitor

might use the entire screen at a normal 60 Hz frequency, but the image

is larger at lower frequencies, and smaller at higher ones. These VGA

monitors use the sync polarities to distinguish between several

vertical refresh frequencies, and to adapt their vertical size. I have

a MAG PMV1448, and it is a fine example of that. At 800x600@70Hz, the

screen is either only half the vertical size, or the entire screen,

depending on sync polarity. So some experimenting with polarities

might give you a full screen, even with >70 Hz refresh rates. (The

same applies to the XF86Config as well)

More expensive monitors, mostly with "digital control", use the sync

polarities COMBINED with the incoming horizontal and vertical

frequencies to determine whether they will take their screen

parameters (amongst which the vertical screen size) from a database of

standard resolutions, or from the (scarce!) user-settable modes. If

you succeed in using one of those default modes, that saves you one

more free user-settable mode. Taxan 875 monitors for example have only

4 programmable modes, and when you've just tweaked X-windows into some

weird modes, you might run out of user-settable modes. It might be a

good idea to try to create standard SVGA timings for your textmodes

(including the correct sync polarities), so your monitor recognises

them.

(6) Font size: "font HxV" selects the horizontal size (character width =

H) and vertical size (character heigth = V) of the textmode font. The

possible ranges are:

H = 8 or 9,

V = 1 to 32 inclusive. Don't be surprised if you cannot

read text of only 1 pixel high!

The VGA chip must know these values in order to render the font

correctly. The font size together with the H- and V-timings from (2)

and (3) determine the number of characters per line, and the number of

lines per screen.

If the font size is not defined, 8x16 will be assumed .

*******************************************************************************

*IMPORTANT: if you want to test a new mode, you'd better do:                  *

*                                                                             *

*              SVGATextMode -n <MyNewMode>          (note the '-n' option)    *

*                                                                             *

*           before doing the real thing (=without '-n'). This way the program *

*           will show you what the new mode will look like, WITHOUT really    *

*           changing anything. This allows you to check if the H and V sync   *

*           values will work with your monitor.                               *

*                                                                             *

*******************************************************************************

And don't forget: no more than 16K (16384) characters on one screen!