X-Mailer: IPAD 2.0

Date: Wed, 01 Jul 1998 08:11:18 +0300

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Marco Denicolai <marco@vistacom.fi>

Subject: What next for longer sparks?

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

After 3 hours of running my TC I got a strike from toroid to the primary that

killed one of my doubler diodes. I had no strike rail that time. I replaced

the single diodes with two diode pairs (two diode in series). In parallel

with each of the 9000KV 350mA diodes a 33Mohm 3.5KV resistor and a series of

two 1.5nF 3KV capacitors. Guys, that was the best I could get from the local store!

I also grounded back my strike rail.

In a few days I'll be testing this new setup.

Anyway, after initial excitement, I was able to precisely measure strike

length to a grounded rod and that was 20" (not 1 meter as I wrote some time

ago). That means that the primary-secondary raises my tank HV roughly by a

factor of 60 (from 6.5 KV to 400 KV).

The voltage drop in my tank (from 8KV to 6.5KV) is due to the third MOT which

works as a reactor in series with the MOT primary windings. Without that I

always blow my 10A fuse.

My questions are:

- is 20" the best I can get for my tank and my secondary size?

- what should I do to increase the streamer length?

I ask this because I feel (primary capacitor size, MOT max current, etc.) I

have got plenty of power to get longer strikes. What is the best way to go:

- bigger toroid?

- bigger secondary?

- more coupling (I didn't go lower than 1.5", I didn't reach overcoupling yet)?

- different spark gap?

*************************************************************************

My coil data:

Secondary 4.33" diameter, 19.7" wind with AWG 26 enamel wire, 7 layers of

epoxy paint. Toroid 3.15" diameter tube, 11.4" outer diameter, 12.7 pF.

Mounted 1.6" over the secondary top. The pair resonates at 240 KHz.

Primary 7 turns of 3/16" copper tube, inner diameter 5.3", turn spacing

0.3". Flat pancake. Clearance from inner turn to secondary 0.5".

Secondary capacitor 0.05uF 50KV. Set to resonate also at 240 KHz.

Secondary first turns 1.5" higher than primary first turn.

Tank supply made of two MOTs and two voltage doublers, output 6.5 KVAC,

almost 0.5A (I believe). A third MOT primary (with shorted secondary)

is in series with the 2 MOT primaries on the 220V side to work as a shunt.

Spark gap is a RQ static gap with blower and two series of 2 of 0.275" gaps

connected in parallel.

________________________________________________________________________

Marco Denicolai

From: "Eriksson" <jan-erik.eriksson@mbox2.swipnet.se>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Cu. tubing dia.?

Date: Wed, 1 Jul 1998 07:32:51 +0200

X-Mailer: IPAD 2.0

X-MimeOLE: Produced By Microsoft MimeOLE V4.72.2106.4

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Hi again, I have been away from this list for a while now but have started building

a new tesla coil. Its a 4" (110mm) coil with 21" winding with #24 wire. I need some

help with the diameter of my primary wire. Ive planed to use copper tubing and wonder how thick it should be? If everything goes along i will use a 60mA 15kV tranny if that will help you.

Thanks,

/Christopher Eriksson

Date: Wed, 01 Jul 1998 06:14:16 -0500

From: Bert Hickman <bert.hickman@aquila.com>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Organization: Stoneridge Engineering

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Cu. tubing dia.?

Sender: tesl-err@emachine.com

Eriksson wrote:

>

> Hi again, I have been away from this list for a while now but have started

> building a new tesla coil. Its a 4" (110mm) coil with 21" winding with #24

> wire. I need some help with the diameter of my primary wire. Ive planed to

> use copper tubing and wonder how thick it should be? If everything goes along

> I will use a 60mA 15kV tranny if that will help you.

> Thanks,

> /Christopher Eriksson

Christopher,

At this power level, 1/4" diameter will be fine. However, if you are

planning to increase the power level any time sonn, you may want to go

with 3/8" tubing now rather than have to wind another primary. Good

luck!

-- Bert --

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Date: Wed, 01 Jul 1998 04:28:28 -0700

From: "Leben Lang und Frei" <fleischwulf@mailexcite.com>

Cc:

X-Sent-Mail: off

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

Subject: Re: What next for longer sparks?

X-Sender-Ip: 208.153.7.9

Sender: tesl-err@emachine.com

Marco,

I have obtained 60" arcs from a 4.5" diameter coil and I believe Mike Hammer

has also. I believe you need to raise the voltage to 12,000 volts minimum.

As Mike Hammer has so often stated in his post, " The current isn't that important.

My tank circuit is capable of delivering 1.5 amps at 14.4 Kv. The most I have used

is .5 amps.

I usually run at .18 amps. The only difference I notice between .5 amp and .18 amps

is that .5 amps seems to produce hotter sparks not longer ones and system has trouble dealing with the heat. If you are limited to 6.5 Kv. I would suggest more top load.

Also at the higher current levels your gap will eventially burn up. No matter how

much air you pump through it, it just won't last very long. My best

performance was accomplished using .180 ma.

I hope this helps

Paul Marshall

---

Einigkeit und Freiheit

>After 3 hours of running my TC I got a strike from toroid to the primary that

>killed one of my doubler diodes. I had no strike rail that time. I replaced

>now the single diodes with two diode pairs (two diode in series). In

>parallel with each of the 9000KV 350mA diodes a 33Mohm 3.5KV resistor

>and a series of two 1.5nF 3KV capacitors. Guys, that was the best I could

>get from the local store! I also grounded back my strike rail.

>In a few days I'll be testing this new setup.

>

>Anyway, after initial excitement, I was able to precisely measure strike

>length to a grounded rod and that was 20" (not 1 meter as I wrote same time

>ago). That means that the primary-secondary rises my tank HV roughly by a

>factor of 60 (from 6.5 KV to 400 KV).

>

>The voltage drop in my tank (from 8KV to 6.5KV) is due to the third MOT which

>works as a reactor in series with the MOT primary windings. Without that I

>always blow my 10A fuse.

>

>My questions are:

>

>- is 20" the best I can get for my tank and my secondary size?

>- what should I do to increase the streamer length?

>

>I ask this because I feel (primary capacitor size, MOT max current, etc.) I

>have got plenty of power to get longer strikes. What is the best way to go:

>

>- bigger toroid?

>- bigger secondary?

>- more coupling (I didn't go lower than 1.5", I didn't reach overcoupling yet)?

>- different spark gap?

>

>*************************************************************************

>My coil data:

>

>Secondary 4.33" diameter, 19.7" wind with AWG 26 enamel wire, 7 layers of

>epoxy paint. Toroid 3.15" diameter tube, 11.4" outer diameter, 12.7 pF.

>Mounted 1.6" over the secondary top. The pair resonates at 240 KHz.

>

>Primary 7 turns of 3/16" copper tube, inner diameter 5.3", turn spacing

>0.3". Flat pancake. Clearance from inner turn to secondary 0.5".

>Secondary capacitor 0.05uF 50KV. Set to resonate also at 240 KHz.

>Secondary first turns 1.5" higher than primary first turn.

>

>Tank supply made of two MOTs and two voltage doublers, output 6.5 KVAC,

>almost 0.5A (I believe). A third MOT primary (with shorted secondary)

>is in series with the 2 MOT primaries on the 220V side to work as a shunt.

>

>Spark gap is a RQ static gap with blower and two series of 2 of 0.275" gaps

>connected in parallel.

>

>

>

>________________________________________________________________________

>

> Marco

Date: Wed, 01 Jul 1998 07:16:57 -0500

From: Bert Hickman <bert.hickman@aquila.com>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Organization: Stoneridge Engineering

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: What next for longer sparks?

Sender: tesl-err@emachine.com

Marco and all,

The 30KV/cm air breakdown rule doesn't apply for long gaps and Tesla

Coils. Because of the way the air breaks down under the repetitive

damped RF-wave "hammering" it sees at the top terminal, the coil output

voltage for a 20" arc will be considerably less than 400 KV, and for 20"

streamers is probably less than 200 KV.

You can roughly estimate the maximum output voltage if you know the

primary and secondary (total secondary self capacitance (Cs) and toroid

(Ct)) capacitances or inductances and the approximately breakdown

voltage of your main gap (Vgap):

Vout(max) ~ Vgap*SQRT(Cp/(Cs+Ct)

OR

Vout(max) ~ Vgap*SQRT(Ls/Lp)

In order to increase output voltage, you can increase the primary

voltage and gap settings (may be difficult for your setup), or increase

the size of the primary capacitance. Either will increase the primary

"bang" size - the amount of energy available for transfer to the

secondary when your gap fires. Increasing the secondary inductance may

help if you use a larger diameter coilform and use larger diameter wire

to reduce losses, but this would require making a new secondary.

However, with a larger diameter coilform you may now end up being

underpowered unless you also increase the size of the primary capacitor.

However, increasing output voltage is only part of the story. There are

other things you can try that also may help.

Some other things you can try:

=============================

Improve your primary gap. During operation, energy from the primary LC

circuit is transferred to the secondary each time your gap fires.

Ideally, the gap extinguishes once all the available primary tank energy

has been transferred to the secondary. However, if your gap fails to

quench, the gap continues to fire, letting energy from the secondary now

transfer back into the primary circuit. Since this steals energy that

would other wise go into making stremers, you get poorer coil

performance.

Try blowing forced air into the gaps to help cool them off and drive off

conductive ions. You may also want to try using more gaps with shorter

individual gap spacing by using more copper pipes and forcing a flow of

air through the gaps with a vacuum cleaner motor (a "vacuum gap"). An

improved gap can result in a very dramtic improvement in coil

performance. Until you're able to make improvements in the gap, you may

want to try raising the relative height of the secondary versus the

primary to reduce the coupling coefficient. This may help somewhat,

especially if your gaps are having difficulty quenching (highly probably

with your higher current MOT-powered system).

Make sure your toroid is relatively smooth so that the top doesn't

prematurely "break out" until the peak voltage is almost reached. Smooth

out any wrinkles, especially if these tend to be sources where the

streamers seem to consistently originate. You may also want to try using

a slightly larger Radius of Curvature (ROC) toroid (say 4") to increase

its holdoff voltage. Hot streamers should originate from a single root

when your topload is properly set up. As you increase coil efficiency

it's not uncommon to go through 3-4 differenc torid sizes in order to

hit the optimum for you coil.

Finally, you could try increasing the outer (large) diameter of the

toroid. This will your coil's operating frequency. While this increases

Ct (and actually decreases peak Vout), the benefit of using a lower

operating frequency often results in improved coil sparklength. High

frequency coils simply don't generate long sparks apparently because of

the complex way that energy gets transferred to the streamers.

You may want to be scouting around for neon transformers as well - an

inexpensive 15 KV 60 KV neon could do wonders for your system. However,

these may not be available in your country.

Hope this helps!

-- Bert --

Marco Denicolai wrote:

>

> After 3 hours of running my TC I got a strike from toroid to the primary that

> killed one of my doubler diodes. I had no strike rail that time. I replaced

> now the single diodes with two diode pairs (two diode in series). In

> parallel with each of the 9000KV 350mA diodes a 33Mohm 3.5KV resistor

> and a series of two 1.5nF 3KV capacitors. Guys, that was the best I could

> get from the local store! I also grounded back my strike rail.

> In a few days I'll be testing this new setup.

>

From: Tom Stock <tomstock@geocities.com>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Second coil update

Date: Wed, 1 Jul 1998 07:57:20 -0700

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

Last night I added a second neon in parallel to my 4.5" coil (along with some chokes,safety gap, and a few more bottle caps) and my spark went from 10-12" to

18-20" easy. This thing is way too loud now to run without ear protection.

The neighbors love it I'm sure. I'll be adding poly caps soon.

Questions:

I get the biggest spark with a toilet ball float. Why? I know I'm tuning correctly.

Could it be because the toroids aren't sitting high enough above the secondary??

My largest toroid (4.5" duct wrapped around 8" piepans for an OD of 17" or DTC of 12.5") doesn't break out any sparks into the air. I'm thinking the problem might be that the piepans are only about 1" above my top secondary winding. The bottom edge of the toroid is about 1 to 2 inches lower than the top wind of the secondary.

I can't wait to get rid of the 12 bottles and go for poly!!

Do bottles ever explode? Lately I've been having this fear of a bottle cracking and dumping salt water all over the floor while the coil is running. I suppose a pan underneath would be a great idea in the meantime.

I need to get 36" of spark so I can drag this thing out into the backyard at midnight. I'm going to start it up, and my wife and I are going to stand around it in aluminum foil looking space suits with fishbowls on our heads looking up at the sky. Like I said, my neighbors love me. I'll try to get a friend to take a picture :)

Thanks

-tom

X-Mailer: IPAD 2.0

Date: Wed, 01 Jul 1998 12:34:32 -0500

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: Cu. tubing dia.?

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

At 07:32 AM 7/1/98 +0200, you wrote:

>Hi again, I have been away from this list for a while now but have started building a new tesla coil. Its a 4" (110mm) coil with 21" winding with #24 wire. I need some help with the diameter of my primary wire. Ive planed to use copper tubing and wonder how thick it should be? If everything goes along i will use a 60mA 15kV tranny if that will help you.

>Thanks,

>/Christopher Eriksson

>

Chris,

For that power level (15kv/60) you cant go wrong with

1/4 inch copper tubing or your closest metric equivalent.

1/4 inch copper will actually work quite well at much higher

power levels. Up to 3 or 4 kva. Above that it would be a good

idea to go to 3/8 or even 1/2 inch tubing.

TTYL!

Mike Hammer

mhammer@misslink.net

Date: Wed, 1 Jul 1998 10:40:13 -0700 (PDT)

From: John <froboy83@yahoo.com>

Subject: Diagrams

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

I'm just getting started with tesla coils, and I am looking for a

diagram that would be easy to make my first coil from.  Were can I

find such a diagram?

John

X-Mailer: IPAD 2.0

Date: Wed, 01 Jul 1998 12:48:53 -0500

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Mike Hammer <mhammer@misslink.net>

Subject: Im back on line

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Hello all,

Im back online after a short absence.

At 4:10 PM CDT Monday afternoon a line of severe

thunderstorms spawning numerous tornados ripped

through this part of West Central Illinois.

I just managed to get the dog and myself into the basement

when the front line of the storm hit packing 70 MPH winds.

About 10 minutes later a tornado passed by and my wind guage

broke at 96 MPH. Numerous trees came down but fortunately

I had no damage to the house except for a few shingles.

Two of my neighbors werent so lucky. One place two houses down from

me lost part of his roof and two houses down from him they have

a large Oak tree in their living room.

Fortunately no one was injured. We were quite likely.

The town of Knoxville just 4 miles Northeast of me

was hit pretty bad and most roads are still impassable

due to downed trees and power lines.

Electricity and phone service was just restored late last night

but the hookup is quite temporary. The line voltage is down to

around 107 volts. But at least we got power so Ive got running

water again.

I spent most of the day yesterday and most of this morning

helping neighbors cut up trees and pulling limbs from their houses.

I got lots of practice with my new chain saw.

I have piles of unanswered email and I will slowly get to it

and try to get caught up.

Summer time in tornado alley.

TTYL!

Mike Hammer

mhammer@misslink.net

Date: Wed, 1 Jul 1998 19:49:31 +0100

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Stephen Bell <steve@breakfix.demon.co.uk>

Subject: Foiled Again!

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Hi guys,

I,m just in the process of collecting all the raw materials I

will need to build my first coil.  For the main cap I intend to make a

poly stacked plate unit.  I've got some poly-sheet, thickness unknown

since I've lost my micrometer :-(, but I've been unable to find any

aluminium flashing (it's probably not a common item here in the UK).

Questions:

What thickness does flashing come in?  Is standard household

foil (the stuff you wrap food in) usable?  For general smoothing and

fixing use on a discharge terminal, is plain aluminium tape OK or do you

need to use some fancy EMI screening tape, with conductive adhesive?

Later Dudes!

------------------------------------------

Stephen Bell  (steve@breakfix.demon.co.uk)

------------------------------------------

X-Originating-IP: [199.217.140.22]

From: "Bill the arcstarter" <arcstarter@hotmail.com>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Im back on line

Date: Wed, 01 Jul 1998 12:41:08 PDT

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

Mike wrote:

>Im back online after a short absence.

>At 4:10 PM CDT Monday afternoon a line of severe

>thunderstorms spawning numerous tornados ripped

>through this part of West Central Illinois.

<snip>

>Electricity and phone service was just restored late last night

>but the hookup is quite temporary. The line voltage is down to

>around 107 volts. But at least we got power so Ive got running

>water again.

Quick - now is the *perfect* time to go grab some "extra" pole

transformers from those snapped -off poles!

Be sure to bring your neon voltage tester along.  Is yours good for

14400?

-Bill

Date: Wed, 01 Jul 1998 12:44:06 -0700

From: Gene Waldo <gwaldo@1stnetusa.com>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Organization: S.A.P.I.E.N.T. (Surly, Agressive, Politically Incorrect Enemies of Non Thinkers)

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Im back on line

Sender: tesl-err@emachine.com

Mike Hammer wrote:

>

> Hello all,

>

> Im back online after a short absence.

> At 4:10 PM CDT Monday afternoon a line of severe

> thunderstorms spawning numerous tornados ripped

> through this part of West Central Illinois.

>

> Summer time in tornado alley.

>

> TTYL!

>

> Mike Hammer

> mhammer@misslink.net

Mike glad to hear you're OK!

The weather here has gone real wierd on us, too!  No rain in... oh, must

be going on 2 weeks!

--

Gene Waldo -  gwaldo@1stnetusa.com  - "Igor in Residence"

From: <Dsurfr@aol.com>

Date: Wed, 1 Jul 1998 17:53:01 EDT

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Diagrams

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

John: There are a number of sites with plans on the Web. I suggest you start

with the Tesla Ring site (http://www.webring.org/cgi-

bin/webring?ring=TeslaRing&list) This is a great collection of links to other

coiler's web sites and many have diagrams. Besides you'll get to see a lot of

great coil pics.    Jim

Date: Wed, 01 Jul 1998 20:44:48 -0400

From: Pat & Heather Moser <moser@madbbs.com>

Organization: Planet Cheese Creations

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: NaCl Sol'n-SW Caps: just so you'll know!

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Awe, hell, why don't you kick it up a notch and add 5 Cups of salt?

X-Originating-IP: [206.112.194.89]

From: "Bill the arcstarter" <arcstarter@hotmail.com>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: If pigs could fly!

Date: Wed, 01 Jul 1998 18:11:52 PDT

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

Coilers,

Perhaps I should have said something like:

"You'll get a free pole transformer only when pigs can fly"

or,

"After a large storm, sometimes poor orphaned distribution transformers

are blown out of their lofty nests.  Be careful - they can BITE when

cornered!"

I'm off the list for a few days (gunna go visit the Int'l Tesla Museum

in Co. Sprgs.!) - so send me CC:s of anything along these lines!

Thanks!

-Tornado Bill D. Arcstarter

Date: Wed, 01 Jul 1998 22:12:57 -0500

From: Bert Hickman <bert.hickman@aquila.com>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Organization: Stoneridge Engineering

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Im back on line

Sender: tesl-err@emachine.com

Mike,

Welcome back! Glad to hear you "weathered the storms" - Now all you need

to do is see if the power company needs any help disposing of some of

the downed pigs, PFC's or insulators... :^)

-- Bert --

Mike Hammer wrote:

>

> Hello all,

>

> Im back online after a short absence.

> At 4:10 PM CDT Monday afternoon a line of severe

> thunderstorms spawning numerous tornados ripped

> through this part of West Central Illinois.

>

<SNIP>

Date: Wed, 01 Jul 1998 20:56:13 -0700

From: Gene Waldo <gwaldo@1stnetusa.com>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Organization: S.A.P.I.E.N.T. (Surly, Agressive, Politically Incorrect Enemies of Non Thinkers)

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: If pigs could fly!

Sender: tesl-err@emachine.com

Bill the arcstarter wrote:

>

> Coilers,

>  Perhaps I should have said something like:

>

> "You'll get a free pole transformer only when pigs can fly"

>

> I'm off the list for a few days (gunna go visit the Int'l Tesla Museum

> in Co. Sprgs.!)

>

> -Tornado Bill D. Arcstarter

Bill,

I assume there will be pics of C. Sprgs, upon your return??

--

Gene Waldo -  gwaldo@1stnetusa.com  - "Igor in Residence"

Date: Wed, 01 Jul 1998 21:04:57 -0700

From: Gene Waldo <gwaldo@1stnetusa.com>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Organization: S.A.P.I.E.N.T. (Surly, Agressive, Politically Incorrect Enemies of Non Thinkers)

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Im back on line

Sender: tesl-err@emachine.com

Bert Hickman wrote:

>

> Mike,

>

> Welcome back! Glad to hear you "weathered the storms" - Now all you need

> to do is see if the power company needs any help disposing of some of

> the downed pigs, PFC's or insulators... :^)

>

> -- Bert --

>

> <SNIP>

Bert, would you suppose that Mike would even have to "...see if the

power company needed help disposing..."? I should think that any civil

minded person would simply lend a hand "disposing", with out having to

be asked!

Especially in the light (or absence there-of) of the confusion and dire

straights. It's an ill wind....

------------------------

Gene Waldo -  gwaldo@1stnetusa.com  - "Igor in Residence"

Date: Thu, 02 Jul 1998 01:07:21 -0500

From: Tom Polk <tpolk@prismnet.com>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Capacitor safety

Sender: tesl-err@emachine.com

I found this on a Theremin listserver:

I got involved with high voltage after trying to build a nitrogen laser

featured in Scientific American. I built up a .05 uF 15kV capacitor to

fire flashtubes. It was not potted, and when charged, it hissed like a

snake. And it bit me.

Knocked me silly, I thought I was a goner. Came in through my elbow and

exited through my butt. Buttock, that is. Entrance and exit wounds

included.

It was a peculiar sensation. I was semiconscious for I guess half a

minute or so. Felt like I was a cartoon character that vibrates when

hit. As I gradually came out of the stupor, as frantically as I could, I

checked to see if my heart was still beating.  Well, it was.

I blurted a fervent prayer of gratitude and fresh humility, during which

it sink in how careless I was, and how dangerous high voltage is.

By the next day I had become careless enough to do it again, and I did.

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Date: Thu, 2 Jul 1998 11:47:29 -0500

Subject: New transformer ?'s

X-Mailer: IPAD 2.0

From: rpittman@juno.com

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Mike (glad all is well) and all,

Well just when I think I have all the parts cut and start

assembling the cap for my new neons plans change.

A friend of mine gave me a potential transformer for free :).

Only problem is that there is no face plate on it and thus I am kind of

clueless.  He said they use it to step down 7.2 kv to 120 but he asked

and the guy said it could be run at 220 in to get 14.4 kv out.  The only

thing it says on the outside is hy bute 60 model 110.  (I think the 110

represents the weight as well as the voltage)

Ok here are my questions;

1.  What is the rated current output on one of these things?

2.  What size of a variac would I need to handle this current ?

3. What size of cap would I need ?

4.  Current limiting would can come from a 235 amp welder I have right?

Thanks again for all the help

Adios

Ronnie

X-Mailer: IPAD 2.0

Date: Thu, 02 Jul 1998 12:36:20 -0500

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: New transformer ?'s

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

At 11:47 AM 7/2/98 -0500, you wrote:

>Mike (glad all is well) and all,

>

>	Well just when I think I have all the parts cut and start

>assembling the cap for my new neons plans change.

>	A friend of mine gave me a potential transformer for free :).

Ron my advice is to store this transformer for future use and go

ahead with a neon powered coil. Non current limited transformers

like PTs and pole pigs are exceedingly dangerous. They cannot

be used without extensive external circuitry to control them.

You need the experience with neon coils first.

>Only problem is that there is no face plate on it and thus I am kind of

>clueless.  He said they use it to step down 7.2 kv to 120 but he asked

>and the guy said it could be run at 220 in to get 14.4 kv out.  The only

>thing it says on the outside is hy bute 60 model 110.  (I think the 110

>represents the weight as well as the voltage)

Running it on 220 V will probably result in core saturation

and very high primary current draw and a quickly overheating

transformer. I wouldnt even attempt it.

>	Ok here are my questions;

>1.  What is the rated current output on one of these things?

Most are rated at 1.5 kva. So at 7200 volts that comes to

208 ma. Most will deliver more than their rating for short periods.

>2.  What size of a variac would I need to handle this current ?

You will need a 120 Volt 1.5kva variac at a minimum. Thats  12.5 amps

at 120 volts. So a 20 amp variac would be a good choice.

>3. What size of cap would I need ?

Sizing the cap to the transformer only really applies to internally

limited transformers like neons. You would want to use as large a cap

as your system will allow. This means matching your secondary resonant

frequency with a sufficient number of primary coil turns to allow

good operation.

Once you start using PTs or pole pigs cap size is determined by

the size of your coil not the transformer.

>4.  Current limiting would can come from a 235 amp welder I have right?

Yes as long as the welder has a variable shunt. You will find

you will also need some resistive ballast to smooth out inductive

surges. These surges can feed back into your AC mains and wreak havoc

with household electronics.

Again I suggest you store this transformer for future use

or consider trading it for some neons. It represents a power

level and complexity you dont need until you have more experience.

TTYL!

Mike Hammer

mhammer@misslink.net

From: <Dsurfr@aol.com>

Date: Thu, 2 Jul 1998 14:31:50 EDT

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: dryer duct toroid

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

All the pics of toroids I have seen seem to be quite smooth (which they should

be). All the foil dryer duct I have been able to find so far has a nylon? mesh

sandwiched in the foil & results in leaving the foil between the rings

wrinkled quite badly when the tube is stretched back out into a circle. Is

there some other type of duct you are using or do you wrap them in foil tape

or what? I'm using what I have for now but I can see all the loss from the

wrinkles when I run it.  Thanks Jim

X-Mailer: IPAD 2.0

Date: Thu, 02 Jul 1998 14:55:34 -0500

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: dryer duct toroid

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

At 02:31 PM 7/2/98 EDT, you wrote:

>All the pics of toroids I have seen seem to be quite smooth (which they should

>be). All the foil dryer duct I have been able to find so far has a nylon? mesh

>sandwiched in the foil & results in leaving the foil between the rings

>wrinkled quite badly when the tube is stretched back out into a circle. Is

>there some other type of duct you are using or do you wrap them in foil tape

>or what? I'm using what I have for now but I can see all the loss from the

>wrinkles when I run it.  Thanks Jim

Wrong kind of duct Jim. The kind to get is all aluminum.

Its not called anything particular except flexible aluminum

dryer duct. I bought mine in 3, 4 and 6 inch diameters at

a local lumber yard.

TTYL!

Mike Hammer

mhammer@misslink.net

X-Mailer: IPAD 2.0

Date: Thu, 02 Jul 1998 15:00:47 -0500

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: Foiled Again!

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

At 07:49 PM 7/1/98 +0100, you wrote:

>Hi guys,

>        I,m just in the process of collecting all the raw materials I

>will need to build my first coil.  For the main cap I intend to make a

>poly stacked plate unit.  I've got some poly-sheet, thickness unknown

>since I've lost my micrometer :-(, but I've been unable to find any

>aluminium flashing (it's probably not a common item here in the UK).

>

>Questions:

>What thickness does flashing come in?

The flashing generally available here in the USA is meant for

roofing use. Its put in the valleys on your roof to channel away

the water. It is approximately 10 mils thick.

>Is standard household foil (the stuff you wrap food in) usable?

Yes. But use the heavy duty variety. Its thicker and doesnt

wrinkle as easily.

>For general smoothing and fixing use on a discharge terminal,

>is plain aluminium tape OK or do you need to use some fancy EMI

>screening tape, with conductive adhesive?

>

>Later Dudes!

>

>------------------------------------------

>Stephen Bell  (steve@breakfix.demon.co.uk)

>------------------------------------------

Plain aluminum tape is just fine.

TTYL!

Mike Hammer

mhammer@misslink.net

From: <Dsurfr@aol.com>

Date: Thu, 2 Jul 1998 18:31:54 EDT

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: dryer duct toroid

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Mike: Thanks for the advice, I thought it must be something like this. In

between the query & the response I was playing with my 3 year old son & came

across a "Slinky". Has anyone ever tried taking one of these & forming a

toroid, wrapped it with plastic strapping tape & then applied aluminum tape

to form an aluminum foil toroid? (it was a plastic "Slinky" not the metal

coils of my own youth) Necessity may indeed be the Mother of Invention.

Thanks again for the quick reply,   Jim

Date: Thu, 02 Jul 1998 15:48:03 -0700

From: Gene Waldo <gwaldo@1stnetusa.com>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Organization: S.A.P.I.E.N.T. (Surly, Agressive, Politically Incorrect Enemies of Non Thinkers)

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: NaCl Sol'n-SW Caps: just so you'll know!

Sender: tesl-err@emachine.com

I think Mike was right (again).  He said just put some salt in your

water, or words to that effect.

I have just completed a moderately scientific test of the question.

With my trusty VOM, a cup of tap water, and some kitchen measuring gear,

here are the results:

Plotting Resistance, kOhms(x) v Salt Content, tsp/Cup(y):

Plain tap water Ohms = 35k

My curve became tangent to 45 a deg slope (point of dim. ret) at .55

tsp/cup=16.5kOhms.

Between 1.5 and 2.5 tsp/Cup the curve was almost vertical at ~12kOhms.

I wouldn't use more than .75 - 1 tsp/Cup.

Next, how would sugar work as an electrolyte? At least when your polys

were done, you could make lemonade!

BETTER THINGS FOR BETTER LIVING THROUGH CHICANERY!

-------------------------------------

Gene Waldo -  gwaldo@1stnetusa.com  - "Igor in Residence"

Date: Fri, 03 Jul 1998 00:50:46 -0500

From: Bert Hickman <bert.hickman@aquila.com>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Organization: Stoneridge Engineering

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: New transformer ?'s

Sender: tesl-err@emachine.com

Ronnie,

Congratulations on the freebie! You actually have the equivalent of a

"piglet". :^) I agree with Mike's assessment that your best bet,

particularly if this is your first coil, is to stick with a neon

transformer power source. Some additional information is provided

below...

rpittman@juno.com wrote:

>

> Mike (glad all is well) and all,

>

>         Well just when I think I have all the parts cut and start

> assembling the cap for my new neons plans change.

>         A friend of mine gave me a potential transformer for free :).

> Only problem is that there is no face plate on it and thus I am kind of

> clueless.  He said they use it to step down 7.2 kv to 120 but he asked

> and the guy said it could be run at 220 in to get 14.4 kv out.  The only

> thing it says on the outside is hy bute 60 model 110.  (I think the 110

> represents the weight as well as the voltage)

While you might be able to drive it to 140 VAC with no problem, you'll

begin saturating the core long before you hit 220 VAC. You can easily

test this by using a 240 volt variac. Connect the LV side of the

potential transformer through an ammeter to the output of the variac.

Leavt the HV side of the potential transformer unconnected (no load).

Now slowly increase the voltage while measuring the primary current.

You'll reach a point where the primary current begins to climb very

rapidly, and the transformer will begin to hum/growl. This is the onset

of core saturation, and attempting to increase the primary voltage much

further will have little impact on increasing the output of the

transformer.

If you could find another potential transformer like the one you have,

you can tie the secondaries in series (with the common "center tap"

connected to ground) to get an equivalent 14.4 KV transformer that can

handle 6-8 KVA for Tesla coil use once you've gotten enough prior

experience using neons.

>         Ok here are my questions;

> 1.  What is the rated current output on one of these things?

It depends upon the weight of the core - yours sounds like its rated at

the typical 1.5 KVA. These will easily handle 3 - 4 KVA for short

run-times. This will give you up to 500 MA at 8400 volts.

> 2.  What size of a variac would I need to handle this current ?

If you operate the unit off a 0-140 Volt variac, a 28A variac will do

the job nicely.

> 3. What size of cap would I need ?

Getting a high enough operating voltage to consistently operate a spark

gap (a rotary gap will virtually be essential at the 4 KVA level) would

be a very difficult problem. Assuming your maximum RMS output voltage is

about 8400 volts, and maximum power level is governed by external

ballasting, the maximum cap size you'd be able to drive can be

estimated:

Input Power   Xfmr Output current

(KVA @140 VAC)  (MA at 60 Hz)      Max Tank Cap

1.0             120               0.039 uF

1.5             180               0.057 uF

2.0             240               0.076 uF

2.5             300               0.095 uF

3.0             360               0.113 uF

3.5             420               0.133 uF

4.0             480               0.153 uF

> 4.  Current limiting would can come from a 235 amp welder I have right?

Should work OK for 0-140 VAC use.

>

> Thanks again for all the help

>

> Adios

> Ronnie

>

Watch out - a potential transformer is just a lethal as a pole pig! A

neon-powered coil will generate very impressive sparks and is very much

easier to control.

Good luck, and safe coiling to you!

-- Bert --

Date: Thu, 02 Jul 1998 23:37:28 -0700

From: Scott MacGregor <smacgregor@aasp.net>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Organization: EES,Co.

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: NaCl Sol'n-SW Caps: just so you'll know!

Sender: tesl-err@emachine.com

Gene Waldo wrote:

>

> Pat & Heather Moser wrote:

> >

> > Awe, hell, why don't you kick it up a notch and add 5 Cups of salt?

>

> Where did y'alls learn to make lemonade?

> --

> Gene Waldo -  gwaldo@1stnetusa.com  - "Igor in Residence"

Hi Gene,

Based on some experiments I did in H.S., (also done by others a zillion

years ago) you can increase the concentration of the NaCl by increasing

the temperature of the solvent. (in this case water) Try bringing the

water to a boil, then add NaCl, slowely, till you can't dissolve any

more. Then measure the resistance. Want LESS resistance? Enclose the

solution in a pressure vessle, and raise the temperature! How low can

you go? (Ohm, wise) ? Can NaCl remain in ionized form in super-heated

steam? If you go to extremes, maybe you could get those 5 cups of salt

into a teaspoon of water!!

--

Regards,

Scott MacGregor EES,Co.

Date: Fri, 03 Jul 1998 07:07:14 -0700

From: Gene Waldo <gwaldo@1stnetusa.com>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Organization: S.A.P.I.E.N.T. (Surly, Agressive, Politically Incorrect Enemies of Non Thinkers)

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: NaCl Sol'n-SW Caps: just so you'll know!

Sender: tesl-err@emachine.com

Scott MacGregor wrote:

>

> Gene Waldo wrote:

> >

> > Pat & Heather Moser wrote:

> > >

> > > Awe, hell, why don't you kick it up a notch and add 5 Cups of salt?

> >

> Hi Gene,

>

> Based on some experiments I did in H.S.,  get those 5 cups of salt

> into a teaspoon of water!!

> --

>

> Regards,

> Scott MacGregor EES,Co.

> E-mail smacgregor@aasp.net

This stupid SW cap is already as complex as I think it should get!

Perhaps someone else can do the work on the superheated regenerative

ionizing steam capacitor! There could be laser measurements of the

disassociated free Na & Cl radicals; perhaps magnetohydrodynamic

dielectric containment; TOCOMAC principles may apply....

Packets of charged ionized gas could be fired into a grid system to

energize the primary coil (eliminating the need for the linear gap;

which is, after all, quite old technology), and a glass primary coil,

super cooled, also filled with a superconductive gas!

The 2ndary should also be wound on a supercooled form... Copper should

be replaced by newer superconductive materials....

What about a simple conductive gas capacitor? Large thermos-bottle-like

capacitors filled with neon or other neat gasses! Pretty!

LEO operation of this system should be spectacular! The long wire to

RFGnd, however, may present some engineering problems!  Who has contacts

at NASA?

We should plan now to move beyond 1880's technology!

For now, I think that ~1 tsp/cup should do, unless there is actual

experience indicating that more is better.

--------------------------------------------------------------- ~~<[;-)

Gene Waldo -  gwaldo@1stnetusa.com  - "Igor in Residence"

X-Mailer: IPAD 2.0

Date: Fri, 03 Jul 1998 17:18:28 -0500

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: Diagrams

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

At 10:40 AM 7/1/98 -0700, you wrote:

>I'm just getting started with tesla coils, and I am looking for a

>diagram that would be easy to make my first coil from.  Were can I

>find such a diagram?

>

>John

>

>

John,

Schematic diagrams for Tesla coils are all pretty much the

same. There is some variation of component placement but otherwise

are identical. The configuration having the spark gap across

the transformer is preferred as it lessens the stresses on

the transformer.

I will paste my almost famous ascii coil schematic here for you.

TESLA COIL SCHEMATIC                             ---------

|       | T1

---------

O

X1                              C1              O

RFC           | |             O  L2

O---------------||||----------| |-----        O

||O    |                   |    | |    |   L1   O

||O    |                   |           |    O   O

-----O||O    * SG                *           |--->O   O

O||O----*---|Gnd              G1             O   O

-----O||O    *                   *                O   O

120   ||O    |                   |                O   O

Volts ||O    |                   |                O   O

O---------------||||----------------------O   O

RFC                           |

|

Gnd

X1 = Neon Sign Transformer, grounded case

SG = Safety Gap.

RFC = RF filter chokes

G1 = Main system (tank circuit) Spark Gap.

C1 = Tank circuit capacitor

L1 = Primary Coil.

L2 = Secondary Coil.

T1 = Toroid discharge terminal

Gnd = RF Ground.

If this gets jumbled by your email program you can get

a similar schematic from my web page at:

http://www.misslink.net/mhammer/coil.htm

TTYL!

Mike Hammer

mhammer@misslink.net

X-Mailer: IPAD 2.0

Date: Fri, 03 Jul 1998 17:29:09 -0500

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: Second coil update

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

At 07:57 AM 7/1/98 -0700, you wrote:

>

>Last night I added a second neon in parallel to my 4.5" coil (along with some >chokes,safety gap, and a few more bottle caps) and my spark went from 10-12"

>to 18-20" easy. This thing is way too loud now to run without ear protection.

>The neighbors love it I'm sure. I'll be adding poly caps soon.

>

>Questions:

>

>I get the biggest spark with a toilet ball float. Why? I know I'm tuning correctly. >Could it be because the toroids aren't sitting high enough above the secondary?

>My largest toroid (4.5" duct wrapped around 8" piepans for an OD of 17" or DTC

>of 12.5") doesn't break out any sparks into the air. I'm thinking the problem

>might be that the piepans are only about 1" above my top secondary winding.

>The bottom edge of the toroid is about 1 to 2 inches lower than the top wind

>of the secondary.

You may be getting longer sparks because your coil is not generating

quite a high enough voltage for the toroid. You can add a breakout point

to your toroid to induce breakouts. I use a bump made from aluminum tape

on the outer edge of the toroid. This also provides a measure of

directional control.

The height of the toroid above the secondary will not materially

affect the output. It will vary the resonant frequency somewhat.

The best plan is to mount it just low enough to suppress corona

at the top of the secondary winding. Keep it as high as possible

to increase the toroid/primary coil distance to discourage strikes

into the primary coil.

>

>I can't wait to get rid of the 12 bottles and go for poly!!

It will make a huge difference. Your sparks wont be a lot longer.

Maybe only a couple inches. But they will be brighter, thicker

and louder. It may also give you that extra punch you need to break

that larger toroid out.

>

>Do bottles ever explode? Lately I've been having this fear of a bottle cracking

>and dumping salt water all over the floor while the coil is running. I suppose

>a pan underneath would be a great idea in the meantime.

While I have heard of it happening its not too common. When I ran

bottle caps all I ever had was a loud "tink" then silence.

>

>I need to get 36" of spark so I can drag this thing out into the backyard at

>midnight. I'm going to start it up, and my wife and I are going to stand around

>it in aluminum foil looking space suits with fishbowls on our heads looking up

>at the sky. Like I said, my neighbors love me. I'll try to get a friend to take

>a picture :)

If you take such a picture be sure to destroy the negatives.

Never know when that might be used against you. :)

TTYL!

Mike Hammer

mhammer@misslink.net

Date: Fri, 03 Jul 1998 20:21:50 -0700

From: Gene Waldo <gwaldo@1stnetusa.com>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Organization: S.A.P.I.E.N.T. (Surly, Agressive, Politically Incorrect Enemies of Non Thinkers)

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Wire use ?

Sender: tesl-err@emachine.com

Mike and Bert,

And anyone who may have an opinion-

I have some interesting wire (cable, technically?), what service limits

would you place on it?

I don't know from the wire gage, but it is a twisted assy in good tough,

vinyl? insulation (flame test?).

The wire bundle mikes at .106" and is in 3 concentric lays: central: 7

strands, twisted, ccw;  mid: 12 strands, wrapped cw; outer: 18 strands,

wrapped ccw. Ea strand is a fairly stiff copper, silver plated, and they

each mike at .0155". I make the total conductive surface (circumference)

per unit length at 1.802".

The insulated conductor mikes at .141". And, it was sort of expensive!

At the moment, I'm bussing the SW caps together with it. I figure it

should be good upto the primary?  It may have value beyond there?

My hunch is that silver was in part used for hiFreq work?

--

Gene Waldo -  gwaldo@1stnetusa.com  - "Igor in Residence"

Date: Sat, 04 Jul 1998 00:42:39 -0700

From: Scott MacGregor <smacgregor@aasp.net>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Organization: EES,Co.

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Wire use ?

Sender: tesl-err@emachine.com

Gene Waldo wrote:

>

> Mike and Bert,

> And anyone who may have an opinion-

>

> I have some interesting wire (cable, technically?), what service limits

> would you place on it?

> I don't know from the wire gage, but it is a twisted assy in good tough,

> vinyl? insulation (flame test?).

>

> The wire bundle mikes at .106" and is in 3 concentric lays: central: 7

> strands, twisted, ccw;  mid: 12 strands, wrapped cw; outer: 18 strands,

> wrapped ccw. Ea strand is a fairly stiff copper, silver plated, and they

> each mike at .0155". I make the total conductive surface (circumference)

> per unit length at 1.802".

>

> The insulated conductor mikes at .141". And, it was sort of expensive!

>

> At the moment, I'm bussing the SW caps together with it. I figure it

> should be good upto the primary?  It may have value beyond there?

>

> My hunch is that silver was in part used for hiFreq work?

>

> --

> Gene Waldo -  gwaldo@1stnetusa.com  - "Igor in Residence"

Gene,

I think that would be about 9 or 10 AWG. 20-26 Amps at normal cooling

for copper. With silver plated? Should be good hook-up wire.

--

Regards,

Scott MacGregor EES,Co.

************************************************************

X-Mailer: IPAD 2.0

Date: Sat, 04 Jul 1998 18:42:34 +0100

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: bob golding <yubba@clara.net>

Subject: rotarys and gennys

Cc: tesla@pupman.com

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Hi all

I have been offered a 25 kva generator with a separate exciter motor. I

plan to use this with a pole pig when one turns up. I am planning to

connect a variac between the exciter motor and the main alternator to give

me the equivalent of a 125 amp variac. I know, I will need a bigger garage.

It then occurred to me that I might be able to drive a rotary gap direct

off the alternator shaft to give a true synchronous output with  phase

angle adjustment accomplished by making the fixed contacts adjustable. Does

this sound workable, sensible?, possible. Have I missed anything. All

feedback welcome.

bob golding lightning without rain

bob yubba@clara.net

Date: Sun, 05 Jul 1998 21:06:06 -0500

From: Bert Hickman <bert.hickman@aquila.com>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Organization: Stoneridge Engineering

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: rotarys and gennys

Sender: tesl-err@emachine.com

bob golding wrote:

>

> >Date: Sat, 04 Jul 1998 18:42:34 +0100

> >To: tesla-2@emachine.com

> >From: bob golding <yubba@clara.net>

> >Subject: rotarys and gennys

> >Cc: tesla@pupman.com

> >

> >Hi all

> >       I have been offered a 25 kva generator with a separate exciter motor. I

> plan to use this with a pole pig when one turns up. I am planning to

> connect a variac between the exciter motor and the main alternator to give

> me the equivalent of a 125 amp variac. I know, I will need a bigger garage.

> It then occurred to me that I might be able to drive a rotorary gap direct

> off the alternator shaft to give a true synchronous output with  phase

> angle adjustment accomplished by making the fixed contacts adjustable. Does

> this sound workable, sensible?, possible. Have I missed anything. All

> feedback welcome.

> >

> >bob golding lightning without rain

> >

> >bob yubba@clara.net

Bob,

You may need a bigger yard! :^)

Connecting the rotary to the alternator shaft with adjustable stationary

electrodes should indeed give you true synchronous operation. However,

running at this power level will require some careful gap design and

substantial electrodes (3/8" tungsten or elkonite).

Good luck taming this beastie!

-- Bert --

X-Mailer: IPAD 2.0

Date: Tue, 07 Jul 1998 08:49:01 +0300

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Marco Denicolai <marco@vistacom.fi>

Subject: First change for longer sparks

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Thanks to everybody who answered my previous email. Among many suggestions a

bigger toroid seemed to be common.

So I replaced my toroid 3.15" diameter tube, 11.4" outer diameter, 12.7 pF,

with a toroid 3.93" diameter tube, 15.7" outer diameter, about 17 pF. The

resonance frequency dropped from 240KHz to about 195KHz and I tuned the

primary tap again.

I got less streamers breaking into air (about 5-6 at the same time, compared

to maybe 10-15 with smaller toroid). These streamers where max. 31" long

(before where max. 20" long).

Still (without using any sharp nail fixed on the toroid) I couldn't manage

to get discharges to a grounded point longer than 24" (compared to 20" before).

Please, recall that my tank supplies only about 6.5 KV (NOT a 14KV NST!).

Because my spark gap is a RQ static gap with blower and two series of 2 of

0.0275" gaps connected in parallel, I think my next step will be to go for

a RQ static gap with 7 mils gaps, to get better quenching and easier

tunability of the gap number.

Any comments or suggestions?

P.S. Yes, my sparks gap gaps are really 0.0275", not 0.275" as from my

previous email. Sorry, I lost one zero :)

Marco Denicolai

X-Mailer: IPAD 2.0

Date: Tue, 07 Jul 1998 09:01:09 +0300

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Marco Denicolai <marco@vistacom.fi>

Subject: Shocking secondary

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Everytime I remove the secondary and I touch its ground connection I get

nice electric shocks. So yesterday I checked this a little bit.

I put the secondary lying on the table (no electrical connections to anything),

I touched the ground connection with an electrician screwdriver (one with the

neon lamp inside) and then I simply put my hand on the coil varnished part

(the TC was of course off).

The neon lamp was glowing bright! It really looked like a capacitor discharge

that lasted some 2-3 seconds. Touching again I could repeat the discharge

effect as many times I wanted.

And I didn't use any wool cloth or friction, just touching the coil...

So if the coil self capacitance is about 8 pF and its resistance about 54 Ohm

I think what we got here is... just electrostatic charging of the insulating

varnish on the secondary?

________________________________________________________________________

Marco Denicolai

Date: Tue, 07 Jul 1998 06:49:54 -0500

From: Bert Hickman <bert.hickman@aquila.com>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Organization: Stoneridge Engineering

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Shocking secondary

Sender: tesl-err@emachine.com

Marco,

Electrostatic charging of the outer surface is exactly what you're

seeing. The thicker the outer coating (assuming it's a good dielectric)

the greater the stored charge. During operation there's a bit of corona

generated from the coil's outer surface. This causes a potential

difference to develop between the inner portion (secondary winding) and

the outer surface. Once you turn the coil off, the winding is typically

at ground (from the grounded base connection), leaving "stranded" charge

at high voltage on the outer surface. With a good dielectric, this

charge can remain for hours or even days during winter. The stranded

charge will act like a small "dust magnet", and will definately shock

you if you happen to charge your body capacitance while bleeding it off

with your hand. This can be an especially shocking experience if you

lift the coilform by it's base to move it and accidently touch your

forehead to the top! The voice of experience... :^)

-- Bert --

Marco Denicolai wrote:

>

> Everytime I remove the secondary and I touch its ground connection I get

> nice electric shocks. So yesterday I checked this a little bit.

>

> I put the secondary lying on the table (no electrical connections to anything),

> I touched the ground connection with an electrician screwdriver (one with the

> neon lamp inside) and then I simply put my hand on the coil varnished part

> (the TC was of course off).

>

> The neon lamp was glowing bright! It really looked like a capacitor discharge

> that lasted some 2-3 seconds. Touching again I could repeat the discharge

> effect as many times I wanted.

>

> And I didn't use any wool cloth or friction, just touching the coil...

>

> So if the coil self capacitance is about 8 pF and its resistance about 54 Ohm

> I think what we got here is... just electrostatic charging of the insulating

> varnish on the secondary?

>

> ________________________________________________________________________

>

>  Marco Denicolai

Date: Tue, 07 Jul 1998 07:08:30 -0500

From: Bert Hickman <bert.hickman@aquila.com>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Organization: Stoneridge Engineering

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: First change for longer sparks

Sender: tesl-err@emachine.com

Marco,

Congratulations - You just got a 50% improvement in sparklength!

For further improvement, you need to go to a larger Radius of Curvature

on your toroid to delay breakout. Try going to 5", or even 6", diameter

ducting. This will allow the coil's secondary voltage to rise to a

significantly higher voltage before the onset of corona breakdown. This

also helps you form single, high current, streamers. The multiple

streamers you're seeing are a symptom of premature breakout.

Insufficient ROC is one of the reasons why your sparks to ground are not

much longer with the new toroid. The other reason is that the maximum

output voltage will tend to go down as you increase topload C. The

output voltage can be increased by increasing the size of your tank

capacitance (as long as your supply can still fully recharge it between

breaks).

Safe coilin' to you!

- Bert --

Marco Denicolai wrote:

>

> Thanks to everybody who answered my previous email. Among many suggestions a

> bigger toroid seemed to be common.

>

> So I replaced my toroid 3.15" diameter tube, 11.4" outer diameter, 12.7 pF,

> with a toroid 3.93" diameter tube, 15.7" outer diameter, about 17 pF. The

> resonance frequency dropped from 240KHz to about 195KHz and I tuned the

> primary tap again.

>

> I got less streamers breaking into air (about 5-6 at the same time, compared

> to maybe 10-15 with smaller toroid). These streamers where max. 31" long

> (before where max. 20" long).

>

> Still (without using any sharp nail fixed on the toroid) I couldn't manage

> to get discharges to a grounded point longer than 24" (compared to 20" before).

>

> Please, recall that my tank supplies only about 6.5 KV (NOT a 14KV NST!).

>

> Because my spark gap is a RQ static gap with blower and two series of 2 of

> 0.0275" gaps connected in parallel, I think my next step will be to go for

> a RQ static gap with 7 mils gaps, to get better quenching and easier

> tunability of the gap number.

>

> Any comments or suggestions?

>

> P.S. Yes, my sparks gap gaps are really 0.0275", not 0.275" as from my

> previous email. Sorry, I lost one zero :)

>

> ________________________________________________________________________

>

>  Marco Denicolai

From: Tom Stock <tomstock@geocities.com>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: RE: Shocking secondary

Date: Tue, 7 Jul 1998 09:18:39 -0700

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

OH SURE, now you tell me. I discovered this the hard way a couple of times. My latest gimmick has been to run the coil for awhile, discharge the caps (of course), and then have one of my friends "help" me put away the coil..."Hey just grab the secondary there with both hands so you don't drop it..."

Works every time. I've done it to myself a dozen times so I know it's not enough to really hurt (at least with my small secondary), but it's definitely a surprise for a first timer!

-tom

> The stranded charge will act like a small "dust magnet", and will

> definitely shock you if you happen to charge your body capacitance

> while bleeding it off with your hand. This can be an especially shocking  > experience if you lift the coilform by it's base to move it and accidently

> touch your forehead to the top! The voice of experience... :^)

-- Bert --

Marco Denicolai wrote:

>

> Everytime I remove the secondary and I touch its ground connection I get

> nice electric shocks. So yesterday I checked this a little bit.

>

> I put the secondary lying on the table (no electrical connections to anything),

> I touched the ground connection with an electrician screwdriver (one with the

> neon lamp inside) and then I simply put my hand on the coil varnished part

> (the TC was of course off).

>

> The neon lamp was glowing bright! It really looked like a capacitor discharge

> that lasted some 2-3 seconds. Touching again I could repeat the discharge

> effect as many times I wanted.

>

> And I didn't use any wool cloth or friction, just touching the coil...

>

> So if the coil self capacitance is about 8 pF and its resistance about 54 Ohm

> I think what we got here is... just electrostatic charging of the insulating

> varnish on the secondary?

>

> ________________________________________________________________________

>

>  Marco Denicolai

Date: Tue, 07 Jul 1998 09:27:28 -0700

From: Gene Waldo <gwaldo@1stnetusa.com>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Organization: S.A.P.I.E.N.T. (Surly, Agressive, Politically Incorrect Enemies of Non Thinkers)

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

CC: TESLA-2@emachine.com

Subject: Re: Formica spacers

Sender: tesl-err@emachine.com

Dan Murphy wrote:

>

> At 05:35 PM 7/2/98, Gene  wrote:

>

> >Dan,

> >I'm to that point, now. 6 more joints to solder, but should have spacers

> >for this.

>   (I printed out the list, will take it with me to the shop tommorrow, but

> can't guarantee I'll have time to get it cut...)

That works. Perhaps, sufficient odd raw scrap material, and I can probably

find time to chop it up? Also, the ply bottom pc is not for the

interior! it must support the bottom, from beneath.

>I don't give a rip about the damn box... I guess that's how you get

> your jollies....

Ah! A major seachange and relaxation of the rules of the road! I assume

tacit permission to employ the thru-hull 'siphon'!

> Punch holes in the top of the box, that way there's easy access to the

> terminals for changing the number of bottles connected!

I plan no need or requirement to change the number of connections. As a

research interim expedient, the proposed SW cap should provide a

CALIBRATED variable capacitance from ~0-max mfd for the 12 bottles. The

derived capacitance values for any of several terminal setups should be

directly convertable to target values for poly/plate cap construction.

The 'Tune-on-the-fly' capacitor should provide us the means of tweaking

the capacitance for each pri tap setting/terminal configuration.

T-2 folk: ANY experience in this area will be welcome! Also, enlightened

curiosity, moral support---!  I'm not getting much from this end.

Granted, this may turn into an un-productive effort-over-kill, but what

the hell!  No one knows the answer to that, until someone tries it!

T-2 folk: is there an empirical setup to measure the terminal

capacitance of the TOROID assembly, as a component? Or is that a derived

value, based upon resonant frequency changes, as the terminal setups vary?

Or somesuch?

------------------------------------------------

Gene Waldo -  gwaldo@1stnetusa.com  - "Igor in Residence"

From: "Eriksson" <jan-erik.eriksson@mbox2.swipnet.se>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: varnish?

Date: Wed, 8 Jul 1998 18:54:43 +0200

X-Mailer: IPAD 2.0

X-MimeOLE: Produced By Microsoft MimeOLE V4.72.2106.4

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Can I use polyurethane varnish for my sec. coil? Or *must* it be polyethane?

Thanks,

//Christopher Eriksson

From: Roy Minnich <rminnich@rolling-meadows.lib.il.us>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: RE: varnish?

Date: Wed, 8 Jul 1998 12:25:54 -0500

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

Hi Christopher,

Yes polyurethane will be fine.  I'm not sure what polyethane is.  Might be a trade name used outside the US for the same product.  Or somebody just misspelled polyurethane.

Roy Minnich

----------

From: 	Eriksson[SMTP:jan-erik.eriksson@mbox2.swipnet.se]

Reply To: 	Tesla-2 Mailing List

Sent: 	Wednesday, July 08, 1998 11:54 AM

To: 	Tesla-2 Mailing List

Subject: 	varnish?

Can I use polyurethane varnish for my sec. coil? Or *must* it be polyethane?

Thanks,

//Christopher Eriksson

From: Dsurfr@aol.com

Date: Wed, 8 Jul 1998 18:54:23 EDT

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Formica spacers

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Dan, you & Gene keep at it. It may or may not prove to be that great a

contribution but for those of us still using SW Caps it's interesting. We need

more innovative thinking along about now & this is a start. Just remember Fr.

Tom's warning that the SW holds a charge. I must have missed the post before

the "Formica spacers" one, but I've followed your thoughts on this for a while

now & may try similar ones myself. Are you using (12) 40OZ beer bottles or

regular long necks?

I've got a 15kV 30mA neon that seems to run best w/ (6) 40OZ bottles. Please

post your construction techniques & resuts when you can.    Jim

Date: Wed, 08 Jul 1998 21:30:17 -0700

From: Gene Waldo <gwaldo@1stnetusa.com>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Organization: S.A.P.I.E.N.T. (Surly, Agressive, Politically Incorrect Enemies of Non Thinkers)

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Formica spacers

Sender: tesl-err@emachine.com

Dsurfr@aol.com wrote:

>

> Dan, you & Gene keep at it. It may or may not prove to be that great a

> contribution but for those of us still using SW Caps it's interesting. We need

> more innovative thinking along about now & this is a start. Just remember Fr.

> Tom's warning that the SW holds a charge. I must have missed the post before

> the "Formica spacers" one, but I've followed your thoughts on this for a while

> now & may try similar ones myself. Are you using (12) 40OZ beer bottles or

> regular long necks?

> I've got a 15kV 30mA neon that seems to run best w/ (6) 40OZ bottles. Please

> post your construction techniques & resuts when you can.    Jim

Jim,

One bottle can be seen on our site (below). Inner electrode is 1/8"

bronze weld rod tapped for nuts and fenders which fit inside and outside

the lids. Lid assy w/RTV goop.   The box Dan turned up handily holds 12

of the 40 oz'ers, so that's what we're running with. I presently have the

12 bussed together, and am concentrating on finishing the box

mechanically and electrically; the tub bottom electrode is for the

moment 3/8" galv hardware cloth. Then comes the fluid bath management

system, which pre any actual measurements, I estimate will hold about

2.5-3 gal of NaCl sol'n., and a bottom fitted ullage jug w/stand and

pulley system to control fluid level on the fly. And final calibration:

I think 0-8" depth v mfd.

Dan predates you in, ".... may not prove to be that great a

contribution...", but since I have received a modicum of atta boy's, and

no one will admit to this folly in the past, it seems I'm the suckka!

And, finally, I won't go to sleep at night, wondering....

Dan has almost washed his hands of the SW cap, accusing me of,

"...engineering the F out of it!"  But then, what does he know? Or I,

for that matter?  He was right about one aspect: this sort of stuff

seems to be how I get some of my jollys....

My one regret is that Samuel Adams or Sierra Nevada don't employ the 40

oz pkg.!

The really frightening part of this whole exercise is that I find I'm

now mentally designing a means of tuning the primary on the fly!!  But

I'm NOT going to go there!

You can do more amazing things in front of a cold TV, nowadays!

Should have more cap pics up in a week or two.

----------------------------------------------------

Gene Waldo -  gwaldo@1stnetusa.com  - "Igor in Residence"

X-Mailer: IPAD 2.0

Date: Thu, 09 Jul 1998 18:28:14 -0400

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Bob <tesla@america.com>

Subject: Happy Birthday

Cc: tesla@pupman.com,tesla-2@emachine.com

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Happy Birthday Nikola

Just a gesture for the day.......

Bob

X-Mailer: IPAD 2.0

Date: Thu, 09 Jul 1998 22:18:38 -0100

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Jim Ryan <jryan@exis.net>

Subject: Re: Happy Birthday

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

At 06:28 PM 7/9/98 -0400, you wrote:

>

>Happy Birthday Nikola

>

>

>

>

>Just a gesture for the day.......

>

>Bob

>

Ditto, Happy Birthday Nikola

Jim

From: Tom Stock <tomstock@geocities.com>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Toroids and spark improvement.

Date: Fri, 10 Jul 1998 12:18:54 -0700

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

I finally got around to making a new toroid since the last one was too big, and the previous one was too small. This time I wrapped two 8" piepans with 3.5" aluminum duct, and then covered the whole thing with foil tape. I'd run the coil, and look for places where the spark always wanted to break out, and smooth that spot down. I kept doing this until I got to a point where 1 to 3 sparks would continuously travel around the toroid, breathing and snapping like mad. I'm also to the point that I need to add my ground ring because the sparks are getting close to my primary. I haven't had a strike yet, but I expect it to be any time now, so the ground ring goes up asap.

Wow! Now things are getting fun. I'm up to 24" with two 30ma 15kv neons. I did have one spark approx 30", but only once.

My spark gap is firing once every 4 seconds or so. I think this is right?

I just can't wait to replace my 12 pack of long necks with some poly caps!

-tom

X-Mailer: IPAD 2.0

Date: Fri, 10 Jul 1998 12:46:03 -0500

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: Toroids and spark improvement.

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

At 12:18 PM 7/10/98 -0700, you wrote:

>

>I finally got around to making a new toroid since the last one was too big,

>and the previous one was too small. This time I wrapped two 8" piepans with

>3.5" aluminum duct, and then covered the whole thing with foil tape. I'd run

>the coil, and look for places where the spark always wanted to break out, and >smooth that spot down. I kept doing this until I got to a point where 1 to 3 >sparks would continuously travel around the toroid, breathing and snapping

>like mad. I'm also to the point that I need to add my ground ring because the >sparks are getting close to my primary. I haven't had a strike yet, but I

>expect it to be any time now, so the ground ring goes up asap.

Sounds like its working good Tom. The aluminum duct generally doesnt

need to be covered with foil tape. The ridges in the ducting are not

a problem. The only place you need the tape is at the joint where you

join the ends.

If you are getting multiple streamers forming at the same time

thats an indication that the radius of curvature of your toroid

could be a little greater. Try using some 4 inch duct and form

it into a toroid close to the same diameter as your smaller one.

This wont affect the tuning very much as toroid capacitance is

determined mainly by the diameter. The greater radius will allow

a higher voltage to build up before break out.

>

>Wow! Now things are getting fun. I'm up to 24" with two 30ma 15kv neons.

>I did have one spark approx 30", but only once.

You should be able to do even better. Try also adjusting your

coupling. Get the secondary as close to the primary as possible

without arcing. This also depends a lot on your gap. If your gap

is poor you wont be able to get the coupling as high before

performance suffers.

>

>My spark gap is firing once every 4 seconds or so. I think this is right?

Do you mean your main gap or the safety gap? If you mean safety

gap then yes thats about right. If you meant your main gap

is only firing every 4 seconds then you have a major problem.

Either way too much capacitance or way too wide a main gap or both.

>

>I just can't wait to replace my 12 pack of long necks with some poly caps!

You will be amazed at the difference. Initially your sparks wont

be very much longer. Maybe a couple inches. But they will be

thicker, brighter and much louder.

Keep up the good work.

TTYL!

Mike Hammer

mhammer@misslink.net

From: Roy Minnich <rminnich@rolling-meadows.lib.il.us>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: RE: Toroids and spark improvement.

Date: Fri, 10 Jul 1998 13:02:53 -0500

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

Hi Tom,

Main gap or safety gap???

Roy Minnich

----------

From: 	Tom Stock[SMTP:tomstock@geocities.com]

Reply To: 	Tesla-2 Mailing List

Sent: 	Friday, July 10, 1998 2:18 PM

To: 	Tesla-2 Mailing List

Subject: 	Toroids and spark improvement.

<SNIP>My spark gap is firing once every 4 seconds or so. I think this is right?

>I just can't wait to replace my 12 pack of long necks with some poly caps!

>-tom

From: "Choad" <choad69@geocities.com>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Formulas

Date: Sat, 11 Jul 1998 00:20:27 -0500

X-Mailer: IPAD 2.0

X-MimeOLE: Produced By Microsoft MimeOLE V4.72.3110.3

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Anyone happen to have the formula for finding the inductance

of a cone primary? I saw it once before but I don't remember where.

Also I can't remember how to find the length of a sprial. I also need

to find the length for the primary.

From: <Dsurfr@aol.com>

Date: Sat, 11 Jul 1998 15:22:25 EDT

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Formulas

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

I've found the wintesla program by Scott Coppersmith to be a great help if you

don't mind his program doing the math for you.

(http://www.aol.com/scopper/index.htm)

Hope this helps, Jim

PS: not much action on the tesla2 list lately. Come on guys this is a

big help site for us newbies

Date: Sat, 11 Jul 1998 15:22:49 -0500

From: Bert Hickman <bert.hickman@aquila.com>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Organization: Stoneridge Engineering

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Formulas

Sender: tesl-err@emachine.com

Choad wrote:

>

>     Anyone happen to have the formula for finding the inductance

> of a cone primary? I saw it once before but I don't remember where.

> Also I can't remember how to find the length of a sprial. I also need

> to find the length for the primary.

Choad,

Here's some formulas for Archimedes, helical, and inverse conical

primaries. The helical and Archimedes forms are from Wheeler, and the

inverse conical is a hybrid form that appropriately weights the vertical

and horizontal components of Helical and Archimedes inductances.

All dimensions are in inches, and L is in microHenries. While the

Inverse Conical calculation is a little "hairier" than the first two,

it's relatively easy to calculate for any desired angle, especially if

set up in a spreadsheet. Many coliers ose a 30 degree cone, but there's

really nothing magical about this angle.

------------------------------------------------------------------------

Case 1: Archimedes Spiral:

Let R = Ave Radius

N = Number of Turns

w = Width of Winding

|   R    |      N Turns

o o o o o o   |   o o o o o o

|    W    |   |

L = (R^2)*(N^2)/(8*R+11*W)   (R^2 = R*R)

------------------------------------------------------------------------

Case 2: Helical Primary:

|<- R ->|

--  o       |       o

|   o       |       o

o       |       o

L   o       |       o  N Turns

o       |       o

|   o       |       o

--  o       |       o

L = R^2*N^2/(9*R+10*L)  (Vertical Helix)

------------------------------------------------------------------------

Case 3: Inverse Conical Primary:

/ \

--  o                          /    o

|   o                        /    o

|    o   N turns            /    o

o                 Z  /    o

h      o                  /    o   /

o                /    o   /

|        o              /    o   /  Angle = X

|         o              \  o   /

--          o               o    ------------

|

|   w   |   R   |

|

|<--  W  -->|

^

Center | Line

Z = Coil Width (hypotenuse length)

X = Angle of Cone

h = Z*sin(X)  Effective vertical Height

w = Z*cos(X)  Effective horizontal Width

W = R + w/2   Average horizontal Radius

L1 = W^2*N^2/(9*W+10*h)  (Vertical Inductance Component)

L2 = W^2*N^2/(8*W+11*w)  (Horizontal Inductance Component)

L = SQRT[(L1*Sin(X))^2 + (L2*cos(X))^2]

------------------------------------------------------------------------

Hope this helps, and safe coilin' to you!

-- Bert --

Date: Sat, 11 Jul 1998 16:36:23 -0500

From: Tom Polk <tpolk@prismnet.com>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Formulas

Sender: tesl-err@emachine.com

Brute force spiral length equation (assumes uniform spacing, I believe):

Length = PI * (d1 + 2*d2 ... + 2*d(n-1)+ dn)/2

Where n= number of turns you desire + 1

and dn= diameter of each turn n

Example: For three complete turns, you have four diameters,

where d1=6", d2=6.5", d3=7", d4=7.5"

PI * (6 + 2*6.5 + 2*7 +7.5)/2 = 63.62" Length of spiral

Wanna save time?

If you have a Sigma equation on you calculator (The Solver in HP95LX is

fantastic for this type of thing) use this:

Length = PI * SIGMA (counter, start, end, turns-1, step,

(counter+counter+step)/2)   where end= turns-1

From: <Dsurfr@aol.com>

Date: Sat, 11 Jul 1998 19:59:38 EDT

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: WinTesla address

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

My apologies to all

The correct address is:http://members.aol.com/rscopper/index.htm

Jim

Comments: Authenticated sender is <megavolt@mail.pcisys.net>

From: megavolt@mail.pcisys.net

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Date: Sun, 12 Jul 1998 08:51:35 +0000

Subject: Arrestors

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Hello All,

I am wondering if anyone has ever tried  using  these arrestors

in a Tesla coil circuit before?  This information is on a plate at

the top terminal:

JOSLYN

HEAVY DUTY

DIST. ARRESTOR

ZQP 81540010

8.40 kV MCOV

10 kV RATING

I found them at my utilities public salvage area.  They looked cool

and were at the right price...free.  I have attached  a 28kb .jpg

format drawing that I did in CorelDraw with this post to show exactly

what they look like.  After doing extensive modifications to my six

inch coil I decided to put the arrestors on.  They are after the

safety gap and before the resistors ,chokes and main gap.  Usually

when I make major changes and start re-tuning I get a lot of kick back

through the safety gap.  This time with the arrestors I didn't have

any kick back while tuning...any thoughts?

Kent

megavolt@pcisys.net

Comments: Authenticated sender is <megavolt@mail.pcisys.net>

From: megavolt@mail.pcisys.net

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Date: Sun, 12 Jul 1998 15:21:14 +0000

Subject: Arrestor jpg

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Bert,

Thank you for your very informative reply.  Sorry,  I guess that I

was in a hurry and forgot the attachment.  Some specs for my coil:

Secondary:

6" PVC wound with 20 ga. to 28",  7 undercoats of

polyurethane, 37 overcoats, capped on both ends with clear

plexiglass.

Primary:

20 turns of 5/16 soft copper,  20 degree angle, 5/16 spacing.

Capacitor:

4 .018 uf rolled .060 high density poly/aluminum roof flashing.

Series/parallel equadrive configuration for .018uf at 20,000 volts?

Main gap:

2 RQ 6 gap statics horizontally mounted with three high volume fans.

Fan-gap-fan-gap-fan.

Transformer:

Now I'm running 2-15/30's and 1-15/60 nst's.  Will probably end up

running 4-15/60's unpotted in mineral oil.

Toroid:

6"x20" corrugated black drain pipe,  Just made a 8"x28" dryer duct

that has not been tested.

Misc:

Safety gap

1200 ohm 100 watt resistors

2mh chokes

10kv arrestors

PFC caps

50 amp Powerstat  metered control cabinet

Date: Sun, 12 Jul 1998 10:57:09 -0500

From: Bert Hickman <bert.hickman@aquila.com>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Organization: Stoneridge Engineering

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Arrestors

Sender: tesl-err@emachine.com

megavolt@mail.pcisys.net wrote:

>

> Hello All,

>

> I am wondering if anyone has ever tried  using  these arrestors

> in  a Tesla coil circuit before?  This information is on a plate at

> the top terminal:

>

> JOSLYN

> HEAVY DUTY

> DIST. ARRESTOR

>

> ZQP 81540010

> 8.40 kV MCOV

> 10 kV RATING

>

> I found them at my utilities public salvage area.  They looked cool

> and were at the right price...free.  I have attached  a 28kb .jpg

> format drawing that I did in CorelDraw with this post to show exactly

> what they look like.  After doing extensive modifications to my six

> inch coil I decided to put the arrestors on.  They are after the

> safety gap and before the resistors ,chokes and main gap.  Usually

> when I make major changes and start re-tuning I get a lot of kick back

> through the safety gap.  This time with the arrestors I didn't have

> any kick back while tuning...any thoughts?

>

> Kent

>

> megavolt@pcisys.net

Kent,

Nice catch on the arrestors... and the price was certainly right! The

.jpg file apparently didn't accompany your message on the list.

Distribution arrestors come in a couple of flavors - Silicon-Carbide-Gap

(SiC) and Metal Oxide Varistor (MOV). The latter type are rated at by

Maximum Continuous Operating Voltage (MCOV), which in the case of your

arrestor is 8.4 KV. The Voltage Rating of 10 KV means that these

arrestors are designed to operate with no more than 10 KV at power line

frequency for up to 30 minutes without overheating. Under higher

overvoltage conditions, the MOV elements inside the arrestor go from a

being a relatively poor conductor to a very good conductor (sort of like

a high voltage Zener Diode in series with a resistor).

While I wasn't able to find the exact specs for your arrestors, a

similar McGraw Edison arrestor is spec'ed to clamp an incoming surge to

no more than 24KV at 500 A. These devices exhibit "soft" breakdown,

unlike the older SiC type with an internal series sparkgap, providing

relatively continuous protection as the overvoltage condition develops.

You didn't give any details about your Tesla Coil's primary circuit

operating voltage. However, if used on each "leg" of the HV supply to

ground, they should work quite will even with 15 KV NST's.

Safe coilin' to you!

-- Bert --

From: <Dsurfr@aol.com>

Date: Mon, 13 Jul 1998 21:13:04 EDT

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Toroid Tips??

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

I was given some advice on making a toroid from aluminum dryer duct & just

wanted to get some feed back from the list:

Use a duct that has about the same cross section as your secondary coil

The outside diameter of the toroid should be about the same as the diameter of

your primary coil.

The bottom edge of the toroid should be about the same level as the top

winding of your secondary

Fully extend the duct to increase the "skin effect" before forming it into

shape, don't just wrap it in its compressed form

Use a nylon bolt to hold the two pie pans together that you are using for a

form

Attach the pans to the duct after it is in place with aluminum foil tape

Use a loop of wire formed from the wire coming from the end of the coil that

is held in place between the nylon bolt & the pie pan

A good toroid is my next big step in increasing my spark output & I want to do

it correctly. Thanks for any advice.  Jim

Date: Mon, 13 Jul 1998 20:28:20 -0500

From: Bert Hickman <bert.hickman@aquila.com>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Organization: Stoneridge Engineering

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Toroid Tips??

Sender: tesl-err@emachine.com

Dsurfr@aol.com wrote:

>

> I was given some advice on making a toroid from aluminum dryer duct & just

> wanted to get some feed back from the list:

> Use a duct that has about the same cross section as your secondary coil

> The outside diameter of the toroid should be about the same as the diameter of

> your primary coil

> The bottom edge of the toroid should be about the same level as the top

> winding of your secondary

> Fully extend the duct to increase the "skin effect" before forming it into

> shape, don't just wrap it in its compressed form

> Use a nylon bolt to hold the two pie pans together that you are using for a

> form

> Attach the pans to the duct after it is in place with aluminum foil tape

> Use a loop of wire formed from the wire coming from the end of the coil that

> is held in place between the nylon bolt & the pie pan

> A good toroid is my next big step in increasing my spark output & I want to do

> it correctly. Thanks for any advice.  Jim

Jim,

For the most part, everything sounds reasonable, except that you don't

need to use nylon bolts to bolt the piepans together - plain ordinary

conductive ones will work just fine. Also, the toroid's Radius of

Curvature is more a function of the output voltage you'll be generating,

and is thus more a function of the tank cap size and primary gap

breakdown voltage than coilform diameter. It's quite possible that a

heavily-driven 4" coil may actually require a toroid with an 8" ROC. You

may end up buulding 2 or 3 toroids as you start increasing power levels.

You're in for some much improved performance once you get an optimally

sized toroid in place...

Good luck, Jim!

-- Bert --

X-Mailer: IPAD 2.0

Date: Tue, 14 Jul 1998 08:40:20 +0300

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Marco Denicolai <marco@vistacom.fi>

Subject: Second change for longer sparks

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

After using a bigger toroid and getting improvements:

air streamers from 20" to 31"

streamers to grounded point from 20" to 24"

I have reduced my RQ spark gap gaps from 0.0275" to about 0.007". This was

mechanically not so easy and I ended up with a clearance between 0.005" and

0.01" (about) different between different sections. Not good...

Before I was using two series of 0.0275" gaps in parallel. Now I could use

a single series of 10 gaps and have the safety gap strike intermittently. I

tested with this setup and I didn't notice any improvement (or worsening) of

the streamers. Spark gap sound was intermittent as with the old gaps.

Then I started reducing the number of gaps until I was using 5 gaps. The spark

gap sound was then almost continuous and I got longer streamers! About 35" to a

grounded point and over 40" into air.

My problems:

- the gap gets extremely hot very quickly

- some of the gap sections get filled with some black stuff (cupper oxide?)

- I don't quite understand what is going on!

Please, help me to figure out what I am doing...

- I suppose I have just increased the spark gap breakout repetition rate as well

as I have fed the primary with less voltage. Higher discharge rate seems to be

better for longer sparks than higher voltage. Am I right?

- Higher repetition rate stresses more the spark gap that gets really hot. Is

that the way it should work or what should I do?

- My ideas (also after reading the list mails) about the relation between:

- spark length

- primary voltage at gap breakout

- breakout repetition rate (60 Hz, 120 Hz, more?)

- spark gap quenching

are VERY confused. Could please someone give a brief (or long if he can)

explanation about the above factor interaction?

P.S. We also noticed a water leak. That means all the entrance parket floor

stripped away and 4 weeks of blowing warm air against the concrete floor to

dry it. Well, this is not Tesla, but it DOES affect my TC development, as you

may guess :)

Marco Denicolai

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Date: Tue, 14 Jul 1998 04:08:43 -0700

From: "Leben Lang und Frei" <fleischwulf@mailexcite.com>

Cc:

X-Sent-Mail: off

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Expiredinmiddle: true

X-Mailer: IPAD 2.0

Subject: Re: Second change for longer sparks

X-Sender-Ip: 208.153.7.9

Organization: MailExcite  (http://www.mailexcite.com:80)

Sender: tesl-err@emachine.com

Marco,

The MOT's produce way too much current. Yes the black stuff is Cupric oxide.

The gaps have to be close for the MOT to work. The copper oxide will build up

to a point were the gaps will close up completely. If you can try to get to

12 kv. If your gaps are too close you will never quench them. If you vent air

thru the gaps by using radial holes, this will help. You will need a very powerful shop vac. As long as you are running high current your gaps will be very hot and prone to oxidation.

Paul M.

---

Einigkeit und Freiheit

>After using a bigger toroid and getting improvements:

>

>air streamers from 20" to 31"

>streamers to grounded point from 20" to 24"

>

>I have reduced my RQ spark gap gaps from 0.0275" to about 0.007". This was

>mechanically not so easy and I ended up with a clearance between 0.005" and

>0.01" (about) different between different sections. Not good...

>

>Before I was using two series of 0.0275" gaps in parallel. Now I could use

>a single series of 10 gaps and have the safety gap strike intermittently. I

>tested with this setup and I didn't notice any improvement (or worsening) of

>the streamers. Spark gap sound was intermittent as with the old gaps.

>

>Then I started reducing the number of gaps until I was using 5 gaps. The spark

>gap sound was then almost continuous and I got longer streamers! About 35" to a

>grounded point and over 40" into air.

>

>My problems:

>

>- the gap gets extremely hot very quickly

>- some of the gap sections get filled with some black stuff (cupper oxide?)

>- I don't quite understand what is going on!

>

>Please, help me to figure out what I am doing...

>

>- I suppose I have just increased the spark gap breakout repetition rate as well

>as I have fed the primary with less voltage. Higher discharge rate seems to be

>better for longer sparks than higher voltage. Am I right?

>

>- Higher repetition rate stresses more the spark gap that gets really hot. Is

>that the way it should work or what should I do?

>

>- My ideas (also after reading the list mails) about the relation between:

>

>   - spark lenght

>   - primary voltage at gap breakout

>   - breakout repetition rate (60 Hz, 120 Hz, more?)

>   - spark gap quenching

>

>are VERY confused. Could please someone give a brief (or long if he can)

>explanation about the above factor interaction?

>

>P.S. We also noticed a water leak. That means all the entrance parket floor

>stripped away and 4 weeks of blowing warm air against the concrete floor to

>dry it. Well, this is not Tesla, but it DOES affect my TC development, as you

>may guess :)

>

>

>

>

> Marco Denicolai

>

Date: Tue, 14 Jul 1998 06:59:48 -0500

From: Bert Hickman <bert.hickman@aquila.com>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Organization: Stoneridge Engineering

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Second change for longer sparks

Sender: tesl-err@emachine.com

Marco Denicolai wrote:

>

> After using a bigger toroid and getting improvements:

>

> air streamers from 20" to 31"

> streamers to grounded point from 20" to 24"

>

> I have reduced my RQ spark gap gaps from 0.0275" to about 0.007". This was

> mechanically not so easy and I ended up with a clearance between 0.005" and

> 0.01" (about) different between different sections. Not good...

>

> Before I was using two series of 0.0275" gaps in parallel. Now I could use

> a single series of 10 gaps and have the safety gap strike intermittently. I

> tested with this setup and I didn't notice any improvement (or worsening) of

> the streamers. Spark gap sound was intermittent as with the old gaps.

Your total gap breakdown voltage is apparently near the maximum primary

supply voltage. Spark gaps don't consistently break down at the same

voltage every time, and the breakdown mechanism itself has a degree of

randomness to it. Also copper oxides will form between the electrodes

with use (the black material), and this material tends to inhibit spark

formation.

> Then I started reducing the number of gaps until I was using 5 gaps. The spark

> gap sound was then almost continuous and I got longer streamers! About 35" to a

> grounded point and over 40" into air.

Streamer length is a function of the amount of energy you have per gap

firing (the "Bang" size) and the repitition rate. While decreasing the

gap spacing reduces the bang size (since the initial energy is limited

to (1/2)*Cp*Vgap^2), increasing the rep rate can actually result in

putting more total power into the streamers. The added power keeps the

air in the arc channels hotter, making the "roots" of the streamers more

conductive, and allowing the streamers to propagate further.

>

> My problems:

>

> - the gap gets extremely hot very quickly

> - some of the gap sections get filled with some black stuff (cupper oxide?)

> - I don't quite understand what is going on!

>

With fewer gaps and smaller total gap spacing, your gaps are firing more

often. With a low impedance HV source, it's possible under some

conditions to get 3-4 gap firings or more per AC mains half-cycle.

Suppose your system was originally firing once per mains half-cycle, and

you now reduced the gap length a bit so that they fired twice per half

cycle. You'll now be processing almost twice the power as before, and

will see almost twice the heat dissipation in your gap. Fewer gaps also

mean that the increased gap dissipation is being shared by fewer

electrodes, so the power dissipation per electrode rises.  It's critical

to remove this heat from the gap electrodes (typically by forced-air

cooling) so that the gaps don't overheat and fail to quench properly.

With copper electrodes, a mixture of cuprous and cupric oxides are

formed on the electrodes as a normal part of their operation. This layer

will tend to reduce the effective gap spacing, and can actually cause

gap fouling with close spacings.

However, the root cause of the problems you're seeing are related to

voltage sag in the voltage doubler circuit under heavy loading. In the

steady-state, true voltage doubling will only occur under relatively

light loading, since the doubling effect relies on the stored charge on

one of the caps being "carried over" from the previous half-cycle to so

that it can be added to the voltage of the present mains half-cycle.

Under heavy loading, the carried-over charge will be less, and the

doubler's output voltage sags, and getting consistent gap firing becomes

progressively more difficult at lower voltages. If you're limited to

using the doubler supply configuration, about the best you can do is

cool the gaps sufficiently, and clean them more frequenctly.

> Please, help me to figure out what I am doing...

>

> - I suppose I have just increased the spark gap breakout repetition rate as well

> as I have fed the primary with less voltage. Higher discharge rate seems to be

> better for longer sparks than higher voltage. Am I right?

Yes, up to a point. There's actually a bit of a balacing act between rep

rate, gap spacing, and toroid radius of curvature required to get best

performance...

.

>

> - Higher repetition rate stresses more the spark gap that gets really hot. Is

> that the way it should work or what should I do?

It's working properly. Add more force-air cooling to remove the

additional heat.

>

> - My ideas (also after reading the list mails) about the relation between:

>

>    - spark lenght

>    - primary voltage at gap breakout

>    - breakout repetition rate (60 Hz, 120 Hz, more?)

>    - spark gap quenching

>

> are VERY confused. Could please someone give a brief (or long if he can)

> explanation about the above factor interaction?

>

Hopefully the above explanations will help.

> P.S. We also noticed a water leak. That means all the entrance parket floor

> stripped away and 4 weeks of blowing warm air against the concrete floor to

> dry it. Well, this is not Tesla, but it DOES affect my TC development, as you

> may guess :)

>

Bummer! Good luck!

> <SNIP>

Safe coilin' to you!

-- Bert --

X-Originating-IP: [199.217.140.22]

From: "Bill the arcstarter" <arcstarter@hotmail.com>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Newest secondary

Date: Tue, 14 Jul 1998 08:13:46 PDT

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

Hi Coilers,

I just finished winding/sealing and installing the endplug + toroid lug

into our latest secondary.

This is a 6.25 inch diameter PVC tube, about 28 inches long, wound with

#22 enamel wire.

Resistance is 25 ohms.  Lsec = 34. mH.  Don't know what the raw

resonant frequency is yet.

Wish us luck!

-Bill the arcstarter

Starting arcs in Cinci, OH

http://www.geocities.com/CapeCanaveral/Hangar/6160

Date: Tue, 14 Jul 1998 23:26:55 +0100

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Stephen Bell <steve@breakfix.demon.co.uk>

Subject: Secondary ratios theory

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Hi Guys,

As a beginner I've been doing a lot of reading of archive list

postings and web pages.  There are many people with many different ideas

and ways of doing things.  I find myself looking for the underlying

theory on which all observed data is reliant.

With this in mind, could someone tell me what determines the

physical dimensions of the secondary coil?  I've heard it said "Tall 'n'

thin is not as good as short 'n' fat" (when referring to coils, not

coilers ;-) ).

Can any comparisons be made to a 1/4 wave monopole (a 250m tall

antenna being optimum for 300kHz operation) with inductance added at its

base to reduce its height?

TIA.

(No coil running yet but I've a cool 10kV/100mA Jacobs Ladder!)

-------------------------------------------

Stephen Bell   (steve@breakfix.demon.co.uk)

-------------------------------------------

X-Mailer: IPAD 2.0

Date: Tue, 14 Jul 1998 17:28:50 -0500

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: Newest secondary

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

At 08:13 AM 7/14/98 PDT, you wrote:

>Hi Coilers,

> I just finished winding/sealing and installing the endplug + toroid lug

>into our latest secondary.

>

> This is a 6.25 inch diameter PVC tube, about 28 inches long, wound with

>#22 enamel wire.

>

> Resistance is 25 ohms.  Lsec = 34. mH.  Don't know what the raw

>resonant frequency is yet.

I make it right around 262 KHZ unloaded.

Should be great.

>

> Wish us luck!

Luck!

TTYL!

Mike Hammer

mhammer@misslink.net

X-Mailer: IPAD 2.0

Date: Tue, 14 Jul 1998 17:54:36 -0500

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: Second change for longer sparks

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

At 08:40 AM 7/14/98 +0300, you wrote:

>After using a bigger toroid and getting improvements:

>

>air streamers from 20" to 31"

>streamers to grounded point from 20" to 24"

>

>I have reduced my RQ spark gap gaps from 0.0275" to about 0.007". This was

>mechanically not so easy and I ended up with a clearance between 0.005" and

>0.01" (about) different between different sections. Not good...

>

>Before I was using two series of 0.0275" gaps in parallel. Now I could use

>a single series of 10 gaps and have the safety gap strike intermittently. I

>tested with this setup and I didn't notice any improvement (or worsening) of

>the streamers. Spark gap sound was intermittent as with the old gaps.

>

>Then I started reducing the number of gaps until I was using 5 gaps. The spark

>gap sound was then almost continuous and I got longer streamers! About 35" to a

>grounded point and over 40" into air.

>

>My problems:

>

>- the gap gets extremely hot very quickly

>- some of the gap sections get filled with some black stuff (cupper oxide?)

>- I don't quite understand what is going on!

>

The heat and rapid oxidation are due to the very high current

from your transformers.

>Please, help me to figure out what I am doing...

>

>- I suppose I have just increased the spark gap breakout repetition rate as well

>as I have fed the primary with less voltage. Higher discharge rate seems to be

>better for longer sparks than higher voltage. Am I right?

Yes. The total delivered power is what drives streamer formation.

By having 2 or more gap firings per 1/2 cycle you are processing

more power even though the primary voltage may be lower.

>

>- Higher repetition rate stresses more the spark gap that gets really hot. Is

>that the way it should work or what should I do?

Yes. Since you are processing more power there is more heat.

The way to lower gap heating is to:

1. Use larger electrodes. Larger electrodes have more thermal mass

so will sink more heat.

2. Use more series gaps. By increasing the number of gaps and

decreasing the individual spacing you will divide the heat

up over a greater number of areas while maintaining the same total

gap distance.

3. Use more forced air. If you arent using forced air do so.

If you are use a stronger source.

Marco I believe you could get the same (or nearly so) results

by doing away with the voltage doubler and using the AC output

of the transformers as is. This will be around 4500 volts for

a pair of typical MOTs in series. Some increase in voltage will

occur with the charging of the primary cap. The difficult part

will be a spark gap to handle the low voltage and high current.

By eliminating the doubler you will greatly simplify your design.

I plan to build just such a coil and have several ideas about how

to accomplish it. Very briefly my plan calls for a low resonant

frequency that will allow for a high primary inductance and

a fairly high primary capacitance. The spark gap will consist

of a large number of very closely spaced gaps. This gap idea has

recently been shown to have some merit by Terry Fritz.

Terry has built several gaps with very small spacing from 4 to 7 mils

and up to 60 (!) electrodes. He reports excellent quenching

with NO FORCED AIR.

Terry's web page is at: http://www.peakpeak.com/~terryf/tesla/main.html

and contains some excellent articles on experiments he has performed.

A picture of his latest gap can be found at:

http://www.peakpeak.com/~terryf/tesla/gap.jpg

TTYL!

Mike Hammer

mhammer@misslink.net

From: ralph.hartwell@emachine.com (Ralph Hartwell)

Subject:  Second change for longer sparks

Date: Wed, 15 Jul 1998 05:06:00 GMT

Organization: The Energy Machine Information System 504-733-3462

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

MD>P.S. We also noticed a water leak. That means all the entrance parket floor

MD>stripped away and 4 weeks of blowing warm air against the concrete floor to

MD>dry it. Well, this is not Tesla, but it DOES affect my TC development, as yo

MD>may guess :)

Yes, that extra humidity does reduce the spark length.  ;)

Ralph

---

X-Mailer: IPAD 2.0

Date: Wed, 15 Jul 1998 08:06:18 +0300

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Marco Denicolai <marco@vistacom.fi>

Subject: Re: Second change for longer sparks

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

At 17:54 14/07/98 -0500, you wrote:

>Marco I beleive you could get the same (or nearly so) results

>by doing away with the voltage doubler and using the AC output

>of the transformers as is. This will be around 4500 volts for

>a pair of typical MOTs in series. Some increase in voltage will

>occur with the charging of the primary cap. The difficult part

>will be a spark gap to handle the low voltage and high current.

>By eliminating the doubler you will greatly simplify your design.

Well, 2 capacitors and diodes you find readily in any microwave oven.

Nothing else "extra" is required for the doubler.

>

>I plan to build just such a coil and have several ideas about how

>to accomplish it. Very briefly my plan calls for a low resonant

>frequency that will allow for a high primary inductance and

>a fairly high primary capacitance. The spark gap will consist

>of a large number of very closely spaced gaps. This gap idea has

>recently been shown to have some merit by Terry Fritz.

>

>Terry has built several gap with very small spacing from 4 to 7 mils

>and up to 60 (!) electrodes. He reports excellent quenching

>with NO FORCED AIR.

Yes, I saw that. In my gap, because 0.007" is too close and 0.0275" is

too wide, I am now building a spark gap with 0.02" clearance, longer

sections (4" instead of 2") and plan to connect two series of about 5 gaps

in parallel. There is (as before) a blower, not a vacuum fan.

If I had more time I would go for a synchronous rotary gap with

"break-before-make"

as originally I thought long time ago, but I have to get the TC to work

for a demonstration to some guys.

The RSG I have in mind has large circular sections to connect the tank to the

capacitor and sensibly shorter circular sections to discharge the capacitor

into the primary. In this way we can benefit from the MOT large current to

charge quickly the capacitor and when we discharge it the MOT are not connected across the spark gap.

Marco Denicolai

X-Originating-IP: [199.217.140.22]

From: "Bill the arcstarter" <arcstarter@hotmail.com>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Newest secondary

Date: Wed, 15 Jul 1998 05:41:50 PDT

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

I wrote:

>> This is a 6.25 inch diameter PVC tube, about 28 inches long, wound with

>>#22 enamel wire.

>>

>> Resistance is 25 ohms.  Lsec = 34. mH.  Don't know what the raw

>>resonant frequency is yet.

Then Mike Hammer wrote:

>I make it right around 262 KHZ unloaded.

>Should be great.

Last night I did a real-world measurement of the (unloaded) resonant

frequency - 247 KHz.  That's fairly close to the predicted value.

-Bill

X-Mailer: IPAD 2.0

Date: Wed, 15 Jul 1998 12:00:31 -0500

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: Second change for longer sparks

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

At 08:06 AM 7/15/98 +0300, you wrote:

<SNIP>

>>Terry has built several gap with very small spacing from 4 to 7 mils

>>and up to 60 (!) electrodes. He reports excellent quenching

>>with NO FORCED AIR.

>

>Yes, I saw that. In my gap, because 0.007" is too close and 0.0275" is

>too wide, I am now building a spark gap with 0.02" clearance, longer

>sections (4" instead of 2") and plan to connect two series of about 5 gaps

>in parallel. There is (as before) a blower, not a vacuum fan.

The larger electrodes will help by sinking more heat.

>

>If I had more time I would go for a synchronous rotary gap with

>"break-before-make"

>as originally I thought long time ago, but I have to get the TC to work

>for a demonstration to some guys.

>The RSG I have in mind has large circular sections to connect the tank to the

>capacitor and sensibly shorter circular sections to discharge the capacitor

>into the primary. In this way we can benefit from the MOT large current to

>charge quickly the capacitor and when we discharge it the MOT are not >connected across the spark gap.

>

>

Its quite possible as Bert Hickman pointed out in his reply to you

that you are having some voltage sag in your doubler circuit. Voltage

multipliers only multiply well under light loads. If you are

having voltage sag then a rotary gap may make it worse.

Your actual primary voltage may be getting down to as low

as 5kv or so. This is what you would get from the two MOTs

alone. This would be easy enough to check. Simply remove

the doubler circuit and run the coil with the two MOTs alone.

You may have to make some gap changes but I think you may find

you will get equivalent performance. Perhaps not. Still I would

be interested in your results if you try this test as I plan

to build a coil around a pair of MOTs as an experiment.

TTYL!

Mike Hammer

mhammer@misslink.net

Date: Wed, 15 Jul 1998 21:33:19 +0100

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Stephen Bell <steve@breakfix.demon.co.uk>

Subject: Secondary Coil theory

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Hi Guys,

As a beginner I've been doing a lot of reading of archive list

postings and web pages.  There are many people with many different ideas

and ways of doing things.  I find myself looking for the underlying

theory on which all observed data is reliant.

With this in mind, could someone tell me what determines the

physical dimensions of the secondary coil?  I've heard it said "Tall 'n'

thin is not as good as short 'n' fat" (when refering to coils, not

coilers ;-) ).

Can any comparisons be made to a 1/4 wave monopole (a 250m tall

antenna being optimum for 300kHz operation) with inductance added at its

base to reduce its height?

TIA.

(No coil running yet but I've a cool 10kV/100mA Jacobs Ladder!)

-------------------------------------------

Stephen Bell   (steve@breakfix.demon.co.uk)

-------------------------------------------

X-Mailer: IPAD 2.0

Date: Wed, 15 Jul 1998 21:05:25 -0500

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: Secondary Coil theory

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

At 09:33 PM 7/15/98 +0100, you wrote:

>Hi Guys,

>        As a beginner I've been doing a lot of reading of archive list

>postings and web pages.  There are many people with many different ideas

>and ways of doing things.  I find myself looking for the underlying

>theory on which all observed data is reliant.

>        With this in mind, could someone tell me what determines the

>physical dimensions of the secondary coil?  I've heard it said "Tall 'n'

>thin is not as good as short 'n' fat" (when refering to coils, not

>coilers ;-) ).

Stephen,

There are several conflicating factors to consider for a secondary coil.

One is the need for high inductance. But high inductance alone doesnt

make for a good secondary. If it did we would all be winding our

coils with AWG 34. RF resistance needs to be low. So this suggests

larger guage wire. Q should be high. This also suggests heavier

wire and a short aspect ratio. Shorter secondary coils have a lower

voltage handling capability. This suggests the need for a tall coil.

All these factors balance out with secondaries with H/D ratios

of between 3 to 1 and 6 to 1 with from 700-1000 turns.

This size of secondary provides reasonably high inductance

with good Q and power handling capability.

>        Can any comparisons be made to a 1/4 wave monopole (a 250m tall

>antenna being optimum for 300kHz operation) with inductance added at its

>base to reduce its height?

Tesla secondaries do not follow conventional 1/4 wave antenna theory.

The 1/4 wave length of the wire is immaterial.

TTYL!

Mike Hammer

mhammer@misslink.net

X-Mailer: IPAD 2.0

Date: Thu, 16 Jul 1998 08:06:04 +0300

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Marco Denicolai <marco@vistacom.fi>

Subject: Re: Second change for longer sparks

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

At 12:00 15/07/98 -0500, you wrote:

>Your actual primary voltage may be getting down to as low

>as 5kv or so. This is what you would get from the two MOTs

>alone. This would be easy enough to check. Simply remove

>the doubler circuit and run the coil with the two MOTs alone.

>You may have to make some gap changes but I think you may find

>you will get equivalent performance. Perhaps not. Still I would

>be interested in your results if you try this test as I plan

>to build a coil around a pair of MOTs as an experiment.

>

>TTYL!

>

>Mike Hammer

>mhammer@misslink.net

>

>

I'll try that in a few weeks: first I have to get my new spark gap

operational and make a demo to some guys that are waiting for it.

I don't like to break anything before that (you never know).

Marco Denicolai

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: yubba <yubba@clara.net>

Subject: RF chokes

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

Date: Mon, 20 Jul 98 02:00:18 GMT

Sender: tesl-err@emachine.com

hi all

I am going to try to make some RF chokes using 2 1/2" ferrite rings from

an old TV scanning coil. It then occurred to me that I might need slots cut in

them to reduce eddy currents. Is this needed at RF frequencies? And "the big question", if not why not? No 4th order differential equations please, I am

still trying to cope with the last simple question concerning raising the secondary!!. keep up the good work guys.

many thanks.

bob golding lighning without rain

bob yubba@clara.net

X-Mailer: IPAD 2.0

Date: Mon, 20 Jul 1998 12:27:33 -0500

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: RF chokes

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

At 02:00 AM 7/20/98 GMT, you wrote:

>hi all

>  I am going to try to make some RF chokes using 2 1/2" ferrite rings

>from an old TV scanning coil. It then occurred to me that I might need

>slots cut in them to reduce eddy currents. Is this needed at RF frequencies?

>And "the big question", if not why not? No 4th order differential equations

>please, I am still trying to cope with the last simple question concerning

>raising the secondary!!.

>keep up the good work guys. many thanks.

>

>bob golding lighning without rain

>

>bob yubba@clara.net

>

Bob,

No slots needed. Cutting a slot in a core to introduce an air

gap is done to prevent core saturation. Core saturation will

not be a problem in your RF chokes. Use them as is.

As ferrites are slightly conductive dont forget to insulate the

cores with 3 layers of good quality vinyl tape to prevent arcing.

TTYL!

Mike Hammer

mhammer@misslink.net

Date: Mon, 20 Jul 1998 23:28:25 +0100

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Stephen Bell <steve@breakfix.demon.co.uk>

Subject: Discharge terminal wiring

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Hi Guys,

From what I've read so far, the top winding of the secondary

coil is usually left flying, with enough length to reach a bolt fixing

on the output terminal (toroid etc).  Is it OK to have some sort of plug

connector fixed to the top of the secondary, to eliminate the problem of

having a flying lead break off at the coil?  Would the shielding effect

from a toroid make the design of the connector less critical?

TIA.

(Just got hold of a 25 meter roll of 25kV EHT cable, and a possible

source of free HV oil!  It's all comming together! :-)

-------------------------------------------

Stephen Bell   (steve@breakfix.demon.co.uk)

-------------------------------------------

X-Mailer: IPAD 2.0

Date: Mon, 20 Jul 1998 19:15:20 -0500

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: Discharge terminal wiring

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

At 11:28 PM 7/20/98 +0100, you wrote:

>Hi Guys,

>        From what I've read so far, the top winding of the secondary

>coil is usually left flying, with enough length to reach a bolt fixing

>on the output terminal (toroid etc).  Is it OK to have some sort of plug

>connector fixed to the top of the secondary, to eliminate the problem of

>having a flying lead break off at the coil?  Would the shielding effect

>from a toroid make the design of the connector less critical?

>

>TIA.

Stephen,

Many people do leave the top of the secondary winding free and just

tuck it up under the toroid. I prefer not to do this. I just never

like the idea of the loose wire that could be broken off.

I mount a central bolt in the top of my secondary form. This is

mounted in a disk of wood epoxied into the top of the form.

Several thicknesses of plexiglass (1/4 inch total) are mounted

beneath the wood disk/central bolt assembly to prevent arcing down

the inside of the form.

The free end of the secondary wire is routed up the OUTSIDE

of the form and over to the central bolt and attached with a

ring terminal. A nut is tightened down on the ring terminal

and the whole top (secondary lead, ring terminal, top of wood

disk) is embedded in epoxy.

This makes a solid permanent connection that cant come loose

or break. All of my discharge terminals simply thread onto the

central bolt. This makes changing terminals quick and easy.

This method seems to work well and I have succeeded in getting 54

inch streamers from a 21 inch long secondary winding.

>

>

>(Just got hold of a 25 meter roll of 25kV EHT cable, and a possible

>source of free HV oil!  It's all comming together! :-)

Free HV oil is the best kind. :)

TTYL!

Mike Hammer

mhammer@misslink.net

From: ralph.hartwell@emachine.com (Ralph Hartwell)

Subject:  Re: RF chokes

Date: Tue, 21 Jul 1998 04:48:00 GMT

Organization: The Energy Machine Information System 504-733-3462

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

MH>>  I am going to try to make some RF chokes using 2 1/2" ferrite rings

MH>>from an old TV scanning coil. It then occurred to me that I might need

MH>>slots cut in them to reduce eddy currents. Is this needed at RF frequencys?

MH>No slots needed. Cutting a slot in a core to introduce an air

MH>gap is done to prevent core saturation. Core saturation will

MH>not be a problem in your RF chokes. Use them as is.

Ferrite is made from finely powdered iron pressed together under very

high pressure.  As such, it has microscopic air gaps between many of the

particles composing the ferrite.  In effect, that provides a built-in

air gap of sorts, making most ferrites much harder to saturate than an

iron structure of the same size.

Ralph

---

Date: Mon, 20 Jul 1998 22:38:47 -0700

From: Gene Waldo <gwaldo@1stnetusa.com>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Organization: S.A.P.I.E.N.T. (Surly, Agressive, Politically Incorrect Enemies of Non Thinkers)

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Que passa?

Sender: tesl-err@emachine.com

All,

Sure been quiet out there, lately! Some of it is my fault, too hot to do squat!

Mike, I know this won't elicit much sympathy, based upon your recent

tribulations, but (Dan will thrill to this!), my computer desk is all

cleared off! You can see it, now!

Not, however, thru dint of any untoward spasms of cleanliness; the book

shelf over the computer gave way and dumped all over me and half the

room! No serious damage, beyond loosing a wee dobble o' good Scotch!

Found my 1996 IRS forms!

I didn't think you could get that much shit on one shelf.... I guess, in

retrospect, you can't!

-------------------------------------------------

Gene Waldo -  gwaldo@1stnetusa.com  - "Igor in Residence"

---------------------------------------------------------------

What's good for the planet is more OZONE! See the machinery form up!

NEW PAGE!   http://members.1stnetusa.com/danmurphy/waldo.htm

X-Mailer: IPAD 2.0

Date: Tue, 21 Jul 1998 12:32:56 -0500

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: Que passa?

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

At 10:38 PM 7/20/98 -0700, you wrote:

>All,

>Sure been quiet out there, lately! Some of it is my fault, too hot to do

>squat!

Since my work place is in a detached garage with temperatures

hovering around 125 F (thats about 52 C for you Celsius types)

inside I havnt had desire to work on my new coil project.

I imagine the heat has put a damper on a lot of projects.

>

>Mike, I know this won't elicit much sympathy, based upon your recent

>tribulations, but (Dan will thrill to this!), my computer desk is all

>cleared off! You can see it, now!

Gene you know that a clean desk is a sign of a sick mind.

That means my mind must be one of the healthiest around.

My guess is your desk wont remain visible for very long.

Perhaps its already heaped full?

>

>Not, however, thru dint of any untoward spasms of cleanliness; the book

>shelf over the computer gave way and dumped all over me and half the

>room! No serious damage, beyond loosing a wee dobble o' good Scotch!

>Found my 1996 IRS forms!

Hey, you cant be spilling the Scotch. Definitely bad form.

>

>I didn't think you could get that much shit on one shelf.... I guess, in

>retrospect, you can't!

Just like the old adage  " How far do I tighten this bolt?"

"Tighten until it breaks then back off a 1/2 turn"

TTYL!

Mike Hammer

mhammer@misslink.net

X-Originating-IP: [199.217.140.22]

From: "Bill the arcstarter" <arcstarter@hotmail.com>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Que passa?

Date: Tue, 21 Jul 1998 11:38:59 PDT

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

Coilers,

It was written:

>>room! No serious damage, beyond loosing a wee dobble o' good Scotch!

>

>Hey, you cant be spilling the Scotch. Definitely bad form.

I must agree whole heartedly! :)

I'd thought about winding a tesla coil on the cardboard tube my last

bottle of Lagavulin lived in... http://www.scotch.com

(note to non scotch drinkers - the better brands are protected by a

cardboard tube)

It's been too hot to do any serious coiling.  I just fixed my truck

with the boom arm lift.  Now I'm prepared to 'rescue and relocate' any

fallen pole transformers knocked out of their lofts by the summer

storms!  Heh heh heh...

SOMEONE (won't say who - will I, Paul?) BURNED OUT my 140KV bridge

rectifier board using a certain bank of pulse capacitors.  96 diodes -

all fused and fragmented in a microsecond!  Fellow mad scientists -

please note that the current limiting resistor should be placed between

the capacitor and the bridge, and NOT just between the bridge and the

transformer.. Arrrrrh!

-Bill the arcstarter

Starting arcs in Cinci, OH

http://www.geocities.com/CapeCanaveral/Hangar/6160

Date: Tue, 21 Jul 1998 20:24:05 -0600 (MDT)

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Daniel Lockhart <molly@cmn.net>

Subject: New Coil...

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

Hey all,

This list had been rather dead lately. It's not too hot where I am, indeed,

I get a nice cool breeze off of timberline every once-in-a-while, so I've

been cool and busy calculating for a new coil. For some excitement here's

what I've come up with. I'd appreciate it if everybody would be real

critical with it, as it would possibly make some good debate, and help show

any errors or brain-clogs I had...

Top to bottom:

--------------------------------------------------------------------------------

Toroid: 	7-inch cross section, 24.5-inch diameter aluminum dryer duct,

calculated capacitance of 20 pF, mounted about 2 to 4 inches

above the secondary

Secondary:	6.5-inch diameter PVC pipe, 28 inches long, wound to 26 inches

with 963 turns of 1638 feet of 22 awg wire. Approximate unloaded

resonate frequency: 247.7 kHz

Primary:	Copper refrigeration tubing, 0.375 inches diameter, spaced at

0.25 inches at a zero degree angle. Inside diameter: 8 inches.

Outside diameter: Whatever it takes to use up 50 feet (more than

10 turns -- I haven't calculated it yet)

Capacitor:	0.027 uF maximum capacitance. Saltwater caps to start out with

and nail down precise capacitance and then either rolled

or stacked poly

Spark Gap:	Eight 1-inch diameter copper electrodes, spaced at 0.02 inches.

Mounted on the outside of a box, with slots cut in the box where

the arcing occurs. A vacuum cleaner will then be connected to

the box to ensure quick quenching.

Chokes:		Two 2-inch diameter PVC pipes, 8 inches long wound to 6 inches

with 28 awg wire. Two power resistors of the appropriate resistance.

Safety Gaps:	Two high voltage electrodes sparking to ground.

Maybe the elegant Waldo design...

Power Tranny:	Four 12 kV, 30 ma neon sign transformers for a total of

12 kV, 120 ma. I haven't two of the needed transformers, so the

remaining two may be 60 ma each, bringing the current to 180 ma.

--------------------------------------------------------------------------------

Possible Upgrades:

The primary could be make with a larger inside dimeter to

allow the placement of a 8-inch diameter secondary. This would

decrease to turns needed for coupling with the 6-inch

secondary however.

The 30 ma transformers could be rebuilt to give 60 ma each,

giving a total of 240 ma to the circuit. This would be

a pain though.

--------------------------------------------------------------------------------

How does everything sound? Is the toroid a little big? I intentionally made

the diameter/cross-section ratio small (about 3.5) to create a higher

breakout voltage.

Is the size of the secondary a little extravagant for the amount of power

I'm giving it? Is the idea to make the six inch secondary replaceable with

an eight inch a good one? Ja, no?

As four neons are rather heavy, I'm planning on making the whole coil one

unit, sans the transformers. A separate enclosure will be made for them and

the variac and the 12 kv lines will be wired some distance from them to the

coil and safety gap. I was also going to add two safety gaps, one at the

coil and one right next to the neons. I'm worried that the lines from the

neons to the coil could pick up some extra voltage from the air.

Comments, complaints, suggestions?

Daniel Lockhart

Date: Tue, 21 Jul 1998 22:06:07 -0500

From: Bert Hickman <bert.hickman@aquila.com>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Organization: Stoneridge Engineering

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: New Coil...

Sender: tesl-err@emachine.com

Daniel,

Just about everything looks OK, except that you need to have at least

1.5" of clearance all around between the secondary and the innermost

turn of the primary, making your primary inside diameter 9.5". A 50 foot

roll of tubing will give you about 11 turns, and about 57 uH of

inductance. Good luck!

-- Bert --

Daniel Lockhart wrote:

>

> Hey all,

>

> This list had been rather dead lately. It's not too hot where I am, indeed,

> I get a nice cool breeze off of timberline every once-in-a-while, so I've

> been cool and busy calculating for a new coil. For some excitement here's

> what I've come up with. I'd appreciate it if everybody would be real

> critical with it, as it would possibly make some good debate, and help show

> any errors or brain-clogs I had...

>

<SNIP>

X-Mailer: IPAD 2.0

Date: Wed, 22 Jul 1998 11:51:12 +0300

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Marco Denicolai <marco@vistacom.fi>

Subject: My simple WWW page

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

I got now a fresh, incomplete and "easy" home page filled with my TC stuff:

pictures and technical data of my coil included.

Check it out at http://vci.vistacom.fi/~marco/

I'll add more data and pictures in the near future.

Marco Denicolai

X-Originating-IP: [168.18.138.99]

From: "Matt Brown" <the_hanged_man@hotmail.com>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Question about capacitors

Date: Wed, 22 Jul 1998 09:32:29 PDT

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

I'm just starting out, and I was wondering about how one determines the

capacitance on home-made saltwater caps.  If you guys could give me a

little help, it'd be much appreciated.

Ciao,

Matt Brown

From: "Basura, Brian" <brian.basura@unistudios.com>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Modified Variac Inductance?

Date: Wed, 22 Jul 1998 10:29:04 -0700

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Hello,

Can anyone tell me what the inductance range is for a modified variac

used to limit current?

Thanks,

Brian D. Basura

USITG - Manager, Technical Support

X-Mailer: IPAD 2.0

Date: Wed, 22 Jul 1998 12:41:52 -0500

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: My simple WWW page

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

At 11:51 AM 7/22/98 +0300, you wrote:

>I got now a fresh, incomplete and "easy" home page filled with my TC stuff:

>pictures and technical data of my coil included.

>

>Check it out at http://vci.vistacom.fi/~marco/

>

>I'll add more data and pictures in the near future.

>

>

>

>

> Marco Denicolai

Marco,

Nice web page and very nice looking coil. Seems to work pretty good too.

Well done. You even included a picture of yourself.

You wont ever see a picture of me on my web page.

Same goes for most coilers pages. I have a feeling that

most coilers are a rather ugly looking bunch. Must be

the ozone.

TTYL!

Mike Hammer

mhammer@misslink.net

X-Mailer: IPAD 2.0

Date: Wed, 22 Jul 1998 12:50:08 -0500

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: Question about capacitors

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

At 09:32 AM 7/22/98 PDT, you wrote:

>I'm just starting out, and I was wondering about how one determines the

>capacitance on home-made saltwater caps.  If you guys could give me a

>little help, it'd be much appreciated.

>

>Ciao,

>Matt Brown

>

Matt,

When I used SW caps I got about 720 pf per 12 oz beer bottle.

It was suprisingly consistant. If using other types bottles

it will be difficult to determine. Best bet is to purchase

or borrow a meter to measure the capacitance.

There is a formula for SW caps that I will paste in here

for you. Ive never used this formula myself so cant vouch

for its accuracy.

2

C = .0884 k ( pi  r   + 2  pi  r  l )

------------------------------

1,000,000 t

C = Capacitance in microfarads

k = dielectric constant

r = jar radius in centimeters

l = height of the jar portion used (in centimeters)

t = thickness of the jar wall in centimeters

pi = 3.141592654

2

r   = r x r  (radius squared)

The k value (dielectric constant) of glass is highly variable

ranging from 5 to 8. Use a nominal value of 7.

Hope this helps.

TTYL!

Mike Hammer

mhammer@misslink.net

Date: Wed, 22 Jul 1998 13:15:45 -0700

From: Gene Waldo <gwaldo@1stnetusa.com>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Organization: S.A.P.I.E.N.T. (Surly, Agressive, Politically Incorrect Enemies of Non Thinkers)

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: My simple WWW page

Sender: tesl-err@emachine.com

Mike Hammer wrote:

>

> > Marco Denicolai

>

> Marco,

> Nice web page and very nice looking coil. Seems to work pretty good too.

> Well done. You even included a picture of yourself.

Ditto, Marco!

> You wont ever see a picture of me on my web page.

> Same goes for most coilers pages. I have a feeling that

> most coilers are a rather ugly looking bunch. Must be

> the ozone.

>

> TTYL!

> Mike Hammer

> mhammer@misslink.net

You are right! Murphy has also exposed himself on his page- and has not

been subjected to much ozone yet! So, you may be wrong about that aspect.

I toy with the (pic) concept, if and when my page ever materializes...

My problem began years ago, when I had an ozone source running most of

the time, in my small, but poorly equipped shop. I just liked the smell!

While it rotted most oxidizable material, it was fun to watch! I could

even be seen playing in it, on occasion.  Do not be too afraid of

entering the 'streamer-zone'- just apply some common sense fear and

dread, coupled with a competent working knowledge of the potentials* for

death, or at least setting your hair afire: strikes to the scalp, if not

clean and ungreased, may present somewhat of a problem to the

unwashed.

*Pun Intended.

--------------------------------------------------

Gene Waldo -  gwaldo@1stnetusa.com  - "Igor in Residence"

X-Mailer: IPAD 2.0

Date: Wed, 22 Jul 1998 16:06:57 -0500

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: New Coil...

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

At 08:24 PM 7/21/98 -0600, you wrote:

>Hey all,

>

>This list had been rather dead lately. It's not too hot where I am, indeed,

>I get a nice cool breeze off of timberline every once-in-a-while, so I've

>been cool and busy calculating for a new coil. For some excitement here's

>what I've come up with. I'd appreciate it if everybody would be real

>critical with it, as it would possibly make some good debate, and help show

>any errors or brain-clogs I had...

>

<SNIP SPECS>

>How does everything sound? Is the toroid a little big? I intentionally made

>the diameter/cross-section ratio small (about 3.5) to create a higher

>breakout voltage.

Toroid is a little large but not too large for the

intended power level. You may need a point discharger

on it to promote breakout for tuning purposes.

>

>Is the size of the secondary a little extravagant for the amount of power

>I'm giving it?

No it seems about right to me.

>Is the idea to make the six inch secondary replaceable with

>an eight inch a good one? Ja, no?

No. If you make it wide enough for an 8 incher then you will

likely have poor coupling with a 6 inch secondary. Best bet

is to remove a couple inside turns when you make the move to a

8 inch secondary. Since most of the inductance is in the outer turns

you wont lose much inductance.

>

>As four neons are rather heavy, I'm planning on making the whole coil one

>unit, sans the transformers. A separate enclosure will be made for them and

>the variac and the 12 kv lines will be wired some distance from them to the

>coil and safety gap. I was also going to add two safety gaps, one at the

>coil and one right next to the neons. I'm worried that the lines from the

>neons to the coil could pick up some extra voltage from the air.

The extra long lines can radiate enough RFI to be a problem.

Keep them no longer than necessary and be sure to use RF chokes

to help keep the RF out of the lines.

I like the idea of two safety gaps.

A better plan may be to place the neons directly underneath

the coil and run long 120V lines to the neons.

Other than those minor details it sounds like a good system to me.

TTYL!

Mike Hammer

mhammer@misslink.net

X-Mailer: IPAD 2.0

Date: Thu, 23 Jul 1998 08:08:33 +0300

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Marco Denicolai <marco@vistacom.fi>

Subject: MOT supply and last TC tests

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Mike Hammer was curious to know about my MOT supply working without doubler.

I estimated that the MOT supply provides 6.5kV, then without doubler should

be one half, i.e. 3.25 kV.

I tested it yesterday and I got, consistently, sensibly shorted streamers:

20" compared with 40" with the doubler. I had also to reduce the number of

gaps (2 instead of 4).

Because my doubler capacitors are 1uF each, and my primary capacitor 20 times

less (0.05uF) it should make sense that the doubler can effectively charge

the primary cap.

Yesterday I tested with full doubler and 4 0.02" gaps and the spark gap was

sligthly warm. Then I tested with 3 0.02" gaps: I got longer streamers to

a grounded rod but the spark gap warmed up fast (I couldn't run for more

than 20 s. at a time). I was surely having a break rate higher than 50 Hz

with 3 gaps.

Conclusion 1: "higher break rate" is better than "50 Hz break rate + higher

voltage".

Conclusion 2: because the spark gap gets hot, I think the best opportunity for

a MOT tank is to use a rotary spark gap. I tested RQ gaps of 0.01": too close,

they get shorted. And gaps of 0.027": too loose, no tuning possibility.

Conclusion 3: the voltage doubler does give benefit at no cost.

Marco Denicolai

X-Originating-IP: [168.18.138.98]

From: "Matt Brown" <the_hanged_man@hotmail.com>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Question about grounding.

Date: Thu, 23 Jul 1998 09:36:42 PDT

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

I've got another question about my coil-in-progress.  What does one do

about grounding the secondary?  Is it acceptable to use the ground from

your wall socket?  Do you actually have to attach it into the ground?

Thanks for your help.

Matt

X-Mailer: IPAD 2.0

Date: Thu, 23 Jul 1998 12:00:43 -0500

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: Question about grounding.

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

At 09:36 AM 7/23/98 PDT, you wrote:

>I've got another question about my coil-in-progress.  What does one do

>about grounding the secondary?  Is it acceptable to use the ground from

>your wall socket?  Do you actually have to attach it into the ground?

>Thanks for your help.

>

>Matt

>

Matt,

You must NOT use your wall socket ground. The base of the

secondary is a high current point and this current must be

given a low impedance path to ground. Allowing this RF current

to flow onto your house ground will cause all manners of problems

as well as giving poor performance.

You need to establish a direct RF ground for your secondary.

The simplest method is to drive a pair of 4 foot ground rods

into the ground near where you intend to run your coil.

These rods are then connected to the base of the secondary

via a run of heavy wire or cable. This run must be as short

as possible and should be as straight as possible.

The longer you have to make this ground lead the heavier

the conductor needs to be to mantain low impedance.

TTYL!

Mike Hammer

mhammer@misslink.net

X-Originating-IP: [168.18.138.120]

From: "Matt Brown" <the_hanged_man@hotmail.com>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Help with my little coil.

Date: Thu, 23 Jul 1998 14:31:02 PDT

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

Okay guys... I've been asking a few questions and doing a lot of

research, and I've put together a tabletop-sized coil.  It seems that

all the individual parts are working correctly, but I'm just not getting

any output at all.  Here are the specs for my coil:

Power source:  15kV, 90mA neon transformer.

Caps: 3 saltwater caps, wired in parallel

Spark Gap:  R Quick type... (wires in a pvc pipe).  Cooled by small

fan.

Primary coil: Helical, 8 turns of thick (1/4 inch or so) copper wire.

Diameter 6 inches

Secondary Coil: 1000ft of 30 awg magnet wire, wound around 3.5 inch

PVC pipe, about 14 inches long.   I've had it grounded to the table

and the floor... I've only been able to work in my 3rd story science

lab so far.

Output terminal: Small sphere (about 1 inch diameter... just a

temporary thing).

I know these aren't the best materials, but I had the considerations of

time and money to work with because I'm doing this at a 6 week,

state-funded program.  I'm just trying to get it working, not

necessarily at optimum, for a presentation tonight.  My only thoughts on

what the problem could be are the grounding of the secondary or the

magnetic coupling between the coils.  Any ideas on a quick fix, or have

a done something totally wrong?  The spark gap fires fine, everything

seems to be running at optimum, exept there's nothing going through the

secondary.

Thanks for all your help,

Matt

From: <ESchulz531@aol.com>

Date: Thu, 23 Jul 1998 17:42:30 EDT

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Help with my little coil.

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Hello,

It seems like your primary resonate frequency isn't low enough.  Try more

saltwater caps or add some more inductance to your primary coil.

Erik

X-Originating-IP: [168.18.138.116]

From: "Matt Brown" <the_hanged_man@hotmail.com>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Help with my little coil.

Date: Thu, 23 Jul 1998 14:52:17 PDT

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

>It seems like your primary resonate frequency isn't low enough.

>Try more saltwater caps or add some more inductance to your primary coil.

Okay... I'll try adding the caps.  How can I add inductance to the

primary, exactly?

Matt

From: <ESchulz531@aol.com>

Date: Thu, 23 Jul 1998 19:36:19 EDT

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Help with my little coil.

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Matt,

Add more turns to the primary or reduce the spacing.  Most people just add

more turns since its easier.  Add about 4 turns to your coil and then try

tapping it at each turn.

Erik Schulz

Date: Thu, 23 Jul 1998 19:41:17 -0500

From: Tedd Payne <tpayne@netnitco.net>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: For Sale:  Neon Transformers & HV Wire

Sender: tesl-err@emachine.com

If interested, please contact me DIRECTLY at tpayne@netnitco.net

Please tell me which one(s) you're interested in, and your ZIP CODE to

determine shipping cost.

Thanks,

Tedd

"CONVENTIONAL" neon transformers, not old, tested, good

working condition.  Many have labels, if not, the ratings are from my

testing/measuring.  Good for Tesla coil, Jacob's ladder, HV cap charging

(with proper rectification), "hi-potting", experiments, etc.

$17 used  15kv30ma  in=porc ins   out=porc ins   no lid

$17 used  15kv30ma  in=AC cord   out=2 wires

$16 used  15kv30ma  in=2 wires   out=2 wires    no lid

$13 used  15kv30ma  in=porc ins   out=porc ins   1 HV ins missing

$11 used  15kv30ma  in=porc ins   out=porc ins   1 HV loose & chipped & 1 missing

$15 used  15kv30ma  in=porc ins  out=porc insul (1 loose HV insul)

$14 used  12kv30ma  in=3 wires   out=2 wires   w/switch,

insul little worn on 1 HV wire

$12 used  12kv30ma  in=porc insul

output was porc insul, but both missing,

now is 1-inch HV wire leads

$10 used  12kv30ma  in=porc ins  out=porc ins

(both HV insul loose, one is chipped)

$14 used  12kv30ma  in=3-inch wires  out=8-inch HV wires

has pull-chain type switch

$13 used  9kv30ma  in=3 wires   out=2 wires   w/switch

$12 used  9kv30ma  in=porc ins  out=porc ins  1 HV ins chipped

$11 used  7.5kv30ma  in=porc ins  out=porc ins

$10 used  6kv30ma  in=porc ins  out=porc ins

NEW Neon transformer HV wire:

(the "gray" has a larger conductor AND is MORE flexible)

Flexible gray, 14 gauge, multistrand copper, rated 15KV, $1.00 per foot.

Standard black, 16 gauge, multistrand copper, rated 15KV, $.50 per foot.

AC power cord for transformer, 3-wire, grounded plug, used, $.50

MODEL T SPARK COIL  $18

- original, not reproduction

- spark a little over 1/4", maybe more with new interrupter set.

- good visual & operating cond.

- operates on 6VDC input

From: <Dsurfr@aol.com>

Date: Thu, 23 Jul 1998 21:17:14 EDT

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Toroid size

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Hey all: A friend gave me a 17"O.D.x4" toroid. I have a 4 1/2"x22"L. winding

of 24G. Secondary but only powered by a 15K 30 mA neon transformer. Is this

way too big or can I try to re-tune to use it? Just wanted to get some advice

before I tried. If it's too large I'll save it for when I add a second

transformer but it looks great sitting on top. Thanks as usual for the advice.

Jim

X-Originating-IP: [168.18.138.99]

From: "Matt Brown" <the_hanged_man@hotmail.com>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Help with my little coil.

Date: Thu, 23 Jul 1998 18:33:06 PDT

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

>Matt,

>Add more turns to the primary or reduce the spacing.  Most people just add

>more turns since its easier.  Add about 4 turns to your coil and then try

>tapping it at each turn.

>

>Erik Schulz

Well, after adding two more saltwater caps (glad I had made spares) for

a total of five, and tapping at the lowest possible, and making a few

adjustments to the positioning of the primary, it worked!  Only minutes

before showtime, it worked.  Not very well, but nonetheless, I was able

to do some demonstrations.  I could do a little plasma sphere out of a

light bulb, I could light a flourescent tube from an inch or two away,

and I could draw about 1/2 inch sparks to a ground wire.  Again, not

terribly impressive, but the audience ate it up.  (Small, science

fair-style show).  I was the most popular booth.  Now, I need to tweak

it to maximum performance.  My first step is better grounding... right

now I'm running a small wire to the floor of my 3rd story labratory.

Any suggestions on improving that?  Any other suggestions... remember,

its just a little table top model... i can only do so much with it.  I

think coiling is going to become a permanent hobby, however!  Thanks

erik, for your last minute help.

Ciao,

Matt

Date: Thu, 23 Jul 1998 20:37:10 -0500

From: Bert Hickman <bert.hickman@aquila.com>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Organization: Stoneridge Engineering

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Toroid size

Sender: tesl-err@emachine.com

Dsurfr@aol.com wrote:

>

> Hey all: A friend gave me a 17"O.D.x4" toroid. I have a 4 1/2"x22"L. winding

> of 24G. Secondary but only powered by a 15K 30 mA neon transformer. Is this

> way too big or can I try to re-tune to use it? Just wanted to get some advice

> before I tried. If it's too large I'll save it for when I add a second

> transformer but it looks great sitting on top. Thanks as usual for the advice.

> Jim

Jim,

Sounds like a nice-sized toroid for your coil! Go for it!

-- Bert --

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Date: Thu, 23 Jul 1998 19:07:58 -0700

From: "Fleischwulf" <fleischwulf@mailexcite.com>

Cc:

X-Sent-Mail: off

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Expiredinmiddle: true

X-Mailer: IPAD 2.0

Subject: Letting the smoke out !!!

X-Sender-Ip: 208.16.148.68

Organization: MailExcite  (http://www.mailexcite.com:80)

Sender: tesl-err@emachine.com

Well, I have designed a new coil. Its a 6.5" with a 35 mh inductance. The primary is 26" in diameter. It consists of 3/8 ref tubing with a inside diameter of 10". I did not have the 6.5" to try so I used my old 4.5" coil. The power level was 14.

4 Kv at 1/2 amp. This gave me an output of 7.2 Kw. I threw some tremendous arcs and wound up setting my rotary gap on fire. The arcs terminated against the ground some 50" away. It was the nearest grounded object. Next I will try the 6.5" I thin

k I'd use a faraday cage. !

Paul M,

---

Leben frei oder Starb !

X-Mailer: IPAD 2.0

Date: Thu, 23 Jul 1998 23:51:51 -0500

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: Help with my little coil.

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Matt,

Im combining responses from a couple of your posts

into a single reply.

At 02:31 PM 7/23/98 PDT, you wrote:

>Okay guys... I've been asking a few questions and doing a lot of

>research, and I've put together a tabletop-sized coil.  It seems that

>all the individual parts are working correctly, but I'm just not getting

>any output at all.  Here are the specs for my coil:

>

>Power source:  15kV, 90mA neon transformer.

This is not a table top coil power level. Way too much

juice here. This is probably three 15kv/30s in parallel.

Use a single neon at first. The excess power is just burning

up your gap and making things harder to tune.

>

>Caps: 3 saltwater caps, wired in parallel

Probably insufficient capacitance. For a single 15kv/30ma

neon you will need about .0053 mfd.

>

>Spark Gap:  R Quick type... (wires in a pvc pipe).  Cooled by small

>fan.

>

>Primary coil: Helical, 8 turns of thick (1/4 inch or so) copper wire.

>Diameter 6 inches

Diameter is good but you probably need more turns. I would

want at least 12.

>

>Secondary Coil: 1000ft of 30 awg magnet wire, wound around 3.5 inch

>PVC pipe, about 14 inches long.   I've had it grounded to the table

>and the floor... I've only been able to work in my 3rd story science

>lab so far.

>

A 14 inch winding of AWG 30 will be about 1250 turns and

an unloaded resonant frequency of about 375 khz. Thats

pretty low for this size of coil. You usually want the

turns count less than 1000 turns. A somewhat longer winding

of AWG 26 would be a better secondary.

>Output terminal: Small sphere (about 1 inch diameter... just a

>temporary thing).

>

>Well, after adding two more saltwater caps (glad I had made spares) for

>a total of five, and tapping at the lowest possible, and making a few

>adjustments to the positioning of the primary, it worked!  Only minutes

>before showtime, it worked.  Not very well, but nonetheless, I was able

>to do some demonstrations.  I could do a little plasma sphere out of a

>light bulb, I could light a flourescent tube from an inch or two away,

>and I could draw about 1/2 inch sparks to a ground wire.

The fact that you are now getting some corona and small sparks

indicates your coil is generating some RF. But not very much.

Its still very much out of tune.

You said "tapping at the lowest possible". What did you mean.

Were you tapping in all of the available primary turns?

If you get best output with all primary turns in that means

your primary circuit is not tuning low enough to match the

secondary. The cure is too add more primary turns. Having

too many is better than not enough. Once you find resonance

you can always rewind a more permanent primary with the proper

number of turns.

>My first step is better grounding... right now I'm running a small wire

>to the floor of my 3rd story labratory.

>Any suggestions on improving that?

Thats going to be difficult. A real RF ground is not easy

3 floors up. You could try metal girders in your building

if it has them. Water pipes would not be a good choice.

You may have to build an artificial ground known as a counterpoise.

A sheet of metal or screen wire several feet in diameter could

work as a makeshift ground for low power. This type of ground

will severely limit performance.

A heavy cable out the window and straight down to several ground rods

in parallel may be better.

TTYL!

Mike Hammer

mhammer@misslink.net

Date: Sat, 25 Jul 1998 13:08:30 +0100

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Stephen Bell <steve@breakfix.demon.co.uk>

Subject: Secondary former material

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Hi Guys,

Before I start with another question I want to thank Mike Hammer

for the answers to my previous questions, and for all my further

questions!  ;-)

I've been in touch with a company that can supply me with some

plastic tubing for my secondary coil former.  Before I get back to them

I need to know which material would be best, in terms if dielectric

strength and dissipation factor etc. (I think the most important

property is a low dissipation factor??).

Acrylic.

High density polyethelyne.

Polypropylene.

On a different note, I haven't yet built my first coil but I

would like to say that there is nothing like playing with a Jacobs

Ladder to give you an appreciating of the potential dangers of high

voltages!  I got hold of an old 'Simco' static-eliminator PSU from work,

with a rated output of 7.5kV@5mA, and got a spark/arc from the

terminals.  As you will probably guess, the spark could jump a fair way

but could only ionise the air and maintain an arc for a tiny gap!

Some weeks later I got hold of my NST (10kV@100mA).  As its

output was only 2.5kV greater than the 'Simco' unit I figured I'd get a

33% increase in spark length (2.5kV being a third of 7.5kV) and didn't

think of what else I might get (boy was I in for a surprise!).

I hooked up a simple Jacobs Ladder with some solid core copper

wire, approx 40 thou diameter, stood back, and switched on!  WOW!!  Did

I get a longer spark?  Couldn't tell!  There was a brilliant fat arc,

angrily buzzing up the wires, stretching out to good 4 inches!!

I found myself sitting a couple of feet further away from where

I started.  My mouth was wide open and a silly grin was starting to

spread from ear to ear!  I now understood how exciting an operating

Tesla coil would be AND how dangerous it could be!

50% of the design will be safety!

TIA & TTYL.

-------------------------------------------

Stephen Bell   (steve@breakfix.demon.co.uk)

-------------------------------------------

Date: Sat, 25 Jul 1998 07:40:03 -0500

From: Bert Hickman <bert.hickman@aquila.com>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Organization: Stoneridge Engineering

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Secondary former material

Sender: tesl-err@emachine.com

Stephen Bell wrote:

>

> Hi Guys,

>         Before I start with another question I want to thank Mike Hammer

> for the answers to my previous questions, and for all my further

> questions!  ;-)

>

>         I've been in touch with a company that can supply me with some

> plastic tubing for my secondary coil former.  Before I get back to them

> I need to know which material would be best, in terms if dielectric

> strength and dissipation factor etc. (I think the most important

> property is a low dissipation factor??).

>

>         Acrylic.

>         High density polyethelyne.

>         Polypropylene.

>

<SNIP>

Stephan,

Most coilers initially think this, and there's also a fair amount of

folklore out there that suggests this. Actually, the most important

parameters are insulation strength and leakage resistance. A well

designed system may have sparks that are over 3X the coil length, and

you'll need all the help you can get to keep it from breaking down under

the stress. Properly treated PVC works very well, even though it has a

significantly higher dissipation factor than PP, PS, or HDPE.

Once you get heavy streamers coming off the top, the Q of the secondary

drops dramatically (typically by a factor of 10:1 or more), and any

dielectric losses coming from the former itself become negligible. You

may, in fact, see a larger contribution of dielectric losses from the

type of wire insulation and sealing coating used over the wire coil due

to the relatively high turn-turn RF voltages that are developed.

Play safe!

-- Bert --

X-Mailer: IPAD 2.0

Date: Sat, 25 Jul 1998 15:48:00 +0100

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: bob golding <yubba@clara.net>

Subject: Re: Secondary former material

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

At 13:08 25/07/98 +0100, you wrote:

>        I hooked up a simple Jacobs Ladder with some solid core copper

>wire, approx 40 thou diameter, stood back, and switched on!  WOW!!  Did

>I get a longer spark?  Couldn't tell!  There was a brilliant fat arc,

>angrily buzzing up the wires, stretching out to good 4 inches!!

>        I found myself sitting a couple of feet further away from where

>I started.  My mouth was wide open and a silly grin was starting to

>spread from ear to ear!  I now understood how exciting an operating

>Tesla coil would be AND how dangerous it could be!

>        50% of the design will be safety!

I have built Jacobs ladders and they are great fun. Try using gas welding

rods. They are copper coated steel so stay parallel. If you use a variac

you can control the speed the arc runs up the ladder. I have noticed that

the arc gets faster as you lower the voltage. Does anyone know why this

is?.  A trick I tried was to fix a couple of sparklers to the ends and let

the arc light them. Great party trick. Does anyone know if the levels of UV

off a Jacobs ladder are dangerous? and if so would a sheet of perspex/lexan

help to reduce this? Remember the arc of a Jacobs ladder is at line

frequency so is a lot more dangerous than the sparks from the secondary of

a Tesla coil, in theory at least.

Also reluctant to be a dummy load in east Anglia.

bob golding

Date: Sat, 25 Jul 1998 23:50:29 +0100

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Stephen Bell <steve@breakfix.demon.co.uk>

Subject: Re: Secondary former material

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

In message <35B9D223.1F6E@aquila.com>, Bert Hickman

<bert.hickman@aquila.com> writes

>Stephen Bell wrote:

>>

>> Hi Guys,

>>         Before I start with another question I want to thank Mike Hammer

>> for the answers to my previous questions, and for all my further

>> questions!  ;-)

>>

>>         I've been in touch with a company that can supply me with some

>> plastic tubing for my secondary coil former.  Before I get back to them

>> I need to know which material would be best, in terms if dielectric

>> strength and dissipation factor etc. (I think the most important

>> property is a low dissipation factor??).

>>

>>         Acrylic.

>>         High density polyethelyne.

>>         Polypropylene.

>>

>

>Most coilers initially think this, and there's also a fair amount of

>folklore out there that suggests this. Actually, the most important

>parameters are insulation strength and leakage resistance. A well

>designed system may have sparks that are over 3X the coil length, and

>you'll need all the help you can get to keep it from breking down under

>the stress. Properly treated PVC works very well, even though it has a

>significantly higher dissipation factor than PP, PS, or HDPE.

>

>Once you get heacy streamers coming off the top, the Q of the secondary

>drops dramatically (typically by a factor of 10:1 or more), and any

>dielectric losses coming from the former itself become negligible. You

>may, in fact, see a larger contribution of dielectric losses from the

>type of wire insulation and sealing coating used over the wire coil due

>to the relatively high turn-turn RF voltages that are developed.

>

>-- Bert --

>

Hi Bert,

from what you say, am I right in thinking that the material the

former is made of is of little importance, compared to the properties of

the materials immediatley adjacent to the wire, i.e. the wire coating

and any polyurathane coating applied?

You mention 'leakage resistance' but I'm not sure what this

means.  Would space-winding (i.e. leaving a gap between each turn ) be

worth while for the secondary, to increase insulation, or would that

have other penalities?

TIA.

-------------------------------------------

Stephen Bell   (steve@breakfix.demon.co.uk)

-------------------------------------------

Date: Sat, 25 Jul 1998 16:35:51 -0700

From: Michael Loftus <loftusm@ix.netcom.com>

Organization: Me

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Secondary former material

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Hi,

Well, I am back from vacation in North Carolina and Virginia and am ready

to tackle my TC again!

Up to this point the coil is winning and I am loosing. . .

I have built a "common" design:

-  4.5" x 22.5" of #24 secondary. Coated with HV potting epoxy.

-  flat spiral primary of 1/4" copper tubing spaced 3/8" between 14 turns;

6.5" inside  diameter.  Coplaner with first winding of secondary.

-  .0069 mf obtained by widing 3 poly/AL caps and placing in series.

-  Various toroids tried, ranging from AL-foil covered volly-ball to a 16" loop           of AL dryer ducting.

-  Simple spark-gap consisting of two stainless cap-nuts on bolts encased in

plastic pipe and end-caps.

-  13.5 kv DC @ 30ma obtained from a 15 kv neon and full-wave rectifier.

-  Variac controlled.

The maximum spark I have obtained is about 8".  Ceiling tubes flicker about 1/2 their length when TC is running.  I have also tried several commercial caps in series and obtained similar sparks, but with higher mf and fewer windings.

At this point I have no idea what to do.  I am guessing that while I am indeed

able to create some level of resonance it must be at other than the fundamental frequency of the secondary.  Also, each of the three caps is about 14" long,

making for a rather long circuit path.

From descriptions I have read, I should be able to coax much longer sparks from this general configuration.  I am sure I need to build a rotary gap of some sort, but I had hoped to do better at this point.  My goal is to have this completed by Fall.

HELP!!!!!!!!!!!!!!!!!!

Mike

From: Tom Stock <tomstock@geocities.com>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Kewl new tesla experiments

Date: Sun, 26 Jul 1998 08:58:30 -0400

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

Yesterday I found a neat trick:

Take a piece of that clear packing tape, and run it from your toroid to a grounded object about twice the distance of your maximum spark. Make sure the grounded object is level with your toroid or slightly higher.

The streamers will start to follow the tape VERY SLOWLY...like water making a little trail on a driveway. It will then start to get close to the grounded object, and will grab it like a demon and spark, scream and fight as it jumps from the tape to ground, giving you a solid arc from the toroid, thru the tape, to the object.

I was getting a solid 4-5ft arc from a 15KV/60ma coil.

try it, it's really neat!

-tom

X-Originating-IP: [208.16.145.139]

From: "Bill the arcstarter" <arcstarter@hotmail.com>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Kewl new tesla experiments

Date: Sun, 26 Jul 1998 09:14:22 PDT

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

Tom wrote:

>The streamers will start to follow the tape VERY SLOWLY...like water

>making a little trail on a driveway. It will then start to get close to

>the grounded object, and will grab it like a demon and spark, scream and

>fight as it jumps from the tape to ground, giving you a solid arc from

That sounds great!  Last year we did something kinda like that, using a

2 ft long piece of PVC sewer pipe.  I ran a length of 30 gauge copper

wire from the toroid over the the top of the PVC.  Once fired - most of

the energy came off the end and middle of the wire (in the form of both

corona and arcs).  But - eventually the arc started wrapping down the

outside of the PVC, at about 2 inches per second.  Eventually it got

down to about 2-3 inches off the ground (concrete) when we turned the

power off since the top of the pipe was well engulfed in flame... :)

A pic of this particular experiment (prior to the fire) is at:

http://www.geocities.com/CapeCanaveral/Hangar/9561/arc4.html

Also:

http://www.geocities.com/CapeCanaveral/Hangar/9561/tesla.html

Someone (Mike Harrison?) says you can get a spark to travel large

distances inside clear vinyl tubing filled with argon - sounds similar!

Wonder what we can burn with our newest 6.25 inch secondary?

Play safe!  Only do this sort of thing at a friend's house, never at

your own! :) (This coil is at Paul's place - since he has the pig.)

-Bill the arcstarter

Starting arcs in Cinci, OH

http://www.geocities.com/CapeCanaveral/Hangar/6160

Date: Sun, 26 Jul 1998 09:47:27 -0700

From: Gene Waldo <gwaldo@1stnetusa.com>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Organization: S.A.P.I.E.N.T. (Surly, Agressive, Politically Incorrect Enemies of Non Thinkers)

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Kewl new tesla experiments

Sender: tesl-err@emachine.com

Tom Stock wrote:

>

> Yesterday I found a neat trick:

>

> Take a piece of that clear packing tape, and run it from your toroid to a grounded object about twice the distance of your maximum spark.

SNIPS

> try it, it's really neat!

> -tom

Tom,

Its stuff like that which could qualify you for an honorary IGOR button*!

Does the tape carbonize, along the way? How long does it last? Data on

MAX run length? Run time? When the tape collapses(?) will it set the

carpet afire? Would lighter fluid help?

* mabee there ought to be such???

I am about to place my 1st computer! out on the curb! 'Texas

Instruments Professional', purchased ca. 1986. Complete: CPU (2- 5.25"

floppys), T-I color monitor, kbd, T-I Omni 800/855 prn, ALL manuals,

cables, software, all docs. Lots of T-I RIX software, and more: DOS,

Word Perfect, Flight Simulator, BASIC, tutorials, SPEECH-COMMAND system

NIB, etc.  Operational except for Prn: PH xverse needs service.

Yours for freight. Otherwise, (sob!) trash!

--

Gene Waldo -  gwaldo@1stnetusa.com  - "Igor in Residence"

---------------------------------------------------------------

What's good for the planet is more OZONE! See the machinery form up!

NEW PAGE!   http://members.1stnetusa.com/danmurphy/waldo.htm

From: wwl@netcomuk.co.uk (Mike Harrison)

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Kewl new tesla experiments

Date: Mon, 27 Jul 1998 00:28:15 GMT

Organization: White Wing Logic

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

On Sun, 26 Jul 1998 08:58:30 -0400, you wrote:

>

>Yesterday I found a neat trick:

>

>Take a piece of that clear packing tape, and run it from your toroid to a grounded object about twice the distance of your maximum spark. Make sure the grounded object is level with your toroid or slightly higher.

>

>The streamers will start to follow the tape VERY SLOWLY...like water making a little trail on a driveway. It will then start to get close to the grounded object, and will grab it like a demon and spark, scream and fight as it jumps from the tape to ground, giving you a solid arc from the toroid, thru the tape, to the object.

>

>I was getting a solid 4-5ft arc from a 15KV/60ma coil.

>

>try it, it's really neat!

>-tom

I've always wondered what video tape would go like - I'd guess the

oxide particles may be conductive enough to do something interesting

before the backing melts - anyone tried it?

____                                                           ____

_/ L_/  Mike Harrison / White Wing Logic / wwl@netcomuk.co.uk  _/ L_/

_/ W_/  Hardware & Software design / PCB Design / Consultancy  _/ W_/

/_W_/  Industrial / Computer Peripherals / Hazardous Area      /_W_/

X-Mailer: IPAD 2.0

Date: Mon, 27 Jul 1998 13:01:14 +0300

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Marco Denicolai <marco@vistacom.fi>

Subject: Rotating spark gap design

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

I'm building a rotating spark gap, using a vegetable shredder (!) AC motor.

The motor is a 12000 rpm 700 W. I plan to mount (at first) 2 bolts for

a total of 400 breaks/sec.

I thought to use a 14 cm diameter disk, but G-10 material here in Finland

is sadly expensive. What will happen if I use lexan? Has anybody ever

tried with anything else than G-10?

Marco Denicolai

X-Mailer: IPAD 2.0

Date: Mon, 27 Jul 1998 12:21:58 -0500

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: Letting the smoke out !!!

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

At 07:07 PM 7/23/98 -0700, you wrote:

>Well, I have designed a new coil. Its a 6.5" with a 35 mh inductance.

>The primary is 26" in diameter. It consists of 3/8 ref tubing with a inside >diameter of 10". I did not have the 6.5" to try so I used my old 4.5" coil.

>The power level was 14.4 Kv at 1/2 amp. This gave me an output of 7.2 Kw. I

>threw some tremendous arcs and wound up setting my rotary gap on fire. The

>arcs terminated against the ground some 50" away. It was the nearest grounded >object. Next I will try the 6.5" I think I'd use a faraday cage. !

>

>Paul M,

>---

Paul,

Sounds like its working well. I imagine the 4 inch secondary

was rather undercoupled inside that roomy primary.

How did you set the gap on fire? Did you have the arc follow

around the moving electrodes instead of breaking the arc?

If so you need a static gap in series with the rotary.

You should never rely on a rotary to do the dirty work.

The rotary should be a commutator only. Let a good static

gap do all the hard work.

Let us all know how the new 6 incher works out.

TTYL!

Mike Hammer

mhammer@misslink.net

X-Mailer: IPAD 2.0

Date: Mon, 27 Jul 1998 12:39:26 -0500

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: Secondary former material

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

At 11:50 PM 7/25/98 +0100, you wrote:

>In message <35B9D223.1F6E@aquila.com>, Bert Hickman

><bert.hickman@aquila.com> writes

>>Stephen Bell wrote:

>>>

>>> Hi Guys,

>>>         Before I start with another question I want to thank Mike Hammer

>>> for the answers to my previous questions, and for all my further

>>> questions!  ;-)

>>>

>>>         I've been in touch with a company that can supply me with some

>>> plastic tubing for my secondary coil former.  Before I get back to them

>>> I need to know which material would be best, in terms if dielectric

>>> strength and dissipation factor etc. (I think the most important

>>> property is a low dissipation factor??).

>>>

>>>         Acrylic.

>>>         High density polyethelyne.

>>>         Polypropylene.

>>>

>>

>>Most coilers initially think this, and there's also a fair amount of

>>folklore out there that suggests this. Actually, the most important

>>parameters are insulation strength and leakage resistance. A well

>>designed system may have sparks that are over 3X the coil length, and

>>you'll need all the help you can get to keep it from breking down under

>>the stress. Properly treated PVC works very well, even though it has a

>>significantly higher dissipation factor than PP, PS, or HDPE.

>>

>>Once you get heacy streamers coming off the top, the Q of the secondary

>>drops dramatically (typically by a factor of 10:1 or more), and any

>>dielectric losses coming from the former itself become negligible. You

>>may, in fact, see a larger contribution of dielectric losses from the

>>type of wire insulation and sealing coating used over the wire coil due

>>to the relatively high turn-turn RF voltages that are developed.

>>

>>-- Bert --

>>

>

>Hi Bert,

>        from what you say, am I right in thinking that the material the

>former is made of is of little importance, compared to the properties of

>the materials immediatley adjacent to the wire, i.e. the wire coating

>and any polyurathane coating applied?

I wouldnt go so far as to say the form material is of little

importance but other factors are far more important.

As Bert mentioned above once the gap fires and the secondary becomes

loaded down with a streamer the system wide Q drops like a rock.

So going to extraordinary lengths to build a super low loss high Q

secondary is pretty much a wasted effort. The effort should go into

building low loss high Q primary components.

A mediocre secondary teamed up with a truly good primary system

will be a great performer. But the greatest high Q secondary

used with a low Q lossy primary system will perform poorly.

The greatest benefit in using acrylic or polyethylene pipe

for a secondary form is that no preparation is necessary.

PVC is porous and will absorb a great deal of moisture.

Untreated PVC will be much more affected by changes in the weather.

The low loss nature of the other materials might gain  you

a couple extra inches of spark. Not enough to warrant paying

a premium price for them. However if you can get them cheap

by all means use them.

>You mention 'leakage resistance' but I'm not sure what this means.

At the multi hundred kilovolt voltages we are producing even some

seemingly good insulators will conduct slightly. This small current

could be termed "leakage". Im pretty sure Bert meant that a suitable

secondary form should be capable of withstanding these kinds of voltages

without "leaking".

> Would space-winding (i.e. leaving a gap between each turn ) be

>worth while for the secondary, to increase insulation, or would that

>have other penalities?

It would increase turn to turn insulation but at the heavy penalty

of greatly reduced inductance. Space winding is not good.

By winding your secondary with 700-1000 turns you are spreading

the voltage across a sufficient number of turns that the voltage

between adjacent turns should be quite low.

TTYL!

Mike Hammer

mhammer@misslink.net

From: <Dsurfr@aol.com>

Date: Mon, 27 Jul 1998 19:27:50 EDT

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Cap calculations

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Hey All,

I need some advice on a cap I'm going to make. I plan on using polypropylene

page protectors I can get from a local office supply store pretty cheaply.

They are 8 1/2" x 11" and each "leaf" is .0035" thick. I'll use heavy duty

aluminum foil to make (10) 7"x 8 1/2" plates with three pieces of the poly

between them. I'll put a piece of 1/8" plexi 9" x 11" (to allow for the "tab"

on the side of the sheet protector - which I'll use to keep everything lined

up ala Brandon Shaw's design) to form a top & bottom "cover" & bind them with

a piece of plastic strapping tape.

Rating per cap:

Dielectric Constant = 2

Area per plate = 60 sq. in.

Separation between plates = .0105"

Rating per cap = .0256

If I connect (4) of these in series I should end up with a rating of .0064

Put these in a plastic tub, use brass bolts for connections & fill with mineral

oil. Seal the top with hot glue or epoxy.

Is my math right? Will this be OK for a 15K 30mA neon? Does the construction

process seem sound?     Thanks to all for any replies.  Jim

X-Mailer: IPAD 2.0

Date: Mon, 27 Jul 1998 21:41:24 -0500

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: Cap calculations

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

At 07:27 PM 7/27/98 EDT, you wrote:

>Hey All,

>I need some advice on a cap I'm going to make. I plan on using polypropelene

>page protectors I can get from a local office supply store pretty cheaply.

>They are 8 1/2" x 11" and each "leaf" is .0035" thick. I'll use heavy duty

>aluminum foil to make (10) 7"x 8 1/2" plates with three pieces of the poly

>between them. I'll put a piece of 1/8" plexi 9" x 11" (to allow for the "tab"

>on the side of the sheet protector - which I'll use to keep everything lined

>up ala Brandon Shaw's design) to form a top & bottom "cover" & bind them with

>a piece of plastic strapping tape.

>

>Rating per cap:

>

>Dielectric Constant = 2

>Area per plate = 60 sq. in.

>Separation between plates = .0105"

>Rating per cap = .0256

>If I connect (4) of these in series I should end up with a rating of .0064 Put

>these in a plastic tub, use brass bolts for connections & fill with mineral

>oil. Seal the top with hot glue or epoxy.

>Is my math right? Will this be OK for a 15K 30mA neon? Does the construction

>process seem sound?     Thanks to all for any replies.

>  Jim

Jim,

Everything looks good to me except your total dielectric thickness.

It appears that you have  3 X 3.5 mils or 10.5 mils per cap.

4 of these in series would yield only a total of 42 mils of

dielectric. That just wont make it at 15kv. You need twice that

much poly. Go to six series caps or double the thickness in each

of the three caps.

One note about hot glue. I found that hot glue tends to debond

from whatever its stuck to once it gets into the mineral oil.

TTYL!

Mike Hammer

mhammer@misslink.net

Date: Tue, 28 Jul 1998 23:07:24 +0100

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Stephen Bell <steve@breakfix.demon.co.uk>

Subject: Tank circuit capacitor

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Hi Guys,

I told myself I would read up on everything I could, about

building a Tesla coil, before buying or building so much as an ON/OFF

switch, but what can I say!  I've probably wasted some money already on

bypass caps (now I know homemade ones are possible) but just in the nick

of time, I now know what's what when it comes to choosing secondary coil

former material (thanks to Bert and MC Hammer - that's MC as in Megavolt

Coils ;-).

I just got my copy of 'Colorado Spring Notes, 1899 - 1900 (UK

Ed.)' this morning, from 'bookshop.co.uk'. I'm still waiting on

'Inventions, Researches and Writings of Nikola Tesla (UK Ed.)'.

Back to the question at hand, i.e. the tank cap, I believe a

value of 0.019uF would match my 10kV/100mA NST.  I'm going to make a

unit consisting of 3 identical stacked plate capacitors, wired in

series.

Each cap will have a value of 0.0564uF and consist of 12"x12" foil

plates with 0.048" total poly thickness (using 6mil sheet), total number

of plates is estimated at about 40.

Q:  Are the above figures about right?

Q:  Would this cap have a good enough voltage rating?

Q:  Would there be any performance increase in building TWO of the

above, but with HALF the capacitance, and wiring them in parallel?

TIA.

P.S.

I've just started up a web site, to organise and document my

Tesla coil project.  Please take a look and let me know if I've made any

stupid mistakes! :-/

I would like comments on the suitability of my bypass capacitors, if

anyone recognises them.

www.breakfix.demon.co.uk

-------------------------------------------

Stephen Bell   (steve@breakfix.demon.co.uk)

-------------------------------------------

X-Mailer: IPAD 2.0

Date: Tue, 28 Jul 1998 22:22:06 -0100

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Jim Ryan <jryan@exis.net>

Subject: Poly material

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Hello again, just wanted to give you an up date on building my coil.

I'm the guy that is using .005 brass shim stock in my poly caps.

After buying poly at the hardware store that was wrinkled, I found some

stuff that is used for temporary AC ducting that is 18" wide flat 2 layers

of .005 poly that can be cut down the middle and make perfect 9" wide

strips for my 6" wide shim stock. This stuff is made so that when it is

pressurized it makes a tube, but when in a roll is a perfectly smooth

poly "NOT THE FIRST WRINKLE" I got it at work and will get you a manufacture

if anyone is interested.

Jim

From: <Dsurfr@aol.com>

Date: Tue, 28 Jul 1998 19:42:10 EDT

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Transformer size

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Mike: As always thanks for the quick & accurate response to my last query! I

read your old posting (mikecap.txt) once again & I'm now wondering if I should

be using my 12kV 30 mA neon instead of my 15kV 30mA, since I hope to buy

another one in the near future to increase my amperage. It seems like I could

keep my construction expenses down by going the 12kV route (& you obviously

are getting great results) but I thought that the 15 kV would give much better

results?(MORE POWER) It seems like the poly caps for a 12kV alone would cost

about half as much to make which would cover the cost of the extra

transformer.

TIA for the advice, just trying to find the bigger "bang" for my buck.

Jim

X-Mailer: IPAD 2.0

Date: Tue, 28 Jul 1998 21:26:33 -0500

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: Transformer size

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

At 07:42 PM 7/28/98 EDT, you wrote:

>Mike: As always thanks for the quick & accurate response to my last querry!

My pleasure. Glad I could help.

>I read your old posting (mikecap.txt) once again & I'm now wondering if

>I should be using my 12kV 30 mA neon instead of my 15kV 30mA, since I hope

>to buy another one in the near future to increase my amperage. It seems

>like I could keep my construction expenses down by going the 12kV route

>(& you obviously are getting great results) but I thought that the 15 kV

> would give much better results?(MORE POWER)

I started with 12kv neons because thats all I could find at first.

When I got a pair of 15kv/30s I found they didnt really provide

any real improvement in output and they blew up my cap.

Ive settled on 12kv now and have never been at a loss for all the

power I needed.

>It seems like the poly caps for a 12kV alone would cost

>about half as much to make which would cover the cost of

>the extra transformer.

One problem with poly caps in oil is that corona can form

along the edges and ends of plates at voltages above 12-13KV

EVEN UNDER OIL. So a single cap is doomed to failure eventually

if run at 15kv. The answer of course is to run several caps in series

to keep the voltage on each individual cap low. Just a lot easier

to accomplish at 12kv than at 15kv.

>TIA for the advice, just trying to find the bigger "bang" for my buck.

>Jim

Arent we all?  :)

TTYL!

Mike Hammer

mhammer@misslink.net

X-Mailer: IPAD 2.0

Date: Tue, 28 Jul 1998 21:33:21 -0500

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: Poly material

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

At 10:22 PM 7/28/98 -0100, you wrote:

>Hello again, just wanted to give you an up date on building my coil.

>

>I'm the guy that is using .005 brass shim stock in my poly caps.

>

>After buying poly at the hardware store that was wrinkled, I found some

>stuff that is used for temporary AC ducting that is 18" wide flat 2 layers

>of .005 poly that can be cut down the middle and make perfect 9" wide

>strips for my 6" wide shim stock. This stuff is made so that when it is

>pressurized it makes a tube, but when in a roll is a perfectly smooth

>poly "NOT THE FIRST WRINKLE" I got it at work and will get you a manufacture

>if anyone is interested.

>

>Jim

Jim,

Thats sounds great. Just be sure that it is actually polyethylene.

A few wrinkles dont really matter. The last roll of poly I bought

had a bunch of wrinkles and I just simply ironed them out with

a small iron on a low heat setting.

TTYL!

Mike Hammer

mhammer@misslink.net

Date: Tue, 28 Jul 1998 22:37:14 -0400

From: Chris Watkins <cwatkins@seidata.com>

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: rotary specs

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

hi guys,

I have been considering building a rotary unit

to go in series with my rq gap.

I need some general guidelines to start from.

here are some of my questions.

what type material works best for the "wheel" ?

how big should the wheel be?

what should I use for a minimum electrode spacing?

how big should the electrodes be?

will copper tungsten make good electrodes?

what about tungesten carbide? (I have some of each).

I am hoping to use a DC motor with a variable speed

controller. any comments?

I have in mind a setup that uses 2 fixed contacts

on each side of a wheel that spins electrode bars

so that both legs of the current are contacted

simultaneously.

thus the "wheel" would have to be non conductive.

any other things I should look out for ?

as always your help/ideas are appreciated.

CHRIS W

Date: Tue, 28 Jul 1998 22:53:27 -0500

From: Tom Polk <tpolk@prismnet.com>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: rotary specs

Sender: tesl-err@emachine.com

Chris Watkins wrote:

> I have in mind a setup that uses 2 fixed contacts

> on each side of a wheel that spins electrode bars

> so that both legs of the current are contacted

> simultaneously.

>

That is like mine. My wheel has operated for years with 21 contacts on

the wheel. Those contacts consist of 8/32 3/4" stainless steel round

head screws. I can change them out cheaply and adjust them at will.

The round screw heads form a ball gap. I experimented putting capnuts on

the end with no noticable improvement. The stationary contacts on each

side of the wheel are 1/4 inch round head stainless screws.

What's the wheel made of? 1/4" model aircraft plywood. Motor is a

universal refrigerator defroster blower motor (120vac) from Graingers

($7.95-15.00). It is probably weaker than lotsa coilers prefer. It takes

a few seconds to spin up to its 1400 rpm,  and about a minute to spin

down. If it's synchronous, it's an accident. It's durable and cheap.

I've gotten 48 inch sparks with one modified NST using this as my only

gap, so it works.

Date: Tue, 28 Jul 1998 21:46:00 -0700

From: Gene Waldo <gwaldo@1stnetusa.com>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Organization: S.A.P.I.E.N.T. (Surly, Agressive, Politically Incorrect Enemies of Non Thinkers)

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: rotary specs

Sender: tesl-err@emachine.com

Chris Watkins wrote:

>

> hi guys,

>

> I have been considering building a rotary unit

> to go in series with my rq gap.

>

> I need some general guidelines to start from.

> here are some of my questions.

SNIPS

> as always your help/ideas are appreciated.

>

> CHRIS W

Chris,

Some months back, I stumbled across a commercial Rotgap unit, of which I

sent 2 or 3 pics and some details to Bert Hickman <bert.hickman@aquila.com >.

He never indicated to me one way or the other on the unit, but may have some impressions....

If anyone is interested further, I can send pics; unit seems in good cond,

with perhaps TungCarb(?) electrodes; 1/4 HP GE motor, 1800 rpm; cost out of the

place was $35, if still there. Actually they had 2 of them; one looked

to be an eariler version by the same mfr. Upon showing pics to one

individual, he said he thought it was out of a radar unit.

------------------------------------

Gene Waldo -  gwaldo@1stnetusa.com  - "Igor in Residence"

---------------------------------------------------------------

What's good for the planet is more OZONE! See the machinery form up!

NEW PAGE!   http://members.1stnetusa.com/danmurphy/waldo.htm

X-Mailer: IPAD 2.0

Date: Wed, 29 Jul 1998 00:15:01 -0500

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: Tank circuit capacitor

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

At 11:07 PM 7/28/98 +0100, you wrote:

>Hi Guys,

>        I told myself I would read up on everything I could, about

>building a Tesla coil, before buying or building so much as an ON/OFF

>switch, but what can I say!  I,ve probably wasted some money already on

>bypass caps (now I know homemade ones are possible) but just in the nick

>of time, I now know what's what when it comes to choosing secondary coil

>former material (thanks to Bert and MC Hammer - that's MC as in Megavolt

>Coils ;-).

Bypass caps are OK if small but larger ones can cause problems

with resonances with the neon secondary. Make those bypasses

something around 150 pf each. Certainly no more than 250 pf.

>

>        I just got my copy of 'Colorado Spring Notes, 1899 - 1900 (UK

>Ed.)' this morning, from 'bookshop.co.uk'. I'm still waiting on

>'Inventions, Researches and Writings of Nikola Tesla (UK Ed.)'.

Good books. The Colorado Springs Notes  (CSN) is pretty heavy

stuff and not too understandable at times. I heartily recommend

you get Richard Hull's companion to the CSN. Its titled

"The Tesla Coil Builders Guide to The Colorado Springs Notes"

>

>        Back to the question at hand, i.e. the tank cap, I believe a

>value of 0.019uF would match my 10kV/100mA NST.  I'm going to make a

>unit consisting of 3 identical stacked plate capacitors, wired in

>series.

I make the matching cap for a 10kv/100ma running at 50hz

to be .0318 mfd.  Thats a lot of capacitance and would require

a pretty large coil. You DONT need this much capacitance.

Its more important that your selected cap allows you to reach resonance.

>

>Each cap will have a value of 0.0564uF and consist of 12"x12" foil

>plates with 0.048" total poly thickness (using 6mil sheet), total number

>of plates is estimated at about 40.

>

>Q:  Are the above figures about right?

Pretty much. I come up with .0524 mfd assuming a k value for

the poly of 2.0

>

>Q:  Would this cap have a good enough voltage rating?

3 caps each with 48 mils of poly in series? Overkill for

10kv. Just two of those caps in series would be nearly bullet proof.

>

>Q:  Would there be any performance increase in building TWO of the

>above, but with HALF the capacitance, and wiring them in parallel?

No.

>

>

>P.S.

>        I've just started up a web site, to organise and document my

>Tesla coil project.  Please take a look and let me know if I've made any

>stupid mistakes! :-/

>I would like comments on the suitability of my bypass capacitors, if

>anyone recognises them.

>

>www.breakfix.demon.co.uk

>-------------------------------------------

>Stephen Bell   (steve@breakfix.demon.co.uk)

>-------------------------------------------

Page looks good Stephen. It will be great when you

get some pics of your coil in action. :)

TTYL!

Mike Hammer

mhammer@misslink.net

X-Mailer: IPAD 2.0

Date: Wed, 29 Jul 1998 08:37:02 +0300

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Marco Denicolai <marco@vistacom.fi>

Subject: What Tesla Coil book?

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

After building my first TC, I feel I eventually need to read one or more

books.

I am seeking for:

1. something about Tesla's experiments

2. a serious explanation of the TC and magnifier working principles

3. project examples and suggestions

Please, this doesn't want to be commercial but in the galaxy of Tesla

related books it is hard for me to pick up one or two. More, I have to

order them from the US, so I really don't like to pick up an unnecessary

book.

I was thinking about "Colorado Springs Notes 1899-1900" (by Nikola Tesla)

and "The Tesla Coil Builder's Guide to the Colorado Springs Notes of Nikola

Tesla" (by Richard L.Hull).

Can you experienced guys out there suggest me some good titles?

Marco Denicolai

X-Mailer: IPAD 2.0

Date: Wed, 29 Jul 1998 12:20:24 -0500

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: rotary specs

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

At 10:37 PM 7/28/98 -0400, you wrote:

>hi guys,

>

>I have been considering building a rotary unit

>to go in series with my rq gap.

C>

>I need some general guidelines to start from.

>here are some of my questions.

>

>what type material works best for the "wheel" ?

>

>how big should the wheel be?

>

>what should I use for a minimum electrode spacing?

>

>how big should the trodes be?

>

>will copper tungsten make good electrodes?

>what about tungesten carbide? (I have some of each).

>

>I am hoping to use a DC motor with a variable speed

>controller. any comments?

>

>I have in mind a setup that uses 2 fixed contacts

>on each side of a wheel that spins electrode bars

>so that both legs of the current are contacted

>simultaneously.

>

>thus the "wheel" would have to be non conductive.

>

>any other things I should look out for ?

>

>as always your help/ideas are appreciated.

>

>CHRIS W

Chris and others,

If you are using neons for your power supply you

do NOT want to build a rotary gap. Rotaries can

really beat up a neon and cause rapid failure.

Unlike a static gap that has a set predetermined

breakdown voltage that doesnt vary a rotary gaps

breakdown voltage is constantly varying. From a few KV

to hundreds of KV. There will be times at the peak of

the charging cycle when the gap is not in position to fire.

This will allow the voltage rise greatly especially if

your cap is well matched to your neon.

A synchronous gap using a 1800 or 3600 RPM synchronous

motor can be used with neons but non synchronous rotary

gaps are too hard on neons.

Besides neons work much better on well built static gap.

So there is no reason to mess with a rotary for neon

powered coils.

TTYL!

Mike Hammer

mhammer@misslink.net

X-Mailer: IPAD 2.0

Date: Wed, 29 Jul 1998 12:34:01 -0500

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: What Tesla Coil book?

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

At 08:37 AM 7/29/98 +0300, you wrote:

>After building my first TC, I feel I eventually need to read one or more

>books.

>

>I am seeking for:

>1. something about Tesla's experiments

>2. a serious explanation of the TC and magnifier working principles

>3. project examples and suggestions

>

>Please, this doesn't want to be commercial but in the galaxy of Tesla

>related books it is hard for me to pick up one or two. More, I have to

>order them from the US, so I really don't like to pick up an unnecessary

>book.

>

>I was thinking about "Colorado Springs Notes 1899-1900" (by Nikola Tesla)

>and "The Tesla Coil Builder's Guide to the Colorado Springs Notes of Nikola

>Tesla" (by Richard L.Hull).

>

>Can you experienced guys out there suggest me some good titles?

Marco,

In my opinion most of the Tesla coil books out there

arent really worth the price. None of them contain any

information that isnt freely available on the Internet.

The CSN and Richard Hull's companion guide to the CSN

are good reads but these are not coiling books. They are more

for historical purposes. They detail the day to day experiments

that Tesla performed in his Colorado Springs lab.

There is very little information to help you build coils.

One book I have found to be of interest is by Brent Turner.

Info on his book is on his web page at :

http://www.apc.net/bturner/hv-page.htm#the book

TTYL!

Mike Hammer

mhammer@misslink.net

From: Tom Stock <tomstock@geocities.com>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: My Web Page

Date: Wed, 29 Jul 1998 11:19:43 -0700

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

Hi Everyone...I just updated my web page, and there is a little bit of info

about my coil on there if you feel like taking a look.

You'll see a good example of what happens when you go from 30ma to 60ma =)

-tom

http://www.geocities.com/CapeCanaveral/1348

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: rotary specs

Date: Wed, 29 Jul 1998 20:15:43 +0000

From: Rune Landen <rune@dircon.co.uk>

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

>If you are using neons for your power supply you

>do NOT want to build a rotary gap. Rotaries can

>really beat up a neon and cause rapid failure.

>Unlike a static gap that has a set predetermined

>breakdown voltage that doesnt vary a rotary gaps

>breakdown voltage is constantly varying. From a few KV

>to hundreds of KV. There will be times at the peak of

>the charging cycle when the gap is not in position to fire.

>This will allow the voltage rise greatly especially if

>your cap is well matched to your neon.

Couldnt you just combine the two? One for allowing extra voltage to spark if it

get too high, and the other to get a low resistance path to the tesla coil,

when it is in position.

X-Mailer: IPAD 2.0

Date: Wed, 29 Jul 1998 15:05:19 -0500

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: rotary specs

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

At 08:15 PM 7/29/98 +0000, you wrote:

>>If you are using neons for your power supply you

>>do NOT want to build a rotary gap. Rotaries can

>>really beat up a neon and cause rapid failure.

>

>>Unlike a static gap that has a set predetermined

>>breakdown voltage that doesnt vary a rotary gaps

>>breakdown voltage is constantly varying. From a few KV

>>to hundreds of KV. There will be times at the peak of

>>the charging cycle when the gap is not in position to fire.

>>This will allow the voltage rise greatly especially if

>>your cap is well matched to your neon.

>

>Couldnt you just combine the two? One for allowing extra voltage

>to spark if it get too high, and the other to get a low resistance

>path to the tesla coil, when it is in position.

Sure. You could put a static gap in parallel with the rotary

and you could set the spacing so it fires when the rotary is out

of position on voltage peaks. But guess what would happen if

you remove the rotary and leave just the static gap? The coil

would operate just as well. Perhaps better if your static gap is

of good design.

The whole idea of using a rotary is to force a break and

provide better quenching. A good static gap can provide just

as good quenching as a rotary. The only time you need a rotary

gap is at higher power levels above 2-3 KVA. Even then a good

static gap can still provide good performance at much less

cost and complexity.

Neons just do not do well on rotary gaps. Many beginners

look at the rotary as the ultimate gap and a sure road

to better performance. Just not true. If your coil performs

poorly on a good static gap then it will still be poor on

a rotary.

What constitutes a good static gap? The cylinder gap popularized

by Richard Quick is a good performer. It consists of pieces

of hard copper pipe mounted side by side radially inside a piece

of PVC pipe. The PVC pipe serves as a housing. Cooling airflow

is supplied by a strong fan.

This same gap using airflow supplied by a strong vacuum cleaner

is even better. This is the type of gap I use. I get optimal

quenching at power levels approaching 1500 watts. A rotary

would do no better.

Gary Weaver also uses a vacuum type cylinder gap patterned after

mine. Gary has used this gap at power levels up to 9600 watts

on his ten inch coil and had streamers in excess of 120 inches.

No rotary gap needed. Check out Gary's web page at:

http://home.earthlink.net/~gweaver/

Terry Fritz has done much experimenting with simple gaps

and has a simple static gap design that provides great

quenching. Check out Terry's page for some excellent

articles and experiments. URL is: http://www.peakpeak.com/~terryf/tesla/main.html

Also checkout the picture of Terry's gap design at:

http://www.peakpeak.com/~terryf/tesla/gap.jpg

TTYL!

Mike Hammer

mhammer@misslink.net

Date: Wed, 29 Jul 1998 15:44:28 -0700 (PDT)

From: John <froboy83@yahoo.com>

Subject: Ions

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

Does anyone know how to make a negative ion generator?

John

Date: Wed, 29 Jul 1998 21:10:39 -0700

From: Michael Loftus <loftusm@ix.netcom.com>

Organization: Me

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Primary

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Mike Hammer wrote:

>

> At 11:50 AM 5/25/98 -0400, you wrote:

> >Hello all, WOW I'm putting together my primary today. I never figured

> >50' of copper tubing could get so twisted. I thought I would open

> >the box and just set in place. Little did I know it comes wound

> >so that both ends are on the large diameters. you need about 6 hands

> >to get it wound or un wound. This project has been a test of all of

> >the skills I have acquired over many years.

> >

> > Jim

>

> Good luck Jim. It can be a pain and sometimes 6 hands arent enough.

> Try to utilize the curvature of the tubing wherever possible.

> Bend it just enough to go where you want it. If you bend it

> too much the copper will work harden and be even harder to shape

> properly.

>

> Dont worry if your primary comes out looking a little ugly.

> An ugly primary has just as much inductance as a pretty one

> and will work just as well. Just dont tell anybody and dont

> take any pictures. No one will be the wiser. :)

>

> TTYL!

>

> Mike Hammer

> mhammer@misslink.netHi!

I know this is an old posting, but I thought I would point-out something.

There are at least two kinds of 1/4" copper tubing; one for water and one

for refrigeration.  The "refer" tubing is much thinner and easier to work

with as it seems "softer" and bends much easier.  I found it at "Home

Base" for $16 for 50 feet. I originally used the "water" tubing and hated

every inch of it!

Mike L.

Date: Wed, 29 Jul 1998 23:42:13 -0500

From: Tom Polk <tpolk@prismnet.com>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Ions

Sender: tesl-err@emachine.com

John wrote:

>

> Does anyone know how to make a negative ion generator?

In general: Use a NE555 IC oscillator feeding a 2N3055 transistor at 12

volts to exite the primary of a car spark coil. Connect the secondary to

a television hi voltage diode (25-35kv, I believe) in reverse with a

tuft of steel wool on the end to help disperse the ions.

I saw plans in one of my old electronics magazines. I'll try to find

them since there were no results at http://www.gernsback.com.

Date: Thu, 30 Jul 1998 18:46:21 -0400

From: Pat & Heather Moser <moser@madbbs.com>

Organization: Planet Cheese Creations

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Spark gap PVC

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

I have toiled at the computer for many months (just ask Mike)

If any you guys have a chunk of 6" PVC lying say six inches in length

that you could part with I would gladly pay shipping.

Give me a email at moser@madbbs.com if you can help me bring my sparkgap

into reality!

Thanks Pat (Electron)

From: <Ashpilot@aol.com>

Date: Thu, 30 Jul 1998 21:34:48 EDT

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: ground

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

>by the way, anyone can help me out with what grounding is used for ordinary

>domestic use? Just all the different ways that grounding is used, in ordinary

>domestic electrical use. And also how grounding the transformer in a tesla

coil

>for protection works. Not the the tesla coil grounding tho, just the

>transformer.

Ordinary household ground is connected through the third (usually bare) copper

wire in the cable that runs throughout the house. The other two being

black(HOT) and white(Neutral) The ground wire goes to the main panel, where a

much heavier wire is run to an external ground, typically a water main.

All fixture boxes are attached to that bare wire, usual color code is

green(the screw head is green)  the ground connection is made to the outlets

also.  Usually appliances are chassis grounded through the outlet, meaning

that any metal exterior/interior frames are connected to ground to provide a

path to ground, thats not through you,  in case of a short in the appliance.

I imagine that transformers may have a similar set up.  In the 10kv xformer

that I have the core and case are connected to ground.   I hope this is what

you wanted to know!

Date: Thu, 30 Jul 1998 19:25:16 -0700

From: Gene Waldo <gwaldo@1stnetusa.com>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Organization: S.A.P.I.E.N.T. (Surly, Agressive, Politically Incorrect Enemies of Non Thinkers)

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Grounding, cant understand it

Sender: tesl-err@emachine.com

Rune Landen wrote:

>

> >Ordinary household ground is connected through the third (usually bare) copper

> >wire in the cable that runs throughout the house. The other two being

> >black(HOT) and white(Neutral) The ground wire goes to the main panel, where a

> >much heavier wire is run to an external ground, typically a water main.

>

> I know that already :)

>

> I want to know how the ground works. Eg what happens in electrical terms when

> any situation occurs that will make current flow through the ground. I cant

> understand the thing! Is it a big capacitor? or what? what is it?

Rune, that ground lead is a direct short to ground, which when (in some

fail mode), bearing sufficient current, will: a)(lacking such a ground),

fry you, or  b)fry open any fuses or ckt brkrs.  We all hope for the

latter! It also helps to dissipate (gnd) some forms of interference.

--------------------

Gene Waldo -  gwaldo@1stnetusa.com  - "Igor in Residence"

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Grounding, cant understand it

Date: Fri, 31 Jul 1998 02:52:51 +0000

From: Rune Landen <rune@dircon.co.uk>

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

>Ordinary household ground is connected through the third (usually bare) copper

>wire in the cable that runs throughout the house. The other two being

>black(HOT) and white(Neutral) The ground wire goes to the main panel, where a

>much heavier wire is run to an external ground, typically a water main.

I know that already :)

>All fixture boxes are attached to that bare wire, usual color code is

>green(the screw head is green)  the ground connection is made to the outlets

>also.  Usually appliances are chassis grounded through the outlet, meaning

>that any metal exterior/interior frames are connected to ground to provide a

>path to ground, thats not through you,  in case of a short in the appliance.

>I imagine that transformers may have a similar set up.  In the 10kv xformer

>that I have the core and case are connected to ground.   I hope this is what

>you wanted to know!

Well, I knew all of that already.

I want to know how the ground works. Eg what happens in electrical terms when

any situation occurs that will make current flow through the ground. I cant

understand the thing! Is it a big capacitor? or what? what is it?

From: "Choad" <choad69@geocities.com>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Rotor Gaps

Date: Thu, 30 Jul 1998 23:46:30 -0500

X-Mailer: IPAD 2.0

X-MimeOLE: Produced By Microsoft MimeOLE V4.72.3110.3

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Hi everyone,

I did some thinking about making a rotor gap for the

MOT transformers that I'm going to use for another coil.

I was thinking that I'd make a sync rotor so that I could

use it on a neon for tests too. Does it just have to break

120 times a sec to be sync? I figured I could make a

gap with a 1200 rmp motor and have 6 electrodes on the

disk. Does the diameter of the disk really matter? For this

first test rotor I plan on just using some bolts on a motor

I have. I don't know the speed of this motor but I plan on

getting a hefty DC motor or a .5 hp AC motor. Anyone

let me know what you think about this. Thanks

X-Mailer: IPAD 2.0

Date: Fri, 31 Jul 1998 08:30:13 +0300

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Marco Denicolai <marco@vistacom.fi>

Subject: Re: Rotor Gaps

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

At 23:46 30/07/98 -0500, you wrote:

>Hi everyone,

>

>    I did some thinking about making a rotor gap for the

>MOT transformers that I'm going to use for another coil.

As you may know, I am in the MOT business too. If you followed

my emails (or go to my tesla page http://vci.vistacom.fi/~marco/mytesla.html)

you know about about my MOT tank supply.

>I was thinging that i'd make a sync rotor so that I could

>use it on a neon for tests too.

I reached the same conclusion: I need a rotary. The MOT current is

so high that puts it somewhere nearer to pole pigs than to NSTs.

>Does it just have to break

>120 times a sec to be sync? I figured I could make a

>gap with a 1200 rmp motor and have 6 electrodes on the disk.

I'm building (trying to build, better said) a rotary with a 12000 rpm

motor and 4 electrodes at first. That will give me 800 breaks/s.

>Does the diameter of the disk really matter?

You can calculate yourself peripheral speed of the disk according

to its diameter. For a 15 cm disk I get 330 Km/h (!). It does

matter. Then there is the quench time, that depends on the bolt size

and the disk speed. For a 6 mm (M6) bolt I get about 140 microseconds

(this is just a "mechanical" quench time, of course).

>For this first test rotor I plan on just using some bolts on a motor

>I have. I don't know the speed of this motor but I plan on getting a

>hefty DC motor or a .5 hp AC motor. Anyone let me know what you think

>about this. Thanks

>

You have better to know the speed of your motor. I measured it

easily with an U-shaped optocoupler, a multimeter with a frequency measure

option and a 5V power supply. You just mount a little disk

with a hole to the motor and that's it.

My motor is a 700W 12000 rpm from a vegetable shredder (Moulinex, good

brand).

If you are running with a break rate high enough, you are no more

concerned about being synchronous. You just use all the halfwaves of

your AC tank and not only one or two particular points of it.

Because my motor is AC I have no speed control: I plan to change the

break rate by just adding/removing bolts.

Any "old guy" from this list correcting some of my possibly wrong

assumptions above?

Marco Denicolai

Date: Thu, 30 Jul 1998 23:38:50 -0700

From: Scott MacGregor <smacgregor@aasp.net>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Organization: EES,Co.

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Grounding, cant understand it

Sender: tesl-err@emachine.com

Rune Landen wrote:

>

> >Ordinary household ground is connected through the third (usually bare) copper

> >wire in the cable that runs throughout the house. The other two being

> >black(HOT) and white(Neutral) The ground wire goes to the main panel, where a

> >much heavier wire is run to an external ground, typically a water main.

>

> I know that already :)

>

> >All fixture boxes are attached to that bare wire, usual color code is

> >green(the screw head is green)  the ground connection is made to the outlets

> >also.  Usually appliances are chassis grounded through the outlet, meaning

> >that any metal exterior/interior frames are connected to ground to provide a

> >path to ground, thats not through you,  in case of a short in the appliance.

> >I imagine that transformers may have a similar set up.  In the 10kv xformer

> >that I have the core and case are connected to ground.   I hope this is what

> >you wanted to know!

>

> Well, I knew all of that already.

>

> I want to know how the ground works. Eg what happens in electrical terms when

> any situation occurs that will make current flow through the ground. I cant

> understand the thing! Is it a big capacitor? or what? what is it?

The "ground" (in a house) is an electrically conductive PATH (the earth)

between all electrical equipment cases, chassis, etc., and the "common"

( or center tap) of the pole transformer that provides your electricity.

This transformer's center tap is also "grounded" (as is your house) to

the earth, with copper ground rods, for lightning protection. From the

transformer's "point of view", large static differences of potential

static charge (potential lightning discharges) should be routed DIRECTLY

to earth. (atmospheric/Earth charges) They are attempting to "isolate "

the power grid from external "static" discharges. (lightning)

Therefore, if a tool motor case (when you touch it) connects

inadvertently to the motor windings, 110 volts may flow THROUGH your

body (wet feet) to the ground. The internal resistance of your body may

be many K-Ohms, but current can be sufficient to cause damage or death.

By grounding (or bonding) all electrical equipment in your house to an

"earth" ground of 10 or 100 Ohms, 99% of the current will flow DIRECTLY

to the pole transformer ground. Fuse blows....you don't!

--

Regards,

Scott MacGregor EES,Co.

************************************************************

Enterprise Engineering Services Co.

E-mail smacgregor@aasp.net

Member: Cable Television Advisory Committee, Plainville, MA

http:/www.expage.com/page/cableguy

************************************************************

X-Mailer: IPAD 2.0

Date: Fri, 31 Jul 1998 12:04:44 -0500

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Mike Hammer <mhammer@misslink.net>

Subject: NST grounding

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

At 09:34 PM 7/30/98 EDT, you wrote:

>

>>Actually I had one, could people look at my grounding question?

<SNIP>

>

>All fixture boxes are attached to that bare wire, usual color code is

>green(the screw head is green)  the ground connection is made to the outlets

>also.  Usually appliances are chassis grounded through the outlet, meaning

>that any metal exterior/interior frames are connected to ground to provide a

>path to ground, thats not through you,  in case of a short in the appliance.

>I imagine that transformers may have a similar set up.  In the 10kv xformer

>that I have the core and case are connected to ground.   I hope this is what

>you wanted to know!

You should not connect your neon transformer to your AC mains

ground. It should instead be connected to your RF ground.

If your neon should fail (and they do) and suffer an internal arc

from the secondary to core this could place high RF voltages

on your house ground. Everything in your house could then be

exposed to this RF. Not a good scenario.

Instead connect the neon case to RF ground where any mishaps

are conducted safely away.

TTYL!

Mike Hammer

mhammer@misslink.net

X-Mailer: IPAD 2.0

Date: Fri, 31 Jul 1998 12:12:18 -0500

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: Rotor Gaps

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

At 11:46 PM 7/30/98 -0500, you wrote:

>Hi everyone,

>

>    I did some thinking about making a rotor gap for the

>MOT transformers that I'm going to use for another coil.

>I was thinging that i'd make a sync rotor so that I could

>you it on a neon for tests too. Does it just have to break

>120 times a sec to be sync? I figured I could make a

>gap with a 1200 rmp motor and have 6 electrodes on the

>disk. Does the diameter of the disk really matter? For this

>forst test rotor I plan on just using some bolts on a motor

>I have. I don't know the speed of this motor but I plan on

>getting a heafy DC motor or a .5 hp AC motor. Anyone

>let me know what you think about this. Thanks

>

Doesnt work this way. Just having 120 breaks per second

does not make a synchronous gap. It must also be run

by a 1800 or 3600 rpm sysnchronous motor. And most

of these motors require modification to lock up at

precisely the required rpm. The motor must also be able to

be rotated relative to the fixed electrodes to adjust the

timing to be in synch with the incoming AC.

A non synchronous gap run with a DC motor or variable speed

AC motor can be used to advantage with large systems using

hefty transformers but results in erratic performance on neon powered

coils. The high pulse rate can also literally beat your capacitors

to death. And the wide variation in breakdown voltages frequently

results in transformer failure.

TTYL!

Mike Hammer

mhammer@misslink.net

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: NST grounding

Date: Fri, 31 Jul 1998 18:25:45 +0000

From: Rune Landen <rune@dircon.co.uk>

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

>If your neon should fail (and they do) and suffer an internal arc

>from the secondary to core this could place high RF voltages

>on your house ground. Everything in your house could then be

>exposed to this RF. Not a good scenario.

I know that too. I wasnt thinking of it. I just need to know how the hell the

thing works! Seeing as grounding is used domestically alot, it would be a good

example of grounding for me.

for example how can a current flow through the ground if there is no complete

circuit? What the hell is happening with the ground.

X-Mailer: IPAD 2.0

Date: Fri, 31 Jul 1998 12:21:29 -0500

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: Rotor Gaps

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

At 08:30 AM 7/31/98 +0300, you wrote:

>At 23:46 30/07/98 -0500, you wrote:

>>Hi everyone,

>>

>>    I did some thinking about making a rotor gap for the

>>MOT transformers that I'm going to use for another coil.

>

>As you may know, I am in the MOT business too. If you followed

>my emails (or go to my tesla page http://vci.vistacom.fi/~marco/mytesla.html)

>you know about about my MOT tank supply.

>

>>I was thinging that i'd make a sync rotor so that I could

>>you it on a neon for tests too.

>

>I reached the same conclusion: I need a rotary. The MOT current is

>so high that puts it somewhere nearer to pole pigs than to NSTs.

>

>

>You have better to know the speed of your motor. I measured it

>easily with an U-shaped optocoupler, a multimeter with a frequency measure

>option and a 5V power supply. You just mount a little disk

>with a hole to the motor and that's it.

>

>My motor is a 700W 12000 rpm from a vegetable shredder (Moulinex, good

>brand).

>

>If you are running with a break rate high enough, you are no more

>concerned about being synchronous. You just use all the halfwaves of

>your AC tank and not only one or two particular points of it.

>

>Because my motor is AC I have no speed control: I plan to change the

>break rate by just adding/removing bolts.

>

>Any "old guy" from this list correcting some of my possibly wrong

>assumptions above?

>

>

>

>

> Marco Denicolai

Marco,

While a rotary may be useful in your case there are a couple

points to consider. First of all extremely high break rates

usually result in poor performance at best and component failure

at worst. The high pulse rates can result in capacitor failure.

I know of 2 coilers that destroyed expensive commercial pulse

caps that way.

12000 rpm is far too high. It will be exceedingly difficult

to properly balance a rotor for such high rpm. Any wobble or

vibration at this kind of speed will result in flying shrapnel.

I believe you mentioned using polycarbonate plastic (Lexan)

for your rotor in a previous post. Lexan is not suitable

for that kind of rpm although it can be used at lower speeds.

I think you would be wise to search for a different lower

speed motor for your gap.

TTYL!

Mike Hammer

mhammer@misslink.net

X-Mailer: IPAD 2.0

Date: Fri, 31 Jul 1998 12:50:47 -0500

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: NST grounding

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

At 06:25 PM 7/31/98 +0000, you wrote:

>>If your neon should fail (and they do) and suffer an internal arc

>>from the secondary to core this could place high RF voltages

>>on your house ground. Everything in your house could then be

>>exposed to this RF. Not a good scenario.

>

>I know that too. I wasnt thinking of it. I just need to know how the hell the

>thing works! Seeing as grounding is used domestically alot, i would be a good

>example of grounding for me.

>

>for example how can a current flow through the ground if there is no complete

>circuit? What the hell is happening with the ground.

There is most certainly a complete circuit. All you

need for current flow is a difference of potential

and a conductor.

Earth (ground) is zero volts. By connecting ground to

a point in a circuit you are trying to force that point to be

zero volts also. If there was some voltage before

attaching the ground there will be a current flow to ground.

Mostly though ground is used simply to provide a reference.

When we say that a point is 120 volts we mean that it is

120 volts with reference to ground.

Take a NST for instance. A NST contains 2 completely independant

secondary windings. The outside lead of each secondary is attached

to the HV outputs of the neon. The inside leads are both tied to

the core thus to each other. The core is also tied to the case.

This means that the case of the neon is connected to the HV

secondary windings. Thats not a problem if the case is grounded

and at zero volts. Lets take a 15kv neon for instance. Each

secondary produces 7500 volts. Between the two outputs is 15kv.

Between each output and the case (core) is 7500 volts.

So ideally you have 7500-0-7500. But lets say the case is not grounded.

Then the case can float up to a higher voltage. Lets say the

case floats up to 100 volts. Then the output may look like

7600-100-7600. Thats still 7500 volts from each output to the

case and 15kv across the two outputs in REFERENCE to the case.

But referenced to ground the case is hot and could give you a shock

if you touched it. This can and does happen. Providing a ground

prevents this kind of problem and provides a safe path in the event

of a failure or mishap.

TTYL!

Mike Hammer

mhammer@misslink.net

X-Mailer: IPAD 2.0

Date: Fri, 31 Jul 1998 13:35:15 -0500

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: NST grounding

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

At 07:06 PM 7/31/98 +0000, you wrote:

<SNIP>

>>Take a NST for instance. A NST contains 2 completely independant

>>secondary windings. The outside lead of each secondary is attached

>>to the HV outputs of the neon. The inside leads are both tied to

>>the core thus to each other. The core is also tied to the case.

>

>Right this more the things I was unclear on.

>

>>This means that the case of the neon is connected to the HV

>>secondary windings. Thats not a problem if the case is grounded

>>and at zero volts. Lets take a 15kv neon for instance. Each

>>secondary produces 7500 volts. Between the two outputs is 15kv.

>>Between each output and the case (core) is 7500 volts.

>>So ideally you have 7500-0-7500. But lets say the case is not grounded.

>>Then the case can float up to a higher voltage. Lets say the

>>case floats up to 100 volts. Then the output may look like

>>7600-100-7600. Thats still 7500 volts from each output to the

>>case and 15kv across the two outputs in REFERENCE to the case.

>>But referenced to ground the case is hot and could give you a shock

>>if you touched it. This can and does happen. Providing a ground

>>prevents this kind of problem and provides a safe path in the event

>>of a failure or mishap.

>

>Could you describe where the current starts and where it ends in a

>situation where a fault occurs. Eg it starts in a power utility somewhere,

>which is connected to the ground or something like that. Does it have some

>kind of allowance for capacitance, eg it will let current "disapear" into it? >Much like static? There is some information I am missing on this and I need to

>work it out.

>

The power company distribution transformer sitting on the pole

in front of your home has a center tapped secondary winding.

It provides 120V from each side to ground and 240V across

the two outputs. (at least in the US) This center tap is connected

to Earth ground. This provides the ground reference for all power

delivered to your home.

Any current going to ground ultimately returns to the transformer

via this ground path.

Perhaps this is what you meant? Your question was so general

that its difficult to answer.

TTYL!

Mike Hammer

mhammer@misslink.net

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: NST grounding

Date: Fri, 31 Jul 1998 19:06:20 +0000

From: Rune Landen <rune@dircon.co.uk>

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

>There is most certainly a complete circuit. All you

>need for current flow is a difference of potential

>and a conductor.

What is the specification of this circuit? The domestic supply is connected to

the earth at the the other side? I know that the earth has electric current

flows through it, so perhaps it could merge along with those.

>Earth (ground) is zero volts. By connecting ground to

>a point in a circuit you are trying to force that point to be

>zero volts also. If there was some voltage before

>attaching the ground there will be a current flow to ground.

Right.

>Mostly though ground is used simply to provide a reference.

>When we say that a point is 120 volts we mean that it is

>120 volts with reference to ground.

These arent the bits I dont understand, its the things that are left out from

normal theory that i cant understand. I know the answer is simple and most

experienced electricians would know it too, but I cant remember any book

telling me about how ground works.

>Take a NST for instance. A NST contains 2 completely independant

>secondary windings. The outside lead of each secondary is attached

>to the HV outputs of the neon. The inside leads are both tied to

>the core thus to each other. The core is also tied to the case.

Right this more the things I was unclear on.

>This means that the case of the neon is connected to the HV

>secondary windings. Thats not a problem if the case is grounded

>and at zero volts. Lets take a 15kv neon for instance. Each

>secondary produces 7500 volts. Between the two outputs is 15kv.

>Between each output and the case (core) is 7500 volts.

>So ideally you have 7500-0-7500. But lets say the case is not grounded.

>Then the case can float up to a higher voltage. Lets say the

>case floats up to 100 volts. Then the output may look like

>7600-100-7600. Thats still 7500 volts from each output to the

>case and 15kv across the two outputs in REFERENCE to the case.

>But referenced to ground the case is hot and could give you a shock

>if you touched it. This can and does happen. Providing a ground

>prevents this kind of problem and provides a safe path in the event

>of a failure or mishap.

Could you describe where the current starts and where it ends in a situation

where a fault occurs. Eg it starts in a power utility somewhere, which is

connected to the ground or something like that. Does it have some kind of

allowance for capacitance, eg it will let current "disappear" into it? Much like

static? There is some information I am missing on this and I need to work it out.

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: NST grounding

Date: Fri, 31 Jul 1998 20:10:41 +0000

From: Rune Landen <rune@dircon.co.uk>

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

>The power company distribution transformer sitting on the pole

>in front of your home has a center tapped secondary winding.

>It provides 120V from each side to ground and 240V across

>the two outputs. (at least in the US) This center tap is connected

>to Earth ground. This provides the ground reference for all power

>delivered to your home.

>Any current going to ground ultimately returns to the transformer

>via this ground path.

>Perhaps this is what you meant? Your question was so general

>that its difficult to answer.

Thats more along the line. yes.

What circuit is used? Is a persons house ground and the distribution ground

both linked by a wire, and then both to the earth? Even then I cant quite see

how if an imbalance occurs a current will flow, unless the powerplant is

grounded too.

X-Mailer: IPAD 2.0

Date: Fri, 31 Jul 1998 14:41:32 -0500

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: NST grounding

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

At 08:10 PM 7/31/98 +0000, you wrote:

>>The power company distribution transformer sitting on the pole

>>in front of your home has a center tapped secondary winding.

>>It provides 120V from each side to ground and 240V across

>>the two outputs. (at least in the US) This center tap is connected

>>to Earth ground. This provides the ground reference for all power

>>delivered to your home.

>>Any current going to ground ultimately returns to the transformer

>>via this ground path.

>

>>Perhaps this is what you meant? Your question was so general

>>that its difficult to answer.

>

>Thats more along the line. yes.

>What circuit is used? Is a persons house ground and the distribution ground

>both linked by a wire, and then both to the earth? Even then I cant quite see

>how if an imbalance occurs a current will flow, unless the powerplant is

>grounded too.

>

>

No. The house ground and distribution ground are linked by Earth.

Thats just as good as being linked by wire.

Current will flow any time there is a difference in potential.

The current will flow from the higher potential to the lower one.

Consider this. Next time you scrape your shoes on the carpet in the

winter and a spark jumps from your finger to a doorknob where is

the circuit? The circuit is from your finger (higher potential)

to ground (lower potential).

TTYL!

Mike Hammer

mhammer@misslink.net

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: NST grounding

Date: Fri, 31 Jul 1998 21:02:22 +0000

From: Rune Landen <rune@dircon.co.uk>

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

>Consider this. Next time you scrape your shoes on the carpet in the

>winter and a spark jumps from your finger to a doorknob where is

>the circuit? The circuit is from your finger (higher potential)

>to ground (lower potential).

Oh I dont know. Anyway thats more of a circular circuit. I dont know. I might

as well go back to what I needed this information for which was grounding my

transformer before I turn on my tesla coil. I dont understand how the standard

grounding protects the transformer, but, well whatever. Also I need to

understand it because my transformer is a non standard one, I've been posting

to USAtesla too, and what i said was that it has the standard 2 in, and 2 out

cords. Plus a little wire that sticks out near the case, and has a small metal

piece attached to the wire with a metal that seems to be corroding my copper

wire (zinc maybe), and that fixes to the case. I tested the resistance, and the

resistance from the hv leads to each other are 30 times less than from the hv

leads to the small wire. I dont really need to know what it is if it is too

difficult to work out without opening it, but I just want this thing to be

safely grounded. If I dont get any satisfacteory information I will have to

look into it myself. Happened last time that I got things done as soon as I

gave up on the internet, and resorted to local papers instead :(

From: <Ashpilot@aol.com>

Date: Fri, 31 Jul 1998 22:33:56 EDT

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Making a Coil, requesting recomendations/advice.

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

I've been experimenting with a small coil that has a few flaws, but I am

wanting to build a bigger unit.  This is what I got to work with...

A 10KV/23ma transformer from a furnace ign.

a ~6600 pf poly cap that I've built.

Primary material will be .25" copper refer tubing...

Secondary diam will be 3.25" and about 16.25" (5:1)?  wound with #26 AWG

(about 950 turns)

What I'd like to know is:  Whats the best Primary shape for this power?  A

flat one? or one at an angle?  I've been experimenting with one thats at ~30'

but don't know how to determine the best shape.

I want to use a safety gap and RF Chokes, how can I determine effective size?

I also would like a little more detail on the RQ spark gap, specifically how

to make one that would run the coil above?

Finally, I wanted to be able to measure the frequency of the coil, a friend in

electronic shop suggested a mag pickup connected to a frequency meter? any one

else try this or know a better way?

Thanks!!

Ed M.

Date: Sat, 01 Aug 1998 08:51:23 -0500

From: Mark Olson <molson1@isd.net>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

X-Accept-Language: en

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Rotor Gaps

Sender: tesl-err@emachine.com

Marco Denicolai wrote:

-snip-

> My motor is a 700W 12000 rpm from a vegetable shredder (Moulinex, good

> brand).

>

> If you are running with a break rate high enough, you are no more

> concerned about being synchronous. You just use all the halfwaves of

> your AC tank and not only one or two particular points of it.

>

> Because my motor is AC I have no speed control: I plan to change the

> break rate by just adding/removing bolts.

This is a universal motor and will vary its speed depending on the load and the

voltage applied to it.  If you put a variac on it you will find that it will

adjust the

speed quite nicely...

Mark Olson

Date: Sat, 01 Aug 1998 12:33:03 -0400

From: Pat & Heather Moser <moser@madbbs.com>

Organization: Planet Cheese Creations

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Toroid Reinforcement

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

I got the aluminum duct stuff last night 4" $5.76 at WalMart!

After forming it into a circle I could see that any toroid that is

heading for the floor it's done for to stuff is very flimsy.  I have

sculpted a plug from 2" insulation foam to join the 2 ends together that

worked very well with a lttle work I have a seam that needs no tape! I

got to thinking could you inject it with spray foam (polyisocyanerite)

to crush proof the toroid I want to inject it into several holes in the

inside center of the toroid that would be hidden by the pie pans in

addition soaping to toroid would prevent the overflow from sticking.

Any opinions would be appreciated. Will this alter the tordiod's

electrical characteristics?

Thanks Pat Moser (Electron)

X-Mailer: IPAD 2.0

Date: Sat, 01 Aug 1998 11:42:49 -0500

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: NST grounding

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

At 09:02 PM 7/31/98 +0000, you wrote:

>>Consider this. Next time you scrape your shoes on the carpet in the

>>winter and a spark jumps from your finger to a doorknob where is

>>the circuit? The circuit is from your finger (higher potential)

>>to ground (lower potential).

>

>Oh I dont know. Anyway thats more of a circular circuit. I dont know. I might

>as well go back to what I needed this information for which was grounding my

>transformer before I turn on my tesla coil. I dont understand how the standard

>grounding protects the transformer, but, well whatever.

Grounding a neon is not so much to protect the neon as it is to

protect YOU and other devices connected to the same 120V circuit

in your home. The neon will work just fine without the core/case

being grounded. I have done it many times. Its just much safer

all around with the case/core grounded.

>Also I need to understand it because my transformer is a non standard one,

>I've been posting to USAtesla too, and what i said was that it has the

>stanard 2 in, and 2 out cords. Plus a little wire that sticks out near the

>case, and has a small metal piece attached to the wire with a metal that

>seems to be corroding my copper wire (zinc maybe), and that fixes to the case.

I would bet that the small wire is connected to the core of your neon.

TTYL!

Mike Hammer

mhammer@misslink.net

X-Mailer: IPAD 2.0

Date: Sat, 01 Aug 1998 11:56:24 -0500

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: Making a Coil, requesting recomendations/advice.

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

At 10:33 PM 7/31/98 EDT, you wrote:

>I've been experimenting with a small coil that has a few flaws, but I am

>wanting to build a bigger unit.  This is what I got to work with...

>A 10KV/23ma transformer from a furnace ign.

>a ~6600 pf poly cap that I've built.

>

>Primary material will be .25" copper refer tubing...

>

>Secondary diam will be 3.25" and about 16.25" (5:1)?

>wound with #26 AWG (about 950 turns)

All sounds good to me Ed.

>

>

>What I'd like to know is:  Whats the best Primary shape for this power?  A

>flat one? or one at an angle?  I've been experimenting with one thats at ~30'

>but don't know how to determine the best shape.

With small coils and low power level a flat spiral may not provide

enough coupling. A regular solenoid type primary works well

with small coils provided you provide a means for raising the secondary

up away from the primary to reduce the coupling. This type of

primary should be more than twice the diameter of the secondary.

7 inches diameter would be good for your secondary size.

Also good would be a conical primary with about a 30 degree

slope. There is nothing special about the angle. The flatter

you make it the less coupling it will provide.

For a conical you need about 1 inch clearance from the secondary

to the inner primary turn. For your 3.25 inch secondary that

means a 5.25 inch inner turn diameter.

>

>I want to use a safety gap and RF Chokes, how can I determine effective size?

Safety gap is very simple. Just a simple static gap placed across

the HV outputs and spaced just slightly wider than your main gap

is fine for a small coil.

RF chokes need to be at least 2mh to be effective.

>

>I also would like a little more detail on the RQ spark gap, specifically how

>to make one that would run the coil above?

Perhaps a simpler design for your small coil would be a number of pieces

of 1/2 or 3/4 inch copper pipe mounted side by side on a piece of

heavy plastic. Use 9 electrodes (8 gaps) each spaced .025 inch

for a total of .200 inch. That should be fine for your coil.

Blow some air on the electrodes during operation to keep them cool.

>

>

>Finally, I wanted to be able to measure the frequency of the coil, a friend in

>electronic shop suggested a mag pickup connected to a frequency meter? any one

>else try this or know a better way?

For this you need some test equipment.

If you have a signal generator you can base feed the

secondary from the generator through a pair of LEDS.

The LEDs will light brightest at the secondary resonant

frequency.

TTYL!

Mike Hammer

mhammer@misslink.net

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: NST grounding

Date: Sat, 1 Aug 1998 18:14:32 +0000

From: Rune Landen <rune@dircon.co.uk>

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Mike Hammer said in Re: NST grounding at 01/Aug/1998, Sat 16:42:49.

>I would bet that the small wire is connected to the core of your neon.

I came to that conclusion yesterday too :)

It was just the guy who sold it to me that told me that it wasnt.

And he had it longer than me and had years of experience in university

while I havent :)

Date: Sat, 01 Aug 1998 16:59:00 -0700

From: Gene Waldo <gwaldo@1stnetusa.com>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Organization: S.A.P.I.E.N.T. (Surly, Agressive, Politically Incorrect Enemies of Non Thinkers)

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Making a Coil, requesting recomendations/advice.

Sender: tesl-err@emachine.com

Mike Hammer wrote:

>

> At 10:33 PM 7/31/98 EDT, you wrote:

SNIPS

> >

> >Finally, I wanted to be able to measure the frequency of the coil, a friend in

> >electronic shop suggested a mag pickup connected to a frequency meter? any one

> >else try this or know a better way?

>

> For this you need some test equipment.

> If you have a signal generator you can base feed the

> secondary from the generator through a pair of LEDS.

> The LEDs will light brightest at the secondary resonant

> frequency.

>

> TTYL!

>

> Mike Hammer

> mhammer@misslink.net

Mike,

like this?

LED type? different pol?

o-->|---------------)

)

S/G  Sine, pk, pk-pk V out?   )  SEC

)

o--|<---------------)

Which brings to mind another of my dumb questions: Is Sine optimal? What

happens if a TC could be driven on Sq, Ramped or Triang wave?  What

might such supplies look like?

S/G driving a great amplifier? Or could a power supply be built to that end?

My guess is such would be much harder on the caps.....

Would controlling the tank input freq have any advantages?

---------------------------------

Gene Waldo -  gwaldo@1stnetusa.com  - "Igor in Residence"

Date: Sat, 01 Aug 1998 21:38:58 -0400

From: Pat & Heather Moser <moser@madbbs.com>

Organization: Planet Cheese Creations

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Toroid Renforcement

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Very good ideal sir

I wonder what the electrical characteristics of high-temp aluminium

engine paint is? It would be interesting to get a can of it then spray

a test and measure the conductivity. It's a shame to put so much work

into creating the perfect surface to cover it with tape. Might be worth

a try considering the labor involved in coving it in all that tape

Pat

From: "crowekri" <crowekri@scott.net>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: wire dia.???

Date: Sat, 1 Aug 1998 21:27:02 -0500

X-Mailer: IPAD 2.0

X-MimeOLE: Produced By Microsoft MimeOLE V4.72.3110.3

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Hi all, I have been a subscriber for several months and always enjoy reading

about everyone and their coiling adventures and have decided to attempt the

construction of my first Tesla coil.

I am still in the planning stages and was wondering about the importance

of wire diameter,  most sec. coils that I have seen appear to be wound with

small gauge wire (24-22 awg).  Is this small dia. wire neccesary? If so why?

Or is the smaller gauge used simply to pack more turns per inch thereby

reducing the overall length of the sec?

Also I plan on building a small size coil first then as I learn more I

would like to build something larger(6"-8" sec.)  I have (2) 12Kv 30ma

nst's which are practically new.  If I use one on say a 3" coil  would it be

possible to later use the both on a 6" coil?  Could I use basically the same

primary circuit(altering my pri. coil of course)  What about safety and

sparkgap adjustments?  Anyway any feedback is useful and appreciated.

Thanks,

crowekri

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: bkibler@quicktel.com (Robert Kibler)

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: PRIMARY WIRE

Date: Sat, 1 Aug 1998 20:16:18 -0700

X-Mailer: IPAD 2.0

X-MimeOLE: Produced By Microsoft MimeOLE V4.72.2106.4

Sender: tesl-err@emachine.com

Does the size of the primary wire really make that much difference?

X-Originating-IP: [208.16.145.181]

From: "Bill the arcstarter" <arcstarter@hotmail.com>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Making a Coil, requesting recomendations/advice.

Date: Sat, 01 Aug 1998 21:05:40 PDT

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

Gene wrote:

>Which brings to mind another of my dumb questions: Is Sine optimal?

>What happens if a TC could be driven on Sq, Ramped or Triang wave?

>What might such supplies look like?

Last time I had the 'scope out measuring the secondary resonant

frequency, I discovered that I could use the sine, square, triangle

outputs with equal success.  That is - using either waveform gave me a

dip, at an identical frequency.  However, using sin gave the sharpest

dip.  You should be able to use squarewave with good luck.

Of course, you still have to have a freq counter to measure the freq of

the source...

-Bill the arcstarter

Starting arcs in Cinci, OH

http://www.geocities.com/CapeCanaveral/Hangar/6160

X-Mailer: IPAD 2.0

Date: Sun, 02 Aug 1998 12:03:56 -0500

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: Making a Coil, requesting recomendations/advice.

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

At 04:59 PM 8/1/98 -0700, you wrote:

>Mike Hammer wrote:

>>

>> At 10:33 PM 7/31/98 EDT, you wrote:

>SNIPS

>> >

>> >Finally, I wanted to be able to measure the frequency of the coil, a

>> >friend in electronic shop suggested a mag pickup connected to a frequency

>> >meter? any one else try this or know a better way?

>>

>> For this you need some test equipment.

>> If you have a signal generator you can base feed the

>> secondary from the generator through a pair of LEDS.

>> The LEDs will light brightest at the secondary resonant

>> frequency.

>>

>> TTYL!

>>

>> Mike Hammer

>> mhammer@misslink.net

>

>

>Mike,

>like this?

>                 LED type? different pol?

>              o-->|---------------)

>                                  )

>    S/G  Sine, pk, pk-pk V out?   )  SEC

>                                  )

>              o--|<---------------)

Gene,

More like this:

)

)

o-->|---|            ) SEC

S/G            |            )

o--|<---|------------)

Use the high brightness LEDs for best results. If your generator

has plenty of drive you can use standard LEDs in a darkened room.

The LEDs will be dark except when the generator is near the

secondary resonant frequency and brightest at the resonant frequency.

Using this method you can determine the capacitance of your discharge

terminals by noting the drop in frequency when adding those terminals

to the secondary. You will find that the actual real capacitance of

the toroids is far less than the programs and formulas indicate.

>

>Which brings to mind another of my dumb questions: Is Sine optimal? What

>happens if a TC could be driven on Sq, Ramped or Triang wave?  What

>might such supplies look like?

Doesnt make much difference. Ive done it with sine and square waves

with equal success.

>S/G driving a great amplifier? Or could a power sup be built to that end?

>My guess is such would be much harder on the caps.....

Dont forget that the Tesla coil tank circuit (gap,cap, primary)

IS an oscillator. This oscillator output is then magnetically

coupled to the secondary. ANY oscillator could do much the

same. BUT only our beloved spark gap oscillator delivers the

high peak power levels and nasty jagged forked lightning.

>

>Would controlling the tank input freq have any advantages?

No. You need to drive the secondary it its natural resonant

frequency. Other frequencies would result in no output.

>

>---------------------------------

>Gene Waldo -  gwaldo@1stnetusa.com  - "Igor in Residence"

>---------------------------------------------------------------

Hang in there Gene.

TTYL!

Mike Hammer

mhammer@misslink.net

X-Mailer: IPAD 2.0

Date: Sun, 02 Aug 1998 13:44:21 -0500

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: wire dia.???

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

At 09:27 PM 8/1/98 -0500, you wrote:

>Hi all, I have been a subscriber for several months and always enjoy reading

>about everyone and their coiling adventures and have decided to attempt the

>construction of my first Tesla coil.

>    I am still in the planning stages and was wondering about the importance

>of wire diameter,  most sec. coils that I have seen appear to be wound with

>small gauge wire (24-22 awg).  Is this small dia. wire neccesary? If so why?

>Or is the smaller gauge used simply to pack more turns per inch thereby

>reducing the overall length of the sec?

There are several conflicting requirements for producing the most

effective secondary coil. You need high Q , high inductance, low RF

resistance and low aspect ratio.

Some of these requirements conflict. You cant have one without

losing the other. So a compromise is reached that gives you

enough of each to have a good performing secondary.

This results in secondaries with aspect ratios (H/D ratio) of

3:1 to 6:1 and from 700-1000 turns. Use the largest guage of

wire that will fit in this space. Turns out this means AWG 22-24

for the most popular secondary coil sizes with AWG 24 being optimum

for a 4 inch diameter secondary and AWG 22 for a 6 incher.

>  Also I plan on building a small size coil first then as I learn more I

>would like to build something larger(6"-8" sec.)  I have (2) 12Kv 30ma

>nst's which are practically new.  If I use one on say a 3" coil  would it be

>possible to later use the both on a 6" coil?  Could I use basically the same

>primary circuit(altering my pri. coil of course)  What about safety and

>sparkgap adjustments?  Anyway any feedback is useful and appreciated.

For that power level (12kv/60) I would recommend a 4 inch secondary.

Wind with about 20-22 inches of AWG 24. Should be capable of 36 inches

easily. The primary for a 4 incher would prove too small for a 6.

You could reuse the material to build a larger primary.

Safety gap and main gap would remain the same since they are based

mostly on primary voltage. You will want about .0066 mfd for a single

12kv/30 and around .0132 for the two in parallel. You dont need these

exact values. Anywhere around those values will be fine.

TTYL!

Mike Hammer

mhammer@misslink.net

X-Mailer: IPAD 2.0

Date: Sun, 02 Aug 1998 13:48:30 -0500

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: PRIMARY WIRE

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

At 08:16 PM 8/1/98 -0700, you wrote:

> Does the size of the primary wire really make that much difference?

>

Depends on your power level and other coil components.

If you have a poor spark gap and lossy SW bottle caps

then the losses in those two areas would mask the loss

in the primary. But once you replace your gap with a

more effective unit and replace those lossy caps with

low loss polyethylene units then the primary loss can

become a significant bottleneck.

Small coils can have several amperes of peak RF current

flowing in the primary circuit. Medium coils tens of amperes.

Hundreds of amperes in large coils. I2R losses become

increasingly significant as power level is increased.

TTYL!

Mike Hammer

mhammer@misslink.net

Comments: Authenticated sender is <megavolt@mail.pcisys.net>

From: megavolt@mail.pcisys.net

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Date: Sun, 2 Aug 1998 19:01:40 +0000

Subject: Re: toroid Renforcement

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Pat,

I'm using paper mache (newspaper strips ,flour, water and

Elmer's glue mix) to cover my eight inch by thirty inch dryer duct

toroid to smooth it out.  I'm then going to fiberglass the whole

thing and cover it with aluminum tape.  It seems like a lot of work

but it should be strong and smooth.  My previous toroid was made from

four inch corrugated black drain pipe and covered with duct tape then

covered with aluminum tape.  This one is four by twenty eight inches

and is very strong. I've noticed that the streamers seem to dance

around a smooth surface better than the plain aluminum dryer duct

where they seem to emit from the same places most of the time.  I

wouldn't think that any foam injecting, paper mache, duct tape or

fiberglass would affect the electrical characteristics...the aluminum

surface is what I believe is important to the performance.

> I got the aluminum duct stuff last night 4" $5.76 at WalMart!

>

> after forming it into a circle I could see that any toroid that is

> heading for the floor it's done for to stuff is very flimsy.

My $.02 worth from a neophyte.

Kent

megavolt@pcisys.net

From: "crowekri" <crowekri@scott.net>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: wire dia.???

Date: Sun, 2 Aug 1998 17:40:12 -0500

X-Mailer: IPAD 2.0

X-MimeOLE: Produced By Microsoft MimeOLE V4.72.3110.3

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Thanks Mike  this is exactly what I wanted to know.

Thanks again,

crowekri

-----Original Message-----

From: Mike Hammer <mhammer@misslink.net>

To: Tesla-2 Mailing List <tesla-2@emachine.com>

Date: Sunday, August 02, 1998 1:48 PM

Subject: Re: wire dia.???

>At 09:27 PM 8/1/98 -0500, you wrote:

>>Hi all, I have been a subscriber for several months and always enjoy reading

>>about everyone and their coiling adventures and have decided to attempt the

>>construction of my first Tesla coil.

>>

>>    I am still in the planning stages and was wondering about the importance

>>of wire diameter,  most sec. coils that I have seen appear to be wound with

>>small gauge wire (24-22 awg).  Is this small dia. wire neccesary? If so why?

>>Or is the smaller gauge used simply to pack more turns per inch thereby

>>reducing the overall length of the sec?

>

>

>

>There are several conflicting requirements for producing the most

>effective secondary coil. You need high Q , high inductance, low RF

>resistance and low aspect ratio.

>

>Some of these requirements conflict. You cant have one without

>losing the other. So a comprimise is reached that gives you

>enough of each to have a good performing secondary.

>

>This results in secondaries with aspect ratios (H/D ratio) of

>3:1 to 6:1 and from 700-1000 turns. Use the largest guage of

>wire that will fit in this space. Turns out this means AWG 22-24

>for the most popular secondary coil sizes with AWG 24 being optimum

>for a 4 inch diameter secondary and AWG 22 for a 6 incher.

>

>>  Also I plan on building a small size coil first then as I learn more I

>>would like to build something larger(6"-8" sec.)  I have (2) 12Kv 30ma

>>nst's which are practically new.  If I use one on say a 3" coil would it be

>>possible to later use the both on a 6" coil?  Could I use basically the same

>>primary circuit(altering my pri. coil of course)  What about safety and

>>sparkgap adjustments?  Anyway any feedback is useful and appreciated.

>

>For that power level (12kv/60) I would recommend a 4 inch secondary.

>Wind with about 20-22 inches of AWG 24. Should be capable of 36 inches

>easily. The primary for a 4 incher would prove too small for a 6.

>You could reuse the material to build a larger primary.

>

>Safety gap and main gap would remain the same since they are based

>mostly on primary voltage. You will want about .0066 mfd for a single

>12kv/30 and around .0132 for the two in parallel. You dont need these

>exact values. Anywhere around those values will be fine.

>

>

>TTYL!

>

>Mike Hammer

>mhammer@misslink.net

>

X-Mailer: IPAD 2.0

Date: Mon, 03 Aug 1998 08:29:03 +0300

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Marco Denicolai <marco@vistacom.fi>

Subject: Re: Making a Coil, requesting recomendations/advice.

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

At 21:05 01/08/98 PDT, you wrote:

>Gene wrote:

>

>>Which brings to mind another of my dumb questions: Is Sine optimal?

>>What happens if a TC could be driven on Sq, Ramped or Triang wave?

>>What might such supplies look like?

>

>Last time I had the 'scope out measuring the secondary resonant

>frequency, I discovered that I could use the sine, square, triangle

>outputs with equal success.  That is - using either waveform gave me a

>dip, at an identical frequency.  However, using sin gave the sharpest

>dip.  You should be able to use squarewave with good luck.

>

Using anything different than sinewave will provide your secondary

with every kind of higher frequencies too, besides your fundamental.

Remember Fourier decomposition: any waveshape can be decomposed as a

sum of sin and cosin shaped higher frequency waves (although of

decreasing amplitude with increasing frequency).

That is why you get the sharpest dip with a sinewave: I wouldn't use

anything else than sinewave, to avoid harmonics.

Marco Denicolai

Comments: Authenticated sender is <megavolt@mail.pcisys.net>

From: megavolt@mail.pcisys.net

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Date: Mon, 3 Aug 1998 05:35:14 +0000

Subject: Re: toroid Renforcement

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Pat,

The high temp aluminum engine paint sounds like an excellent Idea.

I'll try that first and let you know how it works.  I can always tape

over it.

Thank you,

Kent

> Very good idea sir

>

> I wonder what the electrical characteristics of high-temp aluminium

> engine paint are? It would be interesting to get a can of it then spray

> a test and measure the conductivity. It's a shame to put so much work

> into creating the perfect surface to cover it with tape. Might be worth

> a try considering the labor involved in coving it in all that tape

>

> Pat

>

>

megavolt@pcisys.net

From: ralph.hartwell@emachine.com (Ralph Hartwell)

Subject:  Spark gap PVC

Date: Mon,  3 Aug 1998 05:51:00 GMT

Organization: The Energy Machine Information System 504-733-3462

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

P>If any you guys have a chunk of 6" PVC lying say six inches in length

P>that you could part with I would gladly pay shipping.

Pat,

Try a scouting mission around your city and watch for the S&WB crews

working on the water lines.  They usually have scrap sections of pipe

all over the place, which can be liberated after a little chat with the

crew foreman. Another possibility is a visit or phone call to the public

works office, which may result in an invite to dig through the scrap

pile.

Ralph

---

X-Mailer: IPAD 2.0

Date: Mon, 03 Aug 1998 09:06:47 +0300

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Marco Denicolai <marco@vistacom.fi>

Subject: Re: Rotor Gaps

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

At 12:21 31/07/98 -0500, you wrote:

>Marco,

>

>While a rotary may be useful in your case there are a couple

>points to consider. First of all extremely high break rates

>usually result in poor performance at best and component failure

>at worst. The high pulse rates can result in capacitor failure.

>I know of 2 coilers that destroyed expensive commercial pulse

>caps that way.

I'm not scared about lower performance, but capacitor failure

scares me. Have you got any clue about what exactly broke

those capacitors? Extra voltage, higher break rate, temperature,

or what?

>

>12000 rpm is far too high. It will be exceedingly difficult

>to properly balance a rotor for such high rpm. Any wobble or

>vibration at this kind of speed will result in flying shrapnel.

Right. I had problems balancing that "toy" motor. I have got now

a 500W 10000 rpm "handtool". I don't know what that is in english

but it's an hand-held tool (much as a drill) that you can use to

cut steel sheets or pipes. That cost me $20. It is very tough

and already well balanced with my disk.

>

>I believe you mentioned using polycarbonate plastic (Lexan)

>for your rotor in a previous post. Lexan is not suitable

>for that kind of rpm although it can be used at lower speeds.

I gave up with lexan and I use now a 15 cm disk of

"fiber bakelite" (does that mean anything to you?). The disk is

8 mm thick. I am enclosing the motor + disk inside a bigger

thick aluminium and steel box I got, to reduce the noise and

increase the safety. A flying bolt couldn't really make through

the box walls.

>

>I think you would be wise to search for a different lower

>speed motor for your gap.

>

I think you are right but... I still want to try with this. As I said

before, I'm not worried about lower performance and time lost (who

would ever try any change if fear wins always) but I AM worried

about losing my expensive commercial capacitors. Again, any idea

about what destroyed those 2 guys' caps?

Marco Denicolai

From: "Kent Rankin" <kentrankin@theatreorgans.com>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: toroid Renforcement

Date: Mon, 3 Aug 1998 07:14:47 -0400

X-Mailer: IPAD 2.0

X-MimeOLE: Produced By Microsoft MimeOLE V4.72.2106.4

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

I know of a place locally that will let me use their CAM lab.  They even

have a machine that lets you create a computer-based model, and then a

machine makes it from plastic at a rate of about one inch every two minutes.

Would this work for making "the perfect toroid"(covering it with Al tape of

course)?  Or would this plastic somehow absorb some of the electricity?

-Kent Rankin

-----Original Message-----

From: Pat & Heather Moser <moser@madbbs.com>

To: Tesla-2 Mailing List <tesla-2@emachine.com>

Date: Sunday, August 02, 1998 9:37 PM

Subject: Re: toroid Renforcement

>Very good idea sir

>

>I wonder what the electrical characteristics of high-temp aluminium

>engine paint are? It would be interesting to get a can of it then spray

>a test and measure the conductivity. It's a shame to put so much work

>into creating the perfect surface to cover it with tape. Might be worth

>a try considering the labor involved in coving it in all that tape

>

>Pat

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Date: Mon, 03 Aug 1998 04:14:57 -0700

From: "Fleischwulf" <fleischwulf@mailexcite.com>

Cc:

X-Sent-Mail: off

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

Subject: Re: Rotor Gaps

X-Sender-Ip: 208.153.7.9

Organization: MailExcite  (http://www.mailexcite.com:80)

Sender: tesl-err@emachine.com

Marco,

I can tell you a little bit about rotary gaps. Any rotary spinning above 3500

RPM is a real danger. I once worked for a gas turbine manufacture. The deflection due to the weight of the electrodes on the rotor will cause the rotor to deflect.

This deflection over time coupled with a phenomeon known as plastic creep will cause cracks to form in the rotor. Once these cracks form, fracture is imminent.

At these speeds a containment shield is a necessity. BE CAREFUL !!!

Paul Marshall

---

Leben frei oder Starb !

On Mon, 03 Aug 1998 09:06:47   Marco Denicolai wrote:

>At 12:21 31/07/98 -0500, you wrote:

>>Marco,

>>

>>While a rotary may be useful in your case there are a couple

>>points to consider. First of all extremely high break rates

>>usually result in poor performance at best and component failure

>>at worst. The high pulse rates can result in capacitor failure.

>>I know of 2 coilers that destroyed expensive commercial pulse

>>caps that way.

>

>I'm not scared about lower performance, but capacitor failure

>scares me. Have you got any clue about what exactly broke

>those capacitors? Extra voltage, higher break rate, temperature,

>or what?

>

>>

>>12000 rpm is far too high. It will be exceedingly difficult

>>to properly balance a rotor for such high rpm. Any wobble or

>>vibration at this kind of speed will result in flying shrapnel.

>

>Right. I had problems balancing that "toy" motor. I have got now

>a 500W 10000 rpm "handtool". I don't know what that is in english

>but it's an hand-held tool (much as a drill) that you can use to

>cut steel sheets or pipes. That cost me $20. It is very tough

>and already well balanced with my disk.

>

>>

>>I believe you mentioned using polycarbonate plastic (Lexan)

>>for your rotor in a previous post. Lexan is not suitable

>>for that kind of rpm although it can be used at lower speeds.

>

>I gave up with lexan and I use now a 15 cm disk of

>"fiber bakelite" (does that mean anything to you?). The disk is

>8 mm thick. I am enclosing the motor + disk inside a bigger

>thick aluminium and steel box I got, to reduce the noise and

>increase the safety. A flying bolt couldn't really make through

>the box walls.

>

>>

>>I think you would be wise to search for a different lower

>>speed motor for your gap.

>>

>

>I think you are right but... I still want to try with this. As I said

>before, I'm not worried about lower performance and time lost (who

>would ever try any change if fear wins always) but I AM worried

>about loosing my expensive commercial capacitors. Again, any idea

>about what destroyed those 2 guys' caps?

>

>

>

>

> Marco Denicolai

>

X-Originating-IP: [199.217.140.22]

From: "Bill the arcstarter" <arcstarter@hotmail.com>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Making a Coil, requesting recomendations/advice.

Date: Mon, 03 Aug 1998 05:09:32 PDT

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

Marco wrote:

>>Gene wrote:

>>

>>>Which brings to mind another of my dumb questions: Is Sine optimal?

>>>What happens if a TC could be driven on Sq, Ramped or Triang wave?

>>>What might such supplies look like?

>>

>>Last time I had the 'scope out measuring the secondary resonant

>>frequency, I discovered that I could use the sine, square, triangle

>>outputs with equal success.  That is - using either waveform gave me a

>>dip, at an identical frequency.  However, using sin gave the sharpest

>>dip.  You should be able to use squarewave with good luck.

>>

>

>Using anything different than sinewave will provide your secondary

>with every kind of higher frequencies too, besides your fundamental.

>Remember Fourier decomposition: any waveshape can be decomposed as a

>sum of sin and cosin shaped higher frequency waves (althought of

>decreasing amplitude with encreasing frequency).

>

>That is why you get the sharpest dip with a sinewave: I wouldn't use

>anything else than sinewave, to avoid harmonics.

You are certainly correct about the fourier breakdown.  However, from a

purely empirical point of view, I tried it and it appears to work with

square wave excitation.

-Bill

Date: Mon, 03 Aug 1998 09:09:47 -0700

From: Gene Waldo <gwaldo@1stnetusa.com>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Organization: S.A.P.I.E.N.T. (Surly, Agressive, Politically Incorrect Enemies of Non Thinkers)

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: toroid Renforcement

Sender: tesl-err@emachine.com

megavolt@mail.pcisys.net wrote:

>

> Pat,

>

> The high temp aluminum engine paint sounds like an excellent Idea.

> I'll try that first and let you know how it works.  I can always tape

> over it.

>

> Thank you,

>

> Kent

>

Pat,& Kent,

I had the notion to spray Al paint on cap plates a while back; most

suggested that the paint would not be conductive enough. Haven't persued

it.  I suggest that you mask off a 1"x3" test patch on plex or something

and run a conductivity test with VOM, before making the larger

investment of time & materials. Let us know the results!

---------------------------

Gene Waldo -  gwaldo@1stnetusa.com  - "Igor in Residence"

Date: Mon, 03 Aug 1998 18:10:39 -0400

From: Pat & Heather Moser <moser@madbbs.com>

Organization: Planet Cheese Creations

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Sparkgap Length

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Got a question concerning gaps, I went to Lowes and got 5

feet of 1" inch copper pipe since I have plenty for the 9

gaps is there a point where the gaps total length becomes

counterproductive?  My design originally called for 3" long

gaps 9 total I'm starting out with 9KV 30ma but I felt if I

was going to put the work in to it I should build it for

higher voltages when I get there.

In addtion can you solder the pipes to the bolts?  I plan to

use 1/4 20 brass all thread and nuts to hold things

together. I remember Arcstarter Bill brazed them I assume

with my vacuum design the gaps better not get hot enough to

melt solder!

Opinions?

See the Gap Design here - 3d Model coming soon

http://www.netsync.net/users/lrichard/SPARK.gif

From: <Ashpilot@aol.com>

Date: Mon, 3 Aug 1998 18:51:01 EDT

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Toroid Renforcement

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

A suggestion for the manufacture of toroids,  using a dense foam core and

fiber glass ought to be strong enough.  If you use a foam like the insulation

type (made by Styrofoam) that they have at Home Depot, you can get one sheet

cut it into squares a little larger than the final diameter of your toroid.

Then epoxy several Rings to the square, sandwich style, to have a thick enough

piece the right shape. BUT It has to be the type with out the foil face!!

This insul. foam is pretty tough and two layers of Fiber Glass cloth would

make it pretty crush resistant.  It shapes easily with emery paper and a Hack

Saw blade for gross material removal.  I built an Airfoil section 1 inch thick

by 12 inch long. We drove a car over it and it remained intact. It also took

about 250lbs to snap it over a table edge.

Date: Mon, 03 Aug 1998 20:45:13 -0400

From: Pat & Heather Moser <moser@madbbs.com>

Organization: Planet Cheese Creations

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Spark gaps

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

thanks mike every time you give a nice explanation like that

I see a discriptive illustration to go with it are the ions positive or

negative for purposes of illustration?

By the way Ive got the copper should I make 4" gaps or stick with the 3"

longer gap I would assume longer gaps will increase cooling but create a

larger surface area to quench.

Pat

From: "Choad" <choad69@geocities.com>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Fw: New cap design

Date: Tue, 4 Aug 1998 02:43:47 -0500

X-Mailer: IPAD 2.0

X-MimeOLE: Produced By Microsoft MimeOLE V4.72.3110.3

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

hello, everyone

While I was thinking. I came up with a different way to make a rolled cap.

instead of cutting all the layers into sheets like we normally do. Why not make

the poly 2 times as wide and fold it around the plate? Corey Edmunds helped turn this idea into what it is now. There is a pic of the end view of how the layers would be stacked: http//www.geocities.com/SouthBeach/Pier/2756/newcap.jpg

There is only 1 disadvantage we saw at the time. That is trapped air in one end. Maybe with a strong pump this would be overcome. Some of the advantages we saw to this design. Less poly needed to make a cap, and more insulation around the plates. This would work really well for the extended plate rolled cap design as the plates could go straight out of the sides

.

Shaun O'Brien

Member of The Oklahoma Tesla Coil Builders

http://www.geocities.com/SouthBeach/Pier/2756/index.htm

X-Mailer: IPAD 2.0

Date: Tue, 04 Aug 1998 15:20:04 -0500

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: Rotor Gaps

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

At 09:06 AM 8/3/98 +0300, you wrote:

>At 12:21 31/07/98 -0500, you wrote:

>>Marco,

>>

>>While a rotary may be useful in your case there are a couple

>>points to consider. First of all extremely high break rates

>>usually result in poor performance at best and component failure

>>at worst. The high pulse rates can result in capacitor failure.

>>I know of 2 coilers that destroyed expensive commercial pulse

>>caps that way.

>

>I'm not scared about lower performance, but capacitor failure

>scares me. Have you got any clue about what exactly broke

>those capacitors? Extra voltage, higher break rate, temperature,

>or what?

>

<SNIP>

Marco,

Both caps died from excessive break rates. One of the caps

was a commercial pulse cap. It suffered an internal breakdown

that boiled the oil and the plastic housing exploded. Very nasty.

I was mistaken on the 2nd cap. It was a homemade rolled cap

not a commercial one. However this cap had been reliable

and was over two years old and worked well until its owner

started using a rotary gap. It initially gave pretty good results

using a 14.4 KV pole pig and rotary gap. It failed while using

quite high rotary speeds.

Most coilers with high power pig driven coils find that break rates

from 300-400 per second are all that are necessary for best performance.

TTYL!

Mike Hammer

mhammer@misslink.net

From: <Ashpilot@aol.com>

Date: Tue, 4 Aug 1998 18:51:12 EDT

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Spark gaps

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

I have heard the term Quenched in reference to spark gaps, but don't know what

it means, can someone give me a definition?

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Spark gaps

Date: Wed, 5 Aug 1998 00:14:08 +0000

From: Lan <rune@dircon.co.uk>

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

>I have heard the term Quenched in reference to spark gaps, but don't know what

>it means, can someone give me a definition?

When a spark gap fires or passes electricity, the air heats a lot + will contain

more left over ions and remains more likely to spark at a lower voltage. To

keep the sparking voltage from dropping the aim is to get the air to move out

of from between the electrodes. Air blowers, magnets and other things will help.

X-Mailer: IPAD 2.0

Date: Tue, 04 Aug 1998 19:25:41 -0500

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: Spark gaps

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

At 06:51 PM 8/4/98 EDT, you wrote:

>I have heard the term Quenched in reference to spark gaps, but don't know

>what it means, can someone give me a definition?

A good question. Its a term that many throw around without really knowing

what it means.

Rather than type in a long answer here I will paste

in a section of a Tesla coil FAQ that I have been working

on that covers the subject of gap quenching.

This FAQ was nearing completion until a disk failure wiped me

out. Its taken a long time to recreate lots of lost documents.

<begin include>

What is Quenching?

-----------------------

No discussion of spark gaps is complete without at least a rough

definition of "quenching". This term is commonly thrown around

when talking about spark gaps.

Quenching refers to the ability of an established arc to be

extinguished. It is much easier to start a gap firing than it is to

put it out once it has fired. In Tesla coils quenching the arc is

critical to good coil performance.

A cold, non-firing, spark gap is "clean". It contains no plasma

or hot ions. On applying voltage to the gap, a tension is esta-

blished between the electrode faces. Once the tension becomes high

enough to ionize the air between the gap the resistance drops rapidly.

This breakdown ionizes the gas between the electrodes and the arc

begins to ablate and ionize the metal electrodes themselves.

This mixture of ions forms a highly conductive plasma between

the gap electrodes. Without this highly conductive channel through

the gap, efficient tank circuit oscillation would not be possible.

But this plasma also shorts the gap out. A gap choked with hot ions

does not want to open to allow the capacitors to recharge for the

next pulse. The gap is  "dirty" with hot ionized gases and must be

cleaned so it can be extinguished.

To quench the hot gap the built up ions must be removed. This can be

done with a flow of air from a fan, blower or vacuum cleaner motor.

Once the excess ions are removed and the gap is cooled by the airflow

it can quench much easier.

Anything that reduces the amount of heat in the gap will aid quenching.

Series static gaps reduce the heat in each gap by breaking up the gap

in smaller segments. Quenching becomes easier.

<end include>

Comments, criticism and cash donations always welcome.

TTYL!

Mike Hammer

mhammer@misslink.net

From: "Ryan E. Jester" <ryanj@primary.net>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Introduction

Date: Tue, 4 Aug 1998 19:54:41 -0500

X-Mailer: IPAD 2.0

X-MimeOLE: Produced By Microsoft MimeOLE V4.72.3110.3

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Hey everyone.  Just like to introduce myself to the list recipients.

I've built a coil myself.

Here is a rough run-down of what I've done:

- Two homemade caps, both at exactly .022uf these are wired in series.

- Two 12kvac, 30 ma neon sign xfmrs wired in parallel (60ma for the extra umphh)

- Primary coil is a cone shapped helical form about 23" dia, thats 13 turns of

3/8" soft copper tubing.

- Secondary coil is 28" high and 6" dia, ~950 turns of 22awg magnet wire.

- Also a discharged terminal is toroid in shape, about 16" dia, 2 1/2" high.

Ohh, almost forgot the spark gap, seems to be the topic right now.

- Just using a plain ol cylinder spark gap.  Using 6 electrodes which are 1 1/2"   dia hard copper pipe.  These are gapped at .028"  Also a muffin fan on top to

help  get rid of hot ions, in turn priducing better "quenching" as Mike Hammer   described.

Results? 3-3 1/2ft discharge.  Much thanks to Richard Quick who helped me get started.  Any questions or if ya want pictures let me know!

Ryan Jester

ryanj@primary.net

X-Originating-IP: [208.16.149.186]

From: "Bill the arcstarter" <arcstarter@hotmail.com>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Sparkgap Length

Date: Tue, 04 Aug 1998 19:30:36 PDT

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

Moser wrote:

>In addtion can you solder the pipes to the bolts?  I plan to

>use 1/4 20 brass all thread and nuts to hold things

>together. I remember Arcstarter Bill brazed them I assume

>with my vacuum design the gaps better not get hot enough to

>melt solder!

Yep - if you use brass bolts and clean all surfaces with scotch brite/

sandpaper, and use flux - then you can solder them together.  You will

have to use a propane torch or equivalent.

Soldering would be fine.  I was just being difficult!  Actually I used

a phosphor-copper (fake silver solder), since brazing brass to copper is

asking for trouble...

My first gap died due to excessive arcing between the pipe section and

the bolt.  But - to be fair I must also add that we were dumping 7Kw

through the static gap! :)

About the only arcs I've been starting lately involve a bunch of

welding I've been doing out in the garage.  Nothing quite like welding

on a hot summer evening... ?  Hey - gotta get that tan somehow!  Pics

available soon...

-Bill the arcstarter

Professional mad scientist (ask about *free* lessons!)

Still starting arcs in Cinci, OH

http://www.geocities.com/CapeCanaveral/Hangar/6160

X-Originating-IP: [208.16.149.186]

From: "Bill the arcstarter" <arcstarter@hotmail.com>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Sparkgap Length

Date: Tue, 04 Aug 1998 19:51:10 PDT

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

I wrote:

> Yep - if you use brass bolts and clean all surfaces with scotch brite/

>sandpaper, and use flux - then you can solder them together.  You will

>have to use a propane torch or equivalent.

I forgot to say that you should buy solder designed for plumbing - which

is either 50% lead 50% tin or 100% lead (I'm not sure).  The electrical

solder won't really stick to plumbing (like copper pipes) for some

reason which I can't fathom...

Of course there's this nice politically correct plumbing solder which

contains no lead.  I don't use that stuff! :)

-Bill

Date: Tue, 04 Aug 1998 22:09:16 -0500

From: Bert Hickman <bert.hickman@aquila.com>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Organization: Stoneridge Engineering

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Introduction

Sender: tesl-err@emachine.com

Ryan,

Welcome to the list! Sounds like you've got a very nice running coil.

The only thing I'd recommend is perhaps going to a 20" x 4" toroid,

since the current 2 1/2" toroid might be a bit too small for the power

level you're running. Richard Quick unfortunately is no longer actively

coiling, but he may again in the future...

Safe coilin' to you!

-- Bert --

Ryan E. Jester wrote:

>

> Hey everyone.  Just like to introduce myself to the list recipients.

> I've built a coil myself.

>

> Here is a rough run-down of what I've done:

> - Two homemade caps, both at exactly .022uf these are wired in series.

> - Two 12kvac, 30 ma neon sign xfmrs wired in parallel (60ma for the

>   extra umphh)

> - Primary coil is a cone shapped helical form about 23" dia, thats 13

>   turns of 3/8" soft copper tubing.

> - Secondary coil is 28" high and 6" dia, ~950 turns of 22awg magnet wire.

> - Also a discharged terminal is toroid in shape, about 16" dia, 2 1/2" high.

> Ohh, almost forgot the spark gap, seems to be the topic right now.

> - Just using a plain ol cylinder spark gap.  Using 6 electrodes which

> are 1 1/2" dia hard copper pipe.  These are gapped at .028"  Also a

> muffin fan on top to help get rid of hot ions, in turn priducing

> better "quencing" as Mike Hammer discribed.

>

> Results? 3-3 1/2ft discharge.  Much thanks to Richard Quick who helped

> me get started.  Any questions or if ya want pictures let me know!

>

> Ryan Jester

> ryanj@primary.net

Date: Tue, 04 Aug 1998 22:56:21 -0700

From: Gene Waldo <gwaldo@1stnetusa.com>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Organization: S.A.P.I.E.N.T. (Surly, Agressive, Politically Incorrect Enemies of Non Thinkers)

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Spark gaps

Sender: tesl-err@emachine.com

Mike Hammer wrote:

>

> At 06:51 PM 8/4/98 EDT, you wrote:

> >I have heard the term Quenched in reference to spark gaps, but don't know

> >what it means, can someone give me a definition?

>

> A good question. Its a term that many throw around

> without really knowing what it means.

>

SNIPS

> Mike Hammer

> mhammer@misslink.net

Mike, again, your lucid facility serves well!

I recently saw the term, "magnetism"  or "magnets" tossed into the gap

quenching discussion; how does or may this apply?

--------------------------------------

Gene Waldo -  gwaldo@1stnetusa.com  - "Igor in Residence"

Date: Tue, 04 Aug 1998 23:07:03 -0700

From: Gene Waldo <gwaldo@1stnetusa.com>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Organization: S.A.P.I.E.N.T. (Surly, Agressive, Politically Incorrect Enemies of Non Thinkers)

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Introduction

Sender: tesl-err@emachine.com

Ryan E. Jester wrote:

>

> Hey everyone.  Just like to introduce myself to the list recipients.

> I've built a coil myself.

>

> Here is a rough run-down of what I've done:

SNIPS

> Results? 3-3 1/2ft discharge.  Much thanks to Richard Quick who helped

> me get started.  Any questions or if ya want pictures let me know!

>

> Ryan Jester

> ryanj@primary.net

Ryan,

We all want pics! Question bes: what is our current status on binary

E-missions?  Last time, Mike seemed to hold that they were troublesome

to the list server... or some such....

Mike, is this still the case?

----------------------------------------

Gene Waldo -  gwaldo@1stnetusa.com  - "Igor in Residence"

From: "Ryan E. Jester" <ryanj@primary.net>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Picture of spark discharge

Date: Wed, 5 Aug 1998 06:41:37 -0500

X-Mailer: IPAD 2.0

X-MimeOLE: Produced By Microsoft MimeOLE V4.72.3110.3

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Well, hopefully this won't anger anyone....I'm attaching a 99k pic of my coil in action.  Sorry that ya have to d/l it if you dont want it  :)

I will set up a web site soon that has construction in detail.

Oh and BTW, this pic was taken with the shutter open for ~30sec.

Ryan Jester

ryanj@primary.net

Attachment Converted: "C:\MISSING_LINK\EUDORA\Attach\spark002.jpg"

Editors note. Binary attachments are not contained in the archives.

From: "Ryan E. Jester" <ryanj@primary.net>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Picture of my setup

Date: Wed, 5 Aug 1998 06:53:38 -0500

X-Mailer: IPAD 2.0

X-MimeOLE: Produced By Microsoft MimeOLE V4.72.3110.3

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

And here is an overall picture of the configuration of my coil.

Everything is pretty much non-original, I used a lot of Richard Quick ideas

like spark gap design and cap design.  If anyone has seen Richard Quick's video (the one with a coil producing 10ft sparks) he made a vacuum gap that i'm going

to try and base my next spark gap off of.  Either that or build a rotary gap, I dunno which yet. I also need to incorporate a safety gap in this system and build

a better toroid.

Just some of the things on the to-do list.

Ryan Jester

ryanj@primary.net

Attachment Converted: "C:\MISSING_LINK\EUDORA\Attach\setup1.jpg"

Editors note. Binary attachments are not contained in the archives.

Date: Wed, 05 Aug 1998 10:01:05 -0500

From: RODERICK MAXWELL <tank@magnolia.net>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: vacuum gap

Sender: tesl-err@emachine.com

I am building a new vacuum gap using 10 pieces of 3/4" copper tubing

5" long. Can anyone tell me if this gap will stand up to 3Kw. I have a

powerful shop-vac to pull the air through the teeth of the gap. The

first vacuum gap I built used 6 pieces of 1 1/8" copper tubing spaced

30mils apart.

Frankensteins Helper

Max

Date: Wed, 05 Aug 1998 09:09:22 -0700

From: Gene Waldo <gwaldo@1stnetusa.com>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Organization: S.A.P.I.E.N.T. (Surly, Agressive, Politically Incorrect Enemies of Non Thinkers)

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Picture of spark discharge

Sender: tesl-err@emachine.com

Ryan E. Jester wrote:

>

> Well, hopefully this won't anger anyone....I'm attaching a 99k pic of

> my coil in action.

> Oh and BTW, this pic was taken with the shutter open for ~30sec.

>

> Ryan Jester

> ryanj@primary.net

>

Ryan,

Nice pics! Digital or analog camera?

I'm guessing analog; scanned print? scanner res?

I think it would be neat if we began a running commentary on 'The

digital camera and its use in hi-RF environments.'

Main points being: exposure details, optimal exposures, was the camera

fried? There seems to be no seat-of-the-pants yet developed for the

transition from analog to digital in this area. Fear and trepidation

rule!

Analog techniques would suggest for your shot of the discharge, a

slighly under exposed flash or ambient setup shot and a double exposure

in darkness for the sparks, of much less than 30 sec. Or simply more

ambient illumination.  Is almost a no-can-do for the digital!

My impression is that the visual impact of a good discharge is

diminished by the broad strokes created by long exposures.

Would we welcome commentary in this area?

------------------------------------------------

Gene Waldo -  gwaldo@1stnetusa.com  - "Igor in Residence"

Date: Wed, 05 Aug 1998 09:28:25 -0700

From: Gene Waldo <gwaldo@1stnetusa.com>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Organization: S.A.P.I.E.N.T. (Surly, Agressive, Politically Incorrect Enemies of Non Thinkers)

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: J ladder & direct 2ndry?

Sender: tesl-err@emachine.com

Dan has acquired 2 6' lengths of neat steel tube, about 1.5" dia. and no

immediate use for them; I recall running the secondary output in parallel

with a large cap, for some quite different effects on a ladder. Needless to

say, we lost the cap in short order!

I recall that free standing rods (OK for NST disch.) were not suitable,

as electrostatic attraction caused the rods to oscillate, and we got

simply intermittant crash-bangs at the tips; taut wire or rigid ladder

would be req'd.

Has this sort of ladder hookup been tried, off the secondary?

----------------------------------------

Gene Waldo -  gwaldo@1stnetusa.com  - "Igor in Residence"

Date: Wed, 5 Aug 1998 18:33:02 +0100

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Stephen Bell <steve@breakfix.demon.co.uk>

Subject: Re: Picture of spark discharge

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

In message <35C883B2.45F@1stnetusa.com>, Gene Waldo

<gwaldo@1stnetusa.com> writes

>Nice pics! Digital or analog camera?

>

>I'm guessing analog; scanned print? scanner res?

>

>I think it would be neat if we began a running commentary on 'The

>digital camera and its use in hi-RF environments.'

>

>

>Would we welcome commentary in this area?

>

>------------------------------------------------

>Gene Waldo -  gwaldo@1stnetusa.com  - "Igor in Residence"

>

I've been using a digital camera for all my Tesla coil shots.  I've got

as close as 20 cm without frying the electronics.

Maybe I should finish building the coil before boasting! ;-)

------------------------------------------------

Stephen Bell        (steve@breakfix.demon.co.uk)

Stuff also available at www.breakfix.demon.co.uk

------------------------------------------------

X-Mailer: IPAD 2.0

Date: Wed, 05 Aug 1998 12:33:55 -0500

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: Spark gaps

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

At 08:45 PM 8/3/98 -0400, you wrote:

>thanks mike every time you give a nice explanation like that

>I see a discriptive illustration to go with it are the ions positive or

>negative for purposes of illustration?

Hmm. Never gave that any thought. My guess it would be a mixture

of positive and negatives ions as recombination occurs very quickly

once the gap is quenched.

>

>By the way Ive got the copper should I make 4" gaps or stick with the 3"

>longer gap I would assume longer gaps will increase cooling but create a

>larger surface area to quench.

>

>Pat

Depends on your proposed power level. My gap has 3 inch long electrodes

of 3/4 inch copper. Has worked well at power levels up to 1500 watts.

Since you are using larger 1 inch copper I would say 3 inch long electrodes

would be fine.

TTYL!

Mike Hammer

mhammer@misslink.net

X-Mailer: IPAD 2.0

Date: Wed, 05 Aug 1998 12:43:41 -0500

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: Spark gaps

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

At 10:56 PM 8/4/98 -0700, you wrote:

>Mike Hammer wrote:

>>

>> At 06:51 PM 8/4/98 EDT, you wrote:

>> >I have heard the term Quenched in reference to spark gaps, but don't know

>> >what it means, can someone give me a definition?

>>

>> A good question. Its a term that many throw around

>> without really knowing what it means.

>>

>SNIPS

>> Mike Hammer

>> mhammer@misslink.net

>

>Mike, again, your lucid facility serves well!

Thanks. I must have been sober. :)

>

>I recently saw the term, "magnetism"  or "magnets" tossed into the gap

>quenching discussion; how does or may this apply?

>

>--------------------------------------

>Gene Waldo -  gwaldo@1stnetusa.com  - "Igor in Residence"

Gene,

Anything to move those ions from between the electrode faces

will aid quenching. A very strong magnetic field can help

by dispersing those ions. It takes a very strong magnetic

field. Something like those neodymium-iron-boron magnets.

The magnets must be placed very close to the gap thus creating

problems with arcing to the magnets themselves.

Forced air is a much simpler means and offers the added advantage

of cooling the electrodes.

TTYL!

Mike Hammer

mhammer@misslink.net

X-Mailer: IPAD 2.0

Date: Wed, 05 Aug 1998 12:51:22 -0500

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: Introduction

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

At 11:07 PM 8/4/98 -0700, you wrote:

>Ryan E. Jester wrote:

>>

>> Hey everyone.  Just like to introduce myself to the list recipients.

>> I've built a coil myself.

>>

>> Here is a rough run-down of what I've done:

>SNIPS

>> Results? 3-3 1/2ft discharge.  Much thanks to Richard Quick who helped

>> me get started.  Any questions or if ya want pictures let me know!

>>

>> Ryan Jester

>> ryanj@primary.net

>

>Ryan,

>We all want pics! Question bes: what is our current status on binary

>E-missions?  Last time, Mike seemed to hold that they were troublesome

>to the list server... or some such....

>

>Mike, is this still the case?

>----------------------------------------

>Gene Waldo -  gwaldo@1stnetusa.com  - "Igor in Residence"

Gene and all,

The new list server tolerates binary files well sent as a file

attachment. However I would like to discourage the practice

of sending binaries via a mailing list.

Not everyone has an account that can receive binary files

and many subscribers have the list come to their accounts

at work. A large amount of binary files can cause these people

problems.

However an occasional pic shouldnt be too much trouble as long

as its not too large. Use JPG to reduce the size. Do not send

BMP files. They are usually very large.

I would prefer that anyone with pics or other binary files

they wish to share with the list put them up on a web page

and post the URL to the list instead. That way those that

are interested can retrieve them at their convenience and

those that do not want them are not bothered.

I have web space available and would gladly host any pics

for those without a web page.

TTYL!

Mike Hammer

mhammer@misslink.net

X-Mailer: IPAD 2.0

Date: Wed, 05 Aug 1998 13:13:25 -0500

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: Introduction

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

At 07:54 PM 8/4/98 -0500, you wrote:

>Hey everyone.  Just like to introduce myself to the list recipients.

>I've built a coil myself.

>

>Here is a rough run-down of what I've done:

>- Two homemade caps, both at exactly .022uf these are wired in series.

>- Two 12kvac, 30 ma neon sign xfmrs wired in parallel (60ma for the extra umphh)

>- Primary coil is a cone shapped helical form about 23" dia, thats 13 turns

>- of 3/8" soft copper tubing.

>- Secondary coil is 28" high and 6" dia, ~950 turns of 22awg magnet wire.

>- Also a discharged terminal is toroid in shape, about 16" dia, 2 1/2" high.

> Ohh, almost forgot the spark gap, seems to be the topic right now.

>- Just using a plain ol cylinder spark gap.  Using 6 electrodes which are 1 1/2" dia >hard copper pipe.  These are gapped at .028"  Also a muffin fan on top to help get

>rid of hot ions, in turn producing better "quenching" as Mike Hammer described.

>

>Results? 3 - 3 1/2ft discharge.  Much thanks to Richard Quick who helped me get

>started.  Any questions or if ya want pictures let me know!

>

>Ryan Jester

>ryanj@primary.net

>

Ryan,

Sounds like a good coil. 36-42 inches from 12kv/60ma is not bad at all.

I agree with Bert Hickmans advice about your toroid. A thicker cross

section will allow higher voltages to build up before breakout.

I imagine with your thin toroid you are getting many streamers at once.

A thicker toroid will concentrate those into fewer longer streamers.

My first really good toroid was 16 X 4 made from dryer duct.

If you have plenty of primary turns available an even larger

19 X 6 toroid may be better and your coil will benefit

from the lower frequency.

Unfortunately Richard Quick has dropped out of circulation

due to personal problems. I think it unlikely he will resume

coiling any time soon.

TTYL!

Mike Hammer

mhammer@misslink.net

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: My Capacitors Suck

Date: Wed, 5 Aug 1998 19:25:58 +0000

From: Lan <rune@dircon.co.uk>

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

I have a problem with my electrolyte capacitors, basically they suck. One of

the jars forms bright sparks all over the place, for reasons I cant work out

why its doing this. I covered it with engine oil and the bright sparks formed

just on the top of the engine oil, no lines, paths, just a bright burning spark

that appears on places that there shouldnt be any electrical paths too. I

managed somehow to get it working once without any sparking. The oil tenses

around the jars and bottles. Right now it doesnt tense at all, just stays

relaxed and not bothered. And its forming bacteria all over the thing. And the

salt corrodes the copper wires I put inside if they touch the air. Well I have

had enough of that thing. So I will pour it all out when i get the tesla coil

made and have a trial run. I am thinking of using copper plates and oil. Is

that a better idea? How is engine oil for capacitor oil? I use castrol GTX

right now to cover the electrolytes. anyone know better ones?

X-Mailer: IPAD 2.0

Date: Wed, 05 Aug 1998 14:54:26 -0500

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: Picture of spark discharge

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

At 06:41 AM 8/5/98 -0500, you wrote:

>

>Well, hopefully this won't anger anyone....I'm attaching a 99k pic of my coil in action.  Sorry that ya have to d/l it if you dont want it  :)

>I will set up a web site soon that has construction in detail.

>

>Oh and BTW, this pic was taken with the shutter open for ~30sec.

>

>Ryan Jester

>ryanj@primary.net

Ryan,

An occasional binary is OK. Just dont post any huge ones

or a bunch all at once.

Nice picture and your coil appears to be running quite well.

Just a couple suggestions on how to improve your pictures.

Long exposures in nearly total darkness result in a mass of

streamers running everywhere. Most are over exposed and overlap

one another. Its better to illuminate the coil and background

dimly and make your exposure from 1/4-1/2 second on good ASA 400

film. This method will show the coil and the background enough

to give some perspective to the streamers. Plus you will be able

to see the individual streamers.

Check out the pics on my web page to see the results.

http://www.misslink.net/mhammer/

TTYL!

Mike Hammer

mhammer@misslink.net

X-Mailer: IPAD 2.0

Date: Wed, 05 Aug 1998 16:37:41 -0500

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: My Capacitors Suck

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

At 07:25 PM 8/5/98 +0000, you wrote:

>I have a problem with my electrolyte capacitors, basically they suck.

>One of the jars forms bright sparks all over the place, for reasons I cant

>work out why its doing this. I covered it with engine oil and the bright

>sparks formed just on the top of the engine oil, no lines, paths, just a

>bright burning spark that appears on places that there shouldnt be any

>electrical paths too.

What you are seeing is probably corona. Its going to occur

no matter what you do to suppress it. Try to maintain the fluid

levels inside and outside the bottles to equal heights.

>I managed somehow to get it working once without any sparking. The oil tenses

>around the jars and bottles. Right now it doesnt tense at all, just stays

>relaxed and not bothered. And its forming bacteria all over the thing. And the

>salt corrodes the copper wires I put inside if they touch the air. Well I have

>had enough of that thing. So I will pour it all out when i get the tesla coil

>made and have a trial run.

SW bottles should be just an interim measure until you get

some quality polyethylene caps built. True enough that SW caps

"suck" but they do get you sparking at low cost.

>I am thinking of using copper plates and oil. Is that a better idea?

If you mean just using copper plates immersed in oil with

no other dielectric than I would say no it is not a better idea.

Oil is a good insulator and has good dielectric properties but

its breakdown voltage is LESS than that of polyethylene. Most

mineral oils breakdown anywhere from 200-275 volts AC per mil.

Some transformer oils are even less. Polyethylene has a breakdown

rating of around 350 VAC per mil. The BEST caps use poly dielectric

in oil to suppress corona.

>How is engine oil for capacitor oil? I use castrol GTX

>right now to cover the electrolytes. anyone know better ones?

Automotive lubricants are just about the worst thing you

could use for caps. They contain numerous metallic and

semi-metallic anti-wear, anti-foam and anti-who knows what

else additives. They have extremely poor dielectric properties.

Much better is regular mineral oil from the pharmacy.

For a more in depth look at various oils Fr. Tom McGahee

collected some material into a single document. You can find

that file on Dave Bronakowski's archive page. The file name

is oiltypes.txt and the URL is :

http://www.velocity.net/~djb/files.shtml

TTYL!

Mike Hammer

mhammer@misslink.net

From: <Ashpilot@aol.com>

Date: Wed, 5 Aug 1998 18:08:54 EDT

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: My Capacitors Suck

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

I am using two salt water caps now, I built them from old coffee jars.  I used

alum foil on the outside, and black plastic elect. tape to cover the foil on

the outside.  On one I left some foil exposed on the edge and I got a display

like the one you describe. I wrapped more tape to cover the exposed foil and

the sparks went away.  I do get a nice blue glow tho.

I used Steel screws through the cap of each jar for the electrode and I

haven't seen any corrosion yet, and I topped my salt water off with about a

tablespoon full of oil each. (Castrol GTX too!? either its cheap or just

coincidence... :)

From: <Ashpilot@aol.com>

Date: Wed, 5 Aug 1998 18:12:00 EDT

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Introduction

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Does anyone now how to determine the best size toroid for the secondary?

From: "Ryan E. Jester" <ryanj@primary.net>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Picture of spark discharge

Date: Wed, 5 Aug 1998 19:01:41 -0500

X-Mailer: IPAD 2.0

X-MimeOLE: Produced By Microsoft MimeOLE V4.72.3110.3

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

-----Original Message-----

From: Gene Waldo <gwaldo@1stnetusa.com>

To: Tesla-2 Mailing List <tesla-2@emachine.com>

Date: Wednesday, August 05, 1998 11:39 AM

Subject: Re: Picture of spark discharge

>Ryan,

>Nice pics! Digital or analog camera?

>

>I'm guessing analog; scanned print? scanner res?

I used a Minolta 350si camera, 400ASA Mystic Color Lab film (Free sample off

the web!) and had the shutter open for 30sec, no flash, on a tripod, put the

10 sec timer on, cranked up the tesla coil, quickly turned off the lights

and hoped I got the picture :)  I scanned it with a cheap UMAX scanner with

a res of 200dpi (I think) or around that number.    One thing I was thinking

about was I had that camera pretty close at times to the discharges and the

camera having quite a bit off electronics...well it still works.  Probably

should of made sure my tripod was well insulated off of ground....

Ryan Jester

ryanj@primary.net

X-Mailer: IPAD 2.0

Date: Wed, 05 Aug 1998 19:41:10 -0500

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Mike Hammer <mhammer@misslink.net>

Subject: toroid size

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

At 06:12 PM 8/5/98 EDT, you wrote:

>Does anyone now how to determine the best size toroid for the secondary?

There are no rules except as big as possible.

Generally for best output the ROC (radius of curvature)

should be at least the same as the secondary diameter.

The toroid diameter should be as large as your primary

coil can tune to resonance.

The limiting factor is power level. Large toroids

require a correspondingly large power level.

And I dont mean the name plate rating of your transformer.

Just having X number of watts of transformer doesnt

mean you will have a large output power. Once you take

gap loss, cap loss, I2R loss, corona loss, coupling loss

and others into account you may not have much power output.

Reducing these losses is just as good (actually better) than

increasing power level.

For my 4.5 X 21 inch secondary I had good luck with 16 X 4

and 19 X 6 toroids.

One problem with large toroids is that they make tuning difficult.

So when you first bring your coil online you may want to use

a small terminal at first to make tuning easier. A 6 inch diameter

sphere works well for this.

TTYL!

Mike Hammer

mhammer@misslink.net

From: "Adam Parker" <park_e_r@hiwaay.net>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: J ladder & direct 2ndry?

Date: Wed, 5 Aug 1998 20:31:16 -0500

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

I often build my ladders with 18 AWG bare copper wire. Usually I string

them from the cieling with stand-off insulators and for a base used a

single type bolt gap. Works Great!

Adam

X-Mailer: IPAD 2.0

Date: Thu, 06 Aug 1998 00:15:23 -0500

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: vaccuum gap

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

At 10:01 AM 8/5/98 -0500, you wrote:

>I am building a new vaccuum gap using 10 pieces of 3/4" copper tubing

>5" long. Can anyone tell me if this gap will stand up to 3Kw. I have a

>powerful shop-vac to pull the air through the teeth of the gap. The

>first vaccuum gap I built used 6 pieces of 1 1/8" copper tubing spaced

>30mils apart.

>

>

>                               Frankensteins Helper

>                                       Max

Roderick,

Yes I believe a vacuum gap will handle 3kva well.

My design as implemented by Gary Weaver was used

on his 10 inch coil at over 9kva with over 120 inches

of spark output.

This design is a pretty much standard RQ gap with 1/8 inch

slots cut into the PVC housing between the electrodes.

The bottom of the PVC housing is sealed with a disk

of plexiglass. The top also has a plexiglass disk with

a PVC outlet pipe epoxied into it. The vacuum cleaner

hose connects to this outlet pipe with a hose clamp.

Mine uses 3 inch long electrodes from 3/4 inch copper pipe

and gets first notch quenching at up to the highest power

level I've run so far. Thats about 1.5 KVA.

One thing you need to be aware of. Both Gary and myself

used the same surplus vacuum cleaner motor meant for

a shop vac type vacuum cleaner and we found that it pulls

too much air through the gap. The high speed air actually

inhibits gap operation. So we both run ours on a variac

to vary the air speed. You can actually find a point

where the output peaks coinciding with optimal quenching.

Go for it. A vacuum gap is just about the best static

gap there is.

TTYL!

Mike Hammer

mhammer@misslink.net

Date: Wed, 05 Aug 1998 23:13:55 -0500

From: RODERICK MAXWELL <tank@magnolia.net>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: vacuum gap

Sender: tesl-err@emachine.com

Mike Hammer wrote:

>

> At 10:01 AM 8/5/98 -0500, you wrote:

> >I am building a new vacuum gap using 10 pieces of 3/4" copper tubing

> >5" long. Can anyone tell me if this gap will stand up to 3Kw. I have a

> >powerful shop-vac to pull the air through the teeth of the gap. The

> >first vaccuum gap I built used 6 pieces of 1 1/8" copper tubing spaced

> >30mils apart.

> >

> >

> >                               Frankensteins Helper

> >                                       Max

>

> Roderick,

>

> Yes I believe a vacuum gap will handle 3kva well.

> My design as implemented by Gary Weaver was used

> on his 10 inch coil at over 9kva with over 120 inches

> of spark output.

>

> This design is a pretty much standard RQ gap with 1/8 inch

> slots cut into the PVC housing between the electrodes.

> The bottom of the PVC housing is sealed with a disk

> of plexiglass. The top also has a plexiglass disk with

> a PVC outlet pipe epoxied into it. The vacuum cleaner

> hose connects to this outlet pipe with a hose clamp.

>

> Mine uses 3 inch long electrodes from 3/4 inch copper pipe

> and gets first notch quenching at up to the highest power

> level I've run so far. Thats about 1.5 KVA.

>

> One thing you need to be aware of. Both Gary and myself

> used the same surplus vacuum cleaner motor meant for

> a shop vac type vacuum cleaner and we found that it pulls

> too much air through the gap. The high speed air actually

> inhibits gap operation. So we both run ours on a variac

> to vary the air speed. You can actually find a point

> where the output peaks coinciding with optimal quenching.

>

> Go for it. A vacuum gap is just about the best static

> gap there is.

>

> TTYL!

>

> Mike Hammer

> mhammer@misslink.net

Thank you for your reply. I was primarily concerned with overheating

because I'm using smaller tubing in this gap.

Frankensteins Helper

Max

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Date: Thu, 06 Aug 1998 04:18:19 -0700

From: "Fleischwulf" <fleischwulf@mailexcite.com>

X-Sent-Mail: off

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

Subject: Re: vacuum gap

X-Sender-Ip: 208.153.7.9

Organization: MailExcite  (http://www.mailexcite.com:80)

Sender: tesl-err@emachine.com

Max,

I ran 4.5 Kw on a gap just like this. I didn't run into trouble until I hit 7.2 Kw. The gap failed at this power level. I was using a 1.5 or 2 Hp shop vac. Once you go above 5 Kw you will experience problems with heat.

Good luck

Paul M

---

Leben frei oder Starb !

On Thu, 06 Aug 1998 00:15:23   Mike Hammer wrote:

>At 10:01 AM 8/5/98 -0500, you wrote:

>>I am building a new vacuum gap using 10 pieces of 3/4" copper tubing

>>5" long. Can anyone tell me if this gap will stand up to 3Kw. I have a

>>powerful shop-vac to pull the air through the teeth of the gap. The

>>first vaccuum gap I built used 6 pieces of 1 1/8" copper tubing spaced

>>30mils apart.

>>

>>

>>                               Frankensteins Helper

>>                                       Max

>

>

>Roderick,

>

>Yes I believe a vacuum gap will handle 3kva well.

>My design as implemented by Gary Weaver was used

>on his 10 inch coil at over 9kva with over 120 inches

>of spark output.

>

>This design is a pretty much standard RQ gap with 1/8 inch

>slots cut into the PVC housing between the electrodes.

>The bottom of the PVC housing is sealed with a disk

>of plexiglass. The top also has a plexiglass disk with

>a PVC outlet pipe epoxied into it. The vacuum cleaner

>hose connects to this outlet pipe with a hose clamp.

>

>Mine uses 3 inch long electrodes from 3/4 inch copper pipe

>and gets first notch quenching at up to the highest power

>level I've run so far. Thats about 1.5 KVA.

>

>One thing you need to be aware of. Both Gary and myself

>used the same surplus vacuum cleaner motor meant for

>a shop vac type vacuum cleaner and we found that it pulls

>too much air through the gap. The high speed air actually

>inhibits gap operation. So we both run ours on a variac

>to vary the air speed. You can actually find a point

>where the output peaks coinciding with optimal quenching.

>

>Go for it. A vacuum gap is just about the best static

>gap there is.

>

>

>TTYL!

>

>Mike Hammer

>mhammer@misslink.net

>

>

Date: Thu, 06 Aug 1998 15:09:31 +0200

From: Erikzon <jan-erik.eriksson@mbox2.swipnet.se>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: RF Cokes

Sender: tesl-err@emachine.com

Hi

For RF chokes to my 4.33" coil, could I use 2 pieces of 2" (diam.) PVC

tubes wounded with about 150 turns of #24 wire?

//Christopher Eriksson

From: "Kent Rankin" <kentrankin@theatreorgans.com>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: RF chokes

Date: Thu, 6 Aug 1998 09:42:07 -0400

X-Mailer: IPAD 2.0

X-MimeOLE: Produced By Microsoft MimeOLE V4.72.2106.4

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

I have decided to go against what many have told me (that with one

transformer I won't need chokes) and make some RF chokes... I will be making

a coil in the next few days based on a 15kV 60mA tranny.  It will be a 4.5" secondary.

Question is:

How do I design a good set of RF chokes (I have been told that I need two)?

Thanks in advance,

Kent Rankin

Date: Thu, 06 Aug 1998 09:00:25 -0500

From: Bert Hickman <bert.hickman@aquila.com>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Organization: Stoneridge Engineering

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: RF Cokes

Sender: tesl-err@emachine.com

Christopher,

The inductance value for the series chokes is not really very critical.

However, because of the high RF voltages developed across these coils,

you may want to increase the length of each a bit more. 150 turns of

close-wound #24 AWG is about 3 inches of winding length - if you go with

250 turns (about 5 inches) you'll double the inductance (to about 1 mH

each) as well as increasing the voltage standoff capability of these

chokes.

BTW, there's some evidence that using bypass caps (250-500pF from HV

legs to ground) and series power resistors (500-1000 ohms at 50-100

watts depending on your operating power level in each leg from the HV

transformer) may provide better protection for neon sign transformers

than HV inductors or inductors and capacitors alone. The reason seems to

be that there's less chance for high frequency ringing and better

damping due to the added series resistance. Like many areas of coiling,

this area is a somewhat controversial one...

Safe coiling to you!

-- Bert --

Erikzon wrote:

>

> Hi

> For RF chokes to my 4.33" coil, could I use 2 pieces of 2" (diam.) PVC

> tubes wounded with about 150 turns of #24 wire?

> //Christopher Eriksson

Date: Thu, 06 Aug 1998 09:08:42 -0500

From: Bert Hickman <bert.hickman@aquila.com>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Organization: Stoneridge Engineering

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: RF chokes

Sender: tesl-err@emachine.com

Kent,

See my other post to Christopher Eriksson. 200-300 turns of #24-#28 AWG

on 2" or 3" PVC form is sufficient per choke. However, an RC network

may, in fact, be better. BTW, I happen to use 500 pF to ground and 5 mH

chokes in series with each HV leg on my current system. However, using

this configuration I still "fried" one 15 KV 60 MA transformer over the

last year. Based upon some recent analyses done on the main Tesla

Builders' List I'm now considering substituting series resistors for the

inductors to improve protection for the neons...

-- Bert --

Kent Rankin wrote:

>

> I have decided to go against what many have told me (that with one

> transformer I won't need chokes) and make some RF chokes... I will be making

> a coil in the next few days based on a 15kV 60mA tranny.  It will be a 4.5"

> secondary.

>

> Question is:

>

>     How do I design a good set of RF chokes (I have been told that I need two)?

>

>                                         Thanks in advance,

>                                         Kent Rankin

Date: Thu, 06 Aug 1998 09:15:03 -0500

From: RODERICK MAXWELL <tank@magnolia.net>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: vacuum gap

Sender: tesl-err@emachine.com

Tom Polk wrote:

>

> Mike Hammer wrote:

>

> > This design is a pretty much standard RQ gap with 1/8 inch

> > slots cut into the PVC housing between the electrodes.

> > The bottom of the PVC housing is sealed with a disk

> > of plexiglass. The top also has a plexiglass disk with

> > a PVC outlet pipe epoxied into it. The vacuum cleaner

> > hose connects to this outlet pipe with a hose clamp.

>

> Do you have any hints how to create 1/8" slots on the inside of the pvc?

> Also, what diameter pvc tube is it? Thanks.

>

> Please clear up my confusion. First, what is an RQ gap? Rotary Quenched?

> I thought this was a static gap. Please explain. Also, it sounds like

> the vacuum cleaner tries to draw all the air out, i.e. pulling a vacuum,

> without any repenishment of air. Or is there an air inlet, and if so,

> where is it?

>

> Thanks again.

> Tom

If you would like I could send you some pictures of the first vaccuum

gap I designed. It has fewer gaps than the new one I'm working on,

therefore it breaks down at a lower voltage.

Frankensteins Helper

Max

From: "Basura, Brian" <brian.basura@unistudios.com>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: RE: J ladder & direct 2ndry?

Date: Thu, 6 Aug 1998 07:24:11 -0700

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Adam,

What is a "single type bolt gap"?

Thanks,

Brian

> ----------

> From: 	Adam Parker[SMTP:park_e_r@hiwaay.net]

> Reply To: 	Tesla-2 Mailing List

> Sent: 	August 05, 1998 6:31 PM

> To: 	Tesla-2 Mailing List

> Subject: 	Re: J ladder & direct 2ndry?

>

> I often build my ladders with 18 AWG bare copper wire. Usually I string

> them from the cieling with stand-off insulators and for a base used a

> single type bolt gap. Works Great!

>

> Adam

>

Date: Thu, 06 Aug 1998 09:16:18 -0700

From: Gene Waldo <gwaldo@1stnetusa.com>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Organization: S.A.P.I.E.N.T. (Surly, Agressive, Politically Incorrect Enemies of Non Thinkers)

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: math dummy needs hep!

Sender: tesl-err@emachine.com

All,

Quoting some NST-system data/specs: "1200V x 100ma = 1200W."

If VA = W, then why do I get 120W for the above relationship?

AC Ohms Law always did tend to confuse me, some. Impedence, reactance,

reflected inductive reactance, power factories, fuse angle: is Theta = 0 or 90?

Do our archives include Ohms Law for TeslaCases? Something laid out for

quicker comprehension than the deep and esoteric EEngineering exposes I

have been able to dig out?

--------------------------------------

Gene Waldo -  gwaldo@1stnetusa.com  - "Igor in Residence"

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

cc: gwaldo@1stnetusa.com

Subject: Re: math dummy needs hep!

Date: Thu, 06 Aug 1998 11:56:36 CDT

From: "Kent Vander Velden" <graphix@iastate.edu>

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

In message <35C9D6D2.A7C@1stnetusa.com>, Gene Waldo writes:

>All,

>Quoting some NST-system data/specs: "1200V x 100ma = 1200W."

>

>If VA = W, then why do I get 120W for the above relationship?

You are correct - 1200V * 100mA = 120W.

Now, 12000V * 100mA = 1200W.

Did you happen to leave off a zero on the voltage measurement?

From: "Kent Rankin" <kentrankin@theatreorgans.com>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Can anyone lend some ideas towards this coil design?

Date: Thu, 6 Aug 1998 13:04:49 -0400

X-Mailer: IPAD 2.0

X-MimeOLE: Produced By Microsoft MimeOLE V4.72.2106.4

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

I've been listening for a few months to you guys talk on the list, and I've decided to make my first coil within the next few days. Hooray!

I know there are a lot of things that I've surely messed up on.  I layed everything out on paper a few times with a calculator, then I decided to

just use Wintesla.  I suppose that I'm lazy.

I have worked around neon all of my life (my father owns a sign fabrication company). The one joy of the background, is that I've saved up neons for years, and I have about 75 of them in a storeroom, and have a 25,000 square foot

facility for making my coil, along with free polyurethane, plexiglass, and hv wire.  Needless to say, I have the resources,now I just need the plans to do

this well.

Here's what wintesla and I have come up with:

Transformer: 15kV, 60 mA, 60hz, 120V input.

Primary:     10" inside diam., 0.25" wire with 0.25" spacing

18 turns.  0.1663mH inductance

Secondary:   4.5" diameter, 22.5" long, 24awg wire.

H/W ratio of 5:1, 21.96 mH inductance

Secondary freq. of 355.5784Khz

Supposed capacitance of 9.12pF

Needed Cap:  0.0106uF

Toroid:           6" air-conditioning duct with two pie-plates holding

it together.  Supposed capacitance of 16.01pF

Spark Gap:    Not planned

Safety Gap:   Not planned

I suppose that I should now ask as to what the first major failure is in

the planning of this device.

Secondly, I would like to ask what you guys think I should do as far as

advancing the plan.  Mind you, adding 15 more 60mA transformers is not out

of my league, as I have them.  My only problem with a situation like that

is that I cannot afford a rotary spark gap, and I certainly cannot afford

to make the capacitors that such a setup would require.

Although, I might raise the cash for it, if I knew that it would be nice

I just don't have the experience needed, yet).  The main reason for this

is that I'd like to donate one to a local museum that has requested one

from me based upon a few discussions.  They said that they'd even love one

that is just crap and has to be repaired on a weekly basis, as they've

always wanted one for their tour. By the way, the museum is the Naional

Museum of Science and Energy(located in Oak Ridge, TN... site of Oak Ridge National Laboratory).

If anyone has any ideas as to how to make a good, strong tesla coil, but

somehow gear it for public viewing, I'd love to hear them.

Third, what should I do for the caps and gaps?  Saltwater would work, but I'd

like to just go for polyethylene caps from the start.  I'd also like ones that

can withstand upwards of 100kV, as I have a few other experiments in mind.

Thanks in advance,

Kent Rankin

P.S. - I really appreciate how you guys have always helped me with

my supremely advanced stupidity. =)

From: Tom Stock <tomstock@geocities.com>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: RE: RF chokes

Date: Thu, 6 Aug 1998 10:09:29 -0700

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

For my chokes I pulled two ferrits donuts off of the back of some TV picture tubes, and wound them with about 150 turns of #24 wire. They work great, and

were free.

-tom

----------

From: 	Kent Rankin

Sent: 	Thursday, August 06, 1998 6:42 AM

To: 	Tesla-2 Mailing List

Subject: 	RF chokes

I have decided to go against what many have told me (that with one

transformer I won't need chokes) and make some RF chokes... I will be making

a coil in the next few days based on a 15kV 60mA tranny.  It will be a 4.5" secondary.

Question is:

How do I design a good set of RF chokes (I have been told that I need two)?

Thanks in advance,

Kent Rankin

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Capacitors that dont have much to go wrong?

Date: Thu, 6 Aug 1998 18:03:09 +0000

From: Lan <rune@dircon.co.uk>

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

I'm interested in which kind of capactior would not go wrong the easiest. Would

a copper plate and mineral oil be the most robust? As I said already, my

electrolyte one has corroded copper, the wires leading into my transformer have

pulled a bunch of the oil from the capacitor all the way down the wire, its

growing bacteria, it sparks... It just sucks, would copper and oil be the most

robust and easiest to take care of? I dont mind if its capacitance isnt that

huge, and i can line it with PVC to increase the dielectric breakdown, if the

oil breaksdown too easily.

X-Mailer: IPAD 2.0

Date: Thu, 06 Aug 1998 12:18:35 -0500

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: Capacitors that dont have much to go wrong?

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

At 06:03 PM 8/6/98 +0000, you wrote:

>I'm interested in which kind of capacitor would not go wrong the easiest.

>Would a copper plate and mineral oil be the most robust? As I said already,

>my electrolyte one has corroded copper, the wires leading into my transformer >have pulled a bunch of the oil from the capacitor all the way down the wire,

>its growing bacteria, it sparks... It just sucks, would copper and oil be the >most robust and easiest to take care of? I dont mind if its capacitance isnt >that huge, and i can line it with PVC to increase the dielectric breakdown,

>if the oil breaksdown too easily.

>

The problem with an oil only cap is that it will tend to

be quite large. Huge even. You will need wide plate spacing

to get a reasonable voltage rating and this means low capacitance.

So you need large plates and/or a great many plates to get any

meaningful capacitance.

Also if such a cap does breakdown the arc will leave a plume

of conductive carbon in the oil. A couples times and your whole

tank of oil is ruined.

Much better to put polyethylene between the plates and then

place the plates closer together increasing the capacitance.

PVC is not a suitable material for caps. It has a very high

RF dissipation factor and would result in poor performance.

A polyethylene/oil cap is time consuming to build but the results

more than justify the time involved.

TTYL!

Mike Hammer

mhammer@misslink.net

From: "Kent Rankin" <kentrankin@theatreorgans.com>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Capacitors that dont have much to go wrong?

Date: Thu, 6 Aug 1998 13:21:15 -0400

X-Mailer: IPAD 2.0

X-MimeOLE: Produced By Microsoft MimeOLE V4.72.2106.4

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

As in bacteria, or as in corrosion?  I'm just really curious as to

what bacteria would be growing on, when there is only mineral oil and copper

to subsist on.

If, indeed, you mean that it is bacteria, then how do you know so?

What form of a test did you use to conclude such a fact?

Curiously,

Kent Rankin

-----Original Message-----

From: Lan <rune@dircon.co.uk>

To: Tesla-2 Mailing List <tesla-2@emachine.com>

Date: Thursday, August 06, 1998 1:07 PM

Subject: Capacitors that dont have much to go wrong?

I'm interested in which kind of capacitor would not go wrong the easiest.

Would a copper plate and mineral oil be the most robust? As I said already,

my electrolyte one has corroded copper, the wires leading into my transformer

have pulled a bunch of the oil from the capacitor all the way down the wire,

its growing bacteria, it sparks... It just sucks, would copper and oil be the

most robust and easiest to take care of? I dont mind if its capacitance isnt

that huge, and i can line it with PVC to increase the dielectric breakdown, if

the oil breaksdown too easily.

X-Mailer: IPAD 2.0

Date: Thu, 06 Aug 1998 12:35:45 -0500

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: RF chokes

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

At 09:42 AM 8/6/98 -0400, you wrote:

>I have decided to go against what many have told me( that with one

>transformer I won't need chokes) and make some RF chokes... I will be making

>a coil in the next few days based on a 15kV 60mA tranny.  It will be a 4.5" >secondary.

>

>Question is:

>

> How do I design a good set of RF chokes(I have been told that I need two)?

>

>

>                                        Thanks in advance,

>                                        Kent Rankin

>

Kent,

The number of transformers doesnt make any difference. One neon

can die just as surely as more. RF chokes help by knocking down

the level of RF spikes trying to enter your transformer secondary.

The most satisfactory chokes are wound on large ferrite toroids.

Mine are wound with 125 turns of AWG 26 on 2.5 inch diameter cores.

The result is about 2.25 mh.

2-10 mh is about the right range. The value is not critical.

If you cant locate suitable ferrite cores you can wind air

core chokes on 2 inch PVC pipe. Sometimes though these air

core chokes can cause more problems than they solve.

These problems can be minimized by using small guage wire

like AWG 28 and using a damping resistor in series with each choke.

This will lessen the Q of the choke and help damp out any ringing.

A 6 inch winding of AWG 28 on 2 inch PVC will result in about 430

turns and an inductance of about 3.7 mh. Combine this with a

300-1000 ohm series resistor and it should make an effective choke.

Use a power resistor rated at least 10 watts for a 30ma neon.

25 watts may be required for 60ma.

2 chokes are required. One goes into each HV lead.

Do not neglect your safety gap. This is even more important

than chokes. I have run many times without RF chokes but

would never run without a safety gap. More neons die from

simple overvoltage than from RF kickbacks.

TTYL!

Mike Hammer

mhammer@misslink.net

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Capacitors that dont have much to go wrong?

Date: Thu, 6 Aug 1998 18:52:16 +0000

From: Lan <rune@dircon.co.uk>

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Kent Rankin said in Re: Capacitors that dont have much to go wrong? at

06/Aug/1998, Thu 17:21:15.

>        As in bacteria, or as in corrosion?  I'm just really curious as to

>what bacteria would be growing on, when there is only mineral oil and copper

>to subsist on.

its on my salt jar capacitor, and you can see the little blotches of stuff that

appeared out of nowhere, perhaps something fell into my salt solution ;)

I dont know.

X-Mailer: IPAD 2.0

Date: Thu, 06 Aug 1998 14:52:58 -0500

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: Can anyone lend some ideas towards this coil design?

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

At 01:04 PM 8/6/98 -0400, you wrote:

>I've been listening for a few months to you guys talk on the list,

>and I've decided to make my first coil within the next few days. Hooray!

>

<SNIP>

>

>Here's what wintesla and I have come up with:

>

>Transformer: 15kV, 60 mA, 60hz, 120V input.

>Primary:     10" inside diam., 0.25" wire with 0.25" spacing

>                     18 turns.  0.1663mH inductance

If this primary is a solenoid form this is OK. But if you intend

a saucer type primary then than inside diameter is way too large.

It should be about 7 inches in diameter for a 4.5 inch diameter

secondary. Coupling would suffer if made too large.

If its a solenoid then 10 inches is a good diameter.

>Secondary:    4.5" diameter, 22.5" long, 24 awg wire.

>                     H/W ratio of 5:1, 21.96 mH inductance

>                     Secondary freq. of 355.5784Khz

>                     Supposed capacitance of 9.12pF

Sounds good. My secondary like this has a 21 inch winding of AWG 24.

>Needed Cap:  0.0106uF

Correct. But dont be a slave to this value. Considerable variation

from this is fine.

>Toroid:           6" air-conditioning duct with two pie-plates holding it

>                     together. Supposed capacitance of 16.01pF

This will make an effective toroid. But consider starting with

a smaller one until you get your coil in tune. The biggest hurdle

for most new coilers is bringing their coils to resonance. A large

toroid makes it more difficult to find proper tune as it will inhibit

the short sparks your coil will produce when out of tune. Plus a large

toroid may put your secondary out of the tuning range of your primary coil.

For this reason a small spherical terminal makes a good first terminal.

Its sparks easy and is easy to tune.

>Spark Gap:    Not planned

>Safety Gap:   Not planned

>

>

>I suppose that I should now ask as to what the first major failure is in the >planning of this device.

It looks good to me so far.

>

>Secondly, I would like to ask what you guys think I should do as far as

>advancing the plan.  Mind you, adding 15 more 60mA transformers is not out

>of my league, as I have them.  my only problem with a situation like that

>is that I cannot afford a rotary spark gap, and I certainly cannot afford

>to make the capacitors that such a setup would require.

Rarely do neon banks containing more than 4 transformers prove worthwhile.

Once you start getting to that kind of power level you are better off

going to a more robust transformer setup.

You do NOT want a rotary gap for neons. They do much better on a well

designed static gap. See the recent thread on vacuum gaps.

>Third, what should I do for the caps and gaps?  Saltwater would work, but I'd

>like to just go for polyethylene caps from the start.  I'd also like ones that

>can withstand upwards of 100kV, as I have a few other experiments in mind.

Yes I would go straight for the poly caps. You wont regret it.

Building a 100kv cap is going to be difficult. It would require

a large series string. I would think at least 10 caps in series

with each individual cap having 30 mils of poly dielectric.

TTYL!

Mike Hammer

mhammer@misslink.net

From: <Ashpilot@aol.com>

Date: Thu, 6 Aug 1998 19:04:16 EDT

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: toroid size

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

So For my 3.25" secondary could I use a 12x4 inch toroid after tuning?

And If I tuned the coil initially with a 6" sphere would changing to a

toroid require retuning the primary?

Date: Thu, 6 Aug 1998 17:11:47 -0600 (MDT)

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Daniel Lockhart <molly@cmn.net>

Subject: Re: Can anyone lend some ideas towards this coil design?

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

Kent wrote:

>>Secondly, I would like to ask what you guys think I should do as far as

>>advancing the plan.  Mind you, adding 15 more 60mA transformers is not out

>>of my league, as I have them.  my only problem with a situation like that

>>is that I cannot afford a rotary spark gap, and I certainly cannot afford

>>to make the capacitors that such a setup would require.

and Mike replied:

>Rarely do neon banks containing more than 4 transformers prove worthwhile.

>Once you start getting to that kind of power level you are better off

>going to a more robust transformer setup.

Huh. Why is this? I'm disappointed. :-)  Is it more trouble to hook up more

than four-or-so neons than meets the eye (or mind I suppose...), or does

simply adding 25% more current to that amount already not make much

difference?

To me, the idea of connecting multiple neons (four, five or even six) was

attractive. It provided an easy way to add more current, and to a certain

extent, was cheaper than a distribution transformer or the likes.

Please explain more Mike, (or whoever) -- what am I missing?

Daniel Lockhart

Date: Thu, 6 Aug 1998 17:11:50 -0600 (MDT)

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Daniel Lockhart <molly@cmn.net>

Subject: Re: Can anyone lend some ideas towards this coil design?

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

>By the way, the museum is the Naional  Museum of Science and

>Energy (located in Oak Ridge, TN... site of Oak Ridge  National Laboratory)

>

> 					Kent Rankin

Ah! Nice museum there! They also have a great van DeGraf Generator there too...

Daniel Lockhart

Date: Thu, 06 Aug 1998 18:19:34 -0500

From: Bert Hickman <bert.hickman@aquila.com>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Organization: Stoneridge Engineering

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: toroid size

Sender: tesl-err@emachine.com

Ashpilot,

A 12" x 4" toroid should work very well, and will add about 10-11 pF of

topload capacitance as opposed to about 7-8 pF that you had from the 6"

sphere. The higher capacitance will require some primary retuning. If

you were getting good "breakout" with a 6" sphere, you may want to try

going to a larger 5", or even a 6" toroid. Good luck!

-- Bert --

Ashpilot@aol.com wrote:

>

> So For my 3.25" secondary could I use a 12x4 inch toroid after tuning? and If

> I tunned the coild initialy with a 6" sphere would changing to a toroid

> require retuning the primary?

From: "Kent Rankin" <kentrankin@theatreorgans.com>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Can anyone lend some ideas towards this coil design?

Date: Thu, 6 Aug 1998 19:22:45 -0400

X-Mailer: IPAD 2.0

X-MimeOLE: Produced By Microsoft MimeOLE V4.72.2106.4

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

>To me, the idea of connecting multiple neons (four, five or even six) was

>attractive. It provided an easy way to add more current, and to a certain

>extent, was cheaper than a distribution transformer or the likes.

>

>Please explain more Mike, (or whoever) -- what am I missing?

>Daniel Lockhart

Maybe I need to just go and wind my own damned transformer and forget all of

the neons... can anyone speak as to the worth of this?

-Kent Rankin

From: "Kent Rankin" <kentrankin@theatreorgans.com>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Can anyone lend some ideas towards this coil design?

Date: Thu, 6 Aug 1998 19:34:08 -0400

X-Mailer: IPAD 2.0

X-MimeOLE: Produced By Microsoft MimeOLE V4.72.2106.4

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

They have the little one there... the big one is over at the Y-12 facility..

I'm going to have to go see it soon...

-Kent Rankin

-----Original Message-----

From: Daniel Lockhart <molly@cmn.net>

To: Tesla-2 Mailing List <tesla-2@emachine.com>

Date: Thursday, August 06, 1998 7:21 PM

Subject: Re: Can anyone lend some ideas towards this coil design?

>

>

>>By the way, the museum is the Naional  Museum of Science and

>>Energy (located in Oak Ridge, TN... site of Oak Ridge National Laboratory)

>>

>> Kent Rankin

>

>Ah! Nice museum there! They also have a great Van De Graf Generator there too...

>

>

>

>Daniel Lockhart

>

>

Date: Thu, 06 Aug 1998 18:36:02 -0500

From: Tom Polk <tpolk@prismnet.com>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: vacuum gap

Sender: tesl-err@emachine.com

Mike Hammer wrote:

> This design is a pretty much standard RQ gap with 1/8 inch

> slots cut into the PVC housing between the electrodes.

> The bottom of the PVC housing is sealed with a disk

> of plexiglass. The top also has a plexiglass disk with

> a PVC outlet pipe epoxied into it. The vacuum cleaner

> hose connects to this outlet pipe with a hose clamp.

Do you have any hints how to create 1/8" slots on the inside of the pvc?

Also, what diameter pvc tube is it? Thanks.

Please clear up my confusion. First, what is an RQ gap? Rotary Quenched?

I thought this was a static gap. Please explain. Also, it sounds like

the vacuum cleaner tries to draw all the air out, i.e. pulling a vacuum,

without any repenishment of air. Or is there an air inlet, and if so,

where is it?

Thanks again.

Tom

X-Mailer: IPAD 2.0

Date: Thu, 06 Aug 1998 19:18:25 -0500

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: Can anyone lend some ideas towards this coil design?

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

At 05:11 PM 8/6/98 -0600, you wrote:

>Kent wrote:

>

>>>Secondly, I would like to ask what you guys think I should do as far as

>>>advancing the plan.  Mind you, adding 15 more 60mA transformers is not out

>>>of my league, as I have them.  my only problem with a situation like that

>>>is that I cannot afford a rotary spark gap, and I certainly cannot afford

>>>to make the capacitors that such a setup would require.

>

>and Mike replied:

>

>>Rarely do neon banks containing more than 4 transformers prove worthwhile.

>>Once you start getting to that kind of power level you are better off

>>going to a more robust transformer setup.

>

>Huh. Why is this? I'm dissapointed. :-)  Is it more trouble to hook up more

>than four-or-so neons than meets the eye (or mind I suppose...), or does

>simply adding 25% more current to that amount already not make much

>difference?

Mainly because the quality control on neons is very loose.

The voltage on a nominal 15kv rated neon may vary from 13-16kv.

The more neons you add the more the imbalances cause problems.

Plus neons are very inefficient. Large neon banks pull quite

heavy primary currents. You can alleviate this somewhat but

running them in pairs on 240 Volts.

And as you said above modest increases in power do not

increase output much. The power input vs spark output

relationship of Tesla coils is very non linear. Each

step up in spark output takes a bigger and bigger increase

in power. It doesnt take long before a distribution transformer

becomes more practical.

Dont despair though. 3 or 4 15kv/60s is some serious power

and capable of lots of spark. It will take you quite a while

to make the most out of that power level. There is a lot more

to long spark than just raw power.

>

>To me, the idea of connecting multiple neons (four, five or even six) was

>attractive. It provided an easy way to add more current, and to a certain

>extent, was cheaper than a distribution transformer or the likes.

It is more attractive. Just dont get in a rush to push the power

level up too quickly. A pair of 15kv/60s is plenty of power

and should be capable of 6 or 7 feet of spark with a well

designed coil.

TTYL!

Mike Hammer

mhammer@misslink.net

X-Mailer: IPAD 2.0

Date: Thu, 06 Aug 1998 19:26:37 -0500

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: vacuum gap

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

At 06:36 PM 8/6/98 -0500, you wrote:

>Mike Hammer wrote:

>

>> This design is a pretty much standard RQ gap with 1/8 inch

>> slots cut into the PVC housing between the electrodes.

>> The bottom of the PVC housing is sealed with a disk

>> of plexiglass. The top also has a plexiglass disk with

>> a PVC outlet pipe epoxied into it. The vacuum cleaner

>> hose connects to this outlet pipe with a hose clamp.

>

>Do you have any hints how to create 1/8" slots on the inside of the pvc?

>Also, what diameter pvc tube is it? Thanks.

The housing for my gap is 4 inch PVC. I cut slots in the PVC

to admit the airflow by building a jig to hold the PVC and placing

a plastic cutter bit from a Dremel Moto tool in my drill press.

The result is a 1/8 inch wide slot the same length as the electrodes.

>

>Please clear up my confusion. First, what is an RQ gap? Rotary Quenched?

RQ gap means a gap style that was popularized (not invented) by

Richard Quick. Richard was a prominent coiler a few years ago

but has dropped out of circulation due to some personal problems.

It is a cylinder gap with electrodes of hard copper pipe mounted

radially inside a housing of PVC pipe. In the original design the

muffin fan supplied cooling air.

>I thought this was a static gap. Please explain. Also, it sounds like

>the vacuum cleaner tries to draw all the air out, i.e. pulling a vacuum,

>without any repenishment of air. Or is there an air inlet, and if so,

>where is it?

>

It is a static gap. The vacuum cleaner draws air out of the gap.

The inlet is the slots cut into the sides of the housing. This

directs the incoming air directly into the area between the electrodes

where it can do the most good.

TTYL!

Mike Hammer

mhammer@misslink.net

From: <ESchulz531@aol.com>

Date: Thu, 6 Aug 1998 23:48:10 EDT

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Can anyone lend some ideas towards this coil design?  >Transformer<

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Hi,

I have built my own 9kva transformer.  It can be done but it is time

consuming and tedious.  If I could buy a 10kva transformer under $200 I would.

If you want more information on transformer building see me tesla coil page.

Erik Schulz

http://members.aol.com/tesla00/o_index.htm

Date: Thu, 06 Aug 1998 21:51:20 -0700

From: Gene Waldo <gwaldo@1stnetusa.com>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Organization: S.A.P.I.E.N.T. (Surly, Agressive, Politically Incorrect Enemies of Non Thinkers)

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Can anyone lend some ideas towards this coil design?

Sender: tesl-err@emachine.com

Kent Rankin wrote:

>

> I've been listening for a few months to you guys talk on the list, and

> I've decided to make my first coil within the next few days. Hooray!

>

> I know there are a lot of things that I've surely messed up on.  I

> layed everything out on paper a few times with a calculator, then I

> decided to just use Wintesla.  I suppose that I'm lazy.

>

> I have worked around neon all of my life(my father owns a sign

> fabrication company). The one joy of the background, is that I've

> saved up neons for years, and I have about 75 of them in a storeroom,

> and have a 25,000 square foot facility for making my coil, along with

> free polyurethane, plexiglass, and hv wire.  Needless to say, I have

> the resources, now I just need the plans to do this well

>

> Here's what wintesla and I have come up with:

>

SNIPS

Thanks in

> advance,

>                                                         Kent Rankin

>

> P.S. - I really appreciate how you guys have always helped me with

>           my supremely advanced stupidity. =)

Kent,

Advanced stupidity, recognized, is advanced intellect in action.

With the advanced NST resources which you have, would you rather:

a)marry me; b)adopt an elderly orphan; c)consider making some of your

stuff available?

I can't offer much except encouragement, at this point, but it sounds

like you have most of the right suff!

Stay close to the T-2 posts: EXCELLENT materials (T-2 archives) are

there for the browsing! The whole design breakdown is addressed at

several ponts!  Mike and Bert and several others will be of immense

help!

--

Gene Waldo -  gwaldo@1stnetusa.com  - "Igor in Residence"

Date: Fri, 07 Aug 1998 00:30:35 -0500

From: Tom Polk <tpolk@prismnet.com>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: vacuum gap

Sender: tesl-err@emachine.com

RODERICK MAXWELL wrote:

>

> Tom Polk wrote:

> >

> > Mike Hammer wrote:

> >

> > > This design is a pretty much standard RQ gap with 1/8 inch

> > > slots cut into the PVC housing between the electrodes.

> > > The bottom of the PVC housing is sealed with a disk

> > > of plexiglass. The top also has a plexiglass disk with

> > > a PVC outlet pipe epoxied into it. The vacuum cleaner

> > > hose connects to this outlet pipe with a hose clamp.

> >

> > Do you have any hints how to create 1/8" slots on the inside of the pvc?

> > Also, what diameter pvc tube is it? Thanks.

> >

> > Please clear up my confusion. First, what is an RQ gap? Rotary Quenched?

> > I thought this was a static gap. Please explain. Also, it sounds like

> > the vacuum cleaner tries to draw all the air out, i.e. pulling a vacuum,

> > without any repenishment of air. Or is there an air inlet, and if so,

> > where is it?

> >

> > Thanks again.

> > Tom

>

>    If you would like I could send you some pictures of the first vacuum

> gap I designed. It has fewer gaps than the new one I'm working on,

> therefore it breaks down at a lower voltage.

>

>                                Frankensteins Helper

>                                       Max

Sure! Do you mean email some pics? You bet! Thanks.

X-Mailer: IPAD 2.0

Date: Fri, 07 Aug 1998 12:21:23 -0500

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: vacuum gap

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

At 09:15 AM 8/6/98 -0500, you wrote:

>

>   If you would like I could send you some pictures of the first vaccuum

>gap I designed. It has fewer gaps than the new one I'm working on,

>therefore it breaks down at a lower voltage.

>

>

>

>                               Frankensteins Helper

>                                      Max

Roderick,

I think there is enough general interest in vacuum gaps

that you could post a couple pics of yours to the list.

Make them JPGs and keep the size reasonable and send as attachments.

TTYL!

Mike Hammer

mhammer@misslink.net

Date: Fri, 07 Aug 1998 09:24:02 -0500

From: RODERICK MAXWELL <tank@magnolia.net>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: vacuum gap

Sender: tesl-err@emachine.com

Mike Hammer wrote:

>

> At 09:15 AM 8/6/98 -0500, you wrote:

> >

> >   If you would like I could send you some pictures of the first vacuum

> >gap I designed. It has fewer gaps than the new one I'm working on,

> >therefore it breaks down at a lower voltage.

> >

> >

> >

> >                               Frankensteins Helper

> >                                      Max

>

> Roderick,

> I think there is enough general interest in vacuum gaps

> that you could post a couple pics of yours to the list.

> Make them JPGs and keep the size reasonable and send as attachments.

>

> TTYL!

>

> Mike Hammer

> mhammer@misslink.net

Here are a few pictures of my first vacuum gap. If anyone would

like to ask me a question about how it was built ask. The construction

has a few pitfalls that you will have to be aware of or the gap will

short out on the first few seconds it is run.

Frankensteins Helper

Max

Attachment Converted: "C:\MISSING_LINK\EUDORA\Attach\Vac1.jpg"

Attachment Converted: "C:\MISSING_LINK\EUDORA\Attach\Vac2.jpg"

Attachment Converted: "C:\MISSING_LINK\EUDORA\Attach\Vac3.jpg"

Attachment Converted: "C:\MISSING_LINK\EUDORA\Attach\Vac4.jpg"

Editors note. Binary files are not contained in the archives.

Date: Fri, 07 Aug 1998 19:49:09 +0200

From: Erikzon <jan-erik.eriksson@mbox2.swipnet.se>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Homepage! Some off topic but please read..=)

Sender: tesl-err@emachine.com

Dear coilers!

Ive now almost ready with my homepage, it will cover tesla coils and

high voltage. So, all of you who wants to get a link or wants to send me

you coil photos (please do) please mail me!

My email adress: eriksson@mindflow.org

Safe coilin

/Christopher Eriksson

From: <Dsurfr@aol.com>

Date: Fri, 7 Aug 1998 17:22:50 EDT

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: primary shape

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Hey All: I need your input once again. After a couple of answers from Mike & a

few quick tests, I've decided that I should switch to my 12Kv neon instead of

the 15Kv I was using. The cost alone of the poly required for the caps is about

half that required for the 15Kv. 12 Kv neons are cheaper so I can afford to

get another and increase my amperage to 60 Ma. Of course I'll have to build a

bigger cap so the savings will be eaten up, but I'll have 60 Ma to work with

for the same (almost) dollars spent. The spark lengths off my two transformers

(after adjusting the gap, the cap value & the primary tap) seems about the

same, just not as thick or white or loud (but I'm not really close to being

tuned for either of them).

Since I'm going to pretty much rebuild my tank circuit in the process I

decided to follow Dr. Resonance's advice & build some quick "mock-ups" to test

results before commiting to the final(?) design. My current Primary is 18 turns

of #10 copper wire, 1/4" spacing @ 75 degree slope (not helical but almost). I

plan on using 1/4" copper tubing for my next one but I'm debating between a

flat or conical shape. (I've read a lot of postings on the relative values of

both & Terry F's tests seem to lead to the saucer shape being the best?) I

took 50' of TV coax cable (very lossy, I know but it was at hand) & wound a

pancake coil of 17 turns. I noticed that I get longer spark length when I

raise the Primary up about 3" from the bottom turn of the Secondary. Is this

the same effect I would get from a conical coil (which would have the tap

about the same height above) or will the coupling be stronger over all with

the conical coil? Pancake coils seem much easier to wind & I've gathered it's

going to be tough enough working w/ the tubing. Would the advantage of being

able to "raise the primary" (not to start another discussion like "raising the

secondary") be an advantage in tuning a coil w/ a pancake primary? My current

secondary is 4 1/2" O.D. x 22" 24 G. (about 1032 turns - I've often wondered

if I should peel off an inch or so to decrease the number of windings?)

Thanks (sorry for such a long posting), Jim

Date: Fri, 07 Aug 1998 17:22:34 -0500

From: Bert Hickman <bert.hickman@aquila.com>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Organization: Stoneridge Engineering

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: primary shape

Sender: tesl-err@emachine.com

Jim,

As long as you maintain adequate clearance such that you dont get

flashovers from the secondary to the primary, and as long as you can

adjust the coupling coefficient by varying the relative positions of the

primary versus the secondary, it doesn't really matter very much! If you

raise the primary relative to the secondary, you may need to increase

the spacing between the secondary and the innermost primary turn since

the voltage rises quite rapidly as you move up the resonator. It won't

help in tuning, but it will permit you to adjust for best overall coil

performance based upon the quenching capability of your gaps.

However, if you plan on increasing your input power levels in the near

future (as most coilers do...), a shallower 30 or 15 degree saucer or a

pancake coil will allow you to achieve longer streamer lengths, since it

will provide greater physical separation between your toroid and the

primary, and this will tend to reduce the incidence of strike-rail hits.

I'd leave your coil length as it is for now, so that you maintain your

current coil length (again to maximize primary:top-of-secondary

clearance). Fewer turns of larger diameter wire might work better in the

future, particularly if you increase power levels.

Safe coilin' to you, Jim!

-- Bert --

Dsurfr@aol.com wrote:

>

> Hey All: I need your input once again. After a couple of answers from Mike & a

> few quick tests, I've decided that I should switch to my 12Kv neon instead of

> the 15Kv Iwas using. The cost alone of the poly required for the caps is about

> half that required for the 15Kv. 12 Kv neons are cheaper so I can afford to

> get another and increase my amperage to 60 Ma. Of course I'll have to build a

> bigger cap so the savings will be eaten up, but I'll have 60 Ma to work with

> for the same (almost) dollars spent. The spark lengths off my two transformers

> (after adjusting the gap, the cap value & the primary tap) seems about the

> same, just not as thick or white or loud (but I'm not really close to being

> tuned for either of them).

>

> Since I'm going to pretty much rebuild my tank circuit in the process I

> decided to follow Dr. Resonance's advice & build some quick "mock-ups" to test

> results before commiting to the final? design. My current Primary is 18 turns

> of #10 copper wire, 1/4" spacing @ 75 degree slope (not helical but almost). I

> plan on using 1/4" copper tubing for my next one but I'm debating between a

> flat or conical shape. (I've read a lot of postings on the relative values of

> both & Terry F's tests seem to lead to the saucer shape being the best?) I

> took 50' of TV coax cable (very lossy, I know but it was at hand) & wound a

> pancake coil of 17 turns. I noticed that I get longer spark length when I

> raise the Primary up about 3" from the bottom turn of the Secondary. Is this

> the same effect I would get from a conical coil (which would have the tap

> about the same height above) or will the coupling be stronger over all with

> the conical coil? Pancake coils seem much easier to wind & I've gathered it's

> going to be tough enough working w/ the tubing. Would the advantage of being

> able to "raise the primary" (not to start another discussion like "raising the

> secondary") be an advantage in tuning a coil w/ a pancake primary? My current

> secondary is 4 1/2" O.D. x 22" 24 G. (about 1032 turns - I've often wondered

> if I should peel off an inch or so to decrease the number of windings?)

> Thanks (sorry for such a long posting), Jim

Date: Sat, 08 Aug 1998 07:41:36 -0700

From: Gene Waldo <gwaldo@1stnetusa.com>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Organization: S.A.P.I.E.N.T. (Surly, Agressive, Politically Incorrect Enemies of Non Thinkers)

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Picture of spark discharge

Sender: tesl-err@emachine.com

Ryan E. Jester wrote:

>

> -----Original Message-----

> From: Gene Waldo <gwaldo@1stnetusa.com>

> To: Tesla-2 Mailing List <tesla-2@emachine.com>

> Date: Wednesday, August 05, 1998 11:39 AM

> Subject: Re: Picture of spark discharge

>

> >Ryan,

> >Nice pics! Digital or analog camera?

> >

> >I'm guessing analog; scanned print? scanner res?

>

> I used a Minolta 350si camera, 400ASA  One thing I was thinking

> about was I had that camera pretty close at times to the discharges and the

> camera having quite a bit off electronics...well it still works.  Probably

> should of made sure my tripod was well insulated off of ground....

>

> Ryan Jester

> ryanj@primary.net

Ryan, et al,

Since the factors involved in RF induced camera failure remain

unfocused, and no one (to my knowledge) has reported a direct failure,

cause & effects, I should immagine that some sort of small F. Cage or

other klutzy apparatus, w/direct connection to our RFGnd, applied at or

around the camera, would be almost worth the effort.

Since the above is in reality, a question, can we discuss some of the

practical aspects of the answer?

While the nagging fear of loss seems under-weighed by the successes, A

stitch in time...

-------------------------------------------

Gene Waldo -  gwaldo@1stnetusa.com  - "Igor in Residence"

From: <Dsurfr@aol.com>

Date: Sat, 8 Aug 1998 15:37:02 EDT

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: cheap ceramic caps?

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

A friend offered me a whole bunch (100+) of small ceramic caps. The bag was

labeled .001 uf 10 Kv axial ceramic disc capacitor  Matsushita # ECK-D4A102MDA

Would it be possible to wire these in series/parallel for use on a Tesla coil?

Thanks, Jim

X-Mailer: IPAD 2.0

Date: Sat, 08 Aug 1998 15:06:17 -0500

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: cheap ceramic caps?

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

At 03:37 PM 8/8/98 EDT, you wrote:

>A friend offered me a whole bunch (100+) of small ceramic caps. The bag was

>labeled .001 uf 10 Kv axial ceramic disc capacitor  Matsushita # ECK-D4A102MDA

>Would it be possible to wire these in series/parallel for use on a Tesla coil?

>Thanks, Jim

>

Jim,

While it would be possible the results will be poor.

The array of caps needed would add considerable inductance

and the ceramic dielectric is very lossy. The caps will

tend to overheat quickly and can self destruct.

Bear in mind that the 10kv rating is a DC rating.

Cut that in half for AC and derate another 25 percent

for pulse duty. That gives you caps only rated 3750 VAC.

As you can see the array begins to assume large proportions

to get any meaningful capacitance at a decent voltage rating.

If they are cheap (free) then experiment away. But dont spend

any money on them.

TTYL!

Mike Hammer

mhammer@misslink.net

From: "Choad" <choad69@geocities.com>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: cheap ceramic caps?

Date: Sat, 8 Aug 1998 15:19:48 -0500

X-Mailer: IPAD 2.0

X-MimeOLE: Produced By Microsoft MimeOLE V4.72.3110.3

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Jim,

The 10Kv rating is most likely a dc volt rating. I wouldn't use ceramic

caps though. They will get hot after a while. You can make some good rolled

poly caps cheaply and they will last a long time.

Choad

>A friend offered me a whole bunch (100+) of small ceramic caps. The bag was >labeled .001 uf 10 Kv axial ceramic disc capacitor  Matsushita # ECK-D4A102MDA >Would it be possible to wire these in series/parallel for use on a Tesla coil?

>Thanks, Jim

Date: Sat, 8 Aug 1998 17:17:27 -0600 (MDT)

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Daniel Lockhart <molly@cmn.net>

Subject: Re: cheap ceramic caps?

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

Jim wrote:

>>A friend offered me a whole bunch (100+) of small ceramic caps. The bag was

>>labeled .001 uf 10 Kv axial ceramic disc capacitor Matsushita #ECK-D4A102MDA

>>Would it be possible to wire these in series/parallel for use on a Tesla

>>coil?

and Mike replied:

>Jim,

>While it would be possible the results will be poor.

>The array of caps need would add considerable inductance

>and the ceramic dielectric is very lossy. The caps will

>tend to overheat quickly and can self destruct.

>

>Bear in mind that the 10kv rating is a DC rating.

>Cut that in half for AC and derate another 25 percent

>for pulse duty. That gives you caps only rated 3750 VAC.

>

>As you can see the array begins to assume large proportions

>to get any meaningful capacitance at a decent voltage rating.

>

>If they are cheap (free) then experiment away. But dont spend

>any money on them.

and Choad added:

>The 10Kv rating is most likely a dc volt rating.  I wouldn't use ceramic

>caps though. They will get hot after a while. You can make some good rolled

>poly caps cheaply and they will last a long time.

While they may be not so useful for Tesla coils, they are useful for other

non-Tesla related high voltage gadgets such as voltage multipliers and Marx

Generators.

I would get them in a heartbeat.

My two picofarads,

Daniel Lockhart

Date: Sun, 09 Aug 1998 22:43:35 +0200

From: Erikzon <jan-erik.eriksson@mbox2.swipnet.se>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: HV Resistors

Sender: tesl-err@emachine.com

Hi

Ive made two RF chokes with about 5" of 24AWG. But, now Ill need some

HV resistors, and here in sweden its not very easy to get, so can you

make those by yourself? I thought about that watertube resistor, will it

do it? ot do you got some other suggestions?

Thanks,

Christopher Eriksson

Date: Sun, 09 Aug 1998 16:05:27 -0500

From: Bert Hickman <bert.hickman@aquila.com>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Organization: Stoneridge Engineering

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: HV Resistors

Sender: tesl-err@emachine.com

Erikzon wrote:

>

> Hi

> Ive made two RF chokes with about 5" of 24AWG. But, now Ill need some

> HV resistors, and here in sweden its not very easy to get, so can you

> make those by yourself? I thought about that watertube resistor, will it

> do it? ot do you got some other suggestions?

> Thanks,

> Christopher Eriksson

Christopher,

Actually, plain old inductively-wound wirewound power resistors will

work just fine, since they will tend to "look" just like lossy

inductors. Matching resistors should be used on each HV leg, and each

should be in the range of 1000 to 2000 ohms. However, you want to use at

least a 50 watt size (or 100 watt if you are running more than 60 MA or

running off a 15 KV source) since these will be physically large enough

to withstand the high voltage spikes that will be developed across them.

It's these spikes that would otherwise get back into your NST's.

These standard power resistors should be a bit easier to find than true

HV resistors.

Hope this helped, and good luck!

-- Bert --

X-Mailer: IPAD 2.0

Date: Sun, 09 Aug 1998 16:33:45 -0500

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Mike Hammer <mhammer@misslink.net>

Subject: Tesla IRC channel

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Hello all,

Some of you know about the IRC channel Ive run

on Talkcity for the past year and a half. Its been

quite some time since I mentioned it on the list so

some of the newer subscribers may not be aware of it.

Recent changes at Talkcity have made it necessary to

move the channel to another network. We believe we have

found a home at XNET.

I have also given considerable thought about closing

the channel down. It does take a considerable amount

of my time that could be used for more productive endeavors.

So I would like to guage the interest there is in a IRC

discussion group. How many are truly interested in participating

in such a group? Probably best to email me directly but I

will entertain some discussion of it on the list if desired.

TTYL!

Mike Hammer

mhammer@misslink.net

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Tesla IRC channel

Date: Sun, 9 Aug 1998 22:46:22 +0000

From: Lan <rune@dircon.co.uk>

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

>So I would like to guage the interest there is in a IRC

>discussion group. How many are truly interested in participating

>in such a group? Probably best to email me directly but I

>will entertain some discussion of it on the list if desired.

I would be interested if I could find an IRC client that didnt suck. I run a

hotline server that has tesla and science related files on it, and you can talk

and it much easier than on IRC, and the programs interface doesnt suck as much

as IRC clients do, and there is a news post thing to leave messages to people.

i dont think I will get everyone going there ;) especially seeing as i run the

server off my modem so it can only be up certain times. If anyone is interested

you can log in, its called "nothingness". to get hotline you need to go to

their web site at http://www.hotlinesw.com

From: Tom Stock <tomstock@geocities.com>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: RE: Tesla IRC channel

Date: Sun, 9 Aug 1998 18:03:06 -0400

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

Hmm...well, I've been using mIRC and as far as I can tell it works fine and doesn't suck. It's free at www.mirc.com

-tom

>I would be interested if I could find an IRC client that didnt suck. I run a

>hotline server that has tesla and science related files on it, and you can t>alk

Date: Sun, 09 Aug 1998 16:34:21 -0700

From: Gene Waldo <gwaldo@1stnetusa.com>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Organization: S.A.P.I.E.N.T. (Surly, Agressive, Politically Incorrect Enemies of Non Thinkers)

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Tesla IRC channel

Sender: tesl-err@emachine.com

Mike Hammer wrote:

>

> Hello all,

>

> Some of you know about the IRC channel Ive run

> on Talkcity for the past year

> We believe we have found a home at XNET.

> So I would like to guage the interest there is in a IRC

> discussion group. How many are truly interested in participating

> in such a group?

>

> TTYL!

>

> Mike Hammer

> mhammer@misslink.net

Mike,

I seldom like to seem dumber than I am, but here goes!

What's an IRC?

What was TALKCITY?

Where is XNET?

Will I have to DL a bunch of 'plug-ins' or some such, that my server

can't seem to provide?

"... knows of no such plug-in for your application." HaHaHA!

What happens on this channel? Will I get a kick out of it? Will someone

show me the way? Game for most new stuff!

Still a newbie after 5 months....

-----------------------------------------

Gene Waldo -  gwaldo@1stnetusa.com  - "Igor in Residence"

X-Mailer: IPAD 2.0

Date: Sun, 09 Aug 1998 19:54:43 -0400

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Chris Wendorff <chrisw@kellnet.com>

Subject: RE: Tesla IRC channel

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Hi all,

I have been using mIRC for around 3 years and it has been the IRC client I

have seen. I drop by #tesla every once in a while and I would like to see

the channel continue, no matter what server it happens to be on. I haven't

dealt with the server that Mike mentioned but some of the more popular ones

such as DALnet, EFnet, and Undernet seem to work pretty well if that would

be an option for #tesla.

Just my forty-two cents,

Chris Wendorff

At 06:03 PM 8/9/98 -0400, you wrote:

>

>Hmm...well, I've been using mIRC and as far as I can tell it works fine

and doesn't suck. It's free at www.mirc.com

>

>-tom

>

>

>

>>I would be interested if I could find an IRC client that didnt suck. I run a

>>hotline server that has tesla and science related files on it, and you

>>can talk

>

>

From: <ESchulz531@aol.com>

Date: Sun, 9 Aug 1998 21:06:58 EDT

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Using mIRC to connect to the tesla coil mailing list.

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Hi,

I made an instructional text for anyone that doesn't know what or how to use

an IRC client.  Its made to get you to the tesla coil chat room.

http://members.aol.com/tesla00/mirc.htm

Erik Schulz

From: "Choad" <choad69@geocities.com>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: synchronous motor for a gap

Date: Mon, 10 Aug 1998 01:15:52 -0500

X-Mailer: IPAD 2.0

X-MimeOLE: Produced By Microsoft MimeOLE V4.72.3110.3

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Hello everyone,

I'm looking for a synchronous motor to make a sync rotory gap with and I

can't find any good motor anywhere I look. If anyone can help me find one

that'd be great, thanks

Choad

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Date: Mon, 10 Aug 1998 03:51:17 -0700

From: "Fleischwulf" <fleischwulf@mailexcite.com>

Cc:

X-Sent-Mail: off

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

Subject: Re: HV Resistors

X-Sender-Ip: 208.153.7.9

Organization: MailExcite  (http://www.mailexcite.com:80)

Sender: tesl-err@emachine.com

Chris,

Tell me what you need and I'll try and get it here for you in the states. They only cost a few bucks. I might even have some. The only thing I ask is to for you to pay the postage.

Paul M

---

Leben frei oder Starb !

On Sun, 09 Aug 1998 22:43:35   Erikzon wrote:

>Hi

>Ive made two RF chokes with about 5" of 24AWG. But, now Ill need some

>HV resistors, and here in sweden it=B4s not very easy to get, so can you

>make those by yourself? I thought about that watertube resistor, will it

>do it? or do you got some other suggestions?

>Thanks,

>Christopher Eriksson

>

From: Jesper Olsson <jesperol@algonet.se>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Date: Mon, 10 Aug 1998 21:51:54 +0100

X-Mailer: IPAD 2.0

Organization: Algonet Sweden

Subject: Re: HV-resistors

Sender: tesl-err@emachine.com

Hi!

~~~

I was thinking about connecting about 40 .6W 25ohm metal film

resistors in series and use as 1kohm HV-resistors in series

with my airwound RF-chokes. I would cut the legs of the individual

resistors as short as possible and solder them into a long string.

I would then insert the string in a piece of PVC tubing for

insulation and wind the whole lot around a bit of 2" PVC pipe.

As each resistor can handle ~500V @ .6W the whole chain should

be able to handle ~20kV @ 24W.

A such resistor chain would be very cheap as both the ordinary

resistors and the PVC tubing are virtually free.

Will this work? Are there any flaws in this design?

/Jesper Olsson

From: "Basura, Brian" <brian.basura@unistudios.com>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Current Transformers ?

Date: Mon, 10 Aug 1998 15:42:29 -0700

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Hello,

While scrounging around at my local utility scrap-yard I came across a

couple of transformers.  They were marked as 60/120:1 current transformers.

What is the difference between these and a regular transformer?  I know

Brent Turner's book says to steer clear of them but doesn't give any

explanation why?

Their construction seemed to be backward from a regular transformer.  The HV

windings were huge 1"x1/4" and the LV windings looked to be about 10AWG.

Thanks,

Brian D. Basura

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: longest time running a tesla coil

Date: Tue, 11 Aug 1998 00:01:07 +0100

From: Lan <rune@dircon.co.uk>

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Has anyone run a tesla coil for a long time without anything burning out,

overheating, etc etc? I would be interested in that. Especially what features

would make a long duration worthy tesla coil.

Date: Mon, 10 Aug 1998 22:52:25 -0500

From: Bert Hickman <bert.hickman@aquila.com>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Organization: Stoneridge Engineering

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Current Transformers ?

Sender: tesl-err@emachine.com

Basura, Brian wrote:

>

> Hello,

>

> While scrounging around at my local utility scrap-yard I came across a

> couple of transformers.  They were marked as 60/120:1 current transformers.

> What is the difference between these and a regular transformer?  I know

> Brent Turner's book says to steer clear of them but doesn't give any

> explanation why?

>

> Their construction seemed to be backward from a regular transformer.  The HV

> windings were hugh 1"x1/4" and the LV windings looked to be about 10AWG.

>

> Thanks,

>

> Brian D. Basura

Brian,

These are specially constructed metering transformers that typically

consist of a single-turn primary (sometimes just a single,

large-diameter straight conductor, not even a "loop" of any kind)

surrounded by a multiple-turn secondary coil. This configuration is sort

of like a step-down transformer for current instead of voltage. This

configuration will use a relatively low range full-scale range ammeter

(typically 5A Full-Scale) on the secondary side. The full-scale meter

face of the ammeter is "re-scaled" to reflect the ratio of the

transformer.

In the case of your 60:1 current transformer, a 5A full-scale meter

would be scaled to read 0-300A or 0-600A for the 120:1 ratio. Because 5A

meters are very often used, most current transformers will come in

ratios of XXX:5. Also, many consist only of the secondary winding in the

shape of a large ring that can be slipped over a bus-bar, or where the

primary can be a large wire that can be looped through the transformer.

Other than for remote current metering (for example for power

transformer primary current), these rally have little other practical

use in coiling.

-- Bert --

From: "Adam Parker" <park_e_r@hiwaay.net>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: J ladder & direct 2ndry?

Date: Mon, 10 Aug 1998 22:53:35 -0500

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

What I refered to as a single type bolt gap is actually a safety gap i

built that consists of two bolts as electrodes and a U shaped wooden base.

I connect my ladder wire to the ends of the bolts.

Adam

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Date: Tue, 11 Aug 1998 03:56:12 -0700

From: "Fleischwulf" <fleischwulf@mailexcite.com>

Cc:

X-Sent-Mail: off

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

Subject: Re: longest time running a tesla coil

X-Sender-Ip: 208.153.7.9

Organization: MailExcite  (http://www.mailexcite.com:80)

Sender: tesl-err@emachine.com

Lan,

The longest power on time that I have used is about 1 minute.(4.5 Kw.) This was not just to protect the coil but to protect the objects the streamers were striking. At low power levels 1 Kw or less I would think run time could be extended to the 10's of minutes. The gap would have to have a good supply of cooling air.

Paul M

---

Leben frei oder Starb !

On Tue, 11 Aug 1998 00:01:07   Lan wrote:

>Has anyone run a tesla coil for a long time without anything burning >out, overheating, etc etc? I would be interested in that. Especially >what features would make a long duration worthy tesla coil.

>

>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Date: Tue, 11 Aug 1998 04:01:33 -0700

From: "Fleischwulf" <fleischwulf@mailexcite.com>

Cc:

X-Sent-Mail: off

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

Subject: Re: HV Resistors

X-Sender-Ip: 208.153.7.9

Organization: MailExcite  (http://www.mailexcite.com:80)

Sender: tesl-err@emachine.com

Erik,

I have had some time to think about this. I am using 2 ceramic resistors

in a tank circuit at approximatly 30 Kv. This application is not Tesla related but I am sure that it would work. I simply placed the resistors

in a plastic bowel and covered them in paraffin. They work very well.

Just make sure that the wattage is sufficient to handle the current.

Good luck

Paul M

---

Leben frei oder Starb !

On Sun, 09 Aug 1998 22:43:35   Erikzon wrote:

>Hi

>Ive made two RF chokes with about 5" of 24AWG. But, now Ill need some

>HV resistors, and here in sweden its not very easy to get, so can you

>make those by yourself? I thought about that watertube resistor, will it

>do it? or do you got some other suggestions?

>Thanks,

>Christopher Eriksson

>

X-Mailer: IPAD 2.0

Date: Tue, 11 Aug 1998 17:17:06 +0300

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Marco Denicolai <marco@vistacom.fi>

Subject: Re: Homepage! Some off topic but please read..=)

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

At 19:49 07/08/98 +0200, you wrote:

>Dear coilers!

>Ive now almost ready with my homepage, it will cover tesla coils and

>high voltage. So, all of you who wants to get a link or wants to send me

>you coil photos (please do) please mail me!

>My email adress: eriksson@mindflow.org

>Safe coilin

>/Christopher Eriksson

>

>

Grab some of my photos if you want from

http://vci.vistacom.fi/~marco/mytesla.html

Ciao!

Marco Denicolai

X-Mailer: IPAD 2.0

Date: Tue, 11 Aug 1998 17:38:31 +0300

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Marco Denicolai <marco@vistacom.fi>

Subject: Re: HV Resistors

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

At 22:43 09/08/98 +0200, you wrote:

>Hi

>Ive made two RF chokes with about 5" of 24AWG. But, now Ill need some

>HV resistors, and here in sweden its not very easy to get, so can you

>make those by yourself? I thought about that watertube resistor, will it

>do it? or do you got some other suggestions?

>Thanks,

>Christopher Eriksson

>

I made by myself a 250 Kohm water resistor capable of standing 50W.

That took me about 2 hours and costs nothing. There is a WWW site with

some formulas bout it but they are incomplete and finally your test-and

-try is always equired.

Basically you start from this data:

- copper pipe size (diameter) you can get

- flexible hose size (diameter) you can fit to the copper pipe

- space available in your design for the finished resistor

I used two pieces of copper tube diameter 1 cm, about 3 cm long. I compressed one of their sides, welded with a torch some tin to tight

them and made a hole in the flattened part of them. These are the

resistor water-tight terminals.

Then I cut a 10 cm, 1 cm diameter flexible hose piece and arranged in a big

dish some usual fresh water. Then I worked under water, filled the hose

and mounted the two copper terminals taking care to have almost no air

bubbles trapped inside.

Then you measure the resistance: if it is too low, you have better to start

from distilled water, otherwise carefully add say a 10 drops of copper

sulphate in your water and fill-measure again.

You don't usually need EXACTLY a certain resistance and with a few try you'll find almost the correct value. The bigger the resistor, the more

it can dissipate heat.

When you are satisfied, just add two hose rings (what they are in english,

I don't know!) and screw them tight to avoid leakage.

Wanna try?

Marco Denicolai

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: longest time running a tesla coil

Date: Tue, 11 Aug 1998 18:15:19 +0100

From: Lan <rune@dircon.co.uk>

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Fleischwulf said in Re: longest time running a tesla coil at 11/Aug/1998, Tue 11:56:12.

>Lan,

> The longest power on time that I have used is about 1 minute.(4.5 Kw.)

1 minute????

Hasnt anyone (Tesla?) run them for a little longer, I need this information

because I have almost got my tesla coil made, its in the garden now, and wanted to know what would make a tesla coil capable of running for longer. I'm sure Tesla if he had thought his energy distribution systems usable,

and even his building lighting system usable, would have run his coils continuously. could anyone else give more information on this?

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: listserve bug

Date: Tue, 11 Aug 1998 18:25:52 +0100

From: Lan <rune@dircon.co.uk>

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

your listserve sends me a failed delivery message everytime I send to it,

it also "gets through" everytime as well, so it is kind of strange behaviour. Were you aware of this?

X-Mailer: IPAD 2.0

Date: Tue, 11 Aug 1998 12:45:29 -0500

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: listserve bug

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

At 06:25 PM 8/11/98 +0100, you wrote:

>your listserve sends me a failed delivery message everytime I send to it,

>it also "gets through" everytime as well, so it is kind of strange

>behaviour. Were you aware of this?

Rune and all,

Ignore any such error messages. Some ISPs route error messages

back to the original poster rather than to the list error

address. We are aware of it.

Since the list server echoes your post back you can be sure

your post went out the list once you receive it.

TTYL!

Mike Hammer

mhammer@misslink.net

X-Mailer: IPAD 2.0

Date: Tue, 11 Aug 1998 12:53:16 -0500

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: Unsubscribe

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

At 08:04 AM 8/11/98 -0700, you wrote:

>Sir:

>    I would like to remove my name from your subscription list, although

>I have enjoyed reading the mail, I find that I do not have the time to

>devote to the projects.

>                                Thank You, Bill Moran

>

>

Bill and all,

Unsubscribe requests should not be sent to the list.

Instructions for unsubscribing yourself were supplied

in the list server instructions I sent to you when you

first subscribed.

I know, no one ever keeps those files. But you should

keep those files around for when you need to unsubscribe

or suspend your subscription for vacation.

I will repost the pertinent information here.

Please make note of it and keep for future reference.

To quit the list, either permanently or temporarily, such as for vacation,

please send a message to tesla-2-request@emachine.com with one of the

following keywords in the body of the message:  Delete, Remove or

Unsubscribe.  Case is not important.

If you should later wish to rejoin the list, please send a message to

tesla-2-request@emachine.com with one of the following keywords in the

body of the message:  Join, Add or Subscribe.  Case is not important.

One additional note. If you unsubscribe and then try to resubscribe

yourself later on it must be with the same email address. If

you change addresses then the list server sees you as a new subscriber.

New subs still need to go through me.

If you need to change your address email me with the change

rather than unsubscribing. If you have difficulty with unsubscribing

email me direct. Do not post requests to the list.

TTYL!

Mike Hammer

mhammer@misslink.net

X-Mailer: IPAD 2.0

Date: Tue, 11 Aug 1998 13:05:05 -0500

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: longest time running a tesla coil

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

At 06:15 PM 8/11/98 +0100, you wrote:

>Fleischwulf said in Re: longest time running a tesla coil at 11/Aug/1998, Tue

>11:56:12.

>

>>Lan,

>> The longest power on time that I have used is about 1 minute.(4.5 Kw.)

>

>1 minute???

There are many factors governing maximum run times. Gap heating

is probably the biggest factor. Other loss factors like dielectric

heating in your cap and I2R losses in the primary circuit also

necessitate short run times.

Neons also can fail with long run times due to coronal heating

in their secondary windings. This corona will erode the insulation

until a failure occurs.

If you are using glass bottle caps then forget long run times.

If you need long run times for some reason the best way is to

run your coil at greatly reduced power. Using much narrower

gaps will limit primary energy level. Use plenty of forced air

to keep the gap cool. Use low loss poly caps.

My normal run times are usually 30 seconds. I sometimes run as long

as 1 minute when taking photos.

>

>Hasnt anyone (Tesla?) run them for a little longer, I need this

>information because I have almost got my tesla coil made, its in

>the garden now, and >wanted to know what would make a tesla coil

>capable of running for longer.

>I'm sure Tesla if he had thought his energy distribution systems usable, >and even his building lighting system usable, would have run his coils continuously. could anyone else give more information on this?

Tesla never ran his Colorado Springs coil at full power for more

than a few seconds at a time. His caps were glass bottle SW caps

and often failed. He also feared setting fire to the building

from the streamers. The many photos showing massive amounts

of streamers were actually long exposures with the coil being turned

on many times a few seconds at a time.

TTYL!

Mike Hammer

mhammer@misslink.net

X-Mailer: IPAD 2.0

Date: Tue, 11 Aug 1998 13:12:41 -0500

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: Tesla IRC channel

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

All,

Well I guess I overestimated the need for a

IRC discussion channel. Im greatly underwhelmed

by the lukewarm response my inquiry produced.

Out of nearly 200 subscribers only 1 (Gene Waldo)

expressed any interest. Except for Mr. Landen who thinks

all IRC clients "suck". I did receive some comments

from a couple channel regulars but not much.

So if the need and the interest is not there

I will probably discontinue the discussion channel.

TTYL!

Mike Hammer

mhammer@misslink.net

From: <Dsurfr@aol.com>

Date: Tue, 11 Aug 1998 16:37:36 EDT

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: cap dielectric

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Hey All: Still trying to build that perfect cap for next to no money.

I was going over a list of dielectric strengths & puncture voltages/Mil

of various materials & noticed that Polyethelene Terphthalate (Mylar)

has a K of 3.0 - 3.1 & a puncture voltage of 7500. I'm an architect &

before everything got computerized we used to draft on sheets of mylar

all the time. My office still has a bunch in an old drawer that I could

have for nothing. I haven't measured its thickness but would it be an acceptable substitute for HDPE?  Any advice (short of try it & let me know) will be appreciated.

Thanks, Jim

X-Mailer: IPAD 2.0

Date: Tue, 11 Aug 1998 15:46:43 -0500

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: cap dielectric

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

At 04:37 PM 8/11/98 EDT, you wrote:

>Hey All: Still trying to build that perfect cap for next to no money.

>I was going over a list of dielectric strengths & puncture voltages/Mil

>of various materials & noticed that Polyethelene Terphthalate (Mylar)

>has a K of 3.0 - 3.1 & a puncture voltage of 7500. I'm an architect &

>before everything got computerized we used to draft on sheets of mylar

>all the time. My office still has a bunch in an old drawer that I could

>have for nothing. I haven't measured its thickness but would it be an

>acceptable substitute for HDPE? Any advice (short of try it & let me know)

>will be appreciated.

>Thanks, Jim

Sorry Jim but you know the old saying "you dont get something for nothing"?

Mylar (PET) has good voltage characteristics and a better dielectric

constant than poly but has a terrible RF dissipation factor.

So bad that mylar caps subjected to RF voltages will likely

melt the dielectric.

Mylar makes very good DC caps but is pretty much useless for RF.

TTYL!

Mike Hammer

mhammer@misslink.net

Comments: Authenticated sender is <megavolt@mail.pcisys.net>

From: megavolt@mail.pcisys.net

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Date: Tue, 11 Aug 1998 20:52:40 +0000

Subject: Re: longest time running a tesla coil

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Lan,

I have run as long as 20 minutes on the 4th of July at 1800 watts and

eventually would  trip a breaker.  Nothing got warm but the gaps and

the resistors.  Since adding PFC capacirance I can run that long

without tripping a breaker.  The ozone gets so thick that I have to

shut down and open the front door of my apartment.

Kent

megavolt@pcisys.net

Comments: Authenticated sender is <megavolt@mail.pcisys.net>

From: megavolt@mail.pcisys.net

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Date: Tue, 11 Aug 1998 20:59:07 +0000

Subject: Re: Spark gap PVC

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Pat,

Did you find any PVC for your SG?  I have two 19 or so inch  pieces of

green 6.25" that you can have for the shipping.  I used it for my

caps, sparkgap and the secondary.

Kent

megavolt@pcisys.net

Date: Tue, 11 Aug 1998 18:56:02 -0500

From: Tom Polk <tpolk@prismnet.com>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: cap dielectric

Sender: tesl-err@emachine.com

Dsurfr@aol.com wrote:

>...  noticed that Polyethelene Terphthalate (Mylar) has a K of 3.0 -

>3.1 & a puncture voltage of 7500. I'm an architect & before everything got

>computerized we used to draft on sheets of mylar all the time. My office >still has a bunch in an old drawer that I could have for nothing. I haven't

>measured its thickness but would it be an acceptable substitute for HDPE?

>Any advice (short of try it & let me know) will be appreciated.

Four years ago I bought some capacitor grade mylar in an attempt to

reduce the size of my caps. The mylar quickly burned through. I had the

same experience with flexible pvc showerpan material while my workhorse

polyethelene is still in operation today.

With high hopes I spent time and bucks on that stuff and experienced my

own disappointment. There was just no source I could find that said,

"Don't use these materials." This listserver would have been very

helpful to me back then, and I am very appreciative of what Mike and his

crew are doing.

Date: Tue, 11 Aug 1998 20:52:46 -0500

From: Bert Hickman <bert.hickman@aquila.com>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Organization: Stoneridge Engineering

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: cap dielectric

Sender: tesl-err@emachine.com

Dsurfr@aol.com wrote:

>

>Hey All: Still trying to build that perfect cap for next to no money. I was

>going over a list of dielectric strengths & puncture voltages/Mil of various

>materials & noticed that Polyethelene Terphthalate (Mylar) has a K of 3.0 -

>3.1 & a puncture voltage of 7500. I'm an architect & before everything got

>computerized we used to draft on sheets of mylar all the time. My office

>still has a bunch in an old drawer that I could have for nothing. I haven't

>measured its thickness but would it be an acceptable substitute for HDPE?  >Any advice (short of try it & let me know) will be appreciated.

> Thanks, Jim

Jim,

Mylar would be perfect dielectric IF it wasn't so lossy at RF

frequencies. PET is about 200-300X lossier than Low Density

Polyethylene, and tends to rapidly overheat and self-destruct. Because

of it's lossiness, the spark output from your coil will also be much

poorer than an equivalent LDPE cap. And if you have to buy the material,

LDPE is actually much less expensive than PET. It's hard to beat free,

and you CAN use it for other HV insulating purposes - I'd still grab it.

-- Bert --

From: "Adam Parker" <park_e_r@hiwaay.net>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Tesla IRC channel

Date: Tue, 11 Aug 1998 21:13:50 -0500

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Tesla IRC would be a great idea, however, someone would likely need to

organize a schedule.

Adam

X-Mailer: IPAD 2.0

Date: Tue, 11 Aug 1998 23:42:51 -0500

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: Tesla IRC channel

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

At 09:13 PM 8/11/98 -0500, you wrote:

>Tesla IRC would be a great idea, however, someone would

>likely need to organize a schedule.

>

>Adam

Adam,

We tried that when the channel first went online

18 months ago. However since people live in different

time zones and have vastly different schedules it didnt

prove practical.

So now I man the channel whenever I have time and

people come and go as they please. My co-OP on the

channel Corey Edmunds has been of terrific help

and I couldnt of managed it without him.

The most popular times seem to be from around 8-10 PM CDT.

Also several people can usually be found on from 11:30-??? CDT.

Occasionally other times as well.

TTYL!

Mike Hammer

mhammer@misslink.net

From: "Kent Rankin" <kentrankin@theatreorgans.com>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: longest time running a tesla coil

Date: Wed, 12 Aug 1998 01:16:57 -0400

X-Mailer: IPAD 2.0

X-MimeOLE: Produced By Microsoft MimeOLE V4.72.2106.4

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

You run a tesla coil in your apartment?  What about the other tenants?

-Kent Rankin

-----Original Message-----

From: megavolt@mail.pcisys.net <megavolt@mail.pcisys.net>

To: Tesla-2 Mailing List <tesla-2@emachine.com>

Date: Tuesday, August 11, 1998 11:15 PM

Subject: Re: longest time running a tesla coil

Lan,

I have run as long as 20 minutes on the 4th of July at 1800 watts and

eventually would  trip a breaker.  Nothing got warm but the gaps and

the resistors.  Since adding PFC capacirance I can run that long

without tripping a breaker.  The ozone gets so thick that I have to

shut down and open the front door of my apartment.

Kent

megavolt@pcisys.net

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Date: Wed, 12 Aug 1998 04:13:05 -0700

From: "Fleischwulf" <fleischwulf@mailexcite.com>

Cc:

X-Sent-Mail: off

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

Subject: Re: longest time running a tesla coil

X-Sender-Ip: 208.153.7.9

Organization: MailExcite  (http://www.mailexcite.com:80)

Sender: tesl-err@emachine.com

Lan,

I believe I also said that at lower power levels it might be possible to

run for 30 minutes. 1000w or less.

paul M

---

Leben frei oder Starb !

On Tue, 11 Aug 1998 18:15:19   Lan wrote:

>Fleischwulf said in Re: longest time running a tesla coil at 11/Aug/1998,

>Tue 11:56:12.

>

>>Lan,

>> The longest power on time that I have used is about 1 minute.(4.5 Kw.)

>

>1 minute????

>

>Hasnt anyone (Tesla?) run them for a little longer, I need thisinformation

>because I have almost got my tesla coil made, its in the garden now, and

>wanted to know what would make a tesla coil capable of running for longer. >I'm sure Tesla if he had thought his energy distribution systems usable,

>and even his building lighting system usable, would have run his coils >continuously. could anyone else give more information on this?

>

>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: longest time running a tesla coil

Date: Wed, 12 Aug 1998 18:12:27 +0100

From: Lan <rune@dircon.co.uk>

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Fleischwulf said in Re: longest time running a tesla coil at 12/Aug/1998, Wed

12:13:05.

>Lan,

> I believe I also said that at lower power levels it might be possible to

> run for 30 minutes. 1000w or less.

My transformer is 620 watts.

X-Mailer: IPAD 2.0

Date: Wed, 12 Aug 1998 13:00:30 -0500

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: longest time running a tesla coil

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

At 06:12 PM 8/12/98 +0100, you wrote:

>Fleischwulf said in Re: longest time running a tesla coil at 12/Aug/1998, Wed

>12:13:05.

>

>>Lan,

>> I believe I also said that at lower power levels it might be possible

>> to run for 30 minutes. 1000w or less.

>

>My transformer is 620 watts.

>

Its not just your transformer or your input power.

What about your gap? Does it remain cool during runs

or does it get hot? If it gets hot it will fail to

quench leading to poor performance and further gap heating.

What about your caps? Are you using low loss poly caps or

lossy glass bottle caps? Glass is a very poor capacitor

dielectric and heats up rapidly.

Is your primary circuit wiring adequately sized or is there

some heating after a few runs?

All of these problems become much more difficult to control

as you increase run times.

If your coil is operating efficiently with low loss

components then longer run times are certainly possible.

TTYL!

Mike Hammer

mhammer@misslink.net

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: longest time running a tesla coil

Date: Wed, 12 Aug 1998 19:19:56 +0100

From: Lan <rune@dircon.co.uk>

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

>If your coil is operating efficiently with low loss

>components then longer run times are certainly possible.

My coil is nearly made, I have pounded the grounding rods (aluminium)

into the ground, in two places, about 2 metres apart, and just need to

modify the coil and get a good capacitor made before turning it on.

Also I guess running in cold weather could help ;) winter, sibera etc.

The rods are 2 of 0.5 metres, 2 of 1.2 metres, and 1 of 2 metres, pounded

in two groups, my soil is typical plant growing soil, one of the patches

had a lot of moisture in it before I added any to make digging easier.

My transformer is outside at the moment, how will it be affected by the weather do you think? I know it should be able to handle the current mildly hot, mostly unchanging weather (even in the nighttime its warm) here in england, but how will it stand the frost? Its a concrete packed neon transformer. Its protected by a table I put over it with a thick plastic

sheet to keep the rain out, and the transformer I have coated completely

with PVC electricians tape. I tested it by running the leads over the case, and it will only spark if I open it up and touch it to an un PVC'ed place.

Date: Wed, 12 Aug 1998 18:12:57 -0600

From: Jonathan Bazemore <jpbazemo@ouray.cudenver.edu>

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Picture of spark discharge

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

I didn't have any problems with damage to the camera electronics

and I was about 4 feet away from a coil discharging 36" sparks ;-)

One problem--the flash rubbed out some of  the sparks--so practically

speaking, a good way to use the digital camera is from a few feet  away,

with a light-diffusing translucent cover over the flash.  You want a bit

of light for the coil proper, but not enough to make the sparks hard to see.

Gene Waldo wrote:

> Ryan E. Jester wrote:

> >

> > Well, hopefully this won't anger anyone....I'm attaching a 99k pic of

> > my coil in action.

> > Oh and BTW, this pic was taken with the shutter open for ~30sec.

> >

> > Ryan Jester

> > ryanj@primary.net

>

> Ryan,

> Nice pics! Digital or analog camera?

>

> I'm guessing analog; scanned print? scanner res?

>

> I think it would be neat if we began a running commentary on 'The

> digital camera and its use in hi-RF environments.'

>

> Main points being: exposure details, optimal exposures, was the camera

> fried? There seems to be no seat-of-the-pants yet developed for the

> transition from analog to digital in this area. Fear and trepidation

> rule!

>

> Analog techniques would suggest for your shot of the discharge, a

> slighly under exposed flash or ambient setup shot and a double exposure

> in darkness for the sparks, of much less than 30 sec. Or simply more

> ambient illumination.  Is almost a no-can-do for the digital!

>

> My impression is that the visual impact of a good discharge is

> diminished by the broad strokes created by long exposures.

>

> Would we welcome commentary in this area?

>

> ------------------------------------------------

> Gene Waldo -  gwaldo@1stnetusa.com  - "Igor in Residence"

Date: Thu, 13 Aug 1998 13:29:17 -0600 (MDT)

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Daniel Lockhart <molly@cmn.net>

Subject: Cold Tesla Coils (was Re: longest time running a tesla coil)

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

Lan said:

>My coil is nearly made, I have pounded the grounding rods (aluminium) into

>the ground, in two places, about 2 metres apart, and just need to modify the >coil and get a good capacitor made before turning it on. also I guess running >in cold weather could help ;) winter, sibera etc.

What exactly does cold weather have to do with performance? The humidity

would be less, but what does the cold itself do, anything? It can get

pretty cold at my house in the winter (especially at night). It may be

worth it to move the coil outside for that.

What would extreme cold do? I know cryogenic temperatures improve

efficiency in lower voltage applications, but what would it do for Tesla

coils? The spark gap may not be a good idea, but what about sinking the cap

and transformer in liquid nitrogen? While you're at it, you may as well

superconduct the primary and secondary.  :-)  Seriously, has anybody played

around with coils and cryogenic temperatures? I don't know enough about

either to give a good guess...   :-)

Rather hot,

Daniel Lockhart

Date: Thu, 13 Aug 1998 22:04:28 +0200

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: A *Real* Stupid NST Story

From: Stefan.Binder@t-online.de (Stefan Binder)

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Daniel Lockhart wrote:

> To make an irritating story short, the two fuses in the multimeter are

> okay, but that is all that works. Voltage, resistance, frequency, current,

> inductance and capacitance measurements don't work.

The same happened to me when I wanted to measure the capacitance of a hv

cap. I had shorted the cap before, but it somehow recharged to a few

thousand volts - there was a spark, then the multimeter's display went

out.

I burried it in the garden :-)

So long,

Stefan

Date: Thu, 13 Aug 1998 19:49:31 -0500

From: Bert Hickman <bert.hickman@aquila.com>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Organization: Stoneridge Engineering

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Cold Tesla Coils (was Re: longest time running a tesla coil)

Sender: tesl-err@emachine.com

Daniel Lockhart wrote:

>

> Lan said:

>

> >My coil is nearly made, I have pounded the grounding rods (aluminium) into

> >the ground, in two places, about 2 metres apart, and just need to modify

> >the coil and get a good capacitor made before turning it on. also I guess > >running in cold weather could help ;) winter, sibera etc.

>

> What exactly does cold weather have to do with performance? The humidity

> would be less, but what does the cold itself do, anything? It can get

> pretty cold at my house in the winter (especially at night). It may be

> worth it to move the coil outside for that.

>

> What would extreme cold do? I know cryogenic temperatures improve

> efficiency in lower voltage applications, but what would it do for Tesla

> coils? The spark gap may not be a good idea, but what about sinking the cap

> and transformer in liquid nitrogen? While you're at it, you may as well

> superconduct the primary and secondary.  :-)  Seriously, has anybody played

> around with coils and cryogenic temperatures? I don't know enough about

> either to give a good guess...   :-)

>

> Rather hot,

>

> Daniel Lockhart

Dan and all,

I've found that my coil seems to perform the best during cold weather

(mine's on a screended-in, unheated porch). Suspect that it's due to low

humidity and less secondary coilform leakage resistance losses. The low

humidity may also have some enhancing effect relative to streamer

formation and propagation.

-- Bert --

Date: Thu, 13 Aug 1998 22:07:07 -0600 (MDT)

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Daniel Lockhart <molly@cmn.net>

Subject: Re: Cold Tesla Coils

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

Bert wrote:

>Dan and all,

>

>I've found that my coil seems to perform the best during cold weather

>(mine's on a screended-in, unheated porch). Suspect that it's due to low

>humidity and less secondary coilform leakage resistance losses. The low

>humidity may also have some enhancing effect relative to streamer

>formation and propagation.

>

>-- Bert --

Okay, makes sense (although I could hardly argue with you  :-). What about

altitude? The barometric pressure where I live is around 10 psi -- about

68% that of sea level. Again, I would think this would be related to

humidity also. Does this make much difference?

Plasma spheres have an inert gas at a low pressure, much less than one

atmosphere, that the high voltage is applied to. This makes the electrical

output "plasma" like. (I haven't heard otherwise, but I can hardly believe

that that stuff is actually plasma).

Conversely, what would happen if a high voltage were applied to a gas at a

rather high pressure (such as ten to one-hundred atmospheres)? This sounds

like things would get exciting. Plasma spheres, lasers, x-rays, and other

gizmos use low pressure to near-vacuums, but what happens when high voltage

(or high current for that matter) is discharged in a high pressure gaseous

environment?

I hope I'm not bugging anybody -- just curious,

Daniel Lockhart

Date: Fri, 14 Aug 1998 00:25:35 -0500

From: Bert Hickman <bert.hickman@aquila.com>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Organization: Stoneridge Engineering

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Cold Tesla Coils

Sender: tesl-err@emachine.com

Daniel Lockhart wrote:

>

> Bert wrote:

>

> >Dan and all,

> >

> >I've found that my coil seems to perform the best during cold weather

> >(mine's on a screended-in, unheated porch). Suspect that it's due to low

> >humidity and less secondary coilform leakage resistance losses. The low

> >humidity may also have some enhancing effect relative to streamer

> >formation and propagation.

> >

> >-- Bert --

>

> Okay, makes sense (although I could hardly argue with you  :-). What about

> altitude? The barometric pressure where I live is around 10 psi -- about

> 68% that of sea level. Again, I would think this would be related to

> humidity also. Does this make much difference?

>

> Plasma spheres have an inert gas at a low pressure, much less than one

> atmosphere, that the high voltage is applied to. This makes the electrical

> output "plasma" like. (I haven't heard otherwise, but I can hardly believe

> that that stuff is actually plasma).

>

> Conversely, what would happen if a high voltage were applied to a gas at a

> rather high pressure (such as ten to one-hundred atmospheres)? This sounds

> like things would get exciting. Plasma spheres, lasers, x-rays, and other

> gizmos use low pressure to near-vacuums, but what happens when high voltage

> (or high current for that matter) is discharged in a high pressure gaseous

> environment?

>

> I hope I'm not bugging anybody -- just curious,

>

> Daniel Lockhart

Daniel,

No problem! Generally, reducing air pressure tends to reduces breakdown

voltage (Paschen's Law...). This means that you should get longer

streamers and easier breakout at the toroid for a givent input energy

level than you'd get a sea level. However, your main gap will also tend

to break down at a lower voltage, reducing the "bang size" compared to a

sea level unless you readjust the main gap spacing. However, if you

continued to reduce pressure until you achieved a hard vacuum, the

breakdown voltage again increases - vacuum relays and vacuum capacitors

use the excellent insulating properties of a vacuum in this manner.

And, the stuff you're seeing in an excited plasma globe is, indeed, a

plasma. It's just that the gas density (and resulting energy density) is

relatively low.

Increasing gas pressure will increase it's breakdown voltage over that

at standard (sea level) pressure.  This property is often used to

provide improved voltage breakdown characteristics over air at STP.  The

pressurized "gaseous dielectric" can be dry nitrogen, hydrogen, sulfer

hexafluoride, etc. For example, this is often done within certain types

of high voltage circuit breakers for the power industry. Once the gap

breaks down, it will tend to be more conductive at high currents (more

potential charge carriers), and it will tend to recover it's insulating

capability more rapidly once the arc current flow has been interrupted.

Hope this answers your questions...

-- Bert --

Date: Fri, 14 Aug 1998 16:59:44 -0500

From: Tom Polk <tpolk@prismnet.com>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Cold Tesla Coils (was Re: longest time running a tesla coil)

Sender: tesl-err@emachine.com

Bert Hickman wrote:

>

Suspect that it's due to low

> humidity and less secondary coilform leakage ...

Speaking of coilform leakage, I had troubles with streamers running down

the inside of my coil. Upon investigation I discovered a dead mouse, who

had left droppings. Fecal coliform leakage, perhaps?

A gross reminder that any fine scientific instrument can benefit from

occasional cleaning!

From: <Dsurfr@aol.com>

Date: Fri, 14 Aug 1998 21:21:05 EDT

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: SW Cap Banks

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Hi All:

Would there be any advantage in making a bank of SW Caps with a rating of

twice the capacitance I need & then connecting this in parallel with an

identical bank to get back to the initial capacitance level. In other words,

would the spark output be increased since the caps should be able to withstand

a higher voltage? Still trying for a cheap cap. Thanks, Jim

X-Mailer: IPAD 2.0

Date: Fri, 14 Aug 1998 20:44:18 -0500

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: SW Cap Banks

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

At 09:21 PM 8/14/98 EDT, you wrote:

>Hi All:

>Would there be any advantage in making a bank of SW Caps with a rating of

>twice the capacitance I need & then connecting this in parallel with an

>identical bank to get back to the initial capacitance level. In other words,

>would the spark output be increased since the caps should be able to withstand

>a higher voltage? Still trying for a cheap cap. Thanks, Jim

Sorry Jim but it wouldnt help.

And I think you mean to wire the two banks in series

to get back to the original capacitance.

SW caps are cheap. In fact the ones I built cost

exactly zero dollars. But you will find that a poly

cap isnt that expensive and well worth the time and

effort to build it.

TTYL!

Mike Hammer

mhammer@misslink.net

Date: Fri, 14 Aug 1998 21:14:16 -0500

From: Bert Hickman <bert.hickman@aquila.com>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Organization: Stoneridge Engineering

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: SW Cap Banks

Sender: tesl-err@emachine.com

Dsurfr@aol.com wrote:

>

> Hi All:

> Would there be any advantage in making a bank of SW Caps with a rating of

> twice the capacitance I need & then connecting this in parallel with an

> identical bank to get back to the initial capacitance level. In other words,

> would the spark output be increased since the caps should be able to > withstand a higher voltage? Still trying for a cheap cap. Thanks, Jim

Jim,

Right idea, but the wrong connection!

You want to connect equally-sized capacitors in series. This will result

in a capacitor having half the capacitance of the individual caps, but

will double the breakdown voltage.

Connecting the caps in parallel will give you twice the capacitance of

each but at the same breakdown voltage.

-- Bert --

Date: Fri, 14 Aug 1998 21:24:38 -0500

From: Bert Hickman <bert.hickman@aquila.com>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Organization: Stoneridge Engineering

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Cold Tesla Coils (was Re: longest time running a tesla coil)

Sender: tesl-err@emachine.com

Tom Polk wrote:

>

> Bert Hickman wrote:

> >

> Suspect that it's due to low

> > humidity and less secondary coilform leakage ...

>

> Speaking of coilform leakage, I had troubles with streamers running down

> the inside of my coil. Upon investigation I discovered a dead mouse, who

> had left droppings. Fecal coliform leakage, perhaps?

>

> A gross reminder that any fine scientific instrument can benefit from

> occasional cleaning!

Tom,

Hmmm... Now I know where to look if a coil suddenly develops crappy

performance, goes down the tubes, and just outright stinks!... ba dum

DUM!  :^)

-- Bert --

Date: Sat, 15 Aug 1998 13:15:00 -0600 (MDT)

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Daniel Lockhart <molly@cmn.net>

Subject: Re: Cold Tesla Coils

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

Bert wrote:

>No problem! Generally, reducing air pressure tends to reduces breakdown

>voltage (Paschen's Law...). This means that you should get longer

>streamers and easier breakout at the toroid for a givent input energy

>level than you'd get a sea level.

and later...

>Increasing gas pressure will increase it's breakdown voltage over that

>at standard (sea level) pressure.

Huh. Could this be used to make a adjustable breakdown voltage spark gap?

All the electrodes would be in a sealed air-tight container. The pressure

could then be adjusted to change the required voltage needed for it to

breakdown. Additionally, the gap would glow in real cool colors if you used

the right gas...  :-)

Would this be worth experimenting with? Would you end up blowing your

transformer or simply correcting everything with a different primary tap?

>And, the stuff you're seeing in an excited plasma globe is, indeed, a

>plasma. It's just that the gas density (and resulting energy density) is

>relatively low.

Okay. I never was to sure about what that was. The temperature of it seemed

pretty low to call it plasma.

Thanks,

Daniel Lockhart

Date: Sat, 15 Aug 1998 14:44:09 -0500

From: Bert Hickman <bert.hickman@aquila.com>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Organization: Stoneridge Engineering

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Cold Tesla Coils

Sender: tesl-err@emachine.com

Daniel,

It turns out that this is exactly how some commercial high power spark

gaps operate! Maxwell and EG&G are a couple of firms that make

overvoltage and triggerable spark gaps. Spark Gaps are usually used for

high power switching at relatively low rep rates (1-10 pops per minute,

or less, depending on power level). Some have a gas inlet so that the

internal gas pressure and fill gas can be externally controlled. By

controlling the gas pressure and the type of gas, the breakdown voltage

can be adjusted. Alternatively, the pressure can be suddenly reduced to

induce firing.

Running a spark gap at high pressure allows a relatively compact gap to

handle very large breakdown voltages. Often dry nitrogen or pure

hydrogen is used as the "fill" gas, but other gases (and dry air) are

sometimes used as well. Some noble gases break down much easier than air

- argon, neon, and helium for example. Unfortunately, none of these

commercial devices will work very well in Tesla Coil duty, since while

they can handle the voltage and power level, they are not constructed so

that they quench (extinguish the arc) very well.

-- Bert --

Daniel Lockhart wrote:

>

> Bert wrote:

>

> >No problem! Generally, reducing air pressure tends to reduces breakdown

> >voltage (Paschen's Law...). This means that you should get longer

> >streamers and easier breakout at the toroid for a givent input energy

> >level than you'd get a sea level.

>

> and later...

>

> >Increasing gas pressure will increase it's breakdown voltage over that

> >at standard (sea level) pressure.

>

> Huh. Could this be used to make a adjustable breakdown voltage spark gap?

> All the electrodes would be in a sealed air-tight container. The pressure

> could then be adjusted to change the required voltage needed for it to

> breakdown. Additionally, the gap would glow in real cool colors if you used

> the right gas...  :-)

>

> Would this be worth experimenting with? Would you end up blowing your

> transformer or simply correcting everything with a different primary tap?

>

> >And, the stuff you're seeing in an excited plasma globe is, indeed, a

> >plasma. It's just that the gas density (and resulting energy density) is

> >relatively low.

>

> Okay. I never was to sure about what that was. The temperature of it seemed

> pretty low to call it plasma.

>

> Thanks,

>

> Daniel Lockhart

From: Tom Stock <tomstock@geocities.com>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Tesla Coil, instant popularity

Date: Sun, 16 Aug 1998 16:23:44 -0400

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

Today I was playing around with my coil after adding a fan to my spark gap. After running it for awhile I realized I had accidently left my cordless

phone in the vacinity of the coil The phone was fine, but when I went inside,

I noticed my  caller ID light was stuck on and the display was weird.

As I paged through the numbers, I had about 20 calls I had never seen before, including one from  "Albert Einstein", and another from "Charles Dickens".

Apparently the interference put it into some sort of test mode. Neato eh?

Wait till my wife gets home to check the caller id!! "See honey, I told you

I was popular when I was younger"...

-tom

From: <Dsurfr@aol.com>

Date: Sun, 16 Aug 1998 17:26:47 EDT

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Mini Neon Xformr

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Hi All: I picked up a very small neon transformer (5.5KV 22mA) from a scrapped

neon sign. Would it be possible to use this to power a very small coil for

experimental usage? Thanks again for the advice, Jim

Date: Sun, 16 Aug 1998 17:22:20 -0500

From: Bert Hickman <bert.hickman@aquila.com>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Organization: Stoneridge Engineering

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Mini Neon Xformr

Sender: tesl-err@emachine.com

Dsurfr@aol.com wrote:

>

> Hi All: I picked up a very small neon transformer (5.5KV 22mA) from a  >scrapped neon sign. Would it be possible to use this to power a very small

>coil for experimental usage? Thanks again for the advice, Jim

Jim,

A small coil can certainly be made to run off this little transformer,

since 5.5 KV at 22 MA is STILL over 120 VA. Because of the lower output

voltage, you'll need to use a smaller main gap spacing, but this will

still drive a 0.01 uF tank cap. Try using a 3" diameter secondary coil

wound with about 15" of closewound #26 AWG, a 3" x 12" toroid, a 0.01 uF

tank cap, and about a 10 turn flat or inverse conical primary for a

system that will run at about 400 kHz.

Good luck, and safe coilin' to you!

-- Bert --

Date: Mon, 17 Aug 1998 18:04:21 +0100

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Stephen Bell <steve@breakfix.demon.co.uk>

Subject: RF Ground  -  Area v Depth

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Hi guys,

When it comes to making a good RF ground, is depth more

important than area?

I have two 4 foot earth rods (copper plated steel) which I

intend to space 4 foot appart but I've also managed to get a roll of 7

mil copper sheet, one foot wide, approx 20 feet long.  The copper was

free and I thought I might burry it a couple of feet down, arranged as a

number of spokes from the connection point.  Because of the large area

involved, I didn't want to have to dig too deep.  Is two foot enough?

My first coil will be 1KW input power but I understand that the better

the ground connection the better the spark, whatever the power rating.

TIA

------------------------------------------------

Stephen Bell        (steve@breakfix.demon.co.uk)

Stuff also available at www.breakfix.demon.co.uk

------------------------------------------------

X-Mailer: IPAD 2.0

Date: Mon, 17 Aug 1998 16:59:23 -0500

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: RF Ground  -  Area v Depth

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

At 06:04 PM 8/17/98 +0100, you wrote:

>Hi guys,

>     When it comes to making a good RF ground, is depth more

>important than area?

>

>     I have two 4 foot earth rods (copper plated steel) which I

>intend to space 4 foot appart but I've also managed to get a roll of 7

>mil copper sheet, one foot wide, approx 20 feet long.  The copper was

>free and I thought I might burry it a couple of feet down, arranged as a

>number of spokes from the connection point.  Because of the large area

>involved, I didn't want to have to dig too deep.  Is two foot enough?

>

>My first coil will be 1KW input power but I understand that the better

>the ground connection the better the spark, whatever the power rating.

>

>TIA

>

>Stephen Bell        (steve@breakfix.demon.co.uk)

Stephen,

As most ham radio operators find out when putting up

1/4 wave vertical antennas that require an efficient

RF ground you need area much more than depth.

In fact for the above mentioned antenna the radial ground

leads can be layed directly UPON the ground. But you usually

bury them a couple inches to keep them out of the way.

The important thing is surface area in contact with the soil.

Drive in your ground rods and connect them with your copper

sheeting near the top. Bury it just under the surface.

TTYL!

Mike Hammer

mhammer@misslink.net

Date: Tue, 18 Aug 1998 09:25:57 -0500

From: RODERICK MAXWELL <tank@magnolia.net>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Electroplating

Sender: tesl-err@emachine.com

I want to try to build a torus by using wood turned on a lathe, or on

a homemade spindle. After the torus is shaped, sanded, and coated with

varnish I wish to use a method to electroplate the torus mentioned in a

book by Walt Noon "Secrets of Building Lighting Bolt Generators. With

this process you can coat nonconductive objects with copperplate. One of

the chemicals mentioned is Silver Nitrate.

If there is anyone on the list that has a chemistry background I would

like to know what concentration of silver nitrate you would need for

this process. There are many different concentrations listed at Fisher

Scientifics website.

(FSC242C) Silver, Reference Standard Solution 1000ppm  1%#37;

(FSC242C) Silver Chloride Granular

(FSC242C) Silver Nitrate Crystalline

(FSC243C) Silver Nitrate Crystalline

(FSC243C) Silver Nitrate, 10% w/v w/v

(FSC243C) Silver Nitrate Solution 1N

(FSC243C) Silver Nitrate, 0.5N

(FSC243C) Silver Nitrate, 0.171N (1mL = 10mg NaCl)

(FSC243C) Silver Nitrate, 0.141N (1mL = 5mg Cl)

(FSC243C) Silver Nitrate Solution 0.1N (N/10)

(FSC243C) Silver Nitrate, 0.05N

(FSC243C) Silver Nitrate, 0.0282N (1mL = 1mg Cl)

(FSC243C) Silver Nitrate, 0.025N

(FSC243C) Silver Nitrate, 0.0192N (1mL = 2mg CN) For Cyanide

(FSC244C) Silver Nitrate Solution 1mL =1mg NaCl

(FSC244C) Silver Nitrate, 0.0171N (1mL  = 1mg NaCl)

(FSC244C) Silver Nitrate Solution 1mL =0.5mg Cl

(FSC244C) Silver Nitrate, 0.0141N For Chloride (1mL  = 0.5mg Cl)

(FSC244C) Silver Nitrate Concentrate

http://www.fisher1.com/

The reason I want to try this method is I like the finish of a

commercial toroid, but they are horrendously expensive!

Frankensteins Helper

Max

From: <Dsurfr@aol.com>

Date: Tue, 18 Aug 1998 20:18:50 EDT

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: spark gap spacing

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Hi all: I seem to remember being told that I shouldn't exceed 1/4" total gap

for a 15kV 30mA neon but I've also read that I should allow .028" / 1000V

which would give .420" total. Which is correct? Also can I use smaller

diameter but longer length pieces of tubing to keep the heat build-up lower?

(that is 1/2" D. x 4" vs. 3/4" x 3" vs. 1" x 2" etc.) Also should I allow for

even greater gap (more tubes) to allow for tweeking? Thanks,  Jim

X-Mailer: IPAD 2.0

Date: Tue, 18 Aug 1998 20:26:52 -0500

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: spark gap spacing

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

At 08:18 PM 8/18/98 EDT, you wrote:

>Hi all: I seem to remember being told that I shouldn't exceed 1/4" total gap

>for a 15kV 30mA neon but I've also read that I should allow .028" / 1000V

>which would give .420" total. Which is correct? Also can I use smaller

>diameter but longer length pieces of tubing to keep the heat build-up lower?

>(that is 1/2" D. x 4" vs. 3/4" x 3" vs. 1" x 2" etc.) Also should I allow for

>even greater gap (more tubes) to allow for tweeking?

>Thanks,  Jim

Jim,

Ive always recommended a maxmimum total gap spacing of .250 inch

for 12kv and .300 inch for 15kv. You can run wider gaps than this

but I wouldnt do that with neon powered systems. The wider the gap

the greater the strain placed on your transformer and cap.

I have never run any more than .225 inch of total gap for

my 12kv system.

I would go for the larger diameter pipe vs length. I would

use no smaller than 3/4 inch pipe and no shorter than 2 inch

long electrodes.

For your 15kv system I would recommend at least 11 electrodes

3/4 X 3 inches spaced at .03 inch each. This will give 10 series

gaps with a total of .300 inch. If you have the room in your

housing then go to 13 electrodes (12 gaps) spaced .025 inch.

Same total distance but a greater number of gaps. Use plenty

of forced air.

You should not run the whole .300 inch until your coil is in tune.

This is important. GET YOUR COIL IN TUNE WITH NARROW GAPS FIRST.

Running over wide gaps  probably accounts for most neon failures.

TTYL!

Mike Hammer

mhammer@misslink.net

Date: Tue, 18 Aug 1998 22:11:45 -0600 (MDT)

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Daniel Lockhart <molly@cmn.net>

Subject: Tesla Coil Books

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

Hey everybody,

I'm trying to find some good books about Tesla coils and was wondering if

anybody had some input. I'm looking for more or less science books than I

am info and biographies about Tesla himself. I tried looking for books like

this once when I was in Colorado Springs and Telluride, and got real

burned, finding more weirdness than I did sanity.

A quick search at amazon.com found these:

Tesla Coil

by George Trinkaus

Paperback 3rd Rev edition (June 1987)

Alternative Energy Engineering; ISBN: 9991790837

Modern Tesla Coil Theory

by Duane A. Bylund

Paperback (December 1991)

Tesla Book Co; ISBN: 0914119095

The Tesla Coil Builder's Guide to the Colorado Springs

Notes of Nikola Tesla

by Richard L. Hull

Spiral-bound (June 1994)

Twenty First Century Books; ISBN: 0963601229

Has anybody read these (or others) that they can recomend?

Thanks,

Daniel Lockhart

Date: Tue, 18 Aug 1998 23:27:57 -0500

From: Bert Hickman <bert.hickman@aquila.com>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Organization: Stoneridge Engineering

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Tesla Coil Books

Sender: tesl-err@emachine.com

Daniel Lockhart wrote:

>

> Hey everybody,

>

> I'm trying to find some good books about Tesla coils and was wondering if

> anybody had some input. I'm looking for more or less science books than I

> am info and biographies about Tesla himself. I tried looking for books like

> this once when I was in Colorado Springs and Telluride, and got real

> burned, finding more weirdness than I did sanity.

>

> A quick search at amazon.com found these:

>

> Tesla Coil

> by George Trinkaus

> Paperback 3rd Rev edition (June 1987)

> Alternative Energy Engineering; ISBN: 9991790837

>

> Modern Tesla Coil Theory

> by Duane A. Bylund

> Paperback (December 1991)

> Tesla Book Co; ISBN: 0914119095

>

> The Tesla Coil Builder's Guide to the Colorado Springs

> Notes of Nikola Tesla

> by Richard L. Hull

> Spiral-bound (June 1994)

> Twenty First Century Books; ISBN: 0963601229

>

> Has anybody read these (or others) that they can recomend?

>

> Thanks,

>

> Daniel Lockhart

Daniel,

From a practical standpoint, Bylund's book is probably one of the best

ones. It's somewhat technical, and provides some very practical

information. While there are some errors, they are not serious relative

to coil building.

For an excellent modern day "translation" of Tesla's notes while he was

at Colorado Springs, Richard Hull's book is absolutely excellent.

However, it's not intended as a guide to coil building, however. I'm not

familiar with Trinkaus's book.

Safe coilin' to you!

-- Bert --

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Date: Wed, 19 Aug 1998 04:44:09 -0700

From: "Fleischwulf" <fleischwulf@mailexcite.com>

Cc:

X-Sent-Mail: off

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

Subject: Re: Electroplating

X-Sender-Ip: 208.153.7.9

Organization: MailExcite  (http://www.mailexcite.com:80)

Sender: tesl-err@emachine.com

Roderick,

I am a chemical Engineer, please let me know if I can help. What do you want to do with silver nitrate ?

Paul M

---

Leben frei oder Starb !

Date: Tue, 18 Aug 1998 23:43:50 -0500

From: RODERICK MAXWELL <tank@magnolia.net>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Electroplating

Sender: tesl-err@emachine.com

Fleischwulf wrote:

>

> Roderick,

>     I am a chemical Engineer, please let me know if I can help. What do you > want to do with silver nitrate ?

>

> Paul M

> ---

> Leben frei oder Starb !

>

Since I hate to type more than a few paragraphs at a time I'm going

to scan the material and send it to you directly.

Frankensteins Helper

Max

X-Mailer: IPAD 2.0

Date: Tue, 18 Aug 1998 23:54:17 -0500

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: Tesla Coil Books

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Daniel,

There not very many really good books out there on coiling.

Lots of really bad ones and a few fair ones.

At 10:11 PM 8/18/98 -0600, you wrote:

>Hey everybody,

>

>I'm trying to find some good books about Tesla coils and was wondering if

>anybody had some input. I'm looking for more or less science books than I

>am info and biographies about Tesla himself. I tried looking for books like

>this once when I was in Colorado Springs and Telluride, and got real

>burned, finding more weirdness than I did sanity.

>

>A quick search at amazon.com found these:

>

>

>Tesla Coil

>by George Trinkaus

>Paperback 3rd Rev edition (June 1987)

>Alternative Energy Engineering; ISBN: 9991790837

Forget this one as well as any other title by

Mr. Trinkaus.

>

>

>Modern Tesla Coil Theory

>by Duane A. Bylund

>Paperback (December 1991)

>Tesla Book Co; ISBN: 0914119095

Not too bad. A little dated but I kind of like this one.

>

>

>The Tesla Coil Builder's Guide to the Colorado Springs

>Notes of Nikola Tesla

>by Richard L. Hull

>Spiral-bound (June 1994)

>Twenty First Century Books; ISBN: 0963601229

This is meant as a companion guide to Tesla's Colorado Springs Notes.

It doesnt mean a lot on its own. If you buy the Notes the you should

get this book.

Another book I kind of like is Brent Turners. Info

on his book can be found on his web page.

http://www.apc.net/bturner/hv-page.htm#the book

You wont find any information in any of these books that

you cant find on the Internet for free.

TTYL!

Mike Hammer

mhammer@misslink.net

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Cheap Torus

X-Mailer: IPAD 2.0

From: break7@juno.com (Tyla A. Break)

Date: Wed, 19 Aug 1998 01:32:07 EDT

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Hi everyone,    My name is Kyle.  I was added to the list about 3

weeks ago. I've been checking out the Email . I think I have the right

address now. (I don't know anything about computers yet).  I've found it

very interesting!

Max just wrote about making a torus out of wood and plating it. I've

been trying to think a way around getting a com. built  torus also. But

for a form I've found something  that might work quite well. My wife

showed me that  House of Fabrics (one of those art & craft type places)

has nice foam dough nuts. Not styro-foam, but the harder plastic kind. Of

course there not as precision as wood turned on a lathe, but they,re real

good for a few bucks and no time invested. Also doesn't wood have

moisture, sap, and stuff in it that would look conductive to RF. and make

the torus "not hollow" ?    Now for a conductive surface can you

use aluminum paint? I've been told that some kinds actually leave an

aluminum surface! Also a gasket maker sells something called Copper Cote

for head gaskets. Some problems might be that its not actually copper or

the liquid medium might "melt" the form.  Anybody think this is a good

idea?

Kyle   break7@juno.com

X-Mailer: IPAD 2.0

Date: Wed, 19 Aug 1998 20:42:57 -0500

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: Cheap Torus

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

At 01:32 AM 8/19/98 EDT, you wrote:

>	Hi everyone,    My name is Kyle.  I was added to the list about 3

>weeks ago. I've been checking out the Email . I think I have the right

>address now. (I don't know anything about computers yet).  I've found it

>very interesting!

Welcome aboard Kyle.

>Max  just wrote about making a torus out of wood and plating it.  I've

>been trying to think a way around getting a com. built torus also. But

>for a form I've found something  that might work quite well.

Many of us have found that the flexible aluminum dryer ducting

available at most hardware stores makes an effective low cost

toroid. The 4 inch duct can be fashioned into a ring about 16

inches in diameter and the doughnut hole filled with a pair

of aluminum pie pans bolted together back to back.

>My wife showed me that House of Fabrics (one of those art & craft

>type places)has nice foam dough nuts. Not styro-foam, but the harder

>plastic kind.

These can work well if covered with a layer of aluminum foil or tape.

Most of these are too small though. The thickness of the toroid needs

to be from 3 to 6 inches for most medium sized coils.

>Of course there not as precision as wood turned on a lathe, but they,re

>real good for a few bucks and no time invested. Also doesn't wood have

>moisture, sap, and stuff in it that would look conductive to RF. and make

>the torus "not hollow" ?

The RF energy would tend to flow only on the outer surface

due to skin effect.

>Now for a conductive surface can you use aluminum paint? I've been told that >some kinds actually leave an aluminum surface! Also a gasket maker sells >something called Copper Cote for head gaskets. Some problems might be that

>its not actually copper or the liquid medium might "melt" the form.

>Anybody think this is a good idea?

>

>			Kyle   break7@juno.com

While this sounds like a good deal it doesnt work out

that way. The surface you get is not nearly conductive

enough. The actual amount of metal in these paints is very low.

There are paints with higher metal contents but these are rather

expensive. A length of aluminum duct and a couple pie pans

is very cheap and works great.

TTYL!

Mike Hammer

mhammer@misslink.net

From: "eriksson" <jan-erik.eriksson@mbox2.swipnet.se>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Hv resistors..(again)

Date: Thu, 20 Aug 1998 14:07:03 +0200

X-Mailer: IPAD 2.0

X-MimeOLE: Produced By Microsoft MimeOLE V4.72.3110.3

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Im planning to make a HV resistor from about 75 pieces of 14ohm metal film resistors in series. Then Ill get about 26.2kV (75*350V)

and about 45W (75*0.6W) and 1.05kOhms..will this do it?..

Jesper Olsson on the list was first with this idea, but I was off the

list at the moment when he asked about this one. So please give me your opinions.

Thanks.

Christopher Eriksson

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Cc: break7@juno.com

Subject: Build a coil that will worry my wife!

X-Mailer: IPAD 2.0

From: break7@juno.com (Tyla A. Break)

Date: Fri, 21 Aug 1998 02:03:46 EDT

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: break7@juno.com (Tyla A. Break)

To: Tesla-2@emachine.com

Cc: break7@juno.com

Subject: Build a coil that will worry my wife!

Date: Sat, 15 Aug 1998 12:30:59 EDT

Message-ID: <19980815.093046.7919.0.break7@juno.com>

Hi, I have some friends that are real computer nuts. (Not a bad thing).                     With their help I was able to get some photos.  I got pics of  R. Quicks,

Richard Hull, and Brent Turner's  coils and equip. They had a lot of rotary

gaps in series with static gaps .

Does the static gap sync. the rotary gap with peak A. C. voltage?

My 5KV trans. should arrive today.  I will definitely need lots of

advice to build a 5K coil .The trans ended up being a " pole pig".

Problem is I need to wind some sort of current-limiting choke or

ballast. My other pole trans is a 500W control transformer. Without

current-limiting it is not uncommon for the pri. winding to pull 200A!

Got any ideas?

I saw pics of some magnifier coils. I can't tell what all they got

there. But the performance is amazing. Should I consider building one? I

have two pole trans., lots of poly sheet , and 5gal of Diala AX.

Kyle   break7@juno.com

Date: Fri, 21 Aug 1998 20:14:53 +0200

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: TC & X-rays?

From: Stefan.Binder@t-online.de (Stefan Binder)

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Eric Davidson wrote:

>

> Stefan:

>

> A Geiger-Mueller tube is not designed to detect X-rays.  A G-M tube is

> designed for beta radiation.

Hmmm, I don't know, I've got a light bulb that's obviously evacuated,

upon putting it on my TC the glass glowed greenish, and my geiger

counter showed a high radiation level.

So my geiger counter seems to be able to detect x-rays, but I don't know

at which energy level...

Bye,

Stefan

From: ralph.hartwell@emachine.com (Ralph Hartwell)

Subject: synchronous motor for a g

Date: Fri, 21 Aug 1998 19:23:00 GMT

Organization: The Energy Machine Information System 504-733-3462

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

> I'm looking for a synchronous motor to make a sync rotort gap with and I

>can't find any good motor anywhere I look. If anyone can help me find one that'd be great, thanks

Try some of the surplus sales outfits, such as Fair Radio and the

like.  They quite often offer small synchronous motors at reasonable

prices.

You might be able to locate a motor from an old model 15 Teletype

machine or model 19 Teletype tape reader.  These came in two flavors -

series wound governor controlled and 60 Hz synchronous.  I once used one

of these motors to make a RSG for an experimental radar system.

You can also modify a regular induction motor to run synchronous, but

it's not easy.  It requires that you grind flat spots on the side of the

rotor of the motor to force it to lock in to the rotating magnetic

field.  It must be done accurately to maintain rotor balance, but it

does work.

Ralph  W5JGV

Date: Fri, 21 Aug 1998 19:06:25 -0500

From: Tedd Payne <tpayne@netnitco.net>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: My "For Sale" List On-Line, and Email Address Change

Sender: tesl-err@emachine.com

To all,

First, I've changed ISPs, and my new email address is:

teddp@earthlink.net

Second, I finally have my "For Sale" list on-line at:

http://home.earthlink.net/~teddp

For those of you who have web pages and would like to "trade links",

please let me know.

Thanks!

Tedd Payne

Date: Sat, 22 Aug 1998 14:11:14 +0200

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: TC & X-rays?

From: Stefan.Binder@t-online.de (Stefan Binder)

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Bill the arcstarter wrote:

> Oh-ho.  Sounds like it's working...  What type of bulb do you have?

> Sounds like I should bring the survey meter home... :

)

I've got the bulb from surplus (Germany), it's a "normal" E27 230V 25W

bulb, the cardboard box it was in says made by Paulmann, and it's made

in Europe (but Paulmann sounds German). Interestingly the filament is

longer than those in normal bulbs and is held by five "auxilary wires"

(in contrary to 2 in normal bulbs). Perhaps this is some sort of heavy

duty bulb, but what's the advantage of the vacuum over a gas filling?

Maybe the bulb doesn't get as hot?

> Was the glass glowing uniformly green?  Were there any "plasma"

> streamers inside the glass (implying non-evacuated)?

No streamers. The bulb is glowing uniformly green, and the "glass thing"

inside the bulb where the filament is mounted glows blue (?).

> What part of the world/states are you in?  I have an x-ray sensitive

> phosphor screen which I might be talked out of...

Well, Germany is a little far away, what material is this screen made

of? Maybe I could get the chemicals and build an x-ray screen myself.

Bye,

Stefan

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Synchronous Motors

X-Mailer: IPAD 2.0

From: break7@juno.com (Tyla A. Break)

Date: Sat, 22 Aug 1998 13:35:46 EDT

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

What makes a motor synchronous? How can one tell if a motor is

synchronous?  Are they always a particular RPM ?

Kyle Break@juno.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: bkibler@quicktel.com (Robert Kibler)

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: oil in poly caps

Date: Sat, 22 Aug 1998 21:22:52 -0700

X-Mailer: IPAD 2.0

X-MimeOLE: Produced By Microsoft MimeOLE V4.72.2106.4

Sender: tesl-err@emachine.com

Hello all.  I would like to know if it is really worth the trouble of

immersing my poly caps in oil.  It seems that if I just left them dry, there

would be no leaks, no chasing down fancy oil, and less fire hazard.  Is

there such an increase in performance as to make it worth it?

From: Coiler <mycroft@access1.net>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: RE: oil in poly caps

Date: Sat, 22 Aug 1998 21:48:58 -0700

X-Mailer: IPAD 2.0

Encoding: 18 TEXT

Sender: tesl-err@emachine.com

Yes, it is worth the trouble: here is why.

1) without the oil, you will get corona. This represents a loss of power

2) Far more important: Corona produces heat and ozone. Both are bad for poly. So bad in fact that  you can probably measure the life of your caps to be on the order of about 45 minutes runtime.

Or less.

Michael Baumann - Homebrewer, Coiler, and Ghod knows what else....

-----Original Message-----

From:	Robert Kibler [SMTP:bkibler@quicktel.com]

Sent:	Saturday, August 22, 1998 9:23 PM

To:	Tesla-2 Mailing List

Subject:	oil in poly caps

Hello all.  I would like to know if it is really worth the trouble of

immersing my poly caps in oil.  It seems that if I just left them dry, there

would be no leaks, no chasing down fancy oil, and less fire hazard.  Is

there such an increase in performance as to make it worth it?

Date: Sun, 23 Aug 1998 00:50:55 -0700

From: mike nicholas <705728@ican.net>

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: oil in poly caps

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Adding mineral oil increases your voltage rating, capacity and minimizes  corona losses in the air.

Robert Kibler wrote:

> Hello all.  I would like to know if it is really worth the trouble of

> immersing my poly caps in oil.  It seems that if I just left them dry, there

> would be no leaks, no chasing down fancy oil, and less fire hazard.  Is

> there such an increase in performance as to make it worth it?

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Date: Sun, 23 Aug 1998 18:52:58 -0700

From: "Fleischwulf" <fleischwulf@mailexcite.com>

Cc:

X-Sent-Mail: off

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

Subject: Re: Instant Fatality

X-Sender-Ip: 208.16.146.123

Organization: MailExcite  (http://www.mailexcite.com:80)

Sender: tesl-err@emachine.com

Kyle,

Contact with any power source (Pole Pig) will result in an instant Fatality. I ballast my pig with a couple of microwave transformers and a 480 volt power transformer. It is still capable of delivering 18 Kw at 14.4 Kv. This figures out to be a full amp and a quarter. You would never survive even a momentary contact. Be careful. See my Web page at:

http://www.geocities.com/capecanaveral/hangar/9561

paul m

---

Leben frei oder Starb !

On Fri, 21 Aug 1998 02:03:46   Tyla A. Break wrote:

>

>From: break7@juno.com (Tyla A. Break)

>To: Tesla-2@emachine.com

>Cc: break7@juno.com

>Subject: Build a coil that will worry my wife!

>Date: Sat, 15 Aug 1998 12:30:59 EDT

>Message-ID: <19980815.093046.7919.0.break7@juno.com>

>

>   Hi, I have some friends that are real computer nuts. (Not a bad thing).                     >With their help I was able to get some photos.  I got pics of  R. Quicks,

>Richard Hull, and Brent Turner's  coils and equip. They had a lot of rotary

>gaps in series with static gaps .

>

>   Does the static gap sync. the rotary gap with peak A. C. voltage?

>

>  My 5KV trans. should arrive today.  I will definitely need lots of

>advice to build a 5K coil .The trans ended up being a " pole pig".

>Problem is I need to wind some sort of current- limiting choke or

>ballast. My other pole trans is a500W control transformer.  Without

>current-limiting it is not uncommon for the pri. winding to pull 200A!

>            Got any ideas?

>

>   I saw pics of some magnifier coils. I can't tell what all they got

>there. But the performance is amazing. Should I consider building one? I

>have two pole trans., lots of poly sheet , and 5gal of Diala AX.

>

>

> Kyle   break7@juno.com

>

>

Date: Mon, 24 Aug 1998 22:36:12 -0700

From: Gene Waldo <gwaldo@1stnetusa.com>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Organization: S.A.P.I.E.N.T. (Surly, Agressive, Politically Incorrect Enemies of Non Thinkers)

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: spark gap design

Sender: tesl-err@emachine.com

> >From:  terryf@verinet.com [SMTP:terryf@verinet.com]

> >Sent:  Saturday, August 22, 1998 2:06 PM

> >To:  tesla@pupman.com

> >Subject:  Re: spark gap design

> >

> >Hi Jim,

> >

> >    Cleaning the gaps in my 60 x 4 mil gap is easy.  I get a 4 mil

> >ribbon of metal and just force it down through the gap sections.

SNIPS!

Terry,

All that mechanism stuff seems liable to more additional fail points...

does 2-sided tape hold up under the heat, with out slumping?

One mad and crazy notion occurs: What if a series gap was immersed in

some weird liquid?

--

Gene Waldo -  gwaldo@1stnetusa.com  - "Igor in Residence"

Date: Tue, 25 Aug 1998 20:50:23 +0100

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Stephen Bell <steve@breakfix.demon.co.uk>

Subject: Line Filters

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Hi Guys,

I've seen it written that mains line filters, for coiling use,

should be connected in reverse.  Can someone comfirm this and explain

why this is done?

TIA

------------------------------------------------

Stephen Bell        (steve@breakfix.demon.co.uk)

Stuff also available at www.breakfix.demon.co.uk

------------------------------------------------

X-Mailer: IPAD 2.0

Date: Tue, 25 Aug 1998 15:52:49 -0500

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: Line Filters

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

At 08:50 PM 8/25/98 +0100, you wrote:

>Hi Guys,

>        I've seen it written that mains line filters, for coiling use,

>should be connected in reverse.  Can someone comfirm this and explain

>why this is done?

>

>TIA

>

>------------------------------------------------

>Stephen Bell        (steve@breakfix.demon.co.uk)

>Stuff also available at www.breakfix.demon.co.uk

>------------------------------------------------

Stephen,

Yes this is true. If you think about the original use

for a line filter and our intended use on a Tesla coil

you will see why this is true.

AC line filters are meant to filter out any RF hash and spikes

on the AC line going TO your electronics equipment.

When used on a Tesla coil we are using them to filter out

any RF coming FROM our Tesla coil. The direction of the

RF we are trying to filter is opposite so the filter is

turned around when used with a coil.

This filter should be placed in the AC line between your

neons or other transformer and any controller you may have

such as a variac. In my setup I use two such filters.

One goes in between my neons and variac. The other is on the

incoming AC right after the main power switch and main fuse.

2 is probably not necessary but I had them so put them to use.

When selecting a filter choose one with adequate current rating.

2 or 3 parallel neons can pull quite a bit of current so make

sure your filter is up to it.

TTYL!

Mike Hammer

mhammer@misslink.net

X-Originating-IP: [199.217.140.22]

From: "Bill the arcstarter" <arcstarter@hotmail.com>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Guess what I just bought?

Date: Tue, 25 Aug 1998 14:14:21 PDT

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

Coilers,

I just bought an ancient (but workable - model 10E) Shopsmith for the

incredibly low price of $50!  I think this'll make a GREAT coil/choke

winding machine!

http://www.shopsmith.com

-Bill

From: <Dsurfr@aol.com>

Date: Tue, 25 Aug 1998 19:03:40 EDT

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Stacked cap calcs

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Hi All: I just want to make sure I've got these cap calculations right. If I

make a stacked plate cap using (4) layers of .0035" polypropelene between

plates (.014" total) that is rated @ .010uF, I should have a cap with a 14kV

rating? (1 mil = 1,000V.  ??) Three of these connected in series should give

me .003uF @ 42kV? If I combine (2) of these banks in parallel I should have a

total cap value of .006uF @ 42kV?

On the same topic would I be just as well off making (3) caps twice the size

giving me the same total value of .006uF @ 42kV or is it better to spread the

charge around between the two banks of caps? Also can all of these cap

"segments" be housed in the same tank of oil or should each bank be in its own

housing? Is this strong enough for a 15kV 30mA Neon? Thanks for all the help

on this topic but I want to get it close to right the first time I make one.

Jim

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: What's The Actual Voltage ?

X-Mailer: IPAD 2.0

From: break7@juno.com (Tyla A. Break)

Date: Tue, 25 Aug 1998 22:31:14 EDT

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Thanks for the help!   Fleischwulf, I have a few questions about

out voltage.

Questions for anyone really .

I have since solved the ballasting problem. I went to the local

surplus and picked up a LARGE choke or reactor. It's about 110 lbs, .030h

. About 100 turns of #5 on 85 lbs of steels. I've tapped it twice for a

total of three power levels.

About my transformer, I had it built by International Transformer

in Kansas. I asked for a 5KVA neon .They said that they commonly sell

distribution transformers built to "run backwards" for neon contractors.

I told them to send one out. I expected something about the size of a

5Gal bucket.  I was quite surprised .It weighs 371#!  More for my money I

guess.  Less than $1.00 per pound. The shipping is a killer though!

Now the questions. When you"ve got your pot ballasted there is a

voltage drop across your ballast right ?  So instead of 120 240 ,480 or

whatever the input voltage is supposed to be , its lower isn't it ? Or

are you putting the ballast on the high side?

I put my ballast on the low side and when the secondary is at 2amp load

the primary voltage drops to 60 volts. So i'm at about 4kv under load. Of

course without load it's 16kv.

Now doesn't the voltage from neon sign trans drop immediately under load?

So when someone is using a 15kv 60ma neon for they're coil the input

voltage might really be a lot less, right?

I'm I way off with this thinking?  P lease let me know what you think .

Anyone?

In case your wondering about 2 amps for secondary current ,pole

transformers have lots of headroom. SCE considers 150% normal operating

current.

Thanks for listening, Kyle  Break7@juno.com

From: <Hollmike@aol.com>

Date: Wed, 26 Aug 1998 08:14:54 EDT

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Guess what I just bought?

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Hey Bill,

Great catch!  I got something similar at an estate sale for $100, so I must

have paid too much :o)  I am sure you will be making good use of such a

device.

My latest treasure is three 15/60 Franceformer neons for free.  A good

buddy of mine works for a sign shop in Albuquerque and grabbed these for me.

He may be able to get several more, but I won't push my luck for the moment.

I may try to get him to pick up a couple more some day in the future though. I

now have enough neons to total 300ma.  That just about rivals a pole pig if I

can find a circuit to handle the input.  I also have a variac similar to the

one I sent you, except it is a 120V, 45 amp unit, so I can easily regulate the

power to the neons.  Reckon this will be quite enough to run my new 8 inch

secondary.  I now just need to make caps for the system and a new gap for it

and I'll be making some big sparks :o).

Well, keep in touch.  Things may slow down soon so I can start devoting

more time to coiling, and/or the singing arc.

Mike

Date: Wed, 26 Aug 1998 17:08:34 -0600 (MDT)

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Daniel Lockhart <molly@cmn.net>

Subject: Cleaning the Secondary

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

Hey all,

I'm fixin to wind my new 6.625-inch secondary. The PVC pipe is pretty

dirty, and there's quite a bit of printed specs on it (Schedule 40, 6" PVC

Type xxx, etc, etc). What's they best way to get the dirt and the printing

off? I've tried alcohol, and it doesn't work. Hot water takes the dirt out

fine but the printing is still there. I ran into something once that

worked, but I've forgotten what it was!

Thanks,

Daniel Lockhart

Date: Wed, 26 Aug 1998 09:35:25 -0500

From: RODERICK MAXWELL <tank@magnolia.net>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Cleaning the Secondary

Sender: tesl-err@emachine.com

Daniel Lockhart wrote:

>

> Hey all,

>

> I'm fixin to wind my new 6.625-inch secondary. The PVC pipe is pretty

> dirty, and there's quite a bit of printed specs on it (Schedule 40, 6" PVC

> Type xxx, etc, etc). What's they best way to get the dirt and the printing

> off? I've tried alcohol, and it doesn't work. Hot water takes the dirt out

> fine but the printing is still there. I ran into something once that

> worked, but I've forgotten what it was!

>

> Thanks,

>

> Daniel Lockhart

Try acetone, or the clear pipe cleaner they use before cementing two

pieces of pvc together.

Frankensteins Helper

Max

X-Mailer: IPAD 2.0

Date: Wed, 26 Aug 1998 14:20:08 -0500

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: Stacked cap calcs

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

At 07:03 PM 8/25/98 EDT, you wrote:

>Hi All: I just want to make sure I've got these cap calculations right. If I

>make a stacked plate cap using (4) layers of .0035" polypropelene between

>plates (.014" total) that is rated @ .010uF, I should have a cap with a 14kV

>rating? (1 mil = 1,000V.  ??)

Jim,

The values you see published showing voltage ratings above

1kv per mil for poly are DC values. Those must be cut in half

for AC use and derated further for pulse duty. Experiment has

shown that 350 VAC per mil in Tesla coil duty is a realistic

figure. So your 14 mil dielectric caps will handle 4.9 KV with

a little extra safety margin. Since this is a stack of thinner

sheets 5kv should not be a problem

>Three of these connected in series should give me .003uF @ 42kV?

You didnt give the number or size of your plates so I cant comment

on the capacitance except to say that 3 IDENTICAL caps in series

would have 1/3 the capacitance of the individual caps. And the voltage

would be closer to 15kv AC.

>If I combine (2) of these banks in parallel I should have a total cap

>value of .006uF @ 42kV?

Yes. Capacitances in parallel add up. The voltage again would be 15kv.

>On the same topic would I be just as well off making (3) caps twice the size

>giving me the same total value of .006uF @ 42kV or is it better to spread the

>charge around between the two banks of caps?

Doesnt really matter. Personally I would make the larger caps.

The advantage of the smaller caps is the range of values you could

get by using them in various combinations.

>Also can all of these cap "segments" be housed in the same tank of oil or >should each bank be in its own housing?

Yes they can be in the same housing. My flat plate cap consists of 6

caps wired in series all in the same housing.

>Is this strong enough for a 15kV 30mA Neon?

Just barely Jim. The total would be 52 mils spread across

3 caps. It would probably be OK if you run it very conservatively

and would leave very little room for error. I would increase

your dielectric thickness in each cap to 6 layers instead of just 4.

This will give you 21 mils per cap or 63 mils for a series string

of 3. The thicker dielectric will lower your capacitance somewhat.

Compensate by adding a few more plates to each cap.

>Thanks for all the help on this topic but I want to get it close

>to right the first time I make one.

>Jim

>

No problem Jim. Keep us informed

of your progress.

TTYL!

Mike Hammer

mhammer@misslink.net

Date: Wed, 26 Aug 1998 23:23:39 +0100

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Stephen Bell <steve@breakfix.demon.co.uk>

Subject: Status report

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Hi Guys,

I thought I'd let you know how I'm getting on with the making of

my first coil.  To be honest, I have not made anything yet but I'm

actively collecting the raw materials.  All that I am waiting on is the

secondary coil former (160mm O/D polyethylene tube), some 6mm acrylic

for the cap housing and the enamaled wire for the secondary. All the

rest of the bits still outstanding are basic hardware store stuff.

The control unit is comming together very nicely, thanks to scrounged

junk and surplus parts, with the following specifications:-

20 amp single phase line filter on 240VAC/50Hz input

13 amp ciruit breaker

2 pole 240VAC/30A contactor (220V coil running with series resistor)

Emergency Power Off button (a professional latching unit)

EPO tether (tied to operator to kill power if in trouble)

Security lockout (an old combination lock I made)

2KVA Variac

3 phase 20 amp line filter

Output connector (different to mains outlet so bypass not possible)

Some serious looking red power indicator lamps

Enclosure, a large metal equipement case (suitcase)

To power up the unit you need to enter the code number.  Also, the EPO

button and the personal EPO (tied to operator with string) need to be

set 'ready' and the variac has to be at zero.

When it's finished it should look really cool!  ;-)

(Check out my web site for current state of construction.  I'll be

taking lots of photographs as I go; I've got a digital camera and

15MBytes of free web space! :-)

P.S.

Any UK coilers who want some 160mm O/D polyethylene tubing, drop

me a line.  The minimum length I could get was 6m.

------------------------------------------------

Stephen Bell        (steve@breakfix.demon.co.uk)

Stuff also available at www.breakfix.demon.co.uk

------------------------------------------------

Date: Wed, 26 Aug 1998 19:59:14 -0500

From: Bert Hickman <bert.hickman@aquila.com>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Organization: Stoneridge Engineering

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Cleaning the Secondary

Sender: tesl-err@emachine.com

Daniel Lockhart wrote:

>

> Hey all,

>

> I'm fixin to wind my new 6.625-inch secondary. The PVC pipe is pretty

> dirty, and there's quite a bit of printed specs on it (Schedule 40, 6" PVC

> Type xxx, etc, etc). What's they best way to get the dirt and the printing

> off? I've tried alcohol, and it doesn't work. Hot water takes the dirt out

> fine but the printing is still there. I ran into something once that

> worked, but I've forgotten what it was!

>

> Thanks,

>

> Daniel Lockhart

Daniel,

I've always sanded it off. Sanding also removes the outermost layer of

PVC which sometimes may be a bit oxidized, especially if it has been

sitting outside for awhile. Roughing the surface also makes for a strong

bond with whichever sealant you use.

-- Bert --

Date: Wed, 26 Aug 1998 20:42:53 -0500

From: Tom Polk <tpolk@prismnet.com>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Cleaning the Secondary

Sender: tesl-err@emachine.com

Daniel Lockhart wrote:

>

> Hey all,

> What's they best way to get the dirt and the printing off?

Methyl Ethyl Ketone (MEK) at the hardware store works, but there is a

trick to it. Wet a quarter section of paper towel (WEAR GLOVES, WEAR

IMPENETRABLE GLOVES!) with MEK, make one or two swipes. That's all you

get. Then do the same thing with another paper towel.

It's a little easier than sanding, but not much. You'll probably want to

sand anyway, but at least you don't have to work quite as hard getting

the color off. MEK is not as safe as sanding, though.

From: Roy Minnich <rminnich@rolling-meadows.lib.il.us>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: RE: Cleaning the Secondary

Date: Thu, 27 Aug 1998 09:04:28 -0500

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

They sell cleaner for PVC pipe in small cans right next to the PVC cement.  It's probably MEK and works really well.

Roy Minnich

----------

From: 	Tom Polk[SMTP:tpolk@prismnet.com]

Reply To: 	Tesla-2 Mailing List

Sent: 	Wednesday, August 26, 1998 8:42 PM

To: 	Tesla-2 Mailing List

Subject: 	Re: Cleaning the Secondary

Daniel Lockhart wrote:

>

> Hey all,

> What's they best way to get the dirt and the printing

> off?

Methyl Ethyl Ketone (MEK) at the hardware store works, but there is a

trick to it. Wet a quarter section of paper towel (WEAR GLOVES, WEAR

IMPENETRABLE GLOVES!) with MEK, make one or two swipes. That's all you

get. Then do the same thing with another paper towel.

It's a little easier than sanding, but not much. You'll probably want to

sand anyway, but at least you don't have to work quite as hard getting

the color off. MEK is not as safe as sanding, though.

From: Dsurfr@aol.com

Date: Thu, 27 Aug 1998 18:15:14 EDT

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: dielectric voltage rating

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Mike: Thanks, as always, for the reply. 350V. per mil really adds to the

number of layers I'll need to use to get a decent voltage rating per cap.

Is there any "rule of thumb" for building a very safe cap for Neons? I've

read that the voltage rating should be more than double that of the transformer

(30kV to 40kV for a 15kV output). The reason I didn't include the plate size

or number of plates is I'm still checking on available container sizes & the

cost/yield of polyethelene sheets and thickness as opposed to the 8 1/2" x 11"

x .0035" "Poly-P" sheet protectors (that so far seem the most cost effective &

easiest to work with) I guess these caps are going to cost me more than I would

like but as I said I hope to make something I can rely on for a while & it'll

still be a lot less than buying a Maxwell cap (I hope).

Thanks,   Jim

Date: Thu, 27 Aug 1998 19:47:18 -0500

From: Bert Hickman <bert.hickman@aquila.com>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Organization: Stoneridge Engineering

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: dielectric voltage rating

Sender: tesl-err@emachine.com

Dsurfr@aol.com wrote:

>

> Mike: Thanks, as always, for the reply. 350V. per mil really adds to the

> number of layers I'll need to use to get a decent voltage rating per cap. Is

> there any "rule of thumb" for building a very safe cap for Neons? I've read

> that the voltage rating should be more than double that of the transformer

> (30kV to 40kV for a 15kV output). The reason I didn't include the plate size

> or number of plates is I'm still checking on available container sizes & the > cost/yield of polyethelene sheets  and thickness as opposed to the 8 1/2" x 11"

> x .0035" "Poly-P" sheet protectors (that so far seem the most cost effective &

> easiest to work with) I guess these caps are going to cost me more than I would

> like but as I said I hope to make something I can rely on for a while & it'll

> still be a lot less than buying a Maxwell cap (I hope).

> Thanks,   Jim

Interesting Question, Jim!

Tank capacitors are really "stressed" in a Tesla Coil. Not only are they

charged to high voltages, then discharged at relatively high rep rates,

often at currents of many hundreds of amps, but the discharges are very

underdamped. When the main gap fires, the voltage on the cap begins to

swing to the opposite polarity, and 1/2 cycle later may be at 80% of the

original voltage but at an opposite polarity. This is one of the

toughest environments that any cap can see! Since the dielectric "sees"

a voltage swing of almost 2X the initial voltage, you must take this

into consideration in properly rating your cap.

In addition, the peak voltage that the neons provide may be 1.41X the

RMS faceplate rating of the transformer. If you drive it off a 0-140 V

Variac, this needs to be increased by another 17% or so. If driven off

140 VAC, a 15 KV neon can output almost 25kV. If the gap fires at this

peak, the cap's dielectric may see a swing of as much as 45kV.

But this is not all!! Adding insult to injury, if you've "sized" the

tank cap to be the maximum for your neon source (as many Tesla Coil

design programs do...) you are virtually assured of seeing 60 Hz

resonance between the transformer's leakage inductance and your tank

cap. In this case, the maximum voltage your cap can see may be limited

by the lesser of the breakdown voltage of your main gap (hopefully),

safety gap (next best), your HV transformer (bummer), or your tank cap

(real bummer!). If your main and safety gaps are set too widely, the

output voltage of the neon will "try" to climb to as much as 3-5X the

normal level... until it silently carbonizes a shorting path... or your

tank cap goes ZZZzzzztttt!

Taking all of these effects into consideration, I usually recommend

designing/buying caps that have a DC rating AT LEAST 3X the maximum RMS

voltage that your power source can provide. By reducing the main gap

spacing, you might just be able to run reliably with a cap rated at

less, but most coilers find it hard to resist opening the gap "just a

bit" further, and your safety margins become non-existent.

With multiple thicknesses of thin LDPE sheets, you should be able to run

at effective stress levels of 500-600 V/Mil AS LONG as you use at least

the 3X multiplier on your NST's RMS voltage. For example, if you plan

(eventually) to operate off a 15 kV NST, design your cap to withstand

45KV. Using 500 V/Mil, this brings you to about 90 mils of LDPE... 600

V/Mil takes you down to 75 mils. Some coilers have gone even thinner...

but the caps will have an inceased probability of failing prematurely.

I'd rather build my cap once - repairing them is a very oily, messy

undertaking!

Hope this helps, and safe cappin' to you!

-- Bert --

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Big zapper or Little Whippersnapper?

X-Mailer: IPAD 2.0

From: break7@juno.com (Tyla A. Break)

Date: Fri, 28 Aug 1998 01:04:40 EDT

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Fleischwulf

Thanks for the reply.  Now I have lots of questions.

What you are doing sounds very interesting.  You are using

microwave transformers?  As in microwave oven??  Are you using

the filament winding or are you using primary winding and shorting

the HV winding?

My transformer has a primary winding of 240V and a secondary

of 16KV.  Out here in California(L.A.) 16kv is common or actually 16,434.

My transformer was custom built with a 16kv secondary.  The lid comes

off like usual but has no switch inside.

I'm an owner of a small electrical contracting company(c-10)

and wiring separate circuit is what I've done #6 fused at 60amps.  Hook

up to trans and reactor with #6 arc-welding cable via 50 amp

range-receptacle.  I'm not using a variac right now as I'm just testing

transformer with Jacobs ladder.  Short duration use is 125 amps in,

measured 2 amp output but at considerably lower voltage.

When any series circuit is energized the voltage drop is divided

among the loads in series.  If anything is in series with your transformer

you're getting less than full voltage.  I've measured all over circuit at

125amps in,170v drops across the reactor and 60v across the transformer

primary for a total of 240v.

At 125 amps, my electrical service drops 10v from 240 to 230.  I'm sure

the readings are accurate.

I'd like to check your page, but I need to borrow a better computer.

Flaming hot dog?  Is that an electro-dogger like Charles Brush's

(cooking with high voltage)? Also 140kv ex-ray cable! I like! How much?

How big? Where?

Thank you for writing back.  Very interested.

Kyle

break7@juno.com

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Date: Fri, 28 Aug 1998 05:01:55 -0700

From: "Fleischwulf" <fleischwulf@mailexcite.com>

Cc:

X-Sent-Mail: off

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Expiredinmiddle: true

X-Mailer: IPAD 2.0

Subject: Re: Big zapper or Little Whippersnapper?

X-Sender-Ip: 208.153.7.9

Organization: MailExcite  (http://www.mailexcite.com:80)

Sender: tesl-err@emachine.com

Kyle,

I'm using the microwave transformer (Primary in series with the secondaries shorted) this works really well. I have stayed away from the conventional resistive loads that

are commonly used. This system is really cool. By configuring the microwave transformers differently I can vary the power output anyway I choose . If I use a series circuit

I can limit the power output to 3500 W. By using 2 in Parallel I can get 7200  W.

If I use the two Microwave transformers in parallel with a large 480 v power transformer I can get 18,000 W. This really cooks the hotdogs. The most I have feed my primary is 75 or 80 amps. Thats 1.3 amps on the output side. I would strongly suggest that you limit the output of your secondary to 7 or 8 Kw.

Nothing I have built to date survives very long at greater power levels. Gaps burn up no matter how you cool them. I was forcing air directly through the gaps using a 2 Hp shop vac. The gap lasted about 5 minutes total. That is not in continous operation. (short burst 10-15 seconds) My 4" coil will produce 60" arcs at just 3500 W.

questions ?

paul m

---

Leben frei oder Starb !

On Fri, 28 Aug 1998 01:04:40   Tyla A. Break wrote:

>Fleischwulf

>

>	Thanks for the reply.  Now I have lots of questions.

>What you are doing sounds very interesting.  You are using

>microwave transformers?  As in microwave oven??  Are you using

>the filament winding or are you using primary winding and shorting

>the HV winding?

>

>	My transformer has a primary winding of 240V and a secondary

>of 16KV.  Out here in California(L.A.) 16kv is common or actually 16,434.

>

>My transformer was custom built with a 16kv secondary.  The lid comes

>off like usual but has no switch inside.

>

>	I'm an owner of a small electrical contracting company(c-10)

>and wiring separate circuit is what I've done #6 fused at 60amps.  Hook

>up to trans and reactor with #6 arc-welding cable via 50 amp

>range-receptacle.  I'm not using a variac right now as I'm just testing

>transformer with Jacobs ladder.  Short duration use is 125 amps in,

>measured 2 amp output but at considerably lower voltage.

>

>	When any series circuit is energized the voltage drop is divided

>among the loads in series.  If anything is in series with your transformer

>you're getting less than full voltage.  I've measured all over circuit at

>125amps in, 170v drops across the reactor and 60v across the transformer

>primary for a total of 240v.

>

>At 125 amps, my electrical service drops 10v from 240 to 230.  I'm sure

>the readings are accurate.

>

>	I'd like to check your page, but I need to borrow a better computer.

>

>	Flaming hot dog?  Is that an electro-dogger like Charles Brush's

>(cooking with high voltage)? Also 140kv ex-ray cable! I like! How much?

>How big? Where?

>

>	Thank you for writing back.  Very interested.

>

>		Kyle

>			break7@juno.com

Full-Name: Bert Wikkerink

Date: Fri, 28 Aug 1998 08:44:12 -0400

From: Bert Wikkerink <bert@mitra.com>

Organization: Mitra Imaging Inc.

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Big zapper or Little Whippersnapper?

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

60" arcs ??!! Wow !! Do you have pics of both the coil & the arcs anywhere ?.

Thanks,

Bert Wikkerink

Fleischwulf wrote:

>My 4" coil will produce 60" arcs at just 3500 W.

>

> questions ?

>

> paul m

> ---

> Leben frei oder Starb !

Sender: tesl-err@emachine.com

Date: Fri, 28 Aug 1998 07:53:14 -0700

From: Doug Fulton <dfulton@jdeere.rose.hp.com>

Organization: Hewlett-Packard Company

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Cleaning the Secondary

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Daniel Lockhart wrote:

>

> Hey all,

>

> I'm fixin to wind my new 6.625-inch secondary. The PVC pipe is pretty

> dirty, and there's quite a bit of printed specs on it (Schedule 40, 6" PVC

> Type xxx, etc, etc). What's they best way to get the dirt and the printing

> off? I've tried alcohol, and it doesn't work. Hot water takes the dirt out

> fine but the printing is still there. I ran into something once that

> worked, but I've forgotten what it was!

>

> Thanks,

>

> Daniel Lockhart

It seems there are alot of methods for cleaning pvc, and here is another one.

PVC primer works well, apply it in small amounts, and wipe it off right

away. It will remove dirt and printing quite nicely, that's what it is

made for. It will also the gloss of the surface, and leave it a little

bit rough. It has dye in it so that it is easy to see the area that is

primed but if it is wiped off right away the dye will not be a problem,

not sure if it would be a problem anyway.

Doug Fulton

Date: Fri, 28 Aug 1998 13:00:57 -0400 (EDT)

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: fxphoto@centuryinter.net (fxphoto)

Subject: Re: Cleaning the Secondary

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Hi Dan,

I did not notice anyone mention wet sanding. It is the best method I have

found for cleaning PVC. Start with a sheet of 80 to 120 grit silicon carbide

paper(it says wet dry right on it) dip it into a bucket of water (or if your

wife is a sweetheart like mine, use the kitchen sink ;) , put one drop of

dishsoap in the center of the grit side of the paper ( a very small amount

of soap breaks the surface tension and allows the water to stay on the PVC

better. It also helps lubricate and makes the sanding easier. too much soap

just makes a big mess)

The sanding with this grit should remove a lot of material quickly. Use it

to remove any pits and lumps and all of the ink. When everything looks nice

and smooth, rinse off the PVC, then using 240 to 320 grit silicon carbide

paper sand everything again. (it will make it smoother) It should only take a

minute or two of using this grit for you to notice a difference. Rinse off

the PVC once again and if you want a really fine finish use 600 grit silicon

carbide paper.

Bill Turbett

>Hey all,

>

>I'm fixin to wind my new 6.625-inch secondary. The PVC pipe is pretty

>dirty, and there's quite a bit of printed specs on it (Schedule 40, 6" PVC

>Type xxx, etc, etc). What's they best way to get the dirt and the printing

>off? I've tried alcohol, and it doesn't work. Hot water takes the dirt out

>fine but the printing is still there. I ran into something once that

>worked, but I've forgotten what it was!

>

>Thanks,

>

>Daniel Lockhart

X-Mailer: IPAD 2.0

Date: Fri, 28 Aug 1998 14:06:09 -0500

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: Big zapper or Little Whippersnapper?

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

>Fleischwulf wrote:

>

>> My 4" coil will produce 60" arcs at just 3500 W.

>>

>> questions ?

Then Bert Wikkerink wrote:

>60" arcs ??!! Wow !! Do you have pics of both the coil & the arcs anywhere ?.

>

>Thanks,

>

>Bert Wikkerink

Bill Pollack (arcstarter) and Paul Marshall (Fleischwulf)

probably hold the record for output from a 4 inch diameter

secondary. At least as far as I know. I held the record

for some time with 54 inches at a little above 1.2 KVA.

If someone has done better with a 4 incher I would sure like

to hear about it.

I hope to reclaim that record soon with the new .02 mfd

flat plate cap Ive been working on.

Its interesting to note that despite running twice the power

level that I run they only managed an additional 6 inches.

Just goes to show you that it takes more than just raw power

to get long sparks.

Pictures of my coil running are on my web page. Longest

streamers shown are 50 inches. I took  a whole 12 exposure

roll trying to capture some longer streamers on film but

failed. The longest ones just didnt fall into the 1/4-1/2

second exposure time. (they never do)

http://www.misslink.net/mhammer/

TTYL!

Mike Hammer

mhammer@misslink.net

X-Mailer: IPAD 2.0

Date: Fri, 28 Aug 1998 14:13:35 -0500

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: Cleaning the Secondary

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

At 01:00 PM 8/28/98 -0400, you wrote:

>Hi Dan,

> I did not notice anyone mention wet sanding. It is the best method I have

>found for cleaning PVC. Start with a sheet of 80 to 120 grit silicon carbide

>paper(it says wet dry right on it) dip it into a bucket of water (or if your

>wife is a sweetheart like mine, use the kitchen sink ;) , put one drop of

>dishsoap in the center of the grit side of the paper ( a very small amount

>of soap breaks the surface tension and allows the water to stay on the PVC

>better. It also helps lubricate and makes the sanding easier. too much soap

>just makes a big mess)

>  The sanding with this grit should remove a lot of material quickly. Use it

>to remove any pits and lumps and all of the ink. When everything looks nice

>and smooth, rinse off the PVC, then using 240 to 320 grit silicon carbide

>paper sand everything again. (it will make it smoother) It should only tak a

>minute or two of using this grit for you to notice a difference. Rinse off

>the PVC once again

>and if you want a really fine finish use 600 grit silicon carbide paper.

>                                       Bill Turbett

>

Bill and all,

I dont really approve of wet sanding PVC pipe. PVC is porous

and those pores can hold and retain moisture. Its that moisture

content that adds to the high RF loss of PVC. Thats why we bake

and seal our PVC forms with polyurethane.

I take some 220 grit ALOX (aluminum oxide) sand paper and use it

to remove the gloss and any printing. I do this dry. Then use a

paper towel and a little acetone or denatured alcohol to wipe off

the dust. Then into the oven or the heat gun treatment for drying

followed by polyurethane varnish inside and out while still warm.

TTYL!

Mike Hammer

mhammer@misslink.net

X-Mailer: IPAD 2.0

Date: Fri, 28 Aug 1998 14:24:36 -0500

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Mike Hammer <mhammer@misslink.net>

Subject: list archives

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Hello all,

The latest list archive covering posts from May and June

has been completed and placed on Dave Bronakowski's archive

site as well as funet.fi.

The file name is tes2arc12.zip.

The previous archive, tes2arc11.zip, which

covers posts from March/April has been changed.

Those of you that downloaded this archive are urged

to delete it and download the new archive. The new

archive contains a number of "lost" posts. These were

posts that were dropped from the old list server.

I have managed to resurrect those missing posts as

well as repair a couple corrupted ones.

The URL for Daves archive is http://www.velocity.net/~djb/tesla.shtml

Many thanks to Dave for providing this space for us all.

For those that prefer to use funet.fi the URL is:

ftp://nic.funet.fi/pub/sci/electrical/tesla/maillist/tesla2/

Thanks to Kristian Ukkonen for maintaining that site.

TTYL!

Mike Hammer

mhammer@misslink.net

X-Originating-IP: [208.16.149.172]

From: "Bill the arcstarter" <arcstarter@hotmail.com>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Big zapper or Little Whippersnapper?

Date: Fri, 28 Aug 1998 18:28:28 PDT

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

It was written:

>>60" arcs ??!! Wow !! Do you have pics of both the coil & the arcs

anywhere ?.

>>

>>Thanks,

>>

>>Bert Wikkerink

>

>Bill Pollack (arcstarter) and Paul Marshall (Fleischwulf)

>probably hold the record for output from a 4 inch diameter

>secondary. At least as far as I know. I held the record

>for some time with 54 inches at a little above 1.2 KVA.

>If someone has done better with a 4 incher I would sure like

>to hear about it.

Well - if thats true - we sure didn't do it using finesse - but RAW

POWER - something like 3KVA or so!  In terms of feet of arc per KVA I

don't think we're doing that well.

The actual measured diameter of our secondary is 4.25 inches.

Mike still rules! :)

Check out my (and Paul's page - links to it from mine) for info and pics

if anyone cares.

-Bill the arcstarter

Starting arcs in Cinci, OH

http://www.geocities.com/CapeCanaveral/Hangar/6160

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: 4 Inch Big Zappers

X-Mailer: IPAD 2.0

From: break7@juno.com (Tyla A. Break)

Date: Sat, 29 Aug 1998 13:52:04 EDT

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Hi all, I've been reading the recent banter about 4" coils.  Kind

of coil wrestling , 4" Category.

When looking for max. output you want to try to remove all losses

right? I've heard many times that PVC is "lossy". How about using

polyethylene pipe? It's available in all popular coil competition sizes

3,4,6,". I know it's not always perfectly round but it might be lower

loss . It's cheap too.

Kyle

break7@juno.com

_____________________________________________________________________

X-Mailer: IPAD 2.0

Date: Sat, 29 Aug 1998 14:38:09 -0500

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: 4 Inch Big Zappers

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

At 01:52 PM 8/29/98 EDT, you wrote:

>	Hi all, I've been reading the recent banter about 4" coils.  Kind

>of coil wrestling , 4" Category.

>	When looking for max. output you want to try to remove all losses

>right? I've heard many times that PVC is "lossy". How about using

>polyethylene pipe? It's available in all popular coil competition sizes

>3,4,6,". I know it's not always perfectly round but it might be lower

>loss . It's cheap too.

>

>			Kyle

>		break7@juno.com

>

Kyle,

You are correct that you should try to minimize losses in your

coil system. However would it suprise you (and others) if I said

that the secondary is just about the LEAST important component?

Losses on the primary side account for nearly all of the system wide

losses with gap loss leading the pack. In fact gap loss with

a typical poorly designed beginner type gap can be greater than

all other losses combined. Anything you do to reduce gap loss

will pay big dividends. Using multiple gaps in series with adequate

thermal mass to avoid overheating helps here.

After gap loss dielectric loss in the main cap is next on the list.

If you are using glass plates or SW caps this loss will also

be very large. Polyethylene dielectric will reduce this loss greatly.

I2R loss in the Tesla tank circuit can also account for loss in a poorly

built system. Adequately sized conductors for tank wiring and primary

coil will reduce these losses. Eliminate sharp points and put that

cap in oil to minimize corona loss.

If you can get that poly pipe cheap then by all means use

it but it probably wont make much difference in most typical coils.

A mediocre secondary coil coupled with a truly great primary system

will result in a good performing coil. But the worlds best low loss

secondary tossed into a poorly designed low Q primary system will

be a dud. Put that effort into the primary side.

TTYL!

Mike Hammer

mhammer@misslink.net

From: Dsurfr@aol.com

Date: Sat, 29 Aug 1998 16:03:05 EDT

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: dielectric voltage rating

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Hi all: Thanks for the reply Bert. It helps a lot (except I'll need to buy

more poly) The polypropylene sheets I'm thinking of using are .0035" thick so

it looks like 8 or 9 sheets should be OK? My next question is what uF rating

should I be building? WinTesla says .0053 is the ideal match for a 15kV 30mA

neon. My SW's are approx. .0058uF but that's a "guesstimate". Would I be able

to get away with a .005uF cap or would a .006uF be a better choice? Thanks for

all the help. I'll get this built yet!  Jim

Date: Sat, 29 Aug 1998 15:25:58 -0500

From: Bert Hickman <bert.hickman@aquila.com>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Organization: Stoneridge Engineering

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: 4 Inch Big Zappers

Sender: tesl-err@emachine.com

Kyle,

I fully agree with Mike!

PVC, when sanded, dried, and sealed makes a superb coil form (as will

LDPE, HDPE, acrylic, polycarbonate, etc. While PVC is somewhat lossy

when stressed with RF voltage, using it for a coilform really doesn't

measurably detract from a coil's performance at all. This is because the

RF voltage stress between the inside:outside of the coilform is minimal,

and is mostly spread across the entire length of the secondary winding.

PVC is an excellent dielectric, with superb high voltage breakdown

strength, moderately-good arc tracking resistance, and (with

pretreatment) very low leakage current.

Tesla would have LOVED PVC, since the lousy insulators he had to work

with were glass, wood, tar, oil, cotton, and gutta-percha (natural

rubber)! And ozone quickly destroyed the gutta-percha...

Don't be overly concerned about using PVC for your secondary coilform -

it's used in many high performance coils with great success.

-- Bert --

Tyla A. Break wrote:

>

>         Hi all, I've been reading the recent banter about 4" coils.  Kind

> of coil wrestling , 4" Category.

>         When looking for max. output you want to try to remove all losses

> right? I've heard many times that PVC is "lossey". How about using

> polyethylene pipe? It's available in all popular coil competition sizes

> 3,4,6,". I know it's not always perfectly round but it might be lower

> loss . It's cheap too.

>

>                         Kyle

>                 break7@juno.com

>

<SNIP>

Date: Sat, 29 Aug 1998 15:38:17 -0500

From: Bert Hickman <bert.hickman@aquila.com>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Organization: Stoneridge Engineering

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: dielectric voltage rating

Sender: tesl-err@emachine.com

Dsurfr@aol.com wrote:

>

> Hi all: Thanks for the reply Bert. It helps a lot (except I'll need to buy

> more poly) The polypropelene sheets I'm thinking of using are .0035" thick so

> it looks like 8 or 9 sheets should be OK? My next question is what uF rating

> should I be building? WinTesla says .0053 is the ideal match for a 15kV 30mA

> neon. My SW's are approx. .0058uF but that's a "guesstimate". Would I be able

> to get away with a .005uF cap or would a .006uF be a better choice? Thanks for

> all the help. I'll get this built yet!  Jim

Jim,

Tank capacitance versus Neon Sign Transformer (NST) size is not very

critical. Going to 0.006 uF should not be a problem, and using 0.005 uF

will definately be no problem for your 15/30. Just make sure you don't

open your gaps too widely, since you WILL be resonantly charging. This

will also be a bit tougher on your NST, since it will be providing more

cap recharge current than it was rated for. You may want to scout around

for some backup NST's "just in case". :^)

Good luck on your caps!

-- Bert --

From: Dsurfr@aol.com

Date: Sat, 29 Aug 1998 20:23:45 EDT

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: 2 Quick Questions

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Hi All:

While messing with my coil's tuning I placed a 1 1/2" D. chrome plated brass

doorknob on the center of my toroid. I noticed that the grounded connection I

was using as a strike point began to emit noticable corona & 4"-5" arcs to air

& then arced to the doorknob. I know lightning goes from the ground up but I

never noticed this effect from my coil before. It always seemed to arc from

the toroid to the ground.

Second question: How often should my safety gap arc? Since rebuilding my spark

gap I'm getting a frequent (though small) arc across it, where before it never

fired. I guess I'm probably over gapped (+/- 5/16" on a 15kV 30mA neon) & I

don't have any resistors on my RF Chokes (250 turns of 24G. wire) but I'm

worried about frying my neon. Thanks, as always,  Jim

Date: Sat, 29 Aug 1998 19:06:21 -0600 (MDT)

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Daniel Lockhart <molly@cmn.net>

Subject: Re: 2 Quick Questions

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

Jim wrote:

>While messing with my coil's tuning I placed a 1 1/2" D. chrome plated brass

>doorknob on the center of my toroid. I noticed that the grounded connection I

>was using as a strike point began to emit noticible corona & 4"-5" arcs to air

>& then arced to the doorknob. I know lightning goes from the ground up but I

>never noticed this effect from my coil before. It always seemed to arc from

>the toroid to the ground.

Yeah. Why is this? I've noticed the same thing too. I notice it most when a

grounded object that is sharp is brought close to the toroid. If I slowly

bring the tip of an aluminum ski pole to the toroid, sparks come off of the

ski pole and toroid and meet in the middle. What's up?

Daniel Lockhart

From: Ashpilot@aol.com

Date: Sat, 29 Aug 1998 22:22:51 EDT

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: 4 Inch Big Zappers

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Hello!!  I am building a new coil using a 3.25" cardboard tube for my

secondary form. I'm giving it a couple of coats of varnish, but with all the

talk about secondaries I was wondering if this would work out??  I haven't

started winding yet, so I can change now to PVC or what ever if I have to...

X-Mailer: IPAD 2.0

Date: Sat, 29 Aug 1998 21:33:54 -0500

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: 2 Quick Questions

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

At 07:06 PM 8/29/98 -0600, you wrote:

>Jim wrote:

>

>>While messing with my coil's tuning I placed a 1 1/2" D. chrome plated brass

>>doorknob on the center of my toroid. I noticed that the grounded connection I

>>was using as a strike point began to emit noticible corona & 4"-5" arcs to air

>>& then arced to the doorknob. I know lightning goes from the ground up but I

>>never noticed this effect from my coil before. It always seemed to arc from

>>the toroid to the ground.

>

>Yeah. Why is this? I've noticed the same thing too. I notice it most when a

>grounded object that is sharp is brought close to the toroid. If I slowly

>bring the tip of an aluminum ski pole to the toroid, sparks come off of the

>ski pole and toroid and meet in the middle. What's up?

>

>Daniel Lockhart

>

Jim and Dan,

The same mechanism is at work as in lighting.

The high voltage field creates corona around

many objects. During lightning storms this was

called "St. Elmo's Fire". As you both noticed

this effect is greatest around sharp pointed objects.

As your coil power increases you will note many

metal objects in the vicinity of the coil emitting

corona. You can actually draw small arcs off these objects

with a grounded wire.

>Second question: How often should my safety gap arc? Since rebuilding my spark

>gap I'm getting a frequent (though small) arc across it, where before it never

>fired. I guess I'm probably over gapped (+/- 5/16" on a 15kV 30mA neon) & I

>don't have any resistors on my RF Chokes (250 turns of 24G. wire) but I'm

>worried about frying my neon. Thanks, as always,  Jim

Jim,

5/16 inch (.3125) is a bit much for 15kv. I would back that down

about to around .300 inch and see if that lessens the safety gap

firings a bit.

To check out your chokes simply short them out with a clip lead

or something and fire up the coil again. Is there a lot more intense

activity at the safety gap now? If your chokes are lessening the

activity and lowering the intensity then they are doing their job.

BUT, if you actually get more safety gap activity with your chokes in

place then they are ringing up voltages that could be hazardous to your

neons health. Then you need the damping resistors.

TTYL!

Mike Hammer

mhammer@misslink.net

Date: Sat, 29 Aug 1998 21:47:19 -0500

From: Bert Hickman <bert.hickman@aquila.com>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Organization: Stoneridge Engineering

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: 4 Inch Big Zappers

Sender: tesl-err@emachine.com

Ashpilot@aol.com wrote:

>

> Hello!!  I am building a new coil using a 3.25" cardboard tube for my

> secondary form. I'm giving it a couple of coats of varnish, but with all the

> talk about secondarys I was wondering if this would work out??  I haven't

> started winding yet, so I can change now to PVC or what ever if I have to...

Ashpilot,

The main problem with cardboard is that it tends to absorb water out of

the air and then becomes slightly conductive. One way to reduce this is

to heat the cardboard from in the oven (just on "warm" until it's

thoroughly dry. Next coat the INSIDE of the form with a coating of

polyurethane or shellac and let it dry. Then repeat, and coat the

OUTSIDE in the same manner, but cover it with at least two coats. You'll

also want to glue on a top and bottom disk of plexiglass or similar

plastic to help prevent internal flashovers through the coil as well as

reduce future absorbsion of water. However, PVC pipe is a MUCH better

choice than cardboard, and a short scrap of 3" or 4" PVC should be

relatively cheap. BTW, what's your real name?

Safe coilin' to you!

-- Bert --

X-Originating-IP: [208.16.150.6]

From: "Bill the arcstarter" <arcstarter@hotmail.com>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: 4 Inch Big Zappers

Date: Sat, 29 Aug 1998 20:48:20 PDT

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

Uh,

(My grammar teacher never told me to never start a letter with "Uh", so

I guess it'd be ok...)

Let me just say that our 4.25 inch coil was 99% constructed via

information provided to me (via private communication) from Mike Hammer.

At the time we didn't know what the heck we were doing.  (Come to think

of it - not much has changed! :) )

So - fellow coilers - heed Mike's advice.  It worked out rather well for us!

Oh - we almost tried out the 6.25 inch coil this weekend (would've

worked better had I not left it at home...)  We almost had some

incredible arcs!  I wish all you guys could have almost seen it!

-Bill the arcstarter

Starting arcs in Cinci, OH

From: Ashpilot@aol.com

Date: Sun, 30 Aug 1998 10:01:01 EDT

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: 4 Inch Big Zappers

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Thanks Bert I guess I'll be heading to home depot...  BTW Real Name is Ed

Mullin from Nashua NH.  I fly small planes and the FAA abreviation for nashua

is ASH...

Date: Sun, 30 Aug 1998 14:03:25 -0700 (PDT)

From: Sam Barros <sambarros@yahoo.com>

Subject: Is this how it works?

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

Hi everybody! I hope you all get this message...

I'm new to the list, so let me first introduce myself:

My name is Sam Barros and I love science and electronics. I've some

experience with high voltages as I have built several HV circuits,

such as flyback, ignition coil and cascade drivers. Some were designed

by myself. My ignition coil driver puts out 60000V at reasonably high

power and fits inside a small shoebox. My cascade driver uses a 9V

battery to put out 25KV and is the size of a pack of cigarettes

(cascade included).

I also experiment with Microwave Oven Transformers and capacitor

discharge circuits. I saw you were having discussion whether it was

possible to drive a Jacobs ladder with such a transformer some time

ago. Well, heres what I did: I dipped the transformer in oil (to keep

it cool) and I placed a candle under the part where the spark

originated. The hot air from the candle makes the hot arc rise. At

500mA and 2500V, it puts out quite s show!!! You can have the ends

spaced out by almost 5inches, and the arc looks like a hot flame!

Another interesting thing to do with those transformers is to use

them to charge capacitors. I use 2 rated for 1.1uF, 2.5KV. You can

explode water with that!

I am now designing my first Tesla Coil. Here are the figures:

(sorry... I did it on Tesla Cad and they are all metric):

Secondary Coil Design:

Diameter of coil form: 20Cm

Height of coil Form: 70Cm

Winding Height: 60CM

Wire diameter: 0.60mm.

Spacing between turns: 0.00mm

Secondary wire length: 627.69M

Secondary turns: 999

Secondary inductance: 56.94mH

Approximate resonant Frequency: 186.11kHz

Secondary self-capacitance: 12.84pF

Toroid capacitance required: 18.31pF

Primary Winding Design:

Primary capacitance: 0.0127uF

Primary resonant frequency: 119.49kHz

Secondary coil diameter: 20cm

Primary conductor diameter: 9.50mm

Primary turn to turn spacing: 9.50mm

Spacing between the secondary and the inside turn of the primary:5.5CM

Primary tapped between turn 14 and 15 to form resonant circuit at >119.49Khz.

Primary type: Conical with 15degrees angle.

Power supply:

Transformer input voltage: 220V Transformer output voltage: 15Kv

Transformer rated current: 60mA. Transformer frequency: 50Hz

Capacitor required to form a resonant circuit at 50Hz: 0.0127uF

With 8 inches diameter and 25 inches high, Mike Hammer thinks my coil

is too big for a beginner. I agree with him BUT, its too late now I

have washed, sandpaper and dried the coil form. I had it varnished

inside an out with epoxy glue thinned with alcohol and I hand wound

1000 turns of enamel coated magnet wire on the coil form. I varnished

4 coats of high gloss polyurethane sealer on the turns and I covered

the entire coil with epoxy again. My ground connection is a 1 X 2

inches copper sheet glued on the bottom of the secondary. I also built

a toroid for it and I put acrylic caps on both sides. The coil is

pretty much perfect. There are no overlaps or gaps between the turns

and the epoxy/polyurethane insulation is nice and smooth, 0.5mm thick

inside, outside and over the turns.

I want this coil to be as efficient as possible. Please tell me if

you have ANY suggestion at all to make this a better coil. If you

think I should make a different secondary, use different wire, change

the primary, please say so! I really need all the help I can get to

make this a good coil and I would greatly appreciate and acknowledge

any help I get to make it more efficient.

Presently I am working on the primary. I think my design is not very

efficient Could someone tell me how to make it better? Mike suggested

making it flat. I know that would allow more ground distance between

the toroid and the strike rail, but it would also reduce efficiency,

wouldnt it? And I want efficiency. I will tap the toroid with a wire,

so ground strikes wont be a problem even at high power. Efficiency is

the problem.

Another thing I want to know is how high should the toroid be from

the last turn of the secondary. I know this has to be adjusted during

tuning , but I need a rough value...

And, could anyone tell me how to make a good, efficient RF choke?

I have 60 meters of .60 mm (between AWG 22 and 23) wire and two

ferrite cores 14cm long and 1cm diameter. Could they be used? How many

turns would I need? I already insulated the cores.

Thanks!!!!

If anyone wants some info on the High Voltage circuits and

experiments I have tried, just ask!

==

Sam Barros,

sambarros@yahoo.com

ICQ:15156975

"The less you know, the better you sleep"...

Date: Sun, 30 Aug 1998 15:00:25 -0700

From: Gene Waldo <gwaldo@1stnetusa.com>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Organization: S.A.P.I.E.N.T. (Surly, Agressive, Politically Incorrect Enemies of Non Thinkers)

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: 2 Quick Questions

Sender: tesl-err@emachine.com

Daniel Lockhart wrote:

>

> Jim wrote:

>

> >While messing with my coil's tuning I placed a 1 1/2" D. chrome plated brass

> >doorknob on the center of my toroid. I noticed that the grounded connection I

> >was using as a strike point began to emit noticible corona & 4"-5" arcs to air

> Yeah. Why is this?  What's up?

>

> Daniel Lockhart

Dan and Jim,

I think that the effect you see is simply the fact that sparks

concentrating from (or at) or to a point source from the diffuse air

path, become more visible as they concentrate near the source (destination).

Current flow direction should be irrelevent at these points.

Just my guess.

-----------------------------------------

Gene Waldo -  gwaldo@1stnetusa.com  - "Igor in Residence"

From: ESchulz531@aol.com

Date: Sun, 30 Aug 1998 18:41:27 EDT

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Is this how it works?

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Sam,

Your design seems correct although I did not work out the figures.  You

didn't mention what kind of primary spark gap you were going to use.  I am one

of those guys that think the primary spark gap is the most important part of a

tesla coil for efficiency.  A Richard Quick style gap would be a good choice

for the size of your coil.  A spark gap made of about 20-30 smaller gaps would

work just as well for this size of a coil and be much easier to build.  The

easiest way to space a lot of small gaps is with paper and glue.  Here is a

picture of a multi gap from Terry Fritz:

http://www.peakpeak.com/~terryf/tesla/experiments/sgap/Image93.jpg

I couldn't find a good picture of a RQG (Richard Quick style gap) but you can find

a lot of information on it at:http://www.velocity.net/~djb/files.shtml

Later,

Erik

"A tesla coiler sucked into college with no time to work on his coil's anymore. :("

From:          "Malcolm Watts" <MALCOLM@directorate.wnp.ac.nz>

Organization:  Wellington Polytechnic, NZ

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Date:          Mon, 31 Aug 1998 10:54:35 +1200

Subject:       Re: 2 Quick Questions

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

What exactly changed about your spark gap? What was the previous

design, spacing, firing voltage etc. and what are the same specs for

the new one?

Thanks,

Malcolm

> From:           Dsurfr@aol.com

> Date sent:      Sat, 29 Aug 1998 20:23:45 EDT

> To:             "Tesla-2 Mailing List" <tesla-2@emachine.com>

> Subject:        2 Quick Questions

> Send reply to:  "Tesla-2 Mailing List" <tesla-2@emachine.com>

> Hi All:

> While messing with my coil's tuning I placed a 1 1/2" D. chrome plated brass

> doorknob on the center of my toroid. I noticed that the grounded connection I

> was using as a strike point began to emit noticible corona & 4"-5" arcs to air

> & then arced to the doorknob. I know lightning goes from the ground up but I

> never noticed this effect from my coil before. It always seemed to arc from

> the toroid to the ground.

> Second question: How often should my safety gap arc? Since rebuilding my spark

> gap I'm getting a frequent (though small) arc across it, where before it never

> fired. I guess I'm probably over gapped (+/- 5/16" on a 15kV 30mA neon) & I

> don't have any resistors on my RF Chokes (250 turns of 24G. wire) but I'm

> worried about frying my neon. Thanks, as always,  Jim

From: Ashpilot@aol.com

Date: Sun, 30 Aug 1998 19:44:03 EDT

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Paralleling Transformers, not NST's.

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

I'm currently working with a Furnace type transformer, (OBIT) 10Kv/23 ma  I

was wondering if its possible to parallel two of there together like NSTs???

Has anyone tried it?

====

Ed Mullin

NH Coiler

Date: Sun, 30 Aug 1998 18:58:04 -0500

From: Bert Hickman <bert.hickman@aquila.com>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Organization: Stoneridge Engineering

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Paralleling Transformers, not NST's.

Sender: tesl-err@emachine.com

Ashpilot@aol.com wrote:

>

> I'm currently working with a Furnace type transformer, (OBIT) 10Kv/23 ma  I

> was wondering if its possible to parallel two of there together like NSTs???

> Has anyone tried it?

>

> ====

>

> Ed Mullin

> NH Coiler

Ed,

This works fine for neons and should be no problem for OBIT's as long as

they are the same voltage rating and you phase them properly.

-- Bert --

From: Ashpilot@aol.com

Date: Sun, 30 Aug 1998 21:13:40 EDT

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Paralleling Transformers, not NST's.

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Bert Wrote:

>This works fine for neons and should be no problem for OBIT's as long as

>they are the same voltage rating and you phase them properly.

>-- Bert --

How do I determine the phase?  Or is it not such a problem as long as they are

the same rating?? Finally how can I determine when they are in phase??

From: Dsurfr@aol.com

Date: Sun, 30 Aug 1998 21:21:16 EDT

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: 2 Quick Questions

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Malcolm: I'm just a relatively new coiler (although I made my first three

coils some 30 years ago & guess I became addicted then. I decided to get back

into it about a year ago). I'm nowhere near the league that you & some of the

others on the list are in, but if my information can be of any help in your

research, here it is. I'm using a 15kV 30mA neon transformer.My old gap was a

"modification" of the (2) 10 penny nails I used back when. I had (8) 1/4"

brass bolts mounted in a plexi box to give me (4) gaps in series. I had a 250

CFM muffin fan on a rheostat mounted at one end to force air over the gaps.

Total gap spacing was about .375". I liked the easy adjustability of the gap

(by turning the bolts & adjusting the fan speed) but thought I didn't have

enough surface area for good quenching. I intended to make a standard RQ gap

but after reading of Bert Pool's & Terry Fritz's designs I epoxied my (12)

pieces of 3/4" x 3" copper tubing horizontally on a piece of plexi, spaced at

.031" & mounted it at a 15 degree angle. I placed my muffin fan at the bottom

of this, blowing across the pipes. I connected at the 11th pipe (10 gaps @

.31" total). My spark output to ground has increased at least 3" (I'm still

messing around with the tuning) & it is much quieter. The arcs spurt across

the pipe sections, never seeming to stay in one place, so I assume my

quenching is improved thus increasing my output.

I'm going to try putting some 1/2" diameter pieces of copper rod on the ends

of the bolts on my old gap to increase the surface area for the arcs. D.C. Cox

(Dr. Resonance) recently replied with a posting that 3 gaps seemed to perform

best with a 30 mA neon & like I mentioned, I like the easy adjustability of

the gap with the bolts. I'll post my results (for what they're worth).

Glad for a chance to respond to you. Keep up your research & postings on it.

Jim

X-Mailer: IPAD 2.0

Date: Sun, 30 Aug 1998 20:30:42 -0500

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: Paralleling Transformers, not NST's.

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

At 09:13 PM 8/30/98 EDT, you wrote:

>Bert Wrote:

>

>>This works fine for neons and should be no problem for OBIT's as long as

>>they are the same voltage rating and you phase them properly.

>

>>-- Bert --

>

>How do I determine the phase?  Or is it not such a problem as long as they are

>the same rating?? Finally how can I determine when they are in phase??

Simply put if you connect two identical transformers in parallel

out of phase you get no output as the secondaries cancel each other

and in phase you get twice the current as the secondaries aid each other.

I just connect them in parallel and measure the current flow.

If the current flow isnt what is expected one of the transformers

is out of phase. You change the phase by swapping around one set

of primary leads.

You can also determine proper phasing by connecting the primaries

in parallel and connecting together just one set of the secondary

outputs. Plug in both transformers and carefully bring the other

two secondary terminals together. If you get a big arc then the

phasing on one transformer is incorrect. If you get just a tiny

spark or no spark the phasing is correct.

TTYL!

Mike Hammer

mhammer@misslink.net

From:          "Malcolm Watts" <MALCOLM@directorate.wnp.ac.nz>

Organization:  Wellington Polytechnic, NZ

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Date:          Mon, 31 Aug 1998 13:54:38 +1200

Subject:       Re: 2 Quick Questions

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Hi Jim,

Thanks for your reply. I'll chew it over. I am simply here

to listen and learn, no other reason. Your observations are valuable

to me as are other's experiences. If one type of gap gives no safety

gap fire and another type does and the type of gap is all that has

changed (?) in the primary circuit, spacing included, that tells me a

lot about gap behaviour which is an area I am keenly interested in.

Re the few gaps vs many, it may be things peculiar to the TC

characteristics which determine which is the better - i.e. there is

an ideal gap for every coil when running at a particular power level.

Your data is most helpful thank you. Would you mind just confirming

that the type of gap was all that changed - e.g. same chokes were

used etc.?  It does appear that your new gap has a smaller total gap

length, right?

Thanks,

Malcolm

> From:           Dsurfr@aol.com

> Date sent:      Sun, 30 Aug 1998 21:21:16 EDT

> To:             "Tesla-2 Mailing List" <tesla-2@emachine.com>

> Subject:        Re: 2 Quick Questions

> Send reply to:  "Tesla-2 Mailing List" <tesla-2@emachine.com>

> Malcolm: I'm just a relatively new coiler (although I made my first three

> coils some 30 years ago & guess I became addicted then. I decided to get back

> into it about a year ago). I'm nowhere near the league that you & some of the

> others on the list are in, but if my information can be of any help in your

> research, here it is. I'm using a 15kV 30mA neon transformer.My old gap was a

> "modification" of the (2) 10 penny nails I used back when. I had (8) 1/4"

> brass bolts mounted in a plexi box to give me (4) gaps in series. I had a 250

> CFM muffin fan on a rheostat mounted at one end to force air over the gaps.

> Total gap spacing was about .375". I liked the easy adjustability of the gap

> (by turning the bolts & adjusting the fan speed) but thought I didn't have

> enough surface area for good quenching. I intended to make a standard RQ gap

> but after reading of Bert Pool's & Terry Fritz's designs I epoxied my (12)

> pieces of 3/4" x 3" copper tubing horizontally on a piece of plexi, spaced at

> .031" & mounted it at a 15 degree angle. I placed my muffin fan at the bottom

> of this, blowing across the pipes. I connected at the 11th pipe (10 gaps @

> .31" total). My spark output to ground has increased at least 3" (I'm still

> messing around with the tuning) & it is much quieter. The arcs spurt across

> the pipe sections, never seeming to stay in one place, so I assume my

> quenching is improved thus increasing my output.

> I'm going to try putting some 1/2" diameter pieces of copper rod on the ends

> of the bolts on my old gap to increase the surface area for the arcs. D.C. Cox

> (Dr. Resonance) recently replied with a posting that 3 gaps seemed to perform

> best with a 30 mA neon & like I mentioned, I like the easy adjustability of

> the gap with the bolts. I'll post my results (for what they're worth).

> Glad for a chance to respond to you. Keep up your research & postings on it.

> Jim

Date: Sun, 30 Aug 1998 21:59:54 -0600 (MDT)

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Daniel Lockhart <molly@cmn.net>

Subject: Re: Is this how it works?

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

Welcome on the list Sam!

Your coil sounds great. I didn't go through your calculations, but

everything sounds okay to me. I would, however, make the primary flat, and

not at an angle. With only a 3:1 height/diameter ratio (the most

efficient), that doesn't leave a whole lot of space between the toroid and

the primary. I don't think there is much of an efficiency difference as

long as you get the right coupling. I basically built a flat primary for my

first coil, a small three incher (primary at 11.5 degrees) and got great

results. I've just started on my second coil, and am not very experienced,

so don't take my word for it though!...  :-)

An eight-incher? That's big! It wasn't until after I started sanding my

6.625-inch secondary I realised how big it was! How did you wind it? By hand

or did you rig up something?

>  Another interesting thing to do with those transformers is to use

>them to charge capacitors. I use 2 rated for 1.1uF, 2.5KV. You can

>explode water with that!

I know this is off-subject, but does anybody know the minimum amount of

energy it takes to explode water? I thought it took much, much more than

this. Bert? Bill?

Thanks,

Daniel Lockhart

X-Mailer: IPAD 2.0

Date: Mon, 31 Aug 1998 11:21:33 +0300

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Marco Denicolai <marco@vistacom.fi>

Subject: PFC capacitor for a MOT

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Does anybody know how to calculate the correct PFC capacitor value for a MOT?

In microwave ovens there is no PFC capacitor but I got the feeling that my

10A fuse blows (when I don't use any shunt) because I need one.

I use two MOT with primaries in parallel, and a single MOT in series (with

secondary shorted) as a shunt.

How to calculate a suitable PFC capacitor value?

Marco Denicolai

Date: Mon, 31 Aug 1998 02:51:52 -0700 (PDT)

From: Sam Barros <sambarros@yahoo.com>

Subject: Re: Is this how it works?

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

---Daniel Lockhart <molly@cmn.net> wrote:

>

> Welcome on the list Sam!

>

> Your coil sounds great. I didn't go through your calculations, but

> everything sounds okay to me. I would, however, make the primary flat, and

> not at an angle. With only a 3:1 height/diameter ratio (the most

> efficient), that doesn't leave a whole lot of space between the

> toroid and the primary.

I know... But I won't be using it to make huge toroid sparks. I'll

tap the toroid with a wire, so the energy will concentrate at the tip

of the wire and the toroid won't zap the primary.

> An eight-incher? That's big! It wasn't until after I started sanding my

> 6.625-inch secondary I relised how big it was! How did you wind it?

> By hand or did you rig up something?

Yep, it was damn hard! First I sawed the pipe to a length of 30

inches. Than I washed the whole thing with some detergent and I dried

it thoroughly with some tissue paper. I sandpapered it with rough

(#180), medium (#250) and fine (#400) sandpaper. It took a whole day

to get the thing sanded right. I placed it on top of a sawhorse and

sat on top of it. I than sanded it backwards and forwards several times.

I dried the coil form with a heat gun for one hour. Than I coated it

inside and out with thinned epoxy and PU sealer.

I wound it sitting on top of my bed, by hand. I used rubber gloves to

get a good grip on the wire and I used my other hand to turn the pipe.

I also pushed the turns together with my nails. It took 8 hours to

finish it. It was extremely boring, but I did it...

One thing that I recommend you to do: Try thinning some epoxy (slow

setting) with alcohol and paint thinner. You than have one hour to

coat your coil with it. When it dries, you have a beautiful, shiny,

thick coat of epoxy that is far stronger than polyurethane and

insulates electricity even better than it!

> I know this is off-subject, but does anybody know the minimum amount of

> energy it takes to explode water? I thought it took much, much more than

> this. Bert? Bill?

It depends on the amount of water. The 1.2 Kilojoules these

capacitors store can explode a drop. It's VERY loud. It sounds like a

rifle going off. I wouldn't recommend anyone to do this because it's

extraordinarily dangerous, but if you do, use earmuffs. I place the

drop of water on a car sparkplug.

I am trying to get some more capacitors so that I can explode wires.

They cost $25 each, so it will take a while for me to come up with the

money (I am already spending all I have on the coil)..

>

> Thanks,

>

> Daniel Lockhart

Thank you for replying Daniel!!

==

Sam Barros,

sambarros@yahoo.com

ICQ:15156975

"The less you know, the better you sleep"...

Date: Mon, 31 Aug 1998 11:05:01 +0100

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Stephen Bell <steve@breakfix.demon.co.uk>

Subject: MOTs

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Hi Guys,

A quick question about MOTs. Are they current limiting?  I saw

what looked like shunts between the two windings but wanted to be sure

before messing with it!

TIA

------------------------------------------------

Stephen Bell        (steve@breakfix.demon.co.uk)

Stuff also available at www.breakfix.demon.co.uk

------------------------------------------------

Date: Mon, 31 Aug 1998 03:07:10 -0700 (PDT)

From: Sam Barros <sambarros@yahoo.com>

Subject: Re: Is this how it works?

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

---ESchulz531@aol.com wrote:

>

> Sam,

> 	Your design seems correct although I did not work out the figures.

You didn't mention what kind of primary spark gap you were going to

use.  I am one of those guys that think the primary spark gap is the

most important part of a tesla coil for efficiency.

Right now I'm concentrating on having as efficient a coil as

possible. Caps, Transformers and sparkgaps will come later. I was

planning on using a gap like the one you mentioned though. Some 10 or

more stainless steel pipes spaced by 2mm or so, inside a glass box,

with some fans to suck air through it.

> Erik

>

Thanks for replying, Erik!

==

Sam Barros,

sambarros@yahoo.com

ICQ:15156975

"The less you know, the better you sleep"...

Date: Mon, 31 Aug 1998 03:39:41 -0700 (PDT)

From: Sam Barros <sambarros@yahoo.com>

Subject: Re: MOTs

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

> Hi Guys,

>         A quick question about MOTs. Are they current limiting?  I saw

> what looked like shunts between the two windings but wanted to be sure

> before messing with it!

>

> TIA

My 2500V, 0.5A MOT is, but if I run a Jacob's ladder with it for more

than a few seconds it overheats real bad... I'd use some current

limiting if I were you...

==

Sam Barros,

sambarros@yahoo.com

ICQ:15156975

"The less you know, the better you sleep"...

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Date: Mon, 31 Aug 1998 04:04:45 -0700

From: "paul m" <fleischwulf@mailexcite.com>

Cc:

X-Sent-Mail: off

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

Subject: Re: Is this how it works?

X-Sender-Ip: 208.153.7.9

Organization: MailExcite  (http://www.mailexcite.com:80)

Sender: tesl-err@emachine.com

Dale,

As for the exploding water, I don't think 3.4 Joules will come close. I think it is more in the upper 100's to a few kilojoules.

paul m

---

Einigheit und Wahrheit

On Sun, 30 Aug 1998 21:59:54   Daniel Lockhart wrote:

>>  Another interesting thing to do with those transformers is to use

>>them to charge capacitors. I use 2 rated for 1.1uF, 2.5KV. You can

>>explode water with that!

>

>I know this is off-subject, but does anybody know the minimum amount of

>energy it takes to explode water? I thought it took much, much more than

>this. Bert? Bill?

>

>Thanks,

>

>Daniel Lockhart

>

>

>

From: Hollmike@aol.com

Date: Mon, 31 Aug 1998 07:07:36 EDT

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Is this how it works?

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Sam,

I think you are on the right track.  Your coil could handle twice the

power easily though, so you may want to think about that once you have gotten

it to work and are ready to move on.  The only thing you will have to do is

double the power input and the primary cap and reduce the number of turns on

the primary accordingly.

Hope it works as well as it sounds like it should.

Mike Hollingsworth

Date: Mon, 31 Aug 1998 05:16:48 -0700 (PDT)

From: Sam Barros <sambarros@yahoo.com>

Subject: Exploding water...

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

---paul m <fleischwulf@mailexcite.com> wrote:

>

> Dale,

>   As for the exploding water, I don't think 3.4 Joules will come

close. I think it is more in the upper 100's to a few kilojoules.

>

> paul m

> ---

> Einigheit und Wahrheit

Paul,

I don't know exactly what the power rating for microwave capacitors

is. Please tell me if you know how to calculate it. The caps are rated

for 1.1uF and 2.5KV. They are really big! Some 5 inches tall and 3

inches wide... I estimated their energy storage to be 600J each, based

on the results of some similar capacitors. I could be wrong though...

Still, you can't change the facts... Connect 2 of those in parallel to

a .5A, 2500V Microwave transformer and connect their outputs to a

sparkplug and put a drop of water between the sparkplug's contacts.

You'll explode the drop of water with a flash of light, a VERY loud

bang and a cloud of steam. VERY COOL!!! But also dangerous...

Ill get myself some more of those so that I can explode larger

amounts of water. Ill try fitting the sparkplug to the end of a steel

pipe and using the exploding water to fire ball bearings (I know, this

is insane)

==

Sam Barros,

sambarros@yahoo.com

ICQ:15156975

"The less you know, the better you sleep"...

Date: Mon, 31 Aug 1998 07:19:20 -0500

From: Bert Hickman <bert.hickman@aquila.com>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Organization: Stoneridge Engineering

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: MOTs

Sender: tesl-err@emachine.com

Stephen Bell wrote:

>

> Hi Guys,

>         A quick question about MOTs. Are they current limiting?  I saw

> what looked like shunts between the two windings but wanted to be sure

> before messing with it!

>

<SNIP>

AND

from Marco:

> Does anybody know how to calculate the correct PFC capacitor value for a MOT?

<SNIP>

Stephen, Marco and all,

MOT's are very cheaply made, and the degree of current limiting provided

by the shunts varies considerably between manufacturers and even between

identical transformers. The amount of iron and copper used in these

transformers are JUST sufficient for their intended function. Most MOT's

require some type of external current limiting in order to prevent

excessive current flow and rapid overheating, since the shunts are really

there to limit the peak currents seen during abnormal arc-overs within

the magnetron or waveguide. Unlike a Neon Sigh Transformer (NST), these

shunts will not usually limit the current to a level that permits

sustained or pulsed short circuits such as a Jacob's Ladder or a

spark-gap in a Tesla Coil.

Since MOT's have minimal design margins, the heavy copper (I*I*R) and

iron losses cause them to rapidly overheat when they're overloaded.

Adding ballast inductance helps reduce the short-circuit currents to

levels that will allow you longer run-times without smokin' the

trannies, and using an inductive ballast improves overall efficienty.

Marco's question about adding Power Factor Correction (PFC) capacitance

to help neutralize the inductive reactance of the MOT's and Ballast is

an excellent one! Because the "load" looks inductive, the current is out

of phase with the applied voltage, and higher mains current will be

drawn to generate the same amount of real power in your system. Since

your mains fuses and circuit breakers only look at the amount of current

draw, they may trip even though you're not drawing a large amount of

real power.

Adding PFC capacitance (directly across the line) often helps to

significantly reduce the line current drawn for the same output power

level. Plan on around 80-100 uF of PFC capacitance for every 1000 VA of

inductive load at 60 Hz at 120 VAC. Only one fourth of this amount

(20-25 uF) will be necessary if you're system is running off 240 VAC.

Increase these values by about 20% if you're running off 50 Hz. In all

cases, the PFC's must be non-polarized metal film type (DON'T Use Motor

starting capacitors - motor "run" caps should be OK), and the voltage

rating should be at least 3X the incoming line voltage.

Note that adding PFC's only reduces the incoming current drawn... they

will not improve coil performance in any way. However, if you're on the

edge of popping a line fuse or breaker, they may reduce the incoming

current enough to prevent this from occuring.

Hope this helps!

-- Bert --

Date: Mon, 31 Aug 1998 07:02:30 -0700 (PDT)

From: Sam Barros <sambarros@yahoo.com>

Subject: Re: Is this how it works?

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

> What type of paint thinner did you use?

>

> bob golding

>

> bob yubba@clara.net

I used pure (96%) acohol and some ordinary paint thinner. Any kind

will do. The alcohol is more important though.. Don't use too much of

either.. The Thinner I bought just says "THINNER" on the bottle. It's

dutch, so I don't think you'll find where you live...

==

Sam Barros,

sambarros@yahoo.com

ICQ:15156975

"The less you know, the better you sleep"...

X-Originating-IP: [199.217.140.22]

From: "Bill the arcstarter" <arcstarter@hotmail.com>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Exploding water...

Date: Mon, 31 Aug 1998 07:05:46 PDT

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

Sam Barros

> I don't know exactly what the power rating for microwave capacitors

>is. Please tell me if you know how to calculate it. The caps are rated

>for 1.1uF and 2.5KV. They are really big! Some 5 inches tall and 3

>inches wide... I estimated their energy storage to be 600J each, based

>on the results of some similar capacitors. I could be wrong though...

Nope - not 600 J.  The energy stored in a capacitor is computed via

0.5*C*V*V, or, for your capacitor:

0.5*1.1e-6*2500*2500 = 3.43 Joules.

Not enough to explode too much! :)

-Bill

From: ESchulz531@aol.com

Date: Mon, 31 Aug 1998 11:29:29 EDT

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Is this how it works? (winding large coil forms)

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Hello,

When winding any size coil, if you make something to hold the form that

allows it to turn freely it makes the job about 2-5 times faster depending on

how good you are at winding.

When I wound my 10 X 36" PVC secondary I first made X's out of wood.  Then I

cut slots in the X's and hammered them onto the ends of the PVC.  I drilled a

hole in both sides, one on each X, making sure it was centered.  Then I dug

around in my basement looking for two 1' metal rods.  Then I dragged two

benches next to each other.  I placed the metal rods in the holes I drilled in

the wood.  I then set the secondary across the benches so that the PVC was in

air being held up by the metal rods laying against the benches.  Next I nailed

some nails around the metal rods on the benches.  I bent these over the rods

securing them in position.  This gave me a nice free turning coil form.  I

then brought a chair out side to sit on.  When winding this coil I used my

left hand to spin it and my right hand to guide the wire.  It took me 1 hour

to set up and 2 hours to wind.  I used all 10 lbs of my 20 gawg wire.

Side note: If you use 20 gawg or larger it makes it real easy to guide the

wire with only one hand at high speeds.

Erik Schulz

Date: Mon, 31 Aug 1998 08:33:06 -0700 (PDT)

From: Sam Barros <sambarros@yahoo.com>

Subject: Re: Exploding water...

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

> Nope - not 600 J.  The energy stored in a capacitor is computed via

> 0.5*C*V*V, or, for your capacitor:

>

>  0.5*1.1e-6*2500*2500 = 3.43 Joules.

>

> Not enough to explode too much! :)

>

> -Bill

I guess I was quite wrong about the ratings than... Still, it does

explode small drops of water. It's so loud!

Even a "dry" spark from this capacitor bank sounds like a powerful

rifle going off... Hehe.. it can turn worms into firecrackers :-)

Sam.

==

Sam Barros,

sambarros@yahoo.com

ICQ:15156975

"The less you know, the better you sleep"...

Date: Mon, 31 Aug 1998 08:40:14 -0700 (PDT)

From: Sam Barros <sambarros@yahoo.com>

Subject: Re: Is this how it works? (winding large coil forms)

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

Erik Schulz,

Thanks for the idea... I know that many people prefer to wind their

coils in such a way, but I preferred to do it by hand because I think

it gives better results (less overlaps and gaps), specially when you

do it for the first time...

Sam.

==

Sam Barros,

sambarros@yahoo.com

ICQ:15156975

"The less you know, the better you sleep"...

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Material Usage

X-Mailer: IPAD 2.0

From: break7@juno.com (Tyla A. Break)

Date: Mon, 31 Aug 1998 11:40:40 EDT

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Hi Everyone , Questions for anyone who knows.

I have recently aquired a large quantity of corian. Does anybody

know what it's made of? Looks a lot like ultra high molecular weight poly

(UHMW). But when it's being cut it smells like plexi.  I want to use it

for corraling my primary winding. I put a piece in the old microwave (RF

dissapation tester) and it barely gets warm. Also I want to use a piece

in my capacitor tank, how would it hold up to Diala AX?

Thanks, Kyle  break7@juno.com

From: ESchulz531@aol.com

Date: Mon, 31 Aug 1998 11:49:42 EDT

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Is this how it works? (winding large coil forms)

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Hi again,

All tesla coil secondary forms should never have an overlapping turn.  As far

as gaps in your winding, more coils you wind and the more time you spend

setting up the less you have.

Erik Schulz

Date: Mon, 31 Aug 1998 08:59:45 -0700 (PDT)

From: Sam Barros <sambarros@yahoo.com>

Subject: Re: Is this how it works? (winding large coil forms)

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

> Hi again,

> 	All tesla coil secondary forms should never have an overlapping turn.

>  As far as gaps in your winding, more coils you wind and the more time you

>  spend setting up the less you have.

>

> Erik Schulz

I know they should't have overlaps or gaps between turns. That's why

it took me 8 hours to wind the damn thing!

I don't plan on building a new coil very soon (that's why I want this

one to be so good), so time isn't much of a problem...

==

Sam Barros,

sambarros@yahoo.com

ICQ:15156975

"The less you know, the better you sleep"...

From: "Tobin Fricke" <fricke@mae.engr.ucdavis.edu>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: getting started

Date: Mon, 31 Aug 1998 09:05:33 -0700

X-Mailer: IPAD 2.0

X-MimeOLE: Produced By Microsoft MimeOLE V4.72.3110.3

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Hello.  I'd like to build a Tesla coil. I have no idea where to begin. Where

should I start?

Tobin Fricke

UC Berkeley

From: ESchulz531@aol.com

Date: Mon, 31 Aug 1998 12:06:47 EDT

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Is this how it works?

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

HI,

Your design sounds good.  The best way to get maximum efficiency is to try

more than one primary coil.

You can use a conical one like this old picture, way to much coil clearance

http://members.aol.com/tesla00/sec_sec.jpg

Or if you want maximum coupling you can try a multi layered coil like this

http://members.aol.com/tesla00/larger.jpg

And of course you can use a flat spiral

What do you mean by maximum efficiency?  Maximum energy transfer or maximum

spark length.  They are not the same...

For the longest spark you also want to try different toroid sizes.

Erik Schulz

http://members.aol.com/tesla00/

From: ESchulz531@aol.com

Date: Mon, 31 Aug 1998 12:14:05 EDT

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: getting started

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Hi,

A good source for information would be http://www.velocity.net/~djb/files.shtml

A really good paper on coil construction is:

http://www.ticnet.com/bertpool/building.htm

by Bert Pool.

Erik Schulz

http://members.aol.com/tesla00/

Date: Mon, 31 Aug 1998 09:19:32 -0700 (PDT)

From: Sam Barros <sambarros@yahoo.com>

Subject: Re: Is this how it works?

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

Thanks! What I mean by maximum efficiency is getting the maximum

power out with the minimum power in. Basically, I want to have a coil

that wastes as little energy as possible.

The result would be long sparks even with small transformers.

I think that's archieved by having good coupling between the primary

and the secondary, by using low-loss capacitors, by having a small

height to diameter ratio... Can you think of anything else?

==

Sam Barros,

sambarros@yahoo.com

ICQ:15156975

"The less you know, the better you sleep"...

From: ESchulz531@aol.com

Date: Mon, 31 Aug 1998 12:36:47 EDT

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Is this how it works?

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

>> 		Thanks!  What I mean by maximum efficiency is getting the maximum

>> power out with the minimum power in.  Basically, I want to have a coil

>> that wastes as little energy as possible.

>> The result would be long sparks even with small transformers.

>> I think that's achieved by having good coupling between the primary

>> and the secondary, by using low-loss capacitors, by having a small

>> height to diameter ratio...  Can you think of anything else?

And by getting good quenching of the primary spark gap. <- very important

Erik Schulz

http://members.aol.com/tesla00/

From: ralph.hartwell@emachine.com (Ralph Hartwell)

Subject:  Re: MOTs

Date: Mon, 31 Aug 1998 18:45:00 GMT

Organization: The Energy Machine Information System 504-733-3462

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

BH>MOT's are very cheaply made, and the degree of current limiting provided

BH>by the shunts varies considerably between manufacturers and even between

BH>identical transformers. The amount of iron and copper used in these

BH>transformers are JUST sufficient for their intended function. Most MOT's

<snip>

BH>Since MOT's have minimal design margins, the heavy copper (I*I*R) and

BH>iron losses cause them to rapidly overheat when they're overloaded.

<snip>

I use MOT's for building high voltage power supplies for RF power

amplifiers, and I can tell you they are built as cheaply as the mfgr.

can manage.  I've got several of them that overheat badly in about 20

minutes if you simply plug them in to the power line with no load on the

secondary at all.  Far too little iron in the core, I'm afraid, but,

they do work OK for their original use.

Ralph  W5JGV

---

X-Originating-IP: [199.217.140.22]

From: "Bill the arcstarter" <arcstarter@hotmail.com>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: MOTs

Date: Mon, 31 Aug 1998 12:00:33 PDT

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

Ralph wrote:

>   I use MOT's for building high voltage power supplies for RF power

>amplifiers, and I can tell you they are built as cheaply as the mfgr.

>can manage.  I've got several of them that overheat badly in about 20

>minutes if you simply plug them in to the power line with no load on the

>secondary at all.  Far too little iron in the core, I'm afraid, but,

>they do work OK for their original use.

I would agree with Ralph's comments.  My shunted microwave transformers

will still pull over 30 amps at 120 vac when the secondary is shorted

out!  I suppose the shunts are doing something - wonder what the SS

current would be w/o the shunts! Yow!

They certainly aren't as well behaved as an NST is, in terms of current

limiting, etc.

My (typical) microwave oven transformer has 121 turns of primary wire,

and has a measured inductance of 55 mH.  I don't know the core area.

I'm probably going to use a bunch of MOT cores for a custom wound

high-current choke for my DC welder.... Heh heh heh

I had a lot of fun hacking the secondary windings off of one (with a

hatchet!), and inserting a one-turn cable.  Conservation of flux

(amp*turns) and Faraday's law gave me, for this particular unit, about

0.9 volts per turn, and 1600 amp*turns total.  So - one turn gave me a

0.9 volt 1600 amp supply!  Sounds better than it really is, since you

can use up 0.9 volts *real quick* at 1600 amps :) , but it did have it's

uses...  (must use very thick cable to 'see' any large proportion of

this current)  I turned that MOT into a high current low voltage (3

volts) transformer for electroplating.

One of these days I'll do some saturation tests and report my findings

back to the list.

-Bill

X-Originating-IP: [199.217.140.22]

From: "Bill the arcstarter" <arcstarter@hotmail.com>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Exploding water...

Date: Mon, 31 Aug 1998 13:40:12 PDT

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

Hi Sam.

>> Nope - not 600 J.  The energy stored in a capacitor is computed via

>> 0.5*C*V*V, or, for your capacitor:

>>

>>  0.5*1.1e-6*2500*2500 = 3.43 Joules.

>>

>> Not enough to explode too much! :)

>>

>> -Bill

>

> I guess I was quite wrong about the ratings than... Still, it does

>explode small drops of water. It's so loud!

> Even a "dry" spark from this capacitor bank sounds like a powerful

>rifle going off... Hehe.. it can turn worms into firecrackers :-)

>

Well I've never exploded any water - so I'll take your word on it!  We

did notice that below about 600 J - there wasn't really any shockwave

effect while exploding #25 copper wires...

-Bill

From: wwl@netcomuk.co.uk (Mike Harrison)

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Exploding water...

Date: Mon, 31 Aug 1998 20:40:32 GMT

Organization: White Wing Logic

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

On Mon, 31 Aug 1998 05:16:48 -0700 (PDT), you wrote:

>---paul m <fleischwulf@mailexcite.com> wrote:

>>

>> Dale,

>>   As for the exploding water, I don't think 3.4 Joules will come

>close. I think it is more in the upper 100's to a few kilojoules.

I've had some nice bangs from my surge generator

www.netcomuk.co.uk/~wwl/surge.html

by dipping electrodes in a film canister lid filled with water. You

need a fairly high voltage - 2.5KV - nothing, but 4.5KV (30uF) - BOOM!

I've not yet had the courage or containment facilities to investigate

the ballistic properties of this.

____                                                           ____

_/ L_/  Mike Harrison / White Wing Logic / wwl@netcomuk.co.uk  _/ L_/

_/ W_/  Hardware & Software design / PCB Design / Consultancy  _/ W_/

/_W_/  Industrial / Computer Peripherals / Hazardous Area      /_W_/

X-Mailer: IPAD 2.0

Date: Mon, 31 Aug 1998 21:16:18 +0100

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: bob golding <yubba@clara.net>

Subject: safety gaps

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Hi All,

I have got my coil up and running again after rebuilding the cap,and

adding some rf filter chokes and series resistors ( 2mH and 1500 ohms at 50

watts) and EMI filters. I have fitted the filters on the chassis holding

the variac, one on the input and one on the output wired up backwards to

the normal orientation. Still getting interference on the neighbors TV

though. I think it might be their aerial, I told them I will fix them up

will a better one. Until then I am restricted to testing in the mornings.

They are quite interested in the coil though so that seems cool.

I am still trying to get the coil into tune. I am getting 15" sparks to a

ground rod with my 18" x 3" toriod with a breakout point. I am limiting the

gap to 100 mil as I need to break in my cap again and I don't want to put

too much power though the system yet.  I have a question about safety gaps.

I am using the "Gene Waldo special" type of gap with three bolts set at 120

degrees apart. Should the distance between

each "live" bolt and ground be just greater than the total main gap, or

should the total be split between the two "live" bolts and ground. EG main

gap 100 mil SG 120 mil each side, or 60 mil each side. I ask because my

safety gap does not fire at all now. I am hoping it is the chokes but I am

not sure and I do not want to fry my neon.

many thanks

bob golding

bob yubba@clara.net

From: "Choad" <choad69@geocities.com>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Exploding water...

Date: Mon, 31 Aug 1998 16:08:58 -0500

X-Mailer: IPAD 2.0

X-MimeOLE: Produced By Microsoft MimeOLE V4.72.3110.3

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

>Nope - not 600 J.  The energy stored in a capacitor is computed via

>0.5*C*V*V, or, for your capacitor:

>

> 0.5*1.1e-6*2500*2500 = 3.43 Joules.

>

>-Bill

Bill,

I thought the formula for joules was J=(5.*Volts)*C.

I get 0.001375 joules using this formula. It is in a motor controls

book I have for my electronic class. Is this a diffrent kind of

formula that your using?

-Shaun

X-Mailer: IPAD 2.0

Date: Mon, 31 Aug 1998 16:55:06 -0500

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: safety gaps

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

At 09:16 PM 5/21/97 +0100, you wrote:

>Hi All,

>

>  I have got my coil up and running again after rebuilding the cap,and

>adding some rf filter chokes and series resistors ( 2mH and 1500 ohms at 50

>watts) and EMI filters. I have fitted the filters on the chassis holding

>the variac, one on the input and one on the output wired up backwards to

>the normal orientation.

Sounds just fine to me Bob.

>Still getting interference on the neighbors TV though. I think

>it might be their aerial, I told them I will fix them up

>with a better one. Until then I am restricted to testing in the

>mornings. They are quite interested in the coil though so that seems cool.

Restricted operation is far better than banned operation.

At least if they show some interest they may be a bit tolerant.

Some things you can do to reduce interference include

making all tank circuit conductors as short as possible.

Less chance for them to radiate. Also make that ground

lead as short as possible.

>

>  I am still trying to get the coil into tune. I am getting 15" sparks to a

>ground rod with my 18" x 3" toroid with a breakout point. I am limiting the

>gap to 100 mil as I need to break in my cap again and I don't want to put

>too much power though the system yet.

Thats not too bad considering the rather narrow gap and its a good

way to break in your cap. When you add additional gap to your system

do it one at a time and see how your cap likes it.

>I have a question about safety gaps.

>I am using the "Gene Waldo special" type of gap with three bolts set

>at 120 degrees apart. Should the distance between each "live" bolt and

>ground be just greater than the total main gap, or should the total be

>split between the two "live" bolts and ground. EG main gap 100 mil SG

>120 mil each side, or 60 mil each side. I ask because my safety gap does

>not fire at all now. I am hoping it is the chokes but I am

>not sure and I do not want to fry my neon.

>

You want to divide the total up between the two sides

and ground. You are probably having very little kickback

and have a very smooth running coil with your rather narrow

gap. Your gap is also limiting the primary circuit voltage

which is exactly what you want while breaking in a new cap.

You can short out your chokes temporarily with a clip lead and

see if that makes any difference with the safety gap firings but

I bet it wont make much difference. You WILL start to get some

kickbacks as you widen your main gap.

TTYL!

Mike Hammer

mhammer@misslink.net

X-Mailer: IPAD 2.0

Date: Mon, 31 Aug 1998 18:16:24 -0400

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Jim Ryan <jryan@exis.net>

Subject: Re: Is this how it works?

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

>"A tesla coiler sucked into college with no time to work on his coil's

>anymore. :("

>

>Erik, I saw your spark gap. Slick job. What material did you use to mount

the copper pipe on?

Jim

X-Originating-IP: [208.16.145.150]

From: "Bill the arcstarter" <arcstarter@hotmail.com>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Exploding water...

Date: Mon, 31 Aug 1998 17:03:11 PDT

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

It was written:

>>Nope - not 600 J.  The energy stored in a capacitor is computed via

>>0.5*C*V*V, or, for your capacitor:

>>

>> 0.5*1.1e-6*2500*2500 = 3.43 Joules.

>>

>>-Bill

>

>Bill,

>

>    I thought the formula for joules was J=(5.*Volts)*C.

>I get 0.001375 joules using this formula. Is is in a motor controls

>book I have for my electronic class. Is this a diffrent kind of

>formula that your using?

Nope - I don't know what your formula is computing, but E=0.5*C*V^2 is

the only way to compute the energy in a capacitor.

-Bill

From: Hollmike@aol.com

Date: Mon, 31 Aug 1998 20:22:03 EDT

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: getting started

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Tobin,

I would suggest that you purchase a copy of John Couture's Tesla coil

construction guide.  You should be able to contact him through:

JHC Engineering Co.

10823 New Salem Point

San Diego, CA  92126

This book costs about $20 and is a great book for beginners.  I still open it

up frequently to check out things.  It also spells out everything you will

need to build your first coil.

Hope this helps,

Mike Hollingsworth

Date: Mon, 31 Aug 1998 18:24:13 -0700

From: Scott MacGregor <smacgregor@aasp.net>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Organization: EES,Co.

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: safety gaps

Sender: tesl-err@emachine.com

Mike Hammer wrote:

> Sounds just fine to me Bob.

>

> >Still getting interference on the neighbors TV though. I think

> >it might be their aerial, I told them I will fix them up

> >with a better one. Until then I am restricted to testing in the

> >mornings. They are quite interested in the coil though so that seems cool.

>

> Restricted operation is far better than banned operation.

> At least if they show some interest they may be a bit tolerant.

>

> Some things you can do to reduce interference include

> making all tank circuit conductors as short as possible.

> Less chance for them to radiate. Also make that ground

> lead as short as possible.

You might want to try 75 Ohm coax with matching xformers at the antenna

and TV, and a ground block/ground at the house entry point.

--

Regards,

Scott MacGregor

Date: Mon, 31 Aug 1998 20:42:48 -0500

From: Bert Hickman <bert.hickman@aquila.com>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Organization: Stoneridge Engineering

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Exploding water...

Sender: tesl-err@emachine.com

Mike Harrison wrote:

>

> On Mon, 31 Aug 1998 05:16:48 -0700 (PDT), you wrote:

>

> >---paul m <fleischwulf@mailexcite.com> wrote:

> >>

> >> Dale,

> >>   As for the exploding water, I don't think 3.4 Joules will come

> >close. I think it is more in the upper 100's to a few kilojoules.

> I've had some nice bangs from my surge generator

> www.netcomuk.co.uk/~wwl/surge.html

> by dipping electrodes in a film canister lid filled with water. You

> need a fairly high voltage - 2.5KV - nothing, but 4.5KV (30uF) - BOOM!

> I've not yet had the courage or containment facilities to investigate

> the ballistic properties of this.

>

>      ____                                                           ____

Mike and all,

Interesting discussion. For those who may be considering water-arc

experiments, please be VERY careful! Richard Hull and the EM theorist

Peter Graneau conducted some water-arc research a few years ago, using

only a few hundred joules. They were able to punch holes through 1/4"

aluminum plate using JUST the jet of high velocity water that was

created. The velocity of the ejected water jet(s) can be hypersonic, and

there's substantial risk of injury to nearby objects (including the

experimeter!).

A high energy spark under water creates a high-amplitude shock wave. If

not properly controlled, this has the potential to rupture the container

AND injure the experimeter. BTW, focused shockwaves (created by

capacitive discharges in a submerged sparkgap) are used in lithotrypsy -

a treatment for kidney stones. The shockwaves pass relatively harmlessly

through the water in your soft tissues, but breaks up the harder kidney

stones into sand INSIDE your body. Larger pulses can be used to do

hydrosonic metalforming (similar to explosive metalforming). Don't

inadvertantly become a part of your water-arc experiments...

Safe sparkin' to you!

-- Bert --

Date: Mon, 31 Aug 1998 20:59:49 -0500

From: Bert Hickman <bert.hickman@aquila.com>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Organization: Stoneridge Engineering

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Is this how it works?

Sender: tesl-err@emachine.com

Daniel Lockhart wrote:

>

<SNIP>

>

> >  Another interesting thing to do with those transformers is to use

> >them to charge capacitors. I use 2 rated for 1.1uF, 2.5KV. You can

> >explode water with that!

>

> I know this is off-subject, but does anybody know the minimum amount of

> energy it takes to explode water? I thought it took much, much more than

> this. Bert? Bill?

>

> Thanks,

>

> Daniel Lockhart

Daniel and all,

It doesn't take very much to atomize water, especially if an external

switching gap is used to "pulse" the water. Very little water appears to

be vaporized. Instead much of the energy appears to be converted to

kinetic energy. Exploding water in this manner can also be quite

dangerous - see my other post on exploding water.

-- Bert --

Date: Mon, 31 Aug 1998 21:22:37 -0500

From: Bert Hickman <bert.hickman@aquila.com>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Organization: Stoneridge Engineering

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Material Usage

Sender: tesl-err@emachine.com

Tyla A. Break wrote:

>

>         Hi Everyone , Questions for anyone who knows.

>         I have recently aquired a large quantity of corian. Does anybody

> know what it's made of? Looks a lot like ultra high molecular weight poly

> (UHMW). But when it's being cut it smells like plexi.  I want to use it

> for coraling my primary winding. I put a piece in the old microwave (RF

> dissapation tester) and it barely gets warm. Also I want to use a piece

> in my capacitor tank, how would it hold up to Diala AX?

>

>                 Thanks, Kyle  break7@juno.com

>

Kyle,

DuPont indicates that it's a mix of natural materials (ground rock and

minerals) and acrylic (same family as Plexiglas or PMMA). Assuming it's

not porous, it should hold up quite well for holding Diala-X or AX

(basically mineral oil). Safe cappin' to you!

-- Bert --