Copyright (C) 1992, 1995 Aladdin Enterprises.  All rights reserved.

This file is part of Aladdin Ghostscript.

Aladdin Ghostscript is distributed with NO WARRANTY OF ANY KIND.  No author

or distributor accepts any responsibility for the consequences of using it,

or for whether it serves any particular purpose or works at all, unless he

or she says so in writing.  Refer to the Aladdin Ghostscript Free Public

License (the "License") for full details.

Every copy of Aladdin Ghostscript must include a copy of the License,

normally in a plain ASCII text file named PUBLIC.  The License grants you

the right to copy, modify and redistribute Aladdin Ghostscript, but only

under certain conditions described in the License.  Among other things, the

License requires that the copyright notice and this notice be preserved on

all copies.

- - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - -

This file, devices.doc, gives more detailed documentation about

certain specific devices for which Ghostscript can produce output.

For an overview of Ghostscript and a list of the documentation files, see

README.

Devices for which this file currently contains documentation:

SPARCprinter

HP DeskJet 520, 540, and 560C

HP DeskJet 500C & 550C

HP PaintJet, XL, and XL300

DEC LJ250

Apple Dot Matrix Printer (and Imagewriter)

Epson Stylus Color Printer

Canon BJC-600/BJC-4000 and BJC-800 BubbleJet Color Printers

### ------------------------- The SPARCprinter ------------------------- ###

This section was written by Martin Schulte.

Introduction

------------

The SPARCprinter is is connected to SPARCStation via a special SBUS card's

video inferface, the picture is composed on the host and only a bitmap is

send to the printer unit.

Together with a SPARCprinter, you always buy (as far as I know) software

that enables you to do postscript-printing on your SPARCPrinter.

So, the need for a Ghostscript-Interface to the SPARCPrinter seems low,

but on the other hand some Postscript drawings are not correctly printed

with SUN's software: on some pages occured a thin vertical line of rubbish

(reproducable), on some Mathematica drawings the text at the axes wasn't

rotated.

I tried all of these with Ghostscript and always got the expected results.

However, replacing proprietary software should never be a bad idea.

The problem is that there has yet been no effort to make the SPARCPrinter-

driver behave like a BSD output-filter, I made my tests using the script

mentioned under Installation.

Installation

------------

Add sparc.dev to DEVICE_DEVS and compile ghostscript as described in

make.doc.

Afterwards, you can use the following script (the way of handling standard

input versus filename-arguments doesn't look very clever, has anyone a

better idea ?) to print if you substitute <GSPATH> by the place where you

installed the ghostscript binary:

outcmd1='/vol/local/lib/troff2/psxlate -r'

outcmd2='<GSPATH> -I/home/schulte/gs252 -sDEVICE=sparc -sOUTPUTFILE=/dev/lpvi0 -'

if [ $# -eq 0 ]

then

$outcmd1 | $outcmd2

else

cat $* | $outcmd1 | $outcmd2

fi

Problems

--------

Since /dev/lpvi can only be opened for exclusive use, another job having

opened it (engine_ctl_sparc or another ghostscript as the most probable

canidates) will cause to stop ghostscript with "Error: /invalidfileaccess

in --.outputpage--"

In case of common printer problems like out of paper, a warning describing

the reason will be printed to stdout, the driver will try to access again

and again each five seconds.

Due to a problem with the device-driver (in the kernel) the reason of

printer failure is not always correctly reported to program.  This is the

case at least if you open the top cover (Error in the display: E5).  Look

to the display at the printer if a "Printer problem with unknown reason"

is reported.

Fatal errors will cause the print-job to be terminated.

### ------------------------------ End --------------------------------- ###

### ------------------- H-P color inkjet printers ---------------------- ###

###  (DeskJet 500C, DeskJet 550C, PaintJet, PaintJet XL, PaintJet XL300  ###

###  and the DEC LJ250 which can operate in a Paintjet-compatible mode)  ###

This section was written by George Cameron.

Information and tips on usage for the drivers contained in gdevcdj.c

====================================================================

OVERVIEW:

There are 6 generic drivers contained in the source module:

1 - cdj500:      HP DeskJet 500C and 540C

2 - cdj550:      HP DeskJet 550C and 560C

3 - pjxl300:     HP PaintJet XL300 and DeskJet 1200C

4 - pjtest:      HP PaintJet

5 - pjxltest:    HP PaintJet XL

6 - declj250:    DEC LJ250

All of these drivers have 8-bit (monochrome), 16-bit and 24-bit

(colour) and for the DJ 550C 32-bit, (colour, cmyk mode)

options in addition to standard colour and mono drivers.

It is also possible to set various printer-specific parameters

from the gs command line, eg.

gs -sDEVICE=cdeskjet -dBitsPerPixel=16 -dDepletion=1 -dShingling=2 tiger.ps

NB/ The old names cdeskjet, cdjcolor and cdjmono drivers have been retained;

however, their functionality duplicates that available using the above

drivers (and cdeskjet is identical to cdj500), ie. we can use:

gs -sDEVICE=cdj500 -dBitsPerPixel=24 ...	for cdjcolor, and

gs -sDEVICE=cdj500 -dBitsPerPixel=1  ...	for cdjmono

DEFAULT PAPER SIZE:

If the preprocessor symbol A4 is defined, the default paper size is the

European A4 size; otherwise it is the U.S. letter size (8.5"x11"). Other

paper sizes (including A3 for the PaintJet XL and PaintJet XL300) may be

specified on the command line as explained in the Ghostscript documentation.

DEFAULT BITS-PER-PIXEL:

If the preprocessor symbol BITSPERPIXEL is defined as an integer (see below

for the range of allowable values), this number will be used to define the

default bits-per-pixel (ie. bit depth) for the generic drivers. If the

symbol is not defined, the default is set to 24 bits per pixel. It is

of course still possible to specify the value from the command line, as

described below. Note also that the cdeskjet, cdjcolor and cdjmono

drivers are unaffected by setting this symbol, as their default settings

are predefined to be 1, 3 and 24 respectively.

DESKJET PHYSICAL LIMITS:

Maximum printing width = 2400 dots = 8". The printer manuals say that the

maximum recommended printing height on the page is 10.3", but since this

is obviously not true for A4 paper, and I have been unable to detect any

problems in printing longer page lengths, this would seem to be a rather

artificial restriction.

All Deskjets have 1/2" unprintable bottom margin, due to the mechanical

arrangement used to grab the paper. Side margins are approximately 0.25"

for US Letter paper, and 0.15" for A4.

COMMAND LINE PARAMETERS:

Several printer 'properties' have been implemented for these printers.

Those available so far are all integer quantities, and thus may be

specified as eg.

gs -dBitsPerPixel=32 -dShingling=1 ...

which sets the BitsPerPixel parameter to 32 and the Shingling parameter

to 1.

BITS-PER-PIXEL:

All of the drivers in gdevcdj.c accept a command line option to set the

BitsPerPixel property. This gives considerable flexibility in choosing

various trade-offs between speed/quality/colour etc. The valid numbers

are:

1:	This is a standard Ghostscript monochrome driver, and uses

black ink (by installing the separate mono cartridge in

the case of the DeskJet 500C, or automatically for the

other printers)

3:	A standard Ghostscript colour driver, using internal

dithering. This is fast to compute and to print, but

the clustered dithering can lose some detail and

colour fidelity.

8:	An 'error-diffusion' monochrome driver which uses

Floyd-Steinberg dithering to print greyscale images.

The patterns are much more randomised than with the

normal clustered dithering, but the data files can

be much larger and somewhat slower to print.

16:	This is a 'cheaper' version of the following (24-bit)

driver, which generates a Floyd-Steinberg colour dithered

output using the minimum amount of memory (this may be

helpful when using IBM PC's when Ghostscript has not

been compiled using a 32-bit 386-style compiler). The

quality can be almost as good as the 24-bit version.

24:	A high-quality colour driver using Floyd-Steinberg dithering

for maximum detail and colour range. However it is very

memory intensive and thus can be slow to compute (and it

tends to produce rather larger raw data files, so they

can also be slower to print).

32:	This is for the DeskJet 550C only, which uses the black

cartridge and the colour cartridge simultaneously (ie.

CMYK printing). This printer can be both faster and give

higher quality than the DeskJet 500C, because of the

true black ink. (Note that the 24-bit mode also permits

CMYK printing on this printer, and uses less memory. Any

differences between 24-bit and 32-bit should be very small.)

DESKJET PROPERTIES:

The addional properties available for the DeskJets are:

BlackCorrect	(int)	  /* Colour correction to give

* better blacks when using the DJ500C

* in colour mode, eg. the default of 4

* reduces the cyan component to 4/5

* Range accepted: 0 - 9 (0 = none) */

Shingling	(int)	  /* Interlaced, multi-pass printing

* 0 = none, 1 = 50%, 2 = 25%, 2 is

* best & slowest */

Depletion	(int)	  /* 'Intelligent' dot-removal

* 0 = none, 1 = 25%, 2 = 50%, 1 best

* for graphics?

* Use 0 for transparencies */

PAINTJET XL300/PAINTJET XL PROPERTIES:

PrintQuality	(int)	  /* Mechanical print quality

* -1 = fast, 0 = normal, 1 = presentation

* Fast mode reduces ink usage and uses

* single-pass operation for some media

* types. Presentation uses more ink and

* max number of passes, ie. slowest

* printing for highest quality */

RenderType	(int)	  /* 0 = driver does dithering

* 1 = snap to primaries

* 2 = snap black -> white, others to black

* 3 = ordered dither

* 4 = error diffusion

* 5 = monochrome ordered dither

* 6 = monochrome error diffusion

* 7 = cluster ordered dither

* 8 = monochrome cluster ordered dither

* 9 = user-defined dither (not supported)

* 10 = monochrome user-defined dither ns. */

PAINTJET PROPERTIES:

No additional properties

GAMMA CORRECTION:

One consequence of using Floyd-Steinberg dithering rather than Ghostscript's

default clustered ordered dither is that it is much more obvious that the

ink dots are rather larger on the page than their nominal 1/180" or 1/300"

size (clustering the dots tends to minimise this effect). Thus it is often

the case that the printed result is rather too dark. A simple empirical

correction for this may be achieved by preceding the actual postscript

file to be printed by a short file which effectively sets the gamma for

the device, eg.

gs ... gamma.ps colorpic.ps -c quit

where gamma.ps is

%!

{0.333 exp} dup dup currenttransfer setcolortransfer

This example sets the gamma for r, g, and b to 3, which seems to work

reasonably well in practice.

GENERAL TIPS:

For all the above printers, the paper is critically important to the

final results. Smoother, less fibrous paper is generally better (and

suggested types are given in the printer manuals). In particular, the

special ink-jet paper can make a big difference; the colours are

brighter, but most importantly, there is almost no colour bleed, even

with adjacent areas of very heavy inking. Similarly, the special coated

transparencies also work well (and ordinary transparencies do not work

at all!)

The unix-lpr.sh provides one example of setting up a multi-option

colour postscript lpr queue on Unix systems, and includes the ability

to choose a range of different colour options and printer accounting

and error logging.

CAVEAT EMPTOR!:

It is not always easy for me to test all of these drivers, as the only

colour printer I have here is the DeskJet 500C. I rely on others testing

drivers for the additional machines and reporting their findings back to

me.

HP's 600x300 dpi resolution-enhanced mode for inkjet printers

=============================================================

This feature is available on HP's more recent inkjet printers,

including the Deskjet 520 (mono) 540 (mono or colour) and 560C (mono

and colour).

The colour and monochrome drivers for the HP deskjet 550c are

(probably) the best you will get for use with ghostscript, for the

following reasons:

These printers do not offer true 600x300 dpi resolution. Those that

print in colour are strictly 300x300 dpi in colour mode, while in mono

mode there is a pseudo 600x300 dot mode, with the restriction that you

can't print two adjacent dots. Thus, in effect what you have is 600 dpi

dot positioning, but on average you don't get more dots per line.

What this does give is the possibility to have eg. sharper character

outlines, as you can place dots on the edges nearer to their ideal

positions - this is why it is worth doing.

However, HP will not support user-level programming of this

resolution-enhanced mode, one reason being that (I understand) all the

dot spacing has to be done by the driver, and if you get it wrong, you

can actually damage the print head.

To summarise, you may lose a smidgin of (potential) text clarity using

the 550c drivers (cdj550, cdjcolor, cdjmono etc.), but other than that,

they are the ones for the job.

### ------------------------------ End --------------------------------- ###

### ------------------- Apple Dot Matrix Printer  ---------------------- ###

This section was written by Mark Wedel.

The Dot Matrix Driver (DMP) driver is a simple driver I wrote.  It

could more more efficient, but it seems to print the images fine.

The Dot Matrix Printer was a parallel predecessor to the Imagewriter

printer.  As far as I know, the Imagewriter commands are a superset

to those of the Dot Matrix printer, so the driver should work fine at

generating output that can be printed on Imagewriters.

A few notes (from the gdevadmp.c file):

* To print out images, it sets the printer for unidirection printing

* and 15 cpi (120 dpi). IT sets line feed to 1/9 of an inch (72 dpi).

* When finished, it sets things back to bidirection print, 1/8" line

* feeds, and 12 cpi.  There does not appear to be a way to reset

* things to initial values.

*

* This code does not set for 8 bit characters (which is required). It

* also assumes that carriage return/newline is needed, and not just

* carriage return.  These are all switch settings on the DMP, and

* I have configured them for 8 bit data and cr only.

*

* You can search for the strings Init and Reset (in devdemp.c) to find the

* strings that set up the printer and clear things when finished, and change

* them to meet your needs.

*

* Also, you need to make sure that the printer daemon (assuming unix)

* doesn't change the data as it is being printed.  I have set my

* printcap file (sunos 4.1.1) with the string:

* ms=pass8,-opost

* and it works fine.

Mark Wedel

master@cats.ucsc.edu

### ------------------------------ End --------------------------------- ###

### ------------------ The Epson Stylus Color printer ------------------ ###

/*

Epson Stylus-Color Driver, contributed by Gunther Hess (address: see below)

I N T R O D U C T I O N

=======================

This documentation accompanies version 1.20 of the stcolor-driver.

The new feature of this version is the support of deltarow encoding

and some new parameters for direct control of the ESC/P2-Output.

In comparison to the previous public distributed version (1.12) it is

almost a new driver.

Uff, stcinfo.ps finally made it into this release. For those of you,

who work with a ColorAdjustMatrix: The left Color-Triangle is

created without the ColorAdjustMatrix and the Color-Bars with the

primary colors ignore this Matrix too.

U S A G E

=========

This driver is selected with "-sDEVICE=stcolor" and produces output for an

Epson Stylus-Color at 360DpI resolution by default, but it can do much

more with this printer and with significantly better quality, than with

the default-mode and it can also produce code for the monochrome-versions

of this printer.

This can be achieved either via command-line options or via ghostscript-input.

For convienience a Postscript-File is supplied, that can be used as initial

inputfile. Thus, assumed that ghostscript is invoked via "gs" on your computer,

try the following command:

gs -sDEVICE=stcolor -rXDPIxYDPI stcolor.ps ... (e.g.: your input-files)

were XDPI is one of 180/360/720 and YDPI is one of 90/180/360/720. The result

should be significantly better, you may use "stcolor.ps" with other devices

too, but I do not recommend this, since it does nothing then. "stcolor.ps"

should be available with binary distributions and should reside in the

ghostscript input-directory. Thus if ghostscript is part of your

printer-spooler, you can insert

(stcolor.ps) findlibfile { pop run } if pop

to the files you want to run through the improved algorithms and you may want

to adapt this file to your specific needs. The methods and options for this

are described here, but this description is restricted to the gs-options, while

their manipulation at the Postscript-level is documented in "language.doc" and

in the mentioned "stcolor.ps".

Next thing is to explain the options (as written on my unix-system).

The order is somehow related to their use during the printing-Process:

-dUnidirectional      - Force unidirectional printing,

recommended for transparencies

-dMicroweave          - enable the printers "microweave"-feature

(the driver does weaving internally by default,

which can be applied to more X/Ydpi-combinations

and weaving by the driver reduces execution-time)

-dnoWeave             - disable any Weaving, overrides -dMicroweave

(Try it to see the effect of weaving - you won't

use this option again)

-sDithering="name"    - select another dithering-algorithm, available are:

"gscmyk"    : fast color-output, with CMYK-ProcessColorModel [D]

"gsmono"    : fast black & white output

"gsrgb"     : fast color-output, with RGB-ProcessColorModel

"fsmono"    : Floyd-Steinberg, Monochrome

"fsrgb"     : Floyd-Steinberg, with RGB-ProcessColorModel

(Almost identical to cdj550/mjcxxx-Algorithm)

"fsx4"      : Floyd-Steinberg, with CMYK-ProcessColorModel

(shares code with fsmono & fsrgb, but is

algorithmically really bad)

"fscmyk"    : Floyd-Steinberg, with CMYK-ProcessColorModel

and proper modifications for CMYK

"hscmyk"    : modified Floyd-Steinberg with CMYK-Model

("hs" stands for "hess" nor for "high speed",

but the major difference to "fscmyk" is speed)

"fs2"       : algorithm by Steven Singer (RGB)

should be identical to escp2cfs2.

-dBitsPerPixel=1...32    - number of bits used for pixel-storage, the larger

the value, the better the quality - at least in

theory. In fsrgb one can gain some speed, when

restricting to 24 Bits, rather than the default

of 30.

-dFlag0                  - causes some algorithms to select a uniform

initialisation rather than a set of random-values.

May yield "sharper" image-impression at the

cost of "dithering-atrefacts".

(applies to hscmyk and all fs-modi, except for fs2,

which always uses a constant initialization.)

-dFlag1 ... -dFlag4      - available to future algorithms.

-dColorAdjustMatrix={3/9/16 x float}'

- This is a Matrix to adjust the colors. Values should

be between -1.0 and 1.0, and the number of

values depend on the colormodel used by the

selected algorithm. In RGB- and CMYK-modi a matrix

with 1.0 on the diagonal produces no transformation.

(I could not identify a similar feature at the

language-level, so this option was implemented, it

is really required, but I don't know reasonable

values yet.)

-dCtransfer='{float float ...}', -dMtransfer=..., -dY..., -dK... or

-dRtransfer='{float float ...}', -dG..., -dB... or

-dKtransfer='{float float ...}'

- which is used, depends on the algorithm, which

maybe either either CMYK, RGB or monochrome.

The values are arrays of floats in the range from

0 to 1.0, which represent the visible

color-intensity for the device. One may achieve

similar effects with "setcolortransfer" at the

language-level, but this takes more time and the

underlying-code for the driver-specific parameters

is still required. The size of the arrays is

arbitrary and the defaults are {0.0 1.0}, which

is a linear characteristic, most of the code in

"stcolor.ps" are better transfer-arrays.

-dKcoding='{float...}', -dC..., -dM... etc.

- this are again arrays between 0.0 and 1.0, and

they control the internal coding of the

color-values. Clever usage of this arrays may

yield further enhancements, but no experience yet.

-dColorAdjustMatrix={3/9/16 x float}'

- This is a Matrix to adjust the colors. Values should

be between -1.0 and 1.0, and the number of

values depend on the colormodel used by the

selected algorithm. In RGB- and CMYK-modi a matrix

with 1.0 on the diagonal produces no transformation.

(I could not identify a similar feature at the

language-level, so this option was implemented, it

is really required, but I don't know reasonable

values yet.)

-sModel=st800            - causes output to be suitable for the monochrome

Stylus 800 (no Weaving, no Color).

-sOutputCode=            - can be either "deltarow", "plain" or "runlength"

and changes the ESC/P2 (TM) coding-technique used

by the driver. The default is to use the

runlength-encoding. "plain" selects uncompressed

encoding and yields enormeous amounts of data to

generated, while "deltarow" allows for

2D-compression, may reduce size of the data and

printing speed. But there is a *WARNING*

*WARNING* *WARNING*:      "deltarow" sometimes causes really bad hangs

on my printer. Conditions do not depend on the

data: sometimes times the same ESC/P2 prints well.

Thus "rle" seems to be adequate.

-descp_Band=1/8/15/24    - Number of Nozzles of scanlines used in printing.

Useful only with -dnoWeave, thus almost useless.

-descp_Width=            - Number of Pixels Printed in each scan-Line.

(Useful when tuning Margins only, se below)

-descp_Height=           - Length of the entire Page in Pixels

(Parameter of "ESC(C" in default initialization)

-descp_Top=              - Top-Margin in scanlines.

(1st Parameter of "ESC(c" in default initialization)

-descp_Bottom=           - Bottom-Margin in scanlines.

(2nd Parameter of "ESC(c" in default initialization)

-sescp_Init="..."        - Override for the initialization-Sequence.

(Must set Graphics-Mode-1 & Units)

Valid Resolutions:

-r180x90, -r180x180, -r180x360,    -r180x720

-r360x90, -r360x180, -r360x360(D), -r360x720

-r720x90, -r720x180, -r720x360,    -r720x720

Valid Option Combinations: (Setting them is *NOT* required, driver knows this)

escp_Band   ?Weave    escp_Band/#Passes

180x 90  15         no-Weave

180x180  1 , 8, 24  no/u-Weave      15/2 sWeave

180x360                             15/4 sWeave

180x720                             15/8 sWeave

360x 90  15         no-Weave

360x180  1,  8, 24  no/u-Weave      15/2 sWeave

360x360  1,  8, 24  no/u-Weave      15/4 sWeave

360x720                             15/8 sWeave

720x 90  15         no-Weave

720x180                             15/2 sWeave

720x360                             15/4 sWeave

720x720  1          no/u-Weave      15/8 sWeave

*************************************************************************

*************************************************************************

**                                                                     **

**  BEWARE: There are only few validity-checks for parameters. A good **

**          example is "escp_Band": if you set this, the driver tries  **

**          to use your value, even if this value is not supported by  **

**          the printer:                                               **

**                                                                     **

**                     YOU ASKED FOR IT, AND YOU GOT IT!               **

**                                                                     **

*************************************************************************

*************************************************************************

A P P L I C A T I O N  - N O T E

================================

Quite a bunch of Parameters. Hopefully you never need any of them, besides

feeding "stcolor.ps" to ghostscript in front of your input.

If you want to improve color, you may need to deal with the appropiate

parameters, please read the chapter about color adjustment. But

propably a sound background in color-models is required for that too.

(If you are such a guru, your help is welcome!)

One thing, that at least bothers me, are wrong margins or a too much

restricted print-area on the page. I definitely was unable to provide

a general solution for the Stylus Color and there are new Models up-

coming. Thus I decided to provide a maximum degree of freedom to the

ghostscript-user. That is what all the escp_-Stuff is meant for.

In Addition you may need the Standard-Parameters:

PageSize             - Two Floats Entire Page-Width & Height in 1/72"

.HWMargins           - four floats Left, Bottom, Right and Top-Margins 1/72"

Margins              - Two Integers, Negative, Left & Top Margin in Pixels.

(Adjusts the CTM so, that 1st Pixel of 1st Scan

matches the top-left Position on the Page)

Proably you should try MicroWeave or noWeave prior to Softweave,

with new printers. This mode is documented in the german owner's

maunual only. So now let's wait for Epson's 3600DpI-Printer. The

Driver is already capable of supporting it.

A C K N O W L E D G E M E N T S

===============================

This driver was "copied" from gdevcdj.c (ghostscript-3.12), which was

contributed by:

George Cameron      - g.cameron@biomed.abdn.ac.ukis

Koert Zeilstra      - koert@zen.cais.com

Eckhard Rueggeberg  - eckhard@ts.go.dlr.de

Some of the ESC/P2-code was drawn from gdevescp.c, contributed by

Richard Brown       - rab@physics.unimelb.EDU.AU

And several improvements are based on discussions with

Brian Converse      - ag899@osfn.rhilinet.gov

Gero Guenther       - gero@cs.tu-berlin.de

Jason Patterson     - jasonp@fit.qut.edu.au

? Rueschstroer      - rue@ibe.med.uni-muenchen.de

Steven Singer       - S.Singer@ph.surrey.ac.uk

While I wish to thank all this people mentioned above, they are by no means

responsible for bugs in the stcolor-driver - just for the features.

Duisburg 24-SEP-1995, Gunther Hess

up to sometime E-Mail:  hess@ims.fhg.de

After that time, one should use snail-mail or phone:

Gunther Hess                  phone: ++49 203 376273

Richard Wagner Strasse 112

D-47057 Duisburg

Germany

R E C O M M E N D A T I O N S

=============================

The next section is a contribution from Jason Patterson <jasonp@fit.qut.edu.au>,

who evaluated a previous version (1.17). GhostScript was invoked as follows:

gs -sDEVICE=stcolor [-r720x720] -sDithering=... -sOutputFile=escp.out \

stcolor.ps whatsoever.ps

where "..." is the name of the desired algorithm. "stcolor.ps" was omitted

for the gs-algorithms (gsmono, gsrgb and gscmyk), for which it is useless

*and* it would not allow the selection of "gscmyk".

So here comes a very truncated version of Jasons text:

COLOR DITHERING EXPERIMENTS with gdevstc-1.17

~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~

Here is a summary of the results for the following four experiments...

gsmono: What you'd expect from a mono ordered pattern. Like what a lot of

mono laser printers produce.

fsmono: Excellent for monochrome.

gscmyk: Not very good, but then you'd expect that from an ordered pattern.

gsrgb:  A little better than gsrgb. More consistent looking.

fs2:    Good, but not quite as good as fsrgb. Gets the brightness wrong,

too light at 720dpi, too dark at 360dpi.

fsrgb:  Very good, but a little too dark and has a slight blue tint.

hscmyk: Excellent. Slightly better than fsrgb and fs2. On some images, better

than fscmyk. On most, virtually the same.

fscmyk: Excellent. Best. Very, *very* slightly better than hscmyk. Nearly as

good as the EPSON demos (which were done with the Windows driver).

["nearly"? I believe 1.17 is better! G. Hess]

Overall Visual Quality (out of ten):

gsmono |*********

fsmono |*****************

gscmyk |********

gsrgb  |*********

fs2    |****************

fsrgb  |*****************

hscmyk |******************

fscmyk |******************

+---------------------

0 1 2 3 4 5 6 7 8 9 10

best-to-worst order:  color: fscmyk hscmyk fsrgb fs2 gsrgb gscmyk

mono: fsmono gsmono

SANITY NOTE: The above results are only from *four* images, a total of 24

printouts (8 on 720dpi paper, 16 on plain paper). Your results

will almost certainly vary, and your standards might not be

the same as mine, so use these results as a *guide* only, not

as a formal evaluation.

F U T U R E - D E V E L O P M E N T

===================================

Hmm, my husband forces me to say that there will be no future development,

but I think there is a fair chance for bug-fixing and for supplying a

resonable color-adjustment.

Any suggestions, for instance support of the new printer-models, is

welcome, provided that the one who suggests is willing to run some tests.

C O L O R - T R A N S F O R M A T I O N

=======================================

In the initial version of the driver, distributed with Ghostscript-3.33,

the parameter "SpotSize" was the only way to manipulate the colors at the

driver-level. According to the parameters enumerated above, this has changed

significantly with version 1.16 and above. This is the result of

an ongoing discussion about dithering-algorithms and "false color" on the

Epson-Stylus-Color. This initiated the transformation of the stcolor-driver

into a framework for different dithering-algorithms, that provides a generalized

interface to the internal Ghostscript-Color-Models and the other data-structures

related to Ghostscript-Drivers.

The main thing such a framework should be able to do is to deliver the

values the dithering-algorithm needs and since this influences directly

the optical image impression, this transformation should be adjustable without

the need for recompilation and relinking.

In general the process can be described as follows:

ColorAdjustMatrix   Coding                    Transfer

+---------------------+    +---------------------+    +------------------+

|  Ghostscript  Color |    | Ghostscript  Raster |    | Dithering  Data  |

|                     | => | 1/2/4/8/16/24/32Bit | => |   1/3/4x Values  |

|  1/3/4x16Bit Values |    | for all components  |    | (arbitrary type) |

+---------------------+    +---------------------+    +------------------+

Due to the limitations on raster-storage, information is lost in the first

transformation step, except for the 16Bit Monochrome-Mode. So any color

adjustment should take place before this step and this is where the optional

ColorAdjustMatrix works.

The first transformation-step is called "coding" and is controlled by the

?coding-Arrays. The Decoding-process expands the range of values

pontentially to a larger range than that provided by the initial ghostscript

color-model. It is therefore a reasonable place to make device- and/or

algorithm-specific adjustments. This is the place where the ?transfer-Arrays

are used. Array-Access might be not the fastest method, but its generality

is superior, so this step is always based upon internaly algorithm-specific

array-access. If 8Bits are stored per color-component and if the algorithm

uses bytes too, the second transformation is included within the first, what

saves significant computation-time when printing the data.

ColorAdjustMatrix

-----------------

The driver supports different "ProcessColorModel"-Values, which raises the

need for different color-adjustments. In the following "CAM" stands for

ColorAdjustMatrix:

DeviceGray: (3 Floats):

if((r == g) && (g == b))

K' = 1.0 - R;

else

K' = 1.0 - CAM[0] * R + CAM[1] * G + CAM[2] * B;

According to the documentation in drivers.doc, the latter should

never happen.

DeviceRGB: (9 Floats)

if((r == g) && (g == b))

R' = B' = G' = R;

else

R' = CAM[0]*R + CAM[1]*G + CAM[2]*B;

G' = CAM[3]*R + CAM[4]*G + CAM[5]*B;

B' = CAM[6]*R + CAM[7]*G + CAM[8]*B;

The Printer uses always four inks, thus a special treatment of black

is provided. Algorithms may take special action, if r==g==b. Maybe

that in future versions Kcoding & Ktransfer become active in RGB-Mode.

DeviceCMYK: (16 Floats)

if((c == m) && (m == y))

K' = max(C,K);

C' = M' = Y' = 0;

else

K  = min(C,M,Y);

if((K > 0) && ColorAdjustMatrix_present) { => UCR

C -= K;

M -= K;

Y  -=K;

}

C' = CAM[ 0]*C + CAM[ 1]*M + CAM[ 2]*Y + CAM[ 3]*K;

M' = CAM[ 4]*C + CAM[ 5]*M + CAM[ 6]*Y + CAM[ 7]*K;

Y' = CAM[ 8]*C + CAM[ 9]*M + CAM[10]*Y + CAM[11]*K;

K' = CAM[12]*C + CAM[13]*M + CAM[14]*Y + CAM[15]*K;

Again we have a special black-treatment. "max(C,K)" was introduced

because of a slight misbehaviour of ghostscript, that delivers

black under certain circumstances as (1,1,1,0). Normally, when

no special "Black Seperation" and "Undercolor Removal" procedures

are defined at the postscript-level, either (c,m,y,0) or (0,0,0,k)

values are mapped. This would make the extended ColorAdjustMatrix

quite tedious, thus during mapping black-seperartion is done for

(c,m,y,0)-Requests and if there is a ColorAdjustMatrix, undercolor-

removal is used too. In other words the Default-Matrix is:

1 0 0 1

0 1 0 1

0 0 1 1

0 0 0 1

and it is applied to CMYK-Values with seperated and removed Black.

Raising the CMY-Coefficients while lowering the K-coefficients

reduces black and intensifies color. But be careful, even low

deviations from the default cause drastic changes.

If no ColorAdjustMatrix is set, the matrix-computations are skipped. Thus

the transformation reduces to:

- Range-Inversion in Monochrome-Mode

- Black-Separation in CMYK-Mode

RGB/CMYK-coding & -transfer and BitsPerPixel

--------------------------------------------

This two (groups) of parameters are arrays of floatingpoint-numbers in the

range 0.0 to 1.0. They control the truncation to the desired number of

bits stored in the raster-memory (BitsPerPixel) and the ink-density.

The "truncation" may become a nonlinear-function, if any of the ?coding-arrays

are set. Assume the following Ghostscript invocation:

gs -sDEVICE=stcolor -sDithering=fscmyk -dBitsPerPixel=16 \

-dMcoding='{ 0.0 0.09 0.9 1.0 }' \

-dYtransfer='{ 0.0 0.09 0.9 1.0 }' \

-dKcoding='{ 0.0 0.09 0.9 1.0 }' -dKtransfer='{ 0.0 0.09 0.9 1.0 }' \

We may have ?coding and/or ?transfer, thus four combinations are possible

and this four combinations appear in the given example. The resulting mapping

is given in the following tables, where except for the internal Indices

(4 Components * 4 Bits = 16 BitsPerPixel), all values are normalized to the

Range 0-1. The actual range is 0 to 65535 for the ghostscript-color and

0 to 16777215 (2^24-1) for the ink-values delivered to the fscmyk-algorithm.

Sorry for the bunch of numbers following, but you may try this example in

conjunction with "stcinfo.ps", what should give you a graphical

printout of the following numbers, when you issue a "showpage"-command:

CYAN                      MAGENTA

CI/15 gs_color_values  CI    ink  gs_color_values  CI    ink

0.000   0.000 - 0.062   0  0.000   -0.123 - 0.123   0  0.000

0.067   0.063 - 0.125   1  0.067    0.123 - 0.299   1  0.247

0.133   0.125 - 0.187   2  0.133    0.299 - 0.365   2  0.351

0.200   0.188 - 0.250   3  0.200    0.365 - 0.392   3  0.379

0.267   0.250 - 0.312   4  0.267    0.392 - 0.420   4  0.406

0.333   0.313 - 0.375   5  0.333    0.420 - 0.447   5  0.433

0.400   0.375 - 0.437   6  0.400    0.447 - 0.475   6  0.461

0.467   0.438 - 0.500   7  0.467    0.475 - 0.502   7  0.488

0.533   0.500 - 0.562   8  0.533    0.502 - 0.529   8  0.516

0.600   0.563 - 0.625   9  0.600    0.529 - 0.557   9  0.543

0.667   0.625 - 0.687  10  0.667    0.557 - 0.584  10  0.571

0.733   0.688 - 0.750  11  0.733    0.584 - 0.612  11  0.598

0.800   0.750 - 0.812  12  0.800    0.612 - 0.639  12  0.626

0.867   0.813 - 0.875  13  0.867    0.639 - 0.715  13  0.653

0.933   0.875 - 0.937  14  0.933    0.715 - 0.889  14  0.778

1.000   0.938 - 1.000  15  1.000    0.889 - 1.111  15  1.000

The difference between Cyan and Magenta is the presence of a Coding-Array.

The coding-process must map a range of color-values to each of the 16

component-indices. If no coding-array is given, this is accomplished

by a division with 4096 -equivalent to a right-shift by 12 Bits-. The

final ink-density resides in the given interval and moves form the left to

the right side from 0 to 15. In the Magenta-case, there is a coding array

and the ink-value matches the center of the intervals. But the distribution

of the mapped intervals follows the given Coding-Array and is nonlinear in

the linear color-space of ghostscript.

Now let us take a look at the case with Transfer-Arrays:

YELLOW                      BLACK

CI/15 gs_color_values  CI    ink  gs_color_values  CI    ink

0.000   0.000 - 0.062   0  0.000     -0.123-0.123   0  0.000

0.067   0.063 - 0.125   1  0.018      0.123-0.299   1  0.067

0.133   0.125 - 0.187   2  0.036      0.299-0.365   2  0.133

0.200   0.188 - 0.250   3  0.054      0.365-0.392   3  0.200

0.267   0.250 - 0.312   4  0.072      0.392-0.420   4  0.267

0.333   0.313 - 0.375   5  0.090      0.420-0.447   5  0.333

0.400   0.375 - 0.437   6  0.252      0.447-0.475   6  0.400

0.467   0.438 - 0.500   7  0.414      0.475-0.502   7  0.467

0.533   0.500 - 0.562   8  0.576      0.502-0.529   8  0.533

0.600   0.563 - 0.625   9  0.738      0.529-0.557   9  0.600

0.667   0.625 - 0.687  10  0.900      0.557-0.584  10  0.667

0.733   0.688 - 0.750  11  0.920      0.584-0.612  11  0.733

0.800   0.750 - 0.812  12  0.940      0.612-0.639  12  0.800

0.867   0.813 - 0.875  13  0.960      0.639-0.715  13  0.867

0.933   0.875 - 0.937  14  0.980      0.715-0.889  14  0.933

1.000   0.938 - 1.000  15  1.000      0.889-1.111  15  1.000

Yellow uses a transfer-array. There is no linear correspondence between

the color- and the ink-values. This correspondence is defined through the

given array. In other words: the Transfer-arrays define a nonlinear

ink-characteristic, what is exactly the same functionaltity, that

Postscripts "(color)transfer"-function provides.

While in the case of Yellow, the intervals match the intervals used with Cyan,

the inetervals used for Black match the Magenta-Intervals, but watch

the corespondence between the CI/15-values and the Ink-Density for Black:

This is a linear distribution in the Ink-domian.

Not a bad idea, I think. Consider the fs2-algorithm: It uses values in

the range 0-255 (Bytes). If any transfer-array would be supplied alone,

some of the 256 possible values would never be used and others will be

used for adjacent intervals several times. Establishing an identical

coding-array solves this problem, so that the full potential of the

algorithm is utilized.

Another useful feature of the coding-arrays is, that they are internally

normalized to the 0-1 Range. In the 720x720Dpi-Mode the transfer-arrays

in stcolor.ps limit the Dot-Density to about 50%, thus this arrays end

at 0.5 (respectively start at 0.5 in the RGB-case). Due to the automatic

normalization this arrays can be used as coding-arrays too. But of course

in the fs2-case mentioned above, values from 0-127 will never be deliverd

to the algorithm, while values 128-255 are delivered for adjacent intervals.

To clearify the intended use of the three parameters/parameter-groups the

following statements should be kept in mind:

- ColorAdjustMatrix is never used, when transferring gray-values. This

restricts it to what the name says: Adjustment of Colors e.g. the

correction for miscolored ink. Do not use it for staturation or

brightness-control.

- ?transfer-arrays control the values delivered to the driver, which in

turn controls the ink-quantity. This arrays should be used for control

of saturation and brightness. Maybe that a Postscript-Header for the

manipulation of brightness and so on will be provided with future

versions. In general this arrays are identical for all inks.

If they differ they provide a simpler scheme for color-correction,

which is not necessarily faster than the ColorAdjustMatrix.

- ?coding-arrays control the color-value-intervals mapped to

the internal color-indices.

P R I N T - M O D I

===================

The parameters "Unidirectional", "Microweave", "noWeave",

"OutputCode", "Model" and the given resolution provide control over the

data generated for the printer.

Unidirectional

--------------

Simply toggles the unidirectional-mode of the printer. Setting "Unidirectional"

definitly decreases printing-speed, but may increase the quality. I use

this for printing tranparencies, where fast head-movement could smear the ink.

Microweave, noWeave and OutputCode=deltarow

-------------------------------------------

The first are two Booleans, what immediatly tells, that 4 combinations are

possible. Actually only three exist (if you don't count for deltarow):

1. Softweave

2. Microweave

3. noWeave

First and second are functionally identical, their difference is that either

the driver or the printer does the job. So the question

What is weaving ?

arises. The Epson Stylus Color has a Head-Assembly that contains two physically

identifiable Heads. One for Black and one for Cyan/Magenta/Yellow. This

makes 4 logical Heads, one for each color-component. Each of this four heads

has several jets at some Y-distance, so several horizontal lines can be printed

during one pass of the heads. From the experience I think there are 15 Jets

per color spaced at 1/90".

So the question arises, how to print at a Y-Resolution of 360Dpi with this

90DpI-Jets. Simply by divison, one gets 360/90 = 4, what tells us, that

4 Passes of the head-assembly are required to achieve a Y-resolution of

360DpI. Weaving is just the scheme how the 15 jets are utilized to print

adjacent horizontal rows:

Weaving                   noWeave

Pass:     1     2     3     4       1     2     3     4

0/360"   jet0   -     -     -      jet0   -     -     -

1/360"    -    jet1   -     -       -    jet0   -     -

2/360"    -     -    jet2   -       -     -    jet0   -

3/360"    -     -     -    jet3     -     -     -    jet0

4/360"   jet1   -     -     -      jet1   -     -     -

5/360"    -    jet2   -     -       -    jet1   -     -

6/360"    -     -    jet3   -       -     -    jet1   -

....

Now let us assume, that the dot-diameter is different for each individual

jet, but the average among the jets matches the desired resolution. With

weaving adjacent rows are printed by different jets, thus the some averaging

takes place. Without weaving adjacent rows are printed by the same jet and

this makes the dot-diameter-deviations visible as 1/90"-stripes in the printout.

In Softweave-Mode (the default) the driver sends the data properly arranged to

the printer, while in Microweave-Mode the printer does the same job. But in

general the host-processor is much faster than the printers processor and

thus it is advantageous to let the host do this job. In addition to that, for

720DpI 8 Passes are required and the amount of buffer-space required to buffer

the data for the passes is far beyond the printers memory. SoftWeave requires

an odd value of "escp_Band", the Stylus Color provides 15 for that.

"OutputCode" controls the encoding used. In the basic modi, the choice consists

of "plain" and "runlength". The computation of runlength-encoded data does not

take much time, at least less than the datatranfer to the printer, thus this

is the recommended mode and of course the default. With the Stylus Color

Epson introduced some new encoding principles, namely "tiff" and "deltarow".

While the first was omitted from this driver, since there were not potential

advantages found, "deltarow" is available as an option. "Softweave" cannot

be used with this encoding, so if "OutputCode=deltarow" is set, Microweave

becomes the default. Maybe that the size of the ESC/P2-code becomes smaller,

but I have never observed increased printing-speed - things tend to become

slower with deltarow compared to Softweave.

Model

-----

Some ESC/P2-Printers, such as the Stylus 800, do not offer Microweave or

the commands required to do Softweave. Setting Model just changes the defaults

and ommits some parts of the initialization-sequence, which are not compatible

with the given printer model. Currently only "st800" is supported besides the

default (stcolor).

BEWARE: BUGS & PITFALLS

=======================

* The given ?coding and ?transfer arrays should be strictly monotonic.

* It is impossible to change WHITE: that's your paper.

Thus R/G/B-transfer should end at 1.0 and C/M/Y/K-transfer should

start at 0.0.

* Usually 8Bits per component yields fastest operation.

* The ColorAdjustMatrix is not used in the reverse-transformation, which

is used, when Gostscript does the dithering (gs*-Modi). Expect funny

results.

* If BitsPerPixel is less than 6, the entire coding/transfer-process

does not work. This is always true for the gs*-modi and becomes true

for the other modi, if BitsPerPixel is forced to low values.

* 720x720Dpi-Printing should never select the gs-modi and should always

use stcolor.ps. (I prefer 360x720)

A D D I N G   D I T H E R I N G   A L G O R I T H M S

=====================================================

Access to the Ghostscript-sources is required to add a new dithering-algorithm.

The driver-source includes 9 Files:

gdevstc.h    - common C-Header, including algorithm-declarartions

gdevstc1.c   - "gsmono"-Algortihm

gdevstc2.c   - fs-algorithms (access via "fsmono", "fsrgb", "fscmyk")

gdevstc3.c   - "gsrgb"-Algorithm

gdevstc4.c   - "fs2"-Algorithm

gedevstc.c   - basic driver source + gscmyk & hscmyk

stcolor.ps   - Postscript-header with ?transfer-Arrays

stcinfo.ps   - Test-Page for the Printer

and requires modification of:

devices.doc  - That's were this file goes to

devs.mak     - device-specific Make-Rules

unix-end.mak - to add the installation-rule for stcolor.ps

In Beta-versions of the driver this documentation resides in a

seperate file named "gdevstc.doc".

While all sources are required to build the driver, only gdevstc2.c and

gdevstc4.c contain real dithering-algorithms, the other three

"algorithm-sources" are considered to be templates to add new algorithms.

Besides writing the source, a modification in "gdevstc.h" is required

(search for "MODIFY HERE" and follow the instructions).

The Algorithm should be implemented as a single function, with the following

arguments:

int stc_newalgo(                  /* or whatever you like */

stcolor_device *sdev,          /* the device-structure */

int     npixel,                /* Number of Bits/Pixels */

byte   *in,                    /* pixel-values */

byte   *buf,                   /* private buffer */

byte   *out);                  /* one byte per pixel output */

with "in" and "buf" being actually pointers to either "byte", "long" or

"float" values.

The array "in" holds one item of the requested type (byte/long/float) per

color-component (1:K, 3: R G B, 4: C M Y K) for each pixel. Alternatively

the algorithm can request the internal ghostscript-scanline as input, by

setting the flag "STC_DIRECT". This saves memory and potentially time.

The driver-specific values can be found in

((byte/long/float *) (sdev->stc.vals[component]))[component-value]

by such direct-drivers. If a driver works on bytes in most cases the

driver-specific values are stored within the ghostscript-line what

implicitly includes "STC_DIRECT".

Obviously the routine should deliver output to the array of bytes named "out".

It holds one byte per pixel and values should be composed by or'ing

the constants

DevciceGray -> BLACK

DeviceRGB   -> RED GREEN BLUE

DeviceCMYK  -> CYAN MAGENTA YELLOW BLACK

together (There is no "WHITE" defined, since it is different in RGB and

the other color-modes). The initial contents of "out" is the result of the

previous call to the routine. The routine should deliver 0 upon success.

negative values supress printing (only checked upon the initialization-call).

Besides the "normal" invocation, there are two special calls:

Initialization: npixel holds the NEGATIVE number of pixels then.

Initialization should initialize buf (out is already set to white)

and check the parameters.

White-Call: "in" is set to NULL then. This type of call must be

requested by setting the flag "STC_WHITE".

There is another interesting Driver-Flag, named "STC_CMYK10". This is

useful only for CMYK-Drivers and yields 10Bit Component-Values. Thus

higher color-resolution is achieved. BitsPerPixel cannot be manipulated

with the CMYK10-Coding, it is fixed to 32. The coding is as follows:

gx_color_index: aaaa aaaa aabb bbbb bbbb kkkk kkkk kktt

the "tag" (tt) controls what colors are represented by "a" and "b":

00 -> C = k, M = a, Y = b, K = k

01 -> C = a, M = k, Y = b, K = k

02 -> C = a, M = b, Y = k, K = k

03 ->                      K = k, (gray-level)

thus the value "(gx_color_index)" 3 represents white with this coding.

Another specificum with this CMYK10-Coding is, that the Ghostscript-Line

represents an array of "gx_color_index"-values and can be manipulated

as such. This saves reasonable processing-time for direct drivers.

Any "normal" CMYK-Driver defaults to CMYK10-Coding, if:

1. It is not a DIRECT-Driver and

2. The range of Values covers more than 8 Bits

Any setting of BitsPerPixel causes such a driver to fall back to the

normal CMYK-Mode.

T E S T S (version 1.13 and above)

==================================

This section should give an overview over the performance in terms of

processing- & printing-time. Printing is done offline (via cp-instruction)

to measure real printing-speed, since at high-resolutions processing-time

is in the same order of magnitude and thus may become the limiting factor.

The various OutputCodes

-----------------------

I ran several files though ghostscript and recorded the size of the code,

the processing time and the printing-time, at least for some of the files.

Always the following options were used:

"-sDEVICE=stcolor -sPAPERSIZE=a4 stcolor.ps - < file.ps"

(Actually "-sPAPERSIZE=a4" is in my gs_init.ps since I'm a germ.)

"Softweave" means actually, that nothing else was used, it is the default and

implies that odd v=40/h=10/m=15 mode (ESC . 1 40 10 15).

"Microweave" is just "-dMicroweave", which is equivalent to "ESC . 1 10 10 1",

with full skip-optimization and microweave-activated.

"deltarow" is the new encoding principle ("ESC . 3 10 10 1") with Microweave on.

It is activated with "-sOutputCode=deltarow".

Finally I wanted to see the plain Kathy Ireland and used "-sOutputCode=plain",

which is just replacing RLE by no encoding, thus "ESC . 0 40 10 15" is

used then.

(So sorry: Kathy was still blue dressed in front of the blue sea on a blue

air-cushion - nice to see but hard to dither)

So here are the results:

golfer.ps    colorcir.ps      drawing.ps        brief.ps

deltarow   572751/48.180u 643374/41.690u  90142/46.180u/1:50 178563/49.350u/2:22

Softweave  559593/46.810u 669966/44.960u 296168/48.160u/1:30 269808/43.320u/1:55

Microweave 590999/56.060u 754276/42.890u 338885/47.060u/1:50 282314/44.690u/2:22

kathy.ps

deltarow   3975334/111.940u/5:35

Softweave  3897112/101.940u/3:10

Microweave 4062829/100.990u/3:15

plain/soft 5072255/104.390u/3:05

Evaluation:

A.) Might be, that I've not choosen the optimal deltarow-code, but even if

it saves at lot of bytes, printing-speed is not increased.

B.) At least the printer prefers plain-kathy. In other words: Sending a

1 Megabyte or 20% more data, has no impact on printing speed.

[drawing.ps is an exception to this rule: plain prints slower than rle]

C.) But "unclever" coding -especially with deltarow- can significantly

slows down printing. But even if very significant advantages in the

size of the code ar achieved, "deltarow" is not competitive.

[colorcir.ps shows savings in deltarow, but printing is a mess.]

Printing-Time related to other options

--------------------------------------

Full page halftone images printed, unless otherwise noted.

DpI      print-mode    Size   Time comments

180x180 mono      -/uni  358KB  1:15

-/bi   358KB  0:45

micro/bi   205KB  0:45 (not weaving)

soft/bi   179KB  1:25

color     -/bi   641KB  2:45

soft/bi   556KB  1:32

360x360 mono      -/uni  269KB  0:50 (b/w Text)

-/bi   269KB  0:35 (b/w Text)

micro/bi   269KB  2:25 (b/w Text)

soft/uni  250KB  3:15 (b/w Text)

soft/bi   250KB  1:55 (b/w Text)

color     -/bi   346KB  1:00 (sparse color-page, visible displacements)

micro/bi   346KB  1:50 (sparse color-page, looks buggy - printer?)

soft/bi   294KB  1:30 (sparse color-page, O.K.)

-/bi  2218KB  2:45 (visible stripes)

micro/bi  5171KB  3:17

soft/bi  3675KB  3:05

360x720 mono   soft/bi  2761KB  5:40

color  soft/bi  7789KB  6:15 (just a small difference!)

720x360 color  soft/bi  7182KB  5:40

720x720 color micro/bi 14748KB 30:26 (actually beyond printers capabilities)

soft/bi 14407KB 11:08

Processing-Time

---------------

This table is generated on a 16MB i486/DX2 running under Linux, it is

extended for each version of the driver. User- (u) and System-Time (s)

is given seperately in seconds. Besides the files distributed with

ghostscript, the following inputs are used:

tropics.ps:  6MB-Filesize, 1152x900x24 RGB-Image. (entire page covered)

brief.ps:    Full-Text-Page (TeX-fonts) with some Logos. (125K)

drawing.ps:  Sparse technical drawing, about 800K-Filesize

kathy.ps:    2MB 640x480x24 Color-Image. (entire page covered)

january.ps:  Processing of a 100K JPEG-File

DpI    BPP Selected Modi       File                               i486DX2-CPU-Times

1.12/1.13         1.16             1.17             1.20

180x180  8  fsmono,noWeave,Uni  tropics.ps  110.79u   2.94s  117.16u  3.11s  119.96u  3.01s  121.12u  2.89s

180x180  8  fsmono,noWeave      tropics.ps  112.68u   3.44s  132.85u? 3.19s  120.22u  2.95s  123.76u  2.87s

180x180  1  gsmono,Microweave   tropics.ps  119.74u   2.83s  120.95u  2.64s  124.56u  3.01s  121.32u  3.02s

180x180  1  gsmono              tropics.ps  119.27u   2.71s  120.11u  2.97s  124.36u  2.95s  122.76u  2.65s

180x180  4  gsrgb,noWeave       tropics.ps  230.26u   3.01s  287.15u  2.85s  276.06u  3.27s  289.05u  2.74s

180x180  4  gscmyk [DEFAULT]    tropics.ps (231.63u   2.93s) 398.10u  3.11s  394.02u  3.39s  413.78u  2.79s

360x360  1  gsmono,noWeave,Uni  brief.ps      8.64u   0.63s   23.75u  1.01s   28.28u  1.20s  27.48u   0.79s

360x360  8  fsmono,noWeave      brief.ps     16.76u   0.83s   41.93u  1.29s   43.74u  1.16s  41.30u   1.06s

360x360  4  gsrgb,Microweave    brief.ps     22.45u   1.22s   47.18u  1.33s   46.83u  1.41s  46.67u   1.20s

360x360  4  gscmyk              brief.ps    ---.--u  --.--s   23.050  1.20s   23.22u  1.27s  23.33u   1.29s

360x360 24  fsrgb,Uni,Flag0     brief.ps     44.67u   1.04s   57.95u  1.38s   60.11u  1.23s  60.15u   1.16s

360x360 24  fs2                 brief.ps    ---.--u  --.--s   97.75u  1.20s   97.94u  1.17s  99.96u   1.10s

360x360 30  fsrgb               brief.ps    ---.--u  --.--s   85.67u  1.32s   85.42u  1.13s  87.09u   2.25s

(creates colors instead of grays due to random initialization)

360x360 32  hscmyk              brief.ps     63.07u   0.83s   74.90u  1.42s   40.16u! 1.33s  45.13u   1.54s

360x360  4  gscmyk,noWeave      drawing.ps   27.61u   1.27s   24.71u  1.34s   25.89u  1.62s  24.88u   1.74s

360x360 24  fsrgb,Microweave    drawing.ps   52.70u   1.33s   60.21u  1.53s   60.78u  1.29s  62.69u   1.52s

360x360 32  hscmyk              drawing.ps   67.75u   1.22s   71.62u  1.40s   43.70u  2.12s  51.99u   2.05s

360x360 32  hscmyk,escp_Band=29 drawing.ps  ---.--u  --.--s   71.25u  1.62s   42.61u  1.84s  48.89u   1.98s

360x360  4  gscmyk,noWeave      tropics.ps (261.34u   4.99s) 429.73u  4.58s  437.40u  4.31s 445.25u   4.46s

360x360  4  gsrgb,noWeave       tropics.ps  261.34u   4.99s  342.71u  4.54s  347.41u  5.11s 349.88u   4.41s

360x360 24  fsrgb,Microweave    tropics.ps  206.11u  17.81s  216.42u 13.55s  210.40u 13.01s 218.61u  13.14s

360x360 24  fs2                 tropics.ps  ---.--u  --.--s  313.64u 12.27s  320.42u 12.17s 325.77u  12.65s

360x360 32  fscmyk              tropics.ps  ---.--u  --.--s  ---.--u --.--s  262.29u 14.08s 268.47u  14.57s

360x360 32  hscmyk              tropics.ps  206.67u  18.31s  272.93u 16.90s  224.88u 16.55s 239.12u  16.49s

360x720  8  fsmono              tropics.ps  164.69u  15.15s  239.77u 10.31s  240.37u  8.41s 236.25u   9.04s

360x720 16  fsmono              tropics.ps  ---.--u  --.--s  400.94u 26.56s  252.86u 11.82s 247.62u   9.92s

360x720 24  fsrgb               tropics.ps  277.30u  22.40s  303.76u 16.55s  304.46u 16.42s 314.24u  16.02s

360x720 32  hscmyk              tropics.ps  307.70u  21.68s  388.35u 17.11s  297.32u 17.73s 306.50u  17.54s

720x360 32  hscmyk              tropics.ps  299.99u  21.82s  381.58u 17.35s  311.28u 17.70s 525.60u  33.640s?

720x720  8  fsmono              tropics.ps  199.06u  18.87s  334.23u 11.60s  333.80u 11.14s 325.19u  11.03s

720x720 24  fsrgb               tropics.ps  399.96u  35.99s  461.07u 23.85s  454.11u 22.71s 446.52u  23.78s

720x720 32  hscmyk              tropics.ps  473.13u  33.07s  587.53u 25.38s  440.83u 26.02s 429.84u  26.52s

720x720 32  hscmyk,Microweave   tropics.ps  506.10u  33.59s  622.07u 27.71s  437.17u 28.27s 420.25u  26.80s

720x720 32  hscmyk,Microweave   escher.ps                    394.59u  7.85s  223.89u  9.28s 230.01u   6.99s

180x90   1  gsmono,noWeave,Uni  golfer.ps     2.00u   0.45s    6.72u  1.14s    5.30u  0.82s   5.10u   0.69s

180x180  8  fsmono,Microweave   golfer.ps     6.09u   0.83s   17.21u  2.22s   13.67u  0.95s  13.33u   0.74s

180x360  4  gscmyk              escher.ps     ---FAILED!---   48.42u 14.10s   38.32u  1.26s  37.98u   1.25s

180x720 24  fsrgb               colorcir.ps  47.25u   5.24s   62.22u  1.30s   53.78u  1.66s  59.05u   0.94s

180x180  8  fsmono,noWeave      kathy.ps     35.56u   3.48s   45.00u  1.39s   43.91u  1.72s  46.32u   1.38s

360x90   8  fsmono,noWeave      golfer.ps     5.84u   1.08s   14.01u  0.72s   13.56u  0.70s  13.21u   0.70s

360x180  8  fsmono,Microweave   golfer.ps    10.13u   1.92s   25.98u  1.30s   24.84u  1.22s  24.33u   1.13s

360x360 24  fsrgb,Microweave    kathy.ps    105.98u  15.53s  116.18u  4.46s  122.57u  5.09s 115.18u   5.21s

360x360 30  fsrgb               kathy.ps    ---.--u  --.--s  165.05u  5.26s  164.99u  5.89s 166.70u   5.92s

360x720 24  fsrgb               colorcir.ps  91.27u  11.91s  118.96u  2.43s  116.27u  2.53s 105.30u   2.23s

360x720 32  hscmyk              colorcir.ps 128.90u  10.43s  140.37u  1.59s   67.43u  1.87s  81.52u   1.60s

720x90   8  fsmono,noWeave      golfer.ps    10.64u   2.59s   26.39u  1.22s   29.15u  1.67s  24.54u   1.28s

720x180 24  fsrgb               escher.ps    73.60u  10.83s   82.49u  2.31s   78.12u  2.52s  81.84u   2.32s

720x360 24  fsrgb               kathy.ps    179.66u  29.56s  205.95u  7.94s  197.64u  8.29s 201.46u   8.14s

720x360 32  hscmyk              kathy.ps    227.41u  34.00s  273.93u  8.23s  175.54u  8.50s 175.32u   8.36s

720x360 32  hscmyk              january.ps                   293.76u  9.67s  163.49u  9.51s 172.33u  10.12s

720x720 32  hscmyk              january.ps                                   295.64u 13.33s 316.55u  12.98s

### ------------------------------ End --------------------------------- ###

### --------------- The BJC-600/BJC-800/BJC-4000 printers -------------- ###

This section was written by Yves Arrouye <Yves.Arrouye@imag.fr>.

HISTORY

-------

The BJC-600 driver was written in the first place by Yoshio Kuniyoshi

<yoshio@nak.math.keio.ac.jp> and later modified by me, Yves Arrouye

<Yves.Arrouye@imag.fr>. We both tried to make it evolve synchronously,

though Yoshio cannot be reached since a long time.

The drivers are based on code for the HP printers by George Cameron

<g.cameron@biomed.abdn.ac.ukis> (in fact, they are in the same file!),

so he's the first person to thank!

The 2.00 version of the drivers was a complete rewrite of the driver

(arguments, optimization, colour handling, in short: everything!) by

Yves Arrouye. The 2.x release is also the first one to be able to

use the full width of an A3 paper size...

VERSION INFORMATION

-------------------

The BJC-600 driver is version 2.13.11 dated 09/23/95.

The BJC-800 driver is version 2.13.03 dated 09/23/95.

COMPILATION NOTES

-----------------

Configuration

-------------

* Default values for options and other stuff

Configuration for the drivers can be made by modifying defauts values

in the file gdevbjc.h or on the compilation line. If you don't do

that, the drivers use reasonable defaults that make them work "as

expected".

All default values given below are defined in this file if you need

to change them to customize your installation (a bad idea, better use

options...).

* CMYK to RGB color conversion

By default, the drivers use the same algorithm as Ghostscript to

convert CMYK colors to RGB. If you prefer to use Adobe formulas,

define USE_ADOBE_CMYK_RGB when compiling. (See the top of the

gdevcdj.c file to see the difference between the two.)

* Vertical centering of the printable area

The drivers center the imageable area horizontally, but no vertically

so that what can be printed does use the most of the output media. If

you define BJC_DEFAULT_CENTEREDAREA when compiling, then the top and

bottom margins will be the same, resulting in a vertically centered

imageable area too.

* Margins

If you define USE_RECOMMENDED_MARGINS then the top and bottom margins

will be se same (i.e. BJC_DEFAULT_CENTEREDAREA will be defined for

you) and these margins will be those recommended by Canon, 12.4 mm.

Because margins are a complicated thing (due to the fact that one

does rely on the mechanical precision of the printer), the drivers do

something about the bottom margin: by default the bottom margin is

9.54 mm for the bjc600 driver and 7 mm for the bjc800 one. If you

define USE_TIGHT_MARGINS then the bottom margin is 7 mm for both

drivers (but I never managed to get my own bjc600 print a line on this

low bound, hence the greater default). Regardless of the presence of

this define, USE_FIXED_MARGINS will not allow the bjc800 to use the

lower 7 mm bottom margin, so if you have a problem with the bottom

margin on a bjc800, just define that (without defining USE_TIGHT_MARGINS,

of course).

Compilation

-----------

Make sure the bjc600 and/or bjc800 devices are in your DEVICE_DEVS

variable. That is look in the makefile for your platform and add them

if necessary. This means for example adding them to the DEVICE_DEVS6

variable. The line should read something like that:

DEVICE_DEVS6=bj10e.dev bj200.dev bjc600.dev bjc800.dev

Now if you get an error from make saying that it does not know how to

make bjc800.dev it's because you have an old makefile with only the

bjc600 device in it. You then have to copy the lines explaining how to

make bjc800.dev in your makefile. These lines are in devs.mak (under

the lines for making bjc600.dev) and should go just after the lines

for making bjc600.dev.

Testing the margins

-------------------

A quick way to be sure that the margins you selected (see above) are

okay is to print a file whose contents are:

%!

clippath stroke showpage

If the margins are okay, you will obtain a rectangle surrounding the

printable area.

USE OF THE DRIVERS

------------------

There are two drivers here: the "bjc600" one supports the BJC-600 and

BJC-4000 (maybe the BJC-70 as well) and the "bjc800" one supports the

BJC-800 series. When remarks here apply to both drivers, the name

"bjc" will be used.

Supported Options and Defaults

------------------------------

(Note: the names "options", "properties" and "parameters" will be used

to designate the same thing: device parameters that you can change

from gs.)

Preamble: if an option is given an incorrect value, an error will

occur. Unless stated otherwise, this error will be a rangecheckerror.

Options may be set from the gs command-line (using the -d and -s

switches or other predetermined switches if they have an effect on the

driver) or using the setpagedevice Level 2 operator if Ghostscript has

been compiled with the level2 device (it should ;-)). There are *no*

special-purpose operators as one was able to find in Level 1 printers.

The default number of bits per pixel for the bjc is 24 (unless you

change the value of BJC_BITSPERPIXEL) and corresponds to a CMYK

printing. Supported modes are 1 bpp and 4 bpp (gray levels), 8 bpp, 16

bpp, 24 bpp and 32 bpp (colours). Colours are preferrably stored in

the CMYK model (which means that with 16 bpp there are only 16

different shades of each color, for example) but it is possible to

store them as RGB color for some depths.

Some modes do Floyd-Steinberg dithering while some others don't and

use the default Ghostscript halftoning (in fact, when halftoning is used

dithering takes also place but due to the low resolutions it is

usually not eficient and thus invisible).

Here is a short description of each printing mode (expressed in

bpp/colors):

32/4  CMYK Colour printing, Floyd-Steinberg dithering.

24/4	Id. (But each primary colour is stored on 6 bits instead of 8.)

24/3  RGB colour printing, Floyd-Steinberg dithering. This mode will

*not* use the black cartridge (that's why it exists, for when you

don't want to use it ;-)). Each primary colour is stored on 8

bits (we have only three colours here, okay?). (Of course, with

this mode you'll get brownish/pinkish output; it will remind you

the first versions of the driver....). This mode is not supported

anymore.

16/4  CMYK colour printing, halftoned by Ghostscript. FS dithering

is still visible here (but the halftone patterns are visible

too!).

8/4	Id. (But each primary colour is stored on 2 bits instead of 4.)

8/1	Gray-levels printing, Floyd-Steinberg dithering.

3/3   RGB colour printing. This mode is not intended to be

used. What I mean is that it should be used only if you

want to use custom halftone screens *and* the halftoning is

broken using the 8/4 mode (some versions of gs have a

problem).

1/1	Gray-levels printing, halftoned by GhostScript.

These modes are selected using the BitsPerPixel *and* Colors integers

options (either from the command line or in a PostScript program using

setpagedevice). See below.

A note about darkness of what is printed: Canon printers do print dark,

really. And the Floyd-Steinberg dithering may eventually darken your image

too. So you may need to apply gamma correction by calling gs as in

% gs -sDEVICE=bjc600 gamma.ps myfile.ps

where gamma.ps changes the gamma correction (here to 3 for all colors):

{ 0.333 exp } dup dup currenttransfer setcolortransfer

The drivers support printing at 90 dpi, 180 dpi and 360 dpi. Horizontal and

vertical resolutions must be the same or a limitcheck error will happen. A

rangecheck will happen too if the resolution is not 90 * 2**n. (If the

driver is compiled with -DBJC_STRICT a rangecheck will also happen if

the resolution is not one of those supported. This is not the case as we

expect that there may be a 720 dpi bjc someday).

Here are the various options supported by the bjc drivers, along with

their type, supported values, effect(s) and usage:

BitsPerPixel (int)	Choose the depth of the page. Valid values are

1, 8, 16, 24 and 32. Default is 24.

Note that when this is set for the first

time, the Colors property is automatically

adjusted unless it is also specified. Defaults

adjustments are show in the table below,

default choices are indicated by a star (*).

This table gives also the corresponding

color models and the rendering method that is

visible (GS means Ghostscript halftoning, FS

Floyd-Steinberg dithering, and if both are

present it means that the dithering of

halftones is visible).

+-----+--------+---+----------+-------+

| Bpp | Colors | * | C. Model | Dith. |

+-----+--------+---+----------+-------+

|  32 |      4 |   | CMYK     |    FS |

+-----+--------+---+----------+-------+

|  24 |      4 | * | CMYK     |    FS |

|     |      3 |   | RGB      |    FS |

+-----+--------+---+----------+-------+

|  16 |      4 |   | CMYK     | GS FS |

+-----+--------+---+----------+-------+

|   8 |      4 | * | CMYK     | GS    |

|     |      1 |   | K (CMYK) |    FS |

+-----+--------+---+----------+-------+

|   3 |      3 | * | RGB      | GS    |

+-----+--------+---+----------+-------+

|   1 |      4 |   | K (CMYK) | GS    |

|     |      3 |   | K (RGB)  | GS    |

|     |      1 | * | K (CMYK) | GS    |

+-----+--------+---+----------+-------+

Valid Colors values for allowed

BitsPerPixel values.

Also note that automagical change of one

parameter depending on the other one does not

work in a setpagedevice call. This means that

if you want to change BitsPerPixel to a value

whose valid Colors values do not include the

actual Colors value, you must change Colors

too.

HWResolution (floats array)

An array of 2 floats giving the horizontal and

vertical resolution in dots per inch. Supported

values are 90, 180 and 360 and both values

must be the same. Default is 360.

(On the gs command line, the resolution is

changed by saying "-rXDPIxYDPI".)

ManualFeed (bool)	Indicate that the sheets won't be fed automatically

by the printer. Default is false.

(Not meaningful on the BJC-600, I fear.)

MediaType (string)	Choose the media to print on. Values are chose

amongst "PlainPaper", "CoatedPaper",

"TransparencyFilm", "Envelope", "Card" and "Other".

Default is "PlainPaper".

If the chose media is "Envelope", "Card" or

"Other", the driver will make the printer go

in thick mode automatically regardless of the

media weight.

MediaWeight (int or null)

Choose the weight of the media (in g/m2). Using

null indicates that the weight is of no

importance. Default is null.

If the specified media weight is greater

than 105 (i.e. the value of the compilation

default BJC???_MEDIAWEIGHT_THICKLIMIT) then

the printer will be setup to use thick paper.

PrintQuality (string) Choose the quality of printing. For the bjc600

driver it can be one of "Normal", "High" and

"Draft". For the bjc800 driver it can be one

of "Low", "Normal", "High" and "Draft".

Default is "Normal" for both drivers.

For both drivers, "High" means 200% black and

100% cyan, magenta and yellow (on a bjc600 you

will get the "Bk+" light).

For the bjc600 driver, "Normal" lits the

"HQ" light while "Draft" unlits it.

For the bjc800 driver, "Low" has the effect

of making only two printing passes instead of

four (should be twice as fast ;-)). This is

what is known as "CN" (Color Normal) mode.

PrintColors (int)	Mask for printing color. If 0, use black for

any color.

Otherwise, the value must be the sum of any

of 1 (cyan), 2 (magenta), 4 (yellow) and 8

(black), indicating which colors will be used

for printing.

When printing colour, only those colour

specified will be printed (this means that

some planes will be missing).

When printing grays, black is used if it is

present in the PrintColors; otherwise, the

data is printed by superimposing each

requested color.

MonochromePrint (bool)

*For bjc600 only*. Substitute black for Cyan,

Magenta and Yellow when printing (useful for

getting some monochrome output of a dithered

printing for example). Default is false.

This is a hardware mechanism as opposed to

the previous software one. I think that using

this or setting PrintColors to 0 will give the

same results.

Note that the MediaType and ThickMedia options will be replaced by the use

of the device InputAttributes and OutputAttributes as soon as possible.

Please note too that the print mode may be reset at the start of a print,

not at the end. This is the expected behaviour. If you need to reset

the printer to its default state, simply print a file that does just a

showpage.

Device Informations

-------------------

Here are other informations published by the driver that you will find

in the deviceinfo dictionary:

OutputFaceUp (bool)   This has the boolean value true, indicating that

the sheets are stacked face up.

Version (float)	In the form M.mmpp where M is the major version,

mm the bjc drivers minor version and pp the specific

driver minor version (that is, M.mm will always be

the same for the bjc600 and bjc800 drivers).

VersionString (string)

A string that gives the version info plus

other indications. At the moment, things like

'a' or 'b' may follow the version to indicate

alpha or beta versions and the date of the

last change to this version is given in the

form MM/DD/YY (no,  it won't adapt to your

locale!).

Hardware Margins

----------------

The BJC printers have top and bottom hardware margins of 3 mm and 7.1

mm respectively (Canon says 7 mm but this is not usable because of

the rounding of paper sizes to PostScript points).. The left margin is

3.4 mm for A4 and smaller paper sizes, 6.4 mm for US paper sizes,

envelopes and cards. It is 4.0 mm for A3 paper on the BJC-800.

The maximum printing width of a BJC-600 printer is 203 mm, in any event.

The maximum printing width of a BJC-800 printer is 289 mm on A3 paper,

and 203 mm on letter and A4 paper.

Reporting Problems

------------------

When you report a problem please be as descriptive as possible, and

please send information that can be used to reproduce the problem.

Please don't forget to tell me which driver you use and its

version. Version information can be found in this file or preferrably

by issuing the following command in a shell:

% echo "currentpagedevice /VersionString get ==" \

gs -q -sDEVICE=bjc600 -

(the % doesn't cound as part of the command and the device name should

be the device you really use).

Contact Address

---------------

If you have problems with this driver (or if you are extremely

satisfied with it) you may email me at Yves.Arrouye@imag.fr.

I consider reusing the "SpotSize" option of the Stylus driver (see

documentation above). I won't do it unless someone ask (with a good

reason), so if you need it, email me...

Acknowledgements

----------------

I am particularly grateful to Yoshio Kuniyoshi <yoshio@nak.math.keio.ac.jp>

without whom I'd never make these drivers and also to Peter L. Deutsch

<ghost@aladdin.com> who answered *all* my (often silly) questions about

the drivers interface used by Ghostscript.

Thanks also to the people who volunteered to beta-test the v2.x

BJC drivers; David Gaudine <david@donald.concordia.ca>, Robert M. Kenney

<rmk@unh.edu>, James McPherson <jmcphers@attila.stevens-tech.edu> and

Ian Thurlbeck <ian@stams.strath.ac.uk> (in an alphabetic listing) were

particularly helpful by discovering bugs and helping find out exact

paper margins on printers I don't have access to.

### ------------------------------ End --------------------------------- ###