Description of the functions available in CDDRIVE.C. Most of the results are

determined with my own CD-ROM drive, so it is possible that some functions

will not work as mentioned with yours, or that some function will not work at

all due to lack of debugging.

int query_error(void);

This function returns the errorcondition of the last CDDRIVE-function that

was executed. Possible values that can be returned are: NO_ERROR (command

was succesfully completed), WRITE_PROTECT_VIOLATION, UNKNOWN_UNIT,

DRIVE_NOT_READY, UNKNOWN_COMMAND, CRC_ERROR,

BAD_DRIVE_REQUEST_STRUCTURE_LENGTH, SEEK_ERROR, UNKNOWN_MEDIA,

SECTOR_NOT_FOUND, PRINTER_OUT_OF_PAPER (will be rare, probably. *smile*),

WRITE_FAULT, READ_FAULT, GENERAL_FAILURE (most common error),

INVALID_DISK_CHANGE.

int query_busy(void);

This function returns the busy-flag after the last CDDRIVE-function. 0 is not

busy, 1 is busy.

int query_done(void);

This function returns the done-flag after the last CDDRIVE-function. When

using audio, the done flag will go 0 at the moment the disk is paused or

audioplay has ended.

void play_audio(int drive,long int start,long int stop);

This function plays a part of de CD-ROM under drive-letter 'drive'. The

starttime 'start' is absolute and should be given in REDBOOK encoding (which

means a long int, with the MSB 0, the second byte the number of minutes, the

third byte the number of seconds and the LSB the number of frames (75 frames

in one second). The stop address is the number of frames to be played (can be

calculated by subtracting the HSG version of the end address from the HSG

version of the start address).

void stop_audio(int drive);

This function stops the audio playing on drive 'drive'. The current and

ending locations are stored, to make it possible to resume play.

void resume_audio(int drive);

This function resumes audio stopped with the stop_audio() command on drive

'drive'.

unsigned char *audio_channel_info(int drive);

This function gives information about the audio-channels of the CD-ROM drive

in the form of a one dimensional array of unsigned char. This array should

be copied, because other function in CDDRIVE might overwrite the information

in it. The possible index-values are: INPUT_CHANNEL_0, INPUT_CHANNEL_1,

INPUT_CHANNEL_2, INPUT_CHANNEL_3: to which real channel (0 is left and 1 is

right) a certain CD-ROM channel is connected (usually 0-0, 1-1, 2-2, 3-3). It

is possible to make the sound mono using input channel allocation. Other

possible index-values are: VOLUME_CHANNEL_0, VOLUME_CHANNEL_1,

VOLUME_CHANNEL_2, VOLUME_CHANNEL_3: the volume of the four channels. 0 will

always be silent, 255 is maximum and values in between might be available on

some CD-ROM drives.

void audio_channel_control(int drive,int what,int setting);

This function makes it possible to change a setting of the audio channels.

'drive' is again the CD-ROM drive to which the command should be sent, 'what'

is which value should be changed (in audio_channel_info() possible values are

mentioned) and 'setting' is the desired setting.

int device_status(int drive);

This function returns the status of the CD-ROM drive 'drive'. The value

returned can be should be ANDed with one of the following values. If the

result is unequal to 0 then the CD-ROM is in the mentioned state. DOOR_OPEN:

the door of the CD-ROM drive is open. DOOR_UNLOCKED: the door of the CD-ROM

drive is unlocked. SUPPORTS_RAW: The CD-ROM drive supports rawwrite.

SUPPORTS_WRITE: It is possible to write CDs. SUPPORTS_AUDIO_PLAY: It is

possible to play audio. SUPPORTS_INTERLEAVING: ?. SUPPORTS_PREFETCHING: Makes

the CD-ROM faster because sectors can be prefetched. SUPPORTS_AUDIO_CHANNELS:

It is possible to use the audio_channel_control() and audio_channel_info()

calls. SUPPORTS_RED_BOOK: Redbook encoding can be used (see play_audio()).

int media_changed(int drive);

This function indicates when the CD in the drive 'drive' changes. Possible

values are: MEDIA_UNKNOWN (it is unknown whether another CD was entered),

MEDIA_UNCHANGED and MEDIA_CHANGED.

int audio_disk_info(int drive,int track);

This function returns the lowest and highest track on drive 'drive',

depending on the value in 'track': LOWEST_TRACK or HIGHEST_TRACK.

int audio_track_info(int drive,int track);

This function returns a value of the status of the track 'track' on the CD

in drive 'drive'. The MSN (nibble) of the LSB of the value returned is

important. Here are possible bit-patterns and their meaning ("." meaning a

donot care): 00.0 - 2 audio channels without pre-emphasis, 00.1 - 2 audio

channels with pre-emphasis, 10.0 - 4 audio channels without pre-emphasis,

10.1 - 4 audio channels with pre-emphasis, 01.0 - data track, ..0. - digital

copy prohibited, ..1. - digtal copy permitted. This function will be modified

in later versions of CDDRIVE.C.

unsigned char *audio_qchannel_info(int drive);

This function returns a one dimensional array of unsigned char with the

following index-values: CONTROL_ADR, TRACK_NUMBER (the current track),

POINT_INDEX, RUN_MINUTES (minute count of the current track), RUN_SECONDS,

RUN_FRAME, DISK_MINUTES (minute count of the current disk), DISK_SECONDS,

DISK_FRAME.

unsigned char *get_upccode(int drive);

This returns the BAR-code of the CD in the form of an array of unsigned char

with a length of 13 bytes.

int audio_status_info(int drive);

This returns whether the disk is running (DISK_RUNNING) or paused

(DISK_PAUSED) in drive 'drive'.

long int get_track_address(int drive, int track);

This function returns the redbook-address of the start of track 'track' on

the CD in drive 'drive'. 'track' may be between the lowest track and the

highest track (see audio_disk_info()) or LEAD_OUT_TRACK (the length of the

CD), STARTING_LOCATION (the starting location when resume_audio() is called),

ENDING_LOCATION (the ending location when resume_audio() is called).

void eject_disk(int drive);

Eject the disk on drive 'drive'.

void close_tray(int drive);

Closes the tray of drive 'drive'.

void lock_unlock_door(int drive,int door);

Locks/unlocks the door of drive 'drive', depending on the value 'door'.

void reset_drive(int drive);

Resets the drive 'drive'. This function may take some time.

long int redbook_to_hsg(long int redbook);

This function converts a redbook address to a HSG one. HSG can be used to

subtract to addresses to calculate the exact length in frames. 'redbook' is

the redbook address, and returned is the HSG address.