MOZART.SYS, Mozart Device Driver -  Copyright (c) 1995 Robert Muchsel

Initializes Mozart Sound cards for OS/2

Shareware: DM 20.-/US$ 12.50 (see end of this file).

This OS/2 device driver initializes your Mozart sound card so that

the OS/2 Soundblaster driver can be loaded.

Installation

============

The Mozart driver must be loaded before (!) the MMOS/2 driver

and configures the Mozart card to the default parameters of

/A220 /I5 /D1; the driver does not recognize command line

parameters (see "Patches").

If the driver finds a Mozart card and initialization is successful,

you'll hear a cuckoo (else an error message is displayed).

Some noise and hissing before the cuckoo is normal.

Configuring OS/2 - CONFIG.SYS:

------------------------------

DEVICE=xxxxxx\MOZART.SYS

DEVICE=x:\MMOS2\SBD2.SYS 1 1 5 220 4 /N:SBAUD1$

Configuring OS/2 - AUTOEXEC.BAT:

--------------------------------

SET BLASTER=A220 I5 D1

Note:

* You must __NOT__ load the DOS device driver MZTINIT.SYS under OS/2.

* If the MMPM/2 driver won't load ("Soundblaster not found"), try

another version of the MMPM/2 driver (e.g. from TBOS2.ZIP).

Patches

=======

I'm sorry the driver doesn't recognize command line parameters

(maybe a future version will?).

To change parameters, there are patch scripts supplied (to use them,

you'll need the OS/2 PATCH.EXE program, which can be installed under

"System Setup-Selective Install-Serviceability and Diagnostics Aids").

Name       | Parameter

-----------+--------------------------

IO240.CMD  | IO Address 240  (/A240)

DMA3.CMD   | DMA Channel 3   (/D3)

Licence, Registering

====================

Developing this driver has taken a considerable amount of time. I'm

sure you'll honor my work, so there are no restrictions in the

shareware version!

You may try this program for 30 days. If you  want to continue to use

it, you must register. If you don't like the driver, simply DEL it.

!! Warning: If you are using shareware without registering, you'll

!! certainly descend into Hell some day.

Please use the registration form supplied.

Optionally, you can order a disk with the driver.

Price (subject to change w/o notice):

Country     Registration   (optional) Diskette

----------------------------------------------

Europe      DM 20.-        DM 5.-

or US$ 12.50      US$ 3.20

World       DM 20.-        DM 11.-

or US$ 12.50      US$ 6.50

DM=Deutsche Mark        more than 5: please call!

The Author

----------

Robert Muchsel, Hegaublick 2, D-78465 Konstanz, Germany

Fax:    07533-3151 (intl. +49-7533-3151)

e-Mail: rmuchsel@iiic.ethz.ch (until end of 1996)

Fidonet 2:246/8100.11

I'm studying computer science at the Swiss Federal Institute of

Technology, Zurich.

You might know some of my other shareware tools:

AMIsetup - External Setup for AMI-BIOS

HPFSDOS  - HPFS Driver for DOS (Read only)

CD800X   - Audio Support for Apple CD-150/SC-CD under OS/2

EFDD     - Use up to 20 diskette drives under DOS

Important

---------

The author disclaims all warranties as to this software, whether

express or implied, including without limitation any implied

warranties of merchantability, fitness for a particular purpose,

functionality, data integrity or protection, in so far as permitted by

applicable legislation.

All trademarks are property of their respective owners and appear for

identification purposes only.

You are encouraged to copy and pass on this program. However,

- the program must be distributed in its original form

(complete, unmodified)

- the distribution must not exceed the equivalent of DM 15.-

(US$ 8.50).

26. December 1994