BEYOND DES:

DATA COMPRESSION AND THE MPJ ENCRYPTION ALGORITHM

by

Michael Paul Johnson

B. S., University of Colorado at Boulder, 1980

A thesis submitted to the

Faculty of the Graduate School of the

University of Colorado in partial fulfillment

of the requirements for the degree of

Master of Science

Department of Electrical Engineering

1989

Copyright (C) 1989 Michael Paul Johnson.

All Rights Reserved.

This thesis for the Master of Science degree by

Michael Paul Johnson

has been approved for the

Department of

Electrical Engineering

by

Mark A. Wickert

Charles E. Fosha, Jr.

Rodger E. Ziemer

Date

Johnson, Michael Paul (M. S., Electrical Engineering)

Beyond DES:  Data Compression and the MPJ Encryption Algorithm

Thesis directed by Assistant Professor Mark A. Wickert

Many encryption algorithms have come and gone as cryptography, cryptanalysis,

and technology have progressed.  Today's communication and computer

technologies need cryptography to truly secure data in many applications.

The demands on the cryptography needed for some commercial applications

will exceed the security offered by the National Bureau of Standards

Data Encryption Standard (DES) in the near future due to advances

in technology, advances in cryptanalysis, and the increasing rewards

for breaking such a heavily used algorithm.  To meet part of this

need, a new block encryption algorithm is proposed.  A Pascal program

to implement this algorithm is given.

One way to further increase security of encrypted data, as well as

to achieve storage and/or transmission economy, is by redundancy reduction

prior to encryption.  A linguistic approach to redundancy reduction,

together with an example computer program to implement it, is given

for this purpose.

LIST OF FIGURES	viii

I.	INTRODUCTION	1

A.  Motivation	2

B.  Approach	4

II.	HISTORY OF CRYPTOGRAPHY	6

III.	ELEMENTS OF ENCRYPTION	8

A.  Substitution	8

1.  Monoalphabetic	8

2.  Polyalphabetic	10

B.  Permutation	11

C.  Noise Addition	12

D.  Feedback & Chaining	13

1.  Plain Text Feedback	13

2.  Cipher Text Feedback	14

E.  Analog Encryption	15

IV.	FACTORS RELATED TO ENCRYPTION	17

A.  Change of Language	17

B.  Digitization	18

C.  Compression	18

D.  Multiplexing	19

V.	COMPARISON OF SELECTED ALGORITHMS	20

A.  One-Time Key Tape	20

B.  Linear Shift Register Feedback	21

C.  Exponential Encryption	22

D.  Knapsack	24

E.  Rotor Machines	24

F.  Codes	27

G.  Galois Field and Hill Cryptosystems	28

H.  DES	30

VI.	DESIGN CONSIDERATIONS FOR MPJ	33

A.  Strength Based on Key	33

B.  Usability of Random Keys	33

C.  Key Length & Block Size	34

D.  Effort Required to Break	34

E.  Computational Efficiency	35

F.  Communication Channel Efficiency	36

G.  No Back Doors or Spare Keys	36

VII.	MPJ ENCRYPTION ALGORITHM	37

A.  Description	37

1.  Overall Structure of MPJ	37

2.  Substitution Boxes	39

3.  Wire Crossings	39

4.  Key Generation	40

B.  Implementation in Pascal	44

1.  Exceptions from Standard Pascal	44

2.  Main Program	45

3.  Procedures Encrypt & Decrypt	46

4.  Procedures Permute & Ipermute	47

5.  Procedures Substitute & Isubst	47

C.  Implementation in Hardware	47

D.  Strengths & Weaknesses	48

VIII.	DATA COMPRESSION	50

A.  Purpose	50

B.  Linguistic Parsing	53

C.  Huffman Coding	55

D.  Pascal Programs	55

IX.	CONCLUSION	58

REFERENCES	59

APPENDIX

A.	MPJ in Pascal	64

B.	Linguistic Data Compression Programs	72

FIGURE

III.B	Permutation.	11

III.C.	Noise Addition	12

III.D.	Block Cipher Modes.	14

III.E.	Analog Time & Frequency Encryption	15

V.A.	One-time key tape (AKA One-time pad)	21

V.A.1	Typex Rotor Machine	25

V.E.2.	Wired Rotors	25

V.H.1.	DES Enciphering	30

V.H.2.	DES Nonlinear Function	30

V.H.3.	DES Internal Key Generation	31

VII.A.1.	Overall Structure of MPJ	38

VII.A.3.	Wire Crossings	39

I.   INTRODUCTION

The increasing proliferation of digital communication and computer

data base storage has brought with it increasing difficulty of maintaining

the privacy of that data.  There is only one effective way to protect

the privacy of communications sent over such channels as satellites,

terrestrial microwave, and cellular telephones.  This is by encryption.  It

is clearly impossible to deny unauthorized access by a determined

and knowledgeable interceptor to the communications, but it is possible

to render the communications totally unintelligible to all but the

intended receiver(s).

There are many ways to reversibly transform data from its plain form

to something that looks unintelligible, but many of these can be figured

out (broken) by someone else.  The study of how to hide secrets is

cryptology. Trying to figure out the secrets that someone else has

hidden is cryptanalysis.  These two sciences are, of course, very

much intertwined. History reveals many examples of cryptology that

worked, and that didn't [KAH]. Successful cryptanalysis depends on

taking advantage of as many of the following as are available to the

cryptanalyst: (1) taking advantage of the redundancy in any natural

language to determine the validity of assumptions, (2) clues gained

from corresponding plain and cipher text, (3) information that might

be known about the algorithm(s) used, (4) the general expected content

of the cryptograms, (5) all of the cipher text that is available in

the same system & key, (6) compromised keys, (7) as much computational

and analytical power as can be obtained, and (8) mistakes made in

the users of the cryptographic system.  The cryptographer can make

life as difficult as possible for the cryptanalyst by depriving him

of some of these things by (1) using redundancy reduction before encryption,

(2) using an algorithm that is resistant to the known plain text attack,

(3) & (4) using a strong enough algorithm that these clues aren't

really useful, (5) changing keys often and selecting them properly,

(6) guarding keys as closely as the data they protect justifies, (7)

ensuring that there aren't enough computers in the world to do a brute

force attack on the algorithm, and (8) making sure users of the system

understand how to properly use it.

This thesis proposes one solution to the above challenge by proposing

(1) an approach to linguistically based redundancy reduction, and

(2) proposing a new data encryption algorithm (MPJ) that can be used

where DES is in use now, but is more secure.

A.  Motivation

There has been a great deal of discussion of the security of DES in

the open literature.  Most of it has been favorable to DES [MEY],

but there are a few indications that it would be wise to supplement

the aging DES and eventually replace it.

DES has been in use since 1977, and has been used in a large number

of applications where people have had many possible motivations for

trying to break this cipher.  During this time, it is possible that

someone has discovered a computationally feasible method for doing

so [KRU].  Under such circumstances, it is highly unlikely that such

a discovery would be made known.  There is no way to really know this

for sure, unless you are one of the ones who has broken DES.  The

closest thing that I have found to an open admission of breaking DES

is a story of the FBI successfully decrypting a file of drug transaction

records that were encrypted on a PC using a DES board. [MCL] The DES

board that the criminal used has an algorithm to generate a key from

a word or phrase.  By an exhaustive search of an English dictionary

and key names from the criminal's family & friends using a supercomputer,

the file was solved.  This indicates some weakness in DES, but even

more weakness in the way the key was chosen. The key was not chosen

randomly, so the FBI's job was much easier.

DES is subject to attacks that require precomputation that could tie

up a supercomputer for a few years, after which it would take only

a few days to solve a DES cryptogram.  This is becoming less of a

barrier as the price of computers drops and the speed and storage

capacity of computers increase.

The U. S. National Security Agency (NSA) is acting to release some  of  its

own algorithms for ``sensitive but unclassified'' applications, such

as communications between government contractors.  The NSA also releases

some classified algorithms to chip manufacturers for use in classified

devices, but under strict controls [ULT].  This will take some of

the load off of the DES algorithm, but there is a catch.  They intend

to keep the algorithms secret and control the keys.  Since the security

of the algorithm is dependent on the key in a way that only  the NSA  knows,

this gives the NSA the exclusive ability to read everybody's' communications.  That

is not all bad, as it discourages the use of one of those systems

by someone engaged in spying or other illegal activity.  Unfortunately,

these systems are not an option for protection of a corporations proprietary

data that may have a great deal to do  with profits and  losses, but

are not really in the domain of the NSA.

There are other alternatives to DES now, but none of them in the public

domain are even as good, let alone better, for general cryptography.  For

example, RSA encryption has great advantages in the authentication

of digital signatures, but the complications of selecting good keys

and the fact that the security of RSA relies heavily on the failure

of the state of the art in mathematics to progress makes it at least

inconvenient to use and at most insecure.

Because of the above considerations, it is my intentions to suggest

a better algorithm (MPJ) for general use in the private sector.  Although

the MPJ algorithm might be useful for government applications, too,

the design requirements for the two areas of application vary [CHA]

and the latter is kept shrouded in secrecy, so we shall leave it to

the NSA.

B.  Approach

First, the problem is defined.  As explained in the above section,

the basic problem is to create an algorithm to supplement DES.  Design

criteria are then conceived such that an algorithm that meets those

criteria will solve the problem as defined.  The design criteria chosen

for the MPJ algorithm are discussed in chapter VI.  To avoid repetition

of one or more of the many mistakes that have been made throughout

history with cryptography, it is, of course, necessary to research

what was done in the past, both distant and recent.  From this, many

good ideas can be gleaned that apply to the problem at hand.  These

ideas, along with a knowledge of current technology are then applied

to design criteria with a bit of creativity and a lot of hard work

to define the new algorithm.

In the process of doing all of the above, it became apparent that

the security of encrypted data was vastly improved by first removing

as much redundancy from the data as possible.  Therefore, as sort

of a bonus, a linguistic approach to data compression is also presented

that can either be used in conjunction with the MPJ encryption algorithm,

another encryption method, or just by itself for the savings that

it gains in communications channel capacity and/or data storage space.

II.       HISTORY OF CRYPTOGRAPHY

One of the best works on the history of cryptography is <MI>The Codebreakers,

by David Kahn [KAH].  In that book, David Kahn discusses cryptography

from the prehistory to World War II.  The first codes and ciphers

were in written form, and used to protect the privacy of communications

sent by courier or mail through hostile or unknown territory.  Some

of these were reasonably good, but most were not difficult to break

using manual methods, provided that the interceptor had sufficient

cipher text and perhaps some probable text.  The use of radio, especially

by the military, increased the need for cryptography, as well as increasing

the rewards for those who could break the encryption schemes in use.  Kahn's

documentation of the efforts of those who broke some very complex

encryption schemes, like the German Enigma and the Japanese Purple

ciphers, lend great insight to the kind of process cryptanalysis really

is.

Kahn points out the kinds of mistakes the inventors and users of cryptographic

algorithms tend to make that reduce the security of their communications.  For

example, German users of Enigma tended to choose a three-letter indicator

for their messages that consisted of three consecutive letters on

the keyboard.  This substantially reduced the number of keys that

had to be searched to determine the one that they were using.  While

the designer of an algorithm may calculate the great number of combinations

of keys that there are, the cryptanalyst looks at ways to isolate

parts of the key so that the difficulty of a solution is much less

than the size of the key space indicates.  The difference in mind

set between the concealer of secrets and the one who prys into them

has caused many an inventor of an encryption algorithm to be overconfident.

The job of the cryptanalyst is a tedious one.  He tries all kinds

of things to try to unscramble the cipher text in front of him.  Sometimes

the search is fruitless.  Sometimes the search yields something that

looks like a meaningful language.  It is this ability to recognize

a meaningful message when it comes out of the various operations that

the cryptanalyst tries that makes the whole process possible.  It

is also helpful for the cryptanalyst to know some probable plain text

that is contained in a message.  This is almost always the case.  For

example, military messages even now have a very stereotyped format,

with the from, to, and date indicators in the same places in the message.

The cryptanalyst almost always knows what language to expect a message

to be written in, and this is a great help.  Natural languages contain

a great deal of redundancy.  A message that is only 90% recovered

is usually readable.  Natural languages also have very consistent

statistical properties that are very useful in cryptanalysis, especially

when the cryptanalysis is automated.  The only time that these things

don't help the cryptanalyst is in the ``one-time pad.''

III.      ELEMENTS OF ENCRYPTION

Several basic elements make up the basis of a multitude of possible

encryption algorithms, either alone or in combination [BEK][BAL].  Although

most of these elements may be used to form the basis of an encryption

method all by itself, the most secure methods of encryption will several

of them.  For example, substitution and permutation are basic elements

of both DES and MPJ encryption algorithms.  These algorithms, in turn,

are best used with some form of feedback.

A.  Substitution

A substitution cipher simply substitutes one symbol from the plain

text alphabet for another one from a cipher text alphabet.  The plain

text alphabet can be a natural language alphabet, ASCII, EBCDIC, Baudot,

the set {0,1}, or any other finite set.  Cipher text alphabets, likewise,

may be any finite set.  To be able to easily apply computer methods

to the manipulation of these alphabets, it is preferable to use alphabets

that are groups of binary digits (bits).  This is not a severe limitation,

since a correspondence can be set up between any finite set and a

set of binary numbers.

1.  Monoalphabetic

A monoalphabetic substitution cipher is one in which each letter of

the plain text alphabet is always substituted for by the same cipher

text alphabet. The cipher text alphabet need not be in any particular

order.  A subset of the monoalphabetic substitution cipher is the

Caesar cipher.  This cipher replaces each letter with a letter that

is n letters later in the alphabet.  This cipher is so named because

it was reportedly first used by Caesar. [KAH]  With only 26 possible

keys, this is obviously not very secure.  In fact, any substitution

cipher that substitutes one letter for another is vulnerable to solution

by frequency analysis, and can be solved, given enough cipher text

with the same key.  How much cipher text is enough depends on the

nature of the plain text, but for plain English, as many characters

as there are letters in the alphabet is sufficient for the probabilities

of occurrences of letters to betray enough letter identities to make

solution probable.  While ciphers of this type no longer have any

value for serious cryptography, they do make fun puzzles for elementary

school children.  For example, the following cryptogram is presented

for your solution:

UIJT JT B DBFTBS TVCTUJUVUO DJQIFS/  XPVME ZPV USVTU VPVS TFDSFST

UP JU@

Substitution ciphers can be made more secure by operating on substitutions

of more than one letter at a time.  For example, the substitution

could be made on digraphs or trigraphs (groups of two or three letters).  A

more practical example is the Data Encryption Standard.  DES is actually

a substitution cipher with an effective alphabet size of 2^64,

or about 1.84 x 10^19, and the MPJ Encryption Algorithm is a

substitution cipher with an effective alphabet size of 2^128

or about 3.4 x 10^38.   Not only is it difficult to get that

large of a sample to analyze for statistics, but the memory required

to store the substitution table is impractical even for computer systems

using large optical storage disks.

2.  Polyalphabetic

A polyalphabetic substitution cipher is one in which each letter of

the plain text alphabet may be replaced a letter from one of many

cipher alphabets. The selection of which alphabet to take the substitution

from may be determined in many ways.  The most usual method is by

selecting the cipher alphabet by a combination of a message indicator

and the number of characters from the beginning of the message.  The

classic example of this type of cipher is the rotor-based machine.  The

substitution occurs by physical wire crossings within rotors.  After

each character is enciphered, one or more rotors are rotated by a

ratchet mechanism.  The message indicator, which is part of the key,

determines the starting position of the rotors.  This effectively

allows the use of more cipher alphabets than there are letters in

the message.  Because of this, the use of statistical analysis on

any one message to guess at the substitutions is useless.  There is,

however a good way to attack this kind of cipher when it is used heavily.

This is by analyzing the statistics of the first character in each

message, then the second, etc.  This is called analyzing the messages

``in depth.''

Although the rotor-based polyalphabetic ciphers used by Germany and

Japan in World War II contributed greatly to their losses because

their messages were read regularly by the allies, it is possible to

create a more secure polyalphabetic substitution with computer technology.

The primary weakness of the mechanical rotor machines was the infrequent

changing of many of the parts of the key such as rotor wiring and

ratchet construction.  A more general method of using a different

set of alphabets for each message would be very secure.  Unfortunately,

it would result in keys larger than the message.  Since the minimum

key length for absolute, provable cryptographic security, as determined

by C. E. Shannon [SHA], is exactly the length of the message, this

solution is not very efficient with respect to key management.

B.  Permutation

Permutation is taking the same symbols and writing them in a different

order to confuse an interceptor.  There are many ways of doing this

geometrically. For example, words may be written vertically then transcribed

horizontally, or they may be written on a strip of paper wrapped along

the length of a rod of a given diameter, then unwrapped for delivery.

In general, a permutation operation may be considered as an operation

on a block of symbols, where each symbol's position is changed to

another position within the block according to some rule.  The blocks

may overlap each other if the permutation is undone from the end of

the message to the beginning.

The rule(s) determining how the permutation is done may be fixed,

or they may depend in some way on a key.  Although historical permutation

ciphers tended to be rather simple, computer methods allow them to

be potentially rather complex, especially if the symbols that are

scrambled are individual bits.

Permutation, when used in combination with substitution, is very useful

in further increasing the complexity of a cipher, since the two algorithms

act in different ways to baffle the interceptor.  This is significant,

since it is not possible to actually increase the security of a substitution

cipher by simply repeating the same thing with a different key.  The

result in such a case would be another substitution cipher with the

same complexity.

C.  Noise Addition

Noise addition can either be a way of enhancing another encryption

scheme, or of hiding the very existence of a message.  For example,

a grid can be set up on a piece of paper where only certain positions

are significant.  These positions are then concatenated together to

reveal the secret message.  The rest of the paper is then filled with

something that uses those characters, but contains something different,

like a letter to someone's mother.  The same thing can be done electronically,

by defining a block of bits with only part of the bits significant.  The

rest of the bits are then filled with truly random noise, like the

output of a Geiger counter.  This method can be very effective, especially

if the position of the information-bearing bits vary from block to

block in a pseudorandom manner, and if there is a relatively large

proportion of truly random noise.  The obvious disadvantage to this

kind of scheme is that it makes very poor use of communications channels

and data storage facilities.

One useful variation on the noise addition method is to multiplex

several encrypted streams of data (encrypted with another method)

together with some pseudorandom interleaving.  Since the other encrypted

streams of data act like the random noise added to any one stream

of data, this is a good way to improve security when a large volume

of encrypted data must be sent through the same channel.

D.  Feedback & Chaining

A block cipher like DES or MPJ, when applied directly to an input

file with a highly repetitive structure (i. e., lots of spaces between

columns) will also display some of that structure.  Although it may

not be possible to determine the exact contents of what has been encrypted,

it may be rather easy to determine something about the nature of what

has been encrypted.  To deny the interceptor even this information,

and to further increase the complexity of the encrypted information,

feedback and chaining may be used.  Chaining refers to making the

results of the encryption of one block dependent on previous results.  This

is usually done in one of two ways.  One is by adding the plain text

from the last block to the cipher text of this block, modulo two.  The

other is by adding the cipher text from the preceding block to the

cipher text of the current block, modulo two.  These are referred

to as plain text feedback and cipher text feedback, respectively.

1.  Plain Text Feedback

Plain text feedback has the property that any errors in transmission

get propagated clear through the rest of the message.  This may be

a desirable property if it is necessary to detect any tampering with

the message, but it makes it difficult to use real (error-prone) channels.  If

it is necessary to have the property of error propagation to detect

tampering and to use real channels, then the message should be transmitted

using an error detection and correction protocol of some sort.

2.  Cipher Text Feedback

Cipher text feedback also causes some error propagation, but not clear

through to the end of a message.  An error in one block will cause

an error in that block in the bit positions where the error occurred

and in all of the next block, but the receiving station can recover

all subsequent error-free blocks.  This method is a good compromise

between security and error recovery.  It is still desirable to wrap

encrypted data in an error detection and correction protocol to avoid

having a one bit error wipe out many bits.

E.  Analog Encryption

There are several methods of analog encryption.  None of them are

as secure as digital methods can be.  Analog encryption is generally

based on spectrum inversion, spectrum scrambling, time slice scrambling,

and (for video signals) suppression of synchronization information.  These

elements are also used in combination [LOD].  These methods are commonly

used by cable TV companies (and sometimes on satellite channels) to

ensure that people only get the programming that they have paid for.  Articles

on how to build devices to defeat some of these things appear periodically

in electronic hobbyist magazines.

The state of the art in current use for protection of satellite TV

signals is the General Instruments Video Cipher II.  This device used

digital (DES) encryption of the audio information and a partially

analog encryption of the video signal.  This device is a deterrent

to those who receive satellite TV without paying a cable TV operator,

but it has been broken by a hacker who figured out that it was possible

to modify the microcode in the device so that a key purchased for

one machine would work on all of them with modified code.  This is,

of course, illegal, but it allows one person to pay for a service

and many people to use it for free [DOH].  The problem here is not

one of the encryption algorithm, but in the key management.

IV.       FACTORS RELATED TO ENCRYPTION

A.  Change of Language

Although it is not really encryption, the use of a different language

from what the interceptor is likely to know does increase the difficulty

of cryptanalysis.  For example, I know that it would be far more difficult

for me to solve even a simple Caesar substitution cipher in Chinese

than in English, Spanish, or German.  This principal was used effectively

by the United States against Japan in World War II by using the Navajo

language for spoken radio communications [KAH].  Since the Navajo

language had no written form, and was only spoken by a small group

of Native Americans, there were no Japanese who knew the language.  The

Japanese never did figure that ``code'' out.  Thus, the Navajo ``Code

Talkers'' made a great contribution to their country.

Part of the reason for the success of the Navajo Code Talkers was

that the Japanese had no idea what was going on.  Since this success

is now well known, it is difficult to determine the potential success

of a repeat of this kind of approach.  There are still some languages

that are spoken only by a very few people in the world, some of which

have no written form.  These could be used to increase the effective

``key space.''

Although a change of language was a great victory for the Navajo People

and the United States of America during World War II, it is difficult

to adapt this success to computerized communication methods.

B.  Digitization

The very act of digitizing an audio or video signal provides some

protection from the casual eavesdropper.  Although this provides no

protection against a determined snooper, it is an appropriate level

of additional security for protecting the privacy of things like mobile

telephone conversations.  Of course, once an analog signal is digitized,

there is a wide range of digital encryption methods available.  Of

course, some methods would be overkill, since the same communication

would probably be transmitted in the clear over wire and/or microwave

channels as well as by radio between the car and the stationary cellular

transceiver.

Although there has been some ill-conceived efforts by the mobile telephone

industry to legislate privacy for such applications by saying that

it is illegal to listen to someone else's phone calls around 800 MHz

(where cellular phones usually operate), this kind of thing is unenforceable.

Worse yet, such legislation provides a false sense of security in

a world where scanners and other receivers that operate in that range

are readily available.

C.  Compression

Data compression itself tends to obscure things by taking, for example,

and ASCII text file in English and reducing it to a binary file that

is not as easy to read.  There is a more important effect of data

compression, however. The removal of the natural redundancy of the

plain text data before encrypting it with some kind of encryption

algorithm makes it much more difficult to cryptanalyze the cipher

text.  In the extreme case where all of the redundancy of a message

is removed, all messages of a given length would be meaningful, and

it would be impossible for the cryptanalyst to determine which one

was intended.

D.  Multiplexing

Multiplexed signals are less readable to the casual observer, but

standard multiplexing offers no real increase in cryptographic strength.  It

can, however, increase the strength of multiple streams of encrypted

data when the multiplexing is done with a pseudorandom interleave

(as discussed above under noise addition).   This makes the multiplexing

and demultiplexing processes more complex with respect to synchronization

and timing, and is likely to introduce more delay into the system

than more straight forward multiplexing arrangements.

V.   COMPARISON OF SELECTED ALGORITHMS

A.  One-Time Key Tape

The one-time key tape, also called the one-time pad, has a tremendous

advantage.  It is the only commonly used encryption method that is

provably secure.  Proving the security of most other systems generally

reduces to an impossible negative proof <197> an attempt to prove

the lack of a method to defeat it.

The way the one-time key tape works is to add each character of the

message to a character of the key modulo the alphabet size.  When

working with any binary stream of data, this is easy to implement

by exclusive-oring the input stream with the key stream.  At the receiving

end, the same thing is done to decrypt the data.  Since P + K + K

= P (modulo 2), the original stream is recovered.

As implied by the name, it is important that each key be used only

once.  If it were used more than once, then the system would be vulnerable

to attack with the known cipher text and corresponding plain text

attack.  The key is obtained from the boolean identity:  K = (P +

K) + P, where K is the key, P is the plain text, P + K is the cipher

text, and all additions are modulo the alphabet size (usually 2).  The

key thus recovered would then be used to decipher the next message

that used it.

According to C. E. Shannon [SHA], for a cipher to be provably secure,

the number of keys must be as great as the number of potential messages.  The

number of keys he refers to, of course, is the number of keys that

yields a distinctly different transformation of the message text into

cipher text.

This method of encryption is provably secure, since an exhaustive

search of the keys applied to any cipher text of a given length will

yield all possible plain text messages of the same length.  Since

there are many messages of the same length that make sense, many of

which have totally contradictory or unrelated meanings, the cryptanalyst

has no way of knowing which one was intended unless he has the key.

Naturally, any method of encryption this secure has a price.  The

price is the sheer volume of keying material that must be kept secure

and managed.  For any communication network or data storage system

that handles large amounts of data, key management becomes a nightmare

with this system.

B.  Linear Shift Register Feedback

Linear shift registers with selected feedback taps are commonly used

to generate pseudorandom sequences for such things as spread spectrum

communications.  They could be (and are, in some cases) applied to

cryptography by exclusive-oring the pseudorandom stream with the plain

text stream.

The main weakness to this approach is the cipher text with corresponding

plain text attack.  If the cryptanalyst obtains the plain text that

came from a certain cipher text, then he can recreate a portion of

the pseudorandom stream by exclusive-oring the two together.  The

linear shift register feedback function can then be expressed as a

system of linear boolean equations that will solve the portion of

the cycle that was received.  A solution of this system of equations

is possible with only enough bits to be a few times as long as the

shift register at the most [DEN].  Since this amount of plain text

required to break the cipher is so much less than the length of the

pseudorandom sequence, it is likely that this solution can then be

applied to additional cipher text to recover it, too.

C.  Exponential Encryption

The classic algorithm of this type is the RSA algorithm, which is

named after the initials of its creators (R. L. Rivest, A. Shamir,

and L. Aldeman).  The security of this algorithm rests on the

fact that even with supercomputers, it is very difficult to factor

the product of two very large prime numbers.  The RSA algorithm has

a unique property in that the key has two parts, one of which may

be made public.  This makes this algorithm very well suited to the

use of digital signatures.

The RSA algorithm defines the relationships between the plain text

message, the cipher text, and the elements of the key as follows [JAC]:

C = P^e mod m

P = C^d mod m

m = pq

where C = Cipher text, P = Plain text, e = encryption key, d = private

decryption key, m = public modulus, and p and q are randomly chosen

large (> 150 digits each) prime numbers.  The receiver chooses a private

key d and calculates a public key e.  d must be relatively prime to

(p - 1) and (q - 1).  The algorithm works because ed = 1 mod

the least common multiple of (p - 1)(q - 1).  The algorithm

is only secure if very large prime numbers are used.  A number of

100 digits is too small - this size of number can be factored

now with a multiprocessor technique developed by Arjen K. Lenstra

of the University of Chicago and Mark Manasse of Digital Equipment

Corporation [COS].

The key generation for RSA is a bit nasty, but is reasonable using

Rabin's test for primality:  Let n = 2^rd+1, where d is odd.

Choose a at random from 1 < a < n - 1.  Accept n as prime if either

a^d = 1 mod n or a^2jd = -1 mod n for some j such that

0 <= j < r, otherwise reject it.

RSA is excellent for public key and authentication use, but it does

have certain disadvantages for general encryption use.  The processing

time required for key generation and for the encryption/decryption

process makes it a poor choice for use with high data rates, although

there are some reasonably efficient ways to do the exponentiation

[CHI].  The complexity of the algorithm makes it more difficult to

implement than many others [VAN].  The worst problem, though is the

way that the complexity of solving the algorithm could be reduced

by several orders of magnitude by mathematical research.  For example,

J. Sattler and C. P. Schnorr recently published some algorithms that

would appear to reduce the complexity of solving an RSA key by a factor

of about 10^4 [SAT].  Granted that we are talking about taking

9 x 10^8 years instead of 1.5 x 10^13 years, but it is

the potential for other discoveries that may drastically reduce this

time even further that is the real threat.

D.  Knapsack

A knapsack or trapdoor algorithm is one that is relatively easy to

perform, but which is very difficult to invert.  The RSA algorithm

is one such algorithm.  Another one, proposed by Ralph C. Merkle,

uses the following vector operation:  given an integer s and an integer

vector a = a1, a2, ..., an find a vector k

= x1, x2, ..., xn where xi is in {0,1}

such that s = x * a. (* denotes dot product.)  [MER]  This

algorithm is referred to in another paper as having been broken [VAN].  Another

algorithm using matrix operations is suggested by H. Retkin [RET].  This

algorithm may be good, but it is not obvious to me that no one will

come up with a good solution to breaking it, too.  Therefore, I prefer

to use encryption algorithms that have simple brute force solutions

that are well understood, but just take too long to perform to be

of concern.

E.  Rotor Machines

Rotor machines were once the state of the art in cryptography.  They

are a way to form a polyalphabetic solution automatically.  An excellent

history of these machines is contained in [KAH], [DEA], and [KOZ].  Although

they are obsolete now due to the superiority of many computer algorithms,

they played a major role in the history of cryptography.

There were many different variations on the rotor machine, but the

main principle of each of them was that the input, which is normally

a indicated by one of 27 (or whatever the alphabet size in use is)

wires is connected to a voltage level by a keyboard.  This wire is

then connected via rotary contacts to a rotor that physically permutes

the position of the wire.  This causes a simple substitution for each

character, or a poly alphabetic substitution. The output is taken

from rotary contacts on the other side of the rotor. Several of these

rotors are placed in series.  After each character is encrypted, one

or more of the rotors is moved to a new position by a ratchet mechanism.  The

net effect is a different simple substitution for each letter in the

message.  Common additions to this scheme are a plugboard style permutation

performed before or after the rotors and the addition of a reflecting

rotor to run the signal back through the rotors in the opposite direction,

using each rotor twice.

The key used is a combination of several things:  (1) the permutation

performed by each rotor (changed by rewiring it), (2) the order in

which the rotors were assembled, (3) the starting position of the

rotors, (4) the plugboard connections, (4) the number of rotors used,

and (5) the ratchet arrangement.  Taken all together, these form a

large key space.  If the entire key space is varied simultaneously

and for every message, then this polyalphabetic substitution is very

secure.  Due to the construction of these machines, some of the variables

were not as easy to change as others. Therefore in practical use,

the variables were not changed all at once.  For example, the number

of rotors and the ratchet arrangement would probably be fixed for

the life of the machine.  Therefore, if this portion of the key is

solved for once, that is enough until a new kind of machine replaces

it, perhaps years later.  The rotor wiring is tedious to change, and

therefore unlikely to be changed very often.  The order of the rotors

might be changed periodically, but during heavy use, the only thing

that was changed for each message is usually the starting position

of the rotors.

Using an alphabet size of 26 and five rotors, varying only the starting

position of the rotors provides a key space of less than 12 million,

which is within the range of possibility of solution by mechanical

computer, and a quick task for one of today's PCs.  With many messages

encrypted with the same rotors, the rotor wiring can be solved by

frequency analysis ``in depth.''  In other words, the permutation

applied to the first letter of each message can be solved by determining

the frequency of occurrence of each code letter and comparing that

with the language of the source language.  This can be done for each

position in the message.  From these substitutions, the rotor wiring

and plugboard connections may be reduced.  This is a more lengthy

process than solving for the rotor starting position, but once done

the solution is likely to be good for a while.  Using this kind of

approach, isolating the parts of the key, the Allies read many German

and Japanese messages during World War II.

Rotor machines can be simulated in a computer program with more flexibility

than in mechanical hardware, with many improvements.  There are, however,

better methods to use in computer algorithms that were not easy to

use in mechanical devices.  Therefore, rotor machines are interesting

to study for historical value, but they are obsolete at a time when

computers are becoming almost as common as televisions.

F.  Codes

Codes, as opposed to ciphers, operate on linguistic units like words,

phrases, and sentences, rather than directly on the units of the alphabet.  Codes

have the advantage that they generally compress the message.  For

example, the codeword ``APPLE'' might mean ``The supplies will be

shipped on Monday via private courier'' and ``GRAPE'' might mean ``The

bid is too high.  Reduce it by 10%.''  If there are only a few different

messages that might be encoded, a substantial savings in space can

be realized.

Codes are useful for compression of information, even when no encryption

is intended.  For example, the Uniform Commercial Code, used for telegraphy

saves on tolls, even though it is published and gives no real security.  Another

example is the use of Q signals on amateur radio, where encryption

is illegal but brevity is important, especially when using a slower

mode of communication like Morse Code.  There QST means ``The following

is a message of interest to all Amateurs'' and QSL means ``My location

is _____.''

When used to hide the meaning of the message, the code must be changed

often. This is much more difficult to do than to change the key to

an encryption algorithm.  Therefore, this technique has limited value

except for some diplomatic and military codes.  For commercial applications

where brevity and security are both required, it is easier to first

encode the plain text with a fixed code to reduce its size, then to

encrypt the encoded text with an encryption algorithm whose key can

be easily changed.  This technique is more secure than the use of

the encryption algorithm alone, even if the code used is widely known.  See

chapter VIII for more on the effects of data compression when used

with encryption.

G.  Galois Field and Hill Cryptosystems

A Galois Field cryptosystem is one in which the letters of the encryption

alphabet are assigned arbitrarily to the integers from 0 to p^n,

where p is prime and n is an integer.  These integers are then operated

on in blocks of m letters by matrix multiplication with a matrix that

represents monic irreducible polynomials of order m - 1.  All operations

are done modulo p^n.  To decrypt the data, the ciphertext is

multiplied in the same manner by the inverse of the encryption matrix

(obtained using the same modulo arithmetic).  The resulting numbers

are then substituted back for the letters they represent.

The Hill cryptosystem is similar, except that n is fixed at one, and

that p need not be absolutely prime as long as several integers less

than p are relatively prime to it.  For more details on how these

cryptosystems work, see the articles by Hill, Cooper, and Patti [HIL]

[HLL] [COO] [PAT].

Both of these systems are based on linear algebra over a limited field

of integers.  Therefore, they are subject to solution of the key in

use by linear algebra if enough cipher text and corresponding plain

text is available.  If the permutation of the encryption alphabet

in use is known, then the amount of corresponding plain and cipher

text needed for a solution is merely the key length.  If the permutation

of the encryption alphabet in use is not known then there must be

a sufficient quantity to also perform statistical analysis attacks

on the alphabet in use.  Therefore, I do not recommend the use of

these systems for serious encryption because of these vulnerabilities.  Not

only are they less secure than their key length (the combined length

of the coefficients of their matrices) would indicate, they are computationally

less efficient than DES and MPJ.  Key generation for these systems

becomes increasingly complex as the size of the key grows, too, because

the probability of coming up with a valid set of monic irreducible

polynomials that form an invertible matrix decreases rapidly with

matrix size.

These cryptosystems, do, however, provide interesting mathematical

illustrations on what can be done with Galois Fields.  They also provide

adequate security for cases where the information being secured is

not of great commercial value, and is therefore unlikely to be cryptanalyzed.

H.  DES

The National Bureau of Standards (now known as NITS) Data Encryption

Standard (DES) was published in the Federal Information Processing

Standards Publication Number 46, dated January 15, 1977 (FIPS PUB

46) [DES].  For details, please refer to that publication.  A summary

of the algorithm follows.

Encryption consists of an initial permutation, sixteen rounds of encryption,

then an inverse of the initial permutation.  Each of the sixteen rounds

of encryption consist of taking the right half of the input block

(32 of the 64 bits) and running it through a nonlinear function of

the 32 bits and an internal key, then adding this result to the left

half of the input modulo two.  This 32 bit answer becomes the next

round's right half block.  The next round's left half becomes the

right half block without modification.  The nonlinear function used

consists of a bit selection E that selects 48 bits from the input

of 32 (several of the bits are repeated).  These 48 bits are added

modulo 2 to the round key of 48 bits.  The result of that operation

are then fed six bits each into eight substitution boxes.  Each of

the eight substitution boxes are different, but the same set of eight

boxes are used for each round.  Each substitution box gives an output

of 4 bits.  The output of these boxes are fed into a permutation P

that rearranges the output in a fixed manner.

The sixteen internal keys are generated from the 56 bit input key

by feeding the input key into a fixed permutation that rearranges

the order of the key bits.  The key is then split into left and right

halves called C and D.  Each half is shifted left one or two times

(according to a fixed table) before generating each internal key.  Each

of the sixteen internal keys is generated by taking the two halves

of the key as shifted and permuting them in a fixed manner.

The key and the resulting internal keys are the only things that vary

in this algorithm.  The initial and final permutations and the contents

of each of the substitution boxes are constant.  The two permutations

used in generating the internal keys are constant.  The bit selection

and permutation used within the nonlinear function are constant.

The strengths of the DES is that its cryptographic strength depends

only on the key, that the algorithm is easy to implement in a single

IC, that it has been well tested an no one has publicly announced

a solution, that hardware and software that uses it is readily available,

and that the algorithm places very few restrictions on key generation

so that random numbers may be generated by the users for use as keys.

The weaknesses of the DES are that the key is too short for security

in the face of anticipated increases of computing power, that it is

old enough that it is likely that someone has broken it (found a short-cut

solution), that hardware implementations of the DES are too slow for

some applications, and that it limits itself to be simpler than is

really necessary with current technology.

VI.  DESIGN CONSIDERATIONS FOR MPJ

The following paragraphs describe the design considerations that I

used in creating the MPJ Encryption Algorithm, and the reasons for

each.

A.  Strength Based on Key

The strength of the system must rely on the security of the key only.  It

cannot depend on the algorithm being kept secret, because the algorithm

will be published.  Even if the algorithm were not published, it would

probably be reverse engineered from software implementations of the

algorithm.  The algorithm must be constructed in such a way that there

is no computationally feasible way to derive the key from samples

of corresponding plain text and cipher text.

B.  Usability of Random Keys

The key selection should be as easy as the random selection of a number

in a given range.  Selecting a very secure key should be no more difficult

than flipping a coin once for each bit of the key, or generating keys

using a pseudorandom sequence combined with random events such as

timing of keystrokes on a computer.  A one bit change in the key should

provide a drastically different transformation, so that a potential

cryptanalyst has no idea when a key that he guesses might be ``close''

to the right one.

C.  Key Length & Block Size

The key length should be significantly longer than the DES 56 bit

size.  A key size of 128 bits (sixteen 8-bit bytes) was chosen as

being very manageable, yet highly secure even when attacked by multiple

array supercomputers.  The block size was also chosen as 128 bits

(twice the size of DES) to provide a significant increase in complexity

of the encryption.

D.  Effort Required to Break

The effort required to break the algorithm by any method should be

so great as to make such a task unfeasible even if significant advances

are made in computer technology.

This requirement is intimately linked to the choice for key size and

block size.  For example, if one thousand keys could be tried every

nanosecond (10^12 attempts per second), an exhaustive key search

that resulted in success after only testing one millionth of the possible

keys (extremely good luck) would take 2^128/((10^12)(10^6))

= 3.4 x 10^20 seconds = 1 x 10^13 years.  If someone figured

out a computational method or weakness in the algorithm that reduced

the complexity by a factor of a million, and that someone also figured

out how to compute the solution a thousand times faster, and they

still believed in one in a million chances at good luck, they could

possibly come up with a solution in only 10,000 years. By way of contrast,

testing keys at the rate of 10^12 per second for DES with its

56 bit key, all of the keys can be tested in 7.2 x 10^4 seconds,

or about 20 hours.  While trying a trillion keys a second is not realistic

with current general purpose computers, current technology could be

applied to dedicated, highly parallel encryption/decryption engines

that could approach that speed - at a cost that would be prohibitive

for all but large governments at today's prices.

Other than time, the other main measure of complexity of an algorithm

is the storage capacity required to implement a lookup table attack.  Suppose

a lookup table were to be constructed that contained the encrypted

version of just one block of cipher text corresponding to a very common

block of plain text (such as 16 ASCII spaces) for every key possible.  For

the MPJ algorithm, this would take 2^128 x 128 = 4.356 x 10^40

bits.  If this memory were constructed with some kind of device that

could store one bit per atom of silicon, this would take 4.356 x 10^40

bits x 28.086 amu/atom x 1.660531 x 10^-24 grams/amu x 10^-6

metric tons/gram = 2.03 x 10^12 metric tons of silicon.  That

is about one thousandth of the mass of Deimos, the smaller of Mars'

two moons.  [CRC]  For DES, the same table would only require about

215 micrograms of silicon.  Nobody has come up with memory that dense,

and fundamental physical limits make such a task difficult, indeed.  It

is not difficult to conceive of some breakthroughs in optical storage

technology coming close, say using a thousand molecules of something

per bit. Under those circumstances, DES would be vulnerable to such

an attack, where MPJ would still be safe.

E.  Computational Efficiency

The MPJ encryption algorithm must be computationally efficient enough

to be implemented in software on a standard IBM PC or compatible (or

on an Apple computer of comparable power), and fast enough to handle

at least 10 megabits per second when implemented in dedicated hardware.  Note

that this is less restrictive with respect to the hardware for DES,

which was required to be simple enough to implement on a single chip

using 1970s technology.

F.  Communication Channel Efficiency

The MPJ encryption algorithm must not significantly increase the size

of the plain text when encrypting it.  This precludes the use of noise

addition as a technique to be used.

G.  No Back Doors or Spare Keys

While it may be impossible to guarantee that no ``back doors'' or  ways

to decipher a message without the key exist, the algorithm should  be

sufficiently complex combination of simple, well-understood operations  that

no help is offered to the cryptanalyst from the structure of  the

algorithm. Spare keys (the situation where more than one unique  key

will decipher a message) are avoided by making the number of keys  possible

much less than the number of possible transformations that can be

done  on a set of blocks.  This is true for MPJ because 2^128

<< 2^128!.

VII.      MPJ ENCRYPTION ALGORITHM

A.  Description

The MPJ Encryption Algorithm can be looked at from the outside like

a rather large electronic codebook that operates on 128 bit blocks.  The

algorithm may be used in several modes.  It can be used directly in

electronic codebook mode, in block chaining with ciphertext feedback

mode, in block chaining with plain text feedback mode, and in stream

cipher generation mode.

Given a 128 bit key, instructions to encrypt or decrypt, and a 128

bit input block, a unique 128 bit output block comes out. Encryption

and decryption are inverse operations that can be done in either order.  For

example, if P is the plain text block, C is the cipher text block,

EK1 is encryption with key 1, and DK1 is decryption

with key 1, then C = EK1(P); P = DK1(P).  A different

cipher text results if the plain text is decrypted first, but DK1(EK1(P))

= P = EK1(DK1(P)).  Multiple encryption can be done

with several keys, as in these examples with three:

C = EK1(EK2(EK3(P))); P = DK1(DK2(DK3(C)))

or a different method:

C1 = EK1(DK2(EK3(P))); P = DK1(EK2(DK3(C)))

1.  Overall Structure of MPJ

Before encryption or decryption occurs, the substitution boxes are

filled based on the input key.

To encrypt a 128 bit input block, it is run sequentially through ten

rounds of substitution.  Each round of substitution operates on the

16 eight-bit bytes that make up the input block individually.  In

between each round of substitution is a round of permutation, for

a total of 9 rounds of permutation.  The permutation operation is

identical each time.  Each operation (substitution and permutation)

has an inverse operation.  Decryption is done by running the inverse

operation of each of the above steps in the opposite order.

After one round of permutation, each bit in the output block depends

on every bit in the byte that it is in and on eight bits of the key.  After

a round of permutation and the second round of substitution, each

bit in the block depends on eight bytes of the input block and on

eight bytes in the key.  After the third round of substitution, each

bit in the block is functionally dependent on 15 of the input bytes

and on 15 bytes of the key.  After the fourth round of substitution,

each bit in the block is functionally dependent on every bit of the

input block and on each bit of the key.  After the tenth round of

substitution the functional dependence on every bit of the key and

the input block is so complex that it would be infeasable to determine

the key used, even if the cryptanalyst has known corresponding plain

and cipher text and knows the algorithm used.  Because the substitution

boxes are totally nonlinear functions, the problem of finding each

of the 40,960 internal key bits is a tough one, indeed.  It would

be easier to find the original key of only 128 bits by brute force

- something that is difficult, indeed.

2.  Substitution Boxes

The substitution boxes are designed to be big enough to provide a

large number of possible arrangements, but small enough to still fit

comfortably within the memory of an MS-DOS computer.  Each substitution

box is therefore a collection of 16 substitution boxes that operate

on only 8 bits at a time, with a total of 256 entries.  The substitution

boxes may be an array or look-up table in a computer, or they may

be implemented in hardware using RAM.

The substitution boxes are filled during the internal key generation

process with a permutation of numbers that depends on the main key

in a different way for each substitution box.

3.  Wire Crossings

The wire crossings serve to extend the functional dependence of the

output block on the input block across the internal byte boundaries.  This

is done by making each byte of the output of the permutation to contain

one bit from each of eight different bytes.  The selection of bits

was chosen to make a computer implementation fairly efficient.  A

pure hardware implementation of this operation is, of course, much

faster, since it involves only the propagation delay of the signal

from one end of a short wire to the other.  The input block bit positions

are numbered from left to right (MSB to LSB) as 127 down to 0, then

the new positions those bits occupy after the wire crossing are (MSB

to LSB):

55, 46, 37, 28, 19, 10, 1, 120, 111, 102, 93, 84, 75, 66, 57, 48,

39, 30, 21, 12, 3, 122, 113,  104, 95, 86, 77, 68, 59, 50, 41, 32,

23, 14, 5, 124, 115, 106, 97,  88, 79, 70, 61, 52, 43, 34, 25, 16,

7, 126, 117, 108, 99, 90, 81, 72, 63, 54, 45, 36, 27, 18, 9, 0

4.  Key Generation

Internal key generation in the MPJ encryption algorithm consists of

filling all of the substitution boxes (arrays in software or RAM in

hardware).  There are 16 substitution boxes used for each of 10 rounds.  Each

substitution box contains 256 entries of 8 bits each.  Therefore,

the actual internal keys have a combined length of 16 x 10 x 256 x

8 = 327,680 bits.  These are obtained by manipulation of the 128 input

key bits.  Note that trying to attack the MPJ encryption algorithm

by brute force trial and error using internal keys instead of using

the 128 input key bits is ridiculous.  The calculations given above

indicate how difficult even a 128 bit random key is to solve by brute

force.  The only way that an attack on internal keys is useful is

if there are parts of the internal keys that can be solved for separately,

without having to solve for as many as 128 bits at once.

Attacks against the internal key structure are made computationally

intractable in three ways.  First, the smallest chunk of the internal

key that could be meaningfully solved for is the contents of one of

the substitution boxes.  Each of these contain 256 bytes, or 16 times

the length of the input key. Remember that the addition of each bit

doubles the complexity of the solution, so this is not attractive

compared to solving for the input key.  The second impediment to this

attack is to make the relationship between the contents of the substitution

boxes and the input key quite nonlinear.  The way this is done is

by using a combination of rotating bit selection and the use of the

last substitution box filled to compute the contents of the next one.  The

third line of defense against the solution of internal keys is the

use of ten nonlinear rounds of encryption to make any attempt at constructing

a system of linear equations to solve for the internal key given known

plain text and corresponding cipher text infeasable.  Note that each

round of encryption in MPJ causes the whole 128 bit block to be changed,

where each of the rounds in DES only modifies half of its 64 bit block.  Therefore

the internal structure of MPJ is very difficult to solve for, with

each bit of the output having a nonlinear functional dependence on

every bit of the input and on the contents of 1 + 8 + 15 + (7 x 16)

= 136 of the substitution boxes, containing a total of 34,816 bits.  If

the substitution boxes were linear, this solution would be possible,

but there is no method I know of to solve such a system of nonlinear

equations, either here or the simpler case of the DES internal structure.

To find the inverse of the substitution box functions, separate inverse

substitution boxes are formed that place each address of a substitution

box at the address of the data contained in the substitution box.  For

example, if the contents of the first substitution box in the first

round of encryption for the input value of 1 is 219, then the contents

of the first inverse substitution box in the last round of decryption

for the input of 219 will be 1.  In equations, SI[i, j, S[i, j, k]]

= k for all i, j, k, where the first index of the array is the round

of encryption, numbered from 0 to 9, or the round of decryption, numbered

from 9 to 0 (defined differently for programming convenience); the

second index is the position of the 8 bits that the substitution applies

to in the input block, numbered from 0 to 15, left to right; and the

third index is the value of the 8 bits input to the block.

The actual algorithm for substitution box filling consists of three

main parts.  The first one is a nested loop to fill one substitution

box, permute the key with the same permutation used for the encryption

operation, then to run each byte of the key through the substitution

box that was just filled. This is done for each round of encryption,

from 0 to 9, with each substitution box used in one round of encryption,

from left to right, done within each of these rounds.  In other words,

the index of the three dimensional array that selects the position

of the substitution box within each array varies faster than the index

that selects the number of the round of encryption that the box uses.

The second algorithm is the one that fills each substitution box based

on the contents of the input key, as modified by the previous steps.  This

algorithm uses bit selection and rotation to determine where each

value of the output of the substitution box goes.  To be an invertable

function, each of the possible output values from 0 to 255 must appear

exactly once somewhere within the array with 256 possible slots, numbered

from 0 to 255.  The first number placed in the array is 255, and there

are 256 possible places to put it.  The second one is 254, with 255

places to put it.  This is continued until the last value, 0 is placed

in the one remaining slot.  For the first half of the values, the

formula pos = (n * m) div 255 is used to determine which of the unfilled

slots (counting in index order from 0 to n) is to be used to contain

n.  The variable pos is the position (counting only unfilled slots),

* denotes multiplication, and div is the integer division operator

(remainders are ignored).  The variable m is determined for the first

value of n (255) by selecting one bit from each byte of the key, starting

with the least significant bit from the least significant byte of

the key.  The next bit (place value of 2) is selected from the next

most significant bit of the key, and so on until m has eight bits.  For

the next value of n (254), the same thing is done, except that the

bit selected is one bit position to the left of the one selected last

time.  The place value of the bit selected is the same in m as it

is in the byte it came from.  The bit to the left of the MSB in a

byte is the LSB of the byte to the left of it.  The byte to the left

of the most significant byte is the least significant byte.  For the

values of n from 127 to 0, then the same thing is done, except that

only seven bits are selected for m, and the position is determined

from pos = (n * m) div 127. For a more concise explanation of this

process, see the commented pascal procedure makesbox in the program

in appendix A.

The third algorithm used in internal key generation is the generation

of inverse substitution boxes.  This is done with a simple nested

loop that fills the inverse substitution boxes according to the formula

si[i, j, s[i, j, k]] for i from 0 to 9, j from 0 to 15, and k from

0 to 255.  The array si is the collection of inverse substitution

boxes, and the array s is the collection of substitution boxes filled

by the above two algorithms.  Filling of inverse substitution boxes

is only needed for decryption mode.

B.  Implementation in Pascal

Pascal source code for a program to implement the MPJ Encryption Algorithm

is given in Appendix A.

1.  Exceptions from Standard Pascal

This program was written for MS-DOS computers and compiled using Borland

Turbo Pascal 5.0.  To adapt this program for use on another system,

note that the following features of Turbo Pascal that do not conform

to the ANSI/IEEE770X3.97-1983 Standard Pascal were used in this program:

(i)  The assign procedure is used to associate an operating system

file name with a Pascal file name.

(ii)  The nonstandard file handling procedures blockread, blockwrite,

close, and seek, were used.  The nonstandard function filepos was

used.

(iii)  The type longint (a 32 bit integer) was used for input handling

to allow range checking without invoking a system error for a number

that was only slightly out of range.  A 16 bit integer could be used

instead.

(iv)  A generic file type was used, which is not defined in ANSI Pascal.

(v)  The nonstandard operators shl and xor were used.  The shl could

be replaced with integer division by 2 (div 2), and the xor could

be replaced by expressions of the form ((A and (not B)) or ((not A)

and B)).

(vi)  Logical operators (and, or, not, xor) were used with integers

to perform bit-wise operations.

(vii)  The + operator was used to concatenate strings.

(viii)  The uses clause links in separately compiled units.

(ix)  The include comment {$I filename} includes additional source

code to be compiled with the current file.

(v)  Additional nonstandard features were used in writing startup  and

the file handling functions that are unique to MS-DOS.  These functions

would best be rewritten for a different target system.

2.  Main Program

The main program calls startup to initialize variables, get the key

from the user, and determine which files are to be encrypted or decrypted.

It then calls makesbox to generate the internal keys (fill the substitution

boxes). If the decryption mode is to be used, it calls makesi to fill

the inverse substitution boxes.  It then does the actual encryption

or decryption.

The main program allows for the encryption of multiple files (specified

using MS-DOS wildcards), and passes these files to the encryption

routine one block at a time.  To provide additional security, files

are encrypted in place, with each block of output overwriting the

corresponding input block in the same place on the disk.  If it is

desired to keep a copy of both encrypted and plain text versions of

the file, the input file should be copied before running this program.

Because this program uses block chaining with ciphertext feedback,

only the encryption mode of the MPJ algorithm is used, the source

of the feedback (input or output) being determined by the decryption

boolean variable.  This mode is used because (1) it obscures repetitive

patterns in the source file that the electronic codebook mode might

not hide, (2) it accommodates files of any number of bytes (including

a short block at the end) with no complications and results in an

output file that is the same length as the input file, and (3) it

is slightly faster in operation because the procedure makesi does

not have to be called.  This mode does require an initialization vector,

but since the initialization vector is encrypted before use, a constant

initialization vector may be used.

To use the electronic codebook mode, delete the for statement following

the calls to encrypt, replace one call to encrypt with a call to decrypt,

and remove the comment delimiters from around the call to makesi.

3.  Procedures Encrypt & Decrypt

These procedures directly reflect the overall structure of MPJ.  The

are simply calls to the routines that perform the substitutions and

permutations, calling them in the proper order and passing a parameter

to the substitution routines that select the right set of substitution

(or inverse substitution) boxes for the operation being done.  The

procedure encrypt makes 10 calls to substitute, with 9 calls to permute

(one call to permute between each pair of calls to substitute.  The

procedure decrypt makes 10 calls to isubst, with 9 calls to ipermute,

done in the inverse order of encryption.

4.  Procedures Permute & Ipermute

Permute just scrambles the order of the bits in the 128 bit block

in such a way as to maximize the dependence of each byte on every

other byte of the input.  Each byte is formed by selecting the least

significant bit from the least significant bit of the corresponding

input byte, then the next most significant bit from the next byte

to the left, and so on.  The least significant byte is considered

to be to the left of the most significant byte. Ipermute is just the

inverse of this permutation, with bytes being selected to the right

instead of the left.

5.  Procedures Substitute & Isubst

These procedures are simple array lookups in which each byte of the

input block is used as the third index of the array s for the procedure

substitute or si for the procedure isubst.  The output is the contents

of the array.  The first index is the round of encryption or decryption

(counting backwards if it is decryption), and the second index of

the array is the byte position (0 to 15) within the input and output

blocks.

C.  Implementation in Hardware

For use with high data rates (greater than 10 Megabits per second),

it is desirable to implement the MPJ algorithm directly in hardware.

This also provides the advantage of greater security against tampering

than a program on a general purpose computer might be subject to.

The hardware implementation consists of six basic elements:  (1) Key

generation, (2) serial to parallel conversion, (3) substitution, (4)

wire crossings, (5) parallel to serial conversion, and (6) timing

control.  The key generation is done by a program running in a microprocessor.  This

program takes the key as input and fills the substitution boxes, which

are simply static RAM chips. Serial to parallel conversion and parallel

to serial conversion is done with shift registers.  The wire crossings

are done with physical wire crossings (or printed circuit board trace

crossings) - a technique that is hard to beat for speed.  Timing

control is done with a sequential circuit that is synchronized to

the incoming data stream.

The output data stream will, of necessity, be delayed by at least

the block size of 128 bits, and some kind of protocol must be used

to synchronize the blocks at the sending and receiving ends.

The following diagram shows a block diagram of a hardware implementation

of the MPJ Encryption Algorithm.  There are, of course, many variations

possible.  For example, block chaining could be used with hardware

exclusive-or gates to do the modulo 2 addition and flip-flops for

unit delays.

D.  Strengths & Weaknesses

The main strengths of the MPJ encryption algorithm are:

It is much more secure than DES.

When used in conjunction with data compression, MPJ is probably

more secure than many of the best military and diplomatic codes &

ciphers.

It is easy to generate a key for MPJ.

It is capable of high data rates when implemented in hardware.

It will run on a personal computer.

It takes more time to generate internal keys from an external

key than to actually encrypt or decrypt data, making exhaustive key

search attack more difficult than normal use.

The algorithm is published and may be freely used for any

legal purpose.

There has not been a lot of time for anyone to search for

a short-cut solution to MPJ, nor the volume of sensitive material

to motivate such a search as DES has had.

The main weaknesses of the MPJ encryption algorithm are:

The key and block sizes are fixed (optimized for the memory

and processing power of the IBM PC), so it doesn't take very good

advantage of more powerful computers other than running faster.

The software implementation isn't very fast.

There is no way to prove that there is no short-cut solution

to MPJ (or DES, or almost any encryption algorithm except for the

one-time key tape).

Since the MPJ encryption algorithm is my own invention,

I could possibly be blind to some weakness that it might contain.

VIII.     DATA COMPRESSION

A.  Purpose

Data compression has two obvious purposes:  to save on communication

channel capacity required to send a message and to save on media capacity

required to store a message.  The third purpose for data compression,

pointed out by C. E. Shannon, is to make decryption of a message more

difficult [SHA].  In reading through the history of cryptanalysis,

it became very obvious to me that the one thing that all cryptanalysis

relies on the most is the presence of a great deal of redundancy present

in natural languages.  This is especially true for messages that the

cryptanalyst already has something of an idea of what the message

is about.  It is this redundancy that enables the cryptanalyst to

tell which of several possible solutions is the right one - the

one that ``makes sense.''  For example, if the message is between

English-speaking parties, then the of the possible messages ``ACCEPT

THE PROPOSAL,'' ``A;LSDI FUPE ZAARED,'' and ``XXDKFJ DDDLKJDJFFZA,''

the first message is instantly recognized as the only one that makes

sense.  The reason that this is so obvious is because of the redundancy

of natural languages.  If all of the redundancy of a message is removed,

then there is no criteria to select one possible answer over another,

and the cryptanalyst is left baffled.

The task at hand is to try to eliminate or at least reduce the amount

of redundancy in a message before encryption, then restore the message

to its original meaning after decryption.  If all of the redundancy

of a message is removed, it means that all possible messages that

can be constructed from an alphabet have meaning, and that the length

of a message is inversely proportional to its probability.  This ideal

can probably be achieved only for special circumstances with very

limited messages.  For use with general correspondence in a natural

language, however, it is not practical to totally remove the redundancy

from all messages.  In fact, it is very difficult to even measure

redundancy or entropy in a natural language.  The concepts of redundancy

and entropy (as defined in communication theory texts) are useful

for comparisons of various methods of redundancy reduction.

Most existing methods of redundancy reduction use either a manually

constructed code like the amateur radio operator's Q-signals or an

automatic method that operates on either the alphabet or some groupings

of alphabet symbols, like trigraphs.  Manual techniques are fine for

their intended application, but automatic methods that are well-suited

for computer implementation are better for use with encryption.  Automatic

methods are likely to achieve good results with much greater ease

of use and less opportunities for errors.

The probability of occurrence of various letters in most natural languages

are fairly constant over a wide range of types of text, and have been

studied and published many times.  These probabilities for English

letters are published in several of the texts on cryptography listed

in the references section of this thesis.  Probabilities of digraphs

(groups of two letters) and trigraphs (groups of three letters) have

likewise been studied, and are fairly constant for a given language.  Just

changing the representation of trigraphs to variable length codes

that are shorter for more common trigraphs and longer for less common

trigraphs results in a substantial savings in length of an English

message.

There are many such methods for compressing data that are fairly straight

forward.  This and other methods, many of which are discussed by James

Storer in his book on data compression [STO].  There are also several

public domain and share-ware programs that are available on many bulletin

boards that perform data compression.  My favorite is PKARC, written

by Phil Katz.  It dynamically analyzes each input file to determine

which one of five compression methods or storing with no compression

results in the smallest file.  The compression methods used by PKARC

are repetition coding, Huffman encoding, Ziv-Lempel-Welch compression,

and two different variations on Dynamic Ziv-Lempel-Welch compression

[KAT].  PKARC also has an option that allows a rather crude encryption

of the archive files with a password. Although this doesn't provide

strong cryptographic security, it does help to obscure the redundancy

that PKARC adds back in, in the form of cyclic redundancy checks on

files and file headers.  The CRC checks and file headers would be

a good point of vulnerability when trying to cryptanalyze an archived

(compressed) file.

My feeling was that it was possible to compress natural language text

even more than was commonly being done by using linguistic parsing

of the text to form a dictionary.  I was partially right.  My method

does remove more redundancy and make most text files smaller and more

secure.  It won't win any speed contests with PKARC, however, which

I recommend that you use if speed is of great importance or if you

are going to compress binary files (such as executable programs).

In comparing the performance of SQUEEZE with PKARC, I compressed a

4,316,030 byte ASCII text file (the King James Version of the Holy

Bible) to 1,312,493 bytes with a language code file of 163,236 bytes.  PKARC

compressed the same file to 1,779,184 bytes.  This is a removal of

69.6% of the redundancy for SQUEEZE (or 65.8% if you add the size

of the language code file), compared with a redundancy reduction of

only 58.8% for PKARC.  This may not seem like much of a difference,

but the smaller compressed file fits on a 1.44 Megabyte 3 1/2 inch

floppy disk, and the other one doesn't.  This would be more of a significant

achievement if SQUEEZE weren't so slow.  It took well over two hours

to do this running on one of the fastest PC's available (25 MHz 80386),

while PKARC worked for about a minute on the task.  I believe that

both the speed and amount of redundancy reduction can be improved

upon.  I am therefore publishing the source code for what I did in

Appendix B, so that someone else may be able to build upon this idea.

B.  Linguistic Parsing

Instead of developing a Huffman code based on trigraphs or other such

arbitrary chunks of fixed length, why not develop a Huffman code based

on the way people read words?  Letter combinations made from statistically

properly distributed trigraphs will probably contain a lot of valid

English words, but they will contain a lot of garbage, too.  If, however,

all symbols correspond to English words (or punctuation), then a collection

of random symbols will look a lot more like English.  Of course, the

arrangement of words will probably not make sense in either group,

but an intuitive analysis indicates that a code based on linguistic

parsing of words will contain a lot less redundancy than will a fixed-length

code.

To define linguistic symbols, I called any grouping of alphabetic

symbols unbroken by non-alphabetic symbols a word symbol.  I called

any group of contiguous spaces a space symbol.  I called a carriage

return + line feed combination a new line symbol.  Any other punctuation

and numbers were called symbols themselves.  There may be more efficient

ways to split up text into symbols, since, for example, words are

almost always followed by a space.  I also counted upper and lower

case words different symbols.  It may have been more efficient to

use a modifier symbol to indicate that the following word is capitalized

or written in all capitol letters.  Of course, if all of the input

text is all upper case, as would be the case for some telegraphic

type communications systems, this is not a concern.

Once a linguistic symbol is detected in the input text, it is encoded

with its corresponding Huffman code representation.  This is then

decoded at the other end of the communications link or after retrieving

the data from storage, using the same Huffman code.

So that the Huffman code generation does not have to be done each

time a file is compressed, an escape code that meant ``the following

symbol of ____ bits is not in the code, and is to be interpreted directly''

is used.  For these symbols, a simple suppression of the most significant

bit is used, resulting in a savings of one out of eight bits.

C.  Huffman Coding

Given a set of symbols and their associated probabilities of occurrence,

a Huffman code for those symbols is constructed by generating a tree.  Start

with a set of symbols as individual nodes.  While there are still

separate nodes, combine the two nodes with the lowest probability

(resolve ties arbitrarily) into a new node with a probability equal

to the sum of the probabilities of the two nodes that were combined.  Attach

one of the nodes to the new node via a 1 and the other node via a

0.  After all of the nodes are combined, the code for each symbol

is the sequence of ones and zeroes assigned to the branches of the

tree starting at the root and ending at the symbol. [STO]

Note that this algorithm results in a very similar code to a Shannon-Fanno

code (which generates the tree by starting at the root and building

out to the leaves), but it is computationally more efficient to implement.

D.  Pascal Programs

The pascal programs that implement this linguistic file compression

are in appendix B.  The first one, COUNT, merely counts the frequency

of occurrence of words in sample text.  It is not necessary that the

text analyzed be the same text to be compressed, just that it be similar.  For

example, better compression would be expected when using a code based

on personnel records when using that code to compress personnel records

than when compressing medical discussions.  However, the significant

gains made in compressing the more common words in the code offset

the lesser compression (or even possible expansion) encountered when

words not in the code are encountered.  It is also desirable to COUNT

a large sample of text to ensure accurate word frequencies.

The second program, MAKETREE takes the frequency data from the output

file generated by COUNT and generates a Huffman code from it.  It

then writes this tree out into a file that is used by SQUEEZE.  To

get reasonable speed from this program, it uses as much memory as

MS-DOS will let it for the more frequently accessed information, and

uses a temporary file for the rest of the information.  The program

runs significantly faster if this temporary file is on a RAM disk

in extended or expanded memory (beyond the basic 640 KBytes of the

MS-DOS domain).

SQUEEZE, the third program, does the squeezing and unsqueezing of

files based on the output file of MAKETREE.  The same Huffman coding

tree can be used for many different input text files, because the

statistics of English (especially when the same types of text, such

as all business letters or all technical reports) remain fairly constant

even when the content of the text conveys different meanings.  SQUEEZE

handles words not in its dictionary in stride, but highlights them

as they scroll across the screen in processing so that the user can

get a feel for how well the text is matched to the code tree in use.

If only a few proper names and such are highlighted, then the code

in use is a good one to use.  If not, then perhaps a regeneration

of the code would be in order.  Note that the code file used to SQUEEZE

a file must be identical to the one used to unSQUEEZE it, so it must

either be stored or sent with the file, or sent in advance and agreed

upon by communicating parties.

After testing these programs on various ASCII text files, it can be  seen that

significant compression can be obtained, and that the compressed  files can be

recovered reliably.  There is, however, room for improvement  in the exact way

that text is parsed and in the speed of the implementation.  Nevertheless,

when maximum security is desired, I recommend first compressing an  ASCII text

file with SQUEEZE, then encrypting it with CRYPTMPJ.  The  code file

(LANGUAGE.COD) used by SQUEEZE and the encryption key should  be sent by a

separate, secure channel to the receiving party (or stored  in a physically

secure location for data storage) in advance of the  message.  The combination

of minimal natural redundancy in the message  sent and the use of a good

encryption algorithm should be difficult  to crack.

IX.       CONCLUSION

The MPJ Encryption algorithm provides an alternative to DES that is

more secure, providing that the software and/or hardware does not

get corrupted or tampered with, and providing that the keys are managed

effectively.  I have implemented and tested the algorithm in software

for the IBM PC and compatible machines.  The algorithm may be implemented

directly in hardware for faster data rates.  The MPJ Encryption Algorithm

may be used by itself, or it may be improved upon by compressing the

data before encryption.

The use of data compression in conjunction with any encryption algorithm

drastically increases the security of the encrypted data.  For natural

language text, one approach that yields improved compression over

the compression of constant size blocks of data is the compression

of linguistic units, such as words.  I have achieved reversible compression

of such files to less than 35% of their original size using linguistic

parsing and a Huffman code. For binary data, such as computer programs,

the use of existing techniques, such as those in PKARC, written by

Phil Katz and available on most computer bulletin boards, are recommended.

It is hoped that the MPJ encryption algorithm, as well as some of

my ideas on data compression, will make a positive contribution towards

data security, communications privacy, and efficiency of data storage

and transmission.  May God prosper those who use this knowledge and

build upon it for good.

REFERENCES

[ALB]  Douglas J. Albert and Stephen P. Morse,``Combating

Software Piracy by Encryption and Key Management,''Computer,

Vol. 21, No. 5, April 1984, pp. 68-73 Los Alamitos, CA: IEEE Computer

Society.

[AME]  Stanley R. Ames, Jr., Morrie Gasser, and Roger

R. Schell, ``Security Kernel Design and Implementation:  An Introduction,''

Computer, Vol. 16, No. 7, July 1983, pp. 14-22 Los Alamitos,

CA: IEEE Computer Society.

[BAL]  W. W. Rouse Ball & H. S. M. Coxeter, Mathematical

Recreations & Essays.  New York:  The MacMillan Company, 1962.

[BAU]  Friedrich L. Bauer, ``Cryptology - Methods

and Maxims,'' in Proceedings of the Workshop on Cryptography at

Burg Feuerstein, Germany, March 29-April 2, 1982, Berlin, Heidelberg,

New York: Springer-Verlag, 1983.

[BEK]  H. J. Beker, ``A Survey of Encryption Algorithms,''

in International Conference on Secure Communication Systems,

22-23 February 1984, London, New York: Institute of Electrical

Engineers, 1984. TK5102.5.I519.1984

[BET]  Thomas Beth, Introduction to Proceedings of

the Workshop on Cryptography at Burg Feuerstein, Germany, March 29

- April 2, 1982, Berlin, Heidelberg, New York: Springer-Verlag,

1983.

[BON]  D. J. Bond, ``Practical Primality Testing,'' in

International Conference on Secure Communication Systems, 22-23

February 1984, London, New York: Institute of Electrical Engineers,

1984. TK5102.5.I519.1984

[BRO]  C. B. Brookson and S. C. Serpell, ``Security on

the British Telecom Satstream Service'' in International Conference

on Secure Communication Systems, 22-23 February 1984, London,

New York: Institute of Electrical Engineers, 1984. TK5102.5.I519.1984

[BUR]  Dave Bursky, ``Protect Your EEPROM Data and Gain

More Flexibility,'' Electronic Design, 26 May 1988, pp.43-48.

Waseca, MN:  Brown Printing Co.

[CHA]  Leslie S. Chalmers, ``An Analysis of the Differences

Between the Computer Security Practices in the Military and Private

Sectors,'' in Proceedings of the IEEE 1986 Symposium on Security

and Privacy, Oakland, CA, April 7-9, 1986. QA76.9.A25.595.1986

[CHI]  H. R. Chivers, ``A Practical Fast Exponentiation

Algorithm for Public Key,'' in International Conference on Secure

Communication Systems, 22-23 February 1984, London, New York:

Institute of Electrical Engineers, 1984.  TK5102.5.I519.1984

[COO]  R. H. Cooper, ``Linear Transformations in Galois

Fields and their Application to Cryptography,'' Cryptologia Magazine,

Volume 4, Number 3, July 1980, pages 184-188.  Republished electronically

by Tony Patti.

[COS]  Terry Costlow, ``Global Computer Network Cracks

Cryptographic Mathematics Barrier,'' Electronic Engineering Times,

Issue 508, 17 October 1988, p. 14, Manhasset, NY:  CMP Publications,

Inc.

[CRC]  Handbook of Chemistry and Physics, 53rd

Edition, Cleveland, OH: The Chemical Rubber Company, 1972.

[DEA]  Cipher A. Deavers and Louis Kruh, Machine

Cryptography and Modern Cryptanalysis, Norwood, MA: Artech House,

Inc., 1985.  Z103.D43.1985

[DEN]  Dorothy Elisabeth Robling Denning, Cryptography

and Data Security, Reading, MA; Menlo Park, CA; London; Amsterdam;

Don Mills, Ontario; Sydney: Addison-Wesley Publishing Company, 1982.

QA76.9.A25.D46.1982

[DES]  National Bureau of Standards, Federal Information

Processing Standards Publication Number 46 Dated 15 January 1977.

[DOH]  Richard Doherty, ``FBI Nabs Pirated VCII,'' Electronic

Engineering Times, Manhasset, N. Y.:  CMP Publications, Inc., Issue

500, August 22, 1988.

[EET]  ``NSA OKs Decryption Unit,'' Electronic Engineering

Times, August 29, 1988, Manhasset, N. Y.: CMP Publications, Inc.

[FRA]  Ole Immanuel Franksen, Mr. Babbage's Secret

- The Tale of a Cipher and APL, Englewood Cliffs, NJ: Prentice-Hall,

1985. Z103.B2.F72.1985

[FRI]  H. J. Beker, J. M. K. Friend, and P. W. Halliden,

``Simplifying Key Management in Electronic Fund Transfer Point of

Sale Systems,'' in International Conference on Secure Communication

Systems, 22-23 February 1984, London, New York: Institute of

Electrical Engineers, 1984.  TK5102.5.I519.1984

[FRM]  Lester J. Fraim, ``Scomp: A Solution to the Multilevel

Security Problem,'' Computer, Vol. 16, No. 7, July 1983,pp.

26-34, Los Alamitos, CA: IEEE Computer Society.

[GOO]  G. Goos and J. Hartmans, Lecture Notes in

Computer Science.  Z102.5.W67.1982

[GOR]  J. A. Gordon and H. Retkin, ``Are Big S-Boxes

Best?'' in Proceedings of the Workshop on Cryptography at Burg

Feuerstein, Germany, March 29-April 2, 1982, Berlin, Heidelberg,

New York: Springer-Verlag, 1983.

[HAE]  D. G. Haenshke, D. A. Kettler, and E. Oberer,

``Network Management and Congestion in the U. S. Telecommunications

Network,'' IEEE Transactions on Communications, volume COM-29,pp.

376-385, April 1981.

[HEL]  Gilbert Held and Thomas R. Marshall, Data

Compression, Second Edition; Chichester, New York, Brisbane,Toronto,

Singapore: John Wiley & Sons, 1987.  QA76.9.D33.H44.1987

[HIL]  Lester S. Hill, ``Cryptography in an Algebraic

Alphabet,'' American Mathematical Monthly, June 1929, pages

306-312, the American Mathematical Society.  Republished electronically

by Tony Patti by permission.

[HLL]  Lester S. Hill, ``Concerning Certain Linear Transformation

Apparatus of Cryptography,'' American Mathematical Society,

March 1931, pages 135-154.  Republished electronically by Tony Patti

by permission.

[JAC]  A. M. Jackson, N. A. McEvoy, and B. B.Newman,

``Project Universe Encryption Experiment'' in International Conference

on Secure Communication Systems, 22-23 February 1984, London,

New York: Institute of Electrical Engineers, 1984. TK5102.5.I519.1984

[KAH]  David Kahn, The Codebreakers - The Story

of Secret Writing, New York: The MacMillan Company, 1987. Z103.K28

[KRU]  Cipher A. Deavers, David Kahn, Louis Kruh, Greg

Mellen, and Brian Winkel, Cryptology Yesterday, Today and Tomorrow,

Norwood, MA: Artech House, Inc., 1987.  Z103.C76.1987

[LAN]  Carl E. Landwehr, The Best Available Technologies

for Computer Security,'' Computer, Vol. 16, No. 7, July 1983,pp.

86-100, Los Alamitos, CA: IEEE Computer Society.

[KAT]  Phil Katz, PKARC FAST! Archive Create/Update

Utility Version 3.5 04-27-87, published electronically in PKX35A35.EXE.

[KON]  Alan G. Konheim, Cryptography:  A Primer,

New York: John Wiley & Sons.  Z103.K66

[KOZ]  Wladyslaw Kozaczuk, Enigma - How the German

Machine Cipher was Broken and How it was Read by the Allies in World

War Two, University Publications of America, Inc., 1984.D810.C88.K6813.1984

[LIN]  Christer Lind<130>n, ``Electronic Seal for Protection

of Electronic Money in Sweden, Finland, and Norway'' in Proceedings

of 1986 International Carnahan Conference on Security Technology:

Electronic Crime Countermeasures, August 12-14, 1986. QA76.9.A25.C41.1986

[LOD]  N. Lodge, B. Flannaghan, and R. Morcom,``Vision

Scrambling of C-MAC DBS Signals,'' in International Conference

on Secure Communication Systems, 22-23 February 1984, London,

New York: Institute of Electrical Engineers, 1984. TK5102.5.I519.1984

[LU]  W. P. Lu and M. K. Sandareshan, ``A Hierarchical

Key Management Scheme for End-to-End Encryption in Internet Environments''

in Proceedings of the IEEE 1986 Symposium on Security and Privacy,

Oakland, CA, April 7-9, 1986.  QA76.9.A25.595.1986

[MCL]  Vin McLellan, ``Drugs and DES:  A New Connection,''

Digital Review, August 24, 1987.

[MER]  Ralph C. Merkle, Secrecy, Authentication,

and Public Key Systems, Ann Arbor, MI: UMI Research Press,1982.  QA76.9.A25M47.1982

[MEY]  Carl H. Meyer & Stephen M. Matyas,Cryptography:  a

New Dimension in Computer Data Security - A Guide for the Design

and Implementation of Secure Systems, New York, Chichester, Brisbane,

Toronto, Singapore: John Wiley & Sons, 1982.  Z103.M55.1982

[MIT]  C. J. Mitchell, ``A Comparison of the Cryptographic

Requirements for Digital Secure Speech Systems Operating at Different

Bit Rates,'' in International Conference on Secure Communication

Systems, 22-23 February 1984, London, New York: Institute of

Electrical Engineers, 1984.  TK5102.5.I519.1984

[PAT]  Tony Patti, A Galois Field Cryptosystem,

1986. Published electronically by Tony Patti, editor of Cryptosystems

Journal, 9755 Oatley Lane, Burke, VA 22015.

[PIP]  Fred Piper, ``Stream Ciphers,'' in Proceedings

of the Workshop on Cryptography at Burg Feuerstein, Germany, March

29-April 2, 1982, Berlin, Heidelberg, New York: Springer-Verlag,

1983.

[PER]  Tekla S. Perry and Paul Wallich, ``Can Computer

Crime be Stopped?,''  IEEE Spectrum, May, 1984, pp.34-49.  New

York, NY: The Institute of Electrical and Electronic Engineers, Inc.

[RET]  H. Retkin, ``Multi-Level Knapsack Encryption,''

in International Conference on Secure Communication Systems,

22-23 February 1984, London, New York: Institute of Electrical

Engineers, 1984. TK5102.5.I519.1984

[SAT]  J. Sattler and C. P. Schnorr, ``Ein Effizienzvergleich

Der Faktorisierungsverfahren von Morrison-Brillhart und Schroeppel,''

in Proceedings of the Workshop on Cryptography at Burg Feuerstein,

Germany, March 29-April 2, 1982, Berlin, Heidelberg, New York:

Springer-Verlag, 1983.

[SCH]  C. P. Schnorr, ``Is the RSA Scheme Safe?'' in

Proceedings of the Workshop on Cryptography at Burg Feuerstein,

Germany, March 29-April 2, 1982, Berlin, Heidelberg, New York:

Springer-Verlag, 1983.

[SHA]  C. E. Shannon, ``Communication Theory of Secrecy

Systems,'' Bell System Technical Journal, Volume 28, 1949.

[STO]  James A. Storer, Data Compression Methods

and Theory.  Rockville, MD:  Computer Science Press, 1988. QA76.9.D33S76

[TIL]  Henk C. A. van Tilborg, An Introduction to

Cryptology, Boston, Dordrecht, Lancaster: Kluwer Academic Publishers,

1988.  Z103.T54.1987

[RET]  H. Retkin, ``Multi-Level Knapsack Encryption,''

in International Conference on Secure Communication Systems,

22-23 February 1984, London, New York: Institute of Electrical

Engineers, 1984. TK5102.5.I519.1984

[ULT]  Ultron Labs Corporation, ``Crypto Module Processes

TOP SECRET Data,'' EE Product News, October 1988, p. 16, Overland

Park, KS: Intertec Publishing.

[VAN]  J. Vandewalle, R. Govaerts, W. De Becker, M. Decroos,

& G. Speybrouk, ``RSA-Based Implementation of Public Key Cryptographic

Protection in Office Systems'' in Proceedings of 1986 International

Carnahan Conference on Security Technology:  Electronic Crime Countermeasures,

August 12-14, 1986.  QA76.9.A25.C41.1986

APPENDIX A.  MPJ in Pascal

{$R-}    {Range checking off}

{$S-}    {Stack checking off}

{$I+}    {I/O checking on}

{$N-}    {Don't use 80x87 numeric coprocessor}

program cryptmpj;

{ Encryption designed to exceed DES in security value and be implementable

on

an IBM PC compatible machine. }

Uses Crt, Dos;  { Include screen handling & MS DOS interface functions.

}

const maxinbuffer = 16;

ap = #39;                            { Apostrophe for write

statements. }

type blocks = array[0..15] of byte;             { 16 byte (128 bit)

blocks. }

stype = array[0..9,0..15,0..255] of byte;  { Holds substitution

boxes. }

ptrstype = ^stype;  { Dynamic variable required to get beyond

64K limit. }

string80 = string[80];                     { Long character strings.

}

string15 = string[15];                     { Short character

strings. }

var initial,                  { Initialiazation vector. }

feedback,                 { Chaining feedback array. }

key,                      { Encryption/decryption key. }

buffer, outbuf:  blocks;  { File read/write buffers. }

s,                        { Substitution boxes. }

si: ptrstype;             { Inverse substitution boxes. }

bytesdone,                { Number of bytes done. }

number: longint;          { Used to read in key & initialization

vector. }

que,                      { Position in file specification que.

}

i, j, k,                  { Iteration & array indexes. }

actualread: integer;      { Actual number of bytes read in from

file. }

inputfile: file;          { File to encrypt or decrypt in place.

}

keyfile: text;            { External key file. }

intkeyfile: file of stype; {Holds s and si. }

keyname,                  { Name of external key file. }

intkeyname,               { Name of internal key file. }

path,                     { Used in parsing file specification.

}

temp: string[255];        { Used in determining encrypt/decrypt

option. }

inputfilename,            { MS-DOS name of inputfile. }

filespec: string80;       { File specification that may include

wild cards}

fileque: array[0..15] of string80; { Names of multiple file specifications}

name: string15;           { Name of next file to process. }

decryption,               { True iff decryption is desired. }

filefound: boolean;       { At least one file was found to process.}

ch: char;                 { Character input. }

search: searchrec;        { Used in finding matches to wild cards.

}

procedure permute(x: blocks; var y: blocks);

{ This procedure is designed to make each bit of the output dependent

on as

many bytes of the input as possible, especially after repeated application.

Each output byte takes its least significant bit from the corresponding

input byte.  The next higher bit comes from the corresponding bit

of the

input byte to the left.  This is done until all bits of the output

byte

are filled.  Where there is no byte to the left, the byte at the

far right

is used. }

begin

y[0] := (x[0] and 1) or (x[1] and 2) or (x[2] and 4) or

(x[3] and 8) or (x[4] and 16) or (x[5] and 32) or

(x[6] and 64) or (x[7] and 128);

y[1] := (x[1] and 1) or (x[2] and 2) or (x[3] and 4) or

(x[4] and 8) or (x[5] and 16) or (x[6] and 32) or

(x[7] and 64) or (x[8] and 128);

y[2] := (x[2] and 1) or (x[3] and 2) or (x[4] and 4) or

(x[5] and 8) or (x[6] and 16) or (x[7] and 32) or

(x[8] and 64) or (x[9] and 128);

y[3] := (x[3] and 1) or (x[4] and 2) or (x[5] and 4) or

(x[6] and 8) or (x[7] and 16) or (x[8] and 32) or

(x[9] and 64) or (x[10] and 128);

y[4] := (x[4] and 1) or (x[5] and 2) or (x[6] and 4) or

(x[7] and 8) or (x[8] and 16) or (x[9] and 32) or

(x[10] and 64) or (x[11] and 128);

y[5] := (x[5] and 1) or (x[6] and 2) or (x[7] and 4) or

(x[8] and 8) or (x[9] and 16) or (x[10] and 32) or

(x[11] and 64) or (x[12] and 128);

y[6] := (x[6] and 1) or (x[7] and 2) or (x[8] and 4) or

(x[9] and 8) or (x[10] and 16) or (x[11] and 32) or

(x[12] and 64) or (x[13] and 128);

y[7] := (x[7] and 1) or (x[8] and 2) or (x[9] and 4) or

(x[10] and 8) or (x[11] and 16) or (x[12] and 32) or

(x[13] and 64) or (x[14] and 128);

y[8] := (x[8] and 1) or (x[9] and 2) or (x[10] and 4) or

(x[11] and 8) or (x[12] and 16) or (x[13] and 32) or

(x[14] and 64) or (x[15] and 128);

y[9] := (x[9] and 1) or (x[10] and 2) or (x[11] and 4) or

(x[12] and 8) or (x[13] and 16) or (x[14] and 32) or

(x[15] and 64) or (x[0] and 128);

y[10] := (x[10] and 1) or (x[11] and 2) or (x[12] and 4) or

(x[13] and 8) or (x[14] and 16) or (x[15] and 32) or

(x[0] and 64) or (x[1] and 128);

y[11] := (x[11] and 1) or (x[12] and 2) or (x[13] and 4) or

(x[14] and 8) or (x[15] and 16) or (x[0] and 32) or

(x[1] and 64) or (x[2] and 128);

y[12] := (x[12] and 1) or (x[13] and 2) or (x[14] and 4) or

(x[15] and 8) or (x[0] and 16) or (x[1] and 32) or

(x[2] and 64) or (x[3] and 128);

y[13] := (x[13] and 1) or (x[14] and 2) or (x[15] and 4) or

(x[0] and 8) or (x[1] and 16) or (x[2] and 32) or

(x[3] and 64) or (x[4] and 128);

y[14] := (x[14] and 1) or (x[15] and 2) or (x[0] and 4) or

(x[1] and 8) or (x[2] and 16) or (x[3] and 32) or

(x[4] and 64) or (x[5] and 128);

y[15] := (x[15] and 1) or (x[0] and 2) or (x[1] and 4) or

(x[2] and 8) or (x[3] and 16) or (x[4] and 32) or

(x[5] and 64) or (x[6] and 128);

end;

procedure ipermute(x: blocks; var y: blocks);

{ This is the inverse of the procedure permute. }

begin

y[0] := (x[0] and 1) or (x[15] and 2) or (x[14] and 4) or

(x[13] and 8) or (x[12] and 16) or (x[11] and 32) or

(x[10] and 64) or (x[9] and 128);

y[1] := (x[1] and 1) or (x[0] and 2) or (x[15] and 4) or

(x[14] and 8) or (x[13] and 16) or (x[12] and 32) or

(x[11] and 64) or (x[10] and 128);

y[2] := (x[2] and 1) or (x[1] and 2) or (x[0] and 4) or

(x[15] and 8) or (x[14] and 16) or (x[13] and 32) or

(x[12] and 64) or (x[11] and 128);

y[3] := (x[3] and 1) or (x[2] and 2) or (x[1] and 4) or

(x[0] and 8) or (x[15] and 16) or (x[14] and 32) or

(x[13] and 64) or (x[12] and 128);

y[4] := (x[4] and 1) or (x[3] and 2) or (x[2] and 4) or

(x[1] and 8) or (x[0] and 16) or (x[15] and 32) or

(x[14] and 64) or (x[13] and 128);

y[5] := (x[5] and 1) or (x[4] and 2) or (x[3] and 4) or

(x[2] and 8) or (x[1] and 16) or (x[0] and 32) or

(x[15] and 64) or (x[14] and 128);

y[6] := (x[6] and 1) or (x[5] and 2) or (x[4] and 4) or

(x[3] and 8) or (x[2] and 16) or (x[1] and 32) or

(x[0] and 64) or (x[15] and 128);

y[7] := (x[7] and 1) or (x[6] and 2) or (x[5] and 4) or

(x[4] and 8) or (x[3] and 16) or (x[2] and 32) or

(x[1] and 64) or (x[0] and 128);

y[8] := (x[8] and 1) or (x[7] and 2) or (x[6] and 4) or

(x[5] and 8) or (x[4] and 16) or (x[3] and 32) or

(x[2] and 64) or (x[1] and 128);

y[9] := (x[9] and 1) or (x[8] and 2) or (x[7] and 4) or

(x[6] and 8) or (x[5] and 16) or (x[4] and 32) or

(x[3] and 64) or (x[2] and 128);

y[10] := (x[10] and 1) or (x[9] and 2) or (x[8] and 4) or

(x[7] and 8) or (x[6] and 16) or (x[5] and 32) or

(x[4] and 64) or (x[3] and 128);

y[11] := (x[11] and 1) or (x[10] and 2) or (x[9] and 4) or

(x[8] and 8) or (x[7] and 16) or (x[6] and 32) or

(x[5] and 64) or (x[4] and 128);

y[12] := (x[12] and 1) or (x[11] and 2) or (x[10] and 4) or

(x[9] and 8) or (x[8] and 16) or (x[7] and 32) or

(x[6] and 64) or (x[5] and 128);

y[13] := (x[13] and 1) or (x[12] and 2) or (x[11] and 4) or

(x[10] and 8) or (x[9] and 16) or (x[8] and 32) or

(x[7] and 64) or (x[6] and 128);

y[14] := (x[14] and 1) or (x[13] and 2) or (x[12] and 4) or

(x[11] and 8) or (x[10] and 16) or (x[9] and 32) or

(x[8] and 64) or (x[7] and 128);

y[15] := (x[15] and 1) or (x[14] and 2) or (x[13] and 4) or

(x[12] and 8) or (x[11] and 16) or (x[10] and 32) or

(x[9] and 64) or (x[8] and 128);

end;

procedure makesbox(key: blocks);

{ This procedure generates internal keys by filling the substitution

box array

s^ based on the external key given as input. }

var i, j, k: integer;

procedure makeonebox(i, j: integer; key: blocks);

var pos, m, n, p, startbit, bitmask, startbyte, keybyte: word;

empty: array[0..255] of boolean;

begin

for m := 0 to 255 do    { The empty array is used to make sure

that }

empty[m] := true;     { each byte of the array is filled only

once. }

startbit := 1;

startbyte := 0;

for n := 255 downto 128 do  { n counts the number of bytes left

to fill }

begin

keybyte := startbyte;

bitmask := startbit;

m := 0;

for p := 0 to 7 do   { m is obtained by bit selection on

the key }

begin

m := m or (key[keybyte] and bitmask);

bitmask := bitmask shl 1;

if bitmask > 128 then

begin

bitmask := 1;

inc(keybyte);

if keybyte > 15 then keybyte := 0;

end;

end;

pos := (n * m) div 255;  { pos is the position among the

UNFILLED }

m := 0;                  { components of the s^ array that

the    }

p := 0;                  { number n should be placed.  }

while m < pos do

begin

inc(p);

if empty[p] then inc(m);

end;

while not empty[p] do inc(p);

s^[i, j, p] := n;

empty[p] := false;

startbit := startbit shl 1;  { The starting position of

the bit  }

if startbit > 128 then       { selection for the key is

rotated  }

begin                      { left one bit for the next

n.      }

startbit := 1;

inc(startbyte);

if startbyte > 15 then startbyte := 0

end;

end;

startbyte := 0;

startbit := 1;

for n := 127 downto 1 do         { This half of the algorithm

is the   }

begin                          { same as the upper half, except

that }

keybyte := startbyte;        { only 7 bits are selected

for m.     }

bitmask := startbit;

m := 0;

for p := 0 to 6 do

begin

m := m or (key[keybyte] and bitmask);

bitmask := bitmask shl 1;

if bitmask > 64 then

begin

bitmask := 1;

keybyte := keybyte + 3;

if keybyte > 15 then keybyte := keybyte - 16;

end;

end;

pos := (n * m) div 127;

m := 0;

p := 0;

while m < pos do

begin

inc(p);

if empty[p] then inc(m);

end;

while not empty[p] do inc(p);

s^[i, j, p] := n;

empty[p] := false;

startbit := startbit shl 1;

if startbit > 64 then

begin

startbit := 1;

inc(startbyte);

if startbyte > 15 then startbyte := 0

end;

end;

p := 0;

while not empty[p] do

inc(p);

s^[i,j,p] := 0;

end;

begin

new(s);

for i := 0 to 9 do

for j := 0 to 15 do

begin

write(#13,'Filling substitution boxes for round ',i+1,'

of 10, byte ',

j+1,' of 16.  ');

makeonebox(i, j, key);

permute(key, key);            { Shuffle key bit positions.        }

for k := 0 to 15 do           { Run key through last s-box

before }

key[k] := s^[i, j, key[k]]; { making the next s-box.            }

end;

writeln;

end;

procedure makesi;

{ This procedure fills the inverse substitution box array si^.  It

is not

necessary to call this procedure unless the decryption mode is used.  }

var i, j, k: integer;

begin

new(si);

for i := 0 to 9 do

for j := 0 to 15 do

for k := 0 to 255 do

si^[i, j, s^[i, j, k]] := k;

end;

{$I CRYPTINC.PAS}   { Include additional file handling & user interface.

}

procedure substitute(round: integer; x: blocks; var y: blocks);

var i: integer;

begin

for i := 0 to 15 do

y[i] := s^[round,i,x[i]];

end;

procedure isubst(round: integer; x: blocks; var y: blocks);

var i: integer;

begin

for i := 0 to 15 do

y[i] := si^[round,i,x[i]];

end;

procedure encrypt(x: blocks; var y: blocks);

{ Encrypt a block of 16 bytes. }

var z: blocks;

begin

substitute(0, x, y);

permute(y, z);

substitute(1, z, y);

permute(y, z);

substitute(2, z, y);

permute(y, z);

substitute(3, z, y);

permute(y, z);

substitute(4, z, y);

permute(y, z);

substitute(5, z, y);

permute(y, z);

substitute(6, z, y);

permute(y, z);

substitute(7, z, y);

permute(y, z);

substitute(8, z, y);

permute(y, z);

substitute(9, z, y);

end;

procedure decrypt(x: blocks; var y: blocks);

{ Decrypt a block of 16 bytes. }

var z: blocks;

begin

isubst(9, x, y);

ipermute(y, z);

isubst(8, z, y);

ipermute(y, z);

isubst(7, z, y);

ipermute(y, z);

isubst(6, z, y);

ipermute(y, z);

isubst(5, z, y);

ipermute(y, z);

isubst(4, z, y);

ipermute(y, z);

isubst(3, z, y);

ipermute(y, z);

isubst(2, z, y);

ipermute(y, z);

isubst(1, z, y);

ipermute(y, z);

isubst(0, z, y);

end;

begin

startup;  { Initialize files, call makesbox (and makesi if required).

}

que := 0;

while (que < 16) and (fileque[que] <> '') do

begin

filespec := fileque[que];

inputfilename := filespec;

path := '';

repeat

i := pos('\',inputfilename);

if i > 0 then

begin

path := path + copy(inputfilename, 1, i);

inputfilename := copy(inputfilename, i+1, length(inputfilename));

end;

until i = 0;

findfirst(filespec, $22, search);

while doserror = 0 do

begin

inputfilename := path + search.name;

assign(inputfile, inputfilename);

reset(inputfile,1);

filefound := true;

bytesdone := 0;

for i := 0 to 15 do

feedback[i] := initial[i];

if decryption then

begin

writeln('Decrypting ',inputfilename);

while not eof(inputfile) do

begin

blockread(inputfile,buffer,maxinbuffer,actualread);

encrypt(feedback,outbuf);

{ Note that decrypt is not ever called when using

block

chaining with ciphertext feedback.  It would be

called

if the electronic codebook mode were used, instead.

}

for i := 0 to 15 do

begin

outbuf[i] := outbuf[i] xor buffer[i];

feedback[i] := buffer[i]

end;

seek(inputfile, filepos(inputfile) - actualread);

blockwrite(inputfile, outbuf, actualread);

bytesdone := bytesdone + actualread;

write(#13, bytesdone:9,' bytes decrypted. ');

end;

writeln(#13, bytesdone:9,' bytes decrypted.  ');

end

else

begin

writeln('Encrypting ',inputfilename);

while not eof(inputfile) do

begin

blockread(inputfile,buffer,maxinbuffer,actualread);

encrypt(feedback,outbuf);

for i := 0 to 15 do

begin

outbuf[i] := outbuf[i] xor buffer[i];

feedback[i] := outbuf[i]

end;

seek(inputfile, filepos(inputfile) - actualread);

blockwrite(inputfile, outbuf, actualread);

bytesdone := bytesdone + actualread;

write(#13, bytesdone:9,' bytes encrypted.  ');

end;

writeln(#13, bytesdone:9,' bytes encrypted.  ');

end;

close(inputfile);

findnext(search);

end;

inc(que);

end;

if not filefound then writeln('No matching files found.');

writeln('Done.');

end.

APPENDIX B.  Linguistic Data Compression Programs

program count;

{Program to count the probability of occurances of words in text.}

uses dos, crt;

const maxbuf = 4096;

maxstr = 15;

ap = #39;

type wd = string[maxstr];

ptr = ^entry;

yarn = string[80];

entry = record

wrd:  wd;        {Linguistic word}

count: longint;  {How many times this word was found}

next: ptr;       {Next record in sequence}

end;

var outfile,             {File with output report}

infile: text;        {File with input text to be analyzed}

bottom,              {Last entry record}

newbuf,              {New entry record}

prev: ptr;           {Previous entry record}

buffer,              {Current entry record}

top: array[1..255] of ptr;  {First entry record}

words: wd;           {Word under construction}

temp,

path,                    {Path of input file}

filemask,                {File name mask}

filespec,                {Path + name (including wild cards)}

inname,                  {Name of input file}

outname: yarn;           {Name of output file}

ch: char;

i, j, cmdparameter, topindex: integer;

cnt: longint;

filefound: boolean;

inbuf, outbuf: array[1..maxbuf] of byte;

s: searchrec;           {Record fill: array[1..21] of byte;attr:byte;time,

size:longint;name: string[12]}

function exist(filename: wd): boolean;

{Returns TRUE iff the file exists.}

var f: file;

begin {exist}

assign(f,filename);

{$I-}

reset(f);

{$I+}

if IOresult = 0 then

begin

exist := true;

close(f)

end

else

exist := false;

end;  {exist}

function punct(ch: char): boolean;

{TRUE iff ch is not a letter or chr(255).}

var c:  integer;

begin

c := ord(ch);

if ((c > 10) and (c <= 64)) or ((c >= 91) and (c <= 96))

or ((c >= 123) and (c <= 254)) or ((c > 0) and (c < 10)) then

punct := true

else

punct := false;

end;

function letter(ch: char): boolean;

{TRUE iff ch is a letter A-Z or a-z.}

var c: integer;

begin

c := ord(ch);

if ((c >= 65) and (c <= 90)) or ((c >= 97) and (c <= 122)) then

letter := true

else

letter := false;

end;

procedure parse(var words: wd; var ch: char);

{Extracts a word or item of punctuation from input file.}

begin  {parse}

words := '';

while not (letter(ch) or punct(ch) or eof(infile)) do

begin

read(infile,ch);

end;

if punct(ch) then

begin

if ord(ch) = 13 then

begin

words := chr(255)+chr(255);

if not eof(infile) then read(infile, ch);

end

else

if ch = ' ' then

begin

words := ch;

while (not eof(infile)) and (ch = ' ') and (length(words)

< maxstr) do

begin

read(infile,ch);

if ch = ' ' then words := words + ch;

end;

if (length(words) = maxstr) and (not eof(infile)) then

read(infile,ch);

end

else

begin

words := ch; {return one punctuation character}

if not eof(infile) then read(infile, ch);

end;

end

else

begin

if letter(ch) then

begin

words := ch;

while (not eof(infile)) and letter(ch) and (length(words)

< maxstr) do

begin

read(infile,ch);

if letter(ch) then words := words + ch;

end;

if (length(words) = maxstr) and (not eof(infile)) then

read(infile,ch);

end;

end;

end;  {parse}

procedure update(words: wd);

{Increments the count associated with a word.}

begin

topindex := ord(words[1]);

buffer[topindex] := top[topindex];

{Find record that matches word, if it exists.}

while (buffer[topindex]^.wrd < words) and (buffer[topindex]^.next

<> nil) do

begin

prev := buffer[topindex];

buffer[topindex] := buffer[topindex]^.next

end;

if buffer[topindex]^.wrd = words then

{Increment count on word that already exists}

inc(buffer[topindex]^.count)

else

begin  {Add word to middle of list with count of one}

new(newbuf);

newbuf^.wrd := words;

newbuf^.count := 1;

newbuf^.next := prev^.next;

prev^.next := newbuf;

end;

end;

procedure report;

{Produces a report file.}

var total: longint;

begin

assign(outfile, paramstr(1));

rewrite(outfile);

total := 0;

for topindex := 1 to 255 do

begin

buffer[topindex] := top[topindex];

while (buffer[topindex]^.next <> nil) do

begin

buffer[topindex] := buffer[topindex]^.next;

if buffer[topindex]^.count > 0 then

begin

writeln(outfile,buffer[topindex]^.wrd);

writeln(outfile,buffer[topindex]^.count);

total := total + buffer[topindex]^.count;

end;

end;

end;

writeln(outfile, chr(255));

writeln(outfile, '1');

writeln(outfile, 'Total count:  ',total);

close(outfile);

end; {report}

begin

clrscr;

writeln('  This program counts words in a set    This program may

be copied freely ');

writeln('  of input files to determine their     for educational

use or to try it  ');

writeln('  frequency of occurance.  This data    out.  If you like

it, donations   ');

writeln('  is then stored in a file for use by   will be accepted

by the author.   ');

writeln('  MAKETREE, which constructs a code     This program is

believed to be    ');

writeln('  tree with the Huffman algorithm.      reliable, but it

is the user',ap,'s    ');

writeln('  The code tree is used by SQUEEZE.     responsibility

to determine its   ');

writeln('                                        fitness for use.  No

liability    ');

writeln('  Mike Johnson                          will be assumed

by the author.    ');

writeln('  P. O. Box 1151                        Copyright (C) 1988

Mike Johnson.  ');

writeln('  Longmont, CO 80502-1151               All rights reserved.              ');

if paramcount < 2 then

begin

writeln;

writeln('Syntax: COUNT outputfilename inputfilename[s]');

writeln('        input file names may include * and ?');

writeln('        Input files are assumed to be ASCII text.');

writeln;

writeln('If a previous COUNT ouput file named LANGUAGE.OUT exists,

then new counts');

writeln('will be added to the counts in LANGUAGE.OUT.');

writeln('Maximum effectiveness will be obtained if the nature

of the files COUNTed');

writeln('is similar to the nature of the files to be compressed

with SQUEEZE.');

end

else

begin

filefound := false;

new(bottom);

bottom^.wrd := chr(255);

bottom^.count := 0;

bottom^.next := nil;

for topindex := 1 to 255 do

begin

new(top[topindex]);

top[topindex]^.wrd := chr(1);

top[topindex]^.count := 0;

top[topindex]^.next := bottom;

buffer[topindex] := top[topindex];

end;

ch := chr(0);

words := '                              ';

if exist('LANGUAGE.OUT') then

begin

assign(infile,'LANGUAGE.OUT');

reset(infile);

writeln('Reading in existing LANGUAGE.OUT.');

while (not eof(infile)) and (words <> chr(255)) do

begin

readln(infile,words);

if words <> chr(255) then

begin

topindex := ord(words[1]);

readln(infile,cnt);

new(newbuf);

newbuf^.wrd := words;

newbuf^.count := cnt;

newbuf^.next := bottom;

buffer[topindex]^.next := newbuf;

buffer[topindex] := newbuf;

write(#13,words,'            ');

end;

end;

close(infile);

writeln;

end;

{Initialize input files.}

for cmdparameter := 2 to paramcount do

begin

filespec := paramstr(cmdparameter);

filemask := filespec;

path := '';

repeat

i := pos('\',filemask);

if i > 0 then

begin

path := path + copy(filemask, 1, i);

filemask := copy(filemask, i+1, length(filemask));

end;

until i = 0;

findfirst(filespec, readonly + hidden + archive, s);

while doserror = 0 do

begin

filefound := true;

inname := path + s.name;

assign(infile, inname);

reset(infile);

words := '                              ';

writeln('>>>>>>>>>>>>>>>>>>>>> Processing ',inname);

while (not eof(infile)) and (words <> '') and (not keypressed)

do

begin

parse(words,ch);     {Find word or punctuation}

if words = chr(255)+chr(255) then

writeln

else

write(words);

if length(words) > 0 then

update(words);     {Increment word counter}

end;

close(infile);

findnext(s);

end;

end;

if not filefound then writeln('No matching files found.');

report;

end;

end.

{$R+}

{$M 3072,4096,655360}

program maketree;

uses crt;

const maxstr = 15;

ap = #39;

type wd = string[maxstr];

yarn = string[80];

mem = ^mementry;

entry = record

wrd:  wd;        {Which linguistic word in input file}

memory: mem;     {Corresponding mementry.}

end;

mementry = record

count: longint; {Word count.}

onezero: byte;  {Is this record pointed to by 1

or 0?}

root,           {Position of record nearer root.}

next:  mem;     {Position of next record.}

end;

var temp: file of entry; {Temporary file}

infile: text;        {File with input text to be analyzed}

buffer,              {Current entry record}

newbuf: entry;       {New entry record}

mtop,

mbottom,

mbuffer,

mnewbuf,

mprev: mem;

inname,              {Name of input file}

outname,             {Name of output file}

words: wd;           {Word under construction}

ch: char;

cnt1,

cnt2,

cnt:  longint;

i, j: integer;

tempname: yarn;

function exist(filename: wd): boolean;

{Returns TRUE iff the file exists.}

var f: file;

begin {exist}

assign(f,filename);

{$I-}

reset(f);

{$I+}

if IOresult = 0 then

begin

exist := true;

close(f)

end

else

exist := false;

end;  {exist}

procedure fanno;

begin

cnt1 := 0;

repeat

{Find smallest two records.}

mprev := mtop^.next;

mbuffer := mprev^.next;

{Combine lowest 2 probabilities of counts.}

mtop^.next := mbuffer^.next;

new(mnewbuf);

mnewbuf^.count := mbuffer^.count + mprev^.count;

mnewbuf^.onezero := 2;

mnewbuf^.root := nil;

mprev^.root := mnewbuf;

mprev^.onezero := 1;

mbuffer^.root := mnewbuf;

mbuffer^.onezero := 0;

{Place combined entry in proper place in chain.}

mbuffer := mtop;

repeat

mprev := mbuffer;

mbuffer := mbuffer^.next;

until (mnewbuf^.count <= mbuffer^.count);

mprev^.next := mnewbuf;

mnewbuf^.next := mbuffer;

{Check for completion.}

inc(cnt1);

write(#13,cnt1,' nodes combined.  ');

until (cnt1 >= cnt2);

end;  {fanno}

procedure placeit;

begin

write(#13,cnt:11,' ',words,'          ');

mbuffer := mtop;

repeat

mprev := mbuffer;

mbuffer := mbuffer^.next;

until mbuffer^.count >= cnt;

new(mnewbuf);

newbuf.wrd := words;

newbuf.memory := mnewbuf;

write(temp,newbuf);

mnewbuf^.count := cnt;

mnewbuf^.onezero := 2;

mnewbuf^.root := nil;

mnewbuf^.next := mbuffer;

mprev^.next := mnewbuf;

end;

procedure filegen;

var cod: file of byte;

ascnum: yarn;

c: char;

b, d: byte;

i, shifter: integer;

begin

assign(cod,outname);

rewrite(cod);

reset(temp);

repeat

read(temp,buffer);

mbuffer := buffer.memory;

if buffer.wrd <> '' then

begin

for i := 0 to length(buffer.wrd) do

begin

b := ord(buffer.wrd[i]);

write(cod,b);

end;

ascnum := '';

while (mbuffer^.root <> nil) do

begin

if mbuffer^.onezero = 0 then

ascnum := ascnum + '0'

else

ascnum := ascnum + '1';

mbuffer := mbuffer^.root;

end;

d := ord(ascnum[0]);

write(cod,d);

shifter := 1;

b := 0;

for i := d downto 1 do

begin

if ascnum[i] = '1' then b := b + shifter;

shifter := shifter shl 1;

if shifter >= 256 then

begin

write(cod,b);

b := 0;

shifter := 1

end;

end;

if shifter > 1 then write(cod,b);

write(#13,buffer.wrd:maxstr,' = ',ascnum,'                           ');

end;

until eof(temp);

close(cod);

close(temp);

erase(temp);

end;

begin

clrscr;

writeln('  MAKETREE takes the word count         This program may

be copied freely ');

writeln('  data created by COUNT and creates     for educational

use or to try it  ');

writeln('  a code tree using the Huffman         out.  If you like

it, donations   ');

writeln('  algorithm.  The resulting code tree   will be accepted

by the author.   ');

writeln('  is used by SQUEEZE when it            This program is

believed to be    ');

writeln('  compresses ASCII text files.          reliable, but it

is the user',ap,'s    ');

writeln('                                        responsibility

to determine its   ');

writeln('                                        fitness for use.  No

liability    ');

writeln('  Mike Johnson                          will be assumed

by the author.    ');

writeln('  P. O. Box 1151                        Copyright (C) 1988

Mike Johnson.  ');

writeln('  Longmont, CO 80502-1151               All rights reserved.              ');

if paramcount < 2 then

begin

writeln('Syntax: MAKETREE infile outfile [tempfile]');

writeln('                 infile is input file name (generated

by COUNT)');

writeln('                 outfile is output file name to be

used by SQUEEZE');

writeln('                 tempfile is filename to use for temporary

file');

writeln('The temporary file is best put on a RAM disk in extended

or expanded memory,');

writeln('but a hard disk or even a floppy will do if that is

the fastest you have.');

writeln('Don',ap,'t use a RAM disk in conventional (lower 640K)

memory, as this program');

writeln('already uses most of that in generating the coding

tree.');

end

else

begin

inname := paramstr(1);

outname := paramstr(2);

if exist(inname) then

begin

assign(infile,inname);

reset(infile);

end

else

begin

writeln('Unable to open ',inname);

exit

end;

if paramcount > 2 then

tempname := paramstr(3)

else

tempname := 'ZYXWVUTS.$$$';

assign(temp,tempname);

{$I- }

rewrite(temp);

{$I+ }

if ioresult <> 0 then

begin

writeln('I/O error attempting to open temporary file ',tempname);

halt;

end;

new(mtop);

new(mbottom);

mtop^.count := -1;

mtop^.onezero := 2;

mtop^.root := nil;

mtop^.next := mbottom;

mbottom^.count := 2147483647;

mbottom^.onezero := 2;

mbottom^.root := nil;

mbottom^.next := nil;

cnt2 := 2;

writeln('Reading in and sorting word frequency data.');

repeat

readln(infile,words);

readln(infile,cnt);

placeit;

inc(cnt2);

until (words = chr(255)) or eof(infile);

close(infile);

words := chr(255) + chr(255);

placeit;

words := words + chr(255);

placeit;

words := words + chr(255);

placeit;

writeln(#13,cnt2,' nodes to add to Huffman coding tree.                  ');

fanno;

writeln;

writeln('Writing output file.');

filegen;

writeln(#13,'Done.                                          ');

end;

end.

{$R+}

{$M 2048,4096,655360}

program squash;

uses dos, crt;

const maxbuf = 4096;

maxstr = 15;

ap = #39;

type wd = string[maxstr];

yarn = string[80];

ptr = ^entry;

entry = record

wrd:  wd;        {Linguistic word}

count:  byte;    {How many bits in code}

code: longint;   {Code (right justified in 4 byte integer)}

next: ptr;       {Pointer to next entry}

end;

var outfile,                 {File with output report}

infile: file;            {File with input text to be analyzed}

temp,

path,                    {Path of input file}

filemask,                {File name mask}

filespec,                {Path + name (including wild cards)}

inname,                  {Name of input file}

outname: yarn;           {Name of output file}

words: wd;               {Word under construction}

ch: char;

top,

bottom: array[1..32] of ptr;

last,

newbuf,

newline,

endfile,

buf: ptr;

mask,

masked,

place,

long,

cd,

cnt:  longint;

infilepos,              {Current position in input file buffer}

inbitpos,               {Current bit mask in input file buffer}

inbytes,                {Actual number of bytes read}

outfilepos,             {Current position in output file buffer}

outbitpos,              {Current bit mask in output file buffer}

outbytes:  word;        {Actual number of bytes to write}

cmdparameter,           {Command line parameter number}

start,                  {Starting point for command line file

name scan}

strlen,

bite,

b, i, j, k, m: integer;

squish, filefound, done: boolean;

outbite:  byte;

inbuf,

outbuf: array[1..maxbuf] of byte;

s: searchrec;           {Record fill: array[1..21] of byte;attr:byte;time,

size:longint;name: string[12]}

function exist(filename: yarn): boolean;

{Returns TRUE iff the file exists.}

var f: file;

begin {exist}

assign(f,filename);

{$I-}

reset(f);

{$I+}

if IOresult = 0 then

begin

exist := true;

close(f)

end

else

exist := false;

end;  {exist}

procedure getbit(var b: integer);

begin

if infilepos = 0 then

begin

if eof(infile) then

done := true

else

begin

blockread(infile, inbuf, maxbuf, inbytes);

infilepos := 1;

inbitpos := 1;

end;

end;

if not done then

begin

b := inbuf[infilepos] and inbitpos;

if b > 0 then b := 1;

inbitpos := inbitpos shl 1;

if inbitpos > 128 then

begin

inbitpos := 1;

inc(infilepos);

if infilepos > inbytes then infilepos := 0;

end;

end;

end;

procedure getbite(var b: integer);

begin

if infilepos = 0 then

begin

if eof(infile) then

begin

done := true;

b := 0

end

else

begin

blockread(infile, inbuf, maxbuf, inbytes);

infilepos := 1;

end;

end;

if not done then

begin

if inbitpos > 1 then

begin

inc(infilepos);

writeln('Bit sync error in getbite!')

end;

b := inbuf[infilepos];

inbitpos := 1;

inc(infilepos);

if infilepos > inbytes then infilepos := 0;

end;

end;

procedure putbit(b: integer);

begin

if (b > 1) or (b < 0) then writeln('Bad data passed to putbit.');

outbite := outbite or (outbitpos * b);

outbitpos := outbitpos shl 1;

if outbitpos > 128 then

begin

inc(outfilepos);

outbuf[outfilepos] := outbite;

outbite := 0;

outbitpos := 1;

if outfilepos >= maxbuf then

begin

blockwrite(outfile, outbuf, maxbuf, outbytes);

if outbytes < maxbuf then

begin

writeln('Disk full error.');

done := true;

end;

outfilepos := 0;

end;

end;

end;

procedure putbite(b: integer);

begin

if (b > 255) or (b < 0) then writeln('Bad data passed to putbite.');

outbite := b;

inc(outfilepos);

outbuf[outfilepos] := outbite;

outbite := 0;

outbitpos := 1;

if outfilepos >= maxbuf then

begin

blockwrite(outfile, outbuf, maxbuf, outbytes);

if outbytes < maxbuf then

begin

writeln('Disk full error.');

done := true;

end;

outfilepos := 0;

end;

end;

procedure flushbit;

begin

if outbitpos > 1 then

begin

inc(outfilepos);

outbuf[outfilepos] := outbite;

end;

blockwrite(outfile, outbuf, outfilepos, outbytes);

outbite := 0;

outbitpos := 1;

outfilepos := 0;

end;

function punct(ch: char): boolean;

{TRUE iff ch is not a letter or chr(255).}

var c:  integer;

begin

c := ord(ch);

if ((c > 10) and (c <= 64)) or ((c >= 91) and (c <= 96))

or ((c >= 123) and (c <= 254)) or ((c > 0) and (c < 10)) then

punct := true

else

punct := false;

end;

function letter(ch: char): boolean;

{TRUE iff ch is a letter A-Z or a-z.}

var c: integer;

begin

c := ord(ch);

if ((c >= 65) and (c <= 90)) or ((c >= 97) and (c <= 122)) then

letter := true

else

letter := false;

end;

procedure parse(var words: wd; var ch: char);

{Extracts a word or item of punctuation from input file.}

var byt: integer;

begin  {parse}

words := '';

while not (letter(ch) or punct(ch) or done) do

begin

getbite(byt);

ch := chr(byt);

end;

if punct(ch) then

begin

if ord(ch) = 13 then

begin

words := chr(255)+chr(255);

if not done then begin getbite(byt); ch := chr(byt) end;

end

else

if ch = ' ' then

begin

words := ch;

while (not done) and (ch = ' ') and (length(words) < maxstr) do

begin

getbite(byt);

ch := chr(byt);

if ch = ' ' then words := words + ch;

end;

if (length(words) = maxstr) and (not done) then

begin

getbite(byt);

ch := chr(byt)

end;

end

else

begin

words := ch; {return one punctuation character}

if not done then begin getbite(byt); ch := chr(byt) end;

end;

end

else

begin

if letter(ch) then

begin

words := ch;

while (not done) and letter(ch) and (length(words) < maxstr)

do

begin

getbite(byt);

ch := chr(byt);

if letter(ch) then words := words + ch;

end;

if (length(words) = maxstr) and (not done) then

begin

getbite(byt);

ch := chr(byt)

end;

end;

end;

end;  {parse}

begin

clrscr;

writeln('  SQUEEZE is designed to remove         This program may

be copied freely ');

writeln('  redundancy from ASCII text files,     for educational

use or to try it  ');

writeln('  making them smaller for transmission  out.  If you like

it, donations   ');

writeln('  and storage.  It also improves the    will be accepted

by the author.   ');

writeln('  security of files encrypted after     This program is

believed to be    ');

writeln('  SQUEEZING.  It uses LANGUAGE.COD,     reliable, but it

is the user',ap,'s    ');

writeln('  created with COUNT and MAKETREE.      responsibility

to determine its   ');

writeln('                                        fitness for use.  No

liability    ');

writeln('  Mike Johnson                          will be assumed

by the author.    ');

writeln('  P. O. Box 1151                        Copyright (C) 1988

Mike Johnson.  ');

writeln('  Longmont, CO 80502-1151               All rights reserved.              ');

if (paramcount < 3) then

begin

writeln;

writeln('Syntax: SQUEEZE a|s|u|x outputfile inputfile(s)');

writeln('a and s both mean add or squash, u and x both mean

unsquash or extract.');

writeln('Input and output files must be different.');

writeln('Input file name(s) may include wild cards.');

writeln('LANGUAGE.COD  (made with MAKETREE) must be in default

directory.');

end

else

begin

{Read in language code dictionary.}

assign(infile,'LANGUAGE.COD');

reset(infile,1);

infilepos := 0;

inbitpos := 1;

done := eof(infile);

newline := nil;

last := nil;

endfile := nil;

if paramcount >= 1 then

temp := paramstr(1)

else

temp := 'X';

temp := upcase(temp[1]);

if (temp = 'A') or (temp = 'S') then

squish := true

else

squish := false;

writeln('Reading in LANGUAGE.COD.');

for i := 1 to 32 do

begin

new(top[i]);

top[i]^.wrd := '';

top[i]^.count := 0;

top[i]^.code := 0;

top[i]^.next := nil;

bottom[i] := top[i];

end;

last := nil;

while not done do

begin

new(newbuf);

newbuf^.wrd := '';

getbite(strlen);

if not done then

begin

for i := 1 to strlen do

begin

getbite(bite);

newbuf^.wrd := newbuf^.wrd + chr(bite);

end;

getbite(bite);

newbuf^.count := bite;

getbite(bite);

newbuf^.code := bite;

if newbuf^.count > 8 then

begin

getbite(bite);

long := bite;

newbuf^.code := newbuf^.code + (long shl 8);

if newbuf^.count > 16 then

begin

getbite(bite);

long := bite;

newbuf^.code := newbuf^.code + (long shl 16);

if newbuf^.count > 24 then

begin

getbite(bite);

long := bite;

newbuf^.code := newbuf^.code + (long shl

24);

end;

end;

end;

newbuf^.next := nil;

if newbuf^.wrd = chr(255) then

begin

last := newbuf;

end

else

if newbuf^.wrd = chr(255) + chr(255) then

newline := newbuf

else

if newbuf^.wrd = chr(255) + chr(255) + chr(255)

then

endfile := newbuf;

if squish then

begin

if ord(newbuf^.wrd[1]) > 107 then

i := length(newbuf^.wrd) + 15

else

i := length(newbuf^.wrd)

end

else

i := newbuf^.count;

bottom[i]^.next := newbuf;

bottom[i] := newbuf;

write(#13,newbuf^.wrd,'                     ');

end;

end;

if newline = nil then

writeln('Error:  newline symbol not in LANGUAGE.COD! ');

writeln;

close(infile);

{Initialize input & output files and global variables.}

outname := paramstr(2);

filefound := false;

assign(outfile,outname);

if exist(outname) then

begin

reset(outfile,1);

seek(outfile,filesize(outfile))

end

else

rewrite(outfile,1);

outfilepos := 0;

outbitpos := 1;

outbite := 0;

if squish then start := 3 else start := 2;

for cmdparameter := start to paramcount do

begin

filespec := paramstr(cmdparameter);

filemask := filespec;

path := '';

repeat

i := pos('\',filemask);

if i > 0 then

begin

path := path + copy(filemask, 1, i);

filemask := copy(filemask, i+1, length(filemask));

end;

until i = 0;

findfirst(filespec, readonly + hidden + archive, s);

while doserror = 0 do

begin

filefound := true;

inname := path + s.name;

assign(infile, inname);

reset(infile,1);

infilepos := 0;

inbitpos := 1;

done := eof(infile);

{Perform compression on input file(s).}

if squish then

begin

writeln('Compressing ',inname);

ch := chr(0);

while not done do

begin

parse(words,ch);     {Find word or punctuation}

if length(words) > 0 then

begin

if ord(words[1]) > 107 then

buf := top[length(words) + 15]

else

buf := top[length(words)];

mask := 1;

while (buf^.wrd <> words) and (buf^.next <>

nil) do

buf := buf^.next;

if buf^.wrd <> words then

begin

highvideo;

if words = chr(255) + chr(255) then

writeln('Newline code not found in tree!

')

else

write(words);

for i := 1 to last^.count do

begin

masked := last^.code and mask;

mask := mask shl 1;

if masked = 0 then

putbit(0)

else

putbit(1);

end;

for i := 0 to length(words) do

begin

for j := 0 to 6 do

begin

b := (ord(words[i]) shr j) and

1;

putbit(b)

end;

end;

end

else

begin

for i := 1 to buf^.count do

begin

masked := buf^.code and mask;

mask := mask shl 1;

if masked = 0 then

putbit(0)

else

putbit(1);

end;

lowvideo;

if words = chr(255) + chr(255) then

writeln

else

write(buf^.wrd);

end;

end;

end;

mask := 1;

for i := 1 to endfile^.count do

begin

masked := endfile^.code and mask;

mask := mask shl 1;

if masked = 0 then

putbit(0)

else

putbit(1);

end;

end

else

{Decompress input file.}

begin

writeln('Decompressing ',inname);

cd := 0;

mask := 1;

cnt := 0;

while not done do

begin

getbit(b);

if b > 0 then cd := cd or mask;

mask := mask shl 1;

if mask <= 0 then writeln('Error: code too long.');

inc(cnt);

buf := top[cnt];

while (buf^.next <> nil) and

((cnt <> buf^.count) or (cd <> buf^.code))

do

buf := buf^.next;

if (cnt = buf^.count) and (cd = buf^.code) then

begin

if buf = endfile then

done := true

else

if buf^.wrd = chr(255) then

begin

j := 0;

for k := 0 to 6 do

begin

getbit(b);

j := j + (b shl k);

end;

for i := 1 to j do

begin

m := 0;

for k := 0 to 6 do

begin

getbit(b);

m := m + (b shl k)

end;

putbite(m);

highvideo;

write(chr(m));

end;

end

else

if buf^.wrd = chr(255) + chr(255) then

begin

writeln;

putbite(13);

putbite(10);

end

else

begin

for i := 1 to length(buf^.wrd) do

begin

b := ord(buf^.wrd[i]);

putbite(b)

end;

lowvideo;

write(buf^.wrd)

end;

cd := 0;

cnt := 0;

mask := 1;

end;

end;

end;

{Finish up or loop back for more work to do.}

flushbit;

close(infile);

findnext(s);

end;

end;

if not filefound then writeln('No matching files found.');

flushbit;

close(outfile);

end;

end.

{That's all, folks!}