Results of experimentation on Logitech serial mouse.

On my previous computer, the correct settings of the serial port seemed to be:

InitSerialChannel (channel, 2400, 8, OddParity, 2);

i.e. COM2, 2400 baud, 8 data bits, odd parity, 2 stop bits.

The mouse driver on my office machine sets up

COM1, 2400 baud, 8 data bits, no parity, 1 stop bit.

but on my home machine it seems to be

COM1, 1200 baud, 7 data bits, no parity, 1 stop bit.

The input from the mouse seems to be either 3-byte binary or 5-digit hex.

Commands (hard to classify, so this is a rough approximation):

c	returns what looks like a copyright notice

f	replies Z (in binary mode)

k	replies 3 ("   "      "  )

s	replies one-byte status code

t	letter S followed by status code

P	seems to disable mouse

R	enable mouse

W	set hex mode

S	set binary mode

Movement codes: haven't tried to write these down in detail yet,

but it's clear that pressing mouse buttons gives a code which is

the sum of buttons which are held down, and there seems to be an

all-zero code for button release.

In hex mode, the 5 digits seem to be

digit 1		button code: 4=left, 2=middle, 1=right, 0=none

(and these are summed)

digits 2-3	amount of movement right (or left if negative)

digits 4-5	amount of movement up (or down if negative)

My tests always leave the mouse in a funny mode, so it looks as if

I've sent some mode-switching commands which I haven't yet identified.

NOTES FROM MICHAEL WALSH

Setting up the mouse port

======================

SetUpUart(MousePort,TRUE,1200,7,TRUE,none);

(* Initialisation code for mouse belongs here, if deemed necessary

to supply it. (See above) *)

OutByte(MousePort + MCR,0BH);

(* Experiment with the logitech mouse driver in microsoft compatible

mode shows Modem Control register has Data Terminal Ready,Request

to Send, and Out2 all set ie. driving the control lines low. *)

OutByte(MousePort + MSR,10);

OutByte(MousePort + MSR,00);

(* Logitech mouse once initialised will always enter microsoft compatible

mode when Request to Send is toggled, No matter what jumper/software

combination is currently  in used. A Microsoft mouse requires no set

up, so this has no effect. *)