*------------- Telecommunications & Signal Processing Lab --------------

*                          McGill University

*

*

* Module:

*     PROGRAM DFIR

*

*

* Purpose:

*     Design FIR filters

*

*

* Description:

*     This program designs six types of finite impulse response

*     filters.

*     1. Equiripple Linear Phase Bandpass Filter (multiple passbands

*        and stopbands).  The filter is specified in terms of the

*        desired response in bands.  The band specifications include

*        desired value, relative weighting and constraint limits on the

*        allowed values in the band.  The resulting filter is a minmax

*        approximation (with constraints) to the given specifications.

*        The filter coefficients have an even symmetry about the middle

*        of the filter.

*     2. Equiripple Linear Phase Bandpass Filter (multiple passbands

*        and stopbands) with sin(x)/x compensation.  The filter is

*        specified in terms of the desired response in bands.  The band

*        specifications include desired value, relative weighting and

*        constraint limits on the allowed values in the band.  The

*        resulting filter is a minimax approximation (with constraints)

*        to the given specifications.  The filter coefficients have an

*        even symmetry about the middle of the filter.

*     3. Equiripple Differentiator.  The filter is specified in terms

*        of the desired slope in bands.  The band specifications

*        include desired slope, relative weighting and constraint

*        limits on the allowed slopes in the band.  The resulting

*        filter is a minimax approximation (with constraints) to the

*        given specifications.  The filter coefficients have an odd

*        symmetry about the middle of the filter.

*     4. Equiripple Hilbert Transform Filter. The filter is specified

*        in terms of the desired response in bands.  The band

*        specifications include desired value, relative weighting and

*        constraint limits on the allowed values in the band.  The

*        resulting filter is a minimax approximation (with constraints)

*        to the given specifications.  The filter coefficients have an

*        odd symmetry about the middle of the filter.

*     5. Interpolating Filter.  The design is specified in terms of the

*        power spectrum of the input signal.  For this given spectrum,

*        the mean-square interpolating error is minimized.  The filter

*        is linear phase, i.e., the coefficients are symmetrical (even)

*        about the middle of the filter.  If the number of coefficients

*        is odd, the filter coefficients have regularly spaced zero

*        crossings with a spacing equal to the interpolation ratio.

*     6. Equiripple Nyquist Filter.  The design is specified in terms

*        of a zero crossing interval and a stopband weighting.  The

*        resulting filter will be a minimax approximation to the

*        stopband specification with the constraint of the regular zero

*        crossings.  The filter is linear phase, with even symmetry.

*        The total number of coefficients is odd.

*     7. Minimum Phase Factor of an Equiripple Nyquist Filter.  The

*        design is specified in terms of a zero crossing interval and

*        a stopband weighting. The resulting filter will be a minimax

*        approximation to the stopband specification with the

*        constraint of the regular zero crossings.  The filter

*        generated by this program is the minimum phase factor of the

*        overall Nyquist filter. The other factor (maximum phase) can

*        be obtained by time reversing the coefficients of the minimum

*        phase factor.  The overall linear phase Nyquist filter is the

*        convolution of the minimum and maximum phase filters.

*

*     The most common cause of numerical difficulties in designing a

*     filter (signalled by error messages) is the use of too high an

*     order filter for the given specifications. This is particularly

*     so for Interpolating Filter and Nyquist Filter designs.  In all

*     cases, the quality of the final design should be checked by

*     plotting the frequency response curve.

*

*

* Parameters:

*     Data values can be specified in free format.  Multiple values can

*     be separated by spaces or commas.  Keywords can be in either

*     upper or lower case.

*

*       RECORD 1 - Filter_type

*          This is one of the keyword phrases given below (an asterisk

*          indicates the shortest acceptable abbreviation).

*          BPF - Multiple passband/stopband filter

*          REC*EIVING FILTER - Multiple passband/stopband filter used

*             as a receiving filter (sin(X)/X compensation)

*          INT*ERPOLATING FILTER - Interpolating filter

*          DIF*FERENTIATOR - Differentiator

*          HIL*BERT TRANSFORM FILTER - Hilbert transform filter

*          NYQ*UIST FILTER - Nyquist filter

*          MIN*IMUM PHASE - Minimum phase factor of a Nyquist filter

*

*     Bandpass filters, differentiators and Hilbert transform filters:

*

*       RECORD 2 - Ncof [Fsamp [Grid]]

*          This record contains 1, 2 or 3 items.  The last two are

*          optional.

*          1 - Number of filter coefficients (maximum 256)

*          2 - Sampling frequency.  If the sampling frequency is not

*              specified, the normalized value of one is used.  The

*              summary printout uses normalized frequency.

*          3 - Grid density.  The desired function value is translated

*              to a specification on a dense grid of frequency points.

*              The grid density is the number of grid points per

*              extremum of the response.  If the grid density is not

*              specified, it is set as large as possible (given the

*              array space available) up to a maximum of 16.  If the

*              keyword MAX*IMUM is specified, the maximum grid density

*              compatible with the array space is used.

*       RECORD 3 - [File_name]

*          This file is used to store the coefficient values.  If the

*          file name is all blank, the output file is not created;

*          instead the values are printed out.

*       Following RECORD(S) - Freq Value Weight [Low_V High_V]

*          The desired values, weights and constraint values are

*          specified in bands.  Within a band, tabulated values are

*          given.  Piecewise monotonic cubic interpolation is used

*          between the given tabulated points.  Bands are separated

*          with a transition band record.  The specifications must be

*          in order of increasing frequency.

*          - Transition band record (placed between bands): all blank

*            record or the keyword *TRANSITION BAND

*          - Tabulated band values:

*            5 items.  The last two values are constraints.  Within a

*            band, all or none of the points must have constraint

*            values.  The program approximates the best minimax filter

*            subject to the response lying between the given constraint

*            limits.

*            1 - Frequency

*            2 - Desired filter response value (or desired slope for

*                differentiators)

*            3 - Relative weighting for the value (or slope)

*            4 - Lower constraint limit on the filter response (or

*                slope for differentiators)

*            5 - Upper constraint limit on the filter response (or

*                slope for differentiators)

*            A maximum of 200 frequency points can be specified.  The

*            program expects specification records to be entered until

*            one of the following occurs.

*            i) A frequency equal to half of the sampling frequency

*               (normalized frequency of 0.5) is entered,

*            ii) two null lines have been entered (the first signifies

*                a transition band), or

*            iii) an end-of-file is encountered.

*

*     Interpolating filters:

*

*       RECORD 2 - Int_ratio

*          This record specifies the interpolation ratio.

*       RECORD 3 - Ncof [Fsamp]

*          This record has two values, the second being optional.

*          1 - Number of filter coefficients (maximum 399).  The filter

*              can be considered to be composed of IR subfilters, where

*              IR is the interpolating ratio.  A subfilter contains

*              every IR'th coefficient of the original filter.  The

*              maximum length of any subfilter is 42. This means that

*              there is an additional upperbound on the filter length

*              of 42*IR+1.  If the number of coefficients is odd, the

*              zero'th subfilter which includes the middle coefficient

*              will have coefficients 0, ... 0, 1, 0, ... 0, with the

*              unity coefficient being the middle coefficient of the

*              overall filter.  If the filter length is of the form

*              2*L*IR+1, the outer coefficients are both zero.

*          2 - Sampling frequency of the input sequence.  The sampling

*              frequency at the output of the interpolator is IR times

*              this value.  If the sampling frequency is not specified,

*              the normalized value of one is used.  The summary

*              printout uses a normalized sampling frequency.

*       RECORD 4 - [File_name]

*          This file is used to store the coefficient values.  If the

*          file name is all blank, the output file is not created;

*          instead the values are printed out.

*       Following RECORDS - Freq PSpecd [psd]    or   Freq Cpow cos

*          The remaining records give power spectrum values.  The power

*          spectrum of the input sequence is specified in terms of a

*          continuous component and sinusoidal components.  The

*          continuous component is given as tabulated values of the

*          power spectral density.  A piecewise monotonic cubic

*          interpolation is used between the given values.  The

*          sinusoidal components of the power spectrum are specified by

*          frequency and power.  Each input record specifies either a

*          power spectral density value or sinusoidal power value.  For

*          a power spectral density, the first field is the frequency,

*          the second field is the value of the power spectral density,

*          and the third field is either blank or contains the

*          identifying string "psd".  The continuous power spectrum

*          must be specified with frequency values which are in

*          increasing order.  The power spectral density is assumed to

*          be zero outside the given range of frequencies.  In

*          addition, the cubic interpolant for the continuous portion

*          of the power spectrum has zero slope at the first and last

*          points specified.  For a sinusoidal component, the first

*          field is the frequency, the second field the power of the

*          sinusoidal component, and the third field contains the

*          identifying string "cos".

*

*     Nyquist filters:

*

*       RECORD 2 - Nz

*          This record specifies the zero crossing interval (samples)

*       RECORD 3 - Ncof [Fsamp [Grid]]

*          This record contains 1, 2 or 3 items.  The last two tems are

*          optional.

*          1 - Number of filter coefficients for Nyquist filter (odd

*              number, maximum 255)

*          2 - Sampling frequency.  If the sampling frequency is not

*              specified, the normalized value of one is used. The

*              summary printout uses normalized frequency.

*       RECORD 4 - [File_name]

*          This file is used to store the coefficient values.  If the

*          file name is all blank, the output file is not created;

*          instead the values are printed out.

*       Following RECORD(S) - Freq Weight

*          The following records give the stopband weight

*          specifications for the Nyquist filter (2 items to a record).

*          Piecewise monotonic cubic interpolation is used between the

*          given tabulated weights. The values must be given in order

*          of increasing frequency.

*          1 - Frequency.  The first frequency specifies the lower edge

*              of the stopband (normally in the interval (Fs/(2*Nz) to

*              Fs/2, where Fs is the sampling frequency and Nz is the

*              zero crossing interval).

*          2 - Relative weighting for the frequency point.

*          A maximum of 200 frequency points can be specified.  The

*          program expects specification records to be entered until

*          one of the following occurs.

*          i) A frequency equal to half of the sampling frequency

*             (normalized frequency of 0.5) is entered,

*          ii) a null line is been entered, or

*          iii) an end-of-file is encountered.

*

*     Minimum Phase Factor of a Nyquist filter:

*

*       RECORD 2 - Nz

*          This record specifies the zero crossing interval (samples)

*       RECORD 3 - Ncof [Fsamp [Grid]]

*          This record contains 1, 2 or 3 items.  The last two tems are

*          optional.

*          1 - Number of filter coefficients for the minimum phase

*              factor (maximum 128)

*          2 - Sampling frequency.  If the sampling frequency is not

*              specified, the normalized value of one is used. The

*              summary printout uses normalized frequency.

*          3 - Grid density.  The desired stopband weights are

*              translated to a specification on a dense grid of

*              frequency points. The grid density is the number of grid

*              points per extremum of the stopband response. If the

*              grid density is not specified, it is set as large as

*              possible (given the array space available) up to a

*              maximum of 16.  If the keyword MAX*IMUM is specified,

*              the maximum grid density compatible with the array

*              space is used.

*       RECORD 4 - Coefficient file.  This files are used to store the

*          coefficient values of the minimum phase factor of the

*          Nyquist filter.  If a coefficient file is not specified, the

*          coefficients of the minimum minimum phase factor of the

*          Nyquist filter are printed out.

*       Following RECORD(S) - Freq Weight

*          The following records give the Stopband weight

*          specifications for the minimum phase factor (2 items to a

*          record).  Piecewise monotonic cubic interpolation is used

*          between the given tabulated weights.  The values must be

*          given in order of increasing frequency.

*          1 - Frequency.  The first frequency specifies the lower edge

*              of the stopband (normally in the interval (Fs/(2*Nz) to

*              Fs/2, where Fs is the sampling frequency and Nz is the

*              zero crossing interval).

*          2 - Relative weighting for the frequency point.

*          A maximum of 200 frequency points can be specified.  The

*          program expects specification records to be entered until

*          one of the following occurs.

*          i) A frequency equal to half of the sampling frequency

*             (normalized frequency of 0.5) is entered,

*          ii) a null line is been entered, or

*          iii) an end-of-file is encountered.

*

*     Examples -

*     32 tap constrained bandpass filter:

*     Stopbands are from 0 to 0.1 and 0.425 to 0.5, and the passband is

*     from 0.2 to 0.35.  The relative weights are 10 in the stopbands

*     and 1 in the passband. The response is constrained to be positive

*     (between 0 and 1) in the first stopband. The filter coefficients

*     will be written to file filt.cof.

*         bpf

*         32

*         filt.cof

*         0   0 10 0 1

*         0.1 0 10 0 1

*           Transition Band

*         0.2  1 1

*         0.35 1 1

*           Transition Band

*         0.425 0 10

*         0.5   0 10

*     23 tap interpolating filter:

*     The input spectrum is modelled by a power spectrum with a

*     transition band from 0.3 to 0.4 and a dc component.  The

*     interpolating ratio is 4.  The coefficients are written to file

*     fili.cof.

*         Int

*         4

*         23

*         fili.cof

*         0   1000  cos

*         0   1

*         0.3 1

*         0.4 0

*     15 tap minimum phase factor of a Nyquist filter:

*     The zero crossing interval is 4.  An excess bandwidth of 50% is

*     used.  The coefficients of the minimum phase factor are written

*     to file filn.cof.  An equiripple design is used.

*         MIN

*         4

*         15

*         filn.cof

*         0.1875, 1

*         0.5,    1

*

*

* Routines required:

*     FIRDSG - Design a linear phase FIR filter

*     HALT   - Print an error message, stop with error status set

*     INTDSG - Design a min. mean-square error interpolating filter

*     KEYUPC - Match keyword strings (case insensitive)

*     LENSTR - Find the length of a whitespace terminated string

*     MPHDSG - Design a factored Nyquist filter

*     NYQDSG - Design a Nyquist filter

*     PRTCOF - Print FIR filter coefficients

*     PRTFIR - Print FIR filter design parameters

*     PRTINT - Print interpolating filter design parameters

*     PRTMPH - Print Nyquist filter parameters (factored filter)

*     PRTNYQ - Print Nyquist filter design parameters

*     RDLINE - Read a line of input from standard input

*     REDFIR - Read FIR filter specifications

*     REDINT - Read interpolating filter specifications

*     REDMPH - Read Nyquist filter specifications (factored filter)

*     REDNYQ - Read Nyquist filter specifications

*     WRTFIR - Write an FIR filter coefficient file

*     WRTINT - Write an interpolating filter coefficient file

*     WRTMPH - Write a Nyquist filter file (factored filter)

*     WRTNYQ - Write a Nyquist filter coefficient file

*

*

* Author / revision:

*     P. Kabal  Copyright (C) 1993

*     $Revision: 1.4 $  $Date: 1993/01/25 20:16:45 $

*

*

*-----------------------------------------------------------------------