Date: Tue, 01 Sep 1998 13:19:14 +1000

From: corey Kent <corey@enterprise.net.au>

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Solid stae tesla coils

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

hi,

im another new person to the list and i was wondering if there were any

easy to follow Solid state tesla coil plans??? and when i say easy i mean

easy! ok cya's and thanx

Date: Mon, 31 Aug 1998 21:42:48 -0600 (MDT)

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Daniel Lockhart <molly@cmn.net>

Subject: Re: Exploding water...

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

Bert wrote:

>Daniel and all,

>

>It doesn't take very much to atomize water, especially if an external

>switching gap is used to "pulse" the water. Very little water appears to

>be vaporized. Instead much of the energy appears to be converted to

>kinetic energy. Exploding water in this manner can also be quite

>dangerous - see my other post on exploding water.

>

>-- Bert --

How exactly would "an external switching gap" be implemented? Would a

smaller high voltage source trigger a larger one, such as in a flashlamp

for a laser (that's the first thing I thought of). Is this what you mean? I

don't quite understand.

and later,

>Interesting discussion. For those who may be considering water-arc

>experiments, please be VERY careful! Richard Hull and the EM theorist

>Peter Graneau conducted some water-arc research a few years ago, using

>only a few hundred joules. They were able to punch holes through 1/4"

>aluminum plate using JUST the jet of high velocity water that was

>created. The velocity of the ejected water jet(s) can be hypersonic, and

>there's substantial risk of injury to nearby objects (including the

>experimenter!).

>

>A high energy spark under water creates a high-amplitude shock wave. If

>not properly controlled, this has the potential to rupture the container

>AND injure the experimeter.

Could these principles be used in the same way as chemical explosive shaped

charges to be directed and focused (the Munroe Effect)? This would make a

much more powerful jet of water. Just an idea.

Daniel Lockhart

Date: Tue, 01 Sep 1998 00:00:35 -0500

From: Bert Hickman <bert.hickman@aquila.com>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Organization: Stoneridge Engineering

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Exploding water...

Sender: tesl-err@emachine.com

Daniel Lockhart wrote:

>

> Bert wrote:

>

> >Daniel and all,

> >

> >It doesn't take very much to atomize water, especially if an external

> >switching gap is used to "pulse" the water. Very little water appears to

> >be vaporized. Instead much of the energy appears to be converted to

> >kinetic energy. Exploding water in this manner can also be quite

> >dangerous - see my other post on exploding water.

> >

> >-- Bert --

>

> How exactly would "an external switching gap" be implemented? Would a

> smaller high voltage source trigger a larger one, such as in a flashlamp

> for a laser (that's the first thing I thought of). Is this what you mean? I

> don't quite understand.

>

You've got the right idea. An external gap can either be self-triggered,

or be triggered by an external method - UV light, laser pulse, or a

voltage pulse as in a triggatron. The main gap then breaks down, letting

the main storage cap discharge through the object (i.e., water) under

test.

> and later,

>

> >Interesting discussion. For those who may be considering water-arc

> >experiments, please be VERY careful! Richard Hull and the EM theorist

> >Peter Graneau conducted some water-arc research a few years ago, using

> >only a few hundred joules. They were able to punch holes through 1/4"

> >aluminum plate using JUST the jet of high velocity water that was

> >created. The velocity of the ejected water jet(s) can be hypersonic, and

> >there's substantial risk of injury to nearby objects (including the

> >experimeter!).

> >

> >A high energy spark under water creates a high-amplitude shock wave. If

> >not properly controlled, this has the potential to rupture the container

> >AND injure the experimeter.

>

> Could these principles be used in the same way as chemical explosive shaped

> charges to be directed and focused (the Munroe Effect)? This would make a

> much more powerful jet of water. Just an idea.

>

> Daniel Lockhart

High explosives are still used where a very large amount of energy is

required very quickly... the energy density/pound is still much greater

than capacitors. However, a high-speed, low inductance capacitor

discharge appears to do some rather unique things to water above and

beyond that expected by shockwave reflection/focusing, things that may

be a function of the electrical discharge process.

There's no reason why techniques to manipulate and intensify an

electrically induced shockwave as done with shaped explosive charges.

Colliding shock waves can producr material some intersting effects -

under these conditions, seemingly incompressable solids may undego an

increase a temporary density increase of 2:1, 3:1 or much more...

followed by a horrendous elastic rebound. If these  colliding shock

waves result in hypersonic "jets" of material (Munroe effect), virtually

NO material can contain or stop them... including military armor. This

canbe made to  occur in spark-discharge systems with properly designed

chambers. But don't try this at home, folks if you value life and

limb...

-- Bert --

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Date: Tue, 01 Sep 1998 04:18:35 -0700

From: "paul m" <fleischwulf@mailexcite.com>

Cc:

X-Sent-Mail: off

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

Subject: Re: Exploding water...

X-Sender-Ip: 208.153.7.9

Organization: MailExcite  (http://www.mailexcite.com:80)

Sender: tesl-err@emachine.com

Dan,

Unfortunatly there is no way to get arround the law of conservation of energy and matter. Since there are no systems which are 100% efficiant, consider this. There are 1.357 j to 1 foot lb. A lowly .22 cal rifle bullet will deliver 115 foot lbs

to its target that is equivalent to 156 j. This represents 50 times the energy discharged by a microwave capacitor. Water is very efficiant thermally so the energy transfer should be 90% or better. A .308 winchester is used to hunt deer (High

powered rifle) it delivers about 3745 j. A large pulse capacitor can deliver 25-50 Kj. This is almost equal to the energy delivered by a 20mm anti tank round. (62 Kj) I hope this sheds a little light on energy discharge.

paul m.

---

Einigheit und Wahrheit

On Mon, 31 Aug 1998 21:42:48   Daniel Lockhart wrote:

>Bert wrote:

>

>>Daniel and all,

>>

>>It doesn't take very much to atomize water, especially if an external

>>switching gap is used to "pulse" the water. Very little water appears to

>>be vaporized. Instead much of the energy appears to be converted to

>>kinetic energy. Exploding water in this manner can also be quite

>>dangerous - see my other post on exploding water.

>>

>>-- Bert --

>

>How exactly would "an external switching gap" be implemented? Would a

>smaller high voltage source trigger a larger one, such as in a flashlamp

>for a laser (that's the first thing I thought of). Is this what you mean? I

>don't quite understand.

>

>and later,

>

>>Interesting discussion. For those who may be considering water-arc

>>experiments, please be VERY careful! Richard Hull and the EM theorist

>>Peter Graneau conducted some water-arc research a few years ago, using

>>only a few hundred joules. They were able to punch holes through 1/4"

>>aluminum plate using JUST the jet of high velocity water that was

>>created. The velocity of the ejected water jet(s) can be hypersonic, and

>>there's substantial risk of injury to nearby objects (including the

>>experimeter!).

>>

>>A high energy spark under water creates a high-amplitude shock wave. If

>>not properly controlled, this has the potential to rupture the container

>>AND injure the experimeter.

>

>Could these principles be used in the same way as chemical explosive shaped

>charges to be directed and focused (the Munroe Effect)? This would make a

>much more powerful jet of water. Just an idea.

>

>Daniel Lockhart

>

From: wwl@netcomuk.co.uk (Mike Harrison)

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Exploding water...

Date: Tue, 01 Sep 1998 12:35:28 GMT

Organization: White Wing Logic

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

On Mon, 31 Aug 1998 20:42:48 -0500, you wrote:

>Mike Harrison wrote:

>>

>> On Mon, 31 Aug 1998 05:16:48 -0700 (PDT), you wrote:

>>

>> >---paul m <fleischwulf@mailexcite.com> wrote:

>> >>

>> >> Dale,

>> >>   As for the exploding water, I don't think 3.4 Joules will come

>> >close. I think it is more in the upper 100's to a few kilojoules.

>>

>> I've had some nice bangs from my surge generator

>> www.netcomuk.co.uk/~wwl/surge.html

>> by dipping electrodes in a film canister lid filled with water. You

>> need a fairly high voltage - 2.5KV - nothing, but 4.5KV (30uF) - BOOM!

>> I've not yet had the courage or containment facilities to investigate

>> the ballistic properties of this.

>>

>>      ____                                                           ____

>>    _/ L_/  Mike Harrison / White Wing Logic / wwl@netcomuk.co.uk  _/ L_/

>>  _/ W_/  Hardware & Software design / PCB Design / Consultancy  _/ W_/

>> /_W_/  Industrial / Computer Peripherals / Hazardous Area      /_W_/

>

>Mike and all,

>

>Interesting discussion. For those who may be considering water-arc

>experiments, please be VERY careful! Richard Hull and the EM theorist

>Peter Graneau conducted some water-arc research a few years ago, using

>only a few hundred joules. They were able to punch holes through 1/4"

>aluminum plate using JUST the jet of high velocity water that was created.

Interesting - although I'd heard reports of this, nothing I've seen

mentions the energy used - I was under the impression that several KJ

was required. Unless you believe the over-unity stuff that has been

claimed for water arcs, I'd have thought you;d need more than a few

hundred joules to go through 14" ali.

As an aside, does anyone know roughly how many joules are released

when a bullet is fired? I'm sure this could be calculated from weight & velocity.

> The velocity of the ejected water jet(s) can be hypersonic, and

>there's substantial risk of injury to nearby objects (including the

>experimeter!).

Now that would be an interesting one to explain at the hospital

..."yes, I said I blew a hole in my hand with WATER!"

____

From: wwl@netcomuk.co.uk (Mike Harrison)

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Exploding water...

Date: Tue, 01 Sep 1998 12:35:30 GMT

Organization: White Wing Logic

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

On Mon, 31 Aug 1998 21:42:48 -0600 (MDT), you wrote:

>Bert wrote:

>

>>Daniel and all,

>>

>>It doesn't take very much to atomize water, especially if an external

>>switching gap is used to "pulse" the water. Very little water appears to

>>be vaporized. Instead much of the energy appears to be converted to

>>kinetic energy. Exploding water in this manner can also be quite

>>dangerous - see my other post on exploding water.

>>

>>-- Bert --

>

>How exactly would "an external switching gap" be implemented? Would a

>smaller high voltage source trigger a larger one, such as in a flashlamp

>for a laser (that's the first thing I thought of). Is this what you mean? I

>don't quite understand.

There's some info I found on spark gap switches on my site :

www.netcomuk.co.uk/~wwl/pulse.html

____                                                           ____

_/ L_/  Mike Harrison / White Wing Logic / wwl@netcomuk.co.uk  _/ L_/

_/ W_/  Hardware & Software design / PCB Design / Consultancy  _/ W_/

/_W_/  Industrial / Computer Peripherals / Hazardous Area      /_W_/

X-Originating-IP: [199.217.140.22]

From: "Bill the arcstarter" <arcstarter@hotmail.com>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Exploding water...

Date: Tue, 01 Sep 1998 06:16:03 PDT

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

It was written:

>followed by a horrendous elastic rebound. If these colliding shock

>waves result in hypersonic "jets" of material (Munroe effect), virtually

>NO material can contain or stop them... including military armor. This

>canbe made to  occur in spark-discharge systems with properly designed

>chambers. But don't try this at home, folks if you value life and

>limb...

That's right.  Don't try it at your own home.  Follow my example -

I do all my dangerous experiments down at Paul's house...

-Bill :)

From: "Steve B." <steve@breakfix.demon.co.uk>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Line Filters

Date: Tue, 1 Sep 1998 16:16:12 +0100

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Hi Guys,

When working with a single phase supply, can the 2 unused phases on a

3-phase line filter be used for extra filtering by wiring them all in

series?

TIA

----------------------------------------------

Steve Bell  (steve@breakfix.demon.co.uk)

Stuff also available at www.breakfix.demon.co.uk

From: Roy Minnich <rminnich@rolling-meadows.lib.il.us>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: RE: Is this how it works?

Date: Tue, 1 Sep 1998 12:06:49 -0500

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

Hi Erik,

How is the multi-layered primary coil wired??  Is each layer in series with the others?

Roy Minnich

----------

From: 	ESchulz531@aol.com[SMTP:ESchulz531@aol.com]

Reply To: 	Tesla-2 Mailing List

Sent: 	Monday, August 31, 1998 11:06 AM

To: 	Tesla-2 Mailing List

Subject: 	Re: Is this how it works?

HI,

Your design sounds good.  The best way to get maximum efficiency is to try

more than one primary coil.

You can use a conical one like this, old picture way to much coil clearance

http://members.aol.com/tesla00/sec_sec.jpg

Or if you want maximum coupling you can try a multi layered coil like this

http://members.aol.com/tesla00/larger.jpg

And of course you can use a flat spiral

What do you mean by maximum efficiency?  Maximum energy transfer or maximum

spark length.  They are not the same...

For the longest spark you also want to try different toriod sizes.

Erik Schulz

http://members.aol.com/tesla00/

Date: Tue, 01 Sep 1998 10:43:15 -0700

From: Gene Waldo <gwaldo@1stnetusa.com>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: safety gaps

Sender: tesl-err@emachine.com

bob golding wrote:

>

> Hi All,

>

>   I have got my coil up and running again after rebuilding

>   I am still trying to get the coil into tune.

> I am using the "Gene Waldo special" type of gap with three bolts set at 120

> degrees apart. Should the distance between

> each "live" bolt and ground be just greater than the total main gap, or

> should the total be split between the two "live" bolts and ground. EG main

> gap 100 mil SG 120 mil each side, or 60 mil each side. I ask because my

> safety gap does not fire at all now. I am hoping it is the chokes but I am

> not sure and I do not want to fry my neon.

>

> many thanks

>

> bob golding

>

> bob yubba@clara.net

Bob,

My intuitive approach will be to set SG gaps (carefully equal), to 'just'

not fire w/NST deadheaded to SG + RFGnd..  The addition of the tank ckt

may change these settings.

I should imagine that adjusting for a SG snap at no more than 1 or 4

per minute of "normal" operation should provide adequate protection.

Better experience than mine will needs apply.

----------------------------------------

Gene Waldo -  gwaldo@1stnetusa.com  - "Igor in Residence"

X-Originating-IP: [199.217.140.22]

From: "Bill the arcstarter" <arcstarter@hotmail.com>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: safety gaps

Date: Tue, 01 Sep 1998 12:09:16 PDT

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

Gene wrote:

>My intuitve approach will be to set SG gaps (carefull equal), to 'just'

>not fire w/NST deadheaded to SG + RFGnd..

That sounds OK! (thats also how I do it)

>The addition of the tank ckt may change these settings.

Oh, I don't think I like that.  If you are getting extra safety gap

firings at this point, I'd just leave it, or else reduce the main gap

spacing.

The first part of the adjustment sets the max safety gap spacing for the

given transformer voltage.  I think that's just where it should be set!

That's what I think.  Your mileage may vary.

-Bill

X-Mailer: IPAD 2.0

Date: Tue, 01 Sep 1998 14:57:17 -0500

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: safety gaps

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

At 12:09 PM 9/1/98 PDT, you wrote:

>Gene wrote:

>

>>My intuitve approach will be to set SG gaps (carefull equal), to 'just'

>>not fire w/NST deadheaded to SG + RFGnd..

>

>That sounds OK! (thats also how I do it)

As do I but...

>

>>The addition of the tank ckt may change these settings.

>

>Oh, I don't think I like that.  If you are getting extra safety gap

>firings at this point, I'd just leave it, or else reduce the main gap

>spacing.

>

>The first part of the adjustment sets the max safety gap spacing for the

>given transformer voltage.  I think that's just where it should be set!

>

>That's what I think.  Your mileage may vary.

>-Bill

>

Setting the safety gap to just beyond what the neon can break down

on its own is the safest. However in the never ending search for

longer and longer sparks who can resist pushing things just a little

harder than we should? Ive always said "you aint having fun unless

things are breaking".

Most neons can tolerate a LITTLE overvoltage but be careful.

I like to see my safety gap firing every 3 or 4 seconds weakly.

If your safety isnt firing at all close it down a bit until it is.

If its firing constantly AND your main gap is within the maximum

values then open up the safety a little. Be careful though.

If you open it too wide it will cease to be "safe".

In my case since I run unpotted rebuilt neons almost exclusively

I can afford to be very generous with the safety gap settings.

So it really boils down to how daring you want to be and how

many spare neons you got.

TTYL!

Mike Hammer

mhammer@misslink.net

X-Mailer: IPAD 2.0

Date: Tue, 01 Sep 1998 15:02:10 -0500

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: Exploding water...

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

All,

Lets end this thread as it has drifted quite far off topic.

TTYL!

Mike Hammer

mhammer@misslink.net

Date: Tue, 01 Sep 1998 14:07:49 -0700

From: Gene Waldo <gwaldo@1stnetusa.com>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Exploding water...

Sender: tesl-err@emachine.com

Mike Hammer wrote:

>

> All,

> Lets end this thread as it has drifted quite far off topic.

>

> TTYL!

>

> Mike Hammer

> mhammer@misslink.net

All,

Again Mike is correct!  It is probably all mostly my fault...

Sorry!

But, still......

I remain,

--

Gene Waldo -  gwaldo@1stnetusa.com  - "Igor in Residence"

PS: What ever became of Murphy of Waldo-Murphy Project?? Dan has bought

a boat! 22' sloop or some such--  moored far away---

fix-up-paint-up-yar-up-travel-time, etc....  no current time for even

E-Mail, let alone TESLA!

Date: Tue, 1 Sep 1998 14:15:04 -0700 (PDT)

From: Sam Barros <sambarros@yahoo.com>

Subject: Exploding water...

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

> Mike Hammer wrote:

> >

> > All,

> > Lets end this thread as it has drifted quite far off topic.

> >

> > TTYL!

> >

> > Mike Hammer

> > mhammer@misslink.net

Shame! It was making quite an interesting discussion...

Still, Mike is right; This is a Tesla List after all... But since

there are no Water Exploding mailing lists, does anyone mind if I mail

my results for the barrel-mounted, 12J sparkplug design when i get it

done? It won't become a habit... I promiss..

Sam.

==

Sam Barros,

sambarros@yahoo.com

ICQ:15156975

"The less you know, the better you sleep"...

From:          "Malcolm Watts" <MALCOLM@directorate.wnp.ac.nz>

Organization:  Wellington Polytechnic, NZ

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Date:          Wed, 2 Sep 1998 09:27:06 +1200

Subject:       Re: safety gaps

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Hi all,

I said when I joined this list that I was only here to

listen but as Mike Hammer suggested to me when I requested

subscription, I can't keep all that quiet for long. I have an idea

that has been kicking around for a while that someone may be in a

position to try:

> From:           "Bill the arcstarter" <arcstarter@hotmail.com>

> To:             "Tesla-2 Mailing List" <tesla-2@emachine.com>

> Subject:        Re: safety gaps

> Date sent:      Tue, 01 Sep 1998 12:09:16 PDT

> Send reply to:  "Tesla-2 Mailing List" <tesla-2@emachine.com>

> Gene wrote:

>

> >My intuitve approach will be to set SG gaps (carefull equal), to 'just'

> >not fire w/NST deadheaded to SG + RFGnd..

>

> That sounds OK! (thats also how I do it)

>

> >The addition of the tank ckt may change these settings.

>

> Oh, I don't think I like that.  If you are getting extra safety gap

> firings at this point, I'd just leave it, or else reduce the main gap

> spacing.

>

> The first part of the adjustment sets the max safety gap spacing for the

> given transformer voltage.  I think that's just where it should be set!

>

> That's what I think.  Your mileage may vary.

> -Bill

Has anyone ever considered putting a snubbing network directly across

the main gap? If Terry Fritz is correct about the parasitic high

frequencies when the gap ceases to conduct, then why allow such

spikes to reach beyond the gap?

Malcolm

X-Mailer: IPAD 2.0

Date: Tue, 01 Sep 1998 16:49:31 -0500

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: safety gaps

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

At 09:27 AM 9/2/98 +1200, you wrote:

>Hi all,

>         I said when I joined this list that I was only here to

>listen but as Mike Hammer suggested to me when I requested

>subscription, I can't keep all that quiet for long. I have an idea

>that has been kicking around for a while that someone may be in a

>position to try:

>

>> From:           "Bill the arcstarter" <arcstarter@hotmail.com>

>> To:             "Tesla-2 Mailing List" <tesla-2@emachine.com>

>> Subject:        Re: safety gaps

>> Date sent:      Tue, 01 Sep 1998 12:09:16 PDT

>> Send reply to:  "Tesla-2 Mailing List" <tesla-2@emachine.com>

>

>> Gene wrote:

>>

>> >My intuitve approach will be to set SG gaps (carefull equal), to 'just'

>> >not fire w/NST deadheaded to SG + RFGnd..

>>

>> That sounds OK! (thats also how I do it)

>>

>> >The addition of the tank ckt may change these settings.

>>

>> Oh, I don't think I like that.  If you are getting extra safety gap

>> firings at this point, I'd just leave it, or else reduce the main gap

>> spacing.

>>

>> The first part of the adjustment sets the max safety gap spacing for the

>> given transformer voltage.  I think that's just where it should be set!

>>

>> That's what I think.  Your mileage may vary.

>> -Bill

>

>Has anyone ever considered putting a snubbing network directly across

>the main gap? If Terry Fritz is correct about the parasitic high

>frequencies when the gap ceases to conduct, then why allow such

>spikes to reach beyond the gap?

>

>Malcolm

Malcolm,

Im sure that would be of benefit but how much?

Im not sure we even understand exactly what it is

that kills neons. Many assume its RF energy sneaking

back into the neons but maybe we are over analyzing

the situation.

My feeling is that more neons are lost to simple

overvoltage more than anything else. This comes

from spacing the main gap too wide. Many, especially

beginners, just cant resist opening the gap further and

further because it results in more output. And it does.

At least until your cap and/or transformer blows up.

A lot of discussion about various chokes and filter circuits

occurs almost daily but I dont believe its ever been

demonstrated that the RF is the culprit its made out to be.

Common sense dictates that it would be wise to keep the RF out

but maybe all that needs to be done is to limit the voltage

whether its 60hz or RF to a safe value.

Ive had very good luck with my neons. I have never killed

a neon in coil usage. Maybe its just luck or maybe its

my conservative approach to my gap. Ive alway advocated

a maximum of .250 inch for 12kv and .300 inch for 15kv.

I have never used more than .225 inch myself. I think Ive

done quite well. A maximum output of 54 inches at a little

over 1.2KVA on a 4.5 X 21 inch secondary.

Mike Hammer

mhammer@misslink.net

From: "Kent Rankin" <kentrankin@theatreorgans.com>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: What should the Tesla Coil Newsgroup be named?

Date: Tue, 1 Sep 1998 17:56:00 -0400

X-Mailer: IPAD 2.0

X-MimeOLE: Produced By Microsoft MimeOLE V4.72.2106.4

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Okay guys, I'm going to post the request for a tesla-coil/weird science

newsgroup sometime within the next day or so.  I'd like to see it include

all of these HV devices(allthough, it would be nice to see it contain all

weird science type of stuff), but the name is the thing.  Do tell me what

you think that it should be named.

By the way, if it is successful in the alt domain, then I plan to move it

to the sci domain in about 3-6 months.

Thanks in advance,

Kent Rankin

(Editors note. The Tesla coil Usenet newsgroup mentioned in the above

post never happened.)

From: "Choad" <choad69@geocities.com>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Electromagnetic blowout coils & Spark gaps

Date: Tue, 1 Sep 1998 16:56:15 -0500

X-Mailer: IPAD 2.0

X-MimeOLE: Produced By Microsoft MimeOLE V4.72.3110.3

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

All,

I found somthing that might be able to be used in a spark gap to aid

in better quenching. I scanned a pic and it's at

http://www.geocities.com/SouthBeach/Pier/2756/qarc.jpg

This is a method used in motor start contactors to quench the arc while turning

the motor off. I was just wonding if there might be a way to adapt this to a spark

gap & would it do much of any good? Something like this might be more mess then it's

worth but I thought I'd try to get a little feedback on it from you guys.

L8r

Choad

From:          "Malcolm Watts" <MALCOLM@directorate.wnp.ac.nz>

Organization:  Wellington Polytechnic, NZ

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Date:          Wed, 2 Sep 1998 10:33:38 +1200

Subject:       Re: safety gaps

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

OK,

Here are the up-to-date thinks on the problem:

> Date sent:      Tue, 01 Sep 1998 16:49:31 -0500

> To:             "Tesla-2 Mailing List" <tesla-2@emachine.com>

> From:           Mike Hammer <mhammer@misslink.net>

> Subject:        Re: safety gaps

> Send reply to:  "Tesla-2 Mailing List" <tesla-2@emachine.com>

<snip>

> >Has anyone ever considered putting a snubbing network directly across

> >the main gap? If Terry Fritz is correct about the parasitic high

> >frequencies when the gap ceases to conduct, then why allow such

> >spikes to reach beyond the gap?

> >

> >Malcolm

>

>

> Malcolm,

> Im sure that would be of benefit but how much?

> Im not sure we even understand exactly what it is

> that kills neons. Many assume its RF energy sneaking

> back into the neons but maybe we are over analyzing

> the situation.

IMHO all it amounts to is that the damaging mechanism has not yet been

defined properly. Actually, I think it now has and is begging for a

more elegant solution than the usual arbitrary value chokes etc.

From my observations of safety gap operation it seemed to me that

I was observing energy sources trapped between the main gap and the

transformer forcing elevated high frequency rings back to the

transformer terminals. Energies were low but voltages were enormous.

Then it became apparent during Terry Fritz's wide bandwidth

probing of primary current that there were high frequency rings of

considerable amplitude going on during gap zero crossing periods.

I wondered why it could be that the gap didn't conduct when the

amplitude of these rings clearly exceeded the gap firing voltage.

However, a mechanism does exist, at least in theory. The frequency of

the rings is in the 10's of Mhz region. Looking at the oscillation

period it became clear that the ion transition times in the gap are

of the same order - in other words, there comes a time when a spark

gap is no longer useful at very high frequencies.

Suppose #1 above is correct. Then eliminating all long lines and

energy storage components between the main gap and the transformer

should get rid of the problem. It might or it might not. That has yet

to be conclusively proved. If it is correct, then this simple step

solves the problem and renders a safety gap unnecessary (if you don't

believe me, check the equivalent circuit of two gaps in parallel with

no impedances between them).

Suppose instead #2 is correct. Then it seems clear that some

form of energy absorption is required somewhere between the

transformer and gap. Current thinking on this suggests inclusion of

an RC low pass filter where one might previously have had chokes.

BUT, there is huge problem with this approach. There is a necessary

degree of power wastage in the series resistors which makes an

already inefficient supply (NST) worse. There is an obvious solution:

the use of a snubber across the main gap. This in no way interferes

with charging currents and if designed using good quality components

designed to have low ESRs at high frequencies, should be highly

effective. This soluton is taken straight out of the switchmode power

supply book where such problems also exist in some topologies.

> My feeling is that more neons are lost to simple

> overvoltage more than anything else. This comes

> from spacing the main gap too wide. Many, especially

> beginners, just cant resist opening the gap further and

> further because it results in more output. And it does.

> At least until your cap or transformer blows up.

Agree. But it would be satisfying to know why a safety gap set to a

higher firing voltage fires when the main gap is also apparently in

parallel with it. ??

> A lot of discussion about various chokes and filter circuits

> occurs almost daily but I dont believe its ever been

> demonstrated that the RF is the culprit its made out to be.

> Common sense dictates that it would be wise to keep the RF out

> but maybe all that needs to be done is to limit the voltage

> whether its 60hz or RF to a safe value.

>

> Ive had very good luck with my neons. I have never killed

> a neon in coil usage. Maybe its just luck or maybe its

> my conservative approach to my gap. Ive alway advocated

> a maximum of .250 inch for 12kv and .300 inch for 15kv.

> I have never used more than .225 inch myself. I think Ive

> done quite well. A maximum output of 54 inches at a little

> over 1.2KVA on a 4.5 X 21 inch secondary.

> Mike Hammer

> mhammer@misslink.net

I didn't either until I started investigating the nature of

kickbacks. I would like to know so I can design a coil system and

know it's going to last. This is done in every other sphere of

engineering and I see no good reason why it cannot be done with a

system incorporating a NST. Knowing what the mechanisms are that are

killing these is 90% of the solution. It's worth remembering that if

machanism #2 is the culprit, a safety gap is not a guarantee. It's

also worth remembering that SGs behave differently on different

systems.

Regards,

Malcolm

From: ESchulz531@aol.com

Date: Tue, 1 Sep 1998 18:46:06 EDT

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Is this how it works?

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Roy,

>> How is the multilayered primary coil wired??  Is each layer in series with

>> the others?

I wired it in series.  In series inductance is additive, but in parallel its

1 / L = 1 / L2 + 1 / L3 + 1 / L4 ...

Erik Schulz

http://members.aol.com/tesla00/

X-Mailer: IPAD 2.0

Date: Tue, 01 Sep 1998 23:58:53 +0100

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: bob golding <yubba@clara.net>

Subject: Re: safety gaps

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

At 09:27 02/09/98 +1200, you wrote:

>

>Has anyone ever considered putting a snubbing network directly across

>the main gap? If Terry Fritz is correct about the parasitic high

>frequencies when the gap ceases to conduct, then why allow such

>spikes to reach beyond the gap?

Hi malcolm,

I seem to recall someone, Daniel? tristram? frying their neon while trying

out a snubber network. They might have been using it instead of a

choke/series resistor. Any idea of component values,or how to work out what

values would be needed. Should one select for max gap? halfway? or does it

not matter.

bob golding

bob yubba@clara.net

From:          "Malcolm Watts" <MALCOLM@directorate.wnp.ac.nz>

Organization:  Wellington Polytechnic, NZ

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Date:          Wed, 2 Sep 1998 11:03:52 +1200

Subject:       Re: Is this how it works?

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Not quite:

> Roy,

>

> >> How is the multilayered primary coil wired??  Is each layer in series with

> >> the others?

>

>     I wired it in series.  In series inductance is additive, but in parallel its

> 1 / L = 1 / L2 + 1 / L3 + 1 / L4 ...

>

> Erik Schulz

> http://members.aol.com/tesla00/

That only applies if there is no mutual coupling between the

inductances. If the coupling constant approaches 1 as it does in this

situation, the only difference is a reduced conductor resistance (and

consequently higher Q). Same deal as Litz wire to a large degree.

Malcolm

From:          "Malcolm Watts" <MALCOLM@directorate.wnp.ac.nz>

Organization:  Wellington Polytechnic, NZ

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Date:          Wed, 2 Sep 1998 11:15:18 +1200

Subject:       Re: safety gaps

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Hi Bob,

What were the exact circumstances?

>

> At 09:27 02/09/98 +1200, you wrote:

> >

> >Has anyone ever considered putting a snubbing network directly across

> >the main gap? If Terry Fritz is correct about the parasitic high

> >frequencies when the gap ceases to conduct, then why allow such

> >spikes to reach beyond the gap?

>

> Hi malcolm,

>     I seem to recall someone, Daniel? tristram? frying their neon while trying

> out a snubber network. They might have been using it instead of a

> choke/series resistor. Any idea of component values,or how to work out what

> values would be needed. Should one select for max gap? halfway? or does it

> not matter.

>

> bob golding

I think in any case one shouldn't open the main gap beyond what the

transformer can fire with no load applied. That is asking for trouble.

Component values? I expect them to be different for every different

coil. It is an energy related thing employing the most useful 0.5CV^2

relation. One first has to know the magnitude of energy one is

dealing with, then choose a cap such that that amount of energy is

incapable of charging it up beyond a nominated voltage. The resistor

associated with a snubber is used to dissipate this energy as heat.

No hard and fast answers sorry. It would take a high voltage probe and

scope to determine optimum values until a formula for doing this

could be found. Suck it and see approaches are definitely dodgy.

Nothing is guaranteed if one simply bangs in a couple of arbitrary

values and I feel that this approach is apt to lull one into a false

sense of security (until the NST dies that is). I will scope a system

and have a go at designing a network for it when time permits.

Malcolm

X-Mailer: IPAD 2.0

Date: Tue, 01 Sep 1998 20:40:59 -0500

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: safety gaps

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

At 11:15 AM 9/2/98 +1200, you wrote:

>Hi Bob,

>          What were the exact circumstances?

>

>

>> At 09:27 02/09/98 +1200, you wrote:

>> >

>> >Has anyone ever considered putting a snubbing network directly across

>> >the main gap? If Terry Fritz is correct about the parasitic high

>> >frequencies when the gap ceases to conduct, then why allow such

>> >spikes to reach beyond the gap?

>>

>> Hi malcolm,

>>     I seem to recall someone, Daniel? tristram? frying their neon while trying

>> out a snubber network. They might have been using it instead of a

>> choke/series resistor. Any idea of component values,or how to work out what

>> values would be needed. Should one select for max gap? halfway? or does it

>> not matter.

>>

>> bob golding

>

>I think in any case one shouldn't open the main gap beyond what the

>transformer can fire with no load applied. That is asking for trouble.

>Component values? I expect them to be different for every different

>coil. It is an energy related thing employing the most useful 0.5CV^2

>relation. One first has to know the magnitude of energy one is

>dealing with, then choose a cap such that that amount of energy is

>incapable of charging it up beyond a nominated voltage. The resistor

>associated with a snubber is used to dissipate this energy as heat.

>No hard an fast answers sorry. It would take a high voltage probe and

>scope to determine optimum values until a formula for doing this

>could be found. Suck it and see approaches are definitely dodgy.

>Nothing is guaranteed if one simply bangs in a couple of arbitrary

>values and I feel that this approach is apt to lull one into a flase

>sense of security (until the NST dies that is). I will scope a system

>and have a go at designing a network for it when time permits.

>

>Malcolm

Malcolm,

While I have you thinking on this subject (you WERE just going

to listen right?) I want to ask about something Ive been thinking

about a great deal. Why is that some that are using damping resistors

in conjunction with their RF chokes incinerate them while others

have no such problems?

Four different coils and coilers come to mind when I think about this.

One of the four is myself of course. Ive used some 310 ohm 50 watt

resistors in conjunction with 2.25 mh toroidal RF chokes. After

a 2 minute run at 1.2 KVA (long for me) they are stone cold. After

30 minutes of on/off activity they are barely warm.

Another coiler who has a similar coil to mine but at lower

power (360va) uses some air core chokes on 2 inch PVC and

2 watt (yes  TWO) 330 ohm resistors. He reports they get warm

after some long runs.

On another coil similar to the one above except using

a single 15kv/30 has completely fried a pair of 500 ohm

25 watt resistors.

Gary Weaver has quit using resistors at all on his coils

because he keeps burning them up.

Clearly something is going on in these two coils.

And many others.

The two coils that keep burning up resistors have also

killed a number of neons. The two coils that have happy

resistors have lost exactly zero neons.

Where is all this extra current flow coming from?

Clearly its RF currents but how is it being generated

and why do I not have the same problem?

I dont get a chance to ask a question very often anymore.

TTYL!

Mike Hammer

mhammer@misslink.net

Date: Tue, 01 Sep 1998 21:02:59 -0500

From: Bert Hickman <bert.hickman@aquila.com>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Organization: Stoneridge Engineering

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Line Filters

Sender: tesl-err@emachine.com

Steve B. wrote:

>

> Hi Guys,

>         When working with a single phase supply, can the 2 unused phases on a

> 3-phase line filter be used for extra filtering by wiring them all in

> series?

>

> TIA

>

> Steve Bell  (steve@breakfix.demon.co.uk)

> Stuff also available at www.breakfix.demon.co.uk

Steve,

I've never tried this, but I can see no reason in theory why it

shouldn't work! Let us know how well it works out!

-- Bert --

From: Pyrotrons@aol.com

Date: Tue, 1 Sep 1998 22:19:56 EDT

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: getting started

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Hi I'm Justin Hays, and I'm 17.  Glad to hear you're starting on a coil!

For me to help you out, I'll need to know how long of an output spark you

want.  Tesla coil output spark lengths are almost always based on the length

and diameter of the secondary coil.  My friend has a coil thats secondary is

about 4.5" in diameter x 20" long.  Sparks out of it reach to about 2 feet. I

recommend this coil for beginners.

Mail me back with your answer and some more questions!  Some things you could

be looking for in the meantime include places to buy 1/4" copper thinwall

tubing, a NST (neon sign transformer), PVC pipe, and hundreds of feet of 24

gauge enameled wire.

Good luck.....justin

From:          "Malcolm Watts" <MALCOLM@directorate.wnp.ac.nz>

Organization:  Wellington Polytechnic, NZ

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Date:          Wed, 2 Sep 1998 14:32:30 +1200

Subject:       Re: safety gaps

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Hi Mike,

> Date sent:      Tue, 01 Sep 1998 20:40:59 -0500

> To:             "Tesla-2 Mailing List" <tesla-2@emachine.com>

> From:           Mike Hammer <mhammer@misslink.net>

> Subject:        Re: safety gaps

> Send reply to:  "Tesla-2 Mailing List" <tesla-2@emachine.com>

<snip>

> While I have you thinking on this subject (you WERE just going

> to listen right?) I want to ask about something Ive been thinking

> about a great deal. Why is that some that are using damping resistors

> in conjunction with their RF chokes incinerate them while others

> have no such problems?

Let's find out. First thing: what is the value of the primary cap

for each system? In fact, let's go one better and tabulate the

systems for comparison (when filling in the blanks, don't use the

reply marker so the tables aren't tabbed to the right):

#1          #2           #3            #4

Transformer =                           15kV/30mA

Gap Type =

Total gap =

Primary cap =

Resistors =     310 Ohm     330 Ohm      1000 Ohm

Choke value =   4.5mH

BPS =

Resistor Heat = COLD

I think we are not concerned with wallplug power (most people don't

know it anyway and simply guess based on transformer rating).

> Four different coils and coilers come to mind when I think about this.

> One of the four is myself of course. Ive used some 310 ohm 50 watt

> resistors in conjunction with 2.25 mh toroidal RF chokes. After

> a 2 minute run at 1.2 KVA (long for me) they are stone cold. After

> 30 minutes of on/off activity they are barely warm.

>

> Another coiler who has a similar coil to mine but at lower

> power (360va) uses some air core chokes on 2 inch PVC and

> 2 watt (yes  TWO) 330 resistors. He reports they get warm

> after some long runs.

>

> On another coil similar to the one above except using

> a single 15kv/30 has completely fried a pair of 500 ohm

> 25 watt resistors.

>

> Gary Weaver has quit using resistors at all on his coils

> because he keeps burning them up.

>

> Clearly something is going on in these two coils.

> And many others.

>

> The two coils that keep buring up resistors have also

> killed a number of neons. The two coils that have happy

> resistors have lost exactly zero neons.

>

> Where is all this extra current flow coming from?

> Clearly its RF currents but how is it being generated

> and why do I not have the same problem?

>

> I dont get a chance to ask a question very often anymore.

Is it clearly RF currents?  We know next to nothing about the

charging currents for a start. There are lots of blanks in the table

above. Let's do a little sleuthing and see if a pattern pops up out

of the mix.

Malcolm

X-Mailer: IPAD 2.0

Date: Wed, 02 Sep 1998 00:24:31 -0500

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: safety gaps

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

At 02:32 PM 9/2/98 +1200, you wrote:

>Hi Mike,

>

>> Date sent:      Tue, 01 Sep 1998 20:40:59 -0500

>> To:             "Tesla-2 Mailing List" <tesla-2@emachine.com>

>> From:           Mike Hammer <mhammer@misslink.net>

>> Subject:        Re: safety gaps

>> Send reply to:  "Tesla-2 Mailing List" <tesla-2@emachine.com>

>

><snip>

>

>> While I have you thinking on this subject (you WERE just going

>> to listen right?) I want to ask about something Ive been thinking

>> about a great deal. Why is that some that are using damping resistors

>> in conjunction with their RF chokes incinerate them while others

>> have no such problems?

>

>Let's find out. First thing: what is the value of the primary cap[

>for each system? In fact, let's go one better and tabulate the

>systems for comparison (when filling in the blanks, don't use the

>reply marker so the tables aren't tabbed to the right):

Malcolm,

I did the best I could with the data.

Coil #1 is of course mine. #2 belongs to Corey Edmunds.

#3 is a local coiler. #4 is Gary Weaver but I dont know the

particulars of his coil. I emailed him for info.

>

#1          #2             #3             #4

Transformer =   12kv/100ma    12kv/30ma      15kV/30mA       unknown

Gap Type =       Vacuum       flat cu pipe   RQ cylinder

Total gap =      .225         .225           .300

Primary cap =    .015        . 0056          .0045

Resistors =     310 ohm 50W   330 Ohm  2W   500 Ohm 25W

Choke value =   2.25mH        4.1 mh         2.1mh

BPS =            120?          ?              ?

Resistor Heat = COLD          warm           burnt

>

>I think we are not concerned with wallplug power (most people don't

>know it anyway and simply guess based on transformer rating).

>

>

>Is it clearly RF currents?  We know next to nothing about the

>charging currents for a start. There are lots of blanks in the table

>above. Let's do a little sleuthing and see if a pattern pops up out

>of the mix.

Well if its not the charging current then it

must be RF. Some coils may have some 60 hz resonance

in the charging circuit so that could have an effect.

But I never had any problems even when I had 60 hz resonance.

TTYL!

Mike Hammer

mhammer@misslink.net

From:          "Malcolm Watts" <MALCOLM@directorate.wnp.ac.nz>

Organization:  Wellington Polytechnic, NZ

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Date:          Wed, 2 Sep 1998 17:54:37 +1200

Subject:       Re: safety gaps

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Hi Mike,

Thanks for the data so far but there are gaps :(

> Date sent:      Wed, 02 Sep 1998 00:24:31 -0500

> To:             "Tesla-2 Mailing List" <tesla-2@emachine.com>

> From:           Mike Hammer <mhammer@misslink.net>

> Subject:        Re: safety gaps

> Send reply to:  "Tesla-2 Mailing List" <tesla-2@emachine.com>

<snip>

>  I did the best I could with the data.

> Coil #1 is of course mine. #2 belongs to Corey Edmunds.

> #3 is a local coiler. #4 is Gary Weaver but I dont know the

> particulars of his coil. I emailed him for info.

>

> >

>                    #1          #2             #3             #4

>

> Transformer =   12kv/100ma    12kv/30ma      15kV/30mA       unknown

> Gap Type =       Vacuum       flat cu pipe   RQ cylinder

> Total gap =      .225         .225           .300

> Primary cap =    .015        . 0056          .0045

> Resistors =     310 ohm 50W   330 Ohm  2W   500 Ohm 25W

> Choke value =   2.25mH        4.1 mh         2.1mh

> BPS =            120?          ?              ?

> Resistor Heat = COLD          warm           burnt

Unfortunately "vacuum" says nothing about the gap really. I am

talking number of series gaps etc. The gap settings all seem

reasonable. I think we can say that the warm resistor has to be

because of its low power rating. There doesn't seem to be a great

deal of correlation between chokes and resistor heat so far. I think

chokes might not figure in this but we really need more data to form

a definite conclusion. Did any of the coils have RC filters as well?

Sorry for not asking before. Do all coils strike targets regularly or

do they mostly produce air streamers (has quench implications)?  Now

operating frequencies might be interesting to add to the mix. Is

there a correlation between the hot/cold resistors and final

performance? The BPS figure really needs to be known.

Your coil appears to have a non-resonant supply (22nF required)

and is a long way off. Coil #2 doesn't seem to be resonant either

(6.6nF odd) although it's not far off. Same with coil #3 (5.3nF)

although again, it's close. In fact both coils #2 and #3 would be

about the same degree off resonance yet one is cooking while the

other is not. Not enough data yet but it is beginning to look as

though the TC itself might be the real culprit. Here is where its

various parameters start to assume importance.

There might be an easy way of determining such a thing. Instead of

using a TC as a load for the charging circuit, a resistor of a

suitably high power rating to handle E*BPS could do the job. Its

value would be such that it drains the primary cap in the time taken

for two pri-sec energy transfers to approximate loading caused by air

streamers. I'd be interested in doing this myself.

> >I think we are not concerned with wallplug power (most people don't

> >know it anyway and simply guess based on transformer rating).

> >

> >

> >Is it clearly RF currents?  We know next to nothing about the

> >charging currents for a start. There are lots of blanks in the table

> >above. Let's do a little sleuthing and see if a pattern pops up out

> >of the mix.

>

> Well if its not the charging current then it

> must be RF. Some coils may have some 60 hz resonance

> in the charging circuit so that could have an effect.

> But I never had any problems even when I had 60 hz resonance.

Any ideas from anyone else? I suppose this discussion is better

suited to the pupman list but wouldn't it be nice to know the

answers?

Malcolm

X-Mailer: IPAD 2.0

Date: Wed, 02 Sep 1998 01:32:38 -0500

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: safety gaps

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

At 05:54 PM 9/2/98 +1200, you wrote:

>Hi Mike,

>          Thanks for the data so far but there are gaps :(

>

>> Date sent:      Wed, 02 Sep 1998 00:24:31 -0500

>> To:             "Tesla-2 Mailing List" <tesla-2@emachine.com>

>> From:           Mike Hammer <mhammer@misslink.net>

>> Subject:        Re: safety gaps

>> Send reply to:  "Tesla-2 Mailing List" <tesla-2@emachine.com>

>

><snip>

>>  I did the best I could with the data.

>> Coil #1 is of course mine. #2 belongs to Corey Edmunds.

>> #3 is a local coiler. #4 is Gary Weaver but I dont know the

>> particulars of his coil. I emailed him for info.

>>

>> >

>>                    #1          #2             #3             #4

>>

Transformer =   12kv/100ma    12kv/30ma      15kV/30mA       unknown

Gap Type =       9@.025       9@.025         10@.03

Total gap =      .225         .225           .300

Primary cap =    .015        . 0056          .0045

Resistors =     310 ohm 50W   330 Ohm  2W   500 Ohm 25W

Choke value =   2.25mH        4.1 mh         2.1mh

BPS =            120?          ?              ?

Resistor Heat = COLD          warm           burnt

Res Freq  =     254 khz       ~275khz        323khz

Malcolm,

I have supplied a little more info on the gaps as requested.

Hopefully I can get some info from Gary Weaver on his coil as well.

>

>Unfortunately "vacuum" says nothing about the gap really. I am

>talking number of series gaps etc. The gap settings all seem

>reasonable. I think we can say that the warm resistor has to be

>because of its low power rating. There doesn't seem to be a great

>deal of correlation between chokes and resistor heat so far. I think

>chokes might not figure in this but we really need more data to form

>a definite conclusion. Did any of the coils have RC filters as well?

No RC filters. These 3 coils are all very similar as I guided

the owners in their construction. I never recommended bypass

caps as I quickly fried a neon once that way.

>Sorry for not asking before. Do all coils strike targets regularly or

>do they mostly produce air streamers (has quench implications)?

Mine is mostly air streamers and occasional targets. I have

first notch quenching with the vacumm gap. I know Corey

(#2) does most air streamers but dont know about #3.

>Now operating frequencies might be interesting to add to the mix.

My operating frequency is 254 khz. Coil # 2 is 275 khz

and coil number 3 is 323 khz.

Frequency of mine and coil number 3 are measured.

#2 is calculated.

>Is there a correlation between the hot/cold resistors and final

>performance? The BPS figure really needs to be known.

Not that I see. Both of the other coils seem to be typical

performers for their power level.

As the gaps are all static the BPS figure is likely to be 120.

I dont think any of these have sufficient transformer to

allow 240 BPS. Im pretty sure mine doesnt. The gap sound

is decidedly different at 240 bps.

>     Your coil appears to have a non-resonant supply (22nF required)

>and is a long way off. Coil #2 doesn't seem to be resonant either

>(6.6nF odd) although it's not far off. Same with coil #3 (5.3nF)

>although again, it's close. In fact both coils #2 and #3 would be

>about the same degree off resonance yet one is cooking while the

>other is not. Not enough data yet but it is beginning to look as

>though the TC itself might be the real culprit. Here is where its

>various parameters start to assume importance.

>    There might be an easy way of determining such a thing. Instead of

>using a TC as a load for the charging circuit, a resistor of a

>suitably high power rating to handle E*BPS could do the job. Its

>value would be such that it drains the primary cap in the time taken

>for two pri-sec energy transfers to approximate loading caused by air

>streamers. I'd be interested in doing this myself.

>

TTYL!

Mike Hammer

mhammer@misslink.net

From: "Tobin Fricke" <fricke@mae.engr.ucdavis.edu>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: getting started

Date: Tue, 1 Sep 1998 23:41:49 -0700

X-Mailer: IPAD 2.0

X-MimeOLE: Produced By Microsoft MimeOLE V4.72.3110.3

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

>4.5" in diameter x 20" long.  Sparks out of it reach to about 2 feet.

Two feet is good.  My goals are to (a) build a nice tesla coil that produces

visibly impressive results (b) learn about tesla coils (c) not kill myself

and (d) try to do it for <$200, but that's flexible.  I live in Berkeley, CA

if that helps..

Tobin

Date: Wed, 2 Sep 1998 06:33:14 -0400 (EDT)

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: fxphoto@centuryinter.net (fxphoto)

Subject: Re: safety gaps

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Hi Malcolm and Mike,

My ideas unfortunately come as a large number of questions so I will try

to limit them to the most obvious component...the resistor.

1. composition: is it carbon, wirewound, or other? (possible inductance problem)

2. physical size: is it big or small

3  lead size: thin wire or metal tab

4  lead attachment: welded, riveted etc (possible high resistance at joint)

5  physical condition: any fine cracks or chips? (possible internal arcing )

6  where is it: between nst and choke, between choke and cap?

7  Has the resistance been checked at the operating frequency versus dc.

8  has the resistor been checked for operation at the voltage used?

( is there a difference in an arc to ground from the nst with the resistor vs without?)

9  is there any corona around the resistor during operation.

10 are they single resistors i.e. one on each side or are they in parallel or

series in order to make the correct resistance or wattage.

I notice that the one coil with the burned resistor is running 15kv . The

owner might try using two or three resistors in series to split up the

voltage drop and see if that improves anything. Same idea as your series

caps, Mike.

>

>>                    #1          #2             #3             #4

>>

>> Transformer =   12kv/100ma    12kv/30ma      15kV/30mA       unknown

>> Gap Type =       Vacuum       flat cu pipe   RQ cylinder

>> Total gap =      .225         .225           .300

>> Primary cap =    .015        . 0056          .0045

>> Resistors =     310 ohm 50W   330 Ohm  2W   500 Ohm 25W

>> Choke value =   2.25mH        4.1 mh         2.1mh

>> BPS =            120?          ?              ?

>> Resistor Heat = COLD          warm           burnt

>

>Any ideas from anyone else? I suppose this discussion is better

>suited to the pupman list but wouldn't it be nice to know the

>answers?

>

>Malcolm

>

>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Date: Wed, 02 Sep 1998 04:14:35 -0700

From: "paul m" <fleischwulf@mailexcite.com>

Cc:

X-Sent-Mail: off

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

Subject: Re: getting started

X-Sender-Ip: 208.153.7.9

Organization: MailExcite  (http://www.mailexcite.com:80)

Sender: tesl-err@emachine.com

Tobin,

I built my system for about $150.00 minus the transformer. I have four .026 uF caps which I built for about $25.00 a piece. The primary cost about $50.00. Most of the cost was in the plastic componets. You should be able to buy old nst's for around $20.00. Recently I purchased 4)15 kv 30 ma. transformers. Two of these would give you plenty

of power.

paul m.

---

Einigheit und Wahrheit

On Tue, 1 Sep 1998 23:41:49    Tobin Fricke wrote:

>>4.5" in diameter x 20" long.  Sparks out of it reach to about 2 feet. I

>

>

>Two feet is good.  My goals are to (a) build a nice tesla coil that produces

>visibly impressive results (b) learn about tesla coils (c) not kill myself

>and (d) try to do it for >$200, but that's flexible.  I live in Berkeley, CA

>if that helps..

>

>Tobin

>

Date: Wed, 02 Sep 1998 06:58:39 -0500

From: Bert Hickman <bert.hickman@aquila.com>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Organization: Stoneridge Engineering

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: safety gaps

Sender: tesl-err@emachine.com

Malcolm Watts wrote:

>

> Hi Mike,

>           Thanks for the data so far but there are gaps :(

>

> > Date sent:      Wed, 02 Sep 1998 00:24:31 -0500

> > To:             "Tesla-2 Mailing List" <tesla-2@emachine.com>

> > From:           Mike Hammer <mhammer@misslink.net>

> > Subject:        Re: safety gaps

> > Send reply to:  "Tesla-2 Mailing List" <tesla-2@emachine.com>

>

> <snip>

> >  I did the best I could with the data.

> > Coil #1 is of course mine. #2 belongs to Corey Edmunds.

> > #3 is a local coiler. #4 is Gary Weaver but I dont know the

> > particulars of his coil. I emailed him for info.

> >

> > >

> >                    #1          #2             #3             #4

> >

> > Transformer =   12kv/100ma    12kv/30ma      15kV/30mA       unknown

> > Gap Type =       Vacuum       flat cu pipe   RQ cylinder

> > Total gap =      .225         .225           .300

> > Primary cap =    .015        . 0056          .0045

> > Resistors =     310 ohm 50W   330 Ohm  2W   500 Ohm 25W

> > Choke value =   2.25mH        4.1 mh         2.1mh

> > BPS =            120?          ?              ?

> > Resistor Heat = COLD          warm           burnt

>

> Unfortunately "vacuum" says nothing about the gap really. I am

> talking number of series gaps etc. The gap settings all seem

> reasonable. I think we can say that the warm resistor has to be

> because of its low power rating. There doesn't seem to be a great

> deal of correlation between chokes and resistor heat so far. I think

> chokes might not figure in this but we really need more data to form

> a definite conclusion. Did any of the coils have RC filters as well?

> Sorry for not asking before. Do all coils strike targets regularly or

> do they mostly produce air streamers (has quench implications)?  Now

> operating frequencies might be interesting to add to the mix. Is

> there a correlation between the hot/cold resistors and final

> performance? The BPS figure really needs to be known.

>      Your coil appears to have a non-resonant supply (22nF required)

> and is a long way off. Coil #2 doesn't seem to be resonant either

> (6.6nF odd) although it's not far off. Same with coil #3 (5.3nF)

> although again, it's close. In fact both coils #2 and #3 would be

> about the same degree off resonance yet one is cooking while the

> other is not. Not enough data yet but it is beginning to look as

> though the TC itself might be the real culprit. Here is where its

> various parameters start to assume importance.

>     There might be an easy way of determining such a thing. Instead of

> using a TC as a load for the charging circuit, a resistor of a

> suitably high power rating to handle E*BPS could do the job. Its

> value would be such that it drains the primary cap in the time taken

> for two pri-sec energy transfers to approximate loading caused by air

> streamers. I'd be interested in doing this myself.

<SNIP>

>

> Any ideas from anyone else? I suppose this discussion is better

> suited to the pupman list but wouldn't it be nice to know the

> answers?

>

> Malcolm

Malcolm, Mike, and all,

Some thoughts:

Perhaps there's overvolting and subsequent internal arcing (hot spots)

under transient conditions? I notice that the third coil is using a

higher supply voltage, a higher resistance value, and a lower wattage

(25W) wirewound resistor. The 3rd system and the 2nd one are closer (65

Hz) to resonant charging than the first (73 Hz). A typical 25 watt

resistor is only 2" long, while a 50 watt one is more like 4", so I'd

expect the standoff voltage to be much higher on a 50 Watt resistor.

And, since Mike's later post indicates no bypass caps in any of the

systems, this variable is eliminated.

The 2nd system is extremely interesting, in that only a 2 Watt resistor

is used! Are these a carbon composition or wirewound? Generally,

wirewounds would be more inductive, leading to higher peak voltages

developed across them during high frequency transients. Irrespective of

the type, the voltage withstanding capability of this resistor should be

significantly less than the other two - the resistive element in a

typical 2W resistor is only about 1/2" long. Actually, I'm a bit

surprised that 2W resistors are surviving in this environment...

-- Bert --

From:          "Malcolm Watts" <MALCOLM@directorate.wnp.ac.nz>

Organization:  Wellington Polytechnic, NZ

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Date:          Thu, 3 Sep 1998 08:17:59 +1200

Subject:       Re: safety gaps

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Mike,

Thanks again for the facts and figures. Anything else you can

add would be helpful. It is almost a situation which requires me to

duplicate a hot and a cold machine and take measurements using the

storage scope etc. I agree with you about the BPS. Was your

transformer (100mA) a NST? This is a pretty puzzle.

Regards,

Malcolm

From:          "Malcolm Watts" <MALCOLM@directorate.wnp.ac.nz>

Organization:  Wellington Polytechnic, NZ

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Date:          Thu, 3 Sep 1998 08:27:48 +1200

Subject:       Re: safety gaps

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Hi there,

I'd like to comment on your points :

> Date sent:      Wed, 2 Sep 1998 06:33:14 -0400 (EDT)

> To:             "Tesla-2 Mailing List" <tesla-2@emachine.com>

> From:           fxphoto@centuryinter.net (fxphoto)

> Subject:        Re: safety gaps

> Send reply to:  "Tesla-2 Mailing List" <tesla-2@emachine.com>

> Hi Malcolm and Mike,

>     My ideas unfortunately come as a large number of questions so I will try

> to limit them to the most obvious component...the resistor.

>

> 1. composition: is it carbon, wirewound, or other? (possible inductance problem)

Inductance would be negligible beside the chokes so I think we can

rule that out. Composition is a faint possibility perhaps.

> 2. physical size: is it big or small

A 2W resistor is small and it was getting only warm beside a 50W one

that was smoking. Apart from obvious things like voltage handling

(now I wonder just how much of each resistor was effective at those

voltages??), power handling is as much a function of ability to

withstand and get rid of heat as anything else. I wonder about some

small resistors rated at 1W BTW. Sure they might withstand the power

level without disintegrating but they would get b****y hot

dissipating that power level.

> 3  lead size: thin wire or metal tab

Also related to heat removal. Depends on size of sink at terminations.

> 4  lead attachment: welded, riveted etc (possible high resistance at joint)

As significant as the resistor itself? I doubt it unless there is a

fault with the component. But then one would expect the joint to get

significantly hotter than the body, yes?

> 5  physical condition: any fine cracks or chips? (possible internal arcing )

Sort of related to composition.

> 6  where is it: between nst and choke, between choke and cap?

Equivalent circuits are no different from each other.

> 7  Has the resistance been checked at the operating frequency versus dc.

Good question. I expect there would be little change but....

> 8  has the resistor been checked for operation at the voltage used? ( is

> there a difference in an arc to ground from the nst with the resistor vs

> without?)

Excellent question.

> 9  is there any corona around the resistor during operation.

> 10 are they single resistors i.e. one on each side or are they in parallel

> or series in order to make the correct resistance or wattage.

Function of size and heat losing ability. Good question again.

> I notice that the one coil with the burned resistor is running 15kv . The

> owner might try using two or three resistors in series to split up the

> voltage drop and see if that improves anything. Same idea as your series

> caps, Mike.

But the COLD one of similar rating would be dissipoating more one

would expect (cap *much* larger). Possibly related to gap

shorting/dwell time.

Good thinks.

Malcolm

From: Pyrotrons@aol.com

Date: Wed, 2 Sep 1998 17:15:44 EDT

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: getting started

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Okay sounds great.  I'm making a coil right now that is 4.5" x 21.5" also.

200 dollars isnt too much to ask for a coil this size.  Sometimes you can get

the NST free....hit all the neon sign stores that you can find - you might be

surprised.

The first thing i did on my coil is get the spark gap done.  Get a 3 foot

long, 3/4 inch outside diamter copper pipe.  Cut this into 3" sections.  For

the spark gap you will have 11 copper sections, and that will give you 10

gaps.  This will work well for this coil. Each copper tube section is .03"

apart from the others.  This should give you something to work on for now!.

Here is a almost complete materials list for your coil.

1. One 30kV @ 30mA neon sign transformer

(editors note  He meant 15kv/30ma neon. 30 kv is way too high and 30kv

neons do not exist)

2. 8A fuse

3. s.p.s.t. 8A switch

4. one .005uF capacitor

5. .25" O.D. copper thinwall tubing roughly 60' long

6. Four 8 x 6 sheets of .25" thick plexiglass

7. One 9 x 9 sheet of plexiglass

8.  4.5 x 22.5 PVC pipe

9. 1500' of 24 ga. enameled copper wire.

10.  10' HV wire

X-Originating-IP: [199.217.140.22]

From: "Bill the arcstarter" <arcstarter@hotmail.com>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: safety gaps

Date: Wed, 02 Sep 1998 14:24:31 PDT

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

Hi Malcolm, Mike, others.

It was written:

>Inductance would be negligible beside the chokes so I think we can

>rule that out. Composition is a faint possibility perhaps.

Even on the wire-wound units?  Granted - I haven't measured this

inductance...

>> 8.  has the resistor been checked for operation at the voltage used?

>> (is there a difference in an arc to ground from the nst with the resistor vs

>> without?)

>

>Excellent question.

But somewhat hard to check since you can't just throw it across the

neon.  Example - a 2Kohm resistor placed across a NST supplying 60 ma

will only drop 2K*0.060 = 120 volts, much less that the neon's nominal

12K or 15K output. It may be the energy in the capacitor is to blame for

this heating, ie, it is discharging back through the core of the NST,

which would produce a higher voltage across the cap.

Some of this effect can be easily demonstrated - place the cap across

the neon (use with safety gaps of course), and place (the resistor in

series with a spark gap) across the capacitor.  You can surely impress

higher voltages across the resistor in question using this layout than

by placing the resistor directly across the neon's output.

Also - Mike Hammer - have you been keeping track of Terry Fritz's

argon-quenched gap?  It sounds remarkably similar to one which I

proposed to you last October!

I guess whats old is new again...  I just wish I'd built it last year!

-Bill the arcstarter

Starting arcs in Cinci, OH

http://www.geocities.com/CapeCanaveral/Hangar/6160

X-Originating-IP: [199.217.140.22]

From: "Bill the arcstarter" <arcstarter@hotmail.com>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: getting started

Date: Wed, 02 Sep 1998 14:34:57 PDT

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

It was written:

>long, 3/4 inch outside diamter copper pipe.  Cut this into 3" sections.

>For the spark gap you will have 11 copper sections, and that will give you

>10 gaps.  This will work well for this coil. Each copper tube section is .03"

I used an old hacksaw blade as a spacer (was 0.028 inches thick)!

>apart from the others.  This should give you something to work on for now!.

>Here is a almost complete materials list for your coil.

>

>1. One 30kV @ 30mA neon sign transformer

I think you meant to say a 12 or 15 KV unit.  30KV is too high and

you'll never find one.

Someone on this list was selling neons cheap:

http://home.earthlink.net/~teddp

>4. one .005uF capacitor

...which you'll have to make yourself via any number of techniques.

>5. .25" O.D. copper thinwall tubing roughly 60' long

>

>6. Four 8 x 6 sheets of .25" thick plexiglass

>

>7. One 9 x 9 sheet of plexiglass

>

>8.  4.5 x 22.5 PVC pipe

>

>9. 1500' of 24 ga. enameled copper wire.

I'd think using 22 or 20 might be better.  Comments, anyone?

Sounds good!  What is the plexiglass for?

-Bill

From:          "Malcolm Watts" <MALCOLM@directorate.wnp.ac.nz>

Organization:  Wellington Polytechnic, NZ

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Date:          Thu, 3 Sep 1998 09:40:41 +1200

Subject:       Re: safety gaps

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Hi Bill,

> From:           "Bill the arcstarter" <arcstarter@hotmail.com>

> To:             "Tesla-2 Mailing List" <tesla-2@emachine.com>

> Subject:        Re: safety gaps

> Date sent:      Wed, 02 Sep 1998 14:24:31 PDT

> Send reply to:  "Tesla-2 Mailing List" <tesla-2@emachine.com>

> Hi Malcolm, Mike, others.

> It was written:

>

> >Inductance would be negligible beside the chokes so I think we can

> >rule that out. Composition is a faint possibility perhaps.

>

> Even on the wire-wound units?  Granted - I haven't measured this

> inductance...

It is measured in units of microHenries for wirewounds. B-all turns,

no magnetic core etc.....

> >> 8  has the resistor been checked for operation at the voltage used?

> >>(is there a difference in an arc to ground from the nst with the

> >>resistor vs without?)

> >

> >Excellent question.

>

> But somewhat hard to check since you can't just throw it across the

> neon.  Example - a 2Kohm resistor placed across a NST supplying 60 ma

> will only drop 2K*0.060 = 120 volts, much less that the neon's nominal

> 12K or 15K output. It may be the energy in the capacitor is to blame for

> this heating, ie, it is discharging back through the core of the NST,

> which would produce a higher voltage across the cap.

You don't want to throw it across the neon. I think the question was

"is coil performance affected by omitting the resistor altogether?"

> Some of this effect can be easily demonstrated - place the cap across

> the neon (use with safety gaps of course), and place (the resistor in

> series with a spark gap) across the capacitor.  You can surely impress

> higher voltages across the resistor in question using this layout than

> by placing the resistor directly across the neon's output.

I don't think you'd want to do that would you?

Malcolm

X-Mailer: IPAD 2.0

Date: Wed, 02 Sep 1998 17:09:56 -0500

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: safety gaps

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

At 06:58 AM 9/2/98 -0500, you wrote:

>Malcolm Watts wrote:

>>

>> Hi Mike,

>>           Thanks for the data so far but there are gaps :(

>>

>> > Date sent:      Wed, 02 Sep 1998 00:24:31 -0500

>> > To:             "Tesla-2 Mailing List" <tesla-2@emachine.com>

>> > From:           Mike Hammer <mhammer@misslink.net>

>> > Subject:        Re: safety gaps

>> > Send reply to:  "Tesla-2 Mailing List" <tesla-2@emachine.com>

>>

>> <snip>

>> >  I did the best I could with the data.

>> > Coil #1 is of course mine. #2 belongs to Corey Edmunds.

>> > #3 is a local coiler. #4 is Gary Weaver but I dont know the

>> > particulars of his coil. I emailed him for info.

>> >

>> > >

>> >                    #1          #2             #3             #4

>> >

>> > Transformer =   12kv/100ma    12kv/30ma      15kV/30mA       unknown

>> > Gap Type =       Vacuum       flat cu pipe   RQ cylinder

>> > Total gap =      .225         .225           .300

>> > Primary cap =    .015        . 0056          .0045

>> > Resistors =     310 ohm 50W   330 Ohm  2W   500 Ohm 25W

>> > Choke value =   2.25mH        4.1 mh         2.1mh

>> > BPS =            120?          ?              ?

>> > Resistor Heat = COLD          warm           burnt

>>

>

>Malcolm, Mike, and all,

>

>Some thoughts:

>Perhaps there's overvolting and subsequent internal arcing (hot spots)

>under transient conditions? I notice that the third coil is using a

>higher supply voltage, a higher resistance value, and a lower wattage

>(25W) wirewound resistor. The 3rd system and the 2nd one are closer (65

>Hz) to resonant charging than the first (73 Hz). A typical 25 watt

>resistor is only 2" long, while a 50 watt one is more like 4", so I'd

>expect the standoff voltage to be much higher on a 50 Watt resistor.

>And, since Mike's later post indicates no bypass caps in any of the

>system, this variable is eliminated.

>

>The 2nd system is extremely interesting, in that only a 2 Watt resistor

>is used! Are these a carbon composition or wirewound? Generally,

>wirewounds would be more inductive, leading to higher peak voltages

>developed across them during high frequency transients. Irrespective of

>the type, the voltage withstanding capability of this resistor should be

>significantly less than the other two - the resistive element in a

>typical 2W resistor is only about 1/2" long. Actually, I'm a bit

>surprised that 2W resistors are surviving in this environment...

>

>-- Bert --

>

Sorry I didnt keep up with the thread today. A monitor failure

kept me offline for most of the day.

The resistors in use on Corey's system (he is on this list by the way)

are indeed carbon composition 2 watt resistors. I sent him 4 of

these intending for him to use two in series on each side.

He is using only 1 on each side with complete success.

Obviously the only thing those resistors are seeing

is the 30ma or so charging current from the neon.

Any more and they would certainly be getting hot.

These 3 coils system were all built with my help.

Thats whats is so strange about coil #3. He hasnt done

anything out of the ordinary so I fail to see what

the difference is.

The only marked difference among the 3 coils is

the primary geometry. Mine uses a slightly tapered

solenoid. Corey used a shallow (15 deg I think) saucer

primary and coil #3 used a flat spiral.

TTYL!

Mike Hammer

mhammer@misslink.net

X-Mailer: IPAD 2.0

Date: Wed, 02 Sep 1998 17:14:19 -0500

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: safety gaps

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

At 08:17 AM 9/3/98 +1200, you wrote:

>Mike,

>        Thanks again for the facts and figures. Anything else you can

>add would be helpful. It is almost a situation which requires me to

>duplicate a hot and a cold machine and take measurements using the

>storage scope etc. I agree with you about the BPS. Was your

>transformer (100mA) a NST? This is a pretty puzzle.

>

>Regards,

>Malcolm

Malcolm,

I will try to get the figures for the 4th coil as well

as soon as I can talk to Gary Weaver.

My transformer is indeed a rebuilt neon. It started

as a dead France 15kv/60 and has been resurrected as

a 12kv/100ma in oil.

There are a couple pictures of it on my web page at:

http://www.misslink.net/mhammer/rebuild.htm

Its seems pretty much indestructible. (so far)

TTYL!

Mike Hammer

mhammer@misslink.net

X-Mailer: IPAD 2.0

Date: Wed, 02 Sep 1998 17:21:38 -0500

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: safety gaps

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

At 02:24 PM 9/2/98 PDT, you wrote:

>Hi Malcolm, Mike, others.

>It was written:

<SNIP>

>Also - Mike Hammer - have you been keeping track of Terry Fritz's

>argon-quenched gap?  It sounds remarkably similar to one which I

>proposed to you last October!

>

>I guess whats old is new again...  I just wish I'd built it last year!

>

>-Bill the arcstarter

>Starting arcs in Cinci, OH

>http://www.geocities.com/CapeCanaveral/Hangar/6160

>

Bill,

Yes in fact I talked to Terry about it on IRC last night.

I do indeed remember you asking about that earlier.

I believe that I told you argon wouldnt be a proper gas for

use in a spark gap. I was kind of wrong on that one it seems.

Terry's approach is also very similar to the old

spark radio transmitter quench gaps. 15 or so years

ago an old time ham I knew tried to give me a Telefunken

quench gap he had. Not really knowing the significance

of what it was I declined. He is long gone now and

all that old stuff of his is probably in some landfill. :(

TTYL!

Mike Hammer

mhammer@misslink.net

X-Mailer: IPAD 2.0

Date: Wed, 02 Sep 1998 17:36:09 -0500

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: getting started

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

At 02:34 PM 9/2/98 PDT, you wrote:

>It was written:

>

<SNIP>

>>8.  4.5 x 22.5 PVC pipe

>>

>>9. 1500' of 24 ga. enameled copper wire.

>

>I'd think using 22 or 20 might be better.  Comments, anyone?

>

>

>-Bill

>

Been there done both. I have two virtually identical

4.5 inch secondaries. The first was 4.5 X 22 with AWG 22.

Worked good. I pretty much maxed it at 44 inches.

The replacement for it was 4.5 X 21 with AWG 24.

Works much better and has allowed me to get up to 54 inches

of spark. I believe the difference is mostly due to the lower

resonant frequency allowing me to keep more primary inductance

tapped in.

Once you get to a 6 inch secondary I would think going

to AWG 22 would be best. But I usually recommend AWG 24

for a 4 incher.

TTYL!

Mike Hammer

mhammer@misslink.net

From:          "Malcolm Watts" <MALCOLM@directorate.wnp.ac.nz>

Organization:  Wellington Polytechnic, NZ

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Date:          Thu, 3 Sep 1998 10:49:30 +1200

Subject:       Re: safety gaps

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Hi Mike,

> Date sent:      Wed, 02 Sep 1998 17:09:56 -0500

> To:             "Tesla-2 Mailing List" <tesla-2@emachine.com>

> From:           Mike Hammer <mhammer@misslink.net>

> Subject:        Re: safety gaps

> Send reply to:  "Tesla-2 Mailing List" <tesla-2@emachine.com>

<snip>

#1          #2             #3             #4

Transformer =   12kv/100ma    12kv/30ma      15kV/30mA       unknown

Gap Type =       Vacuum       flat cu pipe   RQ cylinder

Total gap =      .225         .225           .300

Primary cap =    .015        . 0056          .0045

Resistors =     310 ohm 50W   330 Ohm  2W   500 Ohm 25W

Choke value =   2.25mH        4.1 mh         2.1mh

BPS =            120?          ?              ?

Resistor Heat = COLD          warm           burnt

> >Malcolm, Mike, and all,

> >

> >Some thoughts:

> >Perhaps there's overvolting and subsequent internal arcing (hot spots)

> >under transient conditions? I notice that the third coil is using a

> >higher supply voltage, a higher resistance value, and a lower wattage

> >(25W) wirewound resistor. The 3rd system and the 2nd one are closer (65

> >Hz) to resonant charging than the first (73 Hz). A typical 25 watt

> >resistor is only 2" long, while a 50 watt one is more like 4", so I'd

> >expect the standoff voltage to be much higher on a 50 Watt resistor.

> >And, since Mike's later post indicates no bypass caps in any of the

> >system, this variable is eliminated.

> >

> >The 2nd system is extremely interesting, in that only a 2 Watt resistor

> >is used! Are these a carbon composition or wirewound? Generally,

> >wirewounds would be more inductive, leading to higher peak voltages

> >developed across them during high frequency transients. Irrespective of

> >the type, the voltage withstanding capability of this resistor should be

> >significantly less than the other two - the resistive element in a

> >typical 2W resistor is only about 1/2" long. Actually, I'm a bit

> >surprised that 2W resistors are surviving in this environment...

> >

> >-- Bert --

> >

>

>

> Sorry I didnt keep up with the thread today. A monitor failure

> kept me offline for most of the day.

>

> The resistors in use on Corey's system (he is on this list by the way)

> are indeed carbon composition 2 watt resistors. I sent him 4 of

> these intending for him to use two in series on each side.

> He is using only 1 on each side with complete success.

> Obviously the only thing those resistors are seeing

> is the 30ma or so charging current from the neon.

> Any more and they would certainly be getting hot.

I would like to see what happens if he changes them for wirewounds.

> These 3 coils system were all built with my help.

> Thats whats is so strange about coil #3. He hasnt done

> anything out of the ordinary so I fail to see what

> the difference is.

What about the coupling constant?

> The only marked difference among the 3 coils is

> the primary geometry. Mine uses a slightly tapered

> solenoid. Corey used a shallow (15 deg I think) saucer

> primary and coil #3 used a flat spiral.

OK, what sparklength does each coil peak at?

Malcolm

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: transformer

Date: Wed, 02 Sep 1998 18:21:32 CDT

From: Kent Vander Velden <graphix@iastate.edu>

Sender: tesl-err@emachine.com

X-Mailer: IPAD 2.0

Hi.  Is there another type of transformer that would

be suitable for Tesla coil use?  I would like something in

the same power output range as a neon, not as lethal as

a pole transformer but as strong as a pole transformer.

Thanks!

---

Kent Vander Velden

kent@iastate.edu

X-Mailer: IPAD 2.0

Date: Thu, 03 Sep 1998 00:57:33 +0100

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: bob golding <yubba@clara.net>

Subject: Re: argon gaps

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

At 19:43 01/09/98 -0600, you wrote:

>snip>

>The gap was quite beautiful.  The argon

>has a turquoise glow and was really neat to watch cooking away.

Hi All,

Does anyone know if the level of harmful UV emitted from the gap is reduced

when using argon or other inert gases?

bob golding

bob yubba@clara.net

From: Hollmike@aol.com

Date: Wed, 2 Sep 1998 20:26:05 EDT

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: transformer

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Kent,

110V can kill you just as dead as a pole pig.  It is just a little easier

to get yourself killed with the pig.  You can gang several neons in parallel

to get whatever power level you desire - even surpass a pole pig if you use

enough of them.  I don't know if this is helpful or not, just tossing in my

two cents worth.

Mike Hollingsworth.

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: transformer

Date: Wed, 02 Sep 1998 20:08:21 CDT

From: Kent Vander Velden <graphix@iastate.edu>

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

In message <7be729b3.35ede21d@aol.com>, Hollmike@aol.com writes:

>Kent,

>    110V can kill you just as dead as a pole pig.  It is just a little easier

>to get yourself killed with the pig.  You can gang several neons in parallel

>to get whatever power level you desire - even surpass a pole pig if you use

>enough of them.  I don't know if this is helpful or not, just tossing in my

>two cents worth.

>Mike Hollingsworth.

This isn't really what I was looking for.  Are there types of

transformers available that have the robustness of a pole transformer

but are in the 10-15kv and 60-120 mA range?  I want a transformer

that can stand up to abuse.

Thanks.

---

Kent Vander Velden

kent@iastate.edu

Date: Wed, 02 Sep 1998 20:09:07 -0500

From: Bert Hickman <bert.hickman@aquila.com>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Organization: Stoneridge Engineering

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: transformer

Sender: tesl-err@emachine.com

Kent Vander Velden wrote:

>

>   Hi.  Is there another type of transformer that would

> be suitable for tesla coil use?  I would like something in

> the same power output range as a neon, not as lethal as

> a pole transformer but as strong as a pole transformer.

>

>   Thanks!

>

> ---

> Kent Vander Velden

> kent@iastate.edu

Kent,

ALL high voltage transformers are dangerous - accidentally getting

across ANY of them can be lethal! It's just that higher power units also

have the nasty capability to burn off chunks of you in the process...

play safe! :^)

And NEVER attempt to run any unballasted transformer without an external

current limiting ballast!

Types of transformers that are generally available to coilers include:

1. Neons (NST's) and Oil Burner Ignition Transformers (OBIT's):

These are the "safest" and are self-ballasting. However, they tend to be

somewhat fragile in Tesla Coil use. OBIT's are probably the most

fragile. 60 mA NST's tend to be a bit hardier than the 30 mA units,

especially when used in resonant charging systems. These tend to be

commonly available, often for free or at relatively low cost. Never buy

one new... You can almost always find them cheaply by looking around a bit...

2. Bombarder Transformers:

Higher current, and most require an external ballasting inductor.

Similar to a small (5-7 KVA distribution transformer. Typically more

expensive than a rebuilt distribution transformer. Avoid unless you can

get one cheaply.

3. Potential Transformers:

Higher current, built more robustly than neons, but not as hardy as a

distribution transformer. Requires an external ballast. Can sometimes be

found in scrapyards, particularly in those that recycle electrical/power

equipment. These, like their current transformer cousins, are used to

safely "instrument" high voltage/high current systems by transforming

them to lower voltage and current levels.

4. Distribution (pole) Transformers:

High current, almost indestructable, but requires external ballast. Not

much more dangerous than options 2, 3 or 5 when connected through a

ballast. These are heavy (typically 250-400+ pounds) and always deserve

your utmost respect!

5. Other High voltage plate, dry plate transformer, or custom wound dry

transformer:

These are considerably less common, can deliver high current, and all

require external ballasting. Dry transformers tend to be less tolerant

to high voltage transients than options 3 or 4 above.

Which is the best alternative?

The best alternative to a NST is a potential transformer (option 3).

These are precision ratio "instrument" transformers typically used for

metering and relay control purposes in substations. Lower voltage units

are "dry" (but internally impregnated with a solid insulating compound),

and typically come in the range of 750 - 3000 VA. These are rated in

step-down ratio, output voltage, and KVA. A 120:1 unit with 120 VAC

output will deliver 14,400 volts when used as a step-up transformer.

While these units require an external ballast, they are built to

withstand significant transient voltages (typically 90 - 130 KV

lightning and switching transients). They are considerably more rugged

and heavier than NST's, but at 75-125 pounds, are considerably lighter

than a distribution transformer. As precision "instruments", they tend

not to be affordable as new devices, but they can often be found for

scrap value when electrical equipment is pulled out of service/scrapped.

Some electrical equipment suppliers may stock used units at affordable prices.

Hope this helps!

-- Bert --

Sender: tesl-err@emachine.com

Date: Wed, 02 Sep 1998 18:20:25 -0700

From: Doug Fulton <dfulton@jdeere.rose.hp.com>

Organization: Hewlett-Packard Company

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: wire gauge

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Hi guys,

I'm collecting parts to build a 6" coil, already have most parts. I have

had a 4 lb. roll of 26 ga. enameled cu. wire on my shelf for years, was

contemplating using it on this coil. I suspect from other conversations

that I heard (seen) on the T2-mail list that this may not be advisable.

Would someone please elaborate on this?

Thanks,

Doug Fulton

dfulton@jdeere.rose.hp.com

From:          "Malcolm Watts" <MALCOLM@directorate.wnp.ac.nz>

Organization:  Wellington Polytechnic, NZ

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Date:          Thu, 3 Sep 1998 13:28:30 +1200

Subject:       Re: wire gauge

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Hi Doug,

What height is your proposed winding? You could do well

with a space wound coil with that gauge. It will score you a higher Q

than a closewind. Final frequency would be the main concern.

Malcolm

> Date sent:      Wed, 02 Sep 1998 18:20:25

- 0700 > From:           Doug Fulton <dfulton@jdeere.rose.hp.com>

> Organization:   Hewlett-Packard Company

> To:             "Tesla-2 Mailing List" <tesla-2@emachine.com>

> Subject:        wire gauge

> Send reply to:  "Tesla-2 Mailing List" <tesla-2@emachine.com>

> Hi guys,

>

> I'm collecting parts to build a 6" coil, already have most parts. I have

> had a 4 lb. roll of 26 ga. enameled cu. wire on my shelf for years, was

> contemplating using it on this coil. I suspect from other conversations

> that I heard (seen) on the T2-mail list that this may not be adviseable.

> Would someone please elaborate on this?

>

>     Thanks,

>     Doug Fulton

>     dfulton@jdeere.rose.hp.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: "Coiler" <mycroft@access1.net>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: RE: Hot resistors (was safety gaps)

Date: Wed, 2 Sep 1998 19:47:21 -0700

X-Mailer: IPAD 2.0

Importance: Normal

X-MimeOLE: Produced By Microsoft MimeOLE V4.72.3155.0

Sender: tesl-err@emachine.com

Malcolm: I am going to throw in my two cents worth, since my resistors get

a little warm.

[note that I lost the posts with the parameter lists.. but I think I can

guess the majority of it]

Power: NST 12KV@30ma (2 in parallel)

Coil: 6.25x25

Primary: .25" copper tubing

Gap: 0.218

Choke: Approx. 3mh (6" of 28awg on 2" ABS)

Resistor: 1K 50W on each side (wirewound, and the inductance is not

something to be ignored..meter showed

something like 2mh [I lost my notes when I moved])

Cap: 0.014uf (approx. it was still increasing slightly from use)

After 5 minutes of operation, the resistor gets warm. But I have a fairly

active gap (I smoked a neon in the first 20 minutes of my coiling experience.. been

gun-shy since)

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: transformer

Date: Wed, 02 Sep 1998 21:47:36 CDT

From: Kent Vander Velden <graphix@iastate.edu>

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

>Which is the best alternative?

>

>The best alternative to a NST is a potential transformer (option 3).

>These are precision ratio "instrument" transformers typically used for

>metering and relay control purposes in substations. Lower voltage units

>are "dry" (but internally impregnated with a solid insulating compound),

>and typically come in the range of 750 - 3000 VA. These are rated in

>step-down ratio, output voltage, and KVA. A 120:1 unit with 120 VAC

>output will deliver 14,400 volts when used as a step-up transformer.

Bert,

Thanks for your wonderful summary of various transformer types!

What is the best way to go about finding a potential transformer?

Just crack open the yellow pages to scrap yards?  What sort

of price range would be reasonable and how can a person be

sure they are not dealing with a lemon?

---

Kent Vander Velden

kent@iastate.edu

X-Originating-IP: [208.16.149.50]

From: "Bill the arcstarter" <arcstarter@hotmail.com>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: transformer

Date: Wed, 02 Sep 1998 20:06:05 PDT

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

It was written:

>>Kent,

>> 110V can kill you just as dead as a pole pig.  It is just a little easier

>>to get yourself killed with the pig.  You can gang several neons in parallel

>>to get whatever power level you desire - even surpass a pole pig if you use

>>enough of them.  I don't know if this is helpful or not, just tossing in my

>>two cents worth.

>>Mike Hollingsworth.

>

>  This isn't really what I was looking for.  Are there types of

>transformers available that have the robustness of a pole transformer

>but are in the 10-15kv and 60-120 mA range?  I want a transformer

>that can stand up to abuse.

>

I think about the only thing like this would be either a pig or

potential transformer with enough external ballasting to limit current

to the desired range.  It would still be lethal, however.  This isn't

probably what you wanted to hear...

-Bill

From:          "Malcolm Watts" <MALCOLM@directorate.wnp.ac.nz>

Organization:  Wellington Polytechnic, NZ

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Date:          Thu, 3 Sep 1998 15:08:32 +1200

Subject:       RE: Hot resistors (was safety gaps)

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Thanks for this:

> Send reply to:  "Tesla-2 Mailing List" <tesla-2@emachine.com>

> From:           "Coiler" <mycroft@access1.net>

> To:             "Tesla-2 Mailing List" <tesla-2@emachine.com>

> Subject:        RE: Hot resistors (was safety gaps)

> Date sent:      Wed, 2 Sep 1998 19:47:21 -0700

> Malcom: I am going to throw in my two cents worth, since my resistors get

> a little warm.

>

> [note that I lost the posts with the parameter lists.. but I think I can

> guess the majority of it]

>

> Power: NST 12KV@30ma (2 in parallel)

> Coil: 6.25x25

> Primary: .25" copper tubing

> Gap: 0.218

> Choke: Approx. 3mh (6" of 28awg on 2" ABS)

> Resistor: 1K 50W on each side (wirewound, and the inductance is not

>                      something to be ignored..meter showed

>                      something like 2mh [I lost my notes when I moved])

> Cap: 0.014uf (approx. it was still increasing slightly from use)

>

> After 5 minutes of operation, the resistor gets warm. But I have a fairly

> active gap (I smoked a neon in the first 20 minutes of my coiling experience.. been

> gun-shy since)

Dear all, note that this system is *very* close to mains resonance

with that cap. But the resistor isn't smoking, just warm. I think we

can eliminate charging artifacts from the mix which brings us to the

TC itself including possibly, the output discharge conditions. My

instincts say that the coupling constant has a lot to do with the

heating. By implication, gap behaviour figures strongly in

this along with transients the resistors see when the gap zero

crosses.

Whatthinks?

Malcolm

X-Originating-IP: [208.16.149.50]

From: "Bill the arcstarter" <arcstarter@hotmail.com>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: safety gaps

Date: Wed, 02 Sep 1998 20:18:00 PDT

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

It was written:

>> But somewhat hard to check since you can't just throw it across the

>> neon.  Example - a 2Kohm resistor placed across a NST supplying 60 ma

>> will only drop 2K*0.060 = 120 volts, much less that the neon's nominal

<SNIP>

>You don't want to throw it across the neon. I think the question was

>"is coil performance affected by omitting the resistor altogether?"

Oh?  I though we were trying to determine the mechanism of the heating

of this resistor in some systems but not others...  My point was that

the max energy the neon (by itself) could dump into that 2K resistor

could only be about (via I^2*R) (0.06^2)*2K = 7 watts, unless some other

mechanism were at work...

>> series with a spark gap) across the capacitor.  You can surely impress

>> higher voltages across the resistor in question using this layout than

>> by placing the resistor directly across the neon's output.

>

>I don't think you'd want to do that would you?

Of course not, well, maybe I just might want to try it :)  Actually, I

already have... heh heh heh So yes - I did (apparently) want to do that!

Seriously - I mentioned it ONLY because of the fact that having the

primary capacitance can change the max voltages (and possibly resistor

dissipations too?) in ways which are not immediately obvious to all.

Malcolm - it's always fun to debate with you, even on this small stuff.

I never did arrive at any definite conclusion re: our many-months-old

thread on measuring resonant frequencies, Rac, Generator vs load

impedances etc.  Sorry!

-Bill

Starting arcs in Cinci, OH

http://www.geocities.com/CapeCanaveral/Hangar/6160

From:          "Malcolm Watts" <MALCOLM@directorate.wnp.ac.nz>

Organization:  Wellington Polytechnic, NZ

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Date:          Thu, 3 Sep 1998 15:27:52 +1200

Subject:       Re: safety gaps

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Hi Bill,

> From:           "Bill the arcstarter" <arcstarter@hotmail.com>

> To:             "Tesla-2 Mailing List" <tesla-2@emachine.com>

> Subject:        Re: safety gaps

> Date sent:      Wed, 02 Sep 1998 20:18:00 PDT

> Send reply to:  "Tesla-2 Mailing List" <tesla-2@emachine.com>

> It was written:

>

> >> But somewhat hard to check since you can't just throw it across the

> >> neon.  Example - a 2Kohm resistor placed across a NST supplying 60 ma

> >> will only drop 2K*0.060 = 120 volts, much less that the neon's  nominal

>

> <SNIP>

>

> >You don't want to throw it across the neon. I think the question was

> >"is coil performance affected by omitting the resistor altogether?"

>

> Oh?  I though we were trying to determine the mechanism of the heating

> of this resistor in some systems but not others...  My point was that

> the max energy the neon (by itself) could dump into that 2K resistor

> could only be about (via I^2*R) (0.06^2)*2K = 7 watts, unless some other

> mechanism were at work...

Having heard some more details I agree absolutely. I think it *has*

to be heating by some pretty mean RF spikes.

> >> series with a spark gap) across the capacitor.  You can surely  impress

> >> higher voltages across the resistor in question using this layout than

> >> by placing the resistor directly across the neon's output.

> >

> >I don't think you'd want to do that would you?

>

> Of course not, well, maybe I just might want to try it :)  Actually, I

> already have... heh heh heh So yes - I did (apparently) want to do that!

>

>  Seriously - I mentioned it ONLY because of the fact that having the

> primary capacitance can change the max voltages (and possibly resistor

> dissipations too?) in ways which are not immediately obvious to all.

If the gaps were set real wide I'd agree but in these cases, the

spacings seem quite reasonable. The circuit can't ring up beyond 1/2

a mains cycle although in theory the peak circulating current could

hit 3x the RMS current rating of the transformer at the end of each

half cycle. I have to wonder why the 2W resistors aren't affected

more - perhaps they are nbeing bypassed altogether.

>  Malcolm - it's always fun to debate with you, even on this small stuff.

> I never did arrive at any definite conclusion re: our many-months-old

> thread on measuring resonant frequencies, Rac, Generator vs load

> impedances etc.  Sorry!

No matter.  Man, do I want the answer to this problem though.

Regards,

Malcolm

X-Originating-IP: [208.16.149.50]

From: "Bill the arcstarter" <arcstarter@hotmail.com>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: safety gaps

Date: Wed, 02 Sep 1998 20:31:11 PDT

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

Mike wrote (in response to me):

>>Also - Mike Hammer - have you been keeping track of Terry Fritz's

>>argon-quenched gap?  It sounds remarkably similar to one which I

>>proposed to you last October!

<SNIP>

>

>Yes in fact I talked to Terry about it on IRC last night.

>I do indeed remember you asking about that earlier.

>I believe that I told you argon wouldnt be a proper gas for

>use in a spark gap. I was kind of wrong on that one it seems.

Well - at the time I poked around and also found (sketchy) refs which

indicated that for some strange reason the erosion rate in gases w/o O2

was much higher.  So at the time it sounded as if you were right.  We'll

see what sorts of results Terry gets.

I never have put any substantial runtime into my infernal tungsten

carbide watercooled sparkgap device...  But I did get to show it off to

Mike Hollingsworth over the summer! :) (Wonder what the airport x-ray

people thought of that thing?!)

I suppose I ought to stick it in a tupperware container and argonate

the thing!

>Terry's approach is also very similar to the old

>spark radio transmitter quench gaps. 15 or so years

>ago an old time ham I knew tried to give me a Telefunken

>quench gap he had. Not really knowing the significance

>of what it was I declined. He is long gone now and

>all that old stuff of his is probably in some landfill. :(

One man's treasure...  As they say on the VW webpages:

"Don't send your car to heaven (the crusher).

Lord knows we need the parts here on earth!"

There is a fairly cool museum of radio here in Cincinnati - the Gray

Wireless Museum or something like that.  They have a whole bunch of old

quenched disk packs, RSGs, vac tubes, etc from the old days of radio.

Everyone 'otta go there and check out the goodies!

-Bill

Sender: tesl-err@emachine.com

Date: Thu, 03 Sep 1998 09:10:05 +0000

From: Tom Stock <tomstock@geocities.com>

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Exploding water...

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Sam, I'm interested in what you're doing...could you tell me a little

more about your setup so I too can do dangerous things in my garage?

thanks

-tom

>  Shame! It was making quite an interesting discussion...

>  Still, Mike is right; This is a Tesla List after all... But since

> there are no Water Exploding mailing lists, does anyone mind if I mail

> my results for the barrel-mounted, 12J sparkplug design when i get it

> done? It won't become a habit... I promiss..

>

>  Sam.

> ==

>  Sam Barros,

> sambarros@yahoo.com

> ICQ:15156975

>

>  "The less you know, the better you sleep"...

Date: Thu, 03 Sep 1998 06:59:55 -0500

From: Bert Hickman <bert.hickman@aquila.com>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Organization: Stoneridge Engineering

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: safety gaps

Sender: tesl-err@emachine.com

Bill the arcstarter wrote:

>

> Mike wrote (in response to me):

> >>Also - Mike Hammer - have you been keeping track of Terry Fritz's

> >>argon-quenched gap?  It sounds remarkably similar to one which I

> >>proposed to you last October!

> <SNIP>

> >

> >Yes in fact I talked to Terry about it on IRC last night.

> >I do indeed remember you asking about that earlier.

> >I believe that I told you argon wouldnt be a proper gas for

> >use in a spark gap. I was kind of wrong on that one it seems.

>

> Well - at the time I poked around and also found (sketchy) refs which

> indicated that for some strange reason the erosion rate in gases w/o O2

> was much higher.  So at the time it sounded as if you were right.  We'll

> see what sorts of results Terry gets.

>

<SNIP>

Bill and all,

This is mentioned in Glasoe's "Pulse Generators" (McGraw-Hill, 1948) in

association with tungsten electrodes in rotary gaps (p. 280): "Tests on

sealed rotary gaps, as well as fixed gaps, indicated that the presence

of oxygen was necessary to obtain a satisfactory life from the tungsten

electrodes". Also, on page 281: "Experimental data obtained both at the

Bell Laboratories and the Radiation Laboratory show that, for pulse

currents from 40 to 170 amp, the rate of cathode wear in air varies

between about 1 and 6 mg/amp-hour, 3 or 4 mg/amp-hour being the median

value. In nitrogen or gas mixtures that do not contain oxygen, the rate

of wear is 10-20 times greater". These studies were for radar pulse

modulators, where the pulses were unipolar - the erosion rates for AC

powered Tesla Coils should be approximately half this.

The best recent reference for spark gaps is Gerhard Schaefer, "Gas

Discharge Closing Switches (Advances in Pulsed Power Technology, Vol

2)", Plenum, 1991. In the section dealing with sparkgap lifetime

considerations, he indicates that erosion is the main factor limiting

lifetime. Shaefer indicates that the best materials for minimal erosion

are graphite (best for I < 15 kA), tungsten, molybdenum, copper, and

noibium (apparently in an air atmosphere). Unfortunately, most of the

spark gap switches described in this book are much higher power that

encountered in Tesla Coil work, and the fill gases are usually air, SF6,

or nitrogen either at STP or elevated pressure.

While I don't remember the exact source, there was also a discussion

regarding the oxide layer that's formed on copper electrodes fired in

air - this layer forms a barrier which tends to increase the minimum

breakdown voltage of the gap. Controlling the thickness (or preventing

it from occuring) may result in a stabler breakdown voltage, and using a

series of gaps also tends to stabilize (statistically) the breakdown

voltage of the chain.

The bottom line:

It looks like the field is still quite open on what constitues the

"best" static gap for Tesla Coils. Terry's argon-filled series gap

certainly looks good! Your mileage may vary!

-- Bert --

Date: Thu, 03 Sep 1998 07:12:01 -0500

From: Bert Hickman <bert.hickman@aquila.com>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Organization: Stoneridge Engineering

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: transformer

Sender: tesl-err@emachine.com

Kent Vander Velden wrote:

>

> >Which is the best alternative?

> >The best alternative to a NST is a potential transformer (option 3).

> >These are precision ratio "instrument" transformers typically used for

> >metering and relay control purposes in substations. Lower voltage units

> >are "dry" (but internally impregnated with a solid insulating compound),

> >and typically come in the range of 750 - 3000 VA. These are rated in

> >step-down ratio, output voltage, and KVA. A 120:1 unit with 120 VAC

> >output will deliver 14,400 volts when used as a step-up transformer.

>

> Bert,

>

>   Thanks for your wonderful summary of various transformer types!

> What is the best way to go about finding a potential transformer?

> Just crack open the yellow pages to scrap yards?  What sort

> of price range would be reasonable and how can a person be

> sure they are not dealing with a lemon?

>

> ---

> Kent Vander Velden

> kent@iastate.edu

Kent,

You're very welcome! The Yellow Pages is a good start - ask them if they

recycle old substation or high voltage equipment, and if they sell to

the public. In any large metropolitan area, there's usually one or more

scrapyards that do. Another option is to call your local power company

or electrical supply distributor(s) and ask if they ever have these

available as surplus.

There are no guarantees at a scrapyard! Take along an analog

Voltohmmeter so that you can verify that the primary and secondary are

good. The Low Voltage winding will typically measure a fraction of an

ohm, while the secondary should be much higher (500 - 5000 ohms). There

should be no evidence of overheating, arcing, or other physical signs of

damage to the unit. Expect to pay the going scrap rate for copper and

iron plus a premium to the scrapyard. This can be up to $1.00 or

more/pound, although I have heard of them going for as cheaply as

$15-20. A surplus unit from a power company or electrical supply house

can typically be purchased for $150 - $250.

In general, these devices are considerably rarer than NST's, so it may

take a while before you score an inexpensive one. I've yet to find one

in my recent visits to our local yards. :^(

Good luck, and happy hunting!

-- Bert --

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Date: Thu, 03 Sep 1998 05:16:55 -0700

From: "paul m" <fleischwulf@mailexcite.com>

Cc:

X-Sent-Mail: off

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

Subject: Re: Exploding water...

X-Sender-Ip: 208.153.7.9

Organization: MailExcite  (http://www.mailexcite.com:80)

Sender: tesl-err@emachine.com

If anyone on this list would like information on exploding wire, exploding water or can crushing / quarter crushing please contact me at: I have a lot of experience in this field. I will answer your questions.

Klugmann@hotmail.com or see my web site:

www.geocities.com/capecanaveral/hangar/9561

PS: let's keep the Tesla2 list to Tesla projects.

Thanks,

Paul M

---

Einigheit und Wahrheit

On Thu, 03 Sep 1998 09:10:05   Tom Stock wrote:

>Sam, I'm interested in what you're doing...could you tell me a little

>more about your setup so I too can do dangerous things in my garage?

>

>thanks

>-tom

>

>

>>  Shame! It was making quite an interesting discussion...

>>  Still, Mike is right; This is a Tesla List after all... But since

>> there are no Water Exploding mailing lists, does anyone mind if I mail

>> my results for the barrel-mounted, 12J sparkplug design when i get it

>> done? It won't become a habit... I promiss..

>>

>>  Sam.

>> ==

>>  Sam Barros,

>> sambarros@yahoo.com

>> ICQ:15156975

>>

>>  "The less you know, the better you sleep"...

>>

Date: Thu, 03 Sep 1998 07:11:35 -0700

From: Gene Waldo <gwaldo@1stnetusa.com>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: transformer

Sender: tesl-err@emachine.com

Kent Vander Velden wrote:

>

> In message <7be729b3.35ede21d@aol.com>, Hollmike@aol.com writes:

> >Kent,

> >110V can kill you just as dead as a pole pig.  It is just a little easier

> >Mike Hollingsworth.

>

>   This isn't really what I was looking for.  I want a transformer

> that can stand up to abuse.

>

>   Thanks.

>

> ---

> Kent Vander Velden

Kent,

What do you call abuse?  My translation of abuse is a poorly tuned

system. I venture Mike or Bert would have good ideas in this area.

---------------------------

Gene Waldo -  gwaldo@1stnetusa.com  - "Igor in Residence"

Date: Thu, 03 Sep 1998 09:15:12 -0500

From: Eric Davidson <edavidson@icva.gov>

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: transformer

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Kent:

The transformer you are looking for is a cold cathode transformer.  Very

similar to a NST, in fact, almost the same.  They are current limited.

Usually 5-15 kV 60-120 mA.  They are available, but they are quite

expensive, usually cheaper to parallel a couple NST, same effect. CCTs

are not as widely used, and are more difficult to find used. Check out

the Magnetek web site www.magnetek.com to see what they have. CCTs are

pretty robust, but NOT in the same league as a distribution transformer.

I'd still use a saftey gap.  By the way, you will not find a transformer

that can source hundreds of mA at 10-15 kV that is not lethal.  Doesn't

matter what type it is, NST, CCT, PT, bombarder, pole pig...they can all

be lethal.  A pole pig will almost certainly provide deep burns as a

bonus with your electrocution.  Please be very careful. Hope this helps.

Eric

--

Have a good day!

Eric Davidson

edavidson@icva.gov

Date: Thu, 03 Sep 1998 07:55:25 -0700

From: Gene Waldo <gwaldo@1stnetusa.com>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: safety gaps

Sender: tesl-err@emachine.com

Bert Hickman wrote:

>

> Bill the arcstarter wrote:

> >

> > Mike wrote (in response to me):

> > >>Also - Mike Hammer - have you been keeping track of Terry Fritz's

> > >>argon-quenched gap?  It sounds remarkably similar to one which I

> > >>proposed to you last October!

> > <SNIP>

> The bottom line:

> It looks like the field is still quite open on what constitues the

> "best" static gap for Tesla Coils. Terry's argon-filled series gap

> certainly looks good! Your mileage may vary!

>

> -- Bert --

Bert and all,

YEARS ago, in a land... I was wont to play in the 2ndary out of

12k/30,60(?)NST.

A diagonally broken microscope slide in 2 alligator cords was used to

draw a ~2.5" vertical spark (more like a quiet flame) from the tips of

the glass.

Glass tips melted into nice round spherical forms, and conducted nicely.

I never worked out what was really happening, there.  No sign of other

activity.  How does glass conduct?

-------------------------------------------

Gene Waldo -  gwaldo@1stnetusa.com  - "Igor in Residence"

---------------------------------------------------------------

What's good for the planet is more OZONE! See the machinery form up!

NEW PAGE!   http://members.1stnetusa.com/danmurphy/waldo.htm

Sender: tesl-err@emachine.com

Date: Thu, 03 Sep 1998 08:19:19 -0700

From: Doug Fulton <dfulton@jdeere.rose.hp.com>

Organization: Hewlett-Packard Company

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: wire gauge

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Malcolm Watts wrote:

>

> Hi Doug,

>            What height is your proposed winding? You could do well

> with a space wound coil with that gauge. It will score you a higher Q

> than a closewind. Final frequency would be the main concern.

>

> Malcolm

>

Hi Malcom,

Thanks for your quick response. I am planning on making the coil about

18" based on Richard Quick's wire gauge chart which shows that 26 ga.

wire takes 58 turns/inch. I will use the win Tesla program to calculate

the frequency and inductance as soon as I get a chance. This is the first

coil that I have built in recent years, the last one was built in the

early 60's and I used a model "T" ford spark coil (buzz box) for the

power source. It was a much smaller coil, 34 ga. wire wound on a

cardboard tube. It produced 4" - 5" sparks, it was ok for a 12 year kid.

>

> >

Thanks,

> >     Doug Fulton

> >     dfulton@jdeere.rose.hp.com

X-Originating-IP: [199.217.140.22]

From: "Bill the arcstarter" <arcstarter@hotmail.com>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: safety gaps

Date: Thu, 03 Sep 1998 10:14:31 PDT

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

>A diagonally broken microscope slide in 2 aligator cords was used to

>draw a ~2.5" vertical spark (more like a quiet flame) from the tips of

>the glass.

>

>Glass tips melted into nice round spherical forms, and conducted nicely.

>

>I never worked out what was really happening, there.  No sign of other

>activity.  How does glass conduct?

Ooh!  Ooh! Ooh!  I know this one!

Many oxides are conductive once molten.  I'm told that some of the large

glass companies actually pass huge currents through molten glass to keep

it molten (can anyone confirm this?).  Molten oxides are also rather

corrosive...

I once cut the corner off a firebrick using an arc from a 15kv 60ma

transformer.  The arc just walked right through the brick (took about 5

minutes).

-Bill

X-Mailer: IPAD 2.0

Date: Thu, 03 Sep 1998 12:56:28 -0500

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: hot resistors/safety gaps

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

At 10:49 AM 9/3/98 +1200, you wrote:

>Hi Mike,

>

>> Date sent:      Wed, 02 Sep 1998 17:09:56 -0500

>> To:             "Tesla-2 Mailing List" <tesla-2@emachine.com>

>> From:           Mike Hammer <mhammer@misslink.net>

>> Subject:        Re: safety gaps

>> Send reply to:  "Tesla-2 Mailing List" <tesla-2@emachine.com>

>

><snip>

>>                    #1          #2             #3             #4

>>

>> Transformer =   12kv/100ma    12kv/30ma      15kV/30mA       unknown

>> Gap Type =       9@.025       9@.025         10@.03

>> Total gap =      .225         .225           .300

>> Primary cap =    .015        . 0056          .0045

>> cap type =      rolled poly   SW bottle      rolled poly

>> Resistors =     310 ohm 50W   330 Ohm  2W    500 Ohm 25W

>> Choke value =   2.25mH        4.1 mh         2.1mh

>> BPS =            120?          ?              ?

>> Resistor Heat = COLD          warm           burnt

>> Res Freq  =     254 khz       ~275khz        323khz

>> Max output      54 inches      22 inches     24 inches

>>

>>

>> Sorry I didnt keep up with the thread today. A monitor failure

>> kept me offline for most of the day.

>>

>> The resistors in use on Corey's system (he is on this list by the way)

>> are indeed carbon composition 2 watt resistors. I sent him 4 of

>> these intending for him to use two in series on each side.

>> He is using only 1 on each side with complete success.

>> Obviously the only thing those resistors are seeing

>> is the 30ma or so charging current from the neon.

>> Any more and they would certainly be getting hot.

>

>I would like to see what happens if he changes them for wirewounds.

I will suggest this but it may be a bit difficult as he

lives in northern Ontario. A little hard to find parts.

I will probably have to mail him a couple resistors to try.

>

> > These 3 coils system were all built with my help.

>> Thats whats is so strange about coil #3. He hasnt done

>> anything out of the ordinary so I fail to see what

>> the difference is.

>

>What about the coupling constant?

I dont know the coupling coefficient for the other systems.

My own coil is run rather tightly. It is currently running around

K .22. I have run it even tighter than this but had too many racing

spark problems. It was at this level of coupling that I set my best

output record of 54 inches at 1.2kva. I was still able to get first

notch quenching at this level of coupling using my vacuum gap.

>

>> The only marked difference among the 3 coils is

>> the primary geometry. Mine uses a slightly tapered

>> solenoid. Corey used a shallow (15 deg I think) saucer

>> primary and coil #3 used a flat spiral.

>

>OK, what sparklength does each coil peak at?

My coil peaks at 54 inches currently. I plan a somewhat larger

primary cap and have 60 inches set as a goal.

Corey's coil (#2) peaks at about 22 inches with consistant

20 inch streamers. Coil number 3 peaked at 24 inches. We

had hoped for better results with it. A somewhat larger cap

and toroid is planned for this coil. Its owner and builder

has some severe time constraints so work progresses rather slowly.

I also added the cap type on the 3 coils in question. Not sure

if its significant but I added it anyway.

TTYL!

Mike Hammer

mhammer@misslink.net

X-Mailer: IPAD 2.0

Date: Thu, 03 Sep 1998 13:03:25 -0500

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: wire gauge

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

At 06:20 PM 9/2/98 -0700, you wrote:

>Hi guys,

>

>I'm collecting parts to build a 6" coil, already have most parts. I have

>had a 4 lb. roll of 26 ga. enameled cu. wire on my shelf for years, was

>contemplating using it on this coil. I suspect from other conversations

>that I heard (seen) on the T2-mail list that this may not be adviseable.

>Would someone please elaborate on this?

>

>    Thanks,

>    Doug Fulton

>    dfulton@jdeere.rose.hp.com

Doug,

My personal opinion would be to save the AWG 26 for some

other project or for some RF chokes and get some AWG 22

for your 6 inch secondary.

Using AWG 26 for a 6 incher will result in a quite high

turns count and a very low resonant frequency. This low

frequency would require a large primary cap along with a

large transformer to charge it.

You could lower the turns count by making the secondary shorter

but doing so will limit the voltage stand off capability of

the secondary.

Assuming 6 inch PVC, a 6.5 X 26 inch secondary wound with

AWG 22 is nearly a perfect secondary. It can handle power

levels up to 5 or 6 KVA and yet work well with a single 12 or 15kv

60ma neon. There are a number of coils around just like this and

this is the coil I want to build next.

TTYL!

Mike Hammer

mhammer@misslink.net

From:          "Malcolm Watts" <MALCOLM@directorate.wnp.ac.nz>

Organization:  Wellington Polytechnic, NZ

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Date:          Fri, 4 Sep 1998 08:38:04 +1200

Subject:       Re: wire gauge

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Hi Doug,

> Date sent:      Thu, 03 Sep 1998 08:19:19 -0700

> From:           Doug Fulton <dfulton@jdeere.rose.hp.com>

> Organization:   Hewlett-Packard Company

> To:             "Tesla-2 Mailing List" <tesla-2@emachine.com>

> Subject:        Re: wire gauge

> Send reply to:  "Tesla-2 Mailing List" <tesla-2@emachine.com>

<snip>

> Thanks for your quick response. I am planning  on making the coil about

> 18" based on Richard Quick's wire gauge chart which shows that 26 ga.

> wire takes 58 turns/inch. I will use the win Tesla program to calculate

> the frequncy and inductance as soon as I get a chance. This is the first

> coil that I have built in recent years, the last one was built in the

> early 60's and I used a model "T" ford spark coil (buzz box) for the

> power source. It was a much smaller coil, 34 ga. wire wound on a

> cardboard tube. It produced 4" - 5" sparks, it was ok for a 12 year kid.

With that wire and a closewind for that height, inductance will be

about 47mH, Cself about 9.3pF so resonant frequency is around

240kHz without a terminal. Sounds like a nice coil but as always,

height is a determinant of how much voltage the coil can stand.

Have fun.

Malcolm

From:          "Malcolm Watts" <MALCOLM@directorate.wnp.ac.nz>

Organization:  Wellington Polytechnic, NZ

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Date:          Fri, 4 Sep 1998 08:52:43 +1200

Subject:       Re: hot resistors/safety gaps

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Hi Mike,

So as not to push the table any further to the right I've

omitted the reply markers. We can add one or two more things to the

table now, primary inductance being the easy one.

>>                    #1          #2             #3             #4

>>

>> Transformer =   12kv/100ma    12kv/30ma      15kV/30mA       unknown

>> Gap Type =       9@.025       9@.025         10@.03

>> Total gap =      .225         .225           .300

>> Primary cap =    .015        . 0056          .0045

>> cap type =      rolled poly   SW bottle      rolled poly

>> Resistors =     310 ohm 50W   330 Ohm  2W    500 Ohm 25W

>> Choke value =   2.25mH        4.1 mh         2.1mh

>> BPS =            120?          ?              ?

>> Resistor Heat = COLD          warm           burnt

>> Res Freq  =     254 khz       ~275khz        323khz

>> Max output      54 inches      22 inches     24 inches

>> Lp =            26uH           60uH          54uH

However, it seems to me that keeping the gap quiet for as long as

possible is a factor so transients generated by it are also a factor

as you suggest. At k=0.22 with first notch quench, your gap is seeing

primary current for just 9uS. I wonder what the others are doing. The

difference between 2 and 3 could be the difference between quench

ability which will be harder for air streamers.

There is also another factor to be considered - physical

construction. What is the relative quality of the primary "wiring"

in each system? I wonder if the difference lies there as well?

>I would like to see what happens if he changes them for wirewounds.

I will suggest this but it may be a bit difficult as he

lives in northern Ontario. A little hard to find parts.

I will probably have to mail him a couple resistors to try.

Didn't mean to make life difficult. I do think the compositions

resistors are a better choice if they can withstand whatever it is

they are being asked to withstand.

I dont know the coupling coefficient for the other

systems. My own coil is run rather tightly.

It is currently running around .22. I have run it even

tighter than this but had too many racing spark problems.

It was at this level of coupling that I set my best output

record of 54 inches at 1.2kva. I was still able to get first

notch quenching at this level of coupling using my vacuum gap.

>

>> The only marked difference among the 3 coils is

>> the primary geometry. Mine uses a slightly tapered

>> solenoid. Corey used a shallow (15 deg I think) saucer

>> primary and coil #3 used a flat spiral.

>

>OK, what sparklength does each coil peak at?

My coil peaks at 54 inches currently. I plan a somewhat larger

primary cap and have 60 inches set as a goal.

Corey's coil (#2) peaks at about 22 inches with consistant

20 inch streamers. Coil number 3 peaked at 24 inches. We

had hoped for better results with it. A somewhat larger cap

and toroid is planned for this coil. Its owner and builder

has some severe time constraints so works progresses rather slowly.

I also added the cap type on the 3 coils in question. Not sure

if its significant but I added it anyway.

The more data the better. I wonder if there are conductive objects

like sheet metal etc near Gary's coil?  An interesting thing I noted

in my current confined coiling space is that resonators sitting

around the place all show some activity when a system is running.

Still digging,

Malcolm

From: Ashpilot@aol.com

Date: Thu, 3 Sep 1998 18:20:44 EDT

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: hot resistors/safety gaps

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

I have been following this thread, and I was wondering, is it possible for

resonant frequencies to build in a conductor like a standing wave?  I mean if

the conductors in the primary circuit were of the right length would that

cause them to be "intune" and resonate to increase voltages???   Or could

they be acting like an antenna for the RF output? I seem to remember something

like this happening in Ham radios that were amateur built..

I don't have a lot of experience with these things yet, so if this is way

off...  I'm just guessin'.

From:          "Malcolm Watts" <MALCOLM@directorate.wnp.ac.nz>

Organization:  Wellington Polytechnic, NZ

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Date:          Fri, 4 Sep 1998 10:31:52 +1200

Subject:       Re: hot resistors/safety gaps

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

> From:           Ashpilot@aol.com

> Date sent:      Thu, 3 Sep 1998 18:20:44 EDT

> To:             "Tesla-2 Mailing List" <tesla-2@emachine.com>

> Subject:        Re: hot resistors/safety gaps

> Send reply to:  "Tesla-2 Mailing List" <tesla-2@emachine.com>

> I have been following this thread, and I was wondering, is it possible for

> resonant frequencies to build in a conductor like a standing wave?  I mean if

> the conductors in the primary circuit were of the right length would that

> cause them to be "intune" and resonate to increase voltages???   Or coulde

> they be acting like an antena for the RF output? I seem to remember something

> like this happening in Ham radios that were amature built..

>

> I don't have a lot of experience with these things yet, so if this is way

> off...  I'm just guessin'.

That can certainly happen if Q is high enough and parasitic

capacitances are low enough. Perhaps it's time we looked at the

length of wiring for each part of each system as well.

Malcolm

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: transformer

Date: Thu, 03 Sep 1998 17:58:45 CDT

From: Kent Vander Velden <graphix@iastate.edu>

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

>What do you call abuse?  My translation of abuse is a poorly tuned

>system.

I have had several people question my choice of words :)  Lets

define `the ability to withstand abuse` as `the ability to handle

unforseen surprises with a larger tolerance than a neon.`

Instances of Murphy's law can happen to any system tuned or

not, Tesla coil or something completely different.

`Lethal` was another word that I received an amazing number

of responses about.  While it is true that 110v can be

lethal (as well as a number of other voltage levels) that was

not really the point that I was getting at.  Lets create a

scale of the potential danger in the range [1, 10].  Transformers

larger and including pole transformers are 10s. Neons are 5s.

Voltages less than and equal to 110v are 1s.  I am looking for

transformers in the [5, 8] inclusive range.

Concerning scrap yards:

Just called ~20 scrap yards.  I was very surprised by the

responses.  Most did not know what a transformer was.  The

others did not deal with them.  Only one scrap yard said they

had in the past but did not any longer.  Shoot.

Thanks.

---

Kent Vander Velden

kent@iastate.edu

Date: Thu, 03 Sep 1998 20:39:21 -0500

From: Bert Hickman <bert.hickman@aquila.com>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Organization: Stoneridge Engineering

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: safety gaps

Sender: tesl-err@emachine.com

Gene Waldo wrote:

>

> Bert Hickman wrote:

> >

> > Bill the arcstarter wrote:

> > >

> > > Mike wrote (in response to me):

> > > >>Also - Mike Hammer - have you been keeping track of Terry Fritz's

> > > >>argon-quenched gap?  It sounds remarkably similar to one which I

> > > >>proposed to you last October!

> > > <SNIP>

>

> > The bottom line:

> > It looks like the field is still quite open on what constitues the

> > "best" static gap for Tesla Coils. Terry's argon-filled series gap

> > certainly looks good! Your mileage may vary!

> >

> > -- Bert --

> Bert and all,

> YEARS ago, in a land... I was wont to play in the 2ndary out of

> 12k/30,60(?)NST.

>

> A diagonally broken microscope slide in 2 alligator cords was used to

> draw a ~2.5" vertical spark (more like a quiet flame) from the tips of

> the glass.

>

> Glass tips melted into nice round spherical forms, and conducted nicely.

>

> I never worked out what was really happening, there.  No sign of other

> activity.  How does glass conduct?

>

> -------------------------------------------

> Gene Waldo -  gwaldo@1stnetusa.com  - "Igor in Residence"

Gene and all,

Interesting question - and very relevant to homemade glass tank caps.

Glass becomes a very good conductor at elevated temperatures. For

example, a low voltage carbon arc can be "fed" with a glass rod -

resulting in a yellow-hot glob of conducting glass that stays heated by

the current flowing through the glass. Similarly, a glass plate

capacitor under RF excitation will heat up initially due to dielectric

losses.

As the temperature rises, dielectric losses also increase, and ions

trapped in the glass matrix become more mobile, causing ohmic leakage

currents to climb as well. These combined factors can rapidly become a

runaway condition that quickly results in puncturing of the glass. Red

hot glass is a surprisingly good conductor!

-- Bert --

From: ralph.hartwell@emachine.com (Ralph Hartwell)

Subject:  Re: hot resistors/safety gaps

Date: Fri,  4 Sep 1998 05:04:00 GMT

Organization: The Energy Machine Information System 504-733-3462

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

>> I have been following this thread, and I was wondering, is it possible for

>> resonant frequencies to build in a conductor like a standing wave?  I mean i

<snip>

>That can certainly happen if Q is high enough and parasitic

>capacitances are low enough. Perhaps it's time we looked at the

>length of wiring for each part of each system as well.

While testing Joseph Newman's energy invention (and we WON'T get into

that!) I was easily able to detect the RF energy radiated from the

connecting wires between the commutator (= spark gap) and the coils of

the machine.  The spectrum analyzer clearly showed the various

frequencies which correlated nicely to the various wire lengths.

I would imagine that the wires in a Tesla system would "ring" quite

well, since they are driven very powerfully by the arc discharge.  It's

surprisingly easy to inadvertently create a parasitic resonant circuit.

Just ask any RF designer.

Ralph  W5JGV

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: R.E.  Transformers

X-Mailer: IPAD 2.0

From: break7@juno.com (Tyla A. Break)

Date: Fri, 04 Sep 1998 01:58:43 EDT

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Thought I'd give a little input on transformers. First I consider

all voltages 60 V and up lethal. All depends how your hooked up and how

long . I've been working as an electrician for the 12 years and have had

plenty shocks. But I myself don't even think about working hot anything

240v or higher. Some people will tell you that neons won't kill you, I

consider 12-15kv  30 or 60ma instant death!

Now about transformers. Keep in mind that many neons have a grounded

center tap. 7500v to the case. Depending how you hook up your system

(grounding the case  or not) The case could be considered "hot".

I recently purchased a potential transformer. It's true they are kinda

rare and most people have not heard of them. What helped me is seeing one

first. I'd been walking by it for years, in a pile of insulators. But

after seeing pictures of one I immediately recognized it!  Brent Turner

has some good pics of one on his page. Mine is a 120 : 1 1750VA. Weighs

72lbs and is quite compact for its power output. As it has been said they

must be externally ballasted. Not really dangerous but will try to draw

unlimited power when not ballasted. Usually results in a blown breaker

before you even see an arc. If attached to a supply that is too "robust"

(see, I'm using the word now) You could smoke your little jewel. In my

case that would be - $50.

A (Pole Pig) would be the next step. And do to the VERY high

currents available they are instantly fatal   No Question! Pole transformers

are quite tough and can always put out more than rated current. One thing you

might look into, miniatures are sometimes available. I got a little one.

Smaller than a 5gal. bucket, about 80lbs with the ridiculously low rating

of 500 watts. It can easily put out five times that amount for short

duration. They are referred to as control transformers . They're used for

opening and closing oil filled switches to add capacitance to

distribution lines. You'll see them up on poles with cap banks.

Hope you or anyone else finds this helpful

Kyle break7@juno.com

Date: Fri, 04 Sep 1998 07:14:03 -0500

From: Bert Hickman <bert.hickman@aquila.com>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Organization: Stoneridge Engineering

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: transformer

Sender: tesl-err@emachine.com

Kent Vander Velden wrote:

>

> >What do you call abuse?  My translation of abuse is a poorly tuned

> >system.

>

>  I have had several people question my choice of words :)  Lets

> define `the ability to withstand abuse` as `the ability to handle

> unforseen surprises with a larger tolerance than a neon.`

> Instances of Murphy's law can happen to any system tuned or

> not, Tesla coil or something completely different.

>   `Lethal` was another word that I received an amazing number

> of responses about.  While it is true that 110v can be

> lethal (as well as a number of other voltage levels) that was

> not really the point that I was getting at.  Lets create a

> scale of the potential danger in the range [1, 10].  Transformers

> larger and including pole transformers are 10s. Neons are 5s.

> Voltages less than and equal to 110v are 1s.  I am looking for

> transformers in the [5, 8] inclusive range.

>

>   Concerning scrap yards:

>

>   Just called ~20 scrap yards.  I was very surprised by the

> responses.  Most did not know what a transformer was.  The

> others did not deal with them.  Only one scrap yard said they

> had in the past but did not any longer.  Shoot.

>

>   Thanks.

>

> ---

> Kent Vander Velden

> kent@iastate.edu

Kent,

Don't give up all hope as yet... If any of the scrapyards process power

and electrical equipment, they may get these in on occasion. The key is

in whether they will allow you to buy something you may be interested

in. Some yards may not have a commercial retail license, and some have

agreements with waste haulers or industrial clients that stipulate that

ALL incoming material will be destroyed. I'm fortunate that about 25

miles from me is a yard that gets in LOTS of electrical, mechanical,

medical, and electronic gear. And, they let you take your toolkit in so

you can remove JUST the pieces you want!

While the operators of these facilities are quite knowledgable about

metals identification and market prices, they are seldom familiar with

the function, or even the names, of the items they get in. So it's not

too surprising that many don't know what a tranformer or capacitor is.

The best bet is to find those yards that will let you look around and...

look around! Even if you don't find a potential transformer, you may

find many other things that ARE useful in coiling or other experiments.

There's often an amazing variety of electrical and electronic equipment

or parts (often brand new!) for the taking at rock-bottom prices.

Also, give a call to your local power company and ask them what they do

with their surplus instrument or control transformers. Also try wrecking

companies - these guys sometimes tear down industrial power gear as part

of their job. And this stuff either ends up in a warehouse or at the

scrap yard.

Good Luck!

-- Bert --

X-Originating-IP: [199.217.140.22]

From: "Bill the arcstarter" <arcstarter@hotmail.com>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: hot resistors/safety gaps

Date: Fri, 04 Sep 1998 05:22:16 PDT

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

Ralph wrote:

>  While testing Joseph Newman's energy invention (and we WON'T get into that!)

Good. That topic (combined with a complete absence of rational thought

by many) was why I left the usa-tesla list long ago.

Stamp out free-energy zealots whenever possible!

I'll be delivering the new 6.25 inch diameter secondary to Paul sometime

today.  Perhaps Paul will get a chance to evaluate it over this weekend!

:)  Oh, we don't expect to extract more energy from the coil than we provide it!

-Bill (who doesn't always let sleeping dogs lie)

Full-Name: Bert Wikkerink

Date: Fri, 04 Sep 1998 09:18:21 -0400

From: Bert Wikkerink <bert@mitra.com>

Organization: Mitra Imaging Inc.

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Starting to build my new coil

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Hi all,

Well, I plan on starting my new (and first) coil this weekend. I just

bought a 10' piece of 4" PVC for my secondary. I would like to build my

caps as well if I have time. Heck, I may even build my spark gap if

things go well. I have a GeoCities web page and will be posting all the

gory details as they become available.

Bert W.

http://www.geocities.com/CapeCanaveral/Launchpad/9408/

(no tesla stuff yet, but soon)

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: PVC

X-Mailer: IPAD 2.0

From: break7@juno.com (Tyla A. Break)

Date: Fri, 04 Sep 1998 10:18:05 EDT

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

I have 10" dia PVC white schd 40. Anyone in California can come

and get some. I also have a couple neons for anyone that has had trouble

getting them.

Let me know, Kyle Break7@juno.com

Date: Fri, 04 Sep 1998 07:58:15 -0700

From: Gene Waldo <gwaldo@1stnetusa.com>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: The MISSING pri coil form??

Sender: tesl-err@emachine.com

All,

The discussion of Pri forms seems to consist of 3 basic: solenoid,

saucer, and flat.  Assuming ~.2-.4" copper tube, etc.

Ignoring for the moment, their advantages of ease of tapping, how would,

say, a final, tweaked coil length do if it were insulated and wound into

a tight 'toroid' or compressed solenoid form?

I immagine that this would throw all inductance factors into a cocked

hat? Would this be compensatable by physical adjustments?

----------------------------------------------

Gene Waldo -  gwaldo@1stnetusa.com  - "Igor in Residence"

Date: Fri, 4 Sep 1998 09:11:26 -0600

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Daniel Lockhart <molly@cmn.net>

Subject: Re: R.E.  Transformers

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

Tyla A. Break wrote:

>A (Pole Pig) would be the next step. And do to the VERY high

>currents available, they are instantly fatal.   No Question!

>> snip! <<

>They're used for

>opening and closing oil filled switches to add capacitance to

>distribution lines. You'll see them up on poles with cap banks.

>

>	Hope you or anyone else finds this helpful

>

>			Kyle break7@juno.com

What kind of cap banks are these? What is their voltage rating and

capacitance? Can these be found at scrapyards too? I'm not interested in

them for coil use, I'm just always interested in big caps!

Thanks,

Daniel Lockhart

Date: Fri, 4 Sep 1998 09:24:28 -0600

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Daniel Lockhart <molly@cmn.net>

Subject: Earth Ground Locations

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

Hey all,

I'm wanting to install a good ground for my six inch coil that I'm in the

process of making (I didn't have one for my three incher :-). My coil is a

long way from being complete, but I figure I better install the ground

system before the snow flies (pounding metal rods through four feet of snow

and frozen ground doesn't sound fun). I just don't know where to install

the thing.

How long can the ground wire be from the ground rod(s) to the coil? I'm

trying to place the ground rods so that the lawn mower won't hit it in the

summer and the snowplow won't in winter. Not easy. Is twenty to thirty feet

an acceptable distance? More? Less?

I haven't though about it much, but I was thinking about using three, four

or five foot rods for the ground. Any longer and I'll hit a rock or real

compressed clay. Is this about right?

There is one spot where I could make it where there is a natural spring and

the ground in continually damp. Would this location be desirable?

Thanks,

Daniel Lockhart

X-Mailer: IPAD 2.0

Date: Fri, 04 Sep 1998 14:03:39 -0500

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: The MISSING pri coil form??

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

At 07:58 AM 9/4/98 -0700, you wrote:

>All,

>The discussion of Pri forms seems to consist of 3 basic: solenoid,

>saucer, and flat.  Assuming ~.2-.4" copper tube, etc.

>

>Ignoring for the moment, their advantages of ease of tapping, how would,

>say, a final, tweaked coil length do if it were insulated and wound into

>a tight 'toroid' or compressed sol. form?

>

>I immagine that this would throw all inductance factors into a cocked

>hat? Would this be compensatable by physical adjustments?

>

>----------------------------------------------

>Gene Waldo -  gwaldo@1stnetusa.com  - "Igor in Residence"

Gene,

I dont know about others but none of my coils have

ever been in a "final, tweaked" form. Im always changing this

or adjusting that. You really need adjustability in the primary coil.

Winding the primary into a tight or compressed form would

also adversely affect the coupling.

TTYL!

Mike Hammer

mhammer@misslink.net

Date: Fri, 4 Sep 1998 12:44:55 -0700 (PDT)

From: Sam Barros <sambarros@yahoo.com>

Subject: !!!PLEASE HELP A DESPERATE COILER!!!

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

Hi!

---Sorry for the long message, but PLEASE read on---

I've been on this list for about a week and havent yet introduced

myself, so I'll do that first:

Ok. My name is Sam Barros and I live in Holland (although I am not

Dutch). I experiment with electricity, chemistry, physics and other

fields of science as a Hobby. I have being doing research and

experimentation with High Voltages for over 4 years now and I decided

it was time to build my first Tesla Coil. I have a good understanding

of TC theory and workings, and am fully capable of building and

operating a TC without getting myself killed by electrocution (my

first High Voltage electric shocks were my last ones), but there are a

few issues I would like to get reviewed by someone with greater

experience in the field than me (i.e. Just about everyone on this list

who has actually built a Coil).

Ok. My design aims for a medium power, very reliable, good-looking

Tesla Coil. I'll be tapping its energy to conduct electromagnetic

repulsion, confinement and acceleration of plasma, so ground strikes

shouldn't be much of a problem.

The main thing I am looking for here is efficiency. I want this coil

to be as efficient as possible. Therefore I've given it a 3:1 H/D

ratio and an angled primary, together with the use of  0.60 (between

AWG 23 and 22) wire (any thicker would be seriously wasted by "skin

effect")

Well, here are the specs for my coil so far:

Secondary:

Coil form:

Diameter: 20cm (8in)

Winding Length: 60 cm (24in)

Wire gauge: 0.60mm (between 23 and 22AWG)

Spacing between turns: NONE

Secondary turns: 1000

Secondary wire length: 627.69m (2028.57FT)

Approximate resonant frequency: 186.11KHz

Secondary quarter wavelength resonant frequency: 119.49KHz

Secondary self-capacitance: 12.84pF

Toroid capacitance required to form quarter wavelength coil: 18.31pF

Primary:

Primary capacitance: 0.0127uF

Primary resonant frequency: 119.49KHz

Primary conductor diameter: 8mm (0.320in)

Primary turn to turn spacing: 8mm (0.320in)

Spacing between the secondary and inside turn of primary: 55mm (2.2in)

Primary tapped between turn 14 and 15 to form a resonant circuit at 119.49KHz

Neon transformer/Capacitor matching

Transformer input voltage: 220V

Transformer output voltage: 15KV

Transformer operating frequency: 50Hz

Transformer output current: 60mA

Ok. Now the problems begin:

1- I have 2 ferrite cores, such as those used in radios. They measure

14 x 1cm (5.6 x 0.4in) and I also have 60 meters of 0.60mm (AWG

23/22)wire, left from the Coil. Can I wind 2 good chokes using them?

Would I need more? I can get more of them, or a different wire, but no

different ferrite cores. How much inductance would be ideal for my

coil? I think 2mH should do, but I could be wrong...

2- Mike Hammer suggested I used the following primary: 18 turns of 8mm

(0.320in) thick copper tubing spaced 8mm (0.320in) from each other,

and with 3.5cm (1.4in) clearance from the secondary and a 15 degrees

angle. I agree that's a better option, but I am not sure if it's the

best option. I feel I can make the secondary/primary clearance less.

Perhaps 0.5in if I insulate the primary (the secondary is already very

well insulated with epoxy and PU sealer). And, such a primary would

have a diameter of about 80cm (32in). That's HUGE!!! It would need

such a massive base. Is that really the best configuration, or can I

make the primary coil shorter and therefore increase inductance? I

want the most efficient design, remember!

3- Here in Holland the largest transformers available (because of

legal restrictions) are rated 8KV/50mA. I've been to all 13 Neon

Shops within a 50 KM range (it took me 5 days) and NOT A SINGLE ONE OF

THEM WILL GIVE OR SELL ME A USED TRANSFORMER. Let alone the 3 I would

need to run my coil. A single new transformer here costs 300 dollars

(rated 10KV, 100mA. They come from Italy and are impossible to get

without an order)

Can some kind soul sell me a 1000W working, used neon transformer?

PLEASE!!! I really need this, and there is no other way I can get it..

I can pay about 50 dollars. It will cover the costs of transportation,

and should leave some extra for the kind soul that made my Tesla Coil

Project possible.

Thank you very much for taking your time and reading such a long

message (supposing you didn't just scroll down looking for the

"delete" button). Please help me if you can... Any questions,

comments, hints are very welcome.

THANKS!!!!

Sam Barros,

sambarros@yahoo.com

ICQ:15156975

"The less you know, the better you sleep"...

X-Mailer: IPAD 2.0

Date: Fri, 04 Sep 1998 15:02:31 -0500

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: Earth Ground Locations

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

At 09:24 AM 9/4/98 -0600, you wrote:

>Hey all,

>

>I'm wanting to install a good ground for my six inch coil that I'm in the

>process of making (I didn't have one for my three incher :-). My coil is a

>long way from being complete, but I figure I better install the ground

>system before the snow flies (pounding metal rods through four feet of snow

>and frozen ground doesn't sound fun). I just don't know where to install

>the thing.

Take it from someone who has pounded ground rods in January.

It aint easy.

>

>How long can the ground wire be from the ground rod(s) to the coil? I'm

>trying to place the ground rods so that the lawn mower won't hit it in the

>summer and the snowplow won't in winter. Not easy. Is twenty to thirty feet

>an acceptable distance? More? Less?

The shorter the better. Use what ever length is necessary but keep

it as short as is practical. With longer leads a heavier conductor

is necessary

>

>I haven't though about it much, but I was thinking about using three, four

>or five foot rods for the ground. Any longer and I'll hit a rock or real

>compressed clay. Is this about right?

Yes. Several short rods connected in parallel are more effective than

a single long one.

>

>There is one spot where I could make it where there is a natural spring and

>the ground in continually damp. Would this location be desirable?

Yes this would be desirable if it doesnt mean a very long ground lead.

You may get better results with a less desirable location and a short

ground lead.

TTYL!

Mike Hammer

mhammer@misslink.net

X-Mailer: IPAD 2.0

Date: Fri, 04 Sep 1998 16:30:37 -0500

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: hot resistors/safety gaps

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

At 08:52 AM 9/4/98 +1200, you wrote:

>Hi Mike,

>

>>>                    #1          #2             #3             #4

>>>

Transformer =   12kv/100ma    12kv/30ma      15kV/30mA       unknown

Gap Type =       9@.025       9@.025         10@.03

Total gap =      .225         .225           .300

Primary cap =    .015        . 0056          .0045

cap type =      rolled poly   SW bottle      rolled poly

Resistors =     310 ohm 50W   330 Ohm  2W    500 Ohm 25W

Choke value =   2.25mH        4.1 mh         2.1mh

BPS =            120?          ?              ?

Resistor Heat = COLD          warm           burnt

Res Freq  =     254 khz       ~275khz        323khz

Max output      54 inches      22 inches     24 inches

Lp =            26uH           60uH          54uH

Pri Material =  AWG6 solid    AWG12 solid    3/16 tubing

>However, it seems to me that keeping the gap quiet for as long as

>possible is a factor so transients generated by it are also a factor

>as you suggest. At k=0.22 with first notch quench, your gap is seeing

>primary current for just 9uS. I wonder what the others are doing. The

>difference between 2 and 3 could be the difference between quench

>ability which will be harder for air streamers.

>     There is also another factor to be considered - physical

>construction. What is the relative quality of the primary "wiring"

>in each system? I wonder if the difference lies there as well?

I have included the primary coil material now in the above table.

I did talk briefly to Gary Weaver on IRC last night and the

coil of his in question no longer exists. He has used parts of it

for other projects. He will try to fill in as much of the data

as he can.

What about some of the rest of you? There are quite a few coils

out there and Im sure some of you are using series damping

resistors. Step in here and let us know what results you have.

Even if you have absolutely no problems we would still like

to know that. But we are most interested in any of you that

have experienced any overheating problems. Dont worry if you

dont know all the specs on your coil. Just fill in what you know.

TTYL!

Mike Hammer

mhammer@misslink.net

From: ESchulz531@aol.com

Date: Fri, 4 Sep 1998 18:36:35 EDT

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: hot resistors/safety gaps / #4 coil

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

#1            #2             #3

Transformer =   12kv/100ma    12kv/30ma      15kV/30mA

Gap Type =      9@.025        9@.025         10@.03

Total gap =     .225          .225           .300

Primary cap =   .015          .0056          .0045

cap type =      rolled poly   SW bottle      rolled poly

Resistors =     310 ohm 50W   330 Ohm  2W    500 Ohm 25W

Choke value =   2.25mH        4.1 mh         2.1mh

BPS =           120?          ?              ?

Resistor Heat = COLD          warm           burnt

Res Freq  =     254 khz       ~275khz        323khz

Max output      54 inches     22 inches      24 inches

Lp =            26uH          60uH           54uH

Pri Material =  AWG6 solid    AWG12 solid    3/16 tubing

#4            #5             #6

Transformer =   12kv/100ma

Gap Type =      30@.01

Total gap =     .300

Primary cap =   0.01

cap type =      salvage GE

Resistors =     300 ohm 12W

Choke value =   4.66 mh

BPS =           120

Resistor Heat = COLD

Res Freq  =     206 khz

Max output      54 inches

Lp =            58 uH

Pri Material =  3/8 tubing

Erik Schulz

http://members.aol.com/tesla00/

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Date: Fri, 04 Sep 1998 19:52:11 -0700

From: "paul m" <fleischwulf@mailexcite.com>

Cc:

X-Sent-Mail: off

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

Subject: First power up for the 6.5" coil "WOW"

X-Sender-Ip: 208.16.146.97

Organization: MailExcite  (http://www.mailexcite.com:80)

Sender: tesl-err@emachine.com

Well coilers Bill and I finally got around to testing our 6.5" coil. It was a

run for cover situation. I fired it at only 3.5 Kw. The streamer length could

not be determined. The streamers terminated in all directions to what ever ground

was available. The greatest distance is about 65" "This was no problem at all".

I'll have to move it outside to get the distance. The arcs seem a lot hotter than

the ones we got from the 4". The run time was approximately 20 to 30 sec. This coil

does not seem to be as sensitive to tunning as the 4" coil. The coil finally failed

when the rotary gap found a new path to ground an all of the little electrons followed

the leader. A screw in the base of the gap actually arced through a 1/2 poly support

to ground.  I think this coil is going to be a real winner. For a while I thought I

was going to have to run for cover.

Have a nice holiday

The paul & bill show,

paul m

---

Einigheit und Wahrheit

X-Originating-IP: [208.23.200.80]

From: "Bill the arcstarter" <arcstarter@hotmail.com>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: First power up for the 6.5

Date: Fri, 04 Sep 1998 20:16:35 PDT

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

Paul wrote:

>The greatest distance is about 65" "This was no problem at all".

>I'll have to move it outside to get the distance. The arcs seem

>a lot hotter than the ones we got from the 4".

Paul - save some for ME to see!

(I wasn't actually present for the power-up- I just gave him the

secondary at noon!  I was busy asking permission to "acquire" a nice,

lonely-looking 1970's vintage 140KV 60 KVA X-ray transformer from the

loading dock at work... heh heh heh ).  With voltages like that coming

RIGHT out of the CAN- you don't NEED any resonance to put on a show!

-Bill (yes I know.  I'm sick!  And everyone's jealous!)

Starting arcs in Cinci, OH

http://www.geocities.com/CapeCanaveral/Hangar/6160

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Big Cappers!

X-Mailer: IPAD 2.0

From: break7@juno.com (Tyla A. Break)

Date: Sat, 05 Sep 1998 00:29:44 EDT

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Daniel, and anyone interested. The cap banks I was speaking of

are for power factor correction. They're big.  About 18" x 12" x 4"

thick. I don't know the exact values. they're rated in kvar . The

voltages are as high as 40kv.  They are available at surplus stores.

Stores usually ask $50-150 each. I just looked at some two weeks ago at a

salvage yard. The only rating on them was 60 kvar's. Does anyone know

what kvars are? Anyway the guy I was talking to would probably sell for

about $10 each! Not only that, they had never been used. I'm going back

in a week or two I'll check em again.  Kyle Break7@juno.com

From: McFalc0n@aol.com

Date: Sat, 5 Sep 1998 00:39:27 EDT

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: New member

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Hi, i'm a high school student who's been tinkering around with high

voltage for about four months (while reading for lots lots more!!!).  I've

built and played with my jacobs ladder, and got interested in Tesla coils.

I decided to build a medium powered one and tinker around with it.

My coil is 450 watts.  I got the NST (15KV, 30MA) and built a RQ

cylindrical spark gap with eight gaps spaced .025" apart.  (not too bad for

someone who's never drilled pipes in PVC before).  I made a 0.005 uf 50,000V

AC Rolled Poly Capacitor that was really cheap.  I got my secondary form, and

it's a white PVC sewer drain pipe with really thin walls for PVC.  I sanded

and cleaned it.  Is it ok that I used Sewer Pipe?  I've been thinking about

building a 30 degree primary out of quarter inch copper tubing.  I also made a

18 by 3 inch toroid out of pie plates, slinkys , and aluminum tape.  Would the

toroid work?

Any sugestions would be great.      Thanx.

From: McFalc0n@aol.com

Date: Sat, 5 Sep 1998 00:49:29 EDT

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Idea and Question

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Primary coil idea:  I heard that copper strapping gave off too much corona

for use in a primary coil, but could you enclose it and bury it with mineral

oil?  wouldn't this also make the coil able to be smaller too?  Would it work?

Which is better to use for the primary tank circuitry's wiring:  HV neon

wiring for the 15KV 30MA transformer or 30 gauge copper stranded wire with

homemade insulation?  Which would hold the current and heat better?

What improvements are necessary to make a tesla coil able to run for a really

long time like 30 minutes?

What should be done to make the primary tank circuit more effective and efficient?

Is a cooling fan really necessary for a 450 watt coil on the spark gap?

And last, does putting a metal plate below the primary that is grounded when

the RF ground is far away help performance?

Sorry if it's a long list of questions, but I'm a little curious.

PS:  Any more tips for safety that aren't in any of the safety documents you've seen?

Thanx.

X-Originating-IP: [208.16.149.155]

From: "Bill the arcstarter" <arcstarter@hotmail.com>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Big Cappers!

Date: Sat, 05 Sep 1998 05:39:18 PDT

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

It was written to tesla-2:

(about surplus power factor correction capacitors)

>are for power factor correction. They're big.  About 18" x 12" x 4"

>thick. I don't know the exact values. they're rated in kvar . The

>voltages are as high as 40kv.  They are avail. at surplus

>stores. Stores usually ask $50-150 each. I just looked at some two weeks

ago at a

>salvage yard. The only rating on them was 60 kvar's. Does anyone know

>what kvars are? Anyway the guy I was talking to would probably sell for

>about $10 each! Not only that, they had never been used. I'm going back

You can learn more about this from Jim Lux's site at:

http://home.earthlink.net/~jimlux

http://home.earthlink.net/~jimlux/hv/hvmain.htm

http://home.earthlink.net/~jimlux/hv/varvom.htm

http://home.earthlink.net/~jimlux/hv/pfc.htm

VARS stands for Volt Amps, Reactive.  It is a measure of the "out of

phase" power drawn by a particular load.

Basically, if you have a PFC capacitor with a rated KVAR value, freq,

and nominal voltage, here is how you convert that back to capacitance:

C = vars/(2*PI*f*v^2)

Where:

C = Farads

PI= 3.1415 or your favorite approximation...

f = Line freq in Hz, usually 60 Hz

v = Nominal voltage, in volts

So a 60KVAR 60Hz 14400 volt cap (a guess on the voltage) would be a:

C = 60000/(2*3.1415*60*14400^2) =  0.767 uF cap.

(with a peak energy of 0.5 * C * V^2 = 72 Joules.)

I'm no expert on this, but I'm fairly sure this is how it works!

Note - I'm not sure that PFC caps will work as tesla coil primary caps

since they are designed to run at 60Hz, not 250 KHz!

-Bill the arcstarter

Starting arcs in Cinci, OH

http://www.geocities.com/CapeCanaveral/Hangar/6160

Date: Sat, 05 Sep 1998 08:09:19 -0500

From: Bert Hickman <bert.hickman@aquila.com>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Organization: Stoneridge Engineering

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Big Cappers!

Sender: tesl-err@emachine.com

Tyla A. Break wrote:

>

>         Daniel, and anyone interested. The cap banks I was speaking of

> are for power factor correction. They're big.  About 18" x 12" x 4"

> thick. I don't know the exact values. they're rated in kvar . The

> voltages are as high as 40kv.  They are avail. at surplus stores.Stores

> usually ask $50-150 each. I just looked at some two weeks ago at a

> salvage yard. The only rating on them was 60 kvar's. Does anyone know

> what kvars are? Anyway the guy I was talking to would probably sell for

> about $10 each! Not only that, they had never been used. I'm going back

> in a week or two I'll check um again.  Kyle Break7@juno.com

>

Kyle and all,

These are power utility Power Factor Correction (PFC) caps. These will

typically use mylar and oil-soaked kraft paper, or oil-soaked kraft

paper as the dielectric, although some newer ones use

polypropylene-kraft paper. Unfortunately, they are not generally

suitable for Tesla Coiling.

There will generally be four faceplate ratings on these capacitors:

operating voltage, operating frequency (not always stated), KVARS

(KiloVolt-Amperes Reactive), and BIL (Basic Impulse Level) - the rated

transient voltage that these will withstand from lightning transients.

These caps often have internal bleeder resistors so that they will

safely discharge after several minutes. They are very rugged, and

extremely dangerous when charged. They are used to help neutralize the

phase shift between the voltage and current for inductive loads,

allowing them to run their power distribution systems more efficiently.

The capacitance of a unit can be computed from the KVAR, operating

voltage (kilovolts), and the frequency, using the following formula:

C (uF) = 1000*KVAR/(2*Pi*F*V^2)

Where  KVAR = Rated faceplate KVAR

Pi = 3.14159...

F = 60

V = in kilovolts

For example, a 60 KVAR capacitor, running at 60 Hz, and rated at 7.2 kV

would have a capacitance of slightly more than 3 uF.

BTW, if you do decide to purchase these, make sure they do NOT contain a

PCB-based dielectric fluid. These should have a label which clearly

states "No PCB's"... otherwise the odds are quite high that they do

contain PCB's. Any PFC caps made before 1980 should also be suspect.

Buying any large capacitors, containing PCB's is asking for a BIG

headache should they rupture, or when the time comes for YOU to dispose

of them safely. And, many states require you to register the fact that

you even own them! I've often been offered large PCB caps for free,

thereby saving the donor the disposal cost (~$100) to shred and

incinerate them. While PCB's are not in themselves very toxic, they are

extremely resistant to natural breakdown processes, and can lead to long

term contamination and pollution problems. Out of curiosity, where are

you geographically located, Kyle?

-- Bert --

Date: Sat, 05 Sep 1998 09:30:55 -0500

From: Bert Hickman <bert.hickman@aquila.com>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Organization: Stoneridge Engineering

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Idea and Question

Sender: tesl-err@emachine.com

My responses are interspersed below...

McFalc0n@aol.com wrote:

>

>         Primary coil idea:  I heard that copper strapping gave off too much corona

> for use in a primary coil, but could you enclose it and bury it with mineral

> oil?  wouldn't this also make the coil able to be smaller too?

> Would it work?

Probably. However, using a larger tank cap and smaller tank inductance

will also allow you to use physically smaller primary. You could also

run a strip of 1/8" x 1" polypropylene strip in a spiral fashion between

the turns to further decrease the primary turn-turn spacing. Most

coilers find they have no problems when using common refrigerator

tubing, since it's much easier to find, and is usually fairly

inexpensive. Stay away from oil immersion.. it's messy and really not

necessary. :^)

> Which is better to use for the primary tank circuitry's wiring:  HV neon

> wiring for the 15KV 30MA transformer or 30 gauge copper stranded wire with

> homemade insulation?  Which would hold the current and heat better?

Neither! The tank circuit wiring should be fairly heavy, since there may

be hundreds of amps flowing through the tank cap, spark gap, and the

primary winding when the gap fires. Use heavy gauge wire (#6 or larger),

and insulate with 1/8" wall flexible PVC tubing if necessary.

> What improvements are necessary to make a tesla coil able to run for a really

> long time like 30 minutes?

Reduce your losses, particularly in the gap and tank cap. Tank caps heat

up due the the RF and high voltage stresses they see. The deep interior

of the cap may build up heat faster than it can be dissipated... and an

extended run may actually melt a hole in the dielectric, shorting it

out. Similarly, neon sign transformers may not be able to withstand the

load (particularly if you've "sized" the tank cap to match the size of

your neon, as many Tesla Coil design programs do), and may overheat and

short-out. The gap may be dissipating anywhere from 20% - 50% of the

total power you're putting into your coil - it must be kept cool enough

to continue quenching without overheating. This is usually done via

forced-air cooling. The UV light given off from the sparkgaps can also

bleach or "solarize" nearby materials, as well as giving you an

unintended sunburn.

If you ran the coil indoors, you can easily build up ozone and nitrogen

oxides to dangerous/toxic levels. Practically speaking, you should also

be considerate of your neighbors who'd like to use their radios,

cordless phones - limit your run times. Check out the safety sheet

referenced below for more information on the known hazards of coiling.

>What should be done to make the primary tank circuit more effective

>and efficient?

Air-cooled multiple gaps, heavy tank circuit wiring, and an efficient

(polypropylene or polyethylene) HV tank cap.

>Is a cooling fan really necessary for a 450 watt coil on the spark gap?

For best operation, yes. Not only to keep the gap cool, but to also help

extinguish the arc at the proper time for best coil performance

("quenching").

> And last, does putting a metal plate below the primary that is grounded when

> the RF ground is far away help performance?

As long as it's not so close to the primary winding that you get large

eddy-currents induced into the plate. The use of a conducting sheet,

plate, foil, (or even chicken wire) below the secondary coil/toroid can

be useful for SMALL coils when a good RF ground is unavailable. This is

called a counterpoise. The base of the coil is connected to this

conductor. The diameter of the counterpoise should be at least the

1-1.5X the height of the top terminal from the floor. For example, if

your small coil is sitting on top of a table, and the top terminal is 5

feet from the floor, your counterpoise should be at least 5-8 feet in

diameter. BTW, the counterpoise WILL be electrically energized when the

coil is running! It's much more desirable to run with a solid RF ground

if at all possible!

>  Sorry if it's a long list of questions, but i';m a little curious.

> PS:  Any more tips for safety that aren't in any of the safety documents

> you've seen?

> Thanx.

Check out the safety sheet at Chip Atkinson's site:

http://www.pupman.com/

And, safe coilin' to you!

-- Bert --

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: R.E. Big Cappers

X-Mailer: IPAD 2.0

From: break7@juno.com (Tyla A. Break)

Date: Sat, 05 Sep 1998 11:50:02 EDT

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

I live in California Los Angles, the arm pit of the U.S.  Coil quick

before they make it illegal!  The only good things are, everything you

can imagine is about 15 minutes away,and the weather is warm when

everyone else is freezing.

Kyle  Break7@juno.com

Date: Sat, 05 Sep 1998 09:57:50 -0700

From: Gene Waldo <gwaldo@1stnetusa.com>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: First power up for the 6.5" coil "WOW"

Sender: tesl-err@emachine.com

paul m wrote:

>

> Well coilers Bill and I finally got around to testing our 6.5" coil.

> I thought I was going to have to run for cover.

>

> Have a nice holiday

>

> The paul & bill show,

>

> paul m

> ---

Paul & Bill,

Congrats!!  I'll open a new bottle of Scotch and ponder my own inertia!

Can someone supply data and figures on the dangers of playing in or near 65"?

A 'FARADAY SUIT' and some James Whale immages come to mind....

--

Gene Waldo -  gwaldo@1stnetusa.com  - "Igor in Residence"

---------------------------------------------------------------

What's good for the planet is more OZONE! See the machinery form up!

NEW PAGE!   http://members.1stnetusa.com/danmurphy/waldo.htm

X-Originating-IP: [208.16.145.146]

From: "Bill the arcstarter" <arcstarter@hotmail.com>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Current transformers?

Date: Sat, 05 Sep 1998 15:16:59 PDT

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

Coilers,

I just got back from Mendelssons's surplus, up in Dayton.  Of course I

was looking for good coiling stuff, like potential transformers, pigs, etc.

The smallest pig they had was 900 lbs!  Geez!  They also had two

refrigerator-sized 500KVA units, 120v to 12300 3phase  That'd work, right? ;)

I did find two potential transformers, but they were 2:1 and 4:1 ratios,

for 240 and 480 volt service only.

But - they did have a large collection of current ratio transformers, of

2000:5, 3000:5 and 4000:5 ratios.

My question is this:  Can these current ratio transformers be used for

coiling?  I'd guess not since the insulation level isn't up to the

levels we need.  Thoughts?

Just thought I'd ask.

-Bill the arcstarter

Starting arcs in Cinci, OH

http://www.geocities.com/CapeCanaveral/Hangar/6160

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Bill's Baby!

X-Mailer: IPAD 2.0

From: break7@juno.com (Tyla A. Break)

Date: Sat, 05 Sep 1998 23:21:08 EDT

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Hi guys, Bill esp. I just saw a potential transformer today !

120:1  120v - 14400V 1750W . Exactly like the one I already have.

Seventy-two pounds. How much would it cost to ship that?  I got the last

one for $50 , I'm trying to pry it away from the owner. It's the last one

and he wants to hold on to it. But I think I can get him to give it up.

On a different subject, does anybody find it challenging to get a

square cut on 10-12" PVC?

A  32" piece wound with #20 should get 1000 turns .

Got a 32" primary -12 turns of .375 copper tubing, Plenty pole-pigs. The

.05+ cap is going to be some work! Does anyone know how to form a trash

can into a toroid? A little arc welding cable for hook-up tap the

electric service, Feeding time!  Sound good to anyone? Not trying to

out-do anyone, I just like a light show.

Kyle							break7@juno.com

X-Originating-IP: [208.16.149.182]

From: "Bill the arcstarter" <arcstarter@hotmail.com>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: MOT saturation tests

Date: Sat, 05 Sep 1998 20:51:44 PDT

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

Coilers,

I finally got around to performing my microwave oven transformer

saturation tests.  The goal was to characterize the primary winding in

terms of current - ie - see just how nonlinear this reactance is as a

function of current.

Setup:

* One MOT (out of a 120vac 60Hz oven)

* This MOT contains about 122 turns of wire on the primary.

* No secondary windings (hacked off the core)

* Shunts undisturbed (left in place).

* The resistance of the primary winding is 0.32 ohms +-0.04 ohms.

*

* Two generic non RMS digital volt/ammeters (Alfa electronics)

*

* Variac.

*

* Fire extinguisher (never hurts...)

Procedure:

Apply various values of 60 Hz voltage.

Record drawn current (as sensed by an inline DVM ammeter).

Results:

V=volts, I=amps, Z=complex ohms

V   I     Z(approx'd as Z=V/I) (Neglecting the 0.32 ohm primary)

50  0.26  192.

60  0.31  194.

70  0.38  184.

80  0.50  100.

90  0.73  123.

100  1.16   86.2

110  2.14   51.4

120  3.86   31.1

130  5.90   22.0

Clearly this core is starting to do "something odd" even for low

currents near 0.5 amps.  An ideal linear (non-saturating) inductance

would show no change in Z as the applied voltage was changed.  Perhaps

this is the action of the shunts?

Interestingly enough - my LCR DVM indicated an inductance of 0.065 H, or

Z=24.5 complex ohms, which is surprisingly close to the measured value

at around 120-130 volts!  I wonder how it 'knew' that?

I'm not sure what conclusions to draw from this data, only that using

MOTs as current limiters will most definitely introduce some odd

nonlinearities as a function of the current, especially at either higher

currents or voltages.  (We've used MOTs as current limiters on the 240 v

side of our pig.)

Possible future directions:

*I suppose I should have measured things at voltages <50V...

*Maybe I'll knock the shunts out tomorrow and redo the measurements.

*Maybe I'll fire up my Tek Type 543B oscilloboatanchoroscope...

*Running the tests at voltages over 130 volts might be amusing!

*Make measurements using a MOT w/ the secondary shorted-out.

Comments invited.

-Bill the arcstarter

Starting arcs in Cinci, OH

http://www.geocities.com/CapeCanaveral/Hangar/6160

X-Originating-IP: [208.16.149.182]

From: "Bill the arcstarter" <arcstarter@hotmail.com>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Bill's Baby!

Date: Sat, 05 Sep 1998 21:01:33 PDT

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

It was written:

>	Hi guys, Bill esp. I just saw a potential transformer today !

>120:1  120v - 14400V 1750W . Exactly like the one I already have.

>Seventy-two pounds. How much would it cost to ship that?  I got the last

>one for $50 , I'm trying to pry it away from the owner. It's the last one

Ooh!  Ooh!  Go get it and drop-ship it directly to me for, ahem, evaluation!

Shipping - where is it coming from?  I was once quoted $100 to ship a

120 pound object about 400 miles by truck.

>	On a different subject, does anybody find it challenging to

>get a square cut on 10-12" PVC?

12 inch?  Why don't you build a nice small sedate one like our 6.25

incher first! :)

Square cuts:  For smaller PVC I use a large piece of newspaper, which I

wrap around the PVC and align the edges back together.  Then I trace the

edge around the PVC w/ a black marker or something.  I learned this

trick from an auto repair guy who was cutting and re-welding driveshafts

in his garage w/o the traditional lathe!

Chuck Curran once jigged up a router such that, with his winding motor

revolving the PVC - he'd feed the router into the end of the PVC -

making a nice square cut.

>out-do anyone, I just like a light show.

Same here.  Thats why I do all this stuff... :)

-Bill the arcstarter

Starting arcs in Cinci, OH

http://www.geocities.com/CapeCanaveral/Hangar/6160

Date: Sun, 06 Sep 1998 10:56:33 -0700

From: Gene Waldo <gwaldo@1stnetusa.com>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: L.A. Basin coilers:

Sender: tesl-err@emachine.com

If interested, send me your E-m's and ZIP.

I'll plot ZIPs and maybe we can drum up a dinner-meet or some such, at a

central localle.... trade horror stories etc. ????

--

Gene Waldo -  gwaldo@1stnetusa.com  - "Igor in Residence"

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: So. Cal Coilers

X-Mailer: IPAD 2.0

From: break7@juno.com (Tyla A. Break)

Date: Sun, 06 Sep 1998 15:38:04 EDT

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

I live in La Canada , Between Pasadena and Glendale.  Hey APEX is

the place!  Also L.A. DWP salvage, Joe Factor, C&H, among others. Is

anybody familiar with Surplus Center (Burden Sales)? I'll E-Mail and chat

soon as time permits.Kyle Break7@juno.com

X-Mailer: IPAD 2.0

Date: Sun, 06 Sep 1998 16:34:28 -0500

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: hot resistors/safety gaps

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

At 06:36 PM 9/4/98 EDT, you wrote:

>                     #1            #2             #3

> Transformer =   12kv/100ma    12kv/30ma      15kV/30mA

> Gap Type =      9@.025        9@.025         10@.03

> Total gap =     .225          .225           .300

> Primary cap =   .015          .0056          .0045

> cap type =      rolled poly   SW bottle      rolled poly

> Resistors =     310 ohm 50W   330 Ohm  2W    500 Ohm 25W

> Choke value =   2.25mH        4.1 mh         2.1mh

> BPS =           120?          ?              ?

> Resistor Heat = COLD          warm           burnt

> Res Freq  =     254 khz       ~275khz        323khz

> Max output      54 inches     22 inches      24 inches

> Lp =            26uH          60uH           54uH

> Pri Material =  AWG6 solid    AWG12 solid    3/16 tubing

>

>                     #4            #5             #6

> Transformer =   12kv/100ma    15kv/50ma

> Gap Type =      30@.01         8@.025

> Total gap =     .300           .200

> Primary cap =   0.01           .0064

> cap type =      salvage GE     rolled poly

> Resistors =     300 ohm 12W    300 ohm 50W

> Choke value =   4.66 mh        none

> BPS =           120            120

> Resistor Heat = COLD           very warm

> Res Freq  =     206 khz        399 khz

> Max output      54 inches      27 inches

> Lp =            58 uH          46 uh

> Pri Material =  3/8 tubing     1/4 tubing

>

Erik,

Thanks for your coil information. I finally got to talk

to Gary Weaver about his coil. The coil that burned up resistors

no longer exists but he tried some out on one of his existing

coils and reported the results. He tried 3 values, 200 25 W, 300 50W

and 430 20 W. All resulted in pretty much the same amount

of resistor heat. They all got pretty warm after a few minutes

of operation.

One unexpected result from his test was than he got LONGER but

thinner streamers with HIGHER resistor values. Not sure I can

explain that one.

Anyone else want to offer some info on their coil? Come on

now. Can't lurk forever.

TTYL!

Mike Hammer

mhammer@misslink.net

X-Mailer: IPAD 2.0

Date: Sun, 06 Sep 1998 16:44:50 -0500

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: Current transformers?

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

At 03:16 PM 9/5/98 PDT, you wrote:

>Coilers,

> I just got back from Mendelssons's surplus, up in Dayton.  Of course I

>was looking for good coiling stuff, like potential transformers, pigs,

>etc.

>

>The smallest pig they had was 900 lbs!  Geez!  They also had two

>refrigerator-sized 500KVA units, 120v to 12300 3phase  That'd work,

>right? ;)

>

>I did find two potential transformers, but they were 2:1 and 4:1 ratios,

>for 240 and 480 volt service only.

>

>But - they did have a large collection of current ratio transformers, of

>2000:5, 3000:5 and 4000:5 ratios.

>

>My question is this:  Can these current ratio transformers be used for

>coiling?  I'd guess not since the insulation level isn't up to the

>levels we need.  Thoughts?

>

>Just thought I'd ask.

>

>-Bill the arcstarter

Bill,

Current transformers are used for metering purposes.

They are used in conjunction with an ammeter to measure

high current levels.

They usually have a single turn primary and a multiple turn

secondary. Their only use would be for measuring current.

The ratios you listed would be for measuring quite large

currents with a 5 amp meter movement. When you start using

that kind of current in your experiments Im moving farther

away. A one state buffer zone isnt enough.

Other than that they dont have much practical use.

Ive seen them at hamfests for a couple bucks apiece.

TTYL!

Mike Hammer

mhammer@misslink.net

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: "Coiler" <mycroft@access1.net>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Safety Gap problems after reconfig

Date: Sun, 6 Sep 1998 15:59:29 -0700

X-Mailer: IPAD 2.0

Importance: Normal

X-MimeOLE: Produced By Microsoft MimeOLE V4.72.3155.0

Sender: tesl-err@emachine.com

Ok, now I am stumptified.

I built a new enclosure for my caps, a way to locate them under my coil (as

I was getting strikes that made me *very* nervous). Since I no longer needed the

flexiblity, I replaced all of my 14AWG Neon wire with 12AWG Solid wire. Most

of the runs less than 1/2 the length of before. My performance has now gone

into the toilet, and my SG is going nuts. Both the one across my neon, and the

one across my cap.

No manner of tuning seems to help.

One other interesting twist: This coil used to have to work real hard to get

the first streamer, then it cooked. Now it throws anywhere from 4-10

streamers at once, though much shorter.

Someone with experience in this please.. what the heck did I break?

[This coil has given me 37" streamers before], oh.. and on the resistor note..

them suckers get *REALLY* hot now.. but then the SG is dumping a lot from the cap.

Michael Baumann

Coiler, Homebrewer, Nerd. mycroft@access1.net

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Date: Sun, 06 Sep 1998 18:11:28 -0700

From: "paul m" <fleischwulf@mailexcite.com>

Cc: "andy marshall" <amarshal@burke.com>

X-Sent-Mail: off

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

Subject: Preformance update 6.5"

X-Sender-Ip: 208.16.146.105

Organization: MailExcite  (http://www.mailexcite.com:80)

Sender: tesl-err@emachine.com

Coilers

Well I moved as much stuff out of the way as I could before resuming the test.

The first time I fired it, I got 65" arc's. Tonight the streamers reached 74"

(Power level 3.5 - 4 Kw). The coil still needs to be fine tuned. This coil looks

like a real performer. If I move things around a little, I think I can reach 85 - 90" beyond this I will have to move out side. The furnace has already taken a few good

hits along with the caps and rotary. Here are the spec's.

Transformer: Sec 14.4 Kv, 25 KVA. (Current limited)

Capacitance: 2 parallel caps in a series circuit total capacitance .026uF

Gap: Rotary

Primary: 3/8 copper tubing with 3/8" spacing between turns. 14 turns.

Inductance: 75 uH (Measured)

Secondary: 6.5" coil form, 22 AWG (26 - 27" winding length) Measured inductance 35 mH.

Top load: Made from 7" diameter dryer duct 28" dia. This rests on a toroid made from 4" dryer duct 20" dia.

More test to follow

Paul M

X-Mailer: IPAD 2.0

Date: Sun, 06 Sep 1998 20:55:50 -0500

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: Safety Gap problems after reconfig

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

At 03:59 PM 9/6/98 -0700, you wrote:

>Ok, now I am stumptified.

>I built a new enclosure for my caps, a way to locate them under my coil

>(as I was getting strikes that made me *very* nervous). Since I no longer

>needed the flexiblity, I replaced all of my 14AWG Neon wire with 12AWG

>solid wire. Most of the runs less than 1/2 the length of before. My performance

>has now gone into the toilet, and my SG is going nuts. Both the one across my

>neon, and the one across my cap.

>

>No manner of tuning seems to help.

>

Michael,

I have heard of this happening before. Your safety gaps are trying

to tell you something. Your primary voltage is way up for some

reason. I would bet without those safety gaps you would have

already toasted your cap and transformer.

Is your neon and main cap matched for 60 hz resonance? If so

this might be why the voltage is so high. Are you using a variac?

If so does your main gap fire at quite low variac settings? This is

a sure sign of 60 hz resonance and the voltage rise that goes with it.

Improving your tank circuit wiring probably improved the Q

of your primary circuit. This will also tend to make tuning

more critical. Poor tuning can also cause excessive safety gap

firings. On my coil I find if Im off tune with too much primary

inductance I have lots of kickbacks. Off tune the same amount

with too little inductance runs smoothly.

Here is what I would do. Back off your main gap a bit. This should quiet things

down some and your cap and neon will thank you. Reduce your coupling somewhat.

This should also tame things down. The idea is to get back to a smoother running

setup.

>One other interesting twist: This coil used to have to work real hard to get

>the first streamer, then it cooked. Now it throws anywhere from 4-10

>streamers at once, though much shorter.

This is usually an indication of too little toroid. Your coil before was having

a hard time reaching the breakout voltage of your toroid. Now it is easily reaching

that voltage. Try using a point discharger on your toroid to concentrate the streamers.

I would increase the minor diameter (thickness) of your toroid.

>

>Someone with experience in this please.. what the heck did I break?

You didnt really break anything. You just changed the characteristics of

your coil. You may have to do a little experimenting to get the individual

parts working well as a system once again.

>

>[This coil has given me 37" streamers before], oh.. and on the resistor

>note.. them suckers get *REALLY* hot now.. but then the SG is dumping a lot from

>the cap.

Yea you are getting some pretty severe kickbacks. All that energy has to go somewhere.

Better off heating up resistors than your neon secondary.

>Michael Baumann

>Coiler, Homebrewer, Nerd. mycroft@access1.net

Hope this helps Michael. Keep us posted and keep at it.

TTYL!

Mike Hammer

mhammer@misslink.net

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: "Coiler" <mycroft@access1.net>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: RE: Safety Gap problems after reconfig

Date: Sun, 6 Sep 1998 20:05:00 -0700

X-Mailer: IPAD 2.0

Importance: Normal

X-MimeOLE: Produced By Microsoft MimeOLE V4.72.3155.0

Sender: tesl-err@emachine.com

>

> Michael,

> I have heard of this happening before. Your safety gaps are trying

> to tell you something.

Yeah.. Like "Give me a break!!!" :)

> Your primary voltage is way up for some

> reason. I would bet without those safety gaps you would have

> already toasted your cap and transformer.

>

> Is your neon and main cap matched for 60 hz resonance? If so

> this might be why the voltage is so high. Are you using a variac?

> If so does your main gap fire at quite low variac settings? This is

> a sure sign of 60 hz resonance and the voltage rise that goes with it.

Yes. And yes, using a  variac. And the main gap starts to fire smoothly at

about 30VAC input to the trannies.. so yes, I would say I am resonant.

>

> Improving your tank circuit wiring probably improved the Q

> of your primary circuit. This will also tend to make tuning

> more critical. Poor tuning can also cause excessive safety gap

> firings. On my coil I find if Im off tune with too much primary

> inductance I have lots of kickbacks. Off tune the same amount

> with too little inductance runs smoothly.

Well, I just did a quick test (no time tonight) and found that I can

go -1 full turn and still get the kicks (but the gap across the cap

never fired) I don't get output either.

>

> Here is what I would do. Back off your main gap a bit. This

> should quiet things down some and your cap and neon will thank you.

> Reduce your coupling somewhat.  This should also tame things down.

> The idea is to get back to a smoother running setup.

>

Hmm. May have to go to a flat wound primary I guess. I am already 3" above

the plane of the lower turn of my primary

> I would increase the minor diameter of your toroid.

*gahhh* Already at 6".. at this rate the topload will dwarf the coil :)

And I'll have to add another 4-5 turns to my primary, guess my original

guess of 20 was closer to the mark than the 14 I wound up with.

>

>

> >

> >Someone with experience in this please.. what the heck did I break?

>

>

> You didnt really break anything. You just changed the characteristics of

> your coil. You  may have to do a little experimenting to get the

> individual parts working well as a system once again.

Yeah.. just in time to scrogg the works again when my pig arrives.. :)

But I'll definitely keep all up to date.

Thanks for the info.

Michael Baumann

Coiler, Homebrewer, Nerd. mycroft@access1.net

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Build the Secondary

X-Mailer: IPAD 2.0

From: break7@juno.com (Tyla A. Break)

Date: Mon, 07 Sep 1998 00:29:56 EDT

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Hi all. I'm working on a big coil. Mike's instructions seem to be

the best. But I don't have them anymore. My little 25Mhz computer was

having trouble handling the 100+ messages from the list this last week!

So your instructions got deleted by accident. I've tried to apply what I

remembered. I sanded with #100 then washed with scotch-bright pad. Then

sanded with #220 , then #400. RE-washed and dried. I heated the pipe to

approx. 130 degrees and varnished while still warm. Inside and out. The

fumes that come off 100 degree varnish are incredible! After another coat

tomorrow I'll start winding! I built a winding jig that works great.

Should take about one hour to wind this new coil. Did I forget any

important steps?

Kyle Break@juno.com

Date: Sun, 06 Sep 1998 23:56:30 -0500

From: Bert Hickman <bert.hickman@aquila.com>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Organization: Stoneridge Engineering

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Build the Secondary

Sender: tesl-err@emachine.com

Kyle,

Sounds like you're doing fine! Good luck on winding it!

-- Bert --

Tyla A. Break wrote:

>

>         Hi all. I'm working on a big coil. Mike's instructions seem to be

> the best. But I don't have them anymore. My little 25Mhz computer was

> having trouble handling the 100+ messages from the list this last week!

> So your instructions got deleted by accident. I've tried to apply what I

> remembered. I sanded with #100 then washed with scotch -bright pad. Then

> sanded with #220 , then #400. RE-washed and dried. I heated the pipe to

> approx. 130 degrees and varnished while still warm. Inside and out. The

> fumes that come off 100 degree varnish are incredible! After another coat

> tomorrow I'll start winding! I built a winding jig that works great.

> Should take about one hour to wind this new coil. Did I forget any

> important steps?

>

>                                 Kyle Break@juno.com

>

Comments: Authenticated sender is <megavolt@mail.pcisys.net>

From: megavolt@mail.pcisys.net

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Date: Mon, 7 Sep 1998 07:44:25 +0000

Subject: Re: hot resistors/safety gaps

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

> Anyone else want to offer some info on their coil? Come on

> now. Can't lurk forever.

> >                    #1              #2               #3

> > Transformer =   12kv/100ma      12kv/30ma         15kV/30mA

> > Gap Type =       9@.025          9@.025             10@.03

> > Total gap =      .225           .225                .300

> > Primary cap =    .015           .0056               .0045

> > cap type =      rolled poly      SW bottle         rolled poly

> > Resistors =     310 ohm 50W     330 Ohm  2W     500 Ohm 25W

> > Choke value =   2.25mH            4.1 mh              2.1mh

> > BPS =            120?             ?                       ?

> > Resistor Heat =  COLD              warm             burnt

> > Res Freq  =      254 khz          ~275khz           323khz

> > Max output       54 inches        22 inches         24 inches

> > Lp =              26uH               60uH           54uH

> > Pri Material =    AWG6 solid    AWG12 solid     3/16 tubing

> >

> >                    #4               #5

> > Transformer =   12kv/100ma       15kv/50ma

> > Gap Type =       30@.01           8@.025

> > Total gap =       .300            .200

> > Primary cap =     0.01            .0064

> > cap type =       salvage GE       rolled poly

> > Resistors =       300 ohm 12W     300 ohm 50W

> > Choke value =    4.66 mh           none

> > BPS =             120              120

> > Resistor Heat =  COLD             very warm

> > Res Freq  =      206 khz          399 khz

> > Max output       54 inches        27 inches

> > Lp =              58 uH           46 uh

> > Pri Material =   3/8 tubing       1/4 tubing

> >

#6

Transformer:    15/120

Gap Type:        12@.028

Total Gap:      .336

Primary Cap:    .017uf

Resistors:      750 ohm @ 1500 watts

Choke Value:       ?

BPS:             120?

Resistor Heat:   HOT

Res Freq:        205 khz

Max Output:      57"

Lp:               ?

Pri Material:    5/16" tubing

I did not look up the equations and compute the Lp or choke values as

I don't know very much about electronics and have never had ANY

training or schooling.  The Primary is 20 turns of 5/16 tubing tapped

at 16, 20 degree incline.  The chokes are 1"x3" ferrite with 100' of

24 ga.  I realize that I should be more knowledgeable about what I am

doing and I should get more serious and do the research.  Right now

I'm just having fun and I am learning...slowly.

Kent

megavolt@pcisys.net

Date: Mon, 07 Sep 1998 10:52:41 -0700

From: Gene Waldo <gwaldo@1stnetusa.com>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: So. CaL (LA) Coilers: meet?

Sender: tesl-err@emachine.com

All,

So far, I can account for 4 bodies who may participate. Looks like

Pomona is most central to our geographics.

What it is: Proposed dinner & drinks type thing at a public place (or a

home if such is offered). Swap stories and resources, plans of attack,etc.

Send me your name, e-adr & ZIP.

---------------

Possible (?): the last time Dan & I trooped up to Griffith Park, I was

close to getting Observatory {Big TESLA} Mgmnt to arranging a jaunt into

the lower dungeons, where the power supply lurks. I still want to do that!

There was no real resistance, just a lack of persistence! With a

correctly composed party, and some advance preparation, this should be no sweat!

I understand that there is a loose consortium of local HiTension Coilers

who may (have) periodically tend(ed) the beast, or have larger

experience and/or expertise....?  Any advice? Avenes of approach?

I'd like to photo-document the event and installation, in detail, if we

can make it happen.

The only parts of this historic coil seen by the public are the boxed

rot'y gap (no thrill!); the Pri: flat, ~10 turn? edge wound Cu strip

~1.25" x ~.25"); conic wound 2ndary (looks like insulated wire?*), base

dia:~4' x top dia:~1.7' x ht:~5' (Dim's all guestimates); spherical

terminal electrode ~8" dia, w/2 points; Disch: ~5' to 2 side-walls.

* word is that a spare or defunct 2ndary is stashed in dungeon, along

with similar Cap's?

--------------------------------------------------------------

Gene Waldo -  gwaldo@1stnetusa.com  - "Igor in Residence"

---------------------------------------------------------------

What's good for the planet is more OZONE! See the machinery form up!

NEW PAGE!   http://members.1stnetusa.com/danmurphy/waldo.htm

X-Mailer: IPAD 2.0

Date: Mon, 07 Sep 1998 14:22:09 -0500

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Mike Hammer <mhammer@misslink.net>

Subject: Peoria IL hamfest

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Just to remind everyone in the area that the Peoria

hamfest is slated for September 19 and 20. It is held

at Exposition Gardens on Northmoor Ave just west of

University on the north end of Peoria.

It is a 2 day hamfest with Saturday historically

being the most popular.

If any coilers plan to attend this hamfest let me

know and we can arrange a little get together. Swap

a few stories and tell a few lies.

TTYL!

Mike Hammer

mhammer@misslink.net

From: Ashpilot@aol.com

Date: Mon, 7 Sep 1998 16:30:35 EDT

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Choke sizes, etc...

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

I have gotten some commercially made RF chokes from salvage, two are rated at

2 mH, 16 A  (and 1.0 ohms?) and one seems to be two windings sharing a common

core and housing, it says on the case 2X5 mH, 16 A.   The two single chokes

are quite small, about 2.5 cm in diameter and 1cm thick,  The 'double' is much

larger approx 5cm across and 2 cm thick.  Does anyone think these would be

sufficient to work in the primary to protect my transformer??   Also can

someone provide a description of how these would be wired into the circuit,

and if resistors or caps would work better for RF damping?  Thanks!!

Ed Mullin  Nashua NH.

P.S. any coiliers out here on the east coast?

From:          "Malcolm Watts" <MALCOLM@directorate.wnp.ac.nz>

Organization:  Wellington Polytechnic, NZ

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Date:          Tue, 8 Sep 1998 11:55:24 +1200

Subject:       Re: The MISSING pri coil form??

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Hi Gene,

> Date sent:      Fri, 04 Sep 1998 07:58:15 -0700

> From:           Gene Waldo <gwaldo@1stnetusa.com>

> Send reply to:  "Tesla-2 Mailing List" <tesla-2@emachine.com>

> To:             "Tesla-2 Mailing List" <tesla-2@emachine.com>

> Subject:        The MISSING pri coil form??

> All,

> The discussion of Pri forms seems to consist of 3 basic: solenoid,

> saucer, and flat.  Assuming ~.2-.4" Cu tube, etc.

>

> Ignoring for the moment, their advantages of ease of tapping, how would,

> say, a final, tweaked coil length do if it were insulated and wound into

> a tight 'toroid' or compressed sol. form?

>

> I immagine that this would throw all inductance factors into a cocked

> hat? Would this be compensatable by physical adjustments?

I've done exactly this by building two-layer pancake primaries. Net

result is that you can get 70% more inductance for the same length

of copper. Now that's something for nothing :)

Malcolm

Date: Mon, 7 Sep 1998 18:14:25 -0600

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Daniel Lockhart <molly@cmn.net>

Subject: Re: Performance update 6.5"

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

>Coilers

>

>Well I moved as much stuff out of the way as I could before resuming the

>test. >The first time I fired it, I got 65" arc's. Tonight the streamers

>reached 74" >(Power level 3.5 - 4 Kw). The coil still needs to be fine

>tuned. This coil looks like a real performer. If I move things around a

>little, I think I can reach 85 - 90"  beyond this I will have to move out side.

>The furnace has already taken a few good hits along with the caps and rotary.

>Here are the spec's.

>> snip! <<

>Primary: 3/8 copper tubing with 3/8" spacing between turns. 14 turns.

>

>Inductance: 75 uH (Measured)

>

>More test to follow

>

>Paul M

>

Congrats! Sounds like a real winner! The next step is to use three pole

pigs,  14.4 kV, 20 amps (or so)...  :-)

Where is your primary tapped? Is it at fourteen turns, or is this the

maximum number of turns the primary can be tapped at? In the six inch coil

I designed, I couldn't get any more than six or seven turns to reach the

required inductance (I wanted twelve or so). Then again, I'm not using such

a big toroid either!...

Thanks,

Daniel Lockhart

From:          "Malcolm Watts" <MALCOLM@directorate.wnp.ac.nz>

Organization:  Wellington Polytechnic, NZ

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Date:          Tue, 8 Sep 1998 12:29:12 +1200

Subject:       Re: Safety Gap problems after reconfig

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Hello Michael,

Here is a chance for us all to learn something:

> Send reply to:  "Tesla-2 Mailing List" <tesla-2@emachine.com>

> From:           "Coiler" <mycroft@access1.net>

> To:             "Tesla-2 Mailing List" <tesla-2@emachine.com>

> Subject:        Safety Gap problems after reconfig

> Date sent:      Sun, 6 Sep 1998 15:59:29 -0700

> Ok, now I am stumptified. I built a new enclosure for my caps, a

> way to locate them under my coil (as I was getting strikes that made

> me *very* nervous). Since I no longer needed the flexiblity, I

> replaced all of my 14AWG Neon wire with 12AWG Solid wire. Most of

> the runs less than 1/2 the length of before. My performance has

> now gone into the toilet, and my SG is going nuts. Both the one

> across my neon, and the one across my cap. No manner of tuning seems

> to help.

>

> One other interesting twist: This coil used to have to work real

> hard to get the first streamer, then it cooked. Now it throws

> anywhere from 4-10 streamers at once, tho much shorter.

>

> Someone with experience in this please.. what the heck did I break?

>

> [This coil has given me 37" streamers before], oh.. and on the resistor note..

> them suckers get *REALLY* hot now.. but then the SG is dumping a lot from the cap.

First thing, get your cap out from under the coil and see what

difference if any that makes. If that makes no difference, replace

the wire with wire of the same length as previously and try again

with the cap away from the coil. If that makes a difference, now

place the cap back under the coil with the longer leads and see what

happens.

In a situation like this, it is important to try one thing at a

time to see where the problem lies. Please let us know what happens.

Malcolm

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Griffith Park coil

X-Mailer: IPAD 2.0

From: break7@juno.com (Tyla A. Break)

Date: Mon, 07 Sep 1998 22:19:09 EDT

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

HI all.

Just read Gene's message . I'm quite interested in meeting. Will E-mail you.

About the big coil, I know some people that worked at the park as

guides. I tried to do just what you're thinking. It didn't happen because

I didn't prod them enough. Most people think it's really boring. I

pestered them for info until I got some. Trouble its really tainted by

the people that wrote it. There are three separate accounts. After

looking quite a few times I think I've sorted out the B.S. First of all

it was part of a half-wave unit. So there is another half somewhere. The

secondary form is 47" tall, 36" at the base 18" at the top. One account

says it is wound with 1000 turns of#22, this I believe. Another says 400

of #14! still another says 231 of something else. The first writing in

1948 seems to be the most correct. people are misled

By what looks like 231 turns of rope dipped in Glyptal wrapped around the

secondary winding. Next the primary is 10 turns of copper buss bar. 3/16

x 3/4". The rotary gap is non- synchronous. It spins @ 1800 RPM , has 14

copper bars mounted in a disk. Now the interesting stuff. The cap is huge

, at least by weight. A latter article says that it is a stack of glass,

30 sheets 24" x 30"! X1/2 " thick!! 29 sheets of copper 15 x14".

Everything is connected with #1 wire. Now the reason I want to see the

basement again. The transformer is said to be 2000 watt with current

limiting windings. But after seeing the performance and reading I'm sure

that is not entirely true. They supply a schematic showing a trans. that

is 10,000v 2000 watt with "line filters" on the primary side. These

filters are two coils, 57 turns each of #12 wound on iron bars 3 1/2"

dia. x 12" long. I believe these are the ballasts that limit the current

to the trans.  Heavy iron cores with a lot of reluctance would make good

high current ballasts. I want to see that trans.! Some writings say the

coin runs at 35,000 Hz another says 70,000, this seems more realistic.

Either way it is impressive for being built in 1930.

Now I got a couple of questions, this might be a dumb idea , but if your

primary is wound CCW should the sec. be the same ??

If the magnetic field of the primary extends upwards the distance of the

diameter, it extends downwards too right ? Would it create a loss or

distort the field if any thing metal (conductive) was placed in this

area? Like caps, gap any wiring?

Please let me know what you think.  I've wanted to use screws in

construction , been worried about screwing things up.

Kyle Break7@juno.com

From: "Tobin Fricke" <fricke@mae.engr.ucdavis.edu>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: So. CaL (LA) Coilers: meet?

Date: Mon, 7 Sep 1998 19:22:03 -0700

X-Mailer: IPAD 2.0

X-MimeOLE: Produced By Microsoft MimeOLE V4.72.3110.3

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

>So far, I can account for 4 bodies who may participate. Looks like

>Pomona is most central to our geographics.

This is horrible! I lived in Orange County until last month, and I always

thought all the active tesla coil people were UP HERE in the Bay Area.. Now

I get here and you're organizing a meeting BACK THERE!  <g>

Anyone up in the bay area / Berkeley?

Tobin Fricke

From: "Tobin Fricke" <fricke@mae.engr.ucdavis.edu>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Griffith Park coil

Date: Mon, 7 Sep 1998 19:23:24 -0700

X-Mailer: IPAD 2.0

X-MimeOLE: Produced By Microsoft MimeOLE V4.72.3110.3

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

> About the big coil, I know some people that worked at the park as

>guides. I tried to do just what you're thinking. It didn't happen because

This sounds interesting.. I missed the beginning of the thread, though.

What's this alleged Griffith Park coil?

Tobin Fricke | UC Berkeley

From: "Tobin Fricke" <fricke@mae.engr.ucdavis.edu>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: getting started

Date: Mon, 7 Sep 1998 19:44:51 -0700

X-Mailer: IPAD 2.0

X-MimeOLE: Produced By Microsoft MimeOLE V4.72.3110.3

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

>Mail me back with your answer and some more questions!  Some things you could

>be looking for in the meantime include places to buy 1/4" copper thinwall

>tubing, a NST (neon sign transformer), PVC pipe, and hundreds of feet of 24

>gauge enameled wire.

Hmm. Any recommendations of where to look for such things?  I'm in Berkeley.

There are lots of neon sign vendors in the phone book.. I'm going to try

them for the NST.  Is there any type of transformer I should look for in

particular?  Where should I look for the other stuff?

thanks,

Tobin Fricke | UC Berkeley

From: "Tobin Fricke" <fricke@mae.engr.ucdavis.edu>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: getting started

Date: Mon, 7 Sep 1998 19:46:06 -0700

X-Mailer: IPAD 2.0

X-MimeOLE: Produced By Microsoft MimeOLE V4.72.3110.3

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

>round $20.00. Recently I purchased 4)15 kv 30 ma. transformers. Two of these

>would give you plenty of power.

Oh.. So I can use multiple transformers?  How would I connect them?  What

sort of power should I aim for?

Tobin Fricke

Date: Mon, 7 Sep 1998 20:47:09 -0600

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Daniel Lockhart <molly@cmn.net>

Subject: Re: Griffith Park coil

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

Kyle Break7@juno.com wrote:

>> regarding the Griffith Park Coil <<

What's the history on this? Who made it? Sounds interesting.

> Now I got a couple of questions, this might be a dumb idea , but if your

>primary is wound CCW should the sec. be the same ??

I don't think it matters. Don't even worry about it.

> If the magnetic field of the primary extends upwards the distance of the

>diameter, it extends downwards too right ? Would it create a loss or

>distort the field if any thing metal (conductive) was placed in this

>area? Like caps, gap any wiring?

Good question. This has always bugged me too. It seems like it would make

some difference.

>  Please let me know what you think.  I've wanted to use screws in

>construction , been worried about screwing things up.

The thing with screws is that they always screw things up. Like when I hang

drywall -- there isn't one piece that I don't screw up!  :-)  Actually, I

think it's fine to use screws for most of the construction, except for the

primary, where absolutely no metal should be used -- screws or nails. I

used wooden dowels.

Daniel Lockhart

From:          "Malcolm Watts" <MALCOLM@directorate.wnp.ac.nz>

Organization:  Wellington Polytechnic, NZ

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Date:          Tue, 8 Sep 1998 14:47:54 +1200

Subject:       Re: Griffith Park coil

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Hi Kyle, all,

I've run thru a few quick figures on this coil. If its

resonant frequency is about 70kHz and it has no or only a small

topload, I reckon it has somewhere around 800 - 900 turns (sorta

close to 1000). Self capacitance is around 30pF.

Malcolm

> To:             "Tesla-2 Mailing List" <tesla-2@emachine.com>

> Subject:        Griffith Park coil

> From:           break7@juno.com (Tyla A. Break)

> Date sent:      Mon, 07 Sep 1998 22:19:09 EDT

> Send reply to:  "Tesla-2 Mailing List" <tesla-2@emachine.com>

>     HI all.

>     Just read Gene's message . I'm quite interested in meeting. Will E-mail you.

> About the big coil, I know some people that worked at the park as

> guides. I tried to do just what you're thinking. It didn't happen because

> I didn't prod them enough. Most people think it's really boring. I

> pestered them for info until I got some. Trouble its really tainted by

> the people that wrote it. There are three separate accounts. After

> looking quite a few times I think I've sorted out the B.S. First of all

> it was part of a half-wave unit. So there is another half somewhere. The

> secondary form is 47" tall, 36" at the base 18" at the top. One account

> says it is wound with 1000 turns of#22, this I believe. Another says 400

> of #14! still another says 231 of something else. The first writing in

> 1948 seems to be the most correct. people are miss- led

<snip>

Date: Mon, 07 Sep 1998 22:51:55 -0500

From: Bert Hickman <bert.hickman@aquila.com>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Organization: Stoneridge Engineering

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Griffith Park coil

Sender: tesl-err@emachine.com

Daniel Lockhart wrote:

>

> Kyle Break7@juno.com wrote:

>

> >> regarding the Griffith Park Coil <<

>

> What's the history on this? Who made it? Sounds interesting.

>

> > Now I got a couple of questions, this might be a dumb idea , but if your

> >primary is wound CCW should the sec. be the same ??

>

> I don't think it matters. Don't even worry about it.

>

> > If the magnetic field of the primary extends upwards the distance of the

> >diameter, it extends downwards too right ? Would it create a loss or

> >distort the field if any thing metal (conductive) was placed in this

> >area? Like caps, gap any wiring?

>

> Good question. This has always bugged me too. It seems like it would make

> some difference.

>

> >  Please let me know what you think.  I've wanted to use screws in

> >construction , been worried about screwing things up.

>

> The thing with screws is that they always screw things up. Like when I hang

> drywall -- there isn't one piece that I don't screw up!  :-)  Actually, I

> think it's fine to use screws for most of the construction, except for the

> primary, where absolutely no metal should be used -- screws or nails. I

> used wooden dowels.

>

> Daniel Lockhart

Daniel and all,

Here's some accurate information about the Griffith Park Observatory

Coil from an article originally written by Dick Aurandt (deceased) in

the Tesla Coil Builders Association (TCBA) News, Volume 5, #1.

This coil was originally one-half of a two-coil half-wave system that

was originally built in the 1920's by Mr. Earle L. Ovington, and was

originally built for Vaudeville demonstrations (a demonstration

interestingly enough misnamed "The Edison Man"!). Mr. Ovington later

gave the coil to a friend and associate, Dr. Frederick Finch Strong, who

donated the coil to the Observatory in 1936. When the coil was first

delivered to the Observatory, initial tests were conducted with the aid

of the famous Kenneth Strickfaded (the creator of many early high

voltage special effects for Hollywood, including the Frankenstein series

and other science fiction flicks).

Lack of space at the museum prevented using both coils, so the system

was converted to single-coil quarter-wave operation. The original system

used 12 Leyden jars in parallel, each 0.01 uF, but only needed five jars

after the conversion to a single resonator. These Leyden jars all

succumbed to breakdown, and were replaced with a glass plate stack

having a value of 0.027 uF. This reduced the power level of the coil,

but was necessary since the resonator had sustained significant

insulation failure at the top of the resonator.

The transformer is made by Roentgen Transformers, and is rated at 20,000

volts, 2000 watts. It has four independent primaries, each rated at 55

volts. Today's system has them all connected in series, running off 117

VAC to generate an output voltage of about 11,000 volts. It's oil

immersed, built quite solidly, and can deliver 10 KVA for short periods

of time.

The original coil did not have a top corona terminal, and over the years

the top two inches of the coil experienced turn-turn breakdown. In 1985,

a group of TCBA volunteers rewound the coil, covering it with insulating

rope and red Glyptol, and installed a 3/4" copper tubing corona ring at

the top.  The glass plate capacitor was replaced with a 0.056 uF

oil-immersed glass plate capacitor made by TCBA member Ed Angell. A 220

Volt, 6 KVA variac was also placed in series with the 2 KVA HV

transformer in order to provide a degree of current limiting.

Here are the coil's current specs per the article:

=================================================

Power Input: 3.5 KVA, 117 VAC at 32 Amps momentarily

Frequency:   120 kHz

Primary:     4.5 Turns

Secondary:   292 turns (including 6 inner primary turns in series)

Sparkgap:    14 Tungsten-carbide tips rotated at 1725 RPM (asynchronous)

Sparklength: 47"

Hope this helps!

-- Bert --

From:          "Malcolm Watts" <MALCOLM@directorate.wnp.ac.nz>

Organization:  Wellington Polytechnic, NZ

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Date:          Tue, 8 Sep 1998 16:15:13 +1200

Subject:       Re: Griffith Park coil

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

HI Bert, all,

<snip>

> Daniel and all,

>

> Here's some accurate information about the Griffith Park Observatory

> Coil from an article originally written by Dick Aurandt (deceased) in

> the Tesla Coil Builders Association (TCBA) News, Volume 5, #1.

<snip>

> Here are the coil's current specs per the article:

> =================================================

> Power Input: 3.5 KVA, 117 VAC at 32 Amps momentarily

> Frequency:   120 kHz

> Primary:     4.5 Turns

> Secondary:   292 turns (including 6 inner primary turns in series)

> Sparkgap:    14 Tungsten-carbide tips rotated at 1725 RPM (asynchronous)

> Sparklength: 47"

>

> Hope this helps!

>

> -- Bert --

Well, Fr was a mile off and so was my calc.  I wryly note that this

is not the world's most efficient coil.

Malcolm

Date: Mon, 07 Sep 1998 23:24:54 -0500

From: Bert Hickman <bert.hickman@aquila.com>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Organization: Stoneridge Engineering

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Griffith Park coil

Sender: tesl-err@emachine.com

Malcolm Watts wrote:

>

> HI Bert, all,

>

> <snip>

> > Daniel and all,

> >

> > Here's some accurate information about the Griffith Park Observatory

> > Coil from an article originally written by Dick Aurandt (deceased) in

> > the Tesla Coil Builders Association (TCBA) News, Volume 5, #1.

>

> <snip>

>

> > Here are the coil's current specs per the article:

> > =================================================

> > Power Input: 3.5 KVA, 117 VAC at 32 Amps momentarily

> > Frequency:   120 kHz

> > Primary:     4.5 Turns

> > Secondary:   292 turns (including 6 inner primary turns in series)

> > Sparkgap:    14 Tungsten-carbide tips rotated at 1725 RPM (asynchronous)

> > Sparklength: 47"

> >

> > Hope this helps!

> >

> > -- Bert --

>

> Well, Fr was a mile off and so was my calc.  I wryly note that this

> is not the world's most efficient coil.

>

> Malcolm

Malcolm and all,

You're certainly right about the coil not being a stellar performer

compared to today's high-performance systems! I suspect the performance

problems stem from:

1. The use of a lossy, glass plate tank capacitor

2. conical resonator probably has a "broader" self-resonant peak than a

simple solenoid.. resulting in a lower Q secondary.

3. The lack of a substantial topload results in multiple/premature

breakouts instead of single, longer, streamer.

It still sounds like an interesting system with an interesting history!

-- Bert --

From: ralph.hartwell@emachine.com (Ralph Hartwell)

Subject:  MOT saturation tests

Date: Tue,  8 Sep 1998 08:00:00 GMT

Organization: The Energy Machine Information System 504-733-3462

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

> I finally got around to performing my microwave oven transformer

>saturation tests.  The goal was to characterize the primary winding in

>terms of current - ie - see just how nonlinear this reactance is as a

>function of current.

<snip>

>V=volts, I=amps, Z=complex ohms

> V   I     Z(approx'd as Z=V/I) (Neglecting the 0.32 ohm primary)

> 50  0.26  192.

> 60  0.31  194.

> 70  0.38  184.

> 80  0.50  100.

> 90  0.73  123.

>100  1.16   86.2

>110  2.14   51.4

>120  3.86   31.1

>130  5.90   22.0

>Clearly this core is starting to do "something odd" even for low

>currents near 0.5 amps.  An ideal linear (non-saturating) inductance

>would show no change in Z as the applied voltage was changed.  Perhaps

>this is the action of the shunts?

Hmm..  Sounds suspiciously like a "swinging choke" to me.  With

the air gap between the shunts and the core, the inductance would be

max at low current, and decrease as the core / shunts start to saturate.

Add more iron!!  Let's see - wasn't the old ham radio rule, "1 Lb. of

iron per 100 watts, and 1 Lb. of iron for core losses?"

Ralph  W5JGV

---

From: ralph.hartwell@emachine.com (Ralph Hartwell)

Subject:  Report

Date: Tue,  8 Sep 1998 08:24:00 GMT

Organization: The Energy Machine Information System 504-733-3462

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

Hi everyone!

I'm sending this email while I'm at the International Tesla Society

convention in Denver, Colorado.  Actually, the convention ended this

afternoon.  Lotsa' neat stuff, but this year, not too much Tesla

related.

One item which was displayed, was a neat looking, well constructed

Tesla coil which was demonstrated to the gathering one evening.

Unfortunately, the coil, although looking pretty snazzy, didn't work very

well.  See how many construction errors you can find:

The machine had roughly the following data as far as I could

determine by eye:

Primary, flat pancake coil, about 6 turns, ID about 8", spaced about

1" between turns, made from 3/8" copper tube, plexi formers and insulation.

Secondary, about 6" diameter by about 20" high, wire and turns count

unknown.  Insulated with red Glyptal paint.  Metal anti-corona plate on

top (it needed it) and a 12" diameter by 4" commercial aluminum toroid

on top.

Power xfmr, 12 kv 120 ma NST.  No RF chokes or Safety Gap present.

In operation, the *best* the unit could do was *maybe* 6 inch sparks

to a grounded terminal!!!  Little corona from the toroid, but a steady

"blue fog" was present around the top of the secondary extending out to

about 8" in all directions.  No secondary earth ground was used during

the demonstration.   As the operator added additional discharge rods to

the top of the toroid, the air discharges increased.

Upon firing, it was immediately apparent that Something Was Wrong!

The primary spark gap sizzled, and was not very loud.  Closer inspection

revealed that it was a 4 section static gap, not fan cooled, and made

from Aluminum rod. Well burnt, at that.

Capacitors were standard HV transmitter caps - I didn't note the

capacity.  All primary wiring was made with 3/8 copper tube or 3/4" wide

strap.

The secondary was probably overcoupled, as the bottom turn of the

secondary appeared to be about 1/2" below the primary center line.

I resisted the urge to break out my pocket toolkit and start

readjusting things right there! <G>

Ralph

ps - Hey Mike!  Wanna' bring coil to the convention next year and show

'em how it's *really* done??!??!

zzzzzz

From: Hollmike@aol.com

Date: Tue, 8 Sep 1998 07:13:06 EDT

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Report

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

In a message dated 9/8/98 2:07:39 AM Mountain Daylight Time,

ralph.hartwell@emachine.com writes:

> ps - Hey Mike!  Wanna' bring coil to the convention next year and show

>  'em how it's *really* done??!??!

Which Mike are you talking to?  This one would be happy to join the fun.

Mike Hollingsworth.

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Date: Tue, 08 Sep 1998 04:27:04 -0700

From: "paul m" <fleischwulf@mailexcite.com>

Cc:

X-Sent-Mail: off

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

Subject: Re: Preformance update 6.5"

X-Sender-Ip: 208.153.7.9

Organization: MailExcite  (http://www.mailexcite.com:80)

Sender: tesl-err@emachine.com

Daniel,

It looks like a lot of my message failed to transmit. The primary is tapped out to the max. All 14 turns for the heavy top load. The tank circuit is made up of four plate capacitors for a total of .026 uF. The gap is rotaty and the inductance

of the secondary is 35 mH. With my current setup I can deliver 18 Kw to the coil. However I start to over couple at about 7.5 Kw. At 18 Kw the coil wouldn't last too long. I'm going to have to put some grounded screens up in the basement to con

tain this monster.

I will continue to update

paul & bill

---

Einigheit und Wahrheit

On Mon, 7 Sep 1998 18:14:25    Daniel Lockhart wrote:

>>Coilers

>>

>>Well I moved as much stuff out of the way as I could before resuming the

>>test. >The first time I fired it, I got 65" arc's. Tonight the streamers

>>reached 74" >(Power level 3.5 - 4 Kw). The coil still needs to be fine

>>tuned. This coil looks >like a

>>real performer. If I move things around a little, I think I can reach 85 -

>>90" >beyond this I will have to move out side. The furnace has already

>>taken a few >good hits along with the caps and rotary. Here are the spec's.

>

>	>> snip! <<

>

>>Primary: 3/8 copper tubing with 3/8" spacing between turns. 14 turns.

>>

>>Inductance: 75 uH (Measured)

>>

>>More test to follow

>>

>>Paul M

>>

>>---

>>Einigheit und Wahrheit

>

>

>Congrats! Sounds like a real winner! The next step it to use three pole

>pigs,  14.4 kV, 20 amps (or so)...  :-)

>

>Were is your primary tapped? Is it at fourteen turns, or is this the

>maximum number of turns the primary can be tapped at? In the six inch coil

>I designed, I couldn't get any more than six or seven turns to reach the

>required inductance (I wanted twelve or so). Then again, I'm not using such

>a big toroid either!...

>

>Thanks,

>

>Daniel Lockhart

>

>

>

>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: "Coiler" <mycroft@access1.net>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: RE: Report

Date: Tue, 8 Sep 1998 07:31:53 -0700

X-Mailer: IPAD 2.0

X-MimeOLE: Produced By Microsoft MimeOLE V4.72.3155.0

Importance: Normal

Sender: tesl-err@emachine.com

And this one.. but that is beside the point (cripes, my 12KV30ma coil

does better than that firing counterpoise)

Michael Baumann

Coiler, Homebrewer, Nerd. mycroft@access1.net

> -----Original Message-----

> From: tesl-err@emachine.com [mailto:tesl-err@emachine.com]On Behalf Of

> Hollmike@aol.com

> Sent: Tuesday, September 08, 1998 4:13 AM

> To: Tesla-2 Mailing List

> Subject: Re: Report

>

>

> In a message dated 9/8/98 2:07:39 AM Mountain Daylight Time,

> ralph.hartwell@emachine.com writes:

>

> > ps - Hey Mike!  Wanna' bring coil to the convention next year and show

> >  'em how it's *really* done??!??!

> Which Mike are you talking to?  This one would be happy to join the fun.

> Mike Hollingsworth.

>

Date: Tue, 08 Sep 1998 08:57:44 -0700

From: Gene Waldo <gwaldo@1stnetusa.com>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Griffith Park coil

Sender: tesl-err@emachine.com

Daniel Lockhart wrote:

>

> Kyle Break7@juno.com wrote:

>

> >> regarding the Griffith Park Coil <<

>

> What's the history on this? Who made it? Sounds interesting.

My data on GP Tesla is buried here, some where, but I recall it was

used as a carnival display(?) long ago, and was donated to GP in the

30's.

> I don't think it matters. Don't even worry about it.

>

> > If the magnetic field of the primary extends  Would it create a loss or

> >distort the field if any thing metal (conductive) was placed in this

> >area? Like caps, gap any wiring?

>

been worried about screwing things up.

>

Many coils I have seen use a close coupled construction: caps, rot gaps

etc almost immediately under the primary. Any ungrounded metal object is

usually always charged to some degree in prox to 2ndary, I believe.

I have .jpg's of one such; will post when I dig'em up.

Don't know the effects of a metal plate under pri.

--------------------------------------------

Gene Waldo -  gwaldo@1stnetusa.com  - "Igor in Residence"

---------------------------------------------------------------

What's good for the planet is more OZONE! See the machinery form up!

NEW PAGE!   http://members.1stnetusa.com/danmurphy/waldo.htm

Date: Tue, 08 Sep 1998 09:28:51 -0700

From: Gene Waldo <gwaldo@1stnetusa.com>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: The MISSING pri coil form??

Sender: tesl-err@emachine.com

Malcolm Watts wrote:

>

> Hi Gene,

>

> > To:             "Tesla-2 Mailing List" <tesla-2@emachine.com>

> > Subject:        The MISSING pri coil form??

>

> > All,

> > The discussion of Pri forms seems to consist of 3 basic: solenoid,

> > saucer, and flat.

> > Ignoring for the moment, their advantages of ease of tapping, how would,

> > say, a final, tweaked coil length do if it were insulated and wound into

> > a tight 'toroid' or compressed sol. form?

> I've done exactly this by building two-layer pancake primaries. Nett

> result is that you can get 70% more inductance for the same length

> of copper. Now that's something for nothing :)

>

> Malcolm

Malcom,

Did you 'optimize' the pri "open", and then wind it tight? Or are other

tweaks involved?

"...2 pancakes..." Insulated and tight? open wind? Series or parallel wind?

How would a single wind do?  Tube, stranded, solid?

Got pics? Is this concept well developed? Sounds like there should be

some serious advantages??  size,  dims re: 2ndary?  Why not in our

folklore?

Going just a little nuts, now: Tygon tube Pri: NaCl sol'n conductor,

w/proper internal sealed electrodes; ???????  Electrolysis may be a

factor...

I'm no EE, so due chastisement accepted.

----------------------------------------

Gene Waldo -  gwaldo@1stnetusa.com  - "Igor in Residence"

---------------------------------------------------------------

What's good for the planet is more OZONE! See the machinery form up!

NEW PAGE!   http://members.1stnetusa.com/danmurphy/waldo.htm

Date: Tue, 08 Sep 1998 09:49:35 -0700

From: Gene Waldo <gwaldo@1stnetusa.com>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Griffith Park coil

Sender: tesl-err@emachine.com

Malcolm Watts wrote:

>

> HI Bert, all,

>

> <snip>

> > Daniel and all,

> >

> > Here's some accurate information about the Griffith Park Observatory

> > Coil from an article originally written by Dick Aurandt (deceased) in

> > the Tesla Coil Builders Association (TCBA) News, Volume 5, #1.

>

I wryly note that this

> is not the world's most efficient coil.

>

> Malcolm

Too bad--  someone should get it back to half~ performance! Given some

of the gorpy, usually broken-down exhibits, IMHO, expanding the space

for such might be neat! It is still fun to watch!

As usual, Bert hits us with the good poop!

Bill Wysock seems to have latest hands-on with this coil---  where's

Bill?

Here's Waldo:

------------------------------------------

Gene Waldo -  gwaldo@1stnetusa.com  - "Igor in Residence"

---------------------------------------------------------------

What's good for the planet is more OZONE! See the machinery form up!

NEW PAGE!   http://members.1stnetusa.com/danmurphy/waldo.htm

Date: Tue, 08 Sep 1998 10:14:22 -0700

From: Gene Waldo <gwaldo@1stnetusa.com>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Griffith Park coil

Sender: tesl-err@emachine.com

Bert Hickman wrote:

>

> Malcolm Watts wrote:

> >

> > HI Bert, all,

> >

> > <snip>

> > > Daniel and all,

> > >

> > > Here's some accurate information about the Griffith Park Observatory

> > > Coil from an article originally written by Dick Aurandt (deceased) in

> > > the Tesla Coil Builders Association (TCBA) News, Volume 5, #1.

> >

> > <snip>

I suspect the perormance

> problems stem from:

> 3. The lack of a substantial topload results in multiple/premature

> breakouts instead of single, longer, streamer.

>

> It still sounds like an interesting system with an interesting history!

>

> -- Bert --

Bert,

Top load is ~9"sphere w/diametral points; breakout is off the points, @

180 deg. to 2 walls, w/no stray streamers as I recall. I also recall 2

Gnd pl on walls? It seems there is no data on topL.?

I noted that standing in front of the display, you could pull ~.03"

sparks off just about anything you touched!

--------------------------------------------

Gene Waldo -  gwaldo@1stnetusa.com  - "Igor in Residence"

---------------------------------------------------------------

What's good for the planet is more OZONE! See the machinery form up!

NEW PAGE!   http://members.1stnetusa.com/danmurphy/waldo.htm

Date: Tue, 08 Sep 1998 10:34:30 -0700

From: Gene Waldo <gwaldo@1stnetusa.com>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Griffith Park coil

Sender: tesl-err@emachine.com

Tyla A. Break wrote:

>

>         HI all.

>         Just read Gene's message . I'm quite interested in meeting. Will

> E-mail you.

>         About the big coil, I know some people that worked at the park as

> guides. I tried to do just what you're thinking. It didn't happen because

> I didn't prod them enough.

This was our problem, too.  I still have the impression that effective

prodding is all that is req'd.

I've wanted to use screws in

> construction , been worried about screwing things up.

>

>                 Kyle Break7@juno.com

An alternative to glued dowell: most hdwr stores sell nylon screws;

judicious drill & tap in wood constr might work, too.

--------------------------------------------

Gene Waldo -  gwaldo@1stnetusa.com  - "Igor in Residence"

---------------------------------------------------------------

What's good for the planet is more OZONE! See the machinery form up!

NEW PAGE!   http://members.1stnetusa.com/danmurphy/waldo.htm

From: ralph.hartwell@emachine.com (Ralph Hartwell)

Subject:  Re: Report

Date: Tue,  8 Sep 1998 20:28:00 GMT

Organization: The Energy Machine Information System 504-733-3462

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

H>> ps - Hey Mike!  Wanna' bring coil to the convention next year and show

H>>  'em how it's *really* done??!??!

H>Which Mike are you talking to?  This one would be happy to join the fun.

H>Mike Hollingsworth.

All of them!  <G>   There's plenty of room for a big bunch of coils to

be displayed.  The small ones are often set up in the convention area,

and the big boys are operated in the parking lot outside the hotel.  I'd

love to see a dozen machines displayed at the convention.

Next year, the convention is planned to be back in Colorado Springs,

where it really ought to' be.  Ya' gotta' feel Tesla is there, in spirit.

I'll tell you one thing, the lightning storms there are sure worth

watching - from a safe distance, of course!

I'll try to post some info about next years convention as the plans

are finalized.  We'd love to have everyone attend.

Ralph

---

Birthdays are significant.  Without them you're dead.

From:          "Malcolm Watts" <MALCOLM@directorate.wnp.ac.nz>

Organization:  Wellington Polytechnic, NZ

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Date:          Wed, 9 Sep 1998 09:10:50 +1200

Subject:       Re: Report

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Hi Ralph,

There is a glaring defficiency in that system:

> From:           ralph.hartwell@emachine.com (Ralph Hartwell)

> Subject:        Report

> Date sent:      Tue,  8 Sep 1998 08:24:00 GMT

> Organization:   The Energy Machine Information System 504-733-3462

> To:             "Tesla-2 Mailing List" <tesla-2@emachine.com>

> Send reply to:  "Tesla-2 Mailing List" <tesla-2@emachine.com>

>

> Hi everyone!

>

>    I'm sending this email while I'm at the International Tesla Society

> convention in Denver, Colorado.  Actually, the convention ended this

> afternoon.  Lotsa' neat stuff, but this year, not too much Tesla

> related.

>

>    One item which was displayed, was a neat looking, well constructed

> Tesla coil which was demonstrated to the gathering one evening.

> Unfortunately, the coil, although looking pretty snazzy, didn't work very

> well.  See how many construction errors you can find:

>

>   The machine had roughly the following data as far as I could

> determine by eye:

>

>   Primary, flat pancake coil, about 6 turns, ID about 8", spaced about

> 1" between turns, made from 3/8" copper tube, plexi formers and

> insulation.

>

>   Secondary, about 6" diameter by about 20" high, wire and turns count

> unknown.  Insulated with red Glyptal paint.  Metal anti-corona plate on

> top (it needed it) and a 12" diameter by 4" commercial aluminum torroid

> on top.

>

>    Power xfmr, 12 kv 120 ma NST.  No RF chokes or Safety Gap present.

>

>    In operation, the *best* the unit could do was *maybe* 6 inch sparks

> to a grounded terminal!!!  Little corona from the torroid, but a steady

> "blue fog" was present around the top of the secondary extending out to

> about 8" in all directions.  No secondary earth ground was used during

> the demonstration.   As the operator added additional discharge rods to

> the top of the torroid, the air discharges increased.

It is contained in the above paragraph. *No ground for the secondary

coil*!!!!!!!!!!!!!!!!!!!!!!!!!!!!!  If it was tuned to resonance with

a ground previously it would have been operating a mile out of tune

and the gaps would have been eaten alive.

Malcolm

From:          "Malcolm Watts" <MALCOLM@directorate.wnp.ac.nz>

Organization:  Wellington Polytechnic, NZ

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Date:          Wed, 9 Sep 1998 09:21:01 +1200

Subject:       Re: The MISSING pri coil form??

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Hi Gene,

> Date sent:      Tue, 08 Sep 1998 09:28:51 -0700

> From:           Gene Waldo <gwaldo@1stnetusa.com>

> Send reply to:  "Tesla-2 Mailing List" <tesla-2@emachine.com>

> To:             "Tesla-2 Mailing List" <tesla-2@emachine.com>

> Subject:        Re: The MISSING pri coil form??

> Malcolm Watts wrote:

> >

> > Hi Gene,

> >

> > > To:             "Tesla-2 Mailing List" <tesla-2@emachine.com>

> > > Subject:        The MISSING pri coil form??

> >

> > > All,

> > > The discussion of Pri forms seems to consist of 3 basic: solenoid,

> > > saucer, and flat.

> > > Ignoring for the moment, their advantages of ease of tapping, how would,

> > > say, a final, tweaked coil length do if it were insulated and wound into

> > > a tight 'toroid' or compressed sol. form?

>

> > I've done exactly this by building two-layer pancake primaries. Nett

> > result is that you can get 70% more inductance for the same length

> > of copper. Now that's something for nothing :)

> >

> > Malcolm

>

> Malcom,

> Did you 'optimize' the pri "open", and then wind it tight? Or are other

> tweaks involved?

No. I simply built it in two layers. The design was made all the more

easy by the fact that the annealed tubing I bought came in a pack all

coiled up as two layers. To use this as is, simply get a piece of

acrylic/lexan etc., cut a hole in it to the inside diameter of the

coil you want to end up with and simply "thread" the coil onto it,

finally spacing the turns from the centre out and hol;ding in place

with dabs of hot melt. Tapping is done by either brazing/soldering

tabs on prior to threading onto the insulator or simply drilling a

hole in the pipe and holding a terminal/tab in with a self-tapping

screw. The outside end of the bottom is the cold side and turns are

tapped in as required. There is no reason why you could not use two

of these assemblies with another sheet of plastic between them but

watch the clearances between the assemblies or flashovers could be a

problem.

> "...2 pancakes..." Insulated and tight? open wind? Series or paralell

> wind?

Series of course. The coiled up pipe starts at the outside, works in

to the centre, then works toward the outside again on the next layer.

> How would a single wind do?  Tube, stranded, solid?

Single wind? It was one pice of pipe.

> Got pics? Is this concept well developed? Sounds like there should be

> some serious advantages??  size,  dims re: 2ndary?  Why not in our

> folklore?

Because few have done it. However, stacked pancakes can be seen in

pictures of early spark gap radio transmitter setups. Advantages

include much higher inductance/unit length of copper due to mutual

coupling between turns and smaller overall size which allows higher

values of k for the same inductance.

> Going just a little nuts, now: Tygon tube Pri: NaCl sol'n conductor,

> w/proper internal sealed electrodes; ???????  Electrolysis may be a

> factor...

> I'm no EE, so due chastisement accepted.

None sent.

Malcolm

Date: Tue, 08 Sep 1998 15:20:04 -0700

From: Gene Waldo <gwaldo@1stnetusa.com>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: The MISSING pri coil form??

Sender: tesl-err@emachine.com

Malcolm Watts wrote:

>

> Hi Gene,

>

>

> > Malcolm Watts wrote:

> > >

> > > Hi Gene,

> > >

> > > > To:             "Tesla-2 Mailing List" <tesla-2@emachine.com>

> > > > Subject:        The MISSING pri coil form??

> > >

> > > > All,

> > > > The discussion of Pri forms seems to consist of 3 basic: solenoid,

> > > > saucer, and flat.

> > > > Ignoring for the moment, their advantages of ease of tapping, how would,

> > > > say, a final, tweaked coil length do if it were insulated and wound into

> > > > a tight 'toroid' or compressed sol. form?

> >

> > > I've done exactly this by building two-layer pancake primaries. Nett

> > > result is that you can get 70% more inductance for the same length

> > > of copper. Now that's something for nothing :)

> > >

> > > Malcolm

> > Going just a little nuts, now: Tygon tube Pri: NaCl sol'n conductor,

> > w/proper internal sealed electrodes; ???????  Electrolysis may be a

> > factor...

> > I'm no EE, so due chastisement accepted.

>

> None sent.

>

> Malcolm

In my youth, we used to circulate distilled H2O in non conducting tube to

cool amplifier tubes. It occured to me then that a conducting fluid

might perform as a conductor???  Well... How wrong can I be?

The fluid primary notion is either mostly untried or proven dumb?!

Thermal flux v. circulation???  Circulation would obviously make this a

dumb idea!

What happens to such a fluid conductor at pri loads?  I'm not in a

position to test, at the moment, but we could all speculate rampantly!

A toroidal tygon primary might be a much simpler construction than all

the wierd pri forms we practice now!

Then there's the potential mess of a 'blown primary'! probably akin to

blown finals--<[:-(  kinda like I got when rotting stays let my ant

down!

-------------------------------------------------------

Gene Waldo -  gwaldo@1stnetusa.com  - "Igor in Residence"

---------------------------------------------------------------

What's good for the planet is more OZONE! See the machinery form up!

NEW PAGE!   http://members.1stnetusa.com/danmurphy/waldo.htm

Date: Tue, 08 Sep 1998 20:13:18 -0500

From: Bert Hickman <bert.hickman@aquila.com>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Organization: Stoneridge Engineering

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Griffith Park coil

Sender: tesl-err@emachine.com

Gene Waldo wrote:

>

> Bert Hickman wrote:

> >

> > Malcolm Watts wrote:

> > >

> > > HI Bert, all,

> > >

> > > <snip>

> > > > Daniel and all,

> > > >

> > > > Here's some accurate information about the Griffith Park Observatory

> > > > Coil from an article originally written by Dick Aurandt (deceased) in

> > > > the Tesla Coil Builders Association (TCBA) News, Volume 5, #1.

> > >

> > > <snip>

>  I suspect the perormance

> > problems stem from:

>

> > 3. The lack of a substantial topload results in multiple/premature

> > breakouts instead of single, longer, streamer.

> >

> > It still sounds like an interesting system with an interesting history!

> >

> > -- Bert --

>

> Bert,

> Top load is ~9"sphere w/diametral points; breakout is off the points, @

> 180 deg. to 2 walls, w/no stray streamers as I recall. I also recall 2

> Gnd pl on walls? It seems there is no data on topL.?

>

> I noted that standing in front of the display, you could pull ~.03"

> sparks off just about anything you touched!

Gene and all,

Thanks for the update! Turns out I didn't turn the page in the article I

referenced - there were a couple of pictures that I missed right after

the end of the article! Although the text doesn't mention them, there

were indeed spherical terminals as you described on the original coils

and on the present coil, which look to be 9" in diameter with what look

to be 1/2" - 3/4" diameter horizontal rods coming out of the sphere at

180 degrees.

Hopefully, this system is continuing to get the TLC it deserves from

knowledgable coiler(s) in the area! :^)

-- Bert --

X-Originating-IP: [208.16.145.177]

From: "Bill the arcstarter" <arcstarter@hotmail.com>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: MOT saturation tests II

Date: Tue, 08 Sep 1998 20:05:59 PDT

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

Coilers,

While the rest of the world was watching baseball on TV, I was out in

the garage chiseling the shunts out of my MOT.  I'm now left with the

main laminations and the primary winding.  The shunts and the secondary

windings are long gone, having fallen to the chisel!

I then remeasured the current as a function of voltage.  Here are the

new results:

(I added the two rightmost columns)

V=volts, I=amps, Z=complex ohms

V   Iold  Zold    Inew  Znew

50  0.26  192.    0.27  185.

60  0.31  194.    0.31  193.

70  0.38  184.    0.38  184.

80  0.50  100.    0.50  160.

90  0.73  123.    0.74  121.

100  1.16   86.2   1.20   83.3

110  2.14   51.4   2.35   46.8

120  3.86   31.1   4.41   27.2

130  5.90   22.0   6.80   19.1

I'm surprised that removing the shunts really didn't do anything to the

behavior of the transformer!  The main difference is a slightly

increased current draw at the higher voltages.

At this point I can only guess that the purpose of the shunts was to

limit the current flowing into the secondary (just like a NST or

welder).  It seems that, as far as using MOTs as current limiting

devices for pigs, the shunts really have little or no effect.

Comments invited!

-Bill

From:          "Malcolm Watts" <MALCOLM@directorate.wnp.ac.nz>

Organization:  Wellington Polytechnic, NZ

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Date:          Wed, 9 Sep 1998 15:57:38 +1200

Subject:       Re: MOT saturation tests II

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Hi Bill,

> From:           "Bill the arcstarter" <arcstarter@hotmail.com>

> To:             "Tesla-2 Mailing List" <tesla-2@emachine.com>

> Subject:        MOT saturation tests II

> Date sent:      Tue, 08 Sep 1998 20:05:59 PDT

> Send reply to:  "Tesla-2 Mailing List" <tesla-2@emachine.com>

> Coilers,

>  While the rest of the world was watching baseball on TV, I was out in

> the garage chiseling the shunts out of my MOT.  I'm now left with the

> main laminations and the primary winding.  The shunts and the secondary

> windings are long gone, having fallen to the chisel!

>

>  I then remeasured the current as a function of voltage.  Here are the

> new results:

>

> (I added the two rightmost columns)

>

>  V=volts, I=amps, Z=complex ohms

>   V   Iold  Zold    Inew  Znew

>   50  0.26  192.    0.27  185.

>   60  0.31  194.    0.31  193.

>   70  0.38  184.    0.38  184.

>   80  0.50  100.    0.50  160.

>   90  0.73  123.    0.74  121.

>  100  1.16   86.2   1.20   83.3

>  110  2.14   51.4   2.35   46.8

>  120  3.86   31.1   4.41   27.2

>  130  5.90   22.0   6.80   19.1

>

> I'm surprised that removing the shunts really didn't do anything to the

> behavior of the transformer!  The main difference is a slightly

> increased current draw at the higher voltages.

>

> At this point I can only guess that the purpose of the shunts was to

> limit the current flowing into the secondary (just like a NST or

> welder).  It seems that, as far as using MOTs as current limiting

> devices for pigs, the shunts really have little or no effect.

Perzaktly.  I either posted this list or it might have been pupman

with a note on what shunts do and how cores behave. The reason

removing the shunts did little is the airgap that exists in series

with the shunts. They shouldn't be carrying much magnetizing force

but come mostly into play when the secondary is loaded.

Malcolm

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: So. Cal. Coilers

X-Mailer: IPAD 2.0

From: break7@juno.com (Tyla A. Break)

Date: Wed, 09 Sep 1998 00:13:27 EDT

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Gene, I tried to E-mail you. I'm not sure if you got it. My HAL 9000 said

it it had a problem with my request , but with persistence it sent

something somewhere. Don't know if the electrons with your name on them

went to you.  Please let me know!

Most common things heard around my place:

My wife- "Does this really make you happy?"  -Oh Yeah!

Others-" What are you going to do with that thing ?"

Kyle

Break7@juno.com

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: MOTs

X-Mailer: IPAD 2.0

From: break7@juno.com (Tyla A. Break)

Date: Wed, 09 Sep 1998 00:43:55 EDT

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

I'm starting to get interested in this subject.

What are the shunts?  The loose steels between the two windings?

Or are they short windings  like a shaded pole motor?  I'm looking

at my MOT right now . It's got a welded core  with three windings on it.

The lower is the primary. Above Seperated by several "loose" steels

(short stips of lamenations) are the filament and HV windings. Filament

winding looks like 3 turns.   I haven't used mine for anything yet , It

is still hooked to the magnetron tube and cap.

Kyle Break7@juno.com

X-Originating-IP: [199.217.140.22]

From: "Bill the arcstarter" <arcstarter@hotmail.com>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: MOTs

Date: Wed, 09 Sep 1998 07:11:54 PDT

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

Kyle wrote:

>	I'm starting to get interested in this subject.

>	What are the shunts?  The loose steels between the two windings?

>     Or are they short windings  like a shaded pole motor?  I'm looking

>     at my MOT right now . It's got a welded core with three windings on it.

>     The lower is the primary. Above Seperated by several "loose" steels

>     (short stips of lamenations) are the filament and HV windings. Filament

The shunts are the short "loose" steel inserts between the primary and

the secondary.  As far as I can tell they limit the current which can be

drawn by the secondary, much as the shunts in a NST do.

Hmm- I wonder how much current you could get out of a MOT (briefly) if

you were to remove the shunts...!

Since my experiments indicate that a MOT w/ the secondary open (or

removed) will limit the current drawn at 120 v to about 5 amps or so,

you could probably use one in series with a pig (off of 120 v). That

would guarantee that the current drawn would be less than 5 amps.

..it just seems like we *should* be able to come up with some great use

for those pesky MOTs.  So far that use has eluded me!  Perhaps we can

make a food-heating device out one one, eh? :)

-Bill

X-Originating-IP: [199.217.140.22]

From: "Bill the arcstarter" <arcstarter@hotmail.com>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Pig w/ only one HV bushing?

Date: Wed, 09 Sep 1998 07:16:35 PDT

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

Coilers,

I have a lead on a nice sounding 10KVA pig.  18KV, 125 lbs, $200!  Not

perfect for coiling but I think it is useable and is definitely a great

deal.

It contains three secondary terminals for 120-0-120 connection, but only

one HV bushing.  The yard doesn't know how to wire the thing.

I assume the other connection is the case of the transformer.  But -

wouldn't we want to ground that as well?  If this is the case then I

can't get a truely floating output out of this unit.

Any comments?

Any more when I look up into the sky (to gaze at pole transformers), I

see that most only have one HV bushing.  But it is hard to tell how the

circuit is completed.

Thanks.

-Bill

Date: Wed, 09 Sep 1998 10:10:35 -0500

From: Eric Davidson <edavidson@icva.gov>

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Pig w/ only one HV bushing?

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Bill,

The other end of the primary (HV) is internally connected to the tank. A

truely floating HV side is impossible to acheive, and should not be

attempted.  The single primary bushing configuration is used almost

exclusively because most all distribution circuits are 4 wire Y.  There

will be a terminal about 8 inches up from the bottom near the back of

the tank for a ground connection.  The center tap of the secondary (LV)

should be connected to the tank and to ground also.  Always make sure

all ground connections are made before energizing the transformer. Check

the voltage of the primary.  On single bushing units the voltage between

the HV bushing and the tank will be the Y connected voltage divided by

sqrt3.

Bill the arcstarter wrote:

>

> Coilers,

>  I have a lead on a nice sounding 10KVA pig.  18KV, 125 lbs, $200!  Not

> perfect for coiling but I think it is useable and is definitely a great

> deal.

>

> It contains three secondary terminals for 120-0-120 connection, but only

> one HV bushing.  The yard doesn't know how to wire the thing.

>

>  I assume the other connection is the case of the transformer.  But -

> wouldn't we want to ground that as well?  If this is the case then I

> can't get a truely floating output out of this unit.

>

> Any comments?

>

> Any more when I look up into the sky (to gaze at pole transformers), I

> see that most only have one HV bushing.  But it is hard to tell how the

> circuit is completed.

>

> Thanks.

> -Bill

>

--

Have a good day!

Eric Davidson

edavidson@icva.gov

Date: Wed, 09 Sep 1998 09:46:04 -0700

From: Gene Waldo <gwaldo@1stnetusa.com>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: So. Cal. Coilers

Sender: tesl-err@emachine.com

Tyla A. Break wrote:

>

> Gene, I tried to E-mail you. I'm not sure if you got it.

Kyle,

Yes, I got the Hampton data.  Our list of those who say they would like

to do a meet has now surged to 5 bodies. Chief among the missing, so

far, is perhaps Bill Wysock, who may have had hands-on at Griffith Park.

Bill could also probably assist with setting up a small tour if we

decided this would be neat.

All,

I also wonder if they have any old pics of the system operating in the

original 2-coil (1/2 wave) mode? Perhaps, based upon the seemingly

confused data available at the observatory, and the spiels delivered by

the guides, a more comprehensive data set could be constructed for their

use. At least, more historical credit to Nickloa should be

available/included.   Perhaps even some links to 'coiling', should a

stray young intellect happen to view the display...

And, get that damn box off the rotary Gap! What kind of service does

this component require?

I first saw the present display in about 1947-  I think the only change

has been to lock-out the patron's button; it is now controlled by the

guide, with a key.

Immense loss of power to a 12 year-old!

I also notice that they have amateur astronomer's fests in the forecourt

on occasion; why not the odd TESLA fest? It's lawn, so something passing

for RFGnds shouldn't be too hard to manage.

I suppose that nowadays, all this tech-stuff would be deemed too

dangerous and weird, and we would, no doubt, encounter resistance from

academe and the others now in charge of our lives...  Thanks be, that

they were not around during the Nation's formative years! Can we

immagine OSHA functionaries, et al, pestering Tesla? Nobel? Curie?

Roentgen? Licenses, permits, inspections, waivers..... convnincing

elected (hired) councils and other idiots that they are irrelevent

impediments to progress?

(SORRY!)

---------------------------------------------------

Gene Waldo -  gwaldo@1stnetusa.com  - "Igor in Residence"

---------------------------------------------------------------

What's good for the planet is more OZONE! See the machinery form up!

NEW PAGE!   http://members.1stnetusa.com/danmurphy/waldo.htm

Date: Wed, 09 Sep 1998 09:54:48 -0700

From: Gene Waldo <gwaldo@1stnetusa.com>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: MOTs

Sender: tesl-err@emachine.com

Bill the arcstarter wrote:

>

> Kyle wrote:

> >       I'm starting to get interested in this subject.

> >       What are the shunts?  The loose steels between the two windings?

> >     Or are they short windings  like a shaded pole motor?  I'm looking

> >at my MOT right now .

> ..it just seems like we *should* be able to come up with some great use

> for those pesky MOTs.  So far that use has eluded me!  Perhaps we can

> make a food-heating device out of one, eh? :)

>

> -Bill

>

Bill,

Great idea! Something like a... oh, say, Microwave Oven? Wet cat

drier?   ~~<[:-)

I'll help you share the royalties!

--

Gene Waldo -  gwaldo@1stnetusa.com  - "Igor in Residence"

---------------------------------------------------------------

What's good for the planet is more OZONE! See the machinery form up!

NEW PAGE!   http://members.1stnetusa.com/danmurphy/waldo.htm

From:          "Malcolm Watts" <MALCOLM@directorate.wnp.ac.nz>

Organization:  Wellington Polytechnic, NZ

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Date:          Thu, 10 Sep 1998 08:28:52 +1200

Subject:       Re: MOTs

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Hi Bill, all,

> From:           "Bill the arcstarter" <arcstarter@hotmail.com>

> To:             "Tesla-2 Mailing List" <tesla-2@emachine.com>

> Subject:        Re: MOTs

> Date sent:      Wed, 09 Sep 1998 07:11:54 PDT

> Send reply to:  "Tesla-2 Mailing List" <tesla-2@emachine.com>

> Kyle wrote:

> >   I'm starting to get interested in this subject.

> >   What are the shunts?  The loose steels between the two windings?

> >     Or are they short windings  like a shaded pole motor?  I'm looking

> >at my MOT right now . It's got a welded core  with three windings on

> it.

> >The lower is the primary. Above Seperated by several "loose" steels

> >(short stips of lamenations) are the filament and HV windings. Filament

>

> The shunts are the short "loose" steel inserts between the primary and

> the secondary.  As far as I can tell they limit the current which can be

> drawn by the secondary, much as the shunts in a NST do.

>

> Hmm- I wonder how much current you could get out of a MOT (briefly) if

> you were to remove the shunts...!

Pretty much what the mains can deliver. They would make a fine mini-

pig when ganged and used with an external charging choke/ballast.

However, you *must* ballast them on the secondary side because primary

side ballasting throws an extra voltage burden on both primary and

secondary winding insulation and insulation to the core. The drawback

of secondary side ballasting is the amount of inductance required. A

shunted transformer secondary would make a useful ballast inductor.

> Since my experiments indicate that a MOT w/ the secondary open (or

> removed) will limit the current drawn at 120 v to about 5 amps or so,

> you could probably use one in series with a pig (off of 120 v). That

> would guarantee that the current drawn would be less than 5 amps.

It is worth noting that while your ammeter says five amps, the

reality is somewhat different when it comes to peak currents which

the meter can't detect. Current waveform (at least in the shunted

transformer) is very distorted with high poeak currents. Can you

observe the primary current waveform of the unshunted transformer on

the o'scope? I'd be interested to know what it looks like.

> ..it just seems like we *should* be able to come up with some great use

> for those pesky MOTs.  So far that use has eluded me!  Perhaps we can

> make a food-heating device out one one, eh? :)

>

> -Bill

Well, two with the primaries paralleled and suitably phased with the

secondaries connected in series and rectified would give a nice DC

supply for use with a charging diode and choke to boost output to the

10kV mark. Highly suitable for a TC with a largish primary cap.

Malcolm

From:          "Malcolm Watts" <MALCOLM@directorate.wnp.ac.nz>

Organization:  Wellington Polytechnic, NZ

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Date:          Thu, 10 Sep 1998 08:31:28 +1200

Subject:       Re: Pig w/ only one HV bushing?

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Hi Bill,

> From:           "Bill the arcstarter" <arcstarter@hotmail.com>

> To:             "Tesla-2 Mailing List" <tesla-2@emachine.com>

> Subject:        Pig w/ only one HV bushing?

> Date sent:      Wed, 09 Sep 1998 07:16:35 PDT

> Send reply to:  "Tesla-2 Mailing List" <tesla-2@emachine.com>

> Coilers,

>  I have a lead on a nice sounding 10KVA pig.  18KV, 125 lbs, $200!  Not

> perfect for coiling but I think it is useable and is definitely a great

> deal.

>

> It contains three secondary terminals for 120-0-120 connection, but only

> one HV bushing.  The yard doesn't know how to wire the thing.

>

>  I assume the other connection is the case of the transformer.  But -

> wouldn't we want to ground that as well?  If this is the case then I

> can't get a truely floating output out of this unit.

>

> Any comments?

Sounds like it is designed for an earth return system. Anyone in

the States care to comment? AFAIK we don't use such a system anywhere

in NZ but I do know it is used in some parts of the USA.

Malcolm

From: McFalc0n@aol.com

Date: Wed, 9 Sep 1998 17:50:11 EDT

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: MEgahertz tesla coil

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Has any one ever thought of making a tesla secondary that resonates in the

megahertz?  It seems possible.  What other eerie FX do you think it could do,

besides possible longer sparks for the watt?  Anyone ever try, experiment,

think, do this?   Just another idea.

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Pole-pig w/one high leg

X-Mailer: IPAD 2.0

From: break7@juno.com (Tyla A. Break)

Date: Wed, 09 Sep 1998 21:20:37 EDT

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

I've seen the transformer you're talking about here in Calif. City of

Burbank is using some in residential areas.  Doesn't look too safe . I've

been out on dozens of calls for high voltages due to bad neutral joints.

The problem could be a lot worse if a higher voltage is "seeking ground".

!! They're probably safest when banked. (lots of redundant Neutral/ground

connections).

From:          "Malcolm Watts" <MALCOLM@directorate.wnp.ac.nz>

Organization:  Wellington Polytechnic, NZ

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Date:          Thu, 10 Sep 1998 13:41:23 +1200

Subject:       Re: MEgahertz tesla coil

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

> From:           McFalc0n@aol.com

> Date sent:      Wed, 9 Sep 1998 17:50:11 EDT

> To:             "Tesla-2 Mailing List" <tesla-2@emachine.com>

> Subject:        MEgahertz tesla coil

> Send reply to:  "Tesla-2 Mailing List" <tesla-2@emachine.com>

> Has any one ever thought of making a teslasecondary that resonates

> in the megahertz?  It seems possible.  What other eirie FX do you

> think it could do, besides possible longer sparks for the watt?

> ANyone ever try, experiment, think, do this?   Just another idea.

It has been done. The sparks aren't any better, generally smaller.

This may have something to do with a small capacitance = lack of

charge storage in the secondary.

Malcolm

X-Originating-IP: [208.16.145.183]

From: "Bill the arcstarter" <arcstarter@hotmail.com>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Pole-pig w/one high leg

Date: Wed, 09 Sep 1998 18:52:24 PDT

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

It was written:

>	I've seen the transformer you're talking about here in Calif. City of

>Burbank is using some in residential areas.  Doesn't look too safe. I've

>been out on dozens of calls for high voltages due to bad neutral joints.

>The problem could be a lot worse if a higher voltage is "seeking ground".

>!! They're probably safest when banked. (lots of redundant Neutral/ground

>connections).

Today on the way home I was looking at each and every transformer on the

string.  Each transformer was tapped off of a different high tension

line.  There were three lines - so the Y-connection idea probably holds.

I suppose it uses earth return - I didn't see a neutral conductor up at

the same level as the three high lines.

Now, below the pigs were the 120-0-120 lines.  The neutral appears to be

connected to the pig cases, but I never saw where(if) it connects to

ground.  Of course, the neutral is connected to ground at the service

box at each house, but I expect other connections to ground too!

The 120-0-120 wires were bussed from pole to pole, and connected to pigs

whenever present.  So it looks like there are two parallel grids

involved.  One is the high tension three phase grid.  The other is a

120-0-120 grid.  Every so many poles along, a pig couples the two

together.

One thing I don't understand is how, if each pig runs off of a different

phase, and is connected from hot to earth (Y-connection), then how can

the phases match on the 120-0-120 side?  Perhaps I'm missing part of the

story...

Another thing - if this pig in question (with one bushing) is meant for

line to earth connection on a Y-system, then what voltage will I see?

All I know is that it is rated for 18KV.  If I apply 240 to the input,

will I see (from the can to the bushing) 18Kv, or 18/sqrt(3) = 10.4Kv??

I guess this depends on how 3 phase systems are measured - line to line,

or line to neutral?  Does anyone know?

Thanks, everyone, for your input!  Please - more comments if there are

any!

-Bill

X-Originating-IP: [208.16.145.183]

From: "Bill the arcstarter" <arcstarter@hotmail.com>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Pig w/ only one HV bushing?

Date: Wed, 09 Sep 1998 19:00:43 PDT

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

Malcolm wrote:

>Sounds like it is designed for an earth return system. Anyone in

>the States care to comment? AFAIK we don't use such a system anywhere

>in NZ but I do know it is used in some parts of the USA.

It seems that the type of pig with two HV bushings are sort of rare out

in the real world, ie, actually hanging on poles!

I need to get some binoculars and go pig watching!

-Bill

From:          "Malcolm Watts" <MALCOLM@directorate.wnp.ac.nz>

Organization:  Wellington Polytechnic, NZ

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Date:          Thu, 10 Sep 1998 14:07:47 +1200

Subject:       Re: Pole-pig w/one high leg

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Hi Bill,

> From:           "Bill the arcstarter" <arcstarter@hotmail.com>

> To:             "Tesla-2 Mailing List" <tesla-2@emachine.com>

> Subject:        Re: Pole-pig w/one high leg

> Date sent:      Wed, 09 Sep 1998 18:52:24 PDT

> Send reply to:  "Tesla-2 Mailing List" <tesla-2@emachine.com>

> It was written:

> >   I've seen the transformer you're talking about here in Calif. City of

> >Burbank is using some in residential areas.  Doesn't look too safe. I've

> >been out on dozens of calls for high voltages due to bad neutral joints.

> >The problem could be a lot worse if a higher voltage is "seeking  ground".

> >!! They're probably safest when banked. (lots of redundant Neutral/ground

> >connections).

>

> Today on the way home I was looking at each and every transformer on the

> string.  Each transformer was tapped off of a different high tension

> line.  There were three lines - so the Y-connection idea probably holds.

> I suppose it uses earth return - I didn't see a neutral conductor up at

> the same level as the three high lines.

>

> Now, below the pigs were the 120-0-120 lines.  The neutral appears to be

> connected to the pig cases, but I never saw where(if) it connects to

> ground.  Of course, the neutral is connected to ground at the service

> box at each house, but I expect other connections to ground too!

>

> The 120-0-120 wires were bussed from pole to pole, and connected to pigs

> whenever present.  So it looks like there are two parallel grids

> involved.  One is the high tension three phase grid.  The other is a

> 120-0-120 grid.  Every so many poles along, a pig couples the two

> together.

>

> One thing I don't understand is how, if each pig runs off of a different

> phase, and is connected from hot to earth (Y-connection), then how can

> the phases match on the 120-0-120 side?  Perhaps I'm missing part of the

> story...

Usually, different phases are run to different houses to balance the

three phase load. Sometimes, two phases are run to a house and wired

to different circuits, again to assist with load balancing although

it is never perfect due to different loads being used at different

times.

> Another thing - if this pig in question (with one bushing) is meant for

> line to earth connection on a Y-system, then what voltage will I see?

> All I know is that it is rated for 18KV.  If I apply 240 to the input,

> will I see (from the can to the bushing) 18Kv, or 18/sqrt(3) = 10.4Kv??

> I guess this depends on how 3 phase systems are measured - line to line,

> or line to neutral?  Does anyone know?

Should be straight turns ratio.

Malcolm

X-Originating-IP: [208.16.145.183]

From: "Bill the arcstarter" <arcstarter@hotmail.com>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: MOTs

Date: Wed, 09 Sep 1998 19:13:37 PDT

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

Malcolm wrote:

>> The shunts are the short "loose" steel inserts between the primary and

>> the secondary.  As far as I can tell they limit the current which can be

>> drawn by the secondary, much as the shunts in a NST do.

>>

>> Hmm- I wonder how much current you could get out of a MOT (briefly) if

>> you were to remove the shunts...!

>

>Pretty much what the mains can deliver. They would make a fine mini-

>pig when ganged and used with an external charging choke/ballast.

Well, I have a problem with the terminology here.  I must insist that a

mini-pig be called a "Piglet" ;)  Nope, sorry, I didn't make this up

-saw it on someone's page long ago...

One thing I do know is that this particular transformer would (before I

hacked off the secondary and chiseled out the shunts) draw 30+ amps at

120 volts if I short-circuited the secondary winding!  If the shunts are

doing much, I can't see what!  My 20 amp fuses didn't care for that too

much.

>However, you *must* ballast them on the secondary side because primary

>side ballasting throws an extra voltage burden on both primary and

>secondary winding insulation and insulation to the core.

Oh?  I don't think I follow this.  Can you spell this out for me?  Why

does the insulation see more voltage if you have a limiting inductance

in the primary circuit?

>It is worth noting that while your ammeter says five amps, the

>reality is somewhat different when it comes to peak currents which

>the meter can't detect. Current waveform (at least in the shunted

>transformer) is very distorted with high poeak currents. Can you

>observe the primary current waveform of the unshunted transformer on

>the o'scope? I'd be interested to know what it looks like.

Well, my curiosity is up, so I'll go 'scope it.  But I don't have a good

way (digitial camera etc) to post any decent results to the web...  I

may have to sketch it (that won't be pretty).

Hmm - I could hold the flatbed scanner up against the screen of the 'scope! :)

Hey - one of my ammeters is an inductive clamp-on type w/ an analog

meter on it.  I wonder if this indicates RMS or average current levels?

Any guesses?

>Well, two with the primaries paralleled and suitably phased with the

>secondaries connected in series and rectified would give a nice DC

>supply for use with a charging diode and choke to boost output to the

>10kV mark. Highly suitable for a TC with a largish primary cap.

Hmm.  Anyone game to try?  Seems like it might be rather fragile.  Most

HV diodes don't take kindly to RF and charged caps all at once!

Ffft!

-Bill

X-Originating-IP: [208.16.145.183]

From: "Bill the arcstarter" <arcstarter@hotmail.com>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Pole-pig w/one high leg

Date: Wed, 09 Sep 1998 19:22:47 PDT

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

Hi Malcolm, list,

>Usually, different phases are run to different houses to balance the three phase load.

>Sometimes, two phases are run to a house and wired to different circuits, again to

>assist with load balancing although it is never perfect due to different loads being

>used at different times.

Well, here in the USA, every house has two phases - that is, the

120-N-120 connection.  All 120 volt loads are connected from one hot to

the neutral.  The breaker boxes are arranged so that half the 120 volt

breakers are from line1 to N, and the others are from line2 to N.

All 240V loads are connected from line1 to line2.

But this is probably the case in all locales, right?

>> Another thing - if this pig in question (with one bushing) is meant for

>> line to earth connection on a Y-system, then what voltage will I see?

>> All I know is that it is rated for 18KV.  If I apply 240 to the input,

>> will I see (from the can to the bushing) 18Kv, or 18/sqrt(3) = 10.4Kv??

>> I guess this depends on how 3 phase systems are measured - line to line,

>> or line to neutral?  Does anyone know?

>

>Should be straight turns ratio.

Well, yes, but thats the part I don't know!  All I know is 18KV, one

bushing, 60 Hz, 120-0-120 secondary!  Kinda frustrating.  The actual

unit isn't nearby so I can't go make a measurement. :(

Thanks!

-Bill

From:          "Malcolm Watts" <MALCOLM@directorate.wnp.ac.nz>

Organization:  Wellington Polytechnic, NZ

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Date:          Thu, 10 Sep 1998 14:33:08 +1200

Subject:       Re: MOTs

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Hi Bill,

Piglet it is.

> From:           "Bill the arcstarter" <arcstarter@hotmail.com>

> To:             "Tesla-2 Mailing List" <tesla-2@emachine.com>

> Subject:        Re: MOTs

> Date sent:      Wed, 09 Sep 1998 19:13:37 PDT

> Send reply to:  "Tesla-2 Mailing List" <tesla-2@emachine.com>

> Malcolm wrote:

>

> >> The shunts are the short "loose" steel inserts between the primary  and

> >> the secondary.  As far as I can tell they limit the current which can  be

> >> drawn by the secondary, much as the shunts in a NST do.

> >>

> >> Hmm- I wonder how much current you could get out of a MOT (briefly)  if

> >> you were to remove the shunts...!

> >

> >Pretty much what the mains can deliver. They would make a fine mini-

> >pig when ganged and used with an external charging choke/ballast.

>

> Well, I have a problem with the terminology here.  I must insist that a

> mini-pig be called a "Piglet" ;)  Nope, sorry, I didn't make this up

> -saw it on someone's page long ago...

>

> One thing I do know is that this particular transformer would (before I

> hacked off the secondary and chiseled out the shunts) draw 30+ amps at

> 120 volts if I short-circuited the secondary winding!  If the shunts are

> doing much, I can't see what!  My 20 amp fuses didn't care for that too

> much.

>

> >However, you *must* ballast them on the secondary side because primary

> >side ballasting throws an extra voltage burden on both primary and

> >secondary winding insulation and insulation to the core.

>

> Oh?  I don't think I follow this.  Can you spell this out for me?  Why

> does the insulation see more voltage if you have a limiting inductance

> in the primary circuit?

I should be more specific: if you are using the transformer to charge

a capacitor and the circuit is resonant at mains which is often the

case for TCs. Suppose you have a perfect transformer. There is no

leakage inductance to effect resonance and the mains sees a capacitor

with some impedance hung across it. Now include an inductor (shunt

leakage, external inductor or whatever). The mains is now in series

with a resonant circuit which can ring up such that the voltage

across its individual components reaches a value many times the

actual transformation ratio. If the inductor is external to the

transformer and in series with the cap and secondary, the voltages

across the cap and inductor hike upwards. If the inductor is in the

transformer, the secondary winding sees a voltage hike. If the

inductor is in the primary circuit, both transformer primary and

secondary see extra voltage (since both are tied by turns ratio in a

perfect transformer). From the primary choke's point of view, a

capacitance transformed by the turns ratio is seen and since voltage

across a capacitance hikes in a series resonant circuit.....

> >It is worth noting that while your ammeter says five amps, the

> >reality is somewhat different when it comes to peak currents which

> >the meter can't detect. Current waveform (at least in the shunted

> >transformer) is very distorted with high poeak currents. Can you

> >observe the primary current waveform of the unshunted transformer on

> >the o'scope? I'd be interested to know what it looks like.

>

> Well, my curiosity is up, so I'll go 'scope it.  But I don't have a good

> way (digitial camera etc) to post any decent results to the web...  I

> may have to sketch it (that won't be pretty).

>

> Hmm - I could hold the flatbed scanner up against the screen of the 'scope! :)

>

> Hey - one of my ammeters is an inductive clamp-on type w/ an analog

> meter on it.  I wonder if this indicates RMS or average current levels?

> Any guesses?

Not peak - that's for sure.

>

> >Well, two with the primaries paralleled and suitably phased with the

> >secondaries connected in series and rectified would give a nice DC

> >supply for use with a charging diode and choke to boost output to the

> >10kV mark. Highly suitable for a TC with a largish primary cap.

>

> Hmm.  Anyone game to try?  Seems like it might be rather fragile.  Most

> HV diodes don't take kindly to RF and charged caps all at once!

>

> Ffft!

Greg Leyh used such a charging system in both a 40kW and 130kW coil.

His charging choke was a high inductancew one in the secondary side

of the power trnasformer/s.

Malcolm

From:          "Malcolm Watts" <MALCOLM@directorate.wnp.ac.nz>

Organization:  Wellington Polytechnic, NZ

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Date:          Thu, 10 Sep 1998 14:38:05 +1200

Subject:       Re: Pole-pig w/one high leg

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Hi Bill,

When I mentioned phases I was referring to the phases of

the three phase side. Since a lot of your appliances are 120V

operated but some are 240V operated, they have a kludged scheme which

involves centre tapping the 240V.

> From:           "Bill the arcstarter" <arcstarter@hotmail.com>

> To:             "Tesla-2 Mailing List" <tesla-2@emachine.com>

> Subject:        Re: Pole-pig w/one high leg

> Date sent:      Wed, 09 Sep 1998 19:22:47 PDT

> Send reply to:  "Tesla-2 Mailing List" <tesla-2@emachine.com>

> Hi Malcolm, list,

>

> >Usually, different phases are run to different houses to balance the

> >three phase load. Sometimes, two phases are run to a house and wired

> >to different circuits, again to assist with load balancing although

> >it is never perfect due to different loads being used at different

> >times.

>

> Well, here in the USA, every house has two phases - that is, the

> 120-N-120 connection.  All 120 volt loads are connected from one hot to

> the neutral.  The breaker boxes are arranged so that half the 120 volt

> breakers are from line1 to N, and the others are from line2 to N.

> All 240V loads are connected from line1 to line2.

I think I repeated you above sorry. It was the three phase side of

things to which I was referring.

> But this is probably the case in all locales, right?

here in NZ all appliances are 230V or so.

> >> Another thing - if this pig in question (with one bushing) is meant  for

> >> line to earth connection on a Y-system, then what voltage will I see?

> >> All I know is that it is rated for 18KV.  If I apply 240 to the input,

> >> will I see (from the can to the bushing) 18Kv, or 18/sqrt(3) =  10.4Kv??

> >> I guess this depends on how 3 phase systems are measured - line to line,

> >> or line to neutral?  Does anyone know?

> >

> >Should be straight turns ratio.

>

> Well, yes, but thats the part I don't know!  All I know is 18KV, one

> bushing, 60 Hz, 120-0-120 secondary!  Kinda frustrating.  The actual

> unit isn't nearby so I can't go make a measurement. :(

Then turns ratio = 18000/240.

Cheers,

Malcolm

X-Originating-IP: [208.16.149.242]

From: "Bill the arcstarter" <arcstarter@hotmail.com>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Pole-pig w/one high leg

Date: Wed, 09 Sep 1998 20:01:30 PDT

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

Greetings Malcolm!

>          When I mentioned phases I was referring to the phases of

>the three phase side. Since a lot of your appliances are 120V

>operated but some are 240V operated, they have a kludged scheme which

>involves centre tapping the 240V.

No arguments here! :)  My comments stemmed from your comment about

sometimes one house being powered by two phases...  Here this is always

true for the 120v phases.  Residences are very very rarely  supplied

higher voltages or multiple phases (other than the goofy 2-phase

120/240).

>> But this is probably the case in all locales, right?

>

>here in NZ all appliances are 230V or so.

Nope - I didn't know that.  So you get one hot and one neutral from the

pole into your house?

So, Malcolm, if you use USA-style NSTs, do you run the primaries in

series and parallel the secondaries??  Or just use a 240:120

transformer?

>> >> Another thing - if this pig in question (with one bushing) is meant for

>> >> line to earth connection on a Y-system, then what voltage will I see?

>> >> All I know is that it is rated for 18KV.  If I apply 240 to the input,

>> >> will I see (from the can to the bushing) 18Kv, or 18/sqrt(3) =  10.4Kv??

>> >> I guess this depends on how 3 phase systems are measured - line to line,

>> >> or line to neutral?  Does anyone know?

>> >

>> >Should be straight turns ratio.

>>

>> Well, yes, but thats the part I don't know!  All I know is 18KV, one

>> bushing, 60 Hz, 120-0-120 secondary!  Kinda frustrating.  The actual

>> unit isn't nearby so I can't go make a measurement. :(

>

>Then turns ratio = 18000/240.

Only if the output is 18KV - which I'm not sure is the case if this pig

was meant for three phase service!

Arggh!  OK, so we assume the "18KV" means Y-connected, measured line to

case.  So 240 * 18000/240 = 18Kv output. ;)  Or so we hope.

I'll just go buy the thing - then I'll know!

-Bill "I want 12-phase power in my basement like the steel companies

have" the arcstarter

X-Originating-IP: [208.16.149.242]

From: "Bill the arcstarter" <arcstarter@hotmail.com>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: MOTs

Date: Wed, 09 Sep 1998 20:09:54 PDT

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

Hi Malcolm, list,

This has been quite the exchange!

I wrote:

>> Well, I have a problem with the terminology here.  I must insist that a

>> mini-pig be called a "Piglet" ;)  Nope, sorry, I didn't make this up

>> -saw it on someone's page long ago...

What - no comments? :) Oink oink oink.

>> >However, you *must* ballast them on the secondary side because primary

>> >side ballasting throws an extra voltage burden on both primary and

>> >secondary winding insulation and insulation to the core.

>>

>> Oh?  I don't think I follow this.  Can you spell this out for me?  Why

>> does the insulation see more voltage if you have a limiting inductance

>> in the primary circuit?

>

>I should be more specific: if you are using the transformer to charge

>a capacitor and the circuit is resonant at mains which is often the

>case for TCs. Suppose you have a perfect transformer. There is no

>leakage inductance to effect resonance and the mains sees a capacitor

>with some impedance hung across it. Now include an inductor (shunt

>leakage, external inductor or whatever). The mains is now in series

>with a resonant circuit which can ring up such that the voltage

>across its individual components reaches a value many times the

>actual transformation ratio. If the inductor is external to the

>transformer and in series with the cap and secondary, the voltages

>across the cap and inductor hike upwards. If the inductor is in the

>transformer, the secondary winding sees a voltage hike. If the

>inductor is in the primary circuit, both transformer primary and

>secondary see extra voltage (since both are tied by turns ratio in a

>perfect transformer). From the primary choke's point of view, a

>capacitance transformed by the turns ratio is seen and since voltage

>across a capacitance hikes in a series resonant circuit.....

Ah yes.  Now I see.  Thanks for writing that out!  I hadn't considered

the case of resonant charging...

>Greg Leyh used such a charging system in both a 40kW and 130kW coil.

>His charging choke was a high inductancew one in the secondary side

>of the power trnasformer/s.

Yea - I recall that vaguely.  Scary stuff!

Is the purpose of that large inductor more of a RF block, or is it's

principal function to store (and flywheel) primary energy?  Or both!

Obviously it doesn't take the place of the primary capacitor, so I'd

guess it's more of an RF blocker...?

Thanks!

-Bill the arcstarter

Starting arcs in Cinci, OH

http://www.geocities.com/CapeCanaveral/Hangar/6160

From:          "Malcolm Watts" <MALCOLM@directorate.wnp.ac.nz>

Organization:  Wellington Polytechnic, NZ

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Date:          Thu, 10 Sep 1998 15:17:40 +1200

Subject:       Re: Pole-pig w/one high leg

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Hi Bill,

> From:           "Bill the arcstarter" <arcstarter@hotmail.com>

> To:             tesla-2@emachine.com, malcolm@directorate.wnp.ac.nz

> Subject:        Re: Pole-pig w/one high leg

> Date sent:      Wed, 09 Sep 1998 20:01:30 PDT

> Greetings Malcolm!

>

> >          When I mentioned phases I was referring to the phases of

> >the three phase side. Since a lot of your appliances are 120V

> >operated but some are 240V operated, they have a kludged scheme which

> >involves centre tapping the 240V.

>

> No arguments here! :)  My comments stemmed from your comment about

> sometimes one house being powered by two phases...  Here this is always

> true for the 120v phases.  Residences are very very rarely  supplied

> higher voltages or multiple phases (other than the goofy 2-phase

> 120/240).

OK.  Here, one might have a capacity of so many amps at 230V from one

of the three distribution phases or if required (perhaps specially

requested), one might have a capacity of so many amps at 230V from

two of the three distribution phases fed to one's home.

> > >> But this is probably the case in all locales, right?

> >

> >here in NZ all appliances are 230V or so.

>

> Nope - I didn't know that.  So you get one hot and one neutral from the

> pole into your house?

Yes. The neutral is grounded both at the distribution transformer

(does a delta-star job from the 11kV delta distribution feed) and at

each house.

> So, Malcolm, if you use USA-style NSTs, do you run the primaries in

> series and parallel the secondaries??  Or just use a 240:120

> transformer?

Never used one. All the NSTs I've come across here have a 220/240V

primary.

> >> >> Another thing - if this pig in question (with one bushing) is meant for

> >> >> line to earth connection on a Y-system, then what voltage will I  see?

> >> >> All I know is that it is rated for 18KV.  If I apply 240 to the input,

> >> >> will I see (from the can to the bushing) 18Kv, or 18/sqrt(3) = 10.4Kv??

> >> >> I guess this depends on how 3 phase systems are measured - line to line,

> >> >> or line to neutral?  Does anyone know?

> >> >

> >> >Should be straight turns ratio.

> >>

> >> Well, yes, but thats the part I don't know!  All I know is 18KV, one

> >> bushing, 60 Hz, 120-0-120 secondary!  Kinda frustrating.  The actual

> >> unit isn't nearby so I can't go make a measurement. :(

> >

> >Then turns ratio = 18000/240.

>

> Only if the output is 18KV - which I'm not sure is the case if this pig

> was meant for three phase service!

If you have a 240V winding and an 18kV winding then surely it is a

single phase situation and you are dealing with a straight

transformer? I mean if you feed 240V into the 240V winding you'll get

18kV out won't you? Or perhaps it is more like you have 18kV between

the phases in the delta distribution system in which case a single

phase transformer like that is star connected to one phase. Then

you'd have 11kV to 240V which sounds more like it. Perhaps it would be

useful to measure the turns ratio.

> Arggh!  OK, so we assume the "18KV" means Y-connected, measured line to

> case.  So 240 * 18000/240 = 18Kv output. ;)  Or so we hope.

>

> I'll just go buy the thing - then I'll know!

>

> -Bill "I want 12-phase power in my basement like the steel companies

> have" the arcstarter

Malcolm (gimme a simple power station).

** If it's under 10 Amps it's leakage current **

From:          "Malcolm Watts" <MALCOLM@directorate.wnp.ac.nz>

Organization:  Wellington Polytechnic, NZ

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Date:          Thu, 10 Sep 1998 15:34:11 +1200

Subject:       Re: MOTs

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Continuing the "exchange":

> From:           "Bill the arcstarter" <arcstarter@hotmail.com>

> To:             tesla-2@emachine.com, malcolm@directorate.wnp.ac.nz

> Subject:        Re: MOTs

> Date sent:      Wed, 09 Sep 1998 20:09:54 PDT

> Hi Malcolm, list,

>  This has been quite the exchange!

>

> I wrote:

> >> Well, I have a problem with the terminology here.  I must insist that  a

> >> mini-pig be called a "Piglet" ;)  Nope, sorry, I didn't make this up

> >> -saw it on someone's page long ago...

>

> What - no comments? :) Oink oink oink.

I did actually, right at the top of that post I think.

> >> >However, you *must* ballast them on the secondary side because  primary

> >> >side ballasting throws an extra voltage burden on both primary and

> >> >secondary winding insulation and insulation to the core.

> >>

> >> Oh?  I don't think I follow this.  Can you spell this out for me?  Why

> >> does the insulation see more voltage if you have a limiting inductance

> >> in the primary circuit?

> >

> >I should be more specific: if you are using the transformer to charge

> >a capacitor and the circuit is resonant at mains which is often the

> >case for TCs. Suppose you have a perfect transformer. There is no

> >leakage inductance to effect resonance and the mains sees a capacitor

> >with some impedance hung across it. Now include an inductor (shunt

> >leakage, external inductor or whatever). The mains is now in series

> >with a resonant circuit which can ring up such that the voltage

> >across its individual components reaches a value many times the

> >actual transformation ratio. If the inductor is external to the

> >transformer and in series with the cap and secondary, the voltages

> >across the cap and inductor hike upwards. If the inductor is in the

> >transformer, the secondary winding sees a voltage hike. If the

> >inductor is in the primary circuit, both transformer primary and

> >secondary see extra voltage (since both are tied by turns ratio in a

> >perfect transformer). From the primary choke's point of view, a

> >capacitance transformed by the turns ratio is seen and since voltage

> >across a capacitance hikes in a series resonant circuit.....

>

> Ah yes.  Now I see.  Thanks for writing that out!  I hadn't considered

> the case of resonant charging...

>

> >Greg Leyh used such a charging system in both a 40kW and 130kW coil.

> >His charging choke was a high inductancew one in the secondary side

> >of the power trnasformer/s.

>

> Yea - I recall that vaguely.  Scary stuff!

I had a good look inside the bunker of Electrum when Greg was here in

NZ.  It was very tidy. The rectifiers were stacked and self-snubbed.

> Is the purpose of that large inductor more of a RF block, or is it's

> principal function to store (and flywheel) primary energy?  Or both!

> Obviously it doesn't take the place of the primary capacitor, so I'd

> guess it's more of an RF blocker...?

His RF blocker consisted of an aircore choke that smalltime TC

builders like me would drool over: 12"x36" closewound with 1mm wire

or so.

The charging inductor was a heavy iron cored beast following on

from the three phase rectifiers. Inductance was in the Henries and

its value was tailored to allow the primary cap to charge to about

2Vpk at the maximum break rate the rotary ran at. The basic idea is

that cap charge current passes thru the inductor which stores energy

while until the cap voltage gets near the peak charging voltage. At

this point, the voltage across the inductor has fallen and in an

attempt to keep the current going, its mag field collapses and

generates an EMF in its windings which continues to charge the cap.

Charging stops when the mag field runs out of steam. A bit crude but

that's the basics. Theoretically you can hit 2Vpk on a half cycle

charge if there are no losses in the system.

Cheers,

Malcolm

Date: Wed, 09 Sep 1998 22:34:58 -0500

From: Bert Hickman <bert.hickman@aquila.com>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Organization: Stoneridge Engineering

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Pole-pig w/one high leg

Sender: tesl-err@emachine.com

Bill,

My comments are interspersed below...

-- Bert --

Bill the arcstarter wrote:

>

<SNIP>

> Today on the way home I was looking at each and every transformer on the

> string.  Each transformer was tapped off of a different high tension

> line.  There were three lines - so the Y-connection idea probably holds.

> I suppose it uses earth return - I didn't see a neutral conductor up at

> the same level as the three high lines.

>

> Now, below the pigs were the 120-0-120 lines.  The neutral appears to be

> connected to the pig cases, but I never saw where(if) it connects to

> ground.  Of course, the neutral is connected to ground at the service

> box at each house, but I expect other connections to ground too!

>

> The 120-0-120 wires were bussed from pole to pole, and connected to pigs

> whenever present.  So it looks like there are two parallel grids

> involved.  One is the high tension three phase grid.  The other is a

> 120-0-120 grid.  Every so many poles along, a pig couples the two

> together.

>

> One thing I don't understand is how, if each pig runs off of a different

> phase, and is connected from hot to earth (Y-connection), then how can

> the phases match on the 120-0-120 side?  Perhaps I'm missing part of the

> story...

Curious indeed!! The phasing MUST be the same on the H1/H2 AND the

X1/X2/X3 connections of each to permit parallel interconnection. Check

again, and I'll look at some of our local configurations as well.

Outputs should really be neither Delta or Wye, but separate, since

residential service would normally be simple single-phase. If some

paralleling is going on, the phasing would have to match...

>

> Another thing - if this pig in question (with one bushing) is meant for

> line to earth connection on a Y-system, then what voltage will I see?

> All I know is that it is rated for 18KV.  If I apply 240 to the input,

> will I see (from the can to the bushing) 18Kv, or 18/sqrt(3) = 10.4Kv??

> I guess this depends on how 3 phase systems are measured - line to line,

> or line to neutral?  Does anyone know?

Single-phase transformers will have either one or two voltage ratings on

the faceplate for the H1-H2 primary inputs. If there's only one number,

this refers to the winding's voltage when connected in Delta (Vwinding =

Vphase-to-phase). If there are two numbers (for example 19,900/34500)

the first number refers to a Delta connection, and the second refers to

a Wye connection, where the transformer would be connected to two others

with all the H1's tied together, and the H2's tied to each phase (or

vice-versa). In this case, the phase-to-phase voltage will be 1.73X the

rating of a single transformer. 19.9KV X 1.73 ~ 34.5 kV. I agree with

Malcolm - your transformer sounds like it will output 18 kV between H1

and H2 with 240 VAC applied to X1 and X3.

>

> Thanks, everyone, for your input!  Please - more comments if there are

> any!

> -Bill

Hope this helps!

-- Bert --

Date: Wed, 9 Sep 1998 22:03:15 -0600

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Daniel Lockhart <molly@cmn.net>

Subject: Re: MOTs

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

>Continuing the "exchange":

>> snip! <<

>> >Greg Leyh used such a charging system in both a 40kW and 130kW coil.

>> >His charging choke was a high inductancew one in the secondary side

>> >of the power trnasformer/s.

>>

>> Yea - I recall that vaguely.  Scary stuff!

>

>I had a good look inside the bunker of Electrum when Greg was here in

>NZ.  It was very tidy. The rectifiers were stacked and self-snubbed.

>

>> Is the purpose of that large inductor more of a RF block, or is it's

>> principal function to store (and flywheel) primary energy?  Or both!

>> Obviously it doesn't take the place of the primary capacitor, so I'd

>> guess it's more of an RF blocker...?

>

>His RF blocker consisted of an aircore choke that smalltime TC

>builders like me would drool over: 12"x36" closewound with 1mm wire

>or so.

>     The charging inductor was a heavy iron cored beast following on

>from the three phase rectifiers. Inductance was in the Henries and

>its value was tailored to allow the primary cap to charge to about

>2Vpk at the maximum break rate the rotary ran at. The basic idea is

>that cap charge current passes thru the inductor which stores energy

>while until the cap voltage gets near the peak charging voltage. At

>this point, the voltage across the inductor has fallen and in an

>attempt to keep the current going, its mag field collapses and

>generates an EMF in its windings which continues to charge the cap.

>Charging stops when the mag field runs out of steam. A bit crude but

>that's the basics. Theoretically you can hit 2Vpk on a half cycle

>charge if there are no losses in the system.

>

>Cheers,

>Malcolm

I've been loosely following this thread, as it's mostly been over my head.

This setup by Greg Leyh as described above looks very interesting. Could

you explain it a bit more simply, Malcom? It sounds very intriguing.

Thanks,

Daniel Lockhart

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: R.E Trans w/ one high leg

X-Mailer: IPAD 2.0

From: break7@juno.com (Tyla A. Break)

Date: Thu, 10 Sep 1998 00:44:01 EDT

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

I'm sure there is a connection to the neutral somewhere, Earth

return would be too dangerous!  Often the grounding is hard to see . It's

usually a solid #4 running down the pole covered with a thin strip of

wood. There is always a ground at every pole with a transformer. I have heard

of cases where the ground connection was damaged or lose or uncovered and

men were killed while climbing.

I also often hear people refer to residential power as 2 phase.

It' actually both ends of a single phase .

Here in Pasadena CA. there are some houses supplied with 3 phase

power! Why I don't know, But they are only using the one phase for air-

conditioning (3 phase)  all other circuits only use the other 2  phases.

It would be great for running other machinery, no expensive rotary

phase - adder needed!

Also the big- timers in Bev. Hills often have 600 -1200 amp 3

phase services.

Perfect for powering(really) BIG coils!

Kyle  Break7@juno.com

From:          "Malcolm Watts" <MALCOLM@directorate.wnp.ac.nz>

Organization:  Wellington Polytechnic, NZ

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Date:          Thu, 10 Sep 1998 16:49:34 +1200

Subject:       Re: MOTs

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Hi Daniel,

> Date sent:      Wed, 9 Sep 1998 22:03:15 -0600

> To:             "Tesla-2 Mailing List" <tesla-2@emachine.com>

> From:           Daniel Lockhart <molly@cmn.net>

> Subject:        Re: MOTs

> Send reply to:  "Tesla-2 Mailing List" <tesla-2@emachine.com>

<snip>

> >     The charging inductor was a heavy iron cored beast following on

> >from the three phase rectifiers. Inductance was in the Henries and

> >its value was tailored to allow the primary cap to charge to about

> >2Vpk at the maximum break rate the rotary ran at. The basic idea is

> >that cap charge current passes thru the inductor which stores energy

> >while until the cap voltage gets near the peak charging voltage. At

> >this point, the voltage across the inductor has fallen and in an

> >attempt to keep the current going, its mag field collapses and

> >generates an EMF in its windings which continues to charge the cap.

> >Charging stops when the mag field runs out of steam. A bit crude but

> >that's the basics. Theoretically you can hit 2Vpk on a half cycle

> >charge if there are no losses in the system.

> >

> >Cheers,

> >Malcolm

>

> I've been loosely following this thread, as it's mostly been over my head.

> This setup by Greg Leyh as described above looks very interesting. Could

> you explain it a bit more simply, Malcom? It sounds very intriguing.

>

> Thanks,

>

> Daniel Lockhart

Not much more simply I'm afraid. It isn't worded all that well I

know :(  Greg wrote an article in TCBA News circa 1991 describing the

40kW coil so you might have a look there.

Basically: suppose you have a battery, a choke, a capacitor.

Connect the lot in series so that the diode conducts. The moment thew

circuit is completed, current starts to flow through the circuit and

builds up over time (inductors oppose current change by generating an

opposing EMF as a magnetic field builds up around them (and in the

core if one exists)). The capacitor starts charging because of the

current flow. As the capacitor charges, the voltage across the

inductor decreases after building up to some value. The moment the

inductor current starts to decrease, its magnetic field starts

collapsing in on the windings generating further current flow. This

current continues to charge the capacitor. At the point where the

inductor starts producing its EMF, the polarity at its terminals

reverses which effectively places the EMF it generates in series with

the battery. The nett result is that the capacitor can charge up

beyond the battery voltage and the diode prevents the cap discharging

back through the circuit. Try this for yourself and measure

the final capacitor voltage.

Resonant circuits are all about energy storage. A resonant

circuit requires at least two components capable of storing energy

and these two components must store energy in different forms from

each other. The oscillation produced by a resonant circuit is the

process of swapping energy from one form (static) to another

(kinetic).

Malcolm

From:          "Malcolm Watts" <MALCOLM@directorate.wnp.ac.nz>

Organization:  Wellington Polytechnic, NZ

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Date:          Thu, 10 Sep 1998 17:06:56 +1200

Subject:       Re: MOTs

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

At the risk of making this even more confusing, a correction to what

I wrote:

>      Basically: suppose you have a battery, a choke, a capacitor.

> Connect the lot in series so that the diode conducts. The moment thew

> circuit is completed, current starts to flow through the circuit and

> builds up over time (inductors oppose current change by generating an

> opposing EMF as a magnetic field builds up around them (and in the

> core if one exists)). The capacitor starts charging because of the

> current flow. As the capacitor charges, the voltage across the

> inductor decreases after building up to some value.

In fact the voltage initially across the inductor was the full

battery voltage minus one diode drop (0.7V or so). It is the current

that builds uip to some value and then falls again. The voltage

across the inductor is falling all the time the cap charges.

> The moment the

> inductor current starts to decrease, its magnetic field starts

> collapsing in on the windings generating further current flow. This

> current continues to charge the capacitor. At the point where the

> inductor starts producing its EMF, the polarity at its terminals

> reverses which effectively places the EMF it generates in series with

> the battery. The nett result is that the capacitor can charge up

> beyond the battery voltage and the diode prevents the cap discharging

> back through the circuit. Try this for yourself and measure

> the final capacitor voltage.

I just did this with a choke of 20H or so I had lying on the bench,

a ten volt power supply, a 1N4004 diode and a 6.8uF polyester

capacitor. I monitored the cap voltage using the o'scope (high

impedance input). My capacitor hit 17Volts which showed that the

circuit has some losses but its Q is considerably greater than 0.5,

maybe 5 - 10.

If you watch what happens to the capacitor on the scope and

short the diode out, you will see the capacitor voltage build up and

oscillate around the power supply voltage until the ring dies out.

My little circuit rings at about 10Hz so the inductor value is

about 37 Henries or so.

Malcolm

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Date: Thu, 10 Sep 1998 05:00:52 -0700

From: "paul m" <fleischwulf@mailexcite.com>

Cc:

X-Sent-Mail: off

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

Subject: My observations (New pig config.)

X-Sender-Ip: 208.153.7.9

Organization: MailExcite  (http://www.mailexcite.com:80)

Sender: tesl-err@emachine.com

Bill,

When I first looked at the new style pigs (Single terminal) I was confused.

I knew that the HV side could not be connected to ground. I mean if the 4 AWG

ground wire should break you would wind up with one hell of an arc, not to mention

the electrocution hazard. The pigs that I saw had both the case and the ground

connected to the neutral on the low voltage side. The other insulated terminal is

connected to the case. The odd thing is that there seems to be a spark gap between

the HV terminal and ground. I talked with a line man for the power company. He tells

me that there is a breaker on the HV side. I think that the gap must be for surge protection.

Your thoughts ?

paul m

---

Einigheit und Wahrheit

From: "Basura, Brian" <brian.basura@unistudios.com>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: RE: Pole-pig w/one high leg

Date: Thu, 10 Sep 1998 07:35:51 -0700

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

I stopped and looked at this setup this morning (In Burbank BTW...).  If one

looks closely you will note that the Neutral wire is bussed between each

transformer but the L1-L2 wires serve groups of houses and are not

paralleled between transformers.  This is a little hard to see since some

poles contain the L1-L2 from different transformers.  These wires are

located close to each other but on different insulators. To further confuse

the casual observer the N is also located on these insulators but bussed

together.

I spoke with a rep from the Burbank utility company about these transformers

last year.  He told me they were configured as auto-transformers and there

were inherent dangers since the HV and LV sides were directly tied together.

He went on to say their primary concern was loss of neutral bonding with the

risk of feeding HV to the residence.

Hope this helps,

Brian

Date: Thu, 10 Sep 1998 09:46:54 -0500

From: Eric Davidson <edavidson@icva.gov>

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Pole-pig w/one high leg

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Bill and others:

As I said in an earlier post, 95% of distribution circuits are 4 wire Y.

Single HV bushing transformers are connected phase to neutral.  The

primary has only one HV bushing, the other end is internally connected

to the tank.  The ground connection is about a third the way up from the

bottom of the tank near the back (where it is connected to the pole).

There is usually a bonding strap connecting the LV neutral to the tank

as well. In any event, the HV neutral and LV neutral are ALL grounded

conductors, connected to the same ground (earth).  Yes, each single

phase transformer is connected to a different phase.  This help balance

the load provides a more stable neutral and reduces third harmonic

voltages. There are not really two parallel grids of HV and LV. The HV

grid is probably supplied from one substation (depending on the size of

the neighborhood or town).  It is there the voltage is stepped from 34.5

or 69 kV to 13,800 for distribution (other voltages are also used).

There is no 120/240 volt grid, and there is no bussing of lines.  As you

observed, each transformer is fed from a different phase, the

secondaries could not be connected together in a parallel single phase

configuration. The phases don't match on the secondary if the primaries

are fed from different phases.  It is possible that your neighbor may be

on a different phase.  I think the part you are missing is that there is

not one LV grid and the transformers are not in parallel.  Each

transformer feeds a separate group of customers.  The voltage you will

see on a single bushing transformer is the phase to neutral (ground)

voltage.  The name plate will probable say something like 12470 GRD

Y/7200 or 13800 GRD Y/7960.  You will get the lower of the numbers for a

HV. Phase to neutral voltage is E/sqrt3.  Hope this helps!

--

Have a good day!

Eric Davidson

edavidson@icva.gov

Date: Thu, 10 Sep 1998 18:33:20 +0100

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Stephen Bell <steve@breakfix.demon.co.uk>

Subject: Spark Gap adjustments

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Hi Guys,

I've been reading all the postings to the list, half of which

were over my head <:-\ and a lot of the rest I have already read about

in archives and other sources of info.  I am not yet qualified to answer

questions posted as I have not finished building my first coil, but I

appreciate the answers I get to my questions  -  and here's another one!

It is often stated that you should start tuning a TC with the spark gap

set small, to limit the max voltage.  Also, a common static gap design

consists of copper tubes in series.  When making adjustments to the

total gap size do you move all the tubes the same amount or just change

the size of one individual gap?

TIA

------------------------------------------------

Stephen Bell        (steve@breakfix.demon.co.uk)

Stuff also available at www.breakfix.demon.co.uk

------------------------------------------------

From: Dsurfr@aol.com

Date: Thu, 10 Sep 1998 17:49:04 EDT

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Spark Gap adjustments

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Steve: Your pipes are spaced so that the total of all the gaps between the

pipes is the desired gap distance. For example I have 11 pipes (10 gaps)

spaced at .03" for a total gap of .3". When you make your tank circuit

connections using the first & last pipes as connecting points you are running

at total gap. To decrease the amount of gap simply connect fewer pipes in

series by connecting to a pipe closer to the first one. I normally run my coil

with the connection at the 10th pipe (9 gaps) for a total gap of .27" To tune

your coil at a lower gap cut down on the number of the pipes you are

connecting - don't reposition them. Mike Hammer, who moderates this list,

recommends a total gap spacing of .25" for a 12kV 30mA neon & .30" for a 15kV

30mA neon and his advice has always been both quite accurate & of much help to

me. Hope this helps, Jim

Date: Thu, 10 Sep 1998 23:14:50 +0100

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Stephen Bell <steve@breakfix.demon.co.uk>

Subject: Re: Spark Gap adjustments

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

In message <c7017ac.35f84950@aol.com>, Dsurfr@aol.com writes

>Steve: Your pipes are spaced so that the total of all the gaps between the

>pipes is the desired gap distance. For example I have 11 pipes (10 gaps)

>spaced at .03" for a total gap of .3". When you make your tank circuit

>connections using the first & last pipes as connecting points you are running

>at total gap. To decrease the amount of gap simply connect fewer pipes in

>series by connecting to a pipe closer to the first one. I normally run my coil

>with the connection at the 10th pipe (9 gaps) for a total gap of .27" To tune

>your coil at a lower gap cut down on the number of the pipes you are

>connecting - don't reposition them. Mike Hammer, who moderates this list,

>recommends a total gap spacing of .25" for a 12kV 30mA neon & .30" for a 15kV

>30mA neon and his advice has always been both quite accurate & of much help to

>me. Hope this helps, Jim

Why didn't I think of that!

Thanks!

------------------------------------------------

Stephen Bell        (steve@breakfix.demon.co.uk)

Stuff also available at www.breakfix.demon.co.uk

------------------------------------------------

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Big Coils

X-Mailer: IPAD 2.0

From: break7@juno.com (Tyla A. Break)

Date: Thu, 10 Sep 1998 23:05:28 EDT

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

I just want to get the general feeling about big coils. I've been

told a 6.5" is pretty big. But I've got a twin brother that's really

pushing me to go bigger. So I finally gave in this week-end . I rigged up

a winding jig and added a gear motor, at 36 RPM coiling goes quickly!  So

here's what I got so far

Form-   10.75 Dia. x 33.5 winding length.

Wire size - 20 gauge .0335

approx 988 turns

approx 2782'  !!

27 ohms (measured)

Of course I followed Mike's directions for form prep.

Question is, how many kw"s can I feed this baby?

If anyone is interested, it only took about 35 minutes to wind.

Kyle Break7@juno.com

From: ralph.hartwell@emachine.com (Ralph Hartwell)

Subject:  Tesla Symposium Report

Date: Fri, 11 Sep 1998 04:07:00 GMT

Organization: The Energy Machine Information System 504-733-3462

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

Hi everyone!  I thought some of you might like to hear about what went

on at the international Tesla Society convention this year.  Michael

posted his report, and, with his permission, I'm forwarding it to the

Tesla-2 list.

Ralph

----------------------------------------

From: Michael Riversong <rivedu@EARTHLINK.NET>

To: Multiple recipients of list USA-TESLA <USA-TESLA@list.iex.net>

Date:         Wed, 9 Sep 1998 13:23:35 -0700

Subject:      Tesla Symposium Report

Here's a report from the perspective of a volunteer with the Tesla Society.

I also do some documentation writing for several scientists.

Just finished up with the 1998 Tesla Symposium, the first one held in

Denver.  As usual, it was great.  We got a lot of enthusiastic feedback from

attendees.  Unfortunately due to the Labor Day weekend scheduling i was

unable to be at some sessions, and did not actually get to hear any entire

presentations.  The timing also cut our attendance to some degree.  However,

i did manage to have several enlightening conversations with a number of

inventors and builders.

On the medical front, things are progressing well.  Most significant is the

refinement of electronic pulse technology by Ed Skilling, who is now in his

70's and still going strong.  He had several new research devices which are

commercially available and viable.  Of course current US laws do not allow

any medical claims to be made, but anyone can buy these things and see for

themselves the medicine of the future -- a cheap and effective set of

technologies which will eventually be able to address almost any physical

condition.

Three new motor builders showed up with promising devices.  These were Mark

Deal, Mark Richards, and Don Martin.  All three are managing to get slight

increases in power delivered from their motors, compared to input.  At this

stage, the extra power is ephemeral and difficult to harness, but it is

there.  Deal in particular has a very good theoretical framework.  Martin

was able to bring two motors down, and although he doesn't have a clue how

they work, he was able to give us some excellent ideas.

Paul Pantone has done excellent work in commercializing new technologies.

He has set up a small and closely held company in Utah which is now able to

distribute several devices.  Most significant is a "plasma tube" which can

be added to cars and significantly increases gas mileage by using a sort of

chemical feedback reaction on the exhaust gases.  These can be purchased as

part of standalone electric generators, or as kits which can go on many

standard engine models.  One guy brought a Cadillac which had just gotten

one installed.  Also demonstrated were some trucks which had been modified.

More standard alternative technology was exhibited by Steven Joyner of

Torrington, Wyoming.  He is singlehandedly reviving the whole field of wind

power.

The field of alternative technology is still in its infancy.  We still have

to contend with possibly fraudulent people, but the situation is not as bad

as it used to be even five years ago.  The worst frauds have been

discredited, partly through the efforts of the Tesla Society.

There is a general consensus that young researchers are very much needed.

Some of the greatest progress will be made by those who are too

inexperienced to know it can't be done.  Several educational kits for

children are available through the Tesla Society.  Also, videos of most

Symposium speakers, including the ones mentioned here, are available.  Phone

number for the Society is 719-475-0918.  Office hours are 10 am - 4 pm,

Monday through Saturday, Mountain time.

I also am willing to privately send addresses of several developers to those

who are interested.

-- Michael Riversong **

P.O. Box 2775, Cheyenne, Wyoming 82003 ** (303)829-0774

Professional Harpist **

Date: Thu, 10 Sep 1998 23:27:19 -0500

From: Bert Hickman <bert.hickman@aquila.com>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Organization: Stoneridge Engineering

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Big Coils

Sender: tesl-err@emachine.com

Tyla A. Break wrote:

>

>         I just want to get the general feeling about big coils. I've been

> told a 6.5" is pretty big. But I've got a twin brother that's really

> pushing me to go bigger. So I finally gave in this week-end . I rigged up

> a winding jig and added a gear motor, at 36 RPM coiling goes quickly!  So

> here's what I got so far

>         Form-   10.75 Dia. x 33.5 winding length.

>         Wire size - 20 gauge .0335

>         approx 988 turns

>         approx 2782'  !!

>         27 ohms (measured)

>

>         Of course I followed Mike's directions for form prep.

>

>         Question is, how many kw"s can I feed this baby?

>         If anyone is interested, it only took about 35 minutes to wind.

>

>                 Kyle Break7@juno.com

>

Kyle,

This is very similar to my 10" x 31" secondary, wound with #21 AWG. I

use a 32 x 8" toroid, and presently drive it with about 3.5 KVA, getting

65" streamers. Although it can handle more power, I'm at the limit of

being able to run it inside the screened porch with me being in the same

room... :^)

Your secondary should easily support at least 7-10 KVA depending upon

how well your secondary is insulated. I've coated mine with a 1/16"

thick layer of Behr Build50 to reduce corona and do E-field smoothing.

At these levels a larger toroid will be necessary (in BOTH dimensions).

Safe coilin' to you!

-- Bert --

From:          "Malcolm Watts" <MALCOLM@directorate.wnp.ac.nz>

Organization:  Wellington Polytechnic, NZ

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Date:          Fri, 11 Sep 1998 16:36:10 +1200

Subject:       Re: Big Coils

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Hi Kyle,

> To:             "Tesla-2 Mailing List" <tesla-2@emachine.com>

> Subject:        Big Coils

> From:           break7@juno.com (Tyla A. Break)

> Date sent:      Thu, 10 Sep 1998 23:05:28 EDT

> Send reply to:  "Tesla-2 Mailing List" <tesla-2@emachine.com>

>     I just want to get the general feeling about big coils. I've been

> told a 6.5" is pretty big. But I've got a twin brother that's really

> pushing me to go bigger. So I finally gave in this week-end . I rigged up

> a winding jig and added a gear motor, at 36 RPM coiling goes quickly!  So

> here's what I got so far

>     Form-   10.75 Dia. x 33.5 winding length.

>     Wire size - 20 gauge .0335

>     approx 988 turns

>     approx 2782'  !!

>     27 ohms (measured)

>

>     Of course I followed Mike's directions for form. prep.

>

>     Question is, how many kw"s can I feed this baby?

>     If anyone is interested, it only took about 35 minutes to wind.

>

>         Kyle Break7@juno.com

Since kW = Ep*BPS  where Ep = primary energy and BPS = Break Rate,

you can go as high as you like in break rate but the limiting factor

is Ep. I reckon a coil that high is at risk of flashing over with

much more than 500 - 600kV output. That being so, you can keep on

increasing Ep as long as it is matched by an increase in Ctop.

Self capacitance of the bare coil is about 17pF. You can relate

output voltage to Ep by:   Vout = SQRT[ Csec/(2*Ep) ]   where Csec =

sum of all secondary capacitances (17pF with no topload) and Ep is

0.5*Cp*Vp^2  where Cp is the primary capacitance and Vp is the

voltage the gap fires at.  All this assumes no losses in transferring

energy from the primary capacitor to the secondary which is never the

case in practice.

Personally I'd be sorely tempted to make the coil considerably

longer so that higher output voltages may be obtained without

flashover problems.

Malcolm

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Date: Fri, 11 Sep 1998 04:11:37 -0700

From: "paul m" <fleischwulf@mailexcite.com>

Cc:

X-Sent-Mail: off

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

Subject: Re: Big Coils

X-Sender-Ip: 208.153.7.9

Organization: MailExcite  (http://www.mailexcite.com:80)

Sender: tesl-err@emachine.com

My 6.5" coil is currently producing 74 - 75" arcs and it still is not fine tuned. I'm running with a .026uF cap and I think for proper tunning I'm going to increase the capacitance to .036. I'm feeding this coil 4 Kw. 75" arcs are about all I can deal

with at the moment.

paul m

---

Einigheit und Wahrheit

X-Originating-IP: [199.217.140.22]

From: "Bill the arcstarter" <arcstarter@hotmail.com>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Pigs and hotdogs.

Date: Fri, 11 Sep 1998 05:44:52 PDT

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

Mad Coiler (Tristan) wrote:

>

>I think Bill the Arcstarter did something like simulating the effect of

>getting  shocked with a hotdog. I think it made a big mess?

Yes, we did.  The results were appalling.  You DON'T want ANY mistakes

with a pig.

We use a 14.4Kv 25KVA pig, inductively limited down to 2-7 KVA

depending on what we are doing!

See the pics on Fleischwulf's page:

http://www.geocities.com/CapeCanaveral/Hangar/9561/paularc.html

We are using x-ray cable with the shield intact, and grounded out.  I

think on one photo we are getting some Blumlein effects, as there was a

different arc characteristic (bright blue pops) when the conductors were

just barely close enough to arc...

...and remember - don't try this at home.  Go to a friend's house first!

:)

-Bill the arcstarter

Starting arcs in Cinci, OH

http://www.geocities.com/CapeCanaveral/Hangar/6160

X-Originating-IP: [199.217.140.22]

From: "Bill the arcstarter" <arcstarter@hotmail.com>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: This pig has wings!

Date: Fri, 11 Sep 1998 07:25:12 PDT

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

Coilers,

Well - I did it.  That single bushing 19900 volt 10KVA 125 lb pig is on

its way!  $94 + ~$50 s/h supposedly, from N. Carolina to Cinci, Ohio.

Thats a heck of a deal.

The 19900 part is somewhat high, but I think it can be dealt with.  The

problem is that if I set the gaps to fire at 14400, then the total

available current is a bit lower than if it were a true 14400 pig...

(In reality, I expect shipping to be more than $50...)

Once I actually receive it, I'll tell you where I got it from!

They might have more...

Wish me luck, and I hope I don't get too big of a BUZZ from it!

-Bill the arcstarter

Looking forward to starting even MORE arcs in Cinci, OH

http://www.geocities.com/CapeCanaveral/Hangar/6160

Date: Fri, 11 Sep 1998 08:38:21 -0700

From: Gene Waldo <gwaldo@1stnetusa.com>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Big Coils

Sender: tesl-err@emachine.com

Tyla A. Break wrote:

>

I rigged up

> a winding jig and added a gear motor, at 36 RPM coiling goes quickly!

>         If anyone is interested, it only took about 35 minutes to wind.

>

>                 Kyle Break7@juno.com

>

Kyle and all,

I'm planning to go to my friend and his machine shop w/several

screw-cutting lathes, to wind our 2ndary; the obvious extension of this

concept leads inevitably to the winding lg caps!

A PVC core of ~2" od, 2 rolls of Al foil, several rolls of poly film,

all jigged to roll-on smoothly, under a couple of idler/pinch rollers...

Containment/immersion leads to more PVC tube, endcaps and vacuum/min-oil

admission, attendant plumbing, valving etc.

Does anyone have have feed back on this sort of process? Especially

final foil/poly wrap/stabilization?

Of course, +/- leads and physical core assy/mounting lead to some

interesting eng. headaches....

Seems that this should lend itself to the production of fairly std

ser/par'l cap assys? This probably won't be economical for 1 or 2 offs,

but if there was a need for many, it might become feasable?

-----------------------------------------------

Gene Waldo -  gwaldo@1stnetusa.com  - "Igor in Residence"

---------------------------------------------------------------

Full-Name: Bert Wikkerink

Date: Fri, 11 Sep 1998 12:08:37 -0400

From: Bert Wikkerink <bert@mitra.com>

Organization: Mitra Imaging Inc.

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Big Coils

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Hi Gene,

I've often had the same thought and now you beat me to it and taken all the

credit. :-(

Bert Wikkerink

Gene Waldo wrote:

> Tyla A. Break wrote:

> >

>  I rigged up

> > a winding jig and added a gear motor, at 36 RPM coiling goes quickly!

> >         If anyone is interested, it only took about 35 minutes to wind.

> >

> >                 Kyle Break7@juno.com

> >

> Kyle and all,

> I'm planning to go to my friend and his machine shop w/several

> screw-cutting lathes, to wind our 2ndary; the obvious extension of this

> concept leads inevitably to the winding lg caps!

>

> A PVC core of ~2" od, 2 rolls of Al foil, several rolls of poly film,

> all jigged to roll-on smoothly, under a couple of idler/pinch rollers...

>

> Containment/immersion leads to more PVC tube, endcaps and vacuum/min-oil

> admission, attendant plumbing, valving etc.

>

> Does anyone have have feed back on this sort of process? Especially

> final foil/poly wrap/stabilization?

>

> Of course, +/- leads and physical core assy/mounting lead to some

> interesting eng. headaches....

>

> Seems that this should lend itself to the production of fairly std

> ser/par'l cap assys? This probably won't be economical for 1 or 2 offs,

> but if there was a need for many, it might become feasable?

>

> -----------------------------------------------

> Gene Waldo -  gwaldo@1stnetusa.com  - "Igor in Residence"

> ---------------------------------------------------------------

>  What's good for the planet is more OZONE! See the machinery form up!

>       NEW PAGE!   http://members.1stnetusa.com/danmurphy/waldo.htm

X-Mailer: IPAD 2.0

Date: Fri, 11 Sep 1998 15:19:57 -0500

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: This pig has wings!

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

At 07:25 AM 9/11/98 PDT, you wrote:

>Coilers,

> Well - I did it.  That single bushing 19900 volt 10KVA 125 lb pig is on

>its way!  $94 + ~$50 s/h supposedly, from N. Carolina to Cinci, Ohio.

>Thats a heck of a deal.

Very good deal indeed.

>The 19900 part is somewhat high, but I think it can be dealt with.  The

>problem is that if I set the gaps to fire at 14400, then the total

>available current is a bit lower than if it were a true 14400 pig...

Once you add in the inevitable loss from ballasting that

figure will drop off somewhat. You can use somewhat more

resistive ballast than you would normally use and still

have plenty of voltage.

>

>

>Once I actually receive it, I'll tell you where I got it from!  They

>might have more...

>

>Wish me luck, and I hope I don't get too big of a BUZZ from it!

>

>-Bill the arcstarter

>Looking forward to starting even MORE arcs in Cinci, OH

>http://www.geocities.com/CapeCanaveral/Hangar/6160

Here are a couple thoughts about untilizing your single bushing pig.

First you could drain the oil and dive inside and do a little rewiring.

Disconnect the other HV lead from the tank and route it out to another

insulator mounted on the lid. Then connect the transformer core to the tank.

I remember Richard Quick doing exactly this with a 7200 25kva single

bushing pig.

If you dont want to attempt the rewiring job then you can use it

as is with a slightly different coil configuration.

---------

|       | T1

---------

O

X1                                               O

G1               O  L2

O--------------------------->*  *<----         O

||O                  |                  |   L1   O

Line in> ||O                  |                  |    O   O

--------O||O               -------               |--->O   O

O||O               _______ C1                 O   O

--------O||O                  |                       O   O

||O                  |                       O   O

||O                  |                       O   O

O------------------|-----------------------O___O

|                         |

|                         |

|                       RF Gnd

RF Gnd

Note that the bottom of the primary and secondary are tied together

and grounded as well as the tank of the pig.

If you want to add a safety gap it would go across the main cap.

Ive seen this circuit used with single bushing potential transformers.

PLEASE NOTE.

This circuit must not be used with neons. It would be highly

dangerous as well as possibly destructive to the neon.

TTYL!

Mike Hammer

mhammer@misslink.net

Date: Fri, 11 Sep 1998 16:35:56 -0500

From: Bert Hickman <bert.hickman@aquila.com>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Organization: Stoneridge Engineering

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: This pig has wings!

Sender: tesl-err@emachine.com

Mike Hammer wrote:

>

> At 07:25 AM 9/11/98 PDT, you wrote:

> >Coilers,

> > Well - I did it.  That single bushing 19900 volt 10KVA 125 lb pig is on

> >its way!  $94 + ~$50 s/h supposedly, from N. Carolina to Cinci, Ohio.

> >Thats a heck of a deal.

>

> Very good deal indeed.

>

> >The 19900 part is somewhat high, but I think it can be dealt with.  The

> >problem is that if I set the gaps to fire at 14400, then the total

> >available current is a bit lower than if it were a true 14400 pig...

>

> Once you add in the inevitable loss from ballasting that

> figure will drop off somewhat. You can use somewhat more

> resistive ballast than you would normally use and still

> have plenty of voltage.

>

> >

> >

> >Once I actually receive it, I'll tell you where I got it from!  They

> >might have more...

> >

> >Wish me luck, and I hope I don't get too big of a BUZZ from it!

> >

> >-Bill the arcstarter

> >Looking forward to starting even MORE arcs in Cinci, OH

> >http://www.geocities.com/CapeCanaveral/Hangar/6160

>

> Here are a couple thoughts about untilizing your single bushing pig.

> First you could drain the oil and dive inside and do a little rewiring.

> Disconnect the other HV lead from the tank and route it out to another

> insulator mounted on the lid. Then connect the transformer core to the tank.

>

> I remember Richard Quick doing exactly this with a 7200 25kva single

> bushing pig.

>

> If you dont want to attempt the rewiring job then you can use it

> as is with a slightly different coil configuration.

>

>                                                       ---------

>                                                       |       | T1

>                                                       ---------

>                                                           O

>          X1                                               O

>                                          G1               O  L2

>            O--------------------------->*  *<----         O

>          ||O                  |                  |   L1   O

> Line in> ||O                  |                  |    O   O

> --------O||O               -------               |--->O   O

>         O||O               _______ C1                 O   O

> --------O||O                  |                       O   O

>          ||O                  |                       O   O

>          ||O                  |                       O   O

>            O------------------|-----------------------O___O

>                               |                         |

>                               |                         |

>                               |                       RF Gnd

>                             RF Gnd

>

> Note that the bottom of the primary and secondary are tied together

> and grounded as well as the tank of the pig.

>

> If you want to add a safety gap it would go across the main cap.

>

> Ive seen this circuit used with single bushing potential transformers.

>

> PLEASE NOTE.

> This circuit must not be used with neons. It would be highly

> dangerous as well as possibly destructive to the neon.

>

> TTYL!

>

> Mike Hammer

> mhammer@misslink.net

Mike and Bill,

Make sure the innermost primary turn and the grounded end of the pig's

secondary are tied together. Also, there's really no reason why the gap

and capacitor can't be reversed in your schematic so that the gap is

still across the transformer.

Bill, I'm interested in the weight (125 lbs??) of your single-bushing

pig - I've got a 19.9 KV 10 KVA beastie... but it stands 44" high and

weighs almost 400 pounds! Good luck!! Let us know if they have any more

- 125 lbs is a downright manageable size!

-- Bert --

X-Originating-IP: [208.16.150.20]

From: "Bill the arcstarter" <arcstarter@hotmail.com>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Big Coils

Date: Fri, 11 Sep 1998 21:01:43 PDT

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

It was written:

>Paul, type of transformer are you using?

I'm going to answer for Paul on this one since I'm here...

It's a 25KVA pig, about 3.5 ft tall, 14400 volts, 485 lbs.  Very large,

but thats all we could find at the time.

Paul feeds it a fine diet of electrons, slowly perculated through a

large variac and a string of MOTs in series with their secondaries

shorted out.

Oh, so far the new coil has caused the rotary gap motor to die (it was

ill anyway) and also let the smoke out of our nice MECI CORCOM 28 amp

line filter...

-Bill

X-Originating-IP: [208.16.150.20]

From: "Bill the arcstarter" <arcstarter@hotmail.com>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: This pig has wings!

Date: Fri, 11 Sep 1998 21:09:47 PDT

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

Hello Tom Stock!

>Hi Bill,

>

>I just wanted to remind you to be careful!!! I know you will :) I read

>all of your posts beginning to end. You are one of the tesla coil

>kings...but even a king needs to be reminded to be careful!

A king, eh?  I appreciate the nomination, but I renounce that throne.  I

couldn't have do this w/o the help of many others.  I suspect that

neither Paul nor myself would have done this w/o the help/goading of the

other guy, etc...

I like being careful.  That gives me the chance to play again some other

day! :)

I do appreciate the safety warnings.  The day I think I am being too

safe is the day my name will appear in the newspaper: "Also injured in

today's fluke 40-block disaster was Bill..." :)

>Have fun!! I'm still playing with neons myself...

We had lots of fun doing that, until we killed a few!

>I'm curious to see what you're going to do about caps with a 19kv

>transformer...

Yea, me too.  Actually the caps will (hopefully) never see that voltage

since I'll set the gaps such that they fire at a more sane level.

-Bill

>

>

>>

>> Coilers,

>>  Well - I did it.  That single bushing 19900 volt 10KVA 125 lb pig is on

>> its way!  $94 + ~$50 s/h supposedly, from N. Carolina to Cinci, Ohio.

>> Thats a heck of a deal.

>

X-Originating-IP: [208.16.150.20]

From: "Bill the arcstarter" <arcstarter@hotmail.com>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: This pig has wings!

Date: Fri, 11 Sep 1998 21:26:26 PDT

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

Hi Mike!

Some comments (writing a whole book here it seems...)

>Here are a couple thoughts about untilizing your single bushing pig.

>First you could drain the oil and dive inside and do a little rewiring.

>Disconnect the other HV lead from the tank and route it out to another

>insulator mounted on the lid. Then connect the transformer core to the tank.

>

>I remember Richard Quick doing exactly this with a 7200 25kva single

>bushing pig.

Yes, well do you think that there is sufficient insulation between the

innermost HV winding turns and the core?  As far as I can tell/(have

been told), these units are always run with the 120/240 side neutral

connected to the case.  So - unless I can determine that I am allowed to

NOT ground the case (proving that the core's insulation inside is

sufficient), only then would I consider re-wiring as you described!

>

>

>                                                      ---------

>                                                      |       | T1

>                                                      ---------

>                                                          O

>         X1                                               O

>                                         G1               O  L2

>           O--------------------------->*  *<----         O

>         ||O                  |                  |   L1   O

>Line in> ||O                  |                  |    O   O

>--------O||O               -------               |--->O   O

>        O||O               _______ C1                 O   O

>--------O||O                  |                       O   O

>         ||O                  |                       O   O

>         ||O                  |                       O   O

>           O------------------|-----------------------O___O

>                              |                         |

>                              |                         |

>                              |                       RF Gnd

>                            RF Gnd

>

>

>

Yes, I agree.  I had already thought about this, and this seems to be

the only reasonably safe arrangement!  The only thing missing is the

120-0-120 side neutral connection.  I would suggest tying it to the case

as well, just because that seems to be the convention.  Comments? I know

we aren't supposed to share the mains neutral with our safety grounds,

but in this case maybe it'd be ok??

My garage uses three power distribution wires which come over,

underground, from the house.  I have 60 amps at 220 available on two #6

direct-buriable copper conductors.  The neutral is a #8 or 10 bare

copper wire, which "comes up out of the ground".  I don't think any of

this wiring is running in any type of conduit, as on both the garage

side and the house side - the wires just go into a hole!  So my neutral

== a good earth ground.

Oops - I don't have a connection to the pig's 120 side neutral if I'm

running this off a single-phase variac... :(  Unless I just feed it

120V...

Hey - I think we have a hi-pot megger at work.  Perhaps I can use it to

test the insulation between the low voltage windings and the case!

Oh, we're doing another HV show in Monroe, OH tomorrow night!  I think

you all 'otta come down for the action!

More later.

-Bill

X-Originating-IP: [208.16.150.20]

From: "Bill the arcstarter" <arcstarter@hotmail.com>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: This pig has wings!

Date: Fri, 11 Sep 1998 21:49:31 PDT

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

Hi Bert.

>Make sure the innermost primary turn and the grounded end of the pig's

>secondary are tied together. Also, there's really no reason why the gap

>and capacitor can't be reversed in your schematic so that the gap is

>still across the transformer.

Yep.  I am usually antisocial and keep the cap across the output of the

transformer, as shown.  Not shown are the blocking inductors, damping

resistors etc...

>Bill, I'm interested in the weight (125 lbs??) of your single-bushing

>pig - I've got a 19.9 KV 10 KVA beastie... but it stands 44" high and

>weighs almost 400 pounds! Good luck!! Let us know if they have any more

>- 125 lbs is a downright manageable size!

Yes, the weight was sort of amazing and very attractive.  Downright

portable!  The guy on the phone assured me that was all it weighs!

If some 400+lb "thing" appears on my porch one afternoon, my wife's

gunna kill me before the pig gets it's chance! :)  Then one of you will get it!

Bert - I must admit - I'm going to make a big-*ss Jacob's ladder out of

this thing at least once.  How much FIRE will I get that way?  You must

have done that by now!  Yes - I will be ballasting it at all times.

For the strong-of-heart - take a lookie at my combination lawn

mower/pole-pig-picker-upper at:

http://www.geocities.com/CapeCanaveral/Hangar/6160/hoist1.jpg

More sickness is at:

http://www.geocities.com/CapeCanaveral/Hangar/6160/se-r/clutchpc.html

-Bill

X-Originating-IP: [208.16.150.20]

From: "Bill the arcstarter" <arcstarter@hotmail.com>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: One More Pig Question.

Date: Fri, 11 Sep 1998 21:53:49 PDT

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

Mike,

One last comment here:

You wrote:

> Disconnect the other HV lead from the tank and route it out to another

> insulator mounted on the lid. Then connect the transformer core to the tank.

I just re-read that last sentence.  Sounds like you don't think the

laminations are presently connected to the case.  I'll have to check

it out, but I suspect that they already are connected to the case,

simply for structural reasons.

Does anyone know differently?

-Bill

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: R.E. When Pigs Can Fly.

X-Mailer: IPAD 2.0

From: break7@juno.com (Tyla A. Break)

Date: Sat, 12 Sep 1998 00:54:31 EDT

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Hi, Bill, about shipping, I got my first Pig..., er. Sow  a

couple of months ago. It's a 5Kva but it's as big as a 37.5 . 16" Dia.

22" tall ,32" including the insulators. It weighs 371 lbs. truck freight

from Kansas to L.A. was $160 !!  Hope you get a better deal.

If it's not dangerous, it's not fun!    Kyle

break7@juno.com

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Potential Trans

X-Mailer: IPAD 2.0

From: break7@juno.com (Tyla A. Break)

Date: Sat, 12 Sep 1998 01:15:11 EDT

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

I went to APEX to try to get the potential trans I mentioned last

week. Aparently a local coiler or someone else has been trying too! After

I was flatly denied, I was told that some guy offered $150 for it.

The old man rents it as a background prop to the studios for $40 a

week.  Hard to compete with that!

If it's not dangerous it's not fun!  Kyle  break7@juno.com

X-Originating-IP: [208.16.150.20]

From: "Bill the arcstarter" <arcstarter@hotmail.com>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: R.E. When Pigs Can Fly.

Date: Fri, 11 Sep 1998 22:26:18 PDT

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

Hi Kyle, others.

>	Hi, Bill, about shipping, I got my first Pig..., er. Sow  a

>couple of months ago. It's a 5Kva but it's as big as a 37.5 . 16" Dia.

>22" tall ,32" including the insulators. It weighs 371 lbs. truck freight

>from Kansas to L.A. was $160 !!  Hope you get a better deal.

Yowch - thats rather huge and kinda low on the KVA/lb ratio... :)

I bet you can run that thing at least 50 hours per day w/o overheating it! :)

Dang - if I receive a pig that heavy - it's for sale...

It's kind of outrageous that everyone has to get these things from 1000+

miles away.  I bet these things are available locally for most with a

bit of digging.  I didn't try too hard to scare one up locally...

-Bill

Sender: tesl-err@emachine.com

Date: Sat, 12 Sep 1998 10:14:46 +0000

From: Tom Stock <tomstock@geocities.com>

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Big Coils

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Paul, type of transformer are you using?

Thanks

-tom

tomstock@geocities.com

paul m wrote:

>

> My 6.5" coil is currently producing 74 - 75" arcs and it still is not fine tuned.

>I'm running with a .026uF cap and I think for proper tunning I'm going to increase

>the capacitance to .036. I'm feeding this coil 4 Kw. 75" arcs are about all I

>can deal with at the moment.

>

> paul m

> ---

> Einigheit und Wahrheit

Sender: tesl-err@emachine.com

Date: Sat, 12 Sep 1998 10:18:51 +0000

From: Tom Stock <tomstock@geocities.com>

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: This pig has wings!

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Hi Bill,

I just wanted to remind you to be careful!!! I know you will :) I read

all of your posts beginning to end. You are one of the tesla coil

kings...but even a king needs to be reminded to be careful!

Have fun!! I'm still playing with neons myself...

I'm curious to see what you're going to do about caps with a 19kv

transformer...

cya

-tom

>

> Coilers,

>  Well - I did it.  That single bushing 19900 volt 10KVA 125 lb pig is on

> its way!  $94 + ~$50 s/h supposedly, from N. Carolina to Cinci, Ohio.

> Thats a heck of a deal.

Date: Sat, 12 Sep 1998 10:17:58 -0500

From: Bert Hickman <bert.hickman@aquila.com>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Organization: Stoneridge Engineering

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: This pig has wings!

Sender: tesl-err@emachine.com

Bill,

My responses are interspersed below...

Bill the arcstarter wrote:

>

> Hi Bert.

<SNIP>

> > Let us know if they have any more

> >- 125 lbs is a downright manageable size!

>

> Yes, the weight was sort of amazing and very attractive.  Downright

> portable!  The guy on the phone assured me that was all it weighs!

>

> If some 400+lb "thing" appears on my porch one afternoon, my wife's

> gunna kill me before the pig gets it's chance! :)  Then one of you will

> get it!

Hmm.. I can see it in your last will and testament: "and all my worldly

good go to my wife, except for my 400 pound trained pole-pig,

effectionately named "Sparky", which will be given to Paul for his

research projects for the betterment of all mankind". If I prematurely

kick the bucket, I've told my wife to give my coiling stuff to some of

the other local coilers - would hate to see it hit the dumpster! Heck,

that's where half of the stuff came from in the first place! It would

make for one hell of an interesting garage sale though!

>

> Bert - I must admit - I'm going to make a big-*ss Jacob's ladder out of

> this thing at least once.  How much FIRE will I get that way?  You must

> have done that by now!  Yes - I will be ballasting it at all times.

You should have no problem reliably hitting 9-10" at the top of the

ladder. More is tougher since the turbulence tends to make the arc

really wander. Use a welder as a ballast - it's MUCH better behaved than

using a resistive ballast (which really looks wimpy!). The first time

the arc goes from bottom to top, it goes right up the ladder to the top.

The following times it seems to stall and the arc move laterally

sometimes, from the left-over turbulence(?). I've got my ladder

contained in a box painted black with a plxiglas front, I mainly use it

for HV demos and on Halloween to entertain the Trick-or-Treaters. A

ladder in free air might behave a bit better (as long as there was no

wind). At higher power levels, the arc gets noticeably fatter, and puts

out lot's more heat.

BTW, for some real snarl, connect a 500-2000 pF 40 KV doorknob cap

across the ladder terminals. This also makes initial ignition at the

bottom easier (and much snappier!)... and it sounds downright nasty! :^]

>

> For the strong-of-heart - take a lookie at my combination lawn

> mower/pole-pig-picker-upper at:

> http://www.geocities.com/CapeCanaveral/Hangar/6160/hoist1.jpg

>

> More sickness is at:

> http://www.geocities.com/CapeCanaveral/Hangar/6160/se-r/clutchpc.html

>

> -Bill

I like the mower/pole-pig-picker-upper - but the tank-o-lantern is even

better! BTW, you guys ever get the Q-shrinker back up and running??

Mine's temporarily down - one of the last runs I did at 8500 Joules

succeeded in fracturing one side of the 1/2" Lexan containment shield -

looks like I'll need to go to 1". =<:^O

Safe coilin' (and whatever-in') to you!

-- Bert --

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: 25Kva Ladder

X-Mailer: IPAD 2.0

From: break7@juno.com (Tyla A. Break)

Date: Sat, 12 Sep 1998 13:53:24 EDT

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

I've been running my 5Kva pig at 100 -125 amps  for short demos .

I'm getting 20 -24" arcs!  Some say" where's  the spark? " , " There"s

only fire! "  Using consumable copper electrodes gives nice green fire .

It's not fun if it's not dangerous!   Kyle

break7@juno.com

Date: Sat, 12 Sep 1998 14:51:08 -0600

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Daniel Lockhart <molly@cmn.net>

Subject: Re: This pig has wings!

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

Bert wrote:

>I like the mower/pole-pig-picker-upper - but the tank-o-lantern is even

>better! BTW, you guys ever get the Q-shrinker back up and running??

>Mine's temporarily down - one of the last runs I did at 8500 Joules

>succeeded in fracturing one side of the 1/2" Lexan containment shield -

>looks like I'll need to go to 1". =<:^O

>

>Safe coilin' (and whatever-in') to you!

What kind of capacitor are you using, Bert?

Daniel Lockhart

Date: Sat, 12 Sep 1998 15:04:36 -0600

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Daniel Lockhart <molly@cmn.net>

Subject: Liquid Capacitors

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

Yo everyone,

I was looking up the "k" values for different plastic types in the

"math.txt" file I downloaded a while ago. I noticed that some liquids were

listed as dielectrics. Their "k" value was quite high, 22 for liquid

ammonia, 25.8 for ethyl alcohol (at 20 C), 33.1 for methyl alcohol and 80

for distilled water.

Can these liquids be used for dielectrics? No puncture voltage thickness

was given. Any idea on what it would be? Are liquids very efficient? What

are the disadvantages?

Secondly, does anybody know where I could get a more complete and/or a more

authoritative listing for "k" values and puncture voltages for various

materials?

Thanks,

Daniel Lockhart

Date: Sat, 12 Sep 1998 16:28:34 -0700

From: Gene Waldo <gwaldo@1stnetusa.com>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: This pig has wings!

Sender: tesl-err@emachine.com

Bert Hickman wrote:

>

> Bill,

>

> My responses are interspersed below...

>

> Bill the arcstarter wrote:

> >

> >

> > Bert - I must admit - I'm going to make a big-*ss Jacob's ladder out of

> > this thing at least once.  How much FIRE will I get that way?  You must

> > have done that by now!  Yes - I will be ballasting it at all times.

>

> You should have no problem reliably hitting 9-10" at the top of the

> ladder. More is tougher since the turbulence tends to make the arc

> really wander.

> Safe coilin' (and whatever-in') to you!

>

> -- Bert --

Bill,

Bite the bullet! Steal or connive some bucks from the old lady--buy a

10" dia x 5'(or whatever) plex tube! Apply some phenolic blocks at the

top and bottom: cagey machining will allow a taut wire ladder! I guess

you know about trigger gap thingo at bottom-- A Cap across the ladder

will mostly last real quick!  but the effect is fun! Especially if you

use a free-topped ladder; an oscillation may set up, causing the upper

ends to swing to and fro- in and out of conductivity- GRRR   GRRRR

GRRR!!!!! Hot, loud tight blue muthas!

A real ozonator!

This is PLAYLAND! Pantywaists need not apply!

----------------------

Gene Waldo -  gwaldo@1stnetusa.com  - "Igor in Residence"

---------------------------------------------------------------

What's good for the planet is more OZONE! See the machinery form up!

NEW PAGE!   http://members.1stnetusa.com/danmurphy/waldo.htm

Date: Sat, 12 Sep 1998 16:38:38 -0700

From: Gene Waldo <gwaldo@1stnetusa.com>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Liquid Capacitors

Sender: tesl-err@emachine.com

Daniel Lockhart wrote:

>

> Yo everyone,

>

> I was looking up the "k" values for different plastic types in the

> "math.txt" file I downloaded a while ago. I noticed that some liquids were

> listed as dielectrics. Their "k" value was quite high, 22 for liquid

> ammonia, 25.8 for ethyl alcohol (at 20 C), 33.1 for methyl alcohol and 80

> for distilled water.

>

> Secondly, does anybody know where I could get a more complete and/or a more

> authoritative listing for "k" values and puncture voltages for various

> materials?

>

> Thanks,

>

> Daniel Lockhart

Dan,

Yes, his name is MH.  I have a whole arch file (somewhere) on the topic.

Someday I'll really have to sort my sockdrawer T-2 printouts!

Ammonia Caps sound like a real eng CHALLENGE! Alchy sounds like a hazard

of some sort! Dist H2O might work, but you might have to use Ag or Au

plates to keep from SOME kind of salts....

---------------------------------------

Gene Waldo -  gwaldo@1stnetusa.com  - "Igor in Residence"

---------------------------------------------------------------

What's good for the planet is more OZONE! See the machinery form up!

NEW PAGE!   http://members.1stnetusa.com/danmurphy/waldo.htm

Date: Sat, 12 Sep 1998 16:44:13 -0700

From: Gene Waldo <gwaldo@1stnetusa.com>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Potential Trans

Sender: tesl-err@emachine.com

Tyla A. Break wrote:

>

>         I went to APEX to try to get

>         If it's not dangerous it's not fun!  Kyle  break7@juno.com

>

Kyle, next time you head for APEX, give a holler--- I'm only 10 min from

there.

Always looking for an excuse to get over there!

I'll be wearing the yellow daffodil    ~~<[:-)

--------------------------------

Gene Waldo -  gwaldo@1stnetusa.com  - "Igor in Residence"

---------------------------------------------------------------

What's good for the planet is more OZONE! See the machinery form up!

NEW PAGE!   http://members.1stnetusa.com/danmurphy/waldo.htm

Date: Sat, 12 Sep 1998 16:49:09 -0700

From: Gene Waldo <gwaldo@1stnetusa.com>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: This pig has wings!

Sender: tesl-err@emachine.com

Bert Hickman wrote:

>

> BTW, for some real snarl, connect a 500-2000 pF 40 KV doorknob cap

> across the ladder terminals. This also makes initial ignition at the

> bottom easier (and much snappier!)... and it sounds downright nasty! :^]

> -- Bert --

OK-- I give! Does someone have a pic or other detailed description of a

door knob? Might be barrels of 'em I've walked right past!

----------------------------------------------------

Gene Waldo -  gwaldo@1stnetusa.com  - "Igor in Residence"

---------------------------------------------------------------

What's good for the planet is more OZONE! See the machinery form up!

NEW PAGE!   http://members.1stnetusa.com/danmurphy/waldo.htm

Date: Sat, 12 Sep 1998 22:47:55 -0500

From: Bert Hickman <bert.hickman@aquila.com>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Organization: Stoneridge Engineering

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: This pig has wings!

Sender: tesl-err@emachine.com

Gene Waldo wrote:

>

> Bert Hickman wrote:

> >

>

> > BTW, for some real snarl, connect a 500-2000 pF 40 KV doorknob cap

> > across the ladder terminals. This also makes initial ignition at the

> > bottom easier (and much snappier!)... and it sounds downright nasty! :^]

>

> > -- Bert --

> OK-- I give! Does someone have a pic or other detailed description of a

> door knob? Might be barrels of 'em I've walked right past!

> ----------------------------------------------------

> Gene Waldo -  gwaldo@1stnetusa.com  - "Igor in Residence"

Gene,

A doorknob cap is a ceramic capacitor that's roughly the size of a

doorknob!. Basically these are high voltage ceramic capacitors,

typically rated at 20 - 40 KV, and typically 500 - 4000 pF. They are

cylindrical in shape, and typically have threaded (6-32 typically) metal

connections on each end. The length is a function of voltage - a 20 kV

one is typically 1" long, while a 40 kV one may be 1.5" long. Diameter

will be a function of the capacitance.

A 500 pF cap may be less than 1" in diameter, while a 4000 pF one may be

~2.5". The outside body is molded or cast plastic, and can be red, green,

tan, or even white in color. They used to be common in television sets

many years ago, but no longer. While most are used for DC filtering

only, there are some that are made for pulse duty - these tend to be

used with pulsed laser systems, and are usually white in color.

Hope this helps!

-- Bert --

From: wwl@netcomuk.co.uk (Mike Harrison)

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: 25Kva Ladder

Date: Sun, 13 Sep 1998 08:44:42 GMT

Organization: White Wing Logic

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

On Sat, 12 Sep 1998 13:53:24 EDT, you wrote:

>	I've been running my 5Kva pig at 100 -125 amps  for short demos .

>I'm getting 20 -24" arcs!  Some say" where's  the spark? " , " There"s

>only fire! "  Using consumable copper electrodes gives nice green fire .

>

Ever thought about joining up sections of different metals so the

colour changes as the arc rises?

____                                                           ____

_/ L_/  Mike Harrison / White Wing Logic / wwl@netcomuk.co.uk  _/ L_/

_/ W_/  Hardware & Software design / PCB Design / Consultancy  _/ W_/

/_W_/  Industrial / Computer Peripherals / Hazardous Area      /_W_/

From: "Thomas McGahee" <tom_mcgahee@sigmais.com>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Dielectric Table

Date: Sun, 13 Sep 1998 10:03:42 -0400

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Dear Tesla-2 Members,

Gene Waldo asked about a table of dielectric constants and

such. Here is a re-post of a FAQ I put together on the

topic of oil. At the end there is a table I filched from

one of Richard Quick's archived posts. I do not know the source

of the original table beyond that.

It is useful sometimes to re-post some of these FAQs, as new

members may not have a copy. So, here is my attempt at

recycling!

*****

Fellow Coilers,

Certain questions tend to come up again and again on this list, and

the topic of oil for capacitors is one of them. In the interest of

gathering and presenting as much information as I could find on this

topic, I present this post in the hope that it will be found useful

by some. It includes parts of posts by Richard Quick, Mike Hammer,

Ralph Down, and many other coilers on the Tesla List (including

myself). I have not marked down who is responsible for what here,

simply because my clip file of data from which this was assembled in

many cases has only the data, and not the name of the person who

posted it. I hope no one is offended by having their name

inadvertantly left out.

If you have additional comments to add to this modest compendium,

please e-mail me at tom_mcgahee@sigmais.com and I will try to keep

this compendium as up-to-date as I can. If I get any significant

e-mails I will repost the updated version when appropriate.

Please note that large sections of this post were taken from posts

originally sent to the Tesla List by many different people. So if

some of the paragraphs sound awfully familiar, it may just be because

it is based on sometheing *you* sent to the Tesla List.

As I have said before, the really good stuff gets recycled again and

again.

Beginning of Discussion on Oil

*******

Coilers use oil to reduce corona and increase High Voltage insulation

in such applications as Capacitors and Transformers.

Mineral oil is a rather generic term given to a whole class

of light oils. There are literally dozens of types.

Veterinary grade mineral oil can be had rather cheaply

in agricultural areas. Sold in 2 and 5 gallon pails.

Pure USP or veterinary grade mineral oil will work fine. The price

varies based on purity and quantity. You can get USP mineral oil at

any pharmacy for less than $2 a pint. While that seems reasonable, it

is much better to buy whatever you use by the gallon or five gallon

container if you want to get a really reasonable price.

Some coilers have even used mineral oil that had a fragrance added.

In *general* try to stay away from all forms of additives, because

you just don't know what properties they bring when used for High

Voltage insulation.

Buy the Transformer Oils from oil distributors. Just tell them you

need a good transformer oil that is relatively inexpensive. Many

prefer transformer oil over mineral oil because it is specifically

made for insulating high voltage, has ALL water rigorously removed,

and is a very THIN oil, so it penetrates MUCH better than mineral

oil.

Whenever possible, go for the Transformer Oil rather than the generic

Mineral Oil. The thinner the oil the better, as thick oil makes

bubble removal harder.

Texaco, Mobil, Conoco, and many others also make transformer oil, but

I don't have any specific info on these products in my files (yet).

Some coilers have used refrigeration oil and silicone oils. Ralph

Down used SUNISO refrigeration oil (bought from local refrigeration

repair shop for $30 for 4 litres.) He was pleasantly surprised to

find that the high dielectric constant of this oil actually DOUBLED

the capacitance value of his homemade capacitor.

Coilers have used drugstore grade mineral oil and animal grade

mineral oil. Animal grade oil is sold as livestock laxative.

Those in agricultural areas can find it easily enough, but

city types might have a problem.

For what it's worth, I built *my* capacitors with Exxon Univolt 61. 5

Gallons cost me $34.50. That turned out to be a most reasonable price

for an EXCELLENT true transformer oil. The dielectric constant of

this stuff was almost exactly the same as the polypropylene that I

was using. By the way, when I did my yellow page searches, I was

quoted prices ranging from $34.50 to $48.00 for the exact same

product. So check out multiple sources before buying!

As to using stuff like motor oil, well, it has been done. Read the

label. Go for the stuff that has the LEAST amount of additives. Most

motor oils are too thick. Go for the lightest weight oil you can get.

Remember, you want the stuff to infiltrate every nook and cranny of

your capacitor. Something the consistency of molasses isn't going to

do you much real good!

In general, stay away from food oils. They tend to go rancid and have

too much water content.

But never be afraid to experiment on a *small* scale. Make up a small

flat plate capacitor consisting of just two plates of relatively

small size and EXPERIMENT. Determine the breakdown voltage yourself,

if you have to. The bottom line is not whether all the bigwigs would

use it, but does it work for *you*? And if you find a good reasonably

priced source for Castor Oil, please let the rest of us know about

it! (Castor Oil has a nice high dielectric constant, more than double

the value of transformer/mineral oil).

All of the following oils have been used successfully by coilers on

the Tesla List.

Mineral Oils:

Shell MVI-65

Tulco Snaplube 100

USP Mineral Oil (Laxative) (available at any pharmacy).

Veterinary Grade Mineral Oil (Laxative)

Commercial transformer oils are excellent! Examples would be:

Shell Diala-X or Shell Diala-AX

Esso VoltEsso 35

Exxon Univolt N60

Exxon Univolt N61

Exxon Univolt 33

r-Temp (Paraffinic - Best for capacitors, but more expensive)

Refrigeration Oil:

SUNISO

There are many other brands of silicone oil

Motor Oils:

Some coilers have used LIGHT GRADES of Motor Oil successfully

Use ones with least number of additives.

Note: Some reports from those using Motor Oil indicate

that over time there is a breakdown of the oil and it

discolors. Not recommended.

Those of you who are scroungers at heart might want to try your local

power utility. Some people have had success getting a couple of

gallons of transformer oil from their local power company cheaply or

even free.

High Voltage transformer oil is used in power distribution

transformers and also in x-ray machines.

When looking to locate a source for *any* item, the FIRST thing is to

use the yellow pages of your phone book. In this case you would look

under OIL. Find the advertisements that specifically mention

transformer oil. Try these first. Call at a time of day when the

person working there is not going to be too busy. That way they will

be more inclined to spend some time talking to you on the phone.

Some people on the other end of the phone will be very knowledgeable

and helpful. Other persons won't have much of an idea of exactly what

you want unless you give them specific information such as "do you

carry Exxon Univolt 61 transformer oil?" Remember, sometimes the

person at the other end of the phone knows more than *you* about the

topic you are discussing (which at the moment is transformer oil, not

Tesla Coils). If it seems like the person is pretty knowledgeable,

then you might divulge the purpose for which you want to use the

transformer oil... you might be surprised and find that he can give

you something even better than what you were asking for.

Besides checking out the Oil Suppliers and Distributers, you might

also discretely try to find out what KIND of customers the Oil

Suppliers are supplying all this transformer oil to. You can

sometimes get one of these customers to sell you something less than

a full five gallon container at their cost (which is usually going to

be LESS than what the supplier quoted *you*, because many customers

buy the stuff 100 gallons or more at a time.)

It is often quite useful to tell people what you are going to use the

oil for. This is especially true if you go some place in person. Take

along some pictures of a Tesla Coil in operation. If you don't have

any good pictures of one of your own coils, download one of somebody

else's magnificent coils and use *that*. Depending on a great many

factors, you can often get the person you are talking to to either

sell you the stuff at cost, or give you a small amount for free.

Below is a copy of a table produced by Richard Quick some time ago.

It includes the dielectric constants and breakdown voltages

per mil of a number of substances, including several kinds of oil.

I could have cut this table down to just include oils, but the

table will have greater utility for all of us if it is kept intact.

=====================================================================

Dielectric strength   Puncture Voltage

Material                       K           per Mil(0.001 inch)

---------------------------------------------------------------------

Miscellaneous

-------------

Vacuum                            1.0

Paper, bond                       3.0            200

Paper, Royal Grey                 3.0            200

*   Paper, telephone, treated         2.5 - 4        200 - 250

Paper, Parafin Coated             2 -3.5

Paper, Kraft                      2.2

Oil, Castor                       4.67

Oil, Mineral, Squibb              2.7            200

Oil, Mineral                      2.2

*   Oil, Transformer                  2.1 - 2.5      75

Rubber                            3.0

*   Rubber, Hard                      3.0            160 - 500

1   Rubber, Vulcanized                3.2 - 3.9

Fibre                             5.0 - 7.5      150 - 180

Fibre, Red                        5.0

Mica                              4.5 - 8.0      3800 - 5600

Mica, Ruby                        5.4            3800 - 5600

Quartz                            3.8 - 5.0      1000

*   Quartz (Fused)                    4.2            150 - 200

Shellac                           2.5 - 4.0      200 - 400

Spar Varnish                      4.8 - 5.5

Steatite, low loss                5.8            150 - 315

Steatites (Magnesium silicate,etc)5.5 - 7.5      200 - 300

2   Cambric (Varnished)               4.0

Alsimag 196                       5.7

3   Gutta Percha                      4.0

4   Amber                             3.0 - 7.0

Resin                             2.48 - 2.57

Enamel                            5.1            450

Mycalex                           7.4            250

Silicone RTV                      3.6            550

Wood                              2.0 - 5.2

*   Wax (Parafin)                     2.1 - 2.5      250 - 450

Beeswax                           2.9 - 3.0

5   Slate                             7.0

Barium titanate(25 C)             1200

Bariam titanate                   6000

Titanium dioxide                  125

Cellulose acetate                 3.3 - 3.9      250 - 600

6   Casein, Moulded                   6.4

A   Polytetraflourethylene            2.0

Aluminum oxide                    8.7

Tantalum pentoxide                22

Glass

-----

Glass                             4.8 - 10       300

Plate Glass                       6.8 - 8.4

Pyrex Glass                       4.8 - 10       335

Window Glass                      7.6 - 7.8      200 - 250

Ceramics

--------

Cordierite ceramics               5.0 - 5.5      100

Magnesium titanate ceramic        12 - 18        150

Porcelain                         5.1 - 7.5      40 - 280

Titanium dioxide ceramic          70 - 90        100

Titanium-zirconium dioxide ceramic40 - 60        150

Plastics

--------

Bakelite                          4.4 - 5.8      300

Bakelite, Mica filled             4.7            325 - 375

Epoxy Circuit Board               5.2            700

7   Formica                           4.6 - 4.9      450

Nylon (lowest values of 3 types)  3.2            407

PVC (rigid type)                  2.95           725

Plexiglass                        2.8            450 - 990

Polyethylene                      2.2 - 2.3      450 - 1200

Polycarbonate (Lexan)             2.96           400

Polyethylene Terphthalate (Mylar) 3.0 - 3.1      7500

Polystyrene                       2.5 - 2.6      500 - 700

Teflon                            2.1            1000 - 2000

Gases

--------

Air (dry air at 1 atm)            1.0006         30 - 70

*   Air (20 atm, 19 deg. C)           1.0108         500

*   Carbon dioxide ( 1 atm, 0 deg. C) 1.000985       36

Carbon dioxide (20 atm, 15 deg. C)1.020

*   Hydrogen (1 atm,0 deg.C)          1.000264       26.1

Liquids

--------

Ammonia (liquid)                  22

Benzene                           2.28

8   Carbon tetrachloride              2.24

9   Chlorinated diphenyl              6.5

Ethyl Alcohol (O C)               28.4

Ethyl alchohol (20 deg.C)         25.8

Methyl alchohol                   33.1

Water (distilled)                 80 - 81

LEGEND:

* = Measured in Kilovolts per centimeter. All others are Volts

per mil (.001 inch) unless otherwise stated.

1 = Vulcanized means it has been melted, or heated in some

way.

2 = Cambric is a finely woven white linen or cotton fabric.

3 = Gutta Percha is a rubbery substance made from the latex of

tropical trees and is used in insulation, waterproofing,

and dentist use it in thin sheets sometimes when working

on teeth.

4 = Amber is a hard, translucent, yellow, orange, or brownish yellow

fossil resin, used in making ornamental objects like jewelry.

5 = Slate is a fine grained metamorphic rock that splits into

thin smooth faced layers. Black Boards for writting are

made of this. Also used as roofing material in some areas.

6 = Casein is a white, tasteless, oderless milk and cheese protein

used in the manufacture of plastics, glues, paints, and food. The

word 'moulded' means it has been shaped by a mold.

7 = Formica is a trademark for any of various high-pressure laminated

plastic sheets of melamine and phenolic materials used for

chemical and heat-resistant surfaces.

8 = Carbon tetrachloride is a very toxic substance. It has also been

shown to cause cancer in lab animals. It is banned in most labs.

It is a liquid that was used as a strong solvent.

9 = Chlorinated diphenyl is a liquid dielectric that is used to

impregnate Kraft paper in small A.C. capacitors. This is a PCB

and may cause cancer, handle carefully.

A = Polytetraflourethylene films retain good properties even at 200

degrees Celsius (200 C).

atm = atmospheres (pressure of air at sea level is 1 atm).

deg.C = degrees Celsius.

Warning - Some liquids and gases listed may be explosive under the

right conditions. Many solids can catch fire and burn. Use

CAUTION and GOOD COMMON SENSE.

Note: Some books gave very different values for each substance so I

have given you the highest and lowest values reported. The

values will depend on the purity of the substance your using.

If you know your substance is very pure then use the higher

value reported. If you know your substance is contaminated or

of poor quality then use the lower values reported. If you have

unknown purity then use the average of the values given.

**** End of Richard Quick's Table

If you have anything to add to this compendium, please e-mail it to:

tom_mcgahee@sigmais.com

Hope this helps someone out there. File it away for later.

Fr. Tom McGahee

X-Originating-IP: [195.121.16.108]

From: "Sam Barros" <sambarros@hotmail.com>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Ozone and nitrates

Date: Sun, 13 Sep 1998 07:05:38 PDT

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

> Bite the bullet! Steal or connive some bucks from the old lady--buy a

>10" dia x 5'(or whatever) plex tube! Apply some phenolic blocks at the

>top and bottom: cagey machining will allow a taut wire ladder! I guess

>you know about trigger gap thingo at bottom-- A Cap across the ladder

>will mostly last real quick!  but the effect is fun! Especially if you

>use a free-topped ladder; an oscillation may set up, causing the upper

>ends to swing to and fro- in and out of conductivity- GRRR   GRRRR

>GRRR!!!!! Hot, loud tight blue muthas!

>A real ozonator!

Really? From my experiences with high voltage I found that ozone is

normally formed when corona is present (it ionizes oxygen in the air) .

High power discharges such as cap discharges and hot electric arcs

normally combine oxygen in the air with nitrogen (just like lightning.

That is the reason why natural rain water has a slightly acidic PH: It

contains nitrates. Those go to the soil and are processed into

fertilizers by some plants) to form Nitrogen oxides. It smells quite

differently from ozone, but cannot be smelled at very low

concentrations. It's a toxic gas as well...

Am I right?

Sam.

Date: Sun, 13 Sep 1998 08:22:41 -0700

From: Gene Waldo <gwaldo@1stnetusa.com>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Ozone and nitrates

Sender: tesl-err@emachine.com

Sam Barros wrote:

>

Hot, loud tight blue muthas!

> >A real ozonator!

>

>  Really? From my experiences with high voltage I found that ozone is

> normally formed when corona is present (it ionizes oxygen in the air) .

> High power discharges such as cap discharges and hot electric arcs

> normally combine oxygen in the air with nitrogen (just like lightning.

> That is the reason why natural rain water has a slightly acidic PH: It

> contains nitrates. Those go to the soil and are processed into

> fertilizers by some plants) to form Nitrogen oxides. It smells quite

> differently from ozone, but cannot be smelled at very low

> concentrations. It's a toxic gas as well...

>

>  Am I right?

>

>  Sam.

Sam, you probably are!  The hot yellow (more a function of I than V?)

flame-like discharges which are best for ladders, rise well in air; and

are probably processing more N2.  The Blue cracklers (more a function of

V than I?) don't rise well and produce more ozone, I think.

Our 'ladder' didn't behave at all with a cap across it; causing the

free-standing poles to attract at the tops, to discharge, release, and

begin the physical oscillations I mentioned.  Realize that this was a

bare bones TC: NPS, pri, 2ndary! With no calculated design parameters or

refinements, such as safety gaps, Static gaps, RFGnd, chokes, tank caps,

etc. Just a kid guessing and messing about with no literature or

guidance.  As I recall, the shorted 2ndary would produce ~10" sparks.

Too bad, because I had been gifted with neat components: 12Kv/60,

12Kv/?mfd; all of which went to smoke heaven, eventually!

Then, this year, another NPS fell into my lap...  I wasn't even on the

net....  I said to Dan, "Show me the Net."  Then I said, "Look up TESLA."

Now we are mired in detail and neat commentary....

---------------------------------------------

Gene Waldo -  gwaldo@1stnetusa.com  - "Igor in Residence"

---------------------------------------------------------------

X-Mailer: IPAD 2.0

Date: Sun, 13 Sep 1998 17:42:08 +0100

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: bob golding <yubba@clara.net>

Subject: coupling

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Hi All,

Have just been playing around with the coupling of my coil.

9 Kv 60mA neon,

Conical 30 degree primary 9 turns 3/8" Cu,

home made flat plate cap 15 nF,

RQ gap 0.125" gap, vacuum quenced

4 1/2" secondary 21" winding length, About 980 turns 23 AWG,

19" x 4" toroid.

I am getting 20" sparks to a grounded rod with the primary tapped at 8 1/2

turns. I started with the base of the secondary windings about 2" above the

first turn of the primary. This gave 20" sparks. I then moved the secondary

down to be in line with the first turn of the primary. This does not seem

to have made any differance to the spark length. is this because the coil

is too far out of tune? I did notice that safety gap activity is less with

the secondary lower. Am I right in thinking that the coupling is at the

minimum when the the secondary is in line with the bottom turn of the

primary, therefore there is no point in adjusting the coupling below this

point?, and  that coupling is increased as the secondary is raised. How

much differance does the distance between the the primary and the secondary

make?. At the moment I have about 1" I would find it hard to tighten this

up due to the design of my coil.

many thanks

bob golding lightning without rain

bob yubba.clara.net

X-Mailer: IPAD 2.0

Date: Sun, 13 Sep 1998 12:20:43 -0500

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: coupling

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

At 05:42 PM 9/13/98 +0100, you wrote:

>Hi All,

>

>	Have just been playing around with the coupling of my coil.

>

>9 Kv 60mA neon,

>Conical 30 degree primary 9 turns 3/8" Cu,

>home made flat plate cap 15 nF,

>RQ gap 0.125" gap, vacuum quenced

>4 1/2" secondary 21" winding length, About 980 turns 23 AWG,

>19" x 4" toroid.

>

>I am getting 20" sparks to a grounded rod with the primary tapped at 8 1/2

>turns. I started with the base of the secondary windings about 2" above the

>first turn of the primary. This gave 20" sparks. I then moved the secondary

>down to be in line with the first turn of the primary. This does not seem

>to have made any differance to the spark length. is this because the coil

>is too far out of tune? I did notice that safety gap activity is less with

>the secondary lower. Am I right in thinking that the coupling is at the

>minimum when the the secondary is in line with the bottom turn of the

>primary, therefore there is no point in adjusting the coupling below this

>point?, and  that coupling is increased as the secondary is raised. How

>much differance does the distance between the the primary and the secondary

>make?. At the moment I have about 1" I would find it hard to tighten this

>up due to the design of my coil.

>

>many thanks

>

>bob golding lightning without rain

>

>bob yubba.clara.net

>

Bob,

You have it backwards. As you move the secondary

closer to the primary this increases coupling.

When you have the bottom turn of the secondary

in line with the plane of the primary turns

thats the point of maximum coupling.

Your gap at a total of .125 inch is rather

narrow even for 9kv. You may want to open that up

somewhat. Ive used as much as .200 inch at 9kv.

Do it in small increments and see how your coil

behaves. If it begins running erratically or the

gap sputters you may have it too wide.

TTYL!

Mike Hammer

mhammer@misslink.net

X-Originating-IP: [195.121.16.79]

From: "Sam Barros" <sambarros@hotmail.com>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Ozone and nitrates

Date: Sun, 13 Sep 1998 10:32:58 PDT

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

>>  Sam.

>Sam, you probably are!  The hot yellow (more a function of I than V?)

>flame-like discharges which are best for ladders, rise well in air; and

>are probably processing more N2.  The Blue cracklers (more a function

of

>V than I?) don't rise well and produce more ozone, I think.

>

>Our 'ladder' didn't behave at all with a cap across it; causing the

>free-standing poles to attract at the tops, to discharge, release, and

>begin the physical oscillations I mentioned.  Realize that this was a

>bare bones TC: NPS, pri, 2ndary! With no calculated design parameters or

>refinements, such as safety gaps, Static gaps, RFGnd, chokes, tank caps,

>etc. Just a kid guessing and messing about with no literature or

>guidance.  As I recall, the shorted 2ndary would produce ~10" sparks.

>

>Too bad, because I had been gifted with neat components: 12Kv/60,

>12Kv/?mfd; all of which went to smoke heaven, eventually!

>Then, this year, another NPS fell into my lap...  I wasn't even on the

>net....  I said to Dan, "Show me the Net."  Then I said, "Look up TESLA."

>Now we are mired in detail and neat commentary....

Thanks!

When I run my TV cascade (25KV) or my ignition coil driver (60KV), I

get quite a lot of corona (those faint purple little streamers). And I

can really smell the ozone. It doesn't bother me or anything (Actually,

I quite like the smell), but standing close to the output leads means

inhaling considerable amounts of ozone (not enough to fill a room though...)

But with my capacitor system (the one with the 2 microwave oven

capacitors), a few minutes of arcing and I have to get out of the room

(if I do it indoors). The smell of nitrates is really strong. Again it

doesn't bother me, but it is toxic, so I would rather not smell it. Same

goes for powerful arcs (such as those from my 2300V 0.5A MOT), but there

the amount of nitrates produced is much smaller.

I TC to ground sparks would generate nitrates instead of ozone, but the

amount would be VERY small. Possibly not enough to be smelled.

One funny though: Nitrous Oxide (N2O) is a well-known anaesthetic. It

is also known as "Laughing Gas". I wonder if running a large Jacobs

ladder in a closed room would give you a buzz... I wouldn't try it

though: NO is likely to form, and it is very toxic...

Sam.

P.S. I made a Jacob's ladder using my MOT. I moved the bottom

electrodes 1cm apart (the mot is rated 2.3KV, so it could never spark

that far), and placed a burning candle under the gap. The candle starts

the arc and provides it with the heat for it to rise (without it the arc

wouldn't rise and the ladder would melt down). The ladder is 60cm tall

and about 15cm apart at the end. Works great! I think I'll try putting

some microwave caps in parallel to the MOT. The neighbours will probably

complain though...

Date: Sun, 13 Sep 1998 17:53:55 -0700

From: Gene Waldo <gwaldo@1stnetusa.com>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: coupling

Sender: tesl-err@emachine.com

bob golding wrote:

>

> Hi All,

>

>         Have just been playing around with the coupling of my coil.

> 9 Kv 60mA neon,

> Am I right in thinking that the coupling is at the

> mininim when the the secondary is in line with the bottom turn of the

> primary, therefore there is no point in adjusting the coupling below this point?

> many thanks

>

> bob golding lightning without rain

>

> bob yubba.clara.net

Bob,

Don't take this as advice, I'm asking the question, too.

My guess is that coupling would be at max w/pri & 2ndary lower windings

in the same plane?

----------------------------

Gene Waldo -  gwaldo@1stnetusa.com  - "Igor in Residence"

---------------------------------------------------------------

Date: Sun, 13 Sep 1998 18:04:47 -0700

From: Gene Waldo <gwaldo@1stnetusa.com>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: coupling

Sender: tesl-err@emachine.com

Gene Waldo wrote:

>

> bob golding wrote:

> >

> > Hi All,

> >

> >         Have just been playing around with the coupling of my coil.

> > 9 Kv 60mA neon,

> > Am I right in thinking that the coupling is at the

> > mininim when the the secondary is in line with the bottom turn of the

> > primary, therefore there is no point in adjusting the coupling below this

> > point?

> > many thanks

> >

> > bob golding lightning without rain

> >

> > bob yubba.clara.net

> Bob,

> Don't take this as advice, I'm asking the question, too.

> My guess is that coupling would be at max w/pri & 2ndary lower windings

> in the same plane?

Just read Mike's reply; Igor guess good, urrgh, ahhh ahhh!

--

Gene Waldo -  gwaldo@1stnetusa.com  - "Igor in Residence"

Date: Sun, 13 Sep 1998 18:20:57 -0700

From: Gene Waldo <gwaldo@1stnetusa.com>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Big Coils

Sender: tesl-err@emachine.com

Bert Wikkerink wrote:

>

> Hi Gene,

>

> I've often had the same thought and now you beat me to it and taken all the

> credit. :-(

>

> Bert Wikkerink

>

Bert,

Tellya what I'm gonna do!  You get setup, and all credit to you, for a

bunch of caps for me! ~~<[:-)

-----------------------------------

Gene Waldo -  gwaldo@1stnetusa.com  - "Igor in Residence"

Date: Sun, 13 Sep 1998 20:23:36 -0500

From: Bert Hickman <bert.hickman@aquila.com>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Organization: Stoneridge Engineering

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: coupling

Sender: tesl-err@emachine.com

Gene Waldo wrote:

>

bob golding wrote:

>

> Hi All,

>

>         Have just been playing around with the coupling of my coil.

> 9 Kv 60mA neon,

> Am I right in thinking that the coupling is at the

> mininmum when the the secondary is in line with the bottom turn of the

> primary, therefore there is no point in adjusting the coupling below this

> point?

> many thanks

>

> bob golding lightning without rain

>

> bob yubba.clara.net

Bob and all,

Actually, the lower you move the secondary versus the plane of the

primary, the higher the coefficient of coupling. BTW, you can continue

to increase coupling by moving the plane of the primary so that it's

ABOVE the bottom-most turn on the secondary. For example, to boost

coupling a bit, I normally run my system with the primary plane about

1/2" above the bottom of the secondary winding. Now if you continue in

this direction, coupling continues to increase - the maximum will

actually be reached when the plane of the primary is centered midway

between the ends of the secondary winding.

However, you normally wouldn't want to run anywhere near this

point, since the coupling would be much too high, and you'd certainly

get secondary-to-primary flashovers long before reaching this point on a

base-grounded secondary.

Hope this helps!

-- Bert --

Date: Sun, 13 Sep 1998 19:04:58 -0700

From: Gene Waldo <gwaldo@1stnetusa.com>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: coupling

Sender: tesl-err@emachine.com

Bert Hickman wrote:

>

> Gene Waldo wrote:

> >

> bob golding wrote:

> >

> > Hi All,

> >

> >         Have just been playing around with the coupling of my coil.

> > 9 Kv 60mA neon,

> > Am I right in thinking that the coupling is at the

> > minimum when the the secondary is in line with the bottom turn of the

> > primary, therefore there is no point in adjusting the coupling below this

> > point?

>

> Bob and all,

>

> Actually, the lower you move the secondary versus the plane of the

> primary, the higher the coefficient of coupling. BTW, you can continue

> to increase coupling by moving the plane of the primary so that it's

> ABOVE the bottom-most turn on the secondary. For example, to boost

> coupling a bit, I normally run my system with the primary plane about

> 1/2" above the bottom of the secondary winding. Now if you continue in

> this direction, coupling continues to increase - the maximum will

> actually be reached when the plane of the primary is centered midway

> between the ends of the secondary winding.

Bert, you just confirmed my worst suspicions! !!!!

The problem now is, the cure for flashover!   2 plex barriers, above and

below pri? ????   closley fitted to 2ndary?  Incorporation of 2 strike

rings???

Or is max coupling not that important?   Igor DUMB!

> Hope this helps!

>

> -- Bert --

--

Gene Waldo -  gwaldo@1stnetusa.com  - "Igor in Residence"

---------------------------------------------------------------

Date: Sun, 13 Sep 1998 21:35:22 -0500

From: Bert Hickman <bert.hickman@aquila.com>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Organization: Stoneridge Engineering

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: coupling

Sender: tesl-err@emachine.com

Gene Waldo wrote:

>

> Bert Hickman wrote:

> >

> > Gene Waldo wrote:

> > >

> > bob golding wrote:

> > >

> > > Hi All,

> > >

> > >         Have just been playing around with the coupling of my coil.

> > > 9 Kv 60mA neon,

> >  Am I right in thinking that the coupling is at the

> > > mininim when the the secondary is in line with the bottom turn of the

> > > primary, therefore there is no point in adjusting the coupling below this

> > > point?

>

> >

> > Bob and all,

> >

> > Actually, the lower you move the secondary versus the plane of the

> > primary, the higher the coefficient of coupling. BTW, you can continue

> > to increase coupling by moving the plane of the primary so that it's

> > ABOVE the bottom-most turn on the secondary. For example, to boost

> > coupling a bit, I normally run my system with the primary plane about

> > 1/2" above the bottom of the secondary winding. Now if you continue in

> > this direction, coupling continues to increase - the maximum will

> > actually be reached when the plane of the primary is centered midway

> > between the ends of the secondary winding.

>

> Bert, you just confirmed my worst suspicions! !!!!

> The problem now is, the cure for flashover!   2 plex barriers, above and

> below pri? ????   closley fitted to 2ndary?  Incorporation of 2 strike

> rings???

>

> Or is max coupling not that important?   Igor DUMB!

>

>

>

> --

> Gene Waldo -  gwaldo@1stnetusa.com  - "Igor in Residence"

> ---------------------------------------------------------------

Gene and all,

Maximum coupling is actually undesirable! Once you've got your system

properly tuned, coupling should be carefully adjusted for best

performance. This is actually quite complex to predict beforehand, and

can really be determined only experimentally, and only after the system

has been brought into tune.

Starting out with the secondary ABOVE the plane of the primary, slowly

lower the secondary in increments of 1/4" and try running the coil

again. This needs to be done in the dark so that you can look for any

signs of inter-turn flashovers on the secondary, any abnormal corona or

flashovers between the secondary and primary, or between the toroid and

the primary. The proper coupling for YOUR system will be achieved when

you get the longest streamers WITHOUT getting any of the abnormal

behavior noted above.

For most systems this is typically reached with the bottom of the

secondary winding at, or above, the plane of the primary (for a pancake

primary), or 1-3" above the bottom of the primary (for an inverse

conical primary). The coupling coefficient for best performance will

typically found to be in the range of 0.15 - 0.22. If you have an

extraordinarily good insulation system on your secondary, and you're

getting good quenching on your gaps, you may be able to wring a bit

better performance with even greater coupling. The practical limit for

classical 2-coil systems is probably no more than 0.28, however.

-- Bert --

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Flat Stack  Cap Death  :^(     !! #@*&!!

X-Mailer: IPAD 2.0

From: break7@juno.com (Tyla A. Break)

Date: Sun, 13 Sep 1998 22:37:35 EDT

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

I just finished building a cap.

.062  HDPE

30 sheets Al flashing

6" x 8" Plates

8" x10" Poly

I put the cap in a Rubber Maid container and filled it with Diala AX.

Waited about 30 minutes for the air to come out. Then energized with

7500@ 18 mils. Everything O.K. Now I started running a coil and started

tuning . Everything was still O.K. but my twin pushed me into trying a

12kv @ 30ma.  The cap was working great 12kv wide open! Now if anyone has

a bro that likes to push your equip JUST SAY NO ! (stolen from Nancy

Reagan). The cap cannot handle a 15 kv @60 ma !!  Small Light Show -

smokey poluted oil, 3 - 10 dead sheets of poly . I know I should have

used at least two in series and probably waited several days for more air

to come out.  Does anyone know what I should use to clean up the oil for

rebuilding? Cap was too big anyway, I clipped the primary all the way out

and still needed more. (12 turns .375 Cu tubing). Is anyone else building

these? I only hear about rolled caps.

I'll	 learn from my stupid mistakes

If it's not dangerous It's not fun!    Kyle  break7@juno.com

Date: Sun, 13 Sep 1998 22:07:34 -0500

From: Bert Hickman <bert.hickman@aquila.com>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Organization: Stoneridge Engineering

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Flat Stack  Cap Death  :^(     !! #@*&!!

Sender: tesl-err@emachine.com

Kyle,

Sorry to hear about your cap! Were these single sheets of 0.062" poly or

were they made from multiple thicknesses of thinner material (best)?

While it may be possible to filter out the suspended carbon by running

the oil through some type of filter, your best bet may be to try

removing the portion of clean oil by carefully removing it with a turkey

baster, avoiding carbon plumes, then discarding the rest. Also, if you

needed to use all the primary turns, it sounds like the cap may have had

too LOW a capacitance.

Both Mike and I use home-made flat plate type capacitors with excellent

success. However, we use multiple sheets of thin poly, and also connect

multiple equally sized capacitors in series to reduce the stress seen on

any single cap. I happen to use 128 mils of total poly, 32 mils in 4

identically sized caps in series, but this was designed for operation

off a 20 KV pole-pig. Operation off a 15/60 NST should be possible with

75 - 90 mils as long as you share the stress across two or more

equally-sized caps. This is essential even when running off 12 KV

NST's...

-- Bert --

Tyla A. Break wrote:

>

>         I just finished building a cap.

>         .062  HDPE

>         30 sheets Al flashing

>         6" x 8" Plates

>         8" x10" Poly

> I put the cap in a Rubber Maid container and filled it with Diala AX.

> Waited about 30 minutes for the air to come out. Then energized with

> 7500@ 18 mills. Everything O.K. Now I started running a coil and started

> tuning . Everything was still O.K. but my twin pushed me into trying a

> 12kv @ 30ma.  The cap was working great 12kv wide open! Now if anyone has

> a bro that likes to push your equip JUST SAY NO ! (stolen from Nancy

> Reagan). The cap cannot handle a 15 kv @60 ma !!  Small Light Show -

> smokey poluted oil, 3 - 10 dead sheets of poly . I know I should have

> used at least two in series and probably waited several days for more air

> to come out.  Does anyone know what I should use to clean up the oil for

> rebuilding? Cap was too big anyway, I cliped the primary all the way out

> and still needed more. (12 turns .375 Cu tubing). Is anyone else building

> these? I only hear about rolled caps.

>

>              I'll        learn from my stupid mistakes

>         If it's not dangerous It's not fun!    Kyle  break7@juno.com

>

From:          "Malcolm Watts" <MALCOLM@directorate.wnp.ac.nz>

Organization:  Wellington Polytechnic, NZ

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Date:          Mon, 14 Sep 1998 15:09:25 +1200

Subject:       Re: Flat Stack  Cap Death  :^(     !! #@*&!!

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Hi Kyle,

Sorry to hear about your capacitor. It may be that not all

bubbles had really left it before you fired at full revs. I've built

several flat plate caps. The first was outbubbling for several months

before use. I was *very* conservative on PE insulation and it consists

of two 10.5nF units that I can use in parallel for lower voltages,

one section at the same voltage rating and two in series for much

higher voltages.

A second cap used PTFE sheeting for its dielectric and I rate it

at 12kVpk in Tesla service. A third will be oil filled this evening

and will be rated at 8kVpk for TC use.

Malcolm

> To:             "Tesla-2 Mailing List" <tesla- 2@emachine.com>

> Subject:        Flat Stack  Cap Death  :^(     !! #@*&!!

> From:           break7@juno.com (Tyla A. Break)

> Date sent:      Sun, 13 Sep 1998 22:37:35 EDT

> Send reply to:  "Tesla-2 Mailing List" <tesla-2@emachine.com>

>     I just finished building a cap.

>     .062  HDPE

>     30 sheets Al flashing

>     6" x 8" Plates

>     8" x10" Poly

> I put the cap in a Rubber Maid container and filled it with Diala AX.

> Waited about 30 minutes for the air to come out. Then energized with

> 7500@ 18 mills. Everything O.K. Now I started running a coil and started

> tuning . Everything was still O.K. but my twin pushed me into trying a

> 12kv @ 30ma.  The cap was working great 12kv wide open! Now if anyone has

> a bro that likes to push your equip JUST SAY NO ! (stolen from Nancy

> Reagan). The cap cannot handle a 15 kv @60 ma !!  Small Light Show -

> smokey poluted oil, 3 - 10 dead sheets of poly . I know I should have

> used at least two in series and probably waited several days for more air

> to come out.  Does anyone know what I should use to clean up the oil for

> rebuilding? Cap was too big anyway, I cliped the primary all the way out

> and still needed more. (12 turns .375 Cu tubing). Is anyone else building

> these? I only hear about rolled caps.

>

>              I'll    learn from my stupid mistakes

>     If it's not dangerous It's not fun!    Kyle  break7@juno.com

>

From:          "Malcolm Watts" <MALCOLM@directorate.wnp.ac.nz>

Organization:  Wellington Polytechnic, NZ

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Date:          Mon, 14 Sep 1998 15:10:34 +1200

Subject:       Re: coupling

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

On coupling:

> Date sent:      Sun, 13 Sep 1998 17:53:55 -0700

> From:           Gene Waldo <gwaldo@1stnetusa.com>

> Send reply to:  "Tesla-2 Mailing List" <tesla-2@emachine.com>

> To:             "Tesla-2 Mailing List" <tesla-2@emachine.com>

> Subject:        Re: coupling

> bob golding wrote:

> >

> > Hi All,

> >

> >         Have just been playing around with the coupling of my coil.

> > 9 Kv 60mA neon,

>  Am I right in thinking that the coupling is at the

> > minimum when the the secondary is in line with the bottom turn of the

> > primary, therefore there is no point in adjusting the coupling below this

> > point?

> > many thanks

> >

> > bob golding lightning without rain

> >

> > bob yubba.clara.net

> Bob,

> Don't take this as advice, I'm asking the question, too.

> My guess is that coupling would be at max w/pri & 2ndary lower windings

> in the same plane?

Highest degree of coupling will occur with the primary halfway up the

secondary. However, that will not be useful for TC service.

Malcolm

From: "Choad" <choad69@geocities.com>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Using an isolation xformer for ballasting

Date: Sun, 13 Sep 1998 22:40:22 -0500

X-Mailer: IPAD 2.0

X-MimeOLE: Produced By Microsoft MimeOLE V4.72.3110.3

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

ello all,

I am trying to do some current limiting for some MOT's I

have & need a little help. I have a 3Kva 480/240v or 240/120v

stepup/down transformer. It has 2 primarys & 2 secondaries.

I can wire it for 120v in and 480v out by parralleling the

primarys & seriesing the secondaries. I am wanting to run the

120v side in series with my 120v power lines and put ballesting

resistors on the other side of it. I don't want it to drop a lot of

voltage across the isolation transformer like it has been. All

I current have is 2 - 12 ohm 200w resistors I've been using in

parrallel for 3 ohm. I understand as I step up the voltage with

this isolation transformer that i'm steping down current. So

would using smaller value resistors make less of a drop across

the transformer? I think I'm on the right track, but I'd hate to

spend more money to find out I am wrong.

Choad

Member of The Oklahoma Tesla Coil Builders

http://www.geocities.com/SouthBeach/Pier/2756/index.htm

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Flat Stack Death

X-Mailer: IPAD 2.0

From: break7@juno.com (Tyla A. Break)

Date: Sun, 13 Sep 1998 23:53:09 EDT

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Thanks for the reply Bert!  I'm only using single sheets of .062.

When you guys say stacks of thin sheets do you mean plastic drop

cloth, or trash bags?  I've been getting 4' x 8' sheets and cutting 'em

up. Your right I had it backwards too small a cap.

Time to rebuild   ~   ~

{x} {x}

>

\/\/\/\/      No pain

No gain!

If it's not dangerous it's not fun!

Kyle  break7@juno.com

Date: Sun, 13 Sep 1998 22:57:20 -0500

From: Tom Polk <tpolk@prismnet.com>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Flat Stack  Cap Death  :^(     !! #@*&!!

Sender: tesl-err@emachine.com

Tyla A. Break wrote:

> Is anyone else building these?

>I only hear about rolled caps.

I am very sorry to hear about your cap failure. I experienced that on my

.0625 rolled caps, but not on my flat cap, which is similar to yours. Mine

has 30 layers, 16 x 12", 1/8" poly, single thicknesses. It has held up

for four years, 15kv@100ma.  I guess with single thicknesses you really

have to overkill.

Date: Sun, 13 Sep 1998 23:30:11 -0500

From: Bert Hickman <bert.hickman@aquila.com>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Organization: Stoneridge Engineering

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Flat Stack Death

Sender: tesl-err@emachine.com

Kyle,

Actually, multiple sheets of thinner material. The real trick is to use

multiple dielectric thicknesses so that a defect won't propagate through

the dielectric under stress - all dielectrics will have minor scratches,

entrapped air bubbles, impurities, or other defects that will become the

weakest link under stress. By using thinner sheets, the dielectric

strength is significantly improved - a very thin sheet may have a

breakdown strength (in volts/mil) that's 3-5X (or more!) than that of a

thicker sheet. And many coiler's have found that 0.062" of poly is NOT

sufficient for 15 KV... the hard way! :^)

There are several tricks that the pros use when they make robust

commercial high voltage capacitors:

1. use multiple dielectric thicknesses between plates

2. connect multiple identical capacitors in series to reduce the voltage

stress seen by an individual cap to 5 KV or less. This is critical,

since higher voltage stresses can actually cause bubbles to form IN THE

OIL, particularly at high-stress areas such as the edges of the

capacitor plates. Once ozone and small sparks (partial discharges) begin

forming, failure of the cap becomes just a matter of time.

3. Use high quality materials, keep dust/dirt out during assembly, and

remove all entrapped air by substituting an appropriate high quality

dielectric oil. The pros pull a vacuum to remove the air, and then

backfill with dielectric oil. Some even add very thin kraft paper to aid

in wicking oil between layers, and as a mechanism to improve

distribution of voltage stresses.

You can use these same tricks to make more reliable caps. Hope this

helps!

-- Bert --

Tyla A. Break wrote:

>

>         Thanks for the reply Bert!  I'm only using single sheets of .062

> .  When you guys say stacks of thin sheets do you mean plastic drop

> cloth, or trash bags?  I've been getting 4' x 8' sheets and cutting 'em

> up. Your right I had it backwards too small a cap.

>

>         Time to rebuild   ~   ~

>                                                  {x} {x}

>                                                      >

>                                                   \/\/\/\/      No pain

> No gain!

>                         If it's not dangerous it's not fun!

>                         Kyle  break7@juno.com

>

X-Mailer: IPAD 2.0

Date: Mon, 14 Sep 1998 08:32:05 +0300

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Marco Denicolai <marco@vistacom.fi>

Subject: My first RSG works!

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

I got my first RSG ready:

- 11000 RPM angle grinder motor

- 15 cm diameter fiber bachelite disk, 8 mm tick

- 2 rotating electrods (4 mm diameter tungsten sticks, 15 mm long) -> 367 BPS

- 2 fixed electrods (same material as above)

- clearance between fixed and rotating electrods is 2 x 1 mm

- the whole setup enclosed in a thuff steel/aluminium case

- the case isolated with noise absorbtion foil (from the inside) to avoid

annoying neightbours calls to the police.

I set my RQ sparkgap wide (5 sections) and connected my RSG in parallel

with it. Instead of the discontinuous rattling I was used to ear I got now

a constant tone. Wow!

It was difficult to measure streamers' length precisely but I am 100% sure they

were about 20% longer than before. I also got more streamers leaving from the

toroid at the same time (I guess 4 - 6).

After several 20 s. runs I opened the RSG case and touched the fixed electrods:

they were maybe 40 degrees (centigrade). The trick is that I screwed each of

them in a brass block 5 X 1.5 X 1.5 cm, used as a termal cooler/mass. There

was also no noticeable trace of erosion.

For those of you that want to know about the other components of my setup,

please check http://vci.vistacom.fi/~marco/mytesla.html. In short, I'm

using a MOT supply and a 0.05 uF primary capacitor.

My next steps will be to built a bigger toroid (same outer diameter but bigger

curvature): from the current 10 cm diameter aluminium duct to a new 16 cm

diameter duct.

As well I intend to build a speed control for the angle grinder, to test also

lower than 367 BPS.

Marco Denicolai                   Vista Communication Instruments, Inc.

Hardware Development Manager      www.vistacom.fi

X-Mailer: IPAD 2.0

Date: Mon, 14 Sep 1998 08:38:37 +0300

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Marco Denicolai <marco@vistacom.fi>

Subject: Motor speed control needed

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

I have a 220V 500W angle grinder that runs at 11000 RPM. I want to build

a speed control for it. Somewhere I heard I need to vary the voltage of the

fixed winding and keep using full voltage for the rotating ones.

Has anybody of you got more precise instructions or even a schematic diagram?

Why I didn't buy an angle grinder with speed control? Because it would have

costed over $100. This rubbish I have got (new) costed $20 and I think should

last enough if you think of its use as a RSG.

________________________________________________________________________

Marco Denicolai                   Vista Communication Instruments, Inc.

Hardware Development Manager      www.vistacom.fi

marco@vistacom.fi

From:          "Malcolm Watts" <MALCOLM@directorate.wnp.ac.nz>

Organization:  Wellington Polytechnic, NZ

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Date:          Mon, 14 Sep 1998 18:07:24 +1200

Subject:       Re: Using an isolation xformer for ballesting

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Hi Choad, all,

Perhaps a little tutorial on transformers would be in

order as there seems to be a gap in understanding out there.

> From:           "Choad" <choad69@geocities.com>

> To:             "Tesla-2 Mailing List" <tesla-2@emachine.com>

> Subject:        Using an isolation xformer for ballesting

> Date sent:      Sun, 13 Sep 1998 22:40:22 -0500

> Send reply to:  "Tesla-2 Mailing List" <tesla-2@emachine.com>

> ello all,

>

>     I am trying to do some current limiting for some MOT's I

> have & need a little help. I have a 3Kva 480/240v or 240/120v

> stepup/down transformer. It has 2 primarys & 2 secondaries.

> I can wire it for 120v in and 480v out by parralleling the

> primarys & seriesing the secondaries. I am wanting to run the

> 120v side in series with my 120v power lines and put ballesting

> resistors on the other side of it. I don't want it to drop a lot of

> voltage across the isolation transformer like it has been. All

> I current have is 2 - 12 ohm 200w resistors I've been using in

> parrallel for 3 ohm. I understand as I step up the voltage with

> this isolation transformer that i'm steping down current. So

> would using smaller value resistors make less of a drop across

> the transformer? I think I'm on the right track, but I'd hate to

> spend more money to find out I am wrong.

The shunts in your MOTs will limit the amount of power you can suck

from their secondaries. I wrote a note on this operation earlier.

Now for the tutorial.

Take an "ideal" transformer (no magnetic shunts in its core). The

more A.C. voltage you apply to a winding, the more current it passes

through that winding. The ampere-turns of the energized winding builds

up a magnetizing force in the core. This magnetizing current is

governed by the inductance of the winding and is related to applied

voltage by: Imag = Vapplied/2*PI*F*L   where F is the applied

frequency.

In an "ideal" transformer, L is infinite and no magnetizing

current flows. In the real world, one *does* want some magnetizing

current to flow or there will be no flux change in the core and hence

no EMF induced in other windings on it.

This inductance can be treated as a separate entity from any

loading on other windings. Looking into the transformer terminals,

one sees this inductance in parallel with whatever load is placed on

the secondary. If the load is a resistance, one sees a resistance

transformed by the turns ratio squared appearing in parallel with the

magnetizing inductance when looking into the energized winding. Why

turns ratio squared?

Take a transformer which has a stepup ratio of 1:2. The secondary

produces 2V if one volt is applied to the primary. Suppose one places

a 2 Ohm resistor across the secondary. The resistor will draw one amp

with 2 volts applied. Secondary power is Vsec*Isec = 2W. Primary

power also = 2W so the AC supply sees a load current of 2 amps.

Secondary V/I = 2V/1A = 2 Ohms

Primary V/I = 1V/2A = 0.5 Ohms

So does a stepup transformer "up" primary volts and "down" primary

amps? Yes, *BUT* that does not prevent you sticking a load of 0.1 Ohm

on the secondary of our hypothetical transformer and sucking 20 Amps

from the secondary. The load current when looking into the primary

would of course be 40 Amps. You can do this ad infinitum with an

ideal transformer. However, transformers are not ideal. Their

windings also have resistance. So as load current goes up, heating in

the windings go up according to the I^2*R law. The result is that

while a transformer can theoretically deliver whatever you care to

suck from it, its windings will suffer heating and the more you load

it, the greater that heating will be. Transformers do **NOT**

automatically somehow limit the current you can suck from their

secondaries if there are no flux shunts in the core like a neon sign

transformer or MOT.

It might look like an attractive proposition to build a

transformer with a core and, say, 100 turns on the primary and 10,000

turns on the secondary. You get a voltage step up ratio of 1:100,

right? So you apply 120VRMS to the primary and get 12000VRMS out.

Fine, but remember the magnetizing factor. Ampere-turns go up as

primary turns go down and the core can only support so many ampere

turns before running out of domains to magnetize. If this happens,

primary inductance all but vanishes, primary current goes through the

roof and you are left in effect with an air core transformer which

exhibits a huge amount of leakage inductance. It is also guaranteed

to blow fuses.

To see why this happens, remember that Imag is related to

inductance and inductance is related to turns squared. Suppose you

have a transformer with 200 turns on its primary and it has an

inductance of 0.5 Henry. You apply 120VRMS at 60Hz. Its magnetizing

current then is:  Imag = 120V/2*PI*F*L   = 637mA

Ampere-turns applied to the core = 200*0.637 = 127.4

You decide you can do better and knock the primary back to 100 turns

and use the same core. What happens? L drops by primary turns squared.

So Lp now = 0.5H/2^2 = 0.125H  since you have halved the primary

turns.  Now Imag = 120V/2*PI*60*0.125 A  = 2.55 Amps. Doesn't sound

like much. BUT now the Ampere-turns applied to the core = 255!!!  and

the core was designed to handle say 140 A-t (to allow for mains

voltage fluctuations) and no more. Manufacturers do not put in more

iron than the transformer needs for their original specifications.

It adds weight, cost and he does want to sell the article in a

useable form for as little as possible to be competitive with other

manufacturers. In fact, if the core was rated for 140 A-t, the real

applied A-t in this situation wil be *humungouly* higher than 255!!

because the core ran out of domains to align at 140 A-t. That means

that Imag will easily hit fuse-popping levels.

A final note for prospective transformer designers out there:

remember that the primary winding must not only carry the reflected

load current you rate the transformer for, it must also carry the

magnetizing current. For this reason, transformers with cores

generally have more copper in their primary windings than in their

secondary windings. This will give some insight into the whys and

wherefores of operating a transformer in reverse.

Malcolm

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Date: Mon, 14 Sep 1998 04:55:50 -0700

From: "paul m" <fleischwulf@mailexcite.com>

Cc:

X-Sent-Mail: off

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

Subject: Re: Big Coils

X-Sender-Ip: 208.153.7.9

Organization: MailExcite  (http://www.mailexcite.com:80)

Sender: tesl-err@emachine.com

Tom,

I'm using a pole pig (25 KVA) it is current limited to 4,000 w.

p. marshall

---

Einigheit und Wahrheit

On Sat, 12 Sep 1998 10:14:46   Tom Stock wrote:

>Paul, type of transformer are you using?

>

>Thanks

>-tom

>tomstock@geocities.com

>

>

>

>paul m wrote:

>>

>> My 6.5" coil is currently producing 74 - 75" arcs and it still is not fine tuned. I'm running with a .026uF cap and I think for proper tunning I'm going to increase the capacitance to .036. I'm feeding this coil 4 Kw. 75" arcs are about all I

>ca

>> n deal with at the moment.

>>

>> paul m

>> ---

>> Einigheit und Wahrheit

>

X-Originating-IP: [199.217.140.22]

From: "Bill the arcstarter" <arcstarter@hotmail.com>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Flat Stack Cap Death :^( !! #@*&!!

Date: Mon, 14 Sep 1998 07:19:11 PDT

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

It was written:

>	I just finished building a cap.

>	.062  HDPE

>	30 sheets Al flashing

>	6" x 8" Plates

>	8" x10" Poly

>I put the cap in a Rubber Maid container and filled it with Diala AX.

This is very similar to the ones Paul built.  We used 0.060 poly which

had an opaque whitish appearance.  Used 48 sheets of (I think) 8x12

plastic.  The foils (Reynold's heavy duty aluminum) left an inch of

clearance on all sides.

Once I charged one up to 30KVDC!  I was drawing hair-thin arcs to a

100-Meg resistor!  That was fun!  We also had some sucess blowing up wet

wood using these caps.  The energy isn't high but we had some results

just the same.

I forget the burn-in process we used, but I think we connected the cap

to a MOT and let it run for a day or so to get most of the air out.  It

sounded like a box full of yellow-jackets! Then we ran it on a neon and

variac to get more air out (NOTE- that is NOT recommended as you can get

a resonant rise effect wihich will kill the cap or neon if you don't use

a safety gap)

Our caps have been running strong for about a year now.  We've been

running four caps, in series-parallel to share the voltage a bit.  But

we did run a coil off a 15kv neon using only one cap for a few days, and

the cap didn't complain! (killed the neon though :(  )

-Bill the arcstarter

Starting arcs in Cinci, OH

http://www.geocities.com/CapeCanaveral/Hangar/6160/tesla.html

Date: Mon, 14 Sep 1998 08:55:15 -0700

From: Gene Waldo <gwaldo@1stnetusa.com>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Flat Stack  Cap Death  :^(     !! #@*&!!

Sender: tesl-err@emachine.com

Tyla A. Break wrote:

>

>         I just finished building a cap.

Everything was still O.K. but my twin pushed me into trying a

> 12kv @ 30ma.  The cap was working great 12kv wide open! Now if anyone has

> a bro that likes to push your equip JUST SAY NO !

Kyle,

Any chance of talking your brother into building his own coil?

He sounds like he'd be a great source of helpfull and time-saving data!

~~<[:-)

------------------------------------------------

Gene Waldo -  gwaldo@1stnetusa.com  - "Igor in Residence"

---------------------------------------------------------------

What's good for the planet is more OZONE! See the machinery form up!

NEW PAGE!   http://members.1stnetusa.com/danmurphy/waldo.htm

From: "Basura, Brian" <brian.basura@unistudios.com>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: RE: Potential Trans

Date: Mon, 14 Sep 1998 09:18:58 -0700

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

I got the same story from the old man...  No deal..   Hey..  Maybe I'm the

other coiler :')

Brian D. Basura

> ----------

> From: 	break7@juno.com[SMTP:break7@juno.com]

> Reply To: 	Tesla-2 Mailing List

> Sent: 	September 11, 1998 10:15 PM

> To: 	Tesla-2 Mailing List

> Subject: 	Potential Trans

>

> 	I went to APEX to try to get the potential trans I mentioned last

> week. Aparently a local coiler or someone else has been trying too! After

> I was flatly denied, I was told that some guy offered $150 for it.

>

>  The old man rents it as a background prop to the studios for $40 a

> week.  Hard to compete with that!

>

> 	If it's not dangerous it's not fun!  Kyle  break7@juno.com

>

X-Mailer: IPAD 2.0

Date: Mon, 14 Sep 1998 12:10:28 -0500

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: Flat Stack Cap Death

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

At 10:37 PM 9/13/98 EDT, you wrote:

>	I just finished building a cap.

>	.062  HDPE

>	30 sheets Al flashing

>	6" x 8" Plates

>	8" x10" Poly

>I put the cap in a Rubber Maid container and filled it with Diala AX.

>Waited about 30 minutes for the air to come out. Then energized with

>7500@ 18 mills. Everything O.K. Now I started running a coil and started

>tuning . Everything was still O.K. but my twin pushed me into trying a

>12kv @ 30ma.  The cap was working great 12kv wide open! Now if anyone has

>a bro that likes to push your equip JUST SAY NO ! (stolen from Nancy

>Reagan). The cap cannot handle a 15 kv @60 ma !!  Small Light Show -

>smokey poluted oil, 3 - 10 dead sheets of poly . I know I should have

>used at least two in series and probably waited several days for more air

>to come out.  Does anyone know what I should use to clean up the oil for

>rebuilding? Cap was too big anyway, I cliped the primary all the way out

>and still needed more. (12 turns .375 Cu tubing). Is anyone else building

>these? I only hear about rolled caps.

>

>             I'll	 learn from my stupid mistakes

>	If it's not dangerous It's not fun!    Kyle  break7@juno.com

>

Kyle,

Sorry to hear about your cap failure. Been there, done that.

This only further proves the point Ive tried to make over

and over that single caps just will not survive at any more

than 9-10kv NO MATTER WHAT THE POLY THICKNESS IS.

A single cap subjected to more than 10kv will develop

corona along the plate edges EVEN UNDER OIL. This corona

will eat away at the poly until a failure occurs. Its inevitable.

Sure several people will say " I built my cap with XX mils

of poly and run it at 15kv and it runs just fine". But if you

disassemble that cap you will see burn marks all along the

plate edges. If the poly is quite thick it may take a while

for the corona to burn its way through. But it will burn through

eventually.

I built a .01 mfd plate cap using 10 layers of 6 mil poly

for each dielectric and plates made from aluminum flashing.

My first attempt to build this cap resulted in poor performance

due to trapped air. I rebuilt the cap under oil. I dipped each

poly sheet in oil and worked out the air bubbles with my fingers

as I added each layer. This was a messy job but resulted in a

very good performing cap that needed no break in time.

However this cap failed after a couple hours of use at 12kv despite

having 60 mils of poly. An autopsy revealed that the poly was severely

damaged along the plate edges and in several locations 4 or 5 of the 10

layers were burnt through. In the area where the failure occurred

the hole through all 10 layers of poly was as big as a half dollar.

The aluminum plates had large dents and looked like they had been

struck with a hammer. The 1/8 inch plexiglass used to sandwich the

cap assembly was shattered.

What was interesting was that during the failure this cap

was in parallel with another plate cap. This 2nd cap survived

despite only having 48 mils of poly. Why did the 60 mil cap

fail while the 48 mil cap survived? Because the 48 mil cap

was composed of 6 series caps each with 8 mils of poly. This

spreads the stress out across many dielectric layers and

the voltage across each individual cap stays low enough

that the damaging corona could not form.

So the lesson learned here is that if you want a good performing

RELIABLE cap use a series string. A minimum of two caps in

series for 12kv and 3 in series for 15kv. If you do this

you can take advantage of cheaper hardware store grade poly

and still have a good performing cap.

About your contaminated oil, forget it. Its full of carbon

particles that will seriously degrade the insulating property

of the oil. If the breakdown was minor then it might not

be contaminated too bad but in my cap failures it was enough

to give the oil a yellowish color. (mineral oil)

I saved the oil however and eventually used it for my

rebuilt neon transformer. It worked just fine in that

capacity. I would use clean fresh oil for your caps.

TTYL!

Mike Hammer

mhammer@misslink.net

Date: Mon, 14 Sep 1998 12:34:17 -0600

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Daniel Lockhart <molly@cmn.net>

Subject: Re: Flat Stack Cap Death

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

Mike wrote:

>Sorry to hear about your cap failure. Been there, done that.

>This only further proves the point Ive tried to make over

>and over that single caps just will not survive at any more

>than 9-10kv NO MATTER WHAT THE POLY THICKNESS IS.

>

>A single cap subjected to more than 10kv will develop

>corona along the plate edges EVEN UNDER OIL. This corona

>will eat away at the poly until a failure occurs. Its inevitable.

>> snip! <<

>However this cap failed after a couple hours of use at 12kv despite

>having 60 mils of poly. An autopsy revealed that the poly was severely

>damaged along the plate edges and in several locations 4 or 5 of the 10

>layers were burnt through. In the area where the failure occurred

>the hole through all 10 layers of poly was as big as a half dollar.

>The aluminum plates had large dents and looked like they had been

>struck with a hammer. The 1/8 inch plexiglass used to sandwich the

>cap assembly was shattered.

>

>What was interesting was that during the failure this cap

>was in parallel with another plate cap. This 2nd cap survived

>despite only having 48 mils of poly. Why did the 60 mil cap

>fail while the 48 mil cap survived? Because the 48 mil cap

>was composed of 6 series caps each with 8 mils of poly. This

>spreads the stress out across many dielectric layers and

>the voltage across each individual cap stays low enough

>that the damaging corona could not form.

>

>So the lesson learned here is that if you want a good performing

>RELIABLE cap use a series string. A miunimum of two caps in

>series for 12kv and 3 in series for 15kv. If you do this

>you can take advantage of cheaper hardware store grade poly

>and still have a good performing cap.

Taking this to extremes, would it be best to make a cap with a very thin

dielectric? Using, say, one mill (0.001 inch) between plates would result

in a very high capacitance and low operating voltage, thus making it

nessesary to put many units in series. I guess that this would be more

efficient, although I don't know why.

To even further extremes, could this same principle be taken to a much

smaller, nearly microscopic level? What would performance be like here? I'm

not thinking about building anything like this (I don't know how you

would), I'm just thinking theoretical here.

Daniel Lockhart

Date: Mon, 14 Sep 1998 14:05:48 -0500

From: Tom Polk <tpolk@prismnet.com>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Flat Stack Cap Death

Sender: tesl-err@emachine.com

Daniel Lockhart wrote:

> Taking this to extremes, would it be best to make a cap with a very thin

> dielectric? Using, say, one mill (0.001 inch) between plates would result

> in a very high capacitance and low operating voltage...

>

> To even further extremes, could this same principle be taken to a much

> smaller, nearly microscopic level?

If you are going to go to that extreme, why not experiment with

distilled water as a dialectric, with its K of 40? That's 15-20 times

the capacitance of poly/oil caps.

I recall some years back when the excimer laser was developed, Popular

Science disclosed that it used a high voltage water capacitor; so there

must be a way for it to work. I always wondered how they got around the

breakdown voltage problem. It supplied very short pulses. Maybe it

discharged before breakdown.

X-Mailer: IPAD 2.0

Date: Mon, 14 Sep 1998 15:22:24 -0500

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: Flat Stack Cap Death

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

At 12:34 PM 9/14/98 -0600, you wrote:

>Mike wrote:

>

>>Sorry to hear about your cap failure. Been there, done that.

>>This only further proves the point Ive tried to make over

>>and over that single caps just will not survive at any more

>>than 9-10kv NO MATTER WHAT THE POLY THICKNESS IS.

>>

>>A single cap subjected to more than 10kv will develop

>>corona along the plate edges EVEN UNDER OIL. This corona

>>will eat away at the poly until a failure occurs. Its inevitable.

>

>	>> snip! <<

>

>>However this cap failed after a couple hours of use at 12kv despite

>>having 60 mils of poly. An autopsy revealed that the poly was severely

>>damaged along the plate edges and in several locations 4 or 5 of the 10

>>layers were burnt through. In the area where the failure occurred

>>the hole through all 10 layers of poly was as big as a half dollar.

>>The aluminum plates had large dents and looked like they had been

>>struck with a hammer. The 1/8 inch plexiglass used to sandwich the

>>cap assembly was shattered.

>>

>>What was interesting was that during the failure this cap

>>was in parallel with another plate cap. This 2nd cap survived

>>despite only having 48 mils of poly. Why did the 60 mil cap

>>fail while the 48 mil cap survived? Because the 48 mil cap

>>was composed of 6 series caps each with 8 mils of poly. This

>>spreads the stress out across many dielectric layers and

>>the voltage across each individual cap stays low enough

>>that the damaging corona could not form.

>>

>>So the lesson learned here is that if you want a good performing

>>RELIABLE cap use a series string. A miunimum of two caps in

>>series for 12kv and 3 in series for 15kv. If you do this

>>you can take advantage of cheaper hardware store grade poly

>>and still have a good performing cap.

>

>Taking this to extremes, would it be best to make a cap with a very thin

>dielectric? Using, say, one mill (0.001 inch) between plates would result

>in a very high capacitance and low operating voltage, thus making it

>nessesary to put many units in series. I guess that this would be more

>efficient, although I don't know why.

>

>To even further extremes, could this same principle be taken to a much

>smaller, nearly microscopic level? What would performance be like here? I'm

>not thinking about building anything like this (I don't know how you

>would), I'm just thinking theoretical here.

>

>Daniel Lockhart

>

Daniel,

This is exactly how commercial HV caps are produced.

High value caps of low voltage rating are made with

very thin polypropylene film. Often as thin as 0.5 mil.

Enough of these units are wired in series to achieve

the desired voltage rating. If more capacitance is

required additional series strings are paralleled

with the first.

These thinner films give much greater PER MIL ratings

than do thicker films. In other words 10 sheets of 6 mil

poly have a much greater voltage rating than a single

thickness of 60 mils.

It would not be practical nor would it be necessary to go

to this extreme for a TC cap. Having 20 or 30 caps in series

would require lots of construction time.

IMO if you want the very best reliability then you should

think about 4 or more series caps. Six 10 mil caps in series

would probably be nearly bullet proof even at 15kv. My .008 mfd

plate cap consisting of six 8 mil caps in series has been bullet

proof through some quite rough usage at 12kv. I have never used

it at 15kv. It would probably be OK but thats pushing things.

TTYL!

Mike Hammer

mhammer@misslink.net

X-Mailer: IPAD 2.0

Date: Mon, 14 Sep 1998 15:25:27 -0500

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: Flat Stack Cap Death

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

At 02:05 PM 9/14/98 -0500, you wrote:

>Daniel Lockhart wrote:

>

>> Taking this to extremes, would it be best to make a cap with a very thin

>> dielectric? Using, say, one mill (0.001 inch) between plates would result

>> in a very high capacitance and low operating voltage...

>

>>

>> To even further extremes, could this same principle be taken to a much

>> smaller, nearly microscopic level?

>

>If you are going to go to that extreme, why not experiment with

>distilled water as a dialectric, with its K of 40? That's 15-20 times

>the capacitance of poly/oil caps.

>

>I recall some years back when the excimer laser was developed, Popular

>Science disclosed that it used a high voltage water capacitor; so there

>must be a way for it to work. I always wondered how they got around the

>breakdown voltage problem. It supplied very short pulses. Maybe it

>discharged before breakdown.

The problem with water is that it must be absolutely pure to

even come close to being an insulator. And that level of purity

would be nearly impossible to maintain for long as water is also

an excellent solvent.

Distilled water isnt nearly pure enough for the purpose.

TTYL!

Mike Hammer

mhammer@misslink.net

Date: Mon, 14 Sep 1998 13:29:25 -0700

From: Gene Waldo <gwaldo@1stnetusa.com>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Flat Stack Cap Death

Sender: tesl-err@emachine.com

Tom Polk wrote:

>

> Daniel Lockhart wrote:

>

> If you are going to go to that extreme, why not experiment with

> distilled water as a dialectric, with its K of 40? That's 15-20 times

> the capacitance of poly/oil caps.

>

> I recall some years back when the excimer laser was developed, Popular

> Science disclosed that it used a high voltage water capacitor;

All,

So, where's the dist. water cap data? Caps seem our major weak link...

Is there a happy medium between reliable performance and construction

complexity?

------------------------------------------------------------

Gene Waldo -  gwaldo@1stnetusa.com  - "Igor in Residence"

---------------------------------------------------------------

What's good for the planet is more OZONE! See the machinery form up!

NEW PAGE!   http://members.1stnetusa.com/danmurphy/waldo.htm

Date: Mon, 14 Sep 1998 15:38:19 -0600

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Daniel Lockhart <molly@cmn.net>

Subject: Re: Flat Stack Cap Death

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

I wrote:

>> Taking this to extremes, would it be best to make a cap with a very thin

>> dielectric? Using, say, one mill (0.001 inch) between plates would result

>> in a very high capacitance and low operating voltage...

>> To even further extremes, could this same principle be taken to a much

>> smaller, nearly microscopic level?

Tom Polk replied:

>If you are going to go to that extreme, why not experiment with

>distilled water as a dialectric, with its K of 40? That's 15-20 times

>the capacitance of poly/oil caps.

Exactly. This is what I was getting at in my posting a while back. It seems

like liquids would be excellent (as far as construction is concerned) for

capacitors with extremely small (liquid) dielectrics.

>I recall some years back when the excimer laser was developed, Popular

>Science disclosed that it used a high voltage water capacitor; so there

>must be a way for it to work. I always wondered how they got around the

>breakdown voltage problem. It supplied very short pulses. Maybe it

>discharged before breakdown.

...and Mike Hammer added:

>The problem with water is that it must be absolutely pure to

>even come close to being an insulator. And that level of purity

>would be nearly impossible to maintain for long as water is also

>an excellent solvent.

>

>Distilled water isnt nearly pure enough for the purpose.

What about other liquids? Alcohol, benzene, toluene, xylene, ammonia, etc.

etc. etc. What chemical structure(s) make a good insulator?

Daniel Lockhart

Date: Mon, 14 Sep 1998 15:49:16 -0600

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Daniel Lockhart <molly@cmn.net>

Subject: A Good Catch at the Electric Company...

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

Hey all,

...no, unfortunately it wasn't a pole pig (I actually wouldn't want one

anyway).

I went in asking (i.e. begging) for a ceramic insulator like the ones that

were attached to power transformers. They looked at me real funny and said

they'd go see what they had. To make a short story shorter, I ended up with

a beautiful 5.5-inch diameter, eight inch long ceramic insulator that

weighs what seems like fifty pounds. They said it came of off a switch from

an electrical substation, and from the size of the insulator, it was one

seriously big switch!

I'm planning on using it as an insulator between my secondary and toroid.

(I just hope it isn't too high! Eight inches?). Nothing big and neat, but a

good catch, seeing as I got it for free.

Daniel Lockhart

Date: Mon, 14 Sep 1998 20:08:25 -0500

From: Bert Hickman <bert.hickman@aquila.com>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Organization: Stoneridge Engineering

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Flat Stack Cap Death

Sender: tesl-err@emachine.com

Tom Polk wrote:

<SNIP>

> I recall some years back when the excimer laser was developed, Popular

> Science disclosed that it used a high voltage water capacitor; so there

> must be a way for it to work. I always wondered how they got around the

> breakdown voltage problem. It supplied very short pulses. Maybe it

> discharged before breakdown.

Tom,

Exactly! As long as the pulses are kept short (<1 uSec), pure deionized

water behaves just like a high k (k=80) dielectric. This technique is

used to make very low impedance, high power transmission lines for very

high power pulse work at places like Lawrence Livermore, Sandia, etc.

In many of these applications, the water is also used to safely

dissipate left-over energy, sometimes very spectacularly. For an example

of this, check out the picture of water breaking down from left over

energy AFTER a high energy (50 Terrawatt) Z-Pinch experiment at Sandia's

Pulsed Power site at:

http://www.sandia.gov/LabNews/LN02-14-97/pbfaz.html

AWESOME!

-- Bert --

X-Originating-IP: [208.16.145.182]

From: "Bill the arcstarter" <arcstarter@hotmail.com>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Flat Stack Cap Death

Date: Mon, 14 Sep 1998 19:57:43 PDT

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

Coilers,

What is old is new again.  About once every 6 months the same

(excellent) topics come back up!

I wonder why this is?  Do most people leave after about that much time?

I'm not flaming anyone at all, just rambling a bit...  The liquid

capacitor is one of such recurrent topics.  Back in Feb of '98 I posted

such ideas, expecially alcohol based capacitors...

Daniel wrote:

>What about other liquids? Alcohol, benzene, toluene, xylene, ammonia, etc.

>etc. etc. What chemical structure(s) make a good insulator?

Check out my page for some related silliness:

http://www.geocities.com/CapeCanaveral/Hangar/6160/teslacurrent.html

-Bill the arcstarter

Starting arcs in Cinci, OH

http://www.geocities.com/CapeCanaveral/Hangar/6160

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Mike Is Right !!

X-Mailer: IPAD 2.0

From: break7@juno.com (Tyla A. Break)

Date: Mon, 14 Sep 1998 23:49:02 EDT

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Mike, thanks for info. You're right , before the cap died it was

glowing blue all over. It only had 3 quarts of oil, no biggie got 17

more.  Autopsy will begin shortly cap has been drip - draining for an

hour.  The oil just got some little smoke strings in it as I was watching

it very closely and shut down immediatly.  I hope my sheets aren't

"Hammered " like Mike's. I will post autopsy results shortly!

If it's not dangerous it's not fun!  Kyle  break7@juno.com

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Insulators

X-Mailer: IPAD 2.0

From: break7@juno.com (Tyla A. Break)

Date: Tue, 15 Sep 1998 00:00:02 EDT

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Daniel, Where did you get your insulator?  I can get loads of

them from DWP salvage . Usually the problem is they're really huge!  Some

are 12" Dia. x 10 feet long!

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Autopsy  Complete !

X-Mailer: IPAD 2.0

From: break7@juno.com (Tyla A. Break)

Date: Tue, 15 Sep 1998 01:42:17 EDT

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Well, every thing Mike said would happen did.

Every plate except the top and bottom had corona - burn around the

outside!  I was quite amazed that many plates were dry in the middle. The

cap was submersed in oil for two days and had been energized with 7500

volts for 30 minutes and also been used in a TC circuit. The actual cause

of failure was only two plates , but I can see that all the plates were

going to fail very shortly. The burn around the plates was small and

could be wiped off all the  poly sheets ,even the two bad ones. I suspect

they're damaged in ways I can't see so I'll toss those. As for the

hammered sheets I only had one . Right on the tongue connection there is

what looks like six center - punch marks almost all the way through.

As for the flammability of Diala AX transformer oil, when I got

it the " safety diamond" for flammability had a zero. I tried a few

experiments in true Arc Starter fashion. I poured the oil on a flame - no

dice. Next water - proof Cannon fuse passing through oil.... Very smokey

but no cigar!  (Next time you're in a Get Smart situation, tied to a keg

of FFG just make sure you have some Diala oil., The old oil up the sleeve

trick) Finally an oil soaked rag with a flame under it did the trick. I

had to wait for it to heat up and start crackling before it would ignite.

Not really impressive.  Hard to tell if the rag was burning or the oil.

But this doesn't mean you shouldn't have a fire-extinguisher handy at all

times. Other oils might be very flammable!

If it's not dangerous it's not fun!  Kyle  break7@juno.com

X-Originating-IP: [199.217.140.22]

From: "Bill the arcstarter" <arcstarter@hotmail.com>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Autopsy Complete !

Date: Tue, 15 Sep 1998 07:28:45 PDT

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

Kyle wrote:

>	As for the flammability of Diala AX transformer oil, when I got

>it the " safety diamond" for flammability had a zero. I tried a few

>experiments in true Arc Starter fashion. I poured the oil on a flame -

Excellent!  I'm surprised someone remembered my flammability tests...

>no dice. Next water - proof Cannon fuse passing through oil.... Very smoky

>but no cigar!  (Next time you're in a Get Smart situation, tied to a keg

>of FFG just make sure you have some Diala oil., The old oil up the sleeve

>trick) Finally an oil soaked rag with a flame under it did the trick. I

>had to wait for it to heat up and start crackling before it would ignite.

OK - so you were able to get it to burn.  I guess I didn't try too hard,

but I learned what I wanted to know - ie - a "casual" ignition source

won't ignite the stuff.

Sure beats using gasoline as a dielectric! :)  (no -don't try this, kids!)

I'm told the old pure PCB xfmr oils positively won't burn.  Can anyone

verify this as a possible identification test?  I'm still thinking about

those 5KVA "bucket-pigs" (spams?) at the yard... :)  I could extract

some oil ala turkey baster style, soak a hunk of asbestos with it, and

hit it with the propane torch...  If it doesn't burn then I don't want

it, right?

Kyle - how much did you pay for your diala-ex??

-Bill

From: "Basura, Brian" <brian.basura@unistudios.com>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: RE: Autopsy Complete !

Date: Tue, 15 Sep 1998 08:10:05 -0700

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Bill,

I just picked up 5 gallons of Diala-AX at my local Shell jobber and the cost

was only $24.95.  Might want to bring your own container as they were going

to charge me extra for the bucket until they found some pre-packaged in

5-gallon quantities.

Brian D. Basura

> ----------

> From: 	Bill the arcstarter[SMTP:arcstarter@hotmail.com]

> Reply To: 	Tesla-2 Mailing List

> Sent: 	September 15, 1998 7:28 AM

> To: 	Tesla-2 Mailing List

> Subject: 	Re: Autopsy Complete !

>

<< Big Snip>>

> Kyle - how much did you pay for your diala-ex??

>

> -Bill

>

>

Date: Tue, 15 Sep 1998 10:53:55 -0600

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Daniel Lockhart <molly@cmn.net>

Subject: Re: Insulators

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

>	Daniel, Where did you get your insulator?  I can get loads of

>them from DWP salvage . Usually the problem is they're really huge!  Some

>are 12" Dia. x 10 feet long!

I got mine at the local electric company. Somebody went "downstairs in the

shop" and brought it up to the office I was in. I asked if they had a scrap

yard or something of the sorts. They said they did, but it was all trash

and I didn't want anything there. I might try asking someone else.

Twelve inches by ten feet, huh? That sounds about right for my next coil...

:-)

Daniel Lockhart

Date: Tue, 15 Sep 1998 10:53:24 -0700

From: tomstock <tomstock@geocities.com>

X-Mailer: IPAD 2.0

X-Accept-Language: en

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Rebuilding Neons (oven method?)

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Hi everyone...I'm trying to rebuild a neon. I've used the method of

chipping out the tar successfully, but finding a container for the naked

transformer is a little bit of a challenge, and chipping and cleaning is

messy and slow going. Also, windings usually end up slightly damaged,

and more winds than desired end up being removed.

Is it possible to heat a Jefferson 15kv transformer on a hot plate until

the tar is soft enough to be poured out? I haven't yet successfully been

able to get the tar hot enough to pour. I've tried several times using

the oven, only to have tar expand,deform the case, and bubble out all

over the pan. This didn't do anything except make a big mess, and the

tar was still way too solid to be removed.

If I could get the tar to flow it would make things a lot easier!  I have

2 more 15kv Neons waiting to be added to my coil (currently 15kv/60ma),

and I'd like to place them back into their original case and remove as

little as possible of the secondary windings.

-tom

http://www.geocities.com/CapeCanaveral/1348/tesla.html

X-Originating-IP: [199.217.140.22]

From: "Bill the arcstarter" <arcstarter@hotmail.com>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Rebuilding Neons (oven method?)

Date: Tue, 15 Sep 1998 12:26:27 PDT

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

Tom Stock wrote:

>If I could get the tar to flow it would make things alot easier!  I have

>2 more 15kv Neons waiting to be added to my coil (currently 15kv/60ma),

>and I'd like to place them back into their original case and remove as

>little as possible of the secondary windings.

I've never tried it - but one somewhat popular technique was as follows:

1) Place the NST in the freezer for a day or two.  Depending on your

home situation, you may have to "disguise" the NST by placing it in an

ice cream box, etc to prevent your wife from throwing a fit when she

sees that dirty oily thing in the 'fridge! :)

2) Plug in the NST for about 1/2 to 1 hr.  This warms the core and

softens the tar near the core.  Possibly someone was feeding the core

excessive voltage to increase the heating...?

3) Chip the still frozen tar from around the warmed core.

Another thought - perhaps you can get an old electric oven (from the

side of the road) and use it as a NST unpotting device.  I'd probably

build a rack which would hold the NST upside-down, or on edge.  Then I'd

heat it mildly (250-300 deg F ??) for some extended time - probably at

least a day.  The tar should run out and flow into some sort of catch

container under the NST...  Again - I'm just inventing this on the fly.

I certainly haven't tried it!

Personally I'd just freeze the NST and take it to the gun range to shoot

the tar off, but the windings might not like that...

-Bill

Date: Tue, 15 Sep 1998 14:03:24 -0700

From: Gene Waldo <gwaldo@1stnetusa.com>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Rebuilding Neons (oven method?)

Sender: tesl-err@emachine.com

tomstock wrote:

>

> Hi everyone...I'm trying to rebuild a neon.

> -tom

> http://www.geocities.com/CapeCanaveral/1348/tesla.html

Just saw your T page... Nice coil! Too much over-spray from top of

2ndary? Caps, too? SW & foil?

What is your safety Gap RFGnd? 3 bolts, min?

--

Gene Waldo -  gwaldo@1stnetusa.com  - "Igor in Residence"

---------------------------------------------------------------

What's good for the planet is more OZONE! See the machinery form up!

NEW PAGE!   http://members.1stnetusa.com/danmurphy/waldo.htm

X-Mailer: IPAD 2.0

Date: Tue, 15 Sep 1998 17:35:03 -0500

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Mike Hammer <mhammer@misslink.net>

Subject: Tom Stocks web page/ATM

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

At 02:03 PM 9/15/98 -0700, you wrote:

>tomstock wrote:

>>

>> Hi everyone...I'm trying to rebuild a neon.

>> -tom

>> http://www.geocities.com/CapeCanaveral/1348/tesla.html

>Just saw your T page... Nice coil! Too much over-spray from top of

>2ndary? Caps, too? SW & foil?

>What is your safety Gap RFGnd? 3 bolts, min?

>--

>Gene Waldo -  gwaldo@1stnetusa.com  - "Igor in Residence"

Tom,

I just took a look at your page also. Coils look nice.

I especially like the copper toilet tank float terminal.

Thats what I used on my first coil too. But as you already

know its not very effective at suppressing corona.

Looks like to me you need to build yourself a poly cap.

Also of considerable interest was the Telescope Making

section. I too am a glass pusher. Ive made several

including a 4.25 F12, a 6 inch F10, 10 F 5.6 and a 12.5 F 4.8.

Unfortunately the high cost of glass these days has kept me

from doing anything recently. I did have a friend give

me a mint condition Edmund Scientific 6 inch kit from

the early 60s. Everything is intact and in perfect shape.

Just might whip me up a 6 F4 one of these days.

If you want to talk ATM email me direct or someone

will blast me for starting an off topic thread. :)

TTYL!

Mike Hammer

mhammer@misslink.net

X-Mailer: IPAD 2.0

Date: Tue, 15 Sep 1998 17:50:37 -0500

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: Rebuilding Neons (oven method?)

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

At 10:53 AM 9/15/98 -0700, you wrote:

>

>Hi everyone...I'm trying to rebuild a neon. I've used the method of

>chipping out the tar successfully, but finding a container for the naked

>transformer is a little bit of a challenge, and chipping and cleaning is

>messy and slow going. Also, windings usually end up slightly damaged,

>and more winds than desired end up being removed.

Tom,

First thing you have to do is convince yourself that rebuilding

neons is actually fun. Once you get past that point everything

else is easy.

If you plan to fire your rebuilds dry you dont really need

a container for it. Ive used them with the core sitting

right on the the floor. Of course if you want to go the

deluxe route can can build a case for them and put them in oil.

I built a custom tank for my 12kv/100ma rebuild from plywood.

The tank is leakproofed with 2 coats of polyester resin on the

inside. Its been over 6 months with no leaks.

Ive also used this type of tank for my plate caps.

Also dont worry too much about losing some secondary turns.

You can lose a couple hundred turns without affecting output

much at all. In fact when I reworked a 15kv neon down to 12kv

I had to remove 11 LAYERS of windings. Lots and lots of turns.

>

>Is it possible to heat a Jefferson 15kv transformer on a hot plate until

>the tar is soft enough to be poured out? I haven't yet successfully been

>able to get the tar hot enough to pour. I've tried several times using

>the oven, only to have tar expand,deform the case, and bubble out all

>over the pan. This didn't do anything except make a big mess, and the

>tar was still way too solid to be removed.

>

Yes this is possible but I dont recommend it. You will need

to get the tar up above 300 deg F. As much as 325 on some.

Problem with that much heat is that many neons have plastic

wedges between windings and core that would melt. Also the molten

tar tends to stick to the core and becomes difficult to remove.

I prefer to chisel away the tar until I can get to the core and

get it disassembled. Remove as much as possible then dump the

parts in separate containers of kerosene to remove the remaining

tar. Any way you go its a dirty messy job.

If you can arrange to freeze the tar block beforehand it will

break away quite readily. I can pretty much unpot a neon in

about 90 minutes.

Jefferson neons are not real easy to rebuild having a rather strange

core design and shunt arrangement. They can be done but its not

too much fun. I did two of them and would prefer not to do anymore.

Franceformers have a nice straightforward core design that lends

itself to an easy rebuild.

TTYL!

Mike Hammer

mhammer@misslink.net

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Diala AX

X-Mailer: IPAD 2.0

From: break7@juno.com (Tyla A. Break)

Date: Tue, 15 Sep 1998 19:33:34 EDT

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Bill, I just included flammability test because I've read some

warnings from other pages. They were really worried about fire. Doesn't

seem much of a problem.

They must like Brian over @ Shell Lubricating Services I paid $30

plus tax for 5 Gal.

If it's not dangerous it's not fun!  Kyle  break7@juno.com

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: R.E. Rebuilding Neons

X-Mailer: IPAD 2.0

From: break7@juno.com (Tyla A. Break)

Date: Tue, 15 Sep 1998 19:48:36 EDT

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

I talked to a guy that owns a transformer building business a

couple years ago. He told me that they have a liquid that you set

your trans. in and it de- pots it . I'll have to find out what it is.

If it's not dangerous it's not fun!  Kyle  break7@juno.com

Date: Tue, 15 Sep 1998 20:50:07 -0500

From: Tom Polk <tpolk@prismnet.com>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: R.E. Rebuilding Neons

Sender: tesl-err@emachine.com

Tyla A. Break wrote:

>

>         I talked to a guy that owns a transformer building business a

> couple years ago. He told me that they have a liquid that that you set

> your trans. in and it de- pots it . I'll have to find out what it is.

>

I've used MEK successfully. You can get the transformer to sparkle.

Takes at least 24 hours; it helps a lot if you agitate it somehow. But

first, chip out as much tar as you can. I used heavy chemical resistant

rubber gloves (a necessity) and a toothbrush to clean the secondaries.

You will need excellent ventilation, and stay away from sparks.

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Dissolving Your Secondary?

X-Mailer: IPAD 2.0

From: break7@juno.com (Tyla A. Break)

Date: Tue, 15 Sep 1998 23:19:47 EDT

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

I would think that some chemicals might dissolve the insulating

varnish. Does anybody know which ones would be a no no ?

If it's not dangerous it's not fun!  Kyle  break7@juno.com

Date: Tue, 15 Sep 1998 21:01:48 -0700

From: Gene Waldo <gwaldo@1stnetusa.com>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Dissolving Your Secondary?

Sender: tesl-err@emachine.com

Tyla A. Break wrote:

>

>         I would think that some chemicals might dissolve the insulating

> varnish. Does anybody know which ones would be a no no ?

>

Kyle,

My guess would be to avoid laquer thinners.  Most electrical enamels

should hold up to the kerosene and regular paint type thinners.

By all means try at test on small wire samples!! Especially look for

good sol rates on the tar.

I have test soaking as I write: 24ga mag wire in toluene. After 10 min.,

no softening of enamel. Tol should do a number on potting! Don't have

any of that available.  Years ago I used to bathe in Tol & Xylene! I

don't recommend that any more!  No problems- but now, all the OSHA crap

is out; And those panywaists will be banning DiHydrogenOxide one of

these days: a deadly commercial solvent! Many people die from exposure

to it every day!

Either Tol or Xyl should be OK for the enamel. Do use good gloves! It's

hell on drying the skin. heavy neoprene, not the wimpy little plastic jobs.

-------------------------------------

Gene Waldo -  gwaldo@1stnetusa.com  - "Igor in Residence"

---------------------------------------------------------------

What's good for the planet is more OZONE! See the machinery form up!

NEW PAGE!   http://members.1stnetusa.com/danmurphy/waldo.htm

Date: Tue, 15 Sep 1998 22:04:58 -0700

From: Gene Waldo <gwaldo@1stnetusa.com>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: LA Basin coilers: meet?

Sender: tesl-err@emachine.com

So far, we have 6 who have expressed interest: Dan Murphy, Gene Waldo,

Michael Baumann, Kyle Break, Eric Maxwell, Brain Basura.

This will be the last call for more of y'alls with an interest. In a few

days I expect we will have to plan decent time and place and agenda....

The geographic center of my map pins falls at about Whittier; but what's

there? Basically, the present scatter is from Sylmar to Riverside and

from Long Beach to Acton.

Any others with interest should send me your ZIP and any other data

and/or comments on the plan: maybe just a spit n' argue session over

dinner or what ever-- troop Apex? Griffith Park?

The plan is wide open for now. Join on!

Oh, fer them as don't know: Apex may be the premier retail gadgeteers

HEAVEN-salvage depot for 'anything' that was ever, or could be electric,

electronic, mechanical or hydraulic. And some stuff we can't classify!

>From li'l itsy-bitsy thingamjigs to full systems. You just get to run

rampant thru the stacks!

-----------------------------------------------------

Gene Waldo -  gwaldo@1stnetusa.com  - "Igor in Residence"

---------------------------------------------------------------

What's good for the planet is more OZONE! See the machinery form up!

NEW PAGE!   http://members.1stnetusa.com/danmurphy/waldo.htm

Sender: tesl-err@emachine.com

Date: Wed, 16 Sep 1998 09:17:58 +0000

From: Tom Stock <tomstock@geocities.com>

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Rebuilding Neons (oven method?)

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

G Just saw your T page... Nice coil! Too much over-spray from top of

> 2ndary? Caps, too? SW & foil?

> What is your safety Gap RFGnd? 3 bolts, min?

Thanks, the pictures of the caps and secondary with tiny terminal is the

first coil I built (very first...sitting on a milk crate!)...the one

below it is my second coil and is constructed somewhat differently

(uhem, more correctly). It too has saltwater caps, but it is a 12 pack,

all connected on top with one solid plate, and the bottom with one solid

plate (which contacts the foil around the bottles). The edges are sanded

smooth and there is no visible corona from the plates.  The safety gap

is just two 1/4 bolts. The chokes are some nice large donuts from the

back some picture tubes...with about 150 turns of #22 wire. I have some

rolled poly caps, but I haven't gotten around to pulling a vaccum on

them yet so they aren't being used.

I like my second coil. Everything is built into a little cart that I can

wheel around. I think I'm going to build a metal frame for it, and try

to make it a more compact unit so I can fire it up in parking lots

around the neighborhood (late at night of course) :)

Anyway, if I get some pictures of the second coil (below the primary)

I'll put them up.

-tom

Sender: tesl-err@emachine.com

Date: Wed, 16 Sep 1998 09:27:51 +0000

From: Tom Stock <tomstock@geocities.com>

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Tom Stocks web page/ATM

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Mike Hammer wrote:

>

> At 02:03 PM 9/15/98 -0700, you wrote:

> >tomstock wrote:

> >>

> >> Hi everyone...I'm trying to rebuild a neon.

> >> -tom

> >> http://www.geocities.com/CapeCanaveral/1348/tesla.html

> Tom,

> I just took a look at your page also. Coils look nice.

> I especially like the copper toilet tank float terminal.

> Thats what I used on my first coil too. But as you already

> know its not very effective at suppressing corona.

Hey guys...lay off the toilet ball float! That picture was my FIRST

coil....look further down and you'll see my second coil...the one with

the nice toroid, and better caps etc (of course I don't show that

part...no pics sorry). I don't use the float anymore..it was just to

show where I started and where I am now...same with the caps, the

primary, and the chokes...

The poly caps are built, but just sitting waiting for a vaccum pump!

> section. I too am a glass pusher. Ive made several

> including a 4.25 F12, a 6 inch F10, 10 F 5.6 and a 12.5 F 4.8.

> Unfortunately the high cost of glass the days has kept me

I just made a 10" F6 out of 1/2" plate glass. Total cost $2

The best part is that it will likely perform better than my 10" F4.5

Meade! ($695) I cut the glass from an old radar screen using a steel

washer and a turntable. Some people say that it won't hold figure

because it will deform when mounted. This is completely wrong! Figuring

it would be even harder than mounting it, and this was a breeze. Whoops

sorry about drifting off topic..Mike started it!

cya!

-tom

Sender: tesl-err@emachine.com

Date: Wed, 16 Sep 1998 09:30:55 +0000

From: Tom Stock <tomstock@geocities.com>

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: R.E. Rebuilding Neons

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Please do!! This could save us all a whole lot of time!

(It's probably paint thinner)

-tom

Tyla A. Break wrote:

>

>         I talked to a guy that ownes a transformer building bussines a

> couple years ago. He told me that they have a liquid that that you set

> your trans. in and it de- pots it . I'll have to find out what it is.

>         If it's not dangerous it's not fun!  Kyle  break7@juno.com

>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Date: Wed, 16 Sep 1998 05:52:02 -0700

From: "paul m" <fleischwulf@mailexcite.com>

Cc:

X-Sent-Mail: off

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Expiredinmiddle: true

X-Mailer: IPAD 2.0

Subject: RF Line filter

X-Sender-Ip: 208.153.7.9

Organization: MailExcite  (http://www.mailexcite.com:80)

Sender: tesl-err@emachine.com

Coilers,

I have been using a 37 amp RF filter between the primary of my pole pig and the main power source. I have had this configuration for almost a year. On Monday my gap stopped firing. I went through the circuitry very carefully. I finally tracked the problem to the RF line filter. It was very hot and under power it actually sizzled. I removed the line filter and reconnected the primary. The arcs returned to normal. I noticed that a lamp

cord located just above the transformer began to arc over to an over head pipe. The pipe had taken a 60" streamer strike. I have never seen this before. I ran the coil again for

a few seconds, pow, (same place between the cord and the pipe) this time there were traces of molten metal in the arc. The coil is running at 3.5 - 4 Kw. Has anyone ever experienced a similar problem ? Could the RF feed back have burned out the filter ? Coil operations have been indefinitely suspended !

PS: Any and all suggestions are welcome

paul m

---

Einigheit und Wahrheit

X-Originating-IP: [199.217.140.22]

From: "Bill the arcstarter" <arcstarter@hotmail.com>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: R.E. Rebuilding Neons

Date: Wed, 16 Sep 1998 06:08:15 PDT

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

It was written:

>	I talked to a guy that ownes a transformer building bussines a

>couple years ago. He told me that they have a liquid that that you set

>your trans. in and it de- pots it . I'll have to find out what it is.

Search the archives for something called "D-Limonine" or something like

this.  Supposedly this is some sort of purified citrus oil which

dissolves tar, etc.

I don't know where to get it.

-Bill

X-Originating-IP: [207.100.20.13]

From: "Philip Mac Duffie" <punk_42@hotmail.com>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Shell Diala X

Date: Wed, 16 Sep 1998 12:48:53 PDT

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

Ok guys, youve lost me all together. What is the differance between the

shell Diala oils? Where the heck can I get it? The power co. refuses to

give me a burned up pig to get it from :(

Thanks,

Philip Mac Duffie

From: "Basura, Brian" <brian.basura@unistudios.com>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: RE: Shell Diala X

Date: Wed, 16 Sep 1998 13:36:56 -0700

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Philip,

Here is Shell's page which does a good job describing the oil:

http://www.shellcan.com/clubes/3-17-04.htm

Their 800 number will hook you up with a distributor in your area.

Brian

> ----------

> From: 	Philip Mac Duffie[SMTP:punk_42@hotmail.com]

> Reply To: 	Tesla-2 Mailing List

> Sent: 	September 16, 1998 12:48 PM

> To: 	Tesla-2 Mailing List

> Subject: 	Shell Diala X

>

>

> Ok guys, you've lost me all together. What is the difference between the

> Shell Diala oils? Where the heck can I get it? The power co. refuses to

> give me a burned up pig to get it from :(

>      Thanks,

>  Philip Mac Duffie

>

From: Dsurfr@aol.com

Date: Wed, 16 Sep 1998 19:49:12 EDT

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Safety gap activity

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Hi All: As usual I'm looking for more free advice. While I'm saving up to buy

the poly for a new cap I built another SW but this time they are immersed in a

salt water solution topped off w/ motor oil, rather than dry, foil wrapped.

From the numerous postings and photos on the lists I expected to see a lot of

corona loss but I don't seem to be getting any at all. I ran my coil in total

darkness last night (except for the two fluorescent tubes over my work bench

that kept lighting up quite a bit) & saw nothing emanating from the bottles or

the tub. The spark output from the toroid was much more active & my safety gap

is also arcing quite a bit more (which worries me a lot) Here are the specs on

my coil to date:

Power Source: 15KV 30mA output "Franceformer" neon transformer. (120V 60 Hzinput)

Safety Gap: (2) 1 1/2" x 1/4" brass bolts w/ brass acorn nuts spaced +/- 3/8" apart

RF Chokes: (2) 1 1/2" x 6 1/2" x 1/8" acrylic tubes w/ 24GA. magnet wire close

wound 4 1/2"L (+/- 200 turns

Main Gap: (12) 3/4" x 3" copper pipes spaced +/- .025" mounted on a sheet of

1/4" plexi @ a 15 degree slope w/ muffin fan at bottom. Currently tapped at

10th pipe ( 9 gaps for a total of +/- .225")

Capacitor: (6) 40 OZ. beer bottles w/  1/4" x 8"L carriage bolt electrodes,

filled to 7" height w/ salt water & topped off w/  5W30 motor oil. Plastic

caps from 16 OZ. Coke bottles used to seal off bottles. Bottles placed in a 13

1/2" x 8 1/2" x 9"H plastic tub filled w/ 7" of salt water & topped off w/

more oil. Rating per bottle:+/- .00098 mf. Bottles connected in parallel for

total rating of +/- .0059 mf.

Primary Coil: Flat "pancake" shape, 18 turns of 1/4" coaxial cable, 6 1/2"

I.D. mounted parallel w/ bottom turn of secondary. Currently tapped @ 16 1/2

turns. (I know it isn't a very good choice of materials, but I am

experimenting with shapes & lengths & it seems to work quite well!)

Secondary Coil: 4 1/2" O.D. x 1/4" thick x 25" L. acrylic tube wound w/ 24 GA.

magnet wire. 22" L. winding (close wound)  (+/- 1000 turns) First turn 1 1/2"

above base.

My question is what can I do to decrease the activity across my safety gap?

I'm getting 16"+ strong, white arcs to a ground & many (12+) 6" - 8" max.

purple arcs to air off the totoid at the same time. The safety sparks almost

constantly when this occurs (after a short "build-up" time) My primary gap

seems quite low for 15kV 30mA & the safety seems about max. Do I need more top

toroid or what?

(I have (3) 4' pieces of 3/4" copper pipe for a ground also) Sorry for so long

a post. Thanks in advance for the help.  Jim

From:          "Malcolm Watts" <MALCOLM@directorate.wnp.ac.nz>

Organization:  Wellington Polytechnic, NZ

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Date:          Thu, 17 Sep 1998 12:31:03 +1200

Subject:       Re: Safety gap activity

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Hi Jim,

I'll suggest a few things to try as I don't have all the

answers. I cannot guarantee that your transformer will survive but

its survival sounds as if its in doubt with that safety gap setting

anyway. Try at your own risk.

> From:           Dsurfr@aol.com

> Date sent:      Wed, 16 Sep 1998 19:49:12 EDT

> To:             "Tesla-2 Mailing List" <tesla-2@emachine.com>

> Subject:        Safety gap activity

> Send reply to:  "Tesla-2 Mailing List" <tesla-2@emachine.com>

> Hi All: As usual I'm looking for more free advice. While I'm saving up to buy

> the poly for a new cap I built another SW but this time they are immersed in a

> salt water solution topped off w/ motor oil, rather than dry, foil wrapped.

> From the numerous postings and photos on the lists I expected to see a lot of

> corona loss but I don't seem to be getting any at all. I ran my coil in total

> darkness last night (except for the two fluorescent tubes over my work bench

> that kept lighting up quite a bit) & saw nothing eminating from the bottles or

> the tub. The spark output from the toroid was much more active & my safety gap

> is also arcing quite a bit more (which worries me a lot) Here are the specs on

> my coil to date:

Judging by your description of the output, cap quality seems somewhat

better perhaps reflecting a lack of corona losses.

> Power Source: 15KV 30mA output "Franceformer" neon transformer. (120V 60 Hz input)

> Safety Gap: (2) 1 1/2" x 1/4" brass bolts w/ brass acorn nuts spaced +/- 3/8"  apart

> RF Chokes: (2) 1 1/2" x 6 1/2" x 1/8" acrylic tubes w/ 24GA. magnet wire close

> wound 4 1/2"L (+/- 200 turns

The first thing I would try *BRIEFLY* is to leave the chokes out and

observe the safety gap operation. A second thing I would check is

shortening the leads between the transformer and the main gap. How

long is the run from the NST secondary to the main gap?

> Main Gap: (12) 3/4" x 3" copper pipes spaced +/- .025" mounted on a sheet of

> 1/4" plexi @ a 15 degree slope w/ muffin fan at bottom. Currently tapped at

> 10th pipe ( 9 gaps for a total of +/- .225")

*NOTE*: your safety gap is firing at a considerably higher voltage

than the main gap. What does that suggest? Anyone?

> Capacitor: (6) 40 OZ. beer bottles w/  1/4" x 8"L carriage bolt electrodes,

> filled to 7" height w/ salt water & topped off w/  5W30 motor oil. Plastic

> caps from 16 OZ. Coke bottles used to seal off bottles. Bottles placed in a 13

> 1/2" x 8 1/2" x 9"H plastic tub filled w/ 7" of salt water & topped off w/

> more oil. Rating per bottle:+/- .00098 mf. Bottles connected in parallel for

> total rating of +/- .0059 mf.

> Primary Coil: Flat "pancake" shape, 18 turns of 1/4" coaxial cable, 6 1/2"

> I.D. mounted parallel w/ bottom turn of secondary. Currently tapped @ 16 1/2

> turns. (I know it isn't a very good choice of materials, but I am

> experimenting with shapes & lengths & it seems to work quite well!)

> Secondary Coil: 4 1/2" O.D. x 1/4" thick x 25" L. acrylic tube wound w/ 24 GA.

> magnet wire. 22" L. winding (close wound)  (+/- 1000 turns) First turn 1 1/2"

> above base.

>

> My question is what can I do to decrease the activity across my safety gap?

> I'm getting 16"+ strong, white arcs to a ground & many (12+) 6" - 8" max.

> purple arcs to air off the totoid at the same time. The safety sparks almost

> constantly when this occurs (after a short "build-up" time) My primary gap

> seems quite low for 15kV 30mA & the safety seems about max. Do I need more top

> toroid or what?

Several things here:

(1) try adjusting primary tune (primary coil taps) and note

difference on performance and safety operation (if any).

(2) try adjusting coupling and again note performance and safety gap

operation. Does the safety gap fire when an arc latches onto

something or do it mostly/exclusively happen with air streamers?

It is worth adjusting primary tapping each time you adjust coupling.

Does this make a difference to the safety gap?

(3) Omit the chokes and stick in a R-C filter. What happens now?

(4) Try adjusting the topload *and* retuning/adjusting coupling each

time. Try both larger and smaller toploads.

> (I have (3) 4' pieces of 3/4" copper pipe for a ground also) Sorry for so long

> a post. Thanks in advance for the help.  Jim

I would appreciate it if you post the results. Personally I think

that continued operation like the coil is at the moment will result

in its eventual failure. I admit I have a vested interest in learning

what happens when the different things I've mentioned are tried.

Malcolm

X-Mailer: IPAD 2.0

Date: Thu, 17 Sep 1998 01:43:39 +0100

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: bob golding <yubba@clara.net>

Subject: ozone

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

HI All,

I have been tuning my coil and adjusting it a lot this week, and have

noticed that I am getting headaches which last about 8 hours. I am pretty

sure they are being caused by ozone build up as I don't have much

ventilation yet. I am planning to put in a extractor fan sometime, but

until then I was going to use a face mask. does anyone know what sort of

filter i should use?, also will I get toxic fumes from the outlet of the

vacuum cleaner i am using on my gap?

Thanks for all the help so far,

bob golding

X-Mailer: IPAD 2.0

Date: Wed, 16 Sep 1998 20:49:08 -0500

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: Safety gap activity

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

At 12:31 PM 9/17/98 +1200, you wrote:

>Hi Jim,

>        I'll suggest a few things to try as I don't have all the

>answers. I cannot guarantee that your transformer will survive but

>its survival sounds as if its in doubt with that safety gap setting

>anyway. Try at your own risk.

>

<SNIP>

>The first thing I would try *BRIEFLY* is to leave the chokes out and

>observe the safety gap operation. A second thing I would check is

>shortening the leads between the transformer and the main gap. How

>long is the run from the NST secondary to the main gap?

>

>> Main Gap: (12) 3/4" x 3" copper pipes spaced +/- .025" mounted on a sheet of

>> 1/4" plexi @ a 15 degree slope w/ muffin fan at bottom. Currently tapped at

>> 10th pipe ( 9 gaps for a total of +/- .225")

>

>*NOTE*: your safety gap is firing at a considerably higher voltage

>than the main gap. What does that suggest? Anyone?

>

Sounds like his RF chokes are ringing to me. I usually

dont recommend air core chokes unless there is no other choice.

Jim are you using some series damping resistors with those chokes?

If so what value and are they running hot or cold?

If not I would add some right away to settle those chokes down.

<MORE SNIP>

TTYL!

Mike Hammer

mhammer@misslink.net

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Paul's Smokey Link

X-Mailer: IPAD 2.0

From: break7@juno.com (Tyla A. Break)

Date: Thu, 17 Sep 1998 00:48:22 EDT

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Hi Fleischwulf, just read about your problem. Too bad about your

filter (smokey link ). What is the free length of that cord.? Could it be

in tune with an elusive harmonic the kind that lets the smoke out of

you're line filter? Could you get your scope and hook it up with a 100

meg probe and find that Hz ? I'm not an E. E. I'm just guessing. Also ,

What does that line filter consist of ? It seems to have in it exactly

what you need in it . It must have been damping something to get that hot

! Maybe we can copy it and make consumable filters if they last a while.

I don't mind burning resistors or cheap chokes if it saves caps and

neons.

If it's not dangerous it's not fun!  Kyle  break7@juno.com

From:          "Malcolm Watts" <MALCOLM@directorate.wnp.ac.nz>

Organization:  Wellington Polytechnic, NZ

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Date:          Thu, 17 Sep 1998 17:03:01 +1200

Subject:       Re: RF Line filter

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Hi Paul,

Unfortunately my mailer truncated your lines to 255 chrs

so I missed some of this.

> To:             "Tesla-2 Mailing List" <tesla-2@emachine.com>

> Date sent:      Wed, 16 Sep 1998 05:52:02 -0700

> From:           "paul m" <fleischwulf@mailexcite.com>

> Copies to:

> Send reply to:  "Tesla-2 Mailing List" <tesla-2@emachine.com>

> Subject:        RF Line filter

> Organization:   MailExcite  (http://www.mailexcite.com:80)

> Coilers, I have been using a 37 amp RF filter between the primary

> of my pole pig and the main power source. I have had this

> configuration for almost a year. On Monday my gap stopped firing.

>

> the problem to the RF line filter. It was very hot and under

> power it actually sizzled. I removed the line filter and

> reconnected the primary. The arcs returned to normal. I noticed

> that a lamp co

> arc over to an over head pipe. The pipe had taken a 60" streamer

> strike. I have never seen this before. I ran the coil again for a

> few seconds, pow, (same place between the cord and the pipe) this

> The coil is running at 3.5 - 4 Kw. Has anyone ever experienced

> a simular problem ? Could the RF feed back have burned out the

> filter ? Coil operations have been indefinatly suspended !

>

> PS: Any and all suggestions are welcome

Was there anything physically different about the setup on this

occasion?  Had your secondary ground dried out or become disconnected?

Do you have more info on the actual grounding arrangements?

Thanks,

Malcolm

Sender: tesl-err@emachine.com

Date: Thu, 17 Sep 1998 09:48:39 +0000

From: Tom Stock <tomstock@geocities.com>

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Coilers in FL?

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Any coilers out there in FL want to get together for a day?

I'm in the St. Pete/Tampa area...

-tom

http://www.geocities.com/CapeCanaveral/1348

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Date: Thu, 17 Sep 1998 04:07:42 -0700

From: "paul m" <fleischwulf@mailexcite.com>

Cc:

X-Sent-Mail: off

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

Subject: Re: RF Line filter

X-Sender-Ip: 208.153.7.9

Organization: MailExcite  (http://www.mailexcite.com:80)

Sender: tesl-err@emachine.com

Malcolm,

The grounding connections are identical to all previous runs. The line filter was

rated at 37 amps, so it remains a mystery to me.

Thanks for responding,

paul m

---

Einigheit und Wahrheit

X-Originating-IP: [199.217.140.22]

From: "Bill the arcstarter" <arcstarter@hotmail.com>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: ozone

Date: Thu, 17 Sep 1998 05:24:51 PDT

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

Bob wrote:

>until then I was going to use a face mask. does anyone know what sort of

>filter i should use?, also will I get toxic fumes from the outlet of the

>vacuum cleaner i am using on my gap?

Yes you will.  In fact I think the gaps make most of the ozone anyway...

If you have enough hose - you could route the vac's exhaust outside a

window or something...

-Bill

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Date: Thu, 17 Sep 1998 06:34:16 -0700

From: "paul m" <fleischwulf@mailexcite.com>

Cc:

X-Sent-Mail: off

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

Subject: Re: ozone

X-Sender-Ip: 208.153.7.9

Organization: MailExcite  (http://www.mailexcite.com:80)

Sender: tesl-err@emachine.com

Actually the gaps help produce a much more toxic gas, Nitrogen dioxide and also some nitrous oxide. The good news is that it doesn't produce much.

paul m

---

Einigheit und Wahrheit

On Thu, 17 Sep 1998 05:24:51   Bill the arcstarter wrote:

>Bob wrote:

>

>>until then I was going to use a face mask. does anyone know what sort of

>>filter i should use?, also will I get toxic fumes from the outlet of the

>>vacuum cleaner i am using on my gap?

>

>Yes you will.  Infact I think the gaps make mosts of the ozone anyway...

>If you have enough hose - you could route the vac's exhaust outside a

>window or something...

>

>-Bill

>

>

X-Originating-IP: [199.217.140.22]

From: "Bill the arcstarter" <arcstarter@hotmail.com>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: ozone

Date: Thu, 17 Sep 1998 07:00:40 PDT

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

Paul wrote:

>Actually the gaps help produce a much more toxic gas, Nitrogen dioxide

>and also some nitrous oxide. The good news is that it doesn't produce much.

Someone on the web-ring has a pic of a stove-bolt gap inside a sealed

pickle jar.  The jar is filled with a yellow-brown gas, which is

apparently NOx compounds produced from air via the arcing.

I can't quite remember who's pic this is.  Chris someone from the UK? He

did a lot of DC coil work...  (sorry - don't have time to find the site)

-Bill

X-Originating-IP: [195.121.16.216]

From: "Sam Barros" <sambarros@hotmail.com>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Ozone and (almost) all you ever wanted to know about HV-produced Gas safety!

Date: Thu, 17 Sep 1998 09:09:43 PDT

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

HI All,

>I have been tuning my coil and adjusting it a lot this week, and have

>noticed that I am getting headaches which last about 8 hours. I am pretty

>sure they are being caused by ozone build up as I don't have much

>ventilation yet. I am planning to put in a extractor fan sometime, but

>until then I was going to use a face mask. does anyone know what sort of

>filter i should use?, also will I get toxic fumes from the outlet of the

>vacuum cleaner i am using on my gap?

>

>Thanks for all the help so far,

>

>bob golding

Bob,

The ozone gas is produced by the streamers ionizing the oxygen in the

air and changing it into ozone. It has a slightly sweet, pleasant smell

and will go through your system in the same way oxygen would (i.e. through

the bloodstream and into all your organs, including your brain).

It is a powerful oxidiser and so inhaling large amounts of it for a

large time could be bad for you. Nevertheless, ozone therapy is used

quite often and it does oxygenate your body, so, in small quantities,

ozone is actually good for you. That is why some people buy

"ozonizers"...

In the other hand, your spark gap will be providing the air with enough

heat to chemically combine the oxygen and the nitrogen, forming nitrogen

oxides. From those there is N2O, which has a very sweet taste and is a

widely used anaesthetic: "Laughing gas", and NO, which is also emitted

by cars exhaust and is VERY toxic, combining with moisture in your lungs

to form nitric acid and also being processed by your system, acting as a

poison. It has a very unpleasant smell.

That goes without mentioning the copper vapour and copper oxide, both

of which are produced by the gap and are also unhealthy to inhale.

Now, Ozone and Nitrogen oxides are gases, and not even the best mask in

the world is going to protect you from it. Masks are made to protect you

against dust, or in the more sophisticated models, vapours, but NOT

gases... Only an oxygen tank would protect you from inhaling those gases...

Maybe you should try shielding your sparkgap and get some ventilation

for your room. But your headaches could also be caused by the noise, as

I don't know of anyone else on this list who has suffered ill effects

from inhaling ozone and nitrogen oxides in the small amounts produced by

Tesla Coils.

I have personally inhaled considerable amounts of those gases (during

capacitor discharge or high voltage experiments) and nothing happened to

me (even though the concentration was high enough to be smelled)...

Questions, comments and criticism are welcome!

Sam.

From: Dsurfr@aol.com

Date: Thu, 17 Sep 1998 12:14:37 EDT

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Safety gap activity

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Mike; I know you've told me this before but what size resistors should I be

using & do they go between the choke & the safety gap or the main gap or both?

Thanks,  Jim

X-Originating-IP: [195.121.16.216]

From: "Sam Barros" <sambarros@hotmail.com>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: ozone

Date: Thu, 17 Sep 1998 09:16:10 PDT

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

>>until then I was going to use a face mask. does anyone know what sort of

>>filter i should use?, also will I get toxic fumes from the outlet of the

>>vacuum cleaner i am using on my gap?

Not fumes: Gases. Nitrogen oxides more specifically. Not too much

though...

Sam.

X-Originating-IP: [195.121.16.216]

From: "Sam Barros" <sambarros@hotmail.com>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: ozone

Date: Thu, 17 Sep 1998 09:20:17 PDT

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

>>Actually the gaps help produce a much more toxic gas, Nitrogen dioxide

>and also some nitrous oxide. The good news is that it doesn't produce

>much.

>

>Someone on the web-ring has a pic of a stove-bolt gap inside a sealed

>pickle jar.  The jar is filled with a yellow-brown gas, which is

>apparently NOx compounds produced from air via the arcing.

Ozone and most Nox are colourless. Wouldn't the brown colur be due to

metal vapour condensing on the sides of the jar?

Or maybe some metal oxide vapours moving around...

Sam.

From: "John Q. Public" <evmax@gte.net>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: R.E. Rebuilding Neons

Date: Thu, 17 Sep 1998 09:54:19 -0700

X-Mailer: IPAD 2.0

X-MimeOLE: Produced By Microsoft MimeOLE V5.00.0518.4

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

D-Limonine... AKA Fast Orange. Not an oil but a very gnarly

cleaner/water soluble solvent. The shop I used to work at switched

from "Safety Kleen" to "Citra Sol" (basically the same as Fast

Orange but in a mix, Like making orange Kool Aid) When the Citra Sol

was brand new, it did a decent job, but as it aged it lost potency

fast and caused some nasty infections that regular solvent never

did. (I know, use gloves. I preferred to get in and out quick). The

pumps and paint in my solvent tanks didn't like it much either. The

Fast Orange is probably your best bet as they sell it at most auto

parts stores. The hot ticket is to get the one gallon jug at

Pic-N-Sav for 2-3 dollars. For normal cleaning you're supposed to

cut it about 20-1 so soaking full strength would probably be worth

trying for under $5.00.

JQP

-----Original Message-----

From: Bill the arcstarter <arcstarter@hotmail.com>

To: Tesla-2 Mailing List <tesla-2@emachine.com>

Date: Wednesday, September 16, 1998 6:16 AM

Subject: Re: R.E. Rebuilding Neons

>It was written:

>> I talked to a guy that owns a transformer building business a

>>couple years ago. He told me that they have a liquid that you set

>>your trans. in and it de-pots it . I'll have to find out what it is.

>

>Search the archives for something called "D-Limonine" or something like

>this.  Supposedly this is some sort of purified citrus oil which

>dissolves tar, etc.

>

> I don't know where to get it.

>

>-Bill

>

X-Originating-IP: [199.217.140.22]

From: "Bill the arcstarter" <arcstarter@hotmail.com>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: ozone

Date: Thu, 17 Sep 1998 10:17:54 PDT

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

Sam wrote:

>>Someone on the web-ring has a pic of a stove-bolt gap inside a sealed

>>pickle jar.  The jar is filled with a yellow-brown gas, which is

>>apparently NOx compounds produced from air via the arcing.

>

>

> Ozone and most Nox are colourless. Wouldn't the brown colur be due to

>metal vapour condensing on the sides of the jar?

> Or maybe some metal oxide vapours moving around...

Don't know for sure.  I tried making some of this "yellow-brown gas"

once, using tungsten faced electrodes inside a clear plastic pipe.

Nothing much happened.  Then again I didn't run it for too long.

Perhaps the color was just copper dust?  Again, I was under the

impression that it was a gas which would blow out of the jar once opened

etc...

Anyway - whatever it is - that's what you're breathing! :)

-Bill

X-Mailer: IPAD 2.0

Date: Thu, 17 Sep 1998 12:53:54 -0500

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: Safety gap activity

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

At 12:14 PM 9/17/98 EDT, you wrote:

>Mike; I know you've told me this before but what size resistors shold I be

>using & do they go between the choke & the safety gap or the main gap or both?

>Thanks,  Jim

Jim use whatever power resistors you can find. The value is not

critical. Try anything from 300-1500 ohms and 20 or more watts.

The higher the value the more damping action they will provide

but higher values will also dissipate more of your charging current.

Ive had good luck with some 310 ohm 50 watt resistors I picked up

at a hamfest cheap. This might be a little too low in your case

since your chokes seem to be ringing pretty bad. Try something

around 500 ohms or a little more.

By the way did you take Malcolm's advice and BRIEFLY run your coil

without the chokes to see the effect that has on the safety gap firings?

I would be interested to know what the results of that are. Also

once you add the resistors let me know if they run hot or cold.

TTYL!

Mike Hammer

mhammer@misslink.net

Date: Thu, 17 Sep 1998 12:57:41 -0500

From: Eric Davidson <edavidson@icva.gov>

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Nitrogen oxides

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Hello all:

Been watching the messages about spark gap fumes for a while and decided

to jump in.  An electric discharge in air will form a number of

different compounds.  A hot arc favors the formation on various nitrogen

oxides. Nitric oxide (NO) is colorless and odorless, it reacts quickly

with oxygen to form the highly toxic nitrogen dioxide (NO2/N2O4), this

is the brown, odorous gas. It is doubtful the much nitrous oxide (N2O)

is formed, and if it is, its basically harmless. Nitrogen pentoxide

(N2O5) may also be formed, but its a solid.  The formation of ozone (O3)

is favored by a corona discharge. Ozone is also formed by the action of

short wave UV light on oxygen, and forms an equilibrium (2O3<-->3O2).

It is quite toxic, but is very reactive and unstable, and doesn't

persist.   It is also possible that small amounts of metal are

vaporized, but these would be quickly oxidized to the metal oxide, which

for the most part are pretty harmless (unless of course you are using

thallium, mercury, arsenic or some other highly unlikely metal in your

electrodes). The brown cloud that has been seen is not vaporized copper

but likely NO2.  And no, you can't form any metal carbonyls in a spark

gap. In my opinion, the sensible operation of a TC spark gap does not

constitute a health risk, at least not from any of the gases formed (I'd

be more concerned about the pole pig you are using to power it!).  I

wouldn't suggest running a large spark gap in a small, enclosed room for

long periods of time, but outside or in a well ventilated garage for the

several minutes a TC is running, is not a problem. Hope this helps.

--

Have a good day!

Eric Davidson

edavidson@icva.gov

X-Originating-IP: [195.121.16.210]

From: "Sam Barros" <sambarros@hotmail.com>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Gases

Date: Thu, 17 Sep 1998 11:35:36 PDT

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

>Don't know for sure.  I tried making some of this "yellow-brown gas"

>once, using tungsten faced electrodes inside a clear plastic pipe.

>Nothing much happened.  Then again I didn't run it for too long.

----- Umm... I guess maybe I should give it a try... I'll do it in 2

weeks, after I come back from Portugal. I'll try streamers, sparks, arcs

and high power discharges (from caps). I think I'll use copper

electrodes, since that's what normal spark gaps are made out off.----

>Perhaps the color was just copper dust?  Again, I was under the

>impression that it was a gas which would blow out of the jar once

opened

>etc...

>

>Anyway - whatever it is - that's what you're breathing! :)

----- Nice way to put it, but I think it might be only part of what you

are breathing... Maybe it's NO2. I remember getting some of that gas

when I made Nitric acid. It smelled aweful! It was more red than brown

though...----------

>Bill

----- Sam.

X-Originating-IP: [207.100.20.15]

From: "Philip Mac Duffie" <punk_42@hotmail.com>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Coilers in FL?

Date: Thu, 17 Sep 1998 13:19:27 PDT

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

Hey, Tom I live in the Daytona Beach area.

Philip Mac Duffie,

From:          "Malcolm Watts" <MALCOLM@directorate.wnp.ac.nz>

Organization:  Wellington Polytechnic, NZ

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Date:          Fri, 18 Sep 1998 08:47:37 +1200

Subject:       Re: ozone

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Nitrogen dioxide is a brown coloured gas. I think NO is colourless.

Malcolm

> From:           "Sam Barros" <sambarros@hotmail.com>

> To:             "Tesla-2 Mailing List" <tesla-2@emachine.com>

> Subject:        Re: ozone

> Date sent:      Thu, 17 Sep 1998 09:20:17 PDT

> Send reply to:  "Tesla-2 Mailing List" <tesla-2@emachine.com>

>

> >>Actually the gaps help produce a much more toxic gas, Nitrogen dioxide

> >and also some nitrous oxide. The good news is that it doesn't produce

> >much.

> >

> >Someone on the web-ring has a pic of a stove-bolt gap inside a sealed

> >pickle jar.  The jar is filled with a yellow-brown gas, which is

> >apparently NOx compounds produced from air via the arcing.

>

>

>  Ozone and most Nox are colourless. Wouldn't the brown colur be due to

> metal vapour condensing on the sides of the jar?

>  Or maybe some metal oxide vapours moving around...

>

>  Sam.

From:          "Malcolm Watts" <MALCOLM@directorate.wnp.ac.nz>

Organization:  Wellington Polytechnic, NZ

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Date:          Fri, 18 Sep 1998 08:49:58 +1200

Subject:       Re: Gases

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Hi Sam,

> From:           "Sam Barros" <sambarros@hotmail.com>

> To:             "Tesla-2 Mailing List" <tesla-2@emachine.com>

> Subject:        Gases

> Date sent:      Thu, 17 Sep 1998 11:35:36 PDT

> Send reply to:  "Tesla-2 Mailing List" <tesla-2@emachine.com>

> >Don't know for sure.  I tried making some of this "yellow-brown gas"

> >once, using tungsten faced electrodes inside a clear plastic pipe.

> >Nothing much happened.  Then again I didn't run it for too long.

>

>

> ----- Umm... I guess maybe I should give it a try... I'll do it in 2

> weeks, after I come back from Portugal. I'll try streamers, sparks, arcs

> and high power discharges (from caps). I think I'll use copper

> electrodes, since that's what normal spark gaps are made out off.----

I've found tungsten is a lot better. It doesn't ablate as readily as

copper and runs at a higher temperature. They're my preference anyway.

Malcolm

> >Perhaps the color was just copper dust?  Again, I was under the

> >impression that it was a gas which would blow out of the jar once

> opened

> >etc...

> >

> >Anyway - whatever it is - that's what you're breathing! :)

>

>

> ----- Nice way to put it, but I think it might be only part of what you

> are breathing... Maybe it's NO2. I remember getting some of that gas

> when I made Nitric acid. It smelled aweful! It was more red than brown

> though...----------

>

>

> >Bill

>

>

> ----- Sam.

From: Dsurfr@aol.com

Date: Thu, 17 Sep 1998 20:03:04 EDT

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: safety gap activity

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Malcolm, Mike & All: Thanks, as always, for the advice. Here is a list of the

results as requested: First the arc across the safety only occurs when there

is a discharge from the toroid (either to air or ground). My coil takes a

couple of seconds to build up a charge strong enough to force break out & this

gets stronger the longer it runs & so does the arc on the safety gap. (I have

no idea if this is normal or not) My safety gap is connected to the

transformer with an 8" length of 16 gauge insulated, stranded copper wire. The

chokes are connected with a 6" length of the same wire. They are mounted

vertically & spaced about 6" apart. They do not seem to heat up when running

but I don't run for more than 15 or 20 seconds at a time, shut it off for a

few seconds & then fire up again. It doesn't take as long to reach break out

point each time I do this so I assume there is some charge still in the tank

circuit or the air around the coil is still somewhat charged? I tried running

without the chokes (VERY BRIEFLY!!) The arc across the safety was outrageous

at break out!! Next I tried placing a 3"x 14" toroid on top of the 4"x17". It

took longer to reach break out & it came in the form of very short, sharp &

loud "shots" but the safety arced each time. Next I tried using the 3" toroid

only but got very little break out. I didn't re-tune much with both of these

changes however, since the safety still arced with discharge. Finally I tried

raising the secondary 1" & dropped down one turn on the primary. This seems to

solve the problem. I guess I was over coupled? I'm getting 14" strong arcs to

ground with no safety gap discharge. It forces the oil out of the cap tub a

bit but like I said its new & needs breaking in. I'll continue this approach

with more tuning & let you know the results. I was unable to find a quick

source for the resistors Mike recommended (Radio Shack etc.) but I can order

them from some of the catalogs I have. They seem to be about $6 to $9 dollars

each. Is this about right? Also do they go between the safety & the chokes or

between the chokes & the main gap, or both? Also I don't know what an R-C

filter is. Could you advise? Once again, thanks for the time,  Jim

From:          "Malcolm Watts" <MALCOLM@directorate.wnp.ac.nz>

Organization:  Wellington Polytechnic, NZ

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Date:          Fri, 18 Sep 1998 12:50:16 +1200

Subject:       Re: safety gap activity

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Hi Jim,

I want to thank you very much for taking the time and trouble.

what we are learning will go a long way to ridding any system of this

nuisance.

> From:           Dsurfr@aol.com

> Date sent:      Thu, 17 Sep 1998 20:03:04 EDT

> To:             "Tesla-2 Mailing List" <tesla-2@emachine.com>

> Subject:        safety gap activity

> Send reply to:  "Tesla-2 Mailing List" <tesla-2@emachine.com>

> Malcolm, Mike & All: Thanks, as always, for the advice. Here is a list of the

> results as requested: First the arc across the safety only occurs when there

> is a discharge from the toroid (either to air or ground). My coil takes a

> couple of seconds to build up a charge strong enough to force break out & this

> gets stronger the longer it runs & so does the arc on the safety gap. (I have

> no idea if this is normal or not) My safety gap is connected to the

> transformer with an 8" length of 16 gauge insulated, stranded copper wire. The

> chokes are connected with a 6" length of the same wire. They are mounted

> vertically & spaced about 6" apart. They do not seem to heat up when running

> but I don't run for more than 15 or 20 seconds at a time, shut it off for a

> few seconds & then fire up again. It doesn't take as long to reach break out

> point each time I do this so I assume there is some charge still in the tank

> circuit or the air around the coil is still somewhat charged? I tried running

> without the chokes (VERY BRIEFLY!!) The arc across the safety was outrageous

> at break out!! Next I tried placing a 3"x 14" toroid on top of the 4"x17". It

> took longer to reach break out & it came in the form of very short, sharp &

> loud "shots" but the safety arced each time. Next I tried using the 3" toroid

> only but got very little break out. I didn't re-tune much with both of these

> changes however, since the safety still arced with discharge. Finally I tried

> raising the secondary 1" & dropped down one turn on the primary. This seems to

> solve the problem. I guess I was over coupled? I'm getting 14" strong arcs to

> ground with no safety gap discharge. It forces the oil out of the cap tub a

> bit but like I said its new & needs breaking in. I'll continue this approach

> with more tuning & let you know the results. I was unable to find a quick

> source for the resistors Mike recommended (Radio Shack etc.) but I can order

> them from some of the catalogs I have. They seem to be about $6 to $9 dollars

> each. Is this about right? Also do they go between the safety & the chokes or

> between the chokes & the main gap, or both? Also I don't know what an R-C

> filter is. Could you advise? Once again, thanks for the time,  Jim

I take it then that the safety gap at no time fired unless the coil

broke out? That is very interesting. Do you have access to a storage

oscilloscope? I would be interested to know the difference between

secondary (easy) or more usefully, primary waveforms under the two

different coupling conditions. Saying one is overcoupled is somewhat

glib and doesn't really get to the heart of the matter which is

knowing the exact electrical conditions existing in the primary both

with and without the problem occurring. I'll read your original post

to see whether your output got better or not. At least we now have a

line of inquiry to pursue.

Thanks again!

Malcolm

suggests two possiblities: transients caused by the

output are a problem, but then, the problem went away when coupling

was loosened.

From:          "Malcolm Watts" <MALCOLM@directorate.wnp.ac.nz>

Organization:  Wellington Polytechnic, NZ

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Date:          Fri, 18 Sep 1998 12:56:43 +1200

Subject:       Re: safety gap activity

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Hi Jim,

An R-C filter has a series resistor in each leg going to the

main gap with a capacitor from each transformer terminal to the ground

at the transformer case. I see output went down a bit. That is not so

surprising as it would increase gap losses. It seems then that as

long as the gap conducts, the trasnformer is safe. How long are the

leads running from the main gap to the transformer?

Thanks again,

Malcolm

From:          "Malcolm Watts" <MALCOLM@directorate.wnp.ac.nz>

Organization:  Wellington Polytechnic, NZ

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Date:          Fri, 18 Sep 1998 14:33:08 +1200

Subject:       Re: safety gap activity

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Sorry Jim,

I should have replied to this bit:

> From:           Dsurfr@aol.com

> Date sent:      Thu, 17 Sep 1998 20:03:04 EDT

> To:             "Tesla-2 Mailing List" <tesla-2@emachine.com>

> Subject:        safety gap activity

> Send reply to:  "Tesla-2 Mailing List" <tesla-2@emachine.com>

> Malcolm, Mike & All: Thanks, as always, for the advice. Here is a list of the

> results as requested: First the arc across the safety only occurs when there

> is a discharge from the toroid (either to air or ground). My coil takes a

> couple of seconds to build up a charge strong enough to force break out & this

> gets stronger the longer it runs & so does the arc on the safety gap. (I have

> no idea if this is normal or not)

It is. My coil at work here takes 10 - 15 seconds to really start

long streamers flying. I think ozone, exotic molecules etc are

responsible as well as local ionization. It sounds as though your

coil is running close to breakout and it takes a few seconds for ions

to form around the top before it can let go. I;ve seen this on a

number of coils. However, no particular magic is happening.

My safety gap is connected to the

> transformer with an 8" length of 16 gauge insulated, stranded copper wire. The

> chokes are connected with a 6" length of the same wire. They are mounted

> vertically & spaced about 6" apart.

Do I understand that your transformer is only inches from the main

gap and that a safety gap across it set to fire at a higher voltage

than the main gap fires with just a few inches of lead between them??

Thnaks more,

Malcolm

Date: Thu, 17 Sep 1998 21:22:32 -0600

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Daniel Lockhart <molly@cmn.net>

Subject: Re: safety gap activity

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

>Malcolm, Mike & All: Thanks, as always, for the advice. Here is a list of the

>results as requested:

>> big snip! <<

>Could you advise? Once again, thanks for the time,  Jim

Jim, I'm no expert, but I noticed my safety gap decreased from some

activity to almost nil when I decreased my capacitance by about 10 to 16

percent. The output from the toroid dramatically increased more than

proportionally. I guess this affected tuning, but it seemed in tune with

the higher capacitance before the adjustment.

My two cents,

Daniel Lockhart

Date: Thu, 17 Sep 1998 21:30:04 -0600

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Daniel Lockhart <molly@cmn.net>

Subject: A Real Stupid NST Question

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

Hey everyone,

I kinda feel dumb asking this, as to me it seems not to be such a great

idea, and is grossly basic to boot, but I guess I'll see. It seems that I

read (or heard) somewhere that to measure the current a neon sign

transformer puts out is to directly measure it across the hv terminals.

Tell me if it is so or not! Measuring current has always been one of those

things I never get straight anyway, so it doesn't help any!

Thanks!

Daniel Lockhart

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: R.E.Ozone

X-Mailer: IPAD 2.0

From: break7@juno.com (Tyla A. Break)

Date: Thu, 17 Sep 1998 23:50:41 EDT

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Hi' thought I'd put in my two cents. I wouldn't worry about the O3,but

the gap can produce NO which immediately reacts with O2 in the air to

form NO2 which dark brown - reddish color. The interesting thing is

several people have mentioned the production of nitric acid. Copper

reacts with HNO3 to produce NO which will immediately turn brown . when

the gas is thin it is quite yellow before it changes.

Hope you find this interesting and helpful!

If it's not dangerous it's not fun!  Kyle  break7@juno.com

From:          "Malcolm Watts" <MALCOLM@directorate.wnp.ac.nz>

Organization:  Wellington Polytechnic, NZ

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Date:          Fri, 18 Sep 1998 16:07:25 +1200

Subject:       Re: safety gap activity

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Hi Daniel,

Was that primary capacitance?  What value was it, what

value is it now and what is your transformer. Are you living in the

States? If not, what is your mains frequency?

Thanks,

Malcolm

> Date sent:      Thu, 17 Sep 1998 21:22:32

- 0600 > To:             "Tesla-2 Mailing List" <tesla-2@emachine.com>

> From:           Daniel Lockhart <molly@cmn.net>

> Subject:        Re: safety gap activity

> Send reply to:  "Tesla-2 Mailing List" <tesla-2@emachine.com>

> >Malcolm, Mike & All: Thanks, as always, for the advice. Here is a list of the

> >results as requested:

>

>     >> big snip! <<

>

> >Could you advise? Once again, thanks for the time,  Jim

>

> Jim, I'm no expert, but I noticed my safety gap decreased from some

> activity to almost nil when I decreased my capacitance by about 10 to 16

> percent. The output from the toroid dramatically increased more than

> proportionally. I guess this affected tuning, but it seemed in tune with

> the higher capacitance before the adjustment.

>

> My two cents,

>

> Daniel Lockhart

From:          "Malcolm Watts" <MALCOLM@directorate.wnp.ac.nz>

Organization:  Wellington Polytechnic, NZ

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Date:          Fri, 18 Sep 1998 16:09:41 +1200

Subject:       Re: A Real Stupid NST Question

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Hi Daniel,

> Hey everyone,

>

> I kinda feel dumb asking this, as to me it seems not to be such a great

> idea, and is grossly basic to boot, but I guess I'll see. It seems that I

> read (or heard) somewhere that to measure the current a neon sign

> transformer puts out is to directly measure it across the hv terminals.

> Tell me if it is so or not! Measuring current has always been one of those

> things I never get straight anyway, so it doesn't help any!

>

> Thanks!

>

> Daniel Lockhart

That is correct but current may vary somewhat with loading ranging

from a dead short (ammeter across terminals) to something higher.

Also, it only measures what a resistive load will draw. Things change

when you hook capacitors across the terminals. In that case, a

measurement in series with the secondary circuit is required.

Malcolm

Date: Thu, 17 Sep 1998 23:09:47 -0500

From: Bert Hickman <bert.hickman@aquila.com>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Organization: Stoneridge Engineering

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: A Real Stupid NST Question

Sender: tesl-err@emachine.com

Daniel Lockhart wrote:

>

> Hey everyone,

>

> I kinda feel dumb asking this, as to me it seems not to be such a great

> idea, and is grossly basic to boot, but I guess I'll see. It seems that I

> read (or heard) somewhere that to measure the current a neon sign

> transformer puts out is to directly measure it across the hv terminals.

> Tell me if it is so or not! Measuring current has always been one of those

> things I never get straight anyway, so it doesn't help any!

>

> Thanks!

>

> Daniel Lockhart

Daniel,

This will work. Also, you can place a relatively low-valued resistor

across the transformer and measure the voltage developed across it. For

example, if you use a 100 ohm power resistor and a 60 MA (RMS)

transformer, you should see about 6 volts (RMS) across the resistor. The

power dissipateion across the resistor will only be 0.36 watts. The

NST's current-limiting action will take over, allowing the output

voltage to collapse. An AC milliameter can also be connected across the

output as well.

In either case, make sure that the connections are nice and tight, and

don't touch any part of the circuit or the meter while taking the

measurement. Also, DON'T try this with a pig, plate transformer, or MOT!

-- Bert --

From:          "Malcolm Watts" <MALCOLM@directorate.wnp.ac.nz>

Organization:  Wellington Polytechnic, NZ

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Date:          Fri, 18 Sep 1998 16:11:37 +1200

Subject:       Re: A Real Stupid NST Question

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Why is it a bad idea? How else do you measure the capability of a

current limited supply? Primary is no indication - the primary

carries magnetizing current as well as load current if any. Remember

that in this type of transformer, the primary is poorly coupled to

the secondary.

Malcolm

> Date sent:      Thu, 17 Sep 1998 23:55:03 -

0700 > From:           mike nicholas <705728@ican.net>

> To:             "Tesla-2 Mailing List" <tesla-2@emachine.com>

> Subject:        Re: A Real Stupid NST Question

> Send reply to:  "Tesla-2 Mailing List" <tesla-2@emachine.com>

>    This is a bad idea, all you need to do is measure the current on the

> primary 115V curcuit and use your turns ratio to calculate your power out.

>

> Daniel Lockhart wrote:

>

> > Hey everyone,

> >

> > I kinda feel dumb asking this, as to me it seems not to be such a great

> > idea, and is grossly basic to boot, but I guess I'll see. It seems that I

> > read (or heard) somewhere that to measure the current a neon sign

> > transformer puts out is to directly measure it across the hv terminals.

> > Tell me if it is so or not! Measuring current has always been one of those

> > things I never get straight anyway, so it doesn't help any!

> >

> > Thanks!

> >

> > Daniel Lockhart

>

>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Not so Stupid Question

X-Mailer: IPAD 2.0

From: break7@juno.com (Tyla A. Break)

Date: Fri, 18 Sep 1998 00:18:39 EDT

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Daniel, When I bought my NST the operating instructions said to

do just that.  Now I got a question I feel really bad asking .  How do

you hook up your primary cir.?  I've seen diagrams for two different

ways. One has the cap directly across the trans. and the gap is in series

with the primary across the trans. also .  The other has the gap directly

across the trans. and the cap in series with the primary connected across

the gap. The coil actually works both ways but the first way seems

easiest on the trans.

If it's not dangerous it's not fun!  Kyle  break7@juno.com

X-Mailer: IPAD 2.0

Date: Thu, 17 Sep 1998 23:37:10 -0500

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: Not so Stupid Question

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

At 12:18 AM 9/18/98 EDT, you wrote:

>	Daniel, When I bought my NST the operating instructions said to

>do just that.  Now I got a question I feel really bad asking .  How do

>you hook up your primary cir.?  I've seen diagrams for two different

>ways. One has the cap directly across the trans. and the gap is in series

>with the primary across the trans. also .  The other has the gap directly

>across the trans. and the cap in series with the primary connected across

>the gap. The coil actually works both ways but the first way seems

>easiest on the trans.

>

>	If it's not dangerous it's not fun!  Kyle  break7@juno.com

>

Kyle,

Actually the circuit with the capacitor across the transformer

is the worst configuration from a neon mortality standpoint.

Think about it. The main cap, gap and primary coil form a

LC oscillator circuit. This circuit has quite high RF voltages.

Those RF voltages appear across the main cap. If your cap is

in parallel with your neon then those same high RF voltages

will appear across your neon secondary.

With the circuit having the gap across the neon those

same RF voltages are prevented (mostly) from reaching

the neon by the short circuit that the energized gap

represents.

There is no difference in performance between the two

configurations but the shunt gap arrangement is much

easier on your transformer.

TTYL!

Mike Hammer

mhammer@misslink.net

From:          "Malcolm Watts" <MALCOLM@directorate.wnp.ac.nz>

Organization:  Wellington Polytechnic, NZ

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Date:          Fri, 18 Sep 1998 16:46:41 +1200

Subject:       Re: A Real Stupid NST Question

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

> Date sent:      Fri, 18 Sep 1998 00:35:29 -0700

> From:           mike nicholas <705728@ican.net>

> To:             "Tesla-2 Mailing List" <tesla-2@emachine.com>

> Subject:        Re: A Real Stupid NST Question

> Send reply to:  "Tesla-2 Mailing List" <tesla-2@emachine.com>

> I was always told not to try to take measurements on the HV side. With the hv side

> shorted you should be able to get a reasonable reading, dosent the transformer law

> state that power in =power out. Or are neons a breed of their own?

Unfortunately a NST doesn't not have coupling approaching unity

between its primary and secondary windings. It is not an "ideal

transformer" in any sense due to the magnetic shunts. Have you

measured the primary current with the secondary open-circuited?

Both sides are pretty lethal.

Malcolm

Date: Thu, 17 Sep 1998 22:48:20 -0600

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Daniel Lockhart <molly@cmn.net>

Subject: Re: A Real Stupid NST Question

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

I sheepishly wrote:

>> I kinda feel dumb asking this, as to me it seems not to be such a great

>> idea, and is grossly basic to boot, but I guess I'll see. It seems that I

>> read (or heard) somewhere that to measure the current a neon sign

>> transformer puts out is to directly measure it across the hv terminals.

>> Tell me if it is so or not! Measuring current has always been one of those

>> things I never get straight anyway, so it doesn't help any!

..and got many replies:

Bert wrote:

>This will work. Also, you can place a relatively low-valued resistor

>across the transformer and measure the voltage developed across it. For

>example, if you use a 100 ohm power resistor and a 60 MA (RMS)

>transformer, you should see about 6 volts (RMS) across the resistor. The

>power dissipateion across the resistor will only be 0.36 watts. The

>NST's current-limiting action will take over, allowing the output

>voltage to collapse. An AC milliameter can also be connected across the

>output as well.

Malcolm added:

>That is correct but current may vary somewhat with loading ranging

>from a dead short (ammeter across terminals) to something higher.

>Also, it only measures what a resistive load will draw. Things change

>when you hook capacitors across the terminals. In that case, a

>measurement in series with the seocndary circuit is required.

..and Mike commented...

>...all you need to do is measure the current on the

>primary 115V circuit and use your turns ratio to calculate your power out.

Hey. Thanks guys! Those were sure quick replies. I wasn't looking forward

to frying my new multimeter. I'll check things out tomorrow.

Thanks,

Daniel Lockhart

Date: Thu, 17 Sep 1998 22:51:00 -0600

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Daniel Lockhart <molly@cmn.net>

Subject: Re: safety gap activity

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

I wrote:

>> Jim, I'm no expert, but I noticed my safety gap decreased from some

>> activity to almost nil when I decreased my capacitance by about 10 to 16

>> percent. The output from the toroid dramatically increased more than

>> proportionally. I guess this affected tuning, but it seemed in tune with

>> the higher capacitance before the adjustment.

Malcolm asked:

>Hi Daniel,

>Was that primary capacitance?  What value was it, what

>value is it now and what is your transformer. Are you living in the

>States? If not, what is your mains frequency?

Yes, that was primary capacitance. I'll let you know about the others.

I can't remember off the top of my head. (about living in the States I

mean...  :-)

Daniel

Date: Thu, 17 Sep 1998 23:55:03 -0700

From: mike nicholas <705728@ican.net>

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: A Real Stupid NST Question

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

This is a bad idea, all you need to do is measure the current on the

primary 115V curcuit and use your turns ratio to calculate your power out.

Daniel Lockhart wrote:

> Hey everyone,

>

> I kinda feel dumb asking this, as to me it seems not to be such a great

> idea, and is grossly basic to boot, but I guess I'll see. It seems that I

> read (or heard) somewhere that to measure the current a neon sign

> transformer puts out is to directly measure it across the hv terminals.

> Tell me if it is so or not! Measuring current has always been one of those

> things I never get straight anyway, so it doesn't help any!

>

> Thanks!

>

> Daniel Lockhart

Date: Fri, 18 Sep 1998 00:35:29 -0700

From: mike nicholas <705728@ican.net>

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: A Real Stupid NST Question

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

I was always told not to try to take measurements on the HV side. With the hv side

shorted you should be able to get a reasonable reading, dosent the transformer law

state that power in =power out. Or are neons a breed of their own?

Malcolm Watts wrote:

> Why is it a bad idea? How else do you measure the capability of a

> current limited supply? Primary is no indication - the primary

> carries magnetizing current as well as load current if any. RTemeber

> that in this type of transformer, the primary is poorly coupled to

> the secondary.

>

> Malcolm

>

>  > Date sent:      Thu, 17 Sep 1998 23:55:03 -

> 0700 > From:           mike nicholas <705728@ican.net>

> > To:             "Tesla-2 Mailing List" <tesla-2@emachine.com>

> > Subject:        Re: A Real Stupid NST Question

> > Send reply to:  "Tesla-2 Mailing List" <tesla-2@emachine.com>

>

> >    This is a bad idea, all you need to do is measure the current on the

> > primary 115V curcuit and use your turns ratio to calculate your power out.

> >

> > Daniel Lockhart wrote:

> >

> > > Hey everyone,

> > >

> > > I kinda feel dumb asking this, as to me it seems not to be such a great

> > > idea, and is grossly basic to boot, but I guess I'll see. It seems that I

> > > read (or heard) somewhere that to measure the current a neon sign

> > > transformer puts out is to directly measure it across the hv terminals.

> > > Tell me if it is so or not! Measuring current has always been one of those

> > > things I never get straight anyway, so it doesn't help any!

> > >

> > > Thanks!

> > >

> > > Daniel Lockhart

> >

> >

From: DavidAllan@aol.com

Date: Fri, 18 Sep 1998 09:02:35 EDT

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Some first-time coiler questions

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Hi. I've just joined the list, and visited the archives. I have a few

questions. If they are answered in the archive, a polite pointer will be

appreciated.

Thanks.

1. Transformer power--I have some oil burner transformers (OBTs). A technician

gave me four, two of which he marked as "good spark" and two which said

"getting weak". These are 10 kV/23 mA OBTs.

a. Do the weakening transformers have lower voltage, current, or both? How

do I tell.

b. How does one connect more than one OBT to power a coil. Wire the HV

output terminals to the Tesla coil in parallel? How do you assure

proper phase between the transformers?

2.  Capacitor construction--I would like to use the rolled cap design

described so well in various text files. According to the WinTesla design

program, I need a capacitance of 0.0061 muF.

a. Should I make a single capacitor with this value, or two in series?

b. How do I adjust the dimensions of the capacitor plates in the coil

(aluminum flashing) to end up with the correct capacitance?

c. Can motor oil be used to cover the capacitor?

d. Where does one buy 0.030 in polyethylene?

I'm sure I'll be back later with more questions. Thanks in advance.

David Allan

X-Originating-IP: [199.217.140.22]

From: "Bill the arcstarter" <arcstarter@hotmail.com>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Some first-time coiler questions

Date: Fri, 18 Sep 1998 07:00:06 PDT

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

David Allan wrote:

>1. Transformer power--I have some oil burner transformers (OBTs). A

>technician gave me four, two of which he marked as "good spark" and

>two which said "getting weak". These are 10 kV/23 mA OBTs.

>    a. Do the weakening transformers have lower voltage, current, or

>       both? How do I tell.

You could compare how far each unit will arc by itself through air.

This will tell you roughly if the voltages are the same.

Are all four of these OBITS the same make and model, or do you have a

bunch of different ones?  I know you said the ratings were the same...

>    b.  How does one connect more than one OBT to power a coil. Wire

>        the HV output terminals to the Tesla coil in parallel?

>         How do you assure proper phase between the transformers?

To properly bank the OBITs - If they are all the same make and model

then you just connect each terminal to the corresponding terminal on the

next transformer.

If they are not of the same make, then you can do a test to determine

proper phasing:

Wire both primaries to the power line.  Phase doesn't matter here.

Connect secondary terminal 1 of OBIT1 to secondary terminal 1 of OBIT2.

Now power the thing up and *carefully* try to draw an arc between the

two remaining secondary terminals.

If the phasing is correct you should get either a very small arc or

(ideally) none at all.

If you get a large arc - STOP!  The phasing is wrong!

To correct the phasing, transpose the two primary lines on only one of

the OBITs.  Alternately you can transpose the secondary leads if that is

easier.

Once you get two phased - be sure to mark which terminals went where so

you don't have to re-discover this all over again.

You can phase-in additional transformers in a similar manner.

Mixing strong with weak transformers might not be a good idea...

Comments, anyone?

More later.

-Bill the arcstarter

Starting arcs in Cinci, OH

http://www.geocities.com/CapeCanaveral/Hangar/6160

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: R. E. David Allan

X-Mailer: IPAD 2.0

From: break7@juno.com (Tyla A. Break)

Date: Fri, 18 Sep 1998 10:17:35 EDT

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Hi, I read your message these are good questions. For the obits

hook the primaries up in parallel arbitrarily . You want to parallel the

secondaries too.   Now hook one HV terminal of each trans together.  Now

with apiece of HV wire (GTO15) try to connect the other two terminals

together while the trans is on. If you see really small sparks or no

sparks at all your O.K.  If you get a large arc switch ,em around. If

your transformers only have one HV connection each , hook the grounds

together and run a piece of HV wire across the HV terminals while the

trans is on. A dowel taped to a regular insulated wire works also. If you

get a large arc switch the polarity of one trans primary.

For poly sheet you might Cadillac Plastics 800 274 1000 Or if you

in the L.A. area you can call Plastics Concepts 213 724 4882

If it's not dangerous it's not fun!  Kyle  break7@juno.com

From: Dsurfr@aol.com

Date: Fri, 18 Sep 1998 12:13:06 EDT

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: safety gap activity

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Malcolm: I have my whole coil set up on top of my workbench (30" x 60") My

transformer is about 3"- 4" from the safety, the chokes are about 4"- 5" from

the safety & my main gap is about 4"- 5" from them. Is this set-up

unadvisable? Also the 14" arc was just moving the tap in one turn - no real

tuning done yet. Thanks again for the help.  Jim

Date: Fri, 18 Sep 1998 11:52:41 -0600

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Daniel Lockhart <molly@cmn.net>

Subject: A *Real* Stupid NST Story

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

Well, I tested the current on my neon a few minutes ago. It didn't go so

well. It *was* really stupid.

I connected the two leads for the multimeter across the output of the neon.

I placed it on "ac" and set it up to measure with a 250 ma fuse protection.

Then I gave the neon power.

The buzzer sounded on the meter, and the battery symbol on the screen went

on (?). I cut the power immediately. No reading ever registered.

To make an irritating story short, the two fuses in the multimeter are

okay, but that is all that works. Voltage, resistance, frequency, current,

inductance and capacitance measurements don't work.

Did I totally misunderstand? This is exactly what I as afraid it would do.

What did I miss?

:-(

Daniel Lockhart

Date: Fri, 18 Sep 1998 12:03:08 -0600

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Daniel Lockhart <molly@cmn.net>

Subject: Re: safety gap activity

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

I wrote:

>> Jim, I'm no expert, but I noticed my safety gap decreased from some

>> activity to almost nil when I decreased my capacitance by about 10 to 16

>> percent. The output from the toroid dramatically increased more than

>> proportionally. I guess this affected tuning, but it seemed in tune with

>> the higher capacitance before the adjustment.

Malcolm asked:

>Hi Daniel,

>Was that primary capacitance?  What value was it, what

>value is it now and what is your transformer. Are you living in the

>States? If not, what is your mains frequency?

I'll try to remember, since I don't have a meter to check things out with.

Let's see, right now the primary capacitance is 0.0067 uF. Before I

decreased it, I think it was around 0.009 to 0.007 uF. This is using

saltwater caps. I just removed one bottle. The change increased the spark

output from around sixteen to over twenty inches.

My transformer is 12 kV, at (I guess) 30 ma.

I am living in the 'States, so my mains frequency is 60 Hz.

I'm using toroid chokes, else I'd be happy to give you info on my resistors!

Hope this helps,

Daniel Lockhart

X-Originating-IP: [195.121.16.122]

From: "Sam Barros" <sambarros@hotmail.com>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Out for a while...

Date: Fri, 18 Sep 1998 12:32:29 PDT

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

ATTENTION!

I'll be in Portugal for the whole of the next week, therefore don't

expect any replies from me until next month or so...

Important or private mail should be sent to sambarros@yahoo.com.

Sam Barros

X-Originating-IP: [195.121.16.122]

From: "Sam Barros" <sambarros@hotmail.com>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Could someone HELP me?

Date: Fri, 18 Sep 1998 13:05:03 PDT

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

Hi!

This is the second time I am posting this message. Last time no one

replied and I havent made any progress since than on my Coil. Since the

answers to these questions are vital to my progress, I urge anyone with

the time and knowledge to answer this to do so...

---Sorry for the long message, but PLEASE read on---

Ok. My design aims for a medium power, very reliable, as efficient as

possible, good-looking Tesla Coil. I'll be tapping its energy to conduct

electromagnetic repulsion, confinement and acceleration of plasma, so

ground strikes shouldn't be much of a problem.

The main thing I am looking for here is efficiency. I want this coil to

be as efficient as possible. Therefore I've given it a 3:1 H/D ratio and

an angled primary, together with the use of  0.60 (between AWG 23 and

22) wire (any thicker would be seriously wasted by "skin effect")

Well, here are the specs for my coil so far:

Secondary:

Coil form:

Diameter: 20cm (8in)

Winding Length: 60 cm (24in)

Total length from toroid bottom to secondary bottom: 80cm (32in)

Wire gauge: 0.60mm (between 23 and 22AWG)

Spacing between turns: NONE

Secondary turns: 1000

Secondary wire length: 627.69m (2028.57FT)

Approximate resonant frequency: 186.11KHz

Secondary quarter wavelength resonant frequency: 119.49KHz

Secondary self-capacitance: 12.84pF

Toroid capacitance required to form quarter wavelength coil: 18.31pF

Primary:

Primary capacitance: 0.0127uF

Primary resonant frequency: 119.49KHz

Primary conductor diameter: 8mm (0.320in)

Primary turn to turn spacing: 8mm (0.320in)

Spacing between the secondary and inside turn of primary: 55mm (2.2in)

Primary tapped between turn 14 and 15 to form a resonant circuit at 119.49KHz

Neon transformer/Capacitor matching (not decided yet):

Transformer input voltage: 220V

Transformer output voltage: 15KV

Transformer operating frequency: 50Hz

Transformer output current: 60mA

Ok. Now the problems begin:

1- I have 2 ferrite cores, such as those used in radios. They measure 14

x 1cm (5.6 x 0.4in) and I also have 60 meters of 0.60mm (AWG 23/22)wire,

left from the Coil. Can I wind 2 good chokes using them? Would I need

more? I can get more of them, or a different wire, but no different

ferrite cores. How much inductance would be ideal for my coil? I think

2mH should do, but I could be wrong...

2- Mike Hammer suggested I used the following primary: 18 turns of 8mm

(0.320in) thick copper tubing spaced 8mm (0.320in) from each other, and

with 3cm (1.2in) clearance from the secondary and a 15 degrees angle. I

agree that's a better option, but I am not sure if it's the best option.

I feel I can make the secondary/primary clearance less. Perhaps 0.5in if

I insulate the primary (the secondary is already very well insulated

with epoxy and PU sealer). And, such a primary would have a diameter of

about 80cm (32in). That's HUGE!!! It would need such a massive base. Is

that really the best configuration, or can I make the primary coil

smaller and therefore increase inductance? I want the most efficient

design, remember!

One idea I had was giving the turns some 1mm spacing, and having 2 cm

clearance from the primary to the secondary. I would than insulate the

space between the turns and between primary and secondary with

Plexiglas, and also insulate the top of the primary and the bottom of

the toroid with Plexiglas. Would this work? Would it be a more efficient

coil? What is the ideal primary for my secondary, using power levels

between 1 and maybe 3KW?

3- Here in Holland the largest transformers available (because of legal

restrictions) are rated 8KV/50mA. I've being to all 13 Neon Shops within

a 50 KM range (it took me 5 days) and NOT A SINGLE ONE OF THE WILL GIVE

OR SELL ME A USED TRANSFORMER. Let alone the 3 I would need to run my

coil. A single new transformer here costs 300 dollars (rated 10KV,

100mA. They come from Italy and are impossible to get without and order)

Can some kind soul sell me a 1000W+ working, used neon transformer?

PLEASE!!! I really need this, and there is no other way I can get it.. I

can pay some 25 dollars plus the costs of transportation

Thank you very much for taking your time and reading such a long

message (supposing you didn't just scroll down looking for the "delete"

button). Please help me if you can... Any questions, comments, hints are

very welcome.

THANKS!!!!

Sam Barros

Date: Fri, 18 Sep 1998 15:47:39 -0500

From: Bert Hickman <bert.hickman@aquila.com>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Organization: Stoneridge Engineering

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: A *Real* Stupid NST Story

Sender: tesl-err@emachine.com

Daniel Lockhart wrote:

>

> Well, I tested the current on my neon a few minutes ago. It didn't go so

> well. It *was* really stupid.

>

> I connected the two leads for the multimeter across the output of the neon.

> I placed it on "ac" and set it up to measure with a 250 ma fuse protection.

> Then I gave the neon power.

>

> The buzzer sounded on the meter, and the battery symbol on the screen went

> on (?). I cut the power immediately. No reading ever registered.

>

> To make an irritating story short, the two fuses in the multimeter are

> okay, but that is all that works. Voltage, resistance, frequency, current,

> inductance and capacitance measurements don't work.

>

> Did I totally misunderstand? This is exactly what I as afraid it would do.

> What did I miss?

>

> :-(

>

> Daniel Lockhart

Daniel,

Bummer! What scale did you use?? If your meter (DMM?) didn't

specifically have an AC milliameter scale, but did have an AC volts

scale, then the only safe way to measure the NST output current is to

use the low-valued (100 ohm) resistor, using the meter to measuring the

AC voltage dropped across the resistor. If you did have an AC

milliameter (typically an analog panel meter), you could also simply

connect it across the output. Using EITHER an AC milliameter alone, or a

low-valued resistor and an AC voltmeter will work since either method

applies a relatively low resistance connection across the output of the

neon.

It sounds like you may have placed the meter across the NST using the AC

VOLTS scale, WITHOUT the 100 ohm resistor. In this configuration, there

would be no low resistance "load" across the NST, and it could then

output a higher voltage - enough to overvolt your DMM.

-- Bert --

X-Mailer: IPAD 2.0

Date: Fri, 18 Sep 1998 16:05:34 -0500

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: A *Real* Stupid NST Story

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

At 11:52 AM 9/18/98 -0600, you wrote:

>Well, I tested the current on my neon a few minutes ago. It didn't go so

>well. It *was* really stupid.

>

>I connected the two leads for the multimeter across the output of the neon.

>I placed it on "ac" and set it up to measure with a 250 ma fuse protection.

>Then I gave the neon power.

>

>The buzzer sounded on the meter, and the battery symbol on the screen went

>on (?). I cut the power immediately. No reading ever registered.

>

>To make an irritating story short, the two fuses in the multimeter are

>okay, but that is all that works. Voltage, resistance, frequency, current,

>inductance and capacitance measurements don't work.

>

>Did I totally misunderstand? This is exactly what I as afraid it would do.

>What did I miss?

>

>:-(

>

>Daniel Lockhart

Daniel,

Very sorry to hear about your mishap. That is indeed a real bummer.

Are you sure your meter was capable of measuring AC milliamps?

And are you sure you had it set correctly?

A meter in current measuring mode has very little resistance.

Certainly not more than a couple ohms. This is essentially

the same as placing a direct short across the neons terminals.

This will cause the neon to go into limiting and the voltage will

drop to a quite low value. Perhaps as low as a couple hundred volts.

Your meter can then read the short circuit current flow.

Ive done this dozens of times with several different meters.

I can only think of a couple of possible explanations.

First is you may have had the meter set incorrectly.

Its easier to do than you think. A meter set to read resistance

or voltage will have a high resistance and the neon will not

go into current limiting and the voltage will remain high.

The second possibility is that you set the meter correctly

but the internal fuse was blown. This would of resulted in an open

circuit, no current limiting, and high voltage.

Either of these could have fried your meter. For some reason

there was not a short circuit applied to the neon.

This method does work. In fact you can take a regular LED and

place it across your neon. Plug in the neon and it will light

normal brightness.

TTYL!

Mike Hammer

mhammer@misslink.net

X-Mailer: IPAD 2.0

Date: Fri, 18 Sep 1998 16:23:10 -0500

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: Could someone HELP me?

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

At 01:05 PM 9/18/98 PDT, you wrote:

> Hi!

>

> This is the second time I am posting this message. Last time no one

>replied and I havent made any progress since than on my Coil. Since the

>answers to these questions are vital to my progress, I urge anyone with

>the time and knowledge to answer this to do so...

>

> The main thing I am looking for here is efficiency. I want this coil to

>be as efficient as possible. Therefore I've given it a 3:1 H/D ratio and

>an angled primary, together with the use of  0.60 (between AWG 23 and

>22) wire (any thicker would be seriously wasted by "skin effect")

> Well, here are the specs for my coil so far:

>

<SNIP SPECS>

> Ok. Now the problems begin:

>

>1- I have 2 ferrite cores, such as those used in radios. They measure 14

>x 1cm (5.6 x 0.4in) and I also have 60 meters of 0.60mm (AWG 23/22)wire,

>left from the Coil. Can I wind 2 good chokes using them? Would I need

>more? I can get more of them, or a different wire, but no different

>ferrite cores. How much inductance would be ideal for my coil? I think

>2mH should do, but I could be wrong...

You want 2mh or more for your RF chokes. It would be hard to

determine the inductance resulting from your ferrite rods

without knowing the characteristics of the material.

Only being .4 inches in diameter will make it difficult

to get much inductance.

I would give them a try but wind them with much finer wire.

Try AWG 26 or 28. This will give you many more turns and a

greater inductance.

>

>2- Mike Hammer suggested I used the following primary: 18 turns of 8mm

>(0.320in) thick copper tubing spaced 8mm (0.320in) from each other, and

>with 3cm (1.2in) clearance from the secondary and a 15 degrees angle. I

>agree that's a better option, but I am not sure if it's the best option.

The actual physical shape of a primary is not that important.

Inductance is inductance. The various shapes differ only in the

amount of coupling they provide. Once you make allowances for

the different coupling levels any of the primary types would

give nearly the same level of performance.

>I feel I can make the secondary/primary clearance less. Perhaps 0.5in if

>I insulate the primary (the secondary is already very well insulated

>with epoxy and PU sealer).

Do not make the spacing too close. This will result in overcoupling

and pri/sec arcing. Attempts to insulate will prove futile.

The only cure for overcoupling is to reduce the coupling.

With a 15kv transformer you need at least 1.25 inches of space

between the secondary and primary to prevent problems.

>And, such a primary would have a diameter of

>about 80cm (32in). That's HUGE!!! It would need such a massive base. Is

>that really the best configuration, or can I make the primary coil

>smaller and therefore increase inductance? I want the most efficient

>design, remember!

Smaller primary turns have LESS inductance. The larger outside

turns have more inductance than the smaller inside turns.

>

> One idea I had was giving the turns some 1mm spacing, and having 2 cm

>clearance from the primary to the secondary. I would than insulate the

>space between the turns and between primary and secondary with

>Plexiglas, and also insulate the top of the primary and the bottom of

>the toroid with Plexiglas. Would this work? Would it be a more efficient

>coil? What is the ideal primary for my secondary, using power levels

>between 1 and maybe 3KW?

This will not make your coil any more efficient. It will

just make arcing between primary turns and the primary/secondary

a problem.

>

>3- Here in Holland the largest transformers available (because of legal

>restrictions) are rated 8KV/50mA. I've being to all 13 Neon Shops within

>a 50 KM range (it took me 5 days) and NOT A SINGLE ONE OF THE WILL GIVE

>OR SELL ME A USED TRANSFORMER. Let alone the 3 I would need to run my

>coil. A single new transformer here costs 300 dollars (rated 10KV,

>100mA. They come from Italy and are impossible to get without and order)

> Can some kind soul sell me a 1000W+ working, used neon transformer?

>PLEASE!!! I really need this, and there is no other way I can get it.. I

>can pay some 25 dollars plus the costs of transportation

Neons are heavy. Especially the larger ones. The shipping costs

to send them to Europe would be staggering. It even costs too

much to ship the larger ones inside the US.

I think you are worrying far too much about efficiency.

And placing that worry in the wrong areas. The places

to work on to provide good efficiency are the spark gap

and main cap.

TTYL!

Mike Hammer

mhammer@misslink.net

From: "Thomas McGahee" <tom_mcgahee@sigmais.com>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: A *Real* Stupid NST Story

Date: Fri, 18 Sep 1998 17:36:44 -0400

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Daniel,

I am sorry to hear that you damaged your meter.

Did you set it to AC *CURRENT* or voltage? You should have set it

for current. Secondly, does your meter require you to move one of the leads

to a different input jack when measuring current? If it does, and you

failed to do so, then that would explain the damage. If you fail to

move the lead, then you are bypassing the current shunt, and dangerously

high voltages can develop inside the meter.

When dealing with high voltage I prefer to connect a low value resistor

across the HV terminals of the neon and then connect the meter in

parallel with this 'shunt' resistor. I usually use a 1 ohm resistor.

Then 1 volt represents 1 amp. I also make sure that the case of the meter

is totally insulated from the neon case. I prefer to use a cheapo

AC D'Arsonval meter movement whenever possible, since it costs just a

couple of bucks and is more expendable (though I have never damaged a

meter yet).

Hope this helps,

Fr. Tom McGahee

> At 11:52 AM 9/18/98 -0600, you wrote:

> >Well, I tested the current on my neon a few minutes ago. It didn't go so

> >well. It *was* really stupid.

> >

> >I connected the two leads for the multimeter across the output of the neon.

> >I placed it on "ac" and set it up to measure with a 250 ma fuse protection.

> >Then I gave the neon power.

> >

> >The buzzer sounded on the meter, and the battery symbol on the screen went

> >on (?). I cut the power immediately. No reading ever registered.

> >

> >To make an irritating story short, the two fuses in the multimeter are

> >okay, but that is all that works. Voltage, resistance, frequency, current,

> >inductance and capacitance measurements don't work.

> >

> >Did I totally misunderstand? This is exactly what I as afraid it would do.

> >What did I miss?

> >

> >:-(

> >

> >Daniel Lockhart

>

Date: Fri, 18 Sep 1998 18:15:27 -0600

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Daniel Lockhart <molly@cmn.net>

Subject: New Upgrades on a 3-inch Coil

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

Hey all,

Unable to measure the current on my neons, and wanting to do something with

them, I hooked three together on my 3-inch diameter coil.

Tripling the current from 30 to 90 ma does wonders! Before my max spark

length was 24 inches. Currently the *min* is 24 inches! The longest sparks

aren't much longer, so the average is probably 27 inches. There is about

twice as many of them off of the toroid (five for six compared to three),

they seem "jumpier" and move around much faster. They're much brighter,

thicker and louder too!

Running at 90 ma will only be temporary. My chokes really started to burn,

my spark gap isn't made for this much current, my toroid is much to small

for that much power, and it didn't look like the secondary was appreciating

everything much. It was sure awesome though!

Coil'n away,

Daniel Lockhart

p.s. Thanks everybody for your help on measuring the current on neons. I

had the meter set right -- there must have been a loose connection

somewhere. I also didn't use the resistor. Oh well. Now I know. Thanks!

X-Originating-IP: [208.16.145.158]

From: "Bill the arcstarter" <arcstarter@hotmail.com>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: A *Real* Stupid NST Story

Date: Fri, 18 Sep 1998 20:35:18 PDT

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

Daniel Lockhart wrote:

>>

>> Well, I tested the current on my neon a few minutes ago. It didn't go

so

>> well. It *was* really stupid.

>>

>> I connected the two leads for the multimeter across the output of the neon.

>> I placed it on "ac" and set it up to measure with a 250 ma fuse protection.

>> Then I gave the neon power.

Well - not that it'll make you feel much better, but I killed a nice DVM

in 1989 measuring the output current of a anti-static DC transformer

(7kv at 5 ma or something real low)  I had the meter in current mode

just like I should have - and poof!  No more meter.  All I can figure

was that something perhaps arc'd through the case, toasting some nice

semiconductor.  That was the last time I killed a meter.

I prefer to think that the meter died all by itself and that I didn't

have anything (except for coincidence) to do with the failure.  Feel

free to feel this way about your event! :)

Since then I keep my DVMs far away from NSTs and static transformers!

Oh, there's a one-eared pole pig in the back of my truck.  Maybe I'll

tell you about it tomorrow... :)

Stay tuned!

-Bill

X-Originating-IP: [195.121.16.215]

From: "Sam Barros" <sambarros@hotmail.com>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Thanks...

Date: Sat, 19 Sep 1998 01:28:47 PDT

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

>I think you are worrying far too much about efficiency.

>And placing that worry in the wrong areas. The places

>to work on to provide good efficiency are the spark gap

>and main cap.

>

>TTYL!

>

>

>Mike Hammer

>mhammer@misslink.net

Thanks a lot Mike!

I think I'll make my spark gap out of stainless steel pipes. I'll put

them in parallel (don't know how many: I don't have a transformer) and

I'll enclose the whole thing inside a glass box, with a fan circulating

air around them. I think I'll give Hydrogen a try. I've worked with it

before and I am confident I can do it safely...

Although it isn't a worry right now, I have started working on the

primary cap. I've done some work with SW bottles and with PVC caps. The

PVC caps were far better. I coated some PVC pipes with aluminium tape

inside and out and put them one inside the other. It made a very nice

cap that stood 60KV with no visible corona... It also discharges

powerfully. I think the PVC might be a bit lossy with RF though (even

though I dried it). Still, a series of progressively smaller PVC pipes

coated with aluminium inside and out in a  closed container (the

outermost pipe) and filled with oil would probably be better than a SW

cap. Right?

Another idea I had was to make a glass cap, but substitute the glass

for Plexiglas. I could put alternating layers of Plexiglas and

aluminium, and dip the whole thing in oil. It should make a nice and

cheap cap. Anyone tried it before?

Anyway, right now I am concentrating on what I have. I paid 100dollars

for 31 meters of 8mm thick copper tubing and I want to make a primary

out of it. When I have the primary, and the secondary all done on a nice

plywood base, than I'll start worrying about the caps and the spark gaps

(supposing I someday manage to get my hands on a suitable transformer).

X-Mailer: IPAD 2.0

Date: Sat, 19 Sep 1998 14:26:30 +0100

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: bob golding <yubba@clara.net>

Subject: Re: Thanks...

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

At 01:28 19/09/98 PDT, you wrote:

>

> I think I'll make my spark gap out of stainless steel pipes. I'll put

>them in parallel (don't know how many: I don't have a transformer) and

>I'll enclose the whole thing inside a glass box, with a fan circulating

>air around them. I think I'll give Hydrogen a try. I've worked with it

>before and I am confident I can do it safely...

Sam,

While I am still acutely aware of the toxic affects of spark gaps, some

alarm bells are ringing about stainless steel electrodes. I recall someone

saying not to use them as they give off toxic fumes (or gases). Nickel?

chromium? Should be ok in a enclosed box though. But be carefull.

bob golding

X-Originating-IP: [199.217.140.22]

From: "Bill the arcstarter" <arcstarter@hotmail.com>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Could someone HELP me?

Date: Sat, 19 Sep 1998 08:50:16 PDT

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

Sam B. wrote:

>3- Here in Holland the largest transformers available (because of legal

>restrictions) are rated 8KV/50mA. I've being to all 13 Neon Shops within

>a 50 KM range (it took me 5 days) and NOT A SINGLE ONE OF THE WILL GIVE

>OR SELL ME A USED TRANSFORMER. Let alone the 3 I would need to run my

>coil. A single new transformer here costs 300 dollars (rated 10KV,

>100mA. They come from Italy and are impossible to get without and order)

> Can some kind soul sell me a 1000W+ working, used neon transformer?

>PLEASE!!! I really need this, and there is no other way I can get it.. I

>can pay some 25 dollars plus the costs of transportation

Well - a 15kv 60 ma transformer probably weighs 50-60 lbs.  You decide

if shipping that thing over to Holland would be worth it!

I'm not sure but I'd guess a 15kv 60ma unit would cost about $50 around here? (USA)

12kv 30ma units go for about $30 but still weigh about 30-40 lbs...

If you are as desperate as you say - I could attempt to lightly pursue

this a bit...

Have you considered the H&R transformers?

Did you check that guy's site where he's selling a whole load of neons?

He's on the ring...

-Bill

Date: Sat, 19 Sep 1998 10:03:45 -0600

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Daniel Lockhart <molly@cmn.net>

Subject: Re: Thanks...

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

Sam Barros wrote:

> I think I'll make my spark gap out of stainless steel pipes. I'll put

>them in parallel (don't know how many: I don't have a transformer) and

>I'll enclose the whole thing inside a glass box, with a fan circulating

>air around them. I think I'll give Hydrogen a try. I've worked with it

>before and I am confident I can do it safely...

Ouch. Have fun with the hydrogen. I've worked with it too and it's awfully

hard to contain. Good luck!

> Although it isn't a worry right now, I have started working on the

>primary cap. I've done some work with SW bottles and with PVC caps. The

>PVC caps were far better. I coated some PVC pipes with aluminium tape

>inside and out and put them one inside the other. It made a very nice

>cap that stood 60KV with no visible corona... It also discharges

>powerfully. I think the PVC might be a bit lossy with RF though (even

>though I dried it). Still, a series of progressively smaller PVC pipes

>coated with aluminium inside and out in a  closed container (the

>outermost pipe) and filled with oil would probably be better than a SW

>cap. Right?

What diameter of pvc pipes are they? What is their thickness? I tried doing

a cap like this once, but but couldn't find the right diameter of pipes

that fit inside each other. Mybe they are only available in Europe?

Thanks,

Daniel Lockhart

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: R.E. Some Help Sam barros

X-Mailer: IPAD 2.0

From: break7@juno.com (Tyla A. Break)

Date: Sat, 19 Sep 1998 12:15:12 EDT

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Hi, Sam I read your message. I'm still amazed at how much things

cost over there! I have been thinking about the transformer problem . I ,

myself have found the 15kv 30ma and 12kv 30ma most common. So they're

cheapest. Now 1000W that is going to be harder to get and cost a lot

more. I bought a 15kv 60ma (900W) new for $127 and weighs 31 lbs  . a

120ma cold cathode trans was $328 !!  It's got to weigh at least 65lbs.

That's too bad about the tubing also, for 31 meters of 9.5mm

tubing I'm paying about $29. Looks like you need to find some sort of

deal on shipping and get your stuff here.  Would a 15kv 60ma be

considered contraband?

If it's not dangerous it's not fun!  Kyle  break7@juno.com

X-Originating-IP: [207.100.20.21]

From: "Philip Mac Duffie" <punk_42@hotmail.com>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: switching neons

Date: Sat, 19 Sep 1998 11:37:29 PDT

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

I contacted the local neon shop and got two 15kv nsts ,but when i went

to pick them up they were switching power supplys. As far as I know they

are totally useless.

Philip Mac Duffie

X-Originating-IP: [207.100.20.47]

From: "Philip Mac Duffie" <punk_42@hotmail.com>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Coilers in FL?

Date: Sat, 19 Sep 1998 17:35:03 PDT

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

Hey, I'd love to but only one problem. I cant travel no licence im only

15. Problem two my coil isnt built yet.

FLORIDA QUESTION

1) can you find nsts in your area?

2) how you found it impossible to get wire?

X-Originating-IP: [208.16.150.24]

From: "Bill the arcstarter" <arcstarter@hotmail.com>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Here pig pig pig

Date: Sat, 19 Sep 1998 20:31:51 PDT

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

Coilers,

Well I am in possession of my pig!  No - it didn't weigh a mere 125 lbs

as advertised.  The true weight is 300 lbs + 25 for the pallet it was

strapped to!

I am now famous at work for having a pole transformer delivered to the

loading dock!  I came back with my pickup and grabbed it (lined up the

powered loading elevator with the bed of the truck and pushed it in

using a pallet jack).

Just tonight I completed welding a cable winch onto the end of my boom

arm - and was able to use said instrument to get the transformer off the

truck, onto the ground. :)

The transformer appears to be BRAND NEW - no rust, white paint, the

wire lugs are tight down and don't appear to have ever clamped a cable.

The can is roughly 16 inches in diameter and 28 inches tall.  The input

insulator is over a foot long which places the top of the insulator at

about 41 inches off the ground!  It isn't small or portable and is not

quite what I expected.

Total bill was $94 + $70 s/h.  Not a bad deal.  I will get the relevant

addresses/URLs for anyone else interested in adopting such an electric

animal.  I also have yet to pursue those mini-pigs in the local scrap

yard.

There is no top access plate so I don't think it has a tap switch.  I

can take the entire top off (including the input bushing) which I will

do to inspect the insides.

There is what appears to be a pressure blow-off port at the top of the

can. (a spring-loaded pull-ring)

It contains the traditional three lugs for 120-0-120 connection, and

the '0', or neutral lug is also bussed to the case of the transformer

(this is removable).  There is also a separate lug welded right to the

case.

I'm going to have to find some sort of wheeled platform for this thing

to sit on.  Either that or I'll put wheels on the pallet and call it

done!!

I was thinking about how to power the darn thing...  I have a 240 v 28

amp variac and various limiting devices (including a welder).  But the

problem is that if I use the variac - what becomes of the neutral

connection on the pig?  I can't connect it to the neutral bus - since if

I did - then one 120 volt primary of the pig would always see a full 120

volts (from N to Hot).  Also - since I have only one HV output and the

other side is connected to the case - I'd really need to keep the case

grounded and connected to neutral.  The only solution I see is to use an

isolation transformer between the variac and the pig.  This should allow

me to tie the pig's N to system N/ground, and power the pig's 12v lugs

off the isolation transformer.

It just so happens that I have a 20 amp 240v toroidal 1:1 isolation

transformer!

Does anyone think I should break the pig's N-to-case link, tie the

pig's case to a good RF ground, and then tie the pig's N to the power

neutral?  I think that might be the best plan.

Oh - I depotted our 37 amp line filter which the tesla coil killed.  It

is hard to tell but it looks like the RF arc'd from one hot wire,

through the potting tar, to the case.  That tar just barely became soft

at 350 degrees (in a toaster oven).  I would think that the heat time to

depot a large neon would be at least 12 hrs since this 4x3x1 inch filter

took over 2 hrs to melt and there wasn't any iron to heat either!

More later.  Comments invited.

-Bill

Date: Sat, 19 Sep 1998 22:52:21 -0500

From: Bert Hickman <bert.hickman@aquila.com>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Organization: Stoneridge Engineering

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Here pig pig pig

Sender: tesl-err@emachine.com

Bill the arcstarter wrote:

>

> Coilers,

>  Well I am in possession of my pig!  No - it didn't weigh a mere 125 lbs

> as advertised.  The true weight is 300 lbs + 25 for the pallet it was

> strapped to!

>

<SNIP>

>  Does anyone think I should break the pig's N-to-case link, tie the

> pig's case to a good RF ground, and then tie the pig's N to the power

> neutral?  I think that might be the best plan.

>

<SNIP>

>

> -Bill

>

<SNIP>

Bill,

Congratulations on your new family member! It must have grown some on

the trip up to Cincinatti! :^)

Re: hookup - the best bet would probably be to break the pig's LV

N-to-case link, and run the RF ground to the common HV-case connection.

Just make sure you always run the unit with your solid RF ground firmly

hooked up to the case!

Please keep us posted on the minipigs at the scrapyard. I just unwrapped

some PCB-soaked kraft paper off a large internal round copper busbar

link from inside a power transformer... the odor is quite sweet and

solvent-like, somewhat similar to benzene. If you pop the lid on the

minipigs and find they contain PCB's, you'll have NO difficulty

distinguishing the odor from mineral oil.

-- Bert --

Sender: tesl-err@emachine.com

Date: Sun, 20 Sep 1998 03:28:08 -0400

From: Tom Stock <tomstock@geocities.com>

X-Mailer: IPAD 2.0

X-Accept-Language: en

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Coilers in FL?

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Hey there's two of us over here, and with you it's three. If we can find

a few more people we can have an offical tesla coil convention....ok so

maybe just some food, books, and wire. But what the heck, we can look at

each other's coils.

-tom

www.geocities.com/CapeCanaveral/1348

Philip Mac Duffie wrote:

> Hey, Tom I live in the Daytona Beach area.

>

>   Philip Mac Duffie,

>

> ______________________________________________________

> Get Your Private, Free Email at http://www.hotmail.com

>

From: "Tobin Fricke" <fricke@mae.engr.ucdavis.edu>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Ozone and (almost) all you ever wanted to know about HV-produced Gas safety!

Date: Sun, 20 Sep 1998 02:04:38 -0700

X-Mailer: IPAD 2.0

X-MimeOLE: Produced By Microsoft MimeOLE V4.72.3110.3

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

>the bloodstream and into all your organs, including your brain).

> It is a powerful oxidiser and so inhaling large amounts of it for a

>large time could be bad for you. Nevertheless, ozone therapy is used

>quite often and it does oxygenate your body, so, in small quantities,

>ozone is actually good for you. That is why some people buy

Are you sure? I was under the impression that Ozone inhalation was damaging

to the lungs.

Tobin

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Piglets

X-Mailer: IPAD 2.0

From: break7@juno.com (Tyla A. Break)

Date: Sun, 20 Sep 1998 13:45:42 EDT

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

For people interested in "piglets" I've measured mine .

Here's what I got.

Dia. 10.5"

Height  16.5"

Insulators 10" long

Weight 98lbs

The transformer looks a little funny because

the insulators are so large compared to the body.  As for voltage,

the low side is 120v .  When used for distibution the two HV terminals

are connected as usual to the line. Depending on the line voltage, you

select low voltage taps. It looks just like 120-0-120 taps, but it's not.

X1-X3 are for 12000 line voltage and X1-X2 are for 16340.

So when running backwards you kinda got a choice 12000 or 16340. Pretty

cool huh?

If it's not dangerous it's not fun!  Kyle  break7@juno.com

From: Dsurfr@aol.com

Date: Sun, 20 Sep 1998 15:55:53 EDT

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: For what its worth??

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Hi All: I'm in the process of re-tuning after raising my secondary 1" & just

for the heck of it I placed a 2"D. solid brass door knob over the bolt that

holds my toroid on. I noticed that the arc to a ground increased about 3/4"

Then I tried moving the ground directly over the knob & got another 1/2" or

so. Don't know if its the added top load of the knob, it's smooth surface or

what. Anybody else ever tried discharges directly overhead instead of off to

the side?

One other odd bit of info. My wife has been complaining about the noise a bit

so I tried placing a piece of 4 1/2"D x 1/4" thick x 14"L plexi tube

vertically over the knob so the discharge passed through it to the ground

above. It cut the noise level dramatically! Kind of a TC "silencer". It also

improved the quality of the arcs a lot. Don't know if it contains charged ions

or the plexi some how helps the conductance (the arc is still through the air

not the wall of the tube) but it makes for a better, quieter light show.  Jim

From:          "Malcolm Watts" <MALCOLM@directorate.wnp.ac.nz>

Organization:  Wellington Polytechnic, NZ

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Date:          Mon, 21 Sep 1998 09:43:13 +1200

Subject:       Re: Thanks...

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Hi Sam,

I think it is now acknowledged that stainless steel is not

a desirable material to use for gap electrodes as the chromium vapours

are rather poisonous.

Malcolm

> From:           "Sam Barros" <sambarros@hotmail.com>

> To:             "Tesla-2 Mailing List" <tesla-2@emachine.com>

> Subject:        Thanks...

> Date sent:      Sat, 19 Sep 1998 01:28:47 PDT

> Send reply to:  "Tesla-2 Mailing List" <tesla-2@emachine.com>

> >I think you are worrying far too much about efficiency.

> >And placing that worry in the wrong areas. The places

> >to work on to provide good efficiency are the spark gap

> >and main cap.

> >

> >TTYL!

> >

> >

> >Mike Hammer

> >mhammer@misslink.net

>

>

>   Thanks a lot Mike!

>

>  I think I'll make my spark gap out of stainless steel pipes. I'll put

> them in parallel (don't know how many: I don't have a transformer) and

> I'll enclose the whole thing inside a glass box, with a fan circulating

> air around them. I think I'll give Hydrogen a try. I've worked with it

> before and I am confident I can do it safely...

<snip>

X-Originating-IP: [208.16.149.147]

From: "Bill the arcstarter" <arcstarter@hotmail.com>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Here pig pig pig

Date: Sun, 20 Sep 1998 19:37:59 PDT

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

Hi Bert, list,

Bert wrote:

>Re: hookup - the best bet would probably be to break the pig's LV

>N-to-case link, and run the RF ground to the common HV-case connection.

>Just make sure you always run the unit with your solid RF ground firmly

>hooked up to the case!

OK - I took a look inside the can - smells like diala-EX - light yellow

color, so I suppose all is well.  There was a computer-printout strapped

to the core which indicated a BIL of 150 (kv?) and a mfg date of 1983 -

well out of the PCB era.

The core laminations are bolted to the case, as is one HV lead.  The HV

windings are wrapped on top of the LV windings.

I can see where the 'other' HV lead is tied to the core - it would be

possible but difficult to extract it (thus forming the more traditional

two HV lead arrangement)

>Please keep us posted on the minipigs at the scrapyard. I just unwrapped

>some PCB-soaked kraft paper off a large internal round copper busbar

>link from inside a power transformer... the odor is quite sweet and

>solvent-like, somewhat similar to benzene.

What mischief were YOU up to? :)

>If you pop the lid on the

>minipigs and find they contain PCB's, you'll have NO difficulty

>distinguishing the odor from mineral oil.

OK.  Thanks for the hint!  The mini-pigs are really Paul's problem

child, but I might try to get over there myself and see what gives.  If

only I didn't have to work during the day...

Watch for further developments...

-Bill

Date: Sun, 20 Sep 1998 22:01:14 -0500

From: Bert Hickman <bert.hickman@aquila.com>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Organization: Stoneridge Engineering

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Here pig pig pig

Sender: tesl-err@emachine.com

Bill the arcstarter wrote:

>

> Hi Bert, list,

>  Bert wrote:

>

> >Re: hookup - the best bet would probably be to break the pig's LV

> >N-to-case link, and run the RF ground to the common HV-case connection.

> >Just make sure you always run the unit with your solid RF ground firmly

> >hooked up to the case!

>

> OK - I took a look inside the can - smells like diala-EX - light yellow

> color, so I suppose all is well.  There was a computer-printout strapped

> to the core which indicated a BIL of 150 (kv?) and a mfg date of 1983 -

> well out of the PCB era.

BIL of 150 KV is Great! Sounds like your pig is of the right pedigree

for coilin'!

>

>  The core laminations are bolted to the case, as is one HV lead.  The HV

> windings are wrapped on top of the LV windings.

>

>  I can see where the 'other' HV lead is tied to the core - it would be

> possible but difficult to extract it (thus forming the more traditional

> two HV lead arrangement)

Fortunately, no need!

>

> >Please keep us posted on the minipigs at the scrapyard. I just unwrapped

> >some PCB-soaked kraft paper off a large internal round copper busbar

> >link from inside a power transformer... the odor is quite sweet and

> >solvent-like, somewhat similar to benzene.

>

> What mischief were YOU up to? :)

I was actually looking for some thick-wall copper pipe to do some eddy

current experiments/demos. This particular piece of copper was about 33"

long, 1.5" OD and 7/8" ID. A supermagnet dropped into the center

"floats" down the center hole like it was in molases, taking almost 12

seconds to complete the journey! The magnet centers itelf so that it

doesn't touch the walls while floating its way down. Nice for

eddy-current demos! Same for some 1/2" thick copper busbar material - a

supermagnet slid across the surface shows unbelievable "drag" from the

induced eddy currents...

>

> >If you pop the lid on the

> >minipigs and find they contain PCB's, you'll have NO difficulty

> >distinguishing the odor from mineral oil.

>

> OK.  Thanks for the hint!  The mini-pigs are really Paul's problem

> child, but I might try to get over there myself and see what gives.  If

> only I didn't have to work during the day...

>

> Watch for further developments...

> -Bill

>

Good luck! It sure is a shame we have to work for a living - there's so

many other FUN things we could be doing! :^)

-- Bert --

X-Originating-IP: [208.16.149.147]

From: "Bill the arcstarter" <arcstarter@hotmail.com>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: The pig farm's having a livestock sale...

Date: Sun, 20 Sep 1998 20:28:52 PDT

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

Hey Coilers,

I've been deluged with requests on where I got my pig, so here are the

gory glorious details (appended within a fellow list member's letter):

Mine is 10KVA supposedly, 19900/19200 (or thereabouts) volts are

developed from the can to the HV bushing when 220 is applied from L1 to

L2.  There is also a N, which should be tied to the can.  Apparently

these were used to produce 120-0-120 from some sort of Y-layout

measuring 19900 from line to neutral.

>company's name and address/phone number would be helpful. :)

Look here:

http://www.austinintl.com/Transformers-Power.html

Send e-mail to Mr. Elijah Stewart at: AUSTININTL@aol.com

Or harass them LIVE! on the phone (like I did) at:  (704) 827-8075

They seem to specialize in HUGE transformers - last time I looked, 10KVA

was the smallest.  A search for "5 KVA" yielded a match on 3125 KVA!

Yow!

Tell him I sent you!  But take the weights at face value.  My 300lb pig

was promised to me both via e-mail and verbally as being a 125 lb

unit...  That was nearly very inconvenient as I had to call the freight

hauler and have it delivered at work instead of at my house (don't have

a loading dock and forktruck at home yet...)

>assuming from your post below that this is a single bushing

>transformer.  Right?

Yes.  One-eared pig.  N is tied to the can.  Your mileage may vary.

>Please verify the the voltage you give is

>not a "Y" voltage.  This is the phase-to-phase voltage that the

>transformer can be connected to in a 3-phase Y connection, but the

>transformer does not develop this voltage when run backwards (or

>see it in the Y connection).  This I'm familiar with because I

>bought one of these and the company did not mention that the voltage

>was the Y rating.  The actual voltage that the transformer sees is

>the Y rating divided by sqrt(3).  That's why you see ratings like

>14400/24900Y.

Yea - that was something which I was unable to determine before I

actually got the transformer.  The pig has NO rating label at all so I

can't answer your questions about % impedance etc.

I took a risk, and it seems that I've gotten away with it (Y vs sqrt(3)

etc) as applying 7500 volts ( half a NST) from HV to the can produces 96

volts from L1 to L2, so applying 240 from L1 to L2 should give

(neglecting losses) 18750 volts +- measurement errors etc.

>Regarding you neutral question, I suspect that the transformer

>came wired for use in a grounded wye configuration.  If I remember

>right (I can't check this -- it was on the transformer that I

>mentioned above and sent back (not fun) a while back) I saw the center

>LV connection grounded to the case on a wiring diagram on the transformer

>when it was connected for GRD Y.  This transformer also showed other

>connections possible, and on these, the neutral was not grounded to

>the case.  I hope I'm remembering this right, but I THINK you can

>just remove the connection.  Probably it was connected this way as

>a way so store the extra hardware that would not be used in other

>connections.

Don't know for sure.  Sounds like you are way ahead of me here!

Good luck hunting those wild pigs.

-Bill D. Arcstarter Pollack

From:          "Malcolm Watts" <MALCOLM@directorate.wnp.ac.nz>

Organization:  Wellington Polytechnic, NZ

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Date:          Mon, 21 Sep 1998 16:14:58 +1200

Subject:       Re: The pig farm's having a livestock sale...

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Been following this with interest Bill:

> From:           "Bill the arcstarter" <arcstarter@hotmail.com>

> To:             "Tesla-2 Mailing List" <tesla-2@emachine.com>

> Subject:        The pig farm's having a livestock sale...

> Date sent:      Sun, 20 Sep 1998 20:28:52 PDT

> Send reply to:  "Tesla-2 Mailing List" <tesla-2@emachine.com>

> Hey Coilers,

>  I've been deluged with requests on where I got my pig, so here are the

> gory glorious details (appended within a fellow list member's letter):

>

> Mine is 10KVA supposedly, 19900/19200 (or thereabouts) volts are

> developed from the can to the HV bushing when 220 is applied from L1 to

> L2.  There is also a N, which should be tied to the can.  Apparently

> these were used to produce 120-0-120 from some sort of Y-layout

> measuring 19900 from line to neutral.

All I want to know is whether it squeals or grunts?  I suspect grunt

is the operative word.

NB list coilers: this is an "ideal transformer" or as close to

it as you get in the real world. The 10kVA plate rating is

conservative to the max. You could suck 20kVA from something like

this with ease in moderate bursts. It don't care too much about

primary kickbacks either - it's insulation is rated for transients

generated by lightning strikes. Forgiving of poor TCs but completely

unforgiving of accidental contact.

Cheers,

Malcolm

From: DavidAllan@aol.com

Date: Mon, 21 Sep 1998 11:13:49 EDT

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Secondary Coil Form Material Source

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

I am gathering materials for my first coil. I plan to wind a 4 inch diameter

secondary coil. I read Richard Quick's information, in which he mentions a

number of different materials that can be used. He says that PVC is not

desirable because of its high loss characteristics, but is often used because

of its availability. He also suggests using a thin-walled material, regardless

of its composition.

I called a local plumbing house to get a scrap of four-inch PVC and they

supplied me with a two-layer pipe: white skin and black underlayer. The wall

thickness is thin, maybe 3/16 inches. I placed a few phone calls to identify

the plastic and learned, to my delight, that the pipe is entirely

polyethylene! Polyethylene is ideal because of its low loss. I suppose it

doesn't need to be sealed, either. The only problem may be gluing end caps to

the pipe.

The company in Michigan that supplied this pipe to my supplier is Hancor Pipe.

Their customer service number is (888) 367-7473. However, you should be able

to find this pipe at any company that installs drains and sewers, I would

guess. I think you should be able to find larger diameters, as well.

If any experienced coiler saw my previous post with a number of questions, I

would appreciate any answers you can give. In particular, how important is it

to select my pulse-discharge capacitance to match my OBITs? And, how do I

adjust the capacitor dimensions (rolled PE/aluminum cap) to achieve the

desired capacitance?

David Allan

From: DavidAllan@aol.com

Date: Mon, 21 Sep 1998 11:13:49 EDT

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Spark Gap

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

I am building my first coil. I will power it with a 10 kV/23 mA OBIT. I may

add a second OBIT in the future. I would like to use a static spark gap. I

have reviewed the plans for the RQ cylindrical gap that uses copper pipe. Is a

gap of this complexity necessary, or is there a simpler gap I can use to get

started? Also, if I need to use the cylindrical gap, can someone explain the

wiring connection to me? I don't quite follow the picture. How do the sparks

go in a multiple gap series configuration like this?

Thanks for your help.

David Allan

Date: Mon, 21 Sep 1998 17:47:35 +0200

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: TC & X-rays?

X-Mailer: IPAD 2.0

From: Stefan.Binder@t-online.de (Stefan Binder)

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Hi!

My physics teacher claims that a high voltage discharge at 1 atm may produce

X-rays (if the voltage is high enough). I've tested my TC with my geiger

counter but didn't detect any radiation.

I really can't believe that electrons at such a high pressure can get fast

enough to produce x-rays. What do you think?

Bye,

Stefan

Date: Mon, 21 Sep 1998 11:21:25 -0500

From: Eric Davidson <edavidson@icva.gov>

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: TC & X-rays?

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Stefan:

A Geiger-Mueller tube is not designed to detect X-rays.  A G-M tube is

designed for beta radiation.  You might try a probe designed for low

energy emitters such as Iodine-125.  These probes use a thallium

activated NaI crystal, and would give you your best shot at detection.

My guess is, that you will be hard pressed to detect any TC generated

X-rays over the ambient background radiation.  Hope this helps!

Eric

Stefan Binder wrote:

>

> Hi!

> My physics teacher claims that a high voltage discharge at 1 atm may produce

> X-rays (if the voltage ist high enough). I've tested my TC with my geiger

> counter but didn't detect any radiation.

> I really can't believe that electrons at such a high pressure can get fast

> enough to produce x-rays. What do you think?

>

> Bye,

>  Stefan

--

Have a good day!

Eric Davidson

edavidson@icva.gov

X-Mailer: IPAD 2.0

Date: Mon, 21 Sep 1998 11:47:07 -0500

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: Spark Gap

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

At 11:13 AM 9/21/98 EDT, you wrote:

>I am building my first coil. I will power it with a 10 kV/23 mA OBIT. I may

>add a second OBIT in the future. I would like to use a static spark gap. I

>have reviewed the plans for the RQ cylindrical gap that uses copper pipe. Is a

>gap of this complexity necessary, or is there a simpler gap I can use to get

>started? Also, if I need to use the cylindrical gap, can someone explain the

>wiring connection to me? I don't quite follow the picture. How do the sparks

>go in a multiple gap series configuration like this?

>

>Thanks for your help.

>

>David Allan

David,

For current limited transformers like OBITs or neons

the cylinder gap popularized by Richard Quick is an excellant

performer.

You can use simpler gaps. In fact you could conceivably just

use two pieces of wire. Performance will suffer greatly. Since

you will be running rather low power to start with your single

OBIT you want to minimize losses. Gap loss is the single greatest

loss factor in Tesla coil systems. The cylinder gap reduces losses

by breaking up the gap into a series of smaller gaps. These small

gaps produce less heat and provide improved quenching.

The RQ instructions in the file you read are not very clear.

Basically what you do is make connection to the first and last

electrodes and the arc jumps from electrode to electrode.

The total amount of gap is the sum of the individual gaps used.

You dont have to use all the gaps in a cylinder gap system.

Suppose you make such a gap with 10 electrodes. There are 9 gaps

between the 10 electrodes if you use all 10. But instead connect

to the first and ninth electrodes. Now its only 8 gaps and

the total gap distance is smaller. Its often desirable to use

less than the maximum amount of gap for tuning or checking

out a new configuration.

For 10kv you probably will want around .200 to .225 inches

of total gap. So if you use 10 electrodes spaced at .025 inch

this will give 9 gaps with a total of .225 inch. Use 3/4 inch

hard copper pipe and electrodes around 3 inches long. 10

electrodes will fit inside 4 inch PVC pipe but you will need

a plastic barrier between the first and last electrodes so

the arc doesnt cheat and take a short cut.

Use a strong fan to supply cooling air flow.

hope this helps.

TTYL!

Mike Hammer

mhammer@misslink.net

X-Mailer: IPAD 2.0

Date: Mon, 21 Sep 1998 12:06:16 -0500

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: Secondary Coil Form Material Source

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

At 11:13 AM 9/21/98 EDT, you wrote:

>I am gathering materials for my first coil. I plan to wind a 4 inch diameter

>secondary coil. I read Richard Quick's information, in which he mentions a

>number of different materials that can be used. He says that PVC is not

>desirable because of its high loss characteristics, but is often used because

>of its availability. He also suggests using a thin-walled material, regardless

>of its composition.

The choice of secondary form materials is not really that important.

System Q is determined mainly by the primary circuit components.

Especially the spark gap. The secondary coil Q has very little

effect on system wide Q.

Many high performance coils have been built on "lossy" PVC pipe.

>

>I called a local plumbing house to get a scrap of four-inch PVC and they

>supplied me with a two-layer pipe: white skin and black underlayer. The wall

>thickness is thin, maybe 3/16 inches. I placed a few phone calls to identify

>the plastic and learned, to my delight, that the pipe is entirely

>polyethylene! Polyethylene is ideal because of its low loss. I suppose it

>doesn't need to be sealed, either. The only problem may be gluing end caps to

>the pipe.

Polyethylene is great. But the black coloring bothers me. There is no

way to tell ahead of time but some of that black coloring agent may

be carbon which would make the plastic somewhat conductive. Not

a good thing for a secondary form.

>If any experienced coiler saw my previous post with a number of questions, I

>would appreciate any answers you can give. In particular, how important is it

>to select my pulse-discharge capacitance to match my OBITs?

If you want the maximum amount of power out of your transformer then

matching the cap is important. What the matching cap does is make the

transformer secondary and main cap resonant at 60hz. This serves to

neutralize some of the built in current limiting of an OBIT or NST.

It is not without risk. The resonant condition can allow the voltage to

rise very quickly. Often to several times the RMS rating of the transformer.

For this reason a properly set safety gap is mandatory.

>And, how do I adjust the capacitor dimensions (rolled PE/aluminum cap) to achieve the

>desired capacitance?

>

Use the following formula:

Capacitance (in picofarads) = (0.224 KA / d) (N-1)

0.224 x Dielectric Constant x Area of plate

C = -----------------------------------------  x (Number of plates - 1)

distance between plates in inches

NOTE: For two plate rolled capacitors the value derived from this

formula must be multiplied by two, or, the value must be computed

with the number of plates (N) equal to three (3), so (N-1 = 2)

Use a nominal value of 2.0 as the dielectric constant (K) of polyethylene.

TTYL!

Mike Hammer

mhammer@misslink.net

X-Mailer: IPAD 2.0

Date: Mon, 21 Sep 1998 13:10:44 -0500

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: switching neons

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

At 11:37 AM 9/19/98 PDT, you wrote:

>I contacted the local neon shop and got two 15kv nsts ,but when i went

>to pick them up they were switching power supplys. As far as I know they

>are totally useless.

>      Philip Mac Duffie

>

Philip,

You are correct. The switching supply neons are

practically worthless for our use. They have protective

circuitry that turns them off in the event of a short circuit.

Not very useful. Unfortunately more and more of the neons

found today are of this type.

TTYL!

Mike Hammer

mhammer@misslink.net

From: "Basura, Brian" <brian.basura@unistudios.com>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Initial Setting Review/Suggestions

Date: Mon, 21 Sep 1998 11:21:56 -0700

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Hello,

I'm getting very close to firing up my coil for the first time and would

welcome input on the initial settings.

I'm looking for starting points for the following settings:

1) Primary tap.

2) Secondary height above primary.

3) Toroid height above secondary.

4) Anything else you see that may be wrong.

My coil specification are:

Secondary

6.5" by 27.5" wound on sealed SDR35 PVC form

22awg for 1000 turns

Adjustable height (Initial setting is 2.5" above primary)

Inductance 34.56 mH, Self-C11.91pf, Freq 248/131,  (From WinTesla)

Inductance 33.8 mH, Self-C12.2pf, Freq 248/138,  (From Tesla.xls)

Primary (Flat Spiral)

9.5"ID by 26"OD

16 turns of .25" copper tubing .50" on center (.25" between turns)

Inductance 140.7mH (From WinTesla)

Inductance 134.6 mH (From Tesla.xls)

Tapped at 11 turns initially

Tank Capacitor (Rolled Poly)

Initially three in series for .015mfd @ ?(18 layers of poly for .108")

RQ Style Gap with blower

Initially 6 gaps for .18"

Toroid (temporary until I get another form)

One unit 22" in diameter with a 3.75" chord stacked directly on top of

another unit 27" in diameter with a 3.75" chord.  This looks like it should

be equivalent to a unit 24.5" by 6.5" or 27 to 30pf (pure guesswork).

Bottom of assembly is set 1.5" above top secondary winding.

HV (PT with external current limiting)

Initially 12,000v @ 100ma

(my target is 180 to 200ma with .040mfd Tank Capacitor)

Inductive current limiting utilizing MOT's with shorted secondary and

4.2ohms series dampening resistance.

Other stuff

Safety gaps (capacitors and PT)

Line filters, Fuses, Indicators, Interlocks, Dead Man Switch, etc.

I have a number of construction pictures on my WEB page and hope to have

some operating pictures soon :')  www.hughes-ec.com/~brianb/teslahome.html

TIA - (Thanks In Advance)

Brian D. Basura

X-Mailer: IPAD 2.0

Date: Mon, 21 Sep 1998 13:29:05 -0500

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Mike Hammer <mhammer@misslink.net>

Subject: hamfest report

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

All,

This past weekend was the Peoria, Il hamfest

and some goodies were found.

List member Kent Vander Veld came over from Iowa

and accompanied me to the hamfest.

Kent bought a really nice (and heavy) TEK #7704 250 MHZ

mainframe scope with a good assortment of plugins for $175.

I purchased a TEK # 5103N 25 MHZ storage scope with two

dual trace vertical plugins and a dual sweep horizontal

plugin for $70. Works great.

I also got some 2 inch diamter ferrite toroids for RF chokes

and other assorted components. No magnet wire was found however.

Bummer. Anybody got a few extra pounds of AWG 22?

Also found was a good assortment of technical books.

I got a copy of the "Radiotron Designers' Handbook"

and a pristine copy of "Principles of Radio Communication"

by J.H. Morecroft.

Also found was a book titled "High Frequency Measurements"

by August Hund. This looks like a treasure trove of useful

information.

Bert you will hate it when I tell you I got these for 50 cents each!

Its a good thing you didnt come to Peoria. You would still be there

rummaging through those books. There was some pretty arcane stuff

in there. Right up your alley. :)

TTYL!

Mike Hammer

mhammer@misslink.net

From: DavidAllan@aol.com

Date: Mon, 21 Sep 1998 15:10:24 EDT

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Secondary Coil Form Material Source

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Mike, Thanks for your helpful advice. I called the vendor (mentioned below)

and found that the black is indeed due to carbon black. They wouldn't tell me

how much, but they did say that the polyethylene is stabilized to last against

50 years of UV radiation. (Maybe I should use it to contain my spark gap :-)

Anyway, I assume that means this material is ruled out for my secondary. Am I

correct in thinking that the presence of conductive material in the plastic,

even if there is no continuous conductive path, would be a BAD thing? I assume

that at least, there would be a lot of RF heating (like a microwave oven) of

the plastic, probably to the point of melting and failure.

Any experts care to comment?

Thanks again.

David Allan

In a message dated 98-09-21 13:14:44 EDT, you write:

<< Subj:	 Re: Secondary Coil Form Material Source

Date:	98-09-21 13:14:44 EDT

From:	mhammer@misslink.net (Mike Hammer)

Sender:	tesl-err@emachine.com

Reply-to:	tesla-2@emachine.com (Tesla-2 Mailing List)

To:	tesla-2@emachine.com (Tesla-2 Mailing List)

At 11:13 AM 9/21/98 EDT, you wrote:

>I am gathering materials for my first coil. I plan to wind a 4 inch diameter

>secondary coil. I read Richard Quick's information, in which he mentions a

>number of different materials that can be used. He says that PVC is not

>desirable because of its high loss characteristics, but is often used

because

>of its availability. He also suggests using a thin-walled material,

regardless

>of its composition.

The choice of secondary form materials is not really that important.

System is determined mainly by the primary circuit components.

Especially the spark gap. The secondary coil Q has very little

efect on system wide Q.

Many high performance coils have been built on "lossy" PVC pipe.

>

>I called a local plumbing house to get a scrap of four-inch PVC and they

>supplied me with a two-layer pipe: white skin and black underlayer. The wall

>thickness is thin, maybe 3/16 inches. I placed a few phone calls to identify

>the plastic and learned, to my delight, that the pipe is entirely

>polyethylene! Polyethylene is ideal because of its low loss. I suppose it

>doesn't need to be sealed, either. The only problem may be gluing end caps

>to the pipe.

Polyethylene is great. But the black coloring bothers me. There is no

way to tell ahead of time but some of that black coloring agent may

be carbon which would make the plastic somewhat conductive. Not

a good thing for a secondary form.

[snip]

X-Mailer: IPAD 2.0

Date: Mon, 21 Sep 1998 16:00:15 -0500

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: Secondary Coil Form Material Source

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

At 03:10 PM 9/21/98 EDT, you wrote:

>Mike, Thanks for your helpful advice. I called the vendor (mentioned below)

>and found that the black is indeed due to carbon black. They wouldn't tell me

>how much, but they did say that the polyethylene is stabilized to last against

>50 years of UV radiation. (Maybe I should use it to contain my spark gap :-)

>

>Anyway, I assume that means this material is ruled out for my secondary. Am I

>correct in thinking that the presence of conductive material in the plastic,

>even if there is no continuous conductive path, would be a BAD thing? I assume

>that at least, there would be a lot of RF heating (like a microwave oven) of

>the plastic, probably to the point of melting and failure.

>

>Any experts care to comment?

>

>Thanks again.

>

>David Allan

Since no experts were available I decided to comment. :)

Yes the presence of carbon definitely rules out the

use of this pipe as a secondary form. It would readily

break down under the high voltage RF assault you plan for it.

I found this out quite a few years ago when I wound a small

coil on black ABS pipe. It died rather quickly.

I wouldnt even consider it even for your spark gap.

TTYL!

Mike Hammer

mhammer@misslink.net

From:          "Malcolm Watts" <MALCOLM@directorate.wnp.ac.nz>

Organization:  Wellington Polytechnic, NZ

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Date:          Tue, 22 Sep 1998 09:20:04 +1200

Subject:       Re: TC & X-rays?

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Hi Stefan,

Can your Geiger counter detect X-rays from a proper X-ray

source? I wonder. Most of them only respond to nuclear-type energies

such as those found in gamma rays and charged particles released from

radioactive decay.

?

Malcolm

> Date sent:      Mon, 21 Sep 1998 17:47:35 +0200

> To:             "Tesla-2 Mailing List" <tesla-2@emachine.com>

> Subject:        TC & X-rays?

> From:           Stefan.Binder@t-online.de (Stefan Binder)

> Send reply to:  "Tesla-2 Mailing List" <tesla-2@emachine.com>

> Hi!

> My physics teacher claims that a high voltage discharge at 1 atm may produce

> X-rays (if the voltage ist high enough). I've tested my TC with my geiger

> counter but didn't detect any radiation.

> I really can't believe that electrons at such a high pressure can get fast

> enough to produce x-rays. What do you think?

>

> Bye,

>  Stefan

>

>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: "Coiler" <mycroft@access1.net>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: RE: Secondary Coil Form Material Source

Date: Mon, 21 Sep 1998 16:07:07 -0700

X-Mailer: IPAD 2.0

Importance: Normal

X-MimeOLE: Produced By Microsoft MimeOLE V4.72.3155.0

Sender: tesl-err@emachine.com

Hmm.. I'd not say I am an expert, but I can tell you that I have successfully

built a 6.5" coil based on the same white/black HDPE you mention. Coil had

performed rather well (best spark so far has been 37") with no hint of

breakdown.

Granted, because I didn't know better, I actually prepped this as you

would have prepped PVC (light sanding, coats of urethane).

There is no apparent heating of the form.

Now when my pig gets here.. who knows?

> -----Original Message-----

> From: tesl-err@emachine.com [mailto:tesl-err@emachine.com]On Behalf Of

> DavidAllan@aol.com

> Sent: Monday, September 21, 1998 12:10 PM

> To: Tesla-2 Mailing List

> Subject: Re: Secondary Coil Form Material Source

>

>

> Mike, Thanks for your helpful advice. I called the vendor (mentioned below)

> and found that the black is indeed due to carbon black. They wouldn't tell me

> how much, but they did say that the polyethylene is stabilized to  last against

> 50 years of UV radiation. (Maybe I should use it to contain my

> spark gap :-)

>

> Anyway, I assume that means this material is ruled out for my secondary. Am I

> correct in thinking that the presence of conductive material in the plastic,

> even if there is no continuous conductive path, would be a BAD thing? I assume

> that at least, there would be a lot of RF heating (like a microwave oven) of

> the plastic, probably to the point of melting and failure.

>

> Any experts care to comment?

>

> Thanks again.

>

> David Allan

>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: R.E.  Choad 69

X-Mailer: IPAD 2.0

From: break7@juno.com (Tyla A. Break)

Date: Mon, 21 Sep 1998 22:22:14 EDT

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Hi, Choad. The little piggie is 26.5 " tall including it's

insulators and its tag says a ridiculously low 500w. I've opened it up

and looked inside. The primary looks about #14 . I've had no trouble

running it at 2400 - 3600 w . I even reduced my current limiter and run

12kw for 5- 10 seconds but that's a little hairy! I'd rather have a

little pig instead of bacon!

So far it seems to be a one- time deal . I got it from a construction

co. for $150 seems a little steep but it was exactly what I needed.

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Magnet Wire

X-Mailer: IPAD 2.0

From: break7@juno.com (Tyla A. Break)

Date: Mon, 21 Sep 1998 22:45:22 EDT

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Hi. Mike Nice score at the hamfest!    I don't have any trouble

getting magnet wire. #9-10-14 17-18-19-20-maybe 22 (haven't found it yet

) , small stuff like 30-40. I get it for $2/lb. Is that expensive? I

bought about 5 spools of assor. sizes. I can look for 22 when I go next

time. Maybe Igor wants to go Sat. morning ?? Full spools are usually $20.

Shipping might ruin the deal   10# to OH was it?

If it's not dangerous it's not fun!  Kyle  break7@juno.com

Date: Mon, 21 Sep 1998 21:50:41 -0500

From: Bert Hickman <bert.hickman@aquila.com>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Organization: Stoneridge Engineering

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: hamfest report

Sender: tesl-err@emachine.com

Mike Hammer wrote:

>

> All,

>

> This past weekend was the peoria, Il hamfest

> and some goodies were found.

>

> List member Kent Vander Veld came over from Iowa

> and accompanied me to the hamfest.

>

> Kent bought a really nice (and heavy) TEK #7704 250 MHZ

> mainframe scope with a good assortment of plugins for $175.

>

> I purchased a TEK # 5103N 25 MHZ storage scope with two

> dual trace vertical plugins and a dual sweep horizontal

> plugin for $70. Works great.

>

> I also got some 2 inch diamter ferrite toroids for RF chokes

> and other assorted components. No magnet wire was found however.

> Bummer. Anybody got a few extra pounds of AWG 22?

>

> Also found was a good assortment of technical books.

> I got a copy of the "Radiotron Designers' Handbook"

> and a pristine copy of "Principles of Radio Communication"

> by J.H. Morecroft.

>

> Also found was a book titled "High Frequency Measurements"

> by August Hund. This looks like a treasure trove of useful

> information.

>

> Bert you will hate it when I tell you I got these for 50 cents each!

> Its a good thing you didnt come to Peoria. You would still be there

> rummaging through those books. There was some pretty arcane stuff

> in there. Right up your alley. :)

>

> TTYL!

>

> Mike Hammer

> mhammer@misslink.net

Mike,

Congratulations on a successful hunt at the Peoria Fest! The big Grays

Lake Hamfest is coming up next weekend, so hopefully I'll luck out

there! Glad to see that the books went to a good coiler! You'll

especially like the Morecroft book - lots of EXCELLENT material on spark

radio... but for 50 cents??? You stole them! :^)

-- Bert --

From: ralph.hartwell@emachine.com (Ralph Hartwell)

Subject:  Re: TC & X-rays?

Date: Tue, 22 Sep 1998 03:12:00 GMT

Organization: The Energy Machine Information System 504-733-3462

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

>            Can your Geiger counter detect X-rays from a proper X-ray

>source? I wonder. Most of them only respond to nuclear-type energies

>such as those found in gamma rays and charged particles released from

>radioactive decay.

Mine sure can!  I have an old dental X-ray machine (quite handy for

seeing what's inside those oddball hamfest goodies and commercial

antennas) and my counter goes nuts when I point the X-ray head at it,

even at a distance of 25 feet.

Ralph

(and yes, I have lots of lead shielding.)  :+)

X-Originating-IP: [208.16.149.5]

From: "Bill the arcstarter" <arcstarter@hotmail.com>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: TC & X-rays?

Date: Mon, 21 Sep 1998 20:28:51 PDT

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

Stefan write:

>My physics teacher claims that a high voltage discharge at 1 atm may produce

>X-rays (if the voltage ist high enough). I've tested my TC with my geiger

>counter but didn't detect any radiation.

>I really can't believe that electrons at such a high pressure can get fast

>enough to produce x-rays. What do you think?

Well - here are my comments.  Generally to produce significant (ie -

able to even penetrate AIR or glass) x-ray radiation you need at the

very least 15 kv and a HARD vacuum.  Having an anode made of a dense

material also helps production.

Having said that - I have heard that some types of completely evacuated

bulbs (like certain old valve tubes) will emit detectable x-ray

radiation if powered via tesla coil.

Our geiger counter at work IS sensitive to x-ray radiation, even that

produced by only 40KV across a legitimate x-ray tube.  Your results may

vary!

Unless you are placing evacuated bulbs on the TC I don't think there is

anything to worry about.

I do have a calibrated x-ray radiation survey meter at work (used for

mapping scatter / leakage x-ray radiation).  If anyone is really

interested I could bring it home and see what it tells me! :)  That is,

once Paul gets the coil running again...

-Bill

X-Originating-IP: [208.16.149.5]

From: "Bill the arcstarter" <arcstarter@hotmail.com>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: TC & X-rays?

Date: Mon, 21 Sep 1998 20:37:39 PDT

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

It was written:

>Stefan:

>

>A Geiger-Mueller tube is not designed to detect X-rays.  A G-M tube is

>designed for beta radiation.

Oh - I don't know about that.  X-ray radiation is definitely considered

"Ionizing Radiation", just like beta is.

The one geiger counter we have at work will respond to both direct x-ray

exposure as well as traditional radioactive sources like old smoke

detectors, radium pellets, etc.  Perhaps others don't work this way?

>You might try a probe designed for low

>energy emitters such as Iodine-125.  These probes use a thallium

>activated NaI crystal, and would give you your best shot at detection.

Hmm.  Don't have any idea.  We do have some large $35000 imaging tubes

at work which have an input window coated with Cesium Iodide, which

ejects electrons when struck by x-ray radiation.  These free electrons

are accelerated and eventually slam into a traditional phosphor window,

just like a CRT.  This device bolts onto the front-end of a video camera

- and is used for the real-time inspection of patient anatomy (or line

filters, network cards, the wife's cats, bullets, or anything else I

feel like sneaking back there! :)

Send me $35000 and I'll get you one!

-Bill

X-Originating-IP: [208.16.149.5]

From: "Bill the arcstarter" <arcstarter@hotmail.com>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: TC & X-rays?

Date: Mon, 21 Sep 1998 20:42:47 PDT

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

Stefan wrote:

>Hmmm, I don't know, I've got a light bulb that's obviously evacuated,

>upon putting it on my TC the glass glowed greenish, and my geiger

>counter showed a high radiation level.

Oh-ho.  Sounds like it's working...  What type of bulb do you have?

Sounds like I should bring the survey meter home... :)

Was the glass glowing uniformly green?  Were there any "plasma"

streamers inside the glass (implying non-evacuated)?

There's somebody else on this list who knows more about this than I

do...  I wonder if he'll pipe up??

What part of the world/states are you in?  I have an x-ray sensitive

phosphor screen which I might be talked out of...

-Bill

Date: Mon, 21 Sep 1998 21:42:07 -0700

From: Gene Waldo <gwaldo@1stnetusa.com>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Magnet Wire

Sender: tesl-err@emachine.com

Tyla A. Break wrote:

>

>              Maybe Igor wants to go Sat. morning ??

>

Kyle,

You talking APEX?  What sort of ETA? I can justify a trip there on about

any grounds.

OFF TOPIC ASIDE: to Mike and Bert,

I have a piece of old stuff I'd just as soon be rid of: RCA speaker,

'monitor'(?) style; ca.~ 1917, P/N label, etc. here somewhere;

operational at last check; was connected (twist pr) to Atwater-Kent

recvr. (flat metal box, w/ 2 or 3 tubes and 40lb brass tun cap?) also

oper at last check, but destroyed in vandal-fire--  ~~<[:-(

I've little interest in the spkr; have it only because I never throw

anything a way  ~~<[:-((((((

I guess I could send pics if anyone wants to keep it out of my dumpster.

I'm slowly trying to get over my 'keep-it' disease! A genetic defect

inherited from my father. I have early recall of the system as 'our

radio' in ~1937.

If I'm not mistaken, it may have leads of the old fabric covered

foil/string-headphone type; probably not issue.

----------------------------------------------

Gene Waldo -  gwaldo@1stnetusa.com  - "Igor in Residence"

---------------------------------------------------------------

What's good for the planet is more OZONE! See the machinery form up!

NEW PAGE!   http://members.1stnetusa.com/danmurphy/waldo.htm

From: "Thomas McGahee" <tom_mcgahee@sigmais.com>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: TC & X-rays & Geiger Counters?

Date: Tue, 22 Sep 1998 08:10:11 -0400

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Stefan,

As one who has used Tesla Coils for many years to produce x-rays, I guess

that I am as qualified as any one else to discuss this subject.

First, a Tesla coil by itself will *not* produce x-rays. You need a high

enough vacuum to get the mean free path long enough to allow the electrons

to get up to a high enough speed to create x-rays upon impact. If you want

to get highly technical, you *might* be able to create x-rays at

atmospheric pressures, but you would need several million volts to

do so, and even then the x-rays would be weak and non-focused.

Having said all that:

A Tesla Coil by itself will not produce x-rays. However, if you

attach any kind of evacuated vessel to the terminal of a Tesla

coil, then you have a VERY good chance of producing x-rays.

When I was in high school back in 1964 I did a series of

experiments along this line, and found that *many* radio tubes

and even certain types of ordinary light bulbs could be made

to produce copious x-rays. The experiments were triggered by

an article I had read in an older Scientific American magazine

in the Amateur Scientist column run by C.L. Stong. The author

of the article was Mr. Harry Simons.

Most modern light bulbs contain a gas such as argon, and do

not have the high vacuums in them that older light bulbs

had. If the evacuated bulb/tube/device gives off a light blue

or fluorescent green light when connected to a Tesla Coil,

then it is most likely generating x-rays. Continued

operation usually results in the glass becoming very hot, and

often a pinhole will develop and the vacuum ceases to exist.

Exercise caution when attaching evacuated devices to the top

of a Tesla Coil. Other than that, don't worry about x-ray

production.

If you *do* connect something like a light bulb to the top of

a Tesla Coil, be aware that the lifetime of the bulb will not

be very long, as the glass walls tend to develop pinhole

punctures due to pinpoint heating of the glass. To produce

useful x-rays it is better to use a regular x-ray tube, as

these have TWO electrodes. A light bulb has only one electrode

and so the electric potential is applied across the glass and

boy does the glass get hot!

Now, as to the ability of a geiger counter to detect x-rays,

the answer is a resounding *yes*! I have used many different

models/brands of geiger counters to detect x-rays. They are

quite good at detecting x-rays. The kind of window used and the

thickness of the window determine the threshold of x-ray

detection.

Hope this helps.

Fr. Tom McGahee

----------

> From: Stefan Binder <Stefan.Binder@t-online.de>

> To: Tesla-2 Mailing List <tesla-2@emachine.com>

> Subject: TC & X-rays?

> Date: Monday, September 21, 1998 11:47 AM

>

> Hi!

> My physics teacher claims that a high voltage discharge at 1 atm may produce

> X-rays (if the voltage ist high enough). I've tested my TC with my geiger

> counter but didn't detect any radiation.

> I really can't believe that electrons at such a high pressure can get fast

> enough to produce x-rays. What do you think?

>

> Bye,

>  Stefan

From: DavidAllan@aol.com

Date: Tue, 22 Sep 1998 08:24:34 EDT

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Magnet Wire

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

In a message dated 98-09-21 23:13:54 EDT, Kyle wrote:

[snip]

I don't have any trouble

getting magnet wire. #9-10-14 17-18-19-20-maybe 22 (haven't found it yet) ,

small stuff like 30-40. I get it for $2/lb. Is that expensive? I bought about

5 spools of assor. sizes. I can look for 22 when I go next time. Maybe Igor

wants to go Sat. morning ?? Full spools are usually $20. Shipping might ruin

the deal   10# to OH was it?

If it's not dangerous it's not fun!  Kyle  break7@juno.com

>>

Can you share your source with the rest of us? I'm about to place an order for

a couple of pounds of AWG 22 wire at $10/lb, plus shipping. (And I thought

that was *good*.)

Thanks.

From: Roy Minnich <rminnich@rolling-meadows.lib.il.us>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: X-ray & Oudin coil

Date: Tue, 22 Sep 1998 14:48:59 -0500

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

Hello Tom and list members,

If you would like to take a trip down memory lane with Tom check out  http://www.noah.org/science/x-ray/index.html  for text and graphics on the below

mentioned x-ray machine.  It uses an Oudin coil to zap the radio tube.  The Oudin

coil circuit looks a lot like a Tesla coil set up!

Roy Minnich

----------

From: 	Thomas McGahee[SMTP:tom_mcgahee@sigmais.com]

Reply To: 	Tesla-2 Mailing List

Sent: 	Tuesday, September 22, 1998 7:10 AM

To: 	Tesla-2 Mailing List

Subject: 	Re: TC & X-rays & Geiger Counters?

<SNIP>

When I was in high school back in 1964 I did a series of

experiments along this line, and found that *many* radio tubes

and even certain types of ordinary light bulbs could be made

to produce copious x-rays. The experiments were triggered by

an article I had read in an older Scientific American magazine

in the Amateur Scientist column run by C.L. Stong. The author

of the article was Mr. Harry Simons.

<SNIP>

Hope this helps.

Fr. Tom McGahee

X-Originating-IP: [199.217.140.22]

From: "Bill the arcstarter" <arcstarter@hotmail.com>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: X-ray & Oudin coil

Date: Tue, 22 Sep 1998 13:35:21 PDT

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

Coilers,

So can anyone explain what is fluorescing when these evacuated bulbs

are placed on the tesla coil?

Is the glass itself glowing?  Is this a form of (?sp) Chrenkov

radiation (same type as seen in the pools of cooling water surrounding

hot nuclear fuel rods).

If I can find such a bulb I will attempt to use my x-ray survey meter

to map out some intensities, etc..

-Bill

Date: Tue, 22 Sep 1998 18:00:06 -0600

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Daniel Lockhart <molly@cmn.net>

Subject: Photographing Tesla Coils (PHOTO 101)

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

Hey all,

I know this subject comes up a lot, so I thought I'd bring it up again. :-)

What is the best way to take photos of Tesla Coils? I did a little

experiment with the only "unelectric" camera I had with different shutter

speeds. I'm no photographer, and I had hiccups while shooting the

several-second-long shots without a tripod, so things turned out a little

fuzzy. Nine out of the thirty-two were decent, 28% not bad, huh? Especially

with a little camera I found in the garage...

Anyway, on this camera I have a shutter speed (infinity to 500), focus, ASA

(25 to 1600), DIN (15 to 33), and a knob that measures from 22 to 2.8.

I know how to use the shutter and focus (imagine that?). I set the ASA to

whatever the film is, am I correct? The ASA and DIN scales are on the same

knob, just opposite each other, thus when you select one, you select the

other. ASA 25 = DIN 15, ASA 1600 = DIN 33 etc. What is the DIN? What do you

use it for? The ASA/DIN knob is actually on top of another knob that is

marked 2.8, 4, 5.6, 8, 11, 16, and 22. What is this? F-stop? (a wild guess)

What should this be set to?

Hope all you photographers aren't cringing too bad...

Thanks,

Daniel Lockhart

From:          "Malcolm Watts" <MALCOLM@directorate.wnp.ac.nz>

Organization:  Wellington Polytechnic, NZ

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Date:          Wed, 23 Sep 1998 12:28:38 +1200

Subject:       Re: Photographing Tesla Coils (PHOTO 101)

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Hi Daniel,

Best photos I've taken are with no ambient light at all

(gap usually supplies a bit), aperture f4 - f5.6 and shutter on

infinity (bulb) with anything from 2s to 6s exposure depending on how

many sparks you want to capture (5s gives a nice display).

Malcolm

> Date sent:      Tue, 22 Sep 1998 18:00:06 -0600

> To:             "Tesla-2 Mailing List" <tesla-2@emachine.com>

> From:           Daniel Lockhart <molly@cmn.net>

> Subject:        Photographing Tesla Coils (PHOTO 101)

> Send reply to:  "Tesla-2 Mailing List" <tesla-2@emachine.com>

> Hey all,

>

> I know this subject comes up a lot, so I thought I'd bring it up again. :-)

>

> What is the best way to take photos of Tesla Coils? I did a little

> experiment with the only "unelectric" camera I had with different shutter

> speeds. I'm no photographer, and I had hiccups while shooting the

> several-second-long shots without a tripod, so things turned out a little

> fuzzy. Nine out of the thirty-two were decent, 28% not bad, huh? Especially

> with a little camera I found in the garage...

>

> Anyway, on this camera I have a shutter speed (infinity to 500), focus, ASA

> (25 to 1600), DIN (15 to 33), and a knob that measures from 22 to 2.8.

>

> I know how to use the shutter and focus (imagine that?). I set the ASA to

> whatever the film is, am I correct? The ASA and DIN scales are on the same

> knob, just opposite each other, thus when you select one, you select the

> other. ASA 25 = DIN 15, ASA 1600 = DIN 33 etc. What is the DIN? What do you

> use it for? The ASA/DIN knob is actually on top of another knob that is

> marked 2.8, 4, 5.6, 8, 11, 16, and 22. What is this? F-stop? (a wild guess)

> What should this be set to?

>

> Hope all you photographers aren't cringing too bad...

>

> Thanks,

>

> Daniel Lockhart

>

>

>

Date: Tue, 22 Sep 1998 19:34:58 -0500

From: Tom Polk <tpolk@prismnet.com>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Photographing Tesla Coils (PHOTO 101)

Sender: tesl-err@emachine.com

Daniel Lockhart wrote:

>

> I know how to use the shutter and focus (imagine that?). I set the ASA to

> whatever the film is, am I correct?

Yes.

>What is the DIN?

It's the European version of ASA. It is redundant so disregard it.

>another knob that is

> marked 2.8, 4, 5.6, 8, 11, 16, and 22. What is this? F-stop?

Yes. If you want short exposures set it to a low number. Since 5.6 is

2.8 times two, a 5.6 setting requires the shutter to stay open twice as

long as 2.8.  You want to use the lower number F stops when you want the

background to be out of focus and your subject in focus. You use the

higher number F stops when you want everything to be in focus, (at the

expense of longer exposures).

> What should this be set to?

In a word, experiment. 400 ASA you can get away with 2-3 sec exposures

anywhere from F 2.8 to 11. Lots of my exposures were 7-15 seconds when I

wanted the visual effect of arcs going everywhere. You should use a

tripod and cable release for the shutter or use the camera timer to

avoid camera movement.

>

> Hope all you photographers aren't cringing too bad...

Naww. It's refreshing, you know, how golf pros would rather teach

someone who's never golfed before?

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Magnet Wire Get it while it's hot !

X-Mailer: IPAD 2.0

From: break7@juno.com (Tyla A. Break)

Date: Tue, 22 Sep 1998 23:16:49 EDT

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Ten dollars a pound!!!??? Lets not do anything rash. I'm in

California and I get it at APEX . This store is pretty much all

electrical surplus. I haven't found #22 yet, but, I'm sure they have it.

Would #21 or #23 be okay? I think I'll have Igor to help me Sat.

morning... ? 9:00 a.m.?  What state do you live in? Maybe I can ship you

some?

If it's not dangerous it's not fun!  Kyle  break7@juno.com

Date: Tue, 22 Sep 1998 21:11:54 -0700

From: mike nicholas <705728@ican.net>

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Photographing Tesla Coils (PHOTO 101)

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

My first time i used 1000 speed film and set my shutter speed, and aperture

when my coil was running with the camera's light meter, The use of a tripod is

necessary, I set the camera on a 3 second delay so I didn't accidentally bump

it, I got about 20 of 24 that were spectacular, but when you use  his high

speed film it tends to lighten the surrounding area.

Daniel Lockhart wrote:

> Hey all,

>

> I know this subject comes up a lot, so I thought I'd bring it up again. :-)

>

> What is the best way to take photos of Tesla Coils? I did a little

> experiment with the only "unelectric" camera I had with different shutter

> speeds. I'm no photographer, and I had hiccups while shooting the

> several-second-long shots without a tripod, so things turned out a little

> fuzzy. Nine out of the thirty-two were decent, 28% not bad, huh? Especially

> with a little camera I found in the garage...

>

> Anyway, on this camera I have a shutter speed (infinity to 500), focus, ASA

> (25 to 1600), DIN (15 to 33), and a knob that measures from 22 to 2.8.

>

> I know how to use the shutter and focus (imagine that?). I set the ASA to

> whatever the film is, am I correct? The ASA and DIN scales are on the same

> knob, just opposite each other, thus when you select one, you select the

> other. ASA 25 = DIN 15, ASA 1600 = DIN 33 etc. What is the DIN? What do you

> use it for? The ASA/DIN knob is actually on top of another knob that is

> marked 2.8, 4, 5.6, 8, 11, 16, and 22. What is this? F-stop? (a wild guess)

> What should this be set to?

>

> Hope all you photographers aren't cringing too bad...

>

> Thanks,

>

> Daniel Lockhart

From: "Kent Rankin" <kentrankin@theatreorgans.com>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Magnet Wire Get it while it's hot !

Date: Wed, 23 Sep 1998 00:23:47 -0400

X-Mailer: IPAD 2.0

X-MimeOLE: Produced By Microsoft MimeOLE V4.72.2106.4

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

I found #22 magnet wire for a little over $4 for a little over 3000

feet(I forget how many pounds).  Have I found a wonderful resource, or are

things just really pricey in California(what am I saying?  things are always

really pricey in CA)?

BTW, I found this price at a place that rewinds electrical motors.

-Kent Rankin

-----Original Message-----

From: Tyla A. Break <break7@juno.com>

To: Tesla-2 Mailing List <tesla-2@emachine.com>

Date: Tuesday, September 22, 1998 11:23 PM

Subject: Magnet Wire Get it while it's hot !

> Ten dollars a pound!!!??? Lets not do anything rash. I'm in

>California and I get it at APEX . This store is pretty much all

>electrical surplus. I haven't found #22 yet, but, I'm sure they have it.

>Would #21 or #23 be okay? I think I'll have Igor to help me Sat.

>morning... ? 9:00 a.m.?  What state do you live in? Maybe I can ship you

>some?

>

> If it's not dangerous it's not fun!  Kyle  break7@juno.com

>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Wire Color

X-Mailer: IPAD 2.0

From: break7@juno.com (Tyla A. Break)

Date: Wed, 23 Sep 1998 00:36:04 EDT

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

A lot of the wire is red, bright red. Some is green and most is

brownish-copper colored. I have a spool of #17 that is red, might be

2000'  There are several coils of #20 and #18 last time I checked,~ 2

weeks ago.

If it's not dangerous it's not fun!  Kyle  break7@juno.com

From:          "Malcolm Watts" <MALCOLM@directorate.wnp.ac.nz>

Organization:  Wellington Polytechnic, NZ

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Date:          Wed, 23 Sep 1998 16:57:18 +1200

Subject:       Re: Wire Color

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Hi Tyla,

If you are doing a closewind and want to varnish your coil,

steer clear of the red stuff. It is self-fluxing and the "enamel" can

melt in some varnish solvents. Not sure about the green. Brown is

best.

Malcolm

> To:"Tesla-2 Mailing List" <tesla- 2@emachine.com>

> Subject:        Wire Color

> From:           break7@juno.com (Tyla A. Break)

> Date sent:      Wed, 23 Sep 1998 00:36:04 EDT

> Send reply to:  "Tesla-2 Mailing List" <tesla-2@emachine.com>

>     A lot of the wire is red, bright red. Some is green and most is

> brownish-copper colored. I have a spool of #17 that is red, might be

> 2000'  There are several coils of #20 and #18 last time I checked,~ 2

> weeks ago.

>

>     If it's not dangerous it's not fun!  Kyle  break7@juno.com

>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: R.E. Wire Color

X-Mailer: IPAD 2.0

From: break7@juno.com (Tyla A. Break)

Date: Wed, 23 Sep 1998 01:13:22 EDT

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

The red wire I'm getting is called Nylese, is that as in Nylon?

Also , what is self fluxing?

If it's not dangerous it's not fun!  Kyle  break7@juno.com

From:          "Malcolm Watts" <MALCOLM@directorate.wnp.ac.nz>

Organization:  Wellington Polytechnic, NZ

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Date:          Wed, 23 Sep 1998 17:30:44 +1200

Subject:       Re: R.E. Wire Color

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

> To:             "Tesla-2 Mailing List" <tesla-2@emachine.com>

> Subject:        R.E. Wire Color

> From:           break7@juno.com (Tyla A. Break)

> Date sent:      Wed, 23 Sep 1998 01:13:22 EDT

> Send reply to:  "Tesla-2 Mailing List" <tesla-2@emachine.com>

>     The red wire I'm getting is called Nylese, is that as in Nylon?

> Also , what is self fluxing?

It means that when you apply heat with a soldering iron, the enamel

becomes a soldering flux and peels back, burns, melts etc. etc. The

kind of wire you do not use in transformers and motors built to

withstand stress and last. One of the main vendors in the vicinity

here won't sell it on principle. It has its uses but....  Check some

varnish out on it before you wind is my advice. I found out the hard

way when I varnished a transformer winding eons ago.

Malcolm

Date: Wed, 23 Sep 1998 10:41:47 GMT

From: pacster@ibm.net

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: help needed with sync gap

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Hello everyone I was hoping to improve my system and from my armchair I

thought that moving to a sync gap was the way to go, so I picked up a second

hand induction motor which runs at 2800rpm, pulled it apart, became confused

and decided to ask for some help.

The first problem was that I want 100BPS (50Hz mains) so I could mill 2 or 4

flats on the rotor for 3000 or 1500 rpm, but at 3000 rpm I'm a bit worried

that the rotor might self destruct so it looks as though 1500rpm is the way

to go. Now at 1500rpm the rotor will turn 25 times a second, so should I

have eight or four moving electrodes to make the gap fire at 100BPS ?(I was

thinking of designing the rotor to be an aluminium disk with contacts bolted

to the edge, the spark will jump diametrically across the disk between two

contacts on either side of the disk).

The next problem is what electrode size should I go for and at what pitch

line velocity? Does the dwell time have to be equal to 1/freq or does it

make no difference. I was thinking of using a 12" disk and 3/8" electrodes

which would give a slow dwell time of around 1.2mS.

Would It be possible for someone to give some details on a sync gap which

has worked, It would be a great help.

Thanks heaps for your time,

Mark B.

Date: Wed, 23 Sep 1998 07:18:45 -0700

From: Gene Waldo <gwaldo@1stnetusa.com>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: R.E. Wire Color

Sender: tesl-err@emachine.com

Tyla A. Break wrote:

>

>         The red wire I'm getting is called Nylese, is that as in Nylon?

> Also , what is self fluxing?

>

Kyle,

Some enamels especially the green, strip back when exposed to

soldering.  The VERY fine leads we use in strain gage installation are

called strip-ease or some such. You just flux the lead, tin and solder

thru the enamel-- it melts away.

I don't see how this would present a problem for winding, though. Below

500F it should be fine. Arc sites may cause problems, however....

--

Gene Waldo -  gwaldo@1stnetusa.com  - "Igor in Residence"

---------------------------------------------------------------

What's good for the planet is more OZONE! See the machinery form up!

NEW PAGE!   http://members.1stnetusa.com/danmurphy/waldo.htm

X-Originating-IP: [199.217.140.22]

From: "Bill the arcstarter" <arcstarter@hotmail.com>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: R.E. Wire Color

Date: Wed, 23 Sep 1998 07:44:54 PDT

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

Gene wrote:

>Some enamels especially the green, strip back when exposed to

>soldering.  The VERY fine leads we use in strain gage installation are

>called strip-ease or some such. You just flux the lead, tin and solder

>thru the enamel-- it melts away.

>

>I don't see how this would present a problem for winding, though. Below

>500F it should be fine. Arc sites may cause problems, however....

I think Malcolm found that certain varnishes (polyurethane?) will eat

the enamel off the self-fluxing wire.  Thats a bad thing on a

secondary...  :)

Also - if you have arcs to or within your secondary I doubt any type of

insulation will keep things running very long!

Later.

-Bill

X-Mailer: IPAD 2.0

Date: Wed, 23 Sep 1998 13:03:09 -0500

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: R.E. Wire Color

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

At 07:44 AM 9/23/98 PDT, you wrote:

>Gene wrote:

>

>>Some enamels especially the green, strip back when exposed to

>>soldering.  The VERY fine leads we use in strain gage installation are

>>called strip-ease or some such. You just flux the lead, tin and solder

>>thru the enamel-- it melts away.

>>

>>I don't see how this would present a problem for winding, though. Below

>>500F it should be fine. Arc sites may cause problems, however....

>

>I think Malcolm found that certain varnishes (polyurethane?) will eat

>the enamel off the self-fluxing wire.  Thats a bad thing on a

>secondary...  :)

>

>Also - if you have arcs to or within your secondary I doubt any type of

>insulation will keep things running very long!

>

>Later.

>-Bill

Bill and all,

Ive used all the various magnet wire colors in secondary coils

without problems. My present secondary coil has the red insulation

and polyurethane varnish did not affect it in anyway. It works

quite well.

My older secondary has the brownish formvar insulation and my

smaller 3 incher has green insulation of unknown composition.

My mini coil is wound with AWG 32 and has blue insulation.

All coils worked very well. I wouldnt be overly concerned.

TTYL!

Mike Hammer

mhammer@misslink.net

X-Mailer: IPAD 2.0

Date: Wed, 23 Sep 1998 14:10:20 -0500

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: Magnet Wire

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

At 10:45 PM 9/21/98 EDT, you wrote:

>	Hi. Mike Nice score at the hamfest!    I don't have any trouble

>getting magnet wire. #9-10-14 17-18-19-20-maybe 22 (haven't found it yet

>) , small stuff like 30-40. I get it for $2/lb. Is that expensive? I

>bought about 5 spools of assor. sizes. I can look for 22 when I go next

>time. Maybe Igor wants to go Sat. morning ?? Full spools are usually $20.

>Shipping might ruin the deal   10# to OH was it?

>

>	If it's not dangerous it's not fun!  Kyle  break7@juno.com

>

Kyle,

Thanks for the offer. $2 per pound is quite good. Typical prices

range from $4-6 per pound. Unfortunately shipping from California

to Illinois would probably make it much less attractive.

Since my finances are at a low ebb I was hoping someone had

a partial spool of AWG 22 laying around with 4 or 5 pounds left

on it I could get cheap. Enough for a six inch secondary.

TTYL!

Mike Hammer

mhammer@misslink.net

From:          "Malcolm Watts" <MALCOLM@directorate.wnp.ac.nz>

Organization:  Wellington Polytechnic, NZ

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Date:          Thu, 24 Sep 1998 09:00:09 +1200

Subject:       Re: help needed with sync gap

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Hi there,

> Date sent:      Wed, 23 Sep 1998 10:41:47 GMT

> From:           pacster@ibm.net

> To:             "Tesla-2 Mailing List" <tesla-2@emachine.com>

> Subject:        help needed with sync gap

> Send reply to:  "Tesla-2 Mailing List" <tesla-2@emachine.com>

> Hello everyone I was hoping to improve my system and from my armchair I

> thought that moving to a sync gap was the way to go, so I picked up a second

> hand induction motor which runs at 2800rpm, pulled it apart, became confused

> and decided to ask for some help.

>

> The first problem was that I want 100BPS (50Hz mains) so I could mill 2 or 4

> flats on the rotor for 3000 or 1500 rpm, but at 3000 rpm I'm a bit worried

> that the rotor might self destruct so it looks as though 1500rpm is the way

> to go. Now at 1500rpm the rotor will turn 25 times a second, so should I

> have eight or four moving electrodes to make the gap fire at 100BPS ?(I was

> thinking of designing the rotor to be an aluminium disk with contacts bolted

> to the edge, the spark will jump diametrically across the disk between two

> contacts on either side of the disk).

There must be a total of four presentations per rev to hit 100 BPS so

four moving electrodes evenly spaced will do it. Or, two moving (got

to maintain wheel balance so four is probably better) and at least

two stationary, the two stationary ones being 90 degrees apart (no

balancing reauired).

> The next problem is what electrode size should I go for and at what pitch

> line velocity? Does the dwell time have to be equal to 1/freq or does it

> make no difference. I was thinking of using a 12" disk and 3/8" electrodes

> which would give a slow dwell time of around 1.2mS.

Electrical dwell is generally independent of wheel speed unless the

speed is humungously high with huge spacing between electrodes. The

spark will start when the electrodes get close enough and finish when

the system has lost sufficient energy by and large.

Malcolm

From:          "Malcolm Watts" <MALCOLM@directorate.wnp.ac.nz>

Organization:  Wellington Polytechnic, NZ

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Date:          Thu, 24 Sep 1998 09:02:40 +1200

Subject:       Re: R.E. Wire Color

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

> Date sent:      Wed, 23 Sep 1998 13:03:09 -0500

> To:             "Tesla-2 Mailing List" <tesla-2@emachine.com>

> From:           Mike Hammer <mhammer@misslink.net>

> Subject:        Re: R.E. Wire Color

> Send reply to:  "Tesla-2 Mailing List" <tesla-2@emachine.com>

> At 07:44 AM 9/23/98 PDT, you wrote:

> >Gene wrote:

> >

> >>Some enamels especially the green, strip back when exposed to

> >>soldering.  The VERY fine leads we use in strain gage installation are

> >>called strip-ease or some such. You just flux the lead, tin and solder

> >>thru the enamel-- it melts away.

> >>

> >>I don't see how this would present a problem for winding, though. Below

> >>500F it should be fine. Arc sites may cause problems, however....

> >

> >I think Malcolm found that certain varnishes (polyurethane?) will eat

> >the enamel off the self-fluxing wire.  Thats a bad thing on a

> >secondary...  :)

The varnish that destroyed my wire was a proper electrical varnish

manufactured by an outfit called Electropar. It was designed for air

drying. It is mostly used to stop windings vibrating in motors but it

does have excellent breakdown properties. Anyway, test whatever you

are going to use first. Better safe than sorry.

Malcolm

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Low Vo Coil

X-Mailer: IPAD 2.0

From: break7@juno.com (Tyla A. Break)

Date: Wed, 23 Sep 1998 22:58:19 EDT

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Hi guys . Sunday I finished rebuilding my flat stack cap. I added

a few more sheets to reach .0128uf. I was dying to try it out so I

started at out at 3750v .I got a little 3"x12" coil and put it in the

middle of my 32" primary, put a 4"x 17" toroid on it and started tuning.

once all the air was out I tried 7500v . I know that's too high but I had

to try. It has been working great! I'm getting solid 18" performance. I

didn't think it would work that good at that low a voltage. So I'm

wondering is the spark length more a function of voltage? I'm using my

15kv 60ma - one leg to ground. So a low 7500v but 60ma. I also noticed

right when the last of the air came out of the cap the power out put went

way up. Tuning is very critical there is a noticeable difference in

output when moving the tap 1/16 of a turn. So now I have to build 3? or 8

more for 15kv? Right now .0128 is too small to tune in my big coil -

without a toroid.

BTW I've wound a coil with the red wire and coated it with

Varathane and it looks and works just fine.

If it's not dangerous it's not fun!  Kyle  break7@juno.com

Date: Wed, 23 Sep 1998 22:11:20 -0600

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Daniel Lockhart <molly@cmn.net>

Subject: Re: Low Vo Coil

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

Tyla wrote:

>Hi guys . Sunday I finished rebuilding my flat stack cap. I added

>a few more sheets to reach .0128uf. I was dying to try it out so I

>started at out at 3750v .I got a little 3"x12" coil and put it in the

>middle of my 32" primary...

>> snip! <<

>BTW I've wound a coil with the red wire and coated it with

>Varathane and it looks and works just fine.

Hey! Three inches isn't that small! I resemble that comment!  :-)

Thanks for the wire info.

Daniel Lockhart

X-Mailer: IPAD 2.0

Date: Fri, 25 Sep 1998 02:47:08 +0100

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: bob golding <yubba@clara.net>

Subject: resonant charging

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Hi all,

I just checked the parameters of my system with the lcr bridge again and

noticed that my cap value has gone down quite a lot from my estimated

value. I designed it to give resonant charging with my 9 kv 60 mA neon. I

made this 17 nF, my cap now measures 12nF. The primary measures 22uH, and

the secondary measures 13 mH. I am using a 18" x  3" toroid. I have 9 turns

on the primary. Will I need to change anything to get the best out of this

set up considering the lower cap value? i am getting 24" sparks to a

grounded rod at the moment. I am using 150 thou of gap with vacuum quenching.

many thanks.

bob golding

bob yubba@clara.net

X-Originating-IP: [208.16.149.199]

From: "Bill the arcstarter" <arcstarter@hotmail.com>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: resonant charging

Date: Thu, 24 Sep 1998 18:59:45 PDT

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

Bob wrote:

>Hi all,

>	I just checked the parameters of my system with the lcr bridge again and

>noticed that my cap value has gone down quite a lot from my estimated

>value. I designed it to give resonant charging with my 9 kv 60 mA neon.

>I made this 17 nF, my cap now measures 12nF. The primary measures 22uH, and

>the secondary measures 13 mH. I am using a 18" x  3" toroid. I have 9 turns

>on the primary. Will I need to change anything to get the best out of this

>set up considering the lower cap value? i am getting 24" sparks to a

>grounded rod at the moment. I am using 150 thou of gap with vacuum quenching.

We saw about an 8% decrease in our capacitance over a 2 month period.

Someone suggested it was due to the poly absorbing the oil and swelling

apart, thus increasing the distance between the plates.

Somebody else claimed that the swelling poly plates snapped the nylon

tie-wraps holding his plate cap together!

I would guess that other than retuning, you shouldn't have to adjust

much of anything for good results!

-Bill

Date: Thu, 24 Sep 1998 21:04:29 -0500

From: Bert Hickman <bert.hickman@aquila.com>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Organization: Stoneridge Engineering

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: resonant charging

Sender: tesl-err@emachine.com

Bob,

Overall, the coil seems to be operating as expected, with Fo in the 300

kHz range. Are you getting multiple streamers or a single streamer off

the toroid. If the former, you may need to go to a bit larger radius of

curvature - try 4". Also, you might want to try opening up the gap a bit

more to see if this improves performance.. but not too much! The biggest

change would be to get closer to 0.017 uF so that you can reap the

benefits of resonance charging (higher breakrates with the same gap

setting), and longer streamers.

Good Luck!

-- Bert --

bob golding wrote:

>

> Hi all,

>         I just checked the parameters of my system with the lcr bridge again and

> noticed that my cap value has gone down quite a lot from my estimated

> value. I designed it to give resonant charging with my 9 kv 60 mA neon. I

> made this 17 nF, my cap now measures 12nF. The primary measures 22uH, and

> the secondary measures 13 mH. I am using a 18" x  3" toroid. I have 9 turns

> on the primary. Will I need to change anything to get the best out of this

> set up considering the lower cap value? i am getting 24" sparks to a

> grounded rod at the moment. I am using 150 thou of gap with vacuum quenching.

>

>  many thanks.

> bob golding

>

> bob yubba@clara.net

X-Originating-IP: [207.100.20.10]

From: "Philip Mac Duffie" <punk_42@hotmail.com>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Flyback

Date: Thu, 24 Sep 1998 19:13:59 PDT

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

Hi all, sorry to be off topic ,but im looking for a guy named Mark that

sent me a flyback after talking in the #tesla_coil irc channel.

Im having problems with it and would like to add him to my list of names

on the credits of my web page :)

Thanks,

Philip Mac Duffie

X-Originating-IP: [208.16.149.199]

From: "Bill the arcstarter" <arcstarter@hotmail.com>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: PIG rating plate decoding?

Date: Thu, 24 Sep 1998 19:20:23 PDT

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

Coilers,

This has been discussed before so I am just writing this to verify the

information.

Turns out my pig DOES have a rating plate: 33200 GRDY/19200

In my case, if I apply 240V to the 120-120 inputs, I get 19200 measured

from the HV bushing to the case (my pig has the 'other' HV lead

internally tied to the case).

Therefore I can verify this transformer was likely part of a 3-phase

Y-distribution system, since 19200 = (approx) 33200/sqrt(3).  Or,

alternately, the line to neutral voltage was 19200 and the line to line

voltage was 33200.

Perhaps knowing what this "33200 GRDY/19200" format actually means will

help some other coiler looking for an appropriate pig.

If anyone else knows how to decode any other popular rating plate

formats, I think that'd be useful information to post to the lists!

-Bill

X-Mailer: IPAD 2.0

Date: Thu, 24 Sep 1998 23:40:37 -0500

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: Error in my caps calc, me rethinkum

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

At 10:12 AM 9/24/97 -0700, you wrote:

>

>

>Ok everyone, I posted a couple of months ago about the caps I was

>planning to build, and I just looked back at the post and saw that

>I made a mistake that no one noticed!!

>I was planning to build two .005 uf caps, and place them in series...

>Uhem but this is only .0025 uf !!! I think everyone (including myself)

>forgot that my caps were to be wired in series not parallel (I'm using

>two 15kv 30ma neons for a total of 60ma).

>

>Ok, so here's the deal. I completed (except for the oil) a .005 uf rolled

>cap with the following specs:

>

>8 layers of 6 mil poly (total thickness of .048)

>Plate size 48" x 10"

>

>This gives me around .008 uf at 16800V ...

>

>Now everyone has said that even though the voltage rating of this cap

>seems high enough for a 15kv neon, the cap will still fail due to internal

>corona at 15KV. This means I can't wire two of these in parallel.

>

>So here's my solution:

>

>If I remove 4 layers of poly, this gives me the following specs:

>

>4 Layers of 6mil (total thickness of .024)

>Plate size 48" x 10"

>

>I get around .017uf at 8400 V.  If I run two of these in SERIES I end up with

>.0085uf.  What I REALLY need is .0106uf, but .0085 might be close enough to get me

>off of my salt water caps. If I'm lucky I could end up around .009 which is fairly

>close to what I need. Besides, I could always make the second one with slightly

>longer plates...say 56"...this would put me right up around .010uf total (when

>wired in series with the first one).

>

>What should I do??

>Thanks

>

>-tom

>tomstock@geocities.com

>

>

Tom,

Two caps with 24 mils of poly in series wont make it at 15kv.

Ive used caps like that for 12kv but its not quite enough to handle 15kv.

You should make each cap with at least 30 mils of poly and adjust

plate size to get the capacitance you need.

Or even better use 3 caps each with 24 mils in series for a total

of 72 mils.

TTYL!

Mike Hammer

mhammer@misslink.net

From: Hollmike@aol.com

Date: Fri, 25 Sep 1998 08:23:34 EDT

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: resonant charging

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Bill, all,

I am slowly getting a plate cap together.  I have finished cutting all the

plates and dielectric sheets(.062 LDPE)s and am now ready for the fun of

cleaning and assembling the works.  I had an idea for preventing excessive

swelling that I thought I'd toss out and get some feedback:

I am planning to use two thick sheets of either PVC or some other

material, maybe wood, with bolt holes drilled around the edges(with enough

spacing to prevent possible arcing or corona troubles) to hold the assembled

cap together.  It seems to me that as long as whatever material I use won't

warp or absorb the mineral oil, this should prevent swelling of the LDPE,

except around the edges of the plates where it would tend to "seal " the

borders and maybe provide better corona resistance at the edges of the plates.

As long as the air is removed from the cap before the swelling can occur, this

seems as though this would be an ideal way to hold the cap together and

minimize any changes in capacitance over time.

Any comments?

Mike Hollingsworth

Date: Fri, 25 Sep 1998 09:49:56 -0700

From: Gene Waldo <gwaldo@1stnetusa.com>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Magnet Wire

Sender: tesl-err@emachine.com

Mike Hammer wrote:

>

> Kyle,

> Thanks for the offer. $2 per pound is quite good. Typical prices

> range from $4-6 per pound. Unfortunately shipping from California

> to Illinois would probably make it much less attractive.

>

> Since my finances are at a low ebb I was hoping someone had

> a partial spool of AWG 22 laying around with 4 or 5 pounds left

> on it I could get cheap. Enough for a six inch secondary.

>

> TTYL!

>

> Mike Hammer

> mhammer@misslink.net

Mike,

If all goes as planned Kyle & I should meet at APEX Sat am.

We'll seek out some 22 ga.

-------------------------------------

Gene Waldo -  gwaldo@1stnetusa.com  - "Igor in Residence"

---------------------------------------------------------------

What's good for the planet is more OZONE! See the machinery form up!

NEW PAGE!   http://members.1stnetusa.com/danmurphy/waldo.htm

Date: Fri, 25 Sep 1998 19:00:17 +0200 (CEST)

From: Jochen Kronjaeger <Kronjaeg@stud-mailer.uni-marburg.de>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: X-ray & Oudin coil

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

>  So can anyone explain what is fluorescing when these evacuated bulbs

> are placed on the tesla coil?

If the tube is well enough evacuated to produce x-rays, there should be no

noticable glow from the remaining gas itself.

The glass wall emits light (flourescence) as well as x-rays when hit by

fast electrons.

Jochen

---------------------------------------------------------------------------

Jochen Kronjaeger                     Kronjaeg@stud-mailer.uni-marburg.de

The High Voltage Page  http://www.mathematik.uni-marburg.de/~kronjaeg/hv/

info, plans, photos of home-built HV-sources, sparks, HV-experiments

---------------------------------------------------------------------------

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Litz Wire?

X-Mailer: IPAD 2.0

From: break7@juno.com (Tyla A. Break)

Date: Fri, 25 Sep 1998 23:21:26 EDT

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Hi all, My brother wanted to get some wire today. So an impromptu

run to APEX at 8:30. A.M. (Still need to go tomorrow). My bro is in the

one-upsmanship mode  and is winding a 13" coil! Well , he found a 57lb

coil of litz wire and had to buy it. It's seven wires, #31 or something.

It totals about #18. There must be 10,000 feet on the spool. He's

borrowed my motorized winding table, and already wound the form.(13" x

40") ~900 turns ~3000'   Does anybody have any comments or questions ? Is

litz wire going to make any noticeable difference? I was able to get the

litz wire and a positively pregnant spool of #19 (21lbs) for $1.75/ lb!!

BTW I'm still playing with my coil. I kept the gap small but

moved up to 2400w (pole pig). Performance about doubled! Gained about 4",

but arcs look 5 times thicker. Also instead of being thin violet

streamers they've become bright blue-white and pretty dangerous looking!

I like it!. A lot! But the power seems to have been too much I have since

incinerated the tiny 3" secondary (sorry if your coil is a 3 incher) A

large strike to the middle fused seven turns together creating a large

shorted winding. So , I'll be winding up a 6.625 er for tomorrow.

If it's not dangerous it's not fun!  Kyle  break7@juno.com

Date: Fri, 25 Sep 1998 22:15:18 -0600

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Daniel Lockhart <molly@cmn.net>

Subject: Re: Litz Wire?

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

Tyla wrote:

>> snip! <<

>	BTW I'm still playing with my coil. I kept the gap small but

>moved up to 2400w (pole pig). Performance about doubled! Gained about 4",

>but arcs look 5 times thicker. Also instead of being thin violet

>streamers they've become bright blue-white and pretty dangerous looking!

>I like it!. A lot! But the power seems to have been too much I have since

>incinerated the tiny 3" secondary (sorry if your coil is a 3 incher) A

>large strike to the middle fused seven turns together creating a large

>shorted winding. So , I'll be winding up a 6.625 er for tomorrow.

>

>	If it's not dangerous it's not fun!  Kyle  break7@juno.com

Ouch! How much current to you give your 3-incher? 166 ma? That's a lot for

a little, ugh, err, coil like that! I don't know if I could push mine past

120 ma without something happening. I guess I'll have to see...

Be sure to let us know how your 6.625-incher turns out. I'm kind of

collecting data on 6.625-incher performance. I'm sure you won't be

dissapointed!

Daniel Lockhart

Date: Sat, 26 Sep 1998 20:10:27 -0700

From: Gene Waldo <gwaldo@1stnetusa.com>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: REPORT:  Waldo-Murphy visits Kyle Break, et al....*

Sender: tesl-err@emachine.com

Well, the day started with a troop to APEX, where I and Kyle and his

brother, *Randy, met and looked for neat stuff.  Seems there wasn't any,

because we all came away empty handed. Seems that they (APEX) have a deal

to sell all 22-24 ga mag wire to some weenie Xfmr mfr! This may explain

the dearth of decent sized wire! Kyle instituted a side deal, where-by,

some 20-24 ga wire may get diverted.... Stay tuned.

Kyle & Randy are neat guys.  One has a house devoted to family and

kinder, the other's is given over to TESLA; Beds are covered in TESLA

paraphinelaia-erisioni, or however it's spelled.  Pigs & piglets are

there.  Along with a bunch of 10-12" wound 2ndary & numerous pri forms.

The 2ndary winder is a simple joy. $40 of Bodine, some 2x4's, belt &

shives.  These guys are both pro electricians, so no wonder they are up

on the game.

Murphy & I spent a happy 2-3 hours in watching sparks and discussing

what-not. There's more here than met the camera.   Kyle and 6" 2ndary

begin the photo tour. If I've screwed up on data, Kyle'll have to correct

the record.

--------------------------------------------------------

Gene Waldo -  gwaldo@1stnetusa.com  - "Igor in Residence"

---------------------------------------------------------------

What's good for the planet is more OZONE! See the machinery form up!

NEW PAGE!   http://members.1stnetusa.com/danmurphy/waldo.htm

Attachment Converted: "C:\MISSING_LINK\EUDORA\Attach\P_08_12.jpg"

Attachment Converted: "C:\MISSING_LINK\EUDORA\Attach\P_08_14.jpg"

(editors note. binary attachments not in archive)

From: "Ryan E. Jester" <ryanj@primary.net>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Cleaning the Secondary

Date: Sun, 27 Sep 1998 02:13:09 -0500

X-Mailer: IPAD 2.0

X-MimeOLE: Produced By Microsoft MimeOLE V4.72.3110.3

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Hardware stores carry PVC pipe cleaner.   It'll be in with the PVC glue and

primer.  Anyway, this takes off the printing quite easily.  I'll have to

look to see what type of solvent it really is.

Ryan Jester

ryanj@primary.net

From: ralph.hartwell@emachine.com (Ralph Hartwell)

Subject:  Offline due to storm!

Date: Sun, 27 Sep 1998 07:35:00 GMT

Organization: The Energy Machine Information System 504-733-3462

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

I'm posting this message to all my friends on several lists and

conferences.

I'll do a final mail run later this morning to check for any messages

before I pull the plug here...

As you may be aware, hurricane Georges is slowly approaching the Gulf

Coast and New Orleans.  This is a bad sucker, and if it does continue in

on the predicted course, we will be in for a horrible time.  I work for

a TV station here, so I get the straight info about the situation.  It

ain't very pleasant.  And I'll have to leave my house and go to work, so

whatever happens here at or to my house will just happen.  I'll find out

about it when and if I get back.

Due to the fact that there is a very good chance this time that my

house will have between 3 and 8 feet of water in it, I will be shutting

down the computer equipment here by late tomorrow.  I expect to be

offline for a few days, maybe longer, depending on what happens.  If you

need to contact me you can try my alternate address,

ralph.hartwell@worldnet.att.net.  I am taking my portable computer (and

my backup tapes) and, god willing, will check for mail when I can.

My mailing lists, TESLA-2 and NEWMAN-L are operated on the Southern

Star server (sstar.com) which is also located in this area.  I don't

know his situation, but even if he does not get flooded, we may be

without power for several weeks, so those lists may also go down.

Since I am shutting my system down, that means that my BBS, The

Energy Machine! will also be offline for the duration.  This will be

only the second time that the BBS has been shut down in ten years of

operation.

It's so strange, walking outside, and looking up at a clear sky, with

the stars shining brightly, not a breath of wind blowing, and knowing

that less than 250 miles away is the storm with winds of 100+ MPH...

and knowing that late tomorrow, it will likely be right here.

Ralph   W5JGV

---

You, you, and you, panic.  Everyone else, come with me.

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: 18" for nothing!!

X-Mailer: IPAD 2.0

From: break7@juno.com (Tyla A. Break)

Date: Sun, 27 Sep 1998 19:26:08 EDT

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Hi all ,

I had Gene and Dan over Sat. Thanks for coming! If you sent pics

to juno the problem is it only recognizes text. No problem, better pics

are now avail.!

More testing and tuning netted ~ 20+ inches.

First I raised the toroid 6" this lowered the operating hz ? I am

now clipped out another turn. I was up to 30" of spark. Adding a fan to

the gap gave me another 18" !! Sparks are also comming out all over the

toroid instead of just one spot.

If it's not dangerous it's not fun!  Kyle  break7@juno.com

Date: Sun, 27 Sep 1998 16:38:18 -0700

From: Gene Waldo <gwaldo@1stnetusa.com>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: 18" for nothing!!

Sender: tesl-err@emachine.com

Tyla A. Break wrote:

>

>         Hi all ,

>          I had Gene and Dan over Sat. Thanks for coming! If you sent pics

> to juno the problem is it only recognizes text. No problem, better pics

> are now avail.!

Kyle,

*OK*! That's a load off my primary! Keep tuning! I'll try to resend last,

sans pics.

----------------------------------

Gene Waldo -  gwaldo@1stnetusa.com  - "Igor in Residence"

---------------------------------------------------------------

What's good for the planet is more OZONE! See the machinery form up!

NEW PAGE!   http://members.1stnetusa.com/danmurphy/waldo.htm

Date: Sun, 27 Sep 1998 17:28:16 -0700

From: Gene Waldo <gwaldo@1stnetusa.com>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: 18" for nothing!!

Sender: tesl-err@emachine.com

Tyla A. Break wrote:

>

>  More testing and tuning netted ~ 20+ inches.

>         First I raised the toroid 6" this lowered the operating hz ? I am

> now clipped out another turn. I was up to 30" of spark. Adding a fan to

> the gap gave me another 18" !!

Kyle, if I read that correctly:

We saw at least 24"--- you futzed around and got 30"; the fan added 18?

18+30=48???  Wowsers! Maybe I'll have to finally spend the money for the

telephoto attach for the camera!  It seems to have weathered the sat

session ok, but I don't know about bringing it close to your new

environment!

A REAL ser GAP and some more? caps; You won't be able to get near your

controls!

Sounds like a strike rail is in your future...... & a real RFGnd?

Question: in looking at the pics, I note 8" of piggie stand-offs-- and

the skinny assed wire therefrom drapes nonchalantly over the edge of the

pig-lid gnd!???? I guess the 'path of least..' stuff holds!

Tried playing with a lg ss salad bowl on top of torroid? Was raising the

tor a net good?  or a bad? I guess the more stuff you do to get tapped

at more pri turns, the better?

> Sparks are also comming out all over the

> toroid instead of just one spot.

>

SERIOUS QUESTION for the knowlegable ones:  I remain sorely tempted to

stick my finger into or near the discharge outer sphere of influence....

After of course testing the water with a grounded probe. What sorts of

dangers exist? Besides getting knocked on ones can?  The obvious

tendancy is to 'stand clear', but accidents do happen.

----------------------------------------------

Gene Waldo -  gwaldo@1stnetusa.com  - "Igor in Residence"

---------------------------------------------------------------

What's good for the planet is more OZONE! See the machinery form up!

NEW PAGE!   http://members.1stnetusa.com/danmurphy/waldo.htm

From: "Basura, Brian" <brian.basura@unistudios.com>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Cc: "'brianb@mail.hughes.net'" <brianb@mail.hughes.net>

Subject: First Light

Date: Mon, 28 Sep 1998 11:44:08 -0700

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Tesla List Members,

I fired up my 6.5" coil for the first time this weekend.

Started with tiny 6" sparks to a ground rod. I then tuned, reduced the

number of gaps, lowering the secondary, increased the size of the capacitor

safety gap, and tuned some more.  I was able to achieve a consistent 32"

breakout into the air and a couple of weak 35" strikes to the grounded rod.

This was at approximately 960 watts and utilizing a .014mfd capacitor.

I then upped my capacitance to .022mfd and re-tuned. The sparks were white

hot and loud.  The distance is a consistent 36" to a grounded target. The

sparks are hot and without a target tend to go upward at a 45 degree angle.

Raising the toroid an additional inch didn't seem to effect the performance

of the sparks but it did change their direction.  The sparks now go straight

out (horizontal) from the toroid.

I then varied the configuration of my current limiter for my PT. I started

with a MOT (shorted secondary) and series resistance.  I found the best

performance using a MOT measuring 11.7mH in parallel with 14.2 ohms.  There

was no HV current change as measured into a resistive load (80ma) but in

operation the color and sound of the sparks did intensify.

I am having a strange problem I can't identify.  As I ramp up the input

voltage the RQ style SG starts to fire (very consistent) and then strong

streamers break out.  After 9-10 seconds of operation the streamers loose

intensity and the SG starts to fire in intermittent bursts (1-2 seconds then

a 1-2 second pause). I've messed with air flow and more seems to make the

problem worse but reducing the airflow also makes it worse.  I did find an

undersized wire in my tank circuit (those gremlins must have switched wires

on me).  I'll fix that before my next run and see if that helps.  Any other

ideas would be welcome?

Before I shut it down last night I increased my input power to 1800W and

made one last run. A white hot 5' streamer came straight at me, curved up

and hit the garage door opener. WOW!!!  Then the safety gaps on my caps and

PT started to fire at which point I immediately shut down.  Obviously I need

to move outside, add my other capacitors (total .044mfd), and do more

tuning.

Construction pictures are at:

http://www.hughes-ec.com/~brianb/teslahome.html

I'll try and get some operating pictures up on the WEB next week.

Thanks to everyone who posts on these lists.  I've learned a tremendous

amount.

Regards,

Brian Basura

Date: Mon, 28 Sep 1998 15:05:26 -0400

From: Bert Wikkerink <bert@mitra.com>

Organization: Mitra Imaging Inc.

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: First Light

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Hi Brian,

Beautiful caps. One question, though. How did you attach the capacitor terminals

to the aluminum ?.

Thanks,

Bert W.

Basura, Brian wrote:

> Tesla List Members,

>

> I fired up my 6.5" coil for the first time this weekend.

>

> Started with tiny 6" sparks to a ground rod. I then tuned, reduced the

> number of gaps, lowering the secondary, increased the size of the capacitor

> safety gap, and tuned some more.  I was able to achieve a consistent 32"

> breakout into the air and a couple of weak 35" strikes to the grounded rod.

> This was at approximately 960 watts and utilizing a .014mfd capacitor.

>

> I then upped my capacitance to .022mfd and re-tuned. The sparks were white

> hot and loud.  The distance is a consistent 36" to a grounded target. The

> sparks are hot and without a target tend to go upward at a 45 degree angle.

> Raising the toroid an additional inch didn't seem to effect the performance

> of the sparks but it did change their direction.  The sparks now go straight

> out (horizontal) from the toroid.

>

> I then varied the configuration of my current limiter for my PT. I started

> with a MOT (shorted secondary) and series resistance.  I found the best

> performance using a MOT measuring 11.7mH in parallel with 14.2 ohms.  There

> was no HV current change as measured into a resistive load (80ma) but in

> operation the color and sound of the sparks did intensify.

>

> I am having a strange problem I can't identify.  As I ramp up the input

> voltage the RQ style SG starts to fire (very consistent) and then strong

> streamers break out.  After 9-10 seconds of operation the streamers loose

> intensity and the SG starts to fire in intermittent bursts (1-2 seconds then

> a 1-2 second pause). I've messed with air flow and more seems to make the

> problem worse but reducing the airflow also makes it worse.  I did find an

> undersized wire in my tank circuit (those gremlins must have switched wires

> on me).  I'll fix that before my next run and see if that helps.  Any other

> ideas would be welcome?

>

> Before I shut it down last night I increased my input power to 1800W and

> made one last run. A white hot 5' streamer came straight at me, curved up

> and hit the garage door opener. WOW!!!  Then the safety gaps on my caps and

> PT started to fire at which point I immediately shut down.  Obviously I need

> to move outside, add my other capacitors (total .044mfd), and do more

> tuning.

>

> Construction pictures are at:

> http://www.hughes-ec.com/~brianb/teslahome.html

>

> I'll try and get some operating pictures up on the WEB next week.

>

> Thanks to everyone who posts on these lists.  I've learned a tremendous

> amount.

>

> Regards,

> Brian Basura

From: "Basura, Brian" <brian.basura@unistudios.com>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: RE: First Light

Date: Mon, 28 Sep 1998 13:33:36 -0700

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Bert/all,

I punched holes in the Aluminum bus with a hole punch.  I then passed a

1/4"x20 round head brass bolt with washer through it.  This bolt (terminal)

was then coated with silicon and screwed from the bottom up into the lexan

cover  which I had previously tapped.  Where the bolt protruded from the top

of the lexan I added another washer and nut.  Immediately after tightening I

cleaned the threads of all silicone since these bolts are now the terminals.

The Aluminum bus was attached to my plates with small brass screws and nuts

(6x32 or 8/32 ?).  All screw edges were smoothed, covered with silicon, and

left to cure.  A couple of small 1" patches of LDPE were placed over the

screws for added protection.

One caution to this design.  The SDR35 end caps are easily broken when

pulling a vacuum.  To prevent this epoxy a 3/8" thick lexan circle into the

bottom of the end cap before gluing it onto the PVC pipe.

Regards,

Brian Basura

> ----------

> From: 	Bert Wikkerink[SMTP:bert@mitra.com]

> Reply To: 	Tesla-2 Mailing List

> Sent: 	September 28, 1998 12:05 PM

> To: 	Tesla-2 Mailing List

> Subject: 	Re: First Light

>

> Hi Brian,

>

> Beautiful caps. One question, though. How did you attach the capacitor

> terminals

> to the aluminum ?.

>

> Thanks,

>

> Bert W.

>

>

Date: Mon, 28 Sep 1998 13:54:55 -0700

From: Gene Waldo <gwaldo@1stnetusa.com>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: First Light

Sender: tesl-err@emachine.com

Basura, Brian wrote:

>

> Tesla List Members,

>

> I fired up my 6.5" coil for the first time this weekend.

>

> Construction pictures are at:

> http://www.hughes-ec.com/~brianb/teslahome.html

>

> I'll try and get some operating pictures up on the WEB next week.

>

> Thanks to everyone who posts on these lists.  I've learned a tremendous

> amount.

>

> Regards,

> Brian Basura

Brian,

Nice stuff!  Just romped thru yor diving pgs, too.  In your burbling

about, I need to rid myself of a PALLARD-BOLEX housing, (only)ca.

1955?  Setup for 16mm Reflex, 10mm Switar.  In case any one still has a

use for the thing.... Includes external light meter (GE DW68)

containment and several ASA calib. plates. Used to dive with Dr. & Zale

Perry back when we were founding the UNDERWATER PHOTOGRAPHIC SOC.

Questions in general:

1) Most folk seem to stack the little toroid on top of the big one...

Any advantage in reversing the order?

2) What happens if the pri is UNDER the base mount? Strike Rail could remain

on top.

Might need square plate. Give one a more stable place to set one's

coffee...  Seems like tapping would be enhanced... Reduce parts count:

pri coil hold-downs could be halved, if coil was flush to base?

------------------------------------------------------------

Gene Waldo -  gwaldo@1stnetusa.com  - "Igor in Residence"

---------------------------------------------------------------

What's good for the planet is more OZONE! See the machinery form up!

NEW PAGE!   http://members.1stnetusa.com/danmurphy/waldo.htm

From:          "Malcolm Watts" <MALCOLM@directorate.wnp.ac.nz>

Organization:  Wellington Polytechnic, NZ

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Date:          Tue, 29 Sep 1998 09:16:58 +1200

Subject:       Re: First Light

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

HI Gene, all,

> Date sent:      Mon, 28 Sep 1998 13:54:55 -0700

> From:           Gene Waldo <gwaldo@1stnetusa.com>

> Send reply to:  "Tesla-2 Mailing List" <tesla-2@emachine.com>

> To:             "Tesla-2 Mailing List" <tesla-2@emachine.com>

> Subject:        Re: First Light

> Basura, Brian wrote:

> >

> > Tesla List Members,

> >

> > I fired up my 6.5" coil for the first time this weekend.

> >

>

> > Construction pictures are at:

> > http://www.hughes-ec.com/~brianb/teslahome.html

> >

> > I'll try and get some operating pictures up on the WEB next week.

> >

> > Thanks to everyone who posts on these lists.  I've learned a tremendous

> > amount.

> >

> > Regards,

> > Brian Basura

>

> Brian,

> Nice stuff!  Just romped thru yor diving pgs, too.  In your burbling

> about, I need to rid myself of a PALLARD-BOLEX housing, (only)ca.

> 1955?  Setup for 16mm Reflex, 10mm Switar.  In case any one still has a

> use for the thing.... Includes external light meter (GE DW68)

> containment and several ASA calib. plates. Used to dive with Dr. & Zale

> Perry back when we were founding the UNDERWATER PHOTOGRAPHIC SOC.

>

> Questions in general:

> 1) Most folk seem to stack the little toroid on top of the big one...

> Any advantage in reversing the order?

Yes! Placing the small one on top of the large one results in the

large one screening the small one from ground and hence, top

capacitance is lower for small on large. You get more for your money

by placing the large on the small.

> 2) What happens if the pri is UNDER the base mount? Strike rail could remain

> on top.

> Might need square plate. Give one a more stable place to set one's

> coffee...  Seems like tapping would be enhanced... Reduce parts count:

> pri coil hold-downs could be halved, if coil was flush to base?

Nice idea but you might want a centre hole in the base so the

secondary could be more tightly coupled if required.

Malcolm

From: "Basura, Brian" <brian.basura@unistudios.com>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: RE: First Light

Date: Mon, 28 Sep 1998 15:43:42 -0700

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Gene/all,

The toroid was made from stuff I had on hand.  The reason the small one is

on top is because I built the large one first then ran out of material for

the center of the smaller one.  I just threw the small one on top of the

larger one hoping it would increase it's ROC.

Malcom said in another note that the large one shields the small one from

ground reducing it's effectiveness.  That is true since removing the smaller

one makes very little difference in tuning.

Anyway, I'll be making a new toroid soon.

Regards,

Brian Basura

> ----------

> From: 	Gene Waldo[SMTP:gwaldo@1stnetusa.com]

> Reply To: 	Tesla-2 Mailing List

> Sent: 	September 28, 1998 1:54 PM

> To: 	Tesla-2 Mailing List

> Subject: 	Re: First Light

>

> Basura, Brian wrote:

> >

> > Tesla List Members,

> >

> > I fired up my 6.5" coil for the first time this weekend.

> >

>

> > Construction pictures are at:

> > http://www.hughes-ec.com/~brianb/teslahome.html

> >

> > I'll try and get some operating pictures up on the WEB next week.

> >

> > Thanks to everyone who posts on these lists.  I've learned a tremendous

> > amount.

> >

> > Regards,

> > Brian Basura

>

> Brian,

> Nice stuff!  Just romped thru yor diving pgs, too.  In your burbling

> about, I need to rid my self of a PALLARD-BOLEX housing, (only)ca.

> 1955?  Setup for 16mm Reflex, 10mm Switar.  In case any one still has a

> use for the thing.... Includes external light meter (GE DW68)

> containment and several ASA calib. plates. Used to dive with Dr. & Zale

> Perry back when we were founding the UNDERWATER PHOTOGRAPHIC SOC.

>

> Questions in general:

> 1) Most folk seem to stack the little toroid on top of the big one...

> Any advantage in reversing the order?

>

> 2) What happens if the pri is UNDER the base mount? Strike rail could remain

> on top.

> Might need square plate. Give one a more stable place to set one's

> coffee...  Seems like tapping would be enhanced... Reduce parts count:

> pri coil hold-downs could be halved, if coil was flush to base?

> ------------------------------------------------------------

> Gene Waldo -  gwaldo@1stnetusa.com  - "Igor in Residence"

Date: Mon, 28 Sep 1998 19:19:23 -0500

From: Tedd Payne <teddp@earthlink.net>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: For Sale: Neon Transformers, Variacs, Meters, HV Wire

Sender: tesl-err@emachine.com

Hi All,

If interested, please contact me DIRECTLY at teddp@earthlink.net

I have the following items for sale, see the link to my list below:

- Neon transformers (15kv 30ma, 15kv 25ma, 12kv 30ma, 9kv 30ma, others)

- Variacs (6A, 10A, 10A, 30A)

- Meter, 0-150 mA AC, can measure transformer current directly

- HV neon transformer wire, flexible *14* gauge & std 16 gauge

- Misc other HV things

Please see the list and tell me what you're interested in, and your ZIP

CODE to determine shipping cost.

Also, if you have a web page and would like to "trade links",

please let me know.

Thanks!

Tedd Payne

----------------------------------------------------------

My FOR-SALE list is at:  http://home.earthlink.net/~teddp

----------------------------------------------------------

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: 6.5" Coil

X-Mailer: IPAD 2.0

From: break7@juno.com (Tyla A. Break)

Date: Mon, 28 Sep 1998 23:31:02 EDT

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Hi, Brian, Read your stuff, sounds great! I actually fired my

6.5" coil first time this weekend. I didn't quite get as good a

performance. My cap is actually one single flat stack. .0128uf. I did get

to ~48" I noted a couple interesting things. First , mine does seem to

want to strike grounded targets. I put a grounded copper ball within the

strike range and it rarely gets hit .A lot of arcs will strike farthar in

a slightly different direction. This was when I was only getting 22"

arcs. I since raised the toroid and added a fan to the gap to get 48". I

also had arcs come straight at me CLOSE! Has anybody been hit? What

happens? Looks like it would hurt real bad!  I had Igor and Dan M. over

sat. and one of them noted my tiny 18ga ground. So I changed it to #8

directly to a ground rod. Started it up and immediately burned the cap!

Another interesting thing is twice I tried increasing power to 5kw and

performance went way down. I'll probably have the cap rebuilt tomorrow

and be back in the game.

If it's not dangerous it's not fun!  Kyle  break7@juno.com

Date: Mon, 28 Sep 1998 22:30:52 -0700

From: Gene Waldo <gwaldo@1stnetusa.com>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: 6.5" Coil

Sender: tesl-err@emachine.com

Tyla A. Break wrote:

>

>         Hi, Brian, Read your stuff, sounds great! I actually fired my

> 6.5" coil first time this weekend. I didn't quite get as good a

> performance. My cap is actually one single flat stack. .0128uf. I did get

> to ~48" I noted a couple interesting things. First , mine does seem to

> want to strike grounded targets. I put a grounded copper ball within the

> strike range and it rarely gets hit.A lot of arcs will strike farthar in

> a slightly different direction. This was when I was only getting 22"

> arcs. I since raised the toroid and added a fan to the gap to get 48". I

> also had arcs come straight at me CLOSE! Has anybody been hit? What

> happens? Looks like it would hurt real bad!  I had Igor and Dan M. over

> sat. and one of them noted my tiny 18ga ground. So I changed it to #8

> directly to a ground rod. Started it up and immediately burned the cap!

Kyle,

I'll bet if you really COMMON the 2ndary base and your 'gnded' target to

earth, the target will see hits.

I asked the same Q about personal strikes, and no one has responded--

maybe they're all too chicken to find out... Or still stuck on the wall?

Like you say, "If it not dangerous..."

If determined to ease into the phenomenon, I would put on good thick

soled tennies, get some really fine bare wire, wind a grip at one end of

a 3' wood dowel, run the wire to the other end and approach gingerly, at

lower power levels.  Hold it in your right hand. One might be able to

develop a decent 'seat-of-the pants' for the case. And impress the

dollies!

We used to stand barefooted on two aluminum plates wired to some sort of

source; the game was to see who still keep toes extended at the highest

meter readings. I think it was a 25w bulb in series with a rheostat, in

series with our feet.

Kyle, the dichotomy here was, that while it was fun, it was probably not

too dangerous...  how do you reconcile this?

------------------------------------------------

Gene Waldo -  gwaldo@1stnetusa.com  - "Igor in Residence"

---------------------------------------------------------------

What's good for the planet is more OZONE! See the machinery form up!

NEW PAGE!   http://members.1stnetusa.com/danmurphy/waldo.htm

Date: Tue, 29 Sep 1998 06:23:59 -0500

From: Bert Hickman <bert.hickman@aquila.com>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Organization: Stoneridge Engineering

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: 6.5" Coil

Sender: tesl-err@emachine.com

Tyla A. Break wrote:

>

>         Hi, Brian, Read your stuff, sounds great! I actually fired my

> 6.5" coil first time this weekend. I didn't quite get as good a

> performance. My cap is actually one single flat stack. .0128uf. I did get

> to ~48" I noted a couple interesting things. First , mine does seem to

> want to strike grounded targets. I put a grounded copper ball within the

> strike range and it rarely gets hit .A lot of arcs will strike farthar in

> a slightly different direction. This was when I was only getting 22"

> arcs. I since raised the toroid and added a fan to the gap to get 48". I

> also had arcs come straight at me CLOSE! Has anybody been hit? What

> happens? Looks like it would hurt real bad!  I had Igor and Dan M. over

> sat. and one of them noted my tiny 18ga ground. So I changed it to #8

> directly to a ground rod. Started it up and immediately burned the cap!

> Another interesting thing is twice I tried increasing power to 5kw and

> performance went way down. I'll probably have the cap rebuilt tomorrow

> and be back in the game.

>

>         If it's not dangerous it's not fun!  Kyle  break7@juno.com

>

Kyle,

Congratulations on first light! Suspect that the cap blowing was

coincidental with the improvement in the ground. If you're rebuilding,

try going the route of 2 equally sized caps in series to share the

voltage stress. The fact that your performance seems to decline as you

increase power is most likely due to your gap(s) being overloaded and

quenching poorly. Are you using multiple gaps to share the power load?

Getting hit by a streamer off a big coil is not usually fatal unless you

also happen to have a primary:secondary flashover. However, it also is

definitely NOT an enjoyable event! The topload capacitance and secondary

self-C acts much like a charged high voltage capacitor, and the 200 -

300 kV jolt you receive can pack quite a wallop!

-- Bert --

X-Mailer: IPAD 2.0

Date: Tue, 29 Sep 1998 11:55:16 -0500

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: My first stacked plate cap

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

At 11:11 AM 9/29/97 -0700, you wrote:

>Ok, I've decided that I'm going to build a flat plate cap for my coil.

>I'll be using a 15KV/60ma Neon.

Good choice. You cant beat the performance of a well built plate cap.

They are tedious to build but well worth the effort.

>

>Please tell me if something isn't right (still learning here):

>

>I'm planning to run 8 caps in series, each made with 24 plates of foil

>seperated by 8 mils of poly.

>Each ends up around .089 uF, which gives me the ~.01uf I'm shooting for.

>The 8 in series gives me 22400V max which might be ok for my 15KV neon

>(?? 64 mils of poly total). Does this sound ok? This is only about 1800V

>per cap.

This sounds fine. My .008 mfd plate cap consists of six series

caps each .048 mfd. Each uses 8 mils of poly consisting of

two 4 mil sheets. Ive beat this cap pretty hard at 12kv and

it hasnt let me down. With 8 series caps yours should be just

fine at 15kv.

>

>Also, I'm thinking of stacking the caps on top of each other, with a

>sheet of plexiglass (or whatever I can find) between them. I'll drill

>holes through the plexiglass for connecting opposite terminals. How will

>this effect the capacitance? Do individual caps need to be separated by

>more than 1/4" (with insulation between them)?

I separated mine with a piece of ceramic tile.

How do you intend to connect the plates together? Foil is a bit

difficult to manage. I connected mine by bunching together the

plates and connecting them with small brass bolts.

>

>Once stacked, the whole thing will be layed on it's side in oil. I'll

>probably build each cap in oil for quicker break in.

I once built a .01 mfd plate in oil. It was a very messy experience

but the cap did work right away with no break in. My .008 mfd

plate cap was oriented vertically and the air bubbled out slowly.

I would recommend vertical orientation as you can never quite get

all the air out. If placed horizontally that air will be trapped.

TTYL!

Mike Hammer

mhammer@misslink.net

Date: Tue, 29 Sep 1998 11:35:50 -0600

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Daniel Lockhart <molly@cmn.net>

Subject: Re: 6.5" Coil

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

The Igor wrote:

>I asked the same Q about personnel strikes, and no one has responded--

>maybe they're all too chicken to find out... Or still stuck on the

>wall?

>Like you say, "If it not dangerous..."

>

>If determined to ease into the phenomenon, I would put on good thick

>soled tennies, get some really fine bare wire, wind a grip at one end of

>a 3' wood dowel, run the wire to the other end and approach gingerly, at

>lower power levels.  Hold it in your right hand. One might be able to

>develop a decent 'seat-of-the pants' for the case. And impress the

>dollies!

>We used to stand barefooted on two aluminum plates wired to some sort of

>source; the game was to see who still keep toes extended at the highest

>meter readings. I think it was a 25w bulb in series with a rheostat, in

>series with our feet.

Stupidly, naively, or bravely -- call it what you want -- I intentionally

have come into contact with the discharge from my coil. Standing on a piece

of carpet with tennis shoes on, I held about a four foot long aluminum ski

pole (with my hand on the aluminum, not handle) and brought it close enough

to the toroid.

With a perfectly tuned 3-inch coil running at 360 watts, you hardly feel

it. Standing on your toes, it becomes slightly uncomfortable however, as

sparks emit from under your toenails and discharge to the ground (maybe

that's why my Nikes lost their air...). It's not really that it hurts, just

feels weird. It's fun to touch somebody on the back of the neck while they

are drawing a spark from the coil. They get a surprising shock, although

not much bigger than one gets when you walk around the house in socks on a

wintry day.

Holding the ski pole slightly farther away from the toroid than the maximum

discharge length into air produces interesting results. Activity increases

around the part of the toroid nearest the pole, and eventually it strikes

it healthily after building up or five to ten seconds.

The geometry of the discharge terminal greatly affects how you feel it.

Drawing an arc from a nail placed on the toroid is barely noticeable -- one

from the side of the toroid slightly more so. However, when you draw a

spark from the top, flat part of the toroid, the spark is about half in

length, much thicker, and hurts real bad.

The sparks from my coil running at 720 watts were quite painful. Anything

over 360 watts and a three inch diameter secondary I would not recommend

touching.

I play the flute, and I've often wanted to replace the ski pole for my old

flute. It would make a cool picture! "Yeah. That sonata really came alive

that performance..." Never mind it was in the garage...

Keep in mind that the sparks were not touching my skin directly. They were

hitting the ski pole which I was holding. Direct contact between a spark

and your skin would probably leave your skin with a perforated feeling...

Daniel Lockhart

X-Mailer: IPAD 2.0

Date: Tue, 29 Sep 1998 14:18:31 -0500

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: 6.5" Coil/streamers to the body

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

At 11:35 AM 9/29/98 -0600, you wrote:

>The Igor wrote:

>

>>I asked the same Q about personnel strikes, and no one has responded--

>>maybe they're all too chicken to find out... Or still stuck on the

>>wall?

>>Like you say, "If it not dangerous..."

>>

>>If determined to ease into the phenomenon, I would put on good thick

>>soled tennies, get some really fine bare wire, wind a grip at one end of

>>a 3' wood dowel, run the wire to the other end and approach gingerly, at

>>lower power levels.  Hold it in your right hand. One might be able to

>>develop a decent 'seat-of-the pants' for the case. And impress the dollies!

>>We used to stand barefooted on two aluminum plates wired to some sort of

>>source; the game was to see who still keep toes extended at the highest

>>meter readings. I think it was a 25w bulb in series with a rheostat, in

>>series with our feet.

>

Then Daniel Lockhart said:

>Stupidly, naively, or bravely -- call it what you want -- I intentionally

>have come into contact with the discharge from my coil. Standing on a piece

>of carpet with tennis shoes on, I held about a four foot long aluminum ski

>pole (with my hand on the aluminum, not handle) and brought it close enough

>to the toroid.

>

>With a perfectly tuned 3-inch coil running at 360 watts, you hardly feel

>it. Standing on your toes, it becomes slightly uncomfortable however, as

>sparks emit from under your toenails and discharge to the ground (maybe

>that's why my Nikes lost their air...). It's not really that it hurts, just

>feels weird. It's fun to touch somebody on the back of the neck while they

>are drawing a spark from the coil. They get a surprising shock, although

>not much bigger than one gets when you walk around the house in socks on a

>wintry day.

>

<SNIP>

I unintentionally took a streamer hit from my coil running in

excess of 1 KVA. This was a 44 + inch streamer and it hit me

on the back of the right hand. It hurt like hell and gave me a

very nasty RF burn. I have a permanent scar to commemorate the event.

I now maintain a respectable distance from the display.

TTYL!

Mike Hammer

mhammer@misslink.net

X-Mailer: IPAD 2.0

Date: Tue, 29 Sep 1998 14:24:49 -0500

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: My first stacked plate cap

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

At 01:22 PM 9/29/97 -0700, you wrote:

>   How do you intend to connect the plates together? Foils is a bit

>difficult to manage. I connected mine by bunching together the

>plates and connecting them with small brass bolts.

>

> I'm not sure. I was thinking that I could lay all of the bus ends

>together, punch hole through them, and connect them with a bolt.

>

>The idea is to alternate the caps so that I can stack them up in series,

>and get everything in a 10 x 20 container (maybe smaller). -tom

Tom,

The only problem I see with this would be getting a good connection

between the foil and your connecting bolts. You would almost

have to put washers and nuts on each side of each group of plates.

I did a similar arrangement with a .01 mfd flat plate cap

made from aluminum flashing. I used washers on each side

of each plate. While it did work OK it was a pain to do.

On my next plate cap Im going back to extended tabs on the foil

plates like I did on my first plate cap.

TTYL!

Mike Hammer

mhammer@misslink.net

Date: Tue, 29 Sep 1998 15:06:03 -0500

From: Tom Polk <tpolk@prismnet.com>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: My first stacked plate cap

Sender: tesl-err@emachine.com

tomstock wrote:

>

>      How do you intend to connect the plates together? Foil is a

>      bit difficult to manage. I connected mine by bunching together

>      the plates and connecting them with small brass bolts.

My foil cap (12 x 16 x 4") uses heavy duty Reynolds Wrap and two pints

of castor oil. There is very little space for oil. The container is 1/4"

Lexan, and the poly plates have about 1/8" gap on each side; that leaves

room for oil to move and to bring 2" wide foil pigtails up from the

bottom to the top.

With a 4" thickness, you can see how the pigtails on the bottom plates

have to be 4" longer than the ones on the top plates. Once the pigtails

are brought straight up in a group, they form a thin stack and, they are

laid over the top about 2" in from the edge. I used a standard notebook

paper hole punch near the end of the pigtail stack, then ran brass 1/4 x

20 flat head bolts with 3/4" dia washers through the stack so that it

points up when the stack is laid flat. On the other side of the stack

(the top) goes a neoprene washer.

This allows one to put a flat lid on the whole container with the bolts

protruding, and screwing them down tight once the lid is on. To screw

them down where they don't leak, you put another neoprene washer on the

top outside, then a brass or steel washer, then the nut. Be sure you

tighen it before you screw the lid on. There is enough play to do this

if you make the pigtails long enough.

This process requires a gentle touch, not only to keep from tearing the

pigtails, but also to keep them dry. I poured oil in the box and spread

it out for every plate, and yes, it is a chore to keep the pigtails dry.

You can always go back and degrease the pigtails, but that is more of a

hassle than keeping them dry. I have not tried assembling a flat plate

cap dry then adding oil. If I do build one that way, the plates will

probably be assembled on edge rather than flat so oil will seep into

cracks better.

From:          "Malcolm Watts" <MALCOLM@directorate.wnp.ac.nz>

Organization:  Wellington Polytechnic, NZ

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Date:          Wed, 30 Sep 1998 09:15:06 +1200

Subject:       Re: 6.5" Coil

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Too chicken? Well try this for size:

> Date sent:      Mon, 28 Sep 1998 22:30:52 -0700

> From:           Gene Waldo <gwaldo@1stnetusa.com>

> Send reply to:  "Tesla-2 Mailing List" <tesla-2@emachine.com>

> Organization:   S.A.P.I.E.N.T. (Surly, Agressive, Politically Incorrect Enemies of Non Thinkers)

> To:             "Tesla-2 Mailing List" <tesla-2@emachine.com>

> Subject:        Re: 6.5" Coil

> Tyla A. Break wrote:

> >

> >         Hi, Brian, Read your stuff, sounds great! I actually fired my

> > 6.5" coil first time this weekend. I didn't quite get as good a

> > performance. My cap is actually one single flat stack. .0128uf. I did get

> > to ~48" I noted a couple interesting things. First , mine does seem to

> > want to strike grounded targets. I put a grounded copper ball within the

> > strike range and it rarely gets hit.A lot of arcs will strike farthar in

> > a slightly different direction. This was when I was only getting 22"

> > arcs. I since raised the toroid and added a fan to the gap to get 48". I

> > also had arcs come straight at me CLOSE! Has anybody been hit? What

> > happens? Looks like it would hurt real bad!  I had Igor and Dan M. over

> > sat. and one of them noted my tiny 18ga ground. So I changed it to #8

> > directly to a ground rod. Started it up and immediately burned the cap!

>

> Kyle,

> I'll bet if you really COMMON the 2ndary base and your 'gnded' target to

> earth, the target will see hits.

>

> I asked the same Q about personnel strikes, and no one has responded--

> maybe they're all too chicken to find out... Or still stuck on the

> wall?

Very simple really. Prepare yourself for a shock (npi) - the coil

is not developing a high enough voltage to go for the grounded target

directly!! Long sparks in TCs are as much a function of ion lifetimes

(associated with repetitively hammering the air) as they are of

output voltage. There is no other explanation. If the voltage is high

enough, the spark will go directly for the nearest object.

Like you say, "If it not dangerous..."

> If determined to ease into the phenomenon, I would put on good thick

> soled tennies, get some really fine bare wire, wind a grip at one end of

> a 3' wood dowel, run the wire to the other end and approach gingerly, at

> lower power levels.  Hold it in your right hand. One might be able to

> develop a decent 'seat-of-the pants' for the case. And impress the

> dollies!

> We used to stand barefooted on two aluminum plates wired to some sort of

> source; the game was to see who still keep toes extended at the highest

> meter readings. I think it was a 25w bulb in series with a rheostat, in

> series with our feet.

>

Worth noting that if you "gingerly" advance a rod towards the

streamers, you are in for a shock because at the outer edge of direct

streamer attachment to the rod, there is a degree of rectification

going on. Attaching to a thick solid spark is far less likely (but

not impossible) to shock you. I don't do it with any more than a

Joule in the primary.

Malcolm

X-Mailer: IPAD 2.0

Date: Tue, 29 Sep 1998 17:00:28 -0500

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: My first stacked plate cap

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

At 04:12 PM 9/29/97 -0700, you wrote:

>> Tom,

>> The only problem I see with this would be getting a good connection

>> between the foil and your connecting bolts. You would almost

>> have to put washers and nuts on each side of each group of plates.

>>

>

>Yes, this is what I am planning to do. I will have extended tabs from the

>foil, each with a washer and nut on each side.

>

>How did you connect the foil tabs on your 6 series caps?

>-tom

Ive included a JPG image of my first flat plate cap.

Its heavily compressed and not as clear as the original

photo but does show the structure of the cap.

Note the six caps oriented vertically. The extended tab

comes out the top of each cap and is bolted to the corresponding

tab on the next cap. Each cap is sandwiched between sheets of

plexiglass.

The white squares seen in the tank are ceramic tile. I used them

to displace oil and as separators between caps.

By the way the tank is made from plywood and has a plexiglass liner.

The plexiglass was attached with RTV silicone and beads of RTV were

run along all seams. Ths worked quite well and Ive had no leaks.

This cap is over 2 years old. I made the tank larger than necessary

as I had originally intended to expand it later.

In my latest tanks Ive dispensed with the plexiglass liner

and instead leakproof the tank the 2 coats of polyester resin

in the interior.

TTYL!

Attachment Converted: "C:\MISSING_LINK\EUDORA\Attach\Mhplate21.jpg"

Mike Hammer

mhammer@misslink.net

(editors note. Binary attachments not in archive)

Date: Wed, 30 Sep 1998 10:21:14 +1000

From: Ralph <rjdown@fan.net.au>

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: My first stacked plate cap

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

tomstock wrote:

A little off topic, but have you noticed your date? - have you found the

secret of time travel?

Ralph Down :)

Date: Tue, 29 Sep 1998 20:33:46 -0500

From: Bert Hickman <bert.hickman@aquila.com>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Organization: Stoneridge Engineering

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: My first stacked plate cap

Sender: tesl-err@emachine.com

tomstock wrote:

>

> Ok, I've decided that I'm going to build a flat plate cap for my coil.

> I'll be using a 15KV/60ma Neon.

>

> Please tell me if something isn't right (still learning here):

>

> I'm planning to run 8 caps in series, each made with 24 plates of foil

> seperated by 8 mils of poly.

> Each ends up around .089 uF, which gives me the ~.01uf I'm shooting for.

> The 8 in series gives me 22400V max which might be ok for my 15KV neon

> (?? 64 mils of poly total). Does this sound ok? This is only about 1800V

> per cap.

>

> Also, I'm thinking of stacking the caps on top of each other, with a

> sheet of plexiglass (or whatever I can find) between them. I'll drill

> holes through the plexiglass for connecting opposite terminals. How will

> this effect the capacitance? Do individual caps need to be seperated by

> more than 1/4" (with insulation between them)?

>

> Once stacked, the whole thing will be layed on it's side in oil. I'll

> probably build each cap in oil for quicker break in.

>

> Thanks

> -tom

> tomstock@geocities.com

Tom,

Sounds reasonable, as long as the 8 mil thickness in each module is made

up of at least two 4 mil sheets. This will prevent a single defect in a

single sheet from causing a module failure. Also, the caps should be

aligned with the plates in a vertical orientation so that entrapped air

can escape upwards. If you connect the modules so that adjacent foil

ends are at the same potential, you should be able to use 1/8" plastic

to physically hold them together without having to worry about standing

off any voltage between modules.

I use a capacitor of similar design, except that I have 4 modules, each

using 32 mils (8 sheets of 4 mil poly) for a total of 128 mils. However,

this is overkill for a 15 KV NST, - my cap was originally designed to

run off a 20 kV pig.

Good Luck!

-- Bert --

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: "Coiler" <mycroft@access1.net>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Quick Question re: Pig Wiring

Date: Tue, 29 Sep 1998 22:33:10 -0700

X-Mailer: IPAD 2.0

Importance: Normal

X-MimeOLE: Produced By Microsoft MimeOLE V4.72.3155.0

Sender: tesl-err@emachine.com

Well.. the ghods of zap have been kind enough to deliver unto me a

5KVA transformer.

Until such time as I can get a 220V Variac, I will have to run this

beastie at 110. Furtheremore, I intend to start out a lower power

until I am comfortable with the tunes. So. The question:

I will initially be using 1.3KVA of resistive ballast at 110V: Must

I rewire the secondaries to bring both 110V windings in parallel, or

can i get away with the (relatively) low power on one set of windings

and let the second float?

eg: Current config

X1      X2 X3       X4

|        | |         |

|         -          |

|         |          |

110V	   110V

Can I just use X1 and X2?  or must I rewire for (X1,X3) and (X2,X4)?

TIA

Michael Baumann

Coiler, Homebrewer, Nerd. mycroft@access1.net

X-Mailer: IPAD 2.0

Date: Wed, 30 Sep 1998 08:32:50 +0200

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Marco Denicolai <marco@vistacom.fi>

Subject: Running higher than 400 BPS?

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

I built an asyncronous RSG with top break rate about 367 BPS. The results I

got during a couple of 30' runs were encouraging. I bet 20% longer

streamers, with biforcations all along the streamers (instead of the usual

long streamer with a few biforcation near its end).

The fact is that varying the RSG speed from slow to fast I could only notice

increasing performance, reaching the maximum together with top speed.

My question is: anybody using an RSG with break rate higher than 400 BPS?

That is, is it worth for me adding a couple more of rotating electrodes to

reach 734 BPS or not?

My tank supply is a 6.5 kV MOT voltage doubler, 0.05uF primary cap: I am not

able to say how fast the cap gets charged by the supply.

My TC and RSG designs can be found at http://vci.vistacom.fi/~marco/mytesla.html

Marco Denicolai                   Vista Communication Instruments, Inc.

Hardware Development Manager      www.vistacom.fi

marco@vistacom.fi

Date: Wed, 30 Sep 1998 07:01:34 -0500

From: Bert Hickman <bert.hickman@aquila.com>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Organization: Stoneridge Engineering

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Running higher than 400 BPS?

Sender: tesl-err@emachine.com

Marco Denicolai wrote:

>

> I built an asyncronous RSG with top break rate about 367 BPS. The results I

> got during a couple of 30' runs were encouraging. I bet 20% longer

> streamers, with biforcations all along the streamers (instead of the usual

> long streamer with a few biforcation near its end).

>

> The fact is that varying the RSG speed from slow to fast I could only notice

> increasing performance, reaching the maximum together with top speed.

>

> My question is: anybody using an RSG with break rate higher than 400 BPS?

> That is, is it worth for me adding a couple more of rotating electrodes to

> reach 734 BPS or not?

>

> My tank supply is a 6.5 kV MOT voltage doubler, 0.05uF primary cap: I am not

> able to say how fast the cap gets charged by the supply.

>

> My TC and RSG designs can be found at http://vci.vistacom.fi/~marco/mytesla.html

>

>

>  Marco Denicolai                   Vista Communication Instruments, Inc.

>  Hardware Development Manager      www.vistacom.fi

>

>  marco@vistacom.fi

Marco,

Higher breakrates generally mean longer streamers up to about 700 BPS.

Certain breakrates also seem to change the character of the spark. For

example, lower rates often create a lazy, wandering, single streamer,

while higher rates can result in "frantic" behavior with rapidly darting

of the streamers, and multiple forks on a given streamer. A number of

coilers on the main Builders List have systems that hit up to 700 BPS,

but most of them use higher inductance primaries, a lower value of tank

capacitance, and significantly higher tank circuit supply voltages in

order to realize the higher rates. The various streamer behaviors, as

well as some of the "sweet spots" at various rates, make hitting higher

breakrates quite desirable. Unfortunately, higher breakrates also place

much more stringent demands and stress on the HV supply, tank cap, and RSG.

Higher breakrates require a "stiffer" power source (i.e., one that can

deliver higher recharge current without voltage collapse) as well as a

well made, low-loss tank cap, since doubling the breakrate doubles the

amount of power that the supply and tank cap must process. A

polypropylene or polyethylene tank cap is mandatory, and many coilers

are forced to go to commercial pulse caps at this stage, run-times

reduced, and cool-down times increased between runs. Higher breakrates

cause significant internal heating deep inside the capacitor rolls. If

the dielectric and conductor heat builds up faster than it can be

conducted away, internal melting and destruction of the cap will result.

Mylar or ceramic caps will rapidly whimper (or sometimes explode!) and

die in this environment.

You'll likely need to beef up your power supply so that it can deliver

the higher recharge current necessary to recharge the tank caps in half

the time. You'll most likely need to increase your supply's output

voltage as well to achieve anywhere near 700 BPS off an AC supply. The

DC "storage" capacitance in the doubler circuit may also need to be

increased in size to reduce the degree of voltage collapse at twice the

breakrate. BTW, you may only need to add a couple more STATIC electrodes

to double your effective breakrate - this may simplify the mechanical

changes to your RSG.

Safe coilin' to you!

-- Bert --

Date: Wed, 30 Sep 1998 07:01:38 -0500

From: Bert Hickman <bert.hickman@aquila.com>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Organization: Stoneridge Engineering

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Quick Question re: Pig Wiring

Sender: tesl-err@emachine.com

Coiler wrote:

>

> Well.. the ghods of zap have been kind enough to deliver unto me a

> 5KVA transformer.

>

> Until such time as I can get a 220V Variac, I will have to run this

> beastie at 110. Furtheremore, I intend to start out a lower power

> until I am comfortable with the tunes. So. The question:

> I will initially be using 1.3KVA of resistive ballast at 110V: Must

> I rewire the secondaries to bring both 110V windings in parallel, or

> can i get away with the (relatively) low power on one set of windings

> and let the second float?

> eg: Current config

>

> X1      X2 X3       X4

> |        | |         |

> |         -          |

> |         |          |

>     110V           110V

>

> Can I just use X1 and X2?  or must I rewire for (X1,X3) and (X2,X4)?

>

> TIA

>

> Michael Baumann

> Coiler, Homebrewer, Nerd. mycroft@access1.net

>

Michael,

Congratulations on getting a mini-pig or potential transformer. You

don't need to re-wire the LV side - just drive it from either X1-X2 or

X2-X3 and you should be OK. BTW, while resistive ballasting will work,

you'll find that an inductive ballast will provide you with much better

performance. A couple of NST's or Microwave Oven transformers can be

used (with secondaries shorted) to provide the current limiting

function.

-- Bert --

X-Mailer: IPAD 2.0

Date: Wed, 30 Sep 1998 15:37:34 +0200

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Marco Denicolai <marco@vistacom.fi>

Subject: Re: Running higher than 400 BPS?

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

At 07:01 30/09/98 -0500, you wrote:

>Marco,

>

><SNIP> A number of

>coilers on the main Builders List have systems that hit up to 700 BPS,

>but most of them use higher inductance primaries, a lower value of tank

>capacitance, and significantly higher tank circuit supply voltages in

>order to realize the higher rates.

Why higher inductance and higher voltage? Isn't higher tank current and

maybe a smaller primary cap enough?

>The various streamer behaviors, as

>well as some of the "sweet spots" at various rates, make hitting higher

>breakrates quite desirable. Unfortunately, higher breakrates also place

>much more stringent demands and stress on the HV supply, tank cap, and

>RSG.

<SNIP>

Thank you very much for your thorough description of all the problems. This

really scares me! My commercial Maxwell cap are "unique" in a way that if

I break them I can't get other pieces used (or affort to buy new). If I

break them I am in big s..t!

I'd rather build some by myself, less than the current 0.05uF, and stress

those. But that takes time, and the temptation to just drill two more holes

in the RSG rotor, add 2 more electrodes and try is big...

I guess I've really better to keep cool, spend my winter rolling caps and

avoid to possibly break my Maxwell caps just to get 10" more lenght.

Marco Denicolai                   Vista Communication Instruments, Inc.

Hardware Development Manager      www.vistacom.fi

marco@vistacom.fi

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: "Coiler" <mycroft@access1.net>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: RE: Quick Question re: Pig Wiring

Date: Wed, 30 Sep 1998 06:39:41 -0700

X-Mailer: IPAD 2.0

Importance: Normal

X-MimeOLE: Produced By Microsoft MimeOLE V4.72.3155.0

Sender: tesl-err@emachine.com

Bert:Thanks for confirming what I suspected.

BTW: It isn't a mini-pig, but the full blown beastie.. just lower

current handling capacity. My understanding is the mini-pigs are

just that, smaller. This one has 8 gallons of oil, and a shipping

wt of over 200lbs - not exactly what I call small :)

I figure I can push it as far as about 7.5KVA for short runs,

which is more than I can pull from the wall. I am planning to install

a 30A 220V service (breaker on each leg) to run the coil from.

Michael Baumann

Coiler, Homebrewer, Nerd. mycroft@access1.net

> >

>

> Michael,

>

> Congratulations on getting a mini-pig or potential transformer. You

> don't need to re-wire the LV side - just drive it from either X1-X2 or

> X2-X3 and you should be OK. BTW, while resistive ballasting will work,

> you'll find that an inductive ballast will provide you with much better

> performance. A couple of NST's or Microwave Oven transformers can be

> used (with secondaries shorted) to provide the current limiting

> function.

>

> -- Bert --

>

>

Date: Wed, 30 Sep 1998 09:29:54 -0500

From: Eric Davidson <edavidson@icva.gov>

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Pig Wiring

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Michael,

You can run on X1 and X2 as long as you keep a few things in mind.  If

the secondary midpoint is connected to the tank, be sure to connect the

neutral to the midpoint and the hot to X1.  X4 will be energized.  The

optimal connection is the parallel connection, X1,X3 + X2,X4, for 120

volt operation. This is easily done inside the tank.  This is a common

connection when the transformer is used as part of a three phase

120/208Y volt bank. One end of the LV winding(s) should be grounded and

bonded to the tank for safety. Even with 240 volt operation, the

secondary midpoint should be bonded to the tank and to ground. Hope this

helps. And though its been said, many times, many ways...be careful.

Eric

Coiler wrote:

>

> Well.. the ghods of zap have been kind enough to deliver unto me a

> 5KVA transformer.

>

> Until such time as I can get a 220V Variac, I will have to run this

> beastie at 110. Furtheremore, I intend to start out a lower power

> until I am comfortable with the tunes. So. The question:

> I will initially be using 1.3KVA of resistive ballast at 110V: Must

> I rewire the secondaries to bring both 110V windings in parallel, or

> can i get away with the (relatively) low power on one set of windings

> and let the second float?

> eg: Current config

>

> X1      X2 X3       X4

> |        | |         |

> |         -          |

> |         |          |

>     110V           110V

>

> Can I just use X1 and X2?  or must I rewire for (X1,X3) and (X2,X4)?

>

> TIA

>

> Michael Baumann

> Coiler, Homebrewer, Nerd. mycroft@access1.net

>

--

Have a good day!

Eric Davidson

edavidson@icva.gov

Date: Wed, 30 Sep 1998 08:43:47 -0700

From: Gene Waldo <gwaldo@1stnetusa.com>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: 6.5" Coil

Sender: tesl-err@emachine.com

Daniel Lockhart wrote:

>

> The Igor wrote:

>

> >I asked the same Q about personnel strikes, and no one has responded--

> >maybe they're all too chicken to find out... Or still stuck on the

> >wall?

> >Like you say, "If it not dangerous..."

> >

> Stupidly, naively, or bravely -- call it what you want -- I intentionally

> have come into contact with the discharge from my coil. Standing on a piece

> of carpet with tennis shoes on, I held about a four foot long aluminum ski

> pole (with my hand on the aluminum, not handle) and brought it close enough

> to the toroid.

>

> With a perfectly tuned 3-inch coil running at 360 watts, you hardly feel

> it. Standing on your toes, it becomes slightly uncomfortable however, as

> sparks emit from under your toenails and discharge to the ground (maybe

> that's why my Nikes lost their air...). It's not really that it hurts, just

> feels weird. It's fun to touch somebody on the back of the neck while they

> are drawing a spark from the coil. They get a surprising shock, although

> not much bigger than one gets when you walk around the house in socks on a

> wintry day.

>

> Holding the ski pole slightly farther away from the toroid than the maximum

> discharge length into air produces interesting results. Activity increases

> around the part of the toroid nearest the pole, and eventually it strikes

> it healthily after building up or five to ten seconds.

>

> The geometry of the discharge terminal greatly affects how you feel it.

> Drawing an arc from a nail placed on the toroid is barely noticeable -- one

> from the side of the toroid slightly more so. However, when you draw a

> spark from the top, flat part of the toroid, the spark is about half in

> length, much thicker, and hurts real bad.

>

> The sparks from my coil running at 720 watts were quite painful. Anything

> over 360 watts and a three inch diameter secondary I would not recommend

> touching.

>

> I play the flute, and I've often wanted to replace the ski pole for my old

> flute. It would make a cool picture! "Yeah. That sonata really came alive

> that performance..." Never mind it was in the garage...

>

> Keep in mind that the sparks were not touching my skin directly. They were

> hitting the ski pole which I was holding. Direct contact between a spark

> and your skin would probably leave your skin with a perforated feeling...

>

> Daniel Lockhart

Daniel,

Excellent data! What were the free air lengths at the settings you

played in?

------------------------------------------------

Gene Waldo -  gwaldo@1stnetusa.com  - "Igor in Residence"

---------------------------------------------------------------

What's good for the planet is more OZONE! See the machinery form up!

NEW PAGE!   http://members.1stnetusa.com/danmurphy/waldo.htm

Date: Wed, 30 Sep 1998 09:17:17 -0700

From: Gene Waldo <gwaldo@1stnetusa.com>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Running higher than 400 BPS?

Sender: tesl-err@emachine.com

Marco Denicolai wrote:

>

> I built an asyncronous RSG with top break rate about 367 BPS. The results I

> got during a couple of 30' runs were encouraging.

> My TC and RSG designs can be found at http://vci.vistacom.fi/~marco/mytesla.html

________________________________________________________________________

>  Marco Denicolai                   Vista Communication Instruments, Inc.

>  Hardware Development Manager      www.vistacom.fi

>

Marco,

Your listed site for designs is rejected by my searcher....????

-----------------------------------------------------------

Gene Waldo -  gwaldo@1stnetusa.com  - "Igor in Residence"

---------------------------------------------------------------

What's good for the planet is more OZONE! See the machinery form up!

NEW PAGE!   http://members.1stnetusa.com/danmurphy/waldo.htm

Date: Wed, 30 Sep 1998 09:43:37 -0700

From: tomstock <tomstock@geocities.com>

X-Mailer: IPAD 2.0

X-Accept-Language: en

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: My first stacked plate cap

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

> tomstock wrote:

>

> A little off topic, but have you noticed your date? - have you found the

> secret of time travel?

>

> Ralph Down :)

>

Uh Whoops. I set the date back to get some expired demo software to run :)

Forgot to set it back.

-tom

Date: Wed, 30 Sep 1998 12:39:37 -0600

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Daniel Lockhart <molly@cmn.net>

Subject: Re: 6.5" Coil

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

I wrote:

>> Holding the ski pole slightly farther away from the toroid than the maximum

>> discharge length into air produces interesting results. Activity increases

>> around the part of the toroid nearest the pole, and eventually it strikes

>> it healthily after building up or five to ten seconds.

Gene asked:

>Excellent data! What were the free air lengths at the settings you

>played in?

On average at 360 watts, depending on how many sparks there are etc, my

mean spark length into air is about 20 inches. The max is 24, and that is

occasional. The discharges to a grounded object (me) were never as long as

in air, so usually I held the pole about 18 inches away. When drawing an

arc to me, the sparks were much brighter, fatter, and lazy.

At higher powers with 60 and 90 ma, the average spark length into air is 24

inches. They have many multiple forks. Actually, if you could put all the

forks into one long forkless arc, it would probably be 75% longer. I did

not however, touch these discharges. Not even if I had a ten foot ski

pole...

Daniel Lockhart

Date: Wed, 30 Sep 1998 14:23:32 -0500

From: Tom Polk <tpolk@prismnet.com>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Running higher than 400 BPS?

Sender: tesl-err@emachine.com

Marco Denicolai wrote:

> My question is: anybody using an RSG with break rate higher than 400 BPS?

> That is, is it worth for me adding a couple more of rotating electrodes to

> reach 734 BPS or not?

Mine runs at 490bps top speed. I get no appreciable performance

enhancement by slowin it down. My guess is that mine functions as a

fixed gap, but with excellent quenching.

From: "Basura, Brian" <brian.basura@unistudios.com>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: RE: First Light

Date: Wed, 30 Sep 1998 12:23:55 -0700

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

All,

I resolved my problem of performance dropping off after a few seconds by

reducing the number of gaps in my main SG.  The coil now "sings" and will

run consistently for as long as I dare.

Added another couple of caps for a total of .043mfd and started adjusting my

current limiter  (currently up to 1400w).  I'm now getting 5-6 concurrent

forked streamers that easily reach 36" targets.

I'll build a new and bigger toroid over the next couple of days and let you

know how it works.

Thanks,

Brian D. Basura

From: "Thomas McGahee" <tom_mcgahee@sigmais.com>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Quick Question re: Pig Wiring

Date: Wed, 30 Sep 1998 17:22:04 -0400

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Bert is right about being able to run on just one set of primaries,

however, if you are planning on pushing your transformer later, then

wire them in parallel (X1,x3   and   X2,x4). That will make the primary

more robust.

Hope this helps.

Fr. Tom McGahee

----------

> From: Bert Hickman <bert.hickman@aquila.com>

> To: Tesla-2 Mailing List <tesla-2@emachine.com>

> Subject: Re: Quick Question re: Pig Wiring

> Date: Wednesday, September 30, 1998 8:01 AM

>

> Coiler wrote:

> >

> > Well.. the ghods of zap have been kind enough to deliver unto me a

> > 5KVA transformer.

> >

> > Until such time as I can get a 220V Variac, I will have to run this

> > beastie at 110. Furtheremore, I intend to start out a lower power

> > until I am comfortable with the tunes. So. The question:

> > I will initially be using 1.3KVA of resistive ballast at 110V: Must

> > I rewire the secondaries to bring both 110V windings in parallel, or

> > can i get away with the (relatively) low power on one set of windings

> > and let the second float?

> > eg: Current config

> >

> > X1      X2 X3       X4

> > |        | |         |

> > |         -          |

> > |         |          |

> >     110V           110V

> >

> > Can I just use X1 and X2?  or must I rewire for (X1,X3) and (X2,X4)?

> >

> > TIA

> >

> > Michael Baumann

> > Coiler, Homebrewer, Nerd. mycroft@access1.net

> >

>

> Michael,

>

> Congratulations on getting a mini-pig or potential transformer. You

> don't need to re-wire the LV side - just drive it from either X1-X2 or

> X2-X3 and you should be OK. BTW, while resistive ballasting will work,

> you'll find that an inductive ballast will provide you with much better

> performance. A couple of NST's or Microwave Oven transformers can be

> used (with secondaries shorted) to provide the current limiting

> function.

>

> -- Bert --

From:          "Malcolm Watts" <MALCOLM@directorate.wnp.ac.nz>

Organization:  Wellington Polytechnic, NZ

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Date:          Thu, 1 Oct 1998 09:56:00 +1200

Subject:       Re: Running higher than 400 BPS?

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Hi Marco,

> Date sent:      Wed, 30 Sep 1998 15:37:34 +0200

> To:             "Tesla-2 Mailing List" <tesla-2@emachine.com>

> From:           Marco Denicolai <marco@vistacom.fi>

> Subject:        Re: Running higher than 400 BPS?

> Send reply to:  "Tesla-2 Mailing List" <tesla-2@emachine.com>

> At 07:01 30/09/98 -0500, you wrote:

> >Marco,

> >

> ><SNIP> A number of

> >coilers on the main Builders List have systems that hit up to 700 BPS,

> >but most of them use higher inductance primaries, a lower value of tank

> >capacitance, and significantly higher tank circuit supply voltages in

> >order to realize the higher rates.

>

> Why higher inductance and higher voltage? Isn't higher tank current and

> maybe a smaller primary cap enough?

Got to mantain tune.

?

Malcolm

> >The various streamer behaviors, as

> >well as some of the "sweet spots" at various rates, make hitting higher

> >breakrates quite desirable. Unfortunately, higher breakrates also place

> >much more stringent demands and stress on the HV supply, tank cap, and

> >RSG.

> <SNIP>

>

> Thank you very much for your thorough description of all the problems. This

> really scares me! My commercial Maxwell cap are "unique" in a way that if

> I break them I can't get other pieces used (or affort to buy new). If I

> break them I am in big s..t!

>

> I'd rather build some by myself, less than the current 0.05uF, and stress

> those. But that takes time, and the temptation to just drill two more holes

> in the RSG rotor, add 2 more electrodes and try is big...

>

> I guess I've really better to keep cool, spend my winter rolling caps and

> avoid to possibly break my Maxwell caps just to get 10" more lenght.

>

>

Date: Wed, 30 Sep 1998 22:05:24 -0500

From: Bert Hickman <bert.hickman@aquila.com>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Organization: Stoneridge Engineering

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Running higher than 400 BPS?

Sender: tesl-err@emachine.com

Marco Denicolai wrote:

>

> At 07:01 30/09/98 -0500, you wrote:

> >Marco,

> >

> ><SNIP> A number of

> >coilers on the main Builders List have systems that hit up to 700 BPS,

> >but most of them use higher inductance primaries, a lower value of tank

> >capacitance, and significantly higher tank circuit supply voltages in

> >order to realize the higher rates.

>

> Why higher inductance and higher voltage? Isn't higher tank current and

> maybe a smaller primary cap enough?

Malcolm's correct - higher primary inductance is necessary in order to

maintain tune. Since a 2-coil system is actually a dual-tuned

transformer, LpCp must equal LsCs for best performance. If no changes

are made to the secondary:toroid system, then Lp must increase if you

decrease Cp.

While driving from a higher voltage source is not essential, it can

often make it easier to achieve higher breakrates without sacrificing

"bang size". This is especially true when your ballast has a significant

amount of resistance versus inductive reactance, and recharging becomes

more exponential in character. The higher the rail voltage, the more

rapidly the tank cap will recharge, and the higher the breakrate can be

for a given gap breakdown voltage.

>

> >The various streamer behaviors, as

> >well as some of the "sweet spots" at various rates, make hitting higher

> >breakrates quite desirable. Unfortunately, higher breakrates also place

> >much more stringent demands and stress on the HV supply, tank cap, and

> >RSG.

> <SNIP>

>

> Thank you very much for your thorough description of all the problems. This

> really scares me! My commercial Maxwell cap are "unique" in a way that if

> I break them I can't get other pieces used (or affort to buy new). If I

> break them I am in big s..t!

>

<SNIP>

Maxwell does build some very nice capacitors, some of which can

withstand relatively high pulse repetition rates (1000 PPS). However,

these caps also need to be substantially derated (voltage-wise) for

Tesla Coil service since we can easily see 80% voltage reversals when

the gap fires. What are the voltage ratings and Maxwell part numbers for

the caps you've got? They MAY be able to take the punishment...

-- Bert --

From: McFalc0n@aol.com

Date: Wed, 30 Sep 1998 23:18:47 EDT

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: begginer...

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Is there any vital difference between using a 30 degree or 15 deg primary???

On a low power 450 watt coil???

Plus, sorry if it is a stupid question,  but what's the best way to tap

quarter inch copper tubing with quarter inch spacing?  Alligator, battery

clips, ...?

Also, will a grounded strike rail completely eliminate secondary to primary

arcing???  HAving a 30 degree primary on my coil might allow the possibility

if it is wound at 30 degrees.

Thanx for ur time.

X-Originating-IP: [208.169.33.91]

From: "chris morgan" <crmorgan@hotmail.com>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: begginer...

Date: Wed, 30 Sep 1998 21:09:29 PDT

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

>Is there any vital difference between using a 30 degree or 15 deg primary???

>

This would effect the strength of the magnetic coupling between the sec.

& pri coils, how much, I don't know.

>	Also, will a grounded strike rail completely eliminate secondary to

primary arcing???

A strike rail is designed to prevent arcing from the toroid to the

primary.  You'd want to use heavy insulation (several coats of

polyurathane on the sec. coil) to prevent arcing from the sec. coil to

the primary

Date: Wed, 30 Sep 1998 22:22:56 -0600

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Daniel Lockhart <molly@cmn.net>

Subject: Re: begginer...

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

You wrote:

>	is there any vital difference between using a 30 degree or 15 deg primary???

>		on a low power 450 watt coil???

As the angle of the primary increases, so does its inductance for the same

amount of turns. So, a 30 degree primary will have more inductance than a

15 degree primary that has the same number of turns.

The geometry doesn't matter as long as you get the right inductance. Making

the primary at a lower angle will place it farther away from the secondary,

however.

>	Plus, sorry if it is a stupid question,  but what's the best way to tap

>quarter inch copper tubing with quarter inch spacing?  Alligator, battery

>clips, ...?

I use a 10 amp battery clip. I don't know if this is the best, but that's

what I use. Positioning it so that it so that it connects on the bottom

side of the primary keeps it out of the way of the secondary.

>	Also, will a grounded strike rail completely eliminate secondary to primary

>arcing???  Having a 30 degree primary on my coil might allow the possibility

>if it is wound at 30 degrees.

>	Thanx for ur time.

Don't know. I don't have a strike rail on my coil, as my primary is at a

real low angle (11 or so degrees). I think it would be incorrect to say a

strike rail would *completely* eliminate strikes on the primary though.

My two pence,

Daniel Lockhart

X-Mailer: IPAD 2.0

Date: Thu, 01 Oct 1998 08:05:54 +0200

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Marco Denicolai <marco@vistacom.fi>

Subject: Re: Running higher than 400 BPS?

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

At 22:05 30/09/98 -0500, you wrote:

>Marco Denicolai wrote:

> ><SNIP> A number of

>> Why higher inductance and higher voltage? Isn't higher tank current and

>> maybe a smaller primary cap enough?

>

>Malcolm's correct - higher primary inductance is necessary in order to

>maintain tune. Since a 2-coil system is actually a dual-tuned

>transformer, LpCp must equal LsCs for best performance. If no changes

>are made to the secondary:toroid system, then Lp must increase if you

>decrease Cp.

Yes, of course. I only thought you meant some other reason for the incresed

inductance.

><SNIP>

>

>Maxwell does build some very nice capacitors, some of which can

>withstand relatively high pulse repetition rates (1000 PPS). However,

>these caps also need to be substantially derated (voltage-wise) for

>Tesla Coil service since we can easily see 80% voltage reversals when

>the gap fires. What are the voltage ratings and Maxwell part numbers for

>the caps you've got? They MAY be able to take the punishment...

I got 2 surplus Maxwell capacitors model 30561 0.1uF 25KV with following

specs:

Capacitance: 0.1uF 25KV

Rated peak current: 5 KA

Max peak current: 25 KA

Max ripple freq.: 10 KHz

Max RMS current: 25 A RMS

Max Dissipation factor: 0.3% @ 120 Hz

Inductance: 80 nH

They are enclosed in metal cans and have got ceramic terminals. I connected

them in series to get 0.05uF. Their case gets slightly warm after some 30'

discontinuous operation at 367 BPS.

With a RQ gap it takes more time (maybe 1 hour?) so that you can start

feeling with your fingers

that they have got warmer.

What it your verdict?

Marco Denicolai                   Vista Communication Instruments, Inc.

Hardware Development Manager      www.vistacom.fi

marco@vistacom.fi

Date: Thu, 01 Oct 1998 06:59:05 -0500

From: Bert Hickman <bert.hickman@aquila.com>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Organization: Stoneridge Engineering

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Running higher than 400 BPS?

Sender: tesl-err@emachine.com

Marco Denicolai wrote:

>

> At 22:05 30/09/98 -0500, you wrote:

> >Marco Denicolai wrote:

> > ><SNIP>

> >> Why higher inductance and higher voltage? Isn't higher tank current and

> >> maybe a smaller primary cap enough?

> >

> >Malcolm's correct - higher primary inductance is necessary in order to

> >maintain tune. Since a 2-coil system is actually a dual-tuned

> >transformer, LpCp must equal LsCs for best performance. If no changes

> >are made to the secondary:toroid system, then Lp must increase if you

> >decrease Cp.

>

> Yes, of course. I only thought you meant some other reason for the increased

> inductance.

One other reason is that the primary tank circuit peak current is

reduced. This will reduce I^2*R heating from the Equivalent Series

Resistance of the pulse cap, and will also make life a bit easier for

the gaps...

>

> ><SNIP>

> >

> >Maxwell does build some very nice capacitors, some of which can

> >withstand relatively high pulse repetition rates (1000 PPS). However,

> >these caps also need to be substantially derated (voltage-wise) for

> >Tesla Coil service since we can easily see 80% voltage reversals when

> >the gap fires. What are the voltage ratings and Maxwell part numbers for

> >the caps you've got? They MAY be able to take the punishment...

>

> I got 2 surplus Maxwell capacitors model 30561 0.1uF 25KV with following

> specs:

>

> Capacitance: 0.1uF 25KV

> Rated peak current: 5 KA

> Max peak current: 25 KA

> Max ripple freq.: 10 KHz

> Max RMS current: 25 A RMS

> Max Dissipation factor: 0.3% @ 120 Hz

> Inductance: 80 nH

>

> They are enclosed in metal cans and have got ceramic terminals. I connected

> them in series> to get 0.05uF. Their case gets slightly warm after some 30' discontinuous

> operation at 367 BPS.

> With a RQ gap it takes more time (maybe 1 hour?) so that you can start

> feeling with your fingers

> that they have got warmer.

>

> What it your verdict?

>

>  Marco Denicolai                   Vista Communication Instruments, Inc.

>  Hardware Development Manager      www.vistacom.fi

>

>  marco@vistacom.fi

Nice caps, Marco! The combined 50 kV rating will provide good margin for

voltage reversals, and these particular caps are indeed low-loss pulse

discharge type. You should be able to run them at higher break rates as

long as you reduce any continuous run-time to no more than 1-2 minutes,

and allow at least 2 minutes of cool-down time for internal heat to

redistribute. All other things being the same (which they rarely are

:^)), you'll see internal capacitor heating that's proportional to the

breakrate - double the BPS and you'll see at least twice the power

dissipation in the caps that you see now. Since they are surviving the

existing environment at 367 BPS, they'll most likey survive at 700 BPS

as long as you don't overheat them.

-- Bert --

X-Originating-IP: [199.217.140.22]

From: "Bill the arcstarter" <arcstarter@hotmail.com>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Running higher than 400 BPS?

Date: Thu, 01 Oct 1998 07:35:32 PDT

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

Bert wrote:

>Nice caps, Marco! The combined 50 kV rating will provide good margin for

>voltage reversals, and these particular caps are indeed low-loss pulse

>discharge type. You should be able to run them at higher break rates as

>long as you reduce any continuous run-time to no more than 1-2 minutes,

>and allow at least 2 minutes of cool-down time for internal heat to

>redistribute. All other things being the same (which they rarely are

>:^)), you'll see internal capacitor heating that's proportional to the

>breakrate - double the BPS and you'll see at least twice the power

>dissipation in the caps that you see now. Since they are surviving the

>existing environment at 367 BPS, they'll most likey survive at 700 BPS

>as long as you don't overheat them.

OK Bert - what is going on?  Why is the capacitor heating proportional

to the break rate?

Has anyone been able to show that the arc is interrupted prior to all

the primary energy being transferred into the secondary?

What I'm wondering is whether you are really able to get 700 bps, or

does the arc last so long that it doesn't stop before the next electrode

lines up??

-Bill

Date: Thu, 01 Oct 1998 09:17:19 -0600

From: Dan Olsen <dan@olsen.org>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Another beginner...

Sender: tesl-err@emachine.com

I have all of the pieces of my first coil constructed, except the primary coil.

I am finding that the mechanics of supporting it, keeping the spacing even,

bending the tubing, etc.,  is much more difficult (for me) than any of the

other parts, or calculations.

Does anyone have any good ideas for supporting a quarter inch tubing with

quarter inch (or so) spacing.  over 16-19 turns.  I am looking for an off the

shelf solution here,  I realize I could measure and cut 20 quarter inch slots

in 10 or 12 wooden spokes. But I figure that Is over 200 cuts that I have to

make 3 or 4 times each to get 1/4 inch, and that will take at least several

hours with the skillsaw.  If I do angle it at 10-15 degrees, what's a good way

to hold it all up.  (I studied physics in college, not structural

engineering.).

I have been rather proud so far of the usability of off-the-shelf,

hardware-store components.  The dryer-exhaust tubing, the PVC pipe, the copper

tube spark gap, the aluminum flashing, polyethylene capacitor, etc. It's like a

McGuyver thing, where you walk-in buy some common items, and create an amazing

contraption.

I'd be really impressed if someone could think of a way to adapt an easily

availiable item, to use as a support and spacer for a primary coil, or if they

just have any ideas that are better than mine above for doing this.  Is there

anything related to hanging clothes or ties or somthing that would have plastic

pegs every half inch or so?  How about it what have you guys done?  If a wooden

form is the best I guess I will go ahead and do that.

Thanks,

--Dan

From: ESchulz531@aol.com

Date: Thu, 1 Oct 1998 12:45:00 EDT

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Another beginner...

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Hi,

I drilled holes in my plastic primary supports and used

plastic ties to hold the copper tubing in place.  The supports

are pieces of a plastic cutting board.  Heres a picture.

http://members.aol.com/tesla00/primary.jpg

Its hard to tell in the picture but I drilled the holes on the

side of the plastic not the top.

Erik Schulz

Date: Thu, 01 Oct 1998 09:56:45 -0700

From: tomstock <tomstock@geocities.com>

X-Mailer: IPAD 2.0

X-Accept-Language: en

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Coil Contact, and Winding Primaries

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Ok everyone, two quick questions:

Lately I've been wanting to touch my streamers. Ok I guess this is kind

of like wanting to stick your fingers in the belts while the car is

running. Anyway, I read somewhere that if I held a metal object firmly

in my hand I could touch the streamers providing there is absoloutely no

secondary to primary arcing going on.

Well, my coil is running from 15KV 60ma and I get somewhere around 24"

streamers. Is this going to hurt?

What about my feet?? Will streamers break from my feet and strike

ground? Isn't this a bad thing?

Should I stand on a milk crate?

I'm not going to do anything until someone tells me it's (sort of) ok.

Second question:

How is everyone winding such nice primaries?? Mine looks good but

definately not as good as some I've seen on the web. Is there a special

tool I can get to make nice smooth turns?

thanks

-tom

Date: Thu, 01 Oct 1998 10:10:20 -0700

From: Gene Waldo <gwaldo@1stnetusa.com>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Another beginner...

Sender: tesl-err@emachine.com

Dan Olsen wrote:

>

> I have all of the pieces of my first coil constructed, except the primary coil.

>

> I am finding that the mechanics of supporting it, keeping the spacing even,

> bending the tubing, etc.,  is much more difficult (for me) than any of the

> other parts, or calculations.

>

> Does anyone have any good ideas for supporting a quarter inch tubing with

> quarter inch (or so) spacing.  over 16-19 turns.   If a wooden

> form is the best I guess I will go ahead and do that.

>

> Thanks,

>

> --Dan

Dan, 8 'spokes' is sufficient. do a 2fer one op: 4 pcs 1" x 2" x Lngth;

on cl drill 17/64" holes at the spacing you desire, and for  the number

of turns you need.  Drill clr holes for screws to attach coil 'spokes'

to base; sawcut lengthhwise thru cl of drilled holes. My suggestion is

install pri UNDER the base pl., which will need a cntral hole to clear

2ndary dia. and provide adjustant for coupling; but first scribe the ID

circle of the pri inner turn & construct your spoke radials; set end of

spoke 1 to the circle; spoke 2 should be out from the circle by:

(turn-to-turn spacing)/(the number of spokes), etc.  Thus if you have

.25" tube at .25" spacing, then each successive spoke should be set out

from the circle by .031", so that the spiral will be consistant. Having

the pri under the base pl makes tapping and strikes much less of a

problem. This obviously leaves the coil flush against the base pl, but I

can't see why that should be a problem; comments?

Completely install all spokes! set small finish nails at each tube

guide-half-hole to restrain and guide the tube as it is layed in.

Remove spokes; lay tubing in nail guides.  Reinstall spokes over tubing,

and screw down. Pull nails.

Instead of screws to hold spokes, one  might want to glue/dowel the mess

together, but only if metal screws could pose a problem in operation.

Comments?

I think allowing for this contingency at base/spoke fab might be a good

idea.

(This is for a pancake.  If you wish a saucer, the engineering becomes

more complex and coupling gets more involved.  And the under-the-pl

approach may not be that applicable, although it should still be

feasable)

You then face the dilemma of: fixed pri or 2ndary? One will have to be

vertically adjustable.

That's kinda how I plan on doing it...

--

Gene Waldo -  gwaldo@1stnetusa.com  - "Igor in Residence"

---------------------------------------------------------------

What's good for the planet is more OZONE! See the machinery form up!

NEW PAGE!   http://members.1stnetusa.com/danmurphy/waldo.htm

Date: Thu, 01 Oct 1998 10:18:12 -0700

From: Gene Waldo <gwaldo@1stnetusa.com>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Another beginner...

Sender: tesl-err@emachine.com

ESchulz531@aol.com wrote:

>

> Hi,

>         I drilled holes in my plastic primary supports and used

> plastic ties to hold the copper tubing in place.  The supports

> are pieces of a plastic cutting board.  Heres a picture.

>

> http://members.aol.com/tesla00/primary.jpg

>

>         Its hard to tell in the picture but I drilled the holes on the

> side of the plastic not the top.

>

> Erik Schulz

Erik,

What's hard to tell, is, is that a 2ndary all lost in your humoungous

primary?

~~<[:-)

--

Gene Waldo -  gwaldo@1stnetusa.com  - "Igor in Residence"

---------------------------------------------------------------

What's good for the planet is more OZONE! See the machinery form up!

NEW PAGE!   http://members.1stnetusa.com/danmurphy/waldo.htm

From: DavidAllan@aol.com

Date: Thu, 1 Oct 1998 13:20:29 EDT

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Should capacitors match input transformer?

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

I am building my first coil, powered by one or two OBITs (10 kV/ 23 mA).

According to WinTesla, I should build a capacitance of 0.006 microfarad to

match with a single transformer, or 0.012 uf to match two OBITs in parallel. A

rolled cap of 0.006 uf would use aluminum flashing of dimensions 14 x 29

inches with a dielectric (PE) thickness of 0.060 in. (Tell me if I'm wrong,here.)

My question: I *could* build a larger cap that doesn't match the OBIT. My

plastic has larger dimensions, etc. Is it better to match the transformer or

to build the biggest cap you can? My goal for this coil is to make long

sparks. (Is that everyone's goal??) (FWIW, I will be using a 4 in secondary

and inverted saucer primary.)

Thanks.

David Allan

From: ESchulz531@aol.com

Date: Thu, 1 Oct 1998 14:06:18 EDT

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Another beginner...

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

In a message dated 10/1/98 12:19:15 PM Central Daylight Time,

gwaldo@1stnetusa.com writes:

> >

>  > Hi,

>  >         I drilled holes in my plastic primary supports and used

>  > plastic ties to hold the copper tubing in place.  The supports

>  > are pieces of a plastic cutting board.  Heres a picture.

>  >

>  > http://members.aol.com/tesla00/primary.jpg

>  >

>  >         Its hard to tell in the picture but I drilled the holes on the

>  > side of the plastic not the top.

>  >

>  > Erik Schulz

>

>  Erik,

>  What's hard to tell, is, is that a 2ndary all lost in your humoungous

>  primary

Actually it fits very nicely with my new 10" * 46" secondary

From: DavidAllan@aol.com

Date: Thu, 1 Oct 1998 14:13:19 EDT

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Another beginner...

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Dan, I'm working on my first coil, and I have been thinking about this, too.

What I *plan* to do is make 4-6 1/4-in plexiglass triangles (or cut off the

points in the center to make trapezoids). Then, drill 1/4 inch holes (or just

under, e.g. 15/64) located to hold the primary in position. Note that each

plexiglass "spoke" will have its holes offset slightly from the spokes before

and after. It may be possible to clamp all the spokes together with the offset

in place and drill straight through the stack. Anyway, after the holes are

drilled, I plan to drill and tap a couple of holes (1/8 in ?) from the top

edge of the spoke. Finally, I'll saw each spoke on a line through the centers

of the 1/4-in holes. That will give me a set of two-piece "clamps" that can be

held in place by the 1/8-in screws.

I've attached a small jpeg sketch of my idea. (I've seen other binaries sent

to the list. If I shouldn't do this, please send me a kind note offlist. I

don't have a website to host this file.)

David Allan

Dan Olsen wrote:

>

> I have all of the pieces of my first coil constructed, except the primary coil.

>

> I am finding that the mechanics of supporting it, keeping the spacing even,

> bending the tubing, etc.,  is much more difficult (for me) than any of the

> other parts, or calculations.

>

> Does anyone have any good ideas for supporting a quarter inch tubing with

> quarter inch (or so) spacing.  over 16-19 turns.   If a wooden

> form is the best I guess I will go ahead and do that.

>

> Thanks,

>

> --Dan

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Date: Thu, 01 Oct 1998 11:32:24 -0700

From: "paul m" <fleischwulf@mailexcite.com>

Cc: "bill pollack" <arcstarter@hotmail.com>

X-Sent-Mail: off

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

Subject: Ground current

X-Sender-Ip: 208.16.146.76

Organization: MailExcite  (http://www.mailexcite.com:80)

Sender: tesl-err@emachine.com

To all:

I was conducting some experiments the other night and decided to check my grounding cable. The case of my pig is connected to line neutral and ground. To my suprise the cable reading was 3 amps. I disconnected the coil and the transformer, still the ground current reading was 3 amps. I broke the connection and checked the voltage, it was too low to be measured < 1v.  I also noticed that when the coil was in operation the ground current dropped to 1 amp. I have seen some strange things since I started coiling but this one is really weird.

Any ideas ?

paul m

---

Einigheit und Wahrheit

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Date: Thu, 01 Oct 1998 11:41:41 -0700

From: "paul m" <fleischwulf@mailexcite.com>

Cc:

X-Sent-Mail: off

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

Subject: Re: Coil Contact, and Winding Primaries

X-Sender-Ip: 208.16.146.76

Organization: MailExcite  (http://www.mailexcite.com:80)

Sender: tesl-err@emachine.com

Tom,

If you are hell bent on self destruction and just have to touch the streamers, make damn sure that you are not grounded ! I talked with a man who has touched streamers. He said that initially he felt a small shock but no real pain.

Good luck,

paul m

---

Einigheit und Wahrheit

On Thu, 01 Oct 1998 09:56:45   tomstock wrote:

>Ok everyone, two quick questions:

>

>Lately I've been wanting to touch my streamers. Ok I guess this is kind

>of like wanting to stick your fingers in the belts while the car is

>running. Anyway, I read somewhere that if I held a metal object firmly

>in my hand I could touch the streamers providing there is absoloutely no

>secondary to primary arcing going on.

>

>Well, my coil is running from 15KV 60ma and I get somewhere around 24"

>streamers. Is this going to hurt?

>

>What about my feet?? Will streamers break from my feet and strike

>ground? Isn't this a bad thing?

>Should I stand on a milk crate?

>

>I'm not going to do anything until someone tells me it's (sort of) ok.

>

>Second question:

>

>How is everyone winding such nice primaries?? Mine looks good but

>definately not as good as some I've seen on the web. Is there a special

>tool I can get to make nice smooth turns?

>

>thanks

>-tom

>

X-Mailer: IPAD 2.0

Date: Thu, 01 Oct 1998 14:03:15 -0500

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: Coil Contact, and Winding Primaries

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

At 09:56 AM 10/1/98 -0700, you wrote:

>Ok everyone, two quick questions:

>

>Lately I've been wanting to touch my streamers. Ok I guess this is kind

>of like wanting to stick your fingers in the belts while the car is

>running. Anyway, I read somewhere that if I held a metal object firmly

>in my hand I could touch the streamers providing there is absoloutely no

>secondary to primary arcing going on.

>

>Well, my coil is running from 15KV 60ma and I get somewhere around 24"

>streamers. Is this going to hurt?

Yep.

>

>What about my feet?? Will streamers break from my feet and strike

>ground? Isn't this a bad thing?

>Should I stand on a milk crate?

Gary Weaver once told me that he was doing just that. He was

drawing streamers to himself using a piece of pipe held in the

hand while standing on a milk crate. During one of those episodes

a streamer broke out from his hip and struck a metal workbench.

He said it hurt like hell and he nearly passed out and fell off

the milk crate. This was the last time he pulled that stunt.

He told me he would never again take streamer hits to the body.

A lack of pri/sec arcing will not make a coil entirely safe.

The output will still contain a significant 60 hz component

as well as a DC component.

The things we do to our coils to maximize spark length make

them even more dangerous. Large toroids will store charge

like any good capacitor should and thereby increase streamer current.

>

>I'm not going to do anything until someone tells me it's (sort of) ok.

Well I wouldnt do it. I bear a permanent scar from a chance

encounter. Chances are you will be fine. But you never know.

Live long and Prosper!

>

>Second question:

>

>How is everyone winding such nice primaries?? Mine looks good but

>definately not as good as some I've seen on the web. Is there a special

>tool I can get to make nice smooth turns?

Well my primary is surely one of the ugliest around.

Ugly primaries and pretty ones provide the same amount of inductance.

If using copper tubing take advantage of the curvature of the tubing.

Bend it only enough to go where you want it. Go slowly and bend it

slowly and evenly.

You can fashion a curved spacer from some wood or plastic

to place between turns as you bend the tubing into shape.

TTYL!

Mike Hammer

mhammer@misslink.net

X-Mailer: IPAD 2.0

Date: Thu, 01 Oct 1998 14:11:04 -0500

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Mike Hammer <mhammer@misslink.net>

Subject: binary attachments

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

At 02:13 PM 10/1/98 EDT, you wrote:

>

>I've attached a small jpeg sketch of my idea. (I've seen other binaries sent

>to the list. If I shouldn't do this, please send me a kind note offlist. I

>don't have a website to host this file.)

>

>David Allan

David,

I think you forgot to attach the JPG image. Binary files are OK

as long as you dont send a megabyte of stuff all at once.

Send as external attachments.

A few small pics in JPG format are just fine. No BMPs please.

If you need some web space I would be glad to host some

pics of yours on my page if you wish. Contact me directly

if interested.

TTYL!

Mike Hammer

mhammer@misslink.net

X-Mailer: IPAD 2.0

Date: Thu, 01 Oct 1998 14:24:09 -0500

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: Should capacitors match input transformer?

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

At 01:20 PM 10/1/98 EDT, you wrote:

>I am building my first coil, powered by one or two OBITs (10 kV/ 23 mA).

>According to WinTesla, I should build a capacitance of 0.006 microfarad to

>match with a single transformer, or 0.012 uf to match two OBITs in parallel. A

>rolled cap of 0.006 uf would use aluminum flashing of dimensions 14 x 29

>inches with a dielectric (PE) thickness of 0.060 in. (Tell me if I'm wrong,

>here.)

Your math is correct.

While this will work at 10kv you will have a more reliable cap

if you use 2 identical units in series. Instead of a single

cap with 60 mils of poly use 2 caps in series each with 30 mils

of poly. Of course each individual cap will have to be twice the

capacitance of your target value.

>

>My question: I *could* build a larger cap that doesn't match the OBIT. My

>plastic has larger dimensions, etc. Is it better to match the transformer or

>to build the biggest cap you can? My goal for this coil is to make long

>sparks. (Is that everyone's goal??) (FWIW, I will be using a 4 in secondary

>and inverted saucer primary.)

>

>Thanks.

>

>David Allan

The matching cap size that the programs show or worked out

by formula is the cap that has the same impedance as your transformer

secondary at 60 hz. In other words your cap and tranformer secondary

are resonant at 60 hz. (or 50 hz as the case may be)

This partially defeats the internal current limiting of a neon

or obit and allows for higher current flow.

It is also much more stressful on the transformer.

This is also usually close to the largest cap size that your

transformer can reliably charge. But it is NOT necessarily the

best size for your particular arrangement.

Generally anything less than the matching size is acceptable.

Use the matching cap size as a guideline only.

TTYL!

Mike Hammer

mhammer@misslink.net

Date: Thu, 1 Oct 1998 14:46:59 -0600

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Daniel Lockhart <molly@cmn.net>

Subject: Re: Coil Contact, and Winding Primaries

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

tomstock wrote:

>Lately I've been wanting to touch my streamers. Ok I guess this is kind

>of like wanting to stick your fingers in the belts while the car is

>running. Anyway, I read somewhere that if I held a metal object firmly

>in my hand I could touch the streamers providing there is absoloutely no

>secondary to primary arcing going on.

>

>Well, my coil is running from 15KV 60ma and I get somewhere around 24"

>streamers. Is this going to hurt?

Using 12 kV, at 60 ma on my *three-inch* coil, it hurt pretty bad. Only a

half-second of contact and I figured out this wasn't a fun thing to do.

With your four inch coil running at 60 ma, it has a greater capability of a

bigger shock.

>What about my feet?? Will streamers break from my feet and strike

>ground? Isn't this a bad thing?

>Should I stand on a milk crate?

Running my coil at 30 ma (with is entirely a different story), I felt a

"buzzing" in a foot if I lifted the other off the ground. It wasn't

sparking off of me, I don't know what it was doing. (?) The electricity has

to go somewhere, and woe be on you if a streamer breaks of off you!

>I'm not going to do anything until someone tells me it's (sort of) ok.

I wouldn't.

My two cents,

Daniel Lockhart

From:          "Malcolm Watts" <MALCOLM@directorate.wnp.ac.nz>

Organization:  Wellington Polytechnic, NZ

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Date:          Fri, 2 Oct 1998 08:48:46 +1200

Subject:       Re: Another beginner...

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

One of the easiest ways to make supports for tube primaries is to get

1/2" thick blocks of plastic as long as the "thickness" or width of

the winding. If your primary is to be a flat spiral, drill a series

of 1/4" holes in line, each spaced at half inch intervals. Then get a

hacksaw and cut along a line going right through the centre of the

holes. Presto! Two supports made. For a conical primary, drill the

holes along the diagonal and cut along the diagnoal. 6 - 8 supports

(3 - 4 blocks) should do for most primaries.

Malcolm

> Date sent:      Thu, 01 Oct 1998 09:17:19 -0600

> From:           Dan Olsen <dan@olsen.org>

> Send reply to:  "Tesla-2 Mailing List" <tesla-2@emachine.com>

> To:             "Tesla-2 Mailing List" <tesla-2@emachine.com>

> Subject:        Another beginner...

> I have all of the pieces of my first coil constructed, except the primary coil.

>

> I am finding that the mechanics of supporting it, keeping the spacing even,

> bending the tubing, etc.,  is much more difficult (for me) than any of the

> other parts, or calculations.

>

> Does anyone have any good ideas for supporting a quarter inch tubing with

> quarter inch (or so) spacing.  over 16-19 turns.  I am looking for an off the

> shelf solution here,  I realize I could measure and cut 20 quarter inch slots

> in 10 or 12 wooden spokes. But I figure that Is over 200 cuts that I have to

> make 3 or 4 times each to get 1/4 inch, and that will take at least several

> hours with the skillsaw.  If I do angle it at 10-15 degrees, what's a good way

> to hold it all up.  (I studied physics in college, not structural

> engineering.).

>

> I have been rather proud so far of the usability of off-the-shelf,

> hardware-store components.  The dryer-exhaust tubing, the PVC pipe, the copper

> tube spark gap, the aluminum flashing, polyethylene capacitor, etc. It's like a

> McGuyver thing, where you walk-in buy some common items, and create an amazing

> contraption.

>

> I'd be really impressed if someone could think of a way to adapt an easily

> availiable item, to use as a support and spacer for a primary coil, or if they

> just have any ideas that are better than mine above for doing this.  Is there

> anything related to hanging clothes or ties or somthing that would have plastic

> pegs every half inch or so?  How about it what have you guys done?  If a wooden

> form is the best I guess I will go ahead and do that.

>

> Thanks,

>

> --Dan

>

>

From:          "Malcolm Watts" <MALCOLM@directorate.wnp.ac.nz>

Organization:  Wellington Polytechnic, NZ

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Date:          Fri, 2 Oct 1998 08:52:22 +1200

Subject:       Re: Ground current

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Hi Paul,

I missed a lot of your post due to the lack of line feeds.

Where you have a power system that has a grounded neutral and if the

cold side of the pig primary is connected to the case, you will find

neutral return currents are shared between the ground and neutral,

the division depending on the relative resistance of the two paths.

Malcolm

From: Hollmike@aol.com

Date: Thu, 1 Oct 1998 20:41:54 EDT

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Another beginner...

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

In a message dated 10/1/98 9:21:15 AM Mountain Daylight Time, dan@olsen.org

writes:

> Does anyone have any good ideas for supporting a quarter inch tubing with

>  quarter inch (or so) spacing.  over 16-19 turns.

Dan,

There are a few options for supporting your primary:

1)  cut out six triangular(or trapezoidal)  pieces of wood or plastic in the

profile that you wish for your primary (15 or whatever you choose) and drill

the holes(slightly larger than 1/4 inch) at the points where the tubing would

feed through.  This is not the easiest option, but it holds the tubing in

place very well.  It can get tedious feeding the tubing through all those

holes though(I know because this is how I did mine)

2)  instead of drilling the holes, go to an automotive supply store and get

some spark plug wire spacers and mount them to the top of the supports.

3)  use tubing clamps.  A particularly good choice of brands is Legris.  These

are black plastic clamps that are mounted with a small wood screw into the

supports.

Hope this at least gives you food for thought.

Mike Hollingsworth

X-Originating-IP: [216.68.41.150]

From: "Bill the arcstarter" <arcstarter@hotmail.com>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Another beginner...

Date: Thu, 01 Oct 1998 19:00:07 PDT

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

Erik,

What is the no-load primary current of your homemade HV transformer?

Thanks!

-Bill the arcstarter

Starting arcs in Cinci, OH

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: R.E. Coiler Mycroft

X-Mailer: IPAD 2.0

From: break7@juno.com (Tyla A. Break)

Date: Thu, 01 Oct 1998 22:23:36 EDT

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Yes, APEX has variacs, tiny 400hz to huge 60amp three phase.  For

mid sizes you have to look around.  They also have about 160 lbs of #16ga

regular and formvar wire.  It might be worth it just to check it out if you

have the time.  This is a shortened message because my computer jumped out

of windows and dumped the last 12 messages.

Kyle

break7@juno.com

P.S.

could you please forward

me the last 10 posts of

Tesla 2 if you have them?

Date: Thu, 01 Oct 1998 22:03:32 -0500

From: Bert Hickman <bert.hickman@aquila.com>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Organization: Stoneridge Engineering

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Running higher than 400 BPS?

Sender: tesl-err@emachine.com

Bill the arcstarter wrote:

>

> Bert wrote:

>

> >Nice caps, Marco! The combined 50 kV rating will provide good margin for

> >voltage reversals, and these particular caps are indeed low-loss pulse

> >discharge type. You should be able to run them at higher break rates as

> >long as you reduce any continuous run-time to no more than 1-2 minutes,

> >and allow at least 2 minutes of cool-down time for internal heat to

> >redistribute. All other things being the same (which they rarely are

> >:^)), you'll see internal capacitor heating that's proportional to the

> >breakrate - double the BPS and you'll see at least twice the power

> >dissipation in the caps that you see now. Since they are surviving the

> >existing environment at 367 BPS, they'll most likey survive at 700 BPS

> >as long as you don't overheat them.

>

> OK Bert - what is going on?  Why is the capacitor heating proportional

> to the break rate?

>

> Has anyone been able to show that the arc is interrupted prior to all

> the primary energy being transferred into the secondary?

>

> What I'm wondering is whether you are really able to get 700 bps, or

> does the arc last so long that it doesn't stop before the next electrode

> lines up??

>

> -Bill

>

Bill and all,

Question #1:

OK... Commercial pulse caps used in Tesla Coil service will see mainly

two types of losses - dielectric and ohmic. The dielectric losses tend

to be mainly due to polarization [during charging] and relaxation

[during discharging], and both are a function of the applied frequency

and ambient temperature. Ceramic caps may also suffer from another type

of loss (ferroelectric hysteresis), and home-made glass caps or poorly

constructed poly caps may also exhibit significant corona losses. Ohmic

losses are associated with the plate and interconnection resistance, and

include skin-effect losses.

Dielectric and ohmic losses only become significant during the time that

the main gap fires. In effect, a small percentage of the energy

initially stored in the tank cap (the "bang size") is converted into

heat within the tank cap every time the gap fires. So... doubling the

break rate doubles the total amount of heat injected inside the

capacitor. With an efficient pulse cap, the amount of heat will be

small, and higher breakrates can be supported without overheating the

internals of the cap. A lossier cap will need shorter runtimes and

longer cool-down times to survive.

Question #2:

The main gap will extinguish when the primary current goes through "0",

and there's insufficient voltage to reignite the gap. For most coils,

some residual voltage will be left on the tank cap when the gap

extinguishes, meaning that there will be some energy "stranded" in the

primary. Certain "magic" primary:secondary coefficients of coupling that

will minimize this effect (0.22. 0.18, 0.153, 0.133...).

Question #3:

It is indeed possible to achieve true breakrates of 700+ BPS, although

it's not trivial. As indicated, this typically requires using a smaller

tank cap, a stiffer HV supply, and using more than one static-rotary set

of gaps connected in series to increase the rate of electrode

seperation.

-- Bert --

From: ESchulz531@aol.com

Date: Thu, 1 Oct 1998 23:12:06 EDT

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Another beginner...

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Bill,

Since I am at college right now my transformer is about 200 miles away.  I

should go back home during thanksgiving.  During this time I will post a lot

more information about my transformer.  Sorry, I know this isn't what you

wanted to here.

Erik Schulz

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Touching Your Coil

X-Mailer: IPAD 2.0

From: break7@juno.com (Kyle A. Break)

Date: Thu, 01 Oct 1998 23:15:27 EDT

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

To all, last tuesday my computer dumped a couple files that run the E

Mail program. So I haven't seen any posts.

Coiler Mycroft; APEX has variacs , tiny400hz to HUGE 60 amp 3 phase

Also they have ~ 160lbs #16 wire, regular or formvar. Cheap Cheap!

Personal contact-  I have a video of Irwin Moon standing on top of what

looks like a magnifier.  Very impressive - 4 to 6 foot arcs comming off

his fingers. My dad has also seen it in person. But extreme danger exists

(not fun!).

As I understand it the conditions must be perfect!

1.  You must be elevated off the ground with a very good insulator.

2. Insulated platfofm must be farther away from ground than coil can strike.

3. Entrance and exit points must be metal.

4. Arcs comming from you must never contact anything grounded.

5. If at any time you are touching your coil and there is a flash to the

primary, you are now connected to 60hz High Current, 10,000 volt supply.

Should you have an arc comming from you to ground (very possible) at this

same moment, you are now passing possibly 10, 000 watts of 60hz current

THROUGH your body.   Fried Chicken! Extra Crispy.

6. If this happens with even a small coil you could be KILLED. Instantly.

This is how I understand it. Does any body agree with this? Is there

anything wrong with thinking?

If it's not (mildly) dangerous it's not fun!  Kyle

break7@juno.com

From:          "Malcolm Watts" <MALCOLM@directorate.wnp.ac.nz>

Organization:  Wellington Polytechnic, NZ

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Date:          Fri, 2 Oct 1998 16:13:38 +1200

Subject:       Re: Touching Your Coil

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

> To:             "Tesla-2 Mailing List" <tesla-2@emachine.com>

> Subject:        Touching Your Coil

> From:           break7@juno.com (Kyle A. Break)

> Date sent:      Thu, 01 Oct 1998 23:15:27 EDT

> Send reply to:  "Tesla-2 Mailing List" <tesla-2@emachine.com>

>   To all, last tuesday my computer dumped a couple files that run the E

> Mail program. So I haven't seen any posts.

>    Coiler Mycroft; APEX has variacs , tiny400hz to HUGE 60 amp 3 phase

> Also they have ~ 160lbs #16 wire, regular or formvar. Cheap Cheap!

>

>   Personal contact-  I have a video of Irwin Moon standing on top of what

> looks like a magnifier.  Very impressive - 4 to 6 foot arcs comming off

> his fingers. My dad has also seen it in person. But extreme danger exists

> (not fun!).

>   As I understand it the conditions must be perfect!

> 1.  You must be elevated off the ground with a very good insulator.

> 2. Insulated platfofm must be farther away from ground than coil can strike.

> 3. Entrance and exit points must be metal.

> 4. Arcs comming from you must never contact anything grounded.

> 5. If at any time you are touching your coil and there is a flash to the

>    primary, you are now connected to 60hz High Current, 10,000 volt supply.

>    Should you have an arc comming from you to ground (very possible) at this

>    same moment, you are now passing possibly 10, 000 watts of 60hz current

>    THROUGH your body.   Fried Chicken! Extra Crispy.

> 6. If this happens with even a small coil you could be KILLED. Instantly.

>

> This is how I understand it. Does any body agree with this?

Totally.

Malcolm

X-Mailer: IPAD 2.0

Date: Fri, 02 Oct 1998 08:45:11 +0200

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Marco Denicolai <marco@vistacom.fi>

Subject: Re: Coil Contact, and Winding Primaries

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

At 09:56 01/10/98 -0700, you wrote:

>How is everyone winding such nice primaries?? Mine looks good but

>definately not as good as some I've seen on the web. Is there a special

>tool I can get to make nice smooth turns?

>

>thanks

>-tom

>

- Buy the pipe in a roll, avoid rolls already smashed.

- Lay the pipe starting from the outer turn, directly into already cut and ready

positioned spacers. Do NOT pretend to first roll the primary and then put it

into place with the spacers.

- Use something to temporarly fix (or press) the pipe to the spacers until

you are done with

all the primary and you can glue it to the spacers.

- The only tool you need is your bare hands and maybe a big welder tip or a

torch if you want to solder one of the primary ends (as I did).

Marco Denicolai                   Vista Communication Instruments, Inc.

Hardware Development Manager      www.vistacom.fi

marco@vistacom.fi

X-Mailer: IPAD 2.0

Date: Fri, 02 Oct 1998 08:52:42 +0200

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Marco Denicolai <marco@vistacom.fi>

Subject: Re: Another beginner...

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

At 09:17 01/10/98 -0600, you wrote:

>Does anyone have any good ideas for supporting a quarter inch tubing with

>quarter inch (or so) spacing.  over 16-19 turns.  I am looking for an off the

>shelf solution here,  I realize I could measure and cut 20 quarter inch slots

>in 10 or 12 wooden spokes. But I figure that Is over 200 cuts that I have to

>make 3 or 4 times each to get 1/4 inch, and that will take at least several

>hours with the skillsaw.  If I do angle it at 10-15 degrees, what's a good way

>to hold it all up.  (I studied physics in college, not structural

>engineering.).

Dan,

You line up the 12 wooden spokes side to side and close them into a vice.

You then make 20 (twenty) cuts, each of then cutting all 12 spokes at once

and that's it. It takes maybe half an hour if you are carefull, less if you

want it done so-so.

This work is nothing compared to winding the secondary or building a spark gap.

Or did you get also those "off-the-shelf"?

Marco Denicolai                   Vista Communication Instruments, Inc.

Hardware Development Manager      www.vistacom.fi

marco@vistacom.fi

X-Mailer: IPAD 2.0

Date: Fri, 02 Oct 1998 09:54:20 +0200

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Marco Denicolai <marco@vistacom.fi>

Subject: TC and magnifier comparison

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Knowing nothing (yet) about magnifiers I was wondering this:

1.For the same tank supply (i.e. same kVA and kVs) do I get better performance

(i.e. longer streamers) if I build a conventional TC or if I build a

magnifier?

2.What are the differences in performance (other than streamer lenght) between

a TC and a magnifier?

3.Why one would build a magnifier rather than a TC?

This is NOT to discredit magnifiers, but just to decide what I'll build

myself next, now that I think I suck all the performance I could from my

first conventional TC.

Marco Denicolai                   Vista Communication Instruments, Inc.

Hardware Development Manager      www.vistacom.fi

marco@vistacom.fi

Date: Fri, 02 Oct 1998 07:09:46 -0500

From: Bert Hickman <bert.hickman@aquila.com>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Organization: Stoneridge Engineering

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: TC and magnifier comparison

Sender: tesl-err@emachine.com

Marco Denicolai wrote:

>

> Knowing nothing (yet) about magnifiers I was wondering this:

>

> 1.For the same tank supply (i.e. same kVA and kVs) do I get better performance

> (i.e. longer streamers) if I build a conventional TC or if I build a

> magnifier?

>

Not better.... just different. In the final analysis, it appears to take

a similar amount of power to create a given length streamer irrespective

of whether the systm that created it is a 2-coil or 3-coil system. It is

possible to get streamers that are significantly longer relative to the

resonator's length with a magnifier. For example a well constructed

classical 2-coil systems is limited to streamer lengths that are 3-4X

the resonator length due to the presence of the primary and the fact

that the base of the resonator is at ground potential.

> 2.What are the differences in performance (other than streamer length) between

> a TC and a magnifier?

>

A well-constructed magnifier can exceed 11X the resonator length. The

best performing magnifier today belongs to Richard Hull of the Tesla

Coil Builders of Richmond (TCBOR). His resonator is only about 1 foot

long, is suspended from the ceiling, and uses a very large toroid - he's

able to achieve steamers that are over 11 times the length of his

resonator. While the theory of operation is well understood for 2-coil

systems, there is much more that needs to be done to fully understand

magnifier operation. The bottom line appears to be that a well

constructed 2-coil system should be able to achieve similar performance

(streamer length) when run at the same input power level as a well

constructed magnifier.

> 3.Why one would build a magnifier rather than a TC?

>

> This is NOT to discredit magnifiers, but just to decide what I'll build

> myself next, now that I think I suck all the performance I could from my

> first conventional TC.

>

Historically, a magnifier is the system that Tesla used to do the

majority of his research with at Colorado Springs. Building a magnifier

is also considerably more challenging than building a 2-coil system.

Success requires broader knowledge and experience... with the latter

being the most important. If you don't have significant prior experience

with 2-coil systems, you'll certainly end up with LOTS of experience by

the time you complete a magnifier (and probably a fair amount of smoke

initially...).

While there are probably thousands of people worldwide who have working

2-coil systems, there are probably less than 20 who have working high

performance magnifiers, and only a handful who've built systems that run

at the 7.5 KVA level or above. While there appears to be no significant

advantage of streamer length versus input power, there can be

considerable satisfaction in the knowledge that you've conquered the

significant electrical, mechanical, materials, and operational problems

that come with designing and constructing a high performance magnifier

system.  Its a mountain you can climb once you've conquered all the

local 2-coil hills. But, like mountain climbing, don't tackle it unless

you're serious and willing to spend substantial effort at it...

Good luck!

-- Bert --

X-Originating-IP: [199.217.140.22]

From: "Bill the arcstarter" <arcstarter@hotmail.com>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Another beginner...

Date: Fri, 02 Oct 1998 06:22:49 PDT

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

Erik wrote:

>Bill,

>	Since I am at college right now my transformer is about 200 miles away.

> I should go back home during thanksgiving.  During this time I will post a lot

>more information about my transformer.  Sorry, I know this isn't what you

>wanted to hear.

OK.  Let me know when you get a chance!  I've thought about winding my

own, but couldn't convince myself that my unit would have an acceptable

no-load current...

Thanks for replying.

-Bill the arcstarter

Starting arcs in Cinci, OH

http://www.geocities.com/CapeCanaveral/Hangar/6160

From: "Basura, Brian" <brian.basura@unistudios.com>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Plastic Sources....

Date: Fri, 2 Oct 1998 10:25:34 -0700

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Here is a good source for ALL types of plastics.

www.usplastic.com

or e-mail them at usp@usplastic.com and request a catalog.

I've been eyeing their "Lay Flat 6mil Poly tubing".  It looks like it would

be great for stacked plate caps and would eliminate a lot of the cutting

involved.

Regards,

Brian D. Basura

From: DavidAllan@aol.com

Date: Fri, 2 Oct 1998 15:35:42 EDT

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: binary attachments

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Thanks for pointing out that the binary didn't make it, Mike. If it's not with

the message this time, then it's because AOL is doing something funny.

However, I'm rethinking the supports I'll use. Some of the suggestions that

just appeared look pretty good to me.

David Allan

In a message dated 98-10-01 15:21:24 EDT, you write:

<< Subj:	 binary attachments

Date:	98-10-01 15:21:24 EDT

From:	mhammer@misslink.net (Mike Hammer)

Sender:	tesl-err@emachine.com

Reply-to:	tesla-2@emachine.com (Tesla-2 Mailing List)

To:	tesla-2@emachine.com (Tesla-2 Mailing List)

At 02:13 PM 10/1/98 EDT, you wrote:

>

>I've attached a small jpeg sketch of my idea. (I've seen other binaries sent

>to the list. If I shouldn't do this, please send me a kind note offlist. I

>don't have a website to host this file.)

>

>David Allan

David,

I think you forgot to attach the JPG image. Binary files are OK

as long as you dont send a megabyte of stuff all at once.

Send as external attachments.

A few small pics in JPG format are just fine. No BMPs please.

If you need some web space I would be glad to host some

pics of yours on my page if you wish. Contact me directly

if interested.

TTYL!

Mike Hammer

mhammer@misslink.net

Attachment Converted: "C:\MISSING_LINK\EUDORA\Attach\PRICLAMP.jpg"

(Editors note. Binary attachments not in archive)

From: DavidAllan@aol.com

Date: Fri, 2 Oct 1998 15:35:43 EDT

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Double posts

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

I think I'm sending double posts. I'm not intending to, obviously. I'm using

AOL FlashSessions to upload my posts. If someone knows what I'm doing, please

send me mail offlist.

Thanks.

David Allan

X-Mailer: IPAD 2.0

Date: Fri, 02 Oct 1998 22:50:05 +0100

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: bob golding <yubba@clara.net>

Subject: Re: Plastic Sources....

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

At 10:25 02/10/98 -0700, you wrote:

>

>I've been eyeing their "Lay Flat 6mil Poly tubing".  It looks like it would

>be great for stacked plate caps and would eliminate a lot of the cutting

>involved.

Hi Brian,

I thought of using this and found that most of the cheaper poly used for

packing  purposes all had small air bubbles in. Get them to send you a

sample. It would be a real shame to use inferior materials on such a good

looking coil. i am green with envy. What did you use for the two round

platforms on your coil?

bob golding

bob yubba@clara.net

From: "Basura, Brian" <brian.basura@unistudios.com>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Primary Clamps (was: binary attachments)

Date: Fri, 2 Oct 1998 14:59:43 -0700

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

David,

I suggest HDPE as a material.  You can get a large cutting board from Costco

or a piece from a plastic supplier in your area.  HDPE machines well and if

you use nylon screws there won't be any metal in your supports.

Regards,

Brian D. Basura

> ----------

> From: 	DavidAllan@aol.com[SMTP:DavidAllan@aol.com]

> Reply To: 	Tesla-2 Mailing List

> Sent: 	October 02, 1998 12:35 PM

> To: 	Tesla-2 Mailing List

> Subject: 	Re: binary attachments

>

> <<File: PRICLAMP.JPG.jpe>>

> Thanks for pointing out that the binary didn't make it, Mike. If it's not

> with the message this time, then it's because AOL is doing something funny.

> However, I'm rethinking the supports I'll use. Some of the suggestions

> that just appeared look pretty good to me.

>

<snip>

>  At 02:13 PM 10/1/98 EDT, you wrote:

>  >

>  >I've attached a small jpeg sketch of my idea. (I've seen other binaries sent

>  >to the list. If I shouldn't do this, please send me a kind note offlist. I

>  >don't have a website to host this file.)

>  >

>  >David Allan

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: APEX

X-Mailer: IPAD 2.0

From: break7@juno.com (Kyle A. Break)

Date: Fri, 02 Oct 1998 23:16:09 EDT

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

For anyone that wants to know. APEX is located at 8909 San

Fernando

Sun Valley CA

91352

From riverside, 10 West, to 57 North, to 210 West. Off at La Tuna

canyon west  (down the canyon) . When you get to the bottom go strait

through the intersection and continue on that street (Tuxford) to San

Fernando Road . Make a right turn on San Fernando road and go about 2

blocks . It'll be right on the left corner.

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: "Coiler" <mycroft@access1.net>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Cc: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Power Applied?, and further wiring tricks..

Date: Sat, 3 Oct 1998 09:16:19 -0700

X-Mailer: IPAD 2.0

Importance: Normal

X-MimeOLE: Produced By Microsoft MimeOLE V4.72.3155.0

Sender: tesl-err@emachine.com

First: To those that told me resistive ballasting sucks, and I really need

inductive ballasting, you were right.. hoo boy were you right!

Using the same neons that I used to use to power the system as ballast

I got far more impressive output.. to the point of where I am

going to have to take the coil outside because it is savaging

the walls and garage door. This means I hitting at least 45" with

the two 12KV/30ma units ballasting my 14.4KV pig.

Second: Query on power usage:

I am installing a 30A/220 circut to feed my pig. I intend to bring

the power to my control box in such a way as I can get some 110V

for the control transformers for the dead-man, etc. Therefore I

intend to use 110-0-110 with 30A breakers on each leg. This got

me thinking (dangerous I know) - Could I use a 110V neon or MOT

on each side of the pig, with the neutral on the CT? Thus each

limiter would only have to handle approximately 3.5KVA max? At

110 Volts no less, far easier to find.  But

the pig would see 7KVA? Or am I wrong, and that wiring of 30A

dual breaker actually delivers twice the power I think it does?

Someone please tell me if I am all wet or not.

requisite crude ascii art diagram:

PIG

|     |      |

B     CT     B

| 110 |  110 |

LINE

Thanks...

Michael Baumann

Coiler, Homebrewer, Nerd. mycroft@access1.net

X-Originating-IP: [195.121.16.239]

From: "Sam Barros" <sambarros@hotmail.com>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Thanks a lot everybody!!!

Date: Sat, 03 Oct 1998 11:42:07 PDT

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

Thanks a lot for everybodys response on my Tesla Coil questions.

I will now start winding my 18 turn, 8mm thick copper tubing, 8mm turn

to turn spacing, 2.5cm-primary/secondary clearance primary coil. Ill

wind it around 6, 8mm thick polyurethane varnished plywood coil forms. I

am not fully satisfied with this coil because it is way too wide to make

my coil truly portable, but still, you guys said it was OK, so it must

be OK

I thank you all for trying to help me with the transformer problem as

well. If anyone can get a 1000W NST for me for less than 60 dollars

(shipping included) I would be very interested. Still, I dont think

thatll be possible.

You know Holland sucks if you want to build a TC. I paid 50 dollars for

650 meters of 0.6mm enamel wire, 80 dollars for 30 meters of 8mm thick

copper tubing, some 100 dollars in materials for my coil form. Hell, a

30X100cm sheet of 8mm thick plywood costs 30 dollars!!!

Now, if you want a transformer, you can buy a 1KW MOT for 150 dollars.

EACH!!!

I simply cannot afford that. This is supposed to be a hobby, not a

rip-off. Specially if you consider the fact that I would need at least 4

MOTs to power a coil (I have one that I got from my ex-microwave oven).

Than they make marijuana legal and they outlaw ALL Neon Light

Transformers above 400W of power. I mean, COME-ON!!! Whats wrong with

this people??!!

Anyway, now that I have expressed my feelings  of jealousy for all you

lucky Americans that can go to the nearest Neon sign shop and get all

the transformers youll ever need for free, or can go to a scrapyard and

get a POLE PIG for 5 dollars, Ill go on to some practical questions:

Ok. So now I have my aluminium dryer duct toroid measuring 13cm tube

diam. and 20cm inner diameter. I have a 1000turn 0.6mm 20cm diameter,

60cm winding height secondary coil, well dry and varnished. And I will

soon have my primary done. Some problems now begin to show up

First of all: Is Enamel Varnish as good as Polyurethane varnish for

insulating HV? How do the two compare?

Second: Could I make a PVC pipe capacitor, or a Plexiglas sheet

capacitor that would compare in performance to those PE caps you guys

use? Polyethylene is non-existent in here But I can get some nice

sheets of Plexiglas for 7 dollars each. They are squared, with some 30 X

30cm and about 0.5mm thick. I figured out that several of them

alternating with aluminium sheeting would make an easy capacitor. It

will cost a lot, but I was wondering if it would be any good How about

PVC capacitors? Too lossy I suppose

Well, I was going to ask something else, but I forgot what it was

Well, Ill talk to you guys later.

Anyway, THANKS A LOT for all the help I got so far!!!

Sam.

P.S. Whats the Tesla Coil chat channel again?

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: bkibler@quicktel.com (Robert Kibler)

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: ballast

Date: Sat, 3 Oct 1998 11:49:48 -0700

X-Mailer: IPAD 2.0

X-MimeOLE: Produced By Microsoft MimeOLE V4.72.2106.4

Sender: tesl-err@emachine.com

Hi all,  this beginner coiler needs to know about the concept of BALLAST.

What is its purpose?  Right now my coil is running off a 5 amp variac to a

12 kv 60 ma neon trans.  So far, I have been using it conservatively,  with

less than 30 volts coming out of the variac.  Using a .01 mf cap. and .15"

gap, my 3" dia. secondary will produce about 10" sparks.  But I would like

to get bigger.  At what point in this "growing up" does one start needing

ballast?  It sounds like other transformers can be used as ballast, but I

don't know what kind or where in the circuit or even why.  There seems to be

very little info out there about it.

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Power Applied?, and further wiring tricks..

X-Mailer: IPAD 2.0

From: break7@juno.com (Kyle A. Break)

Date: Sat, 03 Oct 1998 14:58:59 EDT

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

On Sat, 3 Oct 1998 09:16:19 -0700 "Coiler" <mycroft@access1.net> writes:

>First: To those that told me resistive ballasting sucks, and I really

>need inductive ballasting, you were right.. hoo boy were you right!

>	 Using the same neons that I used to use to power the system

>as ballast I got far more impressive output.. to the point of where I am

>going to have to take the coil outside because it is savaging

>the walls and garage door. This means I hitting at least 45" with

>the two 12KV/30ma units ballasting my 14.4KV pig.

>

>Second: Query on power usage:

>	 I am installing a 30A/220 circut to feed my pig. I intend to bring

>the power to my control box in such a way as I can get some 110V

>for the control transformers for the dead-man, etc. Therefore I

>intend to use 110-0-110 with 30A breakers on each leg. This got

>me thinking (dangerous I know) - Could I use a 110V neon or MOT

>on each side of the pig, with the neutral on the CT? Thus each

>limiter would only have to handle approximately 3.5KVA max? At

>110 Volts no less, far easier to find.  But the pig would see 7KVA?

>Or am I wrong, and that wiring of 30A dual breaker actually delivers

>twice the power I think it does?

>    Someone please tell me if I am all wet or not.

>

>	requisite crude ascii art diagram:

>

>                PIG

>           |     |      |

>           B     CT     B

>           | 110 |  110 |

>               LINE

>

>Thanks...

>Michael Baumann

>Coiler, Homebrewer, Nerd. mycroft@access1.net

>

>

The power across each one would still be 7.5 kva unless they are

put in parallel on one side. With 110-0-110  0 being connected to neutral

and a limiter on each side, the neutral will probably carry almost no

current ~less than 1 amp. In an ideal world , center tap being exactly in

the middle, current limmiters exactly the same resistance - inductance

,and no leakage, current on neut. would be 0 .

If it's not dangerous it's not fun!  Kyle  break7@juno.com

Date: Sat, 03 Oct 1998 14:26:12 -0500

From: Bert Hickman <bert.hickman@aquila.com>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Organization: Stoneridge Engineering

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: ballast

Sender: tesl-err@emachine.com

Robert Kibler wrote:

>

> Hi all,  this beginner coiler needs to know about the concept of BALLAST.

> What is its purpose?  Right now my coil is running off a 5 amp variac to a

> 12 kv 60 ma neon trans.  So far, I have been using it conservatively,  with

> less than 30 volts coming out of the variac.  Using a .01 mf cap. and .15"

> gap, my 3" dia. secondary will produce about 10" sparks.  But I would like

> to get bigger.  At what point in this "growing up" does one start needing

> ballast?  It sounds like other transformers can be used as ballast, but I

> don't know what kind or where in the circuit or even why.  There seems to be

> very little info out there about it.

Hi Bob!

A Neon Sign Transformer (NST) is specially constructed so that it can

safely run with the secondary short-circuited. The internal construction

of a NST is takes a portion of the primary's magnetic flux and "shunts"

it back to the primary before it ever reaches the secondary. This is

done by means of - you guessed it! - magnetic shunt plates, and all

NST's have them. Thus, NST's do not require any external ballast, since

they already have built-in current limiting.

Now most other types of transformers do NOT use this technique - the

primary and secondary are very closely coupled. When the secondary is

shorted on one of these non-shunted transformers, excessive current can

flow which may pop a circuit breaker or overheat and burn up the

transformer's windings. Non-shunted HV transformers, including plate,

utility distribution ("pole-pigs"), and most microwave transformers thus

require some external method to limit current flow, since Tesla coils

short-circuit the secondary whenever the main gap fires. The current

limiting function is typically done by connecting an external inductor,

high power resistor, or a combnation of the two in series with the HV

transformer's primary winding. This external current limiting circuit is

called a ballast. Other common uses for ballasts include gas tube

lighting (fluorescent, mercury or sodium vapor), arc welding, and carbon

arc lighting.

So... your NST-powered coil does not require any external ballast, even

if you begin to run off multiple NST's. You can simply begin operating

at higher variac settings while carefully watching for any signs of

abnormal behavior on your coil. This is often best done in a darkenned

room. Look for any signs of inter-turn flashovers, secondary-to-primary

flashovers, or corona where you don't want it. Also, make sure that your

coil is in tune, and correct any problems you encounter before

proceeding to higher power levels.

And... Safe coilin' to you!

-- Bert --

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: ballast

X-Mailer: IPAD 2.0

From: break7@juno.com (Kyle A. Break)

Date: Sat, 03 Oct 1998 16:15:27 EDT

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

On Sat, 3 Oct 1998 11:49:48 -0700 bkibler@quicktel.com (Robert Kibler)writes:

>Hi all,  this beginner coiler needs to know about the concept of BALLAST.

>What is its purpose?  Right now my coil is running off a 5 amp variac to a

>12 kv 60 ma neon trans.  So far, I have been using it conservatively, with

>less than 30 volts coming out of the variac.  Using a .01 mf cap and .15"

>gap, my 3" dia. secondary will produce about 10" sparks.  But I would like

>to get bigger.  At what point in this "growing up" does one start needing

>ballast?  It sounds like other transformers can be used as ballast, but I

>don't know what kind or where in the circuit or even why.  There seems to be

>very little info out there about it.

>

>

If your using neon sign trans. you don't need a ballast. The

current limiting is done inside. Try shorting the output  What happens?

Nothing much. Because the trans is limiting the output to 60ma. Of course

the voltage has dropped to 0 volts.

Now if you have a pole trans or a potential trans, there is no current

limiting. If you hook it up and put a ammeter on the primary then try to

draw an arc on the secondary you'll find yourself in the dark wondering

what happened. What happens is, when the arc starts you dont even see it

and it looks like a direct short to the trans. So the primary current

will rise to a couple hundred amps and blow your fuse or breaker. This

all happens in a 120th of a second. So you need to wire a heavy current

inductor in series with the primary. Now at idle(no load on secondary)

there is very little voltage drop across the ballast (inductor) so you've

got full voltage on the secondary.  Now when you load the secondary with

a short (arc) there is a large voltage drop across the ballast,  limiting

the current .

So first find a choke or trans with a heavy winding  #12 for 20amps

cont. #10 for 30amp or so on. You can also parallel coils for more

current if they are the same . Transformers that work well will have a

120 volt winding or 100- 125 turns.

If it's not dangerous it's not fun!  Kyle  break7@juno.com

X-Originating-IP: [216.68.40.106]

From: "Bill the arcstarter" <arcstarter@hotmail.com>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Power Applied?, and further wiring tricks..

Date: Sat, 03 Oct 1998 14:13:42 PDT

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

Hello Michael, coilers,

Michael wrote:

>       intend to use 110-0-110 with 30A breakers on each leg. This got

>	 me thinking (dangerous I know) - Could I use a 110V neon or MOT

>       on each side of the pig, with the neutral on the CT? Thus each

>   	 limiter would only have to handle approximately 3.5KVA max? At

>       110 Volts no less, far easier to find.  But

>       the pig would see 7KVA? Or am I wrong, and that wiring of 30A

>       dual breaker actually delivers twice the power I think it does?

>       Someone please tell me if I am all wet or not.

>

>	requisite crude ascii art diagram:

>

>                PIG

>           |     |      |

>           B     CT     B

>           | 110 |  110 |

>               LINE

I would suggest NOT connecting the pig's neutral lug to anything at all.

Just use your two limiting devices in each leg, as you have indicated.

The neutral connection is on the pig ONLY to provide 120 volt service

the for house in question.  For coiling, using the neutral lug is

difficult to do, and affects no increase in safety or performance.

If you are powering this pig from a single-phase variac (ie - not

ganged), then it is imperative that the neutral pig lug NOT be connected

to line neutral.

In theory, in a balanced system, the neutral lug on your pig will be at

exactly the same voltage as the neutral on the power line, and threfore

a conductor connected from those points will pass 0 current and may be

removed completely w/o affecting anything.  But, in reality, unless your

limiters are exactly the same (same V vs I curve), this can't be

considered a balanced system.  I would leave the neutral off to prevent

one limiter from having to take more of the load than it's neighbor.

Since each of your MOTs or neons (hmm - never heard of using a

shorted-out neon to limit current - but I bet it works just great) are

designed to limit current to a particular value at 120 volts, using two

of them in series (as you have indicated) will still limit the current

to that exact same numeric value.  So for example, if each limiter

limits the current to, say, 10 amps, then using two in series would

limit the current to 10 amps.  The max pig power would be 240 x 10 =

2400 watts.

You service panel is capable of providing a maximum current of 30 amps,

by the selection of your fuses.  Thus, if you attach a load from line to

neutral (120 volt), max power is 30x120= 3600 Watts.  If you attach a

load from hot to hot (240) then you get a max of 30x240= 7200 watts.

My $0.0000002.  Comments, anyone? :)

-Bill

Date: Sat, 03 Oct 1998 16:42:20 -0700

From: Scott MacGregor <smacgregor@aasp.net>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Organization: EES,Co.

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Thanks a lot everybody!!!

X-MIME-Autoconverted: from 8bit to quoted-printable by www.aasp.net id QAA19692

Sender: tesl-err@emachine.com

Sam Barros wrote:

<snip>

>  You know Holland sucks if you want to build a TC. I paid 50 dollars for

> 650 meters of 0.6mm enamel wire, 80 dollars for 30 meters of 8mm thick

> copper tubing, some 100 dollars in materials for my coil form. Hell, a

> 30X100cm sheet of 8mm thick plywood costs 30 dollars!!!

>  Now, if you want a transformer, you can buy a 1KW MOT for 150 dollars.

> EACH!!!

>  I simply cannot afford that. This is supposed to be a hobby, not a

> rip-off. Specially if you consider the fact that I would need at least 4

> MOTs to power a coil (I have one that I got from my ex-microwave oven).

>  Than they make marijuana legal and they outlaw ALL Neon Light

> Transformers above 400W of power. I mean, COME-ON!!! Whats wrong with

> this people??!!

<snip>

>  Sam.

Sam,

It seems that Holland is more interested in "re-inventing" society than

"inventing"!!! The "greens" have an aversion to electricity. Maybe you

should move to the USA, where Tesla coils are still considered a hobby.

But hurry, since it probably won't be long till some government official

"discovers" that they are fun. Then, of course, the FUN TAX will be

imposed!!

Regards,

Scott MacGregor

X-Originating-IP: [208.16.145.130]

From: "Bill the arcstarter" <arcstarter@hotmail.com>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Pigs and other barnyard experiments.

Date: Sat, 03 Oct 1998 16:47:49 PDT

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

Coilers,

Here are some results of some pig tests I performed tonight.

First I attempted to measure the inductance of the pig primary, from the

0 to the 120 lug on the LV side.  My battery-powered LCR meter indicated

1.5 mH, which is clearly much too low.  I suppose the little meter just

doesn't put out enough juice to overcome core hysteresis in the iron.

Comments??

Next I measured the pig's no-load current by applying 120v to the 0-120

lugs (with a 65 mH limiting reactor in series to keep accidents down if

the HV side should flash over).  The no-load current was 0.69 amps,

which (assuming 90degrees phase angle) yields a calculated approx. 0.45

H of core inductance, a much more reasonable value.  Comments??

Oh, the primary's resistance is on the order of 0.04 ohms, real real tiny!

Then I tried a bit of Jacob's ladder action.  First I used my 0.065 H

limiting inductor in series with the pig's 120v input.  Note - this

0.065 H inductor is an un-shorted-out MOT primary.  The Jacob's action

was sort of erratic, but more strangely, each time I'd get an arc out

the HV side of the pig, I'd hear a "bump" coming from the variac and the

MOT. Is this normal???  I was powering the whole thing off a 10A variac,

if this makes a difference...

Finally I tried using a resistive ballast, in the form of an old toaster

oven in series with the pig.  Setting the oven on "Toast" provided the

most power to the pig :) .  The Jacob's Ladder action was much much

smoother this way.

I though most people had the best results when using inductive limiting.

This was not the case with me.  What is going on here?  Average current

using the toaster oven ballast was about 12 amps, and average current

using the MOT was something like 6-8 amps, although it swung around

wildly. In a previous experiment I'd determined that the V vs I curve of

this MOT core was rather nonlinear, ie, I increases rapidly with

increasing V.

I can only assume that once the HV arc ceased to conduct, there was

still a current flowing in the MOT (?).  This current would then

collapse, causing the "thump" noise from various components...  Any

clues??

-Bill

(chicken of my pig lest it gets my goat and I become BACON!)

Starting arcs in Cinci, OH

http://www.geocities.com/CapeCanaveral/Hangar/6160

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: "Coiler" <mycroft@access1.net>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: RE: Pigs and other barnyard experiments.

Date: Sat, 3 Oct 1998 17:47:50 -0700

X-Mailer: IPAD 2.0

Importance: Normal

X-MimeOLE: Produced By Microsoft MimeOLE V4.72.3155.0

Sender: tesl-err@emachine.com

>>

> Then I tried a bit of Jacob's ladder action.  First I used my 0.065 H

> limiting inductor in series with the pig's 120v input.  Note - this

> 0.065 H inductor is an un-shorted-out MOT primary.  The Jacob's action

> was sort of erratic, but more strangely, each time I'd get an arc out

> the HV side of the pig, I'd hear a "bump" coming from the variac and the

> MOT. Is this normal???  I was powering the whole thing off a 10A variac,

> if this makes a difference...

Bill: Unshorted MOT?  You won't be able to pass enough current that way.

I have found experimenting with my pig (just got it last week :)) that

leaving my neons unshorted gave me little or no power.  I do assume that

your MOT is current limited.

>

> Finally I tried using a resistive ballast, in the form of an old toaster

> oven in series with the pig.  Setting the oven on "Toast" provided the

> most power to the pig :) .  The Jacob's Ladder action was much much

> smoother this way.

>

> I though most people had the best results when using inductive limiting.

> This was not the case with me.  What is going on here?  Average current

> using the toaster oven ballast was about 12 amps, and average current

> using the MOT was something like 6-8 amps, although it swung around

> wildly. In a previous experiment I'd determined that the V vs I curve of

> this MOT core was rather nonlinear, ie, I increases rapidly with

> increasing V.

I am surprised you got anything.. anyway: I found that for Jacobs Ladder

stuff, the resistive ballast (I used a 1300 W space heater) worked great.

For my coil, it sucked! The voltage loss was so great I had to cut my gap

down to less than 1/2 of what is is with the 12KV neons. This is with a

14.4 KV pig.  With the inductive ballast (those same neons :) ) I get enough

voltage out of the pig to make it useful.

Your 'thumping' I think is a surge.. actually the *release* of the current

allowing the windings to relax.  I think this is why some people put low

value resestors (0.4 ohm) in series with the system.

And that fear is a good thing. I wired up a deadman switch, with a

30A DPDT relay feeding the pig. If I'm not pushing the button, the

pig is totally disconnected.

Michael Baumann

Coiler, Homebrewer, Nerd. mycroft@access1.net

From: McFalc0n@aol.com

Date: Sat, 3 Oct 1998 21:04:35 EDT

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: If you want to get shocked...

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

If anyone really wants to get shocked *safely* by a tesla coil, go to a store

and buy a plasma sphere. They are really solid state Tesla coils.  I've never

heard of anyone getting hurt by one, except for RF burns i got while making

the sparks fly outside of the glass of the one I have.  They didn't hurt very

much, until I used my toroid to do it.

Just put any metal object ontop of the glass and hold your finger close to

it, but don't hold it close to it for a very long time.  If that doesn't

please your desire, take the glass sphere off.  I did this once.  I got half

inch sparks from the wire terminal.

If you do not want to risk RF burns, Use a peice of metal, quarter, can,...

I've never gotten burnes from holding metal in my hand.

The safest way of all is to put the top back on and touch the glass.  all of

these work.

Just be carefull.  Hope it helps you

X-Originating-IP: [208.16.145.180]

From: "Bill the arcstarter" <arcstarter@hotmail.com>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: RE: Pigs and other barnyard experiments.

Date: Sat, 03 Oct 1998 19:25:51 PDT

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

Coilers,

Michael wrote:

>Bill: Unshorted MOT?  You won't be able to pass enough current that way.

>I have found experimenting with my pig (just got it last week :)) that

>leaveing my neons unshorted gave me little or no power.  I do assume that

>your MOT is current limited.

Yes, unshorted.  I found that this unshorted MOT would still pull ~6

amps at 120 volts.  I also found that if I short this MOT it draws

something like 40 amps or so - more than enough to pop my 20A fuse in

about 2 seconds!

Perhaps mine aren't current limited?  They have shunts.  ??  How many

amps will your shorted MOTs draw on the primary?

>I am surprised you got anything.. anyway: I found that for Jacobs Ladder

>stuff, the resistive ballast (I used a 1300 W space heater) worked great.

>For my coil, it sucked! The voltage loss was so great I had to cut my gap

OK - so you are telling me that things are different when using it to

power a coil.  I wouldn't have guessed this.  Any clues why?

I just bought 6 x 1000 watt stove heaters from MECI, for $9 total.  They

also had 37 amps corcom line filters for the low low price of $4.  Can't

beat it!

>Your 'thumping' I think is a surge.. actually the *release* of the current

>allowing the windings to relax.  I think this is why some people put low

>value resestors (0.4 ohm) in series with the system.

Could be.  The stick welder also "bumps" when you aren't doing it right! :)

>And that fear is a good thing. I wired up a deadman switch, with a

>30A DPDT relay feeding the pig. If I'm not pushing the button, the

>pig is totally disconnected.

Yes, we have a similar device rigged up for the other (Paul's) pig.

But, I'm afraid that if it tags you, you will be dead in less than the

relay release time! :( I am also loathe to depend on it, in case the

contacts weld down, as is the most frequent failure mode...

I got ahold of some nice 3ph contactors when the HVAC guys revamped our

air handler (asbestos distributor) at work.  They are rated:

FLA: 75  LRA: 450   NON-IND: 93 amps

I think I get the 93-amp non-inductive rating, but what are the others?

I also snagged some sort of overload detectors.  This is a small box

with three steel loops on the top.  Each of the three power lines from

the contactors was routed through one loop.  It is labeled: "Three Phase

Solid State Overload, Pilot Duty".  Looks like a current monitor, anyone

know more about it?  The make is "Furnas" (just like FURNACE only

different!).  There are two electrical terminals labeled NC, and C.  No

other connections.  Just curious as I've not seen something like this

before.  It wasn't wired in series with the contactor's coils, but the

rest of the harness was cut off... :(  These might be useful for

automatically cutting power should some nasty fault occur?

-Bill

X-Originating-IP: [195.121.16.45]

From: "Sam Barros" <sambarros@hotmail.com>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Important questions...

Date: Sun, 04 Oct 1998 04:53:34 PDT

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

Ok, here are the questions again:

First of all: Is Enamel Varnish as good as Polyurethane varnish for

insulating HV? How do the two compare?

Second: Could I make a PVC pipe capacitor, or a Plexiglas sheet

capacitor that would compare in performance to those PE caps you guys

use? Polyethylene is non-existent in here But I can get some nice

sheets of Plexiglas for 7 dollars each. They are squared, with some 30 X

30cm and about 0.5mm thick. I figured out that several of them

alternating with aluminium sheeting would make an easy capacitor. It

will cost a lot, but I was wondering if it would be any good How about

PVC capacitors? Too lossy I suppose

I remembered what was the third question: I need a very good

High-Voltage RF ground for some dangerous HV experiments (and than later

for my Tesla Coil).

I am thinking of burying a 30 X 30 sheet of brass some 30cm deep into

the ground, putting salt on top of it and covering it with earth. Then I

was going to hammer 3 1m long, 1cm diameter brass pipes into some damp

ground, 30cm away from each other. And, finally, I wanted to put this

metal sheet into this little water channel that runs behind my house and

connect it all together with thick cables. The water channel is quite

long and about 2 metres wide. Would it electrocute the ducks and fry my

neighbours water-pumps if I did this?

Thanks a lot!

X-Mailer: IPAD 2.0

Date: Sun, 04 Oct 1998 07:53:31 -0500

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: Important questions...

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

At 04:53 AM 10/4/98 PDT, you wrote:

> Ok, here are the questions again:

>

> First of all: Is Enamel Varnish as good as Polyurethane varnish for

>insulating HV? How do the two compare?

Probably pretty close but I would favor polyurethane.

I used spray clear enamel on a 3 X 16 inch secondary

a few years ago and while it seemed to work OK I just

wasnt able to build up the same thickness I had with brush

on polyurethane.

>

> Second: Could I make a PVC pipe capacitor, or a Plexiglas sheet

>capacitor that would compare in performance to those PE caps you guys use?

No. PVC is very very lossy at RF frequencies. Same goes

for plexiglass. It would make for a very poor performing

capacitor.

>Polyethylene is non-existent in here But I can get some nice

>sheets of Plexiglas for 7 dollars each. They are squared, with some 30 X

>30cm and about 0.5mm thick. I figured out that several of them

>alternating with aluminium sheeting would make an easy capacitor. It

>will cost a lot, but I was wondering if it would be any good How about

>PVC capacitors? Too lossy I suppose

How about polystyrene? While it does have a bit more loss

than polyethylene or polypropylene it will still make a

very good capacitor. It will have to be a flat plate cap

as its too stiff to roll.

>

> I remembered what was the third question: I need a very good

>High-Voltage RF ground for some dangerous HV experiments (and than later

>for my Tesla Coil).

> I am thinking of burying a 30 X 30 sheet of brass some 30cm deep into

>the ground, putting salt on top of it and covering it with earth. Than I

>was going to hammer 3 1m long, 1cm diameter brass pipes into some damp

>ground, 30cm away from each other. And, finally, I wanted to put this

>metal sheet into this little water channel that runs behind my house and

>connect it all together with thick cables. The water channel is quite

>long and about 2 metres wide. Would it electrocute the ducks and fry my

>neighbours water-pumps if I did this?

This should be fine and I dont think the ducks will mind.

If this is a constantly wet area then you would not need the

buried sheet or the salt. Just several paralleled ground

rods would be more than sufficient.

>P.S. Whats the Tesla Coil chat channel again?

The Tesla coil IRC discussion channel is held on the

XNET IRC network. XNET has servers in the UK, Denmark,

South Africa, Australia as well as several in the USA.

Check out my web page at: http://www.misslink.net/mhammer/irc.htm

for additional information.

For some advice on setting up the IRC client program mIRC

to access XNET check out Erik Schulz's page at:

http://members.aol.com/tesla00/mirc.htm

Anyone is welcome to join. We normally meet each evening

between 8-10 PM CDT and usually after 11PM CDT. Occasionally

other times as well.

TTYL!

Mike Hammer

mhammer@misslink.net

(editors note. The #tesla_coil IRC discussion channel is still in operation

and information can still be found at the above addresses.)

Date: Sun, 04 Oct 1998 08:11:56 -0500

From: Bert Hickman <bert.hickman@aquila.com>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Organization: Stoneridge Engineering

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Important questions...

Sender: tesl-err@emachine.com

Sam Barros wrote:

>

>  Ok, here are the questions again:

>

>  First of all: Is Enamel Varnish as good as Polyurethane varnish for

> insulating HV? How do the two compare?

Depends on the varnish! For example, there are varnishes (Glyptal) that

are used in the electrical industry for insulating dry transformers,

motors, and high voltage equipment which are superior to polyurethane.

If you have a nearby motor rewinding shop, you may be able to talk them

into selling a small amount to you (you want the air-dry type, not the

type that requires baking. Also, the material usually comes in 5 gallon

batches). Coilers have also successfully used 2-part clear epoxy resin

(Behr Build50 is one type in the USA), shellac, or other clear

solvent-based sealers. As long as you use a non-water-based clear

sealer, you should be OK. Avoid clear lacquer, however, as the acetone

solvent may tend to attack the insulation on your magnet wire!

>

>  Second: Could I make a PVC pipe capacitor, or a Plexiglas sheet

> capacitor that would compare in performance to those PE caps you guys

> use? Polyethylene is non-existent in here But I can get some nice

> sheets of Plexiglas for 7 dollars each. They are squared, with some 30 X

> 30cm and about 0.5mm thick. I figured out that several of them

> alternating with aluminium sheeting would make an easy capacitor. It

> will cost a lot, but I was wondering if it would be any good How about

> PVC capacitors? Too lossy I suppose

>

The poorer material would be PVC, but Plexiglas is not a lot better. In

terms of loss, Plexiglas is about 275 times as lossy as Low Density

PolyEthylene (LDPE), while PVC is about 1500 times as lossy! You might

want to look for Polypropylene sheet protectors (at an office supply

store). What material is used for drop cloths in Holland when you do

painting? Over here, the clear film material is LDPE in a variety of

thicknesses up to 6 mils. What is used for clear plastic lunch bags or

plastic bags in general? Most over here are LDPE.

>  I remembered what was the third question: I need a very good

> High-Voltage RF ground for some dangerous HV experiments (and than later

> for my Tesla Coil).

>  I am thinking of burring a 30 X 30 sheet of brass some 30cm deed into

> the ground, putting salt on top of it and covering it with earth. Than I

> was going to hammer 3 1m long, 1cm diameter brass pipes into some damp

> ground, 30cm away from each other. And, finally, I wanted to put this

> metal sheet into this little water channel that runs behind my house and

> connect it all together with thick cables. The water channel is quite

> long and about 2 metres wide. Would it electrocute the ducks and fry my

> neighbours water-pumps if I did this?

>

Using multiple grounding rods distributed in a fairly wide area in damp

ground will provide a better ground than a few closely-spaced

groundrods. The use of salt will also aid conductivity by increasing the

availabilty of conductive ions... however, it may also kill any

vegetation in the immediate area. The effectiveness of your grounding

system will be a function of the peak current you intend to sink - for

example, a high power transmitting facility or an electric utility may

require a very extensive grounding system with thousands of square feet

of buried copper mesh and groundrods to handle the current. And a

similar approach may be required for lightning protection systems.

However, if you are simply doing high voltage work at relatively low

current (1A or less), your proposed grounding system should be adequate.

If necessary, you could inject a low-voltage AC current into the

grounding system to verify that it can handle the desired current

without developing excessive localized voltages. Just out of curiosity,

what are the dangerous high voltage experiments that you intend to

pursue?

-- Bert --

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: R.E. Contactor

X-Mailer: IPAD 2.0

From: break7@juno.com (Kyle A. Break)

Date: Sun, 04 Oct 1998 10:13:01 EDT

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Bill, Sounds you've got a motor starter. RLA = running load amps

LRA = locked rotor amps. Does it have "heaters"on it?  They are set with

the running load of the motor and can be changed to different values. If

to much current is drawn for too long they heat up and trip a resetable

breaker. Is there a button that says RESET on it? I would just by-pass

them if you can.

The overload detector sounds like a good snag C= common NC =

normaly closed . You'll have to test to see what the cut out rate is.

Maybe this is used instead of the old-fashioned heater arrangement?

If it's not dangerous it's not fun!  Kyle  break7@juno.com

Date: Sun, 04 Oct 1998 08:23:44 -0700

From: Gene Waldo <gwaldo@1stnetusa.com>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Thanks a lot everybody!!!

X-MIME-Autoconverted: from 8bit to quoted-printable by 1stnetusa.com id JAA19365

Sender: tesl-err@emachine.com

Scott MacGregor wrote:

>

> Sam Barros wrote:

> <snip>

> >  You know Holland sucks if you want to build a TC.  This is supposed to be a

> > hobby, not a rip-off.  I mean, COME-ON!!! Whats wrong with this, people??!!

> <snip>

> >  Sam.

> Sam,

> It seems that Holland is more interested in "re-inventing" society than

> "inventing"!!! The "greens" have an aversion to electricity. Maybe you

> should move to the USA, where Tesla coils are still considered a hobby.

> But hurry, since it probably won't be long till some government official

> "discovers" that they are fun. Then, of course, the FUN TAX will be

> imposed!!

>

> Regards,

>

> Scott MacGregor

Sam,

Scott has it about nailed!  If we allow the current sitting "president"

to slink from occupancy, the next one will be so green that electricity

may be banned! Better to let him turn slowly in the wind for the next 2

yrs, and hope that that the time can be used to devise alternate

candidates who will even admit that there is a U.S. Constitution!

Given the costs for Tesla components in Holland, are there lessons for

coilers in the U.S?  Could OSHA get involved in hobbies? Could societies

(eg.: an off-shoot of HCI: TCCI (Tesla Coil Control Inc.)) form up, to

protect the terminally ignorant?  Can one immagine Wire mfrs being sued

for injuries* caused by electrical devices?

*Or interference on some old lady's reception of reruns of 'My Little

Margie'?

Dan put it well: "Evolution stops when stupidity is no longer fatal.**"

**He gave no attribution, so he gets the blame!

--

Gene Waldo -  gwaldo@1stnetusa.com  - "Igor in Residence"

---------------------------------------------------------------

What's good for the planet is more OZONE! See the machinery form up!

NEW PAGE!   http://members.1stnetusa.com/danmurphy/waldo.htm

From: NebStrebor@aol.com

Date: Sun, 4 Oct 1998 11:46:17 EDT

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Plasma Balls........

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Does anyone have any ideas on making a plasma ball? all it is is a small tesla

coil right? where would you get a transformer that small?

X-Originating-IP: [195.121.16.172]

From: "Sam Barros" <sambarros@hotmail.com>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Important questions...

Date: Sun, 04 Oct 1998 09:06:57 PDT

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

Thanks for replying!!!

>Depends on the varnish! For example, there are varnishes (Glyptal) that

>are used in the electrical industry for insulating dry transformers,

>motors, and high voltage equipment which are superior to polyurethane.

>If you have a nearby motor rewinding shop, you may be able to talk them

>into selling a small amount to you (you want the air-dry type, not the

>type that requires baking. Also, the material usually comes in 5 gallon

>batches). Coilers have also successfully used 2-part clear epoxy resin

>(Behr Build50 is one type in the USA), shellac, or other clear

>solvent-based sealers. As long as you use a non-water-based clear

>sealer, you should be OK. Avoid clear lacquer, however, as the acetone

>solvent may tend to attack the insulation on your magnet wire!

Well, this is how I did my secondary:

I washed the pipe troughly, dried it with a heat gun for about an hour,

and varnished it inside with one coat of epoxy, 2coats of polyurethane.

Than outside. One epoxy, 3 polyurethane.

Than I wound 1000 turns of enamel coated wire, and varnished: one PU

horizontal, 1PU vertical, 1Epoxy horizontal, 1 Epoxy vertical, 6PU, 1

Enamel, 1PU and finally, one Epoxy.

I think epoxy makes the best insulation material: It insulates

excellently, stands more heat than any other insulator I've tried (you

can solder around it!) and, best of all: Is very, very tough, but yet

not brittle.

My coil has some 1mm of insulation. I've tested it's insulation with a

60KV generator and it didn't punch trough (!).

Epoxing the form wasn't easy though... I had to thin the epoxy with

paint thinner and alcohol until it was the right thickness. Than I had

10 minutes to use it before it dried... It made very thick coats

though...

I used 2Plexiglass sheets epoxied together on top and bottom to seal

the coil...And I have a thinner pipe that holds a screw-on toroid. I can

rise or lower it just by turning. The secondary also screws into the

primary. I can take the whole coil apart in just a few seconds!!! Looks

great Let's just see how it performs (when I get a transformer)...

>The poorer material would be PVC, but Plexiglas is not a lot better. In

>terms of loss, Plexiglas is about 275 times as lossy as Low Density

>PolyEthylene (LDPE), while PVC is about 1500 times as lossy! You might

>want to look for Polypropylene sheet protectors (at an office supply

>store). What material is used for drop cloths in Holland when you do

>painting? Over here, the clear film material is LDPE in a variety of

>thicknesses up to 6 mils. What is used for clear plastic lunch bags or

>plastic bags in general? Most over here are LDPE.

Maybe I have seen polypropylene an didn't recognise it... What does it

look like? A clear flexible plastic I suppose... Any tests for it?

>Using multiple grounding rods distributed in a fairly wide area in damp

>ground will provide a better ground than a few closely-spaced

>groundrods. The use of salt will also aid conductivity by increasing the

>availabilty of conductive ions... however, it may also kill any

>vegetation in the immediate area. The effectiveness of your grounding

>system will be a function of the peak current you intend to sink - for

>example, a high power transmitting facility or an electric utility may

>require a very extensive grounding system with thousands of square feet

>of buried copper mesh and groundrods to handle the current. And a

>similar approach may be required for lightning protection systems.

>

>However, if you are simply doing high voltage work at relatively low

>current (1A or less), your proposed grounding system should be adequate.

>If necessary, you could inject a low-voltage AC current into the

>grounding system to verify that it can handle the desired current

>without developing excessive localized voltages. Just out of curiosity,

>what are the dangerous high voltage experiments that you intend to

>pursue?

>

>-- Bert --

3KV 4uF Capacitor discharge inside a 1000W microwave oven to create

plasma. Than 60KV 1500W to trap the plasma on a cage. I did it today

during the morning.

The results were rather disappointing: First the circuit breaker kept

on tripping (too much power being drawn). So I get an extension from

another room. Than the EMP from the large capacitor discharge shut the

oven down. Than I tried using a smaller capacitor (an 8inch diameter PVC

pipe with aluminium tape inside and out), running at 25KV. The wires

inside the oven were connected to the capacitor. The capacitor than

started working as an antenna and broadcasting the microwaves My

microwave leakage detector went crazy. The power being radiated was

off-scale!

I feel a terrible headache from that, and my eyes are kind of sore...

Hope I'll be OK...

Well, time for another aspirin:-(

Sam Barros.

P.S. On the other hand, it did make plasma.:-) I wouldn't recommend

anyone to stick wires inside their ovens though...

X-Mailer: IPAD 2.0

Date: Sun, 04 Oct 1998 11:07:35 -0500

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: Plasma Balls........

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

At 11:46 AM 10/4/98 EDT, you wrote:

>Does anyone have any ideas on making a plasma ball?

This usually requires a means of obtaining high vacuum.

You can get some plasma from argon or helium at or near

one ATM but a vacuum source is still required to remove

the air before backfilling with your gas.

A clear round globe type light bulb makes a ready

made plasma globe. The 5 inch diameter Sylvania

bulbs make very good plasma streams just sitting

on top of the toroid on my coil

>all it is is a small tesla coil right?

Pretty much.

>where would you get a transformer that small?

Flyback transformers are commonly used. Automotive

ignition coils can generate some plasma but are not

as suitable due to the low frequency.

TTYL!

Mike Hammer

mhammer@misslink.net

X-Originating-IP: [195.121.16.172]

From: "Sam Barros" <sambarros@hotmail.com>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Important questions...

Date: Sun, 04 Oct 1998 09:18:08 PDT

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

Thanks for replying, Mike!

>Date: Sun, 04 Oct 1998 07:53:31 -0500

>To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

>From: Mike Hammer <mhammer@misslink.net>

>Subject: Re: Important questions...

>Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

>

>> First of all: Is Enamel Varnish as good as Polyurethane varnish for

>>insulating HV? How do the two compare?

>

>Probably pretty close but I would favor polyurethane.

>I used spray clear enamel on a 3 X 16 inch secondary

>a few years ago and while it seemed to work OK I just

>wasnt able to build up the thickness I was with brush

>on polyurethane.

I used epoxy to build up 1mm thickness of insulation on my coil. I

tested it to 60KV and it didn't punch trough...

>No. PVC is very very lossy at RF frequencies. Same goes

>for plexiglass. It would make for a very poor performing

>capacitor.

Worst than a salt water filled beer bottle!?!?!? I don't want to use

glass bottles for my coil. They suck! the first one I built exploded

after about 2 weeks. It seems that all the aluminium foil I put inside

it reacted with the salt water to give off some gas. The gas created

pressure and boom. I wasn't even using the damn thing!

>How about polystyrene? While it does have a bit more loss

>than polyethylene or polypropylene it will still make a

>very good capacitor. It will have to be a flat plate cap

>as its too stiff to roll.

You mean expanded polystyrene (its the only one I know off)? The white

foamy stuff? Doesn't it have too many pores on it?

>This should be fine and I dont think the ducks will mind.

>If this is a constantly wet area then you would not need the

>buried sheet or the salt. Just several paralleled ground

>rods would be more than sufficient.

I hope so... Those green lovers would arrest me if I fried the ducks

with high voltage... Personally, I think it would make a nice video:-}

Sam Barros

X-Originating-IP: [195.121.16.172]

From: "Sam Barros" <sambarros@hotmail.com>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: A word of caution..

Date: Sun, 04 Oct 1998 09:41:38 PDT

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

>Flyback transformers are commonly used. Automotive

>ignition coils can generate some plasma but are not

>as suitable due to the low frequency.

>

>TTYL!

>

>Mike Hammer

>mhammer@misslink.net

For all of you who go for that "Always keep one hand in your pocket"

and "Wear good rubber soled shoes" crap:

Read this:

I connected an ordinary 60Watts light bulb to my Ignition coil driver

and turned it on. Since I was only using one coil, the output came in

the form of pulsed HV at about 30KV.

The bulb made quite a nice plasma globe. I was wearing rubber-soled

shoes and standing on a rubber mat, that was lying on top of my wooden

floor.

When I touched the bulb, the thin, almost invisible plasma treads

became quite bright and made some considerable noise, and I felt quite a

bit of pain on my hand and arm. I was also getting some spasms in my

arm. That I decided to test myself and see if, despite the pain and the

spasms, I could pick up the light bulb (you know, that macho crap).

Well, so I put my hand (ONE HAND ONLY!!!) around the bulb and, when I

start to lift it, a spark jumps from the bulb's metal base and hits me

in the finger. All my muscles contracted and I was thrown back. It hurt

like hell and I had trouble breathing for a few seconds.

I was quite aware of the danger involved and I knew that a spark from

that device (which plugs straight into the 220V, 50Hz mains and has only

one resistor to limit the power going to the coil) could be quite

hazardous. Specially because it discharges 300V, 3uF capacitors straight

onto the coil's primary... Oh well, shit happens...

Well, the moral of this message is even with one hand on your pocket

and rubber soled shoes, you will still get electrocuted. Any Ideas why?

I think streamers leaving my body provided a current flow... Again I

remind you that I was not in any way grounded.

Sam Barros.

From: McFalc0n@aol.com

Date: Sun, 4 Oct 1998 12:50:23 EDT

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Plasma Balls........

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Look in your library.  I have a book that has instructions. It is called

Gadgeteers Goldmine.  it also has Tesla instructions, too.  All a plasma spere

is is a small (around 18 watt) solid state tesla coil.  The transformer is a

flyback transformer.  All you need to know is in that book.  Good luck.

X-Mailer: IPAD 2.0

Date: Sun, 04 Oct 1998 11:50:30 -0500

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: Important questions...

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

At 09:18 AM 10/4/98 PDT, you wrote:

> Thanks for replying, Mike!

>

>

>>Date: Sun, 04 Oct 1998 07:53:31 -0500

>>To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

>>From: Mike Hammer <mhammer@misslink.net>

>>Subject: Re: Important questions...

>>Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

>>

>

>>> First of all: Is Enamel Varnish as good as Polyurethane varnish for

>>>insulating HV? How do the two compare?

>>

>>Probably pretty close but I would favor polyurethane.

>>I used spray clear enamel on a 3 X 16 inch secondary

>>a few years ago and while it seemed to work OK I just

>>wasnt able to build up the thickness I was with brush

>>on polyurethane.

>

>

> I used epoxy to build up 1mm thickness of insulation on my coil. I

>tested it to 60KV and it didn't punch trough...

Epoxy is one of the best coatings but far too expensive

for me. I can coat a typical 4 inch diameter secondary with

1/2 pint of varnish and it only costs a couple dollars.

>

>

>

>>No. PVC is very very lossy at RF frequencies. Same goes

>>for plexiglass. It would make for a very poor performing

>>capacitor.

>

>

> Worst than a salt water filled beer bottle!?!?!?

PVC would be nearly as bad as glass. Plexiglass would

be marginally better but still poor. Also forget

mylar. It has a very high RF dissipation factor too.

>

>

>>How about polystyrene? While it does have a bit more loss

>>than polyethylene or polypropylene it will still make a

>>very good capacitor. It will have to be a flat plate cap

>>as its too stiff to roll.

>

>

> You mean expanded polystyrene (its the only one I know off)? The white

>foamy stuff? Doesn't it have too many pores on it?

No. Thats polystyrene foam. I meant solid polystyrene sheeting.

I made some small plate caps using 30 mil thick polystyrene

sheets. They worked quite well but polyethylene is much cheaper.

Try some of those polypropylene sheet protectors. They are meant

for protecting sheets of paper and putting them into a 3 ring

binder. Try an office supply store.

TTYL!

Mike Hammer

mhammer@misslink.net

From:          "Malcolm Watts" <MALCOLM@directorate.wnp.ac.nz>

Organization:  Wellington Polytechnic, NZ

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Date:          Mon, 5 Oct 1998 10:34:45 +1200

Subject:       Re: TC and magnifier comparison

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Hi Marco,

> Knowing nothing (yet) about magnifiers I was wondering this:

>

> 1.For the same tank supply (i.e. same kVA and kVs) do I get better performance

> (i.e. longer streamers) if I build a conventional TC or if I build a

> magnifier?

Not in my experience.

> 2.What are the differences in performance (other than streamer length) between

> a TC and a magnifier?

Very little

> 3.Why one would build a magnifier rather than a TC?

It enables you to keep the discharge point well away from the primary

Malcolm

From:          "Malcolm Watts" <MALCOLM@directorate.wnp.ac.nz>

Organization:  Wellington Polytechnic, NZ

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Date:          Mon, 5 Oct 1998 10:42:29 +1200

Subject:       RE: Pigs and other barnyard experiments.

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Re the "bumping" transformer:

> Send reply to:  "Tesla-2 Mailing List" <tesla-2@emachine.com>

> From:           "Coiler" <mycroft@access1.net>

> To:             "Tesla-2 Mailing List" <tesla-2@emachine.com>

> Subject:        RE: Pigs and other barnyard experiments.

> Date sent:      Sat, 3 Oct 1998 17:47:50 -0700

>

> >>

> > Then I tried a bit of Jacob's ladder action.  First I used my 0.065 H

> > limiting inductor in series with the pig's 120v input.  Note - this

> > 0.065 H inductor is an un-shorted-out MOT primary.  The Jacob's action

> > was sort of erratic, but more strangely, each time I'd get an arc out

> > the HV side of the pig, I'd hear a "bump" coming from the variac and the

> > MOT. Is this normal???  I was powering the whole thing off a 10A variac,

> > if this makes a difference...

> Bill: Unshorted MOT?  You won't be able to pass enough current that way.

> I have found experimenting with my pig (just got it last week :)) that

> leaveing my neons unshorted gave me little or no power.  I do assume that

> your MOT is current limited.

>

> >

> > Finally I tried using a resistive ballast, in the form of an old toaster

> > oven in series with the pig.  Setting the oven on "Toast" provided the

> > most power to the pig :) .  The Jacob's Ladder action was much much

> > smoother this way.

> >

> > I though most people had the best results when using inductive limiting.

> > This was not the case with me.  What is going on here?  Average current

> > using the toaster oven ballast was about 12 amps, and average current

> > using the MOT was something like 6-8 amps, although it swung around

> > wildly. In a previous experiment I'd determined that the V vs I curve of

> > this MOT core was rather nonlinear, ie, I increases rapidly with

> > increasing V.

> I am surprised you got anything.. anyway: I found that for Jacobs Ladder

> stuff, the resistive ballast (I used a 1300 W space heater) worked great.

> For my coil, it sucked! The voltage loss was so great I had to cut my gap

> down to less than 1/2 of what is is with the 12KV neons. This is with a

> 14.4 KV pig.  With the inductive ballast (those same neons :) ) I get enough

> voltage out of the pig to make it useful.

>

> Your 'thumping' I think is a surge.. actually the *release* of the current

> allowing the windings to relax.  I think this is why some people put low

> value resestors (0.4 ohm) in series with the system.

It is probably that the combination of inductances is jumping into

mains resonance with the capacitor. Adding resistance kills the Q of

this circuit somewhat. The result is excessive line current which

increases core magnetostriction by bringing the core close if not

into saturation.

Malcolm

X-Originating-IP: [216.68.40.227]

From: "Bill the arcstarter" <arcstarter@hotmail.com>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: RE: Pigs and other barnyard experiments.

Date: Sun, 04 Oct 1998 17:48:10 PDT

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

Malcolm wrote:

>Re the "bumping" transformer:

<snip>

>> Your 'thumping' I think is a surge.. actually the *release* of the current

>> allowing the windings to relax.  I think this is why some people put low

>> value resestors (0.4 ohm) in series with the system.

>

>It is probably that the combination of inductances is jumping into

>mains resonance with the capacitor. Adding resistance kills the Q of

>this circuit somewhat. The result is excessive line current which

>increases core magnetostriction by bringing the core close if not

>into saturation.

Q?  Who said anything about a capacitor?  This was just a pig and a

MOT...

-Bill

X-Originating-IP: [216.68.40.227]

From: "Bill the arcstarter" <arcstarter@hotmail.com>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: R.E. Contactor

Date: Sun, 04 Oct 1998 18:04:07 PDT

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

Hi Kyle, list,

You wrote:

>	Bill, Sounds you've got a motor starter. RLA = running load amps

> LRA = locked rotor amps. Does it have "heaters"on it?

Like the ones used on magnetic motor starters?  I'm not too familiar

with those types.

My overload units have no reset button, or any sort of access panel

where you'd be changing any components (heaters?).  Let me see if I can

open one (snapped together, supposedly non-disassemblable)

>the running load of the motor and can be changed to different values. If

>to much current is drawn for too long they heat up and trip a resetable

>breaker. Is there a button that says RESET on it? I would just by-pass

>them if you can.

>	The overload detector sounds like a good snag C= common NC =

>normaly closed . You'll have to test to see what the cut out rate is.

>Maybe this is used instead of the old-fashioned heater arrangement?

Don't know.  It had a hand-written cutout current of 80 amps on the

label.  How would I use this thing to meter single phase current?  Loop

the wire through all three sensing loops?  I'll have to fool with  the

thing (send it some controlled currents via the welder) to see what it

does...

The contactor is about 4x4x8 inches (!).  This overload device is 4x4x2

inches, and is very light.

Thanks for your comments on this junk!

-Bill

X-Originating-IP: [216.68.40.227]

From: "Bill the arcstarter" <arcstarter@hotmail.com>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: RE: Pigs and other barnyard experiments.

Date: Sun, 04 Oct 1998 18:14:13 PDT

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

Hi Malcolm,

You wrote:

>Bill,

>       My apologies. I thought I read something about a cap in there.

>I obviously didn't :(  Well, it must be magnetostriction alone.

Hmm.  But - you are still right (I think?) about adding series

resistance to eliminate this "bumping".  At least, that's what I

remember reading somewhere...

At any rate - all of this is MOOT since the whole goal is to power the

coil off of it, and not just the Jacob's! :)  Plus - have fun!

-Bill

From:          "Malcolm Watts" <MALCOLM@directorate.wnp.ac.nz>

Organization:  Wellington Polytechnic, NZ

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Date:          Mon, 5 Oct 1998 13:56:21 +1200

Subject:       RE: Pigs and other barnyard experiments.

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Bill,

My apologies. I thought I read something about a cap in there.

I obviously didn't :(  Well, it must be magnetostriction alone.

Malcolm

> From:           "Bill the arcstarter" <arcstarter@hotmail.com>

> To:             "Tesla-2 Mailing List" <tesla-2@emachine.com>

> Subject:        RE: Pigs and other barnyard experiments.

> Date sent:      Sun, 04 Oct 1998 17:48:10 PDT

> Send reply to:  "Tesla-2 Mailing List" <tesla-2@emachine.com>

> Malcolm wrote:

>

> >Re the "bumping" transformer:

> <snip>

> >> Your 'thumping' I think is a surge.. actually the *release* of the

> current

> >> allowing the windings to relax.  I think this is why some people put

> low

> >> value resestors (0.4 ohm) in series with the system.

> >

> >It is probably that the combination of inductances is jumping into

> >mains resonance with the capacitor. Adding resistance kills the Q of

> >this circuit somewhat. The result is excessive line current which

> >increases core magnetostriction by bringing the core close if not

> >into saturation.

>

> Q?  Who said anything about a capacitor?  This was just a pig and a

> MOT...

>

> -Bill

>

X-Originating-IP: [207.100.20.56]

From: "Philip Mac Duffie" <punk_42@hotmail.com>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Plasma Ball

Date: Sun, 04 Oct 1998 19:25:38 PDT

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

Hi, check out my page Http://hv.hypermart.net/pb.html

it has a bunch of info on plasma balls and if you have any questions

e-mail me..

Philip Mac Duffie

Date: Sun, 04 Oct 1998 21:49:34 -0500

From: Bert Hickman <bert.hickman@aquila.com>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Organization: Stoneridge Engineering

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Important questions...

Sender: tesl-err@emachine.com

Sam Barros wrote:

>

>   Thanks for replying!!!

<SNIP>

>   Well, this is how I did my secondary:

>  I washed the pipe troughly, dried it with a heat gun for about an hour,

> and varnished it inside with one coat of epoxy, 2coats of polyurethane.

>  Than outside. One epoxy, 3 polyurethane.

>  Than I wound 1000 turns of enamel coated wire, and varnished: one PU

> horizontal, 1PU vertical, 1Epoxy horizontal, 1 Epoxy vertical, 6PU, 1

> Enamel, 1PU and finally, one Epoxy.

>  I think epoxy makes the best insulation material: It insulates

> excellently, stands more heat than any other insulator I've tried (you

> can solder around it!) and, best of all: Is very, very tough, but yet

> not brittle.

>  My coil has some 1mm of insulation. I've tested it's insulation with a

> 60KV generator and it didn't punch trough (!).

<SNIP>

Sounds like it should be a very nice secondary!

<SNIP>

>  Maybe I have seen polypropylene an didn't recognise it... What does it

> look like? A clear flexible plastic I suppose... Any tests for it?

LDPE is typically clear, grading to milky in thicker sheets. PP is also

clear, also becoming milky when in thicker sheets, and it tends to be

"stiffer" than LDPE. Both polypropylene and polyethylene are lighter

than water - they will both float. If you carefully heat a sample piece

with a match, polyethylene will burn with a blue flame with a yellow tip

and an odor of paraffin (candle wax). Polypropylene will burn in a

similar manner, but will have a sweet odor. Touching a piece with a hot

soldering iron should also give you a paraffin or sweet odor. BTW, be

very careful when doing any "sniff test" since some plastics (PVC or

even flame-retardant polypropylene), can give off acrid or even toxic

vapors.

<SNIP>

> >what are the dangerous high voltage experiments that you intend to

> >pursue?

> >

>  3KV 4uF Capacitor discharge inside a 1000W microwave oven to create

> plasma. Than 60KV 1500W to trap the plasma on a cage. I did it today

> during the morning.

>  The results were rather disappointing: First the circuit breaker kept

> on tripping (too much power being drawn). So I get an extension from

> another room. Than the EMP from the large capacitor discharge shut the

> oven down. Than I tried using a smaller capacitor (an 8inch diameter PVC

> pipe with aluminium tape inside and out), running at 25KV. The wires

> inside the oven were connected to the capacitor. The capacitor than

> started working as an antenna and broadcasting the microwaves My

> microwave leakage detector went crazy. The power being radiated was

> off-scale!

>  I feel a terrible headache from that, and my eyes are kind of sore...

> Hope I'll be OK...

>

>  Well, time for another aspirin:-(

>

>  Sam Barros.

>

>  P.S. On the other hand, it did make plasma.:-) I wouldn't recommend

> anyone to stick wires inside their ovens though...

>

Sam, please be extremely careful working with microwave radiation! The

radiated energy can very quickly cause blindness by creating cataracts

in your eyes (sort of like cooking an egg)! High power microwave

radiation can induce other undesirable physiological and mental effects.

The particular frequency used in ovens was chosen to maximize the

heating effect on water molecules... the type that make up over 75% of

your body and brain. Don't inadvertantly cook them...

-- Bert --

From: Hollmike@aol.com

Date: Sun, 4 Oct 1998 23:41:55 EDT

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: ballast

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

In a message dated 10/3/98 12:50:33 PM Mountain Daylight Time,

bkibler@quicktel.com writes:

>  But I would like to get bigger.  At what point in this "growing up" does

>  one start needing ballast?

Robert,

Ballast is needed for transformers that have no current limiting shunts.

These would include pole pigs and some MOT's.  Since a NST is current limiting

by design, you do not need such a ballast.  NST's have magnetic shunts that

take care of that for you.  As long as you have some protection such as a

safety gap, and some RF chokes(or some other type of circuit protection) on

the output of your tranny, you should be fine in applying a higher voltage to

your tranny.    I use two simple air core chokes and a safety gap on mine and

it has never complained :o).

Mike Hollingsworth

From: Hollmike@aol.com

Date: Sun, 4 Oct 1998 23:44:29 EDT

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: ballast

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

In a message dated 10/3/98 1:32:22 PM Mountain Daylight Time,

bert.hickman@aquila.com writes:

>  > Right now my coil is running off a 5 amp variac to a

>  > 12 kv 60 ma neon trans.  So far, I have been using it conservatively, with

>  > less than 30 volts coming out of the variac

I accidentally sent that last reply without realizing one thing:  If you go

higher on the voltage with your system, you will need a larger variac to

handle the load.  You will need an 8 amp one.

Happy coiling,

Mike Hollingsworth

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: "Coiler" <mycroft@access1.net>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Advice sought for Safety Gaps

Date: Sun, 4 Oct 1998 21:22:06 -0700

X-Mailer: IPAD 2.0

Importance: Normal

X-MimeOLE: Produced By Microsoft MimeOLE V4.72.3155.0

Sender: tesl-err@emachine.com

Gentlepersons:

I have been trying out my new power transformer, and I think I am running

into some glitches that may be due to my moving from 12KV neons to a

14.4KV pig.  To wit:

1) The gap across my cap is firing erratically, but *way* too much.

Currently this gap is set at 0.375". Am I correct in assuming I really need

to expand this gap? And if so, what is safe for my caps as constructed?

[ 3 caps in series, each consisting of 30mil LDPE dialectric, for a total

of 90mil paralleled with one that is about 72 mil in 2]

2) My safety gap across the pig needs to be amost 1/2" to keep from firing

unloaded, does this sound right for 14.4?? (seems large to me) This is

even with using a Variac to ramp up. [ I am using shorted neons as

inductive ballast until I can build something better.] This gap is

truely across the pig, and not to ground, this is the correct fashion

for a non-centertapped transformer, yes?

3) My main gap is currently at 0.156, I intend to take it to .208 once I

figure out what is going on with the Cap Gap. [ yes I am still using

my RQ style static gap at the moment. Assuming the nameplate on the

trannys are accurate, I am passing around 1100 VA to the pig, and it

is working just fine.]

4) I am using the same chokes and resistors from my NST days still, one on

each leg, but have dropped the bypass cap. Is this a good/bad idea?

All in all, aside from the scare I am getting from capactitor gap, I am very

pleased with the performance since switching to the pig. Even when I was only

running 720VA (I just used the same 12KV/30 paralleled  neons that used to

power the coil) I am getting arcs in the 48" range. The garage is getting too

small, but fortunately, the neighbors think this thing is pretty cool. And

I figure my ground and filters are working well enough, as they are not

complaining about snow on the TV :)

Oh, and I learned one other thing.. don't try and use a high power factor

neon as a current limiter.. it won't pass much.

Thanks for indulging me whilst I pick your brains.. I understood things

(I thought) but this higher power range is making life interesting.

Michael Baumann

Coiler, Homebrewer, Nerd. mycroft@access1.net

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Bert Hickman

X-Mailer: IPAD 2.0

From: break7@juno.com (Kyle A. Break)

Date: Mon, 05 Oct 1998 01:17:41 EDT

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Bert, read about your microwave oven experiments. Several years

ago there was an artical in the Times about a new light bulb. The bulb is

a small quartz sphere with a tiny amount of sulfur and an unidentified

inert gas inside. The bulb is placed in a tube that has a magnetron at

one end. The light output is supposed to be greater than anything

currently available. Have you heard about this?

If it's not dangerous it's not fun!  Kyle  break7@juno.com

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Date: Mon, 05 Oct 1998 04:13:32 -0700

From: "paul m" <fleischwulf@mailexcite.com>

Cc:

X-Sent-Mail: off

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

Subject: Re: A word of caution..

X-Sender-Ip: 208.153.7.9

Organization: MailExcite  (http://www.mailexcite.com:80)

Sender: tesl-err@emachine.com

Sam,

I'm not sure as to why you got shocked but I have gotten supprised by my tank circuit

a couple of times. It seems that the caps in the tank still held a charge after the coil was turned off, it wasn't much but it was one hell of a suprise.

Like you I had one hand in the pocket an thick rubber soles. Since the caps have

broken in I haven't seen any more residual charges. I now discharge all caps before changing taps.

paul m

---

Einigheit und Wahrheit

On Sun, 04 Oct 1998 09:41:38   Sam Barros wrote:

>>Flyback transformers are commonly used. Automotive

>>ignition coils can generate some plasma but are not

>>as suitable due to the low frequency.

>>

>>TTYL!

>>

>>Mike Hammer

>>mhammer@misslink.net

>

>

>  For all of you who go for that "Always keep one hand in your pocket"

>and "Wear good rubber soled shoes" crap:

> Read this:

> I connected an ordinary 60Watts light bulb to my Ignition coil driver

>and turned it on. Since I was only using one coil, the output came in

>the form of pulsed HV at about 30KV.

> The bulb made quite a nice plasma globe. I was wearing rubber-soled

>shoes and standing on a rubber mat, that was lying on top of my wooden

>floor.

> When I touched the bulb, the thin, almost invisible plasma treads

>became quite bright and made some considerable noise, and I felt quite a

>bit of pain on my hand and arm. I was also getting some spasms in my

>arm. That I decided to test myself and see if, despite the pain and the

>spasms, I could pick up the light bulb (you know, that macho crap).

> Well, so I put my hand (ONE HAND ONLY!!!) around the bulb and, when I

>start to lift it, a spark jumps from the bulb's metal base and hits me

>in the finger. All my muscles contracted and I was thrown back. It hurt

>like hell and I had trouble breathing for a few seconds.

> I was quite aware of the danger involved and I knew that a spark from

>that device (which plugs straight into the 220V, 50Hz mains and has only

>one resistor to limit the power going to the coil) could be quite

>hazardous. Specially because it discharges 300V, 3uF capacitors straight

>onto the coil's primary... Oh well, shit happens...

> Well, the moral of this message is even with one hand on your pocket

>and rubber soled shoes, you will still get electrocuted. Any Ideas why?

>I think streamers leaving my body provided a current flow... Again I

>remind you that I was not in any way grounded.

>

> Sam Barros.

Date: Mon, 05 Oct 1998 06:47:48 -0500

From: Bert Hickman <bert.hickman@aquila.com>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Organization: Stoneridge Engineering

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Sulfur uWave lamp

Sender: tesl-err@emachine.com

Kyle,

The microwave experiments were not mine - I confine my microwave

experiments to being completely in the oven with no wires coming out. I

was commenting on the danger of working with high power microwaves to

another poster. In the early days of uWave and RADAR, people used to

stand in the beams to keep warm - at least until they began getting

internally cooked... :^)

I also remember reading about the high intensity sulfur lamp when it was

first announced. I really haven't heard very much more about the

technology since. The lamp was originally developed by Fusion Lighting

in Rockville, Maryland. The noble gas was apparently argon, and the gas

was used to absorb the uWave radiation and transfer energy to form

diatomic (S2) sulfur, which then emits broad spectrum visible light with

little undesirable UV or IR. Later evolutions of the technology

permitted excitation via lower frequency RF instead of uWaves.

However, in doing a web search, I did find a site which has a bit more

information, and at least one outfit is attempting to commercialize the

technology. Check out:

http://www.egc.com/mwave.htm

or the reference sites shown at this location:

http://eande.lbl.gov/CBS/NEWSLETTER/NL6/S-Lamp.html

OR

http://www.webcom.com/%7elightsrc/archive/article3.html

Thanks for the reminder about these very interesting devices!

And, Safe coilin' to you!

-- Bert --

Kyle A. Break wrote:

>

>         Bert, read about your microwave oven expierements. Several years

> ago there was an artical in the Times about a new light bulb. The bulb is

> a small quartz sphere with a tiny amount of sulfur and an un identified

> inert gas inside. The bulb is placed in a tube that has a magnetron at

> one end. The light output is supposed to be greater than anything

> currently avail. Have you heard about this?

>

>         If it's not dangerous it's not fun!  Kyle  break7@juno.com

>

Date: Mon, 05 Oct 1998 06:59:25 -0500

From: Bert Hickman <bert.hickman@aquila.com>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Organization: Stoneridge Engineering

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Advice sought for Safety Gaps

Sender: tesl-err@emachine.com

Michael,

Safety-gap firing typically occurs when the coil is out of tune or if

you're seeing resonant charging. Try adjusting the tuning a bit to see

if this improves the situation at all. A symptom for the latter is that

you suddenly will get sporadic main gap firing at relatively low variac

settings (say 15 - 20% of max). The cure for this is adding or removing

ballast inductance so that you get out of the 60 Hz resonant range.

You may also be seeing some ringing of the chokes. Since the pig does

not need to have the same protection network that the NST required, try

removing the chokes, and instead only use the resistor-capacitor

combination.

Safe coilin' to you!

-- Bert --

Coiler wrote:

>

> Gentlepersons:

> I have been trying out my new power transformer, and I think I am running

> into some glitches that may be due to my moving from 12KV neons to a

> 14.4KV pig.  To wit:

>

> 1) The gap across my cap is firing erratically, but *way* too much.

> Currently this gap is set at 0.375". Am I correct in assuming I really need

> to expand this gap? And if so, what is safe for my caps as constructed?

> [ 3 caps in series, each consisting of 30mil LDPE dialectric, for a total

>   of 90mil paralleled with one that is about 72 mil in 2]

>

> 2) My safety gap across the pig needs to be amost 1/2" to keep from firing

>    unloaded, does this sound right for 14.4?? (seems large to me) This is

>    even with using a Variac to ramp up. [ I am using shorted neons as

>    inductive ballast until I can build something better.] This gap is

>    truely across the pig, and not to ground, this is the correct fashion

>    for a non-centertapped transformer, yes?

>

> 3) My main gap is currently at 0.156, I intend to take it to .208 once I

>    figure out what is going on with the Cap Gap. [ yes I am still using

>    my RQ style static gap at the moment. Assuming the nameplate on the

>    trannys are accurate, I am passing around 1100 VA to the pig, and it

>    is working just fine.]

>

> 4) I am using the same chokes and resistors from my NST days still, one on

>    each leg, but have dropped the bypass cap. Is this a good/bad idea?

>

> All in all, aside from the scare I am getting from capactitor gap, I am very

> pleased with the performance since switching to the pig. even when I was only

> running 720VA (I just used the same 12KV/30 paralleled  neons that used to

> power the coil) I am getting arcs in the 48" range. The garage is getting too

> small, but fortunately, the neighbors think this thing is pretty cool. And

> I figure my ground and filters are working well enough, as they are not

> complaining about snow on the TV :)

>

> Oh, and I learned one other thing.. don't try and use a high power factor

> neon as a current limiter.. it won't pass much.

>

> Thanks for indulging me whilst I pick your brains.. I understood things

> (I thought) but this higher power range is making life interesting.

>

> Michael Baumann

> Coiler, Homebrewer, Nerd. mycroft@access1.net

From: "Basura, Brian" <brian.basura@unistudios.com>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: RE: Plastic Sources....

Date: Mon, 5 Oct 1998 09:13:22 -0700

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Bob,

I used masonite(sp?) which I sanded, sealed, and painted.  Thanks for the

advice regarding the poly.

Regards,

Brian D. Basura

> ----------

> From: 	bob golding[SMTP:yubba@clara.net]

> Reply To: 	Tesla-2 Mailing List

> Sent: 	October 02, 1998 2:50 PM

> To: 	Tesla-2 Mailing List

> Subject: 	Re: Plastic Sources....

>

> At 10:25 02/10/98 -0700, you wrote:

>

> >

> >I've been eyeing their "Lay Flat 6mil Poly tubing".  It looks like it would

> >be great for stacked plate caps and would eliminate a lot of the cutting

> >involved.

>

> 	Hi Brian,

> 	I thought of using this and found that most of the cheaper poly used for

> packing  purposes all had small air bubbles in. Get them to send you a

> sample. It would be a real shame to use inferior materials on such a good

> looking coil. i am green with envy. What did you use for the two round

> platforms on your coil?

>

> bob golding

>

>

> bob yubba@clara.net

>

From: "Basura, Brian" <brian.basura@unistudios.com>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: RE: Pigs and other barnyard experiments.

Date: Mon, 5 Oct 1998 10:16:59 -0700

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Hi,

I'm surprised that you get any current through your MOT without the

secondary shorted.  Mine pull almost 0A with an open secondary.

My first setup used one MOT with shorted secondary (11.7mH) and 6.0 ohms in

series to limit current to my PT. This limited the current to a smooth 9.5A

at 100V. Changing R to any other value seemed to reduce the benefit of using

the inductor and it took much higher input (variac) voltage to fire my coil.

This was not what I expected to happen.

My current setup (which is far from optimized) uses two parallel MOT's

(11.7mH,10.5mH) with shorted secondaries and 6 ohms resistance in parallel

with them.  Any less R and my safety gaps (capacitor and PT) fire at the

same time as my main gap.  Input current is fluctuating wildly around 15A at

110v.  I think my next step will be to place a low R (.4 ohms) in series

with my MOT's and see if that smoothes things out.

PS. Any input on Safety gap settings.  My PT is set to .490 and my

capacitors are set to .320 (utilizing 1/4" stove bolt heads as the safety

gap).

Regards,

Brian D. Basura

> ----------

> From: 	Coiler[SMTP:mycroft@access1.net]

> Reply To: 	Tesla-2 Mailing List

> Sent: 	October 03, 1998 5:47 PM

> To: 	Tesla-2 Mailing List

> Subject: 	RE: Pigs and other barnyard experiments.

>

>

> >>

> > Then I tried a bit of Jacob's ladder action.  First I used my 0.065 H

> > limiting inductor in series with the pig's 120v input.  Note - this

> > 0.065 H inductor is an un-shorted-out MOT primary.  The Jacob's action

> > was sort of erratic, but more strangely, each time I'd get an arc out

> > the HV side of the pig, I'd hear a "bump" coming from the variac and the

> > MOT. Is this normal???  I was powering the whole thing off a 10A variac,

> > if this makes a difference...

> Bill: Unshorted MOT?  You won't be able to pass enough current that way.

> I have found experimenting with my pig (just got it last week :)) that

> leaveing my neons unshorted gave me little or no power.  I do assume that

> your MOT is current limited.

>

> >

> > Finally I tried using a resistive ballast, in the form of an old toaster

> > oven in series with the pig.  Setting the oven on "Toast" provided the

> > most power to the pig :) .  The Jacob's Ladder action was much much

> > smoother this way.

> >

> > I though most people had the best results when using inductive limiting.

> > This was not the case with me.  What is going on here?  Average current

> > using the toaster oven ballast was about 12 amps, and average current

> > using the MOT was something like 6-8 amps, although it swung around

> > wildly. In a previous experiment I'd determined that the V vs I curve of

> > this MOT core was rather nonlinear, ie, I increases rapidly with

> > increasing V.

> I am surprised you got anything.. anyway: I found that for Jacobs Ladder

> stuff, the resistive ballast (I used a 1300 W space heater) worked great.

> For my coil, it sucked! The voltage loss was so great I had to cut my gap

> down to less than 1/2 of what is is with the 12KV neons. This is with a

> 14.4 KV pig.  With the inductive ballast (those same neons :) ) I get

> enough

> voltage out of the pig to make it useful.

>

> Your 'thumping' I think is a surge.. actually the *release* of the current

> allowing the windings to relax.  I think this is why some people put low

> value resestors (0.4 ohm) in series with the system.

>

> And that fear is a good thing. I wired up a deadman switch, with a

> 30A DPDT relay feeding the pig. If I'm not pushing the button, the

> pig is totally disconnected.

>

>

> Michael Baumann

> Coiler, Homebrewer, Nerd. mycroft@access1.net

>

From: DavidAllan@aol.com

Date: Mon, 5 Oct 1998 13:29:30 EDT

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Secondary Insulation, and good display of streamers

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

I am following the thread about secondary insulation. My question: how thick

does the insulation need to be? I have sprayed about a dozen coats of

polyurethane on my 4 inch secondary (wound with 22 ga magnet wire, unknown

insulation)? I can certainly still feel each wire. Do I need to put a layer so

thick that the wires are embedded? If what I've done to this point is

adequate, I'd like to stop here so I can get on with the other components I

need. I will be powering this coil with a 10 kV/23 mA OBIT. I may use a pair

of OBITs in parallel in the future.

I hope to finish this coil before Halloween so I can add it to my outdoor "mad

scientist" display. The coil itself would be under the overhang of my porch.

I'd run it manually (of course), and I'd like to send streamers to  some kind

of grounded strike points. I was thinking of taking some hardware cloth (like

stiff chicken wire) and making a few targets attached to my (to be built) rf

ground. The target could be cut to present a number of pointed ends to the

coil. What do you think? Is this already well-discussed in the archive?

This list is really great. The archive is very usable and complete, and the

experienced (and learning) coil builders seem *very* willing to share. Many

thanks to all participants.

David Allan

From: Roy Minnich <rminnich@rolling-meadows.lib.il.us>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: RE: Plastic Sources....

Date: Mon, 5 Oct 1998 12:43:44 -0500

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

Hi all,

Wouldn't phenolic tubing be good for the secondary coil forms? Is this stuff hydroscopic?  Please tell me again why we seal the ends of the tubing.

Thanks,

Roy Minnich

X-Mailer: IPAD 2.0

Date: Mon, 05 Oct 1998 14:14:20 -0500

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: Secondary Insulation, and good display of streamers

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

At 01:29 PM 10/5/98 EDT, you wrote:

>I am following the thread about secondary insulation. My question: how thick

>does the insulation need to be? I have sprayed about a dozen coats of

>polyurethane on my 4 inch secondary (wound with 22 ga magnet wire, unknown

>insulation)? I can certainly still feel each wire. Do I need to put a layer so

>thick that the wires are embedded? If what I've done to this point is

>adequate, I'd like to stop here so I can get on with the other components I

>need. I will be powering this coil with a 10 kV/23 mA OBIT. I may use a pair

>of OBITs in parallel in the future.

David,

Ive always advocated a couple of coats on the form BEFORE the wire

is added then at least 6 coats on top of the wire. This serves to space

the winding a small distance away from the form and completely imbeds

the winding. I keep adding varnish until the wire can no longer be felt

and the coating is perfectly smooth.

I never had good luck with spray on coatings. Takes too long to build

up a thick enough layer. With brush on polyurethane it only takes 6

medium coats. It only takes 1/2 pint of brush on varnish for a typical

4 inch secondary.

>

>I hope to finish this coil before Halloween so I can add it to my outdoor "mad

>scientist" display. The coil itself would be under the overhang of my porch.

>I'd run it manually (of course), and I'd like to send streamers to some kind

>of grounded strike points. I was thinking of taking some hardware cloth (like

>stiff chicken wire) and making a few targets attached to my (to be built) rf

>ground. The target could be cut to present a number of pointed ends to the

>coil. What do you think? Is this already well-discussed in the archive?

Sounds good to me. I have had good luck placing foil bumps on the outer edge

of my toroid to gain some measure of directional control. Check the pics

on my web page and yu will note all the streamers go to one side only.

>

>This list is really great. The archive is very usable and complete, and the

>experienced (and learning) coil builders seem *very* willing to share. Many

>thanks to all participants.

Thanks for the comment on the archives. I put a lot of effort into

those archives. They are highly edited and reformatted to make them easier

and more enjoyable to read. I delete the off topic posts and edit out

over long signatures as well as overquoting. I try to correct spelling

and grammar errors when I find them.

Many of the posts on the list are formatted very poorly. It would make

things much easier if everyone would limit line lengths to less than 70

characters and use hard carriage returns instead of using local line wrapping.

It may look OK on your machine but its really bad on the receiving end.

Microsoft email products are especially bad as are the many web based

email services.

TTYL!

Mike Hammer

mhammer@misslink.net

X-Mailer: IPAD 2.0

Date: Mon, 05 Oct 1998 14:20:51 -0500

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Mike Hammer <mhammer@misslink.net>

Subject: RE: Plastic Sources....

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

At 12:43 PM 10/5/98 -0500, you wrote:

>Hi all,

>Wouldn't phenolic tubing be good for the secondary coil forms?

Should be. Ive used phenolic for coil forms for ham radio

projects before with good success.

> Is this stuff hydroscopic?

Dont think so. A couple coats of varnish would take care of that.

>Tell me again why we seal the ends of the form.

PVC pipe is porous and the pores will retain moisture.

Once you bake the pipe a little to drive off the moisture

it needs to be sealed to prevent reabsorption.

I do not seal the bottom with disks of plexiglass as

some advocate. I leave the bottom open and instead

coat the inside of the form with varnish.

I do add disks of plexiglass in the top end of the tube

to prevent any arcing down the inside of the form.

One arc down the inside will leave a carbon track

and render the secondary useless.

TTYL!

Mike Hammer

mhammer@misslink.net

X-Originating-IP: [199.217.140.22]

From: "Bill the arcstarter" <arcstarter@hotmail.com>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: RE: Pigs and other barnyard experiments.

Date: Mon, 05 Oct 1998 12:45:33 PDT

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

Coilers,

Brian wrote:

>I'm surprised that you get any current through your MOT without the

>secondary shorted.  Mine pull almost 0A with an open secondary.

This is very different from how mine works.  Mine draws about 6 amps at

120 volts with the secondary open.  WIth it closed - it draws 30-40 amps

- pops the 20 amp fuse in about 1-2 seconds...

<snip>

You also describe using MOTs with parallel resistance (6 ohms) as a

limiting device.  How many people out there place the resistance in

SERIES with their MOT limiters vs in parallel?? I wonder if one way is

universally better than the other...?

-Bill the arcstarter

Starting arcs in Cinci, OH

http://www.geocities.com/CapeCanaveral/Hangar/6160

X-Originating-IP: [199.217.140.22]

From: "Bill the arcstarter" <arcstarter@hotmail.com>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: RE: Pigs and other barnyard experiments.

Date: Mon, 05 Oct 1998 12:45:42 PDT

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

Coilers,

Brian wrote:

>I'm surprised that you get any current through your MOT without the

>secondary shorted.  Mine pull almost 0A with an open secondary.

This is very different from how mine works.  Mine draws about 6 amps at

120 volts with the secondary open.  WIth it closed - it draws 30-40 amps

- pops the 20 amp fuse in about 1-2 seconds...

<snip>

You also describe using MOTs with parallel resistance (6 ohms) as a

limiting device.  How many people out there place the resistance in

SERIES with their MOT limiters vs in parallel?? I wonder if one way is

universally better than the other...?

-Bill the arcstarter

Starting arcs in Cinci, OH

http://www.geocities.com/CapeCanaveral/Hangar/6160

Posted-Date: Mon, 5 Oct 1998 22:13:26 +0200

From: "Peter Staal" <drtesla@casema.net>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject:

Date: Mon, 5 Oct 1998 22:12:47 +0200

X-Mailer: IPAD 2.0

X-MimeOLE: Produced By Microsoft MimeOLE V4.72.2106.4

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Who has or knows of a Tesla Coil in holland?

Because I want to see one for real, in holland, because I live there.

Organization:  Wellington Polytechnic, NZ

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Date:          Tue, 6 Oct 1998 09:08:09 +1200

Subject:       Re: A word of caution..

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

> Sam,

>  I'm not sure as to why you got shocked but I have gotten supprised by my tank

>circuit a couple of times. It seems that the caps in the tank still held a charge

>after the coil was turned off, it was

>you I had one hand in the pocket an thick rubber soles. Since the caps have broken

>in I haven't seen any more residual charges. I now discharge all caps before changing

>taps.

>

> paul m

> ---

> Einigheit und Wahrheit

It is worth remembering you are always capacitively coupled to ground.

Malcolm

From: "Basura, Brian" <brian.basura@unistudios.com>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: RE: A word of caution..

Date: Mon, 5 Oct 1998 14:09:09 -0700

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

My new toroid (6"x29" Al drier ducting) holds a static charge after a run.

It was quite a surprise to get a  discharge to my forehead while changing

the tap (Main caps were grounded as always before working on the primary).

Now I discharge the static on the toroid also.

Brian D. Basura

> ----------

> From: 	Malcolm Watts[SMTP:MALCOLM@directorate.wnp.ac.nz]

> Reply To: 	Tesla-2 Mailing List

> Sent: 	October 05, 1998 2:08 PM

> To: 	Tesla-2 Mailing List

> Subject: 	Re: A word of caution..

>

> > Sam,

> >  I'm not sure as to why you got shocked but I have gotten supprised by

> my tank circuit a couple of times. It seems that the caps in the tank

> still held a charge after the coil was turned off, it was

>

> >  you I had one hand in the pocket an thick rubber soles. Since the caps

> have broken in I haven't seen any more residual charges. I now discharge

> all caps before changing taps.

> >

> > paul m

> > ---

> > Einigheit und Wahrheit

>

> It is worth remembering you are always capacitively coupled to ground.

>

> Malcolm

>

From: "Basura, Brian" <brian.basura@unistudios.com>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: RE: Pigs and other barnyard experiments.

Date: Mon, 5 Oct 1998 14:15:40 -0700

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Bill,

I'll try and verify my previous statement tonight.  I do know the inductance

with an open secondary is approximately 23mH.  As far a series vs. parallel

resistance goes series seems to make more sense and the bigger coils are set

up that way.  Series resistance is the approach I'll be trying as soon as I

get enough R to equal .3-.5 ohms.

Brian D. Basura

> ----------

> From: 	Bill the arcstarter[SMTP:arcstarter@hotmail.com]

> Reply To: 	Tesla-2 Mailing List

> Sent: 	October 05, 1998 12:45 PM

> To: 	Tesla-2 Mailing List

> Subject: 	RE: Pigs and other barnyard experiments.

>

> Coilers,

>  Brian wrote:

> >I'm surprised that you get any current through your MOT without the

> >secondary shorted.  Mine pull almost 0A with an open secondary.

>

> This is very different from how mine works.  Mine draws about 6 amps at

> 120 volts with the secondary open.  WIth it closed - it draws 30-40 amps

> - pops the 20 amp fuse in about 1-2 seconds...

>

> <snip>

>

> You also describe using MOTs with parallel resistance (6 ohms) as a

> limiting device.  How many people out there place the resistance in

> SERIES with their MOT limiters vs in parallel?? I wonder if one way is

> universally better than the other...?

>

> -Bill the arcstarter

> Starting arcs in Cinci, OH

> http://www.geocities.com/CapeCanaveral/Hangar/6160

>

>

> ______________________________________________________

> Get Your Private, Free Email at http://www.hotmail.com

>

From: wwl@netcomuk.co.uk (Mike Harrison)

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Sulfur uWave lamp

Date: Mon, 05 Oct 1998 21:42:14 GMT

Organization: White Wing Logic

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

>I also remember reading about the high intensity sulfur lamp when it was

>first announced. I really haven't heard very much more about the

>technology since. The lamp was originally developed by Fusion Lighting

>in Rockville, Maryland. The noble gas was apparently argon, and the gas

>was used to absorb the uWave radiation and transfer energy to form

>diatomic (S2) sulfur, which then emits broad spectrum visible light with

>little undesirable UV or IR. Later evolutions of the technology

>permitted excitation via lower frequency RF instead of uWaves.

>

>However, in doing a web search, I did find a site which has a bit more

>information, and at least one outfit is attempting to commercialize the

>technology. Check out:

>	http://www.egc.com/mwave.htm

>or the reference sites shown at this location:

>	http://eande.lbl.gov/CBS/NEWSLETTER/NL6/S-Lamp.html

>OR

>	http://www.webcom.com/%7elightsrc/archive/article3.html

>

>Thanks for the reminder about these very interesting devices!

>>

>>         Bert, read about your microwave oven expierements. Several years

>> ago there was an artical in the Times about a new light bulb. The bulb is

>> a small quartz sphere with a tiny amount of sulfur and an un identified

>> inert gas inside. The bulb is placed in a tube that has a magnetron at

>> one end. The light output is supposed to be greater than anything

>> currently avail. Have you heard about this?

>>

>>         If it's not dangerous it's not fun!  Kyle  break7@juno.com

>>

Anyone tried to do a homemade sulphur lamp in a microwave oven - seems

all the ingredients are easily available - welding argon (would

atmospheric pressure do ?). Might be fun even if it only lasted a few

seconds....? Any ideas anyone?

From:          "Malcolm Watts" <MALCOLM@directorate.wnp.ac.nz>

Organization:  Wellington Polytechnic, NZ

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Date:          Tue, 6 Oct 1998 10:32:53 +1200

Subject:       RE: A word of caution..

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Hi Brian,

> From:           "Basura, Brian" <brian.basura@unistudios.com>

> To:             "Tesla-2 Mailing List" <tesla-2@emachine.com>

> Subject:        RE: A word of caution..

> Date sent:      Mon, 5 Oct 1998 14:09:09 -0700

> Send reply to:  "Tesla-2 Mailing List" <tesla-2@emachine.com>

> My new toroid (6"x29" Al drier ducting) holds a static charge after a run.

> It was quite a surprise to get a  discharge to my forehead while changing

> the tap (Main caps were grounded as always before working on the primary).

> Now I discharge the static on the toroid also.

>

> Brian D. Basura

There is something very wrong there. Remember that the toroid should

be grounded through your secondary coil. You might have picked up

some static by touching the secondary coil insulation? That can build

up a considerable charge on a dry day.

Malcolm

From: McFalc0n@aol.com

Date: Mon, 5 Oct 1998 18:39:41 EDT

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Magnet wire

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

What's the best magnet wire?  Best insulation.  I've been looking at 24 gauge

Formvar, and at prices around $80 for 1,100 ft.  Does anyone now of any good

sources around Houston, Texas?  Thanx.

From: Dsurfr@aol.com

Date: Mon, 5 Oct 1998 20:26:04 EDT

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Alternate primary coil materials

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Hi All: I want to try winding a bunch of different primary coils w/ various

shapes, spacings etc to see which ends up performing best w/ my coil set up.

1/4" soft copper tubing is one of the hardest things I have ever worked with &

doesn't seem reusable from one coil to another. #10 solid copper wire also

seems to get "work hardened" and kinked & I want to be able to reuse the

material to keep the costs down. I wound a couple of flat primarys from

coaxial cable (& had good? results) but everyone says it is too lossy & I'm

afraid it will alter the validity of these tests as a result. I have a bunch

of 1/4" aircraft cable that coils very easily but it is stranded & probably

very lossy as well. Would it be usable or does anyone have any other

suggestions? I intend to keep records of the various types & hope to post the

results. (looking for something to keep me busy while I save up to make a poly

cap)  Thanks,  Jim

Date: Mon, 05 Oct 1998 19:32:36 -0500

From: Bert Hickman <bert.hickman@aquila.com>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Organization: Stoneridge Engineering

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Magnet wire

Sender: tesl-err@emachine.com

McFalc0n@aol.com wrote:

>

>         What's the best magnet wire?  Best insulation.  i've been looking at 24 gauge

> Formvar, and at prices around $80 for 1,100 ft.  Does anyone now of any good

> sources around Houston, Texas?  Thanx.

WOW!!

Don't buy it for $80!!

1,100 feet of #24 AWG is less than 1.5 pounds. You should be able to

find Formvar, Thermaleze, Polythermaleze, Imideze, or similar insulated

wire for $5-$8/lb max. Check with local motor rewinding outfits. Stay

away from those that can be easily soldered through the insulation

(Soldereze, for example).

If you have no luck there, try Joe Cassata at the Coax Connection:

The Coax Connection

Joe Cassata

10 S 226 Meadow Lane

Naperville,Il 60564

630-420-0342

Joe's a great guy, and he LOVES Tesla coilers!

-- Bert --

X-Mailer: IPAD 2.0

Date: Mon, 05 Oct 1998 19:41:20 -0500

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: Magnet wire

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

At 06:39 PM 10/5/98 EDT, you wrote:

>What's the best magnet wire?  Best insulation.

Best is rather subjective. Depends on the use.

For Tesla secondaries ordinary formvar is just fine.

Polythermalize provides somewhat better insulation

but its probably overkill for most coils

>  i've been looking at 24 gauge Formvar, and at prices

>around $80 for 1,100 ft.

$80 for 1100 feet? No way! If thats what they

are getting for it Im in the wrong business.

Magnet wire is sold by the pound. AWG 24 is around

800 feet per pound. So thats only a little under

1.5 pounds. The price for magnet wire varies from

$4 - 8 per pound.

I think you need a new source.

> Does anyone now of any good sources around Houston, Texas?

> Thanx.

>

Cant help you with Houston but there is a source in Illinois

that will sell by mail.

The Coax Connection

Joe Cassata

10 S 226 Meadow Lane

Naperville,Il 60564

630-420-0342

These prices are off his price sheet I got from him

at the Princeton Il hamfest in last June.

AWG    ft/lb    $/lb

14      79.7    5.10

16     127      5.30

18     201      5.50

20     315      5.85

22     516      6.05

24     802      7.20

26    1280      8.00

28    2027      8.75

30    3212      9.25

Other sizes available.

Joe is a swell guy and loves Tesla coilers. Tell him its for a Tesla coil.

You can beat Joe's prices at a local motor repair shop but if you havnt got

one in your area this is a reputable place to do business with.

TTYL!

Mike Hammer

mhammer@misslink.net

Date: Mon, 05 Oct 1998 19:46:22 -0500

From: Bert Hickman <bert.hickman@aquila.com>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Organization: Stoneridge Engineering

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Plastic Sources....

Sender: tesl-err@emachine.com

Roy Minnich wrote:

>

> Hi all,

> Wouldn't phenolic tubing be good for the secondary coil forms?  Is this stuff hydroscopic?  Please tell me again why we seal the ends of the tubing.

>

> Thanks,

>

> Roy Minnich

>

Roy,

Phenolic tubng will make a nice coilform. However, the cloth or paper

that's used in the material may make it a bit hydroscopic, so

heat-drying and sealing is still advisable. The ends are usually sealed

to make an arc path throught the INSIDE of the coilform much less

attractive than one OUTSIDE the form. A streamer that forms inside the

form is not usually seen until it begins carbon-tracking and burning the

form...

-- Bert --

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: R.E. Mc Falc0n

X-Mailer: IPAD 2.0

From: break7@juno.com (Kyle A. Break)

Date: Mon, 05 Oct 1998 21:07:37 EDT

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

I can get a couple miles of #25 green for about $20  I'm sure you

need a lot less! If your interested I can check on it.

If it's not dangerous it's not fun!  Kyle  break7@juno.com

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Alternate primary coil materials

X-Mailer: IPAD 2.0

From: break7@juno.com (Kyle A. Break)

Date: Mon, 05 Oct 1998 21:29:26 EDT

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Hi, Jim. I would stay away from steel wire it is a very poor

conductor compared to copper. Aluminum is an excellent conductor and

could be used also. Brass is good- red brass, 95% copper yellow brass,

65% copper

Al tubing is very soft and way lighter than copper.

If it's not dangerous it's not fun!  Kyle  break7@juno.com

On Mon, 5 Oct 1998 20:26:04 EDT Dsurfr@aol.com writes:

>Hi All: I want to try winding a bunch of different primary coils w/ various

>shapes, spacings etc to see which ends up performing best w/ my coil set up.

>1/4" soft copper tubing is one of the hardest things I have ever worked with &

>doesn't seem reusable from one coil to another. #10 solid copper wire also

>seems to get "work hardened" and kinked & I want to be able to reuse the

>material to keep the costs down. I wound a couple of flat primaries from

>coaxial cable (& had good? results) but everyone says it is too lossy & I'm

>afraid it will alter the validity of these tests as a result. I have a bunch

>of 1/4" aircraft cable that coils very easily but it is stranded & probably

>very lossy as well. Would it be usable or does anyone have any other suggestions?

>

>I intend to keep records of the various types & hope to post the

>results. (looking for something to keep me busy while I save up to

>make a poly cap)  Thanks,  Jim

>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Annealing Copper Tubing

X-Mailer: IPAD 2.0

From: break7@juno.com (Kyle A. Break)

Date: Mon, 05 Oct 1998 21:45:44 EDT

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Just thought of this. Might be kinda hard to heat a long coil of

tubing with a torch. How about clipping each end to an arc - welder and

wait...till it's red?

An arc welder may have 1000 uses! I think I'll run out and try

it.

If it's not dangerous it's not fun!  Kyle  break7@juno.com

From:          "Malcolm Watts" <MALCOLM@directorate.wnp.ac.nz>

Organization:  Wellington Polytechnic, NZ

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Date:          Tue, 6 Oct 1998 13:58:21 +1200

Subject:       Re: Alternate primary coil materials

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Hi Jim,

> From:           Dsurfr@aol.com

> Date sent:      Mon, 5 Oct 1998 20:26:04 EDT

> To:             "Tesla-2 Mailing List" <tesla-2@emachine.com>

> Subject:        Alternate primary coil materials

> Send reply to:  "Tesla-2 Mailing List" <tesla-2@emachine.com>

> Hi All: I want to try winding a bunch of different primary coils w/ various

> shapes, spacings etc to see which ends up performing best w/ my coil set up.

> 1/4" soft copper tubing is one of the hardest things I have ever worked with &

> doesn't seem reusable from one coil to another. #10 solid copper wire also

> seems to get "work hardened" and kinked & I want to be able to reuse the

> material to keep the costs down.

You can anneal it. Heat it bright red with a carburizing oxy-

acetylene flame and let it cool naturally.

I'm interested to know the results of your tests. I'd be particularly

interested if one primary came up a clear winner if the coupling

constant you set each combination at is the same. It is important

when conducting a test of this type to disturb as few parameters as

possible. Someone did a test of some different primary shapes awhile

back on the other list and came to a definite conclusion but didn't

eve consider the difference in coupling constant.

> I wound a couple of flat primarys from

> coaxial cable (& had good? results) but everyone says it is too lossy & I'm

> afraid it will alter the validity of these tests as a result. I have a bunch

> of 1/4" aircraft cable that coils very easily but it is stranded & probably

> very lossy as well. Would it be usable or does anyone have any other

> suggestions? I intend to keep records of the various types & hope to post the

> results. (looking for something to keep me busy while I save up to make a poly

> cap)  Thanks,  Jim

Just about anything is better than braid. Even stranded although that

doesn't rate as highly as pipe, strap or litz wire. I think you'd be

better off simply replacing the braid primary with another of the same

type and a better conductor and definitely building a decent cap.

Malcolm

From: McFalc0n@aol.com

Date: Mon, 5 Oct 1998 22:47:36 EDT

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: R.E. Mc Falc0n

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

you're right, i only need a couple thousand feet.  Thanx anyway!

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: "Coiler" <mycroft@access1.net>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Avoiding work hardening.

Date: Mon, 5 Oct 1998 19:56:33 -0700

X-Mailer: IPAD 2.0

Importance: Normal

X-MimeOLE: Produced By Microsoft MimeOLE V4.72.3155.0

Sender: tesl-err@emachine.com

Ok gang.. just a hint from someone that has worked with copper tubing

often for its *original* purpose.

You want to avoid work hardening the copper at all costs. How to do this?

*NEVER* straighten out that coil unless you need to. And when you are

winding a primary, you never need to. For primary I just made I used

about 65' of tubing. To do this I bought 2 50' rolls. I had my pegs

already laid out (1/4" tubing, 1/4" spacing dowels work great) I had

8 spokes. I started from the center, and worked slowly out. The pipe

needed slight inward bending (for my 8.5" inner turn) and then slowly

expanded. I soldered the next piece in when I needed it. I never tried

to do more than 1/8 turn at a time, and adjusted the curve by pulling or

pushing over about 1/2 the span at a time. I've used this technique for

4 primaries now.. and never have had a problem with hardening.

Remember, (time for sexist remark that will get me in trouble) treat the

coil as you would a lovely lady.. slow and gentle handling is always better

in the long run than rough and fast.

If you *do* get it hardened.. you don't need to go to the cherry red.. you

can just use a torch and heat to the point of discoloration. It won't be pretty,

but you can try the gentle bend trick again.

Michael Baumann

Coiler, Homebrewer, Nerd. mycroft@access1.net

Date: Mon, 05 Oct 1998 21:57:59 -0500

From: Tom Polk <tpolk@prismnet.com>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Primary material

Sender: tesl-err@emachine.com

I'm curious why copper or aluminum strapping is not used more. My flat

primaries of 3/4" aluminum and 2" copper are easy to work with. I just

glue eight WalMart combs upside down and wrap the strapping in at

whatever spacing I want to try.

I have never noticed corona off the edges, and I get 48" power arcs with

the 2" copper strapping and one 15 kv NST. If I were to switch to copper

tubing would I get another few inches of spark?

BTW, early pictures of my coil are at the link below this text  I have

since gotten much hotter and longer arcs, mainly due to the big copper

strapping and a bigger toroid. I'll put better pictures on the site. Try

this link for now:

http://www.prismnet.com/~tpolk/coils/coils.html

X-Originating-IP: [208.16.145.163]

From: "Bill the arcstarter" <arcstarter@hotmail.com>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Annealing Copper Tubing

Date: Mon, 05 Oct 1998 20:19:30 PDT

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

Kyle wrote:

>	Just thought of this. Might be kinda hard to heat a long coil of

>tubing with a torch. How about clipping each end to an arc - welder and

>wait...till it's red?

>	An arc welder may have 1000 uses! I think I'll run out and try it.

Once, while re-plumbing a half-dead TIG torch, I tried an experiment

(long story deleted here...)  I passed 80+ amps through a one foot long

section of 1/8 inch copper tubing for one minute.  It didn't even get

hot (just warm)!

Conclusion: Copper is one DAMN good conductor!  To heat a length of 1/4

copper tubing would require probably 100s of amps at least...

But I did see an interesting product in the trade journals.  This was a

machine which sounded strongly like an arc welder.  It was designed for

thawing frozen copper/steel water pipes.  It had two alligator clips.

You'd clip onto the pipe on either side of the frozen section, and turn

it on.  In this manner the pipe is resistance heated w/o using a torch

and burning the building down, etc.

Of course, since it was a *useful* product they wanted some $4000 for

it, or some other highly unlikely price...

-Bill the arcstarter

Starting arcs in Cinci, OH

http://www.geocities.com/CapeCanaveral/Hangar/6160

X-Originating-IP: [208.16.145.163]

From: "Bill the arcstarter" <arcstarter@hotmail.com>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Motor overload detector

Date: Mon, 05 Oct 1998 20:40:35 PDT

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

Coilers,

I opened up that FURNAS motor overload controller...  The insides are

surprisingly complicated.  About 25 components including three

adjustment pots.

Each steel sense loop (through which the primary conductors are passed)

also goes through a bobbin of magnet wire.  These bobbins are dioded

together (sort of) and apparently power the whole circuit.

There is a COB (Chip On Board) integrated module, which apparently runs

the show.

Anyway - until I can through some amps through it, thats all I know.

I wonder if this has any tesla coil applications?

-Bill

From:          "Malcolm Watts" <MALCOLM@directorate.wnp.ac.nz>

Organization:  Wellington Polytechnic, NZ

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Date:          Tue, 6 Oct 1998 16:11:52 +1200

Subject:       Re: Avoiding work hardening.

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Hi Michael,

> If you *do* get it hardened.. you don't need to go to the cherry red.. you can

> just use a torch and heat to the point of discoloration. It won't be pretty,

> but you can try the gentle bend trick again.

>

> Michael Baumann

> Coiler, Homebrewer, Nerd. mycroft@access1.net

The first time I did this I had to anneal the stuff as I built a

primary from straight hardened tubing. I tried heating only to

discolouration but it proved to be insufficient, hence my note.

These days I buy coiled copper like you.

Malcolm

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: "Coiler" <mycroft@access1.net>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: RE: Avoiding work hardening.

Date: Mon, 5 Oct 1998 21:08:14 -0700

X-Mailer: IPAD 2.0

Importance: Normal

X-MimeOLE: Produced By Microsoft MimeOLE V4.72.3155.0

Sender: tesl-err@emachine.com

Zounds!  You were indeed a brave man. However.. mild work hardening does

not compare to rigid pipe :)

Michael Baumann

Coiler, Homebrewer, Nerd. mycroft@access1.net

> -----Original Message-----

> From: tesl-err@emachine.com [mailto:tesl-err@emachine.com]On Behalf Of

> Malcolm Watts

> Sent: Monday, October 05, 1998 9:12 PM

> To: Tesla-2 Mailing List

> Subject: Re: Avoiding work hardening.

>

>

> Hi Michael,

>

> > If you *do* get it hardened.. you don't need to go to the  cherry red.. you

> > can just use a torch and heat to the point of discoloration. It  won't be pretty,

> > but you can try the gentle bend trick again.

> >

> > Michael Baumann

> > Coiler, Homebrewer, Nerd. mycroft@access1.net

>

> The first time I did this I had to anneal the stuff as I built a

> primary from straight hardened tubing. I tried heating only to

> discolouration but it proved to be insufficient, hence my note.

> These days I buy coiled copper like you.

>

> Malcolm

>

From:          "Malcolm Watts" <MALCOLM@directorate.wnp.ac.nz>

Organization:  Wellington Polytechnic, NZ

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Date:          Tue, 6 Oct 1998 16:38:33 +1200

Subject:       Re: Primary material

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Hi Tom,

What primary capacitance are you using? Tubing won't make

any improvement over your strapping.  Nice going!

Malcolm

> Date sent:      Mon, 05 Oct 1998 21:57:59 - 0500

> From:           Tom Polk <tpolk@prismnet.com>

> Send reply to:  "Tesla-2 Mailing List" <tesla-2@emachine.com>

> To:             "Tesla-2 Mailing List" <tesla-2@emachine.com>

> Subject:        Primary material

> I'm curious why copper or aluminum strapping is not used more. My flat

> primaries of 3/4" aluminum and 2" copper are easy to work with. I just

> glue eight WalMart combs upside down and wrap the strapping in at

> whatever spacing I want to try.

>

> I have never noticed corona off the edges, and I get 48" power arcs with

> the 2" copper strapping and one 15 kv NST. If I were to switch to copper

> tubing would I get another few inches of spark?

>

> BTW, early pictures of my coil are at the link below this text  I have

> since gotten much hotter and longer arcs, mainly due to the big copper

> strapping and a bigger toroid. I'll put better pictures on the site. Try

> this link for now:

>

> http://www.prismnet.com/~tpolk/coils/coils.html

Date: Mon, 05 Oct 1998 23:51:18 -0500

From: Tom Polk <tpolk@prismnet.com>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Primary material

Sender: tesl-err@emachine.com

Malcolm Watts wrote:

>

> Hi Tom,

>           What primary capacitance are you using? Tubing won't make

> any improvement over your strapping.  Nice going!

>

> Malcolm

>

> > Date sent:      Mon, 05 Oct 1998 21:57:59 - 0500

> > From:           Tom Polk <tpolk@prismnet.com>

> > Send reply to:  "Tesla-2 Mailing List" <tesla-2@emachine.com>

> > To:             "Tesla-2 Mailing List" <tesla-2@emachine.com>

> > Subject:        Primary material

>

> > I'm curious why copper or aluminum strapping is not used more. My flat

> > primaries of 3/4" aluminum and 2" copper are easy to work with. I just

> > glue eight WalMart combs upside down and wrap the strapping in at

> > whatever spacing I want to try.

> >

> > I have never noticed corona off the edges, and I get 48" power arcs with

> > the 2" copper strapping and one 15 kv NST. If I were to switch to copper

> > tubing would I get another few inches of spark?

> >

> > BTW, early pictures of my coil are at the link below this text  I have

> > since gotten much hotter and longer arcs, mainly due to the big copper

> > strapping and a bigger toroid. I'll put better pictures on the site. Try

> > this link for now:

> >

> > http://www.prismnet.com/~tpolk/coils/coils.html

.015 uf flat plate 1/8" poly, 12 x 16 x 4 "

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Welder Softening?

X-Mailer: IPAD 2.0

From: break7@juno.com (Kyle A. Break)

Date: Tue, 06 Oct 1998 00:56:57 EDT

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Well I tried the welder on 50' of 3/8 tubing. At 300 amps I was able

to get it to about 212 degrees in two minutes. Maybe for small tubing it

would work. Can't expect too much for 6000 watts.

If it's not dangerous it's not fun!  Kyle  break7@juno.com

X-Mailer: IPAD 2.0

Date: Tue, 06 Oct 1998 09:21:16 +0200

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Marco Denicolai <marco@vistacom.fi>

Subject: Again a CAP question...

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Here is my first bunch of stupid questions about capacitors.

1.In my understanding, people design rolled caps (homemade) so that the PE or

whatever dielectric used stands a certain desired voltage. Oil is added

ONLY to prevent corona effects: correct?

2. Now, if the dielectric is correctly calculated, why are some air bubbles

between the layers so damn critical and can cause capacitor destruction?

3. I made a "dry" capacitor with 4 mm plexiglass sheet and copper (you can

see it on my Web page) for a value of 2 X 550 pF. No oil used and surely

there is air (and moisture) trapped between the layers. Been in use ALL the

time with my TC, watched it MANY times, never seen any corona effect or

blue light anywhere. It is used as a filter cap for the MOTs not to get back

the 220 KHz TC resonance frequencies. Of course you may tell me that the

electric field in it is so much lower than in a 0.01uF cap: is that the

only reason my cap is OK?

Marco Denicolai                   Vista Communication Instruments, Inc.

Hardware Development Manager      www.vistacom.fi

marco@vistacom.fi

From: Hollmike@aol.com

Date: Tue, 6 Oct 1998 07:02:57 EDT

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Secondary Insulation, and good display of streamers

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

David,

I have a coil that I used a sprayed on insulating varnish made by Krylon.

I put on about six or eight coat(don't remember now) and still can feel the

windings, but have had no problems.  Be sure that you tune your coil before

applying full power to prevent high voltage nodes from appearing at points

below the top of the coil and you should have no trouble.  I have seen

mistuned coils arcing from one point of a secondary to another point a few

inches away,  and no amount of insulation would prevent this until the coil is

properly tuned.

Hope this helps.

Mike Hollingsworth

Date: Tue, 06 Oct 1998 06:44:42 -0500

From: Bert Hickman <bert.hickman@aquila.com>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Organization: Stoneridge Engineering

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Again a CAP question...

Sender: tesl-err@emachine.com

Marco Denicolai wrote:

>

> Here is my first bunch of stupid questions about capacitors.

>

> 1.In my understanding, people design rolled caps (homemade) so that the PE or

> whatever dielectric used stands a certain desired voltage. Oil is added

> ONLY to prevent corona effects: correct?

>

Oil also fills up any "gaps" between the metal plates and the

dielectric, and between dielectric sheets. These gaps would otherwise be

full of air (k=1), considerably reducing the capacitance of the

home-made cap.

> 2. Now, if the dielectric is correctly calculated, why are air bubbles

> between the layers are so damn critical and can cause capacitor destruction?

It turns out that if you have a capacitor made up of different

dielectrics (oil, LDPE, and air, for example), the voltage stress "seen"

by each dielectric will be a function of its dielectric constant ("k")

versus the other dielectrics. You can liken it to having a number of

mini capacitors connected in series, all of different capacitance. The

capacitor with the lowest "k" will has the lowest capacitance, and will

have the highest voltage developed across it.

When you have an entrapped air bubble, a disproportionate amount of

voltage stres will be applied to the air bubble, since the other

dielectrics have a "k" of 2 or more, while the air has a "k" of 1. If

the stress exceeds the breakdown strength of air, it ionizes, generating

ozone, microscopic electrical discharges, ultraviolet light, and

localized heating. These attack the surrounding oil and the plastic

dielectric so that its just a matter of time before they fail.

>

> 3. I made a "dry" capacitor with 4 mm plexiglass sheet and copper (you can

> see it on my Web page) for a value of 2 X 550 pF. No oil used and surely

> there is air (and moisture) trapped between the layers. Been in use ALL the

> time with my TC, watched it MANY times, never seen any corona effect or

> blue light anywhere. It is used as a filter cap for the MOTs not to get back

> the 220 KHz TC resonance frequencies. Of course you may tell me that the

> electric field in it is so much lower than in a 0.01uF cap: is that the

> only reason my cap is OK?

> >

>> Marco Denicolai

<SNIP>

This capacitor is not seeing the high frequency rapid voltage reversals

seen by the tank cap. It actually takes considerable ionization for you

to see the blue glow... not seeing any does not necessarily mean that it

isn't occuring. The ionization and localized discharge effects are not

immediate... but they are inevitable. After a period of time, the

plastic will show signs of surface destruction and tracking,

particularly around the edges of the metal plates where the E-fields are

strongest.

BTW, It IS possible to make a oil-LDPE capacitor that will withstand the

presence of entrapped air. If you can insure that the air bubble

thickness is relatively small versus the total thickmess of the oil and

plastic dielectric, it's possible to reduce the effective voltage stress

across the air below the point where it cannot ionize. This is another

reason why commercial HV caps are constructed by connecting a number of

identical sections in series so that the overall voltage stress seen by

any module is reduced. For example, if you designed your capacitor

system such that the E-field "seen" by any entrapped air never exceeded

350 V/Mil, the air would not ionize, and the capacitor would survive.

Hope this answers your questions. Safe coilin' to you!

-- Bert --

Date: Tue, 06 Oct 1998 06:57:45 -0700

From: Gene Waldo <gwaldo@1stnetusa.com>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Secondary Insulation, and good display of streamers

Sender: tesl-err@emachine.com

DavidAllan@aol.com wrote:

>

> I am following the thread about secondary insulation.

> I hope to finish this coil before Halloween so I can add it to my outdoor "mad

> scientist" display. The coil itself would be under the overhang of my porch.

> I'd run it manually (of course), and I'd like to send streamers to  some kind

> of grounded strike points. I was thinking of taking some hardware cloth (like

> stiff chicken wire) and making a few targets attached to my (to be built) rf

> ground. The targetcould be cut to present a number of pointed ends to the

> coil.

David,

If you really wanted to go nuts, make some simple MbfB rotary switch

type thing to sequence the RFGnd to your various targets and have the

strikes sequence to them...  Flourescent tubes inside props of one sort

or another... Ignite a 'tiki' lamp?  I'd play with this one a bit..

don't set your place on fire!

-------------------------------------

Gene Waldo -  gwaldo@1stnetusa.com  - "Igor in Residence"

---------------------------------------------------------------

What's good for the planet is more OZONE! See the machinery form up!

NEW PAGE!   http://members.1stnetusa.com/danmurphy/waldo.htm

From: wwl@netcomuk.co.uk (Mike Harrison)

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Annealing Copper Tubing

Date: Tue, 06 Oct 1998 15:05:15 GMT

Organization: White Wing Logic

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

On Mon, 05 Oct 1998 20:19:30 PDT, you wrote:

>Kyle wrote:

>

>>	Just thought of this. Might be kinda hard to heat a long coil of

>>tubing with a torch. How about clipping each end to and arc - welder

>and wait...till it's red?

>>	An arc welder may have 1000 uses! I think I'll run out and try it.

>

>Once, while re-plumbing a half-dead TIG torch, I tried an experiment

>(long story deleted here...)  I passed 80+ amps through a one foot long

>section of 1/8 inch copper tubing for one minute.  It didn't even get

>hot (just warm)!

>

>Conclusion: Copper is one DAMN good conductor!  To heat a length of 1/4

>copper tubing would require probably 100s of amps at least...

The conductivity would be an advantage if you could magnetically

couple power into it. I'd try bending it into a circle (or using 2

lengths joined at both ends), and passing through the centre of a big

(500VA+) toroidal transformer, to make a single shorted turn. Variac

(or ballast) the toroid's primary to limit the power. In theory, it

should even be self-regulating if you got the input power right, as

the R would increase as it heated up, reducing the current.

Remember to keep an eye on the transformer's temperature so the

insulation doesn't melt.

From: "Basura, Brian" <brian.basura@unistudios.com>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: RE: Pigs and other barnyard experiments.

Date: Tue, 6 Oct 1998 08:08:52 -0700

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Bill,

I checked all three of my MOT's last night.  All of them pull .75A with the

secondary open and the voltage under 125vac.  The inductance on the three

different transformers was close and averaged 31.5mH with a open secondary

and 10mH with the secondary shorted.

One more note - all three have current limiting shunts.

Regards,

Brian D. Basura

> ----------

> From: 	Basura, Brian

> Sent: 	October 05, 1998 2:15 PM

> To: 	'Tesla-2 Mailing List'

> Subject: 	RE: Pigs and other barnyard experiments.

>

> Bill,

>

> I'll try and verify my previous statement tonight.  I do know the

> inductance with an open secondary is approximately 23mH.  As far a series

> vs. parallel resistance goes series seems to make more sense and the

> bigger coils are set up that way.  Series resistance is the approach I'll

> be trying as soon as I get enough R to equal .3-.5 ohms.

>

> Brian D. Basura

>

> 	----------

> 	From: 	Bill the arcstarter[SMTP:arcstarter@hotmail.com]

> 	Reply To: 	Tesla-2 Mailing List

> 	Sent: 	October 05, 1998 12:45 PM

> 	To: 	Tesla-2 Mailing List

> 	Subject: 	RE: Pigs and other barnyard experiments.

>

> 	Coilers,

> 	 Brian wrote:

> 	>I'm surprised that you get any current through your MOT without the

> 	>secondary shorted.  Mine pull almost 0A with an open secondary.

>

> 	This is very different from how mine works.  Mine draws about 6 amps at

> 	120 volts with the secondary open.  WIth it closed - it draws 30-40 amps

> 	- pops the 20 amp fuse in about 1-2 seconds...

>

> 	<snip>

>

> 	You also describe using MOTs with parallel resistance (6 ohms) as a

> 	limiting device.  How many people out there place the resistance in

> 	SERIES with their MOT limiters vs in parallel?? I wonder if one way is

> 	universally better than the other...?

>

> 	-Bill the arcstarter

> 	Starting arcs in Cinci, OH

> 	http://www.geocities.com/CapeCanaveral/Hangar/6160

>

>

From: DavidAllan@aol.com

Date: Tue, 6 Oct 1998 11:22:05 EDT

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Secondary Insulation, and good display of streamers

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Gene,

Some good ideas! Thanks. I just hope to get this thing constructed and tuned

in time. I keep running through a mental list of what's left, and it seems a

lot (considering the short bursts of time I have to work): spark gap,

capacitor tanks, primary coil, control panel, safety gap, toroid--geez, all I

really have right now is a secondary and two cap rolls. I'd better get to

work!

David Allan

In a message dated 98-10-06 10:30:02 EDT, you write:

Subj:	 Re: Secondary Insulation, and good display of streamers

Date:	98-10-06 10:30:02 EDT

From:	gwaldo@1stnetusa.com (Gene Waldo)

Sender:	tesl-err@emachine.com

Reply-to:	tesla-2@emachine.com (Tesla-2 Mailing List)

To:	tesla-2@emachine.com (Tesla-2 Mailing List)

DavidAllan@aol.com wrote:

>

> I am following the thread about secondary insulation.

> I hope to finish this coil before Halloween so I can add it to my outdoor "mad

> scientist" display. The coil itself would be under the overhang of my porch.

> I'd run it manually (of course), and I'd like to send streamers to some kind

> of grounded strike points. I was thinking of taking some hardware cloth (like

> stiff chicken wire) and making a few targets attached to my (to be built) rf

> ground. The targetcould be cut to present a number of pointed ends to the

> coil.

David,

If you really wanted to go nuts, make some simple MbfB rotary switch

type thing to sequence the RFGnd to your various targets and have the

strikes sequence to them...  Flourescent tubes inside props of one sort

or another... Ignite a 'tiki' lamp?  I'd play with this one a bit..

don't set your place on fire!

-------------------------------------

Gene Waldo -  gwaldo@1stnetusa.com  - "Igor in Residence"

---------------------------------------------------------------

X-Originating-IP: [195.121.16.22]

From: "Sam Barros" <sambarros@hotmail.com>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Dutch Coilers unite!

Date: Tue, 06 Oct 1998 09:35:22 PDT

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

>From: "Peter Staal" <drtesla@casema.net>

>To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

>Subject:

>Date: Mon, 5 Oct 1998 22:12:47 +0200

>Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

>

>Who has or know a Tesla Coil in holland?

>Because I want to see you for real, in holland, because I live there.

I live in Holland: Bergen N.H. (near Alkmaar, 50KM away from

Amsterdam)), more precisely. Don't take it personally, but your country

sucks for building Tesla Coils. Did you know you have a ban on Neon Sign

Transformers above 400W of power?

I do have a Tesla Coil... It's pretty big as well (relatively)! Some

20cm diameter for the primary coil, 1 meter tall... Should make nice

1-2meters long sparks. 3 meters if I force it But I haven't yet

obtained a transformer that could power it.

By the way, if you are wondering, I am the only person I have notice of

who has tried to build a TC in Holland. No wander though: Things here

cost so much it would be cheaper to have it built in America and brought

here on a first class plane sit!

Mail me privately at sambarros@yahoo.com for more details...

Sam Barros.

X-Mailer: IPAD 2.0

Date: Tue, 06 Oct 1998 11:45:09 -0500

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: Secondary Insulation, and good display of streamers

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

At 07:02 AM 10/6/98 EDT, you wrote:

>David,

>   I have a coil that I used a sprayed on insulating varnish made by Krylon.

>I put on about six or eight coat(don't remember now) and still can feel the

>windings, but have had no problems.  Be sure that you tune your coil before

>applying full power to prevent high voltage nodes from appearing at points

>below the top of the coil and you should have no trouble.  I have seen

>mistuned coils arcing from one point of a secondary to another point a few

>inches away, and no amount of insulation would prevent this until the coil is

>properly tuned.

>  Hope this helps.

>Mike Hollingsworth

Good point Mike. You can even run your secondary coil UNCOATED.

You just have to adjust the coupling and/or primary secondary

spacing to prevent arcing.

If you are overcoupled no amount of insulation will prevent arcing.

TTYL!

Mike Hammer

mhammer@misslink.net

X-Originating-IP: [199.217.140.22]

From: "Bill the arcstarter" <arcstarter@hotmail.com>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Annealing Copper Tubing

Date: Tue, 06 Oct 1998 09:48:58 PDT

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

Mike wrote:

>The conductivity would be an advantage if you could magnetically

>couple power into it. I'd try bending it into a circle (or using 2

>lengths joined at both ends), and passing through the centre of a big

>(500VA+) toroidal transformer, to make a single shorted turn. Variac

>(or ballast) the toroid's primary to limit the power. In theory, it

>should even be self-regulating if you got the input power right, as

>the R would increase as it heated up, reducing the current.

>Remember to keep an eye on the transformer's temperature so the

>insulation doesn't melt.

That reminds me of a set of experiments I did with the MOT with the

secondary removed.  Basically, each turn of wire around the core would

produce 0.9 volts or thereabouts.  But - the current is a bit more

difficult to determine as I shall describe...

I placed one turn of #24 wire around the core of the MOT and energized

the primary.  A current of about 3-4 amps was measured flowing through

the #24 wire.

I repeated this using a #18 wire - and meausured a larger current -

maybe 10 amps or so (can't remember the exact value).

Then I put a loop of #6 copper around the core - the current was >300

amps!  The loop of #6 was magnetically repelled away from the primary

windings.

I think what is going on here is that, for conditions not getting into

the flux limits of the core, the final winding current is computed by

the number of turn times the voltage per turns, divided by the wire

resistance.

For this particular core, I measured/computed the max flux transfer was

to the tune of about 1400 amp*turns around the core.

This has some interesting implications regarding how current you could

possibly get out of a secondary, as a function of wire diameter, etc...

The funny thing was I then went back to the #24 wire, and wrapped two

turns around the core.  The wire current stayed about the same.  This

makes sense because the voltage was doubled (two turns) , but the

resistance was also doubled since the wire length was about twice as long.

So, one might be able to use a MOT or perhaps an even larger xfmr core

to heat up a length of copper tubing hot enough to anneal it...

-Bill the arcstarter

Starting arcs in Cinci, OH

http://www.geocities.com/CapeCanaveral/Hangar/6160

X-Mailer: IPAD 2.0

Date: Tue, 06 Oct 1998 11:56:53 -0500

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: Again a CAP question...

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

At 09:21 AM 10/6/98 +0200, Marco wrote:

>Here is my first bunch of stupid questions about capacitors.

>

>1.In my understanding, people design rolled caps (homemade) so that the PE or

>whatever dielectric used stands a certain desired voltage. Oil is added

>ONLY to prevent corona effects: correct?

Mostly correct. The oil also excludes air which has a lower dielectric

constant and would lower total capacitance.

Most common insulating oils have LOWER breakdown voltages than

polyethylene so would not add to the voltage rating.

>

>2. Now, if the dielectric is correctly calculated, why are air bubbles

>between the layers are so damn critical and can cause capacitor destruction?

Because it is precisely at these air bubbles that corona will form.

This corona will degrade and break down the insulation eventually

leading to failure.

>

>3. I made a "dry" capacitor with 4 mm plexiglass sheet and copper (you can

>see it on my Web page) for a value of 2 X 550 pF. No oil used and surely

>there is air (and moisture) trapped between the layers. Been in use ALL the

>time with my TC, watched it MANY times, never seen any corona effect or

>blue light anywhere. It is used as a filter cap for the MOTs not to get back

>the 220 KHz TC resonance frequencies. Of course you may tell me that the

>electric field in it is so much lower than in a 0.01uF cap: is that the

>only reason my cap is OK?

>

Since your cap is not in the Tesla tank circuit it is not

under the stress that it would be. If it was in the tank circuit

it would be seeing full voltage reversals and high RF currents.

The plexiglass would heat up due to high RF loss and possibly crack.

Just because you dont see visible corona doesnt necessarily mean

its not there. Visible corona is an extreme case.

TTYL!

Mike Hammer

mhammer@misslink.net

X-Originating-IP: [199.217.140.22]

From: "Bill the arcstarter" <arcstarter@hotmail.com>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: RE: Pigs and other barnyard experiments.

Date: Tue, 06 Oct 1998 09:59:44 PDT

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

Brian wrote:

>I checked all three of my MOT's last night.  All of them pull .75A with

>the secondary open and the voltage under 125vac.  The

>inductance on the three different transformers was close and

>averaged 31.5mH with a open secondary and 10mH with the

>secondary shorted.

>

>One more note - all three have current limiting shunts.

>

>Regards,

>Brian D. Basura

Odd.  I'll have to go measure my other MOTs tonight.

The primary current (measured at 120vac) didn't change appreciably when

I removed the shunts.

But - you say you measured 31.5 mH with an open secondary.  If you do

the math - a 0.035 H inductor should draw 120/(2*PI*60*0.035) = 9 amps.

What gives?  What kind of LCR meter did you use?

I can only think that the small LCR meters (like mine) aren't real

accurate when used on something with an iron core.  My LCR measured 1.4

mH from the 0 to 120v leads of my pig, but the pig only draws 0.7 amps

at 120 volts (a computed 0.45 H inductance)...?

-Bill the arcstarter

Starting arcs in Cinci, OH

http://www.geocities.com/CapeCanaveral/Hangar/6160

From:          "Malcolm Watts" <MALCOLM@directorate.wnp.ac.nz>

Organization:  Wellington Polytechnic, NZ

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Date:          Wed, 7 Oct 1998 10:39:00 +1200

Subject:       RE: Pigs and other barnyard experiments.

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Hi Bill,

> From:           "Bill the arcstarter" <arcstarter@hotmail.com>

> To:             "Tesla-2 Mailing List" <tesla-2@emachine.com>

> Subject:        RE: Pigs and other barnyard experiments.

> Date sent:      Tue, 06 Oct 1998 09:59:44 PDT

> Send reply to:  "Tesla-2 Mailing List" <tesla-2@emachine.com>

> Brian wrote:

>

> >I checked all three of my MOT's last night.  All of them pull .75A with

> >the secondary open and the voltage under 125vac.  The

> >inductance on the three different transformers was close and

> >averaged 31.5mH with a open secondary and 10mH with the

> >secondary shorted.

> >

> >One more note - all three have current limiting shunts.

> >

> >Regards,

> >Brian D. Basura

>

> Odd.  I'll have to go measure my other MOTs tonight.

>

> The primary current (measured at 120vac) didn't change appreciably when

> I removed the shunts.

>

> But - you say you measured 31.5 mH with an open secondary.  If you do

> the math - a 0.035 H inductor should draw 120/(2*PI*60*0.035) = 9 amps.

> What gives?  What kind of LCR meter did you use?

That is probably true. I've examined the magnetizing current for a

240V MOT on the scope and discovered the waveform was highly

distorted and peaking at 20A!! when run from a 240V supply. RMS

current was lower of course. In fact when looking at what cores can

stand, peak current is the real quantity of interest and it is worth

remembering that Vpk from the mains is SQRT(2) higher than the RMS

value.

> I can only think that the small LCR meters (like mine) aren't real

> accurate when used on something with an iron core.  My LCR measured 1.4

> mH from the 0 to 120v leads of my pig, but the pig only draws 0.7 amps

> at 120 volts (a computed 0.45 H inductance)...?

The discrepancy probably arises because you were not applying enough

current to overcome core hysteresis. If the domains can't change,

inductance appears vanishingly small.

?

Malcolm

X-Mailer: IPAD 2.0

Date: Tue, 06 Oct 1998 17:41:04 -0500

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Mike Hammer <mhammer@misslink.net>

Subject: latest list archive

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Hello all,

The latest archive covering posts from July

and August has been all edited, pushed, pulled and

smashed. It will be found on Dave Bronakowski's

archive site as well as the funet.fi ftp site.

List traffic was down about 40 percent for this archive period

despite a steady increase in subscribers. More lurkers and

less participants.

The file name for the latest archive is tes2arc13.zip

and Daves URL is: http://www.velocity.net/~djb/tesla.shtml

Many thanks to Dave for providing this space for us all.

For those that prefer to use funet.fi the URL is:

ftp://nic.funet.fi/pub/sci/electrical/tesla/maillist/tesla2/

Thanks to Kristian Ukkonen for maintaining that site.

TTYL!

Mike Hammer

mhammer@misslink.net

From:          "Malcolm Watts" <MALCOLM@directorate.wnp.ac.nz>

Organization:  Wellington Polytechnic, NZ

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Date:          Wed, 7 Oct 1998 10:41:15 +1200

Subject:       RE: Pigs and other barnyard experiments.

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Hi Brian,

> From:           "Basura, Brian" <brian.basura@unistudios.com>

> To:             "Tesla-2 Mailing List" <tesla-2@emachine.com>

> Subject:        RE: Pigs and other barnyard experiments.

> Date sent:      Tue, 6 Oct 1998 08:08:52 -0700

> Send reply to:  "Tesla-2 Mailing List" <tesla-2@emachine.com>

> Bill,

>

> I checked all three of my MOT's last night.  All of them pull .75A with the

> secondary open and the voltage under 125vac.  The inductance on the three

> different transformers was close and averaged 31.5mH with a open secondary

> and 10mH with the secondary shorted.

>

> One more note - all three have current limiting shunts.

>

> Regards,

> Brian D. Basura

The effects of the shunts on magnetizing current should be minimal as

the shunts are in series with an air gap. Load current should be the

object of the shunt's attention.

Malcolm

Date: Tue, 06 Oct 1998 20:27:16 -0500

From: RODERICK MAXWELL <tank@magnolia.net>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Capacitor safety gaps

Sender: tesl-err@emachine.com

I have just finished building my largest (and most expensive)

capacitor yet and I don't want to punch a hole through the dielectric.

The capacitor is 55nf using 60mil poly between the plates. How wide

should I set a safety gap across the cap to prevent overvoltage?

Talk about big and heavy it must weigh 50lbs!

Frankensteins Helper

Max

X-Mailer: IPAD 2.0

Date: Tue, 06 Oct 1998 20:37:55 -0500

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: Capacitor safety gaps

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

At 08:27 PM 10/6/98 -0500, you wrote:

>I have just finished building my largest (and most expensive)

>  capacitor yet and I don't want to punch a hole through the dielectric.

>  The capacitor is 55nf using 60mil poly between the plates. How wide

>  should I set a safety gap across the cap to prevent overvoltage?

>    Talk about big and heavy it must weigh 50lbs!

>

>

>                              Frankensteins Helper

>                                     Max

Roderick,

55 nf? Thats a mighty big cap. No wonder you want

to add a safety gap. I bet it took quite a while to build.

But you didnt mention what your transformer voltage was.

What I would do is set the safety just slightly wider than

the main gap as a starting point and work from there.

I would think somewhere around .375 inch for 12kv and maybe .500

for 15kv as maximum values.

TTYL!

Mike Hammer

mhammer@misslink.net

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Again a CAP question...

X-Mailer: IPAD 2.0

From: break7@juno.com (Kyle A. Break)

Date: Tue, 06 Oct 1998 22:14:13 EDT

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

I've had a lot of recent experience building and blowing poly caps .

I've watched closely as they operate and die. My cap is in a see through

container so I see all that can be seen. First at 60hz I don't

see corona until I reach 7500v. The visable corona is only present where

the air is. Anytime you see corona there is damage being done. Air deep

inside the cap where you can't see is usually the killer. Where ever

corona is, heat is, the heat starts to melt the poly until the voltage

punches through. I've taken apart a cap that looked perfect only to find

strange puddle - shaped depressions that are where air was trapped. The

oil in this area has become thick as honey. Probably caused by corona

burn. Usually my caps die because I run them too long at a time and the

little air that's inside gets hot and a yellow flame - spark errupts  and

coil operation stops .My caps always have air in them because they're not

stacked verticaly. Getting air out is very important!

If it's not dangerous it's not fun!  Kyle  break7@juno.com

On Tue, 06 Oct 1998 06:44:42 -0500 Bert Hickman <bert.hickman@aquila.com>

writes:

>Marco Denicolai wrote:

>>

>> Here is my first bunch of stupid questions about capacitors.

>>

>> 1.In my understanding, people design rolled caps (homemade) so that the PE or

>> whatever dielectric used stands a certain desired voltage. Oil is added

>> ONLY to prevent corona effects: correct?

>>

>

>Oil also fills up any "gaps" between the metal plates and the

>dielectric, and between dielectric sheets. These gaps would otherwise be

>full of air (k=1), considerably reducing the capacitance of the

>home-made cap.

>

>> 2. Now, if the dielectric is correctly calculated, why are air bubbles

>> between the layers so damn critical and can cause capacitor destruction?

>

>It turns out that if you have a capacitor made up of different

>dielectrics (oil, LDPE, and air, for example), the voltage stress "seen"

>by each dielectric will be a function of its dielectric constant ("k")

>versus the other dielectrics. You can liken it to having a number of

>mini capacitors connected in series, all of different capacitance. The

>capacitor with the lowest "k" will has the lowest capacitance, and will

>have the highest voltage developed across it.

>

>When you have an entrapped air bubble, a disproportionate amount of

>voltage stres will be applied to the air bubble, since the other

>dielectrics have a "k" of 2 or more, while the air has a "k" of 1. If

>the stress exceeds the breakdown strength of air, it ionizes,

>generating ozone, microscopic electrical discharges, ultraviolet light,

>and localized heating. These attack the surrounding oil and the plastic

>dielectric so that its just a matter of time before they fail.

>

>>

>> 3. I made a "dry" capacitor with 4 mm plexiglass sheet and copper (you can

>> see it on my Web page) for a value of 2 X 550 pF. No oil used and surely

>> there is air (and moisture) trapped between the layers. Been in use ALL the

>> time with my TC, watched it MANY times, never seen any corona effect or

>> blue light anywhere. It is used as a filter cap for the MOTs not to get back

>> the 220 KHz TC resonance frequencies. Of course you may tell me that the

>> electric field in it is so much lower than in a 0.01uF cap: is that the

>> only reason my cap is OK?

>> >

>

>>> Marco Denicolai

>

><SNIP>

>

>This capacitor is not seeing the high frequency rapid voltage reversals

>seen by the tank cap. It actually takes considerable ionization for you

>to see the blue glow... not seeing any does not necessarily mean that it

>isn't occuring. The ionization and localized discharge effects are not

>immediate... but they are inevitable. After a period of time, the

>plastic will show signs of surface destruction and tracking,

>particularly around the edges of the metal plates where the E-fields are

>strongest.

>

>BTW, It IS possible to make a oil-LDPE capacitor that will withstand the

>presence of entrapped air. If you can insure that the air bubble

>thickness is relatively small versus the total thickmess of the oil and

>plastic dielectric, it's possible to reduce the effective voltage stress

>across the air below the point where it cannot ionize. This is another

>reason why commercial HV caps are constructed by connecting a number of

>identical sections in series so that the overall voltage stress seen by

>any module is reduced. For example, if you designed your capacitor

>system such that the E-field "seen" by any entrapped air never exceeded

>350 V/Mil, the air would not ionize, and the capacitor would survive.

>

>Hope this answers your questions. Safe coilin' to you!

>

>-- Bert --

>

Date: Tue, 06 Oct 1998 21:20:48 -0500

From: RODERICK MAXWELL <tank@magnolia.net>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Capacitor safety gaps

Sender: tesl-err@emachine.com

> Roderick,

>

> 55 nf? Thats a mighty big cap. No wonder you want

> to add a safety gap. I bet it took quite a while to build.

>

> But you didnt mention what your transformer voltage was.

>

> What I would do is set the safety just slightly wider than

> the main gap as a starting point and work from there.

>

> I would think somewhere around .375 inch for 12kv and maybe .500

> for 15kv as maximum values.

>

> TTYL!

>

> Mike Hammer

> mhammer@misslink.net

The voltage will be around 7500v for starters. I am going to use two

H&R transformers in an antiparallel configuration. They will be

rectified by a full wave bridge rectifier and filtered by a doorknob cap

(.005uf). Later I will increase the size of the filter cap to see if

this will increase the supply voltage. It should due to the frequency

increase caused by the bridge (60hz/120hz). Thank you for your help!

Frankensteins Helper

Max

X-Originating-IP: [208.16.145.172]

From: "Bill the arcstarter" <arcstarter@hotmail.com>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: RE: Pigs and other barnyard experiments.

Date: Tue, 06 Oct 1998 19:38:50 PDT

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

Hi Malcolm.

You wrote:

>> But - you say you measured 31.5 mH with an open secondary.  If you do

>> the math - a 0.035 H inductor should draw 120/(2*PI*60*0.035) = 9 amps.

>> What gives?  What kind of LCR meter did you use?

>

>That is probably true. I've examined the magnetizing current for a

>240V MOT on the scope and discovered the waveform was highly

>distorted and peaking at 20A!! when run from a 240V supply. RMS

>current was lower of course. In fact when looking at what cores can

>stand, peak current is the real quantity of interest and it is worth

>remembering that Vpk from the mains is SQRT(2) higher than the RMS

>value.

The sqrt(2) ratio is only true for a nondistorted sine wave.  Other

waveforms will have other converstions from peak to RMS, of course.

A small nit!

I'd better get out the 'scope and see what mine is *really* doing...

>

>> I can only think that the small LCR meters (like mine) aren't real

>> accurate when used on something with an iron core.  My LCR measured 1.4

>> mH from the 0 to 120v leads of my pig, but the pig only draws 0.7 amps

>> at 120 volts (a computed 0.45 H inductance)...?

>

>The discrepancy probably arises because you were not applying enough

>current to overcome core hysteresis. If the domains can't change,

>inductance appears vanishingly small.

Sounds good to me!

-Bill

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Inductive Heating

X-Mailer: IPAD 2.0

From: break7@juno.com (Kyle A. Break)

Date: Tue, 06 Oct 1998 22:42:55 EDT

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Have you ever seen how a machine shop installs pistons on

press-fit rods? They put the small end in a coil, push a foot switch and

in just a few seconds it's red!  Well watts is watts and it takes a lots

to heat a whole primary.

If it's not dangerous it's not fun!  Kyle  break7@juno.com

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Salt Water Caps  Way better design?

X-Mailer: IPAD 2.0

From: break7@juno.com (Kyle A. Break)

Date: Tue, 06 Oct 1998 23:10:36 EDT

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

To anyone that knows, Could you use plastic bottles like 2 liter

soda bottles? This would definitely get you away from glass. The plastic

bottles I've seen have PETE on them ,what kind of plastic is that? Is it

any good? Milk and orange juice bottles are HDPE, but have irregular

shapes, or is that a problem?  Might be a good idea (cheap) in Holland?

If it's not dangerous it's not fun!  Kyle  break7@juno.com

X-Mailer: IPAD 2.0

Date: Tue, 06 Oct 1998 23:53:43 -0500

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: Salt Water Caps  Way better design?

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

At 11:10 PM 10/6/98 EDT, you wrote:

>	To anyone that knows, Could you use plastic bottles like 2 liter

>soda bottles? This would definitely get you away from glass. The plastic

>bottles i've seen have PETE on them,what kind of plastic is that? Is it

>any good? Milk and orange juice bottles are HDPE, but have irregular

>shapes, or is that a problem?  Might be a good idea (cheap) in Holland?

>

>	If it's not dangerous it's not fun!  Kyle  break7@juno.com

Those PET bottles are Mylar. Mylar is a very good dielectric

and makes great caps...for DC. Its RF dissipation factor is

terrible and will likely melt if subjected to any meaningful

RF levels.

It would probably be marginally better than glass but will

have a much lower breakdown voltage than glass bottles.

This will necessitate using these bottles in series which

will result in lower capacitance.

Plus the lower dielectric constant will result in even

less capacitance.

So yes you could make a cap using these but it would take a great

many of them to get any worthwhile capacitance.

TTYL!

Mike Hammer

mhammer@misslink.net

X-Mailer: IPAD 2.0

Date: Wed, 07 Oct 1998 14:39:28 +0930

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Tony Finnis <amf@sa.apana.org.au>

Subject: 'Micro' Tesla Coil

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

I am new to this list, however, I have been lurking on the 'other list'

for a while. I have also had a brief look through the archives of

this list.

The reason for this post is that I have recently come into possession of

some very small high voltage units from a Konica photo-copier plus the

associated power supply. There are two HV units as follows ;

Unit A:

This consists of two very small transformers plus associated circuitry

with +24V DC input, and one unit with -5.5 kV output at -500uA, the other

with -5.5 kV output at -300uA.

Unit B:

This consists of one somewhat larger transformer sealed in a wax-like

substance plus associated circuitry with +24V DC input and 4.2 kV AC

output at 450uA.

Now I accept that the power output of these units is very low, but would

it be possible (or a complete waste of time) to use either of these units

to make a 'micro' TC just for the hell of it? Would it be better to

use Unit A or Unit B? Can the two outputs on Unit A be connected

together to increase the output power?

I know that there are better power sources for TC's, but I am interested

in what can be done with the above units, and if it is possible to make a

very small TC.

I am not currently interested in making a larger TC, leaving this to my

son Mark who is into the larger TC's in an ever increasing way.

His site is worth a look if you havn't already done so and is at

http://www.cobweb.com.au/~dkfinnis

Any comments regarding the above will be appreciated.

Tony Finnis

X-Mailer: IPAD 2.0

Date: Wed, 07 Oct 1998 00:09:48 -0500

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: Capacitor safety gaps

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

At 09:20 PM 10/6/98 -0500, you wrote:

>> Roderick,

>>

>> 55 nf? Thats a mighty big cap. No wonder you want

>> to add a safety gap. I bet it took quite a while to build.

>>

>> But you didnt mention what your transformer voltage was.

>>

>> What I would do is set the safety just slightly wider than

>> the main gap as a starting point and work from there.

>>

>> I would think somewhere around .375 inch for 12kv and maybe .500

>> for 15kv as maximum values.

>>

>> TTYL!

>>

>> Mike Hammer

>> mhammer@misslink.net

>

>

>  The voltage will be around 7500v for starters. I am going to use two

>H&R transformers in an antiparallel configuration. They will be

>rectified by a full wave bridge rectifier and filtered by a doorknob cap

>(.005uf). Later I will increase the size of the filter cap to see if

>this will increase the supply voltage. It should due to the frequency

>increase caused by the bridge (60hz/120hz). Thank you for your help!

>

>

Roderick,

Yes now I remember that you had those H&R transformers. I think

it unlikely that you will exceed 10kv with your supply so your

cap should be more than adequate. You probably dont really even

need a safety gap across that cap as long as your main gap is

not set overwide and you have the safety gaps across the transformers

set properly.

But it certainly wont hurt to have it. I would start at about

.375 inch (3/8) and close up the gap a little at a time until

it just fires. When it goes off it will sound like a rifle shot.

Then open it just slightly. .375 inch is what I set my safety gap

at in my 12kv system.

TTYL!

Mike Hammer

mhammer@misslink.net

X-Mailer: IPAD 2.0

Date: Wed, 07 Oct 1998 08:29:58 +0200

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Marco Denicolai <marco@vistacom.fi>

Subject: My Tesla page has moved!

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

My page old WWW Tesla page address was:

http://vci.vistacom.fi/~marco/mytesla.html

but I have moved it to:

http://www.saunalahti.fi/dncmrc/mytesla.html

Please, update your links and stay tuned!

Thank you

Marco Denicolai                   Vista Communication Instruments, Inc.

Hardware Development Manager      www.vistacom.fi

marco@vistacom.fi

Date: Wed, 07 Oct 1998 07:01:43 -0500

From: Bert Hickman <bert.hickman@aquila.com>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Organization: Stoneridge Engineering

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: 'Micro' Tesla Coil

Sender: tesl-err@emachine.com

Tony,

Welcome aboard, Tony!

Both power supplies are quite small. However, it may be possible to make

a small coil from one of these, especially if you used the DC supply

(Unit A) to charge the tank cap over a longer period of time. The

problem will be in keeping the resulting RF (when the main gap fires)

from getting back into the power supply and destroying the diodes.

A small tank cap (0.0005 - 0.001 uF), and a relatively low duty cycle

(~10 Breaks Per Second) will get you to an acceptable peak power to

generate some respectable sparks with a small (1-2" diameter") coil. A

relatively hefty filter cap (say 0.05 uF) across the output of the

supply, combined with a 1000 ohm 10 W wirewound resistor should do the

trick as far as blocking the RF from the supply.

Safe (future, micro) coilin' to you!

-- Bert --

Tony Finnis wrote:

>

> I am new to this list, however, I have been lurking on the 'other

> list' for a while.  I have also had a brief look through the archives

> of this list.

>

> The reason for this post is that I have recently come into possession

> of some very small high voltage units from a Konica photo-copier plus

> the associated power supply.  There are two HV units as follows ;-

>

> Unit A:

> This consists of two very small transformers plus associated circuitry

> with +24V DC input, and one unit with -5.5 kV output at -500uA, the

> other with -5.5 kV output at -300uA.

> Unit B:

> This consists of one somewhat larger transformer sealed in a wax-like

> substance plus associated circuitry with +24V DC input and 4.2 kV AC

> output at 450uA.

>

> Now I accept that the power output of these units is very low, but

> would it be possible (or a complete waste of time) to use either of

> these units to make a 'micro' TC just for the hell of it?  Would it be

> better to use Unit A or Unit B?  Can the two outputs on Unit A be

> connected together to increase the output power?

>

> I know that there are better power sources for TC's, but I am

> interested in what can be done with the above units, and if it is

> possible to make a very small TC.

> I am not currently interested in making a larger TC, leaving this to

> my son Mark who is into the larger TC's in an ever increasing way.

> His site is worth a look if you heven't already done so and is at :-

> http://www.cobweb.com.au/~dkfinnis

>

> Any comments regarding the above will be appreciated.

>

> Tony F

> Hahndorf  -  South Australia

>

X-Originating-IP: [199.217.140.22]

From: "Bill the arcstarter" <arcstarter@hotmail.com>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Inductive Heating

Date: Wed, 07 Oct 1998 05:26:52 PDT

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

Kyle wrote:

>	Have you ever seen how a machine shop installs pistons on

>press-fit rods? They put the small end in a coil, push a foot switch and

>in just a few seconds it's red!  Well watts is watts and it takes a lots

>to heat a whole primary.

Hmm - never seen induction heating done in a shop environment.  Usually

these guys use an oven to heat their parts.  It works that was for

installing ring gears (engages the starter) onto flywheels...

Any clue what is in this induction heater??

-Bill

Date: Wed, 07 Oct 1998 09:09:46 -0700

From: tomstock <tomstock@geocities.com>

X-Mailer: IPAD 2.0

X-Accept-Language: en

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: 'Micro' Tesla Coil

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Wouldn't these tiny transformers be good for a relatively safe plasma globe?

-tom

X-Mailer: IPAD 2.0

Date: Wed, 07 Oct 1998 23:35:43 +0930

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Tony Finnis <amf@sa.apana.org.au>

Subject: Re: 'Micro' Tesla Coil

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

G'day tomstock, you wrote :-

>

>

>Wouldn't these tiny transformers be good for a relatively safe plasma globe?

Yes - I believe they would.

I intended to try a plasma globe with one of these units if the 'micro' TC

was not possible.  I even might try both seeing that I have 2 units.

Which unit (A or B) would you consider best for a plasma globe and have you

any information which may be of help?

Many Thanks

Tony F

Date: Wed, 07 Oct 1998 11:29:55 -0700

From: Gene Waldo <gwaldo@1stnetusa.com>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Inductive Heating

Sender: tesl-err@emachine.com

Bill the arcstarter wrote:

>

> Kyle wrote:

>

> >       Have you ever seen how a machine shop installs pistons on

> >press-fit rods?>

> Hmm - never seen induction heating done in a shop environment.  Usually

> these guys use an oven to heat their parts.  It works that was for

> installing ring gears (engages the starter) onto flywheels...

>

> Any clue what is in this induction heater??

>

> -Bill

>

Bill,

We used to inductively heat small parts in a ~ 1-3 turn watercooled

coil, driven by major P. supplies. Big tubes and stuff. Used for

welding, annealing etc.

--

Gene Waldo -  gwaldo@1stnetusa.com  - "Igor in Residence"

---------------------------------------------------------------

What's good for the planet is more OZONE! See the machinery form up!

NEW PAGE!   http://members.1stnetusa.com/danmurphy/waldo.htm

X-Originating-IP: [207.100.20.60]

From: "Philip Mac Duffie" <punk_42@hotmail.com>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Bypass caps

Date: Wed, 07 Oct 1998 13:00:45 PDT

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

I was curious about bypass caps, What do they do? Will three 10kv

.001uf caps be sutable for 9kv input?

X-Mailer: IPAD 2.0

Date: Wed, 07 Oct 1998 15:43:41 -0500

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: Bypass caps

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

At 01:00 PM 10/7/98 PDT, you wrote:

>I was courious about bypass caps, What do they do? will three 10kv

>.001uf caps be suitable for 9kv input?

>

Philip,

Bypass caps are meant to go from the HV outputs of your

neon to RF ground. The theory is that any RF energy will be

shunted to ground rather than into your neon secondary.

Your caps are probably rated 10KV DC so will not stand up

to 9kv AC. If you have 6 of them I would try 3 in series

on each side. This will give you 333 pf and 30kv DC.

Personally I wouldnt bother with bypass caps. A properly

set safety gap is far more important. Sometimes bypass

caps can create problems.

Neon protection circuits is a highly debated topic with

no clearly defined answers. Everyone has their own pet

ideas and theories. My take on the subject is that neons

are fragile and we are using them in a hostile environment

and that some are going to fail no matter what we do to

protect them.

I believe more neons are lost from overvoltage due to excessively

wide gaps that for any other reason.

The only sure way to prevent neon loss is to become a neon

rebuilder. Unpot and rebuild those bad neons and you can make

them practically indestructible.

TTYL!

Mike Hammer

mhammer@misslink.net

From:          "Malcolm Watts" <MALCOLM@directorate.wnp.ac.nz>

Organization:  Wellington Polytechnic, NZ

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Date:          Thu, 8 Oct 1998 09:57:27 +1200

Subject:       RE: Pigs and other barnyard experiments.

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Hi Bill,

> From:           "Bill the arcstarter" <arcstarter@hotmail.com>

> To:             "Tesla-2 Mailing List" <tesla-2@emachine.com>

> Subject:        RE: Pigs and other barnyard experiments.

> Date sent:      Tue, 06 Oct 1998 19:38:50 PDT

> Send reply to:  "Tesla-2 Mailing List" <tesla-2@emachine.com>

> Hi Malcolm.

>  You wrote:

> >> But - you say you measured 31.5 mH with an open secondary.  If you do

> >> the math - a 0.035 H inductor should draw 120/(2*PI*60*0.035) = 9 amps.

> >> What gives?  What kind of LCR meter did you use?

> >

> >That is probably true. I've examined the magnetizing current for a

> >240V MOT on the scope and discovered the waveform was highly

> >distorted and peaking at 20A!! when run from a 240V supply. RMS

> >current was lower of course. In fact when looking at what cores can

> >stand, peak current is the real quantity of interest and it is worth

> >remembering that Vpk from the mains is SQRT(2) higher than the RMS

> >value.

>

> The sqrt(2) ratio is only true for a nondistorted sine wave.  Other

> waveforms will have other converstions from peak to RMS, of course.

> A small nit!

The mains *voltage* waveform hopefully is undistorted. Not always

true in some installations though. Due to the proliferation of

computer power supplies with their capacitor input filters in this

establishment, the mains here is actually flat topped reflecting the

spikes of current drawn near Vpk to recharge the 200uF or so in each

supply :(

Cheers,

Malcolm

X-Originating-IP: [199.217.140.22]

From: "Bill the arcstarter" <arcstarter@hotmail.com>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: RE: Pigs and other barnyard experiments.

Date: Thu, 08 Oct 1998 08:33:24 PDT

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

Hi Malcolm, list,

>> The sqrt(2) ratio is only true for a nondistorted sine wave.  Other

>> waveforms will have other converstions from peak to RMS, of course.

>> A small nit!

>

>The mains *voltage* waveform hopefully is undistorted. Not always

>true in some installations though. Due to the proliferation of

>computer power supplies with their capacitor input filters in this

>establishment, the mains here is actually flat topped reflecting the

>spikes of current drawn near Vpk to recharge the 200uF or so in each

>supply :(

Hmm.  Are the (supposedly) power factor corrected switchers still guilty

of this?

-Bill

______________________________________________________

Get Your Private, Free Email at http://www.hotmail.com

X-Originating-IP: [195.121.16.68]

From: "Sam Barros" <sambarros@hotmail.com>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Important questions...

Date: Thu, 08 Oct 1998 11:53:10 PDT

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

>Sam, please be extremely careful working with microwave radiation! The

>radiated energy can very quickly cause blindness by creating cataracts

>in your eyes (sort of like cooking an egg)! High power microwave

>radiation can induce other undesirable physiological and mental

effects.

>The particular frequency used in ovens was chosen to maximize the

>heating effect on water molecules... the type that make up over 75% of

>your body and brain. Don't inadvertantly cook them...

>

>-- Bert --

Bert,

Before starting my research on microwaves I did an extensive research

on the field. Belive me, I am fully aware of all effects exposure to

microwaves can have and I know exactly how it can cook you down to an

inch at the same time is only warms your skin. I also knew that the

wires might radiate some energy and I took some safety measyres against

it (standing back). What I didn't expect was that the energy picked up

from the cooking cavity and radiated would be so big... Well, like I

say: Learn from your mistakes and you'll never forget the lesson!

Sam Barros.

X-Originating-IP: [195.121.16.68]

From: "Sam Barros" <sambarros@hotmail.com>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Sulfur uWave lamp

Date: Thu, 08 Oct 1998 11:56:08 PDT

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

You want to see microwave-induced light? Well, put a neon light inside

your oven and turn it on! It'll glow to 1000W and then explode...

Sam Barros.

P.S. Please try to keep the TESLA LIST for TESLA COIL experiments. Mail

this kind of stuff privately

From:          "Malcolm Watts" <MALCOLM@directorate.wnp.ac.nz>

Organization:  Wellington Polytechnic, NZ

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Date:          Fri, 9 Oct 1998 09:28:56 +1200

Subject:       RE: Pigs and other barnyard experiments.

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Hi Bill,

> From:           "Bill the arcstarter" <arcstarter@hotmail.com>

> To:             "Tesla-2 Mailing List" <tesla-2@emachine.com>

> Subject:        RE: Pigs and other barnyard experiments.

> Date sent:      Thu, 08 Oct 1998 08:33:24 PDT

> Send reply to:  "Tesla-2 Mailing List" <tesla-2@emachine.com>

> Hi Malcolm, list,

>

> >> The sqrt(2) ratio is only true for a nondistorted sine wave.  Other

> >> waveforms will have other converstions from peak to RMS, of course.

> >>A small nit!

> >

> >The mains *voltage* waveform hopefully is undistorted. Not always

> >true in some installations though. Due to the proliferation of

> >computer power supplies with their capacitor input filters in this

> >establishment, the mains here is actually flat topped reflecting the

> >spikes of current drawn near Vpk to recharge the 200uF or so in each

> >supply :(

>

> Hmm.  Are the (supposedly) power factor corrected switchers still guilty

> of this?

No. This is the problem they eliminate. The input from the bridge

rectifier goes straight to an inductor switched at high frequencies.

There is no input filter cap. Current draw from the mains is

continuous and is a function of average DC output current multiplied

by the sinusoidal input. This has the effect of making any load

attached to the output appear resistive at the input. Power factor

can easily be corrected to 95%+ using this technique.

Malcolm

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Date: Thu, 8 Oct 1998 22:34:17 EDT

Subject: Re: Salt Water Caps  Way better design?

X-Mailer: IPAD 2.0

From: alfred.skrocki.sr@juno.com (Alfred A Skrocki)

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

On Tue, 06 Oct 1998 23:10:36 EDT Kyle A. Break

<break7@juno.com> wrote:

>	To anyone that knows, Could you use plastic bottles like 2

>liter soda bottles? This would definitely get you away from glass.

All the soda bottles I've seen are made of mylar which is fantastic for

D.C. but stinks for R.F. A.C.! It turns out that mylar has a high power

factor, read that as high absorption of R.F.and conversion to heat

losses.

>Milk and orange juice bottles are HDPE, but have irregular shapes, or is

that a problem? >Might be a good idea (cheap) in Holland?

The irregular shapes will cause concentrations of electrons at the small

radii and provoke breakdown. If your so inclined, you may want to play

with grinding up the old milk jugs and heating and pressing the

pulverized polyethylene into nice thin sheets for making caps out of. It

will work, that is how polyethylene is recycled in the U.S., BUT it will

take some experimenting to get the right combination of temperature and

pressure.

Alfred A. Skrocki

Alfred.Skrocki.Sr@JUNO.COM

Visit my Do-It-Yourself Aquarium WEB page at:

http://WWW.GeoCities.com/CapeCanaveral/6251

From:          "Malcolm Watts" <MALCOLM@directorate.wnp.ac.nz>

Organization:  Wellington Polytechnic, NZ

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Date:          Fri, 9 Oct 1998 17:02:59 +1200

Subject:       Re: Salt Water Caps  Way better design?

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Well, there is one possibility. I have put it out for interest a

couple of times before on other lists. If you know someone who uses a

Minolta photocopier, grab the toner bottles. They are thick HDPE.  I

reckon they're a good bet. I recall getting about 250pF from a 350

gram bottle.

Malcolm

X-Mailer: IPAD 2.0

Date: Fri, 09 Oct 1998 08:58:55 +0200

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Marco Denicolai <marco@vistacom.fi>

Subject: Double dielectric capacitors?

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

To help to remove air from my rolled capacitor (to be built) I was thinking to

use paper and PE as dielectric. That is

---- Al

---- paper

---- PE

---- Al

---- paper

---- PE

Now the difficult question: if I use as dielectric two materials together, say:

- material A, thickness tA, dielectric constant kA, dielectrict strength vA

- material B, thickness tB, dielectric constant kB, dielectrict strength vB

what is going to be the resulting dielectric constant? And the dielectric

strength?

Marco Denicolai                   Vista Communication Instruments, Inc.

Hardware Development Manager      www.vistacom.fi

marco@vistacom.fi

From: Hollmike@aol.com

Date: Fri, 9 Oct 1998 08:12:41 EDT

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Double dielectric capacitors?

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Marco,

If I remember correctly,  the dielectric strength will be additive of all

the layers.  The dielectric constant will be the weighted average of the

different materials.  In other words, take each layers thickness divided by

the total thickness times its dielectric constant.  If the paper and the poly

were of equal thickness, then the resultant constant would simply be the

average of the individual contansts.   If the poly  is twice as thick as the

paper, then the resultant constant would be two thirds times the poly's

constant plus one third times the paper's constant.

I hope I remember this correctly.

Mike Hollingsworth

X-Originating-IP: [199.217.140.22]

From: "Bill the arcstarter" <arcstarter@hotmail.com>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Double dielectric capacitors?

Date: Fri, 09 Oct 1998 06:59:19 PDT

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

Marco wrote:

>To help to remove air from my rolled capacitor (to be built) I was thinking to

>use paper and PE as dielectric. That is

>

>---- Al

>---- paper

>---- PE

>---- Al

>---- paper

>---- PE

>

>Now the difficult question: if I use as dielectric two materials together, say:

>

>- material A, thickness tA, dielectric constant kA, dielectric strength vA

>- material B, thickness tB, dielectric constant kB, dielectric strength vB

>

>what is going to be the resulting dielectric constant? And the dielectric

>strength?

OK - about the net dielectric constant of a cap made from two parallel

sheets of different dielectric - there is an easy way to visualize what

is going on.  This will lead to a simple solution...

(nothing up my sleeve...)

Here is our capacitor: ---|ab|---  where 'a' is dielectric #1 and 'b' is

dielectric #2.

An equivalent circuit is: ---|a|-----|b|---  which is equivalent to two

caps in series.

Now the problem is how to compute the net capacitance of two caps in

series, which is an easy problem:  Cnet = (C1 * C2)/(C1 + C2) where C1 =

capacitance of the first capacitor, C2 = capacitance of the 2nd

capacitor, etc.

Next - if you make some assumptions such as equivalent areas and

thicknesses, then the effective dielectric constant of two dielectrics

in series is: Keff = a*b/(a+b) where a,b are the dielectric constants of

each material.

Finally, if thicknesses aren't identical, it is easiest to work this

problem as two caps in series.  If you work the math, you will discover

the net capacitance will be:

C=0.224 *A * a*b/(a*y+b*x)

where:

C= capacitance in pf,

A is area in square inches

a = dielectric const of dielectric #1

x = thickness (inches) of dielectric #1

b = dielectric constant of dielectric #2

y = thickness (inches) of dielectric #2

... and if I someone works the math and discovers I've made an error -

please tell me and the list about it! :)

-Bill

From: DavidAllan@aol.com

Date: Fri, 9 Oct 1998 14:11:55 EDT

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Salt Water Caps  Way better design?

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

If one has access to polyethylene (or polystyrene) bottles, what geometric

characteristics (height, diameter, wall thickness) would one want to have in

such a bottle?

David Allan

In a message dated 10/8/98 11:55:47 PM Eastern Daylight Time,

alfred.skrocki.sr@juno.com writes:

> Subj:	 Re: Salt Water Caps  Way better design?

>  Date:	10/8/98 11:55:47 PM Eastern Daylight Time

>  From:	alfred.skrocki.sr@juno.com (Alfred A Skrocki)

>  Sender:	tesl-err@emachine.com

>  Reply-to:	tesla-2@emachine.com (Tesla-2 Mailing List)

>  To:	tesla-2@emachine.com (Tesla-2 Mailing List)

>

>  On Tue, 06 Oct 1998 23:10:36 EDT Kyle A. Break

>  <break7@juno.com> wrote:

>

>  >	To anyone that knows, Could you use plastic bottles like 2

>  >liter soda bottles? This would definitely get you away from glass.

>

>  All the soda bottles I've seen are made of mylar which is fantastic for

>  D.C. but stinks for R.F. A.C.! It turns out that mylar has a high power

>  factor, read that as high absorption of R.F.and conversion to heat

>  losses.

>

>  >Milk and orange juice bottles are HDPE, but have irregular shapes, or is

>  >that a problem? Might be a good idea (cheap) in Holland?

>

>  The irregular shapes will cause concentrations of electrons at the small

>  radii and provoke breakdown. If your so inclined, you may want to play

>  with grinding up the old milk jugs and heating and pressing the

>  pulverized polyethylene into nice thin sheets for making caps out of. It

>  will work, that is how polyethylene is recycled in the U.S., BUT it will

>  take some experimenting to get the right combination of temperature and

>  pressure.

>

>                                 Alfred A. Skrocki

>                            Alfred.Skrocki.Sr@JUNO.COM

Date: Fri, 9 Oct 1998 19:43:43 +0100

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Stephen Bell <steve@breakfix.demon.co.uk>

Subject: Foil plate caps

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Hi Guys,

I'm in the process of making a stacked plate poly capacitor,

using 6 mil polythene and aluminum foil. I was initially going to cut

the plates as a square with a tab (see Common plate shape below) but

thought that a lot of material would be wasted ( 'X' shaded area in

diagram). I then came up with a design in which the foil is folded on

top of itself to form the tab.

With an active area of 12x12 inches and a 2 inch long tab, I

waste 8 instead of 22 sq inches per plate!

I think it will work OK but I wanted you guys to give it a

second opinion.

Common plate shape:

I--I            XXXXXXX----

I  I            XXXXXXX   I

I  I            XXXXXXX   I

--------  I            XXXXXXX   I

I         I            I         I

I         I            I         I

I         I            I         I

I         I            I         I

I         I            I         I

I         I            I         I

-----------            -----------

(cut out plate)        (sheet before plate cut out; X=wasted material)

Proposed plate shape:

I--I

I  I

I  I

----------I  I       -----------

I         I  I       I         I

I         I---       I         I

I         I          I         I

I         I          I         I

I         I          I       XXX

I         I          I       XXX

-----------          --------XXX

TIA.

------------------------------------------------

Stephen Bell        (steve@breakfix.demon.co.uk)

Stuff also available at www.breakfix.demon.co.uk

------------------------------------------------

X-Originating-IP: [195.121.16.4]

From: "Sam Barros" <sambarros@hotmail.com>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Help needed for primary...

Date: Fri, 09 Oct 1998 12:29:43 PDT

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

... Ok... It's me again bothering you people about my coil, in my

non-stop quest for maximum efficiency...

Now, this time I have two relatively simple questions: One is: How

close can I make the turn to turn spacing on the primary coil? I

currently have one-inch primary/secondary clearance, and the primary is

wound 18 turns of 8mm 0.32inches?) Thick copper tubing with an

8mm(0.32in?)  Turn to turn spacing. Now, since the first and the 15/16th

turn will be connected to the primary circuit there shouldn't really be

a large turn to turn potential. So, in theory, if I epoxy the tubing, I

could make the turn to turn spacing 1mm(0.04in?) or less. What effects

would it have on efficiency? Would it make the primary more likely to

overcouple, what exactly would it do?

I am asking this because with a 20cm diameter secondary, my current

design will have almost 1-meter outer diameter. Now, That is way too

much, and I want to explore all possibilities in order to make my

primary both smaller and more efficient. If making it smaller will

compromise efficiency, than I'll just leave it as it is, but otherwise I

want it smaller!

Now, onto the second question:

I see most people make their secondary coil forms out of plywood. I was

even suggested to do so. But from what I have sing trough

experimentation, Plywood will conduct electricity fairly well. Even when

DRY!!!

I can see huge amounts of corona going right trough the plywood with as

little as 25KV.

Will drying with a heatgun and varnishing solve this problem, or shall

I glue a 2mm thick piece of Plexiglas to the top and to both sides of

the supports of my coilform?

Now, the next one is a bit more technical:

How do you calculate the voltage of a coil trough the sparklength. For

example: If my coil makes 1 and a half meter (5foot?) long sparks.

What's the voltage output? And, How many watts do I need per meter of

spark?

Oh yeah, so, if a coil puts out 1million volts, what would the amperage

be? I suppose just a few uA, but than there is also the toroid charge,

so I dont really know how one can calculate it

And now, the most imaginative one: I want to use my coil for

electrostatic repulsion. So Ill have to rectify (make DC) the output.

Will simply stacking several TV 60KV rectifiers in series do the job?

Anyone ever tried this? You think it would work? Would it be nice, would

the voltage be reduced substantially, what would happen?

Sorry for all this questions

And

Thanks a lot guys!!!

Sam Barros.

X-Authentication-Warning: olsen.org: Host [208.14.210.4] claimed to be olsen.org

Date: Fri, 09 Oct 1998 13:36:15 -0600

From: Dan Olsen <dan@olsen.org>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Foil plate caps

Sender: tesl-err@emachine.com

Stephen Bell wrote:

> Hi Guys,

>         I'm in the process of making a stacked plate poly capacitor,

> using 6 mil polythene and aluminium foil. I was initially going to cut

> the plates as a square with a tab (see Common plate shape below) but

> thought that a lot of material would be wasted ( 'X' shaded area in

> diagram). I then came up with a design in which the foil is folded on

> top of itself to form the tab.

>         With an active area of 12x12 inches and a 2 inch long tab, I

> waste 8 instead of 22 sq inches per plate!

>         I think it will work OK but I wanted you guys to give it a

> second opinion.

>

Yea,

That's how I made my rolled poly-flashing cap.  you just need to make sure

that the bent-over part with the edges aren't towards another plate.  This

is possible in a two-plate rolled cap, but I don't know how you would do

it in a stacked cap.... you don't want any rough edges or protrusions from

the plate, even small.

> Common plate shape:

>

>        I--I            XXXXXXX----

>        I  I            XXXXXXX   I

>        I  I            XXXXXXX   I

> --------  I            XXXXXXX   I

> I         I            I         I

> I         I            I         I

> I         I            I         I

> I         I            I         I

> I         I            I         I

> I         I            I         I

> -----------            -----------

> (cut out plate)        (sheet before plate cut out; X=wasted material)

>

> Proposed plate shape:

>

>           I--I

>           I  I

>           I  I

> ----------I  I       -----------

> I         I  I       I         I

> I         I---       I         I

> I         I          I         I

> I         I          I         I

> I         I          I       XXX

> I         I          I       XXX

> -----------          --------XXX

>

> TIA.

>

> ------------------------------------------------

> Stephen Bell        (steve@breakfix.demon.co.uk)

> Stuff also available at www.breakfix.demon.co.uk

> ------------------------------------------------

X-Mailer: IPAD 2.0

Date: Fri, 09 Oct 1998 16:36:56 -0500

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: Foil plate caps

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

At 07:43 PM 10/9/98 +0100, you wrote:

>Hi Guys,

>        I'm in the process of making a stacked plate poly capacitor,

>using 6 mil polythene and aluminum foil. I was initially going to cut

>the plates as a square with a tab (see Common plate shape below) but

>thought that a lot of material would be wasted ( 'X' shaded area in

>diagram). I then came up with a design in which the foil is folded on

>top of itself to form the tab.

>        With an active area of 12x12 inches and a 2 inch long tab, I

>waste 8 instead of 22 sq inches per plate!

>        I think it will work OK but I wanted you guys to give it a

>second opinion.

>

<SNIP

>

>------------------------------------------------

>Stephen Bell        (steve@breakfix.demon.co.uk)

>Stuff also available at www.breakfix.demon.co.uk

>------------------------------------------------

Stephen,

Ive made quite a few caps of both the plate and rolled variety

and the plate caps sure do work well. Well enough to justify

the construction time.

I made my first couple of plate caps using the traditional

plate design with an extended tab. These caps worked well

and still survive and work well today. But you are correct

that cutting the tab involves lots of wasted material and

lots of effort.

My latest design uses no tabs at all. Merely allow each alternate

sheet to overhang the poly by a couple inches. Instead of just an

extended tab make the whole end of the plate one long tab.

Do the same with the plates on the other end.

Instead of having the tabs come out the top have the extended

plates come out each side. Each plate would measure 14 X 12 inches

instead of just 12 X 12. The extra two inches is offset off each end.

If connecting caps in series (as you should) simply line up

the plate ends of the two caps and clamp them between two strips

of aluminum stock and some small bolts.

This has the advantage of a very low resistance/low inductance

connection and should make an already good cap even better.

Oh and one other thing. I dont recommend using plates as large

as 12 X 12 inches. It will be very difficult to remove trapped

air from plates this large. I had this problem with a plate cap

using 10 X 10 plates. It eventually failed. My new cap will have

only 7 X 7 plates.

I would either use a maximum size of 8 X 8 inches or consider

adding a layer of kraft paper between each poly layer to wick

the oil into those dry areas. The only other alternative is

to build the cap under oil. I did this just one. Never again.

hope this helps

TTYL!

Mike Hammer

mhammer@misslink.net

From: Ashpilot@aol.com

Date: Fri, 9 Oct 1998 19:06:54 EDT

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Foil plate caps

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

I'm building a stacked plate cap now too, I have everything stacked and

connected, it is a series of 5 caps with 8 mils dielectric, sandwiched between

.125" glass plates.

I placed the stack in a heavy duty plastic bag and ran a connection from a

vacuum pump to the stack.  I noticed that pockets of air were still trapped

between layers of dielectric.  My last stacked cap failed due to air pockets.

I have the craft paper and I wasn't worried about the reduction of

capacitance, but how or will the paper effect the breakdown voltage or will

it?  I'm using polypropelene dielectric.

X-Originating-IP: [209.50.121.180]

From: "Bill the arcstarter" <arcstarter@hotmail.com>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Kraft paper capacitors?

Date: Fri, 09 Oct 1998 16:58:39 PDT

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

Coilers,

One thing I've never understood is how using Kraft paper inside your

capacitor supposedly 'gets rid' of the air bubble problem.

Seems to me that paper, being porous and initially filled with air,

would only aggravate the situation.  I also don't see how the paper

could possibly let the air inside a cap escape, etc...

Clue me in please!

-Bill

Date: Fri, 09 Oct 1998 19:05:20 -0500

From: Bert Hickman <bert.hickman@aquila.com>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Organization: Stoneridge Engineering

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Double dielectric capacitors?

Sender: tesl-err@emachine.com

Marco Denicolai wrote:

>

> To help to remove air from my rolled capacitor (to be built) I was thinking to

> use paper and PE as dielectric. That is

>

> ---- Al

> ---- paper

> ---- PE

> ---- Al

> ---- paper

> ---- PE

>

> Now the difficult question: if I use as dielectric two materials together, say:

>

> - material A, thickness tA, dielectric constant kA, dielectrict strength vA

> - material B, thickness tB, dielectric constant kB, dielectrict strength vB

>

> what is going to be the resulting dielectric constant? And the dielectric

> strength?

>

>

>  Marco Denicolai                   Vista Communication Instruments, Inc.

>  Hardware Development Manager      www.vistacom.fi

>

>  marco@vistacom.fi

Marco,

This is an excellent question! The technique of using combinations of

kraft paper and plastic film is used by most extended-foil HV capacitor

vendors. A capacitor made from different dielectric materials will

behave as though it was made up of independent "capacitor elements".

Each capacitor element will correspond to a each type of dielectric

material that appears between the capacitor's plates. These capacitor

elements then act as though they are connected in series, as in the

diagram below. By solving for the capacitance contributions of the paper

and PE elements, we can then answers your questions. Fortunately, you

didn't ask about estimating dielectric losses this is a tougher

question to answer in a multiple-dielectric system. :^)

Each capacitor element will have the same plate area (A). However, each

dielectric may have different thickness, number of sheets, and

dielectric constant. We can also make a simplifying assumption that the

mineral oil will thoroughly wet the plastic, filling any voids/spaces

between the Al and PE, and will thoroughly saturate the paper. Once

we've calculated the individual element capacitance, we can then

estimate the voltages and  dielectric stresses seen by each dielectric.

The capacitor's output terminals are connected to V1 and V3, while V2 is

an internal voltage representing how the external voltage is distributed

between the two dielectric systems.

V1       ||    V2     ||      V3

o-------||-----------||-------o

||           ||

Cpaper         Cpe

Depending upon the raw materials and fabrication process, paper can have

widely varying electrical characteristics. Let's assume you'll be using

a good grade of kraft paper. In this case, the mineral-oil-soaked

dielectric constant will range from about 3 (for low density paper) to 4

(for high density paper). You can probably assume a "medium density"

paper with an effective k of about 3.5 and be in the right ballpark.

The general equation for a capacitor is:

C = 0.2248*k*A*(N-1)/(d*x)  (in pF)

where k = the "effective" value for the mineral oil "wetted" dielectric

(2.0 for PE, 3.5 for paper)

A = Effective plate overlap area (sq. inches)

N = Number of conductive plates in the cap. Use 3 for a rolled cap

d = dielectric film thickness/sheet for a given dielectric

(inches)

x = number of dielectric sheets between plates for a given

dielectric

If we use the parameters that correspond to the individual dielectric

materials, we can solve this equation to estimate the individual

capacitor elements, Cpaper and Cpe. The total effective capacitance (Ct)

seen at the external terminals (V1 - V3) will be:

Ct = Cpaper*Cpe/(Cpaper + Cpe)

If we now apply an AC voltage to the external leads of the capacitor,

the voltage developed across each capacitor element can be estimated:

Vpaper = Vapplied*(Ct)/Cpaper

And

Vpe    = Vapplied*Ct/Cpe

The dielectric stress in volts/mil can then be found be simply dividing

the above voltages by the respective total dielectric thickness for each

dielectric material.

V/Mil (paper) = Vpaper/(d*x*1000)

Where d and x are the per-sheet thickness in inches and number of total

sheets of paper between a pair of capacitor plates

Volts/Mil (PE) = Vpe/(D*x*1000)

Where d and x are the per-sheet thickness in inches and number of total

sheets of Polyethylene between a pair of capacitor plates

Let's try an example:

====================

Assume A = 1000 square inches, and let's use 2 sheets of 30 mil PE,

separated by 2 mil thick kraft paper between ALL interfaces to insure

complete oil wicking. Between each set of Al plates, we'll have 3 sheets

of kraft paper, and 2 sheets of PE. The structure (before rolling) would

look like:

---- Al

---- paper

---- PE

---- paper

---- PE

---- paper

---- Al

---- paper

---- PE

---- paper

---- PE

---- paper

Since we're making a rolled cap, we'll set N=3 in the above equations,

and we'll let "k" be 2.0 and 3.5 for the PE and paper respectively.

Finally, let's assume that we need to withstand a 35 kV swing once the

main gap fires.

Solving, we get:

Cpaper = 262267 pF  (x = 3, d = 0.002", N = 3, A = 1000, k = 3.5)

Cpe =  14987 pF  (x = 2, d = 0.030", N = 3, A = 1000, k = 2.0)

Ct =  14177 pF  (the effective terminal-terminal capacitance)

AND:

Vpaper =   1892 Volts

Vpe    =  33108 Volts

The volts/mil stress for each dielectric system will be:

V/Mil (paper) = 315 Volts/Mil

V/Mil (PE)    = 552 Volts/Mil

Hope this does the trick for your (future) designs!

-- Bert --

Date: Fri, 09 Oct 1998 17:10:18 -0700

From: Gene Waldo <gwaldo@1stnetusa.com>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Air in flat pl caps

Sender: tesl-err@emachine.com

All,

My as yet unanswered question is: having a stabalized and perhaps

'lightly' clamped stack, immersed and on an edge, would the cyclic

application of, say, 6 or 12 vdc (at about 12 cpm (.2 Hz?)) induce

sufficient 'clench-pumping' action to help in physically driving

entrapped air up and out?

One should have the means of tightly clamping the stack before

immersion, and then 'relaxing' it slightly under the oil.

Would the stack need "relaxing" (shorting) between the applications of

dcv? Actually I suppose there might be some sort of a jolt here!

Some small motor and simple rotating contacts might do this job

effecively and automatically? You could go out to eat and a movie while

your cap self-evacuates.

The addition of a vacuum over the oil would be a big help but seems the

more complex of the options.

Just be sure to reset the physical clamping before operation. If that

action expells more air, do it all again!

Just $0.02 worth of more stuff to ponder upon.

----------------------------------------------

Gene Waldo -  gwaldo@1stnetusa.com  - "Igor in Residence"

---------------------------------------------------------------

What's good for the planet is more OZONE! See the machinery form up!

NEW PAGE!   http://members.1stnetusa.com/danmurphy/waldo.htm

Date: Fri, 09 Oct 1998 21:48:56 -0500

From: Bert Hickman <bert.hickman@aquila.com>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Organization: Stoneridge Engineering

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Air in flat pl caps

Sender: tesl-err@emachine.com

Gene Waldo wrote:

>

> All,

> My as yet unanswered question is: having a stabalized and perhaps

> 'lightly' clamped stack, immersed and on an edge, would the cyclic

> application of, say, 6 or 12 vdc (at about 12 cpm (.2 Hz?)) induce

> sufficient 'clench-pumping' action to help in physically driving

> entrapped air up and out?

>

> One should have the means of tightly clamping the stack before

> immersion, and then 'relaxing' it slightly under the oil.

>

> Would the stack need "relaxing" (shorting) between the applications of

> dcv? Actually I suppose there might be some sort of a jolt here!

>

> Some small motor and simple rotating contacts might do this job

> effecively and automatically? You could go out to eat and a movie while

> your cap self-evacuates.

>

> The addition of a vacuum over the oil would be a big help but seems the

> more complex of the options.

>

> Just be sure to reset the physical clamping before operation. If that

> action expells more air, do it all again!

>

> Just $0.02 worth of more stuff to ponder upon.

>

> ----------------------------------------------

> Gene Waldo -  gwaldo@1stnetusa.com  - "Igor in Residence"

Gene and all,

Conceptually right, but it takes considerably higher voltages in order

to get sufficient electrostatic forces working. And you're also right

about a "jolt" being more effective in working bubbles out. One way was

to actually use the cap in Tesla Coil service, but with a lower main-gap

setting (at reduced voltage) so that the cap can be broken in without

prematurely overstressing it. The rapid charging and disruptive

discharges are quite effective for working out entrapped air as long as

the air can rise through the oil. I used this approach over a period of

2 weeks on my flat plate caps with no vacuum at all - worked great!.

-- Bert --

From: Ashpilot@aol.com

Date: Fri, 9 Oct 1998 23:19:50 EDT

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Kraft paper capacitors?

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

I believe that the paper is supposed to prevent an airtight seal

between layers of poly.  Then it will act as a wick and aid in

pulling oil in between the sheets of dielelctric

Ed Mullin  Coilin' in NH

From: Ashpilot@aol.com

Date: Fri, 9 Oct 1998 23:35:30 EDT

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Air in flat pl caps

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

What I just tried was to Vacuum bag my stack of plates.  I had a heavy duty

poly bag that I sealed the stack in, and ran a connection to a vacuum pump.

ATM pressure compressed the stack and helped push the air out.  Being curious

when I took the stack out of the bag after about 10 mins of pumping I tried to

peel back part of the top layer. It came apart with some difficulty and seemed

to be almost sealed together.  I'm hopeing to be able to pump down the whole

thing inside the casing with some oil to try to jar loose any bubbles.

X-Mailer: IPAD 2.0

Date: Sat, 10 Oct 1998 11:25:17 +0100

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: bob golding <yubba@clara.net>

Subject: Re: Kraft paper capacitors?

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

At 23:19 09/10/98 EDT, you wrote:

>I believe that the paper is supposed to prevent an airtight seal between

>layers of poly. Then it will act as a wick and aid in pulling oil in

>between the sheets of dielectric

Hi all,

can someone tell me if Kraft paper goes under any other names? Is it

called Kraft paper in the UK? grease proof maybe?

Would silicone paper work. You know the stuff you use for cooking.

many thanks

bob golding

yubba@clara.net

Date: Sat, 10 Oct 1998 11:39:07 +0100

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Stephen Bell <steve@breakfix.demon.co.uk>

Subject: Earthing control unit

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Hi Guys,

After putting in a couple of days of concentrated work, with

lots of drilling, cutting, filing, welding and soldering, my control

unit for the coil is almost finished. I've hit a snag though! How should

it be earthed?

The chassis supporting all the components is steel and therefore

needs to be earthed for operator safety. However, I recall reading some

thing in the list archives about using the dedicated RF ground, not the

domestic outlet ground, to keep interference out of the house wiring.

If the chassis was connected to domestic ground and a live wire

in the unit shorted to the case, the 13A inlet fuse would blow. Would

the same thing happen if I used the RF ground instead (2x4ft earth rods

with 40ft of 6inch wide copper sheet burried in a radiating pattern)?

How about using an RCD on the inlet?

TIA.

------------------------------------------------

Stephen Bell        (steve@breakfix.demon.co.uk)

Bringing Tesla Excitement to Swindon, England

the land of 240VAC @ 50Hz

Stuff also available at www.breakfix.demon.co.uk

------------------------------------------------

From: Hollmike@aol.com

Date: Sat, 10 Oct 1998 07:36:39 EDT

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Salt Water Caps  Way better design?

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

In a message dated 10/9/98 12:14:12 PM Mountain Daylight Time,

DavidAllan@aol.com writes:

> If one has access to polyethylene (or polystyrene) bottles, what geometric

> characteristics (height, diameter, wall thickness) would one want to have in

> such a  bottle?

>

>  David Allan

>

I would guess that tall cylindrical bottles would be best due to the fact

that the surface area per unit volume goes up as the radius goes down.  This

of course means more bottles with more connections to make, so a compromise

seems in order.  Rectangular bottles would have the disadvantage of high

charge density near the corners and may cause the voltage stress to be too

high there.

The wall thickness has to be able to handle the voltage the same with

rolled or plate caps. Of course, one can put banks of these in series to

handle the voltage.

Hope this helps

Mike Hollingsworth

Date: Sat, 10 Oct 1998 09:48:37 -0500

From: Bert Hickman <bert.hickman@aquila.com>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Organization: Stoneridge Engineering

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Earthing control unit

Sender: tesl-err@emachine.com

Stephen Bell wrote:

>

> Hi Guys,

>         After putting in a couple of days of concentrated work, with

> lots of drilling, cutting, filing, welding and soldering, my control

> unit for the coil is almost finished. I've hit a snag though! How should

> it be earthed?

>         The chassis supporting all the components is steel and therefore

> needs to be earthed for operator safety. However, I recall reading some

> thing in the list archives about using the dedicated RF ground, not the

> domestic outlet ground, to keep interference out of the house wiring.

>         If the chassis was connected to domestic ground and a live wire

> in the unit shorted to the case, the 13A inlet fuse would blow. Would

> the same thing happen if I used the RF ground instead (2x4ft earth rods

> with 40ft of 6inch wide copper sheet burried in a radiating pattern)?

> How about using an RCD on the inlet?

>

> TIA.

>

> ------------------------------------------------

> Stephen Bell        (steve@breakfix.demon.co.uk)

>  Bringing Tesla Excitement to Swindon, England

>          the land of 240VAC @ 50Hz

> Stuff also available at www.breakfix.demon.co.uk

> ------------------------------------------------

Stephen,

My recommendation is to connect your control panel chassis to the mains

ground from a safety standpoint. Note that the base of your secondary

coil MUST go to a dedicated RF ground, as well as any HV RF bypass and

safety gap grounds. The RF ground of an operating Tesla Coil is highly

energized by the RF current flowing from the base of the secondary, and

under these conditions (particulalry with a less than optimal RF

ground), you may easily see several hundred volts of RF developed at the

resonator base. You don't want to inject this into your mains wiring,

and you cerainly don't want it connected to your control panel.  :^)

As as you indicate, connecting your control chassis to the mains ground

will also insure that you always have a low impedance path to ground in

case you have an equipment failure.

-- Bert --

Date: Sat, 10 Oct 1998 09:52:51 -0500

From: Bert Hickman <bert.hickman@aquila.com>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Organization: Stoneridge Engineering

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Kraft paper capacitors?

Sender: tesl-err@emachine.com

bob golding wrote:

>

> At 23:19 09/10/98 EDT, you wrote:

> >I believe that the paper is supposed to prevent an airtight seal between

> >layers of poly. Then it will act as a wick and aid in pulling oil in between

> >the sheets of dielectric

>

> Hi all,

>         can someone tell me if Kraft paper goes under any other names? Is it

> called Kraft paper in the UK? grease proof maybe?

> Would silicone paper work. You know the stuff you use for cooking.

>

> many thanks

>

> bob golding

>

>  yubba@clara.net

Bob,

Its similar to the stuff that unbleached paper bags are made from.

Office supply stores in the USA sell it in rolls that are about 3' wide.

It should be of uniform thickness and color, unbleached, and should be

fairly absorbant (certainly not waxed...). Stay away from silicone,

since this would may not "wet" very well.

-- Bert --

Date: Sat, 10 Oct 1998 10:20:34 -0500

From: Bert Hickman <bert.hickman@aquila.com>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Organization: Stoneridge Engineering

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Kraft paper capacitors?

Sender: tesl-err@emachine.com

Bill the arcstarter wrote:

>

> Coilers,

> One thing I've never understood is how using Kraft paper inside your

> capacitor supposedly 'gets rid' of the air bubble problem.

>

>  Seems to me that paper, being porous and initially filled with air,

> would only aggravate the situation.  I also don't see how the paper

> could possibly let the air inside a cap escape, etc...

>

>  Clue me in please!

>

> -Bill

>

Bill,

Yeah... it's not really obvious, is it! I suspect it may be the

combination of capillary action, and that it prevents the smooth platic

layers from forming an air-tight seal. The kind of Kraft paper used in

commmercial capacitors is quite thin (bees wings, if I remember? :^))

and quite dense.

Interestingly enough, it's not essential that all air be removed, only

that any entrapped air never reach a stress level sufficient for

ionization. Very small bubbles tend to be much less of a problem since

the breakdown strength of the air begins to climb as you go to the left

side of the minimum "knee" in the breakdown curve (Paschen's Law) for

smaller airgaps. As long as these small bubbles are prevented from

joining to form larger bubbles, they won't begin the destructive cycle.

Small air bubbles trapped between fibers in the kraft paper will see a

lower e-field stress since the overall volts/mil stress is less for this

portion of the dielectric system. The key appears to be eliminating

large bubbles, and keeping the small ones small enough to avoid internal

partial discharge breakdown.

Whatever the mechanisms, the technique apparently works!

-- Bert --

From: Dsurfr@aol.com

Date: Sat, 10 Oct 1998 11:46:23 EDT

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Help needed for primary...

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Sam: Malcolm Watts has mentioned a design for a "stacked" primary that I'm

going to try myself. It is a flat or pancake shape, half the diameter of a

single primary, that returns back on itself. He uses a continuous piece of

tubing, winds it out & then winds it back in, separating the two coils with a

piece of plexi. He says this shape gives about 70% more inductance, uses the

same amount of tubing & is half as wide. This might be of some help to you &

I'm sure he would offer further advice on it if you posted a request.  JIm

From: McFalc0n@aol.com

Date: Sat, 10 Oct 1998 15:19:56 EDT

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Humidity?

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Does anyone know of humidity's affect on air dielectric strength.  I live in

Houston, TX.  The humidity is extremely high (if you move here, be prepared

for hell for about three months).  How much does humidity affect spark

length?

Thanx.

Date: Sat, 10 Oct 1998 19:08:18 -0500

From: Bert Hickman <bert.hickman@aquila.com>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Organization: Stoneridge Engineering

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Humidity?

Sender: tesl-err@emachine.com

McFalc0n@aol.com wrote:

>

>         Does anyone know of humidity's affect on air dielectric strength.  I live in

> Houston, TX.  The humidity is extremely high (if you move here, be prepared

> for hell for about three months).  How much does humidity affect spark

> length?

> Thanx.

Humidity alone doesn't affect the air's dielectric strength very much.

However, depending upon the materials used to construct your

secondary, high humidity can have a significant effect on coil

performance by increasing the amount of leakage current current. I've

also found that I sometimes get surface flashovers on the PVC used for

the cylindrical static sparkgap under conditions of high humidity. In

general, my coil runs the best in cold/winter weather. It's on a

screened in, unheated, patio.

At one time we had a fellow on the other list that could run his coil

inside an air conditioned room, but the coil failed miserably when taken

outside - he lived in a temperate rainforest area!

-- Bert --

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: "Coiler" <mycroft@access1.net>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Bypass Caps?

Date: Sat, 10 Oct 1998 19:38:52 -0700

X-Mailer: IPAD 2.0

Importance: Normal

X-MimeOLE: Produced By Microsoft MimeOLE V4.72.3155.0

Sender: tesl-err@emachine.com

Once again, I seek the knowledge of the experienced pig herders:)

Thanks for the previous feedback, It turns out I was being *way* too

conservative with my capacitor safety gap. It is now set to about 1/2",

and only fires occasionally.

Also I have reset my safety gap to about the same.

The question: With my pig, do I really need the LCR filter system

I used to use for my neons? Currently, the LR is in the circut, but

I had no good way to put the C in.. and am wondering if I really need

it (or any of it) anyway.

Oh.. and a side note: I got thrown out of the garage. 50" is about 20 more

than my wife wants inside :)  I think she was worried when she saw the

strikes the garage door.

Date: Sat, 10 Oct 1998 22:17:48 -0500

From: Bert Hickman <bert.hickman@aquila.com>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Organization: Stoneridge Engineering

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Bypass Caps?

Sender: tesl-err@emachine.com

Coiler wrote:

>

> Once again, I seek the knowledge of the experienced pig herders:)

>

> Thanks for the previous feedback, It turns out I was being *way* too

> conservative with my capacitor safety gap. It is now set to about 1/2",

> and only fires occasionally.

>

> Also I have reset my safety gap to about the same.

>

> The question: With my pig, do I really need the LCR filter system

> I used to use for my neons? Currently, the LR is in the circut, but

> I had no good way to put the C in.. and am wondering if I really need

> it (or any of it) anyway.

>

> Oh.. and a side note: I got thrown out of the garage. 50" is about 20 more

> than my wife wants inside :)  I think she was worried when she saw the

> strikes the garage door.

Sounding better! With a pig, you really don't need to have the LR or LRC

networks. Leave on the safety gaps to ground though, since a heavy

strike to the primary circuit CAN take out a pig (not very often, but

once is enough if it's yours!).

Good luck doing outside coilin' now! :^)

-- Bert --

From: Hollmike@aol.com

Date: Sun, 11 Oct 1998 08:37:00 EDT

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Help needed for primary...

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

In a message dated 10/10/98 9:48:24 AM Mountain Daylight Time, Dsurfr@aol.com

writes:

> Sam: Malcolm Watts has mentioned a design for a "stacked" primary that I'm

>  going to try myself. It is a flat or pancake shape, half the diameter of a

>  single primary, that returns back on itself. He uses a continuous piece of

>  tubing, winds it out & then winds it back in, separating the two coils with

>  a piece of plexi. He says this shape gives about 70% more inductance, uses the

>  same amount of tubing & is half as wide. This might be of some help to you &

>  I'm sure he would offer further advice on it if you posted a request.  JIm

>

This is interesting.   It would seem like those using center-tap grounded

transformers, like NST's  could use this concept to help prevent primary to

secondary flashovers.  The one difference in what I am thinking compared to

what is described above is that the midpoint of the primary coil where it

"folds back" on itself would be the innermost turn.  Since both ends of the

primary coil are hot when using NST's,  this midpoint should be close to zero

volts and have a lesser tendency to flash over.

It seems as if this would be much more convenient than having the midpoint

of the primary on the outermost turn anyway.  Connecting to the primary coil

near the inner turns would not seem to be very convenient at all.

Comments?

Mike Hollingsworth

Date: Sun, 11 Oct 1998 08:28:19 -0500

From: Bert Hickman <bert.hickman@aquila.com>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Organization: Stoneridge Engineering

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Help needed for primary...

Sender: tesl-err@emachine.com

Hollmike@aol.com wrote:

>

> In a message dated 10/10/98 9:48:24 AM Mountain Daylight Time, Dsurfr@aol.com

> writes:

>

> > Sam: Malcolm Watts has mentioned a design for a "stacked" primary that I'm

> >  going to try myself. It is a flat or pancake shape, half the diameter of a

> >  single primary, that returns back on itself. He uses a continuous piece of

> >  tubing, winds it out & then winds it back in, separating the two coils with a

> >  piece of plexi. He says this shape gives about 70% more inductance, uses the

> >  same amount of tubing & is half as wide. This might be of some help to you &

> >  I'm sure he would offer further advice on it if you posted a request.  JIm

> >

>    This is interesting.   It would seem like those using center-tap grounded

> transformers, like NST's  could use this concept to help prevent primary to

> secondary flashovers.  The one difference in what I am thinking compared to

> what is described above is that the midpoint of the primary coil where it

> "folds back" on itself would be the innermost turn.  Since both ends of the

> primary coil are hot when using NST's,  this midpoint should be close to zero

> volts and have a lesser tendency to flash over.

>     It seems as if this would be much more convenient than having the midpoint

> of the primary on the outermost turn anyway.  Connecting to the primary coil

> near the inner turns would not seem to be very convenient at all.

> Comments?

> Mike Hollingsworth

Mike,

Interesting observation! And this should be true if the number of

primary turns actually used was balanced between the top and the bottom

windings (i.e., you'd need to tap BOTH sides of the primary). Assuming

you've truly balanced the inductances, this point should naturally be

close to 0 volts, and the voltage difference between the center and the

secondary should be minimized.

However, when using NST's, you'd also need to connect the system in a

"equidrive" configuration (a tank cap in series with EACH primary leg)

since otherwise (in a single tank cap system) one end of the primary

would be tied directly to a HV bushing, effectively shorting it to

ground during the charging cycle.

-- Bert --

From: Dsurfr@aol.com

Date: Sun, 11 Oct 1998 15:00:55 EDT

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Help needed for primary...

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Mike: I'm not sure which direction Malcolm wound his coil but what you

mentioned does make sense. If Malcolm reads these posts I'm sure he'll reply.

Jim

From:          "Malcolm Watts" <MALCOLM@directorate.wnp.ac.nz>

Organization:  Wellington Polytechnic, NZ

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Date:          Mon, 12 Oct 1998 10:06:58 +1200

Subject:       Re: Foil plate caps

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Hi all,

on the subject of cap plate design, here is the one I

developed:

_____

/      \

|       |                    plate 2 tab comes out here

|       |                           V

|        \

|         \ ______________

|                          \

|                            \_____________

|                                           \

|                                            |

|                                            |

straight on down to the bottom of the plate.

The idea is to maintain the same creepage distance at all edges of

the plates. Obviously, alternate plates are stacked so that the tabs

come out on opposite sides. Remember that plate-plate clearance

reduces for non-mitred plates where the tabs extend from the stack.

Malcolm

From:          "Malcolm Watts" <MALCOLM@directorate.wnp.ac.nz>

Organization:  Wellington Polytechnic, NZ

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Date:          Mon, 12 Oct 1998 10:16:49 +1200

Subject:       Re: Help needed for primary...

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Slight correction:

The coil has its midpoint closest to the

secondary. It spirals out from the centre on both sides so start and

finish are the outermost turns.  For small coils I tap along one of

the two layers and drive a self-tapping screw into the pipe at the

tap point to connect the cap/gap wire. There is a layer of plexiglass

between the two spiral layers.

An alternative is to wind two separate pancakes and stack them

with a sheet of plexi between them. Connecting them in series becomes

a minor headache though as one must connect the outer of one spiral

to the inner of the other (assuming both spirals are identically

wound).

Malcolm

> From:           Dsurfr@aol.com

> Date sent:      Sat, 10 Oct 1998 11:46:23 EDT

> To:             "Tesla-2 Mailing List" <tesla-2@emachine.com>

> Subject:        Re: Help needed for primary...

> Send reply to:  "Tesla-2 Mailing List" <tesla-2@emachine.com>

> Sam: Malcolm Watts has mentioned a design for a "stacked" primary that I'm

> going to try myself. It is a flat or pancake shape, half the diameter of a

> single primary, that returns back on itself. He uses a continuous piece of

> tubing, winds it out & then winds it back in, separating the two coils with a

> piece of plexi. He says this shape gives about 70% more inductance, uses the

> same amount of tubing & is half as wide. This might be of some help to you &

> I'm sure he would offer further advice on it if you posted a request.  JIm

>

From:          "Malcolm Watts" <MALCOLM@directorate.wnp.ac.nz>

Organization:  Wellington Polytechnic, NZ

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Date:          Mon, 12 Oct 1998 10:19:13 +1200

Subject:       Re: Help needed for primary...

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Hi Mike,

You got it! :)

Malcolm

Date: Sun, 11 Oct 1998 22:03:18 -0500

From: RODERICK MAXWELL <tank@magnolia.net>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: New coil

Sender: tesl-err@emachine.com

I will be testing my new 6" coil this week! Here are the specs.

Secondary form:  rolled polystyrene 6.25" x 34" wound with 21 gauge magnet wire

winding length: 28.25"

coating: epoxy resin, and polyurethane varnish

Primary: flat pancake primary wound with 3/8" copper tubing

spacing between turns: 3/8" ten turns total

Tank capacitance: 56.4nf poly vertical plate capacitor

Power supply: two H&R transformers in antiparallel configuration

rectified by four 15Kv microwave oven diodes in a bridge circuit.

They are rated at 500ma.

Filter: 4700pf ceramic doorknob capacitor rated at 30Kv.

Spark gap: vaccuum gap with 5 gaps total. I will be using a shop vac

to supply the vaccuum.

Misc: safety gaps, filter caps, capacitor gaps, chokes, damping

resistors, etc......

I should have my first low power test within the next couple of days.

I will take    pics, and post them when I get them developed. I don't

want to push it to hard for the next week or two because the capacitor

has to be broken in SLOWLY! It took a long time to build and I would cry

REAL tears if I blew a hole through it!

I'm very curious to see if using a dc power supply instead of the

usual ac will make any difference in performance.

Frankensteins Helper

Max

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Stacked Cap W/ Air  Primary Size

X-Mailer: IPAD 2.0

From: break7@juno.com (Kyle A. Break)

Date: Mon, 12 Oct 1998 02:30:36 EDT

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

To all, I've built several flat stack caps. I use 30-40 sheets of

flashing with single layers of .062 PE. I wrap the stack with 5 zip-ties

not too tight. Then I lay it flat in a Rubber Maid container and pour in

the oil. I've tried energizing with low voltages but seems to need about

1000+ volts to get a "buzz" going. Corona isn't visable till about 7000v.

( not good ). After poly sits in oil for a while it seems to expand? and

get space between layers. I put in the oil before work, when I get home

(~8 hours) I open and compress it with a wooden stick. I push down and

air sqeezes out. I keep doing it like  wringing out a sponge. When air

stops comming out I use it in a TC circuit @ ~ 3500v for a couple 30 sec

blasts then squeeze it again. This removes almost all of the air. To get

the rest you must repeat the process 3-4 more times over several days.

Time seems to be required to allow the poly to spread apart and let air in.

An interesting note, the Rubber Maid container is made of  PP and the

lid is made of LDPE after a while the lid becomes quite unruly and must

be held on with clamps. The lids don't fit well anyway but the oil seems

to worsen the problem.

Primary Size   After running some coils over coupled and seeing the

magnetic field illuminated by corona I think the primary( if it's flat)

must be large to couple the entire secondary. Otherwise it will need to

be saucer shaped or rising helix. Can anyone back this up?

If it's not dangerous it's not fun!  Kyle  break7@juno.com

X-Mailer: IPAD 2.0

Date: Mon, 12 Oct 1998 08:37:29 +0200

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Marco Denicolai <marco@vistacom.fi>

Subject: Re: Double dielectric capacitors?

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

At 06:59 09/10/98 PDT, you wrote:

<SNIP>

>C=0.224 *A * a*b/(a*y+b*x)

>where:

>C= capacitance in pf,

>A is area in square inches

>a = dielectric const of dielectric #1

>x = thickness (inches) of dielectric #1

>b = dielectric constant of dielectric #2

>y = thickness (inches) of dielectric #2

>

>... and if I someone works the math and discovers I've made an error -

>please tell me and the list about it! :)

>

>-Bill

Good! I think you are correct: thank you very much.

Marco Denicolai                   Vista Communication Instruments, Inc.

Hardware Development Manager      www.vistacom.fi

marco@vistacom.fi

X-Mailer: IPAD 2.0

Date: Mon, 12 Oct 1998 08:47:16 +0200

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Marco Denicolai <marco@vistacom.fi>

Subject: Re: Double dielectric capacitors?

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

At 19:05 09/10/98 -0500, you wrote:

<BIG SNIP>

>Solving, we get:

>     Cpaper = 262267 pF  (x = 3, d = 0.002", N = 3, A = 1000, k = 3.5)

>        Cpe =  14987 pF  (x = 2, d = 0.030", N = 3, A = 1000, k = 2.0)

>         Ct =  14177 pF  (the effective terminal-terminal capacitance)

>AND:

>     Vpaper =   1892 Volts

>     Vpe    =  33108 Volts

>

>The volts/mil stress for each dielectric system will be:

>

>     V/Mil (paper) = 315 Volts/Mil

>     V/Mil (PE)    = 552 Volts/Mil

>

>Hope this does the trick for your (future) designs!

>

>-- Bert --

>

Many thanks for your help!

Marco Denicolai                   Vista Communication Instruments, Inc.

Hardware Development Manager      www.vistacom.fi

marco@vistacom.fi

X-Mailer: IPAD 2.0

Date: Mon, 12 Oct 1998 12:14:22 +0200

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Marco Denicolai <marco@vistacom.fi>

Subject: Kraft paper?

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

What kind of paper are you guys using in your rolled capacitors?

I have heard about Kraft paper, but a salesman here in Finland ensured me

that this paper really hardly absorbs oil or any other liquids. So what is

the idea about using kraft paper?

Marco Denicolai                   Vista Communication Instruments, Inc.

Hardware Development Manager      www.vistacom.fi

marco@vistacom.fi

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Date: Mon, 12 Oct 1998 04:32:59 -0700

From: "paul m" <fleischwulf@mailexcite.com>

Cc:

X-Sent-Mail: off

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

Subject: Re: New coil

X-Sender-Ip: 208.153.7.9

Organization: MailExcite  (http://www.mailexcite.com:80)

Sender: tesl-err@emachine.com

Be careful, my 6" coil will produce arc in excess of 70". How much more than that

Im not sure. I can't get an accurate read on it because it always arcs to something.

What are H & R transformers ?

paul m.

---

Einigheit und Wahrheit

On Sun, 11 Oct 1998 22:03:18   RODERICK MAXWELL wrote:

>I will be testing my new 6" coil this week! Here are the specs.

>

>  Secondary form:  rolled polystyrene 6.25" x 34" wound with 21 gauge magnet wire

>   winding length: 28.25"

>   coating: epoxy resin, and polyurethane varnish

>

>  Primary: flat pancake primary wound with 3/8" copper tubing

>   spacing between turns: 3/8" ten turns total

>

>  Tank capacitance: 56.4nf poly vertical plate capacitor

>

>  Power supply: two H&R transformers in antiparallel configuration

>   rectified by four 15Kv microwave oven diodes in a bridge circuit.

>   They are rated at 500ma.

>   filter: 4700pf ceramic doorknob capacitor rated at 30Kv.

>

>  Spark gap: vaccuum gap with 5 gaps total. I will be using a shop vac

>  to supply the vaccuum.

>

>   Misc: safety gaps, filter caps, capacitor gaps, chokes, damping

>   resistors, etc......

>

>

>  I should have my first low power test within the next couple of days.

>I will take    pics, and post them when I get them developed. I don't

>want to push it to hard for the next week or two because the capacitor

>has to be broken in SLOWLY! It took a long time to build and I would cry

>REAL tears if I blew a hole through it!

>  I'm very curious to see if using a dc power supply instead of the

>usual ac will make any difference in performance.

>

>

>

>                                  Frankensteins Helper

>                                          Max

>

Date: Mon, 12 Oct 1998 07:00:55 -0500

From: Bert Hickman <bert.hickman@aquila.com>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Organization: Stoneridge Engineering

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Kraft paper?

Sender: tesl-err@emachine.com

Marco and all,

One of the problems with coiling internationally is that so-called

common materials aren't always common in all parts of the world, and

they're often called by different names! :^)

What you're looking for is relatively dense, natural colored (light

brown, unbleached), thin, uniform thickness, unwaxed (or otherwise

treated) paper. It should have a dull surface finish (not polished or

waxy), and a sample should fairly easily absorb a droplet of mineral oil

placed on its top surface. Try a sample from the salesman you've talked

with - if it meets the above criteria it should work. Another

alternative would be thin manilla paper.

-- Bert --

Marco Denicolai wrote:

>

> What kind of paper are you guys using in your rolled capacitors?

>

> I have heard about Kraft paper, but a salesman here in Finland ensured me

> that this paper really hardly absorbs oil or any other liquids. So what is

> the idea about using kraft paper?

>

>

>  Marco Denicolai                   Vista Communication Instruments, Inc.

>  Hardware Development Manager      www.vistacom.fi

>

>  marco@vistacom.fi

Date: Mon, 12 Oct 1998 07:03:39 -0500

From: Bert Hickman <bert.hickman@aquila.com>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Organization: Stoneridge Engineering

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Stacked Cap W/ Air  Primary Size

Sender: tesl-err@emachine.com

Kyle and all,

Congratulations on the new flat plate caps! LDPE absorbs up to 10% by

weight of mineral oil, swelling significantly in the process.

Polypropylene absorbs  mineral oil as well, but not nearly to the same

degree. One trick to removing air is to orient the stack so that its

vertical so that bouyancy helps remove the air while the cap is being

stressed. However, this approach may require a larger container than

desired. Single sheets of 0.062" will not have the dielectric strength

that two or more thinner sheets of the same total thickness would have.

What voltage are you planning on driving them with?

The blue glow you're seeing is not caused by the magnetic field. It's

actually from the electrostatic (or e-field) ionizing the air between

the toroid and the primary. If you could see the primary's magnetic

field, it would look sort of like a big donut centered around the

primary. The portion of the primary's total magnetic field that will

actually be linked to the secondary (and vice-versa) is called the

coupling coefficient (or "k"). It turns out that, for a variety of

reasons, you'll get best coil performance when "k" is in the range of

0.10 to 0.25.

-- Bert --

Kyle A. Break wrote:

>

>         To all, I've built several flat stack caps. I use 30-40 sheets of

> flashing with single layers of .062 PE. I wrap the stack with 5 zip-ties

> not too tight. Then I lay it flat in a Rubber Maid container and pour in

> the oil. I've tried energizing with low voltages but seems to need about

> 1000+ volts to get a "buzz" going. Corona isn't visable till about 7000v.

> ( not good ). After poly sits in oil for a while it seems to expand? and

> get space between layers. I put in the oil before work, when I get home

> (~8 hours) I open and compress it with a wooden stick. I push down and

> air sqeezes out. I keep doing it like  wringing out a sponge. When air

> stops comming out I use it in a TC circuit @ ~ 3500v for a couple 30 sec

> blasts then squeeze it again. This removes almost all of the air. To get

> the rest you must repeat the process 3-4 more times over several days.

> Time seems to be required to allow the poly to spread apart and let air in.

>  An interesting note, the Rubber Maid container is made of  PP and the

> lid is made of LDPE after a while the lid becomes quite unruly and must

> be held on with clamps. The lids don't fit well anyway but the oil seems

> to worsening the problem.

>

> Primary Size   After running some coils over coupled and seeing the

> magnetic field illuminated by corona I think the primary( if it's flat)

> must be large to couple the entire secondary. Otherwise it will need to

> be sauser shaped or rising helix. Can anyone back this up?

>

>         If it's not dangerous it's not fun!  Kyle  break7@juno.com

>

From: "Thomas McGahee" <tom_mcgahee@sigmais.com>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: KRAFT PAPER

Date: Mon, 12 Oct 1998 09:44:56 -0400

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Marco,

The Kraft paper is not a highly absorbent paper. It doesn't have

to be all that absorbent. It is going to be totally immersed in

oil for the rest of its life! It *does* have to be somewhat

absorbent (no wax paper allowed!) and it must be strong so that

it does not come apart under oil. That eliminates papers such as

butcher's paper and paper towels. The paper should be as flat as

possible. Embossed papers will just give air bubbles a place to

hide.

Besides Kraft paper I have also used various kinds of white

print paper. Stay away from coated papers. The coating is often

a form of fine clay that loosens under oil.

The qualities you are looking for are these:

1) Somewhat absorbent. (Avoid waxed and coated papers)

2) No additives (Avoid colored papers. Use white or natural brown).

3) Strong. (Avoid paper towels and the like).

4) Flat. (Avoid embossed papers)

4) Available. (Kraft paper is parcel paper used for wrapping packages).

5) Cheap. (For those of us with near-empty wallets!)

Kraft paper is one paper that meets all the above criteria. It is

available under different names, but you will most often find it

sold as a wrapping paper for wrapping parcels. It normally comes

on a large roll about three feet wide. I have found it in stores

that sell office supplies, among other places.

Hope this helps.

Fr. Tom McGahee

----------

> From: Marco Denicolai <marco@vistacom.fi>

> To: Tesla-2 Mailing List <tesla-2@emachine.com>

> Subject: Kraft paper?

> Date: Monday, October 12, 1998 6:14 AM

>

> What kind of paper are you guys using in your rolled capacitors?

>

> I have heard about Kraft paper, but a salesman here in Finland ensured me

> that this paper really hardly absorbs oil or any other liquids. So what is

> the idea about using kraft paper?

>

>

>  Marco Denicolai                   Vista Communication Instruments, Inc.

>  Hardware Development Manager      www.vistacom.fi

>

>  marco@vistacom.fi                 Kaisaniemenkatu 13 A

>  fax:    +358-9-622-5610           SF-00100 HELSINKI

>  phone:  +358-9-622-623-15         Finland

>

From: Ashpilot@aol.com

Date: Mon, 12 Oct 1998 12:38:35 EDT

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Kraft paper?

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

I got some plain brown paper, from shipping/receiving, it is mostly like brown

paper bags for shopping.  It absorbs oil well, and seems to hold its physical

strength when wetted.

Ed Mullin

NH

From: Ashpilot@aol.com

Date: Mon, 12 Oct 1998 12:47:50 EDT

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Stacked Cap W/ Air  Primary Size

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

In a message dated 98-10-12 08:10:26 EDT, you write:

Bert wrote:

<<

The blue glow you're seeing is not caused by the magnetic field. It's

actually from the electrostatic (or e-field) ionizing the air between

the toroid and the primary. If you could see the primary's magnetic

field, it would look sort of like a big donut centered around the

primary. The portion of the primary's total magnetic field that will

actually be linked to the secondary (and vice-versa) is called the

coupling coefficient (or "k"). It turns out that, for a variety of

reasons, you'll get best coil performance when "k" is in the range of

0.10 to 0.25.

>>

Bert or any one, how can I determine the coupling coefficient for my coil?

I have a 3.25 inch secondary, being built, and a 7 inch x 12 turn primary solenoid

wound.  I plan to adjust coupling by lifting the secondary upwards away from

the primary...

on a side note, I just finished a flat plate cap, and was testing it last

evening, I noticed that at higher powers, I could see the oil near one of the

foil connections was forming small "standing waves" similar to what you would see

in a hydrosonic cleaner... the effect was not visible anywhere else.  Any ideas

about this??

just curious if it might indicate a potential failure..

Ed Mullin

X-Mailer: IPAD 2.0

Date: Mon, 12 Oct 1998 12:59:09 -0500

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: New coil/H&R transformers

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

At 04:32 AM 10/12/98 -0700, you wrote:

>Be careful, my 6" coil will produce arc in excess of 70". How much more than that Im

>not sure. I can't get an accurate read on it because it always arc's to something.

>What are H & R transformers ?

>

>paul m.

>---

>Einigheit und Wahrheit

>

Paul,

H&R refers to Herbach and Rademan surplus electronics.

http://www.herbach.com/

They have been selling some GE photocopier transformers

that at first glance look real good for coiling. They were

rated at 5kv/300ma. Two in series would yield 10kv/300ma.

An easy way to get 3KVA.

However these turned out to be ferro-resonant transformers

with a tertiary winding that connects to a capacitor. The

resonant condition falls out when the transformer is stressed

as in Tesla coil duty. They will not deliver 5kv in Tesla service.

Operating these as a normal transformer without the resonating

capacitor yields around 2500 Volts. Pretty much the same as

a microwave oven transformer. They do seem to be built better

than a typical MOT.

Several people had some success using these to drive a coil

by using sufficient capacitance to get 60 hz resonance. This

takes around .08 mfd. Thats lots of capacitance and requires

a coil designed for that level of capacitance.

I had considered getting a pair of these but decided against

it when it became apparent these were ferro-resonant transformers.

There was a great deal of discussion about these transformers

on the list. Check the archives for March 1997 and look

for the thread titled "10KV RMS @ 300MA for less than $100"

TTYL!

Mike Hammer

mhammer@misslink.net

From: ESchulz531@aol.com

Date: Mon, 12 Oct 1998 14:12:29 EDT

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Help needed for primary...

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Mike and Jim,

I have made my last 4 primaries the way Mike described.

I have found that for primaries with less then 15 turns there isn't

any noticeable change in coil performance with a multi layered

primary.  With primaries under 25 turns there isn't much

difference between two and four layers.  I didn't try 3 layers

since one of the ends of the coil would have to be by the

secondary.

I have found that building primary's the way Mike described

makes tuning easier since you can change both connections

to the coil easily.  When building my multilayered primaries I

tried sloped layers about +5% -5%.  It didn't make much

difference but it sure did look neat.  I used pieces of poly and

plastic ties to hold it all together.

The biggest performance change I have noticed was with

a 20 turn flat primary.  I changed it to a 16 turn 2 layered

primary.  Running at 900 VA on a 6" secondary with max

coupling with both setups.  I went from 35" sparks to ground

to 39" sparks to ground.  Only about a 10% increase in

performance.  This was the best case I found.

Erik Schulz

http://members.aol.com/tesla00/o_index.htm

From: ESchulz531@aol.com

Date: Mon, 12 Oct 1998 14:30:08 EDT

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Humidity?

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Hi,

I live in Galveston Island just south of Houston

during the winter seasons.  The humidity is so

high down here that when it rains I don't notice

any change in my coils performance. :o)  I

have gotten 43" streamers to my roof with a 6"

coil running at 1.2 kva.  Air streamers were 53"

The only down fall about the humidity is that

all wood conducts way to well.  I have had 15kv

arc through 6" of plywood and a 2 by 2 at the

same time.  This destroyed one of my early

rotary spark gaps.  If you use wood treat it as

though it is metal and insulate it.  This way

you will never have problems.  You could bake

the wood and seal it but I find this to be too much

trouble.  Insulating it is much easier.

Erik Schulz

http://members.aol.com/tesla00/o_index.htm

Date: Mon, 12 Oct 1998 19:29:07 -0500

From: Bert Hickman <bert.hickman@aquila.com>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Organization: Stoneridge Engineering

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Kraft paper?

Sender: tesl-err@emachine.com

Ashpilot@aol.com wrote:

>

> I got some plain brown paper, from shipping/receiving, it is mostly like brown

> paper bags for shopping.  It absorbs oil well, and seems to hold its physical

> strength when wetted.

>

> Ed Mullin

> NH

Ed,

That's the stuff!!

-- Bert --

Date: Mon, 12 Oct 1998 21:09:18 -0500

From: RODERICK MAXWELL <tank@magnolia.net>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: New coil

Sender: tesl-err@emachine.com

paul m wrote:

>

> Be careful, my 6" coil will produce arc in excess of 70". How much more than

>that Im not sure. I can't get an accurate read on it because it always arcs to >something. What are H & R transformers ?

>

> paul m.

> ---

> Einigheit und Wahrheit

>

They are actually photocopier transformers, or so I've heard. They

have an output of 5Kv with the resonance cap that is supplied with the

transformer. But the reports that I have heard is it's not a good idea

to use the resonance caps. So I'm using two of them in antiparallel

configuration. This will give me 5Kv at 300ma. Also to increase voltage

slightly I'm rectifying and filtering them with a full bridge rectifier.

The reason they call them H&R transformers is that is the company

that sells them surplus. They are actually made by General Electric.

Another thing I forgot to mention is if you turn your variac ALL the way

to maximum your voltage output increases to 7.5Kv!

Frankensteins Helper

Max

Date: Mon, 12 Oct 1998 21:38:59 -0500

From: Bert Hickman <bert.hickman@aquila.com>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Organization: Stoneridge Engineering

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Stacked Cap W/ Air  Primary Size

Sender: tesl-err@emachine.com

Ashpilot@aol.com wrote:

<SNIP>

>

> Bert or any one, how can I determine the coupling coefficient for my coil?

> I have a 3.25 inch secondary, being built, and a 7 inch x 12 turn primary

> solenoid wound.  I plan to adjust coupling by lifting the secondary upwards

> away from the primary...

Well, there are a couple of different ways to measure it, but chances

are you really don't have to! Once you've tuned your system, you can

then simply begin raising or lowering the secondary versus the primary

until you get the best overall system performance without getting any

inter-turn flashovers between sections of the secondary. At the end of

the posting I've attached a method to accurately measure "k" if you

really want to know its value (from Terry Fritz on the main Tesla

Builders' list). It does require using an inductance meter (best), or an

accurate calculation of Lp and Ls.

>

>  on a side note, I just finished a flat plate cap, and was testing it last

> evening, I noticed that at higher powers, I could see the oil near one of the

> foil connections was forming small "standing waves" similar to what you would

> see in a hydrosonic cleaner... the effect was not visible anywhere else.

> Any ideas about this??  just curious if it might indicate a potential failure..

>

> Ed Mullin

I suspect that its simply vibration from the capacitor plates. If

there's any looseness in the capacitor stack, the plates will be

alternately compressed and relaxed. As the cap charges, electrostatic

attraction pulls the plates more tightly against the dielectric. When

the cap is disruptively discharged, the plates relax. This process

repeats at whatever your main-gap breakrate happens to be, vibrating the

plates, oil, and container. This should not be a problem.

Measuring K: (from Terry Fritz)

==============================

I have tried all the suggestion I have received (Thanks Malcolm, Fr.

Tom, John C., Mark Rzeszotarski).  The best method I have found that

does not require expensive equipment or great theoretical challenges

consists of the following.

Apply a heavy 60 Hz AC current to the primary coil.  This is best

done by placing a space heater, hair dryer, etc. in series with the

primary to limit the current to about 10 amps.  Measure this current

with a multimeter.  Note that the space heater gives a fairly stable

resistance. Light bulbs have a non-linear resistance through the AC

cycle and distort the measurement (they must cool down substantially at

the nodes of the AC cycle).  Of course, use great caution with the live

AC on the primary so as not to kill yourself. Only the isolated primary

need be connected to the AC. The capacitors, transformers, and other

wiring should be disconnected from the primary for this test.  Be

cautious of the AC finding its way on to the secondary!

Place a 10k ohm resistor and a 1uF capacitor across the secondary and

measure the AC voltage.  It will be on the order of say 100 mV AC.  The

resistor and capacitor will eliminate stray noise picked up by the

secondary and swamp any resonance which is significant at these low

levels.

The mutual inductance is found by:

M = V / (w * I)

Where:

M = Mutual inductance in Henries.

w = the line frequency in radians per second (377 for 60Hz or 314

for 50 Hz).

I = The measured current in the primary in amps AC.

V = The measured secondary voltage in volts AC.

As an example:

If the current in the primary is 10 amps and the frequency is 60Hz

and you measure 0.100 volts AC, you would get:

0.100 / (377 * 10 ) = 26.52 uH for the mutual inductance.

k can then be found by using the formula:

k = M / sqrt(L1 * L2)

Where L1 and L2 are the inductances of the primary and secondary coils.

This method is rock solid in theory and easy to do.  The accuracy is

excellent.  There is little that can go wrong compared to other methods

and you don't need anything special other than a multimeter to do the

test.  The accuracy is dependant on the accuracy of your multimeter.  My

tests could easily get within 1%.

Thanks again for all the great suggestions and do be careful with

the AC if you try this.

Hope this helps!

-- Bert --

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: "Coiler" <mycroft@access1.net>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Saving the Garage Door Opener?

Date: Tue, 13 Oct 1998 06:30:46 -0700

X-Mailer: IPAD 2.0

Importance: Normal

X-MimeOLE: Produced By Microsoft MimeOLE V4.72.3155.0

Sender: tesl-err@emachine.com

I ran my coil outside for the first time this weekend, and was

very disappointed in the performance. Unfortunately where I live

there is fairly constant breezes. Great for staying cool, lousy

for the formation of ion clouds.

So.

I'm trying to find a way to move back into the garage. But there is this

pesky little door opener in the way. And my wife has been fairly understanding

of my purchasing of all the hardware for the coil, but draws the line at

my removing the door opener. Since my door and operating hardware

are all metal, I thought I might be able to protect the system in this

manner, someone please tell me if I am all wet.

1) Attach runners to my RF ground.

2) Build a 'faraday cage' of aluminum mesh around door opener unit

itself, with a door for the antenna to hang down for normal operation,

to be lifted up inside the cage during coil runs. Attach this cage to

RF ground.

3) Yank the plug during the runs

4) Ground the door via clamp.

Do I stand a chance?  I already know that running the coil in the next

bay over (I have a 3-car) confuses the obstruction sensor to the point

where it shuts down, requiring me to unplug the door opener to reset.

Thanks

Michael Baumann

Coiler, Homebrewer, Nerd. mycroft@access1.net

X-Originating-IP: [207.100.20.40]

From: "Philip Mac Duffie" <punk_42@hotmail.com>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Kraft paper?

Date: Tue, 13 Oct 1998 14:42:23 PDT

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

What is the dielectric constant (k) of kraft paper? Is it as good or

better as poly (ldpe) for rolled caps?

Thanks,

Philip Mac Duffie

X-Mailer: IPAD 2.0

Date: Tue, 13 Oct 1998 19:35:35 -0500

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: Kraft paper?

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

At 02:42 PM 10/13/98 PDT, you wrote:

>What is the dielectric constant (k) of kraft paper? Is it as good or

>better as poly (ldpe) for rolled caps?

>    Thanks,

>  Philip Mac Duffie

Philip,

Kraft paper has a similar dielectric constant to polyethylene

but a much lower breakdown voltage. Its RF dissipation fact is somewhat

higher than poly. Its useful as an adjunct to poly but not as a replacement.

I once made some test caps using mineral oil soaked sheets

of white printer paper and aluminum foil. Using 10 such sheets

for each layer and two cap modules in series it ran for a while

on a 7500 volt neon. Performance was fair. The dielectric failed

after a short time and I never pursued the idea any further.

I did calculate the dielectric constant of the paper using the actual

measured capacitance and got a value of 2.3.

Since poly is so cheap and so readily available there isnt any

reason to use paper alone as a dielectric.

TTYL!

Mike Hammer

mhammer@misslink.net

X-Mailer: IPAD 2.0

Date: Tue, 13 Oct 1998 19:39:53 -0500

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: Saving the Garage Door Opener?

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

At 06:30 AM 10/13/98 -0700, you wrote:

>I ran my coil outside for the first time this weekend, and was

>very disappointed in the performance. Unfortunately where I live

>there is fairly constant breezes. Great for staying cool, lousy

>for the formation of ion clouds.

>So.

>I'm trying to find a way to move back into the garage. But there is this

>pesky little door opener in the way. And my wife has been fairly

>understanding

>of my purchasing of all the hardware for the coil, but draws the line at

>my removing the door opener. Since my door and operating hardware

>are all metal, I thought I might be able to protect the system in this

>manner, someone please tell me if I am all wet.

>

>1) Attach runners to my RF ground.

>2) Build a 'faraday cage' of aluminum mesh around door opener unit

>   itself, with a door for the antenna to hang down for normal operation,

>   to be lifted up inside the cage during coil runs. Attach this cage to

>   RF ground.

>3) Yank the plug during the runs

>4) Ground the door via clamp.

>

>Do I stand a chance?  I already know that running the coil in the next

>bay over (I have a 3-car) confuses the obstruction sensor to the point

>where it shuts down, requiring me to unplug the door opener to reset.

>

>Thanks

>Michael Baumann

>Coiler, Homebrewer, Nerd. mycroft@access1.net

>

Michael,

Those are all good ideas and they should help.

But as you well know there is only one sure way to know

for sure. Fire it up! (might want to cross your fingers)

TTYL!

Mike Hammer

mhammer@misslink.net

X-Mailer: IPAD 2.0

Date: Tue, 13 Oct 1998 20:15:44 -0500

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: New coil

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

At 10:03 PM 10/11/98 -0500, you wrote:

>I will be testing my new 6" coil this week! Here are the specs.

>

>  Secondary form:  rolled polystyrene 6.25" x 34" wound with 21 gauge

>  magnet wire  winding lentgh: 28.25"

>  coating: epoxy resin, and polyurethane varnish

>

>  Primary: flat pancake primary wound with 3/8" copper tubing

>  spacing between turns: 3/8" ten turns total

>

>  Tank capacitance: 56.4nf poly vertical plate capacitor

>

>  Power supply: two H&R transformers in antiparallel configuration

>  rectified by four 15Kv microwave oven diodes in a bridge circuit.

>  They are rated at 500ma.

>   filter: 4700pf ceramic doorknob capacitor rated at 30Kv.

>

>  Spark gap: vaccuum gap with 5 gaps total. I will be using a shop vac

>  to supply the  vaccuum.

>

>   Misc: safety gaps, filter caps, capacitor gaps, chokes, damping

>  resistors, etc......

>

>

>  I should have my first low power test within the next couple of days.

>I will take    pics, and post them when I get them developed. I don't

>want to push it to hard for the next week or two because the capacitor

>has to be broken in SLOWLY! It took a long time to build and I would cry

>REAL tears if I blew a hole through it!

>  I'm very curious to see if using a dc power supply instead of the

>usual ac will make any difference in performance.

>

>

>

>                                  Frankensteins Helper

>                                          Max

Roderick,

Since those H&R transformers are very much like MOTs and you

are thinking about going to DC with them you might be interested

in what Marco Denicolai has done.

He is using a pair of MOTs to run a voltage doubler circuit using

common microwave components. He got some pretty good results.

Better than I expected.

Marcos web page is at: http://www.saunalahti.fi/dncmrc/

There was some discussion of his doubler on the list a little

while ago. Look for the thread "8kV MW oven supply and spark gaps"

on May 22.

TTYL!

Mike Hammer

mhammer@misslink.net

Date: Tue, 13 Oct 1998 20:41:21 -0500

From: Bert Hickman <bert.hickman@aquila.com>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Organization: Stoneridge Engineering

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Kraft paper?

Sender: tesl-err@emachine.com

Philip Mac Duffie wrote:

>

> What is the dielectric constant (k) of kraft paper? Is it as good or

> better as poly (ldpe) for rolled caps?

>     Thanks,

>   Philip Mac Duffie

>

Philip,

The kraft paper's function is to help wick oil between the LDPE plates

to help remove entrapped air. Oil-soaked kraft paper will have a

dielectric constant that's somewhere between 3 and 4, depending upon the

density of the kraft paper, while the LDPE will be in the range of 2.0 -

2.2. Kraft paper (alone) makes a comparatively poor dielectric - it's

much lossier and also lacks the breakdown strength of LDPE. So, the

really "hard" work in a composite LDPE/Kraft paper cap will still be

done by the LDPE. The presence of the kraft paper actully makes the cap

somewhat more lossy than a simple LDPE cap. However, it extends the

capacitor's life by helping to prevent destructive internal corona.

-- Bert --

From: McFalc0n@aol.com

Date: Tue, 13 Oct 1998 22:02:41 EDT

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Faraday Cage, stacked primary???

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

I've heard that a faraday cage will protect items from radio effects produced

by a coil.  How do you make a faraday cage?

Does anyone have any ideas on how to make a stacked primary?  Any gains or

losses?

Thanx.

Date: Tue, 13 Oct 1998 21:04:30 -0500

From: RODERICK MAXWELL <tank@magnolia.net>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: New coil

Sender: tesl-err@emachine.com

>

> Roderick,

>

> Since those H&R transformers are very much like MOTs and you

> are thinking about going to DC with them you might be interested

> in what Marco Denicolai has done.

>

> He is using a pair of MOTs to run a voltage doubler circuit using

> common microwave components. He got some pretty good results.

> Better than I expected.

>

> Marcos web page is at: http://www.saunalahti.fi/dncmrc/

>

> There was some discussion of his doubler on the list a little

> while ago. Look for the thread "8kV MW oven supply and spark gaps"

> on May 22.

>

> TTYL!

>

> Mike Hammer

> mhammer@misslink.net

Yes I saw his coil on his webpage. I wanted to make a voltage doubler

out of mine also. But my tank cap only has 60mil. poly for the

dielectric. I don't want to blow it the first time out. Later after it

has had time to "mature" I may change the circuit over to a doubler

circuit. With a voltage doubler on these transformers I should be able

to get 15kv with them in antiparrallel and the variac cranked to maximum

(it will probably blow the !#$^$%#@ out of transformers though due to

over stressing the secondary windings).

Frankensteins Helper

Max

X-Mailer: IPAD 2.0

Date: Wed, 14 Oct 1998 23:05:30 -0500

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: Unsubscribing yet again

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Well looks like its time once again for my almost now famous

unsubscribe instructions. So here we go again.

The first thing anyone should do when joining a mailing

list is to read and keep the list server instructions

that are sent when you first subscribe. These instructions

contain the procedures for using the list server and

even tells you how to unsubscribe yourself from the list.

Rule #1 on almost any list is to never send unsubscribe requests

to the list. These may not always be seen or acted upon by

the proper personnel. And the request is sent out needlessly

to all list subscribers. Unsubcribe commands should instead

be sent to the list server itself. The proper addresses to

use are contained in the information file sent to you when you

first subscribed.

So for those of you that deleted that file rather than retaining

it I will post the unsubscribe procedure here once again. Please make

note of this and keep a copy for future reference. If you have difficulty

unsubscribing DO NOT post a request to the list. Email me direct instead.

To quit the list, either permanently or temporarily, such as for vacation,

please send a message to tesla-2-request@emachine.com with one of the

following keywords in the body of the message:  Delete, Remove or

Unsubscribe.  Case is not important.

If you should later wish to rejoin the list, please send a message to

tesla-2-request@emachine.com with one of the following keywords in the

body of the message:  Join, Add or Subscribe.  Case is not important.

One additional note. If you unsubscribe and then try to resubscribe

yourself later on it must be with the same email address. If

you change addresses then the list server sees you as a new subscriber.

New subs still need to go through me.

If you need to change your address email me with the change

rather than unsubscribing.

TTYL!

Mike Hammer

mhammer@misslink.net

X-Mailer: IPAD 2.0

Date: Thu, 15 Oct 1998 08:44:40 +0200

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Marco Denicolai <marco@vistacom.fi>

Subject: Dielectric strength vs. thickness ?

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

I got yesterday 2 sheets 2 x 1 meters of Polystyrene (PS), 0.5mm (0.02") thick.

That cost about $5 each, from the original manufacturer. As well, I also got

REAL data for THIS material, not general data for all PS's in the world.

Dielectric constant:                    2.5

Dissipation factor (@ 1MHz):            0.0003

Dielectric strength (0.2 mm thickness): >40 kV/mm  (i.e. >1016 kV/mil)

Water absorption:                       <0.1%

So I believe that is a feasible material for my capacitors.

My question is:

I know that dielectric strength (kV/mm) GROWS with decreasing thickness. I got

0.5mm thick PS: how much I should derate its dielectric strength from the

40 kV/mm measured with a 0.2mm thick sheet?

Marco Denicolai                   Vista Communication Instruments, Inc.

Hardware Development Manager      www.vistacom.fi

marco@vistacom.fi

Date: Thu, 15 Oct 1998 06:48:53 -0500

From: Bert Hickman <bert.hickman@aquila.com>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Organization: Stoneridge Engineering

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Dielectric strength vs. thickness ?

Sender: tesl-err@emachine.com

Marco Denicolai wrote:

>

> I got yesterday 2 sheets 2 x 1 meters of Polystyrene (PS), 0.5mm (0.02") thick.

>

> That costed about $5 each, from the original manufacturer. As well, I also got

> REAL data for THIS material, not general data for all PS's in the world.

>

> Dielectric constant:                    2.5

> Dissipation factor (@ 1MHz):            0.0003

> Dielectric strength (0.2 mm thickness): >40 kV/mm  (i.e. >1016 kV/mil)

> Water absorption:                       <0.1%

>

> So I believe that is a feasible material for my capacitors.

>

> My question is:

> I know that dielectric strength (kV/mm) GROWS with decreasing thickness. I got

> 0.5mm thick PS: how much I should derate its dielectric strength from the

> 40 kV/mm measured with a 0.2mm thick sheet?

>

>  Marco Denicolai                   Vista Communication Instruments, Inc.

>  Hardware Development Manager      www.vistacom.fi

>

>  marco@vistacom.fi

Marco,

Polystyrene may have a long-term problem of degrading or dissolving in

mineral oil per a solvent-resistance table I've got for various

plastics. Because of this, I've never used the material for making caps.

You may want to consult with the manufacturer to see if there might be

any long-term problems before actually building the cap.

The manufacturer's quoted breakdown strength of 40 kV/mm converts to

about 1000 Volts/mil for very pure material under DC stres. However, the

ACTUAL breakdown strength of YOUR particulat piece of material will be a

limited by the weakest area (due to scratches/imperfections, impurities,

internal defects, etc.). From a design standpoint, you'll want to have

some safety margin. You can get this by designing your cap using

multiple layers of dielectric, and if you use sufficient total thickness

such that the combination is never stressed beyond 250 volts/mil it

should survive.

For example, for a 15 kV RMS power source, the peak volts will be about

21 kV, which means you'd need about 70 mils (minimum) of material spread

across multiple identical caps in series to share the overall voltage

stress. Using a greater total dielectric thickness will increase the

margins, and the odds of survival.

Safe cappin' to you!

-- Bert --

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Date: Fri, 16 Oct 1998 11:37:14 EDT

Subject: Re: Kraft paper?

X-Mailer: IPAD 2.0

From: alfred.skrocki.sr@juno.com (Alfred A Skrocki)

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

On Tue, 13 Oct 1998 14:42:23 PDT Philip Mac Duffie

<punk_42@hotmail.com> wrote:

>What is the dielectric constant (k) of kraft paper?

Hi Philip, first off you don't wan't to make caps for a tesla oscillator

circuit out of just kraft paper! It's dielectric strength is way too low.

You ALWAYS use kraft paper with an additional dielectric like wax or oil

in which case it's dielectric constant is going to work out between 2 and 3.

>Is it as good or better as poly (ldpe) for rolled caps?

Neither, it's worse! Even though the dielectric constant isn't much

different the low dielectric strength of the kraft paper/oil or wax combo

makes it a poorer choice. To be honest with you, you shouldn't use kraft

paper caps at anything over a few hundred volts.

Alfred A. Skrocki

Alfred.Skrocki.Sr@JUNO.COM

X-Originating-IP: [216.68.40.101]

From: "Bill the arcstarter" <arcstarter@hotmail.com>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: New record!

Date: Sat, 17 Oct 1998 16:12:44 PDT

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

Coilers,

Today I went down to Paul's to witness the running of our 6.5 inch

secondary coil.  I'd not seen it in action yet.

Let me just say it is very impressive!  The toroid is probably too

small as the coil has no trouble maintaining 3 or 4 simultaneous 4ft

arcs to various things...

But the real treat of the day came when the coil decided to throw a

monster record-breaking arc - from the toroid to the control transformer

on the wall!  The HV RF instigated a power arc on the 110V side of the

control relay, vaporizing a PCB trace or two, and emitting a ball of

molten metal!

One I saw that happen I ran for the mains disconnect, as this was the

contactor which gates the pig power!  However, my running was

unnecessary since the contactor released.  As of yet we don't know if

any of that stuff is damaged permanently or not... :)

After we put the fire out (!) we measured the distance.  81 inches!  We

were runnning about 7KVA for this shot...

We hope to get this back together so we can run it out in the front

yard for halloween! Boo!

More later.  Hey - I've been on the mIRC channel recently with a couple

of other coilers.  I'd really suggest you guys on this list get in there

and try it out!

See you on mIRC... (channel is: tesla_coil)

-Bill

(editors note. Bill is referring here to the #Tesla_coil IRC discussion

channel that I run. Info on the channel can be found on my page at:

http://www.misslink.net/mhammer/irc.htm)

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: "Coiler" <mycroft@access1.net>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: RE: New record!

Date: Sat, 17 Oct 1998 22:40:22 -0700

X-Mailer: IPAD 2.0

Importance: Normal

X-MimeOLE: Produced By Microsoft MimeOLE V4.72.3155.0

Sender: tesl-err@emachine.com

>  We hope to get this back together so we can run it out in the front

> yard for halloween! Boo!

*chuckle* must be a thing about us Coilers.. I will be doing the same with

my 6.25" coil, but running at far more sedate power levels I am afraid,

as I only have enough current limiting hardware to get me to about 1400VA

I may try out my 'cobbled' together rotary.. should give me 230BPS,

better than the 120 I get with my static.

>

>  More later.  Hey - I've been on the mIRC channel recently with a couple

> of other coilers.  I'd really suggest you guys on this list get in there

> and try it out!

>

> See you on mIRC... (channel is: tesla_coil)

> -Bill

Which net?? Under, EF, DAL????  btw, the net is IRC, mIRC is one of

many clients. [I favor pIRCh myself]

Michael Baumann

Coiler, Homebrewer, Nerd. mycroft@access1.net

X-Mailer: IPAD 2.0

Date: Sun, 18 Oct 1998 00:58:20 -0500

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Mike Hammer <mhammer@misslink.net>

Subject: RE:IRC channel (was New record!)

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

At 10:40 PM 10/17/98 -0700, you wrote:

>

>>

>>  More later.  Hey - I've been on the mIRC channel recently with a couple

>> of other coilers.  I'd really suggest you guys on this list get in there

>> and try it out!

>>

>> See you on mIRC... (channel is: tesla_coil)

>> -Bill

>Which net?? Under, EF, DAL????  btw, the net is IRC, mIRC is one of

>many clients. [I favor pIRCh myself]

>

>Michael Baumann

>Coiler, Homebrewer, Nerd. mycroft@access1.net

>

Michael,

The #Tesla_coil IRC channel is located on XNET.

XNET has several servers across the US as well

as some in Europe and Australia.

Use the IRC client of your choice and connect to

webserve.nc.us.xnet.org. Then /join #Tesla_coil.

More information on the channel and best times

to try are on my web page at: http://www.misslink.net/mhammer/irc.htm

TTYL!

Mike Hammer

mhammer@misslink.net

X-Originating-IP: [207.100.20.16]

From: "Philip Mac Duffie" <punk_42@hotmail.com>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: TC progress

Date: Sun, 18 Oct 1998 08:00:45 PDT

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

Just wanted to keep everyone informed that I actually am making progress

on my first TC.

Secondary - done - 4.25" x 20"

Primary   - Build Today - 12 turns 1/4 copper tube

Gaps      - Build Tomarrow - Flat RQ style 11 gaps 1" copper pipe

Cap       - Done - .0089uf poly, oil filled ,vaccumed ,but had internal

problems may make another.

Chokes    - Done - 3" toroids with 162 turns awg #26

Topload   - 1/2 Done - 5" sphere

Neon      - Unpoted, Rebuilt - France 9kv 30ma

Nst tank  - Done - 1/2" ply , 4 coats fiberglass resin

P.S. Pics of construction should be up soon on my page

http://hv.hypermart.net

Thanks,

Philip Mac Duffie

Date: Sun, 18 Oct 1998 10:49:28 -0500

From: Bert Hickman <bert.hickman@aquila.com>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Organization: Stoneridge Engineering

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: TC progress

Sender: tesl-err@emachine.com

Philip Mac Duffie wrote:

>

> Just wanted to keep everyone informed that I actually am making progress

> on my first TC.

>

> Secondary - done - 4.25" x 20"

> Primary   - Build Today - 12 turns 1/4 copper tube

> Gaps      - Build Tomarrow - Flat RQ style 11 gaps 1" copper pipe

> Cap       - Done - .0089uf poly, oil filled ,vaccumed ,but had internal

>             problems may make another.

> Chokes    - Done - 3" toroids with 162 turns awg #26

> Topload   - 1/2 Done - 5" sphere

> Neon      - Unpoted, Rebuilt - France 9kv 30ma

> Nst tank  - Done - 1/2" ply , 4 coats fiberglass resin

>

> P.S. Pics of construction should be up soon on my page

> http://hv.hypermart.net

>

>     Thanks,

>  Philip Mac Duffie

>

>

Philip,

It certainly sounds like you're almost there! One problem you may

encounter when using the sphere as a topload is corona breakout from the

top of the secondary winding, since the relatively small diameter sphere

doesn't provide nearly as much "shielding" as a toroid. A simple

dryer-duct toroid (using 3" or 4" diameter duct) may be something to

consider in the future. Good luck and here's hoping you have a

successful "first light"! BTW, nice website!

-- Bert --

Date: Sun, 18 Oct 1998 09:18:25 -0700

From: Gene Waldo <gwaldo@1stnetusa.com>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: New record!

Sender: tesl-err@emachine.com

Bill the arcstarter wrote:

>

> Coilers,

>  Today I went down to Paul's to witness the running of our 6.5 inch

> secondary coil.  I'd not seen it in action yet.

>

>  After we put the fire out (!) we measured the distance.  81 inches!  We

> were runnning about 7KVA for this shot...

>

> Hey - I've been on the mIRC channel recently with a couple

> of other coilers.  I'd really suggest you guys on this list get in there

> and try it out!

>

> See you on mIRC... (channel is: tesla_coil)

> -Bill

>

Bill,

Bad luck! Good coil!

Puts me in mind of the guy (Pollack, of course ~~<[:-) ) who spent years

building a 20' sailboat in his basement....Too big to get out...

Have a scotch and ponder where you left your wire strippers!

Just don't set any Trick-or-Treaters on fire!

-----------------------------

Stupid question, and eng. challenge: consider the ckt:

T

O-----O         X

|             \  SG

2ndary- ( )             \

( )              \

( )               \

P ooooo  ( )  ooooo         \

|                  |

-----                |

-----   C1           SD

|                  |

--------------------

|

RFGND

Where: C1 is perhaps set on a pallet (as are pri&2nd),~ 3'x3',

consisting some several layers of HDPE, plywood, plex? and 2 or 4 sheets

(~3'x3') of sheet metal.... All out in the backyard!

SG is 1 or more 10' lenghts of Cu pipe (or whatever); if more than 1,

they are sequenced by some sequencing Switching Device lash-up....

X may be any sort of incan-neon-flour. assy for visual impact.

Even if C1 is not practical, the rest may have value?

I ask, only because I don't know.

------------------------------------------

Gene Waldo -  gwaldo@1stnetusa.com  - "Igor in Residence"

---------------------------------------------------------------

X-Originating-IP: [207.100.20.19]

From: "Philip Mac Duffie" <punk_42@hotmail.com>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: corona breakout

Date: Sun, 18 Oct 1998 11:09:37 PDT

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

Will too small a toroid do the same? Also i plan only to use the sphere

during tuning..

Thanks,

Philip Mac Duffie

X-Originating-IP: [207.100.20.19]

From: "Philip Mac Duffie" <punk_42@hotmail.com>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Cap Fix

Date: Sun, 18 Oct 1998 11:19:48 PDT

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

I you read my previous post you would see that my cap had a problem, but

the problem is that when i built it (poly rolled cap) the tab was too

short comming off of one plate so it ripped when cap slid around inside the

tubing. I bashed the end cap off so i could fix it, I made a new piece

of foil to the bolt terminal and hooked that to the ripped tab by

squishing them together, i know its not the best but, im now worried

that the oil will get into the connection and stop conduction. If so

what will happen? Im guessing Corona will form and destroy the aluminum

foil.?.? i dunno any ideas welcome.

P.S i just need this cap to work for a school demo (electronics class)

so I plan to build two new caps in series for future use. Although this

cap isnt the best i learned alot about vaccuming them from this one.

Date: Sun, 18 Oct 1998 13:26:04 -0500

From: Bert Hickman <bert.hickman@aquila.com>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Organization: Stoneridge Engineering

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: corona breakout

Sender: tesl-err@emachine.com

Philip Mac Duffie wrote:

>

> Will too small a toroid do the same? Also i plan only to use the sphere

> during tuning..

>    Thanks,

> Philip Mac Duffie

>

Philip,

For tuning purposes, you can always make a large radius of curvature top

terminal break out at lower power levels by attaching a small nail or

tack to the top or outer surface of the toroid. I use a small looped

piece of aluminum tape (the type with adhesive used for

heating/ventillation), but really anything that disrupts the smooth

E-field from the top terminal will work. This even works at higher power

levels if you make a breakout bump from the adhesive foil that's a

slightly raised area where you want the toroid to first break out. This

is often useful in controlling where the streamers go when taking

pictures of the coil. With a slight bump, it doesn't break out too

early.

By doing this, you can optimally tune the system at relatively low

power, then begin ramping up power (first with, then without the

break-out aid).

Good Luck!

-- Bert --

Date: Sun, 18 Oct 1998 13:36:19 -0500

From: Bert Hickman <bert.hickman@aquila.com>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Organization: Stoneridge Engineering

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Cap Fix

Sender: tesl-err@emachine.com

Philip Mac Duffie wrote:

>

> I you read my previous post you would see that my cap had a problem, but

> the problem is that when i built it (poly rolled cap) the tab was too

> short comming off of one plate so it ripped when cap slid around inside

> tubeing. I bashed the end cap off so i could fix it, I made a new peice

> of foil to the bolt terminal and hooked that to the ripped tab by

> squishing them together, i know its not the best but, im now worried

> that the oil will get into the connection and stop conduction. If so

> what will happen? Im guessing Corona will form and destroy the alluminum

> foil.?.? i dunno any ideas welcome.

>

> P.S i just need this cap to work for a school demo (electronics class)

> so I plan to build two new caps in series for future use. Although this

> cap isnt the best i learned alot about vaccuming them from this one.

>

Philip,

The cap will probably function for a while. Oil will get between the

connected plates, but the high voltage will easily arc through this thin

layer of oil. The arcing will generate a bit of carbonized oil, and

eventually the arcing may also erode the aluminum foil connection to the

point where it fails. With luck it'll survive some demos, but a more

permanent repair (or rebuild) will eventually be necessary.

-- Bert --

X-Originating-IP: [216.68.32.72]

From: "Bill the arcstarter" <arcstarter@hotmail.com>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: TC progress

Date: Sun, 18 Oct 1998 12:08:58 PDT

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

Philip wrote:

>> Topload   - 1/2 Done - 5" sphere

<SNIP>

Then Bert wrote:

>It certainly sounds like you're almost there! One problem you may

>encounter when using the sphere as a topload is corona breakout from the

>top of the secondary winding, since the relatively small diameter sphere

>doesn't provide nearly as much "shielding" as a toroid. A simple

>dryer-duct toroid (using 3" or 4" diameter duct) may be something to

>consider in the future. Good luck and here's hoping you have a

>successful "first light"! BTW, nice website!

I agree with Bert.  We tried using a 2 ft diam sphere (had one laying

aroung from a Van De Graph project) and it didn't work well at all.  It

caused many arcs straight down the secondary, arc'd to the primary,

arc'd to all the places it wasn't supposed to.

We had better luck using the potlid than the sphere! :)  Don't let your

woman see you doing that (unless you buy your own and keep the

receipt!)... :)

-Bill

From: Ashpilot@aol.com

Date: Sun, 18 Oct 1998 16:21:20 EDT

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Cap Fix

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

I recently built a flat plate cap consisting of several caps in series.  I

left 3 inch tabs on each plate, then I tightly rolled each set together with

the set from the next cap in series.  I crimped them together and used staples

(yes staples) to hold the crimped rolls together.  I think perhaps this was

not so good an idea:(   But the cap has survived test runs with the gap set at

up to 1/8th inch...   (I'm running a 10kv/21mA obit)  So do you think I should

start building my replacement cap now or wait for it to fail? :)

Ed Mullin

Nashua, New Hampshire

Date: Sun, 18 Oct 1998 16:39:31 -0500

From: Bert Hickman <bert.hickman@aquila.com>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Organization: Stoneridge Engineering

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Cap Fix

Sender: tesl-err@emachine.com

Ashpilot@aol.com wrote:

>

> I recently built a flat plate cap consisting of several caps in series.  I

> left 3 inch tabs on each plate, then I tightly rolled each set together with

> the set from the next cap in series.  I crimped them together and used staples

> (yes staples) to hold the crimped rolls together.  I think perhaps this was

> not so good an idea:(   But the cap has survived test runs with the gap set at

> up to 1/8th inch...   (I'm running a 10kv/21mA obit)  So do you think I should

> start building my replacement cap now or wait for it to fail? :)

>

> Ed Mullin

> Nashua, New Hampshire

Ed,

Because the current in the primary circuit can be hundreds of amps when

the gap fires, it's important that good electrical contact be made and

maintained inside the capacitor after filling with oil. Staples might be

sufficient, but nuts, bolts and washers are better. I'd just run the cap

and fix it if it fails later on. Who knows, you may luck out!

-- Bert --

X-Originating-IP: [207.100.20.34]

From: "Philip Mac Duffie" <punk_42@hotmail.com>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: new cap repair

Date: Sun, 18 Oct 1998 17:14:26 PDT

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

I finally got mad enought to rip it apart and see what the hell is wrong

now. I bashed the endcap off (PVC) and found my repair was making poor

contact, so i was wondering if i could just drill a hole and run the

bolt through the side? the old bolt came out the end of the PVC endcap

Date: Sun, 18 Oct 1998 20:33:14 -0500

From: RODERICK MAXWELL <tank@magnolia.net>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: New coil Online

Sender: tesl-err@emachine.com

I tested my new six inch coil this week. The results were mixed. I was

using two H&R transformers in antiparrallel. I was only able to get the

spark gap to fire after shorting out three of five gaps in my vaccuum

gap. After removing the bridge rectifier, and the filter cap the gap

would not fire at all! Gap spacing is only 30mils. To remedy this

situation I have ordered two dc filter caps. This will let me change the

circuit over to a full wave doubler circuit. The filter caps are double

capacitors. That is to say there are two 5uf caps in each can. I plan to

use two of these double caps in series for a total capacitance in the

doubler of 1.25uf.

Tests with my 12Kv supply and a much smaller cap were much better.

Although I had to tap out to the final turn to get in tune. Discharges

were only about 24". I had to limit the size of the torus to stay in

tune. I could not use the cap that was designed for the coil for fear of

blowing my 12Kv supply. The tank cap is 55nf and the 12Kv supply is only

180ma which is too great a mismatch.

After I build the doubler circuit for the H&Rs I'll give it another

shot.......

Frankensteins Helper

Max

X-Mailer: IPAD 2.0

Date: Mon, 19 Oct 1998 08:57:57 +0200

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Marco Denicolai <marco@vistacom.fi>

Subject: Polystyrene and oil?

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

I have got 2 sheets 2 x 1 meters of Polystyrene (PS), 0.5mm (0.02") thick.

Dielectric constant:                    2.5

Dissipation factor (@ 1MHz):            0.0003

Dielectric strength (0.2 mm thickness): >40 kV/mm  (i.e. >1016 kV/mil)

Water absorption:                       <0.1%

So I believed that was a feasible material for my capacitors until some of you

mentioned that it cannot stand oil. A call to the manufacturer confirmed it:

they spoke about it standing 4 hours, maximum one week.

From the web I found a table about PS were it was possible to see that PS stands

SOME oils but not ALL oils. So I tested two samples of PS:

A - clear PS, 0.25 mm thick

B - white PS, 0.5 mm thick (the one for the capacitors)

I used olive (!) oil and motor oil (full synthetic). I haven't got yet

transformer oil!

Several days at 20C degrees didn't change a thing in the samples. Then I put

the samples in motor oil to bake into my furnace for 5 hours at 75C degrees. The B

(white) sample didn't show any visible change. The A (clear) sample lost its

transparency, became opaque.

With the aid of a microscope I saw on the B sample a lot of microscars:

still the sample is as flexible as it was before, and the scars aren't deep.

I have read that silicon oil is used for transformers and that it doesn't

attack most plastics.

Do you know what is the oil you use for your capacitors ?

Has anybody used PS in rolled capacitors ?

Marco Denicolai                   Vista Communication Instruments, Inc.

Hardware Development Manager      www.vistacom.fi

marco@vistacom.fi

Date: Mon, 19 Oct 1998 06:54:25 -0500

From: RODERICK MAXWELL <tank@magnolia.net>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Polystyrene and oil?

Sender: tesl-err@emachine.com

Marco Denicolai wrote:

>

> I have got 2 sheets 2 x 1 meters of Polystyrene (PS), 0.5mm (0.02") thick.

>

> Dielectric constant:                    2.5

> Dissipation factor (@ 1MHz):            0.0003

> Dielectric strength (0.2 mm thickness): >40 kV/mm  (i.e. >1016 kV/mil)

> Water absorption:                       <0.1%

>

> So I believed that was a feasible material for my capacitors until some of you

> mentioned that it cannot stand oil. A call to the manufacturer confirmed it:

> they spoke about it standing 4 hours, maximum one week.

>

> >From the web I found a table about PS were it was possible to see that PS stands

> SOME oils but not ALL oils. So I tested two samples of PS:

>

> A - clear PS, 0.25 mm thick

> B - white PS, 0.5 mm thick (the one for the capacitors)

>

> I used olive (!) oil and motor oil (full synthetic). I haven't got yet

> transformer oil!

>

> Several days at 20C degrees didn't change a thing in the samples. Then I put

> the samples in motor oil to bake into my furnace for 5 hours at 75C degrees.

> The B (white) sample didn't show any visible change. The A (clear) sample lost its > transparency, became opaque.

>

> With the aid of a microscope I saw on the B sample a lot of microscars:

> still the sample is as flexible as it was before, and the scars aren't deep.

>

> I have read that silicon oil is used fro transformers and that it doesn't

> attack most plastics.

>

> Do you know what is the oil you use for your capacitors ?

>

> Has anybody used PS in rolled capacitors ?

>

>

I have made two capacitors that use white polystyrene sheeting as a

dielectric. They have been standing in oil for over a YEAR with no

damage. The only visable change is a slight curling around the edges

(polyethylene does the same thing). I also have run them flat out on

12Kv with no breakdown (90mils betweeen plates,with 1/2" boarders). They

are vertical plate caps. I chose plate caps for their superior

performance. Also polystyrene is rather stiff compared to polyethylene.

I think the rest of the coilers have a biased opinion when it comes to

using anything but polyethylene in theirs caps. The performance was the

same a polyethylene cap. The types of oil I used was mineral oil

(laxative).

Frankensteins Helper

Max

Date: Mon, 19 Oct 1998 07:12:54 -0500

From: Bert Hickman <bert.hickman@aquila.com>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Organization: Stoneridge Engineering

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Polystyrene and oil?

Sender: tesl-err@emachine.com

Marco,

Most transformer oil today is simply specially refined and dried mineral

oil. From a solvent standpoint, it is similar to regular (not synthetic)

motor oil. While motor oil typically contains other additives which make

it a poor choice as a capacitor oil, you could perform a similar solvent

test with 10 weight regular motor oil and PS to see if there was any

effect. If the only effect is some absorbtion and swelling (as in LDPE

or PP), this is usually no problem. However, if the loss of transparency

you observed is actually due to microscopic surface cracking or

dissolving, then it will most likely lead to dielectric failure in your

capacitor over time.

Also, immersing a stressed piece of plastic in a solvent, such as

bending it to form a rolled cap, may accellerate cracking of the outer

surface. PS also tends to be relatively stiffer and more brittle than

LDPE, so a rolled cap may be considerably more difficult to construct.

Some on the main Tesla Builders list have made rolled caps using stiffer

materials (HDPE and PP), but only with significant difficulty during

construction.

Certain refridgeration oils are silicone based, have a relatively high

dielectric constant, and superior dielectric strength. Unfortunately,

these oils also tend to be significantly more expensive than regular

mineral oil-based transformer oil. Pure capacitor-grade castor oil is

even more expensive, but is an excellent dielectric oil for high

performance pulse caps. A flat plate cap made from PS and using either

type of oil should work if you can afford the oil to fill it. Or, if you

find that mineral oil does minimal solvent damage, use it and let us

know your results.

Safe cappin' to you!

-- Bert --

Marco Denicolai wrote:

>

> I have got 2 sheets 2 x 1 meters of Polystyrene (PS), 0.5mm (0.02") thick.

>

> Dielectric constant:                    2.5

> Dissipation factor (@ 1MHz):            0.0003

> Dielectric strength (0.2 mm thickness): >40 kV/mm  (i.e. >1016 kV/mil)

> Water absorption:                       <0.1%

>

> So I believed that was a feasible material for my capacitors until some of you

> mentioned that it cannot stand oil. A call to the manufacturer confirmed it:

> they spoke about it standing 4 hours, maximum one week.

>

> >From the web I found a table about PS were it was possible to see that PS stands

> SOME oils but not ALL oils. So I tested two samples of PS:

>

> A - clear PS, 0.25 mm thick

> B - white PS, 0.5 mm thick (the one for the capacitors)

>

> I used olive (!) oil and motor oil (full synthetic). I haven't got yet

> transformer oil!

>

> Several days at 20C degrees didn't change a thing in the samples. Then I put

> the samples in motor oil to bake into my furnace for 5 hours at 75C degrees.

> The B (white) sample didn't show any visible change. The A (clear) sample lost

> its transparency, became opaque.

>

> With the aid of a microscope I saw on the B sample a lot of microscars:

> still the sample is as flexible as it was before, and the scars aren't deep.

>

> I have read that silicon oil is used fro transformers and that it doesn't

> attack most plastics.

>

> Do you know what is the oil you use for your capacitors ?

>

> Has anybody used PS in rolled capacitors ?

>

>

>  Marco Denicolai                   Vista Communication Instruments, Inc.

>  Hardware Development Manager      www.vistacom.fi

>

>  marco@vistacom.fi

X-Mailer: IPAD 2.0

Date: Mon, 19 Oct 1998 12:07:41 -0500

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: Polystyrene and oil?

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

At 06:54 AM 10/19/98 -0500, you wrote:

>Marco Denicolai wrote:

<SNIP>

>> I used olive (!) oil and motor oil (full synthetic). I haven't got yet

>> transformer oil!

I wouldnt use a vegetable based oil as these will tend to spoil and

turn rancid. Motor oil (synthetic or otherwise) contains too many

metallic and semi-metallic additives to use as a dielectric oil.

>>

>> I have read that silicon oil is used for transformers and that it doesn't

>> attack most of plastics.

Actually most transformer oils are mineral oil based. They generally

have lower viscosity and contain some inhibiters to control moisture

absorption.

>>

>> Do you know what is the oil you use for your capacitors ?

Ive used pure mineral oil from the pharmacy and Shell Diala-AX

transformer oil.

>>

>> Has anybody used PS in rolled capacitors ?

Polystyrene would not work well in a rolled cap as its far too

stiff and will like crack if you tried to roll it tightly.

However it would be excellant for a flat plate cap.

>Then Roderick Maxwell wrote:

>   I have made two capacitors that use white polystyrene sheeting as a

>dielectric. They have been standing in oil for over a YEAR with no

>damage. The only visable change is a slight curling around the edges

>(polyethylene does the same thing). I also have run them flat out on

>12Kv with no breakdown (90mils betweeen plates,with 1/2" boarders). They

>are vertical plate caps. I chose plate caps for their superior

>performance. Also polystyrene is rather stiff compared to polyethylene.

>  I think the rest of the coilers have a biased opinion when it comes to

>using anything but polyethylene in theirs caps. The performance was the

>same a polyethylene cap. The types of oil I used was mineral oil

>(laxative).

>

I agree. I made two small flat plate caps (.0025 mfd) using polystyrene

in mineral oil and they are just fine after over 2 years. I see the

same type of edge curling that Roderick mentions and it is very similar

to what polyethylene does in oil.

Roderick I dont think coilers have a bias towards using anything but

polyethylene its just that poly is one of the best materials and

it is cheap and readily available. I would use anything that fits

that bill.

TTYL!

Mike Hammer

mhammer@misslink.net

From: EdwMullin@aol.com

Date: Mon, 19 Oct 1998 18:27:34 EDT

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Some very fine wire.

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

I recently came into about 6 partial rolls of enameled copper wire between

32-35 gauge.  I think it is too fine for a secondary but since it was the

right price (free!) I took it.  Anyone know if it would be useful for

anything as applied to coils??

X-Mailer: IPAD 2.0

Date: Mon, 19 Oct 1998 19:20:55 -0500

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: Some very fine wire.

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

At 06:27 PM 10/19/98 EDT, you wrote:

>I recently came into about 6 partial rolls of enameled copper wire between

>32-35 gauge.  I think it is too fine for a secondary but since it was the

>right price (free!) I took it.  Anyone know if it would be usefull for

>anything as applied to coils??

Ed,

Its about the right size for rewinding the secondary of a bad

neon sign transformer. They normally use AWG 30-32 for 60ma units

and AWG 34-36 for 30ma units.

It is pretty small for a secondary coil. However

I did once make a mini coil wound with 660 turns

of AWG 32. This was a 6 inch long winding. I ran

this with a 9kv/30ma neon and succeeded in getting

13 inch streamers from it using a 6 inch diameter

sphere as a terminal.

All attempts at using a 12kv/30ma neon on this coil

failed as the secondary broke down quite easily.

So you could make a secondary with that size of wire

but it certainly wont be a performer nor handle much power.

TTYL!

Mike Hammer

mhammer@misslink.net

X-Originating-IP: [216.68.41.72]

From: "Bill the arcstarter" <arcstarter@hotmail.com>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Some very fine wire.

Date: Mon, 19 Oct 1998 18:00:20 PDT

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

It was written:

>>I recently came into about 6 partial rolls of enameled copper wire between

>>32-35 gauge.  I think it is too fine for a secondary but since it was

<SNIP>

>Its about the right size for rewinding the secondary of a bad

>neon sign transformer. They normall use AWG 30-32 for 60ma units

>and AWG 34-36 for 30ma units.

<SNIP>

Perhaps it'd be good for winding chokes, either ferrite or air-core?

-Bill

X-Mailer: IPAD 2.0

Date: Tue, 20 Oct 1998 07:59:55 +0200

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Marco Denicolai <marco@vistacom.fi>

Subject: Re: Polystyrene and oil?

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

At 12:07 19/10/98 -0500, you wrote:

>At 06:54 AM 10/19/98 -0500, you wrote:

>>Marco Denicolai wrote:

>

><SNIP>

>

>>> I used olive (!) oil and motor oil (full synthetic). I haven't got yet

>>> transformer oil!

>

>I wouldnt use a vegetable based oil as these will tend to spoil and

>turn rancid. Motor oil (synthetic or otherwise) contains too many

>metallic and semi-metallic additives to use as a dielectric oil.

Yes, this was only a test to know did they mean it doesn't stand VEGETABLE oil?

>>> Do you know what is the oil you use for your capacitors ?

>

>Ive used pure mineral oil from the pharmacy and Shell Diala-AX

>transformer oil.

Also I found Shell Diala-AX from the web: let's see if I can also find a place

were to buy it here in Finland

<SNIP>

>Roderick I dont think coilers have a bias towards using anything but

>polyethylene its just that poly is one of the best materials and

>it is cheap and readily available. I would use anything that fits

>that bill.

>

>TTYL!

>

>Mike Hammer

>mhammer@misslink.net

>

See my next mail about availability here were I am!

Marco Denicolai                   Vista Communication Instruments, Inc.

Hardware Development Manager      www.vistacom.fi

marco@vistacom.fi

X-Mailer: IPAD 2.0

Date: Tue, 20 Oct 1998 08:31:33 +0200

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Marco Denicolai <marco@vistacom.fi>

Subject: HDPE found: check my capacitor, please

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

After calling nearly 30 companies here in Finland, I eventually found and

bought 2 sheets of black HPDE, 0.8 mm thick, 1 x 2 meters. The company

ensured me that black color is just pigment, not coal or other conductive stuff.

Declared properties are:

Dielectric constant:                    2.4

Dissipation factor (@ ? Hz):            0.0005

Dielectric strength (? mm thickness):   80 kV/mm  (i.e. >2036 kV/mil)

Water absorption:                       <0.01%

Here the companies are divided between packing foil manufacturer (thickness

around 0.05 mm) and sheet manufacturer (thickness from 1 mm up). PE about

0.5 mm thick was really hard to find. PVC would be available about anywere.

The material is stiff but I have already tried and I can bend it. I have

decided to go with it and to build two capacitors as follows:

- rolled caps, sheets 23 cm wide, 2 m long

- extended foil design with alumininum food packing foil

- I roll the foils around a 25 mm PP pipe were I have cut a long hole to

insert the foil's edge and keep it there. Same idea that cash machines

use to roll their printed paper bills

- Aluminium extended edges are bended about the PP pipe and fixed with

two hose clamps (Gary Lau design). Before to do this I cut away some

parts of the aluminium to allow for oil to reach the capacitor inside

- One of the clamps is taken through a copper strip to the upper

side of the capacitor, so I have both capacitor terminales on the same

side of the capacitor

- Then two bolt terminals and two small valves.

Foils are arranged as:

- paper    INNER LAYER

- Al

- paper

- PE

- paper

- PE

- paper

- Al

- paper

- PE

- paper

- PE      OUTER LAYER

Paper is 0.05 mm (40g/m2) kraft paper vaxed only on one side.

Questions:

- is the sheet design OK?

- is the overall design OK?

- were do you get those small (2-3 mm) valves?

- how much voltage do you believe my capacitor will stand?

- after vacuum and oil fill, should I:

A - fill the whole cap with oil

B - leave some vacuum empty space

C - let some air get in on the top of the oil (then I wouldn't need valves,

just hose fits)?

Marco Denicolai                   Vista Communication Instruments, Inc.

Hardware Development Manager      www.vistacom.fi

marco@vistacom.fi

From: wwl@netcomuk.co.uk (Mike Harrison)

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Some very fine wire.

Date: Tue, 20 Oct 1998 09:22:22 GMT

Organization: White Wing Logic

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

On Mon, 19 Oct 1998 18:27:34 EDT, you wrote:

>I recently came into about 6 partial rolls of enameled copper wire between

>32-35 gauge.  I think it is too fine for a secondary but since it was the

>right price (free!) I took it.  Anyone know if it would be usefull for

>anything as applied to coils??

make it into 6-strand litz wire for a secondary?

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Date: Tue, 20 Oct 1998 04:32:16 -0700

From: "paul m" <fleischwulf@mailexcite.com>

Cc:

X-Sent-Mail: off

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

Subject: Re: Polystyrene and oil?

X-Sender-Ip: 208.153.7.9

Organization: MailExcite  (http://www.mailexcite.com:80)

Sender: tesl-err@emachine.com

Marco,

Try Castor oil.

Paul M

---

Einigheit und Wahrheit

On Mon, 19 Oct 1998 12:07:41   Mike Hammer wrote:

>At 06:54 AM 10/19/98 -0500, you wrote:

>>Marco Denicolai wrote:

>

><SNIP>

>

>>> I used olive (!) oil and motor oil (full synthetic). I haven't got yet

>>> transformer oil!

>

>I wouldnt use a vegetable based oil as these will tend to spoil and

>turn rancid. Motor oil (synthetic or otherwise) contains too many

>metallic and semi-metallic additives to use as a dielectric oil.

>

>

>>>

>>> I have read that silicon oil is used for transformers and that it doesn't

>>> attack most of plastics.

>

>Actually most transformer oils are mineral oil based. They generally

>have lower viscosity and contain some inhibiters to control moisture

>absorption.

>

>>>

>>> Do you know what is the oil you use for your capacitors ?

>

>Ive used pure mineral oil from the pharmacy and Shell Diala-AX

>transformer oil.

>

>>>

>>> Has anybody used PS in rolled capacitors ?

>

>Polystyrene would not work well in a rolled cap as its far too

>stiff and will like crack if you tried to roll it tightly.

>

>However it would be excellant for a flat plate cap.

>

X-Mailer: IPAD 2.0

Date: Tue, 20 Oct 1998 13:13:08 -0500

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: HDPE found: check my capacitor, please

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

At 08:31 AM 10/20/98 +0200, you wrote:

>After calling nearly 30 companies here in Finland, I eventually found and

>bought 2 sheets of black HPDE, 0.8 mm thick, 1 x 2 meters. The company

>ensured me that black color is just pigment, not coal or other conductive stuff.

Marco,

I would be careful with this. Lamplack is a common black pigment

and would render the plastic useless for high voltage work.

It may be just fine but I would test it before using it to build a cap.

>

>Declared properties are:

>

>Dielectric constant:                    2.4

>Dissipation factor (@ ? Hz):            0.0005

>Dielectric strength (? mm thickness):   80 kV/mm  (i.e. >2036 kV/mil)

>Water absorption:                       <0.01%

I assume you meant 2.036 kv per mil and not 2036kv!

You will not realize that kind of rating in Tesla coil service.

Its most likely a DC rating and would have to be cut in half for

AC use and further derated for Tesla pulse duty.

Realistically you will get from 350-400 VAC per mil for polyethylene

in Tesla coil service.

<SNIP>

>

>Questions:

>

>- is the sheet design OK?

Yes but I would not attempt to build a rolled cap with

high density poly. Its too stiff and if you force it into

a roll there will probably be numerous stress fractures

that will lead to failure.

Instead use it for a flat plate cap. Flat plates offer the lowest inductance

and best pulse performance.

>- is the overall design OK?

>- were do you get those small (2-3 mm) valves?

Well I bought mine from McMaster Carr here in the USA but

that source doesnt help you any.

>- how much voltage do you believe my capacitor will stand?

Your 0.8 mm poly is right about 39 mils. At 350 volts per mil

that comes to 13650 volts AC. Thats plenty as you should never run

a single cap at any more than 10kv. In fact it is preferable to

keep the voltage per cap much lower than that. For voltages in excess

of 10kv you really must use a series string of caps to prevent the

formation of corona and resulting dielectric fatigue and failure.

>- after vacuum and oil fill, should I:

>

>  A - fill the whole cap with oil

>  B - leave some vacuum empty space

>  C - let some air get in on the top of the oil (then I wouldn't need valves,

>      just hose fits)?

>

Leave an air space above the cap. This gives you a cushion in case

of a failure.

TTYL!

Mike Hammer

mhammer@misslink.net

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Flat v.s. Rolled?

X-Mailer: IPAD 2.0

From: break7@juno.com (Kyle A. Break)

Date: Tue, 20 Oct 1998 23:55:19 EDT

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

I'm interested, What is the difference in performance between Rolled and

flat poly? I've always had flat stack caps and wonder if rolled caps have

a noticeable difference. They sure look a lot easier too build!

Is the spark length shorter? Are they harder to tune? Do they have a

lazy discharge? I would think they have more internal resistance. Two

tabs for discharge terminals as opposed to 40!  Might seem like no big

deal but at 100A  RF things might be different. Does anybody agree?

If it's not dangerous it's not fun!  Kyle  break7@juno.com

From:          "Malcolm Watts" <MALCOLM@directorate.wnp.ac.nz>

Organization:  Wellington Polytechnic, NZ

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Date:          Wed, 21 Oct 1998 17:04:23 +1200

Subject:       Re: Flat v.s. Rolled?

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

> I'm interested, What is the difference in performance between Rolled and

> flat poly? I've always had flat stack caps and wonder if rolled caps have

> a noticeable difference. They sure look a lot easier too build!

>

>   Is the spark length shorter? Are they harder to tune? Do they have a

> lazy discharge? I would think they have more internal resistance. Two

> tabs for discharge terminals as opposed to 40!  Might seem like no big

> deal but at 100A  RF things might be different. Does anybody agree?

Depends on whether they're extended foil or not. I get great

performance from a dry-fired 0.1uF extended foil cap run at 7-8kV

peak and 100BPS.

Malcolm

X-Mailer: IPAD 2.0

Date: Tue, 20 Oct 1998 23:26:57 -0500

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: Flat v.s. Rolled?

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

At 11:55 PM 10/20/98 EDT, you wrote:

>I'm interested, What is the difference in performance between Rolled and

>flat poly? I've always had flat stack caps and wonder if rolled caps have

>a noticeable difference. They sure look a lot easier too build!

>

>  Is the spark length shorter? Are they harder to tune? Do they have a

>lazy discharge? I would think they have more internal resistance. Two

>tabs for discharge terminals as opposed to 40!  Might seem like no big

>deal but at 100A  RF things might be different. Does anybody agree?

>

>	If it's not dangerous it's not fun!  Kyle  break7@juno.com

>

Kyle,

Flat plate caps generally give the best performance due

to their lower ESR (equivalent series resistance).

This is mostly due to the lower inductance and the shorter

discharge path.

The difference between a good flat plate cap and a good rolled

cap is visible but the difference is not great. The spark lengths

are pretty much the same but the streamers using a flat plate cap

are a bit brighter and snappier.

Ive built and used both kinds and prefer the flat plate kind but

am currently using a pair of .03 mfd rolled caps in series.

I am quite satisfied with the performance.

I made a post on this subject quite a while back on this

subject and I thought I would drag it out and repost it here.

Saves lots of typing. :)

<BEGIN INCLUDE>

Lets run through some of the pros and cons for each type.

Plate cap advantages

1. Great pulse discharge characteristics.

Plate caps have very low inductance so pulse very well.

If the best performance is what you're after then a plate

cap is the way to go.

2. Vertically oriented plates allow air to escape.

Plate caps with their plates arranged vertically will

allow trapped air to rise up out of the cap slowly.

This may take some time but is the best bet if you

dont have access to a vacuum pump.

3. Makes good use of thin dielectric.

Plate caps can use thin 4 or 6 mil hardware store variety

polyethylene. My .01 mfd plate cap uses 10 layers of 6 mil

poly for each dielectric layer.

Plate cap disadvantages.

1. Large physical size. My .01 mfd plate cap required a tank

17 X 13 X 2.5 inches to house it. This large tank required

1.25 gallons of mineral oil. Plate caps are not compact designs.

2. Long construction time. Cutting all the squares of poly and

plates require considerable time. Even the actual assembly

takes quite a while.

3. Tank construction takes considerable time. I was never able to

find a suitable tank for my plate caps so had to construct one.

These were made from plywood with a plexiglass inner liner.

Seams were sealed with silicone RTV.

Rolled cap advantages.

1. Smaller physical size. The fact that the plates are rolled

around upon themselves results in greater capacitance

for the same size plates resulting in a smaller more compact

cap for the same amount of capacitance.

2. Tanks for rolled caps easy to make. I've used PVC pipe for tanks

for my rolled caps. Cut a section of PVC pipe and epoxy plexiglass

end plates on it and there you go. Quick and easy tank.

3. Quick build time. The plates and poly are used in long strips

so require much less time to prepare. And the cap can be rolled

up in just a few minutes.

4. Lower oil requirements. By keeping the PVC pipe tanks only just

large enough to house the cap the amount of oil needed is much lower

than for plate caps. My recent .03 mfd rolled cap required only

1 1/2 pints of mineral oil.

Rolled cap disadvantages

1. High inductance. A rolled cap is after all a coil.

By rolling the conductors you introduce lots of inductance which

limits the pulse performance of the cap.

2. Lots of trapped air. Since the rolled type of cap has so many layers

and wound tightly it will trap more air than plate type caps.

And because of the tightness that air wont easily escape.

For this reason rolled caps should be vacuum pumped to help

remove trapped air. The use of real transformer oil is

recommended. Being much thinner than mineral oil it will

allow trapped air to escape easier.

<END INCLUDE>

The biggest drawback to rolled caps is the high amount

of inductance the rolled up plates represent. This can

be at least partly eliminated by using an extended foil

design. In this the entire length of the rolled plate

is brought out and connected to. This greatly reduces

inductance and discharge length. It also makes the cap

much more complex. Complex enough that Ive never made

one deciding instead on flat plate caps for low inductance.

Take a look at Gary Lau's web page for a good extended foil

rolled cap design. http://people.ne.mediaone.net/lau/tesla/capacitor.htm

TTYL!

Mike Hammer

mhammer@misslink.net

Date: Tue, 20 Oct 1998 23:39:44 -0500

From: Bert Hickman <bert.hickman@aquila.com>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Organization: Stoneridge Engineering

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Flat v.s. Rolled?

Sender: tesl-err@emachine.com

Kyle,

In theory, flat caps should provide better performance since they have

less equivalent series resistance and lower self inductance than an

equivalent rolled counterpart IF you only terminate at the ends. If

you've made an extended foil rolled cap, there's essentially NO

difference between the two styles. Because of the relatively larger

inductance and gap losses in the primary circuit, you'd probably be hard

pressed to tell very much performance difference between the

construction styles in a typical Tesla Coil.

-- Bert --

Kyle A. Break wrote:

>

> I'm interested, What is the difference in performance between Rolled and

> flat poly? I've always had flat stack caps and wonder if rolled caps have

> a noticeable difference. They sure look a lot easier too build!

>

>   Is the spark length shorter? Are they harder to tune? Do they have a

> lazy discharge? I would think they have more internal resistance. Two

> tabs for discharge terminals as opposed to 40!  Might seem like no big

> deal but at 100A  RF things might be different. Does anybody agree?

>

>         If it's not dangerous it's not fun!  Kyle  break7@juno.com

>

X-Mailer: IPAD 2.0

Date: Wed, 21 Oct 1998 08:47:15 +0200

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Marco Denicolai <marco@vistacom.fi>

Subject: Re: HDPE found: check my capacitor, please

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

At 13:13 20/10/98 -0500, you wrote:

>Marco,

>I would be careful with this. Lamplack is a common black pigment

>and would render the plastic useless for high voltage work.

>It may be just fine but I would test it before using it to build a cap.

I'll check this ASAP

>

>>

>>Declared properties are:

>>

>>Dielectric constant:                    2.4

>>Dissipation factor (@ ? Hz):            0.0005

>>Dielectric strength (? mm thickness):   80 kV/mm  (i.e. >2036 kV/mil)

>>Water absorption:                       <0.01%

>

>

>I assume you meant 2.036 kv per mil and not 2036kv!

Yes, sorry. Read it as 2036 V/mil. The manufacturer didn't know at what

thickness that value was tested. My PS manufacturer stated for the PS 40 kV/mm

@ 0.2 mm so maybe HDPE was tested @ 0.05 mm ?

>

>You will not realize that kind of rating in Tesla coil service.

>Its most likely a DC rating and would have to be cut in half for

>AC use and further derated for Tesla pulse duty.

>

>Realistically you will get from 350-400 VAC per mil for polyethylene

>in Tesla coil service.

>

Yes, I realize that 2036 V/mil is an unreal dream. So you suggest 350 Vac/mil

would be a conservative value?

>>

>>- is the sheet design OK?

>

>Yes but I would not attempt to build a rolled cap with

>high density poly. Its too stiff and if you force it into

>a roll there will probably be numerous stress fractures

>that will lead to failure.

Let's see :)

>>- were do you get those small (2-3 mm) valves?

>

>Well I bought mine from McMaster Carr here in the USA but

>that source doesnt help you any.

Ok, but I mean WHAT are they? Car radiator valves, chemical instruments valves,

what ?

>

>>- how much voltage do you believe my capacitor will stand?

>

>Your 0.8 mm poly is right about 39 mils. At 350 volts per mil

>that comes to 13650 volts AC. Thats plenty as you should never run

>a single cap at any more than 10kv. In fact it is preferable to

>keep the voltage per cap much lower than that. For voltages in excess

>of 10kv you really must use a series string of caps to prevent the

>formation of corona and resulting dielectric fatigue and failure.

1 mil = 0.0254 mm thus 0.8 mm = 31.5 mil thus  Vmax = 11023 Vac

I mean, what about the old capacitor building specs suggesting to

use 90 mil for 14 kVac ?

Why there is SO BIG difference in these construction issues?

Who is right?

For me it is a BIG difference to roll 2 sheets of HDPE or 4 sheets of

it. I wouldn't go for 4 if it is not really necessary.

Marco Denicolai                   Vista Communication Instruments, Inc.

Hardware Development Manager      www.vistacom.fi

marco@vistacom.fi

X-Mailer: IPAD 2.0

Date: Wed, 21 Oct 1998 11:47:51 -0500

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: HDPE found: check my capacitor, please

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

At 08:47 AM 10/21/98 +0200, you wrote:

>>>Declared properties are:

>>>

>>>Dielectric constant:                    2.4

>>>Dissipation factor (@ ? Hz):            0.0005

>>>Dielectric strength (? mm thickness):   80 kV/mm  (i.e. >2036 kV/mil)

>>>Water absorption:                       <0.01%

>>

>>

>>I assume you meant 2.036 kv per mil and not 2036kv!

>

>Yes, sorry. Read it as 2036 V/mil. The manufacturer didn't know at

>what thickness that value was tested. My PS manufacturer stated for

>the PS 40 kV/mm @ 0.2 mm so maybe HDPE was tested @ 0.05 mm ?

>

>>

>>You will not realize that kind of rating in Tesla coil service.

>>Its most likely a DC rating and would have to be cut in half for

>>AC use and further derated for Tesla pulse duty.

>>

>>Realistically you will get from 350-400 VAC per mil for polyethylene

>>in Tesla coil service.

>>

>

>Yes, I realize that 2036 V/mil is an unreal dream. So you suggest 350

>Vac/mil would be a conservative value?

Yes this is quite conservative with a generous safety margin.

Ive run 24 mil caps quite successfully at 9kv. That comes

to 375 V/mil. With pure virgin material AND spreading the stress

across several series caps you could realize 400 V/mil.

I verified this voltage rating by destroying dozens of small test caps.

And some not so small non-test caps.  :(

>

>>>

>>>- is the sheet design OK?

>>

>>Yes but I would not attempt to build a rolled cap with

>>high density poly. Its too stiff and if you force it into

>>a roll there will probably be numerous stress fractures

>>that will lead to failure.

>

>Let's see :)

>

>>>- were do you get those small (2-3 mm) valves?

>>

>>Well I bought mine from McMaster Carr here in the USA but

>>that source doesnt help you any.

>

>Ok, but I mean WHAT are they? Car radiator valves, chemical instruments >valves, what ?

Just small brass valves to fit 1/4 inch pipe thread.

These are not high vacuum valves but you dont need

high vacuum for pumping down a cap.

>

>>

>>>- how much voltage do you believe my capacitor will stand?

>>

>>Your 0.8 mm poly is right about 39 mils. At 350 volts per mil

>>that comes to 13650 volts AC. Thats plenty as you should never run

>>a single cap at any more than 10kv. In fact it is preferable to

>>keep the voltage per cap much lower than that. For voltages in excess

>>of 10kv you really must use a series string of caps to prevent the

>>formation of corona and resulting dielectric fatigue and failure.

>

>1 mil = 0.0254 mm thus 0.8 mm = 31.5 mil thus  Vmax = 11023 Vac

Oops. Guess I messed up that conversion. My feeble old

brain just doesnt compute in metric. You are correct.

>

>I mean, what about the old capacitor building specs suggesting to

>use 90 mil for 14 kVac ?

>

>Why there is SO BIG difference in these construction issues?

>

>Who is right?

This subject of poly thickness for various voltages goes

way back to the original rolled cap design by Richard Hull

and the TCBOR group. They used aluminum flashing and 60 mil

poly. Many attempting to duplicate the design used these caps

at voltages up to 15kv despite the fact that they were never

designed or meant to be used above 10kv.

As these caps failed constructors mistakely assumed the problem

was insufficient poly thickness so caps were built using 90 mils

and even 120 mils of poly. Many of these caps also failed because

their builders failed to realize what the failure mode was.

These caps failed because of corona forming along the edges and

ends of the plates. At voltages much above 10kv corona will form

EVEN UNDER OIL. Caps subjected to this damaging corona will

ultimately fail NO MATTER WHAT THE POLY THICKESS IS.

The only way to have truly reliable caps is to use a series

string of caps to keep the voltage across each individual cap

below 10kv. In fact it would be even better to keep the voltage

well below 7500 V per cap. This calls for a MINIMUM of 2 series caps

for 12kv and 3 in series for 15kv.

Once you start using a series string of caps you will find you

do not need massively thick dielectrics. My .015 mfd rolled cap

consists of two 24 mil .03 mfd caps in series. This is the equivalent

of 48 mils. Ive run this cap extensively at 12720 volts with no problems.

Ive run four 15 mil caps (60 mil total) in series successfully at 15kv.

Many coilers still have not learned the lesson and are still recommending

90 + mil single caps for 15kv use that will ultimately fail.

Every one of the series caps I've built survive and work today.

My first flat plate cap consisting of six 8 mil caps in series

works fine today at over 3 years of age. All of the single caps

that I built failed.

TTYL!

Mike Hammer

mhammer@misslink.net

X-Mailer: IPAD 2.0

Date: Wed, 21 Oct 1998 15:59:38 -0500

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Mike Hammer <mhammer@misslink.net>

Subject: some new projects

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Hello all,

Its been quite a while since I did any serious coiling

due to a number of disasters both natural and personal

over the past summer.

But with that all behind me Ive begun to do a little experimenting

once again. So since list traffic has been a little low I thought

to share some of my latest endeavors with the list.

First off I acquired a spool of AWG 27 rather cheaply at

a hamfest and used some of it to wind a new experimental

secondary. It was wound on 4 inch PVC and the winding measures

4.5 X 20.5 inches. This is about 1375 turns and the measured

frequency with a 19 X 6 toroid was 174 KHZ.

I know. I always say not to use more than 1000 turns. But this

secondary is for a special purpose and I needed the low resonant

frequency.

I tried this secondary out on my main coil with my .015 mfd rolled

caps and not suprisingly could not get it anywhere near resonance.

I added my .008 mfd plate in parallel for a total of .023 mfd

and with all 17 primary turns tapped in I was just short of

reaching resonance. Performance was fair at just under 44 inches.

It would have been better if I had another primary turn or two

to tap in.

One thing became readily apparent. With this level of primary

capacitance I really needed a larger toroid. My 19 X 6 toroid

was breaking down with 6 to 8 simultaneous streamers. Even

laying a piece of coat hanger off the side of the toroid didnt help.

I ended the tests at that point as I made this secondary to go

with a new coil project Im starting. I am going to do some experiments

with a pair of MOTs to see if and how they can be adapted for efficient

use on a coil. There will be many problems and much experimentation needed.

My second project was a new spark gap. This gap was inspired by the gap

design Gary Lau has on his web page. Check it out at: http://people.ne.mediaone.net/lau/tesla/onegap.htm

This gap was cheap ($4.75) and quite easy to build.

The tests Ive run with it have shown it to be a good

performer as long as sufficient airflow is provided.

Used on my old coil setup I achieved 48 inch streamers

with a gap setting of .250 inch. This is somewhat more gap

than my vacuum quenched cylinder gap provides. (.225 inch)

I can easily exceed 50 inches with the cylinder gap despite

its smaller gap distance.

Quenching was good as long as the coupling was not too tight.

At my normal coupling level of around .22 the quenching was

pretty much at the 2nd notch dropping to 3rd notch at low

variac settings. At a coupling of .18 first notch quenching

was shown but performance suffered. My old gap provides first

notch quenching all the time.

Conclusion is that this is a pretty good gap. Its performance

is nearly as good as my best gap and was very cheap and quick

to build.

More to come.

Comments and/or criticism always welcome.

TTYL!

Mike Hammer

mhammer@misslink.net

From: Hollmike@aol.com

Date: Wed, 21 Oct 1998 19:40:37 EDT

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: some new projects

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Mike,

Thanks for the update.  I may have to try out that style of gap yet.  I

have not yet tried out the one I have been sloooowly working on, but may try

this one out at the same time with my new coil.

Mike Hollingsworth

X-Originating-IP: [207.100.20.23]

From: "Philip Mac Duffie" <punk_42@hotmail.com>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: NST protection

Date: Wed, 21 Oct 1998 18:28:29 PDT

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

I guess im really paranoid about protecting my nst, perhaps due to the

fact it took 3 months to find one (graciously donated by a friend) I

cant buy a new one in the crummy town I live in so I have to protect

what I have. I know the NST will eventuly die maybe mine faster because

its so fragile (France 9kv 30ma. I was reading on Gary Lau's page about

chokes causeing high voltage spikes and killling the nst. I was wondering

if anyone had luck with resistors and bypass caps only?

X-Mailer: IPAD 2.0

Date: Wed, 21 Oct 1998 20:58:30 -0500

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: NST protection

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

At 06:28 PM 10/21/98 PDT, you wrote:

>I guess im really paranoid about protecting my nst, perhaps due to the

>fact it took 3 months to find one (graciously donated by a friend) I

>cant buy a new one in the crummy town I live in so I have to protect

>what I have. I know the NST will eventuly die maybe mine faster because

>its so fragile (France 9kv 30ma. I was reading on Gary Lau's page about

>chokes causeing high voltage spikes and killing the nst. I was wondering

>if anyone had luck with resistors and bypass caps only?

>

Neon protection is one of those highly debated topics

with no real definitive answer. Everyone has their own pet

theories and solution.

I personally think way too much is made over RF protection.

Its overvoltage - RF or otherwise - that kills neons and

the best way to deal with overvoltage is with a safety gap.

Make sure you have the main gap across the transformer and not

your cap. Use a safety gap set to breakdown just above the transformer

rating. Make sure the main gap is not set too wide. Over wide gaps

probably account for most neon failures.

RF chokes can help but are not strictly necessary. The use of

damping resistors is recommended in conjunction with RF chokes

to prevent ringing and high voltage spikes. I do not recommend

bypass caps as I have seen these destroy neons in short order.

TTYL!

Mike Hammer

mhammer@misslink.net

From:          "Malcolm Watts" <MALCOLM@directorate.wnp.ac.nz>

Organization:  Wellington Polytechnic, NZ

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Date:          Thu, 22 Oct 1998 15:07:49 +1200

Subject:       Re: NST protection

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

> I guess im really paranoid about protecting my nst, perhaps due to the

> fact it took 3 months to find one (graciously donated by a friend) I

> cant buy a new one in the crummy town I live in so I have to protect

> what I have. I know the NST will eventuly die maybe mine faster because

> its so fragile (France 9kv 30ma. I was reading on Gary Lau's page about

> chokes causeing high voltage spikes and killling the nst. I was wondering

> if anyone had luck with resistors and bypass caps only?

Yes.

Malcolm

From: McFalc0n@aol.com

Date: Wed, 21 Oct 1998 23:08:52 EDT

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Secondary Requirements

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

i just wound my secondary on a 4" PVC pipe.  I had trouble winding it.

I ended up doing some by hand and the rest by turning it with a drill.

I was wondering how perfect the turns have to be.  I couldn't make all of the

turns equually tight, and I have 2 imperfactions in the wire, but I do not

have any out of turn wire.  It's not messy, or perfect.  Does anyone think

this secondary will not work.

Thanx

Date: Wed, 21 Oct 1998 22:32:00 -0500

From: Bert Hickman <bert.hickman@aquila.com>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Organization: Stoneridge Engineering

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Secondary Requirements

Sender: tesl-err@emachine.com

McFalc0n@aol.com wrote:

>

>         I just wound my secondary on a 4" PVC pipe.  I had troublae winding it.

>  I ended up doing some by hand and the rest by turning it with a drill.

>  I was wondering how perfect the turns have to be.  i couldn't make all of the

> turns equually tight, and I have 2 imperfactions in the wire, but I do not

> have any out of turn wire.  It's not messy, or perfect.  Does anyone think

> this secondary will not work.

>                                                                                 Thanx

Perfection is not necessary. Wider gaps are no problem, but overlapping

turns can be a problem at higher operating voltages. Also, small "kinks"

in the wire should be OK as long as they don't project outwards to the

point where they become sources for corona. If you have some of these,

try to position the secondary so that these are kept towards the bottom

half of the secondary, and if necessary add a dab of insulating varnish

or anti-corona dope over these to provide some more protection.

Safe coilin' to you!

-- Bert --

Date: Thu, 22 Oct 1998 00:04:26 -0700

From: Scott MacGregor <smacgregor@aasp.net>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Organization: EES,Co.

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Secondary Requirements

Sender: tesl-err@emachine.com

Bert Hickman wrote:

>

> McFalc0n@aol.com wrote:

> >

> >       I just wound my secondary on a 4" PVC pipe.  I had troublae winding it.

> >  I ended up doing some by hand and the rest by turning it with a drill.

> >  I was wondering how perfect the turns have to be.  I couldn't make all of the

> > turns equually tight, and I have 2 imperfactions in the wire, but I do not

> > have any out of turn wire.  It's not messy, or perfect.  Does anyone think

> > this secondary will not work.

> >                             Thanx

>

> Perfection is not necessary. Wider gaps are no problem, but overlapping

> turns can be a problem at higher operating voltages. Also, small "kinks"

> in the wire should be OK as long as they don't project outwards to the

> point where they become sources for corona. If you have some of these,

> try to position the secondary so that these are kept towards the bottom

> half of the secondary, and if necessary add a dab of insulating varnish

> or anti-corona dope over these to provide some more protection.

>

> Safe coilin' to you!

>

> -- Bert --

Hard to remember how many flybacks this stuff saved.....

--

Regards,

Scott MacGregor

X-Mailer: IPAD 2.0

Date: Thu, 22 Oct 1998 08:36:26 +0200

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Marco Denicolai <marco@vistacom.fi>

Subject: Re: HDPE found: check my capacitor, please

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

At 11:47 21/10/98 -0500, you wrote:

<SNIP>

>Once you start using a series string of caps you will find you

>do not need massively thick dielectrics. My .015 mfd rolled cap

>consists of two 24 mil .03 mfd caps in series. This is the equivalent

>of 48 mils. Ive run this cap extensively at 12720 volts with no problems.

>Ive run four 15 mil caps (60 mil total) in series successfully at 15kv.

>

>Many coilers still have not learned the lesson and are still recommending

>90 + mil single caps for 15kv use that will ultimately fail.

>

>Every one of the series caps I've built survive and work today.

>My first flat plate cap consisting of six 8 mil caps in series

>works fine today at over 3 years of age. All of my single caps failed.

>

>TTYL!

>

>

>

>Mike Hammer

>mhammer@misslink.net

>

Thanks Mike. I think this mail of your is pure gold for many on the list.

I'll build 2 (or 4 if I have "time") capacitors with a single layer of 0.8 mm

(31 mils) HDPE and will mark them as "10kV DC" devices. I will then use them

in series. Easier to roll 2 sheets of HDPE per cap then 4 sheets!

As finally the mystery of the 90 mils revealed!

Marco Denicolai                   Vista Communication Instruments, Inc.

Hardware Development Manager      www.vistacom.fi

marco@vistacom.fi

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: "Coiler" <mycroft@access1.net>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Some tests with a 2.25" coil

Date: Wed, 21 Oct 1998 22:47:33 -0700

X-Mailer: IPAD 2.0

Importance: Normal

X-MimeOLE: Produced By Microsoft MimeOLE V4.72.3155.0

Sender: tesl-err@emachine.com

As some of you know, I have been fighting a few battles with my system -

one of which is protecting my door opener :)

The other one has been working with my *expletive* capactitor safety gap.

Since I could not run as high power, I grabbed one of my old coils

to stick in the primary as a load.. it happened to be my 2.25 x 12"

coil, wound with 28AWG wire. I threw my old 3"x10" toroid on there

mostly to get to where I could at least use a few turns of my primary.

The little coil looked lost in there :) [ 8.5" inner turn for my 6.5"

coil]

Specs for test:

2.25x12 Coil, with 3x10 Topload.

Main Gap 0.175

Cap Gap 0.470

Power source: 14.4 Pig, with input voltage limited to 100V (12KV output)

1100VA limiting in place.

Doing short runs (under 5 seconds) things were fine. Produced anywhere from

2-6 14" streamers. After I got things tuned up I let the system run for a bit

(about 20 seconds) when my cap safety gap started to go nuts. I checked my main

gap, and the pipes were *HOT* (my kids had broken my fan plenum - so the air

was not ducted to my RQ gap.) So I got curious. I let it cool down, and fired

up the coil.. things ran fine for about 30 seconds... then *bang* the

safety gap fires.. Again, the gap is hot.

I am speculating that the poor quenching caused by the overheated gap is playing

a role in the problems I am seeing with my safety gap, can any of you give some

additional information in support of or against this idea?

The only other possiblity (which I have not been able to test yet, wife made

me shut down because of the noise) is that the new enclosure I built for my

caps is harboring ionized air from the cap gap.. and that this eventually

leads to the breakdown.

Michael Baumann

Coiler, Homebrewer, Nerd. mycroft@access1.net

X-Mailer: IPAD 2.0

Date: Thu, 22 Oct 1998 09:28:51 +0200

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Marco Denicolai <marco@vistacom.fi>

Subject: Black HDPE testing

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

I used a 8kV 50mA NST to test my 0.8 mm (31 mils) thick black HDPE sheet.

I put a 3 x 3 cm slice of HDPE between two bolts connected to the NST

without a variac. THERE WAS NO ARCING. The HDPE I have got is NOT

conductive!

In complete darkness I could see a slight violet corona around the bold

contacting the HDPE. The nice thing is that as I connected/disconnected

rapidly the NST from the mains, it generated extravoltage that was capable

to jump OVER the HDPE and reach the other bolt! This accounted for a

distance of about 3 cm.

I could also jump 3 cm of air between bolds (with no HDPE) by doing the

same connect/disconnect trick.

This tells me that the 0.8 mm HDPE exceeds 8kV AC of isolation (forgetting

the corona effect, of course) but tells me also that it is safe to have a

safety gap on each cap.

I have also decided to have only a layer of HDPE, not two. My cap design

is now as follows:

- paper    INNER LAYER

- Cu

- paper

- PE

- paper

- Cu

- paper

- PE      OUTER LAYER

Paper is 0.05 mm (40g/m2) kraft paper not waxed.

PE is 0.8 mm (31 mil) HDPE

Cu is 0.1 mm copper sheet

capacitor is filled with oil

I am confident this cap will stand 10 kV DC and connecting two of them in

series I should be able to use them in a typical TC. I will also mount on

each cap a safety gap set as, say, 1 cm. Leaving 2 cm of PE border without

copper inside the cap should ensure that the safety gap fires BEFORE I get

arcing between the copper electrodes edges.

Comments?

Marco Denicolai                   Vista Communication Instruments, Inc.

Hardware Development Manager      www.vistacom.fi

marco@vistacom.fi

Date: Thu, 22 Oct 1998 11:19:14 +0100

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Stephen Bell <steve@breakfix.demon.co.uk>

Subject: Re: Black HDPE testing

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

In message <2.2.32.19981022072851.007165d0@mail.vistacom.fi>, Marco

Denicolai <marco@vistacom.fi> writes

>I used a 8kV 50mA NST to test my 0.8 mm (31 mils) thick black HDPE sheet.

>

>I put a 3 x 3 cm slice of HDPE between two bolts connected to the NST

>without a variac. THERE WAS NO ARCING. The HDPE I have got is NOT

>conductive!

>

I bought some polyethylene tubing, to use as a secondary coil form, but

when it arrived it was BLACK. Remembering comments on the list about

carbon black as a coloring agent I contacted the supplier, only to find

out it DID contain carbon! :-(

I too tested it with my NST (10kV @ 100mA) and could not detect any sign

of a problem. I did the same tests with some expanded polystyrene and

got identical results! :-/

Maybe the problem only occurs at radio frequencies.

I friend of mine, into ham radio, tested the insulators he used by

placing a sample in a microwave oven.  If it got warm then it was no

good.

------------------------------------------------

Stephen Bell        (steve@breakfix.demon.co.uk)

Bringing Tesla Excitement to Swindon, England

the land of 240VAC @ 50Hz

Stuff also available at www.breakfix.demon.co.uk

------------------------------------------------

X-Mailer: IPAD 2.0

Date: Thu, 22 Oct 1998 13:38:39 +0200

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Marco Denicolai <marco@vistacom.fi>

Subject: Re: Black HDPE testing

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

At 11:19 22/10/98 +0100, you wrote:

>I friend of mine, into ham radio, tested the insulators he used by

>placing a sample in a microwave oven.  If it got warm then it was no

>good.

>

>

>------------------------------------------------

>Stephen Bell        (steve@breakfix.demon.co.uk)

All right, that's going to be tonight's test. Let's see

how it goes...

Marco Denicolai                   Vista Communication Instruments, Inc.

Hardware Development Manager      www.vistacom.fi

marco@vistacom.fi

Date: Thu, 22 Oct 1998 06:08:23 -0500

From: Bert Hickman <bert.hickman@aquila.com>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Organization: Stoneridge Engineering

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Black HDPE testing

Sender: tesl-err@emachine.com

Marco Denicolai wrote:

>

> I used a 8kV 50mA NST to test my 0.8 mm (31 mils) thick black HDPE sheet.

>

> I put a 3 x 3 cm slice of HDPE between two bolts connected to the NST

> without a variac. THERE WAS NO ARCING. The HDPE I have got is NOT

> conductive!

>

> In complete darkness I could see a slight violet corona around the bold

> contacting the HDPE. The nice thing is that as I connected/disconnected

> rapidly the NST from the mains, it generated extravoltage that was capable

> to jump OVER the HDPE and reach the other bolt! This accounted for a

> distance of about 3 cm.

>

> I could also jump 3 cm of air between bolds (with no HDPE) by doing the

> same connect/disconnect trick.

>

> This tells me that the 0.8 mm HDPE exceeds 8kV AC of isolation (forgetting

> the corona effect, of course) but tells me also that it is safe to have a

> safety gap on each cap.

>

> I have also decided to have only a layer of HDPE, not two. My cap design

> is now as follows:

>

> - paper    INNER LAYER

> - Cu

> - paper

> - PE

> - paper

> - Cu

> - paper

> - PE      OUTER LAYER

>

> Paper is 0.05 mm (40g/m2) kraft paper not waxed.

> PE is 0.8 mm (31 mil) HDPE

> Cu is 0.1 mm copper sheet

> capacitor is filled with oil

>

> I am confident this cap will stand 10 kV DC and connecting two of them in

> series I should be able to use them in a typical TC. I will also mount on

> each cap a safety gap set as, say, 1 cm. Leaving 2 cm of PE border without

> copper inside the cap should ensure that the safety gap fires BEFORE I get

> arcing between the copper electrodes edges.

>

> Comments?

>

>

>  Marco Denicolai                   Vista Communication Instruments, Inc.

>  Hardware Development Manager      www.vistacom.fi

>

>  marco@vistacom.fi

Marco,

It tells you only that a small section of the material can withstand 8

KV at 50/60 Hz for short times. High voltage RF is a much nastier beast.

It will seek out, and find, ANY weakness in your dielectric system,

focus heating and ionization damage on this spot, and punch its way

through with time. And all dielectric materials have defects! Multiple

thin dielectric sheets prevent expansion and propagation of defects in

any given sheet from "migrating" to the other sheet, stopping the

destrutive process.

Using only one dielectric sheet will increase the odds that a small

defect will cause the single layer to fail. If two identical caps are

connected in series, the failure of first one will overvolt and cause

the failure of the other. For internal reliability, the KEY is to use

multiple thinner dielectrics in EACH cap, and then connecting identical

caps in series to share the total voltage. Using a single dielectric

layer will result in a considerably less reliable capacitor. With the

high voltages, RF frequencies, and large voltage reversals, Tesla Coil

service is the most demanding application any capacitor can see. The

multiple dielectric/series capacitor construction "formula" that Mike

and I have been describing works well, and is the "secret" that ALL HV

RF Capacitor vendors use today.

Copper electrodes will work the same as aluminum, but are considerably

more expensive and offer no performance advantage. The only advantage

with copper is that you can more easily solder to it than aluminum.

-- Bert --

X-Mailer: IPAD 2.0

Date: Thu, 22 Oct 1998 15:10:34 +0200

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Marco Denicolai <marco@vistacom.fi>

Subject: Re: Black HDPE testing

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

At 06:08 22/10/98 -0500, you wrote:

>Using only one dielectric sheet will increase the odds that a small

>defect will cause the single layer to fail. If two identical caps are

>connected in series, the failure of first one will overvolt and cause

>the failure of the other. For internal reliability, the KEY is to use

>multiple thinner dielectrics in EACH cap, and then connecting identical

>caps in series to share the total voltage. Using a single dielectric

>layer will result in a considerably less reliable capacitor. With the

>high voltages, RF frequencies, and large voltage reversals, Tesla Coil

>service is the most demanding application any capacitor can see. The

>multiple dielectric/series capacitor construction "formula" that Mike

>and I have been describing works well, and is the "secret" that ALL HV

>RF Capacitor vendors use today.

Sigh! But I have got also paper layers... Wouldn't that be enough together

with one layer of HDPE?

Or what about adding also one layer of 0.2 mm (7.9 mil) LDPE instead of a

second layer 0.8 mm (31 mil) of HDPE?

>

>Copper electrodes will work the same as aluminum, but are considerably

>more expensive and offer no performance advantage. The only advantage

>with copper is that you can more easily solder to it than aluminum.

>

>-- Bert --

>

I know, copper sheet is $10/meter here. The reason is that here you can buy

aluminum only 0.3 mm thick or thicker (too tick) and in small sheets, or it

is that tiny foil for food (very easy to break). 0.1 mm thick copper is an optimal

compromise, besides for your pocket.

Marco Denicolai                   Vista Communication Instruments, Inc.

Hardware Development Manager      www.vistacom.fi

marco@vistacom.fi

Date: Thu, 22 Oct 1998 11:53:54 -0700

From: tomstock <tomstock@geocities.com>

X-Mailer: IPAD 2.0

X-Accept-Language: en

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: An odd tesla coil at MOSI

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

I looked at MOSI's (Museum of Science and Industry) tesla coil here in

Tampa FL. Now that I actually know something about Tesla coil design,

I've realized that this "3 million volt" tesla coil is actually a big

piece of doodoo. Here's what I saw:

Primary, 4 turns of 1/2" copper tubing, approximately 24" ID x 26" OD

Secondary

500 turns of 12 AWG rubber insulated wire

16" diameter x 6 feet long STEEL PIPE form.

Discharge terminal:

Metal spike, 24" long, 1" in diameter.

Maximum discharge seemed to be about 36". Thought you might find this

interesting. The secondary form really had me wondering.

-tom

From:          "Malcolm Watts" <MALCOLM@directorate.wnp.ac.nz>

Organization:  Wellington Polytechnic, NZ

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Date:          Fri, 23 Oct 1998 09:28:18 +1200

Subject:       Re: Some tests with a 2.25" coil

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Hi Coiler,

Thanks very much for the info on your safety gap

behaviour. I am compiling a list of safety gap observations as I am

currently obsessed with trying to determine why these sorts of

problems are happening. I am far from reaching any conclusion yet.

After previous speculations turned out to be nogo, I am approaching

this with an open mind and simply collecting data with the hope that

this problem might be solved with a concerted attack on it.

Anyone else with safety gap behaviour observations, please post

(to me if it is a problem for the list.

Thanks again,

Malcolm

> As some of you know, I have been fighting a few battles with my system -

> one of which is protecting my door opener :)

>

> The other one has been working with my *expletive* capactitor safety gap.

> Since I could not run as high power, I grabbed one of my old coils

> to stick in the primary as a load.. it happened to be my 2.25 x 12"

> coil, wound with 28AWG wire. I threw my old 3"x10" toroid on there

> mostly to get to where I could at least use a few turns of my primary.

> The little coil looked lost in there :) [ 8.5" inner turn for my 6.5"

> coil]

>

> Specs for test:

> 2.25x12 Coil, with 3x10 Topload.

> Main Gap 0.175

> Cap Gap 0.470

> Power source: 14.4 Pig, with input voltage limited to 100V (12KV output)

>                1100VA limiting in place.

>

> Doing short runs (under 5 seconds) things were fine. Produced anywhere from

> 2-6 14" streamers. After I got things tuned up I let the system run for a

> bit

> (about 20 seconds) when my cap safety gap started to go nuts. I checked my

> main

> gap, and the pipes were *HOT* (my kids had broken my fan plenum - so the air

> was not ducted to my RQ gap.) So I got curious. I let it cool down, and

> fired

> up the coil.. things ran fine for about 30 seconds... then *bang* the

> safety gap fires.. Again, the gap is hot.

>

> I am speculating that the poor quenching caused by the overheated gap is

> playing a role in the problems I am seeing with my safety gap, can any of you

> give some additional information in support of or against this idea?

> The only other possiblity (which I have not been able to test yet, wife made

> me shut down because of the noise) is that the new enclosure I built for my

> caps is harboring ionized air from the cap gap.. and that this eventually

> leads to the breakdown.

>

> Michael Baumann

> Coiler, Homebrewer, Nerd. mycroft@access1.net

>

>

>

From:          "Malcolm Watts" <MALCOLM@directorate.wnp.ac.nz>

Organization:  Wellington Polytechnic, NZ

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Date:          Fri, 23 Oct 1998 09:32:48 +1200

Subject:       Re: Black HDPE testing

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

> Date sent:      Thu, 22 Oct 1998 11:19:14 +0100

> To:             "Tesla-2 Mailing List" <tesla-2@emachine.com>

> From:           Stephen Bell <steve@breakfix.demon.co.uk>

> Subject:        Re: Black HDPE testing

> Send reply to:  "Tesla-2 Mailing List" <tesla-2@emachine.com>

> In message <2.2.32.19981022072851.007165d0@mail.vistacom.fi>, Marco

> Denicolai <marco@vistacom.fi> writes

> >I used a 8kV 50mA NST to test my 0.8 mm (31 mils) thick black HDPE sheet.

> >

> >I put a 3 x 3 cm slice of HDPE between two bolts connected to the NST

> >without a variac. THERE WAS NO ARCING. The HDPE I have got is NOT

> >conductive!

> >

>

> I bought some polyethylene tubing, to use as a secondary coil form, but

> when it arrived it was BLACK. Remembering comments on the list about

> carbon black as a coloring agent I contacted the supplier, only to find

> out it DID contain carbon! :-(

>

> I too tested it with my NST (10kV @ 100mA) and could not detect any sign

> of a problem. I did the same tests with some expanded polystyrene and

> got identical results! :-/

>

> Maybe the problem only occurs at radio frequencies.

Or at far higher voltages typical of an unloaded TC output. The

carbon in the pipe is (as I understand) supposed to turn it into a

high value resistance capable of bleeding static charges away (slowly

obviously) to prevent the possibility of gas explosions in normal use.

Of course such a static charge would build up over time, not be

applied quickly so the resistance requirements would not be that low.

Malcolm

From: Hollmike@aol.com

Date: Thu, 22 Oct 1998 20:22:28 EDT

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Secondary Requirements

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

In a message dated 10/21/98 9:11:46 PM Mountain Daylight Time,

McFalc0n@aol.com writes:

> I just wound my secondary on a 4" PVC pipe.  i had trouble winding it.  I

>  ended up doing some by hand and the rest by turning it with a drill.

>  I was wondering how perfect the turns have to be. I couldn't make all of the

>  turns equually tight, and I have 2 imperfactions in the wire, but I do not

>  have any out of turn wire.  It's not messy, or perfect.  Does anyone think

>  this secondary will not work.

>  										Thanx

>

It should do fine if there are no wires actually crossing each other.  You

wound it already, so why not go ahead and try it out?

X-Mailer: IPAD 2.0

Date: Fri, 23 Oct 1998 01:50:21 +0100

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: bob golding <yubba@clara.net>

Subject: Re: Black HDPE testing

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

At 15:10 22/10/98 +0200, you wrote:

>I know, copper sheet is $10/meter here. The reason is that here you can buy

>aluminum only 0.3 mm thick or thicker (too tick) and in small sheets, or it

>is that tiny foil for food (very easy to break). 0.1 mm thick copper is an optimal

>compromise, besides for your pocket.

Marco,

You must have some printers in Finland. Go ask them for some old litho

plates. they are made of pure aluminium and come in useful thicknesses.

They can be used to make a flat cap. I have loads of them and intend to

make a cap with them sometime.

hope this helps

bob golding

yubba@clara.net

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Date: Thu, 21 May 1998 15:30:12 EDT

Subject: Re: An odd tesla coil at MOSI

X-Mailer: IPAD 2.0

From: alfred.skrocki.sr@juno.com (Alfred A Skrocki)

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

On Thu, 22 Oct 1998 11:53:54 -0700 tomstock

<tomstock@geocities.com> wrote:

>I looked at MOSI's (Museum of Science and Industry) tesla coil here in

>Tampa FL. Now that I actually know something about Tesla coil design,

>I've realized that this "3 million volt" tesla coil is actually a big

>piece of doodoo. Here's what I saw:

>

>Primary, 4 turns of 1/2" copper tubing, approximately 24" ID x 26" OD

>Secondary

>         500 turns of 12 AWG rubber insulated wire

>         16" diameter x 6 feet long STEEL PIPE form.

I don't think so! If the secondary for were metalic it would discharge to

itself!

>Discharge terminal:

>        Metal spike, 24" long, 1" in diameter.

That's a stupid design! If that very same coil had a properly sized

toroid it would probably yield 6 foot sparks!

Alfred A. Skrocki

Alfred.Skrocki.Sr@JUNO.COM

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: "Coiler" <mycroft@access1.net>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: RE: Some tests with a 2.25" coil (Followup, Solved?)

Date: Thu, 22 Oct 1998 20:14:44 -0700

X-Mailer: IPAD 2.0

Importance: Normal

X-MimeOLE: Produced By Microsoft MimeOLE V4.72.3155.0

Sender: tesl-err@emachine.com

Below, you'll find the original message. I did some further testing tonight.

The setup was exactly the same as described below, with one exception.

I set up a 4.25 Box fan about 1 foot away from the safety gap of my

capacitor. This resulted in the most gentle of breezes blowing across it.

Fired up the coil, and ran for almost 5 minutes. During this time

the cap gap fired twice.

Checking after the run:

1) The main gap was too hot to touch [I had to get the fan to run the test

from somewhere]

2) Primary coil was warm to the touch

3) Likewise the tank wiring.

So I think I finally figured out what was going on. The new enclosure for my

cap has about 5" of headroom. But 2.5" of that is actually a trap for ionized gas.

Apparently the HV RF present on the gap was producing enough ions to eventually

cause breakdown.

I'll do a little more testing just to make sure, but I thought I'd fill you all

in. Thanks for the help along the way.

[ For those that wonder, the gap is formed of 1/4-20 bolts, with acorn

at the business end.]

>

> > As some of you know, I have been fighting a few battles with my system -

> > one of which is protecting my door opener :)

> >

> > The other one has been working with my *expletive* capactitor safety gap.

> > Since I could not run as high power, I grabbed one of my old coils

> > to stick in the primary as a load.. it happened to be my 2.25 x 12"

> > coil, wound with 28AWG wire. I threw my old 3"x10" toroid on there

> > mostly to get to where I could at least use a few turns of my primary.

> > The little coil looked lost in there :) [ 8.5" inner turn for my 6.5"

> > coil]

> >

> > Specs for test:

> > 2.25x12 Coil, with 3x10 Topload.

> > Main Gap 0.175

> > Cap Gap 0.470

> > Power source: 14.4 Pig, with input voltage limited to 100V (12KV output)

> >                1100VA limiting in place.

> >

> > Doing short runs (under 5 seconds) things were fine. Produced anywhere from

> > 2-6 14" streamers. After I got things tuned up I let the system run for a bit

> > (about 20 seconds) when my cap safety gap started to go nuts. I checked my main

> > gap, and the pipes were *HOT* (my kids had broken my fan plenum - so the air

> > was not ducted to my RQ gap.) So I got curious. I let it cool down, and fired

> > up the coil.. things ran fine for about 30 seconds... then *bang* the

> > safety gap fires.. Again, the gap is hot.

> >

> > I am speculating that the poor quenching caused by the overheated gap is playing

> > a role in the problems I am seeing with my safety gap, can any of you

> > give some additional information in support of or against this idea?

> > The only other possiblity (which I have not been able to test yet, wife made

> > me shut down because of the noise) is that the new enclosure I built for my

> > caps is harboring ionized air from the cap gap.. and that this eventually

> > leads to the breakdown.

> >

> > Michael Baumann

> > Coiler, Homebrewer, Nerd. mycroft@access1.net

> >

Date: Thu, 22 Oct 1998 22:02:12 -0500

From: Bert Hickman <bert.hickman@aquila.com>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Organization: Stoneridge Engineering

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Black HDPE testing

Sender: tesl-err@emachine.com

Marco Denicolai wrote:

<SNIP>

>

> Sigh! But I have got also paper layers... Wouldn't that be enough together

> with one layer of HDPE?

Not really. The function of the paper is primarily to aid in oil

wicking. Otherwise, the comparatively thin paper layer doesn't see much

of the total voltage - most of the voltage is developed across the

thicker (and lower k) HDPE. In the case of your proposed cap, the HDPE

will see around 90% of the applied voltage, while the oil-soaked kraft

paper will see the remaining 10%.

>

> Or what about adding also one layer of 0.2 mm (7.9 mil) LDPE instead of a

> second layer 0.8 mm (31 mil) of HDPE?

This is heading in the right direction, and would definately be better

than the single thicker HDPE layer. Using 2 layers of 15 mil matrial

would balance the voltage stress better. Four layers of the 7.9 mil

material would also work quite well. Using multiple thin layers in each

cap and multiple caps in series will result in a bullet-proof Tesla Coil

cap.

>

> >

> >Copper electrodes will work the same as aluminum, but are considerably

> >more expensive and offer no performance advantage. The only advantage

> >with copper is that you can more easily solder to it than aluminum.

> >

> >-- Bert --

> >

>

> I know, copper sheet is $10/meter here. The reason is that here you can buy

> aluminum only 0.3 mm thick or thicker (too tick) and in small sheets, or it

> is that tiny foil for food (very easy to break). 0.1 mm thick copper is an optimal

> compromise, besides for your pocket.

>

Heavy Duty cooking (or catering) foil does work quite well for flat

plate caps, but I see your point for a rolled cap. It's sometimes kind

of tough to find just the right materials isn't it! :^)

<SNIP>

Safe cappin' to you!

-- Bert --

X-Mailer: IPAD 2.0

Date: Fri, 23 Oct 1998 08:13:57 +0200

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Marco Denicolai <marco@vistacom.fi>

Subject: Re: Black HDPE testing

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

At 01:50 23/10/98 +0100, you wrote:

>Marco,

>	You must have some printers in Finland. Go ask them for some old litho

>plates. they are made of pure aluminium and come in useful thicknesses.

>They can be used to make a flat cap. i have loads of them and intend to

>make a cap with them sometime.

>

> hope this helps

>

>bob golding

>yubba@clara.net

>

Yes, that is a good source for flat caps, but I am building rolled caps that

require electrode lenght of 2 meters.

Marco Denicolai                   Vista Communication Instruments, Inc.

Hardware Development Manager      www.vistacom.fi

marco@vistacom.fi

X-Mailer: IPAD 2.0

Date: Fri, 23 Oct 1998 08:29:08 +0200

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Marco Denicolai <marco@vistacom.fi>

Subject: Black HDPE testing continued...

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Yesterday I "cooked" some large strips of my black HDPE in my microwave

furnace @ 500 W for about 5 minutes. The stuff was after that perfectly

cold, as before it.

Anybody suggesting more tests?

Marco Denicolai                   Vista Communication Instruments, Inc.

Hardware Development Manager      www.vistacom.fi

marco@vistacom.fi

X-Mailer: IPAD 2.0

Date: Fri, 23 Oct 1998 08:42:51 +0200

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Marco Denicolai <marco@vistacom.fi>

Subject: Polystyrene best WWW pages

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

A good page about polystyrene data is:

http://www.huntsman.com/products/polymers/ppo1o000.htm

About polystyrene "chemical exposure performance" click on the relative

link on the same page (http://www.huntsman.com/products/polymers/ppo1o100.htm):

you'll get a wide choice of materials organized in alphabetical order and how

PS reacts with them.

There you can learn that PS stands well Castor Oil and Mineral Oil,

so-so Motor Oil and not well Essential Oil. So the oil MAKES the

difference.

Marco Denicolai                   Vista Communication Instruments, Inc.

Hardware Development Manager      www.vistacom.fi

marco@vistacom.fi

From: Hollmike@aol.com

Date: Fri, 23 Oct 1998 07:34:34 EDT

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: An odd tesla coil at MOSI

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

In a message dated 10/22/98 7:11:06 PM Mountain Daylight Time,

alfred.skrocki.sr@juno.com writes:

> That's a stupid design! If that very same coil had a properly sized

>  toroid it would probably yield 6 foot sparks!

>                                 Alfred A. Skrocki

Thanks for the info.  If not for the metal coilform, it might do okay.    The

coil(Big Bertha) at the Tesla Museum in Colorado Springs is not much better

designed.  It was not possible to study it in detail when Bill, the

arcstarter, and I met there last July.  It was large diameter, perhaps 16

inch.  It was wound with cotton insulated wire.  I presume some type of

varnish was used, but couldn't tell for sure.  It was probably not more that

500 turns. It was terminated with a sphere of about six inch diameter that had

two horizontal, pointed rods on opposite sides.   It did about five foot

sparks to the metal ceiling and the sparks were yellowish white rather than

purple.  Unfortunately, I didn't get any other details.  Maybe next time.

Mike Hollingsworth

Sender: tesl-err@emachine.com

Date: Fri, 23 Oct 1998 13:23:57 +0000

From: Tom Stock <tomstock@geocities.com>

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: An odd tesla coil at MOSI

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Alfred A Skrocki wrote:

>

> On Thu, 22 Oct 1998 11:53:54 -0700 tomstock

> <tomstock@geocities.com> wrote:

>

> I don't think so! If the secondary for were metallic it would discharge to

> itself!

>

I'm not kidding about the secondary form! I could see where the

secondary end was welded to the pipe...I'm still wondering why this coil

works!

-tom

X-Mailer: IPAD 2.0

Date: Fri, 23 Oct 1998 12:24:59 -0500

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: Black HDPE testing continued...

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

At 08:29 AM 10/23/98 +0200, you wrote:

>Yesterday I "cooked" some large strips of my black HDPE in my microwave

>furnace @ 500 W for about 5 minutes. The stuff was after that perfectly

>cold, as before it.

>

>Anybody suggesting more tests?

>

Marco,

The RF dissipation factor of your black HDPE is not

what is at issue here. Its the breakdown voltage when

subjected to high RF voltages.

Im afraid there is only one sure test. Build a cap with

it and see for yourself. Like I said earlier this stuff

may be perfectly alright. Only one way to know for sure.

Before you go to the effort to build your caps why not

make a couple small test caps. Test them to the point

of destruction. This will let you know exactly how much

voltage it will stand and point out likely failure modes.

Let us know how things work out.

TTYL!

Mike Hammer

mhammer@misslink.net

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Date: Fri, 22 May 1998 14:10:08 EDT

Subject: Re: An odd tesla coil at MOSI

X-Mailer: IPAD 2.0

From: alfred.skrocki.sr@juno.com (Alfred A Skrocki)

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

On Fri, 23 Oct 1998 13:23:57 +0000 Tom Stock

<tomstock@geocities.com> wrote:

>I'm not kidding about the secondary form! I could see where the

>secondary end was welded to the pipe...

I've seen many plastic tubes "welded" to plastic plates that if they were

silver in color would make one swear they were metal. I'm sure that is

what you were looking at.

>I'm still wondering why this coil works!

If indeed that coil is wound on a metal form I too would like to know how

it can possibly work!

Alfred A. Skrocki

Alfred.Skrocki.Sr@JUNO.COM

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Date: Fri, 23 Oct 1998 15:15:27 -0700

Subject: LDPE, HDPE,rolls and bags...

X-Mailer: IPAD 2.0

From: johnquepublic@juno.com (John Q  Public)

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Hello all,

While cruising alt.laser I came across a thread concerning cap design and

construction and after seeing several posts about the problems with

soldering to aluminium or using argon or electroplating the foil edges

with copper I decided to forward a list of links that I lifted from Mike

Hammer's web page so they wouldn't try to reinvent the wheel. One other

poster also directed them to search the Tesla groups for info. His post

is found below with a link to Associated Bag Co. They appear to have a

huge variety of products and the prices seem good. They also have a very

good index of products and offer FREE catalogues that you can order from

their site and FREE SAMPLES to determine their products suitability for

your job. It struck me that they were a pretty cool company and went

above and beyond to service their "potential" customers (sorry for the

bad pun) Just thought I should pass the info along :-)

JQP

Member of C. A. U. C. E.

Coalition Against Unsolicited Commercial  Email

Fight Spam     http://www.cauce.org

***********************************************************

Sam wrote...

Why not use LDPE? Low density polyethylene, commonly used in construction

etc. I've used it building several HV caps for Tesla coils and a Jacobs

ladder. Cheap, doesn't break like glass, better dielectric value, higher

breakdown voltage, etc.

Associated Bag Co. lists 3'x100'x3mil roll for $7.45US. A lot of plastic.

http://www.associatedbag.com

However, it is better to make plate caps out of bags, as they are already

cut to size, just order them a bit bigger than you plan to make the

plates, and 1/2 the thickness you need the dielectric to be. Then stick the

plate in the bag with a bit sticking out the top for connection purposes, then

stack the bag/plates with the mouth pointing one way, then the other. Any good

Tesla coil page will give you much more detailed directions, just search

around for one.

--Sam

Skywise wrote in message <362FAAF0.B8CD03EA@spamless.cwixmail.com>...

>BINGO!!! I think that was the problem seeing as his new picture was even

>worse. I thought that may be what was happening but when he mentioned

>something about a 'new capacitor design' I thought that what I saw drawn may

>in fact be what he meant.

>

>In any case, I tried a similar scenario myself, although not for a N2 laser.

>It will work but the problem is one of size. Either I needed very large glass

>plates to accommodate larger foil sizes or I needed a lot of glass plates to

>make a large stack. The problem is the thickness of the glass. I find it

>difficult to find large sheets of glass that are very thin. That's why the PCB

>capacitor works so well. You can get a very good thin dielectric.

>

>Now, if only I can find PCB's larger than 24" x 24". Can anyone suggest some

>sources? I've already tried the usual on-line electronics suppliers like

>Mouser, Jameco, Allied, All, JDR, Needham, Newark, HSC.....etc....

<snip>

Date: Fri, 23 Oct 1998 15:18:37 -0700

From: Gene Waldo <gwaldo@1stnetusa.com>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

CC: TESLA-2@emachine.com

Subject: Re:

Sender: tesl-err@emachine.com

Jack m Wu wrote:

>

> Hello.  My name is Jack Wu.  I live in Worhtington, Ohio and I'm in ninth

> grade.  I'm doing a science fair project on a tesla coil.  I'm wondering

> if you can help me.

Jack,

>From your list of addressees, Mike Hammer is the one I 'know' can be of

the most help to you. We are all eager to pitch in!

1) I suggest that you send the PLAN specs to Mike for his suggestions;

since he is a recipient, I'll bet he is hot on your case, as I write.

2) Dan & my coil started as a quick li'l 'project', also, but beware of

the devil in the details!

3) My experince tells me that JUNO 'may' limit your ability to receive

graphics on E-m. ??  Can you access another server for this? Just a

thought...

4) Mike can set you towards our TESLA Archives, where there is much good info!

My 1st coil actually produced sparks, but there was next to NO

engineering in it! Just a NEON Xfmr, a primary and a 2ndary. PERIOD!

Then the bells and whistles start. Stay close to Mike and keep us posted!

5) If you are in school setting, you should enlist as much help as you

can, shops and others of like interests;  A decent coil will involve

many elements, which must be planned and engineered as a part of the

grand design whole.

6) "Wulabs"?  Do I sense a 'leg-up' advantage working for you here? Good luck!

------------------------------------------

Gene Waldo -  gwaldo@1stnetusa.com  - "Igor in Residence"

---------------------------------------------------------------

What's good for the planet is more OZONE! See the machinery form up!

NEW PAGE!   http://members.1stnetusa.com/danmurphy/waldo.htm

From: DavidH8083@aol.com

Date: Sat, 24 Oct 1998 10:59:25 EDT

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: LDPE, HDPE,rolls and bags...

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

In a message dated 10/23/98 6:25:07 PM, johnquepublic@juno.com writes:

<<Why not use LDPE? Low density polyethylene, commonly used in construction

etc. I've used it building several HV caps for Tesla coils and a Jacobs

ladder. Cheap, doesn't break like glass, better dielectric value, higher

breakdown voltage, etc.

Associated Bag Co. lists 3'x100'x3mil roll for $7.45US. A lot of plastic.

http://www.associatedbag.com>>

I agree with the use of LDPE, but 3 mils?   30 mils or better would be more

useful for a Tesla cap.

David Hutchison

davidh8083@aol.com

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Date: Sat, 23 May 1998 10:15:08 EDT

Subject: Re: LDPE, HDPE,rolls and bags...

X-Mailer: IPAD 2.0

From: alfred.skrocki.sr@juno.com (Alfred A Skrocki)

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

On Fri, 23 Oct 1998 15:15:27 -0700 John Q Public

<johnquepublic@juno.com> wrote:

>Hello all,

>While cruising alt.laser I came across a thread concerning cap design

>and construction and after seeing several posts about the problems

>with soldering to aluminium or using argon or electroplating the foil

>edges with copper...

I don't know how others handle the problem but I bolt the aluminum foil

plates together and to the copper conductor AND I use liberal amounts of

conductive grease. I've never had a problem with that combination.

Alfred A. Skrocki

Alfred.Skrocki.Sr@JUNO.COM

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Date: Sat, 23 May 1998 11:36:02 EDT

Subject: Re: LDPE, HDPE,rolls and bags...

X-Mailer: IPAD 2.0

From: alfred.skrocki.sr@juno.com (Alfred A Skrocki)

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

On Sat, 24 Oct 1998 10:59:25 EDT David Hutchison

<DavidH8083@aol.com> wrote:

>I agree with the use of LDPE, but 3 mils?   30 mils or better would be

>more useful for a Tesla cap.

David, what we usually do is use like 10 layers of the 3 mil polyethylene

thus resulting in the 30 mils or so. Personaly I prefer to use the 6 mil

polyethylene you can get at home building supply houses like Home Depot,

5 layers are a lot easier to work with than ten!

Alfred A. Skrocki

Alfred.Skrocki.Sr@JUNO.COM

X-Mailer: IPAD 2.0

Date: Sat, 24 Oct 1998 11:54:47 -0500

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: LDPE, HDPE,rolls and bags...

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

At 10:59 AM 10/24/98 EDT, you wrote:

>

>In a message dated 10/23/98 6:25:07 PM, johnquepublic@juno.com writes:

>

><<Why not use LDPE? Low density polyethylene, commonly used in construction

>etc. I've used it building several HV caps for Tesla coils and a Jacobs

>ladder. Cheap, doesn't break like glass, better dielectric value, higher

>breakdown voltage, etc.

>

>Associated Bag Co. lists 3'x100'x3mil roll for $7.45US. A lot of plastic.

>http://www.associatedbag.com>>

>

>I agree with the use of LDPE, but 3 mils?   30 mils or better would be more

>useful for a Tesla cap.

>

>David Hutchison

>davidh8083@aol.com

David and others. Actually a stack of thinner material is the preferred

method. 5 layers of 6 mil poly would have a much greater voltage rating

than a single 30 mil thickness.

Also the 30 mil material is a special order item while poly from 8 mils

on down is readily available at hardware stores and home improvement

centers.

Also be very careful with poly meant to be used for plastic bags.

If the bag was to have printing applied to it such as in a bread wrapper

then the poly is run through a high voltage device that actually punctures

the material with millions of tiny microscopic holes. This is done so the

printing will adhere to the poly. That would make it pretty useless as

a dielectric.

TTYL!

Mike Hammer

mhammer@misslink.net

From: Dsurfr@aol.com

Date: Sat, 24 Oct 1998 14:11:24 EDT

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Cap Clearances

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Hi All: When placing three vertical stacked caps in the same tank (in series)

how much space should be left between the caps - if any? Also are there any

minimum clearances between the caps & the tank?         Thanks, Jim

Date: Sat, 24 Oct 1998 13:43:02 -0500

From: Bert Hickman <bert.hickman@aquila.com>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Organization: Stoneridge Engineering

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Cap Clearances

Sender: tesl-err@emachine.com

Dsurfr@aol.com wrote:

>

> Hi All: When placing three vertical stacked caps in the same tank (in series)

> how much space should be left between the caps - if any? Also are there any

> minimum clearances between the caps & the tank?         Thanks, Jim

Jim,

It depends upon whether the outermost capacitor plates will be at the

same electrical potential when you connect the individual caps in

series. If so (Case A), then you really don't need any clearance. If

they are not at the same potential (Case B), but only separated by 1 cap

(Case B), you'll need to separate them with at least 2+ times the

dielectric material thickness thickness used inside each cap, since the

voltage will be twice that seen in across a single cap. This can be seen

in the sketches below.

Case A

C1          C2           C3

||          ||           ||

-----||----------||-----------||------

||          ||           ||

Vc          Vc           Vc

Case B

2Vc

|-----------|

||   |      ||   |       ||

-----||---|  |---||---|  |----||------

||      |   ||      |    ||

|-----------|

2Vc

For mechanical strength, I used 1/8" PVC plates on the outer sides of

each cap to make indivual stand-alone capacitor modules. These were then

connected in series

Assuming the tank is non-conductive and that there will be nothing

conductive on outer wall of the tank, the cap can butt up against the

wall. A conductive tank will require at least 1/2" of oil from any

energized conductor.

-- Bert --

X-Originating-IP: [207.100.20.77]

From: "Philip Mac Duffie" <punk_42@hotmail.com>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Magnetron Diode

Date: Sun, 25 Oct 1998 18:30:31 PST

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

I know, this is off topic but perhaps one can stear me in the correct

direction. I am wondering if anyone has made a magnetron work out of the

case without cooking themselves? Are microwaves absorbed by all

materials? Will the magnetron work running straight from the mot? Can

microwave radiation be focused by a lens?

Thanks,

Philip Mac Duffie

Date: Sun, 25 Oct 1998 21:27:29 -0600

From: RODERICK MAXWELL <tank@magnolia.net>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: H&R Transformers

Sender: tesl-err@emachine.com

I'm not going to waste another minute of time trying to eke some

sparks out of these transfomers! I have used voltage doublers, removed

all of the shunts, and added more capacitance to the filter to no avail.

Next stop is a PIG! This will be the only way I can charge this brute of

a tank capacitor I built efficiently. I can't wait until next month.

Then I can see if it will fit under the christmas tree!

Frankensteins Helper

Max

Date: Sun, 25 Oct 1998 23:06:55 -0800

From: " Mike N." <705728@ican.net>

Organization: Canadian Pyrotechnics Association

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Magnetron Diode

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Philip Mac Duffie wrote:

>

> I know, this is off topic but perhaps one can stear me in the correct

> direction. I am wondering if anyone has made a magnetron work out of the

> case without cooking themselves?

Yes, but not continuous, these things run hot so a cooling fan might

be a good idea.

> Are microwaves absorbed by all materials?

Microwaves are generally absorbed only by things containig water, a

microwave is designed to excite the water molecule causing heat. Steel

reflects microwaves but can also serve as an antenna.

>Will the magnetron work running straight from the mot?

No, it must use the capacitor/rectifier voltage doubler circuit to

resonate.

>Can microwave radiation be focused by a lens?

no again, microwaves are radio signals and not in the spectrum of light.

I would be very carefull when playing with microwave componants or

experimenting with them, remember the body is 70% water and microwaves

cook from the inside out.   Always short out the cap before handling

--

Where's the KABOOM, there was supposed to be an earth shattering KABOOM.

Sonic Boom

Date: Mon, 26 Oct 1998 16:54:33 +1100

From: Corey Kent <corey@enterprise.net.au>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: [Fwd: Lost For a Power Supply,]

Sender: tesl-err@emachine.com

> hi all,

>

> im left the tesla coil subject for a small time because i couldnt find a power

> supply im in australia and even second hand NST supplies cost AUS $200 here

> thats about $150 U.S. i think.

> what else can i use for a power supply that is cheap and easy to find????

Date: Mon, 26 Oct 1998 17:07:07 +1100

From: Corey Kent <corey@enterprise.net.au>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Australian Tesla Coiler's

Sender: tesl-err@emachine.com

> Australian Tesla Coiler's!

>

> are there any other australian tesla coilers on this list????

X-Mailer: IPAD 2.0

Date: Mon, 26 Oct 1998 09:18:48 +0200

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Marco Denicolai <marco@vistacom.fi>

Subject: Rolled cap margins?

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

To take into account that:

1. It is better to use 2 caps in series, rating each of them e.g. 10 kV DC,

instead of trying to built a single one for 20 kV DC. Thus, it is not worth

using 90 mils of dielectric if you are not going to stress the cap with more

than 10 kV DC (e.g. you can use an external safety gap on the cap).

2. Two dielectric layers are better than only one.

I have (again) modified my tentative capacitor design as follows:

- paper    INNER LAYER

- Cu

- paper

- 0.2 mm LDPE

- paper

- 0.8 mm HDPE

- paper

- Cu

- paper

- 0.2 mm LDPE

- paper

- 0.8 mm HDPE   OUTER LAYER

Paper is 0.05 mm (40g/m2) kraft paper not waxed.

HDPE is 0.8 mm (31 mil) HDPE

LDPE is 0.2 mm (7.9 mil) LDPE

Cu is 0.1 mm copper sheet

capacitor is filled with oil

If I rate this capacitor as a 10 kV DC device, how much wide is the margin

I have to leave on the electrode sheet sides? 1", 1.5" or how much?

Please, help me with this decision, because I wouldn't like to leave too much

space if it is not needed: that will reduce the capacitance very quickly :(

Marco Denicolai                   Vista Communication Instruments, Inc.

Hardware Development Manager      www.vistacom.fi

marco@vistacom.fi

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: "Coiler" <mycroft@access1.net>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Mot Weirdness, and other bizzare things

Date: Sun, 25 Oct 1998 23:28:44 -0800

X-Mailer: IPAD 2.0

Importance: Normal

X-MimeOLE: Produced By Microsoft MimeOLE V4.72.3155.0

Sender: tesl-err@emachine.com

This is to be more of an information dump, than a query. I finally managed

to get around to my storage shed to retreive the MOT from a wounded

microwave I had there.

After the needed dissection, I hooked the MOT up to find out just how the

beastie would work as a limiter for my pig. Alas, I do not have a current

meter capable of 10A+, so I was just checking to see how hot things got.

Unscientific, perhaps, but it works.

I plugged the MOT into my 10A variac, and slowly wound up the voltage. At

about 90V the MOT began a menacing growl. Coils were slightly warm to the touch.

Then I shorted the secondary. The 10A fuse in my variac went *poof*. So

shorting was out, for now. Wired the MOT in series with my pig, open

secondary, and hit the switch. I was not expecting the kind out power arcs

flying out of the system. The MOT seemed to be passing close to 1KVA as

measured by comparing the power arcs with those produced by using shorted neons as

ballast. Windings were hot after 2 minute run.

I disconnected the coil, and just had my gap across the pig. At this

point, I could barely get a peep out of the gap. So I shorted the secondary

and tried. This fired up the gap ok.. but popped my 15A circut breaker.

So, I tried it feeding the tank again, open secondaries, just in case I had

something screwy hooked up. Same results. Tons of current passing. I'll

borrow a clamp on current meter from work, and try and get you better figures.

The other thing comes under the heading of "fun but weird", I had gone back

to one of my smaller ballasts (15KV30ma NST, shorted secondary) and was

mapping fields with a 4' flourescent. I was doing this in the dark ( I feel safe as

I have my coil wired with a deadman, I did not have to find the power

switch, and my pig is located back in a protected corner, I use stripped 150KV Xray

cable to hook it up.) Anyway, I turned the coil off, and was moving back to

the garage light switch when the fl. tube flashed in my hand. Moving it around

I managed to get a good 15-20 flashes.. sometimes 2-4 in the same spot, the

rest scattered in a radius around where I was standing. I figure I was either

collecting and discharing ions in the air, or removing the charge form items that had

picked up one from the coil, or some combo of the above. But was kinda fun,

watching the tube flash every few seconds...

enough rambling for now.

Michael Baumann

Coiler, Homebrewer, Nerd. mycroft@access1.net

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Date: Mon, 26 Oct 1998 04:35:26 -0700

From: "paul m" <fleischwulf@mailexcite.com>

Cc:

X-Sent-Mail: off

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

Subject: Re: Mot Weirdness, and other bizzare things

X-Sender-Ip: 208.153.7.9

Organization: MailExcite  (http://www.mailexcite.com:80)

Sender: tesl-err@emachine.com

Mike,

You need at least 2 MOT's. With two I get about 8-9 Kw. Three will give you about 5 Kw. Some fantastic arcs. See my web page to view my set up.

http://www.geocities.com/capecanaveral/9561

paul m.

---

Einigheit und Wahrheit

Date: Mon, 26 Oct 1998 07:17:36 -0600

From: Bert Hickman <bert.hickman@aquila.com>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Organization: Stoneridge Engineering

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Rolled cap margins?

Sender: tesl-err@emachine.com

Marco,

This depends on the type of construction you use (extended foil or

end-terminated). You may also want to reduce the width of the paper

versus the polyethylene so that the paper does not extend very far

beyond the edge of the adjacent copper plate - this is because

polyethylene resists breakdown from the high E-fields seen at the edges

of the plates better than paper. See the diagrams below for a better

description.

The worst case stress is where the opposite plate extends beyond the

edge of the plastic for termination, so that you only have a creep

distance of D1 (below). This should be no less than 1.5" (D1) for an

extended foil cap, and no less than 1" (D2 below) for a rolled cap using

only end terminations. Since the layers may tend to shift a bit during

fabrication, this will provide a bit of safety margin. In all cases,

wherever the terminations are brought out, there needs to be 1.5"

surface clearance to the opposite plate's edge.

Extended foil:

|<-- D1--->|--------------------------------------- plate

--------------------------------- paper

------------------------------------------- PE

------------------------------------------- paper

------------------------------------------- PE

------------------------------------ paper

----------------------------------------- plate

End-terminated:

|< D2 >|--------------------------- plate

------------------------------- paper

------------------------------------------- PE

------------------------------------------- paper

------------------------------------------- PE

------------------------------- paper

D2  --------------------------- plate

Hope this helps!

-- Bert --

Marco Denicolai wrote:

>

> To take into account that:

>

> 1. It is better to use 2 caps in series, rating each of them e.g. 10 kV DC,

> instead of trying to built a single one for 20 kV DC. Thus, it is not worth

> using 90 mils of dielectric if you are not going to stress the cap with more

> than 10 kV DC (e.g. you can use an external safety gap on the cap).

>

> 2. Two dielectric layers are better than only one.

>

> I have (again) modified my tentative capacitor design as follows:

>

> - paper    INNER LAYER

> - Cu

> - paper

> - 0.2 mm LDPE

> - paper

> - 0.8 mm HDPE

> - paper

> - Cu

> - paper

> - 0.2 mm LDPE

> - paper

> - 0.8 mm HDPE   OUTER LAYER

>

> Paper is 0.05 mm (40g/m2) kraft paper not waxed.

> HDPE is 0.8 mm (31 mil) HDPE

> LDPE is 0.2 mm (7.9 mil) LDPE

> Cu is 0.1 mm copper sheet

> capacitor is filled with oil

>

> If I rate this capacitor as a 10 kV DC device, how much wide is the margin

> I have to leave on the electrode sheet sides? 1", 1.5" or how much?

>

> Please, help me with this decision, because I wouldn't like to leave too much

> space if it is not needed: that will reduce the capacitance very quickly :(

>

>

>  Marco Denicolai                   Vista Communication Instruments, Inc.

>  Hardware Development Manager      www.vistacom.fi

>

>  marco@vistacom.fi

From: wwl@netcomuk.co.uk (Mike Harrison)

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Magnetron Diode

Date: Mon, 26 Oct 1998 13:50:11 GMT

Organization: White Wing Logic

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

On Sun, 25 Oct 1998 23:06:55 -0800, you wrote:

>Philip Mac Duffie wrote:

>Can

>> microwave radiation be focused by a lens?

>

>no again, microwaves are radio signals and not in the spectrum of light.

Not strictly true, although the lens material would be rather

different to glass.

A parabolic reflector (sat. dish) could be used to focus microwaves,

but the effectiveness would  depend on what the beam angle from the

magnetron was.

Date: Mon, 26 Oct 1998 11:35:15 -0800

From: " Mike N." <705728@ican.net>

Organization: Canadian Pyrotechnics Association

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Magnetron Diode

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Mike Harrison wrote:

>

> On Sun, 25 Oct 1998 23:06:55 -0800, you wrote:

>

> >Philip Mac Duffie wrote:

>

> >Can

> >> microwave radiation be focused by a lens?

> >

> >no again, micrwaves are radio signals and not in the spectrum of light.

> Not strictly true, although the lens material would be rather

> different to glass.

> A  parabolic reflector (sat. dish) could be used to focus microwaves,

> but the effectiveness would  depend on what the beam angle from the

> magnetron was.

True but I think that would classify as an antena.

--

Where's the KABOOM, there was supposed to be an earth shattering KABOOM.

Sonic Boom

X-Originating-IP: [195.121.16.38]

From: "Sam Barros" <sambarros@hotmail.com>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Magnetron Diode

Date: Mon, 26 Oct 1998 10:41:47 PST

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

>From: "Philip Mac Duffie" <punk_42@hotmail.com>

>To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

>Subject: Magnetron Diode

>Date: Sun, 25 Oct 1998 18:30:31 PST

>Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

>

>I know, this is off topic but perhaps one can stear me in the correct

>direction. I am wondering if anyone has made a magnetron work out of the

>case without cooking themselves? Are microwaves absorbed by all

>materials? Will the magnetron work running straight from the mot? Can

>microwave radiation be focused by a lens?

>    Thanks,

>Philip Mac Duffie

Philip,

This is a Tesla Coil List, so I shouldn't be posting it here, but

still, microwaves posts have come up so many times that there is an

obvious interest in such devices among list members.

I have personally fully dismantled and reassembled a 1000 watts

microwave oven into a stainless steel box. The box measures 19X 30X 11cm

and has 1mm thick walls. It cost me 250Dollars to get it custom built.

The oven itself is remarkably simple and someone with reasonable

electrical experience/skills will be able to take it apart, remove all

it's non-essential components (lamp, rotating dish, safety interlocks,

etc...) and reassembling it in a metal box.

Now, the oven works with 6000V at 0.5 A. It uses a 1.1uF 3000V

capacitor on a voltage Doubler circuit, so a shock from it WILL probably

kill you. Plus, the entire assembly HAS to be inside a metal box, as the

ground of the magnetron and transformer is on the box itself.

Supposing you don't kill yourself by electrocution and actually make it

right, you'll have yourself a working 1KW microwave "gun".

Now, magnetrons will radiate microwave energy in ALL directions

(except, of course backwards), so the energy exiting that little hole in

front will be quite small due to the fact it is scattered over a large

area. Get microwaves to bounce off a metal and go back into the hole and

you WILL kill your magnetron (my first one). Get microwaves to bounce

off a metal and have that metal attached to your box and you WILL ALSO

kill your magnetron (that was my second). Get them to come back at you

and you'll cook. Happened to me and wasnt very pleasant. Gives you a

KILLER headache!!! Plus, microwaves cook everything down to one inch, so

while your skin fells warm, your internal organs are being destroyed.

Since power is being radiated in all directions, the range will be

pathetically short. 1 meter or so and it won't even light a fluorescent tube.

Bu using a parabolic antenna you can focus them, but as the waves are

going in all directions, getting them all to hit the parabola will be

tricky. Since the magnetron is so massive, placing it in front of the

parabola will ensure that all microwaves come back at it. With a double

parabola system, you ca bet them to focus. It isn't very efficient, and

the heat produced means that even with a good cooling system it won't

work for a long time.

I did it, and I don't advise anyone else to repeat the experiment. It

works, but the dangers and difficulties really aren't worth it, unless

you get a kick out of vaporising organic stuff at a distance.

Personally, I think the 500+ dollars I've spent getting this to work

aren't worth it if you want it for fun. I built mine for scientific

experimentation, so I guess that's all right.

Mike is going to kill me for posting this...

Could you please keep such topics off list? My e-mail is

sambarros@yahoo.com, and if you need any more info, or want to have a

look at the pictures I took of the device (if I ever get them

developed), mail me!

Oh, About your questions: Microwaves bounce off metals. The metals

develop and electrical potential and they receive the microwaves.

Microwaves are absorbed by organic and water containing substances, and

also, to a lesser extent, by some plastics. But not as much You can run

a magnetron off a MOT and a voltage Doubler But the voltage has to be

constant, and right for the magnetron you are using. You cant just

crank up or down the voltage and expect the magnetron to change the

power output. In a oven, power is regulated by switching the magnetron

on and off very quickly

Sam Barros.

Date: Mon, 26 Oct 1998 21:36:25 -0700

From: Dan Olsen <dan@olsen.org>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Magnetron Diode

Sender: tesl-err@emachine.com

This is way off topic but possibly interesting

> Can microwave radiation be focused by a lens?

Yes, not a glass lens though.  In college physics we made a large 2.5 foot

lens out of paraffin.  Paraffin is an excellent lens material for

microwaves(at least for the 3.5cm waves we were using.) With the right shape

you can focus and direct them nicely.  A metal mirror of the right shape

would also reflect and focus them.

We did experiments with a 3 Watt microwave source so we weren't too

concerned about them bouncing around the room and heating us up.  You might

want to excercise more caution if you are doing similar experiments with a

1000 Watt Micowave Oven Source.

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Cap Builders Info

X-Mailer: IPAD 2.0

From: break7@juno.com (Kyle A. Break)

Date: Tue, 27 Oct 1998 23:40:51 EST

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

I've been quiet lately. But I'm still coiling. I'm rebuilding my

flat stack cap for fifth time.:(  I've been using aluminum flashing from

the Home Depot. It's great stuff. About .007 thick and is plenty durable.

I've thrown them in the sink and washed them four times. This is one of

the few good things about flat stack caps, couldn't very well put an 8'

long sheet in the sink!  I've been blowing the caps because I've only

been using one at a time. It's a lot more work than rolled. The poly I've

been using is 4'x8' sheets cut into 8"x10" pieces. The plastic company

tells me that the sheets are extruded and are the highest quality. I just

found out that it comes in rolls! 12" and 8" wide x500 feet! I bought a

paper cutter to cut the plates and plan to use it to cut the sheets of

poly off the roll. Using the table saw to cut the poly was quite a

hassle.

The single caps were stacks of .062 poly -40 sheets  The failure is

almost always a flash around the outside of the stack- from sheet to

sheet. The caps are always submerged in trans. oil so the voltage must be

getting quite high.

When using a single cap (single layers of .062 poly) with 15kv60ma I

haven't had any failures,but when I hook up the piggy it's just a matter

of minutes (~30 running) till it dies. Long run times seem to be the most

likely culprit. As most people have probably noticed the system seems to

build to crescendo after ~10 seconds. With 3600W and a single cap a blow

- out is seconds away!  So on to better things,

The new set up.

Sheets     132   3 stacks of 44 each in series  6" x13.5"

Poly         HDPE .030+LDPE .006 per layer ( two different sheets per layer)

Container    7 1/2 gal polyethylene drum

I'm going to stand them vertically with all connections at the top.

I have a piece of HDPE grate cut from a milk crate for the bottom spacer.

I'm looking for some piggy ears for the top of the lid. Don't know what

I'm going to use for spacers between the stacks, maybe poly cutting board.

I strongly recommend the paper cutter. I was able to rough out 133 sheets

in about 15 minutes.

If anybody is interested, the aluminum flashing cost $56 The roll of

poly 8" x .030 x 500' costs $150 and is coming from Pennsylvania. That's

all for now. Questions or comments?

If it's not dangerous it's not fun!  Kyle  break7@juno.com

Date: Wed, 28 Oct 1998 09:58:09 -0800

From: Gene Waldo <gwaldo@1stnetusa.com>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Cap Builders Info

Sender: tesl-err@emachine.com

Kyle A. Break wrote:

>

>         I've been quiet lately. But I'm still coiling. I'm rebuilding my

> flat stack cap for fifth time.:(  I've been using aluminum flashing from

> the Home Depot. It's great stuff. About .007 thick and is plenty durable.

> I've thrown them in the sink and washed them four times. This is one of

> the few good things about flat stack caps, couldn't very well put an 8'

> long sheet in the sink!  I've been blowing the caps because I've only

> been using one at a time. It's a lot more work than rolled. The poly I've

> been using is 4'x8' sheets cut into 8"x10" pieces. The plastic company

> tells me that the sheets are extruded and are the highest quality. I just

> found out that it comes in rolls! 12" and 8" wide x500 feet! I bought a

> paper cutter to cut the plates and plan to use it to cut the sheets of

> poly off the roll. Using the table saw to cut the poly was quite a

> hassle.

>

>    The single caps were stacks of .062 poly -40 sheets  The failure is

> almost always a flash around the outside of the stack- from sheet to

> sheet. The caps are always submerged in trans. oil so the voltage must be

> getting quite high.

>

>    When using a single cap (single layers of .062 poly) with 15kv60ma I

> haven't had any failures,but when I hook up the piggy it's just a matter

> of minutes (~30 running) till it dies. Long run times seem to be the most

> likely culprit. As most people have probably noticed the system seems to

> build to crescendo after ~10 seconds. With 3600W and a single cap a blow

> - out is seconds away!  So on to better things,

>

>    The new set up.

>       Sheets     132   3 stacks of 44 each in series  6" x13.5"

>       Poly         HDPE .030+LDPE .006 per layer ( two different sheets per layer)

>       Container    7 1/2 gal polyethylene drum

>

>       I'm going to stand them vertically with all connections at the top.

> I have a piece of HDPE grate cut from a milk crate for the bottom spacer.

> I'm looking for some piggy ears for the top of the lid. Don't know what

> I'm going to use for spacers between the stacks, maybe poly cutting board.

>   I strongly recommend the paper cutter. I was able to rough out 133

> sheets in about 15 minutes.

>     If anybody is interested, the aluminum flashing cost $56 The roll of

> poly 8" x .030 x 500' costs $150 and is coming from Pennsylvania. That's

> all for now. Questions or comments?

>

Kyle,

My only suggestion would be to RADIUS all Al plate corners and SMOOTH

them severely! Any little pointy corners, edges or rough spots will be

sources of corona-break-out and flash-over!

I don't know how wide spread this info is, but Dan fwd me notice of a

"HALLOWEEN Haunted House', consisiting of among others, a 'faraday'

walkthru about a coil producing 12' strikes! (Northridge, CA) 8900

DeSoto @ Nordhoff.

Going to make every effort to get there today!  $8.5 adm'n. More details

if and as I get them.

---------------------------------------------

Gene Waldo -  gwaldo@1stnetusa.com  - "Igor in Residence"

---------------------------------------------------------------

What's good for the planet is more OZONE! See the machinery form up!

NEW PAGE!   http://members.1stnetusa.com/danmurphy/waldo.htm

From: McFalc0n@aol.com

Date: Wed, 28 Oct 1998 23:03:35 EST

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Primary Question

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

I have 2 50' rolls of 1/4'' copper tubing, I wound the first roll, but i was

wondering if there is another way besides soddering the connection between the

two tubings to join them.  Anyone have ideas or suggestions???

Thanx

Date: Wed, 28 Oct 1998 23:19:44 -0800

From: " Mike N." <705728@ican.net>

Organization: Canadian Pyrotechnics Association

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Primary Question

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

McFalc0n@aol.com wrote:

>

>         I have 2 50' rolls of 1/4'' copper tubing, I wound the first roll, but i was

> wondering if there is another way besides soddering the connection between the

> two tubings to join them.  Anyone have ideas or suggestions???

>                                                                                                         Thanx

The most seamless way would be to swedge one end of tubing to 5/16"

diameter using a swedging tool available from any appliance repair or

refrigeration shop, then you can slide one end into the other and

solder.

--

Where's the KABOOM, there was supposed to be an earth shattering KABOOM.

Sonic Boom

From: Hollmike@aol.com

Date: Thu, 29 Oct 1998 08:14:16 EST

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Primary Question

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

In a message dated 10/28/98 9:20:20 PM Mountain Standard Time, 705728@ican.net

writes:

>

>  McFalc0n@aol.com wrote:

> >

> >         I have 2 50' rolls of 1/4'' copper tubing, I wound the first roll,

> > but i was wondering if there is another way besides soldering the connection

> > between the two tubings to join them.  Anyone have ideas or suggestions???

>  >

>                              Thanx

>

>      The most seamless way would be to swedge one end of tubing to 5/16"

>  diameter using a swedging tool available from any appliance repair or

>  refrigeration shop, then you can slide one end into the other and solder.

>  --

I would just insert a piece of tinned, 1/8 inch OD wire(assuming 1/16 inch

wall thickness in the tubing) inside and solder them together.  This will

probably require a torch.  This would avoid the bulge at the connection.  Any

blobs of solder could be sanded off to smooth the connection.

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Date: Thu, 29 Oct 1998 11:36:24 EST

Subject: Re: Primary Question

X-Mailer: IPAD 2.0

From: alfred.skrocki.sr@juno.com (Alfred A Skrocki)

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

On Wed, 28 Oct 1998 23:03:35 EST

<McFalc0n@aol.com> wrote:

>	I have 2 50' rolls of 1/4'' copper tubing, I wound the first

>roll, but I was wondering if there is another way besides soddering

>the connection between the two tubings to join them. Anyone have

>ideas or suggestions???

Yuor best bet would be to get a 1 to 2 inch length of 1/8 inch O.D.

copper (#8 gauge) and force half of it inside one tube and then force the other

tube on the other remaining end.

Alfred A. Skrocki

Alfred.Skrocki.Sr@JUNO.COM

Visit my Do-It-Yourself Aquarium WEB page at:

http://WWW.GeoCities.com/CapeCanaveral/6251

From: EdwMullin@aol.com

Date: Thu, 29 Oct 1998 18:15:21 EST

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Primary Question

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Good timing on this question, as I have a section of primary wound now with a

seam waiting to be joined. Thanks!:)

Date: Sat, 31 Oct 1998 18:31:19 -0600

From: RODERICK MAXWELL <tank@magnolia.net>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: H&R Transformer Modification

Sender: tesl-err@emachine.com

Ever since I've been trying to use my H&R transformers I have not been

able to get any output from them that amounts to anything until

yesterday. I have finally found a way to unlock their hidden potential

(excuse the pun).

1. Remove all of the shunts.

2. Take of the top of the transformer by removing the two top bolts.

This will allow you to take off the "I" part of the "E"/"I" assembly.

Use a wooden or plastic mallet to prevent marring of the core that is

being held in place by electrical varnish.

3. After you have freed the top half of the core remove 50 turns of the

primary winding.

4. Reassemble the transformer and add a ballast to the primary.

I have two of these transformers in antiparrallel, after the

modifications I have just mentioned my measured voltage climbed to

9.2Kv. And the current has also dramatically increased. The only

drawback I could find after testing was the pair draws about 6 amps

unloaded due to reduced inductive reactance in the primary.

The output from the secondary is a blue flame about 3" long when the

lead are brought close together (and probably quite lethal, so be

CAREFUL!). I used a 60milliamp 12Kv transformer for ballast and removed

primary turns until the flame was as hot as I wanted.

Each transformer is seeing 75 volts at it's primary at no load with

120 volts applied.

This is due to the voltage drop caused by the ballast. If I desire more

"heat" all that I have to do is remove a few more turns from the

ballast.

I came up with this idea after one of the other members told me he

totally rewound one. But I must confess I'm to lazy and impatient for

the job!

Frankensteins Helper

Max