From: tesla-2@emachine.com (Tesla-2)

Subject: Re: flat plate vs rolled caps

Date: Fri,  1 Aug 1997 21:03:30 GMT

Organization: The Energy Machine Information System 504-734-0526

From: Steve Falco <sfalco@worldnet.att.net>

Date: Fri, 01 Aug 1997 17:03:30 -0400

Subject: Re: flat plate vs rolled caps

> For your 12kv neon you will need 48 mils of poly.  The best results

> will come from running two caps in series. For the .0066 mfd

> you want for your 12kv/30 make two caps using 24 mils of poly in each.

> Make the value of each cap .0132. Then the two in series will be

> equivalent to 48 mils with a value of .0066 mfd.

>

> If you try to make it in one cap increase the dielectric thickness

> to 60 mils.

>

> Its easier to make two reliable 6kv caps in series than one

> reliable 12kv cap.

I'd like to jump into this thread with a question or two.  I am now sold

on the idea of putting two caps in series to make them more robust.  But

I am still debating whether to build rolled units or flat plate units.

I want about 0.08 uf, and my combined set of trannies are rated at

10kv/300ma.  I just bought a roll of 0.040" (40 mil) LDPE plastic from a

local industrial supply house (48" wide, 100 lb - sold by weight, not

length!  But it is estimated at 135' long).  Thus, with two caps in

series, I will wind up with a total of 0.080" effective dielectric

thickness, which I think will be ok for the voltage I am working with.

If I go with a rolled cap design, and use 18" wide heavy Aluminum foil

(common grocery store item), then to get 0.16 uf (so two caps in series

give 0.08 uf), I calculated that each plate has to be about 365" or 30'

long.  This strikes me as very long for the 18" width; I am concerned

about the inductance in the plates.  I could build 4 units instead of 2,

put pairs in parallel, then put the two paralleled pairs in series.

Then the plates would "only" be 15' long.

Is this feasable, or is even 15' just too long for foil plates?  (The

heavy foil measures just about 0.001" thick.)  I could also use 14" wide

roof flashing, which is much thicker, but it is more expensive and

harder to work with.  This would help with DC resistance but probably

would not help much with inductance.

If I build flat plates, it would again be easiest to make one dimension

18".  However, since I want any trapped air to vent easily, the 18"

dimension must be vertical, and that means the tank needed will be

perhaps 24" deep (to accomodate the extra width of the dielectric,

etc.)  This will probably have to be custom fabricated, which is a lot

harder than cutting some 6" PVC to house the rolled caps.  I don't know

if the lower inductance of the flat caps would be worth the trouble.

Comments/opinions/advise welcomed!

Steve Falco

sfalco@worldnet.att.net

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Testing, Testing . .

Date: Fri,  1 Aug 1997 23:41:11 GMT

Organization: The Energy Machine Information System 504-734-0526

From: cjhill@hills.u-net.com (CJ Hill)

Date: Sat, 2 Aug 1997 00:41:11 +0100

Subject: Testing, Testing . .

Hi,

This is really just a test message as I've not been able to post to the list

for some time (my ISP was giving me grief).

I'll take the opportunity though to let you all know of my tesla web page:

http://www.hills2.u-net.com/tesla.htm

It's (still) in development and probably always will be, and there are no

pretty pictures of my tesla coil yet(I need to fix my camera first!). My

page describes my ignition coil-based setup - rather unconventional and

probably soon to be replaced by a neon but I hope it provokes some ideas . . .

Thanks, Chris.

-------------------------------------------------

Chris Hill, Cardiff, UK

cjhill@hills.u-net.com

-------------------------------------------------

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: flat plate vs rolled caps

Cc:

Bcc:

X-Attachments:

At 09:03 PM 8/1/97 GMT, you wrote:

>From: Steve Falco <sfalco@worldnet.att.net>

>Date: Fri, 01 Aug 1997 17:03:30 -0400

>Subject: Re: flat plate vs rolled caps

>

>> For your 12kv neon you will need 48 mils of poly.  The best results

>> will come from running two caps in series. For the .0066 mfd

>> you want for your 12kv/30 make two caps using 24 mils of poly in each.

>> Make the value of each cap .0132. Then the two in series will be

>> equivalent to 48 mils with a value of .0066 mfd.

>>

>> If you try to make it in one cap increase the dielectric thickness

>> to 60 mils.

>>

>> Its easier to make two reliable 6kv caps in series than one

>> reliable 12kv cap.

>

>I'd like to jump into this thread with a question or two.  I am now sold

>on the idea of putting two caps in series to make them more robust.  But

>I am still debating whether to build rolled units or flat plate units.

>

>I want about 0.08 uf, and my combined set of trannies are rated at

>10kv/300ma.  I just bought a roll of 0.040" (40 mil) LDPE plastic from a

>local industrial supply house (48" wide, 100 lb - sold by weight, not

>length!  But it is estimated at 135' long).  Thus, with two caps in

>series, I will wind up with a total of 0.080" effective dielectric

>thickness, which I think will be ok for the voltage I am working with.

>

>If I go with a rolled cap design, and use 18" wide heavy Aluminum foil

>(common grocery store item), then to get 0.16 uf (so two caps in series

>give 0.08 uf), I calculated that each plate has to be about 365" or 30'

>long.  This strikes me as very long for the 18" width; I am concerned

>about the inductance in the plates.  I could build 4 units instead of 2,

>put pairs in parallel, then put the two paralleled pairs in series.

>Then the plates would "only" be 15' long.

>

>Is this feasable, or is even 15' just too long for foil plates?  (The

>heavy foil measures just about 0.001" thick.)  I could also use 14" wide

>roof flashing, which is much thicker, but it is more expensive and

>harder to work with.  This would help with DC resistance but probably

>would not help much with inductance.

>

>If I build flat plates, it would again be easiest to make one dimension

>18".  However, since I want any trapped air to vent easily, the 18"

>dimension must be vertical, and that means the tank needed will be

>perhaps 24" deep (to accomodate the extra width of the dielectric,

>etc.)  This will probably have to be custom fabricated, which is a lot

>harder than cutting some 6" PVC to house the rolled caps.  I don't know

>if the lower inductance of the flat caps would be worth the trouble.

>

>Comments/opinions/advise welcomed!

>

>        Steve Falco

>        sfalco@worldnet.att.net

Steve sounds like you are playing with a pair

of those H&R copier transformers. Let us know how they

work out.

Steve either way I dont think you can hope to make .08 mfd

in a single cap. A rolled cap will be way too long and offer

way to much inductance.

You could do it in a plate cap but it would be better to make

smaller units and parallel them to get that level of capacitance.

Unfortunately your 40 mil poly isnt well suited for the task

being too thin for a single layer and too thick for two layers.

Still if the quality is good and you leave a 2 inch margin all around

your plates a 40 mil cap should handle 10-12kv pretty well.

The use of real transformer oil is recommended.

Try this cap design on for size.

Using 9 inch wide aluminum foil (cut a 18 in wide roll in half)

cut the plates 100 inches long. Thats 900 sq inches of plate area.

With 40 mils of poly in a rolled cap that comes to .02 mfd.

The poly should be about 110 inches long and 12-13 inches wide.

Rolled up this should fit into 3 inch PVC pipe.

4 such units in parallel will give you .08 mfd.

The 40 mils with wide 2 inch margins should handle

your 10kv transformer.

However I would feel better with two 24 mil poly units in series.

If you want to series up units for a better voltage rating

then you're going to need 8 of them because each series string

of 2 caps is only going to be .01 mfd.

You can increase the plate size to pack more capacitance in each unit.

But try not to make them much longer than 10 feet or so.

Not only does it increase inductance but it becomes very difficult

to get everything aligned in a long roll.

Going to 12 inch wide foil by 100 inches is 1200 sq inches per plate.

With a 40 mil poly cap thats about .0268 mfd per cap. 3 of these

gets you very close to .08 mfd.

18 inch wide foil will make a very tall cap. But it will get

you .04 mfd in a single cap with 100 inch long plates and

40 mil poly.

I would be afraid of trapped air in the very wide plates.

My experience tells me to avoid overly large plates for that reason.

To be successful such a cap should be vacuum pump. I recommend

that for any rolled cap.

If you dont have a vacuum pump then use real transformer oil

and allow several weeks for the air to bubble out slowly

before starting break in.

Hope these ideas are of some help.

TTYL!   Mike Hammer  mhammer@misslink.net

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: Testing, Testing . .

Cc:

Bcc:

X-Attachments:

At 11:41 PM 8/1/97 GMT, you wrote:

>From: cjhill@hills.u-net.com (CJ Hill)

>Date: Sat, 2 Aug 1997 00:41:11 +0100

>Subject: Testing, Testing . .

>

>Hi,

>

>This is really just a test message as I've not been able to post to the list

>for some time (my ISP was giving me grief).

>

>I'll take the opportunity though to let you all know of my tesla web page:

>

>http://www.hills2.u-net.com/tesla.htm

>

>It's (still) in development and probably always will be, and there are no

>pretty pictures of my tesla coil yet(I need to fix my camera first!). My

>page describes my ignition coil-based setup - rather unconventional and

>probably soon to be replaced by a neon but I hope it provokes some ideas . . .

>

>Thanks, Chris.

>-------------------------------------------------

>Chris Hill, Cardiff, UK

Chris,

Your post came through just fine.

Your web has some intersting things on it.

The ignition coil driver seems to be well designed

and certainly more robust than the common designs

using 2N3055 transistors.

TTYL!   Mike Hammer  mhammer@misslink.net

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: flat plate vs rolled caps

Date: Sat,  2 Aug 1997 20:13:09 GMT

Organization: The Energy Machine Information System 504-734-0526

From: Steve Falco <sfalco@worldnet.att.net>

Date: Sat, 02 Aug 1997 16:13:09 -0400

Subject: Re: flat plate vs rolled caps

> Unfortunately your 40 mil poly isnt well suited for the task

> being too thin for a single layer and too thick for two layers.

I am confused :-)  I thought two layers of 40 mil LDPE would be just

about right.  The file keely.hvcap.zip on the www.funet.fi archive

suggested that .0625 inch thick polyethylene would be good up to 7500

volts, while .090 inch thick polyethylene would be good up to 15000

volts.  So, I assumed I would need about .080 inch thick polyethylene

for 10000 volts.  Are these numbers way too conservative?

> Still if the quality is good and you leave a 2 inch margin all around

> your plates a 40 mil cap should handle 10-12kv pretty well.

I have two thoughts on using a single layer of 40 mil LDPE:

1) The supplier in my area also stocks 4 mil poly in 4' widths.  I could

add a layer or two of this to have some safety margin.

2) I also have a large quantity of newsprint - the paper used in

printing newspapers, but without the ink!  Would it be a good idea to

add a layer of this on each side of the LDPE?  I am thinking that this

would increase the voltage rating somewhat, and would also help the oil

wick in, to avoid air bubbles.  I did find a surplus vacuum pump, so I

can pull a lot of air out that way as well.

One more question:  how do you make a good connection to aluminum foil?

The flashing I used in my previous cap was easy to terminate:  just

drill a hole, put a machine screw through it, and attach a wire.  But

foil is so thin, it will rip.  So, how do folks connect to it?

Steve Falco

sfalco@worldnet.att.net

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: flat plate vs rolled caps

Cc:

Bcc:

X-Attachments:

At 08:13 PM 8/2/97 GMT, you wrote:

>From: Steve Falco <sfalco@worldnet.att.net>

>Date: Sat, 02 Aug 1997 16:13:09 -0400

>Subject: Re: flat plate vs rolled caps

>

>> Unfortunately your 40 mil poly isnt well suited for the task

>> being too thin for a single layer and too thick for two layers.

>

>I am confused :-)  I thought two layers of 40 mil LDPE would be just

>about right.  The file keely.hvcap.zip on the www.funet.fi archive

>suggested that .0625 inch thick polyethylene would be good up to 7500

>volts, while .090 inch thick polyethylene would be good up to 15000

>volts.  So, I assumed I would need about .080 inch thick polyethylene

>for 10000 volts.  Are these numbers way too conservative?

Those numbers regarding 60 and 90 mil poly are based

on a single thick layer of poly. When using thinner poly

stacked in a layer you get a much better rating.

10 layers of 6 mil poly have a much better rating than a single

thickness of 60 mil poly.

My first experimental rolled cap was very successful at 9kv

despite having only 24 mils of poly. The 24 mils came from

4 layers of 6 mil material.

Your 2 layers of 40 mil poly will have a pretty good voltage

rating but its a balance between voltage rating and capacitance.

The thicker you make your dielectric the better the voltage

rating but the lower your capacitance.

Since you are trying for a very high capacitance it helps

to only use as much dielectric as necessary.

I doubt your H&R transformers will be able to exceed 10KV.

Indeed I think it likely they wont even get up to 10KV.

If I was building caps for use with those transformers I would

be thinking about 2 series caps each with 18 - 24 mils of

poly comprised of layers of 6 mil material. This will give

you the equivalent of 36-48 mils.

Connect enough of these series strings in parallel

to get your required capacitance.

Getting .08 mfd is not going to be easy.

Lots of construction involved any way you go.

The alternative is a commercial cap.

>

>> Still if the quality is good and you leave a 2 inch margin all around

>> your plates a 40 mil cap should handle 10-12kv pretty well.

>

>I have two thoughts on using a single layer of 40 mil LDPE:

>

>1) The supplier in my area also stocks 4 mil poly in 4' widths.  I could

>add a layer or two of this to have some safety margin.

This may help in some of the high stress areas. Especially

the beginning and ends of the plates. Be sure to round the corners

of the plates. No sharp corners allowed.

And the use of real transformer oil will improve

both the rating and robustness of your caps.

>

>2) I also have a large quantity of newsprint - the paper used in

>printing newspapers, but without the ink!  Would it be a good idea to

>add a layer of this on each side of the LDPE?  I am thinking that this

>would increase the voltage rating somewhat, and would also help the oil

>wick in, to avoid air bubbles.  I did find a surplus vacuum pump, so I

>can pull a lot of air out that way as well.

I am against the paper idea. This would be of benefit

if you have no vacuum pump available to help the oil wick

throught the layers. But a vacuum pump will do that without a loss

of capacitance.

>

>One more question:  how do you make a good connection to aluminum foil?

>The flashing I used in my previous cap was easy to terminate:  just

>drill a hole, put a machine screw through it, and attach a wire.  But

>foil is so thin, it will rip.  So, how do folks connect to it?

>

For my rolled cap I swiped an idea from Fr. McGahee.

First of all use the heavy duty foil. Much easier to handle

than the normal thinner variety.

The inner foil is wrapped 5 or 6 times tightly around a piece of 1/2

inch copper tubing. Then taped down. The foil makes contact with the

copper all along its width and provides a good low inductance connection

to the inner foil. Solder a length of heavy wire to the copper

tubing to make the connection to the output terminals. I used

AWG 8. Solder it well in a complete circle around the pipe.

For the outside foil I fold up a length of foil over and over

to make a strip 1 inch wide and about 5 or 6 layers thick.

I fold this strip up in the end of the outside foil. Fold it over

4 or 5 times. Then tape down the foil strip along the outside

of the cap roll. I have found that duct tape does NOT work

well as it begins to dissolve in mineral oil. On the last

rolled cap I used cellophane tape.

The foil strip is routed up to the output terminals.

TTYL!   Mike Hammer  mhammer@misslink.net

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: flat plate vs rolled caps

Date: Sun,  3 Aug 1997 09:14:16 GMT

Organization: The Energy Machine Information System 504-734-0526

From: msnock@theonramp.net (Micheal Huff)

Date: Sun, 03 Aug 1997 05:14:16 -0400

Subject: Re: flat plate vs rolled caps

Tesla-2 wrote:

>

> From: Steve Falco <sfalco@worldnet.att.net>

> Date: Sat, 02 Aug 1997 16:13:09 -0400

> Subject: Re: flat plate vs rolled caps

>

> > Unfortunately your 40 mil poly isnt well suited for the task

> > being too thin for a single layer and too thick for two layers.

>

> I am confused :-)  I thought two layers of 40 mil LDPE would be just

> about right.  The file keely.hvcap.zip on the www.funet.fi archive

> suggested that .0625 inch thick polyethylene would be good up to 7500

> volts, while .090 inch thick polyethylene would be good up to 15000

> volts.  So, I assumed I would need about .080 inch thick polyethylene

> for 10000 volts.  Are these numbers way too conservative?

Steve, 80 mils for a 10kV transformer should be good.  It won't be as

robust as several stacked sheets, but should hold up to that H&R

transformer.  On my coil I use two 5kV neons in series and my caps are

stacked sheet 48 mil caps which should hold up to 10kV fairly well.

> > Still if the quality is good and you leave a 2 inch margin all around

> > your plates a 40 mil cap should handle 10-12kv pretty well.

I used a 3" margin on my present caps that way if anything becomes

misaligned when rolling it is a fairly safe bet you will still have a

good margin.

> I have two thoughts on using a single layer of 40 mil LDPE:

>

> 1) The supplier in my area also stocks 4 mil poly in 4' widths.  I could

> add a layer or two of this to have some safety margin.

That would help, especially around the edges of the plates and

connecting hardware.  Be sure to round off the corners of the plates,

this will reduce the chance of internal arcing.

> 2) I also have a large quantity of newsprint - the paper used in

> printing newspapers, but without the ink!  Would it be a good idea to

> add a layer of this on each side of the LDPE?  I am thinking that this

> would increase the voltage rating somewhat, and would also help the oil

> wick in, to avoid air bubbles.  I did find a surplus vacuum pump, so I

> can pull a lot of air out that way as well.

You can use the newspaper but it will make things harder to align.  I'd

just go with the normal poly/foil cap and forget the aluminum.  Since

you have a vacuum pump there really isn't much sense in the paper

strips.

> One more question:  how do you make a good connection to aluminum foil?

I use strips of flashing which are wrapped up in the aluminum foil

plates.  I then attach a piece of neon sign wire to the flashing strip

using a screw, nut, and crimp-on connector.

--

---------------------------------------------------------------

| SNOCK'S HIGH VOLTAGE PAGE                                   |

| http://www.geocities.com/CapeCanaveral/Lab/5322/hv2.html    |

| Picures and schematics of HV devices, arcs, sparks, Jacob's |

| ladders, etc.                                               |

---------------------------------------------------------------

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Yoke Choke!

Date: Sun,  3 Aug 1997 14:05:38 GMT

Organization: The Energy Machine Information System 504-734-0526

From: cjhill@hills.u-net.com (CJ Hill)

Date: Sun, 3 Aug 1997 15:05:38 +0100

Subject: Yoke Choke!

Hi,

I've been looking to make the safety components of a new miniature Tesla

Coil I am building. In particular, I need to make some new chokes. They must

work well but be reasonably small.

From a quick look over the net and Tesla-2's past postings, the general

consensus seems to be that air-cored chokes damped with a resistor or large

toroid-based chokes are the norm. I favour the toroid option for size.

Problem. I don't seem to be able to find any of these large 2-inch or more

diameter cores mentioned in so many posts.

A quick look through my electronic 'bag of bits' revealed an old deflection

yoke from a T.V. (I'm assured they have nothing to do with eggs).

Is this core suitable?

-------------------------------------------------

Chris Hill, Cardiff, UK

cjhill@hills.u-net.com

-------------------------------------------------

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: flat plate vs rolled caps

Date: Sun,  3 Aug 1997 16:13:05 GMT

Organization: The Energy Machine Information System 504-734-0526

From: "Thomas McGahee" <tom_mcgahee@sigmais.com>

Date: Sun, 3 Aug 1997 12:13:05 -0400

Subject: Re: flat plate vs rolled caps

----------

> From: Tesla-2 <tesla-2@emachine.com>

> To: tom_mcgahee@sigmais.com

> Subject: flat plate vs rolled caps

> Date: Thursday, July 31, 1997 4:49 PM

>

> From: Mike Hammer <mhammer@misslink.net>

> Date: Thu, 31 Jul 1997 15:49:09 -0500

> Subject: flat plate vs rolled caps

>

> Hello all,

>

> Since the list has beena bit quiet lately I thought

> I would stir things up a bit by posting something

> thats been on my mind lately.

>

> Tesla coil capable caps are a big stumbling block for

> many and they only economical way to get decent caps

> is to build them ourselves.

>

> The are two basic types of homemade caps. Those are the

> stacked flat plate design and the rolled up cap.

> Each has its own advantages and disadvantages.

>

> I have always built plate caps and promoted them. They

> just seemed to be more in line with my construction

> skills and available materials. Recently I have built

> a couple experimental rolled caps and can see some advantages

> to that design.

>

> So lets run through some of the pros and cons for each type.

>

> Plate cap advantages

>

> 1. Great pulse discharge characteristics.

>    Plate caps have very low inductance so pulse very well.

>    If the best performance is what you're after then a plate

>    cap is the way to go.

>

> 2. Vertically oriented plates allow air to escape.

>    Plate caps with their plates arranged vertically will

>    allow trapped air to rise up out of the cap slowly.

>    This may take some time but is the best bet if you

>    dont have access to a vacuum pump.

>

> 3. Makes good use of thin dielectric.

>    Plate caps can use thin 4 or 6 mil hardware store variety

>    polyethylene. My .01 mfd plate cap uses 10 layers of 6 mil

>    poly for each dielectric layer.

>

>

> Plate cap disadvantages.

>

> 1. Large physical size. My .01 mfd plate cap required a tank

>    17 X 13 X 2.5 inches to house it. This large tank required

>    1.25 gallons of mineral oil. Plate caps are not compact designs.

>

Custom-built tanks with tight fits can reduce the amount of oil needed.

However, some means must be provided for expansion of the oil during

warm weather and when in operation. One method is to have air space.

Another method is to fill the unit with oil and then push on the large

flat sides of the container to force out as much oil as you can and

then seal the unit while it has this negative pressure. Another

method is to have an EXTERNAL overflow reservoir. This may be vented

to the atmosphere or consist of a flexible container.

> 2. Long construction time. Cutting all the squares of poly and

>    plates require considerable time. Even the actual assembly

>    takes quite a while.

>

> 3. Tank construction takes considerable time. I was never able to

>    find a suitable tank for my plate caps so had to construct one.

>    These were made from plywood with a plexiglass inner liner.

>    Seams were sealed with silicone RTV.

>

You can use commercially available containers, but then you have to

adjust the size of your plates accordingly. This usually results

in non-optimal use of the poly. Be aware of the fact that the weight

of the oil will need to be taken into account, as it will distort

the shape of the thinner containers. Always pre-test the container

by filling it with water. When testing is done, dry it out

thoroughly before using.

Make sure that containers are secure. Lids should be securely

attached... unless you actually like having oil spills. :(

>

>

> Rolled cap advantages.

>

> 1. Smaller physical size. The fact that the plates are rolled

>    around upon themselves results in nearly twice the capacitance

>    for the same size plates resulting in a smaller more compact

>    cap for the same amount of capacitance.

>

The capacitance is *NOT* double that of a stacked plate capacitor.

It is maybe 10% to 30% more due to better use of the poly area.

In *BOTH* types of capacitors, every poly surface should be

sandwiched between a pair of conductive plates.

The advantage of the rolled cap is that there are only *two*

"ends" of poly that require the 2" or greater margin,

whereas in a stacked plate capacitor we have this "end" loss

at *each* and every poly section.

> 2. Tanks for rolled caps easy to make. I've used PVC pipe for tanks

>    for my rolled caps. Cut a section of PVC pipe and epoxy plexiglass

>    end plates on it and there you go. Quick and easy tank.

>

> 3. Quick build time. The plates and poly are used in long strips

>    so require much less time to prepare. And the cap can be rolled

>    up in just a few minutes.

>

> 4. Lower oil requirements. By keeping the PVC pipe tanks only just

>    large enough to house the cap the amount of oil needed is much lower

>    than for plate caps. My recent .03 mfd rolled cap required only

>    1 1/2 pints of mineral oil.

>

>

> Rolled cap disadvantages

>

> 1. High inductance. A rolled cap is after all a coil.

>    By rolling the conductors you introduce lots of inductance which

>    limits the pulse performance of the cap.

>

There *IS* a way around this. Instead of having the capacitor

plates coming out of the extreme ends, have them coming out the

entire length of the SIDES instead (with at least 3" margins at the

ends). One foil set comes out one side, and the other set

comes out the other side.

This method works great with the super heavy duty aluminum foil.

I do *not* recommend it with flashing as flashing is too heavy.

What you do is scrunch the foil together all along each set of edges.

(after the capacitor is all rolled!)

Thus the "rolled" capacitor becomes in effect a sort of modified

"shorted spiral". The inductance of the capacitor then drops to

levels approacing that of the flat capacitor!! Sneaky, huh?!!

> 2. Lots of trapped air. Since the rolled type of cap has so many layers

>    and wound tightly it will trap more air than plate type caps.

>    And because of the tightness that air wont easily escape.

>    For this reason rolled caps should be vacuum pumped to help

>    remove trapped air. The use of real transformer oil is

>    recommended. Being much thinner than mineral oil it will

>    allow trapped air to escape easier.

>

The thing is to *NOT* let air get in to begin with!!!

To this end, pump the air out and backfill the oil

and then pump down again to pull out residual air.

If you just pour oil in an THEN try to get rid of the air,

you have unnecessarily complicated things!!

>

>

It is a good idea to make up a BUNCH of smaller capacitors and

then series/parallel them to get the value you need. This is

true for both rolled and flat caps. You will get the most for

your money if you build for a max voltage rating of about

9KV AC. Put in series for greater voltage ratings. The voltage

rating PER MIL is MUCH HIGHER for thin dielectrics than it is

for thick ones!!! The thinness of the dielectric also allows you to

get quite large capacitances in a fairly reasonable size package.

The trade-off here is in the number of packages that you have

to build. But it is fairly easy to get decent containers in the

smaller sizes, whereas it is hard to find large containers that

will work well. (remember that the smaller containers do not

have to contend with the great weight of oil that the larger

containers do!) Think SMALL!!

>

> My experimental rolled caps have proven to be quite worthwhile.

> I made a vacuum pump from an old dehumidifier compressor.

>

> The plates for this cap were 6 X 110 inches and were heavy duty

> aluminum foil. The poly was 9 X 120 inches and composed of 4 layers

> of 6 mil. Total thickness 24 mils. This was good for 9KV AC.

>

> The cap was pumped down with the vacuum pump and then backfilled

> with mineral oil under vacuum. I let the vacuum in the caps suck

> the oil in. After it was filled I pulled a vacuum again on the cap

> to boil out any remaining air.

>

Do not allow oil to get sucked into your pump if you can at all

avoid it! One good method is to connect a container with two fittings

on it in between the pump and the thing being pumped.

Any oil that gets in the line will go into the second container

instead of into the pump.

> The capacitance was .024 mfd.

> I ran this cap with 9kv/90ma worth of neons and got very good

> 30 inch streamers. I made several long runs of 5 minutes each.

>

> Since the cap was experimental I wanted to see what it would

> hold up too. I increased the voltage to 10kv and it ran well.

> I made several long runs over a two day period at this level.

> At 10500 volts the cap blew up. So I am in the process of

> building a pair of these caps to use in series. The plate

> dimensions were changed to 7 X 110 inches and the poly increased

> to 10 X 120 inches. Still using 4 layers for 24 mils.

> Capacitance increased to .03 mfd so the two in series will give

> .015 mfd and should handle my 12kv neons with ease.

>

> Once these caps are finished I will report on the results with

> my coil.

>

Mark any homemade cap as to its voltage and capacitance

ratings, and date of manufacture.

Keep a log that gives all the gory details of construction.

Such info can be very useful in the future.

I hope that the above added comments help someone out there.

Fr. Tom McGahee

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Rolled Poly Caps

Date: Sun,  3 Aug 1997 16:26:24 GMT

Organization: The Energy Machine Information System 504-734-0526

From: msnock@theonramp.net (Micheal Huff)

Date: Sun, 03 Aug 1997 12:26:24 -0400

Subject: Rolled Poly Caps

Here is a small text file I made a while back on rolled polyethylene

capacitors which Mike coaxed me into posting :) .  In it is construction

information for my rolled cap similar to mine.  Also, this cap is 60

mils thick so if you are only running 12kV or lower you can reduce the

thickness of the poly to 48 mils or so.

Rolled plate capacitors

------------------------------

Rolled plate capacitors are a favorite of many coilers.  They are low

loss, and can be made to be very compact.  They consist of two layers of

dielectric with aluminum foil or aluminum flashing strips sandwiched in

between the two strips of dielectric material.  After this process is

complete the entire assembly can be rolled up into a cylinder and placed

in a PVC pipe that is later filled with oil.  The oil will prevent

flashover and will supress corona.  Rolled plate capacitors are not too difficult

to build.  Low density polyetylene (often called LDPE) is normally the

material of choice here due to it's excellent RF characteristics (it's is very

low loss) and has good voltage handling ability.  Another popular material is

polypropylene.  This is what the commercial Tesla coil capacitors are

made of by Condeser Products.  It too is low loss, but handles slightly less

voltage per mil than does polyethylene.  A 60 mil multiple layer capacitor

should hold up against 15kV neon sign transformers, but it is a good idea to run

them in series and parallel to gain a good voltage margin and while maintaining

the required capacitance.

Construction of a rolled polyetylene capacitor is not too

difficult. The cost per capacitor is around $20 to $50 depending on several

factors, mainly the oil.  Some coilers will use single sheets of 30 mil or 60 mil

LDPE. This can be costly and hard to obtain.  A less expensive and easy to

obtain source for good polyetylene sheeting is from home improvement stores

such as The Home Depot or Lowes.  They sell 6 mil heavy duty LDPE sheeting for

around $10 (US) for 250 square yards.  This can be stacked up for thicknesses

of 60 mils and is better than just a single sheet of polyethylene since if

there is a defect in one strip it won't effect the whole stack, unlike a single

sheet. The material that is most commonly used for the plates is normally foil

or flashing.  Foil makes for a smaller capacitor but flashing is much

easier to work with.  The plates and dielectric material is normally wound on a

small 1" or 3/4" PVC pipe.  The pipe is left to hang out of each end of the roll

an inch or two.  The roll should be tight and then immersed in transformer

or mineral oil.

This capacitor is around .013 uF.

250 square yards of 6 mil polyetylene sheeting

One roll of "Reynold's Wrap" 12" aluminum foil or other heavyduty foil.

21.25" long piece of 3/4" schedule 40 PVC pipe

21" long piece of 4" thinwall PVC pipe

4" PVC end caps

Two 1/4" brass bolts about and 1" long and four 1/4" brass nuts

Two strips of 1/2" to 3/4" wide aluminum flashing 8" in length

Two pieces of 6" long neon sign "GTO" wire or other high voltage wire

Two small 1/8" screws about 3/8" long and nuts

Spray adhesive that is good for plastics (this is not required)

1 gallon of pure mineral oil or transformer oil

Find a nice and clean area to work as dirt may lead to breakdowns.

Start by cutting twenty strips of polyethylene 18" in width six feet

long. This can be difficult to do without a straight edge.  A two by four

works well for this purpose.  Now cut off two pieces of aluminum foil 64" long.

Stack and align ten strips of LDPE until a thickness of 60 mils has been

achieved. Do the same with the other ten strips but keep it separate from the

first stack.  A little amount of spray adhesive will help keep the stacks from

becoming crooked.  You now should have two plates (the aluminum foil)

and two pieces of polyetylene dielectric.  Now place one aluminum foil plate on

the strip of 60 mil LDPE.  Make sure it is well centered.  There should be a

border of 3" on the sides and 4" on the ends.  Now stack the other piece of

60 mil LDPE on top of this.  The remaining plate should now be stacked

on this and centered well.  The well centered plates and LDPE can now be rolled

around the 3/4" PVC pipe. Paper clips will work to hold the sheeting in place

while rolling and so will the spray adhesive if you happened to use it. The

paper clips will obviously have to be removed when the rolling is complete.

When rolling it, is a good idea to have some tension on the opposite end.

Have someone else hold this while you roll it as this will be a tighter roll

and will take much less time to complete.  Make sure while rolling that the

18" wide LDPE stays in the center of the 3/4" PVC pipe.  After the plates

and dielectric has been rolled it can be placed into the 4" PVC tank.  The

bottom end cap should be glued on with some good PVC cement and should have a

water tight seal.  The top end cap must now be drilled to allow holes for the

terminal studs.  Drill two 1/4" holes 1/2" away from the sides of the

end cap. Place both 1/4" bolts in so that the threaded portion will stick up

through the end cap.  The nuts can be twisted on but don't tighten them up yet.

Take the two strips of flashing and drill a 1/8" in diameter hole  in the

center at one end.  Attach the very flexible neon sign wire or other high voltage

wire with the small screws and nuts to the strips of flashing.  The end of

the flashing with no holes or wires attached to it must now be slipped down

into the capacitor roll to make contact with the plates.  Be careful not to

rip the foil.  The two strips of flashing must be placed the farthest possible

distance from eachother to prevent arcing.  Slip the strips down in deep

enough until the screws touch the top of the LDPE strips.  Don't place

the top end cap with the studs in it on yet.  Pour enough oil to cover the whole

rolled assembly completely and then about an inch past that.

The capacitor must now be allowed to sit a week or two to allow air bubbles

to escape. Vibrations will help them escape.  After that amount of waiting time the

high voltage wires can be connected to the studs and the end cap may be

pressed down onto the oil tank.  Don't glue the end cap on.  This will act as a valve

in case high pressures should build up inside.  It will also be easier to

access the plates and dielectric material should failure occur.  The end cap

may be glued on if you so choose but put a "weak spot" in somewhere that will

crack if pressure should build up. You should end up with a fairly compact and

robust pulse discharge rated capacitor good for Tesla coil use.

Mike

--

---------------------------------------------------------------

| SNOCK'S HIGH VOLTAGE PAGE                                   |

| http://www.geocities.com/CapeCanaveral/Lab/5322/            |

| Picures and schematics of HV devices, arcs, sparks, Jacob's |

| ladders, Tesla coils

---------------------------------------------------------------

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: primary

Date: Sun,  3 Aug 1997 17:42:42 GMT

Organization: The Energy Machine Information System 504-734-0526

From: Krumins  Aaron <krumins@bvsd.k12.co.us>

Date: Sun, 3 Aug 1997 11:42:42 -0600 (MDT)

Subject: primary

whats a good gauge wire for my primary coil powered by a 9kv neon sign

transformer?

Thanks, Aaron Krumins

_____________________________________________

|Aaron Krumins                              |

|Devoted learner of computers & the internet|

|Krumins@bvsd.k12.co.us                     |

|                                           |

|"Silicone Valley or bust"                  |

---------------------------------------------

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: Yoke Choke!

Cc:

Bcc:

X-Attachments:

At 02:05 PM 8/3/97 GMT, you wrote:

>From: cjhill@hills.u-net.com (CJ Hill)

>Date: Sun, 3 Aug 1997 15:05:38 +0100

>Subject: Yoke Choke!

>

>Hi,

>

>I've been looking to make the safety components of a new miniature Tesla

>Coil I am building. In particular, I need to make some new chokes. They must

>work well but be reasonably small.

>

>From a quick look over the net and Tesla-2's past postings, the general

>consensus seems to be that air-cored chokes damped with a resistor or large

>toroid-based chokes are the norm. I favour the toroid option for size.

>

>Problem. I don't seem to be able to find any of these large 2-inch or more

>diameter cores mentioned in so many posts.

Hosfelt Electronics used to have a perfect toroid for

RF chokes and was cheap at $1.50 each. But they have sold

out of those and there wont be any more.

Fortunately I purchased 6 of them while they were available.

Marlin P. Jones sometimes has suitable cores but I dont see

anything in their current catalog.

I often see surplus toroids of this type at hamfests.

>

>A quick look through my electronic 'bag of bits' revealed an old deflection

>yoke from a T.V. (I'm assured they have nothing to do with eggs).

>

>Is this core suitable?

I would think so. For a RF choke on a Tesla coil you

do NOT want a low loss high Q choke. You want it lossy

and inefficient. So yes try out your cores. They will

probably be fine for our purpose.

Dont use heavy large guage wire. AWG 26-28 is just fine.

The current they must carry is rather low.

TTYL!   Mike Hammer  mhammer@misslink.net

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: primary

Cc:

Bcc:

X-Attachments:

At 05:42 PM 8/3/97 GMT, you wrote:

>From: Krumins  Aaron <krumins@bvsd.k12.co.us>

>Date: Sun, 3 Aug 1997 11:42:42 -0600 (MDT)

>Subject: primary

>

>whats a good gauge wire for my primary coil powered by a 9kv neon sign

>transformer?

>

>                                Thanks, Aaron Krumins

Aaron,

The general rule for primary coils is to use heavy

low loss conductors. Copper tubing being ideal.

Compare the cost of 1/4 inch copper tubing vs wire.

But when operating a small coil at low power levels

you can get by with considerably smaller conductors.

List member Corey Edmunds used AWG 12 solid copper wire

for his primary and is getting good results with a 12kv/30ma

neon.

I have made small coils with AWG 10 and 12 solid copper wire with

good success.

My 4 inch diameter coil has a primary from AWG 6 solid copper

wire and does well at over 1.2 KVA.

I wouldnt recommend going any smaller than AWG 12.

If you have a choice always go for the heavier material.

After all you may not always be running "just" 9kv/30ma. :)

TTYL!   Mike Hammer  mhammer@misslink.net

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: primary

Date: Mon,  4 Aug 1997 06:12:11 GMT

Organization: The Energy Machine Information System 504-734-0526

From: corr <corr@fullnet.net>

Date: Mon, 04 Aug 1997 00:12:11 -0600

Subject: Re: primary

AWG 12 should be alright

>whats a good gauge wire for my primary coil powered by a 9kv neon sign

>transformer?

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: flat plate vs rolled

Date: Mon,  4 Aug 1997 07:21:00 GMT

Organization: The Energy Machine Information System 504-734-0526

T>The thing is to *NOT* let air get in to begin with!!!

T>To this end, pump the air out and backfill the oil

T>and then pump down again to pull out residual air.

T>If you just pour oil in an THEN try to get rid of the air,

T>you have unnecessarily complicated things!!

Note: The following is not for the faint of heart!

When I used to build my own pulse forming networks for my home built

radar system, I would heat the oil until it was just starting to smoke a

bit, then fill the HV tank with the hot oil. That also boiled the water

out of the cardboard forms I used for some of the coils.  Since the oil

was thin because of the heating, it displaced the air pretty fast. For

some of the larger units I would heat the tank after it was filled with

cool oil.  After the air and water vapor stopped escaping from the hot

oil, I would seal the tank and let it cool.

Ralph

---

A family reunion is an effective form of birth control

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: primary

Date: Mon,  4 Aug 1997 10:03:20 GMT

Organization: The Energy Machine Information System 504-734-0526

From: msnock@theonramp.net (Micheal Huff)

Date: Mon, 04 Aug 1997 06:03:20 -0400

Subject: Re: primary

Tesla-2 wrote:

> From: Krumins  Aaron <krumins@bvsd.k12.co.us>

> Date: Sun, 3 Aug 1997 11:42:42 -0600 (MDT)

> Subject: primary

>

> whats a good gauge wire for my primary coil powered by a 9kv neon sign

>

> transformer?

I'd go with 6 or 8 awg solid wire if cost is to be kept low, otherwise

1/4" soft copper tubing can be used.  Don't use anything below 6 awg as

it will get very hard to wind and could put too much stress on your

supports.  Soft copper refrigeration tubing is the way to go as far as

looks and ease of winding but expect to spend around $20-25 for a 50 ft.

roll.  Smaller rolls of this copper tubing can be soldered together if a

50 ft. roll cannot be found.

Mike

--

---------------------------------------------------------------

| SNOCK'S HIGH VOLTAGE PAGE                                   |

| http://www.geocities.com/CapeCanaveral/Lab/5322/hv2.html    |

| Picures and schematics of HV devices, arcs, sparks, Jacob's |

| ladders, etc.                                               |

---------------------------------------------------------------

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: flat plate vs rolled caps

Cc:

Bcc:

X-Attachments:

At 04:13 PM 8/3/97 GMT, you wrote:

>From: "Thomas McGahee" <tom_mcgahee@sigmais.com>

>Date: Sun, 3 Aug 1997 12:13:05 -0400

>Subject: Re: flat plate vs rolled caps

>

>

>

<SNIP>

>> Rolled cap advantages.

>>

>> 1. Smaller physical size. The fact that the plates are rolled

>>    around upon themselves results in nearly twice the capacitance

>>    for the same size plates resulting in a smaller more compact

>>    cap for the same amount of capacitance.

>>

>

>The capacitance is *NOT* double that of a stacked plate capacitor.

>It is maybe 10% to 30% more due to better use of the poly area.

>In *BOTH* types of capacitors, every poly surface should be

>sandwiched between a pair of conductive plates.

>

Whoa! Hold on there.

Here is the formula for figuring the capacitance

of homemade caps.

Capacitance (in microfarads) = (0.224 KA / d) (N-1)

0.224 x Dielectric Strength x Area of plate

C = -----------------------------------------  x (Number of plates - 1)

distance between plates in inches

NOTE: For two plate rolled capacitors the value must be computed

with the number of plates (N) equal to three (3), so (N-1 = 2)

For example a flat plate cap with 2 plates of 10 X 10 inches

with 24 mils of poly (K=2) is equal to:

0.224 X 2 X 100

------------------  X 1 =  1866.67 pf

.024

Same specs in a rolled cap

0.224 X 2 X 100

------------------  X 2 = 3733.33 pf

.024

Is this not twice the capacitance for the same plate area?

I have used both these formulas in both rolled and flat plate

caps and compared results against a digital LCR meter.

Using a value of 2.0 for the dielectric constant of poly

brings the calculated values very close to real world results.

TTYL!   Mike Hammer  mhammer@misslink.net

From: tesla-2@emachine.com (Tesla-2)

Subject: Beginning  work on my caps.

Date: Mon,  4 Aug 1997 19:37:14 GMT

Organization: The Energy Machine Information System 504-734-0526

From: "Paul Marshall" <PMARSHAL@ETHCINC1.SSW.JNJ.COM>

Date: 04 Aug 1997 14:37:14 EST

Subject: Beginning  work on my caps.

I have just purchased some 4 mil polyethylene and I was wondering

how thick to make my dielectric. My transformer is rated at 15kv

120ma. How thick should the plates be if I make a rectangular stack ?

How thick should the dielectric be if I make a roll Cap ?

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: RIP 1 Neon

Date: Mon,  4 Aug 1997 21:04:22 GMT

Organization: The Energy Machine Information System 504-734-0526

From: Nathan_Bonsal@intuit.com (Nathan Bonsal)

Date: Mon, 4 Aug 1997 14:04:22 -0700

Subject: RIP 1 Neon

While testing one of my neons after unpotting it, I arced one of the

lead terminals to ground to test spark length. One of the secondary

windings began to arc to the core.I measured again with an ohmmeter

and at that low voltage level noticed no change in resistance. Is it

possible that another carbonization track has occurred and appears as

a partial short at high voltage? does anyone think that reimmersing it

in gasoline to get more potting out of the windings would help?

I am not sure whether this neon can be fixed but I would like to know

whether anyone else has had this happen. Next time I will immerse it

in mineral oil, having learned my lesson.

Also this neon's core, instead of being held together with pins or

screws, is kind of dovetailed together and probably spot welded. I am

working on the next one, a France 15K/30 unit, and haven't found yet

how the core is assembled. If anyone can help me, let me know!

Thanks!

Nathan Bonsal

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: flat plate vs rolled caps

Date: Mon,  4 Aug 1997 22:21:33 GMT

Organization: The Energy Machine Information System 504-734-0526

From: "Thomas McGahee" <tom_mcgahee@sigmais.com>

Date: Mon, 4 Aug 1997 18:21:33 -0400

Subject: Re: flat plate vs rolled caps

----------

> From: Tesla-2 <tesla-2@emachine.com>

> To: tom_mcgahee@sigmais.com

> Subject: Re: flat plate vs rolled caps

> Date: Monday, August 04, 1997 1:51 PM

>

> From: Mike Hammer <mhammer@misslink.net>

> Date: Mon, 04 Aug 1997 12:51:16 -0500

> Subject: Re: flat plate vs rolled caps

>

> At 04:13 PM 8/3/97 GMT, you wrote:

> >From: "Thomas McGahee" <tom_mcgahee@sigmais.com>

> >Date: Sun, 3 Aug 1997 12:13:05 -0400

> >Subject: Re: flat plate vs rolled caps

> >

> >

> >

> <SNIP>

>

> >> Rolled cap advantages.

> >>

> >> 1. Smaller physical size. The fact that the plates are rolled

> >>    around upon themselves results in nearly twice the

capacitance

> >>    for the same size plates resulting in a smaller more compact

> >>    cap for the same amount of capacitance.

> >>

> >

> >The capacitance is *NOT* double that of a stacked plate capacitor.

> >It is maybe 10% to 30% more due to better use of the poly area.

> >In *BOTH* types of capacitors, every poly surface should be

> >sandwiched between a pair of conductive plates.

> >

>

>

> Whoa! Hold on there.

> Here is the formula for figuring the capacitance

> of homemade caps.

>

>  Capacitance (in microfarads) = (0.224 KA / d) (N-1)

>

>

>         0.224 x Dielectric Strength x Area of plate

>     C = -----------------------------------------  x (Number of

plates - 1)

>           distance between plates in inches

>

>     NOTE: For two plate rolled capacitors the value must be

computed

>           with the number of plates (N) equal to three (3), so (N-1

= 2)

>

>

>

> For example a flat plate cap with 2 plates of 10 X 10 inches

> with 24 mils of poly (K=2) is equal to:

>

>   0.224 X 2 X 100

> ------------------  X 1 =  1866.67 pf

>       .024

>

*** (Mike, how many POLY sheets here.... ONE)

>

>

> Same specs in a rolled cap

>

>

>   0.224 X 2 X 100

> ------------------  X 2 = 3733.33 pf

>      .024

>

*** (Mike, how many POLY sheets here.... TWO)

>

> Is this not twice the capacitance for the same plate area?

>

> I have used both these formulas in both rolled and flat plate

> caps and compared results against a digital LCR meter.

> Using a value of 2.0 for the dielectric constant of poly

> brings the calculated values very close to real world results.

>

>

> TTYL!   Mike Hammer  mhammer@misslink.net

> Mike Hammer

> mhammer@misslink.net

>

Your flat plate example used ONE sheet of poly, and the rolled cap

used TWO sheets of poly!!!! This is exactly the kind of "half-truth"

that I was trying to point out in my reply to the original post,

Mike! In answer to your statement

>

> Is this not twice the capacitance for the same plate area?

>

I have to say *NO*!!!

The confusion comes because of the meaning of PLATE AREA. Two

capacitors made with the same type and thickness of dielectric

material and having the same CAPACITANCE will by definition have the

same ACTIVE plate area. The key word here is ACTIVE. Each plate has

two main surfaces (ignoring the edges...).

In the rolled capacitor most of BOTH sides of the plates are used

(but not 100%!) In the example of a flat cap with two flat plates,

only ONE side of each plate is ACTIVE. But that is actually a bad

example, because in the real world we STACK plates, and so only the

END plates have unused surface area.

Get it? In a stacked capacitor with 100 plates, 98 of them have TWO

surfaces used, and 2 of them have ONE surface used for a total of

2*(n-1) ACTIVE surfaces. See where the (n-1) comes in? There will

ALWAYS be exactly two unused surfaces in a flat stacked capacitor. As

the number of plates increases, the wasted percentage decreases

because the ratio of (n-1)/n will approach unity. So a really stupid

*TWO* plate flat cap gets a ratio of 1/2, but a 100 plate cap gets a

ratio of 99/100 which is obviously much better. In the real world we

would be using things like 20 poly sheets, so 19/20 or 95% would be

what I would expect to find as an actual percentage. That is a far

cry from the 50% you get when you plug in just ONE poly section into

the equation!

And if you want to get really nit picky and choose really bad

examples to "prove" a point, see what happens when you make a

"rolled" capacitor of exactly *one* turn. Why, it is exactly the same

as the dumb two plate flat capacitor, except you have thrown in an

extra (unused) sheet of poly!! Yeah, but the "extra unused" sheet

WOULD be used if you had more than one turn. right?

If you want to compare capacitor geometries, choosing one turn for

the rolled cap or just two plates for the flat cap is not really a

good choice. THAT is what I object to.

The rolled cap *DOES* have a real advantage in that there is less

wasted poly because it has only two ends, whereas a flat stack cap of

100 poly sections would have 200 ends. It thus saves both on poly and

on plate material... but it is really not totally correct to say that

you get twice the capacitance for the plate area.

By the way, the rolled capacitor will always have an UNUSED portion

of the plate area that is a function of the height and outside

circumference of the outermost turn and height and inside

circumference of the innermost turn. For a SINGLE turn rolled cap

this LOSS is identical to that found in a single flat plate

capacitor. This outer circumference loss is often not taken into

account, and it is one of the reasons why some people get measured

values that differ much from their calculated values. The other

reason is greater than calculated distance between plates caused by

inadequate compression of the roll.

Note that for a rolled capacitor having an outside diameter of D and

a plate height of H the wasted plate area will be 3.14*D*H.

Hope this clears things up.

Fr. Tom McGahee

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: Beginning  work on my caps.

Cc:

Bcc:

X-Attachments:

At 07:37 PM 8/4/97 GMT, you wrote:

>From: "Paul Marshall" <PMARSHAL@ETHCINC1.SSW.JNJ.COM>

>Date: 04 Aug 1997 14:37:14 EST

>Subject: Beginning  work on my caps.

>

>

>     I have just purchased some 4 mil polyethylene and I was wondering

>     how thick to make my dielectric. My transformer is rated at 15kv

>     120ma. How thick should the plates be if I make a rectangular stack ?

>     How thick should the dielectric be if I make a roll Cap ?

>

>

>

Paul,

To make a reliable cap for 15kv use I would consider

making no less than 2 caps and wire them in series.

Its far easier to make 2 reliable 7500 volt caps

than 1 realiable 15kv cap.

You will need a minimum of 60 mils of poly

for 15 kv. That comes to 30 mils per cap.

You can make them either plate or rolled.

The high capactiance value needed is easier to

obtain in flat plate caps.

If you are making a flat plate design the 2 caps

can be in the same housing.

Since you probably are going to want .015-.02 mfd for

that big neon you will need .03-.04 mfd for each cap.

Use 32 mils of poly for each cap. Thats 8 layers

of your 4 mil poly. Allow a 2 inch margin all

around the plates.

Lets look at a couple examples to see how big

a project this is going to be.

Using 8 X 8 inch plates and 8 layers of 4 mil poly

for each layer 46 plates will give .04 mfd.

2 of these in series should handle 15kv well

and result in slightly over .02 mfd.

This requires 46 plates and 344 poly squares per cap.

Thats 92 plates and 688 poly squares to cut.

This would be a huge time consuming project. But it

will result in a very good performing cap.

You can try to squeeze .02 mfd into one cap but it

is almost sure to fail at 15kv. But for the sake of

arguement lets run the numbers.

Lets assume we need a minimum of 60 mils of poly.

Thats 15 layers of your 4 mil material.

Again lets try 8 X 8 plates.

It appears 44 plates fill the bill resulting in .0205 mfd.

This will require 43 X 15 poly sheets for dielectric. 645 sheets.

This will work but is more likely to fail.

If you want to try a rolled up cap you will have the same

dielectric requirements. But its difficult to get this level

of capacitance in a rolled cap. Rolled caps cannot be scaled

up like plate caps. As the roll gets longer the inductance goes

up and limits performance.

What I suggest for a rolled cap is to make 4 caps

each .02 mfd. Run two in series and these two series strings

in parallel. The result will be .02 mfd with the voltage

divide between the caps.

You will want 32 mils of poly (8 X 4 mil)

and plates 12 X 60 inches. Rolled up this gives slightly in excess

of .02 mfd. Trying to roll up 16 layers of poly will be

very difficult. But can be done.

Changing to 6 mil poly sheeting will reduce the required number

of layers.

I think you can begin to see why I normally recommend

that people stay away from 15 kv neons. Just makes it

that much harder to make reliable caps.

These are just a few ideas that came to mind while writing this.

There are other combinations and other parallel/series arrangements

that will work.

These are just a couple possible designs.

I hope you like making caps because you got a lot of work

ahead of you. :)

Lets us know what you decide to do and how

everything works out.

TTYL!   Mike Hammer  mhammer@misslink.net

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Tesla 2 archive

Cc:

Bcc:

X-Attachments:

To all list subscribers.

I have placed the 5th archive file for

the Tesla-2 list on the funet.fi ftp site.

This brings the archive up to July 1 1997.

File name is tes2arc5.zip.

Address for funet.fi is

ftp.funet.fi/pub/sci/electrical/tesla/maillist/tesla2/

TTYL!   Mike Hammer  mhammer@misslink.net

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: Beginning  work on my caps.

Date: Mon,  4 Aug 1997 23:13:51 GMT

Organization: The Energy Machine Information System 504-734-0526

From: Jim <jwbuck@loclnet.com>

Date: Mon, 04 Aug 1997 19:13:51 -0400

Subject: Re: Beginning  work on my caps.

Tesla-2 wrote:

-snip-

>   I hope you like making caps because that's going to take quite a bit

> to match up with that transformer.  I personally would go with a rolled

> plate capacitor since it will end up much smaller and will use up less

> poly.  Rolled plate capacitors don't take much time to make and

> construction is fairly easy.  I don't think 4 mil poly is really what

> you want to use.  I'd go with a 60 mil cap made up of ten sheets of 6

> mil poly.  When rolling it is often times hard to keep the poly aligned

I had the same problem with mine.  I found that thinly spreading mineral

oil between the sheets of poly made everything stick together so the

rolling was much easier.

> so that's why I say to use 6 (or even 8 if you can find it) mil poly.

> You should also use a 3" border on each edge since you are using 15kV

> and the extra margin will allow the poly to be a little misaligned

> without compromising the thickness at the edges.  Having already built

> two rolled caps and made an attempt at a flate plate (takes hours just

> to cut all of the plates out) I would say a rolled .02uF cap is the best

> bet.

>

> Mike

> --

>

>        ---------------------------------------------------------------

>        | SNOCK'S HIGH VOLTAGE PAGE                                   |

>        | http://www.geocities.com/CapeCanaveral/Lab/5322/hv2.html    |

>        | Picures and schematics of HV devices, arcs, sparks, Jacob's |

>        | ladders, etc.                                               |

>        ---------------------------------------------------------------

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: Beginning  work on my caps.

Date: Tue,  5 Aug 1997 08:09:21 GMT

Organization: The Energy Machine Information System 504-734-0526

From: msnock@theonramp.net (Micheal Huff)

Date: Tue, 05 Aug 1997 04:09:21 -0400

Subject: Re: Beginning  work on my caps.

Tesla-2 wrote:

> From: "Paul Marshall" <PMARSHAL@ETHCINC1.SSW.JNJ.COM>

> Date: 04 Aug 1997 14:37:14 EST

> Subject: Beginning  work on my caps.

>

>      I have just purchased some 4 mil polyethylene and I was wondering>

>      how thick to make my dielectric. My transformer is rated at 15kv

>      120ma. How thick should the plates be if I make a rectangular stack ?

>      How thick should the dielectric be if I make a roll Cap ?

>

I hope you like making caps because that's going to take quite a bit

to match up with that transformer.  I personally would go with a rolled

plate capacitor since it will end up much smaller and will use up less

poly.  Rolled plate capacitors don't take much time to make and

construction is fairly easy.  I don't think 4 mil poly is really what

you want to use.  I'd go with a 60 mil cap made up of ten sheets of 6

mil poly.  When rolling it is often times hard to keep the poly aligned

so that's why I say to use 6 (or even 8 if you can find it) mil poly.

You should also use a 3" border on each edge since you are using 15kV

and the extra margin will allow the poly to be a little misaligned

without compromising the thickness at the edges.  Having already built

two rolled caps and made an attempt at a flate plate (takes hours just

to cut all of the plates out) I would say a rolled .02uF cap is the best

bet.

Mike

--

---------------------------------------------------------------

| SNOCK'S HIGH VOLTAGE PAGE                                   |

| http://www.geocities.com/CapeCanaveral/Lab/5322/hv2.html    |

| Picures and schematics of HV devices, arcs, sparks, Jacob's |

| ladders, etc.                                               |

---------------------------------------------------------------

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: flat plate vs rolled caps

Date: Tue,  5 Aug 1997 11:34:50 GMT

Organization: The Energy Machine Information System 504-734-0526

From: Steve Falco <sfalco@worldnet.att.net>

Date: Tue, 05 Aug 1997 07:34:50 -0400

Subject: Re: flat plate vs rolled caps

> Your flat plate example used ONE sheet of poly, and the rolled cap

> used TWO sheets of poly!!!! This is exactly the kind of "half-truth"

> that I was trying to point out in my reply to the original post,

> Mike! In answer to your statement

> >

> > Is this not twice the capacitance for the same plate area?

> >

> I have to say *NO*!!!

I agree with Fr. Tom.  Rolled caps and flat plate caps come out about

the same for a given plate count.  I spent a while thinking about this

when I wrote ntesla.exe.

Two more things to add.  You actually lose a little capacitance in a

rolled cap, because the outer turn does not have usually have metal on

the outside of the last layer of dielectric.  Being pedantic, ntesla

takes this into account.

Also, and I have never seen anyone build one, but if you had an odd

number of plates in a rolled cap, then the capacitance would be the same

as if you had one less plate (the next smaller even number).  Ntesla

allows you to design a 4 plate rolled cap.  Don't know if it can really

be built, but it would be one way to lower the inductance.

Steve Falco

sfalco@worldnet.att.net

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: flat plate vs rolled caps

Cc:

Bcc:

X-Attachments:

At 11:34 AM 8/5/97 GMT, you wrote:

>From: Steve Falco <sfalco@worldnet.att.net>

>Date: Tue, 05 Aug 1997 07:34:50 -0400

>Subject: Re: flat plate vs rolled caps

>

>> Your flat plate example used ONE sheet of poly, and the rolled cap

>> used TWO sheets of poly!!!! This is exactly the kind of "half-truth"

>> that I was trying to point out in my reply to the original post,

>> Mike! In answer to your statement

>> >

>> > Is this not twice the capacitance for the same plate area?

>> >

>> I have to say *NO*!!!

>

>I agree with Fr. Tom.  Rolled caps and flat plate caps come out about

>the same for a given plate count.  I spent a while thinking about this

>when I wrote ntesla.exe.

>

Yea I neglected to take into account the diminishing

effect of the outer plates as the number of plates

is increased.

Not the first time I've been reprimanded by Fr. Tom

and undoubtedly not the last.

This whole concept got me thinking so I decided to

run a few calculations to see how much of a difference

there is for differing numbers of plates.

Remember my examples from the earlier post?

For example a flat plate cap with 2 plates of 10 X 10 inches

with 24 mils of poly (K=2) is equal to:

0.224 X 2 X 100

------------------  X 1 =  1866.67 pf

.024

Same specs in a rolled cap

0.224 X 2 X 100

------------------  X 2 = 3733.33 pf

.024

A two plate flat cap with 200 sq inches of plate material

scores 1866.7 pg.

The same plates rolled up score 3733.33 pf.

Lets look what increasing the number of plates does while

keeping the same 200 sq inches of plate material.

# of plates      Cap (pf)   % of rolled cap (3733.33 pf)

4             2800       75

8             3266       87

16             3500       93

32             3616       96

64             3675       98

Hmm. That pretty well sums it up.

I hadnt considered plate vs rolled caps in this manner before.

Learn something new everyday.

Thanks to Fr. Tom for discovering my error.

Dont you just hate guys like that? :)

TTYL!   Mike Hammer  mhammer@misslink.net

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: flat plate vs rolled caps

Date: Tue,  5 Aug 1997 18:30:19 GMT

Organization: The Energy Machine Information System 504-734-0526

From: "Thomas McGahee" <tom_mcgahee@sigmais.com>

Date: Tue, 5 Aug 1997 14:30:19 -0400

Subject: Re: flat plate vs rolled caps

----------

> From: Tesla-2 <tesla-2@emachine.com>

> To: tom_mcgahee@sigmais.com

> Subject: Re: flat plate vs rolled caps

> Date: Saturday, August 02, 1997 4:13 PM

>

> From: Steve Falco <sfalco@worldnet.att.net>

> Date: Sat, 02 Aug 1997 16:13:09 -0400

> Subject: Re: flat plate vs rolled caps

>

> > Unfortunately your 40 mil poly isnt well suited for the task

> > being too thin for a single layer and too thick for two layers.

>

> I am confused :-)  I thought two layers of 40 mil LDPE would bejust

> about right.  The file keely.hvcap.zip on the www.funet.fi archive

> suggested that .0625 inch thick polyethylene would be good up to7500

> volts, while .090 inch thick polyethylene would be good up to 15000

> volts.  So, I assumed I would need about .080 inch thick polyethylene

> for 10000 volts.  Are these numbers way too conservative?

>

For BEST use of your 40 mil poly, make TWO capacitors of twice the

desired capacitance and then connect them in series.

A fact that is not too well known is that thinner layers of poly can

take higher voltages PER MIL than thicker layers. Thus two caps made

with 40 mil poly and connected in series will have a MUCH higher

final voltage rating than a single 80 mil poly layer would have had.

This does require more in the way of plate material, but if using

heavy foil, the additional cost is minimal. Go for two caps in series

and save yourself a LOT of headaches later!!!

> > Still if the quality is good and you leave a 2 inch margin allaround

> > your plates a 40 mil cap should handle 10-12kv pretty well.

>

> I have two thoughts on using a single layer of 40 mil LDPE:

>

> 1) The supplier in my area also stocks 4 mil poly in 4' widths. I could

> add a layer or two of this to have some safety margin.

>

> 2) I also have a large quantity of newsprint - the paper used in

> printing newspapers, but without the ink!  Would it be a good idea to

> add a layer of this on each side of the LDPE?  I am thinking that this

> would increase the voltage rating somewhat, and would also help the oil

> wick in, to avoid air bubbles.  I did find a surplus vacuum pump, so I

> can pull a lot of air out that way as well.

>

I have used paper with excellent results. The loss in total

capacitance amounts to less than 4%, so GO FOR IT! Paper should be

slightly larger than the poly, or equal in size.

> One more question:  how do you make a good connection to aluminum foil?

> The flashing I used in my previous cap was easy to terminate:  just

> drill a hole, put a machine screw through it, and attach a wire. But

> foil is so thin, it will rip.  So, how do folks connect to it?

>

One way is to make a metal compression clamp. This is just two sheets

of aluminum strips with four or more holes drilled in each. The holes

in one piece should be LARGER than the diameter of the SHEET METAL

SCREWS to be used to hold them together. The other piece should have

holes about 3/4 the diameter of the sheet metal screws. You can then

use the sheet metal screws to go through the big hole, through the

many layers of foil (fold over if desired), and then into the small

hole. If you do not like the idea of using sheet metal screws, then

use bolts and have all holes larger than the bolt size.

Another method is to place a strip of aluminum sheet say 2*W+4" than

the width (W) of the aluminum foil at the end of the foil ends such

that the middle of the strip is even with one edge of the foil. Now

roll the foil tightly around the aluminum strip several times. BEND

the strip over tightly and then continue to roll up any excess foil.

You now have an aluminum strip you can make electrical connections

to.

>         Steve Falco

>         sfalco@worldnet.att.net

>

Steve, I hope the above suggestions give you some ideas as to how you

can help build your caps.

Fr. Tom McGahee

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re[2]: Glass Capacitor

Date: Tue,  5 Aug 1997 21:39:22 GMT

Organization: The Energy Machine Information System 504-734-0526

From: Nathan_Bonsal@intuit.com (Nathan Bonsal)

Date: Tue, 5 Aug 1997 14:39:22 -0700

Subject: Re[2]: Glass Capacitor

If I were to use 9X12 pieces of glass as dielectric (temporarily)

and 8X11  pieces of foil or flashing, how many plates would it take to

get to .01 mfd for my cap?

I do intend to build a rolled cap with polyethelene and a poly stacked

plate,  but I want to know how many layers a stacked plate will need.

I've got 10 layers done already. I'm hoping 10 will be enough. I could

be wrong....

Also what if you parallel a stacked plate with a rolled cap of the

same value? Because of the inductance of the rolled cap, would it

perform differently than two rolled or two stacked?

Just curious. I guess those experiments are actually the reason we

build these things. On that last one I'll try it myself.

I'm also interested in what ages everybody on this list is so I can

get an idea what types of people are out there coiling.

Nathan_Bonsal@Intuit.com

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: flat plate vs rolled caps (LDPE k question)

Date: Tue,  5 Aug 1997 21:58:23 GMT

Organization: The Energy Machine Information System 504-734-0526

From: "Michael Smith" <md.smith@usa.net>

Date: Tue, 5 Aug 1997 16:58:23 -0500

Subject: Re: flat plate vs rolled caps (LDPE k question)

----------

> From: Tesla-2 <tesla-2@emachine.com>

> To: md.smith@usa.net

> Subject: Re: flat plate vs rolled caps

> Date: Tuesday, August 05, 1997 1:26 PM

>

> From: Mike Hammer <mhammer@misslink.net>

> Date: Tue, 05 Aug 1997 13:26:02 -0500

> Subject: Re: flat plate vs rolled caps

>

> At 11:34 AM 8/5/97 GMT, you wrote:

> >From: Steve Falco <sfalco@worldnet.att.net>

> >Date: Tue, 05 Aug 1997 07:34:50 -0400

> >Subject: Re: flat plate vs rolled caps

> >

>  For example a flat plate cap with 2 plates of 10 X 10 inches

>  with 24 mils of poly (K=2) is equal to:

>

>    0.224 X 2 X 100

>  ------------------  X 1 =  1866.67 pf

>        .024

<snippus gigantus>

Hi Mike,

How universally accepted is the use of a k of 2 for LDPE rather than 2.3?

Have you seen this on numerous occasions, after building from

calculations and comparing to the finished product? Are you sure the

other variables were eliminated?

I am asking this because I am poised to begin on a flat Capacitor. I do

not want to be too far off of my final value of .01.  I will be radiusing

my plate corners tonight, then cutting up that magnificent roll of 8 mil

i got, and the almost equally groovy big roll of kraft paper.

Michael Smith

md.smith@usa.net

Ask and it will be given

Seek and you will find

Knock and the door will be opened.

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: Re[2]: Glass Capacitor

Cc:

Bcc:

X-Attachments:

At 09:39 PM 8/5/97 GMT, you wrote:

>From: Nathan_Bonsal@intuit.com (Nathan Bonsal)

>Date: Tue, 5 Aug 1997 14:39:22 -0700

>Subject: Re[2]: Glass Capacitor

>

>     If I were to use 9X12 pieces of glass as dielectric (temporarily)

>     and 8X11  pieces of foil or flashing, how many plates would it take to

>     get to .01 mfd for my cap?

>     I do intend to build a rolled cap with polyethelene and a poly stacked

>     plate,  but I want to know how many layers a stacked plate will need.

>     I've got 10 layers done already. I'm hoping 10 will be enough. I could

>     be wrong....

Nathan,

You calculate it the same way as for poly caps except you use

the dielectric constant for glass which is around 7-8.

Yep thats right glass gives you 3.5 times the capacitance

of poly of the same thickness but its very poor RF dissipation

and heavy weight and fragility more than make up for that.

You need to measure the thickness of your glass.

Assuming 1/8 inch thick window glass and a dielectric

constant of 7 your 10 layers of 8 X 11 foil will give you:

0.224 x Dielectric Strength x Area of plate

C = -----------------------------------------  x (Number of plates - 1)

distance between plates in inches

.224 X 7 X 88

C=  -------------- X 9 = 9934.8 pf

.125

Pretty close to .01 mfd.

This cap will generate lots of corona and heat so putting it in oil

is a must. And expect breakdowns. The heating from the poor RF

qualities of the glass will cause the glass to crack. Keep

run times short.

By a roll of poly and replace this ASAP. You wont regret it.

>     Also what if you parallel a stacked plate with a rolled cap of the

>     same value? Because of the inductance of the rolled cap, would it

>     perform differently than two rolled or two stacked?

You can parallel rolled and plate caps. The inductance of rolled

caps isnt too bad unless you try to make very long rolls.

>

>     Just curious. I guess those experiments are actually the reason we

>     build these things. On that last one I'll try it myself.

You got that right. I dont build em just for the sparks.

I enjoy the construction and building part of it. And figuring

out how to make it work better.

>

>     I'm also interested in what ages everybody on this list is so I can

>     get an idea what types of people are out there coiling.

>

Hmm. That may almost be classified information.

I'm 39 and plan on staying that age for some time to come. :)

TTYL!   Mike Hammer  mhammer@misslink.net

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: Re[2]: Glass Capacitor

Date: Tue,  5 Aug 1997 23:55:31 GMT

Organization: The Energy Machine Information System 504-734-0526

From: corr <corr@fullnet.net>

Date: Tue, 05 Aug 1997 17:55:31 -0600

Subject: Re: Re[2]: Glass Capacitor

>     I'm also interested in what ages everybody on this list is so I can

>     get an idea what types of people are out there coiling.

17 years old from Enid, OK.  Just completed my first coil yesterday

(although it barely works at all)

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: Re[2]: Glass Capacitor

Date: Wed,  6 Aug 1997 01:03:51 GMT

Organization: The Energy Machine Information System 504-734-0526

From: Corey Edmunds <corey@efni.com>

Date: Tue, 05 Aug 1997 21:03:51 -0400

Subject: Re: Re[2]: Glass Capacitor

At 11:55 PM 8/5/97 GMT, you wrote:

>From: corr <corr@fullnet.net>

>Date: Tue, 05 Aug 1997 17:55:31 -0600

>Subject: Re: Re[2]: Glass Capacitor

>

>>     I'm also interested in what ages everybody on this list is so I can

>>     get an idea what types of people are out there coiling.

>

>17 years old from Enid, OK.  Just completed my first coil yesterday

>(although it barely works at all)

>

15 years old from Sturgeon Falls,  Ontario Canada.  I am currently getting

16" from my TC...

You can check out this page for details on my TC:

http://www.geocities.com/capecanaveral/lab/9888

+------------------------------------------+

|Corey Edmunds                             |

|Email: corey@efni.com                     |

|WWW:   http://www.efni.com/~corey         |

|IRC:  chat.talkcity.com:6667 ~ Bad_Command|

+------------------------------------------+

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Our Age

Date: Wed,  6 Aug 1997 01:42:18 GMT

Organization: The Energy Machine Information System 504-734-0526

From: aasphalt@win.bright.net (Lynn Anderson)

Date: Tue, 5 Aug 1997 20:42:18 -0500 (CDT)

Subject: Our Age

To the 17 year old of Enid. You say your coil hardly works at all. Well I'm

43 and I'v been reading this list and another for months and have not yet

started building my coil. You are way ahead of me. Keep on working at it and

when I get to the building stage I'm going to be asking for YOUR help. Keep

on being curious!

Get your "Tars and Stripes" from Anderson Asphalt!

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: Glass Capacitor

Date: Wed,  6 Aug 1997 02:44:10 GMT

Organization: The Energy Machine Information System 504-734-0526

From: Bert Hickman <bert.hickman@aquila.com>

Date: Tue, 05 Aug 1997 19:44:10 -0700

Subject: Re: Glass Capacitor

Organization: Stoneridge Engineering

<SNIP>

>      I'm also interested in what ages everybody on this list is so I can

>      get an idea what types of people are out there coiling.

>

>      Nathan_Bonsal@Intuit.com

>

<SNIP>

Nathan,

49 and counting...

-- Bert --

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: Our Age

Date: Wed,  6 Aug 1997 06:20:53 GMT

Organization: The Energy Machine Information System 504-734-0526

From: corr <corr@fullnet.net>

Date: Wed, 06 Aug 1997 00:20:53 -0600

Subject: Re: Our Age

Ok, but when you do, don't do as I did and get cheap and make a single

spark gap.  BTW - names Jeff and goodlucok with your first coil

> To the 17 year old of Enid. You say your coil hardly works at all. Well I'm

>43 and I'v been reading this list and another for months and have not yet

>started building my coil. You are way ahead of me. Keep on working at it and

>when I get to the building stage I'm going to be asking for YOUR help. Keep

>on being curious!

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: flat plate vs rolled caps (LDPE k question)

Cc:

Bcc:

X-Attachments:

At 09:58 PM 8/5/97 GMT, you wrote:

>> >

>>  For example a flat plate cap with 2 plates of 10 X 10 inches

>>  with 24 mils of poly (K=2) is equal to:

>>

>>    0.224 X 2 X 100

>>  ------------------  X 1 =  1866.67 pf

>>        .024

>

><snippus gigantus>

>

>Hi Mike,

>

>How universally accepted is the use of a k of 2 for LDPE rather than 2.3?

>Have you seen this on numerous occations, after building from

>calculations and comparing to the finished product? Are you sure the

>other variables were eliminated?

The use of 2.0 for the K of poly compensates for our less

than perfect building techniques.

I've built a bunch of plate caps and 3 rolled caps now.

And compared calculated results against a digital LCR meter.

My plate caps have come out a little lower in actual capacitance

and my rolled caps a little higher than the calculated values.

These were calculated using 2.0 for the K value of poly.

I expect the reason for this is the tightness of the plates against

the layers of dielectric. Hard to get flat plates nice and tight

without building them in some sort of compression frame.

Rolled caps can be rolled tighter. In fact I wind mine on

a center spindle and put a wrench on that spindle to crank the

cap down tight. This tightness will get you closer to the real

value of 2.3 for poly.

I continue to use 2.0 for the K value as it seems to work out

pretty close. Great thing about plate caps is if you come up

a little short on capacitance you can throw a couple more plates

on the stack.

>

>I am asking this because I am poised to begin on a flat Capacitor. I do

>not want to be too far off of my final value of .01.  I will be radiusing

>my plate corners tonight, then cutting up that magnificent roll of 8 mil

>i got, and the almost equally groovy big roll of kraft paper.

>

8 mil poly is just great. 8 layers for 64 mils will make

a great plate cap. What size plates you thinking about?

I would only consider going to the trouble of adding the

kraft paper for large plate sizes. The air will come out

of plates smaller than about 7 X 7 fairly well given

a moderate break in period. Those large plates like my

last plate cap with 10 X 10 plates really trap the air

and the kraft paper idea would really help here.

TTYL!   Mike Hammer  mhammer@misslink.net

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: Glass Capacitor

Date: Wed,  6 Aug 1997 11:18:32 GMT

Organization: The Energy Machine Information System 504-734-0526

From: Steve Falco <sfalco@worldnet.att.net>

Date: Wed, 06 Aug 1997 07:18:32 -0400

Subject: Re: Glass Capacitor

> >      I'm also interested in what ages everybody on this list is so I can

> >      get an idea what types of people are out there coiling.

Currently, I'm 41.  Since Doug Adams says the meaning of life, the

universe, and everything is 42, I expect my coil to work better next

year.

Steve Falco

sfalco@worldnet.att.net

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: Glass Capacitor -Reply

Date: Wed,  6 Aug 1997 12:51:22 GMT

Organization: The Energy Machine Information System 504-734-0526

mhammer@misslink.net

From: Kevin Batchelor <KBATCHELOR@arslimited.com>

Date: Wed, 06 Aug 1997 08:51:22 -0400

Subject: Re: Glass Capacitor -Reply

Age:  29

Currently have been running my first coil for about 6 months.  I am

working on enhancing my current coil with a larger plate cap and larger

secondary.  My longest sparks to date are close to 20" with my 15Kv

60ma neon transformer.  Secondary is 26 guage wire for about 1000

turns for 17" on a 4.5" PVC form.  Primary is 1/4" copper tubing.  Cap is a

plate cap of three in series, total of 180 plates (60 in each cap), with 5

layers of 4mil poly, submerged in mineral oil.  RQ type spark gap with 8

gaps air cooled by fan.  Power is controlled through the use of a General

Radio Variac rated at 10 amps.

I plan to rebuild my Neon Transformer soon since it now only has output

from one terminal.  Anyone who has experience unpotting and rebuilding

a Transco transformer advice would be appreciated.

Here's the real question:  Any female coilers out there?

Kevin Batchelor

KBATCHELOR@ARSLIMITED.COM

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: flat plate vs rolled caps (LDPE k question)

Date: Wed,  6 Aug 1997 13:29:15 GMT

Organization: The Energy Machine Information System 504-734-0526

From: "Thomas McGahee" <tom_mcgahee@sigmais.com>

Date: Wed, 6 Aug 1997 09:29:15 -0400

Subject: Re: flat plate vs rolled caps (LDPE k question)

----------

> From: Tesla-2 <tesla-2@emachine.com>

> To: tom_mcgahee@sigmais.com

> Subject: Re: flat plate vs rolled caps (LDPE k question)

> Date: Wednesday, August 06, 1997 2:50 AM

>

> From: Mike Hammer <mhammer@misslink.net>

> Date: Wed, 06 Aug 1997 01:50:47 -0500

> Subject: Re: flat plate vs rolled caps (LDPE k question)

>

> At 09:58 PM 8/5/97 GMT, you wrote:

>

> >> >

> >>  For example a flat plate cap with 2 plates of 10 X 10 inches

> >>  with 24 mils of poly (K=2) is equal to:

> >>

> >>    0.224 X 2 X 100

> >>  ------------------  X 1 =  1866.67 pf

> >>        .024

> >

> ><snippus gigantus>

> >

> >Hi Mike,

> >

> >How universally accepted is the use of a k of 2 for LDPE rather than 2.3?

> >Have you seen this on numerous occations, after building from

> >calculations and comparing to the finished product? Are you sure the

> >other variables were eliminated?

>

>

> The use of 2.0 for the K of poly compensates for our less

> than perfect building techniques.

>

> I've building a bunch of plate caps and 3 rolled caps now.

> And compared calculated results against a digital LCR meter.

> My plate caps have come out a little lower in actual capacitance

> and my rolled caps a little higher than the calaculated values.

> These were calculated using 2.0 for the K value of poly.

>

> I expect the reason for this is the tightness of the plates against

> the layers of dielectric. Hard to get flat plates nice and tight

> without building them in some sort of compression frame.

>

With a compression frame I get a value of 2.2 for the Polypropylene

that I used. Note also that the plate area referred to in the formula

does *NOT* refer to the size of the plates, but the ACTIVE AREA ONLY.

In other words, only the area of the plate that actually faces

another plate should be included in the formula, *NOT* the area of the entire

metal plate. Forget this basic fact and you are guaranteed to end up

with an actual value much smaller than your calculated one!

Also keep in mind that there are different grades of poly out there.

> Rolled caps can be rolled tighter. In fact I wind mine on

> a center spindle and put a wrench on that spindle to crank the

> cap down tight. This tightness will get you closer to the real

> value of 2.3 for poly.

>

We coilers are used to making "round" rolled caps. You can also make

"flat" rolled caps by "rolling" the assembly around a flat core made

from a thin sheet of plexiglass or even dumb old fiberboard (round

the edges!). You can use a flat board to apply lots of pressure each

time you "flip" the unit over during construction. In this way it can

be made quite tight!

This "flat rolled" version will then fit well inside of a rectangular

container. This is an option that I have not seen too many coilers

use, but it *IS* used in many commercial caps such as radar caps.

Such "flat rolled" caps can be easily stacked into series

arrangements all within a single container. In the TCBOR tape report #50 Richard

Hull disassembled a large military surplus radar pulse capacitor,

and you can see that the method of construction used was 12

"flat rolled" caps connected in series (for a voltage rating of 125KV

DC!!)

> I continue to use 2.0 for the K value as it seems to work out

> pretty close. Great thing about plate caps is if you come up

> a little short on capacitance you can throw a couple more plates

> on the stack.

>

>

> >I am asking this because I am poised to begin on a flat Capacitor. I do

> >not want to be too far off of my final value of .01.  I will be radiusing

> >my plate corners tonight, then cutting up that magnificent roll of 8 mil

> >I got, and the almost equally groovy big roll of kraft paper.

> >

>

> 8 mil poly is just great. 8 layers for 64 mils will make

> a great plate cap. What size plates you thinking about?

> I would only consider going to the trouble of adding the

> kraft paper for large plate sizes. The air will come out

> of plates smaller than about 7 X 7 fairly well given

> a moderate break in period. Thos large plates like my

> last plate cap with 10 X 10 plates really trap the air

> and the kraft paper idea would really help here.

>

> TTYL!   Mike Hammer  mhammer@misslink.net

> Mike Hammer

> mhammer@misslink.net

>

Hope that the extra comments I added to this thread are helpful in

some way to somebody out there!

Fr. Tom McGahee

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: Our Age

Date: Wed,  6 Aug 1997 13:33:16 GMT

Organization: The Energy Machine Information System 504-734-0526

From: "Thomas McGahee" <tom_mcgahee@sigmais.com>

Date: Wed, 6 Aug 1997 09:33:16 -0400

Subject: Re: Our Age

Well, I am probably the "old fogey" on this list, with an age of 50.

However, age is relative. I keep myself young in heart and mind by

having an intense curiousity about things. My interests include not

only Tesla coils, but also x-rays, particle accelerators, Van de

Graaf Generators, anything high voltage, anything scientific, and

also art, especially calligraphy. I am happiest when 1) I am *with*

people that I love, 2) when I am sharing people who have like

interests, and 3) when I am just plain tinkering around with stuff.

Numbers 2 and 3 explain why I "hang out" on the Tesla and Tesla-2

lists so much.

Fr. Tom McGahee

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: flat plate vs rolled caps (LDPE k question)

Date: Wed,  6 Aug 1997 15:08:21 GMT

Organization: The Energy Machine Information System 504-734-0526

From: "Michael Smith" <md.smith@usa.net>

Date: Wed, 6 Aug 1997 10:08:21 -0500

Subject: Re: flat plate vs rolled caps (LDPE k question)

----------

> From: Tesla-2 <tesla-2@emachine.com>

> To: md.smith@usa.net

> Subject: Re: flat plate vs rolled caps (LDPE k question)

> Date: Wednesday, August 06, 1997 1:50 AM

>

> From: Mike Hammer <mhammer@misslink.net>

> Date: Wed, 06 Aug 1997 01:50:47 -0500

> Subject: Re: flat plate vs rolled caps (LDPE k question)

>

> At 09:58 PM 8/5/97 GMT, you wrote:

>

> >> >

> >>  For example a flat plate cap with 2 plates of 10 X 10 inches

> >>  with 24 mils of poly (K=2) is equal to:

> >>

> >>    0.224 X 2 X 100

> >>  ------------------  X 1 =  1866.67 pf

> >>        .024

> >

> ><snippus gigantus>

> >

> >Hi Mike,

> >

> >How universally accepted is the use of a k of 2 for LDPE rather than

2.3?

> >Have you seen this on numerous occations, after building from

> >calculations and comparing to the finished product? Are you sure the

> >other variables were eliminated?

>

>

> The use of 2.0 for the K of poly compensates for our less

> than perfect building techniques.

>

> I've building a bunch of plate caps and 3 rolled caps now.

> And compared calculated results against a digital LCR meter.

> My plate caps have come out a little lower in actual capacitance

> and my rolled caps a little higher than the calaculated values.

> These were calculated using 2.0 for the K value of poly.

>

> I expect the reason for this is the tightness of the plates against

> the layers of dielectric. Hard to get flat plates nice and tight

> without building them in some sort of compression frame.

So, it this to say that you have seen 2.3 be real for k when the plates

on your cap were tight? I have a compression frame.

>

> Rolled caps can be rolled tighter. In fact I wind mine on

> a center spindle and put a wrench on that spindle to crank the

> cap down tight. This tightness will get you closer to the real

> value of 2.3 for poly.

>

> I continue to use 2.0 for the K value as it seems to work out

> pretty close. Great thing about plate caps is if you come up

> a little short on capacitance you can throw a couple more plates

> on the stack.

>

>

> >

> >I am asking this because I am poised to begin on a flat Capacitor. I do

> >not want to be too far off of my final value of .01.  I will be radiusing

> >my plate corners tonight, then cutting up that magnificent roll of 8 mil

> >I got, and the almost equally groovy big roll of kraft paper.

> >

>

> 8 mil poly is just great. 8 layers for 64 mils will make

> a great plate cap. What size plates you thinking about?

> I would only consider going to the trouble of adding the

> kraft paper for large plate sizes. The air will come out

> of plates smaller than about 7 X 7 fairly well given

> a moderate break in period. Thos large plates like my

> last plate cap with 10 X 10 plates really trap the air

> and the kraft paper idea would really help here.

I was not going to completely outline the whole cap until it was done,

but for your curiousity i will give the major improtant specs. Plates are

flashing, 16x13 inch with 2x3 tabs. Tab positions alternate between the

plate corner and the center, on the 16 inch side. When I reach .02, the

center tab wil continue as in the first cap, and the other plate becomes

located on the opposite corner. I will bring out all three connections to

the outside, having two .02's in series.

I will use 6 layers of 8 mil poly and between each element there will be

#30 kraft paper. This is 7 layers of kraft paper @ 3mils for 21 mils.

With the poly, interplate distance is .069. It takes 15 layers to get

.02. The whole thing will be on a compression frame, having 1 inch square

steel tubes and ready bolt for the clamp. I will put in in a box of

plywood, lined with fiberglass cloth and polyester resin to contain the

texaco 0600 xfrmr oil. I intend to put 1x2 inch by .069 spacers between

the plates at the ends of the tabs with a bolt up through the center to

maintain plate spacing while enabling a very tight, high surface area

connection with no stress on the plates. The box top will probably be all

plexiglass. the connection bolts will definately be on plexiglass, even

if I decide to make the top of wood.

I will put some Schrader valve fittings on the box to make it easy to

drain the oil when want to. I'll make a pump for that.

So far, I have sheared the plates to 16x16. Next i am going to build a

jig to hold the plates so I can use the milling machine to mill off the

edges of all plates at once, leaving the tabs. I made a tool yesterday

from a piece of 1 7/8 stainless thinwall (.040) pipe to punch the radius

on the plate corners. I tested it and it works great, only very light

sanding with 600 grit will give smooth, no burr edges.The poly and paper

are 2" past the plates everywhere except the tab side, where it is 3"

overlap, coming up to the last inch, which is the tab connection.

I am trying to incorporate as many good ideas gleaned from the tesla list

as I can. And in keeping with that, I will be building up the center

thickness of the compression frame inner plates to squish out the

bubbles, which there won't be many of, since I will build the whole thing

under oil and use the roller I just made to as needed along the way.

comments welcomed.

Michael Smith

BTW, every time I get on #tesla, someone tells me I just missed you. If

anyone is around and wants to see me I can be found alot on DALnet,

#TMIVoyagers.

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: flat plate vs rolled caps (LDPE k question)

Cc:

Bcc:

X-Attachments:

At 03:08 PM 8/6/97 GMT, you wrote:

>From: "Michael Smith" <md.smith@usa.net>

>Date: Wed, 6 Aug 1997 10:08:21 -0500

>Subject: Re: flat plate vs rolled caps (LDPE k question)

>

<SNIP>

>>

>>

>> The use of 2.0 for the K of poly compensates for our less

>> than perfect building techniques.

>>

>> I've building a bunch of plate caps and 3 rolled caps now.

>> And compared calculated results against a digital LCR meter.

>> My plate caps have come out a little lower in actual capacitance

>> and my rolled caps a little higher than the calaculated values.

>> These were calculated using 2.0 for the K value of poly.

>>

>> I expect the reason for this is the tightness of the plates against

>> the layers of dielectric. Hard to get flat plates nice and tight

>> without building them in some sort of compression frame.

>

>So, it this to say that you have seen 2.3 be real for k when the plates

>on your cap were tight? I have a compression frame.

No none of my caps come close to realizing a K of 2.3.

I just get a little more from tightly rolled roll caps.

I use a nominal value of 2.0 for all calculations.

Also bear in mind that the poly we buy is NOT capacitor

grade material and its actual K value can vary.

<MORE SNIP>

>>

>> 8 mil poly is just great. 8 layers for 64 mils will make

>> a great plate cap. What size plates you thinking about?

>> I would only consider going to the trouble of adding the

>> kraft paper for large plate sizes. The air will come out

>> of plates smaller than about 7 X 7 fairly well given

>> a moderate break in period. Thos large plates like my

>> last plate cap with 10 X 10 plates really trap the air

>> and the kraft paper idea would really help here.

>

>I was not going to completely outline the whole cap until it was done,

>but for your curiousity i will give the major improtant specs. Plates are

>flashing, 16x13 inch with 2x3 tabs. Tab positions alternate between the

>plate corner and the center, on the 16 inch side. When I reach .02, the

>center tab wil continue as in the first cap, and the other plate becomes

>located on the opposite corner. I will bring out all three connections to

>the outside, having two .02's in series.

>

>I will use 6 layers of 8 mil poly and between each element there will be

>#30 kraft paper. This is 7 layers of kraft paper @ 3mils for 21 mils.

>With the poly, interplate distance is .069. It takes 15 layers to get

>.02. The whole thing will be on a compression frame, having 1 inch square

>steel tubes and ready bolt for the clamp. I will put in in a box of

>plywood, lined with fiberglass cloth and polyester resin to contain the

>texaco 0600 xfrmr oil. I intend to put 1x2 inch by .069 spacers between

>the plates at the ends of the tabs with a bolt up through the center to

>maintain plate spacing while enabling a very tight, high surface area

>connection with no stress on the plates. The box top will probably be all

>plexiglass. the connection bolts will definately be on plexiglass, even

>if I decide to make the top of wood.

>

>I will put some Schrader valve fittings on the box to make it easy to

>drain the oil when want to. I'll make a pump for that.

>

>So far, I have sheared the plates to 16x16. Next i am going to build a

>jig to hold the plates so I can use the milling machine to mill off the

>edges of all plates at once, leaving the tabs. I made a tool yesterday

>from a piece of 1 7/8 stainless thinwall (.040) pipe to punch the radius

>on the plate corners. I tested it and it works great, only very light

>sanding with 600 grit will give smooth, no burr edges.The poly and paper

>are 2" past the plates everywhere except the tab side, where it is 3"

>overlap, coming up to the last inch, which is the tab connection.

>

>I am trying to incorporate as many good ideas gleaned from the tesla list

>as I can. And in keeping with that, I will be building up the center

>thickness of the compression frame inner plates to squish out the

>bubbles, which there won't be many of, since I will build the whole thing

>under oil and use the roller I just made to as needed along the way.

>

Wow! Sounds like a real winner for a cap. Well thought

out. Yes do use that paper for those large size plates.

Building it under oil is a great way to minimize air

bubbles but it is a really messy job.

I built my 10 X 10 inch plate cap under oil and had oil everywhere.

Had it up to my elbows, on the table, on the floor , in my hair,

on the dog, etc.

But I fired up the cap right after construction and it performs just fine.

>

>Michael Smith

>

>BTW, every time I get on #tesla, someone tells me I just missed you. If

>anyone is around and wants to see me I can be found alot on DALnet,

>#TMIVoyagers.

Best time to find me on the #Tesla IRC channel is 7-9 PM CDT.

I am often also on after 11 PM CDT. Also occasionally on a bit

earlier like 5-6 PM CDT.

TTYL!   Mike Hammer  mhammer@misslink.net

From: tesla-2@emachine.com (Tesla-2)

Subject: Microwave transformers

Date: Wed,  6 Aug 1997 19:42:18 GMT

Organization: The Energy Machine Information System 504-734-0526

From: watkier@earlham.edu

Date: Wed, 06 Aug 1997 14:42:18 EST

Subject: Microwave transformers

Has anyone tried using microwave transformers as a tesla coil power

supply?  I'm still not experienced enough to know if this is worth

pursuing or if I should stick to the tried and true path of neon sign

transformers and ignition coils.  Currently I have two microwave

transformers wired together and this supply is capable of jumping a modest

spark gap.  I would guess its ouput is around 4kv 250-500ma AC.  Any

thoughts on how this might perform as the HV power supply for a tesla coil??

-Erik

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: Our Age

Date: Wed,  6 Aug 1997 19:48:00 GMT

Organization: The Energy Machine Information System 504-734-0526

From: "Michael Smith" <md.smith@usa.net>

Date: Wed, 6 Aug 1997 14:48:00 -0500

Subject: Re: Our Age

----------

> From: Tesla-2 <tesla-2@emachine.com>

> To: md.smith@usa.net

> Subject: Re: Our Age

> Date: Wednesday, August 06, 1997 8:33 AM

>

> From: "Thomas McGahee" <tom_mcgahee@sigmais.com>

> Date: Wed, 6 Aug 1997 09:33:16 -0400

> Subject: Re: Our Age

>

>

>

> Well, I am probably the "old fogey" on this list, with an age of 50.

Ah HA! A geezer. I KNEW it.

> However, age is relative. I keep myself young in heart and mind by

> having an intense curiousity about things.

Uh huh, now the excuses....."I don't FEEL old...."

> My interests include not

> only Tesla coils, but also x-rays, particle accelerators, Van de

> Graaf Generators, anything high voltage, anything scientific, and

> also art, especially calligraphy.

Reanimation of dead tissue, perhaps....?

I am happiest when 1) I am *with*

> people that I love, 2) when I am sharing people who have like

> interests, and 3) when I am just plain tinkering around with stuff.

Which people? The ones you dug up and reassembled?

> Numbers 2 and 3 explain why I "hang out" on the Tesla and Tesla-2

> lists so much.

> Fr. Tom McGahee

#2? Mmmm, hopefully not scatalogical humor. :)  Tinkering I can

definately relate to.

Woo Hoo, feeling mean today.

Michael Smith in Lincoln, NE

Aged 40 years.....*NOT* a geezer.

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: Glass Capacitor

Date: Wed,  6 Aug 1997 21:46:28 GMT

Organization: The Energy Machine Information System 504-734-0526

From: JAMES <057006@bud.cc.swin.edu.au>

Date: Wed, 06 Aug 1997 14:46:28 -0700

Subject: Re: Glass Capacitor

Organization: 057006

Tesla-2 wrote:

>

> From: Nathan_Bonsal@intuit.com (Nathan Bonsal)

> Date: Tue, 5 Aug 1997 14:39:22 -0700

> Subject: Re[2]: Glass Capacitor

>

>      I'm also interested in what ages everybody on this list is so I can

>      get an idea what types of people are out there coiling.

>

>      Nathan_Bonsal@Intuit.com

>

I'm 24 and though I had my coil working about a month ago, it was a very

temporary setup (primary wrapped around a plastic plant pot & secondary

taped upright) now I've made a frame for the primary, and it's a lot

closer to the secondary, the sparks are getting longer but I'm not

expecting much until I can find a neon transformer(I'm currently using

an ignition coil).

James Hurl, Australia.

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: Our Age

Date: Thu,  7 Aug 1997 04:27:36 GMT

Organization: The Energy Machine Information System 504-734-0526

From: Shane Randle <randlec@scnc.addison.k12.mi.us>

Date: Thu, 07 Aug 1997 00:27:36 -0400

Subject: Re: Our Age

Tesla-2 wrote:

>

> From: "Michael Smith" <md.smith@usa.net>

> Date: Wed, 6 Aug 1997 14:48:00 -0500

> Subject: Re: Our Age

>

> ----------

> > From: Tesla-2 <tesla-2@emachine.com>

> > To: md.smith@usa.net

> > Subject: Re: Our Age

> > Date: Wednesday, August 06, 1997 8:33 AM

> >

> > From: "Thomas McGahee" <tom_mcgahee@sigmais.com>

> > Date: Wed, 6 Aug 1997 09:33:16 -0400

> > Subject: Re: Our Age

> >

> >

> >

> > Well, I am probably the "old fogey" on this list, with an age of 50.

>

> Ah HA! A geezer. I KNEW it.

>

> > However, age is relative. I keep myself young in heart and mind by

> > having an intense curiousity about things.

>

> Uh huh, now the excuses....."I don't FEEL old...."

>

> > My interests include not

> > only Tesla coils, but also x-rays, particle accelerators, Van de

> > Graaf Generators, anything high voltage, anything scientific, and

> > also art, especially calligraphy.

>

I must be the youngest one here. I usually tell everyone I'm 17, but to

be precise, I'm 16 years, 5 months and 8 days old today. I think I share

about all of the interests listed above (including an intense curiosity

about everything) as does my father, who's going on 42. I don't feel

particularly young, (I've had my drivers' license for a week, and I

missed the Carter Administration by two months) but I suppose age in

that sense is in the eye of the beholder.

Older coilers have the benefit of experience and knowledge, while young

people learn from their experiences so that when they're older, they

have the extra experience of having started at an early age.

My basic opinion is that age is totally independent of intelligence and

maturity, so if you can accomplish at age 20 what others do at 50, then

by all means, go for it! My goal is to finish a working coil before

starting eleventh grade (which gives me about 4 weeks) :).

-Shane Randle

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: Microwave transformers

Date: Thu,  7 Aug 1997 04:36:54 GMT

Organization: The Energy Machine Information System 504-734-0526

From: Shane Randle <randlec@scnc.addison.k12.mi.us>

Date: Thu, 07 Aug 1997 00:36:54 -0400

Subject: Re: Microwave transformers

Tesla-2 wrote:

>

> From: watkier@earlham.edu

> Date: Wed, 06 Aug 1997 14:42:18 EST

> Subject: Microwave transformers

>

> Has anyone tried using microwave transformers as a tesla coil power

> supply?  I'm still not experienced enough to know if this is worth

> pursuing or  if I should stick to the tried and true path of neon sign

> transformers and ignition coils.  Currently I have two microwave

> transformers wired together and this supply is capable of jumping a modest

> spark gap.  I would guess its ouput is around 4kv 250-500ma AC.  Any

> thoughts on how this might perform as the HV power supply for a tesla coil??

>

> -Erik

>

I'm trying (keyword: "trying") to power my small Tesla coil with only a

single microwave transformer to see if it will work, but it only puts

out about 2200VAC. I'm not done testing it yet, so I don't have any

definite results, but I'll let you know how it works.

Right now, though, I'm thinking about building a plate capacitor to

replace my salt water cap. I actually have two microwave transformers,

and if the one doesn't work, I'll probably wire them together like you

did. As for experience, I doubt I have much more than you, so I can't

really offer any expert advice yet. I should be able to finish testing

it in the next few days, so I'll let you know. :)

-Shane Randle

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: What I did on Vacation!

Date: Thu,  7 Aug 1997 05:46:39 GMT

Organization: The Energy Machine Information System 504-734-0526

Hi folks!

I though you might enjoy the attached JPG taken at the International

Tesla Society Museum last month when I attended the annual

Extraordinary Science Conference.  (That's me on the right.)

Happy Zappin'!

Ralph

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

*** NOTE: The following is a UUEncoded file ***

begin 0777 RALPHP1.JPG

M_]C_X``02D9)1@`!`0$`20!)``#_VP!#``@&!@<&!0@'!P<)"0@*#!0-#`L+

M#!D2$P\4'1H?'AT:'!P@)"XG("(L(QP<*#<I+#`Q-#0T'R<Y/3@R/"XS-#+_

MVP!#`0D)"0P+#!@-#1@R(1PA,C(R,C(R,C(R,C(R,C(R,C(R,C(R,C(R,C(R

M,C(R,C(R,C(R,C(R,C(R,C(R,C(R,C+_P``1"`%R`.0#`2(``A$!`Q$!_\0`

M'P```04!`0$!`0$```````````$"`P0%!@<("0H+_\0`M1```@$#`P($`P4%

M!`0```%]`0(#``01!1(A,4$&$U%A!R)Q%#*!D:$((T*QP152T?`D,V)R@@D*

M%A<8&1HE)B<H*2HT-38W.#DZ0T1%1D=(24I35%565UA96F-D969G:&EJ<W1U

M=G=X>7J#A(6&AXB)BI*3E)66EYB9FJ*CI*6FIZBIJK*SM+6VM[BYNL+#Q,7&

MQ\C)RM+3U-76U]C9VN'BX^3EYN?HZ>KQ\O/T]?;W^/GZ_\0`'P$``P$!`0$!

M`0$!`0````````$"`P0%!@<("0H+_\0`M1$``@$"!`0#!`<%!`0``0)W``$"

M`Q$$!2$Q!A)!40=A<1,B,H$(%$*1H;'!"2,S4O`58G+1"A8D-.$E\1<8&1HF

M)R@I*C4V-S@Y.D-$149'2$E*4U155E=865IC9&5F9VAI:G-T=79W>'EZ@H.$

MA8:'B(F*DI.4E9:7F)F:HJ.DI::GJ*FJLK.TM;:WN+FZPL/$Q<;'R,G*TM/4

MU=;7V-G:XN/DY>;GZ.GJ\O/T]?;W^/GZ_]H`#`,!``(1`Q$`/P#T*_O=(U73

MC;B=U92&BD,#Y5QT/3\/<$BH]'N?,C\E_OIT]O\`]54XK=EZ7,X^C4MQ;O`1

M<K-,QR`Y+?-CV/Z5SNZU+78Z5#TSUJPAXK'ALTDC5Q=7+`C(/F&KD=H@/^NG

MQ_UU-#OT&::GMWJ0?E5!+6,<F2;`[F4U,+6+CF3_`+[;_&C45BX#3OK5-;6/

M.?F/_`S3Q;0\9!_[Z-"`N`]/>E!JK]FA_NGG_:-*+:'^Z?S-%V&A;S2]JKBV

MA_N_K3A;Q?W/UHU%H3TN:A$$7]P4\11_W13U$/R,<&E!'J*9Y:==@_*E\I#_

M``C\J>H#MP]1^=&X?WA^=-\M/[H_*CRT_NC\J6H#MR_WA^=)O7^\/SI/+3^Z

MOY4>6G]U?RH]X-`,B_WQ^=(TJ*"2P_.@QI_=7\JCF1%B8A5!^E2W))C21':_

M)(\F&.X*N!VQG_&K#2!F4['.TYZ>QJ*.)/*7*@DC-.\F/&-M3'F2L-VN2></

M[C_E3(Y50$;'ZD]/4YIIAC]*;Y*?W?UI\T]Q:$C3H75=K%A\V,?AFG&Y4#.Q

M_P`JKB!.<`XSZ]*#$G<<_6GSS'9$B3J@;$,G+$G@<_K14'D)Z'\Z*7M)(+(X

MV(X'(ZFK*@2)L<9!!!JI&>Y.<?I5N,^GWOK^M:6%<--F>*9[*1LE>5/J*V$(

MQQ]*PKU&0)<Q?ZR+D^Z_Y_K6K;3K/"LB-PPR,4EV*+T9&>1WJ7/!JLAYQGBI

M`2>N:!$X:GBH`13QTS2`F!SUIP(&:B!S3@>O-`B8'TI0W2H@13@>,^U,"8&G

M5&#^5.R*$(=FES3"<T;J=P'YHS3<TF>?I0V`_-&::3TI":5PL./2J]V^(QCJ

M34I-5;D[IX4Z\YQ43>A26I:4;5"^@I<BFDTF3WH$.)IO3O2%NU-)YI#'9&,4

MT\<4A;CI3"?>AL!Q;FBHR3ZXHI<UM`T.(B;'R\D&K<38QP?K6>D@&.<G-7$;

M/?GWKI2$7E92,$9!ZYJ"Q;['>/:$81_GC/\`,4^,DX`IM[`981)'Q+$=RD4-

M,$S84]_YU(K=JH6EP+BVCE7'S=O0]ZM*>E'+=#N6`V0>>E/#`57!IZG`Z]:E

MQ`L`X/6EW8%0Y^E.!J6!*CEER5VGT)IX)_6H<T\,/6ET`G##CU%.#"H`1TI^

M[BF(E!R*,C'6H]W6C=VHW`EW=>]-W\BF%O>DW#UJ0L2[N<T$]ZC!_"DR<4`2

M%O6JFX/J./[B?Y_G4V>1Z^U4[1B]Q<R$<;]H/T_R*EWV*1?W4%O>H]U&?6@!

MY/--+')'2FD\4PM2`>6IA8]*87'K32]`#]^/2BH=_J:*5F.QY_&ET?\`EO#C

M_KD?_BJNQQ77'^DH/]V/_P"O56%SUJ_"_JP`KL42+EB."X!&;IO3B->/TJ]#

M;2$<W,I_X"O^%5HI/<^]:4$@QDUO"">Y+91AL9[:_P#)$TJP3?,I&/O>AX_E

M[5K-92%%4RR)M!^8$9/UXJ]:2(Y0%5+*1MS5NYFC`9<#<.*IQ2?+8+F";%QU

MNKC_`+Z'^%*MF>AN;@_\#'^%7'=3U(^HJ+(R>:B5)=AIL@%IS_Q\W&/=Z?\`

M90,'SY\?]=#3MWTQ3LU@X+H.X@MA_P`]9C[^8:D%LH/^LF_[^'_&FA^?>GA\

M=ZR:0[CA;I_STE_[^-_C3A`O]^7_`+^-_C2!QG.12[P*5@N`MT[O+_W\;_&E

M^SQ_WI/^_C?XT@<4X/D<T63`3R$_O2?]_6_QH\A.FZ3C_IHW^-(9/?%&_D\\

MGM1R@.%O'_>DY_Z:-_C2?98\=9#_`-M6_P`:<&]34J8/6KC2N*Y7>&-5W$O\

MH)X<_P"-4=-@$MKYDK2$N2?E=AQ[X-;<ENCVT@+X&TY/IQ5.&-(+6-%Z8S5.

MBF[C4M!GV6,'[TO_`'^?_&D%K&/XI\?]=G_QJ0GFFE_J:Q<$.Y&UO'C&Z?\`

M[_/_`(U']FC'\<^<?\]G_P`:F9N.E1EQ4V2&,-K'VDGQ_P!=F_QJ,VR8_P!;

M.?\`MJW^-2,^/6FLYSZ<TK`1?9T7_EI.?^VK?XT4-(<_*W'UHJK+R#7H<+%*

M0<^^,FK\,Q^7D?E7.QZE9KPUU"/8N*N1:M9?\_D'XR+74F0TSJ+0>>RIN(+'

M`XJX_P#H\KH)%;:<9'>L+2M3M)-0@6&[@:0N-BB0'YNWZXJQ-=Q6A`N9HXR1

MN`<X)'8UNFK$M'06-QFZ@7./G';WI-:NS9:M;79RMO=CRFS_``R*<#_"L&WU

M.UFG2.*YA,CL`N'`Y[5=U&_L-:TB^@6=&$&V17`/`X4D#Z[:3GU3'%=#3$Q.

M1V[4HE!ZUS.C:M]LMDB?>URA\MMB$Y(X'3N?YUH'4H@2"LP(."/(?C]*AU+J

MX<K1K>8._P!*4.,=?UK(&J6YQQ<<>EM)_A3TU.!OX;GC_IVD_P#B:S;0[,U@

MXYYI^_K6<+V-5WF.XVGH?L[_`.%*NIPG^"YQUYMW_P`*ECLS4#\<Y-&_\*H#

M4(R/]7<?]^'_`,*>+V,YPLW_`'Z;_"E:[$7`XZ#/X4X/W-4ENTQ]R7_OTW^%

M2+<*3]R7_OV15<O8=RR6Z=0*3>,C/YTV/Y^!')_WS5Y+!V`;E3[U:II;DW*P

M?O\`UIWF<]:)X9(<_NW/TQ_C51YB`?W,OY#_`!I\K6H%G5+AHM$`0D/<2",'

MZG_ZU-\PD8_"L_7Y&^VZ3;*&**#(VWN>/\_C2B[8<>1)[_*/\:SE)W*L72^:

M;YG/N*I?:R2289?^^1_C1]JQQY;Y]UZUDW<:+9D'(/\`*HR_/;^557NQW60?

M\`-1F\&?N2Y_W#S4`7&?'?\`^O41<],<BJC7<9!.V3_OVW^%1/>)@_+*#_UR

M;_"@"[YA]OQ%%9QOH\_=E_[\M_A11H*QS"Z3I<\4-U'860G3)^S'&"/[IX_*

MKXTG2I(HKR"QLGGC7<UN%4`Y'W6XZCW':G1N)':\L6;)8),63!('7CKD?3VJ

MRC%"+RT:402G=-\@W,!GD#KGVQ3=RKDL&G:8'@O;6RLGDBP7C1%`YZC@=1V^

ME;&K:?;&[%REI!<20QK&8R@(QM''U`Z5G0S+8W"7J^:+-R)94"*<YQAO7Z_G

MZUIW)FL=2^99?(F"`JH7B0X&[/IU'U^M:+X17(?L%G')%J%E#:,4X5TC`7;D

M9.1Z8S6E;V=@BS:BB0*GD>6SI&`I!8'/T&/PYJI$SVUP82LA@D/[L`+B-N2V

M3UYS_GBM"$BTLUM/FQ,6=`%PJ(,<9^N2/;-..FHKG*:JG]C:[::U;X^QS,JS

MA%(&<_*WITX_#OFK+2,DC@DG#$?K6I+;175K<:9<AGA="5;``5>@4>ZD9''I

M7,0RM"[6D[%KF'*2-S\W)PW/J,'\:(2Z`]C5$ISUXJ5)?F]?QK.64L0N<D=Z

ME$N*NY)L3S@%8D.0G`;U-1B3C_Z]5))-Y1MHP5!X]N/Z4+(<]"10]6!?\S/)

MI1)GW%4PY!')R>:>LC<<^U-*X%Q9!TZBI4<#&>/K6<)CT)IPFP,[N*N,K"9L

MPW`1O>M6VN!/&6[CK7)BY([UHZ9=Y@O(^_E%Q^'_`.NKDXR0DB[<W@9B%((!

MK/DF&<^M47N/FY/`[U$TV3R<>]$IJ*L@19MK\7>JR6,NW;'$"DC'[K'M^O7Z

MCN:)"8V9&&&5B#[&L'39_-O]1N.F9?*&.VT8K>U"52\$@)R\*DYS\QZ9_2N>

M]T:,9OQV)^IIHD&3S58R<]3UI/-[`GIT%9-@3M)[?G3#+CH,>]5WE[9J-I=O

MMW`J0++2?+UZ^IJ!G![D?C43R_+R!G/-0-+@9!Q4L98,H#'=U^M%4FF&>HHI

M:@<6GB2_-RLPMH%&""A)VGWQGJ*L1:_J*322)%:!9%'R$$@'U`SQFN=64KP,

MX.*M+/M'0#M72X(1NV^N:O"7VBSVN=Q!&<<=O:MC5M>U=[IO)-I]BE7S(08Q

MG:Q/!XY(.03W(-<JDV/QK5CD\_22_)EM6'/)S&V?R`;]7IJ*:L.YJZ3J6J7M

MU::?=2PM;R2)$^`-P7<!G.WK[U:U#7=3GU*;>+;;%/F,$<K@\8..V,5ST%X]

MM<1SQGYE8.I'L:W-?M5$J:O:L6LKTF0<?ZMCR5/XY_+\35DXVL!./$.H/+&S

MK"&C.Y2/7&/3WJAJEY<SR+>,P61!SQD$?0#T_IGI5**;(`)JR6*$#KD<=\]J

MB,>HBU'+<E=RS0G(R"(S_1JE0W;'_7P\_P#3,_\`Q58]K<BUN9+,*2BJ)(@>

M<IT(_`Y'TQ6G;R(;B,.^%!&['8=Z+`:,ANO,5?.A^50"/*;KZ?>]<U)%]KSS

M<0?]^C_\56>;AQ*V\_,3SZYJ59RIZ_6JVW).ETNU6XD$=Q.I)X`2,@_SJ;4;

M*.V'[F=4;<P`9">GXCWK'TJ]2#4!/(?DA^8C/7L/U/Z4S5KU_P"U;M2Q.)64

M?0'`K7G20K#I%N@25N(&!/\`SQ/_`,541:Z/2>')[>4?_BJKK>!2"3\O<'N*

M:UR!G)YS63:>J'8LEKH?\MHO3/EG_P"*K4T$W+RWJM+&VZT?;B,K@\>]8(N,

MN`3P3US6M%-'8::'4EKJ[C8GT2,$CCW..M$=6,J2?:_,_P!?#G)_Y9'_`.*J

M*22Y5"6G@``R3Y1_^*J,S[E!)&2>#FH+KS[FTEM[=2TLB,J*/4C]/K2E>^@(

M9X=2ZN+2+8R[[B0L<H222<=<CTK;U.:>2_E$$L/DH?+C&PGY1P/XN:@%W!I=

MLMCIZ!FB01O='[SX&"5_N@U0,Y."3G'O4.T5;J4R=GNL$":'T/[H_P#Q5(7N

M@?\`7P9[_NC_`/%56$S`GD$#IDTK3#H/_P!=9@2,UUP?.A]?]4?_`(JF,UWV

MEA_&(]/^^JBEN2D+,H+$*2%!QGVIBZM9NNU+2_5LXR\:8&>Y^;I4[#2N2,]V

MQYFAS_UQ/_Q50%[H$XF@]\QG_P"*I'G&.#SW!-0M-E2,_4T`.)NF.?M$`_[8

M_P#V5%0F5?4?]]4478KGGZRL3C:WXXJPDS=`N?QQ6>)`#COWS4XDX'3FNYH1

MI1SMD#9QG.,UJ:-<3#4$"+_K%:-DSRX(((`QU.<?7%8"2<#)XZU.DQ#@Y^;\

MJ2W`TXIG.5E51Z8)//Y5TN@?:-2T^^TEGB(9!-;)YA/[T$`\;<_=+<`'I]*Q

M3(NJVSOL)U"+!=Q(,3KSDXQDN.#UY`)QU-4X)?*EPY*\%<YY'O56L]0-:ZLK

M_3KE(9DAW2#<IWL`>2.Z@CD'M0T\BL$PAV`J?F/7D^GJ:NWDDE[X?T^\?,C6

MDCVTSL<D`D,F>^.6'X5E,@)>6.16CSN'<\^OO3Y=-`8[43/'8PWH5#+"Y$>U

MNJ?Q*?S!%3PW+2012HJE&4$,.XZT>8BVH1P3$6+-@<IG&"/U^OX9&?`PL;@6

M6<QE=\#+T*ECG'TJ9PML"-V24AP<+M91SGOT/ZBG&67?GY!QG(X_SZ_C6?-.

MHG90V0ORC@C./\DTHF_=8)/R],>E96;$:EM/*9@,QH6&`23C/^<5)JTD\6H;

M6`W-'&[Y/.XQJ3^I-9+RY17`X)/]*N7\ZW^G6]_$Q#1*EM<*?[P!VL/8JOYC

MWH:TL,R+C7Y;6\:W-N"5QAM^,_I1+XG+;"L'5>3O]#CT]`*Q=;F=;N.0'.Y-

MNW'<?_K_`$K*^US%3%)\L;'J1]T^O^/_`.JKA34HZB=[G6)XF;S%!MP#G',F

M/Z5TFIZO*++3&>)60V*DL#C/SM@#C'``KR0)=R7OD^62[.5QNP`<_7&*]'N(

MY-3\&Z1?6T#.FG^=9W!0GI@,CX[#D\\<D5M3II/4%<+767O+NWLX;=6GF<*&

M,A"J#WZ?6I=:\5(%&D:5`J1S2+#-=?\`+209&?H#SQFLKPZO]G^?KUT&40*T

M5LA`)><C`&#V4')/;CZ5B622/K,61N1-TAX]N_YU,]%[HUH=GY\V#EDR1CD?

M_7H,TF`24('/0U2\\!@N<8I//^4C(ZY/-<30R^)I6SC;D\YYI&FE5CPOO5))

ML]\A!SQFD%P>>>IS2Y=!EXR.>?ESWXJ(ROR?EX'IS55)_FP"!CWJ,S@O@'"C

MIQ3Y=+@6GE?L$QV^E1-))C`VY/?N*K/<<G`%1^=AN<[N#S4V`L^9,>0RCZFB

MJ3W"@]0/QHIA9G"AS&<%F*'OG[O^?6K*LI7AF/N#5+)SS@"G!G0Y0?+W7/7Z

M5Z$D2:*$'*AF]1\QJ9-IZR,"?]HUGQS$@$$5.LF[G(W#T[U-M0--&"!7CE=7

M!R&$A&/2MR-EU>-U*;-0`R&23"SD=0RYQN([CJ0!C+9KE$N&5BI_$5/YQ#C#

M`CL<]J:;!'?^&(OM=Y<Z1>)+#'>($EC)*LKJ?E=1Z@Y!'H3[UG"R.E:DEI<2

M`JTOEOM8GC.,?SJ;PCXEO;;5[/[8Z7%JK!0\J;C%G@$-U`Y^G)K:O?#$]U=3

MW.GW\!AGD:002%=X))XQGG!W`%<]>]=$8WU':Z.:DC^S31+O\V+.TYR0V0"1

M_2B>!&L8+N-0TEJ#(`>Y##"'V.:OZ_I%S9PVL<F4F+,Q8@[<\9Y/YU']E:)&

MM`R,9T<C(/S`$$`'Z?3GZU;I"L[F-#<K*BRKD;AG)[>U68[A!G/TSZ<UCO%<

M:??/%(=JR,63<O?O]/\`$&IX69Y%0.H+'`./_KUPRIM2$=0XM$T6W$DH+32,

MX.WB,C`(R.2#P?;'UIJKG1+N-1RD\<DJGD[,,%8>HRV,_P"TOK535K9HIHM.

MB<;;=3R5+%LJ')Z^C8^@%.A4:?X6N;B8`3W^V&'=Q^Z5]SMCORBCKW/X[SCY

M#>Y0E>/``0$KSD]J;MBDC(VJ3D<8SCK5#S'+$;U_`5IZ)9R7E[\\HCB7EW(/

M0<D#U)`(Q6$(.4K!Y&@\-O9Q(&A5;B?Y"2N"B?=/'J2&!/\`C72^`KA&;4[5

MD"HT(G4@<!USCCN>>G&0#7*)#<ZKXA\F-#)</+A(5'"C)P"<]N/ZFNRNKO2?

M#TBZ/I!+7,LFV><'A/X=V3_$`2../F/3C'2X/F&NYS.KW%G<2I%`B+90*1&N

M,,S$Y+'CJ?\`.!7/+,K:I*4"JL<:H%4<<DFK-ZCI=M;K\TVXA@J?A5W0?#?]

MH6]Q<RWBQMYY0`8Y``'\\C\*YZR>HEYE19=K8(./7/>GH68'<0BCC=ZUT">%

M;:%\_;&D91PIC^7ZYSGUX_R8G\+F=$`OCM'`_=@!1G//-<_N]1F&]RI4*!@+

M_G)IHDW.%1F8G@`#DFMK_A%8TW.]^"B\85023TQG./;ZTK>&4)"1W95B/F.,

MG'3U[_3M4<T>HS%DG"_(@X[D=3_]:JYFY_7KBMH^&`8R1>\L<(`GZGGIW_\`

MKTT^&XMSG^T,1IP3Y?4^@YY_QXI.:8&,TH"G_P#54;2@`Y`Y&.:VCX9`;8+T

M;\;L>7T&>_/^>?2LO5-.M],MS*U_YDC#;#$L?+GUSG@>_MWXJ4T]AV*QF13A

MN#_O8HK'\A)?WDP\UVY)/;V'M15\J`PMY![4\,V.,=?6H.]/!SW_`!KT6C,E

M[[E(#?SJ196S@_>^M0@G\Z=]Y<-6;&7%E)'S+G'<9SBK*2*`JR],?(P/;_"J

M5H(S*$GD**?NOC@'_:]O>MZ'1X)$PT[$=L*-P_Q'^?6CF28%$R3#Y\AE'0K]

MT?AVK>TGQ)+;0B"YC6>VR&4,Q#1M_>4@C\CU_6FV^BVD+^8;TJH;&R9!U]P,

MY!K0_L#2+@-)]J1&/!4$*`<GUQQTZ5I%J][@FUL=AI7B2+5;5XY`9"BA@,D-

M@=1D<$?4'IUYJQ_9GVB&&^TN-'B;+B&3C;G(*Y/!Q^'!%<WI^F6FFSPSV]VN

MZ-LE!("4_$=CZ_Y';V1ADMU%K>Q0E\[ECD&&;/7'8^W3FNN,UW-8RON>>>(]

M#N;-%AN1^Z6?;"6Y('\)W>N/PXKF9$E@>17QE'*$CU'?\L?G7OLVCVVKV(^T

M$LY4+,%.0YQC)!'L#GM^%>9^(_"QTZT-O+(PQ(!'/)C`Z@9..AX&><<>AK.?

M+4]2'&VP[0-5BU.XB2^B`E8$,\8X<;-I9ASS@5%J=M87DT6ERWLEG=61:VCW

MP[HWRQ()8'Y>3@G!]:B\+Z->6TIFN8FVK,@"C[S98!@/8@9S[<5+=:5)K'Q$

M-I'*"[W&9#G`"+]_\3@U;A^[7,$KM)G/R:;=PW<L+C;+%D/D8"M_=Y[]?RK=

MMK:Y\R)4F6.&-0&?/5M@)''7GM[FMMK&ZU*_F6.1F#L?G"J`%4;1M7\@/I[U

MNZ3X:L8$_M"YD,T<(!5`V0Q))Y]<GJ<]J?)&&HXP:&:+HDN@66JZFV(II$(@

M?(!YR1UX`)QU]/SY#RH?#2M=3W4#WTJ8BBSOV$\[CC(_R*Z/Q=J37<2+(4M[

M),R;Y'SYAQG[H&6/],5P$]Y8W4[`/(6V9-QM&%XSCRR!D<8Z_GTJ7IJ]V.5D

M17&K7[EQY:(LF2[%"2XQD_,22:2P\37EG9)#;K"D:%B!M/)W$YY/K5%=0:S+

M2M&TY52`LV-C#!_A`ST]#4-C;K+&99PZ6\:C>_7+?W5]2?3Z\UQS][0@WK?7

MM5N1([W$20)@2,803R#@#ZXIL_C'42Y5/(\H#Y1Y9X]SS6)=WB21K##&L<`;

M.,Y8D=R?Q_G5-Q_&&?:1WQQ[5BXQ6PS?_P"$KU7@&2W^7K^ZZ_K33XIU,[OW

MD(W=?W?7]?2N?&?[Q!IQ)ZY)SW-9.*&;_P#PE6I[L[X3V_U7_P!>F_\`"3:E

M\N9(<+_TSZG\_P#.:P^2.#VI&;`RQX&3]/I4\J`VI?%&H*'+20CG)/E]?;K6

M3<7ES?7#7-PX,A&%&W&U>P%7K'3[.6+S;RZC5F^['Y@RH]3[_P`JM/9:1C*7

M2\]`9A^?_P!:BZ6R`PR78D@T5KFSTWC;>HHQ_$^2??J**.=!='&0PRSH'1#@

M^IQ5I;&X)_U>#G^\*?IO%E$<G_)-:J9ZY/%=E6;3:1"6AEBPN!D>5_X\/\:D

M&G7!_P"67_CP_P`:V%)!YZU*G3.WD?C6#J,JR,<:7=?\\\^VX4\Z7?G`6,[%

M&`/,`Q^M;:-D#('TJ42Y4``#OQ4NK(+'/G1KQB"8F_"0?XTHT"[.28R!_P!=

M/_KUTHE`BY/4T\.<$8R<=S4^UF%CG4T"]#;ANSUXFP1^M:EE%?6ZK%=1F>#<

M6!,W*$X!P<]P!D>PZ5HB4`$X'X4H(Y.#CTIQKS3&:>E:EJGAVZ74(M41M+W+

MO65]S;>Z,#T;@\Y[9K;\5^-_">K:;'8M<(\\P!9D7<(^?4=^.GTKR'QG87EQ

M+!/"CR1*FU@HZ'-<W`MS!P\3J1_>'/Y5Z5.IS04WN3SN]D?0?A22#43!8V4T

MC6J@@HZKQD]-W4\'KG-=!J6F:;X;URXU9%`EG5BS,2V,]>,_7\Z^?=(UK5+:

MXC6QNFA;C&T#K^-:OB#Q-XDDG*ZC>B5S_L@?RKGG7E[3EN6J51QYCU/1M;T>

MZNETJUF2VED41"21AR.`$!/^<GOBMSQ1?V-MI26T#"5<@A%?`;''..2,=N!7

MS[I(N-5U2".><(3(H"*"6<YX``]:]1GL=3E`>:VV*#M+2.$!/_`B*UEB);I7

M:#F:5C$U&*6_)PH08V[FDRV,=!Q@#V&*@MO"W[E9[R2-(-_RJI.9/49QT_QK

M;5+>Q&ZY>.:?HL,;;E^K$<8]@?RQS'<7<MTX>4H"%P````/0>E<T\3)ZSW)L

M9,YU5FD6$VRQ_P`*!%^4>V1Z53N+34+T`3S%EYPN`%'T`K=1N0"/KCO2,#GD

M#IZ5S2Q$WHV4<U_8$[CY6CR/0F@Z!=H3MDC!/HQ_PK6U'5[728@]RV&8\*!D

MFF17%_JD5L]H!;B=-ZDKE@N2`?TJ'5DES/1%P@Y.T499T*Z8\M'N)]>OZ4H\

M/W('WHL=\D_X5N7UCK%A:++'<I*Z\L)(P-_Y=*M0R;XPVT*_=.#@^]9QK\ZO

M%W*G1E3^)'-'0+O_`*9'/^T?\*:/#]VQSF(CV)X_2NH+$K@`8SZ4*QR"`H[_

M`%H]K(S.;_L.YY)\K\S_`(5$=#N<<&+GW/'Z5U;X/('Y"H6^_P"G>A56.R.9

M&A7&/OPCZD_X45T(8$9HH]I(#@M/!6SA'^SD?SK14@GZ<U2LEQ;Q<_\`+,?R

MJX@ZX`.*[ZFK9DMB?=R.GM[5.&"@``U"`,`'//>I%QP=W?GO60R9,$].!4@^

M8Y]/4TP*T>,X/&1BI,8.<#/I4V8$N%P">13E`(';UQ2*&/:I(R..,BE8`7'H

M,GMBI>.@'ZTNTY(`)Q5JWMY)Y`B(=S<8"YJHTI2>@KF?<Z>M]M5Y94"Y^XV,

MY]:Y;Q1X9$5FD]BLDC(WS@G)QZUZ+/IS6D[02!EST)7J/:FSVBE&EB1O*`&0

M1T_'Z_SK6,*M-W[">NAXOILD\3-Y<4C%`2<#H!UKK]&TM]9A2^NIL1Y.U4)W

M$@XY/;GTJAI4I^RZ^RKEI$.T`<Y8D?UKI_"-K<P:$GVB&1,NS1AEQE>Q^F<U

MI7BW>21<:DU'EOH:T*"*-$4$A`!D\G%.>5WR[.6([L<Y'UJ?R&7YF3:"NY<C

MK]/\]JA,83(V]_6N+ED*XP-G)QCTJ0`=3R/2A4P>%&/K3U&#M`&`#T%2XL!%

M"@C/WJ1C^\Z$^U3%"FTN-N?F`(ZCI3)2N[;MP".E2XVW*//_`!RKC5+9N=AA

M`'/&03FNH\(K??V9;N,*54*%(!POW@?Q!S7.>.H9/M-K<%3Y9382#P&SG^OZ

M5T7@+5(Y]*FM<`7,";%W-PX))'`Y..GY<UIBDY85-(Z<'*,:NIU-T+K4X@(_

M]'154RC;G<I7((/_`.JLNTBN!;^9-&@RS)N7G=M.,_I6Y&CVXVL(U22/#['S

M@GICT_'-9AD#*L:8$:;B,'(R37FT$[Z;'7C)1Y;/<C8X[`CH?K2%1VP#[4K$

MDY(X%!.#@`'W!KJ/,&LY49'%1,V`&![5*=N<''Y<U"0`>W3VHL,:VT'!X(]Z

M*:V2QPS`=N,T4["N<9:H%@B7&3M';VJQLP1Q4<0("XQ^'TJP"/[N<UZ#U9F/

M/7@<5*D6\A5')(&.U-&P?,%!/IDU(K,7+#O_`'1C^51H@+22>5A8G(.!N?N?

MIZ"I1<.-P8A^QW@,?UJFC`?PG'O4D;`%B1T[9I.<N@S22Z&Q4:WA/T7!/Y58

M2TM9E+)<&':,@2#.3Z9'\ZHP@,X..!UJ<_(`&8$G!Z=/:KC)V][4EFK!#9QJ

M,7J%@,Y",2/PP!Z=ZTX]3LXH5AA`*[?F9T.6/KPU<RC':`,\G`YYJQN5;9AM

MS*02!G&./\_E732Q%M$A6.IDU^SL-&EO)FBNX$SE%QG@<JN3C./S[<UDZG\4

M-$MM+BCM-+:\FG7_`(]3^[\I>1SC=SP#COG->.-J-U+'+<I*\1601!8W.T*=

MQP![%1CGTK1TRV6>3SYE9Y7)+.78EB>I//6ML7B%25SLPU!U=$=%X*BOXM5#

M:?IDJR/DPMA78<Y7.1S@XYXZ<5T7B6\UCP]J<C:E8SWJ%F,+)'Y:*I);:,`Y

MP2>I-5?#<\NG2"6T)213D-N/]:U?$%_=ZJ1)>`SX&`H;@'\*^?K9Q-RM;\CO

M_LU[:6^9@+\2;1FD-UI=TJL`N`@.%XP.H]!77:/_`&+KVDQZA#YD2R9&Q@5/

M!Y/?Z=:\ROK&`1OM4`<]#70^'5\G08%`.,N#SU^8UZ-'&QG&[1PXK".AN=@]

ME9+D6UQ&C`$'>",_4D<57DM3AO*.<<94H0?KMZ?7-9*W+2;E)+*.Q;I4J2F)

MP\>X'&"#R"/2KEB(2Z')8G-I.$+>6#VW;@3^AJ&2%ARWRG'0\5+)(%D&Q<(1

MD#T'<>]4/[9M[[6K;2+)!+=SN8RP/RI@9)/T&3@>E<\XQ;LBD9WBJU3_`(1Z

MZ:Y5@``4./XNV/Q-9'@CPSJ"N^IRVTD404>4SC&X?WAGJ`=OMR?2O9-,\+6E

MH?,N5^TS\?.XX4@]EZ#MZGWK7FLA)&BCC:,#`[$8/]*S<VJ;IKJ=5*"C)2D<

M;=O]OT^U78QG8@2*JX!8<''ZU5OM`N[1E*0M(A0<H,\]Q79VFF/;NY!3:1R0

M,GGWK3$0"CH/3CJ.U<=&$EK(Z,0X222/)61XV964J1_"PI%[`<?05ZE=:=:7

M*A9X(W'KMYKA==TD:9>9B!,).%!Y(.`?Z_I6S1QN%E=&%*@?())SU[5&`%.5

MW?0'@58+9'/?@@FH'./NG\:"+$>XY)!`R<T4UL;CC</HV**=A71R:.,]ZG1@

M3W]J@1<#\<?6K"C\0#FN^1F2`;5R1U_"I!T!YR:C!XX_G4G/I@"H8$HQN/'>

MI8\YR"14(*@'/'UJ6,+G@<U`%E9-A]NW>I0X9LC\15;J>/2IE3Y2JY&>/>E<

M98\P#\_SHW$QNR\<'G\*8"`H&"<TR=LVTKL<81CC\*KF%8\S,I$$4"7!:,N7

M>(9&UN@)['C^9KI=.B5MNUCP/6N-MV_?KVR>*[+2HFX.X@_7I6F92]U'JY5J

MVSM=)VC*@9_#_&K=[*J@@@Y[L.16;I:8<,KL2.V:N7(/)DR,],'^=?)27OGN

MO<YB_"M&Q#8&<]*U_#^&T.`DYY<?7YC63?LP\U2,<GH>U:OAT@Z)`2<`L^!_

MP(U[6'_AGD9ILC0P`Q^7'0&G'AAGCC^E#`9.6Y]:,#T'O6USQA+V^CLM.FFG

M)"1(2I`!(/;`XSR>E8GPJTR6;7+C5(\/]GVQ@L.<OG)]B`I_[ZIGC.?RM`VJ

MW^ME53].O]*ZOX-6X7PS=W&`K/=L"W<JJIC\.6_.NE?P6^[-:2]X].5MN3UR

M>?:GAV)#$`K]:CC3<9"X7[PZCCH/\*D6#:C'.!C/`KRZL<3[3W'[OR_X<Z'R

MDL:?*I[L=QH/+;".#TIT1_<J`.0M1'!.3SC'!KM,UNQCD;\'C`KG_$=NL^@W

M$XR=LJR#'X*?YFM>9LS1JK8RV#CTP:9J""72YK?NZ%?Q/_ZZEFC7NV/+R.OI

MT-02X[?-Z<FN=US5M0L-4EA60+"%#*H0'TZD^^?RK5T^Y>ZL4DE&V3)#93'/

MT/X4.+23.6S18QN)Z#Z\44TR*"0114ZA<Y5<$XS^=3)P,YP:KQD-R!UYJS&!

MR2:]"1D/7'K3E(Y!/2D"@``=ZD48]S6;`<!D`Y]ZD088?SJ,,Q7K^M/4X'3'

MJ*3`L+M)SUP.P[U,&'!)X^E0`DG@8SUIP.,_YS4#)LY8`$@]ZAU"58-,NY9#

MM"Q,?TJ3YF]2>O6LOQ/')<Z!.MNXRA!<+R2!R1[>OX4X*\DF)GG-L"URAXSG

M^E=;I<T^4*E=N.?E/K7*6>X7*`=2<XKM=(01C]YDYYK;,&E$]/*EN=1I4C%M

M\AW>E7;Z:#R2&8+D<'(!S4=A)%;P[W5"ON?F)I+VXL+I`"BGORG'\J^5:YIW

ML>_?4Y*_F8JP,@QNP#UYKH_#RL-#ML]3OY_X&:S]0L8A&KQ.NW/(R<5IZ&O_

M`!*("O8MD?\``C7L4)J4-#Q\SV1H@#))89%.*<ALGI4*DCJ>/3-,FU&UAA,T

ML\:QJ0C,6XW>GUK6S/(.8\>2XAL8>Y9V/X8']37I7POC$'@_31MP)5F=^.I\

MS`_05X]XOU&&^U"`VTRRQK%U7UR<_P`A7N?@F.-/"VF#`51:Q'&<9)0$_J:Z

M*J:HQ3[F]#XF=);2EK^:W[[589[COBKC.79HU(VBL5+F-=<$A!)2`J'`X))'

M^%:D)R=QP`>XKFBSHG&SN6T&V`#KDC^=59]RJ^WAL]JL@@Q@`XYY/3O4<IRI

M.T8Z'^E4S*.C*",<H#D,2">_KUI\YP@;_;7O[U3U*26UMWGB`(5E)!X_BYIY

M=[BP\YN#MW@>U1?H;M:7/#_%]NOGPSE6"EFC<@9SSP/U/Y58TFY,D2[W!+H&

MV@\Y!Y_4BK7CBU8QWI4_ZNY+#\25_DU<[H#`WC2B4+N`.T#C!['\16J3<#DE

M;EOV.ED/S?\`UZ*4@-R<D_6BLTS.QR0<11&1AD*"2![4RWU:WG<*,JQ.%SWK

M&N;ZY:1XE90F".G4=*HB)PP8-@]>E>RJ":]YG/S'<9^84]6PV%K,TB:6ZM=T

MC#<K;>._U]ZTU^1P5QP>,\C\17'*/*[,H>O7C.1GC-/0]>.:A4YY`X_.I1D"

MH8RP"`>/PQ3EP.O3IBHMV">HY_.G*02/<U-@))94AB,DC*J*,DDX`%<WIVJQ

MW/A^_A!C69(V^5`?N[0,\]?2M/6-CZ3<AB%!3&6(`]NHKC-)N(H+744DD"F6

M`JJG/S'G\*WI4[P;\T)O4HV;?Z0N>,')XKM-*D+Q!=^X=^,UQED,W('!XKK=

M+A<1(P.!G.,$U.8)..IZN5=3L[.SB:'&R3<O\0!&*K7,<=O)G=D'@KM(J?2Y

M%`8$Y&.C`XJ+46=R55(AD]0#7S2OSV9[G4R[MH/+;G&.RDG-4M)D.C6%Q>[U

M"7%QY()4D+M7=GK_`+7Z5)JD#B(>4V,CG"UIZ1IWA^ZT2`:I%J<CEB\BPW"J

MF[)&0I''`%>QA>50U>C/(S-[(Q]9U:[N]`NIXYL20RJ#Y1(!7/!YY]#^5<KI

M]U),GV65W-N9%)C#'&?7GO7K,6C_``_"A9--UB3G[INA_3%7!I/PUCB&W0M1

M=P.2]P1D^IPW\A79&M3A%QL>1:[N>)RL"ZD<<<X^IKZB\-V#6>AVD#'[L"+S

MQG"XS^0KYU\5_P!BQ^(I8]&L9[:UC"KLEEWG=W.?2OI.P=TM8H78%54*C>JX

MX-&,DG&#]?T.C#[L+!HF6?81(1(0V.2/\BK_`)AX9#\QZKC&37#S:!KU_P")

M;^:SUEM/@C$8@:&(-DD$LK@D9/(//J*T++3_`!O9)<EM3L+]M@^SF>-H\-N&

M=P4=-N>YYKBM9:,Z)5%?4[5=OD-CGV]*@W9)1L'<,?6N7$WC:..7[18Z>0$)

M!MOF8G'8,XY],USOA[4_',TD:RZ7//;&51)/>1"!X^!DA?EW*.3P,GUID+E6

MMST"\070,0^4`8R>Q['WYQ39&!L,DD?)^7%>7ZGJOQ&LH9]6NX$@AMMK_9T5

M&C9.=Q."6XZ]:],M\C3(A<EG+1@LP3;N..N!TSZ4W%HOF35D?.WQ$O+RT\;:

MO:)>SB$,I$?FG'*J2,9]ZS-+FD:&*9?OAMN?<'(X_$U?\:7T&L>-KW.FB.X:

M4P9$I'F%3MR?0\#\JJ):?V?<S6N%W1%7R"<=L]3Z&N^5O9*-M3CIW]H^QW$;

MB2-7`9@PR"**IZ=*);)#D';\O3IZ?I17G-:@U9V./6WC>Y&],EG<'Z"K,UE`

MD1*H,Y`Z^I`_QJ@FH0K)&SAAM+$C'K5D:DEU(D2*W+#DCWKV9QG<YTT:6G*L

M<<BID`2G'-:&X!B3COTJA8-F-LCDNWYYS5X'(SS@5RSW*'J3[5(ASU[5GW>H

MPV`C\X-\Y(&T4Z+5+>1`V9!GMY;'_&I<9-7`TU88_E]:/XL]/7-41J5L00!+

MG@X$+\?I2_VC;#KYOIGRG_PJ>1]@(?$H8Z'.RLR[=I./XAG&#^?Z5P2<?,5R

MHKL_$&H0RZ-/&A8LVT'<C+W'J*XM=Q`7/&?SKMPJ:AJ1+<GL!BZ'.1CO7;:8

MD2#?A67N&;C-<5IY`N>2!D<D]N178V+MM54CW*3RV,UQY@KJQ[&5;,ZS2Y>N

MW9@]A_\`KJS?;E@+Q1D=,D#C%5[!!Y8V1;,>_6FZF]U%%\B`(>_K7S35ZFA[

M=]3"U%I'V["`.Y[5;TIF33HLXS\V<=_F-9=V&6/KRW()&*@.KC3]&LUC>,NY

M*MGDCKS@5[>&IMQLCQ\S>QU)<MGI02-Q.`:XE/%4J2@9XSR221^53MXL61RL

MCF&$C.Z.+<V?3).,<>E=+PU3L>1S(S/%$36^N2R'&V50X_+!_45[_P##748=

M7\'Z=<E4\Z-/L\O.>4X!/U&#^-?.^L:FVJ['<*1!\JN%V[@3W&<?E[UU?PWU

M"^>2?3$N"+/_`%S1!@,OC'/KG`'UQ6F*3CA^9[Q-<-K4Y+[GT#;BT2YG^S>6

M79M\NP@Y8@*"??"@?A6K&`H(YKB=*,5O>*$?'F(I7!SNS_\`7VUVN>5P<<5Y

M=&ISJYMB:?)*PK,5ZY:@`D<]!VIQ/0FHRX/';^=;G.4-0$#6<R7*JT+(WF!L

M8(P<@]L5B7WBO2ET^16F:%RAP'C/IGKTJ[XC#-ISQJ<`D[B.N`,\?E7F'C:Q

MWZ#=1X)2*-B,$<$8.3^8KG=5^UC!=3NHTDZ;FSS20>1XDL9RS,T[13DL<\L<

MG/'KFM;7%V:M"Q7"2IL8],]OZBL/4IU^V:5(H;,=M%G*D=#GCCG\*V_$$T=Q

M!&T:R[D;@M$R@`CU(]0*]>:=XG!3E[UQ]IJ$EK!Y:E1DY/'?O15""8;#QD$Y

M'/KS_6BN=P5]CKY(O4P2,N`#SFKR6\T$JR,5/\.2:H/USBI(Y=F5WX4GOTKV

MZB;6AY<6=%8WL2QXD;:Y))&./SK02]MBO^NCS_O5R,=R&;#E4&.IS_0&EGGC

MP,,7`/`!Q^-<<L/=FG,K'27MM%>S6Z.NXJ2_#=O_`->/R-7H0(E5%&`HP!GM

M7)KK%QN+F4J<8Z`\#_\`74@UJYZ_:!G_`'1_A42H3M8=T=8NT'CO3P><?CFN

M1&M7*C/VD=/05'_PD-T.5E.?=14?5Y@VD=#XB;.AW'/'R_\`H0KA@<="<UL3

M:C=WUN897S&V"<;>W/I5+[-$N"V\C/J*Z*,73C9DM7>A%9C]\3[5U^BSJIVF

M,,,=NU<[!#;B$S!A&^X($8\L,$Y_#`_.NAT>`-AB<;N^>U<F.:<'<];+';0[

M33G5HCB`GT(^:J^H7,SKY:Q';G(('-1V$;VAR69B3QMXJ?46D==P4*`.=S&O

MF^5*H>W<YJ\9HR1+'PW();I6)>R1FP@F<(YCF$:QYSA>3S@Y&3G\`*T;Z\BF

MDEA=T!7@@-D_E7.RR1"4PJF8W(;>">2,\G\#[5]#@J;2NSP\RFI629%:W\UM

M&T222%)0=R>80N3[#^M7C=S_`&*6UDM_-F=E9+EISNC`_A`SC![U>30]$P&;

M5)%;LJJ>#^527MA9"WWP:Q<S.JD\[N?3D_YYKL=2#>B/*L[%'1[)-3EOX'&'

MQN!QWS6?;376CZP&A?9<0R8RO3C^A]*GT6_^Q7$KK,$+KC<W'?WJJTOG:N9'

MD!WS#+=N3UJK/FDGM8:=K-;GTUH.FQW/D3W#M(Z`LO.`,D''KV]:ZPG#KFN=

MT"51(\(ZH@ZUOL<UXL(J.B.F4Y3^)CG7D?O,;CQ2Y0'`-<SXA\1C1]9T6V*[

MQ>W(A/\`LY[_`-/QKI-W'`Q5NY!A>)3-Y$8A1I&\Q?E"YR.>I[>F?>O*?&E[

M<-:ZA:2*J!(F)VMD$XSG/UKV+4W"0%R.0ISGVKR:X@MM1\0:Y#=+E=R_Q$9'

MEJ"*F%.//SOH:>VDH<BV/)+W4&N?LA*;3!$L><]0.]=&^J_VC;S6LD.UF7]W

ML]1SS^0KDKALSM@\C"CUP.G\JTTG,)$R,5*C.>N*]>I332L84WO<L0*)85+N

M59?EP**HBX11Q(@SSUHJ'"5]&=2G&VI$>2>?S[TSM3FX:F'Z5Z3/,%8`*,-D

M]QZ4B!2PW<#F@_2FGGK4L?45PJNP5MR@\,!U'K3?IUZT'^5(>,>E2!(Y0VX"

MC!!YRP)Z#/ZTD,(D;#R",8R">]1'K[T9)P.3^/2I:&7DM)5(,9#C/4&I)XE,

M,*L2&+@&J",X(",02?7%=%J.B"W\/6%^=8M+R>Y))LH0"\(`_C.<YYQC%0[W

M5V4F6+'2],6<QS0R.N>K,?Z"NX\/^'?#UY(/M%K]!O;^F*\T%M?V<=N_V","

M5-Z8C);&2.?Q!KM+KPEXWT&RL;M[9T6[^XD$C!T.,X8<8./Y5RU*<GU_$N+M

ML>]>%_#.B:;$LME8Q))C[V2Q_4UD^*_#N@W+N\UF/,<Y8JY7G\*\OLK#X@I:

M"Y>^OX;?:2"FJ`$X'`QOR.?:JFJ:-\18[8SW4&I2KNVE9)F=V[\+G+?@#C'.

M*YHT(2>B5RW*?=B:[X5T$282W>(DXWI(<]??(KCS906%[?VFYY(XI=D98YXY

MZUT5CX`\9Z_8WMXMC9B*V1B3.?FD(&2J8!.['KCKUKB$^WP!DCM)4!/(\MCS

M^-=D*4TN5LRYEN=#IXTY5G-S$&<#,>0?\^E5?+MP\TD\BPJ5(53P!5O0/#UE

MJ_A_4]1U+Q`UA<V8+16A50T_RYP"6!)SQC'I7&OG<0S$D=<G-$:7O/4;GH(0

M,G!R`>#ZTJ-M<'CZTF"3C]:7C@^_7-=+,SZ7\&WGVV:*XQCS[9'Z^P-=HS<#

MIS7D_P`*+MI;&QW-G"M&/H"<?I7J>[]UD<D5X<H\LFNQU)Z'E7Q-U(0^-O#<

M:N&>*X24H#SPZX_K7J\<NY`P.5->`?%.XDM_B'!<(26BCC<9/0JQ(Q7M&CW*

MS6$0!!P@QCG([$'TZ5K4C:$1)ZC]?D*:=*PS]T_RKPG6O&*Z7K>L6T5J)1+,

M<R&3&.`.!CVKV_Q"5ETJ3(Q@&OF'7LC7]0'?[0^<^N:O"PC-OF1,W9:&<?O&

M@NQ!!=B#UYH8$/D#/:CVZYKTS`-V/6BEPQY49HHN!8/6D.*>_7.,4TC/:NAD

MC332",>]/QCWIA_6I8Q">.XIIQUIV,@4A'J:E@--'&*4\'Z4@Z<U(!TYK5M+

MRYM@JE5<$9`QR*S$XD4[<XYQ6E-J<,\7_'LL,N<90G:!CT/.:+)[C.MT>VM]

M9U&TM9-3@M'7RPX<E7(X^Z<8)Y`Z^M?1OA3PK:^'K/*RR7,\@YEE?=A>P'8#

MZ5\XZ'XA\(Q31/J.D7$\AE7S/,D#(%`'W0`#G(SU]J]+\7?$'2-%\,)#X,O6

MCN^!Y()*Q(PSG#9Y''3I7/*]E%&JA=7/7$C"S*@Z*2>":L/&DJ%'4,IZ@U\H

M?\+7\90PDIJA$CDC)52!G'([9X_4UOZ'\4KBZ5#XLUC662-D"P::(XE=0.2[

MY#$D]ACZUC[%Q0EJ]SU+XE>(+'0M-2WN-32VCE1MZ(P,WL0N"2#R,_3WKYPN

M=9D,Q6))9"^65I.-P]>>W!KU^+XI^`=-G%U8>&)I+I3E;B:-#(/^!L6;]:XO

MQEXM\.^)(;FYMM#FAU:XD#-=2S!O+3NJA0.OOZFMZ<]HR6@I0UT.`O[JZE"F

M5T`<=$']>M4>@]!4TI)(C)X3@4S;USP/6MDK(@;CCOBEX"$@$'M2G.`<8&1@

M4X*S#KCOF@:/5?A-<%/L\9/'GD#Z%#Q^=>QA\(>._%>"?"RX9=5\L9XDC./J

MP!KW7?@8/))KR,1&U1HV6R/!?BU)O\9OD?=@0<]^I_K7I/P]U-KWPM8RL^71

M/*89_N_*,_@!7F/Q-E$WC.\4](D1<_\``0?ZUU'PCG,FD7,`?+).6Q]5'^!K

M>K']Q%]A+XCTC5&\W3YU_B:)L#WZU\SZX2VN7[`Y_P!(?M[U])W3@L$./NMQ

MGV-?-^MJ1KE\``!Y[D#\:6"^)A/8S1R2!G/04@!QR<8[CGBI"H*@GH/2F8"K

MTSQP/I7H,R&Y4`=<]\9_I14F=O4MSSTHI6`L/@-Q33Z]:D<_-SWI'4JYY''7

M!KH9!$3@=?SIA':I",GZ=:0\GU^E)C&<@9.<4A7.,>E.]<@CM1QM[Y[5($>"

M#]*,9&2?I3]I!_'BD88YP":D8T#H<<5K:':VC7T5UJ4R16,4@:0M@LX')54Z

MDGIZ>M942&65$4$EB`!ZYK1UFU^SZQ=VBD%;:5H00,9"_+G`[G&:B6ONC7<U

M/&>LZ1K&I1RZ3IXMD2-58@!=QQZ#W[U/IE[I=OH8:^@;=)NX!.Z3!'((Z#DC

MD8X[]N8BMI9ITC4%G=M@&>3DXJWJIWWAAAP8+=1#&5S@@=6_$Y;\:RG234:2

M>BUW-8.R<K%8D.2N"%W9`)Z4\R\?*>G3WJ#RV'W@0,TF"K'C![UN9$D;L3M%

M.ED&#OR/05%OPPRO3U-.>2-L$CCOB@`R#(Y[D#H>G%+@'Y>#S]*:A!<DYZ#C

M\*F5#+(L:*6=L!5'?Z4#&`GL,GIS3E7YLCO@5)+!)!(4E0HRGY@1S]*55R0<

M\],4FQI7.H^&LJQ>(I$=NI3&?4.*]B749G)R0<\@#BO%?`L#/XI<`$*L/FDC

MMA@1^N!^->M(V8@IX;L<5YF*7[PTCL>3^/$'_"779SGS%1C_`-\BMOX3,\<V

MHKT"[,`9Z\UB^.7W^*[@=-B(O'0_*#_6MCX93BW74F"@DR*&&<<<UO4_W?[A

M?:/2KF;;*KY&.?Y5X-XB39K]]QSYI(_'FO;[QU?*XSA>0.N6^4?J?TKQ;Q05

M?Q)?;1P)2OY5E@_B8Y&)M]&YHV%FQR/\:EVY&3N.*8RDD`<`=A7HF=BLZLK8

M!`_#_P"M14^0I/)R>>M%*PK(F?&:1NF,]:D=<L#WI"G%=!G<A(&>AR3V%-_4

MG]*G\OI33'GBI&1D$`8'Y"D*'MSWP!4PA./0<<4"!L@YQ_C2LQD.`#DC'!_"

MDR"FX#GZU+Y!SV]Z#&PQCH",4AE_PFBGQ=I9D&42Y5VXZA3G^E4YY&N;N:<G

M+22,Y)[DG-3:6P@>_F)VE+=]A'9F(4?SJBKMD`#`[X[5FE[[8[V1T7@^Q:^\

M5:>C@A1,K'\#7TL+>WT;P[-%IFGVB,7+&)(-JL.!SM'7+#]?2O#O!^JV&E:'

M%=_8WE>.X\RYD9A$%;D1J&/&._XFO5O#?Q1\-LMQ)JNH064A*[(WG,O;DC:N

M!V^N/:N.<FZ]^B-FK4T$>GVYE^TOX;L#.QW;Q:98'UR11INBQ:O?74VKZ+8^

M3A4C22W&3SG/(]OUK>;XI^"$Z^(;4_17/\A6%K_Q7\,*ML^FW\5Z^X[D68PX

M]#\RX/\`2M:E5R@TD9I:['D_CSPM86-Q=?V=IZ0(DC`R`O@'`.,=,<@5YM("

MLA5^&'I_GWKUWQ=JRW^@7>JPV;Q6]](L*N)_-'FALL2>`,JH&`#TKR6Z.ZX;

M'<?_`%JN@[TU<4]QT)'F'\.:G7*N"I((Z$=:@MP&+9'3'/:K6UL=.E:L2&A<

M]><5(!CK^E&P[P3UJ6.W9J+-[%72.V^'.F23+?WB("TC+;JWH!\S?J5KT_[%

M';6ACQN;&2W>N,^%DL0M+W3GR)%D$RMG@A@!T^J_K79ZE<0:=;.;J543!P\C

M@8_&O.KTJCJ.Z*4E8\2\:J5\67G?.WG_`("*T?A\VR:_8Y(5XB0.XYS_`)]Z

MY_5+C^T-5N;HG=YCDJ?;M^F*Z'X?-`->EM[C"I-"6!_VEY_D6KJJ49>QY1<R

MN>@M,L.GB4NI>23)YZ'T_"O%M7#?VO<^9RYD);ZFO>;JSMXK![@Q+\J%L,,]

MJ\1UVY^UZY>3%$3,A7"=../Z5AA*;4FQN2,<*<?U-!4Y.0,8XJ<@<TW%=I-R

M$KSR0#[@45(5#'O15`P;[U(2%''YT-US3&!QVK4PL.W\CM2AN<\4T!L$=Z7&

M`3Q]!2;+L/$@Y!!ST&/6EW]NF:A/48Z]J0^I!QUI786'M(`>V<_G3=Y3&3GZ

M=J3`'5C[4T@LZ(&^\>3Z`=32;&:+HD'A9IF4B:]NPL9/1HXU^8_]],OY>U4[

M._,$(A^Q0R$DG,@R32:I="YEB2$DVT"^5`.A"Y)&1TR22:+>,!!,ZXC3E0?X

MW_P[G\/6LHMQ7-U92AS2L:LFNW5GHMQHL<$#17++-<$KE@_4`<\8R.W45SH=

MRW\1[]*G8/+,TDAW,3DD\Y-(IY(.#BG&'*ASES.XSY\'`<CUQS0"PD!8D`8(

MSGK4Z8<9!.*>%4J05!'OS5$V-S_A,+Z7PS;Z!+;6QLX)?-7]V0Q//4_C]:Q=

M0ODND`%I#$Q.=Z@@G_ZU0D($+&,9QTQ59R6/+;N*48J*:2%+S+=FNT.>""0,

MCVZ_SJX/R&>*K6L96`9!RQ)YJP%)XZTKLI(7-3Q2L!G&0.]0$8`IV/<4U-H'

M&XX:O>V<\BVL\MOO3:Q1B"1G/7KVJI<7EQ?S+Y\\LC]-TKECBIO+(F2979)(

MR&5E."".AIL=LD3%@"7]ZMU>I/*2(-J!?04UKB:V5IH)7CD'1T)##L<'MQ3L

MCKCFFNH8%3C!J/:/J5RER/QSXBAL_LHU.1XCGB55D/YL":SHYWN`TCDEV/S$

M]S3!;%8GB!7:S!LE`6&,C@]>_P#+T%31Q[45`,XZ>]*\5L@2?45N"0"",]12

M8I<8I".U*Y=AA7<<T4[IW_2BB[#E'.!MS@?YS4,W$;445JC.IN,`&X?6G1]!

M1138H[BCK3#_`!?[U%%(.@\`>;TZ?X5#=$B(X)&6`_"BBH91!``9,$#H:T[Q

M5`A`4`>4IX'J***E[HTI?!+Y?F5`H*KD"B;[I_&BBK,QO=OP_K4PYQGGY:**

M&);#)`-K\=5S550/EX[_`.%%%"$]S8BYAB)Y)0$Y[U,0-G0=J**A]!HB/WA^

M%'8_3^M%%24+W3Z_TI._XT44#$/\7XTP]3]#111T&A/6E'4444`'\(_&FGJ:

-**`Z"H!CI11128S_V9._

``

end

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: Microwave transformers

Date: Thu,  7 Aug 1997 06:18:44 GMT

Organization: The Energy Machine Information System 504-734-0526

From: Bert Hickman <bert.hickman@aquila.com>

Date: Wed, 06 Aug 1997 23:18:44 -0700

Subject: Re: Microwave transformers

Organization: Stoneridge Engineering

Tesla-2 wrote:

>

> From: watkier@earlham.edu

> Date: Wed, 06 Aug 1997 14:42:18 EST

> Subject: Microwave transformers

>

> Has anyone tried using microwave transformers as a tesla coil power

> supply?  I'm still not experienced enough to know if this is worth

> pursuing or  if I should stick to the tried and true path of neon sign

> transformers and ignition coils.  Currently I have two microwave

> transformers wired together and this supply is capable of jumping a modest

> spark gap.  I would guess its ouput is around 4kv 250-500ma AC.  Any

> thoughts on how this might perform as the HV power supply for a tesla coil??

>

> -Erik

>

> * Send list Replies to: Tesla-2@emachine.com

>

> * To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

>   in the body of your message to: Listserv@Emachine.com

>

> -----

> Processed with Listserv v2.93 for Wildcat v4

Erik,

Microwave transformers can be made to work in disruptive systems only

with a fair amount of difficulty, since they don't quite have enough

voltage. Most coilers that have been sucessful have used them like plate

transformers in vacuum tube coils.

-- Bert --

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Old Geezer, Huh?

Date: Thu,  7 Aug 1997 12:28:22 GMT

Organization: The Energy Machine Information System 504-734-0526

From: "Thomas McGahee" <tom_mcgahee@sigmais.com>

Date: Thu, 7 Aug 1997 08:28:22 -0400

Subject: Old Geezer, Huh?

> > >

> > >

> > >

> > > Well, I am probably the "old fogey" on this list, with an age of 50.

> >

> > Ah HA! A geezer. I KNEW it.

> >

> > > However, age is relative. I keep myself young in heart and mind by

> > > having an intense curiousity about things.

> >

> > Uh huh, now the excuses....."I don't FEEL old...."

> >

Well, I DON'T! So there!

> > > My interests include not

> > > only Tesla coils, but also x-rays, particle accelerators, Van de

> > > Graaf Generators, anything high voltage, anything scientific, and

> > > also art, especially calligraphy.

> >

> I must be the youngest one here. I usually tell everyone I'm 17, but to

> be precise, I'm 16 years, 5 months and 8 days old today. I think I share

> about all of the interests listed above (including an intense curiosity

> about everything) as does my father, who's going on 42. I don't feel

> particularly young, (I've had my drivers' license for a week, and I

> missed the Carter Administration by two months) but I suppose age in

> that sense is in the eye of the beholder.

>

> Older coilers have the benefit of experience and knowledge, while young

> people learn from their experiences so that when they're older, they

> have the extra experience of having started at an early age.

> My basic opinion is that age is totally independent of intelligence and

> maturity, so if you can accomplish at age 20 what others do at 50, then

> by all means, go for it! My goal is to finish a working coil before

> starting eleventh grade (which gives me about 4 weeks) :).

>  -Shane Randle

>

Shane,

You are right about about the experience part. I built my first Tesla

coil when I was 13 years old. Since then I have built literally

HUNDREDS of them. And from every one of them I have learned something

interesting. This new information then goes into the next iteration.

By the way. sometimes I purposefully build what I KNOW will be a

lousy coil just to gather specific information concerning the

relative lousy-ness. For example, I can definitely tell you that a

thin polyethelene tube makes a MUCH better coil form than one made

from (yuck!) untreated cardboard. Nothing helps you learn as much as

comparative experimentation! Never settle for just book knowledge.

Get your fingers dirty and get in there and actively experiment!!!

And keep your interests as broad and wide ranging as you can. It is

amazing how the information gathered in one area can give you new

insight into an investigation that you are carrying on in a

supposedly unrelated area. (No such thing... EVERYTHING is related in

one way or another!)

Learn the Ancient Art of Dumpster Diving. It is the time-honored way

to get your hands on all sorts of free or very low-cost goodies.

Coilers don't go to the dump to drop stuff off. We go there on a

shopping spree. It's amazing what you can get at a well-stocked

municipal or industrial dump. And hound the ham-fests. Even if you

don't BUY anything, you can learn a lot just by getting to know what

all the stuff IS. It also gives you an incentive to save up your

pennies for the NEXT ham fest!

By the way, for anyone who hasn't already guessed it, Michael Smith

is one of my friends on this list, and he takes malicious delight in

trying to rib me every chance he gets. Probably because I do the same

to him all the time in our private posts. One of the neat things

about Tesla coiling is the interesting people you meet.

Keep up the work on your Tesla coils, Shane, and I hope that all this

is just the beginning of a lifelong interest in MANY wonderful

things!!!

The Old Geezer,

Fr. Tom McGahee

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: Safety hazards??

Date: Thu,  7 Aug 1997 13:19:41 GMT

Organization: The Energy Machine Information System 504-734-0526

From: Bert Hickman <bert.hickman@aquila.com>

Date: Thu, 07 Aug 1997 06:19:41 -0700

Subject: Re: Safety hazards??

Organization: Stoneridge Engineering

Tesla-2 wrote:

>

> From: James <057006@bud.cc.swin.edu.au>

> Date: Thu, 07 Aug 1997 18:11:31 -0700

> Subject: Safety hazards??

> Organization: 057006

>

> Now that I've got my coil working, I guess I have to know what

> precautions to take!

>

>  I've been sticking my hand in the sparks and holding a fluro tube with

> the other, normally I understand that this is a very dangerous thing to

> do (allowing current to pass through your body accross your heart) but

> as I understand it, the high frequency HV from the TC exhibits 'Skin

> effect' which prevents the current from penetrating more than a couple

> of layers of your skin. Am I deluding myself? I'm still alive, but my

> coil is not very powerful. Any pictures I see of people doing this have

> a big warning - don't try this at home!!! Well I did.  Another thing is

> that I'm starting to experience a stinging in my eyes that feels like

> there are tiny bits of metal or something embedded in the balls. I

> assume this is not the case, but could this feeling be caused by UV

> radiation or some such from the coil? Also I'm concerned about the

> possibility of X-ray emmissions when I do the plasma ball thing putting

> a normal light globe on top of the discharge terminal.

>

> Every web site I've seen has a link to some safety precautions, but most

> of the time this section is still "Under construction!"

>

> I'd just like a few safety guidelines to follow (the hand in the pocket

> thing appears to be less important when dealing with the HF.)

>

>   Please help me to perform  these addictive experiments with safety!

>                                           James Hurl, Australia.

>

James,

There are a number of risks associated with Tesla Coiling... only one of

them is electrocution. A fairly good FAQ exists on Chip Atkinson's newer

website:

http://www.pupman.com/

The stinging in your eyes is probably irritation form either ozone or

some of the oxides of nitrogen which are created by the high voltage

discharges. UV damage to your eyes would only occur if you had stared at

your operating spark gap for some period of time... from personal

experience with a carbon arc in my youth, this feels like having hot

sand thrown in your eyes and can be very painful. Read the FAQ so that

you know all the risks you're dealing with.

-- Bert --

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: Our Age

Date: Thu,  7 Aug 1997 14:07:59 GMT

Organization: The Energy Machine Information System 504-734-0526

From: "Thomas McGahee" <tom_mcgahee@sigmais.com>

Date: Thu, 7 Aug 1997 10:07:59 -0400

Subject: Re: Our Age

----------

> From: Tesla-2 <tesla-2@emachine.com>

> To: tom_mcgahee@sigmais.com

> Subject: Re: Our Age

> Date: Wednesday, August 06, 1997 3:48 PM

>

> From: "Michael Smith" <md.smith@usa.net>

> Date: Wed, 6 Aug 1997 14:48:00 -0500

> Subject: Re: Our Age

>

> ----------

> > From: Tesla-2 <tesla-2@emachine.com>

> > To: md.smith@usa.net

> > Subject: Re: Our Age

> > Date: Wednesday, August 06, 1997 8:33 AM

> >

> > From: "Thomas McGahee" <tom_mcgahee@sigmais.com>

> > Date: Wed, 6 Aug 1997 09:33:16 -0400

> > Subject: Re: Our Age

> >

> >

> >

> > Well, I am probably the "old fogey" on this list, with an age of 50.

>

> Ah HA! A geezer. I KNEW it.

You know NOTHING! In ten years YOU will be an old geezer and *I* will

still be coiling circles around you!

>

> > However, age is relative. I keep myself young in heart and mind by

> > having an intense curiousity about things.

>

> Uh huh, now the excuses....."I don't FEEL old...."

>

Well, I DON'T feel all that old. In fact, if you look at "old"

coilers, you will find that almost all of us have wide-ranging

interests and that keeps us younger than you would think. I prefer to

think of us older coilers as more "mature" and "knowledgeable" and

"experienced".

> > My interests include not

> > only Tesla coils, but also x-rays, particle accelerators, Van de

> > Graaf Generators, anything high voltage, anything scientific, and

> > also art, especially calligraphy.

>

> Reanimation of dead tissue, perhaps....?

What? You are maligning the reanimation of dead tissue? Aren't YOU

the guy that bought your oil from the guy that was also supplying the

burial vault business??? Look at all the really good Frankenstein

movies, and what do you see? Tesla coils and Jacob's Ladders and

other cool stuff, that's what!!! All the really Experienced Coilers

have already done the reanimation of dead tissue experiments many

times over. But this is slightly off-topic. If you want more

information, why not join Frank N. Stein's Reanimation of Dead

Tissues List Bodies-R-Us@It-Lives.com

>

> I am happiest when 1) I am *with*

> > people that I love, 2) when I am sharing people who have like interests,

> > and 3) when I am just plain tinkering around with stuff.

>

> Which people? The ones you dug up and reassembled?

>

Picky picky picky!!! It was supposed to read  2) when I am sharing

WITH people who have like interests

Besides, I don't actually dig up people myself. I get my body parts

from Igor's Used Body Parts. I forget the exact URL. Use Altavista

and search for it yourself.

> > Numbers 2 and 3 explain why I "hang out" on the Tesla and Tesla-2

> > lists so much.

> > Fr. Tom McGahee

>

> #2? Mmmm, hopefully not scatalogical humor. :)  Tinkering I can

> definately relate to.

>

> Woo Hoo, feeling mean today.

>

Why should today be any different?

Wasn't it Daffy Duck that was always running around going Woo Hoo? Or

was that Woo Woo?

> Michael Smith in Lincoln, NE

> Aged 40 years.....*NOT* a geezer.

>

You are not a geezer yet, but definitely WORKING on it!!

I am PROUD of the fact that I am 50 years old. I have been coiling

since I was 13, and enjoying every minute of it. Well, there WAS that

time that my brother plugged one of my coils in while I was up to my

armpits in the tank circuit... THAT was a minute I could have done

without!

All kidding aside, I really am happy to still be alive and coiling. I

very much enjoy the comaraderie that is so much a part of the coiling

experience, and hope that through my posts I can perhaps help a few

others to also find much joy in the experience of coiling.

The Old Geezer

Fr. Tom McGahee

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: Microwave transformers

Cc:

Bcc:

X-Attachments:

At 04:36 AM 8/7/97 GMT, you wrote:

>From: Shane Randle <randlec@scnc.addison.k12.mi.us>

>Date: Thu, 07 Aug 1997 00:36:54 -0400

>Subject: Re: Microwave transformers

>

>Tesla-2 wrote:

>>

>> From: watkier@earlham.edu

>> Date: Wed, 06 Aug 1997 14:42:18 EST

>> Subject: Microwave transformers

>>

>> Has anyone tried using microwave transformers as a tesla coil power

>> supply?  I'm still not experienced enough to know if this is worth

>> pursuing or  if I should stick to the tried and true path of neon sign

>> transformers and ignition coils.  Currently I have two microwave

>> transformers wired together and this supply is capable of jumping a modest

>> spark gap.  I would guess its ouput is around 4kv 250-500ma AC.  Any

>> thoughts on how this might perform as the HV power supply for a tesla coil??

>>

>> -Erik

>>

>I'm trying (keyword: "trying") to power my small Tesla coil with only a

>single microwave transformer to see if it will work, but it only puts

>out about 2200VAC. I'm not done testing it yet, so I don't have any

>definite results, but I'll let you know how it works.

>Right now, though, I'm thinking about building a plate capacitor to

>replace my salt water cap. I actually have two microwave transformers,

>and if the one doesn't work, I'll probably wire them together like you

>did. As for experience, I doubt I have much more than you, so I can't

>really offer any expert advice yet. I should be able to finish testing

>it in the next few days, so I'll let you know. :)

> -Shane Randle

>

>

I briefly experimented with a pair of microwave transformers

a couple years ago. Results were disappointing.

Problem is that the voltage is way too low for spark gap

operation. And the high current level makes gap quenching

very difficult.

If you can get neon sign transformers go with them.

Much easier and far better results.

The best results will come from wiring two microwave transformers

in series. Wire the primary windings in parallel and the

secondary windings in series. Check for proper phasing.

This will give you about 4500 volts with typical microwave

transformers. Use of large cap values will help

soak up the available current and minimize flaming at the gap.

You will need closely spaced gaps to operate at this low

voltage. Forced air to remove the large amount of heat generated

will be necessary.

Wiring more than two in series in generally not advisable

due to the likelihood of insulation breakdown.

Let us know what results you get from these experiments.

TTYL!   Mike Hammer  mhammer@misslink.net

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: Safety hazards??

Date: Thu,  7 Aug 1997 19:04:44 GMT

Organization: The Energy Machine Information System 504-734-0526

From: corr <corr@fullnet.net>

Date: Thu, 07 Aug 1997 13:04:44 -0600

Subject: Re: Safety hazards??

> I've been sticking my hand in the sparks and holding a fluro tube with

>the other, normally I understand that this is a very dangerous thing to

>do (allowing current to pass through your body accross your heart) but

>as I understand it, the high frequency HV from the TC exhibits 'Skin

>effect' which prevents the current from penetrating more than a couple

>of layers of your skin. Am I deluding myself? I'm still alive, but my

That is correct as far as I know, but I learned something interesting the

other day.  If a primary-to-secondary arc occurs whil your in the RF field,

it will kill you dead instantly.  I had to rebuild my primary, and the newer

primary had alot of primary to secodary acrs, so I have stopped sticking

my hand in the field, that would be a good idea for you to do as well.

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: Safety hazards??

Date: Thu,  7 Aug 1997 19:20:34 GMT

Organization: The Energy Machine Information System 504-734-0526

From: Jochen Kronjaeger <Kronjaeg@Stud-Mailer.Uni-Marburg.DE>

Date: Thu, 7 Aug 1997 21:20:34 +0200 (CES)

Subject: Re: Safety hazards??

> possibility of X-ray emmissions when I do the plasma ball thing putting

> a normal light globe on top of the discharge terminal.

There definitely no danger here. For X-Rays, you need not only high

voltage but also a pretty good vacuum. Several orders of magnitude below

the pressure in gas discharge tubes, plsama  globes etc.

Jochen

---------------------------------------------------------------------------

Jochen Kronjaeger                     Kronjaeg@stud-mailer.uni-marburg.de

The High Voltage Page  http://www.mathematik.uni-marburg.de/~kronjaeg/hv/

info, plans, photos of home-built HV-sources, sparks, HV-experiments

---------------------------------------------------------------------------

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: first coil

Date: Thu,  7 Aug 1997 19:21:54 GMT

Organization: The Energy Machine Information System 504-734-0526

From: engineer <engineer@techinter.com>

Date: Thu, 07 Aug 1997 14:21:54 -0500

Subject: first coil

Organization: DeVry Institute of Technology

Hello everyone.  I'm trying to get my first coil finished but I've had

problems and not much time to work.  I can't get my coil to run, but I

can't quite get it in tune.  I believe Mike Hammer told me to build a

new primary of 12-14 turns with small copper tubing or at least #12 bare

copper wire.  My coil is 3" in diameter, 12" high powered by a 6KV@23mA

transformer. The cap is .005uF.

I have found some #6 solid copper wire, but I've read that its better to

use tubing for a primary.  I realize that with the high freq. the

current mostly travels on the outside edge of the conductor.  But if

this true, why is it better to use tubing?  If the center of the solid

wire isn't conducting, shouldn't it's resistance be only dependent on

the outer edge?

Is the #6 wire too big?

If copper tubing is used, how thick should it be?

Thanks,

Brian Whildin (engineer@techinter.com)

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: Safety hazards??

Cc:

Bcc:

X-Attachments:

At 07:04 PM 8/7/97 GMT, you wrote:

>From: corr <corr@fullnet.net>

>Date: Thu, 07 Aug 1997 13:04:44 -0600

>Subject: Re: Safety hazards??

>

>> I've been sticking my hand in the sparks and holding a fluro tube with

>>the other, normally I understand that this is a very dangerous thing to

>>do (allowing current to pass through your body accross your heart) but

>>as I understand it, the high frequency HV from the TC exhibits 'Skin

>>effect' which prevents the current from penetrating more than a couple

>>of layers of your skin. Am I deluding myself? I'm still alive, but my

>

>That is correct as far as I know, but I learned something interesting the

>other day.  If a primary-to-secondary arc occurs whil your in the RF field,

>it will kill you dead instantly.  I had to rebuild my primary, and the newer

>primary had alot of primary to secodary acrs, so I have stopped sticking

>my hand in the field, that would be a good idea for you to do as well.

>

>

That type of event will not necessarily kill you instantly

but it will surely diminish your chances of becoming an "OLD

GEEZER" coiler.

Even if you do not have problems with primary/secondary

arcing the power level in medium to large size coils

with large terminal capacitances can be lethal.

If you simply must play Russian Roulette with streamers

isolate your self very well from ground. Standing

on the concrete floor in your sneakers doesnt qualify.

Something like a plastic milk crate would be the minimum

acceptable. Or play it safe and watch the streamers from

a distance.

Live long and prosper.

-----

Processed with Listserv v2.93 for Wildcat v4

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: first coil

Cc:

Bcc:

X-Attachments:

At 07:21 PM 8/7/97 GMT, you wrote:

>From: engineer <engineer@techinter.com>

>Date: Thu, 07 Aug 1997 14:21:54 -0500

>Subject: first coil

>Organization: DeVry Institute of Technology

>

>Hello everyone.  I'm trying to get my first coil finished but I've had

>problems and not much time to work.  I can't get my coil to run, but I

>can't quite get it in tune.  I believe Mike Hammer told me to build a

>new primary of 12-14 turns with small copper tubing or at least #12 bare

>copper wire.  My coil is 3" in diameter, 12" high powered by a 6KV@23mA

>transformer. The cap is .005uF.

>

>I have found some #6 solid copper wire, but I've read that its better to

>use tubing for a primary.  I realize that with the high freq. the

>current mostly travels on the outside edge of the conductor.  But if

>this true, why is it better to use tubing?  If the center of the solid

>wire isn't conducting, shouldn't it's resistance be only dependent on

>the outer edge?

>

>Is the #6 wire too big?

>

>If copper tubing is used, how thick should it be?

>

Brian,

AWG 6 copper wire will do just fine for a primary.

Since you got it use it. Tubing is recommended for

primary coils for several reasons.

First its readily available. And generally cheaper than

the same size wire. It does not have any advantage

electrically over the same size copper wire.

Plus you will find tubing easier to bend than the same size

copper wire.

You ever try to find 1/4 inch diameter copper wire?

And note the price once you do find it. Therefore

copper tubing is the best choice.

If you want to use tubing 1/4 inch is plenty big enough.

But for your low power coil any solid wire from

AWG 12 on down will work well. If you want to plan

for higher power later use 1/4 inch tubing or your AWG 6.

TTYL!   Mike Hammer  mhammer@misslink.net

From: tesla-2@emachine.com (Tesla-2)

Subject: Safety hazards??

Date: Fri,  8 Aug 1997 01:11:31 GMT

Organization: The Energy Machine Information System 504-734-0526

From: James <057006@bud.cc.swin.edu.au>

Date: Thu, 07 Aug 1997 18:11:31 -0700

Subject: Safety hazards??

Organization: 057006

Now that I've got my coil working, I guess I have to know what

precautions to take!

I've been sticking my hand in the sparks and holding a fluro tube with

the other, normally I understand that this is a very dangerous thing to

do (allowing current to pass through your body accross your heart) but

as I understand it, the high frequency HV from the TC exhibits 'Skin

effect' which prevents the current from penetrating more than a couple

of layers of your skin. Am I deluding myself? I'm still alive, but my

coil is not very powerful. Any pictures I see of people doing this have

a big warning - don't try this at home!!! Well I did.  Another thing is

that I'm starting to experience a stinging in my eyes that feels like

there are tiny bits of metal or something embedded in the balls. I

assume this is not the case, but could this feeling be caused by UV

radiation or some such from the coil? Also I'm concerned about the

possibility of X-ray emmissions when I do the plasma ball thing putting

a normal light globe on top of the discharge terminal.

Every web site I've seen has a link to some safety precautions, but most

of the time this section is still "Under construction!"

I'd just like a few safety guidelines to follow (the hand in the pocket

thing appears to be less important when dealing with the HF.)

Please help me to perform these addictive experiments with safety!

James Hurl, Australia.

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Introduction of a new member

Date: Fri,  8 Aug 1997 01:42:06 GMT

Organization: The Energy Machine Information System 504-734-0526

From: LINEMAN23K@aol.com

Date: Thu, 7 Aug 1997 21:42:06 -0400 (EDT)

Subject: Introduction of a new member

All,

My name is Rusty and I am glad to be a new member of this list.  I think it

is great that there are still people out there willing to donate their time

helping others!!!  It would be a nice world if everyone were this generous.

I am in the design process of attempting to build my first Tesla coil and

plan to go all out.  I have aquired four neon sign transformers, three are

15KV@60mA and one is 15KV@30mA.  If I parallel them, will this give me 15KV@210mA?

If so will the power be 3150 (Watts or VA)?  26.25A on the primary? Will a 30A

breaker hold this?  Would somone please explain a transformer being "shunted" or

"non-shunted"?

From the best of my calculations I need a .037uF cap. I plan to build (and

parrallel) the rolled type cap and have found some 20"x50' flashing. Also

found is 6mil poly 10'x100'.  I plan to use double thickness poly (12mils)

24"x96" and leave a 2" border on the sides and a 3" border on the ends of the

flashing 20"x93".  What will be my approximate value for C per cap?  Will the

voltage rating be high enough?  What equation /s are needed to approximate

this?

I intend to use the Richard Quick static gap w/muffin fan.

Do I need RF protection?

Will the one gap be enough for the power I will be delivering?

I appoligize for the long post but I need some answers before I can get

started.

Thanks again

Rusty

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: Introduction of a new member

Date: Fri,  8 Aug 1997 03:00:20 GMT

Organization: The Energy Machine Information System 504-734-0526

From: BadBowTie9@aol.com

Date: Thu, 7 Aug 1997 23:00:20 -0400 (EDT)

Subject: Re: Introduction of a new member

Wait,......Now just what is a Muffin Fan?   I must have missed this one!

Till Later MH?

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: Safety hazards??

Date: Fri,  8 Aug 1997 07:44:44 GMT

Organization: The Energy Machine Information System 504-734-0526

From: corr <corr@fullnet.net>

Date: Fri, 08 Aug 1997 01:44:44 -0600

Subject: Re: Safety hazards??

I am sorry of hearing this too...  it was the coolest trick in the world.

However, I recently received quite a "stop-do-it" warning shock

from my coil.  My primary and secondary are 2.5 inches apart and

it does it seriously.  From what I understand, the only Tesla Coil

related death every was caused by this happening.

>> That is correct as far as I know, but I learned something interesting the

>> other day.  If a primary-to-secondary arc occurs whil your in the RF field,

>> it will kill you dead instantly.  I had to rebuild my primary, and the newer

>> primary had alot of primary to secodary acrs, so I have stopped sticking

>> my hand in the field, that would be a good idea for you to do as well.

>

>

>Corr,

>            WOAH! what a let down. I've been enjoying this little trick,

>to the point where I go out to the TC every few hours for

>"reenergization"! I'm pretty sure there's no danger of this type of

>malfunction occurring though, at least with the power levels I'm using

>at the moment. I've got the primary at least an inch &1/2 away from the

>secondary at the base, I imagine an arc there will just travel to

>ground? The top of the primary is further away as the thing tapers

>outwards, but I'll probably have to be a lot more cautious when I get

>the neon transformer and add more bottles (or some poly's now that their

>construction has been so thoroughly detailed on the list) Also, if I

>place a more attractive "load" than my hand in the field also, attached

>to earth, would the RF energy dissipate through this rather than my

>body? And if all else fails, if I'm standing on a good insulator and

>have one hand in my pocket, and the "DEATH ARC" between the primary &

>secondary occurrs, the usual, 'path of least resistance' thing should

>save my speeding heart from overload, shouldn't it?

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: Re[2]: Glass Capacitor

Date: Fri,  8 Aug 1997 14:08:47 GMT

Organization: The Energy Machine Information System 504-734-0526

From: cjhill@hills.u-net.com (CJ Hill)

Date: Fri, 8 Aug 1997 15:08:47 +0100

Subject: Re: Re[2]: Glass Capacitor

>

>     I'm also interested in what ages everybody on this list is so I can

>     get an idea what types of people are out there coiling.

>

>     Nathan_Bonsal@Intuit.com

>

Nathan, I'm 25.

-------------------------------------------------

Chris Hill, Cardiff, UK

cjhill@hills.u-net.com

-------------------------------------------------

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re[2]: Safety hazards??

Date: Fri,  8 Aug 1997 15:24:48 GMT

Organization: The Energy Machine Information System 504-734-0526

From: Nathan_Bonsal@intuit.com (Nathan Bonsal)

Date: Fri, 8 Aug 1997 08:24:48 -0700

Subject: Re[2]: Safety hazards??

If you're living in a flat on the second floor, How did you accomplish your

ground system? I hope you're not using the ground supplied at the outlets.

Did you construct a ground system?

If not, stop where you are before you kill someone's TV or computer. Then build

a good ground. The problem with being on the second floor is safety. If you

build a ground connnection, and run a wire or cable to it, this cable could

present a high voltage danger to children playing around the building. So your

ground is going to have to be very deep and very conductive. An almost flawless

ground connection is going to have to be made in any case. For that reason I

don't recommend Apartment Coiling. 1. Annoys the neighbors with high levels of

RF interference. 2. you share a circuit with others who may turn you in. 3. In

that close an environment it is impossible to control the influence of the

electric field around the coil. 4. Your other electronic components are also on

your flat and could be fried to a crisp. 5. It probably voilates your lease and

could get you evicted,

Please do something about grounding. Someone could get hurt.

Nathan Bonsal

>

Guess I should have read all the mail first, then replied!

But I'm actually living in a flat on the second

floor! does this qualify as good insulation from ground? I guess that

there are wires under my floor though for the lighting & such on the

floor below.

What's a good all round material to cover the floor with?

Also, I thought of (maybe) a good way to make a toroidal discharge

terminal for a small coil. You know those cake tins that make rounded

cakes? 2 of 'em one on top upside down on the other could be a neat

terminal, but what of joining them so they don't spark? Is welding them

together and filing/smoothing/polishing out of the question? People seem

to use salad bowls just taped together but is this enough? the sharp

edges are surely going to limit capacity even though they're

insulated...

Thanks,

James Hurl, Australia.

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: Introduction of a new member

Date: Fri,  8 Aug 1997 16:04:27 GMT

Organization: The Energy Machine Information System 504-734-0526

From: "Michael Smith" <md.smith@usa.net>

Date: Fri, 8 Aug 1997 11:04:27 -0500

Subject: Re: Introduction of a new member

----------

> From: Tesla-2 <tesla-2@emachine.com>

> To: md.smith@usa.net

> Subject: Introduction of a new member

> Date: Thursday, August 07, 1997 8:42 PM

>

> From: LINEMAN23K@aol.com

> Date: Thu, 7 Aug 1997 21:42:06 -0400 (EDT)

> Subject: Introduction of a new member

>

> All,

>

> My name is Rusty and I am glad to be a new member of this list.  I think it

> is great that there are still people out there willing to donate their time

> helping others!!!  It would be a nice world if everyone were this generous.

>

> I am in the design process of attempting to build my first Tesla coil and

> plan to go all out.  I have aquired four neon sign transformers, three are

> 15KV@60mA and one is 15KV@30mA.  If I parrallel them, will this give me 15KV@210mA?  >If so will the power be 3150 (Watts or VA)?  26.25A on the primary? Will a 30A breaker

> hold this?  Would somone please explain a transformer being "shunted" or "non-hunted"?

>

> From the best of my calculations I need a .037uF cap. I plan to build (and parallel) >the rolled type cap and have found some 20"x50' flashing. Also  found is 6mil poly >10'x100'.  I plan to use double thickness poly (12mils) 24"x96" and leave a 2" border >on the sides and a 3" border on the ends of the  flashing 20"x93".  What will be my >approximate value for C per cap?

>Will the voltage rating be high enough?  What equation /s are needed to

>approximate this?

>

> I intend to use the Richard Quick static gap w/muffin fan.  Do I need RF

> protection?

> Will the one gap be enough for the power I will be delivering?

>

> I appoligize for the long post but I need some answers before I can get

> started.

>

> Thanks again

> Rusty

Hi Rusty,

I bought a box of that 6 mil 10x100 feet and returned it after I got a

good look at it. It was full of thin spots, creases and wrinkles. If you

could find it on a roll you would be much better off. If not, you might

concider laying it out on the driveway or something in the sun to relax

some of the imperfections. Or maybe I just got a bum roll. Others may

tell you they had great success with it. If it is all you can get, and

all the cutting of long lines works for you, then 6 mil is nice to have.

I don't mean to discourage you. I hope yours is better than the one I

got.

Before you build that RQ gap with the muffin fans you may want to

investigate a vacuum gap. You have a hefty bank of neons there. I just

built the RQ gap and am poised to build a vacuum gap, with a few

enhancements already. Quenching on a well designed vacuum gap will be

better.

You really got a nice bunch of neons there. Of my 5 15kV neons, I was

only able to scare up one 60mA. The rest are 30's. In fact, of all the

12kV's I got (8 that are presently working) only one of those is a 60.

Not that I am jealous, but I would feel better if you just shipped them

all to me....   :))

Have you been on the list long enough to see Father Tom's description of

his multi stage rolled capacitor? It is worth a look, and you can save

big bucks using heavy duty foil instead of flashing. Just a thought.

How big a secondary are you planning??

Michael Smith

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: Glass Capacitor

Date: Fri,  8 Aug 1997 16:41:22 GMT

Organization: The Energy Machine Information System 504-734-0526

From: Shane Randle <randlec@scnc.addison.k12.mi.us>

Date: Fri, 08 Aug 1997 12:41:22 -0400

Subject: Re: Glass Capacitor

Tesla-2 wrote:

>

> From: cjhill@hills.u-net.com (CJ Hill)

> Date: Fri, 8 Aug 1997 15:08:47 +0100

>

>     I'm also interested in what ages everybody on this list is so I can

>     get an idea what types of people are out there coiling.

>

>     Nathan_Bonsal@Intuit.com

>

Well, Nathan, there is a great deal of variety in the ages of the list

members here. I'm 16, and not the only teenager on the list, and there

are other members well into their 40's and 50's, and probably every age

in between. The members also come from a variety of locations, ranging

from the US to Europe (I think?) all the way to South Africa. We also

range from college students and teachers with serious scientific

backgrounds, to enthusiastic hobbyists (like me) who just do this for

the thrill of creating our own lightning.

-Shane Randle

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: Introduction of a new member

Cc:

Bcc:

X-Attachments:

At 01:42 AM 8/8/97 GMT, you wrote:

>From: LINEMAN23K@aol.com

>Date: Thu, 7 Aug 1997 21:42:06 -0400 (EDT)

>Subject: Introduction of a new member

>

>All,

>

>My name is Rusty and I am glad to be a new member of this list.  I think it

>is great that there are still people out there willing to donate their time

>helping others!!!  It would be a nice world if everyone were this generous.

Welcome aboard Rusty.

>

>I am in the design process of attempting to build my first Tesla coil and

>plan to go all out.  I have aquired four neon sign transformers, three are

>15KV@60mA and one is 15KV@30mA.  If I parallel them, will this give me >15KV@210mA?

>If so will the power be 3150 (Watts or VA)?  26.25A on the >primary? Will a 30A breaker >hold this?

Hold on there! All those neons paralleled are way too much power.

Especially for a first coil.

Start with your single 15kv/30. This will provide plenty of

power to start with and is capable of 18 inches of spark or

more. Once your coil is operating well then move up to a larger

cap size and your 15kv/60. Take it a step at a time. Start small

and work your way up.

My own 4 inch diameter coil gives a maximum (so far) of 45 inches

of spark with a single 12kv/90ma neon.

>Would somone please explain a transformer being "shunted" or

>"non-shunted"?

Neon sign transformers as well as oil burner ignition transformers

(OBIT) and some microwave oven transformers have magnetic shunts.

These shunts are metal blocks placed in the magnetic path

of the transformer primary winding. These blocks "shunt"

away the magnetic flux from the secondary windings. Only

enough magnetic flux is allowed to reach the secondary for

the rated secondary current to flow.

This means you can short circuit a neon transformer and

only the rated current will flow. You can short circuit

a neon indefinitely without harm. This makes them ideal

as Tesla coil power supplies.

>

>From the best of my calculations I need a .037uF cap. I plan to build (and

>parrallel) the rolled type cap and have found some 20"x50' flashing. Also

>found is 6mil poly 10'x100'.  I plan to use double thickness poly (12mils)

>24"x96" and leave a 2" border on the sides and a 3" border on the ends of the

>flashing 20"x93".  What will be my approximate value for C per cap?  Will the

>voltage rating be high enough?  What equation /s are needed to approximate

>this?

Way too big a cap. Again start small and work your way up.

Start with a single 15kv/30ma neon. This requires only a .0053 mfd

cap. 12 mils of poly is only good for about 4500 volts. You

need 60 mils minimum for 15kv.

The best way to get a good 60 mil .0053 cap is to build two

caps each .01 mfd with 30 mils of poly each. Run these in series

and you get .005 with the equivalent of 60 mils.

Its far easier to build two 7500 volt caps than a single 15kv.

If you are careful you can also use heavy duty aluminum foil

in rolled caps. It works well and makes a slightly more compact

unit.

When you move up to 15kv/60ma build a larger cap of .01 mfd.

Again build two larger units and put them in series. Two

.02 mfd caps in series will give you .01mfd.

>

>I intend to use the Richard Quick static gap w/muffin fan.

Thats fine. The cylinder gap popularized (not invented) by

Richard Quick is a good performer on neons.

> Do I need RF protection?

You should have some in line chokes to help keep RF energy out of

your transformers. Also a RF ac-line filter is recommended

on the primary side of your neons to keep RF out of the AC mains.

>Will the one gap be enough for the power I will be delivering?

Yes. The RQ gap or variants of it can handle several KVA.

Use up to 11 electrodes (10 gaps) for 15kv. I prefer a bit closer

spacing of .025 inch. That gives a .250 inch maximum gap.

That is plenty of gap and wont over stress your caps and neons.

TTYL!   Mike Hammer  mhammer@misslink.net

-----

Processed with Listserv v2.93 for Wildcat v4

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: Safety hazards and toroids

Cc:

Bcc:

X-Attachments:

At 08:46 PM 8/8/97 GMT, you wrote:

>From: James <057006@bud.cc.swin.edu.au>

>Date: Fri, 08 Aug 1997 13:46:54 -0700

>Subject: Re: Safety hazards??

>Organization: 057006

>

>Tesla-2 wrote:

<SNIP>

>

>But I'm actually living in a flat on the second floor!

>Does this qualify as good insulation from ground? I guess that

>there are wires under my floor though for the lighting & such on the

>floor below.

No being on the 2nd floor isnt isolation from ground.

Especially when dealing with very high voltage RF energy.

It will flow along all kinds of materials as it SEEKS ground.

>

>What's a good all round material to cover the floor with?

3 or 4 inches of neoprene rubber. Seriously, if you think

you must play with the streamers build yourself an isolated raised

platform to stand on.

>

> Also, I thought of (maybe) a good way to make a toroidal discharge

>terminal for a small coil. You know those cake tins that make rounded

>cakes? 2 of 'em one on top upside down on the other could be a neat

>terminal, but what of joining them so they don't spark? Is welding them

>together and filing/smoothing/polishing out of the question? People seem

>to use salad bowls just taped together but is this enough? the sharp

>edges are surely going to limit capacity even though they're

>insulated...

Salad bowls make lousy terminals. The sharp edge allows streamers

to break out at low voltage. You will get lots of little streamers.

Joining a couple rounded cake pans sounds like a decent idea.

Try joinging them together with aluminum tape. The tape

can be used to smooth over the ridge where they are joined.

I prefer the flexible aluminum dryer duct. It makes a wonderful

toroid and is available in many sizes. They look good and perform great.

My largest one is 19 X 6 inches and has allowed me to get up

to 45 inches of spark with a single 12kv/90 ma neon.

TTYL!   Mike Hammer  mhammer@misslink.net

-----

Processed with Listserv v2.93 for Wildcat v4

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re:muffin fan?

Cc:

Bcc:

X-Attachments:

At 03:00 AM 8/8/97 GMT, you wrote:

>From: BadBowTie9@aol.com

>Date: Thu, 7 Aug 1997 23:00:20 -0400 (EDT)

>Subject: Re: muffin fan

>

> Wait,......Now just what is a Muffin Fan?   I must have missed this one!

>Till Later MH?

>

>

Mark,

A muffin fan is also known as a boxer fan.

They are normally square and come in all different

sizes from tiny ones to clip on top of CPUs to quite

large 240 VAC ones.

There are quite common in surplus and common hamfest

items. I use a 12 VDC fan on my spark gap and have

a 120 VAC one blowing cooling air on my variac.

In looking for one for a spark gap get a good sized unit

that moves plenty of air.

TTYL!   Mike Hammer  mhammer@misslink.net

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: Safety hazards??

Date: Fri,  8 Aug 1997 20:34:29 GMT

Organization: The Energy Machine Information System 504-734-0526

From: James <057006@bud.cc.swin.edu.au>

Date: Fri, 08 Aug 1997 13:34:29 -0700

Subject: Re: Safety hazards??

Organization: 057006

Tesla-2 wrote:

>

> From: corr <corr@fullnet.net>

> Date: Thu, 07 Aug 1997 13:04:44 -0600

> Subject: Re: Safety hazards??

>

> > I've been sticking my hand in the sparks and holding a fluro tube with

> >the other, normally I understand that this is a very dangerous thing to

> >do (allowing current to pass through your body accross your heart) but

> >as I understand it, the high frequency HV from the TC exhibits 'Skin

> >effect' which prevents the current from penetrating more than a couple

> >of layers of your skin. Am I deluding myself? I'm still alive, but my

>

> That is correct as far as I know, but I learned something interesting the

> other day.  If a primary-to-secondary arc occurs whil your in the RF field,

> it will kill you dead instantly.  I had to rebuild my primary, and the newer

> primary had alot of primary to secodary acrs, so I have stopped sticking

> my hand in the field, that would be a good idea for you to do as well.

Corr,

WOAH! what a let down. I've been enjoying this little trick,

to the point where I go out to the TC every few hours for

"reenergization"! I'm pretty sure there's no danger of this type of

malfunction occurring though, at least with the power levels I'm using

at the moment. I've got the primary at least an inch &1/2 away from the

secondary at the base, I imagine an arc there will just travel to

ground? The top of the primary is further away as the thing tapers

outwards, but I'll probably have to be a lot more cautious when I get

the neon transformer and add more bottles (or some poly's now that their

construction has been so thoroughly detailed on the list) Also, if I

place a more attractive "load" than my hand in the field also, attached

to earth, would the RF energy dissipate through this rather than my

body? And if all else fails, if I'm standing on a good insulator and

have one hand in my pocket, and the "DEATH ARC" between the primary &

secondary occurrs, the usual, 'path of least resistance' thing should

save my speeding heart from overload, shouldn't it?

I'm VERY reluctant to give up ALL bodily interaction with my coil (pant

pant) but seriously, are there ways to  keep my "habit" without too much

risk?

James Hurl,

Australia.

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: Safety hazards??

Date: Fri,  8 Aug 1997 20:36:06 GMT

Organization: The Energy Machine Information System 504-734-0526

From: James <057006@bud.cc.swin.edu.au>

Date: Fri, 08 Aug 1997 13:36:06 -0700

Subject: Re: Safety hazards??

Organization: 057006

Tesla-2 wrote:

>

> From: Jochen Kronjaeger <Kronjaeg@Stud-Mailer.Uni-Marburg.DE>

> Date: Thu, 7 Aug 1997 21:20:34 +0200 (CES)

> Subject: Re: Safety hazards??

>

> > possibility of X-ray emmissions when I do the plasma ball thing putting

> > a normal light globe on top of the discharge terminal.

>

> There definitely no danger here. For X-Rays, youz ned not only high

> voltage but also a pretty good vacuum. Several orders of magnitude below

> the pressure in gas discharge tubes, plsama  globes etc.

>

>    Jochen

Thanks, at least that's something!

James Hurl, Australia.

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: Safety hazards??

Date: Fri,  8 Aug 1997 20:46:54 GMT

Organization: The Energy Machine Information System 504-734-0526

From: James <057006@bud.cc.swin.edu.au>

Date: Fri, 08 Aug 1997 13:46:54 -0700

Subject: Re: Safety hazards??

Organization: 057006

Tesla-2 wrote:

>

> From: Mike Hammer <mhammer@misslink.net>

> Date: Thu, 07 Aug 1997 15:15:20 -0500

> Subject: Re: Safety hazards??

>

> At 07:04 PM 8/7/97 GMT, you wrote:

> >From: corr <corr@fullnet.net>

> >Date: Thu, 07 Aug 1997 13:04:44 -0600

> >Subject: Re: Safety hazards??

> >

> >> I've been sticking my hand in the sparks and holding a fluro tube with

> >>the other, normally I understand that this is a very dangerous thing to

> >>do (allowing current to pass through your body accross your heart) but

> >>as I understand it, the high frequency HV from the TC exhibits 'Skin

> >>effect' which prevents the current from penetrating more than a couple

> >>of layers of your skin. Am I deluding myself? I'm still alive, but my

> >

> >That is correct as far as I know, but I learned something interesting the

> >other day.  If a primary-to-secondary arc occurs whil your in the RF field,

> >it will kill you dead instantly.  I had to rebuild my primary, and the newer

> >primary had alot of primary to secodary acrs, so I have stopped sticking

> >my hand in the field, that would be a good idea for you to do as well.

> >

> >

>

> That type of event will not necessarily kill you instantly

> but it will surely diminish your chances of becoming an "OLD

> GEEZER" coiler.

>

> Even if you do not have problems with primary/secondary

> arcing the power level in medium to large size coils

> with large terminal capacitances can be lethal.

>

> If you simply must play Russian Roulette with streamers

> isolate your self very well from ground. Standing

> on the concrete floor in your sneakers doesnt qualify.

>

> Something like a plastic milk crate would be the minimum

> acceptable. Or play it safe and watch the streamers from

> a distance.

>

> Live long and prosper.

>

> Mike Hammer

> mhammer@misslink.net

>

Guess I should have read all the mail first, then replied!

But I'm actually living in a flat on the second

floor! does this qualify as good insulation from ground? I guess that

there are wires under my floor though for the lighting & such on the

floor below.

What's a good all round material to cover the floor with?

Also, I thought of (maybe) a good way to make a toroidal discharge

terminal for a small coil. You know those cake tins that make rounded

cakes? 2 of 'em one on top upside down on the other could be a neat

terminal, but what of joining them so they don't spark? Is welding them

together and filing/smoothing/polishing out of the question? People seem

to use salad bowls just taped together but is this enough? the sharp

edges are surely going to limit capacity even though they're

insulated...

Thanks,

James Hurl, Australia.

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re:muffin fan? VARIAC?

Date: Fri,  8 Aug 1997 22:33:03 GMT

Organization: The Energy Machine Information System 504-734-0526

From: corr <corr@fullnet.net>

Date: Fri, 08 Aug 1997 16:33:03 -0600

Subject: Re:muffin fan? VARIAC?

>a 120 VAC one blowing cooling air on my variac.

This may be a stupid question, but what is a variac

used for as far as coiling goes?

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re:muffin fan? VARIAC?

Cc:

Bcc:

X-Attachments:

At 10:33 PM 8/8/97 GMT, you wrote:

>From: corr <corr@fullnet.net>

>Date: Fri, 08 Aug 1997 16:33:03 -0600

>Subject: Re:muffin fan? VARIAC?

>

>>a 120 VAC one blowing cooling air on my variac.

>

>This may be a stupid question, but what is a variac

>used for as far as coiling goes?

>

>

Jeff,

A variac is a variable transformer. You put in 120 VAC

from your wall socket and you get a smoothly adjustable

0-120VAC output.

Feeding this adjustable voltage to your neons allows you

to use just the amount of power you want.

Not so important when using a single 30ma neon when

you need all the power you can get but when running

several neons in parallel its indispensable.

Also allows you to bring the power up slowly instead

of full on all at once. Saves wear and tear on neons

and caps.

Some variacs can be wired for 0-140 VAC output from

120 volts input. Running 140 volts AC to your neons

gives you a little more power and doesnt seem to be

harmful. I do it all the time.

240 VAC variacs also exist.

TTYL!   Mike Hammer  mhammer@misslink.net

From: tesla-2@emachine.com (Tesla-2)

Subject: Re:muffin fan? VARIAC?

Date: Sat,  9 Aug 1997 05:51:33 GMT

Organization: The Energy Machine Information System 504-734-0526

T>This may be a stupid question, but what is a variac

T>used for as far as coiling goes?

Not a stupid question at all!  That's what this list is for - to get

answers to all sorts of questions.

A Variac is the trade name for a brand of adjustable auto

transformer. This device has only one winding, instead of two windings

found on must standard transformers.  They consist of a torroidal iron

core on which is wound a single layer winding of copper wire wrapped

through the core.  The AC line is connected to the ends of the winding.

A movable tap is arranged so that it may be rotated around the outside

of the coil winding and is able to connect to any turn on the coil.

There are two main uses for Variacs in coiling.

The first is to provide a variable source of AC power for your power

transformer. By using the adjustability of the Variac, you can easily

adjust the voltage output of your transformer driving the spark gap.

This allows you to tune up the coil by carefully increasing the power a

bit at a time as you make adjustments.  When used in this manner, you

connect the main AC power line feed across the ends of the Variac

winding.  You then connect your spark gap power transformer primary

between the adjustable tap and one of the connections where the AC power

is connected.  By adjusting the tap position on the Variac coil, you may

apply any desired voltage between 0 and 100% of the AC line voltage to

your spark gap transformer.

The second is to act as a current limiter by using the Variac as an

adjustable inductor in series with the primary of your transformer.

When used in this manner, you make connections to one end of the Variac

winding and another connection to the adjustable tap.  The third

connection is not used.  By moving the tap on the Variac winding, you

include more or less turns in the coil in the primary circuit of your

spark gap transformer.  By using the inductive reactance of the

Variac connected in this manner, you may easily adjust the maximum

current which can flow through your transformer at any input voltage

setting.

Obviously, you need one Variac to vary the voltage, and another to

control the current limit.  That's why Variacs are in such great demand

by serious coilers.

Ralph

---

A miser is hard to live with, but makes a fine ancestor.

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: Safety hazards and toroids

Cc:

Bcc:

X-Attachments:

At 09:48 PM 8/9/97 GMT, you wrote:

>>

>> I prefer the flexible aluminum dryer duct. It makes a wonderful

>> toroid and is available in many sizes. They look good and perform great.

>>

>> My largest one is 19 X 6 inches and has allowed me to get up

>> to 45 inches of spark with a single 12kv/90 ma neon.

>>

>> TTYL!   Mike Hammer  mhammer@misslink.net

>> Mike Hammer

>> mhammer@misslink.net

>>

>

>

>Mike,

>  Pardon my ignorance, but what is this stuff normally used for

>(flex. alum .dr duct?), does it come in toroid form already, or are you

>talking about those duct elbows from heating systems?

This is flexible ducting normally used for clothes

dryer vents. Very common in hardware and home improvement

stores.

You take the duct and tape together the ends with aluminum

tape forming a ring.

You can fill the ring with a disk of wood or foam covered with

foil. Or use 2 pie pans back to back. Bolt the pans together

and draw them together. They will clamp down on the duct ring

and make a solid unit.

>

>Also, I'm interested to see what other people use for their toroids,

>such a hard thing to come accross ready made!!

Ive used foil covered styrofoam rings. But the flexible dryer duct

is the best.

> And is it O.K. to penetrate my sealed coil form with an insulated

>threaded rod to support my toroid/sphere? Is this going to act like a

>core inside the loose top turns leading to the discharge terminal? How

>do others do this?

I use a 1/4 20 bolt in the top of my secondary but place it outside

the sealed form. Mount it in a cap that fits on the end of your form.

TTYL!   Mike Hammer  mhammer@misslink.net

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: muffin fan? VARIAC?

Date: Sat,  9 Aug 1997 11:46:14 GMT

Organization: The Energy Machine Information System 504-734-0526

From: Mike Huff <msnock@theonramp.net>

Date: Sat, 09 Aug 1997 07:46:14 -0400

Subject: Re: muffin fan? VARIAC?

Tesla-2 wrote:

> From: corr <corr@fullnet.net>

> Date: Fri, 08 Aug 1997 16:33:03 -0600

> Subject: Re:muffin fan? VARIAC?

>

> >a 120 VAC one blowing cooling air on my variac.

>

> This may be a stupid question, but what is a variac

> used for as far as coiling goes?

>

A variac is a variable transformer, they are also called "powerstats".

As far as coiling goes, they are used to bring up power level slowly or

to limit power for tuning purposes.  They aren't really necessary but

are a good thing to have to help prevent damages to capacitors and

transformers if they coil isn't in good tune.  I don't use variacs,

rather I sometimes use a 7.5kV @ 30mA neon for tuning, but most of the

time I just estimate a tune and let it rip.

Mike

--

---------------------------------------------------------------

| SNOCK'S HIGH VOLTAGE PAGE                                   |

| http://www.geocities.com/CapeCanaveral/Lab/5322/hv2.html    |

| Picures and schematics of HV devices, arcs, sparks, Jacob's |

| ladders, etc.                                               |

---------------------------------------------------------------

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: Introduction of a new member

Date: Sat,  9 Aug 1997 17:27:48 GMT

Organization: The Energy Machine Information System 504-734-0526

From: LINEMAN23K@aol.com

Date: Sat, 9 Aug 1997 13:27:48 -0400 (EDT)

Subject: Re: Introduction of a new member

>If I parallel them, will this give me

>15KV@210mA?  If so will the power be 3150 (Watts or VA)?  26.25A onthe

>primary? Will a 30A breaker hold this?

Hold on there! All those neons paralleled are way too much power.

Especially for a first coil.

Too much power?

Are you saying that a 30A breaker would trip at this power?

What about my calculations, were they correct?

Start with your single 15kv/30. This will provide plenty of

power to start with and is capable of 18 inches of spark or

more. Once your coil is operating well then move up to a larger

cap size and your 15kv/60. Take it a step at a time. Start small

and work your way up.

Was this advice for safety?

>From the best of my calculations I need a .037uF cap.

Way too big a cap. Again start small and work your way up.

Was this that my calculations were wrong or just too big of a cap for the size

coil that you recomend?

Start with a single 15kv/30ma neon. This requires only a .0053 mfd

cap. 12 mils of poly is only good for about 4500 volts. You

need 60 mils minimum for 15kv.

Thanks, I was confused on this issue.  Would 90 mils be even better for 15KV?

What would that do to the capacitance value, lower it?

When you move up to 15kv/60ma build a larger cap of .01 mfd.

Again build two larger units and put them in series. Two

.02 mfd caps in series will give you .01mfd.

Let me see if I understand.  If I series two caps I get half the C but double

the voltage rating?  If I parallel two caps, do I double the C and get half

the voltage rating?

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: Introduction of a new member

Date: Sat,  9 Aug 1997 17:55:17 GMT

Organization: The Energy Machine Information System 504-734-0526

From: LINEMAN23K@aol.com

Date: Sat, 9 Aug 1997 13:55:17 -0400 (EDT)

Subject: Re: Introduction of a new member

In a message dated 97-08-08 15:24:37 EDT, you write:

<< Subj:        Re: Introduction of a new member

Date:  97-08-08 15:24:37 EDT

From:  tesla-2@emachine.com (Tesla-2)

To:    lineman23k@aol.com

From: "Michael Smith" <md.smith@usa.net>

Date: Fri, 8 Aug 1997 11:04:27 -0500

Subject: Re: Introduction of a new member

----------

> From: Tesla-2 <tesla-2@emachine.com>

> To: md.smith@usa.net

> Subject: Introduction of a new member

> Date: Thursday, August 07, 1997 8:42 PM

>

> From: LINEMAN23K@aol.com

> Date: Thu, 7 Aug 1997 21:42:06 -0400 (EDT)

> Subject: Introduction of a new member

>

> All,

>

> My name is Rusty and I am glad to be a new member of this list.  I think it

> is great that there are still people out there willing to donate their time

> helping others!!!  It would be a nice world if everyone were this generous.

>

> I am in the design process of attempting to build my first Tesla coil and

> plan to go all out.  I have aquired four neon sign transformers, three are

> 15KV@60mA and one is 15KV@30mA.

> If I parrallel them, will this give me 15KV@210mA?  If so will

> the power be 3150 (Watts or VA)?  26.25A on the primary? Will a 30A breaker

> hold this?  Would somone please explain a transformer being "shunted" or

> "non-shunted"?

>

> From the best of my calculations I need a .037uF cap. I plan to build (and

> parrallel) the rolled type cap and have found some 20"x50' flashing. Also

> found is 6mil poly 10'x100'.  I plan to use double thickness poly (12mils)

> 24"x96" and leave a 2" border on the sides and a 3" border on the ends of the

> flashing 20"x93".  What will be my approximate value for C per cap?  Will the

> voltage rating be high enough?  What equation /s are needed to approximate

> this?

>

> I intend to use the Richard Quick static gap w/muffin fan.  Do I need RF

> protection?

> Will the one gap be enough for the power I will be delivering?

>

> I appoligize for the long post but I need some answers before I can get

> started.

>

> Thanks again

> Rusty

Hi Rusty,

I bought a box of that 6 mil 10x100 feet and returned it after I got a

good look at it. It was full of thin spots, creases and wrinkles. If you

could find it on a roll you would be much better off. If not, you might

concider laying it out on the driveway or something in the sun to relax

some of the imperfections. Or maybe I just got a bum roll. Others may

tell you they had great success with it. If it is all you can get, and

all the cutting of long lines works for you, then 6 mil is nice to have.

I don't mean to discourage you. I hope yours is better than the one I

got.

Before you build that RQ gap with the muffin fans you may want to

investigate a vacuum gap. You have a hefty bank of neons there. I just

built the RQ gap and am poised to build a vacuum gap, with a few

enhancements already. Quenching on a well designed vacuum gap will be

better.

You really got a nice bunch of neons there. Of my 5 15kV neons, I was

only able to scare up one 60mA. The rest are 30's. In fact, of all the

12kV's I got (8 that are presently working) only one of those is a 60.

Not that I am jealous, but I would feel better if you just shipped them

all to me....   :))

Have you been on the list long enough to see Father Tom's description of

his multi stage rolled capacitor? It is worth a look, and you can save

big bucks using heavy duty foil instead of flashing. Just a thought.

How big a secondary ore you planning??

Michael Smith

>>

Michael,

Thanks for the info on the poly.  On second thought I think I will check on

some plastic manufacturers for some 30 mil poly.

As for the xformers, I gave 25 bucks apiece for the 15KV@60mA and $15 for the

15KV@30mA.  Was it a deal or did I get the short end of the stick?

I have just gotten on the list so I do not know who Father Tom is but I would

like to see the post on his rolled caps.  Saving big bucks sounds like a

winner.  :-)

Please tell me more about the vacuum gap. Sounds like I might want to try it.

I am planning a secondary using 8" sealed PVC 34" long wound with 1000 turns

of 20ga magnet wire. Approx. 125kHz.  What do you think as I am still new at

this?

Rusty

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: Safety hazards??

Cc:

Bcc:

X-Attachments:

At 09:27 PM 8/9/97 GMT, you wrote:

>From: James <057006@bud.cc.swin.edu.au>

>Date: Sat, 09 Aug 1997 14:27:46 -0700

>Subject: Re: Safety hazards??

>Organization: 057006

<SNIP>

>

>O.K,

>     Here's a description of my admittedly crude "coil room"

>

>The walls and floor are tiled, with ceramic (It's the laundry, right up

>the other end(about 15 metres and several walls from my t.v., computer

>etc...)) the windows are blocked up with foam and on them as well as the

>walls is a layer of alfoil. This is connected to the normal wall ground

>- ( I am attempting a kind of room wide sheilding here to prevent the

>kind of RFI you are talking about).

The type of shielding you are talking about is

termed a Faraday cage and is totally unnecessary.

Especially at the low power levels you are running at.

Real radiated RFI from our coils goes only a short

distance from a properly operating and tuned coil.

The energy is dissipated as spark.

However RF conduction can be a problem. This is RF flowing

back into the AC mains. For this reason do not use the

AC mains ground for your coil or coil components.

Use a good quality RF line filter to keep RF from traveling

back to the AC mains.

I use two such filters. One unit is between my neons and the

variac and the second is on the main AC input to my control

box.

I run my coil in the basement of my home at up to 1.5kva

and 45 inch sparks and had minimal problems with RFI.

Except for my cordless phone.

My amateur radio equipment causes far more RFI problems

than my coil.

> The Tesla ground however is not connected to the wall earth, but

> is led outside the laundry via a heavy duty cable & gap under the

> back door (the stairs leading up to this are locked up tight to

> prevent little Johnny down at no#1 from excersizing

>his inquisitive little mind around the earth cable) to a VERY large

>copper water pipe running down my side of the building and deep into the

>ground.

Your ground certainly sounds adequate to me.

<SNIP>

>Any realistic suggestions as to how I can make things safer given my

>circumstances?

>

> I mentioned before that my coil is Ignition coil supplied, and doesn't

>develop sparks of more than 5 - 6 inches, does this not make it slightly

>safer than the beasts you guy's/girls make?

Yes somewhat. The real hazard in coils is 60 hz (or 50) energy.

This energy is lethal. The output streamers are much less

hazardous due to them being RF. The RF skin effect allows

the energy to crawl over the surface of your body with little

effect.

However disruptive (spark gap) coils often have a considerable

60 hz component in the secondary output. Its this energy that

causes shocks and is dangerous.

>

>                                               Thanks,

>                                                        James Hurl,

>Australia.

>

>PS: What sort of laws regulate the firing of TC's in the US/UK &

>anywhere else? I've got the feeling that the authorities in my city

>wouldn't know what the hell the thing was?!

Well in the US spark gap transmitters were made illegal

in the early 1930s. Unlicensed RF transmissions are also illegal.

(Except for certain frequencies and power levels)

When well tuned for spark our coils are probably not

violating any laws.

TTYL!   Mike Hammer  mhammer@misslink.net

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: Introduction of a new member

Cc:

Bcc:

X-Attachments:

At 05:27 PM 8/9/97 GMT, you wrote:

>From: LINEMAN23K@aol.com

>Date: Sat, 9 Aug 1997 13:27:48 -0400 (EDT)

>Subject: Re: Introduction of a new member

>

>

>

>

>   >If I parallel them, will this give me

>   >15KV@210mA?  If so will the power be 3150 (Watts or VA)?  26.25A on

>   the primary? Will a 30A breaker hold this?

>

>      Hold on there! All those neons paralleled are way too much power.

>      Especially for a first coil.

>

>Too much power?

>Are you saying that a 30A breaker would trip at this power?

>What about my calculations, were they correct?

What I meant is this is too much power for a coil. Especially

if its your first coil. It sometimes takes considerable

experimentation to get that first coiled tuned correctly.

This is much easier done at lower power levels.

And you dont need huge power levels for big sparks.

My own 4 inch diameter coil provides up to 45 inches

of spark with a single 12kv/90ma neon and a .01 mfd cap.

All that power into a mistuned coil will only cause component

failures. And it will take a very good gap system to process

that amount of power.

Start with a single neon. Tuning will be easier and simple

gaps will suffice. Add more power as your experience and capabilities

allow.

Neons are notoriusly inefficent. So will probably draw considerably

more than the 26.25 amps you calculated above. In fact a single 15kv/60

ma neon can draw nearly 10 amps.

>

>      Start with your single 15kv/30. This will provide plenty of

>      power to start with and is capable of 18 inches of spark or

>      more. Once your coil is operating well then move up to a larger

>      cap size and your 15kv/60. Take it a step at a time. Start small

>      and work your way up.

>

>Was this advice for safety?

No not really. More to insure some measure of success.

>

>      >From the best of my calculations I need a .037uF cap.

>

>      Way too big a cap. Again start small and work your way up.

>

>Was this that my calculations were wrong or just to big of a cap for the size

>coil that you recomend?

Your calculations are fine. But a .037 cap is a quite large

cap and will give you a very low resonant frequency unless

teamed with a really small primary.

Better results are achieved with a somewhat smaller cap and large

primary coil. Many coilers running pole transformers use only

.025 mfd caps.

Dont worry so much about impedance matching your cap to your transformers.

Use whatever capacitance it takes to resonate your primary at

the same frequency as your secondary with most of your primary

turns in. Then use enough transformer to charge that cap.

Big primary/small cap gives big performance at lower power level.

>

>      Start with a single 15kv/30ma neon. This requires only a .0053 mfd

>      cap. 12 mils of poly is only good for about 4500 volts. You

>      need 60 mils minimum for 15kv.

>

>Thanks, I was confused on this issue.  Would 90 mils be even better for 15KV?

>What would that do to the capacitance value, lower it?

Well if using a single thick layer of poly even 90 wont be good enough.

But if you use many thinner layers of poly 60 mils is more than adequate

for 15 kv. Especially if using several caps in series. This spreads the

load out over many dielectric layers.

I am now running a pair of .03 mfd caps in series. These caps

a rolled units with 24 mils (4 X 6 mils) of poly in each.

Running them in series give the equivalent of 48 mils and

.015 mfd. It has stood up well to 12kv neons.

Increasing the thickness of the dielectric lowers the resulting capacitance

and increases the voltage rating.

>

>      When you move up to 15kv/60ma build a larger cap of .01 mfd.

>      Again build two larger units and put them in series. Two

>      .02 mfd caps in series will give you .01mfd.

>

>Let me see if I understand.  If I series two caps I get half the C but double

>the voltage rating?

Yes if the caps are identical.

> If I parallel two caps, do I double the C and get half

>the voltage rating?

No. When paralled the capacitances add but the voltage rating

is equal to the rating of the weakest cap.

>I am planning a secondary using 8" sealed PVC 34" long wound with 1000 turns

>of 20ga magnet wire. Approx. 125kHz.  What do you think as I am still new at

>this?

This is a quite large coil. You may consider making a smaller

one first to gain experience. Large coils are just that much more

difficult to get going. Try a 4 incher first then move on to the 8.

If you want to go with the 8 incher a coil this size works best at

a H/D ( height/diameter) ratio of 3 to 1. A 6 inch is good

at 4 to 1. A 4 incher at 5 to 1.

8 inch PVC is usually 8.5 inches so that works out to

25.5. Round off to 26 inches. Wound with about 780 turns

of your AWG 20 wire it would be real potent. Resonant

frequency about 234 khz.

I favor somewhat less turns and a higher resonant frequency

on my secondary coils. Lowering that higher frequency back down

with a large toroid gives you much better spark.

A toroid about 20 X 6 would be great. With a .01 mfd cap

and 15kv/60ma neon it should perform well.

TTYL!   Mike Hammer  mhammer@misslink.net

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: Safety hazards??

Date: Sat,  9 Aug 1997 21:27:46 GMT

Organization: The Energy Machine Information System 504-734-0526

From: James <057006@bud.cc.swin.edu.au>

Date: Sat, 09 Aug 1997 14:27:46 -0700

Subject: Re: Safety hazards??

Organization: 057006

Tesla-2 wrote:

>

> From: Nathan_Bonsal@intuit.com (Nathan Bonsal)

> Date: Fri, 8 Aug 1997 08:24:48 -0700

> Subject: Re[2]: Safety hazards??

>

> If you're living in a flat on the second floor, How did you accomplish your

> ground system? I hope you're not using the ground supplied at the outlets.

> Did you construct a ground system?

>

> If not, stop where you are before you kill someone's TV or computer. Then build

> a good ground. The problem with being on the second floor is safety. If you

> build a ground connnection, and run a wire or cable to it, this cable could

> present a high voltage danger to children playing around the building. So your

> ground is going to have to be very deep and very conductive. An almost flawless

> ground connection is going to have to be made in any case. For that reason I

> don't recommend Apartment Coiling. 1. Annoys the neighbors with high levels of

> RF interference. 2. you share a circuit with others who may turn you in. 3. In

> that close an environment it is impossible to control the influence of the

> electric field around the coil. 4. Your other electronic components are also on>

> your flat and could be fried to a crisp. 5. It probably violates your lease and

> could get you evicted.

>

> Please do something about grounding. Someone could get hurt.

>

> Nathan Bonsal

>

O.K,

Here's a description of my admittedly crude "coil room"

The walls and floor are tiled, with ceramic (It's the laundry, right up

the other end(about 15 metres and several walls from my t.v., computer

etc...)) the windows are blocked up with foam and on them as well as the

walls is a layer of alfoil. This is connected to the normal wall ground

- ( I am attempting a kind of room wide sheilding here to prevent the

kind of RFI you are talking about). The Tesla ground however is not

connected to the wall earth, but is led outside the laundry via a heavy

duty cable & gap under the back door (the stairs leading up to this are

locked up tight to prevent little Johnny down at no#1 from excersizing

his inquisitive little mind around the earth cable) to a VERY large

copper water pipe running down my side of the building and deep into the

ground. I fire my coil early in the morning (about 4:00am) when just

about everyone else in the flats are sound asleep, and only in short

bursts of less than a minute because I have to ventilate the room by

opening and swinging the back door to & fro. It can barely be heard

outside due to all the foam insulation and there's no-one living in the

flat below, the owner is reflooring (bastard) and if anyone is annoying

the rest of the folks in my block it's him! I realize that it's not an

ideal setup, but I can't really just move somewhere else, the rent goes

up around the area I'm living in every 6 months and I'm already broke

'cause of it.

Any realistic suggestions as to how I can make things safer given my

circumstances?

I mentioned before that my coil is Ignition coil supplied, and doesn't

develop sparks of more than 5 - 6 inches, does this not make it slightly

safer than the beasts you guy's/girls make?

Thanks,

James Hurl,

Australia.

PS: What sort of laws regulate the firing of TC's in the US/UK &

anywhere else? I've got the feeling that the authorities in my city

wouldn't know what the hell the thing was?!

(although I'm a bit worried about a new branch just starting up called

the "Spectrum police" who may be more than just a bit aware of these and

other forms of EM/RF "pollution")

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: Safety hazards and toroids

Date: Sat,  9 Aug 1997 21:48:17 GMT

Organization: The Energy Machine Information System 504-734-0526

From: James <057006@bud.cc.swin.edu.au>

Date: Sat, 09 Aug 1997 14:48:17 -0700

Subject: Re: Safety hazards and toroids

Organization: 057006

Tesla-2 wrote:

>

> From: Mike Hammer <mhammer@misslink.net>

> Date: Fri, 08 Aug 1997 12:10:50 -0500

> Subject: Re: Safety hazards and toroids

>

> At 08:46 PM 8/8/97 GMT, you wrote:

> >From: James <057006@bud.cc.swin.edu.au>

> >Date: Fri, 08 Aug 1997 13:46:54 -0700

> >Subject: Re: Safety hazards??

> >Organization: 057006

> >

> >What's a good all round material to cover the floor with?

>

> 3 or 4 inches of neoprene rubber. Seriously, if you think

> you must play with the streamers build yourself an isolated raised

> platform to stand on.

>

> >

> > Also, I thought of (maybe) a good way to make a toroidal discharge

> >terminal for a small coil. You know those cake tins that make rounded

> >cakes? 2 of 'em one on top upside down on the other could be a neat

> >terminal, but what of joining them so they don't spark? Is welding them

> >together and filing/smoothing/polishing out of the question? People seem

> >to use salad bowls just taped together but is this enough? the sharp

> >edges are surely going to limit capacity even though they're

> >insulated...

>

> Salad bowls make lousy terminals. The sharp edge allows streamers

> to break out at low voltage. You will get lots of little streamers.

>

> Joining a couple rounded cake pans sounds like a decent idea.

> Try joinging them together with aluminum tape. The tape

> can be used to smooth over the ridge where they are joined.

>

> I prefer the flexible aluminum dryer duct. It makes a wonderful

> toroid and is available in many sizes. They look good and perform great.

>

> My largest one is 19 X 6 inches and has allowed me to get up

> to 45 inches of spark with a single 12kv/90 ma neon.

>

> TTYL!   Mike Hammer  mhammer@misslink.net

> Mike Hammer

> mhammer@misslink.net

>

Mike,

Pardon my ignorance, but what is this stuff normally used for

(flex. alum .dr duct?), does it come in toroid form already, or are you

talking about those duct elbows from heating systems?

Also, I'm interested to see what other people use for their toroids,

such a hard thing to come accross ready made!!

And is it O.K. to penetrate my sealed coil form with an insulated

threaded rod to support my toroid/sphere? Is this going to act like a

core inside the loose top turns leading to the discharge terminal? How

do others do this?

At the moment mine is a plastic funnel upside down securely taped to the

top cap on the form. There's a small screw attaching the end of the coil

to the terminal, and the remaining end of the screw is pushed into the

small hole in the funnel, the screw only inside one of the top turns.

James Hurl, Australia.

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: spark gaps

Date: Mon, 11 Aug 1997 01:13:47 GMT

Organization: The Energy Machine Information System 504-734-0526

From: watkier@earlham.edu

Date: Sun, 10 Aug 1997 20:13:47 EST

Subject: spark gaps

What is there to know about spark gaps?  Do they serve any particular

purpose besides isolating components?  Does the distance of the spark gap

make any difference?  And why rotary spark gaps...what good does that do?

-Erik

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Toroids

Date: Mon, 11 Aug 1997 11:02:59 GMT

Organization: The Energy Machine Information System 504-734-0526

From: James <057006@bud.cc.swin.edu.au>

Date: Mon, 11 Aug 1997 21:02:59 +1000

Subject: Toroids

Organization: 057006

Tesla-2 wrote:

>

> From: Mike Hammer <mhammer@misslink.net>

> Date: Sat, 09 Aug 1997 01:24:17 -0500

> Subject: Re: Safety hazards and toroids

>

> At 09:48 PM 8/9/97 GMT, you wrote:

>

> >>

> >> I prefer the flexible aluminum dryer duct. It makes a wonderful

> >> toroid and is available in many sizes. They look good and perform great.

> >>

> >> My largest one is 19 X 6 inches and has allowed me to get up

> >> to 45 inches of spark with a single 12kv/90 ma neon.

> >>

> >> TTYL!   Mike Hammer  mhammer@misslink.net

> >> Mike Hammer

> >> mhammer@misslink.net

> >>

> >

> >

> >Mike,

> >        Pardon my ignorance, but what is this stuff normally used for

> >(flex. alum .dr duct?), does it come in toroid form already, or are you

> >talking about those duct elbows from heating systems?

>

> This is flexible ducting normally used for clothes

> dryer vents. Very common in hardware and home improvement

> stores.

>

> You take the duct and tape together the ends with aluminum

> tape forming a ring.

> You can fill the ring with a disk of wood or foam covered with

> foil. Or use 2 pie pans back to back. Bolt the pans together

> and draw them together. They will clamp down on the duct ring

> and make a solid unit.

>

> >

> >Also, I'm interested to see what other people use for their toroids,

> >such a hard thing to come accross ready made!!

>

> Ive used foil covered styrofoam rings. But the flexible dryer duct

> is the best.

>

> > And is it O.K. to penetrate my sealed coil form with an insulated

> >threaded rod to support my toroid/sphere? Is this going to act like a

> >core inside the loose top turns leading to the discharge terminal? How

> >do others do this?

>

> I use a 1/4 20 bolt in the top of my secondary but place it outside

> the sealed form. Mount it in a cap that fits on the end of your form.

>

> TTYL!   Mike Hammer  mhammer@misslink.net

> Mike Hammer

> mhammer@misslink.net

>

Thanks for that Mike, but I guess we don't have that stuff over here in

Australia. Last night I made a toroid form out of wood. It's one main

ring with an OD of 90mm and inner of 60, 12 wooden circles of 60mm diam

slot onto the clock positions around the ring... took a while to cut out

all the slots, but it's pretty strong and I've got some heavy duty

alfoil to cover it with. Sound O.K ?

James Hurl, Australia.

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: Safety hazards??

Date: Mon, 11 Aug 1997 11:18:53 GMT

Organization: The Energy Machine Information System 504-734-0526

From: James <057006@bud.cc.swin.edu.au>

Date: Mon, 11 Aug 1997 21:18:53 +1000

Subject: Re: Safety hazards??

Organization: 057006

<CHOP>

> >Any realistic suggestions as to how I can make things safer given my

> >circumstances?

> >

> > I mentioned before that my coil is Ignition coil supplied, and doesn't

> >develop sparks of more than 5 - 6 inches, does this not make it slightly

> >safer than the beasts you guy's/girls make?

>

> Yes somewhat. The real hazard in coils is 60 hz (or 50) energy.

> This energy is lethal. The output streamers are much less

> hazardous due to them being RF. The RF skin effect allows

> the energy to crawl over the surface of your body with little

> effect.

>

> However disruptive (spark gap) coils often have a considerable

> 60 hz component in the secondary output. Its this energy that

> causes shocks and is dangerous.

Are there any ways to make ABSOLUTELY sure that this unwanted 50/60Hz

energy does not get in to my secondary?

> >PS: What sort of laws regulate the firing of TC's in the US/UK &

> >anywhere else? I've got the feeling that the authorities in my city

> >wouldn't know what the hell the thing was?!

>

> Well in the US spark gap transmitters were made illegal

> in the early 1930s. Unlicensed RF transmissions are also illegal.

> (Except for certain frequencies and power levels)

>

> When well tuned for spark our coils are probably not

> violating any laws.

>

> TTYL!   Mike Hammer  mhammer@misslink.net

>

> Mike Hammer

> mhammer@misslink.net

I'm interested to know whether you or anyone else on this list has

attempted any Tesla coil powered radio transmissions,(on the deregulated

frequencies and power levels of course...) using it for this rather than

to create sparks? Also, has anyone tried changing the spark gap to some

sort of more silent unit which you could modulate using low power audio

signals?..what I'm wondering here is whether the quieter brush discharge

you get with just a bare wire (no capacity) at the top of the coil can

make enough of a disturbance in the air to produce audio waves without

the need of a loudspeaker cone (I've done the ion powered rotor thing

and it makes me think that this is possible somehow).

James Hurl, Australia.

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: spark gaps

Cc:

Bcc:

X-Attachments:

At 01:13 AM 8/11/97 GMT, you wrote:

>From: watkier@earlham.edu

>Date: Sun, 10 Aug 1997 20:13:47 EST

>Subject: spark gaps

>

>

>

>What is there to know about spark gaps?  Do they serve any particular

>purpose besides isolating components?  Does the distance of the spark gap

>make any difference?  And why rotary spark gaps...what good does that do?

>

>-Erik

>

Erik,

Thats a very big and important question. The spark gap is one

of the most important components in a Tesla coil and the one most

often poorly implemented. Beginners often neglect this very important

component. Coilers place far too much emphasis on their secondary coils.

The spark gap is the brains of a Tesla coil. Think of it as a high

power high speed switch. No other device can switch such high power

levels as quickly as a spark gap. There is just no modern counterpart.

To understand what the gap does its necessary to understand a bit

about how a TC operates. As the main cap is being charged by our

transformer its voltage begins to rise. At this point the spark

gap is an open circuit. Whatever voltage the cap charges to also

appears across the gap electrodes. As the voltage rises the tension

across the open gap increases. Once the voltage is high enough the

air between the gap electrodes begins to ionize. Once enough ions

build up the gap becomes conductive and an arc jumps the gap.

Once the arc occurs the gap becomes a short circuit. All the

stored energy in the main cap is dumped across the shorted gap

into the primary coil. This then sets up the primary circuit

oscillation. The pulses of energy keep the oscillation between the

primary cap and primary coil going.

Once the voltage across the gap begins to fall the ionization

decreases and the gap once again becomes an open circuit. This

allows the main cap to begin charging once again.

This extinguishing of the arc is termed quenching. A gap that

quenches well will stop conducting very quickly after the

energy is discharged to the primary coil. A gap that quenches

poorly may stay lit long enough to interfere with cap charging.

The result of a gap that wont quench is little or no output.

How do we make sure our gap quenches adequately? By breaking

up the gap into a series of gaps. Instead of a single wide

gap we have a series string of narrow gaps. So instead

of all the generated heat and ions being concentrated in

one space they are spread out over a larger area. And

some forced air blowing through the gap will help remove

hot ions and heat that are clogging up the gap.

The gap distance or spacing determines how high the main cap

will charge before the gap breaks down. The wider the gap the

higher the voltage will rise and the more energy stored in the cap.

If the gap is too wide the voltage can rise to quite high

values and damage the insulation on your neon transformer

or damage your caps.

A rotary gap promotes quenching by forcing the electrodes apart.

The gap conducts only when the stationary and rotating components

of the gap are aligned. A rotary gap is often necessary

with large transformers and high power levels to insure

adequate quenching. But they offer no advantage to neon

powered coils and can actually damage neon transformers

due to the wideness of the gap when the electrodes are

not in alignment and the higher pulse rate the rotary

gap provides. Neon transformers actually work better

with a well designed static gap.

TTYL!   Mike Hammer  mhammer@misslink.net

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: Toroids

Cc:

Bcc:

X-Attachments:

At 11:02 AM 8/11/97 GMT, you wrote:

>From: James <057006@bud.cc.swin.edu.au>

>Date: Mon, 11 Aug 1997 21:02:59 +1000

>Subject: Toroids

>Organization: 057006

>

>Tesla-2 wrote:

>>

>> From: Mike Hammer <mhammer@misslink.net>

>> Date: Sat, 09 Aug 1997 01:24:17 -0500

>> Subject: Re: Safety hazards and toroids

>>

>> At 09:48 PM 8/9/97 GMT, you wrote:

>>

>> >>

>> >> I prefer the flexible aluminum dryer duct. It makes a wonderful

>> >> toroid and is available in many sizes. They look good and perform great.

>> >>

>> >> My largest one is 19 X 6 inches and has allowed me to get up

>> >> to 45 inches of spark with a single 12kv/90 ma neon.

>> >>

>> >> TTYL!   Mike Hammer  mhammer@misslink.net

>> >> Mike Hammer

>> >> mhammer@misslink.net

>> >>

>> >

>> >

>> >Mike,

>> >        Pardon my ignorance, but what is this stuff normally used for

>> >(flex. alum .dr duct?), does it come in toroid form already, or are you

>> >talking about those duct elbows from heating systems?

>>

>> This is flexible ducting normally used for clothes

>> dryer vents. Very common in hardware and home improvement

>> stores.

>>

<SNIP>

>

>Thanks for that Mike, but I guess we don't have that stuff over here in

>Australia. Last night I made a toroid form out of wood. It's one main

>ring with an OD of 90mm and inner of 60, 12 wooden circles of 60mm diam

>slot onto the clock positions around the ring... took a while to cut out

>all the slots, but it's pretty strong and I've got some heavy duty

>alfoil to cover it with. Sound O.K ?

>

>                                             James Hurl, Australia.

>

>

Yea that sounds fine to me. Sounds like it might be a little

on the small size. What is the finished diamter to be?

Keep an eye out for toroidal shaped objects.

You will be suprised what may turn up. Any type

of flexible tubing or duct can be used. Flexible

drainage pipe works well but of course must be covered with foil.

People have also covered tire inner tubes

with foil to create a serviceable toroid.

Be creative.

TTYL!   Mike Hammer  mhammer@misslink.net

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: Safety hazards??

Cc:

Bcc:

X-Attachments:

At 11:18 AM 8/11/97 GMT, you wrote:

>From: James <057006@bud.cc.swin.edu.au>

>Date: Mon, 11 Aug 1997 21:18:53 +1000

>Subject: Re: Safety hazards??

>Organization: 057006

>

<SNIP>

>>

>> However disruptive (spark gap) coils often have a considerable

>> 60 hz component in the secondary output. Its this energy that

>> causes shocks and is dangerous.

>

>

>

>Are there any ways to make ABSOLUTELY sure that this unwanted 50/60Hz

>energy does not get in to my secondary?

>

>

Not in disruptive discharge (spark gap) coils. There will

always be some 60 hz component. A smoothly running well

tuned coil will have low amounts of 60 hz energy.

If you want 60 hz free spark then go to a tube driven coil.

In this type of coil the primary circuit is replaced by

a vacuum tube oscillator. The resulting output sparks

will be quiet, smooth and flamelike.

Tubes coils are rather difficult to implement and difficult

to tune. And deliver much less spark output

I much prefer the noisy jagged intense lightning bolts

from a spark gap driven coil.

>

>I'm interested to know whether you or anyone else on this list has

>attempted any Tesla coil powered radio transmissions,(on the deregulated

>frequencies and power levels of course...) using it for this rather than

>to create sparks?

This can be done over short distances. The idea is to put

a huge toroid on your coil that will not allow any spark

breakouts. If you can tune this large toroid to resonance

then all your energy will be in the form of RF.

The distance it transmits will be small because the secondary

and toroid are very inefficient as an antenna. However

the RF ground currents can be picked up by another secondary

coil tuned to the same frequency as the running coil. This

extra secondary becomes the receiver.

I have done this over a distance of about 40 feet. When

the receiving secondary is well grounded you can draw

some small sparks off it. I was able to light a small flouresecent

tube and pull 2 inch sparks off the receiving coil.

This forms the basis of Tesla's proposed power transmission

system.

> Also, has anyone tried changing the spark gap to some

>sort of more silent unit which you could modulate using low power audio

>signals?..what I'm wondering here is whether the quieter brush discharge

>you get with just a bare wire (no capacity) at the top of the coil can

>make enough of a disturbance in the air to produce audio waves without

>the need of a loudspeaker cone (I've done the ion powered rotor thing

>and it makes me think that this is possible somehow).

>

>                                      James Hurl, Australia.

A vacuum tube driven coil could be run on DC and

modulated in any form you wished. Indeed a tube coil

IS a transmitter not very much unlike a conventional AM

radio station.

Be aware however that once you start modulating

it you are in effect broadcasting and in violation

of quite a number of laws. In the US the FCC takes

a very dim view of unlicensed broadcasting.

TTYL!   Mike Hammer  mhammer@misslink.net

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: Toroids

Date: Tue, 12 Aug 1997 13:58:18 GMT

Organization: The Energy Machine Information System 504-734-0526

To: mhammer@misslink.net

From: "Michael Smith" <md.smith@usa.net>

Date: Tue, 12 Aug 1997 08:58:18 -0500

Subject: Re: Toroids

----------

> From: Tesla-2 <tesla-2@emachine.com>

> To: md.smith@usa.net

> Subject: Re: Toroids

> Date: Monday, August 11, 1997 3:15 PM

>

> From: Mike Hammer <mhammer@misslink.net>

> Date: Mon, 11 Aug 1997 15:15:10 -0500

> Subject: Re: Toroids

>

> At 11:02 AM 8/11/97 GMT, you wrote:

> >From: James <057006@bud.cc.swin.edu.au>

> >Date: Mon, 11 Aug 1997 21:02:59 +1000

> >Subject: Toroids

> >Organization: 057006

> >

> >Tesla-2 wrote:

> >>

> >> From: Mike Hammer <mhammer@misslink.net>

> >> Date: Sat, 09 Aug 1997 01:24:17 -0500

> >> Subject: Re: Safety hazards and toroids

> >>

> >> At 09:48 PM 8/9/97 GMT, you wrote:

> >>

> >> >>

> >> >> I prefer the flexible aluminum dryer duct. It makes a wonderful

> >> >> toroid and is available in many sizes. They look good and perform great.

> >> >>

> >> >> My largest one is 19 X 6 inches and has allowed me to get up

> >> >> to 45 inches of spark with a single 12kv/90 ma neon.

> >> >>

> >> >> TTYL!

> >> >> Mike Hammer

> >> >> mhammer@misslink.net

> >> >>

> >> >

> >> >

> >> >Mike,

> >> >  Pardon my ignorance, but what is this stuff normally used for

> >> >(flex. alum .dr duct?), does it come in toroid form already, or are you

> >> >talking about those duct elbows from heating systems?

> >>

> >> This is flexible ducting normally used for clothes

> >> dryer vents. Very common in hardware and home improvement

> >> stores.

> >>

> <SNIP>

>

> >

> >Thanks for that Mike, but I guess we don't have that stuff over here in

> >Australia. Last night I made a toroid form out of wood. It's one main

> >ring with an OD of 90mm and inner of 60, 12 wooden circles of 60mm diam

> >slot onto the clock positions around the ring... took a while to cut out

> >all the slots, but it's pretty strong and I've got some heavy duty

> >al foil to cover it with. Sound O.K ?

> >

> >                                             James Hurl, Australia.

> >

> >

>

>

> Yea that sounds fine to me. Sounds like it might be a little

> on the small size. What is the finished diamter to be?

>

> Keep an eye out for toroidal shaped objects.

> You will be suprised what may turn up. Any type

> of flexible tubing or duct can be used. Flexible

> drainage pipe works well but of course must be covered with foil.

>

> People have also covered tire inner tubes

> with foil to create a serviceable toroid.

>

> Be creative.

>

> TTYL!

> Mike Hammer

> mhammer@misslink.net

I have a toroid that I am in the process of making from easily available

stuff. It is a child's vinyl swim ring...or inner tube. I turned a piece

of pegboard to just fit the inside diameter, where there is a convenient

lip made by the heat seal. The pegboard circle is glued to that, then the

frilly outer heat seal was taped down with masking tape. Then I applied

newspaper strips dipped in drywall mud. The next layer is drywall tape,

torn in half lengthwise and more mud. Once this hardened, I drilled a

tiny hole to release the pressure inside, now that the structure is

ridged. All I have left to do is finish the rounding and smoothing

process and either cover it in foil or be really bold and metallize it

good enough to have it plated. I think the plated end product would be

excellent, but am not sure if that will be the final product or not.

In the meantime I do have a flexible aluminum and steel pie pan toroid.

Thank you Mike Hammer for the excellent idea. I am searching for larger

flex duct. So far I have not seen any larger than 4 inch...but it will

show up right when I need it, I suspect.

Well, back to Monster capacitor building....

Michael Smith

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Tesla power system

Cc:

Bcc:

X-Attachments:

At 12:51 AM 8/13/97 GMT, you wrote:

>From: James <057006@bud.cc.swin.edu.au>

>Date: Tue, 12 Aug 1997 17:51:30 -0700

>Subject: Re: Safety hazards??

>Organization: 057006

>

>Tesla-2 wrote:

>

<SNIP>

>> >

>> >I'm interested to know whether you or anyone else on this list has

>> >attempted any Tesla coil powered radio transmissions,(on the deregulated

>> >frequencies and power levels of course...) using it for this rather than

>> >to create sparks?

>>

>> This can be done over short distances. The idea is to put

>> a huge toroid on your coil that will not allow any spark

>> breakouts. If you can tune this large toroid to resonance

>> then all your energy will be in the form of RF.

>> The distance it transmits will be small because the secondary

>> and toroid are very inefficient as an antenna. However

>> the RF ground currents can be picked up by another secondary

>> coil tuned to the same frequency as the running coil. This

>> extra secondary becomes the receiver.

>>

>> I have done this over a distance of about 40 feet. When

>> the receiving secondary is well grounded you can draw

>> some small sparks off it. I was able to light a small flouresecent

>> tube and pull 2 inch sparks off the receiving coil.

>> This forms the basis of Tesla's proposed power transmission

>> system.

>>

>

>

>WOW!

>  But this is fantastic! What sized coil/s were you using?

Transmitting coil was my 4 inch coil. Power was about 1.4 KVA.

Receiver was an older 4 inch coil tuned to same frequency.

>Do you think that the wireless power transmission system could have been

>implemented successfully with further development?

No. IMO Tesla's whole concept was flawed and doomed to failure

from the beginning. He based the whole system on what he believed

were stationary waves. He believed he witnessed the waves being

creating by thunderstorms during his stay in Colorado Springs.

More likely what he observed was reflections off the various mountain

ranges.

His Wardenclyfe station would not have transmitted meaningful

amounts of power but would have been instead the worlds most powerful

radio transmitter. Had it been completed it would of put Marconi

out of business.

> I realize that now it's probably all but impossible due to the frequency

>spectrum being nearly booked solid etc.. but still, you are saying that the

>Tesla system works as he claimed, and transmits energy/intelligence through

>the ground in a receivable way while suppressing the aerial propagation

>of EM waves - right?

This was Tesla's plan. But like I said he misinterpreted his

observations. He believed that his equipment could

create the same kind of stationary waves that he had observed

and that they would be conducted losslessly through the earth.

But such is not the case. Ground currents produced that way obey

the same inverse square law as every other form of energy.

If you double the distance the received energy falls to 1/4.

Treble the distance and it falls to 1/9.

The ground effects exist only over a very short distance.

His equipment would of resulted in highly efficient radio

broadcasts but not commercial power.

TTYL!

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: Safety hazards??

Date: Tue, 12 Aug 1997 20:52:50 GMT

Organization: The Energy Machine Information System 504-734-0526

To: mhammer@misslink.net

From: cjhill@hills.u-net.com (CJ Hill)

Date: Tue, 12 Aug 1997 21:52:50 +0100

Subject: Re: Safety hazards??

> I mentioned before that my coil is Ignition coil supplied, and doesn't

>develop sparks of more than 5 - 6 inches, does this not make it slightly

>safer than the beasts you guy's/girls make?

>

>                                               Thanks,

>                                                        James Hurl,

>Australia.

James,

I'm interested to know how your ignition-coil based system is set up. I also

run an ignition-coil system, but I rectify the output to give DC before

supplying this to a fairly conventional Tesla setup. I get approximately 24

inches of discharge at the moment, but this discharge only occurs a few

times per second. The '60Hz' energy in a tesla primary circuit that is often

referred to is in fact 700Hz pulsating d.c. in my setup (the frequency of my

solid-state drive).

Do you use straight AC from the ignition coil? If this is the case the value

of the primary capacitor you use will depend heavily on the frequency of

operation.

For example if I were to use my 700Hz ac output, I'd require a much smaller

value primary cap than I would with a conventional 60Hz neon-based circuit,

which would probably mean shorter sparks from my toroid. With this small

primary cap I'd also need a tesla primary coil with a huge number of turns

(probably in excess of 40 if my calcs are right) so that the primary tune is

in line with the secondary. Also, small primary capacitors and high rates of

discharge through the primary spark gap can cause a problem often known as

'flaming', whereby the sparks never quite extinguish (or quench). The neon

guys use fans or compressors to blow or suck air through the spark gap to

prevent this problem. But at 700Hz with much smaller primary capacitors the

problem is greatly magnified! Needless to say I wouldn't even attempt

operation at 700Hz ac.

One advantage of DC operated Tesla coils is that you have a bit more

flexibility in the value of the primary tank capacitor. You don't need to

balance its value with the impedance of the supply as with neon-based designs.

Info on my design is at http://www.hills2.u-net.com/tesla.htm

-------------------------------------------------

Chris Hill, Cardiff, UK

cjhill@hills.u-net.com

-------------------------------------------------

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: Safety hazards??

Date: Wed, 13 Aug 1997 00:51:30 GMT

Organization: The Energy Machine Information System 504-734-0526

To: mhammer@misslink.net

From: James <057006@bud.cc.swin.edu.au>

Date: Tue, 12 Aug 1997 17:51:30 -0700

Subject: Re: Safety hazards??

Organization: 057006

Tesla-2 wrote:

> >Are there any ways to make ABSOLUTELY sure that this unwanted 50/60Hz

> >energy does not get in to my secondary?

> >

> >

>

> Not in disruptive discharge (spark gap) coils. There will

> always be some 60 hz component. A smoothly running well

> tuned coil will have low amounts of 60 hz energy.

>

> If you want 60 hz free spark then go to a tube driven coil.

> In this type of coil the primary circuit is replaced by

> a vacuum tube oscillator. The resulting output sparks

> will be quiet, smooth and flamelike.

> Tubes coils are rather difficult to implement and difficult

> to tune. And deliver much less spark output

>

> I much prefer the noisy jagged intense lightning bolts

> from a spark gap driven coil.

>

> >

> >I'm interested to know whether you or anyone else on this list has

> >attempted any Tesla coil powered radio transmissions,(on the deregulated

> >frequencies and power levels of course...) using it for this rather than

> >to create sparks?

>

> This can be done over short distances. The idea is to put

> a huge toroid on your coil that will not allow any spark

> breakouts. If you can tune this large toroid to resonance

> then all your energy will be in the form of RF.

> The distance it transmits will be small because the secondary

> and toroid are very inefficient as an antenna. However

> the RF ground currents can be picked up by another secondary

> coil tuned to the same frequency as the running coil. This

> extra secondary becomes the receiver.

>

> I have done this over a distance of about 40 feet. When

> the receiving secondary is well grounded you can draw

> some small sparks off it. I was able to light a small flouresecent

> tube and pull 2 inch sparks off the receiving coil.

> This forms the basis of Tesla's proposed power transmission

> system.

>

WOW!

But this is fantastic! What sized coil/s were you using? Do you

think that the wireless power transmission system could have been

implemented successfully with further development? I realize that now

it's probably all but impossible due to the frequency spectrum being

nearly booked solid etc.. but still, you are saying that the Tesla

system works as he claimed, and transmits energy/intelligence through

the ground in a receivable way while suppressing the arial propagation

of EM waves - right?

> > Also, has anyone tried changing the spark gap to some

> >sort of more silent unit which you could modulate using low power audio

> >signals?..what I'm wondering here is whether the quieter brush discharge

> >you get with just a bare wire (no capacity) at the top of the coil can

> >make enough of a disturbance in the air to produce audio waves without

> >the need of a loudspeaker cone (I've done the ion powered rotor thing

> >and it makes me think that this is possible somehow).

> >

> >                                      James Hurl, Australia.

>

> A vacuum tube driven coil could be run on DC and

> modulated in any form you wished. Indeed a tube coil

> IS a transmitter not very much unlike a conventional AM

> radio station.

> Be aware however that once you start modulating

> it you are in effect broadcasting and in violation

> of quite a number of laws. In the US the FCC takes

> a very dim view of unlicensed broadcasting.

>

> TTYL!

> Mike Hammer

> mhammer@misslink.net

I actually meant that the "ion wind" or whatever you want to call it

produced by a sharp electrode subjected to HV could maybe be used to

produce an audio amplification/distribution without the need for a

speaker cone, eliminating all the usual distortions associated with

these (inertial distortion where the cone is travelling forward on it's

own impetus and must slow down before reversing for the negative cycle

etc...) which cause phase distortion problems in even the most top notch

speaker systems. I've seen snippets of info about this sort of thing

being attempted as a commercial proposition way back but the electrodes

heated up too fast or some such and the idea was dropped. That doesn't

stop me from mucking about with it at home though, I like the idea of

having near perfect audio reproduction while watching the bluish spray

dance in time!

Keeping the output within the audio range would keep me out of spectrum

police trouble, no?

James Hurl,

Australia.

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: Toroids

Date: Wed, 13 Aug 1997 01:14:36 GMT

Organization: The Energy Machine Information System 504-734-0526

To: mhammer@misslink.net

From: James <057006@bud.cc.swin.edu.au>

Date: Tue, 12 Aug 1997 18:14:36 -0700

Subject: Re: Toroids

Organization: 057006

Tesla-2 wrote:

> >> This is flexible ducting normally used for clothes

> >> dryer vents. Very common in hardware and home improvement

> >> stores.

> >>

> <SNIP>

>

> >

> >Thanks for that Mike, but I guess we don't have that stuff over here in

> >Australia. Last night I made a toroid form out of wood. It's one main

> >ring with an OD of 90mm and inner of 60, 12 wooden circles of 60mm diam

> >slot onto the clock positions around the ring... took a while to cut out

> >all the slots, but it's pretty strong and I've got some heavy duty

> >alfoil to cover it with. Sound O.K ?

> >

> >                                             James Hurl, Australia.

> >

> >

>

> Yea that sounds fine to me. Sounds like it might be a little

> on the small size. What is the finished diamter to be?

>

> Keep an eye out for toroidal shaped objects.

> You will be suprised what may turn up. Any type

> of flexible tubing or duct can be used. Flexible

> drainage pipe works well but of course must be covered with foil.

>

> People have also covered tire inner tubes

> with foil to create a serviceable toroid.

>

> Be creative.

>

> TTYL!

> Mike Hammer

> mhammer@misslink.net

Yeah it probably is a bit small, how do you normally go about

determining the required size? also I saw something on the list about

using the toroid not to discharge from but as an electrostatic sheild? I

tried just putting my finished baby toroid on top of my coil, with a

50mm diam hollow chinese "healthy ball" as the discharge terminal

elevated above the centre of the toroid, and the sparks stayed about the

same length but became fatter and more focused, almost no spray from the

windings and ball. I still need a grounded terminal to acheive sparks

though, the only way to get a discharge into the air on my coil so far

has been to use a brass doorknob, and this gives almost sparks, but more

of a purple 'sunflower' discharge all around the equater of the knob.

James Hurl, Australia.

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: Safety hazards??

Date: Wed, 13 Aug 1997 21:38:02 GMT

Organization: The Energy Machine Information System 504-734-0526

To: mhammer@misslink.net

From: James <057006@bud.cc.swin.edu.au>

Date: Wed, 13 Aug 1997 14:38:02 -0700

Subject: Re: Safety hazards??

Organization: 057006

Tesla-2 wrote:

>

> From: cjhill@hills.u-net.com (CJ Hill)

> Date: Tue, 12 Aug 1997 21:52:50 +0100

> Subject: Re: Safety hazards??

>

> > I mentioned before that my coil is Ignition coil supplied, and doesn't

> >develop sparks of more than 5 - 6 inches, does this not make it slightly

> >safer than the beasts you guy's/girls make?

> >

> >                                               Thanks,

> >                                                        James Hurl,

> >Australia.

>

> James,

>

> I'm interested to know how your ignition-coil based system is set up. I also

> run an ignition-coil system, but I rectify the output to give DC before

> supplying this to a fairly conventional Tesla setup. I get approximately 24

> inches of discharge at the moment, but this discharge only occurs a few

> times per second. The '60Hz' energy in a tesla primary circuit that is often

> referred to is in fact 700Hz pulsating d.c. in my setup (the frequency of my

> solid-state drive).

>

> Do you use straight AC from the ignition coil? If this is the case the value

> of the primary capacitor you use will depend heavily on the frequency of

> operation.

>

> For example if I were to use my 700Hz ac output, I'd require a much smaller

> value primary cap than I would with a conventional 60Hz neon-based circuit,

> which would probably mean shorter sparks from my toroid. With this small

> primary cap I'd also need a tesla primary coil with a huge number of turns

> (probably in excess of 40 if my calcs are right) so that the primary tune is

> in line with the secondary. Also, small primary capacitors and high rates of

> discharge through the primary spark gap can cause a problem often known as

> 'flaming', whereby the sparks never quite extinguish (or quench). The neon

> guys use fans or compressors to blow or suck air through the spark gap to

> prevent this problem. But at 700Hz with much smaller primary capacitors the

> problem is greatly magnified! Needless to say I wouldn't even attempt

> operation at 700Hz ac.

>

> One advantage of DC operated Tesla coils is that you have a bit more

> flexibility in the value of the primary tank capacitor. You don't need to

> balance its value with the impedance of the supply as with neon-based designs

.

>

> Info on my design is at http://www.hills2.u-net.com/tesla.htm

>

> -------------------------------------------------

> Chris Hill, Cardiff, UK

>

> cjhill@hills.u-net.com

> -------------------------------------------------

Chris,

The Ignition coil supply is simply 240v AC into a regular

household light dimmer, turned right down and the output of this goes

via a polyester cap to my primary capacitor which is three bottles. The

spark gap is only about 2.5 mm wide and the primary coil is 13 turns of

1/4 inch copper tube. I'll try to do a circut diagram here...

PRI COIL

!---------I=====I---------I I------+ I============I

!-----!-------->

!         I     I      polyester     I  Ignition  I

!     !        >

(240v AC)     I     I       cap          I    coil

>>>>HV----!    ===       >           !         I  O  I                    I

I       Bottles ===       >           !---------I=====I----------------

--_ I============I                !---0 0-->                   light dimmer

!   Gap

!

-----

---

-

James Hurl,

Australia.

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: Toroids

Date: Wed, 13 Aug 1997 21:52:48 GMT

Organization: The Energy Machine Information System 504-734-0526

To: mhammer@misslink.net

From: James <057006@bud.cc.swin.edu.au>

Date: Wed, 13 Aug 1997 14:52:48 -0700

Subject: Re: Toroids

Organization: 057006

Tesla-2 wrote:

>

> From: "Michael Smith" <md.smith@usa.net>

> Date: Tue, 12 Aug 1997 08:58:18 -0500

> Subject: Re: Toroids

>

> ----------

> > From: Tesla-2 <tesla-2@emachine.com>

> > To: md.smith@usa.net

> > Subject: Re: Toroids

> > Date: Monday, August 11, 1997 3:15 PM

> >

> > From: Mike Hammer <mhammer@misslink.net>

> > Date: Mon, 11 Aug 1997 15:15:10 -0500

> > Subject: Re: Toroids

> >

> > At 11:02 AM 8/11/97 GMT, you wrote:

> > >From: James <057006@bud.cc.swin.edu.au>

> > >Date: Mon, 11 Aug 1997 21:02:59 +1000

> > >Subject: Toroids

> > >Organization: 057006

> > >

> > >Tesla-2 wrote:

> > >>

> > >> From: Mike Hammer <mhammer@misslink.net>

> > >> Date: Sat, 09 Aug 1997 01:24:17 -0500

> > >> Subject: Re: Safety hazards and toroids

> > >>

> > >> At 09:48 PM 8/9/97 GMT, you wrote:

> > >>

> > >> >>

> > >> >> I prefer the flexible aluminum dryer duct. It makes a wonderful

> > >> >> toroid and is available in many sizes. They look good and perform> great.

> > >> >>

> > >> >> My largest one is 19 X 6 inches and has allowed me to get up

> > >> >> to 45 inches of spark with a single 12kv/90 ma neon.

> > >> >>

> > >> >> TTYL!

> > >> >> Mike Hammer

> > >> >> mhammer@misslink.net

> > >> >>

> > >> >

> > >> >

> > >> >Mike,

> > >> >  Pardon my ignorance, but what is this stuff normally usedfor

> > >> >(flex. alum .dr duct?), does it come in toroid form already, or are you

> > >> >talking about those duct elbows from heating systems?

> > >>

> > >> This is flexible ducting normally used for clothes

> > >> dryer vents. Very common in hardware and home improvement

> > >> stores.

> > >>

> > <SNIP>

> >

> > >

> > >Thanks for that Mike, but I guess we don't have that stuff over here in

> > >Australia. Last night I made a toroid form out of wood. It's one main

> > >ring with an OD of 90mm and inner of 60, 12 wooden circles of 60mm diam

> > >slot onto the clock positions around the ring... took a while to cut out

> > >all the slots, but it's pretty strong and I've got some heavy duty

> > >al foil to cover it with. Sound O.K ?

> > >

> > >                                             James Hurl, Australia.

> > >

> > >

> >

> >

> > Yea that sounds fine to me. Sounds like it might be a little

> > on the small size. What is the finished diamter to be?

> >

> > Keep an eye out for toroidal shaped objects.

> > You will be suprised what may turn up. Any type

> > of flexible tubing or duct can be used. Flexible

> > drainage pipe works well but of course must be covered with foil.

> >

> > People have also covered tire inner tubes

> > with foil to create a serviceable toroid.

> >

> > Be creative.

> >

> > TTYL!

> > Mike Hammer

> > mhammer@misslink.net

>

> I have a toroid that I am in the process of making from easily available

> stuff. It is a child's vinyl swim ring...or inner tube. I turned a piece

> of pegboard to just fit the inside diameter, where there is a convenient

> lip made by the heat seal. The pegboard circle is glued to that, then the

> frilly outer heat seal was taped down with masking tape. Then I applied

> newspaper strips dipped in drywall mud. The next layer is drywall tape,

> torn in half lengthwise and more mud. Once this hardened, I drilled a

> tiny hole to release the pressure inside, now that the structure is

> ridged. All I have left to do is finish the rounding and smoothing

> process and either cover it in foil or be really bold and metallize it

> good enough to have it plated. I think the plated end product would be

> excellent, but am not sure if that will be the final product or not.

>

> In the meantime I do have a flexible aluminum and steel pie pan toroid.

> Thank you Mike Hammer for the excellent idea. I am searching for larger

> flex duct. So far I have not seen any larger than 4 inch...but it will

> show up right when I need it, I suspect.

>

> Well, back to Monster capacitor building....

>

> Michael Smith

One of the first things I tried on top of my T.C was a spherical ceramic

candle holder that had been plated (cost 1 dollar at a junk shop).

Within about 1/2 an hour the coating was perforated with miniature

holes, and a clean strip around the bottom had been eaten away!

James Hurl, Australia.

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: JHCTES program, k

Date: Thu, 14 Aug 1997 17:34:33 GMT

Organization: The Energy Machine Information System 504-734-0526

To: mhammer@misslink.net

From: Richard Smit <esmit@ilink.nis.za>

Date: Thu, 14 Aug 1997 19:34:33 +0200 (SAT)

Subject: JHCTES program, k

Hi,

Can anyone tell me where I can find and download the JHTES program?

I have seen in my fathers electrical books the formula for coeficient of

couplingbut I don't know how you calculate the mutual inductance, can anyone

help me? Is the formula correct?

M

k = -----------

--------

/ L1 L2

k = Coeficient of coupling

M = Mutual inductance

Thanks

------------------------------------------------------------------------

|Richard Smit  | Telephone: +27-525-32634                                |

|P.O.Box 566   | Email:     esmit@ilink.nis.za                           |

|Bultfontein   | Age:       17 years old                                 |

|9670          | Hobbies:   Tesla Coils; Computors; Paranormal; UFO's    |

|South Africa  | Birthday:  5 October 1979                               |

------------------------------------------------------------------------

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: JHCTES program, k

Cc:

Bcc:

X-Attachments:

At 05:34 PM 8/14/97 GMT, you wrote:

>From: Richard Smit <esmit@ilink.nis.za>

>Date: Thu, 14 Aug 1997 19:34:33 +0200 (SAT)

>Subject: JHCTES program, k

>

>Hi,

>

>Can anyone tell me where I can find and download the JHTES program?

The JHCTES program is a commercial program written by

John Couture. It is not available for download. Only way

to get it is to buy John's over priced book.

IMO the program is not at all worth the cost.

>

>I have seen in my fathers electrical books the formula for coeficient of

>couplingbut I don't know how you calculate the mutual inductance, can anyone

>help me? Is the formula correct?

>                M

>        k = -----------

>             --------

>            / L1 L2

>

>k = Coeficient of coupling

>M = Mutual inductance

>

There is no direct way to calculate coefficient of coupling.

It is a value that must be determined by measurement.

For that you would need some test equipment such as a signal generator

and oscilloscope.

TTYL!

From: tesla-2@emachine.com (Tesla-2)

Subject: RE: Tesla power system

Date: Sun, 17 Aug 1997 22:06:30 GMT

Organization: The Energy Machine Information System 504-734-0526

To: mhammer@misslink.net

From: "Ashley, Marcus" <Marcus.Ashley@actew.com.au>

Return-Receipt-To: <Marcus.Ashley@actew.com.au>

Date: Mon, 18 Aug 1997 08:06:30 +1000

Subject: RE: Tesla power system

G'day to everyone. This post has nothing to do with 'Tesla Powered

Systems' because I am fairly new at this and dont know how to start a

new conversation, but anyway...

I heard that that it is a good idea to keep your primary coils diameter

about the same in length as your secondary is high. Is this because a

magnetic field is at 90 deg. to the electrical conductor (wire). And if

so does that mean that the magnetic field caused by the primary has to

cover the whole coil in order for proper magnetic coupling/resonance of

the coil? Does a toroid discharge terminal help in this magnetic

coupling.

Also, I am using several automotive spark plugs for my spark gap instead

of a single spark gap, it seemed to work better than my rotary gap but I

would of tought that it would be less efficient because the spark had to

jump many gaps instead of one.  If anyone can understand my scattered

thoughts your comment would be appreciated.

Allways listen to experts, they'll tell you what can't be done and why.

Then do it.

Marc Ashley

>

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: Tesla power system

Date: Mon, 18 Aug 1997 01:43:02 GMT

Organization: The Energy Machine Information System 504-734-0526

To: mhammer@misslink.net

From: Bert Hickman <bert.hickman@aquila.com>

Date: Sun, 17 Aug 1997 18:43:02 -0700

Subject: Re: Tesla power system

Organization: Stoneridge Engineering

Tesla-2 wrote:

>

> From: "Ashley, Marcus" <Marcus.Ashley@actew.com.au>

> Return-Receipt-To: <Marcus.Ashley@actew.com.au>

> Date: Mon, 18 Aug 1997 08:06:30 +1000

> Subject: RE: Tesla power system

>

> G'day to everyone. This post has nothing to do with 'Tesla Powered

> Systems' because I am fairly new at this and dont know how to start a

> new conversation, but anyway...

> I heard that that it is a good idea to keep your primary coils diameter

> about the same in length as your secondary is high. Is this because a

> magnetic field is at 90 deg. to the electrical conductor (wire). And if

> so does that mean that the magnetic field caused by the primary has to

> cover the whole coil in order for proper magnetic coupling/resonance of

> the coil? Does a toroid discharge terminal help in this magnetic

> coupling.

>

> Also, I am using several automotive spark plugs for my spark gap instead

> of a single spark gap, it seemed to work better than my rotary gap but I

> would of tought that it would be less efficient because the spark had to

> jump many gaps instead of one.  If anyone can understand my scattered

> thoughts your comment would be appreciated.

>

> Allways listen to experts, they'll tell you what can't be done and why.

>       Then do it.

>                                                 Marc Ashley

G'day Mate!

The primary diameter's not really all that critical. However, flat

primaries with a significant number of turns - say 12-18 - will just

tend to be fairly wide. The magnetic field created from the primary

really only couples strongly to the bottom-most third of the secondary,

and virtually none at all at the top of the secondary or to the toroid.

The degree of coupling (i.e., the portion of magnetic field lines that

link both the primary and secondary), is typically in the range of only

15 - 23% for most 2-coil systems. While this "coupling coefficent"  is

necessary for primary-to-secondary energy transfer, it really doesn't

play a direct role in either primary or secondary resonance.

The toroid's functions are to prevent premature breakout of streamers,

shield the top of the coil from breaking out, and to provide a "lumped"

capacitance at the top of the secondary. However, it plays NO part in

the electromagnetic coupling between the primary and secondary.

Regarding your gap, sparkplugs will work. However, a better method uses

multtiple gaps made from copper pipes, and a much better method forces

air between the gaps to extinguish the arcs between the copper tubes.

It's necessary to extinguish the arc at the right time to prevent energy

transferred to the secondary from transferring back into the primary

circuit. This is called "quenching". Also, multiple gaps actually tend

to perform much better than single gaps! This is because the multiple

metal-arc interfaces tend to extinguish the arc better, and because the

breakdown voltage for a series of gaps tends to be much more consistent

than one large gap.

Check out the funet site for the file sparkgap.zip for some excellent

information on various kinds of static gaps:

http://www.funet.fi/pub/sci/electrical/tesla/misc/

And, safe coilin' to you, Marc!

-- Bert --

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: RE: spark gaps

Date: Mon, 18 Aug 1997 23:00:35 GMT

Organization: The Energy Machine Information System 504-734-0526

To: mhammer@misslink.net

From: "Ashley, Marcus" <Marcus.Ashley@actew.com.au>

Return-Receipt-To: <Marcus.Ashley@actew.com.au>

Date: Tue, 19 Aug 1997 09:00:35 +1000

Subject: RE: spark gaps

I am using an 8750v @110mA neon and would like some advise on a

suitable static spark gap (I'm not exactly sure what you mean by a

static gap). I was using a rotary spark gap until I read this post.

Allways listen to experts, they'll tell you what can't be done and why.

Then do it.

Marc Ashley

>----------

>From:  tesla-2@emachine.com[SMTP:tesla-2@emachine.com]

>Sent:  Tuesday, 12 August, 1997 4:21AM

>To:    Ashley, Marcus

>Subject:       Re: spark gaps

>

>>Subject: spark gaps

>

>

>A rotary gap promotes quenching by forcing the electrodes apart.

>The gap conducts only when the stationary and rotating components

>of the gap are aligned. A rotary gap is often necessary

>with large transformers and high power levels to insure

>adequate quenching. But they offer no advantage to neon

>powered coils and can actually damage neon transformers

>due to the wideness of the gap when the electrodes are

>not in alignment and the higher pulse rate the rotary

>gap provides. Neon transformers actually work better

>with a well designed static gap.

>

>TTYL!

>

>

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: RE: spark gaps

Cc:

Bcc:

X-Attachments:

At 11:00 PM 8/18/97 GMT, you wrote:

>From: "Ashley, Marcus" <Marcus.Ashley@actew.com.au>

>Return-Receipt-To: <Marcus.Ashley@actew.com.au>

>Date: Tue, 19 Aug 1997 09:00:35 +1000

>Subject: RE: spark gaps

>

>I am using an 8750v @110mA neon and would like some advise on a

>suitable static spark gap (I'm not exactly sure what you mean by a

>static gap). I was using a rotary spark gap until I read this post.

>

>Allways listen to experts, they'll tell you what can't be done and why.

>      Then do it.

>                                                Marc Ashley

>

Marc,

The term static gap just means the gap is not moving as in a

rotary gap.

Static gaps can take many forms from a pair of bolts arranged

head to head to multiple electrode arrangements with cooling

air supplied by means of a fan or even a vacuum cleaner motor.

One of the more popular gap arrangements for neons is the cylinder

gap that was popularized (not invented) by Richard Quick. It

is often therefore termed a RQ gap.

This arrangement places electrodes of hard copper tubing

in a radial arrangment inside a piece of PVC pipe. The PVC

becomes the housing and the electrodes are mounted on the inside

wall. Mount a strong muffin or boxer fan on top to supply cooling

air and supply exhaust holes or ports for the hot air and ions

and you have an effective gap.

This type of gap can also be placed on a flat plastic

plate. The electrodes are mounted side by side.

The number and size of electrodes depends on the power level

and voltage your neons are supplying. 7 electrodes (6 gaps)

work well at 9kv and 9 electrodes (8 gaps) work well for

12kv neons. With 15 kv neons 11 electrodes (10 gaps) may be

used. Space each electrode from .025 to .03 inches apart.

Try to get them all the same. This amount of gap is fairly

safe and wont usually over stress your neons or caps.

Use 3/4 inch copper pipe for the electrodes. Cut them 3 or 4

inches long. One inch pipe electrodes will handle more power

and handle the heat better but 3/4 is fine for up to

1500 watts or so.

If your coil is running well a little more gap (additional electrodes)

may be added to maximize spark output. But do so at your

own risk. Each increase in gap increases the stress level on caps

and transformers.

TTYL!

From: tesla-2@emachine.com (Tesla-2)

Subject: Beginning  work on my caps.

Date: Thu, 21 Aug 1997 15:53:10 GMT

Organization: The Energy Machine Information System 504-734-0526

To: mhammer@misslink.net

From: "Paul Marshall" <PMARSHAL@ETHCINC1.SSW.JNJ.COM>

Date: 21 Aug 1997 10:53:10 EST

Subject: Beginning  work on my caps.

I am starting my Capacitor project this week. I think I'm going to build

both a rolled cap and a plate cap. Can someone help me with the

following questions, I'm using 4 mil poly and using 15 sheets between

the plates :

PS: How do I keep the poly sheet from slipping ? Can I tack them together

with something along the edges ? When winding a roll cap how do I keep

the core tight ?

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: Beginning work on my caps.

Cc:

Bcc:

X-Attachments:

At 03:53 PM 8/21/97 GMT, you wrote:

>From: "Paul Marshall" <PMARSHAL@ETHCINC1.SSW.JNJ.COM>

>Date: 21 Aug 1997 10:53:10 EST

>Subject: Beginning  work on my caps.

>

>

>     I am starting my Capacitor project this week. I think I'm going to build

>     both a rolled cap and a plate cap. Can someone help me with the

>     following questions, I'm using 4 mil poly and using 15 sheets between

>     the plates :

>

> PS: How do I keep the poly sheet from slipping ? Can I tack them together

>      with something along the edges ? When winding a roll cap how do I keep

>     the core tight ?

>

Poly is a very slippery material and its a pain trying to keep

all the poly squares aligned. I has considerable difficulty

with it during construction of my first plate cap.

On my 2nd plate cap I put a small puddle of mineral oil in the center

of each poly sheet and layed down the next sheet on it. I smoothed

out the puddle of oil and the oil film helped to hold the sheets

together.

I made up each stacks of sheets for each dielectric layer in advance.

Then when it came time to build the cap I treated each poly stack as

a single unit. This worked out well and assembly went quickly.

This cap subsequently failed because I didnt allow enough time

for the air to escape. When I rebuilt it I built up the cap

one layer at a time dipping each poly sheet completely in oil.

This turned out well and the cap performed well but was really messy.

When winding a rolled cap keeping the poly strips in alignment

is difficult. Try to make the strips only as long as necessary.

Use wider plates rather than longer ones for more capacitance.

You can place paper clips along the length of the poly strips

to hold them in alignment. Just remember to remove them as you

roll.

Wind the roll on a center pipe. I wound mine on a piece

of 1/2 inch copper tubing. I wound the inner foil 5 or 6 times

tightly around the copper pipe and then taped it down with

clear packing tape. This provides a good low impedance

connection to the inner foil. I soldered a AWG 8 solid copper

wire to the copper pipe to make the connection to the cap terminals.

Do this before you roll the cap. The heat may not be good for the poly.

Using the center pipe as a handle you can wind up the cap roll

tighter. In fact I put a pair of pliers on the pipe and cranked

it down good. Dont over do it. You dont want to tear the foil

plates. But you can tighten it pretty tight.

The end of the outer foil is wrapped several times around a

strip of aluminum flashing and taped down tight. A few strips

of tape around the roll will help keep it tight.

Good luck with your cap project.

Keep us posted of your results.

TTYL!

From: tesla-2@emachine.com (Tesla-2)

Subject: Contruction

Date: Thu, 21 Aug 1997 23:00:35 GMT

Organization: The Energy Machine Information System 504-734-0526

To: mhammer@misslink.net

From: BadBowTie9@aol.com

Date: Thu, 21 Aug 1997 19:00:35 -0400 (EDT)

Subject: Contruction

Ok I now have two 6kv 30ma power transformers, and I am getting ready to

build my first coil. what I need to know is, what would be best for the

transformer, such as Wire thickness for the primary and secondary Coils.

I have rf chokes, and I'm using the poor mans Leyden Jars for caps. I also

need more info on "Spark Gaps"

Any Ideas would help....... Thanks M.R.Howard

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: Contruction

Cc:

Bcc:

X-Attachments:

At 11:00 PM 8/21/97 GMT, you wrote:

>From: BadBowTie9@aol.com

>Date: Thu, 21 Aug 1997 19:00:35 -0400 (EDT)

>Subject: Contruction

>

> Ok I now have two 6kv 30ma power transformers, and I am getting ready to

>build my first coil. what I need to know is, what would be best for the

>transformer, such as Wire thickness for the primary and secondary Coils.

By transformer I assume you mean the Tesla coil itself.

A Tesla coil isnt really a transformer. At least not in

the conventional sense. A transformer raises or lowers voltage by means

of its turns ratio. But a Tesla coil achieves its high voltage by

resonant circuit rise.

The size of wire for the secondary depends on the diameter.

For the transformers you have a 3 or 4 inch diameter coil would be

best.

For a 3 incher use AWG 24 or AWG 26. Use AWG 24 and wind a 18 inch

long winding. That will be about 830 turns. Resonant frequency

about 600 khz. Use a small spherical terminal like a 4 or 5 inch

diameter styrofoam ball covered with foil. This will spark easily

at low power.

For the primary use whatever heavy wire you have or can

get. Use at least AWG 12. Make it at least twice the diameter

of your secondary and wind 12-14 turns. Space the turns 1/4-1/2 inch

apart. Make it any form you want. A solenoid form will work well

on a small coil as will the saucer shaped primary.

A flat spiral will probably not couple well enough at your low

power level.

If you have to by wire for your primary check the price of 1/4 inch

copper tubing.

> I have rf chokes, and I'm using the poor mans Leyden Jars for caps. I also

>need more info on "Spark Gaps"

A good file on the cylinder type spark gap can be had from

ftp.funt.fi/pub/sci/electrical/tesla/misc

File name is sparkgap.zip. Since your voltage is a bit low

at 6kv I would recommend maybe 5 or 6 gaps spaced about .02-.025

inch.

Start with one of your transformers then parallel them for more

power once your coil is running well on the single transformers.

Good luck.

TTYL!

From: tesla-2@emachine.com (Tesla-2)

Subject: electomagnetic spark gaps?

Date: Mon, 25 Aug 1997 15:54:10 GMT

Organization: The Energy Machine Information System 504-734-0526

To: mhammer@misslink.net

From: watkier@earlham.edu

Date: Mon, 25 Aug 1997 10:54:10 EST

Subject: electomagnetic spark gaps?

Has anyone heard of or used an electromagnetic spark gap?  I read about

them in _The inventions researches and writing of Nikola Tesla_.  They

involve a strong  magnetic field created across the gap by two

electromagnets.  Apparently the magnetic field helps quench the spark.

Does anyone know how the magnetic field does this or how this type of

spark gap compares to an airblast gap?

Cheers-

Erik

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: electomagnetic spark gaps?

Cc:

Bcc:

X-Attachments:

At 03:54 PM 8/25/97 GMT, you wrote:

>From: watkier@earlham.edu

>Date: Mon, 25 Aug 1997 10:54:10 EST

>Subject: electomagnetic spark gaps?

>

>Has anyone heard of or used an electromagnetic spark gap?  I read about

>them in _The inventions researches and writing of Nikola Tesla_.  They

>involve a strong  magnetic field created across the gap by two

>electromagnets.  Apparently the magnetic field helps quench the spark.

>Does anyone know how the magnetic field does this or how this type of

>spark gap compares to an airblast gap?

>

>Cheers-

>Erik

>

>

Erik,

I dont have any direct experience with magnetic gaps

only what I've read.

Basicly anything you can do to stretch the arc in a gap

improves the quenching. Stretching the arc makes it cooler

and improves the removal of hot ions from between the gap electrodes.

You can stretch the arc by blowing or sucking air through the

gap or by pulling the arc itself with a magnetic field.

This requires extremely strong magnets like neobdyium-iron-boron.

And very close spacing of the magnets to the arc.

The problem I see is to prevent the arc from jumping to the magnets.

I dont believe a magnetic gap offers any better performance than

a well designed vacuum gap.

TTYL!

From: tesla-2@emachine.com (Tesla-2)

Subject: Plans.  Construction guide.

Date: Tue, 26 Aug 1997 13:03:02 GMT

Organization: The Energy Machine Information System 504-734-0526

To: mhammer@misslink.net

From: Suagg@aol.com

Date: Tue, 26 Aug 1997 09:03:02 -0400 (EDT)

Subject: Plans.  Construction guide.

Hi,

Does anyone out there have any plans or a construction guide for a Tesla coil

that they could send me?  It would have to be a basic model considering I am

just starting out.  If someone could help me I would be greatful.

Thanks.

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: Plans.  Construction guide.

Cc:

Bcc:

X-Attachments:

At 01:03 PM 8/26/97 GMT, you wrote:

>From: Suagg@aol.com

>Date: Tue, 26 Aug 1997 09:03:02 -0400 (EDT)

>Subject: Plans.  Construction guide.

>

>Hi,

>

>Does anyone out there have any plans or a construction guide for a Tesla coil

>that they could send me?  It would have to be a basic model considering I am

>just starting out.  If someone could help me I would be greatful.

>

>                                                      Thanks.

>

Existing Tesla coil plans are notoriously poor

performers being mostly based on old outdated methods.

There has been considerable change in the way Tesla coils are built

in the last ten years. A lot of the hard work has been done by others

so that a basic conventional coil is pretty much cut and dried.

There are plans and kits available from companies like

Information Unlimited but the results from them are so

poor that most first timers can build a better coil

from scratch with a little guidance.

I will paste in here a typical schematic and some general

guidelines on building a coil. This has been posted to the

list before but I repeat it here for the benefit of some

of the newer subscribers. Those that have seen this several times

already bear with us.

<begin Include>

Lets look at a coil schematic.

TESLA COIL SCHEMATIC                             ---------

|       | T1

---------

O

X1                              G1              O

RFC                           O  L2

O---------------||||----------* *-----        O

||O    |                   |           |   L1   O

||O    |                   |           |    O   O

-----O||O    * SG              -----         |--->O   O

O||O----*---|Gnd                C1           O   O

-----O||O    *                 -----              O   O

120   ||O    |                   |                O   O

Volts ||O    |                   |                O   O

O---------------||||----------------------O   O

RFC                           |

|

Gnd

X1 = Neon Sign Transformer, grounded case

SG = Safety Gap.

RFC = RF filter chokes

G1 = Main system (tank circuit) Spark Gap.

C1 = Tank circuit capacitor

L1 = Primary Coil.

L2 = Secondary Coil.

T1 = Toroid discharge terminal

Gnd = RF Ground.

X1. The power supply transformer. Most commonly a neon sign transformer.

These are available in a wide range of voltages and current level. The most

useful for TC use are 9000 and 12000 volt with current ratings of 30 or

60 ma amps. These can be paralleled for more power. I currently use a

modified 12kv/60ma neon that outputs 90ma. This gives me 42 inch sparks

on my present coil.

Used neons are available at reasonable cost from neon sign companies.

Check Yellow pages.

It is possible to unpot and repair dead transformers that you can often

get for free. This I have done. Its plenty of work but I got more time than money.

Also useful are ignition transformers from old oil burning furnaces.(OBIT)

These are usually rated at 10kv/23ma. Heating contractors can be a source

for these. I have had some places give them away to me. These aren't as

rugged as neons but do serve well.

SG. This is a close spaced spark gap placed across the transformer output.

Its purpose is to protect the transformer from hi voltage RF kickbacks.

RFC. One of these chokes go into each hi voltage leg to attenuate

any RF kickbacks. Helps to protect transformer. These should have a value

of better than 2 millihenries. A hundred or more turns of wire on a large

ferrite toroid has proven effective.

G1. Main spark gap. This is the least understood component. It serves chiefly

as a high speed hi power switch. Lots of energy is dissipated here so the gap

must deal with the heat. Arrangements vary from a simple gap between 2 bolts

to multielectrode units with cooling fans to rotary gaps.

We will spend lots of time talking about gaps.

C1. Main tank capacitor. This cap resonates with the primary coil at

the same frequency as the secondary. It must be of low loss construction

and be able to withstand twice the transformer voltage at a minimum.

Commercial caps of this kind are expensive.($200-300)

Polyethylene plastic film and aluminum flashing can be fashioned into

servicable homemade caps. These are put into a tank of mineral oil to

improve voltage rating. This can be in the form of alternating layers

of poly and aluminum in a stacked plate. Or two long plates and strips

of poly can be rolled up to produce a rolled cap. These tend to be more

compact.

A simple homemade cap can be made with salt water and beer bottles.

This design is the classic Leyden Jar. The salt water becomes the plates

and the bottle is the dielectric. This is not a low loss cap but does

work rather well and is cheap. I made one of these for my first cap.

I replaced it with a polethylene film cap that performed much better.

The poly cap took 14 hours to build but was worth it.

L1. Primary. Should be of low loss conductors. Heavy wire or cable

or copper tubing. A movable tap is required to tune this coil to

proper frequency. 12 to 14 turns and should be at least twice the diameter

of the secondary. Can be in the form of a solenoid (like the secondary)

or a flat spiral. A compromise between the two is the saucer shaped

primary with a slope of between 15 and 30 degrees.

L2. Secondary. The best secondaries have H/D (height/diameter) ratios

of between 3 to 1 and 6 to 1. Small coils need the higher ratios and the

small ratios for larger coils. A 3 inch dia coil would be best at 6 to 1.

A 6 incher might do with 4 to 1. A 10 incher would be best at 3 to 1.

That means a 3 1/2 diameter coil should have a winding length close

to 18 inches. The proper number of turns is from 700 - 900 turns.

So you pick the wire guage that will allow you to get this number of turns

in the allowable space. That would be 24 or 26 for a 3 incher.

This is wound on white PVC pipe or other plastic pipe. Plexiglass pipe is

good but expensive. Acrylic is good as is polyethylene. Avoid colored

pipe.

DRILL NO HOLES IN YOUR SECONDARY FORM! NEVER. NEVER.

Wind on a few turns and secure it with tape or hot glue. Wind on the needed

turns close spaced. Secure both ends with tape and coat the windings with

polyurethane varnish. Seal the top end of the form with a disk of plexiglass

or Lexan. This is epoxied down inside the form just above the secondary winding.

THis addes electrical strength to the form and prevents arcing down the inside

of the form. Make this disk fairly thick. 3/16- 1/4 inch.

T1. Top discharge terminal. This is necessary for long sparks. Can be a

sphere or toroid. A copper toilet tank float makes a good first electrode.

Also foil covered styrofoam balls. Later larger toroids can be made from

flexible dryer ducting.

These terminals add capacitance to the secondary and lowers the resonant

frequency.

This is why we want a large number of turns in our primary coil. So we

can add a larger toroid and regain the tune by tapping farther out on the primary.

Well there you have it. A short overview of the whole deal.

Look this over and feel free to ask any questions.

TTYL!

From: tesla-2@emachine.com (Tesla-2)

Subject: Number of Turns

Date: Wed, 27 Aug 1997 19:58:25 GMT

Organization: The Energy Machine Information System 504-734-0526

To: mhammer@misslink.net

From: Richard Smit <esmit@ilink.nis.za>

Date: Wed, 27 Aug 1997 21:58:25 +0200 (SAT)

Subject: Number of Turns

Yo dudes,

1. Can anyone tell me what is the maximum and minimum windings needed on

your secondary or isn't there a limit? Will it have any effect on your

performance?

2. What is the difference in performance when you use a coil with 2000

windings on the   secondary and one with 800 turns on the secondary?

3. I have taken a 6.4 inch diameter pvc pipe about 15 inches high and placed

a wooden piece on top. I stand on this unit and then touches the spark with

my gap very small, so that there isn't sparkover between primary and

secondary. If i make the gap bigger and say a sparkover occurs will it be

deadly to me?

If my secondary coil is grounded will it be dangerous to touch the grounded

secondary even if you are isolated from ground?

4. The Richard Quick static gap, the spacing between the pieces of iron is

close together about .025 inch from each other. Isn't that to small?

(According to me the cap won't recharge to it's fullest)

Thanks

------------------------------------------------------------------------

|Richard Smit  | Telephone: +27-525-32634                                |

|P.O.Box 566   | Email:     esmit@ilink.nis.za                           |

|Bultfontein   | Age:       17 years old                                 |

|9670          | Hobbies:   Tesla Coils; Computors; Paranormal; UFO's    |

|South Africa  | Birthday:  5 October 1979                               |

------------------------------------------------------------------------

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: Number of Turns

Date: Wed, 27 Aug 1997 23:02:12 GMT

Organization: The Energy Machine Information System 504-734-0526

To: mhammer@misslink.net

From: corr <corr@fullnet.net>

Date: Wed, 27 Aug 1997 17:02:12 -0600

Subject: Re: Number of Turns

>1. Can anyone tell me what is the maximum and minimum windings needed on

>your secondary or isn't there a limit? Will it have any effect on your

>performance?

>2. What is the difference in performance when you use a coil with 2000

>windings on the    secondary and one with 800 turns on the secondary?

Depends on alot of things, input power, wire size, etc.

>3. I have taken a 6.4 inch diameter pvc pipe about 15 inches high and placed

>a wooden piece on top. I stand on this unit and then touches the spark with

>my gap very small, so that there isn't sparkover between primary and

>secondary. If i make the gap bigger and say a sparkover occurs will it be

>deadly to me?

>If my secondary coil is grounded will it be dangerous to touch the grounded

>secondary even if you are isolated from ground?

15 inches won't really be high enough for a 6" diemeter secondary.  I have a

4" diameter and use a 20" winding section, the tube being 25" inches tall.

>4. The Richard Quick static gap, the spacing between the pieces of iron is

>close together about .025 inch from each other. Isn't that to small?

>(According to me the cap won't recharge to it's fullest)

The static gap has more than one electrode.  With 8 of them, the distance

totalled is .2 inches, which is around the correct distance.  I myself just

have a single gap, and it works good.  Although I made a static gap, I

only got 1 or 2 inches more spark.   The single gap was obviously smaller

so thats what I still use.

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: Number of Turns

Cc:

Bcc:

X-Attachments:

At 07:58 PM 8/27/97 GMT, you wrote:

>From: Richard Smit <esmit@ilink.nis.za>

>Date: Wed, 27 Aug 1997 21:58:25 +0200 (SAT)

>Subject: Number of Turns

>

>Yo dudes,

Richard,

You got some good questions here so lets see what we

can do with them.

>

>1. Can anyone tell me what is the maximum and minimum windings needed on

>your secondary or isn't there a limit? Will it have any effect on your

>performance?

>

>2. What is the difference in performance when you use a coil with 2000

>windings on the secondary and one with 800 turns on the secondary?

Lets take the first two questions together as they are basicly

asking the same thing.

There is no maximum or minimum number of turns. The only real criterion

is that the primary cicuit resonant frequency must match the secondary

resonant frequency. This is where a Tesla develops its high voltage.

Through resonant circuit rise. The turns ratio between the primary

and secondary have very little to do with the output.

Coils generally perform best with H/D (height//diameter) ratios from

3 to 1 to 6 to 1. The larger the secondary the smaller the H/D ratio

should be. 6 to 1 is fine for a 3 inch diameter coil while a 10 inch

one would be best at 3 to 1. 4 to 1 is normally used for 6 inch coils

and 5 to 1 for 4 inchers.

The proper number of turns is between 750-1000 turns. Coils can be made

outside this range but this number of turns combined with the H/D ratios

above will make a better coil.

Secondary coils with many turns of wire (2000 in your question above)

have very low resonant frequencies so require large primary coils and

large primary caps to match that frequency. Those large caps in turn

require large transformers to charge them adequately.

Also since the bulk of the energy coupled into the secondary is in the

bottom 1/3 of the winding a large number of turns is simply wasted wire.

Plus all that wire adds resistance and lowers the Q of the coil.

Too few turns results in a very high resonant frequency. Too high

a frequency increases system losses and a secondary with too few

turns can suffer from overvoltage problems due to the voltage

being spread over too few turns.

>

>3. I have taken a 6.4 inch diameter pvc pipe about 15 inches high and placed

>a wooden piece on top. I stand on this unit and then touches the spark with

>my gap very small, so that there isn't sparkover between primary and

>secondary. If i make the gap bigger and say a sparkover occurs will it be

>deadly to me?

Not necessarily. But there is always a chance. Taking streamers

to the body is not too dangerous with small coils but exceedingly

so with large ones. I wouldnt want to take a hit from any of the 40

inch + streamers from my coil under any circumstances.

>If my secondary coil is grounded will it be dangerous to touch the grounded

>secondary even if you are isolated from ground?

Grounded secondary is no more dangerous than any other.

In fact if you're running your coil with the bottom of your

secondary tied to the bottom of the primary coil like is shown

in some older drawings that is very dangerous. This places 60 hz

primary energy directly on the secondary.

Never tie your secondary and primary together.

>

>4. The Richard Quick static gap, the spacing between the pieces of iron is

>close together about .025 inch from each other. Isn't that to small?

>(According to me the cap won't recharge to it's fullest)

>

For the RQ type of cylinder gap you should use copper pipe

not iron and the spacing is fine between .025 and .03 inch.

This type of gap has several electrodes in series each spaced .025

inch from each other. With 8 gaps that adds up to .200 inch.

That is plenty of gap. When your coil is running smoothly with

this amount of gap you can increase it some by adding another series

electrode. Be careful though. As the amount of total gap increases

so does the stress placed on your caps and transformers. If you get the

gap too wide you WILL have a failure.

TTYL!

From: tesla-2@emachine.com (Tesla-2)

Subject: RE: spark gaps

Date: Thu, 28 Aug 1997 22:38:24 GMT

Organization: The Energy Machine Information System 504-734-0526

To: mhammer@misslink.net

From: "Ashley, Marcus" <Marcus.Ashley@actew.com.au>

Return-Receipt-To: <Marcus.Ashley@actew.com.au>

Date: Fri, 29 Aug 1997 08:38:24 +1000

Subject: RE: spark gaps

I have made a spark gap consisting of seven 1.5inch by 1/2inch pieces of

copper tubing housed in a plastic round container with a muffin fan

placed under the container. My question is do the copper tubes have to

be perfectly vertical and parallel, and does it matter if more than one

spark develops between each piece of tubbing? If so, can someone please

explain why?

Allways listen to experts, they'll tell you what can't be done and why.

Then do it.

Marc Ashley

>

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: RE: spark gaps

Cc:

Bcc:

X-Attachments:

At 10:38 PM 8/28/97 GMT, you wrote:

>From: "Ashley, Marcus" <Marcus.Ashley@actew.com.au>

>Return-Receipt-To: <Marcus.Ashley@actew.com.au>

>Date: Fri, 29 Aug 1997 08:38:24 +1000

>Subject: RE: spark gaps

>

>I have made a spark gap consisting of seven 1.5inch by 1/2inch pieces of

>copper tubing housed in a plastic round container with a muffin fan

>placed under the container. My question is do the copper tubes have to

>be perfectly vertical and parallel,

Yes. The arcing surfaces of each electrode should be as

parallel as possible. And each gap should have as close to the exact

same spacing as possible.

> and does it matter if more than one spark develops between each piece of tubing? If so, can someone please explain why?

This is eaxctly what you want. By making the electrodes

several inches long you are spread the activity over a larger

area. This results in cooler more efficient gap operation.

TTYL!

From: tesla-2@emachine.com (Tesla-2)

Subject: First coil finished!

Date: Sat, 30 Aug 1997 19:43:31 GMT

Organization: The Energy Machine Information System 504-734-0526

To: mhammer@misslink.net

From: engineer <engineer@techinter.com>

Date: Sat, 30 Aug 1997 14:43:31 -0500

Subject: First coil finished!

Organization: DeVry Institute of Technology

Well I've finally got my first coil finished!  It's very small, but I

guess everyone has to start somewhere.  I have a 3 1/2" diameter

secondary, 10" high powered by a 6500V@23mA neon transformer. I can only

draw about an 8" spark to a grounded bar.  I think I may need a smaller

toroid because I get optimum spark length when my primary is tapped at

the very end of the coil.

I'd like to thank everyone on the list for alll the information I

gained.  I wouldn't be able to have got this coil working nor would I be

able to build my next one without all you guys.  I think it's really

great how all experienced coilers are willing to help us beginners.

Live Free or Die!

Brian Whildin (engineer@techinter.com)

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: First coil finished!

Cc:

Bcc:

X-Attachments:

At 07:43 PM 8/30/97 GMT, you wrote:

>From: engineer <engineer@techinter.com>

>Date: Sat, 30 Aug 1997 14:43:31 -0500

>Subject: First coil finished!

>Organization: DeVry Institute of Technology

>

> Well I've finally got my first coil finished!  It's very small, but I

>guess everyone has to start somewhere.  I have a 3 1/2" diameter

>secondary, 10" high powered by a 6500V@23mA neon transformer. I can only

>draw about an 8" spark to a grounded bar.  I think I may need a smaller

>toroid because I get optimum spark length when my primary is tapped at

>the very end of the coil.

Congratulations!

First spark is always great.

My advice would be to add a couple primary turns rather than use

a smaller toroid. Ive added turns to my primary twice. Thats

why I always tell every one to make plenty of turns in their

primary. You will eventually need them.

If your toroid is several inches above your secondary you

can lower it down a bit closer. This will raise your

resonant frequency a little and give you better corona

protection. If you are real close to tune then this

might be enough. Other wise add more primary turns.

A larger primary also helps your gap operate better

by showing a higher impedance to surge currents.

If your primary cap is smaller than about .01 mfd you

can also lower your primary resonant frequency by adding a little

more capacitance. This will also increase primary circuit energy

and if your gap is working well more spark output.

If you already have close to .01 then thats all your transformer

can handle.

TTYL!