To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: polyeth seeting prices

Cc:

Bcc:

X-Attachments:

I've forwarded this message to the list for

Bill Pollack due to problems between hotmail

and the Tesla-2 list server ISP.

<begin forwarded message>

From: "Bill Pollack" <arcstarter@hotmail.com>

, pmarshal@eesus.jnj.com

Subject: Polyeth sheeting prices

Date: Tue, 02 Sep 1997 12:56:20 PDT

Paul and Mike,

I just called Cadillac plastics (they have a branch in Cincinnati)

about pricing and availability of the following:

4 foot by 8 foot plastic, 1/16 inch thick:

Low-density polyethylene:  $17.00/ea, 2 in stock

Polypropylene:             $16.67/ea, 3 in stock

These are also available in a customized roll form, which is basically

anything less than 1/16 inch thick, in whatever width and length I

specify.  I didn't pursure pricing.  There might be a minimum.

The roll stuff sounds especially good for caps, since it has already

been proven it can be rolled tightly!

So - how many should I get? :)

Other thickness are available.

The calculator says if I made a huge single sheet cap out of a 4x8 sheet

- it'd give me something like 0.036 uF!  A multi-sheet unit would be a

bit more compact and practical...  A cube of metal/plastic layers about

7x7x7 inches under HV oil might do it.

Cadillac Plastics

1-800-CPG-1000

1-800-274-1000

(Mike - post this to tesla-2 if you think this info would be of general

interest...)

-Capmaster Bill

To: tesla-2@emachine.com

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: Polyeth sheeting prices

Cc:

Bcc:

X-Attachments:

At 12:56 PM 9/2/97 PDT, you wrote:

>Paul and Mike,

> I just called Cadillac plastics (they have a branch in Cincinnati)

>about pricing and availability of the following:

>

>4 foot by 8 foot plastic, 1/16 inch thick:

>Low-density polyethylene:  $17.00/ea, 2 in stock

>Polypropylene:             $16.67/ea, 3 in stock

Either would be good for caps. If going with a rolled cap

stick with polyethylene. Polypropylene is considerably stiffer

and wont roll as well. 1/16 inch is 62 mils. In a single thick

layer this would be barely adequate for 12kv and not enough for

15 kv.

Since I believe you are going to be using a 15kv/120ma neon

you are going to have problems combining high capacitance with adequate

voltage rating.

A single cap is just not going to be robust enough to handle

15kv. You are going to have to use at least two caps in series.

Its a lot easier to build a robust 7500 volt cap than a 15kv one.

I would recommend starting with a pair of .02 mfd caps. Use these

two caps in series to give you .01 mfd. Start with these and get

your coil running. After the coil is running well build another pair

of .02 caps and also put them in series. Then parallel the two series

strings. This will give you .02 mfd with an adequate voltage rating.

I know this means building 4 caps but its not so bad once you get the first

one done. And if they are plate caps you can put them all in the same tank.

>Other thickness are available.

I would go for 10 mil LDPE or PP and use 3 sheets per dielectric layer.

This will give you 30 mils total per cap but with a much better voltage

rating than a single 30 mil layer.

Two such caps in series will give you the equivalent of 60 mils

and a good HV rating.

>The calculator says if I made a huge single sheet cap out of a 4x8 sheet

>- it'd give me something like 0.036 uF!  A multi-sheet unit would be a

>bit more compact and practical...  A cube of metal/plastic layers about

>7x7x7 inches under HV oil might do it.

Since you indicate a desire for a plate cap lets go with that idea.

With 7 X 7 plates as you indicated above and 3 X 10 mil poly it

will take 30 plates to reach .02 mfd. Now remember we need to run two

of these in series so thats 60 plates total. And you will need 29 dielectric

layers between those 30 plates and each layer has 3 poly squares

so thats 87 poly squares per cap or 174 total. The poly squares need

to be larger than the plates and since we are planning 15kv operation

make them 10 X 10. That gives us a 1 1/2 inch margin all around the plates.

Make your tank large enough to contain 4 of these units. When you

get the first two done you can start using them to work out the bugs

in your coil while building the 2nd set. When the 2nd series string is

completed pop them in the tank and parallel then up with the first two.

This would make a quite good performing cap. It will be lots of work

but if you are after the best performance a flat plate cap is the way to go.

I recommend the use of regular transformer oil. Especially if the cap

isnt going to be vacuum pumped to remove air. Orient the plates vertically

so the air can rise out on its own and break it in slowly with low

voltage and narrow gaps. Running a new cap at full power will only

cause it to break down due to trapped air. I know I've done it.

If you cant get regular transformer oil you can use mineral oil.

I have used it successfully in several caps. But due to its higher

viscosity it will take longer for the trapped air to escape.

Well what do you think? Cap building is usually the most time

consuming and the most costly part of coil construction.

Think on it and if you have further questions, comments, or criticism

email me.

TTYL!

Mike Hammer

mhammer@misslink.net

From: tesla-2@emachine.com (Tesla-2)

Subject: Cap's Breakdown Voltage

Date: Wed,  3 Sep 1997 13:30:08 GMT

Organization: The Energy Machine Information System 504-734-0526

From: "Paul Marshall" <PMARSHAL@ETHCINC1.SSW.JNJ.COM>

Date: 03 Sep 1997 08:30:08 EST

Subject: Cap's Breakdown Voltage

I'm a little confused about the breakdown voltage of the dielectric

used in cap's. I just read an article posted to tesla2 that stated a

laminated thickness of poly had a higher breakdown voltage than

multiple sheets amounting to the same thickness. Is this correct ?

If so Why ?

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: Cap's Breakdown Voltage

Cc:

Bcc:

X-Attachments:

At 01:30 PM 9/3/97 GMT, you wrote:

>From: "Paul Marshall" <PMARSHAL@ETHCINC1.SSW.JNJ.COM>

>Date: 03 Sep 1997 08:30:08 EST

>Subject: Cap's Breakdown Voltage

>

>

>     I'm a little confused about the breakdown voltage of the dielectric

>     used in cap's. I just read an article posted to tesla2 that stated a

>     laminated thickness of poly had a higher breakdown voltage than

>     multiple sheets amounting to the same thickness. Is this correct ?

>     If so Why ?

>

>

>

Paul,

Yes its true. A single thick piece of poly has a much poorer

voltage rating than the same thickness composed of multiple layers.

The per mil rating goes up as the sheets get thinner. Commercial

cap makers often use polypropylene films from .5 to 1 mil thick.

This gives the highest rating PER MIL of material. They then use

a sufficient number of caps in series to get the desired voltage

rating.

Also consider that a single thick layer can be rendered useless

from a single flaw. But if that dielectric was comprised of many

layers of thinner material any defects would be unlikely to line

up.

Ive known people who have blown up caps made with 60 mil poly

using 12kv neons. Such a cap is really only good for 9-10 KV.

Yet my plate cap handles 12kv easily with only 48 mils of poly.

The secret is many layers of thinner poly and the use of several caps

in series. Dont try to make a single cap handle any more than 10 kv

if you want it to survive.

TTYL!

Mike Hammer

mhammer@misslink.net

From: tesla-2@emachine.com (Tesla-2)

Subject: % of eff

Date: Fri,  5 Sep 1997 17:38:12 GMT

Organization: The Energy Machine Information System 504-734-0526

From: "Paul Marshall" <PMARSHAL@ETHCINC1.SSW.JNJ.COM>

Date: 05 Sep 1997 12:38:12 EST

Subject: % of eff

How efficient is the transfer of energy from the primary to the

secondary. Provide that the gap, cap, & coil are all operating at

optima.

i.e. 15,000 v .1 amp = 1,500 watts, what could I expect the secondary

output wattage to be ?

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: % of eff

Cc:

Bcc:

X-Attachments:

At 05:38 PM 9/5/97 GMT, you wrote:

>From: "Paul Marshall" <PMARSHAL@ETHCINC1.SSW.JNJ.COM>

>Date: 05 Sep 1997 12:38:12 EST

>Subject: % of eff

>

>

>     How efficient is the transfer of energy from the primary to the

>     secondary. Provide that the gap, cap, & coil are all operating at

>     optima.

>

I dont know exactly what the efficiency is but it is quite low.

A Tesla coil is not like a transformer with its very tight

coupling and iron core with efficient power transfer.

A Tesla coil is very loosely coupled with an air core so the actual amount of energy transfer is low.

>     i.e. 15,000 v .1 amp = 1,500 watts, what could I expect the secondary

>     output wattage to be ?

There is no way to accurately predict what the output voltage

from the secondary will be. All the various equations that have

been derived fail to predict how a coil really operates.

There are just too many variables involved and too many

non linear elements involved to make any kind of prediction.

TTYL!

Mike Hammer

mhammer@misslink.net

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: poly color

Cc:

Bcc:

X-Attachments:

Message forwarded to Tesla-2 list

for Bill Pollack.

Mike,

It seems that poly sheeting is available in clear and black color.

Does it matter?  I suppose clear would be better...?

How far should the plastic layers extend past the edges of the metal

plates?

Paul and I will probably duplicate your efforts by building four 0.02

uf units using multiple layers of 0.006 or 0.008 for a total plastic

thickness of 0.048 inches per plate.

But you mentioned using three layers of 0.010 per plate.  Is this

sufficient for 7500 volt units?  We may be forced to use 5 x 0.006

layers.

Ignoring the overlap past the edges of the plates, this design could be

accomplished in a cube about 4.5 inches on an edge, and 96 plates with

0.048 between each plate.  Do you have any comments about this

relatively large number of layers?  Would we be better off using larger

plates with fewer layers?

I'll be buying some plastic tomorrow, and possibly some wire on

saturday.  At work we have one xray transformer waiting to be drained,

so I think I'll be able to get some oil.

I've been printing off all these discussions so hopefully I won't forget

anything.

-Bill

arcstarter@hotmail.com

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: Poly color?

Cc:

Bcc:

X-Attachments:

At 07:08 AM 9/4/97 PDT, you wrote:

>Mike,

> It seems that poly sheeting is available in clear and black color.

>

> Does it matter?  I suppose clear would be better...?

DO NOT USE BLACK POLY!  In many instances black plastics are

colored with carbon. I don't need to tell you what that would

be like. Some people have been lucky and had success with black

plastics but you can't tell ahead of time what coloring agent

was used. Best to be safe and avoid colored materials.

This goes also for secondary forms. Black ABS pipe is common

but again carbon may be used. Use only white PVC.

>

> How far should the plastic layers extend past the edges of the metal

>plates?

At least 1 inch. That means you need 9 X 9 poly squares

to cover 7 X 7 plates. I have been increasing this to

1 1/2 inches in my latest caps. That would take 10 X 10

poly squares. Most common failure mode in stacked plate

caps is arcing from plate to plate ALONG THE SURFACE of the

dielectric. Not through it. The only way to decrease the chances

of this happening is to increase the size of the poly squares to

increase the distance an arc must travel to reach to opposite plate.

So allow plenty of margin. I would go for at least 1 1/2 inches.

Also to remember to round the corners and deburr the edges of all

your plates. Sharp edges and corners promote corona which leads to an arc.

>

> Paul and I will probably duplicate your efforts by building four 0.02

>uf units using multiple layers of 0.006 or 0.008 for a total plastic

>thickness of 0.048 inches per plate.

>

>But you mentioned using three layers of 0.010 per plate.  Is this

>sufficient for 7500 volt units?  We may be forced to use 5 x 0.006

>layers.

Yes go to 30 mils of material for each dielectric layer. For

a total of 60 mils. The 48 mil value I spoke of for my caps

was for 12kv use. Since you are using 15 kv you need a little

more.

8 mil poly would be great. 4 sheets for each layer = 32.

Two in series yields 64 mils.

>

>Ignoring the overlap past the edges of the plates, this design could be

>accomplished in a cube about 4.5 inches on an edge, and 96 plates with

>0.048 between each plate.  Do you have any comments about this

>relatively large number of layers?  Would we be better off using larger

>plates with fewer layers?

Well the only problem with small plates is the increase in the number

of dielectric layers. This becomes a problem when you need to cut

5 or 6 poly squares for each layer. I found the best range of plate

size to be 7 X 7 or 8 X 8. They dont have to be square. Rectangular

plates are fine.

Large plates lessen the needed number of plates and poly layers

but the cap soon gets quite large and the large plates seem to

trap more air and its more difficult to remove.

TTYL!

Mike Hammer

mhammer@misslink.net

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: Poly color?

Date: Sat,  6 Sep 1997 21:16:33 GMT

Organization: The Energy Machine Information System 504-734-0526

From: "Thomas McGahee" <tom_mcgahee@sigmais.com>

Date: Sat, 6 Sep 1997 17:16:33 -0400

Subject: Re: Poly color?

> From: Tesla-2 <tesla-2@emachine.com>

> To: tom_mcgahee@sigmais.com

> Subject: Re: Poly color?

> Date: Saturday, September 06, 1997 12:36 PM

>

> From: Mike Hammer <mhammer@misslink.net>

> Date: Sat, 06 Sep 1997 11:36:14 -0500

> Subject: Re: Poly color?

>

Mike, Bill, and other Tesla freaks,

This was a very good reply, and covered most of the bases. I have

added a few additional comments that I hope will be useful to the

discussion.

Fr. Tom McGahee

> At 07:08 AM 9/4/97 PDT, you wrote:

> >Mike,

> > It seems that poly sheeting is available in clear and black color.

> >

> > Does it matter?  I suppose clear would be better...?

>

> DO NOT USE BLACK POLY!  In many instances black plastics are

> colored with carbon. I don't need to tell you what that would

> be like. Some people have been lucky and had success with black

> plastics but you can't tell ahead of time what coloring agent

> was used. Best to be safe and avoid colored materials.

>

> This goes also for secondary forms. Black ABS pipe is common

> but again carbon may be used. Use only white PVC.

>

> >

> > How far should the plastic layers extend past the edges of the metal

> >plates?

>

> At least 1 inch. That means you need 9 X 9 poly squares

> to cover 7 X 7 plates. I have been increasing this to

> 1 1/2 inches in my latest caps. That would take 10 X 10

> poly squares. Most common failure mode in stacked plate

> caps is arcing from plate to plate ALONG THE SURFACE of the

> dielectric. Not through it. The only way to decrease the chances

> of this happening is to increase the size of the poly squares to

> increase the distance an arc must travel to reach to opposite plate.

>

Another common failure mode is due to trapped air. Corona in the air

bubbles creates hot spots that can actually melt the poly. If you are

making a flat capacitor, do all that you can to ensure that air does

not get trapped. Some methods that are useful include a) building the

capacitor under oil. b) painting or dipping oil onto poly and plates

as assembly progresses. c) using a vacuum to remove air. d) using

temporary pressure to remove trapped air prior to sealing up the

unit. e) using a permanent compression frame to keep air out and keep

plates close together.

All of the above steps will also help alleviate the surface

conduction problem that Mike pointed out.

> So allow plenty of margin. I would go for at least 1 1/2.

>

> Also to remember to round the corners and deburr the edges of all

> your plates. Sharp edges and corners promote corona whick leads to

an arc.

>

Sharp edges can also puncture the thinner thicknesses of poly. This

can be a particular problem when the dielectric between plates is

only a single thin layer. It is one reason why I do not like poly

thinner than 8 mils.

>

> >

> > Paul and I will probably duplicate your efforts by building four 0.02

> >uf units using multiple layers of 0.006 or 0.008 for a total plastic

> >thickness of 0.048 inches per plate.

> >

> >But you mentioned using three layers of 0.010 per plate.  Is this

> >sufficient for 7500 volt units?  We may be forced to use 5 x 0.006

> >layers.

>

> Yes go to 30 mils of material for each dielectric layer. For

> a total of 60 mils. The 48 mil value I spoke of for my caps

> was for 12kv use. Since you are using 15 kv you need a little  more.

>

> 8 mil poly would be great. 4 sheets for each layer = 32.

> Two in series yields 64 mils.

>

With 8 mil poly you can also use another technique that enhances the

Q or Quality Factor of the capacitor. This involves placing free

floating (un-connected) aluminum foil plates between each sheet of

poly. These aluminum foil sheets would be 1 1/2" smaller than the

poly along every side. Assume P is Poly, F is Flashing, and A is

Aluminum foil: the capacitor assembly would be like this:

FPAPAPAPFPAPAPAPFPAPAPAPFPAPAPAPF etc.

The problem with this method lies in keeping the free-floating pieces

centered during construction and aftwerwards. It also involves more

material and definitely more work. Therefore it is seldom used by

coilers (who, for all their willingness to work like dogs, still have

a lazy streak in there somewhere). This method should not be used on

homemade caps that use poly thinner than 8 mils, as it can reduce the

voltage rating due to the possibility of holes in multiple layers now

lining up electrically.

>

>

> >

> >Ignoring the overlap past the edges of the plates, this design could be

> >accomplished in a cube about 4.5 inches on an edge, and 96 plates with

> >0.048 between each plate.  Do you have any comments about this

> >relatively large number of layers?  Would we be better off using larger

> >plates with fewer layers?

>

> Well the only problem with small plates is the increase in the number

> of dielectric layers. This becomes a problem when you need to cut

> 5 or 6 poly squares for each layer. I found the best range of plate

> size to be 7 X 7 or 8 X 8. They dont have to be square. Rectangular

> plates are fine.

>

> Large plates lessen the needed number of plates and poly layers

> but the cap soon gets quite large and the large plates seem to

> trap more air and its more difficult to remove.

>

Larger plates reduce the amount of wasted poly (poly that does not

directly contribute to the CAPACITANCE). Since for a given voltage

rating the size of the edge zone is fairly fixed, a larger plate size

translates to less wasted poly. A note about the GEOMETRY of the

plates. The shape that gives the least amount of poly waste is one in

which the OVERLAPPING area of the metal plates is square. As the

overlapping plate area becomes more elongated, then the percentage of

wasted poly to useful poly increases.

By the way, when computing capacitance, the area of the plates

referred to in the equation refers only to the area that OVERLAPS

from plate to plate. Active plate area is NOT equal to the area of

the metal plate, but only the overlapping area.

There are several factors that drive the actual size that you will

use for your capacitor plates:

1) the size that the poly and the flashing come in. You generally

want to cut the poly down such that there is minimal wasted poly.

Assume a sheet that is 48" x 96". This can yield two 48" squares, 8

24" squares, or 32 12" squares. The 12" squares would not be too bad

as far as container size, while the 24" squares would probably

require the construction of a custom enclosure.

(If using poly sheet that comes in long rolls, it is generally the

width of the poly that must be considered.)

The factors COMMON to 48 and 96 are 2x2x2x2x3. So if you decide to

make rectangular rather than square poly sections, then your choices

would have to involve ratios like (2/2) 2/3 2/4 2/6 2/8 2/12 and

2/24. These ratios would allow you to cut the poly with no leftover

material.  Note that for the 48x96 sheet the most desireable cuts are

on the ratio 2/2 2/3 and 2/4. This just gives you the ratios... you

would have to sit down with paper and pencil to determine the cut

sequence such that there was no leftover material.

2) the size of readily available enclosures. In many of my

capacitors, this becomes an important factor, as it will directly

relate to cost, time, and how the finished product looks. When

measuring enclosures, do NOT go by the measurements given by the

manufacturer. Bring along a tape measure and be keenly aware of

things like tapered walls and curved edges and non-flat bottoms and

the like. Find an enclosure that can take the weight and pressure of

the oil. Does it seal completely? How thick is the material? What is

it made out of? Will the TOP survive the presence of oil and possibly

a little heat? Did you take into account the ends of the flashing?

How about electrical connections? Is there room for the oil to

surround the capacitor? How much is the assembled unit going to

weigh? How unwieldy will the completed unit be once it is filled with

oil? If you pick it up, is it going to change shape on you? (If so,

mount it on a piece of plywood and secure it so that you can move it

around by lifting the plywood.)

In general, do not attempt to stack one enclosure on top of another,

as the weight of the cap+oil may distort the case on the bottom.

You have to think about ALL these factors BEFORE you cut up your

precious poly or spend money on an enclosure or parts for an

enclosure.

A word to the wise: when possible use things like Rubbermaid

Polyethelene enclosures. Two smaller enclosures may be more practical

in the long run that one humongous enclosure. It is far better to

have four .04 mfd 6KV capacitors than one .01 mfd 24KV capacitor.

(See appended post below for full details).

Here is a post I sent to the Tesla list, but which contains

information pertinent to the current discussion of capacitors on the

Tesla-2 list.

Fr. Tom McGahee

----------

> From: Tesla List <tesla@pupman.com>

> To: 'Tesla List' <tesla@poodle.pupman.com>

> Subject: rolled caps

> Date: Saturday, September 06, 1997 12:46 AM

>

>

> From:         Mad Coiler[SMTP:tesla_coiler@hotmail.com]

> Sent:         Friday, September 05, 1997 4:16 PM

> To:   tesla@pupman.com

> Subject:      rolled caps

>

> To all that have made rolled caps,

>

> I have just recieved my order of 30 mill LDPE sheets. I plan on making

> the dielectric three sheets thick. This would be 90 mills, and I hoped

> this would be enough for 12000V.

>  Question (1): Should you put aluminum flashing between each sheet of

> the dielectric or just set three sheets together without anything

> inbetween.

>  Question (2): I assume the plastic should be clean. Is LDPE sensitive

> to soap, or should a damp cloth work fine to get the dust off?

>  Question (3): Will three layers of 30 mill also work for 15kV or do I

> need more?

>

> Mad Coiler

>

Dear Mad Coiler,

If you want to get the absolute MOST out of the 30 mil polyethelene,

then make THREE caps using 30 mil sheets and then connect the

capacitors in series to get the best voltage rating. As has been

stated on this list a number of times in different ways by various

individuals, the voltage rating PER MIL of dielectric insulation

INCREASES as the thickness of the dielectric DECREASES. Note that I

said voltage rating PER MIL. I also want to say that the Q or Quality

Factor of a capacitor also increases with decreasing dielectric

thickness. Thus, three 30 mil caps in series will be MUCH better than

a single 90 mil cap. The trade-off is you need more flashing or

aluminum foil and more containers and oil. For the slight increase in

cost, you get a MUCH more ROBUST TC capacitor, and one that is more

versatile and higher quality. (Better discharge characteristics).

Something that we have to keep in mind is that the destruction of

capacitors in Tesla coil use is very much related to breakdown due to

HV corona. In GENERAL, you should not attempt to make the dielectric

breakdown rating of a single capacitor section greater than 12KV when

it is to be used in Tesla coil service.

Above 12KV, the corona gets so wild that it will destroy the cap,

especially around the edges. Avoid sharp edges. Curve corners.

30 mil thickness is a really good thickness to use. By sticking to

just a single 30 mil thickness PER SECTION, you will make the job of

rolling the capacitor much easier. (Remember that you use two

dielectric sheets in a rolled cap, so you will be rolling a total of

60 mils thickness...)

Roll just the two uncleaned sheets of plastic up around a 1" diameter

PVC pipe to get an idea of the diameter of the completed roll. Add a

bit for the flashing, and then get a PVC pipe whose INSIDE diameter

is large enough to accomodate your cap.

Another advantage of building several lower voltage and higher

capacity caps is that you have greater freedom to put them in series

and parallel combinations. For example, four .04 MFD at 6KV

capacitors could be connected singly and in combinations to make:

one  .01   mfd at 24KV (four in series)

one  .0133 mfd at 18KV (three in series)

one  .016  mfd at 15KV (two in parallel with two in series)

two  .02   mfd at 12KV (two in series)

one  .0266 mfd at 9KV (two in parallel with one in series)

one  .03   mfd at 8KV (three in parallel with one in series)

four .04   mfd at 6KV (single)

one  .04   mfd at 12KV (two series groups connected in parallel)

one  .06   mfd at 6KV (two in series with one in parallel)

one  .08   mfd at 6KV (two in parallel)

one  .12   mfd at 6KV (three in parallel)

one  .16   mfd at 6KV (four in parallel)

As regards your question about cleaning, you can use alcohol to

clean. This has the advantages of being fast-drying and

non-contaminating. You can use regular isopropal rubbing alcohol

available at any pharmacy or drug store. I would not want soap

contaminating my LDPE, although I am sure that there are coilers out

there that have used soap and water successfully. But why even take

the chance? Besides, alcohol is something every coiler should have

around as a general purpose cleaning agent. Use stuff that is as

lint-free as possible in the cleaning process. Also, wash your hands

thoroughly and dry them before beginning the final assembly.

As you clean the LDPE it will usually develop a hefty static electric

charge. Not too dangerous, but it can suprise you if you are not

ready for it!! The staic electricity will make the LDPE a wonderful

dust magnet, so you should begin assembly as soon after cleaning as

possible.

Have all tie wraps and other stuff ready, and if at all possible,

have a good helper with you. Explain the procedure clearly before

beginning. Remember that once your hands are full with the rolling,

you can't easily point to something or get it yourself. I always get

the tie wraps linked up before I start rolling.

hope this helps.

Fr. Tom McGahee

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: % of eff

Date: Sun,  7 Sep 1997 17:15:30 GMT

Organization: The Energy Machine Information System 504-734-0526

From: Bert Hickman <bert.hickman@aquila.com>

Date: Sun, 07 Sep 1997 10:15:30 -0700

Subject: Re: % of eff

Organization: Stoneridge Engineering

Tesla-2 wrote:

>

> From: "Paul Marshall" <PMARSHAL@ETHCINC1.SSW.JNJ.COM>

> Date: 05 Sep 1997 12:38:12 EST

> Subject: % of eff

>

>      How efficient is the transfer of energy from the primary to the

>      secondary. Provide that the gap, cap, & coil are all operating at

>      optima.

>

>      i.e. 15,000 v .1 amp = 1,500 watts, what could I expect the secondary

>      output wattage to be ?

>

Paul,

That's a very tough question to answer! There's no easy way to measure

the output wattage coming off a Tesla Coil, since the output is the

simultaneous combination of streamer light, heat, sound, ionized and

excited gases, and EM radiation. It's even quite difficult to measure

input wattage accurately, since the load is highly inductive and the

current draw very non-sinusoidal.

However, recent experiments by a very knowledgeable coiler on the main

Tesla Builders List (Malcolm Watts, from New Zealand), have actually

verified the theoretical contention that the overall transfer of energy

from the resonant primary and secondary of a Tesla Coil can be quite

efficient. This was done by purposely preventing the coil from breaking

out (sparking), and then measuring the relative amounts of output

voltage as energy transferred back and forth between the primary and

secondary. Malcolm found that, for his 2-coil system, fully 87% of the

initial primary energy made it to the secondary coil even though the

magnetic coupling coefficient was only about 0.22. The transfer takes a

number of RF cycles to occur, and since the gap is firing at the same

time, energy is being lost during the transfer. Still, 87% IS

impressively efficient!

From a more practical standpoint, a 15 KV 100 MA power source should

permit you to hit 4 foot long streamers (and maybe more) IF your

secondary coil is 6" + in diameter, you use an optimally sized toroid,

and have a good set of air or vacuum cooled spark gaps for efficient

quenching.

Safe coiling to you, Paul!

-- Bert --

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Construction Secondary Coil.

Date: Sun,  7 Sep 1997 22:24:32 GMT

Organization: The Energy Machine Information System 504-734-0526

From: BadBowTie9@aol.com

Date: Sun, 7 Sep 1997 18:24:32 -0400 (EDT)

Subject: Construction Secondary Coil.

I just finished winding, my secondary coil, and Varnishing it, That only

took about 6 hours, the winding that is!

Now I'm looking fo sugestions or Tips on: Sealing the ends of the PVC, and

conections to the Primary coil, and the Torid. I have a few Ideas, just

looking for some input from Coilers!

Once again this is the First time I have built one so please respond in my

terms, a beginner that is! Thanks M.R.Howard

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: Construction Secondary Coil.

Cc:

Bcc:

X-Attachments:

At 10:24 PM 9/7/97 GMT, you wrote:

>From: BadBowTie9@aol.com

>Date: Sun, 7 Sep 1997 18:24:32 -0400 (EDT)

>Subject: Construction Secondary Coil.

>

> I just finished winding, my secondary coil, and varnishing it, That only

>took about 6 hours, the winding that is!

Gee. I wound my lastest secondary (4.5 inch diameter, 21 inch winding,

975 turns of AWG 24) in less than an hour. I cobbled together a

handcranked winder from some wood scraps and a broom handle.

Also made a cradle for the wire spool so it would unwind easily.

> Now I'm looking for suggestions or Tips on: Sealing the ends of the PVC, and

>conections to the Primary coil, and the Toroid. I have a few ideas, just

>looking for some input from Coilers!

What I do for the top end is cut a disk from wood about 3/4 inch

thick. I sand this round disk so its a snug fit inside the end

of the secondary form. I boil it in paraffin wax for a few minutes to

drive out the moisture but thats probably an unnecessary step.

I drill a hole in the center of this disk and mount a 1/4 bolt in the disk.

The threaded portion of the bolt sticks up in the air. The head of the

bolt is countersunk 1/4-1/2 inch into the wood disk. Fasten the bolt with a

pair of washers and a nut. Fill the countersink with silicone RTV so that it completely fills the hole. Then using RTV attach a disk of 1/8 inch thick

minimum plexiglass or Lexan to the back of the wood disk.

So now you have a wooden disk with a bolt sticking up out of it

with the bolt head covered with RTV and then covered with a plexiglass

disk. Its harder to describe than it actually is.

Here's a crude ascii drawing.

-

- 1/4 bolt

-

-

_______

|       | Wood disk

|_______|

|_______| Plexiglass disk

Roughen up the interior of the secondary form and epoxy the wood/plexi

disk into the top end of the secondary form. Plexiglass side down.

Run the top free end of the secondary wire up

to the 1/4 bolt and attach with a ring terminal. The connection of the

secondary wire to the bolt is done on the outside of the form.

Never inside. The RTV over the countersunk bolt head and the plexi

disk prevent arcing inside the secondary.

The bottom of the secondary winding is soldered to a small rectangular

plate cut from sheet copper or brass. Roughen up the PVC pipe form

and epoxy the plate to the outside of the secondary form.

The top bolt provides an easy method of attaching various

toploads to the coil.

I have used this method on 4 secondary coils so far and never

suffered any secondary breakdowns despite getting sparks more than

twice the length of my secondary. Longest spark to date with my

4 inch secondary is 48 inches.

For my primary coil tap connection I use a large alligator

clip to which Ive soldered extra curved copper jaws to increase

the contact area.

For a toroid I have had great success with flexible aluminum

dryer duct. The 3 inch diameter duct makes a great small toroid

about 15 inches in diameter. The 4 inch duct is great for running

higher power levels and will give you a toroid from 16-18 inches

in diameter. The 6 inch duct is definitely for higher power use

and makes a great 20 inch toroid for power levels about 1000 watts

or so. Use some solid aluminum pie pans mounted back to back in

the center of the duct ring. When you draw the pie pans together

it will grip the duct ring tightly and make a solid unit.

TTYL!

Mike Hammer

mhammer@misslink.net

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: Construction Secondary Coil.

Date: Mon,  8 Sep 1997 03:34:36 GMT

Organization: The Energy Machine Information System 504-734-0526

From: corr <corr@fullnet.net>

Date: Sun, 07 Sep 1997 21:34:36 -0600

Subject: Re: Construction Secondary Coil.

> I just finished winding, my secondary coil, and Varnishing it, That only

>took about 6 hours, the winding that is!

Thats fast for a first timer.  In the future, get a scheme to wind it.  I wound

a new 6" secondary yesterday in less than one hour.

> Now I'm looking fo sugestions or Tips on: Sealing the ends of the PVC, and

I used hot glue on the top of the pipe, with a piece of plexiglass to seal it.

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: progress

Cc:

Bcc:

X-Attachments:

Forwarded to Tesla-2 list for

Bill Pollack.

<Begin forwarded Message>

From: "Bill Pollack" <arcstarter@hotmail.com>

Subject: Progress

Date: Mon, 08 Sep 1997 13:16:01 PDT

Mike,

This weekend I finished winding my first tesla secondary!  I ended up

using 4.5 inch PVC pipe, and applied 24 Gauge wire.  Total coil length

is 22 inches.  Nice shiny red wire which cost $5 for 2000 feet!

I followed your advice and applied polyurethane varnish to the pipe

before winding wire onto it, and also after the wire was wound.  My

motorized winder (which I had from a previous project) was alreay set up

for 4.5 inch PVC.  The winding of the coil took about 40 minutes!  We

had a few "oops" in the winding but these were unwound and corrected.

I also acquired 1000 feet of 0.006 inch poly and one 4x8 foot roll of

0.060 polyethylene.

Can I get away with using kitchen grade aluminum foil for a

stacked-plate capacitor?  I know that some people use roof flashing,

which is of course much thicker, but also a bit harder to deal with.

I was planning on using threaded rods run through holes punched through

the foil as electrical connectors.  An occasional washer will make up

the difference in height between the foil vs foil+poly at the edges of

the cap.

Then I started thinking about building a RQ-style circular static spark

gap. Can I use 1/2 or 3/4 inch copper tubing inside a 4.5 inch PVC?  The

i.d. of the PVC is 4.125 inch.  I don't have any 6 inch and hate to buy

a 10-footer just to use one foot of it!

RF chokes - Now what'll I do?  I have 10 small ferrite toroid choke

cores, with an id of .875 or thereabouts.  I'll try using 5 on each side

of the neon - with as many turns on each core as I can physically fit.

Any thoughts?  Think this'll do it?  Do you have any idea what actual

inductance I'm shooting for?

More later!  Post this to tesla-2 if you think anyone else would be

interested.

-Bill

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: Progress

Cc:

Bcc:

X-Attachments:

At 01:16 PM 9/8/97 PDT, you wrote:

>Mike,

>This weekend I finished winding my first tesla secondary!  I ended up

>using 4.5 inch PVC pipe, and applied 24 Gauge wire.  Total coil length

>is 22 inches.  Nice shiny red wire which cost $5 for 2000 feet!

>

>I followed your advice and applied polyurethane varnish to the pipe

>before winding wire onto it, and also after the wire was wound.  My

>motorized winder (which I had from a previous project) was alreay set up

>for 4.5 inch PVC.  The winding of the coil took about 40 minutes!  We

>had a few "oops" in the winding but these were unwound and corrected.

The new secondary I just wound I did with a hand cranked winder I cobbled

together from some scrap and a broom handle. I wound it in about 50 minutes.

Did you bake the PVC a bit to dry it before applying the polyurethane

varnish? If you forgot its no big deal. But drying and sealing the

PVC will get you a couple extra inches of spark for practically nothing.

Also did you see the post I made to the list regarding sealing up

the top end? I used a piece of wood because I got lots of it.

Even better would be a 3/8 inch thick piece of plexiglass or

Lexan.

>

>I also acquired 1000 feet of 0.006 inch poly and one 4x8 foot roll of

>0.060 polyethylene.

>

>Can I get away with using kitchen grade aluminum foil for a

>stacked-plate capacitor?  I know that some people use roof flashing,

>which is of course much thicker, but also a bit harder to deal with.

Yes foil is fine but get the Reynolds heavy duty foil.

The normal weight foil is just too fragile. Avoid the store

and econo brands. The foil in these is often wrinkled.

Reynolds is usually nice and flat.

>I was planning on using threaded rods run through holes punched through

>the foil as electrical connectors.  An occasional washer will make up

>the difference in height between the foil vs foil+poly at the edges of

>the cap.

Actually its easier with foil plates to just bunch then together

and sandwich them between a couple aluminum strips. Use self tapping

screws to fasten the plates together and clamp the foil tabs.

>

>Then I started thinking about building a RQ-style circular static spark

>gap. Can I use 1/2 or 3/4 inch copper tubing inside a 4.5 inch PVC?  The

>i.d. of the PVC is 4.125 inch.  I don't have any 6 inch and hate to buy

>a 10-footer just to use one foot of it!

Yes my RQ type gap is in 4 inch PVC and you can fit ten electrodes

from 3/4 inch pipe inside. That makes 9 gaps. Epoxy a strip of plexiglass

in between the first and last electrode so the spark doesnt cheat

and arc directly from the first to last electrode.

>

>RF chokes - Now what'll I do?  I have 10 small ferrite toroid choke

>cores, with an id of .875 or thereabouts.  I'll try using 5 on each side

>of the neon - with as many turns on each core as I can physically fit.

>Any thoughts?  Think this'll do it?  Do you have any idea what actual

>inductance I'm shooting for?

Those are a little small. I used large surplus ferrite toroids

2.5 inches in diameter. You can use ferrite rod. Or you can wind

air core chokes on 2 inch PVC. You need from 2-10 mh.

My chokes are 2.5 mh and work well.

A 6 inch long winding of AWG 28 on 2 inch PVC will have about 430

turns and about 3.7mh of inductance.

Since these air core chokes are almost like mini secondary coils

its a good idea to run some series resistance with them to kill

the Q of the chokes and keep them from ringing. Anything from 300-1500

ohms will be fine. Use some hefty resistors. 5 watts is fine for a 30

ma neon but you may need 10 or more watts for a 60.

I dont need resistors with my chokes as they arent likely to ring

due to the low number of turns.

TTYL!

Mike Hammer

mhammer@misslink.net

From: tesla-2@emachine.com (Tesla-2)

Subject: RF choks

Date: Tue,  9 Sep 1997 17:33:12 GMT

Organization: The Energy Machine Information System 504-734-0526

From: "Paul Marshall" <PMARSHAL@ETHCINC1.SSW.JNJ.COM>

Date: 09 Sep 1997 12:33:12 EST

Subject: RF choks

I am building a test coil of a 4.5" dia. PVC tube. The power supply

will be variac controlled. The max power output will be 15Kv

at 180 ma. How should I design my Rf Chokes. Is there an equation or a

general rule to follow ? The test coil is wound using #22 gage wire to

a winding length of 22 1/2". The primary is #6 gage wire wound on a 15

degree slope, 15 to 17 turns. The cap's are in series an are computed

to be about .02Uf. If the testing goes well I plan to build a 6" dia.

coil and possibly a 10".

Please advise.

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: RF chokes

Cc:

Bcc:

X-Attachments:

At 05:33 PM 9/9/97 GMT, you wrote:

>From: "Paul Marshall" <PMARSHAL@ETHCINC1.SSW.JNJ.COM>

>Date: 09 Sep 1997 12:33:12 EST

>Subject: RF choks

>

>

>     I am building a test coil of a 4.5" dia. PVC tube. The power supply\

>     will be variac controlled. The max power output will be 15Kv

>     at 180 ma.

Paul,

Way too much power. Especially for a 4 incher. My own 4 inch

coil has been up to 48 inches with a single 12kv/90ma neon.

15kv/180ma will be very hard to process. All that power

will end up as heat, UV, and noise, not sparks. You would

need a very heavy duty gap to withstand that amount

of current.

I recommend going for 15kv/60. This power level will

get you in the 3 foot range for sparks.

> How should I design my Rf Chokes. Is there an equation or a

> general rule to follow ?

The general rule for RF chokes is 2-10 mh. I have had good

luck with 2.5 mh chokes wound on large surplus ferrite toroids.

Others have used air core chokes wound on 2 inch PVC.

RF chokes do not need large guage wire. AWG 26 or 28 is just

fine. A nice efficient high Q choke is NOT what is needed here.

Low Q and lossy chokes are just what we need.

In fact often times air core chokes with many turns can

act like mini secondary coils and ring up quite high

voltages. So adding a little series resistance to dampen

any oscillations is a good idea with air core chokes.

The value isnt critical. Anything from 300-1500 ohms

will kill the Q of the chokes and keep them from ringing.

Hefty resistors will be needed as your power level goes up.

Here is Wheeler's formula for air core solenoids.

Its quite accurate.

L(uH) = (r^2) * (N^2) / (9*r + 10*h)

where:

r = coil RADIUS in inches

N = number of turns

h = coil height in inches

>The test coil is wound using #22 gage wire t oa winding length of 22 1/2".

Exactly like my older 4 inch secondary. It maxed out at 45 inches

of spark. I replaced it with another 4 incher wound with 21 inches

of AWG 24. This secondary has produced up to 48 inches of spark

and looks like it will go above 50.

Your mileage may vary.

> The primary is #6 guage wire wound on a 15 degree slope, 15 to 17 turns.

> The cap's are in series an are computed to be about .02Uf.

Primary sounds fine.

Is this .02 mfd each so the series string is going to be .01 mfd?

If so thats fine and will match up well with a 15kv/60ma neon.

>If the testing goes well I plan to build a 6" dia.

>coil and possibly a 10".

>

Thats a good plan Paul. The larger coils are much more difficult

and the problems always seem to scale up with the size.

TTYL!

Mike Hammer

mhammer@misslink.net

From: tesla-2@emachine.com (Tesla-2)

Subject: New Coil

Date: Thu, 11 Sep 1997 09:20:21 GMT

Organization: The Energy Machine Information System 504-734-0526

From: Richard Smit <esmit@ilink.nis.za>

Date: Thu, 11 Sep 1997 11:20:21 +0200 (SAT)

Subject: New Coil

Hi,

I am desiding on building a new coil. Can anyone help giving me advice?

Here's the specs:

Secondary

Diameter: 6.4 inches

Height: 26.4 inches

Turns: 800

Frequency(unloaded): 186 kHz

Wire thickness: .033 inch

Inductance: 22.38 mH

Internal C: 32.7 F

Primary Coil

Diameter: 12.8 inches

Height: 5 inches

Transformer

Adjustable from 500Watts up to 5000 Watts(for future planning)

Give so much comments as you want.

Thanks in advance

------------------------------------------------------------------------

|Richard Smit  | Telephone: +27-525-32634                                |

|P.O.Box 566   | Email:     esmit@ilink.nis.za                           |

|Bultfontein   | Age:       17 years old                                 |

|9670          | Hobbies:   Tesla Coils; Computors; Paranormal; UFO's    |

|South Africa  | Birthday:  5 October 1979                               |

------------------------------------------------------------------------

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Coil Study

Date: Thu, 11 Sep 1997 18:15:17 GMT

Organization: The Energy Machine Information System 504-734-0526

To: mhammer@misslink.net

From: Richard Smit <esmit@ilink.nis.za>

Date: Thu, 11 Sep 1997 20:15:17 +0200 (SAT)

Subject: Coil Study

Hi,

I am doing some research in Tesla Coil Maths. I want to design a simulation

programme that you enter your parameters and the program will give a

simulation on your coil.

What am I looking for? Well, basically I want to compare Data between, say

two identical transformers and the performance it will give with various

size Tesla Coils.

Will you please enter the data below(if it is not too much to ask for)

Remember! All inputs should be in inches. And if you designed 3 TC's then

enter the data in on line for example: VOLTAGE: 15000  ;  30000 ;  10000;

and so forth.

Entry form:

*Things like the internal C and Inductance of coil isn't optional, that I

can calculate*

Transformer

-----------

VOLTAGE: _______________________________________________________

AMPS: __________________________________________________________

Capacitor

---------

SIZE(uF): ______________________________________________________

NUMBER OF PLATES: ______________________________________________

SORT OF DIELECTRIC: ____________________________________________

THICKNESS OF DIELECTRIC: _______________________________________

SIZE OF CAP ITSELF(LENGTH * WIDTH):____________________________

________________________________________________________________

Primary Coil

------------

DIAMETER:

________________________________________________________

HEIGHT:

__________________________________________________________

TURNS:

___________________________________________________________

A.W.G WIRE:

______________________________________________________

Secondary coil

--------------

DIAMETER:

________________________________________________________

HEIGHT:

__________________________________________________________

TURNS:

___________________________________________________________

A.W.G. WIRE:

_____________________________________________________

INDUCTANCE:

______________________________________________________

INTERNAL CAPACITANCE:

____________________________________________

Toroid

-------

SIZE:

____________________________________________________________

Sparks

------

SPARK LENGTH:

_____________________________________________________

Spark Gap

---------

TYPE OF SPARK GAP:

________________________________________________

NUMBER OF ELECTRODES:

_____________________________________________

SPACING BETWEEN ELECTRODES:

_______________________________________

NUMBER OF STUDS ON ROTARY:

________________________________________

SPEED OF MOTOR(R.P.M):

____________________________________________

Thanks in advance.

------------------------------------------------------------------------

|Richard Smit  | Telephone: +27-525-32634                                |

|P.O.Box 566   | Email:     esmit@ilink.nis.za                           |

|Bultfontein   | Age:       17 years old                                 |

|9670          | Hobbies:   Tesla Coils; Computors; Paranormal; UFO's    |

|South Africa  | Birthday:  5 October 1979                               |

------------------------------------------------------------------------

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: New Coil

Cc:

Bcc:

X-Attachments:

At 09:20 AM 9/11/97 GMT, you wrote:

>From: Richard Smit <esmit@ilink.nis.za>

>Date: Thu, 11 Sep 1997 11:20:21 +0200 (SAT)

>Subject: New Coil

>

>Hi,

>

>I am desiding on building a new coil. Can anyone help giving me advice?

Ok Richard lets see what you have here.

>Here's the specs:

>        Secondary

>                Diameter: 6.4 inches

>                Height: 26.4 inches

>                Turns: 800

>                Frequency(unloaded): 186 kHz

>                Wire thickness: .033 inch

>                Inductance: 22.38 mH

>                Internal C: 32.7 F

Hmm. Looks like your wire is AWG 20. Thats a little

large for a 6 inch secondary but its usable. AWG 22 is

normally recommended for a 6 incher.

Using your diameter of 6.4 inches and a winding length of 26.4

inches with AWG 20 wire I come up with the following specs:

H/D ratio        4.125

Turns            790  (this depends greatly on insulation thickness)

Freq (unloaded)  328 KHZ

Inductance      21.8 mh

Self C           10.6 pf

So it looks like you did OK except for your self C calculation.

Your way high value gave you an incorrect resonant frequency.

The method to determine self C is with Medhurst's equation.

Medhurst's formula:    C  = K x D

where:  C = capacitance in picofarads

K = constant which depends on the ratio of the coil height to diameter

x = means multiply K times D

D = solenoidal coil diameter in centimeters

H = coil height in centimeters

H/D       K

5.0     0.81

4.5     0.77

4.0     0.72

3.5     0.67

3.0     0.61

2.5     0.56

2.0     0.50

1.5     0.47

1.0     0.46

The value of K is rather linear so you can interpolate for

H/D ratios between the values in the table.

So for your secondary above we have H/D ratio of 4.125

which gives a K value of .7425.

So C = K X D

C = .7425 X (6.4 X 2.54)  (2.54 to convert inches to centimeters)

C = .7425 X 14.33

C = 10.6 pf

Let me propose an alternative secondary coil for you

based on AWG 22 wire.

Diameter        6.5 in  (nominal for 6 inch PVC)

winding length  26 inches

H/D ratio       4

Turns           975

Inductance      34.7 mh

Self C          10.6 pf

Freq unloaded   259 KHZ

This I believe would be a better performer.

Notice that the self C doesnt change. Its based solely on

the height and diameter of the winding.

>        Primary Coil

>                Diameter: 12.8 inches

>                Height: 5 inches

If this is to be a solenoid form primary I recommend

a larger diameter to limit problems with over coupling.

Something around 14 inches diameter seems sensible.

And you should have at least 12 turns. 14 would be better.

A coil this large and with the power level you propose would do

well with a flat spiral primary.

Leave about 1.25-1.5 inch space between the inner turn and the secondary

and space the turns 1/4 inch apart. Wind with 1/4 inch copper tubing.

Again 12-14 turns are about right.

TTYL!

Mike Hammer

mhammer@misslink.net

From: tesla-2@emachine.com (Tesla-2)

Subject: Glass caps with polyethylene

Date: Fri, 12 Sep 1997 18:05:16 GMT

Organization: The Energy Machine Information System 504-734-0526

From: Richard Smit <esmit@ilink.nis.za>

Date: Fri, 12 Sep 1997 20:05:16 +0200 (SAT)

Subject: Glass caps with polyethylene

Hi,

I was wondering! Glass is a bad dielectric for a Tesla Coil but what if you

take a thin piece of polyethylene and place it between the foil and glass.

Will it still be a bad cap or will it be better because of the poly?

Thanks.

------------------------------------------------------------------------

|Richard Smit  | Telephone: +27-525-32634                                |

|P.O.Box 566   | Email:     esmit@ilink.nis.za                           |

|Bultfontein   | Age:       17 years old                                 |

|9670          | Hobbies:   Tesla Coils; Computors; Paranormal; UFO's    |

|South Africa  | Birthday:  5 October 1979                               |

------------------------------------------------------------------------

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: Glass caps with polyethylene

Cc:

Bcc:

X-Attachments:

At 06:05 PM 9/12/97 GMT, you wrote:

>From: Richard Smit <esmit@ilink.nis.za>

>Date: Fri, 12 Sep 1997 20:05:16 +0200 (SAT)

>Subject: Glass caps with polyethylene

>

>Hi,

>

>I was wondering! Glass is a bad dielectric for a Tesla Coil but what if you

>take a thin piece of polyethylene and place it between the foil and glass.

>Will it still be a bad cap or will it be better because of the poly?

>

>Thanks.

It will still be a bad cap. The poly will have very little

effect except for lowering the dielectric constant of

the glass somewhat. The glass will still dissipate much

of your primary energy as heat. Avoid glass at all costs.

TTYL!

Mike Hammer

mhammer@misslink.net

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: Coil Study

Cc:

Bcc:

X-Attachments:

At 06:15 PM 9/11/97 GMT, you wrote:

>From: Richard Smit <esmit@ilink.nis.za>

>Date: Thu, 11 Sep 1997 20:15:17 +0200 (SAT)

>Subject: Coil Study

>

>Hi,

>

>I am doing some research in Tesla Coil Maths. I want to design a simulation

>programme that you enter your parameters and the program will give a

>simulation on your coil.

>

>What am I looking for? Well, basically I want to compare Data between, say

>two identical transformers and the performance it will give with various

>size Tesla Coils.

>

>Will you please enter the data below(if it is not too much to ask for)

>

>Remember! All inputs should be in inches. And if you designed 3 TC's then

>enter the data in on line for example: VOLTAGE: 15000  ;  30000 ;  10000;

>and so forth.

>

>Entry form:

>*Things like the internal C and Inductance of coil isn't optional, that I

>can calculate*

OK Richard here's the info on my coil. This might

be a good way to see what everyone is up to coil wise.

I added a couple extra categories for additional useful information.

Transformer

-----------

VOLTAGE: 12kv

AMPS:    90ma

Capacitor

---------

SIZE(uF): .01 mfd

NUMBER OF PLATES: 16 plates aluminum flashing  10 X 10 inches

SORT OF DIELECTRIC: 6 mil polyethylene

THICKNESS OF DIELECTRIC: ten 6 mil sheets for each layer 60 mil total

SIZE OF CAP ITSELF(LENGTH * WIDTH): tank is 13 X 17 x 3

Type of oil used: Mineral oil

Primary Coil

------------

DIAMETER: 8 inches at bottom 12 inches at top. tapered solenoid

HEIGHT: 11 inches

Turns: 17

Turn spacing: 5/8 inch

A.W.G WIRE: solid AWG 6 copper

Secondary coil

--------------

DIAMETER: 4.5

HEIGHT: 22

TURNS: 756

A.W.G. WIRE: 22

INDUCTANCE: 12.1 mh

INTERNAL CAPACITANCE: 7.9 pf

Resonant Frequency  : 512 KHZ unloaded   323 KHZ with 19 X 6 toroid

Toroid

-------

SIZE: 19 X 6 flexible aluminum duct

Sparks

------

SPARK LENGTH: maximum spark with this configuration was 42 inches

Spark Gap

---------

TYPE OF SPARK GAP: RQ type cylinder gap

NUMBER OF ELECTRODES: 10 (9 gaps)

SPACING BETWEEN ELECTRODES: .025 (.225 total)

NUMBER OF STUDS ON ROTARY:  NA

SPEED OF MOTOR(R.P.M): NA

Comments

--------

I have made some changes to this coil in my never ending

quest for bigger sparks. I have paralleled my two plate caps (.01 and .0077)

to give me .0177 mfd. I have also wound another 4.5 secondary with a lower

resonant frequency so I can tap in more primary turns.

The specs on the new secondary are:

Dia             4.5 inches

Winding length  21 inches

Turns           975

Wire            AWG 24

Resonant freq   395 KHZ unloaded  254 KHZ with 19 X 6 toroid

Inductance      20.9 mh

Self C          7.8 pf

Results of this change have been good. Maximum spark is now up

to 48 inches with the same 12kv/90ma power level.

Future plans include a larger gap to handle more power

and a 6 inch diameter secondary with a flat spiral primary.

TTYL!

Mike Hammer

mhammer@misslink.net

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: New Coil

Date: Sat, 13 Sep 1997 21:19:22 GMT

Organization: The Energy Machine Information System 504-734-0526

From: Richard Smit <esmit@ilink.nis.za>

Date: Sat, 13 Sep 1997 23:19:22 +0200 (SAT)

Subject: Re: New Coil

Mike,

How do you calculate K or where do you get it if it is read from a chart or so?

Thanks for your help. The only thing that I will have a problem with is to

find the transformer I need. If I will ever get one cheap.

Thanks

>The method to determine self C is with Medhurst's equation.

>

> Medhurst's formula:    C  = K x D

>

>where:  C = capacitance in picofarads

>        K = constant which depends on the ratio of the coil height to diameter

>        x = means multiply K times D

>        D = solenoidal coil diameter in centimeters

>        H = coil height in centimeters

>

>        H/D       K

>        5.0     0.81

>        4.5     0.77

>        4.0     0.72

>        3.5     0.67

>        3.0     0.61

>        2.5     0.56

>        2.0     0.50

>        1.5     0.47

>        1.0     0.46

>

>The value of K is rather linear so you can interpolate for

>H/D ratios between the values in the table.

>

>So for your secondary above we have H/D ratio of 4.125

>which gives a K value of .7425.

>So C = K X D

>   C = .7425 X (6.4 X 2.54)  (2.54 to convert inches to centimeters)

>   C = .7425 X 14.33

>   C = 10.6 pf

>

>

>

>

>Let me propose an alternative secondary coil for you

>base on AWG 22 wire.

>

>Diameter        6.5 in  (nominal for 6 inch PVC)

>winding length  26 inches

>H/D ratio       4

>Turns           975

>Inductance      34.7 mh

>Self C          10.6 pf

>Freq unloaded   259 KHZ

>

>This I believe would be a better performer.

>Notice that the self C doesnt change. Its based solely on

>the height and diameter of the winding.

>

>

>>        Primary Coil

>>                Diameter: 12.8 inches

>>                Height: 5 inches

>

>If this is to be a solenoid form primary I recommend

>a larger diamer to limit problems with over coupling.

>Something around 14 inches diameter seems sensible.

>And you should have at least 12 turns. 14 would be better.

>

>A coil this large and with the power level you propose would do

>well with a flat spiral primary.

>Leave about 1.25-1.5 inch space between the inner turn and the secondary

>and spaced turns 1/4 apart. Wind with 1/4 inch copper tubing.

>Again 12-14 turns are about right.

>

>

>TTYL!

>Mike Hammer

>mhammer@misslink.net

>

>

>

------------------------------------------------------------------------

|Richard Smit  | Telephone: +27-525-32634                                |

|P.O.Box 566   | Email:     esmit@ilink.nis.za                           |

|Bultfontein   | Age:       17 years old                                 |

|9670          | Hobbies:   Tesla Coils; Computors; Paranormal; UFO's    |

|South Africa  | Birthday:  5 October 1979                               |

------------------------------------------------------------------------

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Confusion(NOT THE TOPIC)

Date: Sat, 13 Sep 1997 21:19:31 GMT

Organization: The Energy Machine Information System 504-734-0526

From: Richard Smit <esmit@ilink.nis.za>

Date: Sat, 13 Sep 1997 23:19:31 +0200 (SAT)

Subject: Confusion(NOT THE TOPIC)

Hi,

Well, this is off the topic but I find things very hard on this list.

Don't you think there are toooooooo many Richard's on this Tesla coil lists

for example there is RICHARD Hull, RICHARD T. Quick, RICHARD Smit, RICHARD

Wayen, RICHARD Wall, and there is a lot more that I can't come by now. When

they write in a message Richard, I think that theyre talking to me or so on.

See yah.

------------------------------------------------------------------------

|Richard Smit  | Telephone: +27-525-32634                                |

|P.O.Box 566   | Email:     esmit@ilink.nis.za                           |

|Bultfontein   | Age:       17 years old                                 |

|9670          | Hobbies:   Tesla Coils; Computors; Paranormal; UFO's    |

|South Africa  | Birthday:  5 October 1979                               |

------------------------------------------------------------------------

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: New Coil

Cc:

Bcc:

X-Attachments:

At 09:19 PM 9/13/97 GMT, you wrote:

>From: Richard Smit <esmit@ilink.nis.za>

>Date: Sat, 13 Sep 1997 23:19:22 +0200 (SAT)

>Subject: Re: New Coil

>

>Mike,

>

>How do you calculate K or where do you get it if it is read from a chart or so?

>

>Thanks for your help. The only thing that I will have a problem with is to

>find the transformer i need. If I will ever get one cheap.

>

Richard,

You cannot calculate the K value in Medhurst's equation. It is a constant

based on the H/D ratio of the coil. It is supplied in a table

such as I posted here with the equation.

>

>

>>The method to determine self C is with Medhurst's equation.

>>

>> Medhurst's formula:    C  = K x D

>>

>>where:  C = capacitance in picofarads

>>        K = constant which depends on the ratio of the coil height to diameter

>>        x = means multiply K times D

>>        D = solenoidal coil diameter in centimeters

>>        H = coil height in centimeters

>>

>>        H/D       K

>>        5.0     0.81

>>        4.5     0.77

>>        4.0     0.72

>>        3.5     0.67

>>        3.0     0.61

>>        2.5     0.56

>>        2.0     0.50

>>        1.5     0.47

>>        1.0     0.46

>>

>>The value of K is rather linear so you can interpolate for

>>H/D ratios between the values in the table.

>>

TTYL!

Mike Hammer

mhammer@misslink.net

From: tesla-2@emachine.com (Tesla-2)

Subject: My first coil.

Date: Mon, 15 Sep 1997 23:44:01 GMT

Organization: The Energy Machine Information System 504-734-0526

From: "Thomas G." <tgoetzma@execpc.com>

Date: Mon, 15 Sep 1997 18:44:01 -0500

Subject: My first coil.

First off, I'd just like to say "hello" to everybody on the list.

I'm starting to build my first coil, within the next few months. I

should have all winter to work on it (if I can wait that long, anybody

tried firing thiers up in the winter? It seems that the lower humidity

might actually have advantages)

My current plans are for a 3 or 4 inch diameter coil. I'm going out

tomorrow to some sign shops to look for 9 or 12 kv transformers in the

30 to 60 ma range.

I'm currently planning to use simple low-Q Salt Water caps. I was

planning on making them in Snapple jars. The glass is pretty thick, and

should be able to hold (I hope) without having to wire any in series. I

was looking for about .006uf (I think that should be good enough), and

I'm guessing that will take anywhere from 8 to 10 bottles.

If anyone can help with the design of the caps, I'd appreciate anything.

Another problem I can't find an answer for is what sort of wire to

connect the parts of the tank together with. My first idea was 1/2"

braided wire, but I've since learned that braided isn't good for the

reason that Coax isn't good for primaries, the surface or "skin"

currents. Also, what should I use to connect to the primary? (Probably

3/8" copper tubing) Will a hose clamp work well?

For a spark gap I was planning on using 4 - 6 series gaps mounted in a

line, and shoot either compressed or vacuum exhaust air down the length.

What can anyone recommend about the material to use for the contacts or

what distances to start with?

Well, that about covers the tank circuit, but I'm sure I'll have more

questions about the secondary, and filtering of the incomming (more like

blocking outgoing RF, acutally) power.

Thanx VERY much for your time/answers.

Charlie Goetzman

tgoetma@execpc.com

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: unbeliever

Date: Tue, 16 Sep 1997 04:42:39 GMT

Organization: The Energy Machine Information System 504-734-0526

From: Tony Minchew <wamjam@wt.net>

Date: Mon, 15 Sep 1997 23:42:39 -0500

Subject: Re: unbeliever

Organization: Minchew Mfg.

Hi Mike,

I have a friend in class (ITT) who seems to be fairly knowledgable about

Tesla.  From all outward appearances the guy is a genious but he does

not adhere to some of your findings ie. the Wardencliffe Tower not

working as a wireless energy transmitter and the idea of dielectrics not

being made of glass.

I don't know beans about it one way or the other but he and I will be

working on projects for a long time to come and I want to insure our

success as much as possible.

He reads many of the High Voltage Press publications maybe you could

point to one of them as a reference.

If you can spare the time please expound on the Tower and glass

dielectrics.

Thank you very much,

Tony Minchew

Houston, Texas

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: My first coil.

Date: Tue, 16 Sep 1997 04:43:22 GMT

Organization: The Energy Machine Information System 504-734-0526

From: corr <corr@fullnet.net>

Date: Mon, 15 Sep 1997 22:43:22 -0600

Subject: Re: My first coil.

>My current plans are for a 3 or 4 inch diameter coil. I'm going out

>tomorrow to some sign shops to look for 9 or 12 kv transformers in the

>30 to 60 ma range.

Definitely you want to start off with a 4" secondary.  I built my first coil

during the month of July, with big hopes on future coils.  I plan for a 10"

secondary next summer, but a would extremely recommend you start small and work your way up.  About two months later, you'll want to build a 6" most likely

just as I did.

>I'm currently planning to use simple low-Q Salt Water caps. I was

>planning on making them in Snapple jars. The glass is pretty thick, and

>should be able to hold (I hope) without having to wire any in series. I

>was looking for about .006uf (I think that should be good enough), and

>I'm guessing that will take anywhere from 8 to 10 bottles.

I have had bad luck with snapple bottles.  I tried 4 of them at first. They arced

from the foil up to the top.  Use beer bottles for better results.  If you don't

drink (and I don't), just drive around looking at the sides of the roads. I used

9 SW cap bottles for my 4".  While the snapple bottles are thick enough, the

necks are not tall enough.  As for design, most SW caps tell you to wrap the

bottles in aluminum foil.  While this works, it is hard to get a good contact.

I originally had a capacitance of only 1680 picofared.  After putting the bottles

into a SW bath, the capacitance increased to 8160 picofareds and my output

tripled.  (Although even with the best SW caps, I have only gotten 20" max

with my 6" secondary).  They work good at first, but you will definitely want

to make some better caps later.

>Another problem I can't find an answer for is what sort of wire to

>connect the parts of the tank together with. My first idea was 1/2"

>braided wire, but I've since learned that braided isn't good for the

>reason that Coax isn't good for primarys, the surface or "skin"

>currents. Also, what should I use to connect to the primary? (Probably

>3/8" copper tubing) Will a hose clamp work well?

I use #12 solid copper for 15kv at 60ma.

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: My first coil and hello all

Date: Tue, 16 Sep 1997 04:44:55 GMT

Organization: The Energy Machine Information System 504-734-0526

From: "Vince Cothran" <vince@techie.com>

Date: Tue, 16 Sep 1997 00:44:55 -0400

Subject: My first coil and hello all

Hello to everyone on the list.  I am working on my first coil too.  I

joined the list about a month ago and have been enjoying reading the posts.

I'm 45, live in Columbia, South Carolina and also enjoy amateur radio, R/C

aircraft and sailing.  I've been fascinated with high voltage and tesla

coils since boyhood and fondly remember reading articles about the subject

in Popular Science and Popular Electronics magazines.  Several years ago

someone gave me a 15kv 60ma NST which I promptly made into a jacobs ladder.

Sadly it's primary function though has been door-stop and dust collector

but since finding you all that has changed<G>

Here's the specs so far for my coil in progress:

Transformer - 15kv 60ma NST

Primary - 15 degree saucer of 1/4" refrigeration tubing, 10" ID 24" OD

about 14 turns

Cap - aluminum flashing and poly plate cap with two sections of .02 mfd

each, in one tank of acrylic-lined-plywood with an acrylic separator between

the two sections. 35 6" x 8" plates vertically mounted in each section with 6 layers of 6 mil poly between each plate, all four leads brought out through the tank walls.

Secondary - 6.5" OD x 25" of 22AWG heavy enamel magnet wire coated with

acrylic spray

Gap - RQ style static maybe vacuum type

Chokes - still pondering as with the toroid

I acquired some 6.5" OD white PVC at the local building supply house but it

has a rather thick cellular core, about 1/4" I guess.  The trade name for

it is Conex I believe.  I prepared it by baking it in an oven at work at

about 130-140 degrees for 24 hours and sprayed it while it was still warm

with some mil-spec acrylic conformal coating called HumiSeal that I picked

up for a song several years ago at a hamfest.  Well I started wondering

about the breakdown characteristics of this cellular core....the outer skin

of PVC is  pretty thin....i guess 40 to 50 mils......a few days ago I ran

across some 6" green solid PVC so I bought a piece of it.  Now it's down to

the wire and I'm ready to wind the secondary.  Does anyone know which

material would be best?

Thanks,

Vince

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: My first coil.

Cc:

Bcc:

X-Attachments:

At 11:44 PM 9/15/97 GMT, you wrote:

>From: "Thomas G." <tgoetzma@execpc.com>

>Date: Mon, 15 Sep 1997 18:44:01 -0500

>Subject: My first coil.

>

>First off, I'd just like to say "hello" to everybody on the list.

Hello and welcome aboard.

>

>I'm starting to build my first coil, within the next few months. I

>should have all winter to work on it (if I can wait that long, anybody

>tried firing thiers up in the winter? It seems that the lower humidity

>might actually have advantages)

Actually I never seemed to be able to notice that much difference.

Maybe now that I am getting longer sparks up to 48 inches I may

notice some difference. Sometimes when its really humid I do

get poorer results. But there are lots factors affecting

spark length besides humidity.

>

>My current plans are for a 3 or 4 inch diameter coil. I'm going out

>tomorrow to some sign shops to look for 9 or 12 kv transformers in the

>30 to 60 ma range.

Good choices. 9kv is excellent for a small coil and for first

timers. 12kv gets you a little more juice but is still

manageable. 15kv neons are pretty hard on caps so

its best to avoid them at first. I never have felt the need

to move up to 15kv as I have all the power I need with 12kv.

>

>I'm currently planning to use simple low-Q Salt Water caps. I was

>planning on making them in Snapple jars. The glass is pretty thick, and

>should be able to hold (I hope) without having to wire any in series. I

>was looking for about .006uf (I think that should be good enough), and

>I'm guessing that will take anywhere from 8 to 10 bottles.

Good old 12oz beer bottles work well. Dont use the thin clear

glass ones like used by Miller. You want long necks and I had

good results with brown glass bottles. You will get about 700 pf

per 12 oz bottle.

I built mine by filling the bottle with SW to within an inch of

the top. The space above the SW is filled with oil. Motor will

do for this. A long carriage bolt extends down in to SW. I

attached a large washer to the bolt with a couple nuts and attached

the washer to the bottle top with RTV silicone. Leave a 1/2 inch

or so of the threaded portion above the washer to make an electrical

connection using a ring terminal.

Lay a strip of aluminum foil in the bottom of a plastic dishpan or

a bucket. Run the foil across the bottom and up one side. Set the

bottles into the dispan on top of the foil and fill with SW equal to

the SW level inside the bottles. Make the other plate connection to

the foil strip.

SW caps are a cheap and quick way to get sparking. But start thinking

about building a polyethylene dielectric cap as soon as possible.

>

>If anyone can help with the design of the caps, I'd appreciate anything.

>

>Another problem I can't find an answer for is what sort of wire to

>connect the parts of the tank together with. My first idea was 1/2"

>braided wire, but I've since learned that braided isn't good for the

>resason that Coax isn't good for primarys, the surface or "skin"

>currents. Also, what should I use to connect to the primary? (Probably

>3/8" copper tubing) Will a hose clamp work well?

I first used AWG 14 stranded wire to hook up the various

pieces in the primary tank circuit. This worked fine at lower

powers. When I started running with a 12kv/60ma neon (720 VA)

I changed to AWG 8 for all connections. Solid wire is preferable

to stranded but isnt nearly as flexible. Stranded wire isnt too

bad unless the strands are twisted.

My primary coil terminates in a large ring terminal. This connects

to a 1/4 20 bolt. My tank circuit lead also attaches to this bolt.

This makes a solid mechanical connection. All my connections

are either bolted or soldered.

For the primary tap I use a large copper alligator clip.

>

>For a spark gap I was planning on using 4 - 6 series gaps mounted in a

>line, and shoot either compressed or vacum exhaust air down the length.

>What can anyone recommend about the material to use for the contacts or

>what distances to start with?

Hard copper water pipe does fine. 3/4 inch pipe is heavy enough.

Make the electrodes 3 or 4 inches long.

The number of gaps depends on your supply transformer. I had

good luck with 6 gaps (7 electrodes) spaced at .025 inch for

9kv neons. 8 gaps spaced at .025 worked well for 12kv.

I suspect 10 such gaps will work well for 15kv.

This gives total gap spacings of .150 for 9kv, .200 for 12kv

and .250 for 15kv.

These are pretty safe gap spacings. Once your coil is operating

smoothly at these gap spacings you can increase them a little

to get a little more power. But be aware that every time you increase

your gap distance you increase the strain on your caps and transformer.

Too wide a gap and one or both of these will fail.

You dont need very wide gaps to get long sparks. I have never

run any more than .225 inch of gap (9 X .025) and have succeeded

in getting 48 inch sparks with a single 12kv/90ma neon.

>

>

>Well, that about covers the tank circuit, but I'm sure I'll have more

>questions about the secondary, and filtering of the incomming (more like

>blocking outgoing RF, acutally) power.

>

>Thanx VERY much for your time/answers.

>

>Charlie Goetzman

>tgoetma@execpc.com

Keep the questions coming Charlie.

Good luck on your neon hunt.

TTYL!

Mike Hammer

mhammer@misslink.net

From: tesla-2@emachine.com (Tesla-2)

Subject: Hi everybody!

Date: Tue, 16 Sep 1997 19:08:02 GMT

Organization: The Energy Machine Information System 504-734-0526

From: "Eriksson" <jan-erik.eriksson@mbox2.swipnet.se>

Date: Tue, 16 Sep 1997 21:08:02 +0200

Subject: Hi everybody!

Hi first of all I want to say hello to everybody in this list.

My name is Christopher Eriksson and I live in Sweden.

I've been interested in HV for a time now, built some

simple Hv generators and such things. But now I want to build my first coil,

I've downloaded a program called WinTesla and I got one question,

doesn't the "power source" matter? I mean..like the neon signt transformer

or oil ignition transformer or what you use..because,

it must matter if you use a very large coil with a tiny little transformer..

And how do I know what transformer I should use?

well..my English isn't very good but I hope you'll understand.

Thanks

Christopher Eriksson

(have you seen any swedish coilers?, I havn't )

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: My first coil and hello all

Cc:

Bcc:

X-Attachments:

At 04:44 AM 9/16/97 GMT, you wrote:

>From: "Vince Cothran" <vince@techie.com>

>Date: Tue, 16 Sep 1997 00:44:55 -0400

>Subject: My first coil and hello all

>

>

>Hello to everyone on the list.  I am working on my first coil too.  I

>joined the list about a month ago and have been enjoying reading the posts.

>I'm 45, live in Columbia, South Carolina and also enjoy amateur radio, R/C

>aircraft and sailing.  I've been fascinated with high voltage and tesla

>coils since boyhood and fondly remember reading articles about the subject

>in Popular Science and Popular Electronics magazines.  Several years ago

>someone gave me a 15kv 60ma NST which I promptly made into a jacobs ladder.

> Sadly it's primary function though has been door-stop and dust collector

>but since finding you all that has changed<G>

>

>Here's the specs so far for my coil in progress:

>

>Transformer - 15kv 60ma NST

>

>Primary - 15 degree saucer of 1/4" refrigeration tubing, 10" ID 24" OD

>about 14 turns

>

>Cap - aluminum flashing and poly plate cap with two sections of .02 mfd

>each, in one tank of acrylic-lined-plywood with an acrylic separator

>between the two sections. 35 6" x 8" plates vertically mounted in each

> section with 6 layers of 6 mil poly between each plate, all four leads

>brought out through the tank walls.

Vince that sounds like a really great cap. I bet its a dynamite

performer. 6 layers of 6 mil is 36 mils per cap X 2 caps equals

72 mils total with .01 mfd capacitance. That should be a robust

cap and work well with your 15kv/60 neon.

What kind of oil did you use for this cap and did you do anything

special about air bubbles?

>

>

>Secondary - 6.5" OD x 25" of 22AWG heavy enamel magnet wire coated with

>acrylic spray

>

>Gap - RQ style static maybe vacuum type

For 15kv go for 10 gaps (11 electrodes) spaced about .025 inch.

3/4 hard copper pipe 3 or 4 inches long will be fine.

This will give you a maximum of .250 of gap which is plenty

to start with. In fact you will want to use only 4 or 5

of the available gaps to start with. This will keep the primary

power low until you get your cap broken in and your coil in tune.

>

>Chokes - still pondering as with the toroid

For chokes anything that provides from 2 - 10 mfh will be

just fine. Realize that these chokes are not meant to filter

out RF. That would take a more extensive filter system. But

a complex filter system is not needed. You only need to knock

down the level of the RF pulses to a manageable level and back

this up with a properly set safety gap.

I use 125 turns of AWG 26 on a large surplus ferrite toroid.

This results in about 2.5 mh of inductance.

Air core chokes can be wound on 2 inch PVC pipe. 430 turns

of AWG 28 on 2 inch PVC will give a 6 inch winding and

about 3.7 mh of inductance. The value of inductance

is not critical. Air core chokes with lots of turns

should be used with a little series resistance to prevent

them from ringing like mini-Tesla coils. Power resistors

with values from 300-3000 ohms work well. Value is not

critical. Its only purpose is to kill the Q of the chokes

and dampen any oscillations.

For a toroid flexible aluminum dryer duct around

two back to back pie pans makes a neat quick toroid.

The 4 inch duct makes a good toroid for a 4 inch secondary.

The 6 inch is good for higher power levels and would make a good

toroid for your 6 inch secondary.

>

>I acquired some 6.5" OD white PVC at the local building supply house but it

>has a rather thick cellular core, about 1/4" I guess.  The trade name for

>it is Conex I believe.  I prepared it by baking it in an oven at work at

>about 130-140 degrees for 24 hours and sprayed it while it was still warm

>with some mil-spec acrylic conformal coating called HumiSeal that I picked

>up for a song several years ago at a hamfest.  Well I started wondering

>about the breakdown characteristics of this cellular core....the outer skin

>of PVC is  pretty thin....i guess 40 to 50 mils......a few days ago I ran

>across some 6" green solid PVC so I bought a piece of it.  Now it's down to

>the wire and I'm ready to wind the secondary.  Does anyone know which

>material would be best?

>

I had good results from two secondary coils made with the celluar core

PVC pipe. Either one would work well. I would think the celluar

core pipe might even be better. Less lossy PVC.

My latest 4 inch secondary has withstood 48 inch discharges from

a winding only 21 inches long.

TTYL!

Mike Hammer

mhammer@misslink.net

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: My first coil.

Date: Tue, 16 Sep 1997 21:44:58 GMT

Organization: The Energy Machine Information System 504-734-0526

From: corr <corr@fullnet.net>

Date: Tue, 16 Sep 1997 15:44:58 -0600

Subject: Re: My first coil.

>Good choices. 9kv is excellent for a small coil and for first

>timers. 12kv gets you a little more juice but is still

>manageable. 15kv neons are pretty hard on caps so

>its best to avoid them at first. I never have felt the need

>to move up to 15kv as I have all the power I need with 12kv.

And something else to stress is you will want more current

later on.  Voltage is not really all that important.  If you can,

get two neons of the same voltage so you can parallel them

for more current later, or just get a single 60ma neon.

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: unbeliever

Cc:

Bcc:

X-Attachments:

At 04:42 AM 9/16/97 GMT, you wrote:

>From: Tony Minchew <wamjam@wt.net>

>Date: Mon, 15 Sep 1997 23:42:39 -0500

>Subject: Re: unbeliever

>Organization: Minchew Mfg.

>

>Hi Mike,

>

>I have a friend in class (ITT) who seems to be fairly knowledgable about

>Tesla.  From all outward appearances the guy is a genious but he does

>not adhere to some of your findings ie. the Wardencliffe Tower not

>working as a wireless energy transmitter and the idea of dielectrics not

>being made of glass.

>I don't know beans about it one way or the other but he and I will be

>working on projects for a long time to come and I want to insure our

>success as much as possible.

>He reads many of the High Voltage Press publications maybe you could

>point to one of them as a reference.

>If you can spare the time please expound on the Tower and glass

>dielctrics.

>

>Thank you very much,

>Tony Minchew

>Houston, Texas

>

>

Well Tony I doubt anything I can say will change

your friends mind. I have run into a lot of Tesla fanatics

and as a general rule they wont listen to reason.

This is a little off topic for this list but since you asked

I will reply to it here.

In my opinion many of the titles published by High Voltage Press

should be classified as fiction.

If you want to read what Tesla intended to do in his own words

an excellent work by Lelend Anderson is:

_NIKOLA TESLA ON HIS WORK WITH ALTERNATING CURRENTS AND THEIR

APPLICATION TO WIRELESS TELEGRAPHY, TELEPHONY, AND TRANSMISSION

OF POWER_

N. Tesla, edited by Leland I. Anderson, published in 1992 by Sun

Publishing, Div. of Boyle & Anderson, Denver, CO., 80219. Library

of Congress Catalog #92-60482, ISBN 0-9632652-0-2, paper 237pp

This is the transcript of Tesla's deposition for one of his

court cases. In this he outlines what he DID do in his experiments.

Not what the legends said he did.

But realize that Tesla believed his method of power transmission

was viable and would be able to transmit commercial levels

of power anywhere on the planet.

His stay in Colorado Springs was initially only to build a

large coil. Once that was done he tried to determine what the

best way to build one would be and how best to get the maximum

power from it. His experiments were mainly concerned

with building radio equipment as many of his experiments were

testing various kinds of receivers. His goal didnt change to

power transmission until he thought he observed stationary waves

being produced by a thunderstorm.

He had thought in producing these waves he could get around

the inverse square law. This law cites that as the distance

from a transmitter to receiver is doubled the energy collected

by the receiver drops to one fourth.

But we can argue the point endlessly as Tesla never got

the Wardenclyffe plant on line.

Perhaps maybe we should define "wireless transmission of power".

Any radio transmitter sends power wirelessly. Go to a local

AM radio transmitter with a small light bulb and a tuned circuit

tuned to the stations transmitting frequency. You will easily

be able to light the light bulb. But drive 1/2 a mile away and the

same experiment will fail. Why? The inverse square law.

This is why Tesla's scheme would fail. Without the existance

of his "stationary waves" his equipment is nothing more than a radio

transmitter. But a very good one. If Wardenclyffe had become operational

it would of wiped the name of Marconi from history. Tesla's

equipment was decades ahead of Marconi's crude copies.

Now as to glass as a dielectric. Any insulator can be used as a

dielectric. Each one has different characteristics making it

suitable or unsuitable for different applications.

Glass is a good insulator and has a high dielectric constant.

Seems like it would make a good cap. It does. For DC. For AC

use and especially for RF use glass is terrible. It absorbs

RF energy and turns it to heat. Dont believe it? Stick a glass

in the microwave for 30 seconds. Itll be too hot to handle.

Now stick a polyethylene container in the microwave for the same

amount of time. Itll hardly be warm.

Modern plastics like polyethylene or polypropylene are really

the best materials there are for RF pulse caps. Even a relatively

good material like mylar has hundreds of times the RF dissipation

of polyethylene.

TTYL!

Mike Hammer

mhammer@misslink.net

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: Hi everybody!

Cc:

Bcc:

X-Attachments:

At 07:08 PM 9/16/97 GMT, you wrote:

>From: "Eriksson" <jan-erik.eriksson@mbox2.swipnet.se>

>Date: Tue, 16 Sep 1997 21:08:02 +0200

>Subject: Hi everybody!

>

>Hi first of all I want to say hello to everybody in this list.

>My name is Christopher Eriksson and I live in Sweden.

>Ive been interested in HV for a time now, built some

>simple Hv generators and such things. But now I want to build my firstcoil,

>Ive downloaded a program called WinTesla and I got one question,

>doesnt the "power source" matter? I mean..like the neon sign transformer

>or oil ignition transformer or what you use..because,

>it must matter if you use a very large coil with a tiny little transformer.

>And how do I know what transformer i should use?

Yes the power source or transformer matters a great deal.

Often our choice of what transformer to use is limited.

The choice is sometimes made for us. The transformer must

be large enough to charge the primary cap. So yes the transformer

is very important.

If you have lots of transformers to choose from then

select one based on the size of coil you are building. You

will want more power available for larger coils. Larger

coils will usually have larger capacitors that require a

larger transformer to charge.

If you can give me some idea of what size of coil you are

building I can give some more specific answers.

>well..my english isnt very good but I hope youll understand.

>Thanks

>Christopher Eriksson

Your English is just fine.

>(have you seen any swedish coilers?, I havnt) )

Hmm. Havnt seen any here in Illinois. What does one look like? :)

TTYL!

Mike Hammer

mhammer@misslink.net

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: My first coil.

Date: Wed, 17 Sep 1997 00:20:03 GMT

Organization: The Energy Machine Information System 504-734-0526

From: "Thomas McGahee" <tom_mcgahee@sigmais.com>

Date: Tue, 16 Sep 1997 20:20:03 -0400

Subject: Re: My first coil.

----------

> From: Tesla-2 <tesla-2@emachine.com>

> To: tom_mcgahee@sigmais.com

> Subject: Re: My first coil.

> Date: Tuesday, September 16, 1997 5:44 PM

>

> From: corr <corr@fullnet.net>

> Date: Tue, 16 Sep 1997 15:44:58 -0600

> Subject: Re: My first coil.

>

> >Good choices. 9kv is excellent for a small coil and for first

> >timers. 12kv gets you a little more juice but is still

> >manageable. 15kv neons are pretty hard on caps so

> >its best to avoid them at first. I never have felt the need

> >to move up to 15kv as I have all the power I need with 12kv.

>

> And something else to stress is you will want more current

> later on.  Voltage is not really all that important.  If you can,

> get two neons of the same voltage so you can parrallel them

> for more current later, or just get a single 60ma neon.

>

>

Good advice, Corr, but since this is a forum for new

coilers, and some of them might not know all the

elements that go into choosing the proper voltage and

current values for your neons, let me add some additional

information.

Actually, higher voltage is **VERY** desireable, but just

really nasty to work with!! The reason why higher voltage

is desireable is that we are really interested in the amount

of total POWER being processed by the Tesla Coil primary.

Power is basically P=E*I   and this formula can be

massaged and re-arranged to yield P=I^2*R   and also

P=E^2/R     You will notice that doubling the voltage

or current level will QUADRUPLE the power for a given

value of primary resistance. Very desireable!!

Now, the voltage is pretty well directly related to the

voltage from the transformer, whereas the current in

the above equation is NOT the transformer current, but

the current in the Tesla primary coil. Transformer current

will relate to how fast the capacitor can be charged up

to its working voltage.

The flip side of higher voltages is that they are much

harder to work with. The extra corona eats up caps

like you wouldn't believe. Working with a 15KV neon

transformer requires caps that are rated at about 38KV.

That's because you have to account for the PEAK value

of the voltage and also the resonant rise.

In general, you can increase processed power in several

different ways. You can increase the voltage. You can

increase the capacitance (this usually means increased

transformer current...). You can increase the number

of Breaks Per Second (BPS). Note than increasing breaks

per second can be controlled by increasing the charging

current, increasing the charging voltage (different

sides of the same coin actually), decreasing the gap

distance in a static gap, or varying the speed and

gap spacing of a rotary. The phase angle of the rotary

also enters in here if the rotary gap is synchronous.

I guess that the point I am trying to make in all this

is that there are many factors that are going to

affect the power you get into your primary. It is just

a bit too simplistic to say that we want more current

and less voltage.

For the beginning coiler it is better perhaps to increase

the capacitance and parallel in another neon. Better

in the sense of easier and less expensive. Higher

voltages are desireable, but come with many more

problems and difficulties. A major difficulty is the

extra care and work and materials that have to go into

creating capacitors that can survive the higher

voltages. Of course, the same thing can be said about

the design of capacitors that are able to survive

higher break rates. These guys can get so hot they can

literally explode. So proceed with caution.

Hope this helps.

Fr. Tom McGahee

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Transformers

Date: Wed, 17 Sep 1997 09:05:22 GMT

Organization: The Energy Machine Information System 504-734-0526

From: "Eriksson" <jan-erik.eriksson@mbox2.swipnet.se>

Date: Wed, 17 Sep 1997 11:05:22 +0200

Subject: Transformers

Hi again, thanx for the reply!

It's not easy to get a transformer here..it's quite hard actually. But

will ordinary lumiunous tube transformers do it? Whats the major difference

between them and a neon sign transformer? Sorry for all these questions

but I don't know so much about TCs yet..

/Christopher Eriksson

(How the swedish coilers look?..heh..well...i dunno, I havent seen one

myself)

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: My homepage

Date: Thu, 18 Sep 1997 06:35:31 GMT

Organization: The Energy Machine Information System 504-734-0526

From: Richard Smit <esmit@ilink.nis.za>

Date: Thu, 18 Sep 1997 08:35:31 +0200 (SAT)

Subject: My homepage

Hi,

go check my homepage at http://www.nis.za/~esmit/

** It's still under construction but go ***

see yah

hope you guys like it.

------------------------------------------------------------------------

|Richard Smit  | Telephone: +27-525-32634                                |

|P.O.Box 566   | Email:     esmit@ilink.nis.za                           |

|Bultfontein   | Age:       17 years old                                 |

|9670          | Hobbies:   Tesla Coils; FIREWALKING; Computers; UFO's   |

|South Africa  | Birthday:  5 October 1979                               |

------------------------------------------------------------------------

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: Transformers

Cc:

Bcc:

X-Attachments:

At 09:05 AM 9/17/97 GMT, you wrote:

>From: "Eriksson" <jan-erik.eriksson@mbox2.swipnet.se>

>Date: Wed, 17 Sep 1997 11:05:22 +0200

>Subject: Transformers

>

>Hi again, thanx for the reply!

>Its not easy to get a transformer here..its quite hard actually. But,

>will ordinary luminous tube transformers do it? Whats the major difference

>between them and a neon sign transformer? Sorry for all these questions,

>but I dont know so much about TCs yet..

>/Christopher Eriksson

>(How the swedish coilers look?..heh..well...i dunno, I havent seen one

>myself)

>

>

Well unless Im missing something here luminous tube transformers

are neon sign transformers. At least several of my neons

say luminous sign transformer on them.

As far as I know there is no difference.

TTYL!

Mike Hammer

mhammer@misslink.net

From: tesla-2@emachine.com (Tesla-2)

Subject: SV: Transformers

Date: Thu, 18 Sep 1997 17:22:02 GMT

Organization: The Energy Machine Information System 504-734-0526

From: "Eriksson" <jan-erik.eriksson@mbox2.swipnet.se>

Date: Thu, 18 Sep 1997 19:22:02 +0200

Subject: SV: Transformers

Sorry!

Something happened to the last mail..Guess the first mail was sent again

somehow.

And thanks for the last mail..it really helped me a lot.

I shall write you a mail later on!

Take care!

/Christopher Eriksson

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: My homepage

Date: Thu, 18 Sep 1997 19:31:04 GMT

Organization: The Energy Machine Information System 504-734-0526

From: Richard Smit <esmit@ilink.nis.za>

Date: Thu, 18 Sep 1997 21:31:04 +0200 (SAT)

Subject: My homepage

Hi,

I have changed my Homepage for the last time. So please, go check it out and

give feedback please. It is located at http://www.nis.za/~email/index.htm

It gives a bit of info for beginners on construction, if I have to

change/add something, then email me.

Thanks

------------------------------------------------------------------------

|Richard Smit  | Telephone: +27-525-32634                                |

|P.O.Box 566   | Email:     esmit@ilink.nis.za                           |

|Bultfontein   | Age:       17 years old                                 |

|9670          | Hobbies:   Tesla Coils; FIREWALKING; Computers; UFO's   |

|South Africa  | Birthday:  5 October 1979                               |

------------------------------------------------------------------------

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: My first coil and hello all

Date: Fri, 19 Sep 1997 02:58:59 GMT

Organization: The Energy Machine Information System 504-734-0526

From: "Vince Cothran" <vince@techie.com>

Date: Thu, 18 Sep 1997 22:58:59 -0400

Subject: Re: My first coil and hello all

<snip>

>Here's the specs so far for my coil in progress:

<snip>

>

> >Cap - aluminum flashing and poly plate cap with two sections of .02 mfd

> >each, in one tank of acrylic-lined-plywood with an acrylic separator

> >between the two sections. 35 6" x 8" plates vertically mounted in each

> > section with 6 layers of 6 mil poly between each plate, all four leads

> >brought out through the tank walls.

>

> Vince that sounds like a really great cap. I bet its a dynamite

> performer. 6 layers of 6 mil is 36 mils per cap X 2 caps equals

> 72 mils total with .01 mfd capacitance. That should be a robust

> cap and work well with your 15kv/60 neon.

>

> What kind of oil did you use for this cap and did you do anything

> special about air bubbles?

>

These were 'preliminary specs', Mike :o).  I have cut out the plates and

poly but haven't put them together or or finished building the tank yet.  I

plan on using transformer oil and pumping down the cap with a vacuum pump

before backfilling it with the oil.  Building this cap has been a <lot> of

work.....cutting all those sheets of poly.....eeeek!!  I see now why rolled

caps are so popular.

> I had good results from two secondary coils made with the celluar core

> PVC pipe. Either one would work well. I would think the celluar

> core pipe might even be better. Less lossy PVC.

>

> My latest 4 inch secondary has withstood 48 inch discharges from

> a winding only 21 inches long.

>

Thanks for the tips, Mike.  I believe I will use the cellular core PVC for

my secondary form.  I guess I got a bit overly anxious :o)

Vince

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: My first coil and hello all

Cc:

Bcc:

X-Attachments:

At 02:58 AM 9/19/97 GMT, you wrote:

>From: "Vince Cothran" <vince@techie.com>

>Date: Thu, 18 Sep 1997 22:58:59 -0400

>Subject: Re: My first coil and hello all

>

><snip>

>

>>Here's the specs so far for my coil in progress:

>

><snip>

>

>>

>> >Cap - aluminum flashing and poly plate cap with two sections of .02 mfd

>> >each, in one tank of acrylic-lined-plywood with an acrylic separator

>> >between the two sections. 35 6" x 8" plates vertically mounted in each

>> > section with 6 layers of 6 mil poly between each plate, all four leads

>> >brought out through the tank walls.

>>

>> Vince that sounds like a really great cap. I bet its a dynamite

>> performer. 6 layers of 6 mil is 36 mils per cap X 2 caps equals

>> 72 mils total with .01 mfd capacitance. That should be a robust

>> cap and work well with your 15kv/60 neon.

>>

>> What kind of oil did you use for this cap and did you do anything

>> special about air bubbles?

>>

>

>These were 'preliminary specs', Mike :o).  I have cut out the plates and

>poly but haven't put them together or or finished building the tank yet.  I

>plan on using transformer oil and pumping down the cap with a vacuum pump

>before backfilling it with the oil.  Building this cap has been a <lot> of

>work.....cutting all those sheets of poly.....eeeek!!  I see now why rolled

>caps are so popular.

>

>

Ah. A proposed cap. If you make it vertically oriented and

with transformer oil you dont have to pump it down. The air

will come out on its own if given enough time. Neither of

my plate caps were pumped. But if you have the equipment

it will surely cut down on the break in time required.

To make quick work out of cutting all the poly sheets

cut your self 2 pieces of plywood the exact size of your required

poly sheets. Sandwich up to 20 layers of poly between the sheets

of plywood and clamp with a couple C-clamps. Trim away the

excess poly hanging outside the plywood squares with a razor blade.

Quick easy method to cut em 20 at a time.

TTYL!

Mike Hammer

mhammer@misslink.net

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: My homepage = file not found

Date: Sun, 21 Sep 1997 03:20:36 GMT

Organization: The Energy Machine Information System 504-734-0526

From: Tony Minchew <wamjam@wt.net>

Date: Sat, 20 Sep 1997 22:20:36 -0500

Subject: Re: My homepage = file not found

Organization: Minchew Mfg.

Hi Richard,

I went to your page and found this:

File not found.

Must be a snag somwhere. I hope it gets better.

Tony Minchew

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Finaly, it works

Date: Tue, 23 Sep 1997 07:20:14 GMT

Organization: The Energy Machine Information System 504-733-3462

From: pacster@ibm.net

Date: Tue, 23 Sep 1997 07:20:14 GMT

Subject: Finaly, it works

Hello everyone,

On Sunday I decided to do some work on my 2KW coil and the results were

amazing. About three weeks ago the best I could achieve was only 5" of

spark, don't laugh I thought it wasn't bad and now I'm cracking 40" P to P

from my 18" winding (that's over two times the winding length in spark). I

couldn't believe my eyes, thanks heaps to everyone, especially Mike Hammer

for all your help.

I decided to check the coupling after mike told me that a small difference

in height can make a big difference to coil performance, so I adjusted the

height of the secondary coil by means of a rope tied to a plank in the

ceiling and found that critical coupling occurred at around 2.5" above the

first turn of the primary (so before I wasn't even close to critical

before). Then I put on a 4"x20" toris and sparks came out all over the place

so I went to a 6"x26" and then to a 8"x20 and the sparks came out one at a

time to lengths of around 40" (It was great fun).

Some things I'm not sure about are that the spark was a purplish colour

which I thought was an example of poor efficiency, is this correct? The

first things that I will improve in the system are the primary caps. At the

moment they were just thrown together (I didn't even wipe down the plastic

before assembling, which is probably why I blew up 5 of my six 50nF caps),

then I will probably change the hook up wire (in the primary circuit) from

#26 to just straight 3/8" pipe.

I will ask more detailed questions later this week.

Thanks again for all your help.

Mark B.

Here are my current coil specs in case their needed.

Power supply 230V, 10amp at 50 Hz through a varaic.

15Kv at 120ma into primary circuit from two 15Kv at 60ma neon transformers.

Saftey gap : Three brass door knobs with the centre Knob grounded to earth.

Two chokes : 1" tube with two 3/8" ferrite rods down the centre and wound

with 100 turns of number 18 wire.

Primary spark gap : Richard Hull multi cu. cylinder type gap modified to

provide varable inter gap spacing using 1.5" cu. tube by 3" in length,

currently 21 gaps with a spacing of 0.25mm between gaps.

Primary capactors : polyethelene rolled home made caps. 2 x 0.05uF in series

to yield the necessary 0.025uF for resonant charging from 50Hz @ 2KW.

Primary inductor : Inverted 30 degree cone, made with 3/8" tubing spaced

3/8" apart between turns. Inside diameter of coil is 9" and nine turns were used.

Secondary inductor : 6.5" PVC form closewound with 950 turns of number 24

enamalled wire.

Ground : 1" thick power cable run 30 feet over concrete into the river.

Top load : One Aluminium flexiduct covered with aluminium tape, 8" tube

wraped in a circle of 20" diameter. The spacing between the coil and the

bottom torid is 4".

Approximate resonant frequnecy is 155KHz, The base of secondary is 2.5"

above the first turn of the primary.

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: ADMIN - FAILED MESAGES

Date: Tue, 23 Sep 1997 07:59:24 GMT

Organization: The Energy Machine Information System 504-733-3462

Due to a corrupted configuration file, all list messages posted since

about last Friday have hit the Bit Bucket. :(

If you have not received an acknowledgment from the server about any

new messages you posted, please re send your message to the list.

Sorry for the inconvenience.

Ralph

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: Finaly, it works

Cc:

Bcc:

X-Attachments:

At 07:20 AM 9/23/97 GMT, you wrote:

>From: pacster@ibm.net

>Date: Tue, 23 Sep 1997 07:20:14 GMT

>Subject: Finaly, it works

>

>Hello everyone,

>

>On Sunday I decided to do some work on my 2KW coil and the results were

>amazing. About three weeks ago the best I could achieve was only 5" of

>spark, don't laugh I thought it wasn't bad and now I'm cracking 40" P to P

>from my 18" winding (that's over two times the winding length in spark). I

>couldn't believe my eyes, thanks heaps to everyone, especially Mike Hammer

>for all your help.

Now you are getting somewhere. Most beginners have no trouble

getting sparks equal to their secondary winding length.

Good efficiency brings you up to maybe 1.5 X and 2 X

the secondary length is quite good. 3 X is possible with

good components. Right now my coil is putting out 50 inch streamers

with a 21 inch tall secondary.

>

>I decided to check the coupling after Mike told me that a small difference

>in height can make a big difference to coil performance, so I adjusted the

>height of the secondary coil by means of a rope tied to a plank in the

>ceiling and found that critical coupling occurred at around 2.5" above the

>first turn of the primary (so before I wasn't even close to critical

>before). Then I put on a 4"x20" toris and sparks came out all over the place

>so I went to a 6"x26" and then to a 8"x20 and the sparks came out one at a

>time to lengths of around 40" (It was great fun).

Here is some good info that everyone should take note of.

If your coil is putting out many short streamers instead of

one long one then you need a bigger toroid. Particulary

a toroid of greater cross section. This will allow the voltage

to build up to higher values before breaking down.

>

>Some things I'm not sure about are that the spark was a purplish colour

>which I thought was an example of poor efficiency, is this correct?

It has to do with efficiency and power level. Unconnected

streamers that terminate in the air will be fainter and

more purplish than streamers that connect with ground.

Also your cap value and type play a role. Higher value caps

give hotter and brighter sparks. Caps with low inductance (plate)

give hotter brighter sparks.

Any primary circuit lossy areas can keep your sparks dimmer

than they could be.

As you increased your toroid size you should have noticed

the streamers becoming brighter.

>The first things that I will improve in the system are the primary caps. At the

>moment they were just thrown together (I didn't even wipe down the plastic

>before assembling, which is probably why I blew up 5 of my six 50nF caps),

>then I will probably change the hook up wire (in the primary circuit) from

>#26 to just straight 3/8" pipe.

Here is probably a bottle neck in your system.

If your tank circuit is connected with AWG 26 wire you

have some loss here. Considering the power level you are

running I would go to AWG 8 or better.

3/8 pipe for interconnects is probably overkill but

it sure will work OK.

For the 60hz feed from your neons any wire will do as

the current flow is low. But between the gap, cap and primary

coil the RF currents can be quite large. Heavy conductors

are needed here.

>

>Here are my current coil specs in case their needed.

<snip coil specs>

2 things in your coil specs I wanted to comment on.

>Primary spark gap : Richard Hull multi cu. cylinder type gap modified to

>provide varable inter gap spacing using 1.5" cu. tube by 3" in length,

>currently 21 gaps with a spacing of 0.25mm between gaps.

Your gap is basicly OK but I would try fewer gaps

with a somewhat wider spacing. Your total amount of gap using

all 21 is 5.25 mm which is good for a start but you could use

more gap for 15kv. Since your coil is running well you may want

to up your total gap to 6 or 7 mm. This will increase your primary

energy level but also put your caps and neons under greater strain.

Try a total of 12 gaps (13 electrodes) each spaced 0.6 mm.

This will give you 7.2 mm total gap and probably better efficiency.

Too many gaps in series can limit performance.

>Ground : 1" thick power cable run 30 feet over concrete into the river.

I would work on this too. A 30 foot ground lead is quite long.

If you can drive 3 or 4 ground rods nearer to your coil

it would probably provide a better ground.

I have gotten by with two 4 foot ground rods but my ground lead is

only about 4 feet long.

TTYL!

Mike Hammer

mhammer@misslink.net

From: tesla-2@emachine.com (Tesla-2)

Subject: Coil forms

Date: Tue, 23 Sep 1997 16:06:10 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Eric Davidson <edavidson@icva.gov>

Date: Tue, 23 Sep 1997 11:06:10 -0500

Subject: Coil forms

Hi all!

I'm getting ready to build my first coil and am wondering what to use

for the secondary form.  I know PVC is common, but I was curious to know

if it is worth the extra cost to use a lower loss polystyrene

(plexiglas) form. It is about 10 times the cost per foot of PVC. Also,

anyone know the formula for the capacitance of a toroid? Thank you!

Eric Davidson <edavidson@icva.gov>

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: Coil forms

Cc:

Bcc:

X-Attachments:

At 04:06 PM 9/23/97 GMT, you wrote:

>From: Eric Davidson <edavidson@icva.gov>

>Date: Tue, 23 Sep 1997 11:06:10 -0500

>Subject: Coil forms

>

>Hi all!

>

>I'm getting ready to build my first coil and am wondering what to use

>for the secondary form.  I know PVC is common, but I was curious to know

>if it is worth the extra cost to use a lower loss polystyrene

>(plexiglas) form. It is about 10 times the cost per foot of PVC.

For the most part I would say no to form materials other than PVC.

The difference in performance doesnt justify the cost. Especially

for a first coil.

If you make a coil where its important to eek out every possible

bit of performance then it might be worth it.

Plexiglass is very brittle so should be handled carefully.

It can easily crack on you.

Polyethylene tubing would be one of the best choices.

PVC can be made to perform well despite its lossy nature.

Force drying under heat (130 deg F) for an hour or so and then

sealing with polyurethane varnish (1 coat inside, 2 coats outside)

while still warm will dramatically improve the performance of PVC.

Thin wall PVC will be somewhat better than the standard thick wall

schedule 40 pipe.

I used dried and sealed schedule 40 PVC pipe in all my secondary coils

and had good performance.

>Also, anyone know the formula for the capacitance of a toroid? Thank you!

>

There is no good formula for toroid capacitance. Most formulas

over estimate actual toroid capacitance. This is another case

where Tesla coils defy mathematical models.

One formula I have written by

Bert Poole is:

___________________

/    2

C =(1+ (0.2781 - d2/d1)) x  2.8  x      /  2 pi  (d1-d2)(d2/2)

/   -------------------

\/      4 pi

C = capacitance in picofarads (+- 5% )

d1 = outside diameter of toroid in inches

d2 = diameter of cross section (cord) of toroid in inches

Equation courtesy of Bert Poole

If have not used this formula so cannot vouch for its accuracy.

TTYL!

Mike Hammer

mhammer@misslink.net

From: tesla-2@emachine.com (Tesla-2)

Subject: Transformers & resonance

Date: Fri, 26 Sep 1997 17:46:07 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Dave <djb@erie.net>

Date: Fri, 26 Sep 1997 13:46:07 -0400

Subject: Transformers & resonance

Hi,

This list has been quietly recently so thought I'd ask another question to those

of you who like theory like me. I was thinking about resonance, and that the

voltage output of a coil is based on resonance and not a turns ratio. Does

anyone have a good explanation as to why?

It seems something that is resonant, like a Tesla coil, should keep building

up a voltage to infinity as more energy is supplied to it at the right time,

but we know this doesn't happen because sooner or later the insulation

surrounding it, being the air, will break down and conduct.

But now it seems that something based on a turns ratio would be more

efficient. For example, imagine a transformer with a supply voltage of 15

kv's and a primary with 12 turns, and a secondary of 800 turns. The turns

ratio is 66.667:1

The secondary output voltage would be 1 million volts then.

Now if we had a Tesla coil with a primary of 12 turns, and a secondary with

800 turns and neon rated at 15 kv's, might we expect our output voltage to

be 1 million volts, less, more? It would seem hard to imagine that that

setup would give us anything close to 1 million volts. But according to what

I said before, shouldn't resonance allow us to go to infinity until

something breaks down?

It just seems to me that turns ratio is more efficient than resonance, and

the only reason we use a resonant circuit is to produce the higher

frequencies. So what would the output look like from a transformer at 60hz

with a 1 million volt ouput then? How would it compare to a coil with the

same output and a frequency that is a couple hundred kilohertz?

I guess I shouldn't ask about flyback transformers now, heh

Dave

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: Microwave transformers

Date: Fri, 26 Sep 1997 22:17:38 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Quinney <quinney@inforamp.net>

Date: Fri, 26 Sep 1997 18:17:38 -0400

Subject: Re: Microwave transformers

At 04:34 PM 8/7/97 GMT, Mike Hammer wrote:

>

>The best results will come from wiring two microwave transformers

>in series. Wire the primary windings in parallel and the

>secondary windings in series. Check for proper phasing.

>

>This will give you about 4500 volts with typical microwave

>transformers.

>

>Wiring more than two in series in generally not advisable

>due to the likelihood of insulation breakdown.

>

>Let us know what results you get from these experiments.

>

Hi Mike,

How about 4?  ...wiring the secondaries in series on one (110 V.) side of

the 220, and *another* identical two on the other (110 V.) side of the 220.

Join them all up for 9000 Volts. What would happen, if the phasing and

connections were all correct, and the correct secondary centre point is

grounded to the primary neutral ground side of the line(s)? Would the

insulation breakdown odds increase ---> above using only 2 secondaries in

series?

My first reaction is that this is too dangerous, and that the breakdown

odds *would* increase, but I am curious about the line *phasing* between

one side of the 220 and the other. Are they the same or are they different?

And if they *are* different, do you think that would make a difference?

Thanks,

Colin Quinney.

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: Transformers & resonance

Date: Sat, 27 Sep 1997 03:40:40 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Bert Hickman <bert.hickman@aquila.com>

Date: Fri, 26 Sep 1997 20:40:40 -0700

Subject: Re: Transformers & resonance

Organization: Stoneridge Engineering

Tesla-2 wrote:

>

> From: Dave <djb@erie.net>

> Date: Fri, 26 Sep 1997 13:46:07 -0400

> Subject: Transformers & resonance

>

> Hi,

>

> This list has been quietly recently so thought I'd ask another question to those

> of you who like theory like me. I was thinking about resonance, and that the

> voltage output of a coil is based on resonance and not a turns ratio. Does

> anyone have a good explanation as to why?

>

In a transformer, the turns ratio will apply only when you have close

coupling between the primary and secondary, as in 60 Hz power

transformers. For typical tesla coils only 20% or so of the primary flux

lines are coupling to the secondary (and vice versa). If you make both

the primary and secondary resonant circuits by putting capacitance

across each coil, AND if you set CpLp = CsLs, then you have the coupled

tuned circuits which form a 2-coil Tesla Coil. If you start out with a

certain amount of energy stored in the primary tank capacitor, and then

discharge the cap through the primary, the primary circuit oscillates,

transferring energy to the secondary. After a period of time (governed

by the degree of coupling), all the energy that was in the primary gets

transferred to the secondary, less losses due to the gap, resistance,

radiation, etc. Since you can't get more energy in the secondary than

what you had in the primary in the first place, this will limit the

amount of voltage you'll get in the secondary. Neglecting losses, you

can show by the law of Conservation of Energy that:

Vs = Vp*SQRT(Cp/Cs)

OR

Vs = Vp*(SQRT(Ls/Lp

where Vs = maximum secondary voltage

Vp = primary capacitor voltage when the gap fires

SQRT = Square Root function

Cp = Primary Tank Capacitance

Cs = secondary + Toroid capacitance

Lp = Primary Inductance

Ls = Secondary Inductance

> It seems something that is resonant, like a Tesla coil, should keep building

> up a voltage to infinity as more energy is supplied to it at the right time,

> but we know this doesn't happen because sooner or later the insulation

> surrounding it, being the air, will break down and conduct.

If you did this using a vacuum tube amplifier, eventually you'd reach a

balance between energy being put in versus losses due to skin-effect

resistance, leakage currents, radiation, dielectric losses, etc...

>

> But now it seems that something based on a turns ratio would be more

> efficient. For example, imagine a transformer with a supply voltage of 15

> kv's and a primary with 12 turns, and a secondary of 800 turns. The turns

> ratio is 66.667:1

> The secondary output voltage would be 1 million volts then.

>

> Now if we had a Tesla coil with a primary of 12 turns, and a secondary with

> 800 turns and neon rated at 15 kv's, might we expect our output voltage to

> be 1 million volts, less, more? It would seem hard to imagine that that

> setup would give us anything close to 1 million volts. But according to what

> I said before, shouldn't resonance allow us to go to infinity until

> something breaks down?

>

If you could continually add more energy, and if the losses were

negligable, yes. However, this is never true in the real world - an

energy equillibrium is reached. Eventually, the losses from the

breakdown of the air itself predominate, and you once again balance

output to energy input.

> It just seems to me that turns ratio is more efficient than resonance, and

> the only reason we use a resonant circuit is to produce the higher

> frequencies. So what would the output look like from a transformer at 60hz

> with a 1 million volt ouput then? How would it compare to a coil with the

> same output and a frequency that is a couple hundred kilohertz?

>

> I guess I shouldn't ask about flyback transformers now, heh

>

> Dave

>

The dual resonant circuit helps us efficiently transfer the majority of

the initial energy from the primary capacitor to the the secondary. A 60

Hz transformer's output would look more like the arc from a neon sign

transformer, only longer and possibly a brighter color of blue-white if

we increased the current as well.

Hope this helped a bit...

-- Bert --

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: Microwave transformers

Cc:

Bcc:

X-Attachments:

At 10:17 PM 9/26/97 GMT, you wrote:

>From: Quinney <quinney@inforamp.net>

>Date: Fri, 26 Sep 1997 18:17:38 -0400

>Subject: Re: Microwave transformers

>

>At 04:34 PM 8/7/97 GMT, Mike Hammer wrote:

>>

>>The best results will come from wiring two microwave transformers

>>in series. Wire the primary windings in parallel and the

>>secondary windings in series. Check for proper phasing.

>>

>>This will give you about 4500 volts with typical microwave

>>transformers.

>>

>>Wiring more than two in series in generally not advisable

>>due to the likelihood of insulation breakdown.

>>

>>Let us know what results you get from these experiments.

>>

>

>Hi Mike,

>

>How about 4?  ...wiring the secondaries in series on one (110 V.) side of

>the 220, and *another* identical two on the other (110 V.) side of the 220.

>Join them all up for 9000 Volts. What would happen, if the phasing and

>connections were all correct, and the correct secondary centre point is

>grounded to the primary neutral ground side of the line(s)? Would the

>insulation breakdown odds increase ---> above using only 2 secondaries in

>series?

>

>My first reaction is that this is too dangerous, and that the breakdown

>odds *would* increase, but I am curious about the line *phasing* between

>one side of the 220 and the other. Are they the same or are they different?

> And if they *are* different, do you think that would make a difference?

>

>Thanks,

>Colin Quinney.

>

>

Colin,

How you wire up the primaries of these transformers doesnt affect

the breakdown rating of the secondary windings. Thats determined

by the physical construction of the transformer.

What happens when you series more than two is the voltage on

the outside units goes higher than what the transformer

insulation is rated for. If the transformer is built and

insulated well it might handle it.

Another difficulty lies in the fact that many oven transformers

have one of the secondary leads tied to the core. This will have

to be removed in order to use more than two in series. This may

not be possible in some units.

A couple of people have had good results doing this. List member

Jim Buck has a series string of 4 microwave transformers

and gets quite good results. I believe he has his transformers

in oil to suppress corona and improve the breakdown rating.

I would think oil is almost going to be mandatory.

Perhaps Jim will add his experience with oven transformers

to this thread.

Microwave transformers are going to need special attention

to work effectively on a coil. You will need a large cap

to soak up that high level of current and prevent flaming

at the gap. You will also need a very heavy duty gap to handle

the high current and resulting heat.

So its possible to use microwave oven transformers but they

are not easy.

TTYL!

Mike Hammer

mhammer@misslink.net

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: Microwave transformers

Date: Sat, 27 Sep 1997 07:34:26 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Jimbuck1@aol.com

Date: Sat, 27 Sep 1997 03:34:26 -0400 (EDT)

Subject: Re: Microwave transformers

-snip-

>Another difficulty lies in the fact that many oven transformers

>have one of the secondary leads tied to the core. This will have

>to be removed in order to use more than two in series. This may

>not be possible in some units.

I have found that if you don't remove the secondary lead tied to the core the high

voltage produced when tying 3 or 4 in series will arc from the core back to the

primary windings.

>A couple of people have had good results doing this. List member

>Jim Buck has a series string of 4 microwave transformers

>and gets quite good results. I believe he has his transformers

>in oil to suppress corona and improve the breakdown rating.

>I would think oil is almost going to be mandatory.

>Perhaps Jim will add his experience with oven transformers

>to this thread.

When I first started using microwave transformers I ran them dry. I didn't have any problems running 3 in series.  However when I tried connecting 4 I started

seeing arcing in the secondaries.  I then put 4 transformers in mineral oil and theyhave been working fine since.   I was expecting to blow a few transformers as I have abused them by running to much gap, to high of a break rate etc etc..

I really am pleasantly suprised that I haven't had any problems with the transformers.

>Microwave transformers are going to need special attention

>to work effectively on a coil. You will need a large cap

>to soak up that high level of current and prevent flaming

>at the gap. You will also need a very heavy duty gap to handle

>the high current and resulting heat.

Getting a gap to work with the microwave transformers wasn't easy.  I built

a R.Q. static gap with 5 electrodes of 1 1/4" copper pipe.  It worked with a

4hp shop vac blowing through it but if I run it for more than minute the

electrodes get hot enough to start melting the pvc pipe.  I also made a static

gap by building a flat grate with 5 more pieces of copper pipe.  I placed the grate across the output of a squirrel cage fan and sealed everything except the electrodes so that the full output of the fan blows between the gaps.

The only time I had problems with this gap was when I converted my coil over to

a magnifier.  Even then as soon as I had the coil in rough tune the gaps started working fine again.  You may also want to consider a rotary gap.  I just built

one a few days ago and run it in series with the squirrel cage gap.  It works

much better but I still have some work to do on the rotary.

>So its possible to use microwave oven transformers but they

>are not easy.

Its really nice to have all the extra power of the microwave transformers

however everything is quite a bit harder to get working. Like Mike said you

need a lot of caps.  I usually run around .1 uf; I believe the transformers

could use more but around .1uf the primary only taps at 4 turns.  Also your primary wiring and ground have to be super heavy duty compared to a neon driven coil.  I also had to play around with the coupling between primary and secondary to start getting decent results.

Jim Buck

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: howto of bottle caps

Date: Sat, 27 Sep 1997 16:37:50 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Harald Kreuzer <hkreuzer@mmm.at>

Date: Sat, 27 Sep 1997 18:37:50 +0200

Subject: howto of bottle caps

hello everyody,

i just started building my first TC and the only thing i have yet is the

transformer (12 kV, 150mA). next thing i want to do is build the caps.

as i am a beginner i think it's best to build these cheap ones with

bottles. can anybody give me some advice how to do or is there a web page

with instructions?

thanks

harald

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Primary vs. Performance

Date: Sat, 27 Sep 1997 16:50:19 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Richard Smit <esmit@ilink.nis.za>

Date: Sat, 27 Sep 1997 18:50:19 +0200 (SAT)

Subject: Primary vs. Performance

Hi,

I have seen a lot of guys Tesla coils with a 30 to 15 degree primary. All of

their coils seems to perform great.

IS THERE ANY EFFECT ON WHAT SORT OF PRIMARY IS BEST WITH SOME TRANSFORMERS?

Thanks

------------------------------------------------------------------------

|Richard Smit  | Telephone: +27-525-32634                                |

|P.O.Box 566   | Email:     esmit@ilink.nis.za                           |

|Bultfontein   | Age:       17 years old                                 |

|9670          | Hobbies:   Tesla Coils; FIREWALKING; Computers; UFO's   |

|South Africa  | Birthday:  5 October 1979                               |

------------------------------------------------------------------------

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Jacob's Ladders

Date: Sat, 27 Sep 1997 17:27:39 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Shane Randle <randlec@scnc.addison.k12.mi.us>

Date: Sat, 27 Sep 1997 13:27:39 -0400

Subject: Jacob's Ladders

I know this is a little off-subject, but can anyone tell me where to get

information about building a Jacob's Ladder?

-Shane Randle

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: howto of bottle caps

Cc:

Bcc:

X-Attachments:

At 04:37 PM 9/27/97 GMT, you wrote:

>From: Harald Kreuzer <hkreuzer@mmm.at>

>Date: Sat, 27 Sep 1997 18:37:50 +0200

>Subject: howto of bottle caps

>

>hello everyody,

>

>i just started building my first TC and the only thing i have yet is the

>transformer (12 kV, 150mA). next thing i want to do is building the caps.

>as i am a beginner i think it's best to build these cheap ones with

>bottles. can anybody give me some advice how to do or is there a web page

>with instructions?

>

>thanks

>

>harald

>

Harald,

A 12kv/150ma neon? Geez. Wish I had that. That is a real powerhouse.

But it is way too much transformer to start with.

Get yourself a 12kv/30 or even a 9kv/30 to start with.

It will be easier to get your coil working and tuned properly

with the lower power level.

And you dont need that much power to create long sparks.

My own 4 inch diameter coil gives 50 inch sparks with a single

12kv/90ma neon.

Salt water bottle caps are a good way to start. They are cheap

and easy to build and get you up and running quickly.

They are lossy and inefficient and give thin and weak

sparks but its a good start.

Let me paste in a file here on a good design for a bottle cap.

<begin include>

PLANS FOR BOTTLE CAPACITOR

Go drink a sixpack of wine coolers. When your buzz wears off wash and

clean the bottles out. Scrub off the labels.

Discard the pop-tops and get six screw on white plastic caps from

2-litre Schweppes bottles.

Create a supersaturated solution of Morton salt and water. My solution

was 8 tablespoons salt to 6 cups of water. Pour the solution into the

bottles filling up to the level where the bottle is no longer

cylindrical.

For the remaining airspace, top off the bottles with 10W30 motor oil

which will float above the salt solution.

Drill a 1/4 inch hole in the center of each plastic screw cap. Take six

1/4" by 8" long zinc coated carrage bolts, the kind with threads running

only about an inch at the end (these are common). Screw  a hex nut onto

each bolt as far as it will go.  Insert each bolt thread first through

the bottom of each plastic cap through the hole you made  so that the cap

rests on the nut. Screw another nut onto each bolt down over each plastic

cap. There should be about 1/2" of thread left free.

Take these six assemblies and screw one onto each bottle so the head of

the bolt sticks down through the oil into the brine. Fit should be tight

but seal the cap with RTV anyway.

Now wrap the the cylindrical portion of the outside of each bottle with

Aluminum foil allowing the foil the cover the bottom of the bottle. Wrap

over this tightly with electrical tape (at least two layers). Dont wrap

over the neck of the bottle or the foil on the underside.

Construct a box of 3/4" pine or plywood just big enough to house the six

pack and about 6" deep. Paint it flat black and varnish it or use some

other insulating coating. Cover the bottom with a sheet of plate glass

and place a sheet of Aluminum,copper,or foil on the glass.

Place the bottles in the box. The metal/foil on the glass on the box

bottom will contact the foil  on the bottle bottoms forming one terminal

of the capactitor.

A hole should be drilled through the box near the bottom to bring out the

lead away from the top end. Electrically tie the bolt ends (top) of each

bottle together with small pieces of heavy gage wire and ring lugs. That

is the other terminal.

The beauty of this is the convenience of bottle capacitors yet everything

is dry outside. Tuning is as easy as pulling a bottle out or putting

another in.

>From my notebook (1990):

The 'six pack' capacitor is finished and is very satisfactory. I took

some readings of my various capacitors and got unexpected results. My oil

capacitor is 4.8 nF according to the meter. The 'sixpack' is 4.3 nF. The

closeness is amazing considering the advantages of the sixpack over the

bulky oily mess with 9 1/4" plates of glass and 4 qts. of motor oil....

each individual bottle in the sixpack read very close to .74 nF which

suprised me....I wasn't expecting anything so uniform.

Parts List:      (for 1 'sixpack')

6 beer or wine cooler bottles (must have some cylindrical surface area)

6 plastic caps from 2 litre Schweppes bottles

1 quart 10W-30 motor oil

1 box Morton salt

3/4"x6" pine wood

1/4" window glass

1 handfull of ring lugs (for 18AWG or bigger)

Some 18AWG wire or bigger

1 box aluminum foil

6 1/4"x 8" zinc coated carrage  bolts (with nuts and stars)

2 rolls electrical tape

Notes:

Several boxes may be needed wired in parallel.

Each box (in my case) was about 4.3nF.

You can experiment with other solutions instead of salt water.

You can try bigger bottles.

I used un-iodized salt. Iodized salt contains potassium iodide and

dextrose which may or may not give the caps more capacitance

Tesla used a similar caps in Colorado Springs.

I made a special box for tuning with the capacitance value of each

successive bottle halved (by removing foil/tape with an Xacto knife)

I've never had a bottle explode (the deep box will shield you)

<end include>

Some further notes.

When I made my bottle caps I used 12oz beer bottles

and attached a washer containing the bolt to the bottle

top with RTV silicone.

Instead of wrapping foil on the outside of the bottle

I placed the bottles in a tub of saltwater. The saltwater

taking the place of the foil. This resulted in more capacitance

and less corona since the liquid has no sharp edges to promote

corona. Of course it makes the cap heavier and messier.

Use long neck beer bottles to provide enough clearance

between the cap plates to prevent arc overs.

I got a little of 700 pf per 12 oz beer bottle.

If you have more questions feel free to ask.

TTYL!

Mike Hammer

mhammer@misslink.net

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: Jacob's Ladders

Cc:

Bcc:

X-Attachments:

At 05:27 PM 9/27/97 GMT, you wrote:

>From: Shane Randle <randlec@scnc.addison.k12.mi.us>

>Date: Sat, 27 Sep 1997 13:27:39 -0400

>Subject: Jacob's Ladders

>

>I know this is a little off-subject, but can anyone tell me where to get

>information about building a Jacob's Ladder?

>-Shane Randle

>

>

Its not really that much off topic as most of

us have built and used Jacobs ladders at one time or another.

After I unpot and rebuild a neon transformer I test it on a Jacobs

ladder for an extended run.

To build a ladder doesnt require anything to eleborate.

A neon transformer works well. The higher current 60 ma

neons make better ladders.

Simply arrange a pair of stiff wires (coat hanger works well)

in a "V" shape with the closest point at the bottom. The gap

distance on the bottom depends on the voltage. A little wider

gap being needed for higher voltage.

Apply the high voltage from the neon to the two wires

and turn it on. A spark will jump across the narrow gap

at the bottom and be carried upwards by the rising heat.

It will continue to climb until the spacing between the diverging

wire becomes to great to sustain an arc.

Be careful. The arc can catch things on fire easily.

Some things to do include smearing a little table salt

on the wires to give a brilliant yellow arc.

If you can get some strontium or lithium compounds

they will give interesting colors.

One URL I found that describes a Jacobs Ladder is:

http://www.asterisk.no/~oglend/elnikk/jacobs.htm

TTYL!

Mike Hammer

mhammer@misslink.net

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: Jacob's Ladders

Date: Sat, 27 Sep 1997 18:52:57 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Shane Randle <randlec@scnc.addison.k12.mi.us>

Date: Sat, 27 Sep 1997 14:52:57 -0400

Subject: Re: Jacob's Ladders

Tesla-2 wrote:

>

> From: corr <corr@fullnet.net>

> Date: Sat, 27 Sep 1997 13:22:26 -0600

> Subject: Re: Jacob's Ladders

>

> Well, take two aluminum hangers, bend them as straight as you can get them.

> Attach them to a piece of wood or something. Make em in a "V" shape.

> The bottom should be 1/4th-1/8th of an inch wide.  Top about 3-5" wide.

> Hook the hv terminals from your transformer (at least a 9kv, but 12 or 15

> really works great) up to the aluminum rods.  That easy.

>

I'm planning on ordering a 12kV, 60mA neon in the next two weeks or so,

and that would serve as my power source. My main question is this; are

there any capacitors involved in the circuit, or do the rods connect

directly to the hv terminals? Thank you for your information and

assistence.

-Shane

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: Jacob's Ladders

Date: Sat, 27 Sep 1997 19:22:26 GMT

Organization: The Energy Machine Information System 504-733-3462

From: corr <corr@fullnet.net>

Date: Sat, 27 Sep 1997 13:22:26 -0600

Subject: Re: Jacob's Ladders

Well, take two aluminum hangers, bend them as straight as you can get them.

Attach them to a piece of wood or something. Make em in a "V" shape.

The bottom should be 1/4th-1/8th of an inch wide.  Top about 3-5" wide.

Hook the hv terminals from your transformer (at least a 9kv, but 12 or 15

really works great) up to the aluminum rods.  That easy.

>I know this is a little off-subject, but can anyone tell me where to get

>information about building a Jacob's Ladder?

>-Shane Randle

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: howto of bottle caps

Date: Sat, 27 Sep 1997 19:22:28 GMT

Organization: The Energy Machine Information System 504-733-3462

From: corr <corr@fullnet.net>

Date: Sat, 27 Sep 1997 13:22:28 -0600

Subject: Re: howto of bottle caps

>i just started building my first TC and the only thing i have yet is the

>transformer (12 kV, 150mA). next thing i want to do is building the caps.

>as i am a beginner i think it's best to build these cheap ones with

>bottles. can anybody give me some advice how to do or is there a web page

>with instructions?

12kv, 150ma?  With 150ma you may have trouble with the cheap caps.

I run 15kv at 60ma and use the bottle caps, and they work fine for me, but

that might just be too much current.

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: Jacob's Ladders

Date: Sat, 27 Sep 1997 21:05:53 GMT

Organization: The Energy Machine Information System 504-733-3462

From: corr <corr@fullnet.net>

Date: Sat, 27 Sep 1997 15:05:53 -0600

Subject: Re: Jacob's Ladders

>I'm planning on ordering a 12kV, 60mA neon in the next two weeks or so,

>and that would serve as my power source. My main question is this; are

>there any capacitors involved in the circuit, or do the rods connect

>directly to the hv terminals? Thank you for your information and

>assistence.

>-Shane

No caps, nothing.  The only thing needed is a transformer.

Thats why you can build them in 5 minutes.

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: Primary vs. Performance

Cc:

Bcc:

X-Attachments:

At 04:50 PM 9/27/97 GMT, you wrote:

>From: Richard Smit <esmit@ilink.nis.za>

>Date: Sat, 27 Sep 1997 18:50:19 +0200 (SAT)

>Subject: Primary vs. Performance

>

>Hi,

>

>I have seen a lot of guys Tesla coils with a 30 to 15 degree primary. All of

>their coils seems to perform great.

>

>IS THERE ANY EFFECT ON WHAT SORT OF PRIMARY IS BEST WITH SOME TRANSFORMERS?

>

No Richard there really isnt. In my opinion no one primary shape

is really any better than any other. Inductance is inductance.

Some primary shapes may have desirable characteristics for certain

coils but all of them will probably perform similar once you

make allowance for the difference in coupling.

And that is the basic difference between the various primary

coil shapes - coupling.

The solenoid shaped primary has the highest degree of coupling while

a flat spiral has the lowest. The 15-30 degree saucer shaped primary

is somewhere in between.

The main reason for using the saucer shaped primary is to prevent

problems with over coupling. On high power coils where the saucer

primary still over couples then the flat spiral becomes attractive.

Always try to keep as much primary inductance tapped in as possible.

Higher primary inductance helps improve gap operation. This is especially

important if your gap isnt so great.

The great performing coils you spoke of dont get their great performance

solely from a particular primary shape. Indeed this kind of performance

isnt due to any single component. Its due to the coil system as

a whole being well balanced and having all the individual components

well matched for the task at hand.

Beginners are often looking for that magic bullet. Just that

one secret or special design of some component that will give

them great performance.

It takes improving each and every component in your coil trying

to remove the bottlenecks. When one bottleneck is removed another

takes its place.

TTYL!

Mike Hammer

mhammer@misslink.net

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: Transformers & resonance

Date: Mon, 29 Sep 1997 14:26:09 GMT

Organization: The Energy Machine Information System 504-733-3462

From: "Paul Marshall" <PMARSHAL@ETHCINC1.SSW.JNJ.COM>

Date: 29 Sep 1997 09:26:09 EST

Subject: Re: Transformers & resonance

The electromagnetic coupling between the primary and the secondary

should be extremely efficient when resonate. Each time we experience

a cycle at resonate frequency see an increase in voltage. Its almost

like a surfer riding a wave. Our surfer is different however, because he doesn't

ride just one wave. The first wave imparts energy to the surfer by pushing

against him. The surfer begins to move, the residual energy is then boosted by

the surge of the of the next wave and so on. More or less energy will be

imparted to your secondary with each cycle. Therefore the rise in voltage within

the secondary will be a composite, consisting of frequency, resonance and

available power. These cyclic rises in voltage are probably not linear. On the

negative side, losses will occur due to heat and RF loss.

______________________________ Reply Separator _________________________________

Subject: Transformers & resonance

Author:  TESLA2 (INTERNET.TESLA2) at SWIFTMAIL

Date:    9/26/97 3:19 PM

From: Dave <djb@erie.net>

Date: Fri, 26 Sep 1997 13:46:07 -0400

Subject: Transformers & resonance

Hi,

This list has been quietly recently so thought I'd ask another question to

those of you who like theory like me. I was thinking about resonance, and

that the voltage output of a coil is based on resonance and not a turns ratio.

Does anyone have a good explanation as to why?

It seems something that is resonant, like a Tesla coil, should keep building

up a voltage to infinity as more energy is supplied to it at the right time,

but we know this doesn't happen because sooner or later the insulation

surrounding it, being the air, will break down and conduct.

But now it seems that something based on a turns ratio would be more

efficient. For example, imagine a transformer with a supply voltage of 15

kv's and a primary with 12 turns, and a secondary of 800 turns. The turns

ratio is 66.667:1

The secondary output voltage would be 1 million volts then.

Now if we had a Tesla coil with a primary of 12 turns, and a secondary with

800 turns and neon rated at 15 kv's, might we expect our output voltage to

be 1 million volts, less, more? It would seem hard to imagine that that

setup would give us anything close to 1 million volts. But according to what

I said before, shouldn't resonance allow us to go to infinity until

something breaks down?

It just seems to me that turns ratio is more efficient than resonance, and

the only reason we use a resonant circuit is to produce the higher

frequencies. So what would the output look like from a transformer at 60hz

with a 1 million volt ouput then? How would it compare to a coil with the

same output and a frequency that is a couple hundred kilohertz?

I guess I shouldn't ask about flyback transformers now, heh

Dave

-----

Processed with Listserv v2.93 for Wildcat v4

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Tesla-2 list archives

Cc:

Bcc:

X-Attachments:

Notice to all.

I have placed the lastest archives of the

Tesla-2 list on funet.fi.

The files are tes2arc6.zip which contains posts for July

and tes2arc7.zip which is the posts for August.

The URL for the archive site is:

ftp://nic.funet.fi/pub/sci/electrical/tesla/maillist/tesla2/

or

ftp.funet.fi/pub/sci/electrical/tesla/maillist/tesla2/

Many thanks to Kristian Ukkonen for making this space available.

TTYL!

Mike Hammer

mhammer@misslink.net

From: tesla-2@emachine.com (Tesla-2)

Subject: Cap charging

Date: Tue, 30 Sep 1997 06:55:26 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Dave <djb@erie.net>

Date: Tue, 30 Sep 1997 02:55:26 -0400

Subject: Cap charging

Hi again,

Thanks to all who answered my last questions on transformers and resonance.

I have yet another question on resonance now. My question is, when u match

the output impedance of your neon transformer to that of your capacitor at

60 hz, is that the fastest way to charge your cap?

The reason I am asking this is because I am wondering how you determine the

time it takes to charge up a cap. I guess you can measure the resistance of

the circuit and figure out a RC time constant and solve it that way, but

when I think that the circuit is resonant, I am not so sure anymore. Also,

say you use a different value for a capacitor so it is no longer resonant at

60 hz, how do you determine the time? Do you use the same method for

resonance than for not resonance?

I'll show you an example of why I came up with this question. I was trying

to determine the optimum value for a capacitor given a neon rated at 12 kv's

and 60 ma's. The output impedance of that would be 200 K ohms. If I matched

the impedance of a capacitor at 60 hz to that value, I get 13.263 nF.

Now I tried it another way, and came up with another answer. I was thinking

that you would want to have a capacitor that would have as much energy as it

could take and have a potential across it of 12 kv's at the first quarter

cycle of your 60 hz sine wave, and then discharge on the 2nd quarter cycle,

and recharge once more. Here's the values I get for my other capacitor.

The time for one quarter cycle is: 1/60/4 = .004167 seconds

The RMS value for current is 60ma, so that implies that the transformer is

supplying .06 coulombs per second.

So .06 coulombs/sec * .004147 secs = .00025 coulombs

Now Q=C*V so C=Q/V --> C = .00025 coulombs / 12000 volts

C = 20.833 nF

Clearly there is a difference between 13.263 nF and 20.833 nF. Which one is

better? Seems like I would want the larger value as it would be able to

store more energy so when the gap fires, it will deliver more energy into

the primary coil. My whole intention here is to maximize the energy

delivered into the primary when the gap fires without exceeding the voltage

rating of the neon and destroying it. Not counting higher gap losses due to

the increased amount of current in the gap, would this be the right aproach

in determining the right cap size? It seems as if it would store more energy

in the given amount of time, and have the correct voltage to fire the gap,

but I am not sure what happens because I don't understand the difference

between resonant and non-resonant charging.

Thanks for you time. Sorry about the length of this.

Dave

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: Cap charging

Date: Tue, 30 Sep 1997 13:54:30 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Bert Hickman <bert.hickman@aquila.com>

Date: Tue, 30 Sep 1997 06:54:30 -0700

Subject: Re: Cap charging

Organization: Stoneridge Engineering

Dave,

Good questions and analyses! My comments are interspersed below.

Tesla-2 wrote:

>

> From: Dave <djb@erie.net>

> Date: Tue, 30 Sep 1997 02:55:26 -0400

> Subject: Cap charging

>

> Hi again,

>

> Thanks to all who answered my last questions on transformers and resonance.

> I have yet another question on resonance now. My question is, when u match

> the output impedance of your neon transformer to that of your capacitor at

> 60 hz, is that the fastest way to charge your cap?

>

<SNIP>

YES. The reason is that you end up with a series RLC circuit, where L is

the leakage inductance, C the tank capacitance, and R the transformer's

winding resistance. By setting this to resonate around 60 Hz, the

inductive and capacitive reactances partially cancel. In effect, you are

partially "neutralizing" the built-in current limiting inductance of the

transformer when it's charging/discharging the tank cap. There's no more

effective way to quickly re-charge your tank cap from a neon source.

> I was trying

> to determine the optimum value for a capacitor given a neon rated at 12 kv's

> and 60 ma's. The output impedance of that would be 200 K ohms. If I matched

> the impedance of a capacitor at 60 hz to that value, I get 13.263 nF.

Actually, a tank cap in the range of 12-14 nF will work about as well,

since the resonance curve is relatively broad (the Q is usually in the

range of about 5 - 10).

>

> Now I tried it another way, and came up with another answer. I was thinking

> that you would want to have a capacitor that would have as much energy as it

> could take and have a potential across it of 12 kv's at the first quarter

> cycle of your 60 hz sine wave, and then discharge on the 2nd quarter cycle,

> and recharge once more. Here's the values I get for my other capacitor.

>

> The time for one quarter cycle is: 1/60/4 = .004167 seconds

>

> The RMS value for current is 60ma, so that implies that the transformer is

> supplying .06 coulombs per second.

>

> So .06 coulombs/sec * .004147 secs = .00025 coulombs

>

> Now Q=C*V so C=Q/V --> C = .00025 coulombs / 12000 volts

>

> C = 20.833 nF

>

Interesting analysis, but not quite right. Remember, at one quarter

cycle, the capacitor would ideally be charged to the peak voltage of the

trannie, or 12000 x 1.41 or about 16.97 KV. So C = 0.00025/16970 = 14.7

nF which is significantly closer to the value predicted from matching

impedances at 60 Hz.

> Clearly there is a difference between 13.263 nF and 20.833 nF. Which one is

> better? Seems like I would want the larger value as it would be able to

> store more energy so when the gap fires, it will deliver more energy into

> the primary coil. My whole intention here is to maximize the energy

> delivered into the primary when the gap fires without exceeding the voltage

> rating of the neon and destroying it.

The easiest way to prevent this is not to open the gaps too widely!

> Not counting higher gap losses due to

> the increased amount of current in the gap, would this be the right aproach

> in determining the right cap size? It seems as if it would store more energy

> in the given amount of time, and have the correct voltage to fire the gap,

> but I am not sure what happens because I don't understand the difference

> between resonant and non-resonant charging.

>

> Thanks for you time. Sorry about the length of this.

>

> Dave

Whether you use 13.2 or 14.7 nF really doesn't matter very much - you'll

still encounter 60 Hz resonant rise with either. The good news is that

the capacitor recharge current climbs significantly under these

conditions, allowing you to suck more power out of your trannies than

they would normally deliver into a short-circuit. The downside is that

you'll need to carefully need to limit your gap spacing to prevent your

neon from being overvolted. For example, if you removed the gaps, the

transformer voltage will try to rise to Q times the normal voltage.

Assuming Q=10, your 12 KV trannie would try to hit 167 KV.... until it

flashed over or took out your tank cap.

Safe coilin' to you, Dave!

-- Bert --

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Transformer connections

Date: Tue, 30 Sep 1997 15:32:10 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Eric Davidson <edavidson@icva.gov>

Date: Tue, 30 Sep 1997 10:32:10 -0500

Subject: Transformer connections

Hello everyone:

I noticed several postings about using microwave oven transformers in

series for Tesla coils and I thought I would add my 2 cents worth on

connecting transformers in series.  Connecting high voltage transformers

in series is risky for several reasons.  The main problem is the

strength of the secondary insulation.  Most HV transformers (of the neon

sign or cold cathode type) have the midpoint of the secondary attached

to the case/core, or as is the case in oil burner transformers, one end

of the secondary is connected to the case.  This lowers the potential

between the HV bushings and the case, lessening the the size requirement

for the bushings (and in oil burner transformers, eliminating one

bushing) and lessening electrical stresses within the case (between

windings, windings and core, windings and case etc.). In most

transformers the primary winding is isolated from the case and a

separate lug is provided for grounding.  This is always the case for

transformers for 240 or 480 volt operation (for obvious reasons). When

an attempt is made to place transformers in series several problems

begin to arise. How do we deal with that pesky secondary midpoint? If we

unpot and physically remove the connection to the core/case we double

the potential between the case and the HV lugs (and hope that the

designers accounted for this eventuality in their design) and we haven't

even put two in series yet! So, if we put two transformers in series

with their cases grounded, we have 4 times the rated voltage between the

case and the HV lugs! Its doubtful there is this much margin in the

design. We could leave the secondary midpoint grounded and let the cases

float (checking for primary isolation first). By leaving the midpoint

connection we assure that each transformer has its rated voltage from

the HV lugs to the case, but of course now the cases are floating and

very unsafe. They must be insulated from ground (and people). This

brings up the problem of primary isolation. This is a big one, and

basically there is no way around it. The closest you can come is to

power the transformers from a line isolation transformer. Most isolation

transformers have good hi pot numbers (though not up to tens of kV).

This arrangement allows the entire primary to float, and gets around the

fact that the midpoint of the utility company's secondary midpoint is

grounded (actually called neutral, it does carry the unbalanced load).

In my humble opinion, if you are considering a series connection, use an

ODD number of transformers. Keep the secondary midpoints connected to

the core/case. Let the cases of the outer two (if you use 3)

transformers float, ground the case of the center one. You MIGHT be able

to get by if the secondary voltage of each is around 5 kV (I believe

microwave oven transformers are in this range, maybe a little lower).

Remember, the transformers must have equal impedances (output CURRENT

ratings) if the voltage is to divide correctly. If the VOLTAGE ratings

are different, ground the one with the LOWEST rating. Pay attention to

phase! I noticed that some folks immerse the transformers in oil.  While

this is a good idea to prevent flash over on the outside of the case it

increases the stress inside the case.  If a transformer breaks down

inside it is usually ruined because the potting material becomes quite

conductive after burning with an arc.  IMHO, use no oil, let it flash on

the outside if it is going to, consider it a safety gap. Old neon

transformers are notorious for an inside breakdown, especially if they

have been to small for the length tubing they powered. Also, if you have

the choice, it is better to place transformers of a higher voltage and

lower current in PARALLEL than vice versa in SERIES. I do realize you

have to play the hand you are dealt when finding parts!

Eric

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: cap charging

Cc:

Bcc:

X-Attachments:

Forwarded to Teslsa-2 for Bill Pollack.

<begin forwarded message>

From: "Bill Pollack" <arcstarter@hotmail.com>

To: djb@erie.net, mhammer@misslink.net

Subject: Re: Cap charging

Date: Tue, 30 Sep 1997 06:53:24 PDT

----------------------------------------------------------------

Dave,

Due to technical reasons I cannot post to tesla-2 from my account.  But

here are my thoughts on your cap charging questions:

BTW - I am not a great tesla coil expert so Mr. Hammer's advice would

supercede my own.

You wrote:

>I have yet another question on resonance now. My question is, when u match

>the output impedance of your neon transformer to that of your capacitor at

>60 hz, is that the fastest way to charge your cap?

Well - if you are asking how to charge a cap up to the max transformer

voltage, then the *fastest* way is to have an extremely small

capacitance, ie, 0 pf!  This way the transformer sees no load, and the

cap voltage equals the transformer 'open-circuit' voltage!  But of

course this isn't quite as useful as your actual need. :)

Months ago, Mr. Hammer and I spent a few days arguing about the *right*

cap size for a transformer.  I wrote out about two pages of equations,

starting by tracking cap voltage as a function of integrated transformer

output current, etc.  One of my assumptions was that the peak

transformer current was exactly the value stated on the nameplate - ie-

output current is a non-clipped sinewave, with an RMS of (according to

your example) 60 ma RMS.  And in the end - my method yielded the same

results as the traditional impedence matching process, ie, 13 nF for

your particular transformer.

Now - a cap slightly bigger than the one computed by impedence matching

*may* still work - but the current drawn from the transformer will be a

clipped sinewave, which greatly complicates the 'closed-form' solution

of the problem!  Most importantly, the peak cap voltage will occur later

in the cycle than the peak open circuit transformer voltage would have,

ie, 1/4 cycle.  This is because the cap voltage will always lag, or

follow, the open circuit transformer voltage.  This is true for any

capacitance greater than 0, which is to say, any real world capacitance.

The instantaneous cap charging current is a function of:

1) Instantaneous open circuit transformer voltage

2) Instantaneous cap voltage

3) Resistances in the secondary windings, etc.

4) Magnetic shunting (current limiting) effect of the transformer.

So I think that a cap slightly larger than the one computed by impedence

matching would still work.  However, if your cap is much too large,

you'll know it, since it won't ever build enough voltage to cross the

gap!

>Now I tried it another way, and came up with another answer. I was thinking

>that you would want to have a capacitor that would have as much energy as it

>could take and have a potential across it of 12 kv's at the first quarter

>cycle of your 60 hz sine wave, and then discharge on the 2nd quarter cycle,

>and recharge once more. Here's the values I get for my other capacitor.

>

>The time for one quarter cycle is: 1/60/4 = .004167 seconds

>

>The RMS value for current is 60ma, so that implies that the transformer is

>supplying .06 coulombs per second.

>

>So .06 coulombs/sec * .004147 secs = .00025 coulombs

>

>Now Q=C*V so C=Q/V --> C = .00025 coulombs / 12000 volts

>

>C = 20.833 nF

>

>

>Clearly there is a difference between 13.263 nF and 20.833 nF. Which one is

>better? Seems like I would want the larger value as it would be able to

>store more energy so when the gap fires, it will deliver more energy into

>the primary coil. My whole intention here is to maximize the energy

>delivered into the primary when the gap fires without exceeding the voltage

>rating of the neon and destroying it.

<snip>

I don't think you can over-voltage your neon, as it is the neon itself

that provides the voltage on the cap.  MIKE - ANY COMMENTS ABOUT THIS??

-BILL

The only possible flaw I see in your argument about coulombs is that it

assumes the current is constant over the 1/4 cycle.  This is obviously

not true.  The real current is limited to 60ma, or whatever the rating

of the transformer is.

This is a difficult problem to solve w/o knowing the exact V vs I

characteristics of the neon in question.  Even then, its quite nasty to

solve, probably requiring a numeric solution.

Food for thought - I have a small low power tesla coil and I charge my

cap using a HV DC supply - a rectified ignition coil output.  This way I

can run whatever size cap I want - because the HV supply will charge a

large cap just fine - although it obviously takes longer than a smaller

cap.  I get about 4 discharges per second with my 0.03 uF cap and about

20 per sec with a 0.008 uf unit.  Resonant frequency is somewhere near

400 KHz.

Remember that if you change your cap size, that will change the self

resonant frequency of your primary coil and cap combo.  You may have to

change your primary coil tap to keep the primary and secondary resonant

frequencies the same.  There are other ways of keeping these frequencies

lined up.

-Bill

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: Cap charging

Cc:

Bcc:

X-Attachments:

At 06:53 AM 9/30/97 PDT, you wrote:

<BIG SNIP>

>>

>>Clearly there is a difference between 13.263 nF and 20.833 nF. Which one is

>>better? Seems like I would want the larger value as it would be able to

>>store more energy so when the gap fires, it will deliver more energy into

>>the primary coil. My whole intention here is to maximize the energy

>>delivered into the primary when the gap fires without exceeding the

>voltage rating of the neon and destroying it.

>

><snip>

>

>I don't think you can over-voltage your neon, as it is the neon itself

>that provides the voltage on the cap.  MIKE - ANY COMMENTS ABOUT THIS??

>-BILL

>

Very wrong! You can and most likely will very easily over volt your

neon. Especially with a 60hz resonant condition.

Given a wide enough gap your main cap will charge to the peak

value of the transformer voltage. For a 15 kv neon this amounts to

21210 volts. This is way over the neon rating.

Given a 60hz resonant condition the voltage will try to go even

higher. Now a neon secondary isnt very efficient as an inductor

having a Q of perhaps 5. But even that will encourage quite high

voltages. It will try to rise to Q times the peak value. For our

15 kv neon that amounts to 106.05 KV! That is clearly overvolting

your poor neon.

Ok you say but we have our spark gap set to break down at 15kv

so it should clamp the voltage at that value, right? Well yea.

But the voltage is going to seek every opportunity to rise to

its maximum value. Whats happens if your gap misfires? It

happens. Even with static gaps. Voltage soars.

This is why you need a properly set safety gap right at the

neon itself. Its there to protect in case of a misfire or kickback.

Using your neons in 60 hz (or 50 hz for some of you) mode

is a way to extract more power from your neons than the nameplate

rating. But be careful. The harder you push the more likely things

will fail.

TTYL!

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: Cap charging

Date: Tue, 30 Sep 1997 18:06:45 GMT

Organization: The Energy Machine Information System 504-733-3462

From: "Thomas McGahee" <tom_mcgahee@sigmais.com>

Date: Tue, 30 Sep 1997 14:06:45 -0400

Subject: Re: Cap charging

----------

> From: Tesla-2 <tesla-2@emachine.com>

> To: tom_mcgahee@sigmais.com

> Subject: Cap charging

> Date: Tuesday, September 30, 1997 2:55 AM

>

> From: Dave <djb@erie.net>

> Date: Tue, 30 Sep 1997 02:55:26 -0400

> Subject: Cap charging

>

> Hi again,

>

> Thanks to all who answered my last questions on transformers and resonance.

> I have yet another question on resonance now. My question is, when u match

> the output impedance of your neon transformer to that of your capacitor at

> 60 hz, is that the fastest way to charge your cap?

>

> The reason I am asking this is because I am wondering how you determine the

> time it takes to charge up a cap. I guess you can measure the resistance of

> the circuit and figure out a RC time constant and solve it that way, but

> when I think that the circuit is resonant, I am not so sure anymore. Also,

> say you use a different value for a capacitor so it is no longer resonant at

> 60 hz, how do you determine the time? Do you use the same method for

> resonance than for not resonance?

>

> I'll show you an example of why I came up with this question. I was trying

> to determine the optimum value for a capacitor given a neon rated at 12 kv's

> and 60 ma's. The output impedance of that would be 200 K ohms. If I matched

> the impedance of a capacitor at 60 hz to that value, I get 13.263 nF.

>

> Now I tried it another way, and came up with another answer. I was thinking

> that you would want to have a capacitor that would have as much energy as it

> could take and have a potential across it of 12 kv's at the first quarter

> cycle of your 60 hz sine wave, and then discharge on the 2nd quarter cycle,

> and recharge once more. Here's the values I get for my other capacitor.

>

> The time for one quarter cycle is: 1/60/4 = .004167 seconds

>

> The RMS value for current is 60ma, so that implies that the transformer is

> supplying .06 coulombs per second.

>

> So .06 coulombs/sec * .004147 secs = .00025 coulombs

>

> Now Q=C*V so C=Q/V --> C = .00025 coulombs / 12000 volts

>

> C = 20.833 nF

>

>

> Clearly there is a difference between 13.263 nF and 20.833 nF.

>Which one is better? Seems like I would want the larger value as it

>would be able to store more energy so when the gap fires, it will deliver

>more energy into the primary coil. My whole intention here is to maximize the >energy delivered into the primary when the gap fires without exceeding the

>voltage rating of the neon and destroying it. Not counting higher gap

>losses due to the increased amount of current in the gap, would this be the

>right approach in determining the right cap size? It seems as if it would store

>more energy in the given amount of time, and have the correct voltage to fire

>the gap, but I am not sure what happens because I don't understand the

>difference between resonant and non-resonant charging.

>

> Thanks for you time. Sorry about the length of this.

>

> Dave

>

>

Dave,

I wrote rather extensively on this topic back in February on Chip's

Tesla List. You are right in that the actual scenario is not as

simple as it first seems. The following post takes a look at both the

current-limited and resistance-limited approaches to determining the

optimum size HV cap. Please note that BOTH these approaches assume

that you want to fire at the PEAK available HV cap voltage.

The post also covers some related topics. I have left the post

intact, because I think that it is all pertinent to the overal

discussion.

By the way, you will see that I did my calculations based on a 12KV

30 ma transformer. The results that I got are identical to what you

would have derived if you had used the same 12KV 30 ma transformer in

your calculations.

My explanation may be a bit drawn out, but I wanted to explain in

detail what the thinking process was. Not everyone initially sees

these things for themselves.

Hope you find it helpful.

Fr. Tom McGahee

----------

> From: Tesla List <tesla@poodle.pupman.com>

> To: Tesla-list-subscribers@poodle.pupman.com

> Subject: Re: Top Toroid

> Date: Friday, February 07, 1997 2:25 AM

>

> Subscriber: Esondrmn@aol.com Fri Feb  7 00:09:43 1997

> Date: Wed, 5 Feb 1997 16:00:08 -0500 (EST)

> From: Esondrmn@aol.com

> To: tesla@pupman.com

> Subject: Re: Top Toroid

>

> In a message dated 97-02-05 03:38:52 EST, you write:

>

> << John and all

>

> - snip-

>

>  I use only neons. The charge current is absolutely limited by the neon

>  to either 60 or 120ma(depends on neon being used). It would be

>  interesting if someone out there can calculate the time it actually

>  takes to charge a cap from a current limited neon. I have been unable to

>  come up with a formula which satisfies me. The formula would have to

>  assume a rated neon voltage and current and incorporate those parameters

>  in a formula which includes the cap value. With the timing determined,

>  it would be easy to determine if multiple firings are even possible.

>

>  By the way , in some testing I have done, it appears that the neon will

>  put out its max current only when the input voltage is at maximum. At

>  say 60 volts input you can only get 30 ma from a 60ma neon. This must be

>  taken into account in any formula and has been a stumbling point for me.

>

>  Help anyone!!!!

>

>  Skip

First I'll answer one of Skip's questions about the constant current being

less when the input voltage is less. Skip, this makes perfect sense when

you think of how the neon uses magnetic shunts to accomplish this magic.

When you lower the primary voltage you are bound to be lowering the

magnetic flux density from the primary, no? OK, so you have less magnetic

flux from the primary, but you still have these magnetic shunt thingies

that are going to siphon off some of the magnetic force. How much? Well,

they operate sort of like this. The more the secondary draws current, the

more the primary responds by increasing the primary *current* and therefore

the flux intensity. Primary voltage is kept fairly constant. The increase

in flux intensity is countered by the magnetic shunts which have gaps such

that only after the magnetic field reaches a certain value do they kick in

and create a good magnetic coupling. When they *do* kick in, this makes the

magnetic flux left over for the secondary pretty much the same as it was

before. So the current in the secondary tends to remain constant. When one

shunt becomes saturated it lets some of the extra magnetic flux from the

primary get past itself... where it is promptly gobbled up by the next

shunt in line. The point at which the magnetic shunts kick in is a factor

of the primary *voltage* and their gap setting. Gap size remains constant,

but primary voltage can be changed with a variac. With a lower primary

voltage the magnetic shunts cut in more weakly, reducing the current they

will shunt away... but there is already less current to begin with, because

the primary voltage is lower. Net result: lower primary voltage, lower

constant current in secondary. I do not think that you will find that the

relationship is perfectly linear, however. Also, at very low voltages the

resistance of the secondary wires will predominate, and percent of magnetic

loss due to the core characteristics will swamp all other factors.

>   >>

> Skip,

>

> It takes 5 time constants (if I remember correctly) to get to full charge.

> I don't remember the specs on your system.  My new 3" coil uses two 3,000 ohm

> series resistors to help protect the 15 kv, 30 ma transformer. Using 6,000

> ohms and .0047 mfd, one TC = 2.82 x 10 -5 seconds.  5 TC = .14 ms. One half

> cycle is 8.3 ms.

>

> For a current limited transformer, I think we really need to use the

> effective impedance of the transformer for R.  In this case it would be

> 500,000 ohms.  So one TC = 2.3 ms and 5 TC = 11.75 ms.  If one half cycle at

> 60 cycles is 8.3 ms, then my cap will never get fully charged.  I think I am

> missing something here.

>

> Fr. McGahee, what's the deal here?

>

> Ed Sonderman

Ed,

How can I refuse when you asked for me by name? You are right that when

using a simple RC series configuration the DC charge rate is T=R*C where T

is time in seconds to reach about 63% of the DC level to which we are

charging. Because the charge curve in such a circuit is not linear but a

curve that is asymptotic, it will take 5 of these T times to reach about

95% of full charge.

Ah, but Ed, my friend, we is not dealing with the friendly DC here.

We are trying to charge up to a voltage that is constantly *changing*.

Oh Joy! Mr. AC he is so wonderful! Let's skip all the dreadful math and

simply say that the net result of all this is that the capacitor is always charging *towards* the changing AC level. It isn't a matter of whether the AC

wave is going up or going down, but whether it is above or below the

capacitor voltage at any given instant. *That* is what determines the

*direction* of the capacitor voltage.

If you were to be so silly as to plot out what this looks like on a piece

of graph paper, then you would find that after a few cycles the capacitor

voltage waveform will settle down to being a sine wave that is shifted by

up to 90 degrees from the AC sine wave that is charging it. The amplitude

of the waveform at the capacitor is a function of R, C, the AC voltage

charging the capacitor, and the frequency. Assume that R=1 ohm and C=.001

mfd... then in this case the capacitor voltage would track the original AC

signal almost exactly as to both phase and amplitude. Increasing either R

or C would increase the phase difference (to a maximum of -90 degrees),

and decrease the amplitude of the voltage waveform across the capacitor to

something approaching zero. Increasing frequency will cause the maximum

voltage amplitude to decrease also. (The capacitor has less time to

charge). And none of this stuff is linear! No wonder my students prefer DC

theory to AC theory any day of the week!

So that's one reason why your math fails to give you a reasonable answer,

Ed. But have no fear!!! It is not important at all that your silly

capacitor never gets charged to *the* maximum (if by maximum you mean the

maximum voltage from the transformer). What is important is that there is a

maximum charge reached by your capacitor, and *that* is the voltage

you want your spark gap to fire at. It will be a -90 degrees phase

shifted sine wave for all practical purposes.

*** Some stuff on Rotary Gaps related to the Peak Firing Point

In a rotary gap if you are going for Big Bang, you are trying to match the

rotary position of the gap elements to the actual peak of the capacitor

voltage such that the capacitor is at its peak value when the gaps are the

exact distance apart to initiate breakdown. Where exactly that is depends

on the presenting distance of the gap and its phase angle. In a synchronous

rotary spark gap you adjust the phase angle by adjusting the rotational

relationship between the motor housing and the rotating elements mounted to

the motor. If the presenting distance is too close, then the gap may fire

again before the rotating element gets far enough away from the stationary

element to prevent a second discharge. Both the size of the gap elements

and their distance come into play. When run in Big Bang mode there would be

120 pulses per second. The speed of the motor and the arrangement of rotor

elements determines the number of Breaks Per Second. Anything less that 120

Breaks Per Second (at 60 Hz) and you are wasting stored energy.

Faster than 120 Breaks Per Second and you will sometimes be firing early

and therefore not reaching peak voltage before the gap fires. You will, however

have more firings per second, and although the energy per Bang is less, the

results may be more to your liking, as the character of the sparks is

different at different break rates.

An asynchronous rotary spark gap is one in which the speed of the motor can

be varied. These type gaps are easier to build than the synchronous ones,

and allow for more experimentation, but are difficult to get to synchronize

if desired.

By the way, if we remove the energy at any other time than the peak, we

*may* still get the same total energy out, but we will have lost our wonderful *Peaks*.

There are also timing scenarios with a rotary gap that cause unnecessary losses.

If you had capacitors that could take it, and you moved your spark gap opening out to let's say some ridiculous distance like a foot, then the gap would never fire, and *all* the energy would be wasted

every single cycle. Talk about wasteful! By the way, the wasted energy

would show up primarily as heating of the transformer as it alternately

pumped current into the capacitor and then worked to pump it out again.

*** Matching Transformers to Capacitors: Constant Current Approach

On a slightly different note, matching the transformer and tank capacitor

helps us to maximize the ratio of the useable power out to the available

power in. This has very much to do with the rate at which the transformer

can charge the capacitor.

There is a way to calculate the relationship of transformer and capacitor

such that we can maximize the efficiency with which we deliver the *charge*

that we store in the capacitor. Once we have this maximized stored charge,

we want to get the most *use* out of it. That happens when we dump the

energy through the spark gap at the instant the capacitor voltage is

at its maximum voltage level.

I want to say something about the neon transformer as a constant current

device. A perfect constant current source will charge a capacitor linearly.

Thus a perfect constant current AC source would charge and discharge a

capacitor in what would be a Triangle Wave instead of a sine wave.

But the Neon transformer is *not* a perfect constant current device. So I

would imagine that if you were to look at the charge/discharge cycle of a

neon transformer and capacitor (no spark gap or any other load), then you

would see a waveform that was a cross between a triangle and a sine wave.

The top of the waveform would be rounded like a sine wave, but the sides

would be a bit straighter than a sine wave.

As far as charging time goes, it is very easy to determine for a true

constant current source.  V=I*T/C

That is: Voltage at time T is equal to constant current in amps times

Time in seconds divided by capacitance in farads

Rearranging in terms of T we get  T=V*C/I

Please note that in the above equation you must qualify the voltage

as being the voltage at time T. Thus the formula says that the Time in

seconds for the Voltage to reach a particular value is equal to the

Capacitance in farads multiplied by the voltage *at that time* divided by the

constant current in amps.

Now the "*at that time*" is important. Imagine 0 to 180 degrees of a

sine wave. The formula implies that if the sine wave represents the

maximum available voltage that the constant current source can produce *at

any instant in time* then the output voltage can be represented as a

straight line drawn from the 0 degree point to the time point desired along

the sine wave. For example, if its a 60 Hz sine wave with a peak value of

12KV, then 90 degrees would be (90/360)*1/60=.004166 of a second. That would be

the T.

Rearrange the equation in terms of C for a 30 ma transformer and

you get

C=T*I/V        C=.004166*.030/12000   C=.010415 microfarads

So a .01 mfd capacitor will charge to a Peak value of 12KV in 1/4 of a

cycle if the constant current is 30 ma. But please remember that the neon

transformer does *not* come near this ideal! The constant current is more

like a maximum constant current. It falls short of the ideal both near the

low end (0 volts) and the high end (12KV) of its cycle.

*** Matching Transformers to Capacitors: Capacitive Reactance Approach

For those who are interested in the derivation of the formula most commonly

used to calulate the proper matching capacitor for a transformer (neon or

pole pig): It is based on the formula for Capacitive Reactance:

Xc=1/(2*pi*f*C)  where Xc is capacitive reactance in ohms, f is frequency

in hertz, and C is capacitance in Farads.

Now, since matching is actually based on the fact that at resonance Xc=Xl,

and since the Xl of a transformer is equal to its effective

resistance, and since R=V/I  people substitute V/I for Xc.

So now (V/I)=1/(2*pi*f*C)    rearranging for C we get

C=1/(2*pi*f*(V/I))

Substituting values for a 12KV 30 ma neon we get

C=1/(2*3.14*60*12000/.030)   so C=.00634 mfd

Now it should really be mentioned here that the above formula makes the

assumption that Inductive Reactance=V/I for a transformer. No distinction

is made in the formula for the differences that exist between a pole pig

and a neon. This disturbs my sense of reality a bit. I mean, if you short

out a neon rated at 12KV and 30 ma the thing current limits. Take a 14.4KV

@ 1 amp pole pig and short the two leads together, and I sure hope you

connected an arc welder in the primary circuit someplace, because otherwise

it is meltdown time! In a pole pig *you* are responsible for supplying the

current limiting.

Also, the fact that the values used on known operating Tesla coils match

closely to the values given in the formula *could* very well simply mean

that someone used that formula to arrive at their value. *That* says

*nothing* to me about the validity of the formula per se, because that is

an example of circular reasoning. I am somewhat content with the fact that

people like Richard Hull are satisfied with the values this formula gives.

According to them, these are good working values. Richard Hull *also* says

that some people use bigger values with some success. Perhaps these

*bigger* values correspond to the values we get using the formula derived

from the constant current scenario. Maybe what we should be telling people

is that the value of capacitor to be used for a 12KV 30 ma rated

transformer (of any type, assuming it has some form of current limiting or

ballast) is between .00634 and .010415 mfd. That at least would jibe with

the fact that these transformers are performing as something less than a

pure constant current generator, but better than a purely resistive type

source. So the value we should use is *somewhere* in there, *between* those

two values. I can sleep at night with reasoning like that.

And so, I will go off to bed now for a good night's sleep. Comments

welcome.

Fr. Tom McGahee

> Processed with Listserv v2.93 for Wildcat v4

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: Transformer connections

Date: Tue, 30 Sep 1997 19:33:16 GMT

Organization: The Energy Machine Information System 504-733-3462

From: "Thomas McGahee" <tom_mcgahee@sigmais.com>

Date: Tue, 30 Sep 1997 15:33:16 -0400

Subject: Re: Transformer connections

>

> From: Eric Davidson <edavidson@icva.gov>

> Date: Tue, 30 Sep 1997 10:32:10 -0500

> Subject: Transformer connections

>

> Hello everyone:

>

> I noticed several postings about using microwave oven transformers in

> series for Tesla coils and I thought I would add my 2 cents worth on

> connecting transformers in series.  Connecting high voltage transformers

> in series is risky for several reasons.  The main problem is the

> strength of the secondary insulation.  Most HV transformers (of the neon

> sign or cold cathode type) have the midpoint of the secondary attached

> to the case/core, or as is the case in oil burner transformers, one end

> of the secondary is connected to the case.  This lowers the potential

> between the HV bushings and the case, lessening the the size requirement

> for the bushings (and in oil burner transformers, eliminating one

> bushing) and lessening electrical stresses within the case (between

> windings, windings and core, windings and case etc.). In most

> transformers the primary winding is isolated from the case and a

> separate lug is provided for grounding.  This is always the case for

> transformers for 240 or 480 volt operation (for obvious reasons).

> When an attempt is made to place transformers in series several problems

> begin to arise. How do we deal with that pesky secondary midpoint? If we

> unpot and physically remove the connection to the core/case we double

> the potential between the case and the HV lugs (and hope that the

> designers accounted for this eventuality in their design) and we haven't

> even put two in series yet! So, if we put two transformers in series

> with their cases grounded, we have 4 times the rated voltage between the

> case and the HV lugs! Its doubtful there is this much margin in the

> design. We could leave the secondary midpoint grounded and let the cases

> float (checking for primary isolation first). By leaving the midpoint

> connection we assure that each transformer has its rated voltage from

> the HV lugs to the case, but of course now the cases are floating and

> very unsafe. They must be insulated from ground (and people). This

> brings up the problem of primary isolation. This is a big one, and

> basically there is no way around it. The closest you can come is to

> power the transformers from a line isolation transformer. Most isolation

> transformers have good hi pot numbers (though not up to tens of kV).

> This arrangement allows the entire primary to float, and gets around the

> fact that the midpoint of the utility company's secondary midpoint is

> grounded (actually called neutral, it does carry the unbalanced load).

> In my humble opinion, if you are considering a series connection, use an

> ODD number of transformers. Keep the secondary midpoints connected to

> the core/case. Let the cases of the outer two (if you use 3)

> transformers float, ground the case of the center one. You MIGHT be able

> to get by if the secondary voltage of each is around 5 kV (I believe

> microwave oven transformers are in this range, maybe a little lower).

> Remember, the transformers must have equal impedances (output CURRENT

> ratings) if the voltage is to divide correctly. If the VOLTAGE ratings

> are different, ground the one with the LOWEST rating. Pay attention to

> phase!

> I noticed that some folks immerse the transformers in oil. While

> this is a good idea to prevent flash over on the outside of the case it

> increases the stress inside the case.  If a transformer breaks down

> inside it is usually ruined because the potting material becomes quite

> conductive after burning with an arc.  IMHO, use no oil, let it flash on

> the outside if it is going to, consider it a safety gap. Old neon

> transformers are notorious for an inside breakdown, especially if they

> have been to small for the length tubing they powered. Also, if you have

> the choice, it is better to place transformers of a higher voltage and

> lower current in PARALLEL than vice versa in SERIES. I do realize you

> have to play the hand you are dealt when finding parts!

>

> Eric

>

Eric,

The proper way to connect two microwave transformers in series is as

follows:

Connect the two cases together by using a wire, and then connect the

two strapped cases to the AC safety ground. This will ensure that

both cases reside at ground level.

The primaries are then connected in ANTI-PARALLEL. This is just a

fancy term for connecting the primaries in parallel, but with the

phasing of each transformer different. For example, assume that one

transformer has primary wires A and B and the second transformer has

primary wires A' and B'  You would parallel A with B' and B with A'.

This opposite phasing scheme forces the two transformer secondaries

to ALSO be out of phase. Thus when one is producing +max, the other

is producing -max. In effect you have made something very similar to

a neon transformer, where the two HV windings are both attached to

the core. Please note that the primary windings now see a core that

resides at ground, so all is safe.

This method works extremely well, and in fact does not put any

additional strain on the transformers at all.

******* more is more dangerous...

Some persons on the Tesla List have connected up more than two

microwave transformers. Usually the inner two transformers are

connected as described, with the outer transformers connected in a

floating series arrangement. As you noted, this would mean that the

outer transformers would have to have the core disconnected

electrically. The primaries will then be at risk unless the outer

transformer cores are also grounded. But grounding the cores

increases the chances of HV breakdown to the core. You take your

chances with either arrangement. Immersing in oil greatly reduces the

dangers, but it is messy and cumbersome.

Bottom line: connecting up TWO microwave transformers can be

accomplished with 100% chance of success. Attempting more puts the

additional transformers at risk.

Hope this helps.

Fr. Tom McGahee

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Capacitance

Date: Tue, 30 Sep 1997 22:06:53 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Shane Randle <randlec@scnc.addison.k12.mi.us>

Date: Tue, 30 Sep 1997 18:06:53 -0400

Subject: Capacitance

This is just one of those simple little formulas/principals, but I can't

seem to remember it. Can anyone tell me the effect on capacitance when

caps are put in series or in parallel? I don't know how to clarify the

question, but is the capacitance of two caps added when they're in

parallel, or is it something else? I'm sure one of you understands what

I'm trying to ask. Thanks in advance for any information.

-Shane Randle

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: Cap charging

Date: Tue, 30 Sep 1997 22:09:45 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Dave <djb@erie.net>

Date: Tue, 30 Sep 1997 18:09:45 -0400

Subject: Re: Cap charging

<snip>

>Ok you say but we have our spark gap set to break down at 15kv

>so it should clamp the voltage at that value, right? Well yea.

>But the voltage is going to seek every opportunity to rise to

>its maximum value. Whats happens if your gap misfires? It

>happens. Even with static gaps. Voltage soars.

>

>This is why you need a properly set safety gap right at the

>neon itself. Its there to protect in case of a misfire or kickback.

<snip>

>TTYL!

>Mike Hammer

>mhammer@misslink.net

I am not sure what you mean about a gap misfiring Mike. Let's say the gap

was set to fire at aproximately 12 kv's. Under normal conditions it would

fire when the cap voltage reached 12 kv's, or if there were ions still

present in the gap, it would fire at less than 12 kv's. I cannot see the

insulating value of air changing significantly to cause the gap to fire

above 12 kv's if that's what the gap is set for...so how does a misfire

occur and voltage soar from this?

If the gap is set to fire at 12 kv's, shouldn't it always fire at 12 kv's

unless there are unwanted ions present?

Dave

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: off the subject, but...

Date: Tue, 30 Sep 1997 22:59:07 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Shane Randle <randlec@scnc.addison.k12.mi.us>

Date: Tue, 30 Sep 1997 18:59:07 -0400

Subject: Re: off the subject, but...

Tesla-2 wrote:

>

> From: corr <corr@fullnet.net>

> Date: Tue, 30 Sep 1997 17:01:40 -0600

> Subject: off the subject, but...

>

> I know this is a little off the subject, but does anyone here

> have, or know how to build a coil/rail gun?  Or any other

> thing that propels ferro-magnetic material?

>

Over the summer I was investigating the feasability of building such a

device. At first, I was under the impression that a railgun consisted of

many round solenoid type electromagnets, turned on and off in rapid

sequence to propel the object, but apparently, it consists mainly of two

rails, and the projectile goes between them to complete the circuit. The

parallel rails create an electrical field (called "Lorentz Force") which

drives the projectile.

When I was looking into this, I thought that a simple neon transformer

could power it, but it turns out that the power requirements are

somewhere in the thousands of volts, and several hundred amps range.

I find these devices rather fascinating, and if you're still curious

about them (which I'm sure you are) you might want to check out this

site at http://www.pao.nps.navy.mil/railgun.html

-Shane Randle

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: off the subject, but...

Date: Tue, 30 Sep 1997 23:01:40 GMT

Organization: The Energy Machine Information System 504-733-3462

From: corr <corr@fullnet.net>

Date: Tue, 30 Sep 1997 17:01:40 -0600

Subject: off the subject, but...

I know this is a little off the subject, but does anyone here

have, or know how to build a coil/rail gun?  Or any other

thing that propels ferro-magnetic material?

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: Capacitance

Cc:

Bcc:

X-Attachments:

At 10:06 PM 9/30/97 GMT, you wrote:

>From: Shane Randle <randlec@scnc.addison.k12.mi.us>

>Date: Tue, 30 Sep 1997 18:06:53 -0400

>Subject: Capacitance

>

>This is just one of those simple little formulas/principals, but I can't

>seem to remember it. Can anyone tell me the effect on capacitance when

>caps are put in series or in parallel? I don't know how to clarify the

>question, but is the capacitance of two caps added when they're in

>parallel, or is it something else? I'm sure one of you understands what

>I'm trying to ask. Thanks in advance for any information.

>-Shane Randle

>

>

This should take care of you Shane.

CAPACITORS IN PARALLEL:

|      |

|__C1__|

|      |

|__C2__|

|      |

|__C3__|

|      |

Capacitance = C1 + C2 + C3, etc...

Maximum voltage rating will be equal to the voltage rating of the

lowest voltage capacitor of the group.

=======================================================================

CAPACITORS IN SERIES:

|              |

|__C1__C2__C3__|

|              |

|              |

1       1       1

Capacitance = 1 / [---] + [---] + [---], etc...

C1      C2      C3

The total capacitance of several capacitors in series will always be

LESS than that of the smallest capacitor.

Total voltage rating increases with number of capacitors in series.

Simply add the voltage ratings together if the caps have identical

ratings.

When capacitors are placed in series to increase voltage rating they

should have the same capacitance and voltage rating else voltages will

divide unevenly, most likely causing failure.

TTYL!

Mike Hammer

mhammer@misslink.net

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: Cap charging

Cc:

Bcc:

X-Attachments:

At 10:09 PM 9/30/97 GMT, you wrote:

>From: Dave <djb@erie.net>

>Date: Tue, 30 Sep 1997 18:09:45 -0400

>Subject: Re: Cap charging

>

><snip>

>

>>Ok you say but we have our spark gap set to break down at 15kv

>>so it should clamp the voltage at that value, right? Well yea.

>>But the voltage is going to seek every opportunity to rise to

>>its maximum value. Whats happens if your gap misfires? It

>>happens. Even with static gaps. Voltage soars.

>>

>>This is why you need a properly set safety gap right at the

>>neon itself. Its there to protect in case of a misfire or kickback.

>

><snip>

>

>>TTYL!

>

>>Mike Hammer

>>mhammer@misslink.net

>

>I am not sure what you mean about a gap misfiring Mike. Let's say the gap

>was set to fire at aproximately 12 kv's. Under normal conditions it would

>fire when the cap voltage reached 12 kv's, or if there were ions still

>present in the gap, it would fire at less than 12 kv's. I cannot see the

>insulating value of air changing significantly to cause the gap to fire

>above 12 kv's if that's what the gap is set for...so how does a misfire

>occur and voltage soar from this?

>

>If the gap is set to fire at 12 kv's, shouldn't it always fire at 12 kv's

>unless there are unwanted ions present?

>

>Dave

>

Dave,

Lots of things can cause gap misfires. Once of the most common

is RF kickbacks. This surge of RF across the gap disrupts normal

operation.

A primary/secondary strike or arc can cause all manner of problems.

Here is an easy one. Oxidation of gap electrode faces. Often after

a number of long runs and after cooling off my gap wont fire at all

even with the juice cranked up full. Needless to say thats not nice

on your caps or neons. So I clean the faces on my electrodes

rather often.

TTYL!

Mike Hammer

mhammer@misslink.net

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: Capacitance

Date: Tue, 30 Sep 1997 23:59:54 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Dave <djb@erie.net>

Date: Tue, 30 Sep 1997 19:59:54 -0400

Subject: Re: Capacitance

At 10:06 PM 9/30/97 GMT, you wrote:

>From: Shane Randle <randlec@scnc.addison.k12.mi.us>

>Date: Tue, 30 Sep 1997 18:06:53 -0400

>Subject: Capacitance

>

>This is just one of those simple little formulas/principals, but I can't

>seem to remember it. Can anyone tell me the effect on capacitance when

>caps are put in series or in parallel? I don't know how to clarify the

>question, but is the capacitance of two caps added when they're in

>parallel, or is it something else? I'm sure one of you understands what

>I'm trying to ask. Thanks in advance for any information.

>-Shane Randle

CAPS IN PARALLEL : Ceq = C1 + C2 + C3 + ....

CAPS IN SERIES : 1     1     1

-  =  -  +  -  +  ...

Ceq    C1    C2

where Ceq is the equivalent resistance.

* note * Caps in parallel have the same voltage across them, but different

amounts of charge. Caps in series have different potentials, but the same

amount of charge. Hope this helps.

Dave

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: Capacitance

Date: Wed,  1 Oct 1997 00:03:08 GMT

Organization: The Energy Machine Information System 504-733-3462

From: "Thomas McGahee" <tom_mcgahee@sigmais.com>

Date: Tue, 30 Sep 1997 20:03:08 -0400

Subject: Re: Capacitance

----------

> From: Tesla-2 <tesla-2@emachine.com>

> To: tom_mcgahee@sigmais.com

> Subject: Capacitance

> Date: Tuesday, September 30, 1997 6:06 PM

>

> From: Shane Randle <randlec@scnc.addison.k12.mi.us>

> Date: Tue, 30 Sep 1997 18:06:53 -0400

> Subject: Capacitance

>

> This is just one of those simple little formulas/principals, but I can't

> seem to remember it. Can anyone tell me the effect on capacitance when

> caps are put in series or in parallel? I don't know how to clarify the

> question, but is the capacitance of two caps added when they're in

> parallel, or is it something else? I'm sure one of you understands what

> I'm trying to ask. Thanks in advance for any information.

> -Shane Randle

>

Shane,

Capacitances in parallel add up. Ct=C1+C2+C3...

In Series capacitances diminish.  1/Ct=1/C1 + 1/C2 + 1/C3 ...

In Parallel the voltage rating is equal to the smallest cap voltage rating.

In Series the voltage rating is more complex. If ALL the cap

capacitance and voltage ratings are EQUAL, then you can simplify and

say that they add up. If either the capacitance or voltage rating of

any cap is different, then you cannot use this simplification.

Instead you have to use the fact that each cap will store an equal

charge when in series, and then calculate the voltage for each cap

from there.

For Tesla use ALWAYS use identical capacitors when placing them in

series. Or at least keep the capacitance values the same, and then

calculate total voltage based on the SMALLEST voltage rating in the

string.

Hope this helps.

Fr. Tom McGahee

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: Cap charging

Date: Wed,  1 Oct 1997 00:14:52 GMT

Organization: The Energy Machine Information System 504-733-3462

From: "Thomas McGahee" <tom_mcgahee@sigmais.com>

Date: Tue, 30 Sep 1997 20:14:52 -0400

Subject: Re: Cap charging

> From: Dave <djb@erie.net>

> Date: Tue, 30 Sep 1997 18:09:45 -0400

> Subject: Re: Cap charging

>

> <snip>

>

> >Ok you say but we have our spark gap set to break down at 15kv

> >so it should clamp the voltage at that value, right? Well yea.

> >But the voltage is going to seek every opportunity to rise to

> >its maximum value. Whats happens if your gap misfires? It

> >happens. Even with static gaps. Voltage soars.

> >

> >This is why you need a properly set safety gap right at the

> >neon itself. Its there to protect in case of a misfire or kickback.

>

> <snip>

>

> >TTYL!

>

> >Mike Hammer

> >mhammer@misslink.net

>

> I am not sure what you mean about a gap misfiring Mike. Let's say the gap

> was set to fire at aproximately 12 kv's. Under normal conditions it would

> fire when the cap voltage reached 12 kv's, or if there were ions still

> present in the gap, it would fire at less than 12 kv's. I cannot see the

> insulating value of air changing significantly to cause the gap to fire

> above 12 kv's if that's what the gap is set for...so how does a misfire

> occur and voltage soar from this?

>

> If the gap is set to fire at 12 kv's, shouldn't it always fire at 12 kv's

> unless there are unwanted ions present?

>

> Dave

>

Dave,

Sorry, Mike, but Dave is right about the gap firing at 12KV. Now a

rotary is a whole 'nother animal and asynchronous gaps can definitely

fire over a WIDE range of voltages. Synchronous rotary gaps are

better, because they are phase locked. But if the voltage were to cut

out for a second or two and then come back on, then the gap might

take a second or two to lock back in. So a synchronous gap CAN be a

problem if it "slips" and looses sync for any reason. But a STATIC

gap is fixed. Ions can REDUCE the firing point, but not raise it

higher. A blast of air might raise the firing voltage, though. Is

that what you were thinking of, Mike? In a vacuum gap, increasing the

vacuum will definitely change the firing voltage. But once it is set

and the air flow is stable, it will remain there. Humidity and other

environmental factors will influence the breakdown voltage, but these

factors are generally slow moving events. Of course, if you suddenly

shot a blast of some compressed gas between the gaps, then the gap

voltage would temporarily rise. But most of us don't go around

blasting compressed gases through our gaps on a regular basis :)

Hope this helps.

Fr. Tom McGahee

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: Cap charging

Date: Wed,  1 Oct 1997 04:01:03 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Dave <djb@erie.net>

Date: Wed, 1 Oct 1997 00:01:03 -0400

Subject: Re: Cap charging

Hi again,

Thanks everyone for your help. I have another related question about

capacitors and charging as well as resonance. I have seen schematic where

the capacitor is put in series with the primary coil and the gap is in

parallel across the neon secondary. And I have also seen it where they are

switched...ie the capacitor is in parallel with the neon secondary and the

gap is in series with the primary coil.

I know it is preferred to have the capacitor in series and the gap in

parallel to protect the neon from damage, but what I am wondering now is

there much difference now in charging the cap since the current to charge it

must also flow through the primary coil, whereas if the cap was in parallel

with the neon, the current to charge it would not flow through the primary

coil and be effected by it's inductance?

Also, can you consider both of these configurations as series RLC circuits,

or one as series and the other as parallel?

Dave

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: Capacitance

Date: Wed,  1 Oct 1997 04:36:58 GMT

Organization: The Energy Machine Information System 504-733-3462

From: randy@gte.net

Date: Tue, 30 Sep 1997 21:36:58 -0700

Subject: Re: Capacitance

Tesla-2 wrote:

>

> From: Shane Randle <randlec@scnc.addison.k12.mi.us>

> Date: Tue, 30 Sep 1997 18:06:53 -0400

> Subject: Capacitance

>

> This is just one of those simple little formulas/principals, but I can't

> seem to remember it. Can anyone tell me the effect on capacitance when

> caps are put in series or in parallel? I don't know how to clarify the

> question, but is the capacitance of two caps added when they're in

> parallel, or is it something else? I'm sure one of you understands what

> I'm trying to ask. Thanks in advance for any information.

> -Shane Randle

>

Caps act just the opposite of resistors.... series divideds the value,

parallel increases it.

Randy

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: off the subject, but...

Date: Wed,  1 Oct 1997 05:51:36 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Bert Hickman <bert.hickman@aquila.com>

Date: Tue, 30 Sep 1997 22:51:36 -0700

Subject: Re: off the subject, but...

Organization: Stoneridge Engineering

Tesla-2 wrote:

>

> From: corr <corr@fullnet.net>

> Date: Tue, 30 Sep 1997 17:01:40 -0600

> Subject: off the subject, but...

>

> I know this is a little off the subject, but does anyone here

> have, or know how to build a coil/rail gun?  Or any other

> thing that propels ferro-magnetic material?

>

This is s subject better addressed on Steve Roy's High Voltage List.

Briefly, a rail gun uses a pair of parallel conductors (the "rails") and

a sliding conductive element which shorts the two. A huge current is

applied (typically from either a homopolar generator of a capacitor

bank). The large magnetic field which is created acts in such a manner

as to propel the sliding element, which can be an arc or plasma, in such

a direction as to increase the size of the current loop which is formed.

Supersonic velocities can be achieved in short distances if the gun is

fired in a vacuum. However, LOTS of energy is required to do this

effectively.

-- Bert --

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: Cap charging

Date: Wed,  1 Oct 1997 08:12:23 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Dave <djb@erie.net>

Date: Wed, 1 Oct 1997 04:12:23 -0400

Subject: Re: Cap charging

Hi,

Well, I think I figured out the answer to my last question about whether or

not the primary coil makes a difference when charging a cap. I don't think

so now as the impedance at 60 hz would be very small compared to the other

impedances in the circuit.

I was wondering though if the equation Ec=0.5 * C * V^2 would be valid for

charging the capacitor at resonance? If so, I would know how much total

energy the capacitor would have before the gap fired provided it had time to

charge. Now I am just not sure how much of that energy would actually be

used due to gap losses, RC discharge time constant, etc? Any estimates on this?

Also, Bert Hickman talked about a resonance curve. Would you just get that

from plotting the total circuit impedance divided by the resistance over a

frequency range and looking at the cutoff frequencies?

Thanks again for your help.

Dave

PS Thanks Fr Tom for that long post. It is most helpful.

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: Cap charging

Date: Wed,  1 Oct 1997 14:03:05 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Bert Hickman <bert.hickman@aquila.com>

Date: Wed, 01 Oct 1997 07:03:05 -0700

Subject: Re: Cap charging

Organization: Stoneridge Engineering

Dave,

My comments are below...

-- Bert --

Tesla-2 wrote:

>

> From: Dave <djb@erie.net>

> Date: Wed, 1 Oct 1997 04:12:23 -0400

> Subject: Re: Cap charging

>

> Hi,

>

> Well, I think I figured out the answer to my last question about whether or

> not the primary coil makes a difference when charging a cap. I don't think

> so now as the impedance at 60 hz would be very small compared to the other

> impedances in the circuit.

>

> I was wondering though if the equation Ec=0.5 * C * V^2 would be valid for

> charging the capacitor at resonance? If so, I would know how much total

> energy the capacitor would have before the gap fired provided it had time to

> charge. Now I am just not sure how much of that energy would actually be

> used due to gap losses, RC discharge time constant, etc? Any estimates on this?

>

Dave, this is VERY relevent to Disruptive (capacitor discharge) Tesla

Coils. The total amount of energy that's available in the TC primary

circuit each time the gap fires will be 0.5CpVg^2, where Vg is the gap

firing voltage. This chunk of energy, termed the "bang size" represents

the maximum available energy that could be transferred to the secondary

whenever the gap fires. Once the gap fires, the tuned primary LC circuit

begins oscillating, transferring energy to the tuned secondary LC

circuit electromagnetically. The secondary tuned circuit thus gains

energy at a rate that's governed by the degree of electromagnetic

coupling (called k, the coefficient of coupling) between the two.

Typical coils will have a k between 0.1 and 0.25. The sum of the energy

in the primary and secondary will be limited to the initial bang size,

since Tesla Coils do not violate Conservation of Energy.

However, while the gap is firing, tremendous energy is being dissipated

as heat and light, so a fair portion of the original energy is being

lost through the gap. In addition, resistive losses in the primary and

secondary circuits and in the secondary's base path to the RF ground are

also causing energy loss through I^2*R heating. If there were no losses

in a Tesla Coil, ALL the energy original "bang" energy would be

transferred to the secondary over a few cycles. In the real world,

losses in a well designed Tesla Coil limit this to perhaps 80-85% of the

energy making it to the secondary, with 50-65% being the norm for most

typically constructed coils.

> Also, Bert Hickman talked about a resonance curve. Would you just get that

> from plotting the total circuit impedance divided by the resistance over a

> frequency range and looking at the cutoff frequencies?

>

> Thanks again for your help.

>

> Dave

>

> PS Thanks Fr Tom for that long post. It is most helpful.

This is certainly one way, assuming you had a large enough (1000-2000

watt) audio power amplifier to adequately drive the transformer at a

range of frequencies around the line frequency. The problem is that the

neon's internal structure causes it to behave quite non-linear when we

use it near its faceplate current rating - it's inductance is a

non-linear function of secondary current. A typical 12 KV 60 MA trannie

will have an inductive reactance at 60 Hz of about 200,000 (X) ohms,

while the DC resistance (R) will be perhaps 6000 ohms between HV

terminals. Using the ratio X/R would imply a Q of over 33. However,

experiments show that most neons actually have a much lower Q, typically

in the range of of about 4 - 10 at line frequency.

This relatively low Q means that the shape of the resonance curve is

quite broad. It can be shown that the half power points on the resonance

curve will be:

(F2 - F1) = Fo/Q

where F1 = lower half-power frequency

F2 = upper half-power frequency

Fo = line frequency

We will also assume that F1 and F2 are symetrical about Fo.

Plugging in Fo = 60 Hz, and the range of Q's:

Low Q:  (F2 - F1) = 60/4 = 15 Hz

F1 = 52.5 Hz, F2 = 67.5 Hz

High Q:  (F2 - F1) = 60/10 = 6 Hz

F1 = 57 Hz, F2 = 63 Hz

Hope this sheds a little bit more light on the subject!

Safe coilin' to you, Dave!

-- Bert --

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: Transformer connections

Date: Wed,  1 Oct 1997 16:28:42 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Eric Davidson <edavidson@icva.gov>

Date: Wed, 01 Oct 1997 11:28:42 -0500

Subject: Re: Transformer connections

Tesla-2 wrote:

>

> From: "Thomas McGahee" <tom_mcgahee@sigmais.com>

> Date: Tue, 30 Sep 1997 15:33:16 -0400

> Subject: Re: Transformer connections

> Eric,

> The proper way to connect two microwave transformers in series is as

> follows:

>

> Connect the two cases together by using a wire, and then connect the

> two strapped cases to the AC safety ground. This will ensure that

> both cases reside at ground level.

>

> The primaries are then connected in ANTI-PARALLEL. This is just a

> fancy term for connecting the primaries in parallel, but with the

> phasing of each transformer different. For example, assume that one

> transformer has primary wires A and B and the second transformer has

> primary wires A' and B'  You would parallel A with B' and B with A'.

>

> This opposite phasing scheme forces the two transformer secondaries

> to ALSO be out of phase. Thus when one is producing +max, the other

> is producing -max. In effect you have made something very similar to

> a neon transformer, where the two HV windings are both attached to

> the core. Please note that the primary windings now see a core that

> resides at ground, so all is safe.

>

> This method works extremely well, and in fact does not put any

> additional strain on the transformers at all.

>

> ******* more is more dangerous...

>

> Some persons on the Tesla List have connected up more than two

> microwave transformers. Usually the inner two transformers are

> connected as described, with the outer transformers connected in a

> floating series arrangement. As you noted, this would mean that the

> outer transformers would have to have the core disconnected

> electrically. The primaries will then be at risk unless the outer

> transformer cores are also grounded. But grounding the cores

> increases the chances of HV breakdown to the core. You take your

> chances with either arrangement. Immersing in oil greatly reduces the

> dangers, but it is messy and cumbersome.

>

> Bottom line: connecting up TWO microwave transformers can be

> accomplished with 100% chance of success. Attempting more puts the

> additional transformers at risk.

>

> Hope this helps.

> Fr. Tom McGahee

Fr. Tom:

Thanks for your response to my note about transformer connections.  From

your comments I assume that a microwave transformer has a single

secondary bushing, a la oil burner ignition transformers. I guess my

over all opinion is that placing transformers of the neon type in series

is just not worth the hassle and risk.  Your arrangement of two

microwave trannys 'back-to-back' is very acceptable (in fact

electrically duplicating a transformer with a grounded secondary

midpoint, forget about the extra primary for the moment).  I am

surprised to see that oil burner transformers are not used in TC

circuits more often.  They have several advantages: They are what I call

'single ended' (carry over from my tube amp hobby) meaning one end of

the secondary is grounded.  They are usually easy to find used and not

very expensive.  They are very robust since they operate in a higher

temp environment.  On the downside they are usually lower current, but

very easy to place in parallel.  The whole TC circuit seems to cry out

to have one end grounded.  This is not possible with neon/cold cathode

transformers because of the grounded midpoint. Take care.

Eric

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: Cap charging

Cc:

Bcc:

X-Attachments:

At 12:14 AM 10/1/97 GMT, you wrote:

>From: "Thomas McGahee" <tom_mcgahee@sigmais.com>

>Date: Tue, 30 Sep 1997 20:14:52 -0400

>Subject: Re: Cap charging

>

>

>> From: Dave <djb@erie.net>

>> Date: Tue, 30 Sep 1997 18:09:45 -0400

>> Subject: Re: Cap charging

>>

>> <snip>

>>

>> >Ok you say but we have our spark gap set to break down at 15kv

>> >so it should clamp the voltage at that value, right? Well yea.

>> >But the voltage is going to seek every opportunity to rise to

>> >its maximum value. Whats happens if your gap misfires? It

>> >happens. Even with static gaps. Voltage soars.

>> >

>> >This is why you need a properly set safety gap right at the

>> >neon itself. Its there to protect in case of a misfire or kickback.

>>

>> <snip>

>>

>> >TTYL!

>>

>> >Mike Hammer

>> >mhammer@misslink.net

>>

>> I am not sure what you mean about a gap misfiring Mike. Let's say the gap

>> was set to fire at aproximately 12 kv's. Under normal conditions it would

>> fire when the cap voltage reached 12 kv's, or if there were ions still

>> present in the gap, it would fire at less than 12 kv's. I cannot see the

>> insulating value of air changing significantly to cause the gap to fire

>> above 12 kv's if that's what the gap is set for...so how does a misfire

>> occur and voltage soar from this?

>>

>> If the gap is set to fire at 12 kv's, shouldn't it always fire at 12 kv's

>> unless there are unwanted ions present?

>>

>> Dave

>>

>

>Dave,

>Sorry, Mike, but Dave is right about the gap firing at 12KV. Now a

>rotary is a whole 'nother animal and asynchronous gaps can definitely

>fire over a WIDE range of voltages. Synchronous rotary gaps are

>better, because they are phase locked. But if the voltage were to cut

>out for a second or two and then come back on, then the gap might

>take a second or two to lock back in. So a synchronous gap CAN be a

>problem if it "slips" and looses sync for any reason. But a STATIC

>gap is fixed. ions can REDUCE the firing point, but not raise it

>higher. A blast of air might raise the firing voltage, though. Is

>that what you were thinking of, Mike?

<snip>

No. What I was actually thinking about was electrode oxidation.

After some hard use my gap will sometimes sputter and spurt a bit

until I clean up the electrode faces a bit.

I consider that to be misfires.

And I would think that RF kickbacks finding their way into the

main gap could do some unwanted things.

I agree that a properly set static gap fires at the same voltage

time after time. But what I was trying to say there there might be

times where misfires could happen and give some possible examples.

TTYL!

Mike Hammer

mhammer@misslink.net

From: tesla-2@emachine.com (Tesla-2)

Subject: MOV use in place of spark gap

Date: Wed,  1 Oct 1997 19:46:25 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Eric Davidson <edavidson@icva.gov>

Date: Wed, 01 Oct 1997 14:46:25 -0500

Subject: MOV use in place of spark gap

Hi all:

In light of the recent discussions about spark gaps I wondered about

using metal oxide varistors to act as the spark gap in a TC circuit.

There are MOVs that will take 2500 volts or so, and these could be

placed in series using resistors to equalize the voltage across them.

They can also absorb huge amounts of energy for short periods of time.

They are reasonably inexpensive also.  MOVs can turn on and off very

quickly and should produce a nice clean edge on transition as to produce

good HF noise.  If they would work I would think they be competitive

with a spark gap since there would be no hot plasma to quench.  I have

had to deal with HF noise in power supplies using very fast diodes so I

though I'd mention this idea. I am a novice as far as Tesla coils go so

if this has been thought of years ago, forgive me. If not, I'd be

interested in comments. Thanks.

Eric

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: Cap charging

Date: Wed,  1 Oct 1997 20:01:37 GMT

Organization: The Energy Machine Information System 504-733-3462

From: "Thomas McGahee" <tom_mcgahee@sigmais.com>

Date: Wed, 1 Oct 1997 16:01:37 -0400

Subject: Re: Cap charging

----------

> From: Tesla-2 <tesla-2@emachine.com>

> To: tom_mcgahee@sigmais.com

> Subject: Re: Cap charging

> Date: Wednesday, October 01, 1997 2:20 PM

>

> From: Mike Hammer <mhammer@misslink.net>

> Date: Wed, 01 Oct 1997 13:20:28 -0500

> Subject: Re: Cap charging

>

> At 12:14 AM 10/1/97 GMT, you wrote:

> >From: "Thomas McGahee" <tom_mcgahee@sigmais.com>

> >Date: Tue, 30 Sep 1997 20:14:52 -0400

> >Subject: Re: Cap charging

> >

> >

> >> From: Dave <djb@erie.net>

> >> Date: Tue, 30 Sep 1997 18:09:45 -0400

> >> Subject: Re: Cap charging

> >>

> >> <snip>

> >>

> >> >Ok you say but we have our spark gap set to break down at 15kv

> >> >so it should clamp the voltage at that value, right? Well yea.

> >> >But the voltage is going to seek every opportunity to rise to

> >> >its maximum value. Whats happens if your gap misfires? It

> >> >happens. Even with static gaps. Voltage soars.

> >> >

> >> >This is why you need a properly set safety gap right at the

> >> >neon itself. Its there to protect in case of a misfire or

> >kickback.

> >>

> >> <snip>

> >>

> >> >TTYL!

> >>

> >> >Mike Hammer

> >> >mhammer@misslink.net

> >>

> >> I am not sure what you mean about a gap misfiring Mike. Let's say the gap

> >> was set to fire at aproximately 12 kv's. Under normal conditions it would

> >> fire when the cap voltage reached 12 kv's, or if there were ions still

> >> present in the gap, it would fire at less than 12 kv's. I cannot see the

> >> insulating value of air changing significantly to cause the gap to fire

> >> above 12 kv's if that's what the gap is set for...so how does a misfire

> >> occur and voltage soar from this?

> >>

> >> If the gap is set to fire at 12 kv's, shouldn't it always fire at 12 kv's

> >> unless there are unwanted ions present?

> >>

> >> Dave

> >>

> >

> >Dave,

> >Sorry, Mike, but Dave is right about the gap firing at 12KV. Now a

> >rotary is a whole 'nother animal and asynchronous gaps can definitely

> >fire over a WIDE range of voltages. Synchronous rotary gaps are

> >better, because they are phase locked. But if the voltage were to cut

> >out for a second or two and then come back on, then the gap might

> >take a second or two to lock back in. So a synchronous gap CAN be a

> >problem if it "slips" and looses sync for any reason. But a STATIC

> >gap is fixed. ions can REDUCE the firing point, but not raise it

> >higher. A blast of air might raise the firing voltage, though. Is

> >that what you were thinking of, Mike?

>

> <snip>

>

>

> No. What I was actually thinking about was electrode oxidation.

> After some hard use my gap will sometimes sputter and spurt a bit

> until I clean up the electrode faces a bit.

> I consider that to be misfires.

>

Well, that is certainly true :)  I hadn't thought about oxidation per

se, but you are absolutely correct that during a really HOT session

there will be erratic operation if the gap pieces overheat. You

normally notice this AFTER you have turned the thing off and then try

to fire it up again. Cleaning the electrodes make it work right once

again. Oxidation is greatly enhanced by heat and moisture. There is

more than enough heat in the gap to cause rapid oxidation. I have

also had times when large chunks of a gap would get blasted off. Then

the electrodes would be pitted. Oh yes, BTW if the electrodes are not

clean and SMOOTH, then you can also get erratic action. A smooth

surface will fire much more consistently than a pitted or sharp

surface. So I stand corrected. (I can just see Mike grinning from ear

to ear on *this* one!)  Fr. Tom McGahee

> And I would think that RF kickbacks finding their way into the

> main gap could do some unwanted things.

>

True.

> I agree that a properly set static gap fires at the same voltage

> time after time. But what I was trying to say there there might be

> times where misfires could happen and give some possible examples.

>

>

> TTYL!

> Mike Hammer

> mhammer@misslink.net

>

BTW Mike, good job on ALL your recent postings!!

Fr. Tom McGahee

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Secdondary coil

Date: Sun,  5 Oct 1997 03:04:30 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Simon Buck <sbuck@cgo.wave.ca>

Date: Sat, 04 Oct 1997 23:04:30 -0400

Subject: Secdondary coil

Hello, I'm new to coiling and this list, but I have read a lot of the

archives. I just finished constructing my secondary coil, #24 magnet

wire on a 4.25" od  PVC drain pipe. I put 2 coats of polyurethane before

and wound about 44 turns per inch, and ran out of wire at 22", I figure

968 turns, teslac tells me this is too long. Will a conical spiral

primary provide efective coupling with this secondary? The transformer

is a 15kV 30 mA 450VA neon with only one side working (i got a sustained

arc of about 2.25" with a jacobs ladder), and I plan on using beer

bottle caps. Any suggesgtions for a spark gap? Advice is appreciated.

Simon Buck

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: Secondary coil

Cc:

Bcc:

X-Attachments:

At 03:04 AM 10/5/97 GMT, you wrote:

>From: Simon Buck <sbuck@cgo.wave.ca>

>Date: Sat, 04 Oct 1997 23:04:30 -0400

>Subject: Secdondary coil

>

>Hello, I'm new to coiling and this list, but I have read a lot of the

>archives. I just finished constructing my secondary coil, #24 magnet

>wire on a 4.25" od  PVC drain pipe. I put 2 coats of polyurethane before

>and wound about 44 turns per inch, and ran out of wire at 22", I figure

>968 turns, teslac tells me this is too long.

Welcome aboard Simon.

Teslac is a fair program but like many others has its limitations.

It will only calculate for smaller H/D ratios. Thats partly because

the algorithms used lose accuracy at higher ratios.

But that doesnt mean your secondary wont work. Quite the contrary.

Yours will be excellent. I wound a very similar secondary to

yours and it performs splendidly.

I wound 21 inches of AWG 24 on a prepared 4 inch PVC form.

I have succeeded in getting 48 inch streamers with 12kv/100 ma

worth of neons.

Did you dry your PVC form before sealing it with varnish?

Pop it in a warm (not hot) oven for 45 minutes or so to

drive off the moisture. Temperature shouldnt go much above

130 deg F or the PVC will soften on you. If your form wont

fit in the oven (or there are other obstacles) use a heat

gun or hair dryer. The idea is to get it nice and toasty

to drive off any moisture. Apply the varnish while the form

is still warm. 2 coats outside and one coat inside.

Drying and sealing PVC will gain you a couple extra inches

of spark for no cost.

> Will a conical spiral primary provide effective coupling with this secondary?

Yep. In fact it will likely overcouple some forcing you to raise

up the secondary a little away from the primary to reduce coupling.

It will be an effective combination.

Dont skimp on the number of primary turns. Go for at least 12

and 14 would be better. You may not need all of them right

away but you will.

>The transformer is a 15kV 30 mA 450VA neon with only one side working (i got a >sustained arc of about 2.25" with a jacobs ladder), and I plan on using beer

>bottle caps.

First thing is to start hunting for another neon. You can use your

crippled neon as a 7500/30ma neon. It will work fine and you

can expect about 10-12 inches of spark.

My advice is to get some 12kv neons and avoid 15kv ones.

The 15 kv neons are just too hard to build reliable caps for.

You will want about .01 mfd pf capacitance for your

7500/30 ma neon.

I am running two 12kv neons in parallel for more power right

now to give me 12kv/100ma. Got all the power I need.

Any suggesgtions for a spark gap? Advice is appreciated.

Use one of the so called RQ type cylinder gaps. This is really

one of the best gaps around for neon transformers. I just

recently discovered how good my RQ type gap is working on my coil

by looking at the secondary output waveform on an oscilloscope.

If you dont have the file on the cylinder gap check

out the funet.fi ftp server. I believe its in the misc

directory and the file name is sparkgap.zip

TTYL!

Mike Hammer

mhammer@misslink.net

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Source for magnet wire

Cc:

Bcc:

X-Attachments:

Hello all,

Several people have asked me recently where to get magnet

wire to wind secondary coils.

The best source is a local electric motor rebuild shop.

You will get the best pricing from there. Check the yellow

pages under electric motors.

If that fails and you have no other outlet then you can mail order wire

from Joe Cassata in Naperville Il. Lots of coilers have bought

wire from Joe and I've bought coax cable from him and met and talked to

him at hamfests in Illinois. He sells good quality wire.

I picked up a flyer from him at the Peoria Illinois hamfest a

couple weeks ago and thought I would post his current pricing

on magnet wire as well as his current address and phone number.

The Coax Connection

Joe Cassata

10 S 226 Meadow Lane

Naperville,Il 60564

630-420-0342

AWG    ft/lb    $/lb

14      79.7    5.10

16     127      5.30

18     201      5.50

20     315      5.85

22     516      6.05

24     802      7.20

26    1280      8.00

28    2027      8.75

30    3212      9.25

Other sizes available.

You can beat Joe's prices at a local motor repair shop but if you havnt got

one in your area this is a reputable place to do business with.

TTYL!

Mike Hammer

mhammer@misslink.net

From: tesla-2@emachine.com (Tesla-2)

Subject: First coil help needed.

Date: Tue,  7 Oct 1997 04:08:34 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Alex Parker <axparker@javanet.com>

Date: Tue, 07 Oct 1997 00:08:34 -0400

Subject: First coil help needed.

Greetings,

This is my first post on the Tesla-2 list, so if anyone has seen my

post before on the pupman.com list, you may disregard this message.

My name is Alex Parker, and I am a HS senior at Boston Latin School

in Boston, MA. I am looking to do a Tesla coil for my science project

this year, and I have already begun to plan it out. Below is my idea for

a coil. I don't know too much about building coils (except from what

I've read on mailing lists, web pages, etc), so I would expect my

initial design to be highly screwed up. Here's what I have planned so

far:

15 kv 60 mA neon sign transformer

SW capacitor made from beer bottles

Primary coil constructed of 14 turns of 1/4" copper tubing

24" Secondary of 4" sch 40 PVC pipe coated in varnish/polyeurathane

wound with #24 AWG magnet wire

3"x8" Toroid constructed of Aluminum dryer duct, with 2 pie plates back

to back in the center and coveredin aluminum tape.

The spark gap I have not figured out yet.

Ok, here's where I make my pathetic plea for help in hopes that someone

who knows what they're doing will respond.

First of all, I need people to tell me what is WRONG with my design, and

how I can improve it. I particularly need ideas on how long my secondary

should be, what type of primary to build, how to build a good spark gap,

and how to make a better cap than the SW ones (for cheap).

Second, I am badly in need of parts. I have managed to acquire the 15kv

neon, but I am still in search of just about everything else (capacitors

in particular). I will construct the SW caps if necessary, although I

prefer not to, as I have heard what they lack in cost they more than

make up for in inefficiency. So if anyone has any junk lying around that

they feel like sending me (transformers, coils, wire, really really need

good caps), I will gladly pay all shipping costs. I apologize for not

being able to pay for stuff, but my bank account seems mighty light

these days...

Ok, that's all for me. Thanks for listening.

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: Source for magnet wire

Date: Tue,  7 Oct 1997 05:43:34 GMT

Organization: The Energy Machine Information System 504-733-3462

From: randy@gte.net

Date: Mon, 06 Oct 1997 22:43:34 -0700

Subject: Re: Source for magnet wire

Tesla-2 wrote:

>

> From: Mike Hammer <mhammer@misslink.net>

> Date: Mon, 06 Oct 1997 11:39:00 -0500

> Subject: Source for magnet wire

>

> Hello all,

>

> Several people have asked me recently where to get magnet

> wire to wind secondary coils.

> The best source is a local electric motor rebuild shop.

> You will get the best pricing from there. Check the yellow

> pages under electric motors.

>

> If that fails and you have no other outlet then you can mail order wire

> from Joe Cassata in Naperville Il. Lots of coilers have bought

> wire from Joe and I've bought coax cable from him and met and talked to

> him at hamfests in Illinois. He sells good quality wire.

>

> I picked up a flyer from him at the Peoria Illinois hamfest a

> couple weeks ago and thought I would post his current pricing

> on magnet wire as well as his current address and phone number.

>

> The Coax Connection

> Joe Cassata

> 10 S 226 Meadow Lane

> Naperville,Il 60564

> 630-420-0342

>

> AWG    ft/lb    $/lb

> 14      79.7    5.10

> 16     127      5.30

> 18     201      5.50

> 20     315      5.85

> 22     516      6.05

> 24     802      7.20

> 26    1280      8.00

> 28    2027      8.75

> 30    3212      9.25

>

> Other sizes available.

>

> You can beat Joe's prices at a local motor repair shop but if you havnt got

> one in your area this is a reputable place to do business with.

>

> TTYL!

> Mike Hammer

> mhammer@misslink.net

>

Too late right now to try to call  him to verify whether this is still

true, but The Wireman at 1 800 WIRE MAN was the favored hamfest vendor

at the huge Orlando 'fest. Coax, copperweld, you name it, he had it.

Randy

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: First coil help needed.

Cc:

Bcc:

X-Attachments:

At 04:08 AM 10/7/97 GMT, you wrote:

>From: Alex Parker <axparker@javanet.com>

>Date: Tue, 07 Oct 1997 00:08:34 -0400

>Subject: First coil help needed.

>

>Greetings,

>

>    This is my first post on the Tesla-2 list, so if anyone has seen my

>post before on the pupman.com list, you may disregard this message.

Welcome aboard Alex.

>

>    My name is Alex Parker, and I am a HS senior at Boston Latin School

>in Boston, MA. I am looking to do a Tesla coil for my science project

>this year, and I have already begun to plan it out. Below is my idea for

>a coil. I don't know too much about building coils (except from what

>I've read on mailing lists, web pages, etc), so I would expect my

>initial design to be highly screwed up. Here's what I have planned so

>far:

>

>15 kv 60 mA neon sign transformer

This is fine but quite a bit of power for a first

coil. If this is what you got then use it. If you

have a choice of transformers or can get another transformer

consider getting a 12kv/30ma neon to start. The lower power

will be a little easier to handle at first.

>SW capacitor made from beer bottles

This will work and get you up and running. You will

get about 700 pf from each 12 oz bottle. You will want

about .01 mfd for your 15kv/60ma neon so that will

take 12-14 bottles. I would start with 8 bottles

and get things working. Then add one bottle at a time

and retune for best spark. This way you can find just

the right amount of capacitance for best performance.

>Primary coil constructed of 14 turns of 1/4" copper tubing

This will be fine. Best results will come from a saucer shaped

inverted conical primary with a 15-30 degree slope.

>24" Secondary of 4" sch 40 PVC pipe coated in varnish/polyeurathane

>wound with #24 AWG magnet wire

This is a good size for a secondary and is capable of high performance.

A 24 inch long winding of AWG 24 is a bit much. Well over 1000 turns.

Consider scaling back to a 21 inch long winding. This is close to 975

turns and performs quite well. I just built a secondary like that and

am getting 48 inches of spark from it.

Be sure to dry/bake your PVC pipe first. 45 minutes in the oven at not

more than 130 deg F will do the job. Or use a heat gun or hair dryer.

Keep it nice and warm for 45 minutes to an hour. Then varnish the form

with real polyurethane varnish. No latex based varnishes allowed.

Put two coats on the outside of the form and one coat inside. Apply

the varnish while the form is still warm. This will gain you a few

inches of spark at practically no cost by driving the moisture out of

the PVC.

>3"x8" Toroid constructed of Aluminum dryer duct, with 2 pie plates back

>to back in the center and coveredin aluminum tape.

Think bigger. With your 3 inch duct fashion it into a 15 or 16

inch diameter ring. You will eventually want a bigger toroid than this.

4 inch ducting makes a good 16 or 17 X 4 toroid. But at first you

want a small toroid. Or even a spherical terminal. A copper toilet

tank float makes a good first terminal. I used a 6 inch diameter

styrofoam ball covered with foil to good effect.

>The spark gap I have not figured out yet.

For neons there is no better gap than the cylinder gap popularized

(not invented) by Richard Quick.

Check out the funet.fi archive for the file sparkgap.zip

The address is:

ftp.funet.fi/pub/sci/electrical/tesla/misc

>

>Ok, here's where I make my pathetic plea for help in hopes that someone

>who knows what they're doing will respond.

>

>First of all, I need people to tell me what is WRONG with my design, and

>how I can improve it. I particularly need ideas on how long my secondary

>should be, what type of primary to build, how to build a good spark gap,

>and how to make a better cap than the SW ones (for cheap).

Nothing really wrong with your design. Just a few tweaks here and

there for improvements.

Building a successful coil is a learning experience. You need

to get that first coil going and learn by your mistakes.

Quality pulse discharge caps are not cheap.

They require lots of effort to build. Go ahead and build and use

your SW caps for now. Get your coil sparking. Once you have

things running well then think about building some polyethylene

film caps.

>

>Second, I am badly in need of parts. I have managed to acquire the 15kv

>neon, but I am still in search of just about everything else (capacitors

>in particular). I will construct the SW caps if necessary, although I

>prefer not to, as I have heard what they lack in cost they more than

>make up for in inefficiency. So if anyone has any junk lying around that

>they feel like sending me (transformers, coils, wire, really really need

>good caps), I will gladly pay all shipping costs. I apologize for not

>being able to pay for stuff, but my bank account seems mighty light

>these days...

Well the great thing about Tesla coiling is it can be done easily

with a "light" bank account. Other peoples junk is the best source

of parts. Dumpster diving is the preferred method of parts procurement.

Good luck on your project.

TTYL!

Mike Hammer

mhammer@misslink.net

From: tesla-2@emachine.com (Tesla-2)

Subject: New coiler

Date: Tue,  7 Oct 1997 17:12:37 GMT

Organization: The Energy Machine Information System 504-733-3462

From: aasphalt@win.bright.net (Lynn Anderson)

Date: Tue, 7 Oct 1997 12:12:37 -0500 (CDT)

Subject: New coiler

I read with interest the reply to Alex Parker. This is very much like what I

am planning to build. However I had planned to use an air blast gap. It

seems easier to build and I have an abundant supply of compressed air.

Comments? I also had planned to buy a cap. How much should I expect to

spend? I have 2 15-30 neons which I have baked twice and no luck. More work

to do here. Thinking of building a coil winder with a variable speed motor

from a sewing machine. Thoughts?  Thanks.

Get your "Tars and Stripes" from Anderson Asphalt!

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: New coiler

Cc:

Bcc:

X-Attachments:

At 05:12 PM 10/7/97 GMT, you wrote:

>From: aasphalt@win.bright.net (Lynn Anderson)

>Date: Tue, 7 Oct 1997 12:12:37 -0500 (CDT)

>Subject: New coiler

>

>I read with interest the reply to Alex Parker. This is very much like what I

>am planning to build. However I had planned to use an air blast gap. It

>seems easier to build and I have an abundant supply of compressed air.

>Comments?

Nothing wrong with that approach. Air blast gaps can and do

work well. The down side is that they are noisy and need that

compressed air. For a small coil running less than 500-600 watts it

may be overkill. Often a simple multiple gap arrangement with a

little forced air from a blower or fan will suffice.

>I also had planned to buy a cap. How much should I expect to

>spend?

Dig deep. Pulse rated caps from commercial manufacturers cost

from $200 up. You can sometimes find surplus mica caps and

they work well. Typical ceramic doorknob caps will work but

overheat and self destruct easily.

For these reasons most of us decide to build our own caps.

I have never regretted the effort put into my first polyethylene

plate cap. It worked great and continues to do so.

>I have 2 15-30 neons which I have baked twice and no luck.

The baking of neons can restore some to operation that suffer

from carbon tracks in the potting compound. That is a common

failure but not the only way neons fail. I've seen plenty

of dead neons with secondary wires broken off terminals

and the midpoint connections broken off the core. The only

way to fix these problems is to unpot them. Its nasty dirty

work but the results are worth the effort.

> Thinking of building a coil winder with a variable speed motor

>from a sewing machine. Thoughts?

Probably not worth the effort for a single coil. If you were

making several then maybe it would be worthwhile.

I recently wound a 4 inch X 21 inch secondary with AWG 24

in less than an hour using a hand cranked winder cobbled

together from wood scraps and a broom handle.

TTYL!

Mike Hammer

mhammer@misslink.net

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: New coiler

Date: Tue,  7 Oct 1997 23:10:52 GMT

Organization: The Energy Machine Information System 504-733-3462

From: "Thomas McGahee" <tom_mcgahee@sigmais.com>

Date: Tue, 7 Oct 1997 19:10:52 -0400

Subject: Re: New coiler

----------

> From: Tesla-2 <tesla-2@emachine.com>

> To: tom_mcgahee@sigmais.com

> Subject: New coiler

> Date: Tuesday, October 07, 1997 1:12 PM

>

> From: aasphalt@win.bright.net (Lynn Anderson)

> Date: Tue, 7 Oct 1997 12:12:37 -0500 (CDT)

> Subject: New coiler

>

> I read with interest the reply to Alex Parker. This is very much like what

> I am planning to build. However I had planned to use an air blast gap.

> It seems easier to build and I have an abundant supply of compressed air.

> Comments? I also had planned to buy a cap. How much should I expect to

> spend? I have 2 15-30 neons which I have baked twice and no luck.

> More work to do here. Thinking of building a coil winder with a variable

> speed motor from a sewing machine. Thoughts?  Thanks.

>

>            Get your "Tars and Stripes" from Anderson Asphalt!

>

Air blast gaps can be quite good, but also be aware that too much air

blast can be bad. Make sure you can adjust the air flow. Also be

aware that compressed air lines can have a lot of moisture in them,

and thus the suckers will sometimes conduct a bit on you. Just be

aware of it, and act accordingly.

Commercial caps run anywhere from about $200 up, depending on the

rating and where you buy them. Getting used caps is not easy, but

sometimes you can find a good buy at a hamfest. If they are DC rated,

then de-rate them by at least a factor of 4 as regards the voltage

rating when used in Tesla service.

Hmmm. You could open up the bad neons and use the tar at least!

Actually, you might want to try your hand at rebuilding them. It

isn't all that hard. With two neons you should at least be able to

cannabalize the set and end up with one good unit to show for your

work. But visit the local sign shops and beg for some old 12K units,

too.

A coil winder can save a lot of time, as long as you don't spend more

time building the winder. They are especially useful when handling

the larger diameter coils. Go for it!

Fr. Tom McGahee

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: My first Tesla sparks

Date: Wed,  8 Oct 1997 20:51:56 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Simon Buck <sbuck@cgo.wave.ca>

Date: Wed, 08 Oct 1997 16:51:56 -0400

Subject: My first Tesla sparks

Well, I wound my primary and built some SW caps, and got my coil

sparking nicely with my "crippled" neon. At exactly 3:37:47 I got daring

and held a flourescent tube in the streamers. How do I know the exact

time? I was wearing my digital watch and now it permanently displays

that time. I would recommend that anyone interacting with the spark take

off your watch first. I wish I knew that before hand, oh well, live and

learn I guess Next step is to build a better gap and invest in a 12 kV

neon, oh yeah, and buy another watch. :)

TTYL and safe coiling.

Simon Buck <sbuck@cgo.wave.ca>

P.S. Anyone know how to revive a "frozen" watch?

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: My first Tesla sparks

Date: Wed,  8 Oct 1997 22:51:12 GMT

Organization: The Energy Machine Information System 504-733-3462

From: "Thomas McGahee" <tom_mcgahee@sigmais.com>

Date: Wed, 8 Oct 1997 18:51:12 -0400

Subject: Re: My first Tesla sparks

----------

> From: Tesla-2 <tesla-2@emachine.com>

> To: tom_mcgahee@sigmais.com

> Subject: My first Tesla sparks

> Date: Wednesday, October 08, 1997 4:51 PM

>

> From: Simon Buck <sbuck@cgo.wave.ca>

> Date: Wed, 08 Oct 1997 16:51:56 -0400

> Subject: My first Tesla sparks

>

> Well, I wound my primary and built some SW caps, and got my coil

> sparking nicely with my "crippled" neon. At exactly 3:37:47 I got daring

> and held a flourescent tube in the streamers. How do I know the exact

> time? I was wearing my digital watch and now it permanently displays

> that time. I would recommend that anyone interacting with the spark take

> off your watch first. I wish I knew that before hand, oh well, live and

> learn I guess Next step is to build a better gap and invest in a 12kV

> neon, oh yeah, and buy another watch. :)

>

> TTYL and safe coiling.

> Simon Buck <sbuck@cgo.wave.ca>

>

> P.S. Anyone know how to revive a "frozen" watch?

>

Simon,

Try removing the battery and then putting it back in. If there is any

chance that it will "revive", that should do it. We Tesla Coilers

only use Tesla arcs to revive dead tissue. It does not work on

inanimate objects, just inanimate bodies :)

Fr. Tom McGahee

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: My first Tesla sparks

Cc:

Bcc:

X-Attachments:

At 08:51 PM 10/8/97 GMT, you wrote:

>From: Simon Buck <sbuck@cgo.wave.ca>

>Date: Wed, 08 Oct 1997 16:51:56 -0400

>Subject: My first Tesla sparks

>

>Well, I wound my primary and built some SW caps, and got my coil

>sparking nicely with my "crippled" neon. At exactly 3:37:47 I got daring

>and held a flourescent tube in the streamers. How do I know the exact

>time? I was wearing my digital watch and now it permanently displays

>that time. I would recommend that anyone interacting with the spark take

>off your watch first. I wish I knew that before hand, oh well, live and

>learn I guess Next step is to build a better gap and invest in a 12 kV

>neon, oh yeah, and buy another watch. :)

Congratulations Simon. First sparks are always great.

How much output are you getting from your coil?

Once you start getting some spark improvements will

come fast. Better caps , better gap , larger discharge terminal

are all things you should look into.

>

>TTYL and safe coiling.

>Simon Buck <sbuck@cgo.wave.ca>

>

>P.S. Anyone know how to revive a "frozen" watch?

Well as Ive said many times modern electronics

and Tesla coils do not mix. When I went looking for

an oscilloscope to use with my coil I chose a 30 year

old + all tube Tektronix scope. No solid state modern marvels

anywhere near my Tesla coil.

Hmm.. Maybe another jolt will get your watching going again.

Naw. :)

Perhaps removing the battery and powering it up again

will restart it. If not then its fried.

TTYL!

Mike Hammer

mhammer@misslink.net

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: My first Tesla sparks

Date: Thu,  9 Oct 1997 01:40:56 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Reed E Hanson <rehanson@sunflowr.usd.edu>

Date: Wed, 8 Oct 1997 20:40:56 -0500 (CDT)

Subject: Re: My first Tesla sparks

On Wed, 8 Oct 1997, Tesla-2 wrote:

> From: Simon Buck <sbuck@cgo.wave.ca>

> Date: Wed, 08 Oct 1997 16:51:56 -0400

> Subject: My first Tesla sparks

>

> Well, I wound my primary and built some SW caps, and got my coil

> sparking nicely with my "crippled" neon. At exactly 3:37:47 I got daring

> and held a flourescent tube in the streamers. How do I know the exact

> time? I was wearing my digital watch and now it permanently displays

> that time. I would recommend that anyone interacting with the spark take

> off your watch first. I wish I knew that before hand, oh well, live and

> learn I guess Next step is to build a better gap and invest in a 12 kV

> neon, oh yeah, and buy another watch. :)

>

> TTYL and safe coiling.

> Simon Buck <sbuck@cgo.wave.ca>

>

> P.S. Anyone know how to revive a "frozen" watch?

>

--------------------------------------------------------------

High frequency electromagnetic fields do have their interesting effects,

as you found! As I was testing a new toroid on my small 250 kv system,

(I also have a 500 kv system being modified into a magnifying

transmitter), the channels on my tv/vcr just started to roll on their

own! Thusly, my computer is in another part of my house! As for the

watch, try taking the battery out, and reinstalling with the polarity

reversed for a second or two. Might not work, but "it can't hoit".

Have fun!

Reed Hanson

rehanson@sundance.usd.edu>

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: My first Tesla sparks

Date: Thu,  9 Oct 1997 07:11:40 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Simon Buck <sbuck@cgo.wave.ca>

Date: Thu, 09 Oct 1997 03:11:40 -0400

Subject: Re: My first Tesla sparks

You wrote:

> From: Mike Hammer <mhammer@misslink.net>

> Date: Wed, 08 Oct 1997 18:52:23 -0500

> Subject: Re: My first Tesla sparks

>

> At 08:51 PM 10/8/97 GMT, you wrote:

>

<my original text snipped>

> Congratulations Simon. First sparks are always great.

> How much output are you getting from your coil?

>

About 4-5 inches after a bit of fiddling. I'm using a mini-basketball wrapped

in foil for the discharge terminal, nine beer bottle caps, and L brackets and

bolts for the gap. All tied together with 12 AWG solid copper wire. Just

today I picked up some 4 and 6 AWG welding cable from a silver recovery unit,

that was in the trash at work. I plan to upgrade this weekend. I can also get

6 powerstat variacs 120V 50/60 Hz in 0-140V out 10A 1.4 kva. If anyone is

interested, e-mail and I will find out how much the shipping would be.

> Once you start getting some spark improvements will

> come fast. Better caps , better gap , larger discharge terminal

> are all things you should look into.

>

... Not to mention RF chokes. Is the "RQ style" gap much quieter than my

set-up because I can barely see the sparks in the day and my next-door

neighbour is a light sleeper/dickhead. :) I'm going to make some more SW caps

to get up to the optimum capacitance before building messy plate caps. I'm

going out tommorow to pick up some aluminum dryer duct for a classier-looking, higher performance toroid. I think I may have to buy a new transformer, because none of the sign shops in town want to hand over used transformers, and the guy that gave me the crippled one wants it back when the "science fair" is over ehehe). I generally don't like lying to people, but honesty didn't work. :)

> >

> >TTYL and safe coiling.

> >Simon Buck <sbuck@cgo.wave.ca>

> >

> >P.S. Anyone know how to revive a "frozen" watch?

>

> Well as Ive said many times modern electronics

> and Tesla coils do not mix. When I went looking for

> an oscilloscope to use with my coil I chose a 30 year

> old + all tube Tektronix scope. No solid state modern marvels

> anywhere near my Tesla coil.

>

> Hmm.. Maybe another jolt will get your watching going again.

> Naw. :)

>

> Perhaps removing the battery and powering it up again

> will restart it. If not then its fried.

>

> TTYL!

> Mike Hammer

> mhammer@misslink.net

>

I showed everyone at work, then I tried throwing it at the concrete floor and

stomping on it ... That didn't work, so I smacked it against a metal pole

about 6 times, and that did it :) I'm considering writing a letter to TIMEX,

they really do take a lickin' and keep on tickin'.

TTYL and happy coiling.

Simon Buck <sbuck@cgo.wave.ca>

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: My first Tesla sparks

Cc:

Bcc:

X-Attachments:

At 07:11 AM 10/9/97 GMT, you wrote:

>From: Simon Buck <sbuck@cgo.wave.ca>

>Date: Thu, 09 Oct 1997 03:11:40 -0400

>Subject: Re: My first Tesla sparks

>

>

>

>You wrote:

>

>> Congratulations Simon. First sparks are always great.

>> How much output are you getting from your coil?

>>

>

>About 4-5 inches after a bit of fiddling. I'm using a mini-basketball wrapped

>in foil for the discharge terminal, nine beer bottle caps, and L brackets and

>bolts for the gap. All tied together with 12 AWG solid copper wire. Just

>today I picked up some 4 and 6 AWG welding cable from a silver recovery unit,

>that was in the trash at work. I plan to upgrade this weekend.

5 inches isnt bad to start especially with 7500 volts.

A little more capacitance will likely help.

The welding cable will probably not be necessary at this point.

Your AWG 12 wiring is probably adequate. Your peak power

levels are low enough that your wiring isnt a problem.

Hang on to that cable though.

>

>... Not to mention RF chokes.

RF chokes are important to keep your neons healthy. More

important though is a safety gap across the transformer.

Set this gap close enough that it is firing occasionally.

If its not firing its not close enough to limit voltage peaks.

> Is the "RQ style" gap much quieter than my

>set-up because I can barely see the sparks in the day and my next-door

>neighbour is a light sleeper/dickhead. :)

Well my friend Tesla coiling is not a quiet hobby. The more

power you run and the more spark you get the louder it will get.

Pretty soon the output streamers will be louder than the gap.

I wear hearing protection whenever I run my coil.

If you cant run at night because of your neighbor then

run during the day and cover the windows to darken the room.

>I'm going to make some more SW caps

>to get up to the optimum capacitance before building messy plate caps.

Caps messy? Naw. Think of it as an adventure.

>I'm going out tommorow to pick up some aluminum dryer duct for a

>classier-looking, higher performance toroid.

You will probably be disappointed in a large toroidal terminal

at this point. You are probably not yet producing enough power for

a decent toroid.

I would work on caps and gap first. Also adjust the spacing

between the primary and secondary to adjust the coupling.

Often a 1/2 inch change here can make a big difference

in output. Generally you want the secondary as far down

into the primary (tighter coupling) as you can get it

without having pri/sec arcing.

>I think I may have to buy a new

>transformer, because none of the sign shops in town want to hand over used

>transformers, and the guy that gave me the crippled one wants it back when

>the "science fair" is over (hehehe). I generally don't like lying to people,

>but honesty didn't work. :)

They wouldnt even sell you a used transformer? Thats pretty unusual.

Try a couple places and ask for a transformer to light a sign.

They may sell you a transformer for a sign when they wouldnt

consider selling one for a experimental project.

TTYL!

Mike Hammer

mhammer@misslink.net

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: my web page

Cc:

Bcc:

X-Attachments:

Hello All,

Well I finally got around to putting up a web page.

Can anybody guess what the subject matter of my page might be?

Anybody that wants to check it out can find it at the following URL:

http://www.misslink.net/mhammer

Its fairly simple at this point but will contain

more pics and information as my experience with HTML

and my initiative allow.

I would appreciate any and all feedback.

TTYL!

Mike Hammer

mhammer@misslink.net

From: tesla-2@emachine.com (Tesla-2)

Subject: Ready for winding!

Date: Tue, 14 Oct 1997 15:49:14 GMT

Organization: The Energy Machine Information System 504-733-3462

From: "Eriksson" <jan-erik.eriksson@mbox2.swipnet.se>

Date: Tue, 14 Oct 1997 17:49:14 +0200

Subject: Ready for winding!

Hi

I'm about to begin winding my first coil, but I have some questions

that I hope u can help me with. First, my coil specs calculated with

WinTesla.

Primary:

inside diam. 9"

wire diam. 0.236"

turns. 8

space between turns. 0.2"

Secondary

diam. 4.33" (110 mm PVC pipe, dryed, acrylic varnished)

Length. 24"

guage. 24 AWG

wire length. 1247.29ft

turns. not calculated this but about 1100 (!?)

So, the first question, how should I wind my primary, I've seen

many types of windings, should I wind it as a "disc" shaped (flat),

or a upside down turned cone, or just like the secondary, like a

cylinder??

The other question, I've been told that I shouldnt cap the ends of the

PVC pipe. but I've seen so many coils that are capped with some PVC sheet

or something.

But if I don't cap it, how do I then attach the toroid to the top?

Also, I called some neon sign shops here in Sweden, in Bors where I live,

and they got some used transformers. A 12kV 30mA (not sure about that) for 200 Swedish crowns, about $26 and I've also found some oil burner trannys,

10kV (I don't know how much power) for about 2-3$ each, but which should I buy, some oil burner trannys or a neon sign tranny?

I appreciate any help with this.

My english might be a little bit crappy, but I hope you'll understand.

//Christopher Eriksson

Any Swedish coilers in this list?

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: Ready for winding!

Cc:

Bcc:

X-Attachments:

At 03:49 PM 10/14/97 GMT, you wrote:

>From: "Eriksson" <jan-erik.eriksson@mbox2.swipnet.se>

>Date: Tue, 14 Oct 1997 17:49:14 +0200

>Subject: Ready for winding!

>

> Hi

>I'm about to begin winding my first coil, but I have some questions

>that I hope u can help me with. First, my coil specs calculated with

>WinTesla.

>Primary:

>inside diam. 9"

>wire diam. 0.236"

>turns. 8

>space between turns. 0.2"

>

>Secondary

>diam. 4.33" (110 mm PVC pipe, dryed, acrylic varnished)

>Length. 24"

>guage. 24 AWG

>wire length. 1247.29ft

>turns. not calculated this but about 1100 (!?)

Thats a bit too long. Go for a 21 inch long winding of

AWG 24. Thats right at 975 turns. I know that design works

as thats what I used on my latest secondary. And Im getting

50 inches of spark at 1200 watts.

>

>So, the first question, how should I wind my primary, I've seen

>many types of windings, should i wind it as a "disc" shaped (flat),

>or a upside down turned cone , or just like the secondary, like a

>cylinder??

Well my recommendation is to build an inverted conical (saucer)

primary with a slope of between 15 and 30 degrees. The flat spiral

would work but will likely undercouple. A solenoid (cylinder)

primary will also work but will overcouple.

>The other question, I've been told that i shouldnt cap the ends of the

>PVC pipe. but I,ve seen so many coils that is caped with some PVC sheet

> or something.

I epoxy a 1/8 to 3/16 inch thick disk of plexiglass or Lexan down

inside the secondary just above the secondary winding. I then

fit a disk of dry wood in the top of the secondary above the plexiglass

disk. This wood disk has a 1/4 20 bolt mounted in it and countersunk.

I bring the secondary wire up the OUTSIDE of the form up to this

bolt and attach it with a ring terminal.

Instead of the wood disk you could mount a bolt in a PVC pipe end

cap and fit the over the end of the secondary. Do not neglect

the plexiglass or lexan disk mounted inside the secondary. This

is to provide electrical strength to prevent arcing down the inside

of the secondary form.

I attach my discharge electrode to this 1/4 20 bolt.

>Also, I called some neon sign shops here in Sweden, in Bors where I live,

>and they got some used transformers. A 12kV 30mA (not sure about that) for 200 >Swedish crowns, about $26 and I've also found some oil burner trannys,

>10kV (I don't know how much power) for about 2-3$ each, but which should I

>buy, some oil burner trannys or a neon sign tranny?

Most oil burner transformers (OBIT) are 10KV/23ma. They work well.

I would get a couple OBITs since they are pretty cheap. Two

of them in parallel will get you 10kv/46ma. Thats respectable

power. Move up to 12kv neons after you get your coil running well.

>I appreciate any help with this.

>My english might be a little bit crappy, but I hope you'll understand.

>

Hey your English is just fine. Much better than me

trying to speak your language. :)

TTYL!

Mike Hammer

mhammer@misslink.net

From: tesla-2@emachine.com (Tesla-2)

Subject: pig poles

Date: Wed, 15 Oct 1997 06:31:12 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Dsurfr@aol.com

Date: Wed, 15 Oct 1997 02:31:12 -0400 (EDT)

Subject: pig poles

I have noticed a lot of discussion on another Tesla site about "pig poles"

but have no idea of what they are referring to. It seems to be an important

safety feature that I am overlooking and would appreciate any advice on this

or any other safety devices that I should be incorporating in my new coil.

A.J.S.

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: ready for winding

Date: Wed, 15 Oct 1997 06:41:53 GMT

Organization: The Energy Machine Information System 504-733-3462

From: thorn@northlink.com

Date: Tue, 14 Oct 1997 23:41:53 -0700 (MST)

Subject: Re: ready for winding

Hello Mike,

About what was said about transformers.  I went to the local junk yard

today to find a Neon trans.  I found several 15K, and only one 12K.  I don't

even know if they are good or not, and they want to charge me $45.  Is this

a good price for something that could be functionaly questionable?  If not,

how can I determine if it is good or not?  I suppose that I could take along

a meter to measure the windings resistances, but without knowing what

resistance is considered "good" or not, how will I know?  An infinite

resistance I am sure would be due to an "open" existing in the coil.  Does

EMF durring testing alter the meters reading at all?  The meter will induce

a voltage, so does that voltage produce an inhibiting magnetic field over

the coil, thus reducing the meters actual reading?  I don't want to pay $45

for a melted coil, so I will need to know for sure if it works.  Any

suggestions?

Something that I have not heard much spoken about, is spark gaps.  Do you

recommend using stationary/adjustable gaps, or rotory?  The motors that run

spark gaps, is there some reason that rather large motors are used?  Does

the act of the gaping conductor passing between the electrodes cause some

kind of physical resistance?  One would figure that with sufficient

bearings, the rotor should be rather easy to bring to high speeds with

little torque required.  Is excessive vibration a problem, requiring tight

tolerances and balancing?  I can easily imagine the gap changing due to a

small error in the rotor/shaft assembly.  Is this a problem?  I was thinking

of utilizing a plastic turntable from an old record player as the rotor, and

spin it horizontaly. For the gaping conductors, I was thinking of having

twelve.  Each conductor would be a short piece of threaded rod screwed into

a hole that has been tapped for its thread size.  Two brass cap nuts would

then be screwed onto either end of the rod protruding on each side of the

rotor.  I have seen something similar to this on another gap, and they used

tig welding electrodes in theirs as the primary electrodes.  Is this ok to

duplicate?  Would carbon rods from "brush" motors work well?

Thanks,

RMD

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Coil specs

Date: Wed, 15 Oct 1997 07:59:15 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Simon Buck <sbuck@cgo.wave.ca>

Date: Wed, 15 Oct 1997 03:59:15 -0400

Subject: Coil specs

Here is the scoop on my coil. Comments and advice is welcome

My power source is a 15 kV 30 mA neon xfmr both sides now work (Woohoo),

for a gap I have L brackets and bolts. The gap is being upgraded soon. I

don't have a safety gap or RF chokes yet either, what size toroid should

I get and what size wire and how many turns do I need?

I used saltwater and beer bottle caps. I found hamster salt wheels work

well, they were not iodized and were 75 cents each, so it was in my

budget. I used one wheel for every 4.5 bottles. I now have 12 paralleled

for a total of 7 nf, but plan to make some poly plate caps in the next

few months.

I was anxious to get started, so I opted for the flat spiral primary.

For this I just stapled the wire to the board at the appropriate marks,

it was the easiest part. It's almost 50' of thick ground wire spaced

about 1/4" for a total of 15 turns.

The secondary is 21.5" of #24 magnet wire on 4" sealed PVC pipe. The

toroid is a 4x13" aluminum dryer duct. The ground is a copper pipe about

4' underground, with grounding wire coming in through a window, up to a

pipe. The pipe continues through the rafters into another room down some

#12 solid copper wire to the bottom of the secondary.

After some tuning I now get almost 8" sparks.

My wish list in order of priority

bigger sparks (hehe)

RF chokes

more capacitance

new gap

wire in the rafters instead of the pipe

more power

safety gap

new primary

depending on the power level, maybe a bigger toroid

The safety gap is not important until I get a new xfmr, as the guy who

gave me this one wants it back when the "science fair" is over.

Simon Buck <sbuck@cgo.wave.ca>

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: pig poles

Date: Wed, 15 Oct 1997 10:51:36 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Dave <djb@erie.net>

Date: Wed, 15 Oct 1997 06:51:36 -0400

Subject: Re: pig poles

At 06:31 AM 10/15/97 GMT, you wrote:

>From: Dsurfr@aol.com

>Date: Wed, 15 Oct 1997 02:31:12 -0400 (EDT)

>Subject: pig poles

>

>I have noticed a lot of discussion on another Tesla site about "pig poles"

>but have no idea of what they are referring to. It seems to be an important

>safety feature that I am overlooking and would appreciate any advice on this

>or any other safety devices that I should be incorporating in my new coil.

>A.J.S.

HI AJS,

The correct term you are referring to is a 'pole pig'. What it is is a

utility power distribution transformer. These are normally found on street

poles and step down down the high line voltage from the power company to a

voltage that you can use in your home, etc. I believe in Tesla coil use,

they are run backwards as a step up transformer with some form of current

limiting. This allows coilers to be able to use more power than what would

be normally available with the use of neon sign transformers, etc. As a new

coiler, you shouldn't be concerned with them. Even alot of coilers with much

more experience are not using pole pigs. Neon sign transformers, etc should

provide you with years of experience. Hope this helps.

Good luck,

Dave

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: ready for winding

Date: Wed, 15 Oct 1997 11:16:44 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Dave <djb@erie.net>

Date: Wed, 15 Oct 1997 07:16:44 -0400

Subject: Re: ready for winding

At 06:41 AM 10/15/97 GMT, you wrote:

>From: thorn@northlink.com

>Date: Tue, 14 Oct 1997 23:41:53 -0700 (MST)

>Subject: Re: ready for winding

>

>Hello Mike,

>

>   About what was said about transformers.  I went to the local junk yard

>today to find a Neon trans.  I found several 15K, and only one 12K.  I don't

>even know if they are good or not, and they want to charge me $45.  Is this

>a good price for something that could be functionaly questionable?  If not,

>how can I determine if it is good or not?  I suppose that I could take along

>a meter to measure the windings resistances, but without knowing what

>resistance is considered "good" or not, how will I know?  An infinite

>resistance I am sure would be due to an "open" existing in the coil.  Does

>EMF durring testing alter the meters reading at all?  The meter will induce

>a voltage, so does that voltage produce an inhibiting magnetic field over

>the coil, thus reducing the meters actual reading?  I don't want to pay $45

>for a melted coil, so I will need to know for sure if it works.  Any

>suggestions?

>

>   Something that I have not heard much spoken about, is spark gaps.  Do you

>recomend using stationary/adjustable gaps, or rotory?  The motors that run

>spark gaps, is there some reason that rather large motors are used?  Does

>the act of the gaping conductor passing between the electrodes cause some

>kind of physical resistance?  One would figure that with sufficient

>bearings, the rotor should be rather easy to bring to high speeds with

>little torque required.  Is excessive vibration a problem, requiring tight

>tolerances and balancing?  I can easily imagine the gap changing due to a

>small error in the rotor/shaft assembly.  Is this a problem?  I was thinking

>of utilizing a plastic turntable from an old record player as the rotor, and

>spin it horizontaly. For the gaping conductors, I was thinking of having

>twelve.  Each conductor would be a short piece of threaded rod screwed into

>a hole that has been tapped for its thread size.  Two brass cap nuts would

>then be screwed onto either end of the rod protruding on each side of the

>rotor.  I have seen something similar to this on another gap, and they used

>tig welding electrodes in theirs as the primary electrodes.  Is this ok to

>duplicate?  Would carbon rods from "brush" motors work well?

>

>Thanks,

>

>RMD

Hi RMD,

I would not pay $45 for one transformer, but if that is the only place in

the area to get one, then I might be persuaded to buy it having no other

alternative. I don't think measuring the winding resistance of the neon

secondary will tell you much. I believe it varies alot for each

manufacturer. You could naturally see if it is an open, but it wouldn't tell

you much of anything else, such as carbon tracking, insulation breakdown,

etc. I think the best way to test a transformer is to plug in it and see if

each high voltage winding will draw an arc to the grounded case. A good arc

from each side would tell you it is working fine. Don't hold the wire in

your hands while it is plugged in though!

As for rotary gaps, I wouldn't recommend them at all for neon transformers.

They are just not built as rugged at pole pigs, and cannot handle the

voltage transients caused by the rotary gap. A good RQ style static gap

would probably outperform a rotary gap for your power level anyways. Hope

this helps.

Dave

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: my web page

Date: Wed, 15 Oct 1997 11:34:10 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Dave <djb@erie.net>

Date: Wed, 15 Oct 1997 07:34:10 -0400

Subject: Re: my web page

At 08:58 PM 10/13/97 GMT, you wrote:

>From: Mike Hammer <mhammer@misslink.net>

>Date: Mon, 13 Oct 1997 15:58:12 -0500

>Subject: my web page

>

>Hello All,

>

>Well I finally got around to putting up a web page.

>Can anybody guess what the subject matter of my page might be?

>

>Anybody that wants to check it out can find it at the following URL:

>http://www.misslink.net/mhammer

>

>Its fairly simple at this point but will contain

>more pics and information as my experience with HTML

>and my initiative allow.

>

>I would appreciate any and all feedback.

>

>TTYL!

>Mike Hammer

>mhammer@misslink.net

Nice page Mike,

Looking forward to seeing more pics and a section on your 'revelation'. Keep

up the good work.

Dave

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Spark gap question

Date: Wed, 15 Oct 1997 11:56:01 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Dave <djb@erie.net>

Date: Wed, 15 Oct 1997 07:56:01 -0400

Subject: Spark gap question

Hi,

If I had an RQ type gap, and a fixed number of copper pipes in it, and also

a fixed distance between the pipes, would varying the length of the pipes or

the pipe diameter effect the operation of the gap? If so, how can you

determine an optimum value for a given power to maximize quenching, heat

disipation, etc?

Dave

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: Coil specs

Date: Wed, 15 Oct 1997 13:52:55 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Kevin Batchelor <kbatchelor@arslimited.com>

Date: Wed, 15 Oct 1997 09:52:55 -0400

Subject: Re: Coil specs

First off...build a safety gap!  This will help to protect the transformer from

damage.  It can easily get some RF kickback and burn out the transformer. Chokes will also help to reduce this problem.  A few strong kickbacks and the transformer could be toast.  Probably won't happen with the caps your running now...

but in the future with bigger caps it will be a must.

Two big things for better performance:  Better caps and better gap.  The caps will make a great improvement once built...but you will also need a better gap.

Try the static RQ gap.  I built mine with 3/4" copper pipe pieces mounted in a

scrap of 4"pvc with a 4" fan on one end.  Works well and the cost was cheap.

Eventually you will want more power.  I'd recommend a 60ma transformer.  Probably 12,000 volts.  I've got a 60ma 15,000 it works fine but all that extra voltage has its own set of problems.

My primary is made of 1/4" copper tubing in a flat spiral.  Works pretty well.

It sit's above the wood by about 1" of plastic strips that I glued in place with notches to hold the tubing.

Well...if you havent' built a fused power supply with a variac, you may want to

do so.  It make operating your coil nice and you can turn the power output up

and down.  Works well.  When I build a new one, I'm going to put a voltage meter

on it and maybe a light or two to make it look really cool.

Secondary coil and toroid sound good.  Primary sounds good, but you didn't mention how you tuned it.  I have a large battery clip that I move around on the primary.

Build those caps and a good gap.  You should be set after that.

Good Luck and keep me informed.

Kevin

>>> Tesla-2 <tesla-2@emachine.com> 10/15 3:59 AM >>>

From: Simon Buck <sbuck@cgo.wave.ca>

Date: Wed, 15 Oct 1997 03:59:15 -0400

Subject: Coil specs

Here is the scoop on my coil. Comments and advice is welcome

My power source is a 15 kV 30 mA neon xfmr both sides now work (Woohoo),

for a gap I have L brackets and bolts. The gap is being upgraded soon. I

don't have a safety gap or RF chokes yet either, what size toroid should

I get and what size wire and how many turns do I need?

I used saltwater and beer bottle caps. I found hamster salt wheels work

well, they were not iodized and were 75 cents each, so it was in my

budget. I used one wheel for every 4.5 bottles. I now have 12 paralleled

for a total of 7 nf, but plan to make some poly plate caps in the next

few months.

I was anxious to get started, so I opted for the flat spiral primary.

For this I just stapled the wire to the board at the appropriate marks,

it was the easiest part. It's almost 50' of thick ground wire spaced

about 1/4" for a total of 15 turns.

The secondary is 21.5" of #24 magnet wire on 4" sealed PVC pipe. The

toroid is a 4x13" aluminum dryer duct. The ground is a copper pipe about

4' underground, with grounding wire coming in through a window, up to a

pipe. The pipe continues through the rafters into another room down some

#12 solid copper wire to the bottom of the secondary.

After some tuning I now get almost 8" sparks.

My wish list in order of priority

bigger sparks (hehe)

RF chokes

more capacitance

new gap

wire in the rafters instead of the pipe

more power

safety gap

new primary

depending on the power level, maybe a bigger toroid

The safety gap is not important until I get a new xfmr, as the guy who

gave me this one wants it back when the "science fair" is over.

Simon Buck <sbuck@cgo.wave.ca>

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: ready for winding

Date: Wed, 15 Oct 1997 15:03:28 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Eric Davidson <edavidson@icva.gov>

Date: Wed, 15 Oct 1997 10:03:28 -0500

Subject: Re: ready for winding

Tesla-2 wrote:

>

> From: thorn@northlink.com

> Date: Tue, 14 Oct 1997 23:41:53 -0700 (MST)

> Subject: Re: ready for winding

>

> Hello Mike,

>

>    About what was said about transformers.  I went to the local junk yard

> today to find a Neon trans.  I found several 15K, and only one 12K.  I don't

> even know if they are good or not, and they want to charge me $45.  Is this

> a good price for something that could be functionaly questionable?  If not,

> how can I determine if it is good or not?  I suppose that I could take along

> a meter to measure the windings resistances, but without knowing what

> resistance is considered "good" or not, how will I know?  An infinite

> resistance I am sure would be due to an "open" existing in the coil.  Does

> EMF durring testing alter the meters reading at all?  The meter will induce

> a voltage, so does that voltage produce an inhibiting magnetic field over

> the coil, thus reducing the meters actual reading?  I don't want to pay $45

> for a melted coil, so I will need to know for sure if it works.  Any

> suggestions?

> RMD

RMD:

I would not pay $45.00 for a used transformer, especially from a 'junk

yard'. It seems to me that a junk yard should almost give them away, or

charge a small salvage fee ($5.00 maybe) since they are more work than

they are worth to salvage the copper and iron. In my experience there is

no reliable way for the average person to test a NST. Yes, you can test

for an open secondary, but this is rarely the failure mode for NSTs.

Since the reactance of the secondary is made very high to limit the

current, they can tolerate a short in the secondary circuit almost

indefinitely.  The most common fail mode is insulation breakdown, and

most often inside the case.  This is very difficult to spot unless there

is evidence of potting material leaking from the case.  This happens at

the bottom and around the bushings. I would stay away from any NST with

ANY evidence of leakage. The best place to get a used one is from a sign

company that you trust. They usually remove working trannys form working

signs and reuse them. The ones that end up in junk yards have usually

been placed there by sign companies that have passed on them. Hope this

helps.

Eric

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: Spark gap question

Date: Wed, 15 Oct 1997 16:40:03 GMT

Organization: The Energy Machine Information System 504-733-3462

From: "Thomas McGahee" <tom_mcgahee@sigmais.com>

Date: Wed, 15 Oct 1997 12:40:03 -0400

Subject: Re: Spark gap question

----------

> From: Tesla-2 <tesla-2@emachine.com>

> To: tom_mcgahee@sigmais.com

> Subject: Spark gap question

> Date: Wednesday, October 15, 1997 7:56 AM

>

> From: Dave <djb@erie.net>

> Date: Wed, 15 Oct 1997 07:56:01 -0400

> Subject: Spark gap question

>

> Hi,

>

> If I had an RQ type gap, and a fixed number of copper pipes in it, and also

> a fixed distance between the pipes, would varying the length of the pipes or

> the pipe diameter effect the operation of the gap? If so, how can you

> determine an optimum value for a given power to maximize quenching, heat

> disipation, etc?

>

> Dave

>

>

Dave,

The longer the pipe, the greater the width of the arc (or the more parallel

arcs would be more precise...).

The wider the pipe, the greater the cooling ability... and thus the more

current it can handle.

Using forced air or a vacuum will also increase current handling considerably.

The THICKER the pipe wall, the more heat conduction is possible, and thus it

increases the current carrying capacity also.

Note that if you use a good airflow, then you can get by with shorter, less

thick pipes.

Do not use anything less than 3/4" diameter pipe.

Use HARD COPPER pipe/tubing. In general stay away from other materials. They

may work, but not as well or as cheaply.

Length of pipe should be at least 2 1/2". Anything less than this and you will

get localized heating problems.

Many of my static gaps use nine 3/4" diameter pipes 5" long. I have found this

is very inexpensive and able to handle about 3KW when used as a vacuum gap.

Figure 1.5KW with just a muffin fan or two. Make sure airflow is adequate.

Fr. Tom McGahee

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: SV: ready for winding

Date: Wed, 15 Oct 1997 16:51:03 GMT

Organization: The Energy Machine Information System 504-733-3462

From: "Eriksson" <jan-erik.eriksson@mbox2.swipnet.se>

Date: Wed, 15 Oct 1997 18:51:03 +0200

Subject: SV: ready for winding

Mike

Ok, so u say 21" winding with 24 AWG. I'm looking at a AWG list here, it

says that 24 AWG is 0.5mm plain copper wire. I bough 600m of 0.50mm magnet

wire (I bought it from an electric motor service and they said it were 24

AWG). And 975 (turns)* 0.5mm = 48.75cm and that is 19.19". But then I

have'nt count with the insulation. Should I do that?. I'm kinda confused

now. The problem might be that we don't use inch here just meters and

mms. And the primary winding, do u mean like this, (ascii art isn't my

best side)

o = copper tubing

/ = form

HH <----SECONDARY

HH

\ O         HH        O /

\ O       HH       O/

\ O     HH     O/

\O    HH   O/

Hrrrmm..well..heh..not the best but..)

I'm not saying that you're wrong or something like that, but I don't

exactly know how you mean (about the secondary winding).

Safe coilin!

//Christopher Eriksson

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: pig poles

Cc:

Bcc:

X-Attachments:

At 10:51 AM 10/15/97 GMT, you wrote:

>From: Dave <djb@erie.net>

>Date: Wed, 15 Oct 1997 06:51:36 -0400

>Subject: Re: pig poles

>

>At 06:31 AM 10/15/97 GMT, you wrote:

>>From: Dsurfr@aol.com

>>Date: Wed, 15 Oct 1997 02:31:12 -0400 (EDT)

>>Subject: pig poles

>>

>>I have noticed a lot of discussion on another Tesla site about "pig poles"

>>but have no idea of what they are referring to. It seems to be an important

>>safety feature that I am overlooking and would appreciate any advice on this

>>or any other safety devices that I should be incorporating in my new coil.

>>A.J.S.

>

>HI AJS,

>

>The correct term you are referring to is a 'pole pig'. What it is is a

>utility power distribution transformer. These are normally found on street

>poles and step down down the high line voltage from the power company to a

>voltage that you can use in your home, etc. I believe in Tesla coil use,

>they are run backwards as a step up transformer with some form of current

>limiting. This allows coilers to be able to use more power than what would

>be normally available with the use of neon sign transformers, etc. As a new

>coiler, you shouldn't be concerned with them. Even alot of coilers with much

>more experience are not using pole pigs. Neon sign transformers, etc should

>provide you with years of experience. Hope this helps.

>

>Good luck,

>Dave

The only thing I could add to what Dave said here is that

neon sign transformers are internally current limited. This

means you can directly short circuit the output (with a spark gap)

and only the rated current will flow. This makes them ideal

for Tesla coil use.

On the other hand pole pigs have no current limiting. Indeed

they are designed to supply lots of current even well above

their rating. If you short circuit one of these it will attempt

to pull all the current it can to supply the demand.

This could easily be 100s of amps. A very unsafe condition.

To be useful to power a Tesla coil external current limiting

must be provided. This is usually in the form of a large

variable inductance and some high power resistance. A large

30 amp 240 V variac is also necessary. While prices for refurbished

pole pigs are quite reasonable the necessary control circuitry

is not.

Paralled neon sign transformers can provide plenty of power

without the cost or complexity required to utilize a pole

transformer.

One other comment is to advise everyone not to get wrapped

up too much with brute power. There is more to getting long sparks

from a coil than just pumping in more power. I have succeeded

in getting 50 inch long streamers from just 2 paralled 12kv neons.

(12kv/60ma and 12kv/30ma)

Actual measured current flow is 100ma. (1200 VA)

Spend your time and money building efficient caps and quality

spark gaps.

TTYL!

Mike Hammer

mhammer@misslink.net

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: ready for winding

Cc:

Bcc:

X-Attachments:

At 06:41 AM 10/15/97 GMT, you wrote:

>From: thorn@northlink.com

>Date: Tue, 14 Oct 1997 23:41:53 -0700 (MST)

>Subject: Re: ready for winding

>

>Hello Mike,

>

>  About what was said about transformers.  I went to the local junk yard

>today to find a Neon trans.  I found several 15K, and only one 12K.  I don't

>even know if they are good or not, and they want to charge me $45.  Is this

>a good price for something that could be functionaly questionable?

This is quite a bit for something that maybe defective.

I usually get dead neons or unknown neons free or at most a couple

dollars.

>If not how can I determine if it is good or not?  I suppose that I could

>take along a meter to measure the windings resistances, but without knowing >what resistance is considered "good" or not, how will I know?  An infinite

>resistance I am sure would be due to an "open" existing in the coil.

All that a resistance check will show you is if the secondary

winding is intact. If you read a resistance from 2-10K then the

winding is probably OK. That DOES NOT mean it will work. The most

common failure mode for neons is insulation breakdown and carbon

tracking in the tar potting compound. These problems can be repaired

by unpotting the transformer.

One way to test a neon is to take a FUSED power cord with you

and plug in the suspect neon and try to draw an arc from each

HV terminal to the case. If you can draw equal arcs from each

side the neon is probably OK.

But be forewarned this neon could fail in seconds when placed

under any kind of stress. Neons with carbon track shorts in the

potting material can partially heal themselves after sitting idle

for years. Once some stress is applied the old wound opens back up.

Used neons are a risk but one we must take.

>

>Something that I have not heard much spoken about, is spark gaps.  Do you

>recomend using stationary/adjustable gaps, or rotary?

For use with neons a static gap such as the popular cylinder gap

is the best choice. Neons do not appreciate the high breakdown

voltage and pulse rates from a rotary gap. And a rotary is just

not necessary. Better performance can be had with a good static gap.

> The motors that run spark gaps, is there some reason that rather large

> motors are used?

The inertia of a large diameter rotor and air resistance to the spinning

electrodes requires a hefty motor.

<snip>

> I was thinking of utilizing a plastic turntable from an old record player

> as the rotor, and spin it horizontaly.

Like I indicated above forget about a rotary gap. They will not

provide any better performance that a good static gap and can place

greater stress on your neons and caps.

> Would carbon rods from "brush" motors work well?

Carbon is NOT a good electrode material. One important thing a gap

must do is to extinguish (quench) the arc once the power has been

transferred from primary to secondary. Carbon electrodes would defeat

that goal. Think about how a carbon arc lamp works. Once an arc

is struck with carbon electrodes it wont want to go back out.

TTYL!

Mike Hammer

mhammer@misslink.net

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: SV: ready for winding

Cc:

Bcc:

X-Attachments:

At 04:51 PM 10/15/97 GMT, you wrote:

>From: "Eriksson" <jan-erik.eriksson@mbox2.swipnet.se>

>Date: Wed, 15 Oct 1997 18:51:03 +0200

>Subject: SV: ready for winding

>

>Mike

>Ok, so u say 21" winding with 24 AWG. I'm looking at a AWG list here,

>it says that 24 AWG is 0.5mm plain copper wire. I bough 600m of 0.50mm magnet

>wire (I bought it from an electric motor service and they said it were 24

>AWG). And 975 (turns)* 0.5mm = 48.75cm and that is 19.19". But then I

>hav'nt count with the insulation. Should I do that?. I'm kinda confused

>now . The problem might be that we don't use inch here just meters and

>mms.

Chris,

My wire chart gives a diameter of .577 mm for AWG 24 which comes out to

56.25 cm for 975 turns. For us non-metric types thats 22.1 inches.

My chart also gives 46.3 turns per inch for AWG 24. That figures to

21.05 inches for 975 turns. The exact number of turns is not important.

The number of turns will vary according to the thickness of the insulation

of your wire and your skill in winding.

Just wind 21 inches (53.3 cm) and let the number of turns fall where it may.

> And the primary winding, do u mean like this, (ascii art isn't my

>best side)

>

>    o = copper tubing

>    / = form

>

>                         HH <----SECONDARY

>                         HH

>             \ O         HH        O /

>               \ O       HH       O/

>                 \ O     HH     O/

>                   \O    HH   O/

>Hrrrmm..well..heh..not the best but..)

Yes. That looks just fine. Make sure you can raise up your secondary

coil away from the primary several inches to reduce coupling if necessary.

TTYL!

Mike Hammer

mhammer@misslink.net

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: milestone for Tesla-2

Cc:

Bcc:

X-Attachments:

Hello all,

This week we have reached a milestone for the Tesla-2 list.

We have exceeded 100 subscribers. Quite amazing in

only 8 months time.

The honor of being number 100 goes to:

Dennis Hoffman  hoffman@werewolf.net

Welcome aboard Dennis.

TTYL!

Mike Hammer

mhammer@misslink.net

From: tesla-2@emachine.com (Tesla-2)

Subject: RE: ready for winding!

Date: Wed, 15 Oct 1997 18:57:07 GMT

Organization: The Energy Machine Information System 504-733-3462

From: "Eriksson" <jan-erik.eriksson@mbox2.swipnet.se>

Date: Wed, 15 Oct 1997 20:57:07 +0200

Subject: RE: ready for winding!

Mike, something just did just cross my mind. If I change the

secondary winding from 1100 turns to 975, wouldnt the sec. res. frequency

change to? And then I have to re-calculate the primary winding, right?

I've used WinTesla for all my calculations so far.

Safe coilin

/Christopher Eriksson

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: milestone for Tesla-2

Date: Wed, 15 Oct 1997 20:40:12 GMT

Organization: The Energy Machine Information System 504-733-3462

From: "Thomas McGahee" <tom_mcgahee@sigmais.com>

Date: Wed, 15 Oct 1997 16:40:12 -0400

Subject: Re: milestone for Tesla-2

----------

> From: Tesla-2 <tesla-2@emachine.com>

> To: tom_mcgahee@sigmais.com

> Subject: milestone for Tesla-2

> Date: Wednesday, October 15, 1997 2:41 PM

>

> From: Mike Hammer <mhammer@misslink.net>

> Date: Wed, 15 Oct 1997 13:41:40 -0500

> Subject: milestone for Tesla-2

>

> Hello all,

> This week we have reached a milestone for the Tesla-2 list.

> We have exceeded 100 subscribers. Quite amazing in

> only 8 months time.

>

> The honor of being number 100 goes to:

> Dennis Hoffman  hoffman@werewolf.net

>

> Welcome aboard Dennis.

>

> TTYL!

> Mike Hammer

> mhammer@misslink.net

>

Mike,

Congratulations to all who work to make this List such a good Tesla resource

for new coilers. The current number of subscribers is now over 100. Do you know

how many total subscribers have been cumulatively signed up? In other words, I

know that a number of people have signed up, been here for awhile, and then

unsubscribed since they learned what they wanted already. That might also be an

interesting statistic.

The membership of any group such as ours is dynamic rather than static, since

the whole thrust of this list is to help new coilers. After awhile they aren't

first-time coilers anymore. Of course, the list also has something to offer

people who have built several coils, and there are a number of members (myself

included) that hang around here for the purpose of helping to *answer* the

questions that pop up every day. The list needs us ALL! Keep up the good work!!

Fr. Tom McGahee

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: RE: ready for winding!

Cc:

Bcc:

X-Attachments:

At 06:57 PM 10/15/97 GMT, you wrote:

>From: "Eriksson" <jan-erik.eriksson@mbox2.swipnet.se>

>Date: Wed, 15 Oct 1997 20:57:07 +0200

>Subject: RE: ready for winding!

>

>Mike, something just did just cross my mind. If I change the

>secondary winding from 1100 turns to 975, wouldnt the sec. res. frequency

>change to? and then I have to re-calculate the primary winding, right?

>I've used WinTesla for all my calculations so far.

>Safe coilin

>/Christopher Eriksson

>

>

Yes that is correct. A 4 inch diameter secondary with

975 turns calculates out to about 392 KHZ unloaded.

Mine measured 395 KHZ. With a 6 inch diameter sphere

discharger this dropped to 345 KHZ and with a 19 X 6

toroid it dropped to 254 KHZ. These were all actual measured

frequencies.

With 1100 turns of AWG 24 (23.7inches) the frequency

calculates to about 360 KHZ.

With only a 35 KHZ difference I wouldnt change your primary.

When you start adding larger toroids to your system you

will appreciate the extra primary turns.

TTYL!

Mike Hammer

mhammer@misslink.net

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: pig poles

Date: Thu, 16 Oct 1997 04:14:13 GMT

Organization: The Energy Machine Information System 504-733-3462

From: randy@gte.net

Date: Wed, 15 Oct 1997 21:14:13 -0700

Subject: Re: pig poles

Tesla-2 wrote:

>

> From: Dsurfr@aol.com

> Date: Wed, 15 Oct 1997 02:31:12 -0400 (EDT)

> Subject: pig poles

>

> I have noticed a lot of discussion on another Tesla site about "pig poles"

> but have no idea of what they are referring to. It seems to be an important

> safety feature that I am overlooking and would appreciate any advice on this

> or any other safety devices that I should be incorporating in my new coil.

> A.J.S.

>

>

> -----

> Processed with Listserv v2.93 for Wildcat v4

"pole Pigs" (not vicey-versa) are the cylindrical transformers that

generally reside upon "telephone" poles. They convert X- Kilovolts to

your basic 20V with center tap (i.e. 110V). Like all transformers,

("xfmrs") they work just as well backwards, 110 or 220V in = x Kv out.

Randy

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: ready for winding

Date: Thu, 16 Oct 1997 04:31:27 GMT

Organization: The Energy Machine Information System 504-733-3462

From: randy@gte.net

Date: Wed, 15 Oct 1997 21:31:27 -0700

Subject: Re: ready for winding

Tesla-2 wrote:

>

> From: Dave <djb@erie.net>

> Date: Wed, 15 Oct 1997 07:16:44 -0400

> Subject: Re: ready for winding

>

> At 06:41 AM 10/15/97 GMT, you wrote:

> >From: thorn@northlink.com

> >Date: Tue, 14 Oct 1997 23:41:53 -0700 (MST)

> >Subject: Re: ready for winding

> >

> >Hello Mike,

> >

> >   About what was said about transformers.  I went to the local junk yard

> >today to find a Neon trans.  I found several 15K, and only one 12K.  I don't

> >even know if they are good or not, and they want to charge me $45.  Is this

> >a good price for something that could be functionaly questionable?  If not,

> >how can I determine if it is good or not?  I suppose that I could take along

> >a meter to measure the windings resistances, but without knowing what

> >resistance is considered "good" or not, how will I know?  An infinite

> >resistance I am sure would be due to an "open" existing in the coil.  Does

> >EMF durring testing alter the meters reading at all?  The meter will induce

> >a voltage, so does that voltage produce an inhibiting magnetic field over

> >the coil, thus reducing the meters actual reading?  I don't want to pay $45

> >for a melted coil, so I will need to know for sure if it works.  Any

> >suggestions?

> >

> >   Something that I have not heard much spoken about, is spark gaps.  Do you

> >recomend using stationary/adjustable gaps, or rotory?  The motors that run

> >spark gaps, is there some reason that rather large motors are used?  Does

> >the act of the gaping conductor passing between the electrodes cause some

> >kind of physical resistance?  One would figure that with sufficient

> >bearings, the rotor should be rather easy to bring to high speeds with

> >little torque required.  Is excessive vibration a problem, requiring tight

> >tolerances and balancing?  I can easily imagine the gap changing due to a

> >small error in the rotor/shaft assembly.  Is this a problem?  I was thinking

> >of utilizing a plastic turntable from an old record player as the rotor, and

> >spin it horizontaly. For the gaping conductors, I was thinking of having

> >twelve.  Each conductor would be a short piece of threaded rod screwed into

> >a hole that has been tapped for its thread size.  Two brass cap nuts would

> >then be screwed onto either end of the rod protruding on each side of the

> >rotor.  I have seen something similar to this on another gap, and they used

> >tig welding electrodes in theirs as the primary electrodes.  Is this ok to

> >duplicate?  Would carbon rods from "brush" motors work well?

> >

> >Thanks,

> >

> >RMD

>

> Hi RMD,

>

> I would not pay $45 for one transformer, but if that is the only place in

> the area to get one, then I might be persuaded to buy it having no other

> alternative. I don't think measuring the winding resistance of the neon

> secondary will tell you much. I believe it varies alot for each

> manufacturer. You could naturally see if it is an open, but it wouldn't tell

> you much of anything else, such as carbon tracking, insulation breakdown,

> etc. I think the best way to test a transformer is to plug in it and see if

> each high voltage winding will draw an arc to the grounded case. A good arc

> from each side would tell you it is working fine. Don't hold the wire in

> your hands while it is plugged in though!

>

> As for rotary gaps, I wouldn't recommend them at all for neon transformers.

> They are just not built as rugged at pole pigs, and cannot handle the

> voltage transients caused by the rotary gap. A good RQ style static gap

> would probably outperform a rotary gap for your power level anyways. Hope

> this helps.

>

> Dave

>

Find an electrician who knows what a "Megger" is. I dont...

but a megger, I think, shoots large voltages into the load in

question.... carbon tracking would possibly show up here...

On a similar note, I once stumbled across a test from some testing

society... it stated to apply X-volts FOR A GIVEN PERIOD OF TIME!

THE implication being, that dielectric breakdown is NOT necessarily

just a fucntion of voltage, but, rather, time plays a part.

Ever wonder why lightning takes such wicked paths, or why thunder

isn't like the single report of a cannon? It takes TIME for those

electrons to decide where they want to go....and it MAY not

co-incide with your designs....

RV

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: RF

Date: Thu, 16 Oct 1997 21:02:16 GMT

Organization: The Energy Machine Information System 504-733-3462

From: "Paul Marshall" <PMARSHAL@ETHCINC1.SSW.JNJ.COM>

Date: 16 Oct 1997 16:02:16 EST

Subject: RF

How do I calculate the resonate frequency of my secondary ?

What happens when the capacitance of the primary circuit

becomes too high ?

What formula do I use to calculate the max. capacitance of the

primary circuit ?

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: milestone for Tesla-2

Date: Fri, 17 Oct 1997 01:33:12 GMT

Organization: The Energy Machine Information System 504-733-3462

From: randy@gte.net

Date: Thu, 16 Oct 1997 18:33:12 -0700

Subject: Re: milestone for Tesla-2

Tesla-2 wrote:

>

> From: "Thomas McGahee" <tom_mcgahee@sigmais.com>

> Date: Wed, 15 Oct 1997 16:40:12 -0400

> Subject: Re: milestone for Tesla-2

>

> ----------

> > From: Tesla-2 <tesla-2@emachine.com>

> > To: tom_mcgahee@sigmais.com

> > Subject: milestone for Tesla-2

> > Date: Wednesday, October 15, 1997 2:41 PM

> >

> > From: Mike Hammer <mhammer@misslink.net>

> > Date: Wed, 15 Oct 1997 13:41:40 -0500

> > Subject: milestone for Tesla-2

> >

> > Hello all,

> > This week we have reached a milestone for the Tesla-2 list.

> > We have exceeded 100 subscribers. Quite amazing in

> > only 8 months time.

> >

> > The honor of being number 100 goes to:

> > Dennis Hoffman  hoffman@werewolf.net

> >

> > Welcome aboard Dennis.

> >

> > TTYL!

> > Mike Hammer

> > mhammer@misslink.net

> >

>

> Mike,

> Congratulations to all who work to make this List such a good Tesla resource

> for new coilers. The current number of subscribers is now over 100. Do you know

> how many total subscribers have been cumulatively signed up? In other words,I

> know that a number of people have signed up, been here for awhile, and then

> unsubscribed since they learned what they wanted already. That might also be an

> interesting statistic.

>

> The membership of any group such as ours is dynamic rather than static, since

> the whole thrust of this list is to help new coilers. After awhile they aren't

> first-time coilers anymore. Of course, the list also has something to offer

> people who have built several coils, and there are a number of members (myself

> included) that hang around here for the purpose of helping to *answer* the

> questions that pop up every day. The list needs us ALL! Keep up the good work!!

>

> Fr. Tom McGahee

>

I find it kind of alarming <the use of "shocking" here would be rather

lame on my part> that "only" 100 or so folks world-wide have seen fit

to subscribe to this list at any given time. There are millions watching

mindless TV shows/reruns....this is VERY interesting stuff, IMHO. Oh

well, just that much more for me when "we" take over the world.

As Fr. McGahee said, many thanks for those that run the list, and those

that comment on the questions/statments here.

Randy

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: RF

Cc:

Bcc:

X-Attachments:

At 09:02 PM 10/16/97 GMT, you wrote:

>From: "Paul Marshall" <PMARSHAL@ETHCINC1.SSW.JNJ.COM>

>Date: 16 Oct 1997 16:02:16 EST

>Subject: RF

>

>

>     How do I calculate the resonate frequency of my secondary ?

Its possible to calculate secondary resonant frequency quite

accurately. First you must determine the inductance of the secondary

winding using Wheeler's formula.

Wheeler's formula for inductance of solenoids

L(uH) = (r^2) * (N^2) / (9*r + 10*h)

where:

r = coil RADIUS in inches

N = number of turns

h = coil height in inches

Next you determine the secondary winding self capacitance using

Medhurst's formula.

Medhurst's formula for self capacitance

C  = K x D

where:  C = capacitance in picofarads

K = constant which depends on the ratio of the coil height to diameter

x = means multiply K times D

D = solenoidal coil diameter in centimeters

H = coil height in centimeters

H/D       K

5.0     0.81

4.5     0.77

4.0     0.72

3.5     0.67

3.0     0.61

2.5     0.56

2.0     0.50

1.5     0.47

1.0     0.46

Now that you have the inductance and self C then apply the resonant

frequency formula.

1

fo=  ------------------

/----------

2pi \/  LC

>     What happens when the capacitance of the primary circuit becomes

>     too high ?

If the primary cap is too large for the power supply transformer to

charge then little or no output will occur. The cap voltage may not

rise enough to fire the gap.

Also a very large cap will make the primary resonant frequency

very low. Perhaps too low to tune to the secondary resonant frequency.

Or at least causing you to tap in only a very few primary turns.

A large primary inductance is preferrable.

>     What formula do I use to calculate the max. capacitance of the

>     primary circuit ?

You would want to match the primary cap to the supply transformer.

First determine your neon sign transformer (or other transformer's)

impedence:

E

Z  =  ---

I

Z = Impedence

E = volts

I = current in Amps

Then determine the size of cap that has this impedance at 60 HZ.

1

C =   -------------------

2 x pi x Z x .00006

C = capacitance in microfarads needed for primary capacitor.

Z = transformer Impedance

Note: The .00006 is the 60 Hz AC. Use .00005 for 50 HZ.

This gives you the largest cap that your transformer can fully

charge. This may not be the best value however. Some experimentation

is always called for.

TTYL!

Mike Hammer

mhammer@misslink.net

From: tesla-2@emachine.com (Tesla-2)

Subject: Capacitors

Date: Fri, 17 Oct 1997 10:16:21 GMT

Organization: The Energy Machine Information System 504-733-3462

From: thorn@northlink.com

Date: Fri, 17 Oct 1997 03:16:21 -0700 (MST)

Subject: Capacitors

Hello everyone,

Can someone suggest how to determine the individual plate size used in

making ones own capacitor.  If I require, say, a .01uF for my primary cap,

how do I figure out what size to make the plate template?  I know that you

can add or remove plates after measuring the total capacitance with a meter,

until the desired capacitance is reached, but what is the plate size

suggested to begin with?

Thanks all!

Robert

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: milestone for Tesla-2

Date: Fri, 17 Oct 1997 10:52:00 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Simon Buck <sbuck@cgo.wave.ca>

Date: Fri, 17 Oct 1997 06:52:00 -0400

Subject: Re: milestone for Tesla-2

Randy wrote:

> From: randy@gte.net

> Date: Thu, 16 Oct 1997 18:33:12 -0700

> Subject: Re: milestone for Tesla-2

>

> I find it kind of alarming <the use of "shocking" here would be rather

> lame on my part> that "only" 100 or so folks world-wide have seen fit

> to subscribe to this list at any given time. There are millions watching

> mindless TV shows/reruns....this is VERY interesting stuff, IMHO. Oh

> well, just that much more for me when "we" take over the world.

>

Wow, here I thought Tesla coils were just about making sparks. I had no idea "we"

had a plan for world domination. This is getting better and better all the time.

Anyone with experience have any tips for taking over the world? :)

> As Fr. McGahee said, many thanks for those that run the list, and those

> that comment on the questions/statments here.

> Randy

>

Ditto

Simon Buck <sbuck@cgo.wave.ca>

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: Capacitors

Cc:

Bcc:

X-Attachments:

At 10:16 AM 10/17/97 GMT, you wrote:

>From: thorn@northlink.com

>Date: Fri, 17 Oct 1997 03:16:21 -0700 (MST)

>Subject: Capacitors

>

>Hello everyone,

>

>   Can someone suggest how to determine the individual plate size used in

>making ones own capacitor.  If I require, say, a .01uF for my primary cap,

>how do I figure out what size to make the plate template?  I know that you

>can add or remove plates after measuring the total capacitance with a meter,

>until the desired capacitance is reached, but what is the plate size

>suggested to begin with?

>

>Thanks all!

>

Robert,

The plate size used is one factor that determines the

capacitance as well as the physical size of the capacitor.

Larger plates reduce the number of plates required to achieve

your desired capacitance. However large plates tend to trap more

air between them that is harder to remove.

I made a plate cap with 10 X 10 inch plates (actual overlapping area)

but consider that a little too large. Future plate caps will most

likely be 8 X 8.

Here is the formula for determining capacitance of a plate type cap.

Capacitance (in microfarads) = (0.224 KA / d) (N-1)

0.224 x Dielectric Constant x Area of plate

C = -----------------------------------------  x (Number of plates - 1)

distance between plates in inches

NOTE: For two plate rolled capacitors the value derived from this

formula must be multiplied by two, or, the value must be computed

with the number of plates (N) equal to three (3), so (N-1 = 2)

The dielectric constant (K) of polyethylene is from 2.0-2.3

with a nominal value in homemade caps of 2.0.

TTYL!

Mike Hammer

mhammer@misslink.net

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: milestone for Tesla-2

Cc:

Bcc:

X-Attachments:

At 08:40 PM 10/15/97 GMT, you wrote:

>From: "Thomas McGahee" <tom_mcgahee@sigmais.com>

>Date: Wed, 15 Oct 1997 16:40:12 -0400

>Subject: Re: milestone for Tesla-2

>

>

>

>----------

>> From: Tesla-2 <tesla-2@emachine.com>

>> To: tom_mcgahee@sigmais.com

>> Subject: milestone for Tesla-2

>> Date: Wednesday, October 15, 1997 2:41 PM

>>

>> From: Mike Hammer <mhammer@misslink.net>

>> Date: Wed, 15 Oct 1997 13:41:40 -0500

>> Subject: milestone for Tesla-2

>>

>> Hello all,

>> This week we have reached a milestone for the Tesla-2 list.

>> We have exceeded 100 subscribers. Quite amazing in

>> only 8 months time.

>>

>> The honor of being number 100 goes to:

>> Dennis Hoffman  hoffman@werewolf.net

>>

>> Welcome aboard Dennis.

>>

>> TTYL!

>> Mike Hammer

>> mhammer@misslink.net

>>

>

>Mike,

>Congratulations to all who work to make this List such a good Tesla resource

>for new coilers. The current number of subscribers is now over 100. Do you know

>how many total subscribers have been cumulatively signed up? In other words, I

>know that a number of people have signed up, been here for awhile, and then

>unsubscribed since they learned what they wanted already. That might also be an

>interesting statistic.

>

Fr. Tom,

To answer your question I retrieved the unsubscribes from the list

server this morning. There are 38 on that list.

However several names appear multiple times as people unsubscribed

for vacations and ISP changes. So subtracting the names from

the list that are currently resubscribed leaves 28 names.

3 more names should be substracted for people that intend to return.

One is on vacation and one is changing ISPs.

The third is getting married so his future as a coiler may be

in doubt. :)

So that leaves 25 that have joined and left for one reason or another.

Some of them had no idea what they were getting into. Some

have moved on to other things. It seems that those interested

in coils and coiling have stayed on.

What is interesting is that of the original 16 subscribers who came

on board when the list first started 14 are still here.

>The membership of any group such as ours is dynamic rather than static, since

>the whole thrust of this list is to help new coilers. After awhile they aren't

>first-time coilers anymore. Of course, the list also has something to offer

>people who have built several coils, and there are a number of members (myself

>included) that hang around here for the purpose of helping to *answer* the

>questions that pop up every day. The list needs us ALL! Keep up the good work!!

>

A great deal of time and effort is required maintaining a list

such as this. As we go around patting each other on the back

lets not forgot our mostly silent partner without whom this

list would not exist. Ralph Hartwell is the list owner and has

supplied the equipment and his time to make this list possible.

Most of his effort is behind the scenes so goes unnoticed.

Let us all thank Ralph for his time and effort.

I also wish to thank Fr. Tom and Bert Hickman for their time

and participation.  Fr. Tom for making sure my answers make

sense (they dont always) and Bert for patiently answering MY questions.

TTYL!

Mike Hammer

mhammer@misslink.net

From: tesla-2@emachine.com (Tesla-2)

Subject: winding length vs #of turns

Date: Fri, 17 Oct 1997 22:55:16 GMT

Organization: The Energy Machine Information System 504-733-3462

From: "Chris Watkins" <cwatkins@seidata.com>

Date: Fri, 17 Oct 1997 17:55:16 -0500

Subject: winding length vs #of turns

Help!

I have aquired 15.5 lbs of 17ga magnet wire (continious spool)

I also have a 57" long peice of 10" pvc pipe 10.75" od.

Should I wind this for a 3:1 ratio or go for 800+ turns?

I had originaly planned on 43" winding length. =900turns.

but now  after further study I am undecided.

Would it be worth it to get a bigger pipe?

or should i use less turns?

what would you suggest ?

just fyi here are the rest of my components

primary  10.5 turns .375 od tubing  flat spiral spaced 1.062

from secondary with .5 spacing between coils.

caps    sw bottle caps - have not constructed yet

power supply

4 microwave trannys  each in its own pvc container filled

with oil.  the primaries are all 110v in proper phase, with

the secondaries in series. approx 10kv output with a 1uF

cap across the secondary of 1 inside trans.

this is some serious power!!!!

I am working on a current limiter at present time

I'm no electrical expert but I learn more every day.

I have 2 iron cores from variable transformers  4"

that i plan to rewind with just a few loops as limiters.

will this work?

I havent made a toroid yet but plan on something around

12" x 36"  or 12 x 48"

I look forward to any comments

Chris Watkins

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: WinTesla

Date: Sat, 18 Oct 1997 05:22:20 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Chris Federico <cfed@comsource.net>

Date: Sat, 18 Oct 1997 00:22:20 -0500

Subject: WinTesla

I was reading a post where someone mentioned using the program WinTesla for

their calculations.  Any suggestions on where to find it, or would someone

please attach me a copy?

TIA. . .Chris

P.S.  Hello All!  I found the list while looking for help in figuring out

why my first coil fired-up with a whimper instead of a bang.  I probably

should unsubscribe soon because I told myself that this was going to be my

first and last coil, but after reading the posts here for a few days I feel

myself getting hooked.  But for now I'll just lurk for a while and see which

side wins.

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: "Chris"

Date: Sat, 18 Oct 1997 08:02:47 GMT

Organization: The Energy Machine Information System 504-733-3462

From: thorn@northlink.com

Date: Sat, 18 Oct 1997 01:02:47 -0700 (MST)

Subject: "Chris"

Hello Chris,

I have heard it said that being married is a reason for some coilers to

fade into  the background when it comes to continuing to be a coiler.  Well,

I am married, and although my wife does not understand my fascination with

electrocuting myself, I continue with her support. She even helps me find

and pay for parts! (as long as I don't give her any new hair-doos)  Never

wonder if you are a coiler!  Once bitten by the high power bug, you can

never go back.  If you do, it will only sit in the back of your mind,

further supporting the fact that once a coiler, always a coiler!  If you

need to take a break, do so, but don't give up!

I do not have wintesla, but I am sure that you will find all that you need

here in the group.  The people here are very nice and extremely helpfull!

I wonder though, are there any girls in the group?  If not, why?

Happy coiling!!!

Robert

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: WinTesla

Date: Sat, 18 Oct 1997 10:55:42 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Dave <djb@erie.net>

Date: Sat, 18 Oct 1997 06:55:42 -0400

Subject: Re: WinTesla

At 05:22 AM 10/18/97 GMT, you wrote:

>From: Chris Federico <cfed@comsource.net>

>Date: Sat, 18 Oct 1997 00:22:20 -0500

>Subject: WinTesla

>

>

>I was reading a post where someone mentioned using the program WinTesla for

>their calculations.  Any suggestions on where to find it, or would someone

>please attach me a copy?

>

>TIA. . .Chris

>

>

>P.S.  Hello All!  I found the list while looking for help in figuring out

>why my first coil fired-up with a whimper instead of a bang.  I probably

>should unsubscribe soon because I told myself that this was going to be my

>first and last coil, but after reading the posts here for a few days I feel

>myself getting hooked.  But for now I'll just lurk for a while and see which

>side wins.

Hi,

Here's the homepage for R. Scott Coppersmith's Wintesla program.

http://members.aol.com/rscopper/index.htm

From: tesla-2@emachine.com (Tesla-2)

Subject: wire problems!

Date: Sat, 18 Oct 1997 16:38:45 GMT

Organization: The Energy Machine Information System 504-733-3462

From: "Alex Parker" <axparker@javanet.com>

Date: Sat, 18 Oct 1997 09:38:45 -0700

Subject: wire problems!

Consider this scenario for a second...

I receive my pound of #24 magnet wire in the mail, and I happily begin to

wind my first coil secondary. Unfortunately, this is an infuriatingly

time-consuming process. Next thing I know, my parental units are summoning

me to dinner (if this sounds weird, I am 16 and live w/my parents). "yeah

yeah, hold on a minute!", says I. But to no avail, they demand I come and

eat now, so I have no choice but to stop winding and cover the tope

windings with duct tape. When I come back, about 2.5 inches of the coil has

loosened to the point where I have to rewind it. When I rewound it the

second time, however, it was full of little kinks and overlaps.

Now, to the point of this story: I've heard that kinks and overlaps in the

secondary are seriously detrimental to the performance of the coil. What I

need to know is, is it worth scrapping the whole secondary and starting

from scratch, or can I continue on using it. Exactly how much does this

problem affect overall coil performance? Also, is there an easier way to

wind the secondary? (assuming that I can't get one of my friends over my

house for 3 hours to help me wind it).

While I'm on the subject, I'm also having some problems constructing my

primary. My original idea was to use about 14 turns of 1/4 inch copper

tubing. Then I discovered that copper tubing has a particular aversion to

being bent or shaped (especially into a spiral). What I ended up with was

about 10 feet or expensive horribly mangled copper tubing. If anyone has

any good ideas for primary materials that are easier to shape or better

than copper tubing, please say so. Or if there are any ideas on how to

better shape the copper tubing, I'd love to hear them.

My last problem is with magnet wire. A 1lb. spool just isn't enough, and a

10lb. spool, (which is the only other amount I have found magnet wire in)

is just too expensive. Does anyone out there know a place that will

continuously spool the wire by the pound? Or is there a way to tie two 1lb.

spools together without putting massive kinks in the secondary?

Ok, that's the end of my long-windedness. Thanks for listening.

Happy coiling to all.

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

Dave

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: wire problems!

Date: Sat, 18 Oct 1997 15:18:54 GMT

Organization: The Energy Machine Information System 504-733-3462

From: "Kent Vander Velden" <graphix@iastate.edu>

Date: Sat, 18 Oct 1997 10:18:54 -0500

Subject: Re: wire problems!

>Now, to the point of this story: I've heard that kinks and overlaps in the

>secondary are seriously detrimental to the performance of the coil. What I

>need to know is, is it worth scrapping the whole secondary and starting

>from scratch, or can I continue on using it. Exactly how much does this

>problem affect overall coil performance? Also, is there an easier way to

>wind the secondary? (assuming that I can't get one of my friends over my

>house for 3 hours to help me wind it).

How are you winding the coil?  Hopefully not completely by hand...

Also, I have found that winding onto a form that the varnish is still

a little wet on helps.

>While I'm on the subject, I'm also having some problems constructing my

>primary. My original idea was to use about 14 turns of 1/4 inch copper

>tubing. Then I discovered that copper tubing has a particular aversion to

>being bent or shaped (especially into a spiral). What I ended up with was

>about 10 feet or expensive horribly mangled copper tubing. If anyone has

>any good ideas for primary materials that are easier to shape or better

>than copper tubing, please say so. Or if there are any ideas on how to

>better shape the copper tubing, I'd love to hear them.

Get a 25' or 50' spool of fresh tubing from the hardware store.

This stuff is wonderful since it is already rolled in a spiral.  Shaping

the tubing to form the secondary is fairly easy then.  My form has 1/4"

grooves cut in it.  I was able to hold the tubing above the form and

work it into the grooves as I worked around in circles.  Worked really

well.  In another primary, I tried to drill holes which were cut in half and

the tubing sandwiched between but this was much harder and the

result was not as pretty as the grooved primary.

Also 10 feet will not give you many turns.  Treat each turn as a separate

circle and sum up the circumference of each.  Even 50' burns up pretty

fast.  Think that I was able to get 15 turns starting with a 5.5"

inner diameter.  Since the tube is 1/4" in diameter and I used 1/4"

spacing between turns and the circles are growing relative from their

centers, the sum for the total circumference on my primarily looks like:

(5.5 + 6.5 + 7.5 + 8.5 ... ) * Pi

Which gives a result in inches.  I added as many progressively larger

circles into the above sum until all the tubing was used up as full turns.

Of course I was using a non-solenoid type primary, the tubing would

go a lot further with the solenoid type primary.  While other people

can probably say better than on on this topic, my luck with a solenoid

type primary was not good - over-coupled too easily.

>My last problem is with magnet wire. A 1lb. spool just isn't enough, and a

>10lb. spool, (which is the only other amount I have found magnet wire in)

>is just too expensive. Does anyone out there know a place that will

>continuously spool the wire by the pound? Or is there a way to tie two 1lb.

>spools together without putting massive kinks in the secondary?

What does a 10lbs spool cost where you live?  Here it is ~ $65.  While

expensive, you can make several secondary with that and was actually

less expensive than buying several 1lbs segments.  You are going to be

happier without having to deal with multiple pieces anyway.  Perhaps a

shop in your area will be willing to let a partial spool go...

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: wire problems!

Cc:

Bcc:

X-Attachments:

At 04:38 PM 10/18/97 GMT, you wrote:

>From: "Alex Parker" <axparker@javanet.com>

>Date: Sat, 18 Oct 1997 09:38:45 -0700

>Subject: wire problems!

>

>        Consider this scenario for a second...

>

>I receive my pound of #24 magnet wire in the mail, and I happily begin to

>wind my first coil secondary. Unfortunately, this is an infuriatingly

>time-consuming process. Next thing I know, my parental units are summoning

>me to dinner (if this sounds weird, I am 16 and live w/my parents). "yeah

>yeah, hold on a minute!", says I. But to no avail, they demand I come and

>eat now, so I have no choice but to stop winding and cover the tope

>windings with duct tape. When I come back, about 2.5 inches of the coil has

>loosened to the point where I have to rewind it. When I rewound it the

>second time, however, it was full of little kinks and overlaps.

Alex. Doesn't sound weird at all. All of us were 16 at one time

(those were the days!) and all of us had parents.

What I do when winding a secondary and get interupted is to

secure the winding with a strip of masking tape or a dab

of hot glue. The hot glue gun approach works pretty well.

Just put a dab on the winding every couple inches to keep

things from unraveling should you lose your grip on the form.

Later you can peel those blobs of hot glue off before varnishing

your secondary.

>

>Now, to the point of this story: I've heard that kinks and overlaps in the

>secondary are seriously detrimental to the performance of the coil. What I

>need to know is, is it worth scrapping the whole secondary and starting

>from scratch, or can I continue on using it. Exactly how much does this

>problem affect overall coil performance? Also, is there an easier way to

>wind the secondary? (assuming that I can't get one of my friends over my

>house for 3 hours to help me wind it).

Kinks and overlaps are bad news. Best off unwinding down to the problem

and straightening things out.

I made a hand cranked winder from some wood scraps and a broom

handle. Makes the job lots easier. Have a friend crank while

you guide the wire.

>

>While I'm on the subject, I'm also having some problems constructing my

>primary. My original idea was to use about 14 turns of 1/4 inch copper

>tubing. Then I discovered that copper tubing has a particular aversion to

>being bent or shaped (especially into a spiral). What I ended up with was

>about 10 feet or expensive horribly mangled copper tubing. If anyone has

>any good ideas for primary materials that are easier to shape or better

>than copper tubing, please say so. Or if there are any ideas on how to

>better shape the copper tubing, I'd love to hear them.

Did you buy soft refrigeration tubing or the harder plumbing

grade tubing. The soft tubing is not too difficult to shape

or bend and comes in a nice spiral shape right out of the box.

If you got plumbing grade tubing it will be next

to imposssible to shape it adequately.

The idea is to take advantage of the shape the tubing comes in

rather than fight it into a different shape,.

>

>My last problem is with magnet wire. A 1lb. spool just isn't enough, and a

>10lb. spool, (which is the only other amount I have found magnet wire in)

>is just too expensive. Does anyone out there know a place that will

>continuously spool the wire by the pound?

By far the best place to buy magnet wire is a local

electric motor rebuilding shop. These places are everwhere.

Check the yellow pages under electric motors.

>Or is there a way to tie two 1lb. spools together without putting

>massive kinks in the secondary?

It is possible to solder two wire together and file down

the joint as flat as possible. Its not easy to do. I would

suggest practicing on some scrap wire first.

Doing this will result in a weak spot that will promote

corona and arcing from this area. cover it with a thick blob

of epoxy to improvce the insulation. Better yet avoid having

to do this in the first place.

TTYL!

Mike Hammer

mhammer@misslink.net

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: winding length vs #of turns

Date: Sat, 18 Oct 1997 16:40:20 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Bert Hickman <bert.hickman@aquila.com>

Date: Sat, 18 Oct 1997 09:40:20 -0700

Subject: Re: winding length vs #of turns

Organization: Stoneridge Engineering

Chris,

Welcome to coiling! My comments are interspersed below...

-- Bert --

Tesla-2 wrote:

>

> From: "Chris Watkins" <cwatkins@seidata.com>

> Date: Fri, 17 Oct 1997 17:55:16 -0500

> Subject: winding length vs #of turns

>

> Help!

> I have aquired 15.5 lbs of 17ga magnet wire (continious spool)

> I also have a 57" long peice of 10" pvc pipe 10.75" od.

> should I wind this for a 3:1 ratio or go for 800+ turns?

> I had originaly planned on 43" winding length. =900turns.

> but now  after further study I am undecided.

> Would it be worth it to get a bigger pipe?

> or should i use less turns?

> what would you suggest ?

The wire diameter is a little large for the diameter of your coilform,

but it should work OK. Also, the coilform length is not a critical

parameter either. However, I'd recommend looking at an aspect ratio in

the range of around 3.5:1 because of the larger wire diameter. This will

get you to about 37" or so and about 750-780 turns, and should provide

you with a comfortable distance between the toroid and the primary. BTW,

it will take a fair amount of power to drive this coil optimally (2 - 3

KVA). To "get your feet wet", you may want to consider first building a

smaller coil so that you get some experience with working with high

voltages and high frequencies before building a large system. It'll also

save you some $$.

>

> just fyi here are th rest of my componets

>

> primary  10.5 turns .375 od tubing  flat spiral spaced 1.062

>               from secondary with .5 spacing between coils.

>

You'll need to increase the distance between the innermost turn of the

primary and the secondary to at least 1.5" to permit you to adjust the

secondary up or down relative to the primary without getting arc-overs

between them. A flashover at this point will often do significant damage

to the secondary, and has been known to create shorted turns on the

secondary due to welding between adjacent turns. This means your

innermost primary diameter should be at least 13.75" diameter. [BTW,

I've assumed that your spacing between coils means a center-to-center

spacing of 0.5" + 0.375" = 0.875", making your outermost diameter about

33". If you actually meant that the center-to-center spacing was 0.5"

this is much too close (only 1/8" air-gap between turns) and will result

in turn-to-turn flashovers on your primary].

I'd recommend adding at least 5 more turns to your primary to reduce the

primary capacitance neede to bring the system into tune. A 10 turn

primary will give you about 62 uH of primary inductance, and 15 turns

will get you up to 141 uH - you'll need over twice the capacitance in

the primary circuit to bring the 10-turn system into tune.

> caps    sw bottle caps - have not constructed yet

You're going to need a lot of bottles! Assuming you use a 8" x 36"

toroid, your secondary's "loaded" operating frequency will be around 116

kHz. If you were tapped at turn 15, you'll need over 13 nF (0.013 uF) of

tank capacitance to bring your coil into tune. If you only use a

ten-turn primary, it will take over 30 nF (0.03 uF) to do the job. Since

a typical "beer bottle" capacitor will deliver about 650 pF per bottle,

it'll take about 20-25 bottles to get to 13 nF, and about 50 to get you

to 0.03 uF. As you are probably aware, bottle caps deliver poor

performance (very high losses), and are most effectively used on smaller

coils. A coil this size really needs caps made from polyethylene or

polypropylene dielectric to operate optimally.

>

> power supply

>  4 microwave trannys  each in its own pvc container filled

> with oil.  the primaries are all 110v in proper phase, with

> the secondaries in series. approx 10kv output with a 1uF

> cap across the secondary of 1 inside trans.

> this is some serious power!!!!

The biggest problem with this configuration is that you run a very great

risk of flashing over between primary and secondary or from the

secondary to the core and then to the common primaries (if you've

removed the interconnection between the secondary and the core).

Microwave transformers are simply not made to withstand the peak

voltages you'll be applying, and will probably take out the two

outermost transformers the first time you power them up. Individually

immersing them in oil will not necessarily prevent this, since the

insulation may break down internally. This approach won't work reliably,

and is very dangerous.  A much simpler, and better, approach is to use a

number of neon sign transformers in parallel.

Two or three 12 KV 60 MA transformers will work much better and you

won't have to worry about how to current-limit them. Neon sign outfits

usually have ones they've pulled out of service from tear-down jobs that

they'll sell fairly inexpensively ($5 - $15). You can often take

non-working ones and get them to work again with some effort. If you're

using 30 nF of tank capacitance, two 12 KV 60 MA transformers will

marginally drive the system, with three (180 MA) being better. Only two

transformers (120 MA) are required with 13 nF.

>

> I am working on a current limiter at present time

> I'm no electrical expert but i learn more every day.

>

> I have 2 iron cores from variable transformers  4"

> that i plan to rewind with just a few loops as limiters.

> will this work?

>

Stick with neon trannies the first time around. Although the cores from

a vaiable transformer can be made to work with some effort (you need to

cut the cores to make an air gap, and it will take much more than a few

turns of wire), it's a lot easier to use self-limiting transformers.

Trust me, you'll run into a whole lot of other interesting challenges in

getting your coil to work without having to solve this one...

> I havent made a torrid yet but plan on something around

> 12" x 36"  or 12 x 48"

>

Nice size toroids! However, for the size coil you're constructing, I'd

reduce the smaller diameter to about 6" or 8" since aluminum

dryer-ducting is fairly readily available in these sizes. Also, you

probably will not have enough power (i.e., output voltage) to be able to

break out a toroid with a 12" smaller diameter. I presently use a 32" x

8" toroid on my system with excellent results.

> I look forward to any comments

> Chris Watkins

You also need to give very serious consideration to the RF grounding

system you'll be using for the base of the coil, and to the spark gap

system. You'll need a heavy RF ground path going to at least 1-2

eight-foot ground rods going from the base of your secondary. Also, a

vacuum gap will be essential at this power level. A series of hard

copper pipes spaced with about 0.025 - 0.030" gaps with high velocity

air flowing through the gaps is necessary to properly handle the power

and quench the gaps properly.

Safe coilin' to you!

-- Bert --

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: female coilers?

Cc:

Bcc:

X-Attachments:

At 08:02 AM 10/18/97 GMT, you wrote:

>From: thorn@northlink.com

>Date: Sat, 18 Oct 1997 01:02:47 -0700 (MST)

>Subject: "Chris"

>

>Hello Chris,

>

<snip>

>

>  I wonder though, are there any girls in the group?  If not, why?

>

>Happy coiling!!!

>

>Robert

Robert,

No females on the list at present to my knowledge.

While there have been a couple they didnt stay for very long.

Seems that females and coils dont mix well. They just don't seem to

"get it". Coiling is primarily a "guy" thing.

TTYL!

Mike Hammer

mhammer@misslink.net

From: tesla-2@emachine.com (Tesla-2)

Subject: gee...primary problems =)

Date: Sat, 18 Oct 1997 17:51:09 GMT

Organization: The Energy Machine Information System 504-733-3462

From: "Eriksson" <jan-erik.eriksson@mbox2.swipnet.se>

Date: Sat, 18 Oct 1997 19:51:09 +0200

Subject: gee...primary problems =)

Hi, I'm currently winding the primary and I have some problem.

I cant get it that good, so do you have any tips on how to

wind a good primary? What should I use as a form (OR should i use a form?)

I also got medium soft copper tubing, 6mm diameter. Maybe soft would be

better, but medium is all I can get right now.

Thanks

/Christopher Eriksson

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: gee...primary problems =)

Cc:

Bcc:

X-Attachments:

At 05:51 PM 10/18/97 GMT, you wrote:

>From: "Eriksson" <jan-erik.eriksson@mbox2.swipnet.se>

>Date: Sat, 18 Oct 1997 19:51:09 +0200

>Subject: gee...primary problems =)

>

>

>

> Hi, I'm currently winding the primary and i have some problems.

>I cant get it that good, so do you have any tips on how to

>wind a good primary? What should I use as a form (OR should i use a form?)

>I also got medium soft copper tubing, 6mm diameter. Maybe soft would be

>better, but medium is all I can get right now.

>Thanks

>

>/Christopher Eriksson

Christopher,

It all depends on what type of primary you are building.

If its an inverted conical (saucer) shaped primary then

cut out 6 triangular pieces of Lexan or plexiglass. Wood

can even be used successfully.

Make the slope of the triangles equal to the intended slope

of the primary. 15-30 degrees.

Cut notches in the triangles to accept the copper tubing.

You will need to stagger the notches since the coil spirals

outward as well as up.

Mount the triangles in a circular pattern on a solid base

and wind the copper into the notches using the preshaped

spiral the tubing comes in. Work from the inside out.

Bend the tubing only as much as necessary to fit into

the next notch. Secure with some tie wraps or even

blobs of hot glue as you go around.

Dont worry if your primary isnt a work of art. Ugly

primary coils work just as well as pretty ones.

Perhaps some others will share their primary coil experience

with Christopher.

TTYL!

Mike Hammer

mhammer@misslink.net

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: wire problems!

Date: Sun, 19 Oct 1997 02:29:47 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Chris Federico <cfed@comsource.net>

Date: Sat, 18 Oct 1997 21:29:47 -0500

Subject: Re: wire problems!

The Coax Connection:

Joe Cassata,

10 S. Meadow lane

Naperville, Ill. 60564

Phone# (630)-420-0342

I've done business with the above fellow a couple of times.  He needs a

check in advance for the first order, and will COD a second.  He ships

promptly, and will sell you any length you want.

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: female coilers?

Date: Sun, 19 Oct 1997 02:56:38 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Dave <djb@erie.net>

Date: Sat, 18 Oct 1997 22:56:38 -0400

Subject: Re: female coilers?

At 04:52 PM 10/18/97 GMT, you wrote:

>From: Mike Hammer <mhammer@misslink.net>

>Date: Sat, 18 Oct 1997 11:52:33 -0500

>Subject: female coilers?

>

>At 08:02 AM 10/18/97 GMT, you wrote:

>>From: thorn@northlink.com

>>Date: Sat, 18 Oct 1997 01:02:47 -0700 (MST)

>>Subject: "Chris"

>>

>>Hello Chris,

>>

><snip>

>

>>

>>  I wonder though, are there any girls in the group?  If not, why?

>>

>>Happy coiling!!!

>>

>>Robert

>

>Robert,

>No females on the list at present to my knowledge.

>While there have been a couple they didnt stay for very long.

>

>Seems that females and coils dont mix well. They just don't seem to

>"get it". Coiling is primarily a "guy" thing.

>

>TTYL!

>Mike Hammer

>mhammer@misslink.net

I would say engineering is primarily a guy thing unfortunately. There are

only 2 girls in my Electrical engineering classes compared to about 70 guys.

And one of those girls is an exchange student from Peru.

Dave

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: gee...primary problems =)

Date: Sun, 19 Oct 1997 08:42:30 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Simon Buck <sbuck@cgo.wave.ca>

Date: Sun, 19 Oct 1997 04:42:30 -0400

Subject: Re: gee...primary problems =)

Tesla-2 wrote:

> From: Mike Hammer <mhammer@misslink.net>

> Date: Sat, 18 Oct 1997 14:30:12 -0500

> Subject: Re: gee...primary problems =)

>

> At 05:51 PM 10/18/97 GMT, you wrote:

> >From: "Eriksson" <jan-erik.eriksson@mbox2.swipnet.se>

> >Date: Sat, 18 Oct 1997 19:51:09 +0200

> >Subject: gee...primary problems =)

> >

> >

> >

> > Hi, I'm currently winding the primary and i have some problems.

> >I cant get it that good, so do you have any tips on how to

> >wind a good primary? What should I use as a form (OR should i use a form?)

> >I also got medium soft copper tubing, 6mm diameter. Maybe soft would be

> >better, but medium is all i can get right now.

> >Thanks

> >

> >/Christopher Eriksson

>

> Christopher,

> It all depnds on what type of primary you are building.

> If its an inverted conical (saucer) shaped primary then

> cut out 6 triangular pieces of Lexan or plexiglass. Wood

> can even be used successfully.

>

> Make the slope of the triangles equal to the intended slope

> of the primary. 15-30 degrees.

>

> Cut notches in the triangles to accept the copper tubing.

> You will need to stagger the notches since the coil spirals

> outward as well as up.

>

Wouldn't it be easier if you just stacked the triangles and drill say 3 at a

time and just stagger them when you mount them on the base?

> Mount the triangles in a circular pattern on a solid base

> and wind the copper into the notches using the preshaped

> spiral the tubing comes in. Work from the inside out.

> Bend the tubing only as much as necessary to fit into

> the next notch. Secure with some tie wraps or even

> blobs of hot glue as you go around.

>

> Dont worry if your primary isnt a work of art. Ugly

> primary coils work just as well as pretty ones.

>

> Perhaps some others will share their primary coil experience

> with Christopher.

>

> TTYL!

> Mike Hammer

> mhammer@misslink.net

>

For my flat spiral primary, I used 8 guage stranded bare copper wire. I marked

the inner turn of the primary with a compass in the centre of  the square base

and drew 8 lines through the centre at 45 degree angles. I numbered these lines

made 0.5" inch marks along the length of each one, with the next line I started

slighty farther from the circle.

Then I just used a staple-gun to secure the wire. I am currently getting about

12" from my 4"x21" secondary.

Simon Buck <sbuck@cgo.wave.ca>

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Chris and his coil

Date: Sun, 19 Oct 1997 09:13:10 GMT

Organization: The Energy Machine Information System 504-733-3462

From: thorn@northlink.com

Date: Sun, 19 Oct 1997 02:13:10 -0700 (MST)

Subject: Chris and his coil

Hello all,

I am presently making my first primary as well.  I have pondered the how

and why of how to mark and notch the "limbs" that hold the coil.  I have

found that since the coil is a spiral, for the conical type, the coil of

course will not be spaced properly if the notches on all the limbs are

exactly the same.  This will be difficult to explain without a picture, but

here goes an example.

Say you have eight limbs.  Say the coil spacing is 3/8".  The limbs must

be numbered 1-8 for simplicity.  Divide the 3/8" by the number of limbs,

which is eight.  You will get 3/64".  Keep the first limb exactly as it is

in length.  Limb #2, you will add 3/64" to its length.  Limb #3 will be

3/64" longer than #2, #4 will be 3/64" longer than #3,... and so on.  Soon,

all of your limbs will be 3/64" longer than the one before it, 1-8.  Now,

measure all of your slot locations starting from the outside edge of each

limb, so that each slot will have a 3/8" space between them, untill you

reach the inside edge of each limb.  You will see once you start winding the

coil itself, that the spiral will be clean from slot to slot, limb to limb.

I will be drawing this out pretty soon for everyone who has problems

determining how to arrange the slots to get that picture perfect coil every

time.  It sounds like a lot of work, but it is not really, and it does

provide for a very easy to wind and very smooth, curving coil.

Also remember, that all primary coils should be "tapped".  This means

that only the inner end is hard wired to your electronics.  The outer end of

the coil is "floating", meaning that it is not hard wired to the circuit.

Use a clip of your choice to connect to the coil anywhere along the spiral

you wish, in order to tune it.  The clip, of course, is hard wired into the

circuit.  I hope my words are understood and beneficial to all.

Robert

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Recycled: Primary Construction

Date: Sun, 19 Oct 1997 15:54:21 GMT

Organization: The Energy Machine Information System 504-733-3462

From: "Thomas McGahee" <tom_mcgahee@sigmais.com>

Date: Sun, 19 Oct 1997 11:54:21 -0400

Subject: Recycled: Primary Construction

Here is another installment of Recycled material. This installment covers the

CONSTRUCTION of Primaries.

Periodically I will recycle some of the more useful posts that I have collected

on various Tesla Coil related topics. Some of the members of this list may have

already seen some of these posts. They come from this Tesla-2 list and Chip's

Tesla List, and various archives. I will always preface the subject heading

with the word Recycled.

You might find this post a useful addition to your own personal

Tesla Collection.

Hope this helps.

Fr. Tom McGahee

Subject:  Re: Primaries and Copper Tubing

Date:   Fri, 18 Apr 1997 07:14:03 -0700

From:   DR.RESONANCE" <DR.RESONANCE@next-wave.net>

To:   "Tesla List" <tesla@pupman.com>

To: Brendan

When you buy the copper tubing fresh in the box --- don't try to straighten

it first.  Carefully remove the coil from the box and make slow and careful

bends gently as you fit it into your primary holders.  Don't force it and

try to slowly work at least 1 full turn ahead as you gently twist the

entire coil into a tighter spiral as you slowly work outward.  With this

technique you should be able to slowly work up a copper tubing spiral in

about 30-45 minutes for a single 6-10 turn primary.

Once you bend or straighten the tubing first it "work-hardens" and becomes

completely unable to make smooth bends.  If you have done this you will

have to scrap it out and start with a new roll.  Also, best to use K-soft

copper.  There is another type used for plumbing that work hardens much

faster and is much harder to work with.  Guess what --- the electrons won't

know the difference!!!

DR.RESONANCE@next-wave.net

*******************

Subject:  Re: Primaries and Copper Tubing

Date:   Fri, 18 Apr 1997 12:37:06 -0400 (EDT)

From:   Esondrmn@aol.com

To:   tesla@pupman.com

Brendan,

I wound my large primary with 3/8" soft copper (refrigeration) tubing.

I made a frame out of acrylic.  Then suspended the roll of copper on a

wooden shelf about three feet above the primary frame.  Slid a tubing bender

(looks like a spring) on the tubing and just started working it into place.

I would get about one half turn in place then spot tie each intersection of

tubing and acrylic frame support with waxed lacing cord.  Since I wanted 3/8"

spacing, I used a 3/8" drill bit to constantly check and set the spacing

between turns.  It did take about 4 hours and made my back sore, but

looks like a million bucks.

Ed Sonderman

******************

Subject:  Re: Primaries and Copper Tubing

Date:   Fri, 18 Apr 1997 05:01:47 -1100

From:   Ken Smith <ksmith@ihug.co.nz>

To:   Tesla List <tesla@pupman.com>

Brendan

Welcome to coiler's world. What follows is my best shot at primary

construction methods.  It isn't the best I am sure, but it works for me.

Hopefully there will be other replies and we shall all get something new,

since this is a bitch of a job at best and you are right about copper pipe -

it's a difficult material...

Making a Primary 101 - the Flatty.

Rule 1 :  You must have a template, your eyes are not good enough to keep

the spacing accurate.  As far as I can tell really accurate spacing is not

critical to the performance, but it looks wrong.  As a rule what looks good,

is and why risk the investment in time and money in making something that

may not be 110%.

My most recent flatty is rather tidy (IMHO) and it was built as follows.

However I would make changes if I make this one again. MY variations are in

[square brackets].

The base is made of a 36 inch square of 3/4 inch composite floorboard.

The template is a 8 legged star of 1/2 inch PVC pipe.  Now preping the pipe

is a sod.  To keep the coil onto the pipes I use a hot glue gun, but the hot

glue doen't stick at all well to the pipe, so my marking / template had to

incorperate a keying system to the pipe [next time I will use wooden

dowels - but I had the pipe anyway].

Marking up the pipe to privide an accurate keyway is an exercise in

patience.  I used 3/16 pipe with a 1/2 inch spacing between centers,

which gives me 5/16 air space beween copper edges. This seems a reasonable

figure and the numbers run out.  I wanted to make a larger primary than I would

need and as I wanted (by calculation) a 14 turn I went for a 21 turn out of

pipe limitation.  With a 7.5 inch center this would use two packs of pipe.

Figure out how to join the pipe with a small length of bigger pipe before

you start coiling it.

Now make up a drilling template.  My 1/2 inch spacing is easy here.

Calling the start leg 1, the next 7 legs have to increment the coil radius so

that when the center gets back to 1 you are 1/2inch further out.  Easy - 1/16

per leg.  The pipe is 3/16 OD so measure from an edge on some managable material

(I use scrap 120 mil capacitor sheeting) :

<-3/16->

| pipe  | <- 5/16 -> |       |

|   0   |            |   0   | = marks

----------------------------------------------

Mark it all out for leg 1 and then drill holes on the marks.  Now lay the

template onto leg 1 and pin it down starting at the point of start for the

required diameter (internal).  Drill suitable sized holes into the pipe to

fit plastic cocktail sticks.

Now leg 2.  Move the template up from the starting edge by 1/16 inch and fix

in place.  Repeat the drilling.

Repeat for all the rest.  The leg 7 should have a start 7/16 greater than

leg 1.

Push plastic cocktail sticks into all the holes.  Easy to say...

Now lay out the legs onto the baseboard and glue down.

You will now have a very good representation of an angry sea urchin on the

floor.  Find a table and put it on that (this stops back ache - since what

follows really takes a while)

Take your first coil of copper out of the packet and orientate it in the

same coil direction as your template on the table.  Drill a hole alongside

the start point of leg 1 and push through enough pipe to get the leader out

to the edge of the baseboard.  This is the inner feed.  I started out with

my feed on top of the board and it sparked over massively - underneath

there is enough clearance.

Now carefully work the pipe inside the vertical supports formed by the

cocktail sticks.  Get a couple in and then use the glue gun to glue it in

place.  Wait until the glue has hardened and then clip off the surplus bits

of stick.  I worked a couple at a time.  I would form the pipe (carefully,

using its natural curve to avoid kinking) a 1/2 circle at a time and then

glue down two at a time by putting a blob of glue onto the leg between the

sticks and then holding down the pipe.  Then I would pull a larger glue

cover over the pipe between the sticks.  In this way you proceed.  It is

slow but you get a neat strong coil. The trick is to secure as you go.

Otherwise it is impossible.

When you need more pipe, use the connection method previously established

and solder it before you get the pipe end anywhere near the formed coil or

you will melt the glue on the done bits !!!

When you get to the end.  Go make a coffee - you will have earnt it.

Finish off the fly end with one of those nice red plastic caps that came

with the new pipe.

When I make my next flatty (or indeed inclined) I will use the same method,

but if I go for heavier pipe I will probably use wooden dowel and those

cable ties that have the tag for attaching with a screw.  The marking up of

centers etc will be the same.

Hope this helps.  It isn't that comprehensive - but then no-one here has

never seen or tried a primary....  And hopefully this will be just one of

many ideas for this most awkward construction task...

Ken

\\\|///

\\  ~ ~  //

(  @ @  )

-----o00o-(_)-o00o-----

Ken Smith (ksmith@ihug.co.nz)

http://homepages.ihug.co.nz/~ksmith

.ooo0   0ooo.

-----(   )---(   )-----

\ ( AKL ) /

\_)NZ (_/

*********

Subject:  Copper tubing bending

Date:  Thu, 17 Apr 1997 11:01:20 -0600

From:  "Phillip M. Jarvis" <jarvisp@bee.d93.k12.id.us>

Organization:  Hillcrest High School Technology Department

To:   tesla@pupman.com

Brendan Haley writes:

"Also if anyone has found good ways to manipulate the

copper tubing without destroying it,  I would love to hear it."

Brendan:

I've found that a tubing bender from and automotive supply store does

real well with manipulating tubung. It is an item that costs about $2.50

and is small - fitting into the hand nicely.  It has a radius bend with

a tine that holds the tubing in place while pressure is applied. It

makes uniform bends without kinking.

Phil Jarvis

As an addendum to this post, let me say that tubing benders can be purchased at

most places that sell building supplies and copper tubing. They look like a big

metal spring that has a flared end. They are EXCELLENT devices, and really cut

down on the kinks!

*******************

Here's a post on how to form copper tubing into a "perfect" spiral. It is

sometimes useful to do this prior to mounting the copper tubing. Many coilers

skip such a step and just wind the copper tubing directly onto their form. The

technique outlined below takes a little more of your time, but results in a

no-kink primary coil.

Subscriber: wb8jkr@juno.com Wed Feb 12 23:03:10 1997

Date: Wed, 12 Feb 1997 08:50:05 EST

From: Mark S Graalman <wb8jkr@juno.com>

To: tesla@pupman.com

Subject: Re: Esthetically Pleasing Primaries

On Tue, 11 Feb 1997 23:25:06 -0700 Tesla List <tesla@poodle.pupman.com>

writes:

>Subscriber: SSNSanders@aol.com Tue Feb 11 23:13:46 1997

>Date: Tue, 11 Feb 1997 14:46:47 -0500 (EST)

>From: SSNSanders@aol.com

>To: tesla@pupman.com

>Subject: Re: Esthetically Pleasing Primaries

>

>In a message dated 97-02-11 01:44:23 EST, you write:

>

>

>I am the same way Tim and these guys make the primaries without a ripple

>which I find impossible. I dont understand it.   Stephen S.

>

Hi Tim, All,

The way I make decent looking primary coil is to build

what I call a "spool" I'll attempt to make a usable

discription on how to make one.

First, decide what size of conductor you will use, for

the sake of example, I'll pick .5" next, take a piece of

wood such as plywood that is a little larger in outside

dimension than the inside primary turn will be, I.E. if

your inside turn will be 10" in diameter get a piece of

wood about 15" square, make a small mark in the center of the board.

Now, get a dowel that has a circumference

equal to the diameter of the primary conductor, in this

case .5/3.14 or about .160" now temporarily mount this

dowel in the center of the 15" square board and tie a

string to the dowel tightly so it can't slip around it.

Tie a pen or a pencil to the string at a distance from the dowel equal

to the desired inside diameter of the first primary turn.

Carefully draw a circle around the board one full turn,

you will end up with a 1 turn spiral with an offset equal to

the diameter of you primary onductor. Draw a line between the

point where spiral starts and where it stops. Cut this spiral out

with a sabre or jig saw and drill a hole at the point where the

dowel was mounted. Cut a "slot" in it at the flat point

where you drew the line between the start and finish of the spiral, this

is where th primary conductor will be

inserted. Get two more pieces of wood about the same

diameter as the desired outside diameter of the primary

coil and mark and drill holes in the center of these as

well.

Next sandwich the "offset" circle or "spool" between these two

pieces of wood. Take the primary conductor and insert it into the

slot and turn the spool about 1 turn to lock the conductor onto it,

then while pulling tension on the primary conductor rotate the

spool slowly wrapping the conductor around the  offset

center, ALWAYS under tension when the desired amount of

turns have been wrapped release the tension and take the

center bolt out and open up the spool assembly, you'll

have a PERFECT spiral primary!

Mark Graalman

*******************

Another way to make kink-less primaries.

Subscriber: tesla@america.com Wed Feb 12 23:10:36 1997

Date: Wed, 12 Feb 1997 07:24:46 -0500 (EST)

From: Bob Schumann <tesla@america.com>

To: tesla@pupman.com

Subject: Re: Esthetically Pleasing Primaries

>Subscriber: maleske@worldchat.com Mon Feb 10 22:10:57 1997

>Date: Mon, 10 Feb 1997 10:33:41 -0500

>From: Tim Maleske <maleske@worldchat.com>

>To: Tesla List <tesla@pupman.com>, Usa-Tesla List <usa-tesla@usa.net>

>Subject: Esthetically Pleasing Primaries

>

>

>How do you guys keep your primaries looking so nice. All the pictures I

>have seen of primary's from copper tubing they are nice and flat and evenly

>spaced out. What about the spiral type primaries? They all look so

>esthetically pleasing.

>

>

>

Tim,

This one hurts for me to think about but now that I have been

there done that, I know exactly how I would do it again. When I got

my copper tubing, it came it the box sprialed down to the center of

the box and then back out again. I wanted the pipe to just be one spiral

not 2. I found a cylindrical item that was the same size (or close) to

the diameter of my primary coil form (mentioned later). I found a large

coffee can would work. The first turn of the pipe around the bottom of

the coffee can was the toughest. From then on I wound all of the copper

pipe outward in a flat pancake (Archemedes) and wrapped the tubing on

itself. I did have someone helping me to keep the pipe flat on the floor

through each turn. This made a tight coil with no kinks. When I got to

the end, I just let the end loose, there was some unwinding but not much

and pulled the coffee can out. The pipe was ready. I made 8 fin-type

primary forms (There was a hole spacing trick to this, glad the group

was here!) and I made my holes so that the pipe would 'snap in'. I do

not have ant straps holding my primary to the forms. When it came time

to install the pipe, I started from the inside out (the pipe that was

closest to the coffee can) and started snapping it in. The thing was

that it is easier to 'open up' the pipe winds that it is to make them

tighter. It was a brease from there.!

Bob Schumann

http://www.america.com/~tesla

**************

Wouldn't be complete without me putting one of my *own* posts in here...

----------

> From: Tim Maleske <maleske@WORLDCHAT.COM>

> To: Multiple recipients of list USA-TESLA <USA-TESLA@USA.NET>

> Subject: Esthetically Pleasing Primaries

> Date: Monday, February 10, 1997 10:33 AM

>

> How do you guys keep your primaries looking so nice. All the pictures I

> have seen of primary's from copper tubing they are nice and flat and evenly

> spaced out. I tried running mine through hose for insulation, but this did

> not work well. (Boy is it tough to pull 40 feet of 1\4" copper through a

> 5\16 I.D. tube !!!) What about the spiral type primaries? They all look so

> esthetically pleasing.

>

Tim and Others,

There are several ways to ensure that copper tubing primaries look nice.

Here's one that is fairly easy:

Let's assume we want to make a ten turn primary using 3/8" copper tubing

and 3/8" spacing between turns. Let's further assume we want a starting

diameter of 5" to allow a 3" coil and 1" around it before the primary

starts so we can use a separate RF ground and keep primary and secondary

insulated from one another.

That means that from the inside "side" of the copper tubing to the *next*

inside "side" of the spiral will be 6/8". So for ten full turns it will be

60/8 of an inch *plus* the width of the tubing (since we measured from

inside to inside...) for a total of 63/8 of an inch which is 7 7/8 inches.

Allow an inch extra for the inside and an extra inch for the outside and

you have 9 7/8 inches.

Cut three pieces of Lexan or Plexiglass or other GOOD insulating material

to a size of  1+3/8+1  for a total height of 2 3/8 inch and a length of 9

7/8 inch. I prefer 1/2" thick Plexiglass or other plastic for maximum

strength.

I make a drilling guide that is 2 3/8" by 9 7/8" out of thin cardboard or

poster board like material. Draw a line lengthwise down the middle of this

guide. Start at a point 1 3/16" in from one side, and put a mark there

along the center line. Now, from that point mark off nine additional

points, all 3/4" apart. You should now have ten points marked off

altogether. From the centerline draw two parallel lines that are 1/2" away

from the centerline (one above, one below the centerline). Mark off ten

points along each of these two lines, the same as the markings for the

centerline.

Using this carboard guide, place it over one of the plexiglass pieces and

use an awl to transfer the (30) marks to the plastic. Using the awl,

lightly scribe or draw a line through the centerline markings to aid you in

cutting later. Repeat for all three pieces of plastic. Now drill the

outside holes with a 1/4" drill bit, and the inner centerline holes with a

3/8" bit. Repeat for all three pieces of plastic. Now, using the centerline

we drew or scribed earlier, use a jigsaw or bandsaw to cut each of the

three pieces in half lengthwise. You will now have Six identical plastic

pieces. The side with the "C" shaped cuts is the top.

Locate the center of the "board" that you will be mounting the the primary

onto. Mark it. draw a 3" diameter circle around this point. That represents

the 3" secondary. Draw a set of six straight lines radiating from the

center out to the edges of the "board". Number these radial lines 1 through

6. For line #1 make a mark 1" outward from the 3" diameter circle. Now,

since the distance between the inside of one turn and the inside of the

next is 3/4" and we have 6 pieces, then each next # radial will have the

position of the previous plus 3/(4*6)", which comes out to be 3/24 which is

1/8". So #1 is 1" from the 3" diameter circle, #2 is 1 1/8"  #3 is 1 2/8"

#4 is 1 3/8"  #5 is 1 4/8  #6 is 1 5/8. Mark each of these points along

their respective radial lines. These points determine where the inside of

each plastic primary support begins. Using whatever means you choose, mount

all six primary supports, using the radial lines and guide points to

determine how they align. Avoid any kind of fastening device or screw that

is not either copper or platic or other insulating material such as nylon.

Various glues may be used. Once the supports have been mounted (and are

determined to be very sturdy), then we can begin with the copper tubing.

Clean it well, now, before it is coiled, because once it is mounted you

won't be able to clean it easily. The inside of the primary is going to

begin at radial #1. You have to determine whether you want to extend the

copper tubing through the top plate, solder a wire to the tubing, or anchor

the tubing to the support and connect power later via a copper bolt.

If you are going to use the bolt approach, then use a hammer to flatten out

the end of the copper tubing. Then drill or punch a hole that is 1/4" into

the flat side and use a 1/4" copper bolt and copper nut to tighten the

tubing down through the 1/4" hole just below the "C" on radial #1. If using

any other method of electrical connection, then you can drill a 3/8" hole

next to the side of the #1 radial so that the tubing can come up through

the hole and then bend at a 90 degree angle so that it passes through the

"C".

Now route the tubing to the "C" on the next radial. When it is in place,

use a tie-wrap (all nylon or other plastic... NO METAL) to secure the

tubing to the "C" mounting position. The tie-wrap goes around the tubing

and through the 1/4" hole. Be CONSISTENT in the way you run the tie-wraps.

I suggest that at the beginning you NOT pull the tie wraps totally tight.

Just make it snug for now, but don't cut off the tail of the tie wrap until

we are all done. This allows us to jiggle the tubing a little to keep

things smooth. Avoid making tight bends over short sections of tubing. It

is better to make a wide bend and then keep narrowing it down by working on

it with your fingers gently then to make a kink. Form about a foot of the

tubing and then work it in so that it fits the "C" mount. As each one

mounts, tie it down gently. The last turn can be secured in the same manner

as the first, or by any other means that you find appropriate.

Once you have everything exactly the way you want it, you can pull all

tie-wraps as tight as you can, and cut off the excess tail.

Modify the method as desired.

Some people use "V" cuts instead of "C" cuts. Some people use insulating

pegs placed into drilled holes. Some people just use a baseboard with holes

for the tiewraps, and tie it down flush with the baseboard. The key is to

have each radial section *offset* from the next by 1/6th the distance from

the center of one winding to the center of the next. Personally I like a

"C" holder because it matches the shape of the tubing the best and is very

consistent.

By the way, I like to make my secondary coils with MORE than 1" of unwound

distance on the bottom. This is to compensate for the height of the primary

coil so we don't get over-coupled. I also usually glue a Copper sheet to

the bottom of my secondaries and run a 1/2" wide Tab of this same copper up

the side so I have a place to solder the end of the secondary. I have a

matching copper plate that is 2 inches in diameter glued to the baseboard,

right in the middle of the 3" circle. This gives me a good RF ground

connection, as the other side of the plate has a 2" wide copper strip

connected to it and exiting through a slot at the center of the baseboard.

This 2" wide copper strip leads to my external Good RF Ground Connector.

To prepare a secondary wire for soldering to the tab from the bottom

grounding plate you can *gently* scrape the insulation off using a razor

blade, or sand it off with a very fine sandpaper (Stroke only towards the

cut end, not back and forth!). I don't know of any solvent that can be used

to strip the insulation off. If using a thicker wire, such as a size #22,

you can also carefully flatten the wire and then solder. If you have never

done this kind of thing before, then FIRST EXPERIMENT USING WIRE SCRAPS.

Only attempt the real thing once you are confident you can do it right.

After soldering the wire I apply several coats of varnish to prevent the

wire from getting snagged later.

I normally have the top of my coil done up in a similar fashion with a

copper plate on the top. This allows you to set a toroid on it and get an

instant good connection.

Hope this helps you with the construction details. Read them, try to

understand them, and then modify as desired.

Fr. Tom McGahee

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Recycled: Primary Shapes and Formulas

Date: Sun, 19 Oct 1997 16:44:23 GMT

Organization: The Energy Machine Information System 504-733-3462

From: "Thomas McGahee" <tom_mcgahee@sigmais.com>

Date: Sun, 19 Oct 1997 12:44:23 -0400

Subject: Recycled: Primary Shapes and Formulas

This first set of Recycled posts will deal with the various SHAPES of primaries.

Some theory, some formulas, and some experimental data.

Periodically I will recycle some of the more useful posts that I have collected

on various Tesla Coil related topics. Some of the members of this list may have

already seen some of these posts. They come from this Tesla-2 list and Chip's

Tesla List, and various archives. I will always preface the subject heading

with the word Recycled.

You might find this post a useful addition to your own personal

Tesla Collection.

Hope this helps.

Fr. Tom McGahee

**********

We begin with the basic question: What Primary Shape is BEST...

>

>I'm new to Tesla Coils and I'm sure my question will point that out.

>Can someone explain why in the primary coil you would have it spiral up the

>secondary coil at a 30 degree angle.  Why not a 75 or 90 degree angle.

>

>Thanks

>

>Peter

>

>Peter,

It makes no difference why 30 degrees is used. it is a matter of coupling

for a desired purpose.  You can use anything from 0 to 90 degrees if you

know what you are doing.  There are no magic angles.

I like 0 degrees for small systems and 30 degrees for big 2 coil systems and

90 degrees for magnifier drivers.

Richard Hull, TCBOR

********

Here is another response to Peter's question...

Peter,

Welcome to the wonderful world of coiling.  The primary can spiral at any

angle from 0 degrees to 90 degrees.  A 0 degree angle pri. is called a flat

spiral or pancake pri. and is a popular type.  The 90 degree or cylinder pri.

is less popular because it may over-couple if it's too small in dia., and

because sparks may jump from the top of the secondary down to the top of the

primary.  Yet, many fine systems use the cylinder primary.  The 30 degree (or

15, 20, 35, degree, ad nauseum) primary is a compromise that has some of the

attributes of the flat and the cylinder primaries.  To a large degree, the

primary angle is a matter of practical preference, what really matters is the

ultimate coupling or k factor that results.  I personally use the 20 degree

primary in most cases, but in small tube coils, I often use the cylinder

primary because in general tube coils can withstand tighter coupling without

the specter of racing sparks.  In general, the exact angle will have little

effect on the coil's operation.  If the angle is too great, and causes racing

sparks to appear, then just raise the secondary a little to stop them.

John Freau

From: Bert Hickman <bert.hickman@aquila.com>

Date: Mon, 10 Feb 1997 19:40:27 -0800

Subject: Re: Esthetically Pleasing Primaries

Tesla-2 wrote:

>

> From: Nathan_Bonsal@intuit.com (Nathan Bonsal)

> Date: Mon, 10 Feb 1997 12:05:29 -0800

> Subject: Re: Esthetically Pleasing Primaries

>

>

>      I have a question- may be far out but here it is- What benefits do

>      flat spirals have over the helical primary- It is my understanding

>      that in the flat coil the exitation is in the bottom fourth of the

>      secondary while the helical type couples more intensely. However,

>      Exciting the secondary easily leads to overcoupling. Tesla used

>      helical coils on all of his Magnifiers.

>      Here's a what-if-

>         what if instead of either a helix or a cone or a flat coil you

>      instead used a bowl shape halfway between cone and flat? how would

>      that couple?

>         I know that a thirty degree cone shape can be used to get the

>      desirable characteristics of both helix and flat coils without

>      overcoupling, but would any advantage be reached by using a partial

>      dish shape? If you have any insight on this please let me know.

>

> ______________________________ Reply Separator ________________________________

>

> Subject: Esthetically Pleasing Primaries

> Author:  tesla-2@emachine.com (Tesla-2) at Internet

> Date:    2/10/97 3:30 PM

>

> From: "Tim Maleske" <maleske@worldchat.com>

> Date: Mon, 10 Feb 1997 10:30:55 -0500

> Subject: Esthetically Pleasing Primaries

>

> How do you guys keep your primaries looking so nice. All the pictures I

> have seen of primary's from copper tubing they are nice and flat and evenly

> spaced out. I tried running mine through hose for insulation, but this did

> not work well. (Boy is it tough to pull 40 feet of 1\4" copper through a

> 5\16 I.D. tube !!!) What about the SP[iral type primaries? they all look so

> esthetically pleasing.

>

Nathan,

Excellent question!

You can make a Tesla Coil work with any of these configurations. Most

vacuum tube coils use helical primaries. However, since the voltage

rises as you go up the secondary, as you increase power levels, you'll

rapidly reach the point where secondary-to-primary arc-overs start

occuring. By moving the higher primary turns outward, you'll end up with

an "inverse conical" coil - a hybrid between a pancake and a helical

coil. This reduces the problems of secondary-to-primary arc-overs. BTW,

there's nothing particularly "magical" about 30 degrees - a 20 or 45

degree primary will also work.

An "inverse conical" primary is often used on lower power coils.

However, as you further increase power, you'll now begin to get arcovers

from the top of the coil or the toroid to the top of the primary (or to

a protective strike-ring). This is why higher power coils tend to go

with pancake primaries - to provide adequate clearance for the very high

voltages they produce. A higher power coil may easily generate

arc-lengths that are 2-3X the length of the secondary winding.

There's one other advantage with the inverse conical coil -  you can

change the inductance tap without having a major impact on the coupling

coefficient. However, for the same coupling coefficients, either style

will transfer energy to the secondary as effectively.  The pancake

simply gives you more voltage handling capability.

Safe coilin' to you, Nathan!

-- Bert --

*********

A Wonderful post based on a set of experiments by Gary Weaver...

----------

> From: Tesla List <tesla@pupman.com>

> To: tesla@poodle.pupman.com

> Subject: Best Primary Coil

> Date: Tuesday, April 22, 1997 12:23 AM

>

> Subject:  Best Primary Coil

>   Date:   Mon, 21 Apr 1997 13:49:29 -0700

>   From:   Gary Weaver <gweaver@earthlink.net>

>     To:   tesla@pupman.com

>

>

> SEARCH FOR THE BEST PRIMARY COIL

>

> For several months I have been building and testing primary coils.

> Without going into a lot of details and turning this into a 5

> meg file that will take you 2 hours to read I will list only what is

> important.  Each primary coil was tested using the same power

> supply, capacitor, spark gap, secondary coil and toriod.  Then all the

> primary coils were tested again and only one thing in the

> circuit was changes.  Example, in the 2nd test each primary coil was

> tested using secondary coil #1, then repeated using

> secondary coil #2, then secondary coil #3 and so on.  Then each primary

> coil was tested a 3rd time using several different

> spark gaps.  Then each primary coil was tested a 4th time and only the

> toriods were changed.  Then each primary coil was

> tested a 5th time and the power supply was changed.  The goal is to find

> out what works best and gives maximum secondary

> coil spark output.

>

>

> CIRCUIT.  The spark gap is connected in parallel with the power supply.

> The LC circuit is connected in parallel with the

> spark gap.  1 safety gap is connected in parallel with the power

> supply.  2 safety gaps are connected from each high voltage

> terminal to ground.  A choke coil is connected between each high voltage

> terminal and the spark gap.  The secondary coil has

> a good earth gound.

>

>

> SPARK GAPS   I tested each coil using several different spark gaps.  I

> have found that the Richard Quick variable speed

> vacuum fan quenches the spark gap the best.

>

> Richard Quick spark gap. Copper tubes 1"x2". 8 gaps are .025 each.

> Muffin fan cooled.

> Richard Quick spark gap. Copper tubes 1"x2". 8 gaps are .030 each.

> Muffin fan cooled.

> Richard Quick spark gap. 10 tungston carbide gaps spaced .025 each.

> Muffin fan cooled.

> Richard Quick spark gap. Copper tubes 1"x2". 8 gaps are .025 each.

> Vacuum fan cooled.

> Richard Quick spark gap. Copper tubes 1"x2" water filled. 8 gaps are

> .025 each. Muffin fan.

>

>

> POWER SUPPLY   I tested each primary coils with 1 neon then retested

> them with 2 neons in parallel then retested them a

> 3rd time with 3 neons in parallel and so on.  All neons are in phase and

> voltage matched within a few volts.

>

> 12,000. volts 30 ma

> 12,000. volts 60 ma

> 12,000. volts 90 ma

> 15,000. volts 30 ma

> 15,000. volts 60 ma

> 15,000. volts 90 ma

> 15,000. volts 120 ma

> 15,000. volts 150 ma

> 18,000 volts 1 amp with current limiting.

>

> CAPACITORS  I tested each pirmary coil one at a time then added more

> capacitance in the LC circuit and retested each

> primary.  Each capacitor is rolled .010 aluminum flashing and 18 layers

> of .006 mil polyethylene.  Each capacitor is vacuum

> pumped and back filled with high voltage transformer oil.  Each

> capacitor was tested with a capacitor tester for the exact

> value.

>

> .01 uf rated 40,000. volts

> .01 uf rated 40,000. volts

> .0014 uf rated 40,000. volts

>

>

> 3/8" TUBING PRIMARY FLAT WOUND.  The coils tested here are all made with

> 3/8" copper tubing.  All coils are flat

> wound.  The only thing that changes in this test is the spacing between

> turns.  I made coils with .250, .375, .500 and .625

> inches between turns.  The inside diameter of all the coils are the

> same.  All coils have 16 turns.  I tested each primary coil one

> at a time and did not change any thing else in the circuit.  Then I

> retested the primary coils changing things one at a time.  In

> every test the tighter wound primary coil works the best.  The .500

> coils works better than the .625 coil.  The .375 coil works

> better than the .500  or the .625 coil. The .250 coil works better than

> all the other coils.  The tight wound coil is the best.

>

>

> 1/4" TUBING PRIMARY FLAT WOUND.  The coils tested here are all made with

> 1/4" copper tubing.  All coils are flat

> wound.  The only thing that changes in this test is the spacing between

> turns.  I made coils with .250, .375, .500 and .625

> inches between turns.  The inside diameter of all the coils are the

> same.  All coils have 16 turns.  I tested each primary coil one

> at a time and did not change any thing else in the circuit.  Then I

> retested the primary coils changing things one at a time.  In

> every test the tighter wound primary coil works the best.  The .500

> coils works better than the .625 coil.  The .375 coil works

> better than the .500  or the .625 coil. The .250 coil works better than

> all the other coils.  Again the tight wound coil is the best.

>

>

> 1/4" and 3/8" COPPER TUBING.  In this test I compaired the best of the

> best.  The best 1/4" tubing primary is spaced .250

> between turns.  The best 3/8" tubing pirmary is spaced .250 between

> turns.  In all the tests I did the 1/4" copper tubing

> primary out performed the 3/8" copper tubing primary coil.   Center to

> center each turn of the 1/4" tubing is .500" and the 3/8"

> tubing is .625" center to center.   The 1/4" copper tubing primary is

> wound tighter than the 3/8" copper tubing primary.  Again

> the tight wound coil proves to be the best.

>

>

> 1/4" and 3/8" TUBING PRIMARY 30 DEGREE ANGLE.  Now that I know the tight

> wound primary coils work the best I

> built and tested 2 coils helial wound on a 30 degree angle.  One coil is

> made with 1/4" copper tubing .250 between tubes.  The

> other primary is made with 3/8" copper tubing .250 between tubes.  The

> coil wound with the 1/4" copper tubing works better

> than the one wound with the 3/8" tubing.  Even though the 1/4" tubing

> coil works the best the coupling difference here is not as

> much as it was with the 2 flat wound primary coils.

>

>

> 1/4" TUBING PRIMARY FLAT, 15 AND 30 DEGREE ANGLE.  Now that I know the

> tight wound primary made with

> 1/4" copper tubing works the best I built and tested a coil helical

> wound on a 15 degree angle.  I compaired this coil to the

> 1/4" flat wound coil and the 1/4" 30 degree helical coil.  The 15 degree

> angle coil couples much better than the flat wound coil.

> The 30 degree coil couples a little better than the 15 degree coil.

> There is a very big coupling difference between the flat

> primary and the 15 degree primary coil.  In all my tests the secondary

> coil output is much better using the 15 degree primary

> compaired to the flat primary coil.   When compairing the 15 deg coil to

> the 30 degree coil the 30 degree coil coupling is a little

> better but has some disadvantages.   I am getting lots of strikes from

> the toriod to the strike rail with the 30 degree primary.

> With the 16" diameter toroid 21" above the strike rail the sparks prefer

> to travel out and curve around and come back a!

> nd hit the strike rail traveling

>  a distance of 34".  Even with a 27" toroid on the 6.625" dia. x 23"

> tall secondary coil the sparks from the 27" toroid go out

> and curve around and come back and hit the strike rail of the 30 degree

> coil.  I am having a hard time getting any long sparks

> from the toroid because they all want to strike the 30 degree primary

> coil or the strike rail.  Also the 30 degree primary is

> causing me some over coupling problems but the 15 degree primary is on

> the border line of over coupled.  Raising the

> secondary coil up several inches above the 30 degree primary coil solved

> some of the problems.   I made several atempts to

> raise the toroid up several inches above the secondary coil to stop

> arcing to the strike rail of the 30 degree coil but now the

> arcs are coming from the top turn of the secondary coil not the toriod.

> At 15K 150 ma I get only a few strikes to the 15

> degree primary strike rail using a small aluminum foil covered toroid

> but have gotten lots of strikes to the 30 degree pr!

> imary coil and strike rail.!

>  I redu

>  ma and did not receive any strikes to the 15 degree primary or strike

> rail but I am still getting a few strikes to the 30 degree

> primary coil and the strike rail.  With the power reduced to 15K or 12K

> 30 ma I got no strikes to the 30 degree primary coil

> or strike rail. The 15 degree primary is giving me some very long

> secondary coil sparks at any power setting I choose.  The 15

> degree primary coil does fine with any of the 4" or 6" secondary coils.

> The 15 degree primary coil does fine with any toroid I

> choose to use with the secondary coil.  The 15 degree primary coil is

> clearly the best choice within this power range.

>

>

> STRIKE RAIL   I made the strike rail using 1 turn of 1/4" copper

> tubing.  The strike rail is 2" above the primary coil and 1/2"

> short of being a complete turn.  The strike rail is connected to the

> earth ground along with the bottom end of the secondary

> coil.  I was surprised to find that the strike rail some how increases

> primary secondary coil coupling and after readjusting the

> coupling the secondary coil spark discharge length has increased by 1".

> With the 30 degree primary coil a strike rail must be

> used to protect the neons from damage.  I am getting lots of sparks from

> the toroid to the strike rail and primary coil of the 30

> degree primary.  Even with the strike rail on the 30 degree primary I

> lost 2 neon transformers.  With the 15 degree primary coil

> a strike rail is a much needed safety device.  I received only a few

> strikes from the toriod to the strike rail of the 15 degree

> primary coil and no strikes to the primary coil.  I did not receive any

> strikes from the toroi!

> d to the flat primary coil.

>

>

> ARC OVER   The bottom end of the secondary coil must be kept a safe

> distance away from the inside turn of the primary coil

> to prevent arc over.  Arc over can short out your neons.  I have lost 1

> neon from arc over.  Using a .01 uf capacitor in the LC

> circuit and a 12,000. volt 30 ma neon, the primary coil secondary coil

> spacing should be .500". apart between the inside turn

> of the primary coil and the 1st turn of the secondary coil.  Using 12K

> 60 ma the 2 coils should be 1.125" apart.  Using 12K 90

> ma the 2 coils should be 1.750" apart.  If the LC circuit capacitor is

> increased from .01 uf to .02 uf the spacing between the

> coils should be increased by .250".  If the 12K neons are increased to

> 15K  the spacing between the 2 coils should be

> increased by .125".  These tests were done with a 27" toroid.  If the

> toroid diameter is reduced from 27" to 16" diameter then

> the space between the 2 coils can be reduced by 1/8".

>

>

> PRIMARY TURN TO TURN ARCING.  In all the tests I did I never had any

> problems with primary turn to turn arcs.  With

> the Tesla Coil maxed out to its peak and using the 15 degree primary

> with 1/4" copper tubing spaced 1/4" between turns

> tapped at turn 15.75,  secondary coil 6.625" dia. x 23" long wound with

> # 24 wire,  12K 90 ma power supply, .0214 uf

> capacitor, 10" x 27" diameter toroid and a Richard Quick variable speed

> vacuum spark gap with a total gap of .200,  I got

> only 1 turn to turn spark on the primary.  1 spark is not a problem as

> far as I can see.  It was not a spark that acred over and

> over or a long time, it was just one spark that lasted a micro second

> and was never seen again.  This setup got me lots of 51"

> long spark from the toroid.  Maximum spark length so far is 66".

>

>

> VACUUM FAN SPARK GAP.  While doing these tests I discovered that the

> Richard Quick varable speed vacuum fan

> spark gap works the best.  I am using a high speed vacuum fan that I

> bought from American Sicence Surplus.  It has an

> AC/DC type motor.  I am using a variac to adjust the RPM's of the fan. I

> have found that as the fan RPM's increase the

> secondary coil spark output also increases.  There is a limit where the

> tesla coil peaks out at a certain fan RPM.  If the fan

> RPM's are increased past this peak point the spark gap is over quenched

> and the secondary coil output sparks get shorter.

> The variac makes it possible for me to quench the gap to maximum

> performance.  When running the TC on 12K or 15K 30

> ma I am using 4 of the 8 gaps.  If I increase the power to 12K or 15K 60

> ma I am using 6 of the 8 gaps, the primary

> secondary coil coupling has to be adjusted and the vacuum fan RPM's has

> to be increased.  If I increase the power again to

> 12K or 15K 90 ma I am using all 8 of the !

> gaps, the primary secondary coil

>  coupling has to be adjust again, and the vacuum fan RPM's has to be

> increased again.

>

>

> TOROIDS  I tested several toroids.  A  6" copper sphere,  5" x 12" dia.

> stainless steel toroid, Basket Ball covered with

> aluminum foil,  4.5" dia. x 16" dryer hose covered with aluminum foil,

> 10" galvinazed elbows 4 in a circle 27" diameter, and

> NO toriod.  The 10" thick x 27" diameter toroid gave me the longest

> spark output of 66" with 3 neons in parallel.  Using the

> 4.5" x 16" diameter aluminum foil covered toroid I get about 6 or 7

> splitting streamers each about 32" to 37" long.  With all the

> other toroids I had no trouble getting several splitting streamer sparks

> about 36" long.   A large smooth surface toriod with a

> bump gives me 1 very long discharge spark.  A ruff surface toroid like a

> dryer hose and basket ball covered with aluminum foil

> gives me lots of  long splitting streamers.

>

>

> SAFETY GAPS & CHOKE COILS.  With the Tesla Coil fully maxed to the limit

> the safety gap and choke coils don't seem

> to prevent neons damage.   I have never lost a neon running a Tesla

> Coil  with just 1 neon transformer but when running 3

> neons in parallel its just a matter of time and 1 or more neons will get

> fried.  From what I see here the number one cause of

> neon damage is arc over and primary coil strikes from the toroid.  I

> have lost 7 neons so far.

>

>

> SECONDARY COILS.  I tested several secondary coils with all the primary

> coils.  All the coils are close wound on PVC

> pipe and coated with polyurethane.  The larger diameter coils require

> more power input than the small diameter coils.

> Operating the Tesla Coil on just 1 neon of 12K or 15K 30 ma coil #3

> produces the longest and hottest sparks.  When

> operating the Tesla Coil with 3 neons in parallel coil #7 produced the

> longest and hottest sparks.  I am sure this can be

> improved by winding a secondary coil with larger diameter wire.

>

> #1.......3.5" dia. x 15" long #28 wire.

> #2.......3.5" dia. x 15" long #24 wire.

> #3.......4.5" dia. x 11.5" long #24 wire 266 turns 2 wires wound side by

> side connected in parallel.

> #4.......4.5" dia. x 11.5" long #24 wire 532 turns.

> #5.......4.5" dia. x 13" long #24 wire.

> #6.......4.5" dia. x 21" long #24 wire.

> #7.......6.625" dia. x 23" long #24 wire 1064 turns.

> #8.......6.875" dia. x 37" long #24 wire.

>

>

> THE FINAL TEST.  I am using the 15 degree helical wound primary coil,

> secondary coil #7, .0214 uf capacitor, 10" x 27"

> toroid, Richard Quick variable speed vacuum spark gap, 18,000. volt 1

> amp transformer, and a current limiting transformer.

> The primary secondary coil arc over space is 2".  At 2000 and 3000 watts

> there are no strikes from the toroid to the strike

> rail, the vacuum spark gap is doing very well and I am getting very

> thick white hot sparks from the toroid.  At 4000 watts I am

> getting very thick white hot sparks about 7' long from the toroid using

> a #16 nail stuck to a magnet on the side of the toroid.

> Some of the sparks go out from the toroid 3' curve around and come back

> and hit the strike rail traveling a distance of 6'.  I

> moved the nail up higher over the radius of the toroid side so it points

> up at an angle of about 15 degrees and now there are no

> more arcs to the strike rail.  At 4800 watts I am getting very thick

> white hot 8' sparks from the nail.  By mov!

> ing the nail to different locati

> on on the toroid I can direct the sparks at any thing I want.  I removed

> the nail from the toroid and now I am getting 100's of

> random white hot thick sparks in all directions around the toroid to the

> floor, walls, ceiling, strike rail, bicycles, lawn mover,

> garden tiller, mini bike, work bench, light fixture, tool box, desk, air

> compressor, garage door and everything in the room.

> With the nail I can direct all the sparks in one direction but with out

> the nail I have no control over the sparks.  I have run the

> TC several times at 4800 watts for 30 to 45 seconds each time with the

> Richard Quick variable speed vacuum spark gap set

> at about 3/4 full speed and it is doing very well, much better than I

> expected.  I will have to move the TC outside to try it on

> 10,000. watts.  I got no primary secondary coil arc over and no primary

> coil turn to turn arcing.  All the discharge sparks are

> very powerful and white hot.

>

>

> CONCULSION.  I can clearly see that a tight wound primary coil works

> best.  For the power range that I am experement

> with the 15 degree helical wound primary is my 1st choice.  1/4" copper

> tubing has proven to be better than 3/8" copper

> tubing in these tests.  The Richard Quick variable speed vacuum spark

> gap out performed all the other spark gaps I tested and

> performed well up to 4800 watts.  A large smooth toroid with a bump

> produces a long sparks and a ruff surface toroid or

> smooth surface toroid with no bump produces lots of uncontrolable

> sparks.  The .02 uf capacitor produces longer and hotter

> discharges sparks from the toroid than the .01 uf capacitor in the LC

> circuit.  Secondary coil #7 produces the longest and

> hottest sparks from 1000 watts to 4800 watts.  The 30 degree tight wound

> primary coil is a fine choice for a 500 watt Tesla

> Coil.  The 15 degree tight wound primary is the best choice for a Tesla

> Coil 1000 watts to 4800 watts.

>

>

> DRAWING.  I have attached a drawing to this post of the 15 degree

> primary coil that I built.

>

>

> SPECIALIST & GENERALIST.  A specialist learns more and more about less

> and less until he knows everything about

> nothing.  A generalist learns less and less about more and more until he

> know nothing about everything.

>

>

> Gary Weaver

**************

Here's one from our own Mike Hammer...

From: Mike Hammer <mhammer@misslink.net>

Date: Sun, 28 Sep 1997 12:20:26 -0500

Subject: Re: Primary vs. Performance

At 04:50 PM 9/27/97 GMT, you wrote:

>From: Richard Smit <esmit@ilink.nis.za>

>Date: Sat, 27 Sep 1997 18:50:19 +0200 (SAT)

>Subject: Primary vs. Performance

>

>Hi,

>

>I have seen a lot of guys Tesla coils with a 30 to 15 degree primary. All of

>their coils seems to perform great.

>

>IS THERE ANY EFFECT ON WHAT SORT OF PRIMARY IS BEST WITH SOME TRANSFORMERS?

>

No Richard there really isnt. In my opinion no one primary shape

is really any better than any other. Inductance is inductance.

Some primary shapes may have desirable characteristics for certain

coils but all of them will probably perform similar once you

make allowance for the difference in coupling.

And that is the basic difference between the various primary

coil shapes - coupling.

The solenoid shaped primary has the highest degree of coupling while

a flat spiral has the lowest. The 15-30 degree saucer shaped primary

is somewhere in between.

The main reason for using the saucer shaped primary is to prevent

problems with over coupling. On high power coils where the saucer

primary still over couples then the flat spiral becomes attractive.

Always try to keep as much primary inductance tapped in as possible.

Higher primary inductance helps improve gap operation. This is especially

important if your gap isnt so great.

The great performing coils you spoke of dont get their great performance

solely on a particular primary shape. Indeed this kind of performance

isnt due to any single component. Its due to the coil system as

a whole being well balanced and having all the individual components

well matched for the task at hand.

Beginners are often looking for that magic bullet. Just that

one secret or special design of some component that will give

them great performance.

It takes improving each and every component in your coil trying

to remove the bottlenecks. When one bottleneck is removed another

takes its place.

TTYL!

Mike Hammer

mhammer@misslink.net

*************

From:   DR.RESONANCE[SMTP:DR.RESONANCE@next-wave.net]

Sent:   Sunday, August 10, 1997 12:38 PM

To:     Tesla List

Subject:        Re: primary coil windings

To: Kevin

Almost any geometry will work but the trick is to set the coefficient of

coupling between the pri and sec inductors at a value of between 0.18 and

0.22.  Try 0.2 as a starting point.  This is achieved by raising your sec

coil with some scrap plywood about 1/4 inch at a time until you obtain best

spark output.

Many coils in the 8" dia and above seem to operate best with a flat spiral

primary.  Smaller coils in the 2-7 inch dia range seem to perform well with

a short solenoid type winding.  Keep the primary coilform dia at least 2.5

to 2.8 times the sec. dia to avoid overcoupling.

Tune for best spark output and enjoy!

DR.RESONANCE@next-wave.net

***********

Let's Recycle a classic post by Richard Quick...

Subject:  Primary Construction

Date: Thu, 7 Dec 1995 04:20:00 GMT

From: Richard Quick <richard.quick@SLUG.ORG>

Reply-To: USA-TESLA@USA.NET

Organization:St. Louis Users Group

To: Multiple recipients of list USA-TESLA <USA-TESLA@USA.NET>

Quoting "Lima, David" <dlima@analogic.com>:

> Thus far I have not been able to understand the benefits and

> differences between cylindrical, flat ,and saucer shaped primaries,

> Do you have any FAQs or text on this? Or if not maybe suggest a good

> book?

Man, until I write mine there is simply no real good books out there.

I have a rather extensive library on the subject and I can find a

section in one book, a chapter in another, but not one single refer-

ence that has any real dirt on primary coil designs and characteristics.

The standard line is "wind a few turns of heavy conductor".

So let's discuss awhile and see if I can put this in fuller perspective.

The primary coil design and construction is critical to a well balanced

and efficient Tesla coil system.

First lets talk about the actual conductor. The Tank circuit pulse

oscillates with very heavy RF current. This energy exhibits "skin

effect" in that the electrical current does not flow through the

conductor as much as over it. Penetration has been calculated as only

being a few mils (.001 inch = 1 mil). The best conductor for this

type of electrical energy is a smooth flat strap with a few mils of

silver plating. The problem with flat strap primaries is that the

high voltage promotes corona loss from the thin edges, and this

loss can be significant. The sharp edge also provides a jumping off

spot for arcs to strike the secondary when the secondaries are

"overcoupled" (more on this later). One way around this is to use

a bar instead of a strap. The bar can have the thin edges rounded

to reduce corona losses and flashover, but it is not an efficient

use of conductor.

Another conductor of choice is thin wall copper tubing. This has

many advantages. The thin wall tubing is an excellent conductor

for tank circuit energy, and it is available in a wide variety of

outside diameters. The smooth rounded surface reduces corona loss.

I use the thin wall soft copper refrigerator tubing.

Heavy Litz wire would work well.

Conductors to avoid would be solid wire, braided strap, stranded

wire that has a "twist" in the conductor (parallel stranded is OK).

The next aspect of primaries is the size and shape. In the past many

designers have advocated small primaries consisting of five or so

total available turns, and frequently tapped in at three turns or

less. This line of thought is derived from radio theory where the

voltage gain was assumed to result from a ratio of turns trans-

formation. We now know that this is a misconception.

The Tesla secondary is in fact an open ended resonator. While it

is true that the ratio of turns transformation is responsible for

some voltage gain in the secondary, the majority of voltage produced

from a Tesla coil is pure resonant rise such as associated with open

ended transmission line VSWR. With this understanding the importance

of having a high ratio of turns between the primary and secondary

is diminished, and uniform excitation of the resonator becomes a

major concern.

The small five turn primary at the base of a coil does a very poor

job of uniformly exciting the secondary coil. Most of the energy

is forced into the bottom third of the secondary winding on these

systems and these turns are unnecessarily stressed. The solution

was frequently to space wind to unstress the secondary, which in

turn reduced the ratio of turns transformation, which further

crippled the poor design. The small primary, large capacitor, space

wound secondary is a design loser. These systems are highly stressed,

inefficient, and require large and expensive capacitors and trans-

formers that are prone to high failure rates.

The solution of course is to move in the opposite direction. The

primary needs more turns which cover a larger area. This means a

substantially larger primary coil. Instead of a five turn primary

coil tapped in at three turns, the coil needs to have fifteen turns

tapped out at twelve or more turns. This primary design creates

a much larger field flux which completely engulfs the secondary

winding. The resulting excitation is much more uniform.

What happens next is very interesting. Because the secondary coil

is uniformly excited, the secondary coil can be close wound with

magnet wire without breakdowns and excessive corona losses. The

close wound magnet wire secondary has a very high inductance per

unit volume of coil form. When coupled to a large, high inductance

primary, the resulting coil set will achieve the same ratio of turns

transformation as the small primary design discussed previously.

This is a "win-win" situation.

The large primary coil tunes with a much smaller (and cheaper) tank

circuit capacitance which requires a smaller power supply to charge.

Yet the system is able to process power more efficiently because of

the large field flux and uniform excitation of the high inductance

secondary. The overall RF power processing efficiency is so great

that coilers who follow this design method blow away other designs

by producing bigger sparks with much less input power. As an added

boon, the large primary coil and high inductance secondary form a

"slow-wave" system. The tank circuit operates at a lower frequency

with less peak power which in turn reduces the stress on the gaps,

caps, and power supplies. The gaps quench easier and I have found

that the tank circuit capacitors have a much longer life. Run times

are longer with a higher duty cycle. When something finally does

fail, it is cheaper to replace.

So now that I hopefully have convinced everyone that large primaries

and close wound high inductance secondaries are the way to go, what

shape primary coil is best?

The basic primary shapes are the vertically rising helix; the flat

"pancake" Archimedes spiral; and the "hybrid" inverse conical section

or "saucer" coil. I do not like the coupling characteristics and

the shape of the field flux produced by a vertically rising helix

primary. I do not recommend using them unless you are building Tesla

Magnifier systems. For small to medium 1/4 wave coils I would recommend

the "saucer" primary. As I mentioned earlier, I like about 15 primary

turns available, and the conical section should have an angle of slope

of about 30 degrees starting with the lower inside turn and rising to

the outside turn. On larger coils running with higher voltage inputs

and greater peak powers the best primary is going to be a flat pancake.

The reason I like these specific shapes is for their coupling

characteristics, which are derived from the size and shape of the

magnetic field flux they produce. A good primary coil will end up

being about as wide as the secondary coil is tall. When the primary

is tapped out over nine turns or so, the field flux that is generated

couples in the entire secondary winding all the way to the top turn

on the coil. I know this for a fact because in Tesla coil systems

that are overdriven the field flux contains so much energy that it

ionizes the air. Clearly visible corona maps out the field size, shape,

and relative intensity.

Next we need to briefly discuss coupling and over-coupling.

Coupling quite loosely is the inductive interaction between

two coils. In Tesla coil systems the pulse oscillation through

the primary creates a powerful magnetic field. When the gap is

quenched the magnetic field collapses and the energy that was

contained in the system is forced into and trapped in the

secondary. If too much energy is forced into the secondary coil

the resonator frequency will "split". Splitting is the formation

of parasitic harmonic resonances of higher frequency in the

secondary winding. Splitting is observed when the 1/4 wave voltage

peaks of these parasitic harmonics break out as sparks from the

middle of the winding. Sparks will jump up and down the sides of

the coil. Excessive corona resulting in a visible primary/secondary

field flux is another indication that the secondary coil is being

overdriven. Splitting and excess corona indicate that the system

is over-coupled. The cure for over-coupling is to raise the

secondary coil some, which reduces the inductive interaction.

If the primary coil is not properly shaped, like a flat pancake

coupled to a tall skinny secondary, then the coupling will be too

loose and insufficient energy will be transferred to the secondary.

There is no easy cure for this. If the primary under-couples or

couples too "loosely" you have to rework the coils. This is one

reason I suggest that the intended primary be designed and built

with tight or "close" coupling in mind. If the primary over-couples

you can then raise the secondary until the system is "critically

coupled".

The one thing that did not just fall into place in the text above

is the turn to turn spacing of the primary. The space between turns

needs to be kept small. The largest spacing that I have required

is 1/2 inch between the edges of a copper tube primary where the

operating voltage was 20KVAC RMS into the tank circuit. Most primaries

need no more than 1/4 to 3/8 of an inch inter-turn airspace.

> Tesla built his coil in Colorado with the secondary and primary

> having the same diameter, and with a bottom fed 'extra' coil. Have

> you ever tried building a basement sized coil to these geometries?

There is no need. Tesla developed and built large diameter coils in

order to distribute the charge densities over a very large area. Tesla

was working in an era of "stone knives and bear skins" (Spock), his

coils were wound on open wooden forms and his best wire insulation was

natural rubber gutta-percha (loaded with carbon soot from the curing

process) with a cotton wrapping. This is not like what we use today.

Today I build hermetically sealed all plastic coil forms. Modern

plastics, sealers, insulations and adhesives can contain much

greater charge densities in a much smaller volume with less loss.

There is no legitimate reason to wind large diameter coils when a

properly designed and constructed coil of much smaller dimensions

will exceed the performance of the former.

Richard Quick

... If all else fails... Throw another megavolt across it!

___ Blue Wave/QWK v2.12

***************

One of my own more extensive posts on Primaries...

Subscriber: tom_mcgahee@sigmais.com Tue Jan 28 23:15:20 1997

Date: Tue, 28 Jan 1997 23:11:20 -0500

From: Thomas McGahee <tom_mcgahee@sigmais.com>

To: tesla@pupman.com

Subject: Guide to Primaries rev 1.02

THE GUIDE: TESLA COIL PRIMARIES

Rev. 1.02 January 28, 1996

At its simplest level the primary of a Tesla coil is just a coil

of wire that creates a magnetic field when you pump current through it.

One of our basic goals as Coilers is to find ways to create repeating

pulses of very high current in the Tesla primary. This will produce intense

magnetic fields. If we can get these magnetic fields effectively coupled to

the secondary and operating at the self-resonant frequency of the

secondary, then we can throw some pretty big sparks. So the design of the

primary coil is important.

So here are the challenges before us:

1) Make the primary capable of handling repetitive high currents.

2) Keep the the time it takes for a pulse of high current to rise and fall

as short as possible.

3) Make the primary a tuned resonant circuit.

4) Use the high level primary currents to create intense magnetic fields.

5) Shape the resulting magnetic field so that as much of the energy as

possible is transferred to the secondary.

6) Minimize all sources of power loss.

7) Try to do all this and not implode your wallet.

Why do we want repetitive currents that rise and fall rapidly?

One of the factors affecting the strength of a magnetic

field around a wire carrying current is the current itself. Current is

defined in terms of the number of electrons flowing through a conductor per

second. If we store the electrons up (in a capacitor) over a fair period of

time and then dump them all at once, as fast as we possibly can, then even

though our average current flow may be small, we can still make the peak

value very high! At the instant we dump all those electrons into the

primary we can get really obscene peak values if we can keep the resistance

of the primary really really low.

OK, so how do we go about making the primary resistance low? When dealing

with high frequency circuits we want to maximize the surface area.

With DC circuits we would want to increase the cross sectional area, but

high frequencies flow mostly over the surface of conductors, instead of

through them as DC would. This is great, because if we use a thin ribbon of

copper that is fairly wide, it can readily carry lots of high frequency

currents with only a fraction of the total material that a solid conductor

would need.

Flat ribbons have these nasty sharp edges. We don't like sharp endges on

RF conductors, because it promotes energy loss through corona discharge. So

we come up with an almost perfect answer: a conductor with lots of surface

area, little or no wasted cross sectional area, and as non pointy as you

can get (and still be a long conductor). Behold hollow copper tubing! Meets

all the criteria and is readily available to boot. Why copper tubing? Why

not make it out of lead pipe or iron pipe? Because we also need the lowest

possible DC resistance, and the metal should be non-ferromagnetic. If it

was ferromagnetic, then when it produced a magnetic field, the magnetic

field would want to stay close to the iron, because iron conducts

magnetic flux quite well. But we want to get the magnetic field to couple

with the secondary, not hoard it. If you used a ferromagnetic material, the

energy loss due to eddy currents would be horrendous. Much of the energy

would be expended in heating the pipe. This would be bad news, so we don't

do that. We use copper. Copper is king when it comes to Tesla coils. Some

people plate their copper primary with a coating of silver. This gives

about a 4% increase in conductivity.

For smaller coils the 1/4 inch copper refrigerator tubing is often used.

Many medium to large size Tesla coils employ 3/8 inch copper tubing. Really

big coils use 1/2 inch and larger copper tubing. It is best if you can get

a single piece to form the primary, but if not, then pieces can be soldered

or brazed together. If you put a sleeve on the outside to make a splice,

then you might have a corona problem later. An alternative is to stick a

short section of thinner copper tubing partially inside both halves and

then solder. Use fine sand paper to remove any spots that are bumpy and

might cause corona problems. When bending copper tubing, you want to avoid

having the inside of the bend crimp or buckle. One method is to make sure

the inside bend is firmly pressing up against something and to only bend it

a small amount at a time.

We'll cover the topic of the shape of the primary later in this article,

but we need to mention inductive reactance now. Whenever you have a coil,

it builds up a magnetic field when you pass current through it. Because of

the adjacent turns in the coil, self-induction exists in the coil. That is

the tendency of the coil to oppose changes in current flow. A pulse would

normally cause a coil to have an increasing magnetic field. This magnetic

field will produce a current in the adjacent windings that will tend to

oppose increases in current. Then, when the magnetic field collapses, it

will produce a current that will add to the decaying current such that it

tends to prolong the current flow instead of letting it decay.

This tendency to oppose changes in current (it's called inductive

reactance) is a function of the inductance of the coil and the Effective

Frequency of the applied Pulse. I say the effective frequency, because a

pulse will always appear to a coil as if it had a Frequency higher than its

repetition rate. A pure sine wave will have no pulses, and its frequency

and repetition rate will be the same.

Oh Great! So we are supposed to keep the resistance of the primary coil as

low as possible, and now we find that it's got this stupid inductive

reactance thing that makes it act like a dumb resistor. Sheesh! Now what?

How do you  cancel out the effects of inductance? It's really fairly

simple. You put the coil in series with a capacitor that has capacitive

reactance that is equal to the coil's inductive reactance at the effective

frequency of the pulse. This works because inductive reactance and

capacitive reactance are 180 out of phase with one another.

For any given inductor/capacitor combination, there is ONE frequency where

The two reactances are EQUAL in Reactance value and opposite in phase. This

frequency is the resonant rrequency. (It's the same frequency that we

ultimately want to also equal the self-resonant frequency of the secondary

coil.)

Hey, what if we use the same exact capacitor that we used for energy

storage to act as this capacitive reactance thingie?? It happens to be

exactly the idea that Tesla had. (You are getting SO smart!)

By the way, do you see how all the parts constantly interact? The

Transformer and Capacitor are ALSO supposed to be matched for Maximum

Effeciency, but at the frequency of the AC line, which is 50 or 60Hz in

most cases.

Most of the really spectacular things that a Tesla coil can do occur only

when the tuning of the primary circuit matches the tuning of the natural self-resonant frequency of the entire secondary circuit (That includes the toroid,

if any is used).

Consider coupling to be the way the primary's magnetic field interacts with

the secondary. If coupling is too loose, the coil is inefficient and wimpy.

If coupling is too tight, then you may over-stress the primary and have

voltage breakdown occuring along the secondary or between the primary and

the secondary coil. In olden days there was much emphasis on tight

coupling, because many coil builders erroneously thought that a Tesla coil

worked mainly by transformer action. Not so. A Tesla coil is a

transformer that is tuned to resonance. The resonance is just as important

as the magnetic effects. Even more so if you are talking about a Tesla

Magnifier!

The shape of the primary affects its inductance and its coupling to the

secondary. The inductance affects the resonant frequency of the primary

circuit, and the coupling affects the overall energy transfer from the

primary circuit to the secondary circuit.

Many of the early Tesla coils used a rising helical primary that was

closely coupled to the secondary. Flashover from secondary to primary was

common with this design, and many a secondary coil was destroyed because of

the sparks destroying the insulation. When coupling is too close, the

windings of the secondary get over-stressed and the secondary circuit can

experience breakdown between windings, or sparking between the primary and

the secondary. Some people try to compensate for this by cramming as much

solid or liquid insulation as they can between the primary and the

secondary. That works somewhat, but it has its limits and its limitations.

Eventually almost any insulation can be broken down, even oil insulation.

The flat pancake coil and the saucer shaped primary are the most popular

primaries among serious coilers, but there are specialty coils that still

use the rising helical primary.

Spiral coils have less of an over-coupling problem. The usual arrangement

is what is sometimes called an

Archimedes or Archimedian Spiral (because the Greek mathemetician

Archimedes was the one who formulated its characteristics). In this kind of

spiral the distance between adjacent turns is kept constant. If the spiral

is kept flat, it is often referred to as a "pancake" coil. If the coil is

not kept flat, but instead each turn of the spiral also includes a rise,

then you have an inverted cone or saucer shaped primary.

The question usually comes up as to how far apart from the outside of the

secondary should the primary begin. Most coilers have the first primary

turn beginning at a distance of one to two inches from the outside of the

secondary coil. This may be adjusted to a larger starting diameter,

especially if the coil requires looser coupling. Coupling can be adjusted

somewhat if the primary and secondary are made moveable with respect to one

another. Some coilers arrange things so that the secondary may be raised

and lowered. Others prefer to move the primary and leave the secondary

fixed. Which method is used is more a matter of convenience and personal

preference than anything else.

Keep in mind, however, that just because most people do something a

particular way does not mean that that is the only way it can be done. Nor

does it necessarily imply that it is always the best way that it can be

done. For example, Malcolm Watts reports that he has made primary coils

whose inside diameters have been less than that of his secondary coils, and

he reports good results from this arrangement. One of his coils had as its

primary a flat pancake spiral made of 3/8" copper tubing with the first

turn having an inner diameter of about 5", and an outer diameter of about

20". The secondary diameter was about 17". He states: "I have noticed no

substantial difference between performance of a helical primary and spiral

primary of the same inductance, using same cap and transformer, and coupled

to the secondary with the same k. I submit that the gap losses (reduced

with higher values of k and/or higher primary surge impedance) determine

system performance if all other components are of good quality and

unchanged in value. I think Bert's SPICE simulations bear this out."

Why are flat spirals and saucer shaped primaries so popular? Because they

work so well. Why do they work so well? Because each creates a magnetic

field that is large and encompasses (ideally) the entire secondary. You can

actually SEE the beautiful shape of this field if you operate a powerful

Tesla coil in the dark. The corona discharge from the primary will engulf

the entire secondary in a kind of inverted parabolic curve when the

coupling and geometry are just right.

*** Note: there will probably be separate chapters dealing with the nitty

gritty construction details of the primary. This chapter is primarily

designed to give the reader a good idea as to how the primary works, and

what it does, and how to maximize that in general terms. Specifics of

construction are probably best placed in a separate article dealing with

construction issues.

*** end note

***** See my post entitled Recycled: Primary Construction ******

A Tesla coil is a 1/4 wave resonant device. When it is operating properly

the base of the secondary has a low voltage and a high current, while the

top of the secondary has a high voltage and a relatively low current. It

may be useful if you think of it in terms of a standing wave: Imagine one

cycle of a sine wave. It reaches its Peak value at 90 degrees. That is a

quarter of a full wave. If you get a Tesla coil to operate at resonance,

you have a standing wave in which the top of the secondary is operating at

this 90 degree point. It will therefore have maximum voltage at that point.

There is a simple experiment you can try with a child's jumprope that will

illustrate standing waves. Have someone hold one end of the rope tightly in

both hands, with their hands held tight against their stomach so that the

rope at that end will be anchored fairly securely. You grab the other end

of the rope tightly in one hand, and begin moving the rope up and down

about one foot. Start off slowly, and then gradually increase the speed at

which you are moving the rope. Keep the up and down distance you move as

constant as you can. Every time your hand moves the rope up and down it

will send a traveling wave down the rope. But you will reach one particular

rate at which you will no longer see a wave traveling down the rope.

Instead you will see that there is a point right at the center of the rope

that appears to be standing relatively still. At a distance halfway from

the center of the rope to where you are you will see that the rope is

whipping up and down pretty good. That is the 1/4 wave point. That is where

the greatest activity is. When a Tesla coil is operating at resonance, it

is operating at its 1/4 wavelength. Notice that you can get the rope to

have standing waves at higher frequencies. Struggle really hard with the

rope and you might be able to get a standing wave with two nodes standing

still, each about one third of the way down the rope. Notice that it

requires a lot more work on your part to maintain a standing wave with two

nodes, and that the amplitude of the wave is not as big as when you only

had one node. In the same way, you can force a Tesla coil to operate at the

wrong frequency, but it is not achieving full resonance. If a Tesla coil is

improperly tuned the applied energy is wasted because returning waves

cancel the original waves.

The key, then is to achieve this 1/4 wave resonant point. And you want to

do it with efficiency. That's where all the little nitty gritty details

come into play. The frequency we tune the primary circuit to should ideally

match the natural self-resonant frequency of the secondary. Of course, in

real life nothing is quite as simple as that. The secondary's natural

self-resonant frequency varies (in real time!)

***** end of main article on primaries. It is still very incomplete. *****

This is an on-going work in progress.

See any errors? Did I leave something out?

Anything that is now confusing you even more than before?

Fr. Tom McGahee

**************

Bert Hickman was kind enough to provide a wonderful

set of formulas with accompanying ASCII art.

Thanks, Bert!!!

Subscriber: bert.hickman@aquila.com Thu Jan 30 23:04:55 1997

Date: Thu, 30 Jan 1997 21:28:27 -0800

From: Bert Hickman <bert.hickman@aquila.com>

To: tesla@pupman.com

Subject: Re: Guide to Primaries rev 1.01

Tesla List wrote:

>

> Subscriber: tom_mcgahee@sigmais.com Tue Jan 28 23:15:20 1997

> Date: Tue, 28 Jan 1997 23:11:20 -0500

> From: Thomas McGahee <tom_mcgahee@sigmais.com>

> To: tesla@pupman.com

> Subject: Guide to Primaries rev 1.01

>

> THE GUIDE: TESLA COIL PRIMARIES

> Rev. 1.01 January 28, 1996

>

<MAJOR Snippola>

Tom,

Looks pretty good to me! Some fill-in formulas would also be useful for

the helical, Archimedes, and inverse conical primaries. The helical and

Archimedes forms are from Wheeler, and the inverse conical is a hybrid

closed-form that appropriately weights the vertical and horizontal

components of Helical and Archimedes inductances.

All dimensions are in inches, and L is in microHenries. While the

Archimedes calculation is a little "hairier" than the first two, it's

relatively easy to calculate for any desired angle, especially if set up

in a spreadsheet.

------------------------------------------------------------------------

Case 1: Archimedes Spiral:

Let R = Ave Radius

N = Number of Turns

w = Width of Winding

|   R    |      N Turns

o o o o o o   |   o o o o o o

|    W    |

L = R^2*N^2/(8*R+11*W)

------------------------------------------------------------------------

Case 2: Helical Primary:

| R |

--  o       o

|   o       o

o       o

L   o       o  N Turns

o       o

|   o       o

--  o       o

L = R^2*N^2/(9*R+10*L)  (Vertical Helix)

------------------------------------------------------------------------

Case 3: Inverse Conical Primary:

/

\                                        --  o

/    o

|    o   N turns            /    o

o                 Z  /    o

h      o                  /    o   /

o                /    o   /

|        o              /    o   /  Angle = X

o              \  o   /

--         o               o    ------------

|

|   w   |   R   |

|

|<--  W  -->|

Z = Coil Width (hypotenuse)

X = Angle of Cone

h = Z*sin(X)  Effective vertical Height

w = Z*cos(X)  Effective horizontal Width

W = R + w/2   Average horizontal Radius

L1 = W^2*N^2/(9*W+10*h)  (Vertical Inductance Component)

L2 = W^2*N^2/(8*W+11*w)  (Horizontal Inductance Component)

L = SQRT[(L1*Sin(X))^2 + (L2*cos(X))^2]

------------------------------------------------------------------------

Safe coilin' to you, Tom!

-- Bert --

**************

Some stuff on Primary Taps by yours truly...

----------

> From: Tesla-2 <tesla-2@emachine.com>

> To: tom_mcgahee@sigmais.com

> Subject: primary taps

> Date: Saturday, June 14, 1997 2:36 PM

>

> From: Mike Hammer <mhammer@misslink.net>

> Date: Sat, 14 Jun 1997 13:36:13 -0500

> Subject: primary taps

>

> Hello all,

>

> Since I will be taking some time off

> the next couple weeks I hope to get some

> serious coiling done.

>

> One thing I need to work on is my primary tap.

>

> After raising my primary capacitance I noticed

> that my primary tap gets quite warm.

>

> After several extended runs this piece was too hot

> to touch. So obviously my arrangement of a large

> alligator clip for a primary tap has some amount

> of resistive loss. I had attached 1/2 sections of

> small copper tubing to the jaws to increase the contact area.

>

> My question is to those of you with operating coils.

> What are you using for a primary tap?

> Just looking for a few ideas to lower the loss in this area.

>

> TTYL!

>

>

> Mike Hammer

> mhammer@misslink.net

Mike,

As you have noticed, the resistive losses from the primary TAP

mechanism can be excessive. The losses are from several factors.

1) The currents are just obscenely high. But that's what we WANT, so

we will NOT try to reduce *that* in any way :)

2) With extreme high RF currents you need a large surface area to get

the current transferred. The stupid little teeth on even the best

'gator clips are pathetic for RF use. The points just intensify the

problem.

So, get rid of the stinking teeth and increase the surface area.

Grind the jaw structure so that a maximum surface area come in

contact.

Add a more robust surface area by soldering on copper pieces shaped

so as to maximize surface contact (not too easy since the shape of

the spiral changes from turn-to-turn). Easier for you solenoid guys,

as the pitch and radius are constant in that form of primary.

Use a MUSHY metallic material to make the contact. Think metallic

Braid. Think of the outside metallic braid on coax cable. Yeah, I

know that everyone tells us not to use braid. But they are talking

about the main power runs, my friend! HERE is a place where the braid

has a quality that overcomes its defect. The great MUSHINESS of the

braid makes it ideal to create a larger surface area contact.

Try *this* with your existing clip lead setup: wrap two or three

turns of braid **tightly** around the desired tap point. Let the clip

lead sink its old 'gator teeth into the braid. Not perfect, but a

definite improvement! (And ridiculously EASY to implement, for all

you lazy bones coilers out there). How many wraps you can tolerate

depends on the size of your 'gator, and the spacing between primary

turns.

All right, so now that I have convinced you of the joys of braid,

consider attaching the braid right to the inside of the jaws of the

'gator itself. You can file away the teeth and that will give you

more room for the braid. Solder it only OUTSIDE the mouth of the

'gator clip. We want the stuff between the jaws to remain MUSHY. Be

careful, as the braid LOVES to wick up solder. TIN the outside of the

jaws and lightly tin the end of the braid. Heat up the jaw until the

solder melts and then touch the tinned end of the braid into the

solder on the jaw . Hold the braid in a needle nose pliers jaw,

tightly and near the end. This will restrict the solder wicking by

drawing away the heat. It will also prevent your fingers from having

the permanent imprint of wire braid branded into them.

After soldering one end, wrap the braid along the inside of the jaw

and solder the other end on the other side. What you want is a run of

mushy flat braid across the inside of the jaw. Do both the top and

bottom jaws.

An alternative to going across the width of the jaw is to run it down

from the top of the nose of the jaw, loop it INSIDE the jaw, and

around such that it then comes back out across the bottom of the

bottom jaw. Soldered on the outside, of course.

There are MANY variations on the theme. Pick one. Make a mushy jawed

'gator clip. Amaze yourself. Impress the neighbors.

3) With really huge currents the TYPE of conductor becomes very

important. Heavy duty solid COPPER is best. It is also one of the few

metals that really solders well. So if you are going to modify a

'gator clip, I suggest beginning with a solid copper one if you can

get one. Trying to solder aluminum 'gator clips is an exercise in

frustration. Save your energy and your sanity. Buy copper 'gator

clips.

Now, some of you are using those dumb little Radio Shack mini 'gator

clip leads with all the cool little colored booties on them. Doomed

to failure from the word go. Not enough surface area to conduct our

obscene RF currents. These babies get hot enough for the solder to

melt and the plastic booties begin to drip off. Forsake the wimpy

mini gator clip and go for the Macho 'gator clip. Think Surface Area.

4) 'Gator clips are made from two sections. In a high current RF

environment this is bad news, as the second section has only a few

minor points of contact with the main section. This means a high RF

resistance for half of the clip. This in turn means lots of heating

WITHIN the 'gator clip itself. Solder coax braid from the wide top to

the wide bottom. Keep it short and WIDE. Really industrious little

coilers will find ways to do this work from the INSIDE as well as

from the outside. Think Skin Effect.

5) And the final 'gator clip modification: Throw out the stinking

gator clip. Build you own Compression Fitting Primary Tap. This is a

tap that is designed so that it is primarily a PERMANENT TAP, but in

a Temporary sort of way. A 'gator clip is temporary and makes a lousy

permanent tap. Soldering and other such methods are permanent and you

have to live with it.

A compression fitting can be fabricated in many ingenious ways, but

its main feature is that you can move it around, but once you are at

the right place, you can tighten this sucker down and leave it there

forever if you want. The down side is that it takes a bit longer to

move it around. But it makes an EXCELLENT connection.

I have seen small hose clamps used. Properly trimmed they aren't all

that bad. These work best on larger diameter tubing.

The compression fitting has SOME means to clamp the metal tightly

around the primary coil. The metal that gets clamped should have some

flexibility, but most importantly it should make an extremely tight

connection with a good sized surface area.

The material should be something that can take a certain amount of

flexing without breaking. I have used copper strapping to good

advantage. You can make your own from sheet copper, or actually buy

pieces that are about 1/2 to 1" wide at building supply stores. You

can also use wide flat braid as the flexible conductor part and use

brass plates to do the compression part.

* Send list Replies to: Tesla-2@emachine.com

From: tesla-2@emachine.com (Tesla-2)

Subject: Recycled: Primary Shapes and Formulas

Date: Sun, 19 Oct 1997 16:44:23 GMT

Organization: The Energy Machine Information System 504-733-3462

From: "Thomas McGahee" <tom_mcgahee@sigmais.com>

Date: Sun, 19 Oct 1997 12:44:23 -0400

Subject: Recycled: Primary Shapes and Formulas

The compression part consists of some means to pull the strap tightly

around the copper tubing and secure it. I often use brass plates that

I have drilled and tapped so that one plate has holes larger than the

screws and the other has been tapped so that the screw can be secured

by that plate. Also connected to this brass plate is the supply lead.

It may be soldered or also compression clamped. Think surface area.

Sometimes I attach more than one feed lead per clamp. That increases

the surface area of the feed lead while keeping it relatively

flexible.

I normally have the flexible flat copper/brass or braid section

soldered to the two afore-mentioned plates. Solder them well, as the

solder is going to be providing physical strength to the assembly.

Another method is to use another plate to clamp the flexible metal

band to the master plate.

If using braid, consider first rounding the edges of the plates that

are where the braid passes by. Otherwise the sharp edges will bite

through.

However you decide to do it, get the spacings adjusted so that when

the flexible piece is wrapped around the tubing and you tighten down

the screw(s) the unit is firmly attached with a decent surface area.

Hope the information is of use to someone.

Fr. Tom McGahee

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Insanity!!

Date: Mon, 20 Oct 1997 19:11:33 GMT

Organization: The Energy Machine Information System 504-733-3462

From: thorn@northlink.com

Date: Mon, 20 Oct 1997 12:11:33 -0700 (MST)

Subject: Insanity!!

Hello all,

I just received a call from a company regarding magnet wire.  For the

type she mentioned, which I cannot even remember, due to shock.  They were

saying the wire was .51 cents a foot!!!!  This cannot be!!  Does anyone know

of a company that will ship manget wire for #24 guage for my sencondary?  My

winding is 21" of 24guage, that is approx 1200' of wire.  Secondary diameter

is 4".

Thanks,

Robert

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: Insanity!!

Cc:

Bcc:

X-Attachments:

At 07:11 PM 10/20/97 GMT, you wrote:

>From: thorn@northlink.com

>Date: Mon, 20 Oct 1997 12:11:33 -0700 (MST)

>Subject: Insanity!!

>

>Hello all,

>

>   I just received a call from a company regarding magnet wire.  For the

>type she mentioned, which I cannot even remember, due to shock.  They were

>saying the wire was .51 cents a foot!!!!  This cannot be!!  Does anyone know

>of a company that will ship manget wire for #24 guage for my sencondary?  My

>winding is 21" of 24guage, that is approx 1200' of wire.  Secondary diameter

>is 4".

>

>

>Thanks,

>

>Robert

Not to worry Robert. Sounds like you might of called Newark Electronics.

Am I right? Ive done lots of business with Newark but great prices

are not their strong point.

My first advice is to get out the yellow pages and

look under electric motors. You want a place that rewinds and repairs

electric motors. These shops are very common. You will find the

best wire prices at one of the shops. I get most of my wire from

a local shop for $4.50 per pound.

If that fails then call Joe Cassata in Naperville Il.

Joe is a wire and cable dealer and can get you almost

anything wire wise. He is a reputable dealer and a swell guy.

His prices arent as good as a local motor shop but is a good 2nd

choice. I recently posted a current price list from him but

will repost it here for you now.

I picked up a flyer from him at the Peoria Illinois hamfest a

couple weeks ago and thought I would post his current pricing

on magnet wire as well as his current address and phone number.

The Coax Connection

Joe Cassata

10 S 226 Meadow Lane

Naperville,Il 60564

630-420-0342

AWG    ft/lb    $/lb

14      79.7    5.10

16     127      5.30

18     201      5.50

20     315      5.85

22     516      6.05

24     802      7.20

26    1280      8.00

28    2027      8.75

30    3212      9.25

Other sizes available.

TTYL!

Mike Hammer

mhammer@misslink.net

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Parallel Neons

Date: Thu, 23 Oct 1997 13:51:08 GMT

Organization: The Energy Machine Information System 504-733-3462

From: "Paul Marshall" <PMARSHAL@ETHCINC1.SSW.JNJ.COM>

Date: 23 Oct 1997 08:51:08 EST

Subject: Parallel Neons

I read a post to the Tesla-2 a couple of days ago which stated that

neon sign transformers are very likely to fail when placed in

parallel. The same article stated that a single transformer driving

a coil almost never failed. Can multiple transformers be adequately

protected ? What is the primary failure mode ?

I hope to have my coil in operation by November 15. I plan to begin

with one transformer to test and tune. After testing I had hoped to

add another in parallel. How risky is this.

Thanks,

Paul Marshall

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: Parallel Neons

Cc:

Bcc:

X-Attachments:

At 01:51 PM 10/23/97 GMT, you wrote:

>From: "Paul Marshall" <PMARSHAL@ETHCINC1.SSW.JNJ.COM>

>Date: 23 Oct 1997 08:51:08 EST

>Subject: Parallel Neons

>

>

>     I read a post to the Tesla-2 a couple of days ago which stated that

>     neon sign transformers are very likely to fail when placed in

>     parallel. The same article stated that a single transformer driving

>     a coil almost never failed. Can multiple transformers be adequately

>     protected ? What is the primary failure mode ?

Paralleled neons work well but you start gettting into problems

once you have more than 3 or 4 in parallel. Seems like the guys

that have large banks of neons have more failures.

Best bet for protection is individual safety gaps right on

each transformer. This way each transformer gets the best

protection.

Neons are built to pretty loose standards so there is quite

a bit of variation. The output voltages will vary quite a bit.

Once you start paralleling them up this variation can cause

unequal load sharing.

Its best not to mix different manufacturers neons together.

And you cannot mix different voltages. You cannot for example

parallel a 12kv and a 15 kv neon.

However you can safely parallel different current ratings.

You can parallel a 12kv/30 and a 12kv/60 for instance.

>

>     I hope to have my coil in operation by November 15. I plan to begin

>     with one transformer to test and tune. After testing I had hoped to

>     add another in parallel. How risky is this.

>

With two neons not at all risky.

TTYL!

Mike Hammer

mhammer@misslink.net

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: mirror of Tesla-2 archives

Cc:

Bcc:

X-Attachments:

Hello all,

Several list members have offered space in which

to house the list archives. Thanks to all who offered.

The mirror site for the list archives is in Erie, Pennsylvania

and is courtesy of list member Dave Bronakowski.

The address is:

ftp://ftp.velocity.net/usr/djb/tesla2/

(Note: Address was chaged to : ftp://ftp.velocity.net/usr/djb/tesla/tesla2/)

This site offers great speed for downloads and

is a good alternative when funet.fi is down or slow.

The funet address is:

ftp://nic.funet.fi/pub/sci/electrical/tesla/maillist/tesla2/

Thanks Dave.

TTYL!

Mike Hammer

mhammer@misslink.net

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: Parallel Neons

Date: Thu, 23 Oct 1997 18:04:51 GMT

Organization: The Energy Machine Information System 504-733-3462

From: corr <corr@fullnet.net>

Date: Thu, 23 Oct 1997 12:04:51 -0600

Subject: Re: Parallel Neons

>With two neons not at all risky.

Perhaps NORMALLY not risky, but I must point out myself, that when

I parallelled two 15kv, 30ma neons for 15kv @ 60ma, one of the 30ma

neons shorted out.  Whether or not this is related to the neons, or some

other factor such as RF feedback, I am uncertain.  But it *can* happen.

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: mirror of Tesla-2 archives

Date: Thu, 23 Oct 1997 22:07:46 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Dave <djb@velocity.net>

Date: Thu, 23 Oct 1997 18:07:46 -0400 (EDT)

Subject: Re: mirror of Tesla-2 archives

At 03:45 PM 10/23/97 GMT, you wrote:

>From: Mike Hammer <mhammer@misslink.net>

>Date: Thu, 23 Oct 1997 10:45:34 -0500

>Subject: mirror of Tesla-2 archives

>

>Hello all,

>

>Several list members have offered space in which

>to house the list archives. Thanks to all who offered.

>

>The mirror site for the list archives is in Erie, Pennsylvania

>and is courtesy of list member Dave Bronakowski.

>

>The address is:

>ftp://ftp.velocity.net/usr/djb/tesla2/

>

>This site offers great speed for downloads and

>is a good alternative when funet.fi is down or slow.

>The funet address is:

>ftp://nic.funet.fi/pub/sci/electrical/tesla/maillist/tesla2/

>

>

>Thanks Dave.

>

>TTYL!

>Mike Hammer

>mhammer@misslink.net

Hi Mike,

Sorry for the mixup. The address for the mirror is:

ftp://ftp.velocity.net/usr/djb/tesla/tesla2/

I decided to make this a subdirectory of tesla, so I can make other

subdirectories in the tesla category such as for texts, programs, pictures,

etc. All of that will be at a later time though. This address will not

change from now on. Sorry again.

Dave

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: magnet wire

Date: Fri, 24 Oct 1997 00:52:08 GMT

Organization: The Energy Machine Information System 504-733-3462

From: tesla.coil@mailexcite.com

Date: Thu, 23 Oct 1997 19:52:08 -0500

Subject: magnet wire

Organization: MailExcite  (http://www.mailexcite.com)

Does anyone know a good, reliable place to

get about 10k ft. of magnet wire? Has

anyone bought this much or more from

a mail order company? If so, what was the price?

Thanks!

--Jay

Free web-based email, Forever, From anywhere!

http://www.mailexcite.com

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: Parallel Neons

Date: Fri, 24 Oct 1997 03:48:38 GMT

Organization: The Energy Machine Information System 504-733-3462

From: randy@gte.net

Date: Thu, 23 Oct 1997 20:48:38 -0700

Subject: Re: Parallel Neons

Tesla-2 wrote:

>

> From: "Paul Marshall" <PMARSHAL@ETHCINC1.SSW.JNJ.COM>

> Date: 23 Oct 1997 08:51:08 EST

> Subject: Parallel Neons

>

>      I read a post to the Tesla-2 a couple of days ago which stated that

>      neon sign transformers are very likely to fail when placed in

>      parallel. The same article stated that a single transformer driving

>      a coil almost never failed. Can multiple transformers be adequately

>      protected ? What is the primary failure mode ?

>

>      I hope to have my coil in operation by November 15. I plan to begin

>      with one transformer to test and tune. After testing I had hoped to

>      add another in parallel. How risky is this.

>

>                                                  Thanks,

>                                                  Paul Marshall

>

>

From: Randy's Stupid Question Emporium and Misinformation Repository

Exchequer of Rhetorical Questions Department

Would it be possible/practical to wind a bifilar set of primaries,

fed with "identical" trannies...in phase... for the purpose of

essentially isolating the trannies from each other, at least to

the extent of harming each other...I know establishing the proper

tap would be a bear though.

Sorry if I am a pain.

Randy

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: magnet wire

Cc:

Bcc:

X-Attachments:

At 12:52 AM 10/24/97 GMT, you wrote:

>From: tesla.coil@mailexcite.com

>Date: Thu, 23 Oct 1997 19:52:08 -0500

>Subject: magnet wire

>Organization: MailExcite  (http://www.mailexcite.com)

>

>Does anyone know a good, reliable place to

>get about 10k ft. of magnet wire? Has

>anyone bought this much or more from

>a mail order company? If so, what was the price?

>Thanks!

>

>--Jay

Wow. 10000 feet of wire? Sounds pretty serious.

What guage of wire you wanting to get?

As I said here before the best place to get magnet

wire is a local electric motor rebuilding shop.

Consult the yellow pages. You will find the best

prices at one of these places.

Failing that you can mail order some wire at somewhat

higher prices. One reputable place Ive done business

with is Joe Cassata fron Naperville, Il. If you order

wire from him tell him its for a tesla coil. A lot of

coilers have purchased wire from him.

The Coax Connection

Joe Cassata

10 S 226 Meadow Lane

Naperville,Il 60564

630-420-0342

These prices are off his price sheet I got from him

at the Peoria Il hamfest in September.

AWG    ft/lb    $/lb

14      79.7    5.10

16     127      5.30

18     201      5.50

20     315      5.85

22     516      6.05

24     802      7.20

26    1280      8.00

28    2027      8.75

30    3212      9.25

Other sizes available.

You can beat Joe's prices at a local motor repair shop but if you havnt got

one in your area this is a reputable place to do business with.

TTYL!

Mike Hammer

mhammer@misslink.net

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: winding length vs #of turns

Date: Fri, 24 Oct 1997 23:01:04 GMT

Organization: The Energy Machine Information System 504-733-3462

From: "Thomas McGahee" <tom_mcgahee@sigmais.com>

Date: Fri, 24 Oct 1997 19:01:04 -0400

Subject: Re: winding length vs #of turns

----------

> From: Tesla-2 <tesla-2@emachine.com>

> To: tom_mcgahee@sigmais.com

> Subject: winding length vs #of turns

> Date: Friday, October 17, 1997 6:55 PM

>

> From: "Chris Watkins" <cwatkins@seidata.com>

> Date: Fri, 17 Oct 1997 17:55:16 -0500

> Subject: winding length vs #of turns

>

> Help!

SNIP to cut down list clogging!

>

> I am working on a current limiter at present time

> I'm no electrical expert but i learn more every day.

>

> I have 2 iron cores from variable transformers  4"

> that i plan to rewind with just a few loops as limiters.

> will this work?

>

> I havent made a torrid yet but plan on something around

> 12" x 36"  or 12 x 48"

>

> I look forward to any comments

> Chris Watkins

>

>

Chris,

I am really into this recycling thing. Here is a recycled post from Chip's

Tesla list that talks about using variac cores for transformer ballast.

Hope this helps.

Fr. Tom McGahee

---------- Forwarded message ----------

Date: Fri, 17 Oct 1997 01:54:42 -0500

From: "DR.RESONANCE" <DR.RESONANCE@next-wave.net>

To: Tesla List <tesla@pupman.com>

Subject: Re: Transformer Ballasting

To: Gwyn

Try to scrounge the surplus yards or old theater groups to find a burned

out variac --the 30 amp variety has a good cross sectional area which is

the key ingredient.  Remove all the old windings.  Clamp the variac solidly

so it doesn't go "sprong" and carefully saw through with a fine hacksaw.

In the gap epoxy a piece of plastic approx 0.100 inches thick (100 mils).

Next hand wind the variac with 160 turns of #8 AWG insulated wire.  Tap at

150, 140, and 130 turns.  You now have a cheap and very good pole pig

current limiter -- save the welder for welding.  Use a 1/2 Ohm, 10 KW

resistor (about 12 feet of heater nichrome) in series with this limiter and

your coil will run even smoother --- eliminates the 100 kw peak surges.

DR.RESONANCE@next-wave.net

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: newbie questions

Date: Sat, 25 Oct 1997 19:52:26 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Dsurfr@aol.com

Date: Sat, 25 Oct 1997 15:52:26 -0400 (EDT)

Subject: newbie questions

Being an old architect I like to work things out on paper before building

them. I am currently designing my first coil in over 30 years and could use

some advice. Since the frequency of the secondary coil is determined by the

length of wire used what frequency should I be trying to achieve. The

calculations for my current plans come out around 400,000 Hz. Secondly the

old coils I made all had tubular shaped primaries. Everyone nowadays seems to

prefer helical ones. Does this greatly improve output performance or can I

stick with the easier to wind tube ones?

Any advice will be much appreciated.

Jim

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: newbie questions

Cc:

Bcc:

X-Attachments:

At 07:52 PM 10/25/97 GMT, you wrote:

>From: Dsurfr@aol.com

>Date: Sat, 25 Oct 1997 15:52:26 -0400 (EDT)

>Subject: newbie questions

>

>Being an old architect I like to work things out on paper before building

>them. I am currently designing my first coil in over 30 years and could use

>some advice. Since the frequency of the secondary coil is determined by the

>length of wire used what frequency should I be trying to achieve.

Doesnt matter. Any old frequency will do. Generally most coils

fall between 100 and 600 KHZ. The lower frequencies being somewhat

more efficient due to lower losses. But it wont be significant

in a first coil that is likely to have many loss factors.

Build your secondary to the proper H/D (height/diameter) ratio and

fill the space with 700-1000 turns and let the frequency fall where it may.

The range of H/D ratios used will be from 6 to 1 for a 3 inch secondary

coil to 3 to 1 for a 8 inch or larger.

If using a 4 inch form 21 inches of AWG 24 works out to about 975 turns

and performs quite well. Resonant frequency about 395 KHZ unloaded.

>The calculations for my current plans come out around 400,000 Hz.

Sounds fine.

> Secondly the old coils I made all had tubular shaped primaries. Everyone

>nowadays seems to prefer helical ones. Does this greatly improve output >performance or can I stick with the easier to wind tube ones?

>Any advice will be much appreciated.

You can certainly use a solenoid type primary. The reason for the

flat spiral and saucer shaped primary coils is to reduce coupling.

The normal solenoid type primary will overcouple. You would need to

raise the secondary up away from the primary to reduce coupling.

Too high a coupling will cause pri/sec arcing and corona. Raise

the secondary up a 1/2 inch at a time until the arcing stops.

If you do make a solenoid type primary make it more than twice the

diameter of the secondary. For a secondary on 4 inch PVC (4.5 inch OD)

a primary about 10 inches diameter would be fine. Make provisions

for adjusting the height of the secondary in relation to the primary

to adjust coupling.

TTYL!

Mike Hammer

mhammer@misslink.net

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: newbie question

Date: Sun, 26 Oct 1997 22:46:27 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Dsurfr@aol.com

Date: Sun, 26 Oct 1997 17:46:27 -0500 (EST)

Subject: newbie question

I have seen a couple of different wiring diagrams on coil plans. What is the

proper location for the capacitors & spark gap?

capacitor

I----|---------( --0----0---)---I

.I.     \                        I

transformer   [  ]     .\                       I

[  ]      .\   primary         I

[_]         O-----x--x---------I

I              spk gap       I

I--------------------------------I

capacitor

I---------( --0----0---)-------I

.I.               I             I

transformer   [  ]       spk  X                I

[  ]       gap   I               I   primary

[_]               X              I

I                 I             I

I-------------(-0--0--)--------I

capictor

Please excuse the graphics, I'm used to using AutoCad 14

Thanks, Jim

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: newbie question

Date: Mon, 27 Oct 1997 04:23:21 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Dave <djb@velocity.net>

Date: Sun, 26 Oct 1997 23:23:21 -0500 (EST)

Subject: Re: newbie question

At 10:46 PM 10/26/97 GMT, you wrote:

>From: Dsurfr@aol.com

>Date: Sun, 26 Oct 1997 17:46:27 -0500 (EST)

>Subject: newbie question

>

>

>I have seen a couple of different wiring diagrams on coil plans. What is the

>proper location for the capacitors & spark gap?

>

>                                     capacitor

>                       I----|---------( --0----0---)---I

>                      .I.     \                         I

>       transformer   [  ]     .\                       I

>                     [  ]      .\   primary         I

>                     [_]         O-----x--x---------I

>                       I              spk gap      I

>                       I--------------------------------I

>

>

>                                     capacitor

>                       I---------( --0----0---)-------I

>                      .I.               I               I

>       transformer   [  ]       spk  X              I

>                     [  ]       gap   I              I   primary

>                     [_]               X            O

>                      I                 I              I

>                      I-------------(-0--0--)--------I

>                                        capictor

>

>Please excuse the graphics, I'm used to using AutoCad 14

>Thanks, Jim

Hi Jim,

I asked these questions you are asking now awhile back, and there are some

good answers to them in the tesla2 archives. I'll paste the answers here

again for you.

Dave

<paste>

From: Mike Hammer <mhammer@misslink.net>

Date: Fri, 16 May 1997 15:58:55 -0500

Subject: Re: Schematic questions

At 05:01 PM 5/15/97 GMT, you wrote:

>From: Dave <djb@erie.net>

>Date: Thu, 15 May 1997 13:01:02 -0400

>Subject: Schematic questions

>

>

>    TESLA COIL SCHEMATIC                                  --------

>                                                          |      | TC

>                                                          --------

>                                                              O

>                                                              O

>               SG   SC      FCC     PC   SG        P-COIL     O S-COIL

>           O---------------OOOOO---------> <-----    O        O

>      NST  O    |    |              |           |    O        O

>           O    |    |              |           |    O        O

>   -----O||O    *   ---            ---          |--->O        O

>        O||O                                         O        O

>   -----O||O    *   ---            ---               O        O

>   110   | O    |    |              |                O        O

>   Volts | O    |    |              |                O        O

>         | O---------------OOOOO---------------------|        |

>         |                                                    |

>         |                                                    |

>        Gnd                                                  Gnd

>

>

>Hi Everyone,

>

>I have a few questions on this simple schematic above. Is this the most

>common configuration for small coils...ie less than 1000 watts?

This is a common setup and seen in many older designs.

>Are there other configurations that might be more advantageous to use, or

are >there disadvantages to this setup?

The answer is Yes to both questions.

Lets look at a different setup.

TESLA COIL SCHEMATIC                                ---------

|       | T1

---------

O

X1                                 C1              O

RFC           | |             O  L2

O------------------||||----------| |-----        O

||O       |                  |     | |    |   L1   O

||O       |                  |            |    O   O

-----O||O       * SG               * G1         |--->O   O

O||O--Gnd                                       O   O

-----O||O       *                  *                 O   O

120   ||O       |                  |                 O   O

Volts ||O       |                  |                 O   O

O------------------||||----------------------O   O

RFC                           |

|

Gnd

X1 = Neon Sign Transformer, grounded center tap (case)

SG = Safety Gap.

RFC = RF filter chokes

G1 = Main system (tank circuit) Spark Gap.

C1 = Tank circuit capacitor

L1 = Primary Coil.

L2 = Secondary Coil.

T1 = Toroid discharge terminal

Gnd = RF Ground.

As you can see the main difference is that the positions of

the main cap and main gap are swapped.

What does this do? Help to protect our fragile neons from damage

due to RF kickbacks.

With the capacitor in parallel across the neon secondary

the neon will "see" that RF across its secondary winding.

With the gap in parallel across the neon output the neon secondary

will be short circuited by the energized gap while all the big

action is taking place in the primary tank circuit.

Will this prevent damage to the neon? No. Not by itself.

But every little bit of protection helps.

>Also, what is the purpose of the safety gap? I

>can see the chokes would limit the amount of RF kickback, and the safety cap

>would act as a short for RF so it doesn't effect the transformer...so why

>have a saftey gap, and what would make it fire? And how wide should be the

>gap be?

RF chokes have only limited success in prventing Rf from getting into

the neon. The chokes help by knocking down the level of the RF.

The safety gap is there as backup to flash over and clamp the voltage

to a safe level when RF sneaks by the chokes.

The safety gap should be just wide enough so it fires every few

seconds. You want it just wider than your neon transformer

can break down on its own.

>Also could you connect a cap of suitable rating from ground to the bottom of

>the secondary before the chokes? I think it would act as an open for 60 hz,

>but as a short to ground for RF and protect the power supply, etc.

Some people have done this with success but I do not recommemd it.

In some neons bypass caps to ground can set up unwanted resonances

inside the transformer and lead to quick destruction.

I totally fried a France 12kv/30 neon in this manner.

If we take all the above steps and combine them all

we have a fairly good level of protection for our

neons. We want the main gap across the HV output.

We want a safety gap across the HV lines directly

from the transformer. And we want of minimum of 2mh

of RF choke inductance in each HV leg in between the safety

gap and main spark gap.

These 3 measures will go a long way in protecting

neons from damage.

As always comments, criticism, flames, and cash donations welcome!

TTYL!

Mike Hammer

mhammer@misslink.net

<paste>

From: Mike Hammer <mhammer@misslink.net>

Date: Sun, 18 May 1997 00:28:25 -0500

Subject: Re: Primary shape

At 03:23 AM 5/17/97 GMT, you wrote:

>From: Dave <djb@erie.net>

>Date: Fri, 16 May 1997 23:23:53 -0400

>Subject: Primary shape

>

>

>How do you decide what shape to make the primary? What are the pros and cons

>of each primary shape?

>

>Dave

>

Good question Dave.

There are 3 main primary coil shapes.

Solenoid (like secondary coil)

saucer

flat spiral.

The solenoid form of primary is usually twice the diameter of the

secondary or a bit larger. This primary shape often overcouples

forcing you to raise the secondary up out of the primary

field to reduce coupling. Its easy to wind and works well

if you can cope with the high degree of coupling.

If you have many turns the height of this primary can be a

disadvantage as it becomes a favorite target for your streamers.

The flat spiral is easy to wind as the boxes of soft copper

tubing come nearly already formed in this shape. The height

of this primary shape is essentially zero so is not likely to

be hit often with secondary streamers. One disadvantage is this

form will possibly undercouple especially at lower power levels.

The saucer shaped or conical shaped primary is a comprimise

between the flat spiral and solenoid shapes. It combines

the best features of both. The slope of the turns is usually

30 degrees but there is nothing sacred about that value.

The flatter you make the primary the less coupling it will provide.

This type is a little more difficult to construct.

So what type to use? Probably doesn't make that much difference.

Inductance is inductance. They will probably all provide equivalent

performance after making allowance for the difference in coupling.

Solenoid type primaries work well on small coils.

On higher power coils where over coupling may become

a problem a saucer or flat primary may be called for.

For high power coils the flat spiral is definitely the best choice.

Gary Weaver recently performed tests on a number of different

primary coils and tested them on the same secondary. He has determined that

tightly wound (small turn to turn spacing) saucer shaped

primaries with a 15 degree slope performed the best for him.

He also found that primaries wound with 1/4 inch copper tubing

with 1/4 inch turn to turn spacing gave best results.

comments and criticism always welcome

TTYL!

Mike Hammer

mhammer@misslink.net

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: newbie questions

Date: Mon, 27 Oct 1997 04:29:01 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Dave <djb@velocity.net>

Date: Sun, 26 Oct 1997 23:29:01 -0500 (EST)

Subject: Re: newbie questions

>At 07:52 PM 10/25/97 GMT, you wrote:

>>From: Dsurfr@aol.com

>>Date: Sat, 25 Oct 1997 15:52:26 -0400 (EDT)

>>Subject: newbie questions

>>

>>Being an old architect I like to work things out on paper before building

>>them. I am currently designing my first coil in over 30 years and could use

>>some advice. Since the frequency of the secondary coil is determined by the

>>length of wire used what frequency should I be trying to achieve.

Hi,

I hope you don't think that the frequency is determined by just the wire

length alone. The secondary resonates at a frequency determined by it's

inductance and self capacitance along with the capacitance of the top load.

Dave

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: newbie question

Cc:

Bcc:

X-Attachments:

At 10:46 PM 10/26/97 GMT, you wrote:

>From: Dsurfr@aol.com

>Date: Sun, 26 Oct 1997 17:46:27 -0500 (EST)

>Subject: newbie question

>

>

>I have seen a couple of different wiring diagrams on coil plans. What is the

>proper location for the capacitors & spark gap?

>

Your ascii graphics came through a little jumbled

but that happens quite often.

The main gap should be placed in shunt across the HV output

of your transformer and the cap in series.

Often older designs will show these reversed with the

cap in parallel across the transformer.

There is no performance difference between the two.

The shunt gap arrangement affords better protection

for our neons because the energized gap represents a

short circuit across the neon. This reduces the amount

of RF flowing back toward the neon.

Ill past in my famous ASCII schematic here for reference.

TESLA COIL SCHEMATIC                             ---------

|       | T1

---------

O

X1                              C1              O

RFC           | |             O  L2

O---------------||||----------| |-----        O

||O    |                   |    | |    |   L1   O

||O    |                   |           |    O   O

-----O||O    * SG                *           |--->O   O

O||O----*---|Gnd              G1             O   O

-----O||O    *                   *                O   O

120   ||O    |                   |                O   O

Volts ||O    |                   |                O   O

O---------------||||----------------------O   O

RFC                           |

|

Gnd

X1 = Neon Sign Transformer, grounded center tap

SG = Safety Gap.

RFC = RF filter chokes

G1 = Main system (tank circuit) Spark Gap.

C1 = Tank circuit capacitor

L1 = Primary Coil.

L2 = Secondary Coil.

T1 = Toroid discharge terminal

Gnd = RF Ground.

TTYL!

Mike Hammer

mhammer@misslink.net

From: tesla-2@emachine.com (Tesla-2)

Subject: Effect of Super cooling

Date: Mon, 27 Oct 1997 16:25:11 GMT

Organization: The Energy Machine Information System 504-733-3462

From: "Paul Marshall" <PMARSHAL@ETHCINC1.SSW.JNJ.COM>

Date: 27 Oct 1997 11:25:11 EST

Subject: Effect of Super cooling

Has anyone experimented with super cooling the primary / secondary.

I'm thinking about trying liquid Nitrogen to cool various components.

Maybe the gap and the capacitor could be cooled.

What are your thoughts ?

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: teslas on teslas

Date: Mon, 27 Oct 1997 17:55:32 GMT

Organization: The Energy Machine Information System 504-733-3462

From: "Thomas McGahee" <tom_mcgahee@sigmais.com>

Date: Mon, 27 Oct 1997 12:55:32 -0500

Subject: Re: teslas on teslas

----------

> From: Tesla-2 <tesla-2@emachine.com>

> To: tom_mcgahee@sigmais.com

> Subject: teslas on teslas

> Date: Sunday, October 26, 1997 11:55 PM

>

> From: tesla.coil@mailexcite.com

> Date: Sun, 26 Oct 1997 22:55:17 -0600

> Subject: teslas on teslas

> Organization: MailExcite  (http://www.mailexcite.com)

>

> I'm a beginner coiler, and at first, I haphazardly built two coils that operate

>at a mere 25kv. When I built a third, I wondered if I could use the terminals

>from my other two coils as a power source for my third coil. In other words, use

>teslas to power teslas.

>

>I tried to wire it this way, but it didn't work at all. I got a very small,

>insignificant spark off of the secondary. Does anyone know why this is?

>Has anyone ever tried to use a tesla as a power source before?

>

> I heard that the higher the input voltage, the higher the output on the

>secondary. So I wonder, is it possible to build two tesla coils and somehow use >them to power a third? Or is the electricity from the secondary not usable for this >sort of thing?

> Would it even be possible to have, say, 250kv from the terminals as an input

>voltage for another tesla coil? I read something about Tesla himself trying >something like this, but it was very hard to read (a badly scanned copy) and >understand.

>Suggestions and ideas are much welcomed!

>

> --Jay

Jay,

When we use a neon transformer to charge our caps, we are using 60 cycles per

second, so the charging current is kept fairly low. But if you want to charge a

capacitor to say, 250,000 volts from the output of a Tesla coil, you

(A) need to have a capacitor rated at 250,000 volts, and

(B) you need to be able to charge it not at a liesurely 60 times per second,

but HUNDREDS OF THOUSANDS of times per second.

(C) Tesla coils don't like to be loaded down, as it affects their resonance

(D) Even if you rectified the RF to DC and used THAT to power the next stage,

it would take an inordinate amount of time to charge any reasonable sized cap.

If the cap is small, the power that can be processed is likewise small.

(E) Conservation of energy: As you step the voltage up, the available output

current (average) will decrease such that the total power out (and wasted)

equals the total power input.

At the higher frequencies the capacitors would have a very small effective

resistance. Unless you rectified the voltage first, you would find that using

one Tesla coil to power another was very hard to do. The first Tesla coil will

be loaded by the second circuit, and probably fail miserably. It would charge

the second cap MUCH slower than the first, and if you DID get much output from

the second circuit, it would be at longer time intervals. In other words, the

time between discharges would be longer.

To my knowledge, no one has every made anything resembling a spectacular coil

by trying to cascade them in the manner you suggest.

Hope this helps.

Fr. Tom McGahee

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: teslas on teslas

Date: Mon, 27 Oct 1997 18:03:32 GMT

Organization: The Energy Machine Information System 504-733-3462

From: corr <corr@fullnet.net>

Date: Mon, 27 Oct 1997 12:03:32 -0600

Subject: Re: teslas on teslas

I believe what you are referring to is called a Tesla Coil Magnefier.

Your principal is right, but the magnifier has to be constructed in a special

way.

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Tesla coil magnifier

Date: Mon, 27 Oct 1997 20:05:18 GMT

Organization: The Energy Machine Information System 504-733-3462

From: tesla.coil@mailexcite.com

Date: Mon, 27 Oct 1997 14:05:18 -0600

Subject: Tesla coil magnifier

Organization: MailExcite  (http://www.mailexcite.com)

>From: corr <corr@fullnet.net>

>Date: Mon, 27 Oct 1997 12:03:32 -0600

>Subject: Re: teslas on teslas

>

>I believe what you are referring to is called a Tesla Coil Magnefier.

>Your principal is right, but the magnifier has to be constructed in a special

>way.

What exactly is a tesla coil magnifier? Have you ever seen/built one? I've only

heard that this device can produce a tremendous voltage but is very hard to build.

--Jay

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: Effect of Super cooling

Cc:

Bcc:

X-Attachments:

At 04:25 PM 10/27/97 GMT, you wrote:

>From: "Paul Marshall" <PMARSHAL@ETHCINC1.SSW.JNJ.COM>

>Date: 27 Oct 1997 11:25:11 EST

>Subject: Effect of Super cooling

>

>

>     Has anyone experimented with super cooling the primary / secondary.

>     I'm thinking about trying liquid Nitrogen to cool various components.

>     Maybe the gap and the capacitor could be cooled.

>

>     What are your thoughts ?

>

Well my thought is that it would unnecessarily complicate a

relatively simple system with little to no gain. You arent going

to see any type of super conductivity and the amount of heat

generated is so low as to make any type of cooling unnecessary.

The only component that generates any significant heat is the spark gap.

Here some form of positive heat removal is beneficial.

However just cooling the electrodes isnt enough. You need to remove

the hot ions clogging up the space between electrodes.

Forced air is the way to go. In fact a vacuum quench gap

with airflow supplied with a strong vacuum cleaner motor

gives nearly rotary gap performance without the complexity and

with much less strain on components.

TTYL!

Mike Hammer

mhammer@misslink.net

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: teslas on teslas

Date: Mon, 27 Oct 1997 21:16:40 GMT

Organization: The Energy Machine Information System 504-733-3462

From: "Thomas McGahee" <tom_mcgahee@sigmais.com>

Date: Mon, 27 Oct 1997 16:16:40 -0500

Subject: Re: teslas on teslas

----------

> From: Tesla-2 <tesla-2@emachine.com>

> To: tom_mcgahee@sigmais.com

> Subject: Re: teslas on teslas

> Date: Monday, October 27, 1997 1:03 PM

>

> From: corr <corr@fullnet.net>

> Date: Mon, 27 Oct 1997 12:03:32 -0600

> Subject: Re: teslas on teslas

>

> I believe what you are referring to is called a Tesla Coil Magnefier.

> Your principal is right, but the magnifier has to be constructed in a special

> way.

>

>

Fellow Coilers,

We have to be careful with terms here or we can give people the wrong idea. A

Magnifier is NOT a Tesla coil driving a Tesla coil. It is a circuit in which

the first part uses TRANSFORMER action to BASE drive a resonator. The first

part of the Magnifier is designed to provide a large RF CURRENT at reasonable

voltage. This RF current is then used to base drive a coil (and its attendant

capacitance in terms of Cself and topload) that is resonant with the frequency

of the driver.

In his post Jay is talking about stepping up the voltage and then using that

voltage to step up another stage. This is possible with transformers, but the

Tesla coil is NOT a transformer in the usual sense. The Tesla coil may LOOK

like a transformer, and in the past many authors have mistakenly talked about

it in those terms, but it is another thing entirely. The high output voltage of

a Tesla coil is more the result of resonant action than it is the result of

transformer action. That is why you can't calculate the output of a Tesla coil

by simply multiplying the input voltage times the turns ratio.

Tesla coils do NOT like to be loaded down. Loading definitely affects their

resonant frequency when the load is capacitive, and and any OTHER sort of load

cuts the Q of the circuit and drastically lowers output voltage. Load the

sucker down too much and it ceases to be a Tesla coil altogether.

Another thing we all have to keep in mind is that the total power out of a

Tesla coil will never exceed the total power put in. The PEAK power can indeed

be made significantly larger than the PEAK power input, but the average power

in will equal the average power out (assuming all losses are correctly

accounted for).

The Tesla coil is not in any way a Free Energy device. It IS a magnificent way

to produce obscene high voltages and beautiful long arcs of dancing

electricity. But don't expect that you can connect a 9 volt radio battery up to

it and power your house. I *CAN* get a 100,000 volt bolt of electricity out of

a properly constructed Tesla coil driven SOLELY by a 9 volt radio battery. But

it will only produce such bolts in a sort of single-shot mode after many

seconds of time converting the 9 volts to a couple of thousand volts and using

that to charge a capacitor. It is only the STORED energy being dumped into a

resonant circuit that will work the Tesla Coil Magic.

Can you drive a Tesla Coil with another Tesla Coil? Of course. Electricity is

electricity. But don't expect too much from the experiment. Rectify the output

of the first Tesla coil and use that to charge a capacitor which is spark gap

connected to the second Tesla Coil section. Why should you rectify it? Well, if

the first Tesla coil is operating at say 100Khz, then you will be trying to

charge the second capacitor at a 100Khz rate. The capacitor will either be

large and look like a dead short to the RF, or it will be small and during the

00001 second only accumulate a small charge.

Bottom line is that cascading Tesla coils just does not work well in real life.

I would really hate to see someone waste an inordinate amount of time chasing a

pipe dream, when they could better use their time learning how to build a

really GOOD Tesla Coil or Magnifier instead.

Hope this helps.

Fr. Tom McGahee

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: Tesla coil magnifier

Cc:

Bcc:

X-Attachments:

At 08:05 PM 10/27/97 GMT, you wrote:

>From: tesla.coil@mailexcite.com

>Date: Mon, 27 Oct 1997 14:05:18 -0600

>Subject: Tesla coil magnifier

>Organization: MailExcite  (http://www.mailexcite.com)

>

>>From: corr <corr@fullnet.net>

>>Date: Mon, 27 Oct 1997 12:03:32 -0600

>>Subject: Re: teslas on teslas

>>

>>I believe what you are referring to is called a Tesla Coil Magnefier.

>>Your principal is right, but the magnifier has to be constructed in a special

>>way.

>

>

>What exactly is a tesla coil magnifier? Have you ever seen/built one? I've only

>heard that this device can produce a tremendous voltage but is very hard to build.

>

>--Jay

Jay,

Tesla's magnifier is nothing magical. It is merely a more efficient form

of coil. It really offers no particular advantage in small sizes.

It gets its increase in efficiency in that the resonator is not

magneticly coupled to the rest of the system.

It has a specially constructed primary/secondary system that is very

tightly coupled. The output from this is fed to the base of the tertiary

or "extra" coil via a transmission line. This means it can be placed some

distance away from the primary/secondary coil away from their respective

magnetic fields. This allows the voltage to rise without interference.

Magnifiers are not often built because of the complexity and need for

high quality components. You really need top flight caps and a very good gap

system to make a magnifier work. The very tight coupling calls for special

measures to insulate against arc overs.

TTYL!

Mike Hammer

mhammer@misslink.net

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: Effect of Super cooling

Date: Mon, 27 Oct 1997 21:25:22 GMT

Organization: The Energy Machine Information System 504-733-3462

From: "Thomas McGahee" <tom_mcgahee@sigmais.com>

Date: Mon, 27 Oct 1997 16:25:22 -0500

Subject: Re: Effect of Super cooling

----------

> From: Tesla-2 <tesla-2@emachine.com>

> To: tom_mcgahee@sigmais.com

> Subject: Re: Effect of Super cooling

> Date: Monday, October 27, 1997 3:52 PM

>

> From: Mike Hammer <mhammer@misslink.net>

> Date: Mon, 27 Oct 1997 14:52:18 -0600

> Subject: Re: Effect of Super cooling

>

> At 04:25 PM 10/27/97 GMT, you wrote:

> >From: "Paul Marshall" <PMARSHAL@ETHCINC1.SSW.JNJ.COM>

> >Date: 27 Oct 1997 11:25:11 EST

> >Subject: Effect of Super cooling

> >

> >

> >     Has anyone experimented with super cooling the primary / secondary.

> >     I'm thinking about trying liquid Nitrogen to cool various components.

> >     Maybe the gap and the capacitor could be cooled.

> >

> >     What are your thoughts ?

> >

>

> Well my thought is that it would unnecessarily complicate a

> relatively simple system with little to no gain. You arent going

> to see any type of super conductivity and the amount of heat

> generated is so low as to make any type of cooling unnecessary.

>

> The only component that generates any significant heat is the spark gap.

> Here some form of positive heat removal is beneficial.

> However just cooling the electrodes isnt enough. You need to remove

> the hot ions clogging up the space between electrodes.

> Forced air is the way to go. In fact a vacuum quench gap

> with airflow supplied with a strong vacuum cleaner motor

> gives nearly rotary gap performance without the complexity and

> with much less strain on components.

>

> TTYL!

> Mike Hammer

> mhammer@misslink.net

>

>

Hi, Mike, it's me again :)

There *IS* some utility to supercooling the primary and also in the case of the

secondary. In fact Tesla even received a patent on the idea in connection with

his Wardencliffe project. Specifically the supercooling reduces the resistance

and therefore increases the Q of the circuit. Mike is right in that for the

average coiler the gain would be insignificant. But on the HUGE scale that

Tesla was envisioning, it was a gain worth the pain.

For what it's worth...

Fr. Tom McGahee

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Rebuild of Transco Neon

Date: Wed, 29 Oct 1997 21:33:06 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Kevin Batchelor <kbatchelor@arslimited.com>

Date: Wed, 29 Oct 1997 16:33:06 -0500

Subject: Rebuild of Transco Neon

Dear Group:

I am working on disassembly of a Transco 15Kv 60ma neon.  It appears to be a

poor choice in this category.  The core appears to only be disassembled by taki

ng apart each layer of the laminated core sections.  Needless to say this is a

giant Chinese Jigsaw Puzzle and I don't recommend this to anyone.  I can not gu

arantee that once disassembled, it can be re-assembled...or at least not withou

t great effort, frustration, and many curses.  I did however find that gasoline

worked quite effectively at dissolving the tar that the transformer was packed in.

Anyone wishing to donate a 12 or 15Kv 60ma France or Allanson transformer to

my coiling efforts...please feel free to do so.

Additionally, anyone with any suggestions or rebuild stories who would like t

o share these with the group, please post them.

Thank You,

Kevin Batchelor

KBATCHELOR@ARSLIMITED.COM

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: Rebuild of Transco Neon

Date: Thu, 30 Oct 1997 05:15:21 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Chris Federico <cfed@comsource.net>

Date: Wed, 29 Oct 1997 23:15:21 -0600

Subject: Re: Rebuild of Transco Neon

Maybe this is old news to the list, but after seeing a couple of posts about

going to salvage yards to find neon xfrmrs, I thought I'd mention that I got

mine (used) at the first neon sign place I called.  $35 for a France 15kv

30ma, that was tested.  They've also been real good about selling me hook-up

wire at cost, more than once (I'm starting on my third re-wiring effort in

the morning, and haven't gotten a spark out of the thing yet!)

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: Rebuild of Transco Neon

Date: Thu, 30 Oct 1997 15:11:52 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Eric Davidson <edavidson@icva.gov>

Date: Thu, 30 Oct 1997 09:11:52 -0600

Subject: Re: Rebuild of Transco Neon

Kevin:

In my humble opinion the rebuilding of a neon sign transformer should

only be considered as an absolute LAST resort. It is a dirty, filthy and

frustrating job. When you are all done you cross your fingers and plug

it in. It's not worth it. Good quality used ones are not that hard to

find. When you are up to your elbows in asphalt and gasoline (I would

recommend kerosene, much less dangerous), I would be interested to know

how $150.00 for a brand new tranny would sound. I have heard of people

rebuilding them with good results but I will NEVER try it again. I don't

mean to criticize you for your efforts, I just hate to see people

undertake this nasty job. NSTs are not designed to be rebuilt, they

serve their useful life, which may be dozens of years, and then they are

discarded.  Most scrap yards don't even want them because of the potting

material and the labor required to salvage the iron and copper. My

advice is to keep looking, you WILL find one. I would seriously consider

buying a new one too. I know that's easy to say, but its very satisfying

to know you have a fully functional and tested unit, that will provide

years and years of coiling pleasure! :-)

Eric <edavidson@icva.gov>

Tesla-2 wrote:

>

> From: Kevin Batchelor <kbatchelor@arslimited.com>

> Date: Wed, 29 Oct 1997 16:33:06 -0500

> Subject: Rebuild of Transco Neon

>

> Dear Group:

>   I am working on disassembly of a Transco 15Kv 60ma neon.  It appears to be a

>poor choice in this category.  The core appears to only be disassembled by taking >apart each layer of the laminated core sections.  Needless to say this is a

>giant Chinese Jigsaw Puzzle and I don't recommend this to anyone.  I can not

>guarantee that once disassembled, it can be re-assembled...or at least not with

>out great effort, frustration, and many curses.  I did however find that gasoline

>worked quite effectively at dissolving the tar that the transformer was packed in.

>

>  Anyone wishing to donate a 12 or 15Kv 60ma France or Allanson transformer to

> my coiling efforts...please feel free to do so.

>

>  Additionally, anyone with any suggestions or rebuild stories who would like to >share these with the group, please post them.

>

> Thank You,

> Kevin Batchelor

> KBATCHELOR@ARSLIMITED.COM

>

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: Rebuild of Transco Neon

Cc:

Bcc:

X-Attachments:

At 03:11 PM 10/30/97 GMT, you wrote:

>From: Eric Davidson <edavidson@icva.gov>

>Date: Thu, 30 Oct 1997 09:11:52 -0600

>Subject: Re: Rebuild of Transco Neon

>

>Kevin:

>

>In my humble opinion the rebuilding of a neon sign transformer should

>only be considered as an absolute LAST resort. It is a dirty, filthy and

>frustrating job. When you are all done you cross your fingers and plug

>it in. It's not worth it. Good quality used ones are not that hard to

>find. When you are up to your elbows in asphalt and gasoline (I would

>recommend kerosene, much less dangerous), I would be interested to know

>how $150.00 for a brand new tranny would sound. I have heard of people

>rebuilding them with good results but I will NEVER try it again. I don't

>mean to criticize you for your efforts, I just hate to see people

>undertake this nasty job. NSTs are not designed to be rebuilt, they

>serve their useful life, which may be dozens of years, and then they are

>discarded.  Most scrap yards don't even want them because of the potting

>material and the labor required to salvage the iron and copper. My

>advice is to keep looking, you WILL find one. I would seriously consider

>buying a new one too. I know that's easy to say, but its very satisfying

>to know you have a fully functional and tested unit, that will provide

>years and years of coiling pleasure! :-)

>

>Eric <edavidson@icva.gov>

>

Eric,

I have to disagree with you on this. I do not advise people to

buy new neons. Just too much money for something that can be

destroyed in an instant. And functional used units are in

short supply in many areas. There are no used neons within

a 75 mile radius of me. I have bought all there was. And it

wasnt very many. Over 1/2 of them were dead and I unpotted

and restored them all.

What I ended up with was better transformers for much lower

cost. You can easily remove a few of the magnetic shunts

and realize 45 ma output from 30 ma neons and 90 from the 60s.

Once the tar potting compound is removed you have eliminated

the number one failure mode - carbon tracking through the

potting compound. Very often this is all it takes to restore

a dead transformer.

It easy to replace the insulation with more robust materials

or simply plop it into a container of transformer oil.

A rebuilt neon in oil becomes almost indestructible.

I dont use solvents to dissolve the tar. I excavate out the cores

by chipping away the potting compound. I use a little kerosene

at the end for a final cleanup.

If Im not mistaken Kevin's Transco he is unpotting WAS a brand

new unit that failed in TC use after a short time despite the

use of RF chokes and a safety gap. Dont buy new neons.

They are way too fragile in Tesla coil use.

TTYL!

Mike Hammer

mhammer@misslink.net

From: tesla-2@emachine.com (Tesla-2)

Subject: tesla tuner

Date: Thu, 30 Oct 1997 21:43:34 GMT

Organization: The Energy Machine Information System 504-733-3462

From: "Eriksson" <jan-erik.eriksson@mbox2.swipnet.se>

Date: Thu, 30 Oct 1997 22:43:34 +0100

Subject: tesla tuner

Hi!

I was just taking a look in my Information Unlimited catalog when I read

about a "Tesla coil tuner". It said, "Determines frequency and capacitance

of your coil without any physical contact!".

Can you use this for tuning your coil like a variable capacitor, or just for

determining the frequency and capacitance? This may be a dumb question but I

don't know anyone else to ask this question. And, are there any circuit

schematics for this on the web?

Thanks

Christopher Eriksson

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: tesla tuner

Cc:

Bcc:

X-Attachments:

At 09:43 PM 10/30/97 GMT, you wrote:

>From: "Eriksson" <jan-erik.eriksson@mbox2.swipnet.se>

>Date: Thu, 30 Oct 1997 22:43:34 +0100

>Subject: tesla tuner

>

>

>

>Hi!

>

>I was just taking a look in my Information Unlimited catalog when I read

>about a "Tesla coil tuner". It said, "Determines frequency and capacitanc>e

>of your coil without any physical contact!".

>

>Can you use this for tuning your coil like a variable capacitor, or just for

>determining the frequency and capacitance? This may be a dumb question but I

>don't know anyone else to ask this question. And, are there any circuit

>schematics for this on the web?

>

>Thanks

>

>Christopher Eriksson

>

While I dont have direct knowledge of this device experience

has shown that many items offered by Information Unlimited

are less than as advertised.

The description sounds like it might be some type of wavemeter

or dip meter to determine the frequency of the resonant circuits.

It is not a tuning adjunct to be used on your coil like a variable

capacitor.

My advice is to pass on this item.

TTYL!

Mike Hammer

mhammer@misslink.net

From: tesla-2@emachine.com (Tesla-2)

Subject: toroid height

Date: Fri, 31 Oct 1997 03:41:28 GMT

Organization: The Energy Machine Information System 504-733-3462

From: "Chris Watkins" <cwatkins@seidata.com>

Date: Thu, 30 Oct 1997 22:41:28 -0500

Subject: torid height

Hi.

I was wondering about the height of my toroid in relation to the secondary.

will I need to be able to change it for tuning or does it really matter that much.

If it does, is this due to magnetic coupling to the secondary or what?

any comments appreciated.

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: toroid height

Cc:

Bcc:

X-Attachments:

At 03:41 AM 10/31/97 GMT, you wrote:

>From: "Chris Watkins" <cwatkins@seidata.com>

>Date: Thu, 30 Oct 1997 22:41:28 -0500

>Subject: torid height

>

>Hi.

> I was wondering about the height of my toroid in relation to the secondary.

>will I need to be able to change it for tuning or does it really matter that much.

>If it does, is this due to magnetic coupling to the secondary or what?

>any comments appreciated.

>

You want your toroid down very close to the top of

your secondary. Mounted in this way it will suppress

any corona from forming near the top turns of the secondary.

Without a toroid you can often have streamers break out

from the top secondary turns. A low mounted toroid will

prevent that.

The capacitance of the toroid will vary some as its height

is changed. This will affect your secondary resonant frequency

somewhat. The best height is usually determined experimentally.

Make sure when you change the height you retune for maximum

ouput.

TTYL!

Mike Hammer

mhammer@misslink.net

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: toroid height

Date: Fri, 31 Oct 1997 07:25:25 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Bert Hickman <bert.hickman@aquila.com>

Date: Thu, 30 Oct 1997 23:25:25 -0800

Subject: Re: toroid height

Organization: Stoneridge Engineering

Tesla-2 wrote:

>

> From: "Chris Watkins" <cwatkins@seidata.com>

> Date: Thu, 30 Oct 1997 22:41:28 -0500

> Subject: torid height

>

> Hi.

>  I was wondering about the height of my torid in relation to the secondary.

>will I need to be able to change it for tuning or does it really matter that much.

>If it does, is this due to magnetic coupling to the secondary or what?

>any comments appreciated.

>

Moving the toroid higher increases its effective capacitance a bit,

reduces it's protective "shielding" of the top turns on the secondary a

bit, and also increases the breakdown voltage between the toroid and the

primary/strike-ring a bit. Because of the slight increase in

capacitance, it will affect tuning slightly. However, at the top of the

secondary, virtually all the effects are due to electrostatics and

electrostatic field shaping, not magnetic coupling. Hope this helps, and

safe coilin' to you Chris!

-- Bert --

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Changes in resonate frequency

Date: Fri, 31 Oct 1997 13:38:08 GMT

Organization: The Energy Machine Information System 504-733-3462

From: "Paul Marshall" <PMARSHAL@ETHCINC1.SSW.JNJ.COM>

Date: 31 Oct 1997 08:38:08 EST

Subject: Changes in resonate frequency

How do I compensate for changes in resonant frequency due to the

addition of my toroid ? I plan to make the toroid from dryer duct.

Assuming the overall diameter of the toroid is 12" and the diameter of

the dryer duct is 4" , how will this effect my resonant frequency ?

Can I simply change the tap on my primary ?

My secondary is wound using 22 AWG on a 4.5" dia. form for a length of

23" Input voltage is 15 kv at a max current of 180 ma. Primary cap is

.02 mf (two of these in series) I will initially begin using 60ma and

a variac. Power up is planned for 11/15/97. Wish me luck.

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: RF chokes

Date: Fri, 31 Oct 1997 18:32:49 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Chris Federico <cfed@comsource.net>

Date: Fri, 31 Oct 1997 12:32:49 -0600

Subject: RF chokes

I need some suggestions on how to build a couple of RF chokes to protect my

xfrmr.  I built one for the a/c -in to protect the house circuits but I'm not

sure what one should look like for the xfrmr.

Thanks

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Primary design based on power input ?

Date: Fri, 31 Oct 1997 18:37:13 GMT

Organization: The Energy Machine Information System 504-733-3462

From: "Paul Marshall" <PMARSHAL@ETHCINC1.SSW.JNJ.COM>

Date: 31 Oct 1997 13:37:13 EST

Subject: Primary design based on power input ?

There are three basic primary designs:

Cylindrical

Flat spiral

Conical

Which works best at low input power 500 w or <

Which works best at medium power input 1200 or <

which works best at high power > 3000 W

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: RF chokes

Date: Fri, 31 Oct 1997 19:25:14 GMT

Organization: The Energy Machine Information System 504-733-3462

From: Kevin Batchelor <kbatchelor@arslimited.com>

Date: Fri, 31 Oct 1997 14:25:14 -0500

Subject: Re: RF chokes

Chris:

My chokes are about 85+ turns of magnet wire on a ferrite core.  They seem to

work pretty well.  Hard part will be finding the available core to wind.  You

could just make some air chokes...not sure what the specs are.  Someone will

post them here.

Don't forget that safety gap!  The chokes will help but without a safety gap

for those big kickbacks...you'll lose a good transformer in no time.

Kevin

>>> Tesla-2 <tesla-2@emachine.com> 10/31 1:32 PM >>>

From: Chris Federico <cfed@comsource.net>

Date: Fri, 31 Oct 1997 12:32:49 -0600

Subject: RF chokes

I need some suggestions on how to build a couple of RF chokes to protect my

xfrmr.  I built one for the a/c -in to protect the house circuits but I'm not

sure what one should look like for the xfrmr.

Thanks

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: RF chokes

Cc:

Bcc:

X-Attachments:

At 06:32 PM 10/31/97 GMT, you wrote:

>From: Chris Federico <cfed@comsource.net>

>Date: Fri, 31 Oct 1997 12:32:49 -0600

>Subject: RF chokes

>

>

>

>I need some suggestions on how to build a couple of RF chokes to protect my

>xfrmr.  I built one for the a/c -in to protect the house circuits but I'm not

>sure what one should look like for the xfrmr.

>

>Thanks

>

Chris,

What you need in terms of a RF choke is something to

knock down the level of the RF trying to sneak its way

into your neon secondary. You will NOT eliminate this RF.

That would require a much more extensive filter. But you really

dont need to. A little RF wont do any harm but lots of RF

and high voltage spikes will. They will erode away at the

insulation of your neon secondary and cause premature failure.

What you need is something of the order of 2-10 mh.

You need two, one in each HV leg.

The best approach is to wind some chokes on ferrite cores

of some kind. Ferrite toroids are good as well as ferrite rods.

My chokes consist of 125 turns of AWG 26 on 2.5 inch diameter

surplus ferrite toroids. Unfortunately the source for these toroids

has dried up. I do see them often at hamfests.

The inductance is about 2.5 mh.

If you use ferrite cores insulate them with several layers of

good quality vinyl tape. Ferrites are somewhat conductive

and will short out if you dont insulate them.

As an alternative you can wind air core chokes on 2 inch

PVC pipe. A 6 inch long winding of AWG 26 will have about

350 turns and 2.5 mh of inductance. The inductance value

is not at all critical.

Sometimes air core chokes with many turns will ring up

and generate high voltages like mini Tesla coils. Just

what you are trying to prevent. In this case its advisable

to use a little series resistance with these chokes to dampen

any oscillations. Power resistors of at least 10 watts

and values of from 300-1500 ohms will do the job for

small neon powered coils. For larger coils higher power

resistors will be needed. These will introduce some loss

but it is a rather small amount as the current flow is low.

Do not neglect the safety gap. This is even more important

than RF chokes. The RF chokes go in series with the HV lines

in between the safety gap and main gap. Set your safety gap

so it is firing intermittantly. If it isnt firing it isnt

limiting the voltage and isnt protecting your transformer.

TTYL!

Mike Hammer

mhammer@misslink.net

To: tesla-2@emachine.com (Tesla-2)

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: Primary design based on power input ?

Cc:

Bcc:

X-Attachments:

At 06:37 PM 10/31/97 GMT, you wrote:

>From: "Paul Marshall" <PMARSHAL@ETHCINC1.SSW.JNJ.COM>

>Date: 31 Oct 1997 13:37:13 EST

>Subject: Primary design based on power input ?

>

>

>     There are three basic primary designs:

>     Cylindrical

>     Flat spiral

>     Conical

>

>     Which works best at low input power 500 w or <

>     Which works best at medium power input 1200 or <

>     which works best at high power > 3000 W

>

Paul,

In my opinion it really doesnt matter that much.

Any of the 3 basic primary shapes can be made to

perform well at various power levels. The various shapes

differ only in the amount of coupling they provide.

That said I would recommend a regular cylindrical solenoid

for small low power coils that often need all the coupling they

can get.

Conical shapes for medium power coils where overcoupling

with a solenoid will be a problem and flat spirals for

high power coils.

More important in any primary design is to make sure you

have plenty of turns available. Minimum of 12. 14 would be better.

And make allowance for changing the pri/sec spacing to adjust

the coupling.

TTYL!