mpeg2encode

===========

MPEG-2 Video Encoder, Version 1.0, May 1994

MPEG Software Simulation Group

(MPEG-L@netcom.com)

Contents:

=========

1. Overview

2. Features and Limitations

3. Usage

4. Interpreting the status information

5. Description of the Encoder Model

6. References

1. Overview

===========

This is the first public release of our MPEG-2 encoder. It converts an

ordered set of uncompressed input pictures into a compressed bitstream

compliant with ISO/IEC 13818-2 DIS [1] (MPEG-2).

This program will evolve to become: ISO/IEC 13818-5 Software Simulation

of MPEG-2 Systems, Video, and Audio.

2. Features and Limitations

===========================

2.1 Features

- generates constant bit rate streams

- encoder model based on MPEG-2 test model 5 (TM5) rev. 2 [2]

- progressive and interlaced video

- also generates ISO/IEC 11172-2 (MPEG-1) sequences

- input formats: separate YUV, combined YUV, PPM

- trace and statistics output

- verifies user parameter settings are legal within Profile and Level

2.2 Current limitations

The encoder currently does not support

- scalable extensions

- field-coded pictures

- generation of dual prime motion vectors

- MPEG-1 integer pel vectors (half-pel is typically 1 dB better anyway)

and D frame sequences.

- checking for maximum number of generated bits per macroblock

- automatic 3:2 pulldown detection or irregular 3:2 pulldown signaling.

- intra refresh slices (e.g. low delay)

- concealment motion vectors

- special low delay mode rate control

- editing of encoded video

- variable bit rate encoding

- scene change rate control

3. Usage

========

The execution template for the encoder is:

mpeg2encode parameter_file output.m2v

Coding parameters can be modified by editing the parameter_file. Since the

parser expects the operating parameters to be on certain line numbers,

kindly do not insert or delete lines from the file. Lines are

terminated by ^M (Control-M).

The first line of the parameter file is a comment which is inserted near

the beginning of the MPEG bitstream as a user_data field, and can be

used for arbitrary purposes.

The remaining lines are described below:

/* name of source frame files */

A printf format string defining the name of the input files. It has to

contain exactly one numerical descriptor (%d, %x etc.):

Example: frame%02d

Then the encoder looks for files: frame00, frame01, frame02

The encoder adds an extension (.yuv, .ppm, etc.) which depends on the

input file format. Input files have to be in frame format, containing

two interleaved fields (for interlaced video).

/* name of reconstructed frame files */

This user parameter tells the encoder what name to give the reconstructed

frames. These frames are identical to frame reconstructions of decoders

following normative guidelines (except of course for differences caused by

different IDCT implementation). Specifying a name starting with - (or just

- by itself) disables output of reconstructed frames.

The reconstructed frames are always stored in Y,U,V format (see below),

independent of the input file format.

/* name of intra quant matrix file     ("-": default matrix) */

Setting this to a value other than  -  specifies a file containing

a custom intra quantization matrix to be used instead of the default

matrix specified in ISO/IEC 13818-2 and 11172-2. This file has to contain

64 integer values (range 1...255) separated by white space (blank, tab,

or newline), one corresponding  to each of the 64 DCT coefficients. They

are ordered line by line, i.e. v-u frequency matrix order (not by the

zig-zag pattern used for transmission). The file intra.mat contains the

default matrix as a starting point for customization. It is neither

necessary or recommended to specify the default matrix explicitly.

Large values correspond to coarse quantization and consequently more

noise at that particular spatial frequency.

For the intra quantization matrix, the first value in the file (DC value)

is ignored. Use the parameter intra_dc_precision (see below) to define

the quantization of the DC value.

/* name of non intra quant matrix file ("-": default matrix) */

This parameter field follows the same rules as described for the above

intra quant matrix parameter, but specifies the file for the NON-INTRA

coded (predicted / interpolated) blocks. In this case the first

coefficient of the matrix is NOT ignored.

The default matrix uses a constant value of 16 for all 64 coefficients.

(a flat matrix is thought to statistically minimize mean square error).

The file inter.mat contains an alternate matrix, used in the MPEG-2 test

model.

/* name of statistics file */

Statistics output is stored into the specified file. - directs statistics

output to stdout.

/* input picture file format */

A number defining the format of the source input frames.

Code  Format description

----  ------------------------------------------------------------------

0     separate files for luminance (.Y extension), and chrominance (.U, .V)

all files are in headerless 8 bit per pixel format .U and .V must

have half horizontal and vertical resolution (4:2:0).  Note

that in this document, Cb = U, and Cr = V. This format is also

used in the Stanford encoder.

1     similar to 0, but concatenated into one file (extension .yuv)

this format is used by the Berkeley MPEG-1 encoder

2     PPM, Portable PixMap, only the raw format (P6) is supported

/* number of frames */

This defines the length of the sequence in integer units of frames.

/* number of first frame */

Usually 0 or 1, but any other (positive) value is valid.

/* timecode of first frame */

This line is used to set the timecode encoded into the first 'Group of

Pictures' header. The format is based on the SMPTE style:

hh:mm:ss:ff (hh=hour, mm=minute, ss=second, ff=frame (0..picture_rate-1)

/* N (# of frames in GOP) */

This defines the distance between I frames (and 'Group of Pictures'

headers). Common values are 15 for 30 Hz video and 12 for 25 Hz video.

/* M (I/P frame distance) */

Distance between consecutive I or P frames. Usually set to 3.

N has to be a multiple of M.   M = 1 means no B frames in the sequence.

(in a future edition of this program, M=0 will mean only I frames).

/* ISO/IEC 11172-2 stream */

Set to 1 if you want to generate an MPEG-1 sequence. In this case some

of the subsequent MPEG-2 specific values are ignored.

/* horizontal_size */

Pixel width of the frames. It does not need to be a multiple of 16.

You have to provide a correct value even for PPM files (the PPM

file header is currently ignored).

/* vertical_size */

Pixel height of the frames. It does not need to be a multiple of 16.

You have to provide a correct value even for PPM files (the PPM file

header is currently ignored).

/* aspect_ratio_information */

Defines the display aspect ratio. Legal values are:

Code    Meaning

----    --------------

1       square pels

2       4:3 display

3       16:9 display

4       2.21:1 display

MPEG-1 uses a different coding of aspect ratios. In this cases codes

1 to 14 are valid.

/* frame_rate_code */

Defines the frame rate (for interlaced sequences: field rate is twice

the frame rate). Legal values are:

Code  Frames/sec    Meaning

----  ----------    -----------------------------------------------

1     24000/1001    23.976 fps -- NTSC encapsulated film rate

2     24            Standard international cinema film rate

3     25            PAL (625/50) video frame rate

4     30000/1001    29.97 -- NTSC video frame rate

5     30            NTSC drop-frame (525/60) video frame rate

6     50            double frame rate/progressive PAL

7     60000/1001    double frame rate NTSC

8     60            double frame rate drop-frame NTSC

/* bit_rate */

A positive floating point value specifying the target bitrate.

In units of bits/sec.

/* vbv_buffer_size (in multiples 16 kbit) */

Specifies, according to the Video Buffering Verifier decoder model,

the size of the bitstream input buffer required in downstream

decoders in order for the sequence to be decoded without underflows or

or overflows.  You probably will wish to leave this value at 112 for

MPEG-2 Main Profile at Main Level, and 20 for Constrained Parameters

Bitstreams MPEG-1.

/* low_delay */

When set to 1, this flag specifies whether encoder operates in low delay

mode.  Essentially, no B pictures are coded and a different rate control

strategy is adopted which allows picture skipping and VBV underflows.

This feature has not yet been implemented. Please leave at zero for now.

/* constrained_parameters_flag */

Always 0 for MPEG-2. You may set this to 1 if you encode an MPEG-1

sequence which meets the parameter limits defined in ISO/IEC 11172-2

for constrained parameter bitstreams:

horizontal_size <= 768

vertical_size   <= 576

picture_area    <= 396 macroblocks

pixel_rate      <= 396x25 macroblocks per second

vbv_buffer_size <= 20x16384 bit

bitrate         <= 1856000 bits/second

motion vector range <= -64...63.5

/* Profile ID */

Specifies the subset of the MPEG-2 syntax required for decoding the

sequence. All MPEG-2 sequences generated by the current version of

the encoder are either Main Profile or Simple Profile sequences.

Code  Meaning                       Typical use

----  --------------------------    ------------------------

1     High Profile                  production equipment requiring 4:2:2

2     Spatially Scalable Profile    Simulcasting

3     SNR Scalable Profile          Simulcasting

4     Main Profile                  95 % of TVs, VCRs, cable applications

5     Simple Profile                Low cost memory, e.g. no B pictures

/* Level ID */

Specifies coded parameter constraints, such as bitrate, sample rate, and

maximum allowed motion vector range.

Code  Meaning         Typical use

----  --------------- -----------------------------------------------

4     High Level      HDTV production rates: e.g. 1920 x 1080 x 30 Hz

6     High 1440 Level HDTV consumer rates: e.g. 1440 x 960 x 30 Hz

8     Main Level      CCIR 601 rates: e.g. 720 x 480 x 30 Hz

10    Low Level       SIF video rate: e.g. 352 x 240 x 30 Hz

/* progressive_sequence */

0 in the case of a sequences containing interlaced video (e.g.

video camera source), 1 for progressive video (e.g. film source).

/* video_format: 0=comp., 1=PAL, 2=NTSC, 3=SECAM, 4=MAC, 5=unspec. */

/* color_primaries */

Specifies the x, y chromaticity coordinates of the source primaries.

Code   Meaning

----   -------

1      ITU-R Rec. 709 (1990)

2      unspecified

4      ITU-R Rec. 624-4 System M

5      ITU-R Rec. 624-4 System B, G

6      SMPTE 170M

7      SMPTE 240M (1987)

/* transfer_characteristics */

Specifies the opto-electronic transfer characteristic of the source picture.

Code   Meaning

----   -------

1      ITU-R Rec. 709 (1990)

2      unspecified

4      ITU-R Rec. 624-4 System M

5      ITU-R Rec. 624-4 System B, G

6      SMPTE 170M

7      SMPTE 240M (1987)

8      linear transfer characteristics

/* matrix_coefficients */

Specifies the matrix coefficients used in deriving luminance and chrominance

signals from the green, blue, and red primaries.

Code   Meaning

----   -------

1      ITU-R Rec. 709 (1990)

2      unspecified

4      FCC

5      ITU-R Rec. 624-4 System B, G

6      SMPTE 170M

7      SMPTE 240M (1987)

/* display_horizontal_size */

/* display_vertical_size */

Display_horizontal_size and display_vertical_size specify the "intended

display's" active region (which may be smaller or larger than the

encoded frame size).

/* intra_dc_precision */

Specifies the effective precision of the DC coefficient in MPEG-2

intra coded macroblocks. 10-bits usually achieves quality saturation.

Code    Meaning

----    -----------------

0       8 bit

1       9 bit

2       10 bit

3       11 bit

/* top_field_first */

Specifies which of the two fields of an interlaced frame comes earlier.

The top field corresponds to what is often called the "odd field," and

the bottom field is also sometimes called the "even field."

Code  Meaning

----  -----------------

0     bottom field first

1     top field first

/* frame_pred_frame_dct (I P B) */

Setting this parameter to 1 restricts motion compensation to frame

prediction and DCT to frame DCT. You have to specify this separately for

I, P and B picture types.

/* concealment_motion_vectors (I P B) */

Setting these three flags informs encoder whether or not to generate

concealment motion vectors for intra coded macroblocks in the

three respective coded picture types.  This feature is mostly useful

in Intra-coded pictures, but may also be used in low-delay applications

(which attempts to exclusively use P pictures for video signal refresh,

saving the time it takes to download a coded Intra picture across a

channel). concealment_motion_vectors in B pictures are rather pointless

since there is no error propagation from B pictures. This feature is

currently not implemented.  Please leave values at zero.

/* q_scale_type (I P B) */

These flag sets linear (0) or non-linear (1) quantization scale type

for the three respective picture types. This feature has not been

integrated yet (modification to rate control is necessary).  Please

leave flags at zero for now.

/* intra_vlc_format (I P B) */

Selects one of the two variable length coding tables for intra coded blocks.

Table 1 is considered to be statistically optimized for Intra coded

pictures coded within the sweet spot range (e.g. 0.3 to 0.6 bit/pixel)

of MPEG-2.

Code  Meaning

----  -----------------

0     table 0 (= MPEG-1)

1     table 1

/* alternate_scan (I P B) */

Selects one of two entropy scanning patterns defining the order in

which quantized DCT coefficients are run-length coded.  The alternate

scanning pattern is considered to be better suited for interlaced video

where the encoder does not employ sophisticated forward quantization

(as is the case in our current encoder).

Code  Meaning

----  -----------------

0     Zig-Zag scan (= MPEG-1)

1     Alternate scan

/* repeat_first_field */

If set to one, the first field of a frame is repeated after the second by

the display process. The exact function depends on progressive_sequence

and top_field_first.  repeat_first_field is mainly intended to serve as

a signal for the Decoder's Display Process to perform 3:2 pulldown.

/* progressive_frame */

Specifies whether the frames are interlaced (0) or progressive (1).

MPEG-2 permits mixing of interlaced and progressive video. The encoder

currently only supports either interlaced or progressive video.

progressive_frame is therefore constant for all frames and usually

set identical to progressive_sequence.

/* intra_slice refresh picture period (P factor) */

This value indicates the number of successive P pictures in which

all slices (macroblock rows in our encoder model) are refreshed

with intra coded macroblocks.  This feature assists low delay mode

coding.  It is currently not implemented.

/* rate control: r (reaction parameter) */

/* rate control: avg_act (initial average activity) */

/* rate control: Xi (initial I frame global complexity measure) */

/* rate control: Xp (initial P frame global complexity measure) */

/* rate control: Xb (initial B frame global complexity measure) */

/* rate control: d0i (initial I frame virtual buffer fullness) */

/* rate control: d0p (initial P frame virtual buffer fullness) */

/* rate control: d0b (initial B frame virtual buffer fullness) */

These parameters modify the behavior of the rate control scheme. Usually

set them to 0, in which case default values are computed by the encoder.

/* P:  forw_hor_f_code forw_vert_f_code search_width/height */

/* B1: forw_hor_f_code forw_vert_f_code search_width/height */

/* B1: back_hor_f_code back_vert_f_code search_width/height */

/* B2: forw_hor_f_code forw_vert_f_code search_width/height */

/* B2: back_hor_f_code back_vert_f_code search_width/height */

This set of parameters specifies the maximum length of the motion

vectors. If this length is set smaller than the actual movement

of objects in the picture, motion compensation becomes ineffective

and picture quality drops. If it is set too large, an excessive

number of bits is allocated for motion vector transmission, indirectly

reducing picture quality, too.

All f_code values have to be in the range 1 to 9 (1 to 7 for MPEG-1),

which translate into maximum motion vector lengths as follows:

code  range (inclusive)  max search width/height

================================================

1        -8 ...    +7.5       7

2       -16 ...   +15.5      15

3       -32 ...   +31.5      31

4       -64 ...   +63.5      63

5      -128 ...  +127.5     127

6      -256 ...  +255.5     255

7      -512 ...  +511.5     511

8     -1024 ... +1023.5    1023

9     -2048 ... +2047.5    2047

f_code is specified individually for each picture type (P,Bn), direction

(forward prediction, backward prediction) and component (horizontal,

vertical). Bn is the n'th B frame surrounded by I or P frames

(e.g.: I B1 B2 B3 P B1 B2 B3 P ...).

For MPEG-1 sequences, horizontal and vertical f_code have to be

identical and the range is restricted to 1...7.

P frame values have to be specified if N (N = # of frames in GOP) is

greater than 1 (otherwise the sequences contains only I frames).

M - 1 (M = distance between I/P frames) sets (two lines each) of values

have to specified for B frames. The first line of each set defines

values for forward prediction (i.e. from a past frame), the second

line those for backward prediction (from a future frame).

search_width and search_height set the (half) width of the window used

for motion estimation. The encoder currently employs exhaustive

integer vector block matching. Execution time for this algorithm depends

on the product of search_width and search_height and, too a large extent,

determines the speed of the encoder. Therefore these values have to be

chosen carefully.

Here is an example of how to set these values, assuming a maximum

motion of 10 pels per frame in horizontal and 5 pels per frame in

vertical direction and M=3 (I B1 B2 P):

search width / height:

forward  hor.  vert.     backward  hor.  vert.

I -> B1  10     5        B1 <- P   20    10

I -> B2  20    10        B2 <- P   10     5

I -> P   30    15

f_code values are then selected as the smallest ones resulting in a range

larger than the search widths / heights:

3 2  30 15 /* P:  forw_hor_f_code forw_vert_f_code search_width/height */

2 1  10  5 /* B1: forw_hor_f_code forw_vert_f_code search_width/height */

3 2  20 10 /* B1: back_hor_f_code back_vert_f_code search_width/height */

3 2  20 10 /* B2: forw_hor_f_code forw_vert_f_code search_width/height */

2 1  10  5 /* B2: back_hor_f_code back_vert_f_code search_width/height */

4. Interpreting the status information

======================================

4.1 Description of the macroblock_type map

The status information routine prints a two-character code for

each macroblock in the picture currently being coded:

First character:

S: skipped

I: intra coded

F: forward frame predicted

f: forward field predicted

B: backward frame predicted

b: backward field predicted

D: frame interpolated

d: field interpolated

0: forward predicted without motion compensation

Second character:

space: coded, no quantization change

Q:     coded, quantization change

N:     not coded (only predicted)

4.2 Description of the mquant map

The mquant map displays the dynamically changing quantization

parameter as determined by the rate control algorithm. Larger

values correspond to coarser quantization. Not displayed values

indicate same quantization as in the previous macroblock.

Currently all values are even, odd values occur only in conjunction

with the (not yet implemented) non-linear quantization scale table.

5. Encoder model

================

The encoder model describes the "algorithm" or "intelligence" of the

encoder.

5.1 Motion estimation

Exhaustive search pattern with L1 (Minimum Absolute Difference) matching

criteria on the original (non-coded) reference frame.  Performed only

on luminance samples.  Half pel search is done on a neighborhood of 8

bi-linearly interpolated luminance samples from the reconstructed

reference frame.

5.2 Rate control and adaptive quantization

The encoder adheres to a single-pass coding philosophy (no a priori

measurements guide the allocation of bits at the global layers).

Target bit allocation for the current picture being encoded is based

on global bit budget for the Group of Pictures (dependently coded

sequence of pictures), and a ratio of weighted relative coding

complexities of the three picture types.  Coding complexity (X) is

estimated as a picture's product of: average macroblock quantization

step size (Q) and the number of bits (S) generated by coding the picture

(e.g.   X = Q*S).

Local bit allocation is based on two measurements:

1. deviancy from estimated buffer fullness for the Nth macroblock

2. normalized spatial activity

The picture is approximated and estimated to have a uniform distribution

of bits. If the local trend of generated bits begin to stray from this

estimation, a compensation factor emerges to modulate the macroblock

quantization scale (mquant). The compensation factor is the difference

between where the buffer fullness was predicted at the Nth macroblock

and where the buffer fullness truly is)

Local spatial activity is estimated as the minimum variance of the

four luminance 8x8 blocks of the original picture being coded.  The

variance measure is then normalized against the average variance of the

most recently coded picture of the same type.  The scaled product of the

local activity measure and the buffer compensation factor are combined

to modify the final mquant value which will be used in the quantization

stage of the subsequent block coding processes.

5.3 Decision

The macroblock decision process selects the various prediction

switches (yes/no coding of prediction error, yes/no motion

compensation) which provide the best quality for the relative coding

bit cost.  The decision process follows the piecewise path of least

distortion, approximating that the decision stages (whether to use

forwards, backwards compensation or none at all..., etc.) are mostly

uncorrelated (i.e. can be independently decided).

5.4 Macroblock coding

The forward DCT is based on the fast Chen-Wang 1984 algorithm.

Quantization is a mirror of the normative inverse quantizer, as

specified in the MPEG-2 Test Model.  No special dithering or

entropy-constrained methods are currently employed.

6. References

=============

[1] ISO/IEC 13818 Draft International Standard: Generic Coding of

Moving Pictures and Associated Audio, Part 2: video

[2] Test Model 5, ISO/IEC JTC1/SC29/WG11/ N0400, MPEG93/457, April 1993.