The Satellite Tracking Program SatTrack (V3.0)

====================================================

Manfred Bester

1636 Martin Luther King Jr. Way #L

Berkeley, CA 94709, USA

January 11, 1995

GENERAL OVERVIEW

SatTrack is a satellite tracking program that has been written in the

'C' programming language on a Sun/UNIX workstation in order to provide an

alternative to PC oriented programs like QuikTrak, InstantTrack, STSplus

or OrbiTrack and others. It provides two different live displays, for

single or multiple satellites, and two different orbit prediction

facilities. Cursor controls used in both live displays are compatible with

the VT100 standard, which allows the program to be run basically from any

terminal. An X Window color graphics display with the typical world map

tracking chart and a number of important features has been added in this

version. In addition, a stand-alone demo program that shows the same

graphics features and simulates an STS-Mir docking maneuver is included in

this distribution. The orbit prediction can be run either interactively or

in a batch mode. In the latter case all necessary parameters are specified

on the command line.

The program reads the NORAD/NASA two-line Keplerian element (TLE) sets

directly and uses these to run the Simplified General Perturbations

Version 4 (SGP4) orbit propagation algorithm for low-earth orbit

satellites, or alternatively a simplified 'TLE Mean' model for satellites

with an orbital period longer than 225 minutes. The latter model will be

replaced by the Simplified Deep Space Perturbations Version 4 (SDP4)

algorithm in another release int the near future. Other input files contain

information on various default start-up parameters and satellite

information like launch dates, attitudes, uplink and downlink frequencies,

and transmission modes. Another file contains the data base for the ground

stations and the nearest city display.

The present version of SatTrack can track a single satellite and/or display

multiple satellites for a single ground station. It also can also control

suitable ground station hardware, like antennas and radio equipment, and

has an autotrack mode in which it switches automatically between a number

of specified satellites. For calibration purposes tracking of the Sun and

the Moon are also provided. Future releases will have more options. The

present version of SatTrack requires about 8.0 MB of disk space and less

than 1.5 MB of memory at run time. For the single satellite mode, a 68030

type of machine with a math co-processor is adequate. For running the

program in the multi-satellite mode and for the graphics display, a machine

of SPARCstation quality is recommended. A program from NIST to set the

system clock automatically is included in this distribution of SatTrack for

user convenience.

SatTrack is not to be confused with two other programs with the same name.

These were written by different authors on different computers. One, the

Amiga based program, was written by Randy Stackhouse, N4RTL, and the other,

the MacIntosh based program, by Mike Pflueger, WD8KPZ.

INSTALLATION

WARNING: If you want to save some of your special resource files of a

previous SatTrack version, these will have to be saved somewhere else

temporarily, because the procedure described below will overwrite these.

However, the format of some of the resource files needed to be changed to

allow for extra features. The format of the new files may not be compatible

any more with those from previous releases.

To install SatTrack on a UNIX workstation, place the distribution file

'sattrack.V3.0.tar.Z' into your home directory. Uncompress the file by

typing 'uncompress sattrack.V3.0.tar.Z'. If the distribution file name

ends with 'gz', use 'gunzip' instead of 'uncompress'. Unbundle the

distribution file by typing 'tar xvpf sattrack.V3.0.tar'. This will create

the following directories in your UNIX home directory:

SatTrack/data

SatTrack/doc

SatTrack/obj

SatTrack/pred

SatTrack/run

SatTrack/src/*

SatTrack/tle

All source code will be placed in 'SatTrack/src/*' and all auxiliary data

files (see below) in 'SatTrack/data' and in 'SatTrack/tle'. To recompile

SatTrack change into the 'src' directory and type first 'make clean' to

delete all object code and the executables (these files may have been

deleted already for the distribution of SatTrack). Edit the Makefile to

make sure the compiler and linker flags are properly set for the type of

machine being used. Also check other flags that may need to be configured

by the user. The default configuration in the Makefile is for a Sun-4

(SPARCstation 10) with SunOS 4.1.3. An attempt has been made to replace

system calls that are only known to BSD systems, in order to make the

software more portable. The compilation of this software package should

work perfectly on at least the machines listed in the table below. These

are the types of machines the program was tested on.

-----------------------------------------------------------------------

Machine                      Operating system

-----------------------------------------------------------------------

Sun-3/80                     SunOS 4.0.3 / X11R4 / Mwm

Sun SPARCstation IPC         SunOS 4.1.1 / X11R4 / Mwm

Sun SPARCstation 10          SunOS 4.1.3 / Solaris 1.1 / X11R5 / Mwm

Sun SPARCstation 5           SunOS 5.4   / Solaris 2.4 / X11R5 / Mwm

IBM RS6000                   AIX

HP Apollo 9000 Series 700    HPUX

i386                         NetBSD 0.9

i386                         FreeBSD

i386 (Toshiba 4600C)         Linux-1.1.50, Slackware 2.0

486DX33                      Linux, Slackware 2.0

486DX2/66                    Linux-1.1.75, gcc-2.5.8, fvwm-1.23,

XFree86-3.1 (X11R6)

-----------------------------------------------------------------------

Apparently SunOS 4.x.x and SunOS 5.x systems require 'stdlib.h' to be NOT

included. For this reason 'STDLIB' is defined in the Makefile as a

compiler flag for both Sun-3's and Sun-4's. Other machines running NetBSD

or FreeBSD need other include files. For more details look into the

Makefile.

Certain machines use C compilers different from 'cc'. If, for example,

the default C compiler is 'gcc' (the Gnu C compiler), the user needs to

make sure that the Makefile is configured properly. There are five lines

for each of the machine/operating system configurations listed in the above

table, and one more that specifies the location of the X11 include files.

To select one of these, activate the five lines for that particular

configuration and make sure the others are commented out. These lines

can be readily identified as

CPU      = ...

CC_CMACH = ...

CC_LMACH = ...

CC_LBSD  = ...

CC       = ...

and further below in the Makefile

X11      = ...

The default configuration is for a Sun SPARCstation 10 with SunOS 4.1.3

(Solaris 1.1) and X11R5.

Besides choosing the proper machine configuration in the Makefile, some

other parameters in 'sattrack.h' and a variety of options in the Makefile

can be configured by the user. If SatTrack is installed in the user's home

directory, the flag

-DHOMEDIR

has to be included. In this case the environment variable 'HOME' will

be used to create all path names. If SatTrack is to be installed in some

other directory, this flag has to be deleted, and instead the variable

'SATDIR' (in 'sattrack.h') has to reflect where SatTrack is installed.

Furthermore, a corresponding change needs to be made to the C shell script

'gettle' in 'SatTrack/run', since it needs to know where to put the files

with the TLE data.

If calculations of transits of satellites in front of the Sun are not

needed, the flag

-DSUNTRANSITS

has to be commented out. This will suppress certain menu options. This

will not affect the batch mode, however.

If the user wishes to suppress the audible alerts (beep, bell), the flag

-DNOBEEP

has to be included in the compiler options.

If your VT100 type terminal displays reverse video in an unacceptable way,

the flag

-DREVERSEVIDEO

should not be used. This will suppress reverse video, reverse blink video

and underline characters.

If the user is interested in visible observations of satellites only, the

flag

-DVISIBPASSES

has to be included in the compiler options. SatTrack will then list only

those passes of satellites that are visible, i.e. the satellite is in the

sunlight and the observer in darkness, in the short prediction form. In the

long prediction format, the satellite's altitude is shown instead of the

columns that are important for radio communications (Doppler shift, path

loss, squint angle, mean anomaly and mode) for all passes, i.e. not only

visible passes.

SatTrack V3.0 now also has an X Window color graphics display with the

typical world map tracking chart. But in order to allow the program to be

used on machines without X Window (X11R5) capabilities, the flag

-DXWINDOW

is used to switch the X Window option on or off at compile time. If the

flag is included, make sure that the variable 'X11' in the Makefile is

set properly, so that the compiler finds the X11 header files.

The program logic and the precision of the tracking calculations was tried

out on the ISI telescope system (see below in the Tracking Control

section). There are certain interface functions that need to be linked for

this purpose. However, normally these would not be required. To allow the

source code to be compiled and linked without these functions, the compiler

flag

-DISI

has to be commented out. Corresponding changes were made to the Makefile

already in order to allow straight forward compilation.

In order to finally compile all code, type 'make all'. Once the compilation

is accomplished the 'run' directory will contain the executable files and

the 'obj' directory all object code. Edit your '.login' and/or '.cshrc'

files to include '$HOME/SatTrack/run' in your path (or wherever else

SatTrack is installed). The 'tle' directory contains the files with the

NORAD/NASA TLE sets and the 'data' directory the data files

'defaultsn.dat', 'modes.dat', 'cities.dat', and 'satlist_nnn.dat'.

Predictions can be written into an output file with the name being that of

the satellite. This file will be placed in the 'pred' directory.

Previous releases of SatTrack have also been compiled by various users on

different computers. An e-mail collection of patches for machines other

than Sun workstations is provided in the 'doc' directory which contains all

documentation on SatTrack.

INPUT DATA FILES

SatTrack requires five input data files, all in plain ASCII format:

tlex.dat         contains NORAD and/or NASA TLE sets

defaults0.dat    contains default start-up parameters

(there can be several files of this type besides this

file, like defaults1.dat, etc.)

cities.dat       contains data on about 825 cities and other special

locations around the globe

modes.dat        contains additional information on satellites (alias

names, launch dates, attitudes, uplink and downlink

frequencies, transmission modes, etc.)

satlist_nnn.dat  contains a list of satellite names for the multisat

live display ('nnn' denotes the satellite group);

there can be many customized files of this type like:

satlist_all.dat:  all satellites (automatically generated)

satlist_am.dat:   amateur satellites

satlist_com.dat:  communications satellites

satlist_glo.dat:  Glonass navigation satellites

satlist_gor.dat:  Gorizont satellites

satlist_gps.dat:  GPS navigation satellites

satlist_mir.dat:  Mir complex and Mir debris

satlist_noaa.dat: NOAA weather satellites

satlist_sci.dat:  scientific research satellites

satlist_sts.dat:  space shuttles

satlist_tv.dat:   television satellites

satlist_wx.dat:   weather satellites

Example of a few entries in 'tlex.dat':

Mir

1 16609U 86 17  A 92253.83202629  .00010821  00000-0  17200-3 0  5392

2 16609  51.6235 184.3926 0003511 132.9676 227.1601 15.53877561375443

COBE

1 20322U 89 89  A 92253.56214384 -.00000059  00000-0 -26243-4 0  5574

2 20322  99.0083 266.1778 0008361 270.9185  89.1022 14.03352237143838

PACSAT

1 20439U 90  5  D 92246.03439168  .00000070  00000-0  35288-4 0  4890

2 20439  98.6417 327.8611 0011066 315.7837  44.2460 14.29731036136265

DO-17

1 20440U 90  5  E 92251.71853570  .00000070  00000-0  34955-4 0  4902

2 20440  98.6415 333.6363 0010985 298.5546  61.4530 14.29859354137082

TDRS 4

1 21639U 91 54  B 92249.52615843  .00000111  00000-0  00000+0 0  2232

2 21639   0.0810 282.2753 0002729 241.6567 196.0869  1.00275770  2147

EUVE

1 21987U 92 31  A 92232.33773712  .00002181  00000-0  10297-3 0    77

2 21987  28.4322 199.3801 0010738 262.4472  97.5038 15.15165949 11029

STS-47

1 22120U 92 61  A 92259.61569380 0.00076435  00000-0  25599-3 0   124

2 22120  56.9971  93.6720 0008801 289.1809  70.8314 15.89530661   497

The meanings of the numbers in each line are explained in the source code

listing (see 'satelem.c'). TLE sets like the above can for example be

obtained from 'archive.afit.af.mil' (129.92.1.66) via anonymous 'ftp'.

Change into the directory '/pub/space' and get 'tle.new'. Then place this

file as 'tle.dat' in the directory 'SatTrack/tle' and then run 'maketlex'.

This procedure can be automated using 'cron' in the following way: use the

command 'crontab -e' to create a crontab file with the following content:

0 15 * * *    $HOME/SatTrack/run/gettle

This will run 'gettle' every day at 15:00 h local time (in this example).

The executable C shell script 'gettle' needs to be in the directory

'SatTrack/run' and contains the following lines:

#! /bin/csh

#

set ftphost=129.92.1.66

set userid=$USER@`hostname`.`domainname`

cd $HOME/SatTrack/tle

#

ftp -n $ftphost << !

user anonymous -$userid

cd /pub/space

get tle.new

get gorizont.tle

get other.tle

get tvro.tle

get visual.tle

quit

!

#

"cp" tle.dat tle.bak

"mv" tle.new tle.dat

"mv" gorizont.tle gorizont.dat

"mv" other.tle debris.dat

"mv" tvro.tle tvro.dat

"mv" visual.tle visual.dat

#

$HOME/SatTrack/run/maketlex

$HOME/SatTrack/run/maketles wx

#

After placing the data files into 'SatTrack/tle', program 'maketlex'

is invoked. This program generates an extra file 'tlex.dat' with extended

TLE sets that is basically a concatenation of several individual TLE

files. Duplicate element sets will be filtered out on the basis of the

object number, so that there is ONLY ONE ENTRY PER OBJECT, and this will

be the first element set that is found for that particular object in the

list of TLE files. The file 'tlex.dat' is then used as the default data

base for SatTrack. The names of the data files being concatenated are

specified in the input file 'tlelist.dat' in 'SatTrack/data'. The user

needs to make sure that the order of the input files for 'maketlex' as

specified in 'tlelist.dat' is chosen correspondingly, if a TLE set for

a particular satellite occurs in more than one file.

The last line in the above shell script invokes program 'maketles' with

the command line argument 'wx' (in this example) that specifies the

weather satellite group. This will produce the file 'tles.dat' which

contains the TLE sets for those satellites specified in the data file

'satlist_wx.dat'. The file 'tles.dat' is not used with SatTrack, but is

useful for distribution to other users in the community who are interested

only in a particular subset of all TLE sets, in this example those of

weather satellites. The C shell script 'gettle' can also be invoked with

the '-v' option in which case the entire 'ftp' session is monitored.

The TLE sets can also be obtained from the Celestial Bulletin Board System

via telephone by dialing (205) 409-9280 (300 - 9600 bps, 8+1, no parity).

Latest element sets for space shuttle missions can also be obtained from

'spacelink.msfc.nasa.gov' (192.149.89.61) via 'telnet' and 'ftp', or from

an automatic e-mail service (see 'SatTrack/doc/info.tle_shuttle'). An

element set for STS-47, for example, would be placed in 'SatTrack/tle' as

'sts-47.dat'.

Example of 'defaults0.dat':

Site:              Berkeley, CA, USA

Call sign:         DL5KR

Satellite:         16609

Satellite group:   am

Element set TLE:   tlex.dat

Element set STS:   sts.dat

Element set type:  NASA

Live display type: M

Time zone:         PDT

Duration:          6.0

Min elevation:     0.0

Tracking:          0

Antenna:           /dev/ttyp7

Radio A:           /dev/ttyp8

Radio B:           /dev/ttyp9

Display lines:     24

X display lines:   34

X terminal type:   xterm

Pressure:          725.0

Temperature:       15.0

Humidity:          35.0

Amongst these parameters are the defaults that show up at the menu in <>

brackets when SatTrack is started. The entry 'Call sign' specifies the

call sign or other type of ground station identifier (up to 20 characters

long). The satellite can be specified either with its name or with the

object number. However, if the name is ambiguous (as in 'Mir' and

'MIR DEB') the object number needs to be specified. If a satellite group

'am' is specified as the default group for the multisat live display, then

there needs to be a corresponding file called 'satlist_am.dat' with the

names of the satellites in it.

The next three lines deal with the two-line element sets. At program start

SatTrack reads the entire default data file, here specified as 'tlex.dat',

and stores it in memory. If another data file is specified at the menu line

the program will add all satellites from that file to the memory list, as

long as the maximum number of satellites (presently 600) as specified in

'sattrack.h' is not exceeded. Data for satellites that are in the list

already will not be read again. For space shuttle data sets it is important

to specify the type of the element set, mainly because of the convention

for the orbit number. The two possible options are 'NASA' and 'NORAD'.

The line 'Live display type' specifies if the default live display is the

single satellite display ('S') or the multi satellite display ('M').

Options for the time zone are UTC, CEST, CEDT, EEST, EEDT, EST, EDT,

CST, CDT, MST, MDT, PST, PDT, HST, and JST. If other time zones are needed

these will have to be added to 'timeZoneList' and 'timeZoneHour' in

'satstrings.h'. In case a time zone is entered at program start that is

not in this list the time zone is automatically set to UTC. Alternatively,

a time zone can be entered as the numerical value. In this case the local

time is displayed as 'LOC'. SatTrack does not automatically check for

daylight savings time when reading the system time, since it always reads

the UTC system time, and the change over to and from daylight savings time

depends upon the location of the ground station. Therefore the default

start-up parameter for the time zone needs be edited by hand accordingly.

Remember also to edit the shell scripts that run SatTrack in the batch mode

('passes', 'transits', and the data file 'batch.dat') when the time changes

to or from daylight savings time. All calculations are performed in UTC, of

course. All other time zones are used for convenience only in the live

displays and in the prediction tables.

The entries 'Duration' and 'Min elevation' are used for the orbit

prediction mode. The duration is the time interval in days over which the

predictions are performed, and the minimum elevation in degrees is the

cut-off limit below which a pass is discarded.

The entries 'Tracking', 'Antenna', 'Radio A' and 'Radio B' are used for

controlling the antenna and radio interfaces. The default value for the

tracking flag can be set to '0' or '1'. If set to '1', the tracking will

be turned on automatically at program start, and otherwise it will be off.

The three lines 'Antenna', 'Radio A' and 'Radio B' contain the information

as to which interfaces on the computer the antenna controller and the

radio(s) are connected. If one single radio is used for both uplink and

downlink, it needs to be connected to the interface specified in 'Radio A'.

If two different radios are used for uplink and downlink, the downlink

radio needs to be connected to 'Radio A' and the uplink to 'Radio B'. Also,

in the latter case the variable RADIOTYPE needs to be set to DUAL in

'sattrack.h'. The user needs to modify the character strings that are sent

out to all three interfaces, according to what type of equipment is being

used. These changes can be made straight forwardly in 'satctrl.c'.

If both the antenna and the radio(s) are controlled through a single

device like the TrakBox, the ANTENNATYPE would have to specified as TRAKBOX

(in this example) and the radio type as NONE (in 'sattrack.h'). The antenna

interface would be the one the Trakbox is connected to, and the radio

interface would be set to /dev/null (in 'defaults0.dat').

If the data are to be written into files instead of being sent out through

an interface, the file names should be specified with the entire path name.

In the example given above the tracking control sequences will be sent to

three different 'xterm' windows. The default devices are '/dev/null', if

nothing is specified in 'defaultsn.dat'.

The following three lines deal with the type of terminal or window used for

the two different live displays. If the environmental variable 'TERM'

matches the entry in the line 'X terminal type' it is assumed that the

terminal or window can display 132 characters per line and the number of

lines specified in 'X display lines', which has to be at least 24 (if that

number is smaller than 24, the single satellite display will be screwed

up). Otherwise, the width of the terminal or window is assumed to be 80

characters and the number of lines as specified in 'Display lines'. If the

user wishes to always use an 80x24 terminal window, even in the X Window

environment, the variable 'X terminal type' has to be set to something

different from 'xterm', like 'yterm'. Weather or not the graphics window

can be used is independent of these parameters, as long as the user is

logged in on an X terminal.

The entries in the three lines 'Pressure' [mmHg] (760 mmHg = 1013.2 mBar),

'Temperature' [C] and 'Humidity' [%] are used to calculate the refraction

in the atmosphere for tracking satellites visually with an optical

telescope.

Example of 'cities.dat':

`North Pole`,90,0,0

`South Pole`,-90,0,2835

`Abeche, Chad`,14.46,-20.68,0

`Aberdeen, SD, USA`,45.467,98.483,0

`Abilene, TX, USA`,32.465,99.713,0

`Abu Dhabi, United Arab Emirates`,24.47,-54.37,0

`Acapulco, Mexico`,16.85,99.92,0

`Accra, Ghana`,5.6,0.167,0

`Addis Ababa, Ethiopia`,9,-38.733,0

This example shows the first few entries in 'cities.dat'. The first two

lines, i.e. the North and South Pole, have to remain unchanged. The name

of the location has to be included in `` and can be up to 37 characters

long. The following numbers are latitude [deg North], longitude [deg West],

and altitude above sea level [m], all separated by commas. Note that some

other tracking programs count the longitude positive in eastern direction.

Here the convention of positive signs for western longitudes is used. On

the other hand, SatTrack counts the time zone negative in western

direction. More locations can be added to 'cities.dat', if desired. Note

that the maximum number of entries allowed is 825 at present. If more

entries are required the variable MAXCITIES in 'sattrack.h' needs to be

changed accordingly and the source code needs to be recompiled.

Data on cities in the United States can be obtained from a geographic data

base via 'telnet martini.eecs.umich.edu 3000' (telnet 141.212.99.9 3000).

Example of 'modes.dat':

Satellite: AO-13

Object:    19216

Alias:     Phase 3C

Attitude:  180.0   0.0 deg

Frequency: USB LSB   145.900 435.498 150 -1

Frequency: CW  CW    145.900 435.498 150 -1

Frequency: USB NONE  145.812       0   0  0

Frequency: USB USB  2400.730 435.621  36  1

Frequency: CW  CW   2400.730 435.621  36  1

Frequency: USB NONE 2400.664       0   0  0

Mode:      B     0 - 100

Mode:      BS  100 - 130

Mode:      Sb  130 - 132

Mode:      St  132 - 155

Mode:      Sb  155 - 160

Mode:      BS  160 - 180

Mode:      B   180 - 256

Satellite: PACSAT

Object:    20439

Alias:     AO-16

Frequency: USB FM 437.0513 145.900 0 0

Frequency: USB FM 437.0513 145.920 0 0

Frequency: USB FM 437.0513 145.940 0 0

Frequency: USB FM 437.0513 145.960 0 0

Satellite: Mir

Object:    16609

Frequency: FM FM   145.550 145.550 0 0

Frequency: FM NONE 143.625       0 0 0

Mode:      FM 0 - 256

Satellite: STS-65

Object:    23173

Alias:     Columbia

Launch:    08Jul94  16:43:00 UTC

Frequency: FM FM 145.550 144.490 0 0

Frequency: FM FM 145.550 144.910 0 0

Frequency: FM FM 145.550 144.930 0 0

Frequency: FM FM 145.550 144.950 0 0

Frequency: FM FM 145.550 144.970 0 0

Frequency: FM FM 145.550 144.990 0 0

Mode:      FM 0 - 256

Satellite: Meteor 3-4

Object:    21232

Frequency: FM NONE 137.300 0 0 0

Mode:      FM 0 - 256

Note that if a satellite is listed in 'modes.dat' its name in the line

'Satellite:' has to exactly match the name in the NORAD TLE data file with

lower and upper case letters. For example, AO-10 has to be entered as

OSCAR 10 and AO-16 as PACSAT. An alias name with a maximum length of 15

characters can be specified in the 'Alias:' line. This name will be used

in addition to the NORAD satellite name for the live display and the

prediction tables. The specification of the object number is needed to

ensure the proper assignment of parameters, in case there are several

matches for the satellite name, like 'Mir' and 'MIR DEB'. On the other

hand, if for instance the satellite name is specified as 'GPS' and NO

object number is specified, then entries like the frequencies will be used

for all satellites whose names in this case begin with 'GPS'. In other

words, for a group of identical satellites, like 'GPS' or 'Glonass', only

one entry in 'modes.dat' is necessary.

If a launch date is specified the live display will show the mission

elapsed time (MET). The prediction tables will show the MET for the space

shuttle only.

The entries in the 'Frequency:' line are the transmission mode for the

downlink and uplink (in this order!) the downlink and uplink frequency in

MHz (in this order!), the transponder bandwidth in kHz, and the sign of

the transponder's frequency translation (-1 for inverting, and 1 for

non-inverting). If the satellite has no transponder, but a single

frequency repeater or packet robot or the like, the bandwidth and sign

have to be set to zero. If the satellite has a downlink channel or beacon

only, the uplink mode has to be set to 'NONE' and the uplink frequency has

to be set to zero. For details see the examples given above.

If the satellite to be tracked is not listed in 'modes.dat', or if no

downlink frequency is specified, the program will use a default value of

146.000 MHz and 'FM' as the downlink mode.

REMARK: The space shuttle frequencies specified in 'modes.dat' are for

users in the United States. Some of these frequencies are different for

other parts of the world and may have to be changed, depending upon the

location of the ground station.

Example of 'satlist_am.dat':

16609

OSCAR 10

AO-13

PACSAT

WO-18

LO-19

DO-17

FO-20

AO-21

UO-22

KO-23

KO-25

IO-26

AO-27

PO-28

RS-10/11

RS-12/13

RS-15

Note that the satellites' names need to match those in the TLE data file.

If there are ambiguous entries in the TLE file, like 'Mir' and 'MIR DEB',

the object number needs to be specified instead of the name, like '16609'

for Mir. In case of the space shuttle the correct specification of the

object number is rather important since there are pre-launch TLE sets with

a preliminary object number. For instance, in the case of STS-64, the

object numbers used were '64' or '99964' for the pre-launch element sets,

and '23251' for the post-launch element sets. It is VERY IMPORTANT that

the object number matches in all resource files. In case of STS-64 these

are the three files 'sts-64.dat', 'satlist_am.dat' (and for that matter

all other files of the type 'satlist_nnn.dat' that contain STS-64's object

number) and 'modes.dat'. After launch, when the new post-launch object

number is issued, all these resource files need to be edited

correspondingly.

PROGRAM START

To start the program type:

sattrack [-b] [-d] [-fn] [-g] [-m] [-q] [-t] [-v]

Remark: It is possible to combine several options. However, it is NOT

POSSIBLE to combine them in one argument, like 'sattrack -vtq'. The

command line options are as follows:

-b   enables the BATCH mode for orbit predictions. All parameters have to

be entered on the command line ( for example see shell script

'passes' in the run directory). This option cannot be used in

combination with any other option.

-d   enables the DEBUG mode. In this case various intermediate results

and other debugging information will be displayed.

-fn  allows to specify an alternate defaults data FILE (defaultsn.dat)

(if unspecified the program will use 'defaults0.dat')

-g   starts up the graphics window immediately (if possible)

-m   performs all calculations with the simplified MEAN Keplerian

propagation model for both low-earth and deep space objects.

As long as the SDP4 model is not installed yet, the simplified model

will be automatically used for all deep space objects.

-q   starts up SatTrack QUICKLY into the either the single satellite or the

multisat live display, depending upon the setting of the default

parameter 'Live display type' in the resource file 'defaultsn.dat',

with all other parameters as specified in that file --- no questions

asked.

-t   allows specification of another TIME zone instead of the default time

zone as listed in 'defaultsn.dat'. This is useful to quickly run off a

prediction for a site anywhere in the world without having to change

the default data file.

-v   enables the VERBOSE mode. All default parameters, ground station data

and detailed orbital elements will be displayed at program start.

For all keyboard entries the default parameters are shown in <>. Hitting

<CR> each time will use these. Otherwise, both lower and upper case

letters will be accepted. When entering the site and the satellite names a

fraction of the name will be sufficient, as long as they are unambiguous.

Otherwise all ambiguities will be displayed. For the ground station the

Maidenhead locator will be shown. Also, instead of a ground station name,

a Maidenhead locator can be entered.

It should be noted here that SatTrack reads the entire file 'tlex.dat' with

all available two-line element sets at program start, even if another file

is specified at the 'Two line elements' entry line. If a file other than

'tlex.dat' is specified at this entry, data ONLY FOR THOSE SATELLITES THAT

HAVE NOT BEEN READ YET from 'tlex.dat' will be added to the in-memory data

base. This is an important point! If for example the user prepares a

TLE data file, say 'sts-65.dat', from a fresh set of NASA TLE sets for

STS-65, and that file name is then entered, one needs to make sure that

there are no older data for STS-65 in the data file 'tle.dat' which is

the one FTP'ed from the US Space Command (formerly NORAD). Instead of

the new TLE set SatTrack would already have read a TLE set for STS-65

in this case and would therefore ignore the one from the input file

'sts-65.dat'. To prevent this circumstance from happening, the best to

do is to make sure that 'sts-65.dat' is included in the file 'tlelist.dat'

which is read by 'tlexgen' to create the concatenated TLE sets 'tlex.dat'.

Make sure that in this particular case 'sts-65.dat' is LISTED BEFORE

'tle.dat'. If you are not sure what element set is being used, invoke

SatTrack with the verbose option, and it will show all data. If SatTrack

is not invoked with the verbose option, it will still show how old the

TLE set is. In any case, if a new data file 'tlex.dat' was generated,

SatTrack has to be quit and restarted again in order to read this file

again.

The following chapters describe the two different live displays and the

two different orbit prediction features.

MAIN MENU

Upon program start the user is asked to enter the time zone (only if

SatTrack is invoked with the '-t' option), the ground station name, or

alternatively the Maidenhead locator of the ground station, the name or

object number of a preferred satellite, the name of the file with the

TLE sets, and in case the preferred satellite is one of the space shuttles,

the type of the TLE set (NASA or NORAD). Then the main menu is presented

with the following options:

Live Display     : D

Orbit Prediction : P

Restart          : R

Quit             : Q

State Vector Test: T  (not shown as a menu option)

The state vector test allows verification of the calculations with the

SGP4 and SDP4 models against the numerical examples given in the

Spacetrack Report (see source code of 'sattest.c' for exact reference).

The TLE sets needed for these tests are in 'SatTrack/tle/test.dat'. For

testing the SGP4 model specify the satellite name, i.e. the object number

as '40' and the TLE file name as 'test.dat'. For testing the SDP4 model,

the object number needs to be specified as '41' and the TLE file again is

'test.dat' (once the SDP4 model is installed). The results of this test

are described below in the PRECISION section.

LIVE DISPLAY FOR SINGLE AND MULTIPLE SATELLITES

In the live display mode the user can choose between a simultaneous

display for a number of satellites with a shorter list of tracking

parameters, i.e. one line per satellite, or a single satellite with the

full set of tracking parameters, i.e. the full screen. In either case

both the antenna and the radio equipment can be controlled. In the multiple

satellite display the cursor ">" can be moved to choose the satellite

being tracked. This satellite is then called the SELECTED one. The default

cursor position is on the line of the satellite that was chosen at the main

menu. If that satellite is not in the specified group list it will be added

to it. The keyboard commands for the multisat live display are also allowed

in the single satellite display, although those that change the cursor

position will not be effective. The allowed single key commands are:

t: toggles TRACKING "On" and "Off"

T: toggles TRACKING between "Auto" and "Off"

a: moves cursor AUTOMATICALLY to satellite with the next AOS time

(same with "A")

d: moves cursor DOWN one line

D: moves cursor DOWN 10 lines (if less than 10 lines, to bottom of list)

u: moves cursor UP one line

U: moves cursor UP 10 lines (if less than 10 lines, to top of list)

j: moves one page down (if satellite list is longer than one page)

J: moves to bottom page

k: moves one page up (if satellite list is longer than one page)

K: moves to top page

l: adds or deletes a satellite to/from autotrack LIST (marked "-" or ">")

L: adds all unmarked and deletes all marked satellites to/from the

autotrack LIST

n: moves cursor to a new satellite by NAME (or fraction thereof)

(same with "N")

o: moves cursor to a new satellite by OBJECT number (same with "O")

s: changes instantaneously from multisat to SINGLE satellite live display

(same with "S") (also refreshes single satellite display)

m: changes instantaneously from single satellite to MULTISAT live display

(same with "M") (also refreshes multisat live display)

r: REFRESHES either live display screen (same with "R")

g: turns GRAPHICS display on and off (same with "G")

e: adds an EXTRA (secondary) ground station in graphics window

(up to 9 total) (entries can be made while being in multi satellite

display only)

E: deletes all EXTRA (secondary) ground stations in graphics window

(only while being in multi satellite display)

x: cycles to next higher XPONDER (transponder/repeater/beacon) frequency,

as specified in 'modes.dat'

X: cycles to next lower XPONDER (transponder/repeater/beacon) frequency,

as specified in 'modes.dat'

c: CLEARS frequency offset that was entered manually, and resets frequency

step to 100 Hz (same with "C")

>: changes frequency step from 10 to 1 kHz or from 1 kHz to 100 Hz or from

100 Hz to 10 Hz

<: changes frequency step from 1 to 10 kHz or from 100 Hz to 1 kHz or from

10 Hz to 100 Hz

+: increases downlink frequency by the programmed frequency step

(only if transponder bandwidth is specified as non-zero) (same with "=")

-: decreases downlink frequency by the programmed frequency step

(only if transponder bandwidth is specified as non-zero) (same with "_")

0: (zero) enables tracking of the active satellite (only in single

satellite live display, turns off tracking of the Sun or the Moon)

1: enables tracking of the Sun (only in single satellite live display)

2: enables tracking of the Moon (only in single satellite live display)

h: HELP; displays all single character commands (same with "H" and "?")

q: QUIT SatTrack (back to UNIX prompt) (same with "Q")

CR, LF, ESC: interrupt either live display (go back to main menu)

Depending upon the type of terminal used, the multisat live display can

show the orbit prediction model used (the 'M' column), and the ground track

location. This is the case if the terminal is an X Window terminal

(assuming that the window in which the program is started has been set to

a width of 132 characters). Otherwise this part of the live display is

omitted. This feature depends upon the setting of the default parameter

'X display type' in the 'defaultsn.dat' file. If the entry chosen is for

example 'xterm' and the user logs in on an X Window terminal, the program

assumes that the window is set to a size of 132x64 characters. The default

length (i.e. number of lines) can also be set in 'defaultsn.dat'.

Satellites are shown in the order of the time of AOS if they are below the

horizon, or in reverse order of the time of LOS if they are above the

horizon. An asterisk next to the maximum elevation (MEL) of the next pass

indicates a 'good' pass, i.e. one with an elevation of more than 40 deg.

The symbol to the right of the instantaneous satellite elevation indicates,

if the satellite is currently ascending or descending. The letter in the

'M' column shows the type of propagation model used for that satellite. The

three different models used are the SGP4 model ('G'), the SDP4 model ('D'),

or the simplified TLE Mean model ('M'). The latter model is automatically

used for all satellites when SatTrack is invoked with the '-m' flag.

Otherwise SatTrack chooses between the SGP4 and SDP4 model, depending upon

the satellite's orbital period. However, in the present release the SDP4

model is not yet installed and SatTrack always uses the TLE Mean model for

deep space objects.

REMARK: If a satellite is above the horizon already when SatTrack is

started, the displayed AOS time is the present time. If the satellite is

beyond the maximum elevation already at that time, then the maximum

elevation shown is the one it has at that time. Since SatTrack does not

calculate fractions of a pass that happened in the past, this is what the

display means in these cases. This is also true for the prediction tables

and is considered a "feature" rather than a "bug".

When a satellite is above the horizon, the entire line is displayed in

reverse video. When a satellite is below the horizon, but above -3 degrees

in elevation, only the satellite's name is shown in reverse. That will

also be the case for a satellite that gets close to the horizon, but

doesn't quite make it in the current orbit. This feature is useful since

sometimes one can hear a satellite even when it is below the horizon, due

to refraction in the atmosphere or ionosphere. The display line will be

blinking when a satellite is within 1 minute of LOS or AOS.

The multisat live display can be very slow if the computer is not of a

SPARCstation quality and if the number of satellites chosen is large. In

this case the computing power will not be sufficient to ensure timely

control of the antenna or the radio. SatTrack has to find the circumstances

of the next pass for each satellite before the first display update occurs.

In the live display mode for single satellites the program cycles through

a full computation every 5 or 10 seconds and sleeps in between. This time

can be changed, of course, by setting the variables FASTUPDATEINT,

MEDUPDATEINT and SLOWUPDATEINT in 'sattrack.h'. Whenever the environment

variable 'TERM' corresponds to the default parameter 'X display type' in

'defaultsn.dat', the fast update is chosen. This allows the user to login

on a fast console and have the display updated with the fast update time,

whereas on a 9600 Bd type terminal the slow update time is used for the

multisat display and the medium one still for the single satellite display.

In a strict sense, the live displays are not real-time displays since the

program runs under UNIX (this will most likely change in the future). For

most applications, however, this is not a disadvantage. For near real-time

performance it is important that the UNIX system clock is set correctly,

though. Examples of the screen layouts for both live displays are shown

below.

The singlesat live display is rather self-explanatory. The top center of

the display shows the date and time, in the specified time zone.

In case of the space shuttle, the computation of the orbit number is

different from all other satellites. For the shuttle, the orbit

number increments each time it crosses the ascending node. The shuttle is

launched into orbit #1. If the space shuttle is tracked, the source of the

element set, i.e. the element set type has to be specified, since there is

a difference in the convention for definition of the reference orbit

between NASA and NORAD. For satellites other than the space shuttle the

NORAD convention is used and the orbit number increments when the satellite

passes through the perigee. Note that in case of the space shuttle, the

results of the live calculations are displayed only if the current time is

past the epoch time of the element set used. If the current time is between

the launch time and the epoch time of the element set, the display will

show 'Ascending to Orbit'.

The visibility display to the right of the percentage of the current orbit

indicates if the satellite is in the day ('D') or night ('N') part of its

orbit. If the satellite is above the horizon and in the day part of its

orbit and at the same time the Sun is below the nautical twilight limit

(-6 deg elevation), then the display shows 'V' (visible), since the

satellite will be visible "to the naked eye", at least for the larger

satellites. However, the apparent brightness depends on the relative

angles between the Sun, the satellite and the ground station, and on the

shape, size, attitude and distance (range) of the satellite.

The eclipse and visibility conditions are also indicated by 'D', 'N' or

'V' in the lower right corner of the display for the circumstances of the

next pass, i.e. acquisition of signal (AOS), maximum elevation (MEL) and

loss of signal (LOS) of the satellite.

Azimuth and elevation angles of the Sun and the Moon are shown for the

location of the ground station. If tracking of the Sun or the Moon is

enabled the azimuth and altitude displays reserved for the satellite then

also show the bearing of the Sun or the Moon, respectively. All other

entries in the single satellite live display still refer to the satellite

being tracked. The elevation angles for the Sun, the Moon, and the

satellite are corrected for refraction in the Earth's atmosphere using the

atmospheric parameters in 'defaultsn.dat'. This facilitates observations of

satellites with an optical telescope.

The Doppler shift is calculated for both uplink and downlink frequencies,

as specified in 'modes.dat'. The displayed frequencies are the ones

corrected for the Doppler shift. If a transponder bandwidth is specified

for a certain downlink and uplink frequency pair, the downlink frequency

can be stepped up or down in certain steps within the bandwidth interval,

using the "+" and "-" keys. The default step width is 100 Hz (as defined

by FREQSTEP in 'sattrack.h'), but it can be changed to 10 kHz, 1 kHz or

10 Hz with the "<" and ">" keys. The uplink frequency is altered

automatically. The frequency offset and step width are reset to their

default values with the "c" key. The "x" key switches to the next frequency

pair. These keys are active only in the single satellite live display. If

the satellite cursor is moved in the multisat display, the offset and step

width are reset as well, and the frequency selected is the first one listed

in 'modes.dat' for that particular satellite.

REMARK: The above described manual tuning is foreseen only for satellites

with transponders. However, if the user wishes to also be able to slightly

tune the frequencies for single-frequency downlink channels to make some

corrections to imperfect Doppler compensations, one can make some changes

to the entries in 'modes.dat'. As an example, PACSAT is shown below:

Satellite: PACSAT

Object:    20439

Alias:     AO-16

Frequency: USB FM 437.0513 145.900 2 0

Frequency: USB FM 437.0513 145.920 2 0

Frequency: USB FM 437.0513 145.940 2 0

Frequency: USB FM 437.0513 145.960 2 0

The last but one column has been modified for this purpose. The '2'

indicates a tuning bandwidth up to 2 kHz (i.e. +/- 1 kHz from the center

frequency). This will affect only the downlink frequency.

The free-space path loss can be used for link quality calculations. If the

attitude of the satellite is specified in 'modes.dat', the display shows

the squint or off-pointing angle, and otherwise the Sun angle, i.e. the

angle between the ground station, the satellite and the Sun. The latter

angle is particularly important for visual observations of satellites.

The satellite mode depends on the phase of the satellite (the mean anomaly

in units of 0-256), as specified in 'modes.dat'. If no satellite mode is

specified, the display shows blank.

The letters to the right of the elevation and latitude display indicate

if the satellite is on the ascending ('/') or descending ('\') part

of its pass or orbit, respectively.

In case of the space shuttle, distances are displayed in nautical miles

(nm), and otherwise they are in kilometers. Accordingly, velocities are

in nm/s or km/s.

The state vector of the satellite (position and velocity in Cartesian

coordinates) is displayed below the bearing and ground track information.

The circumstances of the next pass are calculated and displayed in the

lower part of the screen. If the next pass has a maximum elevation of more

than 40 deg the maximum elevation and range at maximum elevation are shown

in reverse video, since this would be considered a "good pass". The

countdown time represents the time until next AOS or next LOS. One minute

before AOS and during a satellite pass the countdown time is displayed in

reverse video.

Rise and set times are not calculated for geostationary satellites.

However, one needs to make some kind of distinction between geosynchronous

and geostationary satellites. SatTrack uses two parameters for this

purpose, a limit on the mean motion and a limit on the inclination. If the

mean motion is 1.0 +- 0.05 rev/day the satellite is considered

geosynchronous, and if in addition the inclination is less than 10 degrees,

it is also considered geostationary. Both parameters, GEOSYNCMOT and

GEOSTATINC, are defined in the include file 'sattrack.h'. They can be

changed before recompilation, if necessary. There are quite a few

satellites with inclinations in the range below 10 degrees. If for instance

the ground station is in North America, some of these will never set. On

the other hand, if the ground station is at the South Pole, they will both

rise and set. Specifying the inclination limit is therefore important for

the algorithm that searches for the circumstances of the next pass. The

time limits for rise and set time searches are specified in 'sattrack.h'

by the parameters MAXDAYS and MAXDAYSSYNC for non-geosynchronous and

geosynchronous satellites, respectively.

The mission elapsed time (MET) will be displayed if the launch date and

time are specified in 'modes.dat'.

The ground track location indicates the location of the sub-satellite

point with respect to the nearest location in the city data base

('cities.dat').

An alarm (beep) is generated for each screen update within one minute of

either AOS or LOS. When the satellite being tracked is above the horizon

several numbers on the screen show up in reverse video (the header line,

azimuth, elevation, range, Doppler shift and path loss).

Once the program is running in the live display mode, the screen can be

frozen and unfrozen with ^S and ^Q, respectively.

TRACKING CONTROL

The tracking feature can be used in both live displays. Tracking is

automatically turned off when the cursor is moved from one satellite to

another in the multisat display, but stays active when switching back and

forth between the two live displays. When the tracking is enabled the

satellite orbit calculations will be performed for a time ahead of the

present. The time offset and also the interval at which the calculations

are repeated are specified by the variables TRACKINT and TRACKTIMEAHED

in 'sattrack.h'. Default values are 1 and 10 seconds, respectively. If the

computer that performs both the calculations and the live display does

also the controlling of the ground station hardware, the parameter

TRACKTIMEAHEAD needs to be set to zero. Even though the live display may

only be updated every 5, 10 or 15 seconds, depending upon the setting of

the variables FASTUPDATEINT, MEDUPDATEINT and SLOWUPDATEINT in

'sattrack.h', the tracking calculations and ground station control

commands are still performed every TRACKINT seconds.

If the tracking calculations and control of ground station equipment are

performed on different machines, the calculations may have to be

accomplished several seconds ahead of time to allow for transfer of the

tracking information to the second computer and therefore timely antenna

and radio tracking. This is the case with the Infrared Spatial

Interferometer (ISI), a two-telescope infrared telescope system, on which

this software was tried out. This system has its own antenna/telescope

control computers with the VxWorks real-time operating system (Wind River

Systems) and receives the tracking information from a Sun workstation once

per second and 10 seconds ahead of time via remote procedure calls (RPC)

on the Ethernet. The particular interface code for these RPCs can be found

in 'sattele.c'. If 'ISI' is undefined in the Makefile, the RPC interface

functions will be inactive and the source code can be compiled on other

machines that do not have the interface code for the ISI system.

In the autotrack mode, the program will automatically switch between

those satellites in the multisat live display that have been marked to

be in the autotrack list ('>' or '-'). Once the currently tracked

satellite passes below the horizon, the program automatically switches to

the next satellite in the list, i.e. the one with the next set time, or,

if none of the marked satellites is above the horizon, the one with the

nearest rise time. Satellites can be marked or cancelled from the

autotrack list using the 'l' or 'L' commands (see above). The autotrack

mode works only while in the multisat live display. If the user switches

to the single satellite live display while autotracking is enabled, the

program will not switch satellites.

The devices to which the antenna and radio interfaces are connected is

specified in 'defaultsn.dat'. One can specify devices like '/dev/ttyp1'

and '/dev/ttyp2', for example, to direct the tracking commands to some

'xterm' windows. To control particular ground station equipment the program

code needs to be adapted by the user (see 'satctrl.c' and 'sattrack.h').

It should be noted that the tracking will not work right if the computer

is overloaded. If, for instance the multisat live display is used with

500 satellites (specify satellite group as 'all'), even a SPARCstation IPC

will not be able to perform the tracking calculations on time. The number

of satellites in the multisat live display should be kept small, i.e. of

the order 20, to insure that the tracking calculations can be performed on

time.

ORBIT PREDICTIONS

The orbit prediction mode allows the calculation of satellite passes for

a given ground station. The data will be either displayed on the video

terminal or written into a file in the 'pred' directory which can

later be obtained as a hardcopy. The program generates tables in a short

and in a long form. The short form listing consists of one table per day

with one line per pass, indicating the time at AOS, MEL and LOS, the

duration of the pass, the azimuth at AOS, MEL and LOS, the maximum

elevation of the pass, the visibility code for AOS, MEL and LOS, and

number of the orbit in which the satellite is at AOS. Also, for the space

shuttle, if a launch date is specified in 'modes.dat' an additional column

shows the MET at AOS. Passes with a maximum elevation of more than 40 deg

are marked with an asterisk next to the MEL value.

The prediction in the long format consists of 11 columns. The first column

is the time in specified fixed increments from line to line. Most of the

table is self-explanatory. The column labeled 'Sun Ang' is the phase angle

of the satellite, i.e. the angle between the ground station, the satellite

and the Sun. Optimum viewing occurs if this angle is smaller than a certain

value, for instance 120 degrees, and if the satellite is in the sunlight

and more than for instance 15 degrees above the horizon, and if the Sun is

below the twilight limit from the viewpoint of the ground station. If all

these conditions are true, an asterisk is printed at the end of the line.

Both the limits for the Sun phase angle, VISPHSLIMIT, and for the

elevation, VISELELIMIT, are specified in 'sattrack.h'. The column labeled

'Loss' is the free-space path loss in dB. The squint or off-pointing angle

('SquAng') is calculated if the satellite's attitude is specified in

'modes.dat'. The column labeled 'Phs' is the orbital phase of the

satellite, i.e. the mean anomaly running from 0 through 256. The 'Md'

column is the satellite mode, derived from the phase and the entries in

'modes.dat'. This column will be blank if no modes are specified in

'modes.dat'. The last column is the visibility code of the satellite

('D', 'N' or 'V', see above).

A special feature of the orbit prediction in the long format is the

computation of transits of a given satellite in front of the solar disk.

In this case the output format of the prediction table is different.

The symbol '###' at the end of a line in the prediction table indicates if

the satellite is in front of the solar disk, i.e. less than 15 arcmin from

the center of the Sun.

An example for prediction tables in the short and long forms, and for

solar transits are given in Figures 5.-7. below.

For users that are interested in observations of visible satellite passes

the code can be compiled with the flag '-DVISIBPASSES' (see above and

Makefile). In this case the format of the prediction tables is different.

The columns that important for radio communications are then dropped and

other columns are added. It should be noted that the perigee and apogee

height shown in the line 'Orbit Geometry' have been obtained by subtracting

the EQUATORIAL radius of the Earth. However, the satellite's height is

otherwise calculated as the height above the real Earth, using the geoid

model. Therefore, the user should not be surprised that the real height can

be smaller than the perigee height (by up to 21 km).

The orbit prediction in both short and long form can also be run in the

batch mode. All parameters have to be entered on the command line with

a fixed format. An example for shell scripts that run SatTrack in the

batch mode for a number of satellites are given in 'SatTrack/run' as

'passes' or 'transits', for the prediction of passes over a specified

ground station or transits in front of the Sun, respectively. If SatTrack

is invoked in the batch mode, all other command line options are turned off

automatically.

Another shell script, 'passesx', is equivalent to 'passes', except that

it is generated automatically by a program, 'makepassesx'. This program

reads a satlist file and makes a batch mode entry in 'passesx' for all

satellites in that list, so that in turn 'passesx' can be run to predict

passes for a large number of satellites over a given ground station. Except

for the satellite's names, all other command line parameters need to be

specified in 'batch.dat' in 'SatTrack/data'.

The default print command for obtaining a hardcopy of the prediction tables

is 'enscript'. If the user wishes to use a different print command, this

can be specified in the variable PRINTCMD in 'sattrack.h'. If, for example,

'lpr' is used, the option PRINTOPT may have to be set to something like

'-Plwa' and the point sizes and font may need to be set to something like

'-1' and 'Courier', respectively. Check the man pages for 'lpr' for proper

usage and invoke SatTrack with the '-v' option to see if the print command

is configured properly (the complete print command will be displayed when

it is invoked). Remember to recompile all code if 'sattrack.h' has been

altered.

PRECISION

The results from SatTrack, namely the latitude and longitude of the

sub-satellite point and the azimuth and elevation angles of a satellite

were compared with those calculated with QuikTrak, InstantTrack, STSplus

and MacSat. It was found that all programs agree with each other within

a few tens of a degree. The precision obtained depends amongst other

things on the particular model of the Earth used. SatTrack uses a

non-spherical geoid model. Also, the precision depends on the level of

sophistication used for the astronomical coordinate transformations, i.e.

the motion of the Earth within the solar system. Of course, the precision

also depends on the quality of the set of orbital elements for the orbit

simulation and the level at which the periodic variations of the SGP4 and

SDP4 orbital models are taken into account. The NORAD TLE sets are mean

values with periodic variations removed in a particular way. For high

precision tracking, these variations have to be reintroduced by the

tracking program in the same way they were removed, depending upon what

model was used. This current version of SatTrack does reintroduce these

periodic variations only for low-earth objects, i.e. those with an orbital

period of 225 minutes or less. This is accomplished by using the SGP4

propagation model.

Using the NORAD state vector test mentioned above, the precision of the

calculations of the SatTrack SGP4 state vectors were compared with those

tabulated in the Spacetrack Report No. 3 (F. R. Hoots and R. L. Roehrich,

December 1980). After a propagation time of 1440 minutes (one day) the

agreement of the X, Y and Z coordinates is 3 m in X, 1 m in Y, and 10 m

in Z direction, which is less than a third the size of the space shuttle.

The velocities agree after one day within 1 cm/s in all three axes. These

discrepancies can arise from either a smallish bug in the C language

implementation of the NORAD code, or from a somewhat different, i.e. more

precise choice of the numerical constants like PI in SatTrack. Remember

that the space shuttle typically travels about 75 m in 1/100 seconds! In

any case are these differences small compared with the precision of the

published Keplerian elements for a particular object, once these are a day

old.

By tracking HST, GRO and Mir visually with the ISI telescopes it was found

that the blind pointing accuracy for satellites in low-earth orbit is

usually better than 15 arcmin in azimuth and elevation angles. The TLE

sets used were only a few days old. It is very difficult to achieve an

accuracy better than this, however. It should be noted that the error

arising from the published TLE sets can be much larger. The TLE sets are

updated only if a satellite is more than 5 km away from its predicted

position, mainly in the direction of travel (see explanation by T. S. Kelso

in the file 'SatTrack/doc/info.tle.general'). For a LEO satellite like the

space shuttle, passing overhead at an altitude of 250 km, this error

amounts up to 1.15 deg. This assumes, however, that there are no other

tracking errors involved. If for instance the computer clock is off by one

second and the satellite travels at a speed of 7.5 km/s, another error of

1.7 degrees is introduced. Therefore it is important to set the computer

clock as accurately as possible. Of course, the calculation of the Doppler

correction also relies on the proper time and knowledge of the orbital

elements.

For deep space objects, i.e. those with an orbital period longer than

225 minutes, a simpler propagation model, called the 'TLE Mean Model' is

presently used (the SDP4 model is still under construction). However,

the precision of SatTrack should still be high enough to properly aim

antennas with a gain of 30 dBi or less (beam width of 6.5 deg or larger)

at a satellite. The agreement between the TLE Mean Model and the example

of the SDP4 model in the Spacetrack Report is 1.65 deg after one day.

The position of a number of GPS satellites as calculated from SatTrack

was compared with those given by a GPS receiver, and they all agreed

within 1 degree in azimuth and elevation.

The precision of the calculation of the positions of the Sun and the Moon

were tested on May 10, 1994 with the annular solar eclipse for a ground

station in El Paso, Texas, which was in the center of the shadow path. It

was found that the differential angle between the Sun and the Moon was

smaller than 1.5 arcmin, or 1/20 of the diameter of either object. This

suggests that the absolute precision is about half that value, or

45 arcsec. Furthermore, tracking of the Moon was also checked with an

astronomical telescope, namely one of the 1.65 m telescopes of the Infrared

Spatial Interferometer (ISI) at Mt. Wilson Observatory. It was found that

the precision again is of the order 45 arcsec.

AUXILIARY PROGRAMS

Four of the following eight programs are not used with SatTrack, but are

rather useful for other purposes. These are 'fixchecksum', 'makeamsat',

'maketles', and 'xdemo'. The other four programs are used with SatTrack

to create various input files and shell scripts, and to set the system

clock.

fixchecksum    generates two-line element sets with the old NORAD checksum

convention needed with InstantTrack V1.0. The input file

is for example 'tle.dat' and the corresponding output file

is 'tle.chk', also created in 'SatTrack/tle'.

makeamsat      generates the data file 'amsat.dat' in 'SatTrack/data' with

orbital elements in the AMSAT format from a data file with

orbital elements in the NORAD two-line format.

makepassesx [group]

generates the C shell script 'passesx' that can then be

used in turn to predict passes for all satellites listed in

the file 'satlist_group.dat', where 'group' is the specified

satellite group. Command line parameters for the SatTrack

batch mode are read from 'batch.dat'. If you want to keep

your machine busy for a while, invoke 'makepassesx all'

and in turn 'passesx'. This will run the orbit prediction

for something like 500 satellites, or however many entries

there are in 'tlex.dat'.

maketles [-v]  generates a data file 'tles.dat' with a selected number of

TLE sets from 'tle.dat'. The names of the satellites to be

selected are read from 'satlist_nnn.dat', where 'nnn' needs

to be one of the satellite groups for which a satlist file

exists (e.g. 'am', 'com', 'glo', 'gps', 'sci', 'wx', 'all'

or anything supplied by the user).

maketlex [-v]  concatenates several files with two-line Keplerian element

sets into one file, 'tlex.dat'. Duplicate entries are

filtered out, using the object numbers as a the designator

rather than the satellite names. The TLE input files and the

order in which they are processed are specified in

'SatTrack/data' in the file 'tlelist.dat'. This program also

produces two other output files, namely 'satlist_all.dat'

with the object numbers of all satellites in 'tlex.dat', and

'index.dat' which serves as a reference between object

numbers and satellite names. The latter file is not used by

any program, but is rather meant to be a user friendly

look-up table.

MAKETLEX HAS TO BE RUN WHENEVER ANY OF THE TLE FILES HAVE

BEEN UPDATED OR EDITED.

nistime [-s1]  calls the NIST time server and reports the time offset

between the NIST time and the UNIX system clock. The system

clock is set quickly, but gradually, to a precision of one

second if 'nistime' is invoked with the '-s1' option as

super user. This program can be run under 'cron' as

'nistime -s1 > /dev/null'. For detailed information read the

files 'info.nistime.usage' and 'info.nistime.server' in the

'doc' directory. This program has been developed by NIST.

satfilter [group]

creates a file of the type 'satlist_nnn.dat' with object

numbers of those satellites that have inclinations and

orbital periods within a certain range. Three default

arguments 'group' are possible: 'leoe' for LEO satellites

with an inclination below 35 degrees, 'leom' for satellites

with inclinations between 35 and 65 degrees, and 'leop' for

satellites with inclinations above 65 degrees. The

corresponding output files are 'satlist_leoe.dat',

'satlist_leom.dat' and 'satlist_leop.dat'. If no argument

is specified, the user will be asked to enter the range

of inclination and orbital period, as well as the name

of the satlist file extension ('nnn').

xdemo [-v]     is a demo program to show the features for the X11 style

graphics display in SatTrack V3.0 (see below).

The following table shows how these programs and shell scripts use certain

input files and what output files are created and how these will be used in

turn. Only the principal input and output files are shown here in order to

not confuse the issue. The symbol "->" means "is read by" or "writes", and

">>" means "invokes". The symbol "*" denotes the executables.

----------------------------------------------------------------------------

gettle* -> tle.dat

gettle* >>                maketlex*   -> tlex.dat          -> sattrack*

tlelist.dat -> maketlex*    -> tlex.dat         -> sattrack*

tlex.dat    -> maketles*    -> tles.dat

tles.dat    -> makeamsat*   -> amsat.dat

tlex.dat    -> satfilter*   -> satlist_nnnn.dat -> sattrack*

tlex.dat    -> fixchecksum* -> tlex.chk

batch.dat   -> makepassesx* -> passesx*         >> sattrack* -b

passes*          >> sattrack* -b

transits*        >> sattrack* -b

----------------------------------------------------------------------------

X WINDOW GRAPHICS

The present distribution of SatTrack contains an X11 demo program called

'xdemo' that shows the typical world map and a number of features like the

ground track, circle of visibility of the satellite and the ground station,

the terminator and some surprises. The spacecraft lighting conditions are

toggled in this demo version with every display refresh.

There are eight world maps supplied with this distribution, four in the

'line' format, and four in the 'fill' format. In the 'line' format, the

continents and other land areas are outLINED", whereas in the 'fill' format

they are FILLED. The map sizes are as follows:

world_lines_0.xbm        612 x 372 pixels      360 x 180 degrees

world_lines_1.xbm        612 x 432 pixels      360 x 180 degrees

world_lines_2.xbm        700 x 490 pixels      360 x 180 degrees

world_lines_3.xbm        860 x 430 pixels      360 x 180 degrees

world_fill_0.xbm         500 x 236 pixels      360 x 170 degrees

world_fill_1.xbm         600 x 284 pixels      360 x 170 degrees

world_fill_2.xbm         700 x 330 pixels      360 x 170 degrees

world_fill_3.xbm         860 x 406 pixels      360 x 170 degrees

All maps are in cylindrical projection. The color scheme for the 'line'

maps is different from the 'fill' maps. The user can choose a certain map

type and map size, depending upon personal taste, size of the screen, and

available computer memory and bandwidth, by copying one of the above eight

X11 bitmap files in the 'src/include' directory to 'world.xbm'. The code

needs then to be recompiled.

The demo program requires about 1 MB memory at run time and was used to

develop all the tools and functions needed for the implementation into

SatTrack V3.0. It would be good to know if for instance the sizes of the

graphics windows fit conveniently on the screen of a Linux laptop computer.

It also would be interesting to know if the required computing bandwidth

and speed are adequate on i486 machines, and if the chosen colors are

acceptable. Any additional ideas and thoughts would be appreciated. For the

compilation of 'xdemo' the user may have to change the variable 'X11' in

the Makefile that defines the directory for the X11 include files. It

should be noted that for this graphics application only basic X11 functions

and X11 Toolkit functions were used, but not the Motif widget set, in order

to make the program more portable.

The graphics window in SatTrack shows the same features as the demo

program, except that it is of course tied to the live calculations. The

graphics window is turned on and off with the 'g' option from the keyboard.

It is turned on automatically if SatTrack is invoked with the '-g' option.

The user has the same choice for a particular type and size of the world

map as in the demo program. The selected and marked satellites are shown in

light blue, whereas the unmarked ones are in yellow. The brightness of the

satellite markers reflects the present lighting conditions. The circle of

visibility (foot print) around a selected or marked satellite is shown in

white. The ground track is pre-calculated for three orbits for the selected

satellite only and is shown in light blue. The information in the header

and footer lines also correspond to the selected satellite.

The ground station and its local circle of visibility are shown in red.

This "circle" always refers to the selected satellite and its instantaneous

height above ground, and therefore changes in size for satellites on

elliptical orbits. If a ground station ID is provided in 'cities.dat'

(like 'DGS' for Diego Garcia) that ID is shown near the ground station

marker. The ground station ID must be included in round brackets and must

NOT be the last part of the ground station name. The latter condition is

important because the last part of the name is reserved for the nation it

belongs to, and is also enclosed in round brackets if the location is an

overseas territory. The ground station ID would typically be a radio call

sign, or a three-letter code for the nearest airport, or some other

abbreviation. Valid examples are:

`Cologne (DL5KR), Germany`,50.943,-6.948,50

`Diego Garcia (DGS) (UK)`,-7.33,-72.4,0

`Edwards (EAFB), CA, USA`,34.94,117.86,200

`Merritt Island (MIL), FL, USA`,28.608,80.604,15

If no ground station ID is specified, the display shows the first part of

the ground station location (up to the first comma). In addition to the

primary ground station up to nine more secondary ground stations can be

added in the graphics display (see above). They can be entered either by

their name or by their Maidenhead locator, in case their name is not in

the city data base. The secondary ground stations are also shown in red

with their local circle of visibility for the selected satellite and with

a smaller square. If a ground station ID is specified in 'cities.dat', it

will be shown next to the mark on the screen. Otherwise, the Maidenhead

locator will be shown. Yet another smaller red square marks the location

of the nearest city or ground station with respect to the sub-satellite

point. The Sun is marked as a yellow "*" and the terminator is shown as a

yellow line. Grid lines are spaced 30 degrees in latitude and longitude.

For NASA missions that involve communications through the TDRS satellites,

the graphics display shows the zone of exclusion (ZOE) within which there

is no communication through either the eastern or the western satellite.

In order to show these features, the satellite list for the multisat

display has to be set up in a particular way. The file 'satlist_nnnn.dat'

(for example 'satlist_nasa.dat') has to include the two operational TDRS

satellites, for instance 'TDRS 3' and 'TDRS 4', as well as the other

satellites that are to be shown in the display, like '23340' (STS-66), or

even 'HST' or 'GRO', and also '16609' (Mir) for STS-Mir docking missions.

The TDRS satellites should be listed first to prevent that the ZOE lines

are painted on top of the space shuttle icon. All these satellites are

then shown in the multisat display, and the ZOE is valid for the currently

selected satellite and its instantaneous height above the Earth. The ZOE

is the 'medium grey' vertical oblong feature over the Indian Ocean. Another

interesting feature occurs, if one of the TDRS satellites is selected with

the cursor to be the active satellite. A small grey circle then shows the

ZOE of the other TDRS satellite with respect to the first one, if the first

one could move. If a TDRS satellite is marked or selected, the display

shows its foot print rather than its visibility as seen from another

selected satellite.

If more than one space shuttle is in the multisat list, their mission

numbers are shown next to the space shuttle icon. If a space shuttle is

in the list and it has not been launched yet, it is shown located at

Kennedy Space Center, in launch position (without external tank and SRBs,

though). If this shuttle is selected with the cursor the ground track

represents the first three orbits after launch, or more precisely after

the epoch of the pre-launch element set, which is usually the time of the

beginning of orbit 2. During the time period between launch and start of

the second orbit, the shuttle icon will be placed at the intersection of

the equator and the zero meridian.

Last not least, the user might ask why the shuttle in the launch position

is not shown in 'grey' when it is dark in Florida. Well, one has

to imagine that it is the time of the night viewing and the shuttle it lit

up with high power xenon lights (this is also considered a feature, not a

bug!).

If the error message 'Bad environment variable DISPLAY' occurs when the

graphics window is turned on, this indicates that DISPLAY is ill defined.

It is not sufficient to specify DISPLAY as ":0.0" because this lead to

program crashes on various systems when the graphics window was turned off

and on repeatedly. The DISPLAY variable has to be set to something like

"host:0.0".

BUGS AND IMPERFECTIONS

Some minor bugs from the V2.0 release have been fixed. If additional bugs

are discovered after the V3.0 release, those will be fixed in a subsequent

version of SatTrack. SatTrack has been carefully tested on SPARCstations

with SunOS 4.1.3 or Solaris 2.4, and X11R5.

It was noticed, though, that the graphics window is not safe on a Sun-3

with SunOS 4.1.1 and X11R4, using the Sun C compiler. When the graphics

window is turned off and then on again a second time by typing 'g' on the

keyboard, the program crashes with a segmentation fault and core dump.

It seems that when the graphics window is turned off and back on again,

SatTrack uses more memory, each time this is done. This indicates that

the clean-up procedure for destroying the graphics window is not done

perfectly right yet.

There are some imperfections arising from the fact that SatTrack is not

a pure X Window application. The idea is to keep the main body of SatTrack

general in the sense that it should run on any VT100 type terminal. Two

different keyboard input drivers are presently used, depending upon the

graphics display being turned on or off. If the graphics window is turned

on, the keyboard input driver is a "custom made" one and uses 'BS'. If the

graphics display is turned off, the input driver is 'fgets' and uses 'DEL'

as the delete character. This was done to insure that the graphics display

can be refreshed while the program is waiting for input and the user

at this time exposes the graphics window. Even though this situation is not

ideal, it is not really bothersome either. It will be improved, however, in

a later release.

Also, the event handling needs more work to prevent 'flicker' in case

multiple 'exposure' or 'visibility change' events occur.

It should be mentioned that when the program is running under the X Window

system, with or without the graphics window, SatTrack will be suspended by

the window manager (as are all other programs) while the user is moving or

resizing a window, or more generally, while certain mouse buttons are

depressed. Especially when the tracking is turned on, one should not use

the mouse too often, or at least keep the necessary activities short.

It also should be mentioned that the graphics display has been written in

a rather straight forward way. With every display refresh every single

feature in the map is redrawn, one on top of the other. This is in a

certain way a waste of computing bandwidth, but much easier to program

than keeping track of every single pixel and its present color. The author

figured that during times when a computer is already obsolete by the time

it is purchased, it is easier to not spend too much time on optimizing the

software with respect to bandwidth, because by the time this job would be

accomplished, much faster machines are available already for the same

price.

It seems at times to be difficult to install this software on UNIX machines

other than Sun workstations, because some of the libraries have different

names or are bundled differently. There also seem to be various problems

with the compilation on machines with different versions of UNIX. A

substatial effort has been made, with the help of a number of users of

different machines, in order to make the code as easily portable as

possible.

The author would appreciate to be made aware of bugs or other imperfections

discovered by any users. In this case, or if you have any suggestions,

please send a note to 'manfred@sunspot.ssl.berkeley.edu'.

ACKNOWLEDGEMENTS

The author wishes to thank a number of individuals, in particular

D. Brown, G. Clark II (N5VMF), N. Clifford, C. Degiacomi,

R. Engberg (K0MVL), R. Frohne (KL7NA), B. Jones, K. Kudielka (OE1KIB),

R. Kupiec (N3MML), E. Lipman, W. Marchant (KC6ROL), D. Packman,

C. Panek (KX7L), J. Perry (KG5RG), U. Watson, K. Yamada (JR1EDE) and

PE1CHL for suggestions and/or help with fixing problems of compilation of

this software on various machines. Also many thanks to T. S. Kelso and the

JSC/AMSAT SAREX team who make available the two-line Keplerian element

sets. Without these input data SatTrack would be completely useless.

COPYRIGHT

Copyright (c) 1992-1995 Manfred Bester. All Rights Reserved.

Permission to use, copy, modify, and distribute this software and its

documentation for educational, research and non-profit purposes, without

fee, and without a written agreement is hereby granted, provided that the

above copyright notice and the following three paragraphs appear in all

copies.

Permission to incorporate this software into commercial products may be

obtained from the author at the address given above.

IN NO EVENT SHALL THE AUTHOR BE LIABLE TO ANY PARTY FOR DIRECT, INDIRECT,

SPECIAL, INCIDENTAL, OR CONSEQUENTIAL DAMAGES ARISING OUT OF THE USE OF

THIS SOFTWARE AND ITS DOCUMENTATION, EVEN IF THE AUTHOR HAS BEEN ADVISED

OF THE POSSIBILITY OF SUCH DAMAGE.

THE AUTHOR SPECIFICALLY DISCLAIMS ANY WARRANTIES, INCLUDING, BUT NOT

LIMITED TO, THE IMPLIED WARRANTIES OF MERCHANTABILITY AND FITNESS FOR A

PARTICULAR PURPOSE. THE SOFTWARE PROVIDED HEREUNDER IS ON AN "AS IS"

BASIS, AND THE AUTHOR HAS NO OBLIGATIONS TO PROVIDE MAINTENANCE, SUPPORT,

UPDATES, ENHANCEMENTS, OR MODIFICATIONS.

(the postscript files are not included in all distributions to save space)

Fig. 1:  Example of several live displays on the X Window screen

(see postscript file 'sattrack_xscreen.ps')

Fig. 2:  Example of the single satellite live display

(see postscript file 'sattrack_singlesat_disp.ps')

Fig. 3:  Example of the multi satellite live display

(see postscript file 'sattrack_multisat_disp.ps')

Fig. 4:  Example of the X Window graphics display for multiple satellites

(see postscript file 'sattrack_graphics.ps')

Fig. 5:  Example of a prediction table in short form

(see postscript file 'sattrack_short_pred.ps')

Fig. 6:  Example of a prediction table in long form

(see postscript file 'sattrack_long_pred.ps')

Fig. 7:  Example of a prediction table for solar transits

(see postscript file 'sattrack_solar_transit.ps')