Introduction

SIM68

Version 2.21H

Welcome  to the latest release of SIM68,  the  Motorola  MC68HC11

simulator  designed    to    allow  the    product  engineer  and

programmer to run and  debug  68HC11 .S19 object code  on  a  IBM

or equivalent PC.

SIM68  allows you to load and actually watch your object code run

in  a manner  similar to the format of the BUFFALO  monitor,  but

with a  full screen display on an IBM compatible PC.

Below is a list of some of the SIM68 programming features:

- Set and alter program breakpoints

- Enable a full time display of memory location contents  during

program execution (single bytes)

- Enable the display of 32 bytes of contiguous memory, ie useful

for examining buffers and caches (new feature)

- A CPU cycle stop watch which allows you  to  perform  actual

CPU cycle timing of program loops

- A memory mapping feature displaying the actual block of  memory

that code has been loaded into

- A feature that allows the modification of register and  memory

location contents

- Full support of all 68HC11 Op Codes, for a non-test mode system

- The  ability to run a user defined assembler and  editor  from

within SIM68

- The ability to simulate all of the MC68HC11 interrupts at  any

point during program's execution

- SIM68 now displays source code instead of disassembled object

code.  This feature is based on the presence of an AS11

(Motorola's freeware Assembler) list file ie.

AS11 test.asm -l > test.lst

- The ability to display and modify memory contents  dynamically

during program execution

- Improved  run time simulation speed,  ie faster screen updates

and code interpretation

- Logging of simulator session to disk or printer

- Full EEPROM support

- Full timer and PACC support

- The ability to save and restore previous SIM68 sessions

Disclaimer

No warranties of any kind are expressed or implied by  myself  or

this product.   I assume no responsibility for any damages caused

as a result of using this package.  I hereby commit SIM68 version

2.21H into the public domain, for use in private  and  commercial

engineering environments.

The Future

The  SIM68  simulator  package  will continue to grow and evolve,

gaining the  ability   to  emulate  more  of  the  true  internal

functions  of  the Motorola 68HC11 mirco-controller.

Version 3.0 of Sim68 will have the ability to display "real-world"

I/O screens and A/D voltage levels (see demos), as well as code

profiling and many more new features.  Version 3.0 is expected to

start shipping during December 1993.

The    SIM68   package is written in the C  programming  language

(using Borland's  C/C++ compiler) and TASM assembler.

If  you  have any problems or suggestions please write or fax me at

the following address:

Perry J. Fisch

2801 Orchard Drive

Palm Harbour, FL  34684

U.S.A.

Voice/fax (813)786-3743 (9AM - 9PM US EST)

The Catch...

This version of SIM68 is public  domain,  however,  this  release

of SIM68 does not contain the SIM68 help file and other programming

sample files.  To receive the "full" package, note that the SIM68

file is fully functional, but does not contain the help files and

other 'stuff' (80 page user manual), please send $25.00 U.S.,

non-US orders please add $10 (U.S.) for shipping and handling.

If you like how SIM68 works,  and find it useful I  suggest  that

you  register.    I am sorry that I must ask for money,  however,

this simple project that I started a while ago has turned into  a

project that continues to consume vast amounts of my time.

System Requirements

The  current  SIM68 package (Version 2.21H) requires a minimum of

384Kb or memory,  monochrome or color monitor,  and  an  optional

printer.

As  stated above the 384 Kb of memory is what SIM68 needs to load

and operate,   any additional memory will be used to support  the

external editor and assembler.

Installation of SIM68

Simply  copy  the  SIM68.EXE and SIM68.CFG file to a working

directory.    The SIM68.CFG file contains your color preferences

as well as  the choice of editor, assembler and working directory

name.

To start SIM68 just type:

sim68 <cr>,  where <cr> is the carriage return or enter

key

SIM68  will realize that this is the first time the package   has

been  run    and    automatically  route you to the install SIM68

menu.    Simply answer the questions as presented,  and save your

choices.

HINT: Type sim68 -? for a list of command line options.

Running SIM68

After typing sim68 at the DOS prompt you will be presented with a

menu of six choices:

1) Run SIM68

2) Change Working Directory

3) Run User Defined Editor

4) Run User Defined Assembler

5) Configure SIM68

6) Exit to DOS

Note  this is not a literal list of options,  but I hope it helps

convey the meaning of the options.

Option 1 is obviously the work horse of the system, this is where

the user S19 code is actually run.   After  the  simulator  loads

your  S19  file  into it's memory you will see the working screen

with a display of register contents and a  comment  window.    At

this  point  hit  the  F1  key  and  the package will give a good

description of all of it's abilities.

From  this  point forward you are  on  your  own.    I'm  in  the

middle   of version 3.0 and I've got a lot of work to do,  not to

mention the  fact that  I wrote SIM68 to be a tool not a product,

and I've got  alot  of work to do on my 68HC11 system.

Good luck, and I hope this proves useful you all.   Please let me

know if  you have any problems with the package, I really do want

to   make this a useful product,  not to mention make a few bucks

too!

Perry Fisch

P.S. Remember, type:    SIM68 -? to see a list of program options.