Plot Comparisons

Here are some antennas whose patterns are interesting to compare using

the MNPLOT instant plot compare feature:

2ELYAGI - 3ELYAGI

This compares a 2 element Yagi optimized for highest gain with a 3

element Yagi optimized for smallest sidelobes.  It is surprising that both

antennas have essentially the same forward lobe shape, with the differences all

in the rear half of the plane.  The gain difference between the two antennas is

about 1 dB.

204BA - 3ELYAGI

The above 3 element Yagi is compared with the classic Hygain 204BA 4

element 20 meter beam, widely regarded as a very good short-boom antenna.  At

the center of the band the gains are the same and the rear patterns are very

similar, but the boom length of the smaller Yagi is only 21 feet, whereas the

204BA has a 26 foot boom and an extra element.  When examined over the whole

band the 204BA shows a little better SWR and F/B, but not by much.  This

example illustrates the savings possible using computer-aided antenna design

compared with the traditional empirical approach.

2ELYAGI1 - 2PHASED

A conventional small Yagi is compared with a similar antenna having

both elements driven.  The improved backlobe performance is remarkable.  The

Yagi director was adjusted to provide the same gain as that of the phased

array.  Both designs have a forward gain about 0.5 dB below that obtainable

from an all-out max gain design which ignores sidelobe level.

BEVERAGE - BEVERAG1

You can see immediately where the longer Beverage gets its gain, by

noticing that it drops nulls right on top of the sidelobe peaks of the shorter

antenna.

N6BV - K1FO

It is interesting to compare two highly optimized Yagis, one for HF and

the other for UHF.  If you have ever wondered what the pattern of your big HF

array would look like if you could somehow add another 6 dB of gain, here it

is.

K1FO - W1JR

On 432 MHz we have a 22 el 14 ft Yagi vs. a 31 el 24 ft model.  The

K1FO design was developed for stacking in big UHF arrays, while the W1JR Yagi

is the highest gain single antenna I have yet come across.  It is difficult to

go back to your HF tribander after looking at these lovely patterns.

VDIPOLE - SLOPER45 - SLOPER60

Use the N key to normalize the actual gain difference between the

antennas into the plots.  Both slopers have higher gain than the vertical

dipole, but it is all at high angles and thus not very useful for DX.  However,

the 60 degree sloper does not sacrifice any gain to the dipole at angles below

15 degrees.  Thus it can be used to reject high angles signals off the rear of

the antenna for receiving, while preserving good low angle forward gain for

transmitting.  The 45 degree sloper has better high angle directivity than the

60 degree job, but loses about 1 dB of gain to the vertical at angles below 10

degrees.

3ELQUAD - 3ELYAGI

Both of these antennas were optimized for low sidelobes.  The Yagi lost

about 1.5 dB forward gain in so doing, but the quad only a few tenths of a dB.

The quad has about 2 dB more gain than the Yagi, but about 10 db worse sidelobe

performance.  (The quad really needs more segments for best accuracy, and in

addition, it could be optimized further for better pattern.)

QUADGND - YAGIGND

Each antenna uses 3 elements on a 0.3 wave boom, and the boom of each

is located 0.5 wave above ground.  This plot comparison is designed to test the

widely-held notion that quads have a lower angle of radiation than Yagis when

mounted low to the ground.  This and the previous comparison are good to have

at hand during your next Quad vs. Yagi argument (no matter which side you

take).

3ELYAGI - 3ELYAGI2

Here we have a max gain Yagi vs. one optimized for a small rear

pattern.  The max gain antenna really isn't a very practical design, as the

impedance bandwidth is too narrow to be of much use, except on spot frequencies

or the narrow 12 meter band.  But note how the forward lobe of the max gain

design is significantly narrower than that of the other antenna.  This

illustrates how the use of a single F/B number to describe an antenna's

directivity characteristics can be quite misleading.  If you are a W6 trying to

work Europe on the short path, and want to reject loud East Coast stations as

much as possible, the max gain Yagi would be a better choice than the antenna

having the high F/B.

3ELYAGI - 3OVER3

A single Yagi vs. a stacked pair of the same antennas.  The single Yagi

was optimized in free space for best pattern.  Note how the presence of a

second antenna, 44 feet away, degrades the pattern considerably.  The stacked

pair should be reoptimized as a set.

2ELYAGI - 2OVER2

Identical Yagis used for both.  This Yagi was optimized in free space

for maximum forward gain, resulting in a large back lobe, so that the pattern

degradation due to stacking is not significant.  The pair should probably be

reoptimized as a set for maximum gain, based on the results of stacking the 3

element Yagis.

DIPOLE - VERTICAL

This is perhaps the most surprising plot comparison of the lot.  Note

how the horizontal antenna has several dB advantage over the vertical right

where the vertical is supposed to be king, at low wave angles.  At the lobe

peaks, around 30 deg, the dipole is 7.5 dB better.  Average earth

characteristics were used for both antennas.  Try the comparison with other

earth characteristics.

TURNSTIL - SQUALO

Two more or less omnidirectional horizontal antennas.  The differences

in omnidirectionality are immediately visible using the plot compare feature.

BROADSID - W8JK

Two simple wire arrays yielding about the same gain, but with very

different azimuth and elevation patterns.  One uses broadside elements, and the

other collinear and endfire elements.

LAZYH - BISQUARE

Each has a pair of broadside and a pair of collinear elements, but the

Bisquare elements are not parallel, costing about 3 dB gain.  Note how the

nulls in the Lazy H pattern are filled in for the Bisquare.

2INPHASE - EXTDZEPP

Note how different the azimuth pattern becomes just by lengthening the

wire 28%.