GPLOTLIB Introduction

By John Campbell 3/90

GPLOTLIB is an attempt to create a library of graphics routines out of the

gnuplot package in order to gain access to all the terminal drivers that

have been written.  Note that gnuplot itself does not use these routines,

although the routines here have simply been extracted from the original

code.  Unfortunately for maintainers, some of the routines had to be

changed--generally simplified--in order to create a usable graphics plot

library.

Note that an effort was made to avoid name space pollution.  This wasn't

necessary in the original gnuplot code, but a library that is going to

possibly be embedded in other packages should take some care regarding the

names used by global symbols.  To this end, all globals begin with either

GT or GSR and all external routine names begin with either ``gt_'' or

``gsr_''.  Hence the routine ``test_term()'' from gnuplot/term.c becomes

``gt_test_term()'' in gplotlib/gterm.c.  Many other internal routines

were made static to keep them from being external to the module.

The package itself consists of two 'C' modules and two header files:

gsr.h, gsr.c and gterm.c, gterm.h.  The latter pair directly use the

files in gnuplot/term and should be able to handle new terminal drivers

by changing gterm.c exactly as term.c had to be changed.  This is the

lowest level of the library, providing direct access to the terminal

drivers using the external structure ``GTterm_tbl[]'' and the index

``GTterm''.

In order to use this lowest level of GPLOTLIB, you need to know how the

terminal drivers of gnuplot function.  Russell Lang's README file from

gnuplot/term is included below with some  slight modifications (such as

changing the names of global variables to more closely match those here

in GPLOTLIB).

The other half of the package, gsr.h and gsr.c, provides the same sort of

graphics support routines that gnuplot itself needed in the module

graphics.c.  Of course, gnuplot knew ahead of time what order it was

going to build it's plots and didn't need generic routines.  These gsr

routines attempt to provide a "high-level" user coordinate based plotting

package that allows users to easily do tasks similar to those that gnuplot

does during interactive plotting.

Note that the documentation for all routines is derived directly out of

the 'C' code using a utility I wrote years ago called ``docu''.  Docu looks

for 'C' comments of the form /*- ... -*/ and extracts the text and any lines

of code above the text to stdout.  Thus, only the items deemed of interest

by the library writer are exported to the outside world.  The intent, of

course, is that the documentation is accurate--at least the calling sequences

should be exact, as of the time the documentation was run off, because it

shows the actual routine header.  Note also, that I filter the output of

``docu'' through another program called ``cpr''.   The version of ``cpr''

(code printer) that I use is probably different than all but the most

recently submitted version from Dennis Vadura.

DOCUMENTATION FOR GNUPLOT TERMINAL DRIVER WRITERS

By Russell Lang 1/90

Information on each terminal device driver is contained in term.c and

the term/*.trm files.  Each driver is contained in a .trm file and is

#include'd into term.c.  Each driver has a set of initializers in

gterm.c for GTterm_tbl[], an array of struct termentry.

Here is the definition of the struct termentry from gtplot.h:

struct termentry {

char *name;

char *description;

unsigned int xmax,ymax,v_char,h_char,v_tic,h_tic;

FUNC_PTR init,reset,text,scale,graphics,move,vector,linetype,

put_text,text_angle,justify_text,point,arrow;

};

Here's a brief description of each variable:

The char *name is a pointer to a string containing the name

of the terminal.  This name is used by the 'set terminal' and

'show terminal' commands.

The name must be unique and must not be confused with an abbreviation

of another name.  For example if the name "postscript" exists, it is not

possible to have another name "postscript2".

Keep the name under 15 characters.

The char *description is a pointer to a string containing a

description of the terminal, which is displayed in response

to the 'set terminal' command.

Keep the description under 60 characters.

xmax is the maximum number of points in the x direction.

The range of points used by gnuplot is 0 to xmax-1.

ymax is the maximum number of points in the y direction.

The range of points used by gnuplot is 0 to ymax-1.

v_char is the height of characters, in the same units as xmax and ymax.

The border for labelling at the top and bottom of the plot is

calculated using v_char.

v_char is used as the vertical line spacing for characters.

h_char is the width of characters, in the same units as xmax and ymax.

The border for labelling at the left and right of the plot is

calculated using h_char.

If the _justify_text function returns FALSE, h_char is used to justify

text right or centre.  If characters are not fixed width, then the

_justify_text function must correctly justify the text.

v_tic is the vertical size of tics along the x axis,

in the same units as ymax.

h_tic is the horizontal size of tics along the y axis,

in the same units as xmax.

Here's a brief description of what each term.c function does:

_init()  Called once, when the device is first selected.  This procedure

should set up things that only need to be set once, like handshaking and

character sets etc...

_reset()  Called when gnuplot is exited, the output device changed or

the terminal type changed.  This procedure should reset the device,

possibly flushing a buffer somewhere or generating a form feed.

_scale(xs,ys) Called just before _graphics(). This takes the x and y

scaling factors as information. If the terminal would like to do its

own scaling, it returns TRUE. Otherwise, it can ignore the information

and return FALSE: GSR routines can do the scaling for you. null_scale is

provided to do just this, so most drivers can ignore this function

entirely. The Latex driver is currently the only one providing its own

scaling.

_graphics()  Called just before a plot is going to be displayed.  This

procedure should set the device into graphics mode.  Devices which can't

be used as terminals (like plotters) will probably be in graphics mode

always.

_text()  Called immediately after a plot is displayed.  This procedure

should set the device back into text mode if it is also a terminal, so

that commands can be seen as they're typed.  Again, this will probably

do nothing if the device can't be used as a terminal.

_move(x,y)  Called at the start of a line.  The cursor should move to the

(x,y) position without drawing.

_vector(x,y)  Called when a line is to be drawn.  This should display a line

from the last (x,y) position given by _move() or _vector() to this new (x,y)

position.

_linetype(lt)  Called to set the line type before text is displayed or

line(s) plotted.  This procedure should select a pen color or line

style if the device has these capabilities.

lt is an integer from -2 to 0 or greater.

An lt of -2 is used for the border of the plot.

An lt of -1 is used for the X and Y axes.

lt 0 and upwards are used for plots 0 and upwards.

If _linetype() is called with lt greater than the available line types,

it should map it to one of the available line types.

Most drivers provide 9 different linetypes (lt is 0 to 8).

_put_text(x,y,str)  Called to display text at the (x,y) position,

while in graphics mode.   The text should be vertically (with respect

to the text) justified about (x,y).  The text is rotated according

to _text_angle and then horizontally (with respect to the text)

justified according to _justify_text.

_text_angle(ang)  Called to rotate the text angle when placing the y label.

If ang = 0 then text is horizontal.  If ang = 1 then text is vertically

upwards.  Returns TRUE if text can be rotated, FALSE otherwise.

_justify_text(mode)  Called to justify text left, right or centre.

If mode = LEFT then text placed by _put_text is flushed left against (x,y).

If mode = CENTRE then centre of text is at (x,y).

If mode = RIGHT then text is placed flushed right against (x,y).

Returns TRUE if text can be justified

Returns FALSE otherwise and then _put_text assumes text is flushed left;

justification of text is then performed by calculating the text width

using strlen(text) * h_char.

_point(x,y,point)  Called to place a point at position (x,y).

point is -1 or an integer from 0 upwards.

6 point types (numbered 0 to 5) are normally provided.

Point type -1 is a dot.

If point is more than the available point types then it should

be mapped back to one of the available points.

Two _point() functions called do_point() and line_and_point() are

provided in term.c and should be suitable for most drivers.

do_point() draws the points in the current line type.

If your driver uses dotted line types (generally because it is

monochrome), you should use line_and_point() which changes to

line type 0 before drawing the point.  line type 0 should be solid.

_arrow(sx,sy,ex,ey)  Called to draw an arrrow from (sx,sy) to (ex,ey).

An _arrow() function called do_arrow() is provided in term.c which will

draw arrows using the _move() and _vector() functions.

Drivers should use do_arrow unless it causes problems.

The following should illustrate the order in which calls to these

routines are made:

_init()

_scale(xs,ys)

_graphics()

_linetype(lt)

_move(x,y)

_vector(x,y)

_point(x,y,point)

_text_angle(angle)

_justify(mode)

_put_text(x,y,text)

_arrow(sx,sy,ex,ey)

_text()

_graphics()

.

.

_text()

_reset()

[And, of course, gt_test_term() in gterm.c is a real live example of

calling some of these routines. JDC]