Using the Media Vision ThunderBoard Card with OS/2 2.1

Version 1.0	dated 7/26/93

------------------------------------------------------

As many of you know, Media Vision has perfected Sound Blaster compatibility

and all of our sound cards that claim 100% Sound Blaster are, in fact,

compatible at the hardware level.

Paradoxically, the SoundBlaster driver that ships with OS/2 2.1 (SBD2.SYS)

does not work with the Media Vision ThunderBoard nor the SoundBlaster

section of the Pro AudioSpectrum 16, even though the hardware is the same.

Here's the reason why:  When Media Vision cloned the original Sound Blaster

(calling our clone 'ThunderBoard'), we added some extra features. For example,

the ThunderBoard has the ability to record at up to 22KHz.  We needed a way

to check whether the card in the system was a Sound Blaster or a ThunderBoard

to determine whether or not to allow the enhanced functionality.

The method we came up with was to have a 'hidden' version number.  The first

time a program asks a ThunderBoard (TB) for its version number, the TB

returns the same value the Sound Blaster returns.  The second time, the TB

returns its own version number (which is different from the Sound Blaster

version number.)

The Sound Blaster people discovered our trick and now they put a _double_

version check into their code to detect the ThunderBoard.  We think

that's a fair and reasonable thing to do, as long as they don't crash the

user's system or do anything else destructive.  Claims that the ThunderBoard

isn't 100% compatible are obviously misleading.

The OS/2 driver peforms a double version check and so it's possible to bypass

the second check and allow the ThunderBoard to work.  All that is necessary

is to change a single byte.  We're telling you which byte it is here so that

you can verify the change or perform it yourself.  We've also supplied a

patched version so that you can avoid the details and just start using the

driver.

The byte to change is at offset 3ED8 (hexadecimal) in the file.  Change

the value from AB to 00.  Here's how to install the patched driver.

First, make sure you've installed the MMPM/2 SoundBlaster driver.

Before installing the Sound Blaster driver, be sure you know what

the IRQ jumper setting is for your ThunderBoard.  Use the MINSTALL

program to install the SoundBlaster driver.  After all files have been

copied to your system, a Window entitled "Sound Blaster Installation" will

appear.  It will ask you to select which type of Sound Blaster card is

to be installed.

Choose the:

Sound Blaster (ISA and MCV)

selection.  Be careful.  The default choice is

'SoundBlaster Pro (ISA with OPL2)' and that driver will not work.

Then you will be asked how many Sound Blasters are installed.  Select 1.

The next screen asks you for the DMA, I/O Address and IRQ.  Most cards

should be set to DMA 1 and I/O address 220.  Some ThunderBoard owners

may need to change the IRQ setting to 5 or 2.  Those are the only other

choices that the driver offers.

When the Installation program has finished, go to an OS/2 command window

and perform these commands:

[C:\]cd \mmos2

[C:\MMOS2]copy sbd2.sys sbd2.old

[C:\MMOS2]copy a:\sbd2.sys

The last command assumes the patched SBD2.SYS file provided with this

document is on the A: drive.  If it is not, substitute the proper path

to the patched file.

Now reboot your computer and you should have sounds.

If you wish to patch the Sound Blaster driver yourself, here's how:

Remember that symdeb and debug adds 100h bytes to the offset so the byte

you must change will be at offset 3FD8.

Start a DOS session.

[C:\]cd \mmos2

[C:\MMOS2]copy sbd2.sys sbd2.old

[C:\]debug sbd2.sys

-e 3fd8 00

-w

Writing 8A7A bytes

-q

If the above Debug session does not make sense to you, do not attempt the

patch yourself.  Ask a DOS expert.

MISCELLANEOUS NOTES:

*  Digital audio appears to work fine.

*  FM synthesis (MIDI player) doesn't appear to work.  We don't know why.

If you get it to work, let us know.

*  In theory, this driver should work on the Sound Blaster section of the

PAS 16.  This would allow you to have two Sound drivers working

at the same time in MMPM.  We have not tested this. If you get it

to work, let us know.  If you can't get it to work, we don't want to

know.

*  The ThunderBoard does not have a MIDI interface!  Do not waste your

time or money trying to use a MIDI connector box.

*  Since we have only patched this driver and don't have the ability to

modify the original source code, do not expect any bug fixes or

changes to come from Media Vision.

*  Media Vision's Tech Support department will not be able to assist

you with this driver.  If you have comments or questions about this

driver send them to:

Internet: 75300,2772@compuserve.com

CompuServe: Media Vision [75300,2772]

FAX: (510) 226-2582, Attn. OS/2 engineering

*  Many people have asked whether Media Vision will write a ThunderBoard

driver for OS/2.  The answer is 'probably not'.  We recommend that our

ThunderBoard customers upgrade to the Pro Audio Spectrum 16.  Prices

in computer specialty stores have been as low as $139.  That's an

excellent price!  And the PAS 16 is fully supported under OS/2.  Even

the SCSI interface.

If you'd like to learn more about Media Vision's products, call our

Sales department.

Media Vision, Inc        Main:         (800) 348-7116 or (510) 770-8600

3815 LaurelView Ct.      Sales:        (800) 845-5870

Fremont, CA 94539        FAX:          (510) 770-8648 or (510) 770-9592

Customer Service and Tech Support: (800) 638-2807 or (510) 770-9905

ISV Relations and Developer Assistance: (800) 472-6147

BBS: (510) 770-0968 2400

(510) 770-0527 9600

There are a total of 8 lines and all lines support

8 bits, no parity, 1 stop bit, 300 - 14,400 bps

CIS: GO MEDIAVISION or send CIS mail to 75300,2772

European Customers:

Media Vision Technology GMBH

Raiffeisenalle 16

8024 Munchen-Oberhaching

Germany

Main:                       89-61381-175

Tech Support:               89-61381-167

FAX:                        89-61381-174