COMMUNICATION HP-71 <=> PC

Michael Markov (PPC Paris 301)

This file documments three HP-71 programs I use to upload and download files to

and from my HP110. They can also be used to do the same with IBM PC or

compatible machines equipped with an HP82973 HPIL Interface.

Why do I need special programs to do this, when HPILLINK provides the ability

to upload and download files from the PC keyboard? Because I only want to

exchange files with the PC, not use a remote keyboard for my HP-71. The editor

I have on the PC or the HP110 (Ed Gilbert's Editor, NOT memomaker) is much

faster than any text editors I have on the HP-71, they handle long text files

without slowing down to the speed of cold molasses, and have features HP-71

users only dream of. Editing BASIC programs is usually faster without the delays

introduced by the remote keyboard. Finally - HPILLINK set-up with the remote

keyboard does a number of unfriendly things, such as mess-up my STARTUP, my key

assignments, or even FREEPORT memory I do not want FREEPORTED, if I do not have

KEYBOARD LEX in my machine.

Still, it is nice to have mass storage and printers at work, by way of HPILLINK

and the IBM XT in my department. Being able to share data stored in my HP-71

using the HPILLINK /C option helps keep my bosses happy with my HP-71. Hence,

the routines below.

NOTE: LINK 1.02 (Southern Software) provides similar capabilities. I hear there

soon will be a new version called LINK PLUS, with many interesting features.

I use LINK 1.02 rather than HPILLINK if I want to use the PC as a printer or

as a video interface for my HP-71.

The first program is intended to upload or download a single file at a time,

from the HP-71 keyboard. Before using this program, you must run the

HPILLINK.EXE program on the PC, and connect the HP-71 to the PC HP-IL port.

When you download a file, you must provide the full file name, such as:

JUNK.TEX

When you upload a file, the program will automatically supply the correct file

extension if you have the FTYPE$ keyword available. See source code below.

If not, the extension will be left blank, and you will have to RENAME the file

from the PC keyboard if you want the correct extension.

The file transfer takes little or no time. Once done, disconnect the HP-71,

exit HPILLINK with the F8 key, and do whatever you desire with either the PC or

the HP-71. Enjoy!

0010 ! PC71EX - Transfer HP71 files between a PC and an HP71 with HPILLINK

0020 ! Copyright Michael Markov 1988

0030 !

0040 ! Uses FTYPE$ keyword from DATACOM ROM - See FLTYPELXS for source

0050 ! This is required if you want to provide the proper file extension

0060 ! when uploading files to an IBM PC via HPILLINK /HP82973 Interface card

0070 ! or to an HP110 running HPILLINK.

0080 !

0090 ! If you do not have the FTYPE$ keyword, comment-out the line 300 :

0100 ! 300 T$=FTYPE$(F$)

0110 !

0120 ! The advantage of this program is that it allows file transfers without

0130 ! messing-up your favorite STARTUP, your key assignments, or FREEPORTing

0140 ! stuff you may not want free-ported. This was developed when I started

0150 ! to make heavy use of my HP110 to edit HP71 source code files, which I

0160 ! then assembled using the AREUH (PPC-Paris) assembler-linker before

0170 ! downloading the resulting LEX for testing in the HP71. I had no need for

0180 ! an external keyboard since the EDITOR I use on my HP110 is considerably

0190 ! faster than EDTEXT, and provides more flexibility.

0200 !

0210 LOCAL @ DISP 'Up or Download?(U/D)'

0220 A$=UPRC$(KEYWAIT$) @ IF A$='D' THEN 240

0230 IF A$='U' THEN 280 ELSE 210

0240 LOCAL @ INPUT 'DOWNLOAD FILE? ';F$ @ IF F$='' THEN END

0250 P=DEVADDR(':PC')

0260 SEND UNT TALK P DDT 0 UNT MTA LISTEN P DATA F$ EOL

0270 COPY :P @ GOTO 240

0280 INPUT 'Upload file? ';F$ @ IF F$='' THEN END

0290 P=DEVADDR(':PC') @ T$=''

0300 T$=FTYPE$(F$)[1,3] ! comment out if FTYPES keyword not available

0310 SEND UNL LISTEN P MTA DDL 0 DATA F$,'.',T$,10

0320 COPY F$ TO :P @ GOTO 280

The second program is a minor variant of the PC71EX program given above. It

allows you to upload or download ROM images just as if the PC was an HPIL disk

drive. There is one interesting difference: the file stored on the DOS disk will

take less space than on HPIL mass storage. Apparently, HPILLINK and ROMCOPY

version RCPY:E were designed to work together better than the mass storage

option! Here, we see the advantages of an intelligent mass storage device.

0010 ! PCROM - Transfer HP71 ROM images between a PC and an HP71 with HPILLINK

0011 ! and ROMCOPY LEX.

0020 ! Copyright Michael Markov 1988

0030 !

0040 ! This program is a slightly modified version of PC71EX, elsewhere on this

0041 ! swapdisk. See PCCOMM for documentation.

0042 !

0210 LOCAL @ DISP 'Up or Download?(U/D)'

0220 A$=UPRC$(KEYWAIT$) @ IF A$='D' THEN 240

0230 IF A$='U' THEN 280 ELSE 210

0240 LOCAL @ INPUT 'DOWNLOAD FILE? ';F$ @ IF F$='' THEN END

0241 INPUT 'To :PORT(#)? ','5';N

0250 P=DEVADDR(':PC') @ IF NOT POS(F$,'.ROM') THEN F$=F$&'.ROM'

0260 SEND UNT TALK P DDT 0 UNT MTA LISTEN P DATA F$ EOL

0261 X=POS(UPRC$(F$),'.ROM') @ F$=F$[1,X-1]&':'&STR$(P)

0270 ROMCOPY F$ TO ':PORT('&STR$(N)&')' @ GOTO 240

0280 INPUT 'Upload file? ';F$ @ IF F$='' THEN END

0281 INPUT 'From :PORT(#)? ','5';N

0290 P=DEVADDR(':PC') @ X=POS(UPRC$(F$),'.ROM') @ IF X THEN F$=F$[1,X-1]

0310 SEND UNL LISTEN P MTA DDL 0 DATA F$,'.ROM',10

0320 ROMCOPY ':PORT('&STR$(N)&')' TO :P @ GOTO 280

The last program allows you to upload many files to the PC, under the control

of an HP-71 program. I have used it to transfer entire swapdisks to my HP110,

for friends that have HP-71 machines and PC's, but not HP9114 drives. They then

read the disks as best they can. I understand that there is a new package from

Personalized Software called READHP, that allows IBM machines to read 3.5" disks

formatted by HP machines.

It should be possible to download files from the PC with a PCTODISK program,

just as easily as you can upload entire disks. The easiest approach would be

to start things going by creating a TEXT file with all the desired filenames

and extensions with the DIR A: >DIR.TEX. Then use PC71EX to download the file,

and let your HP-71 control the download to an HPIL mass storage device. I have

not needed to do this, since I am not (as of this date) getting any

contributions on DOS disks. However, if HP abandons HPIL, this may be the only

way to have peripherals for our HP41/HP75/HP71 computers, may they last many

years. (Have you purchased a spare HP-71 yet? NO? Why?).

NOTE: PCTODISK does not yet exist, except as an idea. It is one of my many

'to do sometime' projects.

Anyways, easy PC<==>HP71 communications will greatly ease the transition to the

world of DOS/OS2 machines, where instead of HP BASIC you will be forced to learn

PASCAL, C, MEGA BASIC or QUICK BASIC if you want to do any serious programming.

Obvioulsy, the PC must be running HPILLINK, and the default drive to which

HPILLINK writes the files should be as empty as possible. Load HPILLINK with

A:HPILLINK, then replace the disk in the default drive (usually A:) with a

newly formated disk. On the HP71, run DISKTOPC. When the program stops, replace

the disk in the default drive, and run DISKTOPC on the HP71 to continue the

upload.

NOTE: Do NOT purge the temporary directory file (default DIRFL), except at the

start of the upload, before the first file is transfered. This allows the PDIR

keyword (JPCROM) to create a new file that contains all the information you need

to upload the disk. The HP-71 automatically deletes file entries as the upload

goes on. Any time the DISKTOPC program stops, you can LIST DIRFL to find out

how many files still ave to be uploaded.

Note that the program polls the PC to determine whether the remote load failed

for any reason, such as no space remaining available on the PC. This program is

intended to be used in conjunction with a video interface on the HPIL loop.

The program uses JPCROM to create a directory listing of the disk to be

uploaded to the PC. This is by far the most convenient way. Among other things,

JPCROM implements a poll handler that allows your HP-71 to identify HP-41,

HP-75 and other non-HP71-standard file types. This allows you to tag the file

names to help identify the contents of the file. HPILLINK does not recognize

HP-41 or HP-75 file types. In fact, it does not recognize all HP-71 file types,

atleast, not the version I have (Maybe revision E does better, but I do not have

that yet.

It should be obvious that minor changes to line 240 allow uploading only HP-71

files, or only HP41 files, etc, should you so desire.

0010 ! DISKTOPC - UPLOAD FILES TO PC WITH HPILLINK

0020 ! This program is available, at least in concept, in the HPILLINK

0030 ! documentation. It implements the task in a way that helps minimizes

0040 ! user intervention. It was written to facilitate uploading Swapdisks

0050 ! to an IBM PC equipped with an HPIL interface card.

0060 !

0070 ! Since the HP71 is the controller, you can have a disk drive on the loop,

0080 ! and copy each file from the drive to the IBM PC, using a minimum of

0090 ! HP-71 memory.

0100 ! JPCROM REQUIRED. IF NOT AVAILABLE, CREATE DISK DIRECTORY LISTING WITH

0110 ! SOME BASIC ROUTINE SUCH AS DIR71

0120 CONTROL ON @ RESET HPIL @ RESTORE IO @ DISPLAY IS :DISPLAY

0130 INPUT "Which drive? ";D

0140 P=DEVADDR('%16('&STR$(D)&')')

0150 INPUT 'temp. file? ','DIRFL';F$

0155 ON ERROR GOTO 161

0160 PDIR :P TO F$ @ GOTO 169

0161 OFF ERROR @ DISP ERRM$;'Purge? (Y/N)'

0162 A$=UPRC$(KEYWAIT$) @ IF A$='Y' THEN PURGE F$ @ GOTO 155

0169 OFF ERROR @ ASSIGN #1 TO F$ @ RESTORE #1

0180 DIM A$[60] @ D=DEVADDR(':PC') @ STANDBY ON @ REMOTE

0190 FOR I=0 TO FILESZR(F$)-1 @ READ #1;A$ @ DISP I;A$

0200 IF SPOLL(D)#5 THEN 270

0210 F$=TRIM$(A$[1,10])

0220 T$=TRIM$(A$[14,19])[1,3]

0230 IF T$='41:' OR T$='75:' THEN G$=F$&T$[1,2] @ T$='UNK' ELSE G$=F$

0240 SEND UNL LISTEN D MTA DDL 0 DATA G$,'.',T$,10

0250 COPY F$&':'&STR$(P) TO :D

0260 NEXT I

0270 LOCAL @ DISP 'HP110 Drive A full-delete last file copied'

0280 READ #1,I-2;A$ @ DISP 'last file copied: ';I-2;A$

0281 FOR J=I-2 TO 0 STEP -1 @ DELETE #1,J @ NEXT J

0290 STANDBY OFF @ CONTROL OFF @ END

LEX    'FTYPELEX'  LEX file name

TITLE  FTYPELEX, File type LEX,  Samuel H Chau [242]

*

ID     #F6         compatible with DCLEX in DATACOMM ROM

MSG    0           no message table

POLL   0           no poll handler

*

ENTRY  FTYPE

CHAR   #F          string function

*

KEY    'FTYPE$'    Syntax: FTYPE$('file specifier') or

*                                 FTYPE$(assign channel #)

*                         Returns file type as a 5-char string

*                         Null string returned if file not found

*                         Example:  FTYPE$('FORTHRAM') returns FORTH

TOKEN  1

ENDTXT             end of text table

************************

*      **** EQUate Table ****

*

AVE=D1 EQU #18BB8 update AVMEME from D1 or C

BSERR  EQU #0939A BASIC system error

D0=AVS EQU #09B2C set D0 to address in AVMEMS

D1MSTK EQU #1954E set D1 at MTMSTK (AVMEME)

FDCH#  EQU #114AC find channel # in assign table

FFIB#  EQU #122EF find file number in FIB

FILXQ$ EQU #09B95 filename execute for a string expression

FINDF  EQU #09F77 find a file

FLTDH  EQU #1B223 convert 12-digit flt to HEX integer

FNRTN  EQU #0F23C function return

FTYPDC EQU #06902 file type decompile

FUNCR0 EQU #2F89B function scratch RAM start

POLL   EQU #12337 poll LEX files with process #

POPMTH EQU #1B3DB skip past item on MATH stack

RSTK<R EQU #014A8 restore RSTK level(s) from RSTKBF buffer

R<RSTK EQU #014DD save RSTK level(s) into RSTKBF buffer

STRHED EQU #14C2E generate string head on stack

pFINDF EQU #00017 poll to find a file

pFSPCx EQU #00005 poll to execute file specifier

eFSPEC EQU #0003A invalid filespec error

*

************************

*

NIBHEX C11    one string or numeric parameter

FTYPE  CD0EX

RSTK=C        save D0 (PC) on return stack

GOSBVL R<RSTK save RSTK level(s)  [P=0]

A=DAT1 W      get top of MATH stack in A

LCHEX  A      load C:0 for parameter type check

?C>A   P      integer parameter?

GOYES  FTYP03 yes: interpret as channel #

A=A+1  P

?A#0   B      not string parameter?

GOYES  FTYP01 yes: parameter is invalid

GOTO   FTYP09 no: interpret string as file specifier

FTYP01 LC(4)  eFSPEC load Invalid filespec error

FTYP02 GOVLNG BSERR  BASIC error exit

FTYP03 D1=D1+ 16     pop integer argument off MATH stack

GOSUB  ave=d1 update stack top pointer

GOSBVL FLTDH  convert 12 form channel # to HEX integer

B=A    B      put HEX form in B for FDCH#

GOSBVL FDCH#  find channel # in assign table

GONC   FTYP08 not found: ready for exit

?A=0   B      channel closed? no: A(B)=FIB#

GOYES  FTYP08 yes: punt!

GOSBVL FFIB#  find file # in FIB

GOC    FTYP08 not found? yes: punt! no: D1 @ file # in FIB

D1=D1+ 5      advance to file type entry in FIB

FTYP04 GOSBVL D0=AVS set D0 to AVMEMS

C=0    W      preclear C

DAT0=C W      blank out word at AVMEMS

D0=D0+ 2      point past one null

GOSBVL FTYPDC get file type (5 chars), D0 points past it

GOSUB  d1mstk set D1 to AVMEME

GOSBVL D0=AVS set D0 to AVMEMS

C=DAT0 W      get file type in C

?C=0   B      not at file type string?

GOYES  FTYP06 yes: move pointer down

FTYP05 ?C=0   B      end of file type string?

GOYES  FTYP07 yes: ready for string return

D1=D1- 2      point D1 down 1 char

DAT1=C B      push one ASCII char onto stack

FTYP06 CSR    W

CSR    W      shift to next char in file type string

GOTO   FTYP05 continue building string on stack

FTYP07 GOSBVL STRHED generate string header on stack

GOSBVL RSTK<R restore RSTK level(s)

C=RSTK

CD0EX         restore D0 (PC) from RSTK

GOVLNG FNRTN  function return

FTYP08 GOSUB  d1mstk set D1 to AVMEME

GOTO   FTYP07 return with string

FTYP09 GOSUB  ave=d1 update stack top pointer

C=DAT1 W      read string header into C

CSR    W

CSR    W      shift to string length field

?C=0   A      null string?

GOYES  FTYP08 yes: ready for return

GOSBVL FILXQ$ execute tokenized file specifier

P=     0

LCASC  '  '   load C with 2 ASCII spaces

R0=C          save in R0

GOC    FTYP16 file specifier is mainframe recognizable

GOSBVL POLL   illegal: issue file spec execution poll

CON(2) pFSPCx file specifier execute poll

GOC    FTYP13 error return from poll: error exit

?XM=0         pFSPCx poll handled?

GOYES  FTYP10 yes: continue

GOTO   FTYP01 no: Invalid filespec error

FTYP10 C=R0          last 2 chars of filename into C

R1=C          save in R1

R0=A          filename (blank filled) into R0

GOSUB  d1mstk set D1 to stack top

GOSBVL POPMTH skip past current item on stack

GOSUB  ave=d1 update stack top pointer

GOSBVL POLL   issue poll to find file

CON(2) pFINDF file finder poll

GONC   FTYP11 no error: continue

A=C    A      save C in A

LCHEX  16     load C with HPIL File not found error #

?A=C   B      file not found on HPIL mass storage?

GOYES  FTYP12 yes: ready for return

C=A    A      restore C from A

GONC   FTYP13 B.E.T.

FTYP11 ?XM=0         pFINDF poll handled?

GOYES  FTYP14 yes: continue

FTYP12 GOTO   FTYP08 no: ready for return

FTYP13 GOTO   FTYP02 load BASIC error returned from LEX file

FTYP14 D1=(5) FUNCR0 set D1 to function scratch RAM

C=R0

DAT1=C W      put file attributes into function scratch RAM

D1=D1+ 11     advance to file type field

FTYP15 GOTO   FTYP04 go find file type

FTYP16 GOSUB  ave=d1 update stack top pointer

GOSBVL FINDF  find the file

GOC    FTYP12 not found: ready for return

D1=D1+ 16     move to file type field in file header

GONC   FTYP15 B.E.T.

ave=d1 GOVLNG AVE=D1

d1mstk GOVLNG D1MSTK

END