PROGRAMMING GUIDE TO THE SBOS2 DEVICE DRIVER

Michael Fulbright

October 24, 1992

The SBOS2 device driver provides a low level interface to the Sound Blaster

and Sound Blaster Pro sound cards. This document describes the programming

interface for SBOS2. For more information on SBOS2 send email to

Michael Fulbright

msf@as.arizona.edu

INTRODUCTION TO THE SOUND BLASTER/SOUND BLASTER PRO

---------------------------------------------------

This is intended to be a brief overview of the Sound Blaster (SBREG)

and Sound Blaster Pro (SBPRO) sound cards. There are many cards

available now which are SBREG compatible, so SBREG refers to all of

these cards. I will use SB to refer to both the SBREG and SBPRO sound

cards. This is by no means a complete tutorial on these sound cards,

but hopefully provides enough information to get started.

First lets summarize the hardware differences between the SB and SB Pro.

The SBREG provides a single 8 bit D/A output port and a single 8 bit

A/D input port. It can sample/playback at frequencies between

approximately 4000 and 22000 Hz.  It has no onboard mixer or user

controllable filtering. Newer versions of the SBREG allow recording

from a microphone or from a line in jack.

The SBPRO allows recording and playback in stereo, both channels

being 8 bits wide. It can operate between approximately 4000 and 43500

hz in mono. In stereo the true rate is one half of the programmed

rate, so the maximum rate in stereo is around 22000 hz. Currently only

stereo playback is allowed under SBOS2, basically because I dont know

how stereo recording works!  The SBPRO has an onboard mixer which

allows the levels of numerous inputs and outputs to be software

controlled. It also provides user controllable filters for input and

output.

THE SBOS2 DEVICE DRIVER

-----------------------

The SBOS2 device driver implements up to two new devices under OS/2. If

loaded with the SBREG option the only device created is SBDSP$, whereas

on the SBPRO the additional device SBMIX$ is created. The device SBDSP$

is used to write and read data from the D/A and A/D ports, whereas SBMIX$

is used to program the onboard mixer of the SBPRO. An application can

test for the presence of these devices by attempting to open them with

DosOpen(). If SBDSP$ exists but SBMIX$ does not then one can assume that

the sound board is a SBREG. Currently SBOS2 does not auto-detect the board

present in the computer - it relies upon the user supply the board type

by using the SBREG or PAS16 options.

Only one program can access SBDSP$ at one time. In addition, only one

program under OS/2 should access the DSP part of the SB at any time,

which includes DOS and Windows applications. I've heard that MMPM/2

and SBOS2 can coexist peacefully if both are not running simultaneously,

ie. running a task that uses SBDSP$ while an MMPM/2 task is running.

A version of SBOS2 which includes a VDD to virtualized the access to

the SB for DOS and Windows apps is in the works. Until this is

released be sure to stress to the users of the software that the above

conflicts should be avoided.

The device SBDSP$ does several things when it receives an Open

request.  It first checks to see if it is already open, and if so it

refuses the pending open request. If not it allocates 192k of buffer

space using the AllocPhys DevHelp routine. This allows two 64k chunks

of memory to be found which do not cross a 64k boundary. This makes

the DMA transfers simpler to program. It also tries to install an

interrupt handler for the SB IRQ. If successful it then proceeds to

initialize driver specific data and returns. When SBDSP$ is closed it

releases the 192k of ram and the SB IRQ, so that a task other than

SBOS2 can grab the IRQ.

The device SBMIX$ can be accessed by several programs simultaneously,

although this isnt necessarily desirable. A program does not receive

notification if another task changes the mixer levels, so it is not

safe to assume that the mixer levels are the same as you last set

them.  If a task desires to have exclusive control of SBMIX$ it can

call DosOpen() with the ACCESS_DENYREADWRITE option, which will

prevent any further tasks from opening SBMIX$ (I have not tested

this). The device SBMIX$ only receives Open, Close and IOCtl commands,

you cannot read or write to SBMIX$.

THE SBDSP$ PROGRAMMING INTERFACE

--------------------------------

There are numerous sample programs provided in this archive which

demonstrate how to open and close SBDSP$ and perform various IOCtl

commands. I will cover these topics in detail in this section.

First one should be familiar with the include file sblast_user.h.  It

is required by any program using SBOS2. It defines numerous constants

for the use of SBOS2. These constants should be used instead of hardcoding

numbers. Though it is unlikely these constants will change, it is best

to be safe than sorry.

OPENING AND CLOSING SBDSP$

----------------------------

To open SBDSP$ use the DosOpen() service. The following section of code

does this

------------------------

ULONG status, action;

HFILE audio_handle;

status = DosOpen( "SBDSP$", &audio_handle, &action, 0,

FILE_NORMAL, FILE_OPEN,

OPEN_ACCESS_READWRITE | OPEN_SHARE_DENYNONE | OPEN_FLAGS_WRITE_THROUGH

| OPEN_FLAGS_NOINHERIT | OPEN_FLAGS_NO_CACHE,

NULL );

if (status != 0)

perror ("Error opening audio device");

--------------------

Note all of the flags used to open the device. The primary ones

required are the ones starting with OPEN_FLAGS. The flags

OPEN_ACCESS_READWRITE and OPEN_SHARE_DENYNONE seem to imply more than

one task can access SBDSP$ and perhaps should be different, although

SBDSP$ will deny an open request if it is already open. The variable

action indicates how DosOpen() satisfied the open request. If status

is non-zero then there was an error opening SBDSP$, which means

another task is using SBDSP$ or the device driver SBOS2.SYS is not

loaded. The variable audio_handle recieves the handle to be used by

subsequent i/o commands for SBDSP$.

Once a program is finished with SBDSP$ it should close it by using

DosClose(). SBDSP$ checks to see if any data is presently in the

DMA buffers for output and if so flushes the buffers.

READING FROM SBDSP$

--------------------

To sample data from the the SB's A/D port you use the DosRead()

command, as follows:

status = DosRead(audio_handle, &buf, maxreadsize, &numread);

here

audio_handle  is the handle returned from the DosOpen command above

buf           is a pointer (unsigned char) to the storage buffer

maxreadsize   is the number of bytes requested to be read

numread       gives the actual number of bytes read.

Use of this command is given in OS2RECORD.C.

Once a read command is received by SBDSP$, it continuously fills its

internal DMA buffers. If an application does not empty these buffers

fast enough they will fill up, meaning that data is going to be lost.

The default behavior of SBDSP$ is to create an error, since ususally

you dont want to lose data while sampling. Currently the error

handling is poor, as the user usually has to kill the offending task.

There is a IOCtl command to change this default behavior and have

SBDSP$ ignore the buffer overflow. This is useful is one is trying to

write a real-time task which is not concerned if data is lost, like an

oscilloscope-style display. This IOCtl is covered below.

WRITING TO SBDSP$

------------------

To send output using the SB's D/A interface, use the following command

status = DosWrite(audio_handle, &buf, numtowrite, &numwritten);

here audio_handle and buf are the same as in the example above. The

variable numtowrite gives the number of bytes to write from the buffer

buf[], and numwritten returns the actual number of bytes written. The

file OS2PLAY.C demonstrates how to playback recorded sounds. Unlike

the read command, if you fail to supply data to SBDSP$ as fast as it

plays it, no error results. It just waits till you send more data.

SBDSP$ does not actually output data until the internal DMA buffer is

filled. By default the DMA buffer size is 64k-2 bytes, so at a rate of

8000 hz it will take 8 seconds of output before the buffer is played.

Usually you can send 8 seconds worth of data to SBDSP$ in a fraction

of a second, so this isnt a problem. But keep this in mind when

writing software.

To send stereo data to a SBPRO just alternate left/right samples.

IOCtl commands

--------------

IOCtl stands for I/O Control.  The SB has several configurable parameters

which are accessable using the DosDevIOCtl() command. For example, to

set the playback/sample speed of the SB use the following

datlen=2;

parlen=0;

status=DosDevIOCtl(ahandle, DSP_CAT, DSP_IOCTL_SPEED,

NULL, 0, &parlen,

&speed, datlen, &datlen);

The parameters DSP_CAT and DSP_IOCTL_SPEED are defined the include file

"sblast_user.h". Each IOCtl command has a number associated with it.

Also

speed       is an unsigned short int (16bit) containing the desired speed

datlen      is an unsigned long  int (32bit) containing sizeof(speed)

parlen      is an unsigned long  int (32bit), and should ALWAYS be 0.

There are several other IOCtl commands available. The following documents

each availble to the device SBDSP$

DSP_IOCTL_SPEED    - sets speed

Expects a 2 byte unsigned int containing speed (datlen=2)

The max speed for the SBPRO is given by DSP_MAX_SPEED_PRO, and

for the SBREG it is DSP_MAX_SPEED_REG, whereas the min speed

for both is DSP_MIN_SPEED.

Usually the card cant be set for the exact speed requested,

but is set to something close. The actual speed used is

returned in the variable which contained the desired speed.

If you send a value of 0xFFFF as the speed the current speed

is returned.

DSP_IOCTL_RESET    - resets card

Expects no data (datlen=0)

DSP_IOCTL_VOICE    - turns output speaker on or off

Expects a unsigned char. Zero means off, non-zero means on (datlen=1)

DSP_IOCTL_FLUSH    - flushes output buffer

Expects no data (datlen=0)

DSP_IOCTL_COMPRESS - controls hardware compression

Expects a unsigned char. Defined values given in sblast_user.h

DSP_IOCTL_STEREO   - enables/disables stereo mode on SB Pro only.

Expects an unsigned char. Zero means off, non-zero means on (datlen=1)

DSP_IOCTL_BUFSIZE - set the current DMA buffer size.

Expects an unsigned short int. Min legal value is DSP_MIN_BUFSIZE,

and max value is DSP_MAX_BUFSIZE (def'd in sblast_user.h).

This can be useful command, but it has the potential for

causing	problems.  By default the DMA buffer size is

DSP_MAX_BUFSIZE, which is 64k-2 bytes.  There are real-time

applications where a much smaller buffer size is desirable.

Sending a value of 0xFFFF returns the current buffer size.

The way this command can cause problems is if you change the

buffer size while the buffers are being used. No check is

currently made by SBDSP$. It is best to close and reopen SBDSP$ and

set the new buffer size, just to make sure everything is reset before

changing the buffer size.

DSP_IOCTL_OVERRUN_ERRNO - Set behavior to buffer overflow

Expects a short int.

When sampling it is possible for the internal buffer of

SBDSP$ to fill, resulting in lost data. By default the

SBDSP$ mixer returns an error and usually the application

has to be closed.

By sending a value of 0 this causes SBDSP$ to ignore

overflows. In the case of an overflow it stops sampling until

the user empties the current buffer.

By sending a non-zero value this causes SBDSP$ to return an

error if the buffer overflows. Currently the behavior is

unpredictable afterwards, and it is best to end the application.

The most useful of the above are DSP_IOCTL_RESET, DSP_IOCTL_STEREO and

DSP_IOCTIL_SPEED. The commands DSP_IOCTL_BUFSIZE and

DSP_IOCTL_OVERRUN_ERRNO can be used for real-time applications. Look

at the source for PMOSCOPE for an example. Here the buffer size is

made equal to the number of samples to be displayed at a time on the

screen. The overflow error is set to zero so that if we don't draw as

fast as data comes in we just skip to a current sample.

Some more notes on programing SBDSP$ are in order. Currently the

program assumes that the entire DMA buffer is to be played at a single

speed. So if you set the sample rate to one speed and send data, the

change the speed and send more data, causing the DMA buffer to be

played, the entire buffer will be played at the later speed. A

workaround is to flush the buffer before sending the next sample, if

it is at a different speed. I've had a report this doesnt currently

work so i'm looking into it. This same comment applies to stereo

playback, ie. it is assumed the entire buffer is to be played either

in stereo or mono.

In general, it is not a good idea to change the parameters controlled

by the DSP_IOCTL commands once you start output or input. If your

application needs to do so (for example a program which plays several

VOC files) it is probably a good idea to close and reopen SBDSP$ each

time parameters need to be changed.

MIXER PROGRAMMING

-----------------

The device SBMIX$ is opened and closed just like SBDSP$, except you

specify SBMIX$ as the filename instead of SBDSP$. You dont, however,

read or write to SBMIX$. It only accepts IOCtl commands. In addition,

SBMIX$ only works on the SBPRO, so if you are writing software that uses

SBMIX$ be sure to let the user know it wont work on a regular SB.

Unlike SBDSP$, multiple applications can open SBMIX$. This means that

another application can change the mixer settings without your

application receiving notification.  I'm not sure how to handle this

behavior, since I don't know how to tell the device driver to notify

an application of a change. Probably the best thing to do is to read

the current settings before making a change. Another thing one could

do is to open SBMIX$ with attributes which cause it to not be shared.

The IOCtl commands for the mixer use two data structures, defined in

sblast_user.h, sb_mixer_levels and sb_mixer_params. The following

list gives all the commands available.

Commands using sb_mixer_levels

------------------------------

These commands require datlen=sizeof(struct sb_mixer_levels). Check the

file sblast_user.h for legal values of the fields of the sb_mixer_levels

structure. Note that this structure changed starting with v0.91 of the

driver, with the CD INPUT becoming a stereo mixer instead of a mono mixer.

MIXER_IOCTL_SET_LEVELS

MIXER_IOCTL_READ_LEVELS

These commands write and read the mixer levels. See the programs

setmixer.c or mixertool.c for examples of their use.

Commands using sb_mixer_params

------------------------------

These commands require datlen=sizeof(struct sb_mixer_params). Check the

file sblast_user.h for legal values of the fields of the sb_mixer_params

structure.

MIXER_IOCTL_READ_PARAMS

MIXER_IOCTL_SET_PARAMS

These commands write and read the mixer parameters. See the program

mixertool.c for examples of their use.

Other commands

--------------

MIXER_IOCTL_RESET

Resets the mixer. I dont recommend using this, but rather you

should reset the mixer by sending what you consider default

levels and parameters if you need to 'reset' the mixer.

CONCLUSIONS

------------

The SBOS2 package is still in development, which means the information

in this document may not apply to future releases of the driver. I

believe the data structures are fixed, and the IOCtl command names

will stay the same. It is best to use the constants defined in

sblast_user.h instead of hardcoding numbers, in case I need to change

the numbers.

There are still some serious problems with the driver which are only

discovered when someone tries something new I never thought of. If you

uncover a problem please let me know, i want to fix it. When reporting

a problem it is helpful to know the type of system you have, namely is

it a fast 486 or not. Call me paranoid, but it seems that most SB

problems come up on fast machines. Then let me know in a few sentences

what you were trying to do and the exact response of the system (error

messages, sounds, ect). Please don't send source code unless I ask.

And if you write something you think other people would appreciate

then send it to me. I am starting a directory for contributed

software.  I am also interested in starting a mailing list for

programming related topics, sorta like the sound blaster mailing list.

I think only people who have read this document would know about the

list, so we could keep the conversation related to programming. Send

me email if you're interested, and if you have every managed a mailing

list. I have a machine I can set it up on, i'm just not sure about

software.

I can be reached at

michael fulbright

msf@as.arizona.edu