Date: Fri, 1 May 1998 04:29:23 -0400 (EDT)

From: Paul Anderson <madhobby@geeky1.ebtech.net>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Doorknob (was Re: How I set my safety gaps)

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

On Thu, 30 Apr 1998, Gene Waldo wrote:

>

> Bill,

> What the hell is a doorknob?  I've seen this term bandied about, and

> figured, sit back, Waldo, it'll come to you; well, it hasn't!

>

The term doorknob refers to a doorknob capacitor.  They're a

commercially-made, cheapo (circa $12/each surplus) HV capacitor.  The

reason they're called doorknob caps is that the best way to describe them

is that they like a doorknob.  Except usually smaller diameter.  TTYL!

---

Paul Anderson

madhobby@geeky1.ebtech.net

From: <Tom.Stock@gsbsc.gensig.com>

Date: Fri, 1 May 1998 10:11:03 -0400

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: T2- Spec the best?

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

Yeah I had the opportunity to get a pole pig from my father who used

to work for the power company. The only problem is that they don't

have anything larger than the standard 120V-240V transformers.

Bummer. At least I can get oil.

-tom

______________________________ Reply Separator _________________________________

Subject: T2- Spec the best?

Author:  "Tesla-2 Mailing List" <tesla-2@emachine.com> at Internet2

Date:    04/30/98 10:04 PM

All, Mike, et al:

Seems one of my taberna amigos is a lifelong commercial electrician; his

son is in NEON, in Chicago or some other place.. we chat....  I mention

transformers and like that there... he says, "What do you need?"

Guess what.... I tell him!  Then he asks really some trouble-making

questions: (kinda: how big?) We were talking NPS, and then I drifted

into 'PolePigs', and then it really fell apart! He wanted specs! Like

I'm supposed to know?!

As for NPS: I guessed at 15kV, 120 ma; ??? what's good? it'll cost me

shipping.

As for PolePigs, ... 'IF' we ever get to that point (120vac avail.

only??!), what do we want? His buddies in the poleyards need to know!

I'm two or three scotches into this and this guy is banging on my

shoulder, telling me how dumb I am!

--

Gene Waldo -  gwaldo@1stnetusa.com  - "Igor in Residence"

---------------------------------------------------------------

From: "Bill the arcstarter" <arcstarter@hotmail.com>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: T2- Spec the best?

Date: Fri, 01 May 1998 08:15:46 PDT

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

It was written:

>     Yeah I had the opportunity to get a pole pig from my father who used

>     to work for the power company. The only problem is that they don't

>     have anything larger than the standard 120V-240V transformers.

Yes - but what was the *input* voltage?

The power company uses pigs to convert from a higher voltage (13200 is

typical) down to 120-240.  We (coilers) run them in reverse - converting

either 120/240 back into 13200, etc.

Note: In case anyone forgot - pigs are NOT current-limited and are

extremely dangerous!  Your first mistake may very well be your last

mistake.  Please - be exceedingly careful!

I would think that a 5KVA pig would be sufficient for all but the

largest coils. The rated KVA is for continuous duty - I suspect you can

run a significant overload for short periods - at least until the pig

starts to heat up.   I'm told the power companies are phasing the

smaller sizes <=10KVA) out - so you might be able to get one cheap...

Dont' get one unless it is marked as being Non-PCB filled...

Our pig is huge - a 25KVA unit which converts either 120/240 up to 13200

or so - there is an adjustable tap which controls the exact conversion

ratio.  Weight is 485 lbs.  A smaller one would have been nice but this

was all we could get...

-Bill the arcstarter

Starting arcs in Cinci, OH

http://www.geocities.com/CapeCanaveral/Hangar/6160

From: "Alfred A. Skrocki" <alfred.skrocki@cybernetworking.com>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Date: Fri, 1 May 1998 02:24:56 +0500

Subject: Re: T2- Spec the best?

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

On Thu, 30 Apr 1998 22:04:43 -0700 Gene Waldo

<gwaldo@1stnetusa.com> wrote;

> All, Mike, et al:

> Seems one of my taberna amigos is a lifelong commercial electrician; his

> son is in NEON, in Chicago or some other place.. we chat....  I mention

> transformers and like that there... he says, "What do you need?"

> Guess what.... I tell him!  Then he asks really some trouble-making

> questions: (kinda: how big?) We were talking NPS, and then I drifted

> into 'PolePigs', and then it really fell apart! He wanted specs! Like

> I'm supposed to know?!

> As for NPS: I guessed at 15kV, 120 ma; ??? what's good? it'll cost me

> shipping.

Gene, if your going to get anything you might as well strive for the

biggest! I've seen 15Kv 300 ma neons on occasion or you might try and talk

him into getting you a bombarder, their used in making neon signs to

bombard the tubes inside with electrons untill the glass heats up and

de-gasses, their almost the same as a small (5 killowatt) pole pig! Even

better actually since the come already equiped with a current control.

Sincerely

Alfred A. Skrocki

Alfred.Skrocki.Sr@JUNO.com

alfred.skrocki@cybernetworking.com

X-Originating-IP: [199.217.140.22]

From: "Bill the arcstarter" <arcstarter@hotmail.com>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: T2: Variac as a current limiter?

Date: Fri, 01 May 1998 10:46:53 PDT

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

Coilers,

An old engineer mentioned how he used to use a variac as a variable

inductor.  Basically you only use two terminals of the variac - the

slider lug and the common lug.

Has anyone used one this way as a variable current-limiter for a

polepig etc??

I can't see why this wouldn't work.  Comments??   I'll measure the

inductance range tonight.

I just purchased a DMM-20  DVM (Digital Volt Meter) from Alfa

Electronics. Reads caps, inductances, frequency and duty cycle (plus

volts/amps/ohms).  Pretty cool - should make many coiling tasks easier.

Net $90 delivered to my door in under 48 hrs!  If you don't have a DVM -

this would be a good choice.  This is my third DVM (may DVM #1 rest in

peace...)

http://www.alfaelectronics.com

-Bill the arcstarter

Starting arcs in Cinci, OH

http://www.geocities.com/CapeCanaveral/Hangar/6160

From: "Thomas McGahee" <tom_mcgahee@sigmais.com>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Variac as a current limiter?

Date: Fri, 1 May 1998 14:43:31 -0400

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

----------

> From: Bill the arcstarter <arcstarter@hotmail.com>

> To: Tesla-2 Mailing List <tesla-2@emachine.com>

> Subject: T2: Variac as a current limiter?

> Date: Friday, May 01, 1998 1:46 PM

>

> Coilers,

>  An old engineer mentioned how he used to use a variac as a variable

> inductor.  Basically you only use two terminals of the variac - the

> slider lug and the common lug.

>

>  Has anyone used one this way as a variable current-limiter for a

> polepig etc??

>

You have to saw through the core. Simply remove a "slice" of the

core about the same width as your hacksaw blade. Wedge a piece of

insulating material in there so the gap remains open, and you

is in business.

>  I can't see why this wouldn't work.  Comments??   I'll measure the

> inductance range tonight.

>

>  I just purchased a DMM-20  DVM (Digital Volt Meter) from Alfa

> Electronics. Reads caps, inductances, frequency and duty cycle (plus

> volts/amps/ohms).  Pretty cool - should make many coiling tasks easier.

> Net $90 delivered to my door in under 48 hrs!  If you don't have a DVM -

> this would be a good choice.  This is my third DVM (may DVM #1 rest in

> peace...)

>

Alfa Electronics has an excellent line of reasonably priced

test equipment, including DMMs, LCRs, and frequency meters.

Fr. Tom McGahee

> http://www.alfaelectronics.com

>

> -Bill the arcstarter

> Starting arcs in Cinci, OH

> http://www.geocities.com/CapeCanaveral/Hangar/6160

>

From: "Alfred A. Skrocki" <alfred.skrocki@cybernetworking.com>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Date: Fri, 1 May 1998 20:57:38 +0500

Subject: T2 Re: Variac as a current limiter?

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

OnFri, 1 May 1998 14:43:31 -0400 In Tesla-2

Thomas McGahee <tom_mcgahee@sigmais.com> wrote;

> > From: Bill the arcstarter <arcstarter@hotmail.com>

> > To: Tesla-2 Mailing List <tesla-2@emachine.com>

> > Subject: T2: Variac as a current limiter?

> > Date: Friday, May 01, 1998 1:46 PM

> >

> > Coilers,

> >  An old engineer mentioned how he used to use a variac as a variable

> > inductor.  Basically you only use two terminals of the variac - the

> > slider lug and the common lug.

> >

> >  Has anyone used one this way as a variable current-limiter for a

> > polepig etc??

> >

>

> You have to saw through the core. Simply remove a "slice" of the

> core about the same width as your hacksaw blade. Wedge a piece of

> insulating material in there so the gap remains open, and you

> is in business.

Tom, it isn't necessary to go through all that trouble! If you look in J.H.

Couture's book "Tesla Coil Design Manual" (mine is dated 1992, I don't know

if there's a newer edition) in section 7 - Line filters and Reactors, on

page 7-2 he has an illustration of variac connections and C. Two variac -

Current limiting illustrates exactly what Bill was describing. Below is my

feeble attempt to illustrate in an ASCII drawing.

VARIAC 1        VARIAC 2

---------- )             )

)             )

)             )

)             )

U2          )             )<------> T

T2          )             )         O

I0          )<------------)

LV          )                       T

IO          )                       E

TL          )                       S

YT          )                       L

)

)                       C

)                       O

)                       I

----------)---------------------> L

Variac 1 is being used for Voltage control and Variac 2 is being used for

current control.

Sincerely

Alfred A. Skrocki

Alfred.Skrocki.Sr@JUNO.com

alfred.skrocki@cybernetworking.com

X-Mailer: IPAD 2.0

Date: Fri, 01 May 1998 11:48:55 -0500

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: T2- Spec the best?

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

At 10:04 PM 4/30/98 -0700, you wrote:

>All, Mike, et al:

>Seems one of my taberna amigos is a lifelong commercial electrician; his

>son is in NEON, in Chicago or some other place.. we chat....  I mention

>transformers and like that there... he says, "What do you need?"

>Guess what.... I tell him!  Then he asks really some trouble-making

>questions: (kinda: how big?) We were talking NPS, and then I drifted

>into 'PolePigs', and then it really fell apart! He wanted specs! Like

>I'm supposed to know?!

>As for NPS: I guessed at 15kV, 120 ma; ??? what's good? it'll cost me

>shipping.

Gene,

My feeling is that you would be better served with a

pair of 60ma neons rather than a 120. 120s are pretty rare.

(at least around here) The two 60s give you more flexibility.

The 60s are much more rugged than 30s.

I also favor 12kv neons rather than 15s. The extra voltage doesnt

mean much performance wise but your caps will surely feel the strain.

Ive never had any lack of power running 12kv.

>As for PolePigs, ... 'IF' we ever get to that point (120vac avail.

>only??!), what do we want? His buddies in the poleyards need to know!

>I'm two or three scotches into this and this guy is banging on my

>shoulder, telling me how dumb I am!

For a pole pig you want a dual bushing transformer with a

14400 primary. 5KVA is a good size as the weight is manageable

and will easily run 7 or 8KVA. A 10KVA unit is probably more

common and works as well but weighs considerably more.

These will most commonly have a 240 VAC secondary.

Pigs come in a variety of primary voltages. 14400 and 7200

are the most common around here. 13200 is also a common voltage.

BUT a pig needs to have external current limiting.

If you short the output of a pig (like in a spark gap)

it will try to draw huge amounts of current. So

you need extensive external control circuitry. A large

30 amp minimum variac is a must. Inductive and resistive

ballasting is necessary to limit current.

Pole pigs are supremely dangerous. They can easily remove

limbs. While it is possible to get tangled up in a neon

and live to tell about it that wont happen with a pig.

One mistake and your dead.

Live long and prosper

TTYL!

Mike Hammer

mhammer@misslink.net

From: "Chris Watkins" <cwatkins@seidata.com>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: T2:  variac as current limiter

Date: Fri, 1 May 1998 13:31:15 -0400

X-Mailer: IPAD 2.0

X-MimeOLE: Produced By Microsoft MimeOLE V4.72.2106.4

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Bill I have recently been thinking about  using a variac as

a current limiter.

I beleive  it will work but havent actually tried yet.

Chris W.

X-Mailer: IPAD 2.0

Date: Fri, 01 May 1998 14:15:43 -0500

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: T2: Variac as a current limiter?

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

At 10:46 AM 5/1/98 PDT, you wrote:

>Coilers,

> An old engineer mentioned how he used to use a variac as a variable

>inductor.  Basically you only use two terminals of the variac - the

>slider lug and the common lug.

>

> Has anyone used one this way as a variable current-limiter for a

>polepig etc??

>

> I can't see why this wouldn't work.  Comments??   I'll measure the

>inductance range tonight.

Yes this has been done although Ive never done it myself.

The variac needs to be modified somewhat. The core is cut

to introduce an air gap. This serves to prevent saturation.

From what I understand the range of adjustment is not

linear. Most of the range gets crowded toward one end

making adjustment difficult.

I just dont think I could stand cutting the core of a large

(expensive) variac. A welder is so much more satisfactory

and has the side benefit of doing double duty as a welder!

But if you have a suitable variac to spare it wont hurt to experiment

a little.

>

> I just purchased a DMM-20  DVM (Digital Volt Meter) from Alfa

>Electronics. Reads caps, inductances, frequency and duty cycle (plus

>volts/amps/ohms).  Pretty cool - should make many coiling tasks easier.

>Net $90 delivered to my door in under 48 hrs!  If you don't have a DVM -

>this would be a good choice.  This is my third DVM (may DVM #1 rest in

>peace...)

>

Beware of the readings of inexpensive DVMs. Try them out on known

capacitances and inductors before trusting the readings.

They can also give quite bizarre AC voltage readings sometimes.

Some inexpensive meters do quite well. My Radio Shack DVM

compares quite favorably to my FLUKE LCR meter. Check it

before you trust it.

My meter of choice on the job is my trusty old

analog Tripplett 630 VOM. I would trust it more than

a stack of DVMs.

TTYL!

Mike Hammer

mhammer@misslink.net

From: ESchulz531 <ESchulz531@aol.com>

Date: Fri, 1 May 1998 17:11:39 EDT

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: T2: Variac as a current limiter? (about DVM)

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

I think I bought the same model of DVM from them too, works great.

It measured a precision 0.01 uf cap at 0.0103 uf.

I can live with 3% error. ��)

Later,

Erik

From: "Basura, Brian" <brian.basura@unistudios.com>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: RE: T2: Variac as a current limiter? (about DVM)

Date: Fri, 1 May 1998 15:07:28 -0700

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

I have one on order from them also...  Glad to hear it works good...

Thanks,

Brian D. Basura

> ----------

> From: 	ESchulz531[SMTP:ESchulz531@aol.com]

> Reply To: 	Tesla-2 Mailing List

> Sent: 	Friday, May 01, 1998 2:11 PM

> To: 	Tesla-2 Mailing List

> Subject: 	Re: T2: Variac as a current limiter? (about DVM)

>

> I think I bought the same model of DVM from them too, works great.

> It measured a precision 0.01 uf cap at 0.0103 uf.

>

> I can live with 3% error. ��)

>

> Later,

> Erik

>

>

From: ESchulz531 <ESchulz531@aol.com>

Date: Fri, 1 May 1998 18:12:04 EDT

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: SRe: T2: Variac as a current limiter? (about DVM)

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Sorry I was wrong, it is a TENMA 72-6255

It measured a precision 0.01 uf cap at 10.03 NF.

I can live with 0.3% error.    ��)

Later,

Erik

X-Mailer: IPAD 2.0

Date: Fri, 01 May 1998 18:19:42 -0700

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: MIKE D <gordrew@home.com>

Subject: spark gap

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

I plan on building a gap where i have several lenghs of copper pipe

vacuum quenched. But i can't seem to find any formulas or "general rules" that

would tell me how many or the spacing between the pipes. Are there any?

Thanks,

--MIke Drew

Sender: tesl-err@emachine.com

Date: Fri, 01 May 1998 21:37:18 -0400

From: Steve Falco <sfalco@worldnet.att.net>

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: spark gap

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

MIKE D wrote:

>

>       I plan on building a gap where i have severl lenghs of copper pipe

> vacuum quenched. But i can't seem to find any formulas or "general rules" that

> would tell me how many or the spacing between the pipes. Are there any?

>

> Thanks,

> --MIke Drew

This depends on the voltage and current of your transformer.  For a 12

KV 30 mA neon, I found that 5 to 7 gaps set to about .020" to .025"

worked well.

Steve Falco

X-Mailer: IPAD 2.0

Date: Fri, 01 May 1998 20:48:27 -0500

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: spark gap

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

At 06:19 PM 5/1/98 -0700, you wrote:

>

>

>	I plan on building a gap where i have severl lenghs of copper pipe vacume

>quenched. But i can't seem to find any formulas or "general rules" that

>would tell me how many or the spacing between the pipes. Are there any?

>

>Thanks,

>--MIke Drew

Only some general rules of thumb. It really depends

on your transformer voltage. Higher voltage needs wider gap.

For 12kv 9 elecrodes spaced .03 inches apart will yield

8 series gaps with a total spacing of .240 inch. That

is all the gap you will ever need for 12kv. Consider it

a maximum value. My own gap for 12kv use has 9 series

gaps (10 electrodes) spaced .025 inch for a total of .225 inch.

For 15kv increase the number of electrodes to 11 spaced .03 inch.

This gives a maximum of .300 with 10 series gaps.

If building a vacuum gap make sure the airflow is directly into

the gap area between the electrodes. That gives best performance.

You will probably find that your vacuum cleaner pulls too much

air through the gap. I run mine on a variac to set the speed

for best output.

TTYL!

Mike Hammer

mhammer@misslink.net

X-Mailer: IPAD 2.0

Date: Fri, 01 May 1998 20:20:30 -0700

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: MIKE D <gordrew@home.com>

Subject: cap box

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

I was looking around my garage and found a styrofoam ice chest that would

be perfect for my cap box but was curious as to the weaknesses of the

styrofoam (which makes me think i can't use it) but thought they may be a

chance..

Thanks,

MIke Drew

X-Mailer: IPAD 2.0

Date: Fri, 01 May 1998 23:15:50 -0500

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: cap box

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

At 08:20 PM 5/1/98 -0700, you wrote:

>

>

>	I was looking around my garage and found a styrofoam ice chest that would

>be perfect for my cap box but was cureous as to the weaknesses of the

>styrafoam (which makes me think i can't use it) but thought they may be a

>chance..

>

>Thanks,

>MIke Drew

Mike,

I dont think styrofoam would be what you want.

I can see that material dissolving in transformer oil.

At the very least it will leak due to the porous nature

of the foam.

What I did for my latest plate cap was to build the cap

first. Then I built the tank to fit from plywood. Coat the

interior of the plywood box with two heavy coats of epoxy

or polyester resin. I used polyester resin.

I made a lid from plexiglass held on by small screws.

I also used the same method to build the tank for

my rebuilt 12kv/100ma neon. No leaks.

TTYL!

Mike Hammer

mhammer@misslink.net

X-Mailer: IPAD 2.0

Date: Sat, 02 May 1998 00:45:13 -0400

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Moleking <moleking@concentric.net>

Subject: Re: cap box

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

>

>	I was looking around my garage and found a styrofoam ice chest that would

>be perfect for my cap box but was cureous as to the weaknesses of the

>styrafoam (which makes me think i can't use it) but thought they may be a

>chance..

>

When I was assembling my rolled poly caps, I kept having trouble with the

oil leaking, so I "temporarily" put about two gallons of oil in a fish tank

and put them in there.  They've been there for a good couple of months now.

It's impractical and not very portable, but it works.  Good luck.

Josh Bedell

From: Hollmike <Hollmike@aol.com>

Date: Sat, 2 May 1998 08:39:51 EDT

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: cap box

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Mike,

I would not use styrofoam as a container for mineral oil or any other type

of oil.  They tend to leak and that would be a big mess for you.  It would

probably be ok if you are housing a dry capacitor.  I haven't seen what you

plan as a cap.  I would use a polypropylene tub such a Tupperware.

Hope this helps.

Mike Hollingsworth.

From: AlanM45157 <AlanM45157@aol.com>

Date: Sat, 2 May 1998 12:40:54 EDT

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Help!

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

OK - I've visited a lot of web sites, and read some newgroup listings.  I tend

to be a "hands-on/learn-by-doing" type (left brain/right brain?) and am

itching to get started.  So, how do I get started?

Thanks,

Alan McCulloch

X-Mailer: IPAD 2.0

Date: Sat, 02 May 1998 12:17:28 -0500

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: Help!

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

At 12:40 PM 5/2/98 EDT, you wrote:

>OK - I've visited a lot of web sites, and read some newgroup listings.  I

tend

>to be a "hands-on/learn-by-doing" type (left brain/right brain?) and am

>itching to get started.  So, how do I get started?

>

>Thanks,

>

>Alan McCulloch

Alan,

First thing you need is a neon sign transformer. You really

need to get this first as the rest of the coil is designed

around it. Check out any sign shops in your area. Consult

the yellow pages. Look for 9 or 12kv 30ma neons to start

with.

Once you have your transformer you can build a capacitor

appropriate for it.

Next item to locate is some AWG 24 magnet wire. You will need

2-3 pounds of it. Get plenty so you dont run out. Again consult

the yellow pages and look under electric motors for a motor

repair/rebuilding shop. These places will often have the best

prices on wire.

You can also begin thinking about your secondary coil

size and shop around for some suitable PVC pipe. The

size of the secondary will be somewhat determined by

the size of your neon transformer. For instance if all

you can find is a small 6kv/20ma neon you will want to build

a smaller coil. One of the most flexible secondary coils

can be wound on 4 inch PVC pipe. It will work well at a

range of different power levels.

You will also need some 3/4 inch hard copper pipe for a

spark gap as well as a muffin fan.

For the primary coil you will need some 1/4 inch copper

tubing or heavy solid wire. I use solid AWG 6 for my primary.

If you havn't already done so download and read through the

archives of this list. Many of your questions have already

been asked and answered on the list. Get them from Dave

Bronakowski's well organized archive site. The URL is:

http://www.velocity.net/~djb/tesla.shtml

Once you locate a neon transformer you can begin to

work on the specifics of your coil.

TTYL!

Mike Hammer

mhammer@misslink.net

Date: Sat, 2 May 1998 10:19:31 -0700 (PDT)

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Dan Murphy <danmurphy@1stnetusa.com>

Subject: Re: T-2 Help!

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

At 12:40 PM 5/2/98 EDT, you wrote:

>OK - I've visited a lot of web sites, and read some newgroup listings.  I tend

>to be a "hands-on/learn-by-doing" type (left brain/right brain?) and am

>itching to get started.  So, how do I get started?

>

>Thanks,

>

>Alan McCulloch

Alan -

I don't know about "hands-on/learn-by-doing" as far as a Tesla Coil goes, but:

Decide on your coil design, i.e. resonant freq, inductance, etc (see archives)

Get some wire - Formvar Magnet wire, AWG 22 or 24

You need a coil form - 4" PVC is most common - 24" to 26" in height

Prep the PVC by drying and sealing it (See archives/list)

Wind a secondary Coil -  1000 turns of wire, plus or minus. (around 21")

seal the winding with polyurethane varnish, or shellac (see archives for more)

decide on primary coil design, mounting details.

wind primary (usually 1/4" copper tubing)

power supply - neon sign transformers are most common

match power supply to capacitance, then build a capacitor (see archives)

build a static spark gap (see archives)

construct or obtain a pair of RF chokes  (see archives)

build yourself a safety gap to protect your transformer. (see archives)

hook all this stuff up (again - see archives) and throw the switch!

Nothing to it. ;-)

=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=

MadManDanMadManDanMadManDanMadManDanMadManDanMadManDanMadManDan

--- Fine: a tax for doing wrong.  Tax: a fine for doing right  ---

<danmurphy@1stnetusa.com>

=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=

Date: Sat, 02 May 1998 13:24:21 -0700

From: mike nicholas <705728@ican.net>

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Help!

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

First of all:

1: how much do you want to spend?

2:determine the size of coil you want to build

3:do the math, twice

4:get a life insurance policy with me being the beneficiary ;)

5:determine capacitor style

6:determine primary style

7:decide on gap style

8:length of wire needed for desired secondary winding length

9: aquire parts

10: ask more questions then

AlanM45157 wrote:

> OK - I've visited a lot of web sites, and read some newgroup listings.  I tend

> to be a "hands-on/learn-by-doing" type (left brain/right brain?) and am

> itching to get started.  So, how do I get started?

>

> Thanks,

>

> Alan McCulloch

Sender: tesl-err@emachine.com

From: "Fred Decker" <fdecker@csi.com>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: How I set my safety gaps

Date: Sat, 2 May 1998 17:13:16 -0400

X-Mailer: IPAD 2.0

X-MimeOLE: Produced By Microsoft MimeOLE V4.72.2106.4

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

A "doorknob" is a type of capacitor.  This is what commercial HV caps are called.  They don't really look like doorknobs, they are usually thick cylinders with tapped metal contacts on the ends.

-----Original Message-----

From: Gene Waldo <gwaldo@1stnetusa.com>

To: Tesla-2 Mailing List <tesla-2@emachine.com>

Date: Thursday, April 30, 1998 11:45 PM

Subject: Re: How I set my safety gaps

>Bill the arcstarter wrote:

>>

>> Coilers,

>>  I've not seen any instructions on how to set your safety gaps - but for

>> what it's worth - here is how I do it:

>>

>> You need:

>> *Safety gap

>> *Neon/pig/MOT or whatever

>> *Variac

>> *Nothing else

>>

>> My safety gap is really nasty looking - made out of a couple of steel

>> hardware store "L" brackets with a nut brazed onto one with an eye-screw

>> through the nut - slovenly glued onto an old plastic IC carrier tube!

>> (Dan's/Gene's stuff on mhammer's page is really slick -

>>

>> Example - placing a 500pf doorknob across my neon caused the safety gap to fire...

>> -Bill the arcstarter

>> Starting arcs in Cinci, OH

>> http://www.geocities.com/CapeCanaveral/Hangar/6160

>

>Bill,

>What the hell is a doorknob?  I've seen this term bandied about, and

>figured, sit back, Waldo, it'll come to you; well, it hasn't!

>

>THANKS for the strokes! ~~<:))))  New stuff on our (Mike's) page...

>--

>Gene Waldo -  gwaldo@1stnetusa.com  - "Igor in Residence"

From: BPTesla <BPTesla@aol.com>

Date: Sat, 2 May 1998 17:41:46 EDT

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: helpful hint for rolled poly cap.

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

I have been fighting with my .005 rolled poly cap for some time now. I finally

had a slow day at work, so I had my co-worker help me roll attempt #3. As

usual, it wouldn't fit into the 4" PVC. I left it rolled and taped for another

try. Instead of re- rolling the cap, I carefully inserted 2, 3/8 Dia.. Metal

rods into the wide, inner folds of the poly. The rods were long enough to

extend well beyond each end of the PVC. I then placed 2 small hose clamps over

the rods (1 on each end) and tightened the rods together. Holding the rolled

cap in one hand, I inserted a screwdriver between the two rod and began

twisting, thus tightening the cap. I slid everything very easily into the 4"

PVC, removed the hose clamps and easily slid the rods out.

Bill pierce

Date: Sat, 02 May 1998 16:49:29 -0500

From: Mark Olson <molson1@isd.net>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: disruptive and solid-state comparisons

Sender: tesl-err@emachine.com

Well, I finally did it - with knowledge gained from this list I have

built a coil that i've fired using either disruptive tank circuit or

bottom-fed with a solid-state driver.  The comparison is interesting,

although I haven't even gotten close to what the disruptive circuit

should be capable of.

Drivers:

Disruptive:

2 15kv 60ma neons/chokes/safety gap

.03mfd pulse cap/ safety gap

cylindrical static gap (built from directions gotten from this list)

Primary:   16 turns 1/4" copper tubing; flat spiral

Secondary:  10.75" PVC w/ 29" 22 gauge magnet wire

Toroid:  Plastic wire spool (about 14" diameter and 3" high) covered with

aluminum foil

Since the cap/primary is designed for 15kv/180ma I haven't gotten anywhere near

the discharge length possible.  So far about 48", w/purple streamers - the tank

circuit is made of light gauge wire (it needs to be a LOT heavier).

Interesting difference between 1 neon and 2 - the output more than doubled, not

really a surprise?  ( I wonder what I'll get when I get the final neon??  :)

)

Solid state driver:

4 MOSFET 1/2 bridge design, 340 volt DC supply, run in pulsed mode with

about 500 watts continuous input, though much higher pulsed output.

Primary:  9 turns heavy Litz wire (got it for a song surplus) on a Phillips EC70

ferrite E-core transformer, 200 turns #30 teflon wire secondary

Same secondary coil as above, 10.75"/29" #22 magnet wire, same toroid

produces very bright yellow/white streamers about 19-20" in length, and sparks

about the same length that are a brilliant white.  Very hot discharge, streamers burn

wood in seconds, puncture glass very quickly (poor flourescent tube), and generally

erode the aluminum foil that I used for the toroid.  A light bulb held in the hand

anywhere with a 4' radius gets too hot to hold  - very quickly.  The difference between

the streamers and sparks isn't really very much, although the streamers are thicker and

more "forked" if that description means anything...

It's interesting that, although the disruptive driver generates far longer sparks,

they are obviously not anywhere near as hot as the ones given by the solid-state

driver. Can I expect the same once I get the tank circuit wired and tuned properly?

Oh, and the disruptive coil makes  a LOT more noise!  :)

Thanks for  all the information!  I would have not even have attempted

this project without the knowledge that people here have passed along.

I'm really looking forward to getting this coil running at the level I

think it's capable of!

Mark Olson

X-Mailer: IPAD 2.0

Date: Sat, 02 May 1998 19:58:47 -0400

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Jim Ryan <jryan@exis.net>

Subject: Re: helpful hint for rolled poly cap.

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

At 05:41 PM 5/2/98 EDT, you wrote:

>I have been fighting with my .005 rolled poly cap for some time now. I finally

>had a slow day at work, so I had my co-worker help me roll attempt #3. As

>usual, it wouldn't fit into the 4" PVC. I left it rolled and taped for another

>try. Instead of re- rolling the cap, I carefully inserted 2, 3/8 Dia.. Metal

>rods into the wide, inner folds of the poly. The rods were long enough to

>extend well beyond each end of the PVC. I then placed 2 small hose clamps over

>the rods (1 on each end) and tightened the rods together. Holding the rolled

>cap in one hand, I inserted a screwdriver between the two rod and began

>twisting, thus tightening the cap. I slid everything very easily into the 4"

>PVC, removed the hose clamps and easily slid the rods out.

>

>Bill pierce

>

>Bill, What did you use for the conductor/foil ? I see where

some people use aluminum foil but I can't see that as strong

enough to mount terminals on. I' thinking about buying some

shim stock.

Jim

Sender: tesl-err@emachine.com

Date: Sat, 02 May 1998 20:25:02 -0400

From: Steve Falco <sfalco@worldnet.att.net>

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: helpful hint for rolled poly cap.

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

> >Bill, What did you use for the conductor/foil ? I see where

> some people use aluminum foil but I can't see that as strong

> enough to mount terminals on. I' thinking about buying some

> shim stock.

> Jim

For rolled caps, I used aluminum flashing, which is available in any

home center (like Home Depot or Lowes).  It comes in various widths and

is strong enough to drill and put a 1/4" bolt through for attaching your

wires.

Steve Falco

From: BPTesla <BPTesla@aol.com>

Date: Sat, 2 May 1998 20:25:18 EDT

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re:  Re: helpful hint for rolled poly cap.

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

In a message dated 5/3/98 12:05:51 AM, you wrote:

<<>Bill, What did you use for the conductor/foil ? I see where

some people use aluminum foil but I can't see that as strong

enough to mount terminals on. I' thinking about buying some

shim stock.

Jim

>>

Jim,

The foil often referred to is not household "Reynolds wrap". It is called

aluminum flashing and is available at any hardware store in the roofing

section. It is sold in various widths and lengths. I got a 10"x25' foot roll

for about 8 bucks. I think it is always 10mil thick, much more practical for

rolled caps.

For the best step by step guide on rolled poly caps, go to

http://home.earthlink.net/~gweaver.

Good luck

Bill

X-Mailer: IPAD 2.0

Date: Sat, 02 May 1998 20:41:19 -0500

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: helpful hint for rolled poly cap.

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

At 08:25 PM 5/2/98 -0400, you wrote:

>> >Bill, What did you use for the conductor/foil ? I see where

>> some people use aluminum foil but I can't see that as strong

>> enough to mount terminals on. I' thinking about buying some

>> shim stock.

>> Jim

>

>For rolled caps, I used aluminum flashing, which is available in any

>home center (like Home Depot or Lowes).  It comes in various widths and

>is strong enough to drill and put a 1/4" bolt through for attaching your

>wires.

>

>	Steve Falco

Ive used both flashing and foil in both plate caps and rolled

caps. My conclusion is that foil is the way to go. Sure its flimsy

but with a little care makes a more compact cap. And its lots less

expensive than flashing.

Use the heavy duty grade of foil as its thicker and stronger

than the normal variety. For my rolled caps the inner foil

is wrapped tightly around a 1/2 inch copper pipe as a central

hub. The pipe provides a low loss connection to the inner foil.

A heavy copper wire is soldered to the pipe to connect to the

output terminal of the cap.

For the outer foil I fold over a strip of foil many times to

make a thick strap about 1 inch wide. This strap is rolled

up tightly in the free end of the outer foil and then taped

down along the outside of the cap roll. The strap runs up

along the side of the cap then up to the other output terminal.

For flat plate caps I cut all my plates with a 1 inch wide

extended tab. The tabs act as the buss connections. All

like tabs are bunched together and bolted to connecting

straps with small brass bolts, nuts and washers.

TTYL!

Mike Hammer

mhammer@misslink.net

From: Hollmike <Hollmike@aol.com>

Date: Sun, 3 May 1998 14:26:31 EDT

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: disruptive and solid-state comparisons

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Mark,

I would be interested in seeing the schematic of you solid state driver.

If it is not too much trouble, could you e-mail it to me at Hollmike@aol.com?

Thanks

Mike Hollingsworth

Date: Sun, 3 May 1998 15:22:40 -0400 (EDT)

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: fxphoto@centuryinter.net (fxphoto)

Subject: Thank You Mike Hammer!!! and All!

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Mike,

I think that it is time for me to stop sucking up all this knowledge and

send you and Ralph a big THANK YOU for this list and the effort you two put

into it. I built my first coil in 1969 and have built a few since. My best

on my own was an 8" spark using an OBIT. I have been on this list for maybe

a month or two.

Last night I achieved a twenty inch spark (18" continuous to ground) and I

am still a long way from getting full output from this setup. I was somewhat

puzzled as the night before I was only able to achieve 18" with 16" continuous

with identical settings. Then I realised it was raining outside and it must

have been giving me a better ground. (maybe my ground could use a little

improvement!)

The coil specs are: 4x16 alum dryer duct toroid, 4.25 dia , 21" tall #24

AWG epoxy coated secondary, 24 turns .25" refrigeration (100 ') tubing 30deg

inverted cone primary, RQ type gap 0.030 gaps (two layers of alum flashing),

R.S. 1.2mH chokes in series with R.S. 100uH chokes and a 0.011 uf 25,000

volt (mica or ceramic (mylar wrapped, epoxy dipped)) cap.

The ground is an 8' galvanized ground rod with #4 AWG solid copper running

approx 30' to the attic (my dungeon in the sky).

I am using R.S. mega cable #12 Awg (Radio Shack sells it for sound systems

:it is extremely flexible) copper wire from the cap to the primary

adjustment point. I filed down a small battery clamp with a 1/4 dia file for

max contact area to the 1/4 tubing primary. I am using 2" alum tape to

connect the gap to the primary and the gap to the cap. When the tuning is

closer I will replace this with alum flashing.

A huge Thanks goes out to all the contributers on this list. There are way

too many to name individually, but each one of you deserves thanks for every

hint, tip or bit of information you have posted. It ALL Helps!!!

Bill Turbett

aka Fxphoto and Bill_T

quote: "I may be stupid, but at least I can repeat my misteaks accurately!!"

Date: Sun, 3 May 1998 15:26:51 -0400 (EDT)

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: fxphoto@centuryinter.net (fxphoto)

Subject: Re: disruptive and solid-state comparisons

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Mark,

I would also be interested in the schematic if it is available.

fxphoto@centuryinter.net

Thanks

Bill Turbett

>Mark,

>   I would be interested in seeing the schematic of you solid state driver.

>If it is not too much trouble, could you e-mail it to me at Hollmike@aol.com?

> Thanks

>Mike Hollingsworth

>

>

Date: Mon, 04 May 1998 00:30:56 -0700

From: mike nicholas <705728@ican.net>

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: t2 choke cores

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

I need a source of choke cores in Ontario ?

What size should I look for?

From: <Tom.Stock@gsbsc.gensig.com>

Date: Mon, 4 May 1998 11:24:52 -0400

Subject: Other sources for ferrite cores?

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

Ok, I've tried everywhere. I went to a ham fest Sunday and had no luck

in finding any ferrite donuts whatsoever.

Does anyone know of any scrounge-o-sources for these things? You know,

TV's, Microwaves, etc?

I also didn't find any 30 watt resistors for running in series with an

air core choke.

-tom

tom.stock@gsbsc.gensig.com

From: <Tom.Stock@gsbsc.gensig.com>

Date: Mon, 4 May 1998 11:31:44 -0400

Subject: Question about caps and primary tuning

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

A couple of questions:

How may turns are most people using with two 15kv/30ma power supplies

and about .01 mF, on a flat primary with a 4.5" secondary?

I'm tapped out at 12 turns, with .005 mF with a single 15kv neon and a

12" toroid discharge terminal.  If I add more capacitance can I reduce

the number of turns required on the primary? Will I get much better

results if I just add more turns? I think 16 is the highest I can go

without completey rebuilding my primary table.

I can reduce the number of turns necessary by using a smaller

discharge terminal, but then I get too much corona around the upper

end of the coil.

thanks

-tom

Date: Mon, 04 May 1998 09:12:50 -0700

From: Gene Waldo <gwaldo@1stnetusa.com>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Organization: Self

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Other sources for ferrite cores?

Sender: tesl-err@emachine.com

Tom.Stock@gsbsc.gensig.com wrote:

>

>  Ok, I've tried everywhere. I went to a ham fest Sunday and had no luck

>      in finding any ferrite donuts whatsoever.

>      Does anyone know of any scrounge-o-sources for these things? You know,

>      TV's, Microwaves, etc?

>      I also didn't find any 30 watt resistors for running in series with an

>      air core choke.

>      -tom

>      tom.stock@gsbsc.gensig.com

Re: More junkyard rummaging:

Tom, et al,

I'm not certain about ferrite cores, but I'm 99.9% on choke-series R's...

Pending a new value concensus, I'll 'corner the market' on 300 ohm, 50W

jobs....(or something close).

I'll also inquire about cores.

Question: what about winding a closed circuit Cu doughnut, on which to

wind the choke? No?  Yes: specs?

No EE,

---------------------------------------------------------

Gene Waldo -  gwaldo@1stnetusa.com  - "Igor in Residence"

---------------------------------------------------------------

What's good for the planet is more OZONE! See the machinery form up!

NEW STUFF!   http://www.misslink.net/mhammer/waldo.htm

From: "Chris Watkins" <cwatkins@seidata.com>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: 15/30 franceformer

Date: Mon, 4 May 1998 12:54:34 -0400

X-Mailer: IPAD 2.0

X-MimeOLE: Produced By Microsoft MimeOLE V4.72.2106.4

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Hi coilers

I unpotted a 15/30 Franceformer successfully and It works!

I am now ready to start on a 15/60 .

question is how many ma. can I get out of the 15/30 by removing

shunts before it burns up?  has anyone tried this?

What is the best way to test the ma output?

I shorted the secondaries together and measured the primary current.

only 3.5A at 120v with 2 shims removed from each shunt.

That figures up to 28 ma..

this thing sure is noisy after the tar is removed...

It looks like the secondaries are wound with #24 ga wire.

shouldnt this handle considerably more than 30 ma. ?

I realise the primary will have more load also.

any info , ideas or criticsm  welcome

thanks;

Chris W

cwatkins@seidata.com

Date: Mon, 4 May 1998 12:53:55 -0600

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Daniel Lockhart <molly@cmn.net>

Subject: High Frequency Coils

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

Hey all,

I'm kinda new to Telsa coils and everything, so this may be a dumb

question, I don't know.

Is it possible to make a coil that operates at a relatively high frequency?

Say, 500 kHz? I know smaller coils run higher, but is it possible to make a

medium to large coil at a frequency of around this?

I don't know what the advantage would be -- just longer sparks and maybe a

bit safer it you were to come into contact with it.

Just a thought,

Daniel Lockhart

X-Originating-IP: [199.217.140.22]

From: "Bill the arcstarter" <arcstarter@hotmail.com>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: T2 disruptive and solid-state comparisons

Date: Mon, 04 May 1998 11:54:07 PDT

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

It was written:

>From: Mark Olson <molson1@isd.net>

>Solid state driver:

>

>   4 MOSFET 1/2 bridge design, 340 volt DC supply, run in pulsed mode with

>about 500 watts continuous input, though much higher pulsed output.

I'm not sure just what this arrangement is - are your fets arranged in

an H-bridge format?

Do you use a feedback coil on your transformer or is the frequency

controlled open-loop via a 555 or some other oscillator?

Are you using the avalanche-rated MOSFETS by Intl. Rectifier?  If not -

what kind of suppression are you using?

>Primary:  9 turns heavy Litz wire (got it for a song surplus) on a

>Phillips EC70

>ferrite E-core transformer, 200 turns #30 teflon wire secondary

Did you buy this core for this purpose or did you use what was available

in the junk box? :)

How do you keep the core from saturating?

Is using Litz wire *really* useful at tesla coil frequencies? (Not a

flame - I just wanna know!)

>about the same length that are a brilliant white.  Very hot discharge,

>streamers burn wood in seconds, puncture glass very quickly

>(poor flourescent tube), and generally erode the aluminum foil that I

>used for the toroid.  A light bulb held in the hand anywhere with a 4'

>radius gets too hot to hold  - very quickly.  The difference between

>the streamers and sparks isn't really very much, although the streamers

>are thicker and more "forked" if that description means anything...

>

>

>It's interesting that, although the disruptive driver generates far

>longer sparks, they are obviously not anywhere near as hot as the ones

>given by the solid-state driver. Can I expect the same once I get the tank

>circuit wired and tuned properly?

Neat! Did you observe any tendency for either the solid state or

disruptive driver causing more strikes from the secondary to the primary

than the other?

I have no idea if you can expect the same output from your disruptive vs

a continuous driver- given the same average input power.  If I had to

guess - I would think that the disruptive would yield larger sparks

since it is running in more of a 60/120 hz pulsed-mode.  But that is

just my opinion.

Your observations regarding forked vs smooth streamers seem to match

what I've read before.  I'm not certain - but you might be doing quite

well with your solid state driver - most CW coilers are using vacuum

tubes since they are a bit more forgiving than fets, etc...

Check the tesla web ring - someone (or maybe more than just one) has a

site relating to continuous-wave vacuum tube drivers, etc...

>Oh, and the disruptive coil makes  a LOT more noise!  :)

Try running a rotary gap - even LOUDER!  Like cutting a big piece of

sheet metal on a table saw using a wood blade... :)

Keep us (or at least ME) informed of your work!

-Bill the arcstarter

Starting arcs in Cinci, OH

http://www.geocities.com/CapeCanaveral/Hangar/6160

Date: Mon, 4 May 1998 13:19:14 -0600

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Daniel Lockhart <molly@cmn.net>

Subject: Coil Tuning and more Spark Gap Questions

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

Hey all,

I have everything ready and all set for my first coil -- all except the

spark gap. I'm planning on making a RQ type cylinder gap with seven

electrodes spaced at 0.03 inches for my 9 kV, 30 ma transformer.

The problem is that I just don't seem to like this setup. With all the

copper pipes glued into the pvc pipe, each electrode is not moveable. From

what I can tell, an awful lot of getting your coil working just right is

tweaking and adjusting everything. It just seems like you should be able to

change the distance between the electrodes. Is this important?

After I get everything working okay, I am planning on making either a poly

plate or rolled cap. Same thing here. If you make a nice permanent

capacitor, you can't tweak and adjust it. Do you that beforehand with

saltwater caps? Will you use the same capacitance with poly caps as you

used with saltwater caps?

Is everything a little more concrete than I relise, or am I just being picky?

Anyway, back to spark gaps. Is seven electrodes (six gaps) spaced at 0.03

inches (total arc displacement 0.18 inches) okay for a 9/30 transformer?

More? Less? Sound okay?

Thanks,

Daniel Lockhart

X-Originating-IP: [199.217.140.22]

From: "Bill the arcstarter" <arcstarter@hotmail.com>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Coil Tuning and more Spark Gap Questions

Date: Mon, 04 May 1998 13:00:21 PDT

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

It was written:

>I have everything ready and all set for my first coil -- all except the

>spark gap. I'm planing on making a RQ type cylinder gap with seven

>electrodes spaced at 0.03 inches for my 9 kV, 30 ma transformer.

>

>The problem is that I just don't seem to like this setup. With all the

>copper pipes glued into the pvc pipe, each electrode is not moveable.

>From what I can tell, an awful lot of getting your coil working just

>right is tweaking and adjusting everything. It just seems like you should

>be able to change the distance between the electrodes. Is this important?

If you make a traditional RQ gap - you will have access to all of the

mounting bolts.  You can pick your gap length (in integer multiples of

one gap) by choosing which bolts you wire your primary onto.

For rough tuning you can use an alligator clip to attach to different

pipes, etc.

Other than choosing the number of gaps to use, I don't think anyone

really fools with more precise adjustments of the gap distance.  Anyone

want to to speak up on this???

Certainly for your initial tuning, you won't want to use all 9 gaps!

I'd suggest starting at two gaps (pipe #1 and #3) and increasing in

small steps as long as nothing BAD is happening (ie - strikes from the

primary to seconday, safety gap going berserk, excessive smoke, etc)

Remember - it is the gap setting which determines the peak primary

voltage (and thus - energy) before gap flashover.  The variac adjustment

really doesn't control this. It just lets you ramp up to that point

somewhat more smoothly.

>After I get everything working okay, I am planning on making either a poly

>plate or rolled cap. Same thing here. If you make a nice permanent

>capacitor, you can't tweak and adjust it. Do you that beforehand with

>saltwater caps? Will you use the same capacitance with poly caps as you

>used with saltwater caps?

Yes - unless you decided that you had the wrong amount of salt water caps! :)

The final fine tuning is (traditionally) done by adjusting the primary

inductor tap point.

When we first fired up our coil, we had too much tank capacitance,

requiring a primary tap at turn #3 out of 15.  By doing some

series/parallel combinations of our tank caps, we reduced the net

capacitance and were able to tune the coil at tap #13.  The advantage is

that your adjustments are finer (percentage wise), if you are moving a

few inches across one turn out of 13, rather than out of 3 turns!

On my new gap I brazed the bolt heads to the copper pipes to ensure a

good contact.  Arcing from the pipes to the bolts lead to the thermal

demise of unit #1...

-Bill the arcstarter

Starting arcs in Cinci, OH

http://www.geocities.com/CapeCanaveral/Hangar/6160

X-Mailer: IPAD 2.0

Date: Mon, 04 May 1998 15:15:58 -0500

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: 15/30 franceformer

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

At 12:54 PM 5/4/98 -0400, you wrote:

>    Hi coilers  I unpotted a 15/30 franceformer successfuly and

>   It works!

Great Chris. Welcome to the elite neon rebuilders club.

>I am now ready to start on a 15/60  . question is how many ma. can I get

>out of the  15/30 by removing shunts before it burns up?

>has anyone tried this?

Yep done it many times. 30 ma neons will easily go up

to 40-45 ma without much problem. Some people have pushed

them as high as 50ma but thats pushing things a bit. You can

begin to run into core saturation problems as well as damaging

one of the fragile secondary windings. Only take 3 or 4 plates

from each shunt maximum.

>What is the best way to test the ma output?

With an AC rated milliameter or DVM. Set your meter to read

AC milliamps and connect it right across the HV output then

turn on the power. Yep thats right. Place it right across

the output. The short circuit that the meter represents sends

the neon into limiting and the voltage will remain rather low.

The meter will register the short circuit current of the neon.

NOTE. This only goes for magnetically shunted transformers

like neons or OBITs. Dont try it on other transformers like

MW oven transformers or you will likely smoke your meter.

>I shorted the secondarys together and measured

>the primary  current. only 3.5A at 120v with 2 shims removed from each

>shunt.   That figures up to 28 ma..

The primary current does not correlate well in neons

due to power factor and the rather low efficiency.

> this thing sure is noisy after the tar is  removed...

Did you get all the laminations back in? Did you remember to replace

the wedges that go between the windings and the core to hold things

solidly in place? In most France units these wedges are wood but

in some like Jeffersons they are plastic. Whittle some new wedges

from a scrap of wood and force them between the windings and core.

That should quiet things some.

>It looks like the secondarys are wound with #24  ga

>wire. shouldnt this handle  considerably more than 30 ma. ? I realise the

>primary will have more load also.

Not in a 30ma neon. In most France 30s the secondary windings are AWG 32.

One Jefferson 30 I unpotted was AWG 34. Most of the 60 ma neons Ive

done have been AWG 30.

You may find that the primary of your France 30ma is aluminum wire.

It may look like copper on the outside because of the insulation

but scrape the insulation off and you'll probably find its aluminum.

TTYL!

Mike Hammer

mhammer@misslink.net

X-Mailer: IPAD 2.0

Date: Mon, 04 May 1998 15:44:23 -0500

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: Coil Tuning and more Spark Gap Questions

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

At 01:19 PM 5/4/98 -0600, you wrote:

>Hey all,

>

>I have everything ready and all set for my first coil -- all except the

>spark gap. I'm planing on making a RQ type cylinder gap with seven

>electrodes spaced at 0.03 inches for my 9 kV, 30 ma transformer.

>

>The problem is that I just don't seem to like this setup. With all the

>copper pipes glued into the pvc pipe, each electrode is not moveable. From

>what I can tell, an awful lot of getting your coil working just right is

>tweaking and adjusting everything. It just seems like you should be able to

>change the distance between the electrodes. Is this important?

You change the amount of gap by using more or less of the total

number of electrodes. You use the maximum amount of gap by connecting

to the first and last electrode. You can can select fewer electrodes.

Start by connecting to the first and third electrode. This will

provide 2 gaps at .03 or .06 total. This will keep the power level

low since this small gap will break down at rather low voltage.

Use this setting to check everything out and get your system

in approximate tune. Then include one extra electrode in the

gap at a time tweaking for best tune as you go.

>

>After I get everything working okay, I am planning on making either a poly

>plate or rolled cap. Same thing here. If you make a nice permanent

>capacitor, you can't tweak and adjust it. Do you that beforehand with

>saltwater caps? Will you use the same capacitance with poly caps as you

>used with saltwater caps?

Normally you size your cap to make the best use of your transformer.

For a 9kv/30ma neon this is around .0088 mfd.

All "tweaking" then is done with the primary coil.

>

>Is everything a little more concrete than I realize, or am I

>just being picky?

A little picky but thats OK. Once you get your coil running

things will make a lot more sense.

>

>Anyway, back to spark gaps. Is seven electrodes (six gaps) spaced at 0.03

>inches (total arc displacement 0.18 inches) okay for a 9/30 transformer?

>More? Less? Sound okay?

Sounds just fine Daniel.

TTYL!

Mike Hammer

mhammer@misslink.net

From: "Chris Watkins" <cwatkins@seidata.com>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: 15/30 franceformer

Date: Mon, 4 May 1998 16:48:18 -0400

X-Mailer: IPAD 2.0

X-MimeOLE: Produced By Microsoft MimeOLE V4.72.2106.4

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Thanks Mike

for the valuable info.(again)

the franceformer does have aluminum wire in the primary

and I do need to make a couple wedges!

Chris W

X-Mailer: IPAD 2.0

Date: Mon, 04 May 1998 15:50:19 -0500

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: High Frequency Coils

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

At 12:53 PM 5/4/98 -0600, you wrote:

>Hey all,

>

>I'm kinda new to Telsa coils and everything, so this may be a dumb

>question, I don't know.

>

>Is it possible to make a coil that operates at a relatively high frequency?

>Say, 500 kHz? I know smaller coils run higher, but is it possible to make a

>medium to large coil at a frequency of around this?

>

>I don't know what the advantage would be -- just longer sparks and maybe a

>bit safer it you were to come into contact with it.

>

>Just a thought,

>

>Daniel Lockhart

Daniel,

You could probably build a coil for just about any frequency.

Practically speaking most coils run from 100-500 KHZ.

Some quite large coils may run below 100khz and small ones

higher than 500khz. I once made a small coil that ran at nearly

1 mhz. Higher frequencies give a much different type of output.

Gone is the jagged forked lightning. Instead you get a softer

brush type of output. At higher frequencies it becomes almost

flame like.

For long sparks low frequencies are better. The lower the frequency

the lower the losses and better the efficiency will be.

TTYL!

Mike Hammer

mhammer@misslink.net

X-Originating-IP: [199.217.140.22]

From: "Bill the arcstarter" <arcstarter@hotmail.com>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: High Frequency Coils

Date: Mon, 04 May 1998 14:12:44 PDT

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

It was written:

>Is it possible to make a coil that operates at a relatively high frequency?

>Say, 500 kHz? I know smaller coils run higher, but is it possible to make a

>medium to large coil at a frequency of around this?

Well - if you space-wind a coil that will reduce the inductance and

increase the self-resonant frequency.

(By "space-wind"  I mean winding your secondary with a gap between

adjacent turns.  Usually you have to use a lathe, a toothed-comb or some

other jiggery-pokery to accomplish this...)

However I think your "Q" will suffer since inductance is proportional

the the square of the number of turns while the resistance is

proportional to the number of turns, etc...  High "Q" is good for coils!

I have no idea if a physically large high frequency coil would work

better or worse than a conventional design.

-Bill the arcstarter

Starting arcs in Cinci, OH

http://www.geocities.com/CapeCanaveral/Hangar/6160

Sender: tesl-err@emachine.com

Date: Mon, 04 May 1998 20:16:28 -0400

From: Steve Falco <sfalco@worldnet.att.net>

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Coil Tuning and more Spark Gap Questions

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

> On my new gap I brazed the bolt heads to the copper pipes to ensure a

> good contact.  Arcing from the pipes to the bolts lead to the thermal

> demise of unit #1...

>

> -Bill the arcstarter

> Starting arcs in Cinci, OH

> http://www.geocities.com/CapeCanaveral/Hangar/6160

>

I built an RQ gap, but I used nuts and plumbers solder to secure the

brass bolts to the copper tubing.  The nuts do force the copper tubing

closer to the center of the PVC, and this will not let you put as many

pieces of copper tubing in a 6" PVC pipe, but there were enough sections

for my purposes.

I filed oblong holes to get the adjustment range, and did *not* use

epoxy, so I could remove the tubing for easier cleaning and adjustment.

I recognize that this defeats the purpose of the "never needs

adjustment" concept, but I'm happier with it my way.  (Actually, I

started with epoxy initially, but found that it didn't adhere well to

the PVC (even with sanding), so I cleaned it up and started over

without.)

Steve Falco

X-Mailer: IPAD 2.0

Date: Mon, 04 May 1998 21:29:15 -0500

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: remove me from e-mail list

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

At 05:58 PM 5/4/98 -0700, you wrote:

>Hi,

>I need to be removed from your e-mailing list for the time being.

>Thank you.

>Mike.

>

I posted this a few days ago and evidently not everyone read it.

So here it comes a again. This is the new procedure for unsubcribing

or suspending your subscription.

The procedure for unsubscribing from the list has changed so everyone

should make note of it for future reference. The new software allows

you to temporarily suspend your subscription for vacations and similar

circumstances.

To quit the list, either permanently or temporarily, such as for vacation,

please send a message to tesla-2-request@emachine.com with one of the

following keywords in the body of the message:  Delete, Remove or

Unsubscribe.  Case is not important.

If you should later wish to rejoin the list, please send a message to

tesla-2-request@emachine.com with one of the following keywords in the

body of the message:  Join, Add or Subscribe.  Case is not important.

One additional note. If you unsubscribe and then try to resubscribe

yourself later on it must be with the same email address. If

you change addresses then the list server sees you as a new subscriber.

New subs still need to go through me.

If you need to change your address email me with the change

rather than unsubscribing.

TTYL!

Mike Hammer

mhammer@misslink.net

X-Mailer: IPAD 2.0

Date: Tue, 05 May 1998 00:50:15 -0400

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Jim Ryan <jryan@exis.net>

Subject: A couple of ?'s

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

I see in some illustrations for a primary of the coil that there is a

"Strike Ring". What is it and what is it attached to ?

Ref. www.hills2.u-net.com.tesla/mytesla.htm     page 2.

What are the specifications for air core inductors length, diameter.

#turns, wire size, to protect a 12K 30 ma. trans driving a 4.5" X 21"

secondary, the ferrite cores are to hard to find.

Thanks

Jim

X-Mailer: IPAD 2.0

Date: Mon, 04 May 1998 23:13:13 -0700

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: bob golding <yubba@clara.net>

Subject: Re: Coil Tuning and more Spark Gap Questions

Cc: tesla-2@emachine.com

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

At 13:19 04/05/98 -0600, you wrote:

snip

>I have everything ready and all set for my first coil -- all except the

>spark gap. I'm planing on making a RQ type cylinder gap with seven

>electrodes spaced at 0.03 inches for my 9 kV, 30 ma transformer.

daniel

I have made a RQ type gap and followed the instructions exactly. I found

it a messy process so have lots of meths handy for cleaning the epoxy off

everything. The only problem I came across when making my gap was making

the holes in the pvc pipe to small. I used 1/4" brass bolts and made the

holes in the pvc 5/16" I found this was not enough clearance if you are

drilling the holes by hand. go for 3/8" to give yourself plenty of leeway

for drilling errors. You can always use bigger washers and plenty of epoxy.

I havent tried my gap yet but it looks like it should work. If I was making

a second one I think I would follow "bill the arc starters" advice and

braze the bolts onto the copper first. Hope this helps.

bob golding lightning without rain

bob yubba@clara.net

X-Mailer: IPAD 2.0

Date: Tue, 05 May 1998 01:04:15 -0700

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: bob golding <yubba@clara.net>

Subject: Re: Undeliverable msgs

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

At 08:51 04/05/98 -0700, you wrote:

>Gene Waldo wrote:

>>

>> Gene Waldo wrote:

>> >

>> > Suddenly I have "server will not relay ..." type stuff hanging my

>> > 'outbox' up...    to an e-mail addr which I have used before...

gene:

i am having the same problem!! "aol mailbox full".  my message to the list

was returned twice and as i can now read it in my mail in box i assume

everybody else can. I am really impressed with your chokes and safety gap

and will try to copy them as this is the next stage of my first coil. I too

dont like to bodge thinks but would rather spend time making it look good

to start with as good workmanship means it's got more chance of working

first time, and if it doesn't you can always polish it. keep up the good work.

bob yubba@clara.net

X-Mailer: IPAD 2.0

Date: Tue, 05 May 1998 01:26:51 -0700

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: bob golding <yubba@clara.net>

Subject: mail

Cc: tesla-2@emachine.com

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

hi gene

i am having the same problems. This is my third attempt to try and tell you!!

I get the mailbox full error message as well. No doubt it's another

interesting "feature" which will be upgraded in the next incarnation

without anybody knowing. I really like your safety gap and have the slaves

copying every detail as we speak. The chokes are far out too. Are you using

any resistors in series with them? hope this gets to you maybe in

triplicate if your lucky.

bob yubba@clara.net

From: Hollmike <Hollmike@aol.com>

Date: Tue, 5 May 1998 05:01:25 EDT

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: T2 disruptive and solid-state comparisons

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Mark,

I am not sure if my last letter went through, so I am writing one to be

sure.  I would like to see the schematic for you solid state driver circuit.

I may be interested in trying one myself.

Thanks

Mike Hollingsworth

From: "Thomas McGahee" <tom_mcgahee@sigmais.com>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Coil Tuning and more Spark Gap Questions

Date: Tue, 5 May 1998 08:05:34 -0400

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

----------

> From: Bill the arcstarter <arcstarter@hotmail.com>

> To: Tesla-2 Mailing List <tesla-2@emachine.com>

> Subject: Re: Coil Tuning and more Spark Gap Questions

> Date: Monday, May 04, 1998 4:00 PM

>

SNIP

>

> Remember - it is the gap setting which determines the peak primary

> voltage (and thus - energy) before gap flashover.  The variac adjustment

> really doesn't control this. It just lets you ramp up to that point

> somewhat more smoothly.

>

Bill is correct, once the firing voltage is reached, increasing the

variac does not increase the peak voltage across the capacitor.

The variac *does*, however, control the total energy being delivered

to the system. If you have your variac at say 50% and your gap is

firing, then increasing the variac setting will *NOT* increase the max

voltage across the cap... BUT, it *will* increase how often per

second the cap can charge up to this voltage. Thus the rate of energy

transfer is increased.

Rotary gaps allow even more control in this area. Maximum energy transfer

will occur at certain BPS settings, and is phase angle dependent as well,

meaning that sometimes you will have a setting where the phase angle is

slipping or changing, and the output of the Tesla Coil will grow and

diminish accordingly. Synchronous rotary gaps allow control over the

phase angle such that it remains fairly 'solid', and the output of the

Tesla coil is more consistent. Interestingly, it is often the

inconsistency of the energy delivery system that gives rise to some

of the more spectacular individual strikes.

Hope this helps.

Fr. Tom McGahee

Date: Tue, 05 May 1998 09:15:26 -0700

From: Gene Waldo <gwaldo@1stnetusa.com>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Organization: Self

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: A couple of ?'s

Sender: tesl-err@emachine.com

Jim Ryan wrote:

>

> I see in some illustrations for a primary of the coil that ther is a

> "Strike Ring". What is it and what is it attached to ?

> Ref. www.hills2.u-net.com.tesla/mytesla.htm     page 2.

>

> Thanks

Jim,

For whatever value, I have a Strike Ring in my circuit diag. at our site

below. It serves to protect the Pri and Pri drive Tank components from a

hit from the 2ndary terminal. The ring should be placed just above the

the Pri coil, to attract and ground such strikes.

---------------------------------------------------------

Gene Waldo -  gwaldo@1stnetusa.com  - "Igor in Residence"

---------------------------------------------------------------

What's good for the planet is more OZONE! See the machinery form up!

NEW STUFF!   http://www.misslink.net/mhammer/waldo.htm

X-Mailer: IPAD 2.0

Date: Tue, 05 May 1998 12:39:51 -0500

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: A couple of ?'s

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

At 12:50 AM 5/5/98 -0400, you wrote:

>I see in some illustrations for a primary of the coil that ther is a

>"Strike Ring". What is it and what is it attached to ?

>Ref. www.hills2.u-net.com.tesla/mytesla.htm     page 2.

A strike ring is a grounded ring placed just outside and above

your last primary turn. Its function is to attract and conduct

safely to ground any streamers that want to strike down into

your primary circuit. Streamer hits into the primary can cause

capacitor and transformer failures.

Some coils have a tendency to hit into the primary while

others do not. Saucer shaped primary are often the worst

in this regard. Flat spirals are better because of the greater

distance between the toroid and primary. Solenoid primary coils

often will not have this problem as they are in close to the secondary.

My coil with a tapered solenoid primary has never had any primary

strikes so I never added a strike rail.

*NOTE*

The strike rail should NOT be in the form of a closed loop or

ring. It must be open. One end only of the ring should be grounded

and the other end left free. A closed loop will look like a shorted

turn to your coil and suck up lots of power.

>

>What are the specifications for air core inductors length, diameter.

>#turns, wire size, to protect a 12K 30 ma. trans driving a 4.5" X 21"

>secondary, the ferrite cores are to hard to find.

>

New ferrite and iron powder toroids and rods may be purchased

from Amidon Corp. Their web page is: http://bytemark.com/amidon/

A couple of people have purchased some surplus toroids recently

from Electronic Goldmine. Problem with surplus is here today

gone tomorrow. Plus sometimes you never really know what you

are going to get. Check out Electronic Goldmine at:

http://www.goldmine-elec.com/

The requirements for a RF choke are pretty loose.

Anything from 2-10 mh will work fine. You are not building

a filter. Just something to knock down the RF some.

Try from 350-400 turns of AWG 26 on a piece of 2 inch PVC

pipe. This will result in a winding 6-7 inches long with

and inductance of around 2.5 mh.

To calculate the inductance of your chokes use

Wheeler's formula.

L(uH) = (r^2) * (N^2) / (9*r + 10*h)

where:

r = coil RADIUS in inches

N = number of turns

h = coil height in inches

If using air core chokes its a good idea to include series

damping resistors. Put one in each HV line in between your

neons and choke. Anything from 300-1500 ohms and 10 or

more watts will be fine. If you run higher power you

may need higher power resistors.

I acquired some 310 ohm 50 watt resistors at a hamfest for

10 cents apiece. I included a pair of these in my rebuild neon

right inside the tank of oil. They are a permanent part of the

transformer.

TTYL!

Mike Hammer

mhammer@misslink.net

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: esmit@ilink.nis.za

Subject: unsubscribing

Date: Tue, 5 May 1998 19:41:52 +0200

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Hi,

I am going to leave the mailing list with a unhappy heart.

Why?  Well, i don't read my mail anymore and have a new hobby now( Special

effects like in the movies). I have over the 3000 emails for me waiting to

be downloaded.

I will surely chat in the IRC channel and if i need help with my tesla coil

i will grab one of you guys email address and ask you a question.

Thanks for all your friendly  chats and news.

Love

Richard Smit (South Africa)

X-Mailer: IPAD 2.0

Date: Tue, 05 May 1998 12:45:03 -0500

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: T2 disruptive and solid-state comparisons

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

At 05:01 AM 5/5/98 EDT, you wrote:

>Mark,

>    I am not sure if my last letter went through, so I am writing one to be

>sure.  I would like to see the schematic for you solid state driver circuit.

>I may be interested in trying one myself.

>Thanks

>Mike Hollingsworth

Mike,

Your previous post went through fine.

Disregard any error messages you receive from AOL.

The messages are due to some quirks in the way AOL

handles email errors.

This list serve echoes your post back to you. Once

you receive the echo you can be sure your post

was sent out to the list.

Ive been keeping quite close tabs on the list server

with two seperate mail accounts and their have been

no dropped messages.

TTYL!

Mike Hammer

mhammer@misslink.net

From: "Basura, Brian" <brian.basura@unistudios.com>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: RE: A couple of ?'s

Date: Tue, 5 May 1998 12:31:29 -0700

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Mike,

I think you made a typo in the formula.  Shouldn't it be:

>    L(uH) = (r^2 * N^2) / 9*r + 10*h

>

> where:

>    r = coil RADIUS in inches

>    N = number of turns

>           h = coil height in inches

>

Brian D. Basura

USITG - Manager, Technical Support

(818)777-0979

> ----------

> From: 	Mike Hammer[SMTP:mhammer@misslink.net]

> Reply To: 	Tesla-2 Mailing List

> Sent: 	Tuesday, May 05, 1998 10:39 AM

> To: 	Tesla-2 Mailing List

> Subject: 	Re: A couple of ?'s

>

> At 12:50 AM 5/5/98 -0400, you wrote:

> >I see in some illustrations for a primary of the coil that ther is a

> >"Strike Ring". What is it and what is it attached to ?

> >Ref. www.hills2.u-net.com.tesla/mytesla.htm     page 2.

>

>

> A strike ring is a grounded ring placed just outside and above

> your last primary turn. Its function is to attract and conduct

> safely to ground any streamers that want to strike down into

> your primary circuit. Streamer hits into the primary can cause

> capacitor and transformer failures.

>

> Some coils have a tendency to hit into the primary while

> others do not. Saucer shaped primary are often the worst

> in this regard. Flat spirals are better because of the greater

> distance between the toroid and primary. Solenoid primary coils

> often will not have this problem as they are in close to the secondary.

>

> My coil with a tapered solenoid primary has never had any primary

> strikes so I never added a strike rail.

>

> *NOTE*

> The strike rail should NOT be in the form of a closed loop or

> ring. It must be open. One end only of the ring should be grounded

> and the other end left free. A closed loop will look like a shorted

> turn to your coil and suck up lots of power.

>

>

>

> >

> >What are the specifications for air core inductors length, diameter.

> >#turns, wire size, to protect a 12K 30 ma. trans driving a 4.5" X 21"

> >secondary, the ferrite cores are to hard to find.

> >

>

> New ferrite and iron powder toroids and rods may be purchased

> from Amidon Corp. Their web page is: http://bytemark.com/amidon/

>

> A couple of people have purchased some surplus toroids recently

> from Electronic Goldmine. Problem with surplus is here today

> gone tomorrow. Plus sometimes you never really know what you

> are going to get. Check out Electronic Goldmine at:

> http://www.goldmine-elec.com/

>

> The requirements for a RF choke are pretty loose.

> Anything from 2-10 mh will work fine. You are not building

> a filter. Just something to knock down the RF some.

>

> Try from 350-400 turns of AWG 26 on a piece of 2 inch PVC

> pipe. This will result in a winding 6-7 inches long with

> and inductance of around 2.5 mh.

>

> To calculate the inductance of your chokes use

> Wheeler's formula.

>

>

>

>    L(uH) = (r^2) * (N^2) / (9*r + 10*h)

>

> where:

>    r = coil RADIUS in inches

>    N = number of turns

>    h = coil height in inches

>

>

>

> If using air core chokes its a good idea to include series

> damping resistors. Put one in each HV line in between your

> neons and choke. Anything from 300-1500 ohms and 10 or

> more watts will be fine. If you run higher power you

> may need higher power resistors.

>

> I acquired some 310 ohm 50 watt resistors at a hamfest for

> 10 cents apiece. I included a pair of these in my rebuild neon

> right inside the tank of oil. They are a permanent part of the

> transformer.

>

> TTYL!

>

> Mike Hammer

> mhammer@misslink.net

>

From: "Basura, Brian" <brian.basura@unistudios.com>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: RE: A couple of ?'s

Date: Tue, 5 May 1998 12:43:14 -0700

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Mike,

You're original formula was correct. Sorry for the confusion.  You do need

to multiply the result by .001 to get (uh) though.

Regards,

Brian D. Basura

X-Mailer: IPAD 2.0

Date: Tue, 05 May 1998 15:00:53 -0500

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Mike Hammer <mhammer@misslink.net>

Subject: RE: A couple of ?'s

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

At 12:31 PM 5/5/98 -0700, you wrote:

>Mike,

>

>I think you made a typo in the formula.  Shouldn't it be:

>

>>    L(uH) = (r^2 * N^2) / 9*r + 10*h

>>

>> where:

>>    r = coil RADIUS in inches

>>    N = number of turns

>>           h = coil height in inches

>>

>Brian D. Basura

>USITG - Manager, Technical Support

>(818)777-0979

Brian,

Hmm. Only difference is some of the brackets.

Ive always used the formula in this form and

cut and pasted it into my post. I copied the

formula originally from a RF engineering book

that I no longer have. (never loan a book to a friend)

The Radio Amateurs Handbook shows the formula

sans brackets altogether as:

L(uh)= r^2 N^2 / 9r 10h

and Orrs Radio Handbook shows it the same.

I have no problem with your form but cant

see that it makes much difference.

Also as Bob Schumann pointed out earlier for highest accuracy

the coil radius (r) should include 1/2 the wire diameter.

But the difference is slight and probably not significant

in the types of coils we are using in Tesla coils.

TTYL!

Mike Hammer

mhammer@misslink.net

X-Mailer: IPAD 2.0

Date: Tue, 05 May 1998 15:47:11 -0500

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: A couple of ?'s

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

At 09:22 PM 5/5/98 -0700, you wrote:

>I have a choice between 32 awg and 27 awg. my plan was to use 2" diameter

>by 2/12" long formers for my chokes. I calculate that with the 32 awg this

>will give me 3.5 mh with a dc resistance of 28 ohms or 1.8mh with 8 ohms dc

>resistance using the 27 awg. Any offers on which would be best? should I

>use longer formers and go for higher impedance.? I am using a 9kv 50 mA

>neon. I used win tesla to calculate the impedance  and the wiretronics

>table for the dc resistance. many thanks.

>

>bob golding lightning without rain

>

>bob yubba@clara.net

>

Bob,

For a 2 inch diameter winding 2.5 inches long with AWG 27 I

get about 770 uh of inductance. Thats about 160 turns.

I checked it against WinTesla and got the same figure.

Thats not enough inductance.

I would go with the AWG 27 and lengthen your forms to 6 inches.

That will give you about 2.2 mh and around 390 turns.

Make sure of the diameter of your form. 2 inch PVC is not 2 inches

outside diameter. Its 2.375 inches.

Dont forget the series damping resistors and the safety gap.

The safety gap is even more important than chokes. Ive often run

without chokes but would never do so without a safety gap across

the transformer.

TTYL!

Mike Hammer

mhammer@misslink.net

From: Roy Minnich <rminnich@rolling-meadows.lib.il.us>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: RE: Buyers Warning!

Date: Tue, 5 May 1998 16:26:48 -0500

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

Hi everybody,

I want to say that I bought a 12kv 30ma neon from Tedd and got a good neon and good service from him.  The neon was well packed in bubble wrap and shipped to my door via

UPS within 10 days of my mailing a money order to him.

Thank you Tedd!

Roy Minnich

----------

From: 	Jim Ryan[SMTP:jryan@exis.net]

Reply To: 	Tesla-2 Mailing List

Sent: 	Thursday, April 30, 1998 2:25 PM

To: 	Tesla-2 Mailing List

Subject: 	Re: Buyers Warning!

At 12:20 AM 4/28/98 -0700, you wrote:

>

>

>mike nicholas wrote:

>

>> Its good to know, but I generally consider it a bad idea to send money

to someone

>

Those of you who responded to my post about "buyers warning"

are absolutely correct. I do apologize to Mr.Tedd Payne on this News Group.

I have not been on this group long enough to recognize all the names,

and should have waited or asked more questions first.

Jim Ryan

Attachment Converted: "D:\MISSING_ LINK\EUDORA\Attach\RE Buyers Warning!"

Date: Tue, 5 May 1998 17:12:12 -0600

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Daniel Lockhart <molly@cmn.net>

Subject: RE: Buyers Warning!

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

Hey all,

I received the same service from Tedd and got three neons from him last

weekend. They were shipped UPS and were packed tightly with newspaper and

styrofoam peanuts. All three transformers work, and are in great shape.

Daniel Lockhart

>Hi everybody,

>

>I want to say that I bought a 12kv 30ma neon from Tedd and got a good neon

>and good service from him.  The neon was well packed in bubble wrap and

>shipped to my door via UPS within 10 days of my mailing a money order to

>him.

>

>Thank you Tedd!

>Roy Minnich

From: wwl@netcomuk.co.uk (Mike Harrison)

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: A couple of ?'s

Date: Tue, 05 May 1998 23:29:39 GMT

Organization: White Wing Logic

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

On Tue, 05 May 1998 09:15:26 -0700, you wrote:

>Jim Ryan wrote:

>>

>> I see in some illustrations for a primary of the coil that ther is a

>> "Strike Ring". What is it and what is it attached to ?

>> Ref. www.hills2.u-net.com.tesla/mytesla.htm     page 2.

>>

>> Thanks

>

>Jim,

>For whatever value, I have a Strike Ring in my circuit diag. at our site

>below. It serves to protect the Pri and Pri drive Tank components from a

>hit from the 2ndary terminal. The ring should be placed just above the

>the Pri coil, to attract and ground such strikes.

>

>---------------------------------------------------------

>Gene Waldo -  gwaldo@1stnetusa.com  - "Igor in Residence"

>---------------------------------------------------------------

>

>

Remember not to make it a complete ring i.e. put a small gap in it, to

stop it acting as a shorted turn & draining power.

____                                                           ____

_/ L_/  Mike Harrison / White Wing Logic / wwl@netcomuk.co.uk  _/ L_/

_/ W_/  Hardware & Software design / PCB Design / Consultancy  _/ W_/

/_W_/  Industrial / Computer Peripherals / Hazardous Area      /_W_/

X-Originating-IP: [208.16.149.158]

From: "Bill the arcstarter" <arcstarter@hotmail.com>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: T2 Coil Tuning and more Spark Gap Questions

Date: Tue, 05 May 1998 19:07:03 PDT

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

Steve Falco wrote:

>

>I filed oblong holes to get the adjustment range, and did *not* use

>epoxy, so I could remove the tubing for easier cleaning and adjustment.

>I recognize that this defeats the purpose of the "never needs

>adjustment" concept, but I'm happier with it my way.  (Actually, I

>started with epoxy initially, but found that it didn't adhere well to

>the PVC (even with sanding), so I cleaned it up and started over

>without.)

Hmm.  I tried this before epoxying the tubes down but had a very

difficult time keeping the pipe edges parallel.

Perhaps there is a trick you could share with us for doing this?  How

long does it take you to reinstall and adjust the gaps after cleaning?

Having removable gaps would be great for cleaning purposes...

I had to use lengths of hacksaw blade as a shim between pipes, and

tighten the nuts down in such a way as to draw the two pipes together,

pinching the shim.  All this was done after I had applied epoxy along

the pipe to PVC contact area.

-Bill the arcstarter

Starting arcs in Cinci, OH

http://www.geocities.com/CapeCanaveral/Hangar/6160

From: Hollmike <Hollmike@aol.com>

Date: Tue, 5 May 1998 23:40:23 EDT

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: A couple of ?'s

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

In a message dated 98-05-05 15:32:15 EDT, you write:

<<

>>

Brian,

Mike's formula is correct.  In the format typed, the parentheses

need to be there.

Mike Hollingsworth

From: Hollmike <Hollmike@aol.com>

Date: Tue, 5 May 1998 23:43:56 EDT

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: A couple of ?'s

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Sorry Brian,

Again, Mike's formula is correct for microhenries.  multiplying by 0.001

will yield millihenries, which is the range for coil secondaries.

Mike Hollingsworth.

From: Hollmike <Hollmike@aol.com>

Date: Tue, 5 May 1998 23:49:12 EDT

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: A couple of ?'s

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Let me beat this one to death,

The Wheeler formula that Mike posted for the single layer helical coil is

accurate to within 5% if  2*r > h > 0.2*r, although the accuracy may be less

if skin effect or distributed capacitance is appreciable.

Mike Hollingsworth.

X-Mailer: IPAD 2.0

Date: Tue, 05 May 1998 21:22:45 -0700

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: bob golding <yubba@clara.net>

Subject: Re: A couple of ?'s

Cc: tesla-2@emachine.com

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

At 12:39 05/05/98 -0500, you wrote:

>At 12:50 AM 5/5/98 -0400, you wrote:

>snip

>The requirements for a RF choke are pretty loose.

>Anything from 2-10 mh will work fine. You are not building

>a filter. Just something to knock down the RF some.

>

>Try from 350-400 turns of AWG 26 on a piece of 2 inch PVC

>pipe. This will result in a winding 6-7 inches long with

>and inductance of around 2.5 mh.

>

>To calculate the inductance of your chokes use

>Wheeler's formula.

>

>

>

>   L(uH) = (r^2) * (N^2) / (9*r + 10*h)

>

>where:

>   r = coil RADIUS in inches

>   N = number of turns

>   h = coil height in inches

I have a choice between 32 awg and 27 awg. my plan was to use 2" diameter

by 2 1/2" long formers for my chokes. I calculate that with the 32 awg this

will give me 3.5 mh with a dc resistance of 28 ohms or 1.8mh with 8 ohms dc

resistance using the 27 awg. Any offers on which would be best? should I

use longer formers and go for higher impedance.? I am using a 9kv 50 mA

neon. I used win tesla to calculate the impedance  and the wiretronics

table for the dc resistance. many thanks.

bob golding lightning without rain

bob yubba@clara.net

X-Mailer: IPAD 2.0

Date: Tue, 05 May 1998 23:28:02 -0500

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: T2 Coil Tuning and more Spark Gap Questions

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

At 07:07 PM 5/5/98 PDT, you wrote:

>Steve Falco wrote:

>>

>>I filed oblong holes to get the adjustment range, and did *not* use

>>epoxy, so I could remove the tubing for easier cleaning and adjustment.

>>I recognize that this defeats the purpose of the "never needs

>>adjustment" concept, but I'm happier with it my way.  (Actually, I

>>started with epoxy initially, but found that it didn't adhere well to

>>the PVC (even with sanding), so I cleaned it up and started over

>>without.)

>

>Hmm.  I tried this before epoxying the tubes down but had a very

>difficult time keeping the pipe edges parallel.

>

>Perhaps there is a trick you could share with us for doing this?  How

>long does it take you to reinstall and adjust the gaps after cleaning?

>Having removable gaps would be great for cleaning purposes...

Steve, Bill and others,

On my first RQ type cylinder gap I tried using just two bolts

to hold the electrodes down and no epoxy. I found that the gaps

refused to stay in alignment. Possibly due to differential heating.

So I reworked the gap cutting slots for the mounting bolts

to have plenty of adjustment room. I epoxied down the first

electrode and when the epoxy cured I clamped the next electrode

to it with a .025 feeler guage between them. I used a locking

hemostat at each end as clamps. I left the clamps on until

the epoxy cured. I then repeated the procedure one electrode

at a time.

My electrodes all came out perfectly spaced and parallel

and never go out of adjustment.

Make sure you use a good grade of high temperature epoxy.

Many of the cheaper household 5 minute epoxies soften easily under heat.

For cleaning purposes I attached strips of fine sandpaper

to both sides of a suitable thickness feeler guage with

contact cement. I run this tool down between the electrodes

and it only takes a few swipes to clean them up.

TTYL!

Mike Hammer

mhammer@misslink.net

X-Mailer: IPAD 2.0

Date: Wed, 06 May 1998 00:52:24 -0400

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Jim Ryan <jryan@exis.net>

Subject: Re: T2 Coil Tuning and more Spark Gap Questions

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

>At 07:07 PM 5/5/98 PDT, you wrote:

>>

>

>Make sure you use a good grade of high temperature epoxy.

>Many of the cheaper household 5 minute epoxies soften easily under heat.

>

>For cleaning purposes I attached strips of fine sandpaper

>to both sides of a suitable thickness feeler guage with

>contact cement. I run this tool down between the electrodes

>and it only takes a few swipes to clean them up.

>

>TTYL!

>

Mike, what is the good grade of epoxy you used ?

Thanks

Jim

X-Mailer: IPAD 2.0

Date: Tue, 05 May 1998 22:19:17 -0700

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: bob golding <yubba@clara.net>

Subject: chokes

Cc: tesla-2@emachine.com

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Hi mike

Checked the figures again. must have hit the wrong button first time

round!! It's all this damm converting from imperial to metric that does it.

I am much happier with imperial as I was brought up on it. A question about

filters in eudora. I have just started to use it and would like to know the

best way to set up the filters for the t2 list and the tesla list. I share

my mailbox with two other people but I get the most mail (about 40 Messages

a day) Any ideas? thanks bob.

X-Mailer: IPAD 2.0

Date: Wed, 06 May 1998 10:02:31 -0500

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: T2 Spark Gap Questions/epoxy

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

At 12:52 AM 5/6/98 -0400, you wrote:

>>At 07:07 PM 5/5/98 PDT, you wrote:

>>>

>>

>>Make sure you use a good grade of high temperature epoxy.

>>Many of the cheaper household 5 minute epoxies soften easily under heat.

>>

>>For cleaning purposes I attached strips of fine sandpaper

>>to both sides of a suitable thickness feeler guage with

>>contact cement. I run this tool down between the electrodes

>>and it only takes a few swipes to clean them up.

>>

>>TTYL!

>>

>Mike, what is the good grade of epoxy you used ?

>

>Thanks

>

>Jim

I used Duro 2 ton crystal clear epoxy. This is the one

that comes in two side by side syringes.

You can figure that most quick 5 minute epoxies wont

work well. The stronger epoxies have slower cure times.

The Duro was a 30 minute cure if I remember correctly.

TTYL!

Mike Hammer

mhammer@misslink.net

Date: Wed, 06 May 1998 08:43:15 -0700

From: Gene Waldo <gwaldo@1stnetusa.com>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Organization: Self

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Stay abreast.....

Sender: tesl-err@emachine.com

All,

Note to newbies; reminder to mossbacks:

http://www.lod.org/

A site with great potential, is a worth periodic revisits!

Thanks to Mike Hammer for turning me on to this site!

---------------------------------------------------------

Gene Waldo -  gwaldo@1stnetusa.com  - "Igor in Residence"

---------------------------------------------------------------

What's good for the planet is more OZONE! See the machinery form up!

NEW STUFF!   http://www.misslink.net/mhammer/waldo.htm

From: Roy Minnich <rminnich@rolling-meadows.lib.il.us>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: RE: Buyers Warning!

Date: Wed, 6 May 1998 12:17:17 -0500

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

My 12kv30ma was $17 plus $8 shipping.  It looks brand new except for the

missing name tag and works great!

Roy Minnich

----------

From: 	Fred Decker[SMTP:fdecker@csi.com]

Reply To: 	Tesla-2 Mailing List

Sent: 	Tuesday, May 05, 1998 10:27 PM

To: 	Tesla-2 Mailing List

Subject: 	Re: Buyers Warning!

How much were the transformers and does Tedd have an unlimited supply? <g>

Fred

-----Original Message-----

From: Daniel Lockhart <molly@cmn.net>

To: Tesla-2 Mailing List <tesla-2@emachine.com>

Date: Tuesday, May 05, 1998 7:56 PM

Subject: RE: Buyers Warning!

>Hey all,

>

>I received the same service from Tedd and got three neons from him last

>weekend. They were shipped UPS and were packed tightly with newspaper and

>styrofoam peanuts. All three transformers work, and are in great shape.

>

>Daniel Lockhart

>

>

>>Hi everybody,

>>

>>I want to say that I bought a 12kv 30ma neon from Tedd and got a good neon

>>and good service from him.  The neon was well packed in bubble wrap and

>>shipped to my door via UPS within 10 days of my mailing a money order to

>>him.

>>

>>Thank you Tedd!

>>Roy Minnich

>

>

>

X-Mailer: IPAD 2.0

Date: Wed, 06 May 1998 13:35:37 -0400

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Jim Ryan <jryan@exis.net>

Subject: 24 & 26 GA Wire

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Hello, I have another question. Is there a site on the Net

where one can order magnetic wire?

Thanks

Jim

From: Roy Minnich <rminnich@rolling-meadows.lib.il.us>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: T2  basic math & cap Qs

Date: Wed, 6 May 1998 13:11:40 -0500

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

Hello all,

How do you calculate the wattage a TC system is using?

If you have a neon with no name plate how do you take readings on a neon's windings to determine what the V and A output will be?

When making a rolled poly/oil cap with many layers of .004 poly will a 1000V per mil puncture rating be achieved?  So if I want a 40KV puncture rating and I use 12 layers of 4 mil poly between the plates I will get 12 X 4 = 48 mil X 1000V = 48,000V puncture rating?

I see that the formula for determining the value of a rolled 2 plate poly cap is C = (.224 x K x A/d) x 2 but the formula for a flat plate cap is C = (.224 x K x A/d) x (number of plates -1).  Why the -1 in the flat plate cap formula?

Lastly, will stranded wire be OK for the plate to external screw connectors on my rolled poly caps?  I have some stranded #12 that I was going to use to keep the leads flexible.

Thanks,

Roy Minnich

Date: Wed, 6 May 1998 12:17:31 -0600

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Daniel Lockhart <molly@cmn.net>

Subject: Re: Buyers Warning!

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

>How much were the transformers and does Tedd have an unlimited supply? <g>

>

>Fred

I don't know how big a supply he has, but if I remember correctly, he was

selling about a dozen or so.

I paid $17.00 for a 12/30, and shipping cost $8.00. The transformer runs

great and was made in late 1997 by ACtown-Electrocoil.

My two cents,

Daniel Lockhart

Date: Wed, 6 May 1998 12:26:18 -0600

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Daniel Lockhart <molly@cmn.net>

Subject: Re: T2 Coil Tuning and more Spark Gap Questions

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

Mike Hammer wrote:

>On my first RQ type cylinder gap I tried using just two bolts

>to hold the electrodes down and no epoxy. I found that that gaps

>refused to stay in alignment. Possibly due to differential heating.

>

>So I reworked the gap cutting slots for the mounting bolts

>to have plenty of adjustment room. I epoxied down the first

>electrode and when the epoxy cured I clamped the next electrode

>to it with a .025 feeler guage between them. I used a locking

>hemostat at each end as clamps. I left the clamps on until

>the epoxy cured. I then repeated the procedure one electrode

>at a time.

>

>My electrodes all came out perfectly spaced and parallel

>and never go out of adjustment.

Okay, now I feel real dumb. What is a "feeler gauge?" I haven't a clue on

what they are. I was planning on just finding something with about the

right thickness to measure it. Remember, you're talking to a guy who is

just lucky enough to have a Dremel tool in his shop -- forget anything

fancy like a drill press or anything (where would I put it!?). Are feeler

gauges like micrometers or calipers?

Didn't Bill the arcstarter use a hacksaw blade? Can you tell me more about

this, Bill?

Thanks!,

Daniel Lockhart

From: Roy Minnich <rminnich@rolling-meadows.lib.il.us>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: RE: 24 & 26 GA Wire

Date: Wed, 6 May 1998 13:31:00 -0500

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

Call Joe Cassata at the Coax Connection 630-420-0342.  #26 is $8 a pound (1280 ft) and #24 is $7.20 a pound (802 ft).  Joe has great service and good prices.  He takes checks but no plastic.

Roy Minnich

----------

From: 	Jim Ryan[SMTP:jryan@exis.net]

Reply To: 	Tesla-2 Mailing List

Sent: 	Wednesday, May 06, 1998 12:35 PM

To: 	Tesla-2 Mailing List

Subject: 	24 & 26 GA Wire

Hello, I have another question. Is there a site on the Net

where one can order magnetic wire?

Thanks

Jim

From: Roy Minnich <rminnich@rolling-meadows.lib.il.us>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: T2  basic math & cap Qs #2

Date: Wed, 6 May 1998 14:43:49 -0500

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

Hello,

I'm having problems with my cap design.  Please help!

C = (.224 x K x A/d) x 2 is the formula I'm using to determine the value of a

rolled poly plate cap I want to build.  I'm using 2.15 for K, my plates are

12 in. x 27.5 in. and I'm using .048 mil of poly so.......

.224 x 2.15 x 330/.048) x 2 = 3311 x 2 = 6622 pf  and .000001 x 6622 = .006622 uf....Right??????????

I get a value of 3.606625E-03 uf in STESLA which is (I think)

3.60625 X .001 = .0036066 uf.  This is only half the value it should be.

Am I missing something??  I get similar numbers in Walt Noons The Tesla Coil Designer.

It looks like these two programs are only giving me 1/2 the cap value.

Please help I want to roll it tonight.

Thanks,

Roy Minnich

From: Roy Minnich <rminnich@rolling-meadows.lib.il.us>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: RE: T2 Coil Tuning and more Spark Gap Questions

Date: Wed, 6 May 1998 14:56:55 -0500

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

Hi Dan,

Don't feel dumb it's nothing special.

A feeler gauge is a thin strip of metal , usually steel but I have seen brass ones,

that is made to a specific thickness to use as a tool to gauge the thickness of a

gap between two machine parts.  Back in the old days when cars still had a set of

points in the distributor you had to set the gap between the points using a feeler

gauge of the correct thickness to get the proper dwell setting for the spark ignition timing for the engine.

You can get a set of feeler guages at any auto parts store or any hardware store worth

it's salt.

Or..... if you have a caliper or micrometer you could just measure the thickness of some thin metal or plastic or formica or whatever you might have lying around until you find something of the correct thickness to use as a feeler gauge.

They sell metal strips in hobby stores in various thickness which you could use as a

gauge if you don't have any tools to measure thickness with.

Roy Minnich

----------

From: 	Daniel Lockhart[SMTP:molly@cmn.net]

Reply To: 	Tesla-2 Mailing List

Sent: 	Wednesday, May 06, 1998 1:26 PM

To: 	Tesla-2 Mailing List

Subject: 	Re: T2 Coil Tuning and more Spark Gap Questions

Mike Hammer wrote:

>On my first RQ type cylinder gap I tried using just two bolts

>to hold the electrodes down and no epoxy. I found that that gaps

>refused to stay in alignment. Possibly due to differential heating.

>

>So I reworked the gap cutting slots for the mounting bolts

>to have plenty of adjustment room. I epoxied down the first

>electrode and when the epoxy cured I clamped the next electrode

>to it with a .025 feeler guage between them. I used a locking

>hemostat at each end as clamps. I left the clamps on until

>the epoxy cured. I then repeated the procedure one electrode

>at a time.

>

>My electrodes all came out perfectly spaced and parallel

>and never go out of adjustment.

Okay, now I feel real dumb. What is a "feeler gauge?" I haven't a clue on

what they are. I was planning on just finding something with about the

right thickness to measure it. Remember, you're talking to a guy who is

just lucky enough to have a Dremel tool in his shop -- forget anything

fancy like a drill press or anything (where would I put it!?). Are feeler

gauges like micrometers or calipers?

Didn't Bill the arcstarter use a hacksaw blade? Can you tell me more about

this, Bill?

Thanks!,

Daniel Lockhart

X-Originating-IP: [199.217.140.22]

From: "Bill the arcstarter" <arcstarter@hotmail.com>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: T2 Coil Tuning and more Spark Gap Questions

Date: Wed, 06 May 1998 13:14:52 PDT

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

It was written:

>Okay, now I feel real dumb. What is a "feeler gauge?" I haven't a clue

on...

>Didn't Bill the arcstarter use a hacksaw blade? Can you tell me more

about

>this, Bill?

>

>Thanks!,

>

>Daniel Lockhart

Hello Daniel, other coilers,

A feeler gauge is a set of precision metal shims of various

thicknesses.  This tool is used for various machine shop tasks, and also

for many automotive/small engine repair jobs.  You can get one for <$5

at Auto Zone or your favorite car parts place.

But I don't think you really need one for building your RQ gap.  I

drove over the "Dollar Store" - (home of some worthless Taiwan tools)

and bought 3 or 4 of the cheapest hacksaw blades I could find.

I then cut them on the chopsaw into convenient three inch lengths.

Oh - unless you have a grinder or a cutoff wheel tool - you won't be

able to cut through these blades as they are tempered and/or high

carbon...

I was trying to avoid getting epoxy all over my feeler gauges!!

But you can use any sort of flat parallel spacer - could probably use a

couple of waxed playing cards or something in a pinch.  Just remember

you will probably get epoxy all over everything... :)  The exact spacing

is not critical - but keeping the metal edges parallel certainly is!

My particular gap contains (I think) 10 pipes.  I mounted pipe #6

first.  Then - while the epoxy under #6 was still curing - I started

mounting the next pipes on either side (#5 and #7).  Once I had those

down, I then went on to #4 and #8.  In this way I didn't have to wait

for the epoxy to cure before installing the next set of pipes.

I made a little jig in an attempt to drill all the holes on the pipes

the same spacing apart.  In this way I tried to do a bit of assembly

line work to speed up the process.  Building this gap took at least 10

hours, not including any fan shrouds, etc.  But it was well worth it!

Don't make the spacing so tight that you can't get a piece of sandpaper

down between the pipes for cleaning purposes!

-Bill the arcstarter

Starting arcs in Cinci, OH

http://www.geocities.com/CapeCanaveral/Hangar/6160

From: Roy Minnich <rminnich@rolling-meadows.lib.il.us>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: OBIT

Date: Wed, 6 May 1998 15:21:30 -0500

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

Hello all,

What's an OBIT?

Thanks,

Roy Minnich

X-Mailer: IPAD 2.0

Date: Wed, 06 May 1998 15:25:53 -0500

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: T2 Spark Gap\feeler guage Questions

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

At 12:26 PM 5/6/98 -0600, you wrote:

>

>Mike Hammer wrote:

>

>>On my first RQ type cylinder gap I tried using just two bolts

>>to hold the electrodes down and no epoxy. I found that that gaps

>>refused to stay in alignment. Possibly due to differential heating.

>>

>>So I reworked the gap cutting slots for the mounting bolts

>>to have plenty of adjustment room. I epoxied down the first

>>electrode and when the epoxy cured I clamped the next electrode

>>to it with a .025 feeler guage between them. I used a locking

>>hemostat at each end as clamps. I left the clamps on until

>>the epoxy cured. I then repeated the procedure one electrode

>>at a time.

>>

>>My electrodes all came out perfectly spaced and parallel

>>and never go out of adjustment.

>

>Okay, now I feel real dumb. What is a "feeler gauge?" I haven't a clue on

>what they are. I was planning on just finding something with about the

>right thickness to measure it. Remember, you're talking to a guy who is

>just lucky enough to have a Dremel tool in his shop -- forget anything

>fancy like a drill press or anything (where would I put it!?). Are feeler

>gauges like micrometers or calipers?

Feeler guages are strips of steel of precise thickness.

They are 12 inches long and 1/2 inch wide and available in

.001 inch increments. I use one .025 inch thick to set the

electrode spacing on my gap.

The ones I have are made by The L.S. Starret Co.

They should be available anywhere machinists tools

are available.

>

>Didn't Bill the arcstarter use a hacksaw blade? Can you tell me more about

>this, Bill?

A hacksaw blade seems a little thick to me. Perhaps

someone with a micromenter could measure a couple.

The one Im looking at now I guesstimate at about .04 inch.

You want your gaps set about .025-.03 inch.

TTYL!

Mike Hammer

mhammer@misslink.net

From: Roy Minnich <rminnich@rolling-meadows.lib.il.us>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: RE: spark gap

Date: Wed, 6 May 1998 15:27:15 -0500

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

Hi,

And how big should the safety gap be??

Roy Minnich

----------

From: 	Mike Hammer[SMTP:mhammer@misslink.net]

Reply To: 	Tesla-2 Mailing List

Sent: 	Friday, May 01, 1998 8:48 PM

To: 	Tesla-2 Mailing List

Subject: 	Re: spark gap

At 06:19 PM 5/1/98 -0700, you wrote:

>

>

>	I plan on building a gap where i have several lenghs of copper pipe vacuum

>quenched. But i can't seem to find any formulas or "general rules" that

>would tell me how many or the spacing between the pipes. Are there any?

>

>Thanks,

>--MIke Drew

Only some general rules of thumb. It really depends

on your transformer voltage. Higher voltage needs wider gap.

For 12kv 9 elecrodes spaced .03 inches apart will yield

8 series gaps with a total spacing of .240 inch. That

is all the gap you will ever need for 12kv. Consider it

a maximum value. My own gap for 12kv use has 9 series

gaps (10 electrodes) spaced .025 inch for a total of .225 inch.

For 15kv increase the number of electrodes to 11 spaced .03 inch.

This gives a maximum of .300 with 10 series gaps.

If building a vacuum gap make sure the airflow is directly into

the gap area between the electrodes. That gives best performance.

You will probably find that your vacuum cleaner pulls too much

air through the gap. I run mine on a variac to set the speed

for best output.

TTYL!

Mike Hammer

mhammer@misslink.net

X-Mailer: IPAD 2.0

Date: Wed, 06 May 1998 16:27:47 -0400

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Moleking <moleking@concentric.net>

Subject: Re: 24 & 26 GA Wire

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

I got my wire from Gary Weaver.  The price was good and he shipped it out

rather quickly.  You can get him at gweaver@earthlink.net or his site at

home.earthlink.net/~gweaver.  He's got lots of #24, and some other gauges

too I believe.

Josh Bedell

At 01:35 PM 5/6/98 , you wrote:

>Hello, I have another question. Is there a site on the Net

>where one can order magnetic wire?

>

>Thanks

>

>Jim

>

X-Mailer: IPAD 2.0

Date: Wed, 06 May 1998 16:30:56 -0400

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Moleking <moleking@concentric.net>

Subject: Re: T2  basic math & cap Qs #2

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Roy,

What exactly are these files which come attached to every email I receive

from you?  I hate to complain, but they're filling up my downloads folder!

Is there any way you can disable those for the list?   Thanks.

Josh Bedell

X-Mailer: IPAD 2.0

Date: Wed, 06 May 1998 15:34:21 -0500

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: T2  basic math & cap Qs #2

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

At 02:43 PM 5/6/98 -0500, you wrote:

>Hello,

>

>I'm having problems with my cap design.  Please help!

>

>

>C = (.224 x K x A/d) x 2 is the formula I'm using to determine the value

of a rolled poly plate cap I want to build.  I'm using 2.15 for K, my

plates are 12 in. x 27.5 in. and I'm using .048 mil of poly so.......(.224

x 2.15 x 330/.048) x 2 = 3311 x 2 = 6622 pf  and .000001 x 6622 = .006622

uf....Right??????????

Yep. Exactly right. I would use a nominal value of 2.0 for the

K of poly. This lower value helps to take into account less than

perfect plate alignment and spacing.

>

>I get a value of 3.606625E-03 uf in STESLA which is (I think) 3.60625 X

>.001 = .0036066 uf.  This is only half the value it should be.  Am I

>missing something??  I get similar numbers in Walt Noons The Tesla Coil Designer.

>It looks like these two programs are only giving me 1/2 the cap value.

I cant vouch for the accuracy of these programs as I do not

use either of them. Most people use the formula you used and

get accurate results.

>

>Please help I want to roll it tonight.

Roll away.

TTYL!

Mike Hammer

mhammer@misslink.net

X-Mailer: IPAD 2.0

Date: Wed, 06 May 1998 15:54:01 -0500

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: T2  basic math & cap Qs

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

At 01:11 PM 5/6/98 -0500, you wrote:

>Hello all,

>

>How do you calculate the wattage a TC system is using?

Not easily since the input waveform is not sinusoidal.

Most of us just report the VA rating of the supply

transformer. A 12kv/30ma ma neon is then 360 VA.

>

>If you have a neon with no name plate how do you take readings on a neon's

>windings to determine what the V and A output will be?

The most reliable way to measure the voltage is to input a low AC

voltage into the neon primary and measure the resulting secondary

voltage then divide to get the turns ratio.

For instance inputing 4 volts on the primary of a 12kv neon

may result in 400 volts output. Dividing 400/4 results in

a 100 to 1 turns ratio. So with 120 volts on the primary

100 times that is 12kv.

Be careful. Even 400 volts is enough for a nasty shock.

To determine the current rating the best way is to connect

an AC rated milliammeter or DVM directly across the HV

output then plug the neon into full line voltage. The

meter will short circuit the neon and read the short

circuit current flow. This will not harm the meter or neon.

Make sure your meter can read AC milliamps. If you place

it on the neon in voltage mode you will likely ruin your

meter.

>

>When making a rolled poly/oil cap with many layers of .004 poly will a

1000V per mil puncture rating be achieved?  So if I want a 40KV puncture

rating and I use 12 layers of 4 mil poly between the plates I will get 12 X

4 = 48 mil X 1000V = 48,000V puncture rating?

No. Those 1000-1200 volts per mil ratings are based on 1 mil

films and DC voltages. The per mil rating for poly

in Tesla coil service is from 325-375 volts per mil.

You will get the most reliable cap if you split the 48 mils of

poly into two series caps. Make each cap with 24 mils and

put them in series. Each cap will then need to have twice

the required final capacitance.

Series caps is the way to get reliable caps. With a series

string of caps Ive had 48 mils of poly hold up well to 12kv and

60 mils to 15kv. In a single cap even 90 mils wont survive.

>

>I see that the formula for determining the value of a rolled 2 plate poly

cap is C = (.224 x K x A/d) x 2 but the formula for a flat plate cap is C =

(.224 x K x A/d) x (number of plates -1).  Why the -1 in the flat plate cap

formula?

Because in a flat plate cap the two outside plates are only

contributing to the total capacitance with their inside surface.

So the two unused surfaces result in the loss of one full plate.

>

>Lastly, will stranded wire be OK for the plate to external screw

connectors on my rolled poly caps?  I have some stranded #12 that I was

going to use to keep the leads flexible.

I would prefer to use either some solid copper or flat aluminum strap.

In my flat plate caps I folded over a strip of foil several

times to make a thick strap about 1 inch wide. A wide strap like this

has very low resistance and more importantly low inductance.

TTYL!

Mike Hammer

mhammer@misslink.net

X-Mailer: IPAD 2.0

Date: Wed, 06 May 1998 17:19:36 -0400

From: Kevin Batchelor <kbatchelor@arslimited.com>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: OBIT

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Roy:

An OBIT is an Oil Burning Ignition Transformer...it is used to ignite the oil in oil burning furnaces.  They work ok but are not cut out for long term heavy duty usage.

Try to find a neon transformer.

Kevin

>>> Roy Minnich <rminnich@rolling-meadows.lib.il.us> 05/06 4:21 PM >>>

Hello all,

What's an OBIT?

Thanks,

Roy Minnich

X-Mailer: IPAD 2.0

Date: Wed, 06 May 1998 16:24:43 -0500

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: OBIT

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

At 03:21 PM 5/6/98 -0500, you wrote:

>Hello all,

>

>What's an OBIT?

>

>Thanks,

>Roy Minnich

OBIT is acronym for Oil Burner Ignition Transformer.

These are current limited transformers similar to neons

used to ignite the burners on oil burning furnaces.

The are not quite as rugged as neons not meant for

continuous duty operation but serve well.

They are normally rated at 6kv/23ma or 10kv/23ma.

They have virtually no resale value so you can often

get old ones for free from furnace installers and contractors.

If no one uses heating oil in your area then you probably wont

be able to find any.

TTYL!

Mike Hammer

mhammer@misslink.net

From: Roy Minnich <rminnich@rolling-meadows.lib.il.us>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Bad info??

Date: Wed, 6 May 1998 17:05:34 -0500

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

Hello,

OK, I can't find one program or calculator that will properly calculate the value of a rolled plate cap.

I tried the calculator on the Tesla Coil Madness page and it gives half values too.

If I change the number of plate to 3 rather that 2 it will give me the correct answer.

It also gives the wrong value for a cap match with a neon.

Am I missing something??

I'm sticking to my trusty desk top calculator.

Roy Minnich

From: Roy Minnich <rminnich@rolling-meadows.lib.il.us>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: RE: T2  basic math & cap Qs #2

Date: Wed, 6 May 1998 17:34:31 -0500

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

I don't know as I've never seen one.  Can you send me a copy of a message with one

and I'll show it to our LAN administrator here and see if he has a clue.

Roy Minnich

----------

From: 	Moleking[SMTP:moleking@concentric.net]

Reply To: 	Tesla-2 Mailing List

Sent: 	Wednesday, May 06, 1998 3:30 PM

To: 	Tesla-2 Mailing List

Subject: 	Re: T2  basic math & cap Qs #2

Roy,

What exactly are these files which come attached to every email I receive

from you?  I hate to complain, but they're filling up my downloads folder!

Is there any way you can disable those for the list?   Thanks.

Josh Bedell

From: Roy Minnich <rminnich@rolling-meadows.lib.il.us>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: RE: T2  basic math & cap Qs #2

Date: Wed, 6 May 1998 17:41:21 -0500

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

Thanks Mike!!!!!!!!!!!!!

Roy Minnich

----------

From: 	Mike Hammer[SMTP:mhammer@misslink.net]

Reply To: 	Tesla-2 Mailing List

Sent: 	Wednesday, May 06, 1998 3:34 PM

To: 	Tesla-2 Mailing List

Subject: 	Re: T2  basic math & cap Qs #2

<SNIP>

>C = (.224 x K x A/d) x 2 is the formula I'm using to determine the value

of a rolled poly plate cap I want to build.  I'm using 2.15 for K, my

plates are 12 in. x 27.5 in. and I'm using .048 mil of poly so.......(.224

x 2.15 x 330/.048) x 2 = 3311 x 2 = 6622 pf  and .000001 x 6622 = .006622

>uf....Right??????????

Yep. Exactly right. I would use a nominal value of 2.0 for the

K of poly. This lower value helps to take into account less than

perfect plate alignment and spacing.

<SNIP>

Roll away.

TTYL!

Mike Hammer

mhammer@misslink.net

----------

From: 	Mike Hammer[SMTP:mhammer@misslink.net]

Reply To: 	Tesla-2 Mailing List

Sent: 	Wednesday, May 06, 1998 3:54 PM

To: 	Tesla-2 Mailing List

Subject: 	Re: T2  basic math & cap Qs

At 01:11 PM 5/6/98 -0500, you wrote:

>

>When making a rolled poly/oil cap with many layers of .004 poly will a

1000V per mil puncture rating be achieved?  So if I want a 40KV puncture

rating and I use 12 layers of 4 mil poly between the plates I will get 12 X

4 = 48 mil X 1000V = 48,000V puncture rating?

No. Those 1000-1200 volts per mil ratings are based on 1 mil

films and DC voltages. The per mil rating for poly

in Tesla coil service is from 325-375 volts per mil.

You will get the most reliable cap if you split the 48 mils of

poly into two series caps. Make each cap with 24 mils and

put them in series. Each cap will then need to have twice

the required final capacitance.

Series caps is the way to get reliable caps. With a series

string of caps Ive had 48 mils of poly hold up well to 12kv and

60 mils to 15kv. In a single cap even 90 mils wont survive.

TTYL!

Mike Hammer

mhammer@misslink.net

From: Roy Minnich <rminnich@rolling-meadows.lib.il.us>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Bad Info

Date: Wed, 6 May 1998 18:11:35 -0500

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

Hi Mike,

Thanks again for saving me from sure disaster.  I just wonder why there is so much info. out there that isn't really good.  Even the cap building texts by the famous Richard Quick describe building single caps using 60 mil of poly as being rated at 40 to 50K volt.

Also EVERY dielectric chart I've EVER seen since I started looking into TC information on the web has poly listed as having a 1200 volt puncture rating AND not one mentioned that this rating was for AC or DC. These are all TC related sites so you would assume that the information listed was intended for TC design purposes.

I guess this is why I see so many disclaimers out there.

Roy Minnich

From: Roy Minnich <rminnich@rolling-meadows.lib.il.us>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Cap insights

Date: Wed, 6 May 1998 18:50:15 -0500

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

Hi Mike,

>You will get the most reliable cap if you split the 48 mils of

>poly into two series caps. Make each cap with 24 mils and

>put them in series. Each cap will then need to have twice

>the required final capacitance.

>Series caps is the way to get reliable caps. With a series

>string of caps Ive had 48 mils of poly hold up well to 12kv and

>60 mils to 15kv. In a single cap even 90 mils wont survive.

In my plans for the .006uf cap I was gonna make with .048 worth of poly I discovered

that if I reduce the dielectric to .024 the capacitance goes up to .0123uf which is

exactly twice the value of a .006uf cap so when two .0123uf caps are run in series I

get the .006uf value back and the higher voltage rating for my 12kv neon.

Cool! Easier to assemble caps and high series voltage rating.  What more could you

ask for???

I had started messing around with all the layers of poly and foil and could see that

it wasn't going to be THAT easy to do.  Now with only half the layers of poly to keep

track of this should be a breeze.  I can probably fit the roll in 2 in. PVC rather than

3 in. now.  Less oil needed too!!!!!!

If I made 3 or 4 caps to run in series how would this affect the thickness of my poly dielectric needed to keep the total voltage rating?

Thanks,

Roy

X-Originating-IP: [208.16.144.87]

From: "Bill the arcstarter" <arcstarter@hotmail.com>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: T2 Spark Gap\feeler guage Questions

Date: Wed, 06 May 1998 17:50:54 PDT

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

Mike Hammer wrote:

>A hacksaw blade seems a little thick to me. Perhaps

>someone with a micromenter could measure a couple.

>The one Im looking at now I guesstimate at about .04 inch.

>You want your gaps set about .025-.03 inch.

Well I can't speak for all hacksaw blades, but mine measured 0.028 -

just right! :)

-Bill the arcstarter

Starting arcs in Cinci, OH

http://www.geocities.com/CapeCanaveral/Hangar/6160

Date: Wed, 06 May 1998 19:54:43 -0500

From: Bert Hickman <bert.hickman@aquila.com>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Organization: Stoneridge Engineering

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: OBIT

Sender: tesl-err@emachine.com

Roy Minnich wrote:

>

> Hello all,

>

> What's an OBIT?

>

> Thanks,

> Roy Minnich

>

Roy,

The newspaper column you read to find out who just died?... NAH!! :^)

It stands for Oil Burner Ignition Transformer, a cousin to neon sign

transformers (NST's), but typically rated at only 10 KV and 10-30 MA.

Usually not as robust as a NST either.

-- Bert --

Date: Wed, 06 May 1998 18:39:05 -0700

From: Gene Waldo <gwaldo@1stnetusa.com>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Organization: Self

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: T2  basic math & cap Qs #2

Sender: tesl-err@emachine.com

Roy Minnich wrote:

>

> Hello,

>

> I'm having problems with my cap design.  Please help!

>

> Thanks,

> Roy Minnich

Roy,

Is there something neat you can come up with so that your messages don't

come in 3 screens wide?

-------------------------------------------------------

Gene Waldo -  gwaldo@1stnetusa.com  - "Igor in Residence"

---------------------------------------------------------------

Date: Wed, 06 May 1998 20:41:12 -0500

From: Bert Hickman <bert.hickman@aquila.com>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Organization: Stoneridge Engineering

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: T2  basic math & cap Qs #2

Sender: tesl-err@emachine.com

Roy Minnich wrote:

>

> Hey Josh,

>

> Are they txt files?  like ATT00001.txt or something similar.

>

> Any body else getting these attachments I'm not sending from me??

> Don't want to ruin anybody's day.

>

> Roy Minnich

>

> ----------

> From:   Moleking[SMTP:moleking@concentric.net]

> Reply To:       Tesla-2 Mailing List

> Sent:   Wednesday, May 06, 1998 3:30 PM

> To:     Tesla-2 Mailing List

> Subject:        Re: T2  basic math & cap Qs #2

>

> Roy,

>

> What exactly are these files which come attached to every email I receive

> from you?  I hate to complain, but they're filling up my downloads folder!

> Is there any way you can disable those for the list?   Thanks.

>

> Josh Bedell

>

Roy and all,

This is a normal attachment complements of your Microsoft browser.

MS-Exchange adds these little files to their internet text files to

provides additional formatting information, and as far as I know,

there's no way to stop it from appending it to your files. It's a

feature not a bug... :^)

Going to a different (non-Microsoft) browser such as Netscape will

eliminate it.

-- Bert --

X-Mailer: IPAD 2.0

Date: Wed, 06 May 1998 20:57:02 -0500

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: Bad info??

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

At 05:05 PM 5/6/98 -0500, you wrote:

>Hello,

>

>OK, I can't find one program or calculator that will properly calculate

the value of a rolled plate cap.

>

>I tried the calculator on the Tesla Coil Madness page and it gives half

values too.  If I change the number of plate to 3 rather that 2 it will

give me the correct answer.  It also gives the wrong value for a cap match

with a neon.

>

>Am I missing something??

Maybe. Both formulas are correct. The calculator on Jerry Gore's

page is set up for plate caps. You double the value for a rolled cap.

The formula you are using is for plate caps. You double the

result for a rolled cap.

I also tried the cap match for a neon calculator and it gave

me the correct answer.

For a 12kv/30 ma matching cap is .0066314 mfd. Looks right to me.

TTYL!

Mike Hammer

mhammer@misslink.net

Sender: tesl-err@emachine.com

Date: Wed, 06 May 1998 22:04:03 -0400

From: Steve Falco <sfalco@worldnet.att.net>

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: T2 Coil Tuning and more Spark Gap Questions

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Bill the arcstarter wrote:

>

> Steve Falco wrote:

> >

> >I filed oblong holes to get the adjustment range, and did *not* use

> >epoxy, so I could remove the tubing for easier cleaning and adjustment.

> >I recognize that this defeats the purpose of the "never needs

> >adjustment" concept, but I'm happier with it my way.  (Actually, I

> >started with epoxy initially, but found that it didn't adhere well to

> >the PVC (even with sanding), so I cleaned it up and started over

> >without.)

>

> Hmm.  I tried this before epoxying the tubes down but had a very

> difficult time keeping the pipe edges parallel.

>

> Perhaps there is a trick you could share with us for doing this?  How

> long does it take you to reinstall and adjust the gaps after cleaning?

> Having removable gaps would be great for cleaning purposes...

I used washers on both the inside and outside of the PVC with the

non-burred sides towards the plastic - this seems to help with the

adjustability.  I used a feeler gauge, and gradually/alternately

tightened both bolts.  Sometimes it took a few tries to get a good

adjustment.  As to cleaning, when I was using a 12kv neon, I lost the

transformer before I had to clean the gaps :(.  Now I am using the H&R

transformers, which fry the gap pretty quickly because of the high

current.  I also had to set the gaps very close because of the low

voltage, which causes them to foul more quickly.  Now I am building a

synchronous rotary.  I may use the static gaps too, but this is not

definite yet.

By the way, for folks who want to build a synchronous gap, Hosfelt has a

1 hp new-surplus induction start/induction run treadmill motor for $25

that can be converted to 1800 rpm (sleeve bearings, nice 3.5" long 1/2"

shaft, 2.5" round rubber mounts for easy phase angle adjustments!).  It

is not a trivial conversion, but I just succeeded in doing this on my

first attempt (with lots of hints and advice from John Freau), so it can

be done.  Hosfelt is at  888-264-6464 and the part number is 35-151.

Beware - the motor ends are spot welded on, and you have to carefully

drill and chisel the metal to get it apart.  A good drill press is

recommended.  You also have to drill and use sheet metal screws to put

it back together when you are done.

Following John's recommendation, I filed 4 flats on the rotor at 90

degrees apart, and about 1.25" wide.  The motor syncs up solid and has

power to spare.  Windings are aluminum, so you have to be careful not to

damage them because they are not solderable/repairable.

Incidentally, I made the flats by a combination of filing, drilling,

belt sanding, and grinding (throw everything you have at it, I always

say!).

Steve Falco

From: ralph.hartwell@emachine.com (Ralph Hartwell)

Subject:  Re: T2 Coil Tuning and more Spark Gap Questions

Date: Thu,  7 May 1998 02:29:00 GMT

Organization: The Energy Machine Information System 504-733-3462

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

> A feeler gauge is a set of precision metal shims of various

>thicknesses.  This tool is used for various machine shop tasks, and also

> But I don't think you really need one for building your RQ gap.  I

>drove over the "Dollar Store" - (home of some worthless Taiwan tools)

>and bought 3 or 4 of the cheapest hacksaw blades I could find.

>  I then cut them on the chopsaw into convenient three inch lengths.

> Oh - unless you have a grinder or a cutoff wheel tool - you won't be

>able to cut through these blades as they are tempered and/or high

>carbon...

Just heat them red hot over the kitchen stove, then let them cool

down normally.  That removes the hardness.  I usually just clamp them in

a bench vise and then snap them off at whatever length I need.  Wear

heavy gloves and a face shield, please, if you do that.  Wrapping the

exposed part of the blade in a cloth also helps to prevent fragments

from flying everywhere if the blade shatters.

Ralph

---

� QMPro 1.53 � I don't suffer from insanity, I enjoy every minute of it.

X-Mailer: IPAD 2.0

Date: Wed, 06 May 1998 22:04:16 -0700

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: MIKE D <gordrew@home.com>

Subject: Wire Guage

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

I was wondering what guage the wire connecting the different parts

(gap->cap->etc) should be in a 12kv/30ma coil.

Thanks,

Mike Drew

X-Mailer: IPAD 2.0

Date: Thu, 07 May 1998 02:10:23 -0400

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Jim Ryan <jryan@exis.net>

Subject: WIRE

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Hi All, I did some calling today and got prices for #24 wire

that ranged from $7 to $26 per pound. The $7 was from a motor

rewind company. It pays to shop around.

X-Mailer: IPAD 2.0

Date: Thu, 07 May 1998 01:43:01 -0700

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: bob golding <yubba@clara.net>

Subject: Re: T2 Spark Gap\feeler guage Questions

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

At 15:25 06/05/98 -0500, you wrote:

>At 12:26 PM 5/6/98 -0600, you wrote:

>>snip

>>Mike Hammer wrote:

>A hacksaw blade seems a little thick to me. Perhaps

>someone with a micromenter could measure a couple.

>The one Im looking at now I guesstimate at about .04 inch.

>You want your gaps set about .025-.03 inch.

Hi mike

I just measured a well used hacksaw blade  and came up with 0.024" across

the back not counting the teeth. I remember when I was in the motor repair

trade it was common practice to grind the teeth off a 1" section of blade

and fix it in a metal clamp to use for undercutting commutators on large dc

motors wich generally had 0.025" mica between the segments.

bob

Date: Thu, 7 May 1998 06:05:37 -0400 (EDT)

From: Paul Anderson <madhobby@geeky1.ebtech.net>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: T2 Coil Tuning and more Spark Gap Questions

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

On Wed, 6 May 1998, Daniel Lockhart wrote:

>

> Okay, now I feel real dumb. What is a "feeler gauge?"

>

A feeler gauge is a basically a peice of sheet-metal the thickness you

want to measure.  TTYL!

---

Paul Anderson

madhobby@geeky1.ebtech.net

Date: Thu, 7 May 1998 06:10:50 -0400 (EDT)

From: Paul Anderson <madhobby@geeky1.ebtech.net>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: T2 Spark Gap\feeler guage Questions

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

On Wed, 6 May 1998, Mike Hammer wrote:

>

> The ones I have are made by The L.S. Starret Co.

> They should be available anywhere machinists tools

> are available.

>

Nice 'uns.  Starrett always makes good stuff...  You should check by

http://www.travers.com

And order a free catalog - full colour pictures, great fun to browse.

Most likely the prettiest catalog in the industry.  TTYL!

---

Paul Anderson

madhobby@geeky1.ebtech.net

Date: Thu, 7 May 1998 06:15:10 -0400 (EDT)

From: Paul Anderson <madhobby@geeky1.ebtech.net>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: T2 Coil Tuning and more Spark Gap Questions

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

On Wed, 6 May 1998, Bill the arcstarter wrote:

>

>  Oh - unless you have a grinder or a cutoff wheel tool - you won't be

> able to cut through these blades as they are tempered and/or high

> carbon...

>

The high carbon makes it so they can be brought to a high temper.  You

should be able to anneal them as necessary - use a propane torch to heat

the necessary spot to a cherry red(IIRC), and let it cool on it's own.

TTYL!

---

Paul Anderson

madhobby@geeky1.ebtech.net

Date: Thu, 7 May 1998 06:17:20 -0400 (EDT)

From: Paul Anderson <madhobby@geeky1.ebtech.net>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Bad info??

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

On Wed, 6 May 1998, Roy Minnich wrote:

>

> Am I missing something??

>

I'm going to carefully wager a guess here...  Maybe it's that the cap is

rolled?  Consider: Say you have a flat cap with 20 plates.  Take that cap

and roll it.  If you check the cross-section, you would count 40 plates -

unless I'm totally off here, in which case ignore what I just said.

---

Paul Anderson

madhobby@geeky1.ebtech.net

Date: Thu, 07 May 1998 04:14:10 -0700

From: Eric Davidson <edavidson@icva.gov>

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: OBIT

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Roy,

That would be Oil Burner Ignition Transformer. A question for you: what

is that file that is attached to your post? I have been getting those on

several messages from the Tesla list. It says: application/ms-tnef. Can

you or anyone help with this? Thanks.

Roy Minnich wrote:

>

> Hello all,

>

> What's an OBIT?

>

> Thanks,

> Roy Minnich

>

--

Have a good day!

Eric Davidson

edavidson@icva.gov

From: Roy Minnich <rminnich@rolling-meadows.lib.il.us>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Attachments

Date: Thu, 7 May 1998 19:42:23 -0500

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

I talked to the LAN guy here at work and he says that the attachments can't be removed.  Sorry everybody I'll post as little as possible.

Roy Minnich

X-Mailer: IPAD 2.0

Date: Thu, 07 May 1998 19:50:37 -0500

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: Wire Guage

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

At 10:04 PM 5/6/98 -0700, you wrote:

>

>I was wondering what guage the wire connecting the different parts

>(gap->cap->etc) should be in a 12kv/30ma coil.

>

>Thanks,

>Mike Drew

Mike,

For connecting the main spark gap, capacitor, and primary coil

think big. Like in terms of what your primary coil is wound

with. This area carries very high RF currents and the resistance

should be kept low to minimize loss. Connections should be

well soldered or bolted. Eliminate any sharp points that could

induce corona and arcing.

For my own coil I use solid AWG 6 copper. The primary tap

lead needs to be flexible so it consists of a 18 inch length

of stranded AWG 8.

From the neon to safety gap and RF chokes to the main gap

you can use almost anything as the current therefore the losses

are low. Neon sign wire works well here.

TTYL!

Mike Hammer

mhammer@misslink.net

X-Mailer: IPAD 2.0

Date: Thu, 07 May 1998 20:49:13 -0500

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: Attachments

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

At 07:42 PM 5/7/98 -0500, you wrote:

>I talked to the LAN guy here at work and he says that the attachments

can't be removed.  Sorry everybody I'll post as little as possible.

>

>Roy Minnich

>

Roy,

No. Dont do that. The attachments are one of the really stupid

"features" of Microsoft Mail and you cant do anything about it

if your company requires you to use it. It could be far worse.

You could be using Lotus Notes or heaven forbid cc:mail.

If you have the choice to run a different mail program

then try Eudora. Even the free version runs rings around

most everything else.

The attachments are not that big a problem so

continue to post as needed and dont worry about it.

TTYL!

Mike Hammer

mhammer@misslink.net

From: "Carl Miles" <tronrider@email.msn.com>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: 24 & 26 GA Wire

Date: Thu, 7 May 1998 21:36:02 -0500

X-Mailer: IPAD 2.0

X-MimeOLE: Produced By Microsoft MimeOLE V4.72.2106.4

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

I dont know if you can buy magnetic wire but I do know that you can

magnetize metal by rubbing a magnet on it one way.  (Not back and forth)

-----Original Message-----

From: Jim Ryan <jryan@exis.net>

To: Tesla-2 Mailing List <tesla-2@emachine.com>

Date: Wednesday, May 06, 1998 1:32 PM

Subject: 24 & 26 GA Wire

>Hello, I have another question. Is there a site on the Net

>where one can order magnetic wire?

>

>Thanks

>

>Jim

Date: Thu, 7 May 1998 22:49:18 -0400 (EDT)

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: fxphoto@centuryinter.net (fxphoto)

Subject: Re: T2 Coil Tuning and more Spark Gap Questions

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Dan,

I used some scrap aluminum flashing left over from building my capacitor.

The aluminum flashing measured 0.014 inches thick so I cut two strips to fit

between the copper tubes as spacers while the epoxy dried. This gave me a

gap of 0.028 when I removed the aluminum.

Bill Turbett

>Okay, now I feel real dumb. What is a "feeler gauge?" I haven't a clue on

>what they are. I was planning on just finding something with about the

>right thickness to measure it. Remember, you're talking to a guy who is

>just lucky enough to have a Dremel tool in his shop -- forget anything

>fancy like a drill press or anything (where would I put it!?). Are feeler

>gauges like micrometers or calipers?

>

>Didn't Bill the arcstarter use a hacksaw blade? Can you tell me more about

>this, Bill?

>

>Thanks!,

>

>Daniel Lockhart

>

>

>

>

>

>

Date: Thu, 07 May 1998 19:52:40 -0700

From: Gene Waldo <gwaldo@1stnetusa.com>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Organization: Self

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Primary configuration?

Sender: tesl-err@emachine.com

To Mike Hammer et al,

We will be using 5/16" cu tube for our flat pri winding, and will shoot

for max (~17)turns. What part does the turns spacing play, in general

terms? Just far apart enough to allow convenient tapping, or further?

Can a given coil serve 2 NPS systems? We curently have 7.5kv/30 and

12kv/30 at our disposal.

For optimized output, should each NPS have its' own pri/sec setup, or

can one set of coils be tuned for each, and the various terminals which

may be applied?  The pan of alcohol thing sounds neat! Sparks flambe!

Don't set your hair on fire!

Speaking of NEAT, what are the prospects for two tesla systems, sideby

side, with 180 deg phasing?

Just curious....

-----------------------------------------------------

Gene Waldo -  gwaldo@1stnetusa.com  - "Igor in Residence"

---------------------------------------------------------------

What's good for the planet is more OZONE! See the machinery form up!

NEW STUFF!   http://www.misslink.net/mhammer/waldo.htm

X-Mailer: IPAD 2.0

Date: Fri, 08 May 1998 00:40:34 -0400

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Jim Ryan <jryan@exis.net>

Subject: CAPS

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

I just a couple of questions and one remark.

12k 30 ma. trans  4.5"x21" secondary 15 turn primary .

1. I going to use 2 .005" 6"x90" strips of brass shim stock and

poly insulation .030 thick 5 layers x .006 on my rolled cap.

I calculated that the finished diameter will be 2.80" if I

roll it on a 1/2" dowel. The question is, if the calculated

diam. has .2" space in a 3" PVC pipe should I wrap more poly

on the bundle to keep it tight?

2. How do you pull the air out of the oil with the cap leads

attached/installed? Do you epoxy seal them first and them

pull the Vacuum?

3. Is there any sort of safety valve one can make or buy so if

something goes wrong PVC won't blow the windows out ?

Comment: Attachments in any email program are optional.

if you open the one up that has been posted on this group

it will flip your screen upside down. Funny once, but

10 times is to much !

Thanks to all for the help so far. If you need any help with

fireworks let me know via email NO posting on this group.

Jim

X-Mailer: IPAD 2.0

Date: Fri, 08 May 1998 01:00:52 -0400

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Jim Ryan <jryan@exis.net>

Subject: Re: 24 & 26 GA Wire

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

At 09:36 PM 5/7/98 -0500, you wrote:

>I dont know if you can buy magnetic wire but I do know that you can

>magnetize metal by rubbing a magnet on it one way.  (Not back and forth)

>-----Original Message-----

>From: Jim Ryan <jryan@exis.net>

>To: Tesla-2 Mailing List <tesla-2@emachine.com>

>Date: Wednesday, May 06, 1998 1:32 PM

>Subject: 24 & 26 GA Wire

Somebody replied to my question about magnetic wire but didn't leave their

name.

I asked if anyone knew of a place were you can buy magnetic wire?

They answered:

>"I dont know if you can buy magnetic wire but I do know that you can

>magnetize metal by rubbing a magnet on it one way.  (Not back and forth)"

Yep, you must be some sort of Physics teacher or something like that.

You are a really smart guy !

Thanks

Jim

X-Mailer: IPAD 2.0

Date: Fri, 08 May 1998 00:45:30 -0700

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: bob golding <yubba@clara.net>

Subject: Re: T2 Coil Tuning and more Spark Gap Questions

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

At 22:04 06/05/98 -0400, you wrote:

snip

>> Steve Falco wrote:

>Following John's recommendation, I filed 4 flats on the rotor at 90

>degrees apart, and about 1.25" wide.  The motor syncs up solid and has

>power to spare.  Windings are aluminum, so you have to be careful not to

>damage them because they are not solderable/repairable.

hi all.

I dont understand. what does filing flats on the rotor achieve. Are these

dual wound cap start motors with centrifugal switches, or normal shaded

pole type motors. Can this trick be used on 6" bench grinder motors?. many

thanks.

bob golding lightning without rain

bob yubba@clara.net

X-Mailer: IPAD 2.0

Date: Fri, 08 May 1998 01:09:32 -0700

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: bob golding <yubba@clara.net>

Subject: UK wire supplier

Cc: tesla-2@emachine.com

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

hi all.

Just finished winding my 4" secondary. It looks good.

Will wind the 7" coil form tomorrow while I have the winding jig set up.

After waiting for the local rewind shop to not come up with any wire they

put me on to one of thier suppliers. I had to buy 4.5 Kg's minimum order

but I will probably use it all. They said they would be happy for me to put

thier name on the list and are quite happy to supply this quantify to

individuals. So if any UK coilers are having trouble getting wire here is

their address.

Wire Electric Supplies Ltd

Cooper  St

Monmore Green

Wolverhampton

WV2 2JL

telephone 01902 351700

They take personal cheques but not credit cards and deliver within 2 days.

4.5Kg's .6 mm �30.60p inclusive.

hope this helps.

bob golding lightning without rain

bob yubba@clara.net

From: wwl@netcomuk.co.uk (Mike Harrison)

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: UK wire supplier

Date: Fri, 08 May 1998 10:49:39 GMT

Organization: White Wing Logic

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

On Fri, 08 May 1998 01:09:32 -0700, you wrote:

>hi all.

>	Just finished winding my 4" secondary. It looks good.

>Will wind the 7" coil form tomorrow while I have the winding jig set up.

>After waiting for the local rewind shop to not come up with any wire they

>put me on to one of thier suppliers. I had to buy 4.5 Kg's minimum order

>but I will probably use it all. They said they would be happy for me to put

>thier name on the list and are quite happy to supply this quantify to

>individuals. So if any UK coilers are having trouble getting wire here is

>their address.

>

>Wire Electric Supplies Ltd

>	Cooper  St

>	Monmore Green

>	Wolverhampton

>		WV2 2JL

>

>	telephone 01902 351700

>

>They take personal cheques but not credit cards and deliver within 2 days.

>4.5Kg's .6 mm �30.60p inclusive.

>hope this helps.

>

>bob golding lightning without rain

>

>bob yubba@clara.net

You can get smaller quantites from the Scientific Wire Company,

18 Raven Road, London E17 1HW tel. 0181 505 0002, fax 559 1114

They do every type of wire imaginable -send them an SAE for a

catalogue,

They do a special high-voltage grade called HT200 with thicker enamel

for about �1/KG extra, not listed in the catalogue.

____                                                           ____

_/ L_/  Mike Harrison / White Wing Logic / wwl@netcomuk.co.uk  _/ L_/

_/ W_/  Hardware & Software design / PCB Design / Consultancy  _/ W_/

/_W_/  Industrial / Computer Peripherals / Hazardous Area      /_W_/

Comments: Authenticated sender is <alfred.skrocki@cybernetworking.com>

From: "Alfred A. Skrocki" <alfred.skrocki@cybernetworking.com>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Date: Fri, 8 May 1998 16:43:27 +0500

Subject: Re: OBIT

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

On Wed, 6 May 1998 15:21:30 -0500 In Tesla-2

Roy Minnich <rminnich@rolling-meadows.lib.il.us> wrote;

> Hello all,

>

> What's an OBIT?

Roy, by now many people have probably told you that an OBIT is an Oil

Burner Ignition Transformer. OBIT are not as bad as some people make them

out to be. OBITS typically put out 10Kv at 23 ma. and can be connected in

parallel to get much higher power levels, for example 10 OBITS in parallel

will give you over 2 killowatts! Make sure you NEVER connect OBITS in

series or you will surely burn out their secondaries! Use lots of

transformer protection ie. choke and by-pass capacitors and NEVER use them

with a rotary spark gap UNLESS it is syncronous.

Sincerely

Alfred A. Skrocki

Alfred.Skrocki.Sr@JUNO.com

alfred.skrocki@cybernetworking.com

From: "Chris Watkins" <cwatkins@seidata.com>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: T2   FILE ATTACHMENTS

Date: Fri, 8 May 1998 07:55:47 -0400

X-Mailer: IPAD 2.0

X-MimeOLE: Produced By Microsoft MimeOLE V4.72.2106.4

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Hi all

I use Outlook Express , dont know if mine sends attachments or not

but I usually make sure to send in plain text format.

Guess you all will see if it has attachments..

just my 2 cents worth

Chris W

To:	"Tesla-2" <tesla-2@emachine.com>

Date:	Fri, 08 May 1998 05:07:57 -0700

From:	"Leben Lang und Frei" <fleischwulf@mailexcite.com>

X-Sent-Mail: off

X-Mailer: IPAD 2.0

Subject: 7.2 Kw continous service power supply

X-Sender-Ip: 208.153.7.9

Organization: MailExcite  (http://www.mailexcite.com:80)

Date: Fri, 08 May 1998 07:09:11 -0500

From: fleischwulf@mailexcite.com

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Hey everyone,

I finally discovered the perfect inductive ballast for the 14400 pig. I'm reading

.5 amps delivered on the high voltage side of the transformer. For control I feed

the the pig 240 v at 30 amp via a large variac. The ballast is a 440 v power application

transformer. It is rated for 8 kw of continous service. My current capacitance is

.026 uf. Will I have to increase this to acheive a better match between the cap and

the transformer ? Will I need to use a PFC ?

Thanks for everything

Paul

---

Einigkeit und Freiheit

X-Originating-IP: [199.217.140.22]

From: "Bill the arcstarter" <arcstarter@hotmail.com>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: 24 & 26 GA Wire

Date: Fri, 08 May 1998 06:08:05 PDT

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

It was written:

>>"I dont know if you can buy magnetic wire but I do know that you can

>>magnetize metal by rubbing a magnet on it one way.  (Not back and

forth)"

Only if the metal is iron/nickel/cobalt or similar alloy.

You can rub a piece of copper with a magnet all day and it won't become

magnetic.

-Bill the arcstarter

Starting arcs in Cinci, OH

http://www.geocities.com/CapeCanaveral/Hangar/6160

Date: Fri, 8 May 1998 09:46:40 -0400

From: Aric_C_Rothman@email.whirlpool.com (Aric C Rothman)

Subject: Re[2]: 24 & 26 GA Wire

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

Magnet wire can be had at a very good price from Electro Energy, Inc.

See their site at http://www.electroenergy.com

I purchased a 10lb spool of high temperature wire for about $35.00.

Since copper prices fluctuate, you'll have to call for a quote when

you want to make an order.

Talk to Grant Niemann at 800-551-9474.  He's a great guy.

Aric

X-Mailer: IPAD 2.0

Date: Fri, 08 May 1998 12:17:01 -0400

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Jim Ryan <jryan@exis.net>

Subject: Questions & Comments

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

I just a couple of questions and one remark.

12k 30 ma. trans  4.5"x21" secondary 15 turn primary .

1. I going to use 2 .005" 6"x90" strips of brass shim stock and

poly insulation .030 thick 5 layers x .006 on my rolled cap.

I calculated that the finished diameter will be 2.80" if I

roll it on a 1/2" dowel. The question is, if the calculated

diam. has .2" space in a 3" PVC pipe should I wrap more poly

on the bundle to keep it tight?

2. How do you pull the air out of the oil with the cap leads

attached/installed? Do you epoxy seal them first and them

pull the Vacuum?

3. Is there any sort of safety valve one can make or buy so if

something goes wrong PVC won't blow the windows out ?

Comment: Attachments in any email program are optional.

if you open the one up that has been posted on this group

it will flip your screen upside down. Funny once, but

10 times is to much !

Thanks to all for the help so far. If you need any help with

fireworks let me know via email NO posting on this group.

Jim

X-Mailer: IPAD 2.0

Date: Fri, 08 May 1998 11:47:36 -0500

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: Primary configuration?

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

At 07:52 PM 5/7/98 -0700, you wrote:

>To Mike Hammer et al,

>We will be using 5/16" cu tube for our flat pri winding, and will shoot

>for max (~17)turns. What part does the turns spacing play, in general

>terms? Just far apart enough to allow convenient tapping, or further?

In general tighter spacing gives somewhat higher inductance.

But the spacing needs to be great enough to discourage

turn to turn arcing. 1/4 inch spacing is usually adequate

though slightly more may be necessary with 15kv systems.

You dont want over 1/2 inch turn to turn spacing.

>Can a given coil serve 2 NPS systems? We curently have 7.5kv/30 and

>12kv/30 at our disposal.

Sure. I often run my coil from various neons for different purposes.

If its long sparks Im after I use the big 12kv/100ma neon.

For tuning and setup I use a 9kv/60 and sometimes a 7500/30.

One thing to remember is to be sure each neon you use can charge

the primary cap in your system.

Ill paste in a little chart I made some time ago for reference.

VA 	 Impedance	 Matching Capacitor

7500v @  30MA	225	 250K		 .0106uf

9KV   @  30MA	270 	 300K	       .0088uF

9KV   @  60MA	540	 150K	       .0177uF

9KV   @  90MA     810    100K        .0265uf

12KV  @  30MA	360	 400K	       .0066uF

12KV  @  60MA	720	 200K	       .0133uF

12KV  @  90MA	1080	 133.33K  	 .0199uF

12KV  @ 120MA     1440   100K        .0265uf

15KV  @  30MA	450	 500K	       .0053uF

15KV  @  60MA	900	 250K	       .0106uF

15KV  @  90MA	1350	 166.66K 	 .0159uF

15KV  @ 120MA     1800   125K        .0212uf

>For optimized output, should each NPS have its' own pri/sec setup, or

>can one set of coils be tuned for each, and the various terminals which

>may be applied?

The only consideration is the primary cap and the main gap setting.

Less gap for lower voltages.

>Speaking of NEAT, what are the prospects for two tesla systems, sideby

>side, with 180 deg phasing?

>

Its been done. Robert W. Stevens built a truly impressive coil

of this type. Check out this picture of his system at:

ftp://nic.funet.fi/pub/sci/electrical/tesla/pictures/rws/twin2rws.jpg

TTYL!

Mike Hammer

mhammer@misslink.net

X-Mailer: IPAD 2.0

Date: Fri, 08 May 1998 12:00:12 -0500

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: 7.2 Kw continous service power supply

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

At 05:07 AM 5/8/98 -0700, you wrote:

>Hey everyone,

>

>I finally discovered the perfect inductive ballast for the 14400 pig.

>I'm reading .5 amps delivered on the high voltage side of the transformer.

>For control I feed the the pig 240 v at 30 amp via a large variac.

>The ballast is a 440 v power application transformer.

>It is rated for 8 kw of continous service.

Paul if I understand what you are doing then this is similar

to the system being used by Gary Weaver. Gary is using some

large surplus control transformers as ballasting on his large coil.

Gary has a diagram of this on his web page. Take a look at:

http://home.earthlink.net/~gweaver/current.htm

> My current capacitance is .026 uf. Will I have to increase this

> to acheive a better match between the cap and the transformer?

No. Transformer cap matching is really only for neons or

other current limited transformers. The matching serves

to circumvent the current limiting allowing more current

to be drawn. With a pole pig you have far more current

than you need. The high current capability allows your

cap to be charged and recharged several times per 1/2 cycle.

This increases power throughput. You can increase the throughput

even more by increasing the firing rate of your gap. But be

careful doing this. High break rates can literally beat your

cap to death.

>Will I need to use a PFC ?

PFC (power factor correction) is only necessary if the total

current draw is exceeding the capability of your control

system. (variac) If your variac and ballasting can handle

the load then there is nothing to be gained by adding PFC caps.

>

>Thanks for everything

>

>Paul

>---

>Einigkeit und Freiheit

TTYL!

Mike Hammer

mhammer@misslink.net

X-Mailer: IPAD 2.0

Date: Fri, 08 May 1998 12:01:37 -0500

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: T2   FILE ATTACHMENTS

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

At 07:55 AM 5/8/98 -0400, you wrote:

>Hi all

>I use Outlook Express , dont know if mine sends attachments or not

>but I usually make sure to send in plain text format.

>

>Guess you all will see if it has attachments..

>

>just my 2 cents worth

>

>Chris W

>

No attachments Chris. You are socially acceptable. :)

TTYL!

Mike Hammer

mhammer@misslink.net

Date: Fri, 08 May 1998 10:24:09 -0700

From: Gene Waldo <gwaldo@1stnetusa.com>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Organization: Self

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Primary configuration?

Sender: tesl-err@emachine.com

Mike Hammer wrote:

>

> At 07:52 PM 5/7/98 -0700, you wrote:

> >To Mike Hammer et al,

> >We will be using 5/16" cu tube for our flat pri winding, and will

SNIPS!

>

> The only consideration is the primary cap and the main gap setting.

> Less gap for lower voltages.

I have an idea for a simple 'tunable-on-the fly' blown main gap.  Will

have a concept sketch up to you in a bit... for critique.  If this

sounds dumb or futile initially, let me know and I'll cease. It should

also facilitate periodic cleanings....   It may also, no doubt, have

been tried, but I haven't seen reference to anything like it.

> >Speaking of NEAT, what are the prospects for two tesla systems, sideby

> >side, with 180 deg phasing?

>

> Its been done. Robert W. Stevens built a truly impressive coil

> of this type. Check out this picture of his system at:

> ftp://nic.funet.fi/pub/sci/electrical/tesla/pictures/rws/twin2rws.jpg

Unable to access initial tries.... Something about confusion at their

end??...  "Try again"

> TTYL!

>

> Mike Hammer

> mhammer@misslink.net

Thanks for the chart!

--

Gene Waldo -  gwaldo@1stnetusa.com  - "Igor in Residence"

---------------------------------------------------------------

What's good for the planet is more OZONE! See the machinery form up!

NEW STUFF!   http://www.misslink.net/mhammer/waldo.htm

From: <Tom.Stock@gsbsc.gensig.com>

Date: Fri, 8 May 1998 13:41:04 -0400

Subject: two coils 180' out of phase

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

To run two coils 180 degrees out of phase, can you simple swap the

imputs to the neon or do you swap the outputs of the neon?

-tom

From: <Tom.Stock@gsbsc.gensig.com>

Date: Fri, 8 May 1998 13:59:25 -0400

Subject: Robert Stevens Coils

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

Where is the primary coil on Robert Stevens' coil??

I see what looks like plastic film wrapped around the base of the

secondary. Is this it??

-tom

X-Originating-IP: [199.217.140.22]

From: "Bill the arcstarter" <arcstarter@hotmail.com>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Extreme service water-cooled capacitors?

Date: Fri, 08 May 1998 11:01:33 PDT

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

It was written:

>This increases power throughput. You can increase the throughput

>even more by increasing the firing rate of your gap. But be

>careful doing this. High break rates can literally beat your

>cap to death.

I've read that some commercial induction heating systems (basically

identical to the front-end of a tesla coil) contain water-cooled

capacitor banks which handle upwards of 1000 amps of RF on a continuous

basis.  The output inductor (similar to our primary coils) is water

cooled and resonates with a cap in order to deliver magnetic energy to

the object being heated.  A small unit is 20 KW.  The largest sizes can

run up to 1 MW.  Surely these caps are putting up with much more abuse

than the ones in our tesla coils!

I have a schematic of one of these machines on my site at:

http://www.geocities.com/CapeCanaveral/Hangar/6160/ind1.html

This is a doubly-tuned LC circuit, very similar to our tesla coils.

Does anyone know anything about how these severe service capacitors

were constructed??  Perhaps we can glean a hint from the past and make

our caps more robust for coiling duty!

Does anyone know how reliable the commercially available (and very

expensive) tesla-coil pulse caps are??

I showed my (yet to be tested) water-cooled tungsten carbide spark gap

to one of our machinists - just to see what he'd say about such a

cobbled-together device.  Here's how the conversation went:

(note - B = Bill and M = machinst)

B) Hey- take a look at this thing I've been building!

m) Oh yea.  I know what that is.

B) Oh really?  What do you think it is?

m) That's a spark gap!

B) Well - um - yes - but how did you know that? (dumbfounded)

m) We used to have one in our old induction heating furnace!

I was completely amazed since I'd never told him what I was doing!

Perhaps I'm on the right track afterall??  Old machinists say the

darndest things...

-Bill the arcstarter

Starting arcs in Cinci, OH

http://www.geocities.com/CapeCanaveral/Hangar/6160

X-Mailer: IPAD 2.0

Date: Fri, 08 May 1998 14:27:03 -0500

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: Robert Stevens Coils

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

At 01:59 PM 5/8/98 -0400, you wrote:

>

>

>     Where is the primary coil on Robert Stevens' coil??

>

>     I see what looks like plastic film wrapped around the base of the

>     secondary. Is this it??

>

>     -tom

Tom I dont know the specifics of Roberts coil but remember

him talking about it on another list some time ago.

If Im not mistaken the coils are setup in a magnifier

configuration. This has the RF energy coupled to the base

of the secondary coils via a transmission line rather than

magnetically coupled.

The primary/secondary of the magnifier setup can then be located

some distance away from the output resonator or tertiary coil.

The tertiary then is free to ring up without interference

from any magnetically coupled elements.

This offers some advantages in large coils but really isnt much

of an advantage in small to medium coils.

The plastic film is probably mylar or poly used to prevent

arcing.

TTYL!

Mike Hammer

mhammer@misslink.net

X-Mailer: IPAD 2.0

Date: Fri, 08 May 1998 14:34:56 -0500

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: Extreme service water-cooled capacitors?

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

At 11:01 AM 5/8/98 PDT, you wrote:

>It was written:

>>This increases power throughput. You can increase the throughput

>>even more by increasing the firing rate of your gap. But be

>>careful doing this. High break rates can literally beat your

>>cap to death.

>

> I've read that some commercial induction heating systems (basically

>identical to the front-end of a tesla coil) contain water-cooled

>capacitor banks which handle upwards of 1000 amps of RF on a continuous

>basis.  The output inductor (similar to our primary coils) is water

>cooled and resonates with a cap in order to deliver magnetic energy to

>the object being heated.  A small unit is 20 KW.  The largest sizes can

>run up to 1 MW.  Surely these caps are putting up with much more abuse

>than the ones in our tesla coils!

That would help in the case of dielectric heating but it doesnt

help the actual physical hammering of the plates due to the

pulsing action of the Tesla tank circuit.

>

>

> Does anyone know how reliable the commercially available (and very

>expensive) tesla-coil pulse caps are??

No more or less reliable than any other cap. Several people

had rather spectacular explosive failures of some pulse

caps from Condenser Products that were designed for

Tesla coil service. It all depends on how you treat them.

TTYL!

Mike Hammer

mhammer@misslink.net

From: "Eriksson" <jan-erik.eriksson@mbox2.swipnet.se>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: SV: OBIT

Date: Fri, 8 May 1998 21:59:35 +0200

X-Mailer: IPAD 2.0

X-MimeOLE: Produced By Microsoft MimeOLE V4.72.2106.4

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Fr�n: Roy Minnich <rminnich@rolling-meadows.lib.il.us>

Till: Tesla-2 Mailing List <tesla-2@emachine.com>

Datum: den 6 maj 1998 23:06

�mne: OBIT

>Hello all,

>

>What's an OBIT?

>

Oil burner transformer.

/Christopher Eriksson

From: <Tom.Stock@gsbsc.gensig.com>

Date: Fri, 8 May 1998 16:01:20 -0400

Subject: Inductance? What's the difference?

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

Why do air core chokes need series resistors, and ferrite core chokes

do not?

-tom

X-Originating-IP: [199.217.140.22]

From: "Bill the arcstarter" <arcstarter@hotmail.com>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Inductance? What's the difference?

Date: Fri, 08 May 1998 13:36:25 PDT

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

It was written:

>     Why do air core chokes need series resistors, and ferrite core

>chokes do not?

Chokes wound on ferrite bars will exhibit a bit of energy loss due to

various effects of the ferrite (eddy currents, magnetic hysteresis,

etc).

This energy will show up as heat in the ferrite rod.

Air-wound chokes don't have this energy loss and thus are more likely to

"ring" themselves.  If you are especially unlucky - your choke will ring

at a frequency close to the tank circuit frequency - and produce some

really high voltages in the process!

The resistors (used with air-wound chokes) serve to dampen out these

(potential) oscillations.  Technically - it spoils the "Q" of the choke.

I hope this helps!

-Bill the arcstarter

Starting arcs in Cinci, OH

http://www.geocities.com/CapeCanaveral/Hangar/6160

Date: Fri, 8 May 1998 17:33:00 -0400 (EDT)

From: Paul Anderson <madhobby@geeky1.ebtech.net>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: 24 & 26 GA Wire

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

On Fri, 8 May 1998, Jim Ryan wrote:

>

> Yep, you must be some sort of Physics teacher or something like that.

> You are a really smart guy !

>

If you're looking for wire for a tesla coil secondary, it's not MAGNETIC

wire, it's MAGNET wire.  Double formvar magnet wire, to be precise.

You said you wanted wire that was magnetic.  TTYL!

---

Paul Anderson

madhobby@geeky1.ebtech.net

Date: Fri, 8 May 1998 17:34:42 -0400 (EDT)

From: Paul Anderson <madhobby@geeky1.ebtech.net>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: CAPS

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

On Fri, 8 May 1998, Jim Ryan wrote:

>

> 2. How do you pull the air out of the oil with the cap leads

>    attached/installed? Do you epoxy seal them first and them

>    pull the Vacuum?

>

To use a vacuum to keep air out of the cap, you pull the vacuum, THEN pour

the oil in the cap, -THEN- bring it up to atmospheric.  You don't suck the

air out.  TTYL!

---

Paul Anderson

madhobby@geeky1.ebtech.net

X-Mailer: IPAD 2.0

Date: Fri, 08 May 1998 17:00:10 -0500

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: Inductance? What's the difference?

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

At 04:01 PM 5/8/98 -0400, you wrote:

>

>

>     Why do air core chokes need series resistors, and ferrite core chokes

>     do not?

>

>     -tom

The series resistors for RF chokes are needed to lower the Q

of the chokes and help dampen oscillations. Chokes wound on

ferrite cores sometimes do need this dampening action but

generally do not. Ferrite chokes usually will have far fewer

turns of wire and the core itself introduces some losses.

An air core choke is usually higher Q and has many more

turns of wire to achieve useful inductance. They can have

350-400 turns of wire or more. Almost like a small secondary

coil. And thats exactly what they are. A Tesla secondary

being shock excited by pulses of energy. These pulses can

cause them to ring up just like a Tesla coil secondary.

The voltage spikes can easily be 50kv or more.

By inserting a series resistor this keeps the Q low and

any oscillations quickly die out.

A quick test you can make to see if your chokes are effective

is to fire your coil normally and notice the level of activity

at your safety gap. If the safety is not firing then close it

down slightly until it begins to fire. Now turn off the coil

and short out your chokes to remove them from the circuit.

Fire up your coil again and note what the safety gap is doing now.

If you have less intense safety gap firings with the chokes

in place then they are doing their job. If you have more

activity with the chokes than without then your chokes are ringing

and probably cuasing more harm than good. Add some resistors

or change to a ferrite choke.

TTYL!

Mike Hammer

mhammer@misslink.net

Date: Fri, 08 May 1998 19:10:33 -0500

From: Bert Hickman <bert.hickman@aquila.com>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Organization: Stoneridge Engineering

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: two coils 180' out of phase

Sender: tesl-err@emachine.com

Tom.Stock@gsbsc.gensig.com wrote:

>

>

>      To run two coils 180 degrees out of phase, can you simple swap the

>      imputs to the neon or do you swap the outputs of the neon?

>

>      -tom

Tom,

Neither. The key is connecting the Tesla Coil tank circuit primaries in

series, but in opposite phase, and using a common tank capacitor and gap

to drive them at the same time. The secondaries must be identical and

tuned to the same resonant frequency. Once you meet these balancing

conditions, the effects are awesome! If each coil puts out 5 footers, 10

foot streamers will jump between the two...

-- Bert --

Date: Fri, 08 May 1998 19:17:58 -0500

From: Bert Hickman <bert.hickman@aquila.com>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Organization: Stoneridge Engineering

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Robert Stevens Coils

Sender: tesl-err@emachine.com

Tom.Stock@gsbsc.gensig.com wrote:

>

>

>

>      Where is the primary coil on Robert Stevens' coil??

>

>      I see what looks like plastic film wrapped around the base of the

>      secondary. Is this it??

>

>      -tom

Tom,

If you look carefully, you'll see the primary coils - they are

relatively small and low around the base of each secondary. Each only

has about 5 turns...

-- Bert --

Date: Fri, 08 May 1998 19:19:40 -0500

From: Bert Hickman <bert.hickman@aquila.com>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Organization: Stoneridge Engineering

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Robert Stevens Coils

Sender: tesl-err@emachine.com

Mike Hammer wrote:

>

> At 01:59 PM 5/8/98 -0400, you wrote:

> >

> >

> >     Where is the primary coil on Robert Stevens' coil??

> >

> >     I see what looks like plastic film wrapped around the base of the

> >     secondary. Is this it??

> >

> >     -tom

>

> Tom I dont know the specifics of Roberts coil but remember

> him talking about it on another list some time ago.

> If Im not mistaken the coils are setup in a magnifier

> configuration. This has the RF energy coupled to the base

> of the secondary coils via a transmission line rather than

> magnetically coupled.

>

> The primary/secondary of the magnifier setup can then be located

> some distance away from the output resonator or tertiary coil.

> The tertiary then is free to ring up without interference

> from any magnetically coupled elements.

>

> This offers some advantages in large coils but really isnt much

> of an advantage in small to medium coils.

>

> The plastic film is probably mylar or poly used to prevent

> arcing.

>

> TTYL!

>

> Mike Hammer

> mhammer@misslink.net

Mike and all,

If these are Robert Stephens' systems in Ontario, then these coils are

actually matched twins (standard 2-coil systems) driven with the

primaries in series but out of phase by 180 degrees. A common tank cap

and rotary gap is used to drive both primaries. The secondary & toroids

of each have been adjusted to have the same resonant frequency - when

the top of one is at a positive half-cycle peak, the other is at the

negative half-cycle peak. A very nicely designed system!

Interestingly enough, these coils could be run with NO ground on the

base of the secondaries, but Rob usually runs with a ground to insure

that excessive base voltage swings don't cause secondary:primary

flashovers. The plastic around the base of the secondaries provides a

bit more insulation.

Ed Wingate in Brockport, NY has an even larger twin system that he runs

at upwards of 20 KVA - awesome high power arcs flow between the

secondaries and the noise is literally deafening!

-- Bert --

Sender: tesl-err@emachine.com

Date: Fri, 08 May 1998 20:23:45 -0400

From: Steve Falco <sfalco@worldnet.att.net>

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: T2 Coil Tuning and more Spark Gap Questions

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

bob golding wrote:

>

> At 22:04 06/05/98 -0400, you wrote:

>

> snip

> >> Steve Falco wrote:

> >Following John's recommendation, I filed 4 flats on the rotor at 90

> >degrees apart, and about 1.25" wide.  The motor syncs up solid and has

> >power to spare.  Windings are aluminum, so you have to be careful not to

> >damage them because they are not solderable/repairable.

>

> hi all.

>         I dont understand. what does filing flats on the rotor achieve. Are these

> dual wound cap start motors with centrifugal switches, or normal shaded

> pole type motors. Can this trick be used on 6" bench grinder motors?. many

> thanks.

>

> bob golding lightning without rain

>

> bob yubba@clara.net

Ah, sorry for not being clear.  The motor I described is an induction

start/induction run motor with a centrifugal switch to shut off the

starter winding when the motor starts to speed up.  Nominal running

speed is 1725 RPM, but we really want it to run at 1800 RPM, which makes

it synchronous with the power line.  The reason we want this is: if you

had a wheel with 4 electrodes, turned by an 1800 rpm motor, you would

get pulses at 120 Hz (1800 revolutions per minute divided by 60 seconds

per minute = 30 revolutions per second... So, 30 rps * 4 electrodes =

120 Hz).

Since the power line is 60 Hz (in the US), and there is a positive and a

negative peak to the sine wave, then at 120 Hz, you can fire your coil

each time the voltage hits its peak.  "This is a good thing (TM)".  Of

course, you have to rotate the motor body (or the fixed electrodes) to

adjust the phase angle so you pick the peaks rather than the

zero-crossings ("Bad thing (TM)"), but this is easy to do - just rotate

the motor body for maximum spark, no more than +-45 degrees in either

direction.

This type of motor has 4 poles for the main winding.  If you take the

motor apart, you will see the heavy (main) winding in 4 separate

bunches, 90 degrees apart on the stator (the outer frame of

laminations).  It turns out, that if you file 4 matching flats at 90

degrees apart on the rotor, then the rotor will run locked to the

rotating magnetic field, because the 4 remaining curved parts of the

rotor dominate the magnetic interactions (they are closer, so the field

effect is much stronger on them).  Without the modification, the rotor

is always slipping in the rotating field, which makes it slightly

slower, in this case 1725 RPM.  The slip factor helps develop more

horsepower, which is why motors are built this way, but for our purposes

we can live with less horsepower since we are just spinning a light

electrode wheel with fairly low drag.

Now, I don't know about shaded pole motors - I believe those are the

ones with a deliberate shorted turn in the stator.  If so, they are

generally pretty weak, and I don't imagine they would be suitable for

this modification.  Perhaps someone else on the list can answer that

question for sure though.  I have been told that capacitor

start/induction run motors will work (which makes sense), I have also

been told that capacitor start/capacitor run motors will work.  I have

not tried either of these however.

If any of the above is still not clear, please ask.

Steve Falco

X-Mailer: IPAD 2.0

Date: Fri, 08 May 1998 19:28:07 -0500

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: Questions & Comments

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

At 12:17 PM 5/8/98 -0400, you wrote:

>I just a couple of questions and one remark.

>

>12k 30 ma. trans  4.5"x21" secondary 15 turn primary .

>

>1. I going to use 2 .005" 6"x90" strips of brass shim stock and

>   poly insulation .030 thick 5 layers x .006 on my rolled cap.

>   I calculated that the finished diameter will be 2.80" if I

>   roll it on a 1/2" dowel. The question is, if the calculated

>   diam. has .2" space in a 3" PVC pipe should I wrap more poly

>   on the bundle to keep it tight?

It really doesnt matter. The only reason for filling up the volume

of the tank is to decrease the amount of oil required.

>2. How do you pull the air out of the oil with the cap leads

>   attached/installed? Do you epoxy seal them first and them

>   pull the Vacuum?

I have the cap terminals (1/4 20 bolts) coming out the side of the

PVC pipe tank. The tank sits upright. I sealed the bolts with epoxy

and RTV silicone. The top and bottom covers were 1/4 inch plexiglass

attached with RTV silicone so as to be removable if necessary.

The top cover contains a fitting to attach to my vacuum pump.

A vacuum was pulled to remove as much air as possible then

the tank is backfilled with oil allowing the vacuum to suck

the oil in. Once the pressure equalizes about 90 percent of

the air has been removed. You can then pull a vacuum on the

cap again to bubble out most of the remaining air.

Once I was done with the vacuuming I replaced the top cover

with the vacuum fitting with a plain cover with a tiny

pressure relief hole drilled in it.

If you have the necessary vacuum equipment this is a worthwhile

step as your finished cap needs very little break in time.

>

>3. Is there any sort of safety valve one can make or buy so if

>   something goes wrong PVC won't blow the windows out ?

With the plexiglass top and bottom these will likely crack

before anything serious happens. If your cap fails you will notice

it immediately. I drilled a tiny relief hole in the top covers of

my caps. Poly caps in oil generally wont be expected to explode

although it could happen. The caps Ive destroyed just gurgle

a bit and the coil sputters to a halt.

TTYL!

Mike Hammer

mhammer@misslink.net

X-Mailer: IPAD 2.0

Date: Fri, 08 May 1998 20:36:18 -0500

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: Robert Stevens Coils

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

At 07:19 PM 5/8/98 -0500, you wrote:

>Mike Hammer wrote:

>>

>> At 01:59 PM 5/8/98 -0400, you wrote:

>> >

>> >

>> >     Where is the primary coil on Robert Stevens' coil??

>> >

>> >     I see what looks like plastic film wrapped around the base of the

>> >     secondary. Is this it??

>> >

>> >     -tom

>>

>> Tom I dont know the specifics of Roberts coil but remember

>> him talking about it on another list some time ago.

>> If Im not mistaken the coils are setup in a magnifier

>> configuration. This has the RF energy coupled to the base

>> of the secondary coils via a transmission line rather than

>> magnetically coupled.

>>

>> The primary/secondary of the magnifier setup can then be located

>> some distance away from the output resonator or tertiary coil.

>> The tertiary then is free to ring up without interference

>> from any magnetically coupled elements.

>>

>> This offers some advantages in large coils but really isnt much

>> of an advantage in small to medium coils.

>>

>> The plastic film is probably mylar or poly used to prevent

>> arcing.

>>

>> TTYL!

>>

>> Mike Hammer

>> mhammer@misslink.net

>

>Mike and all,

>

>If these are Robert Stephens' systems in Ontario, then these coils are

>actually matched twins (standard 2-coil systems) driven with the

>primaries in series but out of phase by 180 degrees. A common tank cap

>and rotary gap is used to drive both primaries. The secondary & toroids

>of each have been adjusted to have the same resonant frequency - when

>the top of one is at a positive half-cycle peak, the other is at the

>negative half-cycle peak. A very nicely designed system!

>

>Interestingly enough, these coils could be run with NO ground on the

>base of the secondaries, but Rob usually runs with a ground to insure

>that excessive base voltage swings don't cause secondary:primary

>flashovers. The plastic around the base of the secondaries provides a

>bit more insulation.

>

>Ed Wingate in Brockport, NY has an even larger twin system that he runs

>at upwards of 20 KVA - awesome high power arcs flow between the

>secondaries and the noise is literally deafening!

>

>-- Bert --

Ah. Thats what I get for trying to answer a question

from the few good brain cells I have left.

So Roberts coil is actually a standard two coil system then.

I stand corrected.

TTYL!

Mike Hammer

mhammer@misslink.net

X-Mailer: IPAD 2.0

Date: Fri, 08 May 1998 21:46:49 -0400

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Jim Ryan <jryan@exis.net>

Subject: Started building tonight

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

I started with one of the two inductors hand wound 2" PVC 700 turns

#26 gauge wire as Mike suggested.  When I finished I noticed I

had one over lap. Will that have any adverse effects ?

Jim

Date: Fri, 08 May 1998 19:49:44 -0700

From: Gene Waldo <gwaldo@1stnetusa.com>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Organization: Self

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Extreme service water-cooled capacitors?

Sender: tesl-err@emachine.com

Bill the arcstarter wrote:

>

> It was written:

Lotta snips!

>  I showed my (yet to be tested) water-cooled tungsten carbide spark gap

> to one of our machinists - just to see what he'd say about such a

> cobbled-together device.  Here's how the conversation went:

>

>  (note - B = Bill and M = machinst)

>

> B) Hey- take a look at this thing I've been building!

>  m) Oh yea.  I know what that is.

>

> B) Oh really?  What do you think it is?

>  m) That's a spark gap!

>

> B) Well - um - yes - but how did you know that? (dumbfounded)

>  m) We used to have one in our old induction heating furnace!

>

>  I was completely amazed since I'd never told him what I was doing!

> Perhaps I'm on the right track afterall??  Old machinists say the

> darndest things...

Bill,

Old Machinists are a thing to be cherished! Like Old Scotch! New

machinists don't know diddly about making things or how to make things!

They just follow programs on CMC screens!

Unfortunately, I've lost touch with my inductive heater guy! He could

design damn near anything electronic off the top of his head!

> -Bill the arcstarter

> Starting arcs in Cinci, OH

> http://www.geocities.com/CapeCanaveral/Hangar/6160

>

----------------------------------------------------

Gene Waldo -  gwaldo@1stnetusa.com  - "Igor in Residence"

---------------------------------------------------------------

What's good for the planet is more OZONE! See the machinery form up!

NEW STUFF!   http://www.misslink.net/mhammer/waldo.htm

Date: Fri, 08 May 1998 20:18:24 -0700

From: Gene Waldo <gwaldo@1stnetusa.com>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Organization: Self

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

CC: danmurphy@1stnetusa.com

Subject: R.W.S's coil...!

Sender: tesl-err@emachine.com

Mike, et al,

Why am I the only one who can't see the coils? EVERY time I hit the url

you gave me, I get some canned gobbledegook about 'connection reset by

peer', or some such crap! "Please try again"!  Close to trying the .45

WinMag!

--

Gene Waldo -  gwaldo@1stnetusa.com  - "Igor in Residence"

---------------------------------------------------------------

What's good for the planet is more OZONE! See the machinery form up!

NEW STUFF!   http://www.misslink.net/mhammer/waldo.htm

Date: Fri, 8 May 1998 21:42:07 -0600

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Daniel Lockhart <molly@cmn.net>

Subject: Re: two coils 180' out of phase

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

>Tom.Stock@gsbsc.gensig.com wrote:

>>

>>

>>      To run two coils 180 degrees out of phase, can you simple swap the

>>      imputs to the neon or do you swap the outputs of the neon?

>>

>>      -tom

>

>Tom,

>

>Neither. The key is connecting the Tesla Coil tank circuit primaries in

>series, but in opposite phase, and using a common tank capacitor and gap

>to drive them at the same time. The secondaries must be identical and

>tuned to the same resonant frequency. Once you meet these balancing

>conditions, the effects are awesome! If each coil puts out 5 footers, 10

>foot streamers will jump between the two...

>

>-- Bert --

Hmmm. I wonder how applicable it would be to make a set of smaller coils

like this? Does it scale down well, or is it kind of like magnifier systems

in the way that they don't work well when too small? Building two five foot

coils is a little too much for right now (time wise), but two

eighteen-inchers is a little more reasonable. Would you need to double the

tank circuit power for it to be able to power two coils?

Out of phase coils have always intrigued me. What would happen if you made

six coils, each offset 60 degrees more than the other, and arranged in a

circle so that one would be 180 degrees out of phase to the coil opposite

it? Sounds kinda wild to me...

Daniel Lockhart

X-Mailer: IPAD 2.0

Date: Fri, 08 May 1998 23:15:48 -0500

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: R.W.S's coil...!

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

At 08:18 PM 5/8/98 -0700, you wrote:

>Mike, et al,

>Why am I the only one who can't see the coils? EVERY time I hit the url

>you gave me, I get some canned gobbledegook about 'connection reset by

>peer', or some such crap! "Please try again"!  Close to trying the .45

>WinMag!

>--

>Gene Waldo -  gwaldo@1stnetusa.com  - "Igor in Residence"

I dont know Gene. I have no trouble accessing the funet.fi

archive. Perhaps you could try a slightly different form

of the address. Try this:

ftp://funet.fi/pub/sci/electrical/tesla/pictures/rws/twin2rws.jpg

Its the same without reference to nic.

Are you using a web browser like Netscape or trying to

FTP to the site? If you want to FTP then try:

ftp.funet.fi/pub/sci/electrical/tesla/pictures/rws/twin2rws.jpg

If all else fails Ill email you the pics.

TTYL!

Mike Hammer

mhammer@misslink.net

Date: Fri, 08 May 1998 22:47:29 -0700

From: Gene Waldo <gwaldo@1stnetusa.com>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Organization: Self

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: R.W.S's coil...!

Sender: tesl-err@emachine.com

Mike Hammer wrote:

>

> At 08:18 PM 5/8/98 -0700, you wrote:

> >Mike, et al,

> >Why am I the only one who can't see the coils? EVERY time I hit the url

> >you gave me, I get some canned gobbledegook about 'connection reset by

> >peer', or some such crap! "Please try again"!  Close to trying the .45

> >WinMag!

> >--

> >Gene Waldo -  gwaldo@1stnetusa.com  - "Igor in Residence"

>

> I dont know Gene. I have no trouble accessing the funet.fi

> archive. Perhaps you could try a slightly different form

> of the address. Try this:

>

> ftp://funet.fi/pub/sci/electrical/tesla/pictures/rws/twin2rws.jpg

>

Mike,

That one got me: "Netscape unable to locate server: funet.fi

"Server does not have DNS entry

" Check server name location URL and try again."

>

> Are you using a web browser like Netscape or trying to

> FTP to the site? If you want to FTP then try:

What's  FTP? Remember, you got a real dumb one, here!

> ftp.funet.fi/pub/sci/electrical/tesla/pictures/rws/twin2rws.jpg

Going for last try....^  ~~{:->

> If all else fails Ill email you the pics.

>

> TTYL!

>

> Mike Hammer

> mhammer@misslink.net

--

Gene Waldo -  gwaldo@1stnetusa.com  - "Igor in Residence"

---------------------------------------------------------------

What's good for the planet is more OZONE! See the machinery form up!

NEW STUFF!   http://www.misslink.net/mhammer/waldo.htm

From: "Scott Hayward" <smash@eisa.net.au>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: T2:Black Coil Form and thin wire primary

Date: Sat, 9 May 1998 18:05:46 +1000

X-Mailer: IPAD 2.0

X-MimeOLE: Produced By Microsoft MimeOLE V4.71.1712.3

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

To everyone on the list,

I have made my new secondary and new primary(shifty primary!!!).  I took a

lot of care in making the secondary (4" dia and 22" winding length, 24"total

length with 0.6mm wire, 5 coats of polyurethane varnish).  But it is black.

If it was had carbon in it would  the windings spark to the coil form or

not.

2nd comments-  The primary that I made is about 30 turns of 2mm (that's a

guess as I don't have a vernier gauge) copper wire on a 8.5" polypropelene

bucket.  It seems to work OK.  The reason I did this instead of the usual

.25" copper pipe was because I physically couldn't get the copper pipe

around the bucket.  Why is thinner wire not preferable to .25" pipe anyway.

I know it has something to do with the skin effect of RF.  Does it make the

primary have more reactance.  If so, does this really matter.

From Scott Hayward   smash@eisa.net.au

X-Mailer: IPAD 2.0

Date: Sat, 09 May 1998 02:23:57 -0700

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: bob golding <yubba@clara.net>

Subject: Re: T2 Coil Tuning and more Spark Gap Questions

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

>> At 22:04 06/05/98 -0400, you wrote:

snip:

>Ah, sorry for not being clear.  The motor I described is an induction

>start/induction run motor with a centrifugal switch to shut off the

>starter winding when the motor starts to speed up.  Nominal running

>speed is 1725 RPM, but we really want it to run at 1800 RPM, which makes

>it synchronous with the power line.  The reason we want this is: if you

>had a wheel with 4 electrodes, turned by an 1800 rpm motor, you would

>get pulses at 120 Hz (1800 revolutions per minute divided by 60 seconds

>per minute = 30 revolutions per second... So, 30 rps * 4 electrodes =

>120 Hz).

>

>Since the power line is 60 Hz (in the US), and there is a positive and a

>negative peak to the sine wave, then at 120 Hz, you can fire your coil

>each time the voltage hits its peak.  "This is a good thing (TM)".  Of

>course, you have to rotate the motor body (or the fixed electrodes) to

>adjust the phase angle so you pick the peaks rather than the

>zero-crossings ("Bad thing (TM)"), but this is easy to do - just rotate

>the motor body for maximum spark, no more than +-45 degrees in either

>direction.

>

>This type of motor has 4 poles for the main winding.  If you take the

>motor apart, you will see the heavy (main) winding in 4 separate

>bunches, 90 degrees apart on the stator (the outer frame of

>laminations).  It turns out, that if you file 4 matching flats at 90

>degrees apart on the rotor, then the rotor will run locked to the

>rotating magnetic field, because the 4 remaining curved parts of the

>rotor dominate the magnetic interactions (they are closer, so the field

>effect is much stronger on them).  Without the modification, the rotor

>is always slipping in the rotating field, which makes it slightly

>slower, in this case 1725 RPM.  The slip factor helps develop more

>horsepower, which is why motors are built this way, but for our purposes

>we can live with less horsepower since we are just spinning a light

>electrode wheel with fairly low drag.

>

>Now, I don't know about shaded pole motors - I believe those are the

>ones with a deliberate shorted turn in the stator.  If so, they are

>generally pretty weak, and I don't imagine they would be suitable for

>this modification.  Perhaps someone else on the list can answer that

>question for sure though.  I have been told that capacitor

>start/induction run motors will work (which makes sense), I have also

>been told that capacitor start/capacitor run motors will work.  I have

>not tried either of these however.

>

>If any of the above is still not clear, please ask.

thanks steve it is all clear now.

bob golding lightning without rain

bob yubba@clara.net

From: "Scott Hayward" <smash@eisa.net.au>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: two coils 180' out of phase

Date: Sat, 9 May 1998 20:36:06 +1000

X-Mailer: IPAD 2.0

X-MimeOLE: Produced By Microsoft MimeOLE V4.71.1712.3

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

>>Tom.Stock@gsbsc.gensig.com wrote:

>>>

>>>

>>>      To run two coils 180 degrees out of phase, can you simple swap the

>>>      imputs to the neon or do you swap the outputs of the neon?

>>>

>>>      -tom

>>

>>Tom,

>>

>>Neither. The key is connecting the Tesla Coil tank circuit primaries in

>>series, but in opposite phase, and using a common tank capacitor and gap

>>to drive them at the same time. The secondaries must be identical and

>>tuned to the same resonant frequency. Once you meet these balancing

>>conditions, the effects are awesome! If each coil puts out 5 footers, 10

>>foot streamers will jump between the two...

>>

>>-- Bert --

>

>

>Hmmm. I wonder how applicable it would be to make a set of smaller coils

>like this? Does it scale down well, or is it kind of like magnifier systems

>in the way that they don't work well when too small? Building two five foot

>coils is a little too much for right now (time wise), but two

>eighteen-inchers is a little more reasonable. Would you need to double the

>tank circuit power for it to be able to power two coils?

>

>Out of phase coils have always intrigued me. What would happen if you made

>six coils, each offset 60 degrees more than the other, and arranged in a

>circle so that one would be 180 degrees out of phase to the coil opposite

>it? Sounds kinda wild to me...

>

>Daniel Lockhart

Very Interesting Daniel.  I had always wanted to make 3 Tesla coils with the

right phasing so that they all met at the middle.  Another thing I've wanted

to do was to get two or more Tesla coils with the right phasing and position

them in such a way that you can control the direction of the streamers

without a ground rod.

Just telling everyone else whats going on in my head

(My psychiatrist says that I shouldn't do that too often or people might

think I'm strange.  Isn't everyone else on this list though!!! :)

From Scott Hayward  smash@eisa.net.au

Getting high of ozone in Australia

Date: Sat, 09 May 1998 06:30:19 -0500

From: Bert Hickman <bert.hickman@aquila.com>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Organization: Stoneridge Engineering

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: two coils 180' out of phase

Sender: tesl-err@emachine.com

<SNIP>>

> Hmmm. I wonder how applicable it would be to make a set of smaller coils

> like this? Does it scale down well, or is it kind of like magnifier systems

> in the way that they don't work well when too small? Building two five foot

> coils is a little too much for right now (time wise), but two

> eighteen-inchers is a little more reasonable. Would you need to double the

> tank circuit power for it to be able to power two coils?

>

> Out of phase coils have always intrigued me. What would happen if you made

> six coils, each offset 60 degrees more than the other, and arranged in a

> circle so that one would be 180 degrees out of phase to the coil opposite

> it? Sounds kinda wild to me...

>

> Daniel Lockhart

Daniel,

A small twin would be easy to scale down, and should be easier to make

than a large one.. and should be significantly easier than single or

twin twin magnifiers. A pair of small coils generating 2 foot streamers

each will jump 4 feet between them.

A 3-phase coil has been made by Duane Bylund, the author of the booklet

"Modern Tesla Coil Theory", [Tesla Books - $19.95]. Duane used a group

of phase-locked solid state high power oscillators to base-drive six

separate resonators - with resonator pairs being excited at opposing 180

degree phasing. These were arranged in a circle (hexagon?). This system

apparently put on quite a show as sparks danced between all coil

combinations. No pictures are available unfortunately. Richard Hull

reported seeing this coil in operation a year or two back and said "it

was a real hoot". :^)

-- Bert --

Date: Sat, 09 May 1998 07:00:45 -0500

From: Bert Hickman <bert.hickman@aquila.com>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Organization: Stoneridge Engineering

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Extreme service water-cooled capacitors?

Sender: tesl-err@emachine.com

Mike Hammer wrote:

>

> At 11:01 AM 5/8/98 PDT, you wrote:

> >It was written:

> >>This increases power throughput. You can increase the throughput

> >>even more by increasing the firing rate of your gap. But be

> >>careful doing this. High break rates can literally beat your

> >>cap to death.

> >

> > I've read that some commercial induction heating systems (basically

> >identical to the front-end of a tesla coil) contain water-cooled

> >capacitor banks which handle upwards of 1000 amps of RF on a continuous

> >basis.  The output inductor (similar to our primary coils) is water

> >cooled and resonates with a cap in order to deliver magnetic energy to

> >the object being heated.  A small unit is 20 KW.  The largest sizes can

> >run up to 1 MW.  Surely these caps are putting up with much more abuse

> >than the ones in our tesla coils!

>

> That would help in the case of dielectric heating but it doesnt

> help the actual physical hammering of the plates do to the

> pulsing action of the Tesla tank circuit.

> >

> >

> > Does anyone know how reliable the commercially available (and very

> >expensive) tesla-coil pulse caps are??

>

> No more or less reliable than any other cap. Several people

> had rather spectacular explosive failures of some pulse

> caps from Condenser Products that were designed for

> Tesla coil service.

>

> TTYL!

>

> Mike Hammer

> mhammer@misslink.net

Mike brings up an excellent point - disruptive (spark-gap) Tesla coils

are one of the most demanding applications that any capacitor will ever

see. In addition to being suddenly discharged, the capacitor also

undergoes almost full voltage reversal as the primary circuit "rings".

And these stresses are repeated often many hundreds of times a second.

Capacitor dielectrics heat up, and the capacitor "sandwich" will expand

and contract slightly due to the rapidly changing electrostatic forces

between opposing capacitor plates, behaving sort of like a miniature

accordian. During ringing, the peak primary current may be hundreds, or

even thousands, of amps, and the voltage reversals effectively DOUBLE

the dielectric stress. Although commercial pulse caps are usually fairly

reliable when operated within their ratings, the combined electrical and

mechanical stresses can sometimes lead to very spectacular (and

potentially dangerous) failures.

I've personally had two [very messy] capacitor "explosions" using

cylindrical commercial pulse caps. Others have had mini-explosions using

salt water caps when sudden dielectric breakdown and  arcing thermally

shocks the bottle. In your Tesla Coil designs, remember to

enclose/secure your caps and HV leads so that, if they DO go, they won't

shower the surrounding area with oil and capacitor guts. You need to

secure the HV wires to prevent an ejected high voltage termination from

suddenly flying loose to make contact with something it shouldn't (such

as you or your guests!). Also, stay away from metal-encased "bathtub"

caps (chances are these are mylar anyway) or glass-enclosed "glass-mike"

caps for similar reasons.

Safe coilin' to you!

-- Bert --

Date: Sat, 09 May 1998 06:24:59 -0700

From: Scott MacGregor <smacgregor@aasp.net>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Organization: EES,Co.

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: R.W.S's coil...!

Sender: tesl-err@emachine.com

Gene Waldo wrote:

>

> Mike Hammer wrote:

> >

> > At 08:18 PM 5/8/98 -0700, you wrote:

> > >Mike, et al,

> > >Why am I the only one who can't see the coils? EVERY time I hit the url

> > >you gave me, I get some canned gobbledegook about 'connection reset by

> > >peer', or some such crap! "Please try again"!  Close to trying the .45

> > >WinMag!

> > >--

> > >Gene Waldo -  gwaldo@1stnetusa.com  - "Igor in Residence"

> >

> > I dont know Gene. I have no trouble accessing the funet.fi

> > archive. Perhaps you could try a slightly different form

> > of the address. Try this:

> >

> > ftp://funet.fi/pub/sci/electrical/tesla/pictures/rws/twin2rws.jpg

> >

>

> Mike,

> That one got me: "Netscape unable to locate server: funet.fi

>                  "Server does not have DNS entry

>                  " Check server name location URL and try again."

Gene,

I had the same result. Use:

www.funet.fi/pub/sci/electrical/tesla/pictures/rws/twin2rws.jpg

Amazing picture!

--

Regards,

Scott MacGregor EES,Co.

************************************************************

Enterprise Engineering Services Co.

E-mail smacgregor@aasp.net

Member: Cable Television Advisory Committee, Plainville, MA

http:/www.expage.com/page/cableguy

************************************************************

Date: Sat, 09 May 1998 07:52:53 -0700

From: Gene Waldo <gwaldo@1stnetusa.com>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Organization: Self

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: R.W.S's coil...!

Sender: tesl-err@emachine.com

Scott MacGregor wrote:

>

> Gene Waldo wrote:

> >

> > Mike Hammer wrote:

> > >

> > > At 08:18 PM 5/8/98 -0700, you wrote:

> > > >Mike, et al,

> > > >Why am I the only one who can't see the coils?

> > > >canned gobbledegook about 'connection reset by

> > > >peer', or some such crap! "Please try again"!

>>> Mike,

> > That one got me: "Netscape unable to locate server: funet.fi

> >                  "Server does not have DNS entry

> >                  " Check server name location URL and try again."

>

> Gene,

> I had the same result. Use:

> www.funet.fi/pub/sci/electrical/tesla/pictures/rws/twin2rws.jpg

> Amazing picture!

OKITAY!  YESSSS!!!! Now I get the picture!  http://www..... (what you

said).

Now, them is SPARKS!  ~~<[:-))

> Regards,

> Scott MacGregor EES,Co.

Glad I asked!

-------------------------------------------------------

Gene Waldo -  gwaldo@1stnetusa.com  - "Igor in Residence"

---------------------------------------------------------------

What's good for the planet is more OZONE! See the machinery form up!

NEW STUFF!   http://www.misslink.net/mhammer/waldo.htm

Date: Sat, 09 May 1998 08:34:25 -0700

From: Gene Waldo <gwaldo@1stnetusa.com>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Organization: Self

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Primary configuration?

Sender: tesl-err@emachine.com

Gene Waldo wrote:

>

> Mike Hammer wrote:

> >

> > At 07:52 PM 5/7/98 -0700, you wrote:

> > >To Mike Hammer et al,

> SNIPS!

> >

> > The only consideration is the primary cap and the main gap setting.

> > Less gap for lower voltages.

Elaborating on my original question:

> I have an idea for a simple 'tunable-on-the fly' blown, series  main  gap. I Will

> have a concept sketch up to you in a bit... for critique.

> If this sounds dumb or futile initially, let me know and I'll cease. It

> should also facilitate periodic cleanings....   It may also, no doubt,

> have been tried, but I haven't seen reference to anything like it.

I see the use of this as a means to ramp the voltage/gap on the fly.

Also it could be a calibrated, variable 'universal gap'...

If there is no discussion, I'll drop it.

-------------------------------------------------------

Gene Waldo -  gwaldo@1stnetusa.com  - "Igor in Residence"

---------------------------------------------------------------

What's good for the planet is more OZONE! See the machinery form up!

NEW STUFF!   http://www.misslink.net/mhammer/waldo.htm

X-Mailer: IPAD 2.0

Date: Sat, 09 May 1998 14:49:50 -0400

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Jim Ryan <jryan@exis.net>

Subject: Toroid Top Load

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

12k 30ma. 4.5" x 21"

I just finished my secondary coil it took about 3 hours to hand wind.

My wrists hurt.

Can I use 2  8" pie pan to spread my 4" diam. alum. dryer duct Toroid that

has an inside diam. of 7 1/2" and an outside of 11 1/2 " ? I plan to bolt

the together to hold the Toroid in place and help keep the round shape.

Thanks

Jim

Date: Sat, 09 May 1998 14:04:29 -0500

From: Bert Hickman <bert.hickman@aquila.com>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Organization: Stoneridge Engineering

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Toroid Top Load

Sender: tesl-err@emachine.com

Jim Ryan wrote:

>

> 12k 30ma. 4.5" x 21"

> I just finished my secondary coil it took about 3 hours to hand wind.

> My wrists hurt.

>

> Can I use 2  8" pie pan to spread my 4" diam. alum. dryer duct Toroid that

> has an inside diam. of 7 1/2" and an outside of 11 1/2 " ? I plan to bolt

> the together to hold the Toroid in place and help keep the round shape.

>

> Thanks

>

> Jim

Jim,

Sounds like you're getting there! The pie-pans should work great for a

small toroid!

-- Bert --

X-Mailer: IPAD 2.0

Date: Sat, 09 May 1998 15:27:54 -0400

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Jim Ryan <jryan@exis.net>

Subject: Re: Toroid Top Load

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

At 02:04 PM 5/9/98 -0500, you wrote:

>Jim Ryan wrote:

>>

>> 12k 30ma. 4.5" x 21"

>> I just finished my secondary coil it took about 3 hours to hand wind.

>> My wrists hurt.

>>

>> Can I use 2  8" pie pan to spread my 4" diam. alum. dryer duct Toroid that

>> has an inside diam. of 7 1/2" and an outside of 11 1/2 " ? I plan to bolt

>> the together to hold the Toroid in place and help keep the round shape.

>>

>> Thanks

>>

>> Jim

>

>Jim,

>

>Sounds like you're getting there! The pie-pans should work great for a

>small toroid!

>

>-- Bert --

>

>Bert here is a picture attached.

Jim

Attachment Converted: "D:\MISSING_ LINK\EUDORA\Attach\Image12.jpg"

Editors Note. Binary attachments not included in list archives.  MAH

X-Mailer: IPAD 2.0

Date: Sat, 09 May 1998 14:37:17 -0500

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: safety gap

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

At 07:36 PM 5/9/98 -0700, you wrote:

>hi all

> can someone give me a ballpark figure for setting my safety spark gap. I

>am using gene waldo's basic design (three bolts spaced 120 degrees apart)

>but I am using a 2" ring. I'm not sure if I will have enough clearance. Max

>is about 3/4".  I am using a 9kv 50mA neon, 23 awg 4.25"x21" secondary, 10

>turns 3/8" cu primary, 0.0177uf flat plate cap, 2.4mh air core chokes, 1k

>50watt series resistors 8"x4" toroid for now. many thanks for all the help

>so far.

>

>bob golding lightning without rain

>

>bob yubba@clara.net

Bob,

Your system sounds great.

To properly set your safety gap connect it to your neon

without anything else attached. Just the safety gap

right across the HV outputs. The center electrode should

go to ground but for this test it can be connected to the

neon case.

Adjust the gap just slightly wider than the neon can break down

on its own. This sets the breakdown voltage a little above 9kv

so it affords the best protection.

When you run your coil you can adjust the safety gap slightly

one way or the other depending on the activity level. If

its not breaking down at all during operation then it may be slightly

too wide. If its breaking down a lot it might be slightly too

narrow. Dont open it too much or you will lose the protection

a safety gap provides.

In general you will want your safety gap set just slighty wider

than your main gap.

TTYL!

Mike Hammer

mhammer@misslink.net

X-Mailer: IPAD 2.0

Date: Sat, 09 May 1998 15:06:11 -0500

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: T2:Black Coil Form and thin wire primary

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

At 06:05 PM 5/9/98 +1000, you wrote:

>To everyone on the list,

>I have made my new secondary and new primary(shifty primary!!!).  I took a

>lot of care in making the secondary (4" dia and 22" winding length, 24"total

>length with 0.6mm wire, 5 coats of polyurethane varnish).  But it is black.

>If it was had carbon in it would  the windings spark to the coil form or

>not?

I generally advise against using any colored pipes particulary

black. You never know what the conductivity of the plastic may be like.

>

>2nd comments-  The primary that I made is about 30 turns of 2mm (that's a

>guess as I don't have a vernier gauge) copper wire on a 8.5" polypropelene

>bucket.  It seems to work OK.  The reason I did this instead of the usual

>.25" copper pipe was because I physically couldn't get the copper pipe

>around the bucket.  Why is thinner wire not preferable to .25" pipe anyway.

>I know it has something to do with the skin effect of RF.  Does it make the

>primary have more reactance.  If so, does this really matter.

>From Scott Hayward   smash@eisa.net.au

Copper tubing is the most cost effective material for primary coils

as it puts the copper where it will do the most good. The RF currents

flow mainly along the outside of the conductor. The center sees very

little current flow. Copper tubing offers no performance advantage

over solid copper wire of the same diameter.

You want heavy conductors in the primary circuit (not just the primary

coil) to handle the high RF currents flowing. This current can be

tens of amperes for even small coils and hundreds of amperes for

medium sized coils. I2R losses would mount up rapidly with thinner

conductors.

2 mm wire is pretty close to AWG 12 and that will be sufficient

for a low power coil and should do well up to 400-500 watts

or so. Above that power level it would be wise to change to

heavier conductors.

30 turns is quite a few for a primary with the secondary

you have. You should only need 14-16 turns. But its always

easier to remove extra turns than it is to add some.

Once you get your coil up and running you'll get a better

idea what size primary you will need.

TTYL!

Mike Hammer

mhammer@misslink.net

X-Mailer: IPAD 2.0

Date: Sat, 09 May 1998 18:11:41 -0400

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Jim Ryan <jryan@exis.net>

Subject: Help me understand

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Hi All,

12k 30ma. neon 21" x 4.5" secondary

Finished my secondary, I have a 24 guage wire hanging out of the top and

bottom. I think I can connect the top without without any problems.

I don't understand how you connect a 24 guage wire to a ground wire

that will carry 100-200 amps. How much 24 guage wire can hang in inches out

from the coil before you connect to the heavy ground.

Thanks

Jim

Date: Sat, 09 May 1998 17:32:50 -0500

From: Bert Hickman <bert.hickman@aquila.com>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Organization: Stoneridge Engineering

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Help me understand

Sender: tesl-err@emachine.com

Jim Ryan wrote:

>

> Hi All,

> 12k 30ma. neon 21" x 4.5" secondary

> Finished my secondary, I have a 24 guage wire hanging out of the top and

> bottom. I think I can connect the top without without any problems.

>

> I don't understand how you connect a 24 guage wire to a ground wire

> that will carry 100-200 amps. How much 24 guage wire can hang in inches out

> from the coil before you connect to the heavy ground.

>

> Thanks

>

> Jim

Jim,

The ground wire on your coil will only be handling a few amps of RF

current. However, the primary tank circuit may be handling up to a

couple hundred amps of peak current when your main gap fires.

-- Bert --

X-Mailer: IPAD 2.0

Date: Sat, 09 May 1998 18:46:54 -0500

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: Help me understand

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

At 06:11 PM 5/9/98 -0400, you wrote:

>Hi All,

>12k 30ma. neon 21" x 4.5" secondary

>Finished my secondary, I have a 24 guage wire hanging out of the top and

>bottom. I think I can connect the top without without any problems.

>

>I don't understand how you connect a 24 guage wire to a ground wire

>that will carry 100-200 amps. How much 24 guage wire can hang in inches out

>from the coil before you connect to the heavy ground.

>

>Thanks

>

>Jim

Jim,

On my secondary coils I epoxy a rectangular copper plate to the

side of the secondary form just below the bottom secondary turn.

The secondary wire is then soldered to this plate.

Your heavy ground lead is then attached to the copper plate.

I solder a short flexible lead of AWG 8 directly to the plate

and terminate the free end in a large ring terminal.

When I set the coil up to operate I bolt this ring terminal

to my main RF ground lead.

I dont think any but the largest coils would have 100-200 amps

of ground current. The ground lead needs to be heavy not so much

to carry heavy current as to offer a low impedance to ground.

TTYL!

Mike Hammer

mhammer@misslink.net

Date: Sat, 09 May 1998 17:31:14 -0700

From: Gene Waldo <gwaldo@1stnetusa.com>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Organization: Self

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: safety gap

Sender: tesl-err@emachine.com

bob golding wrote:

>

> hi all

>  can someone give me a ballpark figure for setting my safety spark gap. I

> am using gene waldo's basic design (three bolts spaced 120 degrees apart)

> but I am using a 2" ring. I'm not sure if I will have enough clearance.

> bob golding lightning without rain

> bob yubba@clara.net

The simplest way to ball park your safety gap is to test for the max gap

distance that will start the transformer (alone!) discharging to gnd.,

for each HV terminal, then add an bit more. You want the safety gap to

just barely not spark during normal operation. If anything over that

voltage comes back from the tank, it'll dump to gnd and not enter your

power supply.

--------------------------------------------

Gene Waldo -  gwaldo@1stnetusa.com  - "Igor in Residence"

---------------------------------------------------------------

What's good for the planet is more OZONE! See the machinery form up!

NEW STUFF!   http://www.misslink.net/mhammer/waldo.htm

Date: Sat, 09 May 1998 17:53:15 -0700

From: Gene Waldo <gwaldo@1stnetusa.com>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Organization: Self

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Toroid Top Load

Sender: tesl-err@emachine.com

Jim Ryan wrote:

>

> 12k 30ma. 4.5" x 21"

> I just finished my secondary coil it took about 3 hours to hand wind.

> My wrists hurt.

>

> Can I use 2  8" pie pan to spread my 4" diam. alum. dryer duct Toroid that

> has an inside diam. of 7 1/2" and an outside of 11 1/2 " ? I plan to bolt

> the together to hold the Toroid in place and help keep the round shape.

>

> Thanks

>

> Jim

That is exactly what I did. My fix for the toroid 'join' was a wood plug

1.5" thick, snug fit inside the tube, and very small finishing brads and

nailset. The final assy nailing is a definite 4 hands job! You almost

have to look hard to find my join!  Keep the toroid in tension

(compressed short) as you work up the final length of it; reassembling

the pans will keep it in tension at final assy. Then, never drop it!

that could get ugly!

-----------------------------------

Gene Waldo -  gwaldo@1stnetusa.com  - "Igor in Residence"

---------------------------------------------------------------

What's good for the planet is more OZONE! See the machinery form up!

NEW STUFF!   http://www.misslink.net/mhammer/waldo.htm

From: "Scott Hayward" <smash@eisa.net.au>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Toroid Top Load

Date: Sun, 10 May 1998 12:06:43 +1000

X-Mailer: IPAD 2.0

X-MimeOLE: Produced By Microsoft MimeOLE V4.71.1712.3

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

>12k 30ma. 4.5" x 21"

>I just finished my secondary coil it took about 3 hours to hand wind.

>My wrists hurt.

That's the problem with winding a secondary by hand.  I only recently wound

a 4" by 22" and I had a sore back, sore fingers and sore wrists as well.

Electricity isn't the only hazardous thing that Tesla coils produce!!!!  :)

From Scott Hayward    smash@eisa.net.au

Getting high off ozone in Australia  :)

From: "Scott Hayward" <smash@eisa.net.au>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: T2:Black Coil Form and thin wire primary

Date: Sun, 10 May 1998 12:12:32 +1000

X-Mailer: IPAD 2.0

X-MimeOLE: Produced By Microsoft MimeOLE V4.71.1712.3

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

>>To everyone on the list,

>>I have made my new secondary and new primary(shifty primary!!!).  I took a

>>lot of care in making the secondary (4" dia and 22" winding length, 24"total

>>length with 0.6mm wire, 5 coats of polyurethane varnish).  But it is black.

>>If it was had carbon in it would  the windings spark to the coil form or not.

>

>I generally advise against using any colored pipes particulary

>black. You never know what the conductivity of the plastic may be like.

Dear Mike,

But this still doesn't tell me whether I have carbon in my secondary.  What

will happen if it does.  I've already tried it and I don't see any sparks to

the coil form.  Does that mean it's OK??

From Scott Hayward   smash@eisa.net.au

Getting high off ozone in Australia  :)

X-Mailer: IPAD 2.0

Date: Sat, 09 May 1998 19:36:19 -0700

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: bob golding <yubba@clara.net>

Subject: safety gap

Cc: tesla-2@emachine.com

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

hi all

can someone give me a ballpark figure for setting my safety spark gap. I

am using gene waldo's basic design (three bolts spaced 120 degrees apart)

but I am using a 2" ring. I'm not sure if I will have enough clearance. Max

is about 3/4".  I am using a 9kv 50mA neon, 23 awg 4.25"x21" secondary, 10

turns 3/8" cu primary, 0.0177uf flat plate cap, 2.4mh air core chokes, 1k

50watt series resistors 8"x4" toroid for now. many thanks for all the help

so far.

bob golding lightning without rain

bob yubba@clara.net

X-Mailer: IPAD 2.0

Date: Sun, 10 May 1998 00:23:02 -0400

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Moleking <moleking@concentric.net>

Subject: Re: Toroid Top Load

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

At 07:04 AM 5/10/98 , you wrote:

>At 12:06 10/05/98 +1000, you wrote:

>>

>>

>>>12k 30ma. 4.5" x 21"

>>>I just finished my secondary coil it took about 3 hours to hand wind.

>>>My wrists hurt.

>>

>>

>>That's the problem with winding a secondary by hand.  I only recently wound

>>a 4" by 22" and I had a sore back, sore fingers and sore wrists as well.

>>Electricity isn't the only hazardous thing that Tesla coils produce!!!!  :)

When I wound my 4x22 secondary I put the wire spool in a box, held up with

a rod held in place with some nails.  I sat down on the couch w/ my coil

form, hot glue gun, and a cup of coffee, and proceeded to roll by hand.  i

maintained tension using my sock-laden feet on the spool.  I put a drop of

hot glue on the coil every inch or so (which saved me from disaster several

times).  When I was done the hot glue came off rather easily w/ a

fingernail.  The roll took me about 4-5 hours.

For what it's worth,

Josh Bedell

X-Mailer: IPAD 2.0

Date: Sat, 09 May 1998 23:34:58 -0500

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: T2:Black Coil Form and thin wire primary

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

At 12:12 PM 5/10/98 +1000, you wrote:

>>>To everyone on the list,

>>>I have made my new secondary and new primary(shifty primary!!!).  I took a

>>>lot of care in making the secondary (4" dia and 22" winding length, 24"total

>>>length with 0.6mm wire, 5 coats of polyurethane varnish).  But it is black.

>>>If it was had carbon in it would  the windings spark to the coil form or not.

>>

>>I generally advise against using any colored pipes particulary

>>black. You never know what the conductivity of the plastic may be like.

>

>Dear Mike,

>But this still doesn't tell me whether I have carbon in my secondary.  What

>will happen if it does.  I've already tried it and I don't see any sparks to

>the coil form.  Does that mean it's OK??

>

>From Scott Hayward   smash@eisa.net.au

>

Scott,

There is no way to know if there is carbon coloring

agent in your form or not. If the form is even slightly

conductive you will have much arcing and a short lived

secondary. Since you have run it with no problem so

far then its likely to be OK.

In the future steer away from colored plastics just

to be on the safe side.

TTYL!

Mike Hammer

mhammer@misslink.net

X-Mailer: IPAD 2.0

Date: Sun, 10 May 1998 04:04:39 -0700

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: bob golding <yubba@clara.net>

Subject: Re: Toroid Top Load

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

At 12:06 10/05/98 +1000, you wrote:

>

>

>>12k 30ma. 4.5" x 21"

>>I just finished my secondary coil it took about 3 hours to hand wind.

>>My wrists hurt.

>

>

>That's the problem with winding a secondary by hand.  I only recently wound

>a 4" by 22" and I had a sore back, sore fingers and sore wrists as well.

>Electricity isn't the only hazardous thing that Tesla coils produce!!!!  :)

hi all

I wound my secondary (21" x 4" 23 awg) in about an hour, no back ache or

sore wrists. It is worth planning this operation in advance. What I did was

make up some 4" plugs that just fit inside your coil form. Then make up 2

more about 6" dia. I used 1/4" ply. Drill a hole in the middle and get hold

of some threaded rod from the local hardware store, buy a pack of bolts and

washers to go with it. Make up a simple jig. I used 2 short pieces of

dexion angle. Anything will do. fit your coil form between the two and away

you go. It doesn't need to be very accurate as you are winding it by hand.

A trick to keep the wire tensioned is to put your drum in a cardboard box

that is just wide enough to hold the drum, with the drum just touching the

bottom of the box  place a rod though the middle then feed the wire out of

the top, fold the top of the box over so that the wire feeds over it. Stick

the box behind the coil form and wind towards you keeping the wire  lined

up using your thumb nail. If you need to stop just clamp a pair of vise

grips on one of the end disks. The weight will keep it all in tension. Hope

this makes sense and saves a few backs and sore thumbs. You can varnish the

form on the same jig after winding the coil.

bob golding lightning without rain

bob yubba@clara.net

Date: Sun, 10 May 1998 08:15:04 -0400 (EDT)

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: fxphoto@centuryinter.net (fxphoto)

Subject: T2-Gap Observation

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

To All,

While playing with one of Gene Waldo's ideas I had something interesting

happen. The gap setup was ten 1/2" x 1 1/8" copper couplings set end to end

instead of side to side as in a normal RQ style gap. The gaps were set at

0.014 " (one thickness of my aluminum flashing as I run a narrow gap when

testing).

The tank circuit was connected across all 9 gaps all the time and only the

NST circuit wire was changed to vary the voltage input. Everything worked

fine and the voltage did vary according to the number of gaps spanned.

It was interesting to watch the gaps outside the NST span firing.

My question is: Is it possible to optimise the gap number and distance for

the tank circuit? If so, what would I be looking for as desired parameters?

i.e highest amperage through the gap, highest voltage through the gap, best

quenching characteristics?

Thanks for your input             Bill Turbett

Date: Sun, 10 May 1998 08:58:22 -0500

From: Bert Hickman <bert.hickman@aquila.com>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Organization: Stoneridge Engineering

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: T2-Gap Observation

Sender: tesl-err@emachine.com

fxphoto wrote:

>

> To All,

>   While playing with one of Gene Waldo's ideas I had something interesting

> happen. The gap setup was ten 1/2" x 1 1/8" copper couplings set end to end

> instead of side to side as in a normal RQ style gap. The gaps were set at

> 0.014 " (one thickness of my aluminum flashing as I run a narrow gap when testing).

> The tank circuit was connected across all 9 gaps all the time and only the

> NST circuit wire was changed to vary the voltage input. Everything worked

> fine and the voltage did vary according to the number of gaps spanned.

> It was interesting to watch the gaps outside the NST span firing.

>

>  My question is: Is it possible to optimise the gap number and distance for

> the tank circuit? If so, what would I be looking for as desired parameters?

> i.e highest amperage through the gap, highest voltage through the gap, best

> quenching characteristics?

>  Thanks for your input             Bill Turbett

Bill,

Very interesting questions and observations!

First, if your tank circuit was tied across the entire gap, and one of

the neon HV leads was tapped to an inner gap electrode, this

configuration has the tank capacitor getting recharged through a number

of gaps. The gaps that were passing the charging current from the NST

had already "broken down", leaving the remainder to fire when the cap

reached sufficient voltage. While this configuration will work, it is

less efficient than connecting the the NST across the entire gap and to

the tank circuit and then "tapping" the gap with one of the high current

tank leads. This way your tank cap will recharge beginning from the zero

crossing of the incoming AC waveform rather than having to wait until

the intervening gaps break down.

Your second question is MUCH tougher. When your main gap fires, an arc

is formed between the gap electrodes. The more current you force through

the arc, the less voltage drop it will have. This characteristic is just

the opposite of most other conductors we're familiar with! Because of

this "negative resistance" characteristic, operating with higher tank

circuit currents would seem to be more efficient. However, higher tank

currents also cause heavier gap ionization and localized electrode

incandescence, and the gap becomes MUCH more difficult to break (or

"quench"). It turns out that getting proper quenching is actually more

important, but for this I need to explain a bit about quenching.

When your main gap fires, energy stored in your primary capacitor and

inductor (LC circuit) is transferred to the secondary LC circuit over a

number of cycles until there's no energy left in the primary, and all

the remaining energy now resides in the secondary. This is the ideal

time for the arc to be extinguished or quenched, so that virtually all

of the secondary's energy can go into making streamers. If the main gap

reignites, then you'll begin to lose energy from the secondary as it

transfers energy back to the primary tank circuit.

In an inefficient coil, this process of energy interchange between

primary and secondary can occur many times, causing you to lose

substantial energy to the main gap instead of generating streamers. You

can see this happening if you overload a gap - as you increase input

power, the streamers grow, reach a peak, and then begin to shrink as the

gap ceases proper quenching.

The best quenching with static gaps is obtained by using many gaps,

small gap spacings, and large thermally efficient electrodes. However,

adding more gaps will also increase the total voltage drop across the

gaps when they fire, and closer gap spacings can also cause "fouling" as

the gap electrodes oxidize.

In practice, the most efficient coils typically run with higher

inductance primaries (10-24 turns), and use multiple forced vacuum/air

cooled gaps of between 0.025 and 0.030" each. The number of these gaps

will depend upon the voltage of your input source and the voltage

breakdown capability of your tank cap. For NST-driven systems, the total

gap length is typically in the 0.15" - 0.33" range (6 - 10X the

individual gap spacing). Larger overall gap spacings can be used to

obtin higher coil output, but this can also overstress the NST or tank

cap(s), causing these to prematurely fail. For example, I personally run

18 gaps and 0.54" on my 10" coil with the knowledge that I am

significantly overstressing the 15 KV NST's. The home-brew tank caps

were initially designed to be driven from a 20 KV pole-pig (75 KV

breakdown) and can run all day with this punishment.

Hope this helps!

-- Bert --

X-Mailer: IPAD 2.0

Date: Sun, 10 May 1998 16:47:44 -0400

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Jim Ryan <jryan@exis.net>

Subject: Re: CFM

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Hi All, as a rule of thumb, about how many CFM should a fan or vacuum

push/pull across a 9 pipe copper spark setup?

Thanks Jim

Date: Sun, 10 May 1998 16:28:47 -0500

From: Bert Hickman <bert.hickman@aquila.com>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Organization: Stoneridge Engineering

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: CFM

Sender: tesl-err@emachine.com

Jim Ryan wrote:

>

> Hi All, as a rule of thumb, about how many CFM should a fan or vacuum

> push/pull across a 9 pipe copper spark setup?

>

> Thanks Jim

Jim,

It depends upon the size of your coil. Best bet is to use a universal

vacuum cleaner motor/blower and drive it from a 10A variac. I find that

my coil runs best when the motor is overdriven to 130 VAC. Others have

found that their systems seem to run best at less than 120 VAC. Your

mileage may vary... :^). Best bet - get a 10-12 amp high speed motor and

see what works best for you. If you don't have a spare variac, you can

drill a 1" diameter hole in the box you make behind your vacuum gaps and

then make a sliding plate that allows you to adjust the amount of air

"leakage" you get. This will allow you to control the level of vacuum

and the air velocity through the gaps.

-- Bert --

-- Bert --

X-Mailer: IPAD 2.0

Date: Sun, 10 May 1998 18:30:40 -0400

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Jim Ryan <jryan@exis.net>

Subject: Feeler Gauges

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

For the person who was wondering what a feeler gaugae, I just used a

2"x 5" 1/4"=.250 strip of balsa wood to set the gaps on a 9 pipe

arrrangement. The balsa slides smooth through each gap.

Jim

Date: Sun, 10 May 1998 18:59:48 -0400 (EDT)

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: fxphoto@centuryinter.net (fxphoto)

Subject: Re: Feeler Gauges

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Jim,

What are you running for your power source? That sounds like an awfully

large gap. 2" total?? If you are running a neon sign transformer, check with

Mike Hammer or a few of the other guys who know what they are doing and I

think you will find the gap is usually around 0.025" to 0.030". I believe

too large a gap can hurt a neon sign transformer.

Bill Turbett

>For the person who was wondering what a feeler gaugae, I just used a

>2"x 5" 1/4"=.250 strip of balsa wood to set the gaps on a 9 pipe

>arrrangement. The balsa slides smooth through each gap.

>

>Jim

>

>

X-Mailer: IPAD 2.0

Date: Sun, 10 May 1998 19:37:37 -0400

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Jim Ryan <jryan@exis.net>

Subject: Re: Feeler Gauges

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

At 06:59 PM 5/10/98 -0400, you wrote:

>Jim,

> What are you running for your power source? That sounds like an awfully

>large gap. 2" total?? If you are running a neon sign transformer, check with

>Mike Hammer or a few of the other guys who know what they are doing and I

>think you will find the gap is usually around 0.025" to 0.030". I believe

>too large a gap can hurt a neon sign transformer.

>                             Bill Turbett

>

>>For the person who was wondering what a feeler gaugae, I just used a

>>2"x 5" 1/4"=.250 strip of balsa wood to set the gaps on a 9 pipe

>>arrrangement. The balsa slides smooth through each gap.

>>

>>Jim

>>

>>Yep your correct. I screwed up. I can recover in about 1 hour.

No problem.

Thanks, Gosh I was only 90% off.

Jim

Date: Sun, 10 May 1998 18:08:50 -0600

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Daniel Lockhart <molly@cmn.net>

Subject: Mock Preliminary Coil Testing

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

Hey everybody,

I still haven't got around to making my spark gap for my first coil

(frustrating!). Coming home late from a gig last night and not really

wanting to go to bed, I decided to fire the coil up, RQ gap or not. I made

a pathetic gap out of two nails, and gave it power.

Now first of all, I realise that the circumstances were not at all the best.

The spark gap was terrible, and the cap I was using was 1/8 the correct

size (it seems a lot of the saltwater had evaporated out of my "big"

saltwater cap). I just gave it a go to see what would happen.

Without the toroid I got some corona off the secondary and was able to pull

a whopping 3/4-inch spark to ground. I few things were happening to the

secondary that I didn't like though.

With all the lights out, and about 1/5th the time I gave it power, a spark

would travel from the top of the secondary down two inches for a fraction

of a second. Occasionally more, and once I saw it go down a foot. Is my

secondary internally shorting out? Is this normal for it to happen

occasionally? When I look in the inside of the secondary, I can barely see

a line about the same place where the sparks were traveling down on the

outside. I don't recall seeing the line before. The sparks didn't happen

much, just enough for me to notice. Is my secondary no good, is this to be

expected with the way I have it temporarily set up, or is this normal?

Thanks!

Daniel Lockhart

Date: Sun, 10 May 1998 19:50:54 -0500

From: Bert Hickman <bert.hickman@aquila.com>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Organization: Stoneridge Engineering

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Mock Preliminary Coil Testing

Sender: tesl-err@emachine.com

Daniel Lockhart wrote:

>

> Hey everybody,

>

> I still haven't got around to making my spark gap for my first coil

> (frustrating!). Coming home late from a gig last night and not really

> wanting to go to bed, I decided to fire the coil up, RQ gap or not. I made

> a pathetic gap out of two nails, and gave it power.

>

> Now first of all, I relise that the circumstances were not at all the best.

> The spark gap was terrible, and the cap I was using was 1/8 the correct

> size (it seems a lot of the saltwater had evaporated out of my "big"

> saltwater cap). I just gave it a go to see what would happen.

>

> Without the toroid I got some corona off the secondary and was able to pull

> a whopping 3/4-inch spark to ground. I few things were happening to the

> secondary that I didn't like though.

>

> With all the lights out, and about 1/5th the time I gave it power, a spark

> would travel from the top of the secondary down two inches for a fraction

> of a second. Occasionally more, and once I saw it go down a foot. Is my

> secondary internally shorting out? Is this normal for it to happen

> occasionally? When I look in the inside of the secondary, I can barely see

> a line about the same place where the sparks were traveling down on the

> outside. I don't recall seeing the line before. The sparks didn't happen

> much, just enough for me to notice. Is my secondary no good, is this to be

> expected with the way I have it temporarily set up, or is this normal?

>

> Thanks!

>

> Daniel Lockhart

Daniel,

The main problem was that your tank cap was too small. If it was 1/8 the

size it should have been, chances are you were getting multiple (perhaps

3!) voltage peaks on your secondary winding instead of a single 1/4

voltage peak at the top of the coil. This severe out-of tune condition

can create the kind of inter-turn sparkovers you saw. Overcoupling

between the primary and secondary can also create this condition.

Try adding the proper amount of capacitance and try again WITH a top

discharge terminal attached to protect the top of the secondary winding.

As long as you haven't carbonized a trail on your secondary coil or on

the coilform, it should be OK. You may want to carefully inspect the

arc-line, clean out any signs of carbon, and then recoat with some clear

polyurethane varnish.

-- Bert --

Date: Sun, 10 May 1998 18:12:06 -0700

From: mike nicholas <705728@ican.net>

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: T2 Cheap choke

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

In tv's there is a flyback transformer which contains a ferrite ring,

different ones will have different sizes, but this seems like a good idea to

me, check out your junk box or local junk yard, it might just work!

From: "Scott Hayward" <smash@eisa.net.au>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: T2:Black Coil Form and thin wire primary

Date: Mon, 11 May 1998 15:50:08 +1000

X-Mailer: IPAD 2.0

X-MimeOLE: Produced By Microsoft MimeOLE V4.71.1712.3

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

>Scott,

>There is no way to know if there is carbon coloring

>agent in your form or not. If the form is even slightly

>conductive you will have much arcing and a short lived

>secondary. Since you have run it with no problem so

>far then its likely to be OK.

>

>In the future steer away from colored plastics just

>to be on the safe side.

>

>TTYL!

>

>Mike Hammer

>mhammer@misslink.net

Thanks for the advice.  The problem was that my father bought the pipe for

me and I couldn't take it back.  I just thought that I might be losing power

due to carbon in the coil form because I'm getting fairly wimpy sparks.  I'm

hopefully getting some materials to make a toroid for the top soon.

From Scott Hayward   smash@eisa.net.au

Getting high off ozone in Australia  :)

X-Mailer: IPAD 2.0

Date: Mon, 11 May 1998 14:18:46 -0700

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: bob golding <yubba@clara.net>

Subject: cap calculations

Cc: tesla-2@emachine.com

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

hi all

Could someone check my figures for my cap as I have totally confused

myself! I am using a 9Kv 50 ma neon on 50 Hz. I make that 0.0177uF. I am

using 7 layers of 5 mil polypropylene to give 35 mil. My plate area is

5.25" x8" I make that 44 square inches. I reckon on 2 caps in series at

35400pF each. Voltage rating 350*35*2=24500volts. 0.224*2*44/0.035=563pF

per plate. I need 2 caps of 35400pF (17700pF*2=35400pF) so 35400/563= 63

plates in each cap. Does this sound right/ make sense. many thanks.

bob golding lightning without rain

bob yubba@clara.net

From: wwl@netcomuk.co.uk (Mike Harrison)

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: CFM (controlling vacuum cleaner motors)

Date: Mon, 11 May 1998 09:13:10 GMT

Organization: White Wing Logic

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

>

>It depends upon the size of your coil. Best bet is to use a universal

>vacuum cleaner motor/blower and drive it from a 10A variac. I find that

>my coil runs best when the motor is overdriven to 130 VAC. Others have

>found that their systems seem to run best at less than 120 VAC.

Remember you can also use triac type power controllers (or some light

dimmers if they're rated for inductive loads, or have snubbers) to

control universal motors (but not to overdrive!), smaller and cheaper

than a variac - just keep the HV well away!

____                                                           ____

_/ L_/  Mike Harrison / White Wing Logic / wwl@netcomuk.co.uk  _/ L_/

_/ W_/  Hardware & Software design / PCB Design / Consultancy  _/ W_/

/_W_/  Industrial / Computer Peripherals / Hazardous Area      /_W_/

X-Mailer: IPAD 2.0

Date: Mon, 11 May 1998 11:52:24 -0500

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: cap calculations

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

At 02:18 PM 5/11/98 -0700, you wrote:

>hi all

> Could someone check my figures for my cap as I have totally confused myself!

Dont feel bad Bob. I've been totally confused since 1974.

No further comment.

>I am using a 9Kv 50 ma neon on 50 Hz. I make that 0.0177uF.

Correct.

>I am using 7 layers of 5 mil polypropylene to give 35 mil. My plate area is

>5.25" x 8" I make that 44 square inches.

Uh. Bob. Over here in the USA I make that to be 42 sq inches.

>I reckon on 2 caps in series at 35400pF each. Voltage rating

>350*35*2=24500volts.

Ok.

>0.224*2*44/0.035=563pF per plate. I need 2 caps of 35400pF

(17700pF*2=35400pF) >so 35400/563= 63 plates in each cap.

Does this sound right/ make sense?

Close. First of all you cant have 63 plates. You need an even number

of plates since they work in pairs. Adjusting for your error above

in the plate area I get 34944 pf with 66 plates. Thats a lot of plates.

And you need two sets of them.

35 mils of poly per cap is probably overkill for 9kv. So you

can thin down the dielectric a bit to raise the capacitance.

Try 6 layers of your poly for 30 mils per cap. This will still

be bullet proof for 9kv and will work well at 12kv. It would

probably work at 15kv but its a bit marginal.

Using 30 mils per cap 58 plates comes to 35750 pf.

How about increasing plate area? Using 7 X 7 plates with

49 sq inches per plate and 30 mil poly thickness you need

50 plates for 35854 pf.

As you can see there are lots of ways to do it.

If you were wanting to build the cap contemplating future

15kv operation then I would build a third identical

cap and use 3 in series for 15kv. You will be needing

less capacitance for 15kv neons anyway. A 15kv/60 will

want something around .01 mfd. 3 of your .035 caps in series

will yield around .0117 mfd.  That will probably work just fine

as most neons will handle a slightly larger cap than the

math indicates.

>many thanks.

>

>bob golding lightning without rain

>

>bob yubba@clara.net

TTYL!

Mike Hammer

mhammer@misslink.net

To:	"Tesla-2" <tesla-2@emachine.com>

Date:	Mon, 11 May 1998 11:04:53 -0700

From:	"Leben Lang und Frei" <fleischwulf@mailexcite.com>

X-Sent-Mail: on

X-Expiredinmiddle: true

X-Mailer: IPAD 2.0

Subject: can someone help me ?

X-Sender-Ip: 208.153.7.9

Organization: MailExcite  (http://www.mailexcite.com:80)

Date: Mon, 11 May 1998 13:06:57 -0500

From: fleischwulf@mailexcite.com

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

My coil and lab are both on separate circuits. The coil resides in the lab.

The lights in lab are on a 20 amp breaker. Saturday I ran my coil for the

first time at 7.2 kw. The labs breaker kicked out just minutes into the run.

The coil was still running, I shut it down and resest the breaker. I then powered

up the coil, again the lab breaker blew. The coil is on a stand alone 240 v circuit

with its own breaker and main power switch. The circuit for the coil has a huge RF

filter between  it and the power supply.

What could be happening ?

Paul M

---

Einigkeit und Freiheit

Date: Mon, 11 May 1998 18:16:01 -0700

From: mike nicholas <705728@ican.net>

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: cheap chokes

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Mike,

Would the ferrite cores out of flyback transformers be

suitable to make chokes?

X-Mailer: IPAD 2.0

Date: Mon, 11 May 1998 17:32:59 -0500

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: cheap chokes

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

At 06:16 PM 5/11/98 -0700, you wrote:

>  Mike, Would the ferrite cores out of flyback transformers be

> suitable to make chokes?

>

Yes I think they would. The material is probably

optimized for a much lower frequency (15khz) but

should work OK at coiling frequencies.

After all we are not interested in making a high Q

low loss choke. We want to build a crummy lossy choke.

My only concern would be getting sufficent inductance.

You want at least 2 mh and more would be better.

Wind as many turns as you can fit on the core.

Let us know how they work.

TTYL!

Mike Hammer

mhammer@misslink.net

Date: Mon, 11 May 1998 19:15:07 -0400 (EDT)

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: fxphoto@centuryinter.net (fxphoto)

Subject: Re: cheap chokes

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Mike,

Is it possible to have too much inductance?

The chokes I wound came out to 24 mh.

Bill Turbett

>

>My only concern would be getting sufficent inductance.

>You want at least 2 mh and more would be better.

>Wind as many turns as you can fit on the core.

>

>Let us know how they work.

>

>TTYL!

>

>Mike Hammer

>mhammer@misslink.net

>

>

Date: Mon, 11 May 1998 19:19:13 -0400 (EDT)

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: fxphoto@centuryinter.net (fxphoto)

Subject: T2-Transmission fluid

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

To All,

I just purchased 5 gal of Diala transformer oil and was told that I could

use Dextron transmission fluid in an emergency. The man said that this was

what was used before Diala was invented. Does anyone know anything about this?

Bill Turbett

Date: Mon, 11 May 1998 19:32:57 -0400 (EDT)

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: fxphoto@centuryinter.net (fxphoto)

Subject: Re: CFM (controlling vacuum cleaner motors)

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Mike,

Could you please explain what a snubber is ?

Thanks

Bill Turbett

>

>>

>>It depends upon the size of your coil. Best bet is to use a universal

>>vacuum cleaner motor/blower and drive it from a 10A variac. I find that

>>my coil runs best when the motor is overdriven to 130 VAC. Others have

>>found that their systems seem to run best at less than 120 VAC.

>Remember you can also use triac type power controllers (or some light

>dimmers if they're rated for inductive loads, or have snubbers) to

>control universal motors (but not to overdrive!), smaller and cheaper

>than a variac - just keep the HV well away!

>

>     ____                                                           ____

>   _/ L_/  Mike Harrison / White Wing Logic / wwl@netcomuk.co.uk  _/ L_/

> _/ W_/  Hardware & Software design / PCB Design / Consultancy  _/ W_/

>/_W_/  Industrial / Computer Peripherals / Hazardous Area      /_W_/

>

>

X-Mailer: IPAD 2.0

Date: Mon, 11 May 1998 19:19:22 -0500

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: cheap chokes

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

At 07:15 PM 5/11/98 -0400, you wrote:

>Mike,

> Is it possible to have too much inductance?

>The chokes I wound came out to 24 mh.

>                 Bill Turbett

>>

Probably not. I think though a point will come

when additional inductance wont help any more.

Also remember an inductor is an energy storage device.

With a lot of inductance your chokes could produce

some nasty kickbacks.

What kind of choke did you wind to get 24 mh?

TTYL!

Mike Hammer

mhammer@misslink.net

X-Mailer: IPAD 2.0

Date: Mon, 11 May 1998 19:26:01 -0500

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: T2-Transmission fluid

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

At 07:19 PM 5/11/98 -0400, you wrote:

>To All,

> I just purchased 5 gal of Diala transformer oil and was told that I could

>use Dextron transmission fluid in an emergency. The man said that this was

>what was used before Diala was invented. Does anyone know anything about

this?

>                           Bill Turbett

I would not use any automotive oils. They have many additives

to enhance their capability in their intended usage but that doesnt

do much for the electrical characteristics.

These oils will probably have semi metallic anti wear compounds

and anti foam additives and who knows what else.

TTYL!

Mike Hammer

mhammer@misslink.net

Date: Mon, 11 May 1998 20:27:10 -0500

From: Bert Hickman <bert.hickman@aquila.com>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Organization: Stoneridge Engineering

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: T2-Transmission fluid

Sender: tesl-err@emachine.com

fxphoto wrote:

>

> To All,

>  I just purchased 5 gal of Diala transformer oil and was told that I could

> use Dextron transmission fluid in an emergency. The man said that this was

> what was used before Diala was invented. Does anyone know anything about this?

>                            Bill Turbett

Bill,

They're NOT the same. Diala is a very refined naphthenic mineral oil

with (AX) or without (X) an antioxidant additive. Non-synthetic

Automatic Transmission Fluid (ATF) is basically a mixture of oils,

solvents, and various additives. It's thinner, has more volatile

solvents, the additives are NOT helpful in making capacitors, and the

stuff's flammable to boot! If ATF is mixed with oil, it contaminates it

from a recycling standpoint, and it's classified as a hazardous waste.

The additional solvents in the ATF may also cause adverse swelling of

the LDPE dielectric in your cap. Now that you've got the Diala, take a

small amount and compare it with ATF - the differences are obvious!

Stick with the Diala - it's good stuff!

-- Bert --

X-Mailer: IPAD 2.0

Date: Mon, 11 May 1998 21:02:40 -0500

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: removal/ EVERYONE READ!

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

At 09:05 PM 5/11/98 EDT, you wrote:

>Please remove me from your mailing list.

>

OK people. One more time. This will be the third time

Ive posted instructions for unsubscribing yourself

from the list.

Please make note of this and keep a copy for future

reference. If you have difficulty unsubscribing

DO NOT post a request to the list. Email me direct

instead.

To quit the list, either permanently or temporarily, such as for vacation,

please send a message to tesla-2-request@emachine.com with one of the

following keywords in the body of the message:  Delete, Remove or

Unsubscribe.  Case is not important.

If you should later wish to rejoin the list, please send a message to

tesla-2-request@emachine.com with one of the following keywords in the

body of the message:  Join, Add or Subscribe.  Case is not important.

One additional note. If you unsubscribe and then try to resubscribe

yourself later on it must be with the same email address. If

you change addresses then the list server sees you as a new subscriber.

New subs still need to go through me.

If you need to change your address email me with the change

rather than unsubscribing.

TTYL!

Mike Hammer

mhammer@misslink.net

Date: Mon, 11 May 1998 22:12:05 -0400 (EDT)

From: Paul Anderson <madhobby@geeky1.ebtech.net>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: T2-Transmission fluid

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

On Mon, 11 May 1998, Mike Hammer wrote:

>

> These oils will probably have semi metallic anti wear compounds

> and anti foam additives and who knows what else.

>

SAE 10W30 motor oil (BTW) can often be between 10% and 40% additives.

Seems to me that transmission fluid would be more in line with hydraulic

fluid...  TTYL!

---

Paul Anderson

madhobby@geeky1.ebtech.net

Date: Mon, 11 May 1998 19:12:07 -0700

From: mike nicholas <705728@ican.net>

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: cheap chokes

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Mike Hammer wrote:

> At 06:16 PM 5/11/98 -0700, you wrote:

> >  Mike, Would the ferrite cores out of flyback transformers be suitable to make chokes?

> >

>

> Yes I think they would. The material is probably

> optimized for a much lower frequency (15khz) but

> should work OK at coiling frequencies.

>

> Do I need to cut a slot in it so its not a closed cicuit?

> After all we are not interested in making a high Q

> low loss choke. We want to build a crummy lossy

> choke.

> Can I put more than one layer of wire?

Will #24 be ok?

> My only concern would be getting sufficent inductance.

> You want at least 2 mh and more would be better.

> Wind as many turns as you can fit on the core.

>

> Let us know how they work.

> How will I know?

> TTYL!

>

> Mike Hammer

> mhammer@misslink.net

Date: Mon, 11 May 1998 22:22:08 -0400 (EDT)

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: fxphoto@centuryinter.net (fxphoto)

Subject: Re: cheap chokes

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Mike,

I started out by making two 3" acrylic discs for each choke. I drilled  a

1/4" hole in the center of each. I drilled a 1/16" hole in one of the flats

of a 1/4" nut and soldered the end of a roll of #24 wire into it. Then I

took a 2" x 1/4" bolt and put a fender washer on it followed by one acrylic

disc. Then the 1/4" nut was screwed on and the next disc put on followed by

another fender washer and finally another nut to hold everything together.

The assembly was mounted in my electric hand drill, a big gob of 5 min epoxy

was put on the nut and the sides and I turned the drill on very slowly.

About halfway through I stopped and added more epoxy. I would guess there

are 4 or 5 hundred turns which I terminated by soldering the end of the wire

to a piece of 1/2" copper tubing that I had cut open and shaped to an

L-bracket shape formed to the curve of the disc and with a 1/4" hole drilled

in it. The end result is a 1/4" thick x 3" dia

disc which screws onto the H.V. bushing and has a lug with a 1/4" hole to

mount my H.V. wire to.

Bill Turbett

P.S.I was shooting for 2 to 10 mH and thought I had read that 400 turns of

#24 or #28 wire were necessary.<G>

>Also remember an inductor is an energy storage device.

>With a lot of inductance your chokes could produce

>some nasty kickbacks.

>

>What kind of choke did you wind to get 24 mh?

>

>TTYL!

>

>Mike Hammer

>mhammer@misslink.net

>

X-Mailer: IPAD 2.0

Date: Mon, 11 May 1998 21:27:55 -0500

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Mike Hammer <mhammer@misslink.net>

Subject: web page update

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Hello all,

I posted some new pics and new coil specs

on my web page today for those that may be

interested. Im interested in feedback on

the changes. Email me direct with comments.

TTYL!

Mike Hammer

mhammer@misslink.net

X-Mailer: IPAD 2.0

Date: Mon, 11 May 1998 23:22:37 -0500

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: web page update

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

At 09:27 PM 5/11/98 -0500, you wrote:

>Hello all,

>

>I posted some new pics and new coil specs

>on my web page today for those that may be

>interested. Im interested in feedback on

>the changes. Email me direct with comments.

>

>TTYL!

>

>Mike Hammer

>mhammer@misslink.net

>

Duh! What a idiot. I forgot to post the URL

for my page. Here it is:

http://www.misslink.net/mhammer/

TTYL!

Mike Hammer

mhammer@misslink.net

X-Mailer: IPAD 2.0

Date: Mon, 11 May 1998 23:50:15 -0500

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: can someone help me ?

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

At 11:04 AM 5/11/98 -0700, you wrote:

>My coil and lab are both on seperate circuits. The coil resides in the

lab. The lights in lab are on a 20 amp breaker. Saturday I ran my coil for

the first time at 7.2 kw. The labs breaker kicked out just minutes into the

run. The coil was still

> running, I shut it down and resest the breaker. I then powered up the

coil, again the lab breaker blew. The coil is on a stand alone 240 v

circuit with its own breaker and main power switch. The circuit for the

coil has a huge RF filter between

> it and the power supply. What could be happening ?

>

>Paul M

>---

>Einigkeit und Freiheit

Paul,

You got me on that one. Im not sure what could be causing

that to happen.

The breaker at fault isnt a GFI is it? GFIs dont work

well at all in and around coils. Ive even tripped them

with my ham radio transmitter.

How about your 60 cycle ground? Are you bringing it

anywhere near your coil? How about your RF line filter?

Are you connecting your 60 cycle ground to it?

Im all for not bringing the 60 cycle ground anywhere

near any part of a coil system.

Just a couple wild guesses.

TTYL!

Mike Hammer

mhammer@misslink.net

X-Mailer: IPAD 2.0

Date: Tue, 12 May 1998 00:14:54 -0700

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: bob golding <yubba@clara.net>

Subject: Re: cap calculations

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

At 11:52 11/05/98 -0500, you wrote:

>At 02:18 PM 5/11/98 -0700, you wrote

snip

>Try 6 layers of your poly for 30 mils per cap. This will still

>be bullet proof for 9kv and will work well at 12kv. It would

>probably work at 15kv but its a bit marginal.

>

>Using 30 mils per cap 58 plates comes to 35750 pf.

>

>How about increasing plate area? Using 7 X 7 plates with

>49 sq inches per plate and 30 mil poly thickness you need

>50 plates for 35854 pf.

mike I based the plate size on my poly wallets,which are 12" x 81/2". I was

planning on a generous 1 1/2" clearance all round the plates. should I

reduce this to increase plate area or just make more plates as I have a LOT

of ally sheet due to miscalculations earlier when I thought I needed 17.7

uf not 17.7 nF!!oops. Oh well it's only money.

totally off list question. One of my lecturers was looking for some 500pF

350 volt trimming caps and complaining about the lack of same in the UK.

Anybody help.

bob golding lightning without rain

bob yubba@clara.net

Date: Tue, 12 May 1998 00:39:06 -0700 (PDT)

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Dan Murphy <danmurphy@1stnetusa.com>

Subject: Dan finally gets it up!

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

The following is a forwarded mssg from my ISP webmaster:

>Date: Mon, 11 May 1998

>From: Webmaster <webmaster@1stnetusa.com>

>Organization: 1stNetUSA

>To: danmurphy@1stnetusa.com

>Subject: Your Web Site

>

>We are sorry it took so long to put your page on the Web.  We apologize and

thank you for your patience.  Here is the address of your new web site:    >

http://members.1stnetusa.com/danmurphy

>

>Thank you very much for your patience and your understanding.

So - there it is. You are graciously invited to attend the grand opening.

Who knows when I'll be able to get it up again?

MadManDanMadManDanMadManDanMadManDanMadManDanMadManDanMadManDanMadManDan

--- I don't suffer from insanity. I enjoy every minute of it!  ---

<danmurphy@1stnetusa.com>

MadManDanMadManDanMadManDanMadManDanMadManDanMadManDanMadManDanMadManDan

From: "The Meyer Family" <meyer@clove.net.au>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Radio Control worries with coil

Date: Tue, 12 May 1998 22:22:10 +0930

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Hello,

I am a keen coiler as well as being very keen on flying model

aircraft. I have recently become concerned that my recievers in my aircraft

may be being affected from the use of my coil. I never run my coil when

they are in the same shed I bring them up inside well away. I also never

have them switched on. Is my concern genuine or am I just being paranoid

Thanks very much for any replies

Andrew

meyer@clove.net.au

X-Mailer: IPAD 2.0

Date: Tue, 12 May 1998 09:41:06 -0400

From: Kevin Batchelor <kbatchelor@arslimited.com>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Radio Control worries with coil

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Andrew:

As far as your concern about your radio controlled aircraft, my advice is get

them as far away as possible and/or shield them from your coil.  I recently killed

my cordless phone.  It sits about 30 feet away from my coil in the house.

Needless to say it's now just junk.

Kevin

>>> "The Meyer Family" <meyer@clove.net.au> 05/12 8:52 AM >>>

Hello,

I am a keen coiler as well as being very keen on flying model

aircraft. I have recently become concerned that my recievers in my aircraft

may be being affected from the use of my coil. I never run my coil when

they are in the same shed I bring them up inside well away. I also never

have them switched on. Is my concern genuine or am I just being paranoid

Thanks very much for any replies

Andrew

meyer@clove.net.au

Date: Tue, 12 May 1998 11:00:23 -0700

From: mike nicholas <705728@ican.net>

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: can someone help me ?

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

I would try replacing the breaker, if its not new then the breaker maybe

prematurely kicking off, it has happened to me often in appliance repair.

Mike Hammer wrote:

> At 11:04 AM 5/11/98 -0700, you wrote:

> >My coil and lab are both on seperate circuits. The coil resides in the

> lab. The lights in lab are on a 20 amp breaker. Saturday I ran my coil for

> the first time at 7.2 kw. The labs breaker kicked out just minutes into the

> run. The coil was still

> > running, I shut it down and resest the breaker. I then powered up the

> coil, again the lab breaker blew. The coil is on a stand alone 240 v

> circuit with its own breaker and main power switch. The circuit for the

> coil has a huge RF filter between

> > it and the power supply. What could be happening ?

> >

> >Paul M

> >---

> >Einigkeit und Freiheit

>

> Paul,

>

> You got me on that one. Im not sure what could be causing

> that to happen.

>

> The breaker at fault isnt a GFI is it? GFIs dont work

> well at all in and around coils. Ive even tripped them

> with my ham radio transmitter.

>

> How about your 60 cycle ground? Are you bringing it

> anywhere near your coil? How about your RF line filter?

> Are you connecting your 60 cycle ground to it?

>

> Im all for not bringing the 60 cycle ground anywhere

> near any part of a coil system.

>

> Just a couple wild guesses.

>

> TTYL!

>

> Mike Hammer

> mhammer@misslink.net

Date: Tue, 12 May 1998 11:08:13 -0700

From: mike nicholas <705728@ican.net>

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Radio Control worries with coil

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

With the complicated digital R/C system now days I recomend keeping them far away,

if you are using a Kraft radio put you can almost it on your discharge terminal without hurting it.

> Andrew:

>   As far as your concern about your radio controlled aircraft, my advice is get them

>as far away as possible and/or shield them from your coil.  I recently killed my

>cordless phone.  It sits about 30 feet away from my coil in the house.  Needless to say >it's now just junk.

>

> Kevin

>

> >>> "The Meyer Family" <meyer@clove.net.au> 05/12 8:52 AM >>>

> Hello,

>         I am a keen coiler as well as being very keen on flying model

> aircraft. I have recently become concerned that my recievers in my aircraft

> may be being affected from the use of my coil. I never run my coil when

> they are in the same shed I bring them up inside well away. I also never

> have them switched on. Is my concern genuine or am I just being paranoid

>  Thanks very much for any replies

> Andrew

>

> meyer@clove.net.au

>

From: Roy Minnich <rminnich@rolling-meadows.lib.il.us>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Ferrite ring chokes

Date: Tue, 12 May 1998 16:17:38 -0500

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

Hello,

I wound 27 ft. (140 turns) of # 26 on each of my 2 in. ferrite rings last night.  I wound it side by side with no overlaps and it took up a little over half of each ring.  How do you find the inductance?  I tried the standard calculation using r = .25, N = 140, h = 4.375 and got 26.6 uH.  Sound right?  I think r might be bigger, the ring has squared corners so the .25 value was a guess.

Thanks,

Roy Minnich

----------

From: 	Mike Hammer[SMTP:mhammer@misslink.net]

Reply To: 	Tesla-2 Mailing List

Sent: 	Monday, May 11, 1998 7:19 PM

To: 	Tesla-2 Mailing List

Subject: 	Re: cheap chokes

At 07:15 PM 5/11/98 -0400, you wrote:

>Mike,

> Is it possible to have too much inductance?

>The chokes I wound came out to 24 mh.

>                 Bill Turbett

>>

Probably not. I think though a point will come

when additional inductance wont help any more.

Also remember an inductor is an energy storage device.

With a lot of inductance your chokes could produce

some nasty kickbacks.

What kind of choke did you wind to get 24 mh?

TTYL!

Mike Hammer

mhammer@misslink.net

X-Mailer: IPAD 2.0

Date: Tue, 12 May 1998 16:29:37 -0500

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: Ferrite ring chokes

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

At 04:17 PM 5/12/98 -0500, you wrote:

>Hello,

>

>I wound 27 ft. (140 turns) of # 26 on each of my 2 in. ferrite rings last

night.  I wound it side by side with no overlaps and it took up a little

over half of each ring.  How do you find the inductance?  I tried the

standard calculation using r = .25, N = 140, h = 4.375 and got 26.6 uH.

Sound right?  I think r might be bigger, the ring has squared corners so

the .25 value was a guess.

>Thanks,

>Roy Minnich

Roy,

There is no way to calculate your resulting inductance without

knowing the characteristics of the core material. The inductance

formula posted here on the list recently is for air core coils.

Perhaps you can borrow a LCR meter and measure them or you

could pair one up with a known capacitance and measure the

resonant frequency. But this would require some test equipment

like a signal generator and scope or a grid dip meter.

I would try them out on your coil and see how they work.

TTYL!

Mike Hammer

mhammer@misslink.net

From: Roy Minnich <rminnich@rolling-meadows.lib.il.us>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: RE: Ferrite ring chokes

Date: Tue, 12 May 1998 17:23:05 -0500

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

Hi Mike,

If I try them out on my 2 1/2 coil how will I know they are working or not?  So far

I ran the coil with my 12 kv 30 ma neon and a sheet of foil laminated foam board for

a cap that was way low on uF and got 8 in. arcs with a 4 in. foil covered ball on the secondary.  I built a 8 gap fan cooled RQ cylinder gap that works great but no safety

gap yet.

This is my first coil so what would a kickback do or look like.  I'm guessing the safety would fire if I got a kickback?

Thanks,

Roy Minnich

----------

From: 	Mike Hammer[SMTP:mhammer@misslink.net]

Reply To: 	Tesla-2 Mailing List

Sent: 	Tuesday, May 12, 1998 4:29 PM

To: 	Tesla-2 Mailing List

Subject: 	Re: Ferrite ring chokes

<SNIP>

I would try them out on your coil and see how they work.

TTYL!

Mike Hammer

mhammer@misslink.net

X-Originating-IP: [208.16.152.180]

From: "Bill the arcstarter" <arcstarter@hotmail.com>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Dayton: Meet at booth 2632?!

Date: Tue, 12 May 1998 20:54:39 PDT

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

Coilers,

Since nobody else has any better suggestions - here is my proposal.

Interested coilers should meet at booth/spot #2632.  This is one of the

Dayton Antique radio club booths (or so I'm told...!  )

I'll be there at 12:00 on Fri and Sat-wearing a Nissan hat and a machine

gun shirt.  Should be a fairly unique combination...

I'd pin a crushed quarter on my hat but they're down to 0.540 diameter

so it'd be hard to see...

Hope to see ALL OF YOU there!

Para-Ordinance Renders Compromise Obsolete!

-Bill the arcstarter

X-Originating-IP: [199.217.140.22]

From: "Bill the arcstarter" <arcstarter@hotmail.com>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Oops - I meant booth 2634

Date: Wed, 13 May 1998 05:48:30 PDT

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

I recently wrote:

> Since nobody else has any better suggestions - here is my proposal.

>Interested coilers should meet at booth/spot #2632.  This is one of

Oops - That is the WRONG number.  The correct booth number for John

Heck's ARCO radio club booth (at the Dayton hamfest) is 2634.

Sorry about that.  You can chastize me for my mistake on saturday...

A little confusion can be fun!

-Bill

From: Roy Minnich <rminnich@rolling-meadows.lib.il.us>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Q

Date: Wed, 13 May 1998 16:29:39 -0500

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

Hello all,

Could somebody explain what Q is and how it affects/relates to Tesla coils?

Thanks,

Roy Minnich

From: "Scott Hayward" <smash@eisa.net.au>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Tesla Talk

Date: Thu, 14 May 1998 20:46:16 +1000

X-Mailer: IPAD 2.0

X-MimeOLE: Produced By Microsoft MimeOLE V4.71.1712.3

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Hello all,

I'm sorry I haven't been on the list for a while but I've been extremely

busy lately.  One of the reasons for that is I was doing a tesla coil talk

for both my physics classes.  It seemed to go well and everybody liked the

demo at the end when I fired up my coil.  I think that the operation of the

coil went over most people's heads but I know have a few people who want to

make one now so I think it was  worthwhile.

Just something to talk about

From Scott Hayward  smash@eisa.net.au

Getting high off ozone in Australia :)

From: "sean kelly" <lakeland@multipro.com>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Q

Date: Thu, 14 May 1998 11:47:01 -0500

X-Mailer: IPAD 2.0

X-MimeOLE: Produced By Microsoft MimeOLE V4.72.2106.4

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Hi Roy,

You asked a very good question, but it's a difficult one to answer in a

simple statement.  I'm not sure of how in depth you want the answer to be so

I'll start with the basics.  Every coil has a natural frequency, that is a

freq that it wants to resonate at while requiring a minimum energy input.

It's analogous to pushing a child on a swing.  If you time it right, you

only have to push the swing a little and very quickly the child is flying

high.  So it is also with a resonate circuit. The higher the "Q", the more

output you will get for a given input.  The down side is that the higher the

"Q", the narrower the range in which the coil will naturally oscillate.

Going back to the swing, if you push just a little before or after the

perfect moment, then the child still swings but won't go as high as

possible.  With a Tesla coil,  a wide( aka. low) "Q" offers the easist way

to get things working to get sparks, but you won't get the biggest sparks.

This is where tuning comes into play.  Tuning simply matches the circuit

characteristics to the desired freq so that you are at the peak of the "Q"

curve(aka. pushing at the right moment electrically).

Hope this helps.

Sean Kelly

seankelly@rocketmail.com

-----Original Message-----

From: Roy Minnich <rminnich@rolling-meadows.lib.il.us>

To: Tesla-2 Mailing List <tesla-2@emachine.com>

Date: Wednesday, May 13, 1998 5:09 PM

Subject: Q

>Hello all,

>

>Could somebody explain what Q is and how it affects/relates to Tesla coils?

>

>Thanks,

>

>Roy Minnich

>

>

From: Roy Minnich <rminnich@rolling-meadows.lib.il.us>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: RE: Q

Date: Thu, 14 May 1998 20:07:29 -0500

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

Hi Sean,

OK I'm beginning to understand but is Q something you can calculate or assign

a number to?  It seems that all of the parts of a tank circuit will affect the Q

of the system then right?  Like, that's why we size the cap to the neon and the

primary to the secondary  etc. so the system functions as a resonant whole?

Yes, no?  I thought I read somewhere that Q is like a quality rating.

Would this be a way of looking at it also?

Tell me more.

Thanks,

Roy Minnich

----------

From: 	sean kelly[SMTP:lakeland@multipro.com]

Reply To: 	Tesla-2 Mailing List

Sent: 	Thursday, May 14, 1998 11:47 AM

To: 	Tesla-2 Mailing List

Subject: 	Re: Q

Hi Roy,

You asked a very good question, but it's a difficult one to answer in a

simple statement.  I'm not sure of how in depth you want the answer to be so

I'll start with the basics.  Every coil has a natural frequency, that is a

freq that it wants to resonate at while requiring a minimum energy input.

It's analogous to pushing a child on a swing.  If you time it right, you

only have to push the swing a little and very quickly the child is flying

high.  So it is also with a resonate circuit. The higher the "Q", the more

output you will get for a given input.  The down side is that the higher the

"Q", the narrower the range in which the coil will naturally oscillate.

Going back to the swing, if you push just a little before or after the

perfect moment, then the child still swings but won't go as high as

possible.  With a Tesla coil,  a wide( aka. low) "Q" offers the easist way

to get things working to get sparks, but you won't get the biggest sparks.

This is where tuning comes into play.  Tuning simply matches the circuit

characteristics to the desired freq so that you are at the peak of the "Q"

curve(aka. pushing at the right moment electrically).

Hope this helps.

Sean Kelly

seankelly@rocketmail.com

-----Original Message-----

From: Roy Minnich <rminnich@rolling-meadows.lib.il.us>

To: Tesla-2 Mailing List <tesla-2@emachine.com>

Date: Wednesday, May 13, 1998 5:09 PM

Subject: Q

>Hello all,

>

>Could somebody explain what Q is and how it affects/relates to Tesla coils?

>

>Thanks,

>

>Roy Minnich

>

>

X-Mailer: IPAD 2.0

Date: Thu, 14 May 1998 21:12:23 -0500

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Mike Hammer <mhammer@misslink.net>

Subject: RE: Q

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

At 08:07 PM 5/14/98 -0500, you wrote:

>Hi Sean,

>

>OK I'm beginning to understand but is Q something you can calculate or

assign a number to?  It seems that all of the parts of a tank circuit will

affect the Q of the system then right?  Like, that's why we size the cap to

the neon and the primary to the secondary  etc. so the system functions as

a resonant whole?

>Yes, no?  I thought I read somewhere that Q is like a quality rating.

Would this be a way of looking at it also?

>

>Tell me more.

>

>Thanks,

>Roy Minnich

Roy,

Q is indeed a quality rating or factor. The formula for

Q for an inductor is commonly given as:

Q = Xl/R

This means the the lower the resistance of the coil (or the higher

its reactance) the higher the Q. This shows why larger guage wire

for coils results in higher Q. Larger wire has lower resistance.

Also it shows one reason why a tall skinny candlestick type secondary

with many turns of small guage wire performs poorly. Low Q.

The Q for coils operating at RF frequencies is lower than

this formula indicates because the RF resistance of the coil

is higher than the DC resistance due to skin effect and other

factors.

Calculating RF resistance is an exercise I prefer to

leave to the more mathematically inclined.

Dont place too much importance on Q values. Especially for

secondary coils. Once the gap fires system Q plummets so

going to extraordinary lengths to build a high Q secondary

is a waste of time. Put all that effort into keeping primary

circuit Q up and you'll get the most from your coil.

TTYL!

Mike Hammer

mhammer@misslink.net

From: "sean kelly" <lakeland@multipro.com>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Q

Date: Fri, 15 May 1998 07:39:41 -0500

X-Mailer: IPAD 2.0

X-MimeOLE: Produced By Microsoft MimeOLE V4.72.2106.4

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Hi Mike,

You bring up a point that I've heard, but never really understood.  That

is, if the Q is a function of reactance and resistance(which are in

themselves inherent properties indepedent of current and voltage), then why

does the Q change once the circuit is energized?

thanks,sean

-----Original Message-----

From: Mike Hammer <mhammer@misslink.net>

To: Tesla-2 Mailing List <tesla-2@emachine.com>

Date: Thursday, May 14, 1998 9:50 PM

Subject: RE: Q

>At 08:07 PM 5/14/98 -0500, you wrote:

>>Hi Sean,

>>

>>OK I'm beginning to understand but is Q something you can calculate or

>assign a number to?  It seems that all of the parts of a tank circuit will

>affect the Q of the system then right?  Like, that's why we size the cap to

>the neon and the primary to the secondary  etc. so the system functions as

>a resonant whole?

>>Yes, no?  I thought I read somewhere that Q is like a quality rating.

>Would this be a way of looking at it also?

>>

>>Tell me more.

>>

>>Thanks,

>>Roy Minnich

>

>Roy,

>

>Q is indeed a quality rating or factor. The formula for

>Q for an inductor is commonly given as:

>

>Q = Xl/R

>

>This means the the lower the resistance of the coil (or the higher

>its reactance) the higher the Q. This shows why larger guage wire

>for coils results in higher Q. Larger wire has lower resistance.

>Also it shows one reason why a tall skinny candlestick type secondary

>with many turns of small guage wire performs poorly. Low Q.

>

>The Q for coils operating at RF frequencies is lower than

>this formula indicates because the RF resistance of the coil

>is higher than the DC resistance due to skin effect and other

>factors.

>

>Calculating RF resistance is an exercise I prefer to

>leave to the more mathematically inclined.

>

>Dont place too much importance on Q values. Especially for

>secondary coils. Once the gap fires system Q plummets so

>going to extraordinary lengths to build a high Q secondary

>is a waste of time. Put all that effort into keeping primary

>circuit Q up and you'll get the most from your coil.

>

>TTYL!

>

>Mike Hammer

>mhammer@misslink.net

>

>

Date: Fri, 15 May 1998 07:52:18 -0700

From: Gene Waldo <gwaldo@1stnetusa.com>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Organization: S.A.P.I.E.N.T. (Surly, Agressive, Politically Incorrect Enemies of Non Thinkers)

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Q

Sender: tesl-err@emachine.com

Mike Hammer wrote:

>

> At 08:07 PM 5/14/98 -0500, you wrote:

> >Hi Sean,

> >

> >OK I'm beginning to understand but is Q something you can calculate or

> assign a number to?  It seems that all of the parts of a tank circuit will

> affect the Q of the system then right?

> >Yes, no?  I thought I read somewhere that Q is like a quality rating.

> Would this be a way of looking at it also?

> >

> >Tell me more.

> >

> >Thanks,

> >Roy Minnich

>

> Roy,

>

> Q is indeed a quality rating or factor. The formula for

> Q for an inductor is commonly given as:

>

> Q = Xl/R

>

> This means the the lower the resistance of the coil (or the higher

> its reactance) the higher the Q. This shows why larger guage wire

> for coils results in higher Q. Larger wire has lower resistance.

> Also it shows one reason why a tall skinny candlestick type secondary

> with many turns of small guage wire performs poorly. Low Q.

>

Put all that effort into keeping primary

> circuit Q up and you'll get the most from your coil.

>

> TTYL!

>

> Mike Hammer

> mhammer@misslink.net

When tuning your primary tank, max 'Q' or max 'performance' is primarily

a function of tuning or 'rocking-in' on the resonant frequency of the

pri ckt coil, by means of the tap, all other factors being close, as I

understand it. This tap position will change as various terminal

capacitances are applied to the 2ndary.

If one plots performance or Q vs Freq, given the general shape of

resonant system peaks, you can see the critical effect of the tolerance

on the precise tuning (matching) of the exitatation frequency to the

system frequency.

Perhaps I can illustrate with some numbers out of the air:

(This may not work, given our WYSINWYG*** mail format!)

If you calculate a resonant frequency of about 206.5 MHz, then

max Q*

100- |                           /

|                          *

|

L    |                         ..

o    |                        .  .

g    |                       .    .

|                      .      .

Q    |                     .        .

|                  .             .

(%)   |              .                    .

|         .                             .

|.                                              .

80- |____________________________________________________|

|                          |                         |

206                        206.3277432**

207                                         Log F (MHz)

* system dynamic peak

** excitation peak tune

*** What you see is not what you get.

------------------------------------------------

Gene Waldo  -  gwaldo@1stnetusa.com

Stuff most people don't know about:

"It isn't so much how you play the game,

but winning that counts."

X-Mailer: IPAD 2.0

Date: Fri, 15 May 1998 11:21:26 -0500

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: Q

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

At 07:39 AM 5/15/98 -0500, you wrote:

>Hi Mike,

>    You bring up a point that I've heard, but never really understood.  That

>is, if the Q is a function of reactance and resistance(which are in

>themselves inherent properties indepedent of current and voltage), then why

>does the Q change once the circuit is energized?

>

>thanks,sean

Sean,

You need to think of Q as be more than just X/R.

Think of it as being:

energy stored / energy lost

Some energy is lost through the ohmic resistance of the

wire but other losses come into play in RF circuits.

You have skin effect loss, corona loss, dielectric loss

and in Tesla coils the biggie is gap loss. Gap loss

accounts for most of the losses in a Tesla coil SYSTEM.

You need to think of a coil from a system perspective.

Not just a compilation of individual parts.

Beginners are often perplexed when after making improvements

to their coil that should improve performance they see little

or no gain.

That is because while they may have reduced losses in a particular

component their system wide losses are still quite high.

This is often seen when they change from saltwater bottle

caps to a good poly cap and dont see very much improvement

in spark length. Its usually because they are still using

the same crummy gap they started with. The improvement in

performance from the better caps is masked by the losses

in the poor gap.

Conversely gap improvements can be masked by excessive RF

dissipation in poor capacitors.

Hope this helps Sean.

TTYL!

Mike Hammer

mhammer@misslink.net

X-Mailer: IPAD 2.0

Date: Fri, 15 May 1998 11:36:00 -0500

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: Q

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

At 07:52 AM 5/15/98 -0700, you wrote:

>Mike Hammer wrote:

>>

>> At 08:07 PM 5/14/98 -0500, you wrote:

>> >Hi Sean,

>> >

>> >OK I'm beginning to understand but is Q something you can calculate or

>> assign a number to?  It seems that all of the parts of a tank circuit will

>> affect the Q of the system then right?

>> >Yes, no?  I thought I read somewhere that Q is like a quality rating.

>> Would this be a way of looking at it also?

>> >

>> >Tell me more.

>> >

>> >Thanks,

>> >Roy Minnich

>>

>> Roy,

>>

>> Q is indeed a quality rating or factor. The formula for

>> Q for an inductor is commonly given as:

>>

>> Q = Xl/R

>>

>> This means the the lower the resistance of the coil (or the higher

>> its reactance) the higher the Q. This shows why larger guage wire

>> for coils results in higher Q. Larger wire has lower resistance.

>> Also it shows one reason why a tall skinny candlestick type secondary

>> with many turns of small guage wire performs poorly. Low Q.

>>

> Put all that effort into keeping primary

>> circuit Q up and you'll get the most from your coil.

>>

>> TTYL!

>>

>> Mike Hammer

>> mhammer@misslink.net

>

>

>When tuning your primary tank, max 'Q' or max 'performance' is primarily

>a function of tuning or 'rocking-in' on the resonant frequency of the

>pri ckt coil, by means of the tap, all other factors being close, as I

>understand it. This tap position will change as various terminal

>capacitances are applied to the 2ndary.

>

>If one plots performance or Q vs Freq, given the general shape of

>resonant system peaks, you can see the critical effect of the tolerance

>on the precise tuning (matching) of the exitatation frequency to the

>system frequency.

>Perhaps I can illustrate with some numbers out of the air:

>   (This may not work, given our WYSINWYG*** mail format!)

>

>  If you calculate a resonant frequency of about 206.5 MHz, then

>

>                                       max Q*

>     100- |                           /

>          |                          *

>          |

>     L    |                         ..

>     o    |                        .  .

>     g    |                       .    .

>          |                      .      .

>     Q    |                     .        .

>          |                  .             .

>    (%)   |              .                    .

>          |         .                             .

>          |.                                              .

>      80- |____________________________________________________|

>          |                          |                         |

>          206                        206.3277432**

>207                                         Log F (MHz)

>

>           * system dynamic peak

>          ** excitation peak tune

>         *** What you see is not what you get.

>

>------------------------------------------------

>Gene Waldo  -  gwaldo@1stnetusa.com

Gene,

All that is certainly true. Maximum output and Q will occur

when everything is in exact resonance. But thats not exactly

what I meant when I said to maximize primary Q. After all

everyone should be running there coils at resonance.

What I meant was minimizing losses. This means reducing ohmic

losses through heavy conductors and tight well soldered or

bolted connections. Removing any sharp points to reduce

corona loss. Good gap design to minimize gap loss and

good poly film caps to reduce RF dissipation.

One note about poly caps. Just slapping some aluminum and poly

into a PVC pipe and pouring some mineral on top does not

constitute a low loss pulse cap. Air removal from the many layers

of poly and aluminum are essential. Not only from a voltage

breakdown perspective but from a loss perspective. Trapped

air inside the cap can and will ionize and corona will form.

This represents real power loss.

I once built a .01 plate cap with 60 mils of poly.

This was a single cap to be used at 12kv. Initial performance

was very poor. The culprit was trapped air in the many poly

layers. (10 X 6 mil) After considerable run time the performance

began to improve due to air being displaced by oil.

This cap eventually failed while in parallel with another

plate cap containing 6 series caps. The single 60 mil cap

failed while the 48 mil series string cap survived.

TTYL!

Mike Hammer

mhammer@misslink.net

X-Mailer: IPAD 2.0

Date: Fri, 15 May 1998 13:48:52 -0500

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Mike Hammer <mhammer@misslink.net>

Subject: RE: Ferrite ring chokes

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

At 05:23 PM 5/12/98 -0500, you wrote:

>Hi Mike,

>

>If I try them out on my 2 1/2 coil how will I know they are working or

not?  So far I ran the coil with my 12 kv 30 ma neon and a sheet of foil

laminated foam board for a cap that was way low on uF and got 8 in. arcs

with a 4 in. foil covered ball on the secondary.  I built a 8 gap fan

cooled RQ cylinder gap that works great but no safety gap yet.

Roy,

One way to tell if your chokes are working is by the activity

level of the safety gap. The intensity of the firings should

be greatly reduced by including the chokes in the circuit.

If you see no change you probably dont have enough inductance

and if the chokes tend to make things worse then they may be

ringing. If they are ringing some series resistors will help

to dampen things down.

>

>This is my first coil so what would a kickback do or look like.

>I'm guessing the safety would fire if I got a kickback?

>

>Thanks,

>Roy Minnich

Kickbacks can be mild or severe. Mild kickbacks will just flash

across the safety gap and not interfere with coil operation at all.

Severe kickbacks can destroy components. I have a very severe kickback

once. I was running 12kv/90ma with a single rebuilt neon. I was

getting around 40 inches of spark when a loud thump was heard

from the gap. I was wearing hearing protection and heard this thump

over the noise level of the coil. The thump was followed immediately

by a massive discharge across the safety gap and a foot long

streamer leaping out of the unpotted (dry) neon I was using.

The neon sustained major damage to one secondary and was unrepairable.

This event occurred despite RF chokes and a safety gap. Im not sure

what caused it but I did have the secondary very heavily loaded

with stacked toroids and had the coupling very tight. My guess I was

pushing things too hard and had a gap misfire due to oxidation and

fouling of the electrodes. Ive since spent extra time keeping the gap

electrodes clean and havnt had another kickback like that.

TTYL!

Mike Hammer

mhammer@misslink.net

From: Roy Minnich <rminnich@rolling-meadows.lib.il.us>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: RE: Q & poly cap construction

Date: Fri, 15 May 1998 13:59:42 -0500

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

Hi Mike and all,

So if you stick to the basics while you build/match components of your TC system using solidly connected low impedance conductors in the tank circuit and RF ground and all construction methods/materials are optimized to keep losses to a minimum you SHOULD end up with a high Q system whether you can do the math or not?  Good, sounds like we have one foot in the real world.  It's the same with many things. You model, you calculate then you build it and reality steps in.  I can work with that!  That's when the mechanics take over and tweak the system up to it's full potential.

A thought on rolled poly cap building:  I've seen it mentioned that when building flat plate caps with poly and foil it works well to put a little mineral oil between the layers as you build it up.  Sort of sticks it together AND keeps some air from getting between the layers.  So my idea or question is will this work while rolling a poly cap too?  Or will it just make a slippery, oily mess of everything???

Seems like it might be a good idea and help get a cap up to full working potential a little quicker as there will be less trapped air between the layers.

What do you think?

Thanks,

Roy Minnich

PS I set my aluminum foil laminated foam board cap on fire the other night.  Really exciting especially since I was running the coil in a room we had just carpeted.

No damage thanks to the self extinguishing foam type and my quick reaction cutting the power.  I think I'll work in the garage from here on out.

Mike Hammer wrote:

<SNIP>

One note about poly caps. Just slapping some aluminum and poly

into a PVC pipe and pouring some mineral on top does not

constitute a low loss pulse cap. Air removal from the many layers

of poly and aluminum are essential. Not only from a voltage

breakdown perspective but from a loss perspective. Trapped

air inside the cap can and will ionize and corona will form.

This represents real power loss.

I once built a .01 plate cap with 60 mils of poly.

This was a single cap to be used at 12kv. Initial performance

was very poor. The culprit was trapped air in the many poly

layers. (10 X 6 mil) After considerable run time the performance

began to improve due to air being displaced by oil.

This cap eventually failed while in parallel with another

plate cap containing 6 series caps. The single 60 mil cap

failed while the 48 mil series string cap survived.

TTYL!

Mike Hammer

mhammer@misslink.net

X-Mailer: IPAD 2.0

Date: Fri, 15 May 1998 14:21:31 -0500

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Mike Hammer <mhammer@misslink.net>

Subject: RE: Q & poly cap construction

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

At 01:59 PM 5/15/98 -0500, you wrote:

>Hi Mike and all,

>

>So if you stick to the basics while you build/match components of your TC

system using solidly connected low impedance conductors in the tank circuit

and RF ground and all construction methods/materials are optimized to keep

losses to a minimum you SHOULD end up with a high Q system weather you can

do the math or not?  Good, sounds like we have one foot in the real world.

It's the same with many things. You model, you calculate then you build it

and reality steps in.  I can work with that!  That's when the mechanics

take over and tweak the system up to it's full potential.

I agree completely.  Tesla coils do not model well mathematically.

They are very non linear systems. You can get in the ballpark

using some formulas but in the final analysis its the hands on

tweaking that results in best performance.

>

>A thought on rolled poly cap building:  I've seen it mentioned that when

building flat plate caps with poly and foil it works well to put a little

mineral oil between the layers as you build it up.  Sort of sticks it

together AND keeps some air from getting between the layers.  So my idea or

question is will this work while rolling a poly cap too?  Or will it just

make a slippery, oily mess of everything???

>Seems like it might be a good idea and help get a cap up to full working

potential a little quicker as there will be less trapped air between the

layers.

>

>What do you think?

Ive done this with plate caps. I actually built a plate cap

completely in oil. I dipped each poly sheet in oil before

adding to the stack and then worked the air bubbles out with

the fingers. It was a terrible mess. But the cap performed

well with no break in period. Unfortunately it failed in

service at 12kv because I had not yet learned the lesson that

single caps should never be used above 9kv. Since going to

series connected caps I havn't had a single cap failure.

I would think this would be difficult to do in a rolled cap.

For this reason when I built my rolled caps I pulled a vacuum

on them with an old dehumidifier compressor serving as a vacuum pump.

TTYL!

Mike Hammer

mhammer@misslink.net

Sender: tesl-err@emachine.com

From: "Fred Decker" <fdecker@csi.com>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Q & poly cap construction

Date: Fri, 15 May 1998 15:53:38 -0400

X-Mailer: IPAD 2.0

X-MimeOLE: Produced By Microsoft MimeOLE V4.72.2106.4

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Has anyone seen the solid state coil in a supplement to one of the electronic

magazines several years ago.  I built one but never finished hooking it up.

I heard the spark length wasn't too great.  It also called for 3 500pf caps in

parallel and I found a 1500pf doorknob to use rated for the voltage.  Is that a

mistake?  Should I have 3 parallel caps instead of one?  Also, how can I tweak

this or even use the secondary and possibly the primary for another coil?

Fred

From: Roy Minnich <rminnich@rolling-meadows.lib.il.us>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: RE: Q & poly cap construction

Date: Fri, 15 May 1998 15:37:51 -0500

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

Hi Fred,

What's the power source for your solid state coil.  I have plans for one that sounds similar.

Roy Minnich

----------

From: 	Fred Decker[SMTP:fdecker@csi.com]

Reply To: 	Tesla-2 Mailing List

Sent: 	Friday, May 15, 1998 2:53 PM

To: 	Tesla-2 Mailing List

Subject: 	Re: Q & poly cap construction

Has anyone seen the solid state coil in a supplement to one of the electronic magazines several years ago.  I built one but never finished hooking it up.  Heard the spark length wasn't too great.  It also called for 3 500pf caps in parallel and I found a 1500pf doorknob to use rated for the voltage.  Is that a mistake?  Should I have 3 parallel caps instead of one?  Also, how can I tweak this or even use the secondary and possibly the primary for another coil?

Fred

X-Originating-IP: [208.16.152.140]

From: "Bill the arcstarter" <arcstarter@hotmail.com>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Dayton ham and cheese fest

Date: Fri, 15 May 1998 13:53:21 PDT

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

Coilers,

Jeff Harvey (of tesla-2) and I survived day 1 of the Dayton Hamfest.

But - I didn't get to John Heck's booth until about 1:00 - and he

informs me that two or three fellow coilers were there at about 12:30!

Sorry to have missed you guys- drop me an E-mail off-list if you were

there! (I don't know who it was...)

I WILL be there at booth 2634 at 12:00 saturday sharp, unless there is a

torrential downpour or something...  Incidentally - this booth is one

row away from the Mendelson's MECI tent - the side with all the books.

John will not be there tomorrow (he says) so the spot will be empty

except for whoever is standing around...

So - what'd I get so far?  Lots of stuff! - 6-plug outlet strips for $1,

12 and 120 V muffin fans for $1, 1200 volt 100 amp 3-phase bridge

rectifier modules for $5, and an HP 204C sig gen (demo'd as working) for

$50.  I probably paid too much for that - but HEY - it works and I

wanted it!  Didn't see any doorknob caps, ignitrons, or anything really

industrial...

Also saw some possible tesla/HV buys at the Nebraska Surplus place - 40

KVDC 0.04uf pulse caps for $119, HV knife switches, Jennings vacuum

contactors and vac caps, etc.  Someone else had a 100uf 10 Kv Maxwell

pulse cap for (!) $450...

Hope to see you there tomorrow!  Bring your suntan lotion as it was 94

and clear today...

-Bill

Date: Fri, 15 May 1998 17:51:58 -0700

From: Gene Waldo <gwaldo@1stnetusa.com>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Organization: S.A.P.I.E.N.T. (Surly, Agressive, Politically Incorrect Enemies of Non Thinkers)

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

CC: comahal@cci-29palms.com

Subject: remote ops.....?

Sender: tesl-err@emachine.com

All,

Just for grins.  I'll be at family (Cc, above) all day Sunday near Palm

Springs CA (Sat. wedding type thing). Since I don't know how to readilly

transport my scattered lists of neat URLs, AND if any of you have neat

socko pics posted or other neat sites, AND if they get bored enough, AND

if some URLs were shipped to them, I could run a kind of guided tour of

TESLA LAND! I THINK!  Just a shot in the dark....

Maybe some one would even run with the idea and put up a kind of Tesla

"Reqruitment Poster" site... another shot in the dark...

I'm gonna get myself in the foot, here, in a minute....  ~~<[:-)

--------------------------------------------------

Gene Waldo -  gwaldo@1stnetusa.com  - "Igor in Residence"

---------------------------------------------------------------

What's good for the planet is more OZONE! See the machinery form up!

NEW STUFF!   http://www.misslink.net/mhammer/waldo.htm

From: "Scott Hayward" <smash@eisa.net.au>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Dayton ham and cheese fest

Date: Sat, 16 May 1998 20:02:37 +1000

X-Mailer: IPAD 2.0

X-MimeOLE: Produced By Microsoft MimeOLE V4.71.1712.3

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

>Didn't see any doorknob caps, ignitrons, or anything really

>industrial...

Would someone please tell me what an ignitron is.  I've wanted one though

because apparently you need it for a can crusher and there are a few things

you can use it for a tesla coil.

Thanks for any comments,

From Scott Hayward   smash@eisa.net.au

Getting high off ozone in Australia  :)

From: "Scott Hayward" <smash@eisa.net.au>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Ferrite ring chokes

Date: Sat, 16 May 1998 20:08:42 +1000

X-Mailer: IPAD 2.0

X-MimeOLE: Produced By Microsoft MimeOLE V4.71.1712.3

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

>I had a very severe kickback once. I was running 12kv/90ma with

>a single rebuilt neon. I was getting around 40 inches of spark when

>a loud thump was heard from the gap. I was wearing hearing protection

>and heard this thump over the noise level of the coil. The thump was

>followed immediately by a massive discharge across the safety gap and

>a foot long streamer leaping out of the unpotted (dry) neon I was using.

>The neon sustained major damage to one secondary and was unrepairable.

>

>This event occurred despite RF chokes and a safety gap. Im not sure

>what caused it but I did have the secondary very heavily loaded

>with stacked toroids and had the coupling very tight. My guess I was

>pushing things too hard and had a gap misfire due to oxidation and

>fouling of the electrodes. Ive since spent extra time keeping the gap

>electrodes clean and havnt had another kickback like that.

>Mike Hammer

>mhammer@misslink.net

Sounds like something I wouldn't want to experience!!!, and by the way,

speaking of chokes, I just picked up some pre wound chokes wound on heavy

laminated iron.  They should be great.  I am now not afraid to put on two

extra neon transformers that I have.  Just one problem though.  I tried to

unpot one of them.  I did this by making a fire and putting the neon in the

fire.  This melted most of the tar that I then poured out, but not all of

it.  How can I get the rest of the tar out.  If it is using solvents could

you please specify which ones.

From Scott Hayward  smash@eisa.net.au

Getting high off ozone in Australia  :)

X-Originating-IP: [208.16.152.181]

From: "Bill the arcstarter" <arcstarter@hotmail.com>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Ignitron

Date: Sat, 16 May 1998 04:55:33 PDT

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

Scott wrote:

>Would someone please tell me what an ignitron is.  I've wanted one though

>because apparently you need it for a can crusher and there are a few things

>you can use it for a tesla coil.

Scott,

An ignitron is an old water-cooled mercury pool tube used in the old

days for controlling high powered devices like motors, industrial spot

welders, etc.  A controlling terminal ionizes (?) some mercury which

causes the tube to conduct for the remainder of the AC half cycle.  Some

of the medium sized tubes were capable of gating 1000s of amps at 400

volts typical.

It is basically a switchable diode, largely resembling (in function)

today's SCR components.

I don't have any ignitrons but I saw some the other day... Hmm!

Out of respect for the list I will omit any ramblings about can

crushers, etc. :)

-Bill

Date: Sat, 16 May 1998 08:32:50 -0700

From: Gene Waldo <gwaldo@1stnetusa.com>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Organization: S.A.P.I.E.N.T. (Surly, Agressive, Politically Incorrect Enemies of Non Thinkers)

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Can crushers et al

Sender: tesl-err@emachine.com

Why is this technology so warily approached? Seems like it would have

been right up Nikola's alley! I've been aware of this M.O. for years,

but never was able to gather details....  Anyone got setup, before &

after pics? One of us was going to stick little quarters on his hat...?

Sounds like it would involve C&Imax circuits. Not very warm and fuzzy!

----------------------------------------------------

Gene Waldo -  gwaldo@1stnetusa.com  - "Igor in Residence"

---------------------------------------------------------------

What's good for the planet is more OZONE! See the machinery form up!

NEW STUFF!   http://www.misslink.net/mhammer/waldo.htm

X-Mailer: IPAD 2.0

Date: Sat, 16 May 1998 11:45:14 -0500

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: Can crushers et al

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

At 08:32 AM 5/16/98 -0700, you wrote:

>Why is this technology so warrily approached? Seems like it would have

>been right up Nikola's alley! I've been aware of this M.O. for years,

>but never was able to gather details....  Anyone got setup, before &

>after pics? One of us was going to stick little quarters on his hat...?

>

Gene,

Bill refrained from details about can crushers because this list

is set up for the discussion of Tesla coils. A little bit of delving

into side issues is OK but the main focus of the list is to remain

the construction and operation of Tesla coils.

TTYL!

Mike Hammer

mhammer@misslink.net

X-Mailer: IPAD 2.0

Date: Sat, 16 May 1998 11:56:06 -0500

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: Ferrite ring chokes

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

At 08:08 PM 5/16/98 +1000, you wrote:

>

>>I had a very severe kickback once. I was running 12kv/90ma with a single

>>rebuilt neon. I was getting around 40 inches of spark when a loud thump

>>was heard from the gap. I was wearing hearing protection and heard this

>>thump over the noise level of the coil. The thump was followed immediately

>>by a massive discharge across the safety gap and a foot long

>>streamer leaping out of the unpotted (dry) neon I was using.

>>The neon sustained major damage to one secondary and was unrepairable.

>>

>>This event occurred despite RF chokes and a safety gap. Im not sure

>>what caused it but I did have the secondary very heavily loaded

>>with stacked toroids and had the coupling very tight. My guess I was

>>pushing things too hard and had a gap misfire due to oxidation and

>>fouling of the electrodes. Ive since spent extra time keeping the gap

>>electrodes clean and havnt had another kickback like that.

>

>

>>Mike Hammer

>>mhammer@misslink.net

>

>Sounds like something I wouldn't want to experience!!!, and by the way,

>speaking of chokes, I just picked up some pre wound chokes wound on heavy

>laminated iron.  They should be great.

I dont believe your iron core chokes will be suitable.

Iron or steel cores are not meant for RF use. They work

well at 60 hz. Your chokes may be useful as current

limiting ballasts if wound with heavy enough wire.

For RF use you need ferrite materials or one of the many

varieties of powdered iron cores.

>Just one problem though.  I tried to

>unpot one of them.  I did this by making a fire and putting the neon in the

>fire.  This melted most of the tar that I then poured out, but not all of

>it.  How can I get the rest of the tar out.  If it is using solvents could

>you please specify which ones.

>

>From Scott Hayward  smash@eisa.net.au

I do not recommend melting the tar out of neons. The heat can damage

parts of the transformer and the remaining tar can be difficult

to remove. Neons from Jefferson Electric had plastic bindings

on the primary coil and plastic wedges between windings and core.

Using heat on these transformers will create a problem.

The best method is to chisel away the tar until the core can

be disassembled and the windings removed. Then dump everything

into containers of kerosene for a good soak. The kero will

dissolve the remaining tar. Several changes with fresh kero

will be required until everything is clean.

Kerosene is the safest most inexpensive solvent. Gasoline could

be used but is much more dangerous.

TTYL!

Mike Hammer

mhammer@misslink.net

From: Dsurfr <Dsurfr@aol.com>

Date: Sat, 16 May 1998 13:02:09 EDT

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: condensation on coils

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

I've got my coil set up in the basement & have recently noticed that there is

a bit of condensation (water) forming on the primary coil. I'm going to hook

up the dehumidifier & I assume that the condensation (which occurs only on the

primary - nothing else in the basement) is perhaps caused by a static charge

or the heated air condensing after I run it. I just wondered if anyone else

has experienced this problem. Thanks, Jim

Date: Sat, 16 May 1998 13:38:54 -0600

From: Jonathan Bazemore <jpbazemo@ouray.cudenver.edu>

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Ignitatron

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

I promise to stay on topic--being an avid coiler for a few years (my

last coil was terribly out of tune and Mike helped me out) I was wondering if

the ignitatron is a similar device to the cryotron or clystron tube?

I would be interested in high-repetition devices, improvised or otherwise,

that hold up under tremendous voltage and current requirements.  Chip has

some kind of Lucite timed breaker wheel hooked up to a syncrononous motor--it is a

real beauty!  There are two large, rounded electrodes , and when a metal rod

sticking out of a rotating wheel passes between the electrodes, a

high-current spark jumps the gaps.  Is this the preferred way of breakering the

primary coil circuit, or can one of the tube-type devices be used with a decent

lifetime?  Comments appreciated.

Mike Hammer wrote:

> At 08:32 AM 5/16/98 -0700, you wrote:

> >Why is this technology so warrily approached? Seems like it would have

> >been right up Nikola's alley! I've been aware of this M.O. for years,

> >but never was able to gather details....  Anyone got setup, before &

> >after pics? One of us was going to stick little quarters on his hat...?

> >

>

> Gene,

> Bill refrained from details about can crushers because this list

> is set up for the discussion of Tesla coils. A little bit of delving

> into side issues is OK but the main focus of the list is to remain

> the construction and operation of Tesla coils.

>

> TTYL!

>

> Mike Hammer

> mhammer@misslink.net

Date: Sat, 16 May 1998 13:42:15 -0600

From: Jonathan Bazemore <jpbazemo@ouray.cudenver.edu>

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: hi!

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

How are you doing, Mike?  What have you been up to?  Build any coils

lately?

Drop me a line, and let me know what's going on.

Mike Hammer wrote:

> At 08:32 AM 5/16/98 -0700, you wrote:

> >Why is this technology so warrily approached? Seems like it would have

> >been right up Nikola's alley! I've been aware of this M.O. for years,

> >but never was able to gather details....  Anyone got setup, before &

> >after pics? One of us was going to stick little quarters on his hat...?

> >

>

> Gene,

> Bill refrained from details about can crushers because this list

> is set up for the discussion of Tesla coils. A little bit of delving

> into side issues is OK but the main focus of the list is to remain

> the construction and operation of Tesla coils.

>

> TTYL!

>

> Mike Hammer

> mhammer@misslink.net

Date: Sat, 16 May 1998 13:46:49 -0600

From: Jonathan Bazemore <jpbazemo@ouray.cudenver.edu>

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: condensation on coils

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Hi there!  What kind of wire or copper tubing are you using?  Does the

condensation occur in association with running the coil, or when you don't

use it for a while?

Dsurfr wrote:

> I've got my coil set up in the basement & have recently noticed that there is

> a bit of condensation (water) forming on the primary coil. I'm going to hook

> up the dehumidifier & I assume that the condensation (which occurs only on the

> primary - nothing else in the basement) is perhaps caused by a static charge

> or the heated air condensing after I run it. I just wondered if anyone else

> has experienced this problem. Thanks, Jim

From: Dsurfr <Dsurfr@aol.com>

Date: Sat, 16 May 1998 18:50:37 EDT

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: condensation on coils

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

The condensation is now showing up on the secodary as well! I'm going to get

that de-humidifier going right now. It's been hot here in Michigan lately &

the A/C has been running so I think this is adding to the humidity (which has

been in the 90% range). I've been trying to tune it the past few days (I just

got it up & sparking last week) so I've been running it in short bursts a lot

- I just make sure I dry it off well first.

Thanks for the interest.    Jim

From: "Scott Hayward" <smash@eisa.net.au>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Heavy Iron chokes (was Ferrite ring chokes)

Date: Sun, 17 May 1998 12:49:12 +1000

X-Mailer: IPAD 2.0

X-MimeOLE: Produced By Microsoft MimeOLE V4.71.1712.3

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

>>Sounds like something I wouldn't want to experience!!!, and by the way,

>>speaking of chokes, I just picked up some pre wound chokes wound on heavy

>>laminated iron.  They should be great.

>

>I dont believe your iron core chokes will be suitable.

>Iron or steel cores are not meant for RF use. They work

>well at 60 hz. Your chokes may be useful as current

>limiting ballasts if wound with heavy enough wire.

>

>For RF use you need ferrite materials or one of the many

>varieties of powdered iron cores.

I don't understand.  Any choke should do if it has enough inductance right?

It has many turns and I'm sure the cores won't get saturated with the power

levels I'm using (~1000 W)

Date: Sun, 17 May 1998 00:15:23 -0400 (EDT)

From: Paul Anderson <madhobby@geeky1.ebtech.net>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Can crushers et al

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

On Sat, 16 May 1998, Mike Hammer wrote:

> Bill refrained from details about can crushers because this list

> is set up for the discussion of Tesla coils.

>

There is a list setup for discussing all hv-related issues(including

quartershrinkers, Farnsworth "fusor" type nuclear fusion reactors,

Cockraft-Walton voltage doublers, etc.), called the hvlist.

Unfortunately, I don't have the subscription details handy:(

TTYL!

---

Paul Anderson

madhobby@geeky1.ebtech.net

From: "Scott Hayward" <smash@eisa.net.au>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Can crushers et al

Date: Sun, 17 May 1998 15:08:12 +1000

X-Mailer: IPAD 2.0

X-MimeOLE: Produced By Microsoft MimeOLE V4.71.1712.3

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

>On Sat, 16 May 1998, Mike Hammer wrote:

>

>> Bill refrained from details about can crushers because this list

>> is set up for the discussion of Tesla coils.

>>

>There is a list setup for discussing all hv-related issues(including

>quartershrinkers, Farnsworth "fusor" type nuclear fusion reactors,

>Cockraft-Walton voltage doublers, etc.), called the hvlist.

>Unfortunately, I don't have the subscription details handy:(  TTYL!

This is the address of Steven Roys who runs the list.  I don't know how good

it is or if this is the list you were talking about but here is the address

sroys@anchorage.ab.umd.edu .

From Scott Hayward   smash@eisa.net.au

Getting high off ozone in Australia  :)

Date: Sun, 17 May 1998 00:19:28 -0700

From: mike nicholas <705728@ican.net>

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: condensation on coils

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Air conditioning removes moisture from a house, its better than a dehumidefier.

Dsurfr wrote:

> The condensation is now showing up on the secodary as well! I'm going to get

> that de-humidifier going right now. It's been hot here in Michigan lately &

> the A/C has been running so I think this is adding to the humidity (which has

> been in the 90% range). I've been trying to tune it the past few days (I just

> got it up & sparking last week) so I've been running it in short bursts a lot

> - I just make sure I dry it off well first.

> Thanks for the interest.    Jim

Date: Sun, 17 May 1998 03:49:49 -0400 (EDT)

From: Paul Anderson <madhobby@geeky1.ebtech.net>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Can crushers et al

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

On Sun, 17 May 1998, Scott Hayward wrote:

>

> This is the address of Steven Roys who runs the list.  I don't know how good

> it is or if this is the list you were talking about but here is the address

> sroys@anchorage.ab.umd.edu .

>

Yup, that's the one.  Ask him to put you on the list and he'll happily

oblige.  TTYL!

---

Paul Anderson

madhobby@geeky1.ebtech.net

X-Originating-IP: [208.16.152.141]

From: "Bill the arcstarter" <arcstarter@hotmail.com>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Can crushers et al

Date: Sun, 17 May 1998 03:33:50 PDT

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

Mike Hammer wrote:

>

>Gene,

>Bill refrained from details about can crushers because this list

>is set up for the discussion of Tesla coils. A little bit of delving

>into side issues is OK but the main focus of the list is to remain

>the construction and operation of Tesla coils.

Absolutely correct.  I don't want to see this list degenerate (like

certain other tesla-oriented listservs have) by excessive off-topic

postings (which decrease the signal to noise ratio) and resultant

flame-wars...  This list has a definite focus which I try to limit

myself to.

But you can ask me privately and I'll tell you how we did it - just like

other coilers on this list have done for me!!  Be forewarned - I am by

no means an expert in this area.

Other lists exist for misc. high voltage experimentation - although I am

not susbscribed myself.  But I did read all the archives once!

Stamp out free-energy zealots!

-Bill (the arcstarter!)

X-Originating-IP: [208.16.152.141]

From: "Bill the arcstarter" <arcstarter@hotmail.com>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Heavy Iron chokes (was Ferrite ring chokes)

Date: Sun, 17 May 1998 04:20:25 PDT

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

Coilers,

Scott wrote:

>>>Sounds like something I wouldn't want to experience!!!, and by the way,

>>>speaking of chokes, I just picked up some pre wound chokes wound on heavy

>>>laminated iron.  They should be great.

Then Mike Hammer wrote:

>>I dont believe your iron core chokes will be suitable.

>>Iron or steel cores are not meant for RF use. They work

>>well at 60 hz. Your chokes may be useful as current

>>limiting ballasts if wound with heavy enough wire.

>>

>>For RF use you need ferrite materials or one of the many

>>varieties of powdered iron cores.

Then Scott responded:

>I don't understand.  Any choke should do if it has enough inductance right?

>It has many turns and I'm sure the cores won't get saturated with the power

>levels I'm using (~1000 W)

I would think if the goal is to produce a lossy choke coil, then winding

it on regular old steel would indeed do the job.  The eddy current

losses will be relatively large.

The only question is whether the steel core will saturate or not at the

current levels in use.  This question really stands for ANY cored

solenoid - all materials will saturate - albeit at differing flux

levels...

Mike- any comments on this beyond what you've already mentioned??

I use chokes wound from #20 wire on insulated ferrite toroids in

conjunction with a SMALL bypass cap (200 pf) across the safety gap - I

can tell a difference in the safety gap activity when the choke is in

the circuit.  Don't use a larger bypass - it may cause excessive

resonant rise!

-Bill

From: wwl@netcomuk.co.uk (Mike Harrison)

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Data on Ignitrons, thyratrons, power valves etc. available

Date: Sun, 17 May 1998 14:55:52 GMT

Organization: White Wing Logic

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

I've just got hold of a load of data books covering industrial valves,

ignitrons, thyratrons (Hydrogen, mercury and inert gas), power valves

and other interesting stuff.

List at  http://www.netcomuk.co.uk/~wwl/data.html

If anyone needs data on specific devices I can scan them in - I'll be

putting some of the general notes & descriptions and pictures etc.

online sometime soon.

Has anyone got any hints, circuits etc for H2 thyratron controlled

coils? - I've found from the above books that one of the big valves in

my collection is a h2 thyratron (EEV FX2519A/5949a : 500A Ipk max,

25KV Vf max). Any ideas if it might give better results than a TCBOR

cylindrical gap on a 500W 7KV 4" coil?

____                                                           ____

_/ L_/  Mike Harrison / White Wing Logic / wwl@netcomuk.co.uk  _/ L_/

_/ W_/  Hardware & Software design / PCB Design / Consultancy  _/ W_/

/_W_/  Industrial / Computer Peripherals / Hazardous Area      /_W_/

X-Mailer: IPAD 2.0

Date: Sun, 17 May 1998 12:58:11 -0400

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Jim Ryan <jryan@exis.net>

Subject: RE: Q & poly cap construction

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Hi all, I have finished my 4.5" x 21" 24GA. primary. One or two

overlaps on the wire, is that a problem ?

Received my 30 ma. 12k trans. from Payne. Works great.

I plan to make rolled caps, I'm waiting on 4 rolls of .005 thick

6" x 100" brass shin stock.

Mineral oil ??  is that the same as baby oil ?

1- 2"x6" inductor 26GA. wire rolled.

Spark gap started. Sure is hard to get the spacing correct, I'm

making a 9 part flat 3/4" ID copper pipe on wood.

Thanks for all the help so far.

Jim Ryan

X-Mailer: IPAD 2.0

Date: Sun, 17 May 1998 12:34:19 -0500

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: Heavy Iron chokes (was Ferrite ring chokes)

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

At 04:20 AM 5/17/98 PDT, you wrote:

>Coilers,

>Scott wrote:

>>>>Sounds like something I wouldn't want to experience!!!, and by the

>>>>way,speaking of chokes, I just picked up some pre wound chokes wound on

>>>>heavy laminated iron.  They should be great.

>

>Then Mike Hammer wrote:

>>>I dont believe your iron core chokes will be suitable.

>>>Iron or steel cores are not meant for RF use. They work

>>>well at 60 hz. Your chokes may be useful as current

>>>limiting ballasts if wound with heavy enough wire.

>>>

>>>For RF use you need ferrite materials or one of the many

>>>varieties of powdered iron cores.

>

>Then Scott responded:

>>I don't understand. Any choke should do if it has enough inductance

>>right? It has many turns and I'm sure the cores won't get saturated

>>with the power levels I'm using (~1000 W)

>

>Then Bill interjected:

>I would think if the goal is to produce a lossy choke coil, then winding

>it on regular old steel would indeed do the job.  The eddy current

>losses will be relatively large.

>

>The only question is whether the steel core will saturate or not at the

>current levels in use.  This question really stands for ANY cored

>solenoid - all materials will saturate - albeit at differing flux

>levels...

>

>Mike- any comments on this beyond what you've already mentioned??

I can see a couple problems with Scott's steel core chokes.

First of all you are right we do want a lossy choke and

the RF losses in such a choke would be huge. Eddy current

and hysteresis losses would be huge in a iron core choke

used at RF. All this loss results in heating of the core.

This might not be a problem at low power but it wont take

much RF to heat up an iron core. Iron core chokes and transformers

are never used above audio frequencies.

Further your choke must not exhibit any loss at 60hz.

His chokes sound like they may be something like flourescent

light ballasts. You certainly dont want your RF chokes limiting

your 60 hz charging current.

And if they are flourescent light ballasts or something similar

then they arent likely to enjoy be subjected to 9-15kv and

several times that in RF pulses. The insulation just wont be

up to it.

>

>I use chokes wound from #20 wire on insulated ferrite toroids in

>conjunction with a SMALL bypass cap (200 pf) across the safety gap - I

>can tell a difference in the safety gap activity when the choke in in

>the circuit.  Don't use a larger bypass - it may cause excessive

>resonant rise!

>-Bill

>

I very much agree. In fact I dont use bypass caps at all.

If you do use them make them small. Certainly nothing over 250 pf.

TTYL!

Mike Hammer

mhammer@misslink.net

X-Mailer: IPAD 2.0

Date: Sun, 17 May 1998 12:45:17 -0500

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Mike Hammer <mhammer@misslink.net>

Subject: RE: Q & poly cap construction

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

At 12:58 PM 5/17/98 -0400, you wrote:

>Hi all, I have finished my 4.5" x 21" 24GA. primary. One or two

>overlaps on the wire, is that a problem ?

If you can fix the overlaps I would. If you cant then dont worry

about it. At higher power levels the overlapped wire can arc

to the wire under it causing a shorted turn. Not a good thing.

If that happens then cut out the offending turn and splice

it back together.

>

>Received my 30 ma. 12k trans. from Payne. Works great.

>

>I plan to make rolled caps, I'm waiting on 4 rolls of .005 thick

>6" x 100" brass shin stock.

>Mineral oil ??  is that the same as baby oil ?

Yes. Baby oil is mineral oil but with a couple extras thrown in.

They add fragrance and tocopherals (vitamin E). What they

do to the electrical properties of the oil I cant say.

But I know of at least one person that used baby oil with

complete success. He had the added benefit of nice smelling caps. :)

>

>1- 2"x6" inductor 26GA. wire rolled.

1 inductor? If this is meant to be an RF choke then you need 2.

One goes in each HV lead. Dont forget the damping resistors that

are usually necessary with air core chokes and the safety gap.

>

>Spark gap started. Sure is hard to get the spacing correct, I'm

>making a 9 part flat 3/4" ID copper pipe on wood.

Forget mounting it on wood. Wood is too conductive for this kind

of operation. A single arc and you have nice very conductive carbon.

Mount your electrodes on a piece of heavy plastic. And space the

electrodes up slightly above the plastic to reduce problems with

heat warping or melting the plastic. Use plenty of airflow.

TTYL!

Mike Hammer

mhammer@misslink.net

X-Mailer: IPAD 2.0

Date: Sun, 17 May 1998 17:16:57 -0400

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Jim Ryan <jryan@exis.net>

Subject: RE: Q & poly cap construction

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Mike, OK no wood for the gaps. I went out in the garage and found

a round plastic outdoor light cover 5 1/2" in diameter. I plan to put the 9

3/4" pipes on the inside and the attaching bolts on the out.

It has a 2 7/8" mounting hole at the base that I plan to us as

the air flow for my new shop vac. I guess I'll have to make some

strategic extra holes in the top for gap cooling.

Jim

Date: Mon, 18 May 1998 01:06:27 -0700

From: mike nicholas <705728@ican.net>

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: rQ gap and ferrite chokes

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

I'm running 15 kv at 60ma, how many gaps and what spacing should I have between

them, I was thinking of using 7- 3/4' X 4" copper mounted in a piece of 4" pvc.

Will that do?

When you say that you need at least 2mH of inductance for your ferrite chokes,

do you mean micro or milli Henries?

Sender: tesl-err@emachine.com

From: "Fred Decker" <fdecker@csi.com>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Q & poly cap construction

Date: Sun, 17 May 1998 22:03:01 -0400

X-Mailer: IPAD 2.0

X-MimeOLE: Produced By Microsoft MimeOLE V4.72.2106.4

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

The power source for the solid-state tesla coil is a simple 555 timer feeding power transistors into a car ignition coil.  Does that sound like what you have?

Fred

X-Mailer: IPAD 2.0

Date: Mon, 18 May 1998 00:29:35 -0500

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: rQ gap and ferrite chokes

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

At 01:06 AM 5/18/98 -0700, you wrote:

>  I'm running 15 kv at 60ma, how many gaps and what spacing should I have

between them, I was thinking of using 7- 3/4' X 4" copper mounted in a

piece of 4" pvc.

>Will that do?

7 electrodes means only 6 gaps. Thats not quite enough for 15kv.

You can fit ten 3/4 inch diameter electrodes for 9 series gaps

in a piece of 4 inch PVC. However you will need to add a shield

between the first and last electrode to prevent the arc from cheating

and arcing between those electrodes. I used a 1/4 inch thick piece

of plexiglass in my gap. Space the electrodes .03 inches apart

and that will give you .270 inch maximum gap. That should work well.

>

>When you say that you need at least 2mH of inductance for your ferrite

chokes, do you mean micro or milli Henries?

Milli henries.

TTYL!

Mike Hammer

mhammer@misslink.net

Date: Mon, 18 May 1998 01:53:04 -0600

From: Jonathan Bazemore <jpbazemo@ouray.cudenver.edu>

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: re: dielectric strength.

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Very dumb questions: what is the dielectric strength of 10w40,

and what is the dielectric strength of mineral oil?  My reference books

are in storage, and a yahoo search turns up dielectric strength figures

for PTFE but not oil.  Thanks for your help.

BTW, that WinTesla program is real nice.  Looks like I need a

capacitor 100 times bigger than what I got now, tho.

X-Mailer: IPAD 2.0

Date: Mon, 18 May 1998 11:54:59 -0500

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Mike Hammer <mhammer@misslink.net>

Subject: re: dielectric strength.

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

At 01:53 AM 5/18/98 -0600, you wrote:

>  Very dumb questions: what is the dielectric strength of 10w40, and

>what is the dielectric strength of mineral oil?  My reference books are in

>storage, and a yahoo search turns up dielectric strength figures for PTFE but

>not oil.

>Thanks for your help.

The dielectric strength of automotive oils and lubricants is

very low. They are an extremely poor choice. They contain

many semi-metallic anti wear and anti foam and anti who knows

what additives.

Mineral oil has a dielectric strength of around 200 VAC per mil.

This is highly dependant on the moisture content of the oil with

better grades having less moisture.

Note that this is less than the dielectric strength of polyethylene

but oil has the added advantage of supressing corona. Thats why the

poly/oil combination is so good. It combines low RF dissipation and

good dielectric strength with corona supression.

TTYL!

Mike Hammer

mhammer@misslink.net

Date: Mon, 18 May 1998 11:45:57 -0600

From: Jonathan Bazemore <jpbazemo@ouray.cudenver.edu>

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: dielectric strength.

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

So a capacitor made out of foil and polyethylene, submerged in a

mineral oil bath, is ideal?

Mike Hammer wrote:

> At 01:53 AM 5/18/98 -0600, you wrote:

> >  Very dumb questions: what is the dielectric strength of 10w40, and

> >what is the dielectric strength of mineral oil?  My reference books are in

> >storage, and a yahoo search turns up dielectric strength figures for PTFE but

> >not oil.

> >Thanks for your help.

>

> The dielectric strength of automotive oils and lubricants is

> very low. They are an extremely poor choice. They contain

> many semi-metallic anti wear and anti foam and anti who knows

> what additives.

>

> Mineral oil has a dielectric strength of around 200 VAC per mil.

> This is highly dependant on the moisture content of the oil with

> better grades having less moisture.

> Note that this is less than the dielectric strength of polyethylene

> but oil has the added advantage of supressing corona. Thats why the

> poly/oil combination is so good. It combines low RF dissipation and

> good dielectric strength with corona supression.

>

> TTYL!

>

> Mike Hammer

> mhammer@misslink.net

From: Roy Minnich <rminnich@rolling-meadows.lib.il.us>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: RE: Q & poly cap construction

Date: Mon, 18 May 1998 14:04:29 -0500

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

Hi Fred,

That sounds just like the one I built. I still have the article.  The version I built

was a simplified model using the 555 timer circuit to rapidly switch a 2N3055 on and

off at and adjustable frequency.  The 2N3055 then powers a high energy auto ignition

coil I had lying around.  It's a fun circuit for making variable frequency arcs and a

mini Jacob's ladder but it's a lousy power supply for the  Tesla coil described in the article.  I built the coil too and it runs much better off my 12kv 30ma neon giving out

8 in. arcs.

Roy Minnich

----------

From: 	Fred Decker[SMTP:fdecker@csi.com]

Reply To: 	Tesla-2 Mailing List

Sent: 	Sunday, May 17, 1998 9:03 PM

To: 	Tesla-2 Mailing List

Subject: 	Re: Q & poly cap construction

The power source for the solid-state tesla coil is a simple 555 timer feeding power transistors into a car ignition coil.  Does that sound like what you have?

Fred

-----

X-Mailer: IPAD 2.0

Date: Mon, 18 May 1998 17:33:07 -0500

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: dielectric strength.

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

At 10:41 PM 5/18/98 -0700, you wrote:

>At 11:45 18/05/98 -0600, you wrote:

>

>snip:

>>> Mineral oil has a dielectric strength of around 200 VAC per mil.

>>> This is highly dependant on the moisture content of the oil with

>>> better grades having less moisture.

>

>Hi all

>	Cant you just heat the oil to a temperature above the boiling point of

>water to drive off any moisture?

>

>bob golding lightning without rain

>

>bob yubba@clara.net

>

Yea I suppose. But do you really want a couple gallons

of hot oil cooking on your stove?

Some oils are highly hygroscopic, meaning they readily absorb

water. Some transformer oils are like that and contain inhibitors

to prevent that.

Ive never had any problems using the cheapest grades of mineral oil

I could find. The cheapest source was the local veterinarian.

Works just fine.

TTYL!

Mike Hammer

mhammer@misslink.net

X-Mailer: IPAD 2.0

Date: Mon, 18 May 1998 19:26:54 -0400

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Jim Ryan <jryan@exis.net>

Subject: Re: Plastic not wood

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Mike, OK no wood for the gaps. I went out in the garage and found

a round plastic outdoor light cover 5 1/2" in diameter. I plan to put the 9

3/4" x 2" pipes on the inside and the attaching bolts on the out.

It has a 2 7/8" mounting hole at the base that I plan to us it as

the air flow for my new shop vac. I guess I'll have to make some

strategic extra holes in the top for gap cooling.

Jim

Date: Mon, 18 May 1998 20:50:20 -0600

From: Jonathan Bazemore <jpbazemo@ouray.cudenver.edu>

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: dielectric strength.

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

What is the dielectric strength of baby oil?  The reason I ask is because the

local dollar store is changing hands, and they are getting rid of excess stock.

After the coiler tradition, I'm trying to scrounge a bit.

bob golding wrote:

> At 11:45 18/05/98 -0600, you wrote:

>

> snip:

> >> Mineral oil has a dielectric strength of around 200 VAC per mil.

> >> This is highly dependant on the moisture content of the oil with

> >> better grades having less moisture.

>

> Hi all

>         Cant you just heat the oil to a temperature above the boiling point of

> water to drive off any moisture?

>

> bob golding lightning without rain

>

> bob yubba@clara.net

Date: Mon, 18 May 1998 20:52:13 -0600

From: Jonathan Bazemore <jpbazemo@ouray.cudenver.edu>

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Polyethylene.

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

I got some 4 mil polyethylene at Builder's Square today.  The size

is 3 x 50 feet.  Is this the right type to make a capacitor, if I plan

to submerge it in oil?

X-Mailer: IPAD 2.0

Date: Mon, 18 May 1998 22:41:06 -0700

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: bob golding <yubba@clara.net>

Subject: Re: dielectric strength.

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

At 11:45 18/05/98 -0600, you wrote:

snip:

>> Mineral oil has a dielectric strength of around 200 VAC per mil.

>> This is highly dependant on the moisture content of the oil with

>> better grades having less moisture.

Hi all

Cant you just heat the oil to a temperature above the boiling point of

water to drive off any moisture?

bob golding lightning without rain

bob yubba@clara.net

Date: Mon, 18 May 1998 23:43:47 -0700

From: mike nicholas <705728@ican.net>

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Q & poly cap construction

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

I have one almost the same except I use it on 110 V, it uses a quadrak to chop part

of the ac waveform, it was calles a mini tesla coil, but the frequency was much too low

to get corona without a grounded rod.

Roy Minnich wrote:

> Hi Fred,

>

> That sounds just like the one I built. I still have the article.  The version I built was a simplified model using the 555 timer circuit to rapidly switch a 2N3055 on and off at and adjustable frequency.  The 2N3055 then powers a high energy a

uto ignition coil I had lying around.  It's a fun circuit for making variable frequency arcs and a mini Jacob's ladder but it's a lousy power supply for the  Tesla coil described in the article.  I built the coil too and it runs much better off

my 12kv 30ma neon giving out 8 in. arcs.

>

> Roy Minnich

>

> ----------

> From:   Fred Decker[SMTP:fdecker@csi.com]

> Reply To:       Tesla-2 Mailing List

> Sent:   Sunday, May 17, 1998 9:03 PM

> To:     Tesla-2 Mailing List

> Subject:        Re: Q & poly cap construction

>

> The power source for the solid-state tesla coil is a simple 555 timer feeding power transistors into a car ignition coil.  Does that sound like what you have?

>

> Fred

> -----

>

Date: Mon, 18 May 1998 23:51:42 -0700

From: mike nicholas <705728@ican.net>

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: rq gap idea

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

I was thinking of making an rq gap with 10 pieces of 3/4" copper, but

instead of putting them in a 4" piece of pvc, I was thinking of putting

them between two pieces of plexy glass and mount an old dust-buster  as

the blower. Good idea?

X-Mailer: IPAD 2.0

Date: Tue, 19 May 1998 11:42:04 -0500

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: dielectric strength.

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

At 08:50 PM 5/18/98 -0600, you wrote:

>What is the dielectric strength of baby oil?  The reason I ask is because the

>local dollar store is changing hands, and they are getting rid of excess stock.

>After the coiler tradition, I'm trying to scrounge a bit.

Dont know exactly. But baby oil IS mineral oil with a little

fragance and vitamin E thrown in. Assuming the additives

dont degrade the electrical properties then it should be

good for around 200 VAC per mil same as mineral oil.

Ive never used baby oil but know one coiler that did with success.

TTYL!

Mike Hammer

mhammer@misslink.net

X-Mailer: IPAD 2.0

Date: Tue, 19 May 1998 11:45:31 -0500

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: rq gap idea

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

At 11:51 PM 5/18/98 -0700, you wrote:

>I was thinking of making an rq gap with 10 pieces of 3/4" copper, but

>instead of putting them in a 4" piece of pvc, I was thinking of putting

>them between two pieces of plexy glass and mount an old dust-buster  as

>the blower. Good idea?

Sounds fine to me Mike. You will want to space the electrodes

away from the plexiglass mounts a bit to prevent melting or

scorching the plexiglass. And I dont think a dust buster

pulls enough air to be effective. Use it if you havnt got

anything else but I would look for something a bit stronger.

TTYL!

Mike Hammer

mhammer@misslink.net

Sender: tesl-err@emachine.com

From: "Fred Decker" <fdecker@csi.com>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Q & poly cap construction

Date: Tue, 19 May 1998 14:03:04 -0400

X-Mailer: IPAD 2.0

X-MimeOLE: Produced By Microsoft MimeOLE V4.72.2106.4

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

I was thinking of having it work with the auto coil (can you recommend a high power one?), then using the same power supply to power a neon as another option.  What do you think of that?  I guess I might not be able to deliver enough current to the neon with it, but I am not sure how important this is yet.

I changed a resistor for a pot to also vary the frequency of the pulse timer.  Any other comments of info you had would be great.

Fred

-----Original Message-----

From: Roy Minnich <rminnich@rolling-meadows.lib.il.us>

To: Tesla-2 Mailing List <tesla-2@emachine.com>

Date: Monday, May 18, 1998 3:09 PM

Subject: RE: Q & poly cap construction

Hi Fred,

That sounds just like the one I built. I still have the article.  The version I built was a simplified model using the 555 timer circuit to rapidly switch a 2N3055 on and off at and adjustable frequency.  The 2N3055 then powers a high energy auto ignition coil I had lying around.  It's a fun circuit for making variable frequency arcs and a mini Jacob's ladder but it's a lousy power supply for the  Tesla coil described in the article.  I built the coil too and it runs much better off my 12kv 30ma neon giving out 8 in. arcs.

Roy Minnich

----------

From: Fred Decker[SMTP:fdecker@csi.com]

Reply To: Tesla-2 Mailing List

Sent: Sunday, May 17, 1998 9:03 PM

To: Tesla-2 Mailing List

Subject: Re: Q & poly cap construction

The power source for the solid-state tesla coil is a simple 555 timer feeding power transistors into a car ignition coil.  Does that sound like what you have?

Fred

-----

From: Roy Minnich <rminnich@rolling-meadows.lib.il.us>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: RE: Polyethylene.

Date: Tue, 19 May 1998 14:28:56 -0500

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

Hi,

That's the right stuff.  Watch out for imperfections and dirt in the film.  Use only near perfect sections.  Remember poly film is only good for max 375 volts AC per mil (.001 inch) not the 1200 volts (DC) you see in many lists.

Somebody bought out the Builder's Square near me.

Roy Minnich

----------

From: 	Jonathan Bazemore[SMTP:jpbazemo@ouray.cudenver.edu]

Reply To: 	Tesla-2 Mailing List

Sent: 	Monday, May 18, 1998 9:52 PM

To: 	Tesla-2 Mailing List

Subject: 	Polyethylene.

I got some 4 mil polyethylene at Builder's Square today.  The size

is 3 x 50 feet.  Is this the right type to make a capacitor, if I plan

to submerge it in oil?

From: Roy Minnich <rminnich@rolling-meadows.lib.il.us>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: RE: Q & poly cap construction

Date: Tue, 19 May 1998 14:48:36 -0500

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

Hi Fred,

I don't think you will get very far using the 555/spark coil circuit to power a neon.  Voltage will be way to high on the input side and the amps WAY too low.  The pot idea

is a good one as I built mine with one and it's a nice feature.  There is a GIF of a

circuit that is very similar to the one I built on Gary Weaver's page at http://home.earthlink.net/~gweaver

Look on the "High Voltage" page for a circuit called "High Voltage".  Gary has a lot

of neat info on T coils and other high voltage goodies.

The auto coil I used was from a 1978 Chevy C-10 pick up with a 305 engine.  It's a good

coil to use because it has external wires to each of the windings.  They price out at

about $20 at auto parts stores so you might get one from an auto salvage yard at a savings

or just spend your $20 on a neon from Tedd Payne.

Later,

Roy Minnich

----------

From: 	Fred Decker[SMTP:fdecker@csi.com]

Reply To: 	Tesla-2 Mailing List

Sent: 	Tuesday, May 19, 1998 1:03 PM

To: 	Tesla-2 Mailing List

Subject: 	Re: Q & poly cap construction

I was thinking of having it work with the auto coil (can you recommend a high power one?), then using the same power supply to power a neon as another option.  What do you think of that?  I guess I might not be able to deliver enough current to the neon with it, but I am not sure how important this is yet.

I changed a resistor for a pot to also vary the frequency of the pulse timer.  Any other comments of info you had would be great.

Fred

-----Original Message-----

From: Roy Minnich <rminnich@rolling-meadows.lib.il.us>

To: Tesla-2 Mailing List <tesla-2@emachine.com>

Date: Monday, May 18, 1998 3:09 PM

Subject: RE: Q & poly cap construction

Hi Fred,

That sounds just like the one I built. I still have the article.  The version I built was a simplified model using the 555 timer circuit to rapidly switch a 2N3055 on and off at and adjustable frequency.  The 2N3055 then powers a high energy auto ignition coil I had lying around.  It's a fun circuit for making variable frequency arcs and a mini Jacob's ladder but it's a lousy power supply for the  Tesla coil described in the article.  I built the coil too and it runs much better off my 12kv 30ma neon giving out 8 in. arcs.

Roy Minnich

----------

From: Fred Decker[SMTP:fdecker@csi.com]

Reply To: Tesla-2 Mailing List

Sent: Sunday, May 17, 1998 9:03 PM

To: Tesla-2 Mailing List

Subject: Re: Q & poly cap construction

The power source for the solid-state tesla coil is a simple 555 timer feeding power transistors into a car ignition coil.  Does that sound like what you have?

Fred

-----

X-Originating-IP: [199.217.140.22]

From: "Bill the arcstarter" <arcstarter@hotmail.com>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: more on car ign. coil tomfoolery

Date: Tue, 19 May 1998 13:20:14 PDT

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

It was written:

>I was thinking of having it work with the auto coil (can you recommend a

>high power one?), then using the same power supply to power a neon as

I don't think you'll have enough drive to power a neon - at least not

with a low voltage DC supply like you'd use to drive a car coil.

Also beware - if you don't use the proper drive transistor or provide

some sort of clipping against the inductive back-voltage - your

transistor will be destroyed quickly.  I learned this empirically!  I

now use the R-rated Harris  MOSFETS (such as the IRF840R / IRF240R)

which contain an integral back-blast zener...

Placing an external zener from the gate to the source will also absorb

noise spikes which would otherwise puncture the gate structures of the

FET.  Worth the extra $0.50 definitely.

Another trick - you can power a 12V coil off of >50VDC if you are

CAREFUL to limit the max "ON" time of your transistor.  This limits the

max current while allowing for a faster operation due to the faster

current rise rate.  However - if your FET shorts out (the usual failure

mode) then your coil is toast! :)

I also used the TIP120 bipolar transistors - which are a bit dated but

much tougher than FETs (as far as getting killed by errant HV noise).

>another option.  What do you think of that?  I guess I might not be able

>to deliver enough current to the neon with it, but I am not sure how

>important this is yet.

>

>I changed a resistor for a pot to also vary the frequency of the pulse

>timer.  Any other comments of info you had would be great.

I had reasonable sucess using a variable frequency/variable duty cycle

drive made out of a 555 and an LM339 comparator.   Use the 555 in the

usual way as a variable frequency source.

Connect one input of one of the LM339 comparators to pin 2 of the 555.

Remember - pin 2 produces a high-impedance triangular (more or less)

waveform.

Connect the other input of the same LM339 comparator to the wiper of a

10K pot.  Connect the outer pins of that pot to +5 and ground (or

whatever power supply you are using, etc).

Use the output of the comparator to drive your transistor/fet or

whatever.  Remember - the LM339 is open collector output so you might

need a pullup as a crude base/gate drive...  A purist would build a

proper gate drive cct.  I'm not a purist - I'm a realist! :)

The pot associated with the LM339 will let you pick the duty cycle of

the drive signal from 0 to 100 %, independent of the frequency.

I spent lots of time fooling with these car coil circuits - but

eventually  faced the truth that sometimes there is just no substitute

for raw power (like you have with a neon xfmr) etc.  I used Dodge pickup

and Ford Escort coils 'cause that's what I had in the junk bin...

-Bill the arcstarter

Starting arcs in Cinci, OH

http://www.geocities.com/CapeCanaveral/Hangar/6160

X-Mailer: IPAD 2.0

Date: Tue, 19 May 1998 23:22:00 +0300

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Marco Denicolai <marco@vistacom.fi>

Subject: Hello and first questions

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Hello to everybody on the list!

I'm a new Tesla Coil builder and I'm building a 4"-secondary coil system. The

tank circuit is fed with 8KV generated from microwave oven surplus parts.

And, of course, I got a lot of questions... :)

******************************************************************************

Question n.1

************

Here is an ASCII drawing of my tentative power supply.

It is made of 2 sets of transformer + diode +

capacitor from two microwave ovens. The transformers give about 2KV, which is

doubled with capacitor + diode to 4KV. Put two of these is series and you

get 8KV. See the picture below:

| |

--------|| ||-----| |----*---------- -

* || || *   | |    |

|| ||          -----

220V        || ||          \   /

|| ||           \ /

|| ||          -----

|| ||            |

--------|| ||------------*--- GND

|

|

--------|| ||------------*

* || || *          |

|| ||          -----

|| ||          \   /

220V       || ||           \ /

|| ||          -----

|| ||     | |    |

--------|| ||-----| |----*----------- +

| |

If you are carefull with transformer phasing, you get 8KV pulses at

25 Hz (Europe = 220V @ 50Hz !). Connect the phases wrong and you'll get

4KV pulses at 50 Hz. Between + and - terminals I can get a 0.37" continuous

spark between a simple 2-screw gap.

Any problem to feed a TC with this (forget about filtering for now)?

******************************************************************************

Question n.2

************

I got some surplus Maxwell capacitors model 30561 0.1uF 25KV with following

specs:

Capacitance: 0.1uF 25KV

Rated peak current: 5 KA

Max peak current: 25 KA

Max ripple freq.: 10 KHz

Max RMS current: 25 A RMS

Max Dissipation factor: 0.3% @ 120 Hz

Inductance: 80 nH

Dielectric: polypropylene and paper, oil is I.P.B.P.

They are enclosed in metal cans and have got ceramic terminals.

I intend to use two of them is SERIES (0.05 uF 50 KV).

Will they work with my Tesla Coil (frequency about 150 KHz) or will they

warm-up in 10 seconds and blow?

******************************************************************************

Question n.3

************

One question bothers me: this might be a stupid one...

Because I have read often that a discharge from the secondary stroke the

primary and damaged something, isn't it possible to shield the primary

with a sheet of acryl (or other plastic)?

I tought to cover the primary with a sheet of 5 mm acryl plastic and, on

the top of it, place the strike rail.

Could it possibly work?

-----------

|           |         toroid

-----------

|       |

|       |

|       |

|       |

*     |       |     *     strike rail

------------------------   acryl shield

* * * |       | * * *     primary (3 turns shown)

---------------------------- coil base

******************************************************************************

Best Regards

________________________________________________________________________

Marco Denicolai                   Vista Communication Instruments, Inc.

Hardware Development Manager      www.vistacom.fi

marco@vistacom.fi                 Kaisaniemenkatu 13 A

fax:    +358-9-622-5610           SF-00100 HELSINKI

phone:  +358-9-622-623-15         Finland

Remember, Murphy was an optimist! I am not...

________________________________________________________________________

From: Roy Minnich <rminnich@rolling-meadows.lib.il.us>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: RE: rq gap idea

Date: Tue, 19 May 1998 15:23:35 -0500

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

Hello all,

I have seen construction ideas for RQ cylinder gaps where the builders use threaded

bolts with nuts and washers and epoxy glue to hold the copper tubes in place.

I built a RQ cylinder gap by drilling holes in the copper tubes and threading them

for 8-32 machine screws.  I use steel cap screws with flat washers to secure the tubes

in the PVC pipe.  Adjusting the gaps is easily done by locking the center tube in place

and setting the gaps to each side of center by inserting a hacksaw blade between the

tubes and tightening the cap screws with an Allen driver.  The tubes do not move and

stay in adjustment.  This has worked fine for a gap I made using 1/2 in. copper tubes

and using the thicker walled refrigeration tubing.  I have purchased threaded inserts

that can be locked into thin metal and have tried a few on some test sections of tubing. These inserts are rock solid and would work well in thinner walled copper tubing where tapping threads wouldn't work well.

This construction method would seem to solve several problems involved in making RQ

type gaps.

Roy Minnich

----------

From: 	mike nicholas[SMTP:705728@ican.net]

Reply To: 	Tesla-2 Mailing List

Sent: 	Tuesday, May 19, 1998 1:51 AM

To: 	Tesla-2 Mailing List

Subject: 	Re: rq gap idea

I was thinking of making an rq gap with 10 pieces of 3/4" copper, but

instead of putting them in a 4" piece of pvc, I was thinking of putting

them between two pieces of plexy glass and mount an old dust-buster  as

the blower. Good idea?

Date: Tue, 19 May 1998 15:07:08 -0700

From: Gene Waldo <gwaldo@1stnetusa.com>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Organization: S.A.P.I.E.N.T. (Surly, Agressive, Politically Incorrect Enemies of Non Thinkers)

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

CC: comahal@cci-29palms.com

Subject: New Tesla Page

Sender: tesl-err@emachine.com

All,

Special note of gratitude to Mike Hammer for getting us on the air, and

for his both general and specific help with the Tesla Project!

This is to relieve Mike of the further necessity of keeping our stuff

under foot, and to direct readers to Dan's new Tesla site, in which I

still seem to be under foot! I'm workng on that, but don't expect much

too soon!

Dan's put up a site which is pinched, narrow of view, and he is opinionated!

Mine will be similar, no doubt! But with a wider range of interests.

But the site does show our Tesla Coil status, less of course, the

proposed circuit (and legend), which he may have lost!

---------------------------------------------

Gene Waldo -  gwaldo@1stnetusa.com  - "Igor in Residence"

---------------------------------------------------------------

What's good for the planet is more OZONE! See the machinery form up!

NEW PAGE!   http://members.1stnetusa.com/danmurphy/waldo.htm

X-Mailer: IPAD 2.0

Date: Tue, 19 May 1998 18:40:36 -0400

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Jim Ryan <jryan@exis.net>

Subject: Spark Gap

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Hi all, I'm rebuilding my spark gap for my 4.5" x 21" pri

12KV 30ma. coil. 9   4.5" L x 3/4" copper pipe, with shop

vac for cooling. I guess it's time to ask what the spark does

to the circuit. Is it just for tuning ?

Thanks

Jim

Date: Tue, 19 May 1998 17:45:52 -0600

From: Jonathan Bazemore <jpbazemo@ouray.cudenver.edu>

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Polyethylene.

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Thanks for mentioning that, Roy.  I just noticed, as I unrolled the film, that

there are numerous imperfections: ie., dents, bubbles, surface irregularities, and

perhaps other anomies (heaven forbid, holes).

Well, hopefully, the inconsistencies will cancel out in the finished product.

Skies overhead in Denver look grey and mean.  Like a storm is brewing.

I think it's time to open up a can of whoop-ass and show Mother Nature

who's really in charge.  Mr. Hull, would you do the honors, please?

Roy Minnich wrote:

> Hi,

>

> That's the right stuff.  Watch out for imperfections and dirt in the film.  Use only near perfect sections.  Remember poly film is only good for max 375 volts AC per mil (.001 inch) not the 1200 volts (DC) you see in many lists.

>

> Somebody bought out the Builder's Square near me.

>

> Roy Minnich

>

> ----------

> From:   Jonathan Bazemore[SMTP:jpbazemo@ouray.cudenver.edu]

> Reply To:       Tesla-2 Mailing List

> Sent:   Monday, May 18, 1998 9:52 PM

> To:     Tesla-2 Mailing List

> Subject:        Polyethylene.

>

>     I got some 4 mil polyethylene at Builder's Square today.  The size

> is 3 x 50 feet.  Is this the right type to make a capacitor, if I plan

> to submerge it in oil?

>

>   ------------------------------------------------------------------------

Date: Tue, 19 May 1998 23:56:45 -0600

From: Jonathan Bazemore <jpbazemo@ouray.cudenver.edu>

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Coil Operational.

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

I built the 1st .01 mfd cap and tested out the coil tonight.  I had to do

it outside, because it was so loud.  I talked to my girlfriend, too, and I think

that she will understand.

Tomorrow when she is at work, I intend to test the coil again.

Fortunately, I live on the second floor of an apartment complex with very friendly

people above us: the people across the hall were evicted, and our next door neighbor

is some kind of travelling saleman (he is never here, and when he is, he looks very tired).

So I tested the coil outside with the .01 mfd cap, and I got some sparks

from the top, maybe 1".  There was a purple ring corona discharge from the top of

the coil, but it still looks pretty weak.

I used between 0 and 1/4 inches on the gap.  I was afraid to go further

because I didn't want to stress the new cap.

What should I do next?

TTYL !

X-Mailer: IPAD 2.0

Date: Wed, 20 May 1998 17:37:47 -0500

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: Spark Gap

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

At 06:40 PM 5/19/98 -0400, you wrote:

>Hi all, I'm rebuilding my spark gap for my 4.5" x 21" pri

>12KV 30ma. coil. 9   4.5" L x 3/4" copper pipe, with shop

>vac for cooling.

Sounds great Jim.

>I guess it's time to ask what the spark gap does to the circuit.

>Is it just for tuning ?

Excellent question Jim.

The spark gap is the heart and soul of a Tesla coil.

Without the gap there would be no oscillations, no RF energy,

and no high voltage.

Rather than type in an answer to your question I

thought I would include some material on spark gaps

from a Tesla coil FAQ Ive been working on. This is an

early draft so dont blast me for spelling or grammar errors.

<begin include>

What is a spark gap?

------------------------------

The spark gap is the heart and soul of a Tesla coil. It is the single

most important component and yet one that gets little attention

from beginners. More coils have shown disappointing results due

to poor gaps than any other single cause.

Think of the spark gap as a high speed high power switch.

It appears as an open circuit while the main capacitor

is being charged by the power supply transformer.

Once the cap is fully charged the gap ionizes the air

between the electrodes and begins to conduct.

Once conducting it becomes a short circuit and allows

the cap to dump its charge into the primary coil. Once the cap

discharges the gap must quench (go out) and become non-conductive

once again so that the cap may begin charging once again.

There are several type of gaps and each has its own

advantages and disadtanges.

1. Static gaps.

Static gaps can take several forms. The most basic is just two electrodes

spaced a small distance apart. This can be the heads of two heavy bolts

or two brass knobs from the hardware store.

This is an easy gap to implement and will serve Ok for the beginner.

The spacing between the electrodes depends on the power supply voltage

from your transformer. 9kv neons do well with about .150 inch of gap

while 12kv neons can use up to .200 inch. 15kv neons can use up to .250

inch. Wider gaps than this should be avoided as they put a great deal of

stressn your capacitor and transformer.

The single static gap while easy to build is a poor performer.

All the heat is dissipated in one area and the gap doesnt want to quench.

The hot ions in between the gap electrodes tend to keep the gap lit

at lower voltages. Blowing air through the gap with a fan or blower

will improve the operation of this type of gap by removing these hot ions.

A much better approach is to break up the single large gap into a series

of smaller gaps in series. This is called a series static gap. This divides

the arc up into smaller parts that develop less heat and operate more

efficiently. This type of gap also benefits from a little forced air to

remove hot ions.

One such gap popularized by Richard Quick and often referred to as the

RQ cylinder gap consists of multiple electrodes made from lengths of

hard copper tubing. The electrodes are usually made from 3/4 or 1 inch

copper tubing cut from 2 to 4 inches long. Larger and longer elctrodes

provide more thermal mass and handle the heat better. These electrodes are

mounted inside a piece of PVC pipe and spaced about .025-.03 inch apart.

7 such electrodes providing 6 gaps between then are normally used for 9kv

neons and 9 electrodes providing 8 gaps are used for 12kv neons.

Up to 11 electrodes with 10 gaps can be used for 15kv neons.

A small muffin fan is mounted on the end of the PVC pipe holding the

electrodes to provide cooling airflow.

2. Air blast gaps

Blowing a high speed column of air through the teeth of a

static gap greatly improves the operation by stretching and cooling

the arc and removing hot ions from between the electrodes.

Playing the output of an air compressor between the electrode faces

can produce some good effects. This type of gap is loud and requires

a constant supply of high speed air to be effective.

3. Vacuum gaps

Instead of blowing high speed air through a static gap the vacuum

gap sucks air through the gap. Use a shop vac or purchase a used

or surplus cannister type vacuum cleaner motor for the purpose.

This type of gap is very effective. In fact the vacuum cleaner motor

can replace the muffin fan on a RQ style cylinder gap and increase the

performance of that type of gap.

4. Quench gap.

A special form of static gap is known as the quench gap. In this gap

the electrodes are wide flat plates with their faces parallel and

spaced very close. The electrodes are spaced from each other with

mica rings that enclose the gap area. The gap areas between electrodes

become sealed chambers and the oxygen in these chambers is used up

quickly when the gap is first put into operation.  This type of gap

works fairly well at lower voltages than other gaps and provide

a nearly continous wave (CW) output. These were used in spark radio

transmitters until spark transmitters became obsolete with the invention

of the triode vacuum tube in the 1910s. This gap design is difficult to

implement due to the sealed nature of the gaps and lack of suitable material

for the spacers. The mica rings normally used are difficult to find

and expensive

5. Rotary gaps

A rotary gap is a motor driven gap that has a solid disk with

electrodes mounted around its periphery. These electrodes come

into line with stationary electrodes to form the gap. As the

disk rotates each set of electrodes comes into play in sucession.

With a large number of spinning electrodes and a high rotational speed

the number of gap presentations can become quite high. And quenching

is quite good with a rotary as the electrodes are in firing position for

only a short time.

Rotary gaps work well with higher power systems but are unnecessary with

small to medium power systems running from neon sign transformers.

In fact neons work better with one of the multiple static gap systems and

can actually be damaged by a rotary gap.

What type of spark gap will I need?

-------------------------------------

The type of gap you should use depends a great deal on the kind

of transformers you are using and the power level you are operating at.

Low power operation of less than 500 watts can often do well with a single

static gap using a pair of bolts or brass knobs.

Even here though a series string of gaps like the RQ style cylinder

gap will improve efficiency.

Medium power systems up to 1500 watts using neon sign transformers

do very well with a RQ style cylinder gap. Above this power level

plenty of forced air should be used for cooling or you can go to

a vacuum gap. The vacuum gap works quite well up to 2500 watts.

Above this power level rotary gaps become necessary and are often

used in series with static gaps.

Rotary gaps should generally not be used with neon sign powered coils

as the forced break causes a great deal of strain that neons dont

handle well.

What is Quenching?

-----------------------

No discussion of spark gaps is complete without at least a rough

definition of "quenching". This term is commonly thrown around

when talking about spark gaps.

Quenching refers to the ability of an established arc to be

extinguished. It is much easier to start a gap firing than it is to

put it out once it has fired. In Tesla coils quenching the arc is

critical to good coil performance.

A cold, non-firing, spark gap is "clean". It contains no plasma

or hot ions. On applying voltage to the gap, a tension is esta-

blished between the electrode faces. Once the tension becomes high

enough to ionize the air between the gap the resistance drops rapidly.

This breakdown ionizes the gas between the electrodes and the arc

begins to ablate and ionize the metal electrodes themselves.

This mixture of ions forms a highly conductive plasma between

the gap electrodes. Without this highly conductive channel through

the gap, efficient tank circuit oscillation would not be possible.

But this plasma also shorts the gap out. A gap choked with hot ions

does not want to open and allow the capacitors to recharge for the

next pulse. The gap is  "dirty" with hot ionized gases and must be

cleaned so it can be extinguished.

To quench the hot gap the built up ions must be removed. This can be

done with a flow of air from a fan, blower or vacuum cleaner motor.

Once the excess ions are removed and the gap is cooled by the airflow

it can quench much easier.

Anything that reduces the amount of heat in the gap will aid quenching.

Series static gaps reduce the heat in each gap by breaking up the gap

in smaller segments. Quenching becomes easier.

<end include>

Well there you go Jim. More than you ever wanted to know

about spark gaps. That will teach you for asking a question

like that. :)

Comments, criticism, sarcasm and cash donations welcome.

TTYL!

Mike Hammer

mhammer@misslink.net

X-Mailer: IPAD 2.0

Date: Wed, 20 May 1998 20:49:30 -0400

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Jim Ryan <jryan@exis.net>

Subject: Re: Spark Gap

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Mike, thanks for the FAQ answer. It let me know that I was on the right

track for the coil I'm constructing. Can you do any tuning using different

numbers of gaps ?

Thanks

Jim

OBTW nothing to do with T-COILS try this site for a neat boat, I saw it

up close today. I also make model planes and boats.

http://www.amphistar.com/

X-Mailer: IPAD 2.0

Date: Wed, 20 May 1998 20:54:35 -0500

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: Spark Gap

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

At 08:49 PM 5/20/98 -0400, you wrote:

>Mike, thanks for the FAQ answer. It let me know that I was on the right

>track for the coil I'm constructing. Can you do any tuning using different

>numbers of gaps ?

>

>Thanks

>

>Jim

>

Jim,

It is advisable to do tuning with less than the

maximum amount of gap. A narrow gap keeps primary

energy level low.

Note that changing the spacing of the gap does

not affect the tuning to any great extent. The tuning

is dependant on the LC ratios of the tuned circuits.

The gap spacing will change the primary circuit energy

level by determining how high the cap will charge before

the gap breaks down.

TTYL!

Mike Hammer

mhammer@misslink.net

X-Mailer: IPAD 2.0

Date: Wed, 20 May 1998 21:31:01 -0500

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: Coil Operational.

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

At 11:56 PM 5/19/98 -0600, you wrote:

> I built the 1st .01 mfd cap and tested out the coil tonight.

>I had to do it outside, because it was so loud.  I talked to

>my girlfriend, too, and I think that she will understand.

> So I tested the coil outside with the .01 mfd cap, and I got some

>sparks from the top, maybe 1".

>There was a purple ring corona discharge from the top of the coil, but

>it still looks pretty weak.

>I used between 0 and 1/4 inches on the gap.  I was afraid to go further

>because I didn't want to stress the new cap.

>

>What should I do next?

>

>    TTYL !

Jon,

The corona at the top indictes you are producing some

high voltage RF but not very much of it as you are still

out of tune.

Keep that spark gap down to less than .200 inch. Dont want to

damage your new cap.

What you need to do is try tapping onto each turn of the primary

and checking for an increase in output. As you tap in more primary

turns your primary resonant frequency will get lower and bring

you closer to the frequency of your secondary coil.

If you have all the primary turns in the circuit and still

not getting any more output thats most likely an indication

that you dont have enough primary turns. In that case build

a new larger primary with more turns.

It sometimes takes a little trial and error to get that first

coil in tune but keep at it and it will get easier as you get

more experience.

TTYL!

Mike Hammer

mhammer@misslink.net

Date: Wed, 20 May 1998 21:44:18 -0600

From: Jonathan Bazemore <jpbazemo@ouray.cudenver.edu>

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Quick request

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Hi,

Two addresses were recently posted for mailing lists of interest;

the laser list and the pyrotechnics list.  I tried to subscribe to

both.  The message I sent to the pyrotechnics list bounced, and I haven't

heard anything from the laser list.  Question: is the laser list just low traffic?

And could someone double-check the original address for pyrotechnics to assure

that it is correct?  If it is, no need to repost.  Sorry about the off-topic,

I have questions that should in all appropriateness go to those lists,

if I can get on them,

thanks.

Date: Wed, 20 May 1998 21:50:55 -0600

From: Jonathan Bazemore <jpbazemo@ouray.cudenver.edu>

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: re: Help!  Primary and secondary shorting out.

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

I just tested out my coil again a minute ago, and there is all kinds

of power going in, but nothing going out.  Since it is dark, I saw a glow

inside the primary, between the primary and the secondary.  So I looked

down inside the coil (keeping a safe distance away) and I saw 3 to 4 sparks,

each about 1 inch long, arcing from the secondary to the primary, inside

the coil form.

What can I do?  Should I put some polyethylene insulation in there?

Thanks for your comments!

X-Mailer: IPAD 2.0

Date: Wed, 20 May 1998 23:26:25 -0500

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Mike Hammer <mhammer@misslink.net>

Subject: re: Help!  Primary and secondary shorting out.

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

At 09:50 PM 5/20/98 -0600, you wrote:

> I just tested out my coil again a minute ago, and there is all kinds

>of power going in, but nothing going out.  Since it is dark, I saw a glow

>inside the primary, between the primary and the secondary.  So I looked

>down inside the coil (keeping a safe distance away) and I saw 3 to 4 sparks,

>each about 1 inch long, arcing from the secondary to the primary, inside

>>the coil form.

>

> What can I do?  Should I put some polyethylene insulation in there?

>

> Thanks for your comments!

>

Jon,

Corona and arcing between the primary and secondary coils is

almost always due to overcoupling. Simply raise up the secondary

and inch or two away from the primary to reduce coupling.

Adding insulation in an overcoupled situation usually

never works. The arcs will pop their way through the insulation.

Reducing the coupling by increasing the distance between the

primary and secondary is the answer.

TTYL!

Mike Hammer

mhammer@misslink.net

Date: Thu, 21 May 1998 00:25:19 -0700

From: mike nicholas <705728@ican.net>

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Quick request

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Well I'm on the pyrotecnics newsgroup, Its well worth the time, lots of good

info. If you have more questions on news groups, just contact me direct

Jonathan Bazemore wrote:

>         Hi,

>

>     Two addresses were recently posted for mailing lists of interest;

> the laser list and the pyrotechnics list.  I tried to subscribe to

> both.  The message I sent to the pyrotechnics list bounced, and I haven't heard

> anything from the laser list.  Question: is the laser list just low traffic?  And

> could someone double-check the original address for pyrotechnics to assure

> that it is correct?  If it is, no need to repost.  Sorry about the off-topic,

> I have questions that should in all appropriateness go to those lists,

> if I can get on them, thanks.

From: "Kent Rankin" <kentrankin@theatreorgans.com>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Quick request

Date: Thu, 21 May 1998 10:16:03 -0400

X-Mailer: IPAD 2.0

X-MimeOLE: Produced By Microsoft MimeOLE V4.72.2106.4

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

I would also be interested in finding out how to join this group.

Thanks in advance,

Kent Rankin

-----Original Message-----

From: mike nicholas <705728@ican.net>

To: Tesla-2 Mailing List <tesla-2@emachine.com>

Date: Thursday, May 21, 1998 12:31 AM

Subject: Re: Quick request

>Well I'm on the pyrotecnics newsgroup, Its well worth the time, lots of good

>info. If you have more questions on news groups, just contact me direct

>

>Jonathan Bazemore wrote:

>

>>         Hi,

>>

>>     Two addresses were recently posted for mailing lists of interest;

>> the laser list and the pyrotechnics list.  I tried to subscribe to

>> both.  The message I sent to the pyrotechnics list bounced, and I haven't heard

>> anything from the laser list.  Question: is the laser list just low traffic?

>> And could someone double-check the original address for pyrotechnics to assure

>> that it is correct?  If it is, no need to repost.  Sorry about the off-topic,

>> I have questions that should in all appropriateness go to those lists,

>> if I can get on them, thanks.

>

>

>

X-Mailer: IPAD 2.0

Date: Thu, 21 May 1998 10:50:15 -0500

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: Hello and first questions

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

At 11:22 PM 5/19/98 +0300, you wrote:

>Hello to everybody on the list!

>

>I'm a new Tesla Coil builder and I'm building a 4"-secondary coil system. The

>tank circuit is fed with 8KV generated from microwave oven surplus parts.

>And, of course, I got a lot of questions... :)

>

>***************************************************************************

***

>Question n.1

>************

>Here is an ASCII drawing of my tentative power supply.

>It is made of 2 sets of transformer + diode +

>capacitor from two microwave ovens. The transformers give about 2KV, which is

>doubled with capacitor + diode to 4KV. Put two of these is series and you

>get 8KV. See the picture below:

>

>                         | |

>       --------|| ||-----| |----*---------- -

>             * || || *   | |    |

>               || ||          -----

>   220V        || ||          \   /

>               || ||           \ /

>               || ||          -----

>               || ||            |

>       --------|| ||------------*--- GND

>                                |

>                                |

>       --------|| ||------------*

>             * || || *          |

>               || ||          -----

>               || ||          \   /

>    220V       || ||           \ /

>               || ||          -----

>               || ||     | |    |

>       --------|| ||-----| |----*----------- +

>                         | |

>

>If you are carefull with transformer phasing, you get 8KV pulses at

>25 Hz (Europe = 220V @ 50Hz !). Connect the phases wrong and you'll get

>4KV pulses at 50 Hz. Between + and - terminals I can get a 0.37" continuous

>spark between a simple 2-screw gap.

>

>Any problem to feed a TC with this (forget about filtering for now)?

Yes I see several problems. First of all MW oven transformers

just arent suitable for spark gap driven coils. The voltage

they supply is too low and the current is too high.

You can partially solve the voltage problem with a multiplier

circuit like you have here or simply running several transformers

in series. But this wont help with the excessive current problem.

Oven transformers have magnetic shunts but they are not as

efficient in limiting current as in a neon sign transformer.

The current is typically around 300 ma and make go to 500

or even 600ma when short circuited by a spark gap.

This high current level will play hell with any kind of spark

gap. The excess current will just ablate away the electrodes and

create lots of heat. You will end up with a short lived gap

that just wont quench. Very poor performance.

Secondly your diodes will not take kindly to being short circuited

by an energized spark gap. I would wager their lifetime would

be measured in microseconds.

Much much easier to use a couple neon sign transformers.

They are nearly perfect for coil usage.

>***************************************************************************

***

>Question n.2

>************

>I got some surplus Maxwell capacitors model 30561 0.1uF 25KV with following

>specs:

>

>Capacitance: 0.1uF 25KV

>Rated peak current: 5 KA

>Max peak current: 25 KA

>Max ripple freq.: 10 KHz

>Max RMS current: 25 A RMS

>Max Dissipation factor: 0.3% @ 120 Hz

>Inductance: 80 nH

>Dielectric: polypropylene and paper, oil is I.P.B.P.

>

>They are enclosed in metal cans and have got ceramic terminals.

>I intend to use two of them is SERIES (0.05 uF 50 KV).

>Will they work with my Tesla Coil (frequency about 150 KHz) or will they

>warm-up in 10 seconds and blow?

>

Hard to say. Being polypropylene makes them attractive for coil

use. I would say give them a try and monitor the case temperature

closely. If you detect any temperature rise then better off

not to use them.

Also .05 mfd is quite large. This will require a rather large coil.

This much capacitance will result in a quite low primary resonant

frequency when used with a suitably large primary coil.

Make sure your secondary is designed to resonate at this low frequency.

This level of capacitance also requires a rather large power

transformer. This is usually the range reserved for use with a pole pig.

>***************************************************************************

***

>Question n.3

>************

>One question bothers me: this might be a stupid one...

>Because I have read often that a discharge from the secondary stroke the

>primary and damaged something, isn't it possible to shield the primary

>with a sheet of acryl (or other plastic)?

>

>I tought to cover the primary with a sheet of 5 mm acryl plastic and, on

>the top of it, place the strike rail.

>

>Could it possibly work?

>

>               -----------

>              |           |         toroid

>               -----------

>                |       |

>                |       |

>                |       |

>                |       |

>          *     |       |     *     strike rail

>         ------------------------   acryl shield

>          * * * |       | * * *     primary (3 turns shown)

>       ---------------------------- coil base

Yes you can do this but it probably wont help a lot. The secondary

voltage can be several hundred KV. 5mm of plastic wont seriously

deter this kind of voltage.

TTYL!

Mike Hammer

mhammer@misslink.net

From: Roy Minnich <rminnich@rolling-meadows.lib.il.us>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: RE: Spark Gap

Date: Thu, 21 May 1998 13:00:22 -0500

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

Hi Mike,

Very informative and well done!  Thank you for your excellent FAQ on gaps.

I still think a "vacuum gap" is operating in an evacuated chamber every time I hear it mentioned.  I know that it describes a multigap cooled/quenched by fast moving air provided by a vacuum cleaner fan/motor. Perhaps we should call it "fan quenched gap" to separate it from a "fan cooled gap" cooled with muffin fans on the lower powered coils.  Or is it actually quenched?

Thanks,

Roy Minnich

----------

From: 	Mike Hammer[SMTP:mhammer@misslink.net]

Reply To: 	Tesla-2 Mailing List

Sent: 	Wednesday, May 20, 1998 5:37 PM

To: 	Tesla-2 Mailing List

Subject: 	Re: Spark Gap

At 06:40 PM 5/19/98 -0400, you wrote:

>Hi all, I'm rebuilding my spark gap for my 4.5" x 21" pri

>12KV 30ma. coil. 9   4.5" L x 3/4" copper pipe, with shop

>vac for cooling.

Sounds great Jim.

>I guess it's time to ask what the spark gap does to the circuit.

>Is it just for tuning ?

Excellent question Jim.

The spark gap is the heart and soul of a Tesla coil.

Without the gap there would be no oscillations, no RF energy,

and no high voltage.

Rather than type in an answer to your question I

thought I would include some material on spark gaps

from a Tesla coil FAQ Ive been working on. This is an

early draft so dont blast me for spelling or grammar errors.

<begin include>

What is a spark gap?

------------------------------

The spark gap is the heart and soul of a Tesla coil. It is the single

most important component and yet one that gets little attention

from beginners. More coils have shown disappointing results due

to poor gaps than any other single cause.

Think of the spark gap as a high speed high power switch.

It appears as an open circuit while the main capacitor

is being charged by the power supply transformer.

Once the cap is fully charged the gap ionizes the air

between the electrodes and begins to conduct.

Once conducting it becomes a short circuit and allows

the cap to dump its charge into the primary coil. Once the cap

discharges the gap must quench (go out) and become non-conductive

once again so that the cap may begin charging once again.

There are several type of gaps and each has its own

advantages and disadtanges.

1. Static gaps.

<SNIP>

From: Roy Minnich <rminnich@rolling-meadows.lib.il.us>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Primary question

Date: Thu, 21 May 1998 13:07:11 -0500

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

Hello,

I have to run my small coil with the maximum number of turns tapped on the primary

for the best performance.  If I move the tap even one turn lower there is a noticeable decrease in output.

Should I add more turns to the primary??  It's a solenoid type primary.

Thanks,

Roy Minnich

X-Mailer: IPAD 2.0

Date: Thu, 21 May 1998 13:53:45 -0500

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Mike Hammer <mhammer@misslink.net>

Subject: RE: Spark Gap

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

At 01:00 PM 5/21/98 -0500, you wrote:

>Hi Mike,

>

>Very informative and well done!  Thank you for your excellent FAQ on gaps.

Thanks. Always good to know that my contributions are useful.

That FAQ on spark gaps is part of a larger body of work.

Its a FAQ on a wide range of coiling topics. If I hadnt been

so lazy I would of had it done by now.

Some of the conceptualization and format of the FAQ was supplied

by list member Dave Bronakowski with input from Corey Edmunds.

Ive also drawn heavily on other sources such as Bert Hickman,

Fr. Tom McGahee, and Richard Quick. I owe a lot to these people

and many others.

>

>I still think a "vacuum gap" is operating in an evacuated chamber every

time I >hear it mentioned.  I know that it describes a multigap

cooled/quenched by >fast moving air provided by a vacuum cleaner fan/motor.

Perhaps we should call >it "fan quenched gap" to separate it from a "fan

cooled gap" cooled with >muffin fans on the lower powered coils.  Or is it

actually quenched?

>

>Thanks,

>Roy Minnich

I agree that the term "vacuum gap" is really a misnomer.

Im afraid I just copied what others were calling that type of gap.

Airflow from any source aids quenching but in the RQ type

of cylinder gap the air flow from the muffin fan supplies

mostly cooling air. A strong fan or blower will improve

the quenching of this type of gap.

Note that in my vacuum gap there is too much airflow

from the cannister type vacuum cleaner motor I use.

At full speed the airflow actually inhibits gap operation.

I run the motor on a variac to vary the air speed and

I can actually find a point where the secondary output

peaks. This peak coincides with optimal quenching as

verified by oserving the secondary waveforms on an

oscilloscope.

TTYL!

Mike Hammer

mhammer@misslink.net

X-Mailer: IPAD 2.0

Date: Thu, 21 May 1998 13:59:18 -0500

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: Primary question

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

At 01:07 PM 5/21/98 -0500, you wrote:

>Hello,

>

>I have to run my small coil with the maximum number of turns tapped on the

primary for the best performance.  If I move the tap even one turn lower

there is a noticeable decrease in output.

>

>Should I add more turns to the primary??  It's a solenoid type primary.

>

>Thanks,

>Roy Minnich

Roy,

You undoubtedly need more primary turns. Its likely that you are not

quite reaching resonance with even all your primary turns tapped in.

I would add at least two more turns. Preferably more.

This is why I tell everyone to make sure to have plenty of turns

on their primary. I ended up adding more turns to my primary twice.

Better to start with too many than too few.

TTYL!

Mike Hammer

mhammer@misslink.net

From: Roy Minnich <rminnich@rolling-meadows.lib.il.us>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Another Primary question

Date: Thu, 21 May 1998 14:34:17 -0500

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

Hi Mike,

Thanks for the replies on the gaps and this one.

The plan I built my small coil from required a primary with 25! turns of #12.

I thought this would be plenty but I guess not.  These small coils don't seem to

model like the larger ones at all.

How might incorrect sizing of the cap affect the sizing of the primary??  Too big,

too small of a cap would affect the # of primary turns how???

Thanks,

Roy Minnich

----------

From: 	Mike Hammer[SMTP:mhammer@misslink.net]

Reply To: 	Tesla-2 Mailing List

Sent: 	Thursday, May 21, 1998 1:59 PM

To: 	Tesla-2 Mailing List

Subject: 	Re: Primary question

At 01:07 PM 5/21/98 -0500, you wrote:

>Hello,

>

>I have to run my small coil with the maximum number of turns tapped on the

primary for the best performance.  If I move the tap even one turn lower

there is a noticeable decrease in output.

>

>Should I add more turns to the primary??  It's a solenoid type primary.

>

>Thanks,

>Roy Minnich

Roy,

You undoubtedly need more primary turns. Its likely that you are not

quite reaching resonance with even all your primary turns tapped in.

I would add at least two more turns. Preferably more.

This is why I tell everyone to make sure to have plenty of turns

on their primary. I ended up adding more turns to my primary twice.

Better to start with too many than too few.

TTYL!

Mike Hammer

mhammer@misslink.net

X-Mailer: IPAD 2.0

Date: Thu, 21 May 1998 15:32:50 -0500

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: Another Primary question

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

At 02:34 PM 5/21/98 -0500, you wrote:

>Hi Mike,

>

>Thanks for the replies on the gaps and this one.

>

>The plan I built my small coil from required a primary with 25! turns of

#12.  I thought this would be plenty but I guess not.  These small coils

don't seem to model like the larger ones at all.

>

>How might incorrect sizing of the cap affect the sizing of the primary??

Too big, too small of a cap would affect the # of primary turns how???

>

>Thanks,

>

>Roy Minnich

Too small a main cap will require more primary inductance

to resonate at the secondary coil resonant frequency. So if

you are looking at 20+ primary turns you are using a way too small

capacitor. However if your cap is as large as it can be because

of a small neon or other transformer then your only choice is

more primary turns.

Too large a cap will mean only a few turns on your primary.

Too few turns adversely affects primary/secondary coupling

and spark gap operation. Plus a large cap requires a suitably

large transformer to charge it.

I would recommend increasing your cap size as much as your

transformer will allow and without forcing your primary

coil to less than 10 turns.

Also note that a larger diameter primary coil will provide

higher inductance without increasing the number of turns.

If you are having any overcoupling problems like primary/

secondary arcing then a larger diameter primary would help

in both regards by decreasing the coupling and providing

a higher inductance.

TTYL!

Mike Hammer

mhammer@misslink.net

From: Dsurfr <Dsurfr@aol.com>

Date: Thu, 21 May 1998 17:40:34 EDT

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Secondary shorting?

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

My secondary is 4 1/2" x 22" 24 gauge wire with 4 coats of polyurethane. My

primary is 18 turns #10 copper wire 1/4" spaced 9"D(bottom) 12"D (top) I'm

using a 15k 30ma NST & SW caps. (about .0048uf) I just added another bottle to

the cap to bring it up to this level. When I run the coil now I'm starting to

get sparks breaking out on the secondary in various places from one section to

another. None of the turns are overlapped The coil is grounded to a water pipe

until I get my dedicated ground in this weekend. I was getting about 10" to

12" sparks to a grounded wire before adding the last bottle & had no breakout

problems. I haven't tried running it with a grounded object to strike to yet

because I'm afraid I'll damage the secondary running it with these "shorts"

occurring. Can anybody offer some advice or should I just rewind the primary

as a saucer or pancake?

Thanks, Jim

Date: Thu, 21 May 1998 16:19:58 -0600

From: Jonathan Bazemore <jpbazemo@ouray.cudenver.edu>

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: re: Dual half lambda coil system.

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

What is a dual half lambda coil system?  Lambda means wavelength,

right?  So are these coils that are 180 degrees out of phase, and so

the sparks go from the top of one to the top of the other?

Comments appreciated, thanks.

X-Mailer: IPAD 2.0

Date: Thu, 21 May 1998 17:24:39 -0500

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: Secondary shorting?

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

At 05:40 PM 5/21/98 EDT, you wrote:

>My secondary is 4 1/2" x 22" 24 gauge wire with 4 coats of polyurethane. My

>primary is 18 turns #10 copper wire 1/4" spaced 9"D(bottom) 12"D (top) I'm

>using a 15k 30ma NST & SW caps. (about .0048uf) I just added another bottle to

>the cap to bring it up to this level. When I run the coil now I'm starting to

>get sparks breaking out on the secondary in various places from one section to

>another. None of the turns are overlapped The coil is grounded to a water pipe

>until I get my dedicated ground in this weekend. I was getting about 10" to

>12" sparks to a grounded wire before adding the last bottle & had no breakout

>problems. I haven't tried running it with a grounded object to strike to yet

>because I'm afraid I'll damage the secondary running it with these "shorts"

>occurring. Can anybody offer some advice or should I just rewind the primary

>as a saucer or pancake?

> Thanks, Jim

Sounds like a classic case of overcoupling to me Jim.

If you run your coil in a darkened room you will probably

also see corona between the primary and secondary.

The cure is simple. Just raise up the secondary away

from the primary to reduce the coupling. Raise it just

enough to prevent arcing.

Increasing your cap size probably brought you closer

to resonance and definitely increased your primary

energy level. At lower power levels an overcoupling

situation can go unnoticed.

Your tapered primary is very similar to the primary

on my coil. Let me tell you my primary like that overcouples

greatly. I needed to raise the secondary up 3 inches to

reduce the coupling to a manageable level.

TTYL!

Mike Hammer

mhammer@misslink.net

From: Dsurfr <Dsurfr@aol.com>

Date: Thu, 21 May 1998 21:19:39 EDT

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Secondary shorting

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Mike: As usual, thanks for the rapid & accurate advice. I raised the primary

up 1", decreased the number of turns I was tapping it at, decreased the spark

gap a fraction & all seems well. I guess its time to get to work on a real

toroid (will this help as well?) Right now, to get it in tune, I'm using a 1"x

5"plastic "donut" covered w/ aluminum foil. I'm planning on a 4"x 14" dryer

duct toroid. Do I need to cover the duct w/ aluminum tape or will the aluminum

foil on the duct be sufficient? Is the size "over kill" for a coil of this

size?

Thanks again,  Jim

X-Mailer: IPAD 2.0

Date: Thu, 21 May 1998 21:36:46 -0400

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Jim Ryan <jryan@exis.net>

Subject: Re: Help me understand

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Hi All Again, I hope this is another good not dumb question.

I can only find # 10-12 crimp connectors at HQ to connect all

of the components together with # 10 stranded wire 600V.

I think Mike H. said #8 solid wire is the suggested/best.

How much do you think I will lower the performance of the coil

with #10 stranded wire?

Jim

X-Mailer: IPAD 2.0

Date: Fri, 22 May 1998 08:27:19 +0300

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Marco Denicolai <marco@vistacom.fi>

Subject: Re: Spark Gap

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

At 17:37 20/05/98 -0500, you wrote:

>Static gaps can take several forms...

>The spacing between the electrodes depends on the power supply voltage

>from your transformer. 9kv neons do well with about .150 inch of gap

>while 12kv neons can use up to .200 inch. 15kv neons can use up to .250

>inch. Wider gaps than this should be avoided as they put a great deal of

>stressn your capacitor and transformer.

As a beginner, I have to ask this. When the main spark gap is supposed to trigger ?

1 - When the tank capacitor is changed at full voltage (1.4 * transformer VAC)

OR

2 - When the tank capacitor is changed a suitable voltage LOWER than that?

If the correct answer is n.2, so what is the benefit of having a 14 kV

transformer if the capacitor is discharged always at e.g. 8 kV?

With my (supposed) 8kV tank I can get 0.37 inch sparks so that using a

0.15 inch gap would reduce my used voltage to about a 40% of the available...

More, isn't that shorter gap much more difficult to quench than a wider gap?

________________________________________________________________________

Marco Denicolai                   Vista Communication Instruments, Inc.

Hardware Development Manager      www.vistacom.fi

marco@vistacom.fi                 Kaisaniemenkatu 13 A

fax:    +358-9-622-5610           SF-00100 HELSINKI

phone:  +358-9-622-623-15         Finland

Remember, Murphy was an optimist! I am not...

________________________________________________________________________

X-Mailer: IPAD 2.0

Date: Fri, 22 May 1998 08:43:55 +0300

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Marco Denicolai <marco@vistacom.fi>

Subject: Re: Hello and first questions

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

At 10:50 21/05/98 -0500, you wrote:

>Oven transformers have magnetic shunts but they are not as

>efficient in limiting current as in a neon sign transformer.

>The current is typically around 300 ma and make go to 500

>or even 600ma when short circuited by a spark gap.

>

>This high current level will play hell with any kind of spark

>gap. The excess current will just ablate away the electrodes and

>create lots of heat. You will end up with a short lived gap

>that just wont quench. Very poor performance.

>

>Secondly your diodes will not take kindly to being short circuited

>by an energized spark gap. I would wager their lifetime would

>be measured in microseconds.

I have been running the supply all together maybe 10 minutes with continuous spark

in a 0.37 inch spark gap and the diodes are still there. Maybe you meant that when the

TC is fully operation the primary resonance kicks-back some extra voltage?

>

>Much much easier to use a couple neon sign transformers.

>They are nearly perfect for coil usage.

Unfortunately (for me) I found this list too late and my 8kV tank supply is

already assembled :(.

So I decided I'll give it a try: if something breaks down I will surely go

for a neon.

>>***************************************************************************

>***

>>Question n.2

>>************

>>I got some surplus Maxwell capacitors model 30561 0.1uF 25KV with following

>>specs:

>>

>>Capacitance: 0.1uF 25KV

>

>Also .05 mfd is quite large. This will require a rather large coil.

>This much capacitance will result in a quite low primary resonant

>frequency when used with a suitably large primary coil.

>Make sure your secondary is designed to resonate at this low frequency.

I made all calculation with Excel and that 0.05 uF should be Ok.

>

>This level of capacitance also requires a rather large power

>transformer. This is usually the range reserved for use with a pole pig.

Well, that MW oven tank supply DO have current to give, isn't it :)

>>***************************************************************************

>***

>>Question n.3

>>************

>>One question bothers me: this might be a stupid one...

>>Because I have read often that a discharge from the secondary stroke the

>>primary and damaged something, isn't it possible to shield the primary

>>with a sheet of acryl (or other plastic)?

>Yes you can do this but it probably wont help a lot. The secondary

>voltage can be several hundred KV. 5mm of plastic wont seriously

>deter this kind of voltage.

>

Yeah, this was definitively a stupid idea of my: I'll use just the strike rail.

________________________________________________________________________

Marco Denicolai                   Vista Communication Instruments, Inc.

Hardware Development Manager      www.vistacom.fi

marco@vistacom.fi                 Kaisaniemenkatu 13 A

fax:    +358-9-622-5610           SF-00100 HELSINKI

phone:  +358-9-622-623-15         Finland

Remember, Murphy was an optimist! I am not...

________________________________________________________________________

X-Mailer: IPAD 2.0

Date: Fri, 22 May 1998 11:26:45 +0300

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Marco Denicolai <marco@vistacom.fi>

Subject: 8kV MW oven supply and spark gaps

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Thinking more about my tank supply, I figured out I was wrong with the

voltage waveshape of its output.

| |

--------|| ||-----| |----*---------- -

* || || *   | |    |

|| ||          -----

220V        || ||          \   /

|| ||           \ /

|| ||          -----

|| ||            |

--------|| ||------------*--- GND

|

|

--------|| ||------------*

* || || *          |

|| ||          -----

|| ||          \   /

220V       || ||           \ /

|| ||          -----

|| ||     | |    |

--------|| ||-----| |----*----------- +

| |

If the transformers are in phase, the output (between + and -) is a 8kV DC

oscillating +/- 2kV in respect to ground. But because the + and -

oscillations are in phase, the net potential different between + and - is

always 8kV (i.e. it's a DC output).

V+ |----|    |----|    |----|

|    |----|    |----|    |----

GND ------------------------------     V+ - V- = 8kV DC

|----|    |----|    |----|

V- |    |----|    |----|    |----

If the transformers are NOT in phase, the output is a kind of pulsed DC,

changing from 4kV to 8kV at a frequency of 50 Hz (60 in US).

V+ |----|    |----|    |----|

|    |----|    |----|    |----

GND ------------------------------     V+ - V- = 4kV to 8kV DC pulsed

|    |----|    |----|    |----

V- |----|    |----|    |----|

Anybody has got different opinion about this?

Now, using a static spark gap will probably not work in either cases because

the microwave oven transformers are capable to pump even 500 mA. Or I would

need a jet turbine to quench it.

But what about a rotary spark gap? That should quench in any case and, because

I got almost a DC tank supply, I shouldn't have to bother about being in

phase with the main power supply (220V or 110V).

________________________________________________________________________

Marco Denicolai                   Vista Communication Instruments, Inc.

Hardware Development Manager      www.vistacom.fi

marco@vistacom.fi                 Kaisaniemenkatu 13 A

fax:    +358-9-622-5610           SF-00100 HELSINKI

phone:  +358-9-622-623-15         Finland

Remember, Murphy was an optimist! I am not...

________________________________________________________________________

Date: Fri, 22 May 1998 06:51:53 -0500

From: Bert Hickman <bert.hickman@aquila.com>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Organization: Stoneridge Engineering

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Spark Gap

Sender: tesl-err@emachine.com

Marco,

The voltage required to break down a gap is a strong function of the

geometry of the gap electrodes. Gaps made from multiple round or

cylindrical electrodes will exhibit a much higher breakdown voltage than

gaps made from small radius or pointed electrodes. Mike was referring to

a typical series static gap made up of a number of parallel cylindrical

copper pipes, each having a relatively large radius of curvature. Using

a large number of gaps, combined with the high thermal conductivity of

the copper (will possible assistance from a cooling air flow) provides

stable breakdown voltage and excellent quenching for small to medium

coils.

Now to your question:

Ideally, you want to match the gap spacing/breakdown voltage of your

main gap to the voltage handling capability of your power source and

your tank cap, and to the current sourcing capability of your HV power

source. A NST source (and an air-insulated microwave oven transformer -

MWOT) are quite sensitive to being overvolted - hence Mike's recommended

smaller gap spacing. One flashover, and the transformer is toast...

However, there's another reason to keep the gap spacing down. Because of

the built-in current limiting capability, a NST (and a MWOT to a lesser

extent) has significant difficulty in fully charging a tank capacitor.

Although a NST may deliver 30 MA when short circuited, it will deliver

only a small fraction of this current at maximum output voltage. As a

result, the tank cap will never reach peak voltage unless other more

stressful techniques (such as 50/60 Hz resonant charging) are used to

counteract the transformer's internal current limiting function.

Practically speaking, most coilers find that they get best performance

from their system when the gap fires _more_ than once every half-cycle,

with 2-3X being the norm. Because you can get more "bangs" per half

cycle, you transfer more average power to the secondary, and you'll get

more effective "incineration of the air" (longer streamers!) as a result.

Hope this helps!

-- Bert --

Marco Denicolai wrote:

>

> At 17:37 20/05/98 -0500, you wrote:

> >Static gaps can take several forms...

> >The spacing between the electrodes depends on the power supply voltage

> >from your transformer. 9kv neons do well with about .150 inch of gap

> >while 12kv neons can use up to .200 inch. 15kv neons can use up to .250

> >inch. Wider gaps than this should be avoided as they put a great deal of

> >stressn your capacitor and transformer.

>

> As a beginner, I have to ask this. When the main spark gap is supposed to

> trigger ?

>

> 1 - When the tank capacitor is changed at full voltage (1.4 * transformer VAC)

>       OR

> 2 - When the tank capacitor is changed a suitable voltage LOWER than that?

>

> If the correct answer is n.2, so what is the benefit of having a 14 kV

> transformer if the capacitor is discharged always at e.g. 8 kV?

>

> With my (supposed) 8kV tank I can get 0.37 inch sparks so that using a

> 0.15 inch gap would reduce my used voltage to about a 40% of the available...

>

> More, isn't that shorter gap much more difficult to quench than a wider gap?

>

> ________________________________________________________________________

>

>  Marco Denicolai                   Vista Communication Instruments, Inc.

>  Hardware Development Manager      www.vistacom.fi

>

>  marco@vistacom.fi                 Kaisaniemenkatu 13 A

>  fax:    +358-9-622-5610           SF-00100 HELSINKI

>  phone:  +358-9-622-623-15         Finland

>

>    Remember, Murphy was an optimist! I am not...

>

Date: Fri, 22 May 1998 07:25:07 -0700

From: Gene Waldo <gwaldo@1stnetusa.com>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Organization: S.A.P.I.E.N.T. (Surly, Agressive, Politically Incorrect Enemies of Non Thinkers)

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Optimum tank ckt?

Sender: tesl-err@emachine.com

My dumb Q of the day relates to the posible ckt layouts. I seem to

recall discussion of this, but can't find it now!

I know that ckt 1 operates, because that was my 1st coil, about 45 years

ago. But who knew?

I assume that all(?) of the following will function, but surely cannot

be considered 'equivalent' ckts.(?) Safety Gap, RFGnd not shown.

One must be 'best', or does this change with other factors?

|----------------------|

|                      |

N                      P         ckt 1

P                      R

S                      I

|                      |

|----------------------| (No RFGnd)

|--RFC1--|-----C1------|

|        |             |

N        G             P        ckt 2

P        1             R

S        |             I

|        |             |

|--RFC2--|-------------|

|--RFC1--|------|------|

|        |      |      |

N        G      C      P        ckt 3

P        1      1      R

S        |      |      I

|        |      |      |

|--RFC2--|------|------|

|--RFC1---C1--G1-------|

|                      |

N                      P        ckt 4

P                      R

S                      I

|                      |

|--RFC2----------------|

|--RFC1----|---G1------|

|          |           |

N          C           P       ckt 5

P          1           R

S          |           I

|          |           |

|--RFC2----|-----------|

Have I missed one?

----------------------------------------------

Gene Waldo -  gwaldo@1stnetusa.com  - "Igor in Residence"

---------------------------------------------------------------

What's good for the planet is more OZONE! See the machinery form up!

NEW PAGE!   http://members.1stnetusa.com/danmurphy/waldo.htm

X-Originating-IP: [199.217.140.22]

From: "Bill the arcstarter" <arcstarter@hotmail.com>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Optimum tank ckt?

Date: Fri, 22 May 1998 08:00:36 PDT

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

Coilers,

Here are my two cents (not corrected for inflation)...

>I assume that all(?) of the following will function, but surely cannot

>be considered 'equivalent' ckts.(?) Safety Gap, RFGnd not shown.

> One must be 'best', or does this change with other factors?

>

>|----------------------|

>|                      |

>N                      P         ckt 1

>P                      R

>S                      I

>|                      |

>|----------------------| (No RFGnd)

The above has no primary resonance.  The primary will be excited at 60

Hz at only 30 ma (or whatever the rating of your neon is...)  Any HV

gain will be due to turns-ratio transformer action.

>|--RFC1--|-----C1------|

>|        |             |

>N        G             P        ckt 2

>P        1             R

>S        |             I

>|        |             |

>|--RFC2--|-------------|

The above is the traditional layout.

>|--RFC1--|------|------|

>|        |      |      |

>N        G      C      P        ckt 3

>P        1      1      R

>S        |      |      I

>|        |      |      |

>|--RFC2--|------|------|

The above won't work - all the energy in the cap is dissipated across

the gap w/o going through the primary.  No resonant action.  This will

work more poorly than ckt #1.

>|--RFC1---C1--G1-------|

>|                      |

>N                      P        ckt 4

>P                      R

>S                      I

>|                      |

>|--RFC2----------------|

Do you really want to dump all the energy in your cap through the tiny

magnet wire which is in the secondary of the neon??  This ckt is

terrible! I bet it kills neons immediately.

>|--RFC1----|---G1------|

>|          |           |

>N          C           P       ckt 5

>P          1           R

>S          |           I

>|          |           |

>|--RFC2----|-----------|

This is what we run (contrary to popular suggestion - we keep the cap

across the xfmr instead of the gap).

Conclusion: Only ckts 2,5 are worth the effort for tesla coiling.

-Bill the arcstarter

Starting arcs in Cinci, OH

http://www.geocities.com/CapeCanaveral/Hangar/6160

X-Mailer: IPAD 2.0

Date: Fri, 22 May 1998 11:43:16 -0500

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Mike Hammer <mhammer@misslink.net>

Subject: re: Dual half lambda coil system.

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

At 04:19 PM 5/21/98 -0600, you wrote:

>

>    What is a dual half lambda coil system?  Lambda means wavelength,

>right?  So are these coils that are 180 degrees out of phase, and so

>the sparks go from the top of one to the top of the other?

>

>    Comments appreciated, thanks.

>

Jon,

What you are referring to is known as a 1/2 wave coil or

bipolar coil. It is just basicly two normal 1/4 wave secondaries

joined at their bases and fed by the same primary circuit.

Ive only seen two bipolar coils in operation and both of

them were poor performers. For that reason Ive never

investigated them myself.

TTYL!

Mike Hammer

mhammer@misslink.net

X-Mailer: IPAD 2.0

Date: Fri, 22 May 1998 11:58:38 -0500

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: Secondary shorting

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

At 09:19 PM 5/21/98 EDT, you wrote:

>Mike: As usual, thanks for the rapid & accurate advice. I raised the primary

>up 1", decreased the number of turns I was tapping it at, decreased the spark

>gap a fraction & all seems well.

Good. Glad I could help.

>I guess its time to get to work on a real toroid (will this help as well?)

Yes a larger toroid will provide several benefits.

First due to its capacitance it will lower the secondary resonant

frequency. This will require you to tap in more primary turns

so you can tune the primary to this new lower frequency.

The lower frequency helps by reducing losses due to skin effect

and corona. Te additional primary inductance helps with gap

quenching by increasing the impedance of the primary circuit.

The larger toroid will also increase streamer current by acting

like all capacitors and storing charge.

These all add up to better efficiency and longer streamers.

>Right now, to get it in tune, I'm using a 1"x 5"plastic "donut" covered

>w/ aluminum foil. I'm planning on a 4"x 14" dryer duct toroid. Do I need

to >cover the duct w/ aluminum tape or will the aluminum foil on the duct

be >sufficient?

Dont cover the ducting with foil. The ribs in the ducting dont

affect operation. The duct works better by itself. You will need to

cover the joint where the ends of the duct meet. I use some aluminum tape.

Smooth is out as much as possible.

>Is the size "over kill" for a coil of this size?

No I dont think so. I used a 16 X 4 duct toroid with good success.

Remember you will have to tap in a couple more primary turns to bring

the coil back to resonance. It may help to lay a nail or something

like that on the toroid to promote breakout. This will help with

the tuning. For longest sparks use no pointy dischargers.

TTYL!

Mike Hammer

mhammer@misslink.net

X-Mailer: IPAD 2.0

Date: Fri, 22 May 1998 12:02:59 -0500

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: Help me understand

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

At 09:36 PM 5/21/98 -0400, you wrote:

>Hi All Again, I hope this is another good not dumb question.

>

>I can only find # 10-12 crimp connectors at HQ to connect all

>of the components together with # 10 stranded wire 600V.

>I think Mike H. said #8 solid wire is the suggested/best.

>How much do you think I will lower the performance of the coil

>with #10 stranded wire?

>

>Jim

#10 will probably be just fine Jim at your power level.

If you were running 1500 watts then I would say it would

be a problem.

I dont approve of crimp connectors in the primary circuit.

You can use the connectors but solder them instead of crimping.

I only use well soldered or bolted connections.

TTYL!

Mike Hammer

mhammer@misslink.net

X-Originating-IP: [199.217.140.22]

From: "Bill the arcstarter" <arcstarter@hotmail.com>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: re: Dual half lambda coil system.

Date: Fri, 22 May 1998 10:25:40 PDT

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

It was written:

>What you are referring to is known as a 1/2 wave coil or

>bipolar coil. It is just basicly two normal 1/4 wave secondaries

>joined at their bases and fed by the same primary circuit.

>

>Ive only seen two bipolar coils in operation and both of

>them were poor performers. For that reason Ive never

>investigated them myself.

Seems like the only way to adjust coupling would be to re-tap the

innermost primary to move the active turns away from the secondary.

Is the secondary one continuous winding with the primary positioned near

the midpoint?

-Bill

Date: Fri, 22 May 1998 12:31:20 -0600

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Daniel Lockhart <molly@cmn.net>

Subject: Super First Success, then... Poof!?

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

Hey all,

I finally got my spark gap finished and hooked everything up on my first

coil last night. It didn't take long to tune, and soon I was getting

16-inch sparks to ground and multiple (no less than 50) 12-inch streamers

continually coming off the toroid into the air. I was pretty pleased with

this as I was using saltwater caps and this is (or was...) really kind of a

small coil (9kV, 30 ma transformer, 3.5 inch secondary).

Then something happened. Right before I cut the power, it stopped. I didn't

hear anything -- no pop or crackle or explosion. When I gave it power

again, the transformer just hummed and the safety spark gap fired on one

side. Anyway, I took the transformer out and found that one side didn't

spark to ground. Bummer.

What happened? What did I do wrong? I have a good safety gap, and two huge

toroid chokes. My capacitance is around what I need (according to my

calculations -- I still haven't got around to buying a cap meter). How can

I prevent this in the future? My safety gap was firing a lot -- at least

once per second. I assumed this was because it was set a little close.

I was irked, so I stuck a 12kV, 30ma transformer in its place and cut down

my capacitance level 50%. I get a real healthy 8 to 10-inch spark with this

setup, but no discharges into air by itself. Changing transformers pegged

my primary coil 15 turns -- I suspect this is the problem. Anything more

than fifteen turns seems like a lot though. What to I do? Do I add more

capacitance and tap the primary lower? I played around with this and didn't

get any better results. Do I need to increase my primary to twenty-or-so

turns? This seems like lot. Whatever I do, I guess I need to upgrade my

brand new spark gap for 12kV. Also, at 12kV, my chokes are arcing and

shorting out. Should I coat them in wax or varnish like the secondary?

My questions are: When do you know you are pushing a neon transformer to

hard? Mine just barely got warm. How should I retrofit my coil for a 12 kV,

30ma transformer? Add more windings to the primary? Increase the

capacitance? How will I know this new transformer won't blow?

Daniel Lockhart

X-Mailer: IPAD 2.0

Date: Fri, 22 May 1998 15:18:10 -0500

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Mike Hammer <mhammer@misslink.net>

Subject: re: Dual half lambda coil system.

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

At 10:25 AM 5/22/98 PDT, you wrote:

>It was written:

>>What you are referring to is known as a 1/2 wave coil or

>>bipolar coil. It is just basicly two normal 1/4 wave secondaries

>>joined at their bases and fed by the same primary circuit.

>>

>>Ive only seen two bipolar coils in operation and both of

>>them were poor performers. For that reason Ive never

>>investigated them myself.

>

>Seems like the only way to adjust coupling would be to re-tap the

>innermost primary to move the active turns away from the secondary.

Pretty much. The most control over coupling would be with

the primary diameter so I expect some experimentation with

temporary primary coils would be necessary.

>

>Is the secondary one continuous winding with the primary positioned near

>the midpoint?

Yes this is the most common setup for a bipolar coil.

A 1/2 wave coil with the primary around the middle.

A terminal and streamers off each end. Note that

this type of coil can be run without a RF ground

as the two 1/4 halves work against each other.

An easier to manage but harder to build version

would be two separate 1/4 wave secondaries joined

at their bases. Each has 1/2 of the total primary

inductance. The two primary coils are then run in series

and resonate with a single tank cap.

This is the basis of Robert Stevens twin coil that was

discussed here a few days ago.

TTYL!

Mike Hammer

mhammer@misslink.net

Date: Fri, 22 May 1998 14:20:35 -0600

From: Jonathan Bazemore <jpbazemo@ouray.cudenver.edu>

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Bombarder?

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

What is a bombarder?

X-Mailer: IPAD 2.0

Date: Fri, 22 May 1998 15:42:27 -0500

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: Super First Success, then... Poof!?

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

At 12:31 PM 5/22/98 -0600, you wrote:

>Hey all,

>

>I finally got my spark gap finished and hooked everything up on my first

>coil last night. It didn't take long to tune, and soon I was getting

>16-inch sparks to ground and multiple (no less than 50) 12-inch streamers

>continually coming off the toroid into the air. I was pretty pleased with

>this as I was using saltwater caps and this is (or was...) really kind of a

>small coil (9kV, 30 ma transformer, 3.5 inch secondary).

Very good. 16 inches from a single 9kv/30 is pretty good.

Multiple short streamers is an indication of a too small

toroid.

>

>Then something happened. Right before I cut the power, it stopped. I didn't

>hear anything -- no pop or crackle or explosion. When I gave it power

>again, the transformer just hummed and the safety spark gap fired on one

>side. Anyway, I took the transformer out and found that one side didn't

>spark to ground. Bummer.

>

>What happened? What did I do wrong? I have a good safety gap, and two huge

>torroid chokes. My capacitance is around what I need (according to my

>calculations -- I still haven't got around to buying a cap meter). How can

>I prevent this in the future? My safety gap was firing a lot -- at least

>once per second. I assumed this was because it was set a little close.

Sorry to hear this. Sometimes neons fail despite all the precautions

we take. The safety gaps, RF chokes and damping resistors only

help protect the neon from damage. Its not foolproof.

The only thing I can think of that you may have done

wrong is your total main gap spacing. Having the gap too

wide can damage a neon. For 9kv Ive always found 6 series

gaps at .03 inches to be adequate. Thats a total of .180 inch.

You could probably get away with a little more but the wider

you open it the worse it is for your neon.

If it wasnt gap spacing then perhaps your neon was already

partially damaged. This is common in used neons. A neon may

be taken out of service because of a failure and thrown on

a back shelf for years until you come along and pick it up.

In all those years the tar will settle and move around a bit

allowing a carbon track short to partially heal. When tested

the transformer seems OK but when placed under stress the old

wound opens back up. Ive had this happen several times.

>

>I was irked, so I stuck a 12kV, 30ma transformer in its place and cut down

>my capacitance level 50%. I get a real healthy 8 to 10-inch spark with this

>setup, but no discharges into air by itself. Changing transformers pegged

>my primary coil 15 turns -- I suspect this is the problem. Anything more

>than fifteen turns seems like a lot though. What to I do? Do I add more

>capacitance and tap the primary lower? I played around with this and didn't

>get any better results. Do I need to increase my primary to twenty-or-so

>turns? This seems like lot.

You probably cut your capacitance too much. The matching cap

size for a 9kv/30ma is .0088 mfd and for a 12kv/30 its .0066.

You can probably get by with more. I used as much as .008 mfd

on a single 12kv/30 before with success. Experiment with

various cap sizes.

> Whatever I do, I guess I need to upgrade my brand new spark gap for 12kV.

You probably only need to add a couple more electrodes. I find

8 series gaps at .025-.03 give good performance.

>Also, at 12kV, my chokes are arcing and shorting out. Should I coat them

>in wax or varnish like the secondary?

Did you insulate the cores before you wound the wire on them?

Ferrite and powdered iron core materials are slightly conductive.

Cover them with 2 or 3 layers of good quality vinyl tape before

winding on the wire. After winding I dipped my chokes in polyurethane

mostly just to secure the windings.

>

>My questions are: When do you know you are pushing a neon transformer to

>hard? Mine just barely got warm.

Temperature is usually not an issue. Your run times are not going

to be long enough for heat to be a problem. The safety gap is usually

a good indicator. It should be a little wider than the neon can

break down on its own. This should also be a little wider than your

main gap. If you are getting lots of safety gap activity then you

may have a problem. Poor tuning will often increase safety gap firings.

>How should I retrofit my coil for a 12 kV, 30ma transformer?

Changes should be minimal. Adjustment of the cap size and adding

a couple electrodes to the main gap should suffice.

>How will I know this new transformer won't blow?

You dont. It happens. Take it in stride. Neons are a bit

fragile and we are subjecting them to some pretty harsh treatment.

Failures are bound to occur.

You may want to learn the fine art of neon transformer rebuilding.

Rebuilt neons can be made better than new.

TTYL!

Mike Hammer

mhammer@misslink.net

X-Mailer: IPAD 2.0

Date: Fri, 22 May 1998 15:47:56 -0500

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: Bombarder?

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

At 02:20 PM 5/22/98 -0600, you wrote:

>What is a bombarder?

>

The ultimate of all neons.

This is a large non shunted transformer that

neon shops use to blast out the last bits of

contamination and air from their neon tubes.

They are usually 15kv and current ratings from 150-250 ma

are common. They also come with an external adjustable

inductor for current limiting.

They are also horrendously expensive. Prices over $1000

are normal. For this reason some neon shops have actually

used pole pigs as a bombarders.

You wont find many bombarders around. Perhaps from a neon

shop going out of business. They are just too expensive.

TTYL!

Mike Hammer

mhammer@misslink.net

From: Roy Minnich <rminnich@rolling-meadows.lib.il.us>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: RE: Bombarder?

Date: Fri, 22 May 1998 16:08:33 -0500

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

Hi Jonathan,

I saw a 15kv 500mA transformer at a hamfest so I asked the group what it was and

they said it was a bombarder.  The guys at neon sign shops use the bombarders to

clean the inside of the neon signs of impurities.  The 500mA must burn up anything

in the glass tubes that could later spoil the quality of the gas charge.  I guess.

Roy Minnich

----------

From: 	Jonathan Bazemore[SMTP:jpbazemo@ouray.cudenver.edu]

Reply To: 	Tesla-2 Mailing List

Sent: 	Friday, May 22, 1998 3:20 PM

To: 	Tesla-2 Mailing List

Subject: 	Bombarder?

What is a bombarder?

From: Roy Minnich <rminnich@rolling-meadows.lib.il.us>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: RE: Super First Success, then... Poof!?

Date: Fri, 22 May 1998 17:44:51 -0500

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

Hi,

While we are on the topic of protection devices for our neons I was wondering

about the safety gap.  Is it better to have a center ground electrode or just

one from each leg of the neon??

Thanks and happy holiday weekend!

Roy Minnich

Date: Fri, 22 May 1998 18:01:15 -0500

From: Bert Hickman <bert.hickman@aquila.com>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Organization: Stoneridge Engineering

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Super First Success, then... Poof!?

Sender: tesl-err@emachine.com

Roy Minnich wrote:

>

> Hi,

>

> While we are on the topic of protection devices for our neons I was wondering

> about the safety gap.  Is it better to have a center ground electrode or just

> one from each leg of the neon??

>

> Thanks and happy holiday weekend!

>

> Roy Minnich

>

Roy,

You really need to have the center ground with gaps on each side to the

HV bushings, since this is the ONLY thing that will protect your neon if

you take a "hit" from your secondary to your primary circuit.

-- Bert --

X-Mailer: IPAD 2.0

Date: Fri, 22 May 1998 19:20:07 -0400

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Jim Ryan <jryan@exis.net>

Subject: RE: Spark Gap

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

All, for those of you looking for a .020" or .025" gauge for

spacing gaps,a #11 exacto blade is .020" and your average pin used for

sewing is .025".

Jim

Date: Fri, 22 May 1998 21:34:43 -0500

From: Tedd Payne <tpayne@netnitco.net>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: For Sale:  Neon Transformers (regular & "mini")

Sender: tesl-err@emachine.com

If interested, please contact me DIRECTLY at tpayne@netnitco.net

Please tell me which one(s) you're interested in, and your ZIP CODE to

determine shipping cost.

Thanks,

Tedd

"CONVENTIONAL" neon transformers, not old, not rusty, good

working condition.  If not labeled, the ratings are from my

testing/measuring.  Good for Tesla coil, Jacob's ladder, HV cap charging

(with proper rectification), "hi-potting", experiments, etc.

$18 used  15kv30ma  in=porc ins  out=porc insul (1 loose HV insul)

$18 used  9kv60ma  in=porc ins   out=porc insul (no lid)

$11 used  7.5kv30ma  in=AC cord  out=2 wires, w/switch

$9  used  6kv30ma  in=porc ins   out=porc insul

$7  used  3?kv30ma  in=porc ins   out=porc insul

$14 used  15kv30ma  in=AC cord  out=2 wires  label, w/switch

(one HV wire burned thru insulation near case, to fix splice

new wire or shorten lead and don't route thru case)

$14 used  12kv30ma  in=porc ins   out=porc ins  label

(missing both HV insul)

$12 used  12kv30ma  in=porc ins  out=porc ins  label

(both HV insul broken or loose)

$10 used  12kv30ma  in=porc ins   out=porc ins  label

(label rating is 15kv, but one side is weak, output

is closer to 12kv, one HV insul chipped)

$9  used  6kv30ma  in=porc ins   out=porc ins  label

$19 used  15kv35ma  in=2 wires   out=2 terminal studs, switch

$16 used  15kv30ma  in=3 wires   out=2 term studs, case little bent at corner

$18 used  15kv30ma  in=3 wires   out=2 term studs

$15 used  9kv30ma  in=3 wires   out=2 term studs

$ 9 used  7.5kv35ma  in=AC cord  out=2 wires, replace or bypass bad switch

$10 used  7.5kv35ma  in=porc ins   out=porc ins  missing a little paint

$8  used  6kv25ma  in=porc ins   out=porc ins

$8  used  6kv22ma  in=porc ins   out=porc ins

$9  used  6kv30ma  in=porc ins   out=porc ins  no lid

Those below are small and light weight.  Secondaries not connected to

core, so should be able to connect outputs in series or parallel.

$10 used  5kv10ma  in=2 wires out=2 wires  label  OPEN FRAME 5" long

$10 used  3.5kv8ma  in=2 wires out=2 wires  label  OPEN FRAME 4" long

$10 used  3kv8ma  in=2 wires out=2 wires  label  potted (3x2x1.5")

NEW Neon transformer HV wire:

Flexible gray, 14 gauge, multistrand copper, rated 15KV, $1.00 per foot.

Standard black, 16 gauge, multistrand copper, rated 15KV, $.50 per foot.

AC power cord for transformer, 3-wire, grounded plug, used, $.50

WOOD CHOKE FORMS, might be good for winding chokes to protect neon

transformer on small Tesla coil.  Approximate dimensions in inches:

2.25 length, 1.25 diameter, 4 notches (each .3 wide, .25 deep), .4

diameter hole thru length of form.  6 available @ $1.50 each.

-

From: "Choad" <choad69@geocities.com>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Bombarder?

Date: Sat, 23 May 1998 00:15:11 -0500

X-Mailer: IPAD 2.0

X-MimeOLE: Produced By Microsoft MimeOLE V4.72.2106.4

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Go look here.

http://beasoneon.com/doc/bombarder.html

>What would one look like?

>

>Mike Hammer wrote:

>

>> At 02:20 PM 5/22/98 -0600, you wrote:

>> >What is a bombarder?

>> >

>> The ultimate of all neons.

>> This is a large non shunted transformer that

>> neon shops use to blast out the last bits of

>> contamination and air from their neon tubes.

>>

>> They are usually 15kv and current ratings from 150-250 ma

>> are common. They also come with an external adjustable

>> inductor for current limiting.

>>

>> They are also horrendously expensive. Prices over $1000

>> are normal. For this reason some neon shops have actually

>> used  pole pigs as a bombarders.

>>

>> You wont find many bombarders around. Perhaps from a neon

>> shop going out of business. They are just too expensive.

>>

>> TTYL!

>>

>> Mike Hammer

>> mhammer@misslink.net

Date: Sat, 23 May 1998 00:10:10 -0700

From: mike nicholas <705728@ican.net>

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Bombarder?

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

What would one look like?

Mike Hammer wrote:

> At 02:20 PM 5/22/98 -0600, you wrote:

> >What is a bombarder?

> >

> The ultimate of all neons.

> This is a large non shunted transformer that

> neon shops use to blast out the last bits of

> contamination and air from their neon tubes.

>

> They are usually 15kv and current ratings from 150-250 ma

> are common. They also come with an external adjustable

> inductor for current limiting.

>

> They are also horrendously expensive. Prices over $1000

> are normal. For this reason some neon shops have actually

> used  pole pigs as a bombarders.

>

> You wont find many bombarders around. Perhaps from a neon

> shop going out of business. They are just too expensive.

>

> TTYL!

>

> Mike Hammer

> mhammer@misslink.net

X-Mailer: IPAD 2.0

Date: Sat, 23 May 1998 17:07:44 -0400

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Jim Ryan <jryan@exis.net>

Subject: Tuning

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Hi ALL, I'm ready to construct my primary for my 12KV 30 ma. trans

21" X 4.5" secondary.  9  4.5" X 3/4" spark gaps spaced at .020".

I plan to start the primary first turn 1" larger than the secondary.

20 degrees up 15 turns.

My question is there a Rule Of Thumb on positioning the primary first

turn vertically, in reference to the bottom of the secondary first turn.

Thanks

Jim

X-Mailer: IPAD 2.0

Date: Sun, 24 May 1998 00:33:04 -0500

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: Tuning

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

At 05:07 PM 5/23/98 -0400, you wrote:

>Hi ALL, I'm ready to construct my primary for my 12KV 30 ma. trans

>21" X 4.5" secondary.  9  4.5" X 3/4" spark gaps spaced at .020".

>I plan to start the primary first turn 1" larger than the secondary.

>20 degrees up 15 turns.

>My question is there a Rule Of Thumb on positioning the primary first

>turn vertically, in reference to the bottom of the secondary first turn.

>

>Thanks

>Jim

The point of greatest coupling will be when the bottom turn

of the secondary lies in the same plane as the primary turns.

This will be the lowest the secondary will need to go.

This position will probably overcouple so you will need

to raise the secondary 1 to 2 inches from this position.

Allow for plenty of adjustment range.

TTYL!

Mike Hammer

mhammer@misslink.net

Date: Sun, 24 May 1998 10:00:36 +0300

From: Abdullah Fazaa <FAZAAAM@mail.aramco.com.sa>

Sender: tesl-err@emachine.com

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Thin Wire

X-Mailer: IPAD 2.0

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

hi  every one ...

is the thin wire - which is connecting the tesla coil components -

affecting the performance of the set and the length of the out put

spark..

X-Mailer: IPAD 2.0

Date: Sun, 24 May 1998 17:14:38 -0400

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Jim Ryan <jryan@exis.net>

Subject: Re: Tuning

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

At 12:33 AM 5/24/98 -0500, you wrote:

>At 05:07 PM 5/23/98 -0400, you wrote:

>>Hi ALL, I'm ready to construct my primary for my 12KV 30 ma. trans

>>21" X 4.5" secondary.  9  4.5" X 3/4" spark gaps spaced at .020".

>>I plan to start the primary first turn 1" larger than the secondary.

>>20 degrees up 15 turns.

My question is "this time" how many metal wood screws can I leave in and

around the primary coil supports ?

Thanks for all your help Mike

Jim

Date: Sun, 24 May 1998 18:31:27 -0500

From: Mark Olson <molson1@isd.net>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Tuning

Sender: tesl-err@emachine.com

Jim Ryan wrote:

>

>

> My question is "this time" how many metal wood screws can I leave in and

> around the primary coil supports ?

>

> Thanks for all your help Mike

>

> Jim

This also brings to mind another question:  In many pictures of coils that

I've seen, there are transformers, spark gaps, caps, etc., mounted

underneath the primary coil.  Is there a "rule of thumb" to follow w/r to

this?  I would imagine that there could be an effect on tuning, or an

unknown amount of energy loss if mounted too close or in the wrong

configuration.  Also the coupling of energy back into places it shouldn't be

could perhaps cause damage?

Thanks in advance,

mark

Date: Sun, 24 May 1998 18:15:33 -0600

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Daniel Lockhart <molly@cmn.net>

Subject: Re: Super First Success, then... Poof!?

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

I wrote,

>>Then something happened. Right before I cut the power, it stopped. I didn't

>>hear anything -- no pop or crackle or explosion. When I gave it power

>>again, the transformer just hummed and the safety spark gap fired on one

>>side. Anyway, I took the transformer out and found that one side didn't

>>spark to ground. Bummer.

>>

>>What happened? What did I do wrong? I have a good safety gap, and two huge

>>torroid chokes. My capacitance is around what I need (according to my

>>calculations -- I still haven't got around to buying a cap meter). How can

>>I prevent this in the future? My safety gap was firing a lot -- at least

>>once per second. I assumed this was because it was set a little close.

Mike replied:

>Sorry to hear this. Sometimes neons fail despite all the precautions

>we take. The safety gaps, RF chokes and damping resistors only

>help protect the neon from damage. Its not foolproof.

Yeah. I know. I was just hoping I wouldn't taste reality that soon...  :-)

>>I was irked, so I stuck a 12kV, 30ma transformer in its place and cut down

>>my capacitance level 50%. I get a real healthy 8 to 10-inch spark with this

>>setup, but no discharges into air by itself. Changing transformers pegged

>>my primary coil 15 turns -- I suspect this is the problem. Anything more

>>than fifteen turns seems like a lot though. What to I do? Do I add more

>>capacitance and tap the primary lower? I played around with this and didn't

>>get any better results. Do I need to increase my primary to twenty-or-so

>>turns? This seems like lot.

>

>You probably cut your capacitance too much. The matching cap

>size for a 9kv/30ma is .0088 mfd and for a 12kv/30 its .0066.

>You can probably get by with more. I used as much as .008 mfd

>on a single 12kv/30 before with success. Experiment with

>various cap sizes.

I added separate bottle caps one at a time, tuning each time to see the

difference. I've got it to break out by itself a lot of places and it pulls

a healthy spark to a ground. Performance isn't so good though. The max

spark to a ground is exactly one foot. When there isn't any ground, there's

about six different streamers continually coming off between eight to ten

inches long. I haven't upgraded my spark gap yet. Is this the problem? It

seems like I should be able to get at least sixteen inches.

I put my dog's aluminum water bowl on top of my toroid to increase the

output terminal size just to see what a larger toroid would be like. It

increased in surface area by about 50% (I'm guesstimating). Tuning became

fiendishly difficult. In fact, I couldn't get it tuned. In one spot there

were no breaks into air and about six inches to a ground. Tapped just three

inches further up the primary the secondary would short out -- a problem I

attribute to overcoupling (right?). It was one or the other -- wimpy sparks

or twenty inch sparks down the secondary. Was I increasing the toroid size

too much, or is my secondary a little prone to shorting out?

>>Also, at 12kV, my chokes are arcing and shorting out. Should I coat them

>>in wax or varnish like the secondary?

>

>Did you insulate the cores before you wound the wire on them?

>Ferrite and powdered iron core materials are slightly conductive.

>Cover them with 2 or 3 layers of good quality vinyl tape before

>winding on the wire. After winding I dipped my chokes in polyurethane

>mostly just to secure the windings.

Yup. I wrapped the ferrite cores with two highly overlapping layers of

electrical tape. The arcing is occurring on the outside of the windings,

not the sides next to the ferrite. Should I tape around the windings, or

varnish them?

>You may want to learn the fine art of neon transformer rebuilding.

>Rebuilt neons can be made better than new.

Well, now I guess I have another transformer to practice on...

I'm very pleased with my coil so far and would really like to thank

everybody on this list for helping me out. I would have ended up with a

real lousy performer on my own. Special thanks to Mike Hammer and Bert

Hickman for answering my (sometimes? usually?) newbe questions! Hey --

Thanks a lot everybody.

Note to anybody who hasn't made a Tesla coil yet: It's really cool. I

always knew it would be neat, but it's never like you expect. Even if you

only get sparks half what they should be you'll be satisfied! (for a while)

Don't under estimate making the primary though -- it took me a long time to

make mine.

Daniel Lockhart

X-Mailer: IPAD 2.0

Date: Mon, 25 May 1998 11:50:56 -0400

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Jim Ryan <jryan@exis.net>

Subject: Primary

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Hello all, WOW I'm putting together my primary today. I never figured

50' of copper tubing could get so twisted. I thought I would open

the box and just set in place. Little did I know it comes wound

so that both ends are on the large diameters. you need about 6 hands

to get it wound or un wound. This project has been a test of all of

the skills I have acquired over many years.

Jim

X-Mailer: IPAD 2.0

Date: Mon, 25 May 1998 11:42:31 -0500

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: Thin Wire

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

At 10:00 AM 5/24/98 +0300, you wrote:

>hi  every one ...

>

>is the thin wire - which is connecting the tesla coil components -

>affecting the performance of the set and the length of the out put

>spark..

>

Using small guage wire is OK for connections from

the neon to main gap and components in between.

(safety gap, RF chokes , damping resistors)

The current flow in this area is low so losses

are also low.

However once you get into the Tesla tank circuit

(main gap, main cap, primary coil) the circulating

currents are very high. Peak currents of tens of

amperes are likely in even quite small coils.

Hundreds of amperes in medium sized coil. This kind

of current flow demands large low loss conductors.

The I2R losses in small guage wire would severely

affect the output.

When choosing wiring for your tank circuit look to what

you are using for your primary coil. If you are using 1/4

inch copper tubing for your primary you should have conductors

with a similar cross section for the tank circuit wiring.

TTYL!

Mike Hammer

mhammer@misslink.net

X-Mailer: IPAD 2.0

Date: Mon, 25 May 1998 11:48:18 -0500

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: Tuning

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

At 06:31 PM 5/24/98 -0500, you wrote:

>

>

>Jim Ryan wrote:

>

>>

>>

>> My question is "this time" how many metal wood screws can I leave in and

>> around the primary coil supports ?

>>

>> Thanks for all your help Mike

>>

>> Jim

Jim,

As few as possible. My primary support is made from wood

but is fastened together with wooden pins and glue. No

screws or nails.

>

>  This also brings to mind another question:  In many pictures of coils that

>I've seen, there are transformers, spark gaps, caps, etc., mounted

>underneath the primary coil.  Is there a "rule of thumb" to follow w/r to

>this?  I would imagine that there could be an effect on tuning, or an

>unknown amount of energy loss if mounted too close or in the wrong

>configuration.  Also the coupling of energy back into places it shouldn't be

>could perhaps cause damage?

>

>Thanks in advance,

>

>mark

Mark,

Yes Ive seen a lot of coils built that way. My own

feeling is that you would have some level of power

loss with this many conductors this close to the primary

coil. However the loss would probably be minor as long as

there was a separation of a few inches.

The one thing to watch out for is any closed conductive ring.

This would represent a shorted turn and will really suck the

energy out of your system.

TTYL!

Mike Hammer

mhammer@misslink.net

X-Mailer: IPAD 2.0

Date: Mon, 25 May 1998 11:52:59 -0500

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: Primary

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

At 11:50 AM 5/25/98 -0400, you wrote:

>Hello all, WOW I'm putting together my primary today. I never figured

>50' of copper tubing could get so twisted. I thought I would open

>the box and just set in place. Little did I know it comes wound

>so that both ends are on the large diameters. you need about 6 hands

>to get it wound or un wound. This project has been a test of all of

>the skills I have acquired over many years.

>

> Jim

Good luck Jim. It can be a pain and sometimes 6 hands arent enough.

Try to utilize the curvature of the tubing wherever possible.

Bend it just enough to go where you want it. If you bend it

too much the copper will work harden and be even harder to shape

properly.

Dont worry if your primary comes out looking a little ugly.

An ugly primary has just as much inductance as a pretty one

and will work just as well. Just dont tell anybody and dont

take any pictures. No one will be the wiser. :)

TTYL!

Mike Hammer

mhammer@misslink.net

X-Mailer: IPAD 2.0

Date: Mon, 25 May 1998 12:05:41 -0500

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: Super First Success, then... Poof!?

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

At 06:15 PM 5/24/98 -0600, you wrote:

<SNIP>

>

>I put my dog's aluminum water bowl on top of my toroid to increase the

>output terminal size just to see what a larger toroid would be like. It

>increased in surface area by about 50% (I'm guesstimating). Tuning became

>fiendishly difficult. In fact, I couldn't get it tuned. In one spot there

>were no breaks into air and about six inches to a ground. Tapped just three

>inches further up the primary the secondary would short out -- a problem I

>attribute to overcoupling (right?). It was one or the other -- wimpy sparks

>or twenty inch sparks down the secondary. Was I increasing the toroid size

>too much, or is my secondary a little prone to shorting out?

You need to address that overcoupling problem before proceeding.

Raise up your secondary 1 inch away from the primary coil

and try once again. If it still occurs raise it another inch.

Inspect your secondary coil. If there are many pock marks in

the coating from arcing then I would slap a few more coats

of varnish on it. Also look down the inside of the form

for any evidence of arcing. Internal arcing is bad news

and will require a rebuild.

Once you get the overcoupling under control you want to increase

your cap size to around .0066 mfd for your 12kv/30ma neon.

Then increase your toroid size in order to get the frequency

lower so you can keep as many primary turns tapped in as possible.

In order to get a smoother breakout from your larger toroid try

placing a foil bump along the outer perimeter of the toroid. I use

a piece of foil tape. This will promote breakouts from the bump.

If you look at the pics on my web page you will note that all the

streamers are to one side. This is due to a foil bump. This also

gives you some directional control over the streamers.

>

>>>Also, at 12kV, my chokes are arcing and shorting out. Should I coat them

>>>in wax or varnish like the secondary?

>>

>>Did you insulate the cores before you wound the wire on them?

>>Ferrite and powdered iron core materials are slightly conductive.

>>Cover them with 2 or 3 layers of good quality vinyl tape before

>>winding on the wire. After winding I dipped my chokes in polyurethane

>>mostly just to secure the windings.

>

>Yup. I wrapped the ferrite cores with two highly overlapping layers of

>electrical tape. The arcing is occurring on the outside of the windings,

>not the sides next to the ferrite. Should I tape around the windings, or

>varnish them?

I wouldnt tape them but I did dip my chokes in polyurethane varnish.

A couple dips would give you a nice coating and improve the insulation.

TTYL!

Mike Hammer

mhammer@misslink.net

From: Dsurfr <Dsurfr@aol.com>

Date: Mon, 25 May 1998 18:14:52 EDT

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: transformer ground

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

I just finished my dedicated ground today. Three pieces of 4' x 1" copper pipe

spaced 5' apart connected in parallel 6" below ground level. Hope it's good

enough. My question is whether I should connect the ground to my NST to this

or should it just take the ground from my secondary?

Thanks for the help.

Jim

Date: Mon, 25 May 1998 17:22:42 -0500

From: Bert Hickman <bert.hickman@aquila.com>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Organization: Stoneridge Engineering

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: transformer ground

Sender: tesl-err@emachine.com

Dsurfr wrote:

>

> I just finished my dedicated ground today. Three pieces of 4' x 1" copper pipe

> spaced 5' apart connected in paralel 6" below ground level. Hope it's good

> enough. My question is whether I should connect the ground to my NST to this

> or should it just take the ground from my secondary? Thanks for the help. Jim

Jim,

Connect the ground of your NST to the RF ground. Also, if you are using

a safety gap, connect center grounded electrode to this same RF ground.

This way, you'll safely route any high voltage strikes to your primary

circuit to the RF ground.

-- Bert --

X-Mailer: IPAD 2.0

Date: Tue, 26 May 1998 03:00:10 -0700

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: bob golding <yubba@clara.net>

Subject: cap values

Cc: tesla-2@emachine.com

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

fellow coilers

Yet another cap question! I have finished the first of my caps. I am going

to use three in series. I am using 25 mil of poly between each set of

plates (6" by 8 3/4")I make this 940 pF per plate. 53100/940=56 plates ok?.

I then tested this on my new toy a Wayne Kerr B421 universal bridge. It

came out as 38 nf. I have tested the bridge with some new 2% polyester caps

and it seems to be accurate. I then added more plates to bring it up to

53100nF. This brought the number of plates up to 72. I know the oil will

increase the value some but should I stick with what I have? go by the

maths? or compromise somewhere in the middle. I want to sort this out

before I get everything covered with oil if possible. Is this normal with

homemade caps? Does it matter much? is it better to be above or below the

recommended value?

Many thanks.

bob golding lightning without rain

bob yubba@clara.net

Date: Tue, 26 May 1998 06:27:30 -0500

From: Bert Hickman <bert.hickman@aquila.com>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Organization: Stoneridge Engineering

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: cap values

Sender: tesl-err@emachine.com

bob golding wrote:

>

> fellow coilers

> Yet another cap question! I have finished the first of my caps. I am going

> to use three in series. I am using 25 mil of poly between each set of

> plates (6" by 8 3/4")I make this 940 pF per plate. 53100/940=56 plates ok?.

> I then tested this on my new toy a Wayne Kerr B421 universal bridge. It

> came out as 38 nf. I have tested the bridge with some new 2% polyester caps

> and it seems to be accurate. I then added more plates to bring it up to

> 53100nF. This brought the number of plates up to 72. I know the oil will

> increase the value some but should I stick with what I have? go by the

> maths? or compromise somewhere in the middle. I want to sort this out

> before I get everything covered with oil if possible. Is this normal with

> homemade caps? Does it matter much? is it better to be above or below the

> recommended value? Many thanks.

>

> bob golding lightning without rain

>

> bob yubba@clara.net

Bob,

Not to fear - your math looks OK. For design calculations, use a

dielectric constant of 2.0 to compensate for gaps between the plates and

the LDPE. The significantly lower capacitance you're seeing on the "dry"

cap simply reflects the fact you've got an additional air-space between

the plates and the LDPE. As long as your caps are identically

constructed, you should be OK as far as balancing voltage stress between

them, and the Primary tap adjustment should still allow you to tune your

coil properly. Your design should support being driven off a 15 KV 90MA

source.

Good luck, and safe cappin' to you!

-- Bert --

From: "Thomas McGahee" <tom_mcgahee@sigmais.com>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: cap values

Date: Tue, 26 May 1998 07:51:14 -0400

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Bob,

The oil will bring the value up considerably. Right now you have

a lot of air in there with a dielectric constant of 1. When you

add the oil the air will be displaced with oil with a dielectric constant

close to 2. The addition of the oil will bring you close to your

original calculated value.

Hope this helps.

Fr. Tom McGahee

----------

> From: bob golding <yubba@clara.net>

> To: Tesla-2 Mailing List <tesla-2@emachine.com>

> Cc: tesla-2@emachine.com

> Subject: cap values

> Date: Tuesday, May 26, 1998 6:00 AM

>

> fellow coilers

> Yet another cap question! I have finished the first of my caps. I am going

> to use three in series. I am using 25 mil of poly between each set of

> plates (6" by 8 3/4")I make this 940 pF per plate. 53100/940=56 plates ok?.

> I then tested this on my new toy a Wayne Kerr B421 universal bridge. It

> came out as 38 nf. I have tested the bridge with some new 2% polyester caps

> and it seems to be accurate. I then added more plates to bring it up to

> 53100nF. This brought the number of plates up to 72. I know the oil will

> increase the value some but should I stick with what I have? go by the

> maths? or compromise somewhere in the middle. I want to sort this out

> before I get everything covered with oil if possible. Is this normal with

> homemade caps? Does it matter much? is it better to be above or below the

> recommended value? Many thanks.

>

> bob golding lightning without rain

>

> bob yubba@clara.net

Date: Tue, 26 May 1998 10:19:13 -0700

From: Gene Waldo <gwaldo@1stnetusa.com>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Organization: S.A.P.I.E.N.T. (Surly, Agressive, Politically Incorrect Enemies of Non Thinkers)

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Cap makers: interesting site.

Sender: tesl-err@emachine.com

All,

Don't know what can be done here, but have sent off some inquiries.

Food for thought!

http://www.looksmart.com/r_search?lu&skip=20&key=electrical%2c+conductive+coatings&search=1

----------------------------------------

Gene Waldo -  gwaldo@1stnetusa.com  - "Igor in Residence"

---------------------------------------------------------------

What's good for the planet is more OZONE! See the machinery form up!

NEW PAGE!   http://members.1stnetusa.com/danmurphy/waldo.htm

From: Roy Minnich <rminnich@rolling-meadows.lib.il.us>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: 140vac input

Date: Tue, 26 May 1998 18:55:27 -0500

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

Hi,

What happens when you run your 12kv 30mA NST on the 140 vac output of a 10 amp variac?

If you put 120 vac in and get 12000 vac out you must have a 100 to 1 turn ratio. Right?  So then 140 vac x 100 = 14000 vac?  What happens to the mA?

Thanks,

Roy Minnich

Date: Tue, 26 May 1998 20:48:11 -0700

From: Scott MacGregor <smacgregor@aasp.net>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Organization: EES,Co.

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: 140vac input

Sender: tesl-err@emachine.com

Roy Minnich wrote:

>

> Hi,

>

> What happens when you run your 12kv 30mA NST on the 140 vac output of a 10 amp variac?

>

> If you put 120 vac in and get 12000 vac out you must have a 100 to 1 turn ratio. Right?

>  So then 140 vac x 100 = 14000 vac?  What happens to the mA?

>

> Thanks,

> Roy Minnich

>

Roy,

I'm no expert with NSTs. But, since they are current limiting by design,

I suppose the core temperature will rise due to saturation. The output

current will remain about the same, 30ma. The extra energy applied to

the primary will simply be wasted in the core as heat, which could cause

failure.

--

Regards,

Scott MacGregor EES,Co.

************************************************************

Enterprise Engineering Services Co.

E-mail smacgregor@aasp.net

Member: Cable Television Advisory Committee, Plainville, MA

http:/www.expage.com/page/cableguy

************************************************************

From: Dsurfr <Dsurfr@aol.com>

Date: Tue, 26 May 1998 20:23:08 EDT

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Another cap question

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Sorry to ask another question on caps but I want to move up from my SW's soon.

Can someone explain why rolled poly caps are any better than stacked plate? It

seems like you can adjust the value of the stacked plate caps a little easier

& they sound like they would be a little easier to construct.

I think I understand the concept of building higher rated caps & connecting

them in series to lower their value but allow higher voltage load. Is there

some way to calculate these benefits or is it just a matter of cost? Also is

there any benefit to connecting them in series & then in parallel? Awhile back

Brandon Shaw had an idea for using polypropelene sheet protectors for a

stacked cap that seemed quite good. The dialectric constants seem quite close

to those of polypropelene & the relative cost factor seemed good as well. Has

anyone tried his idea?

Once again I apologize for another cap question from a newbie but if you don't

ask you won't get any answers.

Thanks,

Jim

X-Mailer: IPAD 2.0

Date: Tue, 26 May 1998 20:07:55 -0500

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: 140vac input

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

At 08:48 PM 5/26/98 -0700, you wrote:

>Roy Minnich wrote:

>>

>> Hi,

>>

>> What happens when you run your 12kv 30mA NST on the 140 vac output of a 10 amp variac?

>>

>> If you put 120 vac in and get 12000 vac out you must have a 100 to 1

>>turn ratio. Right?  So then 140 vac x 100 = 14000 vac?  What happens to the mA?

>>

>> Thanks,

>> Roy Minnich

>>

>Roy,

>

>I'm no expert with NSTs. But, since they are current limiting by design,

>I suppose the core temperature will rise due to saturation. The output

>current will remain about the same, 30ma. The extra energy applied to

>the primary will simply be wasted in the core as heat, which could cause

>failure.

>--

>

>Regards,

>Scott MacGregor EES,Co.

Scott, Roy and others,

I run 140VAC input to my neons all the time with no ill

effect. Roy is correct that you will be getting 14kv output

from a 12kv neon with 140 VAC input. The current will also

rise past the name plate rating. Most of my 30ma neons will

put out 34-38 ma with 140 in. Core saturation or heating does

not seem to be any problem.

However remember you are working your neon harder. More

voltage means more wear and tear on the insulation.

Also be sure you take into consideration that excess

voltage when building your caps.

TTYL!

Mike Hammer

mhammer@misslink.net

Date: Tue, 26 May 1998 22:51:23 -0600

From: Jonathan Bazemore <jpbazemo@ouray.cudenver.edu>

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: re: Spark Related Question.

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Sorry again that this may be slightly off topic, but it is

spark-related, and who better to ask.

The question is about those fancy spark plugs that have a serpent's tongue

type double contact at the end--my impression is that anyone in the auto parts

industry informed enough to tell you if they work any better or worse is probably

a paid consultant under non-disclosure.

So for a more balanced assessment, I would like to ask you coiling experts

about spark formation in these 10 dollar high end plugs, and if it is worth the

money, or just a gimmick.

Are you actually getting two sparks out of one plug?

And could one of these things be used as a spark gap for a tesla

coil, if you had some efficient means for blowing the hot gases off the gap

(i.e.,fan, vacuum, etc.)

Thanks for any commentary on these questions--and please don't

reply if you work for a spark plug company ;-)

X-Mailer: IPAD 2.0

Date: Wed, 27 May 1998 08:47:02 +0300

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Marco Denicolai <marco@vistacom.fi>

Subject: 8kV MW oven supply measured

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

I had eventually a chance of performing a real measurement on the circuit

below. I used a digital scope and applied to the mains 3.5 VAC instead

of 220 VAC. After the measurements, I scaled up the results and these

I am reporting in the following.

| |

--------|| ||-----| |----*---------- V-

* || || *   | |    |

|| ||          -----

220V        || ||          \   /

|| ||           \ /

|| ||          -----

|| ||            |

--------|| ||------------*--- GND

|

|

--------|| ||------------*

* || || *          |

|| ||          -----

|| ||          \   /

220V       || ||           \ /

|| ||          -----

|| ||     | |    |

--------|| ||-----| |----*----------- V+

| |

Voltage V+ to GND oscillates between 6270 V and 0 (2200 VAC * 2 * sqrt(2) =

6270 Vpp).

Voltage between V- and GND oscillates between -6270 V and 0.

If the transformers are in phase, voltage between V+ and V- is almost DC at

6270 VDC.

V+ |----|    |----|    |----|

|    |----|    |----|    |

GND ------------------------------     V+ - V- = 6.3kV DC

|----|    |----|    |----|

V- |    |----|    |----|    |----

If the transformers are NOT in phase, the output is a sinusoid at 50 Hz

going from 0

to 6270 Vpp * 2 = 12540 Vpp.

V+ |----|    |----|    |----|

|    |----|    |----|    |----

GND ------------------------------     V+ - V- = 0 to 12.5kV DC pulsed

|    |----|    |----|    |----

V- |----|    |----|    |----|

I intend to use the last configuration (transformers NOT in phase, 12.5kV

pulsed output).

I think static spark gaps won't easily quench, for the microwave oven

transformers are capable to pump even 500 mA. Instead, a rotating spark gap

would be forced to quench at my rotating rate.

Any suggestion about the spark gap choice?

________________________________________________________________________

Marco Denicolai                   Vista Communication Instruments, Inc.

Hardware Development Manager      www.vistacom.fi

marco@vistacom.fi                 Kaisaniemenkatu 13 A

fax:    +358-9-622-5610           SF-00100 HELSINKI

phone:  +358-9-622-623-15         Finland

Remember, Murphy was an optimist! I am not...

________________________________________________________________________

Date: Wed, 27 May 1998 01:33:01 -0600

From: Jonathan Bazemore <jpbazemo@ouray.cudenver.edu>

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: 8kV MW oven supply measured

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

You do seem to be departing a bit from the orthodox lore of

coilbuilding, so it shall be interesting to see how things work out

for you--I hope well, because gutted microwaves are relatively

abundant, so if you could make a coil power supply from one,

I would be quite happy.

> Any suggestion about the spark gap choice?

>

Yes, why don't you use the RQ gap (sp?)  It's like a bunch of copper pipes

colinearly aligned with small gaps between them.  You can cool it with a fan.

The spark is distributed among many small gaps, so heating/quenching issues are

not as acute.

Hope this helps!

X-Mailer: IPAD 2.0

Date: Wed, 27 May 1998 02:28:12 -0700

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: bob golding <yubba@clara.net>

Subject: Re: Another cap question

Cc: tesla-2@emachine.com

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

At 20:23 26/05/98 EDT, you wrote:

snip:

>Brandon Shaw had an idea for using polypropelene sheet protectors for a

>stacked cap that seemed quite good. The dialectric constants seem quite close

>to those of polypropelene & the relative cost factor seemed good as well. Has

>anyone tried his idea?

>Once again I apologize for another cap question from a newbie but if you don't

>ask you won't get any answers. Thanks, Jim

hi Jim

I am experimenting with this idea right now. What I have found is that if

you place one sheet of poly in front of you with the holes to the left and

then take the next sheet of poly and lay it so the holes are to the right

you can  then slide them apart by 1/2" and  use the holes to mark out for

your end pieces. I am using 1/4" acrylic sheet. I drilled 6 holes

altogether 4 at the ends and 2 in the middle. I then cut my ally foil using

mike hammers design( see archives) and placed it inside a sheet of poly and

secured it with 2 small pieces of tape to stop it moving around. I marked

out an outline on card 1" in from all 4 sides to try and get all the pieces

of foil in the same relative position. I then placed cable ties though the

holes in the end plate and started with a foil layer followed by 4 layers

of poly and another foil layer. I then put 4 more layers of poly  followed

by another foil sheet. To save problems with miscounting sheets (VERY easy

to do) I placed a sheet of paper at the start of the first foil layer and

another sheet after the next foil layer . I then pinned the sheets of paper

together with paper clips to indicate a pair of plates. I had lots of

problems before I did this with miscounts. Don't under estimate how long

this will take,  I have spent more time on this than any other part of my

coil. I havent tried out this design yet so there might be unforeseen

problems but it is certainly  a lot cheaper than buying poly sheet from the

hardware store and gets round the problem of the poly sliding around. I

also cut open the side of the sheet opposite the holes to eliminate air

pockets. Hope this makes sense and helps.

bob golding lightning without rain

bob yubba@clara.net

X-Mailer: IPAD 2.0

Date: Wed, 27 May 1998 12:40:17 +0300

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Marco Denicolai <marco@vistacom.fi>

Subject: Re: 8kV MW oven supply measured

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

At 01:33 27/05/98 -0600, you wrote:

>    You do seem to be departing a bit from the orthodox lore of

>coilbuilding, so it shall be interesting to see how things work out

>for you--I hope well, because gutted microwaves are relatively

>abundant, so if you could make a coil power supply from one,

>I would be quite happy.

I'll keep you informed.

>

>

>

>> Any suggestion about the spark gap choice?

>>

>

>    Yes, why don't you use the RQ gap (sp?)  It's like a bunch ofcopper pipes

>colinearly aligned with small gaps between them.  You can cool it with a fan.

>The spark is distributed among many small gaps, so heating/quenching issues

>are not as acute.

>

In my understanding, when the spark gap fires, the HV capacitor and the

transformer are both pumping current through the gap. Now, the capacitor

discharges quite quickly and a NST is capable of feeding max 30 mA of current

through the gap.

But my microwave oven can feed 500 mA (!) of current, so I think it will be much

difficult to quench a static gap by just blowing air.

Instead a rotating gap will "separate" the electrodes thus forcing the arc to

terminate. What do you think about that?

________________________________________________________________________

Marco Denicolai                   Vista Communication Instruments, Inc.

Hardware Development Manager      www.vistacom.fi

marco@vistacom.fi                 Kaisaniemenkatu 13 A

fax:    +358-9-622-5610           SF-00100 HELSINKI

phone:  +358-9-622-623-15         Finland

Remember, Murphy was an optimist! I am not...

________________________________________________________________________

Date: Wed, 27 May 1998 07:00:27 -0500

From: Bert Hickman <bert.hickman@aquila.com>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Organization: Stoneridge Engineering

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: 8kV MW oven supply measured

Sender: tesl-err@emachine.com

Marco Denicolai wrote:

>

> I had eventually a chance of performing a real measurement on the circuit

> below. I used a digital scope and applied to the mains 3.5 VAC instead

> of 220 VAC. After the measurements, I scaled up the results and these

> I am reporting in the following.

>

>                          | |

>        --------|| ||-----| |----*---------- V-

>              * || || *   | |    |

>                || ||          -----

>    220V        || ||          \   /

>                || ||           \ /

>                || ||          -----

>                || ||            |

>        --------|| ||------------*--- GND

>                                 |

>                                 |

>        --------|| ||------------*

>              * || || *          |

>                || ||          -----

>                || ||          \   /

>     220V       || ||           \ /

>                || ||          -----

>                || ||     | |    |

>        --------|| ||-----| |----*----------- V+

>                          | |

>

> Voltage V+ to GND oscillates between 6270 V and 0 (2200 VAC * 2 * sqrt(2) =

> 6270 Vpp).

> Voltage between V- and GND oscillates between -6270 V and 0.

> If the transformers are in phase, voltage between V+ and V- is almost DC at

> 6270 VDC.

>

>   V+ |----|    |----|    |----|

>      |    |----|    |----|    |

>  GND ------------------------------     V+ - V- = 6.3kV DC

>      |----|    |----|    |----|

>   V- |    |----|    |----|    |----

>

> If the transformers are NOT in phase, the output is a sinusoid at 50 Hz

> going from 0

> to 6270 Vpp * 2 = 12540 Vpp.

>

>   V+ |----|    |----|    |----|

>      |    |----|    |----|    |----

>  GND ------------------------------     V+ - V- = 0 to 12.5kV DC pulsed

>      |    |----|    |----|    |----

>   V- |----|    |----|    |----|

>

> I intend to use the last configuration (transformers NOT in phase, 12.5kV

> pulsed output).

>

> I think static spark gaps won't easily quench, for the microwave oven

> transformers are

> capable to pump even 500 mA. Instead, a rotating spark gap would be forced

> to quench at

> my rotating rate.

>

> Any suggestion about the spark gap choice?

>

Marco,

With the voltage doubler circuit, getting either a static or rotary gap

to initially fire should not be a problem if you run with the

transformers out of phase. The downside is that the coil will only

"fire" every other half-cycle of the mains. The in-phase circuit would

allow you to fire every half-cycle, but you'll only have half the

voltage. Assuming high air flow through a static gap, or using a rotary

gap, either approach should work. The major challenge will be in keeping

HV RF from destroying your diodes or transformers.

-- Bert --

X-Originating-IP: [199.217.140.22]

From: "Bill the arcstarter" <arcstarter@hotmail.com>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: re: Spark Related Question.

Date: Wed, 27 May 1998 06:50:03 PDT

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

It was written,

>    The question is about those fancy spark plugs that have a serpent's

tongue type double contact at the end--my impression is that

>anyone in the auto parts industry informed enough to tell you if they

>work any better or worse is

Supposedly they won't help a newer vehicle with a decent modern high

energy ignition system. They might help an older car/truck with a weaker

(non-HEI) ignition.  I'd say spend your money on new wires/cap/rotor or

buy Bosch platinum plugs ($2/ea) before trying these things.

As far as Tesla use - most auto plugs contain a resistive core - which

will cut your primary current down to about zilch.   Good for reducing

EFI, as in a car.  Bad for creating EFI, as in a tesla coil :)

If you want to try using spark plugs as a tesla gap, use nonresistor

lawnmower plugs.  I'm told that most plugs with an "R" in the part

number contain resistor cores.  I fooled with this a bit but wasn't

overly impressed with the results.

Go build an RQ static gap - you will be rewarded for your efforts.

-Bill

Date: Wed, 27 May 1998 08:52:36 -0700

From: Gene Waldo <gwaldo@1stnetusa.com>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Organization: S.A.P.I.E.N.T. (Surly, Agressive, Politically Incorrect Enemies of Non Thinkers)

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

CC: comahal@cci-29palms.com

Subject: Re: W-M project

Sender: tesl-err@emachine.com

Jim Ryan wrote:

>

> In your schematic diagram you have a G3 at the bottom of L2.

> What do you call that Gap? Is that a ground on a pole to

> draw sparks off the secondary ?

>

> Jim

Yes.  Actually that is physically the top of 2ndary. 'G3' is "Play

Space", where one's demands of the heart may be met. 'Pin-wheels', 'live

persons' in various configurations*, ladders, ceiling screen elements,

some say even food, your cat**....

*  ** Do NOT try these without a thorough understanding of the dangers

involved!! Injuries may occur, from LETHAL to severe scratches!

Cats are quite suseptible to hi voltage effects and can make interesting

displays.

I used to stand barefooted in a shallow pan of salted water, which was

connected to the upper terminal of the 2ndary. Granted, we had a

severely under engineered coil, but the effects were still worth the

effort! Once I held my sister's cat.... this one wasn't!

Dan Is reluctant to post the accompanying legend for the schem.

Technical aside: recieved no commentary on the utility, need or

electrical placement of (V) & (I) in the schematic....

------------------------------------------------------

Gene Waldo -  gwaldo@1stnetusa.com  - "Igor in Residence"

---------------------------------------------------------------

What's good for the planet is more OZONE! See the machinery form up!

NEW PAGE!   http://members.1stnetusa.com/danmurphy/waldo.htm

X-Originating-IP: [199.217.140.22]

From: "Bill the arcstarter" <arcstarter@hotmail.com>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: W-M project

Date: Wed, 27 May 1998 10:06:50 PDT

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

It was written:

>Cats are quite suseptible to hi voltage effects and can make

>interesting displays.

Excellent!  I *knew* there had to be practical use for these things

(both cats AND Tesla coils).

Our cats are infatuated with my 15kv 60 ma Jacob's ladder -but my wife

makes me turn it off *just* when things would get really interesting...)

They don't like the coil - too noisy.

Please keep me informed on your pending research into this (previously

unexplored) aspect of coiling :).

I'm confident the key to our sucessful coil was the use of a friend's

cat as a dust brush - used to to de-lint the inside of the 4.25 inch

secondary prior to applying the polyurethane coatings.  Of course - then

I had to de-fur the PVC...

Some say that cats (especially the long-haired breeds - they last

longer) make good lower brushes for the larger Van De Graph generators,

but I haven't had time to test this theory.

"I love animals - they are delicious"

-Bill

From: Roy Minnich <rminnich@rolling-meadows.lib.il.us>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: RE: 140vac input

Date: Wed, 27 May 1998 13:18:07 -0500

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

Hi,

I ran the amp test on my neon (12kv 30mA) and got 29.5mA.  I guess it's

a little out of spec.  I got this neon from Tedd Payne and it had no plate

so it must have been a factory reject due to the slightly low output..... I guess.

Is this the way it goes?  I mean you'd call a neon that puts out 29.xxmA a 30mA?

Roy Minnich

----------

From: 	Mike Hammer[SMTP:mhammer@misslink.net]

Reply To: 	Tesla-2 Mailing List

Sent: 	Tuesday, May 26, 1998 8:07 PM

To: 	Tesla-2 Mailing List

Subject: 	Re: 140vac input

At 08:48 PM 5/26/98 -0700, you wrote:

>Roy Minnich wrote:

>>

>> Hi,

>>

>> What happens when you run your 12kv 30mA NST on the 140 vac output of a 10 amp variac?

>>

>> If you put 120 vac in and get 12000 vac out you must have a 100 to 1

turn ratio. Right?  So then 140 vac x 100 = 14000 vac?  What happens to the mA?

>>

>> Thanks,

>> Roy Minnich

>>

>Roy,

>

>I'm no expert with NSTs. But, since they are current limiting by design,

>I suppose the core temperature will rise due to saturation. The output

>current will remain about the same, 30ma. The extra energy applied to

>the primary will simply be wasted in the core as heat, which could cause

>failure.

>--

>

>Regards,

>Scott MacGregor EES,Co.

Scott, Roy and others,

I run 140VAC input to my neons all the time with no ill

effect. Roy is correct that you will be getting 14kv output

from a 12kv neon with 140 VAC input. The current will also

rise past the name plate rating. Most of my 30ma neons will

put out 34-38 ma with 140 in. Core saturation or heating does

not seem to be any problem.

However remember you are working your neon harder. More

voltage means more wear and tear on the insulation.

Also be sure you take into consideration that excess

voltage when building your caps.

TTYL!

Mike Hammer

mhammer@misslink.net

From: "Thomas McGahee" <tom_mcgahee@sigmais.com>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: 140vac input

Date: Wed, 27 May 1998 15:00:13 -0400

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Roy,

The 30 ma rating is not meant to be exact. +/- 10% would be considered

perfectly OK. These are *not* precision devices!

Hope this helps.

Fr. Tom McGahee

----------

From: Roy Minnich <rminnich@rolling-meadows.lib.il.us>

To: Tesla-2 Mailing List <tesla-2@emachine.com>

Subject: RE: 140vac input

Date: Wednesday, May 27, 1998 2:18 PM

Hi,

I ran the amp test on my neon (12kv 30mA) and got 29.5mA.  I guess it's a

little out of spec.  I got this neon from Tedd Payne and it had no plate so it

must have been a factory reject due to the slightly low output..... I guess.

Is this the way it goes?  I mean you'd call a neon that puts out 29.xxmA a 30mA?

Roy Minnich

Date: Wed, 27 May 1998 12:05:43 -0700

From: Gene Waldo <gwaldo@1stnetusa.com>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Organization: S.A.P.I.E.N.T. (Surly, Agressive, Politically Incorrect Enemies of Non Thinkers)

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: W-M project

Sender: tesl-err@emachine.com

Bill the arcstarter wrote:

>

> It was written:

> >Cats are quite suseptible to hi voltage effects and can make

> >interesting displays.

>

> Excellent!  I *knew* there had to be practical use for these things

> (both cats AND Tesla coils).

> I'm confident the key to our sucessful coil was the use of a friend's

> cat as a dust brush - used to to de-lint the inside of the 4.25 inch

> secondary prior to applying the polyurethane coatings.  Of course - then

> I had to de-fur the PVC...

Bill, Did you try moistening the cat?

Cats moistened with sulpher dioxide (I think it was..) tend to move more

quickly thru cores.... or what ever. Both cats and SO2 tend to vanish

w/o a trace. Maybe some torn-up lawn...

We should probably go off-net; I have more cat stories.... not

necessarily Tesla-related.

-------------------------------------------

Gene Waldo -  gwaldo@1stnetusa.com  - "Igor in Residence"

---------------------------------------------------------------

What's good for the planet is more OZONE! See the machinery form up!

NEW PAGE!   http://members.1stnetusa.com/danmurphy/waldo.htm

From: "Chris Watkins" <cwatkins@seidata.com>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: T2   Re: 8kV MW oven supply measured

Date: Wed, 27 May 1998 15:08:56 -0400

X-Mailer: IPAD 2.0

X-MimeOLE: Produced By Microsoft MimeOLE V4.72.2106.4

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

-----Original Message-----

>>

>> I think static spark gaps won't easily quench, for the microwave oven

>> transformers are

>> capable to pump even 500 mA. Instead, a rotating spark gap would be forced

>> to quench at my rotating rate.

>>

>> Any suggestion about the spark gap choice?

>>

>

>Marco,

Marco;

You should build a beefed up RQ type gap for the microwave xfrmrs.

I have constructed one for a similar setup that easily handles 500ma at 8-9kv .

Go to a large hardware store and get 13 pieces of copper tubing made

for joining 3/4" copper water pipe.

These are .960" OD. and .875" ID.  12" long and are slightly thicker than

average pipe. I picked through the boxfull to get the straightest ones.

I then placed these inside a piece of 6" sewer pipe with .030 spacing

using 8-32 brass screws. this is rather difficult to determine the hole

spacing on the pvc pipe. I had to scrap the first pipe but I found the

right spacing to use on the next one.

using SDR 35 PVC pipe my spacing came out to 1.266" I used a

pair of dial calipers set at this # to scribe the c to c distance .

this  distance will vary with the pipe dia.

with this setup I can run 2 parallel gaps of .180" or 1 gap of .360" .

the pvc pipe is about 2 ft. long and a fan from an old microwave is

installed at one end pulling air thru the gap.

I have yet to get this gap even warm , it even works well without the

fan for about 30 seconds.

hope this helps

safe coiling!!

questions,comments

Chris W

cwatkins@seidata.com

X-Originating-IP: [199.217.140.22]

From: "Bill the arcstarter" <arcstarter@hotmail.com>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Another cap question

Date: Wed, 27 May 1998 12:13:06 PDT

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

It was written:

>From dsurfr@aol.com Tue May 26 17:32:23 1998

>Can someone explain why rolled poly caps are any better than stacked plate?

>It seems like you can adjust the value of the stacked plate caps a little easier

>& they sound like they would be a little easier to construct.

I haven't tried building a rolled cap yet so I can't comment on that

particular style.  It does seem that the rolled cap construction

utilizes a higher percentage of the raw material (since less material is

req'd for insulating the edges) as opposed to the plate construction

style.

All of our caps consist of 48 sheets of 0.060x8x12 polyethylene sheet

which I purchased at a local plastics outlet.  We've been running four

of these caps (in series-parallel) at 15 KV (powered by a pole pig) for

about half a year without any failures.

Each cap is assembled using aluminum foil plates which we cut on a

poseterboard shear (we rounded the edges a bit using scissors).  The

poly pack is held together using nylon tie-wraps, and is under oil in a

tupperware box.

Two bolts emerge from either end of the top of the tupperware box.  I've

installed neutral bonding blocks onto these bolts for easy electrical

connections to the rest of the coil.  You can get neutral bonding blocks

at a large hardware store - they are used for connecting all the neutral

lines together inside of a breaker box.

> I think I understand the concept of building higher rated caps & connecting

>them in series to lower their value but allow higher voltage load. Is there

>some way to calculate these benefits or is it just a matter of cost? Also is

>there any benefit to connecting them in series & then in parallel?

Each of our caps measures 0.026 uf.  We have two identical strings in

parallel with each other.  Each string consists of two caps in series.

The net capacitance of this arrangement is still 0.026 uf.  We arranged

the caps this way to lower the peak voltage applied to each unit.  We

also experimented with various combinations of series/parallel until we

found the setting which allowed us the best tuning performance.

Next time I make one of these flat poly stacked caps - I'll arrange each

box as having two capacitors in it.  Each cap will share a common

ground, and each other lead will protrude from the top of the tupperware

box.  Thus the box will have three electrical connections - the common,

hot1 and hot2.  Cap #1 will be half the value of cap #2.  This allows me

to select one of three different capacitances depending on how I wire

the external connections (c1, c2, or c1 parallel'd with c2).  All the

"even" numbered plates will be connected to the common connection, 1/3

the remaining plates will be connected to the C1 hot post, and the rest

to the C2 hot post etc.

More later.

-Bill the arcstarter

Starting arcs in Cinci, OH

http://www.geocities.com/CapeCanaveral/Hangar/6160

X-Originating-IP: [199.217.140.22]

From: "Bill the arcstarter" <arcstarter@hotmail.com>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Poly plates for sale.

Date: Wed, 27 May 1998 13:13:41 PDT

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

Coilers,

If anyone is having a hard time finding poly plates for capacitors,

etc, you might find this interesting.

I will supply coilers on this list 48 8x12x0.060 inch polyethylene

plates for $25+shipping/handling.  Other similar shapes and thicknesses

are available (0.030, 0.090 etc) for proportionately the same price (ask

for details).

This is enough to make a 0.026 uf cap rated at 8-10 KV.  Or two 0.013

units which could be placed in series to form a 0.007 uf 16-20 KV

package.

(I have five caps built using the above material and 6x10 aluminum foil.

The area of foil overlap was 6x8 inches.)

Basically I have access to an automatic shear back in the machine shop.

This poly is brand new material - virgin opaque white with no sand/

rocks/ bubbles or other trash inside it (unlike the construction grades

of poly).  I'll buy a 4x8 foot slab from the local plastic distributor

and cut it to order.

To build a capacitor, you'd have to provide your own aluminum foil

plates, tupperware box and oil.

E-mail me if interested.  I won't get rich doing this but I realize some

coilers have a very hard time finding raw materials.

If there's enoguh interest I could have a die (for cutting out the

foils) constructed.  This would allow me to sell the foil plates as

well!

-Bill the arcstarter

Starting arcs in Cinci, OH

http://www.geocities.com/CapeCanaveral/Hangar/6160

From: Roy Minnich <rminnich@rolling-meadows.lib.il.us>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: RE: 140vac input

Date: Wed, 27 May 1998 16:12:57 -0500

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

Ah, good I was hoping it wasn't rocket science!

Roy Minnich

----------

From: 	Thomas McGahee[SMTP:tom_mcgahee@sigmais.com]

Reply To: 	Tesla-2 Mailing List

Sent: 	Wednesday, May 27, 1998 2:00 PM

To: 	Tesla-2 Mailing List

Subject: 	Re: 140vac input

Roy,

The 30 ma rating is not meant to be exact. +/- 10% would be considered

perfectly OK. These are *not* precision devices!

Hope this helps.

Fr. Tom McGahee

----------

From: Roy Minnich <rminnich@rolling-meadows.lib.il.us>

To: Tesla-2 Mailing List <tesla-2@emachine.com>

Subject: RE: 140vac input

Date: Wednesday, May 27, 1998 2:18 PM

Hi,

I ran the amp test on my neon (12kv 30mA) and got 29.5mA.  I guess it's a

little out of spec.  I got this neon from Tedd Payne and it had no plate so it

must have been a factory reject due to the slightly low output..... I guess.

Is this the way it goes?  I mean you'd call a neon that puts out 29.xxmA a

30mA?

Roy Minnich

X-Mailer: IPAD 2.0

Date: Thu, 28 May 1998 02:02:01 -0700

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: bob golding <yubba@clara.net>

Subject: Re: Poly plates for sale.

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

At 13:13 27/05/98 PDT, you wrote:

>Coilers,

>This poly is brand new material - virgin opaque white with no sand/

>rocks/ bubbles or other trash inside it (unlike the construction grades

>of poly).  I'll buy a 4x8 foot slab from the local plastic distributor

>and cut it to order.

>

>To build a capacitor, you'd have to provide your own aluminum foil

>plates, tupperware box and oil.

>

>E-mail me if interested.  I won't get rich doing this but I realize some

>coilers have a very hard time finding raw materials.

>

>If there's enough interest I could have a die (for cutting out the

>foils) constructed.  This would allow me to sell the foil plates as

>well!

hi bill

I know it isn't economical to send your sheets across the pond so i am

trying to find a UK supplier as I am sure we can get it over here. What do

you ask for? many thanks.

bob golding lightning without rain

bob yubba@clara.net

From: <Dsurfr@aol.com>

Date: Wed, 27 May 1998 19:57:04 EDT

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Cap Breakdown Voltage

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Hi All,

I saved an old posting from Fr. Tom (I do that with most of his responses)

back in March about using polypropelene vs. polyethylene for caps in which he

said to calculate the "breakdown" voltage for a .01mfd cap of each to

determine which was the least expensive vs. better cap to build. Can anyone

tell me how I calculate this?

Thanks again ,

Jim

X-Originating-IP: [208.16.144.100]

From: "Bill the arcstarter" <arcstarter@hotmail.com>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Poly plates for sale.

Date: Wed, 27 May 1998 19:12:31 PDT

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

Bob Golding wrote:

>hi bill

>I know it isn't economical to send your sheets across the pond so I am

>trying to find a UK supplier as I am sure we can get it over here. What do

>you ask for? many thanks.

Around here all you do is call up the local plastics distributors and

ask for a 4x8 foot of polyethylene steet, 0.060 inch thick, or whatever

you want.  If you get stock <0.090 it will come as a roll about a foot

in diameter, which means I can drive the car instead of the truck...

McMaster-Carr is another good source for all sorts of strange stuff, if

you can con them out of a catalog... :(

This is the sort of industrial plastics house which supplies lexan,

nylon, teflon, tygon tubing, delrin etc for industry and machine shops,

etc.  Most of these houses will also have a scrap bin - containing

cutoffs, end of runs, etc, which they sell by the pound.  A coiler's

dream!

The shop actually gave me a databook on their products.  I can shop at

home and call for quotes on the interesting stuff!

I'm not sure if perhaps the UK has a different term for polyethylene or

not.  My particular outlet has both LDPE and HDPE, which I'm told are

electrically identical.

-Bill

Date: Wed, 27 May 1998 20:50:21 -0700

From: Gene Waldo <gwaldo@1stnetusa.com>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Organization: S.A.P.I.E.N.T. (Surly, Agressive, Politically Incorrect Enemies of Non Thinkers)

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: W-M project

Sender: tesl-err@emachine.com

Jim Ryan wrote:

>

> At 03:43 PM 5/27/98 -0700, you wrote:

> >

> >Yes! Nice looking work! The most daunting part looming on our future is

> >winding a decent primary!

> >Has Dan been helping you? I think I see some of his tools in the

> >background...

> >Glad for any help we may have been!  Send more progress pics! What camera

> >are you using?

>

> Gene, I have been watching the posts on this group. Your pictures

> and others have helped me work out a lot of the details. I still have

> some questions on rolled caps. I plan to use .005 brass shim stock

> 6" X 75" and layers of poly that =40 mils to make 2 .0066 mf caps for

> my 12KV 30ma. trans. How much of a vacuums do I need to pull the air

> out of the cap and back fill with mineral oil?

>

> The camera is a Kodak DC-40 stores 48 pictures. Batteries last about

> 600 pictures.

>

> Jim

I nor Dan are yet qualified on rolled or plate caps! From my reads, a

rolled or plate cap would req. all the vacuum you can muster! My sense

is also that once evacuated, begin a SLOW admission of oil; perhaps

jointly with a low dc voltage, cyclically (on, off, on...) applied to

the cap as filling takes place, to employ the physical "pumping" action

which this would produce, to work out more of the air.  There were once

aspirator type vacuum 'pumps' which ran off a hose bib-faucet; produced

decent vacuums (29-30 in Hg), quite inexpensively.  Be sure to employ a

water trap and competent valving!

I would play with an empty vacuum-cap container, before the real run!

Make certain you'll not collapse the container or admit water!  If

cylindrical, for rolled, collapse is almost not a problem. Boxy

containers will implode or crush w/o a moments hesitation!

My camera is SONY MV-7, image direct to floppy. Only VGA, tho. {:-(

-------------------------------------

Gene Waldo -  gwaldo@1stnetusa.com  - "Igor in Residence"

---------------------------------------------------------------

What's good for the planet is more OZONE! See the machinery form up!

NEW PAGE!   http://members.1stnetusa.com/danmurphy/waldo.htm

From: "Thomas McGahee" <tom_mcgahee@sigmais.com>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Cap Breakdown Voltage

Date: Thu, 28 May 1998 09:12:02 -0400

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Jim,

Don't waste your time. Both polypropelene and polyethylene make

excellent capacitors. For all practical purposes, they are about

the same. The choice to use one or the other actually boils

down to a) what is readily available to *you*, and b) if both

are available, which is cheaper.

Polypropylene is about like HDPE. If making a rolled style

capacitor, stick with LDPE as it is much easier to roll.

Follow the advice of hundreds of posts on this List, and use

multiple thin layers rather than just one thick layer. You

should connect several caps in series to get a high voltage

rating, rather than trying to make a single cap do all the

work. Single caps are OK for 9KV neon transformers, but

above that voltage rating you want to go with multiple series

caps to hold down the nasty effects of corona.

Remember that most failures occur because of arcing around

the poly edges and especially the ends where the foil exits.

I allow 1" margin per 9KV (neon rating) for the sides, and

I double this to 2" per 9KV at the ends where the foil exits.

It is better to be safe and have your caps live long!

Hope this helps.

Fr. Tom McGahee

----------

> From: Dsurfr@aol.com

> To: Tesla-2 Mailing List <tesla-2@emachine.com>

> Subject: Cap Breakdown Voltage

> Date: Wednesday, May 27, 1998 7:57 PM

>

> Hi All,

> I saved an old posting from Fr. Tom (I do that with most of his responses)

> back in March about using polypropelene vs. polyethylene for caps in which he

> said to calculate the "breakdown" voltage for a .01mfd cap of each to

> determine which was the least expensive vs. better cap to build. Can anyone

> tell me how I calculate this?   Thanks again ,   Jim

Date: Thu, 28 May 1998 08:42:08 -0500

From: RODERICK MAXWELL <tank@magnolia.net>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: wood frame

Sender: tesl-err@emachine.com

Daniel Lockhart wrote:

>

> mike nicholas wrote:

>

> >I built my primary out of wood, it has two heavy coats of oil based paint, I

> >know plastic is better. Will wood be ok?

>

> I'm definitely no expert, but on my coil, I wanted the frame of the primary

> black to match a black box I was mounting it on. I gave the hardwood frame

> a coat of black paint and then sanded off most of the paint on the surface

> of the wood after it dried. This only left the paint that had soaked into

> the wood plus a little on the surface. Then I varnished it heavily with

> good poly varnish. The result was a little splotchy, but looked pretty

> good.

>

> My 2 cents,

>

> Daniel Lockhart

On the first coil that I built the the framework of the primary is

also painted black. But I also glued acrylic plastic strips on the tops

of the supports. On top of this I mounted 3/4" diameter acrylic tubing

cut to a length of 3/4". This served as hold downs, and also provided

extra insulation from the painted wood. The black paint contains carbon

black, which I believe is a very good conductor. The only reason I

painted it at all was for looks, and to keep moisture out of the wood.

Frankensteins Helper

Max

Date: Thu, 28 May 1998 09:13:52 -0600

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Daniel Lockhart <molly@cmn.net>

Subject: Secondary Diameter/Transformer Output Relationships

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

Hey all,

I'm going to be taking a long car trip soon. There are three drivers, so

when going across Kansas and Texas I plan on doing some calculations for

various sized coils. After getting my three-incher working great (16

inches) I can only think of one thing -- bigger!

My question is: Are there any basic rules to follow for matching neon

transformer output and the coil size? I would imagine a coil with a 10.5

-inch diameter secondary powered by a 9 Kv/30ma would be *slightly*

underpowered and I would think if you hooked up a pole pig to a 2.5-inch

coil something would go disastrously wrong.

I remember Mike Hammer and fleischwulf saying how they couldn't seem to

squeeze much more out of their four inch systems. Suppose Mike, you

replaced your four inch secondary with larger, and larger diameter

secondaries and kept everything else the same (assuming they would fit in

your primary). How long would it take 'till the sparks you were getting

started looking scrawny compared to the coil size?

I was thinking of doing calculations for secondaries in the range of six to

eight inches in diameter resonating at about 100 Khz to 175 kHz powered

with about 60 to 120 ma of current at 12 kV. Does this sound about right?

Sorry if I don't quite make sense. I had a hard time figuring out what I

was asking myself!

Daniel Lockhart

X-Mailer: IPAD 2.0

Date: Thu, 28 May 1998 11:25:52 -0400

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Jim Ryan <jryan@exis.net>

Subject: Re: Cap Breakdown Voltage

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

At 09:12 AM 5/28/98 -0400, you wrote:

>Jim,

>Don't waste your time. Both polypropelene and polyethylene make

>excellent capacitors. For all practical purposes, they are about

>the same. The choice to use one or the other actually boils

>down to a) what is readily available to *you*, and b) if both

>are available, which is cheaper.

>

>Snip Snip

Tom, thanks for the info. I plan to roll two caps. for my 12KV 30ma

transformer. Mike Hammer suggested  a total of .0066uf. I bought

4 rolls of 6" X 100" .005 brass shim stock at what i considered

a good price. Mike calculated that I need 450 sq inches for each

cap. so I plan to use 2 75 inch strips of the 6" shim stock in each cap.

I looked an my roll of poly and it has a LDPE label at the bottom

of a recycle symbol with a 4 inside of it. I plan to use 8 layers

of the .006 poly in between the shim stock. I'm still thinking

about the best way to make exit connections out of the assembly.

Everything else is finished on the coil. I'm waiting on my 10 amp

variac.

Jim

X-Mailer: IPAD 2.0

Date: Thu, 28 May 1998 11:23:24 -0500

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: wood frame

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

At 12:00 PM 5/28/98 -0700, you wrote:

>I built my primary out of wood, it has two heavy coats of oil based paint, I

>know plastic is better. Will wood be ok?

>

Mike,

I would think it will be fine. However I would have

left the wood bare rather than painting it. Many

paint pigments contain metallic compounds. It will

probably be OK but in the future leave wood surfaces

unpainted or coat them with polyurethane varnish.

It would also be wise to avoid nails or screws. Glue

is much less conductive than a nail.

For the record my primary form is made from unpainted

unvarnished wood and have had no problems.

TTYL!

Mike Hammer

mhammer@misslink.net

X-Mailer: IPAD 2.0

Date: Thu, 28 May 1998 12:29:59 -0400

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Jim Ryan <jryan@exis.net>

Subject: More help

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Hi all again, I lost some email Mike H. sent me and didn't make a hard copy.

I'm making a coil with a 12KV 30ma transformer. Mike indicated I needed

a total cap of .0066uf. He suggested 2 .0033uf The question is how many

sq inches of plate material do I need for each cap with .048 poly spacing?

Thanks

Jim

Date: Thu, 28 May 1998 10:33:41 -0600

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Daniel Lockhart <molly@cmn.net>

Subject: Re: wood frame

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

mike nicholas wrote:

>I built my primary out of wood, it has two heavy coats of oil based paint, I

>know plastic is better. Will wood be ok?

I'm definitely no expert, but on my coil, I wanted the frame of the primary

black to match a black box I was mounting it on. I gave the hardwood frame

a coat of black paint and then sanded off most of the paint on the surface

of the wood after it dried. This only left the paint that had soaked into

the wood plus a little on the surface. Then I varnished it heavily with

good poly varnish. The result was a little splotchy, but looked pretty

good.

My 2 cents,

Daniel Lockhart

X-Mailer: IPAD 2.0

Date: Thu, 28 May 1998 11:33:44 -0500

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: Poly plates for sale.

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

At 01:13 PM 5/27/98 PDT, you wrote:

>Coilers,

> If anyone is having a hard time finding poly plates for capacitors,

>etc, you might find this interesting.

>

>I will supply coilers on this list 48 8x12x0.060 inch polyethylene

>plates for $25+shipping/handling.  Other similar shapes and thicknesses

>are available (0.030, 0.090 etc) for proportionately the same price (ask

>for details).

>

>This is enough to make a 0.026 uf cap rated at 8-10 KV.  Or two 0.013

>units which could be placed in series to form a 0.007 uf 16-20 KV

>package.

>

>(I have five caps built using the above material and 6x10 aluminum foil.

>The area of foil overlap was 6x8 inches.)

>

>Basically I have access to an automatic shear back in the machine shop.

>This poly is brand new material - virgin opaque white with no sand/

>rocks/ bubbles or other trash inside it (unlike the construction grades

>of poly).  I'll buy a 4x8 foot slab from the local plastic distributor

>and cut it to order.

>

>To build a capacitor, you'd have to provide your own aluminum foil

>plates, tupperware box and oil.

>

>E-mail me if interested.  I won't get rich doing this but I realize some

>coilers have a very hard time finding raw materials.

>

>If there's enoguh interest I could have a die (for cutting out the

>foils) constructed.  This would allow me to sell the foil plates as

>well!

>

>-Bill the arcstarter

>Starting arcs in Cinci, OH

>http://www.geocities.com/CapeCanaveral/Hangar/6160

Hey Bill.

Why stop there? If you are going to cut the poly squares

and the plates just go ahead and stack em up. Then you might

as well get a case for it and fill it with oil. And if you

could break it in and then deliver it that would be great.  :)

TTYL!

Mike Hammer

mhammer@misslink.net

X-Originating-IP: [199.217.140.22]

From: "Bill the arcstarter" <arcstarter@hotmail.com>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Poly plates for sale.

Date: Thu, 28 May 1998 11:00:11 PDT

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

Mike wrote:

>Hey Bill.

>Why stop there? If you are going to cut the poly squares

>and the plates just go ahead and stack em up. Then you might

>as well get a case for it and fill it with oil. And if you

>could break it in and then deliver it that would be great.  :)

Sure!  But there'd be a *slight* extra fee for that...

I thought you'd have some comments about the working voltage of my

caps...?

-Bill :)

Date: Thu, 28 May 1998 14:32:15 -0400

From: Aric_C_Rothman@email.whirlpool.com (Aric C Rothman)

Subject: Polypropylene secondary?

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

Would there be sufficient additional benefit to using a polypropylene secondary

coil form instead of the more conventional materials ( sonotube, PVC, etc... )

to justify a cost of $7/ft. at 6" dia. for a two foot section?  Lower loss?

Hydrophobic?

Aric

Date: Thu, 28 May 1998 12:00:39 -0700

From: mike nicholas <705728@ican.net>

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: wood frame

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

I built my primary out of wood, it has two heavy coats of oil based paint, I

know plastic is better. Will wood be ok?

X-Mailer: IPAD 2.0

Date: Thu, 28 May 1998 15:17:48 -0500

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: Polypropylene secondary?

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

At 02:32 PM 5/28/98 -0400, you wrote:

>Would there be sufficient additional benefit to using a polypropylene secondary

>coil form instead of the more conventional materials ( sonotube, PVC, etc... )

>to justify a cost of $7/ft. at 6" dia. for a two foot section?

Polypropylene will make an excellant secondary form. As to whether

its worth the additional cost is debatable. It will defintely

result in a higher Q lower loss secondary. Whether or not that

results in more output depends greatly on the rest of your coil

system. High loss components like caps and gaps can often mask

improvements in other areas.

My .02 is to put the effort and the money into the best caps and gaps

you can possibly produce. This will pay the biggest dividends.

If you can get polypropylene or polyethylene pipe cheaply

then by all means go for it.

By the way you will want more than 2 feet for a 6 inch diameter

secondary. Winding length should be around 26 inches with AWG 22.

If you leave a 2 inch margin on each end of the winding that means

a 30 inch long form.

>Hydrophobic?

I doubt it would be hydrophobic. Hydrophobia is rabies.

Usually confined to canines, raccoons and skunks. :)

I think you meant hygroscopic which means it tends to draw moisture.

Suddenly Im thirsty.

TTYL!

Mike Hammer

mhammer@misslink.net

From: Roy Minnich <rminnich@rolling-meadows.lib.il.us>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: RE: More help

Date: Thu, 28 May 1998 15:20:57 -0500

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

Hi Jim,

It might be better if you hooked them in series to get a higher voltage rating rather than parallel.  In parallel you would use the two .033 uf caps to get the .066 uf to match your neon and both caps have to be rated for the max kv you system will experience.  If you build two .01uf caps and hook them in series you will get a final value of .006 uf AND a higher voltage combined rating for the two caps in series.  You can use less dielectric in each .01 cap too as the voltage is split between the two caps in a series configuration.

The formula for determining the value of a cap you want to build is ((.224 x K x A) / d) x 2   for rolled poly/plate caps and ((.224 x K x A) / d) x # plates - 1 for flat plate caps.  The value is in pf.

For poly/oil caps use 2 as the dielectric strength for K

A is the area of one plate.  d is the thickness of the poly layers between two plates.

So for rolled poly/foil caps with plates 12 in. x 29.5 in. A = 354 square inches

d = .048 in.

So.....   .224 x 2 x 354 = 158.592    158.592 / .048 = 3304    3304 x 2 = 6608 pf

Do this to get uf      .000001 x 6608 = .006608 uf

To run caps in series you use the formula 1/ ( 1/C1 + 1/C2 etc.) to find the final value.  You can see if you are using two caps you need  values twice the final desired value for each series cap.  Two .013 uf caps in series will have a final value of .0065 uf  which is pretty close.

You will also notice from the cap value equation above that if you halve the dielectric thickness you double the caps value.  ((.224 x 2 x 354)/ .024) = 6608 x 2 = 13216 pf  x .000001 = .013216 uf    Also since each cap in 2 cap series needs only half the KV rating you can use .024 in. of poly in each cap.  This makes the caps easier to build with less material in each.

The plates for the above cap to match your 12kv 30mA neon are 354 square inches made from 12 in. wide x 29.5 in. long foil plates.  If you use 10 in. wide foil the plates have to be 354/10 = 35.4 inches long.

Hope this helps,

Roy Minnich

----------

From: 	Jim Ryan[SMTP:jryan@exis.net]

Reply To: 	Tesla-2 Mailing List

Sent: 	Thursday, May 28, 1998 11:29 AM

To: 	Tesla-2 Mailing List

Subject: 	More help

Hi all again, I lost some email Mike H. sent me and didn't make a hard copy.

I'm making a coil with a 12KV 30ma transformer. Mike indicated I needed

a total cap of .0066uf. He suggested 2 .0033uf The question is how many

sq inches of plate material do I need for each cap with .048 poly spacing?

Thanks

Jim

From: <Dsurfr@aol.com>

Date: Thu, 28 May 1998 17:51:22 EDT

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: More help

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Jim, You didn't mention if you're making a rolled or stacked plate cap. Either

way try the "Tesla Coil Madness" web page. He has a good Java based calculator

there that will help you work out the size needed for either of them.

http://www.ticnet.com/jgore/tesla/tesla.htm

Jim

Date: Thu, 28 May 1998 15:22:02 -0700

From: Gene Waldo <gwaldo@1stnetusa.com>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Organization: S.A.P.I.E.N.T. (Surly, Agressive, Politically Incorrect Enemies of Non Thinkers)

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Polypropylene secondary?

Sender: tesl-err@emachine.com

Mike Hammer wrote:

>

> At 02:32 PM 5/28/98 -0400, you wrote:

> >Would there be sufficient additional benefit to using a polypropylene secondary

> >coil form instead of the more conventional materials ( sonotube, PVC, etc... )

> >to justify a cost of $7/ft. at 6" dia. for a two foot section?

>

> Polypropylene will make an excellant secondary form. As to whether

> its worth the additional cost is debatable. It will defintely

> result in a higher Q lower loss secondary. Whether or not that

> results in more output depends greatly on the rest of your coil

> system. High loss components like caps and gaps can often mask

> improvements in other areas.

>

> My .02 is to put the effort and the money into the best caps and gaps

> you can possibly produce. This will pay the biggest dividends.

>

> If you can get polypropylene or polyethylene pipe cheaply

> then by all means go for it.

>

> By the way you will want more than 2 feet for a 6 inch diameter

> secondary. Winding length should be around 26 inches with AWG 22.

> If you leave a 2 inch margin on each end of the winding that means

> a 30 inch long form.

>

>  >Hydrophobic?

>

> I doubt it would be hydrophobic. Hydrophobia is rabies.

> Usually confined to canines, raccoons and skunks. :)

>

> I think you meant hygroscopic which means it tends to draw moisture.

>

> Suddenly Im thirsty.

>

> TTYL!

>

> Mike Hammer

> mhammer@misslink.net

Mike,

He GOTCHA! Hydrophobic: Chem term: Low afinity for water!  Among the

criters, hydrophobia is the term relating to a supposed fear of water

which they may display while displaying symptoms of the disease.

I agree about 'thirsty', but I think I used the last ice cube last nite!

But I don't like a lot of water in my drinks, any way! ~~<[:-)

Back on topic: How does Plexiglass rate as 2ndary form?

--------------------------------------------------

Gene Waldo -  gwaldo@1stnetusa.com  - "Igor in Residence"

---------------------------------------------------------------

What's good for the planet is more OZONE! See the machinery form up!

NEW PAGE!   http://members.1stnetusa.com/danmurphy/waldo.htm

Date: Thu, 28 May 1998 20:18:11 -0500

From: Bert Hickman <bert.hickman@aquila.com>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Organization: Stoneridge Engineering

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Secondary Diameter/Transformer Output Relationships

Sender: tesl-err@emachine.com

Daniel,

Excellent question! Your suspicians are indeed correct. Larger coils

take more power to drive - a 10" coil will require at least 12 KV or 15

KV at 120 MA to drive to its potential (slight pun intended...). This

really has to do with optimal toroid size, coil self-C, and energy/bang

required to achieve breakout. As the topload and self-C increase, it

will take more energy per "bang" to achieve breakout. This means using a

larger tank cap and more average power from a larger HV power source to

properly "fire" it.

You can also overload small coil - this typically shows up as

overvolting - turn-turn flashovers and heating of the bottom-most

portion of the secondary as well as "hitting a wall" on output

performance past a certain power level.

-- Bert --

Daniel Lockhart wrote:

>

> Hey all,

>

> I'm going to be taking a long car trip soon. There are three drivers, so

> when going across Kansas and Texas I plan on doing some calculations for

> various sized coils. After getting my three-incher working great (16

> inches) I can only think of one thing -- bigger!

>

> My question is: Are there any basic rules to follow for matching neon

> transformer output and the coil size? I would imagine a coil with a 10.5

> -inch diameter secondary powered by a 9 Kv/30ma would be *slightly*

> underpowered and I would think if you hooked up a pole pig to a 2.5-inch

> coil something would go disastrously wrong.

>

> I remember Mike Hammer and fleischwulf saying how they couldn't seem to

> squeeze much more out of their four inch systems. Suppose Mike, you

> replaced your four inch secondary with larger, and larger diameter

> secondaries and kept everything else the same (assuming they would fit in

> your primary). How long would it take 'till the sparks you were getting

> started looking scrawny compared to the coil size?

>

> I was thinking of doing calculations for secondaries in the range of six to

> eight inches in diameter resonating at about 100 Khz to 175 kHz powered

> with about 60 to 120 ma of current at 12 kV. Does this sound about right?

>

> Sorry if I don't quite make sense. I had a hard time figuring out what I

> was asking myself!

>

> Daniel Lockhart

Date: Fri, 29 May 1998 00:14:08 -0400 (EDT)

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: fxphoto@centuryinter.net (fxphoto)

Subject: Re: Polypropylene secondary?

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Mike,

I think he meant the other definition " a fear of water"

Perhaps the coil would then be afraid of water!

I didn't realise it also meant rabies!

learn sumpthin new evry day!

Bill Turbett

> >Hydrophobic?

>

>I doubt it would be hydrophobic. Hydrophobia is rabies.

>Usually confined to canines, raccoons and skunks. :)

>

>I think you meant hygroscopic which means it tends to draw moisture.

>

>

>Suddenly Im thirsty.

>

>TTYL!

>

>Mike Hammer

>mhammer@misslink.net

>

>

X-Mailer: IPAD 2.0

Date: Thu, 28 May 1998 23:40:54 -0500

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: Polypropylene secondary?

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

At 12:14 AM 5/29/98 -0400, you wrote:

>Mike,

> I think he meant the other definition " a fear of water"

>Perhaps the coil would then be afraid of water!

>I didn't realise it also meant rabies!

>learn sumpthin new evry day!

>                              Bill Turbett

Yea Im sure thats what he meant. It was a rather

poor attempt at humor.

TTYL!

Mike Hammer

mhammer@misslink.net

Date: Thu, 28 May 1998 22:52:03 -0600

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Daniel Lockhart <molly@cmn.net>

Subject: Primary Design Styles

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

Hi everybody,

I was wondering about the different ways of making the primary coil. It

seems like there are a lot. There are helical primaries, pancake primaries,

elevated "saucer" primaries, and solenoid primaries. To me, there should

only be one primary design that is best.

Helical and Solenoid Primaries

Helical primaries look like the would overcouple easily. They are also

close to the secondary so there is a greater chance of arcing between the

secondary to the primary or from the toroid to the primary. Solenoid

primaries are simaler, but correct these problems by lowering the primary

from the secondary. (correct me if I'm wrong)

If I imagine correctly, these two designs would create an even magnetic

field that was taller than wide that would be parallel to the secondary.

Pancake and Saucer Primaries

Pancake primaries look like they would couple the least, making a wide and

dense magnetic field at the lower end of the secondary, and a weak one on

the upper end of the secondary. Saucer primaries give more coupling by

placing a pancake primary on an inclined plane, making a pancake/helical

hybrid.

Which is best? To me, helical/solenoid and pancake primaries differ

completely. If one is better than the other, then saucer primaries are just

a dumb compromise. (of course, this is the kind I have) I've seen both

styles used in highly successful coils, so I'm kinda at a loss. What am I

not getting? Does the frequency of the coil have anything to do with it? Do

longer (greater amounts of wire, not particularly pipe length) secondaries

need more or less coupling than higher frequency, shorter secondaries? Is

this it?

Daniel Lockhart

Date: Thu, 28 May 1998 22:53:05 -0600

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Daniel Lockhart <molly@cmn.net>

Subject: Re: Secondary Diameter/Transformer Output Relationships

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

Bert wrote regarding my question:

>Daniel,

>

>Excellent question! Your suspicians are indeed correct. Larger coils

>take more power to drive - a 10" coil will require at least 12 KV or 15

>KV at 120 MA to drive to its potential (slight pun intended...). This

>really has to do with optimal toroid size, coil self-C, and energy/bang

>required to achieve breakout. As the topload and self-C increase, it

>will take more energy per "bang" to achieve breakout. This means using a

>larger tank cap and more average power from a larger HV power source to

>properly "fire" it.

>

>You can also overload small coil - this typically shows up as

>overvolting - turn-turn flashovers and heating of the bottom-most

>portion of the secondary as well as "hitting a wall" on output

>performance past a certain power level.

>

>-- Bert --

Ah! Now I comprehend! Thanks Bert! The toroid capacitance has more to it

than I first imagined. Now, for coils that require "more bang" to breakout,

I guess that breakout has a higher potential than breakouts that don't need

so much bang?

Got it,

Daniel Lockhart

X-Mailer: IPAD 2.0

Date: Fri, 29 May 1998 08:06:56 +0300

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Marco Denicolai <marco@vistacom.fi>

Subject: Why brass?

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

My stupid question of the week:

why everywhere is suggested to use brass near primary, spark gap and nearly

every other place? Why steel is not good?

Marco Denicolai

Date: Thu, 28 May 1998 22:25:59 -0700

From: Michael Loftus <loftusm@ix.netcom.com>

Organization: Me

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Here we go!!!

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Hi,

This is my first email on the list.  I built a TC a couple of months ago and quickly

determined that I need help.  I had some success but my caps blew up, I had major

leakage from secondary to primary, and my Neon died!  Some of the TC sites I viewed

seemed to have very strange "explanations" of things.  This list seems a major

improvement. I am real excited!!   So here goes . . .

I have wound a new coil on 4" abs using #22 with 22.5 inches of windings and will coat

it with HV epoxy on Saturday.  I have a 15kv 30ma Neon.  Primary is 1/4" copper tubing

that can be adjusted from flat to vertical.

I have several questions:

1.  How do I calculate the necessary primary capacitance?  I have seen a formula but it

is poorly explained and I can not figure it out.  Any suggestions?

2.  How do I determine the correct size/shape for the top end of the secondary?

3. A good friend of mine is building a TC and is using HV diodes to convert his 10kv AC

to 20kv DC. He says he is getting very good results (I have seen photos and they are

indeed impressive).  He says using DC greatly reduces the stress on the caps.  Is that

reasonable?

4. Why do caps vary so much in size?  I see small ceramic caps that are rated similarly

to much larger non-ceramic caps.  Do they work the same?  Ceramic caps are cheap and

attractive. . .  large oil-filled caps are relatively expensive.

5.  I notice that many TC's incorporate chokes.  Do I need to include them, and if so,

where do I get them?

That is enough for now.  Here in Orange County, CA, obtaiing electronic supplies is

difficult.  The local shops have an incestuous relationship and seem indifferent and

expensive, so I ordered my wire from Mouser.com and other related items from Goldmine.

Both were prompt, friendly, and lower in cost. I obtained my Neons for $25 each from a

local old-fashioned electrical contractor who had a pile in his shop.

Thanks

Mike

Date: Fri, 29 May 1998 06:42:03 -0500

From: Bert Hickman <bert.hickman@aquila.com>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Organization: Stoneridge Engineering

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Primary Design Styles

Sender: tesl-err@emachine.com

Daniel,

Good insights! It turns out that any of the three primary styles can be

made to work effectively, as long as the coupling is adjusted

appropriately. Even a helical coil will work as long as its diameter is

significantly larger than the secondary diameter. The saucer primary

(sometimes called inverse conical) is sort of like a hybrid between the

pancake and helical. The coupling coefficient (the fraction of magnetic

lines that are common to BOTH the primary and secondary) for a 2-coil

system should be in the range of 0.1 - 0.25 for best performance.

Pancake coils are often found to be the best performers for high powered

coils "primarily" because they provide the greatest physical distance to

the toroid. This reduces Electrostatic (E-field) stresses, excessive

secondary:primary corona, and toroid-to-strikerail hits. This

performance could also be achieved with a relatively low helical or

inverse conical primary as well. The key is hitting the right coupling

range and providing sufficient physical clearance to the secondary and

toroid. You may find it interesting that the largest coil built to date

(Greg Lehy's monster 130 KVA system) uses a helical coil that's actually

built inside the base of the secondary! This was driven by artistic

considerations, and presented significant design difficuties for Greg...

Hope this helps!

-- Bert --

Daniel Lockhart wrote:

>

> Hi everybody,

>

> I was wondering about the different ways of making the primary coil. It

> seems like there are a lot. There are helical primaries, pancake primaries,

> elevated "saucer" primaries, and solenoid primaries. To me, there should

> only be one primary design that is best.

>

>         Helical and Solenoid Primaries

>

> Helical primaries look like the would overcouple easily. They are also

> close to the secondary so there is a greater chance of arcing between the

> secondary to the primary or from the toroid to the primary. Solenoid

> primaries are simaler, but correct these problems by lowering the primary

> from the secondary. (correct me if I'm wrong)

>

> If I imagine correctly, these two designs would create an even magnetic

> field that was taller than wide that would be parallel to the secondary.

>

>         Pancake and Saucer Primaries

>

> Pancake primaries look like they would couple the least, making a wide and

> dense magnetic field at the lower end of the secondary, and a weak one on

> the upper end of the secondary. Saucer primaries give more coupling by

> placing a pancake primary on an inclined plane, making a pancake/helical

> hybrid.

>

> Which is best? To me, helical/solenoid and pancake primaries differ

> completely. If one is better than the other, then saucer primaries are just

> a dumb compromise. (of course, this is the kind I have) I've seen both

> styles used in highly successful coils, so I'm kinda at a loss. What am I

> not getting? Does the frequency of the coil have anything to do with it? Do

> longer (greater amounts of wire, not particularly pipe length) secondaries

> need more or less coupling than higher frequency, shorter secondaries? Is

> this it?

>

> Daniel Lockhart

From: Roy Minnich <rminnich@rolling-meadows.lib.il.us>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: RE: Here we go!!!

Date: Fri, 29 May 1998 12:24:42 -0500

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

Hi mike,

Here we go indeed!  I'm about where you are but I've had a good deal of technical help

from generous list members.

First off I recommend you check out Mike Hammer's page at http://misslink.net/mhammer/index.htm  and Gary Weaver's page at http://home.earthlink.net/~gweaver/  as two very good examples of good information that

is out there.  There are many pages with mostly good information but when in doubt ASK SOMEBODY so you don't fry a bunch of neons or get hurt yourself.

Then check out the Tesla Coil Archive at http://www.velocity.net/~djb/files.shtml

Right off I recommend Math.txt to answer your cap sizing questions  then check out Mikecap.txt, Neon.txt, Oiltypes.txt, RFground.txt  and all the Safety files.  All of

the stuff is good but some is a little dated so remember to ask before making a move.

It is possible to build high quality, high voltage pulse discharge caps from common

materials like polyethylene plastic film, aluminum foil, PVC pipe and mineral oil at

a huge savings compared to purchasing commercial models.  It is also possible to rebuild

the neon that fried on you.  You can wind your own safety chokes on two inch PVC and make

a simple safety gap that along with the chokes will help protect your neon from flash-backs in the future.

Download Wintesla.zip from the archive too.  It's a nice Windows based program that calculates things like matching a cap to a neon and the capacity of different top loads

such as spheres and toroids.

In a nut shell it sounds like you had too much coupling between your primary and secondary coils hence the flash-overs.  Caps blow because you didn't have enough KV rating.  You need KV ratings on you caps way higher than the neon output because the voltages produced in the tank circuit (primary, cap(s) and spark gap) are usually twice as high as the neon's rating and can go a lot higher.  If you didn't have a safety gap and shunts you were stressing your neon and likely the caps.

So do some reading, check out the home pages and come on back with your new questions!

Good luck,

Roy Minnich

----------

From: 	Michael Loftus[SMTP:loftusm@ix.netcom.com]

Reply To: 	Tesla-2 Mailing List

Sent: 	Friday, May 29, 1998 12:25 AM

To: 	Tesla-2 Mailing List

Subject: 	Re: Here we go!!!

Hi,

This is my first email on the list.  I built a TC a couple of months ago and quickly

determined that I need help.  I had some success but my caps blew up, I had major

leakage from secondary to primary, and my Neon died!  Some of the TC sites I viewed

seemed to have very strange "explanations" of things.  This list seems a major

improvement. I am real excited!!   So here goes . . .

I have wound a new coil on 4" abs using #22 with 22.5 inches of windings and will coat

it with HV epoxy on Saturday.  I have a 15kv 30ma Neon.  Primary is 1/4" copper tubing

that can be adjusted from flat to vertical.

I have several questions:

1.  How do I calculate the necessary primary capacitance?  I have seen a formula but it

is poorly explained and I can not figure it out.  Any suggestions?

2.  How do I determine the correct size/shape for the top end of the secondary?

3. A good friend of mine is building a TC and is using HV diodes to convert his 10kv AC

to 20kv DC. He says he is getting very good results (I have seen photos and they are

indeed impressive).  He says using DC greatly reduces the stress on the caps.  Is that

reasonable?

4. Why do caps vary so much in size?  I see small ceramic caps that are rated similarly

to much larger non-ceramic caps.  Do they work the same?  Ceramic caps are cheap and

attractive. . .  large oil-filled caps are relatively expensive.

5.  I notice that many TC's incorporate chokes.  Do I need to include them, and if so,

where do I get them?

That is enough for now.  Here in Orange County, CA, obtaiing electronic supplies is

difficult.  The local shops have an incestuous relationship and seem indifferent and

expensive, so I ordered my wire from Mouser.com and other related items from Goldmine.

Both were prompt, friendly, and lower in cost. I obtained my Neons for $25 each from a

local old-fashioned electrical contractor who had a pile in his shop.

Thanks

Mike

Date: Fri, 29 May 1998 10:47:14 -0700

From: Gene Waldo <gwaldo@1stnetusa.com>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Organization: S.A.P.I.E.N.T. (Surly, Agressive, Politically Incorrect Enemies of Non Thinkers)

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

CC: TESLA-2@emachine.com

Subject: problem:post format?

Sender: tesl-err@emachine.com

Roy,

Your and perhaps 1 or 2 other posts always seem to come to my in box

with no 'CRLF's(?) in place....  Posted text is strung out ~3 full

screens wide! Makes for very difficult reading, and my printer looses

all off screen copy!

Is there something I can do ? Or can you locally reset something?

Or, am I the only one seeing this problem?

Perplexed!

--

Gene Waldo -  gwaldo@1stnetusa.com  - "Igor in Residence"

---------------------------------------------------------------

What's good for the planet is more OZONE! See the machinery form up!

NEW PAGE!   http://members.1stnetusa.com/danmurphy/waldo.htm

X-Mailer: IPAD 2.0

Date: Fri, 29 May 1998 13:44:50 -0500

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: problem:post format?

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

At 10:47 AM 5/29/98 -0700, you wrote:

>Roy,

>Your and perhaps 1 or 2 other posts always seem to come to my in box

>with no 'CRLF's(?) in place....  Posted text is strung out ~3 full

>screens wide! Makes for very difficult reading, and my printer looses

>all off screen copy!

>

>Is there something I can do ? Or can you locally reset something?

>Or, am I the only one seeing this problem?

>

>Perplexed!

>

>--

>Gene Waldo -  gwaldo@1stnetusa.com  - "Igor in Residence"

Gene,

Welcome to the wonderful world of non standard standards

where every software company thinks that their way is THE

way things should be done.

Programs like Microsoft Mail and Lotus Notes completely

ignore the set down standards of Internet email instead

imposing their idea of the way things should be.

So there is nothing you can really do about the sometimes

erratic formating. Its a fact of life on the Internet.

Just be thankful that no one on the list is presently

using cc:mail. Now thats truly awful.

Im imagining an Internet where everyone uses Eudora.

Wishful thinking.

TTYL!

Mike Hammer

mhammer@misslink.net

X-Mailer: IPAD 2.0

Date: Fri, 29 May 1998 14:27:29 -0500

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: Here we go!!!

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

At 10:25 PM 5/28/98 -0700, you wrote:

>Hi,

>This is my first email on the list.

Welcome aboard Michael.

>I built a TC a couple of months ago and quickly determined that I need help.

>I had some success but my caps blew up, I had major leakage from secondary

>to primary, and my Neon died!

Sounds like a normal sequence of events for first time coils.

I would bet you had your cap connected in parallel with your

neon and the gap went to the primary coil. This configuration

is hard on neons. Better to have the gap shunted across the neon

and the cap in series with the primary coil.

>Some of the TC sites I viewed seemed to have very strange "explanations"

>of things.  This list seems a major improvement.

The sad truth is that there are quite a few sites out there with

poor, bad and/or just plain wrong information.

We try to provide the best information here that we can.

>So here goes . . .

>

>I have wound a new coil on 4" abs using #22 with 22.5 inches of windings

>and will coat it with HV epoxy on Saturday.  I have a 15kv 30ma Neon.

>Primary is 1/4" copper tubing that can be adjusted from flat to vertical.

>

>I have several questions:

>

>1. How do I calculate the necessary primary capacitance?  I have seen a

>formula but it is poorly explained and I can not figure it out.  Any

>suggestions?

First of you must realize that a Tesla coil is TWO resonant circuits.

The secondary inductance along with distributed + terminal capacitance

forms one resonant circuit. The main cap and primary coil form the

other resonant circuit. Both of these circuits must be tuned to the

same frequency. If they are not the same frequency you have little

or no output.

So with that in mind the proper size for the primary cap is one

that will resonate with the primary coil at the same frequency that

the secondary resonates at. All other considerations are less important.

Now to get the most power transfer from the supply transformer

(neon, OBIT, ETC.) the cap should have an impedance equal to

the impedance of the transformer secondary at 60hz. This gives

us a value to shoot for but remember that this cap must still resonate

with the primary at the secondary resonant frequency.

So you just cant slap a large transformer and correspondingly

large cap on an existing coil and have it work. The large cap

affects the resonant frequency.

To calculate the matching cap for a neon use the following formulas:

First determine your neon sign transformers impedence:

E

Z  =  ---

I

Z = Impedence in ohms

E = volts

I = current in Amps

For example a 12kv/30ma neon.

Z= 12000/.03

Z= 400000 ohms

Now we need to calculate the cap size that has this impedance

at 60hz/

1

C =   -------------------

2 x pi x Z x .00006

C = capacitance in microfarads

Z = Impedence from above calculation

pi = 3.14

.00006 is 60Hz. Use .00005 if your power grid is 50hz.

Continuing our example from above:

C = 1/ 6.28 x 400000 X .00006

C = 1/150.72

C = .0066 mfd

Note this may not be the BEST cap value but makes a good

starting point. Some experimentation is always called for.

>

>2.How do I determine the correct size/shape for the top end of the secondary?

In general larger terminal sizes yield longer sparks. But larger

terminals also require more power for breakout. There are no hard and fast

rules. Its best to start with a smaller terminal and work your way up.

Many beginners make the mistake of using too large a toroid at first

and find their coil almost impossible to tune.

As far as shape a spherical terminal is good but the toroid

is preferred as it helps to suppress corona on the top secondary turns.

>

>3. A good friend of mine is building a TC and is using HV diodes to

>convert his 10kv AC to 20kv DC. He says he is getting very good results

(I have seen photos and they are indeed impressive).  He says using DC

greatly reduces the stress on the caps.  Is that reasonable?

Partly. Whiles its true most caps will hold up better to

DC than AC all bets are off as soon as the gap fires. Then

you have loads of RF energy. RF is AC so you really havnt gained

anything. That combined with the problem of protecting the diodes

from RF make this unattractive.

>

>4. Why do caps vary so much in size?  I see small ceramic caps that are

>rated similarly to much larger non-ceramic caps.  Do they work the same?

>Ceramic caps are cheap and attractive. . .  large oil-filled caps are

relatively expensive.

Capacitors abound in electronics. There are dozens of types.

All have their particular characteristics that make them desirable

for one application or another.

Tesla coil use is about the worst service a capacitor will see.

To be efficient it must have low loss construction and low RF

dissipation. That leaves the vast majority of caps out.

Ceramic caps have high RF loss and will tend to overheat

and self destruct. Polyethylene or polypropylene plastic

film caps are the way to go and filling them with oil suppresses

damaging corona.

>

>5. I notice that many TC's incorporate chokes.  Do I need to include them,

> and if so, where do I get them?

Yes you should have them to help prevent RF energy from the Tesla

coil primary circuit from entering your neon and causing damage.

You make them by winding small guage magnet wire on ferrite toroids

or rods or making air core chokes by winding on PVC pipe. You

need from 2-10 mh of inductance.

You should also include some series resistance to help dampen

any ringing in the chokes and by all means use a safety gap

across the transformer.

TTYL!

Mike Hammer

mhammer@misslink.net

From: Roy Minnich <rminnich@rolling-meadows.lib.il.us>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: cap placement

Date: Fri, 29 May 1998 15:34:07 -0500

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

Hi Mike,

You wrote:

>I would bet you had your cap connected in parallel with your

>neon and the gap went to the primary coil. This configuration

>is hard on neons. Better to have the gap shunted across the neon

>and the cap in series with the primary coil.

I've seen diagrams showing both ways to have the cap wire in the tank circuit.

Why is having the cap in series with the primary easier on the neon?  I get better performance with the cap across the neon and the gap in series with the primary.

Now this is with my small 1.5 in. secondary and a single salt water bottle cap

powered by a 12kv 30mA neon.

Puzzled,

Roy Minnich

Date: Fri, 29 May 1998 16:59:02 -0400

From: Aric_C_Rothman@email.whirlpool.com (Aric C Rothman)

Subject: Re[2]: problem:post format?

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

Just be thankful that no one on the list is presently

^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^

using cc:mail. Now thats truly awful.

^^^^^^^^^^^^^^

Hey!  I resemble that remark!

Aric

X-Mailer: IPAD 2.0

Date: Fri, 29 May 1998 16:12:31 -0500

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: cap placement

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

At 03:34 PM 5/29/98 -0500, you wrote:

>

>Hi Mike,

>

>You wrote:

>

>>I would bet you had your cap connected in parallel with your

>>neon and the gap went to the primary coil. This configuration

>>is hard on neons. Better to have the gap shunted across the neon

>>and the cap in series with the primary coil.

>

>I've seen diagrams showing both ways to have the cap wire in the tank

circuit.  Why is having the cap in series with the primary easier on the

neon?  I get better performance with the cap across the neon and the gap in

series with the primary.  Now this is with my small 1.5 in. secondary and a

single salt water bottle cap powered by a 12kv 30mA neon.

>

>Puzzled,

>

>Roy Minnich

Roy,

Yes many older books show the shunt cap configuration.

It does work but is not a good setup for a neon.

There should be no difference in performance between the

two arrangements.

Remember that the main cap and primary coil form a resonant

circuit. This circuit along with the spark gap form the Tesla

oscillator. This has heavy RF currents flowing in it and high RF

voltages. Any RF appearing on the main cap will also appear on

anything in parallel with it like the secondary of your neon.

Those RF voltages will literally eat away the insulation

in your neon.

By placing the spark gap in parallel with the neon instead

of the cap the neon is now isolated from the RF action taking

place in the tank circuit. When the tank circuit is oscillating

the spark gap is energized. This energized gap places a short

circuit across the neon secondary. This provides some measure

of protection. Its not perfect but every little bit helps.

Combine that with RF chokes and a safety gap and your neon should

be reasonably safe.

TTYL!

Mike Hammer

mhammer@misslink.net

X-Mailer: IPAD 2.0

Date: Fri, 29 May 1998 16:24:16 -0500

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: Re[2]: problem:post format?

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

At 04:59 PM 5/29/98 -0400, you wrote:

>

>Just be thankful that no one on the list is presently

>                      ^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^

>using cc:mail. Now thats truly awful.

>^^^^^^^^^^^^^^

>

>Hey!  I resemble that remark!

>

>Aric

A cc:mail user comes out of the closet! :)

I wont tell anyone if you dont.

TTYL!

Mike Hammer

mhammer@misslink.net

Date: Fri, 29 May 1998 15:04:34 -0700

From: Gene Waldo <gwaldo@1stnetusa.com>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Organization: S.A.P.I.E.N.T. (Surly, Agressive, Politically Incorrect Enemies of Non Thinkers)

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: problem:post format?

Sender: tesl-err@emachine.com

Mike Hammer wrote:

>

> At 10:47 AM 5/29/98 -0700, you wrote:

> >Roy,

> >Your and perhaps 1 or 2 other posts always seem to come to my in box

> >with no 'CRLF's(?) in place....  Posted text is strung out ~3 full

> >screens wide!

> >Perplexed!

> >

> Gene,

> Welcome to the wonderful world of non standard standards

> where every software company thinks that their way is THE

> way things should be done.

>

> Mike Hammer

> mhammer@misslink.net

Mike,

Kinda puts me in mind of Beta video; and I seem to recall a battle over

standards for delivering electrical power to consumers...several years

ago!  Among many othes!

Did Edison go to his grave thinking he was 'right, and beaten unfairly',

or did he get converted, al fin?

---------------------------------------------

Gene Waldo -  gwaldo@1stnetusa.com  - "Igor in Residence"

---------------------------------------------------------------

What's good for the planet is more OZONE! See the machinery form up!

NEW PAGE!   http://members.1stnetusa.com/danmurphy/waldo.htm

Date: Fri, 29 May 1998 15:22:58 -0700

From: Gene Waldo <gwaldo@1stnetusa.com>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Organization: S.A.P.I.E.N.T. (Surly, Agressive, Politically Incorrect Enemies of Non Thinkers)

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Binary posts?

Sender: tesl-err@emachine.com

Mike,

Thanks for being the source for some of the most lucid "keeper-mail'!

Are we any better off now, with our new server, vis-a-vie attached pics,

than we were before?   I note attachments to some posts, which require

seemingly unavailable (to me, at least) 'plug-ins' to see...

-----------------------------------

Gene Waldo -  gwaldo@1stnetusa.com  - "Igor in Residence"

---------------------------------------------------------------

What's good for the planet is more OZONE! See the machinery form up!

NEW PAGE!   http://members.1stnetusa.com/danmurphy/waldo.htm

Date: Fri, 29 May 1998 20:48:47 -0500

From: Bert Hickman <bert.hickman@aquila.com>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Organization: Stoneridge Engineering

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: testing plastics

Sender: tesl-err@emachine.com

bob golding wrote:

>

> hi all

> Does anyone know of a quick and dirty way to test for the different type

> of plastics, p.v.c.,polyethylene,abs.? I have just been given some blue 4"

> water pipe of unknown composition,could be p.v.c.or polythene or a.b.s and

> it would be nice to know what it is. many thanks.

> bob golding lightning without rain

>

> bob yubba@clara.net

Bob,

The simplest way is to try burning a small piece of it. Warning - burn

this outside, and be careful to sample only a very small amount of the

fumes, since in the case of ABS and PVC, they are toxic. In the case of

LDPE/HDPE or PP, the material will float in water. If it's commercial

water pipe it's most likely PVC.

ABS -    burns, but usually extinguishes on removal of flame source

acrid odor, yellow flame with blue edges, possibe dripping,

black smoke with soot in the air.

PVC -    Similar to ABS, except will always self-extinguish

LDPE or

HDPE -   continues to slowly burn with a blue flame with yellow tips,

odor of paraffin, drips molten material. Floats in water.

Polypropylenes - burns with blue flame, sweet odor, drips, and floats in

water. More difficult to scratch than polyethylene.

Acrylics - burns with a blue flame, yellow tips, with a fruity

odor.            Cast material will not drip, molded material will.

Hope this helps!

-- Bert --

Date: Fri, 29 May 1998 21:13:36 -0500

From: Bert Hickman <bert.hickman@aquila.com>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Organization: Stoneridge Engineering

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Why brass?

Sender: tesl-err@emachine.com

Marco Denicolai wrote:

>

> My stupid question of the week:

>

> why everywere is suggested to use brass near primary, spark gap and nearly

> every other place? Why steel is not good?

>

>  Marco Denicolai

Marco,

Actually, the better material is copper as far as the primary coil,

primary wiring, and even the main spark gap. Steel is very poor

electrical conductor compared to copper, especially for RF circuits.

More energy lost in heating the primary circuit means less energy going

into making sparks...

-- Bert --

X-Mailer: IPAD 2.0

Date: Sat, 30 May 1998 02:01:31 -0700

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: bob golding <yubba@clara.net>

Subject: testing plastics

Cc: tesla-2@emachine.com

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

hi all

Does anyone know of a quick and dirty way to test for the different type

of plastics, p.v.c.,polyethylene,abs.? I have just been given some blue 4"

water pipe of unknown composition,could be p.v.c.or polythene or a.b.s and

it would be nice to know what it is. many thanks.

bob golding lightning without rain

bob yubba@clara.net

From: <Dsurfr@aol.com>

Date: Sat, 30 May 1998 14:44:35 EDT

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: decibel production

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

I now understand Mike's warning about wearing some sort of hearing protection.

These coils are loud!! Has anyone done any reasearch on the decibel range of a

sparking coil? I've spent the last hour working on my tuning & my ears are

still ringing.

Jim

Date: Sun, 31 May 1998 10:50:49 -0500

From: RODERICK MAXWELL <tank@magnolia.net>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Primary Design Styles

Sender: tesl-err@emachine.com

Bert Hickman wrote:

>

> Daniel,

>

> Good insights! It turns out that any of the three primary styles can be

> made to work effectively, as long as the coupling is adjusted

> appropriately. Even a helical coil will work as long as its diameter is

> significantly larger than the secondary diameter. The saucer primary

> (sometimes called inverse conical) is sort of like a hybrid between the

> pancake and helical. The coupling coefficient (the fraction of magnetic

> lines that are common to BOTH the primary and secondary) for a 2-coil

> system should be in the range of 0.1 - 0.25 for best performance.

>

> Pancake coils are often found to be the best performers for high powered

> coils "primarily" because they provide the greatest physical distance to

> the toroid. This reduces Electrostatic (E-field) stresses, excessive

> secondary:primary corona, and toroid-to-strikerail hits. This

> performance could also be achieved with a relatively low helical or

> inverse conical primary as well. The key is hitting the right coupling

> range and providing sufficient physical clearance to the secondary and

> toroid. You may find it interesting that the largest coil built to date

> (Greg Lehy's monster 130 KVA system) uses a helical coil that's actually

> built inside the base of the secondary! This was driven by artistic

> considerations, and presented significant design difficuties for Greg...

>

> Hope this helps!

>

> -- Bert --

>

Can anyone tell me the difference between a solenoidal primary and a

helical primary. I know what a solenoid looks like, but what does the

helical coil made.

Frankensteins Helper

Max

Date: Sun, 31 May 1998 23:02:13 -0500

From: Bert Hickman <bert.hickman@aquila.com>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Organization: Stoneridge Engineering

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Primary Design Styles

Sender: tesl-err@emachine.com

RODERICK MAXWELL wrote:

<SNIP>

>     Can anyone tell me the difference between a solenoidal primary and a

> helical primary. I know what a solenoid looks like, but what does the

> helical coil made.

>

>                                 Frankensteins Helper

>                                         Max

Roderick,

A helical primary is really just another name for a solenoid primary.

-- Bert --

Date: Sun, 31 May 1998 11:13:11 -0500

From: RODERICK MAXWELL <tank@magnolia.net>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Primary Design Styles

Sender: tesl-err@emachine.com

Bert Hickman wrote:

>

> RODERICK MAXWELL wrote:

> <SNIP>

> >     Can anyone tell me the difference between a solenoidal primary and a

> > helical primary. I know what a solenoid looks like, but what does the

> > helical coil made.

> >

> >                                 Frankensteins Helper

> >                                         Max

>

> Roderick,

>

> A helical primary is really just another name for a solenoid primary.

>

> -- Bert --

Thanks! That always did bug me!

Frankensteins Helper

Max

Date: Sun, 31 May 1998 20:26:22 -0700

From: Michael Loftus <loftusm@ix.netcom.com>

Organization: Me

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Here we go!!! Part 2 - The Caps.

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Well, that does it.  I have decided to make my own caps rather than purchase them.

Based on the implications of the email on "cap" issues, poly with aluminum/brass and

mineral oil seem popular and effective.  I am curious about the thickness of the poly

layers.  The thinner the poly layers, the greater the capacitance but I would guess the

lower the operating voltage.  The thicker the poly layers, the lower the capacitance but

the operating voltage probably goes up.  There is probably some simple formula to use

when designing the poly layers.  I hear that each insulating layer is best if made-up of

several individual layers rather than one individual layer.  I plan on using a 15kv

Neon.  I have read of layers of poly .025" to .048" thick . . . How do I calculate the

thickness I need?

Mike

P.S.  I spend much of this weekend baking two layers of HV potting epoxy onto my new

4" X 22.5" secondary.  It may not do much electrically, but it sure looks good!

X-Mailer: IPAD 2.0

Date: Mon, 01 Jun 1998 08:35:58 +0300

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Marco Denicolai <marco@vistacom.fi>

Subject: Re: 8kV MW oven supply measured

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

At 01:33 27/05/98 -0600, you wrote:

>    You do seem to be departing a bit from the orthodox lore of

>coilbuilding, so it shall be interesting to see how things work out

>for you--I hope well, because gutted microwaves are relatively

>abundant, so if you could make a coil power supply from one,

>I would be quite happy.

I'll keep you informed.

>

>

>

>> Any suggestion about the spark gap choice?

>>

>

>    Yes, why don't you use the RQ gap (sp?)  It's like a bunch ofcopper pipes

>colinearly aligned with small gaps between them.  You

>can cool it with a fan.  The spark is distributed among many small

>gaps, so heating/quenching issues are not as acute.

>

In my understanding, when the spark gap fires, the HV capacitor and the

transformer re both pumping current through the gap. Now, the capacitor discharges

quite uickly and a NST is capable of feeding max 30 mA of current through the gap.

But my microwave oven can feed 500 mA (!) of current, so I think it will be much

difficult to quench a static gap by just blowing air.

Instead a rotating gap will "separate" the electrodes thus forcing the arc to

terminate. What do you think about that?

Marco Denicolai

X-Mailer: IPAD 2.0

Date: Mon, 01 Jun 1998 08:49:43 +0300

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Marco Denicolai <marco@vistacom.fi>

Subject: Why brass BOLTS?

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

At 21:13 29/05/98 -0500, you wrote:

>Marco Denicolai wrote:

>>

>> My stupid question of the week:

>>

>> why everywere is suggested to use brass near primary, spark gap and nearly

>> every other place? Why steel is not good?

>>

>

>Marco,

>

>Actually, the better material is copper as far as the primary coil,

>primary wiring, and even the main spark gap. Steel is very poor

>electrical conductor compared to copper, especially for RF circuits.

>More energy lost in heating the primary circuit means less energy going

>into making sparks...

>

>-- Bert --

Sorry guys, I forgot to mention that I meant brass bolts and nuts. Why bolts

and nuts must e brass and can't be steel? I mean NEAR primary, spark gap, etc.

even if they are not conducting electricity.

I am asking because this is what reads on many build instructions: use brass

bolts for anything near the primary...

Marco Denicolai

X-Mailer: IPAD 2.0

Date: Mon, 01 Jun 1998 09:11:31 +0300

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Marco Denicolai <marco@vistacom.fi>

Subject: Capacitor discharge

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Because tuning a TC is a sequence of power-on / check / power-off / touch-the-

wires I would like to make sure all capacitors are discharged so that I

don't get any electrical shock (that could be my last one...).

I understand that using an NST and HV cap in series with the primary, the HV

cap will discharge through the primary-chokes-NST which is a pretty low

resistance path.

But I haven't got NST: I use a voltage doubler (the microwave oven design)

and that includes voltage doubling capacitors IN SERIES with the

transformers.

This means that, after power-off, the HV and other capacitors are VERY likely

to be still charged: how to discharge them safely?

I DON'T want to short them every time by hands: also that is a pretty risky

operation if you touch the wrong place (and what if you "forget" doing it?). I

see basically two solutions:

1. A 10 Mohm HV resistor in parallel with each capacitor. This is what is used

in the capacitors employed in microwave ovens (they have got this resistor

INSIDE the cap can). But how that will affect the HV capacitor performance?

2. A kind of delayed-relay battery that will short all caps through, say,

100 Kohm resistors. But then you will have HV wires running through your

TC assembly, which is not very nice. And HV relays will be needed too

(do they exist?).

Any suggestions about this?

Marco Denicolai

X-Mailer: IPAD 2.0

Date: Mon, 01 Jun 1998 00:55:56 -0700

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: bob golding <yubba@clara.net>

Subject: toroids

Cc: tesla-2@emachine.com

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

hi all

I am thinking of making a toroid from moulded fibreglass and getting it

copper plated. can anyone see any problems with this? Incidentally the

error messages are back and I am getting 2 copies of my posts back as well.

gene sorry you got got 4 copies of my post about plastic it wasnt my fault

honest! just another "feature" I suppose. Many thanks

bob golding lightning without rain

bob yubba@clara.net

From: "Thomas McGahee" <tom_mcgahee@sigmais.com>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Capacitor discharge

Date: Mon, 1 Jun 1998 07:55:42 -0400

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

----------

> From: Marco Denicolai <marco@vistacom.fi>

> To: Tesla-2 Mailing List <tesla-2@emachine.com>

> Subject: Capacitor discharge

> Date: Monday, June 01, 1998 2:11 AM

>

>

> 1. A 10 Mohm HV resistor in parallel with each capacitor. This is what is used

> in the capacitors employed in microwave ovens (they have got this resistor

> INSIDE the cap can). But how that will affect the HV capacitor performance?

>

Marco,

Placing a high value resistor across each cap, including the main cap,

is a good idea. Personally, I would place something like 1 Megohm across

each. At 8KV this will bleed charge off at a current draw of about

8 microamps. This will have very negligible effect on the performance

of your TC. Remember that T=RC for the discharge, and you need about 4

or 5 T periods to assure the voltage is low enough not to zap you.

You should wait for whatever turns out to be the LONGEST 5T value.

This will almost certainly be dictated by the size of the doubler

caps. Here you have hundreds of ma of charging current, so you can

afford to use much smaller resistor values than even 1 M. Make sure

you choose a wattage rating that can handle the dissipated power

continuously.

Hope this helps.

Fr. Tom McGahee

From: "Thomas McGahee" <tom_mcgahee@sigmais.com>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Why brass BOLTS?

Date: Mon, 1 Jun 1998 08:00:01 -0400

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

----------

> From: Marco Denicolai <marco@vistacom.fi>

> To: Tesla-2 Mailing List <tesla-2@emachine.com>

> Subject: Why brass BOLTS?

> Date: Monday, June 01, 1998 1:49 AM

>

>

> Sorry guys, I forgot to mention that I meant brass bolts and nuts. Why bolts

> and nuts must be brass and can't be steel? I mean NEAR primary, spark gap, etc.

> even if they are not conducting electricity.

>

> I am asking because this is what reads on many build instructions: use brass

> bolts for anything near the primary...

Marco,

Steel is more magnetic than brass, and, in the presence of the

primary coil, steel bolts and screws and nuts can heat up due to

induction heating. Copper and brass heat up much less, as they

are not ferromagnetic.

Hope this helps.

Fr. Tom McGahee

Date: Mon, 01 Jun 1998 07:01:57 -0500

From: Bert Hickman <bert.hickman@aquila.com>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Organization: Stoneridge Engineering

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Capacitor discharge

Sender: tesl-err@emachine.com

Marco Denicolai wrote:

>

> Because tuning a TC is a sequence of power-on / check / power-off / touch-the-

> wires I would like to make sure all capacitors are discharged so that I

> don't get any electrical shock (that could be my last one...).

>

> I understand that using an NST and HV cap in series with the primary, the HV

> cap will discharge through the primary-chokes-NST which is a pretty low

> resistance path.

>

> But I haven't got NST: I use a voltage doubler (the microwave oven design)

> and that includes voltage doubling capacitors IN SERIES with the

> transformers.

>

> This means that, after power-off, the HV and other capacitors are VERY likely

> to be still charged: how to discharge them safely?

>

> I DON'T want to short them every time by hands: also that is a pretty risky

> operation if you touch the wrong place (and what if you "forget" doing it?). I

> see basically two solutions:

>

> 1. A 10 Mohm HV resistor in parallel with each capacitor. This is what is used

> in the capacitors employed in microwave ovens (they have got this resistor

> INSIDE the cap can). But how that will affect the HV capacitor performance?

>

> 2. A kind of delayed-relay battery that will short all caps through, say,

> 100 Kohm resistors. But then you will have HV wires running through your

> TC assembly, which is not very nice. And HV relays will be needed too

> (do they exist?).

>

> Any suggestions about this?

>

>  Marco Denicolai

Marco,

Putting 10 Mohm HV bleeder resistors in parallel with the tank caps

should work fine, since the primary Q will really be dominated by spark

gap losses. This is a straightforward and relatively foolproof approach.

While the HV relay and lower value bleeder resistor approach will also

work, a HV relay would either need to be found in the surplus market

(and even there, these still tend to be fairly expensive) or you'll need

to make one driven from a solenoid or air cylinder. The more complex the

design, the more possibilities of something not working properly...

-- Bert --

Date: Mon, 01 Jun 1998 08:02:21 -0700

From: Gene Waldo <gwaldo@1stnetusa.com>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Organization: S.A.P.I.E.N.T. (Surly, Agressive, Politically Incorrect Enemies of Non Thinkers)

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Capacitor discharge

Sender: tesl-err@emachine.com

Bert Hickman wrote:

>

> Marco Denicolai wrote:

> > This means that, after power-off, the HV and other capacitors are VERY likely

> > to be still charged: how to discharge them safely?

> >

> > Any suggestions about this?

> >

Marco,

I plan on a simple insulated extension wire from the RFGnd point,

terminating in a brass 'wand', which can be used to short ea cap

terminal to gnd.  This can also be positioned near toroid to 'calibrate'

performance during tuning.

-----------------------------------------

Gene Waldo -  gwaldo@1stnetusa.com  - "Igor in Residence"

From: Mike Hammer <mhammer@misslink.net>

Date: Mon, 01 Jun 1998 10:01:21 -0500

Subject: Re: 18 Kw 14.4 kv  Demo

X-Mailer: QUALCOMM Windows Eudora Light Version 3.0.5 (32)

X-Sender: mhammer@mail.misslink.net

Message-Id: <3.0.5.32.19980601100121.0083c5c0@mail.misslink.net>

To: "Leben Lang und Frei" <fleischwulf@mailexcite.com>

Content-Type: text/plain; charset=us-ascii

Content-Length: 1851

Content-Transfer-Encoding: 7bit

At 07:15 AM 6/1/98 -0700, you wrote:

>Mike,

>

>  Last week I had some friends over including Bill (The arcstarter) and a

friend of his, Charlie. Charlie reciently purchased a pole pig. He didn't

realized just how dangerous they are. I incremented the current output from

180 ma. to 1.25 amps. I used a hot dog to demonstrate the effect on flesh.

At 7.2 Kw the hot dog almost caught fire. It would have if I would have

left the current on a few seconds longer, the smoke wasn'nt so pleasent. At

18 Kw the results were remarkable. The fat and sodium in the hotdog

produced a very hot flame, the fat was igniting almost explosivly. It

actually burned like a fuse. It burned at a rate of about .5" a second.  It

was shrouded in a bright yellow flame, about 8" in diameter. The amp meter

read a full 1.25 amps. (sodium). After the demo, Charlie was ready to sell

the pig. I think he realized this was a death on contact situation! I would

suggest anyone wanting to buy a pig review this. It is very eye opening.

>

>

>

>Paul Marshall

Paul,

I agree very much. I see so many new coilers (mostly teenagers)

coming into the hobby with absolutely no idea how to handle HV.

You can talk safety and working up slowly in power with NSTs

till your blue in the face. These guys see the pics of pig

driven coils throwing 12 foot sparks and thats what they

want right away.

I know a young coiler that nearly killed himself with

a pair of neons. He was very lucky but has now developed

proper respect for the voltages involved. I think its only

a matter of time before someone kills themselves and companies

like T&R Electric and Soloman quit selling to individuals.

Why dont you post your experiment to the list along with

some warnings about the dangers involved with using a pig.

TTYL!

Mike Hammer

mhammer@misslink.net

From: <BPTesla@aol.com>

Date: Tue, 2 Jun 1998 01:08:36 EDT

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: 6 inch PVC....free

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

I have twenty feet of 6" PVC that I need to unload pronto! I wont charge

anything for the pipe, just pay for the postage and shipping costs.

Just specify the length and I will cut it and ship it to you. I am located in

Arizona so those of you in this area may benefit well.

I gotta make it disappear!

Send queries to: BPTesla@aol.com

From: wwl@netcomuk.co.uk (Mike Harrison)

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: UK Teslathon pictures

Date: Tue, 02 Jun 1998 17:53:16 GMT

Organization: White Wing Logic

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

Photos from the UK teslathon :

www.netcomuk.co.uk/~wwl/uktesla.html

____                                                           ____

_/ L_/  Mike Harrison / White Wing Logic / wwl@netcomuk.co.uk  _/ L_/

_/ W_/  Hardware & Software design / PCB Design / Consultancy  _/ W_/

/_W_/  Industrial / Computer Peripherals / Hazardous Area      /_W_/

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Date: Wed, 03 Jun 1998 07:30:32 -0700

From: "Leben Lang und Frei" <fleischwulf@mailexcite.com>

Cc:

X-Sent-Mail: off

X-Mailer: IPAD 2.0

Subject: Wax paper / experiments

X-Sender-Ip: 208.153.7.9

Organization: MailExcite  (http://www.mailexcite.com:80)

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

I was wondering wether anyone has ever tried to make a rolled cap using wax paper

(under oil). In theory it should work. I'm not actually considering this as a working capacitor, my preference is for poly. Does anyone have an idea or feel for what dielectric constant might be ? My plan is to make a working model (multi-layer) experimental cap.

I will try to evaluate the break down voltage and survivability of such a cap. I'll be

using a little Dialx transformer oil and a 2 kv transformer for the break in period.

I will then increase the voltage 600v at a time until the break occurs. I will estimate

the operational voltage (continous operation) to be approximatly 50 - 60% of the break

down number. I will advise.

Paul M

---

Einigkeit und Freiheit

Date: Wed, 03 Jun 1998 07:30:49 -0700

From: Gene Waldo <gwaldo@1stnetusa.com>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Organization: S.A.P.I.E.N.T. (Surly, Agressive, Politically Incorrect Enemies of Non Thinkers)

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: UK Teslathon pictures

Sender: tesl-err@emachine.com

Mike Harrison wrote:

>

> Photos from the UK teslathon :

> www.netcomuk.co.uk/~wwl/uktesla.html

>      ____                                                           ____

Mike,

Neat stuff!

How, and into which well fertilized mind did the inspiration to

microwave CDs occur? All forms and sorts of 'kinetic art' possibilities

now trouble my mind! I must go back to the site and hit that page, which

I missed in my initial romp-thru!

Pinwheels seem to be the extent of Tesla kinetic art to date, but this

has long troubled me....  Well grounded CDs (among other things) in

series, paralell, rotating sets, swinging? Lots of flash and clatter!???

Long streamers are only raw materials! They should DO something! Short

of setting fire to your immediate surrounds!

Also: I recall from distant reads, something about a spark-accoustic

'organ' type of instrument.  Anyone any info? Visualise: many plex

tubes, tuned, containing ladders or gaps, with caps across them to boost

the current and clatter. What effects would the controlled

administration of argon etc (propane?) (Pyrex instead of plex?) have on

timbre, tone and color etc? Keyboard design now becomes an interesting

engineeering challenge! Tesla-fests featuring the "Electric

Whurlblitzer"! Mit flamen und fumin!

Someone out there has to have more engineering-prototype-time and

facilities than I! LOD might just be the hot bed of creativity for this

sort of little item! The "Illuminated Tacata und Fugue"!

Just a thought.

--------------------------------------------

Gene Waldo -  gwaldo@1stnetusa.com  - "Igor in Residence"

---------------------------------------------------------------

Date: Wed, 03 Jun 1998 09:53:38 -0500

From: Tom Polk <tpolk@prismnet.com>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Wax paper / experiments

Sender: tesl-err@emachine.com

Leben Lang und Frei wrote:

>

> I was wondering wether anyone has ever tried to make a rolled cap using wax paper

>(under oil). In theory it should work. I'm not actually considering this as a working >capacitor, my preference is for poly. Does anyone have an idea or feel for what dielectric >constant might be ? My plan is to make a working model (multi-layer) experimental cap.

>I will try to evaluate the break down voltage and suriveability of such a cap. I'll be

>using a little Dialx transformer oil and a 2 kv transformer for the break in period.

>I will then increase the voltage 600v at a time until the break occurs. I will estimate

>the operational voltage (continous operation) to be approximatly 50 - 60% of the break

>down number. I will advise.

>

> Paul M

> ---

> Einigkeit und Freiheit

>

My chart says paraffin coated paper dialectric is 2.5-4.

X-Mailer: IPAD 2.0

Date: Wed, 03 Jun 1998 11:54:51 -0500

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: Wax paper / experiments

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

At 07:30 AM 6/3/98 -0700, you wrote:

>I was wondering wether anyone has ever tried to make a rolled cap using wax paper

>(under oil). In theory it should work. I'm not actually considering this as a working >capacitor, my preference is for poly. Does anyone have an idea or feel for what

>dielectric constant might be ? My plan is to make a working model (multi-layer)

>experimental cap. I will try to evaluate the break down voltage and suriveability

>of such a cap. I'll be using a little Dialx transformer oil and a 2 kv transformer

>for the break in period. I will then increase the voltage 600v at a time until the >breakoccurs. I will estimate the operational voltage (continous operation) to be

>approximatly 50 - 60% of the break down number. I will advise.

>

>Paul M

>---

>Einigkeit und Freiheit

Paul,

Ive never used wax paper but I didnt quite a series of experiments

on oil soaked copying machine paper. The breakdown voltage per sheet

is quite low so a stack of ten sheets was used soaked in mineral oil.

This was a flat plate cap. While the results were not spectacular they

were good enough to produce some nice sparks from my small coil at about 5kv.

Working backwards from the measured capacitance value I got a

dielectric constant of just under 3 for the paper dielctric.

I never did any follow up work on it as the voltage rating

was very low and there wasnt any advantage in using the paper.

Especially since I have a 2500 sq foot roll of 6 mil poly.

TTYL!

Mike Hammer

mhammer@misslink.net

X-Mailer: IPAD 2.0

Date: Wed, 03 Jun 1998 12:17:54 -0500

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Mike Hammer <mhammer@misslink.net>

Subject: If it wasn't for bad luck ...

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

All,

Some of you may have noticed I havnt been posting

much recently and I have tons of unanswered email.

Well I have suffered a variety of problems in the

past couple weeks. First I had a program crash wipe

out my install of Eudora. I was compacting mailboxes

when another running program crashed and locked the

machine up. This corrupted the mailboxes and I lost

15000+ emails.

I proceeded to reinstall a new upgraded version of Eudora

and was in the process of rebuilding my contact list

when a hard drive failure wiped me out. This time I lost

750 Megs of data. This wasnt my boot drive so was still

operational. But the lost data hurt pretty bad. My own

fault for not having backups.

I then ordered a 5G IDE tape drive so I could begin

backing up my data. A little late but at least Ive

got it. I had been working on trying to get the

drive to coexist with the rest of my system but

so far havnt had much luck.

Now comes the clincher. This morning my monitor failed.

Just what I needed. Im using an old 14 incher right now

and it really sucks. After years of 1024 X 768 at 80hz

try going back to 640 X 480 at 60 hz.

Unfortunately I lack the necessary funds for a new monitor

or a new hard drive. I purchased the tape drive with money

I dont have in a moment of weakness. Credit cards are dangerous.

To top it all off Im due to join the ranks of the unemployed

at the end of this month. Thats the reason for the money problems.

As I get things back to together I will answer what mail

I can. If I dont answer inquiries right away then you know why.

TTYL!

Mike Hammer

mhammer@misslink.net

Date: Wed, 03 Jun 1998 14:49:41 -0700

From: Gene Waldo <gwaldo@1stnetusa.com>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Organization: S.A.P.I.E.N.T. (Surly, Agressive, Politically Incorrect Enemies of Non Thinkers)

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: If it wasn't for bad luck ...

Sender: tesl-err@emachine.com

Mike Hammer wrote:

>

> All,

>

> Some of you may have noticed I havnt been posting

> much recently and I have tons of unanswered email.

>

> Well I have suffered a variety of problems in the

> past couple weeks.  If I dont answer inquiries right away then you know why.

>

> TTYL!

>

> Mike Hammer

> mhammer@misslink.net

Mike! Sounds like you have contracted 'WaldoLuck'! A disease which I

thought peculiar only to me!  I thought I was in temporary remission,

but now I find myself trying to keep all 4 jets running on my damn

printer!

Welcome to the ward and keep Hammering!

--

Gene Waldo -  gwaldo@1stnetusa.com  - "Igor in Residence"

---------------------------------------------------------------

X-Mailer: IPAD 2.0

Date: Wed, 03 Jun 1998 19:27:56 -0400

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Jim Ryan <jryan@exis.net>

Subject: Re: CAP Size

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Mike, I just got back online. My hard drive crashed and I lost all of

my TC email and data. I now have constructed / purchased all of the

components for my 12KV 30ma. coil except the rolled capacitors. I'm the guy

that is going to use .005 brass shim stock and 8 layers of .006 poly.

My Question is again,(sorry) how many square inches of brass/plate do I need

per cap. I plan to make 2 caps and connect them in series. I think

you told me the total should be 450 sg.inches for .0066uf. Or was that

450 for each cap. at .0033 ?

Thanks

Jim

From: <Dsurfr@aol.com>

Date: Wed, 3 Jun 1998 21:15:08 EDT

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: If it wasn't for bad luck ...

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Mike: Sory to hear about all the trouble. Seems to me that Chip (who runs the

Tesla List) was suffering some temporary financial problems awhile back & a

lot of those on his list came to his aid. I can't afford much but I'd be more

than willing to kick in a little each month if it would help out. Without your

help and the advice from others on the Tesla2 List I never would have gotten

my coil up & running. We can't loose you now. Just let us know if it would be

OK  Thanks again for everything ,

Jim

X-Mailer: IPAD 2.0

Date: Wed, 03 Jun 1998 21:57:52 -0400

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Jim Ryan <jryan@exis.net>

Subject: Re: If it wasn't for bad luck ...

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

At 09:15 PM 6/3/98 EDT, you wrote:

>Mike: Sorry to hear about all the trouble. Seems to me that Chip (who runs the

>Tesla List) was suffering some temporary financial problems awhile back & a

>lot of those on his list came to his aid. I can't afford much but I'd be more

SNIP SNIP

>

>

Hell yes, I'll be the first, Mike has helped me beyond the normal expected

help. This is like a BBS that we paid for in the past. No problem. If I

spent as much time helping out folks with TC problems, I would expect some help

to keep it alive. Please post an address.

Jim Ryan

Date: Wed, 03 Jun 1998 20:59:05 -0500

From: Tom Polk <tpolk@prismnet.com>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: CAP Size

Sender: tesl-err@emachine.com

Jim Ryan wrote:

>

> Mike, I just got back online. My hard drive crashed and I lost all of

> my TC email and data. I now have constructed / purchased all of the

> components for my

> 12KV 30ma. coil except the rolled capacitors. I'm the guy that is

> going to use .005 brass shim stock and 8 layers of .006 poly.

>

> My Question is again,(sorry) how many sguare inches of brass/plate do I need

> per cap. I plan to make 2 caps and connect them in series. I think

> you told me the total should be 450 sg.inches for .0066uf. Or was that

> 450 for each cap. at .0033 ?

> Thanks

>

> Jim

I assume you are going to have 8 plates with the 8 layers.

For a single capacitor of .0066 use the following table for .006 poly

Dialectric constant	No. of Plates	Sq Inches/PlateCapacitance

2.5			8	10.2

3			8	 8.5

3.5			8	 7.2

4			8	 6.3

8 plates times 8.5 sq inches per plate = 68 sq. inches.

Where did 450 sq inch come from? .006 is very thin, so you don't need a

lot of sq in.  My table says dielectric constant of polyethelene is 2.5.

Date: Wed, 03 Jun 1998 21:01:27 -0500

From: Tom Polk <tpolk@prismnet.com>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: problem:post format?

Sender: tesl-err@emachine.com

Posted text is strung out ~3 full

> > screens wide! Makes for very difficult reading, and my printer looses

> > all off screen copy!

> >

> View

> Wrap long lines

>

> should take care of it on your end.

Can anyone explain what the writer can do to keep the long lines from

appearing in the first place. It is a hassle, admittedly minor, but

still a hassle to click on wrapping long lines.

Date: Wed, 03 Jun 1998 21:31:11 -0500

From: Bert Hickman <bert.hickman@aquila.com>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Organization: Stoneridge Engineering

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: CAP Size

Sender: tesl-err@emachine.com

Jim Ryan wrote:

>

> Mike, I just got back online. My hard drive crashed and I lost all of

> my TC email and data. I now have constructed / purchased all of the

> components for my

> 12KV 30ma. coil except the rolled capacitors. I'm the guy that is

> going to use .005 brass shim stock and 8 layers of .006 poly.

>

> My Question is again,(sorry) how many sguare inches of brass/plate do I need

> per cap. I plan to make 2 caps and connect them in series. I think

> you told me the total should be 450 sg.inches for .0066uf. Or was that

> 450 for each cap. at .0033 ?

> Thanks

>

> Jim

Jim,

The plate overlap area should be about 400 square inches, but you'll

need three plates with this amount of "active" area in each rolled cap

to get 6600 pF per cap. Two of these caps in series gets you to 3300 pF.

While brass will no doubt work, aluminum will function as well, and

would be considerably less expensive.

Safe cappin' to you!

-- Bert --

X-Mailer: IPAD 2.0

Date: Wed, 03 Jun 1998 23:23:02 -0500

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: If it wasn't for bad luck ...

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

At 09:57 PM 6/3/98 -0400, you wrote:

>At 09:15 PM 6/3/98 EDT, you wrote:

>>Mike: Sorry to hear about all the trouble. Seems to me that Chip (who runs the

>>Tesla List) was suffering some temporary financial problems awhile back & a

>>lot of those on his list came to his aid. I can't afford much but I'd be more

>SNIP SNIP

>

>>

>>Hell yes, I'll be the first, Mike has helped me beyond the normal expected

>help. This is like a BBS that we paid for in the past. No problem. If I

>spent as much time helping out folks with TC problems, I would expect some help to keep

>it alive. Please post an address.

>

>Jim Ryan

Guys while I appreciate the sentiment Im not looking for

donations. I just had a bad run of luck and the financial

situation will work itself out. My point in the post was to

let every know I wont be as active as before and to let everyone

know why.

TTYL!

Mike Hammer

mhammer@misslink.net

X-Mailer: IPAD 2.0

Date: Wed, 03 Jun 1998 23:34:48 -0500

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: problem:post format?

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

At 09:01 PM 6/3/98 -0500, you wrote:

>Posted text is strung out ~3 full

>> > screens wide! Makes for very difficult reading, and my printer looses

>> > all off screen copy!

>> >

>> View

>> Wrap long lines

>>

>> should take care of it on your end.

>

>Can anyone explain what the writer can do to keep the long lines from

>appearing in the first place. It is a hassle, admittedly minor, but

>still a hassle to click on wrapping long lines.

Just keep your line lengths less than 80 characters.

70 Characters is a good line length to settle on.

Some email programs like Pegasus Mail let you set

line length to anything you want. Thats one feature

Eudora doesnt have. However I just keep line lengths

short and that will mostly eliminate that kind of problem.

TTYL!

Mike Hammer

mhammer@misslink.net

Date: Wed, 03 Jun 1998 21:37:47 -0700

From: Michael Loftus <loftusm@ix.netcom.com>

Organization: Me

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

CC: ottalini@mindspring.com

Subject: Re: Poly @ 15kv

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Hi!

I mounted two 4" x 4" aluminum squares parallel and very close together

on a piece of wood.  I then connected them to my 15kv 30ma Neon. Kind of

a flat-faced spark-gap with almost no gap.

I then slid an increasing number of 6" x 6" 4mil poly sheets between the

two aluminum plates and turned-on the Neon.  At first the sparks were

something to behold, but at 96 total mils the sparks were absent and a

nice blue corona remained. The poly sheets were lightly/moderately

compressed together and I did not utilize any oil. Tomorrow I will

compress the poly somehow and try again. . .

Question: Does this indicate that I need to build my cap(s) with 96+ mil

poly?  It would seem so to me.

Date: Wed, 03 Jun 1998 23:50:49 -0500

From: Tom Polk <tpolk@prismnet.com>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: CAP Size

Sender: tesl-err@emachine.com

Jim Ryan wrote:

>

> Mike, I just got back online. My hard drive crashed and I lost all of

> my TC email and data. I now have constructed / purchased all of the

> components for my 12KV 30ma. coil except the rolled capacitors.

> I'm the guy that is going to use .005 brass shim stock and 8 layers of .006 poly.

>

> My Question is again,(sorry) how many sguare inches of brass/plate do I need

> per cap. I plan to make 2 caps and connect them in series. I think

> you told me the total should be 450 sg.inches for .0066uf. Or was that

> 450 for each cap. at .0033 ?

> Thanks

>

> Jim

Whoops! I think I goofed sending you THAT particular table. It is

correct for what it is, but I didn't even think that you might really

mean .06 poly rather than .006. At 12kv, breakdown with .006 is a given.

Now, assuming you will use .06, repeat, .06 in two series caps, each

with 8 plates, well, that would be 102 sq. inches per plate (106 sq.in

if .0625 poly), 8 plates for .0066 uF for each cap, yielding .0033 in

series with little chance of breakdown at 12kV.

Date: Wed, 03 Jun 1998 23:56:25 -0500

From: Tom Polk <tpolk@prismnet.com>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Poly @ 15kv

Sender: tesl-err@emachine.com

Michael Loftus wrote:

>

> Hi!

>

> I mounted two 4" x 4" aluminum squares parallel and very close together

> on a piece of wood.  I then connected them to my 15kv 30ma Neon. Kind of

> a flat-faced spark-gap with almost no gap.

>

> I then slid an increasing number of 6" x 6" 4mil poly sheets between the

> two aluminum plates and turned-on the Neon.  At first the sparks were

> something to behold, but at 96 total mils the sparks were absent and a

> nice blue corona remained. The poly sheets were lightly/moderately

> compressed together and I did not utilize any oil. Tomorrow I will

> compress the poly somehow and try again. . .

>

> Question: Does this indicate that I need to build my cap(s) with 96+ mil

> poly?

It would seem so to me.

My flat plate cap (.015 uF) has lasted 4 years with occasional use, long

sessions with 15kV. It has 28 .125 polyethelene sheets (12 x 16) with

aluminum foil in 10 x 14 sheets.

.096 is getting reasonably close to .125, but I know .125 does not break

down at 15kV.

From: wwl@netcomuk.co.uk (Mike Harrison)

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: UK Teslathon pictures

Date: Thu, 04 Jun 1998 08:46:27 GMT

Organization: White Wing Logic

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

On Wed, 03 Jun 1998 07:30:49 -0700, you wrote:

>Mike Harrison wrote:

>>

>> Photos from the UK teslathon :

>> www.netcomuk.co.uk/~wwl/uktesla.html

>>      ____                                                           ____

>Mike,

>Neat stuff!

> How, and into which well fertilized mind did the inspiration to

>microwave CDs occur? All forms and sorts of 'kinetic art' possibilities

>now trouble my mind! I must go back to the site and hit that page, which

>I missed in my initial romp-thru!

Heard about it ages ago on a stupid radio show, but tesla-zapping

microwaved CDs was one of those happy accidents. I thought the edge of

the metallisation on a CD would be a good corona source - it just

happened that the first CD I tried it with just happened to have been

microwaved beforehand!

>Pinwheels seem to be the extent of Tesla kinetic art to date, but this

>has long troubled me....  Well grounded CDs (among other things) in

>series, paralell, rotating sets, swinging? Lots of flash and clatter!???

>Long streamers are only raw materials! They should DO something! Short

>of setting fire to your immediate surrounds!

Problem is the metallisation comes off real quick (and redistributes

itself as little flakes EVERYWHERE!)

____                                                           ____

_/ L_/  Mike Harrison / White Wing Logic / wwl@netcomuk.co.uk  _/ L_/

_/ W_/  Hardware & Software design / PCB Design / Consultancy  _/ W_/

/_W_/  Industrial / Computer Peripherals / Hazardous Area      /_W_/

Date: Thu, 04 Jun 1998 06:53:49 -0500

From: Bert Hickman <bert.hickman@aquila.com>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Organization: Stoneridge Engineering

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Poly @ 15kv

Sender: tesl-err@emachine.com

Michael Loftus wrote:

>

> Hi!

>

> I mounted two 4" x 4" aluminum squares parallel and very close together

> on a piece of wood.  I then connected them to my 15kv 30ma Neon. Kind of

> a flat-faced spark-gap with almost no gap.

>

> I then slid an increasing number of 6" x 6" 4mil poly sheets between the

> two aluminum plates and turned-on the Neon.  At first the sparks were

> something to behold, but at 96 total mils the sparks were absent and a

> nice blue corona remained. The poly sheets were lightly/moderately

> compressed together and I did not utilize any oil. Tomorrow I will

> compress the poly somehow and try again. . .

>

> Question: Does this indicate that I need to build my cap(s) with 96+ mil

> poly?  It would seem so to me.

Michael,

Multiple thicknesses of thinner dielectric material will have much

greater voltage-handling capability. A cap made from a single thickness

of 96 mil dielectric will have substantially less dielectric strength

than one made from the same thickness of multiple 4-mil sheets. This has

to do with how dielectric breakdown within a dielectric is initiated by

defects in the material. Breaking up the dielectric into individual

sheets tends to stop the breakdown process from propagating through the

remaining sheets. If you make your capacitor(s) with 96 mils

of 4 mil material, you should end up with relatively "bullet proof"

caps at 15 KV.

Reducing the overall voltage stress at the plate edges is also very

important. This can be done by "sharing" the stress across identical

series-connected capacitors. Instead of making a single capacitor with

96 mils of dielectric between plates, make three equally valued

capacitors (each 3X the desired final capacity) using 32 mils of

dielectric thickness in each. This will reduce the other major source of

problems, even in oil-immersed caps.

Also, remember to provide adequate margins between the edge of the

plates and the edge of the dielectric, particularly where the plates are

brought out for connection. This is necessary to prevent plate-to-plate

flashover along the dielectric surface - Even with three

series-connected caps, leave at least a minimum of 1". If you decide to

make only one large cap, use at least 2-2.5" of clearance for 15 KV

operation. Also, be very careful to exclude all the air when immersing

the caps in oil - entrapped air will ionize, and eventually "kill" the

dielectric...

BTW, I use a total of 128 mils of 4 mil material, in 4 identical

series-connected caps. These were designed for for use with either 15 KV

neons or 20 KV pole-pig. They take a lickin' and still keep on

tickin'...

Safe cappin' to you!

-- Bert --

From: Roy Minnich <rminnich@rolling-meadows.lib.il.us>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: RE: Poly @ 15kv

Date: Thu, 4 Jun 1998 13:57:03 -0500

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

Hi all,

Good questions and answers!

Here's some questions that have been bugging me:

Let's say I want to build caps that will withstand 40kv.  If I use 375 volts for the puncture rating on 1 mil of poly I would need 40kv/375v = 106.666 mils of poly.  Yet I'm told that 48 mils should be enough.  What ratings do caps need for a 10kv 25ma or a 12kv 30ma, 60ma, 120ma or a 15kv 30ma etc. etc.  Let's forget pole pigs for now.  This is a beginners list.

Now is the thickness of the poly dielectric reduced to 48 mils due to the dielectric properties of the mineral oil?  Or does the quality of the dielectric increase due to the many layers of say 4 mil poly used?  A little of both????  It seems that is what Bert is saying below.

And to clarify another point: the required dielectric thickness must be between any two plates in the cap no mater what kind, i.e. flat plate or rolled poly/foil caps.  Correct?  It looks funny the way some images of rolled caps look.

How much of a voltage rating does one REALLY need for a single 12kv 30ma neon????  It's a pain in the butt to cut and roll many layers of poly and foil.   And the thicker the poly layers the larger the foil area needs to be for a certain cap value.  I want to use the minimum SAFE dielectric thickness and run series caps to get my V rating.  I'd rather make a several physically smaller caps and hook them in series than try to roll some monster "bullet proof " cap.  I seems that a cap with smaller  surface areas will be easier to make, easier to evacuate and fill with oil.  Not to mention the versatility of having a bunch of same value /voltage rated caps to use for experiments with your Tesla coils.

Thanks,

Roy Minnich

----------

From: 	Bert Hickman[SMTP:bert.hickman@aquila.com]

Reply To: 	Tesla-2 Mailing List

Sent: 	Thursday, June 04, 1998 6:53 AM

To: 	Tesla-2 Mailing List

Subject: 	Re: Poly @ 15kv

Michael Loftus wrote:

>

> Hi!

>

> I mounted two 4" x 4" aluminum squares parallel and very close together

> on a piece of wood.  I then connected them to my 15kv 30ma Neon. Kind of

> a flat-faced spark-gap with almost no gap.

>

> I then slid an increasing number of 6" x 6" 4mil poly sheets between the

> two aluminum plates and turned-on the Neon.  At first the sparks were

> something to behold, but at 96 total mils the sparks were absent and a

> nice blue corona remained. The poly sheets were lightly/moderately

> compressed together and I did not utilize any oil. Tomorrow I will

> compress the poly somehow and try again. . .

>

> Question: Does this indicate that I need to build my cap(s) with 96+ mil

> poly?  It would seem so to me.

Michael,

Multiple thicknesses of thinner dielectric material will have much

greater voltage-handling capability. A cap made from a single thickness

of 96 mil dielectric will have substantially less dielectric strength

than one made from the same thickness of multiple 4-mil sheets. This has

to do with how dielectric breakdown within a dielectric is initiated by

defects in the material. Breaking up the dielectric into individual

sheets prevents tends to stop the breakdown process from propagating

through the remaining sheets. If you make your capacitor(s) with 96 mils

of 4 mil material, you should end up with relatively "bullet proof" caps

at 15 KV.

Reducing the overall voltage stress at the plate edges is also very

important. This can be done by "sharing" the stress across identical

series-connected capacitors. Instead of making a single capacitor with

96 mils of dielectric between plates, make three equally valued

capacitors (each 3X the desired final capacity) using 32 mils of

dielectric thickness in each. This will reduce the other major source of

problems, even in oil-immersed caps.

Also, remember to provide adequate margins between the edge of the

plates and the edge of the dielectric, particularly where the plates are

brought out for connection. This is necessary to prevent plate-to-plate

flashover along the dielectric surface - Even with three

series-connected caps, leave at least a minimum of 1". If you decide to

make only one large cap, use at least 2-2.5" of clearance for 15 KV

operation. Also, be very careful to exclude all the air when immersing

the caps in oil - entrapped air will ionize, and eventually "kill" the

dielectric...

BTW, I use a total of 128 mils of 4 mil material, in 4 identical

series-connected caps. These were designed for for use with either 15 KV

neons or 20 KV pole-pig. They take a lickin' and still keep on

tickin'...

Safe cappin' to you!

-- Bert --

Date: Thu, 4 Jun 1998 15:28:48 -0400 (EDT)

From: Paul Anderson <madhobby@geeky1.ebtech.net>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: What now...

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

Okay, I've got myself a 9kv30ma NST sitting right here, in working

condition (I plugged her in and drew a nice 1/8" thick yellow arc to the

case).  Now, the NST I have here, BTW, is potted, but whatever the stuff

is it isn't tar.  Will I have to unpot it and remove any current-limiting

shunts?

I'd appreciate it if someone could point me in the direction of a good TC

calc program.  TTYL!

---

Paul Anderson

madhobby@geeky1.ebtech.net

X-Mailer: IPAD 2.0

Date: Thu, 04 Jun 1998 17:27:04 -0400

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Jim Ryan <jryan@exis.net>

Subject: Rolled Capacitor

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

> Jim

I assume you are going to have 8 plates with the 8 layers.

For a single capicitor of .0066 use the following table for .006 poly

Dielectric constant	No. of Plates	Sq Inches/PlateCapacitance

2.5			8	10.2

3			8	 8.5

3.5			8	 7.2

4			8	 6.3

8 plates times 8.5 sq inches per plate = 68 sq. inches.

Where did 450 sq inch come from? .006 is very thin, so you don't need a

lot of sq in.  My table says dielectric constant of polyethelene is 2.5.

No No, I'm plan to make a rolled capacitor. Sorry about using the word

plate in my email.

Jim Ryan

Date: Thu, 04 Jun 1998 16:46:44 -0500

From: Tom Polk <tpolk@prismnet.com>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Rolled Capacitor

Sender: tesl-err@emachine.com

Jim Ryan wrote:

>

> > Jim

>

> I assume you are going to have 8 plates with the 8 layers.

> For a single capicitor of .0066 use the following table for .006 poly

>

> Dielectric constant     No. of Plates   Sq Inches/PlateCapacitance

>         2.5                     8       10.2

>         3                       8        8.5

>         3.5                     8        7.2

>         4                       8        6.3

>

> 8 plates times 8.5 sq inches per plate = 68 sq. inches.

> Where did 450 sq inch come from? .006 is very thin, so you don't need a

> lot of sq in.  My table says dielectric constant of polyethelene is 2.5.

>

> No No, I'm plan to make a rolled capacitor. Sorry about using the word

> plate in my email.

>

> Jim Ryan

OK, for a dielectric constant of 2.5 you need 565 sq. in., or with 3.0

you need 472 sq. in. per plate to get .0066 uF.

X-Mailer: IPAD 2.0

Date: Thu, 04 Jun 1998 16:49:56 -0500

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Mike Hammer <mhammer@misslink.net>

Subject: RE: Poly @ 15kv

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

At 01:57 PM 6/4/98 -0500, you wrote:

>Hi all,

>

>Good questions and answers!

>

>Here's some questions that have been bugging me:

>

>Let's say I want to build caps that will withstand 40kv.  If I use 375 volts for the >puncture rating on 1 mil of poly I would need 40kv/375v = 106.666 mils of poly.

>Yet I'm told that 48 mils should be enough.  What ratings do caps need for a 10kv

>25ma or a 12kv 30ma, 60ma, 120ma or a 15kv 30ma etc. etc.  Let's forget pole pigs

>for now.  This is a beginners list.

>

>Now is the thickness of the poly dielectric reduced to 48 mils due to the dielectric >properties of the mineral oil?  Or does the quality of the dielectric increase due to

>the many layers of say 4 mil poly used?  A little of both????  It seems that is what

>Bert is saying below.

Roy,

The total amount of poly needed for various voltages depends on

a lot of factors besides the voltage. It depends on the thickness

of the poly sheets and whether it is a single cap or a series string.

When I mentioned that 48 mils stands up well to 12kv I meant

that the 48 mils is achieved through a stack of thinner material

and a series string of caps.

The PER MIL rating of poly goes up as the material becomes thinner.

Said another way lets say for the sake of illustration only that a

10 mil thick sheet of poly is good for 3000V. Does that mean that

a 20 mil sheet is good for 6000? No. the thicker sheet has a lower

PER MIL rating. It might only handle 5000V. The thicker the poly sheets

the lower the per mil rating. Thats why a stack of ten 6 mil sheets

of poly will have a much higher rating than a single 60 mil thick sheet.

Now if we divide up the poly into two seperate caps instead of

one single cap we find that the voltage rating increases still more.

Two 30 mil caps in series have a greating voltage rating and a higher

likelihood of survival than a single 60 mil cap.

>

>And to clarify another point: the required dielectric thickness must be

>between any two plates in the cap no matter what kind, i.e. flat plate or

>rolled poly/foil caps.  Correct?  It looks funny the way some images of

>rolled caps look.

Correct.

>

>How much of a voltage rating does one REALLY need for a single 12kv 30ma neon????

The bare minimum is 1.414 X 12kv = 16968 V. Some extra safety margin

is desirable.

>It's a pain in the butt to cut and roll many layers of poly and foil.

>And the thicker the poly layers the larger the foil area needs to be for

>a certain cap value.  I want to use the minimum SAFE dielectric thickness

>and run series caps to get my V rating.  I'd rather make a several physically

>smaller caps and hook them in series than try to roll some monster "bullet proof"

>cap.  I seems that a cap with smaller surface areas will be easier to make,

>easier to evacuate and fill with oil.  Not to mention the versatility of having a

>bunch of same value /voltage rated caps to use for experiments with your Tesla coils.

Yes this is the best method. Make several large value caps and run them

in series. Two in series seems to work well at 12kv while 3 in series would

be best for 15kv. For your 12kv/30ma neon you would need two .0132 mfd caps

in series to yield a final value of .0066 mfd. Minimum requirement is 24 mils

in each cap for a total of 48 mils. If you want extra safety margin then use

30 mils in each cap.

Also note that trying to make a monster bullet proof single cap is an

excercise in futility. No matter how thick the poly you just cant run

a single cap on 15kv for long. Corona will form even under oil. Ive

seen plenty of 90 mil caps fail at 15kv. I now of at least one 120 mil

cap that worked for a while then failed at 15kv. Dissecting the remains

showed burn and scorch marks all around the plate edges and the plate ends.

This cap was in oil and vacuumed. But using a single cap on 15kv resulted

in corona that ate away at the insulation until it failed.

TTYL!

Mike Hammer

mhammer@misslink.net

X-Mailer: IPAD 2.0

Date: Thu, 04 Jun 1998 16:59:14 -0500

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: What now...

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

At 03:28 PM 6/4/98 -0400, you wrote:

>Okay, I've got myself a 9kv30ma NST sitting right here, in working

>condition(I plugged her in and drew a nice 1/8" thick yellow arc to the

>case).  Now, the NST I have here, BTW, is potted, but whatever the stuff

>is it isn't tar.  Will I have to unpot it and remove any current-limiting

>shunts?

Do not unpot a good working neon. Use it as is and if it fails

then consider unpotting it.

Your neon may be potted in one of the polymers that are appearing

more often now. Ive seen several Transcos with this type of potting

compound. It doesnt appear possible to unpot one of these. If it

fails on you heave it into the nearest land fill.

A 9kv/30 will do nicely for a small coil. Build a good poly

.0088 mfd cap to get the most from this neon.

>I'd appreciate it if someone could point me in the direction of a good TC

>calc program.

>  TTYL!

One of the better ones is WinTesla by Scott Coppersmith.

You can download Win95 and Win3.11 versions from his page

at: http://members.aol.com/rscopper/index.htm

as well as from Dave Bronakowski's archive site:

http://www.velocity.net/~djb/tesla.shtml

TTYL!

Mike Hammer

mhammer@misslink.net

X-Mailer: IPAD 2.0

Date: Thu, 04 Jun 1998 17:12:39 -0500

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: CAP Size

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

At 07:27 PM 6/3/98 -0400, you wrote:

>Mike, I just got back online. My hard drive crashed and I lost all of

>my TC email and data.

Jim,

I likewise suffered a hard drive failure (among other problems)

and lost over 750 M of data. I lost all our previous correspondence

as well as 15000+ emails.

> I now have constructed / purchased all of the components for my

>12KV 30ma. coil except the rolled capacitors. I'm the guy that is

>going to use .005 brass shim stock and 8 layers of .006 poly.

>

>My Question is again,(sorry) how many sguare inches of brass/plate do I need

>per cap. I plan to make 2 caps and connect them in series. I think

>you told me the total should be 450 sg.inches for .0066uf. Or was that

>450 for each cap. at .0033 ?

>Thanks

>

>Jim

OK I remember us talking about the brass shim stock before.

Since you are making two rolled caps you want to divide that

poly up into 4 layers (24 mils) per cap. You may want to

increase this to 30 mils per cap for extra safety margin.

Each cap should then have .0132 mfd capacitance so when

connected in series the resulting capacitance is .0066 mfd.

So to get .0066 mfd both of the caps must be .0132 mfd.

450 sq inches of plate material in 2 plates rolled up

with 30 mils of poly will result in about .0134 mfd.

Close enough. Two of these in series will yield .0067 mfd.

Now that 450 sq inches can be of any dimensions. 9 inches

wide by 50 inches long for example. Make the poly layers

wide enough to prevent arcing. With 9 inch wide plates

go with 12 inch wide poly. That gives a 1.5 inch margin

on each side.

TTYL!

Mike Hammer

mhammer@misslink.net

X-Mailer: IPAD 2.0

Date: Thu, 04 Jun 1998 17:16:52 -0500

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: CAP Size

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

At 11:50 PM 6/3/98 -0500, you wrote:

>Jim Ryan wrote:

>>

>> Mike, I just got back online. My hard drive crashed and I lost all of

>> my TC email and data. I now have constructed / purchased all of the

>> components for my

>> 12KV 30ma. coil except the rolled capacitors. I'm the guy that is

>> going to use .005 brass shim stock and 8 layers of .006 poly.

>>

>> My Question is again,(sorry) how many sguare inches of brass/plate do I need

>> per cap. I plan to make 2 caps and connect them in series. I think

>> you told me the total should be 450 sg.inches for .0066uf. Or was that

>> 450 for each cap. at .0033 ?

>> Thanks

>>

>> Jim

>

>Whoops! I think I goofed sending you THAT particular table. It is

>correct for what it is, but I didn't even think that you might really

>mean .06 poly rather than .006. At 12kv, breakdown with .006 is a given.

>

>Now, assuming you will use .06, repeat, .06 in two series caps, each

>with 8 plates, well, that would be 102 sq. inches per plate (106 sq.in

>if .0625 poly), 8 plates for .0066 uF for each cap, yielding .0033 in

>series with little chance of breakdown at 12kV.

Tom,

What Jim was referring to was using stacks of 6 mil poly

to add up to 48 mils. I discourage the use of single thick

sheets of poly. A stack of thinner sheets has a much better

rating. He was also working on a rolled cap rather than a

plate cap.

TTYL!

Mike Hammer

mhammer@misslink.net

From: Roy Minnich <rminnich@rolling-meadows.lib.il.us>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: RE: Poly @ 15kv

Date: Thu, 4 Jun 1998 17:47:22 -0500

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

Hi Mike,

Very clear concise answer.  Thank you!

Roy Minnich

----------

From: 	Mike Hammer[SMTP:mhammer@misslink.net]

Reply To: 	Tesla-2 Mailing List

Sent: 	Thursday, June 04, 1998 4:49 PM

To: 	Tesla-2 Mailing List

Subject: 	RE: Poly @ 15kv

At 01:57 PM 6/4/98 -0500, you wrote:

>Hi all,

>

>Good questions and answers!

>

>Here's some questions that have been bugging me:

>

>Let's say I want to build caps that will withstand 40kv.  If I use 375 volts for the puncture rating on 1 mil of poly I would need 40kv/375v = 106.666 mils of poly.  Yet I'm told that 48 mils should be enough.  What ratings do caps need for a 1

0kv 25ma or a 12kv 30ma, 60ma, 120ma or a 15kv 30ma etc. etc.  Let's forget pole pigs for now.  This is a beginners list.

>

>Now is the thickness of the poly dielectric reduced to 48 mils due to the dielectric properties of the mineral oil?  Or does the quality of the dielectric increase due to the many layers of say 4 mil poly used?  A little of both????  It seems t

hat is what Bert is saying below.

Roy,

The total amount of poly needed for various voltages depends on

a lot of factors besides the voltage. It depends on the thickness

of the poly sheets and whether it is a single cap or a series string.

When I mentioned that 48 mils stands up well to 12kv I meant

that the 48 mils is achieved through a stack of thinner material

and a series string of caps.

The PER MIL rating of poly goes up as the material becomes thinner.

Said another way lets say for the sake of illustration only that a

10 mil thick sheet of poly is good for 3000V. Does that mean that

a 20 mil sheet is good for 6000? No. the thicker sheet has a lower

PER MIL rating. It might only handle 5000V. The thicker the poly sheets

the lower the per mil rating. Thats why a stack of ten 6 mil sheets

of poly will have a much higher rating than a single 60 mil thick sheet.

Now if we divide up the poly into two seperate caps instead of

one single cap we find that the voltage rating increases still more.

Two 30 mil caps in series have a greating voltage rating and a higher

likelihood of survival than a single 60 mil cap.

>

>And to clarify another point: the required dielectric thickness must be between any two plates in the cap no matter what kind, i.e. flat plate or rolled poly/foil caps.  Correct?  It looks funny the way some images of rolled caps look.

Correct.

>

>How much of a voltage rating does one REALLY need for a single 12kv 30ma neon????

The bare minimum is 1.414 X 12kv = 16968 V. Some extra safety margin

is desirable.

>It's a pain in the butt to cut and roll many layers of poly and foil.

>And the thicker the poly layers the larger the foil area needs to be for

>a certain cap value.  I want to use the minimum SAFE dielectric thickness

>and run series caps to get my V rating.  I'd rather make a several physically

>smaller caps and hook them in series than try to roll some monster "bullet proof"

>cap.  I seems that a cap with smaller  surface areas will be easier to make,

>easier to evacuate and fill with oil.  Not to mention the versatility of having a

>bunch of same value /voltage rated caps to use for >experiments with your Tesla coils.

Yes this is the best method. Make several large value caps and run them

in series. Two in series seems to work well at 12kv while 3 in series would

be best for 15kv. For your 12kv/30ma neon you would need two .0132 mfd caps

in series to yield a final value of .0066 mfd. Minimum requirement is 24 mils

in each cap for a total of 48 mils. If you want extra safety margin the use

30 mils in each cap.

Also note that trying to make a monster bullet proof single cap is an

excercise in futility. No matter how thick the poly you just cant run

a single cap on 15kv for long. Corona will form even under oil. Ive

seen plenty of 90 mil caps fail at 15kv. I now of at least one 120 mil

cap that work for a while then failed at 15kv. Dissecting the remains

showed burn and scorch marks all around the plate edges and the plate ends.

This cap was in oil and vacuumed. But using a single cap on 15kv resulted

in corona that ate away at the insulation until it failed.

TTYL!

Mike Hammer

mhammer@misslink.net

From: "Choad" <choad69@geocities.com>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: What now...

Date: Thu, 4 Jun 1998 20:25:36 -0500

X-Mailer: IPAD 2.0

X-MimeOLE: Produced By Microsoft MimeOLE V4.72.2106.4

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

I actually found that that plastic potting is easier to take off the core than the

tar like stuff. It just seemed to come right off the core once I got it split down the

middle. That's my 2 cent worth.

>At 03:28 PM 6/4/98 -0400, you wrote:

>>Okay, I've got myself a 9kv30ma NST sitting right here, in working

>>condition(I plugged her in and drew a nice 1/8" thick yellow arc to the

>>case).  Now, the NST I have here, BTW, is potted, but whatever the stuff

>>is it isn't tar.  Will I have to unpot it and remove any current-limiting

>>shunts?

>

>Do not unpot a good working neon. Use it as is and if it fails

>then consider unpotting it.

>

>Your neon may be potted in one of the polymers that are appearing

>more often now. Ive seen several Transcos with this type of potting

>compound. I doesnt appear possible to unpot one of these. If it

>fails on you heave it into the nearest land fill.

>

>A 9kv/30 will do nicely for a small coil. Build a good poly

>.0088 mfd cap to get the most from this neon.

>

>

>>I'd appreciate it if someone could point me in the direction of a good TC

>>calc program.

>

>One of the better ones is WinTesla by Scott Coppersmith.

>You can download Win95 and Win3.11 versions from his page

>at: http://members.aol.com/rscopper/index.htm

>

>as well as from Dave Bronakowski's archive site:

>http://www.velocity.net/~djb/tesla.shtml

>

>TTYL!

>

>Mike Hammer

>mhammer@misslink.net

X-Mailer: IPAD 2.0

Date: Fri, 05 Jun 1998 02:01:12 -0700

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: bob golding <yubba@clara.net>

Subject: max toroid size

Cc: tesla-2@emachine.com

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Hi all

should be ready for first firing Sunday. I have my 8" toroid to tune my

coil and am going to make a larger one as well. What limits the max size? I

know Richard Hull says go for the largest size possible but is it the power

output of the transformer or number of primary turns that will limit the

size. I am using 9kv at 50 ma, 10 turns on my primary, and 900 turns 23 awg

on a 4.25" x 21" secondary with a 0.0177uf flat plate cap. many thanks.

bob golding lightning without rain

bob yubba@clara.net

Date: Thu, 04 Jun 1998 20:58:50 -0500

From: Bert Hickman <bert.hickman@aquila.com>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Organization: Stoneridge Engineering

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: max toroid size

Sender: tesl-err@emachine.com

bob golding wrote:

>

> Hi all

>         should be ready for first firing Sunday. I have my 8" toroid to tune my

> coil and am going to make a larger one as well. What limits the max size? I

> know Richard Hull says go for the largest size possible but is it the power

> output of the transformer or number of primary turns that will limit the

> size. I am using 9kv at 50 ma, 10 turns on my primary, and 900 turns 23 awg

> on a 4.25" x 21" secondary with a 0.0177uf flat plate cap. many thanks.

>

> bob golding lightning without rain

> bob yubba@clara.net

Bob,

As you increase the toroid outer diamater you increase it's capacitance.

For a fixed tank cap size, this has the effect of lowering the resonant

frequency of the secondary, forcing you to do the same with the primary.

The limiting factor is either running out of tuning range on the primary

coil, or developing insufficient voltage to achieve breakout.

As you increase the radius of curvature (ROC), you'll increase the

breakout voltage of the toroid. Increase it too much, and your system

won't develop enough output voltage to "break out". At this point,

you'll need to either increase the tank cap capacitance, or increase the

gap breakdown voltage, so that you can get sufficient output voltage. In

former case, the capability to rapidly recharge the larger tank

capacitor (i.e., your HV power supply's output current) will be the

limiting factor. In the latter case, it will be the HV supply and tank

capacitor's voltage withstanding capability.

Based upon the power level of your system, a 3 or 4" ROC toroid should

work fine, with an outer diameter of 3-4X your coilform diameter.

-- Bert --

X-Mailer: IPAD 2.0

Date: Thu, 04 Jun 1998 21:21:01 -0500

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: max toroid size

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

At 02:01 AM 6/5/98 -0700, you wrote:

>Hi all

>	should be ready for first firing Sunday. I have my 8" toroid to tune my

>coil and am going to make a larger one as well. What limits the max size? I

>know Richard Hull says go for the largest size possible but is it the power

>output of the transformer or number of primary turns that will limit the

>size. I am using 9kv at 50 ma, 10 turns on my primary, and 900 turns 23 awg

>on a 4.25" x 21" secondary with a 0.0177uf flat plate cap. many thanks.

>

>

>bob golding lightning without rain

>bob yubba@clara.net

Bob,

The limitations in toroid size are determined by your

power level and the tuning range of your primary circuit.

Adding a larger toroid adds capacitance to your secondary

lowering its resonant frequency. You then have to tap in

more primary turns to match this lower frequency. If your

toroid is too large you may not be able to bring the coil

back to resonance.

Also a larger toroid requires more power to be able to breakout.

Too large a toroid will prevent any streamers from forming.

The best plan is to start with a small toroid and work your way

up progressively in size until you reach the limits of your system.

TTYL!

Mike Hammer

mhammer@misslink.net

X-Mailer: IPAD 2.0

Date: Fri, 05 Jun 1998 09:09:50 +0300

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Marco Denicolai <marco@vistacom.fi>

Subject: Secondary resonance freq

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

I measured yesterday the resonance frequencies of my secondary. It is a

4.3" diameter PVC tube, winded for a length of 19.7" with AWG 26 wire.

I got the following results:

324 KHz

813 KHz

1475 KHz

1766 KHz

2053 KHz

These without any top toroid or sphere. I used a function generator (sine wave)

and oscilloscope.

I was waiting to measure the fundamental and all odd harmonics (x3, x5, etc.)

but can anybody explain to me how I have got the above readings?

Marco Denicolai

X-Mailer: IPAD 2.0

Date: Fri, 05 Jun 1998 10:00:19 -0500

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: Secondary resonance freq

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

At 09:09 AM 6/5/98 +0300, you wrote:

>I measured yesterday the resonance frequencies of my secondary. It is a

>4.3" diameter PVC tube, winded for a length of 19.7" with AWG 26 wire.

>I got the following results:

>

>324 KHz

>813 KHz

>1475 KHz

>1766 KHz

>2053 KHz

>

>These without any top toroid or sphere. I used a function generator (sine wave)

>and oscilloscope.

>

>I was waiting to measure the fundamental and all odd harmonics (x3, x5, etc.)

>but can anybody explain to me how I have got the above readings?

>

>

>

> Marco Denicolai

Marco,

Using your above specs I calculate a resonant frequency of around

345 khz. Considering that your scope is probably loading the

circuit somewhat your 324 khz figure is probably close.

The other resonances that you saw are not uncommon. A large

inductor like this will often show many parasitic responses.

If your generator has sufficient output try base driving

your secondary through a pair of back to back LEDs. Use

the ultra bright LEDs and tune the generator for maximum

brightness. Maximum base current and therefore maximum LED

brightness will occur at the resonant frequency.

This method has the advantage of not loading down the secondary.

TTYL!

Mike Hammer

mhammer@misslink.net

X-Mailer: IPAD 2.0

Date: Fri, 05 Jun 1998 18:18:34 -0500

From: John Brudvik <john.brudvik@wcom.com>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: McMaster Carr Mica sheets

UUEncode: TRUE

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

I was  interested in the discussion of using the McMaster Carr

dielectric.  Has anyone been successful using this material.  I had

planned on making a plate capacitor using this material but shelved

those plans when I heard about the early failure.  Are there any

updates?

John

From: "Scott Hayward" <smash@eisa.net.au>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Chokes

Date: Sat, 6 Jun 1998 13:29:34 +1000

X-Mailer: IPAD 2.0

X-MimeOLE: Produced By Microsoft MimeOLE V4.71.1712.3

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Sorry, I haven't been on for a while, but I have been reading all of the

E-mails I have got from the list.  I do worry about some of the things that

are being done with cats and tesla coils!!!

I have ferrite things that are wound with small gauge magnet wire that were

part of the yoke assembly on the back of a TV.  I have no idea how much

inductance there would be but there are many turns of wire and I could link

about six in series to get the inductance up.  Good idea or Bad?

From Scott Hayward   smash@eisa.net.au

Date: Sat, 06 Jun 1998 07:50:07 -0700

From: Tim Sixberry <tsixber@sj.bigger.net>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Organization: Self

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: 6 inch PVC....free

Sender: tesl-err@emachine.com

BPTesla@aol.com wrote:

>

> I have twenty feet of 6" PVC that I need to unload pronto! I wont charge

> anything for the pipe, just pay for the postage and shipping costs.

>

> Just specify the length and I will cut it and ship it to you. I am located in

> Arizona so those of you in this area may benefit well.

>

> I gotta make it disappear!

>

> Send queries to: BPTesla@aol.com

How much do you think the postage would be for a 90" piece to San Jose,

CA? I have no idea.  Maybe more than buying new?

Tim Sixberry

518 Hermitage Place

San Jose, CA 95134

(408)321-0254

X-Mailer: IPAD 2.0

Date: Sat, 06 Jun 1998 12:16:08 -0500

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: McMaster Carr Mica sheets

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

At 06:18 PM 6/5/98 -0500, you wrote:

>I was  interested in the discussion of using the McMaster Carr

>dielectric.  Has anyone been successful using this material.  I had

>planned on making a plate capacitor using this material but shelved

>those plans when I heard about the early failure.  Are there any

>updates?

>

>John

John,

This material is almost certainly reconstituted mica.

That means its made from small pieces of mica imbedded

in a binder. Its similar to the mica sheets you can get

from inside some neon sign transformers.

This stuff really isnt suitable for caps. The breakdown voltage is

low and the dissipation is much higher because of the binder.

Real capacitor grade mica is extremely rare these days and

quite expensive.

You just cant beat polyethylene.

TTYL!

Mike Hammer

mhammer@misslink.net

X-Mailer: IPAD 2.0

Date: Sat, 06 Jun 1998 12:18:51 -0500

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: Chokes

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

At 01:29 PM 6/6/98 +1000, you wrote:

>Sorry, I haven't been on for a while, but I have been reading all of the

>E-mails I have got from the list.  I do worry about some of the things that

>are being done with cats and tesla coils!!!

>

>I have ferrite things that are wound with small gauge magnet wire that were

>part of the yoke assembly on the back of a TV.  I have no idea how much

>inductance there would be but there are many turns of wire and I could link

>about six in series to get the inductance up.  Good idea or Bad?

>

>From Scott Hayward   smash@eisa.net.au

If there are quite a few turns then I would try a single

one on each HV lead. You may however need to remove the

windings and insulate the core a little better and replace the

windings if arcing is a problem.

TTYL!

Mike Hammer

mhammer@misslink.net

Date: Sat, 06 Jun 1998 12:48:02 -0700

From: "William R. Langston" <blangsto@iwvisp.com>

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: 6 inch PVC....free

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Tim,

I have to come to Vaccaville (between Fairfield and Sacramento) later in about

a month. If you can UPS it to an address in Vaccaville, that would work fine.

Possibly, I could come through San Jose and pick it up. That all depends on how

hard your home is to find.

Let me know which you would prefer. I'll check with UPS and see if I can get a

price to pick up and deliver that 90" piece to several places and let you know what

I find out sometime this coming week. Is that ok with you? If you feel the need,

our phone is 760.372.5680.

Our address is:

14037 Pine St.

Trona, CA  93562

Bill Langston

Tim Sixberry wrote:

> BPTesla@aol.com wrote:

> >

> > I have twenty feet of 6" PVC that I need to unload pronto! I wont charge

> > anything for the pipe, just pay for the postage and shipping costs.

> >

> > Just specify the length and I will cut it and ship it to you. I am located in

> > Arizona so those of you in this area may benefit well.

> >

> > I gotta make it disappear!

> >

> > Send queries to: BPTesla@aol.com

>

> How much do you think the postage would be for a 90" piece to San Jose,

> CA? I have no idea.  Maybe more than buying new?

>

> Tim Sixberry

> 518 Hermitage Place

> San Jose, CA 95134

>

> (408)321-0254

Date: Sat, 06 Jun 1998 12:50:46 -0700

From: "William R. Langston" <blangsto@iwvisp.com>

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: 6 inch PVC....free

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Me again (Bill Langston),

I just realized what you asked in your last email. Sorry. This is definitely

not a high IQ day for me. If you send me your address and zip, I'll check with UPS

for a price to our door.

Thanks

Bill Langston

Tim Sixberry wrote:

> BPTesla@aol.com wrote:

> >

> > I have twenty feet of 6" PVC that I need to unload pronto! I wont charge

> > anything for the pipe, just pay for the postage and shipping costs.

> >

> > Just specify the length and I will cut it and ship it to you. I am located in

> > Arizona so those of you in this area may benefit well.

> >

> > I gotta make it disappear!

> >

> > Send queries to: BPTesla@aol.com

>

> How much do you think the postage would be for a 90" piece to San Jose,

> CA? I have no idea.  Maybe more than buying new?

>

> Tim Sixberry

> 518 Hermitage Place

> San Jose, CA 95134

>

> (408)321-0254

From: "Scott Hayward" <smash@eisa.net.au>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Chokes

Date: Sun, 7 Jun 1998 12:13:56 +1000

X-Mailer: IPAD 2.0

X-MimeOLE: Produced By Microsoft MimeOLE V4.71.1712.3

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

>If there are quite a few turns then I would try a single

>one on each HV lead. You may however need to remove the

>windings and insulate the core a little better and replace the

>windings if arcing is a problem.

I don't think that there should be because there is about 8KV going through

them usually, as the voltage for all of the stuff near the cathode ray tube

is about that.

From Scott Hayward  smash@eisa.net.au

X-Mailer: IPAD 2.0

Date: Sun, 07 Jun 1998 00:47:54 -0400

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Jim Ryan <jryan@exis.net>

Subject: Cap Question

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Mike, thanks for the reply about my plans to make a brass shim stock capacitor.

Is this what you meant for a .0134 mfd? 2 of these in series for a .0067 mfd.

____________________________________/lead 	450 SQ. inches .005 shim stock

____________________________________  		.030 poly +

____________________________________  		450 SQ. inches .005 shim stock

\lead

Brass shim 6" wide X 75" long

I plan to roll the above brass with a 2 inch overlap of the poly.

Jim

Date: Sun, 07 Jun 1998 10:21:26 -0700

From: Tim Sixberry <tsixber@sj.bigger.net>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Organization: Self

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: 6 inch PVC....free

Sender: tesl-err@emachine.com

William R. Langston wrote:

>

> Me again (Bill Langston),

>

>     I just realized what you asked in your last email. Sorry. This is definitely

> not a high IQ day for me. If you send me your address and zip, I'll check with UPS

> for a price to our door.

>

> Thanks

>

> Bill Langston

>

> Tim Sixberry wrote:

>

> > BPTesla@aol.com wrote:

> > >

> > > I have twenty feet of 6" PVC that I need to unload pronto! I wont charge

> > > anything for the pipe, just pay for the postage and shipping costs.

> > >

> > > Just specify the length and I will cut it and ship it to you. I am located in

> > > Arizona so those of you in this area may benefit well.

> > >

> > > I gotta make it disappear!

> > >

> > > Send queries to: BPTesla@aol.com

> >

> > How much do you think the postage would be for a 90" piece to San Jose,

> > CA? I have no idea.  Maybe more than buying new?

> >

> > Tim Sixberry

> > 518 Hermitage Place

> > San Jose, CA 95134

> >

> > (408)321-0254

>

> --

> II*

Bill,

I HAVE NO 6" PVC TO SEND YOU. I'm not the guy with the free PVC. I was

interested in getting a 90 inch piece for myself. I did not write the

message above and I do not live in Arisona. You need to Send your

request to BPTesla@aol.com.

Tim Sixberry

From: <Dsurfr@aol.com>

Date: Sun, 7 Jun 1998 16:16:38 EDT

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: SW Cap question

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

I know salt water caps are at the bottom of the list for best output but they

are very inexpensive to make. I'm currently running a 15K 30ma Neon with SW

caps made from 40 oz. bottles wrapped in foil & then wrapped w/ 2 layers of

electrical tape. I'm able to generate 12" sparks but so far this is the max.

My uf calculates out to .0058 but I've never tested it. While I'm saving up to

buy the materials for poly caps could I get better performance from putting

the bottles in a tank of salt water (better surface area contact) or won't I

notice much difference. After all the work to wrap them I don't want to strip

it all off if it won't help much. For what its worth I haven't noticed any

corona from the bottles but like I said they're wrapped in tape.

Thanks, Jim

Date: Sun, 07 Jun 1998 19:10:13 -0500

From: Tedd Payne <tpayne@netnitco.net>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: For Sale:  Small Variac, Small Neon Transformers, Model T Coil

Sender: tesl-err@emachine.com

If interested, please contact me DIRECTLY at tpayne@netnitco.net

Please tell me which one(s) you're interested in, and your ZIP CODE to

determine shipping cost.

Thanks,

Tedd

These might be usefull in a small Tesla coil or other HV project.

"VARIAC"  $20

- Superior Electric Powerstat type 21

- with knob and dial plate marked 0-100

- seems to be in overall good condition

- in = 120VAC, out = 0-140VAC

- 4.5 amps, could probably be run at higher current intermittently

- in = terminals 1 & 2, out = terminals 1 & 3

SMALL NEON TRANSFORMERS

Small and light weight.  Secondaries not connected to

core, so should be able to connect outputs in series or parallel.

$10 used  5kv10ma  in=2 wires out=2 wires  label  OPEN FRAME 5" long

$10 used  3.5kv8ma  in=2 wires out=2 wires  label  OPEN FRAME 4" long

$10 used  3kv8ma  in=2 wires out=2 wires  label  potted (3x2x1.5")

Larger neon transformers also available, ask for list.

NEW Neon transformer HV wire:

Flexible gray, 14 gauge, multistrand copper, rated 15KV, $1.00 per foot.

Standard black, 16 gauge, multistrand copper, rated 15KV, $.50 per foot.

MODEL T SPARK COIL  $20

- original, not reproduction

- copper cover over interrupter end

- spark about 1/4", maybe would be longer with new interrupter set.

- clean, good visual & operating cond.

- operates on 6VDC input

Date: Sun, 07 Jun 1998 20:55:37 -0500

From: Bert Hickman <bert.hickman@aquila.com>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Organization: Stoneridge Engineering

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: first light

Sender: tesl-err@emachine.com

bob golding wrote:

>

> Hi all

>         Fired up the coil for the first time today. 12" sparks straight off from

> the 8" toroid. We then spent some time playing with the tuning and the

> coupling. With 6 turns tapped in  we got sparks all over the toroid without

> any breakout points. the bottom of the secondary was about level with the

> 3rd turn of the primary. We started with one gap on the spark gap but

> quickly moved up to two as the safety gap was not firing and all seemed

> well . This seemed to increase the power ie noise but not the spark length.

> We then tried  the 19" x 4" toroid after re tuning and got 12" sparks with

> breakout points. we used 8 turns on the tap,and kept the coupling the same.

> We then stopped to find some ear protection and to invite the neighbors to

> a show at dusk. The neighbors were impressed or terrified but stayed for

> awhile. After more tuning and opening up the gap to 0.125" we managed 24"

> to a grounded wire. At that point we stopped as it was 11.30 in the

> evening. The coil is hooked up with 18 awg stranded antenna wire. we are

> also using this to the ground. 2 5 foot 3/4" copper pipes 2 foot apart. I

> feel this might be a weak point. would 1/4" copper tube be better as the

> ground connection? I am still not sure about the coupling. with the small

> toriod we can see corona around the top of the seconary but it dissapears

> with the larger toroid. should we fine tune with the tap and then try

> changing the coupling?. Thanks to Mike Hammer, Bert Hickman, Hollmike and

> everybody else for all their help.

>

> bob golding lightning without rain

>

> bob yubba@clara.net

Bob,

Congratulations, and welcome to the communitiy of active coilers, Bob!

Going to 1/4" copper tubing on the primary will further improve

performance - every loss in the system reduces spark length. Using a

heavier ground connection will also help, especially as you begin to

increase the power level. 1/4" copper tubing should work very well for

this connection. I'd go with the larger toroid, since it seems to be

protecting the top turns of the coil. You want to tune for best spark

first, then adjust coupling for best overall performance. If you begin

getting the racing sparks between the top and lower portions of the

secondary, that's a signal that you're overcoupling; raise the secondary

up a bit until it stops and you're there!

Safe coilin' to you, Bob!

-- Bert --

Date: Sun, 07 Jun 1998 21:44:00 -0700

From: Gene Waldo <gwaldo@1stnetusa.com>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Organization: S.A.P.I.E.N.T. (Surly, Agressive, Politically Incorrect Enemies of Non Thinkers)

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: SW Cap question

Sender: tesl-err@emachine.com

Dsurfr@aol.com wrote:

>

> I know salt water caps are at the bottom of the list for best output but they

> are very inexpensive to make. I'm currently running a 15K 30ma Neon with SW

> caps made from 40 oz. bottles wrapped in foil & then wrapped w/ 2 layers of

> electrical tape. I'm able to generate 12" sparks but so far this is the max.

> My uf calculates out to .0058 but I've never tested it.

>    Thanks, Jim

Jim,

We are in the same boat. Plan to go direct to immersed bottles. 40 oz.

also, How many are you using? Big hang up is suitable bottle/bath

contaiment; no plastic boxes seem to fit exactly....! Still looking.

Meanwhile, the poly/pl caps also seeking containment and further eng.

--

Gene Waldo -  gwaldo@1stnetusa.com  - "Igor in Residence"

Date: Sun, 7 Jun 1998 23:47:50 -0500 (CDT)

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: fxphoto@centuryinter.net (fxphoto)

Subject: Re: Cap Question

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Jim,

Your drawing would work for a flat plate cap, however for a rolled plate

cap you will need to add another layer of .030 poly on top (i.e. Poly, shim,

Poly, shim) so when you roll it up the two shims do not come into contact

but are always seperated by .030 poly.

Bill Turbett

>Mike, thanks for the reply about my plans to make a brass shim stock

>capacitor.

>Is this what you meant for a .0134 mfd? 2 of these in series for a .0067 mfd.

>

>____________________________________/lead 	450 SQ. inches .005 shim stock

>____________________________________  		.030 poly +

>____________________________________  		450 SQ. inches .005 shim stock

>                                    \lead

>

>Brass shim 6" wide X 75" long

>

>I plan to roll the above brass with a 2 inch overlap of the poly.

>

>Jim

>

>

Date: Mon, 8 Jun 1998 00:15:43 -0500 (CDT)

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: fxphoto@centuryinter.net (fxphoto)

Subject: Re: SW Cap question

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Jim,

Since you have not tested the capacitance of your bottles, I would suggest

that you try subtracting one bottle, retuning , and comparing your spark output.

If you have an extra bottle, try adding an extra cap, retuning and again

comparing.

If the spark length is increased by adding or subtracting one bottle, you

might try continuing in the direction of the increase.

That is, if adding one more bottle increased the output, try adding two

bottles and retune, and see if there is a further increase.

If there is no difference observed or a decrease is observed in both cases

(adding or subtracting) then your capacitance is about right for that Neon

sign transformer.

Bill Turbett

>I know salt water caps are at the bottom of the list for best output but they

>are very inexpensive to make. I'm currently running a 15K 30ma Neon with SW

>caps made from 40 oz. bottles wrapped in foil & then wrapped w/ 2 layers of

>electrical tape. I'm able to generate 12" sparks but so far this is the max.

>My uf calculates out to .0058 but I've never tested it. While I'm saving up to

>buy the materials for poly caps could I get better performance from putting

>the bottles in a tank of salt water (better surface area contact) or won't I

>notice much difference. After all the work to wrap them I don't want to strip

>it all off if it won't help much. For what its worth I haven't noticed any

>corona from the bottles but like I said they're wrapped in tape.

>   Thanks, Jim

>

>

Date: Mon, 8 Jun 1998 00:22:35 -0500 (CDT)

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: fxphoto@centuryinter.net (fxphoto)

Subject: Re: SW Cap question

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Gene,

Have you tried building a wooden box and lining it with plastic??

I have seen your beautiful work and you probably would not even need to

line the box with plastic as your joints would fit perfectly!

Bill Turbett

>Dsurfr@aol.com wrote:

>>

>> I know salt water caps are at the bottom of the list for best output but they

>> are very inexpensive to make. I'm currently running a 15K 30ma Neon with SW

>> caps made from 40 oz. bottles wrapped in foil & then wrapped w/ 2 layers of

>> electrical tape. I'm able to generate 12" sparks but so far this is the max.

>> My uf calculates out to .0058 but I've never tested it.

>>    Thanks, Jim

>Jim,

>We are in the same boat. Plan to go direct to immersed bottles. 40 oz.

>also, How many are you using? Big hang up is suitable bottle/bath

>contaiment; no plastic boxes seem to fit exactly....! Still looking.

>Meanwhile, the poly/pl caps also seeking containment and further eng.

>--

>Gene Waldo -  gwaldo@1stnetusa.com  - "Igor in Residence"

X-Mailer: IPAD 2.0

Date: Mon, 08 Jun 1998 00:59:58 -0700

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: bob golding <yubba@clara.net>

Subject: first light

Cc: tesla-2@emachine.com

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Hi all

Fired up the coil for the first time today. 12" sparks straight off from

the 8" toroid. We then spent some time playing with the tuning and the

coupling. With 6 turns tapped in  we got sparks all over the toroid without

any breakout points. the bottom of the secondary was about level with the

3rd turn of the primary. We started with one gap on the spark gap but

quickly moved up to two as the safety gap was not firing and all seemed

well . This seemed to increase the power ie noise but not the spark length.

We then tried  the 19" x 4" toroid after re tuning and got 12" sparks with

breakout points. we used 8 turns on the tap,and kept the coupling the same.

We then stopped to find some ear protection and to invite the neighbors to

a show at dusk. The neighbors were impressed or terrified but stayed for

awhile. After more tuning and opening up the gap to 0.125" we managed 24"

to a grounded wire. At that point we stopped as it was 11.30 in the

evening. The coil is hooked up with 18 awg stranded antenna wire. we are

also using this to the ground. 2 5 foot 3/4" copper pipes 2 foot apart. I

feel this might be a weak point. would 1/4" copper tube be better as the

ground connection? I am still not sure about the coupling. with the small

toriod we can see corona around the top of the seconary but it dissapears

with the larger toroid. should we fine tune with the tap and then try

changing the coupling?. Thanks to Mike Hammer, Bert Hickman, Hollmike and

everybody else for all their help.

bob golding lightning without rain

bob yubba@clara.net

From: <Dsurfr@aol.com>

Date: Mon, 8 Jun 1998 17:38:47 EDT

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: SW Cap question

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Gene: Per the calculations I ran I should be getting about .00098uf per bottle

so right now I'm using 6 bottles for a total of .0058uf. I'm using a 15K 30ma

Neon & the calculations say I should have about .0053uf for a match. I'm using

10" long x 3/8" carriage bolts for electrodes & have the salt water 7" high.

I'm going to make a couple more & see if the results improve at all. I'm also

going to check on assorted tupperware type containers as well as aquariums for

a tank but it seems like wood sealed with epoxy or silicone might work as

well. I'll let you know.

Jim

Date: Mon, 08 Jun 1998 15:17:58 -0700

From: Gene Waldo <gwaldo@1stnetusa.com>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Organization: S.A.P.I.E.N.T. (Surly, Agressive, Politically Incorrect Enemies of Non Thinkers)

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: SW Cap question

Sender: tesl-err@emachine.com

Dsurfr@aol.com wrote:

>

> Gene: Per the calculations I ran I should be getting about .00098uf per bottle

> so right now I'm using 6 bottles for a total of .0058uf.

I'll let you know.

> Jim

Jim,

Don't know if you caught my literal 'pipe dream' (once you've committed

to 'wet bottles'), which consists of a 'siphon' arrangement, whereby the

outer solution may be raised or lowered via a tube to a carboy on a

rope. One should be able thus to calibrate a 'variable capacitor', and

'tune on the fly', with comparative remote ease and safety.

This has the 'potential' be be both dangerous and messy, but as a

development tool it may have some value; due precautions should be

taken!   No one has yet made any comment on the pure electrical aspects

of this approach.

--------------------------------------------

Gene Waldo -  gwaldo@1stnetusa.com  - "Igor in Residence"

From: <Dsurfr@aol.com>

Date: Mon, 8 Jun 1998 19:29:06 EDT

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: SW Cap question

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Gene: Interesting idea! Back when I had tropical fish there was a hook up to

siphon the water through a filter using a small air pump. You could hook one

up to move water into the tank & one to take it out without a lot of mess (not

sure how the oil on the top of the water will react?) but it does seem like

this would let you alter the cap value "on the fly" Also got a good response

from "graphix @iastate with a web site showing his Mountain Dew bottle caps in

aquariums that showed great results from SW's. Might be worth a try. Will get

back.

Jim

Date: Mon, 08 Jun 1998 20:26:10 -0700

From: Gene Waldo <gwaldo@1stnetusa.com>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Organization: S.A.P.I.E.N.T. (Surly, Agressive, Politically Incorrect Enemies of Non Thinkers)

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: SW Cap question

Sender: tesl-err@emachine.com

Dsurfr@aol.com wrote:

>

> Gene: Interesting idea! Back when I had tropical fish there was a hook up to

> siphon the water through a filter using a small air pump.  Will get

> back.

> Jim

Jim,

Forget pumps!!!

Too much electrical involvement! Remember, the bath electrode will be

extended into the 'ullage' container! Simple gravity system! Isolated by

nylon line and suspension/take-up gear.

The oil top will have to be managed---  But I see no real sweat here,

with a bit of logically applied logistics.

Granted, the capacitor will be 'immense', and may not be feasable in

some situations, but if the installation is suited to your site, it

should lessen one of the tuning variables.

My simple fluid handling design approach was developed years ago for

tests in aeropace and worked well; all that is required now is to

insulate the ullage container/fluid level controls. At least we only

working saltwater, not flamables, cryogenics and peroxides!

Do stay in touch!

With me, not the bath electrode!

-----------------------------------------

Gene Waldo -  gwaldo@1stnetusa.com  - "Igor in Residence"

---------------------------------------------------------------

What's good for the planet is more OZONE! See the machinery form up!

NEW PAGE!   http://members.1stnetusa.com/danmurphy/waldo.htm

Date: Tue, 9 Jun 1998 04:58:18 -0400 (EDT)

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: fxphoto@centuryinter.net (fxphoto)

Subject: Re: SW Cap question

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Gene and Jim,

If you were to remove the bottom from a bottle and glue/mount it to the

inside of your tank, you could use air pressure to raise and lower the fluid

level.

Removing the bottom of the bottle allows you to put a floating rubber ball

in it as a safety valve to prevent fluid from going into the air line. (it

should seal nicely in the neck area)  The benefit of this is that all fluid

stays in the main containment. It is a simpler arrangement. If you use an

air pump, it is electrically insulated from the main tank.

Bill Turbett

>Dsurfr@aol.com wrote:

>>

>> Gene: Interesting idea! Back when I had tropical fish there was a hook up to

>> siphon the water through a filter using a small air pump.  Will get

>> back.

>> Jim

>

>Jim,

>Forget pumps!!!

>Too much electrical involvement! Remember, the bath electrode will be

>extended into the 'ullage' container! Simple gravity system! Isolated by

>nylon line and suspension/take-up gear.

>The oil top will have to be managed---  But I see no real sweat here,

>with a bit of logically applied logistics.

>Granted, the capacitor will be 'immense', and may not be feasable in

>some situations, but if the installation is suited to your site, it

>should lessen one of the tuning variables.

>My simple fluid handling design approach was developed years ago for

>tests in aeropace and worked well; all that is required now is to

>insulate the ullage container/fluid level controls. At least we only

>working saltwater, not flamables, cryogenics and peroxides!

>Do stay in touch!

>With me, not the bath electrode!

>-----------------------------------------

>Gene Waldo -  gwaldo@1stnetusa.com  - "Igor in Residence"

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Date: Tue, 09 Jun 1998 04:30:19 -0700

From: "Leben Lang und Frei" <fleischwulf@mailexcite.com>

Cc:

X-Sent-Mail: off

X-Expiredinmiddle: true

X-Mailer: IPAD 2.0

Subject: New experiment

X-Sender-Ip: 208.153.7.9

Organization: MailExcite  (http://www.mailexcite.com:80)

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

I have one more experiment to try. I'm going to reduce the input voltage to 8 Kv - 10 Kv. and increase the capacitance to .08 - .1 uf. I have tried running this coil at 14.4 Kv / 7.2 Kw, this resulted in over coupling an near incineration. I hope that by reducing the input voltage, increasing the tank circuit  capacitance and top load I'll be able to increase the output. Has one out there in Tesla land ever tried any thing like this?

Well I just have to keep trying new stuff. I'll be building a new 7" coil soon.

Thanks for all the help and info

Paul M.

---

Einigkeit und Freiheit

From: "Thomas McGahee" <tom_mcgahee@sigmais.com>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Variable SW Cap

Date: Tue, 9 Jun 1998 13:14:29 -0400

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Gene,

In my earlier response to your letter on siphoning to raise/lower

SW cap SW level I mentioned:

> Be aware that the caps will undergo more strain when the

>water level is lower.

>

>The saltwater, of course, conducts electricity, so be aware of the fact

>that the bucket will have several thousand volts "at your finger tips".

>Be careful, and make sure that whatever you do you have safety in mind!

There is also ANOTHER method for raising/lowering the fluid level that

does NOT involve siphoning at all. If an object such as a brick is slowly

introduced to the fluid bath, the level will rise. Any suitable object

may be lowered and raised into the tub to cause the fluid level to

be adjusted where desired. This might be safer than the siphon method.

The only requirement is that the item being dunked is denser than the

saltwater. Use insulating material for the line!

BTW, if using the raising/lowering method outlined below, the size of the

container being raised and lowered is important. A tall, thin container

will give you finer control than a wide container. The volume of water

transferred per inch of raising/lowering is dependent on the volume

per inch of height of the container.

Hope this helps. Fr. Tom McGahee

----------

> From: Gene Waldo <gwaldo@1stnetusa.com>

> To: Tesla-2 Mailing List <tesla-2@emachine.com>

> Subject: Re: SW Cap question

> Date: Monday, June 08, 1998 6:17 PM

>

> Dsurfr@aol.com wrote:

> >

> > Gene: Per the calculations I ran I should be getting about .00098uf per bottle

> > so right now I'm using 6 bottles for a total of .0058uf.

>  I'll let you know.

> > Jim

>

> Jim,

> Don't know if you caught my literal 'pipe dream' (once you've committed

> to 'wet bottles'), which consists of a 'siphon' arrangement, whereby the

> outer solution may be raised or lowered via a tube to a carboy on a

> rope. One should be able thus to calibrate a 'variable capacitor', and

> 'tune on the fly', with comparative remote ease and safety.

> This has the 'potential' be be both dangerous and messy, but as a

> development tool it may have some value; due precautions should be

> taken!   No one has yet made any comment on the pure electrical aspects

> of this approach.

>

> --------------------------------------------

> Gene Waldo -  gwaldo@1stnetusa.com  - "Igor in Residence"

> From: Gene Waldo <gwaldo@1stnetusa.com>

> Date: Fri, 20 Mar 1998 17:02:53 -0800

> Subject: Re: Bottle Cap outer electrode??

> Organization: Self

>

Snippidity Do Dah!

>

> My next query is about TUNABLE SW cap?  VARY the level of SW! Simple

> 'siphon' tube arrangement from bottom of Cap tub to carboy on pulley...

> ??

>

> I'll have the beer consumed (40 oz. BUD) long before we complete the

> plate caps. 18 bottles w/~90^2" ea. of foil/immersion should give enough

> capacitance to do something.

>

> I've not heard of tuning the cap w/this method, and have no seat of the

> pants feel for its effectiveness. Comments?

>

> Gene Waldo

>

Gene,

Immersing the saltwater caps as Mike Hammer mentioned is the way to go.

Immersion cuts down on corona loss and resuces heating, and absolutely

guarantees that you will get the maximum capacitance possible out

of your SW caps. Be aware of the fact that aluminum foil on the outside

of a SW cap ALWAYS has wrinkles and therefore air gaps. That is where

the corona builds up and causes pinpoint heating that leads to glass

stress and eventual failure. Immersion gets past all that. The downside

is the need for an insulated container, but that is a small price to pay.

You can connect SW caps in parallel by just immersing them in the same

tub and connecting the tops together.

If you want them in series, then you need separate tubs, ***OR***

you can get exactly TWO sets in series per tub by connecting the tops of

one half of the bottles together and calling that electrode (A), and then

doing the same with the others and calling that electrode (B). The salt

water in the tub will interconnect the two sets for you (no electrode

needed here...), and you use (A) and (B) as your capacitor terminals.

Simple and effective.

You asked about raising and lowering the salt water level. While

siphoning will work, it has some problems. Once air gets in the line

the siphoning action stops. A better method is to attach a hose near

the base of the tub and the base of another container, such as a 5 gallon

polyethelene bucket. There are plastic fittings you can buy at a hardware

store that will give you a nipple to attach a hose between the two

containers. Raising or lowering the bucket will cause the salt water level

in the tub to go up or down as desired, adjusting the capacitance over a

wide range.  Be aware that the caps will undergo more strain when the

water level is lower.

The saltwater, of course, conducts electricity, so be aware of the fact

that the bucket will have several thousand volts "at your finger tips".

Be careful, and make sure that whatever you do you have safety in mind!

Hope this helps.

Fr. Tom McGahee

X-Mailer: IPAD 2.0

Date: Tue, 09 Jun 1998 18:46:09 -0400

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Bob <tesla@america.com>

Subject: My site is terminal :(

Cc: tesla-2@emachine.com,USA-TESLA@list.iex.net

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Howdy Coilers,

I have always tried to keep my page going even though

I did not always have time to update it. I also like my email

address of 'tesla@america.com', which was easy to remember

for people. Well I had paid for access for the year and it appears

that my provider is folding. I still see that mail comes to me and

that the web site is still going but I believe it to be a matter of time

before they are both gone, which will include my connection to

the Tesla lists. I downloaded all of my web directory so in the

future it may one day arise again. This is sad as well because I

had just scanned the pictures of my newest static gap of which

I wanted to share. I am gun shy of providers now because I would

not want the same thing to happen again and would want a provider

who is stable and not going anywhere. Anyway, it had been fun and

a great experience and thank you all, I will read email from list as long

as I can and keep the page up until they pull the plug.

Take care all and happy coiling!!

Bob Schumann

tesla@america.com

http://www.america.com/~tesla

X-Mailer: IPAD 2.0

Date: Tue, 09 Jun 1998 19:28:34 -0400

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Jim Ryan <jryan@exis.net>

Subject: Re: Cap Question

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

At 11:47 PM 6/7/98 -0500, you wrote:

> Jim,

>  Your drawing would work for a flat plate cap, however for a rolled plate

>cap you will need to add another layer of .030 poly on top (i.e. Poly, shim,

>Poly, shim) so when you roll it up the two shims do not come into contact

>but are always seperated by .030 poly.

>                               Bill Turbett

>

>>Mike, thanks for the reply about my plans to make a brass shim stock capacitor.

>>Is this what you meant for a .0134 mfd? 2 of these in series for a .0067 mfd.

>>

>>  ____________________________________/lead 	450 SQ. inches .005 shim stock

>>_________________________________________  		.030 poly +

>>  ____________________________________  		450 SQ. inches .005 shim stock

>>                                    \lead

>>

Yep. you are correct. Do you figure 450 SQ. IN. of plate area per rolled cap

is close for a .0066 mfd?

Thanks

Jim Ryan

From: ralph.hartwell@emachine.com (Ralph Hartwell)

Subject:  My site is terminal :(

Date: Wed, 10 Jun 1998 02:59:00 GMT

Organization: The Energy Machine Information System 504-733-3462

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

B> I have always tried to keep my page going even though

B>for people. Well I had paid for access for the year and it appears

B>that my provider is folding. I still see that mail comes to me and

B>a great experience and thank you all, I will read email from list as long

B>as I can and keep the page up until they pull the plug.

Bob,

Are you in a position to obtain internet access for free from any

local source?  If so, Geocities.com has free web space available, as do

other providers.  I use AT&T for some of my email and my web site.  They

have been quite reliable during the time I have used them.  They

currently offer 5 mB of web space to subscribers.  Good luck!

Ralph

---

� QMPro 1.53 � My apartment allows pets.  I have a pony.

Date: Tue, 09 Jun 1998 20:18:56 -0700

From: Gene Waldo <gwaldo@1stnetusa.com>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Organization: S.A.P.I.E.N.T. (Surly, Agressive, Politically Incorrect Enemies of Non Thinkers)

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

CC: TESLA-2@emachine.com

Subject: Re: Variable SW Cap

Sender: tesl-err@emachine.com

Dsurfr@aol.com wrote:

>

> Gene: I went down to the local tropical fish shop & asked about making tanks

> to specific sizes. & then try making a tank to size myself w/ the possibility

> of altering the water level built into the design.

Jim,

I would exhaust the industrial tote box avenue before constructing my

own container!

> I'm not too sure about the importance of the oil on top of the water.

> Seems like there must be other ways to reduce the corona effect.

I can't immagine a better or cheaper way.... and Mike and others

proclaim the virtues.

Did you get the posts from Fr. Tom McGahee?

> Will keep in touch,   Jim Sheen (dsurfr)

----------------------------------------------------

Gene Waldo -  gwaldo@1stnetusa.com  - "Igor in Residence"

Date: Tue, 09 Jun 1998 21:46:17 -0700

From: mike nicholas <705728@ican.net>

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: interference

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Its very likely that your lack of filters on the 115V side is causing emi to go

into the house wiring and possibly back outside the house wiring, and any

length of wire might act as an antena.

bob golding wrote:

> >Date: Wed, 10 Jun 1998 00:06:02 -0700

> >To: tesla@pupman.com

> >From: bob golding <yubba@clara.net>

> >Subject: interference

> >

> >hi all

> >       The nearest neighbor came and told us we are causing interference on

> their TV and cordless phone. We are in a brick built garage with a 20 foot

> or so overhead extension cable which is running next to the TV aerial. We

> are not using any chokes or emi filters yet. Could the lack of chokes  be

> causing the interference?  We have 2 6 amp emi filters which we can use in

> parallel if needed. Would moving the power lead down to ground level

> putting it in conduit and rerouting it away from the TV aerial help? Maybe

> running at other than at prime time TV time might help. Like 2 am.

> >many thanks.

> >

> >bob golding lightning without rain

> >

> >bob yubba.clara.net

X-Mailer: IPAD 2.0

Date: Wed, 10 Jun 1998 00:07:19 -0700

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: bob golding <yubba@clara.net>

Subject: interference

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

>Date: Wed, 10 Jun 1998 00:06:02 -0700

>To: tesla@pupman.com

>From: bob golding <yubba@clara.net>

>Subject: interference

>

>hi all

>	The nearest neighbor came and told us we are causing interference on

their TV and cordless phone. We are in a brick built garage with a 20 foot

or so overhead extension cable which is running next to the TV aerial. We

are not using any chokes or emi filters yet. Could the lack of chokes  be

causing the interference?  We have 2 6 amp emi filters which we can use in

parallel if needed. Would moving the power lead down to ground level

putting it in conduit and rerouting it away from the TV aerial help? Maybe

running at other than at prime time TV time might help. Like 2 am.

>many thanks.

>

>bob golding lightning without rain

>

>bob yubba.clara.net

From: "Scott Hayward" <smash@eisa.net.au>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: New job

Date: Wed, 10 Jun 1998 18:21:04 +1000

X-Mailer: IPAD 2.0

X-MimeOLE: Produced By Microsoft MimeOLE V4.71.1712.3

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

I just got a new job!!!! So you should be hearing from me again in a couple

of weeks once I've finally got some more money and time to do some work on

my little 4" coil.  Needs a toroid for the top and a decent pirmary and

ground.  I doubt it, but I might even make a new gap and caps for it.

Depends on how the funding goes.

From Scott Hayward  smash@eisa.net.au

"Double your hard drive sapce...........Delete Win 95"

(he he he he  I love bagging microsoft)

Date: Wed, 10 Jun 1998 07:01:27 -0500

From: Bert Hickman <bert.hickman@aquila.com>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Organization: Stoneridge Engineering

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: interference

Sender: tesl-err@emachine.com

bob golding wrote:

>

> >Date: Wed, 10 Jun 1998 00:06:02 -0700

> >To: tesla@pupman.com

> >From: bob golding <yubba@clara.net>

> >Subject: interference

> >

> >hi all

> >       The nearest neighbor came and told us we are causing interference on

> their TV and cordless phone. We are in a brick built garage with a 20 foot

> or so overhead extension cable which is running next to the TV aerial. We

> are not using any chokes or emi filters yet. Could the lack of chokes  be

> causing the interference?  We have 2 6 amp emi filters which we can use in

> parallel if needed. Would moving the power lead down to ground level

> putting it in conduit and rerouting it away from the TV aerial help? Maybe

> running at other than at prime time TV time might help. Like 2 am.

> >many thanks.

> >

> >bob golding lightning without rain

> >

> >bob yubba.clara.net

Bob,

Adding the EMI filters will help as well as rerouting the power cord.

Putting the cord in ground-level, grounded conduit would be even better.

However, the spark gap and coil streamers also generate a fair amount of

higher frequency RF interference which can be quite difficult to

suppress. Try the first two measures and let us know how they worked.

-- Bert --

Date: Wed, 10 Jun 1998 08:37:00 -0400 (EDT)

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: fxphoto@centuryinter.net (fxphoto)

Subject: Re: Cap Question

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

>At 11:47 PM 6/7/98 -0500, you wrote:

>> Jim,

>>  Your drawing would work for a flat plate cap, however for a rolled plate

>>cap you will need to add another layer of .030 poly on top (i.e. Poly, shim,

>>Poly, shim) so when you roll it up the two shims do not come into contact

>>but are always seperated by .030 poly.

>>                               Bill Turbett

>>

>>>Mike, thanks for the reply about my plans to make a brass shim stock capacitor.

>>>Is this what you meant for a .0134 mfd? 2 of these in series for a .0067 mfd.

>>>

>>>  ____________________________________/lead 	450 SQ. inches .005 shim stock

>>>_________________________________________  		.030 poly +

>>>  ____________________________________  		450 SQ. inches .005 shim stock

>>>                                    \lead

>>>

>Yep. you are correct. Do you figure 450 SQ. IN. of plate area per rolled cap

>is close for a .0066 mfd?

>

>Thanks

>Jim Ryan

>

Yes Jim,

It will be close if you make two of them and connect them in series.

Bill Turbett

Date: Wed, 10 Jun 1998 07:10:11 -0700 (PDT)

From: Chris Ringwood <ccntesla@yahoo.com>

Subject: 28Kv 0.0011uF Caps & table top coil.

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

I have recently got hold of two 0.0011uF 28Kv caps, and was wondering if

two of them would be enough for a small table top coil I'm planning to build?

Also, It says on them 'Test Conditions 6Amps at 300KCS for 1 Hour'.

What does this mean exactly?

If u can help I would appreciate it.

Chris Ringwood, UK.

Date: Wed, 10 Jun 1998 19:14:30 -0500

From: Bert Hickman <bert.hickman@aquila.com>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Organization: Stoneridge Engineering

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: 28Kv 0.0011uF Caps & table top coil.

Sender: tesl-err@emachine.com

Chris Ringwood wrote:

>

> I have recently got hold of two 0.0011uF 28Kv caps, and was wondering

> if two of them would be enough for a small table top coil I'm planning

> 2 build?

>

> Also, It says on them 'Test Conditions 6Amps at 300KCS for 1 Hour'.

> What does this mean exactly?

>

> If u can help I would appreciate it.

>

> Chris Ringwood, UK.

>

Chris,

From the specs, these sound very much like mica transmitting caps. These

are meant for use in CW transmitters, and because they are mica

dielectric, can take a real pounding as long as you don't over-volt

them. These should work fine in a small system driven off no more than 9

kVAC, and higher if you connect them in series. The ratings are in peak

volts, and the amperage rating is RMS amps for CW use. Nice catch!

The test conditions mean that they didn't overheat after 1 hour of

passing 6A of continuous RF current through them. They'll heat up a bit

under extended use in either Tesla Coil tube or spark excited

applications, but mica caps aren't nearly as heat-sensitive as plastic

film caps.

-- Bert --

X-Originating-IP: [208.23.200.119]

From: "Bill the arcstarter" <arcstarter@hotmail.com>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Solenoid primary?

Date: Wed, 10 Jun 1998 17:16:21 PDT

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

Coilers,

I've noticed that Greg L's humongous 130 Kva Electrum coil contains a

solenoid-type primary physically located inside the secondary.

http://www.lod.org/electrum.html

Has anyone every tried arranging a small tesla coil (4 inch diam

secondary or therabouts) in this fashion?

Just wondering (there goes the neighborhood).

-Bill

Date: Wed, 10 Jun 1998 21:01:52 -0700

From: Gene Waldo <gwaldo@1stnetusa.com>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Organization: S.A.P.I.E.N.T. (Surly, Agressive, Politically Incorrect Enemies of Non Thinkers)

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Solenoid primary?

Sender: tesl-err@emachine.com

Bill the arcstarter wrote:

>

> Coilers,

>  I've noticed that Greg L's humongous 130 Kva Electrum coil contains a

> solenoid-type primary physically located inside the secondary.

>

> http://www.lod.org/electrum.html

>

>  Has anyone every tried arranging a small tesla coil (4 inch diam

> secondary or therabouts) in this fashion?

>

>  Just wondering (there goes the neighborhood).

>

> -Bill

Bill,

Methinks we get problems of scale--and coupling...  I've developed a

simple jig system for winding 1/4" Cu tube (solenoid) coils: 2.5-5" od.

1 leg must be central to the winding; don't know if this is a prob.

The problem I see is pure separation between the Pri & 2ndary, and

correct coupling.

LOD has PLAY SPACE! And factors of scale which a 4" 2ndary form jes'

don't allow!

I would venture that a 12" dia. 2ndary would be needed before internal pri

could become feasable...  PURE GUESS!

Anyone care to 2nd guess?

-------------------------------------------

Gene Waldo -  gwaldo@1stnetusa.com  - "Igor in Residence"

Date: Wed, 10 Jun 1998 21:51:55 -0700

From: Michael Loftus <loftusm@ix.netcom.com>

Organization: Me

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: interference

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Bert Hickman wrote:

>

> bob golding wrote:

> >

> > >Date: Wed, 10 Jun 1998 00:06:02 -0700

> > >To: tesla@pupman.com

> > >From: bob golding <yubba@clara.net>

> > >Subject: interference

> > >

> > >hi all

> > >       The nearest neighbor came and told us we are causing interference on

> > their TV and cordless phone. We are in a brick built garage with a 20 foot

> > or so overhead extension cable which is running next to the TV aerial. We

> > are not using any chokes or emi filters yet. Could the lack of chokes  be

> > causing the interference?  We have 2 6 amp emi filters which we can use in

> > parallel if needed. Would moving the power lead down to ground level

> > putting it in conduit and rerouting it away from the TV aerial help? Maybe

> > running at other than at prime time TV time might help. Like 2 am.

> > >many thanks.

> > >

> > >bob golding lightning without rain

> > >

> > >bob yubba.clara.net

>

> Bob,

>

> Adding the EMI filters will help as well as rerouting the power cord.

> Putting the cord in ground-level, grounded conduit would be even better.

> However, the spark gap and coil streamers also generate a fair amount of

> higher frequency RF interference which can be quite difficult to

> suppress. Try the first two measures and let us know how they worked.

>

> -- Bert --

Hi,

I have a related sad story.  I fired-up my first TC with a very expensive

B&O stereo plugged-in on the other side of the garage wall. I had a poor

set-up. . . Long story short, I sent it out for repairs and expect to

spend many $$$.

Mike

X-Mailer: IPAD 2.0

Date: Thu, 11 Jun 1998 09:53:06 +0300

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Marco Denicolai <marco@vistacom.fi>

Subject: Experiments with a MOT tank supply

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

My tank supply is made of 2 microwave oven transformers and voltage doubler

that make up a 12KV DC pulsed at 50 Hz. The MOTs are capable of outputting

up to 0.5 A.

When I first connected a simple steel bolt spark gap (no primary cap, primary,

nor compressed air quench) I got about 1 cm spark and could keep it on as long

as I liked (1-2 minutes). Never blown a fuse.

Then I built a RQ static gap, used 7 gaps (the max number I could get a spark

with) and in each run I ALWAYS blew a 10A fuse in about 5 seconds. Again,

note that I have no primary cap, no primary coil but I do have 3 mH and

550 pF filters between the gap and the tank supply.

Why this?

I explain it like this: the bolt gap was quenching poorly and the supply

(almost shorted) current limiting stabilized on about 0.5 A current. Almost

no current spikes. The RQ gap quenches better, the supply is pulsed shorted,

I get higher current spikes (the current limiting cannot be so fast to

avoid them) that in a matter of seconds warm and blow my 10A fuse.

This makes me to believe that in the complete TC (using the primary

capacitor) I MUST disconnect the tank supply before discharging the primary

cap through the primary, otherwise again I'll blow a fuse.

MY IDEA:

========

Build a rotary spark gap where the rotating sections will FIRST connect the

primary capacitor to the tank supply and THEN connect it to the primary.

--------------

rotating metal section      ---> |            | ---> movement direction

--------------

||        ||        ||  3 electrodes

||        ||        ||

|         |         |

to tank     to cap    to primary

So I'll get first a spark between the tank and the cap, that will charge it.

Then, when the rotating section moves forward, I'll get a second spark

between the cap and the primary coil, that will discharge the cap and will

not short the tank supply.

By having N metal sections on a rotating disk I can have N discharges for a

50 Hz cycle (supposing a 3000 RPM motor). The distance between the electrodes

and the rotating disk will set the minimum trip voltage.

For instance, with a 7 cm diameter disk, supposing a usable electrod-section-

electrod lenght of 1 cm (that is, the lenght the two electrods are shorted)

and 3000 rpm I can get 1 msec available time for the charge and 1 msec

available for the discharge (roughly).

Has anybody tried this before? Comments and/or suggestions?

Marco Denicolai

X-Mailer: IPAD 2.0

Date: Thu, 11 Jun 1998 01:34:48 -0700

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: bob golding <yubba@clara.net>

Subject: cap  problems

Cc: tesla-2@emachine.com

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

hi all

something very strange is happening to my cap. I removed it from the box

it was sitting in as the oil was creeping up the side of the box and

leaking over the edge. When I took the cap out I noticed that there was a

large bulge on one side and the acrylic sheets holding it together had all

cracked and broken away. It looked as if the whole cap had expanded. It was

still working ok last night so I don't think it is damaged. I have left it

sitting in another container for now and intend to dismantle it next week

some time. Does anyone know why this is happening? The cap is made up of 6

layers of polypropylene sheets between standard cooking foil. it doesnt look

as if the oil has attacked the plastic. It has attacked the tape I used to

pull it all together. I am using shell diala bg tranformer oil. many thanks

bob golding lightning without rain

bob yubba@clara.net

Date: Thu, 11 Jun 1998 06:57:11 -0500

From: Bert Hickman <bert.hickman@aquila.com>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Organization: Stoneridge Engineering

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: cap  problems

Sender: tesl-err@emachine.com

bob golding wrote:

>

> hi all

>         something very strange is happening to my cap. I removed it from the box

> it was sitting in as the oil was creeping up the side of the box and

> leaking over the edge. When I took the cap out I noticed that there was a

> large bulge on one side and the acrylic sheets holding it together had all

> cracked and broken away. It looked as if the whole cap had expanded. It was

> still working ok last night so I don't think it is damaged. I have left it

> sitting in another container for now and intend to dismantle it next week

> some time. Does anyone know why this is happening? The cap is made up of 6

> layers of polypropylene sheets between standard cooking foil. it doent look

> as if the oil has attacked the plastic. It has attacked the tape I used to

> pull it all together. I am using shell diala bg tranformer oil. many thanks

>

> bob golding lightning without rain

>

> bob yubba@clara.net

Bob,

Polyethylene DOES swell a bit when immersed in mineral oil, since it

absorbs a bit of the fluid. This is not, generally, a problem unless the

outside container cannot withstand the slightly increased size of the

capacitor stack.

-- Bert --

From: "Eriksson" <jan-erik.eriksson@mbox2.swipnet.se>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: cap formula

Date: Thu, 11 Jun 1998 17:24:50 +0200

X-Mailer: IPAD 2.0

X-MimeOLE: Produced By Microsoft MimeOLE V4.72.2106.4

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Can smomebody please send me the capacitor formula, to calculate a homebrew plate cap.

Thanks,

/Christopher Eriksson

X-Mailer: IPAD 2.0

Date: Thu, 11 Jun 1998 11:22:50 -0500

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: cap formula

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

At 05:24 PM 6/11/98 +0200, you wrote:

>Can smomebody please send me the capacitor formula, to calculate a homebrew plate cap.

>Thanks,

>/Christopher Eriksson

>

Here ya go Chris.

Capacitance (in picofarads) = (0.224 KA / d) (N-1)

0.224 x Dielectric Constant x Area of plate

C = -----------------------------------------  x (Number of plates - 1)

distance between plates in inches

NOTE: For two plate rolled capacitors the value derived from this

formula must be multiplied by two, or, the value must be computed

with the number of plates (N) equal to three (3), so (N-1 = 2)

TTYL!

Mike Hammer

mhammer@misslink.net

X-Mailer: IPAD 2.0

Date: Thu, 11 Jun 1998 11:38:59 -0500

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Mike Hammer <mhammer@misslink.net>

Subject: things getting better

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

All,

The computer system is coming along after my rash

of failures. The hard drive was still under warranty

and was replaced quickly by Western Digital. If you

have a Western Digital drive download their dignostic

program. They take the results of that diagnostic very

seriously. In my case the diagnostic reported too many bad

sectors and that the drive should be replaced. When I told

tech support the fault codes they didnt even want to discuss

the old drive anymore. They shipped me a brand new drive FEDEX.

The diagnostic program can be found at: http://www.wdc.com/support/diagnostics.html

The new tape drive is now working fine. The IDE cable

supplied with the drive was defective. However HP/Colorado

tech support blamed everything else under the sun for the problem.

They finally washed their hands of the deal saying that my bios

must of been at fault. So much for tech support.

Im still using my old flakey 14 inch monitor after my good

monitor failed. I now am running it in 800X600 mode but its

a bit flickery at 60HZ. But at least its usable.

Now that I got some free HD space once again and at least

partially operational I'll try to catch up on the backlog

of mail. It will take quite a while to recover from the 750 Meg

data loss. Many thanks to those who wrote offering moral

support and/or hardware.

TTYL!

Mike Hammer

mhammer@misslink.net

Date: Thu, 11 Jun 1998 09:48:00 -0700

From: Gene Waldo <gwaldo@1stnetusa.com>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Organization: S.A.P.I.E.N.T. (Surly, Agressive, Politically Incorrect Enemies of Non Thinkers)

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Experiments with a MOT tank supply

Sender: tesl-err@emachine.com

Marco Denicolai wrote:

>

> My tank supply is made of 2 microwave oven transformers and voltage doubler

> that make up a 12KV DC pulsed at 50 Hz. The MOTs are capable of outputting

> up to 0.5 A.

>

> When I first connected a simple steel bolt spark gap (no primary cap,

> primary, nor compressed air quench) I got about 1 cm spark and could keep

> it on as long as I liked (1-2 minutes). Never blown a fuse.

>

> Then I built a RQ static gap, used 7 gaps (the max number I could get a

> spark with) and in each run I ALWAYS blew a 10A fuse in about 5 seconds.

> Again, note that I have no primary cap, no primary coil but I do have 3 mH

> and 550 pF filters between the gap and the tank supply.

>

> Why this?

>

> I explain it like this: the bolt gap was quenching poorly and the supply

> (almost shorted) current limiting stabilized on about 0.5 A current. Almost

> no current spikes. The RQ gap quenches better, the supply is pulsed

> shorted, I get higher current spikes (the current limiting cannot be so

> fast to avoid them) that in a matter of seconds warm and blow my 10A fuse.

>

> This makes me to believe that in the complete TC (using the primary

> capacitor) I MUST disconnect the tank supply before discharging the primary

> cap through the primary, otherwise again I'll blow a fuse.

>

> MY IDEA:

> ========

>

> Build a rotary spark gap where the rotating sections will FIRST connect the

> primary capacitor to the tank supply and THEN connect it to the primary.

>

>                                  --------------

> rotating metal section      ---> |            | ---> movement direction

>                                  --------------

>                                  ||        ||        ||  3 electrodes

>                                  ||        ||        ||

>                                  |         |         |

>                               to tank     to cap    to primary

>

> So I'll get first a spark between the tank and the cap, that will charge it.

> Then, when the rotating section moves forward, I'll get a second spark

> between the cap and the primary coil, that will discharge the cap and will

> not short the tank supply.

>

> By having N metal sections on a rotating disk I can have N discharges for a

> 50 Hz cycle (supposing a 3000 RPM motor). The distance between the

> electrodes and the rotating disk will set the minimum trip voltage.

>

> For instance, with a 7 cm diameter disk, supposing a usable electrode-

> section- electrode length of 1 cm (that is, the lenght the two electrodes

> are shorted and 3000 rpm I can get 1 msec available time for the charge and

> 1  msec available for the discharge (roughly).

>

> Has anybody tried this before? Comments and/or suggestions?

>

Marco,

Can you post ckt and 'doubler' details? Or even snail them to me? Just

for grins, I incorporated your concept into our ckt (see pg. below).

What I wound up with was a rotary gap/sw, which used a diametral rotary

element; 3 alternating diametral stationary sw contacts thru 90 deg, as

follows: 0 deg: 1-1,(make) chg cap; (break); 45 deg: 2-2,(make) dischg

cap to T.pri; (break);  90 deg: 1-1,(make) chg cap; (break);  135 deg:

etc.

This should give a 4x cycle rate for a 3krpm motor or 12k cpm. or 200hz.

(someone check my math, here!)

Dwell time and the additional physical circuitry may present additional

Capacitance problems.  Maybe a Variac speed control?

Contact material may also be a problem, as Tungsten is reportedly eaten in

many cases. Many 'real' gaps employ fin-cooled tungsten electrodes.

Also, would this setup eliminate the need for a pri gap in a std NPS supply?

The .5A availability is interesting, and I would like to see if this

approach could really be optimized for NTS apps.  Mike and Bert seem to

cry 'no rots for NPS!'  And the crash-bang may be too hard on most caps?

From a purely 'parts-count/engineering' view, I think you may be trying

to reinvent the wheel, using lots of flat surfaces!  ~~<[:-)

But, we are in 2 differing realms here, and I know little about either.

Yet.

--------------------------------------------------

Gene Waldo -  gwaldo@1stnetusa.com  - "Igor in Residence"

X-Mailer: IPAD 2.0

Date: Thu, 11 Jun 1998 11:52:30 -0500

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: interference

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

At 12:07 AM 6/10/98 -0700, you wrote:

>>Date: Wed, 10 Jun 1998 00:06:02 -0700

>>To: tesla@pupman.com

>>From: bob golding <yubba@clara.net>

>>Subject: interference

>>

>>hi all

>>	The nearest neighbor came and told us we are causing interference on

>their TV and cordless phone. We are in a brick built garage with a 20 foot

>or so overhead extension cable which is running next to the TV aerial. We

>are not using any chokes or emi filters yet. Could the lack of chokes  be

>causing the interference?  We have 2 6 amp emi filters which we can use in

>parallel if needed. Would moving the power lead down to ground level

>putting it in conduit and rerouting it away from the TV aerial help? Maybe

>running at other than at prime time TV time might help. Like 2 am.

>>many thanks.

>>

>>bob golding lightning without rain

>>

>>bob yubba.clara.net

>

Bob,

The lack of RF chokes and EMI filters could allow RF energy

to enter the AC mains where it could be spread all over the place.

Do place RF chokes in your HV leads. This will knock down the RF

level somewhat and also relieve some of the pressure on your neons.

Do get a suitable AC line filter. I would invest in a good 15 or 20

amp filter. These are available quite cheaply surplus.  Your 6 amp one

should be fine for a single 30 ma neon.

Also work on your tank circuit wiring. Excessive lead length in

the Tesla tank circuit will cause problems with the leads radiating

like antennas. Use short heavy conductors. This will also improve

efficiency.

Shorten up that ground lead and improve your RF ground. Those

heavy RF currents have to flow somewhere. Give them a nice low

impedance path to ground. That will minimize radiated RF.

The arcs from the gap and output streamers will radiate and there

is not much you can do about that. However they usually only travel

a short distance.

Also investigate your neighbors TV installation. As a ham radio

operator Ive contended with TVI for many years. Ive found the

the antenna installation is often so poor that the signal level

is barely adequate. With a good strong TV signal chances of

interference will be much less.

TTYL!

Mike Hammer

mhammer@misslink.net

X-Mailer: IPAD 2.0

Date: Thu, 11 Jun 1998 12:00:18 -0500

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: Solenoid primary?

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

At 09:01 PM 6/10/98 -0700, you wrote:

>Bill the arcstarter wrote:

>>

>> Coilers,

>>  I've noticed that Greg L's humongous 130 Kva Electrum coil contains a

>> solenoid-type primary physically located inside the secondary.

>>

>> http://www.lod.org/electrum.html

>>

>>  Has anyone every tried arranging a small tesla coil (4 inch diam

>> secondary or therabouts) in this fashion?

>>

>>  Just wondering (there goes the neighborhood).

>>

>> -Bill

>

>Bill,

>Methinks we get problems of scale--and coupling...  I've developed a

>simple jig system for winding 1/4" Cu tube (solenoid) coils: 2.5-5" od.

>1 leg must be central to the winding; don't know if this is a problem.

>

>The problem I see is pure separation between the Pri & 2ndary, and

>correct coupling.

>LOD has PLAY SPACE! And factors of scale which a 4" 2ndary form jes'

>don't allow!

>I would venture that a 12" dia. 2nd would be needed before internal pri

>could become feasable...  PURE GUESS!

>Anyone care to 2nd guess?

>

>-------------------------------------------

>Gene Waldo -  gwaldo@1stnetusa.com  - "Igor in Residence"

Bill and Gene,

One big problem with an internal primary would be the

loss of inductance. The primary winding would be limited

in diameter making for a very low inductance coil. This

will severely limit performance. A high primary inductance

is essential to good performance.

Also like Gene mentions the problem with overcoupling

will be extreme. Just not enough space between the windings.

My solenoid type primary is over twice the diameter of

my secondary yet it overcouples like crazy.

TTYL!

Mike Hammer

mhammer@misslink.net

Date: Thu, 11 Jun 1998 15:24:51 -0500

From: Bert Hickman <bert.hickman@aquila.com>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Organization: Stoneridge Engineering

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Experiments with a MOT tank supply

Sender: tesl-err@emachine.com

Marco Denicolai wrote:

>

> My tank supply is made of 2 microwave oven transformers and voltage doubler

> that make up a 12KV DC pulsed at 50 Hz. The MOTs are capable of outputting

> up to 0.5 A.

>

> When I first connected a simple steel bolt spark gap (no primary cap, primary,

> nor compressed air quench) I got about 1 cm spark and could keep it on as long

> as I liked (1-2 minutes). Never blown a fuse.

>

> Then I built a RQ static gap, used 7 gaps (the max number I could get a spark

> with) and in each run I ALWAYS blew a 10A fuse in about 5 seconds. Again,

> note that I have no primary cap, no primary coil but I do have 3 mH and

> 550 pF filters between the gap and the tank supply.

>

> Why this?

>

> I explain it like this: the bolt gap was quenching poorly and the supply

> (almost shorted) current limiting stabilized on about 0.5 A current. Almost

> no current spikes. The RQ gap quenches better, the supply is pulsed shorted,

> I get higher current spikes (the current limiting cannot be so fast to

> avoid them) that in a matter of seconds warm and blow my 10A fuse.

>

> This makes me to believe that in the complete TC (using the primary

> capacitor) I MUST disconnect the tank supply before discharging the primary

> cap through the primary, otherwise again I'll blow a fuse.

>

> MY IDEA:

> ========

>

> Build a rotary spark gap where the rotating sections will FIRST connect the

> primary capacitor to the tank supply and THEN connect it to the primary.

>

>                                  --------------

> rotating metal section      ---> |            | ---> movement direction

>                                  --------------

>                                  ||        ||        ||  3 electrodes

>                                  ||        ||        ||

>                                  |         |         |

>                               to tank     to cap    to primary

>

> So I'll get first a spark between the tank and the cap, that will charge it.

> Then, when the rotating section moves forward, I'll get a second spark

> between the cap and the primary coil, that will discharge the cap and will

> not short the tank supply.

>

> By having N metal sections on a rotating disk I can have N discharges for a

> 50 Hz cycle (supposing a 3000 RPM motor). The distance between the electrodes

> and the rotating disk will set the minimum trip voltage.

>

> For instance, with a 7 cm diameter disk, supposing a usable electrod-section-

> electrod lenght of 1 cm (that is, the lenght the two electrods are shorted)

> and 3000 rpm I can get 1 msec available time for the charge and 1 msec

> available for the discharge (roughly).

>

> Has anybody tried this before? Comments and/or suggestions?

>

<SNIP>

Marco,

You're probably seeing some type of interaction between the filter

inductors and caps, the capacitors that are part of your voltage

doubling circuit, and slight differences in the breakdown

characteristics between the two types of sparkgaps. The behavior of the

charging circuit will be _considerably_ different when you add the tank

caps, a primary, and a secondary. Before building a rotary, I'd suggest

you try repeating this experiment with the rest of the system components

in place.

Re: your rotary gap proposal: A few years back, one of the coilers on

this list tried doing something similar using a pole pig AC source. He

found that in some cases when he got misfires, he also got horrendous

kickbacks as the partially charged tank cap would then discharge back

into the pig. This resulted in large voltage pulses being driven back

into the AC Mains in not only his house, but also into his neighbor's

house. These were of such power that they "popped" incandescent lights!

The good news is that, as long as your diodes hold (which may, in

itself, be a challenge), you shouldn't run into this problem.

Your proposed gap design will encounter a couple of other problems,

however. First, a MOT is designed so that the output voltage "collapses"

as you increase output current. If you reduce the amount of time that

your tank cap can be recharged to 1 mSec, you'll need to provide

significantly more charging current from the transformers than they may

be able to deliver. For example, in order to re-charge your 0.05 uF tank

cap to 12 kV in 1 msec, you need to provide an "average" of 600 mA of

current for the _entire_ charging period. While the MOT may deliver this

under short-circuit conditions, it will not do so at higher voltages.

Recharging will simply take longer than 1 msec. One option may be to

make the stationary charging electrodes in a arc-like shape so that they

provide charging current for a longer portion of the rotor's rotation.

The other problem I see is that is that microwave oven capacitors are

typically in the 0.1 - 0.3 uF range, and you're planning on using a

couple of series-connected Maxwell's to make a tank cap of 0.05 uF. Once

you discharge the tank caps, the tank cap recharge current must flow

through the voltage doubling caps in order to recharge your tank cap.

While this occurs, you will be partially discharging the voltage

previously stored in your voltage doubling caps, reducing the maximum

voltage reached by your tank cap even if you've increased the rotary gap

recharging time. Your voltage doubling caps should be at least 20X the

capacitance of the tank cap in order to minimize this effect. These caps

also need to have a higher breakdown voltage than plain microwave oven

caps, since they will be seeing much harsher service. The same thing may

also apply to the microwave diodes.

Hope this helps...

-- Bert --

X-Mailer: IPAD 2.0

Date: Thu, 11 Jun 1998 16:20:40 -0500

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: cap  problems

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

At 01:34 AM 6/11/98 -0700, you wrote:

>hi all

>	something very strange is happening to my cap. I removed it from the box

>it was sitting in as the oil was creeping up the side of the box and

>leaking over the edge. When I took the cap out I noticed that there was a

>large bulge on one side and the acrylic sheets holding it together had all

>cracked and broken away. It looked as if the whole cap had expanded. It was

>still working ok last night so I don't think it is damaged. I have left it

>sitting in another container for now and intend to dismantle it next week

>some time. Does anyone know why this is happening? The cap is made up of 6

>layers of polypropylene sheets between standard cooking foil. it doesnt look

>as if the oil has attacked the plastic. It has attacked the tape I used to

>pull it all together. I am using shell diala bg tranformer oil. many thanks

>

>bob golding lightning without rain

>

>bob yubba@clara.net

Bob,

Ive seen this in my caps also. Part of it may be due to the

fact that poly swells slightly once placed in oil. Not very much

but when you add up lots of layers it can be significant.

As far as tape goes I found the oil attacked most of them I tried.

While duct tape works well to put things together the adhesive

separates from the backing once placed in oil. The best Ive found

is clear mylar packing tape.

If your container is strong enough you can take advantage of this

swelling. Make the cap a tight fit in the container and when it

swells it will be quite compressed. However  that will put a lot of

pressure on the container and make the cap pretty difficult to remove.

In my case I left the cap pretty loose for a few weeks then installed

wooden spacers to compress the cap as tightly as I could.

TTYL!

Mike Hammer

mhammer@misslink.net

X-Mailer: IPAD 2.0

Date: Thu, 11 Jun 1998 19:00:35 -0700

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: bob golding <yubba@clara.net>

Subject: Re: things getting better

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

At 11:38 11/06/98 -0500, you wrote:

>

>

>The new tape drive is now working fine. The IDE cable

>supplied with the drive was defective. However HP/Colorado

>tech support blamed everything else under the sun for the problem.

>They finaly washed their hands of the deal saying that my bios

>must of been at fault. So much for tech support.

Mike

Glad to see you are sorting your PC problems out. I spent three years

building and repairing PC's and guess what 9 times out of 10 the fault was

the ide cable. some of them are only rated for 20 insertions at most.

Thanks for the info on interference on my coil. I will look at the

neighbors ariel setup and see if it can be improved.

bob golding lightning without rain

bob yubba@clara.net

X-Mailer: IPAD 2.0

Date: Fri, 12 Jun 1998 10:34:36 +0300

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Marco Denicolai <marco@vistacom.fi>

Subject: Re: Experiments with a MOT tank supply

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

At 09:48 11/06/98 -0700, you wrote:

>Marco,

>Can you post ckt and 'doubler' details? Or even snail them to me? Just

>for grins, I incorporated you concept into our ckt (see pg. below).

| |

--------|| ||-----| |----*---------- V-

* || || *   | |    |

|| ||          -----

220V        || ||          \   /

|| ||           \ /

|| ||          -----

|| ||            |

--------|| ||------------*--- GND

|

|

--------|| ||------------*

* || || *          |

|| ||          -----

|| ||          \   /

220V       || ||           \ /

|| ||          -----

|| ||     | |    |

--------|| ||-----| |----*----------- V+

| |

Voltage V+ to GND oscillates between 6270 V and 0 (2200 VAC * 2 * sqrt(2) =

6270 Vpp).

Voltage between V- and GND oscillates between -6270 V and 0.

If the transformers are in phase, voltage between V+ and V- is almost DC at

6270 VDC.

V+ |----|    |----|    |----|

|    |----|    |----|    |

GND ------------------------------     V+ - V- = 6.3kV DC

|----|    |----|    |----|

V- |    |----|    |----|    |----

If the transformers are NOT in phase, the output is a sinusoid at 50 Hz

going from 0

to 6270 Vpp * 2 = 12540 Vpp.

V+ |----|    |----|    |----|

|    |----|    |----|    |----

GND ------------------------------     V+ - V- = 0 to 12.5kV DC pulsed

|    |----|    |----|    |----

V- |----|    |----|    |----|

I intend to use the last configuration (transformers NOT in phase, 12.5kV

pulsed output).

>

>What I wound up with was a rotary gap/sw, which used a diametral rotary

>element; 3 alternating diametral stationary sw contacts thru 90 deg, as

>follows: 0 deg: 1-1,(make) chg cap; (break); 45 deg: 2-2,(make) dischg

>cap to T.pri; (break);  90 deg: 1-1,(make) chg cap; (break);  135 deg:

>etc.

>

>This should give a 4x cycle rate for a 3krpm motor or 12k cpm. or 200hz.

>        (someone check my math, here!)

Sounds OK.

>Contact mtl may also be a problem, as Tungsten is reportedly eaten in

>many cases. Many 'real' gaps employ fin-cooled tungsten electrodes.

The electrodes do not really make contact: there is a small gap, of course.

Or what did you mean?

>

>Also, would this setup eliminate the need for a pri gap in a std NPS

>supply?

>The .5A availability is interesting, and I would like to see if this

>approach could really be optimized for NTS apps.  Mike and Bert seem to

>cry 'no rots for NPS!'

Please, what NPS and NTS mean?

>And the crash-bang may be too hard on most caps?

If the cap is suitable to be discharged through the primary, it must stand

also to be charged through it. It is the same (or less) stress, current

just flows in the opposite direction, but now it is someway limited

(transformer impedance, filters, etc.).

Marco Denicolai

X-Mailer: IPAD 2.0

Date: Fri, 12 Jun 1998 11:18:04 +0300

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Marco Denicolai <marco@vistacom.fi>

Subject: Re: Experiments with a MOT tank supply

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

At 15:24 11/06/98 -0500, you wrote:

>Marco,

>

>You're probably seeing some type of interaction between the filter

>inductors and caps, the capacitors that are part of your voltage

>doubling circuit, and slight differences in the breakdown

>characteristics between the two types of sparkgaps.

I tried with and without the filter components: same result.

>The behavior of the

>charging circuit will be _considerably_ different when you add the tank

>caps, a primary, and a secondary.

How can the primary cap affect the current seen by the tank supply when

the spark gap fires? Doesn't it just add current flowing through the gap?

>Re: your rotary gap proposal: A few years back, one of the coilers on

>this list tried doing something similar using a pole pig AC source. He

>found that in some cases when he got misdfires, he also got horrendous

>kickbacks as the partially charged tank cap would then discharge back

>into the pig. This resulted in large voltage pulses being driven back

>into the AC Mains in not only his house, but also into his neighbor's

>house. These were of such power that they "popped" incandescent lights!

>The good news is that, as long as your diodes hold (which may, in

>itself, be a challenge), you shouldn't run into this problem.

This is a good point: I must think about it. One solution could be a

sufficiently high charge-discharge rate. Then, even if the cap is still

charged, the voltage difference between tank output and charged cap

would be quite small. Another would be a strictly synchronous, 50 Hz

charge-discharge rate, being the charging always synchronous with the

tank supply output voltage being at its maximum (in the 50 Hz cycle).

>

>Your proposed gap design will encounter a couple of other problems,

>however. First, a MOT is designed so that the output voltage "collapses"

>as you increase output current. If you reduce the amount of time that

>your tank cap can be recharged to 1 mSec, you'll need to provide

>significantly more charging current from the transformers than they may

>be able to deliver. For example, in order to re-charge your 0.05 uF tank

>cap to 12 kV in 1 msec, you need to provide an "average" of 600 mA of

>current for the _entire_ charging period. While the MOT may deliver this

>under short-circuit conditions, it will not do so at higher voltages.

>Recharging will simply take longer than 1 msec. One option may be to

>make the stationary charging electrodes in a arc-like shape so that they

>provide charging current for a longer portion of the rotor's rotation.

Good point again: I must think about it. At a first thought, I doubt the

MOT is capable of reacting in 1 msec limiting its current: it has got

some inductance too.

>

>The other problem I see is that is that microwave oven capacitors are

>typically in the 0.1 - 0.3 uF range, and you're planning on using a

>couple of series-connected Maxwell's to make a tank cap of 0.05 uF. Once

>you discharge the tank caps, the tank cap recharge current must flow

>through the voltage doubling caps in order to recharge your tank cap.

>While this occurs, you will be partially discharging the voltage

>previously stored in your voltage doubling caps, reducing the maximum

>voltage reached by your tank cap even if you've increased the rotary gap

>recharging time. Your voltage doubling caps should be at least 20X the

>capacitance of the tank cap in order to minimize this effect.

The two doubler caps I use are 1uF each. In 4 european microwave ovens I

dismantled there were always 1uF caps. Because in the doubler they are in

practise connected in series, shouldn't I have 0.5uF/0.05uF = 10 times

the capacitance of the primary cap?

>These caps

>also need to have a higher breakdown voltage than plain microwave oven

>caps, since they will be seeing much harsher service. The same thing may

>also apply to the microwave diodes.

I'm afraid you are right. More thinking needed.

Marco Denicolai

Date: Fri, 12 Jun 1998 04:01:10 -0700 (PDT)

From: Chris Ringwood <ccntesla@yahoo.com>

Subject: Table top coil & flyback HV generator

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

Would it be possible to run a small table top coil from a HV generator made

from a TV flyback transformer?

I would rectify the O/P first as the capacitors just pass the high frequency

when connected direct.

Can anyone help?

I would appreciate it.

Chris Ringwood, UK.

Have attached circuit of Flyback Drive.

(Editors note. Binary attachments not included in archives)

Date: Fri, 12 Jun 1998 07:09:37 -0500

From: Bert Hickman <bert.hickman@aquila.com>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Organization: Stoneridge Engineering

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Experiments with a MOT tank supply

Sender: tesl-err@emachine.com

Marco Denicolai wrote:

>

> At 15:24 11/06/98 -0500, you wrote:

> >Marco,

> >

> >You're probably seeing some type of interaction between the filter

> >inductors and caps, the capacitors that are part of your voltage

> >doubling circuit, and slight differences in the breakdown

> >characteristics between the two types of sparkgaps.

>

> I tried with and without the filter components: same result.

>

> >The behavior of the

> >charging circuit will be _considerably_ different when you add the tank

> >caps, a primary, and a secondary.

>

> How can the primary cap affect the current seen by the tank supply when

> the spark gap fires? Doesn't it just add current flowing through the gap?

>

No - unfortunately its not nearly that simple. The MOT "looks" inductive

to the incoming AC mains, due to  shunt-plate current limiting. This

also shows up as a reflected inductance in the MOT's secondary circuit -

it's like having a perfect transformer secondary connected in series

with a current-limiting inductor. If your series-connected transformers

can output 4400 Volts RMS, and they current-limit at 500 mA RMS, then

this implies an effective "leakage" inductance in the combined MOT

secondaries of about 28 Henrys.

The two voltage doubling caps and the transformer's "leakage inductance"

are all in series when the gap fires, forming an LC circuit which will

attempt to oscillate at around 43 Hz. However, because of the diodes,

this oscillation will be "clamped" once the combined doubler capacitor

voltage tries to reverse polarity. Now 43 Hz is fairly close to the 50

Hz coming from the mains and you'll get non-linear behavior from the MOT

cores as they partially saturate, and from the presence of the diodes.

All sorts of odd (or even chaotic) behavior can be expected with

relatively slight differences in gap breakdown/quenching charateristics.

> >The other problem I see is that is that microwave oven capacitors are

> >typically in the 0.1 - 0.3 uF range, and you're planning on using a

> >couple of series-connected Maxwell's to make a tank cap of 0.05 uF. Once

> >you discharge the tank caps, the tank cap recharge current must flow

> >through the voltage doubling caps in order to recharge your tank cap.

> >While this occurs, you will be partially discharging the voltage

> >previously stored in your voltage doubling caps, reducing the maximum

> >voltage reached by your tank cap even if you've increased the rotary gap

> >recharging time. Your voltage doubling caps should be at least 20X the

> >capacitance of the tank cap in order to minimize this effect.

>

> The two doubler caps I use are 1uF each. In 4 european microwave ovens I

> dismantled there were always 1uF caps. Because in the doubler they are in

> practise connected in series, shouldn't I have 0.5uF/0.05uF = 10 times

> the capacitance of the primary cap?

>

You are correct! I was going by memory (not quite as reliable as when I

was younger!). The ones I have "liberated" are also 1 uF. Sorry 'bout

that! I'd still recommend going with significantly larger doubler

caps...

<SNIP>

-- Bert --

Date: Fri, 12 Jun 1998 14:19:03 +0200

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Table top coil & flyback HV generator

X-Mailer: IPAD 2.0

From: Stefan.Binder@t-online.de (Stefan Binder)

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Chris Ringwood schrieb:

>

> Would it be possible to run a small table top coil from a HV generator made

>  from a TV flyback transformer?

Yes, I've built a TC powered by 8 flybacks, power consumption about 180W (if

you connect several flybacks, the output power of each unit drops), gives 7"

sparks. The flybacks have an internal rectifier and a small capacitor, I think.

Each flyback has an output voltage of about 10kV, 4 flybacks are connected in

series, so I get a higher power output, two of this 4-flyback-units are

connected in parallel (spark gap break down voltage about 15kV). I had to ground

all the flyback transformer cores as the new primary windings started to glow blue...

Each flyback is driven by the circuit with the two 2N3055's.

It also works with 2 flybacks in series (as I only get 10kV out of one flyback I

need at least two), the spark frequency just drops.

Bye,

Stefan

Date: Fri, 12 Jun 1998 07:14:29 -0700

From: Gene Waldo <gwaldo@1stnetusa.com>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Organization: S.A.P.I.E.N.T. (Surly, Agressive, Politically Incorrect Enemies of Non Thinkers)

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Experiments with a MOT tank supply

Sender: tesl-err@emachine.com

Marco Denicolai wrote:

>

> At 09:48 11/06/98 -0700, you wrote:

>

> >Marco,

> >Can you post ckt and 'doubler' details? Or even snail them to me? Just

> >for grins, I incorporated you concept into our ckt (see pg. below).

>

Mike and Bert seem to

> >cry 'no rots for NPS!'

>

> Please, what NPS and NTS mean?

>

Neon Power Supply or Neon Transformer Supply, I think I was trying to

say... I think the approved term is NST: Neon Supply Transformer.

--

Gene Waldo -  gwaldo@1stnetusa.com  - "Igor in Residence"

X-Originating-IP: [199.217.140.22]

From: "Bill the arcstarter" <arcstarter@hotmail.com>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Pulse cap links

Date: Fri, 12 Jun 1998 08:50:32 PDT

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

Coilers,

If anyone is interested, here are four links regarding some can

crusher/quarter shrinker pulse capacitor experiments we have been

doing...

http://www.geocities.com/CapeCanaveral/Hangar/6160/bigcap.html

http://www.geocities.com/CapeCanaveral/Hangar/9561

(at the bottom of the page)

http://www.geocities.com/CapeCanaveral/Hangar/9561/cap.html

http://www.geocities.com/CapeCanaveral/Hangar/9561/quart.html

If you have any comments please relay them to me directly rather than

clutter up this list with non-tesla messages (like this one).

-Bill the arcstarter

Starting arcs in Cinci, OH

http://www.geocities.com/CapeCanaveral/Hangar/6160

From: "Scott Hayward" <smash@eisa.net.au>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Table top coil & flyback HV generator

Date: Sat, 13 Jun 1998 13:27:36 +1000

X-Mailer: IPAD 2.0

X-MimeOLE: Produced By Microsoft MimeOLE V4.71.1712.3

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

> Would it be possible to run a small table top coil from a HV generator made

>  from a TV flyback transformer?

Yes, I've built a TC powered by 8 flybacks, power consumption about 180W (if

you connect several flybacks, the output power of each unit drops), gives 7"

sparks. The flybacks have an internal rectifier and a small capacitor, I

think.

Each flyback has an output voltage of about 10kV, 4 flybacks are connected in

series, so I get a higher power output, two of this 4-flyback-units are

connected in parallel (spark gap break down voltage about 15kV). I had to

ground all the flyback transformer cores as the new primary windings started to

glow blue...

Each flyback is driven by the circuit with the two 2N3055's.

It also works with 2 flybacks in series (as I only get 10kV out of one

flyback I need at least two), the spark frequency just drops.

I've seen these sort of designs before and they didn't work for me.  Has

anyone got a design for a flyback transformer driver circuit that uses a 555

timer and one or more transistors.  Thanx for your help!

Scott Hayward   smash@eisa.net.au

X-Originating-IP: [208.16.145.157]

From: "Bill the arcstarter" <arcstarter@hotmail.com>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Table top coil & flyback HV generator

Date: Fri, 12 Jun 1998 21:12:14 PDT

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

It was written:

>>Each flyback is driven by the circuit with the two 2N3055's.

>>

>>It also works with 2 flybacks in series (as I only get 10kV out of one

>>flyback I need at least two), the spark frequency just drops.

>

>

>I've seen these sort of designs before and they didn't work for me.

>Has anyone got a design for a flyback transformer driver circuit that uses

>a 555 timer and one or more transistors.  Thanx for your help!

Tell you guys what.  Unless you are fooling with ignition coils and

flybacks just for novelty's sake (which is certainly OK as long as you

are willing to live with the limitations), you will be well ahead of the

game if you just use an NST.

Back when I knew it all, I spent lots of time fooling with ignition

coils exactly this way.  I've been wreaking havoc with car coils since

1975 and I'm 30 now - so you do the math...  Now that I don't know as

much, and am using NSTs I've been getting a LOT more output.  There's

just no substitute for raw available power!

Chris Hill has done quite a bit using DC-powered tesla coils, typically

in a single-shot sort of mode.  These usually use a ignition

coil/diode/cap front-end.  Go find his page for more info (sorry- don't

know the url off the top of my head - he's in the tesla coil web ring)

Finally - I have what I think is a pretty good "crayon" design for

driving an ignition coil, using a 555, a 339 comparator (hooked to pin 2

of the 555 and a pot), and a big honkin' FET (needs mandatory zener

suppression from G to S and D to S) to switch the coil.  This combo

gives independent control over frequency (via the 555) and duty cycle

(via the comparator) of the ign coil power.  If someone is really

desperate I will sketch the circuit and put it on my site.  (But only if

I'm specifically asked since I'm a lazy mad scientist...)  And I'd

better find out someone actually tried it if I go to all that trouble! :)

Later! ZZzzzt! Poof!

-Bill

Date: Sat, 13 Jun 1998 20:48:23 +0200

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re:  Table top coil & flyback HV generator

X-Mailer: IPAD 2.0

From: Stefan.Binder@t-online.de (Stefan Binder)

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Scott Hayward schrieb:

> > Would it be possible to run a small table top coil from a HV generator made

> > from a TV flyback transformer?

> Yes, I've built a TC powered by 8 flybacks, power consumption about 180W (if

> you connect several flybacks, the output power of each unit drops), gives 7"

> sparks. The flybacks have an internal rectifier and a small capacitor, I think.

> Each flyback has an output voltage of about 10kV, 4 flybacks are connected in

> series, so I get a higher power output, two of this 4-flyback-units are

> connected in parallel (spark gap break down voltage about 15kV). I had to ground

> all the flyback transformer cores as the new primary windings started to

> glow blue...

> Each flyback is driven by the circuit with the two 2N3055's.

>

> It also works with 2 flybacks in series (as I only get 10kV out of one

> flyback I need at least two), the spark frequency just drops.

>

>

> I've seen these sort of designs before and they didn't work for me.  Has

> anyone got a design for a flyback transformer driver circuit that uses a 555

> timer and one or more transistors.  Thanx for your help!

>

> Scott Hayward   smash@eisa.net.au

>

>

>

Look at

http://koti.icenet.fi/~dixie/plasma.html

So long,

Stefan

From: <Dsurfr@aol.com>

Date: Sun, 14 Jun 1998 14:46:54 EDT

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: paraffin wax vs. mineral oil

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

On my first coil (my God it's now more than 30 years ago) I made a stacked cap

from sheets of glass & aluminum foil which was sealed with paraffin wax. Has

anyone tried using paraffin instead of mineral oil with poly caps? I know the

heat generated in the cap might re-liquify the wax ( I never had this problem

back then, but that first coil wasn't much to speak of by today's standards)

but if its in a sealed container who cares. Paraffin is a lot cheaper than

mineral oil. Any & all feed back will be appreciated.

Jim

Date: Sun, 14 Jun 1998 15:02:18 -0500

From: Bert Hickman <bert.hickman@aquila.com>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Organization: Stoneridge Engineering

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: paraffin wax vs. mineral oil

Sender: tesl-err@emachine.com

Dsurfr@aol.com wrote:

>

> On my first coil (my God it's now more than 30 years ago) I made a stacked cap

> from sheets of glass & aluminum foil which was sealed with paraffin wax. Has

> anyone tried using paraffin instead of mineral oil with poly caps? I know the

> heat generated in the cap might re-liquify the wax ( I never had this problem

> back then, but that first coil wasn't much to speak of by today's standards)

> but if its in a sealed container who cares. Paraffin is a lot cheaper than

> mineral oil. Any & all feed back will be appreciated.   Jim

Jim,

I've got you beat! 37 years ago, I made a small coil that used paraffin

to insulate the space between the primary and secondary! Used glass

plate caps (and later on doorknobs - when you could still expect to

liberate one/junked-TV) :^)

Paraffin is a superb, very low loss, high dielectric strength

insulator. In "the old days", paper capacitors were often made with

paraffin-soaked paper, and encased by a paraffin coating. The difficulty

you'll encounter in your HV caps is in eliminating entrapped air and air

bubbles. Perhaps if you preheated the entire capacitor, filled it with

paraffin, and pulled a vacuum while the material was still liquid, you'd

be able to eliminate corona from entrapped air. With LDPE dielectric,

the trick would be in doing this without also melting the LDPE.

Transformer oil can be found as cheaply as $5/gallon (or less). Since 5

gallons of transformer oil weighs about 34 pounds, the cost per pound is

about $0.15. While I'm not sure paraffin is less expensive than this,

you CAN at least buy the stuff at your local grocer. If you can get rid

of the air, it should work great!

Safe cappin' to you!

-- Bert --

X-Mailer: IPAD 2.0

Date: Sun, 14 Jun 1998 15:33:05 -0500

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: paraffin wax vs. mineral oil

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

At 02:46 PM 6/14/98 EDT, you wrote:

>On my first coil (my God it's now more than 30 years ago) I made a stacked cap

>from sheets of glass & aluminum foil which was sealed with paraffin wax. Has

>anyone tried using paraffin instead of mineral oil with poly caps? I know the

>heat generated in the cap might re-liquify the wax ( I never had this problem

>back then, but that first coil wasn't much to speak of by today's standards)

>but if its in a sealed container who cares. Paraffin is a lot cheaper than

>mineral oil. Any & all feed back will be appreciated.   Jim

>

Jim,

Paraffin can and has been used but has a few problems.

Since its a solid at room temperature any air bubbles

in the cap will be trapped with no way to escape.

Also as it solidifies it shrinks a great deal.

Making it without voids will be difficult.

Also be aware that poly will dissolve in molten paraffin.

A properly built and functioning poly cap should

not generate enough heat to re-liquify the paraffin.

So in my opinion yes it is possible but might be difficult

to implement.

By the way. If you dissolve some polyethylene in hot paraffin

you end up with an excellent insulator and potting compound.

Ive used it to pot some toroidal RF chokes.

TTYL!

Mike Hammer

mhammer@misslink.net

X-Mailer: IPAD 2.0

Date: Mon, 15 Jun 1998 08:32:36 +0300

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Marco Denicolai <marco@vistacom.fi>

Subject: MOT supply continued and spark gap

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

I found a solution to my problem of blowing my 10A fuse with the RQ gap: I use

in series with my two MOT/doubler tank supply the secondary of a third MOT,

like a reactor. The connection order is MOT/doubler -> reactor -> safety gap

-> filters -> main gap -> primary cap and primary.

Somebody will say I re-invented the wheel: right, I am a beginner, after all...

My tank supply outputs 8KV AC at 0.5 A that makes maybe something between

2 and 3 kW, considering the voltage doublers. My primary cap is 0.05uF and

primary coil about 12 uH.

With this configuration I can run almost one minute runs, being the only problem

my RQ gap that gets hot.

I also learned the 3 mH filter inductors DO stop high frequencies! The potential

developed at their terminal was able to flashover and short them (they are

toroids). I had to add space between terminals and cover them with more layers

of sellac.

Now the source of my worries is (again) the spark gap. I use a RQ static gap

with a blower and 11 cupper tube sections. Each section is 5 cm long and 2 cm

of diameter, the tube spacing being 0.8 mm (0.03").

If I connect 5 gaps, the sparking is intermittent and unreliable, depending

on the degree of polishness of the gaps. In practise I have to sand clean

them after each 1 minute run. If I use 3 gaps in parallel with 3 gaps I get

reliable sparking, but the cupper tubes get hot quickly and the gap cupper

tube surface changes color to a shining blue-gray color.

Does this mean my TC is asking for a rotary gap? What do you guys think

about it?

Marco Denicolai

X-Mailer: IPAD 2.0

Date: Mon, 15 Jun 1998 08:38:53 +0300

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Marco Denicolai <marco@vistacom.fi>

Subject: A good RF ground

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

It's time for my TC to get a good RF ground. I read the Quick instructions

about 4-5 8 feet cupper pipes spaced 8 feet. I understand he meant a kind of

RF GND

|

_________________________|___________  soil level

-----------------

|   |   |   |   |

|   |   |   |   |

|   |   |   |   |

|   |   |   |   |

Has anybody got any other suggestion? Have the pipes to be cupper?

Marco Denicolai

From: "Scott Hayward" <smash@eisa.net.au>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Pole pigs

Date: Mon, 15 Jun 1998 18:22:01 +1000

X-Mailer: IPAD 2.0

X-MimeOLE: Produced By Microsoft MimeOLE V4.71.1712.3

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

To all,

Could somebody please tell me how much they paid for a pole pig, (5 or

10KVA), and tell me where they got it from.  Also please tell me which

country so I can get an idea of the prices over in Australia.

From Scott Hayward     smash@eisa.net.au

X-Mailer: IPAD 2.0

Date: Mon, 15 Jun 1998 05:16:34 -0400

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Dave <djb@velocity.net>

Subject: New Tesla-2 archive available

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Hi,

I'd just like to say that the new Tesla-2 archive is now available. This

covers the messages from this list for the months of March and April. You

can download this archive as well as the others from

http://www.velocity.net/~djb/files.shtml

Many thanks to Mike Hammer for his enormous effort. A lot of time and effort

has been spent by Mike to maintain this list, as well as create it's

archives, and answer questions on the Tesla-2 IRC channel. Without his help

and the service provided by Ralph Hartwell, none of this would be possible.

Thanks to everyone who has also contributed to this list.

Just to let everyone know, I'll be on vacation in Australia for the next 2

months starting on Thursday, so I won't be on this list til I get back. My

web page will still be up though, and hopefully I can find a means there to

add the next archive when it becomes available, as well as improve my page

when I get back. Until then, thanks again for such a great list, and I'm

looking forward to rejoining it once more once I get back. Take care!

Dave Bronakowski

Date: Mon, 15 Jun 1998 06:32:49 -0500

From: Bert Hickman <bert.hickman@aquila.com>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Organization: Stoneridge Engineering

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: MOT supply continued and spark gap

Sender: tesl-err@emachine.com

Marco Denicolai wrote:

>

> I found a solution to my problem of blowing my 10A fuse with the RQ gap: I use

> in series with my two MOT/doubler tank supply the secondary of a third MOT,

> like a reactor. The connection order is MOT/doubler -> reactor -> safety gap

> -> filters -> main gap -> primary cap and primary.

>

> Somebody will say I re-invented the weel: right, I am a beginner, after all...

This ballasting arrangement should work OK. Are you short-circuiting the

primary of the "reactor" MOT? Some coilers have done a similar thing by

connecting the primary of a MOT is series with the primaries of the

MOT's used to generate HV, and short-circuiting the secondary of the MOT

being used as a reactor.

Also, if you are doing these experiments without using a TC secondary,

you are literally "cooking" your gaps, since the energy you're providing

has nowhere to go (i.e., no secondary), and all the power ends up

"frying" your gaps. Once you have a secondary installed and begin

getting breakout, the stress on the gaps will be considerably reduced.

>

> My tank supply outputs 8KV AC at 0.5 A that makes maybe something between

> 2 and 3 kW, considering the voltage doublers. My primary cap is 0.05uF and

> primary coil about 12 uH.

>

> With this configuration a can run almost one minute runs, being the only problem

> my RQ gap that gets hot.

>

> I also learned the 3 mH filter inductors DO stop high frequencies! The potential

> developed at their terminal was able to flashover and short them (they are

> toroids). I had to add space between terminals and cover them with more layers

> of sellac.

Sounds like they are trying to do the job. Some coilers have gone to

single layer air chokes, or have used HV wire to wind the chokes on

larger toroids to avoid getting flashovers. This is HV RF, and is

notoriously hard to insulate.

>

> Now the source of my worries is (again) the spark gap. I use a RQ static gap

> with a blower and 11 cupper tube sections. Each section is 5 cm long and 2 cm

> of diameter, the tube spacing being 0.8 mm (0.03").

>

> If I connect 5 gaps, the sparking is intermittent and unreliable, depending

> on the degree of polishness of the gaps. In practise I have to sand clean

> them after each 1 minute run. If I use 3 gaps in parallel with 3 gaps I get

> reliable sparking, but the cupper tubes get hot quickly and the gap cupper

> tube surface changes color to a shining blue-gray color.

>

> Does this mean my TC is asking for a rotary gap? What do you guys think

> about it?

The discoloration is normal oxidation and occurs with all copper

electrode gaps. If you're having difficulty getting the static gaps to

fire reliably, getting a rotary to fire will be considerably more

difficult. Install your secondary and try again.

-- Bert --

Date: Mon, 15 Jun 1998 06:34:37 -0500

From: Bert Hickman <bert.hickman@aquila.com>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Organization: Stoneridge Engineering

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: A good RF ground

Sender: tesl-err@emachine.com

Marco Denicolai wrote:

>

> It's time for my TC to get a good RF ground. I read the Quick instructions

> about 4-5 8 feet cupper pipes spaced 8 feet. I understand he meant a kind of

>

>                        RF GND

>                          |

> _________________________|___________  soil level

>          -----------------

>          |   |   |   |   |

>          |   |   |   |   |

>          |   |   |   |   |

>          |   |   |   |   |

>

> Has anybody got any other suggestion? Have the pipes to be cupper?

>

Marco,

Copper-plated steel ground rods are usually used. Copper pipe can also

be used. Steel or aluminum will rust/corrode and will not give you a

reliable ground over time.

-- Bert --

X-Mailer: IPAD 2.0

Date: Mon, 15 Jun 1998 15:40:43 +0300

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Marco Denicolai <marco@vistacom.fi>

Subject: Re: MOT supply continued and spark gap

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

At 06:32 15/06/98 -0500, Bert wrote:

>This ballasting arrangement should work OK. Are you short-circuiting the

>primary of the "reactor" MOT? Some coilers have done a similar thing by

>connecting the primary of a MOT is series with the primaries of the

>MOT's used to generate HV, and short-circuiting the secondary of the MOT

>being used as a reactor.

My third MOT secondary is in series with the TC primary capacitor and

primary. So it is on the HV side. Filters are between the MOT and the

rest of the stuff. No, I didn't short the MOT primary. I didn't try all

other possible configurations.

>

>Also, if you are doing these experiments without using a TC secondary,

>you are literally "cooking" your gaps, since the energy you're providing

>has nowhere to go (i.e., no secondary), and all the power ends up

>"frying" your gaps. Once you have a secondary installed and begin

>getting breakout, the stress on the gaps will be considerably reduced.

Thanks for this (and all other advices) Bert. I really couldn't guess that

by myself. The fact is that I haven't got my ground ready yet and I'm afraid to

fire the secondary even only for a short test without a good ground.

>The discoloration is normal oxidation and occurs with all copper

>electrode gaps. If you're having difficulty getting the static gaps to

>fire reliably, getting a rotary to fire will be considerably more

>difficult. Install your secondary and try again.

Yes, I'll do that. Thank you.

Marco Denicolai

Date: Mon, 15 Jun 1998 08:50:24 -0500

From: Eric Davidson <edavidson@icva.gov>

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Pole pigs

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Scott:

I paid $175.00, delivered (sort of) for my 5 kVA 13,800/240/120

transformer.  I got it from T&R Electric in Colman, South Dakota, USA.

I am in Iowa and had to drive about 40 miles to a freight depot to pick it

up. They put it in the back of my pickup with a fork lift. The folks at

T&R were very helpful.  Check them out on the web at www.t-r.com   Hope

this helps.

Scott Hayward wrote:

> To all,

> Could somebody please tell me how much they paid for a pole pig, (5 or

> 10KVA), and tell me where they got it from.  Also please tell me which

> country so I can get an idea of the prices over in Australia.

>

> >From Scott Hayward     smash@eisa.net.au

--

Have a good day!

Eric Davidson

edavidson@icva.gov

Full-Name: Bert Wikkerink

Date: Mon, 15 Jun 1998 10:12:57 -0400

From: Bert Wikkerink <bert@mitra.com>

Organization: Mitra Imaging Inc.

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: It has been asked before

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Hi all,

I have just received from my brother a 15 kv, 30 ma NST. I also have a

friend who runs a motor repair shop. How much and what type of wire

should I ask him for ?. While I'm at it, where can I go to get some

design specs. for building a tesla coil, based on my NST ?. I would like

to design it myself, just to gain the experience.

Thanks,

Bert Wikkerink

From: "Thomas McGahee" <tom_mcgahee@sigmais.com>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: paraffin wax vs. mineral oil

Date: Mon, 15 Jun 1998 11:13:09 -0400

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

See note interspersed below...

----------

> From: Bert Hickman <bert.hickman@aquila.com>

> To: Tesla-2 Mailing List <tesla-2@emachine.com>

> Subject: Re: paraffin wax vs. mineral oil

> Date: Sunday, June 14, 1998 4:02 PM

>

> Dsurfr@aol.com wrote:

> >

> > On my first coil (my God it's now more than 30 years ago) I made a stacked cap

> > from sheets of glass & aluminum foil which was sealed with paraffin wax.

> > Has anyone tried using paraffin instead of mineral oil with poly caps? I know

> > the heat generated in the cap might re-liquify the wax ( I never had this problem

> > back then, but that first coil wasn't much to speak of by today's standards)

> > but if its in a sealed container who cares. Paraffin is a lot cheaper than

> > mineral oil. Any & all feed back will be appreciated.   Jim

>

> Jim,

>

> I've got you beat! 37 years ago, I made a small coil that used paraffin

> to insulate the space between the primary and secondary! Used glass

> plate caps (and later on doorknobs - whne you could still expect to

> liberate one/junked-TV) :^)

>

> Paraffin is a superb, very low loss, high dielectric strength

> insulator. In "the old days", paper capacitors were often made with

> paraffin-soaked paper, and encased by a paraffin coating. The difficulty

> you'll encounter in your HV caps is in eliminating entrapped air and air

> bubbles. Perhaps if you preheated the entire capacitor, filled it with

> paraffin, and pulled a vacuum while the material was still liquid, you'd

> be able to eliminate corona from entrapped air. With LDPE dielectric,

> the trick would be in doing this without also melting the LDPE.

>

Remember that polyethelene will DISSOLVE in hot paraffin! This

implies that using hot paraffin in a poly cap may have some really

bad consequences... Hope this helps.    Fr. Tom

X-Originating-IP: [199.217.140.22]

From: "Bill the arcstarter" <arcstarter@hotmail.com>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Pole pigs

Date: Mon, 15 Jun 1998 08:33:16 PDT

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

It was written:

>> Could somebody please tell me how much they paid for a pole pig, (5 or

>> 10KVA), and tell me where they got it from.  Also please tell me which

>> country so I can get an idea of the prices over in Australia.

Ours was $240 for a 25KVA 120/240 to 14,400 pig - 485 lbs.

-Bill

X-Mailer: IPAD 2.0

Date: Mon, 15 Jun 1998 11:28:06 -0500

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: A good RF ground

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

At 08:38 AM 6/15/98 +0300, you wrote:

>It's time for my TC to get a good RF ground. I read the Quick instructions

>about 4-5 8 feet copper pipes spaced 8 feet. I understand he meant a kind of

>

>                       RF GND

>                         |

>_________________________|___________  soil level

>         -----------------

>         |   |   |   |   |

>         |   |   |   |   |

>         |   |   |   |   |

>         |   |   |   |   |

>

>Has anybody got any other suggestion? Have the pipes to be copper?

>

>

Marco,

Yes the grounds should be copper or copper clad steel.

The number of ground rods needed depends on your power level.

The more power you are running the more extensive your ground should be.

Also important is the length and size of the ground lead

running from the base of your secondary to the RF ground.

Again the more power you are running the heavier this conductor

must be. Make it only as long as necessary and as straight as possible

to minimize off axis inductance.

Ive found that only 2 four foot ground rods work well on my

coil at power levels up to 1500 watts.

TTYL!

Mike Hammer

mhammer@misslink.net

X-Mailer: IPAD 2.0

Date: Mon, 15 Jun 1998 11:34:00 -0500

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: MOT supply continued and spark gap

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

At 03:40 PM 6/15/98 +0300, you wrote:

>At 06:32 15/06/98 -0500, Bert wrote:

>

>>This ballasting arrangement should work OK. Are you short-circuiting the

>>primary of the "reactor" MOT? Some coilers have done a similar thing by

>>connecting the primary of a MOT is series with the primaries of the

>>MOT's used to generate HV, and short-circuiting the secondary of the MOT

>>being used as a reactor.

>

>My third MOT secondary is in series with the TC primary capacitor and

>primary. So it is on the

>HV side. Filters are between the MOT and the rest of the stuff.

>No, I didn't short the MOT primary. I didn't try all other possible

>configurations.

>

Marco you would be better off to current limit your transformers

on their primary side. Simply wire the primary of your ballast

transformer in series with the other primaries and short

circuit the secondary of the ballast transformer.

Like I told you earlier its very difficult to make use of

microwave oven transformers (MOT) due to their low

voltage and high current. Its best to treat them like

non current limited transformers and add external ballasting.

TTYL!

Mike Hammer

mhammer@misslink.net

Date: Mon, 15 Jun 1998 18:42:37 +0200

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: stacked plate capacitor questions

X-Mailer: IPAD 2.0

From: Stefan.Binder@t-online.de (Stefan Binder)

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Hi!

I have two questions about stacked plate capacitors:

1. I can't get the suggested polyethylene, I have the choice between PVC,

polystyrene and polycarbonate, which one shall I take? According to the puncture

voltage and dielectric strength, PVC is the material of choice (and it's the

cheapest!), but I don't have information about RF losses, so is there anything

to say against PVC.

BTW: All three plastics are transparent, is this good or bad?

2. In the file cap3.txt Richard Quick suggests to put the capacitor into a

tupperware-container and suck the air out of it. I've tried to suck the air out

of an empty container (well, actually there was air inside) and it was crushed.

Did I use the false type of container (all containers I've seen don't look as if

they could stand the vacuum) or does a capacitor have to be inside the

container?

Thanx in advance,

Stefan

X-Mailer: IPAD 2.0

Date: Mon, 15 Jun 1998 11:43:48 -0500

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: It has been asked before

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

At 10:12 AM 6/15/98 -0400, you wrote:

>Hi all,

>

>I have just received from my brother a 15 kv, 30 ma NST. I also have a

>friend who runs a motor repair shop. How much and what type of wire

>should I ask him for ?. While I'm at it, where can I go to get some

>design specs. for building a tesla coil, based on my NST ?. I would like

>to design it myself, just to gain the experience.

>

>Thanks,

>

>Bert Wikkerink

>

Bert,

For that power level a secondary wound on 4 inch PVC pipe

with 20-22 inches of AWG 24 is a great performer. And it will

handle considerably more power as you acquire additional neons.

To match your neon and to extract the most from it your main

cap size should be around .0053 mfd. Go with poly caps right

from the start. You wont regret it.

Add a flat spiral or shallow saucer primary with 12-14 turns

and a good cylinder gap with 11 electrodes (10 gaps)spaced no more

than .03 inches and you will have the basis for a very good coil.

Dont forget the safety gap and some RF chokes and some series

resistors if necessary to protect your neon.

Add a small toroid or spherical terminal and you're all set.

Download WinTesla from Dave Bronakowski's archive page to

juggle the values around to fit. Dave's URL is:

http://www.velocity.net/~djb/files.shtml

If you have further questions feel free to ask.

TTYL!

Mike Hammer

mhammer@misslink.net

X-Mailer: IPAD 2.0

Date: Mon, 15 Jun 1998 13:14:40 -0500

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: stacked plate capacitor questions

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

At 06:42 PM 6/15/98 +0200, you wrote:

>                     Hi!

>I have two questions about stacked plate capacitors:

>

>1. I can't get the suggested polyethylene, I have the choice between PVC,

>polystyrene and polycarbonate, which one shall I take? According to the puncture

>voltage and dielectric strength, PVC is the material of choice (and it's the

>cheapest!), but I don't have information about RF losses, so is there anything

>to say against PVC.

>BTW: All three plastics are transparent, is this good or bad?

Stefen,

You do NOT want to use PVC or polycarbonate (Lexan) plastics for capacitors.

They have very high RF dissipation factors. Polystyrene is very good though.

Its not as good as polyethylene but its still good. You will only be

able to make stacked plate caps with it as its very stiff.

Polyethylene is very common here in the USA. Its sold in paint and

hardware stores for covering up things when painting.

>

>2. In the file cap3.txt Richard Quick suggests to put the capacitor into a

>tupperware-container and suck the air out of it. I've tried to suck the air out

>of an empty container (well, actually there was air inside) and it was crushed.

>Did I use the false type of container (all containers I've seen don't look as if

>they could stand the vacuum) or does a capacitor have to be inside the

>container?

In order to pull a vacuum you need a rigid container. If you

orient your flat plate caps vertically so the air can rise

out of the stack you do not need to vacuum it. Ive built 3 plate

caps so far and I never vacuumed any of them.

TTYL!

Mike Hammer

mhammer@misslink.net

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: A good RF ground

Date: Mon, 15 Jun 1998 13:40:54 CDT

From: "Kent Vander Velden" <graphix@iastate.edu>

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

>Marco,

>Yes the grounds should be copper or copper clad steel.

>The number of ground rods needed depend on your power level.

>The more power you are running the more extensive your ground

>should be.

Assuming the rods are the same diameter and ignoring the link, is

there any difference between two four foot sections linked together and

a single eight foot section.  While getting the longer section into the

ground would be more difficult it would seem to have the potential to

make a better ground as it would be reaching moister soil.  Are their

other considerations?

Thanks.

---

Kent Vander Velden

kent@iastate.edu

X-Mailer: IPAD 2.0

Date: Mon, 15 Jun 1998 15:22:13 -0500

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: A good RF ground

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

At 01:40 PM 6/15/98 CDT, you wrote:

>

>>Marco,

>>Yes the grounds should be copper or copper clad steel.

>>The number of ground rods needed depend on your power level.

>>The more power you are running the more extensive your ground

>>should be.

>

>  Assuming the rods are the same diameter and ignoring the link, is

>there any difference between two four foot sections linked together and

>a single eight foot section.  While getting the longer section into the

>ground would be more difficult it would seem to have the potential to

>make a better ground as it would be reaching moister soil.  Are their

>other considerations?

>

>  Thanks.

>

>---

>Kent Vander Velden

>kent@iastate.edu

Kent,

My experience with ground systems used underneath ham radio antenna

systems is that a larger number of shorter ground radials yields

a lower ground impedance than a few long ones.

So I would say that two 4 footers would be better than a single 8 footer.

Place them about 24 inches or more apart and bond together with

a very heavy conductor. Copper strap would be ideal. Large guage

wire would be fine.

There in Iowa your soil should have good conductivity so you

should be able to produce a good ground. If your soil is very

dry and/or sandy then a larger number of paralleled rods will

be necessary.

TTYL!

Mike Hammer

mhammer@misslink.net

From: Roy Minnich <rminnich@rolling-meadows.lib.il.us>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: RE: It has been asked before

Date: Mon, 15 Jun 1998 15:29:38 -0500

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

Hi Mike,

Is #24 going to be better for a 4 in. secondary or #22?

I ordered some #22 for my secondary that I'll be powering

with a 12kv 30 ma neon.

Thanks,

Roy Minnich

----------

From: 	Mike Hammer[SMTP:mhammer@misslink.net]

Reply To: 	Tesla-2 Mailing List

Sent: 	Monday, June 15, 1998 11:43 AM

To: 	Tesla-2 Mailing List

Subject: 	Re: It has been asked before

At 10:12 AM 6/15/98 -0400, you wrote:

>Hi all,

>

>I have just received from my brother a 15 kv, 30 ma NST. I also have a

>friend who runs a motor repair shop. How much and what type of wire

>should I ask him for ?. While I'm at it, where can I go to get some

>design specs. for building a tesla coil, based on my NST ?. I would like

>to design it myself, just to gain the experience.

>

>Thanks,

>

>Bert Wikkerink

>

Bert,

For that power level a secondary wound on 4 inch PVC pipe

with 20-22 inches of AWG 24 is a great performer. And it will

handle considerably more power as you acquire additional neons.

To match your neon and to extract the most from it your main

cap size should be around .0053 mfd. Go with poly caps right

from the start. You wont regret it.

Add a flat spiral or shallow saucer primary with 12-14 turns

and a good cylinder gap with 11 electrodes (10 gaps)spaced no more

than .03 inches and you will have the basis for a very good coil.

Dont forget the safety gap and some RF chokes and some series

resistors if necessary to protect your neon.

Add a small toroid or spherical terminal and you're all set.

Download WinTesla from Dave Bronakowski's archive page to

juggle the values around to fit. Dave's URL is:

http://www.velocity.net/~djb/files.shtml

If you have further questions feel free to ask.

TTYL!

Mike Hammer

mhammer@misslink.net

X-Mailer: IPAD 2.0

Date: Mon, 15 Jun 1998 16:19:02 -0500

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Mike Hammer <mhammer@misslink.net>

Subject: RE: It has been asked before

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

At 03:29 PM 6/15/98 -0500, you wrote:

>Hi Mike,

>

>Is #24 going to be better for a 4 in. secondary or #22?

>I ordered some #22 for my secondary that I'll be powering with a 12kv 30 ma neon.

>

>Thanks,

>Roy Minnich

>

Yes Roy I believe AWG 24 to be about the best for a 4 inch

secondary. However like everything else in Tesla coils there

is considerable leeway.

My first 4 inch secondary was 22 inches of AWG 22. It was a good

performer and was pretty well maxed out at around 44 inches of

spark. This was about 750 turns.

I replaced it with another 4 inch secondary wound with 21 inches

of AWG 24. This was about 975 turns and Ive yet to reach its

limit although Im very close to it now at 54 inches.

You may ask why was one better than the other? Well that

might be hard to answer. From a mathematical standpoint the

secondary wound with AWG 22 should have had a higher Q as well

as a lower RF resistance.

The AWG 24 secondary had a lower resonant frequency, a somewhat

lower Q and a higher RF resistance.

I believe that it was the lower frequency that more than made

up for the other factors. The lower frequency allowed me to run

a larger primary cap without sacrificing primary inductance.

I believe it very important to keep as much primary inductance

tapped in as possible. It lowers RF currents in the primary circuit

reducing I2R and gap losses.

So you can use either size. If I was building a 4 incher today

I would use AWG 24 but if I was using AWG 22 for it I would

probably consider making it a bit longer than normal. About

24 inches instead of 22.

If you do this make sure you included plenty of primary turns

so you can tune the primary circuit frequency low enough.

TTYL!

Mike Hammer

mhammer@misslink.net

X-Mailer: IPAD 2.0

Date: Mon, 15 Jun 1998 21:55:02 -0400

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Jim Ryan <jryan@exis.net>

Subject: Rolled caps

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

If you can't evacuate a flat plate cap in "Tupperware", why do you need to

evacuate a rolled cap ?

Jim

Date: Mon, 15 Jun 1998 21:28:31 -0500

From: Tom Polk <tpolk@prismnet.com>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: A good RF ground

Sender: tesl-err@emachine.com

Mike Hammer wrote:

>

> At 08:38 AM 6/15/98 +0300, you wrote:

> >It's time for my TC to get a good RF ground. I read the Quick instructions

> >about 4-5 8 feet copper pipes spaced 8 feet. I understand he meant a kind of

> >

> >                       RF GND

> >                         |

> >_________________________|___________  soil level

> >         -----------------

> >         |   |   |   |   |

> >         |   |   |   |   |

> >         |   |   |   |   |

> >         |   |   |   |   |

> >

> >Has anybody got any other suggestion? Have the pipes to be copper?

> >

I went to Builder's Depot (ha!) and bought some 10 ft sections of 5/8

inch straight hard copper tubing. For a few dollars more I got a fitting

that connected the male end of my garden hose to a short piece of rubber

hose, which I hose clamped onto the end of the 10 ft copper.

When I opened the spigot to the garden hose, I had a formidable water

drill.

Standing on a ladder, after 15-30 minutes, I had worked the copper pipe

where 8 feet of it was driven straight with buttery smoothness into the

earth. (It helps if you have clay soil and no rocks.)

The two feet I left exposed gave me lots of room for a heavy ground

connection, which I soldered with a torch.

It was so much fun I did another. Sixteen feet and whaddya get?

Grounded.

X-Mailer: IPAD 2.0

Date: Mon, 15 Jun 1998 21:41:05 -0500

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: Rolled caps

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

At 09:55 PM 6/15/98 -0400, you wrote:

>If you can't evacuate a flat plate cap in "Tupperware", why do you need to

>evacuate a rolled cap ?

>

>Jim

>

Jim,

You can vacuum a flat plate cap but tupperware is too flimsy

to withstand a vacuum. And its really not necessary if you orient

the plates vertically.

A rolled cap is wound up much tighter (usually) and tends to trap

much more air. The air wont come out as easily.

However if you use a thin low viscosity oil like transformer

oil and allow plenty of time and a slow break in you can probably

get by without vacuuming a rolled cap.

The big advantage to vacuuming is you get a cap ready to go

with minimal break in required.

TTYL!

Mike Hammer

mhammer@misslink.net

X-Originating-IP: [208.16.147.33]

From: "Bill the arcstarter" <arcstarter@hotmail.com>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Rolled caps

Date: Mon, 15 Jun 1998 20:11:12 PDT

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

It was written:

>>If you can't evacuate a flat plate cap in "Tupperware", why do you need to

>>evacuate a rolled cap ?

Are you asking about the container?

I think the only container which will withstand this vacuuming process

would be a round PVC pipe.  Anything square-ish won't stand the crushing

forces caused by the vacuum, etc.

I've never heard of anyone vacuuming a non-pvc pipe cap.  Anyone??

-Bill

From: "Scott Hayward" <smash@eisa.net.au>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Pole pigs

Date: Tue, 16 Jun 1998 16:09:13 +1000

X-Mailer: IPAD 2.0

X-MimeOLE: Produced By Microsoft MimeOLE V4.71.1712.3

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

>It was written:

>>> Could somebody please tell me how much they paid for a pole pig, (5 or

>>> 10KVA), and tell me where they got it from.  Also please tell me which

>>> country so I can get an idea of the prices over in Australia.

>

>Ours was $240 for a 25KVA 120/240 to 14,400 pig - 485 lbs.

Thanx for the quick response.  Was this second hand or new?

From Scott Hayward  smash@eisa.net.au

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Date: Tue, 16 Jun 1998 04:02:28 -0700

From: "Leben Lang und Frei" <fleischwulf@mailexcite.com>

Cc:

X-Sent-Mail: off

X-Mailer: IPAD 2.0

Subject: Spark differences

X-Sender-Ip: 208.153.7.9

Organization: MailExcite  (http://www.mailexcite.com:80)

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

I was wondering could some tell me why there is a difference between the

arc you get from your transformer and the arc you get from the capacitor ?

The arc from the cap is AC and its hot why then won't it strech out like

the one from the transformer ? What is the difference ?

Paul M.

---

Einigkeit und Freiheit

Date: Tue, 16 Jun 1998 14:49:53 +0200 (CEST)

From: Jochen Kronjaeger <Kronjaeg@stud-mailer.uni-marburg.de>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Spark differences

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

> I was wondering could some tell me why there is a difference between the

> arc you get from your transformer and the arc you get from the capacitor

> ?  The arc from the cap is AC and its hot why then won't it strech out

> like the one from the transformer ? What is the difference ?

Paul,

The arc from a  cap consists of extremely short (but very high

current) disharges that occur at a rate of, say, 100 Hz, so they look like

continous. This means that between each disharge, there is a rather long

period of time with no current flowing. During this time, the ionised air

cools down, ions recombine or are diluted. For every disharge, the air has

to be newly ionised.

The arc from a transformer has only short instances of no current flowing

(sine wave), and these occur at 100 Hz. The air has not enough time to

cool and is still ionised when the current rises again, and the voltage

across the arc drops to only a fraction of the breakthrough voltage. Or,

at a given voltage, a continuously ionised arc can be much longer than the

length of air that the same voltage is able to break through.

Jochen

---------------------------------------------------------------------------

Jochen Kronjaeger                     Kronjaeg@stud-mailer.uni-marburg.de

The High Voltage Page  http://www.mathematik.uni-marburg.de/~kronjaeg/hv/

info, plans, photos of home-built HV-sources, sparks, HV-experiments

---------------------------------------------------------------------------

Date: Tue, 16 Jun 1998 08:17:19 -0700

From: Gene Waldo <gwaldo@1stnetusa.com>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Organization: S.A.P.I.E.N.T. (Surly, Agressive, Politically Incorrect Enemies of Non Thinkers)

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Spark differences

Sender: tesl-err@emachine.com

Leben Lang und Frei wrote:

>

> I was wondering could some tell me why there is a difference between the

>arc you get from your transformer and the arc you get from the capacitor ?

>The arc from the cap is AC and its hot why then won't it strech out like the

>one from the transformer ? What is the difference ?

>

> Paul M.

Paul,

Simply stated, caps tend to reduce V and increase I. Volts reach and

Current makes 'em hot. You can play in hi volts, but keep away from hi

Current! Sparks from a cap are short, loud and nasty. The long flickery

types are generally HiV and may be less apt to kill you.

I used to put a cap across the T.2ndary output for interesting effects.

This is also a good way to kill the cap.

-------------------------------------------

Gene Waldo -  gwaldo@1stnetusa.com  - "Igor in Residence"

Date: Tue, 16 Jun 1998 12:03:30 -0600

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Daniel Lockhart <molly@cmn.net>

Subject: Homemade High-Capacitance Caps

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

Hey all,

This question isn't exactly Tesla-related, but I figured it was close enough.

For some experiments I'm doing, I need a high-voltage, high-capacitance

capacitor. All the info I find about making your own hv caps is tesla

related, and thus the caps are relatively small. I need something in the

order of no less than 3 (preferably more like 60) microfarads at no less

than 12 kv.

Has anybody made a capacitor this big? Is it easily done, or is it more

worthwhile to buy one of those wonderful $1300 Maxwell caps?

Thanks,

Daniel Lockhart

Date: Tue, 16 Jun 1998 13:21:24 -0700

From: Gene Waldo <gwaldo@1stnetusa.com>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Organization: S.A.P.I.E.N.T. (Surly, Agressive, Politically Incorrect Enemies of Non Thinkers)

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Homemade High-Capacitance Caps

Sender: tesl-err@emachine.com

Daniel Lockhart wrote:

>

> Hey all,

>

> For some experiments I'm doing, I need a high-voltage, high-capacitance

> capacitor. All the info I find about making your own hv caps is tesla

> related, and thus the caps are relatively small. I need something in the

> order of no less than 3 (preferably more like 60) microfarads at no less

> than 12 kv.

>

> Thanks,

>

> Daniel Lockhart

I recall some big caps in my junkyard rummages- maybe not that big but

will try to reacquire the data.

What are you planning to vaporize?

--

Gene Waldo -  gwaldo@1stnetusa.com  - "Igor in Residence"

X-Mailer: IPAD 2.0

Date: Tue, 16 Jun 1998 15:57:43 -0500

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: Homemade High-Capacitance Caps

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

At 12:03 PM 6/16/98 -0600, you wrote:

>Hey all,

>

>This question isn't exactly Tesla-related, but I figured it was close enough.

>

>For some experiments I'm doing, I need a high-voltage, high-capacitance

>capacitor. All the info I find about making your own hv caps is tesla

>related, and thus the caps are relatively small. I need something in the

>order of no less than 3 (preferably more like 60) microfarads at no less

>than 12 kv.

>

>Has anybody made a capacitor this big? Is it easily done, or is it more

>worthwhile to buy one of those wonderful $1300 Maxwell caps?

>

>Thanks,

>

>Daniel Lockhart

Daniel,

I just dont think building a cap like that would be practical.

It would take years to build and take miles of foil and poly.

I exaggerate a little but it would be lots of work.

I think you need to buy a commercial cap for this or look

around for one surplus.

TTYL!

Mike Hammer

mhammer@misslink.net

Date: Tue, 16 Jun 1998 15:33:45 -0600

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Daniel Lockhart <molly@cmn.net>

Subject: Re: Homemade High-Capacitance Caps

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

Mike wrote:

>I just dont think building a cap like that would be practical.

>It would take years to build and take miles of foil and poly.

>

>I exaggerate a little but it would be lots of work.

>I think you need to buy a commercial cap for this or look

>around for one surplus.

Hmmm. That's what I was afraid of. How do they do it commercially though?

If the smaller poly caps work almost as well as the commercial ones, how do

they ever manage to pack all that capacitance in such a (relatively) small

box? I was just hoping I was missing something! Do they use some super

insulating material?

Gene wrote:

>I recall some big caps in my junkyard rummages- maybe not that big but

>will try to reacquire the data.

I'll keep looking around in surplus... Maybe I'll find a steal! (under a

few grand). What I really need to do is find a good friend at Los Almos...

>What are you planning to vaporize?

Well, H2O to start with (I always thought you should be able to do

something cool with water). After that a rail gun and then this rail

gun/can crusher idea I have. That's a lot of math away though...

Thanks,

Daniel Lockhart

X-Mailer: IPAD 2.0

Date: Tue, 16 Jun 1998 17:31:42 -0500

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: Homemade High-Capacitance Caps

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

At 03:33 PM 6/16/98 -0600, you wrote:

>Mike wrote:

>

>>I just dont think building a cap like that would be practical.

>>It would take years to build and take miles of foil and poly.

>>

>>I exaggerate a little but it would be lots of work.

>>I think you need to buy a commercial cap for this or look

>>around for one surplus.

>

>Hmmm. That's what I was afraid of. How do they do it commercially though?

>If the smaller poly caps work almost as well as the commercial ones, how do

>they ever manage to pack all that capacitance in such a (relatively) small

>box? I was just hoping I was missing something! Do they use some super

>insulating material?

>

>

Commercial caps are made by rolling very thin polypropylene

and foil with a machine. It is wound very tight and the rolls

can be as big as necessary. This results in a high capacitance

low voltage cap. Enough are wired in series to achieve the

desired voltage rating. If more capacitance is required then

more of these series strings are added in parallel.

Caps the size you describe are huge and heavy. They can

weigh in excess of 100 pounds with equally huge price tags.

TTYL!

Mike Hammer

mhammer@misslink.net

X-Originating-IP: [208.16.152.155]

From: "Bill the arcstarter" <arcstarter@hotmail.com>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Large cap deconstruction

Date: Tue, 16 Jun 1998 16:42:41 PDT

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

Well - let me add my $0.02 about huge capacitors...

I just finished using the air chisel to disassemble Paul's *DEAD!*

Maxwell 32 uf 40 KV 250 lb pulse cap (pics pending)

(but don't worry - we have MORE!  The electron abuse shall continue

unabated!)

It's a long story but basically nobody knows why it died - the folks at

Maxwell didn't think what we were doing with it would put undue stress

on the beast.  More about that in some other post when I get the pics on

the page...

The cap was internally constructed from four major packages in series.

Thus each major pack would have been 128 uf 10 KV.

Each major pack consisted of four minor packs.  Thus each minor pack

would have been 32 uf 10 kv.  Approx dimensions of each minor pack are

0.9 x 10 x 24 inches.

Each minor pack consisted of foil-dielectric-foil-dielectric rolled

tightly together under castor oil.  Foil #1 was connected at one end of

the pack, and foil #2 was connected at the other, etc.  There were no

'floating' foils - ie - all foils either went to end #1 or end #2.

The foils were terminated into massive busses which were apparently (at

least as I can tell?) constructed out of lead/tin sprayed onto the

exposed aluminum tabs.  Each bus was about 1/4 thick tin/lead and ran

the entire end of each major pack.  This capacitor had something like a

5/8 NC bolt sticking out the top for the discharge connection!

The foil measured out at 0.5 thousandths thick.  The total dielectric

thickness was 2.0 thousandths.

However - the dielectric was itself composed of five sheets - each

being (of course) 0.4 thou. thick!

This confirms the advice of the list regarding the use of thin

dielectric layers for such a beast.  If you do the math - looks like

that stuff was good for 5kv/thousandth down at that scale!

The dielectric appeared to be some sort of brown translucent (thin!)

paper!  It didn't appear to be a plastic but of course it's hard to tell

when the stuff is only 0.4 thou thick! :)  I was expecting to find some

sort of metallized film but that wasn't the case.  One friend thinks the

stuff was made out of bee's wings... :)

Overall 'innard' dimensions were 14 x 10 x 24 inches.  If you work out

the area and length you will discover that Mike Hammer's estimate of

nearly miles of material is not that far off the mark!

I had to disassemble the thing so we could dispose of it in small

bite-sized pieces... :)

...now if I cross an electric eel with a castor bean plant and maybe

some kudzu or bamboo... Hmmm... An organic capacitor?

-Bill D. Arcstarter

Starting arcs in Cinci, OH

http://www.geocities.com/CapeCanaveral/Hangar/6160

Date: Tue, 16 Jun 1998 19:47:36 -0500

From: Bert Hickman <bert.hickman@aquila.com>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Organization: Stoneridge Engineering

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Large cap deconstruction

Sender: tesl-err@emachine.com

Bill the arcstarter wrote:

<SNIP>

>

>  The dielectric appeared to be some sort of brown translucent (thin!)

> paper!  It didn't appear to be a plastic but of course it's hard to tell

> when the stuff is only 0.4 thou thick! :)  I was expecting to find some

> sort of metallized film but that wasn't the case.  One friend thinks the

> stuff was made out of bee's wings... :)

>

>  Overall 'innard' dimensions were 14 x 10 x 24 inches.  If you work out

> the area and length you will discover that Mike Hammer's estimate of

> nearly miles of material is not that far off the mark!

<SNIP>

>  ...now if I cross an electric eel with a castor bean plant and maybe

> some kudzu or bamboo... Hmmm... An organic capacitor?

>

<SNIP>

Bill,

Thanks for the great post - the thought of collecting miles of bee's

wings struck me as being quite humorous! Maybe reconstituted bee's

wings? Perhaps with a little more genetic engineering... who knows!

Good luck with your NEXT batch of caps, and safe shrinkin' to you!

-- Bert --

Date: Tue, 16 Jun 1998 20:12:56 -0600

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Daniel Lockhart <molly@cmn.net>

Subject: Real Small Cap Construction

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

I don't want to take this too far, I just have a lot of capacitor questions

at the moment! Hope nobody minds...

I'd really like to replace the saltwater cap on my Tesla coil, as I'm

itching for more sparks (already!). I am fluctuating between buying some

commercial ones and making my own. I can:

(a) get two 0.02 mf, 10 kv caps and hook them together to get 0.01 mf, 20

kv (am I correct?) for my 12kv, 30 ma neon, or

(b) make my own using either the Maxwell materials I already mentioned, or

one gallon plastic bags. Before I do anything, I'm going to get a

capacitance meter and measure how much I'm using because I guesstimate it's

a little over 0.0066 mf.

Which would be more preferable? I doubt I could make a cap as good as a

commercial one, but I never have ever built one, so I don't know if the

difference would be worth it.

If I do go ahead and make my own, I was planning on using one gallon

zip-lock plastic bags. I don't have any reliable means of measuring the

thickness. Does anybody know how many mils it is? Any info on that would

be greatly appreciated!

Thanks,

Daniel Lockhart

Date: Tue, 16 Jun 1998 20:42:14 -0600

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Daniel Lockhart <molly@cmn.net>

Subject: Re: Large cap deconstruction

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

>Well - let me add my $0.02 about huge capacitors...

>I just finished using the air chisel to disassemble Paul's *DEAD!*

>Maxwell 32 uf 40 KV 250 lb pulse cap (pics pending)

Ouch! Sorry to hear about that! Oh well, now you have a good excuse to take

it apart...

>> snip! <<

> The dielectric appeared to be some sort of brown translucent (thin!)

>paper!  It didn't appear to be a plastic but of course it's hard to tell

>when the stuff is only 0.4 thou thick! :)  I was expecting to find some

>sort of metallized film but that wasn't the case.  One friend thinks the

>stuff was made out of bee's wings... :)

Maybe everybody already knows about this stuff, I don't know. While looking

in a surplus catalog for big capacitors (the working kind  :-), I noticed

they are selling rolls of Maxwell "K-Film", Maxwell "Brown Kraft Dielectric

Paper" and dielectric films with aluminum already attached to one side.

Maybe the "brown translucent paper" was indeed Kraft paper?

Daniel Lockhart

Date: Tue, 16 Jun 1998 20:46:57 -0700

From: Gene Waldo <gwaldo@1stnetusa.com>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Organization: S.A.P.I.E.N.T. (Surly, Agressive, Politically Incorrect Enemies of Non Thinkers)

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Large cap deconstruction

Sender: tesl-err@emachine.com

Bill the arcstarter wrote:

>

> Well - let me add my $0.02 about huge capacitors...

>

>  I just finished using the air chisel to disassemble Paul's *DEAD!*

> Maxwell 32 uf 40 KV 250 lb pulse cap (pics pending)

>

Snippedey-snip-snips--

>  I had to disassemble the thing so we could dispose of it in small

> bite-sized pieces... :)

>

>  ...now if I cross an electric eel with a castor bean plant and maybe

> some kudzu or bamboo... Hmmm... An organic capacitor?

>

> -Bill D. Arcstarter

Bill D-

GENENTEK is hot on it! Should be possible! Kelp would be better than

kudzu... I think; may even grow faster. Some botanist type should be

able to supply comparative cm^2/ms growth figures.

And infinitely more benign.

Think I'll have another Scotch.... ~~<[:-)

--

Gene Waldo -  gwaldo@1stnetusa.com  - "Igor in Residence"

Date: Tue, 16 Jun 1998 21:12:59 -0700

From: Gene Waldo <gwaldo@1stnetusa.com>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Organization: S.A.P.I.E.N.T. (Surly, Agressive, Politically Incorrect Enemies of Non Thinkers)

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Real Small Cap Construction

Sender: tesl-err@emachine.com

Daniel Lockhart wrote:

>

> I don't want to take this too far, I just have a lot of capacitor questions

> at the moment!

> If I do go ahead and make my own, I was planning on using one gallon

> zip-lock plastic bags. I don't have any reliable means of measuring the

> thickness. Does anybody know how many mills it is? Any info on that would

> be greatly appreciated!

>

> Thanks,

>

> Daniel Lockhart

Dan,

My 1 gal. GLADLOCK Zipper bag mikes at .0034", double; & .0029" single.

The numbers vary, depending upon where I make the measurements.

My guess is that GLADLOCK is not too critical about their material

thickness... granting wide tolerances to their suppliers. For that

service level, "no holes" is probably the 'pass/fail' criterium.

------------------------------------------------

Gene Waldo -  gwaldo@1stnetusa.com  - "Igor in Residence"

Date: Tue, 16 Jun 1998 22:13:56 -0700

From: Michael Loftus <loftusm@ix.netcom.com>

Organization: Me

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Vacuum Pumps . . .?

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Well,

After deciding to build rolled caps rather than the other alternatives

and being about ready to seal the first one shut, I find that I need a

vacuum pump to pump-down the cap and its oil.  I should have given this

more thought when I started because I can not find one to borrow, rent,

or steal here in Orange County, CA.  Does anyone have any suggestions?

From what I have read, not pumping the cap down may quickly result in a

pipe full of land-fill material.  As a last resort I may try to find an

affordable used pump somewhere, but searching on the web points to about

$450 - $550  as the entry-point!!!  All of a sudden buying commercial

used caps appears attractive.

Mike

Date: Wed, 17 Jun 1998 01:01:03 -0500

From: Tom Polk <tpolk@prismnet.com>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Vacuum Pumps . . .?

Sender: tesl-err@emachine.com

The idea is to get the bubbles out and remove the voids, right?

I've used old refrigerator and air conditioning compressors in reverse.

Maybe even an old paint compressor you don't care about might work.

You don't have to pull a perfect vacuum.

If you want to do it right, but cheap, there are hand operated vacuum

pumps on the market, maybe Edmund, much less expensive than big ol'

heavy pumps. Anybody know about where to get those? I thought they were

about $18-25.

Finally, if you have a storage tank, like an old freon tank (I modified

a propane torch tank) you can take it to a neon sign store or air

conditioning service establishment, and see if they'll pull a vacuum for

you. (Often, servicemen will gladly take on that sort of thing to liven

their day.)

Then you take the tank home and let it suck all the air out of your cap.

It needs to have a lot more volume than the free air space in your cap.

The disadvantage is that you only get one shot at it before having to

"refill" your vacuum. The advantage is that you can leave it there a

while, with a constant reverse pressure on the cap, without having to

keep a pump running.

Michael Loftus wrote:

>

> Well,

> After deciding to build  rolled caps rather than the other alternatives

> and being about ready to seal the first one shut, I find that I need a

> vacuum pump to pump-down the cap and its oil.  I should have given this

> more thought when I started because I can not find one to borrow, rent,

> or steal here in Orange County, CA.  Does anyone have any suggestions?

>

> >From what I have read, not pumping the cap down may quuickly result in a

> pipe full of land-fill material.  As a last resort I may try to find an

> affordable used pump somewhere, but searching on the web points to about

> $450 - $550  as the entry-point!!!  All of a sudden buying commercial

> used caps appears attractive.

>

> Mike

Date: Wed, 17 Jun 1998 06:32:56 -0500

From: Bert Hickman <bert.hickman@aquila.com>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Organization: Stoneridge Engineering

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Vacuum Pumps . . .?

Sender: tesl-err@emachine.com

Michael Loftus wrote:

>

> Well,

> After deciding to build  rolled caps rather than the other alternatives

> and being about ready to seal the first one shut, I find that I need a

> vacuum pump to pump-down the cap and its oil.  I should have given this

> more thought when I started because I can not find one to borrow, rent,

> or steal here in Orange County, CA.  Does anyone have any suggestions?

>

> >From what I have read, not pumping the cap down may quuickly result in a

> pipe full of land-fill material.  As a last resort I may try to find an

> affordable used pump somewhere, but searching on the web points to about

> $450 - $550  as the entry-point!!!  All of a sudden buying commercial

> used caps appears attractive.

>

> Mike

Mike,

Don't dispair. You don't need a perfect vacuum - in fact you may not

need a vacuum pump at all. If you orient the caps so that the plates are

vertical and initially operate the system at low power (not very wide

gap spacings), the internal plates will expand and contract under the

sharply changing electrostatic forces, sort of like an accordian. This

will help work out the entrapped air with time. You can then gradually

increase the power level over a few weeks until you reach maximum power.

Safe cappin' to you!

-- Bert --

From: wwl@netcomuk.co.uk (Mike Harrison)

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Large cap deconstruction

Date: Wed, 17 Jun 1998 11:54:45 GMT

Organization: White Wing Logic

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

On Tue, 16 Jun 1998 16:42:41 PDT, you wrote:

>Well - let me add my $0.02 about huge capacitors...

>

> I just finished using the air chisel to disassemble Paul's *DEAD!*

>Maxwell 32 uf 40 KV 250 lb pulse cap (pics pending)

>

>(but don't worry - we have MORE!  The electron abuse shall continue

>unabated!)

>

> It's a long story but basically nobody knows why it died - the folk at

>Maxwell didn't think what we were doing with it would put undue stress

>on the beast.  More about that some other post when I get the pics on

>the page...

>

> The cap was internally constructed from four major packages in series.

>Thus each major pack would have been 128 uf 10 KV.

Before demolishing it completely, did you try testing the individual

'packs' - if only a few are dead, maybe you could assemble a

reasonable cap from the remaining good ones.

____                                                           ____

_/ L_/  Mike Harrison / White Wing Logic / wwl@netcomuk.co.uk  _/ L_/

_/ W_/  Hardware & Software design / PCB Design / Consultancy  _/ W_/

/_W_/  Industrial / Computer Peripherals / Hazardous Area      /_W_/

Date: Wed, 17 Jun 1998 06:06:25 -0700

From: Scott MacGregor <smacgregor@aasp.net>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Organization: EES,Co.

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Vacuum Pumps . . .?

Sender: tesl-err@emachine.com

Michael Loftus wrote:

>

> Well,

> After deciding to build  rolled caps rather than the other alternatives

> and being about ready to seal the first one shut, I find that I need a

> vacuum pump to pump-down the cap and its oil.  I should have given this

> more thought when I started because I can not find one to borrow, rent,

> or steal here in Orange County, CA.  Does anyone have any suggestions?

>

> >From what I have read, not pumping the cap down may quuickly result in a

> pipe full of land-fill material.  As a last resort I may try to find an

> affordable used pump somewhere, but searching on the web points to about

> $450 - $550  as the entry-point!!!  All of a sudden buying commercial

> used caps appears attractive.

>

> Mike

Mike,

Edmund Scientific has a hand held vacuum pump and gauge for $32.95 in

their 98 catalog. 0-30" of mercury. You can e-mail them for a catalog at

scientifics@edsci.com

--

Regards,

Scott MacGregor EES,Co.

X-Originating-IP: [199.217.140.22]

From: "Bill the arcstarter" <arcstarter@hotmail.com>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Large cap deconstruction

Date: Wed, 17 Jun 1998 06:44:00 PDT

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

It was written:

>>  ...now if I cross an electric eel with a castor bean plant and maybe

>> some kudzu or bamboo... Hmmm... An organic capacitor?

<snip>

>GENENTEK is hot on it! Should be possible! Kelp would be better than

>kudzu... I think; may even grow faster. Some botanist type should be

>able to supply comparative cm^2/ms growth figures.

Yup.  What we need is a ring-type structure - like the rings in a

treetrunk.  Perhaps crossing the eels with some sort of self-metallizing

tree?  An osage orange-drop capacitor?? :)

(Sprague used to make a capacitor known as the "Orange-drop")

Or, for you Larry Niven fans - we could use the Slaver sunflower

plants... (these sci-fi plants contained metallized parabolic reflective

leaves and would focus the sun at their predators by the acre!  But I

digress.. ) (Read Nivens's "Ringworld" books for more info)

Has anyone out there read Dan Simmons' "Hyperion" books?  Complete with

tesla trees and other exotic plantlife... :)

One of the best sci-fi machine wildlife descriptions is found in J.P.

Hogans "Code of the Lifemaker".  This book describes a run-away self

perpetuating mining probe - which eventually organizes itself into a

machine society.  Hogan writes some beautiful prose about the wild

transformer banks, flocking wire winders, plastic extruders gathering

together at the ponds sucking up raw materials, the rare metal

scavengers whtih plasma spectrascopes built into their snouts, etc.  I'm

not doing him justice!

What size capacitor do I need to impedance match my pet electric eel to

my tesla coil? (Geez - I suppose an electric eel would be DC anyway...

Could run a geiger-muller tube off him I suppose..)

-Bill "trying to say on topic but not being very successful"

X-Originating-IP: [199.217.140.22]

From: "Bill the arcstarter" <arcstarter@hotmail.com>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Large cap deconstruction

Date: Wed, 17 Jun 1998 07:12:11 PDT

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

I wrote:

>> The cap was internally constructed from four major packages in series.

>

>>Thus each major pack would have been 128 uf 10 KV.

Then Mike Harrison wrote:

>Before demolishing it completely, did you try testing the individual

>'packs' - if only a few are dead, maybe you could assemble a

>reasonable cap from the remaining good ones.

Many folks have asked me that.  This cap apparently suffered three

simultaneous failures inside the steel case.  Two of the failures were

of such a large magnitude that the steel case was dented outward about

3/4 of an inch.  In one case the welded seam was split.

Bert Hickman thinks that (of course) one happened first - thus possibly

causing an insulation overload in the other sections.  This sounds

plausable.  But after seeing the hydraulic shock - any subsequent

failures may well have been mechanical in nature - followed by the

electrical surge through the short!

No portion of the inside was unaffected by the enormous hydraulic shock

of these failures.  In fact - a 'block' of material about 2x3x1 inches

was displaced outwards (thus forming one of the dents in the case) about

0.5 inches.  It looked as though the pack had been placed in a press and

the material partially jogged through a die, etc...  Hard to imagine but

this is what I found!

I'm afraid the insides are a complete write-off.  :(

-Bill

Date: Wed, 17 Jun 1998 07:43:37 -0700

From: Gene Waldo <gwaldo@1stnetusa.com>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Organization: S.A.P.I.E.N.T. (Surly, Agressive, Politically Incorrect Enemies of Non Thinkers)

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Vacuum Pumps . . .?

Sender: tesl-err@emachine.com

Michael Loftus wrote:

>

> Does anyone have any suggestions?

>

> Mike

There are (or were) quite effective, inexpensive aspirator vacuum pumps

which simply attach to a water faucet or hose bib; effluent used to

water your garden.

Some crafty valving and water trap plumbing would be needed, however.

I recall getting 25-29" Hg from mine, which should be quite adequate.

You should be able to buy the unit and rig the whole set up for about

$20.

I'd suggest care and a slow initial application of vacuum from any

source, as the initial expansive exodus of air may cause the oil to foam

and rush into the pumping system.

As Bert suggests, the additional application of a cycling current to

physically activate the plates would help in pumping the trapped air out

from between them.

I'd also play with a 'dry dummy' cap container to verify the integrity

of the whole system and procedures.

------------------------------------------

Gene Waldo -  gwaldo@1stnetusa.com  - "Igor in Residence"

X-Originating-IP: [199.217.140.22]

From: "Bill the arcstarter" <arcstarter@hotmail.com>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Large cap deconstruction

Date: Wed, 17 Jun 1998 08:06:27 PDT

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

Daniel wrote:

>

>Maybe everybody already knows about this stuff, I don't know. While looking

>in a surplus catalog for big capacitors (the working kind  :-), I noticed

>they are selling rolls of Maxwell "K-Film", Maxwell "Brown Kraft Dielectric

>Paper" and dielectric films with aluminum already attached to one side.

>

>Maybe the "brown translucent paper" was indeed Kraft paper?

I think that Surplus Sales of Nebraska sells this stuff too.

What, exactly is Kraft paper?  Is it a trademark?  Any uses other than

for capacitors?

-Bill

Date: Wed, 17 Jun 1998 10:14:11 -0500

From: Eric Davidson <edavidson@icva.gov>

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Vacuum Pumps . . .?

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Gene et al:

The water aspirator type vacuum pumps can be quite good. The ultimate

vacuum obtainable is related to the vapor pressure (temperature) of the

water. Use cold water at high flow for best vacuum.  Also, have a means

to break the vacuum before turning off the water or you'll suck water

into the system. Hope this helps.

--

Have a good day!

Eric Davidson

edavidson@icva.gov

X-Mailer: IPAD 2.0

Date: Wed, 17 Jun 1998 11:22:03 -0500

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: Vacuum Pumps . . .?

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

At 10:13 PM 6/16/98 -0700, you wrote:

>Well,

>After deciding to build  rolled caps rather than the other alternatives

>and being about ready to seal the first one shut, I find that I need a

>vacuum pump to pump-down the cap and its oil.  I should have given this

>more thought when I started because I can not find one to borrow, rent,

>or steal here in Orange County, CA.  Does anyone have any suggestions?

>

>From what I have read, not pumping the cap down may quickly result in a

>pipe full of land-fill material.  As a last resort I may try to find an

>affordable used pump somewhere, but searching on the web points to about

>$450 - $550  as the entry-point!!!  All of a sudden buying commercial

>used caps appears attractive.

>

>Mike

Mike,

What I used for pumping down my rolled caps was the compressor

from an old dehumidifier. Just cut the compressor loose watching

how the AC wiring is attached so you can power it up.

Generally speaking the larger diameter line is the vacuum line and

the smaller line is the exhaust. When you power it up the first

time it will spew oil out the exhaust. That will diminish in

a couple minutes.

Attach your plumbing fittings of choice to the inlet and you are all

set. A filter of some sort to collect the oil vapor coming out the

exhaust will help prevent breathing it.

The best method is to pump the cap down for a few minutes then

turn off the pump. Through a valve arrangement allow the vacuum

in the cap to draw in the oil. Go slowly to keep the oil from foaming.

Once the pressure equalizes you can pull another vacuum on the cap to

bubble out the rest of the trapped air.

You dont need a fancy vacuum pump as the needed vacuum isnt that great.

TTYL!

Mike Hammer

mhammer@misslink.net

Sender: tesl-err@emachine.com

Date: Wed, 17 Jun 1998 20:19:57 +0000

From: Tom Stock <tomstock@geocities.com>

Organization: X Industries

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Vac pump plans

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Roy Minnich wrote:

> It was said:

>

> >>     I have plans in a Scientific American for a molecular seive based

> >> vacuum pump that is really easy to make and vacuums to only a copuple of

> >> millitorrs.  If you are interested I could scan it in and post it on the

> >> list.

>

> I'd be very interested in such plans! If there's enough people interested

> count me in. If not, and you don't want to have to scan them, then I'd be

> happy to know what issue it was in.

>

I'm interested too.

-tom

Sender: tesl-err@emachine.com

Date: Wed, 17 Jun 1998 16:55:52 -0400

From: Kent Rankin <kentrankin@theatreorgans.com>

Organization: Acidflux Productions

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Vacuum Pumps . . .?

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

>     I have plans in a Scientific American for a molecular seive based

> vacuum pump that is really easy to make and vacuums to only a copuple of

> millitorrs.  If you are interested I could scan it in and post it on the

> list.

BTW, I once calculated what it would run me to build one, and including

a warming coil, the liquid nitrogen, a pyrex tub, and the seive, it was only

about $35.  A great tool to have around.

-Kent Rankin

Date: Wed, 17 Jun 1998 15:13:12 -0600

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Daniel Lockhart <molly@cmn.net>

Subject: Re: Vacuum Pumps . . .?

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

It was said:

>>     I have plans in a Scientific American for a molecular seive based

>> vacuum pump that is really easy to make and vacuums to only a copuple of

>> millitorrs.  If you are interested I could scan it in and post it on the

>> list.

I'd be very interested in such plans! If there's enough people interested

count me in. If not, and you don't want to have to scan them, then I'd be

happy to know what issue it was in.

Thanks,

Daniel Lockhart

Date: Thu, 18 Jun 1998 08:15:51 -0400

From: Bert Wikkerink <bert@mitra.com>

Organization: Mitra Imaging Inc.

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Vac pump plans

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Count me in as well. Due to the high volume of requests, it might be a

good idea to stick them on someones web page and just post the URL.

Bert

Date: Thu, 18 Jun 1998 09:17:36 -0700

From: Gene Waldo <gwaldo@1stnetusa.com>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Organization: S.A.P.I.E.N.T. (Surly, Agressive, Politically Incorrect Enemies of Non Thinkers)

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Vacuum Pumps . . .?

Sender: tesl-err@emachine.com

Kent Rankin wrote:

I have plans in a Scientific American for a molecular seive based

vacuum pump that is really easy to make and vacuums to only a couple of

millitorrs.  If you are interested I could scan it in and post it on the  list.

BTW, I once calculated what it would run me to build one, and including

a warming coil, the liquid nitrogen, a pyrex tub, and the seive, it was

only  about $35.  A great tool to have around.

-Kent Rankin

---------------------------

Kent,

Could you please ref the S.A. issue? My library card is still valid.

At the moment, my interest is academic, and while I don't wish to throw

a wet blanket on the project, some additional factors should be

considered.

(Throwing a wet blanket)

Any thoughts on contamination of this device? Most torr region pumps

require high quality mechanical roughing and exrrrrreme cleanliness of

the pumped system. Diffusion pumps and seives are usually very slow, at

their working pressures.

I used to do He2 mass spectrometer leak testing, and even finger prints

on UUT would give us horrendous outgassing problems.

Thermocouple or McCloud guages would be needed to indicate realistic

levels of vacuum. The construction of the Cold Trap, and general

leakproofing of a torr level systems is apt to be a more massive project

than cap construction.

The $torage logistic$ of a maintained LN2 supply can get kinda hairy for

the garage tinkerer.

The overall efficiency of this sort of system for simple air removal

from our capacitors is something to consider....

Even a shitty mech pump or aspirator would be more than adequate.

Just some ideas to kick around.

-----------------------------------------

Gene Waldo -  gwaldo@1stnetusa.com  - "Igor in Residence"

From: kirt_jarrett@stand-prod.com

X-Mailer: IPAD 2.0

Date: Thu, 18 Jun 98 12:50:43 -0500

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: No subject given

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

I am interested in purchasing a 5 mH choke suitable for a Tesla Coil.

The one I built does fine with the variac below 50% but after that it

starts to skip.  If anyone knows where one can be purchased or is

interested in selling me a custom built unit, please e-mail me at

"kirt_jarrett@stand-prod.com".

Thank you for your support,

Kirt Jarrett

kirt_jarrett@stand-prod.com

Date: Thu, 18 Jun 1998 10:54:11 -0700

From: mike nicholas <705728@ican.net>

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Vac pump plans

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

I have built several vacum pumps out of  rotary fridge compressors, they

pull a fairly deep vacum, 29.5"Hg

X-Mailer: IPAD 2.0

Date: Thu, 18 Jun 1998 13:09:18 -0500

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: 5mh chokes

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

At 12:50 PM 6/18/98 -0500, you wrote:

>

>     I am interested in purchasing a 5 mH choke suitable for a Tesla Coil.

>     The one I built does fine with the variac below 50% but after that it

>     starts to skip.  If anyone knows where one can be purchased or is

>     interested in selling me a custom built unit, please e-mail me at

>     "kirt_jarrett@stand-prod.com".

>

>

>     Thank you for your support,

>

>     Kirt Jarrett

Kirt,

Most coilers build their own chokes. Its not too difficult to make

a choke that doesnt arc over.

Perhaps if you describe your chokes we can figure out where you went

wrong. The most common mistakes are winding on a bare ferrite core

and trying to make a multilayer winding.

Ferrite cores are partially conductive and will cause arcing problems.

3 layers of good quality vinyl tape on the core first will take

care of that problem.

Multilayer coils fail because the layer to layer voltage is too

great for the insulation. Keeping the winding in a single layer

is the simplest cure.

For 5 mh try a choke like a 8 inch long winding of AWG 28

on a piece of 2 inch PVC pipe. That should be about 575 turns

and result in about 5.2 mh.

Such a choke should handle 200 ma easily.

Remember that air core chokes like this should be used with some series

resistance to kill the Q and prevent ringing.

TTYL!

Mike Hammer

mhammer@misslink.net

Date: Thu, 18 Jun 1998 12:04:16 -0700

From: Tom Polk <tpolk@prismnet.com>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: 5mh chokes

Sender: tesl-err@emachine.com

Mike Hammer wrote:

> Remember that air core chokes like this should be used with some series

> resistance to kill the Q and prevent ringing.

How much series resistance?

X-Mailer: IPAD 2.0

Date: Thu, 18 Jun 1998 17:17:36 -0500

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: 5mh chokes

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

At 12:04 PM 6/18/98 -0700, you wrote:

>Mike Hammer wrote:

>

>> Remember that air core chokes like this should be used with some series

>> resistance to kill the Q and prevent ringing.

>

>How much series resistance?

>

Its not really critical. Anything from 300-1500 ohms will do fine.

I prefer the lower end of the range as you have less voltage drop.

Ive been using some 310 ohm 50 watt resistors I bought at a hamfest

with good results. I know some people who have used as much as 3K ohms.

All you need is enough to dampen any ringing that may occur in your

chokes. Air core chokes with lots of turns of wire are especially

prone to this problem. Sometimes they will ring up quite high voltages

just like a small Tesla coil. Precisely the problem you are trying to prevent.

Just make sure you have enough wattage rating to handle the expected

load. 10 watts should be sufficient with most 30 ma neons.

25 watts may be needed at 60ma. Remember if your primary cap

is perfectly matched to your neon you may be drawing more than the

plate rating from the transformer. Make sure your resistors can handle

the extra current.

TTYL!

Mike Hammer

mhammer@misslink.net

From: <Dsurfr@aol.com>

Date: Thu, 18 Jun 1998 18:18:20 EDT

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: spark length vs. power

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

When I add more SW caps to my tank circuit I get a much stronger (all white,

no purple & different sound - more of a crack) arc but it doesn't seem nearly

as thick & is a couple of inches shorter in length when arcing to a ground.

Does this mean I'm over capped or just need to re-tune?

Thanks, Jim

Sender: tesl-err@emachine.com

Date: Fri, 19 Jun 1998 03:16:46 -0400

From: Kent Rankin <kentrankin@theatreorgans.com>

Organization: Acidflux Productions

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Vacuum Pumps . . .?

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

I found the Vacuum Pump Plans from Scientific American.  It is in the

October 1996 issue(pages 114-5).    It made a complete liar of me(although not

by much).  It only pumps down to a little below 10 millitorr.  I claimed a

couple of millitorr(what is the abbreviation for that?)

For a while the Society for Amateur Scientists offered a kit based upon this

plan for $60 including the Pyrex flask, molecular sieve, heating tape, liquid

nitrogen tank, and tygon tubing.  I suppose one would have to come across their

own vacuum tank, vacuum base, and vacuum grease.

It also acknowledged a few resources on the net(although most of you are

probably familiar with them:

Soc. for Amateur Scientists

http://www.thesphere.com/SAS/

The Bell Jar

http://www.tiac.net/users/shansen/belljar/

I will put a page on my home page in a few days disclosing all of the

material contained within the article(including pictures) for you all.  Look for

the page at the following address:

http://www.public.usit.net/~rafalk/Kent/

If there are any questions, or other ways that I can help, then please tell

me.

-Kent Rankin

Date: Fri, 19 Jun 1998 10:49:44 -0700

From: mike nicholas <705728@ican.net>

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Vacuum Pumps . . .?

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

You can build a vacuum pump for free, you find many rotary compressors at the

dump off old fridges, they are round compressors with fins built in, I'm not sure

about militorrs or bars, but it can pull a near perfect vacum, I used one for years

to take the moisture out of sealed systems, it may have cost me $5 to build (I made

it fancy)

Kent Rankin wrote:

>     I found the Vacuum Pump Plans from Scientific American.  It is in the

> October 1996 issue(pages 114-5).    It made a complete liar of me(although not

> by much).  It only pumps down to a little below 10 millitorr.  I claimed a

> couple of millitorr(what is the abbreviation for that?)

>     For a while the Society for Amatuer Scientists offered a kit based upon this

> plan for $60 including the Pyrex flask, molecular sieve, heating tape, liquid

> nitrogen tank, and tygon tubing.  I suppose one would have to come across their

> own vacuum tank, vacuum base, and vacuum grease.

>     It also acknowledged a few resources on the net(although most of you are

> probably familiar with them:

>

> Soc. for Amatuer Scientists

>     http://www.thesphere.com/SAS/

> The Bell Jar

>     http://www.tiac.net/users/shansen/belljar/

>

>     I will put a page on my home page in a few days disclosing all of the

> material contained within the article(including pictures) for you all.  Look for

> the page at the following address:

>

> http://www.public.usit.net/~rafalk/Kent/

>

>     If there are any questions, or other ways that I can help, then please tell me.

>

> -Kent Rankin

From: "The Meyer Family" <meyer@clove.net.au>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Faraday Cage Shielding

Date: Fri, 19 Jun 1998 20:55:34 +0930

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Hello,

I am writing to ask about shielding my coil from my other electronics and

radio control equipment. I have a shed up the back of the house which is

usused so far. To make a faraday cage would I be able to line the concrete

floor with chicken wire, making sure there is a connection between the

walls and the floor, then put a big earth on the whole shed??

Then in theory the shed should become a big Faraday cage, am I right????

Would this be effecient?

Any other suggestions??

Thanks For Any Help

Andrew

meyer@clove.net.au

Date: Fri, 19 Jun 1998 10:32:44 -0500

From: Eric Davidson <edavidson@icva.gov>

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Vacuum Pumps . . .?

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Mike et al:

The compressors used in refigerators and the like, may be used as vacuum

pumps, but please be advised that the vacuum they produce is no where

near perfect. A perfect vacuum is difficult to achieve (actually,

perfect is impossible) and requires diffusion pumping, cryopumping and

molecular friction type pumps.  The good news is: for simple pumping, to

remove air from a home made capacitor, requires little, vacuum wise. I'd

order a simple siphon type pump for your water faucet, even before I

messed around with an old refrigeration compressor. Hope this helps.

mike nicholas wrote:

>

>  You can build a vacuum pump for free, you find many rotary compressors at the

> dump off old fridges, they are round compressors with fins built in, I'm not sure

> about militorrs or bars, but it can pull a near perfect vacuum, I used one for years

> to take the moisture out of sealed systems, it may have cost me $5 to build (I made

> it fancy)

--

Have a good day!

Eric Davidson

edavidson@icva.gov

X-Mailer: IPAD 2.0

Date: Fri, 19 Jun 1998 10:47:36 -0500

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: spark length vs. power

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

At 06:18 PM 6/18/98 EDT, you wrote:

>When I add more SW caps to my tank circuit I get a much stronger (all white,

>no purple & different sound - more of a crack) arc but it doesn't seem nearly

>as thick & is a couple of inches shorter in length when arcing to a ground.

>Does this mean I'm over capped or just need to re-tune?

>Thanks, Jim

Jim,

Generally speaking more primary capacitance will increase

your primary energy level. This larger "bang size" will

definitely result in hotter streamers as long as your transformer

is capable of handling the larger cap.

You will also get hotter streamers by changing to a poly dielectric

cap. Then a lot less of your primary energy will be wasted in a lossy

dielectric material.

As you increase the primary capacitance you must decrease the primary

inductance to keep the coil in tune. This is not a problem unless

carried too far. A low primary inductance leads to low efficiency.

The best method is to increase the toroid size as you increase the

primary capacitance. This will lower your resonant frequency and allow

you to keep the primary inductance high.

In your case I would say you need some retuning.

TTYL!

Mike Hammer

mhammer@misslink.net

X-Mailer: IPAD 2.0

Date: Fri, 19 Jun 1998 10:58:30 -0500

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: Faraday Cage Shielding

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

At 08:55 PM 6/19/98 +0930, you wrote:

>Hello,

>I am writing to ask about shielding my coil from my other electronics and

>radio control equipment. I have a shed up the back of the house which is

>usused so far. To make a faraday cage would I be able to line the concrete

>floor with chicken wire, making sure there is a connection between the

>walls and the floor, then put a big earth on the whole shed??

>Then in theory the shed should become a big Faraday cage, am I right????

>Would this be effecient?

>Any other suggestions??

>Thanks For Any Help

>Andrew

>

>meyer@clove.net.au

While you could do this it most likely is an unnecessary

expenditure. Tesla coils are very poor radiators and generally

dont cause any problems with electronics more than 20 feet or so away.

My computer is about 20 feet from my coil and I usually unplug it

before running the coil but sometimes I forget. Ive not had any

problems.

More important is keep the RF energy from the coil out of the

AC mains. This requires RF chokes in the HV leads and a good

AC line filter on the AC input to your transformer.

Also do NOT bring the AC mains ground anywhere near your coil.

A neon sign transformer should have its case grounded to RF ground-

not the AC mains ground. Also make sure your RF ground provides

a good low impedance path.

If you do these then you are not likely to have any problems.

Just be careful of sensitive electronics in the immediate vicinity

of your coil.

TTYL!

Mike Hammer

mhammer@misslink.net

X-Mailer: IPAD 2.0

Date: Fri, 19 Jun 1998 23:00:34 -0400

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Jim Ryan <jryan@exis.net>

Subject: Re: Vacuum Pumps . . .?

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

At 10:32 AM 6/19/98 -0500, you wrote:

>Mike et al:

>

>The compressors used in refigerators and the like, may be used as vacuum

Clip, Clip>>

>

>If all you need is a constant vaccum, why not use the suction from a vaccum

cleaner ?  It will pull at least 24"-25" of mercury.

Jim Ryan

Date: Fri, 19 Jun 1998 22:32:11 -0700

From: mike nicholas <705728@ican.net>

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Vacuum Pumps . . .?

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Eric Davidson wrote:

> Mike et al:

>

> The compressors used in refigerators and the like, may be used as vacuum

> pumps, but please be advised that the vacuum they produce is no where

> near perfect.

True, but who needs better than 29.??"of Hg, with these compressors I have pulled a 50

micron vacuum, these used to be sold as vacuum pumps until the techs realised they could

easily make there own.

> A perfect vacuum is difficult to acheive (actually,

> perfect is impossible) and requires diffusion pumping, cryopumping and

> molecular friction type pumps.  The good news is: for simple pumping, to

> remove air from a home made capacitor, requires little, vacuum wise. I'd

> order a simple siphon type pump for your water faucet, even before I

> messed around with an old refrigeration compressor. Hope this helps.

>

> mike nicholas wrote:

> >

> >     You can build a vacuum pump for free, you find many rotary compressors at the

> > dump off old fridges, they are round compressors with fins built in, I'm not sure

> > about militorrs or bars, but it can pull a near perfect vacuum, I used one for years

> > to take the moisture out of sealed systems, it may have cost me $5 to build (I made

> > it fancy)

> --

> Have a good day!

> Eric Davidson

> edavidson@icva.gov

From: "Choad" <choad69@geocities.com>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: EMI Filter hookup

Date: Sat, 20 Jun 1998 02:04:55 -0500

X-Mailer: IPAD 2.0

X-MimeOLE: Produced By Microsoft MimeOLE V4.72.2106.4

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

I bought 2 - 40A EMI filters from MECI for $9 each and I recall

someone saying something about if you use one after your variac

to hook the load up the the variac and line to the HV transformer.

Did I remember this right? I'm going to use 1 filter on either side

of my variac.

Thanks.

Date: Sat, 20 Jun 1998 10:07:54 -0600

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Daniel Lockhart <molly@cmn.net>

Subject: More Success (Re: spark length vs. power)

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

Hey all,

I just made some changes to my coil last night that made huge performance

increases. It's probably obvious to anybody but a newbe like me, but I was

awfully pleased so I though I would tell you about it.

First, it seems that I had my safety gap adjusted too jealously. After

moving everything out some, I was getting a whole lot more power through

the system. I don't know why I didn't think of this before! (duh!)

Even with more power, the output off the toroid didn't increase that much.

I decreased the cap size drastically (or so it seemed -- three small

bottles -- about 16%) and retuned to 14.75 turns on the primary. Now things

shaped up. Before I was getting sparks 16 and occasionally 18 inches long.

With the changes it is now consistently 20 to 22 inches. The sparks are

much healthier looking too. They have multiple forks and are much brighter.

This is off of saltwater caps and a 3 inch secondary, so I'm real pleased.

Recalling Jim's (Dsurfr@aol.com) past postings about cap size and spark

length, I would suggest to be sure to remember what the performance was

like before you added capacitance and don't hesitate to experiment with

less capacitance than you have already. It certainly paid off for me.

coil'n away,

Daniel Lockhart

X-Originating-IP: [208.16.145.188]

From: "Bill the arcstarter" <arcstarter@hotmail.com>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: HV Diode spec request??

Date: Sat, 20 Jun 1998 11:08:05 PDT

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

Coilers,

The local surplus place has a stack of HV diodes labeled as such:

VARO

H386C-12

48S137114

My usual sources don't reference this number.  Can anyone out there help me?

The thing is about the size of a cigarette with metal bands on the ends,

kind of like a fuse.  If it were a resistor I'd say it'd be good for

10W.  They claim this is a 20KV diode - but I'm more curious what sort

of current it'd pass.  I don't know the forwards voltage drop yet.

Thanks!

-Bill the arcstarter

Starting arcs in Cinci, OH

http://www.geocities.com/CapeCanaveral/Hangar/6160

Date: Sat, 20 Jun 1998 18:37:03 -0400

From: Chris Watkins <cwatkins@seidata.com>

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Large cap deconstruction

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

hey Guys

can you tell me the name + phone # of this place that is selling

surplus caps?  I am interested in the "K-Film" for some misc.

experiments

Thanks

Chris

cwatkins@seidata.com

snip

> a surplus catalog for big capacitors (the working kind  :-)

>they are selling rolls of Maxwell "K-Film", Maxwell "Brown Kraft Dielectric

>Paper" and dielectric films with aluminum already attached to one side.

>Maybe the "brown translucent paper" was indeed Kraft paper?

>

> I think that Surplus Sales of Nebraska sells this stuff too.

>

> What, exactly is Kraft paper?  Is it a trademark?  Any uses other than

> for capacitors?

>

> -Bill

Date: Sat, 20 Jun 1998 20:32:40 -0500

From: Bert Hickman <bert.hickman@aquila.com>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Organization: Stoneridge Engineering

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Large cap deconstruction

Sender: tesl-err@emachine.com

Bill the arcstarter wrote:

>

> Daniel wrote:

> >

> >Maybe everybody already knows about this stuff, I don't know. While looking

> >in a surplus catalog for big capacitors (the working kind  :-), I noticed

> >they are selling rolls of Maxwell "K-Film", Maxwell "Brown Kraft Dielectric

> >Paper" and dielectric films with aluminum already attached to one side.

> >

> >Maybe the "brown translucent paper" was indeed Kraft paper?

>

> I think that Surplus Sales of Nebraska sells this stuff too.

>

> What, exactly is Kraft paper?  Is it a trademark?  Any uses other than

> for capacitors?

>

> -Bill

>

Guys,

Capacitor-grade Kraft paper is a distant cousin to the paper used in

grocery store paper bags. However, the type that's used to make

paper-oil capacitors is processed to have much finer fibers, fewer

chemical impurities, more uniform thickness and density, is MUCH thinner

(typically only 0.35 - 0.5 mils!), and has less defects (holes,

conductive impurities, etc) than regular Kraft paper. Bill, from the

description, it sounds like the cap you disassembled used this type of

dielectric. Capacitor-grade Kraft paper is much more expensive than

regular Kraft paper (which, BTW, you can get in 36" wide rolls at most

Office Supply places). Capacitor Kraft paper is not usually obtainable

in the small quantities desired by experimenters.

Surplus Sales of Nebraska (1-800-244-4567, http://www.surplussales.com)

apparently bought a whole bunch of Kraft paper and polyester (Mylar)

capacitor dielectric film from Maxwell - upwards of 150,000 pounds of

the stuff! They offer rolls of either material to those who'd like to

try their hand at making high quality caps. They also offer surplus

Maxwell caps for those who don't want to "roll their own". Ask for a

copy of their Catalog 8. There's ALL Sorts of other really interesting

stuff in this catalog - but most of if ain't "hamfest" cheap however...

-- Bert --

Date: Sat, 20 Jun 1998 20:13:29 -0600

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Daniel Lockhart <molly@cmn.net>

Subject: Re: Large cap deconstruction

Cc: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

>hey Guys

>

>can you tell me the name + phone # of this place that is selling

>surplus caps?  I am interested in the "K-Film" for some misc.

> experiments

>

>Thanks

>Chris

>cwatkins@seidata.com

Chris,

The place I was referring to was:

Surplus Sales

1502 Jones Street

Omaha, NE 68102

phone: 402.346.4750

fax: 402.346.2939

http://www.surplusales.com

They carry a rather large selection of hv caps, including Maxwell high

energy caps. They carry the dielectric "K" film and Kraft dielectric paper

made by Maxwell, polyester films, Dupont Mylar films, and metalized films.

The "K" film is a roll 150 mm wide, 1000 m long and 16 um thick (!) @

$75.00 per roll.

I found this stuff kinda interesting too and wanted to get more specs about

it, regarding dielectric "k" strength and puncture voltage. Does anybody on

the list know? I'd think the only way to find out is to call Maxwell...

Daniel Lockhart

Date: Sat, 20 Jun 1998 21:31:49 -0500

From: Bert Hickman <bert.hickman@aquila.com>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Organization: Stoneridge Engineering

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Large cap deconstruction

Sender: tesl-err@emachine.com

Chris Watkins wrote:

>

> hey Guys

>

> can you tell me the name + phone # of this place that is selling

> surplus caps?  I am interested in the "K-Film" for some misc.

>  experiments

>

> Thanks

> Chris

> cwatkins@seidata.com

>

<SNIP>

Chris,

Surplus Sales of Nebraska, 1-800-244-4567, http://www.surplussales.com).

BTW, K-film is polyester (Mylar) film - great for DC but really poor for

Tesla Coil caps...

-- Bert --

X-Mailer: IPAD 2.0

Date: Sat, 20 Jun 1998 23:56:37 -0500

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: EMI Filter hookup

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

At 02:04 AM 6/20/98 -0500, you wrote:

>I bought 2 - 40A EMI filters from MECI for $9 each and I recall

>someone saying something about if you use one after your variac

>to hook the load up the the variac and line to the HV transformer.

>Did I remember this right? I'm going to use 1 filter on either side

>of my variac.

>

>Thanks.

Shaun,

You need to hook them up reversed from their normal orientation.

If you think about their original use and our intended use you

will see why.

In its intended use an AC line filter is meant to filter out

any RF hash, spikes or noise coming FROM the AC line to your

equipment.

In Tesla coil use we want to filter RF coming from our equipment

TO the AC line.

So thats why they should be installed reversed from normal.

Put one in the AC line to your neons after your variac.

The second should go on the AC input to your control box

or control panel.

TTYL!

Mike Hammer

mhammer@misslink.net

X-Originating-IP: [208.16.145.131]

From: "Bill the arcstarter" <arcstarter@hotmail.com>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Large cap deconstruction

Date: Sun, 21 Jun 1998 06:59:32 PDT

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

Chris Watkins wrote:

>hey Guys

>

>can you tell me the name + phone # of this place that is selling

>surplus caps?  I am interested in the "K-Film" for some misc.

> experiments

>

>Thanks

>Chris

>cwatkins@seidata.com

Sorry.  It's from Surplus Sales of Nebraska - but some rube stole my one

and only catalog off my desk - so I don't have the number either.

Try Yahoo or some other net searching device.

-Bill

X-Mailer: IPAD 2.0

Date: Sun, 21 Jun 1998 11:56:33 -0500

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: More Success (Re: spark length vs. power)

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

At 10:07 AM 6/20/98 -0600, you wrote:

>Hey all,

>

>I just made some changes to my coil last night that made huge performance

>increases. It's probably obvious to anybody but a newbe like me, but I was

>awfully pleased so I though I would tell you about it.

>

>First, it seems that I had my safety gap adjusted too jealously. After

>moving everything out some, I was getting a whole lot more power through

>the system. I don't know why I didn't think of this before! (duh!)

Careful here Daniel. Dont open the safety too wide. If its too wide

it will cease to be "safe". It should be adjusted slightly wider

than your neon can break down on its own. If this setting results

in lots of safety gap firings then your main gap may be too wide.

An out of tune situation can also increase safety gap firings.

>

>Even with more power, the output off the toroid didn't increase that much.

>I decreased the cap size drastically (or so it seemed -- three small

>bottles -- about 16%) and retuned to 14.75 turns on the primary. Now things

>shaped up. Before I was getting sparks 16 and occasionally 18 inches long.

>With the changes it is now consistently 20 to 22 inches. The sparks are

>much healthier looking too. They have multiple forks and are much brighter.

>This is off of saltwater caps and a 3 inch secondary, so I'm real pleased.

Thats not too bad for a 3 inch diameter secondary. My old 3 incher

maxed out at 18 inches before I moved up to a 4 incher.

>

>Recalling Jim's (Dsurfr@aol.com) past postings about cap size and spark

>length, I would suggest to be sure to remember what the performance was

>like before you added capacitance and don't hesitate to experiment with

>less capacitance than you have already. It certainly paid off for me.

>

You made a good point about cap size.

The mathematical matching cap size for your neon may NOT be

the best size for your coil. Its only a guide. I have found

that somewhat less than the matching size allowed for more

primary inductance and better output. I always got better

and better results as my primary inductance was increased.

>coil'n away,

>

>Daniel Lockhart

TTYL!

Mike Hammer

mhammer@misslink.net

Date: Sun, 21 Jun 1998 15:21:37 -0500

From: Mark Olson <molson1@isd.net>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: HV Diode spec request??

Sender: tesl-err@emachine.com

I've got a pile of Varo diodes marked "H 486C 2" as well.  If anyone has any

references to Varo diodes, I'd be interested in knowing the specs - I have about

50 of these things.

thanks!

Mark Olson

Bill the arcstarter wrote:

> Coilers,

>  The local surplus place has a stack of HV diodes labeled as such:

>

> VARO

> H386C-12

> 48S137114

>

> My usual sources don't reference this number.  Can anyone out there help me?

>

> The thing is about the size of a cigarette with metal bands on the ends,

> kind of like a fuse.  If it were a resistor I'd say it'd be good for

> 10W.  They claim this is a 20KV diode - but I'm more curious what sort

> of current it'd pass.  I don't know the forwards voltage drop yet.

>

> Thanks!

>

> -Bill the arcstarter

> Starting arcs in Cinci, OH

> http://www.geocities.com/CapeCanaveral/Hangar/6160

>

Date: Sun, 21 Jun 1998 14:48:03 -0600

From: Jonathan Bazemore <jpbazemo@ouray.cudenver.edu>

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Vacuum Pumps . . .?

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Slight problem--my understanding, and please correct me if I'm

wrong (and I probably am) is that molecular sieve needs to be part

of a two-stage vacuum pump system; the molecular sieve itself is

simply incapable of evacuating any significant volume.  However, if

you use a rotary oil pump, or perhaps even a water aspirator, you

can then use the sieve on top of the "soft vacuum" to lower the pressure

even further.

Being able to establish and maintain a vacuum has quite a number of

obviously useful benefits for high-voltage tinkerers and enthusiasts; for

example, you can evacuate an erlymeyer (sp?) flask, back-fill it with

laser gas, and put it near a coil or in the microwave.  Wearing goggles

is recommended, and using implosion containment methods.

From: <Dsurfr@aol.com>

Date: Sun, 21 Jun 1998 16:53:37 EDT

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: More Success (Re: spark length vs. power)

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Dan: what size transformer are you using & how many bottles?

Thanks, JIm

Date: Sun, 21 Jun 1998 19:03:56 -0400 (EDT)

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: fxphoto@centuryinter.net (fxphoto)

Subject: Re: Large cap deconstruction

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Chris,

The URL is    http://www.surplussales.com/Maxwell.html

Bill Turbett

>Chris Watkins wrote:

>>hey Guys

>>

>>can you tell me the name + phone # of this place that is selling

>>surplus caps?  I am interested in the "K-Film" for some misc.

>> experiments

>>

>>Thanks

>>Chris

>>cwatkins@seidata.com

>

>Sorry.  It's from Surplus Sales of Nebraska - but some rube stole my one

>and only catalog off my desk - so I don't have the number either.

>Try Yahoo or some other net searching device.

>

>-Bill

>

X-Mailer: IPAD 2.0

Date: Mon, 22 Jun 1998 08:42:30 +0300

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Marco Denicolai <marco@vistacom.fi>

Subject: First light and problems

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Last weekend I got my ground and my first TC ready: I had no variac and I fired

it up at full power. I had before measured primary and secondary to resonate

both at 240 KHz so the primary tap should have been about in the correct

position.

IT WORKED! I had about 1 meter long sparks running from the toroid into free

air, almost perfectly horizontal. After the first minute I run into problems...

1. I got 1-2 sparks from the strike rail hitting the middle of the secondary.

I suppose my strike rail was too much near the secondary (the sparks made a

10" travel from strike rail to the secondary body).

2. Then I got some sparks from the inner primary turns to the secondary base

and primary to secondary about 3" height from its base. Because it was day

and I was (safely) far from the coil, I couldn't notice any corona or aurea

effects near the primary base, but I suppose the problem was the well known

overcoupling.

3. One diode from my MOT doubler blow up twice (always the same one).

My coil data:

Secondary 4.33" diameter, 19.7" wind with AWG 26 enamel wire, 7 layers of

epoxy paint. Toroid 3.15" diameter tube, 11.4" outer diameter, 12.7 pF.

Mounted 1.6" over the secondary top. The pair resonates at 240 KHz.

Primary 7 turns of 3/16" copper tube, inner diameter 5.3", turn spacing

0.3". Flat pancake. Clearance from inner turn to secondary 0.5".

Secondary capacitor 0.05uF 50KV. Set to resonate also at 240 KHz.

Primary turns and secondary first turns almost on the same horizontal plane.

Tank supply made of two MOTs and two voltage doublers, output 8 KVAC,

almost 0.5A (I believe). A third MOT primary (with shorted secondary)

was in series with the 2 MOT primaries on the 220V side to work as a shunt.

Spark gap is a RQ static gap with blower and two series of 3 of 0.275" gaps

connected in parallel.

About the following solutions I would like very much some comments:

Problem 1: I plan to rework my strike rail as an open ring about 3" high

from the secondary base, about 8" horizontal clearance from the secondary

4.33" pipe.

Problem 2: rework the primary, increase the clearance between inner turn and

secondary to 1", make it a 15 degree upside-down saucer to distribute better

it magnetic field and decrease coupling.

Problem 3: the two MOT employed were a 900W and a huge 1500W. The diode of

the doubler of the last one keeps on blowing. I replaced the 1500W with another

900W and I hope this will stress less the diode.

Your opinion is really welcome...

Marco Denicolai

X-Mailer: IPAD 2.0

Date: Mon, 22 Jun 1998 08:46:36 +0300

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Marco Denicolai <marco@vistacom.fi>

Subject: How to rise the secondary?

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

A construction problem to me is how to build the secondary (or the primary)

so that one of them can be lifted in respect to the other to set up the

coupling. Can anybody suggest a good solution?

Marco Denicolai

X-Mailer: IPAD 2.0

Date: Mon, 22 Jun 1998 08:56:05 +0300

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Marco Denicolai <marco@vistacom.fi>

Subject: Damaged secondary

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

After my first test I got my secondary hit in 5 spots by sparks. Only the

surface paint is blown away there, no sparks going inside the pipe and no

turns really fused or vaporized.

Is there any way to repair it or should I wind a new secondary?

How it is possible to setup a TC for the best coupling if (when overcoupling)

sparks hit your secondary, damages it and you have to wind a new one?

Marco Denicolai

Date: Mon, 22 Jun 1998 06:25:11 -0500

From: Bert Hickman <bert.hickman@aquila.com>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Organization: Stoneridge Engineering

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: How to rise the secondary?

Sender: tesl-err@emachine.com

Marco Denicolai wrote:

>

> A construction problem to me is how to build the secondary (or the primary)

> so that one of them can be lifted in respect to the other to set up the

> coupling. Can anybody suggest a good solution?

>

>  Marco Denicolai

Marco,

I usually make small spacers out of either wood or plastic that can be

plsced under the secondary coil to lift it relative to the primary. By

making a batch of these of various thicknesses (1/8" - 1") and stacking

them, you can "shim" the coil to the right height.

-- Bert --

Date: Mon, 22 Jun 1998 06:35:52 -0500

From: Bert Hickman <bert.hickman@aquila.com>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Organization: Stoneridge Engineering

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Damaged secondary

Sender: tesl-err@emachine.com

Marco Denicolai wrote:

>

> After my first test I got my secondary hit in 5 spots by sparks. Only the

> surface paint is blown away there, no sparks going inside the pipe and no

> turns really fused or vaporized.

> Is there any way to repair it or should I wind a new secondary?

>

> How it is possible to setup a TC for the best coupling if (when overcoupling)

> sparks hit your secondary, damages it and you have to wind a new one?

>

<SNIP>

Marco,

As long as adjacent turns aren't fused together to form shorted turns,

you should be able to simply dab on some Glytol or multiple layers of

polyurethane varnish on the damaged areas. On a new coil, you'd

typically start out with the start of the secondary winding elevated a

few inches above the plane of the primary so that you had relatively low

coupling to begin with.

Once you've verified that your coil is in tune (by carefully adjusting

for maximum spark length at lower power levels), then begin running the

coil at higher levels IN THE DARK to see if you're running into any

excessive corona problems. If the coil runs OK at maximum power, THEN

you can begin the increase coupling by reducing the vertical elevation

of the secondary. Reduce the spacer height by about 1/4 - 3/8" at a time

until you see signs of overcoupling (typically racing sparks or

excessive corona - like a glowing Christmas tree between the top of the

secondary winding and the primary strikerail). Raise the secondary a bit

and you're there!

Later on, if you make significant changes (such as changing toroid

sizes), you'll need to redo the above steps.

-- Bert --

Date: Mon, 22 Jun 1998 06:58:21 -0500

From: Bert Hickman <bert.hickman@aquila.com>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Organization: Stoneridge Engineering

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: First light and problems

Sender: tesl-err@emachine.com

Marco Denicolai wrote:

>

> Last weekend I got my ground and my first TC ready: I had no variac and I fired

> it up at full power. I had before measured primary and secondary to resonate

> both at 240 KHz so the primary tap should have been about in the correct

> position.

>

> IT WORKED! I had about 1 meter long sparks running from the toroid into free

> air, almost perfectly horizontal. After the first minute I run into problems...

>

> 1. I got 1-2 sparks from the strike rail hitting the middle of the secondary.

> I suppose my strike rail was too much near the secondary (the sparks made a

> 10" travel from strike rail to the secondary body).

>

> 2. Then I got some sparks from the inner primary turns to the secondary base

> and primary to secondary about 3" height from its base. Because it was day

> and I was (safely) far from the coil, I couldn't notice any corona or aurea

> effects near the primary base, but I suppose the problem was the well known

> overcoupling.

>

> 3. One diode from my MOT doubler blow up twice (always the same one).

>

> My coil data:

>

> Secondary 4.33" diameter, 19.7" wind with AWG 26 enamel wire, 7 layers of

> epoxy paint. Toroid 3.15" diameter tube, 11.4" outer diameter, 12.7 pF.

> Mounted 1.6" over the secondary top. The pair resonates at 240 KHz.

>

> Primary 7 turns of 3/16" copper tube, inner diameter 5.3", turn spacing

> 0.3". Flat pancake. Clearance from inner turn to secondary 0.5".

> Secondary capacitor 0.05uF 50KV. Set to resonate also at 240 KHz.

> Primary turns and secondary first turns almost on the same horizontal plane.

>

> Tank supply made of two MOTs and two voltage doublers, output 8 KVAC,

> almost 0.5A (I believe). A third MOT primary (with shorted secondary)

> was in series with the 2 MOT primaries on the 220V side to work as a shunt.

>

> Spark gap is a RQ static gap with blower and two series of 3 of 0.275" gaps

> connected in parallel.

>

> About the following solutions I would like very much some comments:

>

> Problem 1: I plan to rework my strike rail as an open ring about 3" high

> from the secondary base, about 8" horizontal clearance from the secondary

> 4.33" pipe.

>

> Problem 2: rework the primary, increase the clearance between inner turn and

> secondary to 1", make it a 15 degree upside-down saucer to distribute better

> it magnetic field and decrease coupling.

>

> Problem 3: the two MOT employed were a 900W and a huge 1500W. The diode of

> the doubler of the last one keeps on blowing. I replaced the 1500W with another

> 900W and I hope this will stress less the diode.

>

> Your opinion is really welcome...

>

<SNIP>

Marco,

Congratulations on first light! At least it did work for a while - many

coilers only see smoke the first time... :^)

Problem 1: The strikerail should be the same diameter as the outermost

turn of the primary, and only about 1.5" above the plane of the primary.

Try to make it from 1/4" or 3/8" diameter copper tubing to reduce

incidence of corona.

Problem 2: Go with at least 1.25 - 1.5" of clearance all around between

the secondary and primary. Also, stay with a flat primary for simplicity

- there's really little advantage to going to a different geometry.

Elevate the secondary versus the primary to change coupling - it's easy

and easily adjustable this way.

Problem 3: I'm not sure I understand the three MOT's - thought you were

running with 2 MOT's with the third simply being used as a current

limiting inductor. Preventing your diodes from shorting is going to

continue to be a problem. These will continue to be the "weak link " in

your system. You'll need to substantially increase the PIV rating of the

diodes you use, and you may also need to get diodes having a higher

current rating. When the gap fires it generates broadband RFI which will

conduct back into your voltage multiplier circuit, often taking out the

diodes. Additional RF filtering may be necessary to adequately protect

your diodes...

-- Bert --

Date: Mon, 22 Jun 1998 16:39:58 -0700

From: Gene Waldo <gwaldo@1stnetusa.com>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Organization: S.A.P.I.E.N.T. (Surly, Agressive, Politically Incorrect Enemies of Non Thinkers)

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Vacuum Pumps . . .?

Sender: tesl-err@emachine.com

Jonathan Bazemore wrote:

>

>     Slight problem--my understanding, and please correct me if I'm

> wrong (and I probably am) is that molecular sieve needs to be part

> of a two-stage vacuum pump system; the molecular sieve itself is

> simply incapable of evacuating any significant volume.  However, if

> you use a rotary oil pump, or perhaps even a water aspirator, you

> can then use the sieve on top of the "soft vacuum" to lower the pressure

> even further.

Johnathan, you are essentially correct, but most 'hard vac' pumps

require much lower pressures to even begin to operate than could be

obtained by an aspirator, or even cheap mechanical pumps.  For they

generally do not 'pump gas', but actually only remove 'stray molecules'

from an existing "vacuum", simply stated.

I have no experience with the Molecular Seive, but my experience with

diffusion pump systems causes me to bet that this is the case here, too.

Look into the Sci. Amer. sites mentioned: where the Mol Seive plans

were illustrated.

-----------------------------

Gene Waldo -  gwaldo@1stnetusa.com  - "Igor in Residence"

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Vacuum Pumps . . .?

Date: Mon, 22 Jun 1998 19:49:12 CDT

From: "Kent Vander Velden" <graphix@iastate.edu>

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

>> you use a rotary oil pump, or perhaps even a water aspirator, you

>> can then use the sieve on top of the "soft vacuum" to lower the pressure

>> even further.

What is a water aspirator?  Is this the vacuum pump that someone else

mentioned that builds it's vacuum using flowing water?  Do these things

really work?  Where can I get more information on them?

Thanks.

---

Kent Vander Velden

kent@iastate.edu

Date: Tue, 23 Jun 1998 10:15:53 -0500

From: Eric Davidson <edavidson@icva.gov>

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Vacuum Pumps . . .?

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Kent:

Yes, a water aspirator creates a vacuum using water flowing through a

venturi like orifice.  They really do work. Do they create the same

level of vacuum as a vacuum pump: NO. Are they as expensive as even a

cheesy vacuum pump: NO. Water aspirators create pretty decent vacuum in

spite of their simple construction. The vacuum produced is proportional

to the temperature of the water (its vapor pressure) and its velocity

through the venturi.  For best results use COLD water at high flow.

Have a valve to purge the vacuum BEFORE you turn off the water, or water

will be drawn into the system. Aspirators can be found in any scientific

catalog (Fisher etc.), about $9.00 for the Nalgene ones.  Vacuum is

around 28.5".  Plenty good for removing air from a ho-made capacitor.

Hope this helps.

Kent Vander Velden wrote:

>

>   What is a water aspirator?  Is this the vacuum pump that someone else

> mentioned that builds it's vacuum using flowing water?  Do these things

> really work?  Where can I get more information on them?

>

>   Thanks.

>

> ---

> Kent Vander Velden

> kent@iastate.edu

--

Have a good day!

Eric Davidson

edavidson@icva.gov

From: Roy Minnich <rminnich@rolling-meadows.lib.il.us>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: RE: Vacuum Pumps . . .?

Date: Tue, 23 Jun 1998 11:09:40 -0500

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

Hi Eric,

> Aspirators can be found in any scientific

>catalog (Fisher etc.), about $9.00 for the Nalgene ones.  Vacuum is

>around 28.5".  Plenty good for removing air from a ho-made capacitor.

>Hope this helps.

Thanks for the great info. on aspirators.  I have never used or even seen one

so a good explanation is welcome.

Roy

Date: Tue, 23 Jun 1998 09:49:10 -0700

From: Gene Waldo <gwaldo@1stnetusa.com>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Organization: S.A.P.I.E.N.T. (Surly, Agressive, Politically Incorrect Enemies of Non Thinkers)

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Vacuum Pumps . . .?

Sender: tesl-err@emachine.com

Kent Vander Velden wrote:

>

> >> you use a rotary oil pump, or perhaps even a water aspirator, you

> >> can then use the sieve on top of the "soft vacuum" to lower the pressure

> >> even further.

>

>   What is a water aspirator?  Is this the vacuum pump that someone else

> mentioned that builds it's vacuum using flowing water?  Do these things

> really work?  Where can I get more information on them?

>

>   Thanks.

> Kent Vander Velden

> kent@iastate.edu

Yes, Kent, there really are aspirators. They really work fine for

sucking down a capacitor, and there would be no advantage of 2nd-staging

your pump with any more clutter. Should be able to get one from a decent

plumbing/hardware shop or science supply house, for under $10.

Just be sure to use a sly plumbing/water trap/valving system to prevent

the back flow of water into your capacitor!

Also sly practice will be required to prevent the initial oily froth

from your cap being sucked into the aspirator, or whatever pump you use.

Now you may have 2 traps: oil and water; clear champaign bottles, 2 hole

rubber stoppers and Cu tube/tygon will suffice. Be sure to 'box' your

bottles in case of any implosions! Glass will leave these scenes at near

sonic velocities!

Water flows thru a venturi arangement in the aspirator and sucks air

into the water stream. These devices can achieve close to 28-29 In Hg,

which can be adequately measured by a cheap mechanical vacuum (Bourdon)

gauge (which will not be adequate for the levels attainable with the

'hard' pumps).

If one were to push ahead with the Seive for other purposes, I would

suggest not using an aspirator as a roughing pump; get a good mechanical

pump, and lube it with a good grade of vacuum oil.

The staged valving systems required for HiVac applications require Zero

leakage and Zero outgassing of components.  Mtls/hardware/construction

costs will be orders of magnitude above those for simple 'cap-suck'

applications!

Now you apply LN2 to your cold trap, as most remaining oil vapours from

the pumps and any other molecules tend to contaminate HiVac pumps and guages.

Note that this may involve the ambient pressure handling of LN2: any

such plumbing will acquire a condensate of LOx: this will collect and

drip from low points. LN2 is a few deg colder than LOx.  Free LOx is a

dangerous companion!

Below 'rough' vacuum, you can forget pipe dope and Teflon tape! 'O'rings

and/or brazed and/or SS flared fittings will be required. Also large

dia. plumbing and valves, >1": as at these pressures, gas is not

'pumped'; molecules 'wander' randomly towards the exit, and should not

encounter obstacles, such as any small passsages.

Then you will need sophisicated pressure measurement devices which may

cost more than your pumping systems. Most of these devices must also be

'staged', as they will generally be destroyed by use at pressures higher

than their rated operating.

-------------------------------------

Gene Waldo -  gwaldo@1stnetusa.com  - "Igor in Residence"

X-Mailer: IPAD 2.0

Date: Wed, 24 Jun 1998 00:39:33 -0400

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Jim Ryan <jryan@exis.net>

Subject: Vaccum Stuff

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Hi all, Does anyone know how much vaccum a vacuum cleaner pulls?

Mine is a 9.2 amp Sears. It can suck a bubble on your hand.

Jim

X-Mailer: IPAD 2.0

Date: Wed, 24 Jun 1998 09:46:48 +0300

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Marco Denicolai <marco@vistacom.fi>

Subject: More lights and NO problems

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

A few hours ago I got my TC tuned and reliably (?) working. Here is what I did:

1. I changed my strike rail: Outer diameter is now 15.8" and it is 2.3"

elevated on the primary (primary outer diameter is about  13.8". Anyway,

I didn't ground it (just to see how it goes) and I never got any spark

from secondary or top toroid to primary. Next I'll try grounding it.

2. I rised the secondary: now it is 1.5" higher than the primary (first

turn to first turn).

3. I repaired the damaged secondary by cleaning the shorted windings with the

aid of a microscope and painting the spots with epoxy varnish. Works fine.

4. I replaced the 1500W MOT "beast" (VERY heavy, not like a PIG but...) with

another 900W and replaced the broken diode. The lower current sourced can be

easily appreciated running the spark gap just on the tank (no primary or cap).

Sparks are more slim and don't "bang" so much.

I didn't rework (yet) the primary, nor rewind the secondary. Instead I followed

a strict tuning procedure: first with half power and 1 gap, then full

power and 2 gaps, then increase coupling by lowering the secondary. I got

my TC tuned and running well in 30 minutes.

I have been able to run 30 sec. sessions for about 1 hour, getting blue

contoured red forked spikes from the toroid, about 28" long. No diode break,

no flashover, no heat in the caps and no damage to my stereos and computer.

It has been raining here two days almost no-stop and it just quit before I

run the TC, so the moisture level was very high. I hope today (a sunny day)

to get even longer sparks.

It seems like the MOT tank is now robust and reliable. I'll post later

more info on it and my results.

Thanks to everybody for your help: I really needed it.

Marco Denicolai

X-Mailer: IPAD 2.0

Date: Thu, 25 Jun 1998 11:58:38 -0500

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Mike Hammer <mhammer@misslink.net>

Subject: unsubscribing from list/EVERYONE READ

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

OK people. Looks like its time for me to address

the subject of unsubscribing from the list once again.

First of all  DO NOT POST UNSUBSCRIBE REQUESTS TO THE LIST.

The commands must be sent to the list server not to the list.

Please make note of this and keep a copy for future

reference. If you have difficulty unsubscribing

DO NOT post a request to the list. Email me direct

instead.

To quit the list, either permanently or temporarily, such as for vacation,

please send a message to tesla-2-request@emachine.com with one of the

following keywords in the body of the message:  Delete, Remove or

Unsubscribe.  Case is not important.

If you should later wish to rejoin the list, please send a message to

tesla-2-request@emachine.com with one of the following keywords in the

body of the message:  Join, Add or Subscribe.  Case is not important.

One additional note. If you unsubscribe and then try to resubscribe

yourself later on it must be with the same email address. If

you change addresses then the list server sees you as a new subscriber.

New subs still need to go through me.

If you need to change your address email me with the change

rather than unsubscribing.

TTYL!

Mike Hammer

mhammer@misslink.net

X-Mailer: IPAD 2.0

Date: Thu, 25 Jun 1998 12:04:37 -0500

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: Vacuum Stuff

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

At 12:39 AM 6/24/98 -0400, you wrote:

>Hi all, Does anyone know how much vaccum a vaccum cleaner pulls?

>Mine is a 9.2 amp Sears. It can suck a bubble on you hand.

>

>Jim

Jim,

I really dont know but I bet it will suck enough to do a cap.

A lot depends on the type of vacuum cleaner motor. Some of

them just spin faster and lose efficiency as the airflow is

restricted. Why not give a try and let us all know how it works out.

I heard of one fellow that pumped down some caps using a vacuum line

off of his car. Just goes to show that extreme vacuum is not needed.

The interest in the molecular sieve vacuum pump may be misplaced.

Such a pump will create hard vacuums but is very slow and not what

you want for cap building.

TTYL!

Mike Hammer

mhammer@misslink.net

From: wwl@netcomuk.co.uk (Mike Harrison)

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Vacuum Stuff

Date: Thu, 25 Jun 1998 20:23:04 GMT

Organization: White Wing Logic

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

>off of his car. Just goes to show that extreme vacuum is not needed.

>The interest in the molecular sieve vacuum pump may be misplaced.

>Such a pump will create hard vacuums but is very slow and not what

>you want for cap building.

.. but could come in handy after you've built the cap and have lots of

volts to play with...!

____                                                           ____

_/ L_/  Mike Harrison / White Wing Logic / wwl@netcomuk.co.uk  _/ L_/

_/ W_/  Hardware & Software design / PCB Design / Consultancy  _/ W_/

/_W_/  Industrial / Computer Peripherals / Hazardous Area      /_W_/

X-Originating-IP: [208.16.149.86]

From: "Bill the arcstarter" <arcstarter@hotmail.com>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Our Coil #2 - and we're off!

Date: Sat, 27 Jun 1998 19:53:53 PDT

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

Hi Coilers.

The list has been quiet lately.  Has all this vacuum talk scared off

all the people actually making telsa coils??  Or is everyone taking

summer vacation?

Paul and I just finished winding our secondary.  6.25 inch diam, #22

magnet wire, about 28 inches widing length.  There's polyurethane all

over the place, myself, the bench, the winding motor...  Did you know

that (when you're not looking) polyurethane will dissolve those cheap

plastic beer cups??? :(

Of course all this hard work makes a person thirsty.  Much scotch and

beer was consumed... ;-)  Gene - wish you were there!  (That might

explain why the turns at the far end were a little wobbly)

Wish us luck on the rest!  Remember - any voltage that won't jump at

least 24 inches is "low voltage".

-Bill the arcstarter

Starting arcs in Cinci, OH

http://www.geocities.com/CapeCanaveral/Hangar/6160

X-Originating-IP: [208.16.145.150]

From: "Bill the arcstarter" <arcstarter@hotmail.com>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Dead pulse cap pics are up!

Date: Sun, 28 Jun 1998 06:34:17 PDT

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

Coilers,

Pictures and narrative of our "Alien Pulse Capacitor Autopsy" are

available on fleischwulf's page at:

http://www.geocities.com/CapeCanaveral/Hangar/9561/deadcap.html

-Bill the arcstarter

Starting arcs in Cinci, OH

http://www.geocities.com/CapeCanaveral/Hangar/6160

From: Tom Stock <tomstock@geocities.com>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Coil Update

Date: Sun, 28 Jun 1998 08:16:33 -0700

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

Ok, I rebuilt my first coil last night. Now, everything is self contained

in a little table with wheels, so I can move it around easily. This coil

has 4.5" x 22" secondary, 14 turn flat primary, and a 10" x 2.5" toroid

discharge terminal. Right now I'm operating without chokes since I'm only

using one 15kv Neon (this will be fixed as soon as I find some donuts). My

caps are 10 reddog bottles.

Right now I can only get about 10-12" of spark, and obviously I want more!

I tried a larger discharge terminal made with 4.5" duct and pie pans (all

covered with aluminum tape) but it was too large, and when tuned spark

wouldn't break out without a grounded object near the terminal.

I don't yet have a fan on my spark gap. Will adding a fan make a large

improvement with a single 30ma neon? I'm planning to do this soon as well.

If someone out there has a set of 2.5" ferrite donuts they'll sell me

cheap, please let me know. I have 3 more 15kv neons patiently waiting.

-tom

tomstock@geocities.com

Date: Sun, 28 Jun 1998 09:18:16 -0700

From: Gene Waldo <gwaldo@1stnetusa.com>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Organization: S.A.P.I.E.N.T. (Surly, Agressive, Politically Incorrect Enemies of Non Thinkers)

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Our Coil #2 - and we're off!

Sender: tesl-err@emachine.com

Bill the arcstarter wrote:

>

> Hi Coilers.

>  The list has been quiet lately.  Has all this vacuum talk scared off

> all the people actually making telsa coils??  Or is everyone taking

> summer vacation?

>

>  Paul and I just finished winding our secondary.  6.25 inch diam, #22

> magnet wire, about 28 inches widing length.  There's polyurethane all

> over the place, myself, the bench, the winding motor...  Did you know

> that (when you're not looking) polyurethane will dissolve those cheap

> plastic beer cups??? :(

Bill,

Can it be that this is a built-in safety mode; to keep drunkards from

consuming too much polyurethane?

>  Of course all this hard work makes a person thirsty.  Much scotch and

> beer was consumed... ;-)  Gene - wish you were there!  (That might

> explain why the turns at the far end were a little wobbly)

Can't imagine that my presence would have reduced the 'wobbly'! But

that doesn't mean that I wouldn't have enjoyed being there! You guys run

a FUN! shop!

Trying to get back to my junkyard rummages- several (previously

mentioned) lg caps in the yard! Some have expressed interest in them.

Will get specs out in due course.

>  Wish us luck on the rest!  Remember - any voltage that won't jump at

> least 24 inches is "low voltage".

>

> -Bill the arcstarter

> Starting arcs in Cinci, OH

> http://www.geocities.com/CapeCanaveral/Hangar/6160

Just romped thru your site!

Commercial possibilities: Try some REAL money: US Ag coin: quarters,

halves, dollars: shrunken to jewlery: rings, earings, pendants etc....

Just now have to create a "TESLA BABES" mkt! ~~<[:-)

Don't know legals, but since our gov't. has divested itself from issuing

actual money*, and now is into script, it may be OK, since Ag coin* must

by now be either relics and/or qurios?

* The classic definition of money is a 'store of wealth'; which surely

our current script and notes are not!

Who got and got away with the Ag which used to circulate in our Nation?

Back on topic:

Coilers,

Dan came up with an ideal containment for our 12 40 oz Bud bottles for

the SW cap.  I'm currently epoxying the lid/electrodes and wiring the 12

together.  Pics shortly, on Dan's Site.

QUESTION: how 'SATURATED' should the NaCl sol'n be? Pre any actual test

for conductivity, methinks a point of diminishing return may be reached

as 'satuation' reaches about 60-80% (or less?)????  Any data?

I want to make the thing a test bed, with a variable depth neg SW bath

to 'tune-on-the-fly', in lieu of fixed outer foils. This will allow us

to jump the pri tap in increments and "tweak" the cap 'hot', for max

spark length.

Dan has some resistance to a thru-hull ftg & valve in the plastic tub; I

guess I'll have buy the damn thing from him!

My electronics store guy has offered to supply his cap meter to help us

calibrate the thing. Don't know if he has a Megger, or if that would be

of any value... should be, I guess? .

------------------------------------------------------

Gene Waldo -  gwaldo@1stnetusa.com  - "Igor in Residence"

X-Mailer: IPAD 2.0

Date: Sun, 28 Jun 1998 11:24:58 -0500

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: Our Coil #2 - and we're off!

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

At 07:53 PM 6/27/98 PDT, you wrote:

>Hi Coilers.

> The list has been quiet lately.  Has all this vacuum talk scared off

>all the people actually making telsa coils??  Or is everyone taking

>summer vacation?

More likely hiding in front of the air conditioner.

Those of us that work in the garage have found 110 deg plus

temps put a real damper on our enthusiasm.

>

> Paul and I just finished winding our secondary.  6.25 inch diam, #22

>magnet wire, about 28 inches widing length.  There's polyurethane all

>over the place, myself, the bench, the winding motor...  Did you know

>that (when you're not looking) polyurethane will dissolve those cheap

>plastic beer cups??? :(

You finally going to give your poor 4 inch secondary a rest?

About time. You've had that thing on the razor edge of destruction.

I expect to see some 7 footers from this very soon.

>

> Of course all this hard work makes a person thirsty.  Much scotch and

>beer was consumed... ;-)  Gene - wish you were there!  (That might

>explain why the turns at the far end were a little wobbly)

>

> Wish us luck on the rest!  Remember - any voltage that won't jump at

>least 24 inches is "low voltage".

Or put another way, "Any current less than 10 amps is leakage"

TTYL!

Mike Hammer

mhammer@misslink.net

X-Mailer: IPAD 2.0

Date: Sun, 28 Jun 1998 11:44:49 -0500

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: Coil Update

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

At 08:16 AM 6/28/98 -0700, you wrote:

>Ok, I rebuilt my first coil last night. Now, everything is self contained

>in a little table with wheels, so I can move it around easily. This coil

>has 4.5" x 22" secondary, 14 turn flat primary, and a 10" x 2.5" toroid

>discharge terminal. Right now I'm operating without chokes since I'm only

>using one 15kv Neon (this will be fixed as soon as I find some donuts). My

>caps are 10 reddog bottles.

Tom,

One neon can fail just as readily as more. Get some protection

on there ASAP. If you cant find any toroids then some air core

chokes on PVC pipe will do if backed up with some series resistance.

>

>Right now I can only get about 10-12" of spark, and obviiously I want more!

>I tried a larger discharge terminal made with 4.5" duct and pie pans (all

>covered with aluminum tape) but it was too large, and when tuned spark

>wouldn't break out without a grounded object near the terminal.

The way to get more spark is with a larger toroid. Right now your system

efficiency isnt high enough to generate the level of voltage required

for breakout of this larger toroid.

What you need is not a smaller toroid but higher efficiency.

>

>I don't yet have a fan on my spark gap. Will adding a fan make a large

>improvement with a single 30ma neon? I'm planning to do this soon as well.

Depends on your gap. Is this a single gap between a couple of bolts

or a RQ style cylinder gap with multiple electrodes. The fan on a cylinder

gap is mainly for cooling. Allows for longer run times. If you have a good

strong fan it will help improve quenching.

>

>If someone out there has a set of 2.5" ferrite donuts they'll sell me

>cheap, please let me know.

Someone told me they bought some ferrite cores from Electronic Goldmine

and had used them with sucesss. I dont remember who that was now.

Try Electronic Goldmine at: http://www.goldmine-elec.com/

> I have 3 more 15kv neons patiently waiting.

What you DONT need right now is more power. Increasing your

power level might by you a couple more inches but most of that

additional power will just be dissipated in your system losses.

Reduce these losses first then the additional power will have

the greatest impact.

First thing I would do is measure your total gap spacing.

If your total spacing is less than about .25 inch add an additional

electrode or two. The additional spacing will increase your primary

energy level. Dont go too far as this puts your neon at greater risk.

About .300 inch is pretty much the maximum for 15kv.

Try adjusting your coupling next. Bring the secondary down closer

to the primary to increase coupling and therefore transfer more energy

to the secondary. If you start getting pri/sec arcing or heavy corona

then back off a little. Increasing the coupling will put additional

strain on your spark gap. Thats why a good gap is important. A good

gap allows you to run tighter coupling without losing efficiency.

Replace those bottle caps with a poly cap. You will be amazed at the

difference. A quite large percentage of your primary energy is just

going into heating that glass dielectric. Where you now have

thin spindly purple streamers a poly cap will give you thicker hotter

brighter blue streamers.

Keep us posted.

TTYL!

Mike Hammer

mhammer@misslink.net

Date: Sun, 28 Jun 1998 10:20:16 -0700

From: Gene Waldo <gwaldo@1stnetusa.com>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Organization: S.A.P.I.E.N.T. (Surly, Agressive, Politically Incorrect Enemies of Non Thinkers)

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Variable  SW Cap specs

Sender: tesl-err@emachine.com

Coilers,

Just ran some numbers on the cap: With an active variable depth of 8",

bottle dia: 3.575', C: 12.25", the perbottle area: 109.23 in^2 x 12

bottles yields a max operating area of 1,310.76 In^2. Divided by 8", I

get 163.845In^2/In. of depth.

I figure .01" of dielectric thickness of bottle glass. We will try to

plot C v In. Depth. Next, I guess , will be some sight gage on the

containment box.  Then I suppose dye in the water bath to make it all

somewhat visible in the murky flickering darkness of our dungeon! What a

tangled web!

--

Gene Waldo -  gwaldo@1stnetusa.com  - "Igor in Residence"

Date: Sun, 28 Jun 1998 10:38:48 -0700

From: Gene Waldo <gwaldo@1stnetusa.com>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Organization: S.A.P.I.E.N.T. (Surly, Agressive, Politically Incorrect Enemies of Non Thinkers)

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Coil Update

Sender: tesl-err@emachine.com

Mike Hammer wrote:

>

> At 08:16 AM 6/28/98 -0700, you wrote:

> >caps are 10 reddog bottles.

>

> Tom,

> One neon can fail just as readily as more.

snips

> Replace those bottle caps with a poly cap. You will be amazed at the

> difference. A quite large percentage of your primary energy is just

> going into heating that glass dielectric. Where you now have

> thin spindly purple streamers a poly cap will give you thicker hotter

> brighter blue streamers.

>

> Keep us posted.

>

> TTYL!

>

> Mike Hammer

> mhammer@misslink.net

Mike,

Would 'peaked' SW capacitance data from a given 'tuned' TC be apt to

transfer readily to the construction of our replacement polys? God, I

hope so!   Or are there other (spindly v hot) 'character' aspects to

consider?

--------------------------------------

Gene Waldo -  gwaldo@1stnetusa.com  - "Igor in Residence"

X-Mailer: IPAD 2.0

Date: Sun, 28 Jun 1998 13:32:59 -0500

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Mike Hammer <mhammer@misslink.net>

Subject: SW caps

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

>

>Back on topic:

>

>Coilers,

>Dan came up with an ideal containment for our 12 40 oz Bud bottles for

>the SW cap.  I'm currently epoxying the lid/electrodes and wiring the 12

>together.  Pics shortly, on Dan's Site.

>

>QUESTION: how 'SATURATED' should the NaCl sol'n be? Pre any actual test

>for conductivity, methinks a point of diminishing return may be reached

>as 'satuation' reaches about 60-80% (or less?)????  Any data?

>

>I want to make the thing a test bed, with a variable depth neg SW bath

>to 'tune-on-the-fly', in lieu of fixed outer foils. This will allow us

>to jump the pri tap in increments and "tweak" the cap 'hot', for max

>spark length.

>

>Dan has some resistance to a thru-hull ftg & valve in the plastic tub; I

>guess I'll have buy the damn thing from him!

>

>My electronics store guy has offered to supply his cap meter to help us

>calibrate the thing. Don't know if he has a Megger, or if that would be

>of any value... should be, I guess? .

>

>------------------------------------------------------

>Gene Waldo -  gwaldo@1stnetusa.com  - "Igor in Residence"

>---------------------------------------------------------------

Gene,

My take on SW caps is that its just a stop gap measure

to get you sparking quick. In that light I think its fruitless

to spend a great deal of effort making a variable SW cap.

First of all the losses in your primary system are going to be

quite high resulting in low Q and quite wide bandwidth. This means

very broad tuning. Given those conditions I dont think your approach

would be worthwhile. Especially since your SW cap is already "variable"

by adding or removing bottles.

The saturation level of the SW solution is not at all critical.

The conductivity of the SW does not vary much with saturation level.

My first SW caps used a saturated solution. On later ones

I just dumped "some" salt in "some" water. Resulting capacitance

levels were identical.

TTYL!

Mike Hammer

mhammer@misslink.net

X-Mailer: IPAD 2.0

Date: Sun, 28 Jun 1998 13:39:25 -0500

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Mike Hammer <mhammer@misslink.net>

Subject: SW vs poly caps

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

At 10:38 AM 6/28/98 -0700, you wrote:

>Mike Hammer wrote:

>>

>> At 08:16 AM 6/28/98 -0700, you wrote:

>> >caps are 10 reddog bottles.

>>

>> Tom,

>> One neon can fail just as readily as more.

>snips

>> Replace those bottle caps with a poly cap. You will be amazed at the

>> difference. A quite large percentage of your primary energy is just

>> going into heating that glass dielectric. Where you now have

>> thin spindly purple streamers a poly cap will give you thicker hotter

>> brighter blue streamers.

>>

>> Keep us posted.

>>

>> TTYL!

>>

>> Mike Hammer

>> mhammer@misslink.net

>

>Mike,

>Would 'peaked' SW capacitance data from a given 'tuned' TC be apt to

>transfer readily to the construction of our replacement polys? God, I

>hope so!   Or are there other (spindly v hot) 'character' aspects to

>consider?

>

>--------------------------------------

>Gene Waldo -  gwaldo@1stnetusa.com  - "Igor in Residence"

Gene,

Yes the level of capacitance you find that works best for your

SW caps will be the same as for a poly cap.

Other than hotter thicker streamers due to much lower losses

one other aspect of poly caps is its positive effect on

primary circuit Q. You will find your system much more sensitive

to proper tune with a very definite peak. With the lower Q

from SW caps the tuning is sometimes so broad that a definite

peak is hard to locate.

TTYL!

Mike Hammer

mhammer@misslink.net

X-Originating-IP: [208.23.202.21]

From: "Bill the arcstarter" <arcstarter@hotmail.com>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: SW caps

Date: Sun, 28 Jun 1998 12:23:18 PDT

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

Mike Hammer wrote:

>My take on SW caps is that its just a stop gap measure

>to get you sparking quick. In that light I think its fruitless

>to spend a great deal of effort making a variable SW cap.

<snip>

I don't usually respond to posts regarding salt water capacitors but I

will to this one.  I agree with Mike's advice.  Try your saltwater caps.

But then (as soon as possible) move to a poly cap.  It will be well

worth your time and money.

>The saturation level of the SW solution is not at all critical.

>The conductivity of the SW does not vary much with saturation level.

Especially if you used tap water which is already chlorinated, ie,

conductive.

>My first SW caps used a saturated solution. On later ones

>I just dumped "some" salt in "some" water. Resulting capacitance

>levels were identical.

This makes sense.  The dielectric is the bottle, not the water!

-Bill the arcstarter

Starting arcs in Cinci, OH

http://www.geocities.com/CapeCanaveral/Hangar/6160

X-Mailer: IPAD 2.0

Date: Sun, 28 Jun 1998 19:24:02 -0100

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Jim Ryan <jryan@exis.net>

Subject: Re: SW caps

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Hi all. Got all my parts built except the poly caps. Tried to roll them

today. I was a pain in the BUTT. I'm the guy who is trying to use .005

brass shim stock. I'm still having problems. I started with 6" x 75"

.005 brass shim stock 5 layers of .006 poly. = brass/poly/brass/poly.

I tried rolling it 4 times today, and kept getting ripples in one or more

of the 5 poly sheets. They tended to make the roll loose. The final diameter

missed 3" PVC by 1/4 "

Will 4 X .006 poly work "OK" with a 30 ma. 12KV transformer and 2 .0033uf

in series for a total cap of .0066uf. 5 layer of poly is kicking my butt, any hints ?

Thanks

Jim Ryan

Date: Sun, 28 Jun 1998 15:43:11 -0500

From: Tom Polk <tpolk@prismnet.com>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: SW caps

Sender: tesl-err@emachine.com

Regarding poly caps, read on if you want a simple, vacuumless method.

My successful flat plate poly cap was made by 29 polyurethane sheets,

16 x 12 x 1/8".

I built a box of 1/4 inch Lexan oversized by only 1/8 inch, so the

sheets fit in with 1/16" to spare. My box was 4" tall. The box was

sealed and glued with acrylic cement. For that to work the matching

surfaces must be flat and smooth. I reinforced some places as needed

with tiny 1/2" wood screws.

The plates were of heavy duty aluminum foil, 10 x 14 inches, with 2"

wide tabs, more like tails. The first (bottom) plate installed had a

long tail, the top plate had a short one, so that the ends matched.

The fun part. You do not have to use a vacuum. You have on hand two

pints of castor oil, which is slightly thicker than mineral oil. Then

grease one side of the first poly sheet with the oil, let the first

plate stick to it.

Pour about 1/4" of oil in the box, taking care not to let it spill on

the rim, which you will have to seal later. Lay the poly sheet in the

oil, foil on bottom side,  pushing it in so the foil stays in place;

work it so the bottom oil flows up, around and through the end gaps and

stars running over the top of the poly.

Then put the next foil sheet on the poly with its terminal opposite the

first. Burnish it against the poly to get all the bubbles out. This

works beautifully and is a satisfying, zen-like experience. The

remaining oil will run over that. Then you can put the next poly sheet

in, push it, work it, etc., alternating the foil tab positions.

Using gravity to fill the gaps requires that you always have enough oil

in the box. And you have to get used to greasy hands. Flat poly is

almost a requirement for this method to work. When you push the stack

down, the oil makes it stick together, but not for real long. Thats why

you have to have enough oil in there so that no air gets sucked in.

With your parts cut, you can build this up in about an hour.

Put one final ungreased poly sheet on top of the stack, lay the tabs

over it. Drill a hole through the Lexan top over each group of tabs.

Bring a screw with a very wide washer from the underside of the tabs so

that it protrudes out the Lexan. Put neoprene washers on the screws at

each lexan surface. Tighen it down and it will seal. Then seal the lexan

top to the box.

You will have a lot of air left. You can drill a tiny hole in the lexan

top and use a syringe to backfill the box, close the hole with a screw.

Date: Sun, 28 Jun 1998 17:38:45 -0500

From: Bert Hickman <bert.hickman@aquila.com>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Organization: Stoneridge Engineering

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Our Coil #2 - and we're off!

Sender: tesl-err@emachine.com

Gene Waldo wrote:

<SNIP>

Re: Quarter shrinkin':

<SNIP

> Commercial possibilities: Try some REAL money: US Ag coin: quarters,

> halves, dollars: shrunken to jewlery: rings, earings, pendants etc....

> Just now have to create a "TESLA BABES" mkt! ~~<[:-)

>

> Don't know legals, but since our gov't. has divested itself from issuing

> actual money*, and now is into script, it may be OK, since Ag coin* must

> by now be either relics and/or qurios?

As far as I can tell, quarter shrinking IS legal as long as you don't

fraudulently use the coins or try to pass them off as legal currency.

The coins are the same mass, density, etc... they're just a "slightly"

different size! :^)

BTW, the older silver coins take a lot less power to shrink, and they

seem to turn out more uniformly. I've only tried a couple of silver

quarters thus far, and haven't gotten the right power level nailed down

as yet. If you use a similar amount of pulse power as for a newer clad

coin, the silver coin partially melts, and the features become sort of

rounded and indistinct. Plus the silver coins become a sort of

tanish-gold color from the heat and oxidation. I need to find a source

for CHEAP, but good looking, silver coins and run some more experiments.

Would like to try a real silver dollar, but I'm too cheap.  :^)

<SNIP>

>

> Back on topic:

>

> Coilers,

> Dan came up with an ideal containment for our 12 40 oz Bud bottles for

> the SW cap.  I'm currently epoxying the lid/electrodes and wiring the 12

> together.  Pics shortly, on Dan's Site.

>

> QUESTION: how 'SATURATED' should the NaCl sol'n be? Pre any actual test

> for conductivity, methinks a point of diminishing return may be reached

> as 'satuation' reaches about 60-80% (or less?)????  Any data?

I'd make it a saturated solution - salt's cheap! Although it won't

change the capacitance at all, it will improve the conductivity of the

solution somewhat, and SW caps need all the help they can get!

<SNIP>

>

> ------------------------------------------------------

> Gene Waldo -  gwaldo@1stnetusa.com  - "Igor in Residence"

X-Mailer: IPAD 2.0

Date: Sun, 28 Jun 1998 18:43:12 -0500

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Mike Hammer <mhammer@misslink.net>

Subject: Re: rolled poly cap (was SW caps)

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

At 07:24 PM 6/28/98 -0100, you wrote:

>Hi all. Got all my parts built except the poly caps. Tried to roll them

>today. I was a pain in the BUTT. I'm the guy who is trying to use .005

>brass shim stock. I'm still having problems. I started with 6" x 75"

>.005 brass shim stock 5 layers of .006 poly. = brass/poly/brass/poly.

>

>

>I tried rolling it 4 times today, and kept getting ripples in one or

>more of the 5 poly sheets. They tended to make the roll loose. The final >diameter missed 3" PVC by 1/4 "

>

>Will 4 X .006 poly work "OK" with a 30 ma. 12KV transformer and 2 .0033uf

>in series for a total cap of .0066uf. 5 layer of poly is kicking my butt,

>any hints ?

>

>Thanks

>

>Jim Ryan

Jim,

Rolling poly caps with stacks of thin poly is sometimes quite frustrating.

Its difficult to keep all the layers lined up and wrinkle free.

One thing I did was to use paper clips along the edges of the stack

to keep the strips in line. Remove the clips as you roll the cap up.

Gary Weaver once suggested to me to attach a couple rubber bands to

the end of the roll with duct tape and to attach the rubber bands to

some immovable object. This will keep some tension on the layers as

you roll it up.

Sometimes all you need is a helper to keep the strips in alignment.

Two caps in series with four 6 mil sheets for the dielectric in each cap

will stand up to 12kv. But this is the lower limit and doesnt allow

much margin or room for error. My present set of rolled caps are .03 mfd

and 24 mils each. In series this is .015 mfd and 48 mils. This has

worked well for me using 12720 volts from my rebuilt neon. My main

gap is rather conservative at .225 inch total.

If you decide to build 24 mil caps then be careful with the main gap.

This will not be a bullet proof cap but should work well with a little care.

TTYL!

Mike Hammer

mhammer@misslink.net

Date: Sun, 28 Jun 1998 19:00:27 -0500

From: Tom Polk <tpolk@prismnet.com>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: SW caps

Sender: tesl-err@emachine.com

Jim Ryan wrote:

>

> Hi all. Got all my parts built except the poly caps. Tried to roll them

> today. I was a pain in the BUTT. I'm the guy who is trying to use .005

> brass shim stock. I'm still having problems. I started with 6" x 75"

> .005 brass shim stock 5 layers of .006 poly. = brass/poly/brass/poly.

>

> I tried rolling it 4 times today, and kept getting ripples in one or more

> of the

> 5 poly sheets. They tended to make the roll loose. The final diameter missed

> 3" PVC by 1/4 "

>

> Will 4 X .006 poly work "OK" with a 30 ma. 12KV transformer and 2 .0033uf

> in series for

> a total cap of .0066uf. 5 layer of poly is kicking my butt, any hints ?

>

> Thanks

>

> Jim Ryan

Can you adapt it to flat plate?

Date: Sun, 28 Jun 1998 19:48:20 -0500

From: Tedd Payne <tpayne@netnitco.net>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: For Sale:  Variacs & Neon Transformers

Sender: tesl-err@emachine.com

If interested, please contact me DIRECTLY at tpayne@netnitco.net

Please tell me which one(s) you're interested in, and your ZIP CODE to

determine shipping cost.

Thanks,

Tedd

General Radio Variac type 100-Q

input 115VAC, output 0-135VAC 18 amps

with knob and dial plate calibrated 0-135

hexagonal screen enclosure

well labeled connections

I tested it, it works, seems good condition

$90

General Radio Variac type 200-CU

input 115VAC, output 0-135VAC

I'd estimate around 7 amps, but not marked

with knob and dial plate calibrated 0-100 (100%, not VAC)

round can-like enclosure

well labeled connections

I tested it, it works, seems fair-good condition

$25

Neon Transformers:

$18 used  15kv30ma  in=3 wires   out=2 term studs

$17 used  15kv30ma  in=3 wires   out=2 term studs  case sl. bent 1

corner

$16 used  15kv30ma  in=porc ins  out=porc insul (1 loose HV insul)

$14 used  12kv30ma  in=porc insul

output was porc insul, but both missing,

now is 1-inch HV wire leads

$12 used  12kv30ma  in=porc ins  out=porc ins

(both HV insul loose, one is chipped)

$14 used  12kv30ma  in=3-inch wires  out=8-inch HV wires

has pull-chain type switch

NEW Neon transformer HV wire:

Flexible gray, 14 gauge, multistrand copper, rated 15KV, $1.00 per foot.

Standard black, 16 gauge, multistrand copper, rated 15KV, $.50 per foot.

AC power cord for transformer, 3-wire, grounded plug, used, $.50

MODEL T SPARK COIL  $18

- original, not reproduction

- spark a little over 1/4", maybe more with new interrupter set.

- good visual & operating cond.

- operates on 6VDC input

Date: Sun, 28 Jun 1998 20:17:36 -0500

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Kerry Ludwig <kilroy@nex.net>

Subject: Quarter shrinkin'

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

>Re: Quarter shrinkin':

>As far as I can tell, quarter shrinking IS legal as long as you don't

>fraudulently use the coins or try to pass them off as legal currency.

>The coins are the same mass, density, etc... they're just a "slightly"

>different size! :^)

Tho' this is getting a bit off topic for the list... I thought I would chime

in with my $0.02 ...

I believe that there is a federal law against defacing or destroying

currency, but only paper currency.  Coins never were covered by this law.  I

assuming this stems from the coins originally having there value due to the

metal content instead of an abstract value assigned to them.

You ever seen the machines that you place a penny in, it flattens it and

imprints a logo or design on it?  (Usually found in tourist spots as

keepsakes and such.)  I think this is the same reason that these machines

are legal.

I have noticed several postings about shrinkin' coins in the past, but since

I only read about 10% of my e-mail... I'm sure that I have missed much.

Where can I find more information on the technique used to do this?  Any one

got any pics of a before and after?

Kerry "Kilroy" Ludwig

kilroy@nex.net

X-Originating-IP: [208.16.150.31]

From: "Bill the arcstarter" <arcstarter@hotmail.com>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Quarter shrinkin'

Date: Sun, 28 Jun 1998 18:37:26 PDT

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

It was written:

>I have noticed several postings about shrinkin' coins in the past, but since

>I only read about 10% of my e-mail... I'm sure that I have missed much.

>Where can I find more information on the technique used to do this?

>Any one got any pics of a before and after?

As far as I know - at the moment, Bert Hickman, Paul (fleischwulf) and

I are the only folk on the planet actively shrinking quarters!

On top of that - I think that the arcstarter and fleischwulf sites are

the only URLs with useable info/pics on the topic.  The sites are:

http://www.geocities.com/CapeCanaveral/Hangar/6160/

http://www.geocities.com/CapeCanaveral/Hangar/9561/

Check it out and ask questions if you like -but probably off list since

this list is for tesla use only!

If anyone else has any URLs of similar sites I'd love to get them

linked up.  I've looked for such URLs and not found them!

BTW - Quarter shrinking (and pulse caps in general) is not for

amateurs.  There is a very real danger of a serious accident most likely

involving the removal of hands/arms/legs etc.  Do NOT doubt these

capacitors are extremely (and very literally) lethal.  Frankly - doing

this scares the hell out of us (or at least ME) but we do it anyway....

:)  If I had any kids there's NO WAY I'd let any of them fool with this stuff...

I don't say that to scare or impress anyone - I just want you to be

aware of how things are.  Please - Go ahead - live vicariously through

our sites!  But don't kill yourselves just to impress your friends.

-Bill the arcstarter

Starting arcs in Cinci, OH

http://www.geocities.com/CapeCanaveral/Hangar/6160

Date: Sun, 28 Jun 1998 20:54:56 -0500

From: Bert Hickman <bert.hickman@aquila.com>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Organization: Stoneridge Engineering

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Quarter shrinkin'

Sender: tesl-err@emachine.com

Kerry Ludwig wrote:

>

> >Re: Quarter shrinkin':

>

> >As far as I can tell, quarter shrinking IS legal as long as you don't

> >fraudulently use the coins or try to pass them off as legal currency.

> >The coins are the same mass, density, etc... they're just a "slightly"

> >different size! :^)

>

> Tho' this is getting a bit off topic for the list... I thought I would chime

> in with my $0.02 ...

>

> I believe that there is a federal law against defacing or destroying

> currency, but only paper currency.  Coins never were covered by this law.  I

> assuming this stems from the coins originally having there value due to the

> metal content instead of an abstract value assigned to them.

>

> You ever seen the machines that you place a penny in, it flattens it and

> imprints a logo or design on it?  (Usually found in tourist spots as

> keepsakes and such.)  I think this is the same reason that these machines

> are legal.

>

> I have noticed several postings about shrinkin' coins in the past, but since

> I only read about 10% of my e-mail... I'm sure that I have missed much.

> Where can I find more information on the technique used to do this?  Any one

> got any pics of a before and after?

>

> Kerry "Kilroy" Ludwig

> kilroy@nex.net

<SNIP>

Kerry:

For some nice pictures, check out Fleischwulf's site:

http://www.geocities.com/CapeCanaveral/Hangar/9561/

The frontpage picture shows a number of quarters shrunk at successively

higher power levels. The coin shrinks in about 10-20 microseconds, while

the surrounding magnet-wire coil literally explodes. 6000-7000 Joules

will shrink a quarter to a diameter less than that of a dime! The

explosive noise is incredible, and the flying fragments of the coil

create bullet-velocity shrapnel.

The pulsed metalforming process works with virtually all coins except

for 1982+ pennies, since the these are copper-plated zinc. Trying to

shrink these blows the surface copper off, then melts the zinc coin

underneath into a glob of metal). Nickels don't work as well because of

their poor electrical conductivity.

As far as I know Fleishwulf's, mine, and one site in Canada (fired only

once) are the only current "shrinkin'" facilities in the world. Don't

try this at home, folks... :^)

-- Bert --

From: ralph.hartwell@emachine.com (Ralph Hartwell)

Subject:  Quarter shrinkin'

Date: Mon, 29 Jun 1998 03:12:00 GMT

Organization: The Energy Machine Information System 504-733-3462

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

KL>>The coins are the same mass, density, etc... they're just a "slightly"

KL>>different size! :^)

KL>Tho' this is getting a bit off topic for the list... I thought I would chime

KL>in with my $0.02 ...

...or $0.002, after shrinking it!

Ralph

---

From: "Choad" <choad69@geocities.com>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Variac Knobs

Date: Sun, 28 Jun 1998 23:42:52 -0500

X-Mailer: IPAD 2.0

X-MimeOLE: Produced By Microsoft MimeOLE V4.72.2106.4

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Anyone know where to get variac knobs at? I'm looking for one for

a 3/8" shaft.

Date: Sun, 28 Jun 1998 21:58:45 -0700

From: Michael Loftus <loftusm@ix.netcom.com>

Organization: Me

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: SW caps

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Jim Ryan wrote:

>

> Hi all. Got all my parts built except the poly caps. Tried to roll them

> today. I was a pain in the BUTT. I'm the guy who is trying to use .005

> brass shim stock. I'm still having problems. I started with 6" x 75"

> .005 brass shim stock 5 layers of .006 poly. = brass/poly/brass/poly.

>

> I tried rolling it 4 times today, and kept getting ripples in one or more

> of the

> 5 poly sheets. They tended to make the roll loose. The final diameter missed

> 3" PVC by 1/4 "

>

> Will 4 X .006 poly work "OK" with a 30 ma. 12KV transformer and 2 .0033uf

> in series for

> a total cap of .0066uf. 5 layer of poly is kicking my butt, any hints ?

>

> Thanks

>

> Jim Ryan

Jim,

I know how you feel.  I wrapped 5 layers of .006 poly 14" wide and 90"

long and even with my wife helping it was a "bad word"!  The aluminum

plates were  10" x 84".

I built a winder based on a 1/4" threaded shaft over which I slide 1/4"

plastic tubing and wooden disks 14" apart and held in place by hose

clamps which prevent the disks from moving apart and prevent the plastic

tube from turning free of the threaded shaft. I taped the two sets of

poly onto the winder's shaft and taped the 2 sheets of aluminum onto

their respective poly (winder end only) and slowly wound it all onto the

plastic-covered shaft.  The hose clamps are on the ends of the tubing and

clamped very tightly.  With care, it works fairly well, with a very small

center space and a tight 'roll.  I still could not do it by myself.

I have two disappointments: first, the diameter of the roll is 3.25",

requiring me to wrap additional poly until it fit shugly into 4" abs, and

4" abs is large, bulky, and the end-caps are expensive.  Secondly, the

value is only .0182 (dry) which is way below the .0200 I was aiming for.

Clearly I have made an error somewhere.  I have no idea where.

My suggestion is that you consider your first cap attempt as a "research

and development" project. Once you seal-up the cap, I kind of think there

is no going back short of building another more!  Good luck.

Mike

P.S.  A good friend loaned me a very nice vacuum pump (29hg) to pump-down

the oil and the caps when I finally get three that have the target value.

If people share equipment in their areas, then perhaps the vacuum issue

is nicely overcome!

Full-Name: Bert Wikkerink

Date: Mon, 29 Jun 1998 08:14:54 -0400

From: Bert Wikkerink <bert@mitra.com>

Organization: Mitra Imaging Inc.

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Quarter shrinkin'

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Over here in Canada, we have a $2.00 coin which is composed of two alloys. I'm

not sure what they are, but it should make for some interesting effects when we

try to shrink those. Don't want to do too many though.....it would get kind of

expensive.

Bert

X-Originating-IP: [199.217.140.22]

From: "Bill the arcstarter" <arcstarter@hotmail.com>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: RE: Coil Update

Date: Mon, 29 Jun 1998 07:58:18 PDT

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

Tom wrote:

>Ok, so then I'm ready to build a poly cap. I'd like to build a rolled

>cap (size is an issue with flat plate caps for me). Cheap would be good

>too. I have a couple of gallons of transformer oil on the way. What can

>I use for the cap? Hardware store poly and aluminum foil? Is aluminum

>sheet (flashing) much better?

I've never built a rolled cap so I can't comment on that.

But our flat plate poly caps have worked well for us!  They also *seem*

much easier to build than a roll cap.  Ours have never even started to

heat up.

We used heavy "oven-grade" aluminum foil from the hardware store. (1.5

thous thick)

Let me know if you would like raw poly - as I can provide 48 x 8x12 inch

sheets of 0.060 polyethylene for $25 +s/h.

-Bill

From: Tom Stock <tomstock@geocities.com>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: RE: Coil Update

Date: Mon, 29 Jun 1998 10:30:03 -0700

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

>Tom,

>One neon can fail just as readily as more. Get some protection

>on there ASAP. If you cant find any toroids then some air core

>chokes on PVC pipe will do if backed up with some series resistance.\

I pulled some nice ferrite (i hope they aren't powdered iron?) toroids about 2.5" in diameter out of a color TV. They look like they will work well. Any ideas on this? I haven't wound the wire around them yet. What size and how much?

>Try adjusting your coupling next. Bring the secondary down closer

>to the primary to increase coupling and therefore transfer more energy

>to the secondary. If you start getting pri/sec arcing or heavy corona

>then back off a little. Increasing the coupling will put additional

>strain on your spark gap. Thats why a good gap is important. A good

>gap allows you to run tighter coupling without losing efficiency.

Right now my bottom winding on the secondary is level with the primary.

I'm not getting any arcing.

>Replace those bottle caps with a poly cap. You will be amazed at the

>difference. A quite large percentage of your primary energy is just

>going into heating that glass dielectric. Where you now have

>thin spindly purple streamers a poly cap will give you thicker hotter

>brighter blue streamers.

Ok, so then I'm ready to build a poly cap. I'd like to build a rolled cap

(size is an issue with flat plate caps for me). Cheap would be good too.

I have a couple of gallons of transformer oil on the way. What can I use for the cap? Hardware store poly and aluminum foil? Is aluminum sheet (flashing) much better?

I've read all of the cap files, but none mention aluminum foil for rolled caps.

Thanks much,

-tom

X-Mailer: IPAD 2.0

Date: Mon, 29 Jun 1998 12:16:15 -0500

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Mike Hammer <mhammer@misslink.net>

Subject: RE: Coil Update

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

At 10:30 AM 6/29/98 -0700, you wrote:

>>Tom,

>>One neon can fail just as readily as more. Get some protection

>>on there ASAP. If you cant find any toroids then some air core

>>chokes on PVC pipe will do if backed up with some series resistance.\

>

>I pulled some nice ferrite (i hope they aren't powdered iron?) toroids

>about 2.5" in diameter out of a color TV. They look like they will work

>well. Any ideas on this? I haven't wound the wire around them yet.

>What size and how much?

Yes that is a ferrite material and it should work just fine. What value of

inductance you end up with will be hard to predict without knowing the

characteristics of that particular ferrite material.

Since the inductance is not critical wind as many turns of AWG 24-28 as you

can comfortably fit on the cores. Since ferrite is somewhat conductive you

will need to insulate the cores with at least 3 layers of good vinyl tape

before adding the winding. If you can fit 100 + turns on those cores they

should work well.

One way to tell if your chokes are helping any is to run your coil

with your safety gap set narrow enough so it fires quite often.

Then add the chokes to the circuit and fire it up again. If the chokes

are doing there job the safety gap firings will be less frequent and

less violent.

>

>>Try adjusting your coupling next. Bring the secondary down closer

>>to the primary to increase coupling and therefore transfer more energy

>>to the secondary. If you start getting pri/sec arcing or heavy corona

>>then back off a little. Increasing the coupling will put additional

>>strain on your spark gap. Thats why a good gap is important. A good

>>gap allows you to run tighter coupling without losing efficiency.

>

>Right now my bottom winding on the secondary is level with the primary.

>I'm not getting any arcing.

I was afraid of that. You are at maximum coupling right now and are

probably still somewhat undercoupled. Thats why I normally dont recommend

flat spirals for small coils. The only way for you to increase coupling is

to rewind the primary with a smaller diameter inner turn and tighter turn

to turn spacing.

However if you plan on pushing this coil to higher power levels then I

would leave it as is. You will appreciate the greater toroid/primary

distance as power level goes up.

>

>>Replace those bottle caps with a poly cap. You will be amazed at the

>>difference. A quite large percentage of your primary energy is just

>>going into heating that glass dielectric. Where you now have

>>thin spindly purple streamers a poly cap will give you thicker hotter

>>brighter blue streamers.

>

>Ok, so then I'm ready to build a poly cap. I'd like to build a rolled cap

>(size is an issue with flat plate caps for me). Cheap would be good too.

>I have a couple of gallons of transformer oil on the way. What can I use for the cap? >Hardware store poly and aluminum foil? Is aluminum sheet (flashing) much better?

I've built both flat plate caps and rolled caps and the flat plates offer

the best performance with the rolled types not far behind. The rolled types

are quicker and easier to build and are usually more compact. You can also take

advantage of standard PVC pipe to make an excellent housing.

I have also used flashing as well as aluminum foil. Flashing is much more

expensive but easier to deal with. Foil makes an excellent cap but must be

handled carefully to prevent wrinkling.

Hardware store 6 mil poly will work just fine. You stack up enough layers

for the voltage rating you need. You need a MINIMUM of 48 mils for 12kv

and 60 mils for 15kv. This is really a minimum and is based on using

multiple layers of thinner material. It does not allow much room for error.

The best method of getting reliable caps is to run them in series. Run at least

two caps in series. If you are running a single 15kv/30 then you need about .0053 mfd.

To get that value in a series string of two caps each cap must be .0106 mfd.

However I believe you said you had several 15kv/30 ma neons so I would

build your poly cap with the plan of using two of those neons in parallel.

Build two .02 mfd caps with at LEAST 30 mils of poly and run them in series

to get .01 mfd. If you want a little extra safety margin make each cap with 36 mils

of poly.

>

>I've read all of the cap files, but none mention aluminum foil for rolled caps.

It can be done. What I would do is build a couple practice caps.

You are sure to mess up the first few. Use the Reynolds heavy duty foil.

Its heavier so has less tendency to wrinkle on you. I use Reynolds rather

than the cheaper store brands as its more likely to come off the roll smooth

and wrinkle free.

Dont discount building a flat plate cap. There are a little more work and will require

more oil than a rolled type but deliver the best performance. I custom build my cap

tanks just the right size from plywood and leakproof them by painting the interior

with two coats of polyester resin. I made this type of tank for one of my plate

caps and my large rebuilt neon. Its worked out very well with no leaks.

TTYL!

Mike Hammer

mhammer@misslink.net

X-Originating-IP: [199.217.140.22]

From: "Bill the arcstarter" <arcstarter@hotmail.com>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: RE: Coil Update

Date: Mon, 29 Jun 1998 11:14:31 PDT

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

It was written:

>>Right now my bottom winding on the secondary is level with the primary.

>>I'm not getting any arcing.

>

>I was afraid of that. You are at maximum coupling right now and are

>probably still somewhat undercoupled. Thats why I normally dont recommend

>flat spirals for small coils. The only way for you to increase coupling is

>to rewind the primary with a smaller diameter inner turn and tighter turn

>to turn spacing.

Can't he just raise his primary up off the base a bit to increase the

coupling? (asuming there's a hole for the secondary to slide down

through)

>I've built both flat plate caps and rolled caps and the flat plates offer

>the best performance with the rolled types not far behind.

Mike - sounds like you are saying that building a roll cap is easier

than a flat place cap.  Is that correct?  I've not built a roll cap but

it seems like a real pain!

Our plate caps were really easy to make - but then again we DID NOT use

multiple layers of poly - just single 0.060 sheets (which is contrary to

most list wisdom but we seem to have gotten away with it).

>The best method of getting reliable caps is to run them in series. Run at least

>two caps in series. If you are running a single 15kv/30 then you need about .0053 mfd.

>To get that value in a series string of two caps each cap must be .0106 mfd.

We are doing this.  Four caps total, each is 0.026 uf, in

series-parallel for a net capacitance of 0.026 uf.  (in case anyone is

curious)

More later!

-Bill the arcstarter

Starting arcs in Cinci, OH

http://www.geocities.com/CapeCanaveral/Hangar/6160

X-Mailer: IPAD 2.0

Date: Mon, 29 Jun 1998 14:24:54 -0500

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Mike Hammer <mhammer@misslink.net>

Subject: RE: Coil Update

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

At 11:14 AM 6/29/98 PDT, you wrote:

>It was written:

>

>>>Right now my bottom winding on the secondary is level with the

>primary.

>>>I'm not getting any arcing.

>>

>>I was afraid of that. You are at maximum coupling right now and are

>>probably still somewhat undercoupled. Thats why I normally dont recommend

>>flat spirals for small coils. The only way for you to increase coupling is

>>to rewind the primary with a smaller diameter inner turn and tighter turn

>>to turn spacing.

>

>Can't he just raise his primary up off the base a bit to increase the

>coupling? (asuming there's a hole for the secondary to slide down

>through)

No. That wont help. With a flat spiral the point of maximum

coupling occurs when the bottom turn of the secondary is even

with the plane of the primary turns.

>

>>I've built both flat plate caps and rolled caps and the flat plates offer

>>the best performance with the rolled types not far behind.

>

>Mike - sounds like you are saying that building a roll cap is easier

>than a flat place cap.  Is that correct?  I've not built a roll cap but

>it seems like a real pain!

Well the rolled caps I built sure were faster than the plate caps

I made. The plate caps required cutting lots of poly squares and

foil plates and stacking them up. Then cutting and fitting plexiglass

squares to sandwich the caps. Then building and leakproofing the tanks.

The rolled caps required only 4 long strips of poly folded in half

and 2 strips of foil. Wind them up on a copper tubing center spindle

and stuff in a piece of PVC pipe then RTV on a couple end plates

of plexiglass. The last one of these I built in less than 2 hours.

The plate caps took days to build.

>

>Our plate caps were really easy to make - but then again we DID NOT use

>multiple layers of poly - just single 0.060 sheets (which is contrary to

>most list wisdom but we seem to have gotten away with it).

A single thick layer of poly is OK as long as its pure virgin

material. The poly I used is the lowest grade construction material.

A huge cost savings. I bought a 2500 sq foot roll of 6 mil poly

for around $25.

Your 60 mil thick poly has a much lower voltage rating than the

same thickness made up from ten 6 mil sheets stacked up. The down

side of the multiple stacks is that there is going to be more

trapped air. This requires vacuum pumping or extensive break in time.

TTYL!

Mike Hammer

mhammer@misslink.net

Date: Mon, 29 Jun 1998 13:33:50 -0700

From: Gene Waldo <gwaldo@1stnetusa.com>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Organization: S.A.P.I.E.N.T. (Surly, Agressive, Politically Incorrect Enemies of Non Thinkers)

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Coil Update

Sender: tesl-err@emachine.com

Mike Hammer wrote:

>

> At 11:14 AM 6/29/98 PDT, you wrote:

> >It was written:

> >

> >>>Right now my bottom winding on the secondary is level with the primary.

> With a flat spiral the point of maximum coupling occurs when the bottom turn

> of the secondary is even with the plane of the primary turns.

>

> TTYL!

>

> Mike Hammer

> mhammer@misslink.net

Mike,

Uninformed (Dumb) question:

|

|

a        //////////////////////////

|

|

|

|

b      /////////////-////////////

1        |     2

|

Is there any thing worthy of comment?

In b) would 1&2 maybe need counter winding (my guess is not)? Loose

2ndry RFGnd? Pri better as sol?

---------------------------

Gene Waldo -  gwaldo@1stnetusa.com  - "Igor in Residence"

X-Mailer: IPAD 2.0

Date: Mon, 29 Jun 1998 21:10:59 -0100

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Jim Ryan <jryan@exis.net>

Subject: Re: SW caps

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

>I know how you feel.  I wrapped 5 layers of .006 poly 14" wide and 90"

>long and even with my wife helping it was a "bad word"!  The aluminum

>plates were  10" x 84".

Wow, you guys really get your wives to help you. That's my problem ! Mine only

complains about the money I spend on the coil and the mess I make.

It's to late to find another wife after 34 years.  Oh well, she never helped

me making fireworks or model boats and planes either.

Thanks for the help.

You gave me some great suggestions.

jim Ryan

From: <Dsurfr@aol.com>

Date: Mon, 29 Jun 1998 19:04:59 EDT

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Gap Quenching

X-Mailer: IPAD 2.0

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

My coil is finally up & running but I've cut a lot of corners to spare expense

of money I didn't have. Now I'm going back & replacing components one by one.

A poly cap to replace the SW's & a better toroid are high on the list but the

spark gap interests me as well. I'm currently using a static gap of 8 brass

bolts (4 gaps) w/ a muffin fan for cooling. All the postings seem to indicate

that I should go for the RQ style gap. I recently tried putting a hair

dryer/blower (despite my wife's complaints) in front of the box my gap is

housed in. I got about 2 more inches of spark & it was much whiter & MUCH more

loud. Richard Quick also has posted plans for an air blast gap that he says

that he says (quote)"the performance of this type gap on coils powered by neon

sign transformers is second to none. If you run neons & want the longest spark

at any cost, then this is the gap for you." Does this mean this type of gap is

harder on neons? Why would his other style gap be so preferred if he himself

rates this design so high?

Thanks, as usual, Jim

X-Originating-IP: [208.23.200.107]

From: "Bill the arcstarter" <arcstarter@hotmail.com>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Quarter shrinkin'

Date: Mon, 29 Jun 1998 18:12:07 PDT

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

Bert wrote:

>the surrounding magnet-wire coil literally explodes. 6000-7000 Joules

>will shrink a quarter to a diameter less than that of a dime! The

>explosive noise is incredible, and the flying fragments of the coil

>create bullet-velocity shrapnel.

...and if those fragments reach your body - you will bleed, suffer,  and

possibly die.  Bert, Paul, Tristan, everyone - Please be exceedingly

careful!!!!

We use >4 layers of plywood, lexan and carpet to contain the mess.

>The pulsed metalforming process works with virtually all coins except

>for 1982+ pennies, since the these are copper-plated zinc. Trying to

>shrink these blows the surface copper off, then melts the zinc coin

>underneath into a glob of metal). Nickels don't work as well because of

>their poor electrical conductivity.

We have only shrunk quarters.  Give us some time! :)

So far we have not been able to compress the lead atoms into gold but

we're working on the problem...

We did pinch a copper pipe down quite nicely!  Sort of looked

wasp-waisted after all was said and done!

>As far as I know Fleishwulf's, mine, and one site in Canada (fired only

>once) are the only current "shrinkin'" facilities in the world.

...and the Canadian fellow almost paid dearly for the experience.

That's why he only did it once.  I'm told it blew out a window and

possibly some equipment on the other side of the room.

We need one of those "warning" sirens from a nuclear plant:

RRR!  RRR!  RRR!  RRR!  The device is now armed!  RRR!  RRR!  RRR!

I'm done preaching now.  Sorry for the non-tesla distractions.

By the way - I just put the last coat on my new secondary.  Looks nice!

How conductive are mosquitoes?  One got embedded in the varnish.  I

suppose it wouldn't have been so bad if it'd been a LIGHTNING bug

instead, eh? :)

-Bill "Ten-fingers" Arcstarter

(so far so good - and I'm not pressing my luck)

X-Originating-IP: [208.23.200.107]

From: "Bill the arcstarter" <arcstarter@hotmail.com>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Gap Quenching

Date: Mon, 29 Jun 1998 18:22:21 PDT

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

Jim wrote:

>spark gap interests me as well. I'm currently using a static gap of 8 brass

>bolts (4 gaps) w/ a muffin fan for cooling. All the postings seem to indicate

That's going to be much better than just using ONE gap - which is what

some folk try!  Are you using stove-type bolts with the domed heads?

What spacing are you using?

>that I should go for the RQ style gap. I recently tried putting a hair

>dryer/blower (despite my wife's complaints) in front of the box my gap is

>housed in. I got about 2 more inches of spark & it was much whiter & MUCH more

>loud. Richard Quick also has posted plans for an air blast gap that he says

>that he says (quote)"the performance of this type gap on coils powered by neon

>sign transformers is second to none. If you run neons & want the longest spark

>at any cost, then this is the gap for you." Does this mean this type of gap is

>harder on neons? Why would his other style gap be so preferred if he himself

>rates this design so high?

Where did you read this (air-blast gap)?  I want to read it too!

-Bill the arcstarter

Starting arcs in Cinci, OH

http://www.geocities.com/CapeCanaveral/Hangar/6160

Date: Mon, 29 Jun 1998 22:33:57 -0500

From: Bert Hickman <bert.hickman@aquila.com>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Organization: Stoneridge Engineering

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Gap Quenching

Sender: tesl-err@emachine.com

Jim and all,

Info on both airblast and cylinder gaps can be obtained on the funet

site at:

ftp://ftp.funet.fi/pub/sci/electrical/tesla/misc/

Download and unZip the file "sparkgap.zip" for further information.

Actually, subsequent testing by some members on the Tesla Coil Builders

List seems to show that a so-called vacuum gap (a series of parallel

copper pipes through which high velocity air is drawn) gives best

overall performance with a neon transformer power source. A vacuum

cleaner motor is often used to draw air throught the gaps - hence the

name "vacuum gap". A vacuum gap can handle higher power than the

cylinder-style gap while also providing superior quenching and lower

sparkgap series-resistance (thus less losses) than a single high

velocity airblast gap.  Much of the original development of these gaps

was actually performed by Richard Hull and the crew at the Tesla Coil

Builders of Richmond (TCBOR).

A simple cylinder gap works great until you reach about 1- 1.5 KVA (most

smaller coils), and is relatively easy to build. A vacuum gap will

easily handle power levels of over 4 KVA. Both are much quieter than an

air blast gap.  :^)

-- Bert --

Dsurfr@aol.com wrote:

>

> My coil is finally up & running but I've cut a lot of corners to spare expense

> of money I didn't have. Now I'm going back & replacing components one by one.

> A poly cap to replace the SW's & a better toroid are high on the list but the

> spark gap interests me as well. I'm currently using a static gap of 8 brass

> bolts (4 gaps) w/ a muffin fan for cooling. All the postings seem to indicate

> that I should go for the RQ style gap. I recently tried putting a hair

> dryer/blower (despite my wife's complaints) in front of the box my gap is

> housed in. I got about 2 more inches of spark & it was much whiter & MUCH more

> loud. Richard Quick also has posted plans for an air blast gap that he says

> that he says (quote)"the performance of this type gap on coils powered by neon

> sign transformers is second to none. If you run neons & want the longest spark

> at any cost, then this is the gap for you." Does this mean this type of gap is

> harder on neons? Why would his other style gap be so preferred if he himself

> rates this design so high?   Thanks, as usual, Jim

From: "Choad" <choad69@geocities.com>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Everyone looking for variac knows or dial plates,

Date: Tue, 30 Jun 1998 00:41:34 -0500

X-Mailer: IPAD 2.0

X-MimeOLE: Produced By Microsoft MimeOLE V4.72.2106.4

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Everyone looking for variac knows or dial plates,

I found that Ohmite has dial plates and knobs for

there highpowered pot's. Their knobs fit 1/8",1/4", & 3/8"

shafts. Their dial plates range from 1 1/4" to 5 1/2".

Go to www.ohmite.com and send for their free catalogs

to find out more.

From: "Choad" <choad69@geocities.com>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Rolled Capacitor Specs?

Date: Tue, 30 Jun 1998 11:08:33 -0500

X-Mailer: IPAD 2.0

X-MimeOLE: Produced By Microsoft MimeOLE V4.72.2106.4

X-MIME-Autoconverted: from 8bit to quoted-printable by geocities.com id JAA06218

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Tom,

I used a K of 2.2 to calc my rolled caps and they came out .015�F.

I guess I didn't get them rolled really tight. Oh well. I would suggest

at least 30 mils of poly. I usually use a K of 2.0 when doing cals for

my new caps so that I don't come pu short. I figure that you should

make the plate 75" long by using 30mils instead of the 24mils.

That's my 2 pasos on that.

>-----Original Message-----

>From: Tom Stock <tomstock@geocities.com>

>To: Tesla-2 Mailing List <tesla-2@emachine.com>

>Date: Tuesday, June 30, 1998 10:02 AM

>Subject: Rolled Capacitor Specs?

>

>Ok, I'm new to poly cap design, so please tell me if I'm anywhere close to

>correct on this:

>

>I need a .01 mfd cap for my next coil upgrade. I'll be using two 15kv/30ma

>neons in parallel and came up with the following specs for my coil:

>

>3 rolled caps in series, each with the following specs:

>

>24 mils of poly (4 layers of hardware store 6 mil)

>Plate length and width is 60" x 12"

>

>Using 300V for the puncture rating (per mil) and 2.3 for the 'K' I come up with:

>

>.03 uF per cap, max 7200V

>

>All three in series gives:

>

>.01 uF, 21,600V

>

>Am I ok so far?  Did I calc this right?

>

>Thanks

>-tom

From: Tom Stock <tomstock@geocities.com>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: RE: Coil Update

Date: Tue, 30 Jun 1998 10:56:14 -0700

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

>>Right now my bottom winding on the secondary is level with the primary.

>>I'm not getting any arcing.

>

>I was afraid of that. You are at maximum coupling right now and are

>probably still somewhat undercoupled. Thats why I normally dont recommend

>flat spirals for small coils. The only way for you to increase coupling is

>to rewind the primary with a smaller diameter inner turn and tighter turn

>to turn spacing.

I'm not sure I understand why this is a problem? My coil works fine and I'm getting

nice 12" sparks from a small toroid discharge terminal. I'm not sure why you are saying

I'm undercoupled?

My primary has an inside diameter of about 7.5" which gives me about 1.5" of clearance

from the bottom of the 4.5" secondary.  Is 1.5" too much?  My secondary is 21.5" of #24 wire. My primary is 13 turns of 1/4" tubing, spaced at .25".

Anyway, basically I just wanted to know what to do next as far as upgrades go. I think

my plan is to make three .03 uF caps, and run them in series. I'll be using two 15kv/30ma neons in parallel along with some nice 3" ferrite toroids I pulled out of some TV's and wrapped with about 150 turns of #24.

sound ok?

-tom

From: Tom Stock <tomstock@geocities.com>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Rolled Capacitor Specs?

Date: Tue, 30 Jun 1998 11:10:31 -0700

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

Ok, I'm new to poly cap design, so please tell me if I'm anywhere close to correct

on this:

I need a .01 mfd cap for my next coil upgrade. I'll be using two 15kv/30ma neons in parallel and came up with the following specs for my coil:

3 rolled caps in series, each with the following specs:

24 mils of poly (4 layers of hardware store 6 mil)

Plate length and width is 60" x 12"

Using 300V for the puncture rating (per mil) and 2.3 for the 'K' I come up with:

.03 uF per cap, max 7200V

All three in series gives:

.01 uF, 21,600V

Am I ok so far?  Did I calc this right?

Thanks

-tom

X-Mailer: IPAD 2.0

Date: Tue, 30 Jun 1998 18:41:30 -0100

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

From: Jim Ryan <jryan@exis.net>

Subject: RE: Coil Update

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

>

>But our flat plate poly caps have worked well for us!  They also *seem*

>much easier to build than a roll cap.  Ours have never even started to

>heat up.

>

>We used heavy "oven-grade" aluminum foil from the hardware store. (1.5

>thous thick)

>

>Let me know if you would like raw poly - as I can provide 48 x 8x12 inch

>sheets of 0.060 polyethylene for $25 +s/h.

>

>-Bill

>

Bill, how do you connect the plates of the foil together ? Every other

one on left and right side ? How do you prepare the foil for connections

to brass screws ? Large washers ?

Thanks

Jim

Date: Tue, 30 Jun 1998 14:30:00 -0700 (PDT)

From: John <froboy83@yahoo.com>

Subject: RF Choke

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

What is an RF Choke, and what does it do?

Thanks

John

X-Originating-IP: [208.16.145.136]

From: "Bill the arcstarter" <arcstarter@hotmail.com>

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: RE: Coil Update

Date: Tue, 30 Jun 1998 18:27:01 PDT

Sender: tesl-err@emachine.com

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

X-Mailer: IPAD 2.0

Jim write:

>Bill, how do you connect the plates of the foil together ? Every other

>one on left and right side ? How do you prepare the foil for connections

>to brass screws ? Large washers ?

Foil #1 went to the left, foil #2 went to the right, foil #3 went to the

left, etc... Up to 47! (48 polys, 47 foils)

We punched a crude hole through the foils and ran a bolt down through

the holes.  A washer and nut on top and bottom keep the pressure on the

foils.  The bolts protrude out the top of the plastic rubbermaid bucket!

We used steel bolts.  Brass would be better but we've not noticed any

heating so apparently the losses aren't too bad.

Finally - I put these neat little aluminum bus bars on the connection

bolts.  These things are known as "Neutral Bonding Blocks" and you can

get them at Home Depot or any large electric department.  They are used

inside a power distribution box to tie all the neutral wires together.

They have large screw-lug connections on them - and that's how I hook my

wiring up to the caps.

-Bill the arcstarter

Starting arcs in Cinci, OH

http://www.geocities.com/CapeCanaveral/Hangar/6160

Date: Tue, 30 Jun 1998 21:09:38 -0500

From: Bert Hickman <bert.hickman@aquila.com>

Reply-To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Organization: Stoneridge Engineering

X-Mailer: IPAD 2.0

To: "Tesla-2 Mailing List" <tesla-2@emachine.com>

Subject: Re: Rolled Capacitor Specs?

Sender: tesl-err@emachine.com

Tom Stock wrote:

>

> Ok, I'm new to poly cap design, so please tell me if I'm anywhere close to correct

> on this:

>

> I need a .01 mfd cap for my next coil upgrade. I'll be using two 15kv/30ma neons in > parallel and came up with the following specs for my coil:

>

> 3 rolled caps in series, each with the following specs:

>

> 24 mils of poly (4 layers of hardware store 6 mil)

> Plate length and width is 60" x 12"

>

> Using 300V for the puncture rating (per mil) and 2.3 for the 'K' I come up with:

>

> .03 uF per cap, max 7200V

>

> All three in series gives:

>

> .01 uF, 21,600V

>

> Am I ok so far?  Did I calc this right?

>

> Thanks

> -tom

Tom,

Looks pretty close, Tom! The capacitance value you want is correct for a

15 KV 60 MA power source, and 72 Mils of total dielectric made from

multiple thicknesses of 0.006" poly should also work fine for your 15 KV

source. However, you should plug in an "effective k" value of about 2 to

compensate for the fact that there's a bit of free space between the

capacitor plates and the dielectric. This gets you to about 0.025

uF/cap, so that you'll end up with about .008 uF when you connect three

in series.

BTW, you might want to check out Gary Weaver's site for some helpful

construction techniques for rolled caps made from HW store poly:

http://home.earthlink.net/~gweaver/cap.htm

Good luck and safe cappin' to you!

-- Bert --